***********************

*    "RECORD COPY"    *

* Schedule No. 7-6-2  *

* Retain Until 12-01  *

***********************

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

POWERTRAIN ELECTRONIC CONTROL STRATEGY BOOK

|                                STRATEGY LEVEL "CDAN2"

FOR USE WITH EEC-V MODULES:  ML2-300, ML2-330, ML2-370

THE PROCEDURE FOR OBTAINING COPIES OF THIS BOOK OR ANY OTHER  AVAILABLE  "CD"

DOCUMENTATION IS EXPLAINED ON THE NEXT PAGE.

ED SCHOENBERG

PHONE: (313) 322-6488

STRATEGY PLANNING & PROCESS DESIGN SECTION

CONTROL SYSTEMS DEPT

POWERTRAIN ELECTRONICS

DEVELOPMENT - PTE

|                        SOFTWARE RELEASE DATE:  APRIL 7, 1995

|                        STRATEGY BOOK RELEASE DATE:  APRIL 10, 1995

OBTAINING DOCUMENTATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|                              "CD" STRATEGY DOCUMENTATION

All current strategy documentation is stored on the VAX computer cluster.

|     Documentation can be obtained by logging into a VAX computer (I.E. SYS2,

|     SYS3, SYS4, etc.) and issuing one or more VAX/DCL commands.

Generally, two types of documentation are available:

|     1. UPDATE PACKAGES -  This file can be used as a quick reference to show

|        the changes which have been made for this release.  Change bars at the

|        left margin are used to indicate where changes in text have occurred

|        since the previous level.  The filename format is CD***UP.MEM, where

|        *** is the desired new strategy level.

|     2. COMPLETE BOOKS - The filename format is CD***.MEM, where *** is the

|        desired strategy level.  Changes in text which have occurred since the

|        previous release will be indicated with change bars.  In most cases

|        the INDEX contains an entry, "CHANGED PAGES," which lists all pages

|        containing changes.

|     The following VAX/DCL commands may be helpful in working with Strategy

|     Book documentation:

TO DETERMINE IF A SPECIFIC STRATEGY BOOK IS AVAILABLE, TYPE:

|                                Dir Strategy:CD***.MEM

|                                Dir Strategy:CD***UP.MEM

TO DETERMINE TARGETING OF EMR'S FOR FUTURE RELEASES, TYPE:

|                                EDTS

|          At the EDTS MAIN MENU, select "Standard Reports Menu"

|          (number 6) then, select "EMR's Within EMR Group"

|          (number 3)

TO DETERMINE THE STATUS OF STRATEGY BOOK DOCUMENTATION, TYPE:

|                                @Strategy:Bookstatus

|     TO OBTAIN A KODAK COPY OF A STRATEGY BOOK

|      LANDSCAPE -

|       Print/Queue=EEE$Kodak_Ansi/Form=Landscape/Notify Strategy:CD***.MEM

|      PORTRAIT  -

|       Print/Queue=EEE$Kodak_Ansi/Form=PORT_66X90/Notify Strategy:CD***.MEM

|       FOR FURTHER PRINT OPTIONS TYPE: HELP KODAK

OBTAINING DOCUMENTATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TABLE OF CONTENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTENTS

CHAPTER 1       STRATEGY EVOLUTION

1.1     OBTAINING STRATEGY EVOLUTION INFORMATION (CDAL0) . 1-2

CHAPTER 2       SYMBOLOGY

2.1     SYMBOLOGY (CDAA0)  . . . . . . . . . . . . . . . . 2-2

CHAPTER 3       EEC OVERVIEW

3.1     OVERVIEW (CDAL0) . . . . . . . . . . . . . . . . . 3-2

CHAPTER 4       HARDWARE INTERFACE

4.1     I/O CHANNEL ASSIGNMENTS (CDAN0)  . . . . . . . . . 4-2

4.2     HARDWARE-SPECIFIC INPUT PROCESSING (CDAN0) . . .  4-38

4.3     HARDWARE-SPECIFIC OUTPUT PROCESSING (CDAN0)  . .  4-42

4.4     OUTPUT STATE CONTROL TABLE (CDAL0) . . . . . . .  4-49

CHAPTER 5       SCP INTERFACES

5.1     FEATURE: SCP - V6.2A_SCP_EATC_PATS (CDAN0) . . . . 5-2

5.1.1     SCP MESSAGE OVERVIEW (CDAA0) . . . . . . . . . . 5-2

5.1.2     SCP BACKGROUND (CDAN0) . . . . . . . . . . . . . 5-6

5.1.3     SCP EXTERNAL INTERRUPT SERVICE ROUTINE (CDAK0)  5-13

5.1.4     SCP MESSAGE TRANSMIT SERVICE ROUTINE (CDAK0) .  5-16

5.1.5     SCP MESSAGE CONTROL (CDAN0)  . . . . . . . . .  5-18

5.1.6     EBC INITIALIZATION (CDAJ0) . . . . . . . . . .  5-25

5.1.7     SIGNIFICANT FEATURES OF SCP OBDII DIAGNOSTICS

(CDAM0)  . . . . . . . . . . . . . . . . . . .  5-27

5.1.8     SCP OUTPUT STATE CONTROL MESSAGE RESPONSE

HANDLING (CDAM0) . . . . . . . . . . . . . . .  5-90

5.1.9     RX_PT_STAT_RQST MESSAGE (CDAN0)  . . . . . . .  5-92

5.1.10    ECT_DEG_C MESSAGE (CDAM0)  . . . . . . . . . .  5-95

5.1.11    PASSIVE ANTI-THEFT (CDAK0) . . . . . . . . . .  5-97

5.1.12    ANTI_THEFT_STATUS Message (CDAK0)  . . . . . . 5-111

5.1.13    ENGINE_PAT_STATUS Message (CDAK0)  . . . . . . 5-113

5.1.14    REQUEST_SECURITY_STATUS (CDAK0)  . . . . . . . 5-115

5.2     SCP OBDII SAE-J1979 MODE 06 SUPPORT MAP (CDAN0)  5-117

5.3     SCP PID LIST (CDAN0) . . . . . . . . . . . . . . 5-122

CHAPTER 6       CRANK/UNDERSPEED/RUN MODE SELECTION STRATEGY

6.1     CRANK/UNDERSPEED/RUN MODE SELECTION (CDAI0)  . . . 6-2

CHAPTER 7       THROTTLE MODE SELECTION STRATEGY

7.1     THROTTLE MODE SELECTION STRATEGY (CDAI0) . . . . . 7-2

TABLE OF CONTENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CHAPTER 8       FUEL STRATEGY

8.1     FUEL STRATEGY OVERVIEW (CDAI0) . . . . . . . . . . 8-2

8.2     OUTPUT SHAFT SPEED LIMITER LOGIC (CDAA0) . . . . . 8-5

8.3     RPM LIMITER HIGH LOAD (CDAA0)  . . . . . . . . . . 8-8

8.4     WARM EGO LOGIC (CDAL0) . . . . . . . . . . . . .  8-12

8.5     LUGGING MODE DETERMINATION (CDAA0) . . . . . . .  8-15

8.6     LAMMUL RESET LOGIC (CDAC0) . . . . . . . . . . .  8-17

8.7     OPEN LOOP FLAG DETERMINATION (CDAM0) . . . . . .  8-20

8.8     CALCULATION OF LAMAVE (CDAA0)  . . . . . . . . .  8-29

8.9     OPEN LOOP LAMBSE CALCULATION (CDAN0) . . . . . .  8-31

8.10    LAMBSE RESET LOGIC (CDAN2) . . . . . . . . . . .  8-52

8.11    LAMBSE CLIP LOGIC (CDAH0)  . . . . . . . . . . .  8-59

8.12    FEATURE: ADAPTIVE FUEL CONTROL AND OBD-II FUEL

SYSTEM MONITOR - V1.5_FADPT (CDAN0)  . . . . . .  8-61

8.12.1    ADAPTIVE FUEL CONTROL LOGIC (CDAN0)  . . . . .  8-61

8.12.2    ADAPTIVE FUEL ENABLE LOGIC (CDAN0) . . . . . .  8-84

8.12.3    FUEL SYSTEM MONITOR (CDAM0)  . . . . . . . . .  8-90

8.13    FEATURE: TRANSIENT FUEL -

V1.1_FTRAN_TRANSIENT_FUEL (CDAM0)  . . . . . . . 8-100

8.13.1    BACKGROUND TRANSIENT FUEL LOGIC (CDAM0)  . . . 8-100

8.13.2    FOREGROUND TRANSIENT FUEL LOGIC (CDAL0)  . . . 8-108

8.14    HOT INJECTOR COMPENSATION (CDAN0)  . . . . . . . 8-112

8.15    INJECTOR DELAY LOGIC (CDAK0) . . . . . . . . . . 8-117

8.16    DECEL FUEL SHUT OFF LOGIC (CDAL0)  . . . . . . . 8-125

8.17    BACKGROUND FUEL (CDAM0)  . . . . . . . . . . . . 8-133

8.18    FEATURE: FUEL PUMP - V2.1_FPUMP (CDAN0)  . . . . 8-139

8.18.1    FUEL PUMP SELECTION LOGIC (CDAN0)  . . . . . . 8-139

8.18.2    FUEL PUMP CONTROL LOGIC (CDAK0)  . . . . . . . 8-142

8.18.3    TWO-SPEED FUEL PUMP CONTROL LOGIC (CDAJ0)  . . 8-144

8.18.4    FUEL PUMP OUTPUT CONTROL STRATEGY (CDAL0)  . . 8-147

8.18.5    FUEL PUMP SPEED OUTPUT (CDAL0) . . . . . . . . 8-149

8.18.6    FUEL PUMP PRIMARY CIRCUIT TEST (CDAM0) . . . . 8-150

8.18.7    FUEL PUMP SECONDARY CIRCUIT TEST (CDAM0) . . . 8-154

8.18.8    FUEL PUMP SPEED CIRCUIT TEST (CDAM0) . . . . . 8-159

8.19    PPCTR CONTROL (CDAA0)  . . . . . . . . . . . . . 8-161

8.20    RPM LIMITER LOGIC (CDAN0)  . . . . . . . . . . . 8-162

8.21    DIS INJECTOR SYNCHRONIZATION (CDAK0) . . . . . . 8-165

8.22    FOREGROUND FUEL CALCULATIONS EXECUTION CONTROL

(CDAL0)  . . . . . . . . . . . . . . . . . . . . 8-169

8.23    PIP INTERRUPT LOGIC FOR FUEL SYNCHRONIZATION

(CDAH0)  . . . . . . . . . . . . . . . . . . . . 8-173

8.24    FOREGROUND FUEL PULSEWIDTH CALCULATION (CDAL0) . 8-175

8.25    FOREGROUND INJECTOR TIMING - TOTAL DELAY

CALCULATION (CDAI0)  . . . . . . . . . . . . . . 8-182

8.26    INJECTOR UPDATE REQUEST (CDAM0)  . . . . . . . . 8-187

8.27    FUEL PULSEWIDTH, MAIN FUEL OFF (CDAI0) . . . . . 8-190

8.28    FUEL SERVICE LOGIC (CDAI0) . . . . . . . . . . . 8-195

8.29    FOREGROUND INJECTOR TIMING - IBETA CALCULATION

(CDAI0)  . . . . . . . . . . . . . . . . . . . . 8-204

8.30    FUEL SCHEDULING LOGIC (CDAM0)  . . . . . . . . . 8-210

8.31    INJECTOR OUTPUT CONTROL (CDAI0)  . . . . . . . . 8-214

8.32    CALCULATE DYNAMIC FUEL CORRECTION (CDAI0)  . . . 8-220

8.33    SCHEDULE DYNAMIC FUEL PULSE ON-EDGE (CDAK0)  . . 8-226

8.34    DYNAMIC FUEL OFF (CDAI0) . . . . . . . . . . . . 8-230

8.35    BACKGROUND INJECTOR CUTOUT (CDAM0) . . . . . . . 8-232

8.36    INJ_ON DETERMINATION LOGIC (CDAN0) . . . . . . . 8-239

8.37    PIP COUNTER FOR CRANKING FUEL (CDAA0)  . . . . . 8-241

8.38    CLOSED THROTTLE NEUTRAL TIMER (CDAA0)  . . . . . 8-243

TABLE OF CONTENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.39    TIME AT IDLE WITH TRANSMISSION IN DRIVE (CDAA0)  8-245

8.40    LOM LOAD TRANSITION TIMER (CDAA0)  . . . . . . . 8-247

8.41    TIME SINCE INCREMENTING LAMMUL (CDAA0) . . . . . 8-248

8.42    TIMER SUMMARY (CDAI0)  . . . . . . . . . . . . . 8-249

8.43    CRK_PIP_INJ SELECTOR LOGIC (CDAL0) . . . . . . . 8-253

8.44    LAMBSE MODULATION (CDAM0)  . . . . . . . . . . . 8-255

8.45    FEATURE: CLOSED LOOP FUEL CONTROL -

V2.8C_CLFUL_NONFLEX_96 (CDAN2) . . . . . . . . . 8-259

8.45.1    CLOSED LOOP LAMBSE CONTROLLER (CDAN0)  . . . . 8-259

8.45.2    CLOSED LOOP LAMBSE CALCULATION (CDAN2) . . . . 8-279

8.45.3    CLOSED LOOP LAMBSE CALCULATION (CDAI0) . . . . 8-282

8.45.4    TRANSPORT DELAY LOOKUP (CDAI0) . . . . . . . . 8-285

8.45.5    REAR EGO TRIM (CDAN2)  . . . . . . . . . . . . 8-288

8.45.5.1  PROCEDURE Faosc_init . . . . . . . . . . . . . 8-288

8.45.5.2  PROCEDURE Faosc2_main  . . . . . . . . . . . . 8-292

8.45.5.3  PROCEDURE Vego_filter  . . . . . . . . . . . . 8-293

8.45.5.4  PROCEDURE Vego_ratchet . . . . . . . . . . . . 8-295

8.45.5.5  PROCEDURE Fault_lockout  . . . . . . . . . . . 8-297

8.45.5.6  FUNCTION Rego_ok_logic . . . . . . . . . . . . 8-299

8.45.5.7  PROCEDURE Closed_loop_timer  . . . . . . . . . 8-301

8.45.5.8  FUNCTION Faosc_p_control . . . . . . . . . . . 8-302

8.45.5.9  FUNCTION Learn_timer . . . . . . . . . . . . . 8-304

8.45.5.10 FUNCTION Faosc_avg . . . . . . . . . . . . . . 8-307

8.45.5.11 PROCEDURE Faosc_scp_report . . . . . . . . . . 8-310

8.45.5.12 FUNCTION Output_routing  . . . . . . . . . . . 8-311

8.45.5.13 FUNCTION Faosc_i_control . . . . . . . . . . . 8-315

8.45.5.14 FUNCTION Final_calc  . . . . . . . . . . . . . 8-317

8.46    COOLING FAIL-SAFE LOGIC (CDAN0)  . . . . . . . . 8-320

8.47    FAR_MUL BACKGROUND LOGIC (CDAK0) . . . . . . . . 8-321

8.48    SPEED LIMITER CONTINUOUS SELFTEST CODE SETTING -

OBDII (CDAN0)  . . . . . . . . . . . . . . . . . 8-327

CHAPTER 9       IGNITION TIMING STRATEGY

9.1     OVERVIEW (CDAI1) . . . . . . . . . . . . . . . . . 9-2

9.2     FINAL SPARK CALCULATION (CDAN0)  . . . . . . . . . 9-4

9.3     SELECT LOWEST SPARK VALUE (CDAL0)  . . . . . . . . 9-8

9.4     BASE EMISSIONS AND DRIVE SPARK (CDAL0) . . . . .  9-11

9.5     BORDERLINE DETONATION SPARK (CDAN0)  . . . . . .  9-15

9.6     SPARK FOR LOW LOAD OPERATING CONDITIONS (CDAA0)   9-20

9.7     TORQUE TRUNCATION VIA SPARK RETARD (CDAE0) . . .  9-23

9.8     MAXIMUM BRAKE TORQUE SPARK (CDAL0) . . . . . . .  9-29

9.9     TORQUE-BASED FEEDBACK SPARK AT IDLE (CDAC0)  . .  9-32

9.10    OSCILLATION MODULATION (CDAA0) . . . . . . . . .  9-47

9.11    TORQUE MODULATION BACKGROUND CALCULATION (CDAK0)  9-49

9.12    KNOCK CONTROL STRATEGY (CDAN0) . . . . . . . . .  9-53

9.13    SPOUT CALCULATION (CDAN0)  . . . . . . . . . . .  9-68

9.14    PIP HIGH/LOW TRANSITION (CDAA0)  . . . . . . . .  9-71

9.15    SPARK INTAKE MANIFOLD RUNNER CONTROL LOGIC

(CDAL0)  . . . . . . . . . . . . . . . . . . . .  9-72

9.16    HIGH DATA RATE ELECTRONIC SPARK (CDAM0)  . . . .  9-74

9.17    VARIABLE CAM TIMING SPARK CONTROL LOGIC (CDAK0)   9-85

9.18    SPARK FOR FLEXIBLE FUEL (CDAL0)  . . . . . . . .  9-86

9.19    TORQUE MODULATION RAMP (CDAK0) . . . . . . . . .  9-88

9.20    SPARK OUTPUT TEST (CDAN0)  . . . . . . . . . . .  9-92

9.21    TRANSIENT SPARK FOR DETONATION PREVENTION (CDAN0) 9-99

9.22    TRANSIENT SPARK FOR DETONATION PREVENTION

(CDAM0)  . . . . . . . . . . . . . . . . . . . . 9-103

TABLE OF CONTENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CHAPTER 10      CATALYST BYPASS CONTROL STRATEGY

10.1    FEATURE: EXHAUST TEMPERATURE MODEL -

V1.9_CAT_TEMP_MODEL (CDAN0)  . . . . . . . . . .  10-2

10.1.1    EXHAUST TEMPERATURE CALCULATION (CDAN0)  . . .  10-2

10.1.2    SOAK TIME CALCULATION (CDAN0)  . . . . . . . . 10-14

10.2    FEATURE: CATALYST_MONITOR (STANDARD) -

V1.8A_CAT_FTP_MONITOR (CDAN0)  . . . . . . . . . 10-15

10.2.1    FTP CATALYST MONITOR (CDAN0) . . . . . . . . . 10-15

CHAPTER 11      EGO MONITOR CONTROL STRATEGY

11.1    FEATURE: HEGO MONITOR - V10.6_EGO_MONITOR (CDAN0) 11-2

11.1.1    EGO MONITOR CONTROL (CDAN0)  . . . . . . . . .  11-2

11.1.2    UPSTREAM EGO MONITOR (CDAM0) . . . . . . . . . 11-18

11.1.3    DOWNSTREAM EGO MONITOR (CDAM0) . . . . . . . . 11-40

11.1.4    EGO HEATER CONTROL (CDAN0) . . . . . . . . . . 11-57

11.1.5    EGO HEATER IO (CDAN0)  . . . . . . . . . . . . 11-93

11.1.6    EGO SWITCHING TEST (CDAN0) . . . . . . . . .  11-103

11.1.7    EGO SWITCHING TEST (CDAM0) . . . . . . . . .  11-132

11.1.8    EGO FAULT FLAG CONTROL (CDAM0) . . . . . . .  11-152

11.1.9    HEGO SENSOR INPUT PROCESSING (CDAN0) . . . .  11-157

11.1.10   HEGO HARDWARE TRANSFER FUNCTION (CDAN0)  . .  11-170

11.2    CSD COMPENSATION ALGORITHM (CDAN0) . . . . . .  11-172

11.3    CSD DETECTION ALGORITHM (CDAN0)  . . . . . . .  11-175

11.4    HEGO INPUT TRANSFER FUNCTION (CDAM0) . . . . .  11-180

11.5    HEGO LAMBSE CLIP STUB (CDAN0)  . . . . . . . .  11-182

11.6    EGO MONITOR CONTROL STRATEGY (CDAM0) . . . . .  11-183

11.7    UPSTREAM SENSOR SWITCHPOINT CALCULATION (CDAM0) 11-185

11.8    HEGO BYPLES CALCULATION (CDAN0)  . . . . . . .  11-186

CHAPTER 12      EGR STRATEGY

12.1    FEATURE: EGR - PFE - V1.11_EGR_PFE (CDAN0) . . .  12-2

12.1.1    EGR SYSTEM INITIALIZATION (CDAN0)  . . . . . .  12-2

12.1.2    EGR ENABLE (CDAL0) . . . . . . . . . . . . . .  12-4

12.1.3    DESIRED EGR MASS FLOW (CDAN0)  . . . . . . . .  12-8

12.1.4    DELTA PRESSURE FEEDBACK EGR (CDAN0)  . . . . . 12-12

12.1.5    EVR CONTROLLER - (EGRDC CALCULATION) (CDAN0) . 12-21

12.1.6    ACTUAL PERCENT EGR (CDAA0) . . . . . . . . . . 12-25

12.1.7    EGR OBDII TEST LOGIC (CDAN0) . . . . . . . . . 12-26

12.2    SCCD OUTPUT COMMAND (CDAN0)  . . . . . . . . . . 12-57

CHAPTER 13      IDLE SPEED CONTROL

13.1    GENERIC IDLE SPEED CONTROL (CDAL0) . . . . . . .  13-2

13.2    FEATURE: ISC_DSDRPM - V1.A1_ISC_DSDRPM (CDAN0) . 13-10

13.2.1    DSDRPM CALCULATION (CDAL0) . . . . . . . . . . 13-10

13.2.2    IS_TRAN_RPM And MAXIMUM CLIP CALCULATION -

(CDAL0)  . . . . . . . . . . . . . . . . . . . 13-12

13.2.3    DSDRPM (HICAM) CALCULATION (CDAJ0) . . . . . . 13-15

13.2.4    DSDRPM (LOADS) CALCULATION (CDAL0) . . . . . . 13-22

13.2.5    IS_TOT_RPM CALCULATION (CDAL0) . . . . . . . . 13-28

13.3    FEATURE: ISC_DESMAF_PID - V1.2_ISC_DESMAF_PID

(CDAN0)  . . . . . . . . . . . . . . . . . . . . 13-28

13.3.1    KAM UPDATE (ISCKAM_UPDATE) (CDAN0) . . . . . . 13-28

TABLE OF CONTENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

13.3.2    ISCKAM_HLT FLAG CONTROL (CDAN0)  . . . . . . . 13-34

13.3.3    IPSIBR CALCULATION (CDAN0) . . . . . . . . . . 13-36

13.3.4    ISLAST And ISFLAG LOGIC (CDAL0)  . . . . . . . 13-42

13.3.5    RPM ERROR CALCULATION (CDAL0)  . . . . . . . . 13-44

13.3.6    HCAMFG LOGIC (CDAL0) . . . . . . . . . . . . . 13-47

13.4    DESIRED MASS AIR FLOW CALCULATION (CDAJ0)  . . . 13-49

13.5    ACPPM CALCULATION (CDAL0)  . . . . . . . . . . . 13-53

13.6    EDFPPM CALCULATION (CDAN0) . . . . . . . . . . . 13-55

13.7    CYLPPM CALCULATION (CDAJ0) . . . . . . . . . . . 13-57

13.8    PS_PPM CALCULATION (CDAJ0) . . . . . . . . . . . 13-58

13.9    DASHPOT CALCULATIONS (CDAJ0) . . . . . . . . . . 13-60

13.10   MODE SELECT (MODE_SELECT) (CDAN0)  . . . . . . . 13-68

13.11   PERLOAD_ISC CALCULATION (CDAC0)  . . . . . . . . 13-75

13.12   IDLE AIR FEEDBACK CONTROL LIMIT TEST (CDAN0) . . 13-76

13.13   ISC DUTY CYCLE CALCULATION (ISCDTY_CALC) (CDAF0) 13-81

13.14   IAC (IDLE SPEED CONTROL) SCCD OUTPUT (CDAL0) . . 13-83

13.15   TIMER SUMMARY (CDAN0)  . . . . . . . . . . . . . 13-87

CHAPTER 14      A/C CLUTCH STRATEGY

14.1    FEATURE: AIR CONDITIONING CONTROL STRATEGY (ACC)

- V2.3_ACC (CDAN0) . . . . . . . . . . . . . . .  14-2

14.1.1    A/C CLUTCH INPUT (CDAN0) . . . . . . . . . . .  14-2

14.1.2    A/C CLUTCH CONTROL (CDAN0) . . . . . . . . . .  14-4

14.1.3    A/C CLUTCH CONTROL TIMERS (CDAN0)  . . . . . . 14-11

14.1.4    A/C CLUTCH OUTPUT (CDAN0)  . . . . . . . . . . 14-14

14.1.5    ACC OUTPUT STATE MONITOR (CDAN0) . . . . . . . 14-18

14.2    A/C CLUTCH DIAGNOSTICS (CDAN2) . . . . . . . . . 14-20

CHAPTER 15      CANISTER PURGE STRATEGY

15.1    FEATURE: EVAPORATIVE PURGE COMPENSATION CONTROL

(PCOMP) - V1.1D_CANP_PCOMP (CDAN0) . . . . . . .  15-2

15.1.1    PCOMP ENABLE (CDAM0) . . . . . . . . . . . . .  15-2

15.1.2    CANISTER PURGE STRATEGY (PCOMP) (CDAN0)  . . . 15-10

15.2    FEATURE: RUNNING LOSS OBD-II EVAPORATIVE SYSTEM

MONITOR. - V1.9A_CANP_MONITOR (CDAN2)  . . . . . 15-24

15.2.1    TANK PRESSURE SENSOR INPUT AND SELF TEST

(CDAJ0)  . . . . . . . . . . . . . . . . . . . 15-24

15.2.2    CVS FMEM (CDAN0) . . . . . . . . . . . . . . . 15-29

15.2.3    VAPOR MANAGEMENT VALVE OSM TEST LOGIC (CDAM0)  15-33

15.2.4    CANISTER PURGE MONITOR (CDAN2) . . . . . . . . 15-37

15.2.5    PURGE MONITOR FILTERING LOGIC (CDAL0)  . . . . 15-64

15.3    CANISTER PURGE STRATEGY (CDAM0)  . . . . . . . . 15-66

15.4    PURGE FLOW SENSOR INPUT AND SELF TEST (CDAJ0)  . 15-75

15.5    CANISTER PURGE SOLENOID FAULT TEST LOGIC (CDAJ0) 15-80

15.6    CANISTER PURGE OBDII TEST SELECT (CDAJ0) . . . . 15-84

15.7    CANISTER PURGE SELECT (CDAJ0)  . . . . . . . . . 15-85

15.8    CANISTER PURGE FLOW TEST (CDAN2) . . . . . . . . 15-86

15.9    VMV FLOW TEST (CDAN0)  . . . . . . . . . . . . . 15-95

CHAPTER 16      ELECTRO-DRIVE FAN STRATEGY

16.1    FEATURE: ELECTRO-DRIVE FAN CONTROL STRATEGY (EDF)

- V3.3_EDF (CDAN0) . . . . . . . . . . . . . . .  16-2

16.1.1    LOW SPEED FAN STRATEGY (CDAH0) . . . . . . . .  16-2

TABLE OF CONTENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

16.1.2    HIGH SPEED FAN STRATEGY (CDAN0)  . . . . . . .  16-5

16.1.3    ELECTRO-DRIVE FAN CONTROL STRATEGY (CDAN0) . . 16-10

16.1.4    FAN OUTPUT STATE MONITOR (CDAJ0) . . . . . . . 16-14

16.1.5    A/C FAN CONTROL (CDAL0)  . . . . . . . . . . . 16-18

16.1.6    ECT FAN CONTROL (CDAL0)  . . . . . . . . . . . 16-21

16.1.7    FINAL FAN DELIVERY (CDAL0) . . . . . . . . . . 16-23

16.1.8    VLCM OUTPUT SIGNAL PROCESSING (CDAL0)  . . . . 16-26

16.1.9    ENGINE COOLANT FAN OUTPUT STATUS TEST (CDAN0)  16-28

16.2    ELECTRO-DRIVE FAN OUTPUT STRATEGY (CDAN0)  . . . 16-34

CHAPTER 17      THERMACTOR AIR STRATEGY

17.1    EAM AND OUTPUT STATE MONITOR CIRCUIT (MID LINE 2)

(CDAN2)  . . . . . . . . . . . . . . . . . . . .  17-2

CHAPTER 18      INTAKE MANIFOLD RUNNER CONTROL STRATEGY

18.1    FEATURE: INTAKE MANIFOLD RUNNER CONTROL -

V1.3A_IMRC_ONOFF (CDAN2) . . . . . . . . . . . .  18-2

18.1.1    IMRC COMMAND LOGIC (CDAN0) . . . . . . . . . .  18-2

18.1.2    IMRC DIAGNOSTICS (CDAM0) . . . . . . . . . . . 18-13

18.1.3    IMRC INPUT LOGIC (CDAM0) . . . . . . . . . . . 18-27

18.1.4    IMRC DRIVER DIAGNOSTICS (CDAM0)  . . . . . . . 18-32

18.1.5    IMRC OUTPUT CONTROL LOGIC (CDAL0)  . . . . . . 18-34

CHAPTER 19      DATA OUTPUT LINK

19.1    PULSE CALCULATION (CDAI1)  . . . . . . . . . . .  19-2

CHAPTER 20      RIDE CONTROL

20.1    ADJUSTABLE SHOCK ABSORBER OUTPUT (CDAB0) . . . .  20-2

CHAPTER 21      EXPORT FEATURES

21.1    SPEED WARNING CHIME (CDAJ0)  . . . . . . . . . .  21-2

CHAPTER 22      ALTERNATIVE CALIBRATION

22.1    CALIBRATION INITIALIZE LOGIC (CDAI0) . . . . . .  22-2

22.2    CALIBRATION CLEAR LOGIC (CDAI0)  . . . . . . . .  22-4

CHAPTER 23      OBDII STRATEGY

23.1    FEATURE: OBDII EXECUTIVE - V1.4B_OBDII_EXEC

(CDAN0)  . . . . . . . . . . . . . . . . . . . .  23-2

23.1.1    DIAGNOSTIC SCHEDULER (CDAN0) . . . . . . . . .  23-2

23.1.2    SYSTEM FMEM (CDAN0)  . . . . . . . . . . . . . 23-25

23.1.3    MIL CONTROLLER (CDAM0) . . . . . . . . . . . . 23-45

23.1.4    FAULT CODES (CDAM0)  . . . . . . . . . . . . . 23-53

23.1.5    TRIPS (CDAM0)  . . . . . . . . . . . . . . . . 23-65

23.1.6    STATE CONTROL-TYPE 1 (CDAK0) . . . . . . . . . 23-73

TABLE OF CONTENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

23.1.7    STATE CONTROL-FUEL (CDAK0) . . . . . . . . . . 23-80

23.1.8    STATE CONTROL-MISFIRE (CDAK0)  . . . . . . . . 23-88

23.1.9    CLOSED LOOP ENTRY MIL CONTROL (CDAK0)  . . . . 23-99

23.1.10   MIL OUTPUT CONTROL (CDAM0) . . . . . . . . .  23-105

23.1.11   KAM VERIFICATION (CDAM0) . . . . . . . . . .  23-113

23.1.12   FREEZE FRAME (CDAM0) . . . . . . . . . . . .  23-121

23.1.13   SIMILAR CONDITIONS (CDAK0) . . . . . . . . .  23-128

23.1.14   DRIVING CYCLE (CDAF0)  . . . . . . . . . . .  23-132

23.1.15   J1979 PIDS (CDAM0) . . . . . . . . . . . . .  23-134

23.2    COMPREHENSIVE COMPONENTS - MONITOR FLAG AND

FAULT CODE SET (CDAF0) . . . . . . . . . . . .  23-137

23.3    FAULT FILTER AND FAULT MONITOR LOGIC (CDAI0) .  23-139

23.4    OUTPUT SUBSTITUTION CONTROL (CDAN2)  . . . . .  23-143

23.5    TRANSMISSION COMPREHENSIVE COMPONENT MONITOR

(CDAK0)  . . . . . . . . . . . . . . . . . . .  23-160

23.6    FEATURE: SELF TEST CONTROL -

V1.8_OBDII_SELF_TEST_CONTROL (CDAN2) . . . . .  23-164

23.6.1    ENGINE OFF SEQUENCE CONTROLER (CDAB0)  . . .  23-164

23.6.2    OUTPUT TEST MODE CONTROLLER (CDAF0)  . . . .  23-171

23.6.3    ENGINE RUNNING SEQUENCE CONTROLLER (CDAN2) .  23-177

CHAPTER 24      AIR CHARGE CALCULATION

24.1    FEATURE: AIR CHARGE - V3.1_AIR_CHARGE_97_IMRC

(CDAM0)  . . . . . . . . . . . . . . . . . . . .  24-2

24.1.1    AIRMETER OVERVIEW (CDAM0)  . . . . . . . . . .  24-2

24.1.2    SAMPLE AIRMETER CALCULATION (CDAG0)  . . . . .  24-4

24.1.3    MAF SENSOR FMEM - FOREGROUND (CDAM0) . . . . . 24-10

24.1.4    MANIFOLD FILLING MODEL (CDAM0) . . . . . . . . 24-12

24.1.5    AIR CHARGE ANTICIPATION (CDAM0)  . . . . . . . 24-16

24.1.6    FINAL CYLINDER AIRCHARGE (CDAM0) . . . . . . . 24-19

24.1.7    AIR CHARGE BACKGROUND CALCULATIONS (CDAM0) . . 24-21

24.1.8    AIR LEAKAGE CALCULATION (CDAM0)  . . . . . . . 24-31

24.1.9    BRAKE COMPENSATION (CDAM0) . . . . . . . . . . 24-34

24.1.10   MAF SENSOR DIAGNOSTIC TEST SELECTION (CDAM0) . 24-39

24.1.11   MAF SENSOR DIAGNOSTIC TESTS - BACKGROUND

(CDAM0)  . . . . . . . . . . . . . . . . . . . 24-42

24.1.12   LOAD CALCULATION (CDAM0) . . . . . . . . . . . 24-57

24.1.13   INFERRED LOAD (CDAM0)  . . . . . . . . . . . . 24-60

24.1.14   J1979_01_10, MASS AIR FLOW, CALCULATION

(CDAM0)  . . . . . . . . . . . . . . . . . . . 24-64

24.1.15   CALCULATION OF SAE J1979 LOAD PARAMETER

(CDAM0)  . . . . . . . . . . . . . . . . . . . 24-65

CHAPTER 25      TRANSMISSION INPUT CONVERSIONS

25.1    PERFORMANCE WEIGHTING DETERMINATION (CDAI0)  . .  25-2

25.2    TRANSMISSION CONTROL INDICATOR LIGHT (CDAM0) . .  25-5

25.3    TRANSMISSION CONTROL SWITCH (CDAM0)  . . . . . . 25-11

25.4    TORQUE NET CALCULATION (CDAN0) . . . . . . . . . 25-15

25.5    VS_RATE CALCULATIONS (CDAF0) . . . . . . . . . . 25-18

25.6    PREDICTIVE SHIFT SCHEDULE COMPENSATION

CALCULATIONS (CDAN0) . . . . . . . . . . . . . . 25-21

25.7    TRANSMISSION SELF TEST ENABLE (CDAM0)  . . . . . 25-25

25.8    RT_NOVS_KAM CALCULATION (CDAJ0)  . . . . . . . . 25-27

25.9    AODE TRANSMISSION CALCULATIONS (CDAL0) . . . . . 25-31

25.10   COLD SHIFT MULTIPLIER (CDAF0)  . . . . . . . . . 25-35

TABLE OF CONTENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

25.11   MLPS CONVERSION (CDAN0)  . . . . . . . . . . . . 25-38

25.12   TORQUE INTERPOLATION CALCULATION (CDAN0) . . . . 25-50

25.13   AODE SHIFT CONTROL OVERVIEW (CDAN0)  . . . . . . 25-53

25.14   THE 4 X 4 LOW SWITCH INPUT PROCESSING AND SELF

TEST (CDAJ0) . . . . . . . . . . . . . . . . . . 25-56

25.15   PWM FUNCTIONALITY TEST (CDAK0) . . . . . . . . . 25-58

25.16   A/C ENABLE CONTROL (CDAK0) . . . . . . . . . . . 25-68

25.17   TRANSMISSION INPUT TORQUE CALCULATION (CDAL0)  . 25-73

25.18   DIGITAL TRANSMISSION RANGE SENSOR INPUT

CONVERSION (CDAN0) . . . . . . . . . . . . . . . 25-76

25.19   IMRC/TRANSMISSION INTERACTION CONTROL (CDAN0)  . 25-87

25.20   OSS INPUT PROCESSING AND SELF TEST (CDAN0) . . . 25-89

25.21   VEHICLE SPEED SELF TEST (CDAN0)  . . . . . . . . 25-95

CHAPTER 26      SHIFT CONTROL

26.1    PRNDL BASED DESIRED GEAR DETERMINATION (CDAN2) .  26-2

26.2    GR_DS, PRNDL = 3 OR 4 LOGIC (CDAM0)  . . . . . .  26-6

26.3    DELAY/VERIFY SHIFT LOGIC (CDAJ0) . . . . . . . . 26-15

26.4    GR_SEQ_PNTR CALCULATION (CDAA0)  . . . . . . . . 26-20

26.5    PRNDL BASED COMMANDED GEAR DETERMINATION (CDAN0) 26-24

26.6    GR_CM, PRNDL = 1 LOGIC (CDAM0) . . . . . . . . . 26-29

26.7    GR_CM, PRNDL = 2 LOGIC (CDAG0) . . . . . . . . . 26-31

26.8    COMMANDED GEAR, AUTO UPSHIFT LOGIC (CDAJ0) . . . 26-33

26.9    GR_CM, PRNDL = 3 OR 4, DOWNSHIFT LOGIC (CDAN0) . 26-37

26.10   LOAD SHIFT IN PROGRESS TIMER (CDAA0) . . . . . . 26-41

26.11   DETERMINE SHIFT SOLENOID STATES (CDAL0)  . . . . 26-45

26.12   SHIFT VALIDATION LOGIC (CDAN0) . . . . . . . . . 26-52

26.13   TORQUE MODULATION CONTROL (WITHOUT TURBINE

SPEED) (CDAN0) . . . . . . . . . . . . . . . . . 26-63

26.14   TORQUE MODULATION FOREGROUND CONTROL (WITHOUT

TURBINE SPEED) (CDAN0) . . . . . . . . . . . . . 26-69

26.15   REVERSE LOCK-OUT SOLENOID CONTROL (CDAI0)  . . . 26-72

26.16   SHIFT SOLENOID FUNCITONAL TEST (CDAK0) . . . . . 26-74

CHAPTER 27      ELECTRONIC PRESSURE CONTROL

27.1    ELECTRONIC PRESSURE CONTROL GUIDE (CDAN0)  . . .  27-2

27.2    START-UP TV LOGIC (CDAA0)  . . . . . . . . . . . 27-16

27.3    ENGAGEMENT/STALL TV LOGIC (CDAA0)  . . . . . . . 27-18

27.4    NORMAL TV CALCULATION (CDAN0)  . . . . . . . . . 27-23

27.5    TQ_IALPHA CALCULATION (CDAA0)  . . . . . . . . . 27-32

27.6    DYNAMIC EPC CALCULATION (CDAM0)  . . . . . . . . 27-34

27.7    EPC VFS CONTROL AND SELF TEST (CDAG0)  . . . . . 27-41

27.8    TRANSMISSION OVERTEMPERATURE TEST (CDAN0)  . . . 27-52

27.8.1    PROCEDURE Transmission_overtemp_test . . . . . 27-52

27.8.2    ANTI-SILTING ENABLING LOGIC (CDAN0)  . . . . . 27-54

CHAPTER 28      CONVERTER CLUTCH CONTROL

28.1    BYPASS CLUTCH CONTROL (CDAM0)  . . . . . . . . .  28-2

28.2    HOT LOCKUP LOGIC (CDAA0) . . . . . . . . . . . .  28-3

28.3    SCHEDULED BYPASS CLUTCH SLIP (CDAN0) . . . . . .  28-4

28.4    UNCONDITIONAL UNLOCK LOGIC (CDAN0) . . . . . . . 28-10

28.5    SHIFT MODULATION LOGIC (CDAN0) . . . . . . . . . 28-22

28.6    BYPASS CLUTCH CONTROL TESTS (CDAK0)  . . . . . . 28-29

TABLE OF CONTENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

28.7    SLIP CALCULATION (CDAM0) . . . . . . . . . . . . 28-31

28.8    SCHEDULED LOCK/UNLOCK LOGIC (CDAN0)  . . . . . . 28-35

28.9    MLUS CONTROL AND SELF TEST (CDAN0) . . . . . . . 28-41

28.10   FAILURE MODE MANAGEMENT LOCK-UP (CDAA0)  . . . . 28-63

28.11   TORQUE CAPACITY DETERMINATION LOGIC (CDAN0)  . . 28-66

28.12   SHIFTING TORQUE CAPACITY CALCULATION (CDAM0) . . 28-68

28.13   NORMAL TORQUE CAPACITY CALCULATION (CDAM0) . . . 28-73

28.14   TORQUE CAPACITY CALCULATION (CDAK0)  . . . . . . 28-81

28.15   TORQUE CAPACITY CONVERSION (CDAK0) . . . . . . . 28-82

28.16   PID SLIP CONTROL (CDAM0) . . . . . . . . . . . . 28-85

CHAPTER 29      INPUT CONVERSIONS AND FILTERS

29.1    NEUTRAL/DRIVE TRANSITION LOGIC (CDAN2) . . . . .  29-2

29.2    BAROMETRIC PRESSURE - INFERRED (CDAL0) . . . . .  29-7

29.3    BAROMETRIC PRESSURE FILTER (CDAL0) . . . . . . . 29-13

29.4    ETV OVERCURRENT MONITOR VOLTAGE (CDAG0)  . . . . 29-15

29.5    POWER MODE (CDAA0) . . . . . . . . . . . . . . . 29-16

29.6    COLD TEMPERATURE DETERMINATION FOR TV SOLENOID

OPERATION (CDAA0)  . . . . . . . . . . . . . . . 29-18

29.7    DYNAMIC TV DUE TO COLD TRANSMISSION (CDAA0)  . . 29-20

29.8    ENGINE SPEED CALCULATION (CDAM0) . . . . . . . . 29-21

29.9    PIP INPUT ROUTINE (CDAM0)  . . . . . . . . . . . 29-25

29.10   PIP INPUT ROUTINE (CDAM0)  . . . . . . . . . . . 29-29

CHAPTER 30      TIMERS

30.1    TIMER SUMMARY (CDAN0)  . . . . . . . . . . . . .  30-2

CHAPTER 31      FAILURE MODE MANAGEMENT

31.1    ANALOG A/C HEAD PRESSURE INPUT (CDAN0) . . . . .  31-2

31.2    AIR CHARGE TEMPERATURE SENSOR INPUT AND SELFTEST

(CDAN2)  . . . . . . . . . . . . . . . . . . . .  31-9

31.3    AODE INPUT EQUATIONS (CDAA0) . . . . . . . . . . 31-15

31.4    BRAKE SWITCH INPUT AND SELF TEST (CDAM0) . . . . 31-17

31.5    DISTRIBUTORLESS IGNITION SYSTEM/MISFIRE FMEM

(CDAL0)  . . . . . . . . . . . . . . . . . . . . 31-21

31.6    ECT SENSOR INPUT AND SELFTEST (CDAN0)  . . . . . 31-27

31.7    OCTANE ADJUST INPUT TEST (CDAL0) . . . . . . . . 31-39

31.8    POWER STEERING PRESSURE SWITCH (PSPS) INPUT AND

SELF TEST (CDAJ0)  . . . . . . . . . . . . . . . 31-41

31.9    POWER STEERING PRESSURE SWITCH INPUT (CDAA0) . . 31-45

31.10   START STOP COUNTER FOR PSPS (CDAN0)  . . . . . . 31-47

31.11   TOT SENSOR INPUT AND SELFTEST (CDAM0)  . . . . . 31-49

31.12   IN-RANGE TP-LOAD TEST (CDAM0)  . . . . . . . . . 31-58

31.13   THROTTLE POSITION SENSOR INPUT AND SELFTEST

(CDAN2)  . . . . . . . . . . . . . . . . . . . . 31-62

31.14   VEHICLE MOVING DETECTOR (CDAL0)  . . . . . . . . 31-76

31.15   VEHICLE SPEED INPUT AND SELF TEST (CDAN0)  . . . 31-81

31.16   CLUTCH SWITCH INPUT AND SELFTEST (CDAL0) . . . . 31-84

31.17   ENGINE SPEED OVER VEHICLE SPEED (NOVS) (CDAL0) . 31-87

31.18   BATTERY VOLTAGE (VBAT) (CDAL0) . . . . . . . . . 31-88

31.19   CLOCK_SEC (CDAL0)  . . . . . . . . . . . . . . . 31-89

31.20   AC PRESSURE (ACP) SWITCH INPUT SIGNAL PROCESSING

(CDAN0)  . . . . . . . . . . . . . . . . . . . . 31-90

TABLE OF CONTENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

31.21   THROTTLE POSITION MULTI SAMPLING (CDAN2) . . . . 31-91

CHAPTER 32      KEEP ALIVE MEMORY

32.1    KEEP ALIVE MEMORY (KAM) QUALIFICATION TEST

(CDAM0)  . . . . . . . . . . . . . . . . . . . .  32-2

32.2    ON DEMAND SELF TEST FOR EEC MEMORY (KAM, RAM,

ROM) (CDAK0) . . . . . . . . . . . . . . . . . .  32-6

CHAPTER 33      ENGINE OFF SELF TEST

33.1    EDIS MODULE TEST (CDAB0) . . . . . . . . . . . .  33-2

CHAPTER 34      ENGINE RUNNING SELF TEST

34.1    TRANSMISSION CONTROL SWITCH CIRCUIT TEST (CDAB0)  34-2

CHAPTER 35      CONTINUOUS SELF TEST

35.1    MISFIRE DETECTION TEST (CDAN2) . . . . . . . . .  35-2

35.2    DISTRIBUTORLESS IGNITION SYSTEM TEST (CDAN0) . . 35-58

35.3    PROFILE CORRECTION LOGIC (CDAN2) . . . . . . . . 35-90

CHAPTER 36      ROM IDENTIFICATION CODE

36.1    ROM IDENTIFICATION CODE (CDAA0)  . . . . . . . .  36-2

14

CHAPTER 1

STRATEGY EVOLUTION

1-1

STRATEGY EVOLUTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

1.1  OBTAINING STRATEGY EVOLUTION INFORMATION (CDAL0)

STRATEGY EVOLUTION was a listing of EMRs implemented in the release

of a particular software version.  The report also listed the URD

number and a short description of the change.  While this information

is no longer contained in each strategy book, it is recognized that

it may still remain useful to some.  To view the information

on-line in a different format you may perform the following steps:

A.  Log onto the VAX system: SYS2, SYS3, SYS4.

B.  At the system prompt, enter EDTS.  The EDTS Main Menu

appears.

C.  Select 6 - STANDARD REPORTS MENU and press ENTER.

The Standard Reports Menu appears.

D.  Select 4 - EMR STATUS REPORT and press ENTER.  The

EMR Status Report Definition screen appears.

|         E.  Type in the Software Version  CDAN2.

If you want to view the report on line (the default),

press ENTER.  You may TAB through the report.

If you want the report sent to a file rather than the

screen, TAB through the screen and at the bottom

change the "S" to an "F" and press ENTER.  When the report

has been successfully written to a file, a message will

appear at the bottom of the screen:

"The report has been sent to a file called EMR_STATUS_132.EDTS

in your current directory."

F.  To exit EDTS, keep pressing ENTER until you are once again

at the system prompt.

1-2

CHAPTER 2

SYMBOLOGY

2-1

SYMBOLOGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

2.1  SYMBOLOGY (CDAA0)

INPUTS

The inputs to most logical operations will be conditional statements  of  the

form:

X > Y

where, X is a variable (RAM), and Y is a calibration constant,  fox  function

or  table  look-up,  or a mathematical expression.  In some cases, Y may also

represent a variable.

Typically, six types of conditional statements will appear  in  the  strategy

logic diagrams.  They are; X > Y, X < Y, X >= Y, X <= Y, X = Y, and X <> Y.

SYMBOL           MEANING

------     ------------------------

=         EQUAL TO

<>        NOT EQUAL TO

>         GREATER THAN

>=        GREATER THAN OR EQUAL TO

<         LESS THAN

<=        LESS THAN OR EQUAL TO

It should be noted that when the expression X > Y or X <  Y  is  encountered,

the  conditional statement can be calibrated such that it will never be true,

and the appropriate strategy action will never take place.  For  example,  if

the  variable  X has a range of 0 to 255, and the calibration constant in the

logical statement, X > Y, is selected to be 255, the statement will always be

false.  This provides a means for calibrating out certain strategy functions.

When any conditional statement is  true,  the  INPUT  STATE  to  the  logical

operation  is  said  to  be 'TRUE', and is assigned a value of '1'.  When the

statement is false, the INPUT STATE is 'FALSE', and is assigned  a  value  of

'0'.

2-2

SYMBOLOGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

LOGICAL OPERATIONS

Two logical operations are used, the 'AND' gate and the 'OR' gate.  An  'AND'

gate is represented by the following symbol:

A ---------------------|

|AND -| C

B ---------------------|

where A and B are INPUT STATES and C is defined as the OUTPUT  STATE  of  the

logical  'AND' operation.  The value of the OUTPUT STATE is a function of the

INPUT STATES as shown in the following truth table:

AND GATE

============================================

INPUT STATE     INPUT STATE     OUTPUT STATE

A               B                C

============================================

0               0                0

--------------------------------------------

0               1                0

--------------------------------------------

1               0                0

--------------------------------------------

1               1                1

============================================

Likewise, the 'OR' gate is represented by:

A ---------------------|

|OR --| C

B ---------------------|

And the OUTPUT STATE for various INPUT STATES is  given  by  the  'OR'  truth

table:

OR GATE

============================================

INPUT STATE     INPUT STATE     OUTPUT STATE

A               B                C

============================================

0               0                0

--------------------------------------------

0               1                1

--------------------------------------------

1               0                1

--------------------------------------------

1               1                1

============================================

2-3

SYMBOLOGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OUTPUTS

The output of all logical operations results in one of two possible paths:

1) The output is an input to another logical operation.

A ----------|

|AND -- C --|

B ----------|           |OR -| E

|

D ----------------------|

In this case, OUTPUT STATE C is an input to an 'OR' gate.  It should  be

treated  like any other conditional statement when determining the value

of the final OUTPUT STATE E.

2) ACTION is taken based upon the OUTPUT STATE.

A ---------------|

|AND -- C --| Energize OUTPUT

B ---------------|           |

| --- ELSE ---

|

| De-energize OUTPUT

The ACTION described in the action box is taken when OUTPUT STATE  C  is

'true'.   If  an  ALTERNATE  ACTION  is  required when OUTPUT STATE C is

'false', the alternate action is described below an  ELSE  statement  in

the  action  box.  If no alternate action is required, no ELSE statement

will appear.

2-4

SYMBOLOGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Multiple "ELSE/ACTION" blocks can appear in a logic diagram in which three or

more alternate actions are possible.  Consider the following example:

A ---------------|

|AND -- C --| ACTION #1

B ---------------|           |

| --- ELSE ---

D ---------------|           |

|AND -- F --| ACTION #2

E ---------------|           |

| --- ELSE ---

|

| ACTION #3

The procedure is:

1.  Test for ACTION #1.  If "C" is true, perform ACTION #1.

2.  Otherwise, test for ACTION #2.  If "F" is true, perform ACTION #2.

3.  Otherwise, perform ACTION #3.

Notes about multiple "ELSE/ACTION" logic:

1.  When logic has multiple "ELSE/ACTION" blocks, only one action  block  can

be performed during a program pass.  Priority is always top down.  In the

example, if "C" and "F" are simultaneously true, "C" takes precedence and

ACTION #1 is performed.

2.  If the final "ELSE/ACTION" block does not have logic as input pointing to

it  (as  in  the  example),  the  final action block is performed when no

preceeding action block is true.  Action is always performed during  each

program pass with this type of logic.

3.  If the final "ELSE/ACTION" block has logic as input pointing to  it,  the

final  action  block is performed only when no preceeding action block is

true and when its input logic is true.  Action is  not  always  performed

during each program pass with this type of logic.

2-5

SYMBOLOGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

HYSTERESIS

Hysteresis in a strategy is a situation in which the logic used  to  make  an

output  true  is  different from the logic used to make the output false.  An

example use would be to prevent on/off cycling of an output because of jitter

in an input parameter.

Hysteresis is represented in strategy  logic  diagrams  using  the  following

"flip-flop" notation:

A ---------------------|S  Q-| C

|

B ---------------------|C

The actual conditional statements and direction  of  hysteresis  will  depend

upon  the  specific  application  in  each strategy module The action of this

hysteresis notation is given by the following truth table:

HYSTERESIS FLIP-FLOP

==========================

S(SET)  C(CLEAR)  Q-OUTPUT

A        B          C

==========================

0        0      no change

--------------------------

0        1          0

--------------------------

1        0          1

--------------------------

1        1          1

==========================

The action of the flip-flop can be described as follows:

When the "A" (set) input is true, regardless of the "B" (clear) input  level,

the  flip-flop  sets  and the "C" output is true.  When the "B" input is true

and the "A" input is false, the flip-flop clears and the "C" output is false.

When "A" and "B" are both false, the "C" output remains unchanged.

ADVICE:

1.  Since the intent of a flip-flop is to provide hysteresis, the state of  a

flip-flop  must  be  remembered from pass to pass.  The output is usually

defined as a flag.

2.  All flip-flops must be serviced every  pass  through  the  program,  even

though  some  flip-flops  are  shown  in  portions  of logic that may not

execute each pass.  The normal  practice  is  to  lump  and  service  the

flip-flops  together at the beginning of a routine.  This guarantees that

all flip-flop outputs will reflect the correct  state  based  on  current

input conditions.  Then, when logic refers to a flip-flop, the logic only

needs to look at the flag which represents the state of the flip-flop.

2-6

SYMBOLOGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

3.  Clear (0) is  the  default  initial  value  of  the  output  flag  for  a

flip-flop.   The  strategy  specification  must  explicitly  state if the

initial value should be set (1).

4.  The set input always takes precedence over the clear  input.   When  both

are  true,  the  flip-flop  output  should  set.   In some instances, the

software practice has been to perform the clear logic first, followed  by

the  set  logic.   The  procedure  may  initially clear the flag and then

reverse the decision later.  This practice could cause  problems  if  the

flip-flop  output  flag is tested during an EOS interrupt because the EOS

can catch the flag in the wrong state.

The flip-flop procedure should always be:

IF set condition met

THEN set flip-flop output flag

ELSE IF clear condition met

THEN clear flip-flop output flag

ELSE

No change to flip-flop output flag

ENDIF

Some flip-flops are specified without a hysteresis term to save  bytes;  this

is the preferred method of presenting flip-flops.  In this case there are two

calibration parameters, one for the set condition,  and  one  for  the  clear

condition.  When flip-flops are specified this way, the calibration parameter

used to set the flip-flop will end in _SH or  _SL.   The  parameter  used  to

clear  will  end  in  _CH  or  _CL.   The H or L determines the larger (H) or

smaller (L) parameter.

Example:

DEL_LOAD < DLOD_SL ----|S  Q-| C

|

DEL_LOAD >= DLOD_CH ---|C

1.0 |

|

DEL_LOAD           |          /|\                |    o = state change

|           | clear region    |

DLOD_CH    |    -------o-----------------|--------------

|           |                 |

|           |                 |

|           | no change  (hysteresis)

DLOD_SL    |    -------|-----------------o--------------

|           |                 |

|           |  set region    \|/

|

0.0 ---------------------------------------------------------

2-7

SYMBOLOGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

HOW TO INTERPRET THE HEXADECIMAL REPRESENTATION

OF A REGISTER CONTAINING BIT FLAGS

1) Display the register on the calibration console.

2) Press the 'HEX' button on the calibration console to display the  register

in hexadecimal format.

3) Two hexadecimal digits will be displayed.  In order to determine which bit

flags are set, use the following hexadecimal-to-binary conversion chart:

HEXADECIMAL DIGIT DISPLAYED         BINARY EQUIVALENT STRING

--------------------------------------------------------------

|

0               |                0000

1               |                0001

2               |                0010

3               |                0011

4               |                0100

5               |                0101

6               |                0110

7               |                0111

8               |                1000

9               |                1001

A               |                1010

B               |                1011

C               |                1100

D               |                1101

E               |                1110

F               |                1111

The LEFTMOST hexadecimal digit represents the  state  of  bit  flags  in  bit

positions  7 through 4.  The RIGHTMOST hexadecimal digit represents the state

of bit flags in bit positions 3 through 0.

Consider the following:  The 8-bit binary string  representing  the  leftmost

and   rightmost  hexadecimal  digits  together,  read  from  left  to  right,

represents the state of bit flags in bit position 7 through bit position 0.

2-8

SYMBOLOGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EXAMPLE:

You want to examine the state of NDSFLG, a bit flag in bit position  5  of  a

register which is at address B0.

1) Display the contents of B0 on the calibration console.

2) Display the contents in hexadecimal format.

3) The hexadecimal value '2F' is displayed.

----------------------------------------------------

| HEXADECIMAL      |       2       |       F       |

|------------------|---------------|---------------|

| BINARY           | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 1 |

|------------------|---------------|---------------|

| BIT POSITION     | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

----------------------------------------------------

This means that bit flags in position 5 (NDSFLG), 3, 2, 1 and 0 are set to 1.

2-9

SYMBOLOGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

THE "DO:" STATEMENT

The DO:  statement is used to call a logic subroutine in strategy.  It is not

used,  as it often is by software, to avoid repeating a frequently used piece

of code.  It is primarily used to allow  a  process  to  be  broken  up  into

smaller  pieces  so  that  the  strategy  can  be  easily represented without

resorting  to  large,  cumbersome,  confusing  pieces  of  logic.   The   DO:

statement  can also provide an "executive routine" or calling structure for a

series of sequential pieces of logic.

The DO:  statement can call either an  entire  strategy  module  or  strategy

process.   A  strategy  module is a segment of strategy that has an overview,

definitions and a process and usually corresponds to a  software  module.   A

strategy  process  is  a  subset  of  a  strategy  module,  usually  used  to

conveniently break up a logic diagram in a strategy book.   Do  not  use  the

version  type  extension  for strategy modules, i.e.  COM1, COM5.  This makes

the calling module independent of various module varieties.  The syntax is:

DO: DSDRD_GR MODULE  or  DO: RPM LIMITER PROCESS

The DO:  statement is used on the  "ACTION"  or  right  hand  side  of  logic

diagram.

The format for the called subroutine (Module or Process) is that it must have

an  explicitly  stated  start  and  end  point.  This shows that the piece of

strategy contained  within  the  start  and  end  points  is  executed  as  a

subroutine,  not necessarily every background loop through the strategy.  The

syntax is:

BEGIN: DSDRD_GR MODULE               BEGIN: RPM LIMITER PROCESS

.                                      .

.                                      .

.                                      .

CONTINUE: DSDRD_GR MODULE            CONTINUE: RPM LIMITER PROCESS

(if required)                          (if required)

.                                      .

.                                      .

.                                      .

END: DSDRD_GR MODULE                  END: RPM LIMITER PROCESS

The CONTINUE:  syntax is used to identify the continuation of a logic diagram

on  to a page that does not have either a START:  or an END:  statement.  The

intent is to have any page of a strategy book be able  to  be  identified  as

being  part  of a subroutine or not.  For example, a logic diagram that takes

up 3 pages will begin with a START:  statement on the first page, begin  with

a  CONTINUE:  statement on the second page and end with an END:  statement on

the third page.

2-10

SYMBOLOGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

At the end of the module or process, there is an implicit "RETURN" statement,

that  is,  the  next  execution step of the strategy must return to the place

where the DO:  statement was called.  For example, if  a  DO:   statement  is

executed  in  the  middle  of  an  ELSE/ACTION  block, the strategy continues

execution after returning to the originating point.

A ---------------------|

|AND -| ACTION 1

B ---------------------|     |

| --- ELSE ---

C ---------------------|     |

|AND -| DO: XYZ PROCESS ------> BEGIN: XYZ PROCESS

D ---------------------|     |                 <--|              .

| --- ELSE ---       |              .

|                    |              .

| ACTION 2           |      END: XYZ PROCESS

|

|              |

|--------------|

E > F -----------------------| ACTION 1

|

| --- ELSE ---

|

| ACTION 2

If the first AND gate containing "A" and "B" is false and the second AND gate

containing  "C" and "D" is true, then the XYZ PROCESS is called and executed.

After returning to the DO:  statement, strategy execution continues with  the

evaluation of the "E > F" condition in the second ELSE/ACTION block.

DO:  statements can be nested,  that  is,  one  DO:   statement  can  call  a

subroutine which contains another DO:  statement within itself.  There are no

restrictions on this other than the basic  rule  continues  to  apply:   Each

process  or module that is executed must return to the originating point upon

its execution.

Flip flops should not be included within a DO:  statement since they must  be

evaluated  every  background  loop.  The strategy designer must have the flip

flops shown as being executed every background loop and use  the  appropriate

flop flop output flags within the DO:  statement.

2-11

SYMBOLOGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The "ROLAV" Statement

The ROLAV statement is used to invoke the  rolling  average  routine  in  the

strategy.   The  EEC-IV  filters  inputs  using this rolling average routine.

This is a difference equation  implementation  of  a  first  order  low  pass

filter.  The filter behaviour is defined by the discrete time solution to the

first order differential equation and takes the form:

- (FK_TMR/TC)

f  =  1  -  e

Where "f" is called the filter constant in the difference equation:

new_average = f * new_value + (1 - f) * old_average

Using the first two terms of the series expansion:

x                 x²

e   =  1  +  x  +  --  +  · · ·

2!

Simplifying, the exponential "f" becomes:

- (FK_TMR/TC)                  1

f  =  1  -  e                  =  1  -  -------------  =  . . .

(FK_TMR/TC)

e

1                      1

f  =  1  -  -----------------  =   ------------------- .

1 + (FK_TMR / TC)       1 + (TC / FK_TMR)

The time constant (TC) is a function of the  input  being  filtered.   It  is

calibratable.   Generally,  a  longer time constant filters more heavily, but

also introduces more time lag into the signal.

For most filters, the sampling period (FK_TMR) will equal the background loop

time  (BG_TMR).   In  those  case where the sample period is not equal to the

background loop time, the true sample period is to be passed to  the  rolling

average routine.  See the calling convention below.

The ROLAV statement is used on the "ACTION" or right hand  side  of  a  logic

diagram.   The  strategy  will  specify  calls to the rolling average routine

using the following convention:

condition -------------| new_average = ROLAV(new_value,TC[,FK_TMR])

Where:

-  old_average = Current value of  new_average  prior  to  filtering.   This

parameter is implicit in the call to the rolling average routine.

-  new_average = Output value of rolling  average  filter.   This  parameter

becomes the old_average on the next filtering event.

-  new_value = Input value to be filtered.

-  TC = Time constant.

-  FK_TMR = Elapsed  time  between  successive  calculations.   This  is  an

optional argument only to be specified if it is different from background

loop time.

2-12

SYMBOLOGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The "pid_def()" Construct

A PID is a way to  access  parameter  values  by  name  via  a  communication

protocol between the Powertrain Control Module and an outside requestor.

Parameter values from within  the  PCM  can  be  accessed  from  the  outside

environment  via the communication network.  This is done by the generic scan

tool as required by CARB-OBDII regulations to obtain information in reference

to  OBDII system monitors and their status.  This can also be done by SBDS or

other systems capable of establish and sustaining  communications  using  the

SCP.

CARB regulations require the availability of a number of PIDs  regarding  the

state of the on board diagnostic system.  These, as well as the Ford specific

PIDs, will be defined via the strategy documents  in  a  similar  manner.   A

construct  will be used in the strategy documents to define the data conveyed

in a PID request.  That construct is the:

pid_def(pid_number, description)

Where:

"pid_number" is the symbol used to refer to the PID.  For example,  all  PIDs

defined  by  SAE  standard  J1979  will  by  of the form j1979_xx[_xxx].  The

portion inside the brackets [] is optional.  Other conventions will  be  used

for j2190 or SBDS specific PIDs.

"description" is the actual data to be used in the transmission of  the  PID.

This  could  reference a register (RAM, KAM or ROM) that is maintained in the

strategy.  For example ECT or TEST_SW.  This could also be an  equation  used

to  compute  the  PID.  This equation may or may not reference other internal

registers in the strategy.  An example of this would be (((ECT - 32) * 5/9) +

40).   The  PID may also be a bit map of a number of flag bits.  In this case

the following convention is used for the "description" parameter:

pid_def(xxxx_xx_xxx,(b0: FLG_1,

b1: FLG_2,

b2: 1,

b3: 0,

b4: FLG_3,

b5: FLG_4,

b6: 0,

b7: 0)

Whenever a parameter is referenced in the "description" and is in lower  case

in  the  pid_definitions  context,  logic will be present in the context that

describes how that  parameter  is  to  be  determined.   The  pid_definitions

context  is  delineated  by  a "BEGIN:  pid_defintions" statement and a "END:

pid_defintions" statement.  The pid_defintions  context  is  located  in  the

module  in the strategy that is most relavent to the PIDs being defined.  The

pid_definitions  contexts  will  therefore  be  distrubuted  throughout   the

strategy.  See the following for an example of a module with PID definitions:

The  "pid_def()"  constructs  do  not  imply   anything   with   respect   to

implementation.  It is left up to the best judgement of the software designer

as to how to implement the PID scheme.

2-13

SYMBOLOGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

MODULE NAME

OVERVIEW

Put overview text here.  .  .

DEFINITIONS

Put definitions text here.  .  .

PROCESS

STRATEGY MODULE: STRATEGY_MODULE_COM1n

BEGIN: pid_definitions

;comment describing that this section is defining PIDs.

REGISTER1 = Y --------------|

|AND -| flag1 := 1

REGISTER2 = X --------------|     |

| --- ELSE ---

|

| flag1 := 0

unconditionally ------------------| parameter1 := (REGISTER3 * M) + B

pid_def(j1979_xx_xxx, REGISTER4)

pid_def(j2190_xx, parameter1)

pid_def(sbds_xx_xxx, FNnnn(REGISTER5))

pid_def(xxxx_xx_xxx, (REGISTER6 * M) + B))

pid_def(xxxx_xx, bo: flag1,

b1: FLG_1,

b2: 0,

b3: 1,

b4: FLG_2,

b5: CAL_SW_1,

b6: 0,

b7: 0)

END: pid_definitions

Continue with the control strategy logic here. . .

2-14

SYMBOLOGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The "send()" Construct

The SCP interface strategy allows for numerous messages to  be  supported  by

the Powertrain Control Module and the vehicle network databus.

These messages can be sent from various locations in the strategy,  depending

on  their functional intent, to be received by one or more recieving nodes on

the bus.

In order to provide a uniform method of sending an SCP message,  a  construct

will  be  used in the strategy documents to define the data to be transmitted

in an SCP message.  That construct is the

send(msg_name, scp_data_[])

Where:

"msg_name" is the name of the particular message to be sent.  For example,  a

response to an SAE standard J1979 request might be named REPORT_OBDII_PID.

"scp_data_[]" is the actual data to be  sent  in  the  message  transmission.

Since  there  are  up  to  seven  (7) data bytes available to be sent in each

message, the values of scp_data_[] could all be defined.  These values  could

be  either  register  names,  or constant hex values, or even a PID which was

defined in the "pid_def()" construct.  While not all seven of the scp_data_[]

values  are  required to be defined in a "send()" command, the construct does

not allow for data to be missing in between defined values.

An example of the use of the "send()" construct follows:

send(REPORT_OBDII_PID, 41h, 01h,

j1979_01_011,

j1979_01_012,

j1979_01_013,

j1979_01_014)

..which tells the software designer to send a message called REPORT_OBDII_PID

over the network with the following defined values:

scp_data_1 = 41h

scp_data_2 = 01h

scp_data_3 = the value defined in pid_def(j1979_01_011,...)

scp_data_4 = the value defined in pid_def(j1979_01_012,...)

scp_data_5 = the value defined in pid_def(j1979_01_013,...)

scp_data_6 = the value defined in pid_def(j1979_01_014,...)

NOTE:  Certain portions of a message  are  not  variable  and  are  thus  not

defined in the "send()" construct.  Thus, REPORT_OBDII_PID must be defined in

the SCP interface strategy for the software designers' reference.

The "send()" constructs do not attempt to imply software implementation,  but

rather  a  means  of  communicating  information  to the software designer to

transmit over the SCP network.

2-15

SYMBOLOGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

2-16

CHAPTER 3

EEC OVERVIEW

3-1

EEC OVERVIEW - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

3.1  OVERVIEW (CDAL0)

STRATEGY MODULE: EEC_CD

ELECTRONIC ENGINE CONTROL SYSTEM OVERVIEW

The Electronic Engine Control system is intended to provide  a  more  optimum

engine  control  strategy  than  is  possible  through  a strictly mechanical

system.  This is accomplished by  using  a  microprocessor  which  interprets

input  data from a number of engine parameter sensors, and based on a control

strategy in the microprocessor's  program  chips,  generates  output  control

signals to a number of actuators.

The control  strategy  is  divided  into  two  segments,  an  engine  control

strategy,  and  self  test diagnostics.  The diagnostics will be discussed in

another section.   The  engine  control  strategy  is  segmented  into  three

principal modes:

-CRANK

-UNDERSPEED

-RUN

The  strategy  description  and  the  entrance  and   exit   conditions   for

CRANK/UNDERSPEED/RUN  are shown on the following pages.  RUN is of particular

interest because it contains the control  logic  for  most  engine  operating

regions.   The  RUN  strategy  is  further  broken  down  into three modes to

facilitate optimum control.  Based primarily on throttle position, they are:

-CLOSED THROTTLE

-PART THROTTLE

-WIDE OPEN THROTTLE

The specific entrance and exit conditions for these modes  are  described  in

the throttle mode selection section.

3-2

EEC OVERVIEW - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EEC IV OUTPUTS

HARDWARE MODULES

AIR MANAGEMENT 1 (AM1) (SFI-MA2)

This Software output controls (via software) a Thermactor  Air  Bypass  (TAB)

valve;  via  a vacuum solenoid.  When AM1 is OFF, the air is bypassed (dumped

to the atmosphere).  When AM1 is ON air is routed to an up/down-stream valve.

AIR MANAGEMENT 2 (AM2) (SFI-MA2)

This Software output controls a vacuum solenoid which controls the  secondary

air  system to a catalyst via an up/down-stream divertor valve (TAD) but this

output only controls if AM1 output is ON.

CANISTER PURGE (CANP)

This software controls a solenoid which  purges  fuel  vapor  from  a  carbon

canister.  The carbon canister collects fuel vapor when the engine is off.

DATA OUTPUT LINK (DOL)

This software generates a digital  signal  to  one  of  the  Economy  Display

Subsystems:   Tripminder, Fuel Economy Meter Clock, etc).  This displays such

information as current Miles per Gallon to the driver.

ELECTRONIC VACUUM REGULATOR (EVR)

This software output controls a solenoid which varies the vacuum to  the  EGR

valve via a varying frequency and/or duty cycle pulse train to the regulator.

The Actuator contains a clamping diode which eliminates the need for a  diode

in the control module.

3-3

EEC OVERVIEW - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FUEL PUMP (FP)

The software for the fuel pump controls  a  relay  which  provides  power  to

operate the electric fuel pump.

IMRC (SFI-MA3)

The Inlet Manifold Control opens a valve (upstream of  the  intake  manifold)

which  increases  the engine airflow.  The Engine strategy uses this valve to

increase the horsepower at high RPM.

INJECTORS

The Sequential Fuel Injection system uses one output driver per injector  (as

compared  to  the  EFI systems which have two or more injectors controlled by

each driver).  The SFI system provides more precise timing  and  duration  of

the  fuel injection.  As most EFI systems, SFI uses high resistance (16 Ohms)

high pressure (39 psi) injectors.

IDLE SPEED CONTROL - BYPASS AIR - CONSTANT CURRENT (ISC-BPA)

The EEC controls a linear  actuator  which  varies  air  intake  (from  0  to

maximum) through a bypass valve in the throttle body.  The software outputs a

duty- cycled pulse train which is coverted into  a  constant  current.   This

output is cycled at a frequency of 160 Hz.

SPARK OUTPUT (SPOUT)

The Spark software generates an ignition control signal  capable  of  driving

the  Thick  Film  Ignition  Module  (TFI).   The driver is a tri-state device

(High, low, open) which interfaces with the TFI-IV module  (This  Module  can

accomodate a 250 Hz/sec acceleration).  The spout signal is referenced to the

PIP signal.  Its timing is controlled by the SAF calculation.  If the EEC  is

in LOS mode, the TFI module causes the spark timing to be equal to 10 deg BTC

(same as PIP).

SPEED WARNING CHIME (SWC)

The software operates a chime module  when  a  calibrated  vehicle  speed  is

exceeded.  This is a legal feature required in Saudi Arabia and is for export

purposes only.

3-4

EEC OVERVIEW - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

HARDWARE PRESENT SWITCHES

Hardware present switches define the types of devices connected  to  the  EEC

module through the wiring harness.

FUEL RELATED HARDWARE PARAMETERS

-  DEGPIP = Degrees per PIP period = 360/ENGCYL.

-  ENGCYL = Number of PIPs per revolution

= cylinder number/2.

-  FN1327 = Fuel pulsewidth to injector map;

0 -> use FUELPW1,

1 -> use FUELPW2.

-  FN1329 = Injector Firing order map.

-  HP_CIDSEL = CID Hardware select switch;

0 -> Hall Effect present,

1 -> VRS sensor.

-  IGN_TYPE = Indiactes the type of ignition hardware;

0 -> Push start or start wireless TFI

1 -> ICCD-TFI

2 -> Dual plug LDR-DIS

3 -> Single plug LDR-DIS

4 -> EDIS HDR-DIS (36 tooth - 1)

-  MHPFD = Signature PIP duty cycle

.99 = Sig PIP distributor not present

.20 = 8 cyl, <= 35% rduty cycle

.24 = 6 cyl, <= 30% duty cycle

.29 = 4 cyl, <= 30% duty cycle.

-  NUMCYL = Number of engine cylinders.

-  NUMEGO = Number of EGO sensors (1 or 2).

3-5

EEC OVERVIEW - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OUTPUTS

-  HPACL = Ride Control Hardware present;

1 -> Ride Control present,

0 -> No Ride Control present.

-  IMRCHP = Inlet Manifold Runner Control hardware present;

1 -> IMRC hardware present,

0 -> IMRC hardware not present.

-  SWC_HP = Speed Warning Chime Hardware Present;

0 -> No SWC hardware present.

1 -> SWC hardware present.

-  THRMHP = Thermactor Hardware Present Switch;

0 -> None of the following hardware present,

1 -> Thermactor Hardware present, AM1, AM2.

-  TSTRAT = Transmission Strategy Switch,

The TSTRAT software switch selects which transmission

control strategy is to be executed;

TSTRAT  0 -> No transmission control,

(Manual trans., AOD, ATX, C6, C3, etc.),

1 -> SIL (Shift Indicator Light),

2 -> A4LD with 3-4 shift control and converter

clutch control,

3 -> AXOD,

4 -> E4OD,

5 -> A4LD-E,

6 -> AXOD-E,

7 -> AOD-I,

8 -> F4E,

9 -> CD4E,

10 -> JATCO

3-6

EEC OVERVIEW - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

INPUTS

-  BIHP = Brake input;

0 -> No Brake input,

1 -> Brake input available.

-  HPOC = Overdrive cancel hardware present switch; -1 -> no OCS (no  OCIL),

0  ->  OCS  is  a  latched relay (no OCIL logic), 1 -> OCS is a momentary

contact (OCIL output is a bulb - LSO-3), 2 -> OCS is a momentary  contact

(OCIL output is a LED - LSO-4).

-  HWRT, ACRT = Heated Windshield recognition times,

If no H/W present, set HWRT = 65,535

and set ACRT = 0

If H/W present, see strategy book description.

-  PSPSHP = Power Steering Pressure Switch;

0 -> No Power Steering Pressure switch input,

1 -> Power steering pressure switch.

-  TRLOAD = Transmission Load;

0 -> Manual Transmission, no clutch or gear switches,

forced neutral state (NDSFLG = 0),

1 -> Manual Transmission, no clutch or gear switch,

2 -> Manual Transmission, one clutch or gear switch,

3 -> Manual Transmission, both clutch and gear switches,

4 -> Auto Transmission, non-electronic, neutral drive switch,

5 -> Auto Transmission, non-electronic, neutral pressure switch

(AXOD),

6 -> Auto Transmission, electronic, PRNDL sensor - park,

reverse, neutral, overdrive, manual 1, manual 2.

-  VSTYPE = Vehicle speed sensor H/W present switch;

0 -> No Vehicle Speed Sensor,

1 -> Vehicle speed sensor present (VSS).

3-7

EEC OVERVIEW - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

INITIALIZATION ROUTINE

After Power is first applied, the software initializes all of the  Read/Write

(RAM)  registers  before  executing  the strategy.  All RAM registers are set

equal to zero unless initialized to another value  (either  set  value  or  a

calculated value) as shown below.

NOTE:  The parameters are not necessarily initialized in the order shown.

RAM/FLAG                                INITIAL VALUE

ACCSTMR                                 63.99 sec

ACTPTMR2                                31.875 sec

ACT                                     60 Deg F

ACT_MIN                                 254 Deg F

AIR_CHG_WOT                             0.001

ANPIP1                                  1

ANPIP2                                  1

APT                                     -1 (Closed Throttle)

BP_INTR                                 1

CONPR                                   650.0 Counts

CRKFLG                                  1 (Crank Mode)

DOL_COUNT                               0.5

ECT                                     60 Deg F

ENPIP1                                  1

ENPIP2                                  1

F_A_RATIO1                              6.8298e-2

F_A_RATIO2                              6.8298e-2

GEAR_CUR                                1

GEAR_OLD                                1

GR_CM                                   GR_CM_KAM

GR_CM_LST                               1

GR_DS                                   1

GR_DS_LST                               1

GR_OLD                                  1

IBAP                                    BP

INJ_PIP_CNT1                            1.0

INJ_PIP_CNT2                            1.0

ISCMOD                                  1.0

LAMAVE1                                 1.0

LAMAVE2                                 1.0

LAMBSE1                                 1.0

LAMBSE2                                 1.0

LAMMUL                                  1.0

LAM_OLD1                                0.0

LAM_OLD2                                0.0

(continued on next page)

3-8

EEC OVERVIEW - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

RAM/FLAG                                INITIAL VALUE

OLFLG                                   1 (Open Loop)

PPCTR                                   PIPNUM

PUMP                                    1.0

RANNUM                                  8193

RT_NOVS                                 1

SAF                                     10 Deg BTDC

SIM_MIN                                 SIMLIM4

TBART                                   RATIV

TC_HRD_OFF                              TCHRDINT

TPBART                                  RATIV

TPBARTV                                 RATIV

TQ_INTRP                                1

TSLMPH                                  ^FF (255 msec.)

UNDSP                                   1 (Underspeed)

VBAT                                    12.5 Volts

The strategy checks the validity  of  the  Adaptive  Fuel  and  Idle  Airflow

(ISCKAM)  after the RAM initialization.  This particular KAM qualification is

described in the KAM Chapter.

3-9

EEC OVERVIEW - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

3-10

CHAPTER 4

HARDWARE INTERFACE

4-1

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

4.1  I/O CHANNEL ASSIGNMENTS (CDAN0)

STRATEGY MODULE: HWINT_IO_CD

APPLICATION SUMMARY

_____________________________________________________________________________

This chapter is intended to accurately summarize I/O information  that  links

hardware  to software.  All features described have been verified to exist in

the strategy.  The applications listed below, however,  may  utilize  only  a

subset of the available features.

NOTE:   ALL  applications  --  developmental,  experimental,  production  and

non-production,  etc.   that are supported at the time of the release of this

book are listed below.  The interface has been verified to support these, and

only these, applications.  Any deviations are noted.

|     S U P P O R T E D   P A T H S / V E R S I O N S:  CDAN2

A P P L I C A T I O N S:

(1)                    VEHICLE:  1996 EN114

ENGINE:  4.6L-2V

TRANSMISSION:  4R70W

MODULE SPECIFICATION:  HCR #387  (Phase 2)

MODULE TYPE:  ML2-333

WIRING DESIGN TRANSMITTAL:  A10363674  Dated: 12/23/93  Level: CP

DEVIATIONS:  See below

(3)                    VEHICLE:  1996 FN116

ENGINE:  4.6L-2V

TRANSMISSION:  4R70W

MODULE SPECIFICATION:  HCR #387  (Phase 2)

MODULE TYPE:  ML2-333

WIRING DESIGN TRANSMITTAL:  A10355172  Dated: 12/20/93  Level: CP

DEVIATIONS:  See below

(4)                    VEHICLE:  1996 SN95

ENGINE:  3.8L

TRANSMISSION:  4R70W, T5

MODULE SPECIFICATION:  HCR #387  (Phase 2)

MODULE TYPE:  ML2-309, ML2-306

WIRING DESIGN TRANSMITTAL:  EF-20399-3  Dated: 11/4/93  Level: N/A

DEVIATIONS:  See below

(5)                    VEHICLE:  1996 SN95

ENGINE:  4.6L-2V

TRANSMISSION:  4R70W, T45

MODULE SPECIFICATION:  HCR #387  (Phase 2)

MODULE TYPE:  ML2-334, ML2-332

WIRING DESIGN TRANSMITTAL:  EF-20326-5  Dated: 11/4/93  Level: N/A

DEVIATIONS:  See below

(continued on next page)

4-2

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

(6)                    VEHICLE:  1996 SN95

ENGINE:  4.6L-4V

TRANSMISSION:  T45

MODULE SPECIFICATION:  HCR #387  (Phase 2)

MODULE TYPE:  ML2-334

WIRING DESIGN TRANSMITTAL:  EF-20390-4  Dated: 11/4/93  Level: N/A

DEVIATIONS:  See below

(7)                    VEHICLE:  1996 MN12

ENGINE:  3.8L

TRANSMISSION:  4R70W

MODULE SPECIFICATION:  HCR #387  (Phase 2)

MODULE TYPE:  ML2-309

WIRING DESIGN TRANSMITTAL:  A10420493 Dated: 6/29/94 Level: Prod.

DEVIATIONS:  See below

(8)                    VEHICLE:  1996 MN12

ENGINE:  4.6L-2V

TRANSMISSION:  4R70W

MODULE SPECIFICATION:  HCR #387  (Phase 2)

MODULE TYPE:  ML2-334

WIRING DESIGN TRANSMITTAL:  A10420504 Dated: 6/29/94  Level: Prod.

DEVIATIONS:  See below

4-3

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

D E V I A T I O N S:

1.  Digital Transmission Range Sensor (DTRS) is supported via  the  following

four  inputs:  TR1 (RPA-2, J1-03), TR2 (DARC PI/O-0, J1-49), TR3A (ACH-2,

J1-64), and TR4 (DARC PI/O-4, J1-40).  This is not currently required and

is not shown on HCR #387 or the Design Transmittal.

2.  Reverse Lockout Solenoid (RLS) is output via UPOD3-2 (J1-53).  This is no

longer required and does not appear on HCR #387.

3.  Low Fan Control (LFC) is supported on UPOD3-3 (J1-28).  This is no longer

required and is not shown on HCR #387.

4.  Acceleration output, or Ride  Control  (ACL)  is  supported  on  SSPOD1-3

(J1-19)  and  is shown on HCR #387 for 4.6L MN12.  This feature, however,

is not supported by Design Transmittal EF-20450-7 for 4.6L MN12.

_____________________________________________________________________________

4-4

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OVERVIEW

The Hardware Interface chapter specifies relationships between  strategy  I/O

parameters  and  EEC-V  module  I/O  operation.  It is intended to accurately

summarize I/O information that links hardware to software, and it attempts to

document  each signal's path from the CPU to the wiring harness.  To do this,

each I/O port and IC device is considered separately.  A description of  each

is  given  first,  followed  by  a  listing  of  each  channel, its specified

function, its associated parameter names, and its I/O path.   Any  additional

information  thought  to  be  of  interest  is  given following each section.

Included information, however, is subject to the following guidelines.

o  The only strategy parameters that are referenced in this module are those

that  are  directly  and  unconditionally  mapped  to  the  I/O  channels

identified  in  this  module.   The   ONLY   exceptions   to   this   are

hardware-present  switches  referenced  in  this  module  FOR INFORMATION

PURPOSES ONLY.

o  NO RUN-TIME LOGIC IS SPECIFIED IN THIS MODULE.  Some I/O channels may  be

configurable  at  calibration time.  Any run-time logic that controls the

mapping of strategy feature I/O parameters to I/O channels  is  specified

in SEPARATE, DEDICATED strategy modules.

The field legend below describes those fields that may be contained in an I/O

description  table.   The  last  four  fields  pertain  only to output driver

devices which communicate via the Serial Peripheral Interface (SPI).   Please

refer to the descriptions preceding each table for more specific information.

F I E L D   L E G E N D:

_____________________________________________________________________________

CHNL-N                      Hardware channel name and number.

FUNCTION:  SAEFUNC     SAE J1930 function name.

REGISTER:  FUNC_ON     Strategy/software parameter.

SWITCHES:  FUNC_SW     Relevant switches and calibration parameters.

INPUT:  PIN-N       Input path reference.

OUTPUT:  PIN-N       Output path reference.

I/O:  PIN-N       Path reference with INPUT or OUTPUT designation.

CONTROL:  CTRL_MECH   Method of POD channel control.

SOURCE:  CHNL_N      Source for parallel control of POD device.

COMMAND REG:  POD_BIT     Command data for serial control of SPI device.

FAULT REG:  POD_FAULT   Fault status received from SPI device.

_____________________________________________________________________________

_____________________________________________________________________________

4-5

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

ENHANCED EEC MODULE/LSI CONFIGURATION SKETCH

-----------------------------------------------------------------------

|                                                                       |

|                                                              ------   | 6

|                  ----------              ------        |----|UPOD1 |===/==>

|  ------         |          |            |  2K  |       |     ------   |

| |      |   SLAP |   8065   |   M-BUS    | RAM  |       |     ------   | 6

| | AICE |--------|          |============| EBC  |       |----|UPOD2 |===/==>

| |      |        | exp mode |   ||       | DUCE |       |     ------   |

|  ------         |          |   ||        ------        |     ------   | 6

|   |              ----------    ||                      |----|UPOD3 |===/==>

|   |                 |  |       ||                      |     ------   |

|   ------------------|  |      M||      ------          |              |

|   PIP              -|--|-     B||     | 2.5K |   SPI   |     ------   | 6

|                4  |      |    U||=====| DARC |---------|----|SSPOD1|===/==>

| -----    =====/===| EDIS |    S||     | I/O  |         |     ------   |

|| CD1 |---||||     |      |     ||      ------          |     ------   | 6

| -----     |||      ------      ||                      |----|SSPOD2|===/==>

| -----     |||                  ||                      |     ------   |

|| CD2 |----|||    --------      ||                      |              |

| -----      ||   |        |     ||                      |     ------   | 2

| -----      ||   |  112K  |=====||                      |----| SCCD |===/==>

|| CD3 |-----||   | Flash  |                             |     ------   |

| -----       |   | EEPROM |                             |              |

| -----       |   |        |                             |     ------   | 4

|| CD4 |------|    --------                              |----| IMP  |===/==>

| -----                                   ML-2                 ------   |

|                                      104 Pin J1                       |

-----------------------------------------------------------------------

(Refer to current hardware specifications for a more complete drawing.)

4-6

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

ANALOG INPUT CHANNELS (ACH)

The sixteen 8065 analog inputs convert analog voltages (0-5V) to  A/D  counts

(0-1023  counts).  Typically, these values are filtered and converted to more

convenient engineering units.  The registers that  contain  the  'raw'  input

values are given below.

_____________________________________________________________________________

ACH-0

FUNCTION:  TOT           Transmission Oil Temperature.

REGISTER:  TOT_CNTS      Raw input counts.

SWITCHES:  none

INPUT:  J1-37

_____________________________________________________________________________

ACH-1

FUNCTION:  FTPT          Fuel Tank Pressure Transducer.

REGISTER:  ICHAN_1       Raw input counts.

SWITCHES:  none

INPUT:  J1-62

_____________________________________________________________________________

ACH-2

FUNCTION:  MLPS/          Manual Lever Position Sensor.

TR3A           DIGITAL TRANSMISSION RANGE SENSOR, BIT 3

REGISTER:  INDS          Raw input counts.

SWITCHES:  none

INPUT:  J1-64

_____________________________________________________________________________

ACH-3

FUNCTION:  TP1           Primary Throttle Position.

REGISTER:  TP_CNTS       Raw input counts.

SWITCHES:  none

INPUT:  J1-89

_____________________________________________________________________________

ACH-4

FUNCTION:  unused

REGISTER:

SWITCHES:

INPUT:

_____________________________________________________________________________

(continued on next page)

4-7

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

_____________________________________________________________________________

ACH-5

FUNCTION:  ACPSW         A/C Pressure Switch.

REGISTER:  IACPSW        Raw input counts.

SWITCHES:  ACPSW_HP

INPUT:  J1-86

FUNCTION:  ACPT          A/C Pressure Transducer.

REGISTER:  IACPRES_SEN   Raw input counts.

SWITCHES:  ACPSEN_HP

INPUT:  J1-86

_____________________________________________________________________________

ACH-6

FUNCTION:  PSPT/PSPS     Power Steering Pressure Transducer/Switch.

REGISTER:  IPSPS         Raw input counts.

SWITCHES:  none

INPUT:  J1-31

_____________________________________________________________________________

ACH-7

FUNCTION:  DPFE (DEPT)   Delta Pressure Feedback EGR. (DEPT is old name).

REGISTER:  IEGR          Raw input counts.

SWITCHES:  none

INPUT:  J1-65

_____________________________________________________________________________

ACH-8

FUNCTION:  ECT           Engine Coolant Temperature.

REGISTER:  ECT_CNTS      Raw input counts.

SWITCHES:  none

INPUT:  J1-38

_____________________________________________________________________________

ACH-9

FUNCTION:  IAT (ACT)     Intake Air Temperature Sensor (ACT is old name).

REGISTER:  ACT_CNTS      Raw input counts.

SWITCHES:  none

INPUT:  J1-39

_____________________________________________________________________________

ACH-10

FUNCTION:  unused

REGISTER:

SWITCHES:

INPUT:

_____________________________________________________________________________

(continued on next page)

4-8

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

_____________________________________________________________________________

ACH-11

FUNCTION:  unused

REGISTER:

SWITCHES:

INPUT:

_____________________________________________________________________________

ACH-12

FUNCTION:  HTRCM         HEGO Heater Current Monitor

REGISTER:  HTRCM_CNTS    Raw input counts.

SWITCHES:  none

INPUT:                Internal to EEC-V module.

_____________________________________________________________________________

ACH-13

FUNCTION:  IMRCM1        Intake Manifold Runner Position Sensor - Bank 1

REGISTER:  IMRCP1_CNTS   Raw input counts.

SWITCHES:  none

INPUT:  J1-08

_____________________________________________________________________________

ACH-14

FUNCTION:  IMRCM2        Intake Manifold Runner Position Sensor - Bank 2

REGISTER:  IMRCP2_CNTS   Raw input counts.

SWITCHES:  none

INPUT:  J1-09

_____________________________________________________________________________

ACH-15

FUNCTION:  VMV-OSM       Vapor Management Valve Output State Monitor.

REGISTER:  VMV_OSM_CNTS  Raw input counts.

SWITCHES:  none

INPUT:  --

_____________________________________________________________________________

_____________________________________________________________________________

4-9

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

HIGH SPEED DIGITAL INPUTS (HSI)

Each of the eight 8065 high-speed digital inputs (HSIs) can be configured  to

capture the state of the input signal after either (1) each transition with a

rising-edge or (2) each transition with either edge, rising or falling.   The

latter is the configuration of each HSI unless otherwise noted below.

All inputs, except HSI-0, are updated at 1ms (nominal) intervals and  at  all

PIP  edges.   HSI-0  is  updated at all PIP up and down edges.  Actual values

vary with engine operating conditions.

_____________________________________________________________________________

HSI-0

FUNCTION:  PIP           Profile Ignition Pickup signal.

REGISTER:  PIP_HIGH      Most recently captured state of input.

SWITCHES:  none

INPUT:  via EDIS

_____________________________________________________________________________

HSI-1

FUNCTION:  KS1           Knock Sensor.

REGISTER:  KNK_HIGH      Most recently captured state of input.

SWITCHES:  KIHP          Knock hardware present switch.

INPUT:  J1-57         via AICE.

_____________________________________________________________________________

HSI-2

FUNCTION:  unused

REGISTER:

SWITCHES:

INPUT:

_____________________________________________________________________________

HSI-3

FUNCTION:  unused

REGISTER:

SWITCHES:

INPUT:

_____________________________________________________________________________

HSI-4

FUNCTION:  unused

REGISTER:

SWITCHES:

INPUT:

_____________________________________________________________________________

(continued on next page)

4-10

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

_____________________________________________________________________________

HSI-5

FUNCTION:  unused

REGISTER:

SWITCHES:

INPUT:

_____________________________________________________________________________

HSI-6

FUNCTION:  CID1         Cylinder ID #1.

REGISTER:  CID_HIGH     Most recently captured state of input.

SWITCHES:  HP_CIDSEL    CID hardware type select -- determines type

of CID sensor (Hall Effect or VRS).

INPUT:  J1-85        Via AICE.

_____________________________________________________________________________

HSI-7

FUNCTION:  EDM          Electronic Diagnostic Monitor.

REGISTER:  EDM_HIGH     Most recently captured state of input.

SWITCHES:  none

INPUT:  via EDIS

_____________________________________________________________________________

_____________________________________________________________________________

4-11

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

ROM PORT INPUTS (RPA)

Each of the five  ROM-port  digital  inputs  is  individually  read  by  8065

software  at  1  ms  (nominal)  intervals.   They  are  typically filtered by

software for noise rejection.

_____________________________________________________________________________

RPA-0

FUNCTION:  FPM           Fuel Pump Secondary Circuit Monitor.

REGISTER:  F_PUMP_S_MON  Most recently captured state of input.

SWITCHES:  none

INPUT:  J1-40

_____________________________________________________________________________

RPA-1

FUNCTION:  TCS           Transmission Control (Over-Drive Cancel) Switch.

REGISTER:  ITCS          Most recently captured state of input.

SWITCHES:  none

INPUT:  J1-29

_____________________________________________________________________________

RPA-2

FUNCTION:  TR1  DIGITAL TRNS. RANGE SENSOR, BIT 1

REGISTER:  TR1

SWITCHES:  NONE

INPUT:  J1-17

_____________________________________________________________________________

RPA-3

FUNCTION:  OCTADJ        Octane Adjust.

REGISTER:  OCTADJ        Most recently captured state of input.

SWITCHES:  none

INPUT:  J1-30

_____________________________________________________________________________

RPA-4

FUNCTION:  LFCM (EDFM)   Low-Speed Fan Control Monitor (EDFM is old name).

REGISTER:  EDFM          Most recently captured state of input.

SWITCHES:  none

INPUT:  J1-04

_____________________________________________________________________________

_____________________________________________________________________________

4-12

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

HIGH SPEED DIGITAL OUTPUTS (HSO)

The 8065 has ten high-speed digital outputs (HSOs).  Software  schedules  HSO

events  and  calculates  the  corresponding  event  times  during  PIP and/or

time-slice interrupts.  HSO EVENTS are the rising-edge and falling-edge state

transitions  on  the  HSOs.   HSO event TIMES are the desired event execution

times, and are calculated directly in terms of 8065 clock  time.   The  event

times  are  used  by  the  CPU  in scheduling the HSO events via the 8065 HSO

'carousel'.  The carousel executes the HSO events  at  the  scheduled  times,

independent of current CPU operation.

There are no RAM parameters that show the desired or  actual  states  of  the

HSOs,  nor  are  there  any  that  show the 8065 clock times of the HSO state

transitions.  Output circuit fault data is not directly available from 8065

During HLOS, HSOs are in the OFF state.

_____________________________________________________________________________

HSO-0

FUNCTION:  INJ1          Output for injector #1.

REGISTER:  n/a

SWITCHES:  none

OUTPUT:  UPOD1-5

_____________________________________________________________________________

HSO-1

FUNCTION:  INJ2          Output for injector #2.

REGISTER:  n/a

SWITCHES:  none

OUTPUT:  UPOD1-4

_____________________________________________________________________________

HSO-2

FUNCTION:  INJ3          Output for injector #3.

REGISTER:  n/a

SWITCHES:  none

OUTPUT:  UPOD1-1

_____________________________________________________________________________

HSO-3

FUNCTION:  INJ4          Output for injector #4.

REGISTER:  n/a

SWITCHES:  none

OUTPUT:  UPOD1-0

_____________________________________________________________________________

(continued on next page)

4-13

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

_____________________________________________________________________________

HSO-4

FUNCTION:  INJ5          Output for injector #5.

REGISTER:  n/a

SWITCHES:  none

OUTPUT:  UPOD2-5

_____________________________________________________________________________

HSO-5

FUNCTION:  INJ6          Output for injector #6.

REGISTER:  n/a

SWITCHES:  none

OUTPUT:  UPOD2-4

_____________________________________________________________________________

HSO-6

FUNCTION:  INJ7          Output for injector #7.

REGISTER:  n/a

SWITCHES:  NUMCYL        Number of cylinders in engine.

OUTPUT:  UPOD2-1

_____________________________________________________________________________

HSO-7

FUNCTION:  INJ8          Output for Injector #8.

REGISTER:  n/a

SWITCHES:  NUMCYL        Number of cylinders in engine.

OUTPUT:  UPOD2-0

_____________________________________________________________________________

HSO-8

FUNCTION:  SAPW          Spark Advance Pulse-Width.

REGISTER:  n/a

SWITCHES:  none

OUTPUT:  to EDIS

_____________________________________________________________________________

HSO-9

FUNCTION:  KTS           Knock Threshold Select output.

REGISTER:  n/a

SWITCHES:  KIHP          Knock hardware present.

OUTPUT:  to AICE

_____________________________________________________________________________

_____________________________________________________________________________

4-14

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

LOW SPEED DIGITAL OUTPUTS (LSO)

Each of the eight 8065 LSOs are controlled directly by the 8065 CPU software.

The  CPU  controls  the  state  of  an  LSO  by  writing  a "1" or a "0" to a

memory-mapped location corresponding to the LSO.

When the 8065 is in I/O Expander mode with Memory Expansion, only one LSO  is

available.  It is updated at 1 ms (nominal) intervals.  The other outputs are

used for hardware functions required to  support  the  expanded  capabilities

(clocks, memory bank selects, etc.)

Output circuit fault data is not directly available from 8065 LSOs.

During HLOS, LSOs are in the OFF state.

_____________________________________________________________________________

LSO-3

FUNCTION:  DOL            Data Output Link.

REGISTER:  DOL            Commanded state of output.

SWITCHES:  DOLHP          DOL hardware present.

OUTPUT:  SSPOD1-0

_____________________________________________________________________________

_____________________________________________________________________________

4-15

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DUTY CYCLE EXPANDER OUTPUTS (DUCE/DCO)

The DUCE chip provides 8 programmable pulse width modulated outputs.  The CPU

controls  the  outputs by writing a period and pulse width to DUCE registers.

The pulse width for each is calculated in software from  an  associated  duty

cycle.   The  period  for  each  channel  is  controlled  by  a corresponding

calibration scalar (xxx_PRD_DT) whose units are 'DUCE ticks'.  If the  output

frequency of a particular DUCE channel is known, the period can be calculated

from the formula:

f(clock)

xxx_PRD_DT =   ----------------- - 1

54 * f(channel)

Where:   f(clock) = CPU clock frequency (Hz, example:  18,000,000)

f(channel) = Desired DUCE channel output frequency (Hz)

Optionally, DUCE channels 6 and 7 can be configured to output Variable  Force

Solenoid  (VFS)  data.   In  addition,  any  channel  can be configured as an

'on/off' output if its associated period scalar (xxx_PRD_DT) is set to 0.  In

this case, any non-zero duty cycle will turn the output on.

The output pins are listed for reference only.  Special methods must be  used

to monitor these outputs.

_____________________________________________________________________________

DCO-0

FUNCTION:  unused

REGISTER:

SWITCHES:

OUTPUT:

_____________________________________________________________________________

DCO-1

FUNCTION:  unused

REGISTER:

SWITCHES:

OUTPUT:

_____________________________________________________________________________

DCO-2

FUNCTION:  VMV-CC               Vapor Management Valve - Constant Current.

REGISTER:  PG_DC                Purge duty cycle.

VMV_PRD_DT           VMV signal period.

SWITCHES:  PCOMP_SW             PCOMP strategy present switch.

OUTPUT:  J1-56                (ISC-IC/ACSPD-IC)

_____________________________________________________________________________

(continued on next page)

4-16

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

_____________________________________________________________________________

DCO-3

FUNCTION:  MCC-H                Modulated Converter Clutch-High Impedence

REGISTER:  BCSDC_OUT            MCC duty cycle.

LUS_PRD_DT           MCC signal period.

SWITCHES:  TSTRAT               Transmission strategy switch.

OUTPUT:  UPOD3-5              Parallel input to UPOD.

_____________________________________________________________________________

DCO-4

FUNCTION:  unused

REGISTER:

SWITCHES:

OUTPUT:

_____________________________________________________________________________

DCO-5

FUNCTION:  unused

REGISTER:

SWITCHES:

OUTPUT:

_____________________________________________________________________________

DCO-6

FUNCTION:  EPCDA1               Electronic Pressure Control-VFS mode.

REGISTER:  TV_COUNTS            TV pressure counts to VFS.

SWITCHES:  TSTRAT               Transmission strategy switch.

OUTPUT:  J1-81

_____________________________________________________________________________

DCO-7

FUNCTION:  unused

REGISTER:

SWITCHES:

OUTPUT:

_____________________________________________________________________________

_____________________________________________________________________________

4-17

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

ANALOG INPUT CONDITIONER CALIBRATION (AICE)

The AICE IC provides dedicated input interfaces for  a  variety  of  sensors.

Commands  and  information  are communicated to the AICE via either dedicated

parallel lines or the Serial Link Address Protocol (SLAP) interface.

The AICE provides the A/D conversion for four HEGO sensor signals,  the  Mass

Air  Flow  (MAF)  sensor  signal,  and  eight additiona1 "external" A/D input

channels.  Each is described in the sections that follow.

When computing calibration constants, however,  other  AICE  inputs  must  be

considered.   The  AICE  has  inputs  and  detection  circuitry for two knock

sensors, a channel for  receiving  and  conditioning  the  Misfire  Detection

Sensor (MFDS), and a dedicated input for a standard PIP signal.

Parameter           Description

_____________________________________________________________________________

AICE_VERS           AICE IC version select switch.

(Correct setting is essential for proper operation.)

1 = AICE-1, 2 = AICE-2.

AICE_SETR1_B        Miscellaneous AICE function controls.

AICE_SETR1_C        Sensor mode and type selects.

AICE_SETR1_D        OSS sensor type select.

AICE_VRS_VAL        Sensor hysteresis values.

_____________________________________________________________________________

_____________________________________________________________________________

N O T E:

For specific hardware information, please refer  to  the  HCR(s)  and  Wiring

Diagram(s)  listed  in  the  APPLICATION  SUMMARY  on  the first page of this

chapter.

4-18

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

AICE ANALOG-TO-DIGITAL INPUTS (AICE A/D)

The AICE has analog inputs and conditioning circuits for  a  MAF  sensor  and

four HEGO sensors.  The signal on each of these input pins is conditioned and

then multiplexed into the AICE's analog-to-digital converter.

A/D conversions of the MAF,  HEGO1I,  and  HEGO2I  inputs  are  initiated  by

software  or by hardware (at PIP transitions).  HEGO3I and HEGO4I conversions

are done at the request of software only.

_____________________________________________________________________________

A/D-1  MAFI

FUNCTION:  MAF           Mass Air Flow Sensor.

REGISTER:  IMAF          Raw input counts.

SWITCHES:  none

INPUT:  J1-88/36      88-MAF;  36-MAFRTN.

_____________________________________________________________________________

A/D-2  HEGO1I

FUNCTION:  HO2S-11       Bank 1 front heated O2 sensor (HEGO-11).

REGISTER:  IEGO11        Raw input counts.

SWITCHES:  none

INPUT:  J1-60

_____________________________________________________________________________

A/D-3  HEGO2I

FUNCTION:  HO2S-21       Bank 2 front heated O2 sensor (HEGO-21).

REGISTER:  IEGO21        Raw input counts.

SWITCHES:  none

INPUT:  J1-87

_____________________________________________________________________________

A/D-4  HEGO3I

FUNCTION:  HO2S-12       Bank 1 rear heated O2 sensor (HEGO-12).

REGISTER:  IEGO12        Raw input counts.

SWITCHES:  none

INPUT:  J1-35

_____________________________________________________________________________

A/D-5  HEGO4I

FUNCTION:  HO2S-22       Bank 2 rear heated O2 sensor (HEGO-22).

REGISTER:  IEGO22        Raw input counts.

SWITCHES:  none

INPUT:  J1-61

_____________________________________________________________________________

_____________________________________________________________________________

4-19

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

AICE EXTERNAL ANALOG-TO-DIGITAL INPUTS (AICE E-A/D)

The AICE IC has eight "external" general-purpose analog input channels.  Each

input has an applied voltage range of 0-5 volts, and can be read as either an

analog or a digital input.  If the channel  is  configured  as  digital,  the

high/low center point value is 2.5 volts with a hysteresis of 1 volt.  If the

channel is configured as  analog,  the  digital  representation  of  the  pin

voltage  has  a  range  of 0-1023 'A/D counts' and is mapped linearly with no

offset.  Overall gain is 1.0.

A/D conversion of the voltage on any of these inputs is performed in response

to a request from software.

_____________________________________________________________________________

E-A/D-0

FUNCTION:  IVPWR         Vehicle Power.

REGISTER:  IIVPWR        Raw input counts.

SWITCHES:  none

INPUT:  --            ANALOG.

_____________________________________________________________________________

E-A/D-1

FUNCTION:  EPCOCM1       EPC Over-Current Monitor.

REGISTER:  EPCOCM1_CNTS  Raw input counts.

SWITCHES:  none

INPUT:  --            ANALOG.

_____________________________________________________________________________

E-A/D-2

FUNCTION:  unused

REGISTER:

SWITCHES:

INPUT:

_____________________________________________________________________________

E-A/D-3

FUNCTION:  BOO           Brake on/off.

REGISTER:  BOO_LVL       Most recently captured state of input.

SWITCHES:  none

INPUT:  J1-92         DIGITAL.

_____________________________________________________________________________

E-A/D-4

FUNCTION:  ACCS          A/C Clutch Switch.

REGISTER:  ACCS          Most recently captured state of input.

SWITCHES:  none

INPUT:  J1-41         DIGITAL.

_____________________________________________________________________________

(continued on next page)

4-20

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

_____________________________________________________________________________

E-A/D-5

FUNCTION:  PFSNS            Purge Flow Sensor

REGISTER:  PFS_CNTS         Raw input counts.

SWITCHES:  none

INPUT:  J1-11            ANALOG

_____________________________________________________________________________

E-A/D-6

FUNCTION:  unused

REGISTER:

SWITCHES:

INPUT:

_____________________________________________________________________________

E-A/D-7

FUNCTION:  unused

REGISTER:

SWITCHES:

INPUT:

_____________________________________________________________________________

_____________________________________________________________________________

4-21

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DUAL ACCESS RAM CONTROLLER (DARC)

The DARC-I/O LSI provides the following, each of  which  is  described  in  a

section to follow:

o  6 configurable parallel bidirectional digital I/Os (PI/O).

o  12 configurable multiplexed parallel bidirectional digital I/Os (MPI/O).

o  8 programmable pulse-width  modulated  outputs  (with  configurable  HLOS

states) (PWM).

o  6 Delta-Time capture Inputs (DTI).

The  DARC  also  provides  a  Serial  Peripheral  Interface  (SPI)  port  for

communication  of  commanded  output states and output fault data between the

DARC and EEC-V module power output driver devices.

DARC BI-DIRECTIONAL PARALLEL I/O PORT (DUCE PI/O)

The DARC chip provides a memory-mapped six-channel bidirectional parallel I/O

port.   Each  I/O channel can be programmed to serve as either an input or an

output.  The inputs are filtered for noise rejection,  and  may  be  inverted

prior to filtering.

_____________________________________________________________________________

PI/O-0

FUNCTION:  TR2  DIGITAL TRNS. RANGE SENSOR, BIT 2 (INPUT)

REGISTER:  TR2

SWITCHES:  NONE

I/O VIA:  J1-49

_____________________________________________________________________________

PI/O-1

FUNCTION:  unused.

REGISTER:

SWITCHES:

I/O VIA:

_____________________________________________________________________________

PI/O-2

FUNCTION:  4X4L           Four wheel drive, low input switch.

REGISTER:  I4X4L          Most recently captured state of input.

SWITCHES:  none

I/O VIA:  J1-14          INPUT (INVERTED).

_____________________________________________________________________________

(continued on next page)

4-22

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

_____________________________________________________________________________

PI/O-3

FUNCTION:  EAMM           Electric Air Management Secondary Circuit Mon.

REGISTER:  EAMM           Most recently captured state of input.

SWITCHES:  none

I/O VIA:  J1-05          INPUT.

_____________________________________________________________________________

PI/O-4

FUNCTION:  TR4  DIGITAL TRNS. RANGE SENSOR, BIT 4

REGISTER:  TR4

SWITCHES:  NONE

I/O VIA:  J1-50

_____________________________________________________________________________

PI/O-5

FUNCTION:  unused

REGISTER:

SWITCHES:

I/O VIA:

_____________________________________________________________________________

_____________________________________________________________________________

4-23

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DARC DELTA-TIME INPUTS (DARC DTI)

The DARC chip provides 6 channels of timer-capture inputs that  automatically

compute  a  24-bit  delta-time  for  each channel.  The 24-bit delta-time and

24-bit  prior  delta-time  are  available  to  software  via  memory   mapped

locations.  These locations are not, however, accessible via RCON/VDAS.  Each

of the six DTI's can be configured to identify each rising, each  falling  or

each of both types of input signal transition edges.

_____________________________________________________________________________

DTI-0

FUNCTION:  unused

REGISTER:

SWITCHES:

INPUT:

_____________________________________________________________________________

DTI-1

FUNCTION:  unused

REGISTER:

SWITCHES:

INPUT:

_____________________________________________________________________________

DTI-2

FUNCTION:  VSS+/-        Vehicle Speed Sensor.

REGISTER:  n/a

SWITCHES:  none

INPUT:  J1-58/33      58-VSS+;  33-VSS-.

_____________________________________________________________________________

DTI-3

FUNCTION:  unused

REGISTER:

SWITCHES:

INPUT:

_____________________________________________________________________________

DTI-4

FUNCTION:  OSS           Output Shaft Speed Sensor.

REGISTER:  n/a

SWITCHES:  none

INPUT:  J1-84

_____________________________________________________________________________

DTI-5

FUNCTION:  unused

REGISTER:

SWITCHES:

INPUT:

_____________________________________________________________________________

_____________________________________________________________________________

4-24

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DARC PULSE-WIDTH MODULATED OUTPUTS (DARC PWM)

The DARC chip provides a general purpose pulse  width  generator  with  eight

individually  software-programmable  PWM  output  channels.  The CPU controls

each output by writing a period and pulsewidth to registers on the DARC chip.

The  output  period  or  frequency of each channel is calibratible (units are

seconds or Hertz, respectively).  The pulsewidth is  calculated  in  software

from an associated duty cycle.

_____________________________________________________________________________

PWM-0

FUNCTION:  unused

REGISTER:

SWITCHES:

OUTPUT:

_____________________________________________________________________________

PWM-1

FUNCTION:  FP            Fuel Pump

REGISTER:  FPUMP_DC      Fuel Pump output duty cycle.

FPUMP_FREQ    Fuel Pump output signal frequency.

SWITCHES:  none

OUTPUT:  SSPOD1-1      Parallel input to SSPOD1.

_____________________________________________________________________________

PWM-2

FUNCTION:  unused

REGISTER:

SWITCHES:

OUTPUT:

_____________________________________________________________________________

PWM-3

FUNCTION:  IMCC          Intake Manifold Communicator Control

REGISTER:  IMCC_DC       IMCC output duty cycle.

IMCC_FREQ     IMCC output signal frequency.

SWITCHES:  IMCCHP        IMCC hardware present switch.

OUTPUT:  UPOD2-2       Parallel input to UPOD2.

_____________________________________________________________________________

(continued on next page)

4-25

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

_____________________________________________________________________________

PWM-4

FUNCTION:  unused

REGISTER:

SWITCHES:

OUTPUT:

_____________________________________________________________________________

PWM-5

FUNCTION:  unused

REGISTER:

SWITCHES:

OUTPUT:

_____________________________________________________________________________

PWM-6

FUNCTION:  unused

REGISTER:

SWITCHES:

OUTPUT:

_____________________________________________________________________________

PWM-7

FUNCTION:  unused

REGISTER:

SWITCHES:

OUTPUT:

_____________________________________________________________________________

_____________________________________________________________________________

4-26

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DARC MULTIPLEXED PARALLEL PORT (DARC MPI/O)

The DARC chip provides a memory-mapped multiplexed  parallel  port.   The  MP

port  provides  12  general-purpose  bidirectional  I/O  channels that can be

individually programmed as either inputs or outputs.  Currently, 8 of the  12

channels  (0-7)  are  used  by  software to enable the various driver devices

(UPOD, SSPOD, SCCD).  The four remaining are unused at this time.

_____________________________________________________________________________

MPI/O-8

FUNCTION:  unused

REGISTER:

SWITCHES:

I/O:

_____________________________________________________________________________

MPI/O-9

FUNCTION:  unused

REGISTER:

SWITCHES:

I/O:

_____________________________________________________________________________

MPI/O-10

FUNCTION:  unused

REGISTER:

SWITCHES:

I/O:

_____________________________________________________________________________

MPI/O-11

FUNCTION:  unused

REGISTER:

SWITCHES:

I/O:

_____________________________________________________________________________

_____________________________________________________________________________

4-27

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

UNIVERSAL POWER OUTPUT DRIVER (UPOD)

Each UPOD provides 6 outputs that may be controlled serially  (via  the  DARC

chip) or by dedicated parallel inputs.  The "CONTROL" field indicates whether

an input is serial or  parallel.   If  an  input  is  serial,  the  parameter

containing  the  commanded  output state is given in the "COMMAND REG" field.

If an input is parallel, the CPU/DARC/DUCE source of the output is listed  in

the "SOURCE" field.

UPODs will return output fault information  to  the  DARC.   Typically,  this

information  is  stored  in  RAM.  The fault status parameter is given in the

"FAULT REG" field.

_____________________________________________________________________________

UPOD1-0

FUNCTION:  INJ4          Injector #4.

CONTROL:  PARALLEL

SOURCE:  HSO-3

FAULT REG:  INJ4_FAULT    Fault status of output driver.

SWITCHES:  none

OUTPUT:  J1-100

_____________________________________________________________________________

UPOD1-1

FUNCTION:  INJ3          Injector #3.

CONTROL:  PARALLEL

SOURCE:  HSO-2

FAULT REG:  INJ3_FAULT    Fault status of output driver.

SWITCHES:  none

OUTPUT:  J1-74

_____________________________________________________________________________

(continued on next page)

4-28

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

_____________________________________________________________________________

UPOD1-2

FUNCTION:  LFC (EDF)     Low Fan Control (formerly Electro-Drive Fan)

CONTROL:  SERIAL

COMMAND REG:  EDF           Commanded state of output.

FAULT REG:  EDF_STAT1     Fault status of output driver.

SWITCHES:  EDFHP = 1     EDF hardware present switch.

OUTPUT:  J1-45

_____________________________________________________________________________

UPOD1-3

FUNCTION:  MIL           Malfunction Indicator Light.

CONTROL:  SERIAL

COMMAND REG:  MIL           Commanded state of output.

FAULT REG:  MIL_FAULT     Fault status of output driver.

SWITCHES:  none

OUTPUT:  J1-02

_____________________________________________________________________________

UPOD1-4

FUNCTION:  INJ2          Injector #2.

CONTROL:  PARALLEL

SOURCE:  HSO-1

FAULT REG:  INJ2_FAULT    Fault status of output driver.

SWITCHES:  none

OUTPUT:  J1-101

_____________________________________________________________________________

(continued on next page)

4-29

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

_____________________________________________________________________________

UPOD1-5

FUNCTION:  INJ1          Injector #1.

CONTROL:  PARALLEL

SOURCE:  HSO-0

FAULT REG:  INJ1_FAULT    Fault status of output driver.

SWITCHES:  none

OUTPUT:  J1-75

-----------------------------------------------------------------------------

UPOD2-0

FUNCTION:  INJ8          Injector #8.

CONTROL:  PARALLEL

SOURCE:  HSO-7

FAULT REG:  INJ8_FAULT    Fault status of output driver.

SWITCHES:  NUMCYL        Number of cylinders in engine.

OUTPUT:  J1-98

_____________________________________________________________________________

UPOD2-1

FUNCTION:  INJ7          Injector #7.

CONTROL:  PARALLEL

SOURCE:  HSO-6

FAULT REG:  INJ7_FAULT    Fault status of output driver.

SWITCHES:  NUMCYL        Number of cylinders in engine.

OUTPUT:  J1-72

_____________________________________________________________________________

(continued on next page)

4-30

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

_____________________________________________________________________________

UPOD2-2

FUNCTION:  HFC (HEDF)    High Fan Control (formerly High Speed EDF)

CONTROL:  SERIAL

COMMAND REG:  HEDF          Commanded state of output.

FAULT REG:  HEDF_STATUS   Fault status of output driver.

SWITCHES:  HEDFHP        HEDF hardware present switch.

IMCCHP        IMCC hardware present switch.

OUTPUT:  J1-46

FUNCTION:  IMCC          Intake Manifold Communitor Control

CONTROL:  PARALLEL

SOURCE:  PWM-3

FAULT REG:  IMCC_FAULT    Fault status of output driver.

SWITCHES:  IMCCHP        IMCC hardware present switch.

OUTPUT:  J1-46

_____________________________________________________________________________

UPOD2-3

FUNCTION:  IMRC          Intake Manifold Runner Control.

CONTROL:  SERIAL

COMMAND REG:  IMRC_OUT      Commanded state of output.

FAULT REG:  IMRC_FAULT    Fault status of output driver.

SWITCHES:  IMRCHP        IMRC hardware present.

OUTPUT:  J1-42

_____________________________________________________________________________

UPOD2-4

FUNCTION:  INJ6          Injector #6.

CONTROL:  PARALLEL

SOURCE:  HSO-5

FAULT REG:  INJ6_FAULT    Fault status of output driver.

SWITCHES:  none

OUTPUT:  J1-99

_____________________________________________________________________________

(continued on next page)

4-31

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

_____________________________________________________________________________

UPOD2-5

FUNCTION:  INJ5          Injector #5.

CONTROL:  PARALLEL

SOURCE:  HSO-4

FAULT REG:  INJ5_FAULT    Fault status of output driver.

SWITCHES:  none

OUTPUT:  J1-73

-----------------------------------------------------------------------------

UPOD3-0

FUNCTION:  SS1           Shift Solenoid #1.

CONTROL:  SERIAL

COMMAND REG:  FLG_SS_1      Commanded state of output.

FAULT REG:  SS1_FAULT     Fault status of output driver.

SWITCHES:  TSTRAT        Transmission strategy switch.

OUTPUT:  J1-27

_____________________________________________________________________________

UPOD3-1

FUNCTION:  SS2           Shift Solenoid #2.

CONTROL:  SERIAL

COMMAND REG:  FLG_SS_2      Commanded state of output.

FAULT REG:  SS2_FAULT     Fault status of output driver.

SWITCHES:  TSTRAT        Transmission strategy switch.

OUTPUT:  J1-01

_____________________________________________________________________________

UPOD3-2

FUNCTION:  RLS           Reverse Lockout Solenoid

CONTROL:  SERIAL

COMMAND REG:  FLG_RLS       Commended state of output.

FAULT REG:  none

SWITCHES:  TSTRAT        Transmission strategy switch.

OUTPUT:  J1-53

_____________________________________________________________________________

UPOD3-3

FUNCTION:  unused

CONTROL:

COMMAND REG:

FAULT REG:

SWITCHES:

OUTPUT:

_____________________________________________________________________________

(continued on next page)

4-32

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

_____________________________________________________________________________

UPOD3-4

FUNCTION:  TCIL          Transmission Control Indicator Light.

CONTROL:  SERIAL

COMMAND REG:  TCIL_B        Commanded state of output.

FAULT REG:  TCIL_FAULT    Fault status of output driver.

SWITCHES:  TSTRAT        Transmission strategy switch.

OUTPUT:  J1-79

FUNCTION:  FPL           Fuel Pump high/low speed control.

CONTROL:  SERIAL

COMMAND REG:  FPL           Commanded state of output.

FAULT REG:  FP_2SPD_ERR   Fault status of output driver.

SWITCHES:  FP_TYPE       Fuel pump type.

OUTPUT:  J1-79

_____________________________________________________________________________

UPOD3-5

FUNCTION:  TCC-H         Torque Converter Clutch - High impedence.

CONTROL:  PARALLEL

SOURCE:  DCO-3

FAULT REG:  TCC_H_FAULT   Fault status of output driver.

SWITCHES:  TSTRAT        Transmission strategy switch.

OUTPUT:  J1-54

_____________________________________________________________________________

_____________________________________________________________________________

4-33

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SIX SERIAL/PARALLEL OUTPUT DRIVER (SSPOD)

Each SSPOD provides 6 outputs which may be controlled serially (via the  DARC

chip).   Alternatively,  the  first three channels (0-2) on each SSPOD may be

controlled by dedicated  parallel  inputs.   The  "CONTROL"  field  indicates

whether an input is serial or parallel.  If an input is serial, the parameter

containing the commanded output state is given in the  "CONTROL  REG"  field.

If  an input is parallel, the CPU/DARC/DUCE source of the output is listed in

the "SOURCE" field.

SSPODs will return output fault information to  the  DARC.   Typically,  this

information  is  stored  in  RAM.  The fault status parameter is given in the

"FAULT REG" field.

_____________________________________________________________________________

SSPOD1-0

FUNCTION:  DOL           Data Output Link.

CONTROL:  PARALLEL

SOURCE:  LSO-3

FAULT REG:  DOL_FAULT     Fault status of output driver.

SWITCHES:  none

OUTPUT:  J1-43

_____________________________________________________________________________

SSPOD1-1

FUNCTION:  FP            Fuel Pump

FSV           Fuel Solenoid Valve

CONTROL:  PARALLEL

SOURCE:  PWM-1

FAULT REG:  F_PUMP_ERROR

SWITCHES:  none

OUTPUT:  J1-80

_____________________________________________________________________________

SSPOD1-2

FUNCTION:  WAC           A/C cutout relay.

CONTROL:  SERIAL

COMMAND REG:  ACR           Commanded state of output.

FAULT REG:  ACC_STATUS    Fault status of output driver.

SWITCHES:  none

OUTPUT:  J1-69

_____________________________________________________________________________

(continued on next page)

4-34

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

_____________________________________________________________________________

SSPOD1-3

FUNCTION:  ACL           Acceleration output for Ride Control

CONTROL:  SERIAL

COMMAND REG:  ACL           Commanded state of output.

FAULT REG:  none

SWITCHES:  HPACL         Ride Control hardware present switch.

OUTPUT:  J1-19

FUNCTION:  SWC           Speed Warning Chime

CONTROL:  SERIAL

COMMAND REG:  SWC           Commanded state of output.

FAULT REG:  SWC_FAULT     Fault status of output driver.

SWITCHES:  SWC_HP        Speed Warning Chime hardware present switch.

OUTPUT:  J1-19

_____________________________________________________________________________

SSPOD1-4

FUNCTION:  CANP          Canister Purge Solenoid.

CANVNT        Canister Vent Valve.

CONTROL:  SERIAL        Serviced at 1ms intervals.

COMMAND REG:  CANVNT_CMD    Commanded state of output.

FAULT REG:  CANP_FAULT    Fault status of output driver.

CANVT_FAULT   Fault status of output driver.

SWITCHES:  PCOMP_SW      PCOMP strategy present switch.

OUTPUT:  J1-67

_____________________________________________________________________________

SSPOD1-5

FUNCTION:  EAIR (EAM)    Electric Secondary Air Injection

(formerly Electric Air Management)

CONTROL:  SERIAL

COMMAND REG:  EAM           Commanded state of output.

FAULT REG:  EAM_FAULT     Fault status of output driver.

SWITCHES:  THRMHP        Thermactor hardware present.

OUTPUT:  J1-70

_____________________________________________________________________________

_____________________________________________________________________________

4-35

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SWITCHING CONSTANT CURRENT DRIVER (SCCD)

The SCCD provides two average-current outputs (SCCD-A, SCCD-B) used to  drive

inductive loads.

During Normal operation, a digital value  corresponding  to  a  desired  load

current  is  sent  to  the  SCCD  via  the  SPI.  The SCCD creates a sawtooth

waveform by oscillating the output current (+/-10%) around the digital value.

This  results  in  an  average current output equal to the desired value.  By

using this method, a constant current is supplied to the inductive loads  but

with a lower power dissipation in the drivers.

In Normal mode, the SCCD can detect an over-current fault  (shorted  load  or

short-to-Vbatt)  on  an  output  while  the  output  is  ON,  and  can detect

open-circuit and short-to-ground faults on an output when it is  ON  or  OFF.

The fault information is sent from the SCCD to the CPU via the SPI.

_____________________________________________________________________________

SCCD-A

FUNCTION:  EVR-CC        Exhaust Gas Recirculation - Constant Current.

COMMAND REG:  EVR_CURRENT   Percent current register.

FAULT REG:  EVR_OPEN      Open-circuit fault status of output driver.

(Bit 5 of fault data from SCCD via SPI)

EVR_SHORT     Short-circuit fault status of output driver.

(Bit 4 of fault data from SCCD via SPI)

SWITCHES:  none

OUTPUT:  J1-47

_____________________________________________________________________________

SCCD-B

FUNCTION:  IAC (ISCBPA)  Idle Air Control

(formerly Idle Speed Control - Bypass Air)

COMMAND REG:  ISC_CURRENT   ISC percent current register.

FAULT REG:  IAC_NO_CUR    Open circuit or short-to-ground detected.

(Bit 3 of fault data from SCCD via SPI)

IAC_OVER_CUR  Shorted load or short-to-VPWR detected.

(Bit 2 of fault data from SCCD via SPI)

SWITCHES:  none

OUTPUT:  J1-83

_____________________________________________________________________________

_____________________________________________________________________________

4-36

HARDWARE INTERFACE, EEC-V I/O CHANNEL ASSIGNMENTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

INTELLIGENT MOSFET PRE-DRIVER (IMP)

The IMP IC has four outputs which can control  external  MOSFET  drivers  for

four  external loads.  The four outputs are controllable by means of periodic

serial communication from the  DARC  via  the  SPI.   The  four  outputs  are

configured  in  pairs.   The  IMP can detect over-current faults on an output

pair while the pair is commanded ON,  and  open-circuit  and  short-to-ground

faults  on  an output pair when the pair is commanded OFF.  The IMP transmits

to the DARC (via the SPI) the fault status of each of its outputs.

_____________________________________________________________________________

IMP-0

FUNCTION:  HTR-11        Bank 1 front HEGO heater

COMMAND REG:  HEGOHTR11     Commanded state of output.

FAULT REG:  HTR_FAULT11   Fault status of output driver.

SWITCHES:  none

OUTPUT:  J1-93

_____________________________________________________________________________

IMP-1

FUNCTION:  HTR-21        Bank 2 front HEGO heater

COMMAND REG:  HEGOHTR21     Commanded state of output.

FAULT REG:  HTR_FAULT21   Fault status of output driver.

SWITCHES:  none

OUTPUT:  J1-94

_____________________________________________________________________________

IMP-2

FUNCTION:  HTR-12        Bank 1 rear HEGO heater

COMMAND REG:  HEGOHTR12     Commanded state of output.

FAULT REG:  HTR_FAULT12   Fault status of output driver.

SWITCHES:  none

OUTPUT:  J1-95

_____________________________________________________________________________

IMP-3

FUNCTION:  HTR-22        Bank 2 rear HEGO heater

COMMAND REG:  HEGOHTR22     Commanded state of output.

FAULT REG:  HTR_FAULT22   Fault status of output driver.

SWITCHES:  none

OUTPUT:  J1-96

_____________________________________________________________________________

_____________________________________________________________________________

4-37

HARDWARE INTERFACE, HARDWARE-SPECIFIC INPUT PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

4.2  HARDWARE-SPECIFIC INPUT PROCESSING (CDAN0)

OVERVIEW

Input  functions  which  require   special   processing   due   to   hardware

configuration are listed below.

DEFINITIONS

Registers:

-  ICHAN_1 = 8065 Analog input, channel 1, counts.

-  IPSPS = Power steering analog input.

-  TPR_CNTS = Tank pressure sensor input counts.

Bit Flags:

-  CURRENT_CKD = Semaphore to signal when the HTRCM data is available.

-  EDF = Electro-Drive Fan output driver #1 commanded state; 1 -> driver ON.

-  EDF_STATUS = EDF output driver fault status.

-  PSPS_LVL = Power steering pressure switch input.

4-38

HARDWARE INTERFACE, HARDWARE-SPECIFIC INPUT PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: HWINT_INPUT_CD

ANALOG CHANNEL 1 INPUT SELECTION

Analog channel 1 provides Fuel Tank Pressure input.  This logic is  performed

once per background loop.

unconditionally -----------------------| TPR_CNTS := ICHAN_1

POWER STEERING PRESSURE SWITCH STATE DETERMINATION

The analog input IPSPS is converted to the bit flag  PSPS_LVL  based  upon  a

switchpoint of 512 A/D counts (~2.5 volts).

IPSPS > 512 counts --------------------| PSPS_LVL := 1

|

| --- ELSE ---

|

| PSPS_LVL := 0

4-39

HARDWARE INTERFACE, HARDWARE-SPECIFIC INPUT PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

LOW SPEED FAN OUTPUT DRIVER FAULT STATUS PROCESSING

The Low Speed Electro-Drive Fan (EDF) is directed to one of two UPOD outputs,

depending  upon  the  calibration of VCAMHP.  Since each output furnishes its

own fault information, the actual EDF output  driver  fault  status  must  be

determined based upon which EDF output is used.

To further complicate matters, the UPOD circuitry will erroneously indicate a

fault when the driver is ON and the output impedence is greater than 45 ohms.

Some CD applications utilize a IRCM which has an output impedence of 60 -  80

ohms.   For  this  reason,  the fault status of the EDF output driver is only

saved if the driver state is OFF.

EDF = 0 -------------------------|

;driver is OFF                  |

;fault status is valid          |AND -| EDF_STATUS := 0

|     |  ;indicates driver OK

edf_fault = 0 -------------------|     |

;no fault present                     |

| --- ELSE ---

EDF = 0 -------------------------|     |

;driver is OFF                  |     |

;fault status is valid          |AND -| EDF_STATUS := 1

|     |  ;indicate driver fault

edf_fault = 1 -------------------|

;fault is present

4-40

HARDWARE INTERFACE, HARDWARE-SPECIFIC INPUT PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

HEATER CURRENT MONITOR SEMAPHORE

This logic is necessary for Current Monitoring  strategy  compatibility  with

both ML-1 and ML-2 module input configurations.

Analog channel 12 is read once per background loop and software transfers the

ACH-12  data  to  HTRCM_CNTS.  The logic below sets a semaphore to notify the

Current Monitoring strategy that data is available.

Once per background loop:

unconditionally ----------------------| CURRENT_CKD := 1

4-41

HARDWARE INTERFACE, HARDWARE-SPECIFIC OUTPUT PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

4.3  HARDWARE-SPECIFIC OUTPUT PROCESSING (CDAN0)

OVERVIEW

Outputs which require special processing due to hardware  configurations  are

listed below.

DEFINITIONS

Registers:

-  DOL_COUNT = Number of pulses to be output to  the  fuel  economy  display

device.

-  EGRDC = EGR duty-cycle.

-  EVR_CURRENT = SCCD percent current register for EGR EVR.

-  ISC_CURRENT = SCCD percent current register for ISC.

-  ISCDTY = Idle speed duty cycle.

-  PG_DC = Canister Purge duty cycle.

-  PGM_CVS_DC = Purge monitor canister vent solenoid duty cycle.

-  PRG_ON_TIME = Time per output period  that  CANP/CANVNT  output  is  held

high, clock ticks.

-  PRG_PERIOD = CANP/CANVNT output period, clock ticks.

-  START_CVS_PER = Time stamp of beginning of last  CANP/CVS  period,  clock

ticks.

-  VMV_PW_DTKS = VMV pulsewidth, DUCE ticks.

-  VMV_PW_DUCE = DUCE hardware register for VMV output.

Bit Flags:

-  CANVT_CMD = Commanded CANVNT output state; 1 -> output energized.

-  CHTIL_CMD = Desired state of Cylinder Head Temperature Indicator Light; 1

-> CHTIL on.

-  CHTIL = Cylinder Head Temperature Indicator  Light  output  state;  1  ->

output energized.

-  DOL = Commanded Data Output Link output state; 1 -> output energized.

-  PRG_OUT_ENA = Foreground CANP/CANVNT output routine enable flag.

4-42

HARDWARE INTERFACE, HARDWARE-SPECIFIC OUTPUT PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  CANP_PRD_DT = Output period of CANP signal, DUCE ticks.

-  COOL_CAL_SW = Fail Safe Cooling strategy enable; 1 -> enabled.

-  CVS_PERIOD = Output period of Canister Vent Valve solenoid; clock ticks.

-  DOLHP = Data Output Link hardware present switch.

-  VMV_PRD_DT = Output period of VMV signal, DUCE ticks.

4-43

HARDWARE INTERFACE, HARDWARE-SPECIFIC OUTPUT PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: HWINT_OUTPUT_CD

CYLINDER HEAD TEMPERATURE INDICATOR LIGHT OUTPUT PROCESSING

Cylinder Head Temperature Indicator Light (CHTIL) is controlled by the  value

of  CHTIL_CMD,  provided  the feature is calibrated in.  NOTE:  This function

shares an output with EDF and VCT1.  The software  makes  no  provisions  for

enabling more than one of these functions.

COOL_CAL_SW = 1 ------------------|

|AND -| CHTIL := 0

CHTIL_CMD = 0 --------------------|     |

| --- ELSE ---

|

COOL_CAL_SW = 1 ------------------------| CHTIL := 1

IDLE SPEED CONTROL BYPASS AIR SOLENOID OUTPUT PROCESSING

ISCDTY is translated in software to a 'percent current', ISC_CURRENT, using a

SCCD current conversion factor (sccf).

always ---------------------------------| ISC_CURRENT := sccf(ISCDTY)

EGR ELECTRONIC VACUUM REGULATOR (EVR) OUTPUT PROCESSING

EGRDC is translated in software to a 'percent current', EVR_CURRENT, using  a

SCCD current conversion factor (sccf).

always ---------------------------------| EVR_CURRENT := sccf(EGRDC)

VAPOR MANAGEMENT VALVE DUCE OUTPUT PROCESSING

The Vapor Management Valve (VMV) signal is output on  a  DUCE  channel.   The

on-time  of  the  signal  is  computed  in  software  from the VMV strategy's

calibrated output period and its calculated duty-cycle.  The on-time is  then

written by software to a memory mapped dedicated register on the DUCE chip.

BEGIN: vapor_management_valve_output

; This process is executed only when called.

unconditionally ------------------------| VMV_PW_DTKS := PG_DC * VMV_PRD_DT

|  ;clip to 65535 duce_ticks

|

| VMV_PW_DUCE := VMV_PW_DTKS

END: vapor_management_valve_output

4-44

HARDWARE INTERFACE, HARDWARE-SPECIFIC OUTPUT PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DATA OUTPUT LINK OUTPUT PROCESSING

The Data Output Link outputs a number of pulses proportional to the amount of

fuel  consumed.  The register DOL_COUNT contains the current number of pulses

needing to be output.  The duration of each pulse is approximately 1 ms.

; Once per millisecond:

DOLHP = 1 ------------------------|

|

DOL_COUNT > 0.5 ------------------|AND -| DOL_COUNT := DOL_COUNT - 0.5

|     | DOL := 1

DOL = 0 --------------------------|     |

| --- ELSE ---

DOLHP = 1 ------------------------|     |

|AND -| DOL_COUNT := DOL_COUNT - 0.5

DOL_COUNT > 0.5 ------------------|     | DOL := 0

|

| --- ELSE ---

|

DOLHP = 1 ------------------------------| DOL_COUNT := 0.5

4-45

HARDWARE INTERFACE, HARDWARE-SPECIFIC OUTPUT PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CANISTER PURGE SOLENOID OUTPUT PROCESSING

Canister Purge Solenoid, (CANP) shares a serially  controlled  SSPOD  channel

with Canister Vent Valve (CANVNT).

The commanded output is updated once every 20 ms when  PRG_FLG_ENA  =  0,  or

every 1 ms when PRG_FLG_ENA = 1.

This process is called from the Canister Purge Selection logic.

BEGIN: canister_purge_output

; If the use of a Canister Purge solenoid (CANP) is desired, the

; period of the output signal is determined by converting the scalar

; CANP_PRD_DT from DUCE ticks to clock ticks.

; This process is executed only when called.

PG_DC = 0 ------------------------------| PRG_OUT_ENA := 0

; CANP full off                         | ; Disable 1 ms output routine.

| CANVT_CMD := 0

| ; Turn CANP off during next

| ; 20 ms timeslice.

|

| --- ELSE ---

|

PG_DC = 1.0 ----------------------------| PRG_OUT_ENA := 0

; CANP full on                          | ; Disable 1 ms output routine.

| CANVT_CMD := 1

| ; Turn CANP on during next

| ; 20 ms timeslice.

|

| --- ELSE ---

|

| PRG_PERIOD := CANP_PRD_DT * 9 / 16

| ; Convert CANP period from DUCE

| ; ticks to clock ticks.

| PRG_ON_TIME := PRG_PERIOD * PG_DC

| PRG_OUT_ENA := 1

| ; Enable 1 ms output routine.

END: canister_purge_output

4-46

HARDWARE INTERFACE, HARDWARE-SPECIFIC OUTPUT PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CANISTER VENT VALVE OUTPUT PROCESSING

Canister Vent Valve (CANVNT) shares a serially controlled SSPOD channel  with

Canister Purge Solenoid (CANP).

The commanded output is updated once every 20 ms when  PRG_FLG_ENA  =  0,  or

every 1 ms when PRG_FLG_ENA = 1.

This process is called from the Canister Purge Selection logic.

BEGIN: canister_vent_valve_output

; If the use of a Canister Vent Valve (CANVNT) is desired, the period of

; the output signal is the scalar CVS_PERIOD (units are clock ticks).

; This process is executed only when called.

PGM_CVS_DC = 0 -------------------------| PRG_OUT_ENA := 0

; CANVNT full off                       | ; Disable 1 ms output routine.

| CANVT_CMD := 0

| ; Turn CANVNT off during next

| ; 20 ms timeslice.

|

| --- ELSE ---

|

PGM_CVS_DC = 1.0 -----------------------| PRG_OUT_ENA := 0

; CANVNT full on                        | ; Disable 1 ms output routine.

| CANVT_CMD := 1

| ; Turn CANVNT on during next

| ; 20 ms timeslice.

|

| --- ELSE ---

|

| PRG_PERIOD := CVS_PERIOD

| PRG_ON_TIME := PRG_PERIOD *

|                 PGM_CVS_DC

| PRG_OUT_ENA := 1

| ; Enable 1 ms output routine.

END: canister_vent_valve_output

4-47

HARDWARE INTERFACE, HARDWARE-SPECIFIC OUTPUT PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: purge_1ms_output_routine

; This routine determines the commanded state of the CANP/CANVNT output

; (CANVT_CMD) when 0 < duty-cycle < 100%.  A PWM signal is generated by

; software using a calculated duty-cycle, a calibrated period, and the

; system clock.

;

; The commanded state of the output is updated each millisecond.

;    |<------------ PRG_PERIOD ----------->|

;  ->|             |<- PRG_ON_TIME

;     _____________                         ____________

;    |             |                       |            |

;____|             |_______________________|            |____________________

;    A                       A

;    |<----- elapsed_time -->|

;    |                       |

; START_CVS_PER            clock

; This process is executed once per millisecond:

PRG_OUT_ENA = 0 ------------------------| EXIT this process.

clock - START_CVS_PER < PRG_ON_TIME ----| CANVT_CMD := 1

|

| --- ELSE ---

|

clock - START_CVS_PER < PRG_PERIOD -----| CANVT_CMD := 0

|

| --- ELSE ---

|

| START_CVS_PER := clock

END: purge_1ms_output_routine

4-48

HARDWARE INTERFACE, OUTPUT STATE CONTROL TABLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

4.4  OUTPUT STATE CONTROL TABLE (CDAL0)

STRATEGY MODULE: HWINT_OSC_CD

OVERVIEW

There exists a table of values related to the Output State  Control  function

in  the  PCM.  This table is used to hold the actual values that the offboard

diagnostic unit wishes to use in palce of those calculated by  the  strategy.

This table is also used to hold the enable mask transmitted from the offboard

diagnostic unit to the PCM to unlock the  individual  channels.   The  enable

mask  is  to  be  compared  to a predetermined, hardcoded enable bit pattern.

This acts as a key to unlock each individual channel in the table.  There are

currently  20 channels substitution is allowed to be performed on.  These are

shown in a diagram below.  If one or more of these parameters are not present

in  a  given  strategy,  a  request  from  the  ODU to override would have no

external effect.  This is because the substitute()  process  would  never  be

invoked with the channel number that does not exist in the strategy.

IMPLEMENTATION NOTE:  The OSC_VALUE[] parameters will have binary scaling  to

match  the  parameter  they will be used to override.  This implies that each

OSC_VALUE[] will have its own unique  scaling.   The  OSC_VAULE[]  parameters

have   been   created   in  the  EDTS  parameter  dictionary  as  OSC_VALUE0,

OSC_VALUE1...  with the appropriate scaling.  The binary scaling  information

is  also  to  be  provided  to the ODU designers so that they will load a bit

pattern into OSC_VALUE[] that is scaled properly and no further  action  will

be  required  by the EEEC once the value has been transmitted and loaded into

the table.

4-49

HARDWARE INTERFACE, OUTPUT STATE CONTROL TABLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OSC TABLE

--- -----

+----------+--------------+----------------+----------------+-----------+

| channel* |  parameter   |  OSC_VALUE[]   |   OSC_ENA[]    | SCALING** |

+----------+--------------+----------------+----------------+-----------+

|  0       |  ISCDTY      |       .        |      .         | b15,u,w   |

|  1       |  EAM         |       .        |      .         | flag      |

|  4       |  LAMBSE1     |       .        |      .         | b15,u,w   |

|  5       |  LAMBSE2     |       .        |      .         | b15,u,w   |

|  7       |  EGRDC       |   *---------------------------* | b15,u,w   |

|  8       |  GR_CM       |   |These values are initially | | b1,u,b    |

|  9       |  PG_DC       |   |equal to zero.  The ODU    | | b15,u,w   |

|          |              |   |will download parameters   | |           |

|          |              |   |through custom SCP commands| |           |

|          |              |   |to this area.              | |           |

|          |              |   *---------------------------* |           |

|  10      |  PGM_CVS_DC  |       .        |      .         | b15,u,w   |

|  11      |  FLG_SS_1    |       .        |      .         | flag      |

|  12      |  FLG_SS_2    |       .        |      .         | flag      |

|  15      |  BCSDC_OUT   |       .        |      .         | b15,u,w   |

|  16      |  TV_PRES_BAR |       .        |      .         | b1,u,b    |

|  17      |  SAF         |       .        |      .         | b2,s,w    |

|  19      |  FPUMP_DC    |       .        |      .         | b10,u,w   |

|  20      |  FPUMP_SPEED |       .        |      .         | flag      |

|  21      |  IMRC_CMND   |       .        |      .         | flag      |

|  25      |  HEGOHTR11   |       .        |      .         | flag      |

|  26      |  HEGOHTR12   |       .        |      .         | flag      |

|  27      |  HEGOHTR21   |       .        |      .         | flag      |

|  28      |  HEGOHTR22   |       .        |      .         | flag      |

+----------+--------------+----------------+----------------+-----------+

'*'...Channel numbers shown are offset 128 from SCP hex values.

'**'...Where scaling references a flag, the  least  significant  bit  of  the

substitute  word  shall be used to control the channel value (ie.  the LSb is

the flag state).  When scaling references a byte, the least significant  byte

commands the value.

4-50

CHAPTER 5

SCP INTERFACES

5-1

SCP INTERFACES

5.1  FEATURE:  SCP - V6.2A_SCP_EATC_PATS (CDAN0)

5.1.1  SCP MESSAGE OVERVIEW (CDAA0)

STRATEGY MODULE: SCP_OVERVIEW_COM1

SCP Messages - Reception & Transmission

Message Format:

Messages are composed of a  variable  number  of  bytes.   Currently  defined

messages  are  from 53 to 101 bits in length, depending upon the message type

and content.  Each message is composed of fields which possess  a  predefined

interpretation.   Fields  are  organized along byte boundaries.  All messages

must begin with a Start Of Message (SOM) followed by a field which determines

bus  priority as well as defining the message type.  All messages end with an

End Of Message (EOM) field.

The SCP protocol currently has defined  6  specific  network  message  types.

They  are:   Functional,  Broadcast,  Function  Query, Function Read, Node to

Node, and BCC Access.  Further definition of these messages can be  found  in

the EEC Bus Controller (EBC) Functional Specification.

The general message format is:

Message Field        Length     Field Description & Data Handling

----------------------------------------------------------------------------

SOM                  2 bits     Start Of Message indicator

Priority/Type        1 Byte     Priority (bits 7:4) & Type (bits 3:0)

(NOT STORED IN MBFIFO)

Target Specifier     1 Byte     Outgoing Message Target Specifier Byte

Depending upon the message type, this

byte could be a Physical Address or a

Generic Function code.

(NOT STORED IN MBFIFO)

Source Address       1 Byte     Outgoing Message Source Address

(STORED IN MBFIFO)

Data               0-7 Bytes    Depending on application, this field may

contain further definition of a command

function, or it may carry a variable to

be passed on to the receiving node.

(STORED IN MBFIFO)

CRC                  1 Byte     Message Cyclic Redundancy Check.

(NOT STORED IN MBFIFO)

EOD                  1 bit      End Of Data Field; idle bus indicates

conclusion of sender transmission.

ACK                  1 Byte     Response by receiving node(s) to indicate

the message was received properly, OR,

single byte of function read data (or BCC

data) to be sent to the requesting node.

(FUNCTION READ DATA STORED IN MBFIFO)

5-2

SCP INTERFACES, SCP MESSAGE OVERVIEW - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EOM                  2 bits     End Of Message field; idle bus indicates

completion of message.

5-3

SCP INTERFACES, SCP MESSAGE OVERVIEW - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Typical SCP Communications:

The strategy includes a construct, used to  identify  the  message  name  and

variable  data  to  be sent with the message, which is called "send()".  This

construct is described in detail in the SYMBOLOGY section.

Most messages are transmitted only when a set of  conditions  are  met.   The

message  definition  in  these cases will define a "Message Transmit Routine"

which controls the "send" command.   Messages  which  are  transmitted  every

background  loop  still  require the "send()" command for message initiation,

with  an  "unconditionally"  statement  included  in  the  "Message  Transmit

Routine".

When a message is received, the EEC will execute a "Receive Message Routine",

which  is  only  executed  upon  reception  of  a  particular message.  These

routines can include merely loading a register with a new value, or  decoding

the  data  bytes  of the incoming message in order to affect an action in the

EEC strategy.

All SCP messages are defined in the Module:  SCP_MSG_LST_COM*, which provides

an  overview  of the messages supported by a particular strategy as well as a

detailed definition of the message specification.  Other data defined in  the

message  spec  include  Message  Type, Purpose, Data Flow, Scaling and Format

information, and Strategy which controls the EEC handling of that message.

Message Reception:

The EEC Bus Controller (EBC) chip was designed to require minimal host inter-

vention  for  transmission  and  reception  of  messages  on the SCP Bus.  In

message reception, the EBC monitors bus transactions  on  the  network  on  a

individual  message  basis  without  intervention  from  the  host.   Network

communications intended for the EEC must be qualified by the  EBC  before  it

can  acknowledge  the  message.   Qualification  of a message is performed by

examining the Message Type and Target  Specifier,  which  are  found  in  the

3-Byte header of all messages.

For  non-physically  addressed  messages  (such  as  Functional,   Broadcast,

Function  Read,  and  Function Query messages), qualification of a message is

based on comparing the Target Specifier  Byte  of  the  message  to  internal

look-up  tables  in the EBC.  The appropriate look-up table, based on message

type, is searched for a match  against  the  Target  Specifier  Byte  of  the

incoming  message.   A  match on any of the bytes in the tables qualifies the

message.

Physically addressed  messages  do  not  require  qualification  through  the

look-up  table  process.  Therefore, Node to Node and BCC Access messages are

qualified by comparing the Target Specifier  Byte  of  the  incoming  message

against  the  physical  Node Address Register (NAR), which is downloaded from

ROM to the EBC upon initialization.

After message qualification, the first byte stored  in  the  EBC's  256  Byte

buffer  (MBFIFO)  is  the message number.  This is simply the location of the

look-up table entry found in qualification of a message relative to the start

of  the look-up table except for Node to Node messages, for which the message

number is always 00H.  Because Look-up  Table  2  is  only  used  to  qualify

Function  Read  messages,  which are not transferred to the host, the message

number, for this case has no meaning.

5-4

SCP INTERFACES, SCP MESSAGE OVERVIEW - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Once the message number has been  stored  in  the  MBFIFO's  first  available

location,  the source identifier and following data, up to EOD (end of data),

are then stored in sequential locations after the message  number.   Received

messages are emptied from the MBFIFO once per background loop.  There are two

options for handling message processing.  The  first  option,  called  Direct

Data Storage, writes the received data directly to locations defined for that

message number.  The second option, called  Routine  Dispatch,  will  call  a

routine which performs some action based on the message contents.

Information on which type of message handling is required is stored  in  ROM,

in  a  structure  called  a Receive Message Descriptor Block (RMDB).  An RMDB

describes the length of the expected  message  as  well  as  its  destination

address.

Message Transmission:

To unburden the host software of message transmission delays, a simple  queue

is  used to quickly store the messages needing transmission.  As each message

in the queue is sent, the next message is retrieved and loaded into  the  EBC

via  an  interrupt  indicating  that  the  previous  message  was  sent.  The

background routine continues without having to interface  directly  with  the

EBC.   Messages are transmitted from the queue on a FIFO basis.  The EBC will

recognize and transmit any new message which was added to  the  queue  during

the transmission sequence.

Each queue entry holds a status byte (empty, pending, complete & status)  and

a  pointer  to  the structure used to define transmitted messages, called the

Transmit Message Descriptor Block (TMDB).  The TMDB list resides in  ROM  and

contains the body of the message to be sent, along with information on how to

handle the results of the transmission (acknowledgers, errors, etc.).

When a message is to be transmitted on  the  bus,  the  host  must  load  the

outgoing  message  body (using the TMDB) into an EBC transmit buffer and then

tell the EBC to transmit the message.  When this request  is  made,  the  EBC

will  arbitrate for control of the bus in order to send the message.  The EBC

will notify the host  that  the  message  has  been  transmitted  through  an

external  interrupt,  which  is used to empty the transmit queue or reset the

strategy to wait for any future transmit message command in background.

5-5

SCP INTERFACES, SCP BACKGROUND - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

5.1.2  SCP BACKGROUND (CDAN0)

OVERVIEW

Each of the following logic blocks contain an execution timing  specification

which  is  required  for proper SCP communications.  The main background loop

logic contains monitors for various state machines which control the EEC  SCP

communication    characteristics.     This    logic    handles   the   normal

background-based communications as well as referencing  those  communications

which  reference foreground data to be transmitted at a faster rate.  Another

type of communication are those  messages  which  are  transmitted  upon  EEC

initialization,  called the Initial Message Set.  Function Read registers, if

defined, are updated in this specification as well.

The initial message set is a set of messages which  are  transmitted  shortly

after  key-on,  or  after  a crank-run transition of the ignition key.  These

messages describe the module's current state to the  network.   The  messages

are  queued  at  6 +/-2 msec intervals.  This process is enabled by the flag:

SCP_ENA_IMS.

THE INITIAL MESSAGE SET IS TRANSMITTED ONLY ONCE DURING NETWORK

INITIALIZATION.  THERE IS NO RE-QUEUEING STRATEGY FOR ANY TYPE

ERROR DURING IMS EXECUTION.

All SCP messages and  function  reads  are  identified  in  separate  message

modules.   The  strategy  module  controling  the message transmits is called

SCP_MSG_CONTROL_pp_COMx (where pp is a specific strategy path reference).

DEFINITIONS

Registers:

-  DLOAD_EXEC_KEY = Command Key byte  to  enable  execution  of  a  download

program.

-  DLOAD_PROG_STRT = Address at which  to  begin  execution  of  a  download

program.

-  ODU_UP_TMR = Timer used to  indicate  the  Offboard  Diagnostic  Unit  is

on-line.  (incrementing timer, msec).

-  PUTMR = Time since last power-up, sec.

-  SCP_DIAG_ST = State register for the On-demand test scheduler.

-  SCP_DWNLD_ST = Current level of Download and Execute State Machine.

-  SCP_TMR = SCP network initialization timer, msec.

5-6

SCP INTERFACES, SCP BACKGROUND - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Bit Flags:

-  A_RATE_ENA = Flag set when SCP Packets are requested at Type 'A' rate.

-  CRKFLG = Crank flag; 1 -> in crank mode.

-  DEMAND_MODE = Flag set when an On-Demand selt test mode is active.

-  NO_FUEL_SCP = SCP logic indicates that injectors should be disabled

(Latched bit); 1 -> DO NOT deliver any fuel to engine.

-  ODU_UP_STATE = Flag set to indicate status of ODU tester; 1 -> active.

-  RESULTS = Flag set when an On-Demand test has executed successfully.

-  SCP_ENA_IMS = Flag to enable transmit of the PCM Initial Message Set upon

-  SCP_UP = Flag to indicate the EBC chip has been successfully initialized.

-  UNDSP = Underspeed Flag; 1 -> in underspeed mode.

-  KAM_ERROR = KAM error flag; 1 -> KAM data invalid.

Calibration Constants:

-  ODU_TIMEOUT = Time-out value for certain offboard diagnostic routines.

-  SCP_DT1 = Time delay to begin transmitting SCP  network  initial  message

set.

-  SCP_DT2 = Time delay to stop transmitting  SCP  network  initial  message

set,  begin transmitting SCP status requests, and to begin sending normal

broadcast messages.

-  SCP_IT1 = Time to which SCP_TMR is set  upon  initialization,  to  ensure

that the PCM is synchronized with the rest of the network modules.

5-7

SCP INTERFACES, SCP BACKGROUND - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SCP_BKGND_COM1

;parameter_definitions

The following assignments define key parameter logicals which are

referenced throughout this chapter. They are not necessarily intended

to be executable.

unconditionally ------------------------| dwnld_not_ready = 0

(Definitions used for SCP_DWNLD_ST     | dwnld_entrd = 1

documentation purposes only)         | dwnld_data = 2

| execute_ready = 3

| dwnld_&_execute = 4

|

(Definitions used for SCP_DIAG_ST       | initial_state = 0

documentation purposes only)         | no_results_avail = 1

| test_running = 2

| results_avail = 3

;kam_init_scp

Execute Every Background Loop.

Clear EBC RAM area when a reset of KAM is detected.

KAM_ERROR = 1 --------------------------| Clear EBC User RAM

| address location 3F

;ram_init_scp

Execute only once upon RAM initialization.

unconditionally ------------------------| SCP_TMR := SCP_IT1

| ; synchronize time with network

|

| ODU_UP_TMR := 31.875

| ; ensure that heartbeat is sent

5-8

SCP INTERFACES, SCP BACKGROUND - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;scp_background_logic

Execute Every Background Loop.

SCP_UP = 0 -----------------------------| SCP_DWNLD_ST = 0

(EBC not up)                           | SCP_DIAG_ST = 0

|  (reset state machines)

| A_RATE_ENA = 0

|  (disable repetitive transmissions)

|

| SCP_ENA_IMS = 0

|

| Do: INITIALIZE_EBC

|  (detailed in SCP_INIT module)

|

| --- ELSE ---

|

SCP_TMR < SCP_DT1 ----------------------| SCP_ENA_IMS = 0

(receive-only mode)                  |

| Do: UPDATE_FUNCTION_READS

|  (detailed in MSG_CONTROL module)

|

| --- ELSE ---

|

SCP_TMR < SCP_DT2 ----------------------| SCP_ENA_IMS = 1

(send initial messages)              |  (enable Initial Message Set)

|

| Do: UPDATE_FUNCTION_READS

|  (detailed in MSG_CONTROL module)

|

| --- ELSE ---

|

| ;Begin Normal Network Activity

|

| SCP_ENA_IMS = 0

|

| Do: SEND_STATUS_REQUESTS

| Do: UPDATE_FUNCTION_READS

| Do: TRANSMIT_VEHICLE_MESSAGES

|  (detailed in MSG_CONTROL module)

5-9

SCP INTERFACES, SCP BACKGROUND - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;Empty_MBFIFO

Execute Every Background Loop

SCP Receive Message Routine

SCP_UP = 1 -----------------------------| Read FSTAT/MBFIFO to check for

| any messages_waiting.

if message_waiting ---------------------| Empty buffer and perform

| appropriate individual

| "Receive Message" operations.

|  (See individual message logic)

;reset_state_machines

Execute Every Background Loop

The following logic determines if the EEC will allow

access to its Download State or for Rapid Data Transfer.

ODU_UP_TMR <= ODU_TIMEOUT --------------| ODU_UP_STATE = 1

(ODU connection is active)            |

| --- ELSE ---

|

| ODU_UP_STATE = 0

| SCP_DWNLD_ST = 0

| A_RATE_ENA = 0

| (disable repetitive transmissions)

;state_machine_servicing

Execute Every Background Loop

PUTMR <= 4 -----------------------------| SCP_DIAG_ST = initial_state

(Upon EEC power-up...)                | SCP_DWNLD_ST = dwnld_not_ready

|  (Initialize state registers to 0)

|

| --- ELSE ---

|

DEMAND_MODE = 1 ------------------------| SCP_DIAG_ST = test_running

(When OBDII Exec sets this flag...)   |  (An On-Demand test is running; 2)

|

| --- ELSE ---

|

RESULTS = 1 ----------------------------| SCP_DIAG_ST = results_avail

(When OBDII Exec sets this flag...)   |  (A test executed successfully; 3)

|

| --- ELSE ---

|

| SCP_DIAG_ST = no_results_avail

|  (A test aborted, or wasn't run; 1)

5-10

SCP INTERFACES, SCP BACKGROUND - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;jump_to_dwnld_routine_logic

Execute Every Background Loop

The following two logic blocks determine if a Downloaded Program

is to be executed and, if so, which mode of execution is selected.

THESE TWO EXECUTION MODES ARE MUTUALLY EXCLUSIVE.

SCP_DWNLD_ST >= execute_ready (3)-|

|

ODU_UP_STATE = 1 -----------------|AND -| Execute routine at RAM address:

|     | DLOAD_PROG_STRT with the Normal

DLOAD_EXEC_KEY = 8002h -----------|     | Background List enabled.

This logic will truncate the EEC-V background sequence to merely execute  SCP

and  whatever  Foreground  necessary to keep out of HLOS mode.  The parameter

NO_FUEL_SCP is serviced  only  within  the  SCP  logic  and  thus  cannot  be

overwritten  by any other portion of the strategy.  This will insure that the

injectors are disabled in the following Download and Execute mode.

SCP_DWNLD_ST >= execute_ready (3)-|

|

ODU_UP_STATE = 1 -----------------|

|AND -| NO_FUEL_SCP = 1

DLOAD_EXEC_KEY = 8102h -----------|     |   (disable all injectors)

|     | Execute routine at RAM

CRKFLG = 1 -----------------|     |     | address:  DLOAD_PROG_STRT

|OR --|     | with the SBDS Background

UNDSP = 1 ------------------|           | List enabled.

|

| --- ELSE ---

|

| NO_FUEL_SCP = 0

|   (normal injector service)

5-11

SCP INTERFACES, SCP BACKGROUND - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;ebc_lockup_check_logic

Execute Every Background Loop

Check for an EBC Lock-up condition.

(More than 25 msec have passed since

last message transmit without the

occurrence of any of the following

IR2 interrupts:  Bits 7, 6, or 5) -----| Do: INITIALIZE_EBC

|  (reset EBC)

|

| --- ELSE ---

|

| Exit (no action)

;rapid_packet_logic

Execute on a Fixed Midground Interrupt

Send Type A packets as specified.

A_RATE_ENA = 1 -------------------------| send(REPORT_RAPID_PACKET)

(Packets enabled at Type 'A' rate)    |  (Send type 'A' packets which are

|   queued for transmission - cycle

|   through ALL packets, transmitting

|   no more than two packets at

|   intervals less then or equal to

|   12 msec).

5-12

SCP INTERFACES, EXTERNAL INTERRUPT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

5.1.3  SCP EXTERNAL INTERRUPT SERVICE ROUTINE (CDAK0)

OVERVIEW

This module services the external interrupt which is  generated  by  the  EBC

chip  upon  completion  of an SCP message transmission or upon detection of a

fault condition within the EBC.  The registers  and  commands  referenced  in

lower  case  in  this logic are EBC-specific and not accessible via the RCON.

This interrupt carries the same execution priority as the PIP interrupt.

If an EBC error has occurred, the status of the EBC faults would  be  carried

in  interrupt  registers IR2(b3 & b4) and IR3(b1 & b7).  These registers must

be read/cleared by the Host Software.  If an EBC error condition does  exist,

the   Host  Software  will  reinitialize  the  EBC  and  continue  on  (where

applicable) to transmit the next pending message.  Note that IR3 is  examined

first.   If  an  error  is  found  through the access to IR3, then IR2 is not

tested.

If an EBC error is not indicated, the interrupt is due to a resolution of the

last message transmit attempt.  A transmitted message should always result in

an EBC interrupt being raised, with one of the three MS  bits  of  IR2  being

set.   The  Host  software  first  determines  if the message was transmitted

successfully ("Tx_OK" flag).  If so, this routine checks the Transmit Message

Descriptor  Block (TMDB) for that message, which resides in software only, to

determine how  to  handle  the  SCP  acknowledge  byte.   If  the  TMDB  flag

"sav_ret_dat"  is  set  (as in Function Read messages), the host will execute

the appropriate routine described in the TMDB (which must reset the ECR  byte

to 72h).  If this flag is not set, then the PCM will carry on to transmit any

awaiting messages in the internal software transmit queue.

If the interrupt is not due to any internal EBC error or due  to  a  "Tx_OK",

then  the  interrupt was probably due to either a "loss of arbitration" error

or a "transmit error" on the network.  If the failure is due to  a  "transmit

error",  then  the  PCM  will  reset  the ECR register to 72h (since the last

transmit attempt may have driven the network single-ended), and carry  on  to

transmit  any  awaiting messages in the internal software transmit queue.  If

the failure is due to a "loss of arbitration" error, the PCM treats it as  if

it  were  a  valid  transmit  attempt and carries on to transmit any awaiting

messages in the host software transmit queue.  NOTE:  There is  no  'software

retry'  message  failure  management  logic for either loss of arbitration or

lack of acknowledgment errors.  However, the Function Read Retry strategy may

be  invoked  in  the  message  transmit  logic  in order to drive the network

single-ended if the Function Read Acknowledgement is not received.

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

Since an interrupt can be set high while this routine is  executing  (without

being  recognized  by the host software), the appropriate interrupt registers

(IR2 and IR3) are read and cleared upon exiting this routine.

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

5-13

SCP INTERFACES, EXTERNAL INTERRUPT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

!!!!  NOTE  !!!!

IN ORDER TO PREVENT THE EBC FROM LOCKING UP DUE TO NOISE ON THE SCP LINK, A

CHECK IS MADE IN THE BACKGROUND (SCP_BKGND_COMx MODULE) TO ENSURE THAT

RESOLUTION IS OBTAINED FOR ANY TRANSMIT MESSAGE (IE.  Tx_OK, Tx_Error, LOA).

IF MORE THAN 25 MILLISECONDS PASSES IN BETWEEN MESSAGE TRANSMIT AND RESO-

LUTION OF THE MESSAGE, THEN THE EBC IS RE-INITIALIZED AND THE TRANSMIT

ROUTINES CONTINUE WHERE THEY LEFT OFF (AFTER THE LAST TRANSMIT COMMAND).

DEFINITIONS

INPUTS

Bit Flags:

-  illegal_op = Bit 1 of IR3 in EBC chip indicating an illegal read or write

of an EBC register (reset required):  NOT DISPLAYABLE.

-  intrnl_err = Bit 3 of IR2 in EBC chip indicating an  internal  EBC  error

was detected (reset required):  NOT DISPLAYABLE.

-  reset_occurd = Bit 7 of IR3 in EBC chip indicating a EBC occurred  (reset

required):  NOT DISPLAYABLE.

-  ntwk_dvr_wd = Bit 4 of IR2  in  EBC  chip  indicating  a  Network  Driver

Watchdog   Failure  was  detected  by  the  EBC  (reset  required):   NOT

DISPLAYABLE.

-  sav_ret_dat = Bit 7 of the SCP message ACK Handler  Byte  (in  the  TMDB)

which  tells  the  external  interrupt handling routine to either save or

discard  the  data  returned  over  the  SCP  link  at  the  end  of   an

EEC-initiated message transmission:  NOT DISPLAYABLE.

-  tx_ok = Bit 7 of IR2 in EBC chip indicating that  the  previous  transmit

attempt was successfully completed:  NOT DISPLAYABLE.

OUTPUTS

Registers:

-  ECR = EBC Control Register (NOT DISPLAYABLE).

5-14

SCP INTERFACES, EXTERNAL INTERRUPT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SCP_EXTINT_COM1

Execute the following routine ONLY when the EBC external

interrupt (INT pin) has been pulsed high.

Tests of the bits in IR2 and IR3 implies that a

read/clear command was executed on those registers.

These registers are also read/cleared

upon exit of this logic sequence.

reset_occurd = 1 -----------------|

(Bit 7 of IR3 in EBC. NOT due    |

to any Host S/W command)        |

|OR --| Do: INITIALIZE_EBC

illegal_op = 1 -------------------|     |  (Reset EBC chip)

(Bit 1 of IR3 in EBC chip)       |     |

|     | Read and Clear registers IR2 & IR3

ntwk_drvr_wd = 1 -----------------|     |

(Bit 4 of IR2 in EBC chip)       |     |

|     |

intrnl_err = 1 -------------------|     |

(Bit 3 of IR2 in EBC chip)             |

| --- ELSE ---

tx_ok = 1 ------------------------|     |

(Bit 7 of IR2 in EBC chip)       |AND -| Execute Returned Data Service

|     | Logic (in message specification)

sav_ret_dat = 1 ------------------|     |

(Bit 7 of Ack Handler Byte in TMDB)    | Read and Clear registers IR2 & IR3

|

| Transmit next pending message

| (if any are queued up)

|

| --- ELSE ---

|

tx_ok = 1 ------------------------------| Read and Clear registers IR2 & IR3

(Bit 7 of IR2 in EBC chip)             |

| Transmit next pending message

| (if any are queued up)

|

| --- ELSE ---

|

| ECR := 72h

|  ; Reset EBC to Differential SCP

|  ; PRIOR to sending another msg.

|

| Read and Clear registers IR2 & IR3

|

| Transmit next pending message

| (if any are queued up)

5-15

SCP INTERFACES, LOAD TRANSMIT BUFFER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

5.1.4  SCP MESSAGE TRANSMIT SERVICE ROUTINE (CDAK0)

OVERVIEW

This module is executed when it has been determined that a message should  be

transmitted and that the SCP bus is idle.

The process executed when a message is sent is paraphrased as follows:

1) The Transmit Message Descriptor Block (TMDB) offset is obtained  from  the

host  software  message  transmit  queue, which will locate the TMDB in EEC-V

ROM.  The TMDB describes message content and characteristics.

2) Begin TMDB processing to assemble and load the appropriate  message  bytes

into  the  transmit buffer on the EBC chip.  During this process, the message

type is obtained from the TMDB PRI/TYP byte.  If the message  is  a  function

read,  a  process  (UPDATE_FRECR)  is  called to service the the ECR register

prior to actually transmitting the message.  This process  will  ensure  that

the  software  retry  strategy for function reads (which drives the SCP first

differential, then Single-ended on the A Bus, then Single-ended on the B Bus)

is  executed for the correct message.  If the message is not a function read,

then the UPDATE_FRECR process is not called.

3) Finally, when the message is completely loaded into the EBC Chip  transmit

buffer,  the  message transmit command is issued via the ACB/DBR registers in

the EBC chip.

DEFINITIONS

INPUTS

Bit Flags:

-  FRECR =  Commanded  ECR  register  state  for  a  Function  Read  message

transmit.

OUTPUTS

Bit Flags:

-  ECR = EBC Control Register (NOT DISPLAYABLE).

-  FRECR = See above.

5-16

SCP INTERFACES, LOAD TRANSMIT BUFFER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SCP_LOAD_TX_BUFFER_COM1

Execute the following routine when it has been determined that a

message should be sent and that the SCP Bus is in an idle state.

unconditionally ------------------------| Get TMDB data to control loading

| of message bytes into the

| EBC Transmit buffer.

TMDB msg Type nibble = 3h --------------| ; SCP bus driver configuration.

(Function Read message)                 | Do: UPDATE_FRECR

(Obtained from PRI/TYP byte in TMDB)    | ECR := FRECR

unconditionally ------------------------| Complete loading of message bytes

| from TMDB into the EBC Transmit

| buffer.

BEGIN:  UPDATE_FRECR

PRIVATE process called only when a Function Read is to be transmitted.

FRECR = 72h ----------------------------| FRECR := 52h

; Last try was Differential             | ; Next try is on Bus 'A' only

|

| --- ELSE ---

|

|

FRECR = 52h ----------------------------| FRECR := 32h

; Last try was Single-ended 'A' Bus     | ; Next try is on Bus 'B' only

|

| --- ELSE ---

|

| FRECR := 72h

| ; Default to Differential

END:  UPDATE_FRECR

5-17

SCP INTERFACES, SCP MESSAGE CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

5.1.5  SCP MESSAGE CONTROL (CDAN0)

OVERVIEW

SCP Message List Summary

The table below outlines the messages which are serviced by the  PCM  on  the

SCP  Network.   Additional  information  is supplied to describe the Function

Read Data Registers, which are refreshed each background  loop  by  the  host

software.   Function  Read  messages can be accessed by any other node on the

bus and require no direct host intervention.

The details of each message are specified on the following pages,  which  are

referenced  by  the  message  name provided in the table below.  Each message

specification includes a routine reference to identify  the  conditions  upon

which  to  send  the  EEC-initiated  transmit  messages,  and to identify the

modules affected by any received messages.

The "Targ Spec" column indicates the value of the generic  function  code  or

node  address, depending on whether the message is functionally or physically

addressed.  The "Spec Func" column indicates either a secondary function code

or  data  which  is  required  to further define the message.  The "Num Data"

column indicates how many data bytes are included in the message;  the  count

includes the byte from the Spec Func column, if used.

5-18

SCP INTERFACES, SCP MESSAGE CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Message    Targ  Spec  Num

Type     Spec  Func  Data   Message Name                    Description

---------------------------------------------------------------------------

MESSAGES - Module Receiving:

Broadcast   6Ah   xxh  yy     PROCESS_OBDII_REQUEST   (Req J1979 data/action)

Broadcast   54h   var   2     RX_PT_STAT_RQST    (Stat. Req. From SCP to EEC)

Broadcast   93h   C1h   1     REQUEST_SECURITY_STATUS   (PATS status request)

Node-Node   10h   2Ch  yy     DEFINE_RAPID_PACKET (Rapid data transfer block)

Node-Node   10h   31h  yy     DIAGNOSTIC_ROUTINE_ENTRY    (Request Self Test)

Node-Node   10h   32h   3     DIAGNOSTIC_ROUTINE_EXIT  (Exit/Abort Self Test)

Node-Node   10h   34h   7     DOWNLOAD_ROUTINE_ENTRY (Enter Program Rcv Mode)

Node-Node   10h   37h   2     DOWNLOAD_ROUTINE_EXIT   (Stop Downloading Data)

Node-Node   10h   36h  yy     DOWNLOAD_BLOCK_DATA (Block Data Transfer Mes'g)

Node-Node   10h   B1h  yy     EXECUTE_ROUTINE (Specific Execute Routine Mode)

Node-Node   10h  other yy     MODE_NOT_SUPPORTED   (Any other node-node mode)

Node-Node   10h   13h   1     REQUEST_ALL_CODES     (All Set & Pending DTC's)

Node-Node   10h   25h   1     REQUEST_ALL_DATA_STOP    (Stop Repetitive Data)

Node-Node   10h   14h  yy     REQUEST_CLEAR_CODES           (Clear All DTC's)

Node-Node   10h   33h   2     REQUEST_DIAG_RESULTS     (On-Demand test codes)

Node-Node   10h   23h  yy     REQUEST_DMR_ACCESS (One Request/One Respns DMR)

Node-Node   10h   03h   1     REQUEST_MIL_CODES    (Legislated Codes request)

Node-Node   10h   22h  yy     REQUEST_PID_ACCESS (One Request/One Respns PID)

Node-Node   10h   2Ah  yy     REQUEST_RAPID_PACKET (Packet data at optl rate)

Node-Node   10h   3Fh   1     TESTER_PRESENT          (ODU heartbeat message)

----------------------------------------------------------------------------

where:  xxh = J1979 request modes (01h, 02h, 03h, 04h, 06h, or 07h).

yy = Message length determined by request type (0 to 7 bytes).

var = Variable data - determined by message transmit routine.

zz = Message length determined by type of information reported.

Fxh = Node Address register of Offboard Diagnostic Unit (ODU).

other = Any other Node to Node message which is NOT listed above.

5-19

SCP INTERFACES, SCP MESSAGE CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Message    Targ  Spec  Num

Type     Spec  Func  Data   Message Name                    Description

---------------------------------------------------------------------------

MESSAGES - Module Transmitting:

Func Read   97h   N/A   0     ANTI_THEFT_STATUS  (Anti-theft command message)

Broadcast   49h   10h   2     ECT_DEG_C      (Engine Coolant Temp. in Deg. C)

Broadcast   93h   var   2     ENGINE_PAT_STATUS  (Anti-theft results for PCM)

Broadcast   6Bh   43h   7     REPORT_OBDII_CODES   (Send CARB mil_codes Only)

Broadcast   6Bh   47h   7     REPORT_OBDII_CONT_RESULTS  (Send Pending DTC's)

Broadcast   6Bh   42h  zz     REPORT_OBDII_FF_PID        (Send CARB PID Data)

Broadcast   6Bh   41h  zz     REPORT_OBDII_PID           (Send CARB PID Data)

Broadcast   6Bh   44h   1     REPORT_OBDII_RESET   (Send CARB Reset Complete)

Broadcast   6Bh   46h   7     REPORT_OBDII_TRIP_RESULTS  (Send Mode 06 Data)

Node-Node   Fxh   54h   1     CONFIRM_CODES_CLEAR     (Report of OBDII Reset)

Node-Node   Fxh   73h   7     DIAGNOSTIC_RESULTS       (Send On-Demand Codes)

Node-Node   Fxh   7Fh   6     GENERAL_RESPONSE     (Generic Accept or Reject)

Node-Node   Fxh   53h   7     REPORT_ALL_CODES (Send all Set & Pending DTC's)

Node-Node   Fxh   63h  zz     REPORT_DMR_DATA (Send single response DMR data)

Node-Node   Fxh   43h   7     REPORT_MIL_CODES     (Send CARB mil_codes Only)

Node-Node   Fxh   62h  zz     REPORT_PID_DATA (Send single response PID data)

Node-Node   Fxh   6Ah  zz     REPORT_RAPID_PACKET (Send Packet data requestd)

----------------------------------------------------------------------------

where:  xxh = J1979 request modes (01h, 02h, 03h, 04h, 06h, or 07h).

yy = Message length determined by request type (0 to 7 bytes).

var = Variable data - determined by message transmit routine.

zz = Message length determined by type of information reported.

Fxh = Node Address register of Offboard Diagnostic Unit (ODU).

other = Any other Node to Node message which is NOT listed above.

5-20

SCP INTERFACES, SCP MESSAGE CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

INITIAL MESSAGE SET:

The transmission of IMS messages is controlled by the flag

SCP_ENA_IMS, which is set in the module SCP_BKGND_COMx.

Other logic blocks are also allowed to send these messages:

ECT_DEG_C

FUNCTION READ MESSAGE TABLE:

Genr

Func              Message Name                    Description

------------------------------------------------------------------

29h             vsbar_kph           (EEC Calculated VS in Kph)

------------------------------------------------------------------

DEFINITIONS

Registers:

-  SCP_ECT_TMR = Free running timer used to record interval elapsed between

two consecutive ECT_DEG_C messages.

-  FR_97_COUNT = Register indicating type of bus utilization for FR of SCP

ANTI_THEFT_STATUS message.

Bit Flags:

-  KAM_ERROR = Indicates Keep Alive RAM invalid.

-  OBDII_RESET = Flag which represents the receipt of an OBD-II scan tool

reset message.

-  PAT_FAULT = Flag indicating the SCP link between the PATS and PCM is bad.

-  VSFMFLG = Vehicle speed sensor (FMEM) flag, 1 = sensor failed.

-  SCP_ENA_IMS = SCP Initial Message Set enable flag.

-  SEND_ECT = Broadcast periodic ECT (deg. C) message on SCP; 1 -> active.

Calibration Constants:

-  ECT_TIME = Time threshold to broadcast ECT_DEG_C message.

-  SEND_ECT_SW = Broadcast ECT message at fixed periodic rate; 1 -> enabled.

This constant should be calibrated to 1 only for vehicles with the EATC

(Electr. Automatic Temp. Control) module.

5-21

SCP INTERFACES, SCP MESSAGE CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SCP_MSG_CONTROL_V6_COM1

; network_ims

Execute the following logic once during network initialization.

Broadcast the Initial Message Set messages only ONCE.

(No re-queueing or SCP fault strategies are enabled).

SCP_ENA_IMS = 1 ------------------------| send(ECT_DEG_C)

(Initial Message Set enabled)

;reset_scp_data

Execute the following logic once per background loop.

Logic to reset diagnostic parameters due to Offboard Tool command.

OBDII_RESET = 1 ------------------|

;full diagnostic reset requested |OR --| PAT_FAULT := 0

|     | FR_97_COUNT := 0

KAM_ERROR = 1 --------------------|

;KAM reset was detected

5-22

SCP INTERFACES, SCP MESSAGE CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: CONFIGURE_EBC

Functional Read Data Register's:

(Initialize function read registers to FF => invalid data)

FRDR1 = FFh

Function Read Message Lookup Table:

(Set up to recognize function read messages)

Generic Function Code          Reference Message

--------------------------------------------------------------------

29h                     vsbar_kph

--------------------------------------------------------------------

Functional Message Lookup Table:

(Set up to recognize incoming Functional messages)

Generic Function Code          Reference Message

--------------------------------------------------------------------

6Ah                     PROCESS_OBDII_REQUEST

54h                     RX_PT_STAT_RQST

93h                     REQUEST_SECURITY_STATUS

Other EBC Registers:

UIMR = FFh;             (User Input Mask Register)

RCR  = F9h;             (Receiver Control Register)

ECR  = 72h;             (EBC Control Register)

UIMR :  Mask out PIP and overvoltage interrupts (Hardware Default

- Software overwrite not necessary).

RCR  :  Mask out - RCV OK, RCV Error, RCV Overrun, Unable to ACK,

Network Activity and Network Fault Interrupts.

Disable  - RCV Node-Node only and Loopback Modes.

ECR  :  Enable   - Network Drivers A & B at 41.6 KBaud

and Never Sleep Mode.

Disable  - Sleep Mode operation and Test Mode Key.

END: CONFIGURE_EBC

5-23

SCP INTERFACES, SCP MESSAGE CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: UPDATE_FUNCTION_READS

Provide Vehicle Speed (in Kph) in a Function Read.

VSFMFLG = 1 -----------------------| vsbar_kph = FFh

(VS sensor failed)                | load vsbar_kph in FR register

|

| --- ELSE ---

|

| vsbar_kph = min((1.60934 * VSBAR),254)

| load vsbar_kph in FR register

|  (Range: 0 - 254 kph => 00 - FE hex)

|  (Res:  1 kph per bit)

|  (FF hex => Invalid vsbar_kph data)

END: UPDATE_FUNCTION_READS

BEGIN: TRANSMIT_VEHICLE_MESSAGE

ECT_DEG_C broadcast command logic.  Note that SEND_ECT is set

via Status Request for the ECT_DEG_C message.  ECT_TIME is a

calibratable constant which should be set between 1-5 seconds.

SEND_ECT = 1 -----------------|

;SR for ECT_DEG_C Broadcast  |

|OR --|

SEND_ECT_SW = 1 --------------|     |

;Fixed time broadcast enable       |

|AND -| send(ECT_DEG_C)

|     | SCP_ECT_TMR := 0

SCP_ECT_TMR >= ECT_TIME ------------|     | ;reset the timer

;At least ECT_TIMR expired since

;the last ECT_DEG_C message transmit

;attempt

END: TRANSMIT_VEHICLE_MESSAGES

5-24

SCP INTERFACES, EBC INITIALIZATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

5.1.6  EBC INITIALIZATION (CDAJ0)

OVERVIEW

This module contains the initialization procedure for  EEC/EBC  power-up  and

EBC  software  reset.   Chip  configuration is executed after all time delays

have elapsed.

DEFINITIONS

INPUTS

Registers:

-  CIR = EBC Configuration Identifier Register (NOT DISPLAYABLE).

-  NAR = EBC Node Address Register (NOT DISPLAYABLE).

OUTPUTS

Registers:

-  ECR = EBC Control Register (NOT DISPLAYABLE).

-  NAR = See above (NOT DISPLAYABLE).

Bit Flags:

-  SCP_UP = Flag to indicate the EBC chip has been successfully initialized.

5-25

SCP INTERFACES, EBC INITIALIZATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SCP_INIT_COM2

BEGIN: INITIALIZE_EBC

;This logic block is called from the SCP_BKGND module upon power up init.

;It may also be called from the SCP_EXTINT module upon recognition of the

;appropriate interrupt bit being set in the EBC.

;It may also be called from the module which commands an EBC reset as a

;failsafe in case the chip should become "locked-up" during a transmit.

unconditionally ------------------------| ECR = 80h; (reset EBC)

| read CIR

CIR <> 02h -----------------------------| EXIT; (no action)

(wrong CIR value)                      |

| --- ELSE ---

|

| NAR = 10h

| read NAR

NAR <> 10h -----------------------------| EXIT; (no action)

(wrong NAR value)                      |

| --- ELSE ---

|

| Do: CONFIGURE_EBC

|  (in MSG_CONTROL module)

|

| SCP_UP = 1

END: INITIALIZE_EBC

5-26

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OVERVIEW

Note that the node to node communications  strategy  assumes  that  only  one

Offboard  Diagnostic  Unit  (ODU)  will  be  connected  to  the  network  and

requesting diagnosic data at any given time.

5.1.7  SIGNIFICANT FEATURES OF SCP OBDII DIAGNOSTICS (CDAM0)

-------------------------------------------------

PIDs and DMR Data Retrieval & SAE Required Communications

A) Data retrieval; supported by  either  the  Parameter  Identifier  (PID)

method,  or  via  a  Direct  Memory  Request  (DMR).  The PID is a unique,

two-byte, hex number assigned to a particular parameter.  It is used  with

parameters  which are frequently referenced.  A list of the PIDs supported

by this strategy is included in the SCP chapter.  The DMR is a request for

the contents of an EEC memory address location.

When data is requested by DMR, a three byte reference is sent on SCP.  The

Upper  byte  controls  whether  the address requested is ROM data (01h for

Bank 1 or 08h for Bank 8), or RAM data (00h).  In any case,  the  response

message  will  include  the  contents  of  the  requested address plus the

following three memory locations.  PID and DMR data can ONLY  be  reported

as a single response.  PID data can only be in the form of a byte, a word,

or a double-word, as referenced in the tables.

Support of the Emission-related messages required by the  SAE  Recommended

Practice  "J1979 E/E Diagnostic Test Modes" includes:  Single PID requests

for EEC performance data; PID requests for regulated  Freeze  Frame  data;

Diagnostic  Trouble  Codes  (DTCs)  which  illuminate the MIL; Clearing of

DTCs, and; Monitoring of the OBDII Oxygen sensor(s).  These  messages  are

typically  used  by  the  OBDII  generic  Scan  Tool  via a Broadcast type

message, however some can be referenced physically from the SBDS tools.

NOTE THAT THE J1979 PIDS ARE NOT INTENDED FOR SERVICE  BY  ANY  REPETITIVE

DATA  TRANSMISSION  SPECIFICATION.   A breakdown of the EEC service of PID

and DMR data requests is presented on the following page.

5-27

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SIGNIFICANT FEATURES OF THE SCP MODULE (continued)

------------------------------------------------------

DATA FORMATS FOR VARIOUS PID/DMR REQUEST & RESPONSE MESSAGES

OBDII SCAN TOOL            ALL OTHER NODES

Data Type       Must be Broadcast        Broadcast or Node-Node

------------------------------------------------------------------------

J1979 PID data      SINGLE REQ/RESP             SINGLE REQ/RESP

(Only Low Byte  (Low Byte of PID precedes  (Two Byte PID number precedes

of PID req'd)   data in returned message)   data in returned message)

------------------------------------------------------------------------

SBDS PID data            N/A                    SINGLE REQ/RESP

(Two Byte PID                             (Two Byte PID number precedes

no. required)                             data in returned message)

-OR-

REPETITIVE PACKET

(Only data returned in Packet)

------------------------------------------------------------------------

SBDS DMR data            N/A                    SINGLE REQ/RESP

(Three Byte DMR                         (DMR memory address precedes FOUR

address required)                       contiguous bytes of data beginning

with requested address in returned

message)

-OR-

REPETITIVE PACKET

(Single byte of data returned in

Packeted data message)

------------------------------------------------------------------------

5-28

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SIGNIFICANT FEATURES OF THE SCP MODULE (continued)

------------------------------------------------------

On-Demand Test Support

B)  Support of On-Demand tests which are built into the EEC include:

the Engine Off sequence; the Engine Running sequence, and; the

Output Test Mode.  These features will allow a quick powertrain

diagnosis of any emission and non-emission related problems, etc.

Part of the On-Demand test sequence is the retrieving of any DTCs

which may have been sent during the test.

The set trouble codes will remain in EEC memory until the next engine

warm-up, or until the next On-Demand test is initiated by the ODU.

However, these codes will only be transmitted once over SCP.

Further definition of the On-Demand state machine is presented in the

figure: ON-DEMAND TEST STATE MACHINE.  In this figure, note that the

SCP messages are shown in UPPERCASE text.  In some cases, to conserve

space, the message names were abbreviated.

The ODU will always follow the same general on-demand sequence...

ODU sends:                       EEC responds:

DIAGNOSTIC_ROUTINE_ENTRY

if entry conditions are met .. GENERAL_RESPONSE <affirmative>

if not ....................... GENERAL_RESPONSE <sequence error>

DIAGNOSTIC_ROUTINE_EXIT

<exit if complete>

if test completed normally ... GENERAL_RESPONSE <normal w/results>

if test aborted .............. GENERAL_RESPONSE <abnormal w/o results>

if test still running ........ GENERAL_RESPONSE <busy repeat request>

-OR-

<abort test>

if test still running ........ GENERAL_RESPONSE <abnormal w/o results>

if test not running .......... GENERAL_RESPONSE <sequence error>

REQUEST_PID_ACCESS               REPORT_PID_DATA

(number of DTC's requested) ...  (OD_CODE_CNT returned)

REQUEST_DIAG_RESULTS

if test exited normally ...... DIAGNOSTIC_RESULTS (send codes once only)

if not ....................... GENERAL_RESPONSE  <sequence error>

5-29

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

KEY:  <- = incoming message

-> = outgoing message

ON-DEMAND TEST STATE MACHINE

____________________________

.REQUEST_DIAG_RESULTS <- OR                            .power up OR

.DIAG_ROUTINE_EXIT           +-------------0           .OBDII_RESET=1

(abort) <- OR          +-->|initial_state|<--------- --------------

.DIAG_ROUTINE_EXIT       |   +-+----+------+           .RESULTS=0

(if complete) <- OR    |     |    |                  .DEMAND_MODE=0

.DIAG_ROUTINE_ENTRY <-   +-----+    |

-------------------------           |PUTMR>4 seconds

.GENERAL_RESPONSE                   |

(seq error) ->                    |               .DIAG_ROUTINE_EXIT <-

+----+       |        +----+ ------------------

|    |       |        |    | .GENERAL_RESPONSE ->

.REQ_DIAG_RESULTS<- OR |  +-+-------V--------+-+  |  (abnormal w/o results)

.[.test NOT ready &    +->|  TEST NOT RUNNING  |<-+

.DIAG_ROUTINE_ENTRY] <-   |  no_results_avail  |<------------------------+

-----------------------   1----A-----A--+------+  .REQ_DIAG_RESULTS <-   |

.GENERAL_RESPONSE              |     |  |         ---------------------  |

(seq error) ->               |     |  |         .DIAGNOSTIC_RESULTS -> |

|     |  |         .RESULTS=0             |

|     |  |                                |

|     |  | .test ready &                  |

+-------------------------------+     |  | .DIAG_ROUTINE_ENTRY <-         |

|.DIAG_ROUTINE_EXIT                   |  | ----------------------         |

| (abort; not OTM) <-                 |  | .GENERAL_RESPONSE              |

| -------------------                 |  |   (affirmative) ->             |

|.GENERAL_RESPONSE ->  .test NOT ready|  | .enable self test              |

| (abnrml w/o rslts)   ---------------|  | .RESULTS=0                     |

|.disable test         .disable test  |  | .DEMAND_MODE=1                 |

|.DEMAND_MODE=0        .DEMAND_MODE=0 |  |                                |

+---------------------------------+   |  |                                |

|   |  |                                |

.DIAG_ROUTINE_ENTRY<- OR +-----+ |   |  |  +-----+ .DIAG_ROUTINE_EXIT    |

.DIAG_ROUTINE_EXIT       |     | |   |  |  |     |   (if complete) <-    |

(wrong test) <- OR      |   +-+-+---+--V--+-+   |  --------------       |

.REQ_DIAG_RESULTS <-     +-->|  test_running |<--+ .GENERAL_RESPONSE     |

---------------------        2-+------+------+      (busy repeat req) -> |

.GENERAL_RESPONSE              |      |                                  |

(sequence error) ->          |      |                                  |

|      |.test_MON=1                       |

.DIAG_ROUTINE_EXIT        |      |--------------                    |

(exit or abort OTM) <-   |      |.disable test                     |

------------------------  |      |.RESULTS=1                        |

.GENERAL_RESPONSE ->      |      |.DEMAND_MODE=0                    |

(normal w/results) +-----V------V-------+                          |

.disable test       |  TEST NOT RUNNING  |--------------------------+

.DEMAND_MODE=0      |    results_avail   |------------------+

.RESULTS=1          3----A----+--------A-+                  |

|    |        |                    |

.[.test NOT ready &        +----+        +--------------------+

.DIAG_ROUTINE_ENTRY] <- OR               .DIAG_ROUTINE_EXIT <-

.REQ_DIAG_RESULTS (wrong test) <-        ------------------

-------------------------------          .GENERAL_RESPONSE

.GENERAL_RESPONSE (sequence error) ->      (normal exit with results) ->

5-30

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Diagnostic Trouble Code Reporting

C) Retrieval of DTCs  other  than  emission-related  codes  is  supported.

However,  the  EEC  will not report more than one list of codes at any one

point in time.

ODU Heartbeat Message

D) The TESTER_PRESENT message  is  relied  upon  as  a  time-out  feature,

indicating  that the ODU has gone off-line.  If the ODU timer ever reaches

a calibratable time delay during a diagnostic  process,  certain  offboard

communications  will  be  terminated.   The  typical  update rate for this

message is every 3 to 5 seconds.  NOTE:  If the EEC has been placed in the

Security   Access   state   (as   part   of   Output   Substitution),  the

SECURITY_REQUEST message which is used to transmit the 'KEY' can take  the

place of the 'heartbeat' requirement.  Both the TESTER_PRESENT and a valid

ODU 'KEY' transmission will clear the ODU_UP_TMR timer.

Rapid Data Transfer

E) A method of reporting a combination of PID and DMR  data  in  the  same

message  (called  Rapid  Data  Transfer)  is  supported through the use of

definable Rapid Packets.  A Packet is a message which is  defined  by  the

ODU  through a dynamic process described below.  The process begins by the

ODU requesting to define a Rapid Packet.  Since  only  one  portion  of  a

rapid  packet  can  be  defined  at  a  time,  several DEFINE_RAPID_PACKET

messages   are   required   to   describe   the   contents   of   a   full

REPORT_RAPID_PACKET message.

Each definition message identifies the position of  a  specific  parameter

within  the  data  packet.   There  are  five  byte positions dedicated to

conveying parameter information in the report message.

Parameters in the packet definition  message  may  be  identified  by  PID

number  or  DMR.   This  allows  a  data packet to include mixed parameter

definition types.   Definition  by  PID  Offset  is  NOT  supported.   ROM

parameters cannot be defined by DMR - only RAM parameters are allowed.

The DEFINE_RAPID_PACKET message  is  variable  in  length.   The  reported

message REPORT_RAPID_PACKET is fixed at seven (7) data bytes.

The following guidelines are set for the definition  and  transmission  of

data with rapid packets on the SCP network:

1) UP TO FIFTEEN PACKETS ARE AVAILABLE FOR  DEFINITION/TRANSMISSION.

The  packet  definition messages define packets, to be referenced as

an EEC RAM array.  This array is to be part of a 200 Byte  ODU  SBDS

RAM  area  which  is  also available for program download/execution.

The RAM array is referenced as SCP_PACKET[i,j]  (where  i  =  packet

number 1-15, and j = data position 1-5 for each packet).  A TOTAL OF

75 BYTES OF DATA ARE AVAILABLE THROUGH RAPID PACKETS.

2) Each packet response message contains a unique SEVEN byte  string

consisting  of  a  pre-defined  mode  number byte (6A hex), a packet

number byte (01h to 0Fh), plus 5 bytes of data (or 00h)  as  defined

via  the  packet  definition  process.   A  request for an undefined

packet will result in a 00h filled response message.

5-31

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SIGNIFICANT FEATURES OF THE SCP MODULE (continued)

------------------------------------------------------

Rapid Data Transfer (continued)

3) Data within a packet will be sent in the exact order in which  it

was specified.  In the event the ODU requests data beyond 5 bytes in

a given packet, the EEC will truncate the data  to  5  bytes.   This

means that the data will NOT be wrapped to another packet.

4) A Single rate is available to the user  to  send  packets.   This

rate  is  FIXED  in  E-EEC memory.  In order to maintain the kind of

packet to be sent, a symbolic 'queue' is utilized in this chapter.

The second data byte (scp_data_2)  of  the  Packet  Request  Message

defines  the rate at which to send that group of packets.  A rate of

00h will temporarily disable repetitive transmission for a group  of

packets.   A  rate  of  01h or 02h will result in a GENERAL_RESPONSE

message stating that the requested update rate is not supported.   A

rate  of  04h will result in the requested packets to be transmitted

at a Type 'A' update rate.

Note:  For backward compatiblity, requests for rate 03h and 04h will

both be supported by the Type 'A' response.

5) Type 'A' rate reporting will cycle through the list of  requested

packets,  transmitting  two  packets  every  10  +/- 2 milliseconds.

There may exist packets which have been defined, but  not  requested

for  repetitive  transmission.   The  E-EEC will only transmit those

packets which are requested via the REQUEST_RAPID_PACKET message.

6)   To   stop   a   single   Packet    from    transmitting,    the

REQUEST_RAPID_PACKET   message  is  sent  for  that  Packet  with  a

transmission rate of  00h.   The  REQUEST_ALL_DATA_STOP  message  is

required in order to stop all Packet repetitive data transmissions.

SCP Network Bandwidth Considerations

F) The EESE group has indicated that the SCP network  traffic  should  not

exceed  40% of the databus bandwidth during diagnostic communications.  It

is left to the OFFBOARD UNIT to see that the SCP databus is not overloaded

and that the vehicle performance is not affected by the diagnostic tests.

Bus loading beyond 40% bandwidth may cause increasing message latency.

5-32

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SIGNIFICANT FEATURES OF THE SCP MODULE (continued)

------------------------------------------------------

Downloading and Executing a RAM Program

G) A facility to download data and  diagnostic  routines  to  EEC  RAM  is

provided  to  improve  vehicle  serviceability.   The  routine  will allow

download to RAM registers ONLY.  There are currently no boundary checks to

limit  the download to the 200 Byte SBDS RAM area; however, such a feature

could be incorporated into future releases of this strategy.

The download sequence is designed around  a  finite  state  machine  which

requires  a distinct order of execution to successfully store a program to

EEC RAM and execute  it.   If  this  execution  sequence  is  not  met,  a

GENERAL_RESPONSE message will be sent informing the user of the error.

The EEC will have two types of download program execution  which  must  be

specified with the message requesting the program execution; one maintains

the normal background routine executions sequence, while the other  spawns

a separate SBDS background sequence.  The latter consists primarily of the

SCP interfacing software and the downloaded program itself.  This type  of

program  can  ONLY  be executed while the engine is in CRANK or UNDERSPEED

Modes.

The process of downloading and executing RAM  programs  is  shown  in  the

DOWNLOAD & EXECUTE STATE MACHINE, but generally follows this sequence:

(Initialize SCP_DWNLD_ST to zero, state=0)

ODU sends:                          EEC responds:

DOWNLOAD_ROUTINE_ENTRY              GENERAL_RESPONSE (set state=1)

(START ADDRESS & Byte COUNT)       (Good data & PUTMR>4: affirmative)

DOWNLOAD_BLOCK_DATA                 No response required (state=2)

(repeat as necessary)

DOWNLOAD_ROUTINE_EXIT               GENERAL_RESPONSE (set state=3)

(when data is complete)             (normal exit if COUNT = 0)

(abnormal exit if COUNT <> 0)

EXECUTE_ROUTINE                     GENERAL_RESPONSE (state=3)

(xqt at address and with            (affirmative)

background execute sequence)        begin execution at address

Routine termination via Key         No response required.

Off or via program.

IN ORDER TO MAINTAIN EXECUTION OF A DOWNLOADED ROUTINE,

THE OFFBOARD TESTER MUST EITHER SERVICE THE ODU_UP_TMR OR

PERIODICALLY SEND THE TESTER_PRESENT MESSAGE (3-5 sec).

5-33

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DOWNLOAD & EXECUTE STATE MACHINE

________________________________

KEY:  <- = incoming message

-> = outgoing message

+-----+

.DWNLD_BLOCK_DATA <- OR   |     |

.DWNLD_ROUTINE_EXIT <- OR |   +-+--------------------+

.EXECUTE_ROUTINE <-       +-->|  download_not_ready  |<------- .power up

-------------------           |      State = 0       |

.GENERAL_RESPONSE             +---A--+---------------+

(sequence error) ->              |  |   .PUTMR>4 seconds AND

|  |   .DOWNLOAD_ROUTINE_ENTRY <-

|  |    .Start Address

.DOWNLOAD_ROUTINE_EXIT <-  |  |    .Byte Count

-------------------------  |  |   --------------------

.GENERAL_RESPONSE          |  |   .GENERAL_RESPONSE

(affirmative) ->          |  |    (affirmative) ->

|  |

|  |

.DWNLD_ROUTINE_ENTRY <- OR        |  |

.EXECUTE_ROUTINE <-       +-------+--V---------+

--------------------      |  download_entered  |

.GENERAL_RESPONSE     +-->|     State = 1      |

(sequence error) ->  |   +-+--------+---------+

|     |        | .DOWNLOAD_BLOCK_DATA <-

+-----+        | ----------------------

| .data is downloaded

.DOWNLOAD_BLOCK_DATA <-              |

----------------------  +-----+      |      +-----+ .EXECUTE_ROUTINE <- OR

.data is downloaded     |     |      |      |     | .DWNLD_ROUT_ENTRY <-

|  +--+------V------+-+   | ---------------------

+->| downloading_data |<--+ .GENERAL_RESPONSE

|    State = 2     |      (sequence error) ->

+---------+--------+

|

.DWNLD_BLOCK_DATA <- OR              | .DOWNLOAD_ROUTINE_EXIT <-

.DWNLD_ROUTINE_EXIT <-               | -------------------------

-------------------                  | .GENERAL_RESPONSE

.GENERAL_RESPONSE    +-----+         |  (affirmative) ->

(sequence error) -> |     |         |

|   +-+---------V--------+

.EXECUTE_ROUTINE <-  +-->|    execute_ready   |

-------------------  +-->|     State = 3      |

.GENERAL_RESPONSE    |   +-+------A--+--------+

(affirmative) ->    |     |      |  |

+-----+      |  | .DOWNLOAD_ROUTINE_ENTRY <-

|  |   .Start Address

.DOWNLOAD_ROUTINE_EXIT <-   |  |   .Byte Count

--------------------------- |  | -------------------------

.GENERAL_RESPONSE           |  | .GENERAL_RESPONSE

(affirmative) ->           |  |  (affirmative) ->

+--------+--V--------+

.DWNLD_ROUT_ENTRY <- OR  | download_&_execute |

.EXECUTE_ROUTINE <-  +-->|     State = 4      |<--+ .DOWNLOAD_BLOCK_DATA <-

-------------------  |   +-+----------------+-+   | ----------------------

.GENERAL_RESPONSE    |     |                |     | .data is downloaded

(sequence error) ->  +-----+                +-----+

5-34

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SIGNIFICANT FEATURES OF THE SCP MODULE (continued)

------------------------------------------------------

Output Substitution (Output State) Control

H) This capability allows the ODU to  verify  proper  operation  of  external

output  components  and  circuitry by isolation techiques.  Direct control of

output devices can be achieved with this  feature.   Since  substitution  may

cause  the EEC to operate in a manner which is unsuitable for "down-the-road"

operation, a security access routine must be  successfully  processed  before

substitution  is allowed.  Additionally, the "Tester Present" message is also

required to ensure the the EEC is returned to  normal  before  returning  the

vehicle  to  the  customer.  This mode is for use by Ford authorized SBDS and

End-Of-Line service technicians only.  To ensure a  substitution  request  is

properly  handled  (and  the  request  is  granted  or denied), it is a basic

assumption that the service technician will wait  for  a  response  from  one

request before sending another request for the same channel.

There is a four  layer  security  scheme  in  place  to  prevent  inadvertent

substitution.   It  is extremely important that the substitution of values in

the control strategy never occurs unless the vehicle is in  the  service  bay

and  being worked on by the service technician.  The security scheme has been

designed to ensure  this.   The  implementation  in  the  strategy  has  been

designed  to  prevent  activation  by  any  means  other than through the SCP

interface with an authorized ODU.

The  first  layer  requires  an  ODU  to  transmit  a   'heartbeat'   message

periodically.  This tells the strategy an ODU is currently on the network and

actively communicating.  NOTE:  This  can  either  be  accomplished  via  the

TESTER_PRESENT  message, or (when in the Security Access state - See Below) a

transmission of the valid 'KEY' value via the SECURITY_REQUEST message.

The second layer requires the ODU to  request  a  seed  (randomly  generated)

value   from   the  EEC  and  succesfully  transform  it  via  a  proprietary

transformation algorithm.  The ODU then returns the transformed value.  If it

matches  the internally calculated value, this layer's requirements have been

met.  This ensures only authorized ODUs gain access to the OSC mode.

The third layer requires the ODU to transmit an  enable  mask  and  requested

substitution  value  for  the channel it wishes to override.  The enable mask

must match the predetermined enable mask that is coded into  the  EEC.   This

allows  for  unlocking  of  each  individual  channel number in the OSC table

seperately, thus minimizing the risk that other channels get overridden  when

not requested.

The final layer verifies, each background loop when the parameter is  written

to,  that  the vehicle is in an operational state that allows the override of

the  requested  channel(s)  and  that  the  override  value  is  within  some

guidelines.   If  the  override  value  is  acceptable and the enable mask is

correct the channel's normal value will be overridden and will remain in that

state  until  one  of  the security layers is breached or the ODU changes the

value.  This layer ensures that the ODU will not damage the vehicle or  place

the vehicle in an unsuitable operating state.

5-35

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Pictorially the security scheme looks something like the following:

|.ODU present

+----V---------------------+

|                          |

|    |.key is correct      |

|+---V--------------------+|

||                        ||

||   |.enable mask O.K.   ||

||+--V-------------------+||

|||                      |||

|||  |.override currently|||

|||  | allowed           |||

||| +V--------------+    |||

||| | OSC enabled   |    |||

||| +---------------+    |||

||+----------------------+||

|+------------------------+|

+--------------------------+

Portions of the Output  State  Control  feature  are  located  in  the  OBDII

chapter,  under  the  heading  OBDII_OSC_COMx.   All  details  concerning the

communications interface for OSC are described in this module.

The messages used to enable and control the PCM through Output State  Control

are  listed, in order, below.  In general, this is the process used to unlock

and control the outputs of  the  Powertrain  Control  Module.   However,  the

process below does not necessarily cover all of the message combinations.

MESSAGING PROCESS FOR OUTPUT STATE CONTROL

1)  TESTER_PRESENT must have been issued within the last ODU_TIMEOUT (3-5)

seconds.  This requirement continues throughout the OSC process.  If

the ODU 'times-out', then the OSC is halted and the PCM returns to

its normal operating mode.

2)  SECURITY_REQUEST must have been issued with an 01h identifier to

request the SEED.

3)  The PCM must have a non-zero 'clock' byte from which the SEED is

generated.  If not, then the ODU is informed that the PCM is  'busy'

and that it should 'repeat its request'.  This is transmitted via the

GENERAL_RESPONSE.  If the 'clock' is non-zero, then the SEED is

transmitted to the ODU via SECURITY_RESPONSE.

4)  The ODU must transmit a calculated KEY value, per the proprietary

algorithm shown in this document, via the SECURITY_REQUEST message,

with an 02h identifier in the second byte.

5)  The ODU key must match the PCM-calculated eec_key value.  The message

GENERAL_RESPONSE is used to either confirm 'access granted' or

indicate 'access denied - device secured'.

5-36

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

MESSAGING PROCESS FOR OUTPUT STATE CONTROL (cont'd)

6)  If 'access granted', then the ODU must transmit an OSC channel 'enable

mask' for each channel it whishes to manipulate.  The channel must be

supported by the PCM and the mask must be equivalent to the ROM-coded

mask byte within  the PCM.  The mask is transmitted via the

EXECUTE_ROUTINE message, with a command word of 0047h.

7)  The PCM again uses GENERAL_RESPONSE to indicate success or failure of

the EXECUTE_ROUTINE request.

8)  The OSC must then utilize the PARAMETER_SUBSTITUTION message to

control the unlocked output channel.  Once again, the channel must be

supported by  the PCM.  In addition, the value must be within the

bounds specified in the OBDII_OSC_COMx strategy chapter, or else the

PCM will clip the commanded value to the internally-specified max/min

limit.  The ODU WILL NOT be notified of this clip.

9)  The PCM uses GENERAL_RESPONSE to indicate success or failure of

PARAMETER_SUBSTITUTION requests.

10) The ODU can disable OSC for a given channel by  transmitting the

PARAMETER_SUBSTITUTION without a substitute value.  It can exit OSC by

merely  ceasing to transmit the TESTER_PRESENT, or by  issuing an

incorrect KEY value, via SECURITY_REQUEST.

5-37

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  CODES_COUNT = Count of all codes with PxxxFAULT = , or PxxxSTATE > 0.

-  DLOAD_EXEC_KEY = Command Key byte  to  enable  execution  of  a  download

program.

-  DLOAD_PROG_STRT = Address at which  to  begin  execution  of  a  download

program.

-  KAMQA = KAM Qualification test register 1.

-  NUM_CODES = Count of all MIL-codes (with PxxxFAULT = 1).

-  OD_CODE_CNT = Count of all On-demand codes (with PxxxCNT = 0).

-  ODU_UP_TMR = Timer used to  indicate  the  Offboard  Diagnostic  Unit  is

on-line.  (incrementing timer, msec).

-  ODU_KEY = Key transmitted to the EEC from an  ODU  as  part  of  the  SCP

Security Access process.

-  OSC_ENAx = Output State Control Enable Mask, where 'x' = channel number.

-  OSC_SEED = Randomly generated Seed value transmitted to the  ODU  by  the

EEC as part of the SCP Security Access process.

-  OSC_STATE = Output State Control state variable.

-  OSC_VALUEx = Output State Control substitution  value  for  a  given  'x'

channel.

-  OUTPUT_MODE = Level of EEC outputs to affect (in Output Test Mode only).

-  PUTMR = Time since last power up (seconds).

-  PxxxSTATE = State register for any given diagnostic trouble code.

-  PxxxCNT = Warm-up counter register for any given diagnostic trouble code.

-  SCP_DIAG_ST = State register for the On-demand test scheduler.

-  SCP_DWNLD_ST = State register for the Download and Execute scheduler.

-  SCP_PACKET[] = Array of  [1..15,1..5]  data  locations  to  maintain  the

latest Rapid Data Packet message DATA BYTES to be sent on SCP.

-  SCP_TST_REQ = Last On-demand test requested from ODU.

5-38

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Bit Flags:

-  A_RATE_ENA = Flag set when SCP Packets are requested at Type 'A' rate.

-  CRKFLG = CRANK FLAG (1 = In Crank mode).

-  DEMAND_MODE = Flag set when an On-Demand selt test mode is active.

-  OBDII_RESET = Flag used to indicate the  receipt  of  an  SCP  diagnostic

reset message of any type.

-  ODU_UP_STATE = Flag set when Offboard Unit is actively communicating with

the EEEC over SCP; 1 -> on-line and communicating.

-  PxxxFAULT = Fault bit for any given diagnostic trouble code.

-  RESULTS = Flag set when an On-Demand test has executed successfully.

-  SCPEO_START = Signal to start the engine off self test.

-  SCPEO_EXIT = Signal to abort the engine off self test.

-  SCPER_START = Signal to start the engine running self test.

-  SCPER_EXIT = Signal to abort the engine running self test.

-  SCPOTM_START = Signal to start output test mode diagnostic procedure.

-  SCPOTM_EXIT = Signal to abort the output test mode diagnostic procedure.

-  UNDSP = Underspeed flag; 1 -> Underspeed.

Calibration Constants:

-  ODU_TIMEOUT = Time-out value for certain offboard diagnostic routines.

-  OSC_MASK = Mask byte which ODU must match in order to unlock a given  OSC

channel for parameter substitution (part of Output State Control).

OTHER

-  scp_data_n = Data byte in SCP message (n = 1 through 7).

-  scp_target = SCP message Target Specifier byte for Broadcast messages.

-  scp_pri_type = SCP message Priority/Type byte for transmit messages.

-  scp_src_nar =  Source  Node  Address  Register  from  received  Node-Node

message.

5-39

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SCP_OBDII_DIAG_COM1

The following assignments define key parameter logicals which are  referenced

throughout this chapter.  It is not necessarily intended to be executable.

BEGIN: parameter_definitions

unconditionally ------------------------| dwnld_not_ready = 0

(Definitions used for SCP_DWNLD_ST     | dwnld_entered = 1

documentation purposes only)         | dwnld_data = 2

| execute_ready = 3

| dwnld_&_execute = 4

|

(Definitions used for SCP_DIAG_ST       | initial_state = 0

documentation purposes only)         | no_results_avail = 1

| test_running = 2

| results_avail = 3

|

(Definitions for OSC)                   | osc_initial_state := 0

| osc_seed_sent := 1

| key_received := 2

END: parameter_definitions

5-40

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS_OBDII_REQUEST

Message Type:  Broadcast Receive.

Purpose of Message:  Request specific J1979 OBDII-related action to be taken

This message is used to broadcast a diagnostic request of J1979 Mode 01

(get PID data), Mode 02 (get Freeze Frame PID data), Mode 03 (get the

set MIL-code DTCs), Mode 04 (Clear all emissions related information),

Mode 06 (request on-board monitoring test results for non-continuously

monitored systems), and Mode 07 (request on-board monitoring test results

for continuously monitored systems).

Data Flow:  From ODU (ex. scan tool).

Message as retreived from the Receive Buffer:

scp_src_nar (ODU Node Address)     = Fxh

scp_data_1 (J1979 Mode)            = xxh

scp_data_2 (optional J1979 data)   = xxh

scp_data_3 (optional J1979 data)   = xxh

Receive Message Routine (executed only upon reception of message):

scp_data_1 = 01 ------------------------| pid_lo = scp_data_2

(send response message < 100ms after   |  (lo byte of PID,pid_hi=00h)

receiving PROCESS_OBDII_REQUEST)      |  (If pid is not found: EXIT)

(Single Request/Single Response Only)  | send(REPORT_OBDII_PID)

(Get PID request)                      |

| --- ELSE ---

|

scp_data_1 = 02 ------------------------| ff_pid_lo = scp_data_2

(send response message < 100ms after   |  (lo byte of PID,ff_pid_hi=00h)

receiving PROCESS_OBDII_REQUEST)      |  (If pid is not found: EXIT)

(Single Request/Single Response Only)  | send(REPORT_OBDII_FF_PID)

(Get Freeze Frame data)                |

| --- ELSE ---

|

scp_data_1 = 03 ------------------------| send(REPORT_OBDII_CODES)

(send response message < 100ms after   |  (send all codes in the set of

receiving PROCESS_OBDII_REQUEST)      |   mil_codes with PxxxFAULT=1)

(Get mil_code data)                    |

| --- ELSE ---

|

scp_data_1 = 04 ------------------------| OBDII_RESET = 1

(send response message < 100ms after   | (Executive Resets All DTC's)

receiving PROCESS_OBDII_REQUEST)      | send(REPORT_OBDII_RESET)

|

| --- ELSE ---

|

scp_data_1 = 06 ------------------------| test_id = scp_data_2

(send response message < 100ms after   | Do: report_test_data

receiving PROCESS_OBDII_REQUEST)      |  ; Request Test Results

|

| --- ELSE ---

(continued on next page)

5-41

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS_OBDII_REQUEST

(continued from previous page)

|

scp_data_1 = 07 ------------------------| send (REPORT_OBDII_CONT_RESULTS)

(send response message < 100ms after   |  (send all codes in the set of

receiving PROCESS_OBDII_REQUEST)      |   mil_codes with

(Send Pending Code Data)               |   PxxxSTATE>0 and PxxxFAULT=0)

BEGIN: report_test_data

The purpose of this test mode is to allow access to the results of on-board

diagnostic monitoring tests of specific components/systems performed

during normal vehicle operation.

Requests are made via test IDs.  A test ID identifies a specific test result.

The manufacturer is responsible for defining test IDs and component IDs.

Only one test limit is included in a response message, and the limit can

be either a minimum or a maximum.  If a test has both a minimum and a maximum

to report, two reporting messages will be sent for the request.  Also, if

more than one component performs the same test, multiple messages will be

sent to report the test results for each component.

Test IDs are in the range 00h to FFh.  These are divided into 8 sets

of 32 Test IDs.  Because of the large number of tests, and components for

which this could be used  to report test results, test IDs are not

standardized.  Furthermore, there is a requirement to be able to report

which test IDs do exist in any given block of 32 IDs.

In effect, there are two possible reporting formats.  The first, as

mentioned above, requires a message to be returned that reports the

supported test IDs out of a block of 32 into a 4 byte message response.

The responses for this message are bitmapped into calibratable parameters.

The second type reports the test value, test limit, test limit type, and

component ID for a specific Test ID.  Multiple messages of this type will

be sent if conditions warrant, as described above.

5-42

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS_OBDII_REQUEST

The following chart illustrates the message formats for messages received

and transmitted for Mode 06 reporting.

SCP MODE 06 MESSAGE DATA BYTES

|==================================================|

|          scp_data_n (Hex)                        |

|--------------------------------------------------|

| #1 |  #2  |   #3       | #4  | #5   | #6  | #7   |

|==================================================|

|         Request Test Results (EEC Receives)      |

|--------------------------------------------------|

| 06 | Test |                                      |

|    |  ID  |                                      |

|==================================================|

|==================================================|

|         Report Test ID Support (EEC Sends)       |

| (Test IDs 00h, 20h, 40h, 60h, 80h, A0h, C0h, E0h)|

|--------------------------------------------------|

| 46 | Test |   FF       | Support for the next 32 |

|    |  ID  |            | Test IDs following the  |

|    |      |            | requested ID.           |

|==================================================|

|         Reort Test Results (EEC Sends)           |

|--------------------------------------------------|

| 46 | Test | Test Limit | test value | test limit |

|    |  ID  | type and   |-------------------------|

|    |      | component  | MSB | LSB  | MSB | LSB  |

|    |      |    ID      |-------------------------|

|==================================================|

5-43

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS_OBDII_REQUEST

See the Response Table in SCP_MODE06_MAP_xx for reply message

content.

Note that the number of messages sent in response to a given

Test ID is equal to the number of row entries in the Response

Table that match the requested Test ID.

unconditionally for each --------| send (REPORT_OBDII_TRIP_RESULTS,46h,

matching Test ID                 |       test_id,scp_data_3,

entry in the SCP MODE 06        |       scp_data_4,scp_data_5,

TEST ID MAP Response Table       |       scp_data_6,scp_data_7)

END: report_test_data

5-44

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINE_RAPID_PACKET

Message Type:  Node to Node Receive.

Purpose of Message:  Request to set up diagnostic rapid data transfer  packet

which is used for quick transmission of EEC data on SCP

Data Flow:  From Offboard Diagnostic Unit (ODU).

Data Description:   Several  DEFINE_RAPID_PACKET  messages  are  required  to

define  the  contents  of a 'fully packed' REPORT_RAPID_PACKET message.  Each

definition message identifies the position of a specific parameter within the

data  packet.   The  are five byte positions dedicated to conveying parameter

information in the REPORT_RAPID_PACKET message.  The positions  are  referred

to as locations 1 thru 5; where position 1 is 'scp_data_2' of the message.

Parameters in the packet definition message may be identified by  PID  number

or  DMR  address.   This  allows  a  data  packet  to include mixed parameter

definition types.  As can be  seen  in  the  message  below,  two  bytes  are

required  for  a  PID  identifier,  and  three  bytes  are required for a DMR

identifier.  Definition by PID offset is NOT supported.

The  normal   response   to   each   DEFINE_RAPID_PACKET   message   is   the

CONFIRM_RAPID_PACKET message.

Message Specification:

scp_src_nar (ODU Node Address)     = Fxh

scp_data_1 (Define Rapid Packet)   = 2Ch

scp_data_2 (Rapid Data Packet num) = pkt_num

scp_data_3 (Type/Position/Length)  = tpl_data

scp_data_4 (Offset/DMR/PID spec)   = odp_spec

OPTIONAL >>> scp_data_5 (DMR/PID specification) = dp_spec

OPTIONAL >>> scp_data_6 (DMR specification)     = d_spec

Other Specifications:

The construct SCP_PACKET[packet number, position in packet] is used below for

referencing rapid packet formation.

All packet definition structures created via this message are to be stored in

the 200 byte area designated for SBDS/EOL useage.

5-45

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINE_RAPID_PACKET

Receive Message Routine (only executed upon message reception):

unconditionally -----------------------| type = bits 7,6 of tpl_data

| posit = bits 5,4,3 of tpl_data

| length = bits 2,1,0 of tpl_data

pkt_num = 00h -------------------|

|

pkt_num > 0Fh -------------------|

|

type > 02h ----------------------|

|

posit > 05h ---------------------|OR --| send(GENERAL_RESPONSE: 7Fh,2Ch,

|     |          scp_data_2,scp_data_3,

length = 00h --------------------|     |          scp_data_4,12h)

|     |  (sub-func not supported,

length > 04h --------------------|     |   invalid format)

|     |

posit + length > 6 --------------|     |

|     |

type = 01 -----------------|     |     |

|AND -|     |

PID not supported in map --|           |

| --- ELSE ---

|

posit = 00h ---------------------------| SCP_PACKET[pkt_num,1] = 00h

(clear postns 1..5 in pkt_num)        | SCP_PACKET[pkt_num,2] = 00h

| SCP_PACKET[pkt_num,3] = 00h

| SCP_PACKET[pkt_num,4] = 00h

| SCP_PACKET[pkt_num,5] = 00h

| send(GENERAL_RESPONSE: 7Fh,2Ch,

|      pkt_num,tpl_data,odp_spec,00h)

|   (confirm packet was defined)

|

| --- ELSE ---

|

type = 02h ----------------------------| ad_rqst = scp_data_5,scp_data_6

(define up to 4 bytes RAM data by DMR)|   (ad_rqst is a RAM address)

| Do: load_scp_packet

|

| --- ELSE ---

(continued on next page)

5-46

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINE_RAPID_PACKET

(continued from previous page)

|

type = 01h -----------------------------| pidxxxx = scp_data_4,scp_data_5

(define up to 4 bytes by PID)          | SCP_PACKET[pkt_num,posit] =

|                      (pidxxxx data)

|  (1 to 4 Bytes of PID data

|   requested)

| send(GENERAL_RESPONSE: 7Fh,2Ch,

|      pkt_num,tpl_data,odp_spec,00h)

|   (confirm packet was defined)

|

| --- ELSE ---

|

| send(GENERAL_RESPONSE: 7Fh,2Ch,

|           scp_data_2,scp_data_3,

|           scp_data_4,12h)

|   (invalid format)

BEGIN: load_scp_packet

length = 01h --------------| SCP_PACKET[pkt_num,posit] = (pointer to ad_rqst)

(define 1 byte)           | send(GENERAL_RESPONSE: 7Fh,2Ch,

|      pkt_num,tpl_data,odp_spec,00h)

|   (confirm packet was defined)

|

| --- ELSE ---

|

length = 02h --------------| SCP_PACKET[pkt_num,posit] = (pointer to ad_rqst)

(define 2 contiguous bytes)| SCP_PACKET[pkt_num,posit+1] = (ad_rqst+1 pntr)

| send(GENERAL_RESPONSE: 7Fh,2Ch,

|      pkt_num,tpl_data,odp_spec,00h)

|   (confirm packet was defined)

|

| --- ELSE ---

|

length = 03h --------------| SCP_PACKET[pkt_num,posit] = (pointer to ad_rqst)

(define 3 contiguous bytes)| SCP_PACKET[pkt_num,posit+1] = (ad_rqst+1 pntr)

| SCP_PACKET[pkt_num,posit+2] = (ad_rqst+2 pntr)

| send(GENERAL_RESPONSE: 7Fh,2Ch,

|      pkt_num,tpl_data,odp_spec,00h)

|   (confirm packet was defined)

|

| --- ELSE ---

|

length = 04h --------------| SCP_PACKET[pkt_num,posit] = (pointer to ad_rqst)

(define 4 contiguous bytes)| SCP_PACKET[pkt_num,posit+1] = (ad_rqst+1 pntr)

| SCP_PACKET[pkt_num,posit+2] = (ad_rqst+2 pntr)

| SCP_PACKET[pkt_num,posit+3] = (ad_rqst+3 pntr)

| send(GENERAL_RESPONSE: 7Fh,2Ch,

|      pkt_num,tpl_data,odp_spec,00h)

|   (confirm packet was defined)

END: load_scp_packet

5-47

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DIAGNOSTIC_ROUTINE_ENTRY

Message Type: Node to Node Receive.

Purpose of Message: Request to enable on-demand self test strategies.

Data Flow: When an ODU sends this message to the EEC, one of three Self Test

enable flags are set. The on-demand strategy determines whether or not Self

Test can be executed. If the Self Test can be fully executed, the EEC will

send the GENERAL_RESPONSE message indicating a "normal exit with results".

If for any reason the Self Test was not completed, or aborted, the EEC will

send GENERAL_RESPONSE to indicate this to the ODU.

Message Specification:

scp_src_nar (ODU Node Address)     = Fxh

scp_data_1 (Diag. Routine Entry)   = 31h

scp_data_2 (Specific Diag Routine) = xxh

Receive Message Routine (only executed upon message reception):

SCP_DIAG_ST = initial_state ------|

(=0)  |OR --| send(GENERAL_RESPONSE: 7Fh,31h,

SCP_DIAG_ST = test_running -------|     |         scp_data_2,00h,00h,22h)

(not ready for OD testing) (=2)        |   (sequence error)

(test already in progress)             |

| --- ELSE ---

|

scp_data_2 = 81h -----------------------| SCP_TST_REQ = scp_data_2

(Engine Off test requested)            | SCPEO_START = 1

|   (Enable Engine Off Test)

|

| --- ELSE ---

|

scp_data_2 = 82h -----------------------| SCP_TST_REQ = scp_data_2

(Engine Running test requested)        | SCPER_START = 1

|   (Enable Engine Running Test)

|

| --- ELSE ---

|

scp_data_2 = 84h -----------------------| SCP_TST_REQ = scp_data_2

(Output Test Mode requested)           | SCPOTM_START = 1

| OUTPUT_MODE = 00h

|   (Enable Output Test Mode)

|

| --- ELSE ---

|

| SCP_TST_REQ = 00h

| send(GENERAL_RESPONSE: 7Fh,31h

|         scp_data_2,00h,00h,12h)

|  (sub-function not supported)

5-48

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DIAGNOSTIC_ROUTINE_EXIT

Message Type:  Node to Node Receive.

Purpose of Message:  Request to exit from on-demand self test.

Data Flow:  When an ODU sends this message to the EEC, one of three Self Test

disable flags can be set.  The following strategy determines the current

state of the diagnostic executive via SCP_DIAG_ST register and takes the

appropriate action.  If the ODU breaks the on-demand routine sequence, a

"sequence error" will be sent.  If an 'exit if complete' is requested for

the routine currently executing (with the exception of the Output Test Mode),

the EEC will respond with "busy - repeat request".  Otherwise, the routine

will set the appropriate disable flag, for the diagnostic executive to abort

its test and send the appropriate GENERAL_RESPONSE message confirming the

abort.  The EEC will wait for the ODU to request the test results.

Message Specification:

scp_src_nar (ODU Node Address)     = Fxh

scp_data_1 (Diag. Routine Entry)   = 32h

scp_data_2 (Specific Diag Routine) = xxh

scp_data_3 (Type of EXIT requested)= xxh

Receive Message Routine (only executed upon message reception):

SCP_TST_REQ <> scp_data_2 --------|

|

SCP_TST_REQ = 00h ----------------|OR --| send(GENERAL_RESPONSE: 7Fh,32h,

(referencing wrong test)       |     |      scp_data_2,scp_data_3,00h,22h)

|     |   (sequence error)

SCP_DIAG_ST = initial state ------|     |

(not ready for test: state=0)          |

| --- ELSE ---

|

SCP_DIAG_ST = results_avail ------------| send(GENERAL_RESPONSE: 7Fh,32h,

(test completed successfully) (=3)     |      scp_data_2,scp_data_3,00h,61h)

|   (normal exit with results)

|

| --- ELSE ---

|

SCP_DIAG_ST = no_results_avail ---------| send(GENERAL_RESPONSE: 7Fh,32h,

(test was aborted/never ran)  (=1)     |      scp_data_2,scp_data_3,00h,64h)

|   (abnormal exit without results)

|

| --- ELSE ---

|

scp_data_2 = 84h -----------------------| OUTPUT_MODE = 00h

(requesting end of OTM tests)         | SCPOTM_EXIT = 1

|   (Disable Output Test Mode)

|

| --- ELSE ---

(continued on next page)

5-49

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DIAGNOSTIC_ROUTINE_EXIT

(continued from previous page)

scp_data_3 = 00h -----------------------| send(GENERAL_RESPONSE: 7Fh,32h,

(query test status via the            |      scp_data_2,scp_data_3,00h,21h)

'exit if complete' request           |   (busy - repeat request)

ie. "ARE YOU DONE YET?")             |

| --- ELSE ---

|

scp_data_3 = 01h -----------------------| Do: abort_routine

(abort unconditionally requested)      |

| --- ELSE ---

|

| send(GENERAL_RESPONSE: 7Fh,32h,

|      scp_data_2,scp_data_3,00h,12h)

|   (sub-function not supported)

BEGIN: abort_routine

scp_data_2 = 81h -----------------------| SCPEO_EXIT = 1

(requested abort of EO test)          |   (Disable Engine Off Test)

|

| --- ELSE ---

|

scp_data_2 = 82h -----------------------| SCPER_EXIT = 1

(requested abort of ER test)          |   (Disable Engine Running Test)

END: abort_routine

5-50

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DOWNLOAD_ROUTINE_ENTRY

Message Type:  Node to Node Receive.

Purpose of Message: The First Step in the Download Sequence which requests

that the EEC enter a program download receive mode.

Block transfer formats other than 80 are not supported.

Format 80 means "no handshaking required".

Data Flow:  From Offboard Diagnostic Unit (ODU).

Message Specification:

scp_src_nar (ODU Node Address)     = Fxh

scp_data_1 (Dwnld Routine Entry)   = 34h

scp_data_2 (Block Transfer Format) = xxh

scp_data_3 (Hi Byte Total Byte Cnt)= xxh

scp_data_4 (Lo Byte Total Byte Cnt)= xxh

scp_data_5 (Hi Byte Start Address) = xxh

scp_data_6 (Mid Byte Start Address)= xxh

scp_data_7 (Lo Byte Start Address) = xxh

Receive Message Routine (only executed upon message reception):

unconditionally ------------------------| total_bytes = scp_data_3,scp_data_4

| start_adrs = scp_data_6,scp_data_7

scp_data_2 <> 80h ----------------------| send(GENERAL_RESPONSE: 7Fh,34h,

(format other than 80 requested)       |          scp_data_2,scp_data_3,

|                 scp_data_4,12h)

| (invalid format)

|

| --- ELSE ---

PUTMR > 4 seconds ----------------|     |

|AND -| SCP_DWNLD_ST = dwnld_entered (=1)

SCP_DWNLD_ST = dwnld_not_ready ---|     | (save total_bytes count and

(ready for first download request; =0) |  start_adrs for use with the

|  DOWNLOAD_BLOCK_DATA messages)

| (begin download at start_adrs)

| send(GENERAL_RESPONSE: 7Fh,34h,

|          scp_data_2,scp_data_3,

|                 scp_data_4,00h)

|   (affirmative)

|

| --- ELSE ---

(continued on following page)

5-51

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DOWNLOAD_ROUTINE_ENTRY

(continued from previous page)

|

SCP_DWNLD_ST = execute_ready -----------| SCP_DWNLD_ST = dwnld_&_execute(=4)

(already entered download) (=3)        | (save total_bytes count and

|  start_adrs for use with the

|  DOWNLOAD_BLOCK_DATA messages)

| (begin download at start_adrs)

| send(GENERAL_RESPONSE: 7Fh,34h,

|          scp_data_2,scp_data_3,

|                 scp_data_4,00h)

|   (affirmative)

|

| --- ELSE ---

|

| send(GENERAL_RESPONSE: 7Fh,34h,

|          scp_data_2,scp_data_3,

|                 scp_data_4,22h)

|   (conditions not correct)

5-52

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DOWNLOAD_BLOCK_DATA

Message Type:  Node to Node Receive.

Purpose of Message:  The Second step in downloading a program; transmits

the actual data to be stored in EEC RAM.

Data Flow:  From Offboard Diagnostic Unit (ODU) to EEC.

FIXED LENGTH MESSAGE

Message Specification:

scp_src_nar (ODU Node Address)     = Fxh

scp_data_1 (Block Data Transfer)   = 36h

scp_data_2 (data_byte)             = xxh

scp_data_3 (data_byte or 00h)      = xxh

scp_data_4 (data_byte or 00h)      = xxh

scp_data_5 (data_byte or 00h)      = xxh

scp_data_6 (data_byte or 00h)      = xxh

scp_data_7 (data_byte or 00h)      = xxh

Receive Message Routine (only executed upon message reception):

SCP_DWNLD_ST = dwnld_entered -----|

(State Machine at State = 1)     |OR --| SCP_DWNLD_ST = dwnld_data

|     | (Set State Machine to State 2)

SCP_DWNLD_ST = dwnld_data --------|     | (Start loading at 'start_adrs' as

(State Machine at State = 2)           |  defined in DOWNLOAD_ROUTINE_ENTRY

|  until 'total_bytes' as defined in

|  same message have been saved.)

|

| --- ELSE ---

|

SCP_DWNLD_ST = dwnld_&_execute ---------| SCP_DWNLD_ST = dwnld_&_execute

(State Machine at State = 4)           | (Set State Machine to State 4)

| (Start loading at 'start_adrs' as

|  defined in DOWNLOAD_ROUTINE_ENTRY

|  until 'total_bytes' as defined in

|  same message have been saved.)

|

| --- ELSE ---

|

| send(GENERAL_RESPONSE: 7Fh,36h,

|          scp_data_2,scp_data_3,

|                 scp_data_4,22h)

|   (conditions not correct)

5-53

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DOWNLOAD_ROUTINE_EXIT

Message Type:  Node to Node Receive.

Purpose of Message:  Third step in Download Sequence.  This message

requests that the EEC exit the download receive mode.

Data Flow:  From Offboard Diagnostic Unit (ODU).

Message Specification:

scp_src_nar (ODU Node Address)     = Fxh

scp_data_1 (Dwnld Routine Exit)    = 37h

scp_data_2 (Block Transfer Format) = xxh

Receive Message Routine (only executed upon message reception):

SCP_DWNLD_ST = dwnld_&_execute ---|

(State Machine is in State = 4)  |OR --| send(GENERAL_RESPONSE: 7Fh,37h,

|     |         scp_data_2,00h,00h,00h)

SCP_DWNLD_ST = dwnld_data --------|     |   (affirmative)

(State Machine is in State = 2)        | SCP_DWNLD_ST = execute_ready

|  (Set State Machine at State=3)

|

| --- ELSE ---

|

SCP_DWNLD_ST = dwnld_entered -----------| send(GENERAL_RESPONSE: 7Fh,37h,

(State Machine is at State = 1)        |         scp_data_2,00h,00h,00h)

|   (affirmative)

| SCP_DWNLD_ST = dwnld_not_ready

|  (Set State Machine at State=0)

|

| --- ELSE ---

|

| send(GENERAL_RESPONSE: 7Fh,37h,

|         scp_data_2,00h,00h,22h)

|   (conditions not correct)

5-54

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EXECUTE_ROUTINE

Message Type:  Node to Node Receive.

Purpose of Message:  Fourth step in the Download & Execute process,

requesting that the EEC execute a specified routine

(which is downloaded or internal to the EEC).

- OR -

Second step in the Output Test Mode sequence,

requesting that the EEC change the state of a set

of pre-defined outputs.

- OR -

Third step in the Output Substitution Mode sequence,

requesting that the EEC open up a variable to be

manipulated via OSC.  The EEC will assign the mask

value to a table which is examined each background

loop as part of the OSC security scheme.

- OR -

Request the reset of EEC KAM (Keep Alive Memory).

Data Flow:  From Offboard Diagnostic Unit (ODU).

Message Specification:

scp_src_nar (ODU Node Address)     = Fxh

scp_data_1 (Execute Diag Command)  = B1h

scp_data_2 (Hi Byte Diag Command)  = xxh

scp_data_3 (Lo Byte Diag Command)  = xxh

scp_data_4 (Optional Data Byte)    = xxh

scp_data_5 (Optional Data Byte)    = xxh

Receive Message Routine (only executed upon message reception):

unconditionally ------------------------| command = scp_data_2,scp_data_3

;assign names to message               | xqt_adrs = scp_data_4,scp_data_5

;bytes as needed in logic below        | channel = scp_data_4 - 128

|  ; Convert channel to decimal

|  ; since PCM uses 80h -> FFh only.

| mask = scp_data_5

SCP_DWNLD_ST = execute_ready -----|

(State Machine in State=3)       |AND -| send(GENERAL_RESPONSE: 7Fh,B1h,

|     |          scp_data_2,scp_data_3,

command = 8002h ------------------|     |                 scp_data_4,00h)

(execute downloaded routine at a       |   (affirmative response)

2-byte address with the normal        | DLOAD_PROG_STRT = xqt_adrs

EEC strategy running)                 | DLOAD_EXEC_KEY = command

| (Enable execution of the routine

|  as part of the NORMAL background

|  sequence - actual decision to

|  jump to the routine is detailed

|  in the SCP_BKGND_COMx logic)

|

| --- ELSE ---

(continued on next page)

5-55

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EXECUTE_ROUTINE

(continued from previous page)

SCP_DWNLD_ST = execute_ready -----|     |

(State Machine at State=3)       |     |

|AND -| send(GENERAL_RESPONSE: 7Fh,B1h,

UNDSP = 1 ------------------|     |     |          scp_data_2,scp_data_3,

|OR --|     |                 scp_data_4,00h)

CRKFLG = 1 -----------------|     |     |   (affirmative response)

|     | DLOAD_PROG_STRT = xqt_adr

command = 8102h ------------------|     | DLOAD_EXEC_KEY = command

(execute downloaded routine at        |  (enable execution of the routine

a 2-byte address which will          |   as part of the SBDS background

take control of the EEC)             |   sequence - actual decision to

|   jump to the routine is detailed

|   in the SCP_BKGND_COMx logic)

|

| --- ELSE ---

command = 0025h ------------------|     |

(Execute an OTM output change)  |     |

|     |

SCP_DIAG_ST = test_running -------|     |

(State Machine is in State=2)    |AND -| OUTPUT_MODE = scp_data_4

|     | (pass value to OTM logic)

SCP_TST_REQ = 84h ----------------|     |

(OTM test was last requested)          |

| --- ELSE ---

|

command = 0047h ------------------|     |

(Request an OSC mask enablement) |AND -| Do: transform_seed

|     |  ;assigns eec_key temp register

OSC_STATE = key_received ---------|     |  ;reference SECURITY_REQUEST msg.

(State Machine is in State=2)    |     |  ;ODU_KEY checked on each request

|     |

ODU_UP_TMR < ODU_TIMEOUT ---------|     | Do: process_osc_mask

;Test tool is present            |     |  ;allow or disallow OSC mask

|     |

Channel scp_data_2 is             |     |

found in OSC Table ---------------|     |

;OSC Table support documented in       | ; Do not store eec_key in RAM.

;the module OBDII_OSC_COMx.            |

| --- ELSE ---

|

command = 8010h ------------------|     |

(Request to execute KAM reset)   |AND -| KAMQA = 0

|     |  ; Overwrite the KAM qualification

CRKFLG = 1 -----------------------|     |  ; pattern A to force the KAM

(Engine is in crank mode)              |  ; reset

|

| send(GENERAL_RESPONSE: 7Fh,B1h,

|          scp_data_2,scp_data_3,

|                 scp_data_4,00h)

|   (affirmative response)

|

| --- ELSE ---

(continued on next page)

5-56

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EXECUTE_ROUTINE

(continued from previous page)

|

| send(GENERAL_RESPONSE: 7Fh,B1h,

|          scp_data_2,scp_data_3,

|                 scp_data_4,22h)

|  (conditions not correct)

BEGIN: process_osc_mask

Determine whether to allow/disallow mask overwrite.

(Security level must be verified.)

mask = OSC_MASK ------------------|

(ODU mask matches EEC mask)      |

|AND -| OSC_ENA[channel] = mask

eec_key <> 0 ---------------------|     | (pass value to OSC logic)

|     |

eec_key = ODU_KEY ----------------|     | send(GENERAL_RESPONSE: 7Fh,B1h,

(security level verified)              |          scp_data_2,scp_data_3

|                 scp_data_4,00h)

|   (affirmative response)

|

| --- ELSE ---

|

mask <> OSC_MASK -----------------------| send(GENERAL_RESPONSE: 7Fh,B1h,

(OSC masks do not match)               |          scp_data_2,scp_data_3

|                 scp_data_4,31h)

|   (request out of range)

|

| --- ELSE ---

|

| OSC_STATE = osc_initial_state

|  ;Illegal access determined,

|  ;Reset OSC state machine.

|

| send(GENERAL_RESPONSE: 7Fh,B1h,

|          scp_data_2,scp_data_3,

|                 scp_data_4,33h)

|  (access denied - device secured)

END: process_osc_mask

5-57

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

MODE_NOT_SUPPORTED

Message Type:  Node to Node Receive.

Purpose of Message:  Unknown.  Message requested does not correspond with

any existing EEC supported modes.

Data Flow:  From Offboard Diagnostic Unit (ODU) to EEC.

Message Specification:

scp_src_nar (ODU Node Address)     = Fxh

scp_data_1 (Any Unsupported Mode)  = xxh

scp_data_2 (Optional data byte)    = xxh

scp_data_3 (Optional data byte)    = xxh

scp_data_4 (Optional data byte)    = xxh

scp_data_5 (Optional data byte)    = xxh

scp_data_6 (Optional data byte)    = xxh

scp_data_7 (Optional data byte)    = xxh

Other Specifications:

THIS MESSAGE REPRESENTS ANY NODE TO NODE REQUEST WHICH DOES NOT MATCH

AN EXISTING EEC MODE.  THE LIST OF SUPPORTED MODES IS FOUND IN THE

OVERVIEW FOR THIS MODULE.

Receive Message Routine (only executed upon message reception):

unconditionally ------------------------| send(GENERAL_RESPONSE: 7Fh,

|      scp_data_1,scp_data_2,

|      scp_data_3,scp_data_4,11h)

5-58

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PARAMETER_SUBSTITUTION

Message Type:  Node to Node Receive.

Purpose of Message:  Allows technician  to  manipulate  direct  EEC  outputs.

This  message  requires  proper  security  access  via  the  seed/key  system

discussed in the overview section of  this  module  as  well  as  the  module

OBDII_OSC_COMx.

Data Flow:  From Offboard Diagnostic Unit (ODU) to EEC.

Message Specification:

scp_src_nar (ODU Node Address)     = Fxh

scp_data_1 (Substitute by Data ID) = 30h

scp_data_2 (OSC Channel to Subs)   = xxh

scp_data_3 (High Byte Subs Value)  = xxh

scp_data_4 (Low Byte Subs Value)   = xxh

Other Specifications:

When OSC scaling references a flag or byte, the least significant bit/byte

of the substitute word shall be used to control the channel, respectively.

Receive Message Routine (only executed upon message reception):

unconditionally --------------------------| channel = scp_data_2 - 128

;Note that scp_data_2 is offset          |  ;Convert channel to Ford range

;by 128 from OSC_TABLE list.             |  ;Ford uses 80h = 0 and FFh = 127

Channel scp_data_2 is found in OSC Table -| value = scp_data_3,scp_data_4

;OSC Table support documented in         |  ;See OBDII_OSC_COMx for scaling

;the module OBDII_OSC_COMx.              |

| --- ELSE ---

|

| send(GENERAL_RESPONSE: 7Fh,30h,

|          scp_data_2,scp_data_3,

|                 scp_data_4,31h)

|   (request out of range)

|

| EXIT routine

5-59

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OSC_STATE = key_received -----------|

(State Machine is in State=2)      |

|AND -| Do: transform_seed

|     | ;Assigns eec_key temp register

ODU_UP_TMR < ODU_TIMEOUT -----------|     | ;Reference SECURITY_REQUEST msg.

;Test tool is present                    | ;ODU_KEY checked on each request

|

| Do: process_osc_subs

| ;Allow or disallow substitution.

| ;Do not store eec_key in RAM.

|

| --- ELSE ---

|

| send(GENERAL_RESPONSE: 7Fh,30h,

|          scp_data_2,scp_data_3,

|                 scp_data_4,22h)

|   (conditions not correct)

5-60

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: process_osc_subs

Determine whether to allow/disallow output parameter overwrite.

(Security level must be verified.)

eec_key = 0 ----------------------|

|OR --| OSC_STATE = osc_initial_state

eec_key <> ODU_KEY ---------------|     |  ;Illegal access determined,

;illegal security access               |  ;Reset OSC state machine.

|

| send(GENERAL_RESPONSE: 7Fh,30h,

|          scp_data_2,scp_data_3,

|                 scp_data_4,33h)

|  (access denied - device secured)

|

| --- ELSE ---

|

if scp_data_3 is present ---------------| OSC_VALUE[channel] = value

|  ; Update substitution value.

|  ; Reference the OSC_TABLE for

|  ; scaling information on each

|  ; channel (in OBDII_OSC_COMx).

| SUBST_REQ[channel] = 1

| send(GENERAL_RESPONSE: 7Fh,30h,

|          scp_data_2,scp_data_3,

|                 scp_data_4,00h)

|    (affirmative)

|  ; This affirmative response MUST

|  ; be deleted when OSC effort for

|  ; URD 11738 is complete.

|

| --- ELSE ---

|

| Increment OSC_ENA[channel]

|  ; Disable substitution

|  ; for this channel only.

|  ; Allow value to roll to zero.

|

| send(GENERAL_RESPONSE: 7Fh,30h,

|          scp_data_2,scp_data_3,

|                 scp_data_4,00h)

|    (affirmative)

END: process_osc_subs

5-61

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

REQUEST_ALL_CODES

Message Type:  Node to Node Receive.

Purpose of Message:  Request for all of the Diagnostic Trouble Codes

which are 'set' or 'pending' in KAM.

Data Flow:  From Offboard Diagnostic Unit (ODU) to EEC.

Data Description:  If the CODES_COUNT register is greater than zero, the list

of all_codes are searched.  Any mil_codes with the FAULT bit set, or ones

with the STATE register greater than zero are sent over the SCP network.

In addition, any non_mil_codes with a FUALT bit set are also sent.

The mil_codes will be transmitted first, followed immediately by the

non_mil_codes.  Only the FINAL message can be packed with 00hex.

Message Specification:

scp_src_nar (ODU Node Address)     = Fxh

scp_data_1 (Request all codes mode)= 13h

Other Specifications:

ONLY ONE SET OF CODES ARE TO BE REQUESTED OR TRANSMITTED AT ANY ONE TIME.

Receive Message Routine (only executed upon message reception):

CODES_COUNT <> 0 -----------------------| send(REPORT_ALL_CODES)

|  (send all codes in the set of

|   mil_codes with PxxxFAULT = 1 or

|                  PxxxSTATE > 0)

|  (send all codes in the set of

|   non_mil_codes with PxxxFAULT = 1)

|

| --- ELSE ---

|

CODES_COUNT = 0 ------------------------| send(GENERAL_RESPONSE: 7Fh,13h,00h,

|                        00h,00h,62h)

|   (normal exit without results)

5-62

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

REQUEST_ALL_DATA_STOP

Message Type:  Node to Node Receive.

Purpose of Message:  Request to discontinue sending all repetitive data.

Data Flow:  From Offboard Diagnostic Unit (ODU) to EEC.

Message Specification:

scp_src_nar (ODU Node Address)     = Fxh

scp_data_1 (Request all codes mode)= 25h

Receive Message Routine (only executed upon message reception):

unconditionally ------------------------| A_RATE_ENA = 0

| send(GENERAL_RESPONSE: 7Fh,25h,00h,

|                        00h,00h,00h)

|  (affirmative)

5-63

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

REQUEST_CLEAR_CODES

Message Type:  Node to Node Receive.

Purpose of Message:  Request to clear all of the Diagnostic Trouble Codes

which are 'set' or 'pending' in KAM.

Data Flow:  From Offboard Diagnostic Unit (ODU) to EEC.

Data Description:  Upon receiving this message, the reset flag (OBDII_RESET)

is set, which is recognized and acted upon by the diagnostic executive.

Although this message allows for individual clearing of DTC's, the EEC

will execute a COMPLETE reset of ALL codes whenever any type of code

reset is requested.

The message is variable in length.

Message Specification:

scp_src_nar (ODU Node Address)     = Fxh

scp_data_1 (Request to clear codes)= 14h

scp_data_2 (High byte Code # - opt)= xxh

scp_data_3 (Low byte Code # - opt) = xxh

Receive Message Routine (only executed upon message reception):

if scp_data_2 or scp_data_3 have data --| send(GENERAL_RESPONSE: 7Fh,14h,

|      scp_data_2,scp_data_3,00h,12h)

|   (individual clearing of codes

|         is not supported)

|

| --- ELSE ---

|

| OBDII_RESET = 1

|   (Set flag to clear all codes)

| send(CONFIRM_CODES_CLEAR)

5-64

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

REQUEST_DIAG_RESULTS

Message Type:  Node to Node Receive.

Purpose of Message:  Request to report results of on-demand self test.

Data Flow:  From the ODU to the EEC.

Other Specifications:  The ODU can only request results of the on-demand test

after  completion  of that on-demand sequence (as referenced by SCP_TST_REQ).

The ODU cannot ask for results of a test which it did not execute.  When  the

EEC  receives  this  request,  it  cycles through all codes looking for 'set'

codes, indicated by a zeroed 'warm-up'  counter.   Upon  finding  some  codes

'set', the codes are transmitted via DIAGNOSTIC_RESULTS.

If the EEC is powered down, or another test is requested prior to  requesting

codes  from  a  previous  test, the initial results are erased.  Codes may be

obtained only once via this message.

ONLY ONE SET OF CODES ARE TO BE REQUESTED OR TRANSMITTED AT ANY ONE TIME.

Message Specification:

scp_src_nar (ODU Node Address)     = Fxh

scp_data_1 (Diag. Routine Results) = 33h

scp_data_2 (Specific Diag Routine) = xxh

Receive Message Routine (only executed upon message reception):

SCP_DIAG_ST = results_avail ------|

(State Machine is at State=3)    |

|

SCP_TST_REQ = scp_data_2 ---------|AND -| send(DIAGNOSTIC_RESULTS)

(Correct test requested)         |     |   (send all codes in the set of

|     |      all_codes with PxxxCNT = 0)

OD_CODE_CNT <> 0 -----------------|     | RESULTS = 0

(Codes were set for the test)          |

| --- ELSE ---

SCP_DIAG_ST = results_avail ------|     |

(State Machine is at State=3)    |     |

|     |

SCP_TST_REQ = scp_data_2 ---------|AND -| send(DIAGNOSTIC_RESULTS: 73h,00h,

(Correct test requested)         |     |              00h,00h,00h,00h,00h)

|     |    (send NULL code set)

OD_CODE_CNT = 0 ------------------|     | RESULTS = 0

(No codes were set for the test)       |

| --- ELSE ---

|

| send(GENERAL_RESPONSE: 7Fh,33h,

|         scp_data_2,00h,00h,22h)

|  (sequence error)

5-65

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

REQUEST_DMR_ACCESS

Message Type:  Node to Node Receive.

Purpose of Message:  Request EEC data to be transmitted over the SCP network

by Direct Memory Request (DMR).

Data Flow:  From ODU (ex. scan tool) to the PCM.

Data Description:  Data can ONLY be a single request/single response.

Message Specification:

scp_src_nar (ODU Node Address)     = Fxh

scp_data_1 (DMR Data Access Mode)  = 23h

scp_data_2 (High Byte Address)     = xxh

scp_data_3 (Middle Byte Address)   = xxh

scp_data_4 (Low Byte Address)      = xxh

Receive Message Routine (executed only upon reception of message):

unconditionally ------------------------| dmr_type = scp_data_2

| dmr_rqst = scp_data_3,scp_data_4

dmr_type = 01h -------------------------| send(REPORT_DMR_DATA)

|  (Send single response consisting

|   of four bytes of ROM Bank1 data

|   starting at address: dmr_rqst)

|

| --- ELSE ---

|

dmr_type = 08h -------------------------| send(REPORT_DMR_DATA)

|  (Send single response consisting

|   of four bytes of ROM Bank8 data

|   starting at address: dmr_rqst)

|

| --- ELSE ---

|

dmr_type = 00h -------------------------| send(REPORT_DMR_DATA)

|  (Send single response consisting

|   of four bytes of RAM data

|   starting at address: dmr_rqst)

|

| --- ELSE ---

|

| send(GENERAL_RESPONSE: 7Fh,23h,

|          scp_data_2,scp_data_3,

|                 scp_data_4,12h)

|   (invalid format)

5-66

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

REQUEST_MIL_CODES

Message Type:  Node to Node Receive.

Purpose of Message:  Message requesting same data as the J1979 Mode 03

message (all set mil_code DTC's).

Data Flow:  From Offboard Diagnostic Unit (ODU) to EEC.

Message Specification:

scp_src_nar (ODU Node Address)     = Fxh

scp_data_1 (Request mil_codes mode)= 03h

Other Specifications:

ONLY ONE SET OF CODES ARE TO BE REQUESTED OR TRANSMITTED AT ANY ONE TIME.

Receive Message Routine (only executed upon message reception):

NUM_CODES <> 0 -------------------------| send(REPORT_MIL_CODES)

|  (send all codes in the set of

|   mil_codes with PxxxFAULT=1)

|

| --- ELSE ---

|

NUM_CODES = 0 --------------------------| send(GENERAL_RESPONSE: 7Fh,03h,00h,

|                        00h,00h,62h)

|   (normal exit without results)

5-67

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

REQUEST_PID_ACCESS

Message Type:  Node to Node Receive.

Purpose of Message:  Request EEC data to be transmitted over the SCP network

by Parameter Identifier (PID).

Data Flow:  From ODU (ex. scan tool) to the PCM.

Data Description:  Data can ONLY be a single request/single response.

Message Specification:

scp_src_nar (ODU Node Address)     = Fxh

scp_data_1 (PID Data Access Mode)  = 22h

scp_data_2 (High Byte PID number)  = pid_hi

scp_data_3 (Low Byte PID number)   = pid_lo

Receive Message Routine (executed only upon reception of message):

unconditionally ------------------------| pid_rqst = scp_data_2,scp_data_3

pid_rqst not found in map --------------| send(GENERAL_RESPONSE: 7Fh,22h,

(bad PID number requested)           |     scp_data_2,scp_data_3,00h,12h)

|   (invalid format)

|

| --- ELSE ---

|

| send(REPORT_PID_DATA)

|  (send single response of pid_rqst

|   as defined in SCP_PID_MAP_COMx)

5-68

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

REQUEST_RAPID_PACKET

Message Type:  Node to Node Receive.

Purpose of Message:  Request EEC Rapid Data Tranfer packet data to be

transmitted over the SCP network at an optional rate.

Data Flow:  From ODU (ex. scan tool) to the PCM.

Data Description:  SEE THE CHAPTER OVERVIEW FOR GUIDELINES OF DEFINITION AND

TRANSMISSION OF DATA WITH RAPID PACKETS ON THE SCP NETWORK.

Message Specification:

scp_src_nar (ODU Node Address)     = Fxh

scp_data_1 (Packet Request Mode)   = 2Ah

scp_data_2 (Data Packet Rate)      = xxh

scp_data_3 (1st Packet Number)     = xxh

OPTIONAL >>> scp_data_4 (2nd Packet Number)     = xxh

OPTIONAL >>> scp_data_5 (3rd Packet Number)     = xxh

OPTIONAL >>> scp_data_6 (4th Packet Number)     = xxh

OPTIONAL >>> scp_data_7 (5th Packet Number)     = xxh

Response Message:  REPORT_RAPID_PACKET

Receive Message Routine (executed only upon reception of message):

unconditionally ------------------------| pkt_a = scp_data_3

| pkt_b = scp_data_4

| pkt_c = scp_data_5

| pkt_d = scp_data_6

| pkt_e = scp_data_7

5-69

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

REQUEST_RAPID_PACKET

scp_data_2 = 01h -----------------|

|

scp_data_2 = 02h -----------------|

|

scp_data_2 > 04h -----------------|

|

scp_data_3 > 0Fh -----------------|

|

scp_data_4 > 0Fh -----------------|

|

scp_data_5 > 0Fh -----------------|

|

scp_data_6 > 0Fh -----------------|OR --| send(GENERAL_RESPONSE: 7Fh,2Ah,

|     |          scp_data_2,scp_data_3,

scp_data_7 > 0Fh -----------------|     |                 scp_data_4,12h)

|     |  (invalid format)

scp_data_3 = 00h -----------------|     |

|     |

scp_data_4 = 00h -----------------|     |

|     |

scp_data_5 = 00h -----------------|     |

|     |

scp_data_6 = 00h -----------------|     |

|     |

scp_data_7 = 00h -----------------|     |

| --- ELSE ---

|

scp_data_2 >= 03h ----------------------| Enable pkt_a at Type 'A' rate

(update the 'A' queue to include      | Enable pkt_b at Type 'A' rate

these packets)                       | Enable pkt_c at Type 'A' rate

| Enable pkt_d at Type 'A' rate

| Enable pkt_e at Type 'A' rate

|   (enable the transmission of

|    each packet requested at

|    the Type 'A' data rate)

| A_RATE_ENA = 1

|

| --- ELSE ---

|

scp_data_2 = 00h -----------------------| Disable the transmission of pkt_a

| Disable the transmission of pkt_b

| Disable the transmission of pkt_c

| Disable the transmission of pkt_d

| Disable the transmission of pkt_e

|   (disable the transmission of

|    each packet requested)

5-70

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SECURITY_REQUEST

Message Type:  Node to Node Receive.

Purpose of Message:  To restrict access and  prevent  unauthorized  intrusion

into  the  EEC.   The  security access method described in this strategy book

satisfies the requirements for tamper protection specified in SAE J2186.  The

process  is  split into two steps:  1) Request and obtain an algorithm 'seed'

value, and 2) Submit processed 'key' code in order to obtain security access.

Data Flow:  From ODU (ex. scan tool) to the PCM.

Message Specification:

scp_src_nar (ODU Node Address)       = Fxh

scp_data_1 (Security Access Request) = 27h

scp_data_2 (Seed/Key Indicator)      = xxh (01h or 02h)

scp_data_3 (High byte KEY)           = xxh (If scp_data_2=02h)

scp_data_4 (Low byte KEY)            = xxh (If scp_data_2=02h)

Receive Message Routine (only executed upon message reception):

OSC_STATE = osc_initial_state ----|

(Not in the substitute state)    |

(State Machine is in State=0)    |

|AND -| send(GENERAL_RESPONSE: 7Fh,27h,

scp_data_2 = 01h -----------------|     |          scp_data_2,scp_data_3,

(Request for Access Seed value)  |     |                 scp_data_4,21h)

|     |   ;Busy - repeat request

All bytes of 'clock' are zero ----|     |

;OSC_SEED cannot be equal to zero      | ODU_UP_TMR = 0

|  ;Accept that tester is 'present'

|

| EXIT this routine.

|

| --- ELSE ---

OSC_STATE = osc_initial_state ----|     |

(State Machine is in State=0)    |AND -| OSC_SEED = (least significant,

(Not in the substitute state)    |     |        non-zero byte of 'clock')

|     |  ;Use as random number for OSC_SEED

scp_data_2 = 01h -----------------|     |

(Request for Access Seed value)        | OSC_STATE = osc_seed_sent

|  ;Set State Machine to State=1

|

| ODU_UP_TMR = 0

|  ;Accept that tester is 'present'

|

| send(SECURITY_RESPONSE: 67h,01h,

|                    00h,OSC_SEED)

|  ;Provide seed for ODU algorithm

|

| --- ELSE ---

(continued on next page)

5-71

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SECURITY_REQUEST

(continued from previous page)

OSC_STATE = osc_seed_sent --------|     |

(State Machine is in State=1)    |AND -| ODU_KEY = scp_data_3,scp_data_4

(Just supplied new Seed value)   |     |  ;Assign KEY word from ODU.

|     |

scp_data_2 = 02h -----------------|     | Do: transform_seed

(Submit Security Key value)            |  ;Assigns eec_key temp register.

|

| Do: check_key

|  ;Allow/disallow security access.

|  ;Do not store eec_key in RAM.

|

| --- ELSE ---

|

| send(GENERAL_RESPONSE: 7Fh,27h,

|          scp_data_2,scp_data_3,

|                 scp_data_4,22h)

|  ;Sequence error

BEGIN: transform_seed

;PRIVATE process used to transform the seed value to produce the key

;to unlock the output substitution control.  The same process must be

;installed in the ODU to provide the key.

;The transform function and algorithm are shown below:

|

65535 +                                    *

|                                   *

|                                   *

|                                   *

k      |                                  *

e      |                                  *

y      |                                 *

|              *                  *

|          *       *             *

|      *               *        *

|   *                     *    *

| *                         * *

0 *----------------------------*--------+--------->

0                                    255     seed

unconditionally --------| eec_key := abs(2*OSC_SEED**2 - 253*OSC_SEED)

END: transform_seed

5-72

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SECURITY_REQUEST

BEGIN: check_key

PRIVATE process to determine if ODU key matches EEC calculated key.

eec_key <> 0 ---------------|

|AND -| OSC_STATE = key_received

eec_key = ODU_KEY ----------|     |  ; substitution allowed

; valid KEY was received         |

| send(GENERAL_RESPONSE: 7Fh, 27h,

|          scp_data_2, scp_data_3,

|                  scp_data_4,34h)

|  ; access allowed

|

| ODU_UP_TMR = 0

|  ; Accept that tester is 'present'

|

| --- ELSE ---

|

| OSC_STATE = osc_initial_state

|  ; Reset state machine

|

| send(GENERAL_RESPONSE: 7Fh, 27h,

|          scp_data_2, scp_data_3,

|                  scp_data_4,33h)

|  ; access denied - device secured

END: check_key

5-73

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TESTER_PRESENT

Message Type:  Node to Node Receive.

Purpose of Message:  Heartbeat message indicating that ODU is currently

on-line and communicating with the EEC.

If the EEC has been put in an 'unlocked' state

(e.g. parameter substitution), then this message

is used to prompt a check of the validity of that

state.  If the state is invalid, then reset the

security access state machine.

Data Flow:  From Offboard Diagnostic Unit (ODU) to EEC.

Message Specification:

scp_src_nar (ODU Node Address)     = Fxh

scp_data_1 (Tester Present code)   = 3Fh

Receive Message Routine (only executed upon message reception):

unconditionally ------------------------| ODU_UP_TMR = 0

|   (Reset ODU timeout counter)

| ODU_UP_STATE = 1

| send(GENERAL_RESPONSE: 7Fh, 3Fh,

|           00h, 00h, 00h, 00h)

|   (affirmative)

OSC_STATE = key_received ---------------| Do: transform_seed

; if currently unlocked...             | ; Ensure access is legal.

(State Machine is in State=2)          | ; The 'transform' process located

| ; in SECURITY_REQUEST algorithm.

| ; Process returns 'eec_key'.

|

| Do: check_legal_access

| ;Do not store eec_key in RAM.

BEGIN: check_legal_access

PRIVATE process to ensure that security access has not been back-doored.

eec_key <> ODU_KEY ---------------|

|OR --| OSC_STATE = osc_initial_state

ODU_KEY = 0 ----------------------|     | ; Reset State Machine.

| ; Do not allow security access.

END: check_legal_access

5-74

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

REPORT_OBDII_CODES

Message Type:  Broadcast Transmit.

Purpose of Message:  This message is sent in response to receipt of scan

tool message PROCESS_OBDII_REQUEST, with a Mode 03 request.

Data Flow:  From PCM to SCP Network.

THIS MESSAGE MUST BE SENT ON THE SCP NETWORK WITHIN 100 MSEC OF

RECEIVING THE "PROCESS_OBDII_REQUEST" MESSAGE.

USE OF THE 'list' CONCEPT IS MERELY FOR DOCUMENTATION

PURPOSES AND IN NO WAY IMPLIES AN IMPLEMENTATION.

Message Specification:

scp_pri_type                       = 41h

scp_target    (Target Specifier)   = 6Bh

scp_data_1 (Report OBDII DTC's)    = 43h

scp_data_2 (high byte code 1)      = list_of_codes

scp_data_3 (low byte code 1)       = list_of_codes

scp_data_4 (high byte code 2 or 00)= list_of_codes

scp_data_5 (low byte code 2 or 00) = list_of_codes

scp_data_6 (high byte code 3 or 00)= list_of_codes

scp_data_7 (low byte code 3 or 00) = list_of_codes

Other Specifications:

Data Format:

list_of_codes :  (code_hi = hi byte of 1st code

code_lo = lo byte of 1st code

code_hi = hi byte of 2nd code (or 00h)

code_lo = lo byte of 2nd code (or 00h)

:

:

code_hi = hi byte of n'th code (or 00h)

code_lo = lo byte of n'th code (or 00h))

The 4 digit codes are sent in BCD format.  They occupy 2 data bytes

each, with the upper nibble of the upper byte encoded to indicate either

legislated codes, or Ford specific codes as specified in SAE spec J1979.

The codes are packed 3 to a message, with the hi byte first.

If there are less than 3 codes, the message is filled with zeros so

that a fixed length message is transmitted (data bytes 2-7 filled).

Requesting Message:  PROCESS_OBDII_REQUEST

5-75

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

REPORT_OBDII_CONT_RESULTS

Message Type:  Broadcast Transmit.

Purpose of Message:  This message is sent in response to receipt of scan

tool message PROCESS_OBDII_REQUEST, with a Mode 07 request.

Data Flow:  From PCM to SCP Network.

THIS MESSAGE MUST BE SENT ON THE SCP NETWORK WITHIN 100 MSEC OF

RECEIVING THE "PROCESS_OBDII_REQUEST" MESSAGE.

USE OF THE 'list' CONCEPT IS MERELY FOR DOCUMENTATION

PURPOSES AND IN NO WAY IMPLIES AN IMPLEMENTATION.

Message Specification:

The requirement for codes to be sent in response to mode 07 is

the state to be greater than zero, and the fault flag equal zero, ie.

send all codes in the set of mil_codes with PxxxSTATE>0 and PxxxFAULT=0

scp_pri_type                       = 41h

scp_target    (Target Specifier)   = 6Bh

scp_data_1 (Report OBDII DTC's)    = 47h

scp_data_2 (high byte code 1)      = list_of_codes

scp_data_3 (low byte code 1)       = list_of_codes

scp_data_4 (high byte code 2 or 00)= list_of_codes

scp_data_5 (low byte code 2 or 00) = list_of_codes

scp_data_6 (high byte code 3 or 00)= list_of_codes

scp_data_7 (low byte code 3 or 00) = list_of_codes

Other Specifications:

Data Format:

list_of_codes :  (code_hi = hi byte of 1st code

code_lo = lo byte of 1st code

code_hi = hi byte of 2nd code (or 00h)

code_lo = lo byte of 2nd code (or 00h)

:

:

code_hi = hi byte of n'th code (or 00h)

code_lo = lo byte of n'th code (or 00h))

The 4 digit codes are sent in BCD format.  They occupy 2 data bytes

each, with the upper nibble of the upper byte encoded to indicate either

legislated codes, or Ford specific codes as specified in SAE spec J1979.

The codes are packed 3 to a message, with the hi byte first.

If there are less than 3 codes, the message is filled with zeros so

that a fixed length message is transmitted (data bytes 2-7 filled).

Requesting Message:  PROCESS_OBDII_REQUEST

5-76

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

REPORT_OBDII_FF_PID

Message Type:  Broadcast Transmit.

Purpose of Message:  This message is sent in response to receipt of scan

tool message PROCESS_OBDII_REQUEST, with a Mode 02 request.

Data Flow:  From PCM to SCP Network.

THIS MESSAGE MUST BE SENT ON THE SCP NETWORK WITHIN 100 MSEC OF

RECEIVING THE "PROCESS_OBDII_REQUEST" MESSAGE.

Data Description:  The EEC will transmit the appropriate Freeze Frame PID

data, as defined in the SAE specification J1979.  EXAMPLE:  An OBDII

FF PID request of the trouble code which cause the CARB required freeze

frame data storage (J1979 Mode 02, PID 02) will prompt the response of

the following:

A priority/type byte which complies with the J1979 spec (41h); A GENERIC

function code identifying the message as a J1979 Response (6Bh);

A SPECIFIC function code identifying the message as a PID response (42h);

The PID number which was requested via Scan Tool (02h); the freeze frame

number associated with the trouble code (which will be 00h, by default);

and the two byte trouble code which resides in pid_def(j1979_02_02).

The J1979 PID values and scaling are described with "pid_def()"

constructs throughout the strategy.

The SCP Freeze Frame PID table is contained in the module:

SCP_PID_MAP_COMx.

Message Specification:

scp_pri_type                       = 41h

scp_target   (J1979 response byte) = 6Bh

scp_data_1 (PID Report Funct Code) = 42h

scp_data_2 (PID number returned)   = ff_pid_lo

scp_data_3 (J1979 frame number)    = 00h

scp_data_4 (J1979 PID data)        = yyh

scp_data_5 (optional PID data)     = yyh

scp_data_6 (optional PID data)     = yyh

scp_data_7 (optional PID data)     = yyh

Where:

ff_pid_lo = FF PID number (scp_data_2 from PROCESS_OBDII_REQUEST).

yyh = Freeze Frame PID data per SAE specification J1979.

Other Specifications:

IF FF PID IS NOT SUPPORTED in SCP_PID_MAP_COMx, DO NOT SEND RESPONSE.

Requesting Message:  PROCESS_OBDII_REQUEST

5-77

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

REPORT_OBDII_TRIP_RESULTS

Message Type:  Broadcast Transmit.

Purpose of Message:  This message is sent in response to receipt of

scan tool message PROCESS_OBDII_REQUEST, with a Mode 06 request.

It provides on-board monitoring test results for non-continuously

monitored systems.

The Response Table in SCP_MODE06_MAP_xx details exactly what data

should be used for each data byte in the response message(s), for

a given Test ID.

Data Flow:  From PCM to SCP Network.

THIS MESSAGE MUST BE SENT ON THE SCP NETWORK WITHIN 100 MSEC OF

RECEIVING THE "PROCESS_OBDII_REQUEST" MESSAGE.

Message Specification:

scp_pri_type                 = 41h

scp_target (J1979 Targ Spec) = 6Bh

scp_data_1 (Data Mode)       = 46h

scp_data_2 (Returned scp_data_2) = test_id

scp_data_3 (Returned scp_data_3) = FFh or Test Limit Type

and Component ID

scp_data_4                       = test support bitmap 1 or

test value MSB

scp_data_5                       = test support bitmap 2 or

test value LSB

scp_data_6                       = test support bitmap 3 or

test result MSB

scp_data_7                       = test support bitmap 4 or

test result LSB

Other Specifications:

IF TEST ID IS NOT SUPPORTED, DO NOT SEND RESPONSE.

Requesting Message:  PROCESS_OBDII_REQUEST

5-78

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

REPORT_OBDII_PID

Message Type:  Broadcast Transmit.

Purpose of Message:  This message is sent in response to receipt of scan

tool message PROCESS_OBDII_REQUEST, with a Mode 01 request.

Data Flow:  From PCM to SCP Network.

THIS MESSAGE MUST BE SENT ON THE SCP NETWORK WITHIN 100 MSEC OF

RECEIVING THE "PROCESS_OBDII_REQUEST" MESSAGE.

Data Description:  The EEC will transmit the appropriate PID data, as defined

in the SAE specification J1979.  EXAMPLE:  An OBDII PID request of MIL

status (J1979 Mode 01, PID 01) will prompt the response of the following:

A priority/type byte which complies with the J1979 spec (41h); A GENERIC

function code identifying the message as a J1979 Response (6Bh);

A SPECIFIC function code identifying the message as a PID response (41h);

The PID number which was requested via Scan Tool (01h), and; four bytes

of information, as described in the send message command, which are:

j1979_01_011, j1979_01_012, j1979_01_013 and j1979_01_014.  The J1979

PID values and scaling are described with "pid_def()" constructs

throughout the strategy.

The SCP PID table is contained in the module:  SCP_PID_MAP_COMx.

Message Specification:

scp_pri_type                       = 41h

scp_target   (J1979 response byte) = 6Bh

scp_data_1 (PID Report Funct Code) = 41h

scp_data_2 (PID number returned)   = pid_lo

scp_data_3 (J1979 data)            = yyh

scp_data_4 (optional PID data)     = yyh

scp_data_5 (optional PID data)     = yyh

scp_data_6 (optional PID data)     = yyh

Where:

pid_lo = PID number (scp_data_2 from PROCESS_OBDII_REQUEST).

yyh = PID data per SAE specification J1979.

Other Specifications:

IF PID IS NOT SUPPORTED in SCP_PID_MAP_COMx, DO NOT SEND RESPONSE.

Requesting Message:  PROCESS_OBDII_REQUEST

5-79

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

REPORT_OBDII_RESET

Message Type:  Broadcast Transmit.

Purpose of Message:  This message is sent in response to receipt of

scan tool message PROCESS_OBDII_REQUEST, with a Mode 04 request.

Data Flow:  From PCM to SCP Network.

THIS MESSAGE MUST BE SENT ON THE SCP NETWORK WITHIN 100 MSEC OF

RECEIVING THE "PROCESS_OBDII_REQUEST" MESSAGE.

Message Specification:

scp_pri_type                       = 41h

scp_target    (Target Specifier)   = 6Bh

scp_data_1 (Code for OBDII Reset)  = 44h

Other Specifications:

Requesting Message:  PROCESS_OBDII_REQUEST

5-80

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONFIRM_CODES_CLEAR

Message Type:  Node to Node Transmit.

Purpose of Message:  This message is sent in response to the request message

REQUEST_CLEAR_CODES to indicate that a code reset has

been completed by the EEC.

Data Flow:  From EEC to ODU.  The scp_target address, is defined

by the scp_src_nar byte contained in the requesting message.

Message Specification:

scp_pri_type                       = C4h

scp_target  (ODU Node Address)     = Fxh

scp_data_1 (Confirm codes cleared) = 54h

Requesting Message:  REQUEST_CLEAR_CODES

5-81

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DIAGNOSTIC_RESULTS

Message Type:  Node to Node Transmit.

Purpose of Message:  This message is sent in response to receipt of the

message REQUEST_DIAG_RESULTS.  It is used to report any codes which may

have been set during the most recent On-Demand test sequence.

Data Flow:  From EEC to ODU.  The scp_target address, is defined

by the scp_src_nar byte contained in the requesting message.

USE OF THE 'list' CONCEPT IS MERELY FOR DOCUMENTATION

PURPOSES AND IN NO WAY IMPLIES AN IMPLEMENTATION.

Message Specification:

scp_pri_type                       = C4h

scp_target (ODU Node Address)      = Fxh

scp_data_1 (Report stored codes)   = 73h

scp_data_2 (high byte code 1)      = list_of_codes

scp_data_3 (low byte code 1)       = list_of_codes

scp_data_4 (high byte code 2 or 00)= list_of_codes

scp_data_5 (low byte code 2 or 00) = list_of_codes

scp_data_6 (high byte code 3 or 00)= list_of_codes

scp_data_7 (low byte code 3 or 00) = list_of_codes

Other Specifications:

Data Format:

list_of_codes :  (code_hi = hi byte of 1st code

code_lo = lo byte of 1st code

code_hi = hi byte of 2nd code (or 00h)

code_lo = lo byte of 2nd code (or 00h)

:

:

code_hi = hi byte of n'th code (or 00h)

code_lo = lo byte of n'th code (or 00h))

The 4 digit codes are sent in BCD format.  They occupy 2 data bytes

each, with the upper nibble of the upper byte encoded to indicate either

legislated codes, or Ford specific codes as specified in SAE spec J1979.

The codes are packed 3 to a message, with the hi byte first.

If there are less than 3 codes, the message is filled with zeros so

that a fixed length message is transmitted (data bytes 2-7 filled).

Requesting Message:  REQUEST_DIAG_RESULTS

5-82

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

GENERAL_RESPONSE

Message Type:  Node to Node Transmit.

Purpose of Message:  This Node to Node message is sent in response to many

Node to Node requests to indicate various types of EEC response.

Data Flow:  From EEC to ODU.  The scp_target address, is defined

by the scp_src_nar byte contained in the requesting message.

Data Description:  It is suggested that the EEC maintain the requesting

message data (scp_data_1 to scp_data_4, where applicable) for

possible use with this message as shown below.  In most instances,

however,the strategy which sends this message will specify the values

to load into the GENERAL_RESPONSE message scp_data_2 to scp_data_6.

Message Specification:

scp_pri_type                       = C4h

scp_target (ODU Node Address)      = Fxh

scp_data_1 (Generic Response mode) = 7Fh

scp_data_2 (Mode of request msg.)  = xxh

scp_data_3 (Argument #1 of request)= xxh (or 00h)

scp_data_4 (Argument #2 of request)= xxh (or 00h)

scp_data_5 (Argument #3 of request)= xxh (or 00h)

scp_data_6 (General Response Code) = xxh

Other Specifications:

There are 255 possible values of scp_data_6.  Some of these are shown here:

00h  General Affimative

10h  General Reject

11h  Mode not Supported

12h  Invalid Format - Sub-funtion not supported

21h  Busy - Repeat request

22h  Conditions not correct - Sequence error

23h  Routine not complete

31h  Request out of range

33h  Access denied - device secured

61h  Normal Exit with results

62h  Normal Exit without results

63h  Abnormal Exit with results

64h  Abnormal Exit without results

5-83

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

REPORT_ALL_CODES

Message Type:  Node to Node Transmit.

Purpose of Message:  This message is sent in response to receipt of the

message REQUEST_ALL_CODES.  It is used to report ALL Pending and Set

mil_codes and all no_mil_codes set in EEC KAM.

Data Flow:  From EEC to ODU.  The scp_target address, is defined

by the scp_src_nar byte contained in the requesting message.

USE OF THE 'list' CONCEPT IS MERELY FOR DOCUMENTATION

PURPOSES AND IN NO WAY IMPLIES AN IMPLEMENTATION.

Message Specification:

scp_pri_type                       = C4h

scp_target (ODU Node Address)      = Fxh

scp_data_1 (Report stored codes)   = 53h

scp_data_2 (high byte code 1)      = list_of_codes

scp_data_3 (low byte code 1)       = list_of_codes

scp_data_4 (high byte code 2 or 00)= list_of_codes

scp_data_5 (low byte code 2 or 00) = list_of_codes

scp_data_6 (high byte code 3 or 00)= list_of_codes

scp_data_7 (low byte code 3 or 00) = list_of_codes

Other Specifications:

Data Format:

list_of_codes :  (code_hi = hi byte of 1st code

code_lo = lo byte of 1st code

code_hi = hi byte of 2nd code (or 00h)

code_lo = lo byte of 2nd code (or 00h)

:

:

code_hi = hi byte of n'th code (or 00h)

code_lo = lo byte of n'th code (or 00h))

The 4 digit codes are sent in BCD format.  They occupy 2 data bytes

each, with the upper nibble of the upper byte encoded to indicate either

legislated codes, or Ford specific codes as specified in SAE spec J1979.

The codes are packed 3 to a message, with the hi byte first.

If there are less than 3 codes, the message is filled with zeros so

that a fixed length message is transmitted (data bytes 2-7 filled).

Requesting Message:  REQUEST_ALL_CODES

5-84

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

REPORT_DMR_DATA

Message Type:  Node to Node Transmit.

Purpose of Message:  This message is sent in response to receipt of the

message REQUEST_DMR_ACCESS.

Data Flow:  From EEC to ODU.  The scp_target address, is defined

by the scp_src_nar byte contained in the requesting message.

Data Description:  The EEC will transmit the appropriate DMR data, plus three

bytes of contiguous addresses following the requested address.

EXAMPLE:  REQUEST_DMR_ACCESS is received with the following byte string

from scp_data_2 on: 00h,0Bh,10h.  An upper byte of 00h, indicates a

request for RAM data (a request of 01h or 08h would be for ROM data).

And since the scp_data_6 byte was not included, the request is for a

single response only.  The following response is sent:

A priority/type byte (C4h); the ODU address which issued the request

(Fxh); the SCP-FORD specificied response mode number in scp_data_1

(63h); the Middle Byte of the referenced DMR in scp_data_2 (xxh); the

Low Byte of the referenced DMR in scp_data_3 (xxh), and; the

corresponding DMR values in RAM registers 0B10, 0B11, 0B12, & 0B13,

in scp_data_4 through scp_data_7.  The message is ALWAYS a fixed length.

Message Specification:

scp_pri_type                       = C4h

scp_target (ODU Node Address)      = Fxh

scp_data_1 (Report DMR data mode)  = 63h

scp_data_2 (Returned DMR mid addrs)= dmr_mid

scp_data_3 (Returned DMR low addrs)= dmr_lo

scp_data_4 (dmr_rqst address data) = yyh

scp_data_5 (dmr_rqst+1 addrs data) = yyh

scp_data_6 (dmr_rqst+2 addrs data) = yyh

scp_data_7 (dmr_rqst+3 addrs data) = yyh

Where:

dmr_rqst = Direct Memory address requested (dmr_mid,dmr_lo)

dmr_mid = DMR middle byte (scp_data_3 from requesting msg).

dmr_lo = DMR low byte (scp_data_4 from requesting msg).

yyh = Contiguous RAM data following address dmr_rqst.

Requesting Message:  REQUEST_DMR_ACCESS

5-85

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

REPORT_MIL_CODES

Message Type:  Node to Node Transmit.

Purpose of Message:  This message is sent in response to receipt of the

message REQUEST_MIL_CODES.

Data Flow:  From EEC to ODU.  The scp_target address, is defined

by the scp_src_nar byte contained in the requesting message.

USE OF THE 'list' CONCEPT IS MERELY FOR DOCUMENTATION

PURPOSES AND IN NO WAY IMPLIES AN IMPLEMENTATION.

Message Specification:

scp_pri_type                       = C4h

scp_target (ODU Node Address)      = Fxh

scp_data_1 (Report OBDII codes)    = 43h

scp_data_2 (high byte code 1)      = list_of_codes

scp_data_3 (low byte code 1)       = list_of_codes

scp_data_4 (high byte code 2 or 00)= list_of_codes

scp_data_5 (low byte code 2 or 00) = list_of_codes

scp_data_6 (high byte code 3 or 00)= list_of_codes

scp_data_7 (low byte code 3 or 00) = list_of_codes

Other Specifications:

Data Format:

list_of_codes :  (code_hi = hi byte of 1st code

code_lo = lo byte of 1st code

code_hi = hi byte of 2nd code (or 00h)

code_lo = lo byte of 2nd code (or 00h)

:

:

code_hi = hi byte of n'th code (or 00h)

code_lo = lo byte of n'th code (or 00h))

The 4 digit codes are sent in BCD format.  They occupy 2 data bytes

each, with the upper nibble of the upper byte encoded to indicate either

legislated codes, or Ford specific codes as specified in SAE spec J1979.

The codes are packed 3 to a message, with the hi byte first.

If there are less than 3 codes, the message is filled with zeros so

that a fixed length message is transmitted (data bytes 2-7 filled).

Requesting Message:  REQUEST_MIL_CODES

5-86

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

REPORT_PID_DATA

Message Type:  Node to Node Transmit.

Purpose of Message:  This message is sent in response to receipt of the

message REQUEST_PID_ACCESS.

Data Flow:  From EEC to ODU.  The scp_target address, is defined

by the scp_src_nar byte contained in the requesting message.

Data Description:  The EEC will transmit the appropriate PID data, as defined

in the module SCP_PID_MAP_COMx.

Message Specification:

scp_pri_type                       = C4h

scp_target (ODU Node Address)      = Fxh

scp_data_1 (Report PID data mode)  = 62h

scp_data_2 (Returned PID number)   = pid_hi

scp_data_3 (Returned PID number)   = pid_lo

scp_data_4 (PID data)              = yyh

scp_data_5 (optional PID data)     = yyh

scp_data_6 (optional PID data)     = yyh

scp_data_7 (optional PID data)     = yyh

Where:

pid_hi = PID number high byte (scp_data_2 from requesting msg).

pid_lo = PID number low byte (scp_data_3 from requesting msg).

yyh = Data per the PID table in SCP_PID_MAP_COMx.

Other Specifications:

IF PID is NOT SUPPORTED in PID MAP, send GENERAL_RESPONSE message as

defined in the requesting message.  The data will always be of the

forms:  byte, word, or double-word.  They can point to ROM or RAM data.

Requesting Message:  REQUEST_PID_ACCESS

5-87

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

REPORT_RAPID_PACKET

Message Type:  Node to Node Transmit.

Purpose of Message:  This message is sent in response to receipt of the

message REQUEST_RAPID_PACKET.

Data Flow:  From EEC to ODU.  The scp_target address, is defined

by the scp_src_nar byte contained in the requesting message.

Data Description:  The EEC will transmit the appropriate packet's data, as

defined dynamically through the message DEFINE_RAPID_PACKET and

initiated via REQUEST_RAPID_PACKET.  Only the Type 'A' rate is supported.

To manage which packets are actually enabled for transmission, a transmit

"queue" is symbolically referenced which maintains the status of each

rapid packet message.  A packet can also be removed from the "queue" by

using the REQUEST_RAPID_PACKET message (with a 00h in scp_data_2).

USE OF THE queue CONCEPT IS MERELY FOR DOCUMENTATION

PURPOSES AND IN NO WAY IMPLIES AN IMPLEMENTATION.

Message Specification:

scp_pri_type                       = C4h

scp_target (ODU Node Address)      = Fxh

scp_data_1 (Report Packet mode)    = 6Ah

scp_data_2 (Packet Number 01..0Fh) = pkt_num

scp_data_3 (1st Byte of pkt data)  = yyh

scp_data_4 (2nd Byte of pkt data)  = yyh

scp_data_5 (3rd Byte of pkt data)  = yyh

scp_data_6 (4th Byte of pkt data)  = yyh

scp_data_7 (5th Byte of pkt data)  = yyh

Other Specifications:

NOTE THAT THE J1979 PIDS ARE NOT INTENDED FOR SERVICE BY

ANY REPETITIVE DATA TRANSMISSION SPECIFICATION.

PACKETED MESSAGES ARE ALWAYS A FIXED LENGTH OF SEVEN DATA BYTES.

Any undefined packet (or portion of a packet) is to be zero filled.

Requesting Message:  REQUEST_RAPID_PACKET

5-88

SCP INTERFACES, OBDII DIAGNOSTIC MESSAGE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SECURITY_RESPONSE

Message Type:  Node to Node Transmit.

Purpose of Message:  This Node to Node message is sent in response to the

Node to Node request for access to secured routines within the EEC.

Data Description:

The second data byte contains an identifier which  informs  the  ODU  how  to

interperet  the  remaining  message  contents.   A  01h  indicates the EEC is

responding with the OSC_SEED value, as requested by the ODU.  Scp_data_3  and

scp_data_4  contain  the  high/low  bytes  of  the  security access algorithm

OSC_SEED value (NOTE that since the EEC's  algorithm  requires  only  a  byte

input,  scp_data_3  will  be  packed  with 00h).  A "send(SECURITY_RESPONSE)"

message command is used in this case.

If scp_data_2 from the SECURITY_REQUEST message is 02h, the  strategy  issues

the  "send(GENERAL_RESPONSE)"  message  command.  A 02h indicates the EEC has

processed the ODU's KEY value and made a decision to allow  or  disallow  the

requested security access.

SECURITY_RESPONSE IS A FIXED LENGTH MESSAGE.

Data Flow:  From EEC to ODU.  The scp_target address, is defined

by the scp_src_nar byte contained in the requesting message.

Message Specification:

scp_pri_type                       = C4h

scp_target (ODU Node Address)      = Fxh

scp_data_1 (Security Response mode)= 67h

scp_data_2 (Response Type Byte)    = 01h

scp_data_3 (Variable data byte)    = xxh

scp_data_4 (Variable data byte)    = xxh

5-89

SCP INTERFACES, OUTPUT STATE CONTROL RESPONSE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

5.1.8  SCP OUTPUT STATE CONTROL MESSAGE RESPONSE HANDLING (CDAM0)

OVERVIEW

OSC_RESPONSE  is  responsible  for  sending  Output  State  Control   related

GENERAL_RESPONSE  messages  to the SCP network.  This routine accepts the OSC

channel number and the hexadecimal response code as arguments, and uses  them

to  look  up  the  appropriate  data to send with the GENERAL_RESPONSE.  This

routine  is  designed  to  generically  handle  all  SCP  general   responses

associated with OSC that were sent from non-SCP modules.

DEFINITIONS

Registers:

-  OSC_VALUEnn = Output State Control substitution value for channel nn.

-  SUBST_REQnn = Substitution Requested flag for channel nn.

5-90

SCP INTERFACES, OUTPUT STATE CONTROL RESPONSE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SCP_OSC_RESPONSE_COM1

BEGIN: OSC_RESPONSE( channel, resp_code )

channel is word type -----------| send (GENERAL_RESPONSE: 7Fh, 30h, channel,

|       high byte of OSC_VALUE[channel],

|       low byte of  OSC_VALUE[channel],

|       resp_code)

|  ; OSC_VALUE is a word

|

| --- ELSE ---

|

channel is byte type -----------| send (GENERAL_RESPONSE: 7Fh, 30h, channel,

|       00h, OSC_VALUE[channel], resp_code)

|  ; OSC_VALUE is a byte

|

| --- ELSE ---

|

channel is bit type ----|       |

|AND ---| send (GENERAL_RESPONSE: 7Fh, 30h, channel,

OSC_VALUE[channel] = 0 -|       |       00h, 00h, resp_code)

|  ; OSC_VALUE is a bit

|

| --- ELSE ---

|

channel is bit type ----|       |

|AND ---| send (GENERAL_RESPONSE: 7Fh, 30h, channel,

OSC_VALUE[channel] = 1 -|       |       00h, 01h, resp_code)

|  ; OSC_VALUE is a bit

unconditionally ----------------| SUBST_REQ[channel] = 0

|  ; Clear flag that indicates a new

|  ; substitution has been requested for

|  ; the selected channel

END: OSC_RESPONSE

5-91

SCP INTERFACES, RECEIVE STATUS REQUEST MESSAGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

5.1.9  RX_PT_STAT_RQST MESSAGE (CDAN0)

OVERVIEW

This SCP module is used by various modules in the multiplexed  data  bus  for

requesting  Powertrain-specific information by transmitting the corresponding

Status Request message.

There are three types of status requests available in the SCP network  (Body,

Chassis,  and  Powertrain).   The  PCM  is expected to receive and respond to

Powertrain-specific status request messages alone.

The data sent in the message is the actual target specifier of  the  required

SCP  message (accompanied by any secondary specifier as necessary).  The node

which recognizes this message must  examine  its  list  of  transmit  message

target  specifier (/secondary ID) bytes to determine if it is responsible for

transmitting the requested information.  If a match occurs,  the  message  is

transmitted by that node and the requesting node receives the data.

DEFINITIONS

INPUTS

Registers:

-  scp_data_n = Data byte in SCP message (n = 1 through 7).

-  SCP_ECT_TMR = Free running timer used to record time elapsed between  two

consecutive ECT_DEG_C messages

OUTPUTS

Registers:

-  SEND_ECT = Flag to enable the conditional transmission of the  ECT  (deg.

C) as a result of a Status Request message.

========================================================================

Message Representation:

TARG  SOURCE

PRI/TYP  SPEC   NAR    DATA1  DATA2

-------  ----  ------  -----  -----

61h    54h    ??h     var    var

CONTROL

RECEIVE CONDITIONS              FLAGS

-----------------------------  -------

Unknown.                        none

5-92

SCP INTERFACES, RECEIVE STATUS REQUEST MESSAGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

========================================================================

5-93

SCP INTERFACES, RECEIVE STATUS REQUEST MESSAGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SCP_RX_PT_STAT_RQST_COM3

RX_PT_STAT_RQST

Message Type:  Broadcast Receive.

Data Flow:  From SCP network to EEEC.

Receive Buffer Data Bytes:

Sending Node Address                 = ??h (not used in strategy)

scp_data_1 (Primary ID of req'd msg) = xxh (mandatory)

scp_data_2 (Sec ID of req'd msg)     = xxh (or 00h as required)

Receive Message Execution (only when this message is received):

scp_data_1 = 49h -----------------|

|AND -| send(ECT_DEG_C)

scp_data_2 = 10h -----------------|     | SCP_ECT_TMR := 0

|  reset the free-running tmr

| SEND_ECT := 1

| ; enable periodic transmit

5-94

SCP INTERFACES, ECT_DEG_C MESSAGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

5.1.10  ECT_DEG_C MESSAGE (CDAM0)

OVERVIEW

This SCP message sends the Engine Coolant Temperature (ECT) from the EEEC  to

the Electronic Automatic Temperature Control (EATC) or to any other node with

the same functional address.  The message is sent per the conditions detailed

in  the  SCP_MSG_CONTROL_xx_COMx  module  (where  xx is the specific strategy

utilizing this message).  The data carried by  this  message  must  meet  SAE

standard content requirements.

DEFINITIONS

Registers:

-  ECT = Engine Coolant Temperature in Degrees F.

Bit Flags:

-  CFMFLG = ECT failure mode flag.  1 -> failure mode set.

Other

-  scp_data_n = Data byte in SCP message (n = 1 through 7).

-  scp_gen_func = SCP message primary identifier for functional messages.

-  scp_pri_type = SCP message Priority/Type byte.

========================================================================

Message Representation:

TARG  SOURCE

PRI/TYP  SPEC   NAR    DATA1  DATA2

-------  ----  ------  -----  -----

61h    49h    10h     10h    var

TRANSMIT CONDITIONS & MESSAGE CONTROL

--------------------------------------

SCP_MSG_CONTROL_... module

========================================================================

5-95

SCP INTERFACES, ECT_DEG_C MESSAGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SCP_ECT_DEG_C_COM1

ECT_DEG_C

Message Type:  Broadcast Transmit.

Data Flow:  From PCM to SCP Network as defined in SCP_MSG_CONTROL module.

Message Specification:

scp_pri_type                              = 61h

scp_gen_func (primary msg ID)             = 49h

scp_data_1 (Secondary ID of ECT msg)      = 10h

scp_data_2 (ECT value for SCP in deg.C)   = scpect_c

Message Transmit Routine:

Execute only upon send command for this message.

Change CFMFLG flag to SAE standard.

CFMFLG = 1 -----------------------------| scpect_c = FFh

|  ;ECT failure mode flag set

|

| --- ELSE ---

|

| scpect_c = ((ECT - 32) * 5/9) + 40

|  (ECT in deg C with 40 deg offset)

|

|  (Range: -40 - 214 deg C

|          => 00 - FE hex)

|  (Res:  1 deg C per bit)

|  (FF hex => Invalid scpect_c data)

5-96

ANTI-THEFT, PASSIVE ANTI-THEFT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

5.1.11  PASSIVE ANTI-THEFT (CDAK0)

OVERVIEW

+----------------------------+

|         ANTI_THEFT         |

+----------------------------+

| . definitions              |

| . anti_theft_reset         |

| . anti_theft_init          |

| . anti_theft_input         |

| . anti_theft_state_control |

| . anti_theft_timer_control |

| . anti_theft_controller    |

| . pat_fault_detection      |

| . store_a_code()           |

+----------------------------+

The Anti-theft strategy is used to disable the engine when an offboard device

(typically  the PATS module or LCM) determines a theft condition exists.  The

PCM performs a function read message directed at the offboard device.  If the

resulting  information indicates a theft condition is present, the anti-theft

strategy will output the FLG_THFT_FCO to the  fuel  control  strategy.   When

set,  this  flags  directs  the  fuel  control  strategy  to discontinue fuel

delivery to the engine.  The communications  between  the  PCM  and  offboard

device are carried out over the SCP vehicle network.

5-97

ANTI-THEFT, PASSIVE ANTI-THEFT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The anti-theft strategy is based on the parameter THEFT_STATE which is to  be

maintained in KAM.  This parameter tells us which state the anti-theft system

is in at all times.  The allowable states the system can be in are:

1.  DISABLE - This state is one in which the  anti-theft  strategy  has  been

disabled.   This  can be entered either by calibrating out the system or when

an SCP fault occurs after initial vehicle  start.   The  second  path  is  to

prevent  stranding  the  vehicle  operator when a system or component failure

occurs after the vehicle has successfully passed all the  criteria  to  start

the  car once.  It is assumed that a valid key and starting sequence has been

used to get the engine started, so it does not make sense to  prevent  engine

start  if the SCP network goes down later.  In the disable state, the vehicle

is allowed to start normally.

2.  START_ENGINE - This state is one in which  the  anti-theft  strategy  has

been  successfully  exercised and all security criteria have been met so that

the engine is allowed to continue starting.  The normal mode  of  entry  into

this  state  is by obtaining the 'ok' response from the function read message

that the PCM performs during the starting sequence.  The  START_ENGINE  state

can  also  be  entered  from  the  DISABLE  state  if  the calibration switch

SW_ANTI_THFT  is  set  and  the   SCP   network   is   functioning   properly

(PAT_FAULT=0).

3.  STOP_ENGINE - This state is one in  which  the  anti-theft  strategy  has

detected a breach in one of the security criteria.  The fuel delivery will be

stopped and the engine will not be allowed to start.  This state  is  entered

in  one of two ways.  The first is if the result of the function read results

in a 'theft' response.  The second is if a 'no_response'  or  'wait'  results

from the function read (i.e., the PATS module never responded to the function

read message or told us to wait because they were not ready yet.) and the PCM

has  been  in  run  mode  for more than 1 second.  This second path has a one

second time limit to ensure the vehicle has had  very  little  time  to  move

under it's own power prior to the engine fuel delivery being cut-out.

4.  DELAY - This state is entered on KAM  reset  and  RAM  initialization  if

currently  not  in the DISABLE state.  The strategy waits in this state until

the SCP network is up and capable of supporting  the  initial  function  read

message.   This  is  based  on SCP_DT2.  When this time has passed, the first

function read message takes place and the DELAY state is exited.

5.  RETRY - If the first function read after a KAM or RAM reset results in  a

'no_response'  or  'wait'  response  from the PATS module, the RETRY state is

entered.  This state will be exited when the next function  read  message  is

performed  or the PCM has been in run mode for more than one second or if KAM

or RAM initialization takes place again.

5-98

ANTI-THEFT, PASSIVE ANTI-THEFT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The following diagram shows the input/output relationship of  the  anti-theft

strategy:

SCP BASED ANTI-THEFT STRATEGY

-----------------------------

+---------------+

time -------------->|               |--------> FLG_THFT_FCO

SW_ANTI_THFT ------>|  ANTI-THEFT   |--------> FR_PAT_DATA

UNDSP ------------->|   CONTROL     |--------> P1260(theft, wait)

SCP_TMR ----------->|   STRATEGY    |--------> U1451(no response)

KAM_ERROR --------->|               |

+---------------+

|Local data:    |

|. THEFT_STATUS |

|. THEFT_STATE  |

|. PAT_FAULT    |

|. THEFT_TMR    |

|. SCP_PATS_TMR |

|. FR_PAT_DATA  |

+---------------+

The strategy is structured per the following diagram:

+------------+               +-------------------+

| background |               | RAM initialization|

+------+-----+               +---------+---------+

|                               |

+-----------+-----------+          +--------+--------+

| anti_theft_controller |          | anti_theft_init |

+-----------+-----------+          +-----------------+

|

+-------------+----+--------+------------+-----------------+

|             |             |            |                 |

+---+---+ +-------+-------+ +---+---+ +------+--------+ +------+-----------+

| reset | | timer_control | | input | | state_control | | pat_fault_detect |

+-------+ +---------------+ +-------+ +------+--------+ +------------------+

|

+-----+--------+

| store_a_code |

+--------------+

5-99

ANTI-THEFT, PASSIVE ANTI-THEFT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

A finite state machine will be used to graphically represent how the strategy

functions.

Below is a description of how a finite state machine functions.

A finite state machine is a way to graphically illustrate the functioning of

a system or process that can be in a set of DISCRETE states.  The diagram

shows how and when transitions can occur between the states.

+----------------------+

|                      |

|       STATE X        |<-------------------

|                      |        condition

+-----+----------A-----+        ---------

|          |               action

|          |

condition    |          |    condition

---------    |          |    ---------

action      |          |     action

|          |

|          |

|          |

+-----V----------+-----+

|                      |

|       STATE Y        |

|                      |

+----------------------+

CONVENTIONS:

· BOXES represent STATES of the system.

· ARROWS represent TRANSITIONS between states.

· CONDITIONS will CAUSE a transition to be taken.

· ACTIONS are done WHEN a transition is taken.

RULES:

· The system can be in only one state at a time.

· The system remains in a given state until an event

occurs to cause a transition.

· No other transitions than those shown can be taken.

5-100

ANTI-THEFT, PASSIVE ANTI-THEFT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FINITE STATE REPRESENTATION

---------------------------

SW_ANTI_THFT=0

--------------

FLG_THFT_FCO=0                      +---+SCP_PATS_TMR>=PATS_RETRY

THEFT_STATUS=no_resp                |   |---------------

THEFT_TMR=0                         |   |FR_PAT_DATA=0

from        SCP_PATS_TMR=0              +-------+-+ |send(ANTI_THEFT_STATUS)

any ----------------------------------->| DISABLE*|<+SCP_PATS_TMR=0

state                                   +--+----A-+

PAT_FAULT=0 &  |    |

SW_ANTI_THFT=1 |    | PAT_FAULT=1

-------------- |    | --------------

FLG_THFT_FCO=0 |    | FLG_THFT_FCO=0

send(ENGINE_PAT_STATUS) |    | send(ENGINE_PAT_STATUS)

|    |

+-V----+-+  SCP_PATS_TMR>=

| START  |-+             PATS_RETRY

+----------------------->| ENGINE*| |---------------

|     THEFT_STATUS=OK &  +-+-A--A-+ |FR_PAT_DATA=0

|     pu_time > SCP_DT2    | |  |   |send(ANTI_THEFT_STATUS)

|     -----------------    | |  +---+SCP_PATS_TMR=0

|     FLG_THFT_FCO=0       | |

SCP_TMR>=SCP_DT2 |  send(ENGINE_PAT_STATUS) | |  THEFT_TMR < 1 second &

---------------- |                          | |  THEFT_STATUS=OK

FR_PAT_DATA=0    | +------------------------+ |  -----------------

send(ANTI_       | |   power up               |  FLG_THFT_FCO=0

THEFT_STATUS)    | |   --------------         |  send(ENGINE_PAT_STATUS)

+-------+ | |   THEFT_STATUS=NO_RESP   |

|       | | |   FLG_THFT_FCO=0         |

+---->+-+-+-V-+                    +---+---+

------------>| DELAY |------------------->| RETRY |-+SCP_TMR>=SCP_DT2

KAM reset    +-A-+-A-+ [THEFT_STATUS=WAIT +-+-+-A-+ |----------------

-------------- | | |    or NO_RESP]&        | | |   |FR_PAT_DATA=0

FLG_THFT_FCO=0 | | |   pu_time > SCP_DT2    | | +---+send(ANTI_THEFT_STATUS)

THEFT_STATUS=  | | |   ------------------   | |

no_resp        | | |   FLG_THFT_FCO=0       | |

PAT_FAULT=0    | | |                        | |

THEFT_TMR=0    | | +------------------------+ |

SCP_PATS_TMR=0 | |     power up               |

| |     --------------         | THEFT_STATUS=THEFT OR

| |     THEFT_STATUS=NO_RESP   | [[THEFT_STATUS=WAIT

| |     FLG_THFT_FCO=0         | or NO_RESP] &

| |                            | THEFT_TMR >= 1 second]

| +--------------------------+ | ------------------

|       THEFT_STATUS=THEFT & | | FLG_THFT_FCO=1

|       pu_time > SCP_DT2    | | store_a_code(THEFT_STATUS)

|       -------------------- | | send(ENGINE_PAT_STATUS)

|       FLG_THFT_FCO=1       | |

|       store_code(P1260)    | |

|   send(ENGINE_PAT_STATUS)  | |

|                          +-V-V----+

+--------------------------|  STOP  |

power up           | ENGINE |

--------------     +--------+

THEFT_STATUS=NO_RESP

FLG_THFT_FCO=0

5-101

ANTI-THEFT, PASSIVE ANTI-THEFT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Where:

1. SCP_DT2 represents the time to wait prior to first function read message.

2. THEFT_STATUS is the returned data from the anti-theft function read

message.

3. The finite state machine is discrete, updated once per background pass or

during RAM initialization or during KAM initialization.

4. SCP_PATS_TMR is free running up counter in these states, otherwise it is

held at zero.

5-102

ANTI-THEFT, PASSIVE ANTI-THEFT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  FRECR = Commanded EBC Control Register (ECR) state for  a  Function  Read

SCP message transmit command.

-  FR_97_COUNT = State encoded register indicating which  type  of  SCP  bus

utilization is enabled for Function Read ANTI_THEFT_STATUS (target 97h).

State 0:  Invalid State (message not yet sent).

1:  Differentially Ended (+ & -).

2:  Single Ended - (Bus A).

3:  Single Ended + (Bus B).

-  KAM_ERROR = Indicates keep alive RAM invalid.

-  SCP_PATS_TMR = Control timer to send Anti-theft message  in  engine_start

state.

-  THEFT_STATE = SCP Anti-theft finite state machine control register.

-  THEFT_STATUS = State encoded byte for  anti-theft  status  determination,

obtained from function read message ANTI_THEFT_STATUS.

-  THEFT_TMR = Control timer to disable engine as a result of Anti-theft.

Bit Flags:

-  PAT_FAULT = Communication fault  indicated  between  PCM  and  Anti-theft

module; 1 -> fault, 0 -> no fault.

-  FLG_THFT_FCO = Flag signaling when fuel is being  cut  out  due  to  anti

theft system action; 1 -> cutting out fuel.

-  FR_PAT_DATA = Anti-theft data available flag; 1 ->  data  available  from

SCP.

-  UNDSP = Underspeed flag; 1 -> in underspeed mode.

Calibration Constants:

-  PATS_RETRY = Time delay to re-verify the SCP connection between the PCM &

the PATS module, seconds.

-  SCP_DT2 = Time delay to begin transmitting SCP status request messages.

-  SW_ANTI_THFT = Software  switch  to  enable/disable  the  EEC  controlled

vehicle anti theft strategy; 1 -> enable.

5-103

ANTI-THEFT, PASSIVE ANTI-THEFT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OUTPUTS

Registers:

-  ECR = EBC Control Register (NOT DISPLAYABLE).

-  FRECR = See above.

-  FR_97_COUNT = See above.

-  SCP_PATS_TMR = See above.

-  THEFT_STATE = See above.

-  THEFT_STATUS = See above.

-  THEFT_TMR = See above.

Bit Flags:

-  FLG_THFT_FCO = See above.

-  FR_PAT_DATA = See above.

-  PAT_FAULT = See above.

OTHER

-  P1260 = Engine disabled due to detected vehicle theft.

-  U1451 = Engine disabled due to  lack  of  response  from  the  anti-theft

module.

5-104

ANTI-THEFT, PASSIVE ANTI-THEFT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: ANTI_THEFT_COM4

BEGIN: definitions

;define parameters used to make the strategy more readable.  This is not to

;explicitly executed.

unconditionally ------------------------| delay := 0

| disable := 1

| start_engine := 2

| retry := 3

| stop_engine := 4

| no_resp := 80h

| ok := 7Fh

| wait := 01h

| theft := 00h

| true := 1

| false := 0

END: definitions

BEGIN: anti_theft_init

;The following logic is to executed during RAM initialization.

SW_ANTI_THFT = 0 -----------------------| THEFT_STATE := disable

;Anti-theft calibrated out             | FLG_THFT_FCO := false

| THEFT_STATUS := no_resp

| THEFT_TMR := 0

| SCP_PATS_TMR := 0

|

| --- ELSE ---

THEFT_STATE = start_engine -------|     |

|     |

THEFT_STATE = retry --------------|OR --| THEFT_STATE := delay

|     | THEFT_STATUS := no_resp

THEFT_STATE = stop_engine --------|     | FLG_THFT_FCO := false

| THEFT_TMR := 0

| SCP_PATS_TMR := 0

END: anti_theft_init

5-105

ANTI-THEFT, PASSIVE ANTI-THEFT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: anti_theft_controller

;execute the following once per background to control the anti-theft

;strategy.

SW_ANTI_THFT = 0 -----------------------| THEFT_STATE := disable

;Anti-theft calibrated out             | FLG_THFT_FCO := false

| THEFT_STATUS := no_resp

| THEFT_TMR := 0

| SCP_PATS_TMR := 0

|

| --- ELSE ---

|

| Do: anti_theft_reset

| Do: anti_theft_timer_control

| Do: anti_theft_state_control

| Do: anti_theft_input

| Do: pat_fault_detection

END: anti_theft_controller

5-106

ANTI-THEFT, PASSIVE ANTI-THEFT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: anti_theft_reset

;Logic that handles the KAM reset values of the anti-theft strategy.

KAM_ERROR = 1 --------------------------| THEFT_STATE := delay

| FLG_THFT_FCO := false

| THEFT_STATUS := no_resp

| PAT_FAULT := false

| THEFT_TMR := 0

| SCP_PATS_TMR := 0

END: anti_theft_reset

BEGIN: anti_theft_timer_control

;Logic process used to control THEFT_TMR, a high resolution timer for

;anti-theft strategy, and the fault detection message pacing timer,

;SCP_PATS_TMR.

UNDSP = 1 ------------------------------| THEFT_TMR := 0

THEFT_STATE <> start_engine ------|

|AND -| SCP_PATS_TMR := 0

THEFT_STATE <> disable -----------|

END: anti_theft_timer_control

BEGIN: store_a_code(status)

;Logic process used to interface to the diagnostic executive.

status = theft -------------------|

|OR --| store_code(P1260)

status = wait --------------------|     |

| --- ELSE ---

|

status = no_resp -----------------------| store_code(U1451)

END: store_a_code(status)

5-107

ANTI-THEFT, PASSIVE ANTI-THEFT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: anti_theft_state_control

;Logic that specifies operation of the FSM shown in the overview.

PAT_FAULT = 1 --------------------|

|AND -| THEFT_STATE := disable

THEFT_STATE = start_engine -------|     | FLG_THFT_FCO := false

| send(ENGINE_PAT_STATUS;

|         81h, THEFT_STATUS)

| ; Engine enabled - PATS disabled

|

| --- ELSE ---

THEFT_STATUS = ok ----------|           |

|AND -|     |

THEFT_STATE = delay --------|     |     |

|     |

THEFT_TMR < 1 second -------|     |     |

|     |     |

THEFT_STATUS = ok ----------|AND -|OR --| THEFT_STATE := start_engine

|     |     | FLG_THFT_FCO := false

THEFT_STATE = retry --------|     |     | send(ENGINE_PAT_STATUS;

|     |         81h, THEFT_STATUS)

PAT_FAULT = 0 --------------|     |     | ; Engine enabled

|AND -|     |

THEFT_STATE = disable ------|           |

| --- ELSE ---

THEFT_STATUS = wait --------|           |

|OR --|     |

THEFT_STATUS = no_resp -----|     |     |

|     | ; Retry PATS function read

SCP_TMR > SCP_DT2 ----------------|AND -| THEFT_STATE := retry

|     | FLG_THFT_FCO := false

THEFT_STATE = delay --------------|     |

| --- ELSE ---

THEFT_STATUS = theft -------|           |

|           |

THEFT_STATE = delay --|     |AND -|     |

|OR --|     |     |

THEFT_STATE = retry --|           |     |

|OR --| THEFT_STATE := stop_engine

THEFT_STATUS = wait --|           |     | FLG_THFT_FCO := true

|OR --|     |     | store_a_code(THEFT_STATUS)

THEFT_STATUS=no_resp -|     |     |     | send(ENGINE_PAT_STATUS;

|AND -|     |         01h, THEFT_STATUS)

THEFT_TMR >= 1 second ------|           | ; Engine disabled

|

THEFT_STATE = retry --------|

END: anti_theft_state_control

5-108

ANTI-THEFT, PASSIVE ANTI-THEFT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: anti_theft_input

;Logic process to obtain the THEFT_STATUS value via SCP from the PATS/LCM.

;To be executed once per background loop.

SCP_TMR >= SCP_DT2 ---------------|

|AND -| FR_PAT_DATA := 0

THEFT_STATE = delay --------|     |     | send(ANTI_THEFT_STATUS)

|OR --|     | ; Load Transmit Queue

THEFT_STATE = retry --------|

END: anti_theft_input

5-109

ANTI-THEFT, PASSIVE ANTI-THEFT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: pat_fault_detection

Logic process used to test the network connection between  the  PCM  and  the

PATS/LCM anti-theft module - ONLY execute if in link verification states.

NOTE:  The Function Read retry process continues until either three  attempts

are  made,  or until the message is properly acknowledged by the target Node.

If no acknowledge is received after  a  set  number  of  attempts,  then  the

message is considered faulted and the appropriate FMEM action is enabled.

THEFT_STATE <> start_engine ------|

|AND -| Exit (No Action)

THEFT_STATE <> disable -----------|     |

;Not in SCP link verification mode     |

| --- ELSE ---

|

FR_97_COUNT <> 1 -----------------|     | ; Queue up first Link

;First attempt not been queued   |     | ; Verification message.

|     |

FRECR = 0 ------------------------|AND -| FR_PAT_DATA := 0

;First attempt not been sent     |     | FR_97_COUNT := 1

|     | send(ANTI_THEFT_STATUS)

SCP_PATS_TMR >= PATS_RETRY -------|     |  ; Load Transmit Queue

;Waited enough time to retry link      |

| SCP_PATS_TMR := 0

|

| --- ELSE ---

FR_97_COUNT = 1 ------------|           |

;First attempt was queued  |           |

|           |

FRECR = 72h ----------------|AND -|     |

;First attempt was sent    |     |     |

|     |     |

FR_PAT_DATA = 0 ------------|     |     |

;Not yet seen good 'ack'         |OR --| ; Attempt to obtain good

|     | ; Acknowledge byte from

FR_97_COUNT = 2 ------------|     |     | ; Anti-Theft node FR register.

;Second attempt was queued |     |     |

|     |     | Increment FR_97_COUNT

FRECR = 52h ----------------|AND -|     | send(ANTI_THEFT_STATUS)

;Second attempt was sent   |           |  ; Load Transmit Queue

|           |

FR_PAT_DATA = 0 ------------|           | SCP_PATS_TMR := 0

;Not yet seen good 'ack'               |

| --- ELSE ---

FR_97_COUNT = 3 ------------------|     |

;Third attempt was queued        |     | ; Detected link fault.

|     |

FRECR = 32h ----------------------|AND -| PAT_FAULT := true

Third attempt was sent           |     | store_code(U1451)

|     |

FR_PAT_DATA = 0 ------------------|     | ; Set up for next link retry

;Still not seen good 'ack'             | FRECR := 0

END: pat_fault_detection

5-110

SCP INTERFACES, ANTI_THEFT_STATUS MESSAGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

5.1.12  ANTI_THEFT_STATUS Message (CDAK0)

OVERVIEW

Purpose of Message:

This Function Read message is sent from the EEEC to determine the  status  of

the  Anti-theft  system.   The message is sent per the conditions detailed in

the ANTI_THEFT section.  Successful receipt of the Function Read message will

also  indicate to the strategy that the SCP link is alive between the PCM and

the Anti-theft control module.

DEFINITIONS

OUTPUTS

Registers:

-  FRECR = Commanded EBC Control Register (ECR) state for  a  Function  Read

SCP message transmit command.

-  FR_97_COUNT = Register indicating type  of  bus  utilization  for  FR  of

ANTI_THEFT_STATUS (target 97h) message.

-  ECR = EBC Control Register (NOT DISPLAYABLE).

-  THEFT_STATUS = State encoded byte for  anti-theft  status  determination,

obtained from function read message ANTI_THEFT_STATUS.

Bit Flags:

-  FR_PAT_DATA = Anti-theft data available flag.

-  PAT_FAULT = Fault flag indicating a communications problem exists between

the PCM and the Anti-theft module.

========================================================================

Message Representation:

TARG  SOURCE

PRI/TYP  SPEC   NAR             ack

-------  ----  ------          -----

43h    97h    10h             var

TRANSMIT CONDITIONS & MESSAGE CONTROL

--------------------------------------

ANTI_THEFT_COMx

========================================================================

5-111

SCP INTERFACES, ANTI_THEFT_STATUS MESSAGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SCP_ANTI_THEFT_STATUS_COM1

ANTI_THEFT_STATUS

Message Type:  Function Read Transmit

Data Flow:  From PCM to network, responsible node returns a data byte

back to the PCM containing theft status.

Message Specification:

PCM sends:     scp_pri_type                     = 43h

scp_gen_func (primary msg ID)    = 97h

node responds:     scp_frdr_dat (theft status reply)= xxh

Acknowledgment Handler (execute upon reception of returned data byte ONLY):

scp_frdr_dat = 00h ---------------|

(THEFT indicated)                 |

|

scp_frdr_dat = 01h ---------------|OR --| THEFT_STATUS := scp_frdr_dat

(WAIT indicated)                  |     | FR_PAT_DATA := 1

|     | PAT_FAULT := 0

scp_frdr_dat = 7Fh ---------------|     | ECR := 72h

(OK indicated)                          | FRECR := 00h

| FR_97_COUNT := 0

|

| --- ELSE ---

|

| ECR := 72h

| FRECR := 00h

| FR_97_COUNT := 0

| No change to THEFT_STATUS

| No change to FR_PAT_DATA

| No change to PAT_FAULT

5-112

SCP INTERFACES, ENGINE_PAT_STATUS MESSAGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

5.1.13  ENGINE_PAT_STATUS Message (CDAK0)

OVERVIEW

Purpose of Message:

This Broadcast message is sent to provide results  of  the  PCM's  Anti-theft

logic (engine ENabled due to PATS, engine DISabled due to PATS, or PATs Logic

disabled due to communications problem).  The receiving node uses  this  data

to  either  set  or  clear  a  Fault  Indication light on the dashboard.  The

message is sent per the conditions detailed in the ANTI_THEFT chapter.

The variable data bytes provided in this message are defined  in  the  send()

construct which issues the transmit command.

DEFINITIONS

INPUTS

Registers:

-  THEFT_STATUS = State encoded byte for  anti-theft  status  determination,

obtained from function read message ANTI_THEFT_STATUS.

========================================================================

Message Representation:

TARG  SOURCE  SEC

PRI/TYP  SPEC   NAR    ID    data

-------  ----  ------  ----  ----

61h    93h    10h    var   var

TRANSMIT CONDITIONS & MESSAGE CONTROL

--------------------------------------

ANTI_THEFT_COMx

========================================================================

5-113

SCP INTERFACES, ENGINE_PAT_STATUS MESSAGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SCP_ENGINE_PAT_STATUS_COM1

ENGINE_PAT_STATUS

Message Type:  Broadcast Transmit

Data Flow:  From PCM to network.

Message Specification:

scp_pri_type                     = 61h

scp_gen_func (primary msg ID)    = 93h

scp_data_1 (secondary msg ID)    = xxh

scp_data_2 (THEFT_STATUS echo)   = xxh

5-114

SCP INTERFACES, ENGINE PAT DATA STATUS REQUEST MESSAGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

5.1.14  REQUEST_SECURITY_STATUS (CDAK0)

OVERVIEW

This SCP message can be used by various modules (on multi-node  vehicles)  to

request  a  broadcast  of  the  message  transmitted  by  the PCM immediately

following a decision to either start the vehicle or disable it as part  of  a

detected  theft  countermeasure.  Even though this message is quite specific,

SAE rules dictate that a Secondary ID be evaluated to determine what  message

is actually being requested for broadcast.

DEFINITIONS

INPUTS

Registers:

-  scp_data_n = Data byte in SCP message (n = 1 through 7).

========================================================================

Message Representation:

TARG  SOURCE

PRI/TYP  SPEC   NAR    DATA1

-------  ----  ------  -----

61h    93h    ??h     var

CONTROL

RECEIVE CONDITIONS              FLAGS

-----------------------------  -------

Unknown.                        none

========================================================================

5-115

SCP INTERFACES, ENGINE PAT DATA STATUS REQUEST MESSAGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SCP_REQUEST_SECURITY_STATUS_COM1

REQUEST_SECURITY_STATUS

Message Type:  Broadcast Receive.

Data Flow:  From SCP network to EEEC.

Receive Buffer Data Bytes:

Sending Node Address                 = ??h (not used in strategy)

scp_data_1 (Sec ID of req'd msg)     = xxh

Receive Message Execution (only when this message is received):

scp_data_1 = C1h -----------------------| send(ENGINE_PAT_STATUS)

5-116

SCP INTERFACES, SCP OBDII MODE 06 REPORTING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

5.2  SCP OBDII SAE-J1979 MODE 06 SUPPORT MAP (CDAN0)

OVERVIEW

This strategy specific module identifies the test  results  and  test  limits

required for replying to an OBDII Mode 06 request for data.  Mode 06 is fully

documented in SAE J1979.   The  Response  Table  included  here  is  used  in

conjunction   with   the   logic   in   SCP_OBDII_DIAG_COMn  to  reply  to  a

PROCESS_OBDII_REQUEST message.

DEFINITIONS

Registers:

-  CMIS1FIL = Misfire test cylinder 1 rolling average fault filter.

-  CMIS2FIL = Misfire test cylinder 2 rolling average fault filter.

-  CMIS3FIL = Misfire test cylinder 3 rolling average fault filter.

-  CMIS4FIL = Misfire test cylinder 4 rolling average fault filter.

-  CMIS5FIL = Misfire test cylinder 5 rolling average fault filter.

-  CMIS6FIL = Misfire test cylinder 6 rolling average fault filter.

-  CMIS7FIL = Misfire test cylinder 7 rolling average fault filter.

-  CMIS8FIL = Misfire test cylinder 8 rolling average fault filter.

-  DELPR_FLW = Filtered DELPR during test for mode 06 reporting, in H2O.

-  DELPR_HOSE = Filtered DELPR during test for mode 06 reporting, in H2O.

-  EGO1_RF_AVE = Rear to front EGO1 average switch ratio.

-  EGO2_RF_AVE = Rear to front EGO2 average switch ratio.

-  EGRDC_FLW = Filtered EGRDC during test for mode 06 reporting.

-  EPTBAR_OPN = Filtered EPTBAR during test mode 06 reporting.

-  EPT_OPN_MAX = EPTZER_OFF + EPT_VOPN_IDL stored failure condition for mode

06.

-  FAO_SCP_AVG1 = The average value of the integral  value  of  R_BIAS1  for

mode 06.

-  FAO_SCP_AVG2 = The average value of the integral  value  of  R_BIAS2  for

mode 06.

-  MIN_DELPR_FL = Filtered MIN_delpr during test for mode 06  reporting,  in

H2O.

5-117

SCP INTERFACES, SCP OBDII MODE 06 REPORTING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PGM_BLD_DLTK = Purge  monitor  bleed  pressure  allowed  before  test  is

failed, from FN_PGM_BLEED(PGM_P0_WPEK), KAM version.

-  PGM_P0_BEGK = Value of PGM_PS0_BEGP stored to KAM for SCP reporting.

-  PGM_P2_DLTPK = Value of PGM_PS2_DLTP stored to KAM for SCP reporting.

-  PGM_P4_DELPK = Value of PGM_PS4_DELP stored to KAM for SCP reporting.

-  PGM_P4_DLTPK = Value of PGM_PS4_DLTP stored to KAM for SCP reporting.

-  SAIR_TMRK1 = KAM  snapshot  of  SAIR_LN_TMR  at  completion  of  upstream

thermactor flow test.

-  SAIR_TMRK2 = KAM snapshot of  SAIR_LN_TMR  at  completion  of  downstream

thermactor flow test.

-  TOTMISFIL = Overall misfire fault rolling average fault filter .

-  V_MIS_TSTCNT =  Register  containing  number  of  misfire  test  testable

events.

-  VEGO_AMP11 = Amplitude (volts) of difference between rich  and  lean  for

bank1, fore HEGO.

-  VEGO_AMP21 = Amplitude (volts) of difference between rich  and  lean  for

bank2, fore HEGO.

-  VEGO11_FIL2 = Second order filtered value of bank1 upstream HEGO voltage.

-  VEGO12_FIL2 = Second  order  filtered  value  of  bank1  downstream  HEGO

voltage.

-  VEGO21_FIL2 = Second order filtered value of bank2 upstream HEGO voltage.

Calibration Constants:

-  DELPR_THRES1 = Threshold on DELPR to check  for  downstream  vacuum  hose

off.

-  DELPR_THRES3 = Threshold on DELPR to check for upstream vacuum hose off.

-  DS_SWTPOINT = Downstream HEGO switch point.

-  EGO1_FL_RAT = Value at which Bank 1 catalyst is considered failed.

-  EGO2_FL_RAT = Value at which Bank 2 catalyst is considered failed.

-  LEAN_AMP_M = U.S.  EGO voltage amplitude to signal a  failure  when  area

ratio is not shifted rich or lean (median).

-  MISLEVEL1000 = Percent misfire threshold for emissions.

-  PGM_PS0_LO = Low tank pressure value of phase zero deadband.

-  PGM_PS0_HI = High tank pressure value of phase zero deadband.

5-118

SCP INTERFACES, SCP OBDII MODE 06 REPORTING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PGM_PS4_DP = Maximum allowable delta presure in phase four to allow purge

monitor test.

-  RBIAS_EGO_MX = Maximum allowable value of R_BIAS.

-  RBIAS_EGO_MN = Minimum allowable value of R_BIAS.

-  SAIR_EGO_LVL = Voltage on EGO sensor indicating lean/rich boundry.

-  SAIR_LN_TM = Total amount of time that both egos must  be  lean  to  pass

thermactor flow test.

-  SAIR_LN_TM2 = Total amount of time that downstream ego must  be  lean  to

pass thermactor flow test.

-  TESTIDMAP_1 = Bitmapped word containing supported Test IDs in  the  range

01h - 10h.

-  TESTIDMAP_2 = Bitmapped word containing supported Test IDs in  the  range

11h - 20h.

-  TESTIDMAP_3 = Bitmapped word containing supported Test IDs in  the  range

21h - 30h.

-  TESTIDMAP_4 = Bitmapped word containing supported Test IDs in  the  range

31h - 40h.

-  TESTIDMAP_5 = Bitmapped word containing supported Test IDs in  the  range

41h - 50h.

-  TESTIDMAP_6 = Bitmapped word containing supported Test IDs in  the  range

51h - 60h.

-  US_SWTPOINT = Upstream HEGO switch point.

-  V_EGRDC_MAX = Minimum EVR duty cycle for running insufficient flow test.

5-119

SCP INTERFACES, SCP OBDII MODE 06 REPORTING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SCP_MODE06_MAP_NAAO_COM1

BEGIN: SCP_MODE06_MAP

The following response table shows the parameters that must be

returned to meet the mode 06 reporting requirements.   Both test

support and test result reporting are handled through this chart.

Note that the number of messages sent in response to a given

Test ID is equal to the number of row entries in the following

table whose Test ID matches the requested Test ID.

The reply message this data is used for is REPORT_OBDII_TRIP_RESULTS,

detailed in the SCP_OBDII_DIAG_COMn module.  This table is for use

with NAAO strategy logic.

5-120

SCP INTERFACES, SCP OBDII MODE 06 REPORTING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SCP MODE 06 TEST ID MAP

|---------------------------------------------------------------------------|

| Test |Test Limit  |                                                       |

|  ID  |Type and    |                  PARAMETER NAME                       |

|(hex) |Component ID|Parameter MSB|Parameter LSB|Parameter MSB|Parameter LSB|

|------|------------|-------------|-------------|-------------|-------------|

|      |scp_data_3  | scp_data_4  | scp_data_5  | scp_data_6  | scp_data_7  |

|------|------------|-------------|-------------|-------------|-------------|

|  00  |    FF      |      TESTIDMAP_1          |      TESTIDMAP_2          |

|  01  |    91      |      VEGO_AMP11           |      LEAN_AMP_M           |

|  01  |    A1      |      VEGO_AMP21           |      LEAN_AMP_M           |

|  02  |    91      |      FAO_SCP_AVG1         |      RBIAS_EGO_MN         |

|  02  |    11      |      FAO_SCP_AVG1         |      RBIAS_EGO_MX         |

|  02  |    A1      |      FAO_SCP_AVG2         |      RBIAS_EGO_MN         |

|  02  |    21      |      FAO_SCP_AVG2         |      RBIAS_EGO_MX         |

|  03  |    81      |      US_SWTPOINT          |         0000h             |

|  03  |    82      |      DS_SWTPOINT          |         0000h             |

|  10  |    11      |      EGO1_RF_AVE          |      EGO1_FL_RAT          |

|  10  |    21      |      EGO2_RF_AVE          |      EGO2_FL_RAT          |

|  20  |    FF      |      TESTIDMAP_3          |      TESTIDMAP_4          |

|  21  |    80      |      PGM_P0_BEGK          |      PGM_PS0_LO           |

|  21  |    00      |      PGM_P0_BEGK          |      PGM_PS0_HI           |

|  22  |    00      |      PGM_P2_DLTPK         |      PGM_BLD_DLTK         |

|  23  |    00      |      PGM_P4_DELPK         |      PGM_PS4_DP           |

|  24  |    00      |      PGM_P4_DLTPK         |      PGM_BLD_DLTK         |

|  30  |    11      |      VEGO11_FIL2          |      SAIR_EGO_LVL         |

|  30  |    21      |      VEGO21_FIL2          |      SAIR_EGO_LVL         |

|  30  |    12      |      VEGO12_FIL2          |      SAIR_EGO_LVL         |

|  31  |    80      |      SAIR_TMRK1           |      SAIR_LN_TM           |

|  31  |    81      |      SAIR_TMRK2           |      SAIR_LN_TM2          |

|  40  |    FF      |      TESTIDMAP_5          |      TESTIDMAP_6          |

|  41  |    91      |      DELPR_HOSE           |      DELPR_THRES3         |

|  41  |    12      |      DELPR_HOSE           |      DELPR_THRES1         |

|  45  |    20      |      EPTBAR_OPN           |      EPT_OPN_MAX          |

|  4A  |    B0      |      DELPR_FLW            |      MIN_DELPR_FL         |

|  4B  |    30      |      EGRDC_FLW            |      V_EGRDC_MAX          |

|  50  |    00      |      TOTMISFIL            |      MISLEVEL1000         |

|  51  |    01      |      CMIS1FIL             |      MISLEVEL1000         |

|  51  |    02      |      CMIS2FIL             |      MISLEVEL1000         |

|  51  |    03      |      CMIS3FIL             |      MISLEVEL1000         |

|  51  |    04      |      CMIS4FIL             |      MISLEVEL1000         |

|  51  |    05      |      CMIS5FIL             |      MISLEVEL1000         |

|  51  |    06      |      CMIS6FIL             |      MISLEVEL1000         |

|  51  |    07      |      CMIS7FIL             |      MISLEVEL1000         |

|  51  |    08      |      CMIS8FIL             |      MISLEVEL1000         |

|  52  |    00      |      V_MIS_TSTCNT         |         0000h             |

|------|------------|---------------------------|---------------------------|

Note: All Mode 06 parameters that are not a full word in length MUST be

loaded in the LSB of the parameter field, and zero loaded in the MSB of

the parameter field.

END: SCP_MODE06_MAP

5-121

SCP INTERFACES, SCP PARAMETER IDENTIFIER (PID) LIST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

5.3  SCP PID LIST (CDAN0)

OVERVIEW

SCP Parameter ID Map Definitions

The tables below outline the  SCP  Parameter  Identifiers  (PIDs)  which  are

supported  by  the  EEEC.   A  PID  is  a  four  digit  (two  byte) numerical

representation which is used to quickly reference a  particular  standardized

EEEC  parameter;  such that if the number is requested by an offboard device,

the EEEC will respond with the value of the parameter which is mapped to that

number.   PIDs  can  reference  EEEC  RAM,  KAM,  or ROM parameters.  All PID

numbers have been standardized so that the same list  of  PIDs  can  be  used

across  all  FORD vehicle lines (which utilize SCP communications).  The list

of PIDs which are supported by this strategy is  presented  in  three  tables

which follow this overview.

The first two tables support PID's which are legislated by the specification;

"SAE  RECOMMENDED  PRACTICE J1979 (E/E DIAGNOSTIC TEST MODES)".  These tables

are referenced only when reporting J1979 emissions-related  diagnostic  data.

The  first  table  represents  the  latest RAM calculation of the J1979 data,

while the second table represents a snapshot of data taken at the  occurrence

of  an  OBDII  system monitor failure (as specified in the CARB regulations).

The second table is the KAM Freeze Frame PID map as legislated in J1979.

The J1979 parameters are typically requested by the OBDII Scan Tool, but  are

also  available  (for  single  request, single response) for request by other

devices via the SCP network.  The OBDII Scan Tool can ONLY reference PID data

from the J1979 PID tables (the first two PID tables).

The OBDII Scan Tool is a generic device (ie.  manufacturer independent) which

requires  the parameter information requested to be in a specific format.  It

is also capable of obtaining and clearing  any  emmission-related  Diagnostic

Trouble  Codes  which may be stored in KAM, as well as obtaining results from

the vehicle oxygen sensor monitor tests.

The third table supports the manufacturer specific PID set which is  included

with this vehicle application.  The third PID list is the ONLY list which can

be referenced when requesting ANY repetitive  data  diagnostics.   This  list

CANNOT be referenced with an OBDII Generic SCAN TOOL.

The PID table columns are defined as follows:  the PID  number;  the  defined

parameter  (or  pid_definition)  which  is  referenced by the PID number; the

output resolution in engineering units  per  count;  the  output  scaling  in

binary point, and; the data type (byte, word, signed, or unsigned).

5-122

SCP INTERFACES, SCP PARAMETER IDENTIFIER (PID) LIST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

OBDII J1979 Parameter Definitions

The following parameters are necessary to comply with the OBDII  requirements

as  stated in the SAE J1979 specification.  The EEEC must respond to the Scan

Tool within 100 msec of  receiving  a  request.   Parameter  information  may

consist  of one or more bytes of binary, hex, or bitmap data as specified per

parameter.  Examples include engine load as a percent or oxygen sensor  as  a

voltage.  The parameters may be variable or constant.  They may represent the

most recent calculation of that parameter (j1979_01_xx) or a snapshot in time

(j1979_02_xx) as required by the J1979 mandated Freeze Frame.

j1979_01_00   This four byte parameter consists of a bitmap of the PIDS

supported by this system.  This shall be a calibrated constant

parameter such that bit 7 (MSB) of the first byte (1st byte

sent in response to the request) corresponds to PID 1 and

bit 0 (LSB) of the first byte corresponds to PID 8.  The

next byte identifies PIDS 9 through 16.  The last will

identify PIDS 25 through 32.  If the bit is a one then the

PID is supported.  If it is a zero, the PID is not supported.

An example of the bitmap written in hex is 'FF FE C0 00'.  In

this example PIDs 1 through 15, 17, and 18 are supported.

j1979_01_011  This single byte parameter contains the number of emission

trouble codes stored.  This variable is an integer which can

range from zero to 128 in the lower six bits of the byte.

It shall represent the current number of emission codes

identified by the monitor processes which are required by

CARB for OBDII.  The high order bit (7) is set to a one if

the MIL light is commanded on by this module or set to a

zero if it is not commanded on by this module.  This KAM

parameter shall be initialized to zero upon KAM reset.

j1979_01_012  This single byte parameter consists of a bitmap of the

continuous monitoring tests supported by the system.  This

shall be a calibrated constant as follows: bits 7 through 3

are not assigned, bit 2 refers to the components test, bit 1

refers to the fuel test, and bit 0 refers to the misfire

monitor test.  If the bit is a zero the test is not supported.

j1979_01_013  This single byte parameter consists of a bitmap of the

emission evaluation tests supported by the system.  (These

are the tests which are run once per trip.)  This byte

shall be a calibrated constant as follows: bit 7 (MSB) refers

to the EGR system, bit 6 is the oxygen sensor heater, bit 5 is

the oxygen sensor, bit 4 is the A/C refrigerant, bit 3 is the

secondary AIR system, bit 2 is the evaporative purge system,

bit 1 is heated catalyst, and bit 0 is the catalyst.  If the

bit is a zero then the test is not supported.

5-123

SCP INTERFACES, SCP PARAMETER IDENTIFIER (PID) LIST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

J1979 Regulated PID Definitions (continued)

j1979_01_014  This single byte parameter consists of a bitmap of the

emission evaluation tests status for those tests supported by

the system.  ( These are the tests which are run once per

trip. )  This byte shall be updated as appropriate.  The

bit assignments correspond to j1979_01_013.  If the

tests are not supported there, then the corresponding bits

here have no function.  The assignments are: bit 7 (MSB) refers

to the EGR system test completion, bit 6 is the oxygen sensor

heater test completion, bit 5 is the oxygen sensor test

completion, bit 4 is the A/C refrigerant test completion, bit

3 is the secondary AIR system test completion, bit 2 is the

evaporative purge system test completion, bit 1 is heated

catalyst test completion, and bit 0 is the catalyst test

completion.  If a bit is a one then the test has not been

completed. (See j1979_01_013.)  This parameter shall

be initialized to FF hex and maintained in KAM.

j1979_01_031  This single byte parameter consists of a bitmap of the current

BANK 1 fuel control status.  This RAM variable is defined

as follows: bit 7 through 5 are reserved and undefined, bit 4

is set to one when fuel control is closed loop but a fault

exists with at least one oxygen sensor ( Fuel control may be

using a single oxygen sensor.)  It is reset when this

condition no longer exists.  Bit 3 is set to one when fuel

control is open loop due to a detected system fault.  It is

reset when this condition no longer exists.  Bit 2 is set when

fuel control is open loop because of driving conditions.  It

is reset when this condition does not exist.  Bit 1 is set

when fuel control is closed loop and oxygen sensor(s) are

being used for feedback.  It is reset when this condition does

not exist.  Bit 0 is set when fuel control is open loop and

has not satisfied the conditions to go closed loop.  These

bits only apply to BANK 1 (contains cylinder 1).  Note that

only one bit may be set at any point in time.  This parameter

shall be initialized to one upon power-up of the EEC module.

j1979_01_04   This single byte variable contains the current load on the

engine expressed as a percent where 00h corresponds to

zero percent and FFh corresponds to 100 percent of maximum

load of the engine.  In other words, one count represents

1/255 percent of maximum load.  This RAM parameter shall be

initialized to zero.  The J1979 definition of load is:

CURRENT AIRFLOW    SEA LEVEL ATMOSPHERIC PRES.

CLV =  ------------------- X --------------------------- X100

PEAK AIRFLOW @ SEA       BAROMETRIC PRESSURE

Note that wide open throttle is 100%.

j1979_01_05   This single byte variable contains the current engine coolant

temperature, expressed in Degrees C (plus 40).  A value of 00h

corresponds to -40 Degrees C and a value of FFh corresponds to

215 Degrees C.  This RAM parameter shall be initialized to 00h

The value reported must reflect the current value of the

sensor input and not a substituted value.

5-124

SCP INTERFACES, SCP PARAMETER IDENTIFIER (PID) LIST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

J1979 Regulated PID Definitions (continued)

j1979_01_06   This single byte variable contains the current BANK 1 fuel

trim adjustment from stochiometry which is considered SHORT

TERM.  It is expressed as a percent of range.  A value of 00h

corresponds to a LAMBSE1 of infinity (or 100% lean).  A value

of FFh corresponds to a LAMBSE1 of 0.50196 (or 99.22% rich).

A value of 80h corresponds to a LAMBSE1 of 1 (or at stoich).

This RAM parameter shall be initialized to zero.

j1979_01_07   This single byte variable contains the current BANK 1 fuel

trim adjustment from stochiometry which is considered LONG

TERM.  It is expressed as a percent of range.  A value of 00h

corresponds to a KAMRF1 of zero (or 100% lean).  A value

of FFh corresponds to a KAMRF1 of 1.9922 (or 99.22% rich).

A value of 80h corresponds to a KAMRF1 of 1 (or at stoich).

This RAM parameter shall be initialized to zero.

j1979_01_08   This single byte variable contains the current BANK 2 fuel

trim adjustment from stochiometry which is considered SHORT

TERM.  It is expressed as a percent of range.  A value of 00h

corresponds to a LAMBSE2 of infinity (or 100% lean).  A value

of FFh corresponds to a LAMBSE2 of 0.50196 (or 99.22% rich).

A value of 80h corresponds to a LAMBSE2 of 1 (or at stoich).

This RAM parameter shall be initialized to zero.

j1979_01_09   This single byte variable contains the current BANK 2 fuel

trim adjustment from stochiometry which are considered LONG

TERM.  It is expressed as a percent of range.  A value of 00h

corresponds to a KAMRF2 of zero (or 100% lean).  A value

of FFh corresponds to a KAMRF2 of 1.9922 (or 99.22% rich).

A value of 80h corresponds to a KAMRF2 of 1 (or at stoich).

This RAM parameter shall be initialized to zero.

j1979_01_0C   This double byte variable contains the current engine speed

in units of one quarter RPM.  The range is from zero to

16,383.75.  This RAM parameter shall be initialized to zero.

j1979_01_0D   This single byte variable contains the current vehicle

speed in kilometers per hour.  The range is from zero to

255 KPH.  This RAM parameter shall be initialized to zero.

j1979_01_0E   This single byte variable contains the current ignition

timing for the number one cylinder.  The range will be from

-64.0 to +63.5 and 128 counts corresponds to 0 degrees crank.

Thus, 0 counts corresponds to -64.0 and 255 to +63.5 degrees.

One count equals one half degree crank.  This RAM parameter

shall be initialized to 148 counts (10 degrees).

5-125

SCP INTERFACES, SCP PARAMETER IDENTIFIER (PID) LIST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

J1979 Regulated PID Definitions (continued)

j1979_01_0F   This single byte variable contains the current inlet AIR

temperature of the engine expressed in Degrees C (plus 40).

A value of 00h corresponds to -40 Degrees C and a value of FFh

corresponds to 215 Degrees C.  This RAM parameter shall be

initialized to zero.  The value reported must reflect the

current value of the sensor input, not a substituted value.

j1979_01_10   This double byte variable contains the current mass AIR

flow in hundredths of grams per second.  One count equals .01

gm/sec.  The range is from zero to 655.35 gm/sec.  This RAM

parameter shall be initialized to zero.  The value reported

must reflect the current value of the sensor input and not

a substituted value.

j1979_01_11   This single byte variable contains the current absolute

throttle position as a percent.  A value of 00h corresponds to

absolute minimum position (zero% of range) and a value of FFh

is 100 percent of maximum throttle position range.  This RAM

parameter shall be initialized to 128.    The value reported

must reflect the current value of the sensor input and not a

substituted value.

j1979_01_12   This single byte contains a bitmap of the commanded secondary

AIR system.  This RAM variable is defined as follows: Bits 3

through 7 are reserved and should be set to zero, bit 2 is

zero for active operation and one for vented to atmosphere,

bit 1 is one for downstream of the first catalytic converter

inlet operation and zero is not, bit 0 is one for upstream of

first catalytic converter operation and zero is not.  These

three bits (0-2) can only have one bit set to a one at a time.

This byte shall indicate the current status of the secondary

AIR, not an FMEM substituted value.

j1979_01_13   This single byte contains a bitmap of the oxygen sensor

locations.  This ROM calibration parameter is defined as:

Bits 7 and 3 are unused, bit 6 is bank 2 sensor 3, bit 5 is

bank 2 sensor 2, bit 4 is bank 2 sensor 1, bit 2 is bank 1

sensor 3, bit 1 is bank 1 sensor 2, and bit 0 is bank 1

sensor 1.  A one indicates a sensor is present at that

LOCAtion.  A zero indicates not present.  The value for four

sensors would be 51 decimal.

5-126

SCP INTERFACES, SCP PARAMETER IDENTIFIER (PID) LIST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

J1979 Regulated PID Definitions (continued)

j1979_01_14   This single byte RAM variable contains the current oxygen

sensor voltage for BANK ONE, SENSOR ONE.  The range is from

zero to 1.275 volts. One count corresponds to .005 volts.

(This scaling presumes a one volt full scale sensor.)

j1979_01_151  This single byte RAM variable contains the current oxygen

sensor voltage for BANK ONE, SENSOR TWO.  The range is from

zero to 1.275 volts. One count corresponds to .005 volts.

(This scaling presumes a one volt full scale sensor.)

j1979_01_152  This single byte RAM variable represents the SHORT TERM fuel

trim associated with the oxygen sensor: BANK ONE, SENSOR TWO.

Current strategies do not support this feature in the fuel

calculations, so the parameter j1979_01_152 is set to 255

(or FFh) per the J1979 specification.

j1979_01_18   This single byte RAM variable contains the current oxygen

sensor voltage for BANK TWO, SENSOR ONE.  The range is from

zero to 1.275 volts. One count corresponds to .005 volts.

(This scaling presumes a one volt full scale sensor.)

j1979_01_191  This single byte RAM variable contains the current oxygen

sensor voltage for BANK TWO, SENSOR TWO.  The range is from

zero to 1.275 volts.  One count corresponds to .005 volts.

(This scaling presumes a one volt full scale sensor.)

j1979_01_192  This single byte RAM variable represents the SHORT TERM fuel

trim associated with the oxygen sensor: BANK ONE, SENSOR TWO.

Current strategies do not support this feature in the fuel

calculations, so the parameter j1979_01_192 is set to 255

(or FFh) per the J1979 specification.

j1979_01_1C   This single byte parameter will tell the requester the OBD

requirements to which vehicle is designed. Its value is

controlled by the calibration constant J1979_01_1C, where:

01 - OBD II (California ARB)

02 - OBD (Federal EPA)

03 - OBD and OBD II

04 - OBD I

05 - Not intended to meet any OBD requirements

5-127

SCP INTERFACES, SCP PARAMETER IDENTIFIER (PID) LIST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

J1979 Regulated Freeze Frame PIDs

A freeze frame PID refers to the value of a specific parameter at the

time an OBDII failure is detected requiring freeze frame data storage,

per the SAE specification J1979.  For further definition on the freeze

frame or any other OBDII related data format, please refer to J1979.

j1979_02_00   This four byte parameter consists of a bitmap of the freeze

frame PIDS supported by this system.  This shall be a

calibrated constant parameter such that bit 7 (MSB) of the

first byte (1st byte sent in response to the request)

corresponds to freeze frame PID 1 and bit 0 (LSB) of the

first byte corresponds to freeze frame PID 8.  The next byte

identifies freeze frame PIDS 9 through 16.  The last will

identify freeze frame PIDS 25 through 32.  If the bit is a one

then the freeze frame PID is supported.  If it is a zero, the

freeze frame PID is not supported.  An example of the bitmap

written in hex is 'FF FE C0 00'.  In this example freeze frame

PIDs 1 through 15, 17, and 18 are supported.

j1979_02_02   This two byte parameter is the OBDII diagnostic code which

triggered the freeze frame.  This KAM parameter shall be a

special format as follows.  The upper nibble (bits 7,6,5,4) of

the first byte represents the trouble code type (zero, one,

two and three are reserved for Powertrain trouble codes).  The

lower nibble of the upper byte (bits 3,2,1,0) will contain the

first decimal digit of the OBDII trouble code.  The second

byte shall contain the middle and last decimal digits

corresponding to bits 7 through 4 and bits 3 through 0,

respectively.  An example for code 0298 would be hex '02 98'.

j1979_02_031  This single byte parameter consists of a bitmap of the freeze

frame fuel control status for BANK 1 (contains cylinder 1).

This KAM variable is defined as the value of j1979_01_031 at

the last required freeze frame storage.  The parameter

shall be initialized to one upon EEEC KAM initialization.

j1979_02_04   This single byte variable contains the freeze frame load

on the engine.  It is expressed as a percent where 00h

corresponds to zero percent and FFh corresponds to 100

percent of maximum load of the engine.  In other words, one

count represents 1/255 percent of maximum load.  This KAM

parameter shall be initialized to zero and set to the percent

load, j1979_01_04, at the last required freeze frame storage.

j1979_02_05   This single byte variable contains the freeze frame coolant

temperature of the engine expressed in Degrees C (plus 40).

A value of 00h corresponds to -40 Degrees C and a value of FFh

corresponds to 215 Degrees C.  This KAM parameter shall be

initialized to zero and set to j1979_01_05 upon the last

required freeze frame storage.

5-128

SCP INTERFACES, SCP PARAMETER IDENTIFIER (PID) LIST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

J1979 Regulated Freeze Frame PIDs (continued)

j1979_02_06   This single byte variable contains the SHORT TERM freeze frame

fuel trim adjustment from stochiometry (BANK 1 only).  It is

expressed as a percent of range.  A value of 00h corresponds

to -100% of max. lean.  The maximum value of FFh corresponds

to +99.22% of max. rich.  This KAM parameter shall be

initialized to zero.  This parameter reflects the value of

j1979_01_06 upon the last required freeze frame storage.

j1979_02_07   This single byte variable contains the LONG TERM freeze frame

fuel trim adjustment from stochiometry (BANK 1 only).  It is

expressed as a percent of range.  A value of 00h corresponds

to -100% of max. lean.  The maximum value of FFh corresponds

to +99.22% of max. rich.  This KAM parameter shall be

initialized to zero.  This parameter reflects the value of

j1979_01_07 upon the last required freeze frame storage.

j1979_02_08   This single byte variable contains the SHORT TERM freeze frame

fuel trim adjustment from stochiometry (BANK 2 only).  It is

expressed as a percent of range.  A value of 00h corresponds

to -100% of max. lean.  The maximum value of FFh corresponds

to +99.22% of max. rich.  This KAM parameter shall be

initialized to zero.  This parameter reflects the value of

j1979_01_08 upon the last required freeze frame storage.

j1979_02_09   This single byte variable contains the LONG TERM freeze frame

fuel trim adjustment from stochiometry (BANK 2 only).  It is

expressed as a percent of range.  A value of 00h corresponds

to -100% of max. lean.  The maximum value of FFh corresponds

to +99.22% of max. rich.  This KAM parameter shall be

initialized to zero.  This parameter reflects the value of

j1979_01_09 upon the last required freeze frame storage.

j1979_02_0C   This double byte variable contains the freeze frame engine

speed in units of one quarter RPM.  The range is from zero to

16,383.75.  It shall have an accuracy of +/- 50RPM.  This KAM

parameter shall be set to zero upon EEEC KAM initialization.

It shall reflect the value of j1979_01_0C upon the last

required freeze frame storage.

j1979_02_0D   This single byte variable contains the freeze frame vehicle

speed in kilometers per hour.  The range is from zero to 255

kilometers per hour.  It shall have an accuracy of +/- 8KPH.

This KAM parameter shall be set to zero upon EEEC KAM

initialization.  It shall reflect the value of j1979_01_0D

upon the last required freeze frame storage.

5-129

SCP INTERFACES, SCP PARAMETER IDENTIFIER (PID) LIST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Other SCP Specific Parameter Definitions

----------------------------------------

BITMAP_FMEMx  A bitmapped register indicating status of various EEEC FMEM

strategies (see pid_def definitions for bitmap).

IDBLOCKH_ADR  A single-byte cal constant which represents the ROM Bank in

which the 256-byte Flash EEPROM ID Block is located.

IDBLOCK_ADR   A two-byte cal constant which contains the starting address

(High Byte, Low Byte) of the 256-byte Flash EEPROM ID Block.

{The value of this parameter would be 9F00h for an 88K chip,

and FF00h for a 112K chip.}.

IDBLOCK_FMT   A single-byte register which is assigned the CONTENTS of the

memory location offset 13h from IDBLOCK_ADR each BG Loop.

(ie.  9F13 for the 88K chip, and FF13 for the 112K chip).

The value of this memory location is FFh as a default, but

can be modified via the SBDS Flash procedure.  It is meant to

signify how the 256-byte ID block data is formatted such that

an ODU can request specific information from the EEC with a

Direct Memory Request.

idblock_adrs  The four-byte data string which is created when the following

data is concatenated: IDBLOCKH_ADR, IDBLOCK_ADR & IDBLOCK_FMT,

representing ROM Bank, Starting Address of the 256-byte Flash

ID Block, and the Block format type, respectively.

SCPBITMAP_x   A bitmapped register indicating status of various EEEC control

flags (see pid_def definitions for bitmap).

scp_odu_ram   This parameter is a PID of single word length which provides

the beginning address of the ODU user RAM area.  This PID will

allow SBDS to download diagnostic routines to the user RAM

area without having to consult the DOC files of the

corresponding strategy.

BITMAP_MON_x  A bitmapped register indicating status of various EEEC

hardware monitor inputs (see pid_def definitions for bitmap).

BITMAP_OFD_x  A bitmapped register indicating status of various EEEC output

fault detection flags. 1 = fault detected (see pid_def

definitions for bitmap).

5-130

SCP INTERFACES, SCP PARAMETER IDENTIFIER (PID) LIST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

AFT1_ADRS     A double-word cal constant which contains the following data:

Starting address (High Byte, Low Byte) of Bank 1 KAM Adaptive

Fuel Table, followed by the total number of bytes in this

table, followed by a byte which indicates the number of rows

in the upper nibble and the special adaptive idle cell format

type in the lower nibble.

FOR INSTANCE:  An AFT1_ADRS of 37F15681 would indicate that KAM

Table 1 starts at RAM address 37F1, has 86 adaptive cells, is

formatted with 8 rows (and 10 columns), and the 6 special

adaptive idle cells are of format type 1.  Idle cell count is

always the difference between total number of cells and product

of rows & columns.  See example below:

AFT2_ADRS     Same as AFT1_ADRS, except it applies to Adaptive Table 2.

AFTn_ADRS Format Example

37h F1h 56h 81h

//  //  //  //

Starting address Upper Byte ____//  //  //  //

Starting address Lower Byte ____//  //  //

Total table cell count _________//  //

Row count Nibble _______________//

Special Cell Type Nibble ______/

AFTn_ADRS Special Adaptive Cell Type Definitions

Type

(Nibble value)     Description

--------------     ------------------------------

0 Hex          No special adaptive cells present.

1 Hex          Special adaptive cells per ISFLAG:

0  ->  Auto in Drive

1  ->  Auto in Drive  +  A/C

2  ->  Manual or Auto in Neutral

3  ->  Manual or Auto in Neutral + A/C

5-131

SCP INTERFACES, SCP PARAMETER IDENTIFIER (PID) LIST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

STRATEGY MODULE: SCP_PID_MAP_CD

BITMAPPED REGISTER DEFINITIONS FOR CDAN2

BEGIN: pid_definitions

This section defines PIDs which are set up for SBDS/EOL

but are NOT part of the regulated J1979 PIDs.

They WILL BE allowed in repetitive data SCP messages.

THE BITMAPS ARE STANDARDIZED FOR SCP

(The bitwise definition of these parameters WILL NOT change, with

the exception of a standardized definition of any 'unused' bits.)

pid_def(BITMAP_FMEM1,                    ;SCP diagnostic PID #1106

b0: 0,             ;TSS FMEM status (TSFMFLG)

b1: VSFMFLG,       ;VS FMEM status

b2: EFMFLG,        ;EGR FMEM status

b3: OFMFLG,        ;ETV FMEM status

b4: MFMFLG,        ;MAF FMEM status

b5: TFMFLG,        ;TP FMEM status

b6: CFMFLG,        ;ECT FMEM status

b7: AFMFLG)        ;ACT FMEM status

pid_def(BITMAP_FMEM2,                    ;SCP diagnostic PID #1107

b0: FFG_CID,       ;CID FMEM status

b1: OSFMFLG,       ;Output Shaft Speed sensor FMEM

b2: TOT_FM_FLG,    ;Trans Oil Temp sensor FMEM

b3: FLG_MLUS_FM,   ;1=unlock due to excessive converter

clutch slip

b4: EGO2FMFLG,     ;EGO21 (bank2 upstream) FMEM status

b5: EGO1FMFLG,     ;EGO11 (bank1 upstream) FMEM status

b6: ADT2FMFLG,     ;Adaptive Table 2 FMEM status

b7: ADT1FMFLG)     ;Adaptive Table 1 FMEM status

pid_def(BITMAP_FMEM3,                    ;SCP diagnostic PID #1689

b0: 0,             ;1=CHT is currently unreliable

(FFG_CHT)

b1: PGM_PFS_FM,    ;1=in PFS sensor FMEM

b2: PGM_CVS_FM,    ;1=in canister not venting FMEM

b3: PGM_TPR_FM,    ;1=in TPR sensor FMEM

b4: 0,             ;1=Fuel Rail Pressure Transducer is

currently unreliable(FFG_FRP)

b5: 0,             ;1=Fuel Rail Temperature Sensor

Bank 1 is currently unreliable

(FFG_FRT1)

b6: 0,             ;1=Fuel Rail Temperature Sensor

Bank 2 is currently unreliable

(FFG_FRT2)

b7: 0)             ;1=Power Steering Pressure

Transducer is currently unreliable

(FFG_PSPT)

5-132

SCP INTERFACES, SCP PARAMETER IDENTIFIER (PID) LIST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BITMAPPED REGISTER DEFINITIONS FOR CDAN2 (continued)

(continue with pid_definitions)

pid_def(SCPBITMAP_1,                  ;SCP diagnostic PID #1101

b0: ACSW,        ;A/C clutch Demand Switch

b1: BIFLG,       ;BRAKE INPUT LEVEL

b2: 0,           ;1=4X4 LOW requested(FLG_4X4L)

b3: NDSFLG,      ;NEUTRAL/DRIVE FLAG 1=DRIVE

b4: ITCS,        ;1=OVERDRIVE CANCEL SWITCH DEPRESSED

b5: PDL_ERROR,   ;PRNDL POSITION ERROR

b6: 0,           ;PERFORMANCE/NORMAL SWITCH(IPES)

b7: 0)           ;1=POWER STEERING LOAD PRESENT(POWSFG)

pid_def(SCPBITMAP_2,                  ;SCP diagnostic PID #1102

b0: ACPSW,       ;AC Medium Pressure Switch Input

b1: 0,           ;Upstream sensor heater(s). 1 output

controls both banks. 0=on(HEGOHTR1)

b2: 0,           ;Downstream sensor heater(s). 1 output

controls both banks. 0=on(HEGOHTR2)

b3: OCTADJ,      ;Field Octane Adjust input

b4: PIP_LVL,     ;1=PIP input level is high

b5: FFG_MISFIRE  ;Misfire Monitor flag, 1 = Misfiring

b6: 0,           ;unused

b7: 0)           ;unused

pid_def(SCPBITMAP_3,                  ;SCP diagnostic PID #1103

b0: 0,           ;Heated Windshield Status Flag(HWFLAG)

b1: 0,           ;1=Purging canister(PRGFLG)

b2: LSF_FLG,     ;1=Low Speed Fan commanded on

b3: HSF_FLG,     ;1=High speed fan commanded on

b4: IMRC_CMND,   ;1=Intake Manifold Runner Control

commanded open

b5: MIL_ON,      ;STATE OF MIL OUTPUT

b6: OLFLG,       ;OPEN LOOP FLAG

b7: TRIP)        ;Flag indicating OBDII trip completed

pid_def(SCPBITMAP_4,                  ;SCP diagnostic PID #1104

b0: ACCFLG,      ;A/C CLUTCH STATUS, 0=disengaged

b1: 0,           ;Ride Control output state (ACL/ACL1)

b2: TCIL_STATE,  ;Current State Of TCIL output

b3: 0,           ;1=unlock torque convertor for

traction control(TRAC_ULFLG)

b4: EAM,         ;Electric Air Management Control Flag

1=pump on

b5: 0,           ;Winter weather switch state(NWSW)

b6: CID_HIGH,    ;Input level of CID sensor

b7: 0)           ;Torque modulation requested flag

(SPK_TQM_REQ)

5-133

SCP INTERFACES, SCP PARAMETER IDENTIFIER (PID) LIST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BITMAPPED REGISTER DEFINITIONS FOR CDAN2 (continued)

(continue with pid_definitions)

pid_def(SCPBITMAP_5,                  ;SCP diagnostic PID #1105

b0: 0,           ;Positive Temp. Coeff. Heater Input

(PTC_FLG)

b1: FLG_OT_LK,   ;1=In Trans Overtemp Lockup Mode

b2: OTEMP_FM_FLG,;1=Transmission overtemp FMEM

b3: 0,           ;1=traction assist is present on this

vehicle(TRAC_OPTION)

b4: FLG_SS_1,    ;1=Shift Solenoid #1 commanded on

b5: FLG_SS_2,    ;1=Shift Solenoid #2 commanded on

b6: 0,           ;1=Shift Solenoid #3 commanded on

(FLG_SS_3)

b7: 0)           ;1=Shift Solenoid #4 (Coast Clutch)

commanded on(FLG_SS_4)

pid_def(SCPBITMAP_6,                  ;SCP diagnostic PID #1631

b0: HEGOHTR11,   ;1=bank1 upstream output on

b1: HEGOHTR12,   ;1=bank1 downstream output on

b2: HEGOHTR21,   ;1=bank2 upstream output on

b3: HEGOHTR22,   ;1=bank2 downstream output on

b4: HTR_FAULT11, ;1=bank1 upstream output fault detected

b5: HTR_FAULT12, ;1=bank1 downstream output fault

detected

b6: HTR_FAULT21, ;1=bank2 upstream output fault detected

b7: HTR_FAULT22) ;1=bank2 downstream output fault

detected

pid_def(SCPBITMAP_7,                  ;SCP diagnostic PID #163C

b0: OL_HOLD,     ;1=open loop - conditions not satisfied

to go closed loop

b1: hego_fdback, ;1=closed loop

b2: OL_DRIVE,    ;1=open loop due to driving conditions

b3: OL_FMEM,     ;1=open loop due to FMEM

b4: hego_fault,  ;1=closed loop with HEGO fault

b5: 0,           ;unused

b6: 0,           ;unused

b7: 0)           ;unused

Note: b0 through b4 of SCPBITMAP_7 correspond to b0 thru b4 of J1979_01_031.

They are defined as part of SCPBITMAP_7 so that they can be referenced when

requesting ANY repetitive data diagnostics.

5-134

SCP INTERFACES, SCP PARAMETER IDENTIFIER (PID) LIST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BITMAPPED REGISTER DEFINITIONS FOR CDAN2 (continued)

(continue with pid_definitions)

pid_def(SCPBITMAP_10,                  ;SCP diagnostic PID #1688

b0: 0,           ;1=cyl head temp (CHT) indicator

light is on(CHTIL_CMD)

b1: 0,           ;1=CHT indicator light output fault

detected(CHTIL_FAULT)

b2: SWC,         ;1=speed warning chime is on

b3: SWC_FAULT,   ;1=speed warning chime output fault

detected

b4: FPUMP_SPEED, ;1=hi speed desired for 2 speed fuel

pump

b5: 0,           ;1=Thermactor Air Bypass is supplying

secondary air (versus dumping it

overboard) for either upstream or

downstream of the forward HO2S(AM1)

b6: 0,           ;1=Thermactor Air Diverter is

diverting secondary air to upstream

(versus downstream) of the forward

HO2S(AM2)

b7: 0)           ;1=secondary spark plugs of Dual Plug

Ignition System are enabled

(DPI_STATE)

5-135

SCP INTERFACES, SCP PARAMETER IDENTIFIER (PID) LIST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BITMAPPED REGISTER DEFINITIONS FOR CDAN2 (continued)

(continue with pid_definitions)

pid_def(BITMAP_MON_1,                   ;SCP diagnostic PID #110C

b0: F_PUMP_S_MON, ;Fuel Pump Secondary Monitor

b1: EAMM,         ;Thermactor Air Pump Monitor

b2: 0,            ;Electro Drive Fan Monitor(EDFM)

b3: 0,            ;High Speed Fan Monitor (HEDFM)

b4: 0,            ;HEGO11 Voltage Monitor(HTRM11)

b5: 0,            ;HEGO12 Voltage Monitor(HTRM12)

b6: 0,            ;HEGO21 Voltage Monitor(HTRM21)

b7: 0)            ;HEGO22 Voltage Monitor(HTRM22)

pid_def(BITMAP_KAM_1,                   ;SCP diagnostic PID #160D

b0: MIS_MON_KAM,  ;Misfire OBD2 Trip flag (complete=1)

b1: 0,            ;1=Intake Manifold Communication

Control valve 1 commanded closed

(IMCC1)

b2: 0,            ;1=Intake Manifold Communication

Control valve 2 commanded closed

(IMCC2)

b3: DOL,          ;state of Data Output Link(DOL)

b4: 0,            ;1=operation of engine is being

disabled by anti-theft system

(ATH_DISABLED)

b5: 0,            ;1=PTO (Power Take Off) is active

(disable affected monitors)

(PTO_ACTIVE)

b6: 0,            ;1=Shift Indicator Light commanded

on(SIL)

b7: 0)            ;1=Shift Indicator Light output

fault detected(SIL_FAULT)

pid_def(BITMAP_OFD_1,                   ;SCP diagnostic PID #162D

b0: INJ1_FAULT,   ;1=injector #1 output fault detected

b1: INJ2_FAULT,   ;1=injector #2 output fault detected

b2: INJ3_FAULT,   ;1=injector #3 output fault detected

b3: INJ4_FAULT,   ;1=injector #4 output fault detected

b4: INJ5_FAULT,   ;1=injector #5 output fault detected

b5: INJ6_FAULT,   ;1=injector #6 output fault detected

b6: INJ7_FAULT,   ;1=injector #7 output fault detected

b7: INJ8_FAULT)   ;1=injector #8 output fault detected

5-136

SCP INTERFACES, SCP PARAMETER IDENTIFIER (PID) LIST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BITMAPPED REGISTER DEFINITIONS FOR CDAN2 (continued)

(continue with pid_definitions)

Assign value to scp_162E_b7 for use in BITMAP_OFD_2

In this strategy/hardware we have a problem with using FP_2SPD_ERR. FPL and

TCIL are on the same output UPOD3-4. FP_2SPD_ERR and TCIL_FAULT are both

defined as the same bit (as the fault bit for UPOD3-4). If the output is not

being used for FPL, and if a fault occurs, then we do not want the SCP PID

#162E bit7 to show a low speed fuel pump output fault. The software flag

FP_2SPD_ERR could be set because there is a fault with the TCIL or because

the unused output is not properly terminated. In these two cases, we do not

want the FP_2SPD_ERR bit of BITMAP_OFD_2 to be set. This could confuse the

technician.

The following logic is intended to force the FP_2SPD_ERR bit of BITMAP_OFD_2

to be zero when UPOD3-4 is not being used to control the speed of the fuel

pump (FPL).

FP_TYPE = 1 ----------------------------| scp_162E_b7 = FP_2SPD_ERR

;2 speed fuel pump hardware is present |  ; 1=low speed fuel pump fault

|  ; detected(FP_2SPD_ERR)

|

| --- ELSE ---

|

| scp_162E_b7 = 0

|  ; set bit to zero if FPL hardware

|  ; is not used.

pid_def(BITMAP_OFD_2,                   ;SCP diagnostic PID #162E

b0: IAC_NO_CUR,   ;1=IAC (ISC-BPA) output fault (open

circuit or short to ground)

detected

b1: IAC_OVER_CUR, ;1=IAC (ISC-BPA) over current output

fault (shorted load or short to

VPWR) detected

b2: EVR_OPEN,     ;1=EVR output open detected

b3: EVR_SHORT,    ;1=EVR output shorted to ground

detected

b4: MIL_FAULT,    ;1=MIL output fault detected

b5: ACC_STATUS,   ;1=ACR output fault detected

b6: F_PUMP_ERROR, ;1=fuel pump output fault detected

b7: scp_162E_b7)  ;1=low speed fuel pump fault

detected(FP_2SPD_ERR)

5-137

SCP INTERFACES, SCP PARAMETER IDENTIFIER (PID) LIST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BITMAPPED REGISTER DEFINITIONS FOR CDAN2 (continued)

(continue with pid_definitions)

Assign value to scp_162F_b2 for use in BITMAP_OFD_3

In this strategy/hardware we have a problem with using CANP_FAULT. CANP and

CANVNT are on the same output SSPOD1-4. CANP_FAULT and CANVT_FAULT are both

defined as the same bit (as the fault bit for SSPOD1-4). If the output is not

being used for CANP, and if the flag CANP_FAULT is set, then we do not want

the SCP PID #162F bit2 to show a canister PURGE output fault. The software

flag CANP_FAULT could be set because there is a fault with the CANVT or

because the unused output is not properly terminated. In these two cases, we

do not want the CANP_FAULT bit of BITMAP_OFD_3 to be set. This could confuse

the technician.

The following logic is intended to force the CANP_FAULT bit of BITMAP_OFD_3

to be zero when SSPOD1-4 is not being used to control the canister PURGE

(CANP).

PGM_SELECT = 1 -------------------------| scp_162F_b2 = CANP_FAULT

;Canister PURGE hardware is present    |  ; 1=canister PURGE output fault

|  ; detected(CANP_FAULT)

|

| --- ELSE ---

|

| scp_162F_b2 = 0

|  ; set bit to zero if Canister

|  ; PURGE hardware is not used.

pid_def(BITMAP_OFD_3,                   ;SCP diagnostic PID #162F

b0: EDF_STATUS,   ;1=low spd fan output fault detected

b1: HEDF_STATUS,  ;1=high speed fan output fault

detected(HEDF_STATUS)

b2: scp_162F_b2,  ;1=canister PURGE output fault

detected(CANP_FAULT)

b3: EAM_FAULT,    ;1=2ndary Air output fault detected

b4: DOL_FAULT,    ;1=DOL/FFDOL output fault detected

(DOL_FAULT)

b5: 0,            ;1=intake manifold communication

control output fault detected

(IMCC_FAULT)

b6: IMRC_FAULT,   ;1=IMRC output fault detected

b7: 0)            ;1=traction control secondary

throttle output fault detected

(TAPW_FAULT)

5-138

SCP INTERFACES, SCP PARAMETER IDENTIFIER (PID) LIST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BITMAPPED REGISTER DEFINITIONS FOR CDAN2 (continued)

(continue with pid_definitions)

Assign value to scp_1630_b3 for use in BITMAP_OFD_4

In this strategy/hardware we have a problem with using CANVT_FAULT. CANVNT

and CANP are on the same output SSPOD1-4. CANVT_FAULT and CANP_FAULT are both

defined as the same bit (as the fault bit for SSPOD1-4). If the output is not

being used for CANVNT, and if the flag CANVT_FAULT is set, then we do not

want the SCP PID #1630 bit3 to show a canister VENT output fault. The

software flag CANVT_FAULT could be set because there is a fault with the CANP

or because the unused output is not properly terminated. In these two cases,

we do not want the CANVT_FAULT bit of BITMAP_OFD_4 to be set. This could

confuse the technician.

The following logic is intended to force the CANVT_FAULT bit of BITMAP_OFD_4

to be zero when SSPOD1-4 is not being used to control the canister VENT

(CANVNT).

PGM_SELECT = 0 -------------------------| scp_1630_b3 = CANVT_FAULT

;Canister VENT hardware is present     |  ; 1=canister VENT output fault

|  ; detected(CANVT_FAULT)

|

| --- ELSE ---

|

| scp_1630_b3 = 0

|  ; set bit to zero if Canister

|  ; VENT hardware is not used.

pid_def(BITMAP_OFD_4,                   ;SCP diagnostic PID #1630

b0: 0,            ;1=variable cam timing 1 output

fault detected(VCT1OSM)

b1: 0,            ;1=variable cam timing 2 output

fault detected(VCT2OSM)

b2: 0,             ;1=ride control output fault

detected(ACLOSM)

b3: scp_1630_b3,  ;1=canister VENT output fault

detected(CANVT_FAULT)

b4: 0,            ;unused

b5: 0,            ;unused

b6: 0,            ;unused

b7: 0)            ;unused

5-139

SCP INTERFACES, SCP PARAMETER IDENTIFIER (PID) LIST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BITMAPPED REGISTER DEFINITIONS FOR CDAN2 (continued)

(continue with pid_definitions)

Assign value to scp_1632_b7 for use in BITMAP_OFD_5

In this strategy/hardware we have a problem with using TCIL_FAULT. TCIL and

FPL are on the same output UPOD3-4. TCIL_FAULT and FP_2SPD_ERR are both

defined as the same bit (as the fault bit for UPOD3-4). If the output is not

being used for TCIL, and if the flag TCIL_FAULT is set, then we do not want

the SCP PID #1632 bit7 to show a transmission control indicator light output

fault. The software flag TCIL_FAULT could be set because there is a fault

with the FPL or because the unused output is not properly terminated. In

these two cases, we do not want the TCIL_FAULT bit of BITMAP_OFD_5 to be

set. This could confuse the technician.

The following logic is intended to force the TCIL_FAULT bit of BITMAP_OFD_5

to be zero when UPOD3-4 is not being used to control the transmission control

indicator light (TCIL).

TSTRAT = 7 -----------------------------| scp_1632_b7 = TCIL_FAULT

;AOD-E (AOD-I) automatic transmission  |  ; 1=TCIL output fault detected

|  ; (TCIL_FAULT)

|

| --- ELSE ---

|

| scp_1632_b7 = 0

|  ; set bit to zero if TCIL hardware

|  ; is not used.

pid_def(BITMAP_OFD_5,                   ;SCP diagnostic PID #1632

b0: SS1_FAULT,    ;1=SS1 output fault detected

b1: SS2_FAULT,    ;1=SS2 output fault detected

b2: 0,            ;1=SS3 output fault detected

(SS3_FAULT)

b3: 0,            ;1=SS4 output fault detected

(SS4_FAULT)

b4: 0,            ;unused

b5: 0,            ;unused

b6: MCC_H_FAULT,  ;1=modulated converter clutch output

fault detected

b7: scp_1632_b7)  ;1=TCIL output fault detected

(TCIL_FAULT)

Assign values to idblock_adrs.

unconditionally ------------| IDBLOCK_FMT := (Contents of memory location

|                 at address (IDBLOCK_ADR+13h))

| ; See definitions for data description

pid_def(idblock_adrs:  IDBLOCKH_ADR,  IDBLOCK_ADR,  IDBLOCK_FMT)

;ROM Bank     ADRmid,ADRlo  Block Format

END: pid_definitions

5-140

SCP INTERFACES, SCP PARAMETER IDENTIFIER (PID) LIST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

J1979 SCP PID MAP - CDAN2

(NOT TO BE USED IN RAPID DATA TRANSFER)

|      | pid_def() or |         OUTPUT          |  OUTPUT   |    DATA   |

| PID# |PARAMETER NAME|       RESOLUTION        |  SCALING  |    TYPE   |

|------|--------------|-------------------------|-----------|-----------|

| 0000 | j1979_01_00  |         bitmap          |    N/A    |double word|

|------|--------------|-------------------------|-----------|-----------|

| 0001 | j1979_01_011 |         bitmap          |    N/A    |   byte    |

|      | j1979_01_012 |         bitmap          |           |   byte    |

|      | j1979_01_013 |         bitmap          |           |   byte    |

|      | j1979_01_014 |         bitmap          |           |   byte    |

|------|--------------|-------------------------|-----------|-----------|

| 0003 | j1979_01_031 |         bitmap          |    N/A    |   byte    |

|      |    00 hex    |                         |           |   byte    |

|------|--------------|-------------------------|-----------|-----------|

| 0004 | j1979_01_04  |    100/255 % load       |    N/A    |   byte    |

|------|--------------|-------------------------|-----------|-----------|

| 0005 | j1979_01_05  |       1 deg C           |    N/A    |   byte    |

|------|--------------|-------------------------|-----------|-----------|

| 0006 | j1979_01_06  |     100/128 % rich      |    N/A    |   byte    |

|------|--------------|-------------------------|-----------|-----------|

| 0007 | j1979_01_07  |     100/128 % rich      |    N/A    |   byte    |

|------|--------------|-------------------------|-----------|-----------|

| 0008 | j1979_01_08  |     100/128 % rich      |    N/A    |   byte    |

|------|--------------|-------------------------|-----------|-----------|

| 0009 | j1979_01_09  |     100/128 % rich      |    N/A    |   byte    |

|------|--------------|-------------------------|-----------|-----------|

| 000C | j1979_01_0C  |       0.25 rpm          |    N/A    |   word    |

|------|--------------|-------------------------|-----------|-----------|

| 000D | j1979_01_0D  |          1 kph          |    N/A    |   byte    |

|------|--------------|-------------------------|-----------|-----------|

| 000E | j1979_01_0E  |   0.5 deg spark advance |    N/A    |   byte    |

|------|--------------|-------------------------|-----------|-----------|

| 000F | j1979_01_0F  |        1 deg C          |    N/A    |   byte    |

|------|--------------|-------------------------|-----------|-----------|

| 0010 | j1979_01_10  |      0.01 gm/sec        |    N/A    |   word    |

|------|--------------|-------------------------|-----------|-----------|

| 0011 | j1979_01_11  |    100/255 % throttle   |    N/A    |   byte    |

|------|--------------|-------------------------|-----------|-----------|

| 0012 | j1979_01_12  |         bitmap          |    N/A    |   byte    |

|------|--------------|-------------------------|-----------|-----------|

| 0013 | j1979_01_13  |         bitmap          |    N/A    |   byte    |

|------|--------------|-------------------------|-----------|-----------|

| 0014 | j1979_01_14  |      0.005 volts        |    N/A    |   byte    |

|      | j1979_01_06  |     100/128 % rich      |           |   byte    |

|------|--------------|-------------------------|-----------|-----------|

| 0015 | j1979_01_151 |      0.005 volts        |    N/A    |   byte    |

|      | j1979_01_152 |     100/128 % rich      |           |   byte    |

|------|--------------|-------------------------|-----------|-----------|

| 0018 | j1979_01_18  |      0.005 volts        |    N/A    |   byte    |

|      | j1979_01_08  |     100/128 % rich      |           |   byte    |

|------|--------------|-------------------------|-----------|-----------|

| 0019 | j1979_01_191 |      0.005 volts        |    N/A    |   byte    |

|      | j1979_01_192 |     100/128 % rich      |           |   byte    |

|------|--------------|-------------------------|-----------|-----------|

5-141

SCP INTERFACES, SCP PARAMETER IDENTIFIER (PID) LIST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

J1979 SCP PID MAP - CDAN2 (continued)

(NOT TO BE USED IN RAPID DATA TRANSFER)

|      | pid_def() or |         OUTPUT          |  OUTPUT   |    DATA   |

| PID# |PARAMETER NAME|       RESOLUTION        |  SCALING  |    TYPE   |

|------|--------------|-------------------------|-----------|-----------|

| 001C | j1979_01_1C  |           1.0           | Bin   0   |   byte    |

|------|--------------|-------------------------|-----------|-----------|

5-142

SCP INTERFACES, SCP PARAMETER IDENTIFIER (PID) LIST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

J1979 FREEZE FRAME PID MAP - CDAN2

(NOT TO BE USED IN RAPID DATA TRANSFER)

|      | pid_def() or |         OUTPUT          |  OUTPUT   |    DATA   |

| PID# |PARAMETER NAME|       RESOLUTION        |  SCALING  |    TYPE   |

|------|--------------|-------------------------|-----------|-----------|

| 0000 | j1979_02_00  |         bitmap          |    N/A    |double word|

|------|--------------|-------------------------|-----------|-----------|

| 0002 | j1979_02_02  |    trouble code map     |    N/A    |   word    |

|------|--------------|-------------------------|-----------|-----------|

| 0003 | j1979_02_031 |         bitmap          |    N/A    |   byte    |

|      |    00 hex    |                         |           |   byte    |

|------|--------------|-------------------------|-----------|-----------|

| 0004 | j1979_02_04  |    100/255 % load       |    N/A    |   byte    |

|------|--------------|-------------------------|-----------|-----------|

| 0005 | j1979_02_05  |       1 deg C           |    N/A    |   byte    |

|------|--------------|-------------------------|-----------|-----------|

| 0006 | j1979_02_06  |     100/128 % rich      |    N/A    |   byte    |

|------|--------------|-------------------------|-----------|-----------|

| 0007 | j1979_02_07  |     100/128 % rich      |    N/A    |   byte    |

|------|--------------|-------------------------|-----------|-----------|

| 0008 | j1979_02_08  |     100/128 % rich      |    N/A    |   byte    |

|------|--------------|-------------------------|-----------|-----------|

| 0009 | j1979_02_09  |     100/128 % rich      |    N/A    |   byte    |

|------|--------------|-------------------------|-----------|-----------|

| 000C | j1979_02_0C  |       0.25 rpm          |    N/A    |   word    |

|------|--------------|-------------------------|-----------|-----------|

| 000D | j1979_02_0D  |          1 kph          |    N/A    |   byte    |

|------|--------------|-------------------------|-----------|-----------|

5-143

SCP INTERFACES, SCP PARAMETER IDENTIFIER (PID) LIST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SCP DIAGNOSTIC PID MAP - CDAN2

|      | pid_def() or |         OUTPUT          |  OUTPUT   |    DATA   |

| PID# |PARAMETER NAME|       RESOLUTION        |  SCALING  |    TYPE   |

|------|--------------|-------------------------|-----------|-----------|

| 0100 | TRIP_COUNT   |   1 OBDII Trip/count    | Bin   0   |  B    U   |

| 0101 | DRIVE_COUNT  | 1 OBDII Drive Cycle/cnt | Bin   0   |  W    U   |

| 0200 | CODES_COUNT  |    1 Code per count     | Bin   0   |  B    U   |

| 0202 | OD_CODE_CNT  |    1 Code per count     | Bin   0   |  B    U   |

| 1100 | idblock_adrs |   ASCII / Hex values    | Bin   0   |  DW   U   |

| 1101 | SCPBITMAP_1  |         Bitmap          | Bin   0   |  B    U   |

| 1102 | SCPBITMAP_2  |         Bitmap          | Bin   0   |  B    U   |

| 1103 | SCPBITMAP_3  |         Bitmap          | Bin   0   |  B    U   |

| 1104 | SCPBITMAP_4  |         Bitmap          | Bin   0   |  B    U   |

| 1105 | SCPBITMAP_5  |         Bitmap          | Bin   0   |  B    U   |

| 1106 | BITMAP_FMEM1 |         Bitmap          | Bin   0   |  B    U   |

| 1107 | BITMAP_FMEM2 |         Bitmap          | Bin   0   |  B    U   |

| 110C | BITMAP_MON_1 |         Bitmap          | Bin   0   |  B    U   |

| 1123 | ACT          |       2 degrees F       | Bin  -1   |  B    S   |

| 1125 | APT          |     -1,0,1 unitless     | Bin   0   |  B    S   |

| 1126 | ATMR1        |       1 second          | Bin   0   |  B    U   |

| 1127 | BP           |    0.125 inches Hg      | Bin   3   |  B    U   |

| 1135 | DSDRPM       |        16 rpm           | Bin  -4   |  B    U   |

| 1139 | ECT          |       2 degrees F       | Bin  -1   |  B    S   |

| 113C | EGRDC        |100/32768 % on duty cycle| Bin  15   |  W    U   |

| 1141 | FUELPW1      |    32 clock tics        | Bin  -1   |  W    U   |

| 1142 | FUELPW2      |    32 clock tics        | Bin  -1   |  W    U   |

| 114A | ACT_CNTS     |    0.0156 A/D count     | Bin   6   |  W    U   |

| 114D | ECT_CNTS     |    0.0156 A/D count     | Bin   6   |  W    U   |

| 114E | IEGR         |    0.0156 A/D count     | Bin   6   |  W    U   |

| 1151 | INDS         |    0.0156 A/D count     | Bin   6   |  W    U   |

| 1153 | ISCDTY       |100/32768 % on duty cycle| Bin  15   |  W    U   |

| 1154 | TP_ENG       |    0.0156 A/D count     | Bin   6   |  W    U   |

| 1156 | KAMRF1       |   0.00391 A/F ratio     | Bin   8   |  W    U   |

| 1157 | KAMRF2       |   0.00391 A/F ratio     | Bin   8   |  W    U   |

| 1158 | LAMBSE1      |   1/32768 unitless      | Bin  15   |  W    U   |

| 1159 | LAMBSE2      |   1/32768 unitless      | Bin  15   |  W    U   |

| 115A | LOAD         | 1/32768 of std air chrg | Bin  15   |  W    U   |

| 1165 | N            |        0.25 rpm         | Bin   2   |  W    U   |

| 1166 | PG_DC        |100/32768 % on duty cycle| Bin  15   |  W    U   |

| 1167 | PGM_CVS_DC   |     1 EEEC-IV count     | Bin  15   |  W    U   |

| 1169 | RATCH        |    0.0156 count         | Bin   6   |  W    U   |

| 116B | SAFTOT       |   0.25 degrees S.A.     | Bin   2   |  W    S   |

| 1172 | VBAT         |      0.0625 volts       | Bin   4   |  B    U   |

| 1173 | VEGO11       |    0.000977 volts       | Bin  10   |  W    U   |

| 1174 | VEGO12       |    0.000977 volts       | Bin  10   |  W    U   |

| 1175 | VEGO21       |    0.000977 volts       | Bin  10   |  W    U   |

| 1176 | VEGO22       |    0.000977 volts       | Bin  10   |  W    U   |

| 11B0 | BCSDC_OUT    |100/32768 % on duty cycle| Bin  15   |  W    U   |

| 11B2 | EPCOCM1      |    0.0156 volts         | Bin   6   |  W    U   |

| 11B3 | GR_CM        |         N/A             | Bin   1   |  B    U   |

|------|--------------|-------------------------|-----------|-----------|

5-144

SCP INTERFACES, SCP PARAMETER IDENTIFIER (PID) LIST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SCP DIAGNOSTIC PID MAP (continued) - CDAN2

|      | pid_def() or |         OUTPUT          |  OUTPUT   |    DATA   |

| PID# |PARAMETER NAME|       RESOLUTION        |  SCALING  |    TYPE   |

|------|--------------|-------------------------|-----------|-----------|

| 11B5 | NO           |        0.25 rpm         | Bin   2   |  W    U   |

| 11B6 | PDL          |         N/A             | Bin   1   |  B    U   |

| 11B7 | RT_GR_CUR    |         N/A             | Bin  14   |  W    U   |

| 11B8 | SLIP_ABS     |       0.25 rpm          | Bin   2   |  W    U   |

| 11BA | SPD_RATIO    |         N/A             | Bin  14   |  W    U   |

| 11BD | TOT_CNTS     |    0.0156 A/D count     | Bin   6   |  W    U   |

| 11C0 | TV_PRES_BAR  |          0.5  psi       | Bin   1   |  B    U   |

| 11C1 | VS           |     0.001953  mph       | Bin   9   |  W    U   |

| 1600 | MAF_FIL_MAX  |     1 fault count       | Bin   0   |  B    U   |

| 1601 | ACT_FIL_MAX  |     1 fault count       | Bin   0   |  B    U   |

| 1602 | ECT_FIL_MAX  |     1 fault count       | Bin   0   |  B    U   |

| 1603 | TP_FIL_MAX   |     1 fault count       | Bin   0   |  B    U   |

| 1604 | VS_FIL_MAX   |     1 fault count       | Bin   0   |  B    U   |

| 1605 | HTR_RET_MX11 |     1 retry count       | Bin   0   |  B    U   |

| 1606 | HTR_RET_MX12 |     1 retry count       | Bin   0   |  B    U   |

| 1607 | HTR_RET_MX21 |     1 retry count       | Bin   0   |  B    U   |

| 1608 | HTR_RET_MX22 |     1 retry count       | Bin   0   |  B    U   |

| 160B | AFT1_ADRS    |       HEX values        | Bin   0   |  DW   U   |

| 160C | AFT2_ADRS    |       HEX values        | Bin   0   |  DW   U   |

| 160D | BITMAP_KAM_1 |        Bitmapped        | Bin   0   |  B    U   |

| 160E | nummis1      |     1 misfire count     | Bin   0   |  W    U   |*

| 160F | nummis2      |     1 misfire count     | Bin   0   |  W    U   |*

| 1610 | nummis3      |     1 misfire count     | Bin   0   |  W    U   |*

| 1611 | nummis4      |     1 misfire count     | Bin   0   |  W    U   |*

| 1612 | nummis5      |     1 misfire count     | Bin   0   |  W    U   |*

| 1613 | nummis6      |     1 misfire count     | Bin   0   |  W    U   |*

| 1614 | nummis7      |     1 misfire count     | Bin   0   |  W    U   |*

| 1615 | nummis8      |     1 misfire count     | Bin   0   |  W    U   |*

| 1616 | nummis       |     1 misfire count     | Bin   0   |  W    U   |*

| 1617 | totevents    |     1 cylinder event    | Bin   0   |  W    U   |*

| 1618 | mis_nocall   |     1 cylinder event    | Bin   0   |  W    U   |*

| 1619 | nummis1_h    |     1 misfire count     | Bin   0   |  B    U   |*

| 161A | nummis2_h    |     1 misfire count     | Bin   0   |  B    U   |*

| 161B | nummis3_h    |     1 misfire count     | Bin   0   |  B    U   |*

| 161C | nummis4_h    |     1 misfire count     | Bin   0   |  B    U   |*

| 161D | nummis5_h    |     1 misfire count     | Bin   0   |  B    U   |*

| 161E | nummis6_h    |     1 misfire count     | Bin   0   |  B    U   |*

| 161F | nummis7_h    |     1 misfire count     | Bin   0   |  B    U   |*

| 1620 | nummis8_h    |     1 misfire count     | Bin   0   |  B    U   |*

| 1621 | nummis_h     |     1 misfire count     | Bin   0   |  B    U   |*

| 1622 | totevents_h  |     1 cylinder event    | Bin   0   |  B    U   |*

| 1623 | mis_nocall_h |     1 cylinder event    | Bin   0   |  B    U   |*

| 1627 | PFS_CNTS     |    0.0156 A/D count     | Bin   6   |  W    U   |

| 162B | NTBART       |       0.25 rpm          | Bin   2   |  W    U   |

|------|--------------|-------------------------|-----------|-----------|

NOTE:  The *'d PIDs exist in the strategy as three byte parameters.  They

are represented here as separate PIDs since the EEEC does not support

three-byte PIDs.  The parameters are identified in VECTOR by their

word PID names.

5-145

SCP INTERFACES, SCP PARAMETER IDENTIFIER (PID) LIST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SCP DIAGNOSTIC PID MAP (continued) - CDAN2

|      | pid_def() or |         OUTPUT          |  OUTPUT   |    DATA   |

| PID# |PARAMETER NAME|       RESOLUTION        |  SCALING  |    TYPE   |

|------|--------------|-------------------------|-----------|-----------|

| 162D | BITMAP_OFD_1 |        Bitmapped        | Bin   0   |  B    U   |

| 162E | BITMAP_OFD_2 |        Bitmapped        | Bin   0   |  B    U   |

| 162F | BITMAP_OFD_3 |        Bitmapped        | Bin   0   |  B    U   |

| 1630 | BITMAP_OFD_4 |        Bitmapped        | Bin   0   |  B    U   |

| 1631 | SCPBITMAP_6  |        Bitmapped        | Bin   0   |  B    U   |

| 1632 | BITMAP_OFD_5 |        Bitmapped        | Bin   0   |  B    U   |

| 1633 | IMAF         |    0.0156 A/D count     | Bin   6   |  W    U   |

| 1634 | IMRCP1_CNTS  |    0.0156 A/D count     | Bin   6   |  W    U   |

| 1636 | VMV_OSM_CNTS |    0.0156 A/D count     | Bin   6   |  W    U   |

| 1638 | IACPRES_SEN  |    0.0156 A/D count     | Bin   6   |  W    U   |

| 1639 | TPR_CNTS     |    0.0156 A/D count     | Bin   6   |  W    U   |

| 163A | TQ_NET       |         2 ft-lbs        | Bin  -1   |  B    U   |

| 163C | SCPBITMAP_7  |        Bitmapped        | Bin   0   |  B    U   |

| 163E | R_BIAS1      |     1/8192 unitless     | Bin  13   |  W    S   |

| 163F | R_BIAS2      |     1/8192 unitless     | Bin  13   |  W    S   |

| 1671 | j1979_01_10  |0.01 gram of air mass/sec| Bin   0   |  W    U   |

| 1672 | FPUMP_DC     |100/32768 % on duty cycle| Bin  15   |  W    U   |

| 1674 | TOT_ENG      |     0.125 degrees F     | Bin   3   |  W    S   |

| 1675 | far_mul_1    |       HEX values        | Bin   0   |  W    U   |

| 1676 | far_mul_2    |       HEX values        | Bin   0   |  W    U   |

| 1677 | SLIP_DES     |       0.0625  rpm       | Bin   4   |  W    U   |

| 1679 | HTRCM11      |       1/256 Amps        | Bin   8   |  W    U   |

| 167A | HTRCM12      |       1/256 Amps        | Bin   8   |  W    U   |

| 167B | HTRCM21      |       1/256 Amps        | Bin   8   |  W    U   |

| 167C | HTRCM22      |       1/256 Amps        | Bin   8   |  W    U   |

| 1687 | PGM_TANK_PRS |   1/512 inches of H2O   | Bin   9   |  W    S   |

| 1688 | SCPBITMAP_10 |        Bitmapped        | Bin   0   |  B    U   |

| 1689 | BITMAP_FMEM3 |         Bitmap          | Bin   0   |  B    U   |

| 1695 | S_VEGO11     |     0.000977 volts      | Bin  10   |  W    S   |

| 1696 | S_VEGO21     |     0.000977 volts      | Bin  10   |  W    S   |

| 1699 | S_VEGO12     |     0.000977 volts      | Bin  10   |  W    S   |

| 169A | S_VEGO22     |     0.000977 volts      | Bin  10   |  W    S   |

| E100 | scp_odu_ram  |     Hex Address Values  | Bin   0   |  W    U   |

|------|--------------|-------------------------|-----------|-----------|

NOTE:  The *'d PIDs exist in the strategy as three byte parameters.  They

are represented here as separate PIDs since the EEEC does not support

three-byte PIDs.  The parameters are identified in VECTOR by their

word PID names.

5-146

CHAPTER 6

CRANK/UNDERSPEED/RUN MODE SELECTION STRATEGY

6-1

CRANK/UNDERSPEED/RUN MODE SELECTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

6.1  CRANK/UNDERSPEED/RUN MODE SELECTION (CDAI0)

OVERVIEW

The EEC IV  strategy  operation  is  divided  into  three  distinct  strategy

segments.  These are:

1)  CRANK

2)  UNDERSPEED

3)  RUN

The CRANK mode is entered after a power-up initialization or after an  engine

stall.   CRANK  employs  a special strategy to aid engine starting.  When the

CRANK logic first  becomes  false,  the  UNDERSPEED  mode  is  entered.   The

UNDERSPEED  mode  employs  a  special spark and fuel strategy in place of the

normal engine control strategy (RUN).  After start, the RUN mode  is  entered

and  the  normal engine control strategy is executed.  If the engine stumbles

during RUN mode, the UNDERSPEED mode can again be  entered  to  help  recover

from the stumble and prevent a stall.

The specific strategies are:

CRANK STRATEGY

Fuel                     Fire all injector ports simultaneously

every CRKPIP PIPS. See the Fuel strategy.

Spark Advance            10 degrees BTDC (on PIP signal)

Thermactor Air           bypass

EGR                      disabled

Purge                    disabled

ISC                      FN884(TCSTRT)

Data Output Link         execute strategy

A/C Clutch               disabled

S.I.L.                   disabled

Thermactor Pump Clutch   disabled

6-2

CRANK/UNDERSPEED/RUN MODE SELECTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

UNDERSPEED STRATEGY

Fuel                     Fire all injector ports in the

same manner as in the RUN mode.

An additional multiplier (FN387

or FN387A) is included in the

pulsewidth equation. See the

Fuel Background Pulsewidth module.

Spark Advance            10 degrees BTDC (on PIP signal)

Other outputs are the same as the RUN mode.

RUN STRATEGY

The normal engine control strategy is described  in  the  remainder  of  this

book.

6-3

CRANK/UNDERSPEED/RUN MODE SELECTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  ACSTRT = Temperature of ACT at startup, deg F.

-  ECT = Engine coolant temperature, deg F.

-  ECTCNT = Number of times ECT sensor input was read.

-  N = Engine RPM.

-  PIPCNT = Number of PIPs which have occurred.

-  RUNUPTMR = Time since runup rpm exceeded (0.125 secs).

-  TCSTRT = Temperature of Engine Coolant at Cold Startup, deg F.

-  TOT = Transmission oil temperature.

-  TSLPIP = Time since last PIP.

Bit Flags:

-  CRKFLG = Engine Mode Flag; 1 -> Crank Mode, 0 <> Crank Mode.

-  PTSCR = Part throttle since crank mode flag; 0 -> driver has  not  tipped

in since start, 1 -> driver tipped in, kick down desired RPM.

-  UNDSP = Run/Underspeed Flag; 1 -> Underspeed (or CRANK), 0 -> Run.

Calibration Constants:

-  CRKPIP = Number of PIPs between injector outputs during Crank.

-  FN384(ECT) = Maximum engine speed to exit Crank versus ECT.

-  FN385(ECT) = Underspeed-to-run transition as a function of ECT.

-  FN387A = Fuelpw multiplier versus ECT;  purpose:   underspeed  mode  fuel

control; Input = ECT.

-  FN884(TCSTRT) = ISC Duty Cycle in Crank, deg.

-  NCNT = Minimum number of PIP necessary to exit CRANK Mode.

-  NSTALL = Engine Stall speed to re-enter CRANK Mode.

-  UNRPMH = Hysteresis term for UNDERSPEED Mode.

6-4

CRANK/UNDERSPEED/RUN MODE SELECTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OUTPUTS

Registers:

-  ACSTRT_ACCUM = ACSTRT accumulator, deg F.

-  ECTCNT = See above.

-  INIT_TOT = TOT at  start-up;  arithmetic  average  of  the  first  8  TOT

reading.

-  N = See above.

-  N_RUN = Engine RPM to exit Crank mode [=FN384(ECT)].

-  PIPCNT = See above.

-  TCSTRT_ACCUM = TCSTRT accumulator, deg F.

Bit Flags:

-  AIR_CRK_INT =  Flag  that  indicates  if  the  air  registers  have  been

initialize; 0 -> registers not initialiezed yet, 1 -> registers have been

initialiezed.

-  AIR_PIP_CNT = Counter that counts the first four pip edges.

-  CRKFLG = See above.

-  FIRST_PIP = Indicates that first PIP has been received.

-  FLG_STALL =  Flag  indicating  a  stall  has  occurred;  transition  from

underspeed/run to crank.

-  RUNUP_FLG = Flag indicating that initial runup is complete;  1  ->  Runup

complete.

-  UNDSP = Run/Underspeed Flag; 1 -> Underspeed (or CRANK), 0 -> Run.

6-5

CRANK/UNDERSPEED/RUN MODE SELECTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CRK_UNDSP_RUN_COM2

Always -------------------------------------| FLG_STALL = 0

| N_RUN = FN384(ECT)

CRKFLG = 1 ---------------------------|     | (Crank Mode)

|AND -| PIPCNT = 0

N <= N_RUN ---------------------------|     | PTSCR = 0

| UNDSP = 1

|

| --- ELSE ---

CRKFLG = 1 ---------------------------|     |

|     |

N > N_RUN ----------------------------|     | (Exit Crank)

|AND -| CRKFLG = 0

ECTCNT >= 8 --------------------------|     | UNDSP = 1

|     |

PIPCNT >= NCNT -----------------------|     |

| --- ELSE ---

|

CRKFLG = 1 ---------------------------|     | (Crank Mode)

|AND -| UNDSP = 1

N > N_RUN ----------------------------|     |

| --- ELSE ---

|

| (Engine stall)

N < NSTALL ---------------------------------| PIPCNT = 0

| CRKFLG = 1

| FLG_STALL = 1

| ECTCNT = 0

| PTSCR = 0

| TCSTRT_ACCUM = 0

| UNDSP = 1

| ACSTRT_ACCUM = 0

| INIT_TOT = 0

| RUNUP_FLG = 0

| RUNUPTMR = 0

|

| --- ELSE ---

|

| (Enter Underspeed)

N < FN385(ECT) -----------------------------| PIPCNT = 0

| UNDSP = 1

|

| --- ELSE ---

(continued on next page)

6-6

CRANK/UNDERSPEED/RUN MODE SELECTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

N > (FN385(ECT) + UNRPMH) ------------|     |

|AND -| (Enter Run)

UNDSP = 1 ----------------------------|     | PIPCNT = 0

| UNDSP = 0

|

| --- ELSE ---

|

| (No Change)

| PIPCNT = 0

TSLPIP >= 800 msec -------------------------| N = 0

| FIRST_PIP = 0

| AIR_CRK_INT = 0

| AIR_PIP_CNT = 0

6-7

CRANK/UNDERSPEED/RUN MODE SELECTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

6-8

CHAPTER 7

THROTTLE MODE SELECTION STRATEGY

7-1

THROTTLE MODE SELECTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

7.1  THROTTLE MODE SELECTION STRATEGY (CDAI0)

OVERVIEW

The throttle mode scheduler is used to determine what engine operating region

is  currently  extant.   The  variable APT (At Part Throttle flag) is used to

indicate throttle mode and is assigned the following values:

Throttle Mode            APT

------------------       ---

CLOSED THROTTLE          -1

PART THROTTLE             0

WIDE OPEN THROTTLE        1

The value of APT is determined by the logic  shown  on  the  following  page.

Briefly,  throttle  angle breakpoints, in terms of counts, are used to define

the CLOSED/PART_THROTTLE and PART/WIDE_OPEN_THROTTLE transitions.  Hysteresis

is incorporated in both breakpoints to prevent jitter between modes.

TP_REL is a parameter which indicates the amount of throttle movement  beyond

the  closed  throttle/idle  setting.  TP_REL is computed by subtracting RATCH

from TP.  Larger values of  TP_REL  indicates  wide  open  throttle,  smaller

values  of  TP_REL  indicates  part  throttle, and near zero TP_REL indicates

closed throttle.

The variable RATCH is the output of a ratchet  algorithm  which  continuously

seeks the minimum throttle angle corresponding to a CLOSED THROTTLE position.

This alleviates the necessity to set  the  throttle  position  sensor  at  an

absolute  position and compensates for system changes and differences between

vehicles.  The ratchet algorithm uses  filtered  throttle  position  for  the

determination of RATCH.

A more detailed explanation of the throttle position  ratchets  and  throttle

position filter is contained in the INPUT CONVERSIONS AND FILTERS chapter.

7-2

THROTTLE MODE SELECTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  APT = Status of Part Throttle; -1 -> Closed  Throttle,  1  ->  Wide  Open

Throttle, 0 -> Part Throttle.

-  TP_REL = Relative Throttle position, counts.

Bit Flag:

-  CRKFLG = Flag indicating engine mode; 1 -> Crank.

Calibration Constants:

-  DELTA = CT/PT Breakpoint value above RATCH.

-  HYST2 = Hysteresis term to enter WOT mode.

-  HYSTS = A/D counts, Closed Throttle to Part Throttle hysteresis value.

-  THBP2 = PT/WOT Breakpoint value above RATCH.

OUTPUTS

Registers:

-  APT = See inputs above.

Bit Flags:

-  CL_THRTL = Indicates that the throttle is closed; 1 -> closed throttle, 0

-> part throttle or wide open throttle.

-  CTPTFG = Closed throttle to PT/WOT transition flag.

-  PTSCR = Part throttle mode since exiting CRANK Flag.

7-3

THROTTLE MODE SELECTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

THROTTLE MODE SELECTION LOGIC

The logic described below considers the current position of the throttle  and

compares its value to the RATCH, Closed Throttle, plus the change in throttle

position from the last setting.  If both flip-flops in the logic clear,  then

Part Throttle is set.

PROCESS

STRATEGY MODULE: THROT_COM1

TP_REL <= DELTA ----------------------|S  Q-| Closed Throttle mode

|     | APT = -1

TP_REL > DELTA + HYSTS ---------------|C    | CL_THRTL = 1

|

| --- ELSE ---

|

TP_REL > THBP2 + HYST2 ---------------|S  Q-| Wide Open Throttle

|     | mode

TP_REL <= THBP2 ----------------------|C    | APT = 1

| CL_THRTL = 0

|

| --- ELSE ---

|

| Part Throttle Mode

| APT = 0

| CL_THRTL = 0

NOTE: PTSCR is initialized to 0.

previous APT = -1 --------------------|

|

current APT <> -1 --------------------|AND -| CTPTFG = 1

|     | Closed Throttle to

CRKFLG = 1 ---------------------------|     | Part Throttle transition

|

| --- ELSE ---

previous APT = -1 --------------------|     |

|     |

current APT <> -1 --------------------|AND -| CTPTFG = 1

|     | Closed Throttle to

CRKFLG = 0 ---------------------------|     | Part Throttle transition

| PTSCR = 1

|

| --- ELSE ---

|

| CTPTFG = 0

7-4

CHAPTER 8

FUEL STRATEGY

8-1

FUEL STRATEGY, OVERVIEW - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.1  FUEL STRATEGY OVERVIEW (CDAI0)

The A/F ratio control actuators consist  of  injectors  whose  fuel  metering

function is affected by energizing and de-energizing the injector solenoids.

Each injector has a metering needle or  pintle  which  opens  or  closes  the

injector nozzle to release fuel.

A high pressure fuel pump delivers fuel to the injectors at approximately  42

PSI.

Based upon the  calculated  air  mass  value,  the  software  calculates  the

injector pulsewidths required to give the desired A/F ratio.

The desired A/F ratio for all operating conditions is determined by  the  A/F

strategy and calibration.

The strategy is designed to handle any reasonable injector configuration  and

firing patterns.  Example configurations are:

1.  1 or 2 output CFI

2.  1 or 2 output bank EFI

3.  4, 6 or 8 output Sequential EFI

The strategy can  run  on  4,  6  or  8-cylinder  engines.   The  calibration

parameters ENGCYL and NUMCYL control the engine type.  The user must set both

parameters for the strategy to work correctly.

On SEFI applications, each cylinder has an injector  located  in  the  intake

port near the intake valve.  The injectors are individually fired in an order

that matches the firing order of the engine.   The  injector  output  numbers

correspond   to   engine   cylinder  numbers.   This  allows  for  consistent

nomenclature for the module pinouts and the  wiring  harness.   Under  normal

engine  running,  each injection is timed to occur at an optimum point in the

intake event.  Injector timing is determined by strategy and calibration.

Timed sequential fuel injection requires identification of cylinder #1.

8-2

FUEL STRATEGY, OVERVIEW - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

STEREO EGO FUEL SYSTEM CONFIGURATION

The following Side 1 and Side 2 designations are used.

|                                                      |

|                                                      |

|                                                      |

|       EGO SENSOR ----- E     E -----    EGO SENSOR   |

|       EGO-1                             EGO-2        |

|       HEGO-1                            HEGO-2       |

|                                                      |

|                                                      |

|                                                      |

|                        -------                       |

|                       | I   I |                      |

| SIDE 1 INJECTORS -->  | I   I |  <-- SIDE 2 INJECTORS|

|                       | I   I |                      |

|                    #1 | I   I |                      |

|                        -------                       |

|                                                      |

|                                                      |

|                                                      |

|                                                      |

|                                                      |

|______________________________________________________|

FRONT OF VEHICLE

(TOP VIEW)

NUMEGO

A calibration switch exists which is used to tell the computer how  many  EGO

sensors  are  present.   If  NUMEGO  = 1, the system is treated as a mono EGO

sensor system and all fuel pulsewidths are calculated  from  the  single  EGO

sensor  (treated  as  EGO-1).  If NUMEGO = 2:  Fuel pulsewidths for the EGO-2

injectors are calculated from EGO-2 sensor information; Fuel pulsewidths  for

the  EGO-1  injectors  are  calculated from EGO-1 sensor information.  FN1327

links the injector outputs to the appropriate EGO sensor.

8-3

FUEL STRATEGY, OVERVIEW - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FUEL MODES

Fuel control strategy is divided into 2 mutually exclusive modes:

-  OPEN LOOP

-  CLOSED LOOP

OPEN LOOP MODE

During open  loop  operation,  the  computer  calculates  the  injector  fuel

pulsewidths  required  to provide a pre-determined A/F ratio or lambda value.

The desired lambda values (LAMBSE1, LAMBSE2) can vary with  engine  operating

conditions  and  are calibration-dependent.  During open loop, LAMBSE1 equals

LAMBSE2.

CLOSED LOOP MODE

During closed loop operation, the computer ramps the  desired  lambda  values

(LAMBSE1,  LAMBSE2)  in  a limit cycle manner about stoichiometry.  Using the

EGO (Exhaust Gas Oxygen) sensor, the computer increases or  decreases  lambda

at  a  calculated  rate  of  change.   The  rate  at  which lambda changes is

calibration dependent.  For Stereo EGO operation, LAMBSE1  and  LAMBSE2  vary

independently using EGO-1 and EGO-2 sensors.  For Mono EGO operation, LAMBSE1

equals LAMBSE2.

8-4

FUEL STRATEGY, OUTPUT SHAFT SPEED LIMITER LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.2  OUTPUT SHAFT SPEED LIMITER LOGIC (CDAA0)

OVERVIEW

The output shaft speed limiter  logic  protects  vehicle  function  in  three

stages.

These stages are shown below:

1) Stage One:  Reduces the engine's power  output  in  a  gradual  manner  by

enriching  the  fuel  and  retarding  the spark (See Open Loop Fuel and Spark

Strategies).

2) Stage Two:  Further reduces the engine output by disabling the outputs  to

one-half  of the injector ports.  This action will occur only if the fuel and

spark strategy in Stage One is ineffective.

3) Stage Three:  Turns off all injector outputs.

DEFINITIONS

INPUTS

Registers:

-  NOBAR = Filtered output shaft speed, rpm.

Calibration Constants:

-  HNO_CL = Disable stage 1 no speed limiter, rpm.

-  HNO_SH = Enable stage 1 no speed limiter, rpm.

-  NOMAX = Stage 3 Output Shaft speed limit.

-  VHNO_CL = Disable stage 2 no speed limiter, rpm.

-  VHNO_SH = Enable stage 2 no speed limiter, rpm.

OUTPUTS

Bit Flags:

-  FOFFLG2 = Stage 2 alternate fire VS exceeded.

-  HSPFLG = High Speed Mode Flag; 1 -> High  Speed  Alt.   Fuel/Spark  Speed

Limit.

-  NLMT_FLG2 = Stage 3 no fuel VS exceeded.

-  NO_LIM_STG1 = Stage 1 output shaft speed flag

8-5

FUEL STRATEGY, OUTPUT SHAFT SPEED LIMITER LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  NO_LIM_STG2 = Stage 2 output shaft speed flag

8-6

FUEL STRATEGY, OUTPUT SHAFT SPEED LIMITER LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_NO_LIM_COM1

NOBAR > VHNO_SH ---------------|S Q -| NO_LIM_STG2

(very high speed)             |

|

NOBAR < VHNO_CL ---------------|C

NOBAR > HNO_SH ----------------|S Q -| NO_LIM_STG1

(high speed)                  |

|

NOBAR < HNO_CL ----------------|C

NOBAR > NOMAX -----------------------| STAGE 3

| (Turn off all injectors)

| NLMT_FLG2 = 1

|

| --- ELSE ---

|

NO_LIM_STG2 = 1 ---------------------| STAGE 2

| (Fire alternate injectors

|  using normal fuel strategy)

| FOFFLG2 = 1

| HSPFLG = 0

| NLMT_FLG2 = 0

|

| --- ELSE ---

|

NO_LIM_STG1 = 1 ---------------------| STAGE 1

| (Do fuel enrichment and spark

|  retard)

| FOFFLG2 = 0

| HSPFLG = 1

| NLMT_FLG2 = 0

|

| --- ELSE ---

|

| FOFFLG2 = 0

| HSPFLG = 0

| NLMT_FLG2 = 0

8-7

FUEL STRATEGY, RPM LIMITER HIGH LOAD - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.3  RPM LIMITER HIGH LOAD (CDAA0)

OVERVIEW

This mode of the RPM limiter is intended for the SuperCoup package ONLY.   It

should be calibrated to have no effect in ALL other strategies.

To calibrate this strategy to give the normal rpm  limit  strategy,  set  the

following as shown.

NLMTLO_1 = 7000

NLNTLO_2 = 7000

FIRPM_3 = 7000

DEFINITIONS

INPUTS

Registers:

-  ECT = Coolant temp, degree F.

-  FINCTR_1 = Mid gear high rpm timer.

-  LOAD = Universal load as ratio of air charge over standard.

-  N = RPM.

-  PIPLMT_HIG = Stage 3 rev limit in use.

-  PIPLMT_LOW = Stage 2 rev limit in use.

-  TP_REL = RELATIVE TP ( TP - RATCH ).

-  VS_LIM_BAR = VSBAR - high range.

Bit Flags:

-  NDSFLG = Neutral/drive flag; 1 -> drive.

Calibration Constants:

-  FIECT_3_SH = ECT to force alternate fire, high load.

-  FIECT_3_CL = ECT to exit alternate fire, high load.

-  FILOAD_3 = LOAD to force alternate fire.

-  FINCECT_2 = ECT to force alternate fire, top gear.

-  FINCTM_1H = Mid gear high rev duration timer clip.

8-8

FUEL STRATEGY, RPM LIMITER HIGH LOAD - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FINCTM_1L = Mid gear high rev duration to force rev limit.

-  FINVSBP_1 = Low gear/mid gear breakpoint.

-  FINVSBP_2 = Mid gear/top gear breakpoint.

-  FIRPM_3 = RPM to force alternate fire.

-  FITP_3 = TP to force alternate fire.

-  NLMTHI_0 = Low gear stage 3 rev limit.

-  NLMTHI_1 = Mid gear stage 3 rev limit.

-  NLMTHI_2 = Top gear stage 3 rev limit.

-  NLMTLO_0 = Low gear stage 2 rev limit.

-  NLMTLO_1 = Mid gear stage 2 rev limit.

-  NLMTLO_2 = Top gear stage 2 rev limit.

-  NLMTNEUHI_0 = Neutral stage 3 rev limit.

-  NLMTNEULO_0 = Neutral stage 2 rev limit.

OUTPUTS

Registers:

-  FINCTR_1 = Mid gear high rpm timer.

-  FINCTR_2 = Top gear high rpm timer.

Bit Flags:

-  FOFFLG3 = Stage 2 alternate fire TP exceeded.

8-9

FUEL STRATEGY, RPM LIMITER HIGH LOAD - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE:  FUEL_RPM_LIM_BG_COM3

FINVSBP_2 <= N/VS_LIM_BAR < FINVSBP_1 -|

|AND -| Increment timer FINCTR_1

N > NLMTLO_1 --------------------------|     |

| --- ELSE ---

|

| Decrement timer FINCTR_1

FINVSBP_2 <= N/VS_LIM_BAR < FINVSBP_1 -|

|

N > NLMTLO_1 --------------------------|

|AND -| FINCTR_1 = FINCTM_1H

FINCTR_1 > FINCTM_1L ------------------|

Note : FINCTM_1L should be calibrated to be less than FINCTM_1H.

8-10

FUEL STRATEGY, RPM LIMITER HIGH LOAD - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

ECT > FIECT_3_SH -----------------|S Q -|

|     |

ECT < FIECT_3_CL -----------------|C    |

|

LOAD > FILOAD_3 ------------------------|

|AND -| FOFFLG3 = 1

TP_REL > FITP_3 ------------------------|     |

|     | --- ELSE ---

N > FIRPM_3 ----------------------------|     |

| FOFFLG3 = 0

| PIPLMT_HIG =

ECT > FINCECT_2 ------------------------|     | Fn_A( NLMTHI_2 )

|AND -| PIPLMT_LOW =

N/VS_LIM_BAR < FINVSBP_2 ---------------|     |  Fn_A( NLMTLO_2 )

|

| --- ELSE ---

|

FINCTR_1 >= FINCTM_1L ------------|S Q -|     |

|     |     |

FINCTR_1 = 0 ---------------------|C    |     |

|OR --| PIPLMT_HIG =

ECT > FINCECT_1 ------------------|     |     |  Fn_A( NLMTHI_1 )

|     |     | PIPLMT_LOW =

FINVSBP_2 <= N/VS_LIM_BAR         |AND -|     |  Fn_A( NLMTLO_1 )

< FINVSBP_1 -----|           |

| --- ELSE ---

|

NDSFLG = 0 -----------------------------------| PIPLMT_HIG =

|  Fn_A( NLMTNEUHI_0 )

| PIPLMT_LOW =

|  Fn_A( NLMTNEULO_0 )

|

| --- ELSE ---

|

| PIPLMT_HIG =

|  Fn_A( NLMTHI_0 )

| PIPLMT_LOW =

|  Fn_A( NLMTLO_0 )

Where y = Fn_A( x ) is defined as x rpm converted to a pip period y in ticks.

8-11

FUEL STRATEGY, WARM EGO LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.4  WARM EGO LOGIC (CDAL0)

OVERVIEW

The Warm HEGO Logic determines if the HEGO sensor is  warm  enough  to  enter

Closed Loop control.  Time since start-up and coolant temperature at start-up

are used to determine if the sensor is warm.  The output from  the  logic  is

the flag, 'WRMEGO'.

The strategy also offers the alternative to use EXT_FEG, temperature  of  the

front  EGO sensors to decide if the EGO is warm.  If this approach is used, a

URD should be issued to remove the time at temperature  inference  from  this

module.

DEFINITIONS

Registers:

-  ATMR1 = Time since start (time since exiting crank mode), sec.

-  ATMR2 = Time since ECT became greater than TEMPFB, sec.

-  ATMR4 = Time since reaching ECTSTABL, sec.

-  ATMR1_HI_RES = Time since engine start with higher resolution than ATMR1.

-  ECT = Engine coolant temperature, deg F.

-  EXT_FEG = Inferred front HEGO tip temperature, degrees.

-  TCSTRT = Temperature of ECT at Cold Startup, deg F.

Bit Flags:

-  CRKFLG = Engine Mode Flag; 1 -> Crank Mode, 0 -> not in Crank Mode.

-  FLG_ECTSTABLQ = ECT stabilized flag; 1 -> ECT stabilized, use FN1360.

-  FLG_STABLECT = ATMR4 flip flop flag; 1 -> output enabled.

-  WRMEGO = HEGO sensor should be warm flag.

Calibration Constants:

-  CTHIGH = Hot Start Minimum Engine coolant Temperature, deg F.

-  CTLOW = Cold Start Maximum ECT, deg F.

-  ECTSTABL = Minimum ECT for stabilized Open Loop fuel table, deg F.

-  ECTSTABL_CL = ECTSTABL - hysteresis.

-  ECTSTABLTIM = Minimum time above ECTSTABL to use FN1360.

8-12

FUEL STRATEGY, WARM EGO LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  HEGO_WARM = Temperature of front EGO when it can be used for closed  loop

fuel control, degrees.

-  FNWRMENG = Time into run mode before  allowing  closed  loop  fuel  as  a

function of coolant temp at start.

-  OPCLT1 = ATMR1 Closed Loop enable time delay for TCSTRT <= CTLOW.

-  OPCLT2 = ATMR1 Closed Loop enable time delay for CTLOW < TCSTRT < CTHIGH.

-  OPCLT3 = ATMR1 Closed Loop Enable Time delay for TCSTRT >= CTHIGH.

-  OPCLT4 = ATMR2 Closed Loop enable time delay for TCSTRT <= CTLOW.

-  OPCLT5 = ATMR2 Closed Loop Enable Time Delay for CTLOW < TCSTRT < CTHIGH.

8-13

FUEL STRATEGY, WARM EGO LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_WRMEGO_COM1

BEGIN: warm_ego_logic

; Execute the following process once per background pass.

EXT_FEG > HEGO_WARM --------|

;EGO is warm               |

|AND -|

ATMR1_HI_RES >              |     |

FNWRMENG(ECT) ---|     |

;engine is warm.                 |

|

TCSTRT >= CTHIGH -----------|     |

|AND -|

ATMR1 >= OPCLT3 ------------|     |

|

CTLOW < TCSTRT < CTHIGH ----|     |

|AND -|OR --| WRMEGO = 1

ATMR1 >= OPCLT2 ------|     |     |     |

|AND -|     |     |

ATMR2 >= OPCLT5 ------|           |     |

|     |

TCSTRT <= CTLOW ------------|     |     |

|AND -|     |

ATMR1 >= OPCLT1 ------|     |           |

|AND -|           |

ATMR2 >= OPCLT4 ------|                 |

| --- ELSE ---

|

| WRMEGO = 0

CRKFLG = 0 -----------------|

|AND -|S Q -| FLG_STABLECT

ECT >= ECTSTABL ------------|     |

|

ECT < ECTSTABL_CL ----------|     |

|OR --|C

CRKFLG = 1 -----------------|

FLG_STABLECT = 1 -----------------------| Increment ATMR4

|

| --- ELSE ---

|

| ATMR4 = 0

ATMR4 > ECTSTABLTIM --------------------| FLG_ECTSTABLQ = 1

|  (ECT stabilized)

|

| --- ELSE ---

|

| FLG_ECTSTABLQ = 0

|  (cold engine)

END: warm_ego_logic

8-14

FUEL STRATEGY, LUGGING MODE DETERMINATION - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.5  LUGGING MODE DETERMINATION (CDAA0)

OVERVIEW

The strategy provides the capacity of Open Loop Fuel Control during "lugging"

conditions; lugging usually occurs at part throttle, high load, and low rpm.

The "lugging" open loop multiplier is adjusted for altitude through  the  use

of FN212A and FN129A.

CALIBRATION NOTES:

1.  LDMH and LDMHH should correspond to high loads (near WOT).

2.  LDLTM must be less than or equal to LUGTIM (since LUGTMR is

clipped to LUGTIM).

DEFINITIONS

INPUTS

Registers:

-  APT = At Part Throttle; -1 -> Closed throttle, 0 -> Part throttle,  1  ->

Wide Open throttle.

-  ECT = Engine Coolant Temperature, deg.  F.

-  PERLOAD = Percent of Peak LOAD at any altitude, unitless.

Bit Flags:

-  WRMEGO = Ego sensor should be warm flag.

Calibration Constants:

-  LDEL = Minimum ECT to enable Lugging Open Loop, deg.  F.

-  LDEH = Maximum ECT to enable Lugging Open Loop, deg.F.

-  LDMH = Minimum PERLOAD to enable Lugging Open Loop (near W.O.T.).

-  LDMHH = Hysteresis for LDMH.

-  LDLTM = Minimum time in lugging mode before forcing open loop, sec.

OUTPUTS

Bit Flags:

-  LDFLG = Lugging Mode Flag; 1 -> Lugging Mode, 0 -> Not in Lugging Mode.

8-15

FUEL STRATEGY, LUGGING MODE DETERMINATION - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE:  FUEL_LUG_MODE_COM1

APT = 0 ------------------------------|

(part throttle)                      |

|

LDEL < ECT < LDEH --------------------|

|

LUGTMR >= LDLTM ----------------------|AND ---| LDFLG = 1

|       |

PERLOAD >= LDMH + LDMHH ---|S Q-------|       | --- ELSE ---

|          |       |

PERLOAD < LDMH ------------|C         |       | LDFLG = 0

|

WRMEGO = 1 ---------------------------|

8-16

FUEL STRATEGY, LAMMUL RESET LOGIC - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.6  LAMMUL RESET LOGIC (CDAC0)

OVERVIEW

This module calculates the parameter, LAMMUL.  LAMMUL is used to prevent cold

engine stalls following transmission engagement.

DEFINITIONS

INPUTS

Registers:

-  ALT_CAL_FLG = Alternate calibration flag; 1 -> Alternate  calibration  is

used for spark plug fouling prevention.

-  MULTMR = Time since incrementing LAMMUL, sec.

-  REVFLG = Reverse engagement flag; 1 -> reverse, 0 -> drive.

Bit Flags:

-  DNDSUP = Delayed Neutral/Drive flag; 0 -> neutral, 1 -> drive.

-  NEUFLG = N/D transition occurred.

Calibration Constants:

-  FN371 = Initial LAMMUL as a function of ECT.  This is a  Fuel  Multiplier

to provide fuel compensation during Drive Engagement.

-  FN371_ALT = FN371 which is used during alternate  calibration  for  spark

plug fouling prevention.

-  FN393F = Time between LAMMUL decrements - forward gear.

-  FN393R = Time between LAMMUL changes - reverse gear.

-  TRLOAD = Transmission Load switch;

0 -> Manual Transmission, no clutch or gear switches,

forced neutral state (NDSFLG = 0).

1 -> Manual Transmission, no clutch or gear switch.

2 -> Manual Transmission, one clutch or gear switch.

3 -> Manual Transmission, both clutch and gear switches.

4 -> Auto Transmission, non-electronic, neutral drive switch.

5 -> Auto Transmission, non-electronic, neutral pressure switch,

(AXOD).

6 -> Auto Transmission, electronic, PRNDL sensor - park,

reverse, neutral, overdrive, manual 1, manual 2.

8-17

FUEL STRATEGY, LAMMUL RESET LOGIC - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OUTPUTS

Registers:

-  LAMMUL = Multiplier which is used to prevent cold-engine stalls following

transmission engagement.

-  MULTMR = See above.

Bit Flags:

-  NEUFLG = See above.

PROCESS

STRATEGY MODULE:  FUEL_LAMMUL_COM4

ALT_CAL_FLG = 1 ---------------------------| fn371 = FN371_ALT(ECT)

|

| --- ELSE ---

|

| fn371 = FN371(ECT)

TRLOAD <= 3 -------------------------------| No LAMMUL RESET

(Manual Transmission)                     | No change to LAMMUL

| No change to NEUFLG

|

| --- ELSE ---

DNDSUP = 1 --------------------------|     |

(Transmission in gear)              |AND -| LAMMUL RESET

|     | LAMMUL = fn371

NEUFLG = 1 --------------------------|     | NEUFLG = 0

(Transition from neutral)                 |

| --- ELSE ---

|

DNDSUP = 0 --------------------------------| NEUFLG = 1

(Transmission in neutral)

NOTE:  LAMMUL, FN371_ALT, and FN371 have a range  of  0  through  1.99.   The

LAMMUL  Reset  logic is done both in open loop and closed loop fuel.  LAMMUL,

however, is only applied in open loop fuel.

LAMMUL can either jump lean (ramp rich) or jump rich (ramp  lean),  depending

upon  whether  neutral  to drive transitions cause rich or lean errors.  This

logic is done both in open loop and closed loop fuel.

8-18

FUEL STRATEGY, LAMMUL RESET LOGIC - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

RAMP BACK LOGIC

REVFLG = 1 --------------|

|AND -|

MULTMR >= FN393R --------|     |

(in reverse)                  |OR --|

|     |

REVFLG = 0 --------------|     |     |

|AND -|     |        (LAMMUL was RESET RICH)

MULTMR >= FN393F --------|           |AND -| LAMMUL = LAMMUL + 0.0039

(in forward)                        |     | MULTMR = 0

|     | Clip LAMMUL to

LAMMUL < 1.0 ------------------------|     | 1.0 as a maximum

|

| --- ELSE ---

REVFLG = 1 --------------|                 |

|AND -|           |

MULTMR >= FN393R --------|     |           |

(in reverse)                  |OR --|     |

|     |     |

REVFLG = 0 --------------|     |     |     |

|AND -|     |     | (LAMMUL was RESET LEAN)

MULTMR >= FN393F --------|           |AND -| LAMMUL = LAMMUL - 0.0039

(in forward)                        |     | MULTMR = 0

|     | Clip LAMMUL to

LAMMUL > 1.0 ------------------------|     | 1.0 as a minimum

8-19

FUEL STRATEGY, OPEN LOOP FLAG DETERMINATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.7  OPEN LOOP FLAG DETERMINATION (CDAM0)

OVERVIEW

This module determines the state of the open loop desired flag, OL_DESIRED.

DEFINITIONS

Registers:

-  APT = At Part Throttle; -1 -> Closed throttle, 0 -> Part throttle,  1  ->

Wide Open throttle.

-  BG_TMR = Background timer, sec.

-  CAT_MAX_CLR = Lower Cat Midbed temp to clear EXT_FLG, with or  without  a

reliable EXT_CMD inference, Deg F.

-  CAT_MAX_SAFE = Desired Maximum Cat Midbed temperature that the  Cat  Temp

A/F  Controller will maintain; with & without failure mode safety factor,

Deg F.

-  COOL_FLG = Cylinder head overtemperature flag, lack of coolant  strategy;

1 -> overtemperature.

-  CTNTMR = Closed throttle neutral timer.

-  CTTMR = Time since entering Closed throttle mode, secs.  CTTMR is used to

delay Decel Fuel Shutoff (DFSO).

-  EXT_CMD = Signed inferred catalyst midbed gas temperature.

-  IDLTMR = In-gear idle time.

-  LOAD = Universal load as ratio of air charge over standard.

-  N = Engine speed rpm.

-  NUMEGO = Switch indicating number of HEGO sensors present, select mono or

stereo.

-  OLTMR = Open loop delay timer, sec.

-  OLTMR_CROWD = Open loop delay timer specifically for FLG_OL_337, sec.

-  PERLOAD = Percent of Peak LOAD at any altitude, unitless.

-  PPCTR = PIP counter for decel fuel ramp.

-  VSBAR = Vehicle speed, mph.

Bit Flags:

8-20

FUEL STRATEGY, OPEN LOOP FLAG DETERMINATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  CHKAIR = Thermactor Status flag; 0 -> Forced open loop, 1 ->  not  forced

open loop.

-  CTNFLG = 1 -> closed throttle neutral idle.

-  DIS_FMEM = HDR DIS strategy controlled FMEM flag.

-  DS_EGO_OL = Flag used to force open loop fuel control during a forced A/F

excursion to test the downstream EGOs.

-  ER_FUL_REQ = Utility  flag  used  during  on-demand  sequenced  tests  to

require LAMBSE override.

-  EXT_FLG = Catalyst exhaust overtemperature flag.

-  EXT_FMM_FLG = Failure Mode Flag to indicate  that  the  inferred  Exhaust

Temperature prediction output is suspect; 1 -> EXT model is suspect.

-  EXT_SET_FLG = Flip flop set flag for EXT_FLG.

-  FL_OLTMR_337 = Flip flop flag for FN337.

-  FL_OLTMR_338 = Flip flop flag for FN338.

-  FLG_ECTSTABLQ = ECT stabilized flag; 1 -> ECT stabilized, use FN1360.

-  FLG_OL_320A = Open loop due to FN320A, cold crowd enrichment.

-  FLG_OL_337 = Open loop due to FN337, hot stabilized crowd enrichment.

-  FLG_OL_338 = Open loop due to FN338, decel.

-  FLG_OL_VS = Open loop due to high vehicle speed.

-  FLG_OLTMR_VSBAR = Flip flop flag for VSBAR.

-  HALF_FUEL = Flag which indicates that  alternate  injections  are  to  be

cut-out; 1 -> Skip alternate injections.

-  HSPFLG = High Speed Mode Flag; 1 -> High speed alternate Fuel/Spark.

-  LDFLG = Lugging Mode flag; 1 -> Lugging mode.

-  LEGOFG11 = Lack of HEGO11 switching flag.

-  LEGOFG21 = Lack of HEGO21 switching flag.

-  NO_FUEL = Flag which indicates that all injectors are to be cut-out; 1 ->

Skip all injections.

-  OL_DES = EXT-independant open loop desired flag; 1 -> open  loop  desired

regardless of the exhaust gas temperature prediction.

-  OL_DESIRED = Open loop desired flag; 1 -> open loop desired, 0 ->  closed

loop desired.

-  OSC_OL = 1 -> Output State Control override of LAMBSE is in effect.

8-21

FUEL STRATEGY, OPEN LOOP FLAG DETERMINATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PxxxMALF = Malfunction flag for code Pxxx; 1 => a  malfunction  currently

exists for fault Pxxx.

-  WRMEGO = EGO sensor should be warm flag.

Calibration Constants:

-  CAT_MAX = Maximum catalyst temperature,  at  which  temp.   control  will

begin.

-  CAT_MAX_CL = Catalyst temperature which temp.  control is exited.

-  CAT_TEMP_SW = Cataylst temperature control switch to enable  cat.   temp.

control via open loop and/or torque control

-  CROWD_DEC = A scalar to mulitply BG_TMR by and then subtract the  product

from OLTMR_CROWD to create and up/down counter.

-  DCEVS = Minimum vehicle speed to do Decel Combustion Enrichment.

-  DCEVSH = DCEVS hysteresis (mph).

-  DCECTTM =  Minimum  time  at  closed  throttle  to  do  Decel  Combustion

Enrichment.

-  EXT_CMD_SF  =  Safety  factor  used  to  adjust  the   maximum   catalyst

temperature during periods of unreliable EXT_CMD calculation, Deg F.

-  FN320A(ECT) = Minimum  PERLOAD  required  for  Open  Loop  fuel  control,

unitless.

-  FN337(N) = Minimum PERLOAD required for Open Loop Fuel, unitless.

-  FN338(N) = Maximum LOAD required for Open Loop decels.

-  HLDTM1 = Open loop time delay for FN320A(ECT), sec.

-  HLDTM4 = Open loop time delay hysteresis for FN337(N), sec.

-  HLDTM_CR = Open loop time delay for FN337(N), seconds (msec resolution).

-  HLODH = Hysteresis term for FN320A(ECT) and FN337(N).

-  LLDTM = Open loop time delay for FN338(N), sec.

-  LOLODH = Hysteresis term for FN338(N).

-  NIOLD = Maximum time to allow closed loop fuel if  in  neutral  at  idle,

secs.

-  OLITD1 = Time allowed at idle before entering open loop.

-  PIPNUM = Number of PIPs for DFSO exit fuel ramp.

-  VS_OL_CL = Maximum vehicle speed to allow entry into closed loop logic.

-  VS_OL_SH = Minimum vehicle speed to force open loop lambda calculation.

8-22

FUEL STRATEGY, OPEN LOOP FLAG DETERMINATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OTHER

-  P0132 = Heated oxygen sensor cicuit voltage out of range, sensor 11.

-  P0135 = Fault code for EGO11 sensor heater circuit malfunction.

-  P0152 = Heated oxygen sensor circuit voltage out of range, sensor 12.

-  P0155 = Fault code for EGO21 sensor heater circuit malfunction.

8-23

FUEL STRATEGY, OPEN LOOP FLAG DETERMINATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_OLFLG_COM28

Intermediate Open loop desired logic.

ER_FUL_REQ = 1 ------------------------|

(Engine Running On-Demand testing     |

requires specific O.L. LAMBSE values)|

|

FLG_OL_320A = 1 -----------------|     |

|AND -|

OLTMR >= HLDTM1 -----------------|     |

|

FLG_OL_337 = 1 ------------------|     |

|AND -|

OLTMR_CROWD > HLDTM_CR ----------|     |

|

FLG_OL_338 = 1 ------------------|     |

|AND -|

OLTMR >= LLDTM ------------------|     |

|

APT = 1 -------------------------------|

|

PPCTR < PIPNUM ------------------------|

(decel fuel shutoff)                  |

|

CTNTMR > NIOLD ------------------|     |

(neutral idle)                  |AND -|

|     |

CTNFLG = 1 ----------------------|     |OR --| ;Open loop desired

|     | ;for drive mode.

IDLTMR > OLITD1 -----------------------|     |

(drive idle)                          |     | OL_DES = 1

|     |

WRMEGO = 0 ----------------------------|     |

|     |

CHKAIR = 0 ----------------------------|     |

|     |

HSPFLG = 1 ----------------------------|     |

|     |

HALF_FUEL = 1 -------------------------|     |

|     |

NO_FUEL = 1 ---------------------------|     |

|     |

LDFLG = 1 -----------------------------|     |

|     |

FLG_OL_VS = 1 -------------------------|     |

|     |

DIS_FMEM = 1 --------------------------|     |

|     |

DS_EGO_OL = 1 -------------------------|     |

|     |

OSC_OL = 1 ----------------------------|     |

|     |

COOL_FLG = 1 --------------------------|     |

(continued on next page)

8-24

FUEL STRATEGY, OPEN LOOP FLAG DETERMINATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|     |

P0132MALF = 1  ------------|           |     |

|OR --|     |     |

P0135MALF = 1 -------------|     |     |     |

|     |     |

LEGOFG11 = 1 --------------------|AND -|     |

|     |     |

NUMEGO = 1 ----------------------|     |     |

(sensor overvoltage or                |     |

heater malfunction exists and        |     |

sensor is in lack of swithcing       |     |

and is only a single bank            |     |

application)                         |     |

|     |

P0152MALF = 1 -------------|           |     |

|OR --|     |     |

P0155MALF = 1 -------------|     |     |     |

|     |     |

LEGOFG21 = 1 --------------------|     |     |

|     |     |

P0132MALF = 1 -------------|     |     |     |

|OR --|AND -|     |

P0135MALF = 1 -------------|     |           |

|           |

LEGOFG11 = 1 --------------------|           |

|           |

NUMEGO = 2 ----------------------|           |

(sensor overvoltage or                      |

2 heater malfunctions exist and            |

sensors are in lack of swithcing           |

and is a dual bank                         |

application)                               |

| --- ELSE ---

|

| ;Open loop not commanded by

| ;driver demand, idle, or

| ;other exception conditions.

|

| OL_DES = 0

;Decide whether or not to force Open Loop Fuel.

OL_DES = 1 -----------------------|

|OR --| ;Force Open Loop.

EXT_FLG = 1 ----------------------|     |

;exhaust temp. forcing Open Loop       | OL_DESIRED = 1

|

| --- ELSE ---

|

| ;Open Loop not desired.

|

| OL_DESIRED = 0

8-25

FUEL STRATEGY, OPEN LOOP FLAG DETERMINATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The following timers delay Open Loop fuel control during short transients;

which is helpful during manual shifts or other fast throttle movements.

OLTMR - OPEN LOOP TIMER

(0.125 sec resolution)

OLTMR_CROWD - Actuated-up counting Open Loop Timer for Crowd Enrichment.

(0.001 sec resolution)

PERLOAD >= FN320A ---------------------|S Q -| FLG_OL_320A

(cold crowd enrichment)               |

|

PERLOAD < FN320A - HLODH --------------|C

PERLOAD >= FN337(N) -------------------|S Q -| FL_OLTMR_337

(stabilized crowd enrichment)         |

|

PERLOAD < FN337(N) - HLODH ------------|C

FL_OLTMR_337 = 1 ----------------------|

|AND -| FLG_OL_337 = 1

FLG_ECTSTABLQ = 1 ---------------------|     |

| --- ELSE ---

|

| FLG_OL_337 = 0

LOAD <= FN338 -------------------------|S Q -| FL_OLTMR_338

(decel load)                          |

|

LOAD > FN338 + LOLODH -----------------|C

VSBAR >= DCEVS ------------------------|S Q -| FLG_OLTMR_VSBAR

|

VSBAR < DCEVS - DCEVSH ----------------|C

FL_OLTMR_338 = 1 ----------------------|

|

FLG_OLTMR_VSBAR = 1 -------------------|AND -| FLG_OL_338 = 1

|     |

CTTMR >= DCECTTM ----------------------|     |

| --- ELSE ---

|

| FLG_OL_338 = 0

8-26

FUEL STRATEGY, OPEN LOOP FLAG DETERMINATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLG_OL_320A = 1 -----------------------|

|OR --| Increment OLTMR

FLG_OL_338 = 1 ------------------------|     |

|

| --- ELSE ---

|

| OLTMR := 0

FLG_OL_337 = 1 ----------------| Increment OLTMR_CROWD

| OLTMR_CROWD = MIN(OLTMR_CROWD, HLDTM4)

|

| --- ELSE ---

|

| ;Decrement timer according to this logic.

| ;NOTE: If CROWD_DEC is calibrated to 1.0,

| ;the timer must function as a typical

| ;UP/DOWN actuated timer.

| OLTMR_CROWD = OLTMR_CROWD

|                - (BG_TMR * CROWD_DEC)

Determine the state of forced open loop based on vehicle speed flag:

VSBAR > VS_OL_SH ----------------------|S Q -| FLG_OL_VS

|

VSBAR < VS_OL_CL ----------------------|C

8-27

FUEL STRATEGY, OPEN LOOP FLAG DETERMINATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;Determine Open Loop request for exhaust gas temperature concerns.

;Calculate Maximum allowable Catalyst Temperatures.

EXT_FMM_FLG = 1 ------------------| CAT_MAX_SAFE = CAT_MAX - EXT_CMD_SF

;Exhaust Temp Model is suspect   | CAT_MAX_CLR = CAT_MAX_CL - EXT_CMD_SF

|

| --- ELSE ---

|

| CAT_MAX_SAFE = CAT_MAX

| CAT_MAX_CLR = CAT_MAX_CL

;Service the EXT_FLG flip flop.

EXT_CMD > CAT_MAX_SAFE -----------|S Q -| EXT_SET_FLG

(exhaust temp. control)          |

|

EXT_CMD <= CAT_MAX_CLR -----------|C

;EXT_FLG servicing logic.

EXT_SET_FLG = 1 -----------------------|

|

FLG_ECTSTABLQ = 1 ---------------------|AND -| EXT_FLG = 1

|     |

CAT_TEMP_SW = 1 -----------------------|     |

| --- ELSE ---

|

| EXT_FLG = 0

8-28

FUEL STRATEGY, CALCULATION OF LAMAVE - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.8  CALCULATION OF LAMAVE (CDAA0)

OVERVIEW

This module calculates the  LAMBSE  average  to  be  used  in  Self  Test  to

determine if the adaptive tables have reached their clip.

The LAMBSE average, LAMAVE1/LAMAVE2,  is  only  calculated  in  Closed  Loop.

Also,  it  requires  at least two HEGO switches after entering Closed Loop to

calculate an average.

DEFINITIONS

Registers:

-  LAMBSE1 = Desired equivalence ratio for HEGO11 injectors.

-  LAMBSE2 = Desired equivalence ratio for HEGO21 injectors.

-  LAM_OLD1 = Value of LAMBSE1 the last time HEGO11 switched.

-  LAM_OLD2 = Value of LAMBSE2 the last time HEGO21 switched.

Bit Flags:

-  OLFLG = Open Loop Fuel flag; 1 -> open loop, 0 -> closed loop possible.

-  SWTFL11 = Bank1 upstream HEGO11 switch flag; 0-> no  HEGO11  switch,  1->

sensor HEGO11 switch.

-  SWTF211 = Bank1 upstream HEGO21 switch flag; 0-> no  HEGO21  switch,  1->

sensor HEGO21 switch.

OUTPUTS

Registers:

-  LAMAVE1 = Average of LAMBSE1;  averages  the  value  of  LAMBSE1  at  the

current HEGO11 switch and last HEGO11 switch.

-  LAMAVE2 = Average of LAMBSE2;  averages  the  value  of  LAMBSE2  at  the

current HEGO21 switch and last HEGO21 switch.

-  LAM_OLD1 = See above.

-  LAM_OLD2 = See above.

8-29

FUEL STRATEGY, CALCULATION OF LAMAVE - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_LAMAVE_COM1

OLFLG = 1 -----------------------------| LAM_OLD1 = 0

| LAM_OLD2 = 0

| LAMAVE1 = 1.0

| LAMAVE2 = 1.0

|

| Exit FUEL_LAMAVE_COM1 module

SWTFL11 = 1 ---------------------|

(HEGO11 switch)                |

|AND -| LAM_OLD1 = LAMBSE1

OLFLG = 0 -----------------------|     |  (not enough HEGO switches

|     |   since going Closed Loop

LAM_OLD1 = 0 --------------------|     |   to calculate LAMAVE1)

|

| --- ELSE ---

SWTFL11 = 1 ---------------------|     |

(HEGO11 switch)                |AND -| LAMAVE1 = (LAM_OLD1 + LAMBSE1) / 2

|     | LAM_OLD1 = LAMBSE1

OLFLG = 0 -----------------------|     |  (update LAM_OLD1 for next time)

SWTFL21 = 1 ---------------------|

(HEGO21 switch)                |

|AND -| LAM_OLD2 = LAMBSE2

OLFLG = 0 -----------------------|     |  (not enough EGO switches

|     |   since going Closed Loop

LAM_OLD2 = 0 --------------------|     |   to calculate LAMAVE2)

|

| --- ELSE ---

SWTFL21 = 1 ---------------------|     |

(HEGO21 switch)                |AND -| LAMAVE2 = (LAM_OLD2 + LAMBSE2) / 2

|     | LAM_OLD2 = LAMBSE2

OLFLG = 0 -----------------------|     |  (update LAM_OLD2 for next time)

NOTE:  LAMAVE1 and LAMAVE2 are initialized to 1.0, and LAM_OLD1 and  LAM_OLD2

are initialized to 0.0.

8-30

FUEL STRATEGY, OPEN LOOP LAMBSE CALCULATION -  CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.9  OPEN LOOP LAMBSE CALCULATION (CDAN0)

OVERVIEW

The purpose of the Open Loop LAMBSE Calculation  is  to  schedule  a  desired

equivalence  ratio  (lambda  value)  when  engine operating conditions either

prevent the use of the HEGO sensor or stoic operation is not appropriate.

Due  to  the  current  design  of  the  HEGO  sensor,  only  operation  about

stoichiometry  can  be utilized.  In the future, Universal HEGOs will be able

to provide feedback at points either rich or  lean  of  stoic.   Closed  loop

operation  is  also  required  for adaptive fuel to "learn" the variabilities

associated with production tolerances of airflow measuring and fuel  metering

devices.  Given, however, that closed loop fuel is not always the appropriate

mode of operation, the open loop logic performs two functions:

1.  To determine whether open loop fuel is appropriate.

2.  To schedule the optimum equivalence ratio (LAMBSE).

Optimum Air/Fuel (A/F) ratio at any given engine  speed-load  is  that  which

will  develop the required torque with the lowest fuel consumption consistent

with smooth reliable operation.  This optimum A/F ratio is not  constant  but

depends  on  many  factors.   The proper A/F ratio for each particular set of

operating conditions is most conveniently viewed under a number of  headings:

stabilized,  cold  engine/startup, catalyst temperature protection, cylinders

deactivated, and wide open throttle condition.  Stabilized is taken  to  mean

continuous  operation  with  normal engine temperatures.  Cold engine/startup

includes starting and warming up or when the HEGO  sensor  is  not  ready  to

switch.  Catalyst temperature protection enriches the A/F ratio to control to

a  set  desired  catalyst  operating   temperature.    When   cylinders   are

deactivated,  the torque control module issues an A/F ratio to use.  Finally,

for performance requirements, LBT may needed  to  provided  peak  torque  A/F

ratio fueling.

Notes on Cylinder Disabling (Torque Control request):

The torque control strategy controls any situation  in  which  cylinders  are

disabled.  When cylinder(s) are disabled, the air fuel must be scheduled lean

of stoichiometry.  If not, the unburned fuel from any rich cylinder  and  the

oxygen  from the disabled cylinder(s) will ignite in the catalyst, causing an

over temperature condition.

One excepting to scheduling a lean A/F when disabling cylinders is  when  the

catalyst  is  cold  (during  a cold start up).  LAMBSE will be allowed to run

rich until the catalyst mid-bed temperature reaches EXT_COLD_CAT.  This is to

prevent  cold  stalls that would occur if a cold engine is forced to run lean

because it was disabling cylinders (ie.  transmission torque reduction).

8-31

FUEL STRATEGY, OPEN LOOP LAMBSE CALCULATION -  CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Notes on Cold/Start-up Open Loop Fuel:

A.  Entry conditions:

-  WRMEGO = 0, HEGO sensor may not be ready to switch or,

-  ECT < ECTSTABL, engine coolant temperature too cold and OL_DESIRED = 1;

standard open loop conditions met.

B.  Fuel scheduling:

-  Fuel may be scheduled as a function of load, temperature and time since

start.

Notes on Stabilized Engine Open Loop Fuel:

A.  Mixture conditions:

Mixture requirements for lowest fuel consumption can generally be  adequately

described  as a function of engine load which for purposes of this discussion

will be defined as the ratio of aircharge to standard aircharge.

Idle, extremely light loads or deceleration -

No useful work is being done at idle other than driving accessory loads, i.e.

brake torque is zero.  The lowest fuel consumption is that A/F which provides

steady, reliable cylinder firing.  Extremely light loads or  decels  at  high

RPMs  may  require  A/F scheduling.  This is because the mass of residual gas

tends to be constant, therefore,  at  closed  throttle/slight  part  throttle

decels, the percentage of residual diluting the fresh charge increases, hence

the need to enrich the incoming charge.

Medium loads, typical part throttle operation at road load -

Best economy is around 18:1 A/F ratio, however, this may  lead  to  excessive

highway  NOx  levels.   Also,  current  catalyst  technology makes use of the

closed loop fuel limit cycle to maximize conversion efficiency.  This, closed

loop  fuel  operation  is  desirable  from both an adaptive fuel and catalyst

efficiency standpoint.

Full load, typical W.O.T. operation -

A rich A/F ratio is required to maximize the  torque  output  of  the  engine

during  periods  of  high driver demand.  Maximum torque output is about 13:1

A/F ratio (LBT).  Best fuel economy is sacrificed in return for higher torque

output.

8-32

FUEL STRATEGY, OPEN LOOP LAMBSE CALCULATION -  CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Best economy at a given load is independent of RPM at least down to idle type

loads,  under conditions at which fuel distribution is good.  Departures from

best fuel economy may be required due to the following:

.  Poor distribution (usually a function of TP and RPM).

.  To reduce temperature of hot spots such as exhaust

valves, spark plug points or piston crowns - that is,

to assist cooling.

B.  Entry Conditions:

-  WRMEGO = 1 - engine coolant hot and engine is stabilized, HEGO sensor  is

ready to switch.

-  High Load enrichment required - scheduled as a  function  of  speed/load.

An  open  loop  delay  time is provided to prevent unnecessary enrichment

during high load spikes such as during shifts.  A second enrichment timer

provides  the  ability  to  schedule  fuel  as  a function of time in the

enrichment mode to enhance durability in heavy truck applications.

-  Decel loads - either enrichment or  enleanment  can  be  scheduled  as  a

function of speed/load.

-  Extended idle - the ability  to  go  open  loop  for  idle  stability  is

provided as a function of time at idle.

-  CHKAIR = 0 - thermactor air control can put upstream air into the exhaust

manifold  ahead  of the HEGO sensor causing the HEGO to always read lean.

Open loop is therefore required.

-  Lack of HEGO switching requires the scheduling of open loop operation.

C.  Fuel Scheduling:

-  Fuel may be scheduled as a function of load, RPM, temperature and time at

high load.

Notes on execution of LAMBSE_RESET logic:

The LAMBSE_RESET logic is only executed in Closed Loop Fuel Control mode, but

parameters  used  in  LAMBSE_RESET  must be updated each background pass.  As

such, logic was added to execute the call to FUEL_LAMBSE_RESET_COMx from this

module and assign the values of ISCFLG_LST and OLFLG_LST.  This specification

provides a more descriptive execution sequence  which  is  required  for  the

proper function of the FUEL_LAMBSE_RESET_COMx module.

8-33

FUEL STRATEGY, OPEN LOOP LAMBSE CALCULATION -  CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Notes on Catalyst Temperature Protection:

The catalyst temperature model provides an inferred catalyst  temperature  1"

into  the  first  brick  from  the  inlet.   This  is  considered the hottest

operating point in the catalyst, which we refer to as the midbed temperature,

or simply EXT_CMD.

The air/fuel controller requires  little  calibration  (CAT_MAX,  CAT_MAX_CL,

CAT_MAX_SAFE  and  LAM_EXT_STEP)  once  the  real  time  exhaust algorithm is

calibrated to good agreement with the actual thermocouple measurements.  As a

result, the controller should exhibit good temperature control (always within

one air/fuel step below the maximum) with no  overshoot  and  no  hunting  of

LAMBSE.

If the catalyst midbed temperature (EXT_CMD) is above  CAT_MAX_SAFE  but  not

below  CAT_MAX_CLR  (EXT_FLG  =  1), LAMBSE_EXT is calculated to maintain and

operate the midbed temperature as close to, but not over CAT_MAX_SAFE.

Control starts by calculating what the steady state condition  would  be  for

the   midbed   temperature  operating  at  current  conditions  (EXT_SS_FL  +

EXT_SS_EXO - EXT_LS_CIN).  This calculation is possible by using the  exhaust

model  parameters  that do not have any of the systems delays associated with

them.  If this steady  state  speculation  is  above  CAT_MAX_SAFE,  fuel  is

enrichened  by  decrementing LAMBSE_EXT by LAM_EXT_STEP each background loop,

or until the lower limit of LAMBSE_EXT is reached,  LAM_EXT_MIN.   Otherwise,

if  the  steady state temperature is below CAT_MAX_SAFE, further calculations

are performed to see if LAMBSE_EXT can be  enleaned  so  as  to  operate  the

catalyst midbed temperature closer to, but not above CAT_MAX_SAFE.

LAMBSE_EXT will not be allowed to return to  stoichiometry  (go  leaner  than

LAM_EXT_MAX) until the EXT_FLG is reset.  This is to allow a cool down period

in the catalyst before exposing the recently hot, rich catalyst to the oxygen

levels associated with stoichiometry.

If "lambse_try" is not leaner than LAM_EXT_MAX, then the steady state  midbed

temperature is calculated to find the midbed temperature that would result if

lambse_try  was  used.   If  this  resultant  midbed  temperature  is   above

CAT_MAX_SAFE,  LAMBSE_EXT  is  not changed.  Otherwise, LAMBSE_EXT can assume

the leaner value of lambse_try (which results in a midbed temperature  closer

to CAT_MAX_SAFE and a leaner LAMBSE_EXT).

The effects of LAMBSE1,2 on catalyst midbed temperatures  (given  a  constant

load, spark retard, and percent EGR) are shown below:

|                   +

|              +         +

EXT_CMD  |           +   \           +

|         +      \             +

|                 \LAM_EXT_MAX

|     +                            +

|      \

|   +   \LAM_EXT_MIN                  +

|

|________________________________________

rich   0.8       1.0         1.1   lean

LAMBSE1,2 scheduling

8-34

FUEL STRATEGY, OPEN LOOP LAMBSE CALCULATION -  CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  ACT = Air charge temperature, degrees farenheit.

-  AM = Air mass flow bin 10.

-  APT = At Part Throttle; -1 -> Closed throttle, 0 -> Part throttle,

1 -> Wide Open throttle.

-  ATMR1 = Time since start (time since exiting crank mode), sec.

-  ATMR1_HI_RES = Time since start with  a  greater  resolution  than  ATMR1

(time since exiting crank mode), sec.

-  BG_TMR = Background loop timer.

-  BP = Barometric pressure (Note:  Upper byte of BP_WORD).

-  CAT_MAX_CLR = Lower Cat Midbed temp to clear EXT_FLG, with or  without  a

reliable EXT_CMD inference, Deg F.

-  CAT_MAX_SAFE = Desired Maximum Cat Midbed temperature that the  Cat  Temp

A/F  Controller will maintain; with & without failure mode safety factor,

Deg F.

-  ECT = Coolant temperature degrees farenheit.

-  ER_LAM_DSD1 = LAMBSE1 value requested by engine running  on  demand  self

test.

-  ER_LAM_DSD2 = LAMBSE2 value requested by engine running  on  demand  self

test.

-  EXT_CMD = Mid bed catalyst temperature.

-  EXT_LS_CIN = Steady  state  temperature  loss  in  exhaust  pipe  between

exhaust flange and catalyst inlet, degrees.

-  EXT_SS_EXO = Steady state value of inferred rise in  temperature  in  the

catalyst, the "exotherm", in degrees.

-  EXT_SS_FLS = Steady state exhaust flange temperature at a stoic  air/fuel

mixture, degrees.

-  EXT_SS_FL = Steady state exhaust flange temperature, degrees.

8-35

FUEL STRATEGY, OPEN LOOP LAMBSE CALCULATION -  CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FUEL_SOURCE1 = Value to indicate the source of the final open loop LAMBSE

calculation for bank 1.

0 -> No open loop fuel control (closed loop).

1 -> Cylinder disablement logic is enabled.

2 -> Exhaust temperature fuel controller logic.

3 -> Wide open throttle performance logic.

4 -> Cold engine/startup open loop logic.

5 -> Stabilized normal drivability logic.

6 -> Flex fuel lean error logic (if applicable).

7 -> Engine running EGO monitoring testing.

8 -> Engine running On-Demand testing.

9 -> Cylinder disablement logic enabled on a cold start.

12 -> Secondary air monitor flow testing.

-  FUEL_SOURCE2 = Same as FUEL_SOURCE1, but for bank 2.

-  INJ_ON = Number of injectors actively producing engine torque.

-  ISCFLG = Mode indicator flag.

-  ISCFLG_LST = Last pass value of ISCFLG.

-  LAMBSE1 = Desired  equivalence  ratio  for  Bank  1  injectors.   LAMBSE1

appears in the fuel pulsewidth equation for HEGO11.

-  LAMBSE2 = Desired  equivalence  ratio  for  Bank  2  injectors.   LAMBSE2

appears in the fuel pulsewidth equation for HEGO21.

-  LAMBSE_DRV = Desired LAMBSE for drivability concerns.

-  LAMBSE_EXT = Desired LAMBSE for catalyst temperature control concerns.

-  LAMMUL = Multiplier which is used to prevent cold-engine stalls following

transmission engagement.

-  LOAD = Universal load as ratio of aircharge over standard.

-  N = Engine speed, rpm.

-  NOBAR = Filtered output shaft speed, rpm.

-  OL_LEAN_TMR = Open loop timer used for enleanment, sec.

-  OV_TMR_11 = EGO11 overvoltage timer.

-  OV_TMR_21 = EGO21 overvoltage timer.

-  PERLOAD = Percent of peak LOAD at any altitude.

-  PPCTR = PIP counter for torque management to control open loop fueling.

-  SAIR_LAM_DSD = Required lambse value for continuous secondary air test.

-  SPK_LAMBSE = Value of LAMBSE to be used in spark calculations.

-  TCSTRT = Temperature of ECT at Cold Start, deg F.

8-36

FUEL STRATEGY, OPEN LOOP LAMBSE CALCULATION -  CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TSLAMUn = Time at which last LAMBDA update occurred, clock ticks.

-  VS_RATE = Filtered vehicle acceleration rate.

Bit Flags:

-  ALT_CAL_FLG = Flag to indicate use of alternate calibration.

-  DIS_FMEM = Flag indicating that a primary side failure has occurred on  a

DIS system and that an alternate strategy has been requested.

-  DNDSUP = Delayed Neutral/Drive flag; 0 -> neutral, 1 -> drive.

-  DS_EGO_LEAN1 = Flag used by the downstream EGO monitor to trigger a  lean

A/F excursion for bank1; 1 -> excursion in progress.

-  DS_EGO_LEAN2 = Flag used by the downstream EGO monitor to trigger a  lean

A/F excursion for bank2; 1 -> excursion in progress.

-  DS_EGO_OL = Flag used to force open loop fuel control during a forced A/F

excursion to test the downstream EGOs.

-  DS_EGO_RICH1 = Flag used by the downstream EGO monitor to trigger a  rich

A/F excursion for bank1; 1 -> excursion in progress.

-  DS_EGO_RICH2 = Flag used by the downstream EGO monitor to trigger a  rich

A/F excursion for bank2; 1 -> excursion in progress.

-  EGO1FMFLG = EGO1 FMEM flag; 1 -> EGO1 sensor failure.

-  EGO2FMFLG = EGO2 FMEM flag; 1 -> EGO2 sensor failure.

-  ER_FUL_REQ = Flag to signal fuel system to set LAMBSE to KOER_LQM_DSD.

-  EXT_FLG = The exhaust catalyst temperature is over CAT_MAX_SAFE.

-  FFG_CSD11 = Fault flag for CSD on sensor 11.

-  FFG_CSD21 = Fault flag for CSD on sensor 21.

-  FLG_ECTSTABLQ = ECT stabilized flag; 1 -> ECT stabilized, 0 -> ECT cold.

-  HALF_FUEL = Flag which indicates one half of the injectors are cut out; 1

-> half_fuel mode.

-  HEGO_FAULT = Flag indicating at least one EGO sensor is faulted.

-  HEGO_FDBACK = Flag indicating normal closed loop operation.

-  HSPFLG = High speed mode flag; 1 -> High Speed alternate fuel/spark.

-  LDFLG = "Lugging" mode open loop flag; 1 -> lugging mode.

-  LEGOFG11 = Lack of HEGO11 switching.

-  LEGOFG21 = Lack of HEGO21 switching.

8-37

FUEL STRATEGY, OPEN LOOP LAMBSE CALCULATION -  CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  LESFLG1 = Lack of Bank1 HEGO switching flag; 1 -> HEGO is not  switching,

0 -> HEGO is switching.

-  OL_DRIVE = Open  loop  due  to  driving  conditions  (not  FMEM  or  HOLD

related).

-  OLFLG = Open loop flag; 1 -> open loop mode, 0 -> closed loop mode.

-  OLFLG_LST = Last pass value of OLFLG.

-  OL_FMEM = Open loop due to detected system fault flag; 1 ->open loop  due

to FMEM.

-  OL_HOLD = Present conditions not satisfied to go closed loop flag;  1  ->

hold in open.

-  OL_DES = EXT-independant open loop desired flag; 1 -> open  loop  desired

regardless of the exhaust gas temperature prediction.

-  OL_DESIRED = Open loop desired flag; 1 -> open loop desired, 0 ->  closed

loop desired.

-  SAIR_TST_ENA = OBD-II secondary air test enable flag; 1 -> test enabled.

-  WRMEGO = If set; HEGO sensor should be warm and flag set  to  1  if  HEGO

sensor  is switching, and reset to 0 if it has cooled down.  Its state is

controlled by the WRMEGO logic.

Calibration Constants:

-  CAT_MAX = Desired operating temperature of  catalyst  that  the  catalyst

temperature controller will control to, degrees.

-  CAT_MAX_CL = Catalyst temperature which temp.  control is exited.

-  CTHIN = Maximum TCSTRT value to use NUMPR.

-  CTLOW = Cold Start Maximum ECT, deg F.

-  DFSLAM = Relative LAMBSE value to exit decel fuel shutoff.

-  EXT_COLD_CAT = Catalyst temperature below which AFR is allowed to be rich

during torque control cylinder cutout, degrees F.

-  FN022B = Temperature normalizing function; used for table lookup.

Input = temp_b = FRCBFT * ACT + (1 - FRCBFT) * ECT

--OR--

Input = temp_s = FRCSFT * ACT + (1 - FRCSFT) * ECT

-  FN034A = LOAD normalizing function for FNCYL_OFF table lookup.

Input = LOAD.

Output = Normalized load.

-  FN070C = RPM normalizing function for FNCYL_OFF input.

Input = N.

Output = Normalized RPM.

8-38

FUEL STRATEGY, OPEN LOOP LAMBSE CALCULATION -  CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FN072A = PERLOAD normalizing function; used for table lookup.

Input = PERLOAD.

Output = Normalized perload.

-  FN129A(BP) = Multiplier to FN309 to compensate for altitude.

-  FN212A(BP) = Multiplier to FN308 to compensate for altitude.

-  FN300 = Multiplier as a function of ACT, modifies FN1360.

-  FN301 = Multiplier for closed throttle as a function of engine speed, N.

-  FN303 = WOT Fuel Multiplier as a function of engine speed N.

-  FN308 = Sea level fuel multiplier, RPM.

-  FN309 = Altitude lugging fuel multiplier, RPM.

-  FN321 = Multiplier used to lean-out fuel upon initial closed loop.

-  FN325 = Multiplier as a function of ECT for FN1360.

-  FN396C = High Speed Fuel enrichment, mph.

-  FN441A('lambse')  =  Effect  of  LAMBSE  on   inferred   exhaust   flange

temperature.  (NOTE:  various lambda inputs are used with this function).

-  FN448(AM) = Steady-state increase in exhaust temp in the  catalyst  as  a

function of Air Mass, at an equivalence ratio of 1.0; degrees F.

-  FN448A('lambse') = Steady-state exotherm in  the  catalyst  for  a  given

LAMBSE;  deg  F.   (NOTE:   Various  lambda  inputs  are  used  with this

function).

-  FN911(LOAD) = LAMBSE multiplier during DIS FMEM.

-  FN1360 = Stabilized Open Loop Fuel Table = an 8  X  10  table  of  lambda

values as a function of engine speed N, and PERLOAD.

X = FN070B(N)

Y = FN072B(PERLOAD)

-  FN1361_H_RES = Startup Open Loop Fuel Table = a 8 x 10  table  of  lambda

values  as  a function of temp_s (ie.  FRCSFT * ACT + (1 - FRCSFT) * ECT)

and ATMR1_HI_RES.  TABSFT is the Synonym for this Table.

X = FN022B(temp_s)

Y = FN018C(ATMR1_HI_RES)

-  FN1362 = Base Open Loop Fuel table = a 10 x 8 table of lambda values as a

function  of  temp_b  (FRCBFT  *  ACT  + (1 - FRCBFT) * ECT) and PERLOAD.

TABBFT is the Synonym for this Table.

X = FN022B(temp_b)

Y = FN072A(PERLOAD)

-  FN1362_ALT = Alternative FN1362 used in Anti-plug fouling strategy.

8-39

FUEL STRATEGY, OPEN LOOP LAMBSE CALCULATION -  CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FNCYL_OFF = Lambse value to use when cylinders are disabled.   Use  of  a

lean  A/F  ensures protection of the catalyst, while use of richer values

closer to stoic (still lean) may be needed in low  speed/low  conditions.

Use  of  the  round robin cylinder logic protects the valves when running

lean.

X = FN070C(N)

Y = FN034A(LOAD)

-  FRCBFT = Act fraction for FN1362 lookup.

-  FRCSFT = ACT fraction for FN1361 lookup.

-  LAM_COLD_ENG = LAMBSE value to be used by spark if the engine is cold.

-  LAM_EXT_MAX = Base LAMBSE value when controlling catalyst temperature via

open loop fuel.

-  LAM_EXT_MIN = Lowest LAMBSE is allowed to go when controlled by  catalyst

temperature protection logic.

-  LAM_EXT_STEP = LAMBSE increment/decrement size done per  background  loop

to  control  catalyst temperatures DIVIDED by the average background loop

time (40ms).

-  LEAN_EXCRSN = LAMBSE to command while forcing an A/F excursion  to  force

the downstream EGO sensors lean.

-  MINACCL = Minimum  Vehicle  Speed  rate  of  change  to  enable  FN303(N)

fueling.

-  NUMCYL = Number of engine cylinders.

-  NUMEGO = Number of EGO sensors.

-  NUMPR = Open Loop Fuel multiplier.

-  OLIDRV = Drive Open Loop Idle fuel multiplier.

-  OLINEUT = Neutral Open Loop Idle fuel multiplier.

-  OLMCL = Open loop fuel multiplier,

-  OLMTD1 = NUMPR open loop fuel multiplier time delay, seconds.

-  PIPNUM = Number of PIPs before DFSO exit fuel ramp.

-  RICH_EXCRSN = LAMBSE to command while forcing an A/F excursion  to  force

the downstream EGO sensors rich.

8-40

FUEL STRATEGY, OPEN LOOP LAMBSE CALCULATION -  CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TRLOAD = Transmission Load switch;

0 -> Manual Transmission, no clutch or gear switches,

forced neutral state

1 -> Manual Transmission, no clutch or gear switch.

2 -> Manual Transmission, one clutch or gear switch.

3 -> Manual Transmission, both clutch and gear switches.

4 -> Auto Transmission, non-electronic, neutral drive switch.

5 -> Auto Transmission, non-electronic, neutral pressure switch,

(AXOD).

6 -> Auto Transmission, electronic, PRNDL sensor - park,

reverse, neutral, overdrive, manual 1, manual 2.

8-41

FUEL STRATEGY, OPEN LOOP LAMBSE CALCULATION -  CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_OL_LAMBSE_COM25

BEGIN: open_loop_ram_init

Execute this logic ONLY during RAM initialization.

unconditionally ------------------------| OL_LEAN_TMR := 255

| ; to provide consistent starts.

|

| OLFLG_LST := OLFLG

| ISCFLG_LST := ISCFLG

| ; Initialize to same value.

| ; For LAMBSE Reset Logic.

END: open_loop_ram_init

BEGIN: pid_definitions

;The J1979 PID definitions may be executed upon demand by SCP

;or each background loop, as determined by the S/W implementor.

pid_def(j1979_01_031, b0:OL_HOLD,

b1:HEGO_FDBACK,

b2:OL_DRIVE,

b3:OL_FMEM,

b4:HEGO_FAULT,

b5:0,

b6:0,

b7:0)

END: pid_definitions

FUEL SYSTEM STATUS FOR BANK1

OLFLG = 0 ------------------------|

|

LEGOFG21 = 0 ---------------------|

|

LEGOFG11 = 0 ---------------------|

|

FFG_CSD11 = 0 --------------------|AND -| HEGO_FDBACK = 1

|     |

FFG_CSD21 = 0 --------------------|     |

|     |

OV_TMR_11 = 0 --------------------|     |

|     |

OV_TMR_21 = 0 --------------------|     |

| --- ELSE ---

|

| HEGO_FDBACK = 0

8-42

FUEL STRATEGY, OPEN LOOP LAMBSE CALCULATION -  CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OLFLG = 0 ------------------------|

|AND -| HEGO_FAULT = 1

HEGO_FDBACK = 0 ------------------|     |

| --- ELSE ---

|

| HEGO_FAULT = 0

Open loop due to detected system fault.

DIS_FMEM = 1 ---------------------|

|

NUMEGO = 2 -----------------|     |

|     |

LEGOFG11 = 1 ---------|     |     |

|     |     |

FFG_CSD11 = 1 --------|OR --|     |

|     |     |

OV_TMR_11 > 0 --------|     |     |OR --| OL_FMEM := 1

|AND -|     |  ;CSD or Lack of

LEGOFG21 = 1 ---------|     |     |     |  ;switching or

|     |     |     |  ;overvoltage on

FFG_CSD21 = 1 --------|OR --|     |     |  ;an upstream

|           |     |  ;sensor

OV_TMR_21 > 0 --------|           |     |

|     |

NUMEGO = 1 -----------------|     |     |

|     |     |

LEGOFG11 = 1 ---------|     |AND -|     |

|     |           |

FFG_CSD11 = 1 --------|OR --|           |

|                 |

OV_TMR_11 > 0 --------|                 |

| --- ELSE ---

|

| OL_FMEM := 0

Present conditions not satisfied to go closed loop.

OL_FMEM = 0 ----------------------|

|AND -| OL_HOLD = 1

WRMEGO = 0 -----------------------|     |

| --- ELSE ---

|

| OL_HOLD = 0

8-43

FUEL STRATEGY, OPEN LOOP LAMBSE CALCULATION -  CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;Assign last OLFLG value for LAMBSE_RESET

unconditionally ------------------------| OLFLG_LST = OLFLG

Open/Closed Loop decision logic.

OL_FMEM = 0 ----------------------|

|

OL_DESIRED = 0 -------------------|AND -| ;closed loop fuel.

|     | OLFLG = 0

OL_HOLD = 0 ----------------------|     | OL_DRIVE = 0

| FUEL_SOURCE1 = FUEL_SOURCE2 = 0

|

| ;Execute the LAMBSE_RESET logic.

| Do: FUEL_LAMBSE_RESET_COMx

| ISCFLG_LST = ISCFLG

|

| ;Allow OSC to force Open Loop

| ;This will NOT override LAMBSE.

| Do: substitute(4,LAMBSE1)

| Do: substitute(5,LAMBSE2)

|

| Do: determine_spk_lambse

|

| Exit FUEL_OL_LAMBSE module.

|

| --- ELSE ---

WRMEGO = 0 -----------------------|     |

(cold HEGO)                      |OR --| ;Cold Engine/Startup Fuel Tables

|     | Do: c_multiplier_logic

FLG_ECTSTABLQ = 0 ----------------|     |

(ECT cold)                             | ;temporary FN1361 & FN1362 inputs

| temp_s = FRCSFT * ACT +

|              (1 - FRCSFT) * ECT

| temp_b = FRCBFT * ACT +

|              (1 - FRCBFT) * ECT

| Do: get_fn1362_output

|

| LAMBSE_DRV = C * LAMMUL * OLMCL *

| [FN300(ACT) * fn1362 -

|  FN1361_H_RES(temp_s,

|               ATMR1_HI_RES)] *

|  FN321(OL_LEAN_TMR)

| fuel_source = 4

|

| --- ELSE ---

|

| ;stabilized engine fuel table

| Do: c_multiplier_logic

| LAMBSE_DRV =

|   FN1360(N,PERLOAD) *

|   FN325(ECT) * FN300(ACT) *

|   C * LAMMUL * OLMCL *

|   FN321(OL_LEAN_TMR)

| fuel_source = 5

8-44

FUEL STRATEGY, OPEN LOOP LAMBSE CALCULATION -  CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: get_fn1362_output

;Execute only when called.

ALT_CAL_FLG = 1 ------------------------| fn1362 = FN1362_ALT(temp_b,PERLOAD)

|

| --- ELSE ---

|

| fn1362 = FN1362(temp_b,PERLOAD)

END: get_fn1362_output

;Store last pass value of LAMBSE_EXT

unconditionally ------------------------| lam_ext_last = LAMBSE_EXT

;Find lambse (LAMBSE_EXT) for catalyst temperature protection.

EXT_FLG = 0 ----------------------------| LAMBSE_EXT = LAM_EXT_MAX

|

| --- ELSE ---

EXT_SS_FL + EXT_SS_EXO -                |

EXT_LS_CIN > CAT_MAX_SAFE --| LAMBSE_EXT = lam_ext_last -

|             (LAM_EXT_STEP * BG_TMR)

| (Clip to LAM_EXT_MIN as a minimum)

|

| --- ELSE ---

|

lambse_try > LAM_EXT_MAX ---------------| LAMBSE_EXT = LAM_EXT_MAX

|

| --- ELSE ---

(EXT_SS_FLS * FN441A(lambse_try)) +     |

(FN448(AM) * FN448A(lambse_try)) -   |

EXT_LS_CIN < CAT_MAX_SAFE ----------| LAMBSE_EXT = lambse_try

|

| --- ELSE ---

|

| No Change to LAMBSE_EXT.

Where lambse_try is calculated as follows:

lambse_try = lam_ext_last + (LAM_EXT_STEP * BG_TMR)

8-45

FUEL STRATEGY, OPEN LOOP LAMBSE CALCULATION -  CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;Determine Final LAMBSE (note Output State Control substitution):

unconditionally ------------------------| Do: determine_final_lambse(1)

| Do: substitute(4,lambse1)

|   (output state control override)

| Do: determine_final_lambse(2)

| Do: substitute(5,lambse2)

|   (output state control override)

| LAMBSE1 = lambse1

| LAMBSE2 = lambse2

|

| ; Required for next LAMBSE_RESET

| ISCFLG_LST = ISCFLG

|

| Do: determine_spk_lambse

8-46

FUEL STRATEGY, OPEN LOOP LAMBSE CALCULATION -  CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;Determine reason for going Open Loop (J1979 requirement):

OL_FMEM = 0 ----------------------|

|

OL_HOLD = 0 ----------------------|AND -| OL_DRIVE = 1

|     |

OLFLG = 1 ------------------------|     |

| --- ELSE ---

|

| OL_DRIVE = 0

BEGIN: determine_final_lambse(bank)

HALF_FUEL = 1 --------------|

(speed limiting or OFM FM) |OR --|

|     |

INJ_ON < NUMCYL ------------|     |AND -| ; Cylinder disablement - Cold CAT

(failsafe cooling active)        |     | lambse(bank) = LAMBSE_DRV

|     | OLFLG = 1

EXT_CMD < EXT_COLD_CAT -----------|     | TSLAMU(bank) = clock

(CAT temp margin available)            | FUEL_SOURCE(bank) = 9

|

| --- ELSE ---

HALF_FUEL = 1 --------------------|     |

(speed limiting or OFM_FMEM = 1) |OR --| ;disabling cylinders.

|     | lambse(bank) = FNCYL_OFF(N,LOAD)

INJ_ON < NUMCYL ------------------|     | OLFLG = 1

(cylinders disabled; failsafe cooling) | TSLAMU(bank) = clock

| FUEL_SOURCE(bank) = 1

|

| --- ELSE ---

PPCTR < PIPNUM -------------------|     |

(decel fuel shut off)            |AND -| Decel Fuel Shutoff Exit

|     | (ramp fuel)

DIS_FMEM = 0 ---------------------|     | lambse(bank) = 1 + DFSLAM -

|  [(PPCTR/PIPNUM) * DFSLAM]

| OLFLG = 1

| TSLAMU(bank) = clock

| FUEL_SOURCE(bank) = 1

|

| --- ELSE ---

|

ER_FUL_REQ = 1 -------------------------| lambse(bank) = ER_LAM_DSD(bank)

(Engine Running On-Demand testing      | OLFLG = 1

requires specific LAMBSE values)      | TSLAMU(bank) = clock

| FUEL_SOURCE(bank) = 8

|

| --- ELSE ---

|

SAIR_TST_ENA = 1 -----------------------| lambse(bank) = SAIR_LAM_DSD

(secondary air monitor flow test       | OLFLG = 1

requires specific LAMBSE values)      | TSLAMU(bank) = clock

| FUEL_SOURCE(bank) = 12

|

| --- ELSE ---

|

(Continued on following page)

8-47

FUEL STRATEGY, OPEN LOOP LAMBSE CALCULATION -  CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(Continued from previous page)

DS_EGO_OL = 1 --------------------|     |

|AND -| Do: ego_monitor_control(bank)

DS_EGO_RICH(bank) = 1 ------|     |     | OLFLG = 1

|OR --|     | TSLAMU(bank) = clock

DS_EGO_LEAN(bank) = 1 ------|           | FUEL_SOURCE(bank) = 7

|

| --- ELSE ---

EXT_FLG = 1 ----------------------|     |

;Cat Temp Model says over temp   |     |

|AND -| lambse(bank) = LAMBSE_EXT

LAMBSE_DRV > LAMBSE_EXT ----|     |     | OLFLG = 1

;Lean LAMBSE demanded      |OR --|     | TSLAMU(bank) = clock

|           | FUEL_SOURCE(bank) = 2

OL_DES = 0 -----------------|           |

;Open Loop for EXT_FLG only            |

| --- ELSE ---

|

| lambse(bank) = LAMBSE_DRV

| OLFLG = 1

| TSLAMU(bank) = clock

| FUEL_SOURCE(bank) = fuel_source

END: determine_final_lambse

8-48

FUEL STRATEGY, OPEN LOOP LAMBSE CALCULATION -  CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: c_multiplier_logic

DIS_FMEM = 1 ---------------------------| DIS Failure Mode

| 'C' = FN911(LOAD)

|

| --- ELSE ---

|

HSPFLG = 1------------------------------| High Vehicle Speed

| Limiting Mode

| 'C' = FN396C(NOBAR)

|

| --- ELSE ---

|

LDFLG = 1 ------------------------------| Lugging Mode

| 'C' = FN308(N) * FN212A(BP)

|  + FN309(N) * FN129A(BP)

|

| --- ELSE ---

DNDSUP = 1 -----------------|           |

(in drive)                 |OR --|     |

|     |     |

TRLOAD <= 3 ----------------|     |     |

(manual trans)                   |AND -| 'C' = FN301(N)

|     |

APT = -1 -------------------------|     |

|     |

WRMEGO = 0 -----------------|     |     |

(cold HEGO)                |OR --|     |

|           |

FLG_ECTSTABLQ = 0 ----------|           |

(ECT cold)                             |

| --- ELSE ---

DNDSUP = 0 -----------------------|     |

(in neutral)                     |     |

|     |

APT = -1 -------------------------|     |

(closed throttle)                |AND -| Mid ambient neutral idle

|     | 'C' = OLINEUT * NUMPR

CTLOW < TCSTRT < CTHIN -----------|     |

|     |

ATMR1 < OLMTD1 -------------------|     |

| --- ELSE ---

DNDSUP = 0 -----------------------|     |

(In neutral)                     |AND -| Neutral Idle

|     | 'C' = OLINEUT

APT = -1 -------------------------|     |

(closed throttle)                      |

| --- ELSE ---

(continued on next page)

8-49

FUEL STRATEGY, OPEN LOOP LAMBSE CALCULATION -  CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

DNDSUP = 1 -----------------------|     |

(in drive)                       |AND -| Drive Idle

|     | 'C' = OLIDRV

APT = -1 -------------------------|     |

(closed throttle)                      | --- ELSE ---

|

APT = 1 --------------------------|     | Wide Open Throttle

|AND -| 'C' = FN303(N)

VS_RATE >= MINACCL ---------------|     |

| --- ELSE ---

|

| 'C' = 1

END: c_multiplier_logic

8-50

FUEL STRATEGY, OPEN LOOP LAMBSE CALCULATION -  CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: determine_spk_lambse

Service SPK_LAMBSE and OL_LEAN_TMR in this section.

SPK_LAMBSE is 1.0 when the  fuel  calculation  is  closed  loop  because  the

air/fuel mixture will be at stoichiometry regardless of the value of the Open

Loop LAMBSE calculation.  By assuming SPK_LAMBSE to be 1.0, the spark is  not

erroneously  corrected  for  mixture  errors when closed loop fuel control is

corrected to stoic.

When the engine is cold, the Open Loop LAMBSE is set overly rich (to  account

for  'lost  fuel'  down  the  cylinder  walls),  thus  the  SPK_LAMBSE should

represent a value which is closer to what air/fuel mixture is actually in the

cylinder.

Since each bank can obtain a unique LAMBSE during HEGO monitoring, the  value

used for SPK_LAMBSE is the average of both banks.

OLFLG = 0 ------------------------------| SPK_LAMBSE = 1.0

(closed loop)                          | OL_LEAN_TMR = 0

|

| --- ELSE ---

|

FLG_ECTSTABLQ = 0 ----------------------| SPK_LAMBSE = LAM_COLD_ENG

(cold engine)                          | Increment OL_LEAN_TMR

|

| --- ELSE ---

|

LAMBSE1 <> LAMBSE2 ---------------------| SPK_LAMBSE =(LAMBSE1 + LAMBSE2) / 2

(HEGO monitoring test)                 | Increment OL_LEAN_TMR

|

| --- ELSE ---

|

| SPK_LAMBSE = LAMBSE1

| Increment OL_LEAN_TMR

END: determine_spk_lambse

BEGIN: ego_monitor_control(bank)

The following logic will overwrite LAMBSE1 or LAMBSE2 if necessary

during the intrusive portion of the downstream EGO monitor.

DS_EGO_RICH(bank) = 1 ------------------| lambse(bank) = RICH_EXCRSN

|

| --- ELSE ---

|

DS_EGO_LEAN(bank) = 1 ------------------| lambse(bank) = LEAN_EXCRSN

END: ego_monitor_control

8-51

FUEL STRATEGY, LAMBSE RESET LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.10  LAMBSE RESET LOGIC (CDAN2)

OVERVIEW

The following logic is executed under  the  conditions  defined  in  strategy

module  FUEL_OL_LAMBSE_COMx.   LAMBSE  is  reset  to  (1 + BIAS_Gx) under the

conditions listed below, provided it is in Closed Loop fuel control.   LAMBSE

is  not  reset  in  Open  Loop  Fuel  Control  because the value of LAMBSE is

calculated using the Open Loop Fuel logic.

A LAMBSE reset is commanded for the conditions described below:

1.  When entering or exiting Closed Loop RPM Control mode.

2.  When Changing load states.

3.  Any time a transition is made from Open loop to Closed Loop Fuel control.

When  this  transition  is detected, special logic will increase the ramp

rate to quickly bring LAMBSEx towards the stoichiometric point.

4.  When a tip out occurs while purging.

When LAMBSE is clipped to (1 + BIAS_Gx) as a maximum, the intent is to  allow

rich  errors  and  to prevent lean errors, given that running rich should not

cause any drivability concerns.  Since Open Loop lambda's may be  excessively

rich  or  lean  of stoic (based upon driving conditions as well as to control

Catalyst temperatures), the closed loop ramp rate multiplier logic is used to

command  the  quickest  return  to  1.0  without noticeably affecting vehicle

performance or inducing excess feedgas emissions.

8-52

FUEL STRATEGY, LAMBSE RESET LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  ANPIPn = Actual number of PIP signals since the last HEGO switch; clipped

to a maximum of 255; reset to zero when not in closed loop.

-  APT = At Part Throttle; -1 -> Closed throttle, 0 -> Part throttle,  1  ->

Wide open Throttle.

-  BIAS_G1 = Closed loop fuel biasing term for bank one.

-  BIAS_G2 = Closed loop fuel biasing term for bank two.

-  ENPIPn = Expected number of PIP signals between HEGO switches; reset to 1

when not in closed loop.

-  ISCFLG = ISC mode indicator flag;  -1  ->  Dashpot  Mode,  0  ->  Dashpot

Preposition Mode, 1 -> closed Loop RPM Control Mode, 2 -> Closed Loop RPM

Control (Lock_out entry to RPM control).

-  ISCFLG_LST = Last pass value of ISCFLG.

-  ISFLAG = Flag that indicates the degree of loading on the engine at idle;

0  -> Drive, 1 -> Drive and A/C (WAC Relay De-Energized), 2 -> Neutral, 3

-> Neutral and A/C (WAC Relay De-Energized).

-  ISLAST = Register which indicates the engine load state from the previous

background pass.

-  LAMBSE1 = Desired open loop  equivalence  ratio  for  bank  1  injectors.

LAMBSE1 appears in the fuel pulsewidth equation for HEGO11.

-  LAMBSE2 = Desired open loop  equivalence  ratio  for  bank  2  injectors.

LAMBSE2 appears in the fuel pulsewidth equation for HEGO21.

-  LESTMR11 = Lack of HEGO11 switching timer, sec.

-  LESTMR21 = Lack of HEGO21 switching timer, sec.

-  TSLAMUn = Time at which last LAMBDA update occurred, clock ticks.

Bit Flags:

|      -  DFIB_LEANn1 = Flag which is set when the lo catcher calculation is stuck.

|         Initiates  a  lambse  reset from the lean clip as part of the switchpoint

|         recovery process.

|      -  DFIB_RICHn1 = Flag which is set when the hi catcher calculation is stuck.

|         Initiates  a  lambse  reset from the rich clip as part of the switchpoint

|         recovery process.

-  EGOFL11 = Bank1 upstream HEGO flag.

-  EGOFL21 = Bank2 upstream HEGO flag.

-  OLFLG_LST = Last pass value of OLFLG;

1 -> Previously in Open Loop Fuel,

0 -> Previously Closed Loop.

8-53

FUEL STRATEGY, LAMBSE RESET LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  RESET_LEAN1 = Flag passed from lambse  reset  to  closed  loop  logic  to

indicate  whether  LAMBSE1  was  lean  or rich upon last transition; 1 =>

lean, 0 => rich.

-  RESET_LEAN2 = Flag passed from lambse  reset  to  closed  loop  logic  to

indicate  whether  LAMBSE2  was  lean  or rich upon last transition; 1 =>

lean, 0 => rich.

-  RESET_REQ1 = Flag passed from lambse reset to closed loop logic to enable

use of ramp rate multiplier for Bank 1.

-  RESET_REQ2 = Flag passed from lambse reset to closed loop logic to enable

use of ramp rate multiplier for Bank 2.

-  V_LAMJMP1 = Self test flag indicating a LAMBSE1 jump has occured.

-  V_LAMJMP2 = Self test flag indicating a LAMBSE2 jump has occured.

-  PC_RESET = Purge Lambse reset flag; 1 -> purge requesting lambse reset.

Calibration Constants:

-  LAM_ISC_SW = LAMBDA Reset Switch for entering/exiting Idle Speed  control

modes; 1 => Allow LAMBSE reset due to Idle Speed Control logic.

-  LAM_LOAD_SW = LAMBDA Reset Switch for Load State Changes at  Idle;  1  =>

Allow LAMBSE reset due to Load State Change at closed throttle.

-  PRG_LAM_SW = Calibration switch  to  enable  PCOMP/Purge  induced  lambse

resets.

-  LMBJMP = Desired rich correction.

-  LAMMIN = Min LAMBSE clip when closed loop.

8-54

FUEL STRATEGY, LAMBSE RESET LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_LAMBSE_RESET_COM13

BEGIN: FUEL_LAMBSE_RESET_COM13

;THIS MODULE SHOULD ONLY BE EXECUTED IN CLOSED LOOP MODE.

;IT IS CALLED FROM MODULE FUEL_OL_LAMBSE_COMx.

1)  Lambse Reset - Entering/Exiting Closed Loop RPM Control mode.

ISCFLG = 1 -------------------|

|

ISCFLG_LST <> 1 --------------|

(entering RPM control mode)  |

|AND -| Clip LAMBSE1 to (1 + BIAS_G1) as

LAMBSE1 > (1 + BIAS_G1) ------|     |     a maximum

(operating lean)             |     | LESTMR11 = 0

|     | ANPIP1 = 0

LAM_ISC_SW = 1 ---------------|     | ENPIP1 = 1

| V_LAMJMP1 = 1

| TSLAMU1 = clock

ISCFLG = 1 -------------------|

|

ISCFLG_LST <> 1 --------------|

(entering RPM control mode)  |

|AND -| Clip LAMBSE2 to (1 + BIAS_G2) as

LAMBSE2 > (1 + BIAS_G2) ------|     |  a maximum

(operating lean)             |     | ANPIP2 = 0

|     | ENPIP2 = 1

LAM_ISC_SW = 1 ---------------|     | LESTMR21 = 0

| V_LAMJMP2 = 1

| TSLAMU2 = clock

LAM_ISC_SW = 1 ---------------|

|

ISCFLG = 0 -------------------|AND -| LAMBSE1 = LAMBSE1

|     |  - LMBJMP

ISCFLG_LST = 1 ---------------|     | LAMBSE2 = LAMBSE2

(leaving RPM control mode and      |  - LMBJMP

entering Dashpot preposition)     | Clip LAMBSE1 and LAMBSE2 to

|  to LAMMIN as a minimum.

| V_LAMJMP1 = 1

| V_LAMJMP2 = 1

8-55

FUEL STRATEGY, LAMBSE RESET LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

2)  Lambse Reset - Load State Change at Idle

APT = -1 ---------------------|

(closed throttle)            |

|

ISFLAG <> ISLAST -------------|AND -| Clip LAMBSE1 to (1 + BIAS_G1) as

(load state change)          |     |   a maximum

|     | ANPIP1 = 0

LAMBSE1 > (1 + BIAS_G1) ------|     | ENPIP1 = 1

|     | LESTMR11 = 0

LAM_LOAD_SW = 1 --------------|     | V_LAMJMP1 = 1

| TSLAMU1 = clock

APT = -1 ---------------------|

(closed throttle)            |

|

ISFLAG <> ISLAST -------------|

(load state change)          |AND -| Clip LAMBSE2 to (1 + BIAS_G2) as

|     |   a maximum

LAMBSE2 > (1 + BIAS_G2) ------|     | ANPIP2 = 0

|     | ENPIP2 = 1

LAM_LOAD_SW = 1 --------------|     | LESTMR21 = 0

| V_LAMJMP2 = 1

| TSLAMU2 = clock

8-56

FUEL STRATEGY, LAMBSE RESET LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

3)  Lambse Reset - Open Loop to Closed Loop Transition

;(Executed only once per Open-Closed transition, or during

|         ;an adaptive switchpoint recovery action triggered by the

|         ;DFIB_LEAN/RICH flags setting for one background loop.)

|     DFIB_LEAN11 = 1 --------------------|

|     (reset lambse from lean clip)       |

|

OLFLG_LST = 1 ----------------|     |OR --| RESET_REQ1 = 1

(previously in Open Loop)     |     |     | RESET_LEAN1 = 1

|     |     | ANPIP1 = 0

LAMBSE1 > (1 + BIAS_G1) ------|AND -|     | ENPIP1 = 1

(Lean A/F commanded)          |           | LESTMR11 = 0

|           | V_LAMJMP1 = 1

EGOFL11 = 0 ------------------|           | TSLAMU1 = clock

(Lean A/F sensed by HEGO)                 |

| --- ELSE ---

|     DFIB_RICH11 = 1 --------------------|     |

|     (reset lambse from rich clip)       |     |

|     |

OLFLG_LST = 1 ----------------|     |OR --| RESET_REQ1 = 1

(previously in Open Loop)     |     |     | RESET_LEAN1 = 0

|     |     | ANPIP1 = 0

LAMBSE1 <= (1 + BIAS_G1) -----|AND -|     | ENPIP1 = 1

(Rich A/F commanded)          |           | LESTMR11 = 0

|           | V_LAMJMP1 = 1

EGOFL11 = 1 ------------------|           | TSLAMU1 = clock

(Rich A/F sensed by HEGO)

|     DFIB_LEAN21 = 1 --------------------|

|     (reset lambse from lean clip)       |

|

OLFLG_LST = 1 ----------------|     |OR --| RESET_REQ2 = 1

(previously in Open Loop)     |     |     | RESET_LEAN2 = 1

|     |     | ANPIP2 = 0

LAMBSE2 > (1 + BIAS_G2) ------|AND -|     | ENPIP2 = 1

(Lean A/F commanded)          |           | LESTMR21 = 0

|           | V_LAMJMP2 = 1

EGOFL21 = 0 ------------------|           | TSLAMU2 = clock

(Lean A/F sensed by HEGO)                 |

| --- ELSE ---

|     DFIB_RICH21 = 1 --------------------|     |

|     (reset lambse from rich clip)       |     |

|     |

OLFLG_LST = 1 ----------------|     |OR --| RESET_REQ2 = 1

(previously in Open Loop)     |     |     | RESET_LEAN2 = 0

|     |     | ANPIP2 = 0

LAMBSE2 <= (1 + BIAS_G2) -----|AND -|     | ENPIP2 = 1

(Rich A/F commanded)          |           | LESTMR21 = 0

|           | V_LAMJMP2 = 1

EGOFL21 = 1 ------------------|           | TSLAMU2 = clock

(Rich A/F sensed by HEGO)

8-57

FUEL STRATEGY, LAMBSE RESET LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

4) Lambse Reset - tip out during purging

(Logic enabled from within PCOMP or Purge logic)

PC_RESET = 1 -----------------|

|

LAMBSE1 > (1 + BIAS_G1) ------|AND -| Clip LAMBSE1 to (1 + BIAS_G1) as

(operating lean)             |     |  a maximum

|     | ANPIP1 = 0

PRG_LAM_SW = 1 ---------------|     | ENPIP1 = 1

(reset calibrated in)              | LESTMR11 = 0

| V_LAMJMP1 = 1

| TSLAMU1 = clock

PC_RESET = 1 -----------------|

|

LAMBSE2 > (1 + BIAS_G2) ------|AND -| Clip LAMBSE2 to (1 + BIAS_G2) as

(operating lean)             |     |  a maximum

|     | ANPIP2 = 0

PRG_LAM_SW = 1 ---------------|     | ENPIP2 = 1

(reset calibrated in)              | LESTMR21 = 0

| V_LAMJMP2 = 1

| TSLAMU2 = clock

END: FUEL_LAMBSE_RESET_COM13

8-58

FUEL STRATEGY, LAMBSE CLIP LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.11  LAMBSE CLIP LOGIC (CDAH0)

OVERVIEW

LAMBSE1 and LAMBSE2 (the desired equivalence ratios)  are  clipped  as  shown

below:

DEFINITIONS

INPUTS

Bit Flags:

-  OLFLG = Open Loop Flag; 1 -> Open Loop Fuel Control,  0  ->  Closed  Loop

Fuel Control.

Calibration Constants:

-  LAMMAX = Maximum LAMBSE clip when in closed loop.

-  LAMMIN = Minimum LAMBSE clip when in closed loop.

OUTPUTS

Registers:

-  LAMBSE1 = Desired equivalence ratio for HEGO11 injectors.

-  LAMBSE2 = Desired equivalence ratio for HEGO21 injectors.

8-59

FUEL STRATEGY, LAMBSE CLIP LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_LAMBSE_CLIP_COM4

OLFLG = 0 ----------------------------| CLIP LAMBSE1 AND

| LAMBSE2 BETWEEN

| LAMMIN AND LAMMAX

|

| --- ELSE ---

|

| CLIP LAMBSE1 AND

| LAMBSE2 BETWEEN

| 0.0000305 AND

| 1.9999695

8-60

FUEL STRATEGY, LAMBSE CLIP LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.12  FEATURE:  ADAPTIVE FUEL CONTROL AND OBD-II FUEL SYSTEM MONITOR -

V1.5_FADPT (CDAN0)

8.12.1  ADAPTIVE FUEL CONTROL LOGIC (CDAN0)

OVERVIEW

Fuel injected systems may exhibit vehicle to vehicle steady state  A/F  ratio

errors  due  to  normal  variability in fuel system components.  The adaptive

fuel strategy attacks this problem by memorizing the characteristics  of  the

individual  fuel  system  being  used.  This memorized information is used to

predict what the system will do based on past experience.

The ability to predict fuel system  behavior  improves  both  open  loop  and

closed  loop  fuel  control.  As an example, the memorized information can be

used on cold starts to achieve better open loop fuel control before the  HEGO

sensor reaches operating temperature.

The chief benefit of the adaptive fuel strategy will be to reduce the effects

of product variability in the field.

The memorized or adaptive information is stored in table or  vector  form  in

the  Keep  Alive  Memory  (KAM).   KAM is continuously powered by the vehicle

battery even when the vehicle is shut off.  As a result, this information  is

not lost on vehicle shutdown.

APPROACH

The adaptive fuel tables, LTMTB1 and  LTMTB2,  are  2-dimensional  arrays  of

learned  fuel  system  corrections.  Ideally, if LAMBSE1 = 1.0, LAMBSE2 = 1.0

and data from a mature adaptive fuel table  is  used,  a  stoichiometric  A/F

ratio  would  result at whatever speed-load point adaptive learning had taken

place.

Present table size is 8(rows) x 10(columns) or 80  cells  for  each  Adaptive

Fuel Table, plus depending on the strategy family, either 4 or 6 special idle

adaptive cells.  There will always be 86 total cells per table.

The total learned fuel system correction for the HEGO11 side of the engine is

called  KAMRF1  where  KAMRF1  =  0.5  + LTMTB1[r,c].  The corresponding fuel

correction for the HEGO21 side  is  called  KAMRF2,  where  KAMRF2  =  0.5  +

LTMTB2[r,c].

During adaptive learning, only the LTMTB1  and  LTMTB2  cells  are  modified.

Therefore,  the  ranges  of  each of the KAMRF1 and KAMRF2 multipliers (0.5 +

0.0) to (0.5 + 1.0) or 0.5 to 1.5.

The range of the LTMTB1 and LTMTB2 cells can be further restricted by use  of

the calibration parameter clips, MINADP and MAXADP.

The KAMRF1 and KAMRF2  are  used  in  the  desired  fuel/air  ratio  equation

(strategy module FUEL_BG_CALC_COM).

If KAM fails the KAM validation test (described later), or if there is a lack

of  HEGO  switching  (described  in  the  Failure Mode Strategy chapter), all

8-61

FUEL STRATEGY, ADAPTIVE FUEL CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

LTMTB1 and/or LTMTB2 cells are initialized to 0.5 or 80 (HEX) resulting in  a

value of KAMRF1 = KAMRF2 = 0.5 + 0.5 = 1.0.

Updates to LTMTB1 and LTMTB2 are randomly distributed in the vicinity of  the

speed-load  operating  point, except in the case of the idle cells.  Only the

current idle cell is updated, no  distribution  is  done.   Over  time,  this

random  distribution will approximate a statistical, weighted distribution of

the fuel/air error into the KAM table.

Data extracted from the table undergoes a 4-point linear interpolation.  This

is explained further under the FN1325A description.  Note that the idle cells

do not undergo four point interpolation.  Only the current idle cell is used.

LTMTB1, LTMTB2 and FN1325A share the same normalizing functions.   FN021A  is

the  LOAD  normalizing  function.  FN041 is the AM normalizing function.  The

calibration constant, ADAPT_SEL, selects whether the AM or  LOAD  normalizing

function  should  be  used  for  the  Y-axis.  FN070D is the engine speed, N,

normalizing function.

As an example of an adaptive fuel table, LTMTB1, is shown on the  next  page.

HEGO21 Adaptive Fuel Table, LTMTB2, uses similar nomenclature.

ADAPTIVE FUEL TABLE (LTMTB1)

LTMTB1[r,c] CELLS

(LTMTB2 has similar designation)

Special idle  ------>|80 81 82 83 84 85

adaptive cells       |

7    |70 71 72 73 74 75 76 77 78 79

FN021A           |

NORMALIZED    6    |60 61 62 63 64 65 66 67 68 69

ENGINE           |

LOAD       5    |50 51 52 53 54 55 56 57 58 59

(LOAD)           |

4    |40 41 42 43 44 45 46 47 48 49

--- OR ---         |

3    |30 31 32 33 34 35 36 37 38 39

FN041           |

NORMALIZED    2    |20 21 22 23 24 25 26 27 28 29

AIR MASS          |

(AM)       1    |10 11 12 13 14 15 16 17 18 19

|

0    |00 01 02 03 04 05 06 07 08 09

------------------------------

0  1  2  3  4  5  6  7  8  9

FN070D NORMALIZED

ENGINE SPEED

(N)

The special Adaptive Idle cells are not defined by FN021A or FN070D, they are

8-62

FUEL STRATEGY, ADAPTIVE FUEL CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

defined by the value of the ISFLAG.  (See ISFLAG definition)

8-63

FUEL STRATEGY, ADAPTIVE FUEL CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FN1325A DESCRIPTION

FN1325A is a 9(rows) x 10(columns) table containing 1 cell  corresponding  to

each  cell in the adaptive fuel table LTMTB1 or LTMTB2.  (The 9th row is used

to reference the idle cells and is not accessible from FN021A which only goes

from 0 to 7).

The normalizing functions for FN1325A, LTMTB1 and LTMTB2 are shared.   FN021A

is the LOAD normalizing function.  FN041 is the AM normalizing function.  The

calibration constant, ADAPT_SEL, selects whether the AM or  LOAD  normalizing

function  should  be  used  for  the  Y-axis.  FN070D is the engine speed, N,

normalizing function.

FN1325A is designed to do the following:

-  Identify LTMTB1 and LTMTB2 cells where learning is allowed to occur.

Learning is allowed in any LTMTB1 and LTMTB2  cells  whose  corresponding

FN1325A[r,c]  cell  contains  a value >= 0.  Negative FN1325A cell values

disallow learning in the corresponding LTMTB1 and LTMTB2 cell(s).

-  Define a high confidence speed-load region that can  be  referenced  from

any other speed-load point.

This occurs whenever a negative value is entered  into  a  FN1325A  cell.

The  negative  number serves as an offset to LTMTB1[0,0] and LTMTB2[0,0].

If 1 of the 4 cells used by the 4-point linear  interpolation  LTMTB1  or

LTMTB2  table  lookup  routine  contained -42, the cell value used by the

interpolation routine for the cell that contained the -42  would  be  the

value  found  in the LTMTB1 or LTMTB2 cell located at the intersection of

row 4 and column 2.  In the extreme, if -42 was entered into  every  cell

of  FN1325A[r,c]  except  for  the  cell corresponding to LTMTB1[4,2] and

LTMTB2[4,2], learning would be  allowed  only  in  cell  LTMTB1[4,2]  and

LTMTB2[4,2]  and  the  learned  correction in LTMTB1[4,2] and LTMTB2[4,2]

would  be  applied  to  all  speed-load  points  for  fuel   calculations

referencing   HEGO11   (every  cell  referenced  by  the  4-point  linear

interpolation routine during the LTMTB1 table lookup would point to  cell

LTMTB1[4,2]).  This calibration for FN1325A is shown on the next page.

-  Specify the values of the  Loop  Counters  (LPCT1L,  LPCT1R,  LPCT2L  and

LPCT2R) required to update an individual LTMTB1 or LTMTB2 cell(s).

This is done by entering a value into FN1325A that is >=  0.   The  value

entered  represents 1/2 the required update value.  A value of 20 entered

would require (LPCT1R, LPCT1L, LPCT2L and LPCT2R) to be greater  than  40

for an update to occur.

NOTE:  FN1325A controls the update rate and update area for both  LTMTB1  and

LTMTB2.

8-64

FUEL STRATEGY, ADAPTIVE FUEL CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

LTMTB1 AND LTMTB2 LEARNING/USE CONTROL TABLE FN1325A

FN1325A CELLS

Special idle

adaptive cells ---->|-42 -42 -42 -42 -42 -42

|

7    |-42 -42 -42 -42 -42 -42 -42 -42 -42 -42

FN021A          |

NORMALIZED   6    |-42 -42 -42 -42 -42 -42 -42 -42 -42 -42

ENGINE          |

LOAD      5    |-42 -42 -42 -42 -42 -42 -42 -42 -42 -42

(LOAD)          |

4    |-42 -42  20 -42 -42 -42 -42 -42 -42 -42

--- OR ---        |

3    |-42 -42 -42 -42 -42 -42 -42 -42 -42 -42

FN041           |

NORMALIZED   2    |-42 -42 -42 -42 -42 -42 -42 -42 -42 -42

AIR MASS         |

(AM)      1    |-42 -42 -42 -42 -42 -42 -42 -42 -42 -42

|

0    |-42 -42 -42 -42 -42 -42 -42 -42 -42 -42

---------------------------------------

0   1   2   3   4   5   6   7   8   9

FN070D NORMALIZED

ENGINE SPEED

(N)

CALIBRATION NOTES

To disable adaptive learning, the FADPT_ENABLE flag must  be  held  at  zero.

Please  refer  to  the FADPT_ENABLE strategy module for information on how to

disable this feature entirely.

It is not possible to reference a cell which  is  itself  referencing.   Such

double  referencing  must  be  avoided,  as  it  will inhibit learning in the

present implementation.

It is possible to reference a normal cell to a special idle cell.  It is  not

possible  to  reference a special idle cell to normal cell, as the four-point

interpolation scheme will not be invoked.

HCAMSW is a calibration switch allowing  the  developer  to  select  how  the

adaptive fuel idle cells are to be used.  If HCAMSW is set to 0, the adaptive

fuel idle cells are used as soon as the filtered air mass region  is  entered

(REFFLG  =  1).  If HCAMSW is set to 1, the adaptive fuel idle cells are used

only when in the filtered air mass region and no RPM adder above base idle is

present (HCAMFG = 0).  This includes FN825A, FN825B, FN026 and BZZRPM.

8-65

FUEL STRATEGY, ADAPTIVE FUEL CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  AM = Air mass flow, lb/min.

-  BIAS_G1/BIAS_G2 = Closed loop fuel biasing term = FN1353(N,LOAD).

-  CHKSUM = KAM memory word which contains the sum of the LTMTB1 and  LTMTB2

table contents.

-  COLTBU = A register which contains the  column  number  of  the  adaptive

learning cell to be updated.

-  ECT = Coolant temperature, degrees F.

-  HEGO = Heated Exhaust Gas Oxygen Sensor.  HEGO11 and HEGO21 represent the

HEGO sensors in stereo mode; generic representation = HEGOxy.

-  EGOCT11 = HEGO11 switches since last Adaptive Fuel update.

-  EGOCT21 = HEGO21 switches since last Adaptive Fuel update.

-  KAM = Keep Alive Memory.

-  KAM_MON_TMR = Adaptive cell residency timer (required for monitoring).

-  KAMCOL = Table index value used  for  interpolation.   This  value  comes

directly from FN070D (real).

-  KAMCOL_CUR = Current adaptive  fuel  table  column  indicator  (integer).

This is the integer portion of KAMCOL, obtained through truncation.

-  KAMCOL_OLD = Adaptive fuel table column  indicator  (integer)  from  last

background pass.

-  KAMROW = Table index value used  for  interpolation.   This  value  comes

directly from either FN021A or FN041 (real).

-  KAMROW_CUR = Current adaptive fuel table row indicator  (integer).   This

is the integer portion of KAMROW, obtained through truncation.

-  KAMROW_OLD = Adaptive  fuel  table  row  indicator  (integer)  from  last

background pass.

-  KAMRF1/KAMRF2 = Adaptive Fuel strategy correction factor.  It is composed

of the value LTMTB1[r,c] + 0.5.

-  KWUCTR = KAM warm-up counter.  Stores number of warm-ups in  KAM.   Reset

to zero if KAM is corrupted battery disconnect, etc.)

-  LAMBSE1 = Desired open loop  equivalence  ratio  for  bank  1  injectors.

LAMBSE1 appears in the fuel pulsewidth equation for HEGO11.

-  LAMBSE2 = Desired open loop  equivalence  ratio  for  bank  2  injectors.

LAMBSE2 appears in the fuel pulsewidth equation for HEGO21.

8-66

FUEL STRATEGY, ADAPTIVE FUEL CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  LAMWIN1/LAMWIN2 = LAMBSE window outside which adaptive is enabled.

-  LOAD = Universal Load parameter, unitless; aircharge  normalized  to  Sea

Level.

-  LPCT1L = Number of background loops that the LAMBSE1 outside  a  deadband

in the rich direction, with HEGO11 sensor reading lean.

-  LPCT2L = Number of background  loops  that  the  LAMBSE2  was  outside  a

deadband in the rich direction, with HEGO21 sensor reading lean.

-  LPCT1R = Number of background  loops  that  the  LAMBSE1  was  outside  a

deadband in the lean direction, with HEGO11 sensor reading rich.

-  LPCT2R = Number of background  loops  that  the  LAMBSE2  was  outside  a

deadband in the lean direction, with HEGO21 sensor reading rich.

-  LSTCOL = Last normalized column value.

-  LSTROW = Last normalized row value.

-  LTMTB1[r,c] = HEGO11 Adaptive Fuel Table cell.

-  LTMTB2[r,c] = HEGO21 Adaptive Fuel Table cell.

-  N = Engine revolutions, RPM.

-  PG_DC = Purge duty cycle, unitless.

-  PTPAMP1/PTPAMP2 = Limit cycle peak-to-peak amplitude.

-  RANNUM = Random number adder.

-  ROWTBU = Register which contains the row number of the Adaptive Fuel cell

to be updated.

-  TCSTRT = Temperature of ECT at cold start, degrees F.

-  UPRATE = KAM update rate.

Bit Flags:

-  CRKFLG = Crank flag; 1 -> in crank mode, 0 -> not in crank mode.

-  EGOFL11 = Bank 1 upstream HEGO11 flag.

-  EGOFL21 = Bank 2 upstream HEGO21 flag.

-  EGO1FMFLG = EGO1 FMEM flag; 1 -> EGO1 sensor failure.

-  EGO2FMFLG = EGO2 FMEM flag; 1 -> EGO2 sensor failure.

-  FADPT_ENABLE = Flag indicating enable state of adaptive fuel strategy.

-  FFG_CSD11 = Fault flag for CSD on sensor 11.

-  FFG_CSD21 = Fault flag for CSD on sensor 21.

8-67

FUEL STRATEGY, ADAPTIVE FUEL CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  HCAMFG = Flag indicating state of Hi Cam.

-  ISCFLG = ISC mode indicator flag;  -1  ->  Dashpot  Mode,  0  ->  Dashpot

Preposition Mode, 1 -> closed Loop RPM Control Mode, 2 -> Closed Loop RPM

Control (Lock-out entry to RPM control).

-  ISFLAG = Flag that indicates the degree of loading on the engine at Idle.

See  the  ISC  Chapter;  0  ->  Drive,  1  ->  Drive  +  A/C  (WAC  Relay

De-Energized), 2 -> Neutral, 3 -> Neutral + A/C (WAC Relay De-Energized).

-  KAM_ERROR = KAM error flag, set for 1  background  loop  after  error  is

detected; 1 -> KAM data invalid.

-  LEGOFG11 = Lack of HEGO11 switching.

-  LEGOFG21 = Lack of HEGO21 switching.

-  LIMIT_PURGE = Flag which indicates purge duty cycle is being limited  due

to lambse being clipped; 1 -> limited purge.

-  SWTFL11 = Bank 1 upstream HEGO11 switch flag; 0-> no HEGO11  switch,  1->

sensor HEGO11 switch.

-  SWTFL21 = Bank 2 upstream HEGO21 switch flag; 0-> no HEGO21  switch,  1->

sensor HEGO21 switch.

-  UNDSP = Underspeed flag; 1 -> underspeed mode or crank.

-  WARM_UP = Indicates engine warm-up occurred.

Calibration Constants:

-  ADAPT_SEL = AM versus LOAD select switch; 0 -> use LOAD  in  FN1325A  and

LTMTB1/2 tables, 1 -> use AM in FN1325A and LTMTB1/2 tables.

-  ADEGCT = HEGO switch requirement to increment counters.

-  AMPMUL = Multiplier to determine LAMWINx from PTPAMPx.

-  DELCOL = Calibration constant (normalized engine speed N) which  provides

the  ability  to  lock  out  table  updates  under  transient conditions;

establishes  an  operating  range  (engine  speed)   within   which   the

appropriate loop counter may be incremented.

-  DELROW = Same function as DELCOL but for either  normalized  LOAD  or  AM

(depending upon the value of ADAPT_SEL).

-  DELAMB = Deadband (around LAMBSE1 = 1.0) within which loop counter values

are not altered.

-  FADPT_SW = Switch allowing the complete disablement of adaptive fuel  and

the  fuel  system  monitor.   FADPT_SW  =  0 -> disable all adaptive fuel

activity.

-  FAEGCT = Fast Adaptive HEGO count.  Number of HEGO switches  required  to

permit  adaptive  learning  when  KWUCTR < KWUCNT.  Should be set to 0 to

permit fast adaptive learning for the first few warm-up cycles.

8-68

FUEL STRATEGY, ADAPTIVE FUEL CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FN021A = LOAD normalizing function for LTMTB1(LTMTB2) and FN1325A.

Input = Load.

Output = Normalized Load.

-  FN041 = AM normalizing function.

-  FN070D = N normalizing function.

-  FN1325A[r,c] = LTMTB1 and LTMTB2 learning/use control table.

-  HCAMSW = Calibratable switch which allows selection  of  when  and  which

adaptive fuel cells are to be used.

-  IDLE_CELL_SW = Calibration switch to  enable  use  of  the  special  idle

cells; 1 -> special cells enabled.

-  KAM_NO_RESET = Switch which indicates if KAM should be reset upon an  EGO

failure; 1 -> do not reset KAM.

-  KWUCNT = Maximum number of warm-up  cycles  to  use  fast  adaptive  HEGO

count.  It should be set to approx.  3 to 5 warm-ups.

-  MAXADP = Maximum adaptive correction.

-  MINADP = Minimum adaptive correction.

-  NUMEGO = Switch indicating number of HEGO sensors present; select mono or

stereo.

-  RANMUL = Multiplier for Random number generation.

-  VECT3 = Minimum coolant temperature (engine on), degrees F.

-  VECT5 = Starting coolant temperature for warm-up counter, degrees F.

8-69

FUEL STRATEGY, ADAPTIVE FUEL CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FADPT_GENERIC_COM1

BEGIN: ADAPTIVE_FUEL_CONTROL

The Adaptive Fuel Control strategy, generic version.

Execute once per background pass.

Check the integrity of the adaptive fuel area in  KAM  after  initial  system

startup,  during  execution  of  the  RAM_INIT software module.  The checksum

should also be validated when KAM corruption is detected.  This can  be  done

either during KAM qualification, or within the adaptive fuel software module.

First background pass -------------|

|OR --| Do: FADPT_SELF_TEST

KAM_ERROR = 1 ---------------------|

Update the adaptive  fuel  values  in  KAM  if  necessary,  check  for  error

conditions  which  may  require reset of the KAM values, and assign values to

the correction term for each bank.

Unconditionally -------------------------| Do: FADPT_UPDATE_STATUS

| Do: FADPT_EGO_FAILURE_RESET

| Do: FADPT_UPDATE_TABLE

| Do: FADPT_UPDATE_KAMRF

| Do: FADPT_FUEL_MONITOR

END: ADAPTIVE_FUEL_CONTROL

8-70

FUEL STRATEGY, ADAPTIVE FUEL CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: FADPT_SELF_TEST

Each time the vehicle is started, the data stored in KAM may or  may  not  be

valid.   Power interruptions, noise, etc., may have altered the KAM contents.

Alternatively, the computer  may  not  be  reading  KAM  registers  correctly

because  of a hardware fault.  The KAM qualification test judges the validity

of the KAM data, and KAM can  be  initialized  as  required.   Based  on  the

results  of  the  KAM qualification test, validate the adaptive fuel table as

follows.

CHKSUM is a KAM memory word containing  the  sum  of  the  LTMTB1  or  LTMTB2

contents.   CHKSUM  is  incremented  or  decremented  each time any LTMTB1 or

LTMTB2 cell is updated.  A one count difference between the present  sum  and

the  stored  sum is allowed to account for the case of power down after a KAM

update but prior to CHKSUM update.

This routine should be executed during RAM initialization, and  each  time  a

KAM error is detected.

[(SUM OF ALL ADAPTIVE FUEL

KAM CELLS) - CHKSUM] <= 1 ---| CHKSUM := SUM OF ADAPTIVE FUEL CELLS

| ;Assume the adaptive fuel data in KAM

| ; is valid.

|

| --- ELSE ---

|

| ;Assume the adaptive fuel data in KAM

| ; is wrong.

| ;Do a total initialization of the

| ; adaptive fuel data in KAM.

| For each cell:

|  1) Set LTMTB1[r,c] := 0.5

|     Set LTMTB2[r,c] := 0.5

|  2) Set CHKSUM := 22016

| KWUCTR := 0

END: FADPT_SELF_TEST

8-71

FUEL STRATEGY, ADAPTIVE FUEL CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: FADPT_UPDATE_TABLE

Calculate the random number to be used this background pass.

Unconditionally -------------------------| RANNUM := RANNUM*RANMUL + RANMUL/4

| rancol := rannum_hi

|  ;The high byte of the low

|  ; word of RANNUM

| ranrow := rannum_lo

|  ;The low byte of the low

|  ; word of RANNUM

| RANNUM := the low word of RANNUM

|  ;Save for next pass

Calculate the cell to be updated this background pass.

IDLE_CELL_SW = 1 ------------------|

;Idle cells enabled               |

|AND -| KAMCOL := ISFLAG

ISCFLG = 1 ------------------------|     | COLTBU := ISFLAG

;RPM control                      |     | KAMROW := 8.0

|     | ROWTBU := 8.0

HCAMFG = 0 ------------------|     |     |

|OR --|     |

HCAMSW = 0 ------------------|           |

| --- ELSE ---

|

ADAPT_SEL = 0 ---------------------------| KAMCOL := FN070D(N)

| COLTBU := KAMCOL + 0.5 + rancol

| KAMROW := FN021A(LOAD)

| ROWTBU := KAMROW + 0.5 + ranrow

|

| --- ELSE ---

|

| KAMCOL := FN070D(N)

| COLTBU := KAMCOL + 0.5 + rancol

| KAMROW := FN041(AM)

| ROWTBU := KAMROW + 0.5 + ranrow

8-72

FUEL STRATEGY, ADAPTIVE FUEL CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Service adaptive cell residency timer, which  is  used  by  the  OBD-II  Fuel

System Monitor.

Unconditionally -------------------------| KAMCOL_CUR := int(KAMCOL)

| KAMROW_CUR := int(KAMROW)

KAMCOL_CUR <> KAMCOL_OLD ----------|

|OR --| KAM_MON_TMR := 0

KAMROW_CUR <> KAMROW_OLD ----------|     | ;Reset the adaptive residency

| ; timer to prevent similar

| ; conditions reset

Unconditionally -------------------------| KAMCOL_OLD := KAMCOL_CUR

| KAMROW_OLD := KAMROW_CUR

Determine if adaptive is enabled/disabled or if adaptive area has changed.

FADPT_ENABLE = 0 ------------------|

|

abs(KAMROW - LSTROW) > DELROW -----|OR --| LPCT1R := 0

|     | LPCT1L := 0

abs(KAMCOL - LSTCOL) > DELCOL -----|     | LPCT2R := 0

| LPCT2L := 0

| EGOCT11 := 0

| EGOCT21 := 0

| LSTCOL := KAMCOL

| LSTROW := KAMROW

|

| Do: FADPT_NEW_UPRATE_CALC

| Exit: FADPT_UPDATE_TABLE

8-73

FUEL STRATEGY, ADAPTIVE FUEL CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calculate the LAMBSE windows, LAMWIN1 and LAMWIN2.  LAMBSE values must be

outside this

window to initiate adaptive fuel table updates.

Unconditionally -------------------------| LAMWIN1 := DELAMB + (PTPAMP1 *

|             AMPMUL)

| LAMWIN2 := DELAMB + (PTPAMP2 *

|             AMPMUL)

Calculate the EGO based learning rate

This logic gives the system the capability  to  learn  faster  during  "green

engine" conditions.

WARM_UP = 0 -----------------------|

|

TCSTRT < VECT5 --------------------|

|AND -| WARM_UP := 1

ECT >= VECT3 ----------------------|     | KWUCTR := KWUCTR + 1

|     | KWUCTR := MIN(KWUCTR, 255)

UNDSP = 0 -------------------------|

KWUCTR < KWUCNT -------------------------| egolearn_rat := FAEGCT

;First few warm-up cycles               |  ;Use fast learning rate

|

| --- ELSE ---

|

| egolearn_rat := ADEGCT

|  ;Use normal learning rate

Update the EGO switch counters used by the adaptive fuel logic.

SWTFL11 = 1 -----------------------------| EGOCT11 := EGOCT11 + 1

SWTFL21 = 1 -----------------------|

|AND -| EGOCT21 := EGOCT21 + 1

NUMEGO = 2 ------------------------|

8-74

FUEL STRATEGY, ADAPTIVE FUEL CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calculate Bank 1 loop counters, LPCT1L and LPCT1R.

EGOCT11 >= egolearn_rat -----------|

|

EGOFL11 = 1 -----------------------|

;HEGO11 is rich                   |AND -| LPCT1R := LPCT1R + 1

|     | LPCT1R := MIN(LPCT1R, 255)

LAMBSE1 >= (1 + BIAS_G1 + LAMWIN1)-|     | ;Increment rich counter, bank 1

|     |

LIMIT_PURGE = 1 -------|           |     |

|AND -|     |     |

PG_DC = 0 -------------|     |OR --|     |

|           |

LIMIT_PURGE = 0 -------------|           |

|

| --- ELSE ---

EGOFL11 = 0 -----------------------|     |

;HEGO11 is lean                   |     |

|AND -| LPCT1L := LPCT1L + 1

LAMBSE1 <= (1 + BIAS_G1 - LAMWIN1)-|     | LPCT1L := MIN(LPCT1L, 255)

|     | ;Increment lean counter, bank 1

EGOCT11 >= egolearn_rat -----------|

Calculate Bank 2 loop counters, LPCT2L and LPCT2R.

NUMEGO = 2 ------------------------|

|

EGOCT21 >= egolearn_rat -----------|

|

EGOFL21 = 1 -----------------------|AND -| LPCT2R := LPCT2R + 1

;HEGO21 is rich                   |     | LPCT2R := MIN(LPCT2R, 255)

|     | ;Increment rich counter, bank 2

LAMBSE2 >= (1 + BIAS_G2 + LAMWIN2)-|     |

|     |

LIMIT_PURGE = 1 -------|           |     |

|AND -|     |     |

PG_DC = 0 -------------|     |OR --|     |

|           |

LIMIT_PURGE = 0 -------------|           |

|

| --- ELSE ---

NUMEGO = 2 ------------------------|     |

|     |

EGOFL21 = 0 -----------------------|     |

;HEGO21 is lean                   |AND -| LPCT2L := LPCT2L + 1

|     | LPCT2L := MIN(LPCT2L, 255)

LAMBSE2 <= (1 + BIAS_G2 - LAMWIN2)-|     | ;Increment lean counter, bank 2

|

EGOCT21 >= egolearn_rat -----------|

8-75

FUEL STRATEGY, ADAPTIVE FUEL CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Determine if adaptive cells should be incremented/decremented, bank 1.

LTMTB1 cells are updated when the following conditions are  satisfied.   Note

that r = ROWTBU and c = COLTBU in the following charts.

FN1325A[r,c] >= 0 -----------------|

|

LPCT1R > 2 * UPRATE ---------------|AND -| LTMTB1[r,c] := LTMTB1[r,c]

|     |                - 0.00390625

LTMTB1[r,c] > MINADP --------------|     | CHKSUM := CHKSUM - 1

|  ;Decrement table by one bit

|  ; (unsigned bin-8) and update

|  ; checksum (unsigned word bin-0)

| LPCT1R := 0

| EGOCT11 := 0

|  ;Reset rich loop counter and

|  ; HEGO switch counter for bank 1

|

| --- ELSE ---

|

LPCT1R > 2 * UPRATE ---------------------| LPCT1R := 0

| EGOCT11 := 0

|  ;Reset rich loop counter and

|  ; HEGO switch counter for bank 1

|

| --- ELSE ---

LPCT1L > 2 * UPRATE ---------------|     |

|     |

LTMTB1[r,c] < MAXADP --------------|AND -| LTMTB1[r,c] := LTMTB1[r,c]

|     |                + 0.00390625

FN1325A[r,c] >= 0 -----------------|     | CHKSUM := CHKSUM + 1

|  ;Increment table by one bit

|  ; (unsigned bin-8) and update

|  ; checksum (unsigned word bin-0)

| LPCT1L := 0

| EGOCT11 := 0

|  ;Reset lean loop counter and

|  ; HEGO switch counter for bank 1

|

| --- ELSE ---

|

LPCT1L > 2 * UPRATE ---------------------| LPCT1L := 0

| EGOCT11 := 0

|  ;Reset lean loop counter and

|  ; HEGO switch counter for bank 1

8-76

FUEL STRATEGY, ADAPTIVE FUEL CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Determine if adaptive cells should be incremented/decremented, bank 2.

FN1325A[r,c] >= 0 -----------------|

|

LPCT2R > 2 * UPRATE ---------------|

|AND -| LTMTB2[r,c] := LTMTB2[r,c]

LTMTB2[r,c] > MINADP --------------|     |                 - 0.00390625

|     | CHKSUM := CHKSUM - 1

NUMEGO = 2 ------------------------|     |  ;Decrement table by one bit

|  ; (unsigned bin-8) and update

|  ; checksum (unsigned word bin-0)

| LPCT2R := 0

| EGOCT21 := 0

|  ;Reset rich loop counter and

|  ; HEGO switch counter for bank 2

|

| --- ELSE ---

NUMEGO = 2 ------------------------|     |

|AND -| LPCT2R := 0

LPCT2R > 2 * UPRATE ---------------|     | EGOCT21 := 0

|  ;Reset rich loop counter and

|  ; HEGO switch counter for bank 2

|

| --- ELSE ---

LPCT2L > 2 * UPRATE ---------------|     |

|     |

LTMTB2[r,c] < MAXADP --------------|     |

|AND -| LTMTB2[r,c] := LTMTB2[r,c]

FN1325A[r,c] >= 0 -----------------|     |                + 0.00390625

|     | CHKSUM := CHKSUM + 1

NUMEGO = 2 ------------------------|     |  ;Decrement table by one bit

|  ; (unsigned bin-8) and update

|  ; checksum (unsigned word bin-0)

| LPCT2L := 0

| EGOCT21 := 0

|  ;Reset lean loop counter and

|  ; HEGO switch counter for bank 2

|

| --- ELSE ---

NUMEGO = 2 ------------------------|     |

|AND -| LPCT2L := 0

LPCT2L > 2 * UPRATE ---------------|     | EGOCT21 := 0

|  ;Reset lean loop counter and

|  ; HEGO switch counter for bank 2

END: FADPT_UPDATE_TABLE

8-77

FUEL STRATEGY, ADAPTIVE FUEL CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: FADPT_EGO_FAILURE_RESET

Reset the adaptive fuel table in  KAM  due  to  EGO  failure,  as  necessary.

Execute when called from ADAPTIVE_FUEL_CONTROL.

CHKSUM is the sum of all 86 bytes in both adaptive fuel tables, regardless of

the  NUMEGO  calibration.  All table cells are treated as integer values when

computing this sum.  This feature  (table  reset  on  HEGO  failure)  can  be

disabled by setting KAM_NO_RESET.

EGO1FMFLG = 1 --------------------|

|AND -| LTMTB1[x,y] := 0.5

KAM_NO_RESET = 0 -----------------|     |  ;Reset entire table

| KWUCTR := 0

| CHKSUM := sum of all adaptive

fuel cells

NUMEGO = 2 ------------------------|

|

EGO2FMFLG = 1 ---------------------|AND -| LTMTB2[x,y] := 0.5

|     |  ;Reset entire table

KAM_NO_RESET = 0 ------------------|     | KWUCTR := 0

| CHKSUM := sum of all adaptive

fuel cells

END: FADPT_EGO_FAILURE_RESET

8-78

FUEL STRATEGY, ADAPTIVE FUEL CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: FADPT_NEW_UPRATE_CALC

This algorithm calculates the average loop counter value to  be  used  during

the  adaptive  algorithm.   The  standard four point interpolation routine is

used.  If, however, one or more  of  the  cells  in  the  four  points  is  a

reference  cell, then the average of the values in the positive cells is used

in place of the negative values.

If the system is in the special idle cells, the UPRATE value is equal to  the

value in FN1325A as specified by KAMCOL.

Note:  kamrow_max = maximum number of table rows (excluding the Special Idle

Cells row).

kamrow_spec = row number for the Special Idle Adaptive Cells.

kamcol_max = maximum number of table columns.

KAMROW = kamrow_spec ------------| Read the value in FN1325A

|   specified by KAMCOL.

| If negative, then use value from

|   referenced cell of FN1325A

| Load this value in UPRATE.

|

| Exit: FADPT_NEW_UPRATE_CALC

Separate KAMCOL and KAMROW into an integer and remainder.

Unconditionally -----------------| kamcol_rem := KAMCOL - int(KAMCOL)

| kamcol := int(KAMCOL)

| kamrow_rem := KAMROW - int(KAMROW)

| kamrow := int(KAMROW)

Check for row and column boundary conditions.

kamcol < kamcol_max -------|

|AND -| adapt1_uprat := FN1325A(kamcol,kamrow)

kamrow < kamrow_max -------|     | adapt2_uprat := FN1325A(kamcol + 1,kamrow)

| adapt3_uprat := FN1325A(kamcol,kamrow + 1)

| adapt4_uprat :=

|            FN1325A(kamcol + 1,kamrow + 1)

|

| --- ELSE ---

|

kamcol = kamcol_max -------|     |

|AND -| adapt1_uprat := FN1325A(kamcol,kamrow)

kamrow = kamrow_max -------|     | adapt2_uprat := adapt1_uprat

| adapt3_uprat := adapt1_uprat

| adapt4_uprat := adapt1_uprat

|  ;Use FN1325A 4 times so not to wrap

|  ; around table or use special idle cells

|

| --- ELSE ---

(continued on next page)

8-79

FUEL STRATEGY, ADAPTIVE FUEL CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|

kamcol = kamcol_max -------------| adapt1_uprat := FN1325A(kamcol,kamrow)

| adapt2_uprat := adapt1_uprat

| adapt3_uprat := FN1325A(kamcol,kamrow + 1)

| adapt4_uprat := adapt3_uprat

|  ;Use kamcol_max values twice so not to

|  ; wrap around table

|

| --- ELSE ---

|

kamrow = kamrow_max -------------| adapt1_uprat := FN1325A(kamcol,kamrow)

| adapt2_uprat := FN1325A(kamcol + 1,kamrow)

| adapt3_uprat := adapt1_uprat

| adapt4_uprat := adapt2_uprat

|  ;Use kamrow_max values twice so not to

|  ; wrap around table or use special

|  ; idle cells

Unconditionally -----------------| uprat_cnt := 0

Check for reference cells:

adapt1_uprat >= 0 ---------------| uprat_cnt := uprat_cnt + 1

| adapt1_uprat_ref := 0

|

| --- ELSE ---

|

| adapt1_uprat := 0

| adapt1_uprat_ref := 1

adapt2_uprat >= 0 ---------------| uprat_cnt := uprat_cnt + 1

| adapt2_uprat_ref := 0

|

| --- ELSE ---

|

| adapt2_uprat := 0

| adapt2_uprat_ref := 1

adapt3_uprat >= 0 ---------------| uprat_cnt := uprat_cnt + 1

| adapt3_uprat_ref := 0

|

| --- ELSE ---

|

| adapt3_uprat := 0

| adapt3_uprat_ref := 1

8-80

FUEL STRATEGY, ADAPTIVE FUEL CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

adapt4_uprat >= 0 ---------------| uprat_cnt := uprat_cnt + 1

| adapt4_uprat_ref := 0

|

| --- ELSE ---

|

| adapt4_uprat := 0

| adapt4_uprat_ref := 1

If all cells are referenced cells, set UPRATE to a maximum value.  Otherwise,

find UPRATE average.

uprat_cnt = 0 -------------------| UPRATE := 127 (maximum value)

|

| --- ELSE ---

|

| uprate_avg := (adapt1_uprat +

|       adapt2_uprat + adapt3_uprat +

|       adapt4_uprat) / uprat_cnt

|

| Truncate UPRATE to an integer

adapt1_uprat_ref = 1 ------------| adapt1_uprat := uprate_avg

adapt2_uprat_ref = 1 ------------| adapt2_uprat := uprate_avg

adapt3_uprat_ref = 1 ------------| adapt3_uprat := uprate_avg

adapt4_uprat_ref = 1 ------------| adapt4_uprat := uprate_avg

Unconditionally -----------------| Do four point interpolation using the real

|  numbers KAMROW and KAMCOL, and the four

|  adaptn_uprat values calculated above.

|  Store this value in UPRATE.

END: FADPT_NEW_UPRATE_CALC

8-81

FUEL STRATEGY, ADAPTIVE FUEL CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: FADPT_UPDATE_KAMRF

The adaptive fuel tables stored in KAM are used as a reference for both  open

and closed loop fuel control.  The method by which the adaptive fuel table is

referenced as shown below.

When four-point interpolation is used  to  retrieve  the  KAMRF1  and  KAMRF2

values,  the row and column numbers must be clipped to 7 and 9, respectively.

If the interpolation is to be based on a cell of row 7, the values  from  the

two  cells  of  row  7 should be used again in place of (non-existent) row 8.

The same is true of column 9, where the two cell values of column 9 should be

used  in  place  of column 10 in the interpolation equation.  Finally, if the

base cell for interpolation is LTMTB1[7,9], no interpolation is necessary.

For purposes of interpolation, the LTMTBn 80  to  LTMTBn  85  cells  are  not

included.  These cells should correspond to the special idle cells.

Execute this routine when called from ADAPTIVE_FUEL_CONTROL.

FADPT_SW = 0 ----------------|

|

CRKFLG = 1 ------------------|

;Crank mode                 |

|OR --| KAMRF1 := 1.0

LEGOFG11 = 1 ----|           |     | KAMRF2 := 1.0

|OR --|     |     |  ;Use no interpolation

FFG_CSD11 = 1 ---|     |     |     |

|AND -|     |

LEGOFG21 = 1 ----|     |           |

|OR --|           |

FFG_CSD21 = 1 ---|                 |

| --- ELSE ---

IDLE_CELL_SW = 1 ------------|     |

;Idle cells enabled         |     |

|     |

ISCFLG = 1 ------------------|AND -| KAMRF1 := 0.5 + LTMTB1[8,ISFLAG]

|     | KAMRF2 := 0.5 + LTMTB2[8,ISFLAG]

HCAMFG = 0 ------------|     |     |  ;Use no interpolation

;No RPM adder         |OR --|     |

|           |

HCAMSW = 0 ------------|           |

;Ignore HCAMFG                    |

| --- ELSE ---

|

| r := KAMROW;  c := KAMCOL

| KAMRF1 := 0.5 + LTMTB1[r,c]

| KAMRF2 := 0.5 + LTMTB2[r,c]

| ;Use 4 point interpolation, and use

| ; reference cells when FN1325A is

| ; negative

8-82

FUEL STRATEGY, ADAPTIVE FUEL CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Lack of HEGO switching affect on KAMRF1/2.  If there is only one  HEGO  bank,

KAMRF2  will  follow KAMRF1.  If either HEGO stops switching, the KAMRF value

from the other bank will be used.

LEGOFG21 = 1 ----------------|

|

FFG_CSD21 = 1 ---------------|OR --| KAMRF2 := KAMRF1

|     |

NUMEGO = 1 ------------------|     |

| --- ELSE ---

|

LEGOFG11 = 1 ----------------|     |

|OR --| KAMRF1 := KAMRF2

FFG_CSD11 = 1 ---------------|

END: FADPT_UPDATE_KAMRF

BEGIN: FADPT_PID_DEFINITIONS

Update the LONG TRIM 1 and LONG TRIM 2 diagnostic parameters.

Unconditionally ------------------------| long_trim_1 := 128 * KAMRF1

| long_trim_2 := 128 * KAMRF2

pid_def(j1979_01_07, long_trim_1 )

pid_def(j1979_01_09, long_trim_2 )

END: FADPT_PID_DEFINITIONS

8-83

FUEL STRATEGY, ADAPTIVE FUEL ENABLE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.12.2  ADAPTIVE FUEL ENABLE LOGIC (CDAN0)

OVERVIEW

The Adaptive Fuel feature is intended to compensate for variability  in  fuel

system  components.   This  is  achieved by allowing the system to learn fuel

system errors, and then to  apply  this  information  directly  to  the  fuel

pulsewidth calculated from the desired fuel/air ratio.  This learned error is

retained in keep-alive memory,  and  is  stored  in  the  form  of  either  a

speed-load  table,  or  single  air mass column.  There is one table for each

engine bank (if NUMEGO = 2).

This module controls the FADPT_ENABLE flag.   When  set,  the  adaptive  fuel

control  system  will  be allowed to learn fuel system errors.  When cleared,

the learning mechanism is disabled.  In either case, the compensation  factor

retrieved from the table will be used in the fuel pulsewidth calculation.

A secondary flag, FADPT_READY, is used for handshaking with the purge system.

This  flag  indicates  that all enable conditions have been met, and adaptive

learning will proceed as soon as PCOMP is fully disabled.

CALIBRATION NOTES

To completely disable the  Adaptive  Fuel  Control  and  OBD-II  Fuel  System

Monitor  features, set the calibration switch FADPT_SW to 0.  This will force

KAMRF1 and KAMRF2 to 1.0, disable all table updates,  and  disable  the  Fuel

System Monitor.

To disable only the table update portion of the adaptive  fuel  feature,  use

values  of  AFECT1  and  AFECT2  such  that  AFECT1 > AFECT2.  This will also

require a non-zero value of ADAPTM.  In  this  case,  existing  data  in  the

adaptive  fuel  tables will be used to calculate KAMRF1 and KAMRF2, but table

updates and the Fuel System Monitor will be disabled.

8-84

FUEL STRATEGY, ADAPTIVE FUEL ENABLE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  ACT = Air charge temperature, degrees F.

-  ADPTMR = Adaptive learning enable timer.

-  ECT = Engine coolant temperature, degrees F.

-  EFTR = Equilibrium fuel transfer rate.

-  LOAD = Universal normalized load parameter.

-  TP_REL = Relative throttle position, counts.

-  PG_DC = Purge valve duty cycle, % on.

Bit Flags:

-  ADPTMR_FLG = Adaptive Fuel Time Flag.

-  AFMFLG = ACT Failure Mode (FMEM) flag.

-  CFMFLG = ECT FMEM flag.

-  FADPT_ENABLE = Flag to indicate enable state of Adaptive Fuel Strategy.

-  FADPT_READY = Flag indicating that  the  conditions  to  enable  adaptive

learning  have  been  satisfied.  Adaptive fuel will be enabled as soon a

PCOMP is disabled.

-  LAM_MOD_FLG = Flag used to initiate high frequency  fuel  modulation  for

the upstream EGO monitor.

-  OLFLG = Open loop fuel flag; 1-> open loop fuel, 0-> closed loop fuel.

-  MAF_INTP_FLG = Flag indicating that the MAF sensor signal is  unreliable;

0 -> MAF signal OK, 1 -> MAF signal unreliable.

-  MFMFLG = MAP/MAF FMEM flag.

-  PCOMP_ENA = PCOMP strategy enabled flag; 1 -> PCOMP is enabled  (adaptive

fuel disabled).

-  TFMFLG = TP FMEM flag.

-  UNDSP = Underspeed flag; 1 -> Underspeed mode or Crank.

8-85

FUEL STRATEGY, ADAPTIVE FUEL ENABLE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  ADAPTM = Adaptive learning enable time delay, seconds.

-  ADEFTR = Transient fuel threshold to update adaptive fuel.

-  ADPLOD = Minimum allowable load to enable adaptive learning.

-  ADPTP = Maximum allowable TP_REL to enable adaptive fuel.

-  AFACT1 = Minimum ACT to Update Adaptive Fuel Table degrees F.

-  AFACT2 = Maximum ACT to update Adaptive Fuel Table, degrees F.

-  AFECT1 = Minimum ECT for starting the Adaptive Fuel timer, degrees F.

-  AFECT2 = Over-temperature ECT to zero the adaptive fuel timer.

-  FADPT_SW = Switch allowing the complete disablement of adaptive fuel  and

the  fuel  system  monitor.   FADPT_SW  =  0 -> disable all adaptive fuel

activity.

-  PCOMP_SW = PCOMP strategy select switch; 1 -> execute PCOMP strategy.

8-86

FUEL STRATEGY, ADAPTIVE FUEL ENABLE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FADPT_UPDATE_STATUS_COM1

BEGIN: FADPT_UPDATE_STATUS

Module to control the state of the Adaptive Fuel Control status flags.

Execute once per background pass.

The ADPTMR timer allows Adaptive Fuel to be enabled if the engine is  in  run

mode and the inferred engine temperature is within a certain band.  Since the

engine temperature is inferred  here  from  the  Engine  Coolant  Temperature

sensor, an ECT sensor failure will reset this timer.  This way, if the ADPTMR

feature is disabled in the calibration (i.e., ADAPTM  =  0),  an  ECT  sensor

failure will not inhibit adaptive learning.

ADPTMR is an actuated timer, which counts up whenever ADPTMR_FLG is set.

UNDSP = 0 ------------------------|

;In run mode                     |

|

AFECT1 <= ECT <= AFECT2 ----------|AND -| COUNT UP ADPTMR

;Reasonable engine temperature   |     | ADPTMR_FLG := 1

|     |

CFMFLG = 0 -----------------------|     |

;Failed ECT sensor                     |

|

| --- ELSE ---

|

| ADPTMR := 0

| ADPTMR_FLG := 0

8-87

FUEL STRATEGY, ADAPTIVE FUEL ENABLE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Check  Adaptive  Fuel  Control  enable  conditions.   The  FADPT_READY   flag

indicates  that  learning  can  continue  as soon as PCOMP is fully disabled.

This flag is used for handshaking with the Purge Compensation feature.

FADPT_SW = 0 ---------------------|

;Adaptive Fuel feature disabled  |

|

LAM_MOD_FLG = 1 ------------------|

;HEGO monitor in progress        |

|

OLFLG = 1 ------------------------|OR --| FADPT_READY := 0

;Open loop fuel control          |     |  ;Adaptive system not ready to

|     |  ; learn

|EFTR| > ADEFTR ------------------|     |

;High transient fuel value       |     |

|     |

ACT <= AFACT1 --------------------|     |

|     |

ACT >= AFACT2 --------------------|     |

;Extreme ambient temperature     |     |

|     |

AFMFLG = 1 -----------------------|     |

;Failed ACT sensor               |     |

|     |

ADPTMR < ADAPTM ------------------|     |

;Stable engine temperature       |     |

|     |

TP_REL > ADPTP -------------------|     |

;High TP value                   |     |

|     |

TFMFLG = 1 -----------------------|     |

;Failed throttle position sensor |     |

|     |

LOAD < ADPLOD --------------------|     |

;Low LOAD value                  |     |

|     |

MFMFLG = 1 -----------------------|     |

;MAF sensor failed               |     |

|     |

MAF_INTP_FLG = 1 -----------------|     |

;MAF sensor unreliable                 | --- ELSE ---

|

| FADPT_READY := 1

|  ;Adaptive learning conditions

|  ; are satisfied

8-88

FUEL STRATEGY, ADAPTIVE FUEL ENABLE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Enable Adaptive Fuel Control learning, provided all enable conditions are are

met, and the Purge Compensation feature is fully disabled.

FADPT_READY = 1 ------------------|

|

PCOMP_ENA = 0 --------|           |AND -| FADPT_ENABLE := 1

|AND -|     |     |

PG_DC = 0 ------------|     |OR --|     |

|           |

PCOMP_SW = 0 ---------------|           |

| --- ELSE ---

|

| FADPT_ENABLE := 0

END: FADPT_UPDATE_STATUS

8-89

FUEL STRATEGY, FUEL SYSTEM MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.12.3  FUEL SYSTEM MONITOR (CDAM0)

OVERVIEW

As fuel control system components parameters change from  nominal  with  time

the  adaptive fuel table contents will reflect the change.  As the parameters

continue to change the adaptive table will reach the  adaptive  tables  clip.

This  means  that  the  adaptive  table  cannot compensate for any additional

changes in fuel control component parameters.  Further  change  in  the  fuel

control  system components parameters will cause LAMBSEn switch point to move

toward its clips.  As this trend continues the ability of  the  fuel  control

system to control fuel will cease.

The adaptive fuel monitor test examines KAM_BARn and LAM_BARn (i.e., filtered

values  of  LAMBSEn  and KAMRFn, respectively).  When KAM_BARn indicates that

KAMRFn is near its limit and LAM_BARn indicates that LAMBSEn is about to lose

its  ability  to  control  the  fuel,  the  test will indicate a fuel control

failure.  The criteria for failure detection is shown below.

XXXXXXXXXXXXXXXXXXXXXX           XXXXXXXXXXXXXXXXXXXXXX

XXXXXXXXXXXXXXXXXXXXXX           XXXX   KAM_BARn   XXXX

XXXX   LAM_BARn   XXXX           XXXXXXXXXXXXXXXXXXXXXX

- -XXXXXXXXXXXXXXXXXXXXXX - - - - - - - - - - - - - - - - - - V_KAMBAR_MAX

XXXXXXXXXXXXXXXXXXXXXX

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - V_LAMBAR_MAX

------------------------------------------------------------- 1.0

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - V_LAMBAR_MIN

XXXXXXXXXXXXXXXXXXXXXX

- - - - - - - - - - - - - - - - - - XXXXXXXXXXXXXXXXXXXXXX- - V_KAMBAR_MIN

XXXXXXXXXXXXXXXXXXXXXX           XXXX   LAM_BARn   XXXX

XXXX   KAM_BARn   XXXX           XXXXXXXXXXXXXXXXXXXXXX

XXXXXXXXXXXXXXXXXXXXXX           XXXXXXXXXXXXXXXXXXXXXX

RICH ENGINE FAILURE              LEAN ENGINE FAILURE

This module will also control the two adaptive fuel failure mode  flags,  one

for  each adaptive table in a two HEGO control system (ADT1FMFLG, ADT2FMFLG).

These indicate a general fuel system failure on the corresponding HEGO bank.

FMEM flags indicating failed LAMBSE1 or LAMBSE2 are also maintained  by  this

module.

8-90

FUEL STRATEGY, FUEL SYSTEM MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  AM = Mass air flow bin 10.

-  FADPT_LEAN11 = Flag indicating a lean fuel system error on bank 1.

-  FADPT_LEAN21 = Flag indicating a lean fuel system error on bank 2.

-  FADPT_RICH11 = Flag indicating a rich fuel system error on bank 1.

-  FADPT_RICH21 = Flag indicating a rich fuel system error on bank 2.

-  KAM_MON_TMR = Adaptive cell residency timer (required for monitoring).

-  KAMRF1 = Total learned fuel system correction (from HEGO11).

-  KAMRF2 = Total learned fuel system correction (from HEGO21).

-  KAM_BAR1 = Average KAMRF1, used both to filter  codes  and  to  determine

exit conditions for adaptive fuel (see PCOMP Enable logic).

-  KAM_BAR2 = Average KAMRF2, used both to filter  codes  and  to  determine

exit conditions for adaptive fuel (see PCOMP Enable logic).

-  LAM_BAR1 = Average LAMBSE1, used both to filter codes  and  to  determine

exit conditions for adaptive fuel (see PCOMP Enable logic).

-  LAM_BAR2 = Average LAMBSE2, used both to filter codes  and  to  determine

exit conditions for adaptive fuel (see PCOMP Enable logic).

-  LAMBSE1 = Lambda equivalence ratio (EGO-1).

-  LAMBSE2 = Lambda equivalence ratio (EGO-2).

-  LOAD = Universal normalized load parameter.

-  N = Engine speed, RPM.

Bit Flags:

-  ADT1FMFLG = Adaptive table 1 failure mode.

-  ADT2FMFLG = Adaptive table 2 failure mode.

-  FADPT_ENABLE = Flag indicating enable state of adaptive fuel strategy.

-  FFG_LAMBSE1 = OBD-II system FMEM flag for LAMBSE1; 1  =  LAMBSE1  is  not

currently reliable.

-  FFG_LAMBSE2 = OBD-II system FMEM flag for LAMBSE2; 1  =  LAMBSE2  is  not

currently reliable.

-  FUEL_ENA_OLD = previous background value of FUEL_TST_ENA.

-  FUEL_MON = Completion flag for fuel monitor; 1 -> all  fuel  faults  have

been monitored at least once since power-up.

8-91

FUEL STRATEGY, FUEL SYSTEM MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FUEL_TST_ENA = Fuel monitor enable flag; 1 -> testing enabled.

-  FUEL_TST_RDY = Fuel monitor ready flag; 1 -> all  entry  conditions  have

been met; ready to run fuel monitor.

-  KAM_ERROR = Flag indicating KAM failure or uninitialized  KAM.   Set  and

cleared by the KAM Qualification strategy module.

-  OBDII_RESET = Flag which represents the receipt of an  OBD-II  scan  tool

reset message.

-  PxxxMALF = Malfunction flag for fault XXX; 1 -> malfunction.

-  SWTFL11 = Bank 1 upstream HEGO switch flag.

-  SWTFL21 = Bank 2 upstream HEGO switch flag.

Calibration Constants:

-  ADAPT_SEL = 0 -> use load, 1 -> use AM.

-  CADP_RES_TM = Adaptive cell residency timer threshold.

-  KAM_BAR_FK = Filter constant for calculating filtered values KAM_BAR1 and

KAM_BAR2.

-  LAM_BAR_FK = Filter constant for LAM_BAR1 and LAM_BAR2.

-  NUMEGO = Number of HEGO sensors.

-  V_ADPAM_MAX = Maximum AM to perform adaptive fuel monitor test.

-  V_ADPAM_MIN = Minimum AM to perform adaptive fuel monitor test.

-  V_ADPLD_MAX = Maximum LOAD to perform adaptive fuel monitor test.

-  V_ADPLD_MIN = Minimum LOAD to perform adaptive fuel monitor test.

-  V_ADPN_MAX = Maximum N to perform adaptive fuel monitor test.

-  V_ADPN_MIN = Minimum N to perform adaptive fuel monitor test.

-  V_KAMBAR_MIN = Lower limit of filtered KAMRF, below  which  a  rich  fuel

system fault will be indicated.

-  V_KAMBAR_MAX = Upper limit of filtered KAMRF, above  which  a  lean  fuel

system fault will be indicated.

-  V_LAMBAR_MIN = Lower limit of filtered LAMBSE, below which  a  lean  fuel

system fault will be indicated.

-  V_LAMBAR_MAX = Upper limit of filtered LAMBSE, above which  a  rich  fuel

system fault will be indicated.

8-92

FUEL STRATEGY, FUEL SYSTEM MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OTHER

-  fuel_codes = SET OF {P0171,P0172,P0174,P0175}.  The set of  OBD-II  fault

codes that relate to the fuel monitor.

-  malfunction(fuel,PXXX) = Public procedure updating the MIL control status

provided by the MIL control module.

-  P0171 = lean clipped value from HEGO11 adaptive fuel table was used.

-  P0172 = rich clipped value from HEGO11 adaptive fuel table was used.

-  P0174 = lean clipped value from HEGO21 adaptive fuel table was used.

-  P0175 = rich clipped value from HEGO21 adaptive fuel table was used.

8-93

FUEL STRATEGY, FUEL SYSTEM MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FADPT_FUEL_MONITOR_COM1

BEGIN: FADPT_FUEL_MONITOR

Reset KAM memory locations and all filters used in fault determination,  This

must occur upon detection of a KAM error or during an OBD-II system reset (as

requested by the OBD-II Executive).

It is not necessary to  clear  malfunctions  at  this  time,  as  the  OBD-II

Executive will clear all codes in this case.

KAM_ERROR = 1 --------------|

|OR --| KAM_BAR1 := 1.0

OBDII_RESET = 1 ------------|     | KAM_BAR2 := 1.0

| LAM_BAR1 := 1.0

| LAM_BAR2 := 1.0

| Do: FADPT_FM_FAULT_CLEAR

If the Fuel System  Monitor  entry  conditions  are  satisfied,  perform  the

necessary filtering and check for a fuel system failure.

Unconditionally ------------------| Do: FADPT_FM_READY_CHK

| Do: FADPT_FM_FAULT_CHK

| Do: FADPT_FMEM_CHECK

END: FADPT_FUEL_MONITOR

8-94

FUEL STRATEGY, FUEL SYSTEM MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: FADPT_FM_READY_CHK

Information for control of fuel ready flag based on transition of the  OBD-II

executive  Fuel  System  Monitor enable flag.  This logic is used to hold the

FUEL_TST_RDY flag at zero for the first background pass after the monitor  is

disabled.   After  one  background pass with FUEL_TST_RDY held low, this flag

will again reflect the state of  the  Fuel  Monitor  entry  conditions.   The

FUEL_TST_RDY flag is used in the handshake with the OBD-II Executive.

FUEL_TST_ENA = 0 -----------------|

;Current ena state               |AND -| ena_transition_to_off := 1

|     |  ;Enable transition to off

FUEL_ENA_OLD = 1 -----------------|     |  ;has occurred

;Previous ena state                    | FUEL_ENA_OLD := FUEL_TST_ENA

|

| --- ELSE ---

|

| ena_transition_to_off := 0

|  ;no on to transition off

|  ;transition this time

| FUEL_ENA_OLD := FUEL_TST_ENA

ADAPT_SEL = 0 --------|

;Table adaptive      |

|

LOAD >= V_ADPLD_MIN --|AND -|

;Min load to test    |     |

|     |

LOAD <= V_ADPLD_MAX --|     |

;Max load to test          |OR --|

|     |

ADAPT_SEL = 1 --------|     |     |

;Column adaptive     |     |     |

|     |     |

AM >= V_ADPAM_MIN ----|AND -|     |

;Min AM to test      |           |

|           |

AM <= V_ADPAM_MAX ----|           |

;Max AM to test                  |AND -| FUEL_TST_RDY := 1

|     |

N <= V_ADPN_MAX ------------------|     |

;Max RPM to test                 |     |

|     |

N >= V_ADPN_MIN ------------------|     |

;Min RPM to test                 |     |

|     |

ena_transition_to_off = 0 --------|     |

;No ena trans to off             |     |

|     |

FADPT_ENABLE = 1 -----------------|     |

;Adaptive enabled                      |

| --- ELSE ---

|

| FUEL_TST_RDY := 0

| KAM_MON_TMR := 0

END: FADPT_FM_READY_CHK

8-95

FUEL STRATEGY, FUEL SYSTEM MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: FADPT_FM_FAULT_CHK

Calculate of stable average LAMBSE and KAMRF values for banks 1 and 2.  These

are used in the Fuel System Monitor as a failure criteria.

FADPT_ENABLE = 1 --------|

|AND -| LAM_BAR1 := ROLAV_FK(LAMBSE1,LAM_BAR_FK)

SWTFL11 = 1 -------------|     |  ;Filter constant rolling average of

|  ; LAMBSE1 at the time of the EGO11 switch

| KAM_BAR1 := ROLAV_FK(KAMRF1,KAM_BAR_FK)

|  ;Filter constant rolling average of KAMRF1

NUMEGO = 2 --------------|

|

FADPT_ENABLE = 1 --------|AND -| LAM_BAR2 := ROLAV_FK(LAMBSE2,LAM_BAR_FK)

|     |  ;Filter constant rolling average of

SWTFL21 = 1 -------------|     |  ; LAMBSE2 at the time of the EGO21 switch

| KAM_BAR2 := ROLAV_FK(KAMRF2,KAM_BAR_FK)

|  ;Filter constant rolling average of KAMRF2

If the Fuel Monitor is enabled by the OBD-II  Executive  (FUEL_TST_ENA),  and

the entry conditions are satisfied (FUEL_TST_RDY), then check for fuel system

failure on each applicable HEGO bank.  Clear all  faults  when  the  test  is

disabled  by the OBD-II Executive.  The flag FUEL_MON indicates to the OBD-II

Executive that the fuel monitor has executed.

FUEL_TST_ENA = 1 -----------------|

;OBD-II enable flag which        |

; enables this test              |AND -| Do: FADPT_FM_TEST_EGO11_KAM

|     | Do: FADPT_FM_TEST_EGO21_KAM

FUEL_TST_RDY = 1 -----------------|     | FUEL_MON := 1

;Entry conditions satisfied            |

| --- ELSE ---

|

FUEL_TST_ENA = 0 -----------------------| Do: FADPT_FM_FAULT_CLEAR

;Executive is disabling this monitor

END: FADPT_FM_FAULT_CHK

8-96

FUEL STRATEGY, FUEL SYSTEM MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: FADPT_FM_TEST_EGO11_KAM

Check for a rich or lean fault on HEGO bank 1.  Hysteresis is provided by the

KAM_MON_TMR  timer,  which  is  controled  in  the base Adaptive Fuel Control

strategy module.

KAM_BAR1 <= V_KAMBAR_MIN ---------|

|AND -| FADPT_RICH11 := 1

LAM_BAR1 >= V_LAMBAR_MAX ---------|     | FADPT_LEAN11 := 0

| Do: malfunction(fuel,P0172)

|  ;Rich failure detected

|

| --- ELSE ---

KAM_BAR1 >= V_KAMBAR_MAX ---------|     |

|AND -| FADPT_LEAN11 := 1

LAM_BAR1 <= V_LAMBAR_MIN ---------|     | FADPT_RICH11 := 0

| Do: malfunction(fuel,P0171)

|  ;Lean failure detected

|

| --- ELSE ---

|

KAM_MON_TMR > CADP_RES_TM --------------| FADPT_RICH11 := 0

| FADPT_LEAN11 := 0

|  ;No HEGO11 adaptive table

|  ;failures this pass

| Do: check_fuel_conds(P0171)

| Do: clear_malf(P0171)

| Do: check_fuel_conds(P0172)

| Do: clear_malf(P0172)

END: FADPT_FM_TEST_EGO11_KAM

8-97

FUEL STRATEGY, FUEL SYSTEM MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: FADPT_FM_TEST_EGO21_KAM

Check for a rich or lean fault on HEGO bank 2, if necessary.   Hysteresis  is

provided  by  the  KAM_MON_TMR timer, which is controled in the base Adaptive

Fuel Control strategy module.

NUMEGO = 1 -----------------------------| FADPT_RICH21 := 0

| FADPT_LEAN21 := 0

|

| --- ELSE ---

KAM_BAR2 <= V_KAMBAR_MIN ---------|     |

|AND -| FADPT_RICH21 := 1

LAM_BAR2 >= V_LAMBAR_MAX ---------|     | FADPT_LEAN21 := 0

| Do: malfunction(fuel,P0175)

|  ;Rich failure detected

|

| --- ELSE ---

KAM_BAR2 >= V_KAMBAR_MAX ---------|     |

|AND -| FADPT_LEAN21 := 1

LAM_BAR2 <= V_LAMBAR_MIN ---------|     | FADPT_RICH21 := 0

| Do: malfunction(fuel,P0174)

|  ;Lean failure detected

|

| --- ELSE ---

|

KAM_MON_TMR > CADP_RES_TM --------------| FADPT_RICH21 := 0

| FADPT_LEAN21 := 0

|  ;No HEGO11 adaptive table

|  ;failures this pass

| Do: check_fuel_conds(P0174)

| Do: clear_malf(P0174)

| Do: check_fuel_conds(P0175)

| Do: clear_malf(P0175)

END: FADPT_FM_TEST_EGO21_KAM

8-98

FUEL STRATEGY, FUEL SYSTEM MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: FADPT_FMEM_CHECK

This procedure sets and clears two sets of adaptive fuel failure mode  flags,

one  set  for each HEGO bank.  The FFG_LAMBSEn flags indicate that the LAMBSE

parameter for that bank is not in control.  The ADTnFMFLG indicate a  general

fuel system error on the bank.

FADPT_LEAN11 = 1 -----------------|

|OR --| ADT1FMFLG := 1

FADPT_RICH11 = 1 -----------------|     | FFG_LAMBSE1 := 1

|

| --- ELSE ---

|

| ADT1FMFLG := 0

| FFG_LAMBSE1 := 0

FADPT_LEAN21 = 1 -----------------|

|OR --| ADT2FMFLG := 1

FADPT_RICH21 = 1 -----------------|     | FFG_LAMBSE2 := 1

|

| --- ELSE ---

|

| ADT2FMFLG := 0

| FFG_LAMBSE2 := 0

END: FADPT_FMEM_CHECK

BEGIN: FADPT_FM_FAULT_CLEAR

Unconditionally ------------------------| FADPT_LEAN11 := 0

| FADPT_RICH11 := 0

| FADPT_LEAN21 := 0

| FADPT_RICH21 := 0

END: FADPT_FM_FAULT_CLEAR

8-99

FUEL STRATEGY, FUEL SYSTEM MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.13  FEATURE:  TRANSIENT FUEL - V1.1_FTRAN_TRANSIENT_FUEL (CDAM0)

8.13.1  BACKGROUND TRANSIENT FUEL LOGIC (CDAM0)

OVERVIEW

Background:

A liquid fuel film resides on the walls of the  intake  manifold.   The  film

mass  varies  primarily  with  manifold  absolute  pressure and manifold wall

temperature.  During steady state conditions, the film mass is constant.  The

rates  of  condensation  and  evaporation  on  the  manifold walls are equal.

During transients, the film mass changes creating air/fuel ratio errors.

.  During accelerations, the film mass increases.  Fuel will condense faster

on  the manifold walls until equilibrium is reached.  In an uncompensated

system at stoichiometry, fuel is diverted from the  cylinders,  resulting

in a momentary lean condition.

.  During decelerations, the  film  mass  decreases.   Fuel  will  evaporate

faster  from  the  manifold  walls  until  equilibrium is reached.  In an

uncompensated system at stoichiometry, fuel is added  to  the  cylinders,

resulting in a momentary rich condition.

The problem is magnified in closed loop fuel systems because the fuel control

will incorrectly chase the transient air/fuel excursions.

Intent:

The Transient Fuel Compensation Strategy (TFC) augments the closed/open  loop

fuel control to keep cylinder events at the desired air/fuel ratio during all

engine transients.  The goals are:

.  To eliminate lean air/fuel excursions during accelerations.

.  To eliminate rich air/fuel excursions during decelerations.

8-100

FUEL STRATEGY, BACKGROUND TRANSIENT FUEL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Approach:

.  The film mass rate of change is proportional to the amount of  fuel  that

must be added to or subtracted from the manifold film.

/                     \

FILM MASS           1            |  STEADY       ACTUAL  |

RATE OF      = --------------- * |  STATE    -           |

CHANGE          TIME CONSTANT    | FILM MASS   FILM MASS |

\                     /

EFTR = (EISF_FG - AISF)/EFTC

WHERE:

EISF_FG = FN1321(ECT,LOAD) * FN313(N) * MTEISF + (foreground correction)

EFTC = FN1322DL(ECT,LOAD) * MTEFTC  (during decels)

-- or --

= FN1322AL(ECT,LOAD) * MTEFTC  (during accels)

.  Steady state film mass is calculated from LOAD (+  forground  correction)

and ECT and must be calibrated for different applications.

.  The actual film mass is a time integration  of  the  film  mass  rate  of

change.

/                           \

ACTUAL      ACTUAL       | FILM MASS        TIME SINCE |

FILM     =  FILM    +    | RATE OF     *      LAST     |

MASS        MASS         | CHANGE            UPDATE    |

\                           /

AISF = AISF + (EFTR * DT12S * T_TO_S)

.  The computer adjusts fuel flow to match the transient  fuel  flow  to  or

from the manifold fuel film.

COMPENSATED   CLOSED/OPEN LOOP   FILM MASS RATE OF CHANGE

FUEL FLOW  =    FUEL FLOW     +   OR FUEL FLOW

(ACTUAL)      (BASE STRATEGY)    (TFC STRATEGY)

Please  Note:   To  deliver  increased  transient  fuel  resolution  this

strategy  now  uses  a  double word specification on AISF, EISF, EFTR and

TFC_HR.  This is contained within two parameters  identifying  the  upper

and lower words, ie (EISFU, EISFLO).  Unique parameter names are required

for each word to enable monitoring via the RCON.

The RCON  can  only  display  16  bit  (word)  length  registers,  it  is

considered necessary, for transient fuel control, that both the upper and

lower words are availible for in-vehicle development.

8-101

FUEL STRATEGY, BACKGROUND TRANSIENT FUEL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  AISFLO = Actual Intake Surface Fuel, (Lower word).

-  AISFU = Actual Intake Surface Fuel, (Upper word).

-  ATMR1 = Time since exiting Crank Mode, secs.

-  CYL_AIR_CHG  =  Current  cylinder  aircharge  to  be  used  in  the  fuel

calculation, lbm.

-  DSDRPM = Idle speed control desired RPM.

-  DT12S = The period of time between two  adjacent  rising  edges  of  PIP,

ticks/pip.

-  EFTCA = Time constant for transient fuel - ACCEL., sec.

-  EFTCD = Time constant for transient fuel - DECEL., sec.

-  EFTR = Equilibrium Fuel Transfer Rate during the previous  program  pass.

lbm/sec.

-  EISFU = Background based steady state fuel film mass,(Upper word) - Lb.

-  EISFLO = Background based steady state fuel film mass,(Lower word) - Lb.

-  EISF_FGU = Foreground based steady state fuel film mass, (Upper  word)  -

Lb.

-  EISF_FGL = Foreground based steady state fuel film mass, (Lower  word)  -

Lb.

-  EFTC = Time Constant for transient fuel, sec.

-  ISCFLG = ISC Mode Flag; 1 -> rpm control mode.

-  LOAD = Universal load as ratio of aircharge over standard.

-  N = RPM.

-  TFC_HR = Fuel  mass  per  injection  from  transient  fuel  compensation,

lbs/cylinder.

-  TFC_MULTA = Transient fuel multiplier - ACCEL.

-  TFC_MULTD = Transient fuel multiplier - DECEL.

-  TFC_CHG = Aircharge that transient fuel (background)  was  evaluated  at,

Lb.

-  TFC_SLOPE = Change in the  transient  fuel  puddle  mass  per  change  in

aircharge.

-  TFC_TMR = Transient fuel timer, secs.

8-102

FUEL STRATEGY, BACKGROUND TRANSIENT FUEL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Bit Flags:

-  CRKFLG = Flag indicating engine mode; 1 -> engine Cranking, 0  ->  engine

not Cranking.

-  DFSFLG = Indicates Decel Fuel Shutoff.

-  EFFLG1 = Equilibrium Fuel Flag.  This flag controls the initialization of

AISF.

-  TFC_FG_EN = Foreground TFC_HR calculation enable  flag;  1  ->  calculate

TFC_HR, 0 -> don't calculate TFC_HR.

-  UNDSP = 1 -> engine in UNDERSPEED or CRANK, 0 -> engine not in UNDERSPEED

or CRANK.

Calibration Constants:

-  AISFM = Multiplier on AISF when in DFSO.   Determines  Fuel  Puddle  size

upon re-entering normal fuel.

-  FN018 = ATMR1 normaling function for valve temp.  compensator tables.

-  FN022C = ECT normalizing function for transient fuel tables.

-  FN071L = LOAD Normalizing function; used for table lookup.

-  FN312S(ECT) = Change in the transient fuel  puddle  mass  per  change  in

aircharge.

-  FN313(N) = Equilibrium  intake  surface  fuel  engine  speed  multiplier,

unitless.

-  FN1323A = Transient fuel accel time  since  start  multiplier,  unitless.

This  empirical  table  compensates  for intake valve temperatures during

warm-up.  X = FN022C(ECT) Y = FN018(ATMR1)

-  FN1323D = Transient fuel decel time  since  start  multiplier,  unitless.

This  empirical  table  compensates  for intake valve temperatures during

warm-up.  X = FN022C(ECT) Y = FN018(ATMR1)

-  FN1321 = Equilibrium Intake Surface Fuel Table.

X = FN022C(ECT)

Y = FN071L(LOAD)

-  FN1322AL = Transient fuel time constant for accels.

X = FN022C(ECT)

Y = FN071L(LOAD)

-  FN1322DL = Transient fuel time constant for decels.

X = FN022C(ECT)

Y = FN071L(LOAD)

-  MTEFTC = Equilibrium Fuel transfer constant multiplier.

-  MTEISF = Equilibrium intake surface fuel multiplier.

8-103

FUEL STRATEGY, BACKGROUND TRANSIENT FUEL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  SARCHG = Standard Air Charge, lbm/Intake = 4.4256 * 10E-5 * (Engine  size

in cubic inches / number of cylinders).

-  TFC_IDLE_OFF = Calibration switch which disables use of transient fuel at

idle; 1 -> no TFC at idle, 0 -> TFC at idle.

-  TFCISW = Transient fuel compensation initialization switch.

-  TFCTM  =  Time  delay  after  start  before   enabling   Transient   Fuel

compensation.

-  TFC_MIN = The minimum compensation clip.

-  TFSMN = Maximum rpm deadband above Idle rpm  to  disable  Transient  Fuel

during Dashpot Mode, rpm.

8-104

FUEL STRATEGY, BACKGROUND TRANSIENT FUEL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FTRAN_XTMODEL_BG

TRANSIENT FUEL COMPENSATION BACKGROUND CALCULATIONS

The Transient Fuel Compensation (TFC) background calculations  are  performed

during  each  program  pass  (background  loop),  except that none of the TFC

background calculations are performed during Engine  Running  (ER)  On-Demand

Self Test.

BEGIN: tfc_bg_stage1_calculations

; Calculate (EISFU,EISFLO), the Equilibrium Intake Surface Fuel,

; to be used in the foreground TFC calculations if conditions permit.

; LOAD is recalculated here with the updated value for CYL_AIR_CHG.

; TFC_CHG is stored for use in the delta aircharge calculation in the

; foreground.

; This routine is not executed during Engine Running (ER) On-Demand

; VIP Self Test.

UNDSP = 0 ------------------------------| Increment TFC_TMR

;in run mode.                          |   ;clips at 255.

|

| --- ELSE ---

|

| TFC_TMR := 0

TFC_TMR < TFCTM ------------------------| EFFLG1 := 0

;make sure manifold emptying is over.  | TFC_HR := 0

| TFC_FG_EN := 0

| EXIT this process.

|

|  ;Do not perform transient

|  ;fuel compensation.

|

| --- ELSE ---

|

| LOAD := CYL_AIR_CHG / SARCHG

|   ;update load value.

|

| Do: tfc_bg_stage2_calculations

END: tfc_bg_stage1_calculations

8-105

FUEL STRATEGY, BACKGROUND TRANSIENT FUEL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: tfc_bg_stage2_calculations

;This routine is executed ONLY when it is invoked from within the

;tfc_bg_stage1_calculations routine.  Please note:  order of

;execution in the action blocks is most critical.

EFFLG1 = 0 -----------------------|

|AND -| EFFLG1 := 1

TFCISW = 1 -----------------------|     | (AISFU,AISFLO) := FN1321(ECT,LOAD)

|       * FN313(N) * MTEISF

| (EISFU,EISFLO) := (AISFU,AISFLO)

| TFC_HR := 0

| TFC_FG_EN := 0

|

|  (The FN1321 look-up routine MUST

|   return a 16 bit result from the

|   table interpolation algorithm,

|   NOT the standard 8 bit result!)

|

| --- ELSE ---

EFFLG1 = 0 -----------------------|     |

|AND -| EFFLG1 := 1

TFCISW = 0 -----------------------|     | (AISFU,AISFLO) := 0

;assume dry manifold at startup        | TFC_HR := 0

| TFC_FG_EN := 0

|

| --- ELSE ---

|

DFSFLG = 1 -----------------------------| (AISFU,AISFLO) := (EISFU,EISFLO)

|                    * AISFM

;in DFSO                               | TFC_HR := 0

| Do: tfc_bg_stage3_calculations

| TFC_FG_EN := 0

|

| --- ELSE ---

TFC_IDLE_OFF = 1 -----------|           |

|AND -|     |

ISCFLG > 0 -----------------|     |     |

;rpm control or lockout          |OR --| (AISFU,AISFLO) := (EISFU,EISFLO)

|     | TFC_HR := 0

ISCFLG = -1 ----------------|     |     | Do: tfc_bg_stage3_calculations

|AND -|     | TFC_FG_EN := 0

N - DSDRPM < TFSMN ---------|           |

;low rpm decel                         |

| --- ELSE ---

|

| Do: tfc_bg_stage3_calculations

| TFC_FG_EN := 1

END: tfc_bg_stage2_calculations

8-106

FUEL STRATEGY, BACKGROUND TRANSIENT FUEL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: tfc_bg_stage3_calculations

;This routine is executed ONLY when it is invoked from within the

;tfc_bg_stage2_calculations routine.

;Calculate the Equilibrium Fuel Time Constant (EFTC), the Transient

;Fuel Compensation MULTiplier (TFC_MULT), and TFC_SLOPE, (EISFU,EISFLO).

;NOTE:  For increase resolution a 32 bit representation (double word)

;be used for AISF and EISF.  These parameters are held in two word

;registers referenced as (parm(U), parm(LO)).

unconditionally ------------------------| TFC_SLOPE := FN312S(ECT)

|             * FN313(N)

| TFC_CHG := CYL_AIR_CHG

| (EISFU,EISFLO) := FN1321(ECT,LOAD)

|                * FN313(N) * MTEISF

|

| FPW_FG_ISF_A := (FPW_FG_ISF_C *

|                  (EISFU,EISFLO)) /

|                   / AHISL_TICK

|

|  (The FN1321 look-up routine MUST

|   return a 16 bit result from the

|   table interpolation algorithm,

|   NOT the standard 8 bit result!)

| EFTCD := FN1322DL(ECT,LOAD)

|        *  MTEFTC

| TFC_MULTD := FN1323D(ECT,ATMR1)

|            * T_TO_S * DT12S

|  ;use decel multiplier

|

| EFTCA := FN1322AL(ECT,LOAD)

|        *  MTEFTC

| TFC_MULTA := FN1323A(ECT,ATMR1)

|            * T_TO_S * DT12S

|  ;use accel multiplier

END: tfc_bg_stage3_calculations

8-107

FUEL STRATEGY, FOREGROUND TRANSIENT FUEL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.13.2  FOREGROUND TRANSIENT FUEL LOGIC (CDAL0)

OVERVIEW

Background:

A liquid fuel film resides on the walls of the  intake  manifold.   The  film

mass  varies  primarily  with  manifold  absolute  pressure and manifold wall

temperature.  During steady state conditions, the film mass is constant.  The

rates  of  condensation  and  evaporation  on  the  manifold walls are equal.

During transients, the film mass changes creating air/fuel ratio errors.

.  During accelerations, the film mass increases.  Fuel will condense faster

on  the manifold walls until equilibrium is reached.  In an uncompensated

system at stoichiometry, fuel is diverted from the  cylinders,  resulting

in a momentary lean condition.

.  During decelerations, the  film  mass  decreases.   Fuel  will  evaporate

faster  from  the  manifold  walls  until  equilibrium is reached.  In an

uncompensated system at stoichiometry, fuel is added  to  the  cylinders,

resulting in a momentary rich condition.

The problem is magnified in closed loop fuel systems because the fuel control

will incorrectly chase the transient air/fuel excursions.

Intent:

The Transient Fuel Compensation Strategy (TFC) augments the closed/open  loop

fuel control to keep cylinder events at the desired air/fuel ratio during all

engine transients.  This module documents those actions that  take  place  in

the foreground execution task.  The goals are:

.  To eliminate lean air/fuel excursions during accelerations.

.  To eliminate rich air/fuel excursions during decelerations.

DEFINITIONS

Registers:

-  AISFLO = Actual Intake Surface Fuel - Lower word of a double word  -  bin

32.

-  AISFU = Actual Intake Surface fuel - Upper word of a double  word  -  Bin

32.

-  CYL_AIR_CHG = Current cylinder air charge.

8-108

FUEL STRATEGY, FOREGROUND TRANSIENT FUEL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  DT12S = The period of time between two adjacent rising edges of PIP.

-  DT23S = Last DT12S time.

-  EFTC = Time constant for transient fuel.

-  EFTCA = Time constant for transient fuel - accel.

-  EFTCD = Time constant for transient fuel - decel.

-  EFTR = Equil fuel transfer rate; bin 16, lbm/sec.

-  EISFLO = Equilibrium intake surface fuel - lower word of  double  word  -

bin 32.

-  EISFU = Equilibrium intake surface fuel - upper word of double word - bin

32.

-  EISF_FGL = Equilibrium intake surface fuel lower word of  double  word  -

bin 32.

-  EISF_FGU = Equilibrium surface fuel foreground upper word of double  word

- bin 32.

-  TFC_CHG = Aircharge that transient fuel (background) was evaluated at.

-  TFC_FUEL = Computed fuel mass that is lost or gained in the cylinder from

the puddles.

-  TFC_MULT = Transient fuel multiplier.

-  TFC_MULTA = Transient fuel multiplier - accel.

-  TFC_MULTD = Transient fuel multiplier - decel.

-  TFC_SLOPE = Change in the  transient  fuel  puddle  mass  per  change  in

aircharge.

-  TFC_HR = Transient fuel compensation - high resolution.

Bit Flags:

-  ALT_PIP_MODE = Flag indicating that fuel and air foreground  calculations

are occurring every-other PIP.

-  TFC_FG_EN = Foreground update TFC_FUEL enable flag; 1 -> update TFC_FUEL.

Calibration Constants:

-  TFC_MIN = TFC_HR minimum magnitude clip.

8-109

FUEL STRATEGY, FOREGROUND TRANSIENT FUEL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FTRAN_XTMODEL_FG

TRANSIENT FUEL COMPENSATION FOREGROUND CALCULATIONS

BEGIN: fuel_transient_fg_calculations

;Called from within the FUEL_FG_PIPEDG strategy module.

;In the PIP service routine, just after the calculation of CYL_AIR_CHG,

;calculate TFC_HR.

1. Determine if foreground is enabled.

TFC_FG_EN = 0 --------------------------| EXIT THIS ROUTINE

(foreground disabled)                  |

| --- ELSE ---

|

| CONTINUE THIS ROUTINE

2. Calculate the equilibrium puddle mass based on EISF (background) and the

new CYL_AIR_CHG.

(EISF_FGU,EISF_FGL) = (EISFU,EISFLO) + [(CYL_AIR_CHG - TFC_CHG) * TFC_SLOPE]

3.  Decide if currently in an accel or decel condition and use appropriate

EFTC and TFC_MULT parameter

(EISF_FGU,EISF_FGL) < (AISFU,AISFLO) --------| TFC_MULT = TFC_MULTD

| EFTC = EFTCD

|

| --- ELSE ---

|

| EFTC = EFTCA

| TFC_MULT = TFC_MULTA

4. Calculate EFTR; the puddle mass transfer rate.

EFTR = ((EISF_FGU,EISF_FGL)  - (AISFU,AISFLO)) / EFTC

5. Calculate AISF; the actual intake surface fuel.

ALT_PIP_MODE = 1 ----------------------------| dt_time := DT12S + DT23S

|

| --- ELSE ---

|

| dt_time := DT12S

(AISFU,AISFLO) = (AISFU,AISFLO) + (EFTR * dt_time * T_TO_S)

(clip AISF to zero as a minimum)

8-110

FUEL STRATEGY, FOREGROUND TRANSIENT FUEL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

6. Calculate TFC_HR; the transient fuel compensation.

tfc_hr = EFTR * TFC_MULT

|tfc_hr| > TFC_MIN ---------------------| TFC_HR = tfc_hr

(magnitude of tfc_hr great enough)     |

| --- ELSE ---

|

| TFC_HR = 0

END: fuel_transient_fg_calculations

8-111

FUEL STRATEGY, HOT INJECTOR COMPENSATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.14  HOT INJECTOR COMPENSATION (CDAN0)

OVERVIEW

Under conditions of high injector tip temperatures,  injector  fuel  delivery

decreases  as  a function of increasing injector tip temperature.  The amount

of vaporized fuel delivered by  the  injector  increases  as  hot  soak  time

increases,  or  as  conducted  heat (from cylinder head) and/or radiated heat

(from intake/exhaust manifold) increases.  Higher fuel pressure or lower fuel

volatility  helps the situation, but fuel volatility is beyond the developers

control.  Hot Injector Compensation has therefore been applied to the desired

fuel flow.

The engine-off soak timer  is  incorporated  for  those  programs  with  this

hardware  feature.   Also,  values of ACT and ECT are periodically updated in

KAM (when the HICOMP value has become sufficiently  small),  where  they  are

retained  until  the  next  power  up.   The  implementation  below  uses the

following in the calculation of this enrichment factor.

-  For ACT and ECT, the difference between the temperature  during  previous

run, and the current temperature.  These values are used in FN1338, which

is used to calibrate the enrichment  factor  as  a  function  of  current

temperature vs.  temperature change during soak.

-  In FN1348, the enrichment factor is calibrated  as  a  function  of  time

spent in run mode vs.  current air mass.

-  The engine-off soak timer is also used.   FN1347  allows  the  enrichment

factor to be characterized as a function of soak time vs.  temperature at

previous shut down.

The initial value of FN1338 is now used in the HICOMP calculation, insuring a

smooth   ramp-down   based   solely   upon   FN1348   and  FN1347.   Using  a

continuously-updated value of FN1338 leads to a faster ramp down  of  HICOMP,

and excessively lean operation.

CALIBRATION NOTES

If it is desired to update the ACT and ECT values in KAM (ACTOFF and  ECTOFF)

continuously, set HICOMP_MIN to the upper clip.

With the addition of the soak timer (FN1347), it may be possible to eliminate

FN1338  from  the HICOMP calculation altogether.  Assigning a value of 1.0 to

each component of this table will achieve this.

To reset the engine-off  soak  time,  simply  entering  run  mode  should  be

sufficient.   A  low  value  of  SOAK_RST_TM  (e.g.,  0-5  seconds) should be

satisfactory for most applications.

8-112

FUEL STRATEGY, HOT INJECTOR COMPENSATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  ACT = Air Charge Temperature, degrees F.

-  ACTOFF = Air Charge Temperature last read  during  closed  loop  control,

degrees F.

-  AM = Air mass flow, lb/min.

-  ATMR3 = Time since entering RUN mode, seconds.

-  ECT = Engine Coolant Temperature, degrees F.

-  ECTOFF = Engine Coolant Temperature last read during closed loop control,

degrees F.

-  HICOMP = Hot injector compensation enrichment factor.

-  INITL_FN1338 = The value of FN1338 taken  at  start-up.   This  value  is

determined  during  the  first pass through the background loop, and then

never again updated.

-  SOAK_HICOMP = Duration of last soak, in seconds.  This is  obtained  from

the  engine-off  soak  timer (ENG_OFF_TMR), and is reset when the vehicle

enters run mode.  Retained in KAM.

-  SOAK_TIME = Local copy of SOAK_HICOMP, used only  in  the  HICOMP  logic;

stored in RAM.

Bit Flags:

-  HICOMP_INIT = Flag indicating that the HICOMP strategy has been  executed

since   power-up.    Use  for  one-time  only  initialization  of  HICOMP

registers.

-  KAM_ERROR = Indicates keep-alive RAM is invalid.

-  UNDSP =  Crank/underspeed  mode  flag;  0  ->  run  mode,  1->  crank  or

underspeed mode.

Calibration Constants:

-  FN001 = Delta temperature (actual minus  shutdown)  normalizing  function

for HICOMP.

-  FN001A = Absolute temperature normalizing function for HICOMP.

-  FN001B = Key-off power-off engine temperature  normalizing  function  for

HICOMP.

-  FN007 = AM normalizing function.

-  FN008 = ATMR3 normalizing function.

-  FN008B = SOAK_TIME normalizing function.

8-113

FUEL STRATEGY, HOT INJECTOR COMPENSATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FN1338 = HICOMP multiplier.  X = FN001(delta_temp) Y = FN001A(abs_temp)

-  FN1347   =   HICOMP   multiplier.    X   =    FN008B(SOAK_TIME)    Y    =

FN001B(power_off_temp)

-  FN1348 = HICOMP multiplier.  X = FN008(ATMR3) Y = FN007(AM)

-  HC_ACT_MUL1 = ACT  multiplier  used  in  delta  temperature  computation,

unitless.

-  HC_ACT_MUL2 = ACT multiplier used in  absolute  temperature  computation,

unitless.

-  HC_ACT_MUL3  =  ACT  multiplier  used  in  computation  involving  engine

temperatures  at  the  most  recent  key-off power-down event.  Unitless.

Used to define a normalized combination of ECTOFF and ACTOFF.

-  HC_ECT_MUL1 = ECT  multiplier  used  in  delta  temperature  computation,

unitless.

-  HICOMP_MIN = Minimum HICOMP value before HICOMP is reset.

-  SOAK_RST_TM = Minimum time required in run mode to initiate an update  of

the engine-off soak timer at next power up, in seconds (0-255).

8-114

FUEL STRATEGY, HOT INJECTOR COMPENSATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_HOT_INJ_COMP_COM4

Initialize values of ACT and ECT stored in KAM, when KAM error is detected.

***************************************************************

***  NOTE: The values of registers ACT and ECT must be      ***

***        established prior to executing the following     ***

***        initialization of ACTOFF and ECTOFF during a     ***

***        KAM reset                                        ***

***************************************************************

KAM_ERROR = 1 ---------------------| ACTOFF := ACT

| ECTOFF := ECT

On the first background pass, make a local copy of the soak-time  accumulator

for  HICOMP.   The  accumulator (SOAK_HICOMP) is calculated in the soak timer

input  processing  logic.   Upon  each  power-up,  the   actual   soak   time

(ENG_OFF_TMR)  will  be  added  to SOAK_HICOMP.  This HICOMP logic will clear

SOAK_HICOMP when the engine has entered run mode for a significant period  of

time (see below).

HICOMP_INIT = 0 -------------------| SOAK_TIME := SOAK_HICOMP

Check if duration in run mode  is  sufficient  to  trigger  a  reset  of  the

engine-off   soak   timer   accumulator   for   Hot   Injector   Compensation

(SOAK_HICOMP).

ATMR3 > SOAK_RST_TM ---------------| SOAK_HICOMP := 0

| ;Been in run mode long enough to reset

| ; the soak timer

Calculate intermediate values required  for  the  hot  injector  compensation

enrichment factor.

Unconditionally -------------------| delta_act := ACT - ACTOFF

| delta_ect := ECT - ECTOFF

| delta_temp := HC_ECT_MUL1 * delta_ect

|             + HC_ACT_MUL1 * delta_act

|

| abs_temp := ECT + HC_ACT_MUL2 * ACT

| power_off_temp := ECTOFF +

|                 HC_ACT_MUL3 * ACTOFF

Get the initial value of FN1338, and indicate that the  HICOMP  strategy  has

been initialized this power up.

HICOMP_INIT = 0 -------------------| INITL_FN1338 := FN1338

|                 (delta_temp,abs_temp)

| HICOMP_INIT := 1

8-115

FUEL STRATEGY, HOT INJECTOR COMPENSATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calculate the Hot Injector Compensation enrichment factor.

Unconditionally -------------------| HICOMP := INITL_FN1338 *

|         FN1348(ATMR3,AM) *

|         FN1347(SOAK_TIME,power_off_temp)

Reset HICOMP, as necessary.

HICOMP <= HICOMP_MIN --------------| ACTOFF := ACT

| ECTOFF := ECT

|

| --- ELSE ---

|

| ;No change to ACTOFF or ECTOFF; use

| ; last value read while closed loop.

| ; ACTOFF and ECTOFF are stored in KAM.

8-116

FUEL STRATEGY, INJECTOR DELAY LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.15  INJECTOR DELAY LOGIC (CDAK0)

OVERVIEW

Injector timing is the delay before each injector output is fired.  The delay

is  taken from TDC of the reference PIP signal for each injector output.  The

units for injector timing are absolute engine crank degrees.   The  range  is

clipped between 0 and 716 degrees ATDC with 4 degrees resolution.

Either edge (on or off) of the injector pulse can be timed.  The  calibration

switch INJREF defines the reference edge for injector timing.

INJREF = 0, Use start of fuel pulse.

INJREF = 1, Use end of fuel pulse.

Regardless of which edge is used, the injector delay is used only to start  a

fuel pulse.  Once started, pulsewidth accuracy has top priority.

When the fuel system is not in sync (SYNFLG =  0),  the  injector  timing  is

coincident  with  the relevant PIP signal.  The injector outputs are fired in

sequence after receiving the  rising  edge  of  the  reference  PIP  signals.

Actual  timing  will  be  the result of the random link between signature PIP

(cylinder #1) and #1 injector output.

During RAM initialization, the rolling average filter for  INJDLY  is  loaded

with an initial value, INJIV.  This action eliminates the excessive length of

time it used to take to filter INJDLY from its initial  value  of  0  to  its

desired   timing.   Note:   For  1989  Model  year,  because  of  a  software

limitation, INJIV must be calibrated to less than ENGCYL * 90 degrees.   This

will be corrected for 1990.

When the fuel system is in sync (SYNFLG = 1), injector timing is based on the

calculation defined within this strategy module.

NOTES:

1.  The final value of injector delay is limited to a range of  greater  than

or  equal  to 0, and less than 720, engine crank angle degrees.  Intermediate

calculations and results are maintained in an unlimited fashion.

2.  The user must set DEGPIP to match his engine.  DEGPIP is  the  number  of

engine  crank  degrees  per  PIP  interval.   (90  for  8-cylinder,  120  for

6-cylinder, 180 for 4-cylinder).

8-117

FUEL STRATEGY, INJECTOR DELAY LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EXAMPLE - Injector Timing

INJDLY = 400 DEGREES

ENGINE CYCLE

TDC            BDC            TDC             BDC

|              |              |               |                |

|              |              |               |                |

|<----POWER--->|<---EXHAUST-->|<---INTAKE---->|<-COMPRESSION-->|

|     STROKE   |    STROKE    |    STROKE     |  STROKE        |

|              |              |               |                |

0             180            360             540              720

| 1ST PIP

PIP SIGNAL

| #1 PIP

|

****  *****    *****  *****   *****   *****   *****   *****    **

*  *  *   *    *   *  *   *   *   *   *   *   *   *   *   *    *

*  *  *   *    *   *  *   *   *   *   *   *   *   *   *   *    *

*  *  *   *    *   *  *   *   *   *   *   *   *   *   *   *    *

**  ****   ******   ****   *****   *****   *****   *****   ******

|    |        |      |       |       |       |

0   80       170    260     350     440     530

TDC                                 |

|                                  |

|<--------- INJDLY = 400 --------->|

|                                  |

|ON       INJECTOR  1         ******

|                             *    *

|                             *    *

|OFF             INJREF = 1   *    *

***********************************    ******************************

|                                  |

|         INJECTOR  1              |

|         INJREF = 0               ******

|                                  *    *

|                                  *    *

|                                  *    *

****************************************    *************************

8-118

FUEL STRATEGY, INJECTOR DELAY LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  ATMR1 = Time since exiting crank mode, sec.

-  ATMR2 = Time since engine coolant temperature became greater than TEMPFB,

sec.

-  ECT = Engine Coolant Temperature, deg F.

-  INJDLY = Injector delay in degrees.

-  LOAD = Universal load as ratio of aircharge over standard.

-  N = Engine speed, rpm.

-  TCSTRT = Temperature of engine coolant (ECT) at initial startup, deg F.

Bit Flags:

-  CRKFLG = Flag indicating when in CRANK mode; 1 -> crank.

-  DNDSUP = Delayed Neutral/Drive flag; 0 -> neutral, 1 -> drive.

-  IDLE_TEMP_SW = INJDLY temperature switch; 1 -> temperature compensation

in idle, 0 -> no temperature compensation in idle.

-  IDLFLG = Flag indicating transmission in Drive and at Idle.

-  TIM_INT_FLG = First pass injector timing calculation.

Calibration Constants:

-  CIDRSW = Calibration switch to enable Special Fuel Timing at Idle,

unitless; 1 -> Enable, 0 -> Disable.

-  CINTSW = Calibration switch to enable special fuel timing in Neutral,

unitless;

1 -> enable special neutral fuel timing.

-  CINTV = Injector Timing Value in Neutral, deg F.

-  CTHIGH = Coolant temperature at Hot start, deg F.

-  CTLOW = Coolant temperature at Cold start, deg F.

-  DEGPIP = Engine degrees per PIP period, deg. (90 deg = 8 cylinder;

120 deg = 6 cylinder; 180 = 4 cylinder)

-  FN070 = Normalizing function for engine speed N as X-input to FN1315.

-  FN085 = Normalizing function for LOAD as Y-input to FN1315.

-  FN336(ECT) = Injector timing versus ECT (X-input).

8-119

FUEL STRATEGY, INJECTOR DELAY LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FN1315 = An 10x8 table which gives injector delay as a function of

engine speed and load.

X = FN070(N)

Y = FN085(LOAD)

-  IDKADD = Injector delay timing adder.

-  IDKMUL = Injector delay timing multiplier.

-  INJDLY_MIN = Minimum allowable injector delay.

-  INJREF = Indicates which edge of the fuel pulse is the reference edge for

fuel timing, INJDLY; 0 -> start edge.

-  MIDTV = Injector timing value for Idle in Drive, deg.  Range of 0 to 720.

-  NITMR1 = ATMR1 timed delay to enter Closed Loop fuel  after  Cold  start,

sec.  Range of 0-255 sec., accuracy 1 sec.

-  NITMR2 = ATMR1 timed delay to enter Closed Loop fuel after Medium  start,

sec.  Range of 0-255 sec., accuracy 1 sec.

-  NITMR3 = ATMR1 timed delay to enter Closed Loop  fuel  after  Hot  start,

sec.  Range of 0 to 255 sec., accuracy 1 sec.

-  NITMR4 = ATMR2 timed delay to enter Closed Loop fuel  after  Cold  start,

sec.  Range of 0 to 255 sec., accuracy 1 sec.

-  NITMR5 = ATMR2 timed delay to enter Closed Loop fuel after Medium  start,

sec.  Range of 0 to 255 sec., accuracy 1 sec.

-  TCINJD = Time constant for INJDLY, sec.

OUTPUTS

Registers:

-  INJDLY = Injector delay in degrees.

-  INJDLY_BETA = The injector timing in percent of PIP periods.

Bit Flags:

-  TIM_INT_FLG = See above.

8-120

FUEL STRATEGY, INJECTOR DELAY LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_INJDLY_COM1

; Determine if the conditions to use special fuel timing in neutral are met.

TCSTRT >= CTHIGH -----------|

|AND -|

ATMR1 >= NITMR3 ------------|     |

|

CTLOW < TCSTRT < CTHIGH ----|     |

|AND -|

ATMR1 >= NITMR2 ------|     |     |OR --| "A"

|AND -|     |

ATMR2 >= NITMR5 ------|           |

|

TCSTRT <= CTLOW ------------|     |

|AND -|

ATMR1 >= NITMR1 ------|     |

|AND -|

ATMR2 >= NITMR4 ------|

8-121

FUEL STRATEGY, INJECTOR DELAY LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

; Compute the desired value for injector delay.  This is based on calibration

; values MIDTV and CINTV, ECT-based temperature compensation (FN336) and

; normalized speed/load compensation.

CRKFLG = 1 -----------------------------| Exit FUEL_INJDLY

| ;Exit background injector timing

| ; calculation

|

| --- ELSE ---

DNDSUP = 0 -----------------------|     |

|     |

CINTSW = 1 -----------------------|     |

|AND -| ;Neutral timing with temperature

"A" ------------------------------|     | ; compensation

|     | injdly := CINTV + FN336(ECT)

IDLE_TEMP_SW = 1 -----------------|     |

| --- ELSE ---

DNDSUP = 0 -----------------------|     |

|     |

CINTSW = 1 -----------------------|AND -| ;Neutral timing without temperature

|     | ; compensation

"A" ------------------------------|     | injdly := CINTV

|

| --- ELSE ---

IDLFLG = 1 -----------------------|     |

|     |

CIDRSW = 1 -----------------------|AND -| ;Idle timing with temperature

|     | ; compensation (in drive)

IDLE_TEMP_SW = 1 -----------------|     | injdly := MIDTV + FN336(ECT)

|

| --- ELSE ---

IDLFLG = 1 -----------------------|     |

|AND -| ;Idle timing without temperature

CIDRSW = 1 -----------------------|     | ; compensation (in drive)

| injdly := MIDTV

|

| --- ELSE ---

|

| injdly :=

|        IDKMUL * (FN1315(N,LOAD)

|        + FN336(ECT)) + IDKADD

8-122

FUEL STRATEGY, INJECTOR DELAY LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

; The following section calculates the desired injector delay in absolute

; crank angle degrees after TDC of the cylinder associated with the

; reference PIP signal edge.

; Normalize the desired injector delay to be between 0° and 720°

unconditionally ---------------------| injdly := MOD(injdly,720)

; If this is the first pass through the background loop, store injdly as

; the 'old' value of INJDLY (required for the rolling average).

TIM_INT_FLG = 0 ------------------------| INJDLY := injdly

;First pass only                       | TIM_INT_FLG := 1

|  ;TIM_INT_FLG remains set after the

|  ; first execution of this block

|  ; after RAM initialization

; Walk the injector timing INJDLY toward the desired value injdly, taking

; the shortest route.  Note that this may go indirectly, through 0° and

; wrapping back to 720°, or vise versa.

ABS(injdly - INJDLY) > 360 ----|

|AND -| injdly := injdly - 720

injdly > INJDLY ---------------|     | ;Adjust the rolling average in a

| ; negative direction, through 0°

|

| --- ELSE ---

ABS(injdly - INJDLY) > 360 ----|     |

|AND -| injdly := injdly + 720

injdly < INJDLY ---------------|     | ;Adjust the rolling average in a

| ; positive direction, through 720°

8-123

FUEL STRATEGY, INJECTOR DELAY LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

; Use a first-order filter to "walk" the current injector delay INJDLY

; toward the desired value.  Wrap the result from 0° to 720° or back,

; using modulo 720 arithmetic.  Clip the final result to INJDLY_MIN as

; a minimum.

unconditionally ----------------------| INJDLY := ROLAV(injdly,TCINJD)

| INJDLY := MOD(INJDLY,720)

| INJDLY := MAX(INJDLY,INJDLY_MIN)

; The following section calculates the desired injector delay in units of

; percent of PIP period, "betas", relative to the associated PIP up-edge

; event.  If the injector delay (INJDLY, in units of degrees after top

; dead center) plus the PIP offset before TDC (10 degrees) is greater than

; or equal to 720 degrees, then the requested delay will be past TDC and

; 720 can be subtracted to give the same delivered timing.

unconditionally ------------------------| temp := INJDLY + 10

temp < 720 -----------------------------| INJDLY_BETA := temp/DEGPIP

|

| --- ELSE ---

|

| INJDLY_BETA := (temp - 720)/DEGPIP

8-124

FUEL STRATEGY, DECEL FUEL SHUT OFF LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.16  DECEL FUEL SHUT OFF LOGIC (CDAL0)

OVERVIEW

The Fuel Shutoff strategy is divided into two sub-strategies:

1) Decel Fuel Shut Off

2) Manual Transmission Shift Fuel Shut Off.

The Decel Fuel Shut Off strategy turns off the fuel flow during specific load

and  rpm  condition only during Closed Throttle Mode.  The FUELPWs are zeroed

during a decel until the rpm is within the band created by [DFSRPM -  DFSRPH]

of the idle rpm range.

The Manual Transmission Shift Fuel Shut Off strategy turns off fuel  flow  at

the  beginning  of  the  shift,  until  the rpm is within the band created by

[SHFRPM - SHFHYS] of the idle speed (or until the transmission  has  been  in

Neutral for DSTM2 seconds).

Calibration Guide:

The use of Decel Fuel Shutoff can aggravate Clunk.   Therefore,  DFSO  should

occur  only at low airflow in order to minimize the rate of change of Torque.

CTDFSO can be used to delay DFSO.

CTEDSO should be greater than CTDFSO to prevent DFSO after Tip-in, Tip-out.

For MANUAL TRANSMISSIONS:  SHFRPM should be greater than DFSRPM.

Notes on Alternate calibration for Misfire Profile Correction logic.

The normal Decel Fuel Shut Off sub-strategy can be controlled by two sets  of

calibratable  parameters,  depending upon the state of the Profile Correction

logic.  Whenever the appropriate KAM correction  factors  have  not  matured,

there  exists the need to apply the DFSO strategy in a more aggressive manner

in order  to  allow  adequate  unfueled  engine  rotation  to  determine  the

crankshaft profile signature.  The RAM bit CF_KAM_MTR is utilized within this

strategy to indicate the state of the Profile Correction KAM  maturity.   The

more  aggressive  calibration  constants  will allow DFSO to occur in a wider

operating region than those assigned in  the  normal  powertrain  development

process.   This  alternate  calibration is intended to occur for only a short

period of vehicle operation whenever KAM has been reset.  The  strategy  will

automatically revert back to the normal DFSO calibration upon indication that

the Profile Correction factors have matured.

8-125

FUEL STRATEGY, DECEL FUEL SHUT OFF LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  APT = At Part Throttle; -1 -> Closed throttle, 0 -> Part throttle,

1 -> Wide Open throttle.

-  CTTMR = Time since entering Closed throttle mode, secs.  CTTMR is used to

delay Decel Fuel Shutoff (DFSO).

-  CYLARC_BG = Air Charge Mass inducted per Intake Stroke, lbm/Intake.

-  DSDRPM = Desired engine speed.

-  ECT = Engine Coolant Temperature, deg F.

-  LOAD = Normalized CYLARC_BG divided by SARCHG.

-  N = Engine speed, rpm.

-  NDDTIM = Time since Transmission shift, sec.  In the DFSO Strategy,  this

timer  is  used  to  delay  Decel Fuel Shutoff if a vehicle with a manual

transmission shifts into Neutral during the decel.

-  NOVS = N/VSBAR to infer transmisssion gear.

-  PPCTR = PIP counter for Fuel Ramp, unitless.

-  VSBAR = Vehicle speed, mph.

Bit Flags:

-  ALT_DFSO_FLG = Flip-flop flag used to enable alternate  DFSO  calibration

for profile correction strategy.

-  CF_KAM_MTR = Profile  Correction  indicates  that  the  KAM  has  matured

correction factors; 1 -> CF's are mature.

-  COOL_FLG = Flag indicating  the  lack  of  coolant  strategy  is  active;

cylinder head overtemperature condition; 1 -> overtemperature.

-  DCELQ4 = Flip flop state flag using engine  speed  to  disable  DFSO  for

manual transmissions during gear shifts.

-  DCELQ5 = Flip flop state flag using engine speed and load to engage  DFSO

during closed throttle decelerations.

-  DFSFLG = Indicates DECEL Fuel shutoff.

-  DFSVS_HYS_FG = Flip flop state flag using vehicle speed to engage DFSO.

-  FLG_DFSO_LOAD = Flip flop state flag using low load values to indicate  a

hard decel to engage DFSO.

-  FLG_DFSO_NOVS = Flip flop state flag using transmission  gear  to  engage

DFSO.

8-126

FUEL STRATEGY, DECEL FUEL SHUT OFF LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  NDSFLG = Neutral/Drive Flag; 1 -> Drive.

Calibration Constants:

-  CTDFSO = Minimum time delay after  entering  Closed  Throttle  to  enable

DFSO, sec.

-  CTDFSO_PC = Profile Correction - Minimum time delay after entering Closed

Throttle to enable DFSO, sec.

-  CTEDSO = Decel Fuel Shutoff time delay during extended decels.  This time

delay  allows  DFSO during extended decels (ie., down mountains), even if

times at Part Throttle are very short (light Tip-ins), sec.

-  CTEDSO_PC = Profile Correction - Decel Fuel  Shutoff  time  delay  during

extended  decels.   This  time  delay  allows DFSO during extended decels

(ie., down mountains), even if times at  Part  Throttle  are  very  short

(light Tip-ins), sec.

-  DFHLOD = Minimum load for hard decel fuel shut off in lower gears.

-  DFHLOD_PC = Profile Correction - Minimum load for hard  decel  fuel  shut

off in lower gears.

-  DFHLODH = Hysteresis for DFHLOD.

-  DFLOD = Maximum load to enable decel Fuel Shut-off.

-  DFLOD_PC = Profile  Correction  -  Maximum  load  to  enable  decel  Fuel

Shut-off.

-  DFLODH = Hysteresis for DFLOD.

-  DFNOVS = NOVS value below which DFSO permitted.

-  DFNOVS_PC = Profile Correction - NOVS value below which DFSO permitted.

-  DFNOVH = Hysteresis for DFNOVS.

-  DFSECT = Minimum ECT to allow Decel Fuel Shutoff.

-  DFSECT_PC = Profile Correction - Minimum ECT to allow Decel Fuel Shutoff.

-  DFSO_PC_SW = Switch to  allow  use  of  alternate  DFSO  calibration  for

Profile Correction strategy; 1 -> use alternative DFSO calibration.

-  DFSVS = Minimum VSBAR for Decel fuel shut off, mph.

-  DFSVS_PC = Profile Correction - Minimum VSBAR for Decel  fuel  shut  off,

mph.

-  DFSVSH = Hysteresis for DFSVS.

-  DFSRPH = Hysteresis for DFSRPM, unitless.

-  DFSRPM = Minimum rpm for Decel fuel shutoff, rpm.

8-127

FUEL STRATEGY, DECEL FUEL SHUT OFF LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  DFSRPM_PC = Profile Correction - Minimum rpm for Decel fuel shutoff, rpm.

-  DSTM2  =  Maximum  time  that  DFSO  logic  is  enabled   during   manual

transmission shifts, secs.

-  NDFSO = Neutral deceleration fuel shut off; 1 -> DFSO in neutral.

-  PIPNUM = Number of PIPS to remain in Open Loop Fuel after DFSO.  Prevents

LAMBSE from ramping rich due to normal transport delay time.  Set to 1 to

calibrate out.

-  SHFHYS = Hysteresis term, rev/min.  If the rpm  drops  this  amount,  the

normal Fuel Strategy operates.

-  SHFRPM = Minimum rpm at which DFSO  logic  is  enabled,  rev/min.   (This

corresponds to the rpms during the shift.)

-  TRLOAD = Transmission Load switch;

0 -> Manual Transmission, no clutch or gear switches,

forced neutral state (NDSFLG = 0).

1 -> Manual Transmission, no clutch or gear switch.

2 -> Manual Transmission, one clutch or gear switch.

3 -> Manual Transmission, both clutch and gear switches.

4 -> Auto Transmission, non-electronic, neutral drive switch.

5 -> Auto Transmission, non-electronic, neutral pressure

switch, (AXOD).

6 -> Auto Transmission, electronic, PRNDL sensor - park,

reverse, neutral, overdrive, manual 1, manual 2.

8-128

FUEL STRATEGY, DECEL FUEL SHUT OFF LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_DFSO_COM9

BEGIN:  alternate_dfso_calibration

;Execute prior to any DFSO logic service.

CF_KAM_MTR = 0 ----------------|

; Profile Correction Logic    |

; indicates the correction    |AND -|S Q -| ALT_DFSO_FLG

; factor KAM is NOT mature.   |     |

|     |

DFSO_PC_SW = 1 ----------------|     |

; Calibratable switch.              |

|

|

CF_KAM_MTR = 1 ----------|           |

; KAM is mature.        |AND -|     |

|     |     |

DFSFLG = 0 --------------|     |     |

; Out of DFSO.                |OR --|C

|

DFSO_PC_SW = 0 ----------------|

; Calibratable switch.

ALT_DFSO_FLG = 1 --------------------| ctdfso = CTDFSO_PC

; Use alternative DFSO for profile  | ctedso = CTEDSO_PC

; correction factor KAM update.     | dfsvs  = DFSVS_PC

| dfhlod = DFHLOD_PC

| dfnovs = DFNOVS_PC

| dfsrpm = DFSRPM_PC

| dflod  = DFLOD_PC

| dfsect = DFSECT_PC

|

| --- ELSE ---

|

; Use normal DFSO calibration.      | ctdfso = CTDFSO

| ctedso = CTEDSO

| dfsvs  = DFSVS

| dfhlod = DFHLOD

| dfnovs = DFNOVS

| dfsrpm = DFSRPM

| dflod  = DFLOD

| dfsect = DFSECT

END: alternate_dfso_calibration

8-129

FUEL STRATEGY, DECEL FUEL SHUT OFF LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Evaluate each flip flop each background pass:

VSBAR >= dfsvs ----------------------|S Q -| DFSVS_HYS_FG

|

VSBAR < dfsvs - DFSVSH --------------|C

LOAD <= dfhlod ----------------------|S Q -| FLG_DFSO_LOAD

(hard decel)                        |

|

LOAD > dfhlod + DFHLODH -------------|C

NOVS <= dfnovs ----------------------|S Q -| FLG_DFSO_NOVS

(in higher gear)                    |

|

NOVS > dfnovs + DFNOVH --------------|C

N - DSDRPM > SHFRPM -----------------|S Q -| DCELQ4

|

N - DSDRPM <= SHFRPM - SHFHYS -------|C

(Short rpm)

N - DSDRPM > dfsrpm -----------|

(RPM high enough)             |AND -|S Q -| DCELQ5

|     |

LOAD <= dflod -----------------|     |

(Not making power)                  |

|

N - DSDRPM < dfsrpm - DFSRPH --|     |

|OR --|C

LOAD > dflod + DFLODH ---------|

8-130

FUEL STRATEGY, DECEL FUEL SHUT OFF LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

"A" LOGIC

MANUAL OR AUTO TRANS - DFSO DURING CLOSED THROTTLE DECEL

NDSFLG = 1 --------------------|

(Trans. in gear)              |OR --|

|     |

NDFSO = 1 ---------------------|     |

|

CTTMR >= ctdfso ---------------------|

|

CTTMR > ctedso ----------------|     |

(Extended decel)              |     |AND -| "A"

|     |

FLG_DFSO_LOAD = 1 -------------|OR --|

|     |

FLG_DFSO_NOVS = 1 -------------|     |

|

DCELQ5 = 1 --------------------------|

"B" LOGIC

MANUAL TRANSMISSION - DFSO DURING GEAR CHANGE

TRLOAD <= 3 -------------------------|

(Manual transmission)               |

|

NDSFLG = 0 --------------------------|

|AND -| "B"

NDDTIM < DSTM2 ----------------------|

|

DCELQ4 = 1 --------------------------|

8-131

FUEL STRATEGY, DECEL FUEL SHUT OFF LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

"A" ---------------------------|

|OR --|

"B" ---------------------------|     |

|

APT = -1 ----------------------------|

(Closed throttle)                   |

|

ECT >= dfsect -----------------------|AND -| TURN OFF FUEL

(Engine warmed up)                  |     | D = 0

|     | DFSFLG = 1

DFSVS_HYS_FG = 1 --------------------|     | PPCTR = 0

|     |

COOL_FLG = 0 ------------------------|     |

(no engine overheating)                   |

| --- ELSE ---

|

| TURN ON FUEL

| (Driver wants to

|  accelerate)

| D = 1.0

| DFSFLG = 0

NOTE:  PPCTR is incremented in the foreground.

8-132

FUEL STRATEGY, FUEL PULSEWIDTH, BACKGROUND FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.17  BACKGROUND FUEL (CDAM0)

OVERVIEW

The desired fuel air ratio for each bank is calculated  once  per  background

loop, under non-cranking conditions.  In addition, a multiplier that contains

the decel fuel multiplier and the multiplier that indicates pulses per engine

event  are calculated each time.  Under underspeed conditions, the multiplier

also contains  the  underspeed  fuel  multiplier.   In  crank  mode,  a  fuel

pulsewidth  value  is  calculated  based  on  number  of PIP events in crank,

barometric pressure, and engine temperature.

NOTE:  Bracketed parameters are non-displayable temporary registers.

DEFINITIONS

Registers:

-  ACT = Air Charge Temperature, deg F.

-  AHISL_TICK  =  Dual  slope  high  slope  in  lbs/tick,  compensated   for

temperature.

-  ALOSL_TICK  =  Dual  slope  low  slope  in  lbs/tick,   compensated   for

temperature.

-  BKPT_PW  =  Dual  slope  fuel  breakpoint  divided  by  the   low   slope

(temperature compensation), ticks.

-  BP = Barometric Pressure, " Hg.

-  CRK_FUL = Fuel pulsewidth used in crank mode.

-  CRKPIP_CTR_BG = Background PIP counter for crank fuel.

-  CRK_PIP_INJ = Number of PIP periods per injector firing.

-  ECT = Engine Coolant Temperature, deg F.

-  F_A_RATIOn = Desired fuel-to-air ratio for EGO-n.

-  FB_DIFF = Difference between FUEL_BKPT divided by temperature compensated

high  slope  and  FUEL_BKPT divided by temperature compensated low slope,

ticks.

-  FUELPW1 = Fuel pulsewidth in clock ticks (EGO-1).

-  FUELPW2 = Fuel pulsewidth in clock ticks (EGO-2).

-  HICOMP = Hot injector compensation enrichment factor.

-  KAMRFn = Fuel correction factor obtained from the  adaptive  fuel  tables

that  use  the  EGO  sensor(s)  for input.  KAMRFn = [0.5 + LTMTBn(r,c)],

where:  c = column, r = row, n = bank number.

8-133

FUEL STRATEGY, FUEL PULSEWIDTH, BACKGROUND FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  LAM_FINAL1 = Commanded LAMBSE value for bank1.  Summation of  LAMBSE  and

LAM_MOD.

-  LAM_FINAL2 = Commanded LAMBSE value for bank2.  Summation of  LAMBSE  and

LAM_MOD.

-  LAMBSE_MOD1 = LAMBSE fuel modulation signal, added  to  LAMBSE1  to  give

LAM_FINAL1.

-  LAMBSE_MOD2 = LAMBSE fuel modulation signal, added  to  LAMBSE2  to  give

LAM_FINAL2.

-  LAMBSEn = The stoichiometric equivalence value; n = bank number.

-  LBMF_INJn = Fuel mass injected when an injector is turned on, lbm.

-  MINPW_LBMF = MINPW * ALOSL.

-  MINPWT = Minimum fuel pulsewidth in ticks.

-  PWOFF = Injector pulsewidth offset, clock ticks.

-  PWOFS_D = DISPLAY ONLY; injector pulsewidth offset  -  output  of  FN367,

msec.

-  VBAT = Battery voltage inferred from IIVPWR, volts.

Bit Flags:

-  ALT_CAL_FLG = Flag to indicate use of alternate calibration.

-  CRKFLG = Engine mode flag; 1 -> crank mode, 0 -> underspeed/run mode.

-  DFSFLG = Indicates DECEL Fuel shutoff is active; 1 -> DFSO requested.

-  UNDSP = Engine mode flag; 1 -> crank/underspeed mode, 0 -> run mode.

Calibration Constants:

-  AHISL = High fuel flow injector slope, lb/sec.

-  ALOSL = Low fuel flow injector slope, lb/sec.

-  FN023(CRKPIP_CTR_BG) = Normalizing function for  number  of  PIP's  since

start of crank.

-  FN024(ACT) = Normalizing function for air charge temperature.

-  FN348A(ECT) = Crank fuel pulsewidth multiplier as a function of ECT.

-  FN348BP(BP) = Crank fuel pulsewidth multiplier as a function of BP.

-  FN348RPM(RPM) = Cranking pulsewidth enleanment as a function of RPM.

-  FN367(VBAT) = Injector offset as a function of battery voltage, msec.

-  FN387A(ECT) = Fuel pulsewidth multiplier as a function of ECT.

8-134

FUEL STRATEGY, FUEL PULSEWIDTH, BACKGROUND FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FN387_ALT(ECT) = Alternate FN387A used when ALT_CAL_FLG = 1.

-  FN1350 = Crank fuel pulsewidth

X = FN023(CRKPIP_CTR_BG)

Y = FN024(ACT)

-  FUEL_BKPT = The amount of fuel flow when transfer is made  from  the  low

slope to the high slope.

-  MINPW = Minimum pulsewidth for requested fuel.

-  NOMINAL_AFR = Nominal stoichiometric air/fuel ratio.

-  NUMCYL = Number of cylinders for this engine.

-  THBP_DECH = Throttle breakpoint for dechoke control, counts.

Non-Calibratable Constants:

-  [stcf] = Seconds-to-Ticks conversion factor.

8-135

FUEL STRATEGY, FUEL PULSEWIDTH, BACKGROUND FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_BG_CALC_COM18

Calculate  the  following  parameters  for  use  in   the   foreground   fuel

calculation.

always -------------------| AHISL_TICK := AHISL / [(1 + HICOMP) * [stcf]]

| ALOSL_TICK := ALOSL / [(1 + HICOMP) * [stcf]]

|            := AHISL_TICK * ALOSL / AHISL

| BKPT_PW := FUEL_BKPT / ALOSL_TICK

| FB_DIFF := (FUEL_BKPT / AHISL_TICK) - BKPT_PW

| MINPWT := MINPW * [stcf]

| MINPW_LBMF := MINPWT * ALOSL_TICK

CRKFLG = 1 ---------|

;in crank mode     |AND -| CRK_FUL := FN348A(ECT)

|     |           * FN348BP(BP)

TP_REL < THBP_DECH -|     |           * FN348RPM(N)

;below dechoke b/point   |           * FN1350(CRKPIP_CTR_BG,ACT)

|           * CRK_PIP_INJ

|

| LBMF_INJ1 := [(CRK_FUL * [stcf]) + FB_DIFF] *

|              AHISL_TICK

| LBMF_INJ2 := LBMF_INJ1

|

|  ;Obtain the injector ON duration, representing a

|  ;given cylinder's fuel flow requirement (units:

|  ;seconds/cycle) during engine start conditions.

|

| crank_fuel := CRK_FUL * (NUMCYL / CRK_PIP_INJ)

|

|  ;Convert crank_fuel to units of ticks/cycle and

|  ;assign its value into FUELPW during crank.

|  ;NOTE: The resulting value is clipped to one bit

|  ;less than the maximum value, since the maximum

|  ;value has a distinct meaning to the Foreground

|  ;Fuel strategy (ie. 'Injector was not fueled for

|  ;the last firing event for this cylinder').

|

| crank_fuel := min(FFFEh , (crank_fuel * [stcf]))

|

| FUELPW1 := crank_fuel

| FUELPW2 := crank_fuel

|

| --- ELSE ---

|

CRKFLG = 1 ---------------| (de-choke)

| CRK_FUL := 0

| LBMF_INJ1 := 0

| LBMF_INJ2 := 0

| FUELPW1 := 0

| FUELPW2 := 0

8-136

FUEL STRATEGY, FUEL PULSEWIDTH, BACKGROUND FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Note:  During EGO monitoring, intrusive fuel control actions are taken.

For the upstream EGO sensors monitoring requirements, a symmetric

modulation signal is superimposed over the normal closed loop

limit cycle.  The limit cycle is not altered with the exception

that the EGO signals are forced to switch at a programmed frequency.

The term LAM_MOD, shown in the logic below, carries the modulation

signal and is zero when not performing upstream EGO sensor monitoring.

unconditionally ----------| LAM_FINAL1 := LAMBSE1 + LAMBSE_MOD1

| LAM_FINAL2 := LAMBSE2 + LAMBSE_MOD2

ALT_CAL_FLG = 1-----------| fn387a := FN387_ALT(ECT)

|

| --- ELSE ---

|

| fn387a := FN387A(ECT)

DFSFLG = 1 ---------------| F_A_RATIO1 := 0

;In Decel Fuel Shutoff   | F_A_RATIO2 := 0

|

| --- ELSE ---

|

UNDSP = 1 ----------------| F_A_RATIO1 := (KAMRF1 * fn387a) /

;in underspeed mode      |               (NOMINAL_AFR * LAMBSE1)

| F_A_RATIO2 := (KAMRF2 * fn387a) /

|               (NOMINAL_AFR * LAMBSE2)

|  ;check for divide by zero

|

| --- ELSE ---

|

| F_A_RATIO1 := KAMRF1 /

|               (NOMINAL_AFR * LAM_FINAL1)

| F_A_RATIO2 := KAMRF2 /

|               (NOMINAL_AFR * LAM_FINAL2)

|  ;check for divide by zero

Calculate the fuel pulsewidth offset from zero  to  low  slope  intercept  in

milliseconds  (PWOFS_D)  and in ticks (PWOFF).  NOTE:  The intermediate units

conversion calculation is for documentation purposes.  It  is  not  meant  to

force  a  particular  implementation of the conversion from PWOFS_D to PWOFF,

except to ensure that the proper engineering units are maintained.

always -------------------| PWOFS_D := FN367(VBAT)

|

|  ;first convert PWOFF_D from msec to seconds

| pwoff_sec := PWOFF_D / 1000

|  ;then convert to ticks for PWOFF

| PWOFF := pwoff_sec * stcf

8-137

FUEL STRATEGY, FUEL PULSEWIDTH, BACKGROUND FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DUAL SLOPE INJECTOR CALIBRATION PARAMETERS

FUEL MASS VERSUS FUEL PULSEWIDTH DIAGRAM

FUEL (lbm/injection)

3.0e-5 |

|                                                 ..

|                                               ..

|                                             ..

|                                           ..

2.5e-5 |                                         ..

|                                       ..

|                                     ..|

|                                   ..  |

|                                 ..    |

|                               ..      | AHISL

|                             ..        |

|                           ..__________|

|                         ..

|                       ..

1.5e-5 |                     ..

| ----------------->.<---- FUEL_BKPT

|                  .

|                 .|

|                . |ALOSL

1.0e-5 |               .  |

|              .___|

|             .

|            .

|            |

|         (.)|

|        (.) |

|       (.)  |

|      (.)   |

|     (.)    |

0 .__________________________________________________________*

|  ^      |         |    |    |    |    |    |    | Pulsewidth

0.5 |     1.5       2.5       3.5       4.5       5.5  (msec)

|      |

|<---->|MINPW

|

FN367(VBAT)

NOTES:  FN367 (a.k.a. PWOFF) is added into the pulsewidth in the

foreground in module FUEL_INJ_OUT_COMx.

A calibration guide is available for determination of

ALOSL, AHISL, FN367, and FUEL_BKPT from injector flow data.

8-138

FUEL STRATEGY, FUEL PULSEWIDTH, BACKGROUND FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.18  FEATURE:  FUEL PUMP - V2.1_FPUMP (CDAN0)

8.18.1  FUEL PUMP SELECTION LOGIC (CDAN0)

OVERVIEW

This module determines which  set  of  control  and  diagnostic  strategy  is

executed  depending  upon  which  type  of  fuel pump is being utilized.  The

strategy supports single-speed (FP_TYPE = 0), and dual speed  (FP_TYPE  =  1)

fuel pump.

A single-speed fuel pump system is shown below.   The  EEC  sends  an  on/off

(100%/0% PWM) signal (via FPUMP_DC) to the fuel pump relay and reads the pump

secondary circuit monitor (F_PUMP_S_MON).

+----------+  F_PUMP_S_MON input

|          |<---------------------+

|  EEEC    |                      |          +------+

|          |----+pump             +----------|      |

+----------+    |(PWM output)                | fuel |

|                 +--------->| pump |

+---v---+             |          |      |

+12v  >--|-o  /o-|-------------+          +------+

+-------+

Fuel Pump

Relay

A dual-speed fuel pump system is  shown  below.   The  EEC  sends  an  on/off

(100%/0%  PWM)  signal  (via  FPUMP_DC) to the fuel pump relay and a high/low

signal (FPUMP_SPEED) to a solid  state  relay  which  routes  current  either

directly  to  the  fuel pump or through a dropping resistor.  The FPUMP_SPEED

command may also be sent via SCP for those applications using  a  VLCM.   The

EEC reads the pump secondary circuit monitor (F_PUMP_S_MON).

+----------+   F_PUMP_S_MON input

|          |<---------------------------------------------+

|  EEEC    |   FPL output                                 |

|          |---------------------------+Solid State       |

|          |----+pump                  |Relay             |

+----------+    |PWM output         +--v----+             |

|             +-----|-o---o-|-----+       |

+---v---+         |     +-------+     |    +--+---+

+12v  >--|-o  /o-|---------+                   |--->| fuel |

+-------+         |                   |    | pump |

Fuel Pump         +-----|/|/|/|/------+    +------+

Relay                  Dropping

Resistor

8-139

FUEL STRATEGY, FUEL DELIVERY SYSTEM, FUEL PUMP SELECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Calibration Constants:

-  FP_TYPE = Type of fuel pump:  0->ON/OFF, 1->2-speed,

PROCESS FLOW

+-------------+

FP_TYPE=0  |  fuel pump  |  FP_TYPE=1

+------------|  selection  |-------------+

|            +-------------+             |

|                                        |

+------v------+                          +------v------+

| pump on/off |                          | pump on/off |

|   control   |                          |   control   |

+------+------+                          +------+------+

|                                        |

|                                 +------v------+

|                                 | pump hi/lo  |

|                                 | speed cntrl |

|                                 +------+------+

|                                        |

+------v------+                          +------v------+

|  pump PWM   |                          |  pump PWM   |

|   output    |                          |   output    |

+------+------+                          +------+------+

|                                        |

|                                 +------v------+

|                                 | pump hi/lo  |

|                                 | spd output  |

|                                 +------+------+

|                                        |

+------v------+                          +------v------+

|   driver    |                          |   driver    |

| circuit test|                          | circuit test|

+------+------+                          +------+------+

|                                        |

+------v------+                          +------v------+

|  secondary  |                          |  secondary  |

| circuit test|                          | circuit test|

+------+------+                          +------+------+

|                                        |

|                                 +------v------+

|                                 | pump hi/lo  |

|                                 | circuit test|

|                                 +------+------+

|            +-------------+             |

+----------->|    done     |<------------+

+-------------+

8-140

FUEL STRATEGY, FUEL DELIVERY SYSTEM, FUEL PUMP SELECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FPUMP_SELECT_COM2

BEGIN: fuel_pump_control_selection

; This module is executed once per background loop.

; Determine which fuel pump strategy to execute:

FP_TYPE = 0 ---------------------| Do:  fp_on_off_control

; On/Off fuel pump               | Do:  fp_pwm_output

| Do:  fp_driver_test

| Do:  fp_secondary_test

|

| --- ELSE ---

|

FP_TYPE = 1 ---------------------| Do:  fp_on_off_control

; Dual speed fuel pump           | Do:  fp_hi_lo_control

| Do:  fp_pwm_output

| Do:  fp_hi_lo_output

| Do:  fp_driver_test

| Do:  fp_secondary_test

| Do:  fp_hi_lo_test

END: fuel_pump_control_selection

8-141

FUEL STRATEGY, FUEL DELIVERY SYSTEM, FUEL PUMP CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.18.2  FUEL PUMP CONTROL LOGIC (CDAK0)

OVERVIEW

This module controls the on/off control of a single or dual speed  fuel  pump

(FP_TYPE  =  0  or  1).   The output is specified as a duty cycle to make the

output compatible with strategies that are capable  of  modulating  the  fuel

pump  output.   Also,  this  utilizes  the parallel control to the SSPOD/UPOD

driver, thereby eliminating unnecessary serial communications overhead.

The output duty cycles required to turn the fuel pump  full  ON  or  OFF  are

specified by the scalars PUMP_ON_DC and PUMP_OFF_DC, respectively.

DEFINITIONS

INPUTS

Registers:

-  FPMP_ON_TMR = Timer to limit the on time during engine off testing.

-  TSLPIP = Teime since last PIP low-to-high transition, msec.

Bit Flags:

-  FLG_THFT_FCO = Flag signalling when fuel is being cut  out  due  to  anti

theft system action; 1 -> cutting out fuel.

-  OSM_EO_OFF = OFF state requested for outputs during engine off  on-demand

test

-  OSM_EO_ON = ON state requested for outputs during  engine  off  on-demand

test

Calibration Constants:

-  FPMP_ON_TM = Maximum time for fuel  pump  to  be  on  during  engine  off

on-demand test.

-  PUMP_OFF_DC = Output duty cycle required to command fuel pump full OFF.

-  PUMP_ON_DC = Output duty cycle required to command fuel pump full ON.

OUTPUTS

Registers:

-  FPMP_ON_TMR = See above.

-  FPUMP_DC = Desired fuel pump output duty cycle.

8-142

FUEL STRATEGY, FUEL DELIVERY SYSTEM, FUEL PUMP CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FPUMP_CTRL_COM1

BEGIN: fp_on_off_control

; This process is executed only when called.

OSM_EO_ON = 1 -------------------|

; On demand test - pump ON       |

; state requested                |AND -| fpump_dc := PUMP_ON_DC

|     | ; Command pump full ON

FPMP_ON_TMR < FPMP_ON_TM --------|     |

; Time limit in ON state               |

; not yet expired                      | --- ELSE ---

|

OSM_EO_ON = 1 -------------------------| fpump_dc := PUMP_OFF_DC

; On demand test - pump ON state       | ; Command pump full OFF

; requested.  Time limit expired.      |

|

| --- ELSE ---

OSM_EO_OFF = 1 ------------------|     |

; On demand test - pump OFF      |     |

; state requested.               |     |

|     |

TSLPIP >= 1000 ms ---------------|OR --| fpump_dc := PUMP_OFF_DC

; Last PIP more than             |     |

; 1 second ago                   |     | FPMP_ON_TMR := 0

|     | ; Clear pump ON timer

FLG_THFT_FCO = 1 ----------------|     |

;Theft condition exists                |

| --- ELSE ---

|

| fpump_dc := PUMP_ON_DC

|

unconditionally-------------------------| Do: substitute(19,fpump_dc)

| FPUMP_DC := fpump_dc

END: fp_on_off_control

8-143

FUEL STRATEGY, FUEL DELIVERY SYSTEM, TWO-SPEED FUEL PUMP CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.18.3  TWO-SPEED FUEL PUMP CONTROL LOGIC (CDAJ0)

OVERVIEW

This module is designed to determine the high/low speed for a two speed  fuel

pump.    Note  that  the  on/off  control  is  performed  by  another  module

(FPUMP_COMn).

For the two speed pump, the speed is selected by a Solid  State  Relay  (SSR)

which  either routes Vbat directly to the pump or through a dropping resistor

which results in a lower pump speed.

+----------+   FPL output

|          |---------------------------+

|  EEEC    |                           |Solid State

|          |----+pump                  |Relay

+----------+    |PWM output         +--v----+

|             +-----|-o---o-|-------+

+---v---+         |     +-------+       |

+12v  >--|-o  /o-|---------+                     +----> To fuel

+-------+         |                     |       pump

Fuel Pump         +-----|/|/|/|/--------+

Relay                  Dropping

Resistor

DEFINITIONS

INPUTS

Registers:

-  AM = Air Mass flow, lb/min.

-  N = Engine RPM.

-  VBAT = Battery voltage, VDC.

-  WRMEGO = EGO sensor should be warm flag; 1 -> EGO is warm.

Bit Flags:

-  FP_HI_ENABL1 = Hi fuel-pump speed enable flag for  AM  >  a  function  of

VBAT; 1 -> enable hi speed.

-  FP_HI_ENABL2 = Hi fuel-pump speed  enable  flag  due  to  sufficient  RPM

regardless; 1 -> enable hi speed.

8-144

FUEL STRATEGY, FUEL DELIVERY SYSTEM, TWO-SPEED FUEL PUMP CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  FNFP_VBAT = AM hi-speed fuel-pump enabling function, lb/min.

-  FP_N_ENA_SH = RPM above which hi-speed fuel pump  is  enabled  (set  FF),

RPM.

-  FP_N_ENA_CL = RPM below which hi-speed fuel pump is disabled (clear  FF),

RPM.

-  FP_AM_HYS = Hysteresis used with FNFP_VBAT to avoid flickering fuel  pump

states, lb/min.

OUTPUTS

Bit Flags:

-  FP_HI_ENABL1 = See above.

-  FP_HI_ENABL2 = See above.

-  FPUMP_SPEED = Desired pump speed; 0 -> low speed, 1 -> high speed.

8-145

FUEL STRATEGY, FUEL DELIVERY SYSTEM, TWO-SPEED FUEL PUMP CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FPUMP_2SPD_CTRL_COM1

BEGIN: fp_hi_lo_control

; This process is executed only when called.

AM >= FNFP_VBAT(VBAT) -----------|S Q -| FP_HI_ENABL1

|     | ; AM high enough with respect to

AM < FNFP_VBAT(VBAT)- FP_AM_HYS -|C    | ; VBAT to require high pump speed

N >= FP_N_ENA_SH -----------------|S Q -| FP_HI_ENABL2

|     | ; RPM high enough to use

N < FP_N_ENA_CL ------------------|C    | ; require high pump speed

FP_HI_ENABL1 = 1 -----------------|

|

FP_HI_ENABL2 = 1 -----------------|OR --| fpump_speed := 1

|     | ; Command high speed

WRMEGO = 0 -----------------------|     |

|

| --- ELSE ---

|

| fpump_speed := 0

| ; Command low speed

unconditionally ------------------------| Do: substitute(20,fpump_speed)

| FPUMP_SPEED := fpump_speed

END: fp_hi_lo_control

8-146

FUEL STRATEGY, FUEL DELIVERY SYSTEM, FUEL PUMP OUTPUT CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.18.4  FUEL PUMP OUTPUT CONTROL STRATEGY (CDAL0)

OVERVIEW

This module controls the output for the fuel pump when it is driven by a DARC

PWM  output.   The desired signal period and pulsewidth are determined from a

specified duty cycle (FPUMP_DC) and frequency (FPUMP_FREQ) and are written to

memory-mapped  DARC  PWM  registers.  The DARC PWM signal is driven by one of

the  "smart"  drivers  --  typically  a  UPOD,  which  returns  driver  fault

information  via  the  SPI.   See  the  Hardware  Interface  chapter for more

information.  If the fuel pump is driven by a power amplifier rather  than  a

relay, the signal must be inverted.

DEFINITIONS

INPUTS

Registers:

-  FPUMP_DC = Fuel pump output duty cycle.

Calibration Constants:

-  FP_DRIVER = Fuel pump driver type; 0->relay, 1->power amp.

-  FPUMP_FREQ = Output frequency for fuel pump signal.

OUTPUTS

Registers:

-  dtscf = DARC ticks to seconds conversion factor.  Not displayable.

-  fpump_period = Output signal period for fuel pump.  Not displayable.

-  fpump_pw = Output signal pulsewidth for fuel pump.  Not displayable.

8-147

FUEL STRATEGY, FUEL DELIVERY SYSTEM, FUEL PUMP OUTPUT CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FPUMP_PWM_OUTPUT_COM1

BEGIN: fp_pwm_output

; This process is executed only when called.

FP_DRIVER = 0 ---------------| fpump_period :=

; Relay driver               |  dtscf / FPUMP_FREQ

| ; Convert pump frequency

| ; to a period in DARC ticks.

|

| fpump_pw :=

|  FPUMP_DC * fpump_period

| ; Calculate pump pulsewidth

|

| --- ELSE ---

|

| fpump_period :=

|  dtscf / FPUMP_FREQ

| ; Convert pump frequency

| ; to a period in DARC ticks.

|

| fpump_pw :=

|  (1 - FPUMP_DC) * fpump_period

| ; Calculate pump pulsewidth

| ; and invert for power amp

| ; driver.

unconditionally -------------| Output fpump_period and fpump_pw

| to appropriate DARC PWM registers.

END: fp_pwm_output

8-148

FUEL STRATEGY, FUEL DELIVERY SYSTEM, FUEL PUMP SPEED OUTPUT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.18.5  FUEL PUMP SPEED OUTPUT (CDAL0)

OVERVIEW

This module controls dual speed  fuel  pump  (FP_TYPE  =  1)  high/low  speed

output.  The desired fuel pump speed is output to a dedicated EEEC output.

The Fuel Pump Low (FPL) output controls a normally closed solid  state  relay

which is connected in parallel to a dropping resistor as shown below.

+----------+   FPL output

|          |---------------------------+

|  EEEC    |                           |Solid State

|          |----+pump                  |Relay

+----------+    |PWM output         +--v----+

|             +-----|-o---o-|-------+

+---v---+         |     +-------+       |

+12v  >--|-o  /o-|---------+                     +----> To fuel

+-------+         |                     |       pump

Fuel Pump         +-----|/|/|/|/--------+

Relay                  Dropping

Resistor

DEFINITIONS

Bit Flags:

-  FPL = Fuel Pump Low commanded output; 0->High speed, 1->Low speed.

-  FPUMP_SPEED = 2-speed fuel pump desired speed; 1 -> high speed.

PROCESS

STRATEGY MODULE: FPUMP_SPEED_OUTPUT_COM1

BEGIN: fp_hi_lo_output

; This process is executed only when called.

FPUMP_SPEED = 0 ------------------------| FPL := 1

; Low speed fuel pump requested         | ; Energize normally closed

| ; relay.  Route current

| ; through resistor.

|

| --- ELSE ---

|

| FPL := 0

| ; Close relay.  Bypass

| ; dropping resistor.

END: fp_hi_lo_output

8-149

FUEL STRATEGY, FUEL DELIVERY SYSTEM, FUEL PUMP PRIMARY CIRCUIT TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.18.6  FUEL PUMP PRIMARY CIRCUIT TEST (CDAM0)

OVERVIEW

This module checks the output driver  status  returned  from  the  UPOD/SSPOD

driver via the SPI.  If a fault is present for a calibratible amount of time,

the appropriate malfunction code is set.

NOTE, for fuel pump output on an SSPOD:

This driver circuit test can only detect an open circuit or a short to ground

while  the pump is requested OFF.  If the pump is requested ON, all this test

can detect is a short to VBAT.

Since a failed fuel pump will result in a no-start, the ability to  test  the

fuel  pump  upon  power-up  can  be calibrated in by setting FP_PU_DT_SW = 1.

This allows testing whenever the key is ON,  prior  to  any  CCM  test  being

enabled.  Should that feature be calibrated out, the KOEO and OTM diagnostics

can be calibrated in by setting FP_EO_DT_SW and FP_OT_DT_SW.  These  switches

allow  the  OSM_EO_ON and OSM_EO_OFF tests to be calibrated out individually,

which Mazda badge vehicles may want to do.

DEFINITIONS

Registers:

-  PUTMR = Time after CPU power up.

-  V_FP_ER_TMR1 = Fuel  pump  monitor  timer  for  primary  circuit  failure

(P0230).

Bit Flags:

-  CCM_ER_ENA = Engine running on demand test for CCM enabled.

-  CCM_TST_ENA = OBD-II Comprehensive Component Test  enable  flag.   (1  =>

test enabled)

-  F_PUMP_ERROR = Fuel pump driver circuit error status;  1->driver  circuit

fault.

-  OBDII_RESET = Flag used to simulate the receipt of an  OBD-II  scan  tool

reset message.

-  OSM_EO_OFF = OFF state requested for OSM outputs  during  engine  off  on

demand test.

-  OSM_EO_ON = ON state requested for  OSM  outputs  during  engine  off  on

demand test.

Calibration Constants:

8-150

FUEL STRATEGY, FUEL DELIVERY SYSTEM, FUEL PUMP PRIMARY CIRCUIT TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FP_EO_DT_SW = Switch to enable FP primary circuit test during KOEO.

-  FP_ER_DT_SW = Switch to enable FP primary circuit test during KOER.

-  FP_OT_DT_SW = Switch to enable FP primary circuit test when  OSM_EO_ON  =

1.

-  FP_PU_DT_SW = Switch to enable FP primary circuit test on power-up.

-  FP_PU_TM = EEC stabilization time before testing fuel pump.

-  FPMP_OSM_ENA = Switch to enable fuel pump primary circuit test.

-  PUMP_OFF_DC = Duty cycle to command to turn off the fuel pump.

-  V_FP_ER_TM1 =  Time  delay  before  setting  fuel  pump  primary  circuit

malfunction flag (P0230).

OTHER

-  P0230 = Fuel pump primary circuit failure.

8-151

FUEL STRATEGY, FUEL DELIVERY SYSTEM, FUEL PUMP PRIMARY CIRCUIT TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FPUMP_DRVR_TST_COM1

BEGIN: fp_driver_test

; This process is executed only when called.

; Check if all conditions are met to test fuel pump driver circuit.

OSM_EO_ON = 1 -----------------------|

;KOEO or OTM requests pump ON       |AND -| fp_dt_ena := 0

|     |  ;Disable the test

FP_OT_DT_SW = 0 ---------------------|     |

;Disallow test with pump ON,              |

;engine OFF                               | --- ELSE ---

|

OSM_EO_ON = 1 -----------------------|     |

;KOEO or OTM requests pump ON       |     |

|     |

FP_PU_DT_SW = 1 ---------------|     |     |

;Test enabled                 |     |     |

;on power-up                  |     |     |

|     |     |

PUTMR > FP_PU_TM --------------|AND -|     |

;EEC powered up long          |     |     |

;enough to stabilize          |     |     |

|     |     |

CCM_TST_ENA = 0 ---------------|     |     |

;Continuous test not running        |     |

|     |

OSM_EO_OFF = 1 ----------------|     |     |

;KOEO or OTM requests         |     |     |

;pump OFF                     |AND -|     |

|     |     |

FP_EO_DT_SW = 1 ---------------|     |     |

;FP primary test                    |OR --| fp_dt_ena := 1

;enabled in KOEO                    |     |  ;Enable the diagnostic test

|     |

CCM_ER_ENA = 1 ----------------|     |     |

;KOER test in prog            |AND -|     |

|     |     |

FP_ER_DT_SW = 1 ---------------|     |     |

;FP primary test                    |     |

;enabled in KOER                    |     |

|     |

CCM_TST_ENA = 1 ---------------|     |     |

; CCM test in prog             |AND -|     |

|           |

FPMP_OSM_ENA = 1 --------------|           |

; FP primary circuit                       |

; test is enabled                          |

| --- ELSE ---

|

| fp_dt_ena := 0

|  ;Disable the test

|

8-152

FUEL STRATEGY, FUEL DELIVERY SYSTEM, FUEL PUMP PRIMARY CIRCUIT TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

fp_dt_ena = 1 -----------------------|

|AND -| Do: fp_test_driver_circuit

OBDII_RESET = 0 ---------------------|     |

|

| --- ELSE ---

|

| V_FP_ER_TMR1 := 0

| ; Clear primary circuit

| ; test timer

|

| clear_malf(P0230)

| ; Erase code

END: fp_driver_test

BEGIN: fp_test_driver_circuit

; This process is executed only when called.

F_PUMP_ERROR = 0 ------------------------| V_FP_ER_TMR1 := 0

; Driver circuit okay                    | ; Clear driver circuit

| ; error timer.

|

| Do: clear_malf(P0230)

|

| --- ELSE ---

F_PUMP_ERROR = 1 ------------------|     |

; Driver circuit fault exists      |AND -| Do: store_code(P0230)

|     |

V_FP_ER_TMR1 > V_FP_ER_TM1 --------|     |

; Sufficient time with fault             | --- ELSE ---

|

| Allow V_FP_ER_TMR1 to increment.

END: fp_test_driver_circuit

8-153

FUEL STRATEGY, FUEL DELIVERY SYSTEM, FUEL PUMP SECONDARY CIRCUIT TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.18.7  FUEL PUMP SECONDARY CIRCUIT TEST (CDAM0)

OVERVIEW

This module tests the fuel pump secondary circuit  monitor.   In  a  properly

functioning   system,   the   fuel  pump  secondary  circuit  monitor  input,

F_PUMP_S_MON, is HIGH when the  fuel  pump  is  ON.   If  the  fuel  pump  is

commanded  on  and  F_PUMP_S_MON remains low for a sufficient period of time,

code P0231 is set.  This indicates a break in the line  between  the  battery

and  the  F_PUMP_S_MON  input  to  the ECA or no contact inside the fuel pump

relay.

Conversely, F_PUMP_S_MON should be LOW when the fuel pump  is  OFF.   If  the

fuel  pump  is commanded off and F_PUMP_S_MON is high for a sufficient period

of time, code P0232 is set.  This indicates that either the fuel  pump  relay

is stuck closed or there is an open circuit between the ECA and the fuel pump

ground allowing the ECA pull-up circuit to hold F_PUMP_S_MON high.

Since a failed fuel pump will result in a no-start, the ability to  test  the

fuel  pump  upon  power-up  can be calibrated in.  This allows testing when a

start is attempted, prior to any CCM test being enabled.

A Mazda requirement is that the test be disabled during KOEO or  OTM  if  the

pump  is  requested  ON.  This is accomplished by setting FP_OT_ST_SW = 0 and

FP_EO_ST_SW = 1.

To run the diagnostics any time the pump is on with the engine off, calibrate

FP_PU_ST_SW  = 1.  Mazda, as well as NAAO, may calibrate this test in, as the

test described in the previous paragraph takes precedence.

DEFINITIONS

Registers:

-  FPUMP_DC = Modulated Fuel Pump Duty cycle output.

-  PUTMR = Time after CPU power up.

-  V_FP_ER_TMR2 = Fuel pump monitor timer for P0231MALF.

-  V_FP_ER_TMR3 = Fuel pump monitor timer for P0232MALF.

-  VBAT = Battery voltage, volts.

Bit Flags:

-  CCM_ER_ENA = Engine running on demand test for CCM enabled.

-  CCM_TST_ENA = OBD-II Comprehensive Component Test  enable  flag.   (1  =>

test enabled)

-  F_PUMP_ERROR = Fuel pump driver circuit error status; 1 -> driver circuit

fault.

8-154

FUEL STRATEGY, FUEL DELIVERY SYSTEM, FUEL PUMP SECONDARY CIRCUIT TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  F_PUMP_S_MON = Fuel pump secondary monitor; 0 = pump off, 1 = pump on.

-  OBDII_RESET = Flag used to simulate the receipt of an  OBD-II  scan  tool

reset message.

-  OSM_EO_OFF = OFF state requested for OSM outputs  during  engine  off  on

demand test.

-  OSM_EO_ON = ON state requested for  OSM  outputs  during  engine  off  on

demand test.

Calibration Constants:

-  FP_EO_ST_SW = Switch to enable FP secondary circuit test during KOEO.

-  FP_ER_ST_SW = Switch to enable FP secondary circuit test during KOER.

-  FP_OT_ST_SW = Switch to enable FP secondary circuit test when OSM_EO_ON =

1.

-  FP_PU_ST_SW = Switch to enable FP secondary circuit test on power-up.

-  FP_PU_TM = EEC stabilization time before testing fuel pump.

-  FPMP_SMON_EN = Switch to enable fuel pump secondary circuit test.

-  PUMP_OFF_DC = Output duty cycle required to command fuel pump full OFF.

-  PUMP_ON_DC = Output duty cycle required to command fuel pump full ON.

-  VBAT_CCM_MIN = Minimum voltage to perform circuit monitor test.

-  V_FP_ER_TM2 = Time delay before setting P0231MALF.

-  V_FP_ER_TM3 = Time delay before setting P0232MALF.

OTHER

-  P0231 = Fuel pump secondary circuit monitor LOW with pump commanded ON.

-  P0232 = Fuel pump secondary circuit monitor HIGH with pump commanded OFF.

8-155

FUEL STRATEGY, FUEL DELIVERY SYSTEM, FUEL PUMP SECONDARY CIRCUIT TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FPUMP_SMON_TST_COM1

BEGIN: fp_secondary_test

; This process is executed only when called.

; Check if all conditions are met to test fuel pump secondary circuit.

OSM_EO_ON = 1 -----------------------|

;KOEO or OTM requests pump ON       |AND -| fp_st_ena := 0

|     |  ;Disable the test

FP_OT_ST_SW = 0 ---------------------|     |

;Disallow test with                       |

;pump on, engine off                      | --- ELSE ---

|

OSM_EO_ON = 1 -----------------------|     |

;KOEO or OTM requests pump ON       |     |

|     |

FP_PU_ST_SW = 1 ---------------|     |     |

;Test enabled                 |     |     |

;on power-up                  |     |     |

|     |     |

PUTMR > FP_PU_TM --------------|AND -|     |

;EEC powered up long          |     |     |

;enough to stabilize          |     |     |

|     |     |

CCM_TST_ENA = 0 ---------------|     |     |

;Continuous test not running        |     |

|     |

OSM_EO_OFF = 1 ----------------|     |OR --| fp_st_ena := 1

;KOEO or OTM requests         |     |     |  ;Enable the diagnostic test

;pump OFF                     |AND -|     |

|     |     |

FP_EO_ST_SW = 1 ---------------|     |     |

;FP secondary test                  |     |

;enabled in KOEO and OTM            |     |

|     |

CCM_ER_ENA = 1 ----------------|     |     |

;KOER test in prog            |AND -|     |

|     |     |

FP_ER_ST_SW = 1 ---------------|     |     |

;FP secondary test                  |     |

;enabled in KOER                    |     |

|     |

CCM_TST_ENA = 1 ---------------|     |     |

; CCM test in prog             |AND -|     |

|           |

FPMP_SMON_EN = 1 --------------|           |

; FP Secondary circuit                     |

; test present                             |

| --- ELSE ---

|

| fp_st_ena := 0

|  ;Disable the test

|

8-156

FUEL STRATEGY, FUEL DELIVERY SYSTEM, FUEL PUMP SECONDARY CIRCUIT TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

fp_st_ena = 1 -----------------------|

|

VBAT > VBAT_CCM_MIN -----------------|

; Battery voltage stable             |AND -| Do: fp_test_secondary_circuit

|     |

OBDII_RESET = 0 ---------------------|     |

|     |

F_PUMP_ERROR = 0 --------------------|     |

;Driver circuit OK                        |

|

| --- ELSE ---

|

| clear_malf(P0231)

| clear_malf(P0232)

| ; Clear pump codes

|

| V_FP_ER_TMR2 := 0

| V_FP_ER_TMR3 := 0

| ; Clear test timers

END: fp_secondary_test

8-157

FUEL STRATEGY, FUEL DELIVERY SYSTEM, FUEL PUMP SECONDARY CIRCUIT TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: fp_test_secondary_circuit

; This process is executed only when called.

FPUMP_DC = PUMP_ON_DC ------------|

; Fuel pump ON                    |

|

F_PUMP_S_MON = 0 -----------------|AND -| Do: store_code(P0231)

; Secondary circuit monitor       |     | ; Indicate pump ON fault

; indicates pump OFF              |     |

|     | Do: clear_malf(P0232)

V_FP_ER_TMR2 > V_FP_ER_TM2 -------|     | ; Erase any pump OFF fault

; Sufficient time with fault            |

| V_FP_ER_TMR3 := 0

| ; Clear pump OFF test timer

|

| --- ELSE ---

FPUMP_DC = PUMP_ON_DC ------------|     |

; Fuel pump ON                    |AND -| Do: clear_malf(P0232)

|     |

F_PUMP_S_MON = 0 -----------------|     | V_FP_ER_TMR3 := 0

; Secondary circuit monitor             | ; Clear pump OFF test timer

; indicates pump OFF                    |

| --- ELSE ---

FPUMP_DC = PUMP_OFF_DC -----------|     |

; Fuel pump OFF                   |     |

|     |

F_PUMP_S_MON = 1 -----------------|AND -| Do: store_code(P0232)

; Secondary circuit monitor       |     | ; Indicate pump OFF fault.

; indicates pump ON               |     |

|     | Do: clear_malf(P0231)

V_FP_ER_TMR3 > V_FP_ER_TM3 -------|     | ; Clear any pump ON fault.

; Sufficient time with fault            |

| V_FP_ER_TMR2 := 0

| ; Clear pump ON test timer

|

| --- ELSE ---

FPUMP_DC = PUMP_OFF_DC -----------|     |

; Fuel pump OFF                   |AND -| Do: clear_malf(P0231)

|     |

F_PUMP_S_MON = 1 -----------------|     | V_FP_ER_TMR2 := 0

; Secondary circuit monitor             |

; indicates pump ON                     | --- ELSE ---

|

| V_FP_ER_TMR2 := 0

| Do: clear_malf(P0231)

|

| V_FP_ER_TMR3 := 0

| Do: clear_malf(P0232)

END: fp_test_secondary_circuit

8-158

FUEL STRATEGY, FUEL DELIVERY SYSTEM, FUEL PUMP SPEED CIRCUIT TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.18.8  FUEL PUMP SPEED CIRCUIT TEST (CDAM0)

OVERVIEW

This module checks the fuel  pump  speed  driver  status  returned  from  the

UPOD/SSPOD  driver  via  the  SPI.   If a fault is present for a calibratible

amount of time, the appropriate malfunction code is set.

DEFINITIONS

Registers:

-  PUTMR = Time after CPU power up.

-  V_FP_ER_TMR4 = Fuel pump speed monitor timer for primary circuit  failure

(P1232).

Bit Flags:

-  CCM_EO_ENA = Engine off on demand test for CCM enabled.

-  CCM_ER_ENA = Engine running on demand test for CCM enabled.

-  CCM_TST_ENA = OBD-II Comprehensive Component Test  enable  flag.   (1  =>

test enabled)

-  FPMP_OSM_ENA = Switch to enable Fuel Pump OSM test.

-  FP_2SPD_ERR = Fuel pump high-speed driver circuit fault flag.

-  OBDII_RESET = Flag used to simulate the receipt of an  OBD-II  scan  tool

reset message.

Calibration Constants:

-  FP_EO_HL_SW = Switch to enable FP hi-speed primary  circuit  test  during

KOEO.

-  FP_ER_HL_SW = Switch to enable FP hi-speed primary  circuit  test  during

KOER.

-  V_FP_ER_TM4 = Time delay before setting fuel pump speed  primary  circuit

malfunction (P1232)

OTHER

-  P1232 = Fuel pump speed primary circuit malfunction.

8-159

FUEL STRATEGY, FUEL DELIVERY SYSTEM, FUEL PUMP SPEED CIRCUIT TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FPUMP_HI_LO_TST_COM1

BEGIN: fp_hi_lo_test

; This process is executed only when called.

CCM_EO_ENA = 1 -------|

;KOEO test in prog   |AND -|

|     |

FP_EO_HL_SW = 1 ------|     |

;FP hi-lo test             |

;enabled in KOEO           |OR --|

|     |

CCM_ER_ENA = 1 -------|     |     |

;KOER test in prog   |AND -|     |

|     |     |

FP_ER_HL_SW = 1 ------|     |     |

;FP hi-lo test             |     |

;enabled in KOER           |     |

|     |

CCM_TST_ENA = 1 ------|     |     |AND -| No action

; CCM test in prog    |AND -|     |     |  ;Allow V_FP_ER_TMR4 to

|           |     |  ;increment as a free-running

FPMP_OSM_ENA = 1 -----|           |     |  ;timer.

; FP hi-lo test                   |     |

; present                         |     |

|     |

OBDII_RESET = 0 ------------------|     |

|     |

FP_2SPD_ERR = 1 ------------------|     |

;primary hi speed monitor              |

;indicates error                       |

| --- ELSE ---

|

| V_FP_ER_TMR4 := 0

|  ;no errors

V_FP_ER_TMR4 > V_FP_ER_TM4 -------------| store_code(P1232)

;fault present long enough             |  ;pump speed primary circuit

|  ;failure.

|

| --- ELSE ---

|

| Do: clear_malf(P1232)

END: fp_hi_lo_test

8-160

FUEL STRATEGY, PPCTR CONTROL - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.19  PPCTR CONTROL (CDAA0)

OVERVIEW

This counter counts PIPs when not in Decel Fuel Shut-Off.  PPCTR  is  updated

at PIP rising edge before injector pulsewidth is calculated and output.

DEFINITIONS

INPUTS

Bit Flags:

-  DFSFLG = Decel Fuel Shut-Off flag; 1 -> the decel fuel multiplier is  not

one.

Calibration Constants:

-  PIPNUM = Number of PIPs to remain in Open Loop fuel after DFSO.  Prevents

LAMBSE  from ramping off rich due to normal transport delay time.  Set to

1 to calibrate out.

OUTPUTS

Registers:

-  PPCTR  =  PIP  counter;  updated  at  PIP  rising  edge  before  injector

pulsewidth is calculated and output.

PROCESS

STRATEGY MODULE: FUEL_PPCTR_COM1

DFSFLG = 0 ----------------------| Increment PPCTR every PIP

|  (Clip at PIPNUM)

|

| --- ELSE ---

|

| PPCTR = 0

8-161

FUEL STRATEGY, RPM LIMITER LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.20  RPM LIMITER LOGIC (CDAN0)

OVERVIEW

The RPM limiter logic turns off the  fuel  if  the  engine  RPM  exceeds  its

"red-line"  limit.   Note  that the parameter DT12SH represents the PIP delta

time value which exceeds the specified range  of  the  byte-length  parameter

DT12S.   For  this  reason,  if the parameter DT12SH contains a value greater

than zero, the speed limiting logic need not cut out fuel - since  the  PIP's

are much farther apart than DT12S (ie.  slow engine speed).

DEFINITIONS

Registers:

-  DT12S = The period of time between two adjacent rising edges of PIP.

-  DT12SH = Hi byte of DT12S - stores overflow of PIP delta time.

-  DT23S = Last DT12S time.

-  PIPLMT_HIG = Stage 3 rev limit in use.

-  PIPLMT_LOW = Stage 2 rev limit in use.

Bit Flags:

-  FOFFLG2 = Stage 2 alternate fire VS exceeded.

-  FOFFLG3  =  Stage  2  alternate  fire  TP  exceeded  (engine   oil   temp

protection).

-  FUEL_CUT = Fuel cutout active.  Set in  foreground  for  continuous  self

test

-  HALF_FUEL = Flag which indicates one-half of the  injections  are  to  be

cut-out; 1 -> half fuel.

-  MLUS_HF_FLG = Converter Clutch Failed flag; 1 => Enter Half Fuel.

-  NLMT_FLG2 = Stage 3 no fuel VS exceeded.

-  NO_FUEL = Flag which indicates that all injections are to be  cut-out;  1

-> no fuel.

-  OFM_FMEM =  ETV  overcurrent  based  alternate  fire  request  flag;  1->

Alternate fire requested, 0 -> Normal fire.

-  PIP_ST2_FLG = Flag indicating Stage 2 rev limiting required  due  to  PIP

condition.

Calibration Constants:

-  MNPIPn = Minimum PIP period, clock ticks; n = 4  (4-cylinder  engine),  6

(6-cylinder engine), or 8 (8-cylinder engine).

8-162

FUEL STRATEGY, RPM LIMITER LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PIPLIMHYT = Rev limiter hystersis, clock ticks.

8-163

FUEL STRATEGY, RPM LIMITER LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_RPM_LIM_FG_COM4

BAD PIP DETERMINATION (done in PIP_DATA on PIP_HIGH)

dt12s < MNPIPn -------------------------| Exit PIP_DATA routine

(RPM higher than PIP noise filter)     |  (ignore this PIP)

|

| --- ELSE ---

|

| DT23S = DT12S

| DT12S = Low word (dt12s)

| DT12SH = Hi byte (dt12s)

DT12SH <> 0 ----------------------------| HALF_FUEL = 0

;PIP time very long                    | NO_FUEL = 0

| EXIT this module.

DT12S < PIPLMT_LOW ---------|

|AND -|S Q -| PIP_ST2_FLG

DT23S < PIPLMT_LOW ---------|     |

|

DT12S > PIPLMT_LOW +              |

PIPLIMHYT ---------|     |

|AND -|C

DT23S > PIPLMT_LOW +        |

PIPLIMHYT ---------|

DT12S < PIPLMT_HIG ---------|

|AND -|

DT23S < PIPLMT_HIG ---------|     |OR --| HALF_FUEL = 0

|     | NO_FUEL = 1

NLMT_FLG2 = 1 --------------------|     | FUEL_CUT = 1

|

| --- ELSE ---

PIP_ST2_FLG = 1 ------------------|     |

|     |

FOFFLG2 = 1 ----------------------|     |

|     |

FOFFLG3 = 1 ----------------------|OR --| HALF_FUEL = 1

|     | NO_FUEL = 0

MLUS_HF_FLG = 1 ------------------|     | FUEL_CUT = 1

|     |

OFM_FMEM = 1 ---------------------|     |

| --- ELSE ---

|

| HALF_FUEL = 0

| NO_FUEL = 0

The flag, FUEL_CUT, is cleared in Background by the continuous selftest

routine.

8-164

FUEL STRATEGY, DIS INJECTOR SYNCHRONIZATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.21  DIS INJECTOR SYNCHRONIZATION (CDAK0)

OVERVIEW

The mechanism used for fuel  synchronization  is  the  Camshaft  I.D.   (CID)

sensor for non VCT engine and CID1 and CID2 for engine with VCT.  Engine with

variable cam logic and hardware will bypass the  FUEL_CID_SYNC_COM1  and  the

FUEL_PIP_INT_SYNC_COM2  modules.   The  CIDn,  VCT and SYNC_CTR positions are

determined in the VCT_CID_SYNC_COM1 and the VCT_INPUT_COM1  modules.   VCAMHP

is used to determine when VCT hardware and software is used.

VCAMHP is set to zero for engine without VCT hardware or logic.   The  signal

from  the  CID  sensor  is  used  together  with  the  PIP signal to locate a

particular cylinder (generally #1).  The CID signal indicates  the  cycle  of

the  cylinder  while  the  PIP signal indicates the crank position.  Once the

first cylinder is located, a register called the SYNC_CTR is used to indicate

the position and cycle of the remaining cylinders.

HP_CIDSEL is the calibration constant set by the calibrator to indicate which

type  of  CID hardware is present, Hall Effect sensing or Variable Reluctance

sensing (Hall = 0 and VRS = 1).  Figure 1 shows the  CID  signal  of  a  Hall

system.   Figure  2 shows the CID signal of a VRS system after the raw signal

(a single sinusoidal pulse every two crank revolutions) is  processed.   When

Hall  CID  hardware is present the signal has a 50 percent duty cycle and the

low-to-high transition is positioned at a nominal 26 degrees (crank) ATDC for

cylinder #1.  With VRS CID hardware present the high-to-low transition of the

signal is positioned at a nominal 22.5 degrees (crank) ATDC for cylinder #1.

For a Hall system, the CID signal is processed when either a  low-to-high  or

high-to-low  transition  is  recognized.   For  a  VRS system processing only

occurs after recognition of a high-to-low transition.   Regardless  of  which

system is employed, processing involves synchronizing the SYNC_CTR to the CID

signal.

Figure 1 shows how synchronization is accomplished when Hall sensing hardware

is  present.   The  SYNC_CTR is incremented by the low-to-high transition (up

edge) of the PIP signal.  To accomplish synchronization, the SYNC_CTR is  set

to  0  when the up edge of the CID signal occurs or it is set to ENGCYL (3 in

this case) when the down edge (high-to-low transition) occurs.  The  SYNC_CTR

is always reset to 0 when the up edge of the CID signal is recognized.

Figure 2 shows that synchronization of the SYNC_CTR occurs similarly when VRS

hardware  is  present.   The difference is that the VRS CID signal allows the

SYNC_CTR to be synchronized at only one point (the reset point) in the engine

cycle.

The CID_PROCESSING  strategy  is  designed  such  that  if  the  SYNC_CTR  is

synchronized  and  the  engine is in the run mode, the request to synchronize

fuel with PIP is issued.

If the CID signal is lost, the value of SYNC_CTR  will  become  greater  than

NUMCYL after the engine cycle (720 degrees crank) is completed.  The strategy

executed during the PIP interrupt  checks  if  the  CID  signal  is  lost  by

determining  if SYNC_CTR is greater than NUMCYL.  If a CID signal is not seen

for two cycles (1440 degrees) of the  engine,  the  system  will  assume  CID

failure and revert to injector on-edge timing at the nearest PIP edge.

8-165

FUEL STRATEGY, DIS INJECTOR SYNCHRONIZATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|<-- TDC CYL #1 - BEGINNING POWER STROKE

|

|<--10° BTDC

PIP       ____      ____      ____      ____      ____      ____      ____

SIGNAL: _|    |____|    |____|    |____|    |____|    |____|    |____|    |__

SYNC_CTR |         |         |         |         |         |         |

VALUE:   6-->0     1         2         3         4         5         6-->0

SYNC_CTR = 0 (RESET)           IF SYNC_CTR <> 3

|                      THEN SYNC_CTR = 3

|<--26° ATDC                  |

CID         _____________________________

SIGNAL: ___|                             |__________ nominal for CIDSEL = 0

and ENGCYL = 3

Condition A = CID High

Figure 1:  SYNC_CTR Values for a Six Cylinder Engine with a Hall CID Sensor

|<-- TDC CYL #1 - BEGINNING POWER STROKE

|

|<--10° BTDC

PIP       ____      ____      ____      ____      ____      ____      ____

SIGNAL: _|    |____|    |____|    |____|    |____|    |____|    |____|    |__

SYNC_CTR |         |         |         |         |         |         |

VALUE:   6-->0     1         2         3         4         5         6-->0

SYNC_CTR = 0 (RESET)

|

|<--22.5° ATDC

CID     ___  ____________________________________ nominal for CIDSEL = 1

SIGNAL:    ||

--

Condition A = CID Low

Figure 2:  SYNC_CTR Values for a Six Cylinder Engine with a VRS CID Sensor

8-166

FUEL STRATEGY, DIS INJECTOR SYNCHRONIZATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  SYNC_CTR = Counter which counts cylinders from the PIP signal and  resets

when equal to the number of cylinders in the engine.

Bit Flags:

-  CID_HIGH = CID input level; 1 -> high state, 0 -> low state.

Calibration Constants:

-  ENGCYL = Number of cylinders that fire during one engine revolution.

-  HP_CIDSEL = CID Hardware select switch; 0 -> Hall Effect  present,  1  ->

VRS sensor.

-  NUMCYL = Number of cylinders in the engine.

-  VCAMHP = Variable CAM timing hardware present; 1 - VCT present.

OUTPUTS

Registers:

-  SYNC_CTR = See above.

Bit Flags:

-  FUEL_IN_SYNC = Fuel synchronized with PIP.

-  SYNC_FAIL = Indicates sync signal has been lost or was incorrectly placed

after system was in sync; 0 -> sync okay, 1 -> bad or lost sync signal.

-  FIRST_SYNC = Indicates whether the first CID edge has been detected;  1->

the first CID edge has been detected.

-  SYNFLG = Cylinder #1 correctly identified flag;  1  ->  SYNC_CTR  aligned

with cylinder #1, 0 -> SYNC_CTR not aligned with cylinder #1.

8-167

FUEL STRATEGY, DIS INJECTOR SYNCHRONIZATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_CID_SYNC_COM1

CID_PROCESSING

HP_CIDSEL = 0 ---------|

;Hall effect sensor   |AND -|

|     |

CID_HIGH = 1 ----------|     |

;Rising edge of signal      |OR --| Condition A

|

HP_CIDSEL = 1 ---------|     |

;VRS sensor           |AND -|

|

CID_HIGH = 0 ----------|

;Falling edge of signal

VCAMHP = 0 ------------------|

|

Condition A -----------------|AND -| SYNC_CTR = 0

|     | SYNFLG = 0

FIRST_SYNC = 1 --------------|     | FUEL_IN_SYNC = 0

|     |  ;Sync pulse rec'd, not the first

SYNC_CTR <> NUMCYL ----------|     |  ; one, and SYNC_CTR is incorrect

|

| --- ELSE ---

Condition A -----------------|     |

|AND -| SYNC_CTR = 0

VCAMHP = 0 ------------------|     | SYNFLG = 1

| FIRST_SYNC = 1

| SYNC_FAIL = 0

|  ;In sync or first sync pulse

|

| --- ELSE ---

HP_CIDSEL = 0 ---------------|     |

|     |

CID_HIGH = 0 ----------------|     |

|AND -| SYNC_CTR = NUMCYL/2

SYNC_CTR <> NUMCYL/2 --------|     | SYNFLG = 0

;Counter out of phase       |     | FUEL_IN_SYNC = 0

|     |

VCAMHP = 0 ------------------|     |

|

| --- ELSE ---

HP_CIDSEL = 0 ---------------|     |

|     |

CID_HIGH = 0 ----------------|     |

|AND -| FIRST_SYNC = 1

SYNC_CTR = NUMCYL/2 ---------|     | SYNFLG = 1

|

VCAMHP = 0 ------------------|

8-168

FUEL STRATEGY, FOREGROUND FUEL CALCULATIONS EXECUTION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.22  FOREGROUND FUEL CALCULATIONS EXECUTION CONTROL (CDAL0)

OVERVIEW

This module specifies execution conditions and  required  execution  sequence

for  certain PIP interrupt foreground fuel routines that are external to this

module.  The execution hierarchy of relevant strategy modules is as follows.

- FUEL_PIP_INT_SYNC

- FTRAN_XTMODEL (foreground portion only)

- FUEL_FG_CALC

- FUEL_TOTAL_DELAY_CALC

- FUEL_INJ_UPDATE_REQUEST

- FUEL_MAIN_OFF (see notes below)

- FUEL_SERVICE

- FUEL_IBETA_CALC

- FUEL_SCHEDULE

- FUEL_INJ_OUT

- FUEL_DYN_CALC

- FUEL_DYN_FIT

- FUEL_DYN_OFF

The FUEL_PIP_INT_SYNC logic, which is responsible for  maintaining  SYNC_CTR,

is  executed  only  on  the  rising-edge of the PIP signal.  The Fuel Service

routine is always executed on every PIP edge (on both rising and falling  PIP

edge).

All other routines are executed on the PIP edge  specified  by  AIR_DOWN_CAL.

Furthermore,  execution  of  transient  fuel,  dynamic  fuel,  and  the  fuel

pulsewidth calculations may be restricted to every-other  PIP  at  high  RPM.

This  RPM  threshold  is  specified by the calibration constant FULPIPALT_DT,

which is compared with DT12S.

NOTES:

1.  The    FUEL_MAIN_OFF    strategy    module    is    called    from    the

FUEL_INJ_UPDATE_REQUEST  strategy  module,  and  also  from  an interrupt

service routine.  This  interrupt  is  enabled  within  the  FUEL_INJ_OUT

module,  and  is  configured  to  execute  when the pulsewidth on-edge is

issued.

2.  The FUEL_MAIN_OFF module  may  also  be  invoked  from  the  FUEL_INJ_OUT

module,  in  the  case  of  a  short  pulse  (where  both edges are to be

scheduled at the same time).

8-169

FUEL STRATEGY, FOREGROUND FUEL CALCULATIONS EXECUTION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  DATA_TIME = Time of the current PIP edge, in ticks.

-  DT12S = The period of time between two  adjacent  rising  edges  of  PIP,

ticks/pip.

-  LAST_HI_PIP = Time of the latest low-to-high PIP transition, in ticks.

Bit Flags:

-  ALT_PIP_MODE = Flag indicating that fuel and air foreground  calculations

are occurring every-other PIP.

-  PIP_HIGH = PIP Input level; 1 -> the most recent  PIP  signal  transition

was  a  rising  edge;  0  ->  the most recent PIP signal transition was a

falling edge.

-  SYNC_CTR = Counter which counts cylinders from the PIP signal and  resets

when equal to the number of cylinders in the engine.

-  SYNFLG = 1 -> SYNCTR values are synchronized with signature PIP.

Calibration Constants:

-  AIR_DOWN_CAL = Indicates on which PIP  edge  to  calculate  Cylinder  Air

Charge; 0 -> use PIP up-edge; 1 -> use PIP down-edge.

-  FULPIPALT_DT = DT12S value below which some fuel  calculations  occur  on

every-other PIP edge.

STRATEGY MODULES

Entry Points:

-  FUEL_FG_PIPEDG = The main entry point to this strategy module.

External Routines:

-  fuel_fg_transient_calculations = Routine  which  performs  the  necessary

foreground  transient  fuel  calculations,  located  in  strategy  module

FTRAN_XTMODEL_COMn.

-  FUEL_DYN_CALC =  Routine  which  performs  the  foreground  dynamic  fuel

calculations, located in strategy module FUEL_DYN_CALC_COMn.

-  FUEL_FG_CALC = Routine  which  performs  the  necessary  foreground  fuel

pulsewidth calculations, located in strategy module FUEL_FG_CALC_COMn.

-  FUEL_PIP_INT_SYNC = Routine which updates SYNC_CTR, and checks  for  loss

of injector synchronization.

-  FUEL_SERVICE = Routine for foreground injector  output  calculations  and

scheduling, located in strategy module FUEL_SERVICE_COMn.

8-170

FUEL STRATEGY, FOREGROUND FUEL CALCULATIONS EXECUTION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_FG_PIPEDG_COM3

BEGIN: FUEL_FG_PIPEDG

Execute this routine at each rising PIP edge and at each  falling  PIP  edge.

This  strategy  module  corresponds  to  a  portion of the PIP_DATA module in

software.

NOTE:  The calibration flag AIR_DOWN_CAL also determines on  which  edge  the

cylinder  air  charge  calculation  is performed.  The air charge calculation

must occur before this routine begins.

PIP_HIGH = 1 ---------------------------| LAST_HI_PIP := DATA_TIME

| ;Store the time of each rising

| ; PIP edge

| Do: FUEL_PIP_INT_SYNC

Transient fuel, fuel pulsewidth, and dynamic fuel are  always  calculated  on

the  rising PIP edge when injectors are not synchronized.  When synchronized,

these calculations are made on the PIP edge  specified  by  AIR_DOWN_CAL  (if

AIR_DOWN_CAL  =  0, calculations occur on the rising PIP edge).  Fuel Service

is execution on every  PIP  edge,  regardless  of  the  calibrated  value  of

AIR_DOWN_CAL, and regardless of the value of SYNFLG.

This logic determines if this PIP edge is the  correct  edge  for  foreground

fuel processing.  If not, execute only the Fuel Service routine and exit.

PIP_HIGH = 0 ---------------|

;Falling PIP edge          |

|AND -|

SYNFLG = 0 -----------|     |     |

;Not in sync         |OR --|     |

|           |

AIR_DOWN_CAL = 0 -----|           |

;Execute on pip up-edge          |

|OR --| Do: FUEL_SERVICE

PIP_HIGH = 1 ---------------|     |     | Exit: FUEL_FG_PIPEDG

;Rising PIP edge           |     |     |  ;On the wrong PIP edge

|     |

SYNFLG = 1 -----------------|AND -|

;Fuel in sync              |

|

AIR_DOWN_CAL = 1 -----------|

;Execute on PIP down-edge

8-171

FUEL STRATEGY, FOREGROUND FUEL CALCULATIONS EXECUTION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

This is the correct edge for foreground fuel processing.   Determine  whether

limited foreground processing should be employed due to high RPM.

At high RPM, execute the transient fuel and fuel pulsewidth  calculations  on

every-other  PIP.   At high RPM, dynamic fuel calculations are disabled.  The

Fuel Service routine is executed every PIP, regardless of RPM.

DT12S >= FULPIPALT_DT ------------------| ALT_PIP_MODE := 0

;Low RPM value                         | Do: fuel_fg_transient_calculations

| Do: FUEL_FG_CALC

| Do: FUEL_SERVICE

| Do: FUEL_DYN_CALC

| Exit: FUEL_FG_PIPEDG

|

| --- ELSE ---

|

DT12S < FULPIPALT_DT -------------|     |

;Higher RPM value                |AND -| ALT_PIP_MODE := 1

|     | Do: fuel_fg_transient_calculations

SYNC_CTR is even -----------------|     | Do: FUEL_FG_CALC

;Alternate PIP up-edges or             | Do: FUEL_SERVICE

; alternate PIP down-edges             | Exit: FUEL_FG_PIPEDG

|

| --- ELSE ---

|

| ALT_PIP_MODE := 1

| Do: FUEL_SERVICE

END: FUEL_FG_PIPEDG

8-172

FUEL STRATEGY, PIP INTERRUPT LOGIC FOR FUEL SYNCHRONIZATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.23  PIP INTERRUPT LOGIC FOR FUEL SYNCHRONIZATION (CDAH0)

OVERVIEW

The  synchronization  logic  processed  during  the  PIP  high  interrupt  is

responsible  for  incrementing the SYNC_CTR and determining if the CID signal

is lost.  The SYNC_CTR is a register used to identify the position and  cycle

of  the  engine's cylinders.  When the CID signal is lost the SYNC_CTR is not

reset to 0 by the CID_PROCESSING logic.  If  the  PIP  high  interrupt  logic

observes  that the counter has not been reset, i.e., SYNC_CTR is greater than

NUMCYL (the number of cylinders), it resets  the  counter  and  sets  a  flag

(SYNC_FAIL).  If the counter does not get reset by the CID_PROCESSING logic a

second time, the command to synchronize fuel is canceled.

DEFINITIONS

INPUTS

Registers:

-  SYNC_CTR = Counter which counts cylinders from the PIP signal and  resets

when equal to the number of cylinders in the engine.

Bit Flags:

-  SYNC_FAIL = Indicates sync signal has been lost or was incorrectly placed

after system was in sync; 0 -> sync okay, 1 -> bad or lost sync signal.

Calibration Constants:

-  NUMCYL = Number of cylinders in the engine.

-  VCAMHP = Variable CAM timing hardware present; 1 -> VCT present.

OUTPUTS

Registers:

-  SYNC_CTR = See above.

Bit Flags:

-  FUEL_IN_SYNC = Fuel synchronized with PIP.

-  SYNC_FAIL = See above.

-  SYNFLG = Cylinder 1 correctly identified flag; 1 -> SYNC_CTR aligned with

cylinder 1, 0 -> SYNC_CTR not aligned with cylinder 1.

8-173

FUEL STRATEGY, PIP INTERRUPT LOGIC FOR FUEL SYNCHRONIZATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_PIP_INT_SYNC_COM2

PIP INTERRUPT PROCESSING

(processed during the PIP high interrupt)

VCAMHP = 0 ----------------------------------| SYNC_CTR = SYNC_CTR + 1

VCAMHP = 0 ----------------------------|

|

SYNC_CTR > NUMCYL ---------------------|AND -| SYNC_CTR = 0

|     | SYNC_FAIL = 1

SYNC_FAIL = 0 -------------------------|     |

| --- ELSE ---

SYNC_CTR > NUMCYL ---------------------|     |

|AND -| SYNC_CTR = 0

VCAMHP = 0 ----------------------------|     | SYNFLG = 0

| FUEL_IN_SYNC = 0

8-174

FUEL CONTROL STRATEGY, FUEL PULSEWIDTH, FOREGROUND FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.24  FOREGROUND FUEL PULSEWIDTH CALCULATION (CDAL0)

OVERVIEW

This module is designed to calculate updated main fuel  pulsewidth  for  each

engine  bank, using the latest air charge value calculated upon completion of

the air charge computation during the PIP interrupt.   Included  within  this

module are:  1) minimum and maximum pulsewidth clips; 2) a check for a change

greater than a calibrated  amount  (PW_MIN_PCT)  in  computed  pulsewidth  to

determine  whether  to  update previously-scheduled fuel pulsewidth off-edges

(if the injectors are synchronized with the prior request); and 3) a  request

for  a  new  computation  of  the  injector timing, performed by the strategy

module FUEL_INJ_UPDATE_REQUEST.

PURGE COMPENSATION (PCOMP)

The Purge Compensation (PCOMP) is a new type of  adaptive  routine  which  is

used  to  correct  for errors in the fuel calculation that are induced by the

introduction of purge vapors into the engine.  The  PCOMP  strategy  operates

only  during  closed  loop  engine operation with the canister purge enabled.

The PCOMP register, PCOMP_LBM, is reset to "0" during open loop operation and

when key power is turned off.

PCOMP corrects for errors in the fuel calculation by  subtracting  the  PCOMP

term from the base fuel equation:

base fuel = (CYL_AIR_CHG * F_A_RATIO[n]) + TFC_HR

new base fuel = base fuel - PCOMP_LBM

PCOMP will increment or decrement to keep  LAMBSE  at  a  value  of  "1".   A

calibratable  PCOMP  Integrator  Scaling factor (PC_SCALE) is used to control

the rate at which PCOMP responds to LAMBSE deviations from  "1".   A  typical

PC_SCALE value is 0.0015.  PCOMP is calculated as follows:

PCOMP_LBM = PCOMP_LBM + PC_SCALE * [(LAMBSE1 - 1) *

(CYL_AIR_CHG * F_A_RATIO1)]

The PCOMP_LBM computation is done in the background  in  the  CANP_PCOMP_COMn

module.

NOTE:  Parameters whose alphabetic characters  are  in  the  lower  case  are

non-displayable temporary registers.

8-175

FUEL CONTROL STRATEGY, FUEL PULSEWIDTH, FOREGROUND FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  AHISL_TICK  =  Dual  slope  high  slope  in  lbs/tick,  compensated   for

temperature.

-  ALOSL_TICK  =  Dual  slope  low  slope  in  lbs/tick,   compensated   for

temperature.

-  CYL_AIR_CHG = Current cylinder air charge used in fuel calculation, lbm.

-  DT12S = The period between the last two PIP low-to-high transitions.

-  DT12SH = The high byte in the three byte time between PIP up-edges.

-  F_A_RATIO[n] = Desired fuel/air ratio  for  the  identified  engine  bank

number n, where n = 1, 2.

-  FAR_MUL = Multiplier to the fuel-air ratio,  used  to  introduce  a  fuel

system  error.   This value is set in the background fuel strategy module

FUEL_BG_CALC_COMx.

-  FB_DIFF = Difference between FUEL_BKPT divided by temperature compensated

high  slope  and  FUEL_BKPT divided by temperature compensated low slope,

ticks.

-  FUELPW[n] = The previous injector pulsewidth (in  clock  ticks)  for  the

identified engine bank number n, where n = 1, 2.

-  MINPWT = Minimum desired fuel pulsewidth, ticks.

-  MINPW_LBMF = Fuel Mass at minimum fuel pulsewidth, LBM.

-  PCOMP_LBM = Purge compensation, lbs mass/cylinder.

-  PWOFF = Injector pulsewidth offset, clock ticks.

-  TFC_HR = Fuel  mass  per  injection  from  transient  fuel  compensation,

lbs/cylinder.

Bit Flags:

-  CHANGE_FUELPW = A flag that indicates that  the  desired  pulsewidth  for

in-progress  fuel  injections  needs  to  be  modified because the latest

calculated fuel pulsewidth value for that injection has changed  by  more

than the percent amount calibrated as PW_MIN_PCT.

-  CRKFLG = Flag indicating when in CRANK mode; 1 -> crank.

-  FAR_MUL_FLG = Flag indicating the an error should be introduced into  the

fuel system, via the fuel-air ratio multiplier FAR_MUL.

8-176

FUEL CONTROL STRATEGY, FUEL PULSEWIDTH, FOREGROUND FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  FUEL_BKPT = The amount of fuel flow when transfer is made  from  the  low

slope to the high slope.

-  NUMCYL = Number of cylinders in the engine.

-  NUMEGO = Switch indicating number of HEGO sensors present.

-  PW_MIN_PCT = Minimum percent change in FUELPW (PIP-to-PIP) to  trigger  a

pulsewidth update to the injector event presently in-progress.

OUTPUTS

Registers:

-  FUELPW[n] = See above.

-  LBMF_INJ[n] = Two registers which contain the fuel mass per injection for

each engine bank; n = 1, 2.

Bit Flags:

-  CHANGE_FUELPW = See above.

TEMPORARY REGISTERS

-  fgbeta[n] = The fuel pulsewidth in percent of a PIP period.   This  is  a

temporary register that will be used to calculate injector on-edge time.

-  fuel_bk[n] = A temporary register that contains the  fuel  mass  required

for the identified engine bank number n, where n = 1, 2.

-  fuelpw[n] = A temporary register that contains the interim value of  fuel

pulsewidth  (in ticks) for the identified engine bank number n, where n =

1, 2.

8-177

FUEL CONTROL STRATEGY, FUEL PULSEWIDTH, FOREGROUND FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

STRATEGY MODULES

Entry Points:

-  FUEL_FG_CALC = Main  entry  point  for  the  foreground  fuel  pulsewidth

calculation strategy module.

Local Routines:

-  compute_fuel_pulsewidth(n)  =  Routine  to  compute  the  fuel  mass  and

corresponding pulsewidth for the desired bank.

External Routines:

-  FUEL_INJ_UPDATE_REQUEST =  Injector  update  request  logic;  located  in

strategy module FUEL_INJ_UPDATE_REQUEST_COMn.

-  FUEL_TOTAL_DELAY_CALC  =  Computes  new  values  for   TOTAL_DELAY1   and

TOTAL_DELAY2; located in strategy module FUEL_TOTAL_DELAY_CALC_COMn.

8-178

FUEL CONTROL STRATEGY, FUEL PULSEWIDTH, FOREGROUND FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_FG_CALC_COM14

BEGIN: FUEL_FG_CALC

; Called from within the FUEL_FG_PIPEDG strategy module, and executed on

; either the rising or falling PIP edge.

; This strategy module corresponds to the FRGRND_FUEL_CALC module in

; software.

; This foreground fuel calculation is not required during crank mode.

CRKFLG = 1 -----------------------------| Exit FUEL_FG_CALC

;In crank mode                         |

| --- ELSE ---

|

| Continue FUEL_FG_CALC

; For each engine bank, compute the fuel required by a cylinder in that bank,

; and compute the corresponding pounds mass per injection.

Unconditionally ------------------------| Call: compute_fuel_pulsewidth(1)

| Call: FUEL_TOTAL_DELAY_CALC(1)

NUMEGO > 1 -----------------------------| Call: compute_fuel_pulsewidth(2)

| Call: FUEL_TOTAL_DELAY_CALC(2)

; Update the injector off-edge times that are already scheduled in the

; high-speed output carousel.  Whether the update is performed is

; dependent upon the state of the flag CHANGE_FUELPW.

CHANGE_FUELPW = 1 ----------------------| Call: FUEL_INJ_UPDATE_REQUEST

| CHANGE_FUELPW := 0

END: FUEL_FG_CALC

8-179

FUEL CONTROL STRATEGY, FUEL PULSEWIDTH, FOREGROUND FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: compute_fuel_pulsewidth(n)

; Compute the fuel mass required for this bank.

Unconditionally ------------------------| fuel_bk[n] := (CYL_AIR_CHG *

|   F_A_RATIO[n]) + TFC_HR

|    - PCOMP_LBM

| fuel_bk[n] := MAX(fuel_bk[n],0)

FAR_MUL_FLG = 1 ------------------------| fuel_bk(n) := fuel_bk(n) * FAR_MUL

|  ;Introduce a fuel system error

; Save the fuel mass value, and determine the corresponding pulsewidth

; using the piecewise-linear model of fuel delivery.

fuel_bk[n] < FUEL_BKPT -----------------| LBMF_INJ[n] := fuel_bk[n]

;Run mode and low slope                | fuelpw[n] := fuel_bk[n] /

|                     ALOSL_TICK

|

| --- ELSE ---

|

;Run mode and high slope               | LBMF_INJ[n] := fuel_bk[n]

| fuelpw[n] := (fuel_bk[n] /

|            AHISL_TICK) - FB_DIFF

; The fuel pulsewidth is checked to ensure that it exceeds some minimum

; value, and if it does not, the minimum pulsewidth is output. The only

; exception to this is if the pulsewidth is zero, in which case no fuel

; is output.

fuelpw[n] < MINPWT -------------|

|AND -| fuelpw[n] := MINPWT

F_A_RATIO[n] <> 0 --------------|     | LBMF_INJ[n] := MINPW_LBMF

8-180

FUEL CONTROL STRATEGY, FUEL PULSEWIDTH, FOREGROUND FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

; Check to see if the pulsewidth would require the injector to be on more

; than all the time, in which case, clip the injector 'on' time to be 1/32

; of a PIP period less than 'full on'.

DT12SH = 0 ---------------------|

;Time is in 2 bytes            |

|AND -| fuelpw[n] := (NUMCYL - 0.03125) *

(fuelpw[n] /                    |     |                DT12S

DT12S) >= NUMCYL -------------|     | fgbeta[n] := (NUMCYL - 0.03125)

;Fuel is on too long                 |               + (PWOFF / DT12S)

| LBMF_INJ[n] := (fuelpw[n] +

|              FB_DIFF) * AHISL_TICK

|

| --- ELSE ---

|

DT12SH = 0 ---------------------------| fgbeta[n] := (fuelpw[n] / DT12S)

|                   + (PWOFF / DT12S)

|

| --- ELSE ---

|

| fgbeta[n] := 0

; If the currently calculated fuel pulsewidth, fuelpw[n], has changed by more

; than PW_MIN_PCT from the last computed value (stored in FUELPW[n]), and if

; the last computed value is not zero (e.g., as in the case of fuel cut-off),

; then set the CHANGE_FUELPW flag. At the next check of the output queue, if

; any injector has a pulse in progress with the off-edge in the queue, then

; the off-edge will be updated to provide the latest pulsewidth. In the case

; of a reduction in pulsewidth size, the off-edge will not be scheduled to be

; any earlier than the current time.

FUELPW[n] <> 0 ---------------------|

|AND -| CHANGE_FUELPW := 1

ABS(fuelpw[n] - FUELPW[n])          |

/ FUELPW[n] > PW_MIN_PCT -------|

; Set the stored pulsewidth equal to the currently calculated pulsewidth.

Unconditionally --------------------------| FUELPW[n] := fuelpw[n]

END: compute_fuel_pulsewidth

8-181

FUEL STRATEGY, INJECTOR TIMING CALCULATION, FOREGROUND FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.25  FOREGROUND INJECTOR TIMING - TOTAL DELAY CALCULATION (CDAI0)

OVERVIEW

The total delay calculation is part of the foreground fuel  software  module,

largely  because  TOTAL_DELAY[n]  is most sensitive to FUELPW[n] changes when

off-edge timing is  requested.   Another  reason  is  that  fgbeta,  the  new

FUELPW[n] in terms of beta units (PIP fractions) is calculated as a temporary

register in the foreground fuel calculation and is  required  for  the  total

delay calculation.

Injection events need to be timed to occur at some point  before  the  intake

cycle  occurs  for  a  cylinder.  Because an engine is cyclical, the injector

timing may also be considered as a delay after an identifying cylinder event.

Because  the  SYNC_CTR  mechanism  readily  identifies  Top Dead Center (TDC)

between the compression and power strokes for each cylinder, it is useful  to

identify  injection  events in terms of delay after TDC rather than as a time

before an intake event.

In general, it is desirable to specify an end of pulse position  for  a  fuel

pulse,  to  ensure  proper  placement  before  an intake cycle.  However, the

accurate delivery of the off edge at the  desired  engine  position  requires

some tradeoffs at any unsteady state:

-  The on edge time is determined by  the  desired  off  edge  position  (in

degrees  converted to time, based on the angular velocity at the time the

on edge was scheduled) minus the fuel pulsewidth.

-  Accurate fuel pulsewidth delivery has priority over off edge placement.

-  The angular velocity  may  change  between  the  time  the  on  edge  was

scheduled and the time the off edge occurs.

-  The fuel pulsewidth requested may change between the time the on edge was

scheduled and the time the off edge is scheduled.

The term TOTAL_DELAY is used to describe the whole and fractional PIP periods

from  TDC  to  the start of the fuel pulse for a cylinder.  One difficulty in

using angular displacement from TDC for  fuel  pulses  is  that  the  desired

placement  for on edges frequently crosses TDC in normal operation.  A change

in TOTAL_DELAY from just greater than 0 to just less than NUMCYL is  actually

a very small change in injector timing.

TOTAL DELAY CALCULATION NOTES:

1.  The change in injector timing between  consecutive  injector  firings  is

limited  to  a maximum of one PIP period.  This has the effect of walking

the injector timing for large timing changes.

2.  Synchronized operation is implicit, since the fuel  service  module  will

not set FUEL_IN_SYNC when the system is not synchronized.

8-182

FUEL STRATEGY, INJECTOR TIMING CALCULATION, FOREGROUND FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

3.  Adjusting INJ_PIP_CNT[n] for changes to TOTAL_DELAY[n] results in  faster

response  for  more  accurate  fuel  pulse placement and scheduling under

dynamic conditions.

DEFINITIONS

INPUTS

Registers:

-  INJDLY_BETA = The injector timing in percent of PIP periods after the top

dead  center  PIP.   Depending  on  INJREF,  this  may  be  a request for

placement of either the ON edge or the OFF edge to the  specified  engine

rotational position.

-  INJ_PIP_CNT[n] = Injector PIP counter for bank n, which tracks the number

of PIP edges until an injector in bank n is due to be serviced.

-  TOTAL_DELAY[n] = Delay in percent of PIP period from the reference PIP to

the starting edge of the fuel pulse, for bank n.

Bit Flags:

-  FUEL_IN_SYNC = Indicates whether TOTAL_DELAY1 and TOTAL_DELAY2 have  been

assigned initial calculated values; 0 -> initial values not yet assigned,

1 = values have been assigned.

-  SYNFLG = Indicates that SYNC_CTR is aligned with the engine so that  fuel

may be synchronized to the desired position with respect to the engine.

Calibration Constants:

-  INJREF = Indicates which edge is to be scheduled by INJDLY_BETA; 0 ->  ON

edge  of  fuel  pulse  should  be  at INJDLY_BETA past TDC; 1 -> OFF edge

should be at INJDLY_BETA past TDC.

-  NUMCYL = Number of cylinders in the engine.

OUTPUTS

Registers:

-  INJ_PIP_CNT[n] = See above.

-  TOTAL_DELAY[n] = See above.

TEMPORARY REGISTERS

-  bank_num = The EGO bank number currently being processed, either  bank  1

or bank 2.

-  delta_delay = Temporary register containing total_delay -  TOTAL_DELAY[n]

value

8-183

FUEL STRATEGY, INJECTOR TIMING CALCULATION, FOREGROUND FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  fgbeta = The fuel pulsewidth in percent of a PIP period.

-  total_delay = Temporary working location for new TOTAL_DELAY[n] value

STRATEGY MODULES

Entry Points:

-  FUEL_TOTAL_DELAY_CALC(n) = The main entry point for  this  module,  which

computes the TOTAL_DELAY value for bank n.

8-184

FUEL STRATEGY, INJECTOR TIMING CALCULATION, FOREGROUND FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_TOTAL_DELAY_CALC_COM1

BEGIN: FUEL_TOTAL_DELAY_CALC(bank_num)

; Called from within the FUEL_FG_CALC strategy module.

; This strategy module corresponds to a portion of the FRGRND_FUEL_CALC

; software module.

; Determine if a new total delay is to be calculated for the bank

; specified by bank_num.  Do not calculate TOTAL_DELAY[n] until an

; initial value has been assigned to it at the first sequential

; fuel event.

FUEL_IN_SYNC = 0 -----------------------| Exit FUEL_TOTAL_DELAY_CALC

|  ;Wait for first

|  ; sequential event

;Calculate the new total delay.

;NOTE: With delay in percent of PIP period and if fuel timing reference

;      is off-edge, then subtract FUELPW in percent of PIP units.

INJREF = 1 -----------------------------| total_delay := INJDLY_BETA

|     - fgbeta

|  ;Off-edge timing, subtract

|  ; FUELPW in PIP periods to get

|  ; the on edge

|

| --- ELSE ---

|

| total_delay := INJDLY_BETA

;If delay is negative, delay + NUMCYL gives positive delay and

;same position.

;NOTE: Use modulo NUMCYL arithmetic.

total_delay < 0 ------------------------| total_delay :=

|   total_delay + NUMCYL

| total_delay :=

|   MOD(total_delay, NUMCYL)

;Compute the difference between the old and new delays.

Unconditionally ------------------------| delta_delay := total_delay

|    - TOTAL_DELAY[bank_num]

8-185

FUEL STRATEGY, INJECTOR TIMING CALCULATION, FOREGROUND FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;If delta is too large, reverse direction by adding or subtracting

;NUMCYL, 2 engine revolutions, from delta_delay.

|delta_delay| >= NUMCYL/2 --------|

|AND -| delta_delay :=

delta_delay > 0 ------------------|     |      delta_delay - NUMCYL

|  ;If positive delta and more

|  ; than one engine revolution

|  ; difference, subtract NUMCYL

|  ; for shortest path to

|  ; synchronization.

|

| --- ELSE ---

|

|delta_delay| >= NUMCYL/2 --------|     |

|AND -| delta_delay :=

delta_delay < 0 ------------------|     |      delta_delay + NUMCYL

|  ;If large negative delta, make

|  ; positive by adding two

|  ; rotations

;Clip the change in TOTAL_DELAY[n] to 1.0 PIP maximum, and adjust

;INJ_PIP_CNT[n] by the number of half PIPs that TOTAL_DELAY[n] has

;changed.

;NOTE: TOTAL_DELAY[n] is in full PIP units, while INJ_PIP_CNT[n] is in

;      half PIPs.

|delta_delay| <= 1.0 -------------------| TOTAL_DELAY[bank_num] :=

|       total_delay

| INJ_PIP_CNT[bank_num] :=

|    INJ_PIP_CNT[bank_num]

|    + (2 * delta_delay)

|

| --- ELSE ---

|

delta_delay > 0 ------------------------| TOTAL_DELAY[bank_num] :=

|   TOTAL_DELAY[bank_num] + 1.0

| INJ_PIP_CNT[bank_num] :=

|    INJ_PIP_CNT[bank_num] + 2

|

| --- ELSE ---

|

delta_delay < 0 ------------------------|  TOTAL_DELAY[bank_num] :=

|    TOTAL_DELAY[bank_num] - 1.0

| INJ_PIP_CNT[bank_num] :=

|    INJ_PIP_CNT[bank_num] - 2

;NOTE: Use modulo NUMCYL arithmetic.

TOTAL_DELAY[bank_num] < 0 --------------| TOTAL_DELAY[bank_num] :=

|   TOTAL_DELAY[bank_num] + NUMCYL

END: FUEL_TOTAL_DELAY_CALC

8-186

FUEL STRATEGY, FUEL PULSEWIDTH, INJECTOR UPDATE REQUEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.26  INJECTOR UPDATE REQUEST (CDAM0)

OVERVIEW

This module is executed when one or more injectors have an off edge scheduled

in  the  output carousel, and it has been determined that the fuel pulsewidth

used to compute the off-edge timing is in error with respect to  the  current

pulsewidth  calculation.   The  amount of error causing this recalculation is

specified in the FUEL_FG_CALC strategy module.  This condition is denoted  by

setting  the  CHANGE_FUELPW  flag  in the FUEL_FG_CALC strategy module before

calling this module.

The injector off-edge  is  initially  entered  into  the  carousel  when  the

pulsewidth  on-edge is issued, at which time an interrupt generates a call to

the FUEL_MAIN_OFF strategy module.  When an update has been requested to  the

injector  off-edge timing, this module will recalculate the off-edge time for

each scheduled pulse, and overwrite the  existing  time  in  the  appropriate

reserved slot of the carousel.

SOFTWARE NOTES

The state of HSO  output  pins  can  be  checked  using  the  following  flag

definitions,  where  HSO_PIN_STATE  is defined as a word representing the HSO

STATES REGISTER:

INJ1_PIN_STATE      FLAG      ^01,HSO_PIN_STATE

INJ2_PIN_STATE      FLAG      ^02,HSO_PIN_STATE

INJ3_PIN_STATE      FLAG      ^04,HSO_PIN_STATE

INJ4_PIN_STATE      FLAG      ^08,HSO_PIN_STATE

INJ5_PIN_STATE      FLAG      ^10,HSO_PIN_STATE

INJ6_PIN_STATE      FLAG      ^20,HSO_PIN_STATE

INJ7_PIN_STATE      FLAG      ^40,HSO_PIN_STATE

INJ8_PIN_STATE      FLAG      ^80,HSO_PIN_STATE

Pending HSO interrupts can be checked via  the  bits  of  the  word  register

HSO_INT_1_PEND.

DEFINITIONS

INPUTS

Calibration Constants:

-  NUMCYL = Number of cylinders in the engine.

OUTPUTS

Bit Flags:

8-187

FUEL STRATEGY, FUEL PULSEWIDTH, INJECTOR UPDATE REQUEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  LONG_PULSE = Flag indicating that at least one of the requested  off-edge

times  is  more  than a clock roll-over into the future, and so cannot be

scheduled at this time.

TEMPORARY REGISTERS

-  i = Injector number to have fuel pulse off-edge time updated.

STRATEGY MODULES

Entry Points:

-  FUEL_INJ_UPDATE_REQUEST = The main entry point for this strategy module.

Local Routines:

-  update_injector(i) = Conditionally update the off-edge time for  injector

i, only if the corresponding HSO output pin is high.

External Routines:

-  FUEL_MAIN_OFF(i) = Strategy routine which reschedules the off-edge timing

of the pulse for injector i (i = 0..NUMCYL-1).

8-188

FUEL STRATEGY, FUEL PULSEWIDTH, INJECTOR UPDATE REQUEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_INJ_UPDATE_REQUEST_COM1

BEGIN: FUEL_INJ_UPDATE_REQUEST

Called from within the FUEL_FG_CALC strategy module.

This strategy module corresponds to  the  INJECTOR_UPDATE_REQUEST  module  in

software.

Clear the LONG_PULSE flag, which is set in the FUEL_MAIN_OFF strategy module.

unconditionally ------------------------| LONG_PULSE := 0

Update  off-edge  timing  for  each  injector.   These  off-edge  times  were

initially  computed  and entered into the high-speed output carousel when the

on-edge was issued, at which time an interrupt  service  routine  called  the

FUEL_MAIN_OFF strategy module.

FOR i := 0 TO NUMCYL - 1 ---------------| Do: update_injector(i)

| ;Attempt to reschedule the off-

| ; edge time for each injector

END: FUEL_INJ_UPDATE_REQUEST

BEGIN: update_injector(i)

Update only those injectors which are presently actuated.  The state of  each

HSO  output  pin  is  available  through the HSO state register.  Pending HSO

interrupts are indicated by the HSO_INT_1_PEND register in software.

HSO for injector i active --------|

|AND -| Do: FUEL_MAIN_OFF(i)

HSO interrupt not pending for i --|     | ;Only called if output is high

| ; and HSO interrupt not pending

END: update_injector

8-189

FUEL STRATEGY, FUEL PULSEWIDTH, MAIN FUEL OFF - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.27  FUEL PULSEWIDTH, MAIN FUEL OFF (CDAI0)

OVERVIEW

This module is executed for each non-zero main fuel pulse.  This  routine  is

called before a main fuel pulse off-edge is placed in the output carousel for

a high speed output port.  The routine calculates the off-edge time from  the

on-edge time and the fuel pulsewidth.  If there is sufficient space to insert

the off-edge into the carousel and the off-edge  time  is  within  the  early

limit  (i.e.,  there  is  not enough time to first place it into the software

queue), then the edge is placed in the carousel.  Otherwise, the direction of

the edge transition and the edge time are placed in the software queue.

LST_PWx, LBMF_TABx, and FUEL_SUM are also updated by this  routine.   LST_PWx

is  a  table  containing  the main pulsewidths output to each of the engine's

cylinders by the most recent of their respective main  fuel  pulses.   It  is

used  by the Dynamic Fuel strategy.  LBMF_TABx is a table containing the mass

of fuel injected to each of the engine's cylinders  by  the  most  recent  of

their  respective  main  fuel pulses.  FUEL_SUM keeps track of the total fuel

consumption for the trip computer.

NOTE:  Parameters whose alphabetic characters  are  in  the  lower  case  are

non-displayable temporary registers.

SOFTWARE NOTES

1.  The arrays  on_edge_time  and  off_edge_time  correspond  to  the  arrays

LAST_ON_INJ  and LAST_OFF_INJ in software, respectively.  This information is

also written to the high-speed output carousel, when an on-edge  or  off-edge

is scheduled.

2.  As a potential chronometric thrift, temporary registers may be  used  for

updating FUEL_SUM.  When CHANGE_FUELPW is set, the change in fuel mass can be

added to a temporary register (by adding the new fuel mass,  and  subtracting

out the previous fuel mass).  When CHANGE_FUELPW is not set, the value in the

temporary register can be transferred into the FUEL_SUM  register.   See  the

INJECTOR_OFF  code segment in [LIBRARY.EOS]INJ_OUT_CG56K_EOS_1.AST;(3) for an

example.

8-190

FUEL STRATEGY, FUEL PULSEWIDTH, MAIN FUEL OFF - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  CRK_FUL = Fuel pulsewidth used in crank mode, sec.

-  FPW_FG_ISF_A = Term added to the fuel pulsewidth  to  compensate  for  an

injector  which did not fire in the previous cycle, in ticks.  This value

is determined within the transient fuel strategy modules.

-  FUELPW[bank_num] = The main fuel  pulsewidth  (in  ticks,  not  including

PWOFF)  most  recently calculated for a cylinder in the engine bank whose

'bank number' is bank_num, where bank_num is 1 or 2.

-  FUEL_SUM  =  An  accumulator  which  stores  the  total  amount  of  fuel

delivered; lbm.

-  LBMF_INJ[bank_num] = The Fuel Mass (lbm) of  the  main  fuel  pulse  most

recently  delivered  by a SEFI injection to a cylinder in the engine bank

whose 'bank number' is bank_num, where bank_num is 1 or 2.

-  PCOMP_LBM = Purge compensation, in pounds mass per cylinder.

-  PWOFF = Injector pulsewidth offset, clock ticks.

Bit Flags:

-  CHANGE_FUELPW  =  Flag  indicating  that  the  desired   pulsewidth   for

in-progress  fuel  injections  needs  to  be  updated  because the latest

calculated fuel pulsewidth value has changed by more  than  a  calibrated

amount.

-  CRKFLG = Engine Mode Flag; 1 -> Crank Mode, 0 -> Not in Crank Mode.

-  PCOMP_ENA = Purge compensation strategy enabled flag; 1 -> PCOMP enabled.

Non-Calibratable Constants:

-  stcf = Seconds-to-ticks conversion.

OUTPUTS

Registers:

-  FUEL_SUM = See above.

-  LBMF_TAB[inj_num] = The fuel mass of the main fuel  pulse  most  recently

delivered  by  the  injector whose 'injector number' is inj_num.  (One of

the registers in an array of registers composed of one register for  each

injector on the engine.)

-  LST_PW[inj_num] = The main fuel  pulsewidth  (in  ticks)  (not  including

PWOFF)  most  recently scheduled for the injector whose 'injector number'

is inj_num.  (One of the registers in an array of registers  composed  of

one register for each injector on the engine.)

8-191

FUEL STRATEGY, FUEL PULSEWIDTH, MAIN FUEL OFF - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TEMPORARY REGISTERS

-  mainpw_bank_num = The bank number  of  the  engine  bank  for  which  the

subject  main  fuel  pulsewidth  was calculated; can equal either 1 or 2.

This temporary  variable  is  calculated  elsewhere  in  a  part  of  the

foreground fuel strategy that executes synchronously with this module.

-  mainpw_inj_num = The 'injector number' of  the  injector  for  which  the

subject  main  fuel  pulse is scheduled.  (Note:  An injector's 'injector

number' is NOT the same as its 'firing sequence number'.) This  temporary

variable  is  calculated  elsewhere  in  a  part  of  the foreground fuel

strategy that executes synchronously with this module.

-  off_edge_time[i] = The time at which the fuel off-edge command is  to  be

executed  to  turn  off  the fuel on the injector for which the main fuel

pulse is  to  be  scheduled;  clock  ticks.   (The  information  in  this

temporary  register  is to be transformed as appropriate and communicated

to the appropriate hardware structure, in a manner  consistent  with  all

required  h/w and opsys protocol, so that the off-edge event is correctly

scheduled.)

-  on_edge_time[i] = The time at which the injector is to be turned  on  for

the  cylinder  for  which  the  main fuel pulse is to be scheduled; clock

ticks.  This temporary variable is calculated elsewhere in a part of  the

foreground fuel strategy that executes synchronously with this module.

8-192

FUEL STRATEGY, FUEL PULSEWIDTH, MAIN FUEL OFF - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_MAIN_OFF_COM11

BEGIN: FUEL_MAIN_OFF(i)

; Invoked from within the FUEL_INJ_UPDATE_REQUEST strategy module, and from

; the interrupt service routine invoked when the fuel pulse on-edge is

; issued.  May also be invoked from FUEL_INJ_OUT for short pulses.

; This strategy module corresponds to the INJECTOR_OFF module in software.

; Calculate and schedule the pulsewidth off-edge.

CRKFLG = 1 -----------------------------| off_temp = on_edge_time[i]

|    + (CRK_FUL * stcf) + PWOFF

|

| --- ELSE ---

|

| off_temp = on_edge_time[i]

|    + FUELPW[mainpw_bank_num]

|    + PWOFF

; Compensate for an injector which was cut-out during the previous cycle.

CRKFLG = 0 -----------------------|

|AND -| off_temp =

LST_PW[mainpw_inj_num] = ^FFFF ---|     |   off_temp + FPW_FG_ISF_A

; Check that the requested off-edge is not more than 1 clock roll-over into

; the future, and set the LONG_PULSE flag if it is.  Recompute all scheduled

; off-edges in 50 milliseconds.

off_temp - clock > 65535 ---------------| LONG_PULSE = 1

| Enable a 50ms interrupt to invoke

|   module FUEL_INJ_UPDATE_REQUEST

| Exit: FUEL_MAIN_OFF

|

| --- ELSE ---

|

| off_edge_time[i] = off_temp

8-193

FUEL STRATEGY, FUEL PULSEWIDTH, MAIN FUEL OFF - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

; Update last pulsewidth, last fuel mass, and adjust the running fuel sum.

Unconditionally ------------------------| LST_PW[mainpw_inj_num] =

|          FUELPW[mainpw_bank_num]

| LBMF_TAB[mainpw_inj_num] =

|          LBMF_INJ[mainpw_bank_num]

| FUEL_SUM = FUEL_SUM +

|          LBMF_INJ[mainpw_bank_num]

CHANGE_FUELPW = 0 ----------------|

|AND -| FUEL_SUM = FUEL_SUM

PCOMP_ENA = 1 --------------------|     |    + PCOMP_LBM

END: FUEL_MAIN_OFF

8-194

FUEL STRATEGY, FUEL SERVICE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.28  FUEL SERVICE LOGIC (CDAI0)

OVERVIEW

This module contains the control strategy necessary to request the output  of

fuel  injection  events at the desired injector timing.  In consists of entry

logic and fuel scheduling control logic.  This module operates in conjunction

with the IBETA calculation module, and the fuel scheduling module.

INTENT

The primary purpose of the fuel service module is to deliver fuel  pulses  at

the   appropriate  requested  timing.   There  are  several  basic  modes  of

operation:

1.  Crank fuel - When in crank mode, all injectors are fired  simultaneously.

The  amount  of  fuel on each pulse, the interval between pulses, and the

PIP edge  on  which  crank  fuel  is  delivered  are  all  determined  by

calibration constants.

2.  Unsynchronized  fuel  -  If  the  strategy   is   unable   to   recognize

synchronization   before   leaving   crank   mode,  injectors  are  fired

simultaneously every NUMCYL number of PIPs.  If synchronization has still

not  been  detected after a specified amount of time is spent in run mode

(determined by the calibration constant MAX_RM_SIMUL), operation  reverts

to unsynchronized sequential mode.

3.  Transition from simultaneous to synchronized sequential fuel -  When  the

CID (or signature PIP) is recognized, the actual engine position is known

to the fuel strategy.  The  flag  SYNFLG  is  set,  to  communicate  this

recognition.

When out of crank mode and SYNFLG is set, the first sequential fuel event

is  scheduled.   The flag FIRST_SEQNTL indicates that this event has been

scheduled.  The injector serviced by the first event is the  one  at  the

top   dead  center  of  its  intake  stroke.   This  is  accomplished  by

initializing INJ_BANK1 and INJ_BANK2 to select the cylinder whose  firing

order  sequence number is the current value of SYNC_CTR + NUMCYL/2 (using

modulo NUMCYL arithmetic).  TOTAL_DELAY1 and TOTAL_DELAY2 are initialized

to  NUMCYL/2  to  reflect  the placement of the first pulse, and the flag

FUEL_IN_SYNC is set to allow new TOTAL_DELAY values to be calculated.

4.  Synchronized sequential fuel - Once initial synchronization is  attained,

TOTAL_DELAYn  changes  are  clipped  to  1.0  per fuel event.  Changes to

TOTAL_DELAYn  of  more  than  one  PIP  cause  immediate  adjustments  to

INJ_PIP_CNTn,  the  number of PIP edges until the fuel injection event is

scheduled on bank n.  The IBETA calculation logic makes small adjustments

to IBETAn and INJ_PIP_CNTn to keep the fuel synchronized to the requested

TOTAL_DELAYn value.

8-195

FUEL STRATEGY, FUEL SERVICE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  ATMR3 = Time since entering RUN mode, seconds.

-  CRK_PIP_INJ = Number of PIP periods per injector firing.

-  FUELPW1 = HEGO11 injector pulsewidths, clock ticks.

-  FUELPW2 = HEGO21 injector pulsewidths, clock ticks.

-  INJ_BANK1 = Bank 1 injector pointer, in terms of  firing  order  sequence

(modulo NUMCYL).

-  INJ_BANK2 = Bank 2 injector pointer, in terms of  firing  order  sequence

(modulo NUMCYL).

-  INJ_PIP_CNT1 = Injector PIP counter for bank 1, which tracks  the  number

of PIP edges until an injector in bank 1 is due to be serviced.

-  INJ_PIP_CNT2 = Injector PIP counter for bank 2, which tracks  the  number

of PIP edges until an injector in bank 2 is due to be serviced.

-  SYNC_CTR = Counter which counts cylinders from the PIP signal and  resets

when equal to the number of cylinders in the engine.

Bit Flags:

-  CRKFLG = Crank flag; 1 -> engine in crank mode.

-  FIRST_PIP = Flag indicating whether the first low PIP has been  received;

1 -> first low PIP has been received.

-  FIRST_SEQNTL = Flag which indicates  whether  sequential  mode  has  been

entered; 1 -> the first sequential event has occurred.

-  PIP_HIGH = Indicates that the most recent PIP transition was low to high.

-  SYNFLG = Indicates when number one cylinder has been identified  and  the

SYNC_CTR has been aligned with number one cylinder.

Calibration Constants:

-  EDSEL = Switch for selective crank fuel timing at falling-edge of PIP.

-  FN1327 = Fuel pulsewidth register map;,  used  to  determine  which  fuel

register is used.

X = Injector output number.

Y = Null.

-  FN1329 = Injector firing order used for correcting INJ_BANKn  to  correct

the firing order.

X = INJ_BANKn.

Y = Null.

8-196

FUEL STRATEGY, FUEL SERVICE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  MAX_RM_SIMUL = Maximum amount of time that simultaneous  fuel  scheduling

may be used after entering run mode.

-  NUMCYL = Number of cylinders in the engine.

-  NUMEGO = Number of EGO sensors present, either stereo or mono.

OUTPUTS

Registers:

-  IBETA1 = The injector delay time in PIP deltas, bank 1.  This value is to

be used by the next injection event on bank 1.

-  IBETA2 = The injector delay time in PIP deltas, bank 2.  This value is to

be used by the next injection event on bank 2.

-  INJ_BANK1 = See above.

-  INJ_BANK2 = See above.

-  INJ_PIP_CNT1 = See above.

-  INJ_PIP_CNT2 = See above.

Bit Flags:

-  CRK_PIP_INT = CRK_PIP_INJ initialization indicator; 1 -> CRK_PIP_INJ  has

been initialized.

-  FIRST_SEQNTL = See above.

-  FUEL_IN_SYNC = Fuel synchronized with PIP.

-  SIMULTANEOUS = Simultaneous injection in progress flag.

TEMPORARY REGISTERS

-  bank_num = The EGO bank number currently being processed, either  bank  1

or bank 2.

-  lcl_inj_bank = A temporary holding register to retain the  current  value

of INJ_BANK1 during simultaneous injector scheduling.

-  mainpw_inj_num = The injector number of the injector for which  the  main

fuel pulsewidth is calculated.

-  mainpw_bank_indicator = A numeric indicator of the engine bank for  which

the  main fuel pulsewidth is calculated.  NOTE:  This is not equal to the

engine bank number, but rather a value of 0 corresponds to engine bank 1,

and a value of 2 corresponds to engine bank 2.

8-197

FUEL STRATEGY, FUEL SERVICE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

STRATEGY ROUTINES

Entry Points:

-  FUEL_SERVICE = Main entry point for the fuel service logic.

Local Routines:

-  fuel_service_entry_logic = The front-end of the fuel service module.

-  fuel_service_exit_check = Routine used to determine  whether  to  execute

the fuel service module for a second EGO bank.

-  sequential_fuel = Routine  to  handle  sequential  fuel  scheduling,  the

normal mode of operation.

-  simultaneous_fuel = Routine to handle simultaneous fuel scheduling, which

applies only during crank mode.

-  simultaneous_fuel_scheduling   =   This   routine    is    called    from

simultaneous_fuel,  and  will  invoke the fuel scheduling routine for all

injectors in the engine.

External Routines:

-  FUEL_IBETA_CALC = Performs the computation of IBETA1 and IBETA2;  located

in strategy module FUEL_IBETA_CALC_COMx.

-  FUEL_SCHEDULE =  Routine  to  schedule  the  main  fuel  pulse  for  each

injection event; located in strategy module FUEL_SCHEDULE_COMx.

8-198

FUEL STRATEGY, FUEL SERVICE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_SERVICE_COM1

(Performed in the FUEL_SERVICE software module)

BEGIN: FUEL_SERVICE

;Initialize the fuel service bank index, and execute the entry logic for

;EGO bank 1.  Subsequent calls to the routine fuel_service_exit_check will

;determine if the fuel service routine must also be performed on EGO

;bank 2.

unconditionally ------------------------| bank_num := 1

| DO: fuel_service_entry_logic

END: FUEL_SERVICE

BEGIN: fuel_service_entry_logic

;Determine whether to use simultaneous or sequential fuel scheduling.

;Use simultaneous fuel scheduling of in crank mode.  Also use

;simultaneous scheduling if not synchronized, not already using

;sequential scheduling, and in run mode for less than MAX_RM_SIMUL

;seconds.

CRKFLG = 1 -----------------------|

|

SYNFLG = 0 -----------------|     |OR --| SIMULTANEOUS := 1

;Not in sync               |     |     | FIRST_SEQNTL := 0

|     |     | FUEL_IN_SYNC := 0

FIRST_SEQNTL = 0 -----------|AND -|     | IBETA2 := 0

;Not in sequential mode    |           | DO: simultaneous_fuel

|           |

ATMR3 < MAX_RM_SIMUL -------|           |

| --- ELSE ---

|

| SIMULTANEOUS := 0

| DO: sequential_fuel

END: fuel_service_entry_logic

8-199

FUEL STRATEGY, FUEL SERVICE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: simultaneous_fuel

;Injectors are fired simultaneously in crank mode, or in underspeed or run

;mode for up to MAX_RM_SIMUL seconds if sync has not been recognized.

INJ_PIP_CNT1 > 1 -----------------------| INJ_PIP_CNT1 := INJ_PIP_CNT1 - 1

| INJ_PIP_CNT2 := INJ_PIP_CNT2 - 1

| Exit simultaneous_fuel

|

| --- ELSE ---

|

CRKFLG = 1 -----------------------|     |

|AND -| INJ_PIP_CNT1 := 1

FIRST_PIP = 0 --------------------|     | INJ_PIP_CNT2 := 1

| Exit simultaneous_fuel

|  ;Wait for next PIP edge

|

| --- ELSE ---

|

CRKFLG = 1 -----------------------|     |

|     |

EDSEL = 0 ------------|           |     |

|AND -|     |AND -| INJ_PIP_CNT1 := 1

PIP_HIGH = 1 ---------|     |     |     | INJ_PIP_CNT2 := 1

|OR --|     | CRK_PIP_INT := 1

EDSEL = 1 ------------|     |           | Exit simultaneous_fuel

|AND -|           |  ;Wait for next PIP edge

PIP_HIGH = 0 ---------|                 |

| --- ELSE ---

|

CRKFLG = 1 -----------------------------| INJ_PIP_CNT1 := CRK_PIP_INJ

| INJ_PIP_CNT2 := CRK_PIP_INJ

| CRK_PIP_INT := 1

| DO: simultaneous_fuel_scheduling

|

| --- ELSE ---

|

PIP_HIGH = 1 ---------------------------| INJ_PIP_CNT1 := NUMCYL * 2

;Not in crank mode - use               | INJ_PIP_CNT2 := NUMCYL * 2

; simultaneous scheduling              | DO: simultaneous_fuel_scheduling

; each NUMCYL PIPs                     |

| --- ELSE ---

|

PIP_HIGH = 0 ---------------------------| INJ_PIP_CNT1 := NUMCYL * 2 - 1

;Adjust so next event                  | INJ_PIP_CNT2 := NUMCYL * 2 - 1

; occurs on high PIP                   | DO: simultaneous_fuel_scheduling

END: simultaneous_fuel

8-200

FUEL STRATEGY, FUEL SERVICE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: simultaneous_fuel_scheduling

;Schedule all injectors for simultaneous firing.  For each injector,

; set INJ_BANK1 to the appropriate value and invoke the fuel schedule

;module.  The Duce and watchdog timer must be serviced upon scheduling

;every-other injector.

unconditionally ------------------------| lcl_inj_bank := INJ_BANK1

| IBETA1 := 0

| IBETA2 := 0

| INJ_BANK1 := 0

while INJ_BANK1 < NUMCYL ---------------| DO: FUEL_SCHEDULE

|  ;Service Duce HLOS and watchdog

|  ; after each even sequence

|  ; numbered injector

| INJ_BANK1 := INJ_BANK1 + 1

unconditionally ------------------------| INJ_BANK1 := lcl_inj_bank

END: simultaneous_fuel_scheduling

8-201

FUEL STRATEGY, FUEL SERVICE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: sequential_fuel

;Schedule only the current injector for sequential firing.

INJ_PIP_CNT[bank_num] > 1 --------------| INJ_PIP_CNT[bank_num] :=

|   INJ_PIP_CNT[bank_num] - 1

| DO: fuel_service_exit_check

|

| --- ELSE ---

|

FIRST_SEQNTL = 0 -----------------|     |

|AND -| INJ_BANK1 := SYNC_CTR

FUELPW[bank_num] <> 0 ------------|     |      - NUMCYL/2 - 1

| INJ_BANK2 := SYNC_CTR

|      - NUMCYL/2 - 1

| INJ_BANK1 := MOD(INJ_BANK1,NUMCYL)

| INJ_BANK2 := MOD(INJ_BANK2,NUMCYL)

| INJ_PIP_CNT[bank_num] :=

|      INJ_PIP_CNT[bank_num] + 1

| FIRST_SEQNTL := 1

|

| --- ELSE ---

|

FIRST_SEQNTL = 0 -----------------------| INJ_PIP_CNT1 := 1

;FUELPWn is zero                       | INJ_PIP_CNT2 := 1

| Exit sequential_fuel

unconditionally ------------------------| INJ_BANK[bank_num] :=

|   INJ_BANK[bank_num] + 1

| INJ_BANK[bank_num] :=

|   MOD(INJ_BANK[bank_num],NUMCYL)

| mainpw_inj_num :=

|   FN1329(INJ_BANK[bank_num],0)

| mainpw_bank_indicator :=

|   FN1327(mainpw_inj_num,0)

8-202

FUEL STRATEGY, FUEL SERVICE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;The index, bank_num, of the bank being processed must match the bank

;indicator for the injector.  If not, exit without servicing the injector.

;It must only be serviced using its own banks TOTAL_DELAYn, IBETAn etc.

;For a stereo ego system, half the time the injector will be on the wrong

;bank, because each INJ_BANKn increments through all the injectors.  The

;FN1327 run time check is the only way to tell which injectors are on which

;bank.

mainpw_bank_indicator = 0 --|

|AND -|

bank_num = 1 ---------------|     |

|OR --| DO: FUEL_IBETA_CALC

mainpw_bank_indicator = 2 --|     |     | DO: FUEL_SCHEDULE

|AND -|     | DO: fuel_service_exit_check

bank_num = 2 ---------------|           |

|

| --- ELSE ---

|

| DO: fuel_service_exit_check

END: sequential_fuel

BEGIN: fuel_service_exit_check

;Repeat fuel service routine for second EGO bank, if necessary

bank_num = 1 ---------------------|

|AND -| bank_num := 2

NUMEGO = 2 -----------------------|     | DO: fuel_service_entry_logic

|

| --- ELSE ---

|

| Exit fuel_service_exit_check

END: fuel_service_exit_check

8-203

FUEL STRATEGY, FOREGROUND INJECTOR TIMING - IBETA CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.29  FOREGROUND INJECTOR TIMING - IBETA CALCULATION (CDAI0)

OVERVIEW

Fuel is synchronized to engine events so that an injection event for a  given

cylinder  is  started some amount of engine rotation after top dead center of

the PIP on its compression stroke 10 degrees before its power  stroke.   This

PIP  is  the  identifying PIP for the cylinder.  Once SYNFLG is set, SYNC_CTR

tracks which cylinder is identified by the last high  PIP.   TOTAL_DELAYn  is

the  number  of  whole  and  fractional  PIPs from the identifying PIP to the

desired start of the injection event for that cylinder on ego  bank  n.   The

most  common  reason  why  TOTAL_DELAY1  may differ from TOTAL_DELAY2 is that

FUELPW1 and FUELPW2 are different, and that off-edge timing has been selected

by calibrating INJREF = 1.

IBETAn is an offset from the LAST high PIP to the start of the next injection

event  on bank n.  IBETAn has an allowable range of 0 - 1.5 PIPs.  If the new

IBETAn falls  out  of  the  range  0.5  -  1.5,  an  adjustment  is  made  to

INJ_PIP_CNTn  to  lengthen  or  shorten  the  number of PIP edges to the next

injection event on the bank to  bring  IBETAn  back  into  this  range.   The

TOTAL_DELAYn  calculation  is performed at each FUELPWn calculation.  It will

attempt  to  keep  IBETAn  in  range  by  adjusting   INJ_PIP_CNTn   whenever

TOTAL_DELAYn changes by 1/2 PIP or more.

APPROACH

To calculate a new IBETAn, it is essential to know which cylinder's  injector

is to be serviced next.  INJ_BANKn tracks the injectors on a bank in terms of

firing order sequence.  Each INJ_BANKn counts from 0 to NUMCYL - 1, as if the

engine  only has one bank, incrementing each time an injection event would be

due if all cylinders were on the bank.  In fact, for stereo ego systems, half

the  cylinders are on the other bank so no actual injection event occurs, but

this method of accounting is useful, because it is not known which  cylinders

are  actually  on which bank until the firing sequence number is converted to

an injector number via FN1329, and the actual bank of the injector is  looked

up  in  FN1327.   The  INJ_BANKn  mechanism  assures  that  all injectors are

serviced in order, regardless of abrupt timing changes.

The new IBETAn, in PIPs from last high PIP, is determined by the equation

IBETAn = INJ_BANKn + TOTAL_DELAYn - SYNC_CTR    (modulo NUMCYL)

where INJ_BANKn is the cylinder to be serviced, TOTAL_DELAYn is the delay  in

PIPs  after TDC PIP, and SYNC_CTR is the cylinder for which the last high PIP

is TDC PIP.

This equation may also be expressed as

IBETAn = TOTAL_DELAYn - (SYNC_CTR - INJ_BANKn)    (modulo NUMCYL)

8-204

FUEL STRATEGY, FOREGROUND INJECTOR TIMING - IBETA CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Pictorially,  the  relationship  between  TOTAL_DELAYn,   SYNC_CTR,   IBETAn,

INJ_BANKn,  and  the fuel pulse is shown below.  The example shown is for the

number 3 cylinder in the firing order with a TOTAL_DELAYn = 2.7.   The  IBETA

value will be calculated to be 0.7.

TDC for cylinder 3

\

SYNC_CTR  |-----2-----|-----3-----|-----4-----|-----5-----|-----6-----|-----

INJ_BANKn |-----0-----|-----1-----|-----2-----|-----3-----|-----4-----|-----

+-----+     +-----+     +-----+     +-----+     +-----+     +----

PIP       |     |     |     |     |     |     |     |     |     |     |

-----+     +-----+     +-----+     +-----+     +-----+     +-----+

|                       |        |

|<--------TOTAL_DELAYn---------->|

|                       |        |

|_(SYNC_CTR-INJ_BANKn)__|_IBETAn_|

\        \

\    Fuel pulse turns on here

\

Fuel pulse is scheduled here

This module contains the calculation of IBETA.  Injections are  scheduled  on

either PIP edge, based on the previous IBETAn and INJ_PIP_CNTn calculations.

8-205

FUEL STRATEGY, FOREGROUND INJECTOR TIMING - IBETA CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  INJ_BANK1 = Bank 1 injector pointer, in terms of  firing  order  sequence

(modulo NUMCYL).  This logic assumes all injectors are on bank 1.

-  INJ_BANK2 = Bank 2 injector pointer, in terms of firing  order  sequence.

(modulo NUMCYL).  This logic assumes all injectors are on bank 2.

-  INJ_PIP_CNT1 = Number of PIP  edges  until  an  injector  is  due  to  be

serviced on Bank 1.  This logic assumes all injectors are on bank 1.

-  INJ_PIP_CNT2 = Number of PIP  edges  until  an  injector  is  due  to  be

serviced on Bank 2.  This logic assumes all injectors are on bank 2.

-  SYNC_CTR = Counter of  cylinder  events;  reset  upon  identification  of

cylinder one, counts up to NUMCYL - 1.

-  TOTAL_DELAYn = Delay from PIP near TDC on compression stroke until  start

of injection for cylinders on bank n.

Bit Flags:

-  FIRST_SEQNTL  =  Flag  which  indicates  whether  sequential  mode   fuel

scheduling  has  been  entered;  1  ->  the  first  sequential  event has

occurred.

-  FUEL_IN_SYNC = Fuel synchronized with PIP.

-  PIP_HIGH = Indicates that the most recent PIP transition was low to high.

-  SYNFLG = Indicates when number one cylinder has been identified  and  the

SYNC_CTR has been aligned with number one cylinder.

Calibration Constants:

-  NUMCYL = Number of cylinders in the engine.

OUTPUTS

Registers:

-  IBETA1 = The injector delay time in PIP deltas, bank 1.  This value is to

be used by the next injection event on bank 1.

-  IBETA2 = The injector delay time in PIP deltas, bank 1.  This value is to

be used by the next injection event on bank 1.

-  INJ_PIP_CNT1 = See above.

-  INJ_PIP_CNT2 = See above.

8-206

FUEL STRATEGY, FOREGROUND INJECTOR TIMING - IBETA CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TOTAL_DELAY1 = See above.

-  TOTAL_DELAY2 = See above.

Bit Flags:

-  FUEL_IN_SYNC = See above.

TEMPORARY REGISTERS

-  bank_num = The EGO bank number currently being processed, either  bank  1

or bank 2.

-  lcl_ibeta = A temporary holding register  to  hold  the  newly-calculated

IBETAn  until the current injector output is scheduled, to ensure the use

of the previous data for the current event.

STRATEGY MODULES

Entry Points:

-  FUEL_IBETA_CALC = The main entry point for the IBETA calculation module.

8-207

FUEL STRATEGY, FOREGROUND INJECTOR TIMING - IBETA CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_IBETA_CALC_COM1

BEGIN: FUEL_IBETA_CALC

;This module is called from the FUEL_SERVICE strategy module.

FUEL_IN_SYNC = 0 -----------------|

|

FIRST_SEQNTL = 1 -----------------|AND -| FUEL_IN_SYNC := 1

|     | TOTAL_DELAY1 := SYNC_CTR

SYNFLG = 1 -----------------------|     |     - INJ_BANK[bank_num]

| TOTAL_DELAY2 := TOTAL_DELAY1

| lcl_ibeta := 0

|

| --- ELSE ---

|

FIRST_SEQNTL = 1 -----------------|     |

|AND -| lcl_ibeta := INJ_BANK[bank_num]

SYNFLG = 1 -----------------------|     |   + TOTAL_DELAY[bank_num]

|   - SYNC_CTR

| lcl_ibeta := MOD(lcl_ibeta,NUMCYL)

|

| --- ELSE ---

|

| IBETA[bank_num] := 0.0

| Exit FUEL_IBETA_CALC

8-208

FUEL STRATEGY, FOREGROUND INJECTOR TIMING - IBETA CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;If more than 1 engine revolution error, assume timing is actually too late

lcl_ibeta > NUMCYL/2 -------------------| lcl_ibeta := 0.0

lcl_ibeta > 1.5 ------------------------| IBETA[bank_num] := 1.5

;Too early, adjust next                | INJ_PIP_CNT[bank_num] :=

;event timing                          |   INJ_PIP_CNT[bank_num] + 2

|  ;Wait longer before next injector

|

| --- ELSE ---

|

lcl_ibeta >= 1.0 -----------------|     |

|AND -| IBETA[bank_num] := lcl_ibeta

PIP_HIGH = 1 ---------------------|     | INJ_PIP_CNT[bank_num] :=

;On high PIP edge                      |   INJ_PIP_CNT[bank_num] + 1

|  ;Schedule next pulse on low PIP

|  ; if 1.0 <= lcl_ibeta <=1.5 for

|  ; 0.5 to 1.0 PIP lead time

|

| --- ELSE ---

|

0.5 < lcl_ibeta < 1.0 ------------|     |

|AND -| IBETA[bank_num] := lcl_ibeta

PIP_HIGH = 0 ---------------------|     | INJ_PIP_CNT[bank_num] :=

;On Low PIP edge                       |   INJ_PIP_CNT[bank_num] - 1

|  ;Service next event on high PIP

|  ; for 0.5 to 1.0 PIP lead time

|

| --- ELSE ---

|

lcl_ibeta < 0.5 ------------------------| IBETA[bank_num] := lcl_ibeta

;Too late, schedule next               | INJ_PIP_CNT[bank_num] :=

; event one PIP edge sooner            |   INJ_PIP_CNT[bank_num] - 1

|

| --- ELSE ---

|

| IBETA[bank_num] := lcl_ibeta

END: FUEL_IBETA_CALC

8-209

FUEL STRATEGY, FUEL SCHEDULING LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.30  FUEL SCHEDULING LOGIC (CDAM0)

OVERVIEW

This module is executed for each main fuel pulse.  Based on the  engine  bank

for  which  the  main  fuel  pulse was calculated, this module determines the

injector for which the main pulse would normally be scheduled in the  absence

of any injector cut-out requirements.

Then, based on the current status of the various injector cut-out conditions,

the module determines whether the main pulse is actually to be delivered.  If

the main pulse is to be output, then this module  directs  the  main  pulse's

on-edge to be scheduled for the correct injector.

Note:   Parameters  whose  alphabetic  characters  are  in  lower  case   are

non-displayable temporary registers.

DEFINITIONS

Registers:

-  CRK_FUL = Fuel pulsewidth used in crank mode, in seconds.

-  DT12S = The most recent PIP Period, i.e.,  the  elapsed  time,  in  clock

ticks,  between  the  most  recent  rising edge of the PIP signal and the

rising edge immediately prior to that one.

-  DT23S = The previous value of DT12S, i.e., the value of  DT12S  prior  to

the most recent rising edge of the PIP signal.

-  FUELPW1 = HEGO11 injector pulsewidths, in clock ticks.

-  FUELPW2 = HEGO21 injector pulsewidths, in clock ticks.

-  INJ_BANK1 = The firing sequence number of the SEFI injector  in  cylinder

Bank 1 for which the next main fuel pulse for that bank would normally be

scheduled.

-  INJ_BANK2 = The firing sequence number of the SEFI injector  in  cylinder

Bank 2 for which the next main fuel pulse for that bank would normally be

scheduled.

-  INJOFF = A word whose bits are used as flags that are associated with the

fuel injectors.  A SET flag indicates that its associated injector is not

to be fired.  The bit that is identified is the  flag  whose  bit  number

equals  the  value  of the indexing variable (where the bit number of the

least significant bit is zero).

-  INJ_ACTUAL = A word whose bits represent disabled cylinder events is used

to track when cylinder actually are turned off in the forground.

8-210

FUEL STRATEGY, FUEL SCHEDULING LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  LST_PW[inj_num] = The main fuel  pulsewidth  (in  ticks)  (not  including

PWOFF)  most  recently scheduled for the injector whose 'injector number'

is inj_num.  (It is one  of  the  registers  in  an  array  of  registers

composed  of  one  register for each SEFI injector on the engine.) (NOTE:

This table is used exclusively by the Dynamic Fuel strategy to  determine

whether the currently calculated main fuel pulsewidth is greater than the

pulsewidth that was previously calculated and scheduled as the main pulse

to the cylinder that is currently on its intake stroke.)

-  PIPLMT_HIG = The PIP Period value (in clock ticks)  corresponding  to  an

engine  RPM  above  which  the  main  pulsewidth  is  not to be delivered

(regardless of the injector for which it would  normally  be  scheduled).

(Note:   An  engine  RPM  ABOVE  the  RPM breakpoint corresponds to a PIP

Period value LESS than PIPLMT_HIG.

Bit Flags:

-  CRKFLG = Crank flag.

-  FLG_THFT_FCO =  Flag  signaling  when  fuel  is  being  cut  out  due  to

anti-theft system action; 1 -> cutting out fuel.

-  NO_FUEL_SCP = SCP logic  indicates  that  injectors  should  be  disabled

(Latched bit); 1 -> DO NOT deliver any fuel to the engine.

Calibration Constants:

-  FN1329 = A single-row table that  maps  an  injector's  'firing  sequence

number'  to  its  'injector  number'.  Arranged in firing sequence number

order.

X input  =  'firing sequence number'

Y input  =  Null

Output  =  'injector number'

-  SW_ANTI_THFT = Calibration switch to enable the  EEC  controlled  vehicle

anti-theft strategy.

TEMPORARY REGISTERS

-  mainpw_inj_num = The injector  number  of  the  injector  for  which  the

subject  main  fuel  pulse  would  normally  be  scheduled.   For  a SEFI

injector, it identifies the location in  the  engine  of  the  injector's

corresponding  cylinder.   (Note:  An injector's 'injector number' is NOT

the same as its 'firing sequence number'.)

-  mainpw_seq_num = The firing sequence number of the injector for which the

subject  MAIN  fuel  Pulse  would  normally  be  scheduled.   (Note:   An

injector's 'firing sequence number' is NOT  the  same  as  its  'injector

number'.)

8-211

FUEL STRATEGY, FUEL SCHEDULING LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_SCHEDULE_COM13

BEGIN: FUEL_SCHEDULE

; This module is invoked from within the FUEL_SERVICE strategy module.

; This strategy module corresponds to the FUEL_OUTPUT code segment in

; the FUEL_SERVICE software module.

unconditionally ------------------------| mainpw_seq_num :=

|       INJ_BANK[bank_num]

| mainpw_inj_num :=

|       FN1329[mainpw_seq_num,0]

8-212

FUEL STRATEGY, FUEL SCHEDULING LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Do not allow fuel pulsewidth delivery to injector  #"mainpw_inj_num"  if  bit

#"mainpw_seq_num"  in  INJOFF  is set.  (Note:  The FIRING SEQUENCE number of

injector #"mainpw_inj_num" is "mainpw_seq_num".) Also, check  for  an  engine

overspeed  condition, an SCP-commanded cut-out, and activation of the Passive

Anti-Theft System.

INJOFF[mainpw_seq_num] = 1 -------|

;Cut this injector               |

|

DT12S < PIPLMT_HIG ---------|     |

|AND -|

DT23S < PIPLMT_HIG ---------|     |

;RPM too high                    |

|

FLG_THFT_FCO = 1 -----------|     |

|AND -|OR --| ;Do NOT schedule the

SW_ANTI_THFT = 1 -----------|     |     | ; main fuel pulse's on-edge

;PATS is cal'd in and active     |     | ;STORE the current clock time

|     | ; as the previous off-edge time

CRKFLG = 1 -----------------|     |     |

|AND -|     | LST_PW[mainpw_inj_num] := ^FFFF

CRK_FUL = 0 ----------------|     |     | ;Mislead the dynamic fuel

|     | ; strategy to prevent it from

CRKFLG = 0 -----------------|     |     | ; subsequently scheduling a

|AND -|     | ; fuel pulse for that injector

FUELPW[bank_num] = 0 -------|     |     |

|     | INJ_ACTUAL =

NO_FUEL_SCP = 1 ------------------|     |    (INJ_ACTUAL + INJ_ACTUAL) + 1

| ;Rotate a bit into inj_actual,

| ; allowing high bit to overflow

|

| --- ELSE ---

|

| INJ_ACTUAL =

|      INJ_ACTUAL + INJ_ACTUAL

| ;rotate inj_actual left one bit.

|

| Call: FUEL_INJ_OUT

| ;SCHEDULE the main fuel pulse's

| ; on-edge for injector number

| ; mainpw_inj_num

END: FUEL_SCHEDULE

8-213

FUEL STRATEGY, INJECTOR OUTPUT CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.31  INJECTOR OUTPUT CONTROL (CDAI0)

OVERVIEW

This module should be invoked when the on-edge of a new injector pulse is  to

be  scheduled.  The edge will be scheduled, by issuing an appropriate command

to the high-speed output (HSO) hardware.  This will  result  in  the  on-edge

time  being  entered  into  a  carousel slot which has been reserved for this

injector.  This command will also direct the  HSO  hardware  to  generate  an

interrupt  when  this on-edge is issued.  The interrupt service routine, when

invoked, will generate a call to the  FUEL_MAIN_OFF  strategy  module,  which

will  schedule the off-edge for this pulse (i.e., load the off-edge time into

the carousel slot reserved for this injector).

In the case of short pulsewidths (i.e., less  than  300  ticks),  it  becomes

necessary to schedule both the on edge and the off edge at the same time.  In

this case, the interrupt described above is not enabled for either edge.

This strategy module, in conjunction with other foreground fuel  modules,  is

intended to satisfy the following requirements:

1.  The injector pulse start should be based on pulsewidth and timing  values

calculated from the most recent input data (PIP, LOAD, lambda, etc.).

2.  The injector timing value is used  only  to  start  the  injector  pulse,

regardless of which edge is used as the reference.

3.  Pulsewidth accuracy has top priority after an injector pulse is started.

4.  If possible, the pulsewidth should be updated while in progress  to  take

advantage of the most recent input data.  That is, the injector pulse can

end early or extend later as conditions change.

CALIBRATION NOTES

Injector Output Load:

On stereo HEGO applications, each injector  output  must  be  linked  to  the

proper  HEGO  sensor.   FN1327  is a 1x9 table that is used to match injector

output to HEGO sensor.  Each slot in the table corresponds to  an  individual

injector  output.   The  table  value  is an address offset to select between

FUELPW1 and FUELPW2.

If FN1327(i,0) = 0, then use FUELPW1 for injector i.

If FN1327(i,0) = 2, then use FUELPW2 for injector i.

For this register addressing scheme, FUELPW1 and FUELPW2 must be  consecutive

word registers and FUELPW1 must be first.

The FN1327 table reserves space to control up to 8 injector outputs (the zero

slot  is never referenced).  The actual number is determined by NUMCYL, which

is the number of cylinders in the engine.   The  user  must  fill  the  table

locations  to  match  NUMCYL.   The  FN1327  table must be calibrated for all

8-214

FUEL STRATEGY, INJECTOR OUTPUT CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

applications, regardless of the system (stereo- or mono-HEGO).  NOTE that  if

NUMEGO  =  1 (mono-HEGO), all values in FN1327 must be zero (corresponding to

FUELPW1), or no fuel will be sent to bank 2.

5.0L Injector Output Load Examples:

SEFI                  SEFI

STEREO EGO            MONO-EGO

NUMCYL=8,NUMEGO=2     NUMCYL=8,NUMEGO=1

TABLE    INJECTOR    ------------------    ------------------

SLOT     OUTPUT     FN1327 PULSEWIDTH     FN1327 PULSEWIDTH

------   --------    ------ -----------    ------ -----------

0      NOT USED       0   FILLER            0   FILLER

1      INJ-1          0   FUELPW2           0   FUELPW1

2      INJ-2          0   FUELPW2           0   FUELPW1

3      INJ-3          0   FUELPW2           0   FUELPW1

4      INJ-4          0   FUELPW2           0   FUELPW1

5      INJ-5          2   FUELPW1           0   FUELPW1

6      INJ-6          2   FUELPW1           0   FUELPW1

7      INJ-7          2   FUELPW1           0   FUELPW1

8      INJ-8          2   FUELPW1           0   FUELPW1

Injector Firing Order:

On SEFI applications, each injector output is linked to it's matching  engine

cylinder.   The injector output number is the cylinder number.  The injectors

must be fired in the proper order to achieve sequential operation.

FN1329 is a 1x8 table which is used to control  the  injector  firing  order.

The table contains the firing order of the engine.

5.0L INJECTOR FIRING ORDER EXAMPLE

SEFI

FN1329        NUMCYL = 8

TABLE       ---------------

SLOT        FN1329  OUTPUT

------      ------ --------

0           1    INJ-1

1           5    INJ-5

2           4    INJ-4

3           2    INJ-2

4           6    INJ-6

5           3    INJ-3

6           7    INJ-7

7           8    INJ-8

8-215

FUEL STRATEGY, INJECTOR OUTPUT CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SOFTWARE NOTES

1.  Overview of 8065 injector output structure:

The 8065 carousel structure allows reserving carousel slots for explicit  use

by  certain  functions.   The injector output procedure utilizes one reserved

slot per injector output, which is typically used  for  scheduling  both  the

on-edge  and  the  off-edge of a pulse.  The advantage of this scheme is that

the location of all injector commands is readily discernible in  case  update

of a command time is required by the strategy.

The on edge is scheduled into the reserved slot for  the  injector,  and  the

load  command  requests  that  an interrupt be generated when this on edge is

issued.  At that time, the off edge for the pulse will be scheduled into  the

reserved  slot,  which has been vacated.  An exception to this scheme is used

when the pulsewidth is small (less than 300 ticks).  Both the on edge and the

off  edge  are scheduled at the same time; the on edge is scheduled using the

auto-load feature, while the off-edge is manually loaded  into  the  reserved

slot.

The 3 byte time of the last scheduled on and off transition is maintained  in

tables so time conflicts can be identified before a new command is issued.  A

table documenting the last FUELPW issued by an injector is maintained by  the

off-edge  scheduling procedure FUEL_MAIN_OFF.  It is used by the dynamic fuel

procedures to determine if a dynamic make-up pulse is required.  A  table  of

fuel  mass  delivered  in the last main fuel pulse is also maintained for the

benefit of the tripminder computer FUEL_SUM calculation.

The tables for injector last on and off times, the carousel slot to  be  used

by  and injector edge command, and the last pulsewidth and pounds mass tables

are all readily indexed by a common pointer based  on  the  injector  number.

The  injectors bank number can be looked up readily in FN1327 using this same

injector pointer  allowing  bank  specific  parameters  to  also  be  readily

derived.   This approach reduces the arguments which need to be passed to the

procedures to essentially only the injector number.  In  some  cases,  it  is

convenient  to  pass  additional  information  which  is already in temporary

registers due to previous calculations.

2.  Loading the carousel in manual mode:

The 8065 CPU provides two modes for loading command and time  data  into  the

carousel.   These  modes, automatic and manual, are selected via bits 6 and 7

in the CAM_SLOT_SELECT special  function  register  (SFR).   Manual  mode  is

selected  when  bit  6,  defined  in  strategies as MANUAL_MODE, is set to 1.

Automatic mode is selected whenever MANUAL_MODE is not  set.   This  is  most

commonly accomplished by clearing byte CAM_SLOT_SELECT.

In the automatic mode, as in the 8061, data is loaded into the next available

carousel slot as soon as the output command holding buffer, commonly known as

HSO_CMD, has a command written to it.  The CPU continuously steps through the

carousel  slots  completing  one  complete cycle each 32 machine states, or 1

tick.

Manual mode allows a command and time tag to be written to a  user  specified

slot  in the carousel.  A slot may be reserved to prevent automatic mode from

writing to it, i.e.  a reserved slot may only be loaded by writing to  it  in

manual mode.

To write to a slot in manual mode,  the  MANUAL_MODE  flag  must  be  set  in

8-216

FUEL STRATEGY, INJECTOR OUTPUT CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CAM_SLOT_SELECT.   Both  the  hardware and software manual recommend that the

target carousel slot number also should be specified  in  CAM_SLOT_SELECT  at

this  time,  although  the examples shown do not appear to require it.  Next,

the target time  and  command  values  are  written  to  the  holding  buffer

registers HSO_TIME and HSO_CMD.  Finally, CAM_SLOT_SELECT is written with the

slot number in bits 0-4, bit 5 cleared, and with both  MANUAL_MODE  (bit  6),

and MANUAL_WRITE (bit 7) set.

Data will not be  transferred  from  the  holding  buffer  registers  to  the

carousel  until  up  to 32 machine states after MANUAL_WRITE is set.  In both

manual and auto mode, care must be taken to ensure that data in  the  holding

buffer  is  not  overwritten  with new data before it is transferred into the

carousel.  Bit 7 in the IO_STATUS register, defined as OUTPUT_BUFFER_FULL  in

most  strategies, when set indicates that the output buffer is still occupied

with data awaiting loading into the carousel.  The  CAM_SLOT_SELECT  register

also must not be altered until the output buffer is vacated.

Finally, reserved slots  may  add  to  the  time  required  for  data  to  be

transferred  from the holding buffer to the carousel in automatic mode (up to

32 machine states, or 1 tick, after HSO_CMD is loaded).  In some  strategies,

an  additional  tick  may  be added to some immediate commands to prevent the

edges from going out 350 milliseconds late.

3.  Reserving a carousel slot:

A slot is reserved by loading it when the RESERVE_SLOT flag  (bit  7  of  SFR

CAM_SLOTS_AVBL)  is  set.   Manual mode must be used to specify which slot to

reserve.  A slot  may  not  be  unreserved,  once  reserved,  except  through

hardware  reset.   If  RESERVE_SLOT  is  set during auto load mode, whichever

available slot received the command will become reserved, thereby removing it

from  availability for further auto loads.  Ultimately, a deadlock may result

in which all slots are reserved, but the holding buffer remains full with  an

auto load command.

8-217

FUEL STRATEGY, INJECTOR OUTPUT CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  DT12S = The period between the last two PIP low-to-high  transitions,  in

clock ticks.

-  FUELPW1 = The HEGO11 injector pulsewidth, in clock ticks.

-  FUELPW2 = The HEGO21 injector pulsewidth, in clock ticks.

-  IBETA1 = The injector delay time in PIP deltas, bank 1.

-  IBETA2 = The injector delay time in PIP deltas, bank 2.

-  LAST_HI_PIP = Time of latest low-to-high transition, in clock ticks.

-  PWOFF = Injector pulsewidth offset, clock ticks.

OUTPUTS

Registers:

-  LAST_ON_INJ[i] = The time at which the most  recently  scheduled  on-edge

for injector i is to be issued.

TEMPORARY REGISTERS

-  bank_num = The EGO bank number currently being processed, either  bank  1

or 2.

-  mainpw_inj_num = The injector number of the injector for which  the  main

fuel pulsewidth is calculated.

-  off_edge_time[i] = Array containing the most recently scheduled  off-edge

time for injector i.  This value is written both to the RAM register pair

LAST_OFF_HI[i] and LAST_OFF_IN[i], and  to  the  HSO  carousel  when  the

off-edge is scheduled.

-  on_edge_time[i] = Similar to off_edge_time[i], but contains the  injector

on-edge  time.   This  value  is  written  both  to the RAM register pair

LAST_ON_HI[i] and LAST_ON_INJ[i],  and  to  the  HSO  carousel  when  the

on-edge is scheduled.

8-218

FUEL STRATEGY, INJECTOR OUTPUT CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_INJ_OUT_COM12

BEGIN: FUEL_INJ_OUT

; Called from within the FUEL_SCHEDULE strategy module.

; This strategy module correspond to the INJECTOR_EDGE routine

; in the INJECTOR_OUTPUT software module.

; Determine the injector on-edge time.

Unconditionally ----------------| i = mainpw_inj_num

| on_edge_time[i] = LAST_HI_PIP

|   + (IBETA[bank_num] * DT12S)

; Check that the on-edge time is in the future, and that there

; is sufficient time to load the carousel.

on_edge_time[i] < clock + 10 ---| on_edge_time[i] = clock + 10

| ;Start pulse in 10 clock ticks

; If an off-edge has been previously loaded for this injector, and has

; not yet been issued, determine whether the resulting two pulses will

: be distinct or combined.

off-edge scheduled for

mainpw_inj_num ---------|

|AND -| Cancel old off edge

on_edge_time[i] <         |     | LAST_ON_INJ[i] = on_edge_time[i]

off_edge_time[i] -------|     | Load new on-edge into carousel

|   using the auto-load feature

|

| --- ELSE ---

off-edge scheduled for          |

mainpw_inj_num ---------|     |

|AND -| LAST_ON_INJ[i] = on_edge_time[i]

on_edge_time[i] >=        |     |                + PWOFF

off_edge_time ----------|     | Load new on-edge into carousel

|   using the auto-load feature

; Enable an HSO interrupt, to occur when the on-edge is issued, or

; schedule the off-edge now for a short pulse.

FUELPW[bank_num] > 300 ---------| Enable the HSO interrupt for

|   injector mainpw_inj_num

|

| --- ELSE ---

|

| Do: FUEL_MAIN_OFF(i)

END: FUEL_INJ_OUT

8-219

FUEL STRATEGY, CALCULATE DYNAMIC FUEL CORRECTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.32  CALCULATE DYNAMIC FUEL CORRECTION (CDAI0)

OVERVIEW

The dynamic fuel correction is a fuel pulse that is injected  into  the  open

valve  of  the  cylinder  that  is currently on its intake stroke.  This fuel

pulse is the result of the difference between:  (1) the air  charge  used  in

the  main  fuel  pulsewidth  calculation  most  recently  performed  for that

cylinder, and (2) the air charge used in the current calculation of the  main

fuel  pulsewidth  (being performed for a cylinder that is later in the firing

sequence, and earlier in its cycle, than is the  cylinder  currently  on  its

intake stroke).

The dynamic fuel correction is limited to MINPW for a minimum; for a maximum,

it  is  limited  so  that  it will not extend past the point in time when the

cylinder will no longer induct the fuel injected on this intake event.

NOTE:  Parameters whose alphabetic characters  are  in  the  lower  case  are

non-displayable temporary registers.

DEFINITIONS

INPUTS

Registers:

-  CYL_AIR_CHG = Current cylinder air charge used in fuel calculation, lbm.

-  DYNPW = Dynamic fuel correction pulsewidth, clockticks.

-  ENABLE_DYNAM = Counter that enables the dynamic fuel make-up  pulse  four

engine revolutions after the strategy has identified number one cylinder.

-  FUELPWn = The previous injector  pulsewidth  (in  clock  ticks)  for  the

identified engine bank number (n); n = 1,2.

-  LST_PW(x) = The array of registers containing the  fuel  pulsewidths  (in

units  of  clockticks)  most recently delivered by each of the individual

injectors; indexed by 'injector number'.

-  MINPWT = Minimum allowed pulsewidth for requested fuel; ticks.

-  SYNC_CTR = Counter that counts occurrences of successive engine cylinders

being  at 10-deg BTDC, by incrementing at every PIP rising edge, and then

resets when equal to the number of cylinders in the engine.

Bit Flags:

-  SYNFLG  =  Flag  which  indicates  when  number  one  cylinder  has  been

identified  and  the  SYNC_CTR has been aligned with the occurance of PIP

for number one cylinder.

8-220

FUEL STRATEGY, CALCULATE DYNAMIC FUEL CORRECTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  AIR_DOWN_CAL = A switch that indicates when to perform the  cylinder  air

charge  calculation:   0  ->  perform  calculation  on  PIP up-edge; 1 ->

perform calculation on PIP down-edge.

-  DYN_MIN_ARC = Minimum air charge value below which dynamic  make-up  fuel

is  disabled.  This parameter should be used to prevent dynamic fuel upon

recoveries from decels.

-  ENGCYL = Number of intake strokes per engine revolution.

-  FN1327 = A single-row table that maps an injector's 'injector number'  to

a   numeric  indicator  of  the  engine  bank  in  which  the  injector's

corresponding cylinder is located.  (NOTE:  This numeric indicator is NOT

equal  to  the  engine  bank's  'bank number'; a table output of 0 (zero)

corresponds to engine bank #1; a table output of 2 corresponds to  engine

bank #2.)

X input  =  'injector number'

Y input  =  Null

table output  =  'engine bank indicator'

-  FN1329 = A single-row table that  maps  an  injector's  'firing  sequence

number'  to  its  'injector  number'.  Arranged in firing sequence number

order.

X input  =  'firing sequence number'

Y input  =  Null

Output  =  'injector number'

-  PWCAL = Value above which the ratio of the current fuel pulsewidth to the

previous main fuel pulsewidth will produce a dynamic fuel correction.

OUTPUTS

Registers:

-  DYNPW = See above.

-  ENABLE_DYNAM = See above.

8-221

FUEL STRATEGY, CALCULATE DYNAMIC FUEL CORRECTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TEMPORARY REGISTERS

-  current_time = The value in  the  'master  I/O  timer'  (a  h/w-dedicated

register);  unreferenced relative time (not a time interval referenced to

any  event(s));  rolls-over  after  attaining  maximum  possible   value;

increments  by  1 (one) every '(clock)tick', where a '(clock)tick' is the

elapsed  time  equal  to  the  product:    (period_of_CPU_machine_state)*

(the_number_of_slots_in_the_output_carousel).

-  dyn_on_edge = Time that the dynamic fuel pulse is to begin.

-  int_inj_num = The 'INJector NUMber' of the injector  whose  corresponding

cylinder  is  currently  on  its  INTake  stroke.  (NOTE:  This temporary

register, calculated in this module, is used  in  this  module  and  also

input  by other foreground fuel modules.) (Note:  An injector's 'injector

number' identifies the  location  in  the  engine  of  its  corresponding

cylinder; it is NOT the same as its 'firing sequence number'.)

-  int_seq_num  =  The  'firing  SEQuence  NUMber'  of  the  injector  whose

corresponding cylinder is currently on its INTake stroke.  Updated when a

new air charge is to be calculated.   (NOTE:   This  temporary  register,

calculated in this module, is used in this module and also input by other

foreground fuel modules.) (Note:  An injector's 'firing sequence  number'

is NOT the same as its 'injector number' which identifies the location in

the engine of the injector's corresponding cylinder.)

-  intcyl_bank_indicator = A numeric indicator of the engine bank  in  which

the  cylinder  currently  on  its intake stroke is located.  (Note:  This

numeric indicator is NOT equal to the  engine  bank's  'bank  number';  a

numeric  value of 0 (zero) corresponds to engine bank #1; a numeric value

of 2 corresponds to engine bank #2.)

-  off_edge_time =  Time  of  the  off-edge  of  the  main  fuel  pulsewidth

(previously  calculated) for the cylinder that is currently on its intake

stroke.

-  t_current_pw = Value of the currently calculated  main  fuel  pulsewidth;

ticks.

STRATEGY MODULES

Entry Points:

-  FUEL_DYN_CALC

External Routines:

-  FUEL_DYN_FIT

8-222

FUEL STRATEGY, CALCULATE DYNAMIC FUEL CORRECTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_DYN_CALC_COM3

BEGIN: FUEL_DYN_CALC

; Called from the FUEL_FG_PIPEDG strategy module; executed in the foreground.

; Determine if the dynamic fuel calculation should be performed.

SYNFLG = 0 -----------------------------| ENABLE_DYNAM = ENGCYL * 4

| Exit FUEL_DYN_CALC

|

| --- ELSE ---

SYNFLG = 1 -----------------------|     |

|AND -| ENABLE_DYNAM = ENABLE_DYNAM - 1

ENABLE_DYNAM <> 0 ----------------|     |   (decrement enabling counter)

| Exit FUEL_DYN_CALC

|

| --- ELSE ---

|

CYL_AIR_CHG < DYN_MIN_ARC --------------| Exit FUEL_DYN_CALC

; Next, identify the firing sequence number of the cylinder that is currently

; on its intake stroke.

ENGCYL = 4 ------------------------|

|AND -| int_seq_num = SYNC_CTR - ENGCYL

AIR_DOWN_CAL = 0 ------------------|     |

| --- ELSE ---

|

| int_seq_num =

|         SYNC_CTR - (ENGCYL - 1)

int_seq_num <= 0 ------------------------| int_seq_num =

|    int_seq_num + (ENGCYL * 2)

; Next, identify the engine bank that contains the cylinder that is currently

; on its intake stroke.

Unconditionally ------------------------| int_inj_num = FN1329(int_seq_num,0)

| intcyl_bank_indicator =

|     FN1327(int_inj_num,0)

8-223

FUEL STRATEGY, CALCULATE DYNAMIC FUEL CORRECTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

; Then obtain the latest pulsewidth calculated for the engine bank that

; contains the cylinder currently on its intake stroke.

intcyl_bank_indicator = 0 --------------| t_current_pw = FUELPW1

|

| --- ELSE ---

|

| t_current_pw = FUELPW2

; The dynamic fuel correction is calculated for the cylinder that is

; currently on the intake stroke of its cycle.

; Determine if there is a need for the dynamic pulse:

Unconditionally -------------------------| ratio_puls_width = t_current_pw

|        / LST_PW[int_inj_num]

| delta_puls_width = t_current_pw

|        - LST_PW[int_inj_num]

ratio_puls_width > PWCAL ----------|

|

delta_puls_width > 0 --------------|AND -| DYNPW = delta_puls_width

|     |  (need normal dynamic pulse)

delta_puls_width > MINPWT ---------|     |

| --- ELSE ---

ratio_puls_width > PWCAL ----------|     |

|AND -| DYNPW = MINPWT

delta_puls_width > 0 --------------|     |  (need minimum dyn PULSE)

|

| --- ELSE ---

|

| DYNPW = 0

|  (no pulse needed)

8-224

FUEL STRATEGY, CALCULATE DYNAMIC FUEL CORRECTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Determine if the dynamic pulse can be output immediately or whether  it  must

wait  for  the  main  fuel pulse to complete.  If the main fuel pulse has not

completed, then determine if the off-edge is in the queue or the carousel and

schedule the dynamic pulse on-edge appropriately.

DYNPW > 0 -------------------------|

|AND -| dyn_on_edge = current_time

off_edge_time < current_time ------|     | Call FUEL_DYN_FIT

|  (send out dyn pulse now)

|

| --- ELSE ---

|

DYNPW > 0 -------------------------------| dyn_on_edge = off_edge_time

| Call FUEL_DYN_FIT

|

| --- ELSE ---

|

| Exit FUEL_DYN_CALC

END: FUEL_DYN_CALC

8-225

FUEL STRATEGY, DYNAMIC FUEL, SCHEDULE PULSE ON-EDGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.33  SCHEDULE DYNAMIC FUEL PULSE ON-EDGE (CDAK0)

OVERVIEW

This routine uses the current dynamic fuel pulsewidth  value,  together  with

the time of its on-edge, to calculate the desired time of its off-edge.  This

routine then determines if the dynamic fuel pulse will be able to fit between

the  time  of  its  on-edge  and  the time that the cylinder currently on its

intake stroke will stop inducting charge.  If the entire pulse will not  fit,

then  the  routine  will determine if the minimum allowed pulse will fit.  If

the minimum pulse will not fit, then no dynamic pulse will be output; if  the

minimum  pulse  will fit, then the time of the off-edge for the maximum pulse

that will fit is determined.

If a (non-zero) pulse is to be output, then this routine:  1)  schedules  the

pulse's  on-edge,  and  then  2)  calls  a  routine  which  will schedule the

off-edge.

The graph shown below illustrates the three mutually exclusive cases  defined

by the conditions specified in the routine.

The assumptions shown below derive from the routine's assumption that  MINPWT

<=  DYNPW  and  that  off_edge_time[i]  <=  dyn_on_edge,  together  with  the

equations defining dyn_min_tim and dyn_off_tim in the above routine.

Assumptions:   dyn_on_edge  <  dyn_min_tim    <=   dyn_off_tim

V               V                   V

----------|---------------|-------------------|--------------->time

dyn_tim_limit     dyn_tim_limit      dyn_tim_limit

Cases:                   (III)              (II)               (I)

Case I:    dyn_on_edge < dyn_min_tim <= dyn_off_tim <= dyn_tim_limit

Case II:   dyn_on_edge < dyn_min_tim <= dyn_tim_limit < dyn_off_tim

Case III:  dyn_on_edge <= dyn_tim_limit < dyn_min_tim <= dyn_off_tim

Note:  Parameters whose names are alphabetic characters are in lower case are

non-displayable temporary registers.

8-226

FUEL STRATEGY, DYNAMIC FUEL, SCHEDULE PULSE ON-EDGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  DT12S = Time period between the latest two PIP  low-to-high  transitions,

in clock ticks.

-  DYNPW = Pulsewidth of the dynamic fuel pulse, in clock ticks.

-  LAST_HI_PIP = Time of the latest PIP low-to-high transition.

-  MINPWT = Minimum allowed fuel pulsewidth, in clock ticks.

-  PWOFF = Injector pulsewidth offset, on clock ticks.

Calibration Constants:

-  CALCLP = Percent of a PIP period, from the PIP up-edge at the start of an

intake  stroke  to  the point when fuel will no longer be inducted on the

current intake event.

OUTPUTS

Registers:

-  DYNPW = see above.

TEMPORARY REGISTERS

-  dyn_min_tim = Time that the injector would turn  off  if  delivering  the

minimum allowed fuel pulsewidth.

-  dyn_off_tim = Time that the injector  will  turn  off  the  dynamic  fuel

pulse.

-  dyn_on_edge = Time that the injector  is  to  turn  on  for  the  current

dynamic fuel pulse.

-  dyn_tim_limit = Calculated time when no additional fuel can  be  inducted

by the cylinder that is currently on its intake stroke.

-  off_edge_time[i] = Array containing the most recently scheduled  off-edge

time  for  injector  number  i.   This  value  is written both to the RAM

register pair LAST_OFF_HI[i] and LAST_OFF_IN[i], and to the HSO  carousel

when the off-edge is scheduled.

-  separate_pulse = A local flag which specifies whether the  dynamic  pulse

will  be appended directly to a main fuel pulse in progress, or issued as

a distinct make-up pulse after the completion of the main pulse.

ROUTINES CALLED

-  FUEL_DYN_OFF = Routine to schedule the dynamic fuel pulse off-edge.

8-227

FUEL STRATEGY, DYNAMIC FUEL, SCHEDULE PULSE ON-EDGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_DYN_FIT_COM2

BEGIN: FUEL_DYN_FIT

; Called from the FUEL_DYN_CALC strategy module.

; This strategy module corresponds to the WILL_PULS_FIT and PUT_OUT_PULSE

; routines in the DYNAMIC_FUEL software module.

; Note: This routine assumes that MINPWT <= DYNPW and that off_edge_time[i]

; <= dyn_on_edge.  (Ref.: See the graph on the following page.)

Unconditionally ------------------------| dyn_tim_limit :=

|      LAST_HI_PIP + (DT12S * CALCLP)

| i := int_inj_num

dyn_on_edge = off_edge_time[i]  --------| dyn_off_tim := dyn_on_edge + DYNPW

;The dynamic fuel pulse is to          | ;Do not include the pulsewidth

; be contiguous in time with           | ; offset when the dynamic fuel

; the main fuel pulse                  | ; pulse will be contiguous with

| ; the main fuel pulse

|

| dyn_min_tim := dyn_on_edge

| ;There need not be a non-zero

| ; minimum duration of the dynamic

| ; pulse when the dynamic pulse will

| ; be contiguous with the main pulse

|

| separate_pulse := 0

|

| --- ELSE ---

|

| dyn_off_tim := dyn_on_edge +

|                  DYNPW + PWOFF

| dyn_min_tim := dyn_on_edge +

|                  MINPWT + PWOFF

| separate_pulse := 1

8-228

FUEL STRATEGY, DYNAMIC FUEL, SCHEDULE PULSE ON-EDGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

dyn_off_tim <= dyn_tim_limit -----------| SCHEDULE dynamic fuel on-edge

|  to occur at dyn_on_edge.

(Case I)                           | Call: FUEL_DYN_OFF

| ;Dynamic fuel pulse will fit, so

| ; maintain current desired values

| ; of DYNPW and dyn_off_tim

|

| --- ELSE ---

|

dyn_min_tim <= dyn_tim_limit -----|     |

|AND -| dyn_off_tim := dyn_tim_limit

separate_pulse = 1 ---------------|     | DYNPW := (dyn_off_tim - dyn_on_tim)

;Normal dynamic pulse is               |            - PWOFF

; distinct from main pulse             | SCHEDULE dynamic fuel on-edge

|  to occur at dyn_on_edge.

(Case IIa)                         | Call: FUEL_DYN_OFF

| ;Normal dynamic pulse won't fit,

| ; but the minimum pulse will fit;

| ; determine maximum allowed pulse

|

|

| --- ELSE ---

|

dyn_min_tim <= dyn_tim_limit -----------| dyn_off_tim := dyn_tim_limit

;Dynamic pulse is appended to          | DYNPW := dyn_off_tim

; the main fuel pulse                  |           - dyn_on_edge

| SCHEDULE dynamic fuel on-edge

(Case IIb)                         |  to occur at dyn_on_edge.

| Call: FUEL_DYN_OFF

| ;Normal dynamic pulse won't fit,

| ; but the minimum pulse will fit;

| ; determine maximum allowed pulse

|

|

| --- ELSE ---

(Case III)                         |

| DYNPW := 0

| Exit FUEL_DYN_FIT

| ;The minimum allowed pulse will

| ; not fit, so cancel dynamic fuel

| ; pulse

END: FUEL_DYN_FIT

8-229

FUEL CONTROL STRATEGY, FUEL PULSEWIDTH, DYNAMIC FUEL OFF - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.34  DYNAMIC FUEL OFF (CDAI0)

OVERVIEW

This routine is called after:  (1) a  dynamic  fuel  pulse  on-edge  for  the

injector  whose  corresponding cylinder is currently on its intake stroke has

been scheduled, AND (2) the fuel pulse's off-edge has been calculated.   This

routine schedules the fuel pulse off-edge for the injector.

NOTE:  Parameters whose alphabetic characters  are  in  the  lower  case  are

non-displayable temporary registers.

DEFINITIONS

INPUTS

Registers:

-  AHISL_TICK  =  Temperature  compensated  high  slope  for  a  dual  slope

characterization of injector flow, lbm/tick.

-  ALOSL_TICK  =  Temperature  compensated  low  slope  for  a  dual   slope

characterization of injector flow, lbm/tick.

-  BKPT_PW = Dual slope breakpoint in lbm divided by temperature compensated

low slope (lbm/tick), ticks.

-  DYNPW = Dynamic fuel correction pulsewidth, ticks.

-  FB_DIFF = Difference between FUEL_BKPT divided by temperature compensated

high  slope  and  FUEL_BKPT divided by temperature compensated low slope,

ticks.

-  FUEL_SUM = An accumulator which stores the total amount of  fuel  output,

lbm.

-  PWOFF = Injector pulsewidth offset, ticks.

Calibration Constants:

-  FUEL_BKPT = The amount of fuel flow when transfer is made  from  the  low

slope to the high slope, lbm/injection.

OUTPUTS

Registers:

-  FUEL_SUM = See above.

8-230

FUEL CONTROL STRATEGY, FUEL PULSEWIDTH, DYNAMIC FUEL OFF - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TEMPORARY REGISTERS

-  dyn_off_tim = The desired time that the dynamic fuel  pulse  is  to  end;

i.e., the time at which the injector is to turn-off.

-  inj_dyn = Fuel mass delivered by the dynamic fuel pulse, lbm.

PROCESS

STRATEGY MODULE: FUEL_DYN_OFF_COM2

BEGIN: FUEL_DYN_OFF

; Invoked from within the FUEL_DYN_FIT strategy module.

Unconditionally -------------------| Schedule the dynamic fuel off-edge

| to occur at time dyn_off_tim

| (calculated elsewhere).

; Determine the mass of fuel that has been injected and update certain

; registers. (FUEL_SUM maintains the total fuel injected, for the DOL

; calculation.)

DYNPW < BKPT_PW -------------|

|AND -| inj_dyn = DYNPW * ALOSL_TICK

DYNPW > 0 -------------------|     |

| --- ELSE ---

|

DYNPW > 0 -------------------------| inj_dyn = (DYNPW + FB_DIFF)

|   * AHISL_TICK

Unconditionally -------------------| FUEL_SUM = FUEL_SUM + inj_dyn

END: FUEL_DYN_OFF

8-231

FUEL STRATEGY, BACKGROUND INJECTOR CUTOUT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.35  BACKGROUND INJECTOR CUTOUT (CDAM0)

OVERVIEW

Turning off injectors must be done in a specific order depending on how  many

injectors are requested to be on.  Generally the highest frequency is desired

for turning off a portion of injectors( i.e.  1 off, 1 on, 1 off, etc.  not 4

on  4  off).  Once an injector is turned off it may be desirable to keep that

injector off to minimize transient fuel  effects.   However,  this  may  have

adverse  affects  on cylinder valve temperatures and possibly catalyst midbed

temperatures.

To avoid a possible exhaust valve overtemp condition, this logic  allows  the

injector  cutout  bit  pattern to be rotated to ensure that all cylinders are

equally cooled.  Every INJ_HOLD number of engine events, the bit  pattern  is

shifted  left one bit.  Pattern shifting is conducted in the background, thus

providing one BG loop resolution on pattern rotation.  A calibratable  switch

is  provided  to  enable  this  round-robin type of cylinder rotation, or the

default is a set pattern (non-rotating).

8 Cylinder pattern example:

EXAMPLE:

========                =====================================

Value of INJ_BANKn:       7   6   5   4   3   2   1   0

Injector number:         [8] [4] [5] [6] [2] [7] [3] [1] (4.6L FIRING ORDER)

bit position value:      128  64  32  16  8   4   2   1

-------------------------------

INJ_ON=8 -> INJ_OFF :=  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

-------------------------------

INJ_ON=7 -> INJ_OFF :=  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  = INJ_7ON = 1

-------------------------------

INJ_ON=6 -> INJ_OFF :=  | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |  = INJ_6ON = 17

-------------------------------

INJ_ON=5 -> INJ_OFF :=  | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 |  = INJ_5ON = 22

-------------------------------

INJ_ON=4 -> INJ_OFF :=  | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |  = INJ_4ON = 85

-------------------------------

INJ_ON=3 -> INJ_OFF :=  | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 1 |  = INJ_3ON = 117

-------------------------------

INJ_ON=2 -> INJ_OFF :=  | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 1 |  = INJ_2ON = 119

-------------------------------

INJ_ON=1 -> INJ_OFF :=  | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  = INJ_1ON = 127

-------------------------------

INJ_ON=0 -> INJ_OFF :=  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  = INJ_0ON = 255

-------------------------------

(0 = ON)

(1 = OFF)

8-232

FUEL STRATEGY, BACKGROUND INJECTOR CUTOUT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

6 Cylinder pattern example:

EXAMPLE:

========                =====================================

Value of INJ_BANKn:               5   4   3   2   1   0

Injector number:                 [5] [6] [2] [4] [3] [1] (3.0L FIRING ORDER)

bit position value:               32  16  8   4   2   1

-------------------------------

INJ_ON=6 -> INJ_OFF :=  | x | x | 0 | 0 | 0 | 0 | 0 | 0 |

-------------------------------

INJ_ON=5 -> INJ_OFF :=  | x | x | 0 | 0 | 0 | 0 | 0 | 1 |  = INJ_5ON = 1

-------------------------------

INJ_ON=4 -> INJ_OFF :=  | x | x | 0 | 0 | 1 | 0 | 0 | 1 |  = INJ_4ON = 9

-------------------------------

INJ_ON=3 -> INJ_OFF :=  | x | x | 0 | 0 | 1 | 0 | 1 | 1 |  = INJ_3ON = 11

-------------------------------

INJ_ON=2 -> INJ_OFF :=  | x | x | 0 | 1 | 1 | 0 | 1 | 1 |  = INJ_2ON = 27

-------------------------------

INJ_ON=1 -> INJ_OFF :=  | x | x | 0 | 1 | 1 | 1 | 1 | 1 |  = INJ_1ON = 31

-------------------------------

INJ_ON=0 -> INJ_OFF :=  | x | x | 1 | 1 | 1 | 1 | 1 | 1 |  = INJ_0ON = 63

-------------------------------

(0 = ON)           ( INJ_6ON = 0 ; INJ_7ON = 0 )

(1 = OFF)

(x = don't care, assume 0)

4 Cylinder pattern example:

EXAMPLE:

========                =====================================

Value of INJ_BANKn:                       3   2   1   0

Injector number:                         [4] [2] [3] [1] (1.9L FIRING ORDER)

bit position value:                       8   4   2   1

-------------------------------

INJ_ON=4 -> INJ_OFF :=  | x | x | x | x | 0 | 0 | 0 | 0 |

-------------------------------

INJ_ON=3 -> INJ_OFF :=  | x | x | x | x | 0 | 0 | 0 | 1 |  = INJ_3ON = 1

-------------------------------

INJ_ON=2 -> INJ_OFF :=  | x | x | x | x | 0 | 1 | 0 | 1 |  = INJ_2ON = 5

-------------------------------

INJ_ON=1 -> INJ_OFF :=  | x | x | x | x | 0 | 1 | 1 | 1 |  = INJ_1ON = 7

-------------------------------

INJ_ON=0 -> INJ_OFF :=  | x | x | x | x | 1 | 1 | 1 | 1 |  = INJ_0ON = 15

-------------------------------

(0 = ON)      ( INJ_4ON = 0 ; INJ_5ON = 0 ;

(1 = OFF)       INJ_6ON = 0 ; INJ_7ON = 0 )

(x = don't care, assume 0)

8-233

FUEL STRATEGY, BACKGROUND INJECTOR CUTOUT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  DISCUTOUT1 = One of the injectors that should not be fueled due.

-  DISCUTOUT2 = One of the injectors that should not be fueled due.

-  INJ_CTR = Number of pip up edges since last bit pattern change.

-  INJ_OFF = A byte containing bit flags indicating injector sequence to  be

turned off.

-  INJOFF_FMEM = Injectors which should not be fired due to a coil failure.

-  INJ_OFF_LST = A byte containing the bit flags of the previous  background

loop.

-  INJ_OFF_NEXT = A byte containing bit flags  indicating  injectors  to  be

turned off upon the next round robin rotation.

-  INJ_ON = Number of injectors desired to be on, cylinders.

-  INJ_ON_LST = Last background pass value of INJ_ON, number of cylinders.

Bit flags:

-  INJ_NEXT_FLG = Flag signalling to the foreground  to  rotate  the  cutout

pattern  when  the  firing  order is on INJ_RR_TRIG cylinder, 1 -> change

pattern when injector INJ_RR_TRIG is scheduled.

Calibration Constants:

-  FN1329 = A one-row table that maps an injector's 'firing sequence number'

to its 'injector number'.

-  INJ_7ON = Value which indicates bit pattern for 7 injector on.

-  INJ_6ON = Value which indicates bit pattern for 6 injectors on.

-  INJ_5ON = Value which indicates bit pattern for 5 injectors on.

-  INJ_4ON = Value which indicates bit pattern for 4 injectors on.

-  INJ_3ON = Value which indicates bit pattern for 3 injectors on.

-  INJ_2ON = Value which indicates bit pattern for 2 injectors on.

-  INJ_1ON = Value which indicates bit pattern for 1 injectors on.

-  INJ_0ON = Value which indicates bit pattern for 0 injectors on.

-  INJCUTOUT_SW = Switch to select injector cutout method to use; 1 -> round

robin, 0 -> normal injector cutout, set bit pattern.

-  INJ_HOLD = Number of complete engine cycles to hold current bit pattern.

8-234

FUEL STRATEGY, BACKGROUND INJECTOR CUTOUT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  INJ_RR_TRIG = Firing order cylinder number in which round robin rotations

will occur at.

-  NUMCYL = Number of cylinders.

8-235

FUEL STRATEGY, BACKGROUND INJECTOR CUTOUT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_INJ_OFF_BG_COM3

BEGIN:  determine_bit_pattern

INJ_ON = NUMCYL --------------------| inj_off = 0

(all injectors on)                 | ;no change to INJ_CTR

| ;no change to INJ_OFF_NEXT

| ;no change to INJ_NEXT_FLG

|

| --- ELSE ---

|

INJ_ON <> INJ_ON_LST ---------------| Do: set_bit_pattern

(injector count has changed)       | INJ_CTR = 0

| ;no change to INJ_OFF_NEXT

| ;no change to INJ_NEXT_FLG

|

| --- ELSE ---

|

INJ_CTR >=                          | inj_off = INJ_OFF_LST

INJ_HOLD * NUMCYL ------------|     | Do: injector_round_robin(1)

|AND -| INJ_OFF_NEXT = inj_off

INJCUTOUT_SW = 1 -------------|     | INJ_NEXT_FLG = 1

| inj_off = INJ_OFF_LST

| INJ_CTR = 0

|

| --- ELSE ---

|

| inj_off = INJ_OFF_LST

| ;no change to INJ_CTR

| ;no change to INJ_OFF_NEXT

| ;no change to INJ_NEXT_FLG

DISCUTOUT1 <> 0 --------------|

|OR --| Do: dis_fmem_bits

DISCUTOUT2 <> 0 --------------|     |

| --- ELSE ---

|

| INJOFF_FMEM = 0

;This logic combines the injector inj_off bits and the DIS_FMEM bits.

inj_off(bit(n)) = 1 ----------|

|OR --| INJ_OFF(bit(n)) = 1

INJOFF_FMEM(bit(n)) = 1 ------|     |

| --- ELSE ---

|

| INJ_OFF(bit(n)) = 0

unconditionally --------------------| INJ_OFF_LST = INJ_OFF

| INJOFF = INJ_OFF

END: determine_bit_pattern

8-236

FUEL STRATEGY, BACKGROUND INJECTOR CUTOUT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: foreground_round_robin

;Execute the following logic upon every PIP up edge:

unconditionally --------------------| INJ_CTR = INJ_CTR + 1

|  (increment count of cylinder

|    events for this bit pattern)

;Do the following logic before INJ_OFF is checked in foreground:

INJ_NEXT_FLG = 1 -------------|     | INJOFF = INJ_OFF_NEXT

|AND -| INJ_NEXT_FLG = 0

INJ_BANK1 = INJ_RR_TRIG ------|     |

| --- ELSE ---

|

| no change INJ_OFF

| no change INJ_NEXT_FLG

END: foreground_round_robin

BEGIN: dis_fmem_bits

;Execute the following process only when called.

;Convert DISCUTOUT1,2 into a sequence number and build a bit pattern.

unconditionally --------------------| count = 0

;Execute the following routines 'numcyl' time(s).

FN1329(count) = DISCUTOUT1 ---|

|OR --| INJOFF_FMEM(bit(count)) = 1

FN1329(count) = DISCUTOUT2 ---|     |

| --- ELSE ---

|

| INJOFF_FMEM(bit(count)) = 0

unconditonally ---------------------| count = count + 1

END: dis_fmem_bits

8-237

FUEL STRATEGY, BACKGROUND INJECTOR CUTOUT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: set_bit_pattern

Set the master bit pattern each time INJ_ON changes.

INJ_ON = 7 -------------------------| inj_off = INJ_7ON

|

| --- ELSE ---

|

INJ_ON = 6 -------------------------| inj_off = INJ_6ON

|

| --- ELSE ---

|

INJ_ON = 5 -------------------------| inj_off = INJ_5ON

|

| --- ELSE ---

|

INJ_ON = 4 -------------------------| inj_off = INJ_4ON

|

| --- ELSE ---

|

INJ_ON = 3 -------------------------| inj_off = INJ_3ON

|

| --- ELSE ---

|

INJ_ON = 2 -------------------------| inj_off = INJ_2ON

|

| --- ELSE ---

|

INJ_ON = 1 -------------------------| inj_off = INJ_1ON

|

| --- ELSE ---

|

| inj_off = INJ_0ON

END: set_bit_pattern

BEGIN: injector_round_robin(shift)

Do the following routines 'shift' time(s).

unconditionally ------------------| inj_off = inj_off * 2

(shifted left by one bit)

inj_off >= 2 ** NUMCYL -----------| inj_off = inj_off + 1

(MS bit is wrapped to LS bit)    |

| --- ELSE ---

|

| no change to inj_off

END: injector_round_robin

8-238

FUEL STRATEGY, INJ_ON LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.36  INJ_ON DETERMINATION LOGIC (CDAN0)

OVERVIEW

This module is for non-Torque Control based strategies  so  that  the  INJ_ON

parameter can be assigned and then used by the generic round robin logic.

DEFINITIONS

Registers:

-  COOL_PI_INT = Fail safe cooling output, number of cylinders to turn off.

-  INJ_ON = Number of injectors to be turned (left) on.

-  INJ_ON_LST = Previous background state of INJ_ON.

-  INJ_TR = Torque ratio obtained from turning off a number of injectors.

Bit Flags:

-  COOL_FLG = Overtemp flag:  1 -> overtemp.

-  DFSFLG = Indicates DECEL fuel shutoff, 1 -> active.

-  DIS_FMEM = Coil failure flag, associated cylinders will be disabled.

-  HALF_FUEL = Flag which indicates one-half of the  injections  are  to  be

cut-out; 1 -> HALF FUEL.

-  NO_FUEL = FLag which indicates that all injections are to be  cut-out;  1

-> no fuel.

Calibration Constants:

-  ENGCYL = Number of pip's per engine revolution, NUMCYL/2.

-  NUMCYL = Number of cylinders in the engine.

8-239

FUEL STRATEGY, INJ_ON LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_INJ_ON_COM1

BEGIN: injector_ratio_calc

;Execute the following process once per background loop.

unconditionally ------------------------| INJ_ON_LST = INJ_ON

COOL_FLG = 1 ---------------------------| INJON = NUMCYL - COOL_PI_INT

;fail safe cooling                     |

| --- ELSE ---

NO_FUEL = 1 ----------------------|     |

|OR --| INJ_ON = 0

DFSFLG = 1 -----------------------|     | ;no injectors on

|

| --- ELSE ---

|

HALF_FUEL = 1 --------------------------| INJ_ON = ENGCYL

| ;numcyl divided by 2

|

| --- ELSE ---

|

| INJ_ON = NUMCYL

DIS_FMEM = 1 ---------------------------| INJ_TR = min( 2 / NUMCYL,

;cylinders being disabled              |           INJ_ON / NUMCYL )

|

| --- ELSE ---

|

| INJ_TR = INJ_ON / NUMCYL

END: injector_ratio_calc

8-240

FUEL STRATEGY, PIP COUNTER FOR CRANKING FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.37  PIP COUNTER FOR CRANKING FUEL (CDAA0)

OVERVIEW

This module updates the pip counter used in crank mode, CRKPIP_CTR_W and  its

background equivalent, CRKPIPCTR_BW.

DEFINITIONS

INPUTS

Registers:

-  ATMR1_LST = Last calculated value of ATMR1.

-  BP = Barometric absolute pressure.

-  CRKPIP_CTR_W = Foreground word register to count PIPs for crank fuel.

-  CRKPIPCTR_BW = Background word register to count PIPs for crank fuel.

Bit Flags:

-  CRKFLG = Crank flag; 1 -> engine in Crank mode.

-  FLG_STALL  =  Flag  indicating  a  stall  has  occured;  transition  from

underspeed/run to crank

Calibration Constants:

-  CRKCTR_RESET = CRKPIP_CTR reset switch; 1 -> reset CRKPIP_CTR upon stall.

-  CRKPIPCNT2_W  =  CRKPIPCTR_BW  reset  value  for  underspeed   to   crank

transitions, sec.

-  CRKTM1 = Time in run or underspeed below which CRKPIP_CTR_W is  reset  to

CRKPIPCNT2_W, sec.

-  CRKTMR_INC = Calibration switch  which  determines  whether  CRKPIP_CTR_W

runs when the engine state is out of crank.

OUTPUTS

Registers:

-  CRKPIP_CTR_W = See above.

-  CRKPIPCTR_BW = See above.

8-241

FUEL STRATEGY, PIP COUNTER FOR CRANKING FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_CRKPIPCTR_COM1

CRKPIP_CTR_W - PIP COUNTER FOR CRANKING FUEL

CRKPIP_CTR_W is used as a crank fuel multiplier in order to do a lean-out  or

fuel shut-off during sub-zero cold operation.  Some calibrators are using the

logic to improve restarts following a stall  during  -20  deg  F  testing  by

turning  off  the  fuel  during  the first seconds of crank.  This allows the

engine to restart on the residual fuel remaining in the  manifold  after  the

stall.

CRKCTR_RESET = 1 ------------|

|

FLG_STALL = 1 ---------------|AND -| Reset counter part way

|     | CRKPIP_CTR_W = CRKPIPCNT2_W

ATMR1_LST <= CRKTM1 ---------|     | CRKPIPCTR_BW = CRKPIPCNT2_W

(in run or underspeed less        |

than CRKTM1 seconds)             |

| --- ELSE ---

CRKFLG = 1 ------------------|     |

(in crank)                  |OR --| CRKPIP_CTR_W is incremented

|     | every rising edge of PIP

CRKTMR_INC > 0 --------------|     | CRKPIPCTR_BW = CRKPIP_CTR_W

|

| --- ELSE ---

|

| Freeze CRKPIP_CTR_W

| CRKPIP_CTR_W = CRKPIPCTR_BW

In Foreground PIP_DATA module

always ----------------------------| Increment CRKPIP_CTR_W every

| rising edge of PIP,

| Clip CRKPIP_CTR_W at 65535 as

| a maximum

8-242

FUEL STRATEGY, CLOSED THROTTLE NEUTRAL TIMER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.38  CLOSED THROTTLE NEUTRAL TIMER (CDAA0)

OVERVIEW

This module updates the closed throttle mode neutral timer, CTNTMR.

DEFINITIONS

INPUTS

Registers:

-  APT = Throttle mode flag; -1 -> closed throttle, 0 -> part throttle, 1 ->

wide open throttle.

-  ECT = Engine coolant temperature, deg.  F.

-  N = Engine speed, RPM.

Bit Flags:

-  CTNFLG = 1 -> closed throttle neutral idle.

-  NDSFLG = Neutral/Drive flag; 1 -> Drive.

Calibration Constants:

-  CTHIGH = Hot star minimum engine coolant temperature, Deg F.

-  FN880(CTNTMR) = DSDRPM adder vs.  time at Idle, sec.

-  INLRPM = Maximum RPM to increment CTHTMR.

-  INLRPH = Hysteresis for INLRPM.

-  NIHYS = Neutral Idle Hysteresis, sec.  Base value is 2,  range  is  0  to

100.

-  NIOLD = Neutral Idle Open Loop Delay, sec.  Base value is 255, range is 0

to 255.

OUTPUTS

Registers

-  CTNTMR = Closed throttle mode neutral timer, sec.

Bit Flags:

-  CTNFLG = 1 -> closed throttle neutral idle.

8-243

FUEL STRATEGY, CLOSED THROTTLE NEUTRAL TIMER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_CTNTMR_COM1

ECT > CTHIGH -----------------------|

|

APT = -1 ---------------------------|

(closed throttle)                  |

|AND -| Count up CTNTMR

NDSFLG = 0 -------------------------|     | Clip to NIOLD + NIHYS

(neutral)                          |     | CTNFLG = 1

|     |

N < INLRPM + FN880 -----------|S Q -|     |

|           |

N > INLRPM + FN880 + INLRPH --|C          |

| --- ELSE ---

|

| Count down CTNTMR

| CTNFLG = 0

| Clip CTNTMR to 0

8-244

FUEL STRATEGY, TIME AT IDLE WITH TRANSMISSION IN DRIVE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.39  TIME AT IDLE WITH TRANSMISSION IN DRIVE (CDAA0)

OVERVIEW

This module updates the timer, IDLTMR.

DEFINITIONS

INPUTS

Registers:

-  APT = Throttle mode flag; -1 -> closed throttle, 0 -> part throttle, 1 ->

wide open throttle.

-  N = Engine speed, RPM.

Bit Flags:

-  DNDSUP = Drive/Neutral select.

-  IDLFLG = Flag indicating transmission in drive and at idle.

Calibration Constants:

-  IDLRPM = Maximum RPM for Closed Throttle Mode Idle, rpm.  Range of  0  to

7000, accuracy of 25 RPM.

-  IDRPMH = Hysteresis for IDLRPM.  Range of 0 to 1000, accuracy of 25 RPM.

OUTPUTS

Bit Flags:

-  IDLFLG = Flag indicating transmission in drive and at idle.

Registers

-  IDLTMR = Time at idle with transmission in drive.

8-245

FUEL STRATEGY, TIME AT IDLE WITH TRANSMISSION IN DRIVE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_IDLTMR_COM1

DNDSUP = 1 ---------------------|

(drive)                        |

|AND -| Increment IDLTMR

APT = -1 -----------------------|     | IDLFLG = 1

(CLOSED THROTTLE mode)         |     |

|     |

N < IDLRPM ---------------|S Q -|     |

|           |

N > IDLRPM + IDRPMH ------|C          |

| --- ELSE ---

|

| IDLTMR = 0

| IDLFLG = 0

8-246

FUEL STRATEGY, LOM LOAD TRANSITION TIMER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.40  LOM LOAD TRANSITION TIMER (CDAA0)

OVERVIEW

This module updates the timer LUGTMR.

DEFINITIONS

INPUTS

Registers:

-  APT = Throttle mode flag; -1 -> closed throttle, 0 -> part throttle, 1 ->

wide open throttle.

-  LOAD = Universal normalized load parameter.

Calibration Constants:

-  FN125 = LOM Load function to activate LOM spark  strategy.   Input:   RPM

and Output:  load.

-  LUGSW = LUGTMR logic switch.

-  LUGTIM = Engine load transition time, sec.

OUTPUTS

Registers

-  LUGTMR = LOM load transition timer, sec.

PROCESS

STRATEGY MODULE: FUEL_LUGTMR_COM1

LOAD > FN125 -------------------|

|OR --| Count up LUGTMR

APT = 1 ------------------------|     |  (clip at LUGTIM)

(Wide Open Throttle)                 |

| --- ELSE ---

LOAD <= FN125 ------------------|     |

|AND -| LUGTMR = 0

LUGSW = 1 ----------------------|     |

| --- ELSE ---

|

| Count down LUGTMR

| (Clip at 0)

8-247

FUEL STRATEGY, TIME SINCE INCREMENTING LAMMUL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.41  TIME SINCE INCREMENTING LAMMUL (CDAA0)

OVERVIEW

This module shows the logic for updating the timer MULTMR.

DEFINITIONS

OUTPUTS

Registers:

-  MULTMR = Time since incrementing LAMMUL, msec.

PROCESS

STRATEGY MODULE: FUEL_MULTMR_COM1

Always -------------------------------------| Count up MULTMR

(Note: MULTMR is periodically set to 0 within

the Open Loop Fuel Logic)

8-248

FUEL STRATEGY, FUEL TIMER CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.42  TIMER SUMMARY (CDAI0)

TIMER             DESCRIPTION

-----             -----------

ATMR1           Time since start (time since exiting crank mode) (sec)

ATMR1_HI_RES   Time since start (time since exiting crank mode)

with a higher resolution that ATMR1. (sec)

ATMR2           Time since engine coolant temperature became

greater than TEMPFB (sec)

ATMR3           Time since Entering RUN Mode (sec)

CTTMR           Time at closed throttle timer (sec)

NDDTIM          Time since neutral/drive switch state change (sec)

DEFINITIONS

INPUTS

Registers:

-  APT = Throttle mode flag; -1 -> closed throttle, 0 -> part throttle, 1 ->

wide open throttle.

-  ATMR1_LST = Last calculated value of ATMR1.

-  ECT = Engine coolant temperature, deg.  F.

Bit Flags:

-  CRKFLG = Crank flag; 1 -> engine in Crank mode.

-  DNDSUP = Drive/Neutral select.

-  UNDSP = Underspeed flag; 1 -> Underspeed mode or Crank.

Calibration Constants:

-  TEMPFB = Minimum ECT required to start ATMR2 timer, deg F.

8-249

FUEL STRATEGY, FUEL TIMER CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OUTPUTS

Registers:

-  ATMR1_LST = Last calculated value of ATMR1.

8-250

FUEL STRATEGY, FUEL TIMER CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_TIMERS_COM1

FUEL TIMER CONTROL LOGIC

ATMR1 - TIME SINCE ENGINE START-UP

CRKFLG = 0 --------------------------| ATMR1_LST = ATMR1

(RUN or UNDERSPEED mode)             | (save last calculated value of ATMR1)

| Count up ATMR1

| Count up ATMR1_HI_RES

| (calculate new values)

|

| --- ELSE ---

|

| ATMR1_LST = ATMR1

| (save last calculated value of ATMR1)

| ATMR1 = 0

| ATMR1_HI_RES = 0

ATMR2 - TIME SINCE ENGINE COOLANT TEMPERATURE BECAME GREATER THAN TEMPFB

ECT > TEMPFB ------------------|S Q -| COUNT UP ATMR2

|     |

CRKFLG = 1 --------------------|C    |

| --- ELSE ---

|

| ATMR2 = 0

8-251

FUEL STRATEGY, FUEL TIMER CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

ATMR3 - TIME SINCE ENTERING RUN MODE

UNDSP = 0 ---------------------------| Increment ATMR3

(Run Mode)

CTTMR - TIME AT CLOSED THROTTLE

APT = -1 ----------------------------| COUNT UP CTTMR

(closed throttle mode)             |

| --- ELSE ---

|

| CTTMR = 0

NDDTIM - TIME SINCE NEUTRAL/DRIVE SWITCH STATE CHANGE

NEUTRAL/DRIVE SWITCH STATE CHANGE ---| NDDTIM = 0

|

| --- ELSE ---

|

| COUNT UP NDDTIM

8-252

FUEL STRATEGY, CRANK PIP INJECTOR SELECTOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.43  CRK_PIP_INJ SELECTOR LOGIC (CDAL0)

OVERVIEW

This module switches between the default (initial) value and CRK_PIP_CAL  for

parameter  CRK_PIP_INJ  (number  of  PIPs  per  injector firing) based on the

number of background loops where the instantaneous  ECT  (ECT_INST)  is  LESS

THAN  CRK_PIP_ECT.   This  logic  uses  the  calibrated  method  of  injector

servicing as a default until such time as the actual ECT level is believed to

be accurate.  It is expected that the ECT will be verified in time to support

the 'trickle' fuel alternate calibration for extremely cold conditions, since

ECT accuracy is believed to be related to the number of background loops from

EEC power-up which, during cold engine conditions,  will  occur  much  sooner

than  the  first  recognized  PIP.   The recommended method of start injector

servicing is 'big-bang', where all injectors are fired  simultaneously,  once

every NUMCYL pips.  Trickle fueling is only recommended during extremely cold

starting conditions.

DEFINITIONS

Registers:

-  CRK_LOOP_CTR = Crank BG loop counter for assigning CRK_PIP_INJ.

-  CRK_PIP_INJ = Number of PIP periods per injector firing.

-  ECT_INST = Instantaneous ECT (no rolling average).

-  TSLPIP = Time sine last PIP.

Bit Flags:

-  CRK_PIP_INT = CRK_PIP_INJ initialization  indicator,  set  in  foreground

logic; 1 -> CRK_PIP_INJ is initialized.

Calibration Constants:

-  CRK_PIP_ECT  =  ECT  break  point  which  selects  between  the   default

CRK_PIP_INJ (1), and CRK_PIP_CAL.

-  CRK_PIP_CAL = Number of PIP periods per injector firing for higher ECTs.

8-253

FUEL STRATEGY, CRANK PIP INJECTOR SELECTOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_CRK_INJ_COM1

CRK_PIP_INJ SELECTOR LOGIC

CRK_PIP_INT = 1 --------------|

|AND -| CRK_PIP_INT := 0

TSLPIP > 800 msec ------------|     | CRK_LOOP_CTR := 0

;stall or aborted start

CRK_PIP_INT = 1 --------------------| (Exit module)

|

| --- ELSE ---

|

ECT_INST <= CRK_PIP_ECT ------------| CRK_LOOP_CTR := CRK_LOOP_CTR + 1

;cold engine indicated

CRK_LOOP_CTR >= 3 ------------------| CRK_PIP_INJ := 1

;definitely a cold engine          | CRK_LOOP_CTR := 3

| ;use trickle fuel injector method

|

| --- ELSE ---

|

| CRK_PIP_INJ := CRK_PIP_CAL

| ;use calibrated injector fueling

8-254

FUEL STRATEGY, LAMBSE MODULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.44  LAMBSE MODULATION (CDAM0)

OVERVIEW

In order to discriminate between marginal HEGO sensors and bad HEGO  sensors,

a closed loop fuel control limit cycle at a frequency higher than the natural

frequency of the system is required.

This module is designed to generate a signal  to  be  superimposed  over  the

normal  closed loop PI fuel control signal.  This signal will be in the range

of 2.5HZ - 5 Hz and take any wave form as defined in FN339B.  The signal will

be added to LAMBSEn when calculating required fuel flow.

The signal generator is implemented by the parameters FN339B,  DSRD_FREQ  (in

Hertz),  and  AMP_MULT  as  shown  below.  The output, multiplied by AMP_MULT

takes values in units of normalized A/F and is stored in LAMBSE_MODn.

|

1  +                        *

|                 *             *

|             *                     *

|          *                           *

F       |

N       |       *                                 *

3       |

3       |

9       |   *                                         *

B       |

|

|*                                               *

0   *-------------------------------------------------*----------->

0                                                 1    FN_INPUT

(normalized time)

in real time, the plot is:

|

AMP_MULT +           *

|       *       *

|    *             *

LAMBSE_MODn |

| *                   *

|                                           1/(DSRD_FREQ)

LAM_OFFSET  *------------------------*------------------------*---------->

|                                                         time

|                           *                   *

|

|                              *              *

|                                 *         *

- AMP_MULT +                                      *

|

8-255

FUEL STRATEGY, LAMBSE MODULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  FN_GEN_TMR  =  LAMBSE   fuel   modulation   function   generator   timer.

Free-running up counter.  Units are seconds.

Resolution = 0.001, Range 0 - 1, Initial value = 0.0.

-  LAM_CYCLES = Number of commanded  LAMBSE_MOD  cycles  that  have  occured

since starting the upstream EGO monitor.

Bit Flags:

-  LAM_MOD_FLG = Flag used to initiate high frequency  fuel  modulation  for

the upstream EGO monitor.

Resolution = 1, Range = 0 - 1, Base value = 0.

-  SMPL_EGO_FLG = Flag used to enable the foreground  EGO  sampling  process

used on the upstream EGO sensors.

Calibration Constants:

-  DSRD_FREQ = Desired frequency of LAMBSE fuel modulation, Hertz.

Resolution = 0.25, Range = 1 - 10, Base value = 2.5.

-  AMP_MULT =  LAMBSE  fuel  modulation  amplitude  multiplier.   Units  are

normalized A/F.

Resolution = 0.001, Range = 0 - 0.5, Base value = 0.1.

-  FN339B(x) = LAMBSE fuel modulation function generator.  Output has  units

of normalized A/F, input has units of normalized time (FN_INPUT), ranging

from 0 - 20 (useful range 0 - 2).

Resolution = 0.001, Range = 0 - 1, 10 xy pairs.

Base calibration = (0.0, 0.0), (0.2, 0.588), (0.35, 0.891),

(0.45, 0.988), (0.5, 1.0), (0.55, 0.988), (0.65, 0.891),

(0.8, 0.588), (1.0, 0.0).

-  LAM_MOD_MAX = Maximum value for LAMBSE MODn, NORM A/F.

-  LAM_MOD_MIN = Minimum value for LAMBSE MODn, NORM A/F.

OUTPUTS

Registers:

-  LAMBSE_MODn = LAMBSE fuel modulation signal (n = 1, 2).  This register is

added  to  LAMBSE when calculating fuel flow to give a high frequency EGO

switch.  Units are normalized A/F.

Resolution = 0.00003, Range = -.5 to +.5, Initial value = 0.0.

-  FN_INPUT = LAMBSE fuel modulation function generator  input.   Units  are

normalized time.

Resolution = 0.0005, Range = 0 - 20, Initial value = 0.0.

8-256

FUEL STRATEGY, LAMBSE MODULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_LAMBSE_MOD_COM1

LAM_MOD_FLG = 1 -----------------------| function_input =

|   2 * DSRD_FREQ * FN_GEN_TMR

|

| --- ELSE ---

|

| No action

LAM_MOD_FLG = 1 -----------------|

|AND -| FN_INPUT = MOD(function_input,2)

function_input > 2 --------------|     | FN_GEN_TMR = 0

| amplitude_multiplier = AMP_MULT

| LAM_CYCLES = LAM_CYCLES + 1

|

| --- ELSE ---

LAM_MOD_FLG = 1 -----------------|     |

|AND -| FN_INPUT = function_input - 1

function_input > 1 --------------|     | amplitude_multiplier = - AMP_MULT

|

| --- ELSE ---

|

LAM_MOD_FLG = 1 -----------------------| FN_INPUT = function_input

| amplitude_multiplier = AMP_MULT

|

| --- ELSE ---

|

| FN_INPUT = 0

| FN_GEN_TMR = 0

LAM_MOD_FLG = 1 -----------------------| lambse_mod1 = (amplitude_

|                multiplier *

|   FN339B(FN_INPUT))

| lambse_mod2 = (-1) * lambse_mod1

|

| LAM_OFFSET = FN1353A(N,LOAD)/2

|

| lambse_mod1 = lambse_mod1 +

|               LAM_OFFSET

| lambse_mod1 = min(lambse_mod1,

|                   LAM_MOD_MAX)

| LAMBSE_MOD1 = max(lambse_mod1,

|                   LAM_MOD_MIN)

| lambse_mod2 = lambse_mod2 +

|               LAM_OFFSET

| lambse_mod2 = min(lambse_mod2,

|                   LAM_MOD_MAX)

| LAMBSE_MOD2 = max(lambse_mod2,

|                   LAM_MOD_MIN)

|

| --- ELSE ---

|

| LAM_OFFSET = 0

| LAMBSE_MOD1 = 0

| LAMBSE_MOD2 = 0

8-257

FUEL STRATEGY, LAMBSE MODULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

LAM_MOD_FLG = 0 -----------------|

|OR --| LAM_CYCLES = 0

SMPL_EGO_FLG = 0 ----------------|

8-258

FUEL STRATEGY, LAMBSE MODULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.45  FEATURE:  CLOSED LOOP FUEL CONTROL - V2.8C_CLFUL_NONFLEX_96 (CDAN2)

8.45.1  CLOSED LOOP LAMBSE CONTROLLER (CDAN0)

OVERVIEW

NOTE:  In the following OVERVIEW, the term BIAS is used to describe the  bias

from  stoichiometry.   In  the  definitions and process, the term is actually

named BIAS_Gx.  BIAS_Gx has more resolution than the previous BIAS register.

The following description applies to each side of an engine if  operating  in

stereo HEGO mode.

1.  Independent closed loop ramp rates and  jumpback  (LAMBSE1  and  LAMBSE2)

amounts are calculated.

2.  Calibration values for peak-to-peak amplitude, bias, and transport  delay

are common to both sides.

3.  ANPIP1, ANPIP2, ENPIP1, ENPIP2, and LAMBSE  calculations  are  unique  to

each side.

4.  The injector  pulsewidths,  FUELPW1  and  FUELPW2,  are  calculated  from

LAMBSE1 and LAMBSE2, respectively.

NOTE:  If system uses only one HEGO sensor, the  strategy  uses  LAMBSE1  and

FUELPW1 for the desired LAMBSE and fuel Pulsewidth.  (NUMEGO = 1)

INTENT

The goals of the closed loop strategy are:

1.  To add capability  of  introducing  large  amounts  of  Air/Fuel  biasing

(expressed as ratio of bias to peak-to-peak amplitude).

2.  To maximize the feedback limit cycle frequency for all bias values.

3.  To maintain a simple calibration procedure to describe  the  closed  loop

limit cycle.

8-259

FUEL STRATEGY, CLOSED LOOP LAMBSE CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

APPROACH

The fuel flow is driven in a limit cycle manner about  stoichiometry.   Using

the  HEGO  (Heated  Exhaust  Gas  Oxygen)  sensor,  the computer increases or

decreases the injector pulsewidths in a controlled manner.  If the HEGO reads

rich,  the  pulsewidths will be decreased (made leaner) at a calculated rate.

If the HEGO reads lean, the pulsewidths will be increased (made richer) at  a

calculated rate.

When an HEGO switch occurs, an instantaneous change (or "jumpback")  is  made

in  the air/fuel ratio back towards stoichiometry.  The jump is made relative

to the A/F ratio (lambda) value at the HEGO switch.

The limit cycle can be biased to operate on the average richer (or leaner) of

stoichiometry.

8-260

FUEL STRATEGY, CLOSED LOOP LAMBSE CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

An example of the closed loop limit cycle is shown below:

LIMIT CYCLE DESCRIPTION

(lean bias example)

- - - - - - - - - *

|               * *                                           *

|             *   *                                         *

|           *     *              LEAN                     *

|         *       *                                     *

|       *         *                                   *

|     *           *                                 *

|   *       |       *                             *

| *-  -  - --- -  -  -* -  -  -  -  -  -  -  -  *  -  - AVERAGE

|                       *                       *       A/F RATIO

|                         *                     *

| --------------------------*-------------------*----- STOICHIOMETRY

|           |                 *                 *

|           |                   *               *

|         BIAS_Gx                 *             *

|         TERM                      *           *

|                                     *         *

|                         RICH          *       *

|--PEAK TO PEAK                           *     *

|  AMPLITUDE                                *   *

|                                             * *

- - - - - - - - - - - - - - - - - - - - - - - - *

HEGO SENSOR STATUS

RICH **************                             ********************

*                             *

*                             *

*                             *

LEAN              *******************************

INJECTOR PULSEWIDTH STATUS

ON **      ****     *****    ******   *******  *******    *****

*      *  *     *   *    *    *   *     *  *     *    *   *

*      *  *     *   *    *    *   *     *  *     *    *   *

*      *  *     *   *    *    *   *     *  *     *    *   *

OFF ********  *******   ******    *****     ****     ******   ******

|                             |

<--DECREASING --->|<---------INCREASING ------->|<---DECREASING --->

PULSEWIDTH     |          PULSEWIDTH         |    PULSEWIDTH

8-261

FUEL STRATEGY, CLOSED LOOP LAMBSE CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The bias term is used to make the limit cycle operate at an average A/F ratio

rich  or  lean  of  stoichiometry.   For  zero bias, the average A/F ratio is

stoichiometry.

Based on calibration information supplied by the user,  the  fuel  pulsewidth

ramp rates and jumpback distances are calculated automatically to produce the

proper limit cycle.

The direction of the bias is controlled by the sign of the  bias  value.   If

the bias term is negative, a rich bias is indicated.  If the sign of the bias

term is positive, a lean bias is indicated.

Peak-to-Peak amplitude determines the labmda  excursions  allowed  in  Closed

Loop  Fuel.   Too large an excursion will be felt as a road load surge or rpm

variations at idle.  Because of the RPM rolls at idle, the capability  exists

in the strategy to use a specific idle peak-to-peak, ISC_PTP.

System transport lag time; time delay from when a fuel change is made until

the HEGO sensor indicates this change; varies with engine speed, units are

REVs.

***********************

***** WARNING *****

***************

It is imperative that an accurate value  for  the  system  transport  lag  be

entered.   An  incorrect  value  will  result  in  greatly  reduced  catalyst

efficiencies due to excessively fast or slow ramp rates,  incorrect  jumpback

amounts, etc.

***************

***** WARNING *****

***********************

8-262

FUEL STRATEGY, CLOSED LOOP LAMBSE CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Two methods of achieving the desired bias are employed.

1.  Variable jumpback distance on one side of the limit cycle with  symmetric

pulsewidth ramp rates.

2.  No jumpback on one side of the limit  cycle  with  asymmetric  pulsewidth

ramp rates.

The  logic  used  to  achieve  biasing  depends  on  the  absolute  value  of

BIAS_G/PTPAMPx (|BIAS_G/PTPAMP|):  where:

BIAS_Gx = Amount of bias from stoichiometry.

PTPAMPx = Limit cycle peak-to-peak amplitude.

Any calculated value of |B/P| exceeding LC_RATIO, results  in  PTPAMP  to  be

increased.  This is done to avoid extremely long limit cycle periods.

FOR NO BIAS  |BIAS_G/PTPAMP| = 0

The limit cycle has full jumpback on both sides  of  stoichiometry  and  uses

symmetric pulsewidth ramp rates.

FOR SMALL BIAS  0 < |BIAS_G/PTPAMP| <= 0.171573  (0.171573 = 3 - SQRT(8))

The limit cycle has a partial  jumpback  on  one  side  of  stoichiometry  to

achieve biasing and uses symmetric pulsewidth ramp rates.

FOR LARGE BIAS  0.171573 < |BIAS_G/PTPAMP| <= LC_RATIO

The limit cycle has no  jumpback  on  one  side  of  stoichiometry  and  uses

asymmetric pulsewidth ramp rates to achieve biasing.

Examples of biasing are shown on the next page.

8-263

FUEL STRATEGY, CLOSED LOOP LAMBSE CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CLOSED LOOP LIMIT CYCLE EXAMPLES

(not drawn to scale)

NO BIAS   | BIAS_G/PTPAMPx | = 0

*       *       *       *       *

**      **      **      **      **

* *     * *     * *     * *     * *     SYMMETRIC RAMP RATES

*  *    *  *    *  *    *  *    *  *     FULL JUMPBACK IN

-----*---*---*---*---*---*---*---*---*---  BOTH DIRECTIONS

*  *    *  *    *  *    *  *    *

* *     * *     * *     * *     *

**      **      **      **      *

*       *       *       *       *

SMALL BIAS   | BIAS_G/PTPAMPx | < 0.171573

*          *          *          *

**         **         **         **

* *        * *        * *        * *         SYMMETRIC RAMP RATES

*  *       *  *       *  *       *  *         PARTIAL JUMPBACK

-----*------*---*------*---*------*---*----     IN ONE DIRECTION

*     *    *     *    *     *    *

*   *      *   *      *   *      *   *

* *        * *        * *        * *

**         **         **         **

*          *          *          *

MODERATE BIAS   | BIAS_G/PTPAMPx | = 0.171573

*           *           *           *

**          **          **          **

* *         * *         * *         * *    SYMMETRIC RAMP RATES

*  *        *  *        *  *        *  *    NO JUMPBACK

-----*-------*---*-------*---*-------*---*--  IN ONE DIRECTION

*      *    *      *    *      *    *

*     *     *     *     *     *     *

*   *       *   *       *   *       *

* *         * *         * *         *

*           *           *

LARGE BIAS   | BIAS_G/PTPAMPx | > 0.171573

*                    *

* *                  * *

*   *                *   *                *   ASYMMETRIC RAMP RATES

*              *     *              *     NO JUMPBACK

-----*------------*-------*------------*-----  IN ONE DIRECTION

*          *         *          *

*        *           *        *

*     *              *     *

*  *                 *  *

*                    *

NOTE:  Limit cycle frequency decreases with increasing bias.

8-264

FUEL STRATEGY, CLOSED LOOP LAMBSE CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

After an HEGO switch, a finite amount of time (equal to  the  transport  lag)

should  pass  before  the HEGO can switch.  Noise in the HEGO system could be

interpreted by the computer as HEGO switches.  These phantom  switches  could

occur at a faster rate than dictated by the system transport lag time.  Since

the jumpback is made from the lambda when the HEGO switches, phantom switches

could make the jumpback go beyond the average Air/Fuel ratio.  A high rate of

phantom switches would create a high rate of jumps.  A special feature of the

closed loop strategy prevents this problem.

A full jumpback is done only if the proper transport lag  time  has  elapsed.

If  not, the jumpback distance is reduced to match the reduction in transport

lag time.  The actual  strategy  uses  PIP  signal  counts  to  control  this

feature.

The jumpback distance is multiplied by the ratio piprat.

piprat = Ratio of the actual number of PIP signals since the last HEGO switch

to  the  expected  number  of PIP signals between HEGO switches; clipped to a

maximum of 1.0.  piprat is a temporary register.

piprat = ANPIPn/ENPIPn

ANPIPn = Actual number of PIP signals since the last HEGO switch; clipped  to

a  maximum  of 255; reset to one when not in closed loop; reset to zero after

the HEGO switches and the jumpback distance is calculated; n = bank.

ENPIPn = Expected number of PIP signals between HEGO  switches;  reset  to  1

when not in closed loop; n = bank.

The transport delay is entered as REVS.  The  strategy  uses  units  of  both

seconds and PIPS.  The transport delay in PIPS is:

tdpip = tdrevs * ENGCYL

Transport delay in seconds is:

tdsec = tdrevs * 60/N

tdpip and tdsec are temporary registers.

8-265

FUEL STRATEGY, CLOSED LOOP LAMBSE CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

NOTE:  In the following logic diagrams, an HEGO switch is defined as either:

1.  The HEGO sensor reads rich during the  current  background  loop  and  it

reads lean during the previous background loop.

2.  The HEGO sensor reads lean during the  current  background  loop  and  it

reads rich during the previous background loop.

The strategy copes with failed sensors in a way that attempts to minimize the

emmisions  and  driveability impacts.  When a single sensor failure in a dual

bank system occurs, control will be switched  to  the  opposite,  functioning

bank.  However the EGOSTATE will be calculated 180 degrees out of phase so as

to drive LAMBSE in opposite directions in an attempt to cancel out the torque

fluculations  that can result when both banks are are in phase.  In addition,

the downstream EGO sensor will continue to be used in the faulty bank to trim

the  lambse  that  is  being generated from the functional bank.  In this way

emmisions are kept in check during FMEM.

This strategy also has LAMBSE reset logic that will modify the ramp  rate  at

the  open  loop  to  closed  loop  transition.   This will allow coming under

control quicker and reduce the emmisions  created  during  the  [potentially]

long ramp towards stoich.  The normal ramp calculation is of the form:

LAMBSE = LAMBSE + (ramp_rate * delta_time)

This is being modified to:

LAMBSE = LAMBSE + (RR_MULTn * (ramp_rate * delta_time))

Where the RR_MULTn is calculated a function of  the  difference  between  the

current  LAMBSE  and the LAMBSE of one.  Once the value of one is attained or

an EGO switch occurs, the RR_MULTn is held at a  value  of  one  and  has  no

effect.  The calculation of RR_MULTn is as follows:

RR_MULTn = abs(1 - LAMBSE) * RR_MULT_GAIN + 1

A suggested value for RR_MULT_GAIN of 32  will  give  fast  reset  times  and

little  or  no overshoot of the stoich point, depending of stack-up of system

errors.

8-266

FUEL STRATEGY, CLOSED LOOP LAMBSE CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The following calling structure applies to the processes found in this file:

PROCESS                                LOCATION

-----------------------------          ----------------------

pid_definitions                        CLFUL_CONTROLLER_COM

closed_loop_fuel_control               CLFUL_CONTROLLER_COM

egostate_calc                    CLFUL_CONTROLLER_COM

bias_lookup                      CLFUL_BIAS_COM

ptpamp_lookup                    CLFUL_PTPAMP_COM

transport_delay_lookup           CLFUL_TRANS_DELAY_COM

fmem_calculations                CLFUL_CONTROLLER_COM

fuel_state_control               CLFUL_CONTROLLER_COM

jump_calc()                CLFUL_CONTROLLER_COM

ramp_calc()                CLFUL_CONTROLLER_COM

anpip_control                          CLFUL_CONTROLLER_COM

closed_loop_fuel_init                  CLFUL_CONTROLLER_COM

8-267

FUEL STRATEGY, CLOSED LOOP LAMBSE CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  ANPIPn = Actual number of PIP signals since the last HEGO switch; clipped

to  a maximum of 255; reset to one when not in closed loop; reset to zero

after the HEGO switches and the jumpback  distance  is  calculated;  n  =

bank.

-  BIAS_Gx = Closed Loop biasing term = FN1353E(N,LOAD).

-  ENPIPn = Expected number of PIP signals between HEGO switches; reset to 1

when not in closed loop; n = bank.

-  EGOSTATE1 = State of the EGO sensor 11.

-  EGOSTATE2 = State of the EGO sensor 21.

-  FAILED_BANK = Register indicating which bank is in lack of EGO switching.

-  FUELPW1 = HEGO11 injector pulsewidths, clock ticks.

-  FUELPW2 = HEGO21 injector pulsewidths, clock ticks.

-  KAM_ERROR = Indicates Keep Alive RAM invalid.

-  LAMBSE1 = Desired open loop (or closed loop) equivalence ratio for  Bank1

injectors.  LAMBSE1 appears in the fuel pulsewidth equation for HEGO11.

-  LAMBSE2 = Desired open loop (or closed loop) equivalence ratio for  Bank2

injectors.  LAMBSE2 appears in the fuel pulsewidth equation for HEGO21.

-  LOAD = Universal normalized load parameter.

-  MINPWT = MINPW converted to clock ticks.

-  N = Engine RPM.

-  OV_TMR_11 = EGO11 onervoltage timer.

-  OV_TMR_21 = EGO21 overvoltage timer.

-  PM = Normalized value of percentage of methanol, unitless.

-  PTPAMP1 = Limit cycle peak_to_peak amplitude for bank 1.

-  PTPAMP2 = Limit cycle peak_to_peak amplitude for bank 2.

-  R_BIAS1 = Rear BIAS trim for bank 1.

-  R_BIAS2 = Rear BIAS trim for bank 2.

-  RR_MULT1 = Bank 1 Closed Loop Ramp  Rate  Multiplier  -  serviced  within

LAMBSE reset logic.

-  RR_MULT2 = Bank 2 Closed Loop Ramp  Rate  Multiplier  -  serviced  within

LAMBSE reset logic.

8-268

FUEL STRATEGY, CLOSED LOOP LAMBSE CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TSLAMU1 = Time at which last LAMBSE1 update occurred, clock ticks.

-  TSLAMU2 = Time at which last LAMBSE2 update occurred, clock ticks.

Bit Flags:

-  EGOFL11 = Bank1 upstream HEGO flag.

-  EGOFL21 = Bank2 upstream HEGO flag.

-  FFG_CSD11 = Fault flag for CSD on sensor 11.

-  FFG_CSD21 = Fault flag for CSD on sensor 21.

-  LEGOFG11 = Lack of HEGO11 switching flag; 1 -> HEGO11 not switching.

-  LEGOFG21 = Lack of HEGO21 switching flag; 1 -> HEGO21 not switching.

-  LAM_MOD_FLG = Flag used to initiate high frequency  fuel  modulation  for

the upstream EGO monitor.

-  OLFLG = Open loop fuel flag; 1 -> open loop fuel, 0 -> closed loop fuel.

-  RESET_LEAN1 = Flag to indicate wheter LAMBSE1 was lean or rich upon  last

transition; 1 -> lean, 0 -> rich.

-  RESET_LEAN2 = Flag to indicate wheter LAMBSE2 was lean or rich upon  last

OL_CL transition; 1 -> lean, 0 -> rich.

-  RESET_REQ1 = Flag passed from LAMBSE reset to closed loop logic to enable

use of ramp rate multiplier for Bank 1.

-  RESET_REQ2 = Flag passed from LAMBSE reset to closed loop logic to enable

use of ramp rate multiplier for Bank 2.

-  SWTFL11 = Bank1 upstream HEGO11 switch flag; 0 -> no HEGO11 switch, 1  ->

HEGO11 switch.

-  SWTFL21 = Bank2 upstream HEGO21 switch flag; 0 -> no HEGO21 switch, 1  ->

HEGO21 switch.

Calibration Constants:

-  ENGCYL = Number of PIPS per engine revolution; or Number of cylinders/2.

-  FN339 = Closed  Loop  ramp  rate  Multiplier  versus  absolute  value  of

BIAS_G/PTPAMP.

Input = |BIAS_G/PTPAMP|.

Output = Ramp rate multiplier.

-  FN342 = Closed Loop Jumpback distance Multiplier  versus  |BIAS_G/PTPAMP|

(Toward BIAS_G).

Input = |BIAS_G/PTPAMP|.

Output = Jumpback distance Multiplier.

-  FN344 = Closed Loop Jumpback Distance Multiplier  versus  |BIAS_G/PTPAMP|

(Opposite BIAS_G).

Input = |BIAS_G/PTPAMP|.

Output = Jumpback distance multiplier.

8-269

FUEL STRATEGY, CLOSED LOOP LAMBSE CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FN346 = Expected number of PIPs multiplier.

Input = |BIAS_G/PTPAMP|.

Output = Multiplier.

-  LAMMAX = Max Lambse clip when closed loop.

-  LAMMIN = Min Lambse clip when closed loop.

-  NUMEGO = Number of HEGO sensors present; mono or stereo only.

-  RR_MULT_GAIN = roportional gain for the control of closed loop ramp  rate

multipliers serviced in LAMBSE reset.

8-270

FUEL STRATEGY, CLOSED LOOP LAMBSE CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CLFUL_CONTROLLER_COM1

BEGIN:  pid_definitions

unconditionally ---------------------| short_trim_1 := 128 / LAMBSE1

unconditionally ---------------------| short_trim_2 := 128 / LAMBSE2

pid_def(j1979_01_06, short_trim_1 )

pid_def(j1979_01_08, short_trim_2 )

END:  pid_definitions

BEGIN: closed_loop_fuel_control

;Execute the following once per background pass.

KAM_ERROR = 1 --------------| FAILED_BANK := 0

unconditionally ------------| failed_bank := FAILED_BANK

OLFLG = 0 ------------|

|

LEGOFG21 = 1 ---|     |AND -| FAILED_BANK := 2

|     |     |   ;mono HEGO or stereo

FFG_CSD21 = 1 --|OR --|     |   ;HEGO with lack of

|           |   ;HEGO21 switching;

NUMEGO = 1 -----|           |   ;use HEGO11 sensor

|           |   ;only for closed loop

OV_TMR_21 > 0 --|           |   ;control of LAMBSE1 and

|   ;LAMBSE2.

| Do: egostate_calc

| Do: bias_lookup

| Do: ptpamp_lookup

| Do: transport_delay_lookup

| Do: fmem_calculations

| Do: fuel_state_control

|

| --- ELSE ---

(continued on next page)

8-271

FUEL STRATEGY, CLOSED LOOP LAMBSE CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

OLFLG = 0 ------------|     |

|AND -| FAILED_BANK := 1

LEGOFG11 = 1 ---|     |     |   ;stereo HEGO with lack

|OR --|     |   ;of HEGO11 switching.

FFG_CSD11 = 1 --|           |   ;use HEGO21 sensor

|           |   ;only for closed loop

OV_TMR_11 > 0 --|           |   ;control of LAMBSE2 and

|   ;LAMBSE1.

| Do: egostate_calc

| Do: bias_lookup

| Do: ptpamp_lookup

| Do: transport_delay_lookup

| Do: fmem_calculations

| Do: fuel_state_control

|

| --- ELSE ---

|

OLFLG = 0 ------------------| FAILED_BANK := 0

|   ;normal stereo HEGO.

|   ;use HEGO11 sensor

|   ;for closed loop

|   ;control of LAMBSE1

|   ;        and

|   ;use HEGO21 sensor

|   ;for closed loop

|   ;control of LAMBSE2.

| Do: egostate_calc

| Do: bias_lookup

| Do: ptpamp_lookup

| Do: transport_delay_lookup

| Do: fmem_calculations

| Do: fuel_state_control

END: closed_loop_fuel_control

8-272

FUEL STRATEGY, CLOSED LOOP LAMBSE CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: fuel_state_control

FAILED_BANK <> 0 -----------|

|AND -| LAMBSE(FAILED_BANK) := 1

failed_bank <> FAILED_BANK -|     | ANPIP(FAILED_BANK) := 0

|   ;reset LAMBSE to 1 if a fault

|   ;is detected.

For bank = 1 and 2 perform the following logic:

swtfl(bank) = 1 ------------------| Do: jump_calc(bank)

|

|  -- ELSE --

|

| Do: ramp_calc(bank)

unconditionally  -----------------| lambse := min(lambse,LAMMAX)

| LAMBSE(bank) :=

|           max(lambse,LAMMIN)

|

| TSLAMU(bank) := clock

;end for loop.

END: fuel_state_control

BEGIN: egostate_calc

EGOFL11 = 1 ----------------| EGOSTATE1 := 1

|

|   -- ELSE --

|

| EGOSTATE1 := -1

EGOFL21 = 1 ----------------| EGOSTATE2 := 1

|

|   -- ELSE --

|

| EGOSTATE2 := -1

END: egostate_calc

8-273

FUEL STRATEGY, CLOSED LOOP LAMBSE CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: fmem_calculations

;The following process, called from the closed_loop_fuel_control

;process is used to substitute fmem values when single bank failures

;are present.

FAILED_BANK = 1 ------|

|

NUMEGO = 2 -----------|AND -| egostate1 := (-1) * EGOSTATE2

|     | egostate2 := EGOSTATE2

HEGO_CONFIG <> 0 -----|     | swtfl1 := SWTFL21

;a rear ego exists         | swtfl2 := SWTFL21

| fmem_offset1 := R_BIAS1

| fmem_offset2 := 0

|

| --- ELSE ---

|

FAILED_BANK = 2 ------|     |

|AND -| egostate2 := (-1) * EGOSTATE1

HEGO_CONFIG <> 0 -----|     | egostate1 := EGOSTATE1

;a rear ego exists         | swtfl2 := SWTFL11

| swtfl1 := SWTFL11

| fmem_offset2 := R_BIAS2

| fmem_offset1 := 0

|

| --- ELSE ---

FAILED_BANK = 1 ------|     |

|AND -| egostate2 := EGOSTATE2

NUMEGO = 2 -----------|     | egostate1 := EGOSTATE2

;no rear ego exists        | swtfl2 := SWTFL21

| swtfl1 := SWTFL21

| fmem_offset2 := 0

| fmem_offset1 := 0

|

| --- ELSE ---

|

FAILED_BANK = 2 ------------| egostate2 := EGOSTATE1

;no rear ego exists        | egostate1 := EGOSTATE1

| swtfl2 := SWTFL11

| swtfl1 := SWTFL11

| fmem_offset2 := 0

| fmem_offset1 := 0

|

| --- ELSE ---

|

| egostate2 := EGOSTATE2

| egostate1 := EGOSTATE1

| swtfl2 := SWTFL21

| swtfl1 := SWTFL11

| fmem_offset2 := 0

| fmem_offset1 := 0

END: fmem_calculations

8-274

FUEL STRATEGY, CLOSED LOOP LAMBSE CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: jump_calc(bank)

;Jumpback and ENPIP calculations based on bias and HEGO state:

FAILED_BANK = bank ------------| lambse := 1 + fmem_offset(bank)

| ENPIP(bank) := 1

| RR_MULT(bank) := 1

| RESET_REQ(bank) := 0

| Exit this process

|

| --- ELSE ---

|

| piprat(bank) := min((ANPIP(bank)

|                 / ENPIP(bank)), 1)

|   ;otherwise restrict the jumpback to

|   :one half of PTPAMP

unconditionally ---------------| ANPIP(bank) := 0

| b_over_p := abs(BIAS_G(bank))/PTPAMP(bank)

| RR_MULT(bank) := 1

| RESET_REQ(bank) := 0

|  ;switch occured, stop

|  ;reset, if in progress

|  ;and reset actual PIP counter.

BIAS_G(bank) >= 0 --|

|AND -|

egostate(bank) = 1 -|     |

|OR --| lambse := LAMBSE(bank) +

BIAS_G(bank) < 0 ---|     |     |        (egostate(bank) * (PTPAMP(bank)

|AND -|     |        + abs(BIAS_G(bank)))

egostate(bank) = -1-|           |        * FN344(b_over_p)

|        * piprat(bank))

|

| ENPIP(bank) := tdpip(bank)

|

| --- ELSE ---

|

| lambse := LAMBSE(bank) +

|        (egostate(bank) * PTPAMP(bank)

|        * FN342(b_over_p)

|        * piprat(bank))

|

| ENPIP(bank) := tdpip(bank)

|                * FN346(b_over_p)

END: jump_calc

8-275

FUEL STRATEGY, CLOSED LOOP LAMBSE CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ramp_calc(bank)

;Ramp-rate calculations based on bias and HEGO state:

;(executed when the HEGO(bank)1 sensor has not switched)

RESET_LEAN(bank) = 0 -|

;coming from rich   |

;LAMBSE             |AND -|

|     |

LAMBSE(bank) < 1.0 ---|     |

;still rich               |OR --|

|     |

RESET_LEAN(bank) = 1 -|     |     |

;coming from lean   |     |     |

;LAMBSE             |AND -|     |AND -| RR_MULT(bank) :=

|           |     |      (abs(1-LAMBSE(bank)) *

LAMBSE(bank) > 1.0 ---|           |     |       RR_MULT_GAIN) + 1

;still lean                     |     |   ;calculate the ramp rate mult.

|     |   ;based on the distance between

RESET_REQ(bank) = 1 --------------|     |   ;LAMBSE and 1 and the calibrated

|   ;gain.

|   ;This feature allows a fast ramp

|   ;back to stoich when entering

|   ;closed loop from open loop and

|   ;eliminates the need for an

|   ;uncontrolled jump to 1.0

|

| --- ELSE ---

|

| RR_MULT(bank) := 1

| RESET_REQ(bank) := 0

|   ;stop the lambse reset,

|   ;LAMBSE >=1 or  a switch

|   ;occured.

unconditionally ------------------| b_over_p := abs(BIAS_G(bank))

|             /PTPAMP(bank)

|

| p_minus_b := PTPAMP(bank)

|              - abs(BIAS_G(bank))

| p_minus_b := max(0, p_minus_b)

|  ;clip this difference to 0 to ensure

|  ;limit cycle stability for all

|  ;combinations of bias and PTPAMP.

8-276

FUEL STRATEGY, CLOSED LOOP LAMBSE CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BIAS_G(bank) >= 0 -----|

|AND -|

egostate(bank) = -1 ---|     |

|OR --| lambse := LAMBSE(bank) +

BIAS_G(bank) < 0 ------|     |     |        (egostate(bank)

|     |     |        * p_minus_b

egostate(bank) = 1 ----|AND -|     |        * (1 - FN339(b_over_p)

|           |        * (clock - TSLAMU(bank))

FUELPW(bank) > MINPWT -|           |        * RR_MULT(bank)

|        / tdsec(bank))

|

| --- ELSE ---

|

FUELPW(bank) > MINPWT -------------| lambse := LAMBSE(bank) +

|        (egostate(bank)

|        * p_minus_b

|        * FN339(b_over_p)

|        * (clock - TSLAMU(bank))

|        * RR_MULT(bank)

|        / tdsec(bank))

|

| --- ELSE ---

|

| lambse := LAMBSE(bank)

RESET_REQ(bank) = 1 ----------|

|

lambse > 1 -------|           |AND -| RESET_REQ(bank) := 0

|AND -|     |     | RR_MULT(bank) := 1

LAMBSE(bank) < 1 -|     |     |     | lambse := 1

|OR --|     |   ;lambse has crossed one

lambse < 1 -------|     |           |   ;while a reset was in progress.

|AND -|           |   ;Stop the accelerated

LAMBSE(bank) > 1 -|                 |   ;ramp and set lambse to one.

END: ramp_calc

8-277

FUEL STRATEGY, CLOSED LOOP LAMBSE CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: anpip_control

;Execute the following on the rising edge of PIP.

unconditionally -------------------| ANPIP1 := min((ANPIP1 + 1),255)

| ANPIP2 := min((ANPIP2 + 1),255)

END: anpip_control

BEGIN: closed_loop_fuel_init

;Perform the following during RAM initialization

unconditionally -------------------| ANPIP1 := 1

| ANPIP2 := 1

| ENPIP1 := 1

| ENPIP2 := 1

| LAMBSE1 := 1

| LAMBSE2 := 1

| TSLAMU1 := 0

| TSLAMU2 := 0

| EGOSTATE1 := 0

| EGOSTATE2 := 0

END: closed_loop_fuel_init

8-278

FUEL STRATEGY, CLOSED LOOP LAMBSE CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.45.2  CLOSED LOOP LAMBSE CALCULATION (CDAN2)

OVERVIEW

This module is part of the closed loop fuel control.  It is used to  look  up

the closed loop fuel bias from a table that is a function of engine speed and

engine load.  A full description of how the bias affects the closed loop fuel

signal is given in the closed loop fuel control module.

This module supports the following features:

1.  Normal Bias:  FN1353E(N,LOAD), ,FN1353EB2(N,LOAD), FN315(VSBAR)

2.  EGO Monitor Bias:  0.

|     3.  Provides for  a  calibratable  idle  speed  bias  clip  ISC_BIAS_MAX  and

|     ISC_BIAS_MIN.

This module will calculate bias for fore  aft  oxygen  sensor  control,  full

OBD-II vehicles with the FTP catalyst monitor.

Block Diagram of Bias Look-up Module

+--------------+

LAM_MOD_FLG --->|              |

|              |

R_BIAS1/2 ----->|              +---> BIAS_G1

|              |

N ------------->| Bias Look-up |

|              |

LOAD ---------->|              +---> BIAS_G2

|              |

VSBAR --------->|              |

|                     |              |

|     ISCFLG -------->|              |

|                     |              |

|     ISC_PTP_SW ---->|              |

+--------------+

DEFINITIONS

Registers:

-  BIAS_G1 = Closed Loop biasing term for bank 1.

-  BIAS_G2 = Closed Loop biasing term for bank 2.

8-279

FUEL STRATEGY, CLOSED LOOP LAMBSE CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|      -  ISCFLG = ISC mode flag 1 -> RPM control mode.

-  LOAD = Universal load as ratio of aircharge over standard.

-  N = Engine RPM.

-  R_BIAS1 = Rear bias trim for bank 1.

-  R_BIAS2 = Rear bias trim for bank 2.

-  VSBAR = Filtered vehicle speed.

Bit Flags:

-  LAM_MOD_FLG = Flag used to initiate high frequency  fuel  modulation  for

the upstream EGO monitor.

Calibration Constants:

-  BIAS_MAX = Maximum total bias allowed.

-  BIAS_MIN = Minimum total bias allowed.

-  FN021 = Load normalizing function; Input  =  Load,  Output  =  Normalized

Load.

-  FN039E = Speed normalizing function for BIAS table.

-  FN315 = A/F bias multiplier in part throttle mode as a function of VSBAR.

-  FN1353E = Amount of Bias from stoichiometry for bank 1.

X = FN039E(N)

Y = FN021(LOAD)

Output = Bias from stoichiometry. (Store in register).

-  FN1353EB2 = Amount of Bias from stoichiometry for bank 2.

X = FN039E(N)

Y = FN021(LOAD)

Output = Bias from stoichiometry. (Store in register).

|      -  ISC_BIAS_MAX = Idle speed bias maximum clip.

|      -  ISC_BIAS_MIN = Idle speed bias minimum clip.

|      -  ISC_PTP_SW = Flag for idle speed peak-to-peak value to be used;

|                      1 -> use ISC_PTP.

8-280

FUEL STRATEGY, CLOSED LOOP LAMBSE CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CLFUL_BIAS_COM4

BEGIN: bias_lookup

LAM_MOD_FLG = 1 ------------------------| BIAS_G1 := 0

;HEGO monitor in progress              | BIAS_G2 := 0

|

| --- ELSE ---

|

| bias1 := FN1353E(N,LOAD) *

|             FN315(VSBAR)

| bias1 := bias1 + R_BIAS1

| bias1 := min(bias1, BIAS_MAX)

| BIAS_G1 := max(bias1, BIAS_MIN)

|

| bias2 := FN1353EB2(N,LOAD) *

|               FN315(VSBAR)

| bias2 := bias2 + R_BIAS2

| bias2 := min(bias2, BIAS_MAX)

| BIAS_G2 := max(bias2, BIAS_MIN)

|     ISC_PTP_SW = 1 --------------|

|      ;Idle speed PTP value       |

|      ;desired                    |

|                                  | AND -----| BIAS_G1 :=

|                                  |          |         min(BIAS_G1, ISC_BIAS_MAX)

|                                  |          | BIAS_G1 :=

|                                  |          |         max(BIAS_G1, ISC_BIAS_MIN)

|     ISCFLG > 0 ------------------|          |

|                                             | BIAS_G2 :=

|                                             |         min(BIAS_G2, ISC_BIAS_MAX)

|                                             | BIAS_G2 :=

|                                             |         max(BIAS_G2, ISC_BIAS_MIN)

END: bias_lookup

8-281

FUEL STRATEGY, CLOSED LOOP LAMBSE CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.45.3  CLOSED LOOP LAMBSE CALCULATION (CDAI0)

OVERVIEW

This module is part of the closed loop fuel control.  It is used to  look  up

the  closed  loop fuel peak to peak amplitude from a table that is a function

of engine speed and engine load.  A full description of how the peak to  peak

amplitude  affects  the  closed  loop fuel signal is given in the closed loop

fuel control module.

This module supports the following features:

1.  Normal PTPAMP:  FN1352(N,LOAD)

2.  Idle PTPAMP:  ISC_PTP

This module will calculate bias for fore  aft  oxygen  sensor  control,  full

OBD-II vehicles, FTP Catalyst monitor.

Block Diagram of PTPAMP Look-up Module

+--------------+

BIAS_G1 ------->|              |

|              |

BIAS_G2 ------->|              |

|              |

ISCFLG -------->|              +---> PTPAMP1

|              |

N ------------->|    PTPAMP    |

|    Look up   |

LOAD ---------->|              +---> PTPAMP2

|              |

ISC_PTP_SW ---->|              |

|              |

LC_RATIO ------>|              |

+--------------+

8-282

FUEL STRATEGY, CLOSED LOOP LAMBSE CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  ISCFLG = ISC MODE Flag; 1 -> RPM CONTROL mode.

-  LOAD = Universal normalized load parameter.

-  N = Engine RPM.

Calibration Constants:

-  FN021 = Load normalizing function; Input  =  Load,  Output  =  Normalized

Load.

-  FN039 = Engine speed (N) normalizing function.

-  FN1352 = Closed Loop Peak-to-Peak amplitude, units are lambdas.

X = FN039(N)

Y = FN021(LOAD)

Output = Peak-to-Peak amplitude, PTPAMP.

-  LC_RATIO  =  Maximum  BIAS_G/PTPAMP  ratio  allowed  before   PTPAMP   is

increased.

-  ISC_PTP = Idle speed peak-to-peak value.

-  ISC_PTP_SW = Idle speed peak-to-peak value to be used at idle flag; 1  ->

use ISC_PTP.

-  PTPAMP_MAX = Maximum value of PTPAMP allowed.

OUTPUTS

Registers:

-  BIAS_G1 = Closed loop fuel bias Bank 1.

-  BIAS_G2 = Closed loop fuel bias Bank 2.

-  PTPAMP1 = Limit cycle peak_to_peak amplitude,bank 1.

-  PTPAMP2 = Limit cycle peak_to_peak amplitude, bank 2.

8-283

FUEL STRATEGY, CLOSED LOOP LAMBSE CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CLFUL_PTPAMP_COM4

Calculation of PTPAMP

BEGIN: ptpamp_lookup

ISCFLG > 0 -----------------------|

;idle speed RPM                  |

;control                         |AND -| ptpamp_temp := ISC_PTP

|     |

ISC_PTP_SW = 1 -------------------|     |

;idle speed PTP value desired          |

| --- ELSE ---

|

| ptpamp_temp := FN1352(N,LOAD)

|BIAS_G1/ptpamp_temp| > LC_RATIO -------| ptpamp_temp1 = |BIAS_G1/LC_RATIO|

| PTPAMP1 = min(ptpamp_temp1,

|                PTPAMP_MAX)

|

| --- ELSE ---

|

| PTPAMP1 = ptpamp_temp

|BIAS_G2/ptpamp_temp| > LC_RATIO -------| ptpamp_temp2 = |BIAS_G2/LC_RATIO|

| PTPAMP2 = min(ptpamp_temp2,

|                PTPAMP_MAX)

|

| --- ELSE ---

|

| PTPAMP2 = ptpamp_temp

END: ptpamp_lookup

8-284

FUEL STRATEGY, CLOSED LOOP LAMBSE CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.45.4  TRANSPORT DELAY LOOKUP (CDAI0)

OVERVIEW

This module is part of the closed loop fuel control.  It is used to  look  up

the  closed  loop  fuel  transport  delay  from a table that is a function of

engine speed and engine load.  A full description of how the transport  delay

affects  the closed loop fuel signal is given in the closed loop fuel control

module.

This module supports the following features:

1.  Normal Transport Delay:  FN1351(N,LOAD), TDMULT, FN346A(ECT)

2.  EGO Monitor Transport Delay:  FN1351E(N,LOAD)

This module will calculate transport delay for flexible fuel, fore aft oxygen

sensor control, full OBD-II vehicles with different delays from bank to bank,

FTP Catalyst monitor.

Block Diagram of Transport Delay Look-up Module

+--------------+

LAM_MOD_FLG --->|              |

|              +---> tdpip1

ECT ----------->|              +---> tdsec1

|   Transport  |

N ------------->|    Delay     |

|   Look-up    +---> tdpip2

LOAD ---------->|              +---> tdsec2

|              |

+--------------+

8-285

FUEL STRATEGY, CLOSED LOOP LAMBSE CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  ECT = Engine Coolant Temperature, deg F.

-  LOAD = Universal normalized load parameter.

-  N = Engine RPM.

Bit Flags:

-  CAT_TST_PRG = Flag indicating start  of  catalyst  test;  1  ->  test  in

progress, 0 -> no catalyst test.

-  LAM_MOD_FLG = Flag used to initiate high frequency  fuel  modulation  for

the upstream EGO monitor.

Calibration Constants:

-  ENGCYL = Number of PIPs per engine revolution; or Number of cylinders/2.

-  FN021 = Load normalizing function; Input  =  Load,  Output  =  Normalized

Load.

-  FN039 = Engine speed (N) normalizing function.

-  FN346A(ECT) = Cold engine multiplier.

-  FN1351 = System transport lag time; time delay from when a fuel change is

made until the HEGO sensor indicates this change, units are REVs.

X = FN039(N)

Y = FN021(LOAD)

Output = Transport delay, REVs.

-  TDMULT = Transport time delay multiplier for bank 2; tdrevs2 =  FN1351  *

TDMULT.

OUTPUTS

Registers:

-  tdpip1 = transport delay in pips for bank1.

-  tdpip2 = transport delay in pips for bank2.

-  tdsec1 = transport delay in seconds for bank 1.

-  tdsec2 = transport delay in seconds for bank 2.

8-286

FUEL STRATEGY, CLOSED LOOP LAMBSE CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CLFUL_TRANS_DELAY_COM2

Calculation of tdpip and tdsec

BEGIN: transport_delay_lookup

LAM_MOD_FLG = 1 ------------------------| tdpip1 := FN1351E(N, LOAD) * ENGCYL

;HEGO monitor in progress              | tdsec1 := FN1351E(N, LOAD) * 60/N

| tdpip2 := tdpip1

| tdsec2 := tdsec1

|

| --- ELSE ---

|

| tdpip1 := FN1351(N,LOAD) * ENGCYL *

|            FN346A(ECT)

| tdsec1 := FN1351(N,LOAD) * 60/N *

|            FN346A(ECT)

| tdpip2 := tdpip1 * TDMULT

| tdsec2 := tdsec1 * TDMULT

END: transport_delay_lookup

8-287

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.45.5  REAR EGO TRIM (CDAN2)

8.45.5.1  PROCEDURE Faosc_init

|     BEGIN:  faosc_init

|     FUNCTIONAL REQUIREMENTS

This module calculates the BIAS trim on the front HEGOs, when Fore-Aft Oxygen

Sensor  Control  is  enabled.   The output of this module is R_BIAS1/R_BIAS2,

which are used in the closed loop fuel calculation.

|     This module takes the downstream HEGO voltage and calculates a  R_BIAS  using

|     Proportional  and  Integral  control.   The R_BIAS value will keep the closed

|     loop fuel control operating in the catalyst window and  therefore  allow  the

|     control system to compensate for aged front HEGOs.

How FAOSC is implemented in a vehicle:

|                                      +---------+

_/|         |\_

|                                   | /|         |\ |

| /| ENGINE  |\ |<----------+

| /|         |\ |           |

| /|         |\ |           |

| |+---------+| |           |

| |  |     |  | |           |

| |   |   |   | |           |

|O|    ---    |O|           |

| |           | |           |

| |           | |           |

|   |         |   |          |

| C |         | C |          |

| 1 |         | 2 |          |

|   |         |   |          |

| |           | |           |

| |           | |           |

HEGO12  |O|---+   +---|O|  HEGO22   |

| |   |   |   | |           |

|   |                 |

V   V                 |

+-------+               |

|                                    ___|_______|___            |

|                                   |               |           |

|      EEC      |----->-----+

|               |  R_BIAS1/R_BIAS2

|               |

+---------------+

The control of the FAOSC module can be modeled with a simple P/I controller:

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-288

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|                                   +-----------+

+------>|   Ki/s    |--------+

desired   +          |       +-----------+        | R_BIAS  +---------+

VEGO ------>[ ]------|                            |-------->| ENGINE  |---+

-  A       |       +-----------+        |         +---------+   |

VEGO |       +------>|    Kp     |--------+                       |

|               +-----------+                                |

|                      +-----------+                         |

+----------------------| REAR HEGO |<------------------------+

+-----------+

This diagram shows the integral and proportional values are determined from

the error in the downstream ego voltage. The implementation of this module,

|     however, allows for the calibration of the integral and proportional

|     controllers on the basis of measured downstream VEGO only. This is done

|     through functions FNFOASCI for the intergral control and FNFAOSCP for the

proportional control.

|     The following is an example of integral control (FNFAOSCI) centered around

0.45v with no dead band and a constant gain.

FIGURE 1: FNFAOSCI (bias/min)

^

|                                          |

0.001****************************

|                                          |

|                                          |

|                                          |

|                                          |

< -----------------------+---------------------------------->

|                                          0      VEGO_BAR_Ix2 - FNVEGOSP(N,LOAD)

|                                          |

|                                          |

|                                          |

************************|-0.001

v

FNFAOSCI with a small dead band around the setpoint.

FIGURE 2: FNFAOSCI (bias/min)

^

|                                          |

0.001     ****************************

|                                          |    *

|                                          |    *

|                                          |    *

|    *

< ------------------***********----------------------------->

|                                     *    0      VEGO_BAR_x2 - FNVEGOSP(N,LOAD)

|                                     *    |

|                                     *    |

|                                     *    |

********************    |-0.001

v

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-289

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The proportional control can work in conjunction with the integral control

to provide a better overall system. The proportional control is generated

from FNFAOSCP.

|     The following is an example of a proportional control that would

correspond with the integral control of FIGURE 2:

|                              FIGURE 3: FNFAOSCP (bias)

^

|                                          |

0.001****************************

|                                          |

|                                          |

|                                          |

|                                          |

< -----------------------+---------------------------------->

0      VEGOx2_BAR - FNVEGOSP(N,LOAD)

|                                          |

|                                          |

|                                          |

************************|-0.001

v

|     When the proportional and integral controls are working in parallel with an

|     integral dead ban (FIGURES 2 & 3), the proportional will keep VEGO centered

|     around 0.45v when VEGO is in the dead band and the integral will push the

voltage back into the dead band if the signal drifts out of the window.

The module also features a running average of the integral control. This

|     allows the system to compensate for the aged HEGO drift even though the

system is not in the learning window of operation. This average is also used

as the starting point of the integral control when the conditions are right

for continued learning.

|     In summary, the integral control provides the necessay R_BIAS to keep the

front fuel control loop operating in the catalyst window, the proportional

|     fine tunes the R_BIAS to center or oscillate around a point within this

|     window, and the average of the integral control allows the system to

adjust to an aged or drifted HEGO over a period of time.

;Perform during raminit.

|     LOGICAL REQUIREMENTS

|     Logic Diagram

|     This structure diagram shows calling relations between

|     logic charts in this process.

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-290

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|          LEVEL     ENTITY CALLED

|            1       Chart faosc_init_0

|     EXTERNAL INTERFACE

|     Inputs:

|      -  R_BIAS_AVG1

|      -  R_BIAS_AVG2

|     Outputs:

|      -  FAOS_DT1

|      -  FAOS_DT2

|      -  R_BIAS_INT1

|      -  R_BIAS_INT2

|     PARAMETER DECLARATIONS

|     Registers:

|      -  FAOS_DT1 = ELAPSED TIME SINCE THE UPDATE OF THE AVERAGE.

|            Resolution: 0.000977    Units: seconds          Init. Value: 0.050000

|            Min. Value: 0.000000    Max. Value: 63.999001   Reg. Type: RAM

|      -  FAOS_DT2 = ELAPSED TIME SINCE THE UPDATE OF THE AVERAGE.

|            Resolution: 0.000977    Units: seconds          Init. Value: 0.050000

|            Min. Value: 0.000000    Max. Value: 63.990002   Reg. Type: RAM

|      -  R_BIAS_AVG1 = THE AVERAGE VALUE OF THE INTEGRAL VALUE OF R_BIAS1

|            Resolution: 0.000122    Units: UNITLESS         Init. Value: 0.000000

|            Min. Value: -3.990000   Max. Value: 3.990000    Reg. Type: KAM

|      -  R_BIAS_AVG2 = THE AVERAGE VALUE OF THE INTEGRAL VALUE OF R_BIAS2

|            Resolution: 0.000122    Units: UNITLESS         Init. Value: 0.000000

|            Min. Value: -3.990000   Max. Value: 3.990000    Reg. Type: KAM

|      -  R_BIAS_INT1 = THE INTEGRAL VALUE OF R_BIAS1

|            Resolution: 0.000122    Units: UNITLESS         Init. Value: 0.000000

|            Min. Value: -3.990000   Max. Value: 3.990000    Reg. Type: RAM

|      -  R_BIAS_INT2 = THE INTEGRAL VALUE OF R_BIAS2

|            Resolution: 0.000122    Units: UNITLESS         Init. Value: 0.000000

|            Min. Value: -3.990000   Max. Value: 3.990000    Reg. Type: RAM

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-291

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|     Logic Chart faosc_init_0

|     Unconditionally -----------| FAOS_DT1 := .05

|                                |

|                                | FAOS_DT2 := .05

|                                |  ;50 milliseconds is used as a representative

|                                |  ;background loop time.

|                                |

|                                | R_BIAS_INT1 := R_BIAS_AVG1

|                                |

|                                | R_BIAS_INT2 := R_BIAS_AVG2

|                                |

END:  faosc_init

8.45.5.2  PROCEDURE Faosc2_main

BEGIN:  faosc2_main

|     FUNCTIONAL REQUIREMENTS

;Execute routines that must be run every loop, regardless:

|     LOGICAL REQUIREMENTS

|     Logic Diagram

|     This structure diagram shows calling relations between logic charts  in  this

|     process.

|          LEVEL     ENTITY CALLED

|            1       Chart faosc2_main_0

|            2          Procedure closed_loop_timer

|            2          Procedure faosc_scp_report

|            2          Procedure fault_lockout

|            2          Procedure vego_filter

|            2          Procedure vego_ratchet

|            2          Function faosc_avg

|            2          Function faosc_p_control

|            2          Function learn_timer

|            2          Function output_routing

|            2          Function rego_ok_logic

|     EXTERNAL INTERFACE

|     Inputs:

|     No parameters are applicable.

|     Outputs:

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-292

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|     No parameters are applicable.

|     PARAMETER DECLARATIONS

|     No parameters applicable.

|     Logic Chart faosc2_main_0

|     Unconditionally -----------------------| DO: vego_filter

|                                            |

|                                            | DO: vego_ratchet

|                                            |

|                                            | DO: fault_lockout

|                                            |

|                                            | DO: rego_ok_logic(1)

|                                            |

|                                            | DO: rego_ok_logic(2)

|                                            |

|                                            | DO: closed_loop_timer

|                                            |

|                                            | DO: faosc_p_control(1)

|                                            |

|                                            | DO: faosc_p_control(2)

|                                            |

|                                            | DO: learn_timer(1)

|                                            |

|                                            | DO: learn_timer(2)

|                                            |

|                                            | DO: faosc_avg(1)

|                                            |

|                                            | DO: faosc_avg(2)

|                                            |

|                                            | DO: faosc_scp_report

|                                            |

|                                            | DO: output_routing(1)

|                                            |

|                                            | DO: output_routing(2)

|                                            |

END:  faosc2_main

8.45.5.3  PROCEDURE Vego_filter

BEGIN:  vego_filter

|     FUNCTIONAL REQUIREMENTS

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-293

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;Filter HEGO sensors for use in the fore/aft control:

|     LOGICAL REQUIREMENTS

|     Logic Diagram

|     This structure diagram shows calling relations between logic charts  in  this

|     process.

|          LEVEL     ENTITY CALLED

|            1       Chart vego_filter_0

|            2          Function rolav

|     EXTERNAL INTERFACE

|     Inputs:

|      -  BG_TMR

|      -  VEGO12

|      -  VEGO22

|     Outputs:

|     No parameters are applicable.

|     PARAMETER DECLARATIONS

|     Registers:

|      -  VEGO12 = Bank1 downstream HEGO voltage

|            Resolution: 0.001000    Units: VOLT             Init. Value: 0.000000

|            Min. Value: 0.000000    Max. Value: 5.000000    Reg. Type: RAM

|      -  VEGO12_BAR = FILTERED BANK1 DOWNSTREAM HEGO VOLTAGE.

|            Resolution: 0.001000    Units: VOLT             Init. Value: 0.000000

|            Min. Value: 0.000000    Max. Value: 5.000000    Reg. Type: RAM

|      -  VEGO22 = Bank2 downstream HEGO voltage

|            Resolution: 0.001000    Units: VOLT             Init. Value: 0.000000

|            Min. Value: 0.000000    Max. Value: 5.000000    Reg. Type: RAM

|      -  VEGO22_BAR = FILTERED BANK2 DOWNSTREAM HEGO VOLTAGE.

|            Resolution: 0.001000    Units: VOLT             Init. Value: 0.000000

|            Min. Value: 0.000000    Max. Value: 5.000000    Reg. Type: RAM

|      -  VEGO_BAR_I12 = FILTERED BANK1 DOWNSTREAM HEGO VOLTAGE USED  FOR  INTEGRAL

|         CONTROL.

|            Resolution: 0.001000    Units: VOLT             Init. Value: 0.000000

|            Min. Value: 0.000000    Max. Value: 5.000000    Reg. Type: RAM

|      -  VEGO_BAR_I22 = FILTERED BANK2 DOWNSTREAM HEGO VOLTAGE USED  FOR  INTEGRAL

|         CONTROL.

|            Resolution: 0.001000    Units: VOLT             Init. Value: 0.000000

|            Min. Value: 0.000000    Max. Value: 5.000000    Reg. Type: RAM

|     Calibration Constants:

|      -  TC_VEGO_FA = TIME CONSANT FOR REAR HEGO FILTER TO BE USED IN FORE/AF

|              T CONTROL.

|            Resolution: 0.001000    Base Value: 0.000000    Units: seconds

|            Min. Value: 0.000000    Max. Value: 63.990002   Cal. Level: RCON/VECTR

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-294

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|      -  TC_VEGO_FA_I = TIME CONSANT FOR REAR HEGO FILTER TO BE USED IN FORE/  AFT

|         INTEGRALCONTROL.

|            Resolution: 0.001000    Base Value: 0.000000    Units: seconds

|            Min. Value: 0.000000    Max. Value: 63.990002   Cal. Level: RCON/VECTR

|     Timers:

|      -  BG_TMR = BACKGROUND LOOP TIMER

|            Resolution: 0.000977    Units: seconds          Init. Value: 0.000000

|            Clip Min.: 0.000000     Clip Max.: 63.999001

|            Timer Type: FREE RUN  Direction: UP

|     Logic Chart vego_filter_0

|     Unconditionally -----------| VEGO12_BAR := rolav(VEGO12, TC_VEGO_FA)

|                                |

|                                | VEGO22_BAR := rolav(VEGO22, TC_VEGO_FA)

|                                |

|                                | VEGO_BAR_I12 := rolav(VEGO12, TC_VEGO_FA_I)

|                                |

|                                | VEGO_BAR_I22 := rolav(VEGO22, TC_VEGO_FA_I)

|                                |

END:  vego_filter

8.45.5.4  PROCEDURE Vego_ratchet

BEGIN:  vego_ratchet

|     FUNCTIONAL REQUIREMENTS

;Capture peak rich and lean values of the HEGO voltage to determine  ;if  the

sensor is warmed up and ready to use.

|     LOGICAL REQUIREMENTS

|     Logic Diagram

|     This structure diagram shows calling relations between logic charts  in  this

|     process.

|          LEVEL     ENTITY CALLED

|            1       Chart vego_ratchet_0

|            1       Chart vego_ratchet_1

|            1       Chart vego_ratchet_2

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-295

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|     EXTERNAL INTERFACE

|     Inputs:

|      -  ATMR1

|      -  VEGO12_BAR

|      -  VEGO22_BAR

|     Outputs:

|      -  VEGO_OK_L1

|      -  VEGO_OK_L2

|      -  VEGO_OK_R1

|      -  VEGO_OK_R2

|     PARAMETER DECLARATIONS

|     Registers:

|      -  VEGO12_BAR = FILTERED BANK1 DOWNSTREAM HEGO VOLTAGE.

|            Resolution: 0.001000    Units: VOLT             Init. Value: 0.000000

|            Min. Value: 0.000000    Max. Value: 5.000000    Reg. Type: RAM

|      -  VEGO22_BAR = FILTERED BANK2 DOWNSTREAM HEGO VOLTAGE.

|            Resolution: 0.001000    Units: VOLT             Init. Value: 0.000000

|            Min. Value: 0.000000    Max. Value: 5.000000    Reg. Type: RAM

|      -  VEGO_OK_R2 = Bank 2 rear EGO OK on the rich side for FAOSC use.

|            Resolution: 1.000000    Units:  FLAG            Init. Value: 0.000000

|            Min. Value: 0.000000    Max. Value: 1.000000    Reg. Type: RAM

|     Bit Flags:

|      -  VEGO_OK_L1 = Bank 1 rear EGO OK on the lean side for FAOSC use.

|                                 Initial Value: 0

|      -  VEGO_OK_L2 = Bank 2 rear EGO OK on the lean side for FAOSC use.

|                                 Initial Value: 0

|      -  VEGO_OK_R1 = Bank 1 rear EGO OK on the rich side for FAOSC use.

|                                 Initial Value: 0

|     Calibration Constants:

|      -  REGO_TM = Time delay into run mode before allowing FAOSC voltage

|              monitoring of the downstream sensors.

|            Resolution: 1.000000    Base Value: 0.000000    Units: seconds

|            Min. Value: 0.000000    Max. Value: 255.000000  Cal. Level: RCON/VECTR

|      -  VEGO_FAOS_MN = Minimum voltage required to allow FAOSC.

|            Resolution: 0.001000    Base Value: 5.000000    Units: VOLTS

|            Min. Value: 0.000000    Max. Value: 5.000000    Cal. Level: RCON/VECTR

|      -  VEGO_FAOS_MX = Maximum voltage required to allow FAOSC.

|            Resolution: 0.001000    Base Value: 0.000000    Units: VOLTS

|            Min. Value: 0.000000    Max. Value: 5.000000    Cal. Level: RCON/VECTR

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-296

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|     Timers:

|      -  ATMR1 = TIME SINCE ENGINE START

|            Resolution: 1.000000    Units: seconds          Init. Value: 0.000000

|            Clip Min.: 0.000000     Clip Max.: 255.000000

|            Timer Type: ACTUATED  Direction: UP

|     Logic Chart vego_ratchet_0

|     ATMR1 < REGO_TM -----------------| Exit this process.

|                                      |  ;do not look for peaks until the engine

|                                      |  ;has run for a little while

|                                      |

|     Logic Chart vego_ratchet_1

|     VEGO12_BAR >= VEGO_FAOS_MX ------------------| VEGO_OK_R1 := 1

|                                                  |

|                                                  |  --- ELSE ---

|                                                  |

|     VEGO12_BAR <= VEGO_FAOS_MN ------------------| VEGO_OK_L1 := 1

|                                                  |

|     Logic Chart vego_ratchet_2

|     VEGO22_BAR >= VEGO_FAOS_MX ------------------| VEGO_OK_R2 := 1

|                                                  |

|                                                  |  --- ELSE ---

|                                                  |

|     VEGO22_BAR <= VEGO_FAOS_MN ------------------| VEGO_OK_L2 := 1

|                                                  |

END:  vego_ratchet

8.45.5.5  PROCEDURE Fault_lockout

BEGIN:  fault_lockout

|     FUNCTIONAL REQUIREMENTS

;Disable FAOSC when faults on EGR, secondary air and misfire are detected.

|     LOGICAL REQUIREMENTS

|     Logic Diagram

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-297

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|     This structure diagram shows calling relations between logic charts  in  this

|     process.

|          LEVEL     ENTITY CALLED

|            1       Chart fault_lockout_0

|     EXTERNAL INTERFACE

|     Inputs:

|      -  FFG_EGR

|      -  FFG_MISFIRE

|      -  FFG_SAIR

|     Outputs:

|      -  FAOSC_FMEM

|     PARAMETER DECLARATIONS

|     Bit Flags:

|      -  FAOSC_FMEM = Flag for FAOSC fault lockout

|                                 Initial Value: 0

|      -  FFG_EGR = OBDII system FMEM flag for the EGR system - 1 ->  EGR  s  ystem

|         not currently operationg reliably.

|                                 Initial Value: 0

|      -  FFG_MISFIRE = OBDII system FMEM flag for engine misfire - 1 -> c urrently

|         misfiring.

|                                 Initial Value: 0

|      -  FFG_SAIR = OBDII system FMEM flag for the sair system - 1 -> sair s  ytem

|         is not currently under control reliably

|                                 Initial Value: 0

|     Calibration Constants:

|      -  FAOSC_ENA_SW = Calibration switch for FAOSC fault lockout logic

|            Resolution: 1.000000    Base Value: 0.000000    Units: SWITCH

|            Min. Value: 0.000000    Max. Value: 1.000000    Cal. Level: RCON/VECTR

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-298

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|     Logic Chart fault_lockout_0

|     FAOSC_ENA_SW = 1 ----------------------|     | FAOSC_FMEM := 1

|                                            |     |

|     FFG_MISFIRE = 1 -----------------|     |AND -|

|                                      |     |     |

|     FFG_SAIR = 1 --------------------|OR --|     |

|                                      |           |

|     FFG_EGR = 1 ---------------------|           |

|                                                  |

|                                                  |  --- ELSE ---

|                                                  |

|                                                  | FAOSC_FMEM := 0

|                                                  |

END:  fault_lockout

8.45.5.6  FUNCTION Rego_ok_logic

|     BEGIN:  rego_ok_logic

|     FUNCTIONAL REQUIREMENTS

|     ;Determine if rear EGO is ready to be used for trimming:

|     LOGICAL REQUIREMENTS

|     Logic Diagram

|     This structure diagram shows calling relations between logic charts  in  this

|     process.

|          LEVEL     ENTITY CALLED

|            1       Chart rego_ok_logic_0

|            2          Function rego_ok

|            2          Function VEGO_OK_L

|            2          Function VEGO_OK_R

|     EXTERNAL INTERFACE

|     Inputs:

|      -  bank - Argument

|      -  EGO_DS_MON

|      -  FAOSC_FMEM

|     Outputs:

|     There are no returns from this function.

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-299

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|     PARAMETER DECLARATIONS

|     Bit Flags:

|      -  EGO_DS_MON = Flag used to signal when the downstream EGO monitor has

|              run to completion.

|                                 Initial Value: 0

|      -  FAOSC_FMEM = Flag for FAOSC fault lockout

|                                 Initial Value: 0

|     Calibration Constants:

|      -  HEGO_CONFIG = HEGO CONFIGURATION REGISTER

|            Resolution: 1.000000    Base Value: 1.000000    Units: UNITLESS

|            Min. Value: 0.000000    Max. Value: 3.000000    Cal. Level: RCON/VECTR

|     Logic Chart rego_ok_logic_0

|     bank = 2 ------------------------|                 | rego_ok(bank) := 1

|                                      |AND -|           |

|     HEGO_CONFIG = 2 -----------------|     |           |

|                                            |OR --|     |

|     bank = 1 ------------------------|     |     |     |

|                                      |AND -|     |     |

|     HEGO_CONFIG <> 0 ----------------|           |     |

|                                                  |     |

|     FFG_EGO(bank) 2 = 0 -------------------------|     |

|                                                  |     |

|     EGO_DS_MON = 1 ------------------------|     |AND -|

|      ;downstream sensor is monitored       |     |     |

|                                            |OR --|     |

|     VEGO_OK_R(bank) = 1 -------------|     |     |     |

|                                      |AND -|     |     |

|     VEGO_OK_L(bank) = 1 -------------|           |     |

|                                                  |     |

|     FAOSC_FMEM = 0 ------------------------------|     |

|                                                        |

|                                                        |  --- ELSE ---

|                                                        |

|                                                        | rego_ok(bank) := 0

|                                                        |

|     END:  rego_ok_logic

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-300

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.45.5.7  PROCEDURE Closed_loop_timer

|     BEGIN:  closed_loop_timer

|     FUNCTIONAL REQUIREMENTS

|     ;Calculate closed loop time:

|     LOGICAL REQUIREMENTS

|     Logic Diagram

|     This structure diagram shows calling relations between logic charts  in  this

|     process.

|          LEVEL     ENTITY CALLED

|            1       Chart closed_loop_timer_0

|     EXTERNAL INTERFACE

|     Inputs:

|      -  OLFLG

|     Outputs:

|      -  RBIAS_CL_TMR

|     PARAMETER DECLARATIONS

|     Bit Flags:

|      -  OLFLG = OPEN LOOP FLAG; 1->Open Loop, 0->Closed Loop.

|                                 Initial Value: 0

|     Timers:

|      -  RBIAS_CL_TMR = TIME SINCE ENTRY INTO CLOSED LOOP.

|            Resolution: 1.000000    Units: seconds          Init. Value: 0.000000

|            Clip Min.: 0.000000     Clip Max.: 255.000000

|            Timer Type: FREE RUN  Direction: UP

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-301

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|     Logic Chart closed_loop_timer_0

|     OLFLG = 0 -----------------------| No action

|                                      |  ;allow free running upcounting

|                                      |  ;RBIAS_CL_TMR to increment

|                                      |

|                                      |  --- ELSE ---

|                                      |

|                                      | No action

|                                      |  ;reset closed loop timer

|                                      |

|                                      | RBIAS_CL_TMR := 0

|                                      |

|     END:  closed_loop_timer

8.45.5.8  FUNCTION Faosc_p_control

|     BEGIN:  faosc_p_control

|     FUNCTIONAL REQUIREMENTS

;Look up the value for the proportional R_BIAS control.

|     LOGICAL REQUIREMENTS

|     Logic Diagram

|     This structure diagram shows calling relations between logic charts  in  this

|     process.

|          LEVEL     ENTITY CALLED

|            1       Chart faosc_p_control_0

|            2          Function fox

|            2          Function rego_ok

|            2          Function R_BIAS_PROP

|            2          Function table

|     EXTERNAL INTERFACE

|     Inputs:

|      -  bank - Argument

|      -  error

|      -  EXT_REG

|      -  LOAD

|      -  N

|     Outputs:

|     There are no returns from this function.

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-302

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|     PARAMETER DECLARATIONS

|     Registers:

|      -  EXT_REG = Signed inferred exhaust wall temperature at the rear HEGO

|              location.

|            Resolution: 0.125000    Units: degrees          Init. Value: 0.000000

|            Min. Value: -4095.000000Max. Value: 4096.000000 Reg. Type: KAM

|      -  LOAD = UNIVERSAL LOAD AS RATIO OF AIR CHARGE OVER STANDARD

|            Resolution: 0.000031    Units: UNITLESS         Init. Value: 0.000000

|            Min. Value: 0.000000    Max. Value: 1.999969    Reg. Type: RAM

|      -  N = Engine speed (RPM)

|            Resolution: 0.250000    Units: REV/MIN          Init. Value: 0.000000

|            Min. Value: 0.000000    Max. Value: 16383.750000Reg. Type: RAM

|     Calibration Constants:

|      -  FAOP_EGO_TMP = MINIMUM REGO TEMPERATURE REQUIRED TO USE PROPORTIONAL BIAS

|            Resolution: 0.125000    Base Value: 0.000000    Units: degrees

|            Min. Value: -4096.000000Max. Value: 4095.875000 Cal. Level: RCON/VECTR

|      -  FNFAOSCP = Proportional feedback bias from FAOSC.

|            Resolution: 0.000122    Base Value: NIL         Units: NORM A/F

|            Min. Value: -0.015000   Max. Value: 0.015000    Cal. Level: RCON/VECTR

|            XY Pairs: 7  X Input: SIGNED ERROR X Input Units: VOLTS

|      -  FNVEGOSP = VEGO SETPOINT FOR FAOSC.

|            Resolution: 0.003900    Units: VOLTS      Table Size: 10X8

|            X Input: NIL            X Normalizing Fcn: FN021

|            Min. Value: 0.000000    Max. Value: 0.990000

|            Y Input: N              Normalizing Fcn: FN039E

|      -  PROP_LD_MAX = MAXIMUM LOAD ALLOWED TO USE PROPORTIONAL BIAS

|            Resolution: 0.000031    Base Value: 0.000000    Units: UNITLESS

|            Min. Value: 0.000000    Max. Value: 1.990000    Cal. Level: RCON/VECTR

|      -  PROP_LD_MIN = MINIMUM LOAD REQUIRED TO USE PROPORTIONAL BIAS

|            Resolution: 0.000031    Base Value: 1.990000    Units: UNITLESS

|            Min. Value: 0.000000    Max. Value: 1.990000    Cal. Level: RCON/VECTR

|      -  PROP_N_MAX = MAXIMUM RPM ALLOWED TO USE PROPORTIONAL BIAS

|            Resolution: 0.250000    Base Value: 0.000000    Units: RPM

|            Min. Value: 0.000000    Max. Value: 16383.000000Cal. Level: RCON/VECTR

|      -  PROP_N_MIN = MINIMUM RPM REQUIRED TO USE PROPORTIONAL BIAS

|            Resolution: 0.250000    Base Value: 16383.000000Units: RPM

|            Min. Value: 0.000000    Max. Value: 16383.000000Cal. Level: RCON/VECTR

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-303

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|     Logic Chart faosc_p_control_0

|     N > PROP_N_MIN ------------------|     | setpoint := FNVEGOSP(N, LOAD)

|                                      |     |  ;the table lookup above should

|     N < PROP_N_MAX ------------------|     |  ;return a word interpolated value.

|                                      |     |

|     LOAD > PROP_LD_MIN --------------|     | error := VEGO(bank) 2 _BAR -

|                                      |AND -|  setpoint

|     LOAD < PROP_LD_MAX --------------|     |

|                                      |     | R_BIAS_PROP(bank) := FNFAOSCP(error)

|     EXT_REG > FAOP_EGO_TMP ----------|     |

|                                      |     |

|     rego_ok(bank) = 1 ---------------|     |

|                                            |

|                                            |  --- ELSE ---

|                                            |

|                                            | R_BIAS_PROP(bank) := 0

|                                            |

|     END:  faosc_p_control

8.45.5.9  FUNCTION Learn_timer

|     BEGIN:  learn_timer

|     FUNCTIONAL REQUIREMENTS

;Determine if drive conditions are acceptable for learning:

|     LOGICAL REQUIREMENTS

|     Logic Diagram

|     This structure diagram shows calling relations between logic charts  in  this

|     process.

|          LEVEL     ENTITY CALLED

|            1       Chart learn_timer_0

|            2          Function abs

|            2          Function FAO_MON_FLG

|            2          Function LAMAVE

|            2          Function LAMBSE

|            2          Function learn_timer_running

|            2          Function RBIAS_LNTMR

|     EXTERNAL INTERFACE

|     Inputs:

|      -  bank - Argument

|      -  ECT

|      -  LAM_MOD_FLG

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-304

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|      -  LOAD

|      -  MFMFLG

|      -  N

|      -  VSBAR

|     Outputs:

|     There are no returns from this function.

|     PARAMETER DECLARATIONS

|     Registers:

|      -  ECT = COOLANT TEMP

|            Resolution: 2.000000    Units: degrees          Init. Value: 60.000000

|            Min. Value: -256.000000 Max. Value: 254.000000  Reg. Type: RAM

|      -  LOAD = UNIVERSAL LOAD AS RATIO OF AIR CHARGE OVER STANDARD

|            Resolution: 0.000031    Units: UNITLESS         Init. Value: 0.000000

|            Min. Value: 0.000000    Max. Value: 1.999969    Reg. Type: RAM

|      -  N = Engine speed (RPM)

|            Resolution: 0.250000    Units: REV/MIN          Init. Value: 0.000000

|            Min. Value: 0.000000    Max. Value: 16383.750000Reg. Type: RAM

|      -  VSBAR = FILTERED VEHICLE SPEED

|            Resolution: 0.500000    Units: RPM              Init. Value: 0.000000

|            Min. Value: 0.000000    Max. Value: 127.500000  Reg. Type: RAM

|     Bit Flags:

|      -  LAM_MOD_FLG = Flag used to initiate high frequency fuel modulation

|              for the upstream EGO monitor.

|                                 Initial Value: 0

|      -  MFMFLG = MAP/MAF FMEM FLAG

|                                 Initial Value: 0

|     Calibration Constants:

|      -  LAM_DIFF_FAO = MAXIMUM ALLOWABLE DIFFERENCE IN LAMBSE FROM LAMAVE

|            Resolution: 0.000031    Base Value: 0.000000    Units: UNITLESS

|            Min. Value: 0.000000    Max. Value: 1.999969    Cal. Level: RCON/VECTR

|      -  RBIAS_ECT_MN = MINIMUM ECT  REQUIRED  TO  USE  REAR  EGO  FOR  FRONT  EGO

|         TRIMMING.

|            Resolution: 2.000000    Base Value: -20.000000  Units: degrees

|            Min. Value: -40.000000  Max. Value: 254.000000  Cal. Level: RCON/VECTR

|      -  RBIAS_ECT_MX = MAXIMUM  ECT  ALLOWED  TO  USE  REAR  EGO  FOR  FRONT  EGO

|         TRIMMING.

|            Resolution: 2.000000    Base Value: 254.000000  Units: degrees

|            Min. Value: -40.000000  Max. Value: 254.000000  Cal. Level: RCON/VECTR

|      -  RBIAS_LAM_MN = Minimum allowable average A/F ratio for  FAOSC  in  tegral

|         learning.

|            Resolution: 0.000030    Base Value: 1.000000    Units: --

|            Min. Value: 0.000000    Max. Value: 1.990000    Cal. Level: RCON/VECTR

|      -  RBIAS_LAM_MX = Maximum allowable average A/F ratio for  FAOSC  in  tegral

|         learning.

|            Resolution: 0.000030    Base Value: 1.000000    Units: --

|            Min. Value: 0.000000    Max. Value: 1.990000    Cal. Level: RCON/VECTR

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-305

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|      -  RBIAS_LD_MAX = MAXIMUM LOAD ALLOWED TO USE REAR EGO TO LEARN BIAS.

|            Resolution: 0.000031    Base Value: 0.900000    Units: UNITLESS

|            Min. Value: 0.000000    Max. Value: 1.990000    Cal. Level: RCON/VECTR

|      -  RBIAS_LD_MIN = MINIMUM LOAD REQUIRED TO USE REAR EGO TO LEARN BIAS.

|            Resolution: 0.000031    Base Value: 0.100000    Units: UNITLESS

|            Min. Value: 0.000000    Max. Value: 1.990000    Cal. Level: RCON/VECTR

|      -  RBIAS_N_MAX = MAXIMUM RPM ALLOWED TO USE REAR EGO TO LEARN BIAS.

|            Resolution: 0.250000    Base Value: 5000.000000 Units: RPM

|            Min. Value: 0.000000    Max. Value: 16383.000000Cal. Level: RCON/VECTR

|      -  RBIAS_N_MIN = MINIMUM RPM REQUIRED TO USE REAR EGO TO LEARN BIAS.

|            Resolution: 0.250000    Base Value: 1000.000000 Units: RPM

|            Min. Value: 0.000000    Max. Value: 16383.000000Cal. Level: RCON/VECTR

|      -  RBIAS_VS_MIN = MINIMUM VEHICLE SPEED REQUIRED  BEFORE  BIAS  TRIMMING  IS

|         ALLOWED.

|            Resolution: 0.500000    Base Value: 35.000000   Units: MPH

|            Min. Value: 0.000000    Max. Value: 127.500000  Cal. Level: RCON/VECTR

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-306

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|     Logic Chart learn_timer_0

|     VSBAR >= RBIAS_VS_MIN -----------------|     | No action

|      ;cruise condition.                    |     |  ;allow free running

|                                            |     |  ;upcounting RBIAS_LNTMR(bank

|     N > RBIAS_N_MIN -----------------------|     |  ;) to increment

|                                            |     |

|     N < RBIAS_N_MAX -----------------------|     | learn_timer_running(bank) := 1

|      ;rpm is within learning range.        |     |

|                                            |     |

|     LOAD > RBIAS_LD_MIN -------------------|     |

|                                            |     |

|     LOAD < RBIAS_LD_MAX -------------------|     |

|      ;LOAD is within learning range.       |     |

|                                            |     |

|     ECT > RBIAS_ECT_MN --------------------|     |

|                                            |AND -|

ECT < RBIAS_ECT_MX --------------------|     |

;ECT is within learning range.        |     |

|     |

LAM_MOD_FLG = 0 -----------------------|     |

;not doing EGO response rate test.    |     |

|     |

MFMFLG = 0 ----------------------------|     |

;air meter functional.                |     |

|     |

abs(LAMBSE(bank) - LAMAVE(bank)) <     |     |

|      LAM_DIFF_FAO -------------------------|     |

|     |

LAMAVE(bank) <= RBIAS_LAM_MX ----------|     |

|     |

LAMAVE(bank) >= RBIAS_LAM_MN ----------|     |

|                                                  |

|                                                  |  --- ELSE ---

|                                                  |

|                                                  | No action

|                                                  |  ;reset learn timer

|                                                  |

| RBIAS_LNTMR(bank) := 0

|

|                                                  | learn_timer_running(bank) := 0

|                                                  |

|                                                  |

| FAO_MON_FLG(bank) := 0

|                                                  |

END:  learn_timer

8.45.5.10  FUNCTION Faosc_avg

|     BEGIN:  faosc_avg

|     FUNCTIONAL REQUIREMENTS

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-307

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;Calculate  the  average  of  the  integral  bias  trim.    ;This   algorithm

essentially  implements  a  rolav() function, but will ;allow very large time

constants.  The rolav() function will not perform ;adequately for  very  long

time  constants  because  when  the  time  constant  ;is  big, the amount the

filtered value must change in one background pass ;is  very  small.   If  the

change  is  less than the resoulution of the filtered ;parameter, the rolav()

function will add (or subtract) one  bit  from  the  ;filtered  value.   This

results  in  a ramp, not an exponential response.  ;This routine will not add

|     (or subtract) one bit if the filtered value should ;change by less  than  the

|     resolution  of the filtered value.  Instead the ;delta time between execution

of the filter equation will be accumulated.  ;The accumulated delta time will

then  be  used  next execution cycle to compute ;a new filter constant.  This

accumulation of delta times will continue until ;at least on full  resolution

of  the filtered value should be added (or ;subtracted) during this execution

cycle.  In this way the exponential ;behaviour can be obtained for very  long

|     time   constants.    ;The  software  must  carry  enough  resolution  in  the

|     calculation of  the  ;'new_average'  intermediate  value  to  decide  if  the

difference  between  it  ;and the current filtered value is less than one bit

resolution.

|     LOGICAL REQUIREMENTS

|     Logic Diagram

|     This structure diagram shows calling relations between logic charts  in  this

|     process.

|          LEVEL     ENTITY CALLED

|            1       Chart faosc_avg_0

|            2          Function FAOS_DT

|            2          Function R_BIAS_AVG

|            2          Function R_BIAS_INT

|            1       Chart faosc_avg_1

|            2          Function FAOS_DT

|            2          Function new_average

|            2          Function R_BIAS_AVG

|            2          Function R_BIAS_INT

|     EXTERNAL INTERFACE

|     Inputs:

|      -  bank - Argument

|      -  BG_TMR

|     Outputs:

|     There are no returns from this function.

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-308

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|     PARAMETER DECLARATIONS

|     Calibration Constants:

|      -  TCRBIAS = TIME CONSTANT FOR THE AVERAGE RBIAS

|            Resolution: 0.007812    Base Value: 150.000000  Units: seconds

|            Min. Value: 0.000000    Max. Value: 511.992187  Cal. Level: RCON/VECTR

|     Timers:

|      -  BG_TMR = BACKGROUND LOOP TIMER

|            Resolution: 0.000977    Units: seconds          Init. Value: 0.000000

|            Clip Min.: 0.000000     Clip Max.: 63.999001

|            Timer Type: FREE RUN  Direction: UP

|     Logic Chart faosc_avg_0

|     R_BIAS_AVG(bank) = R_BIAS_INT(bank) ---| FAOS_DT(bank) := BG_TMR

|                                            |  ;stop averaging

|                                            |

|                                            | EXIT this process.

|                                            |

|     Logic Chart faosc_avg_1

|     Unconditionally -----------| f := 1 / (1 + (TCRBIAS / FAOS_DT(bank)))

|                                |  ;calculate fiter constant for long term first

|                                |  ;order filter.

|                                |

|                                | new_average(bank) := f * R_BIAS_INT(bank) + (1 -

|                                |  f) * R_BIAS_AVG(bank)

|                                |

|     abs(new_average(bank)   -    R_BIAS_AVG(bank))    <    (BIAS_G_RES    /    2)

-----------------|  FAOS_DT(bank)  := BG_TMR | + FAOS_DT(bank) | ;the step to

|     be taken cannot | ;be contained in the | ;resolution  of  the  |  ;R_BIAS_AVG

|     term.   | ;Instead of taking the step, | ;increase the delta time input | ;to

the filter and recompute | ;next background pass.  |  |  ---  ELSE  ---  |  |

R_BIAS_AVG(bank) := | new_average(bank) | FAOS_DT(bank) := BG_TMR

END:  faosc_avg

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-309

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.45.5.11  PROCEDURE Faosc_scp_report

BEGIN:  faosc_scp_report

|     FUNCTIONAL REQUIREMENTS

;Store the average values  into  separate  KAM  parameters  so  they  can  be

;assigned  to  FFFFh when hardware is not present.  They are required by ;SCP

Mode06 reporting.

|     LOGICAL REQUIREMENTS

|     Logic Diagram

|     This structure diagram shows calling relations between logic charts  in  this

|     process.

|          LEVEL     ENTITY CALLED

|            1       Chart faosc_scp_report_0

|     EXTERNAL INTERFACE

|     Inputs:

|      -  FFFFh

|      -  R_BIAS_AVG1

|      -  R_BIAS_AVG2

|     Outputs:

|      -  FAO_SCP_AVG1

|      -  FAO_SCP_AVG2

|     PARAMETER DECLARATIONS

|     Registers:

|      -  FAO_SCP_AVG1 = Copy of R_BIAS_AVG1, assigned to FFFFh when HEGO is n

|              ot present.

|            Resolution: 0.000122    Units: UNITLESS         Init. Value: 0.000000

|            Min. Value: -3.990000   Max. Value: 3.990000    Reg. Type: KAM

|      -  FAO_SCP_AVG2 = Copy of R_BIAS_AVG2, assigned to FFFFh when HEGO is  n  ot

|         present.

|            Resolution: 0.000122    Units: UNITLESS         Init. Value: 0.000000

|            Min. Value: -3.990000   Max. Value: 3.990000    Reg. Type: KAM

|      -  R_BIAS_AVG1 = THE AVERAGE VALUE OF THE INTEGRAL VALUE OF R_BIAS1

|            Resolution: 0.000122    Units: UNITLESS         Init. Value: 0.000000

|            Min. Value: -3.990000   Max. Value: 3.990000    Reg. Type: KAM

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-310

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|      -  R_BIAS_AVG2 = THE AVERAGE VALUE OF THE INTEGRAL VALUE OF R_BIAS2

|            Resolution: 0.000122    Units: UNITLESS         Init. Value: 0.000000

|            Min. Value: -3.990000   Max. Value: 3.990000    Reg. Type: KAM

|     Calibration Constants:

|      -  HEGO_CONFIG = HEGO CONFIGURATION REGISTER

|            Resolution: 1.000000    Base Value: 1.000000    Units: UNITLESS

|            Min. Value: 0.000000    Max. Value: 3.000000    Cal. Level: RCON/VECTR

|      -  NUMEGO = Number of HEGO sensors used.

|            Resolution: 1.000000    Base Value: 2.000000    Units: ENUMERATED

|            Min. Value: 1.000000    Max. Value: 2.000000    Cal. Level: RCON/VECTR

|     Logic Chart faosc_scp_report_0

|     HEGO_CONFIG = 0 -----------------------| FAO_SCP_AVG1 := FFFFh

|      ;no rear EGO exists.                  |

|                                            | FAO_SCP_AVG2 := FFFFh

|                                            |

|                                            |  --- ELSE ---

|                                            |

|     NUMEGO = 1 ----------------------|     | FAO_SCP_AVG1 := R_BIAS_AVG1

|      ;single bank                    |OR --|

|                                      |     | FAO_SCP_AVG2 := FFFFh

|     HEGO_CONFIG = 3 -----------------|     |

|      ;y - pipe configuration               |

|                                            |

|                                            |  --- ELSE ---

|                                            |

|                                            | FAO_SCP_AVG1 := R_BIAS_AVG1

|                                            |

|                                            | FAO_SCP_AVG2 := R_BIAS_AVG2

|                                            |

END:  faosc_scp_report

8.45.5.12  FUNCTION Output_routing

|     BEGIN:  output_routing

|     FUNCTIONAL REQUIREMENTS

;Determine if Fore-Aft Oxygen Sensor Control has been requested:

|     LOGICAL REQUIREMENTS

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-311

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|     Logic Diagram

|     This structure diagram shows calling relations between logic charts  in  this

|     process.

|          LEVEL     ENTITY CALLED

|            1       Chart output_routing_0

|            2          Function faosc_i_control

|            2          Function final_calc

|            2          Function learn_timer_running

|            2          Function max

|            2          Function min

|            2          Function R_BIAS

|            2          Function R_BIAS_AVG

|            2          Function R_BIAS_INT

|            2          Function R_BIAS_PROP

|            1       Chart output_routing_1

|     EXTERNAL INTERFACE

|     Inputs:

|      -  bank - Argument

|      -  FAILED_BANK

|      -  KAM_ERROR

|     Outputs:

|     There are no returns from this function.

|     PARAMETER DECLARATIONS

|     Registers:

|      -  FAILED_BANK = Register indicating which bank is in lack of EGO switching.

|            Resolution: 1.000000    Units: UNITLESS         Init. Value: 0.000000

|            Min. Value: 0.000000    Max. Value: 10.000000   Reg. Type: KAM

|     Bit Flags:

|      -  FALD_BNK_OLD = Last pass value of FAILED_BANK

|                                 Initial Value: 0.000000

|      -  KAM_ERROR = INDICATES KEEP ALIVE RAM INVALID

|                                 Initial Value: 0

|     Calibration Constants:

|      -  HEGO_CONFIG = HEGO CONFIGURATION REGISTER

|            Resolution: 1.000000    Base Value: 1.000000    Units: UNITLESS

|            Min. Value: 0.000000    Max. Value: 3.000000    Cal. Level: RCON/VECTR

|      -  RBIAS_AVG_MN = Minimum allowable value of R_BIAS_AVG

|            Resolution: 0.000122    Base Value: -0.015000   Units: UNITLESS

|            Min. Value: -3.990000   Max. Value: 3.990000    Cal. Level: RCON/VECTR

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-312

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|      -  RBIAS_AVG_MX = Maximum allowable value of R_BIAS_AVG

|            Resolution: 0.000122    Base Value: 0.015000    Units: UNITLESS

|            Min. Value: -3.990000   Max. Value: 3.990000    Cal. Level: RCON/VECTR

|      -  RBIAS_MAX = MAXIMUM ALLOWABLE VALUE OF R_BIAS

|            Resolution: 0.000122    Base Value: 0.015000    Units: UNITLESS

|            Min. Value: -3.990000   Max. Value: 3.990000    Cal. Level: RCON/VECTR

|      -  RBIAS_MIN = MINIMUM ALLOWABLE VALUE OF R_BIAS

|            Resolution: 0.000122    Base Value: -0.015000   Units: UNITLESS

|            Min. Value: -3.990000   Max. Value: 3.990000    Cal. Level: RCON/VECTR

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-313

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|     Logic Chart output_routing_0

|     FAILED_BANK <> bank -|           | R_BIAS(bank) := 0

|                          |AND -|     |

|     FALD_BNK_OLD = bank -|     |     | R_BIAS_AVG(bank) := 0

|                                |     |

|     HEGO_CONFIG = 0 -----------|     | R_BIAS_INT(bank) := 0

|                                |     |

|     KAM_ERROR = 1 -------------|OR --| R_BIAS_PROP(bank) := 0

|                                |     |

|     R_BIAS_AVG(bank) <         |     |

|      RBIAS_AVG_MN -------------|     |

|                                |     |

|     R_BIAS_AVG(bank) >         |     |

|      RBIAS_AVG_MX -------------|     |

|                                      |

|                                      |  --- ELSE ---

|                                      |

|     learn_timer_running(bank) = 1 ---| DO: faosc_i_control(bank)

|                                      |

|                                      | DO: final_calc(bank)

|                                      |

|                                      | rbias := R_BIAS_INT(bank) + R_BIAS_PROP(

|                                      |  bank)

|                                      |

|                                      | rbias := min(rbias, RBIAS_MAX)

|                                      |

|                                      | R_BIAS(bank) := max(rbias, RBIAS_MIN)

|                                      |

|                                      |  --- ELSE ---

|                                      |

|                                      | rbias := R_BIAS_AVG(bank) + R_BIAS_PROP(

|                                      |  bank)

|                                      |

|                                      | rbias := min(rbias, RBIAS_MAX)

|                                      |

|                                      | R_BIAS(bank) := max(rbias, RBIAS_MIN)

|                                      |

|                                      | R_BIAS_INT(bank) := R_BIAS_AVG(bank)

|                                      |

|     Logic Chart output_routing_1

|     Unconditionally -----------------| FALD_BNK_OLD := FAILED_BANK

|                                      |

END:  output_routing

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-314

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.45.5.13  FUNCTION Faosc_i_control

|     BEGIN:  faosc_i_control

|     FUNCTIONAL REQUIREMENTS

;Calculate BIAS_SUM(bank):

|     LOGICAL REQUIREMENTS

|     Logic Diagram

|     This structure diagram shows calling relations between logic charts  in  this

|     process.

|          LEVEL     ENTITY CALLED

|            1       Chart faosc_i_control_0

|            2          Function BIAS_SUM

|            2          Function FAO_MON_FLG

|            2          Function fox

|            2          Function RBIAS_LNTMR

|            2          Function rego_ok

|            2          Function table

|     EXTERNAL INTERFACE

|     Inputs:

|      -  bank - Argument

|      -  BG_TMR

|      -  error

|      -  LOAD

|      -  N

|      -  RBIAS_CL_TMR

|     Outputs:

|     There are no returns from this function.

|     PARAMETER DECLARATIONS

|     Registers:

|      -  LOAD = UNIVERSAL LOAD AS RATIO OF AIR CHARGE OVER STANDARD

|            Resolution: 0.000031    Units: UNITLESS         Init. Value: 0.000000

|            Min. Value: 0.000000    Max. Value: 1.999969    Reg. Type: RAM

|      -  N = Engine speed (RPM)

|            Resolution: 0.250000    Units: REV/MIN          Init. Value: 0.000000

|            Min. Value: 0.000000    Max. Value: 16383.750000Reg. Type: RAM

|     Calibration Constants:

|      -  FNFAOSCI = Integral feedback

|            Resolution: 0.000008    Base Value: NIL         Units: LAMBDA/MIN

|            Min. Value: -0.025000   Max. Value: 0.249000    Cal. Level: RCON/VECTR

|            XY Pairs: 7  X Input: SIGNED ERROR X Input Units: VOLTS

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-315

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|      -  FNVEGOSP = VEGO SETPOINT FOR FAOSC.

|            Resolution: 0.003900    Units: VOLTS      Table Size: 10X8

|            X Input: NIL            X Normalizing Fcn: FN021

|            Min. Value: 0.000000    Max. Value: 0.990000

|            Y Input: N              Normalizing Fcn: FN039E

|      -  RBIAS_CL_TM = TIME REQUIRED  IN  CLOSED  LOOP  BEFORE  BIAS  TRIMMING  IS

|         ALLOWED.

|            Resolution: 1.000000    Base Value: 10.000000   Units: seconds

|            Min. Value: 0.000000    Max. Value: 255.000000  Cal. Level: RCON/VECTR

|      -  RBIAS_LN_TM = TIME REQUIRED AT SPEED/LOAD CONDITION TO START LEARNING.

|            Resolution: 1.000000    Base Value: 10.000000   Units: seconds

|            Min. Value: 0.000000    Max. Value: 255.000000  Cal. Level: RCON/VECTR

|     Timers:

|      -  BG_TMR = BACKGROUND LOOP TIMER

|            Resolution: 0.000977    Units: seconds          Init. Value: 0.000000

|            Clip Min.: 0.000000     Clip Max.: 63.999001

|            Timer Type: FREE RUN  Direction: UP

|      -  RBIAS_CL_TMR = TIME SINCE ENTRY INTO CLOSED LOOP.

|            Resolution: 1.000000    Units: seconds          Init. Value: 0.000000

|            Clip Min.: 0.000000     Clip Max.: 255.000000

|            Timer Type: FREE RUN  Direction: UP

|     Logic Chart faosc_i_control_0

|     rego_ok(bank) = 1 ---------|     | setpoint := FNVEGOSP(N, LOAD)

|      ;rear EGO bank O. K.      |     |  ;the table lookup above should return a

|                                |     |  ;word interpolated value.

|     RBIAS_LNTMR(bank) >        |     |

|      RBIAS_LN_TM --------------|AND -| error := VEGO_BAR_I(bank) 2 - setpoint

|      ;learning conditions      |     |

|      ;stabilized.              |     | BIAS_SUM(bank) := ((FNFAOSCI(error) / 60)

|                                |     |  * BG_TMR) + BIAS_SUM(bank)

|     RBIAS_CL_TMR >             |     |

|      RBIAS_CL_TM --------------|     | FAO_MON_FLG(bank) := 1

|      ;have been closed loop          |  ;This flag actuates the timer

|      ;long enough for catalyst       |

|      ;to stabilize.                  | : FAO_MON_TMR(bank) to increment.

|                                      |

|                                      |  --- ELSE ---

|                                      |

|                                      | BIAS_SUM(bank) := 0

|                                      |

|                                      | FAO_MON_FLG(bank) := 0

|                                      |  ;The timer FAO_MON_TMR(bank) holds its

|                                      |  ;current value.

|                                      |

|     END:  faosc_i_control

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-316

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.45.5.14  FUNCTION Final_calc

|     BEGIN:  final_calc

|     FUNCTIONAL REQUIREMENTS

|     LOGICAL REQUIREMENTS

|     Logic Diagram

|     This structure diagram shows calling relations between logic charts  in  this

|     process.

|          LEVEL     ENTITY CALLED

|            1       Chart final_calc_0

|            1       Chart final_calc_1

|            2          Function RBIAS_EGOSW

|            1       Chart final_calc_2

|            2          Function abs

|            2          Function BIAS_SUM

|            2          Function max

|            2          Function min

|            2          Function RBIAS_EGOSW

|            2          Function R_BIAS_INT

|            1       Chart final_calc_3

|     EXTERNAL INTERFACE

|     Inputs:

|      -  bank - Argument

|      -  FAILED_BANK

|      -  R_BIAS_INT1

|      -  swtfl

|     Outputs:

|     There are no returns from this function.

|     PARAMETER DECLARATIONS

|     Registers:

|      -  FAILED_BANK = Register indicating which bank is in lack of EGO switching.

|            Resolution: 1.000000    Units: UNITLESS         Init. Value: 0.000000

|            Min. Value: 0.000000    Max. Value: 10.000000   Reg. Type: KAM

|      -  R_BIAS_INT1 = THE INTEGRAL VALUE OF R_BIAS1

|            Resolution: 0.000122    Units: UNITLESS         Init. Value: 0.000000

|            Min. Value: -3.990000   Max. Value: 3.990000    Reg. Type: RAM

|      -  R_BIAS_INT2 = THE INTEGRAL VALUE OF R_BIAS2

|            Resolution: 0.000122    Units: UNITLESS         Init. Value: 0.000000

|            Min. Value: -3.990000   Max. Value: 3.990000    Reg. Type: RAM

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-317

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|     Calibration Constants:

|      -  BIAS_G_RES = RESOLUTION OF BIAS_G

|            Resolution: 0.000000    Base Value: 0.000244    Units: UNITLESS

|            Min. Value: 0.000000    Max. Value: 0.000488    Cal. Level:

|         Not_Modifiable

|      -  HEGO_CONFIG = HEGO CONFIGURATION REGISTER

|            Resolution: 1.000000    Base Value: 1.000000    Units: UNITLESS

|            Min. Value: 0.000000    Max. Value: 3.000000    Cal. Level: RCON/VECTR

|      -  RBIAS_INT_MN = Minimum allowable FAOSC integral term.

|            Resolution: 0.000122    Base Value: -0.015000   Units: --

|            Min. Value: -3.990000   Max. Value: 3.990000    Cal. Level: RCON/VECTR

|      -  RBIAS_INT_MX = Maximum allowable FAOSC integral term.

|            Resolution: 0.000122    Base Value: 0.015000    Units: --

|            Min. Value: -3.990000   Max. Value: 3.990000    Cal. Level: RCON/VECTR

|     Logic Chart final_calc_0

|     FAILED_BANK = bank --------| other_bank := 3 - bank

|                                |

|                                | swtfl := SWTFL(other_bank) 1

|                                |  ;if the current bank has a failed upstream EGO

|                                |  ;, the switch flag from the other bank should

|                                |  ;be used to pace the learning.

|                                |

|                                |  --- ELSE ---

|                                |

|                                | swtfl := SWTFL(bank) 1

|                                |

|     Logic Chart final_calc_1

|     swtfl = 1 -----------------| RBIAS_EGOSW(bank) := RBIAS_EGOSW(bank) + 1

|                                |

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-318

FUEL STRATEGY, REAR EGO TRIM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|     Logic Chart final_calc_2

|     abs(BIAS_SUM(bank)) >=           | rbias_int := R_BIAS_INT(bank) + BIAS_SUM(

|      BIAS_G_RES ---------------|     |  bank)

|      ;sum is greater that      |     |

|      ;resolution of BIAS_G.    |AND -| rbias_int := min(rbias_int, RBIAS_INT_MX)

|                                |     |

|     RBIAS_EGOSW(bank) >= 1 ----|     | R_BIAS_INT(bank) := max(rbias_int,

|      ;at least one EGO switch        |  RBIAS_INT_MN)

|      ;has occurred since last        |

|      ;RBIAS update.                  | BIAS_SUM(bank) := 0

|                                      |

|                                      | RBIAS_EGOSW(bank) := 0

|                                      |

|     Logic Chart final_calc_3

|     HEGO_CONFIG = 3 -----------------| R_BIAS_INT2 := R_BIAS_INT1

|                                      |

END:  final_calc

|                      STRATEGY FILE: CLFUL_FAOSC_COM1/GEN=25A2

8-319

FUEL STRATEGY, COOLING FAIL-SAFE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.46  COOLING FAIL-SAFE LOGIC (CDAN0)

OVERVIEW

The module provides the null assignment to COOL_FLG for all those  strategies

that  do  not  have the cylinder head temperature protection logic (fail safe

cooling).

DEFINITIONS

Bit Flags:

-  COOL_FLG   =   Flag   indicating   overtemperature   conditions;   1   ->

overtemperature occured.

PROCESS

STRATEGY MODULE: FUEL_COOL_FAIL_COM2

Execute the following process once per background pass.

unconditionally ------------------------| COOL_FLG = 0

| ; Ensure flag remains clear

8-320

FUEL STRATEGY, BACKGROUND F/A RATIO MULTIPLIER LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.47  FAR_MUL BACKGROUND LOGIC (CDAK0)

OVERVIEW

This logic provides an Offboard  Diagnostic  Unit  (ODU)  the  capability  to

control  the delivered LBMF_INJn independently from the main strategy control

in order to allow CARB verification of the OBDII Adaptive Fuel Monitor.

The process by which an ODU manipulates the EEC-V delivered fuel mass  is  as

follows:

1) The ODU queries the EEC-V for two  two-byte  Parameter  IDs  (PIDs)  which

provide  the  recommended  rich  and lean fuel mass multiplier limits for use

during the OBDII Adaptive Fuel Monitor demonstration, the EEC's copy  of  the

desired  multiplier,  and  a response code indicating the status of the test.

The data will be reported over the SCP link.  The first message contains  the

recommended  rich  FAR_MUL  limit  (FAR_RICH_LIM)  and  the  recommended lean

FAR_MUL limit (FAR_LEAN_LIM).  The second message contains the  EEC  copy  of

the multiplier (FAR_MUL_SCP) and the response code (FAR_MUL_RSP).

2) The ODU will next download two bytes of contiguous data, starting  at  the

199th  byte  of  the  200  byte  ODU  user  RAM array as described in the SCP

chapter.  The starting address of this 200 byte array can be obtained via the

scp_odu_ram  PID.   The  first  data byte will represent the appropriate Fuel

Mass  Multiplier  data  value  (FAR_MUL_SCP).   The  second  data  byte  will

represent  the  one's  compliment  of  FAR_MUL_SCP  (FAR_MUL_CHK).  The one's

complement  (Exclusive  OR)  is  used  here  in  order  to  provide  strategy

robustness.

3) The two RAM bytes will be used within the EEC-V, per the following  logic,

to  either deliver a fuel mass multiplier to the foreground fuel logic, or to

disable the use of the multiplier in the final LBMF_INJn calculation.   If  a

multiplier  is  used,  it  will  be ramped from a value of 1.0 to the desired

magnitude by the EEC to produce  a  smooth  transition  and  avoid  potential

engine  start-up  problems.   The time period over which the ramp is executed

(FAR_MUL_RAMP) is calibrateable.

If  the  ODU  should  go  offline  or   otherwise   stop   transmitting   the

TESTER_PRESENT  message,  then  the EEC-V will revert back to its normal fuel

scheduling logic and  the  FAR_MUL  multiplier  will  be  set  to  1.0  after

ODU_TIMEOUT seconds have passed.

NOTE:  The PID described above (and the substitution process it allows)  will

not  be  available  to  any  offboard  SCP tool other than those specifically

designated  by  the  Powertrain  Control  Systems  and   Diagnostic   Systems

Departments.   This  process  is  NOT  intended  for  any  other  use besides

demonstrating the ability of the EEC-V system to detect and respond to  large

errors  in  the  engine fuel and/or air mass calculations, per the applicable

OBDII specifications.

8-321

FUEL STRATEGY, BACKGROUND F/A RATIO MULTIPLIER LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  DLOAD_EXEC_KEY = Command KEY byte to enable  execution  of  a  downloaded

program   (via  SCP)  -  S/W  parm  is:   DLOAD_EXEC_KEY,  EDTS  parm  is

DLOAD_EXEC_K.

-  FAR_MUL_CHK = Check byte of FAR_MUL_SCP.  Must be  One's  compliment  for

demonstration of OBDII Adaptive Fuel Monitor for CARB.

-  FAR_MUL_SCP =  Byte  downloaded  (via  SCP)  to  load  into  FAR_MUL  for

demonstration  of  OBDII  Adaptive  Fuel  Monitor.  Used to impose a fuel

error in strategy.

-  N = Engine speed, rpm.

-  SCP_DWNLD_ST = Current level of SCP Download & Execute State Machine.

Bit Flags:

-  DEMAND_MODE = Flag set when and On-Demand self test mode is active.

-  FAR_MUL_RDY  =  Flag  indicating  readiness  of  Adaptive  Fuel   Monitor

demonstration test; 0 -> test not ready, 1 -> test ready to run.

-  FAR_MUL_RUN  =  Flag  indicating   state   of   Adaptive   Fuel   Monitor

demonstration test; 0 -> test not running, 1 -> test running.

-  UNDSP = Engine underspeed/run mode flag; 1  ->  in  crank  or  underspeed

mode, 0 -> in run mode.

Calibration Constants:

-  FAR_LEAN_LIM = Lean limit of fuel mass (or  fuel-air  ratio)  multiplier,

unitless.

-  FAR_MUL_RAMP = Desired duration of fuel multiplier ramp, sec.

-  FAR_MUL_SW = Switch to enable use of fuel mass multiplier for development

purposes  ONLY,  unitless.   (Value  must be set to 1 with RCON to enable

use.  This parameter will not be calibratable at VECTOR level.)

-  FAR_RICH_LIM = Rich limit of fuel mass (or  fuel-air  ratio)  multiplier,

unitless.

8-322

FUEL STRATEGY, BACKGROUND F/A RATIO MULTIPLIER LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OUTPUTS

Registers:

-  FAR_MUL = Percentage of fueling error to  impose  into  EEC-V  system  in

order  to demonstrate the OBDII Adaptive Fuel Monitor for CARB.  Assigned

via SCP.

-  FAR_MUL_CHK = See above.

-  FAR_MUL_SCP = See above.

-  FAR_MUL_RSP = Response code indicating the status of  the  test  for  SCP

communications;

00h -> affirmative response

22h -> conditions not correct

44h -> ready for download

-  FAR_MUL_SLP = Desired change in multiplier per unit time, 1/sec.

-  FAR_MUL_TMR = Free-running timer used to ramp fuel multiplier to  desired

value, sec.

Bit Flags:

-  FAR_MUL_FLG = Foreground  flag  which  enables  FAR_MUL  to  be  used  in

injector  fuel  pulsewidth calculations in order to demonstrate the OBDII

Adaptive Fuel Monitor for CARB.

-  FAR_MUL_RDY = See above.

-  FAR_MUL_RUN = See above.

OTHER

-  far_mul_1 = A two byte parameter used for SCP control of  the  Fuel  Mass

Multiplier.   The  first  byte  contains the value for FAR_RICH_LIM.  The

second byte contains the value for FAR_LEAN_LIM.

-  far_mul_2 = A two byte parameter used for SCP control of  the  Fuel  Mass

Multiplier.   The  first  byte  contains the EEC's copy of the downloaded

multiplier, FAR_MUL_SCP.   The  second  byte  contains  a  response  code

indicating the status of the test, FAR_MUL_RSP.

-  far_mul_scp = Clipped value of dowloaded multiplier, unitless.

-  far_mul_tmr = Clipped value of timer used for ramp, sec.

-  spc_odu_ram = This parameter  is  a  PID  of  single  word  length  which

provides  the  beginning address of the ODU user RAM area.  This PID will

allow SBDS to download diagnostic routines to the user RAM  area  without

having  to  consult  the  DOC  files of the corresponding strategy.  This

parameter is not displayable.

8-323

FUEL STRATEGY, BACKGROUND F/A RATIO MULTIPLIER LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FUEL_FAR_MUL_COM1

BEGIN: fuel_mass_multiplier

;This process is executed once per background loop.

SCP_DWNLD_ST = 3 -----------------|

;SCP is in proper                |

;DLXQT state                     |

|

DLOAD_EXEC_KEY <> 8002h ----------|AND -| ;Allow LBMF_INJn

;Currently not executing         |     | ;Substitution in FG.

;any other routine               |     |

|     | Do: continue_test

DEMAND_MODE = 0 ------------------|     |

;No other On Demand test               | FAR_MUL_RSP = 00h

;test is active                        |  ;General response code for

|  ;affirmative response

|

| --- ELSE ---

|

FAR_MUL_SW = 1 -------------------------| ;Allow substitution for

;Enable multiplier for                 | ;calibration development only.

;development work                      | ;Switch must be poked with RCON

|

| Do: continue_test

|

| FAR_MUL_RSP = 44h

|  ;General response code for

|  ;ready for download

|

| --- ELSE ---

|

| ;Disable LBMF_INJn

| ;substitution.

|

| FAR_MUL = 1.0

| FAR_MUL_FLG = 0

| FAR_MUL_SCP = 0

| FAR_MUL_CHK = 0

| FAR_MUL_RDY = 0

| FAR_MUL_RUN = 0

|

| FAR_MUL_RSP = 22h

|  ;General response code for

|  ;conditions not correct

END: fuel_mass_multiplier

8-324

FUEL STRATEGY, BACKGROUND F/A RATIO MULTIPLIER LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: continue_test

;This process is executed only when explicitly called.

unconditionally ------------------------| FAR_MUL_SCP = (Contents of memory

|                location at address

|                (scp_odu_ram+C6h))

| ;Desired fuel mass multiplier as

| ;downloaded to 199th byte of ODU

| ;user RAM

|

| FAR_MUL_CHK = (Contents of memory

|                location at address

|                (scp_odu_ram+C7h))

| ;One's complement of desired

| ;multiplier as downloaded to 200th

| ;byte of ODU user RAM.

|

| :See definitions for description

| ;of scp_odu_ram.

N = 0 ----------------------------|

;Engine is not running           |

|AND -| ;Engine must be stopped to begin

FAR_MUL_CHK = (1's compliment     |     | ;test.

of FAR_MUL_SCP) ---|     |

;Robustness check of data              | FAR_MUL_RDY = 1

|  ;Test is ready to run.

|

| --- ELSE ---

UNDSP = 0 ------------------------|     |

;Engine is running               |AND -| ;Ramp in multiplier as soon as the

|     | ;engine is running.

FAR_MUL_RDY = 1 ------------------|     |

;Multiplier downloaded                 | Do: initialize_ramp

;and verified                          |

| far_mul_tmr = min(FAR_MUL_TMR,

|                   FAR_MUL_RAMP)

|  ;Clip to stop ramping at desired

|  ;multiplier value.  This clip is

|  ;necessary because the timer is

|  ;free-running.

|

| FAR_MUL = clip((FAR_MUL_SLP*

|                far_mul_tmr) + 1,

|                FAR_LEAN_LIM,

|                FAR_RICH_LIM)

|  ;Clip to ensure improper values

|  ;are not passed to foreground

|

| FAR_MUL_FLG = 1

|  ;Multiplier is active

END: continue_test

8-325

FUEL STRATEGY, BACKGROUND F/A RATIO MULTIPLIER LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: initialize_ramp

;This process is executed only when explicitly called.

FAR_MUL_RUN = 0 ------------------------| far_mul_scp = clip(FAR_MUL_SCP,

|                    FAR_LEAN_LIM,

|                    FAR_RICH_LIM)

|  ;Clip to ensure that slope

|  ;calculated below is accurate.

|

| FAR_MUL_SLP = (far_mul_scp - 1) /

|             FAR_MUL_RAMP

| FAR_MUL_RUN = 1

|  ;Lock out further resets while

|  ;running.

| FAR_MUL_TMR = 0

|  ;Reset ramp timer.

END: initialize_ramp

BEGIN: pid_definitions

The following logic defines PID messages for the fuel  mass  multiplier  used

for  demonstration  of the adaptive fuel monitor.  RAM and ROM parameters are

sent in separate messages to facilitate software coding.

pid_def(far_mul_1, FAR_RICH_LIM, FAR_LEAN_LIM)

;                      |             |

;rich limit -----------+             |

;lean limit -------------------------+

pid_def(far_mul_2, FAR_MUL_SCP, FAR_MUL_RSP)

;                      |            |

;EEC copy of desired   |            |

;multiplier -----------+            |

;response code ---------------------+

END: pid_definitions

8-326

FUEL STRATEGY, SPEED LIMITER SELFTEST CODE SETTING - OBDII - **S**

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8.48  SPEED LIMITER CONTINUOUS SELFTEST CODE SETTING - OBDII (CDAN0)

OVERVIEW

To assist service diagnosis of reported misfire a continuous self  test  code

is set when the fuel cut off speed limiter is invoked.

DEFINITIONS

INPUTS

Bit Flags:

-  CCM_TST_ENA = Continuous self test enabled.

-  FUEL_CUT = Fuel cutout active.  Set in  foreground  for  continuous  self

test.

OUTPUTS

Bit Flags:

-  FUEL_CUT = See above.

OTHERS

-  store_code (Pxxx) = Stores the fault code specified by Pxxx.

-  P1270 = Faultcode, RPM or VSS limit reached.

PROCESS

STRATEGY MODULE: FUEL_RPM_LIM_OBDII_COM1

CCM_TST_ENA = 1 ------------------|

|AND -| store_code(P1270)

FUEL_CUT = 1 ---------------------|     |  (store fault code P1270 in memory)

|

| FUEL_CUT = 0

|

| --- ELSE ---

|

| no action

8-327

FUEL STRATEGY, SPEED LIMITER SELFTEST CODE SETTING - OBDII - **S**

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8-328

CHAPTER 9

IGNITION TIMING STRATEGY

9-1

IGNITION TIMING STRATEGY, OVERVIEW - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

9.1  OVERVIEW (CDAI1)

OVERVIEW

The relationships of the calculations are shown  below  via  variable  usage.

The entries in the boxes are headings of sections in the strategy.

+------------+              +----------------+

| BORDERLINE |              | BASE EMISSIONS |              +---------------+

|   SPARK    |<-SPK_LAMBSE--|       AND      |--SPK_LAMBSE->| MAXIMUM BRAKE |

| DETONATION |              |  DRIVE SPARK   |              | TORQUE SPARK  |

+------------+              +----------------+              +---------------+

|                              |                              |       |

SPK_BDL                        SPK_BASE                      SPK_M_B_T   |

|                              |                              |       |

+--------------+     +---------+                              |       |

|     |                                        |       |

|     |    +----------------------+            |       |

|     |    |  SPARK FOR LOW LOAD  |      SPK_MBT_LAST  |

|     |    | OPERATING CONDITIONS |            |       |

|     |    +----------------------+            |       |

|     |       |                                |       |

|     |       |                                V       |

|     |       |             +-------------------+      |

|     |   SPK_LOW_LOAD      | TORQUE TRUNCATION |      |

|     |       |             | VIA SPARK RETARD  |      |

|     |       |             +-------------------+      |

+-------------+      |     |       |                  |      |              |

| OSCILLATION |      |     |       |      +-----------+      |              |

| MODULATION  |      |     |       |      |                  |              |

+-------------+      |     |       |  SPK_TRUNC              |              |

|       |         |     |       |      |                  |              |

|   SPK_OSC_OFF   V     V       V      V                  |              |

|       |      +---------------------------+              |              |

|       +----->| SELECT LOWEST SPARK VALUE |         LAST_LD_MBT         |

|              +---------------------------+              |              |

|                     |                                   |              |

|                SPK_ABS_LIM                              |              |

|                SPK_FLEX_LIM                             V              |

|                     |                   +-----------------------+      |

|                     +------------------>| TORQUE BASED FEEDBACK |      |

|                     |                   |     SPARK AT IDLE     |      |

SPK_OSCMOD               |                   +-----------------------+      |

|                     |                               |                  |

|                     |                          SPK_IDLE_FB             |

|                     |                           SPK_STATE              |

|                     V                               |                  |

|             +-------------------------+<------------+                  |

+------------>| FINAL SPARK CALCULATION |                                |

+-------------------------+                                |

|                                             |

SAF                                            |

|                                             |

V                                             |

+----------------+                                    |

| spark delivery |                         TR_SPK_DELTA

+----------------+                                    |

|                                             |

9-2

IGNITION TIMING STRATEGY, OVERVIEW - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

spout transition                                      V

9-3

IGNITION TIMING STRATEGY, FINAL SPARK CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

9.2  FINAL SPARK CALCULATION (CDAN0)

OVERVIEW

Spark advance final (SAF) is the value of the spark advance angle calculated

by the background strategy modules.  It is further modified by the foreground

modules and converted into a coded pulsewidth which is sent to the EDIS.

background      foreground      foreground

spark angle     spark angle     pulsewidth

------------------------------------------

SAF    --->   SAFTOT   --->    SAPW

SPK_SOURCE values if using the torque control chapter:

0       Crank or underspeed

1       Base

2       Borderline

3       Torque control - see TQ_SOURCE for further

clarification of torque truncation source

6       Stabilized low load

7       Cold engine low load

8       Idle feedback at minimum clip

9       Idle feedback

10      Spark set by on-demand self-test

11      Oscillation modulation modifying base

12      Oscillation modulation modifying borderline

13      Oscillation modulation modifying torque control

SPK_SOURCE values if not using the torque control chapter:

0       Crank or underspeed

1       Base

2       Borderline

3       Tip-in torque truncation

4       Transmission gear torque truncation

5       Vehicle speed limiter torque truncation

6       Stabilized low load

7       Cold engine low load

8       Idle feedback at minimum clip

9       Idle feedback

10      Spark set by on-demand self-test

11      Oscillation modulation modifying base

12      Oscillation modulation modifying borderline

13      Oscillation modulation modifying torque control

DEFINITIONS

Registers:

-  ER_SPARK = Value of SAF during on demand self test, deg.

-  SAF = Spark Advance Final, deg.

-  SPK_ABS_LIM = Spark is never allowed to go above this  value  because  of

detonation, torque limitation or base spark requirements.

9-4

IGNITION TIMING STRATEGY, FINAL SPARK CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  SPK_FLEX_LIM = This is a desirable spark  limit  which  may  be  exceeded

briefly when torque is required by a strategy such as feedback spark.

-  SPK_IDLE_FB = Spark by Feedback Spark logic.

-  SPK_OSCMOD = OSCMOD correction to other spark calculations.

-  SPK_SOURCE = Identifies the source calculation for the final spark value.

-  SPK_STATE = Defines the use of feedback spark, 0 -> feedback spark is not

being  used,  1  ->  transitioning  into  feedback  spark, 2 -> using the

feedback spark calculation, 3 -> transitioning out of feedback spark.

Bit Flags:

-  ER_SPK_REQ = Indicates that the on demand self test requires  SAF  to  be

equal to ER_SPARK; 1 -> SAF = ER_SPARK.

-  UNDSP = Underspeed Flag; 1 -> Underspeed or Crank Mode, 0 -> Run mode.

Calibration Constants:

-  SPK_ADD  =  Calibration  adder  to  over-all  spark  equation.   Used  to

explicitly specify spark advance when SPK_MUL is calibrated to zero, deg.

-  SPK_MUL = Calibration multiplier to over-all spark equation.   Typically,

SPK_MUL  is  set  to  zero  and  then SPK_ADD explicitly determines spark

advance, deg.

-  SPLCLP = Lower limit spark clip for rotor registry.

-  SPUCLP = Upper limit spark clip for rotor registry.

-  SUBST_REQ17 = Substitution required flag for channel 17.

9-5

IGNITION TIMING STRATEGY, FINAL SPARK CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SPARK_FINAL_CALC_COM2

BEGIN:  spark_final_calc_main

; This module executes every background loop.  Note that it reads top down.

; Determine if feedback spark calculation is being used.  Select

; either feedback spark or the flexible limit as the first limiting

; spark value.

SPK_STATE = 0 ---------------| saf :=  SPK_FLEX_LIM + SPK_OSCMOD

(Not in Feed                | spk_source_saf := spk_source_flex

Back Spark)                |

| --- ELSE ---

|

| saf = SPK_IDLE_FB + SPK_OSCMOD

| spk_source_saf := spk_source_fb

; Clip the value of the final spark advance so that it does not exceed

; SPK_ABS_LIM (although it may do so after being modified by the spark

; adder and multiplier, SPK_ADD and SPK_MUL).

SPK_ABS_LIM <= saf ----------| saf_fin := SPK_ABS_LIM * SPK_MUL + SPK_ADD

| spk_source_fin := spk_source_abs

|

| --- ELSE ---

|

| saf_fin := saf * SPK_MUL + SPK_ADD

| spk_source_fin := spk_source_saf

; Set final spark advance and note the source of the value.

ER_SPK_REQ = 1 --------------| t_saf := ER_SPARK

| SPK_SOURCE := 10

|  ; Spark for on-demand self-test

|

| --- ELSE ---

|

UNDSP = 0 -------------------| t_saf := saf_fin

| SPK_SOURCE := spk_source_fin

|  ; Normal spark

|

| --- ELSE ---

|

| t_saf := 10

| SPK_SOURCE := 0

|  ; Spark for crank/underspeed

9-6

IGNITION TIMING STRATEGY, FINAL SPARK CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUE:  spark_final_calc_main

; The final background value of spark advance in the temporary register,

; t_saf, is limited to the range:

;

;                      SPLCLP <= t_saf <= SPUCLP

unconditionally ----------| t_saf := MAX(t_saf,SPLCLP)

| t_saf := MIN(t_saf,SPUCLP)

| SAF := t_saf

; Output state control over-ride of final spark advance.  This routine will

; over-ride the SAF value if the engine-running self-test is not being

; performed and the engine is not underspeed.  This value over-rides all

; background spark calculations elsewhere in this module.

ER_SPK_REQ <> 1 -------|

|AND -| DO: substitute(17,saf_tmp)

UNDSP = 0 -------------|     | SAF := saf_tmp

|

| --- ELSE ---

|

SUBST_REQ17 = 1 -------------| DO: osc_response(17,22h)

|  ; OSC requested but operating conditions will

|  ; not allow substitute to occur

END:  spark_final_calc_main

9-7

IGNITION TIMING STRATEGY, SELECT LOWEST SPARK VALUE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

9.3  SELECT LOWEST SPARK VALUE (CDAL0)

OVERVIEW

Select the most retarded value of  spark  from  the  base  spark,  borderline

detonation limit, torque truncation and cold start spark calculations.

DEFINITIONS

Registers:

-  SPK_ABS_LIM = Spark is never allowed to go above this  value  because  of

detonation, torque limitation or base spark requirements.

-  SPK_BASE = Desired spark advance for optimal emissions and  driveability,

deg.

-  SPK_BDL = Borderline detonation spark limit, deg.

-  SPK_FLEX_LIM = This is a desirable spark  limit  which  may  be  exceeded

briefly  when  torque is required by a strategy such as feedback spark or

oscmod.

-  SPK_LOW_LOAD = Idle emissions spark ceiling.  Lowest of Cold Start  Spark

and Inspection Maintenance requirements.

-  SPK_OSC_OFF = The amount by which the base spark calculation  is  reduced

to make room for an increase as calculated by the OSCMOD strategy.

-  SPK_TRUNC = Spark ceiling when engine torque needs to be limited.

9-8

IGNITION TIMING STRATEGY, SELECT LOWEST SPARK VALUE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SPARK_LOWEST_COM1

BEGIN:  spark_lowest_main

; This process executes once each background loop.  It must be executed

; before the SPARK_FINAL_CALC_COMn module.  Note that it reads top-down.

unconditionally ----------| DO:  spark_flexible_limit

| DO:  spark_absolute_limit

END:  spark_lowest_main

BEGIN:  spark_flexible_limit

; This process executes when called from spark_lowest_main.  Note that it

; reads top-down.

; Calculate the flexible spark limit, SPK_FLEX_LIM, by determining the more

; retarded of the sum of SPK_ABS_LIM plus the OSCMOD offset and

; SPK_LOW_LOAD.  This value of spark may be exceeded by the oscillation

; modulation and feedback spark calculations.

abs_plus_off > SPK_LOW_LOAD ------------| SPK_FLEX_LIM := SPK_LOW_LOAD

| spk_source_flex := spk_source_low

|

| --- ELSE ---

|

| SPK_FLEX_LIM := abs_plus_off

| spk_source_flex := 10

|                  + spk_source_abs

unconditionally ------------------------| abs_plus_off := SPK_ABS_LIM +

SPK_OSC_OFF

END:  spark_flexible_limit

BEGIN: spark_absolute_limit

; This process executes when called from spark_lowest_main.  Note that it

; reads top-down.

; Calculate the absolute spark limit, SPK_ABS_LIM by determining the

; most retarded of base, borderline and torque truncation spark values.

intermed_lim > SPK_BASE ----------------| SPK_ABS_LIM := SPK_BASE

| spk_source_abs := 1

|

| --- ELSE ---

|

| SPK_ABS_LIM := intermed_lim

| spk_source_abs := spk_source_int

9-9

IGNITION TIMING STRATEGY, SELECT LOWEST SPARK VALUE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUE:  spark_absolute_limit

; Calculate the intermediate spark limit which is needed by the

; absolute spark limit calculation, by determining the most retarded

; of the borderline and torque truncation values.

SPK_BDL > SPK_TRUNC --------------------| intermed_lim := SPK_TRUNC

| spk_source_int := spk_source_trunc

|

| --- ELSE ---

|

| intermed_lim := SPK_BDL

| spk_source_int := 2

END:  spark_absolute_limit

9-10

IGNITION TIMING STRATEGY, BASE EMISSIONS AND DRIVE SPARK - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

9.4  BASE EMISSIONS AND DRIVE SPARK (CDAL0)

OVERVIEW

The base  spark  strategy  is  essentially  MBT  spark  modified  to  achieve

functional requirements for emissions and drive quality.

DEFINITIONS

INPUTS

Registers:

-  BP = Barometric Pressure, " Hg.

-  DASPOT = Dashpot contribution to idle air  flow,  ppm.   See  Idle  Speed

Control  chapter  for  calculation.   Used here as input to Dashpot Spark

multiplier, FN839.

-  ECT = Engine coolant temperature, deg F.

-  EGRACT = Actual EGR percent = 100 * EM / AMPEM.

-  ISCFLG = ISC mode indicator; -1 -> Dashpot Mode; 0 -> Dashpot Preposition

Mode;  1  ->  Closed  Loop RPM Control Mode; 2 -> Closed Loop RPM Control

(Lock-out entry to RPM control.)

-  LOAD = Universal load variable.  Normalized CYLARC_BG divided by  SARCHG,

unitless.

-  N = Engine RPM.

-  SPKBASE_IMRC =  Base  spark  adder  when  using  intake  manifold  runner

control, set to zero otherwise.  Degrees, positive for advance.

-  SPKBASE_VCT = Base spark adder when using variable  cam  timing,  set  to

zero otherwise.  Degrees, negative for retard, positive for advance.

-  SPK_CMB_ADJ = Spark modifier for engine combustion noise, degrees.

-  SPK_LAMBSE = Value of LAMBSE to be used in spark calculations, unitless.

-  TP_REL = RELATIVE TP ( TP - RATCH ).

Bit Flags:

-  ALT_CAL_FLG = Flag to indicate use of alternate calibration.   1  ->  use

anti-plug fouling strategy, 0 -> use standard strategy.

Calibration Constants:

-  FN012(LOAD) = Normalized LOAD for full-sized spark tables.

9-11

IGNITION TIMING STRATEGY, BASE EMISSIONS AND DRIVE SPARK - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FN013(LOAD) = Normalized LOAD for half-sized spark tables.

-  FN014(ECT) = Normalized ECT for spark calculation.

-  FN014_ALT(ECT) = Normalized ECT for spark  calculation  during  anti-plug

fouling mode.

-  FN015(SPK_LAMBSE) = Normalized LAMBSE for spark calculation.

-  FN016(N) = Normalized rpm for full-sized spark tables.

-  FN017(N) = Normalized rpm for half-sized spark tables.

-  FN700(TP_REL) = Throttle position multiplier for engine combustion  noise

spark modifier.

-  FN702(ECT) = Engine coolant temperature multiplier for engine  combustion

noise spark modifier.

-  FN711(LOAD) = The increase in spark for each percent of  EGR,  deg  /  _%

EGR.

-  FN712(BP) = Proportions the contribution from the base sea level and base

altitude spark tables as a function of altitude, unitless.

-  FN839(DASPOT) = Decel spark multiplier as a function of dashpot air flow.

-  FN2100(N,LOAD) = Base spark table at sea level.  The values in this table

should  be  used  at  all  altitudes  unless  there are specific altitude

related emissions reasons, deg.

X = FN016(N)

Y = FN012(LOAD)

-  FN2110(N,LOAD) = Base spark table at altitude.  These  values  should  be

the  same  as the base sea level table unless there are specific altitude

emissions related reasons to justify a change, deg.

X = FN016(N)

Y = FN012(LOAD)

-  FN2115(N,LOAD) = Engine combustion noise spark modifier, degrees.

X = FN016(N)

Y = FN012(LOAD)

-  FN2120(ECT,LOAD) = The change in desired spark as a  function  of  engine

coolant temperature, deg.

X = FN014(ECT) be visible.

Y = FN013(LOAD)

-  FN2120_ALT(ECT,LOAD) = The change in  desired  spark  as  a  function  of

engine coolant temperature and load during anti-plug fouling mode, deg.

X = FN014_ALT(ECT)

Y = FN013(LOAD)

-  FN2140(N,SPK_LAMBSE) =  This  table  is  intended  to  adjust  spark  for

whatever  calibration  adjustments  which  may be necessary.  Examples of

such  adjustments  are  retarding  spark  when  lean  for  improved  NOX,

adjusting  spark  when  rich due to wide open throttle and over-advancing

spark to reduce catalyst temperatures.

X = FN017(N)

9-12

IGNITION TIMING STRATEGY, BASE EMISSIONS AND DRIVE SPARK - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Y = FN015(SPK_LAMBSE)

OUTPUTS

Registers:

-  SPK_BASE = Desired spark advance for optimal emissions and  driveability.

Degrees BTDC.

9-13

IGNITION TIMING STRATEGY, BASE EMISSIONS AND DRIVE SPARK - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SPARK_BASE_COM10

BEGIN:  spark_base_main

; This module executes each background loop.  Note that it reads top down.

; Alternate calibration for anti-plug fouling strategy.

ALT_CAL_FLG = 1 ---------| fn2120 := FN2120_ALT(ECT,LOAD)

|

| --- ELSE ---

|

| fn2120 := FN2120(ECT,LOAD)

; Calculate modifier for engine combustion noise reduction.  This

; should be calibrated such that little or no spark retard is used

; when the driver is asking for power (via throttle position TP_REL).

; Also, it may be calibrated for little or no spark retard at low

; or high temperatures.

unconditionally ---------| SPK_CMB_ADJ := FN2115(N,LOAD)

|              * FN700(TP_REL)

|              * FN702(ECT)

; Main base spark calculation.

unconditionally ----------| spk_base_temp := FN2100(N,LOAD) * FN712(BP)

| + FN2110(N,LOAD) * (1 - FN712(BP))

| + fn2120

| + FN711(LOAD) * EGRACT

| + FN2140(N,SPK_LAMBSE)

| + SPK_CMB_ADJ

| + SPKBASE_IMRC

| + SPKBASE_VCT

; If in dashpot mode of idle speed control, use a dashpot multiplier

; on the base spark calculation.

ISCFLG = -1 --------------| SPK_BASE := spk_base_temp * FN839(DASPOT)

|

| --- ELSE ---

|

| SPK_BASE := spk_base_temp

END:  spark_base_main

9-14

IGNITION TIMING STRATEGY, BORDERLINE DETONATION SPARK - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

9.5  BORDERLINE DETONATION SPARK (CDAN0)

OVERVIEW

This logic calculates the spark advance at which borderline level  detonation

takes place.

One part of BORDERLINE_SPK is  the  Transient  Spark  strategy  (also  called

lugging mode strategy) which attempts to reflect the ability of the engine to

tolerate higher spark  without  detonation  during  transient  conditions  as

opposed  to steady state conditions.  One possible explanation is that during

a tip-in cylinder temperature increases from one  stabilized  temperature  to

another  higher  temperature and that this change is not reflected adequately

in ECT.  The result is that the engine can tolerate a greater  spark  advance

for a few seconds until the cylinder temperature stabilizes at its new level.

It is also thought that the amount of swirl of the air  charge  may  have  an

effect.  This strategy provides a means to supply an increase in spark during

transitions.

DEFINITIONS

Registers:

-  ACT = Air Charge Temperature, deg F.

-  BP = Barometric Pressure, " Hg.

-  CYL_AIR_CHG = Current cylinder air charge.

-  DASPOT = Dashpot contribution to idle air  flow,  ppm.   See  Idle  Speed

Control  chapter  for  calculation.   Used here as input to Dashpot Spark

multiplier, FN839.

-  ECT = Engine coolant temperature, deg F.

-  EGRACT = Actual EGR percent = 100 * EM / AMPEM.

-  ISCFLG = ISC mode indicator; -1 -> Dashpot Mode; 0 -> Dashpot Preposition

Mode;  1  ->  Closed  Loop RPM Control Mode; 2 -> Closed Loop RPM Control

(Lock-out entry to RPM control.)

-  LOAD = Universal load variable.

-  LUGTMR =  The  time  since  the  engine  transitioned  into  higher  load

operating conditions.

-  N = Engine RPM.

-  OCTADJ = Field Octane Adjust input, unitless;  1  ->  Retard  for  octane

adjustment is desired, 0 -> No retard is desired.

-  SPK_BDL = Borderline detonation spark limit.  Degrees BTDC.

9-15

IGNITION TIMING STRATEGY, BORDERLINE DETONATION SPARK - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  SPK_BDL_FFS = Borderline spark adder when using  flexible  fuel,  set  to

zero otherwise.  Degrees, negative for retard, positive for advance.

-  SPK_BDL_VCT = Borderline spark adder when using variable cam timing,  set

to zero otherwise.  Degrees, negative for retard, positive for advance.

-  SPK_LAMBSE = Value of LAMBSE to be used in spark calculations, unitless.

-  SPK_TIP_IMRC = Difference between IMRC open and IMRC  closed  slopes  for

the foreground borderline update.

Bit Flags:

-  SPK_BDL_UPDT = Flag to inform the background manager that the  borderline

spark module must be executed on a priority basis.

Calibration Constants:

-  FN012(LOAD) = Normalized LOAD for full-sized spark tables.

-  FN013(LOAD) = Normalized LOAD for half-sized spark tables.

-  FN015(SPK_LAMBSE) = Normalized LAMBSE for spark calculation.

-  FN016(N) = Normalized RPM for full-sized spark tables.

-  FN017(N) = Normalized RPM for half-sized spark tables.

-  FN721(LOAD) = Increase in the detonation limit as a function  of  percent

EGR, deg / % EGR.

-  FN722(BP) = The octane level of fuel sold at high altitude  is  typically

lower than equivalent fuel grades sold as sea level.  This function gives

the calibrator the option of compensating the spark calculation for  fuel

quality as a function of altitude.

-  FN723(LUGTMR) = Transient increase in the detonation limit  following  an

increase in load, degrees.

-  FN724A(ECT) = Multiplier which corrects  for  the  non-linear  effect  of

coolant

-  FN725A(ACT) = Multiplier which corrects  for  the  non-linear  effect  of

ambient air temperature on borderline detonation, unitless.

-  FN726(N) = Change to the borderline calculation when  the  octane  adjust

pin is grounded, deg.

-  FN839(DASPOT) = Decel spark multiplier as a function of dashpot air flow.

-  FN2200(N,LOAD) = Base borderline detonation table, degrees.

X = FN016(N)

Y = FN012(LOAD)

-  FN2220(N,LOAD)   =   Borderline   detonation   adjustment   for   coolant

temperature, degrees spark/degree F.

X = FN017(N)

Y = FN013(LOAD)

9-16

IGNITION TIMING STRATEGY, BORDERLINE DETONATION SPARK - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FN2230(N,LOAD)  =  Borderline  detonation  adjustment  for   air   charge

temperature, degrees spark/degree F.

X = FN017(N)

Y = FN013(LOAD)

-  FN2240(N,SPK_LAMBSE) = Borderline detonation adjustment for a  change  in

Air/Fuel ratio.

X = FN017(N)

Y = FN015(SPK_LAMBSE)

-  FN2250(N,LOAD) = Increase in spark values that the  engine  can  tolerate

without  detonation after a load increase but before the engine begins to

knock.

X = FN017(N)

Y = FN013(LOAD)

-  OCTADJHP = Octane adjust input hardware present switch; 0  ->  no  OCTADJ

input exists, 1 -> an OCTADJ input is present.

-  SARCHG = Air charge mass of cylinder vol.  at standard BP.

9-17

IGNITION TIMING STRATEGY, BORDERLINE DETONATION SPARK - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SPARK_BORDERLINE_COM7

BEGIN:  spark_borderline_main

; This module executes each background loop.  Note that it reads top down.

; Calculate borderline spark adder for the octane adjust service switch.

OCTADJ = 1 ------------------------|

; octane adjust input is high      |AND -| bl_octane_adjust_add := FN726(N)

|     |

OCTADJHP = 1 ----------------------|     |

; octane adjust feature enabled          |

| --- ELSE ---

|

| bl_octane_adjust_add := 0

; If this calculation of SPK_BDL is for the pre-emptive tip-in retard

; logic, the value of LOAD must be updated to reflect the current value

; of cylinder air charge.

SPK_BDL_UPDT = 1 ------------------------| LOAD := CYL_AIR_CHG / SARCHG

; Calculate borderline detonation spark angle.

always --------------| spk_bdl_temp := FN2200(N,LOAD)

| + FN2220(N,LOAD) * FN724A(ECT)

| + FN2230(N,LOAD) * FN725A(ACT)

| + FN2240(N,SPK_LAMBSE)

| + FN2250(N,LOAD) * FN723(LUGTMR)

| + FN721(LOAD) * EGRACT

| + FN722(BP)

| + bl_octane_adjust_add

| + SPK_BDL_FFS

| + SPK_BDL_IMRC

| + SPK_BDL_VCT

; Add dashpot spark multiplier if in dashpot idle speed control mode.

ISCFLG = -1 ---------| SPK_BDL := spk_bdl_temp * FN839(DASPOT)

|

| --- ELSE ---

|

| SPK_BDL := spk_bdl_temp

9-18

IGNITION TIMING STRATEGY, BORDERLINE DETONATION SPARK - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUE:  spark_borderline_main

; Clear the pre-emptive calculation flag used for the foreground tip-in

; retard logic.

unconditionally -----| SPK_BDL_UPDT := 0

END:  spark_borderline_main

9-19

IGNITION TIMING STRATEGY, SPARK FOR LOW LOAD OPERATING CONDITIONS - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

9.6  SPARK FOR LOW LOAD OPERATING CONDITIONS (CDAA0)

OVERVIEW

Spark retard is used to increase catalyst temperatures by  forcing  increased

air  mass  flow.   This approach is used in the Cold Start Low Load Spark and

Stabilized Low Load Spark strategies.   A  second  application  is  to  limit

piston slap (cold knock) at colder engine temperatures.

The Cold Idle Spark logic provides  an  upper  clip  for  spark  during  cold

starts.   The clip serves to retard the spark below MBT which causes the idle

speed control to deliver a higher air mass flow through the engine  in  order

to  maintain  the  desired  rpm.  The higher air mass flow serves to heat the

catalyst so that it can begin to function sooner.

During extended idles the air mass flow is low enough that the catalyst cools

and  becomes  less efficient.  The Stabilized Low Load strategy retards spark

under stabilized idle conditions to maintain catalyst temperature.

DEFINITIONS

INPUTS

Registers:

-  AM = Air mass flow, lb/min.

-  APT = At Part Throttle; -1 -> Closed throttle, 0 -> Part throttle,  1  ->

Wide Open throttle.

-  ATMR3 = Time since entering run mode, sec.

-  ECT = Engine coolant temperature, deg F.

-  ISCFLG = ISC mode indicator flag;  -1  ->  Dashpot  Mode,  0  ->  Dashpot

Preposition Mode, 1 -> closed Loop RPM Control Mode, 2 -> Closed Loop RPM

Control (Lockout entry to RPM control).

-  LOAD = Universal load variable.  Normalized CYLARC_BG divided by  SARCHG,

unitless.

-  N = Engine RPM.

-  STB_LOLD_TMR  =  Timer  to  control  spark  clip  for  Stable  Low   Load

conditions, sec.

Calibration Constants:

-  CLLS_AT_PT = Enables and disables the cold low load spark  clip  at  part

throttle, unitless.

-  FN012(LOAD) = Normalized LOAD for full-sized spark tables.

9-20

IGNITION TIMING STRATEGY, SPARK FOR LOW LOAD OPERATING CONDITIONS - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FN013(LOAD) = Normalized LOAD for half-sized spark tables.

-  FN016(N) = Normalized RPM for full-sized spark tables.

-  FN017(N) = Normalized RPM for half-sized spark tables.

-  FN018B(ATMR3) = ATMR3 normalizing function.

-  FN020C(ECT) = Temperature normalizing function for ECT.

-  FN741(STB_LOLD_TMR) = Ceiling for spark  limiting  for  Stable  Low  Load

conditions.

-  FN2000 = Maximum spark allowed during a cold start.  This  spark  depends

on  the  coolant  temperature which is essentially the temperature at the

start for the relatively short time during which this table is used.

X = FN020C(ECT)

Y = FN018B(ATMR3)

-  FN2001 = Adds spark to the cold idle  clip  when  the  speed  load  point

results  in an air mass flow which is high enough so that retarding spark

is not necessary to achieve catalyst light off.  It also allows disabling

the  clip  when  the load is high enough to indicate that the customer is

demanding performance.

X = FN017(N)

Y = FN013(LOAD)

-  SPUCLP = Rotor registry upper clip, degrees BTDC.

-  STAB_LOLD_AM = Value of AM below which spark may be  clipped  for  Stable

Low Load conditions.

Bit Flags:

-  DNDSUP = Delayed Neutral/Drive flag; 0 -> neutral, 1 -> drive.

OUTPUTS

Registers:

-  SPK_LOW_LOAD = Emissions spark value at idle.  It is  the  lower  of  the

Cold Low Load and Stable Low Load calculations.

-  STB_LOLD_TMR  =  Timer  to  control  spark  clip  for  Stable  Low   Load

conditions, sec.

Bit Flags:

-  SLL_DWN_FLG = Controls STB_LOLD_TMR; 1 -> decrement STB_LOLD_TMR, 0 -> no

change to STB_LOLD_TMR.

-  SLL_UP_FLG = Controls STB_LOLD_TMR; 1 -> increment STB_LOLD_TMR, 0 ->  no

change to STB_LOLD_TMR.

9-21

IGNITION TIMING STRATEGY, SPARK FOR LOW LOAD OPERATING CONDITIONS - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SPARK_LOW_LOAD_COM1

Set SPK_LOW_LOAD to the more retarded of Cold Low Load and  Stable  Low  Load

spark:

spk_lo_ld_cold > spk_lo_ld_stab ------| SPK_LOW_LOAD = spk_lo_ld_stab

| spk_source_low = 6

|

| --- ELSE ---

|

| SPK_LOW_LOAD = spk_lo_ld_cold

| spk_source_low = 7

Cold Idle Spark:

CLLS_AT_PT = 1 -----------------|

|OR --| spk_lo_ld_cold = FN2000(ECT,ATMR3)

APT = -1 -----------------------|     |               + FN2001(N,LOAD)

(closed throttle)                    |

| --- ELSE ---

|

| spk_lo_ld_cold = SPUCLP

Stabilized Low Load strategy:

DNDSUP = 0 ---------------------------| spk_lo_ld_stab = FN741(STB_LOLD_TMR)

(in neutral)                         |

| --- ELSE ---

|

| spk_lo_ld_stab = SPUCLP

ISCFLG > 0 ---------------------|

(RPM control or lockout)       |

|AND -| Increment STB_LOLD_TMR

AM <= STAB_LOLD_AM -------------|     | SLL_UP_FLG = 1

(Low exhaust flow)                   | SLL_DWN_FLG = 0

|

| --- ELSE ---

ISCFLG > 0 ---------------------|     |

(RPM control or lockout)       |     |

|AND -| Decrement STB_LOLD_TMR

AM > STAB_LOLD_AM --------------|     | SLL_UP_FLG = 0

(High exhaust flow)                  | SLL_DWN_FLG = 1

|

| --- ELSE ---

|

| STB_LOLD_TMR = 0

| SLL_UP_FLG = 0

| SLL_DWN_FLG = 0

9-22

IGNITION TIMING STRATEGY, TORQUE TRUNCATION VIA SPARK RETARD - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

9.7  TORQUE TRUNCATION VIA SPARK RETARD (CDAE0)

OVERVIEW

Torque truncation is the reduction of spark in order  to  reduce  the  torque

delivered  by  the  engine.   This  approach is used here for three different

problems:

1.  Limit  vehicle   speed   because   of   tire   manufacturers   speed

restrictions.

2.  Limit  engine  torque  in  because  under  certain  conditions  some

automatic transmissions are unable to handle maximum engine torque.

3.  Limit the rate of rise of engine torque  during  tip-ins  to  reduce

powertrain windup and impact caused by drive train lash.

DEFINITIONS

INPUTS

Registers:

-  BG_TMR = Background loop time, secs.

-  DNDT_SPK = Rate of change of  engine  RPM  for  use  in  tip  in  retard,

RPM/S*S.  This parameter is calculated at the start of the spark routine,

which means a recent value of DNDT is used.

-  DNDT_LST = Last pass value for DNDT_SPK, RPM/S*S.

-  ECT = Engine coolant temperature, deg F.

-  LOAD = Universal load variable.  Normalized CYLARC_BG divided by  SARCHG,

unitless.

-  N = Engine RPM.

-  NOVS = Ratio of engine speed to vehicle speed used to infer  transmission

gear, rpm/mph.

-  SAF = Spark Advance Final, deg.

-  SPK_M_B_T = Spark advance required to achieve Maximum Brake Torque.

-  SPK_MBT_LAST = Last pass value of SPK_M_B_T, degrees.

-  TIP_ALLOW_LD = Allowed load limit with spark assumed to be  at  MBT,  for

tip in spark retard, unitless.

-  TIP_LAST_LOAD = Last background loop load value, unitless.

9-23

IGNITION TIMING STRATEGY, TORQUE TRUNCATION VIA SPARK RETARD - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TR_TQ_LIMIT = Torque ratio required to achieve TQ_LIMIT.

-  VSBAR = Filtered vehicle speed.

Bit Flags:

-  FLG_TQ_TRUN = Torque truncation flag; 1 -> torque truncation required.

-  HSPFLG = High speed mode flag; 1 -> high speed alternate fuel/spark.

Calibration Constants:

-  SPK_DNDT_C = DNDT hysterysis to restore tip-in retard, RPM/S*S.

-  SPK_TIP_ECT = ECT below which tip-in retard is disabled, deg F

-  SPK_TIP_NOVS = NOVS below which tip-in retard is disabled, RPM/MPH.

-  SPK_LDRATE = Allowable rate of  increase  in  LOAD  to  determine  tip-in

retard spark advance, LOAD/SEC.

-  FN013(LOAD) = Normalized LOAD for half-sized spark tables.

-  FN017(N) = Normalized RPM for half-sized spark tables.

-  FN179(NOBAR) = Offset from SPK_M_B_T which determines  maximum  allowable

spark  when  it  is  desired  to limit vehicle speed through spark.  This

function contains negative numbers indicating retard from SPK_M_B_T.

-  FN766 = Transfer function which calculates the torque ratio at  which  an

engine is operating from the spark delta from MBT.

-  FN799(TIP_TR or TR_TQ_LIMIT) = Transfer function which  returns  a  spark

retard from MBT as a function of desired torque ratio, degrees.

-  SPUCLP = Rotor registry upper clip, degrees BTDC.

-  TIP_MIN_SPK = Spark calculated using the tip in spark retard strategy  is

not allowed below this calibration parameter, degrees.

-  TIP_TR_TRIG = Torque ratio above which tip in spark retard  is  disabled.

This is a fraction just less than 1, unitless.

9-24

IGNITION TIMING STRATEGY, TORQUE TRUNCATION VIA SPARK RETARD - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OUTPUTS

Registers:

-  LAST_LD_MBT = Load from the last pass corrected to the value  load  would

have  to have if spark was at MBT in order for the engine to generate the

same torque output.

-  SPK_TRUNC = Highest allowed value of spark for torque truncation reasons.

-  TIP_ALLOW_LD = Allowed load limit with spark assumed to be  at  MBT,  for

tip in spark retard, unitless.

-  TIP_DELTA = Incremental increase load  (corrected  to  MBT)  allowed  for

current background loop, unitless.

-  DNDT_LST = Last pass value of DNDT_SPK, RPM/S*S.

-  TIP_LAST_LD = Last background loop load value, unitless.

-  TIP_TR = Calculated torque ratio allowed  for  current  background  loop,

unitless.

9-25

IGNITION TIMING STRATEGY, TORQUE TRUNCATION VIA SPARK RETARD - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SPARK_TRQ_TRNC_COM1

Calculate the torque truncation spark, SPK_TRUNC, as the lowest value of  the

sources  of torque truncation:  1) vehicle speed limiting, 2) low gear torque

limiting and 3) tip in torque retard:

spk_trunc_low > spk_trunc_tipret ------| SPK_TRUNC = spk_trunc_tipret

| spk_source_trunc = 3

|

| --- ELSE ---

|

| SPK_TRUNC = spk_trunc_low

| spk_source_trunc = spk_source_tlow

spk_trunc_vss > spk_trunc_gear --------| spk_trunc_low = spk_trunc_gear

| spk_source_tlow = 4

|

| --- ELSE ---

|

| spk_trunc_low = spk_trunc_vss

| spk_source_tlow = 5

Vehicle speed limiter.  The spark speed limiter strategy must be  coordinated

with the fuel speed limiter strategy.

HSPFLG = 1 ----------------------------| spk_trunc_vss = SPK_M_B_T +

(vehicle speed too high)              |                   FN179(NOBAR)

| --- ELSE ---

|

| spk_trunc_vss = SPUCLP

Torque truncation for transmission.

FLG_TQ_TRUN = 1 -----------------------| spk_trunc_gear =

|   SPK_M_B_T + FN799(TR_TQ_LIMIT)

|

| --- ELSE ---

|

| spk_trunc_gear = SPUCLP

9-26

IGNITION TIMING STRATEGY, TORQUE TRUNCATION VIA SPARK RETARD - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TIP-IN RETARD

Tip-in retard limits the rise in engine torque by comparing the ratio of  the

desired  torque  to  the  actual  torque  if  at  MBT.   Once  this  ratio is

calculated, the spark is retarded to reach the desired torque.  Load is  used

in  this strategy instead of torque because it is roughly proportional to the

indicated torque and readily available and widely understood.

Calculate LAST_LD_MBT which is defined  as  the  load  from  the  last  pass,

corrected  to  the value load would have to have if spark was at MBT in order

for the engine to generate the same torque output.  The calculation uses  the

last pass values of TIP_LAST_LD and SAF.

LAST_LD_MBT = TIP_LAST_LD * FN766(SPK_MBT_LAST - SAF)

Calculate the allowable increase in torque expressed as  load.   If  DNDT  is

decreasing  rapidly  allow  spark back to the maximum value (indicates engine

has moved through the lash region).  Tip-in retard can be  disabled  in  high

gears  by  NOVS.   It  can  also  be disabled at cold ECT if the spark retard

creates poor combustion issues at the low temperatures.

DNDT_SPK < DNDT_LST - SPK_DNDT_C -|

|

NOVS < SPK_TIP_NOVS --------------|OR --| TIP_DELTA = 1

|     |

ECT < SPK_TIP_ECT ----------------|     |

| --- ELSE ---

|

| TIP_DELTA = SPK_LDRATE * BG_TMR

Calculate the allowed MBT load from the allowed  load  calculated  last  pass

using  the last pass value of TIP_ALLOW_LD and the actual delivered MBT load,

LAST_LD_MBT.

TIP_ALLOW_LD < LAST_LD_MBT ------------| TIP_ALLOW_LD = TIP_ALLOW_LD +

|                   TIP_DELTA

|

| --- ELSE ---

|

| TIP_ALLOW_LD = LAST_LD_MBT +

|                   TIP_DELTA

9-27

IGNITION TIMING STRATEGY, TORQUE TRUNCATION VIA SPARK RETARD - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Prepare the uncompensated load, TIP_LAST_LD, and the current DNDT_SPK for the

next pass:

TIP_LAST_LD = LOAD

DNDT_LST = DNDT_SPK

Using new values calculate the allowed torque ratio:

TIP_TR = TIP_ALLOW_LD / LOAD

Make sure this value of spark is  calculated  only  when  tip  in  retard  is

desired.

TIP_TR < TIP_TR_TRIG ------------------| spk_tip = SPK_M_B_T +

|             FN799(TIP_TR)

|

| --- ELSE ---

|

| spk_tip = SPUCLP

Do not allow a spark value below a calibratable minimum, TIP_MIN_SPK:

spk_tip < TIP_MIN_SPK -----------------| spk_trunc_tipret = TIP_MIN_SPK

|

| --- ELSE ---

|

| spk_trunc_tipret = spk_tip

9-28

IGNITION TIMING STRATEGY, MAXIMUM BRAKE TORQUE SPARK - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

9.8  MAXIMUM BRAKE TORQUE SPARK (CDAL0)

OVERVIEW

The MBT spark calculation is used for real  time  torque  calculations.   The

calibration   is  provided  by  engine  mapping.   The  use  of  a  different

calibration  will  affect  the  torque  accuracy.   For  this   reason,   the

calibration provided by engine mapping should not be modified.

DEFINITIONS

Registers:

-  ECT = Coolant temperature degrees farenheit.

-  EGRACT = Actual EGR rate.

-  LOAD = Universal LOAD as ratio of air charge over standard.

-  N = Engine speed RPM.

-  SPK_LAMBSE = The lambse value used in  spark  calculations.   It  is  set

equal to 1 when fuel is in closed loop otherwise it is LAMBSE.

-  SPK_M_B_T = Spark advance  required  to  achieve  Maximum  Brake  Torque.

Degrees BTDC.

-  SPK_MBT_IMRC = Maximum brake torque  adder  when  using  intake  manifold

runner control, set to zero otherwise.  Degrees, positive for advance.

-  SPK_MBT_LAST = Last pass value of SPK_M_B_T.  Degrees BTDC.

-  SPK_MBT_VCT = Maximum brake torque spark adder when  using  variable  cam

timing,  set  to  zero otherwise.  Degrees, negative for retard, positive

for advance.

Calibration Constants:

-  FN013(LOAD) = Normalized LOAD for half sized spark tables, unitless.

-  FN014(ECT) = Normalized ECT for spark calculation, unitless.

-  FN034A(LOAD)  =  Load  normalizing  function  for   torque   calculation,

unitless.

-  FN070C(N) = RPM normalizing function for torque calculation, unitless.

-  FN730(SPK_LAMBSE) = MBT spark adder for rich and lean A/F.

-  FN731(LOAD) = Function of load  which  multiplies  percent  EGR  for  the

purpose of adjusting MBT spark.

-  FN2300(N,LOAD) = MBT spark at 14.6 A/F and 0% EGR.

X = FN070C(N)

Y = FN034A(LOAD)

9-29

IGNITION TIMING STRATEGY, MAXIMUM BRAKE TORQUE SPARK - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FN2320(ECT,LOAD) = Compensates MBT for engine coolant temperature.

X = FN014(ECT)

Y = FN013(LOAD)

9-30

IGNITION TIMING STRATEGY, MAXIMUM BRAKE TORQUE SPARK - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SPARK_MBT_COM1

BEGIN: SPARK_MBT

; This module executes each background loop.  Note that it reads top down.

; Store MBT value calculated last background loop.

unconditionally ----------| SPK_MBT_LAST := SPK_M_B_T

; Calculate maximum brake torque spark.

unconditionally ----------| SPK_M_B_T := FN2300(N,LOAD)

| + FN2320(ECT,LOAD)

| + FN731(LOAD) * EGRACT

| + FN730(SPK_LAMBSE)

| + SPK_MBT_IMRC

| + SPK_MBT_VCT

END: SPARK_MBT

9-31

IGNITION TIMING STRATEGY, TORQUE BASED FEEDBACK SPARK AT IDLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

9.9  TORQUE-BASED FEEDBACK SPARK AT IDLE (CDAC0)

STRATEGY MODULE: SPARK_IDLE_FB_COM5

OVERVIEW

The feedback spark strategy is a closed loop control algorithm which operates

at  idle  and  attempts  to  maintain  a desired engine rpm by increasing and

decreasing engine torque through spark advance  adjustmnts.   The  controller

reacts  to  the rpm error from the desired rpm with a proportional correction

of spark.  RPM control is accomplished in conjunction  with  the  Idle  Speed

Control  and Fuel strategies.  The reason that spark is used for this problem

is that spark advance is the quickest responding EEC  output  which  directly

affects  engine  output  torque.   This spark strategy supplies engine torque

quickly  and  the  correction  disappears  as  the  fundamental  response  of

increased air flow supplied by the ISC strategy phases in.

The approach to control rpm through spark advance adjustments is  intuitively

understandable  but  the  task  of  accomplishing  it  is  complicated by two

nonlinearities.  First, the relationship between spark advance and torque  is

non-linear,  and  second, the relationship between an rpm error and torque is

non-linear.  This strategy takes advantage  of  a  torque  ratio  formulation

which results in three benefits:

1.  The strategy  linearizes  the  relationship  between  spark  advance  and

torque.  In other words the requested torque is accurately delivered.

2.  The desired torque compensation is correct regardless of engine operating

conditions such as rpm and load.

3.  The control algorithm is calibratable with one  parameter  (SPK_LOAD_RES)

which  effectively  defines the amount of torque the strategy will supply

on demand.  The operating spark which supplies this reserve of torque  is

calculated in the strategy without additional calibration.

The strategy is based on mapping data supplied by the Calibration Optimzation

and Analysis section in PEDD.  The data is entered into FN766 as shown below:

FN766

=====

1.0     |*********

Engine Torque           |         *******

-------------           |                ****

Torque @ MBT            |                    ***

0.7     |________________________

0                       30

Spark Retard from MBT (deg)

The input is the amount (a positive number) spark is retarded from MBT  spark

advance  in degrees.  The output is the ratio of delivered engine torque at a

particular spark advance to the engine torque delivered when  the  engine  is

operating  at  MBT.   In other words, when the engine is operating at MBT the

torque ratio is 1, and when the operating point  is  retarded  from  MBT  the

torque ratio will be a fractional dimensionless value such as .9 or .8.

9-32

IGNITION TIMING STRATEGY, TORQUE BASED FEEDBACK SPARK AT IDLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The strategy also  requires  the  mirror  image  of  this  function.   It  is

calibrated in FN799:

FN799

=====

30      |**********

Spark Retard            |          *******

from MBT (deg)          |                 *****

|                      ***

0      |__________________________

0.7                      1.0

Engine Torque

-------------

Torque @ MBT

The strategy is an implementation of the following calculation:

TORQUE RATIO ENGINE   =     TORQUE RATIO AVAILABLE

- TORQUE RATIO RESERVE

+ TORQUE RATIO CORRECTION

The components are defined as follows:

TORQUE RATIO ENGINE = Torque ratio to be supplied by the engine.

This value is called tr_eng in the strategy and will be converted

to a spark advance by evaluating:  SPK_M_B_T + FN799(tr_eng)

TORQUE RATIO AVAILABLE = This is the ratio of torque available for use.

It is the torque ratio supplied if the engine were to operate at

the absolute spark limit.  It is typically calculated from the

equation:  TR_ABS = FN766(SPK_M_B_T - SPK_ABS_LIM)

TORQUE RATIO RESERVE = This is a reserve of torque expressed as a torque

ratio which is created by retarding spark below MBT.  This torque is

available to the controller to compensate for torque disturbances.

This reserve is essentially the only calibratable part of the

strategy. The desired torque to be held in reserve by operating

retarded from MBT is calibrated through the parameter SPK_LOAD_RES.

TORQUE RATIO CORRECTION = This is the amount of torque expressed as a

torque ratio which reflects a torque disturbance as measured by an

rpm error. This torque called TR_COR in the strategy, compensates

for  the torque disturbance. It is supplied by the engine as a result

of advancing spark.

9-33

IGNITION TIMING STRATEGY, TORQUE BASED FEEDBACK SPARK AT IDLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

These components are shown graphically below:

Engine Torque

Torque Ratio = -------------

Torque @ MBT

Torque Ratio = 1.0     -|--- Torque Ratio at MBT (a value of 1.0 is

|    the maximum possible)

|

|       This is torque which is not available

|       because another calculation such as

|       borderline spark has limitted spark.

|

----  TR_ABS  -|--- TORQUE AVAILABLE

{               |

{               |       TORQUE RESERVE: Reserve of torque set

{               |       aside for load rejection.

{               |

TR_RES  {       tr_eng -|--- Torque Ratio to be delivered by engine.

{               |   } Defined: tr_eng = TR_ABS - TR_RES + TR_COR

{               |   }

{               |   }           TORQUE CORRECTION: Engine torque

{               |   } TR_COR    adjustment via spark in

{               |   }           response to a load disturbance

{               |   }           to the engine as measured by

---------------|---            an rpm error.

|

.

.

.

|

Torque Ratio = 0.0     -|

9-34

IGNITION TIMING STRATEGY, TORQUE BASED FEEDBACK SPARK AT IDLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The relationships of the calculations are shown  below  via  variable  usage.

the entries in the boxes are headings of sections in the strategy.

SPK_M_B_T    SPK_ABS_LIM     LAST_LD_MBT

|         SPK_FLEX_LIM         |

|              |               |

|              |               |

|              |               |

|              |               |

|              |               V

|              |     +-----------------+       +-------+

|              |     | DESIRED RESERVE |       | SPARK |

|              |     |  TORQUE RATIO   |       | STATE |

|              |     +-----------------+       +-------+

|              |             |                     |

|              |             |                SPK_STATE

|              |         tr_res_des                |

|              |             |                     |

|              |             |     +---------------+---+---------------+

|              |             |     |                   |               |

|              |             |     |                   |               |

|              |             |     |                   |               |

|              |             |     |                   |               |

|              |             V     V                   V               |

|              |       +----------------+         +------------+       |

|              +------>| RESERVE TORQUE |         |  TORQUE    |       |

+--------------------->|    RATIO       |         | CORRECTION |       |

|                      +----------------+         +------------+       |

|                              |                         |             |

|                              |                         |             |

|                           TR_RES                    TR_COR           |

|                           TR_ABS                       |             |

|                              |                         |             |

|                              |                         |             |

|                              +--------+     +----------+             |

|                                       |     |                        |

|                                       |     |                        |

|                                       V     V                        |

|                                   +-------------+                    |

+---------------------------------->| IDLE SPARK  |                    |

+-------------+                    |

|                           |

|                           |

SPK_IDLE_FB                      |

|                           |

V                           V

9-35

IGNITION TIMING STRATEGY, TORQUE BASED FEEDBACK SPARK AT IDLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

IDLE SPARK

DEFINITIONS

INPUTS

Registers:

-  SPK_M_B_T = Spark advance required to achieve Maximum Brake Torque.

-  TR_ABS = Torque ratio calculated from SPK_ABS_LIM, unitless.

-  TR_COR = Torque ratio correction required to respond to an rpm dip.  This

term is in direct proportion to an rpm error from DSDRPM, unitless.

-  TR_RES = Torque ratio reserve which is created by retarding  spark  below

MBT.   It is available to the system to be added to compensate for an rpm

error by advancing spark up to a maximum limit, unitless.

Bit Flags

-  ALT_CAL_FLG = Determines when alternate  anti  plug  fouling  calibration

should be used; 1 -> use alternate calibration.

Calibration Constants:

-  FN799(tr_eng) = Transfer function which returns a spark retard  from  MBT

as a function of desired torque ratio, degrees.

-  SPK_FBS_MIN = Most retarded spark allowed when in Feedback Spark.

-  SPK_ALT_MIN = Most retarded spark allowed  when  in  feedback  spark  and

running alternate anti plug fouling calibration.

OUTPUTS

Registers:

-  SPK_IDLE_FB = Spark by Feedback Spark logic.

9-36

IGNITION TIMING STRATEGY, TORQUE BASED FEEDBACK SPARK AT IDLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

Calculate SPK_IDLE_FB:

spk_feedback < SPK_ALT_MIN ------|

|AND -| SPK_IDLE_FB = SPK_ALT_MIN

ALT_CAL_FLG = 1 -----------------|     | spk_source_fb = 8

|

| --- ELSE ---

|

spk_feedback < SPK_FBS_MIN ------------| SPK_IDLE_FB = SPK_FBS_MIN

| spk_source_fb = 8

|

| --- ELSE ---

|

| SPK_IDLE_FB = spk_feedback

| spk_source_fb = 9

spk_feedback = SPK_M_B_T + FN799(tr_eng)

tr_eng = TR_ABS - TR_RES + TR_COR

9-37

IGNITION TIMING STRATEGY, TORQUE BASED FEEDBACK SPARK AT IDLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

RESERVE TORQUE RATIO

DEFINITIONS

INPUTS

Registers:

-  BG_TMR = Background loop time, secs.

-  SPK_ABS_LIM = Spark is never allowed to go above this  value  because  of

detonation, torque limitation or base spark requirements.

-  SPK_FLEX_LIM = This is a desirable spark  limit  which  may  be  exceeded

briefly  when  torque is required by a strategy such as feedback spark or

oscmod.

-  SPK_M_B_T = Spark advance  required  to  achieve  Maximum  Brake  Torque,

degrees.

-  TP = Instantaneous throttle position, counts.

-  TP_LAST = Previous A to D reading of TP (throttle position), counts.

-  TR_RES = Torque ratio reserve which is created by retarding  spark  below

MBT.   It is available to the system to be added to compensate for an rpm

error by advancing spark up to a maximum limit, unitless.

-  tr_res_des = SPK_LOAD_RES expressed as a torque ratio with respect to the

current operating LOAD, unitless.

Calibration Constants:

-  FBS_ENTRY_TC = Time constant which paces the entry into  feedback  spark,

sec.

-  FKEXIT_MIN = Minimum clip on the exit spark filter constant.

-  FN766 = Transfer function which calculates the torque ratio at  which  an

engine is operating from the spark delta from MBT.

-  TARMAX = Minimum value of TAR that will make "fkexit" =  1.0,  degrees  /

sec.

9-38

IGNITION TIMING STRATEGY, TORQUE BASED FEEDBACK SPARK AT IDLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OUTPUTS

Registers:

-  TP_LAST = See above.

-  TR_ABS = Torque ratio calculated from SPK_ABS_LIM, unitless.

-  TR_RES = Torque ratio reserve which is created by retarding  spark  below

It  is available to the system to be added to compensate for an rpm error

by advancing spark up to a maximum limit, unitless.

-  TR_RES_LAST = Last pass value of TR_RES, unitless.

9-39

IGNITION TIMING STRATEGY, TORQUE BASED FEEDBACK SPARK AT IDLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

Upon entry into SPK_STATE 1, TR_RES is ramped exponentially from a  value  of

tr_flex  to a value of tr_res using a rolling average routine.  It remains at

the latter value until the strategy exits via SPK_STATE  3  to  SPK_STATE  0,

when  it decays to a value of tr_flex at a rate based on throttle angle rate:

the greater the throttle rate, the faster the decay.

Torque ratio reserve calculation:

SPK_STATE = 1 -------------------------| TR_RES_LAST = TR_RES

| TR_RES = ROLAV(tr_res,FBS_ENTRY_TC)

|

| --- ELSE ---

|

SPK_STATE = 2 -------------------------| TR_RES_LAST = TR_RES

| TR_RES = tr_res

|

| --- ELSE ---

|

SPK_STATE = 3 -------------------------| TR_RES_LAST = TR_RES

| TR_RES = (1 - fkexit) * TR_RES

|          + fkexit * tr_res_base

| fkexit = spk_tar / TARMAX

| Clip fkexit between

|      FKEXIT_MIN and 1

|

| --- ELSE ---

|

| TR_RES_LAST = TR_RES

| TR_RES = tr_res_base

Calculate the discrete derivative of throttle position.  The factor  of  9.57

is  the  conversion from Analogue to Digital counts into degrees of rotation.

As a result spk_tar has units of degress per second:

|           TP - TP_LAST

TP - TP_LAST > 0 ----------------------| spk_tar = ------------

|           BG_TMR * 9.57

| TP_LAST = TP

|

| --- ELSE ---

|

| spk_tar = 0

| TP_LAST = TP

9-40

IGNITION TIMING STRATEGY, TORQUE BASED FEEDBACK SPARK AT IDLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Select the larger of the existing and desired reserve torque ratios:

tr_res_des > tr_res_base --------------| tr_res = tr_res_des

|

| --- ELSE ---

|

| tr_res = tr_res_base

Torque reserve which is already available due to the difference  between  the

absolute and flexible spark clips:

tr_res_base = TR_ABS - tr_flex

Torque ratio calculated from the absolute and flexible spark clips:

SPK_M_B_T > SPK_ABS_LIM ---------------| TR_ABS = FN766(SPK_M_B_T -

|          SPK_ABS_LIM)

| tr_flex = FN766(SPK_M_B_T -

|           SPK_FLEX_LIM)

|

| --- ELSE ---

|

SPK_M_B_T > SPK_FLEX_LIM --------------| TR_ABS = 1

| tr_flex = FN766(SPK_M_B_T -

|           SPK_FLEX_LIM)

|

| --- ELSE ---

|

| TR_ABS = 1

| tr_flex = 1

9-41

IGNITION TIMING STRATEGY, TORQUE BASED FEEDBACK SPARK AT IDLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DESIRED RESERVE TORQUE RATIO

OVERVIEW

In order to have the ability  to  add  torque  to  compensate  for  a  torque

disturbance,  it  is  necessary to design in a torque reserve which is always

available.  It is desirable to define  this  reserve  as  a  fixed  value  of

torque,  however, torque is not measurable in EEC-IV strategies.  Load can be

used instead of torque because they are directly proportional to each  other.

Torque  or  load can be expressed as equivalent dimensionless ratios referred

to here as a torque ratio.  When a ratio is used many  simplifications  occur

in  the  equations.  The formulation shown allows the calibration of a torque

reserve which remains constant for all speed load combinations.  The  reserve

is created by retarding spark.

To determine the desired amount of torque reserve a procedure is  recommended

where  the  calibrator  operates  the engine at a normal idle noting load and

rpm.  He then applies an external load which is representative  of  the  load

rejection  desired.   The  engine  is allowed to stabilize to the same rpm as

before and the increase in load required to maintain this rpm is  calculated.

This increase in load is calibrated in SPK_LOAD_RES.

The amount of retard required  to  provide  appreciable  load  rejection  may

require  an  unacceptably  large  spark retard at idle.  As a result feedback

spark may be used primarily for stabilizing the idle.

DEFINITIONS

INPUTS

Registers:

-  LAST_LD_MBT = Load from the last pass corrected to the value  load  would

have  to have if spark was at MBT in order for the engine to generate the

same torque output.

Calibration Constants:

-  SPK_LOAD_RES = Desired torque (expressed  as  load)  which  the  feedback

spark  calculation  can  supply to compensate for a torque disturbance to

the engine, unitless.

OUTPUTS

Registers:

-  tr_res_des = SPK_LOAD_RES expressed as a torque ratio with respect to the

current operating LOAD, unitless.

PROCESS

Calculate the desired reserve  torque  ratio  using  load  corrected  to  its

equivalent value at MBT.

tr_res_des = SPK_LOAD_RES / LAST_LD_MBT

9-42

IGNITION TIMING STRATEGY, TORQUE BASED FEEDBACK SPARK AT IDLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TORQUE CORRECTION

DEFINITIONS

INPUTS

Registers:

-  DSDRPM = Idle speed control desired RPM.

-  N = Engine RPM.

-  SPK_STATE = Defines the use of feedback spark; 0 -> feedback spark is not

being  used,  1  ->  transitioning  into  feedback  spark, 2 -> using the

feedback spark calculation, 3 -> transitioning out of feedback spark.

Calibration Constants:

-  FBS_ENTRY_TC = Time constant which paces the entry into  feedback  spark,

sec.

-  SPK_FBS_GAIN = Feedback spark control proportional gain  term.   Normally

calibrated to one in order to exactly compensate for torque disturbances,

unitless.

OUTPUTS

Registers:

-  TR_COR = Torque ratio correction required to respond to an rpm dip.  This

term is in direct proportion to an rpm error from DSDRPM, unitless.

PROCESS

tr_cor = [(DSDRPM - N)/DSDRPM] * SPK_FBS_GAIN

SPK_STATE = 1 -------------------------| TR_COR = ROLAV(tr_cor,FBS_ENTRY_TC)

|

| --- ELSE ---

|

SPK_STATE = 2 -------------------------| TR_COR = tr_cor

|

| --- ELSE ---

|

| TR_COR = 0

9-43

IGNITION TIMING STRATEGY, TORQUE BASED FEEDBACK SPARK AT IDLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SPARK STATE

DEFINITIONS

INPUTS

Registers:

-  APT = At Part Throttle; -1 -> Closed throttle, 0 -> Part throttle,  1  ->

Wide Open throttle.

-  DSDRPM = Idle speed control desired RPM.

-  ISCFLG = ISC mode indicator flag;  -1  ->  Dashpot  Mode,  0  ->  Dashpot

Preposition Mode, 1 -> Closed Loop RPM Control Mode, 2 -> Closed Loop RPM

Control (Lockout entry to RPM control).

-  SPK_STATE = Defines the use of feedback spark; 0 -> feedback spark is not

being  used,  1  ->  transitioning  into  feedback  spark, 2 -> using the

feedback spark calculation, 3 -> transitioning out of feedback spark.

-  TR_RES = Torque ratio reserve which is created by retarding  spark  below

MBT.   It is available to the system to be added to compensate for an rpm

error by advancing spark up to a maximum limit, unitless.

-  TR_RES_LAST = Last pass value of TR_RES, unitless.

-  VSBAR = Filtered vehicle speed.

Calibration Constants:

-  DRBASE = Base desired engine speed in drive.

-  FBS_ENTRY_TC = Time constant which paces the entry into  feedback  spark,

sec.

-  MINMPH = Minimum speed to enter C/L  RPM  control.   Applies  to  systems

having  VSS.   Should be set below the speed at which an automatic trans.

vehicle rolls along in drive without the  brakes.   This  is  to  prevent

going  into  RPM control during parking lot maneuvers.  Typical value - 3

MPH.

-  NUBASE = Base desired engine speed in neutral.

-  SPKCTL = Threshold above base DSDRPM, below which spark feedback  can  be

entered.

-  TR_DELTA = Deviation from TR_RES which controls entry into feedback spark

state 2, unitless.

Bit Flags:

-  DNDSUP = Delayed Neutral/Drive flag; 0 -> neutral, 1 -> drive.

9-44

IGNITION TIMING STRATEGY, TORQUE BASED FEEDBACK SPARK AT IDLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OUTPUTS

Registers:

-  SPKTMR = Timer used to pace the transition into feedback spark.

-  SPK_STATE = Defines the use of feedback spark, 0 -> feedback spark is not

being  used,  1  ->  transitioning  into  feedback  spark, 2 -> using the

feedback spark calculation, 3 -> transitioning out of feedback spark.

PROCESS

Spark state transition logic:

SPK_STATE = 0 -------------------|

(current state is 0)            |AND -| SPK_STATE = 1

|     |  (transition to idle spark)

fbs_entry_ok = 1 ----------------|     | SPKTMR = 0

|

| --- ELSE ---

SPK_STATE = 1 -------------------|     |

(current state is 1)            |AND -| SPK_STATE = 3

|     |  (exit from idle spark)

fbs_exit_ok = 1 -----------------|     | SPKTMR is running

|

| --- ELSE ---

SPK_STATE = 1 -------------------|     |

(current state is 1)          |AND -| SPK_STATE = 2

|     |  (start doing feedback spark)

fbs_entry_comp = 1 --------------|     | SPKTMR is running

|

| --- ELSE ---

SPK_STATE = 2 -------------------|     |

(current state is 2)          |AND -| SPK_STATE = 3

|     |  (exit from idle spark)

fbs_exit_ok = 1 -----------------|     | SPKTMR is running

|

| --- ELSE ---

SPK_STATE = 3 -------------------|     |

|AND -| SPK_STATE = 1

fbs_entry_ok = 1 ----------------|     |  (transition back into state 1)

(see entry logic)                     | SPKTMR = 0

|

| --- ELSE ---

SPK_STATE = 3 -------------------|     |

|AND -| SPK_STATE = 0

fbs_exit_comp = 1 ---------------|     |  (transition into state 0)

(see exit complete logic)             | SPKTMR is running

|

| --- ELSE ---

|

| No change to SPK_STATE

| SPKTMR is running

9-45

IGNITION TIMING STRATEGY, TORQUE BASED FEEDBACK SPARK AT IDLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Entry conditions to spark state 1.   Begin  transition  into  feedback  spark

calculation.

ISCFLG >= 1 ---------------------|

(rpm control or lockout)        |

|

DNDSUP = 1 ----------|           |AND -| fbs_entry_ok = 1

|AND -|     |     |

DSDRPM - DRBASE      |     |     |     |

<= SPKCTL ---|     |     |     |

(near idle rpm)           |OR --|     |

|           |

DNDSUP = 0 ----------|     |           |

|AND -|           |

DSDRPM - NUBASE      |                 |

<= SPKCTL ---|                 |

| --- ELSE ---

|

| fbs_entry_ok = 0

Entry conditions to spark state 3.  Begin transition out  of  feedback  spark

calculations.

APT >= 0 ------------------------|

(tip in)                        |OR --| fbs_exit_ok = 1

|     |

VSBAR > MINMPH ------------------|     |

(vehicle moving)                    |

| --- ELSE ---

|

| fbs_exit_ok = 0

Entry conditions to spark state 2.  Entry transition  to  feedback  spark  is

complete,  begin  normal feedback spark calculation.  Note:  This calculation

uses last pass values for TR_RES and TR_RES_LAST.

SPKTMR > FBS_ENTRY_TC * 2 -------|

|OR --| fbs_entry_comp = 1

|TR_RES_LAST - TR_RES|           |     |

< TR_DELTA ------|     |

| --- ELSE ---

|

| fbs_entry_comp = 0

Entry conditions to spark state 0.  Exit transition from  feedback  spark  is

complete, do not use the feedback spark calculation.  Note:  This calculation

uses last pass values for TR_RES and TR_RES_LAST.

|TR_RES_LAST - TR_RES| < TR_DELTA -----| fbs_exit_comp = 1

|

| --- ELSE ---

|

| fbs_exit_comp = 0

9-46

IGNITION TIMING STRATEGY, OSCILLATION MODULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

9.10  OSCILLATION MODULATION (CDAA0)

OVERVIEW

OSCMOD stands for oscillation modulation.  Engine  torque  is  modulated  via

spark  advance  as  a  function  of  the  rate  of  change of rpm in order to

stabilize oscillations in rpm.  This  is  a  derivative  control  method  and

functions as a dashpot.  This correction may be useful during closed throttle

or negative crowd decelerations to control shuffle.   It  can  also  be  used

without  interference  with  feedback  spark.   The  areas  and  magnitude of

correction are controlled by non-zero values in FN2020.

DEFINITIONS

INPUTS

Registers:

-  DNDT_SPK = Filtered rate of change of  RPM  for  OSCMOD  spark,  RPM/S*S.

This  parameter  is  calculated  at the start of the spark routine which,

means a recent value of DNDT is used.

-  LOAD = Universal load variable.  Normalized CYLARC_BG divided by  SARCHG,

unitless.

-  N = Engine speed in revolution per minute, RPM.

-  NOVS = Ratio of engine speed to vehicle speed used to infer  transmission

gear, rpm/mph.

-  SPK_OSC_OFF = The amount by which the base spark calculation  is  reduced

to make room for an increase as calculated by the OSCMOD strategy.

Calibration Constants:

-  FN013(LOAD) = Normalized LOAD for half-sized spark tables.

-  FN017(N) = Normalized RPM for half-sized spark tables.

-  FN745(DNDT_SPK) = Adds spark in order to add torque when  engine  rpm  is

dropping  and  takes  spark  away  when rpm is increasing.  A value of -1

returns all the spark taken out with FN2020.

-  FN746(NOVS) =  Engine  torque  transmission  is  lower  in  higher  gears

(calibrate  to  a  value  less than one) reducing the need for the OSCMOD

relative to first gear (calibrate to one), unitless.

-  FN2020 = Reduces base spark in order to reduce torque so that there is  a

reserve  of  torque  available  to  be added back in as determined by the

SPK_OSCMOD calculation.

X = FN017(N)

Y = FN013(LOAD)

9-47

IGNITION TIMING STRATEGY, OSCILLATION MODULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OUTPUTS

Registers:

-  SPK_OSC_OFF = See above.

-  SPK_OSCMOD = OSCMOD correction to other spark calculations.

PROCESS

STRATEGY MODULE: SPARK_OSCMOD_COM1

SPK_OSCMOD = SPK_OSC_OFF * FN745(DNDT_SPK)

SPK_OSC_OFF = FN2020(N,LOAD) * FN746(NOVS)

9-48

IGNITION TIMING STRATEGY, TORQUE MODULATION BACKGROUND CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

9.11  TORQUE MODULATION BACKGROUND CALCULATION (CDAK0)

OVERVIEW

The transmission strategy calculates the maximum  torque  required  during  a

shift  (TQ_LEVEL).   The  ratio between TQ_LEVEL and TQ_MBT is calculated and

used in FN799 to calculate the amount spark must  be  retarded  from  MBT  in

order  to  achieve the required torque.  The actual spark advance required to

give the required torque (SAF_DES) is then  calculated  by  subtracting  that

amount from the MBT spark advance.

Maximum spark  advace  is  clipped  based  on  transmission  oil  temperature

(FLG_TQM_ENA  ).   In  addition, there is a unconditional maximum clip on the

spark retard requested .

Spark is ramped at the start and end of torque modulation.  At the  start  of

toruqe modulation, a slope (SPKSLP) for the spark ramp is calculated based on

the differential amount of spark retard requested  (SAFTOT  -  SPK_DELT_OUT).

and  the  time  calibrated  (SPK_TM_OUT).   At the end of the shift, spark is

stepped back in via discrete steps (SPK_DELT_IN) until spark is  returned  to

its normal state (SAFTOT = SAF_DES) or the ramp timer expires (SPK_TM_IN).

DEFINITIONS

INPUTS Registers:

-  GR_CM = Commanded gear for shift solenoids.

Bit Flags:

-  FLG_SPK_TQM = Torque modulation spark retard flag.

-  FLG_TQM_ENA = Flag to signal transmission is sufficiently  warmed  up  to

permit full control over spark retard via torque modulation.

-  FLG_UP_TQM = Upshift torque modulation flag.

Calibration Constants:

-  FN799 = Transfer function which returns a spark  retard  from  MBT  as  a

function of desired torque ratio, deg.

-  SPKDELT = Delta spark stepped in at end  of  upshift  torque  modulation,

deg.

-  SPKDELT2  =  Delta  spark  stepped  in  at  end  of  1-2  upshift  torque

modulation, deg.

-  SPKTM2_IN = Time to ramp spark in  for  1-2  upshift  toruqe  modulation,

msec.

-  SPKTM2_OUT = Time to ramp spark out for 1-2  upsfhit  torque  modulation,

msec.

9-49

IGNITION TIMING STRATEGY, TORQUE MODULATION BACKGROUND CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  SPKTM_IN = Time to ramp spark in for upshift torque modulation, msec.

-  SPKTM_OUT = Time to ramp spark out for upshift torque modulation, msec.

-  TQM_COLD_CLP = Maximum delta  spark  clip  for  torque  modulation;  cold

powertrain, deg.

-  TQM_MAX_CLP = Maximum delta spark clip for torque modulation, deg.

OUTPUTS

Registers:

-  RAMP_TM_IN = Default time for spark ramp at  end  of  torque  modulation,

msec.

-  RAMP_TM_OUT = Default time for spark ramp  start  of  torque  modulation,

msec.

-  SAF_DES = Spark advance required to  achieve  the  desired  torque  level

during torque modulation, deg.

-  SAFTOT = Total spark advance, deg.

-  SPK_M_B_T = Spark advance required to achieve maximum brake torque, deg.

-  SPKSLP = Spark ramp rate for torque modulation, deg/msec.

-  TQ_MBT = Maximum net torque that could be produced by the engine, ft-lbs.

-  TQ_LEVEL = Maximum torque required by transmission for torque modulation,

ft-lb.

9-50

IGNITION TIMING STRATEGY, TORQUE MODULATION BACKGROUND CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SPARK_TQM_BG_COM5

always --------------------------| tr_mod = TQ_LEVEL / TQ_MBT

| delta_mod = FN799(tr_mod)

FLG_TQM_ENA = 0 -----------|

|AND -| SAF_DES = SPK_M_B_T + delta_mod

delta_mod > TQM_COLD_CLP --|     |

| --- ELSE ---

|

FLG_TQM_ENA = 0 -----------------| SAF_DES = SPK_M_B_T + TQM_COLD_CLP

| (clip spark if powertrain is cold)

|

| --- ELSE ---

FLG_TQM_ENA = 1 -----------|     |

|AND -| SAF_DES = SPK_M_B_T + delta_mod

delta_mod > TQM_MAX_CLP ---|     |

| --- ELSE ---

|

FLG_TQM_ENA = 1 -----------------| SAF_DES = SPK_M_B_T + TQM_MAX_CLP

9-51

IGNITION TIMING STRATEGY, TORQUE MODULATION BACKGROUND CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

GR_CM = 2 -----------------|

|

FLG_UP_TQM = 1 ------------|AND -| SPKSLP = (SAFTOT - SAF_DES)/

|     |           SPKTM2_OUT

FLG_SPK_TQM = 0 -----------|     | (If SPKTM2_OUT = 0, set SPKSLP

|  to its max value)

| SPK_DELT_IN = SPKDELT2

| RAMP_TM_IN = SPKTM2_IN

| RAMP_TM_OUT = SPKTM2_OUT

|

| --- ELSE ---

FLG_UP_TQM = 1 ------------|     |

|AND -| SPKSLP = (SAFTOT - SAF_DES)/

FLG_SPK_TQM = 0 -----------|     |           SPKTM_OUT

| (If SPKTM_OUT = 0, set SPKSLP

|  to its max value)

| SPK_DELT_IN = SPKDELT

| RAMP_TM_IN = SPKTM_IN

| RAMP_TM_OUT = SPKTM_OUT

9-52

IGNITION TIMING STRATEGY, KNOCK CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

9.12  KNOCK CONTROL STRATEGY (CDAN0)

OVERVIEW

The knock control strategy consists of many  different  sub-strategy  pieces.

Some  are  executed in background, but most are executed in foreground, since

knock is a PIP period related event.  The sub-strategies are:

1)  KNOCK STRATEGY ENABLE LOGIC (background) This  logic  determines  whether

engine conditions are correct for using the knock strategy.

2)  RETARD AND WINDOW CALCULATIONS (background)  This  logic  determines  the

knock  window  position  and  window  width  as  well as potential retard

increment in background.  This is done to ease foreground load.

3)  TIP IN SENSE LOGIC (background) This logic anticipates tip in  detonation

by sensing rapid tip-ins from closed throttle and setting a flag which is

used by the foreground to retard the spark.

4)  KNOCK WINDOW OUTPUT (PIP  rising  edge)  This  logic  outputs  the  knock

window,  KTS  output,  at  the background determined location relative to

PIP.

5)  KNOCK DETECTION LOGIC (PIP  rising  edge)  This  logic  determines  if  a

cylinder is knocking by processing the knock indicated input.

6)  TIP IN RETARD LOGIC (TIPRET) (PIP rising edge)  This  logic  retards  and

advances spark based on tip in conditions.

7)  KNOCK RETARD LOGIC (PIP rising edge) This logic retards  the  SPKAD  term

based on knock sensed by the knock detection logic.

8)  SPARK ADVANCE LOGIC (background) This logic advances the SPKAD  terms  to

the advance limit (knk_advlim) at a calibratable rate.

The logic segments that follow match the order of  software  execution.   The

numbers point to the above descriptions.

DEFINITIONS

Registers:

-  ACT = Air charge temperature deg.  faren.

-  DT12S = Period of time between two adjacent rising edges of PIP, ticks.

-  ECT = Engine coolant temperature, deg F.

-  ER_LAM_DSD = LAMBSE value requested  by  engine  running  in-demand  self

test.

-  ER_SPARK = Required value of SAF during on demand self test.

9-53

IGNITION TIMING STRATEGY, KNOCK CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  ER_STATUS = State pointer that indicated current state of engine  running

on-demand test.

-  KNK_ADVLIM_D = DISPLAY ONLY; knock advance limiter from FN1135.

-  KWCTR = Cancel window counter incorporated each PIP period.

-  LOAD = Universal normalized load parameter, unitless.

-  N = Engine speed, RPM.

-  RETINC = Calculated as a function of RPM  and  is  subtracted  from  each

SPKAD corresponding to a knocking cylinder.  (positive degree)

-  RETMAX = Knock retard limit, degrees.

-  SAFTOT = Total spark advance.

-  SPKAD = Spark adder term.  It  is  added  to  SAF,  may  be  positive  or

negative degrees.

-  SPK_MBT_DELT = Difference between MBT spark and SAFTOT.

-  SPK_TIPMAX = Initial amount of retard following a  Tip-in.   (Must  be  a

negative number; units are degrees)

-  TBART = Average Filtered Throttle Position = ROLAV (TP,TCTPT)

-  TCF = Value of the difference between Throttle position (TP) and TBART.

-  TIPRET = Tip-in retard.

-  TIP_COUNT = Number of PIP cycles that have  elapsed  following  a  tip-in

retard.

-  TP = Instantaneous throttle position, counts.

-  TSLADV = Free-running millisecond timer which counts the time  since  the

spark was last advanced by the KNOCK Strategy.

-  WINDOW_BETA = Fraction of PIP period at which to issue knock window.

-  WINDOW_DELTA = Knock window width, clock ticks.

Bit Flags:

-  CTFLG = 1 -> closed throttle Tip-in.

-  GOOS_KNK_DET = Knock detected during goose test.

-  KNK_HIGH = Knock input level.

-  KNK_INT = Knock data available when set.

-  KNOCK_DETECTED = 1 -> Knock occured in current PIP half period.

-  KNOCK_ENABLED = Knock Strategy enabled.

9-54

IGNITION TIMING STRATEGY, KNOCK CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  KNOCK_OCCURRED = 1 -> (in the knock routine) knock occured in the current

or last PIP period.

-  NEW_PIP = New PIP rising edge occurred this background loop.

-  PIP_HIGH = PIP input level.

-  PxxxMALF = Malfunction flag for code PXXX; 1 -> a  malfunction  currently

exists for fault PXXX.

-  SYNFLG = SYNCTR in SYNC with SIG PIP.

-  TIPFLG = 1 -> Tip-in.

-  VIP_KNOCK = Knock was detected in VIP mode when set.

Calibration Constants:

-  ECTIP = Minimum ECT to enable Tip-in Retard, deg F.

-  ECTNOK = Disable Knock control below this value of ECT, deg F.

-  FN013(LOAD) = Load normalizing function; used for table lookup.

Input = Load

Output = Normalized load

-  FN013A(LOAD) = Load normalizing function; used for table lookup.

Input = Load

Output = Normalized load

-  FN017(N) = Engine speed normalizing function; used for table lookup.

Input = N (rpm)

Output = Normalized N (rpm)

-  FN017C(N) = Engine speed normalizing function; used for table lookup.

Input = N

Output = Normalized N

-  FN030A(ACT) = KTS window multiplier based on air charge temperature.

Input = ACT

Output = KTS window multiplier.

-  FN033A(ACT) = Air charge temperature normalizing function; used for table

lookup.

Input = ACT

Output = Normalized ACT

-  FN070(N) = Engine speed Normalizing function; used for table lookup.

Input = N

Output = Normalized N

-  FN070F(N) = Engine speed normalizing function; used for table lookup  for

FN1144C(N,LOAD).

Input = N

Output = Normalized N

-  FN070O(N)     =     Engine     speed     normalizing     function     for

FN1147B(N,SPK_MBT_DELT).

Input = N

9-55

IGNITION TIMING STRATEGY, KNOCK CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Output = Normalized N

-  FN071(LOAD) = Load normalizing function; used for table lookup.

Input = LOAD

Output = Normalized load

-  FN071A(LOAD) = Load Normalizing function; used for table FN1144C.

Input = LOAD

Output = Normalized load

-  FN097B(SPK_MBT_DELT) = Normalizing function for retard from MBT.

Input = SPK_MBT_DELT

Ouput = Normalized SPK_MBT_DELT

-  FN143A(N) = Retard increment versus RPM, deg.

-  FN145A(N) = Variable knock threshold window  position.   Input  =  engine

Speed in RPM Output = fraction of PIP period.

-  FN146B(N) = Spark advance Rate versus RPM, sec/deg.

-  FN1132(ACT,LOAD) = Knock retard limit adder, degrees.

X = FN033A(ACT)

Y = FN013A(LOAD)

-  FN1134(N,LOAD) = Nominal knock retard limit based  on  engine  speed  and

load, degrees.

X = FN017C(N)

Y = FN013A(LOAD)

-  FN1135(N,LOAD) = Knock advance limit table as a function of RPM and LOAD,

deg.   If  FN1135 is calibrated to 0, the knock strategy will not be able

to advance spark past SAF.   FN1135  is  an  8  X  10  table  and  shares

normalizing functions with all the other spark tables.

X = FN070(N)

Y = FN071(LOAD)

FN1135(N,LOAD) is stored as KNK_ADVLIM_D for ease of development.

-  FN1144C(N,LOAD) = Variable knock threshold window open time, fraction  of

PIP period.

X = FN070F(N)

Y = FN071A(LOAD)

-  FN1147B(N,SPK_MBT_DELT) = KTS window modifier based on retard from MBT.

X = FN070O(N)

Y = FN097B(SPK_MBT_DELT)

-  FN2290(N,LOAD) = Initial retard angle following a tip-in.

X = FN017(N)

Y = FN013(LOAD)

-  GOOSE_KTS_B = Knock window width  during  goose  test,  fraction  of  PIP

period.

-  GOOSE_LAM = Required value of LAMBSE during goose test, unitless.

-  GOOS_ER_CODE =  Value  of  ER_STATUS  to  indicate  that  goose  response

detected.

9-56

IGNITION TIMING STRATEGY, KNOCK CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  GOOS_ER_INIT = Value for state of ER_STATUS to initialize the goose test.

-  GOOS_ER_TEST = Value for state of ER_STATUS to require goose test  to  be

run.

-  GOOS_SPK_INT = Spark advance used during goose test to induce knock, deg.

-  GOOS_SPK_TST = Spark advance used during goose test once knock  has  been

detected, deg.

-  KACRAT = Change in TP equivalent to a Tip-in Retard, counts.

-  KIHP = Knock Hardware Present Switch; 1 -> Knock sensor present.

-  LODNOK = Minimum load for knock control, unitless.

-  NTIP = Maximum RPM to enable Tip-in retard (restricted to  less  than  or

equal to 2000 to assure TIPRET ramp back function.)

-  RPMCNL = Threshold RPM below which the window is always open.

-  RPMMIN = Disable Knock control below this RPM.

-  RPMMAX = SPKAD is not updated above this RPM.

-  TCTPT = Time constant for TBART (TP filter).

-  TIPINC = Advance per PIP following a Tip-in retard.  (Must be a  positive

number; units are degrees)

-  TIPLOD_SL = Maximum Load to set CTFLG for Tip-in Retard.

-  TIPLOD_CH = Minimum Load to clear CTFLG for Tip-in Retard.

-  TIP_RMP_DLY = Number of PIP cycles to delay ramping back spark  following

a tip-in retard.

-  WINCLD = Number of PIPs threshold window is to be closed.

-  WINLEN_B = Minimum amount of time threshold window is open,  fraction  of

PIP period.

-  WOPEN = Position of window opening, fraction of PIP period.

OTHER

-  spark_codes = set of {P0325}.

-  P0325 = Fault code, for knock sensor failure.

9-57

IGNITION TIMING STRATEGY, KNOCK CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SPARK_CYL_KNOCK_COM1

Background module KNK_WINDOW_CALCULATION

(called from background manager)

RETARD AND WINDOW CALCULATIONS

NEW_PIP = 1 -----------------------------| WINDOW_BETA = WOPEN + FN145A(N)

(new PIP came in this                   | (calculate window opening as a

background loop)                       |  fraction of PIP period)

| SPK_MBT_DELT := SPK_M_B_T - SAFTOT

| WINDOW_DELTA :=

|     [ WINLEN_B

|     + FN1144C(N,LOAD)

|     - FN1147B(N,SPK_MBT_DELT) ]

|     * FN030A(ACT)

|     * DT12S

|

| (calculate window width in

|  clock ticks)

| RETINC = FN143A(N)

| (determine retard amount in case

|  of knock)

| RETMAX = FN1134(N,LOAD)

|        + FN1132(ACT,LOAD)

| (maximum retard allowed)

ER_STATUS = GOOS_ER_INIT ----------------| WINDOW_DELTA =

|   GOOSE_KTS_B * DT12S

|  (Window width used during

|   goose test)

| ER_SPARK = GOOS_SPK_INT

|  (Set initial spark advance

|   for goose test)

| ER_LAM_DSD = GOOSE_LAM

|  (set lambse for goose test)

| GOOS_KNK_DET = 0

|  (Clear knock detected during

|   goose test flag)

|

| --- ELSE ---

|

ER_STATUS = GOOS_ER_TEST ----------------| WINDOW_DELTA =

|   GOOSE_KTS_B * DT12S

| (Window width used during

|  goose test)

|

| --- ELSE ---

ER_STATUS = GOOS_ER_CODE ----------|     |

|AND -| store_code(P0325)

GOOS_KNK_DET = 0 ------------------|     | P0325MALF = 1

9-58

IGNITION TIMING STRATEGY, KNOCK CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

8)

SPARK ADVANCE LOGIC

NEW_PIP = 1 -----------------------|

|

KNOCK_ENABLED = 1 -----------------|

|AND -| SPKAD = SPKAD + 0.25 deg

TSLADV >= FN146B/4 ----------------|     | clip SPKAD to

|     |  FN1135(N,LOAD) as a maximum

N < RPMMAX ------------------------|     | TSLADV = TSLADV - FN146B/4

NOTE: FN1135 is stored as KNK_ADVLIM_D for ease of development.

If the Knock Strategy is enabled and engine speed is less  than  RPMMAX,  the

spark  adder SPKAD is incremented 0.25 degrees every FN146B/4 seconds.  SPKAD

is clipped to FN1135(N,LOAD) as a maximum.  If FN1135 = 0, the KNOCK STRATEGY

will not advance the spark beyond SAF.

NOTE:  If the Knock Strategy is enabled and no cylinders  are  knocking,  the

spark  to each cylinder will advance to SAF + FN1135 If a particular cylinder

is knocking,  the  Retard  Strategy  will  tend  to  dominate  the  advancing

mechanism.   To  insure that the spark to knocking cylinders is retarded more

than the strategy can advance it, FN146B should be greater than or  equal  to

1/FN143A.   FN146B  is  in  degrees  of  advance per second while FN143 is in

degrees of retard per PIP, therefore, FN143 must be converted to degrees  per

second  at  a  given  RPM  for a valid comparison to be made.  When FN146B is

large, then the spark advance rate is small.  For example, FN146B  =  0.5  is

equivalent  to  a  spark  advance  rate  of  2 degrees/sec.  FN146B = 0.25 is

equivalent to spark advance rate of 4 degrees/sec.

9-59

IGNITION TIMING STRATEGY, KNOCK CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

3)

TIP IN SENSE LOGIC

This logic responds to  Tip-in  detonation,  and  even  to  potential  Tip-in

detonation, by retarding the spark TIPRET degrees.

Tip-in detonation is a result of the relatively slow response  of  both  LOAD

and  N,  which are average values during a PIP period, to the sudden increase

in manifold pressure and decrease in engine speed, respectively, which  occur

within  a PIP period during a Tip-in.  The result is that the delivered spark

is over-advanced for the instantaneous conditions until the LOAD  calculation

has  updated to reflect the higher manifold pressure and the engine speed has

recovered.  The recovery from a Tip-in is normally complete within a few  PIP

periods.

The KNOCK STRATEGY is designed to anticipate detonation  following  a  Tip-in

from  idle  (the  worst  case  Tip-in condition) and respond by retarding the

spark before detonation  occurs.   Tip-in  from  part-  throttle  results  in

retarded  spark  only  if  knock  is sensed.  In both cases, Tip-in retard is

applied to whichever cylinders follow the Tip-in, not to individual cylinders

as is usually done in the individual cylinder knock strategy.  Thus, there is

no  need  to  wait  an  entire  engine  cycle  before  responding  to  Tip-in

detonation.

The Tip-in condition is recognized by comparing TP to a filtered  TP,  called

TBART.   (NOTE  that TBART is initialized to the same initialization value as

RATCH) If TCF, the difference between TP and TBART, exceeds  KACRAT,  and  if

either  the  Tip-in  occurred from idle or if the knock is sensed following a

Tip-in from part-throttle, then the spark for the next  PIP  is  retarded  by

TIPMAX  degrees.  On the ensuing PIPS, the amount of retard is decremented by

TIPINC degrees until all Tip-in retard is removed.  The Tip-in logic  can  be

disabled by setting KACRAT = 1023.

always ----------------------------------| TBART = ROLAV(TP,TCTPT)

| (filter TP for rate calculation)

| TCF = TP - TBART

LOAD < TIPLOD_SL ------------------|S Q -| CTFLG

|

LOAD > TIPLOD_CH ------------------|C

ECT > ECTIP -----------------------|

|

TCF >= KACRAT ---------------------|

|AND -| TIPFLG = 1

N < NTIP --------------------------|     |

|     |

CTFLG = 1 -------------------------|     |

(last pass value)                       |

| --- ELSE ---

|

| TIPFLG = 0

9-60

IGNITION TIMING STRATEGY, KNOCK CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Logic to set amount of tip-in retard based on engine speed and load.  This is

done  to  optimize tip-in knock control and engine power at all engine speeds

and loads so that the worst-case retard angle does not have to  be  used  all

the time.

always -----------------| SPK_TIPMAX = FN2290(N,LOAD)

1)

KNOCK STRATEGY ENABLE LOGIC

KIHP = 1 --------------------------|

|

LOAD > LODNOK ---------------------|

|

ECT > ECTNOK ----------------------|

|

N > RPMMIN ------------------------|AND -| KNOCK_ENABLED = 1

|     |

SYNFLG = 1 ------------------------|     |

| --- ELSE ---

|

| KNOCK_ENABLED = 0

| TSLADV = 0

| SPKAD = 0

LODNOK, ECTNOK, and RPMMIN define the minimum engine operating conditions  to

enable the Knock Control Strategy.

9-61

IGNITION TIMING STRATEGY, KNOCK CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

4)

KNOCK WINDOW OUTPUT (KTS)

The software periodically opens a window which allows a Noise threshold

charging pulse called KTS to raise the Knock Threshold level of the Hardware

circuit. The window always opens once per PIP period unless the RPM exceeds

RPMCNL. The engine developer defines the window during which the charging

pulse is on by means of two fox functions and two calibration constants. The

pulsewidth of KTS defines the period of time that the capacitor in the RC

circuit will be charged. Wide KTS pulses cause the threshold to increase. The

timing of the KTS pulse must coincide with the optimum non-knocking portion

of the PIP period over all engine RPM. Since Knock tends to to extend longer

through the PIP period with increasing RPM, the KTS pulse should be timed

late in the current PIP period or early in the following PIP period (95-110%

PIP period).

Noise threshold elevation will result when the capacitor charging rate

greatly exceeds the discharge rate or when the KTS pulse is output during

conditions of Knock. When knock occurs at high RPM, the charging pulse window

is kept closed for WINCLD PIP periods to prevent elevating the NOISE

threshold to the level of KNOCK, thereby preventing the EEC hardware circuit

from sensing additional spark knock.

The calculations shown below are checked every rising edge of PIP:

The pulsewidth of KTS is equal to:

WINDOW_DELTA = [WINLEN_B + FN1144C(N,LOAD) - FN1147B(N,SPK_MBT_DELT)] *

FN030A(ACT) * DT12S in clock ticks

Where, WINLEN_B is minimum KTS pulsewidth, as a fraction of PIP period,

FN1144C(N,LOAD) is expressed as a fraction of PIP period.

The timing of KTS is equal to:

WINDOW_BETA = WOPEN + FN145A(N), fraction of PIP period

Where, WOPEN is the minimum delay after the rising edge of PIP before the

KTS pulse will be output,

FN145A(N) is a fraction of PIP period.

Note:  The KTS pulse is output even if the  knock  strategy  is  disabled  to

refresh  the  threshold  level  in  the event that the Knock strategy becomes

enabled.  The absence of the KTS pulse for more than a few PIP periods  would

result in full retard upon entering Knock strategy.

9-62

IGNITION TIMING STRATEGY, KNOCK CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

5)

KNOCK DETECTION LOGIC

Foreground module SINGLE_KNOCK (EOS_WINDOW_OUTPUT)

KIHP = 0 --------------------------------| NO KNOCK SENSOR

| Do not output KTS

|

| --- ELSE ---

N <= RPMCNL -----------------------|     |

|     |

KWCTR >= WINCLD -------------------|OR --| KWCTR = 0

|     | OPEN WINDOW AT CALCULATED TIME

KWCTR = 0 -------------------|     |     |

|AND -|     |

KNOCK_DETECTED = 0 ----------|           |

(no KNOCK in current PIP half period)   |

| --- ELSE ---

|

| INCREMENT KWCTR

| DO NOT OPEN WINDOW

RPMCNL is the threshold RPM below which  the  WINDOW  is  always  open.   The

WINDOW  is  not  opened  during  a  signature  PIP  period  (if Signature PIP

distributor is present), or if KNOCK has been detected during the current PIP

first half period.

Foreground segment located in PIP_DATA.

KNOCK_DETECTED = 1 ----------------|

|AND -| KNOCK_OCCURRED = 1

KNK_HIGH = 1 ----------------------|     |

| --- ELSE ---

KNOCK_DETECTED = 1 ----------------|     |

|AND -| KNOCK_OCCURRED = 1

KNK_HIGH = 0 ----------------------|     | KNOCK_DETECTED = 0

(KI currently indicates                 |

NO KNOCK)                              |

| --- ELSE ---

|

| KNOCK_OCCURRED = 0

Foreground module SINGLE_KNOCK (EOS_INPUT)

KIHP = 0 --------------------------------| Prevent knock interrupts

| from being recognized

|

| --- ELSE ---

|

KNK_INT = 1 -----------------------------| Service knock input handler

(Knock input transition occurred)       | KNOCK_DETECTED = 1

| VIP_KNOCK = 1

| KNK_INT = 0

9-63

IGNITION TIMING STRATEGY, KNOCK CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

KNOCK SIGNAL DETECTION

:                                       :

:............. PIP PERIOD ..............:

:                                       :

---------------------                   ----------

|                   |                   |

PIP     |                   |                   |

|                   |                   |

------                   ---------------------

:                                        :

:<-- WINDOW_BETA = WOPEN + FN145A(N) --> :

:                                        :

------                                  ------

|    |                                  |    |

KTS  --->|    |<--- WINDOW_DELTA =               |    |

|    |      [WINLEN_B + FN1144C]*DT12S  |    |

----------    ------------------------------------    ----

:    :                                  :    :

:    :                                  :    :

:    :                                  :    :

:    :                **                :    :

:    :               *  *               :    :

:    :              *    *              :    :

:    :             *      *             :    :

:    :-------     *        *            :    :---

NOISE    :   /:       ----*--        *           :   /:

-        :  / :          *   -------  *          :  / :

----    : /  :         * :         ---*---      : /  :

-------/   :        *  :             *  -------/   :

:    :       *   :            : *       :    :

:    :      *    :            :  *      :    :

:    :     *     :            :   *     :    :

KNOCK    :    :    *      :            :    *    :    :

SENSOR   :    :   *       :            :     *   :    :

*************        :            :      ************

:            :

:            :

:            :

:            :

--------------

|            |

KI                        |            |

|            |

-----------------------            -------------------

(IF KNOCK > NOISE, KI=1; OTHERWISE, KI=0)

9-64

IGNITION TIMING STRATEGY, KNOCK CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|<--"LAST PIP PERIOD"-->|

_____________           _____________           ____

|           |           |           |           |

|           |           |           |           |

|           |           |           |           |

PIP  ___|           |___________|           |___________|

|<-----------A------->|

_____                   _____

|   |                   |   |

|<B>|         |         | | |         |           |

|   |                   |   |

KTS  _|   |___________________|   |__________________________

___________

|         |

|     |     |   |       |           |           |

|         |

KI   ---------|         |------------------------------------

___________________

|                 |

|                 |

|     |     |           |           |

KNOCK ________|                 |_______________________|____

DETECTED

_________________________

|                       |

|           |           |           |           |

|                       |

KNOCK __________________________|                       |_____

OCCURRED

Where A = (WOPEN + FN145A)*("LAST PIP PERIOD")

B = [WINLEN_B + FN1144C]*DT12S = KTS

Note: Range of A is typically 90 - 110 % of PIP period.

9-65

IGNITION TIMING STRATEGY, KNOCK CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

6)

TIP IN RETARD LOGIC (TIPRET)

Foreground segment in HDR_SPOUT_KNOCK_CALCULATION

The following Tip-in logic is checked before SPOUT issues (rising or falling  edge

of  PIP).   Note  that  the tip-in retard spark value is based on engine speed and

load and is calculated in the background tip-in sense logic.

This logic includes a delay before spark is ramped back following a tip-in retard.

The logic waits TIP_RMP_DELAY PIP cycles before performing the ramp.

TIPRET = 0 ------------------------|

|AND -| TIPRET := SPK_TIPMAX

TIPFLG = 1 ------------------------|     | TBART := TP

| TIP_COUNT := 0

|

| --- ELSE ---

|

TIP_COUNT >= TIP_RMP_DLY ----------|     |

|     |

TIPRET <> 0 -----------------------|AND -| TIPRET := TIPRET + TIPINC

|     |  ; Clip max TIPRET to 0

PIP_HIGH = 1 ----------------------|     |

|

| --- ELSE ---

|

TIPRET <> 0 -----------------------|     |

|AND -| TIP_COUNT := TIP_COUNT + 1

PIP_HIGH = 1 ----------------------|

9-66

IGNITION TIMING STRATEGY, KNOCK CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

7)

KNOCK RETARD LOGIC

Whenever the  Knock  strategy  is  enabled,  the  software  calculates  RETINC  in

background  as  a  function  of RPM.  RETINC is subtracted from SPKAD.  To prevent

excessive retard (perhaps due to  erroneous  knock  sense)  SPKAD  is  clipped  to

RETMAX.

During a particular PIP period, the software makes adjustments to SPKAD  based  on

whether Knock was sensed during the previous PIP period and uses SPKAD, calculated

during the previous engine cycle (NUMCYL PIP periods ago) to determine  the  final

value of spark advance for the next spark output.  If knock is detected during the

goose test (on demand self test), the GOOS_KNK_DET  flag  is  set  and  the  spark

advance  set  to  a  new lower value.  Spark advance would have been set to a high

value in order to induce knock.  Once knock is detected, spark advance is  reduced

to avoid engine damage.

If the engine speed is greater than RPMMAX, SPKAD is not updated.

Foreground segment in HDR_SPOUT_KNOCK_CALCULATION

ER_STATUS = GOOS_ER_TEST ----------|

|AND -| GOOS_KNK_DET = 1

KNOCK_OCCURRED = 1 ----------------|     | ER_SPARK = GOOS_SPK_TST

|  (Set spark to lower value once

|   knock has beeb detected)

KNOCK_ENABLED = 1 -----------------|

|

KNOCK_OCCURRED = 1 ----------------|

(knock sensed during last PIP     |

Period)                          |

|AND -| SPKAD = SPKAD - RETINC

TIPRET = 0 ------------------------|     | (Clip min. SPKAD to RETMAX)

|

N < RPMMAX ------------------------|

|

PIP_HIGH = 1 ----------------------|

9-67

IGNITION TIMING STRATEGY, SPOUT CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

9.13  SPOUT CALCULATION (CDAN0)

OVERVIEW

This module determines the total desired foreground spark  angle.   Supported

in this calculation are the following features:

Knock sensor-based knock control

Air charge-based tip-in knock control

Throttle-based tip-in knock control

Torque modulation during shift events

DEFINITIONS

Registers:

-  SAF = Final spark advance in degrees.

-  SAF_MOD = Maximum spark advance when torque modulation is required, deg.

-  SAFTOT = Final spark advance including  both  background  and  foreground

modifiers.

-  SAFTOT_PREV = Last pass value of SAFTOT.

-  SPKAD = Spark adder for knock.  It is added to SAF and may be posative or

negative, degrees.

-  SPK_BDL_CLP = Borderline detonation spark limit modified  for  change  in

cylinder  air  charge from when borderline detonation limit (SPK_BDL) was

computed.

-  SPK_RMP_TMR = Spark ramp timer for torque moduleation, msec.

-  SPK_SOURCE = Identifies the source calculation for the final spark value.

-  TIPRET = Tip in retard, degrees.

Bit Flags:

-  FLG_SPK_TQM = Torque modulation required flag;  1  ->  torque  modulation

required.

-  OSC_SAF_FLG = Flag denoting control of SAF by output state control; 1  ->

SAF determined by OSC, 0 -> SAF determined regularly.

9-68

IGNITION TIMING STRATEGY, SPOUT CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SPARK_SPOUT_COM4

BEGIN:  spark_spout_pid_definitions

; This section is defining a pid. It does not necessarily have to be

; implemented in the foreground.

unconditionally ----------| saf_pid := (2 * SAFTOT) + 128

pid_def(j1979_01_0E, saf_pid)

END:  spark_spout_pid_definitions

BEGIN:  spark_spout_main

unconditionally ----------| Call "Knock retard logic"

| Call "Tip-in retard logic"

; Add retard for cylinder knock and tip-in retard if not in output state

; control mode.  Only allow spark to advance past nominal value if

; borderline-limited.

OSC_SAF_FLG <> 1 -------|

|AND -| t_saftot := SAF + TIPRET + SPKAD

SPKAD < 0 --------|     |     |

|OR --|     |

SPK_SOURCE = 2 ---|           |

| --- ELSE ---

|

OSC_SAF_FLG <> 1 -------------| t_saftot := SAF + TIPRET

|

| --- ELSE ---

|

| t_saftot := SAF

; Compare value to that desired by torque modulation if not in output state

; control mode.

FLG_SPK_TQM = 1 ------------|

|OR --|

SPK_RMP_TMR > 0 ------------|     |

|AND -| t_saftot := SAF_MOD

t_saftot > SAF_MOD ---------------|

|

OSC_SAF_FLG <> 1 -----------------|

9-69

IGNITION TIMING STRATEGY, SPOUT CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUE:  spark_spout_main

; Apply foreground borderline update if it is more retarded than current

; value of spark advance.

OSC_SAF_FLG <> 1 ---------|

|AND -| t_saftot := SPK_BDL_CLP

SPK_BDL_CLP < t_saftot ---|

; Store value of SAFTOT from previous cylinder event and call high data

; rate spark module.

unconditionally ----------| SAFTOT_PREV := SAFTOT

| SAFTOT := t_saftot

| Do: high_data_rate_spark

END:  spark_spout_main

9-70

IGNITION TIMING STRATEGY, PIP HIGH/LOW TRANSITION - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

9.14  PIP HIGH/LOW TRANSITION (CDAA0)

OVERVIEW

The spark routine (SPOUT_CALCULATION) is called from the PIP edge  processing

routine.   If  the  engine speed is low, the SPOUT_CALCULATION is called from

both the high-to-low (down) edge of PIP and the low-to-high (up) edge of PIP,

otherwise  SPOUT_CALCULATION is only called from the low-to-high (up) edge of

PIP.  When the engine speed is low and a spark retard is requested, a make-up

SAPW pulsewidth is output from the low-to-high edge of PIP.

DEFINITIONS

INPUTS

Registers:

-  UNDSP = Run/Underspeed flag; 1 -> underspeed (or CRANK), 0 -> run.

Bit Flags:

-  SAPW_HI_FLG = Edge to calculate main SAPW; 1 -> use PIP up edge, 0 -> use

PIP down edge.

PROCESS

STRATEGY MODULE: SPARK_PIP_HI_LO_COM1

PIP High Transition

UNDSP = 0 --------------------------------| Call SPOUT_CALCULATION

(in RUN mode)                            | "Continue with PIP processing"

|

| --- ELSE ---

|

| "Continue with PIP processing"

PIP Low Transition

SAPW_HI_FLG = 0 --------------------------| Call SPOUT_CALCULATION

(engine speed low)                       | "Continue with PIP processing"

|

|

| --- ELSE ---

|

| "Continue with PIP processing"

9-71

IGNITION TIMING STRATEGY, SPARK INTAKE MANIFOLD RUNNER CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

9.15  SPARK INTAKE MANIFOLD RUNNER CONTROL LOGIC (CDAL0)

OVERVIEW

The Intake Manifold Runner Control (IMRC) Logic adds several degrees of spark

based  on  engine  speed, load and rate of IMRC valve transition from open to

closed.

DEFINITIONS

Registers:

-  IMRCSPK_W  =  Weighting  factor  for  IMRC  spark  modifiers.    Unitless

multiplier.  from 0 (IMRC valve fully Closed) to (IMRC valve fully open).

-  LOAD = Universal load variable.

-  N = Engine RPM.

-  SPKBASE_IMRC =  Base  spark  adder  when  using  intake  manifold  runner

control, set to zero otherwise.  Degrees, positive for advance.

-  SPK_BDL_IMRC = Borderline spark adder when using intake  manifold  runner

control, set to zero otherwise.  Degrees, positive for advance.

-  SPK_MBT_IMRC = Maximum brake torque  adder  when  using  intake  manifold

runner control, set to zero otherwise.  Degrees, positive for advance.

Calibration Constants:

-  FN011(N) = Normalized engine speed.  Intended to span only the upper RPM

range for "high rpm" features.

-  FN013(LOAD) = Normalized LOAD for half-sized spark tables.

-  FN2160A(N,LOAD) = Spark adder for base spark calculation with IMRC

control.

X = FN011(N)

Y = FN013(LOAD)

-  FN2260A(N,LOAD) = Spark adder for borderline spark calculation with IMRC

control.

X = FN011(N)

Y = FN013(LOAD)

-  FN2360A(N,LOAD) = Spark adder for maximum brake torque spark calculation

with IMRC control.

X = FN011(N)

Y = FN013(LOAD)

-  IMRCHP = IMRC hardware present flag; 1 -> IMRC hardware present.

9-72

IGNITION TIMING STRATEGY, SPARK INTAKE MANIFOLD RUNNER CONTROL LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SPARK_IMRC_COM3

BEGIN:  SPARK_IMRC_ADDERS

; This module executes once every background loop before the base,

; borderline and maximum brake torque spark calculations.

IMRCHP = 1 ------------------| SPKBASE_IMRC := IMRCSPK_W * FN2160A(N,LOAD)

| SPK_BDL_IMRC := IMRCSPK_W * FN2260A(N,LOAD)

| SPK_MBT_IMRC := IMRCSPK_W * FN2360A(N,LOAD)

|

| --- ELSE ---

|

| SPKBASE_IMRC := 0

| SPK_BDL_IMRC := 0

| SPK_MBT_IMRC := 0

END:  SPARK_IMRC_ADDERS

9-73

IGNITION TIMING STRATEGY, HIGH DATA RATE ELECTRONIC SPARK - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

9.16  HIGH DATA RATE ELECTRONIC SPARK (CDAM0)

The High Data Rate Electronic Spark System consists of three components:

a) EEC-IV processor

b) EDIS module

c) A 36-tooth wheel and sensor

The 36-tooth wheel is located on the crankshaft with  a  variable  reluctance

sensor  (VRS) to produce a signal.  The wheel is missing one tooth at 90 BTDC

for 4-cyl, at 60 BTDC for 6-cyl, and at 50 BTDC for  8-cyl  engines  for  one

cylinder to allow signal reference.

The EDIS module is an EED-supplied module which has the capability to receive

the  output of the VRS sensor, as well as a signal from the EEC-IV processor.

The EDIS module provides coil  switching  for  distributorless  ignition  and

outputs signals to the coil for charging and firing.  In addition, the module

provides a signal to the EEC-IV processor, specifically, a synthetic 50% duty

cycle  with  the  low-to-high transition occuring at 10 degrees BTDC for each

cylinder and a high-to-low transition  occuring  at  (180/ENGCYL)  after  the

low-to-high  transition.   The module also provides LOS function whenever the

EEC-IV processor fails to provide a Spark Angle Pulse Width (SAW).   Included

within  these  LOS conditions is the SPOUT line remaining in either a high or

low state for more than five PIP periods or if the pulsewidth of  the  signal

is outside the clip limits.  LOS spark is positioned at 10 BTDC.

The EEC-IV processor uses the synthetic  50%  duty  cycle  as  reference  for

scheduling fuel and spark pulse widths.  The EEC-IV processor calculates:  1)

The  time  that  the  Spark  Output  (SPOUT)  line  to  the  EDIS  should  be

transitioned  from  low-to-high;  2)  The  SAPW; and 3) The time of the SPOUT

high-to-low transition.

During low engine speeds, SAPW_HI_FLG = 0, the pulsewidth  is  calculated  on

both  the  PIP  up and down edge, with the output of the main SAPW on the PIP

down edge and when required, the output of the makeup  SAPW  on  the  PIP  up

edge.   During  higher  engine  speeds,  SAPW_HI_FLG  =  1, the pulsewidth is

calculated on the PIP up edge and output at 20 degrees ATDC.  The makeup SAPW

requirement  exists  when  the requested spark is retarded between 5 and some

variable amount dependent on engine speed from the inital requested  position

that was output on the main SAPW on the PIP down edge.

Below 1000 RPM, multiple spark firing attempts can be  requested.   When  the

calibration  constant  REP_SPK_DSRD  = 1, the strategy will modify the output

pulsewidth to indicate to the EDIS module to fire the  coil  at  the  desired

spark angle and successively until reaching TDC.  This provides the increased

probability of igniting the mixture and not creating a mis-fire.   Repetitive

spark  is initialized by writing sending a 2048 SAPW.  It is initialized each

time it is first requested.

9-74

IGNITION TIMING STRATEGY, HIGH DATA RATE ELECTRONIC SPARK - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

REQUESTED SPARK ANGLE TO SAPW TRANSFER FUNCTION

SAPW  |

(usec) |

|

2048..| <- Crank with Repetitive Spark

|

|

|

|

1796..|

|\

| \

|  \

|   \

|    \

|     \

|      \

|       \

|        \

1284..|.........\ <- Crank and underspeed in Normal Spark

|         .\

|         . \

|         .  \

|         .   \

|         .    \

|         .     \

|         .      \

|         .       \

|         .        \

|         .         \

|         .          \

|         .           \

|         .            \

|         .             \

|         .              \

|         .               \

|         .                \

|         .                 \

|         .                  \

|         .                   \

|         .                    \

|         .                     \

|         .                      \

|         .                       \

68  ------------------------------------------- SAFTOT

|         |                        |        (degrees BTDC)

-10        10                      57.5

9-75

IGNITION TIMING STRATEGY, HIGH DATA RATE ELECTRONIC SPARK - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  DT12S = The period of time between two adjacent rising edges of PIP.

-  HFDLTA = Time period from last PIP up edge to last PIP down edge.

-  LAST_HI_PIP = Time of most recent high PIP edge.

-  N = Engine RPM.

-  NEXT_SPOUT_BETA = Time period from the reference pip edge to the  desired

transition of the SPOUT/SAPW output from low to high.

-  SAFTOT = Final spark advance including  both  background  and  foreground

modifiers.

-  SAFTOT_PREV = Last pass value of SAFTOT.

-  SAPW = HDR spark pulsewidth, microseconds.

-  SPOUT_LOW_TIME = Time period during which the SPOUT/SAPW output  is  high

and used to calculate the time of transition from low to high.

Bit Flags:

-  CRKFLG = Engine Mode Flag; 1 -> Crank Mode, 0 <> Crank Mode.

-  MAKUPSPK_FLG = Within the  SPOUT_CALCULATION  module,  indicates  if  the

current  PIP  edge is an up edge, if the main calculation is performed on

the PIP down edge, and if the change in requested spark is in the  retard

direction;  1  ->  meets  conditions for make up SAPW, 0 -> does not meet

conditions.

-  PIP_HIGH = State of PIP transition; 1 ->  low-to-high  transition,  0  ->

high-to-low transition

-  REPET_SPK = Indicates when to use repetitive spark;  ignition  module  to

discharge the coil across the spark plug.

-  SAPW_HI_FLG = A flag indicating which PIP  edge  to  calculate  the  main

spark angle pulsewith; 1 -> PIP up edge, 0 -> PIP down edge.

-  UNDSP = Flag that indicates when not in run mode.

Calibration Constants:

-  ENGCYL = The number of cylinders in one engine revolution.

-  REP_SPK_DSRD = Flag;  1  ->  repetitive  spark  desired  when  conditions

permit.

-  REPET_SPK_HI  =  Repetitive  spark  disabled  above  this  engine  speed,

rev/min.

9-76

IGNITION TIMING STRATEGY, HIGH DATA RATE ELECTRONIC SPARK - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  REPET_SPK_LO = Repetitive spark enabled below this engine speed, rev/min.

-  SAPW_UP_SH = Engine speed set for SAPW_HIGH = 1.

-  SAPW_UP_CL = Engine speed clear for SAPW_HIGH.

Non-Calibratible Constants:

-  ustcf = Microsecond-to-ticks conversion.

OUTPUTS

Registers:

-  NEW_TIME = The time on  the  master  clock  when  the  SAPW  output  will

transition from low to high.

-  NEXT_SPOUT_BETA = See above.

-  SAPW = See above.

-  SPOUT_LOW_TIME = See above.

Bit Flags:

-  IMMEDIATE = A flag that indicates that the output is to be placed in  the

hardware output carousel as soon as possible.

-  MAKUPSPK_FLG = Within the  SPOUT_CALCULATION  module,  indicates  if  the

current  PIP  edge is an up edge, if the main calculation is performed on

the PIP down edge, and if the change in requested spark is in the  retard

direction;  1  ->  meets  conditions for make up SAPW, 0 -> does not meet

conditions.

-  REPET_SPK = See above.

-  SAPW_HI_FLG = A flag indicating which PIP  edge  to  calculate  the  main

spark angle pulsewith; 1 -> PIP up edge, 0 -> PIP down edge.

9-77

IGNITION TIMING STRATEGY, HIGH DATA RATE ELECTRONIC SPARK - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SPARK_HDR_COM1

BEGIN:  HIGH_DATA_RATE_SPARK

Unconditionally -------------------| DO:  ENGINE_SPEED_FLIP_FLOP

UNDSP = 1 -------------------|

|

PIP_HIGH = 0 ----------------|OR --| MAKUPSPK_FLG := 0

|     |

SAPW_HI_FLG = 1 -------|     |     |

|AND -|     |

PIP_HIGH = 1 ----------|           |

| --- ELSE ---

UNDSP = 0 -------------------|     |

|     |

SAPW_HI_FLG = 0 -------------|     |

|AND -| MAKUPSPK_FLG := 1

PIP_HIGH = 1 ----------------|     |

|     |

SAFTOT < SAFTOT_PREV --------|     |

| --- ELSE ---

|

| MAKUPSPK_FLG := 0

| EXIT: HIGH_DATA_RATE_SPARK

Compute the pulsewidth which is equivalent to the desired spark and  clip  to

minimum  and  maximum  limits  equivalent  to  10  ATDC  and  57.5  BTDC.  In

repetitive  spark  mode,  maximum  limit  is  50  BTDC.   During  CRANK   and

UNDERSPEED, SAPW is set to the equivalent of 10 degree BTDC.

UNDSP = 0 --------------------------| t_sapw := 1540 - (256 * SAFTOT/10)

|

| --- ELSE ---

|

| t_sapw := 1284

t_sapw < 260 usec ------------|

|AND -| t_sapw := 260 usec

REPET_SPK = 1 ----------------|     |

| --- ELSE --

|

t_sapw < 68 usec -------------------| t_sapw := 68 usec

|

| --- ELSE ---

|

t_sapw > 1796 usec -----------------| t_sapw := 1796 usec

9-78

IGNITION TIMING STRATEGY, HIGH DATA RATE ELECTRONIC SPARK - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUE:  HIGH_DATA_RATE_SPARK

If repetitive spark is desired, the calibration parameter REP_SPK_DSRD is set

to  one (1); and a calibration value of 2048 usec will be sent in CRANK mode.

During the transition from UNDERSPEED to RUN modes, there  is  no  SAPW  sent

out.   When  repetitive  spark  is  desired  and  the engine speed is below a

calibratable value, the value of 2048 usec is added  to  the  computed  SAPW.

Repetitive  spark  is  enabled  when  the  engine  speed  is  less  than  the

calibratable value REPET_SPK_LO and disabled when  engine  speed  is  greater

than the calibratable value REPET_SPK_HI.

REP_SPK_DSRD = 1 ---------------|

|AND -| t_sapw := 2048 usec

CRKFLG = 1 ---------------|     |     | REPET_SPK := 1

|OR --|     |

N < REPET_SPK_LO ---|     |           |

|AND -|           |

REPET_SPK = 0 ------|                 |

| --- ELSE ---

REP_SPK_DSRD = 1 ---------------|     |

|     |

N < REPET_SPK_HI ---------------|AND -| t_sapw := t_sapw + 2048 usec

|     | REPET_SPK := 1

REPET_SPK = 1 ------------------|     |

| --- ELSE ---

|

| REPET_SPK := 0

9-79

IGNITION TIMING STRATEGY, HIGH DATA RATE ELECTRONIC SPARK - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUE:  HIGH_DATA_RATE_SPARK

Compute spark angle pulsewidth in ticks and store as  SPOUT_LOW_TIME.   Round

up  the  result if the remainder is greater than or equal to a half tick.  If

main SAPW is output from the PIP high edge, compute the on time for  SAPW  to

be at 20 ATDC.

always -----------------------------| t_angle := (30 * ENGCYL / 360) * DT12S

SAPW_HI_FLG = 1 --------------------| NEXT_SPOUT_BETA := t_angle

| t_slt := t_sapw * ustcf

| (high speed main pulse)

|

| --- ELSE ---

SAPW_HI_FLG = 0 --------------|     |

|AND -| t_slt := t_sapw * ustcf

MAKUPSPK_FLG = 0 -------------|     | (crank, underspeed, & low speed main

|  pulse)

|

| --- ELSE ---

MAKUPSPK_FLG = 1 -------------|     |

|     |

t_sapw - SAPW > 128 ----------|AND -| SPOUT_LOW_TIME := (t_sapw - SAPW)

|     |                    * ustcf

t_sapw - SAPW <               |     | (low speed makeup pulse)

(t_angle / 3) ---------------|     |

| --- ELSE ---

MAKUPSPK_FLG = 1 -------------|     |

|     |

t_sapw - SAPW > 128 ----------|AND -| SPOUT_LOW_TIME := (t_angle / 3)

|     | (low speed makeup pulse clip)

(t_sapw - SAPW) >=            |     |

(t_angle / 3) ---------------|     |

| --- ELSE ---

|

| Do not put out pulse

| Exit routine

9-80

IGNITION TIMING STRATEGY, HIGH DATA RATE ELECTRONIC SPARK - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUE:  HIGH_DATA_RATE_SPARK

Prevent a pulsewidth that is too long.

SAPW_HI_FLG = 1 --------------|

|AND -| SPOUT_LOW_TIME := (DT12S -

t_slt > =                     |     |                   NEXT_SPOUT_BETA)

(DT12S - NEXT_SPOUT_BETA) --|     |

| SAPW := SPOUT_LOW_TIME / ustcf

|

| REPET_SPK := 0

|

| --- ELSE ---

SAPW_HI_FLG = 0 --------------|     |

|AND -| SPOUT_LOW_TIME := HFDLTA

t_slt >= HFDLTA --------------|     |

| SAPW := HFDLTA / ustcf

|

| REPET_SPK := 0

|

| --- ELSE ---

|

MAKUPSPK_FLG = 0 -------------------| SPOUT_LOW_TIME := t_slt

|

| SAPW := t_sapw

9-81

IGNITION TIMING STRATEGY, HIGH DATA RATE ELECTRONIC SPARK - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUE:  HIGH_DATA_RATE_SPARK

Determine when to start the spark angle pulse.  When the  make-up  pulsewidth

is  output,  it  must  be  sent  as soon as possible after the PIP up edge to

ensure that the off edge goes out  prior  to  10  ATDC.   The  limit  of  1.0

millisecond on the make-up pulse should meet this requirement since the worst

case is an 8 cylinder engine at 1500 RPM allows 2.2 milliseconds between  the

PIP  up-edge and the the 10 ATDC position.  The main SAPW is sent from either

20 ATDC or the PIP down edge, dependent on the state of SAPW_HI_FLG.

MAKUPSPK_FLG = 1 -------------------| IMMEDIATE := 1

| (put out makeup pulsewidth equal to

|  SPOUT_LOW_TIME)

|

| --- ELSE ---

|

SAPW_HI_FLG = 0 --------------------| IMMEDIATE := 1

| (put out main SAPW with up_edge at

|  PIP low edge and pulsewidth equal to

|  SPOUT_LOW_TIME)

|

| --- ELSE ---

|

SAPW_HI_FLG = 1 --------------------| NEW_TIME := LAST_HI_PIP +

|             NEXT_SPOUT_BETA

| (put out main SAPW with up_edge at

|  NEW_TIME and pulsewidth equal to

|  SPOUT_LOW_TIME)

|

| --- ELSE ---

|

| Do not put out pulse

| Exit routine

END: HIGH_DATA_RATE_SPARK

9-82

IGNITION TIMING STRATEGY, HIGH DATA RATE ELECTRONIC SPARK - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN:  ENGINE_SPEED_FLIP_FLOP

; This process executes when called from HIGH_DATA_RATE_SPARK.  Note that it

; reads top down.

The  SAPW_HI_FLG  logic  determines  when  the  high  data  rate  spark  main

calculation  is  performed.   When the flag is set, the main SAPW calculation

occurs on the PIP low-to-high (high,up) transition and  is  delivered  at  20

degrees  ATDC.   When  the flag is clear, the main SAPW calculation occurs on

the PIP high-to-low (low, down) transition and is delivered immediately.  The

maximum recommended value for SAPW_UP_SH is:

3750 RPM -- 4 cylinder engine

2500 RPM -- 6 cylinder engine

1875 RPM -- 8 cylinder engine

The  recommended  value  for  SAPW_UP_CL  is  300  RPM  below  the  value  of

SAPW_UP_SH.

UNDSP = 0 ------------|

|AND -|S Q -| SAPW_HI_FLG

N > SAPW_UP_SH -------|     |

|

UNDSP = 1 ------------|     |

|OR --|C

N < SAPW_UP_CL -------|

END:  ENGINE_SPEED_FLIP_FLOP

9-83

IGNITION TIMING STRATEGY, HIGH DATA RATE ELECTRONIC SPARK - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

HIGH DATA RATE SPARK

(High speed operation)

PIP

________          ________          ________          ________          __

|        |        |        |        |        |        |        |        |

__|        |________|<-\     |________|        |________|        |________|

.   \             .        .         .

.    \LAST_HI_PIP .        .         .

.                 .        .         .

t_sapw             .                 .        .         .

__             .  __             .  __    .         . __

____|  |____________._|  |______________|  |___._________.|  |______________|

20 ATDC --.>|                        .         .

--.>|  |<-- SPOUT_LOW_TIME   .         .

desired spark       .                      --->.         .<-----

(SAFTOT) computed   .                      |__ desired spark __|

prior to ---------->.<-- t_sapw computed        delivered here

HIGH RATE SPARK

(Low speed with retard in progress)

PIP

________          ________          ________          ________          __

|        |        |        |        |        |        |        |        |

__|        |________|        |________|        |________|        |________|

:        .        :         .

t_sapw    :        .        :         .

:__      ._       :___     _.        ____              ___

___________|  |_____| |______|   |____||_______|    |____________|   |_____

:        .        :         .

:        .        :         .

|<-t_sapw computed here and .

delivered within this range |<------->|

.

|<-make-up SAPW computed here

9-84

IGNITION TIMING STRATEGY, VARIABLE CAM TIMING SPARK CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

9.17  VARIABLE CAM TIMING SPARK CONTROL LOGIC (CDAK0)

OVERVIEW

This module produces incremental  base,  borderline,  and  MBT  spark  timing

adjustments  which  are  desired for engines with variable cam timing.  These

incremental adjustments are included in determination of  the  overall  spark

timing by the spark strategy.

Note that this module is intended to be  used  for  strategies  that  do  not

intend to use variable cam timing and thus set the adders to zero.

DEFINITIONS

OUTPUTS

Registers:

-  SPKBASE_VCT = Base spark adder when using variable  cam  timing,  set  to

zero otherwise.  Degrees, negative for retard, positive for advance.

-  SPK_BDL_VCT = Borderline spark adder when using variable cam timing,  set

to zero otherwise.  Degrees, negative for retard, positive for advance.

-  SPK_MBT_VCT = Maximum brake torque spark adder when  using  variable  cam

timing,  set  to  zero otherwise.  Degrees, negative for retard, positive

for advance.

PROCESS

STRATEGY MODULE: SPARK_NO_VCT_COM1

BEGIN: SPARK_VCT_ADDERS

; This routine is executed once every background loop.

unconditionally ------------------------| SPKBASE_VCT := 0

| SPK_BDL_VCT := 0

| SPK_MBT_VCT := 0

END:  SPARK_VCT_ADDERS

9-85

IGNITION TIMING STRATEGY, SPARK FOR FLEXIBLE FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

9.18  SPARK FOR FLEXIBLE FUEL (CDAL0)

OVERVIEW

The spark advance at which borderline detonation occurs will  vary  according

to  the  proportion of methanol in the fuel.  This logic calculates an offset

which is dependant on Engine Speed, Load and the proportion  of  methanol  in

the fuel.

This module is a placeholder for strategies that do not  use  flexible  fuel.

It sets the spark adder to zero.

DEFINITIONS

OUTPUTS

Registers:

-  SPK_BDL_FFS = Borderline spark adder when using  flexible  fuel,  set  to

zero otherwise.  Degrees, negative for retard, positive for advance.

9-86

IGNITION TIMING STRATEGY, SPARK FOR FLEXIBLE FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SPARK_NO_FLEX_FUEL_COM1

BEGIN:  SPARK_FLEX_FUEL_ADDERS

; This module executes once every background loop.

unconditionally ----------| SPK_BDL_FFS := 0

END:  SPARK_FLEX_FUEL_ADDERS

9-87

IGNITION TIMING STRATEGY, TORQUE MODULATION RAMP - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

9.19  TORQUE MODULATION RAMP (CDAK0)

OVERVIEW

To insure a smooth transition in torque  changes  during  torque  modulation,

spark  is  tapered  in and out via a ramp.  The ramp is executed on a 16 msec

repeater.

The flag FLG_REP_ENA is controlled in background to insure that the  16  msec

repeater is executed only when necessary.

DEFINITIONS

INPUTS

Registers:

-  SPKSLP = Spark ramp rate for torque modulation, deg/msec.

-  SPK_RMP_TMR = Timer used to control  the  duration  of  spark  ramps  for

torque modulation, msec.

9-88

IGNITION TIMING STRATEGY, TORQUE MODULATION RAMP - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Bit Flags:

-  FLG_DN_TQM = Downshift Torque Modulation Flag.

-  FLG_REP_ENA = Indicates 16 msec repeater is enabled for spark  ramps  for

torque modulation.

-  FLG_SPK_TQM = Torque Modulation Spark Retard flag.

OUTPUTS

Registers:

-  SAF_DES = Spark advance required to  achieve  the  desired  torque  level

during torque modulation, deg.

-  SAF_MOD = Maximum total spark advance during torque modulation, deg.

-  SAF_MOD_LST = Last pass value of SAF_MOD, deg.

-  SPK_CLK_LST = Last pass value of CLOCK; used to compute spark  ramps  for

toruqe modulation, msec.

-  SPK_DELT_IN = Delta spark stepped in during torque modulation, deg.

-  SPK_DELT_OUT = Delta spark stepped out during torque modulation, deg.

Bit Flags:

-  FLG_REP_ENA = See above.

9-89

IGNITION TIMING STRATEGY, TORQUE MODULATION RAMP - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SPARK_TQMOD_RMP_COM2

BEGIN: MAIN

; This process is executed every background loop.

FLG_SPK_TQM = 1 ---------------|

|OR --| FLG_REP_ENA := 1

FLG_SPK_TQM = 0 ---------|     |     |

|AND -|     |

SPK_RMP_TMR > 0 ---------|           |

| --- ELSE ---

|

| FLG_REP_ENA := 0

|  ; disable 16 msec repeater

END: MAIN

9-90

IGNITION TIMING STRATEGY, TORQUE MODULATION RAMP - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: 16_MSEC_REPEATER

; This process is executed once every 16 milliseconds.  Note that

; SPK_DELT_IN is set in SPARK_TQM_BG_COMn.

FLG_REP_ENA = 0 ---------------------| EXIT: 16_MSEC_REPEATER

|

| --- ELSE ---

FLG_SPK_TQM = 1 ---------------|     |

|AND -| spk_tm := ( current CLOCK time

SPK_RMP_TMR > 0----------------|     |            - SPK_CLK_LST )

| SPK_DELT_OUT := SPKSLP * spk_tm

|  ; Convert spk_tm to milliseconds

|  ; for the SPK_DELT_OUT calculation

| SAF_MOD := SAF_MOD_LST - SPK_DELT_OUT

| SAF_MOD := max( SAF_MOD, SAF_DES )

|

| --- ELSE ---

|

FLG_SPK_TQM = 1 ---------------------| SAF_MOD := SAF_DES

|

| --- ELSE ---

|

| SAF_MOD := SAF_MOD_LST + SPK_DELT_IN

always ------------------------------| SAF_MOD_LST := SAF_MOD

| SPK_CLK_LST := current CLOCK time

END: 16_MSEC_REPEATER

9-91

CONTINUOUS SELF TEST, SPARK OUTPUT TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

9.20  SPARK OUTPUT TEST (CDAN0)

OVERVIEW

This strategy detects a fault (P1359malf) in the SAW  output.   If  the  EDIS

module  does  not  receive  a  SAW  pulse,  it fires the coil at a default 10

degrees BTDC.  The IDM low to high transition (IDMUPEDGTM) happens  when  the

coil  is  fired, so it is possible to calculate the actual spark advance.  If

the required spark advance (SAFTOT_OLD2) is greater  than  the  actual  spark

advance  (SPK_ACTUAL)  by  a calibrateable amount (SPK_POS_ERR), a failure is

indicated.

There is a two PIP delay between the  calculation  of  SAFTOT  and  the  EDIS

module giving that spark advance to the engine.  For this reason the value of

SAFTOT which occurred two PIP periods ago (SAFTOT_OLD2) is saved and compared

with the current actual spark advance.

A free run timer (SAW_ER_TMR) is set to zero every  time  a  failure  is  not

detected.  Therefore if a failure is detected the counter is allowed to count

up.  When this  counter  reaches  a  calibrateable  value  (SAW_ER_TIM),  the

failure code is set.

During rapid accelerations  or  decelerations  the  PIP  period  is  changing

rapidly.   This  could  cause  the  test  to  incorrectly  detect  an  error.

Therefore the  test  is  disabled  under  these  conditions  (DT12S/DT23S  or

DT23S/DT12S  >  V_DIS_PIPDOT) and also, if the IDM pulse doesn't occur within

the Spout test window.

IDM_TM will be calculated only if the requested spark  advance  (SAFTOT_OLD2)

is  between  B  and  C  as  shown  in  Fig.3.  If the requested spark advance

(SAFTOT_OLD2) falls within A to B or C to D, spout test will set  IGNTST_RSIR

= 0, skip calculation of IDM_TM.

9-92

CONTINUOUS SELF TEST, SPARK OUTPUT TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|<---------------- DT12S ---------------->|

|--------------------|                    |------------

PIP        |                    |                    |<-- LAST_HI_PIP

---|                    |--------------------|

IDM_TM  |<---->|

|--|

IDM                            IDMUPEDGTM ---->|  |

---------------------------------------|  |---------------

Fig. 1

|<---------------- DT12S ---------------->|

|--------------------|                    |------------

PIP        |                    |                    |<-- LAST_HI_PIP

---|                    |--------------------|

IDM_TM  |<--->|

|--|

IDM                                        IDMUPEDGTM ---->|  |

---------------------------------------------------|  |----

Fig. 2

DEFINITIONS

Registers:

-  DATA_TIME = 3 byte time of transition.

-  DT12S = The period of time between two  adjacent  rising  edges  of  PIP,

ticks/pip.

-  DT23S = Last DT12S time.

-  IDM_TM = Time between last PIP up-edge and last IDM up-edge.

-  IDMUPEDGTM = IDM up edge transition time.

-  IGNTSTMAXADV  =  Requested  spark  advance  above  which  Spout  test  is

performed ,degrees BTDC

-  IGNTSTMINADV = Requested spark retard (a  negative  number)  above  which

Spout test is performed ,degrees ATDC

-  LAST_HI_PIP = Most recent PIP up edge transition time.

-  SAFTOT = Total spark advance, degrees

-  SAFTOT_OLD1 = Value of SAFTOT one PIP period ago, degrees.

-  SAFTOT_OLD2 = Value of SAFTOT two PIP periods ago, degrees.

9-93

CONTINUOUS SELF TEST, SPARK OUTPUT TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  SAW_ER_TMR = Timer that  runs  when  a  fault  with  the  SAW  output  is

detected, sec.

-  SPK_ACTUAL = Actual spark advance as determined from IDM signal, degrees.

Bit Flags:

-  CCM_ER_ENA = Engine running on demand test for CCM enabled.

-  CCM_TST_ENA = OBD-II Comprehensive Component Test enable flag, (1 => test

enabled).

-  ER_DONE = Value of ER_STATUS when engine running test is complete

-  ER_INIT = Value for state of ER_STATUS when  engine  running  test  first

inits.

-  ER_STATUS = State pointer that indicated current state of engine  running

on demand test.

-  KAM_ERROR = When set to 1, it indicates Keep Alive Memory is invalid.

-  OBDII_RESET = Flag used to  simulate  the  receipt  of  obdii  scan  tool

message.

-  PxxxMALF = Malfunction flag for code Pxxx; 1 -> a  malfunction  currently

exists for fault Pxxx.

-  IGNTST_RSIR  =  It  is  set  to  1,  when  the  requested  spark  advance

(SAFTOT_OLD2) is within the spout testing range.

Calibration Constants:

-  ENGCYL = Number of PIP's per engine revolution.

-  IGNTSTADVTOL = Number of degrees from the pip edge (either  up  or  down)

spout test is not performed.

-  SAW_ER_TIM = Time SAPW failure must be  detected  for  in  order  to  set

failure code.

-  SPK_POS_ERR = Maximum possible no fault spark error, degrees.

-  SPTST_CONT = When set to 1, spout testing is performed during  continuous

ccm test is enabled (ccm_tst_ena = 1).

-  SPTST_ER_ENA = When set to 1, spout testing is  performed  during  engine

running on demand ccm test is enabled(ccm_er_ena = 1).

-  V_DIS_PIPDOT = PIP period maximum rate of  change  to  recognize  an  IDM

fault unitless.

OTHER

-  P1359 = Fault code, SAPW failure.

9-94

CONTINUOUS SELF TEST, SPARK OUTPUT TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Guideline for IGNTSTMAXADV and IGNTSTMINADV parameters:

:<---- spout test enabled ---->:

:                              :

:    :                              :    :

----------|    :              |--------------------|

PIP            |    :              |               :    |

|-------------------|               :    |--------

:    :                              :    :

:    :                              :    :

tolerance ->:    :<--                        -->:    :<- tolerance

A    B                              C    D

Fig. 3

IDM_TM will be calculated only if the requested spark  advance  (SAFTOT_OLD2)

is  between  B  and  C  as  shown  in  Fig.3.  If the requested spark advance

(SAFTOT_OLD2) falls within A to B or C to D, spout test will set  IGNTST_RSIR

= 0, skip calculation of IDM_TM and IDMUPEDGTMO

IGNTSTMAXADV := last PIP Down degrees - degrees of tolerance

or

B     :=        A                -      tolerance

IGNTSTMINADV := next PIP Down degrees + degrees of tolerance

or

C     :=        D                +      tolerance

|-----------------------------------------------------------------|

|ENGCYL|NUMCYL|     A      |            |            |    D       |

|      |      |Last PIPDown|IGNTSTMAXADV|IGNTSTMINADV|Next PIPDown|

|-----------------------------------------------------------------|

|      |      |            |            |            |            |

|  2   |  4   |    100     |     B      |     C      |    -80     |

|-----------------------------------------------------------------|

|      |      |            |            |            |            |

|  3   |  6   |     70     |     B      |     C      |    -50     |

|-----------------------------------------------------------------|

|      |      |            |            |            |            |

|  4   |  8   |     55     |     B      |     C      |    -35     |

|-----------------------------------------------------------------|

|      |      |            |            |            |            |

|  5   | 10   |     46     |     B      |     C      |    -26     |

|-----------------------------------------------------------------|

9-95

CONTINUOUS SELF TEST, SPARK OUTPUT TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE:SPARK_SPOUT_TEST_COM1

BEGIN: SPOUT_POWERUP_INIT

;execute only once during power-up

unconditionally -----------------| IGNTSTMAXADV :=

|  (360/(ENGCYL * 2)) + 10 - IGNTSTADVTOL

| IGNTSTMINADV :=

|  (-360/(ENGCYL * 2)) + 10 + IGNTSTADVTOL

END: SPOUT_POWERUP_INIT

BEGIN: SPOUT_PIPUP_FG

;perform the following in the PIP up routine prior to spout calculation

unconditionally ------------------| SAFTOT_OLD2 = SAFTOT_OLD1

| SAFTOT_OLD1 = SAFTOT

| DT23S := MIN(65535, DT12S)

| DT12S := DATA_TIME - LAST_HI_PIP

| LAST_HI_PIP := DATA_TIME

END: SPOUT_PIPUP_FG

BEGIN: SPOUT_IDMUP_FG

;execute this logic in the IDM up interrupt routine

uncondtionally -------------------| IDMUPEDGTM :=  DATA_TIME

|  ;save the value of idm rising edge

END: SPOUT_IDMUP_FG

BEGIN: SPOUT_PIPDOWN_FG

;execute in the pip down routine

SAFTOT_OLD2 < IGNTSTMAXADV -|

|AND -| IDM_TM := LAST_HI_PIP - IDMUPEDGTM

SAFTOT_OLD2 > IGNTSTMINADV -|     |  ;idm-up to pip-up time

| IGNTST_RSIR := 1

|  ;requested spark advance in spout

|  ;testing range

|

| --- ELSE

|

| IGNTST_RSIR := 0

|  ;requested spark advance out of

|  ;spout testing range

END: SPOUT_PIPDOWN_FG

9-96

CONTINUOUS SELF TEST, SPARK OUTPUT TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;begin background processing

BEGIN: SPOUT_TEST_BG

;execute once per bg loop

KAM_ERROR = 1 -----------------------|

|

CCM_TST_ENA = 0 ---------------------|

;cont test enbld by obdii exec       |

|

SPTST_CONT = 0 ----------------------|

;spout cont test cal'd out           |

|

CCM_ER_ENA = 0 ----------------------|

;er test enabld by obdii exec        |

|

SPTST_ER_ENA = 0 --------------------|

;spout er test cal'd out             |

|

OBDII_RESET = 1 ---------------------|OR --| SAW_ER_TMR := 0

;scan tool reset msg                 |     |  ;reset timer

|     |

ER_STATUS = ER_INIT -----------------|     | (exit spout_test_bg)

;start self-test                     |     |

|     |

ER_STATUS = ER_DONE -----------------|     |

;end self-test                       |     |

|     |

IGNTST_RSIR = 0 ---------------------|     |

;req. SA out of spout test range     |     |

|     |

(Min(DT12S, 65535)/DT23S)            |     |

> V_DIS_PIPDOT ---|     |

|     |

(DT23S/Min(DT12S, 65535))            |     |

> V_DIS_PIPDOT ---|     |

;rapid accels/decels detected              |

| --- ELSE ---

|

| Do: SPOUT_CALC_SPKACTUAL

|  ;calc actual spark advance

| Do: SPOUT_MALF

|  ;set/clear malf code

END: SPOUT_TEST_BG

9-97

CONTINUOUS SELF TEST, SPARK OUTPUT TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: SPOUT_CALC_SPKACTUAL

;execute when called

unconditionally  ---| SPK_ACTUAL =

|   ((IDM_TM / Min(DT12S, 65535)) * (360 / ENGCYL)) + 10

END: SPOUT_CALC_SPKACTUAL

BEGIN: SPOUT_MALF

;execute when called

abs(SAFTOT_OLD2 - SPK_ACTUAL)

< SPK_POS_ERR  ---| SAW_ER_TMR = 0

;difference is less than possible error    | ;no fault, reset timer

|

| clear_malf(P1359)

|

| --- ELSE ---

|

SAW_ER_TMR > SAW_ER_TIM  ------------|     |

|AND -| malfunction(ccm,P1359)

P1359MALF = 0 -----------------------|

;failure has been present for the required

;time and code has not been previously set

END: SPOUT_MALF

;end background processing

9-98

IGNITION TIMING STRATEGY, TRANSIENT SPARK FOR DETONATION PREVENTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

9.21  TRANSIENT SPARK FOR DETONATION PREVENTION (CDAN0)

OVERVIEW

This is  the  foreground  portion  of  the  transient  spark  for  detonation

prevention.  It is based on cylinder air charge.

The borderline spark value is calculated  in  the  background  and  does  not

reflect  the  rapidly  changing air charge value during a tip-in event.  This

incorrect  borderline  value  means  knock  cannot  be  controlled  via   the

borderline calculation and other means must be employed.

The value of the cylinder air charge is stored in the  background  such  that

the current foreground value may be compared to it.  If the difference in the

background to foreground values is significant, it is marked as a tip-in.

The difference in cylinder air charge is multiplied by a gain value such that

the  result  determines  the  amount  of  spark to retard from the borderline

value.  This is known as the "foreground borderline update."

The foreground logic executes in 4 states (SPK_TIPSTATE):

0 = No tip-in is present

1 = Tip-in event has been sensed

2 = Searching for the peak rise-rate of the cylinder air charge

3 = End of tip-in event has been sensed

The presence of a tip-in event is sensed based on the difference between  the

background  and  foreground values of cylinder air charge.  If the difference

exceeds a threshold, a tip-in event has been sensed.

The peak rise-rate of the cylinder air charge is required to denote  when  it

is  valid to begin searching for the end of the tip-in event.  The end of the

event is sensed when the cylinder-to-cylinder difference  in  air  charge  is

smaller than some threshold.

Once the end of the tip-in event has been found, the  background  manager  is

informed  that  the  borderline spark module should be executed on a priority

basis (after the  current  background  module  is  completed).   Once  a  new

borderline  value has been calculated, it is used for the spark advance value

and the tip-in event is over.

DEFINITIONS

Registers:

-  CYL_AIR_CHG = Current cylinder air charge.

-  SPKAD = Spark adder for knock.  It is added to SAF and may be posative or

negative, degrees.

-  SPK_AIR_CHG = Cylinder air charge used to calculate load for use in spark

calculations.

9-99

IGNITION TIMING STRATEGY, TRANSIENT SPARK FOR DETONATION PREVENTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  SPK_ARCH_PRV = Cylinder air charge value for previous cylinder event.

-  SPK_BDL_CAL =  Borderline  detonation  spark  limit,  degrees,  including

effects of SPK_ADD, SPK_MUL and SPKAD modifiers.

-  SPK_BDL_CLP = Borderline detonation spark limit modified  for  change  in

cylinder  air  charge from when borderline detonation limit (SPK_BDL) was

computed.

-  SPK_DARCHMAX = Maximum rise rate of cylinder air charge during tip-in.

-  SPK_MAX_TRET = Maximum spark retard during air-charge based tip-in  knock

control.

-  SPK_TIPSTATE = Spark state transition variable during tip-in event.

Bit Flags:

-  PIP_HIGH = PIP input level.

-  SPK_BDL_UPDT = Flag to inform the background manager that the  borderline

spark module must be executed on a priority basis.

Calibration Constants:

-  SPK_ARCH_MIN = Cylinder-to-cylinder difference in air charge to sense end

of tip-in event.

-  SPK_D_AIRCHG = Minimum difference in  cylinder  air  charge  to  allow  a

tip-in retard event.

-  SPK_TIP_BDL = Multiplier to increase in aircharge, the product  of  which

is subtracted from SPK_BDL.

9-100

IGNITION TIMING STRATEGY, TRANSIENT SPARK FOR DETONATION PREVENTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SPARK_TIPIN_CONTROL_FG_COM1

BEGIN:  spark_tipin_control_fg_main

; This process executes on each rising PIP edge.  The timing relative

; to the up edge is not critical but this must be executed after the

; foreground knock control logic that sets the value of SPKAD and

; before the SPOUT/HDR modules.  Note that it reads top down.

; Execute this logic only once per cylinder event on the high PIP edge.

PIP_HIGH = 0 -------------| EXIT:  spark_tipin_control_fg_main

; Calculate the change in cylinder air charge between the background and

; foreground.  Calculate difference in cylinder air charge from value

; during last cylinder event.  SPK_AIR_CHG should be initialized to its

; maximum value to ensure that a tip-in event is not triggered when this

; module is first executed.  SPK_ARCH_PRV should be initialized to zero

; to maximize the difference in cylinder-to-cylinder air charge values

; when this module is first executed.

unconditionally ----------| d_air_chg := CYL_AIR_CHG - SPK_AIR_CHG

| d_air_chg := MAX(d_air_chg,0)

| d_cyl_air_chg := CYL_AIR_CHG - SPK_ARCH_PRV

| d_cyl_air_chg := MAX(d_cyl_air_chg,0)

| SPK_ARCH_PRV := CYL_AIR_CHG

; Determine, based on the change in cylinder air charge between the

; background and foreground, if a tip-in is occurring.  If no tip-in is

; in progress, the borderline clip value includes the knock adder so that

; spark is allowed to advance past the nominal borderline value.

d_air_chg < SPK_D_AIRCHG ---------| SPK_BDL_CLP := SPK_BDL_CAL + SPKAD

| EXIT:  spark_tipin_control_fg_main

|

| --- ELSE ---

|

SPK_TIPSTATE = 0 -----------------| SPK_TIPSTATE := 1

| SPK_DARCHMAX := 0

; Clip spark adder for knock control to a maximum advance of zero if a

; tip-in is in progress to ensure that spark is not over-advanced.

unconditionally -----------------| SPKAD := MIN(SPKAD,0)

9-101

IGNITION TIMING STRATEGY, TRANSIENT SPARK FOR DETONATION PREVENTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUE:  spark_tipin_control_fg_main

; Calculate value of SPK_BDL_CLP which is the retard from the borderline

; calculation modified by the calibration adder and multiplier.

unconditionally ----------| t_spk_bdl_clp := SPK_BDL_CAL

|                - SPK_TIP_BDL * d_air_chg

| SPK_BDL_CLP := MAX(t_spk_bdl_clp,SPK_MAX_TRET)

; Search for peak rate of increase of cylinder air charge during tip-in

; event.  Once this has been found, it is okay to search for the end of

; the tip-in event.

SPK_DARCHMAX < d_cyl_air_chg ---------| SPK_DARCHMAX := d_cyl_air_chg

|  ; Still searching for peak

|

| --- ELSE ---

|

SPK_TIPSTATE = 1 ---------------------| SPK_TIPSTATE := 2

|  ; Peak has been found

; Once the peak cylinder-to-cylinder air charge value has been found,

; determine when end of tip-in event occurs.  If the change in

; cylinder air charge has become small enough and the peak change has

; been reached, the tip-in event is determined to be over and a flag,

; SPK_BDL_UPDT, is set to inform the background manager that the

; borderline spark module must be executed on a priority basis.  This

; is to be accomplished by inserting the borderline module as the next

; background task.  When the borderline value has been calculated, the

; background manager should return to the module that was next on the

; list before being pre-empted by the borderline module.

d_cyl_air_chg <= SPK_ARCH_MIN --|

|AND -| SPK_BDL_UPDT := 1

SPK_TIPSTATE = 2 ---------------|     | SPK_TIPSTATE := 3

; If SPK_BDL_UPDT has been cleared by the borderline module, the value of

; SPK_BDL has been updated so the tip-in retard event may be completed.

; Since the tip-in event is over, the borderline clip includes the knock

; adder so that spark is allowed to advance past the nominal borderline

; value.

SPK_BDL_UPDT = 0 ---|

|AND -| SPK_BDL_CAL := (SPK_BDL * SPK_MUL) + SPK_ADD

SPK_TIPSTATE = 3 ---|     | SPK_BDL_CLP := SPK_BDL_CAL + SPKAD

| SPK_AIR_CHG := CYL_AIR_CHG

| SPK_TIPSTATE := 0

END:  spark_tipin_control_fg_main

9-102

IGNITION TIMING STRATEGY, TRANSIENT SPARK FOR DETONATION PREVENTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

9.22  TRANSIENT SPARK FOR DETONATION PREVENTION (CDAM0)

OVERVIEW

This is  the  background  portion  of  the  transient  spark  for  detonation

prevention.  It is based on cylinder air charge.

The borderline spark value is calculated  in  the  background  and  does  not

reflect  the  rapidly  changing air charge value during a tip-in event.  This

incorrect  borderline  value  means  knock  cannot  be  controlled  via   the

borderline calculation and other means must be employed.

The value of the cylinder air charge is stored in the  background  such  that

the current foreground value may be compared to it.  If the difference in the

background to foreground values is significant, it is  marked  as  a  tip-in.

The background value is not updated during a tip-in event.

DEFINITIONS

Registers:

-  CYL_AIR_CHG = Current cylinder air charge.

-  N = Engine speed (RPM).

-  SPK_AIR_CHG = Cylinder air charge used to calculate load for use in spark

calculations.

-  SPK_BDL = Borderline detonation spark limit.

-  SPK_BDL_CAL =  Borderline  detonation  spark  limit,  degrees,  including

effects of SPK_ADD, SPK_MUL and SPKAD modifiers.

-  SPK_DARCHMAX = Maximum rise rate of cylinder air charge during tip-in.

-  SPK_MAX_TRET = Maximum spark retard during air-charge based tip-in  knock

control.

-  SPK_TIPSTATE = Spark state transition variable during tip-in event.

Calibration Constants:

-  FN704(N) = Maximum spark retard  during  air-charge  based  tip-in  knock

control.

-  SPK_ADD = Calibration  adder  to  over-all  spark  equation.   Explicitly

specificy spark advance with SPK_MUL = 0.

-  SPK_MUL = Calibration multiplier to over-all spark equation.  Set to zero

and control spark with SPK_ADD.

9-103

IGNITION TIMING STRATEGY, TRANSIENT SPARK FOR DETONATION PREVENTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SPARK_TIPIN_CONTROL_BG_COM1

BEGIN:  spark_tipin_control_bg_main

; This process executes once each background loop.  It must be executed

; after the borderline spark module.  Note that it reads top-down.

; If a tip-in is not occurring (based on the change in cylinder air charge

; between the background and foreground calculations), set the value of

; air charge for this background loop.  Also set maximum amount of retard

; during a tip-in.  During a tip-in event, the value of SPK_AIR_CHG is not

; updated so that there is a reference of the air charge value when the

; tip-in event began.

; SPK_BDL_CAL does not include the knock adder since it is undesirable to

; over-retard and unsafe to over-advance.

SPK_TIPSTATE = 0 ---------| SPK_AIR_CHG := CYL_AIR_CHG

| SPK_BDL_CAL := (SPK_BDL * SPK_MUL) + SPK_ADD

| SPK_MAX_TRET := FN704(N)

| SPK_DARCHMAX := 0

END:  spark_tipin_control_bg_main

9-104

CHAPTER 10

CATALYST BYPASS CONTROL STRATEGY

10-1

CATALYST BYPASS CONTROL STRATEGY

10.1  FEATURE:  EXHAUST TEMPERATURE MODEL - V1.9_CAT_TEMP_MODEL (CDAN0)

10.1.1  EXHAUST TEMPERATURE CALCULATION (CDAN0)

OVERVIEW

The  below  algorithm  implements  the  real-time  time  exhaust  temperature

prediction  algorithm.   The predicted temperature has at least the following

uses:

1) HEGO heater control, provides input to turn on and off the HEGO heaters.

2) The open loop fuel scheduling for catalyst temperature protection.

3) Torque control via spark retard is limited by exhaust temperatures.

Variables currently accounted for include:

-  Engine speed, N.

-  Engine air charge (EEC variable LOAD).

-  Distance from MBT spark (SPK_M_B_T - SAF), or EEC register SPK_DELTA.

Note, spark is often retarded in real time to prevent engine knock.

So SPK_DELTA is a function of air charge temperature ACT and engine

coolant temperature ECT.

-  Actual percent EGR, EGRACT.

-  Desired equivalence (air/fuel) ratio SPK_LAMBSE. This is LAMBSE without

the perturbations in closed loop feedback air/fuel control mode.

-  Ratio of number of cylinders on over the total number of

cylinders, INJ_TR.  Temperatures for cylinder cutout are only accounted

for during Torque Controlled events, and is assumed that the A/F is

scheduled lean during cutout.  This model will not be accurate for

conditions with rich A/f and cylinder cutout.  A temperature of

(ACT+ECT)/2 is assumed to be the temperature of non-firing cylinders.

10-2

CATALYST BYPASS CONTROL STRATEGY, EXHAUST TEMPERATURE CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The real time exhaust calculation is used to control the front and rear HEGO

heaters.  A summary of the HEGO heater control algorithm is as follows:

Maintain a min. HEGO temperature via the heater so it properly senses the

A/F.

Subject to:

1) Keep the HEGO below its high temperature design limit by turning off

the heater at high inferred exhaust gas temperatures.  This applies

mainly to the front HEGO since the exhaust gas cools down quite a bit

before reaching the rear HEGO.

2) Delay turning on the heater until water vapor has been burned off the

sensor element, or it may crack.  This applies mainly to the rear

HEGO since it takes relatively long to warm up.

In the design of the initialization algorithm, the following was assumed:

1) For V engines the left and right front HEGOs are assumed to be at

equal temperatures.

2) For V engines the left and right rear HEGOs (if so equipped) may be

at different temperatures, but the functions defining the drop in

temperature from the catalyst midbed to the rear HEGO should be

entered for the longest (coolest) side.  No provision for unequal

length pipes were made in the strategy.

The purpose of the inferred ambient temperature is initially to support the

HEGO heater control algorithm wherein inferred ambient is used to seed the

rear HEGO temperature.  It is expect to have other applications in the

future.  The goal of the algorithm is:

Infer the ambient temperature in Deg.F. as accurately as possible upon

key-on and during engine running modes.

The inferred ambient temperature algorithm has three basic modes:

1) KEY-ON

2) RUNNING RATCH algorithm

3) RUNNING ROLLING AVERAGE algorithm

10-3

CATALYST BYPASS CONTROL STRATEGY, EXHAUST TEMPERATURE CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The KEY-ON algorithm:

1) Ambient temperature on keyon is the assigned the lowest value of

the previously stored ambient or current ACT reading.

2) If ECT is within AM_KEON_DLT (small value) of ACT on startup, it is

assumed that the engine is at equilibrium or ambient temperature.

This check is redundant to the SOAK_CAT logic, but was put back

to bulletproof the SOAK_CAT logic from being zeroed due to

key-on bounce/gitter.

The RUNNING RATCH algorithm:

1) When the engine is running the ACT sensor always reads a value

greater than or equal to ambient. No matter what the rolling

average equation predicts if ACT is less than the current

INFAMB_KAM value plus a tolerance (AMB_ACT_TOL) then INFAMB_KAM will

be ratcheted down. If the rolling average tends to runs +- 15

Deg.F. then might be very satisfied to be within 10 Deg.F. with the

certainty of the ratchet algorithm, so AMB_ACT_TOL is recommended to

be about 10 Deg.F.

The RUNNING ROLLING AVERAGE algorithm:

1) While underhood temperatures are potentially very high at idle,

the nature of the inferred ambient strategy is such that we can

exclude idle updates and wait for the vehicle to achieve a minimum

speed for updates (AMB_VS_MIN). The delta between ACT and ambient

was found to be smaller and more predictable in a moving vehicle.

It is calibrated via AMB_DELTA with values on the order of 20-50

degrees observed.  It is recommended that the average observed

delta value be used so that under very hot underhood conditions

INFAMB_KAM will be on the higher end of the 15 degree error band,

and when the underhood conditions are cooler the RUNNING RATCH

algorithm will take over.

2) The ratchet algorithm will dominate during the warm up period so

the task of the rolling average algorithm is to provide INFAMB_KAM

for fully warmed up engines. To this end the gate AMB_ECTMIN is

recommended to be set just above thermostat temperature.

3) The status the air conditioning clutch (ACCFLG=1 when engaged)

has a significant effect on underhood temperatures and is applied

as a subtractor (AMB_ACC_SUB) to INFAMB_KAM. This delta is

calibrated with the clutch on full time. The higher the duty cycle

of the A/C the greater the influence of the A/C on the rolling

average INFAMB value.

10-4

CATALYST BYPASS CONTROL STRATEGY, EXHAUST TEMPERATURE CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  ACT = Air charge temperature, degrees.

-  AM = Air mass flow, ppm.

-  ECT = Coolant temp.  deg.  faren.

-  ECTCNT = Number of times ECT sensor input was read.

-  EGRACT = Actual percent EGR.

-  EXT_CATMID = Unsigned inferred Catalyst midbed temperature, degrees.

-  EXT_CIN = Signed inferred Catalyst inlet temperature, degrees.

-  EXT_CMD = Signed inferred Catalyst midbed temperature, degrees.

-  EXT_CMS = Instantaneous catalyst midbed gas temperature at stoichiometric

A/F, Deg.F.

-  EXT_EXO_MLT =  Catalyst  exotherm  multiplier  used  when  cylinders  are

disabled.

-  EXT_EXD = Instantaneous  difference  in  catalyst  exotherm  temperatures

between current lambse and stoich A/F, degrees.

-  EXT_FEH = Inferred heat rise in front EGO tip temperature due  to  heater

on., degrees.

-  EXT_FEG = Inferred signed exhaust  tip  temperature  at  the  front  hego

sensor location (hego furthest from flange), degrees.

-  EXT_FEU = Inferred unheated tip temperature  at  the  front  HEGO  sensor

location, degrees.

-  EXT_FL  =  Instantaneous  inferred  signed  exhaust  Flange  temperature,

degrees.

-  EXT_LS_CIN = Signed steady state exhaust gas temperature drop per  degree

of potential drop between flange and catalyst inlet, deg F / deg F.

-  EXT_LS_FEU = Signed steady state exhaust gas temperature drop per  degree

of potential drop between flange and front hego tip, deg F / deg F.

-  EXT_LS_REG = Signed steady state exhaust gas temperature drop per  degree

of potential drop between catalyst midbed and rear hego, deg F / deg F.

-  EXT_REG = Inferred temperature at the Rear hEGo location, degrees.  Note,

for  a  dual  bank HEGO application, this represents the rear HEGO on the

bank having the longest path from the flange to the rear HEGO.

-  EXT_SS_CMS  =  steady  state   catalyst   midbed   gas   temperature   at

stoichiometric A/F, Deg.F.

10-5

CATALYST BYPASS CONTROL STRATEGY, EXHAUST TEMPERATURE CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  EXT_SS_EXD = Steady State signed difference in  exotherm  temperature  of

the catalyst from stoich air fuel exotherm, degrees F.

-  EXT_SS_EXO = Steady state temperature  difference  between  the  catalyst

inlet  and  catalyst  midbed;  positive  temp.   delta  due to exothermic

reation, Deg.F.

-  EXT_SS_FEH = Steady-state heat rise in front EGO tip temperature  due  to

heater on., degrees.

-  EXT_SS_FEU = Steady-state unheated tip  temperature  at  the  front  hego

sensor location.  , degrees.

-  EXT_SS_FL = Steady State signed exhaust FLange temperature, degrees.

-  EXT_SS_FLS = Steady State signed gas  exhaust  FLange  temperature  at  a

Stoich air/fuel mixture, degrees.

-  EXT_SS_REG = Steady state signed temperature at the Rear  hEGo  location,

degrees.

-  INFAMB_KAM = Inferred ambient air temperature, degrees F.

-  INJ_TR = Torque ratio obtained from turning off a number of injectors.

-  LOAD = air inducted per intake stroke (air charge) normalized by a engine

design specific constant (SARCHG).

-  N = engine speed (RPM).

-  SAF = Final spark advance in degrees.

-  SOAK_CAT = Elapsed time since the EEC was powered down until the catalyst

temperature model has been initialized, min.

-  SPK_LAMBSE = equals the equivalence ratio (EEC-IV variable  LAMBSE)  when

open loop, equals 1.0 when closed loop.

-  SPK_DELTA = Retard from MBT = SPK_M_B_T - SAF.

-  SPK_M_B_T = Spark advance required to achieve maximum brake torque.

-  TC_CNTR = Used to extend the range of the ROLAV() beyond the current  one

minute limit (extends it up to a 16 minute time constant).

-  VSBAR = Filtered vehicle speed.

Bit Flags:

-  ACCFLG = A/C clutch status 0 = disengaged, 1 = engaged.

-  AFMFLG = ACT sensor failure flag.

-  CFMFLG = ECT sensor failure flag.

-  DFSFLG = Indicates decel fuel shutoff.

10-6

CATALYST BYPASS CONTROL STRATEGY, EXHAUST TEMPERATURE CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  EXT_FMM_FLG = Inferred temperature calculations are potentially wrong due

to a sensor failure.

-  EXT_INIT = Initialization flag to indicate if  the  catalyst  temperature

model has been seeded with the correct intial conditions.

-  HEGOHTR11 = HEGO heater 11 state, 1 -> heater on.

-  KAM_ERROR = Indicates keep alive ram is invalid.

-  MFMFLG = Air meter failure flag.

-  UNDSP = Run/Underspeed Flag; 1 -> Underspeed (or CRANK), 0 -> Run.

Calibration Constants:

-  AMB_ACC_SUB = Adjustment needed when calculating the ambient due  to  the

increase in underhood temepratures from the A/C (which increases ACT).

-  AMB_ACT_TOL = If the ACT is less than the current  INFAMB_KAM  plus  this

tolerance value, INFAMB_KAM is allowed to be rached down.

-  AMB_DELTA = Average observed temperature delta (ACT from ambient).

-  AMB_ECT_MIN = Minimum ECT temperature to enable using the ACT sensor when

calculating the ambient temperature.

-  AMB_KEON_DLT = A delta temperature used on key-ons between  ACT  and  ECT

below  which  you  can  conclude  that the car has been sitting off for a

while, degree.

-  AMB_VS_MIN = Minimum vehicle speed to enable using the  ACT  sensor  when

calculating the ambient temperature.

-  EXT_EXO_CL = Temperature below which exothermic reations in the midbed do

not occur.

-  EXT_EXO_SH = Midbed temperature above which exothermic reations occur.

-  EXT_FEH_INT = Intercept of linear equation describing temperature rise of

front  EGO  tip  due  to heater on versus the temperature of the tip with

heater off, Deg.F.

-  EXT_FEH_SLP = Slope of linear equation  describing  temperature  rise  of

front  EGO  tip  due  to heater on versus the temperature of the tip with

heater off, Deg.F.

-  FN4441(N,LOAD) = Base  steady-state  exhaust  flange  temperature  versus

speed and LOAD at 14.6 A/F, 0% EGR, MBT spark, degrees.

X = F070C(N)

Y = FN034A(LOAD)

-  FN441A(SPK_LAMBSE)  =  Effect  of  LAMBSE  on  inferred  exhaust   flange

temperature, multiplier.

-  FN441B(SPK_DELTA) = Effect of SPK on inferred exhaust flange temperature,

multiplier.

10-7

CATALYST BYPASS CONTROL STRATEGY, EXHAUST TEMPERATURE CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FN441C(EGRACT) = Effect of EGR on inferred  exhaust  flange  temperature,

multiplier.

-  FN441T(AM) = Exhaust temperature modifier for eninge coolant temperature,

degrees exhaust flange temperature per degees engine coolant temperature.

-  FN442(AM) = Time constant for instantaneous  exhaust  flange  temperature

versus airmass, seconds.

-  FN443(AM) = Time constant for front hego unheated tip temperature  versus

airmass, seconds.

-  FN443L(AM) = Temperature loss per degree of potential loss between flange

and front hego unheated tip versus airmass, Deg.F./Deg.F.

-  FN445L(AM) = Temperature loss per degree of potential loss between flange

and catmid location versus airmass, Deg.F./Deg.F.

-  FN448(AM) = Multiplier of  FN448A(SPK_LAMBSE)  to  reflect  the  loss  of

catalyst efficiency at high space velocities.

-  FN448A(SPK_LAMBSE) = Steady-state increase in exhaust temperature in  the

catalyst , the "exotherm", as a function of SPK_LAMBSE, in degrees.

-  FN449(AM) = Time constant of the catalyst substrate as a function of  AM,

seconds.

-  FN449A(AM) = Time constant for the change in catalyst exotherm with  A/F,

seconds.

-  FN450(AM)  =  Time  constant  for  instantaneous   rear   HEGO   location

temperature versus airmass, seconds.

-  FN450L(AM) = Temperature loss per degree of potential loss between catmid

and rear HEGO location versus airmass, Deg.F./Deg.F.

-  FNEXP(x) = Exponential function, e raised to the x.

-  TC_DFSO_FL = Time constant for for the change in flange temp.   when  all

cylinders are off (decel fuel shut off), seconds.

-  TC_INFAMB = Inferred ambient air temperature filter constant.

-  TC_RUN_FEH = Time constant during engine running for the  rise  in  front

hego tip temperature due to the heater, sec.

-  TC_SOAK_CMS = Time constant during soak time for the catmid, sec.

-  TC_SOAK_FEH = Time constant during soak time for the  front  hego  heater

contribution temperature, sec.

-  TC_SOAK_FEU = Time constant during soak time for the unheated front  hego

tip temperature, sec.

-  TC_SOAK_FL = Time constant during soak time for the flange, sec.

-  TC_SOAK_REG = Time constant during soak time for the rear hego, sec.

10-8

CATALYST BYPASS CONTROL STRATEGY, EXHAUST TEMPERATURE CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CAT_TEMP_COM1

AFMFLG = 1 -----------------|

;ACT failure               |

|

MFMFLG = 1 -----------------|OR --| EXT_FMM_FLG = 1

;Air meter failure         |     |

|     |

CFMFLG = 1 -----------------|     |

;ECT falilure                    |

| --- ELSE ---

|

| EXT_FMM_FLG = 0

Setup a counter to extend the range of the ROLAV(), allowing time

constants greater than one minute.

TC_CNTR <= 1 ---------------------| TC_CNTR = 16

|

| --- ELSE ---

|

| TC_CNTR = TC_CNTR - 1

Catalyst Temperature Calling Routine.

KAM_ERROR = 0 --------------|

|AND -| ;no action

ECTCNT < 8 -----------------|     |

;reading sensors still suspect   |

| --- ELSE ---

KAM_ERROR = 1 --------------|     |

;stored values corrupt     |OR --| Do: initialize_cat_model

|     |

EXT_INIT = 0 ---------------|     |

;model not initialized           |

| --- ELSE ---

|

UNDSP = 0 ------------------------| Do: cat_temp_model

;engine is running               | Do: inferred_ambient

|

| --- ELSE ---

|

| Do: inferred_ambient

10-9

CATALYST BYPASS CONTROL STRATEGY, EXHAUST TEMPERATURE CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: initialize_cat_model

Calculate the initial conditions (temps) prior to engine running.

abs(ACT - ECT) <

AMB_KEON_DLT ---|

|OR --| INFAMB_KAM = ACT

KAM_ERROR = 1 -------|     | EXT_SS_FL = ECT

;stored values corrupt    | EXT_FL = ECT

| EXT_FEU = ECT

| EXT_FEH = 0

| EXT_CMS = ACT

| EXT_REG = ACT

| SOAK_CAT = 0.0

| EXT_INIT = 1

|

| --- ELSE ---

|

| INFAMB_KAM = min(ACT,INFAMB_KAM)

| texp = FNEXP(-SOAK_CAT / TC_SOAK_FL)

| EXT_FL = ((EXT_FL - INFAMB_KAM) *

|              texp ) + INFAMB_KAM

| EXT_SS_FL = EXT_FL

| texp = FNEXP(-SOAK_CAT / TC_SOAK_FEH)

| EXT_FEH = EXT_FEH  * texp

| texp = FNEXP(-SOAK_CAT / TC_SOAK_FEU)

| EXT_FEU = ((EXT_FEU - INFAMB_KAM) *

|               texp ) + INFAMB_KAM

| texp = FNEXP(-SOAK_CAT / TC_SOAK_CMS)

| EXT_CMS = ((EXT_CMS - INFAMB_KAM) *

|               texp ) + INFAMB_KAM

| texp = FNEXP(-SOAK_CAT / TC_SOAK_REG)

| EXT_REG = ((EXT_REG - INFAMB_KAM) *

|               texp ) + INFAMB_KAM

| SOAK_CAT = 0.0

| EXT_INIT = 1

END: initialize_cat_model

10-10

CATALYST BYPASS CONTROL STRATEGY, EXHAUST TEMPERATURE CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: cat_temp_model

Calculate the steady-state exhaust flange gas temperature for speed,

LOAD, spark, EGR and air/fuel, number of cylinders on.

unconditionally ----------------| SPK_DELTA = SPK_M_B_T - SAF

| EXT_SS_FLS = [((FN4441(N,LOAD) *

|   FN441B(SPK_DELTA) * FN441C(EGRACT)) +

|   FN441T(AM)*(ECT-200) ) * INJ_TR ] +

|   ((1 - INJ_TR) * (ACT + ECT)/2 )

| EXT_SS_FL = EXT_SS_FLS *

|   FN441A(SPK_LAMBSE)

Calculate the gas temperature at the flange.

DFSFLG = 0 ---------------------| EXT_FL = ROLAV(EXT_SS_FL, FN442(AM))

;not in decel shutoff          |

| --- ELSE ---

|

TC_CNTR <= 1 -------------------| EXT_FL = ROLAV(EXT_SS_FL, TC_DFSO_FL)

|

| --- ELSE ---

|

| no change to EXT_FL

Calculate the temperature at the front unheated tip temperature (EXT_FEU).

TC_CNTR <= 1 -------------------| EXT_LS_FEU = FN443L(AM) *

|   (EXT_FEU(old) - INFAMB_KAM)

| EXT_SS_FEU = EXT_FL - EXT_LS_FEU

| EXT_FEU = ROLAV(EXT_SS_FEU,FN443(AM))

HEGOHTR11 = 1 ------------|

;front heater on         |AND -| EXT_SS_FEH = EXT_FEH_INT -

|     |   (EXT_FEH_SLP * EXT_FEU)

TC_CNTR <= 1 -------------|     |

| --- ELSE ---

|

| EXT_SS_FEH = 0.0

TC_CNTR <= 1 -------------------| EXT_FEH = ROLAV(EXT_SS_FEH,TC_RUN_FEH)

| EXT_FEG = EXT_FEH + EXT_FEU

Calculate the steady-state temperature drop in exhaust gas traveling from

the flange to the catalyst inlet and calculate the catalyst inlet temp.

TC_CNTR <= 1 -------------------| EXT_LS_CIN = FN445L(AM) * ((EXT_FL +

|   EXT_CIN(old))/2 - INFAMB_KAM )

| EXT_CIN = EXT_FL - EXT_LS_CIN

10-11

CATALYST BYPASS CONTROL STRATEGY, EXHAUST TEMPERATURE CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Determine if the cataylst is lit off.

EXT_CMD < EXT_EXO_CL -------------| EXT_EXO_MLT = 0

|

| --- ELSE ---

TC_CNTR <= 1 ---------------|     |

|AND -| EXT_EXO_MLT = FN448(AM) * INJ_TR

EXT_CMD > EXT_EXO_SH -------|     |

| --- ELSE ---

|

| no change to EXT_EXO_MLT

Calculate the steady state exotherm temperature needed/used by the open

loop fuel A/F controller.

unconditionally ----------------| EXT_SS_EXO = EXT_EXO_MLT *

|    FN448A(SPK_LAMBSE)

Calculate a midbed temperature as if at stoic, an exotherm difference

from stoic and actual A/F, then the total (sum) is the catmid temperature.

TC_CNTR <= 1 -------------------| EXT_SS_EXD = ( FN448A(SPK_LAMBSE) -

|    FN448A(1) ) * EXT_EXO_MLT

| ;steady-state temp. diff. in exotherm from

| ;running stoic and running current a/f.

| EXT_SS_CMS = (FN448A(1) *

|    EXT_EXO_MLT ) + EXT_CIN

| ;steady-state cat midbed temp at stoic.

| EXT_EXD = ROLAV(EXT_SS_EXD,FN449A(AM))

| EXT_CMS = ROLAV(EXT_SS_CMS,FN449(AM))

| EXT_CMD = EXT_CMS + EXT_EXD

| ;add stoich value and difference to form

| ;catalyst midbed temp

| EXT_CATMID = max(EXT_CMD,0)

10-12

CATALYST BYPASS CONTROL STRATEGY, EXHAUST TEMPERATURE CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calculate the steady-state temperature drop in exhaust gas traveling from

the catalyst midbed to the rear hego location.

TC_CNTR <= 1 ---------------------| EXT_LS_REG = FN450L(AM) * ((EXT_CMD +

|   EXT_REG(old))/2 - INFAMB_KAM)

Calculate the steady-state drop in exhaust gas temperature.

TC_CNTR <= 1 ---------------------| EXT_SS_REG = EXT_CMD - EXT_LS_REG

Calculate the current wall temperature at the rear HEGOx2 location.

TC_CNTR <= 1 ---------------------| EXT_REG = ROLAV(EXT_SS_REG,FN450(AM))

END: cat_temp_model

BEGIN: inferred_ambient

Account for the A/C before calculating the ambient.

ACCFLG = 1 -----------------------| amb_adj = AMB_ACC_SUB

|

| --- ELSE ---

|

| amb_adj = 0

Store a value of inferred ambient air temperature for initializing the

model on the next restart.

ECT > AMB_ECT_MIN ----------|

|

VSBAR > AMB_VS_MIN ---------|AND -| ambient = ACT - AMB_DELTA - amb_adj

|     | INFAMB_KAM = ROLAV(ambient,TC_INFAMB)

TC_CNTR <= 1 ---------------|     |

| --- ELSE ---

|

| no change to INFAMB_KAM

Allow the inferred ambient to be ratched down.

ACT - AMB_ACT_TOL < INFAMB_KAM ---| INFAMB_KAM = ACT - AMB_ACT_TOL

|

| --- ELSE ---

|

| no change to INFAMB_KAM

END: inferred_ambient

10-13

INPUT AND FAILURE MODE AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

10.1.2  SOAK TIME CALCULATION (CDAN0)

OVERVIEW

This logic uses the hardware present on the AICE2 chip to determine EEC power

off time.

DEFINITIONS

Registers:

-  ENG_OFF_TMR =  Elapsed  time  of  the  most  recent  EEC  module  key-off

powered-down period, minutes.

-  SOAK_HICOMP = Soak time duration in minutes.  This copy is used  only  by

the Hot Injector Compensation logic.

-  SOAK_CAT = Soak time duration in minutes.  This copy is used only by  the

exhaust temperature model logic.

PROCESS

STRATEGY MODULE: INPUT_SOAK_TIMER_COM2

BEGIN:  input_soak_timer

;Execute the following process once during RAM initialization.

unconditonally ------------------| ;read ENG_OFF_TMR

| SOAK_HICOMP = SOAK_HICOMP + ENG_OFF_TMR

| SOAK_CAT = SOAK_CAT + ENG_OFF_TMR

| ENG_OFF_TMR = 0.0

END: input_soak_timer

10-14

INPUT AND FAILURE MODE AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

10.2  FEATURE:  CATALYST_MONITOR (STANDARD) - V1.8A_CAT_FTP_MONITOR (CDAN0)

10.2.1  FTP CATALYST MONITOR (CDAN0)

OVERVIEW

The Catalyst Monitor is an algorithm designed to detect when the Catalsyt  is

displaying  poor emission conversion efficiency.  The detection capability is

based on the premis that the ratio  of  rear  HEGO  switches  to  front  HEGO

switches  is  related  directly  to  Catalyst  efficieny.   As  the  Catalyst

efficiency reduces the ratio of REAR to FRONT HEGO switches  increases.   The

monitor  collects  HEGO switch data and after a sufficient number of switches

are collected it calculates the REAR/FRONT  switch  ratio.   When  the  ratio

exceeds  a  pre-determined  value the Catalyst considered to be exceeding the

threshold emissions limit.  This sets a Catalyst failed flag which is used by

the OBDII executive to light the MIL.

The Catalyst Monitor is divided into several modules to simplify the  control

structure.  There is a main control module that contains the entry conditions

for the monitor as well as calling the other modules that  are  necessary  to

run  the  module.   The  entry  conditions to run the monitor include coolant

temperature, closed loop fuel control, midbed catalyst temperature, and being

enable by the OBDII executive.  If these conditions are met, the monitor runs

and the other modules are called.

The other modules are used to perform the following functions;

The ftp_codes module:- reports fault codes if they are requested and the test

has been completed.

The ftp_reset_bank modules:- reset the  counters  for  front  and  rear  HEGO

switches when a test has been completed or a reset is demanded.

The ftp_trackingsp_bank module:- processes the  raw  HEGO  voltage  data  and

generates switch flags that correspond to HEGO voltage swings/switches.  This

module detects voltage  maxima-minima  and  corresponding  voltage  amplitdue

changes.   This  data  is used to confirm that a true HEGO voltage switch has

occured.

The ftp_filter module:- filters the HEGO switches to ensure  that  HEGO  Buzz

does not intefere with the test.

The ftp_am_cell module:- sets up an air mass  cell  array  and  indicates  in

which cell of the array the test is currently operating.

The ftp_switch_bank modules:- count the front and rear  HEGO  switches.   The

switches  are  stored  in different air mass cell registers.  These registers

are used to ensure that sufficient number of switches have been collected  in

all  air  mass  cells before a trip is considered complete and a switch ratio

calculated.

The ftp_ratio_bank modules:- Determines that a suffiecient number of switches

have  been  collected  using  the TIER1/2 test.  It then take a rear to front

ratio of the HEGO switches and indicates whether or not the catalyst is  good

or bad based on that ratio.

10-15

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

+-------------+

| ftp_monitor | ; main module

+-------------+

|

|

|                +-------------+

|--------------> |  ftp_codes  | ; set and clear codes

|                +-------------+

|

|                +-----------------+

|--------------> | ftp_reset_bank1 | ; reset counters

|                +-----------------+

|

|                +-----------------+

|--------------> | ftp_reset_bank2 | ; reset counters

|                +-----------------+

|

|                +----------------------+

|--------------> | ftp_trackingsp_bank1 | ; Tracking Switchpoint

|                +----------------------+   Algorithm

|

|

|                +----------------------+

|--------------> | ftp_trackingsp_bank2 | ; Tracking Switchpoint

|                +----------------------+   Algorithm

|

|                +-------------+

|--------------> | ftp_filter  | ; filter HEGO switches

|                +-------------+

|

|                +-------------+

|--------------> | ftp_am_cell | ; select air mass cell

|                +-------------+

|

|                +------------------+

|--------------> | ftp_switch_bank1 | ; count switches

|                +------------------+

|

|                +------------------+

|--------------> | ftp_switch_bank2 | ; count switches

|                +------------------+

|

|                +-----------------+

|--------------> | ftp_ratio_bank1 |  ; rear to front switch ratio

|                +-----------------+

|

|                +-----------------+

|--------------> | ftp_ratio_bank2 |  ; rear to front switch ratio

+-----------------+

10-16

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  ACT = Air charge temperature, degrees F.

-  CAT_ACT_MAX = Maximum ACT for entry into Catalyst Monitor.

-  CAT_ACT_MIN = Minimum ACT for entry into Catalyst Monitor.

-  AM = Air Mass.

-  AM_CELL = Ftp Catalyst Monitor air mass cell identifier.

-  APT = At Part Throttle flag.

-  ATMR1_HI_RES = Time since engine start with higher resolution than ATMR1.

-  BG_TMR = Background loop time.

-  CAT_ACT_TF1 = Actuated timer flag associated with timer CAT_TMR_ACT.

-  CAT_SW_AMP12 = TSP maxima-minima amplitude.

-  CAT_SW_AMP21 = TSP maxima-minima amplitude.

-  CAT_SW_AMP22 = TSP maxima-minima amplitude.

-  CAT_TMR_ACT = Time spent collecting HEGO switch data.

-  CAT_TP_LAST = Last Back Ground value of Throttle Position.

-  CAT_TP_RATE = Throttle rate.

-  CAT_TST_TMR = Time since start of test, sec.

-  DELTA_VEGO12 = The voltage difference between two succesive HEGO  voltage

samples.

-  DELTA_VEGO22 = The voltage difference between two succesive HEGO  voltage

samples.

-  ECT = Engine Coolant Temperature.

-  EGO1_RF_AVE = Rear to front EGO1 average switch ratio.

-  EGO2_RF_AVE = Rear to front EGO2 average switch ratio.

-  EGO1_RF_RAT = Rear to front EGO1 switch ratio.

-  EGO2_RF_RAT = Rear to front EGO2 switch ratio.

-  EXT_CATMID = Midbed catalyst temperature.

-  FEGO1_CT1 THROUGH FEGO1_CT9 = Front EGO1 switch count cells.

-  FEGO1_CT21 = Bank 1 first air mass cell front hego alternative count.

10-17

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FEGO1_CT22 = Bank 1 second air mass cell front hego alternative count.

-  FEGO1_CT23 = Bank 1 third air mass cell front.

-  FEGO1_CT24 = Bank 1 fourth air mass cell front.

-  FEGO1_CT25 = Bank 1 fifth air mass cell front.

-  FEGO1_CT26 = Bank 1 sixth air mass cell front.

-  FEGO1_CT27 = Bank 1 seventh air mass cell front.

-  FEGO1_CT28 = Bank 1 eigth air mass cell front.

-  FEGO1_CT29 = Bank 1 ninth air mass cell front.

-  FEGO2_CT1 THROUGH FEGO2_CT9 = Front EGO2 switch count cells.

-  FEGO2_CT21 = Bank 2 first air mass cell front h.

-  FEGO2_CT22 = Bank 2 second air mass cell front he.

-  FEGO2_CT23 = Bank 2 third air mass cell front.

-  FEGO2_CT24 = Bank 2 fourth air mass cell front.

-  FEGO2_CT25 = Bank2 5th cell alt count.

-  FEGO2_CT26 = Bank2 sixth cell alt count.

-  FEGO2_CT27 = Bank2 seventh cell alt count.

-  FEGO2_CT28 = Bank2 8th cell alt count.

-  FEGO2_CT29 = Bank2 9th cell alt count.

-  FEGO1_CT_TOT = Total front EGO1 switches (sum of all switch count cells).

-  FEGO2_CT_TOT = Total front EGO2 switches (sum of all switch count cells).

-  KAM_ERROR = Indicates keep alive RAM invalid.

-  KWUCTR = Warmup counter for adaptive learning.

-  LOAD = Universal load as ratio of air charge over standard, unitless.

-  OBD_RCLK_DIF = Time since last EGO conversion in seconds.

-  OBD_RCLK_LST = Last clock ticks.

-  OBD_SW11_DUR = Time since last front EGO1 switch.

-  OBD_SW12_DUR = Time since last rear EGO1 switch.

-  OBD_SW21_DUR = Time since last front EGO2 switch.

-  OBD_SW22_DUR = Time since last rear EGO2 switch.

10-18

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PTTMR = Time since exiting closed throttle.

-  QD_CL_TMR2 = Time for catalyst test since entering run mode.

-  REGO1_CT = Total rear EGO1 switches.

-  REGO1_CT2 = Bank 1 total rear ego switches, alternative count.

-  REGO2_CT = Total rear EGO2 switches.

-  REGO2_CT2 = Bank 2 total rear ego switches, alternative count.

-  TP = Throttle position.

-  TP_REL = RElative TP (TP-RATCH).

-  TSP_MXMN_V12 = TSP HEGO voltage amplitude  values  during  maxima/minima,

volts.

-  TSP_MXMN_V22 = TSP HEGO voltage amplitude  values  during  maxima/minima,

volts.

-  UNDSP = Underspeed flag.

-  VEGO11 = Bank1 upstream HEGO voltage.

-  VEGO12 = Rear bank-1 EGO voltage for catalyst tests.

-  VEGO21 = Bank2 upstream HEGO voltage.

-  VEGO22 = Rear bank-2 EGO voltage for catalyst tests.

-  VEGO22 = TSP HEGO voltage input -new value bank 2 rear.

-  VEGO_OLD11 = TSP HEGO voltage input -old value.

-  VEGO_OLD12 = TSP HEGO voltage input -old value bank1, rear.

-  VEGO_OLD21 = TSP HEGO voltage input - old value bank2, front.

-  VEGO_OLD22 = TSP HEGO voltage input -old value bank2 rear.

-  VEGO_SLOPE12 = Flag to indicate the gradient sign of the bank1, rear HEGO

voltage; 1 -> Positive gradient, 0 -> Negative gradient.

-  VEGO_SLOPE22 = Flag to indicate the gradient sign of the bank2, rear HEGO

voltage; 1 -> Positive gradient, 0 -> Negative gradient.

Bit Flags:

-  CAT_AQU_LS1 = Cat-1 data acquisition start flag.

-  CAT_AQU_LS2 = Cat-2 data acquisition start flag.

-  CAT_DN1_FLG = Catalyst monitor completed on Bank 1.

-  CAT_DN2_FLG = Catalyst monitor completed on Bank 2.

10-19

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  CAT1_FAILED = Bank 1 catalyst failed indicator.

-  CAT2_FAILED = Bank 2 catalyst failed indicator.

-  CAT_MON = Monitor flag for catalyst test; 1 -> monitored at least once.

-  CAT_SWTFL11 = Front EGO bank 1 switch flag, filtered.

-  CAT_SWTFL12 = Rear EGO bank 1 switch flag, filtered.

-  CAT_SWTFL21 = Front EGO bank 2 switch flag, filtered.

-  CAT_SWTFL22 = Rear EGO bank 2 switch flag, filtered.

-  CAT_TST_PRG = Catalyst Test in Progress Flag.

-  CAT_TST_ENA = Catalyst Monitor enable by executive.

-  CAT_TST_RDY = Data ready to be reported by the test.

-  CCM_TST_ENA = Continuous component monitor flag.

-  GRAD_CHNG12 = TSP HEGO voltage gradient change has occurred - HEGO12.

-  GRAD_CHNG22 = TSP HEGO voltage gradient change has occurred - HEGO22.

-  LAM_MOD_FLG = Flag used to initiate high frequency  fuel  modulation  for

the upstream EGO monitor.

-  MIN_FLG_BK1 = Minimum number of counts in all air mass cells.

-  MIN_FLG_BK2 = Minimum number of counts in all air mass cells.

-  OBDII_RESET = Test reset by executive.

-  OBD_PARM_RST = OBDII parameter reset signal; 1-> reset parameters.

-  OLFLG = Open Loop Flag.

-  P0420MALF = Fault code, Bank-1 catalyst failed.

-  P0430MALF = Fault code, Bank-2 catalyst failed.

-  SAIR_TST_ENA = OBD-II Secondary Air Test enable flag; 1 -> test enabled.

-  SWTFL11 = Front EGO bank 1 switch flag, unfiltered.

-  SWTFL21 = Front EGO bank 2 switch flag, unfiltered.

-  TST_SW_FLG12 = TSP rear EGO bank 1 switch flag, unfiltered.

-  TST_SW_FLG22 = TSP rear EGO bank 2 switch flag, unfiltered.

Calibration Constants:

-  AM1_MIN through AM9_MIN = Minimum value to enter air mass windows.

10-20

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  AM1_MAX through AM9_MAX = Maximum value to exit air mass windows.

-  CAT_AMPCAL12 = Minimum voltage difference to counts VOLTS.

-  CAT_AMPCAL22 = Minimum voltage difference to count HEGO switches.

-  CAT_APT_FLG = Calibration to prevent counting of EGO switches when APT  =

-1.

-  CAT_COLD_TME = Catalyst warm up time if temperature at start is low.

-  CAT_CTPT_TME = Time after leaving closed throttle before Catalyst Monitor

will collect data.

-  CAT_ECT_MAX = Maximum ECT to exit catalyst monitor.

-  CAT_ECT_MIN = Minimum ECT to enter catalyst monitor.

-  CAT_HOT_TME = Catalyst warm-up time when temp at start is a high value.

-  CAT_KWU_CNT =  KAM  warm  up  value  above/below  which  catalyst  filter

contstant is scheduled, unitless.

-  CAT_LOW_LOAD = Low load to deactivate catalyst monitor, unitless.

-  CAT_LOW_TP =  Low  throttle  position  to  deactivate  catalyst  monitor,

unitless

-  CAT_MAX_MID = Maximum midbed temperature to run test.

-  CAT_MED_TME = Catalyst  warm-up  time  if  temp  at  start  is  a  medium

temperature.

-  CAT_MIN_MID = Minimum midbed temperature to run test.

-  CAT_TIER1 = Catalyst Monitor timer value used to  complete  the  Catalyst

Monitor time out value, seconds.

-  CAT_TM_CL = Minimum time since closed loop to enter catalyst monitor.

-  CAT_TP_MAX = Maximum rate for the throttle to perform test.

-  CAT_TP_TC = Time constant to filter throttle rate.

-  CAT_TM_STRT = Minimum time since start to enter catalyst monitor.

-  EGO1_FL_RAT = Value at which Bank 1 catalyst is considered failed.

-  EGO2_FL_RAT = Value at which Bank 2 catalyst is considered failed.

-  EGO_CAT_FKA = Filter constant for catalyst test, used for initial average

calculation, unitless.

-  EGO_CAT_FKB = Filter constant for  catalyst  test,  mature  calculations,

unitless.

-  EGO_INIT_RF1 = Initial Rear/front EGO switch ratio - bank 1.

10-21

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  EGO_INIT_RF2 = Initial Rear/front EGO switch ratio - bank 2.

-  EGO_RF_DEL = Rear/front EGO switch ratio delta.

-  FEGO1_C_MIN1 through FEGO1_C_MIN9 = Minimim ego  switches  for  air  mass

cells 1-9 :- bank1.

-  FEGO1_MX_CT1 through FEGO1_MX_CT9 = Maximum EGO1 switch count.

-  FEGO1_MX_TOT = Total EGO1 switch counts.

-  FEGO2_MX_CT1 through FEGO2_MX_CT9 = Maximum EGO2 switch count.

-  FEGO2_MX_TOT = Total EGO2 switch counts.

-  HEGO_CONFIG = HEGO configuration register.

-  NUMEGO = Indicates the number of upstream EGOs.

-  OBD_MINPER = Minimum half-period for ego buzz filter in CATALYST MONITOR.

-  TSP_DBAND1 = Cat Monitor TSP voltage deadband (bank1) value, volts.

-  TSP_DBAND2 = Cat Monitor TSP voltage deadband (bank2) value, volts.

OTHER

-  malfunction = defined routine to store trouble codes.

-  clear_malf = defined routine to clear trouble codes.

10-22

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CAT_FTP_STANDARD_MONITOR

BEGIN: ftp_monitor

;This module is executed once every background loop.

;KAM reset logic.

KAM_ERROR = 1 --------------------------| EGO1_RF_AVE := EGO_INIT_RF1

;KAM has been corrupted                | EGO2_RF_AVE := EGO_INIT_RF2

| ;reset averages

;Timer logic

UNDSP = 1 ------------------------------| CAT_TST_TMR := 0

; Underspeed                            | ;free running timer

| ;time since engine start

OLFLG = 1 ------------------------------| QD_CL_TMR2 := 0

;in open loop                          | ;free running timer

| ;time in closed loop

;Try to report malfunctions

;Perform Filtering operation

APT = -1 -------------------------------| PTTMR := 0

| ;free running timer

| ;time since leaving closed loop

unconditionally ------------------------| Do: ftp_codes

| Do: ftp_trackingsp_bank1

| Do: ftp_trackingsp_bank2

| Do: ftp_filter

; try to report codes, start

; tracking immediately.

;Calculate throttle angle

unconditionally ------------------------| tp_rate := ABS[(TP -

|        CAT_TP_LAST)*1/9.57)]/BG_TMR

|  ;take absolute value of

|  ;throttle rate

| CAT_TP_RATE :=

|       ROLAV_TC(tp_rate,CAT_TP_TC)

|  ;calculate throttle rate

| CAT_TP_LAST := TP

|  ;store throttle postion

10-23

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;Entry conditions for catalyst monitor.

CAT_TST_TMR > CAT_TM_STRT -----------------------|

;Time since start exceeded X sec.                |

|

TCSTRT < ECTCOLD --------------------|           |

|AND -|     |

ATMR1_HI_RES > CAT_COLD_TME ---------|     |     |

|     |

ECTCOLD < TCSTRT < ECTHOT -----------|     |     |

|AND -|OR --|

ATMR1_HI_RES > CAT_MED_TME ----------|     |     |

|     |

TCSTRT >= ECTHOT --------------------|     |     |

|AND -|     |

ATMR1_HI_RES > CAT_HOT_TME ----------|           |

;special temperature stabalization logic         |AND -| CAT_TST_RDY := 1

|     | ;test can be run

ECT > CAT_ECT_MIN -------------------------------|     |

;coolant warm enough for test                    |     |

|     |

ECT < CAT_ECT_MAX -------------------------------|     |

;coolant cool enough for test                    |     |

|     |

QD_CL_TMR2 > CAT_TM_CL2 -------------------------|     |

;Catalyst warmed up                              |     |

|     |

EXT_CATMID < CAT_MAX_MID ------------------------|     |

;catalyst midbed temp low enough                 |     |

|     |

EXT_CATMID > CAT_MIN_MID ------------------------|     |

;catalyst midbed temp high enough                |     |

|     |

ACT > CAT_ACT_MIN -------------------------|     |     |

|AND--|     |

ACT <= CAT_ACT_MAX ------------------------|           |

|     |

LOAD  >= CAT_LOW_LOAD ---------------------|     |     |

|AND -|     |

TP_REL >= CAT_LOW_TP ----------------------|           |

| --- ELSE ---

|

| CAT_TST_RDY := 0

| ;test not ready

10-24

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

LAM_MOD_FLG <> 1 -----------------|

|

SAIR_TST_ENA = 0 -----------------|

|

OBDII_RESET = 0 ------------------|

;no reset from exec               |

|

CAT_TST_ENA = 1 ------------------|

;Continuous monitor               |

|AND -| CAT_TST_PRG := 1

CAT_TST_RDY = 1 ------------------|     |  ;Test in progress

;test ready                       |     | Do: ftp_am_cell

|     | Do: ftp_switch_bank1

CAT_MON = 0 ----------------------|     | Do: ftp_switch_bank2

| Do: ftp_ratio_bank1

| Do: ftp_ratio_bank2

|

| CAT_ACT_TF1 = 1

| ; actuate CAT_TMR_ACT

|

| --- ELSE ---

OBD_PARM_RST = 1 -----------------|     |

;reset from exec                  |     |

|     | CAT_TST_PRG := 0

CAT_TST_ENA = 0 ------------|     |     | ;test aborted/reset

|AND -|OR --|

CAT_TST_RDY = 1 ------------|     |     |

|     | Do: ftp_reset_bank1

CAT_MON = 1 ----------------------|     | ;reset bank1 counters

| Do: ftp_reset_bank2

| ;reset bank2 counters

| CAT_DN1_FLG := 0

| CAT_DN2_FLG := 0

| ;reset done flags

|

| CAT_ACT_TF1 = 0

| CAT_TMR_ACT = 0

| ;reset timer

|

| --- ELSE ---

|

| CAT_TST_PRG := 0

| ;test suspended

|

| CAT_ACT_TF1 = 0

| ;Suspend timer CAT_TMR_ACT

END: ftp_monitor

10-25

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ftp_reset_bank1

;This module is executed only when explicitly called

;reset counter for bank1

unconditionally ------------------------| FEGO1_CT1 := 0

| FEGO1_CT2 := 0

| FEGO1_CT3 := 0

| FEGO1_CT4 := 0

| FEGO1_CT5 := 0

| FEGO1_CT6 := 0

| FEGO1_CT7 := 0

| FEGO1_CT8 := 0

| FEGO1_CT9 := 0

| REGO1_CT := 0

| FEGO1_C2_TOT := 0

| FEGO1_CT_TOT := 0

| REGO1_CT := 0

| REGO1_CT2 := 0

| FEGO1_CT21 := 0

| FEGO1_CT22 := 0

| FEGO1_CT23 := 0

| FEGO1_CT24 := 0

| FEGO1_CT25 := 0

| FEGO1_CT26 := 0

| FEGO1_CT27 := 0

| FEGO1_CT28 := 0

| FEGO1_CT29 := 0

END: ftp_reset_bank1

10-26

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ftp_reset_bank2

;This module is executed only when explicitly called

;reset counter for bank2

unconditionally ------------------------| FEGO2_CT1 := 0

| FEGO2_CT2 := 0

| FEGO2_CT3 := 0

| FEGO2_CT4 := 0

| FEGO2_CT5 := 0

| FEGO2_CT6 := 0

| FEGO2_CT7 := 0

| FEGO2_CT8 := 0

| FEGO2_CT9 := 0

| FEGO2_CT_TOT := 0

| REGO2_CT := 0

| FEGO2_C2_TOT := 0

| FEGO2_CT_TOT := 0

| REGO2_CT := 0

| REGO2_CT2 := 0

| FEGO2_CT21 := 0

| FEGO2_CT22 := 0

| FEGO2_CT23 := 0

| FEGO2_CT24 := 0

| FEGO2_CT25 := 0

| FEGO2_CT26 := 0

| FEGO2_CT27 := 0

| FEGO2_CT28 := 0

| FEGO2_CT29 := 0

END: ftp_reset_bank2

10-27

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ftp_tracking_bank1

;This module executes a tracking switchpoint algorithm. This algorithm will

;effectively detrend the HEGO input voltage data i.e. it will count HEGO

;voltage switches that are superimposed upon a floating DC voltage. The

;algorithm detects a change in gradient sof the hego voltage signal. Upon

;detecting a gradient change it then compares the hego voltages between this

;gradient change and the last detected change. If the voltage is

;sufficiently high it recognizes that a switch has occurred.

unconditionally ------------------------|  slope_temp = VEGO_SLOPE12

|  DELTA_VEGO12 =

|        (VEGO12 - VEGO_OLD12)

|  CAT_SW_AMP12 =

|  ABS(VEGO12 - TSP_MXMN_V12)

ABS(DELTA_VEGO12) < TSP_DBAND1 ---------| exit this routine

| ; the signal amplitude is

| ; considered to be noise.

DELTA_VEGO12 > 0 -----------------------| VEGO_SLOPE12 = 1

|  ;positive slope

|

| --- ELSE ---

|

| VEGO_SLOPE12 = 0

|  ;negative edge

VEGO_SLOPE12 <> slope_temp -------------| GRAD_CHNG12 = 1

| TSP_MXMN_V12 = VEGO12

|

|

|

| VEGO_OLD12 = VEGO12

|

| --- ELSE ---

|

| GRAD_CHNG12 = 0

| VEGO_OLD12 = VEGO12

CAT_SW_AMP12 > CAT_AMPCAL12 ------|

|AND -| TST_SW_FLG12 = 1

GRAD_CHNG12 = 1 ------------------|     | GRAD_CHNG12 = 0

|

|

| --- ELSE ---

|

| GRAD_CHNG12 = 0

| TST_SW_FLG12 = 0

END: ftp_trackingsp_bank1

10-28

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ftp_tracking_bank2

;This module executes a tracking switchpoint algorithm. This algorithm will

;effectively detrend the HEGO input voltage data i.e. it will count HEGO

;voltage switches that are superimposed upon a floating DC voltage. The

;algorithm detects a change in gradient sof the hego voltage signal. Upon

;detecting a gradient change it then compares the hego voltages between this

;gradient change and the last detected change. If the voltage is sufficiently

;high it recognizes that a switch has occurred.

unconditionally ------------------------|  slope_temp = VEGO_SLOPE22

|  DELTA_VEGO22 =

|      (VEGO22 - VEGO_OLD22)

|  CAT_SW_AMP22 =

|  ABS(VEGO22 - TSP_MXMN_V22)

ABS(DELTA_VEGO22) < TSP_DBAND2 ---------| exit this routine

| ; the signal amplitude is

| ; considered to be noise.

DELTA_VEGO22 > 0 -----------------------| VEGO_SLOPE22 = 1

|  ;positive slope

|

| --- ELSE ---

|

| VEGO_SLOPE22 = 0

|  ;negative edge

VEGO_SLOPE22 <> slope_temp -------------| GRAD_CHNG22 = 1

| VEGO_OLD22 = VEGO22

| TSP_MXMN_V22 = VEGO22

|

| --- ELSE ---

|

| GRAD_CHNG22 = 0

| VEGO_OLD22 = VEGO22

CAT_SW_AMP22 > CAT_AMPCAL22 ------|

|AND -| TST_SW_FLG22 = 1

GRAD_CHNG22 = 1 ------------------|     | GRAD_CHNG22 = 0

|

| --- ELSE ---

|

| GRAD_CHNG22 = 0

| TST_SW_FLG22 = 0

END: ftp_trackingsp_bank2

10-29

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ftp_filter

;This module is executed only when explicitly called.  The filter uses time

;intervals between successive detected HEGO switches.  If these switches time

;interval is below a calibratable amount they are considered to be noise or

;HEGO BUZZ and are discarded.

;Ego filter logic

unconditionally ------------------------| OBD_RCLK_DIF := (clock -

|                  OBD_RCLK_LST) *

|                  [stfc]

| ;Time since last EGO conversion

|

| OBD_RCLK_LST := clock

| ;Save current clock

;Bank 1.

unconditionally ------------------------| OBD_SW11_DUR := OBD_SW11_DUR +

|                  OBD_RCLK_DIF

| ;Time since last front EGOn switch

|

| OBD_SW12_DUR := OBD_SW12_DUR +

|                  OBD_RCLK_DIF

| ;Time since last rear EGO1 switch

SWTFL11 = 1 ----------------------|

;front switch                    |AND -| CAT_SWTFL11 := 1

|     |

OBD_SW11_DUR > OBD_MINPER --------|     |

| --- ELSE ---

|

| CAT_SWTFL11 := 0

TST_SW_FLG12 = 1 -----------------|

;rear switch                     |AND -| CAT_SWTFL12 := 1

|     |

OBD_SW12_DUR > OBD_MINPER --------|     |

| --- ELSE ---

|

| CAT_SWTFL12 := 0

SWTFL11 = 1 ----------------------------| OBD_SW11_DUR := 0

;reset time if switch seen

TST_SW_FLG12 = 1 -----------------------| OBD_SW12_DUR := 0

;reset time if switch seen

10-30

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;Bank 2.

NUMEGO = 1 -----------------------|

|AND -| Exit ftp_filter

HEGO_CONFIG = 1 ------------------|     | ;single bank application.

unconditionally ------------------------| OBD_SW21_DUR := OBD_SW21_DUR +

|                  OBD_RCLK_DIF

| ;Time since last front EGOn switch

|

| OBD_SW22_DUR := OBD_SW22_DUR +

|                  OBD_RCLK_DIF

| ;Time since last rear EGO2 switch

SWTFL21 = 1 ----------------------|

;front switch                    |AND -| CAT_SWTFL21 := 1

|     |

OBD_SW21_DUR > OBD_MINPER --------|     |

| --- ELSE ---

|

| CAT_SWTFL21 := 0

TST_SW_FLG22 = 1 -----------------|

;rear switch                     |AND -| CAT_SWTFL22 := 1

|     |

OBD_SW22_DUR > OBD_MINPER --------|     |

| --- ELSE ---

|

| CAT_SWTFL22 := 0

SWTFL21 = 1 ----------------------------| OBD_SW21_DUR := 0

;reset time if switch seen

TST_SW_FLG22 = 1 -----------------------| OBD_SW22_DUR := 0

;reset time if switch seen

END: ftp_filter

10-31

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ftp_am_cell

;This module is executed only when explicitly called.

;If AM_CELL := 0 we are not in an air mass cell and no

;ego, front or rear, switches are to be counted.

AM > AM1_MIN ---------------------|

|AND -| AM_CELL := 1

AM < AM1_MAX ---------------------|     | Exit ftp_am_cell

;AM in cell 1                          |

| --- ELSE ---

AM > AM2_MIN ---------------------|     |

|AND -| AM_CELL := 2

AM < AM2_MAX ---------------------|     | Exit ftp_am_cell

;AM in cell 2                          |

| --- ELSE ---

AM > AM3_MIN ---------------------|     |

|AND -| AM_CELL := 3

AM < AM3_MAX ---------------------|     | Exit ftp_am_cell

;AM in cell 3                          |

| --- ELSE ---

AM > AM4_MIN ---------------------|     |

|AND -| AM_CELL := 4

AM < AM4_MAX ---------------------|     | Exit ftp_am_cell

;AM in cell 4                          |

| --- ELSE ---

AM > AM5_MIN ---------------------|     |

|AND -| AM_CELL := 5

AM < AM5_MAX ---------------------|     | Exit ftp_am_cell

;AM in cell 5                          |

| --- ELSE ---

AM > AM6_MIN ---------------------|     |

|AND -| AM_CELL := 6

AM < AM6_MAX ---------------------|     | Exit ftp_am_cell

;AM in cell 6                          |

| --- ELSE ---

AM > AM7_MIN ---------------------|     |

|AND -| AM_CELL := 7

AM < AM7_MAX ---------------------|     | Exit ftp_am_cell

;AM in cell 7                          |

| --- ELSE ---

AM > AM8_MIN ---------------------|     |

|AND -| AM_CELL := 8

AM < AM8_MAX ---------------------|     | Exit ftp_am_cell

;AM in cell 8                          |

| --- ELSE ---

AM > AM9_MIN ---------------------|     |

|AND -| AM_CELL := 9

AM < AM9_MAX ---------------------|     | Exit ftp_am_cell

;AM in cell 9                          |

| --- ELSE ---

|

| AM_CELL := 0

END: ftp_am_cell

10-32

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ftp_switch_bank1

;This module is executed only when explicitly called.

;Rear/front ego switch ratio subroutine

;Bank 1.

APT = -1 -------------------|

|AND -|

CAT_APT_FLG = 1 ------------|     |

;closed throttle                  |

|

CAT_TP_RATE > CAT_TP_MAX ---------|

;Throttle stable                  |

|

AM_CELL = 0 ----------------------|

;not in an AM cell                |OR --| Exit ftp_switch_bank1

|     |   ;not appropriate to count

FEGO1_CT[AM_CELL] >=              |     |   ;HEGO switch.

FEGO1_MX_CT[AM_CELL] --|     |

;The current air mass cell        |     |

;has been filled                  |     | CAT_AQU_RDY1 = 0

|     | CAT_AQU_TMR1 = 0

CAT_DN1_FLG = 1 ------------------|     | ;zero free running timer

|     |

PTTMR < CAT_CTPT_TME -------------|     |

;Time since leaving closed        |     |

;throttle for transient condition |     |

;stabalization                          |

| --- ELSE ---

|

|

| CAT_AQU_RDY1 = 1

CAT_AQU_TMR1 <= CAT_STBLE_TME ----------| CAT_AQU_LS1 = 0

| Exit ftp_switch_bank1

|

| --- ELSE ---

|

| CAT_AQU_LS1 = 1

CAT_SWTFL11 = 1 ------------------|

;FEGO for bank 1 switched         |

|AND -| FEGO1_CT[AM_CELL] :=

FEGO1_CT[AM_CELL]                 |     |   FEGO1_CT[AM_CELL] + 1

> FEGO1_C_MIN[AM_CELL] --|     | FEGO1_CT_TOT :=

|    FEGO1_CT_TOT + 1

|  ;count switch for cell

|  ;and total (main count only)

10-33

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CAT_SWTFL11 = 1 ------------------|

;FEGO for bank 1 switched         |

|AND -| FEGO1_CT[AM_CELL] :=

FEGO1_CT[AM_CELL]                 |     |   FEGO1_CT[AM_CELL] + 1

< FEGO1_C_MIN[AM_CELL] -|     | FEGO1_CT_TOT :=

|   FEGO1_CT_TOT + 1

|  ;count switch for cell

|  ;and total

|

| FEGO1_CT2[AM_CELL] :=

|   FEGO1_CT2[AM_CELL] + 1

| FEGO1_C2_TOT :=

|    FEGO1_C2_TOT + 1

|  ;count switch for cell

|  ;and total (alterrnative counts)

CAT_SWTFL12 = 1 ------------------|

;REGO for bank 1 switched         |

|AND -| REGO1_CT := REGO1_CT + 1

FEGO1_CT[AM_CELL]                 |     | ;count switch for rear total

<= FEGO1_C_MIN[AM_CELL] ----|     |

| REGO1_CT2 := REGO1_CT2 + 1.

| ;count alternative method

CAT_SWTFL12 = 1 ------------------|

;REGO for bank 1 switched         |

|AND -| REGO1_CT := REGO1_CT + 1

FEGO1_CT[AM_CELL]                 |     | ;count switch for rear total

> FEGO1_C_MIN[AM_CELL] -----|

FEGO1_CT1 >= FEGO1_C_MIN1 --------|

|

FEGO1_CT2 >= FEGO1_C_MIN2 --------|

|

FEGO1_CT3 >= FEGO1_C_MIN3 --------|

|

FEGO1_CT4 >= FEGO1_C_MIN4 --------|

|

FEGO1_CT5 >= FEGO1_C_MIN5 --------|AND -| MIN_FLG_BK1 = 1

|     |

FEGO1_CT6 >= FEGO1_C_MIN6 --------|     |

|     |

FEGO1_CT7 >= FEGO1_C_MIN7 --------|     |

|     |

FEGO1_CT8 >= FEGO1_C_MIN8 --------|     |

|     |

FEGO1_CT9 >= FEGO1_C_MIN9 --------|     |

| --- ELSE ---

|

| MIN_FLG_BK1 = 0

END: ftp_switch_bank1

10-34

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ftp_switch_bank2

;Rear/Front EGO Switch Ratio Subroutine

;Bank 2.

APT = -1 -------------------|

|AND -|

CAT_APT_FLG = 1 ------------|     |

;closed throttle                  |

|

CAT_TP_RATE > CAT_TP_MAX ---------|

;Throttle stable                  |

|

AM_CELL = 0 ----------------------|OR --| Exit ftp_switch_bank2

;not in an AM cell                |     |   ;not appropriate to count

|     |   ;HEGO switches

FEGO2_CT[AM_CELL] >=              |     |

FEGO2_MX_CT[AM_CELL] --|     |

;The current air mass cell        |     | CAT_AQU_RDY2 = 0

;has been filled                  |     | CAT_AQU_TMR2 = 0

|     | ;zero free running timer

NUMEGO = 1 -----------------|     |     |

|AND -|     |

HEGO_CONFIG = 1 ------------|     |     |

|     |

CAT_DN2_FLG = 1 ------------------|     |

| --- ELSE ---

|

|

| CAT_AQU_RDY2 = 1

CAT_AQU_TMR2 <= CAT_STBLE_TME ----------| CAT_AQU_LS2 = 0

| Exit ftp_switch_bank1

|

| --- ELSE ---

|

| CAT_AQU_LS2 = 1

CAT_SWTFL21 = 1 ------------------|

;FEGO for bank 1 switched         |

|AND -|

FEGO2_CT[AM_CELL]                 |     | FEGO2_CT[AM_CELL] :=

> FEGO2_C_MIN[AM_CELL] --|     |   FEGO2_CT[AM_CELL] + 1

| FEGO2_CT_TOT :=

|    FEGO2_CT_TOT + 1

|  ;count switch for cell

|  ;and total (main count only)

10-35

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CAT_SWTFL21 = 1 ------------------|

;FEGO for bank 2 switched         |

|AND -| FEGO2_CT[AM_CELL] :=

FEGO2_CT[AM_CELL]                 |     |   FEGO2_CT[AM_CELL] + 1

< FEGO2_C_MIN[AM_CELL] -|     | FEGO2_CT_TOT :=

|   FEGO2_CT_TOT + 1

|  ;count switch for cell and total

|

|  FEGO2_CT2[AM_CELL] :=

|    FEGO2_CT2[AM_CELL] + 1

|  FEGO2_C2_TOT :=

|     FEGO2_C2_TOT + 1

|   ;count switch for cell

|   ;and total (alterrnative counts)

CAT_SWTFL22 = 1 ------------------|

;REGO for bank 1 switched         |

|AND -| REGO2_CT := REGO2_CT + 1

FEGO2_CT[AM_CELL]                 |     | ;count switch for rear total

<= FEGO2_C_MIN[AM_CELL] ----|     |

| REGO2_CT2 := REGO2_CT2 + 1

| ;count alternative method

CAT_SWTFL22 = 1 ------------------|

;REGO for bank 1 switched         |

|AND -| REGO2_CT := REGO2_CT + 1

FEGO2_CT[AM_CELL]                 |     | ;count switch for rear total

> FEGO2_C_MIN[AM_CELL] -----|

FEGO2_CT1 >= FEGO2_C_MIN1 --------|

|

FEGO2_CT2 >= FEGO2_C_MIN2 --------|

|

FEGO2_CT3 >= FEGO2_C_MIN3 --------|

|

FEGO2_CT4 >= FEGO2_C_MIN4 --------|

|

FEGO2_CT5 >= FEGO2_C_MIN5 --------|AND -| MIN_FLG_BK2 = 1

|     |

FEGO2_CT6 >= FEGO2_C_MIN6 --------|     |

|     |

FEGO2_CT7 >= FEGO2_C_MIN7 --------|     |

|     |

FEGO2_CT8 >= FEGO2_C_MIN8 --------|     |

|     |

FEGO2_CT9 >= FEGO2_C_MIN9 --------|     |

| --- ELSE ---

|

| MIN_FLG_BK2 = 0

END: ftp_switch_bank2

10-36

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ftp_ratio_bank1

;This module is executed only when explicitly called.

;Ratio calculation.

;Bank 1.

CAT_DN1_FLG = 1 ------------------------| exit routine

| ; bank already completed once this

| ;  trip

FEGO1_CT_TOT = 0 -----------------------| EGO1_RF_RAT1 := 0

|  ;prevent divide by zero

|

| --- ELSE ---

|

| EGO1_RF_RAT1 :=

|  REGO1_CT / FEGO1_CT_TOT

|   ;Compute switch ratio

FEGO1_C2_TOT = 0 -----------------------| EGO1_RF_RAT2 := 0

|  ;prevent divide by zero

|

| --- ELSE ---

|

| EGO1_RF_RAT2 :=

| REGO1_CT2/ FEGO1_C2_TOT

|  ;Compute switch ratio

10-37

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CAT_TMR_ACT > CAT_TIER1 ----------|

;exceeded minimum time in test    |

;TIER1 TEST                       |AND -| CAT_DN1_FLG := 1

|     | ;Catalyst test completed (alt)

MIN_FLG_BK1 = 1 ------------------|     | EGO1_RF_RAT := EGO1_RF_RAT2

;The result is valid only if all        |

;cells have at least minimum number     |

;of switches - TIER2 TEST.              |

| --- ELSE ---

|

FEGO1_CT_TOT >= FEGO1_MX_TOT -----------| CAT_DN1_FLG := 1

| ;Catalyst test completed

| EGO1_RF_RAT :=  EGO1_RF_RAT1

|

| --- ELSE ---

|

| Exit ftp_ratio_bank1

|  ;test not yet complete

;Test completed -- reset counters

;decide which filter constant to use based on KAM warm-up counter value.

;NOTE: If you need to use a single filter constant value then set EGO_CAT_FKA

;      equal to EGO_CAT_FKB.

;      IF EGO_CAT_FKA DOES NOT EQUAL ECG_CAT_FKB THEN 1.3 X THRESHOLD

EMISSIONS

MUST BE MET

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

KWUCTR < CAT_KWU_CNT -------------------| fk_temp = EGO_CAT_FKA

|

| --- ELSE ---

|

| fk_temp = EGO_CAT_FKB

EGO1_RF_RAT > EGO1_RF_AVE + EGO_RF_DEL -| EGO1_RF_AVE := EGO1_RF_RAT

;ratio out of believable range         |  ;use new ratio for average

;-- catastrophic failure               |

| --- ELSE ---

|

| EGO1_RF_AVE := ROLAV(EGO1_RF_RAT,

|                      fk_temp)

|  ;put ratio in averaging function

EGO1_RF_AVE > EGO1_FL_RAT --------------| CAT1_FAILED := 1

;ratio indicates failure               |

| --- ELSE ---

|

| CAT1_FAILED := 0

unconditionally ------------------------| Do: ftp_reset_bank1

| ;reset counters

END: ftp_ratio_bank1

10-38

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ftp_ratio_bank2

;This module is executed only when explicitly called.

;Ratio calculation.

;Bank 2.

NUMEGO = 1 -----------------------|

|AND -| Exit ftp_ratio_bank2

HEGO_CONFIG = 1 ------------------|

CAT_DN2_FLG = 1 ------------------------| exit routine

| ; bank already completed once this

| ;  trip

FEGO2_CT_TOT = 0 -----------------------| EGO2_RF_RAT1 := 0

|

|  ;prevent divide by zero

|

| --- ELSE ---

|

| EGO2_RF_RAT1 :=

|        REGO2_CT / FEGO2_CT_TOT

|  ;Compute switch ratio

FEGO2_C2_TOT = 0 -----------------------| EGO2_RF_RAT2 := 0

|   ;prevent divide by zero

|

| --- ELSE ---

|

| EGO2_RF_RAT2 :=

|        REGO2_CT2/ FEGO2_C2_TOT

| ;Compute switch ratio

10-39

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CAT_TMR_ACT > CAT_TIER1 ----------|

;exceeded minimum time in test    |

;TIER1 TEST                       |AND -| CAT_DN2_FLG := 1

|     | ;Catalyst test completed (alt)

MIN_FLG_BK2 = 1 ------------------|     | EGO2_RF_RAT :=  EGO2_RF_RAT2

;The result is valid only if all        |

;cells have at least minimum number     |

;of switches - TIER2 TEST.              |

| --- ELSE ---

|

| CAT_DN2_FLG := 1

|  ;Catalyst test completed

FEGO2_CT_TOT >= FEGO2_MX_TOT -----------| EGO2_RF_RAT := EGO2_RF_RAT1

|

| --- ELSE ---

|

| Exit ftp_ratio_bank2

|  ;test not yet complete

;Test completed -- reset counters

;decide which filter constant to use based on KAM warm-up counter value.

;NOTE: If you need to use a single filter constant value then set EGO_CAT_FKA

;      equal to EGO_CAT_FKB.

;      IF EGO_CAT_FKA DOES NOT EQUAL ECG_CAT_FKB THEN 1.3 X THRESHOLD

EMISSIONS

MUST BE MET

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

KWUCTR < CAT_KWU_CNT -------------------| fk_temp = EGO_CAT_FKA

|

| --- ELSE ---

|

| fk_temp = EGO_CAT_FKB

EGO2_RF_RAT > EGO2_RF_AVE + EGO_RF_DEL -| EGO2_RF_AVE := EGO2_RF_RAT

;ratio out of believable range         |  ;use new ratio for average

;-- catastrophic failure               |

| --- ELSE ---

|

| EGO2_RF_AVE := ROLAV(EGO2_RF_RAT,

|                      fk_temp)

|  ;put ratio in averaging function

EGO2_RF_AVE > EGO2_FL_RAT --------------| CAT2_FAILED := 1

;ratio indicates failure               | ;Catalyst failed

|

| --- ELSE ---

|

| CAT2_FAILED := 0

| ;Catalyst passed

10-40

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

unconditionally ------------------------| Do: ftp_reset_bank2

| ;reset counters

END: ftp_ratio_bank2

BEGIN: ftp_codes

;This module is executed only when explicitly called.

;Set trouble codes if Catalyst failed.

CAT_DN1_FLG = 1 ------------------|

;test complete once              |

|

CAT1_FAILED = 1 ------------------|AND -| Do: malfunction(catalyst,P0420)

;bank one failed                 |     |

|     |

CAT_TST_ENA = 1 ------------------|     |

;enabled by exec                       |

| --- ELSE ---

CAT_TST_ENA = 1 ------------------|     |

;enabled by exec                 |AND -| Do: clear_malf(P0420MALF)

|

CAT_DN1_FLG = 1 ------------------|

;test complete once

CAT_DN2_FLG = 1 ------------------|

;test complete once               |

|

CAT2_FAILED = 1 ------------------|AND -| Do: malfunction(catalyst,P0430)

;bank two failed                 |     |

|     |

CAT_TST_ENA = 1 ------------------|     |

;enabled by exec                       |

| --- ELSE ---

CAT_TST_ENA = 1 ------------------|     |

;enabled by exec                 |AND -| Do: clear_malf(P0430MALF)

|

CAT_DN2_FLG = 1 ------------------|

;test complete once

CAT_DN1_FLG = 1 ------------------|

;bank1 done                       |

|

CAT_DN2_FLG = 1 ------------|     |

;bank2 done                 |     |AND -| CAT_MON := 1

|OR --|     | CAT_DN1_FLG := 0

NUMEGO = 1 -----------|     |     |     | CAT_DN2_FLG := 0

;only one bank        |AND -|     |     |  ;test completed at least once

|           |

HEGO_CONFIG = 1 ------|           |

|

CAT_TST_ENA = 1 ------------------|

;test enabled

END: ftp_codes

10-41

CATALYST BYPASS CONTROL STRATEGY, FTP CATALYST MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

10-42

CHAPTER 11

EGO MONITOR CONTROL STRATEGY

11-1

EGO MONITOR CONTROL STRATEGY

11.1  FEATURE:  HEGO MONITOR - V10.6_EGO_MONITOR (CDAN0)

11.1.1  EGO MONITOR CONTROL (CDAN0)

+----------------------------+

| EGO_MONITOR_CONTROL_COM10  |

+----------------------------+

|PUBLIC PROCEDURES:          |

|. ego_monitor_controller    |

+----------------------------+

|PRIVATE PROCEDURES:         |

|. disable_fuel_modulation   |

|. ego_tst_rdy_control       |

+------------------+---------+

|

+-----------------------------+                  |

| EGO_INPUT_COM10             |                  |

+-----------------------------+                  | (This process called

|. ego_input_init             |<-- RAM init.     |  in the background.)

|. background_ego_input       |<-----X     ------|

+-----------------------------+                  |

|

+-----------------------------+                  |

| EGO_CSD_DETECTION_COM       |                  | (CSD FFG_CSD flag

+-----------------------------+                  |  logic)

|. csd_detection_init         |<-----------------|  COM1 -> FFG_CSD algo

|. csd_detection_kam_reset    |<-----------------|  COM2 -> stub

|. csd_detection_process      |<-----------------|

+-----------------------------+                  |

|

+----------------------------------+             |

| EGO_CONTINUOUS_TEST_COM10        |             |

+----------------------------------+             |

|PUBLIC PROCEDURES:                |             |

|. bg_check_entry_conditions       |<------------|

+----------------------------------+             |

|

+----------------------------------+             |

| EGO_US_MONITOR_COM10             |             |

+----------------------------------+             |

|PUBLIC PROCEDURES:                |             |

|. check_upstream_entry_conditions |<------------|

|. check_upstream_results          |<------------|

|. check_upstream_static_shift     |<------------|

|. upstream_ego_monitor            |<------------|

|. upstream_ego_sampling           |<-- 8ms      |

|. upstream_initialization         |<------------|

|. upstream_overvoltage_check      |<------------|

|. reset_overvoltage_us            |<------------|

+----------------------------------+             |

|

(continued on next page)

11-2

EGO MONITOR CONTROL STRATEGY, EGO MONITOR CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

+----------------------------------+             |

| EGO_DS_MONITOR_COM10             |             |

+----------------------------------+             |

|PUBLIC PROCEDURES:                |             |

|. detect_ds_ego_malf              |<------------|

|. downstream_ego_monitor          |<------------|

|. downstream_initialization       |<-- RAM init.|

|. forced_downstream_excursion     |<------------|

|. determine_flg_ego_idle          |<------------|

|. abort_ds_ego_monitor            |<------------|

|. stop_excursion_check            |<------------|

|. downstream_overvoltage_check    |<------------|

|. reset_overvoltage_ds            |<------------|

+----------------------------------+             |

|

+-----------------------------+                  |

| EGO_HEATER_CONTROL_COM10    |                  |

+-----------------------------+                  |

|PUBLIC PROCEDURES:           |                  |

|. heater_initialization      |<-- RAM init.     |

|. ego_heater_monitor         |<-----------------|

+-----------------------------+                  |

|

+-----------------------------+                  |

| EGO_REV_COUNTER_COM         |                  | (update sync count)

+-----------------------------+                  |  COM2 -> V10 engine

|. rev_counting_process       |<-----------------+  COM1 -> all others

+-----------------------------+

11-3

EGO MONITOR CONTROL STRATEGY, EGO MONITOR CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EGO MONITOR INTERFACE REQUIREMENTS TO

REMAINDER OF CONTROL STRATEGY

+-----------------+

| FUEL_LAMBSE_MOD |<------+---- LAM_MOD_FLG

+-----------------+       |

|

+------------+            |

| FUEL_ADAPT |<-----------+ (disable adaptive)

+------------+            |

|

+-------------------+     |

| CANP_PCOMP_ENABLE |<----+ (disable purge)

+-------------------+     |

|

+----------------+        |

| FUEL_CL_LAMBSE |<-------+ (use unique bias, tdrevs)

+----------------+        |

|

+--------------+          |

| FUEL_BG_CALC |<---------+ (modify commanded A/F ratio)

+--------------+          |

|

+----------------+        |

| FUEL_REGO_TRIM |<-------+ (disable R_BIAS updates)

+----------------+

+----------------+<----------- DS_EGO_RICHn (rich excursion)

| FUEL_OL_LAMBSE |<----------- DS_EGO_LEANn (lean excursion)

+----------------+

+------------+

| FUEL_OLFLG |<---------------- DS_EGO_OL (force open loop

+------------+                             during excursion)

Main entry point for the HEGO Monitor.  All HEGO tests, with the exception of

the KOER self test, are called from this module.

This module is used to control the OBDII EGO monitor for  Y-pipe,  dual  bank

and  single  bank  applications using FAOSC method #2.  This includes control

and coordination of the upstream, and downstream EGO monitors as well as  the

EGO heater monitor.  OBDII regulations require monitoring of the upstream EGO

sensors to detect when the deterioration of the sensor  has  proceeded  to  a

point  where  the  emissions requirements have been exceeded by 1.5 times the

standard for a new vehicle.

Five main software modules are used to fulfill this  monitoring  requirement.

These    modules    are    shown    above   pictorially   and   include   the

EGO_MONITOR_CONTROL, EGO_US_MONITOR, EGO_DS_MONITOR, EGO_HEATER_CONTROL,  and

EGO_CONTINUOUS_TEST.

11-4

EGO MONITOR CONTROL STRATEGY, EGO MONITOR CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Shown in the diagram above are the names of the  processes  contained  within

each  one of the modules.  Some of the processes are PUBLIC.  That means they

are invoked from somewhere external to the  module.   The  remainder  of  the

processes  are PRIVATE (not shown in the diagram above).  That means they are

only invoked from within the module in which they reside and no other process

has any knowledege of their existence.

Additionally, a number of other software modules  are  modified  slightly  to

allow  proper  operation  of  the  powertrain  during  all  phases of the EGO

monitor.  These additional modules and the interface flags  used  to  control

execution  flow  within them during the EGO monitor is shown in the INTERFACE

REQUIREMENTS diagram shown above.

This module contains the public  process,  ego_monitor_controller,  which  is

invoked once each background pass.  It monitors system parameters and signals

from the OBDII diagnostic executive and uses this  information  to  initiate,

continue, or abort an EGO monitor test sequence.

Also contained within this module are  two  private  processes.   The  first,

rev_counting_process,  is  a  foreground  process.  It executes each PIP high

interrupt and is used to count engine revolutions.   It  is  used  to  ensure

stabalized engine operation after initiating high frequency fuel modulation.

The    second    private    process    contained    within    this    module,

disable_fuel_modulation,  is  used  to  place  the  EGO monitor in an initial

state.  This can be invoked when the diagnostic executive  disables  the  EGO

monitor,  or  when  the  local  entry  conditions to execute the upstream EGO

monitor are violated during a test of the upstream EGO sensors.

During normal operation of the vehicle,  the  EGO  monitor  will  follow  the

following execution sequence:

I.  Test is enabled from the Diagnostic Executive.

II.  Local entry conditions are satisified.

III.  High frequency fuel modulation begins.

IV.  After a number of engine  revolutions,  sampling  of  the  upstream  EGO

sensors  starts  occuring every 16 milliseconds.  Upstream EGO sensor voltage

statistics are computed.

V.  After a number of comanded  LAMBSE  cycles  occur,  high  frequency  fuel

modulation is disabled, and 16 millisecond sampling is disabled.

VI.  EGO signal amplitude is calculated for each  of  the  upstream  sensors.

These  statistics are compared to calibrated limits and a decision is made as

to the health of the upstream EGO sensors.

VII.  The voltage envelope of the downstream EGO sensors is checked,  and  if

it  not  sufficently  large  because  a  minimum  rich voltage on bank 1 or a

minimum lean voltage on bank 2 has not been observed, then the  fuel  control

is forced into open loop and a rich A/F ratio is commanded for bank 1.

11-5

EGO MONITOR CONTROL STRATEGY, EGO MONITOR CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

VIII.  After the calibrated number of seconds times out or the required  rich

voltage  is  seen  the  rich  A/F  excursion  is discontinued.  If a required

minimum lean EGO sensor voltage has not been seen up to this point  then  the

fuel  control  is forced into open loop operation and a lean and A/F ratio is

commanded for a calibrated number of seconds for the appropriate bank.

IX.  After the calibrated number of seconds times out or  the  required  lean

EGO sensor voltage is seen, the lean A/F excursion is discontinued.

X.  A decision is made on the relative health of the downstream  EGO  sensors

based on the voltage envelope attained on the downstream EGO sensors.

Simultaneously to all of the above, whenever an overvoltage condition  occurs

on  any  EGO  sensor,  a  malfunction  will  be  identified to the diagnostic

executive and no further testing  of  the  malfunctioning  sensor  will  take

place.

Also, the EGO heaters will be testes by rapidly turning them on and then  off

and  then  back  on again and recording the activity on the OSM (output state

monitor) input for each of the EGO heaters.  Malfunctions will be  identified

to the diagnostic executive when a faulty heater is found.

In order to ensure EGO tests can be included/excluded as  required  to  match

the particular application a set of configuration switches are made available

to the calibrator.  The table below shows the calibration in  order  to  turn

all EGO monitor tests on:

11-6

EGO MONITOR CONTROL STRATEGY, EGO MONITOR CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EGO MONITOR CALIBRATION ENABLEMENT

EGO TEST NAME                           APPLICABLE CALIBRATION SWITCH

--------------------------              ----------------------------------

UPSTREAM EGO MONITOR                    *EGO_TST_SW = 1

DOWNSTREAM EGO MONITOR                  *EGO_TST_SW = 1

EGO HEATER MONITOR (CONT)               *EGO_HTR_SW = 1 & [EGO_TST_SW = 1

OR CCM_TST_SW = 1]

EGO HEATER MONITOR (KOER)               *EGO_HTR_SW = 1

EGO HEATER CHECK (KOEO)                 EGO_HTR_SW = 1

EGO SWITCH TEST (CONT)                  *V_EGO_ENA = 1

EGO U.S. SWITCH TEST (KOER)             V_EGO_ENA = 1

EGO D.S. SWITCH TEST (KOER)             V_EGO_ENA = 1 & DS_KOER_SW = 1

OVERVOLTAGE (CONT)                      *EGO_TST_SW = 1 OR CCM_TST_SW = 1

OVERVOLTAGE (KOER)                      *unconditional

OVERVOLTAGE (KOEO)                      *unconditional

* :  Test can also be allowed to execute, but failure limits can be  used  to

calibrate out the test.

11-7

EGO MONITOR CONTROL STRATEGY, EGO MONITOR CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  CCM_EO_ENA = Engine off on demand test for CCM enabled.

-  DS_DLY_TMR = Timer used to measure the amount of time  an  A/F  excursion

has  been active to test the downstream EGO sensors.  Timer, Free running

down counter, Units:  SECONDS, Range:  0 - 60, Resolution:  0.25.

-  DS_HTR_TMR = Elapsed time downstream EGO heaters have been on, seconds.

-  DS_STATE    =    State    variable    to    be    used     during     the

forced_downstream_excursion EGO monitor test.

-  EGO_REV_CTR = Count of the number of revolutions of the engine that  have

occured     since    starting    the    rev_counting_process    in    the

ego_monitor_controller.  Register, Units:  engine revs., Range:  0 - 100,

Resolution:  1.

-  EXT_FEU = Inferred unheated tip temperature  at  the  front  HEGO  sensor

location, degrees.

-  EXT_FLANGE = Instantaneous inferred exhaust flange temperature.

-  EXT_REG = Inferred tip temperature for the downstream EGO sensor.

-  KAM_ERROR = Indicates Keep Alive RAM invalid.

-  LAM_CYCLES = Number of commanded LAM_MOD cycles that have  occured  since

starting the upstream EGO monitor.

-  LOAD = Universal load as a ratio of air charge over standard air charge.

-  LOAD_EGO = Filtered LOAD for use in the EGO monitor.

-  N = Engine RPM.

-  N_EGO = Filtered engine speed for use in the EGO monitor.

-  OL_DESIRED = Open loop fuel desired flag.

-  VS = Instantaneous vehicle speed, MPH.

-  VS_EGO = Filtered VS for use in the EGO monitor.

Bit Flags:

-  ABORT_DONE = Flag indicating the EGO monitor abort process  is  complete.

1 -> abort is complete.

-  CCM_ER_ENA = Engine running on demand test for CCM enabled.

-  CCM_TST_ENA = OBD-II Comprehensive Component Test enable flag; 1 ->  test

enabled.

-  CCM_TST_RDY = CCM test ready flag; 1 -> all local  conditions  have  been

met; ready to run CCN test.

11-8

EGO MONITOR CONTROL STRATEGY, EGO MONITOR CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  CURR_TST_ENA = Flag used to start the EGO heater current measurement.

-  DS_EGO_LEAN1 = Flag used by the downstream EGO monitor to trigger a  lean

A/F excursion for bank 1; 1 -> excursion is in progress.

-  DS_EGO_LEAN2 = Flag used by the downstream EGO monitor to trigger a  lean

A/F excursion for bank 2; 1 -> excursion is in progress.

-  DS_EGO_OL = Flag used to force open loop fuel control during a forced A/F

excursion to test the downstream EGO sensors.

-  DS_EGO_RICH1 = Flag used by the downstream EGO monitor to trigger a  rich

A/F excursion for bank 1; 1 -> excursion is in progress.

-  DS_EGO_RICH2 = Flag used by the downstream EGO monitor to trigger a  rich

A/F excursion for bank 2; 1 -> excursion is in progress.

-  EGO_DS_MON = Flag used to signal when the downstream EGO monitor has  run

to completion.

-  EGO_HTR_MON = Monitor flag for the EGO heaters; 1 -> all EGO faults  have

been monitored at least once since power-up.

-  EGO_MON11 = EGO11 monitored flag; 1 -> sensor monitored.

-  EGO_MON21 = EGO21 monitored flag; 1 -> sensor monitored.

-  EGO_RETEST = Flag used to trigger a retest of the upstream EGO monitor; 1

-> Retest the upstream EGOs.

-  EGO_REV_ENA =  Flag  used  to  enable  the  rev_counting_process  in  the

ego_monitor_controller; 1 -> enable the foreground rev_counting_process.

-  EGO_REV_FLG = Flag used to trigger the capture of  the  SYNC_CTR  in  the

foreground  rev_counting_process;  1 -> capture SYNC_CTR on next PIP high

interrupt.

-  EGO_STAT11 = Flag indicating the first EGO11 switch has been encountered,

start the area ratio calculation.

-  EGO_STAT21 = Flag indicating the first EGO21 switch has been encountered,

start the area ratio calculation.

-  EGO_TST_ENA = OBD-II EGO test enable flag; 1 -> test enabled.

-  EGO_TST_RDY = EGO test ready flag; 1 -> all local  conditions  have  been

met; ready to run EGO test.

-  EGO_US_MON = Flag used to signal when the upstream EGO monitor has run to

completion.

-  EGO_WIND_SET = Flag indicating EGO monitor steady state window conditions

have been set; 1 -> window conditions set.

-  LAM_MOD_FLG = Flag used to initiate high frequency  fuel  modulation  for

the upstream EGO monitor; 1 -> fuel modulation active.

11-9

EGO MONITOR CONTROL STRATEGY, EGO MONITOR CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PxxxMALF = Malfunction flag for code Pxxx; 1 -> a  malfunction  currently

exists for fault Pxxx.

-  SMPL_EGO_FLG = Flag used to enable the  foreground  EGO  samling  process

used on the upstream EGO sensors; 1 -> enable the sampling.

-  STOP_EXCURS = Flag used to signal when a forced downstream ego monitoring

excursion is not allowed.

Calibration Constants:

-  DS_HEAT_TM = Amount of time into run mode when the downstream EGO sensors

heaters  have  been on long enough to allow testing of the downstream EGO

sensors.   Calibration  Constant,  Units:   SECONDS,  Range:   0  -  255,

Resolution:  1.  Base value:  60.

-  DS_OV_ON = Minimum downstream EGO tip temperature to  enable  overvoltage

monitor to run.

-  EGO_HTR_SW =  Calibration  switch  to  enable  EGO  heater  controls  and

monitoring.

-  EGO_TST_SW = EGO test present calibration switch; 1 -> EGO test present.

-  LAM_CYC_MAX = Number  of  commanded  LAMBSE  fuel  modulation  cycles  to

perform for the EGO monitor w/FAOSC active.

-  NUMEGO = Number of EGO sensors.

-  OBDII_RESET = Flag used to simulate the receipt of an  OBD-II  scan  tool

reset message.

-  TCLOAD_EGO = Time constant for filtered LOAD for use in the EGO  monitor,

sec.

-  TCN_EGO = Time constant for filtered N for use in the EGO monitor, sec.

-  TCVS_EGO = Time constant for filtered VS for use in the EGO monitor, sec.

-  US_OV_ON = Minimum upstream EGO tip  temperature  to  enable  overvoltage

monitor to run.

-  US_REV_DLY = Number  of  engine  revolutions  to  delay  entry  into  the

upstream  EGO  monitor  after  starting  high  frequency fuel modulation.

Claibration Constant,  Units:   engine  revolutions,  Range:   0  -  100,

Resolution:  1.

OTHER

-  ego_codes   =   set   of    (upstream_ego_codes,    downstream_ego_codes,

ego_htr_codes,  ego_on_demand_codes).  The set of OBD-II fault codes that

relate to the ego monitor.

-  P0132 = Heated oxygen sensor circuit voltage out of range, sensor 11.

-  P0152 = Heated oxygen sensor circuit voltage out of range, sensor 12.

11-10

EGO MONITOR CONTROL STRATEGY, EGO MONITOR CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EGO_MONITOR_CONTROL_COM10

BEGIN: ego_monitor_controller

;

;This procedure is executed once per background pass and controls the

;execution of the HEGO monitor, including the upstream and downstream

;functional checks, heater control and monitor logic, and the continuous

;switching test.

;

unconditionally -----------------------| Do: ego_lambse_clips

|  ;set adaptive lambse clips

|  ;to cal constants (if adaptive

|  ;algorithm not installed).

EGO_TST_SW = 1 ------------------------| LOAD_EGO = rolav(LOAD, TCLOAD_EGO)

;filter inputs to define steady       | VS_EGO = rolav(VS, TCVS_EGO)

;state and start continuous tests.    | N_EGO = rolav(N, TCN_EGO)

| Do: update_ego_cl_tmr

| ego_window_set := EGO_WIND_SET

|    ;capture current value of window

| Do: check_upstream_entry_conditions

|    ;this updates value of window

| Do: determine_flg_ego_idle

| Do: stop_excursion_check

| Do: ego_tst_rdy_control

EGO_HTR_SW = 1 ------------------------| Do: ego_heater_control

EGO_TST_ENA = 1 -----------------|

|

CCM_ER_ENA = 1 ------------------|

|OR --| test_enabled := 1

CCM_TST_ENA = 1 -----------------|     |

|     |

CCM_EO_ENA = 1 ------------------|     |

| --- ELSE ---

|

| test_enabled := 0

11-11

EGO MONITOR CONTROL STRATEGY, EGO MONITOR CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

P0132MALF = 1 -------|

|

P0152MALF = 1 -------|AND -|

|     |

NUMEGO = 2 ----------|     |

|OR --|

P0132MALF = 1 -------|     |     |

|AND -|     |

NUMEGO = 1 ----------|     |     |AND -| Do: upstream_overvoltage_check

|     |     |  ;only test for overvoltage if

OL_DESIRED = 0 ------------|     |     |  ;closed loop is desired and the

|     |  ;sensor location is hot enough OR

test_enabled = 1 ----------------|     |  ;overvoltage fault has forced

|     |  ;open loop.

EXT_FEU > US_OV_ON --------------|     |

| --- ELSE ---

|

| Do: reset_overvoltage_us

|  ;freeze OV timers.

DS_HTR_TMR > DS_OV_TIME ---------|

|

test_enabled = 1 ----------------|AND -| Do: downstream_overvoltage_check

|     |  ;only test for overvoltage if

EXT_REG > DS_OV_ON --------------|     |  ;heaters have been on long enough

|  ;and the sensor location is

|  ;hot enough.

| --- ELSE ---

|

| Do: reset_overvoltage_ds

|  ;freeze OV timers.

test_enabled = 1 ----------------------| Do: downstream_ego_monitor

|  ;perform functional check of

|  ;downstream sensors.

| Do: check_upstream_static_shift

|  ;check the FAOSC R_BIASn values.

test_enabled = 1 ----------------|

|AND -| Do: csd_detection_process

CCM_EO_ENA = 0 ------------------|     |  ;check upstream EGO sensors for

|  ;characteristic shift downward.

11-12

EGO MONITOR CONTROL STRATEGY, EGO MONITOR CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OBDII_RESET = 1 -----------------|

;system reset.                  |OR --| Do: disable_fuel_modulation

|     | Do: upstream_initialization

KAM_ERROR = 1 -------------------|     | EGO_TST_RDY := 0

|    ;interface requirement from

|    ;the executive to hold this

|    ;low for at least one BG pass.

| EGO_MON11 := 0

| EGO_MON21 := 0

| EGO_US_MON := 0

| EGO_DS_MON := 0

| EGO_HTR_MON := 0

| Do: abort_ds_ego_mon

|    ;take actions that occur only on

|    ;disable from executive.

| CURR_TST_ENA := 0

| ABORT_DONE := 1

| Do: csd_detection_init

KAM_ERROR = 1 -------------------------| Do: reset_ego_heater

| Do: reset_us_ego_monitor

| Do: csd_detection_kam_reset

EGO_TST_ENA = 0 -----------------|

|AND -| EGO_TST_RDY := 0

ABORT_DONE = 0 ------------------|     |    ;interface requirement from

;test is disabled from the            |    ;the executive to hold this

;executive.                           |    ;low for at least one BG pass.

| Do: disable_fuel_modulation

| DS_EGO_OL := 0

| DS_EGO_RICH1 := 0

| DS_EGO_RICH2 := 0

| DS_EGO_LEAN1 := 0

| DS_EGO_LEAN2 := 0

| DS_DLY_TMR := 0

| CURR_TST_ENA := 0

| ABORT_DONE := 1

NUMEGO = 1 ----------------------------| EGO_MON21 := true

;single bank system.

11-13

EGO MONITOR CONTROL STRATEGY, EGO MONITOR CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EGO_TST_ENA = 1 -----------------|

;Test enabled from the executive|

|

EGO_WIND_SET = 1 ----------------|

;entry conditions not met       |

;last pass.                     |

|AND -| LAM_MOD_FLG := 1

LAM_MOD_FLG = 0 -----------------|     |    ;initiate fuel modulation.

;test not currently running.    |     | EGO_REV_FLG := 1

|     |    ;trigger foreground process

EGO_MON11 = 0 -------------|     |     |    ;to capture sync_ctr.

;upstream monitor not done|     |     | EGO_REV_CTR := 0

;for bank1.               |OR --|     | EGO_REV_ENA := 1

|           |    ;enable the rev_counting_process

EGO_MON21 = 0 -------------|           |    ;in the foreground.

;upstream monitor not done            | ABORT_DONE := 0

;for bank2.                           | Do: upstream_initialization

|    ;get ready to perform testing

| Do: csd_detection_init

|

|

| --- ELSE ---

EGO_TST_ENA = 1 -----------------|     |

;Test enabled from the executive|     |

|     |

EGO_WIND_SET = 0 ----------------|     |

;local entry conditions not     |     |

;met.                           |     |

|AND -| Do: disable_fuel_modulation

ego_window_set = 1 --------------|     |    ;aborted due to local entry

;window was set last pass and is|     |    ;conditions not being met.

;closed now.                    |

|

EGO_MON11 = 0 -------------|     |

;upstream monitor not done|     |

;for bank1.               |OR --|

|

EGO_MON21 = 0 -------------|

;upstream monitor not done

;for bank2.

11-14

EGO MONITOR CONTROL STRATEGY, EGO MONITOR CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EGO_TST_ENA = 1 -----------------|

;Test enabled from the executive|

|

EGO_WIND_SET = 1 ----------------|

|AND -|    ;delay complete, perform monitor.

EGO_REV_CTR >= US_REV_DLY -------|     | EGO_REV_ENA := 0

|     |    ;disable the rev_counting_process

EGO_MON11 = 0 -------------|     |     |    ;in the foreground.

;upstream monitor not done|     |     | SMPL_EGO_FLG := 1

;for bank1.               |OR --|     |    ;enable 16ms EGO sampling

|           | Do: upstream_ego_monitor

EGO_MON21 = 0 -------------|

;upstream monitor not done

;for bank2.

EGO_TST_ENA = 1 -----------------|

;Test enabled from the executive|

|

EGO_WIND_SET = 1 ----------------|

|AND -|  ;upstream monitor complete.

EGO_REV_CTR >= US_REV_DLY -------|     | Do: check_upstream_results

|     | Do: disable_fuel_modulation

LAM_CYCLES >= LAM_CYC_MAX -|     |

|OR --|

EGO_MON11 = 1 -------|     |

|AND -|

EGO_MON21 = 1 -------|

;overvoltage or RBIAS

;extreme.

EGO_TST_ENA = 1 -----------------|

;test is enabled from the       |

;diagnostic executive.          |

|

EGO_MON11 = 1 -------------------|

;upstream monitor done          |

;for bank1.                     |AND -| EGO_US_MON := 1

|     |    ;upstream monitor complete.

EGO_MON21 = 1 -------------------|

;upstream monitor done          |

;for bank2.                     |

|

EGO_US_MON = 0 ------------------|

11-15

EGO MONITOR CONTROL STRATEGY, EGO MONITOR CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EGO_TST_ENA = 1 -----------------|

;test is enabled from the       |

;diagnostic executive.          |

|

EGO_US_MON = 1 ------------------|AND -|  ;upstream monitor complete.

;upstream monitor complete.     |     |  ;AND valid results were achived.

|     | Do: forced_downstream_excursion

DS_STATE <> final_state ---------|     |  ;check downstream ego monitor

;final_state is defined in      |     |  ;and force rich and/or lean air

;the downstream ego monitor     |     |  ;fuel excursion to check

|     |  ;downstream sensors.

DS_HTR_TMR > DS_HEAT_TM ---------|

;downstream heaters are warm.

EGO_TST_ENA = 1 -----------------|

;test is enabled from the       |

;diagnostic executive.          |

|AND -|  ;downstream monitoring complete

EGO_DS_MON = 0 ------------------|     | Do: detect_ds_ego_malf

;downstream monitor is not done |     | EGO_DS_MON := 1

|

DS_STATE = final_state ----------|

EGO_US_MON = 1 ------|

|AND ----|

EGO_DS_MON = 1 ------|        |

|OR ---| CURR_TST_ENA := 1

CCM_EO_ENA = 1 ---------------|      |   ;once per drive.

|      |   ;perform current monitoring

CCM_ER_ENA = 1 ---------------|

EGO_DS_MON = 1 ------------------|

|

EGO_US_MON = 1 ------------------|AND -| EGO_MON := 1

|     |  ;ego monitoring complete.

EGO_HTR_MON = 1 -----------------|

unconditionally -----------------------| Do: bg_check_entry_conditions

|  ;continuous lack of switching test

| Do: ego_er_switching_test

|  ;koer hego test

| Do: ego_ffg_control

|  ;ffg_ego fault flag checks

END: ego_monitor_controller

11-16

EGO MONITOR CONTROL STRATEGY, EGO MONITOR CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ego_tst_rdy_control

;This process, called from the ego_monitor_controller once per background

;pass is used to control the interface to the OBD-II Diagnostic Executive

;via the EGO_TST_RDY flag.  The flag will be set during the upstream monitor

;while the speed/load window is open. After completion of the

;upstream monitor and during the execution of the downstream monitor and

;afterward until the ego heater monitor is complete.

EGO_WIND_SET = 1 ----------------------|

|

EGO_US_MON = 1 ------------------|     |

|AND -|

STOP_EXCURS = 0 -----------------|     |OR --| EGO_TST_RDY := 1

|     |

EGO_DS_MON = 1 ------------------|     |     |

|     |     |

EGO_HTR_MON = 0 -----------------|AND -|     |

|           |

EGO_US_MON = 1 ------------------|           |

| --- ELSE ---

|

| EGO_TST_RDY := 0

END: ego_tst_rdy_control

BEGIN: disable_fuel_modulation

;This process is used to abort the EGO monitor in the middle of execution

;due to local driving conditions falling outside the operating window or

;diagnostic executive intervention. This process is initiated from within

;the EGO monitor controller.

;It places the EGO monitor in a state ready to be run again when

;the entry conditions have been again met.

unconditionally -----------------------------| LAM_MOD_FLG := 0

|  ;disable fuel modulation.

| SMPL_EGO_FLG := 0

|  ;disable 16ms sampling.

| EGO_REV_ENA := 0

| EGO_REV_CTR := 0

|  ;disable rev counting.

| EGO_STAT11 := 0

| EGO_STAT21 := 0

END: disable_fuel_modulation

11-17

EGO MONITOR CONTROL STRATEGY, UPSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

11.1.2  UPSTREAM EGO MONITOR (CDAM0)

OVERVIEW

+----------------------------------+

| EGO_US_MONITOR                   |

+----------------------------------+

|PUBLIC PROCEDURES:                |

|. check_upstream_entry_conditions |

|. check_upstream_results          |

|. upstream_ego_monitor            |

|. upstream_ego_sampling           |

|. upstream_initialization         |

|. upstream_overvoltage_check      |

|. update_ego_cl_tmr               |

|. check_upstream_static_shift     |

|. reset_us_ego_monitor            |

|. reset_overvoltage_us            |

+----------------------------------+

|PRIVATE PROCEDURES:               |

|. calculate_ego_stats(n)          |

|. detect_us_ego_malf(n)           |

|. ego_voltage_calculation         |

|. slow_ego_malf(n)                |

|. no_slow_ego_malf(n)             |

+----------------------------------+

This module is used to perform all  the  actions  necessary  to  monitor  the

upstream   EGO  sensors  per  OBD-II  regulations.   It  is  used  for  FAOSC

applications for single bank, dual bank,and Y-Pipe applications.  It contains

a  number  of  PUBLIC  processes  that  act as seperate entry points into the

module.  See the descriptions below and within the module at the site of each

of the procedures for a complete description of what the module does.

This      module       contains       processes       (upstream_ego_sampling,

ego_voltage_calculation) designed to sample the IEGOn_NEW {n:  11, 21} analog

inputs and record the peak rich and lean values seen.  Below is a crude graph

of one half cycle in a typical EGO sensor output signal.  This shows the rich

half cycle, the lean half cycle is similar.

This module also contains a process (check_upstream_entry_conditions) used to

enable  the  OBD-II EGO monitor.  For each of the entry conditions to be met,

an acceptable window of operations is calibrated to be the  range  where  the

EGO  monitor could be run.  Once initiated, a tighter window is chosen and if

the operating state falls out of this region,  the  test  is  aborted.   This

gives  the  calibrator  the  flexibility to define the necessary steady state

conditions that must be maintained during the test.  The test will attempt to

continually enable the monitor when the appropriate conditons are met.  It is

the resonsibility of the OBD-II Operating System  Executive  to  disable  the

monitor when it is no longer required.

11-18

EGO MONITOR CONTROL STRATEGY, UPSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

+--(VEGORH_BARn - VEGO_DLTA)              VEGOn_NEW ^

|                                                   |

|   VEGO_PK_RHn ------>+                            +------>

|       ______________- -__________                   time

V      /                           \

- - - -.- - - - - - - - - - - - - - -.- - - -

/                               \

/ .                             . \       US_SWTPOINT -+

/                                   \                   |

/   .                             .   \                  V

------------------------------------------------------------------------

/.    .                             .     \

/                                           \

/  .    .                             .       \

/                                               \

/    .    .                             .         \

-----/                                                   \---------------

.    .                             .

1               +-----------------------------+

VEGO_RHAVGn     |                             |

0---------------+                             +--------------------------

1          +----+

PK_RH_FLGn |    |

0----------+    +--------------------------------------------------------

Each EGO sensor will be sampled  once  every  16  milliseconds  in  order  to

accurately  capture  the  peak  voltage  achieved  during  the  rich  or lean

excursion from stoichiometery.  One of the upstream sensors will  be  sampled

at each 8 millisecond time interval as shown below:

time(ms)     0   2   4   6   8  10  12  14  16  18   20  22  24  26  28  ...

EGO sampled 11              21              11               21         ...

Additional processes are used to continuously monitor EGO sensor voltage  for

an  overvoltage  condition.  When one is encountered a malfunction is flagged

to the diagnostic executive and no further test of the  upstream  sensors  is

allowed.   The RBIAS statistic is aslo check once per background pass against

a pair of limits.  If it is found to exceed these limits, all further testing

of the sensor is discontinued.

11-19

EGO MONITOR CONTROL STRATEGY, UPSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  ECT = Coolant temperature, degrees farhenheit.

-  EGO11_FREQ = Calculated EGO11 average  switching  frequency  during  area

ratio  calculation.   Range  = 0 - 15Hz, Resolution = 0.0625, Initially =

0.0

-  EGO11_TIME = Elapsed  time  to  accumulate  MAX_CYCLES  number  of  EGO11

cycles.   Maximum  value represents 1.5Hz switch frequency for 20 cycles.

Range = 0 - 14 seconds, Resolution = 0.0625 second, Initially = 0.0.

-  EGO21_FREQ = Calculated EGO21 average  switching  frequency  during  area

ratio  calculation.   Range  = 0 - 15Hz, Resolution = 0.0625, Initially =

0.0

-  EGO21_TIME = Elapsed  time  to  accumulate  MAX_CYCLES  number  of  EGO21

cycles.   Maximum  value represents 1.5Hz switch frequency for 20 cycles.

Range = 0 - 14 seconds, Resolution = 0.0625 second, Initially = 0.0.

-  EGO_CL_TMR = Elapsed time  since  entering  closed  loop  while  the  EGO

monitor is enabled.

-  EGO_LOAD_ST = LOAD upon entry to the EGO monitor.

-  EGO_MON_TMR = Timer used  to  measure  length  of  time  EGO  monitor  is

enabled, seconds.

-  EGO_N_ST = Engine speed upon entry to the EGO monitor.

-  EGO_VS_ST = VS upon entry to the EGO monitor.

-  FAILED_BANK = Register indicating which bank is in lack of EGO switching,

unitless.

-  FAO_MON_TMR1 = Total accumulated time in the  integral  learning  window,

seconds.

-  FAO_MON_TMR2 = Total accumulated time in the  integral  learning  window,

seconds.

-  IEGOn_NEW = Value of A-to-D converter for the EGO channel, counts.

-  LAMAVE1 = Average of LAMBSE1.  Averages  the  value  of  LAMBSE1  at  the

current EGO-1 switch and last EGO-1 switch.

-  LAMAVE2 = Average of LAMBSE2.  Averages  the  value  of  LAMBSE2  at  the

current EGO-2 switch and last EGO-2 switch.

-  LD_EGO_DLTA = Absolute change in LOAD since last background pass, used to

determine steady state for the EGO monitor.

-  LEAN_COUNT11 = Count of lean samples taken at  16  millisecond  intervals

for  the  area  ratio  calculation.   Range  =  0  - 900, Resolution = 1,

Initially = 0.

11-20

EGO MONITOR CONTROL STRATEGY, UPSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  LEAN_COUNT21 = Count of lean samples taken at  16  millisecond  intervals

for  the  area  ratio  calculation.   Range  =  0  - 900, Resolution = 1,

Initially = 0.

-  LOAD_EGO = Filtered LOAD for use in the EGO monitor.

-  N_EGO = Filtered engine speed for use in the EGO monitor.

-  N_EGO_DLTA = Absolute change in engine speed since last background  pass,

used to determine steady state for the EGO monitor.

-  OV_TMR_11 = EGO11 overvoltage timer.

-  OV_TMR_21 = EGO21 overvoltage timer.

-  PxxxMALF = Obdii malfunction flag for fault xxx.

-  R_BIAS1 = Rear BIAS trim for bank one.

-  R_BIAS2 = Rear BIAS trim for bank two.

-  R_BIAS_AVG1 = The average value of the integral value of R_BIAS1.

-  R_BIAS_AVG2 = The average value of the integral value of R_BIAS2.

-  RICH_COUNT11 = Count of rich samples taken at  16  millisecond  intervals

for  the  area  ratio  calculation.   Range  =  0  - 900, Resolution = 1,

Initially = 0.

-  RICH_COUNT21 = Count of rich samples taken at  16  millisecond  intervals

for  the  area  ratio  calculation.   Range  =  0  - 900, Resolution = 1,

Initially = 0.

-  SAMPLE_CNT11 = Count of the number of samples taken on EGO11  during  the

ego monitor.

-  SAMPLE_CNT21 = Count of the number of samples taken on EGO21  during  the

ego monitor.

-  SWITCH_CNT11 = Count of the number  of  EGO11  switches  during  the  ego

monitor.

-  SWITCH_CNT21 = Count of the number  of  EGO21  switches  during  the  ego

monitor.

-  S_USVEGOn1 = Bank n upstream HEGO voltage, minus hardware voltage bias.

-  SWTP_BARn1 =  The  filtered  adaptive  switchpoint  calculated  for  HEGO

upstream sensor bank n.

-  VEGO_AMP11 = Amplitude (volts) of difference between rich  and  lean  for

bank 1, fore HEGO.

-  VEGO_AMP12 = Amplitude (volts) of difference between rich  and  lean  for

bank 1, aft.

-  VEGO_AMP21 = Amplitude (volts) of difference between rich  and  lean  for

bank 2, fore HEGO.

11-21

EGO MONITOR CONTROL STRATEGY, UPSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  VEGO_AMP22 = Amplitude (volts) of difference between rich  and  lean  for

bank 2, aft HEGO.

-  VEGO_DLTA = Voltage delta from rich/lean average below/above  which  peak

value is recorded, volts.

-  VEGO_LSTn = Last calculated EGO voltage, volts.

-  VEGOLN_BARn = Average lean EGO voltage, volts.

-  VEGOn_NEW = Calculated EGO voltage, volts.

-  VEGOPKLEANn = Average peak lean EGO voltage, volts.

-  VEGO_PK_LNn = Instantaneous peak lean EGO voltage, volts.

-  VEGO_PK_RH11 = Instantaneous peak rich EGO voltage, volts.

-  VEGO_PK_RH21 = Instantaneous peak rich EGO voltage, volts.

-  VEGOPKRICHn = Average peak rich EGO voltage, volts.

-  VEGORH_BARn = Average rich EGO voltage, volts.

-  VS_EGO = Filtered VS for use in the EGO monitor.

-  VS_EGO_DLTA = Absolute change in VS since last background pass,  used  to

determine steady state for the EGO monitor.

Bit Flags:

-  EGOMON_ENTRY = Flag indicating EGO monitor has been entered; 1 -> monitor

entered.

-  EGO_MON_RDY = Flag indicating EGO monitor is  ready  to  look  for  entry

conditions; 1 -> monitor ready.

-  EGO_MON11 = EGO11 monitored flag; 1 -> sensor monitored.

-  EGO_MON21 = EGO21 monitored flag; 1 -> sensor monitored.

-  EGO_STAT11 = Flag indicating the first EGO11 switch has been encountered,

start the area ratio calculation.

-  EGO_STAT21 = Flag indicating the first EGO21 switch has been encountered,

start the area ratio calculation.

-  EGO_WIND_SET = Flag indicating EGO monitor steady state window conditions

have been set; 1 -> window conditions set.

-  FRSTSWITCH11 = Flag indicating if the first switch seen during area ratio

calculation  was  rich-to-lean  (0)  or lean-to-rich (1).  Range = 0 - 1,

Resolution = 1, Initially = 0.

-  FRSTSWITCH21 = Flag indicating if the first switch seen during area ratio

calculation  was  rich-to-lean  (0)  or lean-to-rich (1).  Range = 0 - 1,

Resolution = 1, Initially = 0.

11-22

EGO MONITOR CONTROL STRATEGY, UPSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  LST_SWITCH11 = Flag indicating if the last switch seen during area  ratio

calculation  was  rich-to-lean  (0)  or lean-to-rich (1).  Range = 0 - 1,

Resolution = 1, Initially = 0.

-  LST_SWITCH21 = Flag indicating if the last switch seen during area  ratio

calculation  was  rich-to-lean  (0)  or lean-to-rich (1).  Range = 0 - 1,

Resolution = 1, Initially = 0.

-  OLFLG = Open loop fuel flag.  1 -> Open Loop fuel; 0 -> Closed loop fuel

-  OV_DOWN_11 = Flag, when set triggers OV_TMR_11 to decrement.

-  OV_DOWN_21 = Flag, when set triggers OV_TMR_21 to decrement.

-  OV_UP_11 = Flag, when set, triggers OV_TMR_11 to increment.

-  OV_UP_21 = Flag, when set triggers OV_TMR_21 toi increment.

-  OV_FMEM_n1 = Flag which is set when  the  overvoltage  timer  triggers  a

fault  condition.   This  flag  gets  cleared  when the overvoltage timer

starts counting down after fault condition clears.

-  PK_LN_FLGn = Flag indicating the lean peak cannot be updated yet; 1 -> do

not update the peak lean value.

-  PK_RH_FLGn = Flag indicating the rich peak cannot be updates yet; 1 -> do

not update the peak rich value.

-  SMPL_EGO_FLG = Flag used to enable the  foreground  EGO  samling  process

used on the upstream EGO sensors.  1 -> enable the sampling.

-  SWTP_STABLn1  =  Flag  indicating  that  the  adaptive   switchpoint   is

stabalized at either normal or csd voltage levels.

-  VEGO_LNAVGn = Flag indicating when lean EGO voltage averaging/peak update

is allowed; 1 -> allow update.

-  VEGO_RHAVGn = Flag indicating when rich EGO voltage averaging/peak update

is allowed; 1 -> allow update.

Calibration Constants:

-  ACT_CATEGOMX = Maximum ACT to allow the CAT and EGO  monitors  to  begin.

Units:  Degress F, Range:  120 - 254, Resolution:  2, Base value:  254.

-  DSRD_FREQ = Desired frequency of LAMBSE fuel modulation, Hertz.

-  EGO_CL_TM = Elapsed time in closed loop with EGO monitor  enabled  before

running the upstream EGO monitor.

-  ECT_EGO_MIN = Minimum engine coolant temperature to  allow  upstream  EGO

sensor  monitoring.  Calibration Constant, Units:  Degrees F, Range:  0 -

254, Resoultion:  2.0.  Base value:  0.

-  ECT_EGO_MAX = Maximum engine coolant temperature to  allow  upstream  EGO

sensor  monitoring.  Calibration Constant, Units:  Degrees F, Range:  0 -

254, Resoultion:  2.0.  Base value:  0.

11-23

EGO MONITOR CONTROL STRATEGY, UPSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FAOS_MON_TM = Accumulated time  in  the  FAOS  integral  learning  window

before a malfunction can be signalled, seconds.

-  FREQERR_BAND = Error from DSRD_FREQ allowable to accept the results  from

the upstream EGO monitor.  Calibration Constant, Units:  HERTZ, Range:  0

- 15, Resolution:  0.0625.  Base value:  15.

-  HEGO_FC_AVG = EGO filter constant.

-  HEGO_FC_PEAK = EGO filter constant for peak voltages.  Range = 0  -  1.0,

Resolution = 0.03125, Base value = 0.875.

-  LAMAVE_MAX = Maximum LAMAVE to enter the EGO monitor.

-  LAMAVE_MIN = Minimum LAMAVE to enter the EGO monitor.

-  LDEGO_DLTMX = Maximum absolute change in LOAD since entering EGO  monitor

to keep the test enabled.

-  LEAN_AMP_M = Upstream EGO voltage amplitude to signal a failure when  the

area  ratio  is not shifted rich or lean (median).  Calibration Constant,

Units:  VOLTS, Range:  0 - 1, Resolution:  0.001.  Base value:  0.

-  LOAD_EGO_MAX = Maximum LOAD to enter the EGO monitor.

-  LOAD_EGO_MIN = Minimum LOAD to enter the EGO monitor.

-  MAX_CYCLES = The number of cycles over which the  area  ratio  is  to  be

calculated.  Range = 0 - 40, Resolution = 1, Base value = 10.

-  MAX_EGO_VOLT  =  EGO  voltage  at  which  an  overvoltage  condition   is

signalled.    Calibration   Constant,  Units:   VOLTS,  Range:   0  -  2,

Resolution:  0.001.  Base value:  2.

-  NEGO_DLTMX = Maximum absolute change in engine speed since  entering  EGO

monitor to keep the test enabled.

-  N_EGO_MAX = Maximum engine speed to enter the EGO monitor.

-  N_EGO_MIN = Minimum engine speed to enter the EGO monitor.

-  NUMEGO = Number of EGO sensors.

-  OV_CLEAR_TM = Reset overvoltage timer to this value after fault condition

clears, seconds.

-  OV_TIME = Time at overvoltage on the EGO sensor to signal a malfunction.

-  RBIAS_EGO_MX = Maximum allowable value of R_BIAS.

-  RBIAS_EGO_MN = Minimum allowable value of R_BIAS.

-  RICH_AR = AREA_RATIO threshold to use the rich voltage thresholds for the

upstream  EGO  monitor  fault  detection.   Calibration  Constant, Units:

NONE, Range:  0 - 5, Resolution:  0.03125.  Base value:  0.

-  VSEGO_DLTMX = Maximum absolute change in VS since entering EGO monitor to

keep the test enabled.

11-24

EGO MONITOR CONTROL STRATEGY, UPSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  VS_EGO_MAX = Maximum VS to enter the EGO monitor.

-  VS_EGO_MIN = Minimum VS to enter the EGO monitor.

-  US_SWTPOINT = Upstream EGO input switch point, volts.

OTHER

-  upstream_ego_codes = set of (P0132, P0133, P1130,  P1131,  P1132,  P1133,

P1134,  P0152,  P0153,  P1150,  P1151,  P1152, P1153, P1154).  The set of

OBD-II fault codes that relate to the ego monitor.

-  malfunction(ego, Pxxx) = Public procedure updating the MIL control status

provided by the MIL control module.

-  P0131 = EGO sensor circuit out of range low (bank 1, sensor 1).

-  P0132 = Oxygen sensor circuit high voltage (bank 1 upstream).

-  P0133 = Oxygen sensor circuit slow response (bank 1 upstream).

-  P0135 = Fault code for EGO11 sensor heater circuit malfunction.

-  P0151 = EGO sensor circuit out of range low (bank 2, sensor 1).

-  P0152 = Oxygen sensor circuit high voltage (bank 2 upstream).

-  P0153 = Oxygen sensor circuit slow response (bank 2 upstream).

-  P0155 = Fault code for EGO21 sensor heater circuit malfunction.

11-25

EGO MONITOR CONTROL STRATEGY, UPSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EGO_US_MONITOR_COM10

BEGIN: upstream_initialization

;To be performed during RAM initialization or when called from within the

;EGO monitor control process (during an abort_ego_mon sequence).

For n = 11, 21:

unconditionally -------------------------| VEGORH_BARn := US_SWTPOINT

| VEGOLN_BARn := US_SWTPOINT

| VEGOPKRICHn := US_SWTPOINT

| VEGOPKLEANn := US_SWTPOINT

| VEGOn_NEW := US_SWTPOINT

| VEGO_LSTn := US_SWTPOINT

END: upstream_initialization

BEGIN: reset_overvoltage_us

;The following process is called from the EGO montior control process

;and is used to freeze the overvoltage timers when the test is not

;is not currently enabled.  This will prevent the timers from being

;started while enabled and then become dissabled, but keep running.

unconditionally -------------------------| OV_UP_11 := 0

| OV_UP_21 := 0

| OV_DOWN_11 := 0

| OV_DOWN_21 := 0

|  ;freeze both timers.

END: reset_overvoltage_us

11-26

EGO MONITOR CONTROL STRATEGY, UPSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: upstream_overvoltage_check

;This process, initiated from the EGO monitor control process is used to

;detect an overvoltage condition on the EGO sensor. once detected, the

;upstream EGO monitor is disabled.

P0135MALF = 0 ---------------------|

|AND -| OV_UP_11 := 1

VEGO11 > MAX_EGO_VOLT -------------|     | OV_DOWN_11 := 0

;Check for overvoltage when heater     | ;count up OV_TMR_11

;is functional only.                   |

|

| --- ELSE ---

|

| OV_UP_11 := 0

| OV_DOWN_11 := 1

| ;count down OV_TMR_11

NUMEGO = 2 ------------------------|

|

P0155MALF = 0 ---------------------|AND -| OV_UP_21 := 1

|     | OV_DOWN_21 := 0

VEGO21 > MAX_EGO_VOLT -------------|     | ;count up OV_TMR_21

;Check for overvoltage.                 |

| --- ELSE ---

|

| OV_UP_21 := 0

| OV_DOWN_21 := 1

| ;count down OV_TMR_21

OV_DOWN_11 = 1 --------------------|

|AND -| OV_TMR_11 := OV_CLEAR_TM

OV_FMEM_11 = 1 --------------------|     |  ;begin OV timer countdown

| OV_FMEM_11 := 0

|  ;clear OV countdown flag

OV_TMR_11 > OV_TIME ---------------------| Do: malfunction(ego, P0132)

|  ;high voltage on ego11.

| EGO_MON11 := 1

|  ;lock out the U.S. ego

|  ;monitor for this bank.

| OV_FMEM_11 := 1

|  ;arm OV countdown flag

|

| --- ELSE ---

|

OV_TMR_11 = 0 ---------------------------| Do: clear_malf(P0132)

OV_DOWN_21 = 1 --------------------|

|AND -| OV_TMR_21 := OV_CLEAR_TM

OV_FMEM_21 = 1 --------------------|     |  ;begin OV timer countdown

| OV_FMEM_21 := 0

|  ;clear OV countdown flag

11-27

EGO MONITOR CONTROL STRATEGY, UPSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OV_TMR_21 > OV_TIME ---------------------| Do: malfunction(ego, P0152)

|  ;high voltage on ego21.

| EGO_MON21 := 1

|  ;lock out the U.S. ego

|  ;monitor for this bank.

| OV_FMEM_21 := 1

|  ;arm OV fault flag

|

| --- ELSE ---

|

OV_TMR_21 = 0 ---------------------------| Do: clear_malf(P0152)

END: upstream_overvoltage_check

11-28

EGO MONITOR CONTROL STRATEGY, UPSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: upstream_ego_sampling

;This process is executed once every 16 milliseconds for each of the

;upstream EGO sensors.  That implies that the process is executed once evey

;8 milliseconds, alternating between bank 1 EGO sensor and bank 2 EGO

;sensor sampling (if NUMEGO = 2).

;The results are stored in a RAM buffer. Each background loop the buffer is

;emptied and the upstream_ego_monitor process is performed for each value

;that was stored in the buffer.  The register IEGOn_NEW will be updated at

;an 16ms rate and available via the RCON or similar device.

SMPL_EGO_FLG = 1 ------------------|

|AND -| Sample EGO11 and place in

16ms elapsed since                 |     | buffer and in IEGO11_NEW.

IEGO11_NEW was sampled --------|     |  ;the entry is also tagged with

|  ;sensor number 11.

NUMEGO = 2 ------------------------|

|

SMPL_EGO_FLG = 1 ------------------|AND -| Sample EGO21 and place in

|     | buffer and in IEGO21_NEW.

16ms elapsed since                 |     |  ;the entry is also tagged with

IEGO21_NEW was sampled --------|     |  ;sensor number 21.

END: upstream_ego_sampling

BEGIN: upstream_ego_monitor

;The following process is performed once for each entry in the buffer. 'n'

;is determined from the entry in the buffer.

;

;Convert the HEGO sensor A/D counts to voltage using the same process

;as described in the ego input processing module (please refer to the

;ego_input_com10 module).  For the CSD compensation voltage (csd_bias)

;which normalizes VEGOxy_NEW, an averaged value is used for the adaptive

;switchpoint parameter.

;

unconditionally --------------------| VEGO_LSTn = VEGOn_NEW

| S_USVEGOn = ego_hw_xfer_fn(IEGOn_NEW)

|   ;convert A/D counts to volts

| csd_bias = US_SWTPOINT - SWTP_BARn

| VEGOn_NEW = max(S_USVEGOn + csd_bias,0)

| Do: ego_voltage_calculation(n)

| Do: frequency_calculation(n)

END: upstream_ego_monitor

11-29

EGO MONITOR CONTROL STRATEGY, UPSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: frequency_calculation(n)

;This process is used to calculate the parameters necessary to calculate the

;EGO_FREQn statistic.  This statistic is used to reject the results of a

;very slow EGO sensor.  This process is initiated from within the

;upstream_ego_monitor process.

EGO_MONn = 1 ---------------------------| EXIT this process.

EGO_STATn = 0 --------------------------| EGO_STATn := 1

| SAMPLE_CNTn := 0

| SWITCH_CNTn := 0

| VEGO_LSTn := VEGOn_NEW

| EGOn_FREQ := 0

|   ;if this bank has not completed,

|   ;re-initalize the statistics and

|   ;set VEGO_LST = VEGO_NEW to

|   ;prevent SWITCH_CNT to increment

|   ;on a dead sensor.

VEGOn_NEW >= US_SWTPOINT ---|

|AND -|

VEGO_LSTn < US_SWTPOINT ----|     |

|OR --| SWITCH_CNTn := SWITCH_CNTn + 1

VEGOn_NEW < US_SWTPOINT ----|     |

|AND -|

VEGO_LSTn >= US_SWTPOINT ---|

unconditionally ------------------------| SAMPLE_CNTn := SAMPLE_CNTn + 1

END: frequency_calculation(n)

11-30

EGO MONITOR CONTROL STRATEGY, UPSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ego_voltage_calculation(n)

;This process is used to calculate EGO voltage statistics used to determine

;the EGO signal amplitude.  EGO signal amplitude is a measure of the

;response rate of the sensor and has been correlated to emmision levels.

;This process is initiated from within the upstream_ego_monitor process.

;NOTE: n = sensor number (11, 21).

VEGOn_NEW > (VEGORH_BARn - VEGO_DLTA) ---| VEGO_RHAVGn = 1

(rich - allow updating of rich          |  (rich updates allowed)

average)                               |

| PK_RH_FLGn = 0

|  (rich peak will be updated)

|

| --- ELSE ---

|

| VEGO_RHAVGn = 0

|  (rich updates not allowed)

VEGOn_NEW < (VEGOLN_BARn + VEGO_DLTA) ---| VEGO_LNAVGn = 1

(lean - allow updating of lean peak     |  (lean updates allowed)

and lean average)                      |

| PK_LN_FLGn = 0

|  (peak lean will be updated)

|

| --- ELSE ---

|

| VEGO_LNAVGn = 0

|  (lean updates not allowed)

VEGOn_NEW >= US_SWTPOINT ----------|

(rich updates allowed)            |

|

VEGO_LSTn < US_SWTPOINT -----------|AND -| VEGOPKLEANn =

(rich updates were not allowed    |     | rolav(VEGO_PK_LNn, HEGO_FC_PEAK)

last sample)                     |     |   (filter the last peak lean

|     |    voltage seen)

PK_LN_FLGn = 0 --------------------|

(average lean + delta attained)

VEGOn_NEW >= US_SWTPOINT ----------|

(rich updates allowed)            |AND -| VEGO_PK_RHn = US_SWTPOINT

|     |  (reset peak rich value)

VEGO_LSTn < US_SWTPOINT -----------|     |

(rich updates were not allowed          | PK_LN_FLGn = 0

last sample)                           |  (stop looking for peak lean

|   value)

|

| PK_RH_FLGn = 1

|  (look for new peak rich value)

VEGO_RHAVGn = 1 -------------------|

(rich updates allowed)            |AND -| VEGO_PK_RHn = VEGOn_NEW

|     |  (update peak rich voltage seen

VEGOn_NEW > VEGO_PK_RHn -----------|     |   this cycle)

(new rich peak voltage observed)

11-31

EGO MONITOR CONTROL STRATEGY, UPSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

VEGO_RHAVGn = 1 -------------------------| VEGORH_BARn =

(rich updates allowed)                  | rolav(VEGOn_NEW, HEGO_FC_AVG)

|  (average in last rich value)

VEGOn_NEW < US_SWTPOINT -----------|

(lean updates allowed)            |

|

VEGO_LSTn >= US_SWTPOINT ----------|AND -| VEGOPKRICHn =

(lean updates were not allowed    |     | rolav(VEGO_PK_RHn, HEGO_FC_PEAK)

last sample)                     |     |  (filter the last peak rich

|     |   voltage seen)

PK_RH_FLGn = 0 --------------------|

(average rich + delta attained)

VEGOn_NEW < US_SWTPOINT -----------|

(lean updates allowed)            |AND -| VEGO_PK_LNn = US_SWTPOINT

|     |  (reset peak lean value)

VEGO_LSTn >= US_SWTPOINT ----------|     |

(lean updates were not allowed          | PK_RH_FLGn = 0

last sample)                           |  (stop looking for peak rich

|   value)

|

| PK_LN_FLGn = 1

|  (look for new peak lean value)

VEGO_LNAVGn = 1 -------------------|

(lean updates allowed)            |AND -| VEGO_PK_LNn = VEGOn_NEW

|     |  (update peak lean voltage seen

VEGOn_NEW < VEGO_PK_LNn -----------|     |   this cycle)

(new lean peak voltage observed)

VEGO_LNAVGn = 1 -------------------------| VEGOLN_BARn =

(lean updates allowed)                  | rolav(VEGOn_NEW, HEGO_FC_AVG)

|  (average in last lean value)

END: ego_voltage_calculation

11-32

EGO MONITOR CONTROL STRATEGY, UPSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN:  check_upstream_entry_conditions

;Once per background loop, when EGO_TST_SW=1 perform the following:

; Check switchpoint stability as part of the Upstream monitor entry

; conditions.  The switchpoint stable flags are set in the adaptive

; switchpoint algorithm and indicate that the switchpoint has

; stabalized at either normal or csd voltage levels.

;

SWTP_STABL11 = 1 -----------------|

|AND -| swtpoint_rdy := true

SWTP_STABL21 = 1  ----------------|     |

| --- ELSE ---

|

| swtpoint_rdy := false

swtpoint_rdy = true ---------------|

|

EGO_TEST_MON = 1 ------------------|

|

EGO_CL_TMR > EGO_CL_TM ------------|

|

LAMAVE1 < LAMAVE_MAX --------------|

|

LAMAVE1 > LAMAVE_MIN --------------|

|

LAMAVE2 < LAMAVE_MAX --------|     |

|OR --|

NUMEGO = 1 ------------------|     |

|

LAMAVE2 > LAMAVE_MIN --------|     |

|OR --|

NUMEGO = 1 ------------------|     |

|

ECT < ECT_EGO_MAX -----------------|AND -| EGO_MON_RDY := 1

|     |  ;allow entry to or continuation

ECT > ECT_EGO_MIN -----------------|     |  ;of EGO monitor

|     |

OLFLG = 0 -------------------------|     |

(Closed Loop Fuel)                |     |

|     |

ACT < ACT_CATEGOMX ----------------|     |

| --- ELSE ---

|

| EGO_MON_RDY := 0

|  ;abort monitor

11-33

EGO MONITOR CONTROL STRATEGY, UPSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EGO_MON_RDY = 1 -------------------|

|

LOAD_EGO < LOAD_EGO_MAX -----------|

|

LOAD_EGO > LOAD_EGO_MIN -----------|

|

VS_EGO < VS_EGO_MAX ---------------|AND -| ego_mon_entry := 1

|     |  ;entry conditions met,

VS_EGO > VS_EGO_MIN ---------------|     |  ;start monitoring

|     |

N_EGO < N_EGO_MAX -----------------|     |

|     |

N_EGO > N_EGO_MIN -----------------|     |

| --- ELSE ---

|

| ego_mon_entry := 0

|  ;don't start monitoring yet

11-34

EGO MONITOR CONTROL STRATEGY, UPSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;Once the entry conditions have been met, a tighter  window  defining  steady

;state  for  the  EGO  monitor  is  used  to determine if the EGO monitor can

;continue to execute.  Capture the starting point:

EGOMON_ENTRY = 0 ------------------|

|

ego_mon_entry = 1 -----------------|AND -| EGO_WIND_SET := 1

|     | EGO_VS_ST := VS_EGO

EGO_WIND_SET = 0 ------------------|     | EGO_LOAD_ST := LOAD_EGO

| EGO_N_ST := N_EGO

unconditionally -------------------------| EGOMON_ENTRY := ego_mon_entry

Compute the absolute variation from the entry condition:

EGO_WIND_SET = 1 ------------| VS_EGO_DLTA := abs(EGO_VS_ST - VS_EGO)

| LD_EGO_DLTA := abs(EGO_LOAD_ST - LOAD_EGO)

| N_EGO_DLTA := abs(EGO_N_ST - N_EGO)

:Exit the EGO monitor if steady state is breached and record the elapsed time

:since entering steady state:

EGO_WIND_SET = 1 ------------------|

|

VS_EGO_DLTA > VSEGO_DLTMX ---|     |

|     |AND -| EGO_WIND_SET := 0

LD_EGO_DLTA > LDEGO_DLTMX ---|     |     | EGOMON_ENTRY := 0

|     |     |  ;abort monitor

N_EGO_DLTA > NEGO_DLTMX -----|     |

|OR --|

LOAD_EGO >= LOAD_EGO_MAX ----|

|

LOAD_EGO <= LOAD_EGO_MIN ----|

|

EGO_MON_RDY = 0 -------------|

END:  check_upstream_entry_conditions

11-35

EGO MONITOR CONTROL STRATEGY, UPSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: check_upstream_results

;This process is called from the ego_monitor_controller to compute

;amplitude and check it against calibrated limits.

EGO_MON11 = 0 ---------------------------| Do: calculate_ego_stats(11)

;this sensor not monitored yet.

;this implies not overvoltaged.

NUMEGO = 1 ------------------------------| VEGO_AMP21 := 0xffffh

;single bank applications               |  ;set to all '1's

|

| --- ELSE ---

NUMEGO = 2 ------------------------|     |

|AND -| Do: calculate_ego_stats(21)

EGO_MON21 = 0 ---------------------|

;this sensor not monitored yet.

;this implies not overvoltaged.

END: check_upstream_results

BEGIN: calculate_ego_stats(n)

;This procedure calculates voltage envelope of the selected

;ego sensor. 'n' represents the sensor for which the voltage

;envelope is to be calculated.  This procedure is called after the upstream

;test is complete with valid results.

unconditionally ------------------------| ego_sampling_period = 0.016 sec.

| EGOn_FREQ := (SWITCH_CNTn/

| (SAMPLE_CNTn*ego_sample_period*2))

| EGO_STATn := 0

abs(EGOn_FREQ - DSRD_FREQ)

<= FREQERR_BAND --------------| VEGO_AMPn := (VEGOPKRICHn -

;good test, within tolerance band.     |               VEGOPKLEANn)

| EGO_MONn := 1

| Do: detect_us_ego_malf(n)

|

| --- ELSE ---

|

EGOn_FREQ < EGO_FREQ_MIN ---------------| VEGO_AMPn := 0

;extremely slow sensor or shifted      | EGO_MONn := 1

;fuel system.  Signal a fault.         | Do: detect_us_ego_malf(n)

|

| --- ELSE ---

|

|  ;re-run the test.

| EGO_MONn := 0

END: calculate_ego_stats(n)

11-36

EGO MONITOR CONTROL STRATEGY, UPSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: detect_us_ego_malf(n)

;This procedure determines if an upstream ego sensor has malfunctioned to a

;point where it is causing the vehicle exhaust emmisions to exceed the

;levels that require illumination of the malfunction indicator light.  'n'

;represents the sensor for which the determination is to be made.  This

;procedure is called after the voltage envelope has been

;calculated.

VEGO_AMPn < LEAN_AMP_M ----------------------| Do: slow_ego_malf(n)

|

| --- ELSE ---

|

| Do: no_slow_ego_malf(n)

END: detect_us_ego_malf(n)

11-37

EGO MONITOR CONTROL STRATEGY, UPSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: slow_ego_malf(n)

;This procedure will set the appropriate malfunction code when a fault is

;detected and signal the OBD-II Diagnostic Executive that a malfunction

;has occured.

n = 11 ----------------------------------| Do: malfunction(ego,P0133)

|

| --- ELSE ---

|

n = 21 ----------------------------------| Do: malfunction(ego,P0153)

END: slow_ego_malf(n)

BEGIN: no_slow_ego_malf(n)

;This procedure will clear the appropriate malfunction code when a fault is

;not detected.

n = 11 ----------------------------------| Do: clear_malf(P0133)

|

| --- ELSE ---

|

n = 21 ----------------------------------| Do: clear_malf(P0153)

END: no_slow_ego_malf(n)

BEGIN: update_ego_cl_tmr

;PUBLIC process executed on demand from the ego_monitor_controller process

;and done once per background pass to determine the elapsed

;time since entering closed loop fuel control while the EGO monitor has

;been enabled.  EGO_CL_TMR is a free running upcounting timer.

OLFLG = 1 -------------------------------| EGO_CL_TMR := 0

END: update_ego_cl_tmr

BEGIN: reset_us_ego_monitor

;The following logic process is used to reset the KAM parameters VEGO_AMP

;on KAM reset.  It is called from the EGO monitor control module.

unconditionally ------------------------| VEGO_AMP11 := 1

| VEGO_AMP21 := 1

| EGO11_FREQ := 0

| EGO21_FREQ := 0

END: reset_us_ego_monitor

11-38

EGO MONITOR CONTROL STRATEGY, UPSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: check_upstream_static_shift

;This PUBLIC logic process is called on demand from the

;ego_monitor_controller process once per background pass when the

;appropriate conditions have been met.

FAO_MON_TMR1 >= FAOS_MON_TM -------|

|

FAILED_BANK <> 1 ------------------|AND -| malfunction(ego,P1134)

|     | EGO_MON11 := 1

R_BIAS_AVG1 > RBIAS_EGO_MX --------|     |

;rich shift on ego11                    |

| --- ELSE ---

|

| Do: clear_malf(P1134)

FAO_MON_TMR1 >= FAOS_MON_TM -------|

|

FAILED_BANK <> 1 ------------------|AND -| malfunction(ego,P1133)

|     | EGO_MON11 := 1

R_BIAS_AVG1 < RBIAS_EGO_MN --------|     |

;lean shift on ego11                    |

| --- ELSE ---

|

| Do: clear_malf(P1133)

FAO_MON_TMR2 >= FAOS_MON_TM -------|

|

FAILED_BANK <> 2 ------------------|AND -| malfunction(ego,P1154)

|     | EGO_MON21 := 1

R_BIAS_AVG2 > RBIAS_EGO_MX --------|     |

;rich shift on ego21                    |

| --- ELSE ---

|

| Do: clear_malf(P1154)

FAO_MON_TMR2 >= FAOS_MON_TM -------|

|

FAILED_BANK <> 2 ------------------|AND -| malfunction(ego,P1153)

|     | EGO_MON21 := 1

R_BIAS_AVG2 < RBIAS_EGO_MN --------|     |

;lean shift on ego21                    |

| --- ELSE ---

|

| Do: clear_malf(P1153)

END: check_upstream_static_shift

11-39

EGO MONITOR CONTROL STRATEGY, DOWNSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

11.1.3  DOWNSTREAM EGO MONITOR (CDAM0)

OVERVIEW

+----------------------------------+

| EGO_DS_MONITOR                   |

+----------------------------------+

|PUBLIC PROCEDURES:                |

|. detect_ds_ego_malf              |

|. downstream_ego_monitor          |

|. downstream_initialization       |

|. downstream_overvoltage_check    |

|. forced_downstream_excursion     |

|. determine_flg_ego_idle          |

|. abort_ds_ego_monitor            |

|. stop_excursion_check            |

+----------------------------------+

This module is designed to monitor the downstream EGO sensors.  It will  work

for  single  bank,  dual  bank, or Y-Pipe configurations.  The downstream EGO

sensor voltage envelope is monitored.  The envelope is compared to threshold.

If the voltage envelope is larger than the threshold envelope, the downstream

EGO sensors are assumed to be functional.

This module contains a process to gather the peak rich and lean voltages seen

on each of the downstream EGO sensors (downstream_ego_monitor).

Another process (forced_downstream_excursion) is excersied  when  faulty  EGO

sensors are present or when a very green catalyst is installed.  This process

will force an A/F excursion in the direction(s) in which the EGO  signal  has

not  met  some  minimum value for a calibratable amount of time (or until the

required minimum voltage has been seen).

There is also a process to determine if one of the sensors is  malfunctioning

and signal the malfunction to the diagnostic executive.

11-40

EGO MONITOR CONTROL STRATEGY, DOWNSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

A finite state machine representation of the  forced_downstream_excursion  is

shown  below.  The transitions cannot be labeled in detail in the diagram and

still have it readable, so only a brief statement of what event  is  occuring

is put on the diagram.

+-------------------+

|  initial_state    |----------------------------+

+-----------------------|                   |------------------------+   |

|                +----->|                   |<-----+                 |   |

|           abort|      +------A----+-------+      |abort            |   |

|start           |        abort|    |start         |            start|   |

|                |             |    |              |                 |   |

+-V----------------++    +-------+----V------+      ++-----------------V+  |

|    rich_bank1     |<---|   rich_bank1/     |----->|    lean_bank2     |  |

|                   |    |   lean_bank2      |      |                   |  |

+-+---------------+-+    +------+----+-------+      +-+---------------+-+  |

|               |     time out|    |completed       |               |    |

|time out       |             |    |                |time out       |    |

|      completed|       +-----V----V--------+       |      completed|    |

|               +------>|part2_initial_state|<------+               |    |

+---------------------->|                   |<----------------------+    |

|                   |<---------------------------+

+-+-A--A----+-A-+-+-+    no excursion necessary

| |  |    | | | |

| |  |    | | | +------------------------------+

+-------------------------+ |  |    | | +----------------------------+   |

|                +----------+  |    | +------------+                 |   |

|           abort|        abort|    |start         |abort            |   |

|start           |             |    |              |            start|   |

+-V----------------++     +------+----V-------+     ++-----------------V+  |

|    lean_bank1     |     |    rich_bank2/    |     |    rich_bank2     |  |

|                   |     |    lean_bank1     |     |                   |  |

+--------+----A-----+     +-+---+------------++     +------A---+--------+  |

time out|    |rich complete|   |time out    |lean complete|   |time out   |

OR    |    +-------------+   |  OR        +-------------+   |  OR       |

complete|                      |complete                      |complete   |

|                +-----V-------------+                |           |

+--------------->|    final_state    |<---------------+           |

|                   |<---------------------------+

+-------------------+  no excursion necessary

11-41

EGO MONITOR CONTROL STRATEGY, DOWNSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  DS_DLY_TMR = Timer used to measure the amount of time  an  A/F  excursion

has  been active to test the downstream EGO sensors.  Timer, Free running

down counter, Units:  SECONDS, Range:  0 - 60, Resolution:  0.25.

-  DS_STATE    =    State    variable    to    be    used     during     the

forced_downstream_excursion EGO monitor test.

-  EXT_CMD = Signed inferred catalyst midbed gas temperatures, degrees.

-  FUEL_SOURCE1 = Value to indicate the source of the final open loop LAMBSE

caluculation for bank 1.

-  FUEL_SOURCE2 = Value to indicate the source of the final open loop LAMBSE

caluculation for bank 2.

-  OV_TMR_12 = Ego12 overvoltage timer.

-  OV_TMR_22 = Ego22 overvoltage timer.

-  VEGO_PK_RH12 = Instantaneous peak rich EGO voltage, volts.

-  VEGO_PK_RH22 = Instantaneous peak rich EGO voltage, volts.

-  VEGO_RH_FF12 =  Fault  filter  for  downstream  bank  1  EGO  sensor  for

overvoltage  fault  detection.   Register,  Units:   NONE, Range:  0 - 1,

Resolution:  0.001.

-  VEGO_RH_FF22 =  Fault  filter  for  downstream  bank  2  EGO  sensor  for

overvoltage  fault  detection.   Register,  Units:   NONE, Range:  0 - 1,

Resolution:  0.001.

Bit Flags:

-  DS_EGO_LEAN1 = Flag used by the downstream EGO monitor to trigger a  lean

A/F excursion for bank 1; 1 -> excursion is in progress.

-  DS_EGO_LEAN2 = Flag used by the downstream EGO monitor to trigger a  lean

A/F excursion for bank 2; 1 -> excursion is in progress.

-  DS_EGO_OL = Flag used to force open loop fuel control during a forced A/F

excursion to test the downstream EGO sensors.

-  DS_EGO_RICH1 = Flag used by the downstream EGO monitor to trigger a  rich

A/F  excursion  for  bank 1; 1 -> excursion is in progress.  excursion to

test the downstream EGO sensors.

-  DS_EGO_RICH2 = Flag used by the downstream EGO monitor to trigger a  rich

A/F excursion for bank 2; 1 -> excursion is in progress.

-  DS_LEAN1 = Flag used by the downstream EGO monitor used to signal when  a

lean  A/F excursion is required for bank 1; 1 -> excursion is complete or

not required.

11-42

EGO MONITOR CONTROL STRATEGY, DOWNSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  DS_LEAN2 = Flag used by the downstream EGO monitor used to signal when  a

lean  A/F excursion is required for bank 2; 1 -> excursion is complete or

not required.

-  DS_RICH1 = Flag used by the downstream EGO monitor used to signal when  a

rich  A/F excursion is required for bank 1; 1 -> excursion is complete or

not required.

-  DS_RICH2 = Flag used by the downstream EGO monitor used to signal when  a

rich  A/F excursion is required for bank 2; 1 -> excursion is complete or

not required.

-  FLG_EGO_IDLE = Flag used to prevent A/F excursion to test the  downstream

ego sensors during idle.

-  OV_DOWN_12 = Flag, when set triggers OV_TMR_12 to decrement.

-  OV_DOWN_22 = Flag, when set triggers OV_TMR_22 to decrement.

-  OV_UP_12 = Flag, when set triggers OV_TMR_12 to increment.

-  OV_UP_22 = Flag, when set triggers OV_TMR_22 to increment.

-  OV_FMEM_n2 = Flag which is set when  the  overvoltage  timer  triggers  a

fault  condition.   This  flag  gets  cleared  when the overvoltage timer

starts counting down after fault condition clears.

-  STOP_EXCURS = Flag used to signal when a forced downstream ego monitoring

excursion is not allowed.

-  UNDSP = Run/Underspeed Flag; 1 -> Underspeed (or CRANK), 0 -> Run.

Calibration Constants:

-  CAT_MX_LEAN = Maximum catalyst midbed temperature to allow the downstream

monitor, degrees.

-  DS_DELAY = Maximum time which an A/F ratio excursion  can  be  active  in

order  to  test the downstream EGO sensors.  Calibration Constant, Units:

SECONDS, Range:  0 - 60, Resolution:  0.25.  Base value:  0.

-  HEGO_CONFIG = HEGO configuration register.

-  LEAN_THRESH = Downstream EGO voltage threshold to signal  a  healthy  EGO

sensor.   Calibration Constant, Units:  VOLTS, Range:  0 - 1, Resolution:

0.001.  Base value:  0.

-  MAX_EGO_VOLT  =  EGO  voltage  at  which  an  overvoltage  condition   is

signalled.    Calibration   Constant,  Units:   VOLTS,  Range:   0  -  2,

Resolution:  0.001.  Base value:  2.

-  N_IDLE_CH = Engine RPM  above  which  downstream  EGO  A/F  excursion  is

allowed.

-  N_IDLE_SL  =  Engine  RPM  below  which  downstream  EGO  monitoring  A/F

excursion is disabled.

11-43

EGO MONITOR CONTROL STRATEGY, DOWNSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  OV_CLEAR_TM = Reset overvoltage timer to this value after fault condition

clears, seconds.

-  OV_TIME = Time at overvoltage on the EGO sensor to signal a malfunction.

-  RICH_THRESH = Downstream EGO voltage threshold to signal  a  healthy  EGO

sensor.   Calibration Constant, Units:  VOLTS, Range:  0 - 1, Resolution:

0.001.  Base value:  0.

-  VEGO_FK = EGO sensor  overvoltage  fault  filter  constant.   Calibration

Constant,  Units:   NONE, Range:  0 - 1, Resolution:  0.001.  Base value:

1.

OTHER

-  downstream_ego_codes = set of (P0138, P0136, P0158, P0156).  The  set  of

OBD-II fault codes that relate to the downstream ego monitor.

-  malfunction(ego, Pxxx) = Public procedure updating the MIL control status

provided by the MIL control module.

-  P0138 = Oxygen sensor circuit high voltage (bank 1 downstream).

-  P0136 = Oxygen sensor circuit malfunction (bank 1 downstream).

-  P0156 = Oxygen sensor circuit malfunction (bank 2 downstream).

-  P0158 = Oxygen sensor circuit high voltage (bank 2 downstream).

11-44

EGO MONITOR CONTROL STRATEGY, DOWNSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EGO_DS_MONITOR_COM10

BEGIN: definitions

;the following logic chart is not to be specifically executed.  It

;merely exists to make documentation more readable.

unconditionally --------------------------| initial_state := 0

| rich_bank1 := 1

| rich_bank1_lean_bank2 := 2

| lean_bank2 := 3

| part2_initial_state := 4

| lean_bank1 := 5

| rich_bank2_lean_bank1 := 6

| rich_bank2 := 7

| final_state := 8

| no_aft_hego := 0

| single_bank := 1

| dual_bank := 2

| y_pipe := 3

END: definitions

BEGIN: downstream_initialization

;executed on powerup.

unconditionally --------------------------| VEGO_PK_RH12 := 0

| VEGO_PK_RH22 := 0

| VEGO_PK_LN12 := 1

| VEGO_PK_LN22 := 1

END: downstream_initialization

11-45

EGO MONITOR CONTROL STRATEGY, DOWNSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: reset_overvoltage_ds

;The following process is called from the EGO montior control process

;and is used to freeze the overvoltage timers when the test is not

;is not currently enabled.  This will prevent the timers from being

;started while enabled and then become dissabled, but keep running.

unconditionally -------------------------| OV_UP_12 := 0

| OV_UP_22 := 0

| OV_DOWN_12 := 0

| OV_DOWN_22 := 0

|  ;freeze both timers.

END: reset_overvoltage_ds

BEGIN: determine_flg_ego_idle

;This process is called from within the EGO monitor controller

;and is used to control the flag FLG_EGO_IDLE, used by the forced

;downstream excursion.

N < N_IDLE_SL ----------------------|S Q -| FLG_EGO_IDLE

|

N > N_IDLE_CH ----------------------|C

END: determine_flg_ego_idle

11-46

EGO MONITOR CONTROL STRATEGY, DOWNSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: downstream_overvoltage_check

;This process, initiated from the EGO monitor control process is used to

;detect an overvoltage condition on the EGO sensor.

HEGO_CONFIG <> no_aft_hego --------|

|

P0141MALF = 0 ---------------------|AND -| OV_UP_12 := 1

|     | OV_DOWN_12 := 0

VEGO12 > MAX_EGO_VOLT -------------|     | ;count up OV_TMR_12

;Check for overvoltage when heater     |

;is functional only.                   |

| --- ELSE ---

|

| OV_UP_12 := 0

| OV_DOWN_12 := 1

| ;count down OV_TMR_12

HEGO_CONFIG = dual_bank -----------|

|

P0161MALF = 0 ---------------------|AND -| OV_UP_22 := 1

|     | OV_DOWN_22 := 0

VEGO22 > MAX_EGO_VOLT -------------|     | ;count up OV_TMR_22

;Check for overvoltage.                |

| --- ELSE ---

|

| OV_UP_22 := 0

| OV_DOWN_22 := 1

| ;count down OV_TMR_22

OV_DOWN_12 = 1 --------------------|

|AND -| OV_TMR_12 := OV_CLEAR_TM

OV_FMEM_12 = 1 --------------------|     |  ;begin OV timer countdown

| OV_FMEM_12 := 0

|  ;clear OV countdown flag

OV_TMR_12 > OV_TIME ---------------------| Do: malfunction(ego, P0138)

|  ;high voltage on ego12.

| DS_RICH1 := 1

| DS_LEAN1 := 1

|  ;lock out D.S. functional

|  ;check on this bank.

| OV_FMEM_12 := 1

|  ;arm OV countdown flag

|

| --- ELSE ---

|

OV_TMR_12 = 0 ---------------------------| Do: clear_malf(P0138)

OV_DOWN_22 = 1 --------------------|

|AND -| OV_TMR_22 := OV_CLEAR_TM

OV_FMEM_22 = 1 --------------------|     |  ;begin OV timer countdown

| OV_FMEM_22 := 0

|  ;clear OV countdown flag

11-47

EGO MONITOR CONTROL STRATEGY, DOWNSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OV_TMR_22 > OV_TIME ---------------------| Do: malfunction(ego, P0158)

|  ;high voltage on ego22.

| DS_RICH2 := 1

| DS_LEAN2 := 1

|  ;lock out D.S. functional

|  ;check on this bank.

| OV_FMEM_22 := 1

|  ;arm OV countdown flag

|

| --- ELSE ---

|

OV_TMR_22 = 0 ---------------------------| Do: clear_malf(P0158)

END: downstream_overvoltage_check

11-48

EGO MONITOR CONTROL STRATEGY, DOWNSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: downstream_ego_monitor

;This procedure is called from within the EGO  monitor controller and

;ratchets on the peak rich and lean voltages seen while monitoring the

;downstream EGO signals.

HEGO_CONFIG <> no_aft_hego ---------|

|

VEGO12 > VEGO_PK_RH12 --------------|AND -| VEGO_PK_RH12 := VEGO12

|

UNDSP = 0 --------------------------|

;new rich peak voltage observed.

HEGO_CONFIG = dual_bank ------------|

|

VEGO22 > VEGO_PK_RH22 --------------|AND -| VEGO_PK_RH22 := VEGO22

|

UNDSP = 0 --------------------------|

;new rich peak voltage observed.

HEGO_CONFIG <> no_aft_hego ---------|

|

VEGO12 < VEGO_PK_LN12 --------------|AND -| VEGO_PK_LN12 := VEGO12

|

UNDSP = 0 --------------------------|

;new lean peak voltage observed.

HEGO_CONFIG = dual_bank ------------|

|

VEGO22 < VEGO_PK_LN22 --------------|AND -| VEGO_PK_LN22 := VEGO22

|

UNDSP = 0 --------------------------|

;new lean peak voltage observed.

HEGO_CONFIG = no_aft_hego ----------|

|OR --| DS_LEAN1 := 1

VEGO_PK_LN12 <= LEAN_THRESH --------|

HEGO_CONFIG = no_aft_hego ----------|

|OR --| DS_RICH1 := 1

VEGO_PK_RH12 >= RICH_THRESH --------|

HEGO_CONFIG <> dual_bank -----------|

|OR --| DS_LEAN2 := 1

VEGO_PK_LN22 <= LEAN_THRESH --------|

HEGO_CONFIG <> dual_bank -----------|

|OR --| DS_RICH2 := 1

VEGO_PK_RH22 >= RICH_THRESH --------|

END: downstream_ego_monitor

11-49

EGO MONITOR CONTROL STRATEGY, DOWNSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: stop_excursion_check

;The following logic is used to determine if the forced downstream excursion

;should be stopped based on vehicle operational state.  It is called from

;within the ego_monitor_control process.

APT = 1 --------------------------------------|

|

FLG_EGO_IDLE = 1 -----------------------------|

|

FUEL_SOURCE1 = 1 -----------------------------|

|OR --| STOP_EXCURS := 1

FUEL_SOURCE2 = 1 -----------------------------|     |  ;downstream ego

|     |  ;monitor, not able

EXT_CMD > CAT_MX_LEAN ------------------|     |     |  ;to run at WOT,

|     |     |  ;idle, cat overtemp

DS_LEAN1 = 0 ---------------------|     |AND -|     |

|OR --|           |

DS_LEAN2 = 0 ---------------------|                 |

| --- ELSE ---

|

| STOP_EXCURS := 0

END: stop_excursion_check

11-50

EGO MONITOR CONTROL STRATEGY, DOWNSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: forced_downstream_excursion

;This process is used to determine if the downstream sensors have been

;active, i.e., a large variation in voltage has been seen on them.  If

;this is not the case the air fuel ratio is forced rich and/or lean

;in an open loop manner to some predetermined point to attempt to force

;the downstream sensor to switch.  The air fuel ratio is held some

;calibratiable number of seconds at this open loop value.

;This process is called from within the EGO monitor controller.

HEGO_CONFIG <> no_aft_hego -------|

|

DS_STATE = initial_state ---------|

|

DS_LEAN2 = 1 ---------------------|AND -| DS_STATE := rich_bank1

|     | DS_EGO_OL := 1

DS_RICH1 = 0 ---------------------|     | DS_EGO_RICH1 := 1

|     | DS_DLY_TMR := DS_DELAY

STOP_EXCURS = 0 ------------------|     |

| --- ELSE ---

HEGO_CONFIG = dual_bank ----------|     |

|     |

DS_STATE = initial_state ---------|     |

|     |

DS_LEAN2 = 0 ---------------------|AND -| DS_STATE := rich_bank1_lean_bank2

|     | DS_EGO_OL := 1

DS_RICH1 = 0 ---------------------|     | DS_EGO_RICH1 := 1

|     | DS_EGO_LEAN2 := 1

STOP_EXCURS = 0 ------------------|     | DS_DLY_TMR := DS_DELAY

|

| --- ELSE ---

HEGO_CONFIG = dual_bank ----------|     |

|     |

DS_STATE = initial_state ---------|     |

|     |

DS_LEAN2 = 0 ---------------------|AND -| DS_STATE := lean_bank2

|     | DS_EGO_OL := 1

DS_RICH1 = 1 ---------------------|     | DS_EGO_LEAN2 := 1

|     | DS_DLY_TMR := DS_DELAY

STOP_EXCURS = 0 ------------------|     |

| --- ELSE ---

DS_STATE = initial_state ---------|     |

|     |

DS_LEAN2 = 1 ---------------------|AND -| DS_STATE = part2_initial_state

|

DS_RICH1 = 1 ---------------------|

11-51

EGO MONITOR CONTROL STRATEGY, DOWNSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DS_STATE = rich_bank1 ------------|

|AND -| DS_STATE := part2_initial_state

DS_DLY_TMR = 0 -------------------|     | DS_RICH1 := 1

| DS_LEAN1 := 1

| DS_EGO_OL := 0

| DS_EGO_RICH1 := 0

|

| --- ELSE ---

DS_STATE = rich_bank1 ------------|     |

|AND -| DS_STATE := part2_initial_state

DS_RICH1 = 1 ---------------------|     | DS_EGO_OL := 0

| DS_EGO_RICH1 := 0

| DS_DLY_TMR := 0

|

| --- ELSE ---

DS_STATE = rich_bank1 ------------|     |

|AND -| DS_STATE := initial_state

STOP_EXCURS = 1 ------------------|     | DS_EGO_OL := 0

| DS_EGO_RICH1 := 0

| DS_DLY_TMR := 0

DS_STATE = lean_bank2 ------------|

|AND -| DS_STATE := part2_initial_state

DS_DLY_TMR = 0 -------------------|     | DS_RICH2 := 1

| DS_LEAN2 := 1

| DS_EGO_OL := 0

| DS_EGO_LEAN2 := 0

|

| --- ELSE ---

DS_STATE = lean_bank2 ------------|     |

|AND -| DS_STATE := part2_initial_state

DS_LEAN2 = 1 ---------------------|     | DS_EGO_OL := 0

| DS_EGO_LEAN2 := 0

| DS_DLY_TMR := 0

|

| --- ELSE ---

DS_STATE = lean_bank2 ------------|     |

|AND -| DS_STATE := initial_state

STOP_EXCURS = 1 ------------------|     | DS_EGO_OL := 0

| DS_EGO_LEAN2 := 0

| DS_DLY_TMR := 0

11-52

EGO MONITOR CONTROL STRATEGY, DOWNSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DS_STATE = rich_bank1_lean_bank2 -|

|AND -| DS_STATE := part2_initial_state

DS_DLY_TMR = 0 -------------------|     | DS_RICH1 := 1

| DS_LEAN1 := 1

| DS_RICH2 := 1

| DS_LEAN2 := 1

| DS_EGO_OL := 0

| DS_EGO_RICH1 := 0

| DS_EGO_LEAN2 := 0

|

| --- ELSE ---

DS_STATE = rich_bank1_lean_bank2 -|     |

|     |

DS_RICH1 = 1 ---------------------|AND -| DS_STATE := part2_initial_state

|     | DS_EGO_OL := 0

DS_LEAN2 = 1 ---------------------|     | DS_EGO_RICH1 := 0

| DS_EGO_LEAN2 := 0

| DS_DLY_TMR := 0

|

| --- ELSE ---

DS_STATE = rich_bank1_lean_bank2 -|     |

|AND -| DS_STATE := rich_bank1

DS_LEAN2 = 1 ---------------------|     | DS_EGO_LEAN2 := 0

|

| --- ELSE ---

DS_STATE = rich_bank1_lean_bank2 -|     |

|AND -| DS_STATE := lean_bank2

DS_RICH1 = 1 ---------------------|     | DS_EGO_RICH1 := 0

|

| --- ELSE ---

DS_STATE = rich_bank1_lean_bank2 -|     |

|AND -| DS_STATE := initial_state

STOP_EXCURS = 1 ------------------|     | DS_EGO_OL := 0

| DS_EGO_LEAN2 := 0

| DS_EGO_RICH1 := 0

| DS_DLY_TMR := 0

11-53

EGO MONITOR CONTROL STRATEGY, DOWNSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

HEGO_CONFIG = dual_bank ----------|

|

DS_STATE = part2_initial_state ---|

|

DS_LEAN1 = 1 ---------------------|AND -| DS_STATE := rich_bank2

|     | DS_EGO_OL := 1

DS_RICH2 = 0 ---------------------|     | DS_EGO_RICH2 := 1

|     | DS_DLY_TMR := DS_DELAY

STOP_EXCURS = 0 ------------------|     |

| --- ELSE ---

HEGO_CONFIG = dual_bank ----------|     |

|     |

DS_STATE = part2_initial_state ---|     |

|     |

DS_LEAN1 = 0 ---------------------|AND -| DS_STATE := rich_bank2_lean_bank1

|     | DS_EGO_OL := 1

DS_RICH2 = 0 ---------------------|     | DS_EGO_RICH2 := 1

|     | DS_EGO_LEAN1 := 1

STOP_EXCURS = 0 ------------------|     | DS_DLY_TMR := DS_DELAY

|

| --- ELSE ---

HEGO_CONFIG <> no_aft_hego -------|     |

|     |

DS_STATE = part2_initial_state ---|     |

|     |

DS_LEAN1 = 0 ---------------------|AND -| DS_STATE := lean_bank1

|     | DS_EGO_OL := 1

DS_RICH2 = 1 ---------------------|     | DS_EGO_LEAN1 := 1

|     | DS_DLY_TMR := DS_DELAY

STOP_EXCURS = 0 ------------------|     |

| --- ELSE ---

DS_STATE = part2_initial_state ---|     |

|     |

DS_LEAN1 = 1 ---------------------|AND -| DS_STATE = final_state

|

DS_RICH2 = 1 ---------------------|

DS_STATE = rich_bank2 ------------|

|AND -| DS_STATE := final_state

DS_DLY_TMR = 0 -------------|     |     | DS_RICH2 := 1

|OR --|     | DS_EGO_OL := 0

DS_RICH2 = 1 ---------------|           | DS_EGO_RICH2 := 0

| DS_DLY_TMR := 0

|

| --- ELSE ---

DS_STATE = rich_bank2 ------------|     |

|AND -| DS_STATE := part2_initial_state

STOP_EXCURS = 1 ------------------|     | DS_EGO_OL := 0

| DS_EGO_RICH2 := 0

| DS_DLY_TMR := 0

11-54

EGO MONITOR CONTROL STRATEGY, DOWNSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DS_STATE = lean_bank1 ------------|

|AND -| DS_STATE := final_state

DS_DLY_TMR = 0 -------------|     |     | DS_LEAN1 := 1

|OR --|     | DS_EGO_OL := 0

DS_LEAN1 = 1 ---------------|           | DS_EGO_LEAN1 := 0

| DS_DLY_TMR := 0

|

| --- ELSE ---

DS_STATE = lean_bank1 ------------|     |

|AND -| DS_STATE := part2_initial_state

STOP_EXCURS = 1 ------------------|     | DS_EGO_OL := 0

| DS_EGO_LEAN1 := 0

| DS_DLY_TMR := 0

DS_STATE = rich_bank2_lean_bank1 -|

|

DS_DLY_TMR = 0 -------------------|AND -| DS_STATE := final_state

|     | DS_RICH2 := 1

DS_RICH1 = 1 ---------------|     |     | DS_LEAN1 := 1

|OR --|     | DS_EGO_OL := 0

DS_LEAN2 = 1 ---------------|           | DS_EGO_RICH2 := 0

| DS_EGO_LEAN1 := 0

| DS_DLY_TMR := 0

|

| --- ELSE ---

DS_STATE = rich_bank2_lean_bank1 -|     |

|AND -| DS_STATE := rich_bank2

DS_LEAN1 = 1 ---------------------|     | DS_EGO_LEAN1 := 0

|

| --- ELSE ---

DS_STATE = rich_bank2_lean_bank1 -|     |

|AND -| DS_STATE := lean_bank1

DS_RICH2 = 1 ---------------------|     | DS_EGO_RICH2 := 0

|

| --- ELSE ---

DS_STATE = rich_bank2_lean_bank1 -|     |

|AND -| DS_STATE := part2_initial_state

STOP_EXCURS = 1 ------------------|     | DS_EGO_OL := 0

| DS_EGO_LEAN1 := 0

| DS_EGO_RICH2 := 0

| DS_DLY_TMR := 0

HEGO_CONFIG = y_pipe -------------------| DS_EGO_RICH2 := DS_EGO_RICH1

| DS_EGO_LEAN2 := DS_EGO_LEAN1

|  ;if y-pipe application, ensure

|  ;both banks go on open loop

|  ;excursion together.

END: force_downstream_excursion

11-55

EGO MONITOR CONTROL STRATEGY, DOWNSTREAM EGO MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: detect_ds_ego_malf

;This process is called from within the EGO monitor controller and is

;used to determine if the downstream sensors are malfunctioning.

HEGO_CONFIG <> no_aft_hego ---------|

|AND -| Do: malfunction(ego, P0136)

VEGO_PK_LN12 > LEAN_THRESH ---|     |     |

|OR --|     |

VEGO_PK_RH12 < RICH_THRESH ---|           |

| --- ELSE ---

|

| Do: clear_malf(P0136)

HEGO_CONFIG = dual_bank ------------|

|AND -| Do: malfunction(ego, P0156)

VEGO_PK_LN22 > LEAN_THRESH ---|     |     |

|OR --|     |

VEGO_PK_RH22 < RICH_THRESH ---|           |

| --- ELSE ---

|

| Do: clear_malf(P0156)

END: detect_ds_ego_malf

BEGIN: abort_ds_ego_monitor

;this process will perform all reset actions necessary to put the

;downstream EGO monitor back in state prior to it being executed.

unconditionally --------------------------| Do: downstream_initialization

| DS_STATE := initial_state

| DS_EGO_OL := 0

| DS_EGO_RICH1 := 0

| DS_EGO_RICH2 := 0

| DS_EGO_LEAN1 := 0

| DS_EGO_LEAN2 := 0

| DS_RICH1 := 0

| DS_RICH2 := 0

| DS_LEAN1 := 0

| DS_LEAN2 := 0

| DS_DLY_TMR := 0

END: abort_ds_ego_monitor

11-56

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

11.1.4  EGO HEATER CONTROL (CDAN0)

This file contains the following processes used to perform EGO heater control

and monitoring.

+-------------------------------+

| EGO_HEATER_CONTROL            |

+-------------------------------+

|PUBLIC PROCEDURES:             |

|. ego_heater_control_init      |

|. ego_heater_control           |

|. reset_ego_heater             |

+-------------------------------+

|PRIVATE PROCEDURES:            |

|. check_us_faults              |

|. check_ds_faults              |

|. current_monitor_state_control|

|. current_fault_check          |

|. ds_fault_filtering           |

|. ego_heater_current_monitor   |

|. ego_heater_diag_state_control|

|. ego_heater_fault_report      |

|. ego_heater_mode_select       |

|. ego_heater_mon_control       |

|. ego_heater_state_control     |

|. ego_heater_timer_control     |

|. koeo_test                    |

|. reset_diag_state             |

|. us_fault_filtering           |

|. update_retry_max             |

+-------------------------------+

11-57

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The following calling structure exists within this module:

PROCESS NAME                                   LOCATION

-----------------------------------------      ---------------------------

background->ego_heater_control                 ego_heater_control_com

ego_heater_fault_status                   ego_heater_io_com

ego_heater_state_control                  ego_heater_control_com

ego_heater_control_init           ego_heater_control_com

ego_heater_io_init                ego_heater_io_com

koeo_test                         ego_heater_control_com

malfunction                 obdii executive

clear_malf                  obdii executive

ego_heater_mode_select                    ego_heater_control_com

ego_heater_diag_state_control             ego_heater_control_com

clear_malf                        obdii executive

reset_diag_state                  ego_heater_control_com

check_us_faults                   ego_heater_control_com

check_ds_faults                   ego_heater_control_com

us_fault_filtering                ego_heater_control_com

update_retry_max            ego_heater_control_com

clear_malf                  obdii executive

ds_fault_filtering                ego_heater_control_com

update_retry_max            ego_heater_control_com

clear_malf                  obdii executive

ego_heater_fault_report           ego_heater_control_com

malfunction                 obdii executive

ego_heater_timer_control                  ego_heater_control_com

ego_heater_current_monitor                ego_heater_control_com

current_monitor_state_control     ego_heater_control_com

current_fault_check         ego_heater_control_com

ego_heater_mon_control                    ego_heater_control_com

ego_heater_output                         ego_heater_io_com

substitute                        obdii executive

osc_response                      scp control

KAM init->reset_ego_heater                     ego_heater_control_com

RAM init->ego_heater_control_init              ego_heater_control_com

11-58

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

This module is build around state machine logic which encodes  the  parameter

HEATER_STATE  with the following information:  heater operation mode (normal,

voltage diagnositics, and current monitoring), upstream pair  on/off  status,

and  downstream  pair  on/off status.  The diagram below shows an overview of

HEGO heater control.

+-----------+               +-----------+

---------->|           |-------------->|           |

EXT_FEU,   |   State   | HEATER_STATE  |   I/O     |<----> IMP IC,

EXT_REG    |  Machines |               | Interface |       heaters

|           |<--------------|           |

(temp      +-----------+   HTR_FAULTxy +-----------+

controls)   (ego_heater_                (ego_heater_     (Hardware)

control_com)                  io_com)

Inferred tip temperatures at  the  front  and  rear  HEGO  sensors  (EXT_FEU,

EXT_REG)  control  state  machine  operation.   In normal operatoin, when tip

temperature increases above a low threshold, heaters are commanded on.   This

low  temperature  threshold protects the heater ceramic element from cracking

due to large thermal gradients when cold.  The heaters will be commanded  off

if  the  tip  temperature  rises above an upper threshold.  This protects the

heater element from damage due to overtemperature.

Fault flags are returned by the heater interface module if voltage faults are

detected.   Once per trip, heaters will be turned on and off, and these fault

flags will be checked to report on heater status.  If a fault  flag  is  set,

the  faulty  heater will be commanded on/off in an attempt to clear the fault

condition.  If the fault does not clear after repeated retries, a malfunction

code  will  be  set.   Also, if a fault flag is set at any time during normal

heater operation, the state machine will be  placed  back  into  diagnositics

mode and the retry process above will be repeated.

Once per trip, after all heaters have been commanded on and are warmed up,  a

current  measurement  will be performed.  The OBD-II regulations state:  "For

heated oxygen sensors, the heater circuit shall be considered  malfunctioning

when  the  current  or  voltage  drop  in the circuit is no longer within the

manufacturer's specified limits for proper operation."

See the circuit diagrams below for a description of the  state  machines  for

heater and current monitoring operation.

11-59

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The upstream EGO heaters are electrically configured as shown in the  diagram

below.   The  downstream  pair is similarily configured.  When an application

does not have either HEGO21, or HEGO22, the same control scheme is used.  The

heater is missing from the diagram, but the control FETs must be turned on in

pairs to prevent burning out the sense resistor.

PWR

o

|

+-------+-------+

|               |

|               |

+-+             +-+

| |             | |

|    | |HTR11        | |HTR21 |

|    | |             | |      |

heater11|    +-+             +-+      |heater21

current |     |  +---------+  |       |current

|     +--|         |--+       |

|     |  +---------+  |       |

|     |      R        |       |

V     |       sense   |       V

|---|               |---|

HEGOHTR11 o---|                       |---o HEGOHTR21

(normally ON) |---|               |---|     (normally ON)

|               |

|               |

--+--           --+--

---             ---

-               -

In order to measure the current through the heater, the following  steps  are

taken.

1.  The HEGOHTRxy output associated with the heater that the current will  be

measured in is switched off.

2.  The HEGO heater A/D input is selected via the AMUX by taking ACHMUX1 &  2

high.

3.  The A/D channel is read and the result stored into HTRCMxy for the heater

being read.

4.  The HEGOHTRxy output is switched back on.

11-60

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

When the HEGOHTRxy output is switched off, all the current  from  the  heater

being  measured  is  directed  through  the  sense  resistor  and through the

opposite FET to ground.  The sense resistor is 1 ohm, so the volotage read by

the A/D is the same, numerically, as the current through the heater.  See the

diagram below showing current monitoring configuration for heater 21.

PWR

o

|

+-------+-------+

|               |

|          |    |

+-+ heater21|   +-+

|      | |  current|   | |

| HTR11| |         |   | |HTR21

|      | |         |   | |

heater11|      +-+   <-----+   +-+

current |       |  +---------+  |

|       B--|         |--A     V   = (heater21 current)*(1 ohm)

|       |  +---------+  |      AB

|       |      R        |

V       |       sense   |

|---|               |---|

HEGOHTR11 o---|                       |---o HEGOHTR21

(ON)        |---|               |---|     (OFF)

|               |

|               |

--+--           --+--

---             ---

-               -

Prior to commencement of current measurements, all EGO sensor heaters must be

on  for  enough time to allow the heater current to stabalize.  Additionally,

all heaters must be  operating  normally,  i.e.,  no  faults  detected.   The

strategy  enforces  this  by  testing  the  HTR_FAULTxy bits prior to current

monitoring.  These bits are set in the heater I/O module (ego_heater_io)  and

indicate heater hardware fault status.

11-61

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EGO HEATER CONTROLS

The following diagrams describe, using finite state  automata,  operation  of

the EGO heaters during non-current monitoring modes.  The decision to turn on

pairs of EGO sensor heaters is  based  on  temperature  in  the  exhaust  gas

stream.

+---------+

|         |

|  ds_on  |

|         |

+-+--A----+

not in run mode OR     |  |in run mode &

EXT_REG >= DS_HTR_OFF  |  |EXT_FEU >= US_HTR_OFF &

---------------------- |  |EXT_REG >= DS_HTR_ON &

all heaters off        |  |EXT_REG < DS_HTR_OFF

|  |-----------------------

+---------+  |  |upstream heaters off

|         |<-+  |downstream heaters on

|         |-----+

--------------------------------->| all_off |

power-up OR OSM_EO_OFF=1          |         |-----+

---------------                   |         |<-+  |in run mode &

all heaters off                   +---------+  |  |EXT_FEU > US_HTR_ON &

|  |EXT_FEU < US_HTR_OFF

|  |-----------------------

not in run mode OR      |  |upstream heaters on

EXT_FEU >= US_HTR_OFF   |  |downstream heaters off

------------------------|  |

all heaters off         |  |

+-+--V----+

|         |

|  us_on  |

|         |

+---------+

11-62

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EGO HEATER CONTROLS, CONTINUED.

+---------+

|         |

|  us_on  |

|         |

+--+--A---+

DS_HTR_ON <= EXT_REG  &|  |

DS_HTR_OFF > EXT_REG   |  |EXT_REG >= DS_HTR_OFF

-----------------------|  |----------------------

upstream heaters on    |  |upstream heaters on

downstream heaters on  |  |downstream heaters off

|  |         +---------+

|  +---------|         |

+----------->|         |

| all_on  |

+----------->|         |

|  +---------|         |

|  |         +---------+

EXT_FLANGE < US_HTR_OFF|  |EXT_FEU >= US_HTR_OFF

-----------------------|  |------------------------

upstream heaters on    |  |upstream heaters off

downstream heaters on  |  |downstream heaters on

+--+--V---+

|         |

|  ds_on  |

|         |

+---------+

OBDII_RESET=1  or

ER_STATUS=ER_DONE or

ER_STATUS=ER_INIT

---------------|

HTR_RETRYxy=0  |

all heaters off|

|

+------V--+                                   +---------+

|         |                                   |         |

| all_off |                                   | all_on  |

|         |<----------------------------------|         |

+---A-----+            not in run mode        +----A----+

|                  ---------------             |

|OSM_EO_OFF=1      all heaters off             |OSM_EO_ON=1

|---------------                               |--------------

|all heaters off                               |all heaters on

11-63

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EGO HEATER CURRENT MONITORING

The following finite state automata is used to conduct current monitoring  on

all the EGO sensor heaters present in the given application.

+---------+

+---------------------------|         |

|CURRENT_CKD=1              |  cm_22  |

|HTR_FAULTxy = 0         +--|         |

|----------------        |  +-A--A----+

|heater 22 on            |    |  |

|EGO_HTR_MON=1           +----+  |CURRENT_CKD=1

|CURRENT_CKD=0            "a"    |HTR_FAULTxy = 0

|                                |HEATER_HP22 = 1

|         CURRENT_CKD=1          |---------------

|         HEATER_HP22 = 0        |heater 12 on

|         HTR_FAULTxy=0          |heater 22 off

|         ---------------        |CURRENT_CKD=0

|         EGO_HTR_MON=1          |

|         all heaters on    +----+----+

|         CURRENT_CKD=0     |         |

|  +------------------------|  cm_12  |

|  |                     +--|         |<------------+

|  |                     |  +-A--A----+             |

|  |                     |    |  |CURRENT_CKD=1     |

+----V--V-+                   +----+  |HTR_FAULTxy = 0   |

|         |                    "a"    |HEATER_HP12 = 1   |

| all_on  |                           |---------------   |

|         |                           |heater 21 on      |

+----+--A-+                           |heater 12 off     |

|  |                             |CURRENT_CKD=0     |

|  +--------------          +----+----+             |CURRENT_CKD=1

|   from any state          |         |             |HEATER_HP21 = 0

|   HTR_FAULTxy = 1         |  cm_21  |             |---------------

|   or CURR_TST_ENA=0    +--|         |             |heater 11 on

|   or not in run mode   |  +-A--A----+             |heater 21 off

|   ------------------   |    |  |CURRENT_CKD=1     |CURRENT_CKD=0

|   all heaters on       +----+  |HTR_FAULTxy = 0   |

|   CURRENT_CKD=1         "a"    |HEATER_HP21 = 1   |

|                                |---------------   |

|CURR_TST_ENA = 1                |heater 11 on      |

|HTR_FAULTxy = 0                 |heater 21 off     |

|----------------                |CURRENT_CKD=0     |

|heater 11 off              +----+----+             |

|CURRENT_CKD=0              |         |-------------+

+-------------------------->|  cm_11  |

+--|         |

|  +-A-------+

|    |

+----+

"a"

11-64

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Where transition "a" is defined as shown below:

CURRENT_CKD = 1

---------------------------------------------

transfer FN_HTR_XFER(HTRCM_CNTS) into HTRCMxy

A seperate process reads the HTRCM ATOD channel and sets CURRENT_CKD.

The following diagram describes what happens when a fault is detected on  one

or  more of the HEGO heaters.  A special diagnostic sequence is entered where

the sensor is toggled off and then on.  The fault status is checked  in  both

states  and  if a fault was detected, the retry counter is incremented.  This

repeats until the retry counter reaches RETRY_ALLOW or no fault is detected.

HTR_MALF=0  &

HTR_FAULTxy=0

-------------

HTR_MONxy=1

HTR_RETRYxy=0

+-----------+

|           |

+--+-----------V-+

|                |

HTR_MALFxy=1 or+--->| xxx_on_diag_on |---+

HTR_FAULTxy=1  |    +-+------A-------+   |

---------------|      |      |           |

inc HTR_RETRYxy+------+      |           |

|           |

|           |

+--------+--------+  |

+--->| xxx_on_diag_off |  |

HTR_FAULTxy=1|    +-+------A--------+  |

-------------|      |      |           |

HTR_MALFxy=1 +------+      |           |

|           |

TEST_ENABLED=1 &|           |

[RETRY_ALLOW>   |           |

HTR_RETRYxy>0 or|           |

xy_FAULTED=1  or|           |

NORMAL_MODE=0]  |           |

----------------|           |

HTR_MALFxy=0    |           |

+----+---+       |

|        |<------+

| xxx_on |

|        |

+--------+

NOTE: xxx_on can be any one of us_on, ds_on, or all_on.

11-65

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  DS_HTR_TMR = Length of time the downstream  EGO  heaters  have  been  on,

seconds.

-  ER_STATUS = State pointer that indicated current state of engine  running

on demand test.

-  EXT_FEU = Inferred tip temperature of the upstream  EGO  sensor  (without

heater effects), degrees.

-  EXT_REG = Inferred tip temperature of the downstream ego sensor  (without

heater effects), degrees.

-  HEATER_STATE = State pointer for EGO heater controls.

-  HTRCM11 = Current measured in EGO heater 11.

-  HTRCM12 = Current measured in EGO heater 12.

-  HTRCM21 = Current measured in EGO heater 21.

-  HTRCM22 = Current measured in EGO heater 22.

-  HTRCM_CNTS = ATOD input for heater current measurement, counts.

-  HTR_EXE_TMR = Timer used to pace execution of the heater controller.

-  HTR_RET_MX11 = Highest number of retries seen on EGO heater 11.

-  HTR_RET_MX12 = Highest number of retries seen on EGO heater 12.

-  HTR_RET_MX21 = Highest number of retries seen on EGO heater 21.

-  HTR_RET_MX22 = Highest number of retries seen on EGO heater 22.

-  HTR_RETRY11 = The number of retries executed for EGO heater 11.

-  HTR_RETRY12 = The number of retries executed for EGO heater 12.

-  HTR_RETRY21 = The number of retries executed for EGO heater 21.

-  HTR_RETRY22 = The number of retries executed for EGO heater 22.

-  OSM_EO_OFF = Off state  requested  for  OSM  outputs  during  engine  off

on-demand test.

-  OSM_EO_ON =  On  state  requested  for  OSM  outputs  during  engine  off

on-demand test.

-  US_HTR_TMR = Timer activated after the upstream heaters turn on,  so  the

heaters can warm up prior to allowing the current monitor to run.

-  VBAT = Battery voltage.

11-66

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Bit Flags:

-  CCM_ER_ENA = Engine running ondemand test for CCM enabled.

-  CCM_TST_ENA = OBD-II Comprehensive Component Test enable flag, (1 -> test

enabled).

-  CURRENT_CKD = Semaphore to signal when the HTRCM data is available.

-  CURR_TST_ENA = Flag used to start the EGO heater current measurement.

-  EGO_CUR_MON = Flag indicating  current  monitoring  has  been  completed,

unitless.

-  EGO_TST_ENA = OBD-II test enable flag, (1 -> test enable).

-  EO_INIT_FLG = Catch the rising edge of EO_ACTIVE.

-  EXT_FMM_FLG = FMEM flag set by inferred temperature model due to  ECT/ACT

failure, to indicate temperature data may be inaccurate.

-  HTR_FAULT11 = Fault status of EGO heater 11.

-  HTR_FAULT12 = Fault status of EGO heater 12.

-  HTR_FAULT21 = Fault status of EGO heater 21.

-  HTR_FAULT22 = Fault status of EGO heater 22.

-  HTR_MALF11 = Flag used to latch a heater  malfunction  during  an  off-on

cycle, unitless.

-  HTR_MALF12 = Flag used to latch a heater  malfunction  during  an  off-on

cycle, unitless.

-  HTR_MALF21 = Flag used to latch a heater  malfunction  during  an  off-on

cycle, unitless.

-  HTR_MALF22 = Flag used to latch a heater  malfunction  during  an  off-on

cycle, unitless.

-  HTR_MON11 = Flag indicating when the heater has been monitored, unitless.

-  HTR_MON12 = Flag indicating when the heater has been monitored, unitless.

-  HTR_MON21 = Flag indicating when the heater has been monitored, unitless.

-  HTR_MON22 = Flag indicating when the heater has been monitored, unitless.

-  NORMAL_MODE = Flag defining if the EGO heater strategy is in normal  mode

or intrusive test mode.

-  OBDII_RESET = Flag used to simulate the receipt of an  OBD-II  scan  tool

reset message.

-  OLDHTRFALT11 = Previous b.g.  pass value of HTR_FAULT11.

11-67

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  OLDHTRFALT12 = Previous b.g.  pass value of HTR_FAULT12

-  OLDHTRFALT21 = Previous b.g.  pass value of HTR_FAULT21.

-  OLDHTRFALT22 = Previous b.g.  pass value of HTR_FAULT22.

-  TEST_ENABLED = Flag indicating whether or not the EGO  heater  tests  are

enabled.

-  UNDSP = Underspeed Flag; 1 -> Underspeed or Crank Mode, 0 -> Run mode.

-  US_FAULTED = Flag indicating one of the upstream heaters is faulted.

Calibration Constants:

-  CM_HEAT_TM = Amount of time the downstream EGO heaters must be  on  prior

to current monitoring, seconds.

-  DS_CM_MAX  =  Maximum  downstream  EGO  temperature  to  enable   current

monitoring, degrees.

-  DS_CM_MIN = Minimum downstream  EGO  temperature  to  enable  EGO  heater

current monitoring, degrees.

-  DS_CUR_HI =  Maximum  allowed  EGO  heater  current  for  the  downstream

heaters, amps.

-  DS_CUR_LOW = Minimum  allowed  EGO  heater  current  for  the  downstream

heaters, amps.

-  DS_HTR_OFF = Temperature at which to turn off the downstream ego heaters.

-  DS_HTR_ON = Temperature at which to turn on the downstream ego heaters.

-  FN_HTR_XFER(HTRCM_CNTS) = Heater current ATOD input transfer function.

-  ER_INIT = Value for state of ER_STATUS when  engine  running  test  first

inits.

-  EXT_REGON_SF = Temperature offset to delay downstream heater turn  on  if

the inferred temperature model FMEM flag is set, degrees.

-  HEATER_HP11 = EGO heater 11 hardware present switch.

-  HEATER_HP12 = EGO heater 12 hardware present switch.

-  HEATER_HP21 = EGO heater 21 hardware present switch.

-  HEATER_HP22 = EGO heater 22 hardware present switch.

-  HTRCM_SW = Switch to enable current monitoring, unitless.

-  HTR_EXE_TM = Minimum  time  between  passes  of  the  heater  controller,

seconds (milli-second resolution).

-  HTR_VOLT_NOM  =  Nominal  battery  voltage  at  which   current   monitor

thresholds have been established.

11-68

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  RETRY_ALLOW = The total number of retries allowed when detecting a  fault

of an EGO heater.

-  US_CM_MAX = Maximum upstream EGO temperature to perform  the  EGO  heater

current monitor, degrees.

-  US_CM_MIN = Minimum upstream EGO temperature to perform  the  EGO  heater

current monitor, degrees.

-  US_CUR_HI = Maximum allowed EGO heater current for the upstream  heaters,

amps.

-  US_CUR_LOW = Minimum allowed EGO heater current for  the  upstream  EGOs,

amps.

-  US_HTR_ON = Temperature at which to turn on upstream EGO  sensor  heater,

degrees.

-  US_HTR_OFF = Temperature at which to turn off upstream ego sensors.

-  US_HEAT_TM = Amount of time the upstream EGO heaters must be on prior  to

EGO heater current monitoring.

OTHER

-  P0135 = Fault code for EGO11 sensor heater circuit malfunction, MIL_CODE.

-  P0141 = Fault code for EGO12 sensor heater circuit malfunction, MIL_CODE.

-  P0155 = Fault code for EGO21 sensor heater circuit malfunction, MIL_CODE.

-  P0161 = Fault code for EGO22 sensor heater circuit malfunction, MIL_CODE.

11-69

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EGO_HEATER_CONTROL_COM10

BEGIN: definitions

;The following represent definitions used throughout this module.  This

;is not to be explicitly executed.  It exists only to make the documentation

;more readable.

unconditionally ------------------------| on := 1

| off := 0

| true := 1

| false := 0

| fault := 1

| no_fault := 0

| all_on := 1

| all_on_diag_on := 2

| all_on_diag_off := 3

| us_on := 4

| us_on_diag_on := 5

| us_on_diag_off := 6

| ds_on := 7

| ds_on_diag_on := 8

| ds_on_diag_off := 9

| all_off := 10

| keyon_init := 11

| us_toggle := 12

| ds_toggle := 13

| cm_11 := 100

| cm_12 := 101

| cm_21 := 102

| cm_22 := 103

| cm_done := 104

END: definitions

BEGIN: reset_ego_heater

;The following logic, called on demand from the EGO monitor control

;process, is used to reset KAM parameters associated with the

;EGO heater control strategy.

unconditionally ------------------------| HTRCM11 := 1

| HTRCM21 := 1

| HTRCM12 := 1

| HTRCM22 := 1

| HTR_RET_MX11 := 0

| HTR_RET_MX12 := 0

| HTR_RET_MX21 := 0

| HTR_RET_MX22 := 0

END: reset_ego_heater

11-70

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ego_heater_control_init

;The following process is used to initialize the EGO heaters.  It is to be

;executed during a power-up iniitialization prior to exiting HLOS (hardware

;requirement) and when called at the start and end of KOER EGO test.

unconditionally -----------| HTR_RETRY11 := 0

| HTR_RETRY12 := 0

| HTR_RETRY21 := 0

| HTR_RETRY22 := 0

| HTR_MALF11 := false

| HTR_MALF12 := false

| HTR_MALF21 := false

| HTR_MALF22 := false

| HTR_MON11 := false

| HTR_MON12 := false

| HTR_MON21 := false

| HTR_MON22 := false

| TEST_ENABLED := false

| CURRENT_CKD := false

| HEATER_STATE := keyon_init

| CURR_TST_ENA := false

| HTR_EXE_TMR := 0

| EGO_CUR_MON := false

| EGO_HTR_MON := false

END: ego_heater_control_init

11-71

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: update_retry_max(n)

;The following process, called from within the ego_fault_filter routine,

;is used to update HTR_RET_MXn for use by the scan tool.

unconditionally ------------------------| HTR_RET_MX(n) := max(HTR_RET_MX(n),

HTR_RETRY(n))

END: update_retry_max

BEGIN: ego_heater_control

;The following process is called from the ego monitor control process.

;It acts as the 'main' program for EGO heater controls.  The actual

;hardware interface is implemented in lower level software processes

;(external to this module) which interface directly to the IMP heater

;driver chip, and the heater voltage A/D channel.  Ideally, the

;heater control strategy, IMP I/F, and the heater voltage A/D

;would be synchronous processes.  However, they are not, as the

;following paragraphs will describe.

;

;The IMP I/F process, which sends the actual heater on/off commands

;and checks fault status, is timer driven and currently runs at about 26.5ms

;rates.  The heater controller strategy runs in the background loop,

;asynchronous from the IMP HW I/F process.  The heater controller

;requires the IMP I/F process to run at least twice between each

;strategy background loop for proper detection of heater fault flags.

;Since background loop times can approach 50ms, this condition may not

;always be met.

;

;The A/D process for the heater current monitor is implemented as part

;of the background loop.  A/D action assumes the heaters are in the

;HEATER_STATE as set by the strategy.  However, because the background

;loop is asynchronous to the IMP servicer, the A/D could sample the

;the HEATER_STATE prior to the IMP servicer issuing the new heater

;commands.  This lack of synchronization could produce bad current

;measurements.

;

;Slowing down the heater control logic will aviod these problems.

;A milli-second resolution timer, HTR_EXE_TMR, is used to force

;synchronization between all these processes by allowing the heater

;controller strategy to execute in a background loop only if a calibratable

;period of time has ellapsed.  This calibration constant should be set

;to a value of (at least) twice the IMP I/F service rate, as shown below.

;This guarantees fault flags will be detected, and accurate current

;measurements.  Although not an ideal fix, it was implemented to meet

;Job #1 requirements for the heater current monitor.

;

11-72

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;

;  IMP servicer (send heater on/off commands, read fault status)

;  ------|----------|----------|----------|----------|----------|------>

;        <-  ~26ms ->

;

;  Background loops (current monitor A/D action)

;  ---------|------------------|------------------|------------------|->

;           <--  ~50ms (min) -->

;

;  Heater control strategy (set heater on/off states)

;  ---------|-------------------------------------|-------------------->

;           <------  HTR_EXE_TM ------->

;

HTR_EXE_TMR >= HTR_EXE_TM -| Do: ego_heater_fault_status

| Do: ego_heater_state_control

| Do: ego_heater_mode_select

| Do: ego_heater_diag_state_control

| Do: ego_heater_timer_control

| Do: ego_heater_current_monitor

| Do: ego_heater_mon_control

| Do: ego_heater_output

| HTR_EXE_TMR := 0

|

| --- ELSE ---

|

| ;allow the free running timer,

| ;HTR_EXE_TMR, to increment.

;

;Setting the cal constant HTR_EXE_TM = 0 will effectively

;calibrate out the timer so the heater controller runs in

;every background loop.

;

END: ego_heater_control

11-73

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ego_heater_current_monitor

;The following process is executed only when called and is used to

;perform current monitoring at the appropriate time.  Current

;monitor entry conditions during nomral operation require that

;the US/DS heaters have been turned on and are warmed up to their

;steady state resistance values.  Temperature conditions are

;inferred with the US/DS_HTR_TMRs.

;

;The downstream heaters must be on for CM_HEAT_TM seconds before

;steady state is assumed.  Note that CM_HEAT_TM is different

;from the DS_HEAT_TM constant used in the Ego controller to

;initiate the DS monitor.  This gives flexibility to adjust

;downstream heater readiness for each of these monitors.

;

;Normal time/temp entry conditions into the current monitor will

;be bypassed during KOEO.  KOEO current monitoring is intended

;to detect disconnected heaters with a low current fault check,

;which does not require the heaters to be at steady state.

;

;The flag EGO_CUR_MON is set to true upon completion of the

;current monitor, and is used here to lock out the test from

;re-running after it completes.

;

CURR_TST_ENA = true -----------------|

|

HEATER_STATE = all_on ---------|     |

|     |

HEATER_STATE = cm_11 ----------|     |

|     |

HEATER_STATE = cm_12 ----------|OR --|

|     |

HEATER_STATE = cm_21 ----------|     |

|     |

HEATER_STATE = cm_22 ----------|     |

|     |

HEATER_STATE = cm_done --------|     |

|AND -| Do:current_monitor_state_control

EGO_CUR_MON = false -----------------|

|

US_HTR_TMR > US_HEAT_TM -|           |

|           |

DS_HTR_TMR > CM_HEAT_TM -|           |

|           |

EXT_REG < DS_CM_MAX -----|           |

|AND -|     |

EXT_REG > DS_CM_MIN -----|     |     |

|     |     |

EXT_FEU < US_CM_MAX -----|     |OR --|

|     |

EXT_FEU > US_CM_MIN -----|     |

| ;bypass time/temp entry conditions

OSM_EO_ON = true --------------| ;for KOEO test

END: ego_heater_current_monitor

11-74

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ego_heater_mon_control

;The following process is executed only when called and is used to signal

;back to the ego monitor control process that EGO heater monitoring

;is complete.

HEATER_HP11 = false --|

|OR --|

HTR_MON11 = true -----|     |

|

HEATER_HP21 = false --|     |

|OR --|

HTR_MON21 = true -----|     |

|AND -| heaters_monitored := true

HEATER_HP12 = false --|     |     |

|OR --|     |

HTR_MON12 = true -----|     |     |

|     |

HEATER_HP22 = false --|     |     |

|OR --|     |

HTR_MON22 = true -----|           |

| --- ELSE ---

|

| heaters_monitored := false

heaters_monitored = true ---|

|AND -| EGO_HTR_MON := true

HTRCM_SW = false -----|     |

|OR --|

EGO_CUR_MON = true ---|

END: ego_heater_mon_control

11-75

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: koeo_test

;The following process is only executed when called and is used to perform

;heater fault monitoring during a KOEO on demand test.

HTR_FAULT11 = true ---------|

|AND -| Do: malfunction(ego, P0135)

HEATER_HP11 = true ---------|     |

| --- ELSE ---

|

| Do: clear_malf(P0135)

HTR_FAULT21 = true ---------|

|AND -| Do: malfunction(ego, P0155)

HEATER_HP21 = true ---------|     |

| --- ELSE ---

|

| Do: clear_malf(P0155)

HTR_FAULT12 = true ---------|

|AND -| Do: malfunction(ego, P0141)

HEATER_HP12 = true ---------|     |

| --- ELSE ---

|

| Do: clear_malf(P0141)

HTR_FAULT22 = true ---------|

|AND -| Do: malfunction(ego, P0161)

HEATER_HP22 = true ---------|     |

| --- ELSE ---

|

| Do: clear_malf(P0161)

END: koeo_test

11-76

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: current_fault_check

;The following process is only executed when called and is used to report

;malfunctions to the OBD-II Diagnostic Executive after current monitoring.

;

;The high current fault check is bypassed during KOEO.  KOEO current

;monitoring is intended to detect disconnected heaters with a low

;current fault check.  The high current check will be inaccurate,

;since the heaters will not reach their steady state resistance

;values (which are based on temperature) with the engine off.

;

HTRCM11 < US_CUR_LOW ------|

|OR --|

HTRCM11 > US_CUR_HI -|     |     |

|AND -|     |AND -| Do: malfunction(ego, P0135)

OSM_EO_ON = false ---|           |     |

|     |

HEATER_HP11 = true --------------|     |

| --- ELSE ---

|

| Do: clear_malf(P0135)

HTRCM12 < DS_CUR_LOW ------|

|OR --|

HTRCM12 > DS_CUR_HI -|     |     |

|AND -|     |AND -| Do: malfunction(ego, P0141)

OSM_EO_ON = false ---|           |     |

|     |

HEATER_HP12 = true --------------|     |

| --- ELSE ---

|

| Do: clear_malf(P0141)

HTRCM21 < US_CUR_LOW ------|

|OR --|

HTRCM21 > US_CUR_HI -|     |     |

|AND -|     |AND -| Do: malfunction(ego, P0155)

OSM_EO_ON = false ---|           |     |

|     |

HEATER_HP21 = true --------------|     |

| --- ELSE ---

|

| Do: clear_malf(P0155)

HTRCM22 < DS_CUR_LOW ------|

|OR --|

HTRCM22 > DS_CUR_HI -|     |     |

|AND -|     |AND -| Do: malfunction(ego, P0161)

OSM_EO_ON = false ---|           |     |

|     |

HEATER_HP22 = true --------------|     |

| --- ELSE ---

|

| Do: clear_malf(P0161)

END: current_fault_check

11-77

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: current_monitor_state_control

;The following process is only executed when called and is used to control

;state of the EGO heaters when current monitoring is in progress.

HTRCM_SW = false --------------| HEATER_STATE := all_on

| EGO_CUR_MON := true

|

| --- ELSE ---

|

HEATER_STATE = all_on ---------| HEATER_STATE := cm_11

| CURRENT_CKD := false

|

| --- ELSE ---

HEATER_STATE = cm_11 ----|     |

|     |

HEATER_HP21 = true ------|AND -| htrcm11 := FN_HTR_XFER(HTRCM_CNTS)

|     | HTRCM11 := htrcm11 * HTR_VOLT_NOM / VBAT

CURRENT_CKD = true ------|     | HEATER_STATE := cm_21

| CURRENT_CKD := false

|

| --- ELSE ---

HEATER_STATE = cm_11 ----|     |

|AND -| HTRCM11 := FN_HTR_XFER(HTRCM_CNTS)

CURRENT_CKD = true ------|     | HTRCM11 := htrcm11 * HTR_VOLT_NOM / VBAT

| HEATER_STATE := cm_12

| CURRENT_CKD := false

|

| --- ELSE ---

HEATER_STATE = cm_21 ----|     |

|AND -| htrcm21 := FN_HTR_XFER(HTRCM_CNTS)

CURRENT_CKD = true ------|     | HTRCM21 := htrcm21 * HTR_VOLT_NOM / VBAT

| HEATER_STATE := cm_12

| CURRENT_CKD := false

|

| --- ELSE ---

(continued on next page)

11-78

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

HEATER_STATE = cm_12 ----|     |

|     |

HEATER_HP22 = true ------|AND -| htrcm12 := FN_HTR_XFER(HTRCM_CNTS)

|     | HTRCM12 := htrcm12 * HTR_VOLT_NOM / VBAT

CURRENT_CKD = true ------|     | HEATER_STATE := cm_22

| CURRENT_CKD := false

|

| --- ELSE ---

HEATER_STATE = cm_12 ----|     |

|AND -| htrcm12 := FN_HTR_XFER(HTRCM_CNTS)

CURRENT_CKD = true ------|     | HTRCM12 := htrcm12 * HTR_VOLT_NOM / VBAT

| HEATER_STATE := cm_done

| CURRENT_CKD := false

|

| --- ELSE ---

HEATER_STATE = cm_22 ----|     |

|AND -| htrcm22 := FN_HTR_XFER(HTRCM_CNTS)

CURRENT_CKD = true ------|     | HTRCM22 := htrcm22 * HTR_VOLT_NOM / VBAT

| HEATER_STATE := cm_done

| CURRENT_CKD := false

|

| --- ELSE ---

|

HEATER_STATE = cm_done --------| HEATER_STATE := all_on

| EGO_CUR_MON := true

| Do: current_fault_check

;Allow one background loop to execute to clear fault bits set during

;current monitoring.

END: current_monitor_state_control

11-79

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ego_heater_state_control

;The following process is executed only when called and is used to control

;the state of the EGO heaters during all normal, non-diagnostic modes.

;

;Reset all heater control parameters at the start and end of the

;KOEO/KOER self tests.  For KOEO, force current monitoring (bypass

;normal time/temp entry conditions) and the koeo heater on/off state

;voltage checks.  For KOER, allow the heater monitor to run naturally,

;based on it's normal time and temp entry conditions.

;

HTR_RETRY11 = 0 ---------------|

|OR --|

HTR_RETRY11 = RETRY_ALLOW -----|     |

|

HTR_RETRY12 = 0 ---------------|     |

|OR --|

HTR_RETRY12 = RETRY_ALLOW -----|     |

|AND -| ok_to_change_state := true

HTR_RETRY21 = 0 ---------------|     |     |

|OR --|     |

HTR_RETRY21 = RETRY_ALLOW -----|     |     |

|     |

HTR_RETRY22 = 0 ---------------|     |     |

|OR --|     |

HTR_RETRY22 = RETRY_ALLOW -----|           |

| --- ELSE ---

|

| ok_to_change_state := false

; Check inferred temperature model FMEM flag.  If set, add temperature

; offset to downstream heater turn on temperature.  This will delay

; heater turn on to avoid potential ceramic cracking during cold starts.

; Local parameter used so calculation is only done once.

ds_htr_on_fmm = DS_HTR_ON + EXT_FMM_FLG*EXT_REGON_SF

;

;Detect start of the KOEO self test, and initialize heater control

;parms.  Look for the rising and falling edges of EO_ACTIVE.

;For 96 applications, move KOEO init logic into the Self

;Test strategy modules with the rest of the KOEO controller.

;

EO_ACTIVE = 1 --------------------|

|AND -| Do: ego_heater_control_init

EO_INIT_FLG = 0 ------------------|     | Do: ego_heater_io_init

| EO_INIT_FLG := 1

| ;set koeo start flag

|

| --- ELSE ---

EO_ACTIVE = 0 --------------------|     |

|AND -| EO_INIT_FLG := 0

EO_INIT_FLG = 1 ------------------|     | ;clear koeo start flag

11-80

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OBDII_RESET = true ---------------------|

|

ER_STATUS = ER_INIT --------------------|

|OR --| Do: ego_heater_control_init

ER_STATUS = ER_DONE --------------------|     | Do: ego_heater_io_init

|     | ;clean up before and after

EO_STATUS = EO_DONE --------------------|     | ;self tests.

|

| --- ELSE ---

|

HEATER_STATE = keyon_init --------------------| HEATER_STATE := us_toggle

| ;toggle upstream heaters

| ;on/off to reset IMP IC

| ;fault flags and heater

| ;state at powerup.

|

| --- ELSE ---

|

HEATER_STATE = us_toggle ---------------------| HEATER_STATE := ds_toggle

| ;Toggle upstream heater pair

| ;first. (Turning on one pair

| ;at a time avoids excessive

| ;surge currents.)

|

| --- ELSE ---

|

HEATER_STATE = ds_toggle ---------------------| HEATER_STATE := all_off

| ;Reset of the IMP IC fault

| ;flags and heater states

| ;complete.

|

| --- ELSE ---

OSM_EO_ON = true -----------------------|     |

|     |

EGO_CUR_MON = false --------------------|AND -| HEATER_STATE := all_on

|     | ;trigger current monitor

HEATER_STATE = all_off -----------------|     | ;for KOEO

|

| --- ELSE ---

OSM_EO_ON = true -----------------------|     |

|AND -| HEATER_STATE := all_on

EGO_CUR_MON = true ---------------------|     | Do: koeo_test

| ;wait for current monitor

| ;to complete

|

| --- ELSE ---

|

OSM_EO_ON = true -----------------------------| Do nothing

| ;Prevent underspeed flag

| ;from shutting heaters off

| ;while in KOEO current mon.

|

| --- ELSE ---

|

(continued on next page)

11-81

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

OSM_EO_OFF = true ----------------------------| HEATER_STATE := all_off

| Do: koeo_test

|

| --- ELSE ---

|

ok_to_change_state = true --------------|     |

|     |

HEATER_STATE = all_off -----|           |     |

|           |     |

EXT_FEU > US_HTR_ON --------|           |AND -| HEATER_STATE := us_on

|AND -|     |     |

EXT_FEU < US_HTR_OFF -------|     |     |     |

|     |     |     |

UNDSP = 0 ------------------|     |OR --|     |

|           |

HEATER_STATE = all_on ------|     |           |

|AND -|           |

EXT_REG >= DS_HTR_OFF ------|                 |

| --- ELSE ---

ok_to_change_state = true---------------|     |

|     |

HEATER_STATE = all_off -----|           |     |

|           |     |

EXT_FEU >= US_HTR_OFF ------|           |     |

|           |AND -| HEATER_STATE := ds_on

ds_htr_on_fmm <= EXT_REG ---|AND -|     |     |

|     |     |     |

EXT_REG < DS_HTR_OFF -------|     |     |     |

|     |     |     |

UNDSP = 0 ------------------|     |     |     |

|OR --|     |

HEATER_STATE = all_on ------|     |           |

|AND -|           |

EXT_FEU >= US_HTR_OFF ------|                 |

| --- ELSE ---

ok_to_change_state = true---------------|     |

|     |

UNDSP = 1 ------------------------|     |     |

|     |     |

HEATER_STATE = us_on -------|     |     |AND -| HEATER_STATE := all_off

|AND -|     |     |

EXT_FEU >= US_HTR_OFF ------|     |     |     |

|OR --|     |

HEATER_STATE = ds_on -------|     |           |

|AND -|           |

EXT_REG >= DS_HTR_OFF ------|                 |

| --- ELSE ---

(continued on next page)

11-82

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

ok_to_change_state = true --------------|     |

|     |

HEATER_STATE = us_on -----  |           |     |

|           |     |

ds_htr_on_fmm <= EXT_REG ---|AND -|     |     |

|     |     |AND -| HEATER_STATE := all_on

EXT_REG < DS_HTR_OFF -------|     |     |

|     |

HEATER_STATE = ds_on -------|     |     |

|AND -|OR --|

EXT_FEU < US_HTR_OFF -------|     |

|

CURR_TST_ENA = false -------|     |

|     |

HEATER_STATE = cm_11 -|     |AND -|

|     |

HEATER_STATE = cm_12 -|     |

|OR --|

HEATER_STATE = cm_21 -|

|

HEATER_STATE = cm_22 -|

END: ego_heater_state_control

11-83

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN:  ego_heater_mode_select

;The following process is executed only when called and is used to

;determine if the state controls should be in a normal mode or a

;test mode.

TEST_ENABLED = true ---------------------|

|

HEATER_STATE = us_on --------|           |

|AND -|     |

HTR_MON11 = false -----|     |     |     |AND -|

|OR --|     |     |     |

HTR_MON21 = false -----|           |     |     |

|     |     |

HEATER_STATE = ds_on --------|     |     |     |

|AND -|OR --|     |

HTR_MON12 = false -----|     |     |           |

|OR --|     |           |

HTR_MON22 = false -----|           |           |

|           |

HEATER_STATE = all_on -------|     |           |

|     |           |

HTR_MON11 = false -----|     |AND -|           |

|     |                 |

HTR_MON12 = false -----|     |                 |

|OR --|                 |

HTR_MON21 = false -----|                       |

|                       |

HTR_MON22 = false -----|                       |OR --| NORMAL_MODE := false

|     |

HTR_RETRY11 = RETRY_ALLOW ---------------|     |     |

|     |     |

HTR_FAULT11 = no_fault ------------------|     |     |

|AND -|     |

OLDHTRFALT11 = fault --------------------|     |     |

|     |     |

HEGOHTR11 = OLDHEGOHTR11 ----------------|     |     |

|     |

HTR_RETRY12 = RETRY_ALLOW ---------------|     |     |

|     |     |

HTR_FAULT12 = no_fault ------------------|     |     |

|AND -|     |

OLDHTRFALT12 = fault --------------------|     |     |

|     |     |

HEGOHTR12 = OLDHEGOHTR12 ----------------|     |     |

|     |

HTR_RETRY21 = RETRY_ALLOW ---------------|     |     |

|     |     |

HTR_FAULT21 = no_fault ------------------|     |     |

|AND -|     |

OLDHTRFALT21 = fault --------------------|     |     |

|     |     |

HEGOHTR21 = OLDHEGOHTR21 ----------------|     |     |

|     |

(continued on next page)

11-84

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

HTR_RETRY22 = RETRY_ALLOW ---------------|     |     |

|     |     |

HTR_FAULT22 = no_fault ------------------|     |     |

|AND -|     |

OLDHTRFALT22 = fault --------------------|           |

|           |

HEGOHTR22 = OLDHEGOHTR22 ----------------|           |

| --- ELSE ---

|

| NORMAL_MODE := true

END: ego_heater_mode_select

BEGIN: reset_diag_state

;The following process is only executed when called and is used to return

;to a non-diagnostic state when testing gets disabled.

HEATER_STATE = us_on_diag_on -----|

|OR --| HEATER_STATE := us_on

HEATER_STATE = us_on_diag_off ----|     |

| --- ELSE ---

HEATER_STATE = ds_on_diag_on -----|     |

|OR --| HEATER_STATE := ds_on

HEATER_STATE = ds_on_diag_off ----|     |

| --- ELSE ---

HEATER_STATE = all_on_diag_on ----|     |

|OR --| HEATER_STATE := all_on

HEATER_STATE = all_on_diag_off ---|

END: reset_diag_state

11-85

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: check_us_faults

;The following process is only executed when called and latches faults on

;the upstream EGO heaters.

HTR_FAULT11 = fault ----------| HTR_MALF11 := true

HTR_FAULT21 = fault ----------| HTR_MALF21 := true

END: check_us_faults

BEGIN: check_ds_faults

;The following process is only executed when called and latches faults on

;the downstream EGO heaters.

HTR_FAULT12 = fault ----------| HTR_MALF12 := true

HTR_FAULT22 = fault ----------| HTR_MALF22 := true

END: check_ds_faults

11-86

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: us_fault_filtering

;The following process is only executed when called and is used to perform

;fault filtering on the upstream EGO heaters.

HTR_FAULT11 = fault ------|

|OR --| HTR_RETRY11 := min((HTR_RETRY11+1),

HTR_MALF11 = true --------|     |                    RETRY_ALLOW)

| Do: update_retry_max(11)

|

| --- ELSE ---

| Do: clear_malf(P0135)

| HTR_MON11 := true

| HTR_RETRY11 := 0

HTR_FAULT21 = fault ------|

|OR --| HTR_RETRY21 := min((HTR_RETRY21+1),

HTR_MALF21 = true --------|     |                    RETRY_ALLOW)

| Do: update_retry_max(21)

|

| --- ELSE ---

| Do: clear_malf(P0155)

| HTR_MON21 := true

| HTR_RETRY21 := 0

END: us_fault_filtering

BEGIN: ds_fault_filtering

;The following process is only executed when called and is used to perform

;fault filtering on the downstream EGO heaters.

HTR_FAULT12 = fault ------|

|OR --| HTR_RETRY12 := min((HTR_RETRY12+1),

HTR_MALF12 = true --------|     |                    RETRY_ALLOW)

| Do: update_retry_max(12)

|

| --- ELSE ---

|

| Do: clear_malf(P0141)

| HTR_MON12 := true

| HTR_RETRY12 := 0

HTR_FAULT22 = fault ------|

|OR --| HTR_RETRY22 := min((HTR_RETRY22+1),

HTR_MALF22 = true --------|     |                    RETRY_ALLOW)

| Do: update_retry_max(22)

|

| --- ELSE ---

|

| Do: clear_malf(P0161)

| HTR_MON22 := true

| HTR_RETRY22 := 0

END: ds_fault_filtering

11-87

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ego_heater_diag_state_control

;The following process is only executed when called and is used to control

;the state of the EGO heaters during non-current monitoring diagnostics.

unconditionally --------------------------| old_test_enabled := TEST_ENABLED

EGO_TST_ENA = true -----------------|

|

CCM_ER_ENA = true ------------------|OR --| TEST_ENABLED := true

|     |

CCM_TST_ENA = true -----------------|     |

| --- ELSE ---

|

| TEST_ENABLED := false

old_test_enabled = true ------------|

|AND -| HTR_RETRY11 := 0

TEST_ENABLED = false ---------------|     | HTR_RETRY21 := 0

| HTR_RETRY12 := 0

| HTR_RETRY22 := 0

| HTR_MON11 := false

| HTR_MON21 := false

| HTR_MON12 := false

| HTR_MON22 := false

| HTR_MALF11 := false

| HTR_MALF21 := false

| HTR_MALF12 := false

| HTR_MALF22 := false

| EGO_HTR_MON := false

| Do: clear_malf(P0135)

| Do: clear_malf(P0155)

| Do: clear_malf(P0141)

| Do: clear_malf(P0161)

| Do: reset_diag_state

UNDSP = 1 --------------------------------| EXIT this process.

11-88

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

HTR_FAULT11 = fault ----------|

|AND -|

HTR_RETRY11 < RETRY_ALLOW ----|     |

|OR --| US_FAULTED := true

HTR_FAULT21 = fault ----------|     |     |

|AND -|     |

HTR_RETRY21 < RETRY_ALLOW ----|           |

| --- ELSE ---

|

| US_FAULTED := false

HTR_FAULT12 = fault ----------|

|AND -|

HTR_RETRY12 < RETRY_ALLOW ----|     |

|OR --| DS_FAULTED := true

HTR_FAULT22 = fault ----------|     |     |

|AND -|     |

HTR_RETRY22 < RETRY_ALLOW ----|           |

| --- ELSE ---

|

| DS_FAULTED := false

11-89

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

HEATER_STATE = us_on_diag_off -------------------| Do: check_us_faults

| HEATER_STATE :=

|   us_on_diag_on

|

| --- ELSE ---

|

HEATER_STATE = ds_on_diag_off -------------------| Do: check_ds_faults

| HEATER_STATE :=

|   ds_on_diag_on

|

| --- ELSE ---

|

HEATER_STATE = all_on_diag_off ------------------| Do: check_us_faults

| Do: check_ds_faults

| HEATER_STATE :=

|   all_on_diag_on

|

| --- ELSE ---

|

HEATER_STATE = us_on_diag_on --------------------| Do: us_fault_filtering

| HEATER_STATE := us_on

|

| --- ELSE ---

|

HEATER_STATE = ds_on_diag_on --------------------| Do: ds_fault_filtering

| HEATER_STATE := ds_on

|

| --- ELSE ---

|

HEATER_STATE = all_on_diag_on -------------------| Do: us_fault_filtering

| Do: ds_fault_filtering

| HEATER_STATE := all_o

|

| --- ELSE ---

TEST_ENABLED = true -----------------------|     |

|     |

HEATER_STATE = us_on ----------------------|     |

|     |

HTR_RETRY11 > 0 ---------------|           |AND -| HEATER_STATE :=

|AND -|     |     |   us_on_diag_off

HTR_RETRY11 < RETRY_ALLOW -----|     |     |     | HTR_MALF11 := false

|     |     | HTR_MALF21 := false

HTR_RETRY21 > 0 ---------------|     |     |     |

|AND -|     |     |

HTR_RETRY21 < RETRY_ALLOW -----|     |OR --|     |

|           |

US_FAULTED = true -------------------|           |

|           |

NORMAL_MODE = false -----------------|           |

| --- ELSE ---

(continued on next page)

11-90

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|

TEST_ENABLED = true -----------------------|     |

|     |

HEATER_STATE = ds_on ----------------------|     |

|     |

HTR_RETRY12 > 0 ---------------|           |AND -| HEATER_STATE :=

|AND -|     |     |   ds_on_diag_off

HTR_RETRY12 < RETRY_ALLOW -----|     |     |     | HTR_MALF12 := false

|     |     | HTR_MALF22 := false

HTR_RETRY22 > 0 ---------------|     |     |     |

|AND -|     |     |

HTR_RETRY22 < RETRY_ALLOW -----|     |OR --|     |

|           |

DS_FAULTED = true -------------------|           |

|           |

NORMAL_MODE = false -----------------|           |

| --- ELSE ---

TEST_ENABLED = true -----------------------|     |

|     |

HEATER_STATE = all_on ---------------------|     |

|     |

HTR_RETRY11 > 0 ---------------|           |     |

|AND -|     |     |

HTR_RETRY11 < RETRY_ALLOW -----|     |     |     |

|     |     |

HTR_RETRY21 > 0 ---------------|     |     |     |

|AND -|     |AND -| HEATER_STATE :=

HTR_RETRY21 < RETRY_ALLOW -----|     |     |     |   all_on_diag_off

|     |     | HTR_MALF11 := false

HTR_RETRY12 > 0 ---------------|     |     |     | HTR_MALF21 := false

|AND -|     |     | HTR_MALF12 := false

HTR_RETRY12 < RETRY_ALLOW -----|     |     |     | HTR_MALF22 := false

|     |

HTR_RETRY22 > 0 ---------------|     |     |

|AND -|     |

HTR_RETRY22 < RETRY_ALLOW -----|     |OR --|

|

US_FAULTED = true -------------------|

|

DS_FAULTED = true -------------------|

|

NORMAL_MODE = false -----------------|

TEST_ENABLED = true -----------------| Do: ego_heater_fault_report

END: ego_heater_diag_state_control

11-91

EGO MONITOR CONTROL STRATEGY, EGO HEATER CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN:  ego_heater_fault_report

;The following process is only executed when called and is used to control

;the fault codes associated with non-current monitoring diagnostics.

HTR_RETRY11 >= RETRY_ALLOW ---------| Do: malfunction(ego, P0135)

| HTR_MON11 := true

HTR_RETRY12 >= RETRY_ALLOW ---------| Do: malfunction(ego, P0141)

| HTR_MON12 := true

HTR_RETRY21 >= RETRY_ALLOW ---------| Do: malfunction(ego, P0155)

| HTR_MON21 := true

HTR_RETRY22 >= RETRY_ALLOW ---------| Do: malfunction(ego, P0161)

| HTR_MON22 := true

END: ego_heater_fault_report

BEGIN: ego_heater_timer_control

;The following process is only executed when called and is used to control

;timers that indicates how long the EGO heaters have been on.

HEATER_STATE = all_off -------|

|OR --| DS_HTR_TMR := 0

HEATER_STATE = us_on ---------|     |

| --- ELSE ---

|

|  ;Allow the free running timer,

|  ;DS_HTR_TMR, to increment.

HEATER_STATE = all_off -------|

|OR --| US_HTR_TMR := 0

HEATER_STATE = ds_on ---------|     |

| --- ELSE ---

|

|  ;Allow the free running timer,

|  ;US_HTR_TMR, to increment.

END: ego_heater_timer_control

11-92

EGO MONITOR CONTROL STRATEGY, EGO HEATER IO - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

11.1.5  EGO HEATER IO (CDAN0)

This module interfaces the EGO heater control and  monitoring  logic  to  the

heater IMP IC for ML-II hardware.  It contains the following processes.

+-------------------------------+

| EGO_HEATER_IO_COM10           |

+-------------------------------+

|PUBLIC PROCEDURES:             |

|. ego_heater_io_init           |

|. ego_heater_output            |

|. ego_heater_fault_status      |

+-------------------------------+

Two parameters interface the heater control  state  machine  logic  with  the

hardware:   HEATER_STATE  and  HTR_FAULTxy.   HEATER_STATE  is encoded by the

state  machine  with  the  following  information:   heater   mode   (normal,

diagnostics, current monitoring), upstream heater pair on/off, and downstream

heater pair on/off.  The ego_heater_output process decodes this parameter and

sets the HEGOHTRxy flags on or off.

The HEGOHTRxy flags are placed by software into a command byte  sent  to  the

IMP  IC  to  control  heater  hardware.   For each command byte sent, the IMP

returns a status byte for the previous command.  Software  reads  this  value

and  places  heater  status into the HTR_FAULTxy flags.  These flags are used

back in the control module for heater diagnositics.

Refer to the IMP Device Description and Design Specification for  a  complete

description of the IMP IC operation.

11-93

EGO MONITOR CONTROL STRATEGY, EGO HEATER IO - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  ER_STATUS =  During  the  engine  running  self  test  (KOER)  this  byte

indicates which part of the test is running.  For example, it will be set

to state "ER_INIT" at the beginning of the test .

-  HEATER_STATE = State machine  set  in  the  heater  control  module,  and

decoded by this module to set the HEGOHTRxy flags on/off.

Bit Flags:

-  HEGOHTRxy = EGO heater on/off flags.  These flags are put into a  command

byte  by  software,  and  sent to the IMP IC.  'xy' is array notation for

bank x, location y (upstream or downstream).

-  HTR_FAULTxy = Heater Fault  flags.   The  IMP  responds  to  each  heater

command  with  a  heater  status  byte,  which software places into these

flags.  Value of 1 indicates voltage fault for heater 'xy'.

-  OBDII_RESET = Flag used to simulate the receipt of an  OBD-II  scan  tool

reset message.

-  OFF_FAULTxy = Voltage fault detected for heater 'xy'  when  it  was  last

off.

-  OLDHEGOHTRxy = Previous background pass value of HEGOHTRxy.

-  OLDHTRFALTxy = Previous background value of HTR_FAULTxy.

-  ON_FAULTxy = Voltage fault detected for heater 'xy' when it was last on.

-  SUBST_REQ25:28 = Output State Control  commanded  substitution  requested

for heater channels 25-28.

Calibration Constants:

-  ER_DONE  =  Value  for  state  of  ER_STATUS  when  engine  running  test

completes.

-  ER_INIT = Value for state of ER_STATUS when  engine  running  test  first

inits.

-  HEATER_HPxy = EGO heater xy hardware present switch.

11-94

EGO MONITOR CONTROL STRATEGY, EGO HEATER IO - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EGO_HEATER_IO_COM2

BEGIN: definitions

;The following represent definitions used throughout this module.  This

;is not to be explicitly executed.  It exists only to make the documentation

;more readable.

unconditionally ------------------------| on := 1

| off := 0

| true := 1

| false := 0

| fault := 1

| no_fault := 0

| all_on := 1

| all_on_diag_on := 2

| all_on_diag_off := 3

| us_on := 4

| us_on_diag_on := 5

| us_on_diag_off := 6

| ds_on := 7

| ds_on_diag_on := 8

| ds_on_diag_off := 9

| all_off := 10

| keyon_init := 11

| us_toggle := 12

| ds_toggle := 13

| cm_11 := 100

| cm_12 := 101

| cm_21 := 102

| cm_22 := 103

| cm_done := 104

END: definitions

11-95

EGO MONITOR CONTROL STRATEGY, EGO HEATER IO - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ego_heater_io_init

;The following process is used to initialize the EGO heater heater interface

;parameters.  It is executed during a power-up iniitialization prior to

exiting

;HLOS (hardware requirement) and when called at the start and end of KOER

EGO

;test in the ego_heater_control module.

;

unconditionally -----------| HEGOHTR11 := off

| HEGOHTR21 := off

| HEGOHTR12 := off

| HEGOHTR22 := off

| OLDHEGOHTR11 := off

| OLDHEGOHTR21 := off

| OLDHEGOHTR12 := off

| OLDHEGOHTR22 := off

| HTR_FAULT11 := no_fault

| HTR_FAULT12 := no_fault

| HTR_FAULT21 := no_fault

| HTR_FAULT22 := no_fault

| OLDHTRFALT11 := no_fault

| OLDHTRFALT12 := no_fault

| OLDHTRFALT21 := no_fault

| OLDHTRFALT22 := no_fault

| ON_FAULT11 := false

| ON_FAULT12 := false

| ON_FAULT21 := false

| ON_FAULT22 := false

| OFF_FAULT11 := false

| OFF_FAULT12 := false

| OFF_FAULT21 := false

| OFF_FAULT22 := false

END: ego_heater_io_init

11-96

EGO MONITOR CONTROL STRATEGY, EGO HEATER IO - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ego_heater_output

;The following process, executed only when called is used to write

;to the output parameters to turn on or off the heaters.

unconditionally -----------| OLDHEGOHTR11 := HEGOHTR11

| OLDHEGOHTR12 := HEGOHTR12

| OLDHEGOHTR21 := HEGOHTR21

| OLDHEGOHTR22 := HEGOHTR22

HEATER_STATE = all_off --------------|

|

HEATER_STATE = all_on_diag_off-|     |

|     |

ON_FAULT11 = true -------|     |     |

|     |     |

OFF_FAULT11 = true ------|     |     |

|OR --|     |

ON_FAULT21 = true -------|     |     |

|     |     |

OFF_FAULT21 = true ------|     |     |

|AND -|OR --| hegohtr11 := off

ON_FAULT12 = true -------|     |     |     | hegohtr12 := off

|     |     |     | hegohtr21 := off

OFF_FAULT12 = true ------|     |     |     | hegohtr22 := off

|OR --|     |     |

ON_FAULT22 = true -------|           |     |

|           |     |

OFF_FAULT22 = true ------|           |     |

|     |

HEATER_STATE = us_on_diag_off -------|     |

|     |

HEATER_STATE = ds_on_diag_off -------|     |

| --- ELSE ---

HEATER_STATE = all_on ---------------|     |

|OR --| hegohtr11 := on

HEATER_STATE = all_on_diag_on -------|     | hegohtr12 := on

| hegohtr21 := on

| hegohtr22 := on

|

| --- ELSE ---

HEATER_STATE = us_on ----------------|     |

|     |

HEATER_STATE = us_on_diag_on --------|     |

|OR --| hegohtr11 := on

HEATER_STATE = all_on_diag_off-|     |     | hegohtr12 := off

|     |     | hegohtr21 := on

ON_FAULT12 = true -------|     |AND -|     | hegohtr22 := off

|     |           |

OFF_FAULT12 = true ------|     |           |

|OR --|           |

ON_FAULT22 = true -------|                 |

|                 |

OFF_FAULT22 = true ------|                 |

| --- ELSE ---

(continued on next page)

11-97

EGO MONITOR CONTROL STRATEGY, EGO HEATER IO - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|

HEATER_STATE = ds_on ----------------|     |

|     |

HEATER_STATE = ds_on_diag_on --------|     |

|OR --| hegohtr11 := off

HEATER_STATE = all_on_diag_off-|     |     | hegohtr12 := on

|     |     | hegohtr21 := off

ON_FAULT11 = true -------|     |AND -|     | hegohtr22 := on

|     |           |

OFF_FAULT11 = true ------|     |           |

|OR --|           |

ON_FAULT21 = true -------|                 |

|                 |

OFF_FAULT21 = true ------|                 |

| --- ELSE ---

|

HEATER_STATE = cm_11 ----------------------| hegohtr11 := off

| hegohtr12 := on

| hegohtr21 := on

| hegohtr22 := on

|

| --- ELSE ---

|

HEATER_STATE = cm_21 ----------------------| hegohtr11 := on

| hegohtr12 := on

| hegohtr21 := off

| hegohtr22 := on

|

| --- ELSE ---

|

HEATER_STATE = cm_12 ----------------------| hegohtr11 := on

| hegohtr12 := off

| hegohtr21 := on

| hegohtr22 := on

|

| --- ELSE ---

|

HEATER_STATE = cm_22 ----------------------| hegohtr11 := on

| hegohtr12 := on

| hegohtr21 := on

| hegohtr22 := off

|

| --- ELSE ---

|

HEATER_STATE = cm_done --------------------| hegohtr11 := on

| hegohtr12 := on

| hegohtr21 := on

| hegohtr22 := on

|

| --- ELSE ---

|

(continued on next page)

11-98

EGO MONITOR CONTROL STRATEGY, EGO HEATER IO - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

HEATER_STATE = keyon_init -----------------| hegohtr11 := off

| hegohtr12 := off

| hegohtr21 := off

| hegohtr22 := off

|

| --- ELSE ---

|

HEATER_STATE = us_toggle ------------------| hegohtr11 := on

| hegohtr12 := off

| hegohtr21 := on

| hegohtr22 := off

|

| --- ELSE ---

|

HEATER_STATE = ds_toggle ------------------| hegohtr11 := on

| hegohtr12 := on

| hegohtr21 := on

| hegohtr22 := on

;

;The state cm_done is intended to allow fault flags, which get set during

;current monitoring, to clear prior to resuming operation of the normal

;heater controller.  This prevents the heater controller from jumping

;into diagnostics mode to clear the fault bits, and the KOEO test from

;detecting inadvertant fault flags left over from the current monitor.

;

;The state keyon_init begins the powerup reset sequence for the IMP IC

;to clear any erroneous IMP fault bits or heater states which may have

;latched on at powerup.  The state us_toggle turns on the upstream

;heaters first, followed by the state ds_toggle to turn on the downstream

;pair.  Upstream/downstream heater turn is staggered to avoid excessive

;surge currents during cold starts.

;

11-99

EGO MONITOR CONTROL STRATEGY, EGO HEATER IO - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;Output State Control override logic:

; Notify user if requested channel is not present for this vehicle:

SUBST_REQ25 = 1 -----|

|AND -| Do: osc_response(25,10h)

HEATER_HP11 = 0 -----|

SUBST_REQ26 = 1 -----|

|AND -| Do: osc_response(26,10h)

HEATER_HP12 = 0 -----|

SUBST_REQ27 = 1 -----|

|AND -| Do: osc_response(27,10h)

HEATER_HP21 = 0 -----|

SUBST_REQ28 = 1 -----|

|AND -| Do: osc_response(28,10h)

HEATER_HP22 = 0 -----|

OBDII_RESET <> 1 ----|     |

|     |

ER_STATUS <>         |     |

ER_INIT -----|AND -| Do: substitute(25,hegohtr11)

|     | HEGOHTR11 := hegohtr11

ER_STATUS <>         |     | Do: substitute(26,hegohtr12)

ER_DONE ------|     | HEGOHTR12 := hegohtr12

| Do: substitute(27,hegohtr21)

| HEGOHTR21 := hegohtr21

| Do: substitute(28,hegohtr22)

| HEGOHTR22 := hegohtr22

END: ego_heater_output

11-100

EGO MONITOR CONTROL STRATEGY, EGO HEATER IO - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ego_heater_fault_status

;The following process is only executed when called and is used to control

;the reading of the fault status fromthe SPI inputs for each EGO heater.

unconditionally --------------| OLDHTRFALT11 := HTR_FAULT11

| OLDHTRFALT12 := HTR_FAULT12

| OLDHTRFALT21 := HTR_FAULT21

| OLDHTRFALT22 := HTR_FAULT22

HEATER_HP11 = true -----------| HTR_FAULT11 := Fault bit from driver.

|

| --- ELSE ---

|

| HTR_FAULT11 := no_fault

HEATER_HP21 = true -----------| HTR_FAULT21 := Fault bit from driver.

|

| --- ELSE ---

|

| HTR_FAULT21 := no_fault

HEATER_HP12 = true -----------| HTR_FAULT12 := Fault bit from driver.

|

| --- ELSE ---

|

| HTR_FAULT12 := no_fault

HEATER_HP22 = true -----------| HTR_FAULT22 := Fault bit from driver.

|

| --- ELSE ---

|

| HTR_FAULT22 := no_fault

HEGOHTR11 = on ---------------| ON_FAULT11 := HTR_FAULT11

|

| --- ELSE ---

|

| OFF_FAULT11 := HTR_FAULT11

HEGOHTR21 = on ---------------| ON_FAULT21 := HTR_FAULT21

|

| --- ELSE ---

|

| OFF_FAULT21 := HTR_FAULT21

11-101

EGO MONITOR CONTROL STRATEGY, EGO HEATER IO - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

HEGOHTR12 = on ---------------| ON_FAULT12 := HTR_FAULT12

|

| --- ELSE ---

|

| OFF_FAULT12 := HTR_FAULT12

HEGOHTR22 = on ---------------| ON_FAULT22 := HTR_FAULT22

|

| --- ELSE ---

|

| OFF_FAULT22 := HTR_FAULT22

END: ego_heater_fault_status

11-102

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

11.1.6  EGO SWITCHING TEST (CDAN0)

OVERVIEW:

This  test  determines  that  the  EGO  sensor  is  switching  properly  when

conditions  are present that will allow a functional fuel system to cause the

EGO sensor to switch.

The fundamental principles of the EGO/fuel system test are;

1 - that while the EGO test conditions are true  a  functioning  fuel  system

will not violate any EGO test criteria.

2 - while the EGO test conditions are false the EGO sensor cannot be expected

to  switch, therefore all failure indicators are reset (LAMBSE and time since

last EGO switch criteria) or suspended (number of switches criterion).

Three separate failure criteria are used to determine if the EGO/fuel  system

is malfunctioning;

1 - total number of EGO switches; if the  EGO  sensor/fuel  system  is  in  a

condition  where  the EGO sensor(s) do not switch at all then, after the test

has accumulated sufficient test time (to give the sensors the opportunity  to

switch  the required number of times) and a sufficient number of switches has

not occurred, then the test will indicate a malfunction.

2 - LAMBSE at clip; if LAMBSE is at a clip  and  remains  there  for  V_LESTM

seconds  while  the  test  conditions  are  true  the  test  will  indicate a

malfunction.

3 - Time since last EGO switch; if at any time the test conditions  are  true

and the time since the last EGO switch took place exceeds V_LEGO_MAX the test

will indicate a malfunction.  When one or both sensors have failed while  the

EGO  test  was  inactive, special conditions are checked to allow testing for

time since last EGO switch while purging is taking place.  See the  procedure

"PURG_NOT_CAUSE_FOR_FAILURE".

DEFINITIONS

Registers:

-  ATMR1 = Time since exiting crank mode.

-  EGOSS11 = Number of EGO11 switches since start.

-  EGOSS21 = Number of EGO21 switches since start.

-  EXT_FEU = Inferred tip temperature of the upstream  EGO  sensor  (without

heater effects).

-  INFAMB_KAM = Inferred ambient air temperature,  used  to  initialize  the

cat.  temp.  model.

11-103

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  KAMRF1 = Total learned fuel system correction from EGO11.

-  KAMRF2 = Total learned fuel system correction from EGO21.

-  LAMBSE1 = Desired air/fuel ratio, EGO11.

-  LAMBSE2 = Desired air/fuel ratio, EGO21.

-  LEGONOTPURG = In EGO test, when equal to  1,  testing  while  purging  is

appropriate.

-  LOAD = Nondimensional, generic engine load.

-  LAMMAXn = Upper lambse clip calculated for bank n.

-  LAMMINn = Lower lambse clip calculated for bank n.

-  P1130FIL = Lack of EGO11 switch, adaptive fuel at limit fault filter.

-  P1131FIL = Lack of EGO11 switch, EGO indicates lean fault filter.

-  P1132FIL = Lack of EGO11 switch, EGO indicates rich fault filter.

-  P1150FIL = Lack of EGO21 switch, adaptive fuel at limit fault filter.

-  P1151FIL = Lack of EGO21 switch, EGO21 indicates lean fault filter.

-  P1152FIL = Lack of EGO21 switch, EGO21 indicates rich fault filter.

-  PUTMR = Time since powerup.

-  TP_REL = Relative Throttle Position, counts.  TP - RATCH.

-  V_EGOTST_TMR = Time since self test closed loop conditions have been met.

-  V_LEGOTMR1 = Self test time since last EGO11 switch while test conditions

are active.

-  V_LEGOTMR2 = Self test time since last EGO21 switch while test conditions

are active.

-  V_LESTMR1 = Self test lack of EGO11 switchng timer.

-  V_LESTMR2 = Self test lack of EGO21 switching timer.

Bit Flags:

-  AFMFLG = Flag indicating that ACT sensor is in/out of range.

-  CFMFLG = Flag indicating that ECT sensor is in/out of range.

-  CRKFLG = Flag indicating crank mode.

-  EGOFL11 = EGO11 state flag; 1 -> rich.

-  EGOFL21 = EGO21 state flag; 1 -> rich.

11-104

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  EGOTSTCUMFLG = Flag used to start and halt EGOTSTCUMTMR; 0 -> halt.

-  EGREN = Flag indicating EGR is enabled; 1 -> enabled.

-  FFG_CSD11 = Fault flag for CSD on sensor 11.

-  FFG_CSD21 = Fault flag for CSD on sensor 21.

-  ISCFLG = Indication of Idle Air Flow; 1 = Idle Air Flow.

-  LEGOFG11 = Lack of EGO11 switching.

-  LEGOFG21 = Lack of EGO21 switching.

-  MFMFLG = MAP failure flag set to 1 if MAP sensor fails.

-  OL_DESIRED = Open loop desired flag; 1 = open loop desired.

-  OPEN_EGOn1 = Flag indicating open  circuit  voltage  condition  found  on

Upstream HEGO bank n sensor.

-  PG_DC = Purge enabled flag.

-  SWTFL11 = EGO switch flag; 1 -> EGO11 switched this background loop.

-  SWTFL21 = EGO switch flag; 1 -> EGO21 switched this background loop.

-  TFMFLG = Flag indicating TP sensor is in/out of range.

-  V_LAMJMP1 = 1 -> base strategy caused a LAMBSE1  jump  since  last  EGO11

switch.

-  V_LAMJMP2 = 2 -> base strategy caused a LAMBSE2  jump  since  last  EGO21

switch.

-  V_EGO1_BYPS = Prevents additional EGO11 service code.

-  V_EGO1L_BYPS = Prevents additional EGO11 service code.

-  V_EGO2_BYPS = Prevents additional EGO21 service code.

-  V_EGO2L_BYPS = Prevents additional EGO21 service code.

-  V_EGR_DLYFG  =  EGO  test  EGR  transition  indication  flag;  1  ->  EGR

transition has occurred.

-  V_EGR_OLD = Previous background EGR state.

-  V_INJ_ER_FLG = In range injector error flag.

-  V_INRANG_FLG =  Continuous  EGO  test  intermediate  test  flag  used  by

in-range sensor tests.

-  V_MAF_ER_FLG = In range MAF error flag.

-  V_TP_ER_FLG = In range TP error flag.

Calibration Constants:

11-105

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  CSD_MIN_AMB = Minimum ambient temperature to allow storage of  CSD  fault

codes.

-  ETST_SWCUMTM =  Accumulated  time  EGO  test  active  before  failure  is

indicated because of number of switches failure criteria.

-  FN1351(N,LOAD)= Transport delay in REVS.

-  MAXADP = Maximum adaptive correction.

-  MINADP = Minimum adaptive correction.

-  NUMEGO = Number of EGO sensors.

-  PRG_DEC = Purge DC decrement amount when purge overwhelms fuel control.

-  PURGSW = When switch = 1, purging allowed during open loop fuel control.

-  P1130LVL = Lack of EGO11 switch, adaptive  fuel  at  limit  fault  filter

threshold.

-  P1130UP = Lack of EGO11 switch,  adaptive  fuel  at  limit  fault  filter

increment.

-  P1131LVL = Lack of  EGO11  switch,  EGO11  indicates  lean  fault  filter

threshold.

-  P1131UP = Lack  of  EGO11  switch,  EGO11  indicates  lean  fault  filter

increment.

-  P1132LVL = Lack of  EGO11  switch,  EGO11  indicates  rich  fault  filter

threshold.

-  P1132UP = Lack  of  EGO11  switch,  EGO11  indicates  rich  fault  filter

increment.

-  P1150LVL = Lack of EGO21 switch, adaptive  fuel  at  limit  fault  filter

threshold.

-  P1150UP = Lack of EGO21 switch,  adaptive  fuel  at  limit  fault  filter

increment.

-  P1151LVL = Lack of  EGO21  switch,  EGO21  indicates  lean  fault  filter

threshold.

-  P1151UP = Lack  of  EGO21  switch,  EGO21  indicates  lean  fault  filter

increment.

-  P1152LVL = Lack of  EGO21  switch,  EGO21  indicates  rich  fault  filter

threshold.

-  P1152UP = Lack  of  EGO21  switch,  EGO21  indicates  rich  fault  filter

increment.

-  V_EGO_ENA = Continuous EGO test enable switch; 1 -> enable.

-  V_EGOIDL_ENA = Switch to enable EGO sensor test at idle air flows;  1  ->

enable.

11-106

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  V_EGOLD_MAX = Maximum LOAD to perform EGO sensor test when not at idle.

-  V_EGOLD_MIN = Minimum LOAD to perform EGO sensor test when not at idle.

-  V_EGOTEMP = Temperature at which to enable the continuous  EGO  switching

test, degrees.

-  V_EGOTST_TM = Time since closed loop conditions minus EGO input have been

met.

-  V_LEGO_MAX  =  Maximum  time  since  last  EGO-n  switch  before  failure

indication.

-  V_LEGO_MAX2  =  Maximum  time  since  last  EGOn  switch  before  failure

indication when failure took place while EGO testing was suspended.

-  V_LESTM1 = Time since last EGO11 switch  limit  to  fail  continuous  EGO

switching test.

-  V_LESTM2 = Time since last EGO21 switch  limit  to  fail  continuous  EGO

switching test.

11-107

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EGO_CONTINUOUS_TEST_COM10

BEGIN: BG_CHECK_ENTRY_CONDITIONS

;

; Execute this process only when called from the monitor control module.

;

; This process is the entry point for the continuous lack of switching tests.

; Check global entry conditions first, and then call the lack of switching

; test window is in-range.

;

DEMAND_MODE = 0 --------|

(not in KOEO or KOER)  |

|

V_EGO_ENA = 1 ----------|AND -| Do: EGR_TRANSITION_DELAY_FLAG_CONTROL

(test cal'd in)        |     | Do: EGO_PURG_CHK

|     | Do: PURG_NOT_CAUSE_OF_FAILURE

CCM_TST_ENA = 1 --|     |     | Do: EGO_SWITCHING_FAILURE_INDICATION_CONTROL

|OR --|     | Do: ACCUM_TIMER_CONTROL

EGO_TST_ENA = 1 --|           | Do: IN_RANGE_ENTRY

|  (test conditions checked)

|

| --- ELSE ---

|

V_EGO_ENA = 1 ----------------| EGOTSTCUMTMR := 0

(test cal'd in)              | Do: EGO_TEST_TMR_CLEAR

|  (test is bypassed, clear timers

|   until test is executed)

END: BG_CHECK_ENTRY_CONDITIONS

11-108

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: IN_RANGE_ENTRY

The following test conditions plus the conditions in EGOTST_TMR_CONTROL

must be true for V_EGOTST_TM seconds before the fuel system can be tested.

The conditions in IN_RANGE_ENTRY are separated from the conditions in

EGOTST_TMR_CONTROL for use by the In-Range-Failure-Test via V_INRANG_FLG.

OL_DESIRED = 0 ---------------|

(closed loop desired)        |

|

EXT_FEU >= V_EGOWARM ---|     |

|OR --|

ATMR1 >= V_EGORNTM -----|     |

|

AFMFLG = 0 -------------------|

|AND -| V_INRANG_FLG := 1

CFMFLG = 0 -------------------|     |  (for use by in-range-failure

|     |   test)

MFMFLG = 0 -------------------|     | Do: EGOTST_TMR_CONTROL

|     | Do: EGOTST_TMR_CHK

TFMFLG = 0 -------------------|     |

(no failed sensors)          |     |

|     |

V_EGR_DLYFG = 0 --------------|     |

(no egr transitions)               |

| --- ELSE ---

|

| V_INRANG_FLG := 0

|  (for use by in-range-failure

|   test)

| Do: EGOTST_TMR_CONTROL

END: IN_RANGE_ENTRY

11-109

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: EGOTST_TMR_CONTROL

The following test conditions logic must be true for V_EGOTST_TM

seconds before the fuel system can be tested.

V_INRANG_FLG = 1 -----------------|

(above conditions met)           |

|

TP_REL=> V_EGOTP_MIN -|           |

(min TP_REL to test) |           |

|           |AND -| increment V_EGOTST_TMR

ISCFLG <= 0 ----------|           |     | EGOTSTCUMFLG := 1

(not at idle)        |AND -|     |     |  (allow EGOTSTCUMTMR to run)

|     |     |     |

LOAD > V_EGOLD_MIN ---|     |     |     |

(load indicator)     |     |     |     |

|     |OR --|     |

LOAD < V_EGOLD_MAX ---|     |           |

|           |

ISCFLG > 0 -----------|     |           |

(at idle)            |AND -|           |

|                 |

V_EGOIDL_ENA = 1 -----|                 |

(enable at idle)                       |

| --- ELSE ---

|

| Do: EGO_TEST_TMR_CLEAR

|  (test is bypassed, clear

|   timers until test is executed)

| Do: VEGOFIL_ZERO

|  (control fault filter for

|   MIL control)

| EGOTSTCUMFLG := 0

|  (EGOTSTCUMTMR halted)

END: EGOTST_TMR_CONTROL

11-110

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: EGOTST_TMR_CHK

Control of how long the test conditions must be present before testing is to

take place is accomplished with the calibration parameter V_EGOTST_TM.

V_EGOTST_TMR > V_EGOTST_TM -----|

(test conditions present       |

sufficiently long to test)    |AND -| Do: EGO11_TESTS

|     | Do: EGO11_MALF

NUMEGO = 2 ---------------------|     | Do: EGO1FMFLG_CONTROL

| Do: EGO21_TESTS

| Do: EGO21_MALF

| Do: EGO2FMFLG_CONTROL

|

| --- ELSE ---

|

V_EGOTST_TMR > V_EGOTST_TM -----------| Do: EGO11_TESTS

(test conditions present             | Do: EGO11_MALF

sufficiently long to test)          | Do: EGO1FMFLG_CONTROL

EGOTSTCUMTMR > ETST_SWCUMTM ----------| EGO_TEST_MON := 1

(sufficient cumulative test time)

END: EGOTST_TMR_CHK

BEGIN: ACCUM_TIMER_CONTROL

The timer EGOTSTCUMTMR is a cumulative timer that documents total accumulated

time that the EGO test conditions are true while there are no purging

restrictions since the last time crank mode was exitted.  If crank mode is

re-entered, this timer is cleared (see "Test Entry Conditions").

EGOTSTCUMFLG = 1 -----------------| allow EGOTSTCUMTMR to run

(test conditions true)           |

|

| --- ELSE ---

|

| freeze EGOTSTCUMTMR

END: ACCUM_TIMER_CONTROL

11-111

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: EGO_TEST_TMR_CLEAR

unconditionally ------------------------| V_LESTMR1 := 0

| V_LESTMR2 := 0

| V_EGOTST_TMR := 0

| V_LEGOTMR1 := 0

| V_LEGOTMR2 := 0

END: EGO_TEST_TMR_CLEAR

BEGIN: EGO_PURG_CHK

PG_DC = 0 ---------------------------|

(purge off)                         |

|

PRG_DEC = 0 -------------------------|OR --| ego_purg_byps := 0

(purge does not effect              |     |  (no purging restrictions

non pcomp fuel systems)            |     |   on ego test)

|     |

PCOMP_SW = 1 ------------------------|     |

(pcomp purge system)                      |

| --- ELSE ---

|

| ego_purg_byps := 1

|  (purging restrictions

|   on ego test in effect)

END: EGO_PURG_CHK

11-112

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: PURG_NOT_CAUSE_OF_FAILURE

This logic recognizes whether one or both EGO sensors has stop switching.  In

the case where one sensor stopped switching, the control system causes both

banks to be controlled by the remaining sensor.  In the case where both

sensors fail the system will go open loop.  Normally a rich failure cannot be

recognized unless purging has been removed, eliminating purge as the cause of

the apparent failure.  In the first case above, if the ego test was inactive

from the time the ego stopped switching to the time the LEGOFGn flag becomes

set a failure will have to be recognized while purging is taking place.  If

one EGO is switching properly then the inference is that purging is not

overdriving the non switching sensor.  In the second case (both sensors

failed) failure recognization can take place if purging is suspended during

open loop operation (PURGSW = 0).

The following logic recognizes the above conditions and indicates to the

failure assessment logic that one of the above situations exist.  The flag

LEGONOTPURG indicates whether or not a test for ego failure may be performed

if one of the above situations has occurred.

;

;The lesflgn1 flags are used locally to this process.

;

LEGOFG11 = 0 ---------------------|

|AND -| lesflg11 := 0

FFG_CSD11 = 0 --------------------|     |  ;switching OK

|

| --- ELSE ---

|

| lesflg11 := 1

|  ;lack of switching

LEGOFG21 = 0 ---------------------|

|AND -| lesflg21 := 0

FFG_CSD21 = 0 --------------------|     |

| --- ELSE ---

|

| lesflg21 := 1

11-113

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PCOMP_SW = 0 ------------------|

(non pcomp purge)             |

|

LEGOFG11 = 1 ------|           |

(byples timeout)  |           |

|AND -|     |

lesflg21 = 0 ------|     |     |

|     |     |

NUMEGO > 1 --------|     |     |

(two forward egos)      |     |AND -| LEGONOTPURG := 1

|     |     |  (conditions imply that purge

NUMEGO > 1 --------|     |     |     |   was not the cause of EGO11 or

|     |     |     |   EGO21 stuck rich)

lesflg11 = 0 ------|AND -|     |     |

|     |     |     |

LEGOFG21 = 1 ------|     |     |     |

(byples timeout)        |     |     |

|     |     |

NUMEGO > 1 --------|     |OR --|     |

|     |           |

LEGOFG11 = 1 ------|     |           |

|AND -|           |

LEGOFG21 = 1 ------|     |           |

|     |           |

PURGSW = 0 --------|     |           |

|           |

NUMEGO = 1 --------|     |           |

|     |           |

LEGOFG11 = 1 ------|AND -|           |

|                 |

PURGSW = 0 --------|                 |

(no purge while open loop)          |

|

| --- ELSE ---

|

| LEGONOTPURG := 0

END: PURG_NOT_CAUSE_OF_FAILURE

11-114

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN:  VEGOFIL_ZERO

Fault filters not currently above their level (PxxxFIL <= PxxxLVL)

get a fresh start each time the test conditions become true.

P1130FIL <= P1130LVL -----------------| P1130FIL := 0

P1131FIL <= P1131LVL -----------------| P1131FIL := 0

P1132FIL <= P1132LVL -----------------| P1132FIL := 0

P1150FIL <= P1150LVL -----------------| P1150FIL := 0

P1151FIL <= P1151LVL -----------------| P1151FIL := 0

P1152FIL <= P1152LVL -----------------| P1152FIL := 0

END: VEGOFIL_ZERO

11-115

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EGO11/Fuel System Tests:

Base strategy will assign LAMBSE1 the value  of  LAMBSE2  after  LESTMR11  >=

BYPLES.   Failure  codes  P1130, P1131 or P1132 as appropriate will be stored

upon failure, with the MIL being illuminated.  But as a result  of  the  Base

Strategy action, the failure condition is overwritten.  The fault filter will

down count.  The service code will remain stored in KAM, but the MIL will  be

turned off.

The following logic segment example below  will  maintain  the  fault  filter

value  after  failure  (PxxxFIL > PxxxLVL) until the sensor starts switching,

causing the malfunction to remain until the actual failure condition  is  not

present.

PxxxFIL > PxxxLVL ------------|

(failure has been indicated) |AND -|

|     |

LEGOFG11 = 1 -----------------|     |

(ego hasn't switched since         |

failure)

(EXAMPLE LOGIC SEGMENT)

11-116

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: EGO11_TESTS

;

;The lesflgn1 flags are used locally to this process.

;

LEGOFG11 = 1 ---------------------|

|OR --| lesflg11 := 1

FFG_CSD11 = 1 --------------------|     |  ;lack of switching

|

| --- ELSE ---

|

| lesflg11 := 0

|  ;switching OK

P1130FIL > P1130LVL ---------------------|

(failure has been indicated)            |AND -| P1130FIL := P1130FIL +

|     |             P1130UP

lesflg11 = 1 ----------------------------|     |

(ego hasn't switched since                    | P1131FIL := P1131FIL - 1

failure)                                     | P1132FIL := P1132FIL - 1

|

| --- ELSE ---

P1131FIL > P1131LVL ---------------------|     |

(failure has been indicated)            |AND -| P1131FIL := P1131FIL +

|     |             P1131UP

lesflg11 = 1 ----------------------------|     |

(ego hasn't switched since                    | P1130FIL := P1130FIL - 1

failure)                                     | P1132FIL := P1132FIL - 1

|

| --- ELSE ---

P1132FIL > P1132LVL ---------------------|     |

(failure has been indicated)            |AND -| P1132FIL := P1132FIL +

|     |             P1132UP

lesflg11 = 1 ----------------------------|     |

(ego hasn't switched since                    | P1130FIL := P1130FIL - 1

failure)                                     | P1131FIL := P1131FIL - 1

|

| --- ELSE ---

(continued on next page)

11-117

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

V_LESTMR1 >                                    |

max(V_LESMUL*BYPLES_TM1,V_LESTM1) -|           |

(failure indicated)              |           |

|           |

EGOSS11 < V_EGOSWNUM --------|     |           |

(fail from start)           |AND -|OR --|     |

|     |     |     |

EGOTSTCUMTMR > ETST_SWCUMTM -|     |     |     |

(sufficient cumulated test time)  |     |AND -| P1130FIL := P1130FIL +

|     |     |             P1130UP

V_LEGOTMR1 > V_LEGO_MAX -----------|     |     |

|     | P1131FIL := P1131FIL - 1

KAMRF1 = 0.5 + MINADP -------------|     |     | P1132FIL := P1132FIL - 1

(adaptive clip)                   |OR --|     |  (fuel system failure)

|           |

KAMRF1 = 0.5 + MAXADP -------------|           |

| --- ELSE ---

V_LESTMR1 >                                    |

max(V_LESMUL*BYPLES_TM1,V_LESTM1) -|           |

(failure indicated)              |           |

|           |

V_EGO1L_BYPS = 0 ------------|     |           |

|     |           |

EGOSS11 < V_EGOSWNUM --------|AND -|OR --|     |

(fail from start)          |     |     |     |

|     |     |     |

EGOTSTCUMTMR > ETST_SWCUMTM -|     |     |     |

(sufficient cumulated test time)  |     |AND -| P1131FIL := P1131FIL +

|     |     |             P1131UP

V_LEGOTMR1 > V_LEGO_MAX -----------|     |     |

|     | P1130FIL := P1130FIL - 1

EGOFL11 = 0 -----------------------------|     | P1132FIL := P1132FIL - 1

(lean)                                        |  (ego circuit failure)

| V_EGO1_BYPS := 1

|

| --- ELSE ---

(continued on next page)

11-118

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

LEGONOTPURG = 1 -------------|                 |

(purge not implicated as    |                 |

cause of failure)          |AND -|           |

|     |           |

V_LEGOTMR1 > V_LEGO_MAX2 ----|     |           |

(failure indicated)               |           |

|           |

V_LESTMR1 >                        |           |

max(V_LESMUL*BYPLES_TM1,V_LESTM1) -|           |

(failure indicated)               |           |

|           |

EGOSS11 < V_EGOSWNUM --------|     |OR --|     |

(fail from start)           |     |     |     |

|     |     |     |

V_EGO1_BYPS = 0 -------------|AND -|     |     |

|     |     |AND -| P1132FIL := P1132FIL +

EGOTSTCUMTMR > ETST_SWCUMTM -|     |     |     |             P1132UP

(sufficient cumulated test time)  |     |     |

|     |     | P1130FIL := P1130FIL - 1

V_LEGOTMR1 > V_LEGO_MAX -----------|     |     | P1131FIL := P1131FIL - 1

(gross lack of ego switch)              |     |  (ego circuit failure)

|     | V_EGO1L_BYPS := 1

EGOFL11 = 1 -----------------------------|     |

(rich)                                        |

| --- ELSE ---

|

| P1130FIL := P1130FIL - 1

| P1131FIL := P1131FIL - 1

| P1132FIL := P1132FIL - 1

|  (no failures present)

END: EGO11_TESTS

11-119

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EGO21/Fuel System Tests:

Base strategy will assign LAMBSE2 the  value  of  LAMBSE1  after  LESTMR2  >=

BYPLES.   Failure codes 1151, 1152 or 1153 as appropriate will be stored upon

failure, with the MIL being  illuminated.   But  as  a  result  of  the  Base

Strategy action, the failure condition is overwritten.  The fault filter will

down count.  The service code will remain stored in KAM, but the MIL will  be

turned off.

The following logic segment example below  will  maintain  the  fault  filter

value  after  failure  (PxxxFIL > PxxxLVL) until the sensor starts switching,

causing the malfunction to remain until the actual failure condition  is  not

present.

PxxxFIL > PxxxLVL ------------|

(failure has been indicated) |AND -|

|     |

LEGOFG21 = 1 -----------------|     |

(ego hasn't switched since         |

failure)

(EXAMPLE LOGIC SEGMENT)

11-120

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: EGO21_TESTS

;

;The lesflg21 flag is used locally to this process.

;

LEGOFG21 = 1 ---------------------|

|OR --| lesflg21 := 1

FFG_CSD21 = 1 --------------------|     |  ;lack of switching

|

| --- ELSE ---

|

| lesflg21 := 0

|  ;switching OK

P1150FIL > P1150LVL ---------------------|

(failure has been indicated)            |AND -| P1150FIL := P1150FIL +

|     |             P1150UP

lesflg21 = 1 ----------------------------|     |

(ego hasn't switched since                    | P1151FIL := P1151FIL - 1

failure)                                     | P1152FIL := P1152FIL - 1

|

| --- ELSE ---

P1151FIL > P1151LVL ---------------------|     |

(failure has been indicated)            |AND -| P1151FIL := P1151FIL +

|     |             P1151UP

lesflg21 = 1 ----------------------------|     |

(ego hasn't switched since                    | P1150FIL := P1150FIL - 1

failure)                                    | P1152FIL := P1152FIL - 1

|

| --- ELSE ---

P1152FIL > P1152LVL ---------------------|     |

(failure has been indicated)            |AND -| P1152FIL := P1152FIL +

|     |             P1152UP

lesflg21 = 1 ----------------------------|     |

(ego hasn't switched since                    | P1150FIL := P1150FIL - 1

failure)                                     | P1151FIL := P1151FIL - 1

|

| --- ELSE ---

(continued on next page)

11-121

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

V_LESTMR2 >                                    |

max(V_LESMUL*BYPLES_TM2,V_LESTM2) -|           |

(failure indicated)              |           |

|           |

EGOSS21 < V_EGOSWNUM --------|     |           |

(fail from start)           |AND -|OR --|     |

|     |     |     |

EGOTSTCUMTMR > ETST_SWCUMTM -|     |     |     |

(sufficient cumulated test time)  |     |     |

|     |AND -| P1150FIL := P1150FIL +

V_LEGOTMR2 > V_LEGO_MAX -----------|     |     |             P1150UP

|     |

KAMRF2 = 0.5 + MINADP -------------|     |     | P1151FIL := P1151FIL - 1

(adaptive clip)                   |OR --|     | P1152FIL := P1152FIL - 1

|           | (fuel system failure)

KAMRF2 = 0.5 + MAXADP -------------|           |

|  --- ELSE ---

V_LESTMR2 >                                    |

max(V_LESMUL*BYPLES_TM2,V_LESTM2) -|           |

(failure indicated)              |           |

|           |

V_EGO2L_BYPS = 0 ------------|     |           |

|     |OR --|     |

EGOSS21 < V_EGOSWNUM --------|AND -|     |     |

(fail from start)          |     |     |     |

|     |     |     |

EGOTSTCUMTMR > ETST_SWCUMTM -|     |     |AND -| P1151FIL := P1151FIL +

(sufficient cumulated test time)  |     |     |             P1151UP

|     |     |

V_LEGOTMR2 > V_LEGO_MAX -----------|     |     | P1150FIL := P1150FIL - 1

|     | P1152FIL := P1152FIL - 1

EGOFL21 = 0 -----------------------------|     |  (ego circuit failure)

(lean)                                        | V_EGO2_BYPS := 1

|

| --- ELSE ---

(continued on next page)

11-122

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

LEGONOTPURG = 1 -------------|                 |

(purge not implicated       |                 |

as cause of failure)       |AND -|           |

|     |           |

V_LEGOTMR2 > V_LEGO_MAX2 ----|     |           |

(failure indicated)               |           |

|           |

V_LESTMR2 >                        |           |

max(V_LESMUL*BYPLES_TM2,V_LESTM2) -|           |

(lambse at clip)                  |           |

|           |

EGOSS21 < V_EGOSWNUM --------|     |           |

(fail from start)           |     |           |

|AND -|OR --|     |

V_EGO2_BYPS = 0 -------------|     |     |     |

|     |     |     |

EGOTSTCUMTMR > ETST_SWCUMTM -|     |     |     |

(sufficient cumulated test time)  |     |AND -| P1152FIL := P1152FIL +

|     |     |             P1152UP

V_LEGOTMR2 > V_LEGO_MAX -----------|     |     |

(gross lack of ego switch)              |     | P1150FIL := P1150FIL - 1

|     | P1151FIL := P1151FIL - 1

EGOFL21 = 1 -----------------------------|     |  (ego circuit failure)

(rich)                                        | V_EGO2L_BYPS := 1

|

| --- ELSE ---

|

| P1150FIL := P1150FIL - 1

| P1151FIL := P1151FIL - 1

| P1152FIL := P1152FIL - 1

|  (no failures present)

END: EGO21_TESTS

11-123

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: EGR_TRANSITION_DELAY_FLAG_CONTROL

The EGR transition flag V_EGR_DLYFG when set indicates that an EGR transition

has taken place.  To allow the EGO test to disregard EGR transitions,

calibrate V_EGO_EGR_SW = 0.  Otherwise any EGR transition (on/off, off/on)

will reset the EGO test timer and associated EGO fuel system failure

indicators.

EGRDC = 0 ------------------------------| temp := 1

(egr off)                              |

| --- ELSE ---

|

| temp := 0

V_EGR_OLD <> temp ----------------|

(egr transition)                 |AND -| V_EGR_DLYFG := 1

|     |  (ego test reset due to

V_EGO_EGR_SW = 1 -----------------|     |   egr transition)

(egr considered to                     | V_EGR_OLD := temp

affect ego)                           |

| --- ELSE ---

|

| V_EGR_DLYFG := 0

(egr not affecting ego test)

END: EGR_TRANSITION_DELAY_FLAG_CONTROL

11-124

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: EGO_SWITCHING_FAILURE_INDICATION_CONTROL

This procedure serves two functions: 1) allows  timers (V_LESTMR1,

V_LESTMR2) to run indicating the time LAMBSE1 or LAMBSE2 has

been at its clip.  2) maintain rolling averages of transport

delay times and EGO switching times.

VIP Lack of EGO11 Switching Timer:

LAMBSE1 <= LAMMIN1 -------------|

(rich clip, lean system)       |

|OR --| (V_LESTMR1 runs indicating

LAMBSE1 >= LAMMAX1 -------|     |     |  potential lambse/ego based

(lean clip, rich system) |AND -|     |  failure)

|           |

ego_purg_byps = 0 --------|           |

(no purging restrictions)            |

| --- ELSE ---

|

| V_LESTMR1 := 0

| (no lambse/ego based

|  errors indicated)

Vip Lack of EGO21 Switching Timer:

NUMEGO = 2 ---------------------------|

|

LAMBSE2 <= LAMMIN2 -------------|     |AND -| (V_LESTMR2 runs indicating

(rich clip, lean system)       |     |     |  potential lambse/ego based

|OR --|     |  failure)

LAMBSE2 >= LAMMAX2 -------|     |           |

(lean clip, rich system) |AND -|           |

|                 |

ego_purg_byps = 0 --------|                 |

(no purging restrictins)                   |

| --- ELSE ---

|

| V_LESTMR2 := 0

|  (no lambse/ego based

|    errors indicated)

11-125

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Time Since Last EGO11 Switch Logic:

SWTFL11 = 1 ------------------------| V_LEGOTMR1 := 0

(ego11 switched this BG)           |

| --- ELSE ---

V_LAMJMP1 = 1 ----------------|     |

(LAMBSE1 jump occurred)      |     |

|OR --| V_LEGOTMR1 := 0

ego_purg_byps = 1 ------|     |     | V_LAMJMP1 := 0

(purging is affecting  |     |     |  (reinitialization due to

ego performance)      |     |     |    LAMBSE1 jump)

|AND -|

EGOFL11 = 1 ------------|

(rich)                 |

|

LEGONOTPURG = 0 --------|

(purge implicated as a

cause of failure)

Time Since Last EGO21 Switch Logic:

NUMEGO = 2 -------------------------|

|AND -| V_LEGOTMR2 := 0

SWTFL21 = 1 ------------------------|     |

(ego21 switched this BG)                 |

| --- ELSE ---

NUMEGO = 2 -------------------------|     |

|     |

V_LAMJMP2 = 1 ----------------|     |AND -| V_LEGOTMR2 := 0

(LAMBSE2 jump occurred)      |     |     | V_LAMJMP2 := 0

|OR --|     |  (reinitialization due to

ego_purg_byps = 1 ------|     |           |    LAMBSE2 jump)

(purging is affecting  |     |

ego performance)      |     |

|AND -|

EGOFL21 = 1 ------------|

(rich)                 |

|

LEGONOTPURG = 0 --------|

(purge implicated as a

cause of failure)

END: EGO_SWITCHING_FAILURE_INDICATION_CONTROL

11-126

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: EGO11_MALF

(process to interface to the diagnostic executive)

OPEN_EGO11 = 1 ---------------|

|AND -|  Do: malfunction(ego, P0130)

P1130FIL > P1130LVL ----------|     |   ;open circuit fault.

|

| --- ELSE ---

FFG_CSD11 = 1 ----------------|     |

|     |

INFAMB_KAM > CSD_MIN_AMB -----|AND -|  Do: malfunction(ego, P0131)

|     |    ;CSD fault identified.

P1130FIL > P1130LVL ----------|     |

|

| --- ELSE ---

INFAMB_KAM > LES_MIN_AMB -----|     |

|     |

P1130FIL > P1130LVL ----------|AND -| Do: malfunction(cl, P1130)

|     |

FFG_CSD11 = 0 ----------------|     |

| --- ELSE ---

|

| Do: clear_malf(P1130)

OPEN_EGO11 = 1 ---------------|

|AND -|  Do: malfunction(ego, P0130)

P1131FIL > P1131LVL ----------|     |   ;open circuit fault.

|

| --- ELSE ---

FFG_CSD11 = 1 ----------------|     |

|     |

INFAMB_KAM > CSD_MIN_AMB -----|AND -|  Do: malfunction(ego, P0131)

|     |    ;CSD fault identified.

P1131FIL > P1131LVL ----------|     |

|

| --- ELSE ---

INFAMB_KAM > LES_MIN_AMB -----|     |

|     |

P1131FIL > P1131LVL ----------|AND -| Do: malfunction(cl, P1131)

|     |

FFG_CSD11 = 0 ----------------|     |

| --- ELSE ---

|

| Do: clear_malf(P1131)

11-127

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OPEN_EGO11 = 1 ---------------|

|AND -|  Do: malfunction(ego, P0130)

P1132FIL > P1132LVL ----------|     |   ;open circuit fault.

|

| --- ELSE ---

FFG_CSD11 = 1 ----------------|     |

|     |

INFAMB_KAM > CSD_MIN_AMB -----|AND -|  Do: malfunction(ego, P0131)

|     |    ;CSD fault identified.

P1132FIL > P1132LVL ----------|     |

|

| --- ELSE ---

INFAMB_KAM > LES_MIN_AMB -----|     |

|     |

P1132FIL > P1132LVL ----------|AND -| Do: malfunction(cl, P1132)

|     |

FFG_CSD11 = 0 ----------------|     |

| --- ELSE ---

|

| Do: clear_malf(P1132)

OPEN_EGO11 = 0 ---------------------| Do: clear_malf(P0130)

FFG_CSD11 = 0 ----------------------| Do: clear_malf(P0131)

END: EGO11_MALF

11-128

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: EGO21_MALF

(process to interface to the diagnostic executive)

OPEN_EGO21 = 1 ---------------|

|AND -|  Do: malfunction(ego, P0150)

P1150FIL > P1150LVL ----------|     |   ;open circuit fault.

|

| --- ELSE ---

FFG_CSD21 = 1 ----------------|     |

|     |

INFAMB_KAM > CSD_MIN_AMB -----|AND -|  Do: malfunction(ego, P0151)

|     |    ;CSD fault identified.

P1150FIL > P1150LVL ----------|     |

|

| --- ELSE ---

INFAMB_KAM > LES_MIN_AMB -----|     |

|     |

P1150FIL > P1150LVL ----------|AND -| Do: malfunction(cl, P1150)

|     |

FFG_CSD21 = 0 ----------------|     |

| --- ELSE ---

|

| Do: clear_malf(P1150)

OPEN_EGO21 = 1 ---------------|

|AND -|  Do: malfunction(ego, P0150)

P1151FIL > P1151LVL ----------|     |   ;open circuit fault.

|

| --- ELSE ---

FFG_CSD21 = 1 ----------------|     |

|     |

INFAMB_KAM > CSD_MIN_AMB -----|AND -|  Do: malfunction(ego, P0151)

|     |    ;CSD fault identified.

P1151FIL > P1151LVL ----------|     |

|

| --- ELSE ---

INFAMB_KAM > LES_MIN_AMB -----|     |

|     |

P1151FIL > P1151LVL ----------|AND -| Do: malfunction(cl, P1151)

|     |

FFG_CSD21 = 0 ----------------|     |

| --- ELSE ---

|

| Do: clear_malf(P1151)

11-129

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OPEN_EGO21 = 1 ---------------|

|AND -|  Do: malfunction(ego, P0150)

P1152FIL > P1152LVL ----------|     |   ;open circuit fault.

|

| --- ELSE ---

FFG_CSD21 = 1 ----------------|     |

|     |

INFAMB_KAM > CSD_MIN_AMB -----|AND -|  Do: malfunction(ego, P0151)

|     |    ;CSD fault identified.

P1152FIL > P1152LVL ----------|     |

|

| --- ELSE ---

INFAMB_KAM > LES_MIN_AMB -----|     |

|     |

P1152FIL > P1152LVL ----------|AND -| Do: malfunction(cl, P1152)

|     |

FFG_CSD21 = 0 ----------------|     |

| --- ELSE ---

|

| Do: clear_malf(P1152)

OPEN_EGO21 = 0 ---------------------| Do: clear_malf(P0150)

FFG_CSD21 = 0 ----------------------| Do: clear_malf(P0151)

END: EGO21_MALF

11-130

CONTINUOUS SELF TEST, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: EGO1FMFLG_CONTROL

(The following process determines the state of the EGO1FMFLG flag.)

P1130FIL > P1130LVL ---------------|

(lack of HEGO11 sw,adapt at clip) |

|

P1131FIL > P1131LVL ---------------|OR --| EGO1FMFLG := 1

(lack of HEGO11 sw, lean)         |     | EGO_MON11 := 1

|     |

P1132FIL > P1132LVL ---------------|     |

(lack of HEGO11 sw, rich)               |

| --- ELSE ---

P1130FIL < P1130LVL - FILHYS ------|     |

|     |

P1131FIL < P1131LVL - FILHYS ------|AND -| EGO1FMFLG := 0

|

P1132FIL < P1132LVL - FILHYS ------|

END: EGO1FMFLG_CONTROL

BEGIN: EGO2FMFLG_CONTROL

(The following process determines the state of the EGO2FMFLG flag.)

P1150FIL > P1150LVL ---------------|

(lack of HEGO21 sw,               |

adapt at clip)                   |

|

P1151FIL > P1151LVL ---------------|OR --| EGO2FMFLG := 1

(lack of HEGO21 sw, lean)         |     | EGO_MON21 := 1

|     |

P1152FIL > P1152LVL ---------------|     |

(lack of HEGO21 sw, rich)               |

| --- ELSE ---

P1150FIL < P1150LVL - FILHYS ------|     |

|     |

P1151FIL < P1151LVL - FILHYS ------|AND -| EGO2FMFLG := 0

|

P1152FIL < P1152LVL - FILHYS ------|

END: EGO2FMFLG_CONTROL

11-131

EGO MONITOR CONTROL STRATEGY, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

11.1.7  EGO SWITCHING TEST (CDAM0)

OVERVIEW

+----------------------------------+

| EGO_ER                           |

+----------------------------------+

|PUBLIC PROCEDURES:                |

|. definitions                     |

|. ego_er_switching_test           |

+----------------------------------+

|PRIVATE PROCEDURES:               |

|. abort_ego_er                    |

|. filter_ego_voltages             |

|. bank1_lean_bank2_rich           |

|. bank1_rich_bank2_lean           |

|. bank1_rich_bank2_rich           |

|. bank1_lean_bank2_lean           |

|. report_codes                    |

+----------------------------------+

This module is used in  an  On-Demand  mode  (engine  running)  to  test  the

upstream  and  downstream  EGO sensors for a single bank, dual bank or y-PIPE

system.  The test will ramp fuel lean in one bank and rich in the  other  and

monitor  the  response  of  the EGO sensors.  The banks are then reversed and

ramped in the opposite direction and again  the  response  of  the  EGOs  are

recorded.

At the end of the test, the recorded EGO responses  are  compared  and  fault

codes  are  stored  that  can  identify  crossed  upstream  sensors,  crossed

downstream sensors or faulty sensors.

During KOER self test, exhaust temperature is used to turn on the  downstream

EGO  heaters and ER_HEAT_TIME seconds from entry to the EGO switching test is

used to allow testing of all the sensors.   When  EXT_REG  >  DS_HTR_ON,  the

downstream  heaters  will  be  turned on and the heater monitor voltage check

will be performed on the heater pair.  The EGO_MONx2 flags will be  set  upon

completion of the voltage check, and the DS_HTR_TMR timer will begin counting

up.  The time delay ER_HEAT_TIME is used to allow the downstream Hego sensors

time to warm up after heater turn on.

For systems with the current monitor installed, this time delay can  also  be

calibrated to allow time for the current monitor to run.  The current monitor

will wait CM_HEAT_TM seconds before starting.  Calibrating ER_HEAT_TM greater

than this value will allow the current monitor to complete.

11-132

EGO MONITOR CONTROL STRATEGY, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

If the downstream heaters fail, ER_HEAT_TIME is not used and  the  downstream

EGO  sensors  will  not  be  tested.  The powertrain will be too cold in this

situation to reliably test the downstream EGO sensors.

|<-EXT_REG  > DS_HTR_ON ->|<- ER_HEAT_TIME ->|

+---+-------------------------+------------------+---------------->time

|   |                         |                  |

|   |                         |                  +-- Allow Test

|   |                         |                      of sensors.

|   |                         |

|   |                         +-- Turn on downstream heaters.

|   +-- Entry to run mode.

+-- Power-up.

11-133

EGO MONITOR CONTROL STRATEGY, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  BG_TMR = Background loop timer.

-  DEMAND_STATE = Utility state register  used  during  on-demand  sequenced

tests.

-  DEMAND_TIMER = Utility timer for sequenced on-demand tests.

-  DS_HTR_TMR = Elapsed time downstream EGO heaters have been on.

-  ER_FUL_REQ = Utility  flag  udes  during  on-demand  sequenced  tests  to

require LAMBSE override.

-  ER_LAM_DSD = Utility register used during on-demand  sequenced  tests  to

provide LAMBSE override value.

-  ER_STATUS = State pointer that incicated current state of engine  running

on demand test.

-  LAMBSE1 = LAMBDA equivalence ratio (EGO-1).

-  VEGO11 = Bank1 upstream HEGO voltage.

-  VEGO12 = Bank1 downstream HEGO voltage.

-  VEGO21 = Bank2 upstream HEGO voltage.

-  VEGO22 = Bank2 downstream HEGO voltage.

-  VEGOBAR11 = Filtered EGO11 voltage.

-  VEGOBAR12 = Filtered EGO12 voltage.

-  VEGOBAR21 = Filtered EGO21 voltage.

-  VEGOBAR22 = Filtered EGO22 voltage.

Bit Flags:

-  DEMAND_ABORT = Flag indicating that an on demand test is in the  progress

of a abort.

-  DS_SWAPPED_1 = Flag  indicating  potential  for  swapped  downstream  EGO

sensors exists.

-  DS_SWAPPED_2 = Flag  indicating  potential  for  swapped  downstream  EGO

sensors exists.

-  DS_TST_OK =  Flag  indicating  it  is  o.k.   to  perform  KOER  test  of

downstream EGO sensors.

-  FLG_11_LN = Flag that indicates when the EGO11 has been lean; 1 ->  EGO11

has been lean.

11-134

EGO MONITOR CONTROL STRATEGY, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FLG_11_RH = Flag that indicates when the EGO11 has been rich; 1 ->  EGO11

has been rich.

-  FLG_12_LN = Flag that indicates when the EGO12 has been lean; 1 ->  EGO12

has been lean.

-  FLG_12_RH = Flag that indicates when the EGO12 has been rich; 1 ->  EGO12

has been rich.

-  FLG_21_LN = Flag that indicates when the EGO21 has been lean; 1 ->  EGO21

has been lean.

-  FLG_21_RH = Flag that indicates when the EGO21 has been rich; 1 ->  EGO21

has been rich.

-  FLG_22_LN = Flag that indicates when the EGO22 has been lean; 1 ->  EGO22

has been lean.

-  FLG_22_RH = Flag that indicates when the EGO22 has been rich; 1 ->  EGO22

has been rich.

-  FLG_EGO_ER = Flag that indicates when the engine  running  EGO  switching

test is running; 1 -> test running.

-  HTR_MON11 = Flag indicating when the heater has been monitored, unitless.

-  HTR_MON12 = Flag indicating when the heater has been monitored, unitless.

-  HTR_MON21 = Flag indicating when the heater has been monitored, unitless.

-  HTR_MON22 = Flag indicating when the heater has been monitored, unitless.

-  PxxxMALF = Malfunction flag for code Pxxx; 1 -> a  malfunction  currently

exists for fault Pxxx.

-  US_SWAPPED_1 = Flag indicating potential for swapped upstream EGO sensors

exists.

-  US_SWAPPED_2 = Flag indicating potential for swapped upstream EGO sensors

exists.

-  US_TST_OK = Flag indicating it is ok to perform KOER test of upstream EGO

sensors.

Calibration Constants:

-  DS_HTR_ON = Temperature at which  to  turn  on  downstream  ego  heaters,

degrees F.

-  DS_SWTPOINT = Downstream HEGO switch point.

-  EGO_ER_DONE = Value for state pointer when engine  running  EGO  test  is

complete.

-  EGO_ER_INIT = Value for state of ER_STATUS to indicate engine running EGO

test is to be run.

11-135

EGO MONITOR CONTROL STRATEGY, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  EGO_KOER_SW = Switch to enable/disable KOER EGO test.

-  EGO_KOER_TM1 = Time to ramp bank lean and  bank2  rich  during  KOER  EGO

test.

-  EGO_KOER_TM2 = Time to ramp bank2 lean during the KOER EGO switch test.

-  EGO_KOER_TM3 = Time to ramp bank1 rich and bank2  rich  during  KOER  EGO

tset of y-pipe vehicle.

-  EGO_KOER_TM4 = Time to ramp bank1 lean and bank2  lean  during  KOER  EGO

test of y-pipe vehicles.

-  ER_HEAT_TIME = Time downstream EGO heaters must be on prior to  KOER  EGO

switch test.

-  HEGO_CONFIG = HEGO configuration register.

-  LEQV = Lean limit for LAMBDA.

-  NUMEGO = Number of EGO sensors.

-  REQV = Rich limit for LAMBDA.

-  US_SWTPOINT = Upstream HEGO switch point.

-  V_EGO_ENA = Alternate Continuous VIP EGO test enable switch; 1 -> enable.

-  VIPLR1 = Rate to ramp lean, LAMBDAS/sec.

-  VTCEGO = EGOBAR time constant.

OTHER

-  ego_on_demand_codes = set of {P0141, P0161, P1131, P1132,  P1137,  P1138,

P1151,  P1152,  P1157, P1158}.  The set of OBD-II fault codes that relate

to the ego switching test.

-  P0141 = Fault code for EGO12 sensor heater circuit malfunction.

-  P0161 = Fault code for EGO22 sensor heater circuit malfunction.

-  P1127 = Exhaust system not warm, downstream EGO sesnors not tested during

KOER.

-  P1131 = Lack of HO2S switch, HO2S11 indicates lean.

-  P1132 = Lack of HO2S switch, HO2S11 indicates rich.

-  P1137 = Lack of HO2S switch, HO2S12 indicates lean.

-  P1138 = Lack of HO2S switch, HO2S12 indicates rich.

-  P1151 = Lack of HO2S switch, HO2S21 indicates lean

11-136

EGO MONITOR CONTROL STRATEGY, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  P1152 = Lack of HO2S switch, HO2S21 indicates rich.

-  P1157 = Lack of HO2S switch, HO2S22 indicates lean.

-  P1158 = Lack of HO2S switch, HO2S22 indicates rich.

11-137

EGO MONITOR CONTROL STRATEGY, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EGO_ER_COM10

BEGIN: definitions

;The following block is used to define locally used symbols.  The logic is

;not intended to be specifically executed, it only exists to make the

;documentation more readable.

unconditionally -----------------------| true := 1

| false := 0

| init_ego_er := 1

| us_test_config := 2

| ds_test_config := 3

| begin_test := 4

| b1_lean_b2_rich := 5

| b1_rich_b2_lean := 6

| b1_rich_b2_rich := 7

| b1_lean_b2_lean := 8

| test_complete := 0

| no_aft_hego := 0

| single_bank := 1

| dual_bank := 2

| y_pipe := 3

END: definitions

BEGIN: ego_er_switching_test

;The following logic controls entry to and exit from the engine running

;on-demand ego switching test.  It is executed once per background pass.

ER_STATUS = EGO_ER_INIT ---------|

|AND -| ER_STATUS := EGO_ER_DONE

EGO_KOER_SW = false -------------|     |  ;test cal'd out

| Exit this process.

|

| --- ELSE ---

ER_STATUS = EGO_ER_INIT ---------|     |

;Time to perform test.          |     |

|AND -| DEMAND_STATE := init_ego_er

DEMAND_ABORT = false ------------|     | FLG_EGO_ER := true

|     |  ;prepare to start the test.

FLG_EGO_ER = false --------------|     |

| --- ELSE ---

FLG_EGO_ER = true ---------------|     |

;Test aborted.                  |AND -| Do: abort_ego_er

|     | Exit this process.

DEMAND_ABORT = true -------------|     |

| --- ELSE ---

FLG_EGO_ER = false --------------|     |

|OR --| Exit this process.

ER_STATUS <> EGO_ER_INIT --------|

11-138

EGO MONITOR CONTROL STRATEGY, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;the following logic is used on entry to the KOER test

;in order to perform specific initialization actions

;necessary to perfrom the KOER EGO switching test.

DEMAND_STATE = init_ego_er ------------| ER_LAM_DSD1 := LAMBSE1

;Test needs to be initialized         | ER_LAM_DSD2 := LAMBSE2

| ER_FUL_REQ := true

| FLG_11_RH := false

| FLG_12_RH := false

| FLG_21_RH := false

| FLG_22_RH := false

| FLG_11_LN := false

| FLG_12_LN := false

| FLG_21_LN := false

| FLG_22_LN := false

| US_SWAPPED_1 := false

| US_SWAPPED_2 := false

| DS_SWAPPED_1 := false

| DS_SWAPPED_2 := false

| US_TST_OK := false

| DS_TST_OK := false

| DEMAND_STATE := us_test_config

;First determine if it is o.k. to perform both the upstream and

;downstream tests.  It is only o.k. if the heaters have been

;monitored and there are no heater malfunctions.  For the downstream

;sensors, the heaters also have to be on in order to perform the

;tests.

DEMAND_STATE = us_test_config ----|

|

HTR_MON11 = true -----------------|

|

HTR_MON21 = true -----------------|AND -| US_TST_OK := true

|     | DEMAND_STATE := ds_test_config

P0135MALF = false ----------------|     |

|     |

P0155MALF = false ----------------|     |

| --- ELSE ---

|

DEMAND_STATE = us_test_config ----------| US_TST_OK := false

| DEMAND_STATE := ds_test_config

11-139

EGO MONITOR CONTROL STRATEGY, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DS_KOER_SW = true ----------------|

|

DEMAND_STATE = ds_test_config ----|

|

HTR_MON12 = true -----------------|

|AND -| DS_TST_OK := true

HTR_MON22 = true -----------------|     |

|     |

P0141MALF = false ----------------|     |

|     |

P0161MALF = false ----------------|     |

| --- ELSE ---

|

DEMAND_STATE = ds_test_config ----------| DS_TST_OK := false

;If any of the EGO sensors heaters won't be coming on during this

;test, store a code indicating the exhaust was too cold to test

;at least one sensor.

DEMAND_STATE = ds_test_config ----|

|AND -| Do: store_code(P1127)

US_TST_OK = false ----------|     |     | DEMAND_STATE := begin_test

|OR --|     |

DS_TST_OK = false ----------|           |

| --- ELSE ---

|

DEMAND_STATE = ds_test_config ----------| DEMAND_STATE := begin_test

11-140

EGO MONITOR CONTROL STRATEGY, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;The following if-then-else block is used as the main controller

;for the KOER procedure to test the EGO sensors.  After an

;initial warm-up fuel is ramped lean/rich then rich/lean, then if

;the vehicle is a y-pipe configuration, rich/rich and lean/lean.

;Following this, the data recorded during the fuel ramps will

;be analyzed and fault codes reported to the diagnostic executive.

DEMAND_STATE = begin_test --------|

|

DS_HTR_TMR < ER_HEAT_TIME --------|AND -| DEMAND_STATE := begin_test

|     |   ;wait for warm-up.

DS_TST_OK = true -----------------|     |

| --- ELSE ---

DEMAND_STATE = begin_test --------|     |

|AND -| DEMAND_STATE := b1_lean_b2_rich

US_TST_OK = true -----------|     |     | DEMAND_TIMER := 0

|OR --|     | VEGOBAR11 := VEGO11

DS_TST_OK = true -----------|           | VEGOBAR12 := VEGO12

| VEGOBAR21 := VEGO21

| VEGOBAR22 := VEGO22

| Do: filter_ego_voltages

| Do: bank1_lean_bank2_rich

|

| --- ELSE ---

|

DEMAND_STATE = b1_lean_b2_rich ---------| Do: filter_ego_voltages

| Do: bank1_lean_bank2_rich

|

| --- ELSE ---

|

DEMAND_STATE = b1_rich_b2_lean ---------| Do: filter_ego_voltages

| Do: bank1_rich_bank2_lean

|

| --- ELSE ---

HEGO_CONFIG = y_pipe -------------|     |

|AND -| Do: filter_ego_voltages

DEMAND_STATE = b1_rich_b2_rich ---|     | Do: bank1_rich_bank2_rich

|

| --- ELSE ---

|

DEMAND_STATE = b1_rich_b2_rich ---------| DEMAND_STATE := test_complete

|

| --- ELSE ---

|

DEMAND_STATE = b1_lean_b2_lean ---------| Do: filter_ego_voltages

| Do: bank1_lean_bank2_lean

|

| --- ELSE ---

|

DEMAND_STATE = test_complete -----------| Do: report_codes

| Do: abort_ego_er

|

| --- ELSE ---

|

| Do: abort_ego_er

END: ego_er_switching_test

11-141

EGO MONITOR CONTROL STRATEGY, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: abort_ego_er

;This process, called on-demand from other EGO switching test processes,

;is used to put the control system back in the state it was found in.

;It will signal back to the diagnostic sequencer that the EGO engine

;running test is done.

unconditionally -----------------------| DEMAND_STATE := test_complete

| DEMAND_TIMER := 0

| ER_FUL_REQ := false

| FLG_EGO_ER := false

| ER_STATUS := EGO_ER_DONE

END: abort_ego_er

BEGIN: filter_ego_voltages

;This process is used to compute filtered EGO voltages.  It is executed

;on-demand from the ego_er_switching_test process.

unconditionally -----------------------| VEGOBAR11 := rolav(VEGO11, VTCEGO)

HEGO_CONFIG <> no_aft_hego ------------| VEGOBAR12 := rolav(VEGO12, VTCEGO)

NUMEGO = 2 ----------------------------| VEGOBAR21 := rolav(VEGO21, VTCEGO)

HEGO_CONFIG = dual_bank ---------------| VEGOBAR22 := rolav(VEGO22, VTCEGO)

END: filter_ego_voltages

11-142

EGO MONITOR CONTROL STRATEGY, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: bank1_lean_bank2_rich

;The following process, executed on-demand form the ego_er_switching_test

;is used to ramp fuel in bank 1 lean and the fuel in bank 2 rich.

;Seperate bank control is necessary to detect crossed sensors.

unconditionally ------------------------| ER_LAM_DSD1 := min(ER_LAM_DSD1 +

|            (VIPLR1 * BG_TMR), LEQV)

| ER_LAM_DSD2 := max(ER_LAM_DSD2 -

|            (VIPRR1 * BG_TMR), REQV)

| ER_FUL_REQ := true

;Compare the voltage upstream of the catalyst on the EGO sensors

;to the switchpoint.  If bank 1 comes lean and bank 2 comes

;rich, everything is o.k.  If bank 1 goes/stays rich AND

;bank 2 goes/stays lean at the end of this phase of the test

;then the potential exists for swapped sensors and a flag

;will be set indicating such.

;Wait until the time limit for this phase (as set by EGO_KOER_TM1)

;has expired before sampling voltages.  This enables HEGO

;voltages to settle from pre-existing levels.

:

;Set the phase I "test o.k." flags if any of the following

;conditions are satisfied:

;  1) Not testing

;  2) Hardware not present

;  3) DEMAND_TIMER >= EGO_KOER_TM1, and VEGO at expected

;     lean/rich levels, respectively.

DEMAND_TIMER >= EGO_KOER_TM1 -----|

|AND -|

VEGOBAR11 < US_SWTPOINT ----------|     |OR --| FLG_11_LN := true

|

US_TST_OK = false ----------------------|

DEMAND_TIMER >= EGO_KOER_TM1 -----|

|AND -|

VEGOBAR21 > US_SWTPOINT ----------|     |

|OR --| FLG_21_RH := true

NUMEGO = 1 -----------------------------|

|

US_TST_OK = false ----------------------|

NUMEGO = 2 -----------------------|

|

VEGOBAR11 > US_SWTPOINT ----------|

|AND -| US_SWAPPED_1 := true

VEGOBAR21 < US_SWTPOINT ----------|     |

|     |

US_TST_OK = true -----------------|     |

| --- ELSE ---

|

| US_SWAPPED_1 := false

11-143

EGO MONITOR CONTROL STRATEGY, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;Compare the voltage downstream of the catalyst on the EGO sensors

;to the switchpoint.  If bank 1 comes lean and bank 2 comes

;rich, everything is o.k.  If bank 1 goes/stays rich AND

;bank 2 goes/stays lean at the end of this phase of the test

;then the potential exists for swapped sensors and a flag

;will be set indicating such.  The option exists to disable

;the downstream test by clearing the DS_KOER_SW.

DEMAND_TIMER >= EGO_KOER_TM1 -----|

|AND -|

VEGOBAR12 < DS_SWTPOINT ----------|     |

|

DS_TST_OK = false ----------------------|OR --| FLG_12_LN := true

|

HEGO_CONFIG = no_aft_hego --------------|

|

HEGO_CONFIG = y_pipe -------------------|

DEMAND_TIMER >= EGO_KOER_TM1 -----|

|AND -|

VEGOBAR22 > DS_SWTPOINT ----------|     |

|OR --| FLG_22_RH := true

DS_TST_OK = false ----------------------|

|

HEGO_CONFIG <> dual_bank ---------------|

HEGO_CONFIG = dual_bank ----------|

|

VEGOBAR12 > DS_SWTPOINT ----------|

|AND -| DS_SWAPPED_1 := true

VEGOBAR22 < DS_SWTPOINT ----------|     |

|     |

DS_TST_OK = true -----------------|     |

| --- ELSE ---

|

| DS_SWAPPED_1 := false

FLG_11_LN = true -----------|

|

FLG_12_LN = true -----------|

|AND -|

FLG_21_RH = true -----------|     |

|     |OR --| DEMAND_STATE := b1_rich_b2_lean

FLG_22_RH = true -----------|     |     | DEMAND_TIMER := 0

|

DEMAND_TIMER >= EGO_KOER_TM1 -----|

END: bank1_lean_bank2_rich

11-144

EGO MONITOR CONTROL STRATEGY, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: bank1_rich_bank2_lean

;The following process, executed on-demand form the ego_er_switching_test

;is used to ramp fuel in bank 2 lean and the fuel in bank 1 rich.

;Seperate bank control is necessary to detect crossed sensors.

unconditionally ------------------------| ER_LAM_DSD2 := min(ER_LAM_DSD2 +

|            (VIPLR1 * BG_TMR), LEQV)

| ER_LAM_DSD1 := max(ER_LAM_DSD1 -

|            (VIPRR1 * BG_TMR), REQV

| ER_FUL_REQ := true

;Compare the voltage upstream of the catalyst onthe EGO sensors

;to the switchpoint.  If bank 2 comes lean and bank 1 comes

;rich, everything is o.k.  If bank 2 goes/stays rich AND

;bank 1 goes/stays lean at the end of this phase of the test

;then the potential exists for swapped sensors and a flag

;will be set indicating such.

;If phase 1 detected swapped HEGO's, wait until the time limit

;for this phase (as set by EGO_KOER_TM2) has expired before

;sampling voltages.  This enables HEGO voltages to settle from

;levels of the previous phase.

;

;Set the phase II "test o.k." flags if any of the following

;conditions are satisfied:

;  1) Not testing

;  2) Hardware not present

;  3) Phase I swapped, DEMAND_TIMER >= EGO_KOER_TM2, and VEGO at

;     expected lean/rich levels

;  4) Phase I not swapped, and VEGO switches to expected

;     lean/rich levels

US_SWAPPED_1 = true --------------|

|

DEMAND_TIMER >= EGO_KOER_TM2 -----|AND -|

|     |

VEGOBAR11 > US_SWTPOINT ----------|     |

|

US_SWAPPED_1 = false -------------|     |OR --| FLG_11_RH := true

|AND -|

VEGOBAR11 > US_SWTPOINT ----------|     |

|

US_TST_OK = false ----------------------|

11-145

EGO MONITOR CONTROL STRATEGY, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

US_SWAPPED_1 = true --------------|

|

DEMAND_TIMER >= EGO_KOER_TM2 -----|AND -|

|     |

VEGOBAR21 < US_SWTPOINT ----------|     |

|

US_SWAPPED_1 = false -------------|     |

|AND -|OR --| FLG_21_LN := true

VEGOBAR21 < US_SWTPOINT ----------|     |

|

NUMEGO = 1 -----------------------------|

|

US_TST_OK = false ----------------------|

NUMEGO = 2 -----------------------|

|

VEGOBAR21 > US_SWTPOINT ----------|

|AND -| US_SWAPPED_2 := true

VEGOBAR11 < US_SWTPOINT ----------|     |

|     |

US_TST_OK = true -----------------|     |

| --- ELSE ---

|

| US_SWAPPED_2 := false

11-146

EGO MONITOR CONTROL STRATEGY, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;Compare the voltage downstream of the catalyst on the EGO sensors

;to the switchpoint.  If bank 2 comes lean and bank 1 comes

;rich, everything is o.k.  If bank 2 goes/stays rich AND

;bank 1 goes/stays lean at the end of this phase of the test

;then the potential exists for swapped sensors and a flag

;will be set indicating such.  The option exists to disable

;the downstream test by clearing the DS_KOER_SW.

DS_SWAPPED_1 = true --------------|

|

DEMAND_TIMER >= EGO_KOER_TM2 -----|AND -|

|     |

VEGOBAR22 < DS_SWTPOINT ----------|     |

|

DS_SWAPPED_1 = false -------------|     |

|AND -|OR --| FLG_22_LN := true

VEGOBAR22 < DS_SWTPOINT ----------|     |

|

DS_TST_OK = false ----------------------|

|

HEGO_CONFIG <> dual_bank ---------------|

DS_SWAPPED_1 = true --------------|

|

DEMAND_TIMER >= EGO_KOER_TM2 -----|AND -|

|     |

VEGOBAR12 > DS_SWTPOINT ----------|     |

|

DS_SWAPPED_1 = false -------------|     |

|AND -|OR --| FLG_12_RH := true

VEGOBAR12 > DS_SWTPOINT ----------|     |

|

DS_TST_OK = false ----------------------|

|

HEGO_CONFIG = no_aft_hego --------------|

|

HEGO_CONFIG = y_pipe -------------------|

HEGO_CONFIG = dual_bank ----------|

|

VEGOBAR22 > DS_SWTPOINT ----------|

|AND -| DS_SWAPPED_2 := true

VEGOBAR12 < DS_SWTPOINT ----------|     |

|     |

DS_TST_OK = true -----------------|     |

| --- ELSE ---

|

| DS_SWAPPED_2 := false

11-147

EGO MONITOR CONTROL STRATEGY, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLG_11_RH = true -----------|

|

FLG_12_RH = true -----------|

|AND -|

FLG_21_LN = true -----------|     |

|     |OR --| DEMAND_STATE := b1_rich_b2_rich

FLG_22_LN = true -----------|     |     | DEMAND_TIMER := 0

|

DEMAND_TIMER >= EGO_KOER_TM2 -----|

HEGO_CONFIG = y_pipe -------------------| FLG_12_RH := false

| FLG_12_LN := false

END: bank1_rich_bank2_lean

BEGIN: bank1_rich_bank2_rich

;The following process, executed on-demand form the ego_er_switching_test

;is used to ramp fuel in bank 1 & 2 rich to test the single downstream

;EGO sensor in the y-pipe configuration.

unconditionally ------------------------| ER_LAM_DSD1 := max(ER_LAM_DSD1 -

|            (VIPRR1 * BG_TMR), REQV)

| ER_LAM_DSD2 := ER_LAM_DSD1

| ER_FUL_REQ := true

;Compare the voltage downstream of the catalyst on the EGO sensors

;to the switchpoint.  If bank 2 comes rich and bank 1 comes

;rich, everything is o.k.  The option exists to disable

;the downstream test by clearing the DS_KOER_SW.

DS_TST_OK = false ----------------|

|OR --| FLG_12_RH := true

VEGOBAR12 >= DS_SWTPOINT ---------|

FLG_12_RH = true -----------------|

|OR --| DEMAND_STATE := b1_lean_b2_lean

DEMAND_TIMER >= EGO_KOER_TM3 -----|     | DEMAND_TIMER := 0

END: bank1_rich_bank2_rich

11-148

EGO MONITOR CONTROL STRATEGY, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: bank1_lean_bank2_lean

;The following process, executed on-demand form the ego_er_switching_test

;is used to ramp fuel in bank 1 & 2 lean to test the single downstream

;EGO sensor in the y-pipe configuration.

unconditionally ------------------------| ER_LAM_DSD1 := min(ER_LAM_DSD1 +

|            (VIPLR1 * BG_TMR), LEQV)

| ER_LAM_DSD2 := ER_LAM_DSD1

| ER_FUL_REQ := true

;Compare the voltage downstream of the catalyst on the EGO sensors

;to the switchpoint.  If bank 2 comes lean and bank 1 comes

;lean, everything is o.k.  The option exists to disable

;the downstream test by clearing the DS_KOER_SW.

DS_TST_OK = false ----------------|

|OR --| FLG_12_LN := true

VEGOBAR12 <= DS_SWTPOINT ---------|

FLG_12_LN = true -----------------|

|OR --| DEMAND_STATE := test_complete

DEMAND_TIMER >= EGO_KOER_TM4 -----|

END: bank1_lean_bank2_lean

11-149

EGO MONITOR CONTROL STRATEGY, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: report_codes

;The following process is executed when the KOER test is over and reports

;fault codes to the diagnostic executive.

US_SWAPPED_1 = true --------------|

|AND -| Do: store_code(P1128)

US_SWAPPED_2 = true --------------|     |   ;upstream EGO sensors

|   ;swapped, probably.

DS_SWAPPED_1 = true --------------|

|AND -| Do: store_code(P1129)

DS_SWAPPED_2 = true --------------|     |   ;downstream EGO sensors

|   ;swapped, probably.

P1128MALF = false ----------------|

|AND -| Do: store_code(P1132)

FLG_11_LN = false ----------------|     |

| --- ELSE ---

|

| Do: clear_malf(P1132)

P1128MALF = false ----------------|

|AND -| Do: store_code(P1131)

FLG_11_RH = false ----------------|     |

| --- ELSE ---

|

| Do: clear_malf(P1131)

P1129MALF = false ----------------|

|AND -| Do: store_code(P1138)

FLG_12_LN = false ----------------|     |

| --- ELSE ---

|

| Do: clear_malf(P1138)

P1129MALF = false ----------------|

|AND -| Do: store_code(P1137)

FLG_12_RH = false ----------------|     |

| --- ELSE ---

|

| Do: clear_malf(P1137)

11-150

EGO MONITOR CONTROL STRATEGY, EGO SWITCHING TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

P1128MALF = false ----------------|

|AND -| Do: store_code(P1152)

FLG_21_LN = false ----------------|     |

| --- ELSE ---

|

| Do: clear_malf(P1152)

P1128MALF = false ----------------|

|AND -| Do: store_code(P1151)

FLG_21_RH = false ----------------|     |

| --- ELSE ---

|

| Do: clear_malf(P1151)

P1129MALF = false ----------------|

|AND -| Do: store_code(P1158)

FLG_22_LN = false ----------------|     |

| --- ELSE ---

|

| Do: clear_malf(P1158)

P1129MALF = false ----------------|

|AND -| Do: store_code(P1157)

FLG_22_RH = false ----------------|     |

| --- ELSE ---

|

| Do: clear_malf(P1157)

END: report_codes

11-151

EGO MONITOR CONTROL STRATEGY, EGO FAULT FLAG CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

11.1.8  EGO FAULT FLAG CONTROL (CDAM0)

OVERVIEW

+----------------------------------+

| EGO_FFG_CONTROL                  |

+----------------------------------+

|PUBLIC PROCEDURES:                |

|. definitions                     |

|. ego11_ffg_control               |

|. ego21_ffg_control               |

|. ego12_ffg_control               |

|. ego22_ffg_control               |

+----------------------------------+

This module is used to control the EGO system FMEM flags as required  by  the

Diagnostic  Executive  interface  requirements.   These flags will be used to

disable other OBDII system monitors when relevant  EGO  sensor  failures  are

present or suspected to be present.

For the upstream EGO sensors, the appropriate flag will  set  when  a  single

overvoltage  sample  is  received,  when  aged  EGOs  are detected by the EGO

monitor, when a  heater  malfunction  is  detected  or  when  the  continuous

switching test detects a lack of EGO sensor switching.

For the downstream EGO sensors, the appropriate  flag  will  be  set  when  a

single  overvoltage  sample  is received, when a dead EGO signal is detected,

when insufficient activity  is  detected  on  the  sensor,or  when  a  heater

malfunction is detected.

DEFINITIONS

Registers:

-  OV_TMR_11 = EGO11 overvoltage timer.

-  OV_TMR_12 = EGO12 overvoltage timer.

-  OV_TMR_21 = EGO21 overvoltage timer.

-  OV_TMR_22 = EGO22 overvoltage timer.

Bit Flags:

-  EGO1FMFLG = EGO1 FMEM flag; 1 -> EGO1 sensor failure.

-  EGO2FMFLG = EGO2 FMEM flag; 1 -> EGO2 sensor failure.

-  FFG_CSD11 = Fault flag for CSD on sensor 11.

-  FFG_CSD21 = Fault flag for CSD on sensor 21.

11-152

EGO MONITOR CONTROL STRATEGY, EGO FAULT FLAG CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FFG_EGO11 = OBDII system FMEM flag for EGO11; 1 -> EGO11 is not currently

switching reliably.

-  FFG_EGO12 = OBDII system FMEM flag for EGO12; 1 -> EGO12 is not currently

switching reliably.

-  FFG_EGO21 = OBDII system FMEM flag for EGO21; 1 -> EGO21 is not currently

switching reliably.

-  FFG_EGO22 = OBDII system FMEM flag for EGO22; 1 -> EGO22 is not currently

switching reliably.

-  PxxxMALF = Malfunction flag for code Pxxx; 1 -> a  malfunction  currently

exists for fault Pxxx.

Calibration Constants:

-  HEGO_CONFIG = HEGO configuration register.

-  NUMEGO = Number of EGR sensors.

OTHER

-  P0133 = Oxygen sensor circuit slow response (bank 1 upstream).

-  P0135 = Fault code for EGO11 sensor heater circuit malfunction.

-  P0136 = Oxygen sensor circuit malfunction (bank 1 downstream).

-  P0141 = Fault code for EGO12 sensor heater circuit malfunction.

-  P0153 = Oxygen sensor circuit slow response (bank 2 upstream).

-  P0155 = Fault code for EGO21 sensor heater circuit malfunction.

-  P0156 = Oxygen sensor circuit malfunction (bank 2 downstream).

-  P0161 = Fault code for EGO22 sensor heater circuit malfunction.

-  P1133 = Oxygen sensor switchpoint indicates lean (EGO11).

-  P1134 = Oxygen sensor switchpoint indicates rich (EGO11).

-  P1153 = Oxygen sensor switchpoint indicates lean (EGO21).

-  P1154 = Oxygen sensor switchpoint indicates rich (EGO21).

11-153

EGO MONITOR CONTROL STRATEGY, EGO FAULT FLAG CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EGO_FFG_CONTROL_COM10

BEGIN: definitions

;This section defines symbols used to make the strategy more readable.

;It is not to be explicitly executed.

unconditionally ------------------------| no_aft_hego := 0

| single_bank := 1

| dual_bank := 2

| y_pipe := 3

END: definitions

BEGIN: ego11_ffg_control

;PUBLIC process to control FFG_EGO11.  This is to be executed once per

;background loop.

FFG_CSD11 = 1 --------------------|

;CSD detected.                   |

|

OV_TMR_11 > 0 --------------------|

;overvoltage.                    |

|

P0133MALF = 1 --------------------|

;slow response.                  |

|

P0135MALF = 1 --------------------|

;heater fault.                   |OR --| FFG_EGO11 := 1

|     |

EGO1FMFLG = 1 --------------------|     |

;lack of switching.              |     |

|     |

P1133MALF = 1 --------------------|     |

;static shift                    |     |

|     |

P1134MALF = 1 --------------------|     |

;static shift                          |

| --- ELSE ---

|

| FFG_EGO11 := 0

END: ego11_ffg_control

11-154

EGO MONITOR CONTROL STRATEGY, EGO FAULT FLAG CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ego21_ffg_control

;PUBLIC process to control FFG_EGO21.  This is to be executed once per

;background loop.

NUMEGO = 2 -----------------------|

|

FFG_CSD21 = 1 --------------|     |

;CSD detected.             |     |

|     |

OV_TMR_21 > 0 --------------|     |

;overvoltage.              |     |

|     |AND -| FFG_EGO21 := 1

P0153MALF = 1 --------------|     |     |

;slow response.            |     |     |

|     |     |

P0155MALF = 1 --------------|     |     |

;heater fault.             |OR --|     |

|           |

EGO2FMFLG = 1 --------------|           |

;lack of switching.        |           |

|           |

P1153MALF = 1 --------------|           |

;static shift              |           |

|           |

P1154MALF = 1 --------------|           |

;static shift                          |

| --- ELSE ---

|

| FFG_EGO21 := 0

END: ego21_ffg_control

BEGIN: ego12_ffg_control

;PUBLIC process to control FFG_EGO12.  This is to be executed once per

;background loop.

HEGO_CONFIG <> no_aft_hego -------|

|

OV_TMR_12 > 0 --------------|     |AND -| FFG_EGO12 := 1

;overvoltage.              |     |     |

|     |     |

P0136MALF = 1 --------------|OR --|     |

;insufficient activity.    |           |

|           |

P0141MALF = 1 --------------|           |

;heater fault.                         |

| --- ELSE ---

|

| FFG_EGO12 := 0

END: ego12_ffg_control

11-155

EGO MONITOR CONTROL STRATEGY, EGO FAULT FLAG CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ego22_ffg_control

;PUBLIC process to control FFG_EGO22.  This is to be executed once per

;background loop.

HEGO_CONFIG = dual_bank ----------|

|

OV_TMR_22 > 0 --------------|     |AND -| FFG_EGO22 := 1

;overvoltage.              |     |     |

|OR --|     |

P0156MALF = 1 --------------|           |

;insufficient activity.    |           |

|           |

P0161MALF = 1 --------------|           |

;heater fault.                         |

| --- ELSE ---

|

| FFG_EGO22 := 0

END: ego22_ffg_control

11-156

EGO MONITOR CONTROL STRATEGY, HEGO SENSOR INPUT PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

11.1.9  HEGO SENSOR INPUT PROCESSING (CDAN0)

OVERVIEW

+-----------------------------------+

| EGO_INPUT_COM10                   |

+-----------------------------------+

|PUBLIC PROCEDURES:                 |

| .pid_definitions                  |

| .ego_input_init                   |

| .background_ego_input_processing  |

+-----------------------------------+

|PRIVATE PROCEDURES                 |

| .read_downstream_sensors          |

| .read_upstream_sensors            |

| .upstream_open_circuit_check      |

+-----------------------------------+

|

|          +------------------------------+

|          | EGO_INPUT_XFER_COM1          |

|   ______>+------------------------------+

(Get A/D voltage for  |  /       |PUBLIC PROCEDURES:            |

upstream HEGOs.)     | /        | .background_ego_sampling     |

INPUT_XFER -> normal |/         +------------------------------+

ENHYS -> RLCF        +

|\         +------------------------------+

| \        | EGO_ENHYS_COM1               |

|  \       +------------------------------+

|   ------>|PUBLIC PROCEDURES:            |

|          | .background_ego_sampling     |

|          +------------------------------+

|

|      +------------------------------+

(Calculate adaptive   |      | EGO_CSD_COMPENSATION_COM     |

HEGO switchpoint.)   |      +------------------------------+

COM1 -> CSD algo     |      |PUBLIC PROCEDURES:            |

COM2 -> stub         |----->| .update_adaptive_switchpoint |

|      +------------------------------+

|

|      +------------------------------+

(Calculate byples     |      | EGO_BYPLES_COM               |

value.)              |      +------------------------------+

COM1 -> normal       |      |PUBLIC PROCEDURES:            |

COM2 -> RCLF         |----->| .update_byples               |

|      +------------------------------+

|

(Offset US_SWTPOINT   |      +------------------------------+

stoich voltage for   |      | EGO_US_SWITCHPOINT_COM       |

alternate fuels.)    |      +------------------------------+

COM1 -> gasoline     |      |PUBLIC PROCEDURES:            |

COM2 -> PM           +----->| .update_fixed_switchpoint    |

COM3 -> CNG                 +------------------------------+

Up to 4 HEGO sensors may be connected to an EEEC module.  In accordance  with

OBDII  protocol,  the  names  of  these  sensors  contain  reference to their

location and can be presented as HEGOxy, where

11-157

EGO MONITOR CONTROL STRATEGY, HEGO SENSOR INPUT PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

x = 1 for engine Bank1 and 2 for engine Bank2;########################

y = 1 for upstream HEGO sensor and 2 for downstream (rear) HEGO sensor.

For example, HEGO12 and HEGO22 sensors are located  downstream  of  catalysts

for engine Bank1 and Bank2 respectively.  Strategy parameters (flags, timers,

counters, etc.) which are associated with certain HEGO sensor have  the  same

designations  as  HEGO  sensor.   For  example,  EGOFL11  and EGOFL21 are the

rich/lean state flags for the upstream sensors.

However, some strategy parameters which are not associated with engine  Bank1

or   Bank2,   but   rather  associated  with  location,  have  only  location

designation.   For  example,  HEGO  switch  point  for  upstream  HEGO's   is

US_SWTPOINT,  and  for  downstream  HEGO's  is  DS_SWTPOINT.  It is therefore

important to remember that any HEGO parameters with single digit naming could

be  referring to a Bank or a location (up/downstream).  US_SWTPOINT refers to

an upstream parameter, while LAMBSE1 refers to a Bank1 related parameter.

HEGO sensor output is defined as a voltage rather than counts  in  accordance

with  the  hardware  transfer  function formula.  For the upstream HEGOS this

conversion is done in the ego_enhys_com module for Robust  Closed  Loop  Fuel

(RCLF)  applications,  and  in  the  ego_input_xfer_com  module  for non-RCLF

applications.  Under normal operating conditions  HEGO  voltage  will  switch

between  from  0V  (lean)  to  1V (rich).  S_VEGOxy is the raw sensor voltage

parameter after conversion from A/D counts.

The raw sensor output voltage for the upstream HEGOs is fed into an  adaptive

switchpoint  algorithm.  This algorithm tracks the current value for the HEGO

switchpoint, in order to compensate  for  voltage  shifts  away  from  normal

levels,  which  may be induced due to the sensor becoming contaminated.  This

condition appears as a Characteristic Shift  Downward  (CSD)  of  the  sensor

switching  voltages.   The  adaptive switchpoint value represents the average

sensor voltage shift away from the  normal  stoichiometry  switchpoint  value

(US_SWTPOINT) for gasoline fuel.

11-158

EGO MONITOR CONTROL STRATEGY, HEGO SENSOR INPUT PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(A/D Counts                 Raw HEGO        (Add offset

from HW)                   voltage)         due to CSD)

+--------------+

IEGOn1    |   Convert    |  S_VEGOn1            +---+   VEGOn1

--------->|  A/D Counts  |-----+--------------->| + |---------->

|   to Volts   |     |                +---+

+--------------+     |                  |      (To Closed

(ego_input_xfer_com)   |                  |       Loop Fuel)

OR             |              (0.45V -

(ego_enhys_com)      |             SWTPOINTn1)

v                  |

+-------------+           | (Adaptive switchpoint,

|  Adaptive   |           |  SWTPOINTn1)

| Switchpoint |-----------+

|  Algorithm  |

+-------------+

(ego_csd_compensation_com)

1V   +-+ +-+                             +-+ +-+ +-+ +-+ +-+ +-+   1V

| | | | +-+                         | | | | | | | | | | | |

| | | | | |                         | | | | | | | | | | | |

| | | | | | +-+                     | | | | | | | | | | | |

0V --+ +-+ +-+ | | |                   --+ +-+ +-+ +-+ +-+ +-+ +-+ 0V

S_VEGOn1  +-+ | +-+ +-+                    VEGOn1

| | | | |

+-+ | | |  <--- (CSD)

| | |

-1V       |             +-+ +--                           |

|                                               |

|                                               |

|        0.45V --------   <-- (US_SWTPOINT)     |

|                        \                      |

+--->          SWTPOINTn1  \                >---+

\

-0.55V                  --------

If  an  upstream  HEGO  is  in  CSD,  the  adaptive  switchpoint  calculation

SWTPOINTn1  is used to compensate for this negative voltage drift by shifting

the unsigned voltage parameter VEGOn1 back up to normal voltage levels.   The

figures above illustrate this input processing for the upstream HEGOs.  (If a

downstream HEGO goes into CSD, it will get clipped to 0V in VEGOn2.)

11-159

EGO MONITOR CONTROL STRATEGY, HEGO SENSOR INPUT PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  BYPLES_TMn = Calculated time delay before lack of ego switching  on  bank

n.

-  EGOCNTn1 = Number of switches since upstream, bank n sensor failed.

-  EGOSSn1 = Switches since start for upstream, bank n sensor.

-  EGOSSS1 = Switches since start for both upstream sensors.

-  IEGOxy = A/D counts for HEGO bank x, location y sensor.  Location = 1  is

upstream, location = 2 is downstream of the catalyst.

-  lam_at_clip = Temporary clip flag LAMBSEn (bank n), 1 -> clip  level  has

been reached.

-  LAMBSEn = Desired equivalence ratio for Bank n injectors.

-  LESTMRn1 = Lack of HEGO switch timer for bank n sensor, sec.

-  LAMMINn = Lower lambse clip calculated for bank n.

-  LAMMAXn = Upper lambse clip calculated for bank n.

-  NUMEGO = Switch indicating number of HEGO sensors present; select mono or

stereo.

-  P0130FIL = Lack of EGO11 switch, EGO11 open circuit fault filter.

-  P0150FIL = Lack of EGO21 switch, EGO21 open circuit fault filter.

-  S_VEGOxy = HEGO sensor output voltage after counts to  volts  conversion,

bank x, location y.

-  SWTP_BARn1 = Filtered value for SWTPOINTn1, used in the upstream  monitor

to  reduce  variance  and  attenuation  in  the  calculation  of the peak

rich/lean signals.

-  SWTPOINTn1 = Adaptive switchpoint calculation for bank n, volts.

-  tempfl = Temporary register for EGOFLxy flag.

-  US_OFFSET = Offset voltage for the upstream HEGO sensor switchpoint, from

the  calibrated  stoichiometric  voltage  US_SWTPOINT,  due  to  methanol

content or compressed natural gas fuels.

-  VEGOn1 = Bank n, upstream  HEGO  voltage  compensated  with  an  adaptive

switchpoint  calculation  to  maintain  switching  output  between 0 to 1

volts.

-  VEGOn2 = Bank n downstream HEGO voltage.

Bit Flags:

11-160

EGO MONITOR CONTROL STRATEGY, HEGO SENSOR INPUT PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  DFIB_LEANn1 = Flag which is set when the lo catcher calculation is stuck.

Initiates a lambse reset from the lean clip to as part of the switchpoint

recovery process.

-  DFIB_RICHn1 = Flag which is set when the hi catcher calculation is stuck.

Initiates a lambse reset from the rich clip to as part of the switchpoint

recovery process.

-  EGOFLn1 = Bank n upstream HEGO flag.

-  LEGOFGn1 = Lack of HEGO switching flag, bank n.

-  LESFLG1 = Lack of HEGO switching flag; 1 -> HEGO is not switching,  0  ->

HEGO is switching.

-  OLFLG = Open loop flag; 1 -> open loop fuel; 0 -> closed loop fuel.

-  OPEN_EGOn1 = Flag indicating open circuit detected on Upstream EGO bank n

sensor.

-  SWTFLn1 = Bank n, upstream HEGO switch flag; 0 -> no HEGO  switch,  1  ->

HEGO switch.

Calibration Constants:

-  BYPLES1 = Time delay before lack of switch.

-  DS_SWTPOINT = Downstream HEGO input switch point, volts.

-  FILHYS =  Hysterisis  band  (number  of  counts)  before  clearing  fault

filters.

-  HEGO_CONFIG = HEGO configuration register.

-  P0130LVL =  Lack  of  EGO11  switch,  EGO11  open  circuit  fault  filter

threshold.

-  P0150LVL =  Lack  of  EGO21  switch,  EGO21  open  circuit  fault  filter

threshold.

-  P0130UP  =  Lack  of  EGO11  switch,  EGO11  open  circuit  fault  filter

increment.

-  P0150UP  =  Lack  of  EGO21  switch,  EGO21  open  circuit  fault  filter

increment.

-  SWTP_FC_AVG = Time constant for the switchpoint averaging filter.

-  TC_BYPLES = Time constant for ROLAV of time delay before lack of switch.

-  US_SWTPOINT = Upstream HEGO input switch point, volts.

-  VABS_OPEN_HI = Upper limit for the open circuit voltage band, volts.

-  VABS_OPEN_LO = Lower limit for the open circuit voltage band, volts.

11-161

EGO MONITOR CONTROL STRATEGY, HEGO SENSOR INPUT PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EGO_INPUT_COM10

BEGIN:  pid_definitions

pid_def(j1979_01_13, EGOMAP_J1979)

pid_def(j1979_01_14, min((VEGO11*200), 255))

;this pid has resolution of 1.

pid_def(j1979_01_151, min((VEGO12*200), 255))

;this pid has resolution of 1.

pid_def(j1979_01_152, 255)

;this pid has resolution of 1.

NUMEGO = 2 -----| pid_def(j1979_01_18, min((VEGO21*200), 255))

|  ;this pid has resolution of 1.

|

| --- ELSE ---

|

| pid_def(j1979_01_18, 255)

HEGO_CONFIG <> 3 --| pid_def(j1979_01_191, min((VEGO22*200), 255))

|  ;this pid has resolution of 1.

|

| --- ELSE ---

|

| pid_def(j1979_01_191, 255)

pid_def(j1979_01_192, 255)

END:  pid_definitions

11-162

EGO MONITOR CONTROL STRATEGY, HEGO SENSOR INPUT PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: definitions

;The following assignments are made to make the documentation more

;readable.

unconditionally ----| no_aft_hego := 0

| single_bank := 1

| dual_bank := 2

| y_pipe := 3

| rich := 1

| lean := 0

| rich_state := 1

| lean_state := -1

| true := 1

| false := 0

| open_loop := 1

| closed_loop := 0

| closed_throttle := -1

| stereo := 2

END: definitions

11-163

EGO MONITOR CONTROL STRATEGY, HEGO SENSOR INPUT PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ego_input_init

;Execute this process only when called (by monitor control module).

unconditionally ----| EGOFL11 := 0

| EGOFL21 := 0

| SWTFL11 := 0

| SWTFL21 := 0

| LESTMR11 := 0

| LESTMR21 := 0

| EGOCNT11 := 0

| EGOCNT21 := 0

| LESFLG1 := 0

| OPEN_EGO11 := 0

| OPEN_EGO21 := 0

| P0130FIL := 0

| P0150FIL := 0

| SWTP_BAR11 := US_SWTPOINT

| SWTP_BAR21 := US_SWTPOINT

END: ego_input_init

BEGIN: background_ego_input_processing

;Perform the following once per background pass

unconditionally -------------------| Do: read_upstream_sensors

| Do: read_downstream_sensors

| Do: upstream_open_circuit_check

LEGOFG11 = 1  ---------|

|

LEGOFG21 = 1 ----------|AND -|

|     |

NUMEGO = 2 ------------|     |OR --| LESFLG1 := 1

|     |  (HEGO is not switching)

LEGOFG11 = 1 ----------|     |     |

|AND -|     |

NUMEGO = 1 ------------|           |

| --- ELSE ---

|

| LESFLG1 := 0

|  (HEGO is switching)

END: background_ego_input_processing

11-164

EGO MONITOR CONTROL STRATEGY, HEGO SENSOR INPUT PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: read_upstream_sensors

;Perform the following process only when called.

For n := 11 and 21 perform the following:

n = 11 ----------------------------| bank := 1

|

| --- ELSE ---

|

| bank := 2

;

; The process background_ego_sampling samples the Upstream sensor voltage

; at background rates.  The parameter SWTPOINTn1 contains the

; adaptive switchpoint calculation.

;

bank = 1 --------------------|

|OR --| S_VEGO[n] :=

bank = 2 --------------|     |     |   background_ego_sampling(bank,IEGO[n])

|AND -|     | Do: update_adaptive_switchpoint(bank)

NUMEGO = 2 ------------|           |   ;update SWTPOINTn1 parameter

| csd_bias := US_SWTPOINT - SWTPOINT[n]

| VEGO[n] := max(S_VEGO[n] + csd_bias,0)

| tempfl := EGOFL[n]

| SWTP_BAR[n] := rolav(SWTPOINT[n],

|                      SWTP_FC_AVG)

|   ;filter switchpoint for US Mon.

|

| --- ELSE ---

|

| S_VEGO[n] := 0

| VEGO[n] := 0

|  ;bank2 does not exist.

;

; Add in upstream switchpoint voltage offset the calibrated

; stoichiometry value US_OFFSET due to methanol content or

; compressed natural gas fuels.  This offset will be 0 for

; gasoline applications.

;

unconditionally ---------| Do: update_fixed_switchpoint

| us_swtpoint := US_SWTPOINT + US_OFFSET

VEGO[n] > us_swtpoint -------------| EGOFL[n] := 1

|

| --- ELSE ---

|

| EGOFL[n] := 0

11-165

EGO MONITOR CONTROL STRATEGY, HEGO SENSOR INPUT PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;

; Switches forced by the adaptive switchpoint recovery process

; don't count.

;

DFIB_LEAN[n] = 0 -----------------|

|

DFIB_RICH[n] = 0 -----------------|

|

tempfl <> EGOFL[n] ---------------|AND -| SWTFL[n] := 1

|

| --- ELSE ---

|

| SWTFL[n] := 0

LAMBSE[bank] >= LAMMAX[bank] ----|

|OR --| lam_at_clip := 1

LAMBSE[bank] <= LAMMIN[bank] ----|     |

| --- ELSE ---

|

| lam_at_clip := 0

11-166

EGO MONITOR CONTROL STRATEGY, HEGO SENSOR INPUT PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

LESTMR[n] - LACK OF EGO SWITCHING TIMER (TIME SINCE LAST EGO SWITCH)

lam_at_clip = 0 -------------|

|OR --| LESTMR[n] := 0

OLFLG = 1 -------------------|     |

| --- ELSE ---

|

| Count up LESTMR[n]

Additional logic associated with LESTMR[n];

SWTFL[n] = 1 ----------------------| Increment EGOCNT[n]

Calculate HEGO failure time delay criteria.

unconditionally -------------------| t_byples = update_byples(bank)

| ;calculate time delay to declare

|  a HEGO failure (units = seconds)

|

| t_byples = max(t_byples,BYPLES1)

| ;clip value to BYPLES1 as a minimum

Assign value to BYPLES_TM.

t_byples >= BYPLES_TM[bank] -------| BYPLES_TM[bank] = t_byples

| ;jump to new value to BYPLES_TM

|

| --- ELSE ---

|

| BYPLES_TM[bank] = rolav(t_byples,

|                         TC_BYPLES)

| ;gradually decrease BYPLES time delay

LESTMR[n] >= BYPLES_TM[bank] ------| LEGOFG[n] := 1

| EGOCNT[n] := 0

EGOCNT[n] > 0 ---------------------| LEGOFG[n] := 0

11-167

EGO MONITOR CONTROL STRATEGY, HEGO SENSOR INPUT PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Upstream HEGO11 counters:

SWTFL[n] = 1 ----------------------| Increment EGOSS[n]

| Increment EGOSSS1

|

| --- ELSE ---

|

| Freeze Counters

END: read_upstream_sensors

BEGIN: read_downstream_sensors

;

;Perform the following process only when called.

;

HEGO_CONFIG <> no_aft_hego --------| S_VEGO12 = ego_hw_xfer_fn(IEGO12)

|   ;convert A/D counts to volts

| VEGO12 := max(S_VEGO12,0)

|

| --- ELSE ---

|

| S_VEGO12 := 0

| VEGO12 := 0

NUMEGO = stereo -------------|

|AND -| S_VEGO22 = ego_hw_xfer_fn(IEGO22)

HEGO_CONFIG <> y_pipe -------|     |   ;convert A/D counts to volts

| VEGO22 := max(S_VEGO22,0)

|

| --- ELSE ---

|

| S_VEGO22 := 0

| VEGO22 := 0

END: read_downstream_sensors

11-168

EGO MONITOR CONTROL STRATEGY, HEGO SENSOR INPUT PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: upstream_open_circuit_check

;Perform the following process when called from

;background_ego_input_processing.

;Check for open circuit fault conditions for each

;EGO sensor.

S_VEGO11 < VABS_OPEN_HI -----|

|AND -| P0130FIL := P0130FIL +

S_VEGO11 > VABS_OPEN_LO -----|     |             P0130UP

|

| --- ELSE ---

|

| P0130FIL := P0130FIL - 1

P0130FIL > P0130LVL ---------------| OPEN_EGO11 := 1

|  ;Open circuit detected.

|

| --- ELSE ---

|

P0130FIL < P0130LVL - FILHYS ------| OPEN_EGO11 := 0

S_VEGO21 < VABS_OPEN_HI -----|

|AND -| P0150FIL := P0150FIL +

S_VEGO21 > VABS_OPEN_LO -----|     |             P0150UP

|

| --- ELSE ---

|

| P0150FIL := P0150FIL - 1

P0150FIL > P0150LVL ---------------| OPEN_EGO21 := 1

|  ;Open circuit detected.

|

| --- ELSE

|

P0150FIL < P0150LVL - FILHYS ------| OPEN_EGO21 := 0

END: upstream_open_circuit_check

11-169

EGO MONITOR CONTROL STRATEGY, HEGO HARDWARE TRANSFER FUNCTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

11.1.10  HEGO HARDWARE TRANSFER FUNCTION (CDAN0)

OVERVIEW

+-----------------------------------+

| EGO_HW_XFER                       |

+-----------------------------------+

|PUBLIC PROCEDURES:                 |

| .ego_hw_xfer_fn                   |

+-----------------------------------+

HEGO sensor input A/D counts to volts transfer function for ML-II hardware.

This module was created so the same hardware transfer function  call  can  be

shared between the background ego input, the upstream ego monitor, and Robust

Closed Loop Fuel fixed rate processing  modules.   This  module  helps  align

strategy with the software implementation.

DEFINITIONS

Registers:

-  hego_volts = Local parameter used only by this function.

Calibration Constants:

11-170

EGO MONITOR CONTROL STRATEGY, HEGO HARDWARE TRANSFER FUNCTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EGO_HW_XFER_COM1

BEGIN:  ego_hw_xfer_fn(counts)

;

; ML-II A/D counts to volts transfer function for HEGO sensor input.

; This function receives A/D counts as an input, converts counts to

; volts using hardware transfer function, and returns this voltage

; as an output:

;

; For ML-II, the transfer function is:

;   HEGO sensor volts = (1/1023) * (5/3) * A/D counts

;

; ML-II voltages can have negative values.  Sign information is

; transmitted in B5 of the transmitted data during A/D conversion.

; For ML-II, the "counts" input can range from -1024 to 1023.

;

; ML-II hardware does not need to use the IVCAL reference voltage

; in the transfer function (as does ML-I).

;

unconditionally -------------------| hego_volts := (counts/1023) *

|       (5.0/3.0)

;

; This process should be implemented like a function, and return the

; value of the "hego_volts" parameter.  This parameter should be

; defined in software (i.e. size, bin point resolution, signed) the

; same as the HEGO voltage parameters S_VEGOxy and S_USVEGOxy.

;

unconditionally -------------------| Return hego_volts

END: ego_hw_xfer_fn(counts)

11-171

EGO MONITOR CONTROL STRATEGY, CSD COMPENSATION ALGORITHM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

11.2  CSD COMPENSATION ALGORITHM (CDAN0)

OVERVIEW

+------------------------------+

| EGO_CSD_COMPENSATION         |

+------------------------------|

|PUBLIC PROCEDURES:            |

|. init_adaptive_switchpoint   |

|. update_adaptive_switchpoint |

+------------------------------+

|PRIVATE PROCEDURES: (none)    |

+------------------------------+

This module  is  "stubbed"  into  applications  which  do  not  use  the  CSD

compensation  (aka adaptive switchpoint) algorithm.  Global parameter outputs

from the algorithm are stubbed by this module to their default values.

This module allows common strategy and software modules to  be  used  by  the

rest of the HEGO Monitor with and without the algorithm.

DEFINITIONS

Registers:

-  ADP_STATEn1 = Current state for switchpoint stuck detection controller.

-  CSD_CUMLTn1  =  Count  the  cumulative  number  of  times  the   adaptive

switchpoint  calculation  falls  below the adaptive switchpoint crossover

threshold (indicating HEGO CSD conditions).  KAM parameter.

-  SWTPOINTn1 = Normally use the fixed constant US_SWTPOINT,  but  when  CSD

occurs  use  the adaptive switchpoint calculation until the CSD condition

clears.

Bit Flags:

-  DFIB_LEANn1 = Flag which is set when the lo catcher calculation is stuck.

Initiates a lambse reset from the lean clip to as part of the switchpoint

recovery process.

-  DFIB_RICHn1 = Flag which is set when the hi catcher calculation is stuck.

Initiates a lambse reset from the rich clip to as part of the switchpoint

recovery process.

-  SWTP_STABLn1 = Flag which is set if the switchpoint is stable  at  normal

or  csd  operation  levels.   This  flag  is derived from the switchpoint

movement state machine above.

11-172

EGO MONITOR CONTROL STRATEGY, CSD COMPENSATION ALGORITHM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EGO_CSD_COMPENSATION_COM2

BEGIN: definitions

;

; The following represent definitions used throughout this module.  This

; is not intended to be executed.  It exists only to make the documentation

; more readable.

;

unconditionally ---------------------| true := 1

| false := 0

|  ;adaptive switchpoint states

| switching_ok := 0

| hi_catcher_stuck := 1

| lo_catcher_stuck := 2

| lost_and_retrying := 3

| lost_and_stopped := 255

|  ;switchpoint movement states

| normal := 0

| drifting_down := 1

| drifting_up := 2

| in_csd := 3

END: definitions

BEGIN: update_adaptive_switchpoint(bank)

;

; Stub the global parameter outputs from the adaptive switchpoint

; algorithm.

;

unconditionally ---------------------| SWTPOINT[bank]1 := US_SWTPOINT

| ADP_STATE[bank]1 := switching_ok

| SWPT_STABL[bank]1 := 1

| DFIB_LEAN[bank]1 := 0

| DFIB_RICH[bank]1 := 0

| ; stub adaptive switchpoint

END: update_adaptive_switchpoint(bank)

11-173

EGO MONITOR CONTROL STRATEGY, CSD COMPENSATION ALGORITHM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: init_adaptive_switchpoint(bank)

;

; Execute the following process during RAM init and when called.

;

unconditionally ---------------------| SWTPOINT[bank]1 := US_SWTPOINT

| ADP_STATE[bank]1 := switching_ok

| SWTP_STABL[bank]1 := 1

| DFIB_LEAN[bank]1 := 0

| DFIB_RICH[bank]1 := 0

| ; stub global RAM parameters

|

| CSD_CUMLT[bank]1 := 0

| ; clear KAM parameter

END: init_adaptive_switchpoint(bank)

11-174

EGO MONITOR CONTROL STRATEGY, CSD DETECTION ALGORITHM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

11.3  CSD DETECTION ALGORITHM (CDAN0)

OVERVIEW

+----------------------------------+

| EGO_CSD_DETECTION                |

+----------------------------------+

|PUBLIC PROCEDURES:                |

|. csd_detection_init              |

|. csd_detection_kam_reset         |

|. csd_detection_process           |

+----------------------------------+

|PRIVATE PROCEDURES:               |

+----------------------------------+

This module monitors HEGO sensor voltage levels for detection  Characteristic

Shift  Downward (CSD) conditions.  CSD can occur several ways, but the bottom

line is the sensor voltage is shifted  downward  by  up  to  1  volt  and  is

rendered  un-usable to the rest of the system.  The purpose of this module is

to let the system know when this has happened.

This module will output FFG_CSDn1  failure  mode  flags  to  indicate  sensor

switching  has  stopped  due  to CSD.  Logic to set the FFG_CSD flags is very

similiar to the lack of switching LEGOFG logic.  In a CSD  condition,  either

an  FFG_CSD  flag,  OR an LEGOFG flag, but not both, will be set depending on

the calibration of BYPLES time constant multiplier CSDMUL.  If  CSDMUL  <  1,

then the FFG_CSD flag gets set, otherwise the LEGOFG is set.

Because of their similiar definitions, FFG_CSD and LEGOFG flags's are  'OR'ed

to control FMEM actions, including:

-  1) EGO Continuous Test:  Hold  fault  filters  after  lack  of  switching

fault.

-  2) Adaptive Fuel:  set the KAMRFn flags to 1 to  disable  adaptive  table

updates.

-  3) Closed Loop Fuel:  Set the FAILED_BANK parameter.

-  4) Open Loop Fuel:  Set the OL_FMEM failure mode flag to go open loop.

If the CSD Detection algorithm is not used, then the "Com2" version should be

installed in place of this module to "stub" the FFG_CSD flags.

11-175

EGO MONITOR CONTROL STRATEGY, CSD DETECTION ALGORITHM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  BYPLES_TMn = Calculated time delay before lack of ego switching  on  bank

n.

-  CSD_SWITCHn1 = Number of EGO switches counted on bank n sensor, while the

sensor is in CSD.

-  CSD_SWITCH21 = Number of EGO switches since CSD detection on  senosr  21,

counts.

-  CSD_TIMERn1 = Amount of time after S_VEGOn1 goes negative before flagging

a CSD likely condition, seconds.

-  S_VEGOn1 = Bankn upstream signed HEGO voltage.

-  LAMMINn = Lower lambse clip calculated for bank n.

Bit Flags:

-  CSD_LIKELYn1 = Flag set when voltage, switching,  and  lambse  conditions

are such that CSD is suspected to be present on bank n sensor.

-  FFG_CSDn1 = Fault flag for CSD on bank n sensor.

-  OLFLG = Open loop fuel flag; 1 -> Open Loop fuel; 0 -> Closed loop fuel

-  SWTFLn1 = Bank n upstream HEGO switch flag.

Calibration Constants:

-  CSDMUL = Multiplier on BYPLES_TM to signal CSD fault, unitless.

-  CSD_SWITCHES = Number of EGO switches to recover from CSD FMEM, counts.

-  CSD_TM = Minimum amount of time after EGO voltage dips below  CSD_VOLTAGE

(goes negative) to set CSD_LIKELY flag, seconds.

-  CSD_VOLTAGE = If the ego voltage S_VEGO is  below  this  threshold,  then

enable CSD timers and switch counters to increment.

-  US_SWTPOINT = Upsteam sensor switchpoint, volts.

11-176

EGO MONITOR CONTROL STRATEGY, CSD DETECTION ALGORITHM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EGO_CSD_DETECTION_COM1

BEGIN: csd_detection_init

;

;Executed during RAM initialization or when called from within the

;EGO monitor control process (during an abort_ego_mon sequence).

;

CSD_LIKELY11 = 1 ------------------------| CSD_TIMER11 := CSD_TM

|

| --- ELSE ---

|

| CSD_TIMER11 := 0

CSD_LIKELY21 = 1 ------------------------| CSD_TIMER21 := CSD_TM

|

| --- ELSE ---

|

| CSD_TIMER21 := 0

END: csd_detection_init

BEGIN: csd_detection_kam_reset

;

;Reset CSD detection KAM parameters during a KAM reset. This process

;is called from the EGO monitor control module.

;

unconditionally ------------------------| CSD_LIKELY11 := 0

| CSD_LIKELY21 := 0

END: csd_detection_kam_reset

11-177

EGO MONITOR CONTROL STRATEGY, CSD DETECTION ALGORITHM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: csd_detection_process

;This process, called on demand from the EGO_MONITOR_CONTROL

;module is used to detect a characteristic shift downward in

;the upstream EGO sensor voltage.  This CSD can result from a

;couple of things.  First if the sensor element is soaked with

;fuel and the reference side gets contaminated, the sensor

;output voltage tends to drift lower, even going negative

;eventually.

;Also microscopic cracks in the sensor element, caused by fluid

;droplets impinging on a hot element can allow leakage of

;exhaust gases into the reference side of the sensor, again

;leading to CSD.

;The transfer function between the EGO sensor and the AtoD converter in

;the module will allow the direct measurment of negative voltage on the

;EGO sensor (up to about -0.3 volts).  The EGO voltage magnitude is

;deleivered to the EEC fromthe AICE during AtoD conversion in the normal

;10 bits.  In addition bit 5 of register B inthe transmit data format

;from the AICE contains a sign bit.  This information is being read in the

;EGO_INPUT module and stored in the register S_VEGOxy.

;When negative voltage persists long enough to disrupt the normal closed

;loop fuel control and drive the system to a rich or lean clip for a

;calibrated amount of time, a CSD fault will be identified.  A fault code

;will only be stored if this fault results in the continuous ego switching

;test (see EGO_CONTINUOUS_TEST_COM) identifying a fault also.  A seperate

;code signifying CSD is present will be stored when lack of switching goes

;to store a code.

SWTFL11 = 1 --------------------------| CSD_SWITCH11 = CSD_SWITCH11 + 1

SWTFL21 = 1 --------------------------| CSD_SWITCH21 = CSD_SWITCH21 + 1

S_VEGO11 < CSD_VOLTAGE ---------------| CSD_TIMER11 := CSD_TM

| CSD_SWITCH11 := 0

| CSD_LIKELY11 := 1

S_VEGO21 < CSD_VOLTAGE ---------|

|AND -| CSD_TIMER21 := CSD_TM

NUMEGO = 2 ---------------------|     | CSD_SWITCH21 := 0

| CSD_LIKELY21 := 1

11-178

EGO MONITOR CONTROL STRATEGY, CSD DETECTION ALGORITHM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

S_VEGO11 >= CSD_VOLTAGE --------|

|

CSD_TIMER11 = 0 ----------------|AND -| CSD_LIKELY11 := 0

|     | FFG_CSD11 := 0

CSD_SWITCH11 > CSD_SWITCHES ----|

S_VEGO21 >= CSD_VOLTAGE --------|

|

CSD_TIMER21 = 0 ----------------|AND -| CSD_LIKELY21 := 0

|     | FFG_CSD21 := 0

CSD_SWITCH21 > CSD_SWITCHES ----|

OLFLG = 0 ----------------------|

|

LAMBSE1 <= LAMMIN1 -------------|

|

LESTMR11 > CSDMUL * BYPLES_TM1 -|AND -| FFG_CSD11 := 1

|     |  ;signal to the continuous

S_VEGO11 < US_SWTPOINT ---------|     |  ;EGO switching test CSD

|     |  ;has been discovered.

CSD_LIKELY11 = 1 ---------------|

OLFLG = 0 ----------------------|

|

LAMBSE2 <= LAMMIN2 -------------|

|

LESTMR21 > CSDMUL * BYPLES_TM2 -|AND -| FFG_CSD21 := 1

|     |  ;signal to the continuous

S_VEGO21 < US_SWTPOINT ---------|     |  ;EGO switching test CSD

|     |  ;has been discovered.

CSD_LIKELY21 = 1 ---------------|

END: csd_detection_process

11-179

EGO MONITOR CONTROL STRATEGY, HEGO INPUT TRANSFER FUNCTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

11.4  HEGO INPUT TRANSFER FUNCTION (CDAM0)

OVERVIEW

+-----------------------------------+

| EGO_INPUT_XFER_COM1               |

+-----------------------------------+

|PUBLIC PROCEDURES:                 |

| .background_ego_sampling          |

+-----------------------------------+

Sample and return the Upstream HEGO sensor input voltage at background  input

processing rates.  This module will be used by applications which do NOT have

Robust Closed Loop Fuel installed.

This module was split from the HEGO  input  processing  module  in  order  to

install hooks into the fixed sampling rate process used by Robust Closed Loop

Fuel  (RCLF)  applications.   The  purpose  of  this  module  is   keep   the

ego_input_com10  module  common for RCLF and non-RCLF applications until RCLF

becomes installed across the board.

DEFINITIONS

Registers:

-  bkgnd_rate_volts = Local parameter used only by this process.

Calibration Constants:

11-180

EGO MONITOR CONTROL STRATEGY, HEGO INPUT TRANSFER FUNCTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EGO_INPUT_XFER_COM1

BEGIN:  background_ego_sampling(bank,counts)

;

;This process should be implemented like a function, and

;return the Upstream ego sensor voltage to the calling

;process.  The ego voltage is calculated from the "counts"

;input parameter using the A/D counts to volts transfer

;function for ML-II hardware.

;

;The parameter "bkgnd_rate_volts" is used locally to this

;function and should be defined in software (i.e. size,

;bin point resolution, signed) the same as the HEGO voltage

;parameter SVEGO_RCLFn1.

;

;The "banks" input parameter is not used by this function

;but is maintained for a common calling sequence with the

;non-RCLF function.

;

unconditionally -------------------| bkgnd_rate_volts :=

|      ego_hw_xfer_fn(counts)

| Return bkgnd_rate_volts

END: background_ego_sampling(bank,counts)

11-181

EGO MONITOR CONTROL STRATEGY, HEGO LAMBSE CLIP STUB - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

11.5  HEGO LAMBSE CLIP STUB (CDAN0)

OVERVIEW

Set the lambse  clip  parameters  LAMMINn  and  LAMMAXn  to  the  calibration

constants LAMMIN and LAMMAX.  This module is used only for applications which

do NOT yet support the adaptive lambse calculation.

For applications which DO have the new algorithm, use  the  COM2  version  of

this module.

The ego_lam_clip_comx modules are intended to allow  common  strategy  to  be

used  independent of the lambse clip logic.  This module, and the call to it,

can be absorbed back into the main strategy body after  the  adaptive  lambse

clip algorithm is installed across the board.

DEFINITIONS

Registers:

-  LAMMINn = Lower lambse clip calculated for bank n.

-  LAMMAXn = Upper lambse clip calculated for bank n.

Calibration Constants:

-  LAMMIN = Lower fixed lambse clip.

-  LAMMAX = Upper fixed lambse clip.

PROCESS

STRATEGY MODULE: EGO_LAM_CLIP_COM1

BEGIN:  ego_lambse_clips

;

; Execute this process only when called.  Set the lambse

; clips to the calibratable limits, because the adaptive

; lambse clip algorithm is not yet installed.

;

unconditionally -------------------| LAMMIN[1] := LAMMIN

| LAMMAX[1] := LAMMAX

| LAMMIN[2] := LAMMIN

| LAMMAX[2] := LAMMAX

END: ego_lambse_clips

11-182

EGO MONITOR CONTROL STRATEGY, REV MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

11.6  EGO MONITOR CONTROL STRATEGY (CDAM0)

OVERVIEW

+------------------------+

|   EGO_REV_COUNTER_COM  |

+------------------------+

| PUBLIC PROCESSES:      |

| . rev_counting_process |

+------------------------+

This module is used to count  engine  revolutions  during  the  upstream  EGO

monitor  intrusive fuel control.  The revolutions counted will act as a delay

to allow stabilization of the fuel controller  prior  to  measuring  the  EGO

sensor output voltage amplitude.

DEFINITIONS

Registers:

-  EGO_REV_CTR = Count of the number of revolutions of the engine that  have

occured since starting the rev cntg proc.

-  EGO_SYN_CTR = Captured value  of  SYNC_CTR  at  the  PIP  high  interrupt

following the point when EGO_REV_FLG is set.

-  SYNC_CTR = Sequence counter incremented every PIP  rising  edge  that  is

used to indentify the cycle and position.

Bit Flags:

-  EGO_REV_ENA =  Flag  used  to  enable  the  rev_counting_process  in  the

foreground.

-  EGO_REV_FLG = Flag used to trigger the capture of  the  SYNC_CTR  in  the

foreground rev_counting_process.

11-183

EGO MONITOR CONTROL STRATEGY, REV MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EGO_REV_COUNTER_COM1

BEGIN: rev_counting_process

;This process is triggered from the background

;ego_monitor_controller.  It is used to count revolutions of the engine

;used to delay entry into the ego monitor procedures.  This procedure is

;executed during the PIP high interrupt.

EGO_REV_FLG = 1 ------------------|

|AND -| EGO_SYN_CTR := SYNC_CTR

EGO_REV_ENA = 1 ------------------|     |  ;capture starting point.

EGO_REV_FLG = 0 ------------------|

|

SYNC_CTR = EGO_SYN_CTR -----------|AND -| EGO_REV_CTR := EGO_REV_CTR + 1

|

EGO_REV_ENA = 1 ------------------|

EGO_REV_FLG = 1 ------------------|

|AND -| EGO_REV_FLG := 0

EGO_REV_ENA = 1 ------------------|     |  ;stop capturing the starting

|  ;point.

END: rev_counting_process

11-184

EGO MONITOR CONTROL STRATEGY, UPSTREAM SENSOR SWITCHPOINT CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

11.7  UPSTREAM SENSOR SWITCHPOINT CALCULATION (CDAM0)

OVERVIEW

+-----------------------------------+

| EGO_US_SWTPOINT_COM               |

+-----------------------------------+

|PUBLIC PROCEDURES:                 |

| .update_fixed_switchpoint         |

+-----------------------------------+

This module calculates  an  offset  voltage  for  the  upstream  HEGO  sensor

switchpoint,  from  the  normal stoichiometric voltage US_SWTPOINT calibrated

for gasoline fuel type applications, due to methanol  content  or  compressed

natural gas fuels.

This module was split from the HEGO  input  processing  module  in  order  to

remove  this  hardware  dependent  logic.   This  allows the input processing

module to be common all applications regardless of fuel type.

DEFINITIONS

Registers:

-  US_OFFSET = Offset voltage for the upstream HEGO sensor switchpoint, from

the  calibrated  stoichiometric  voltage  US_SWTPOINT,  due  to  methanol

content or compressed natural gas fuels.

Bit Flags:

Calibration Constants:

PROCESS

STRATEGY MODULE: EGO_US_SWTPOINT_COM1

BEGIN: update_fixed_switchpoint

;

;The process is a stub, used only to maintain a common software

;module for ego input processing.

;

;This module supports gasoline fuel type applications.

;

;Gasoline applications use the cal constant US_SWTPOINT for the

;upstream sensor switchpoint.  No offset voltage from this

;calibrated stoichiometry value, due to methanol content or

;compressed natural gas fuels, is required.

;

unconditionally ----| US_OFFSET := 0

|  ;no offset from calibrated stoich needed.

END: update_fixed_switchpoint

11-185

EGO MONITOR CONTROL STRATEGY, HEGO BYPLES CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

11.8  HEGO BYPLES CALCULATION (CDAN0)

OVERVIEW

Calculate the HEGO Bypass Lack of Switching (byples) time  constant  for  the

Upstream  HEGOs.   This  time constant is tied to the transport delay for the

engine bank, and is used to set the  time  threshold  for  which  lambse  can

remain at a clip before flagging a lack of switching fault condition.

This module is only used for applications which do  not  have  Robust  Closed

Loop  Fuel (RCLF).  For applications which do have RCFL, use the COM2 version

of this module.

The ego_byples_comx modules allow the ego_input_com10 module to remain common

for both RCLF and non-RCLF applications.

DEFINITIONS

Registers:

-  BIAS_Gn = Closed loop fuel bias Bank n.

-  LOAD = Universal normalized load parameter.

-  N = Engine RPM.

-  PTPAMPn = Limit cycle peak-to-peak amplitude for bank n.

-  rpm = Local parameter used only by this function.

-  t_byples = Local parameter used only by this function.

Calibration Constants:

-  FN1351 = System transport lag time table.

-  FN346 = Expected number of PIPs multiplier.  Input  =  |BIAS/PTPAMP|.   O

Output = Multiplier.

-  TDREV_DELAY = Number of transport delays to  wait  prior  to  going  open

loop.

11-186

EGO MONITOR CONTROL STRATEGY, HEGO BYPLES CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EGO_BYPLES_COM1

BEGIN:  update_byples(bank)

;

; Byples time calculation function for the input engine bank parameter.

;

unconditionally -------------------| rpm = max(60,N)

| ;clip value to 60rpm as a minimum

|

| t_byples = FN1351(N,LOAD) *

|            FN346(|BIAS_G[bank]|/

|                         PTPAMP[bank]) *

|            TDREV_DELAY * 60 / rpm

| ;calculate time delay to declare

| ;a HEGO failure (units = seconds)

;

; This process should be implemented like a function, and return the

; value of the "t_byples" parameter.  This returned parameter should

; be defined in software (i.e. size, bin point resolution, signed)

; the same as the ego_input_com10 parameters BYPLES_TMn.

;

unconditionally -------------------| Return t_byples

END: update_byples(bank)

11-187

EGO MONITOR CONTROL STRATEGY, HEGO BYPLES CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

11-188

CHAPTER 12

EGR STRATEGY

12-1

EGR STRATEGY

12.1  FEATURE:  EGR - PFE - V1.11_EGR_PFE (CDAN0)

12.1.1  EGR SYSTEM INITIALIZATION (CDAN0)

OVERVIEW

This module contains initialization logic for the  PFE  EGR  control  system.

EGR parameters will be initialized whenever EGR_INIT_FLG is set to zero.

DEFINITIONS

Registers:

-  EGR_SHT_TMR = EGR shut timer.

-  EPTBAR = Filtered EGR sensor reading, counts.

-  EPTZER = Learned EGR valve position sensor zero reading, counts.

-  EPTZER_TMR2 = Time since EPTZER updated.

-  IEGR = Unfiltered EPT sensor analog channel reading.

Bit Flags:

-  EGR_INIT_FLG = Flag to indicate EGR parameters have been initialized.

-  EGR_RETRY = Flag to indicate EGR monitor is on second pass

Calibration Constants:

-  EGR_REP_SW = Enables EGR fault retesting.  1  ->  retest  alwalys,  2  ->

retest after first pass.

12-2

EGR STRATEGY, PRESSURE FEEDBACK EGR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EGR_INIT_COM1

The following logic initializes EGR parameters after a RAM clear.

EGR_INIT_FLG = 0 -----------------|

;initialization in progress      |AND -| EGR_RETRY = 1

|     |  ;set second pass flag

EGR_REP_SW = 2 -------------------|     |

;do not retest on first pass           |

| --- ELSE ---

|

EGR_INIT_FLG = 0 -----------------------| EGR_RETRY = 0

;initialization in progress            |  ;start out on first pass

EGR_INIT_FLG = 0 -----------------------| EPTBAR = EPTZER

|  ;initialize to closed position

| EPTZER_TMR2 = 255

|  ;set time since EPTZER update

|  ;to max

| IEGR = 307

|  ;set to intermediate value

| EGR_SHT_TMR = 31.875

|  ;initialize to close valve value

| EGR_INIT_FLG = 1

|  ;initialization complete

12-3

EGR STRATEGY, EGR ENABLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

12.1.2  EGR ENABLE (CDAL0)

OVERVIEW

This module enables or disabled the PFE EGR system based on engine  operating

conditions.

DEFINITIONS

Registers:

-  ACSTRT = Air Charge temperature at start.

-  APT = Throttle Mode Flag; -1 -> Closed Throttle, 1 -> Wide Open Throttle,

0 -> Part Throttle.

-  EGRENA_TM = Time since start at which EGR is enabled, sec.

-  EGREN_TMR = EGR Enable timer.

-  EGR_WOT_TMR = Time at WOT for EGR enable, seconds.

-  N = Engine speed, rpm.

-  TCSTRT = Temperature of ECT at cold start, deg F.

-  TP_REL = Throttle position relative to closed position.

Bit Flags:

-  AFMFLG = Flag indicating that ACT sensor has failed.

-  BFMFLG = Flag indicating that BP sensor has failed; 1 -> failure.

-  CFMFLG = Flag indicating that ECT sensor is in/out of range.

-  CRKFLG = Flag indicating engine status; 1 -> Crank mode.

-  DIS_FMEM = Distributorless ignition failure mode.

-  EFMFLG = Flag indicating that EVP EGR sensor has failed.

-  EGREN = Flag which indicates EGR enabled if set to 1.

-  EGR_HOSE_TST = Flag to Enable EGR OBDII Hose Integrity checks.

-  EGR_TPCUTOUT = Disable EGR based on throttle position flag.

-  ER_EGR_FLG = Engine Running Self Test requests EGR.

-  MFMFLG = Flag indicating that MAF sensor has failed.

-  TFMFLG = Flag indicating that TP sensor has failed.

12-4

EGR STRATEGY, EGR ENABLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  EGREN_TM_RT = ACT at start weighting in FN230A input.

-  EGR_DYNO_ENA = EGR unconditional enable switch.

-  EGR_DYNO_DIS = EGR unconditional disable switch.

-  EGR_TP_HYST = EGR TP cutout hysteresis.

-  FN203(N) = TP_REL above which to disable EGR.

-  FN230A(TCSTRT) = Time since start to enable EGR, sec.

-  PFEHP = Switch to select EGR strategy; 0 -> use Sonic strategy, 1 ->  use

PFE strategy, 2 -> do not use any EGR.

-  WOTEGRTM = Maximum amount of time to allow WOT EGR.

12-5

EGR STRATEGY, EGR ENABLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EGR_ENABLE_COM2

CRKFLG = 1 -----------------------------| EGREN_TMR = 0

;in crank mode                         |  ;reset time since start

always ---------------------------------| EGRENA_TM = FN230A(ACSTRT *

|     EGREN_TM_RT + TCSTRT *

|     (1 - EGREN_TM_RT))

|  ;calc enable time as fn of

|  ;ECT and ACT at start

TP_REL > FN203(N) + EGR_TP_HYST --------|S Q -| EGR_TPCUTOUT

;throttle above cutout threshold       |     |  ;EGR cutout flag

|

TP_REL <= FN203(N) ---------------------|C

;throttle below cutout threshold

;EGR_WOT_TMR is a free running up counting timer.  It is reset to zero

;when not at WOT.

APT <> 1 -------------------------------------| EGR_WOT_TMR = 0

;not at wide open throttle                   |  ;reset wide open throttle

|  ;timer

12-6

EGR STRATEGY, EGR ENABLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EGR_DYNO_ENA = 1 -----------------------|

;egr unconditionally enabled           |

|

EGR_DYNO_DIS = 0 -----------------|     |

;egr not disabled                |     |

|     |

PFEHP <> 2 -----------------------|     |OR --| EGREN = 1

;EGR hardware present            |     |     |  ;enable EGR

|     |     |

EGR_HOSE_TST = 0 -----------------|     |     |

;EGR hose test disabled          |     |     |

|     |     |

ER_EGR_FLG = 1 -------------|     |     |     |

;self test requests EGR    |     |AND -|     |

|     |           |

EGREN_TMR >                 |     |           |

EGRENA_TM -----|     |     |           |

;time since start    |     |     |           |

;long enough         |     |     |           |

|     |     |           |

APT <> -1 ------------|     |OR --|           |

;not closed          |     |                 |

;throttle            |     |                 |

|     |                 |

EGR_TPCUTOUT = 0 -----|     |                 |

;no throttle cutout  |     |                 |

|     |                 |

EGR_WOT_TMR <=        |     |                 |

WOTEGRTM ----|     |                 |

;not at WOT too long |     |                 |

|     |                 |

CRKFLG = 0 -----------|     |                 |

;not in crank mode   |     |                 |

|     |                 |

AFMFLG = 0 -----------|     |                 |

;air temp sensor o.k.|     |                 |

|AND -|                 |

BFMFLG = 0 -----------|                       |

;BP sensor O.K.      |                       |

|                       |

CFMFLG = 0 -----------|                       |

;ect sensor o.k.     |                       |

|                       |

EFMFLG = 0 -----------|                       |

;EGR sensor o.k.     |                       |

|                       |

MFMFLG = 0 -----------|                       |

;mass flow meter o.k.|                       |

|                       |

TFMFLG = 0 -----------|                       |

;tp sensor o.k.      |                       |

|                       |

DIS_FMEM = 0 ---------|                       |

;dis o.k.                                    |

| --- ELSE ---

|

| EGREN = 0

|  ;disable EGR

12-7

EGR STRATEGY, DESIRED EGR MASS FLOW - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

12.1.3  DESIRED EGR MASS FLOW (CDAN0)

OVERVIEW

This module calculates the desired egr flow rate, EGRATE, in percent of total

engine  mass  flow  rate  and  desired egr mass flow rate, DESEM, in lbs/min.

Desired EGR flow is calculated as a  function  engine  speed,  load,  coolant

temperature, air charge temperature, and percent load.

DEFINITIONS

Registers:

-  ACT = Air charge temperature degrees Farenheit.

-  AM = Air mass flow, lb/min.

-  DESEM = Desired EM, lb/min.

-  ECT = Coolant temperature degrees Farenheit.

-  EGRATE = Desired EGR rate, percent.

-  LOAD = Universal load at ratio of air charge over standard.

-  N = Engine speed rpm.

-  PCT_LOAD = Percent of maximum load attainable for current conditions.

-  TCSTRT = Temperature of ECT at cold start, deg F.

-  TSEGRE = Accumulated time EGR is enabled.

Bit Flags:

-  CAT_TST_PRG = Catalyst Test in Progress flag, indicates that closed  loop

fuel control is turned over to downstream HEGO sensors; 1 -> control from

downstream HEGO sensors.

-  EGREN = Flag which indicates EGR enabled if set to 1.

-  ER_EGR_FLG = Engine Running Self Test requests EGR.

-  TSEGRE_FLG = Flag to enable counting up of timer TSEGRE.

Calibration Constants:

-  CTLOW = Cold Start maximum ECT, deg F.

-  CAT_EGRATE = Desired EGR flow rate during catalyst test, percent.

-  EGR_ADD = Constant EGR adder.

-  EGR_MUL = EGRATE multiplier for development.

12-8

EGR STRATEGY, DESIRED EGR MASS FLOW - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  EGRMPT = Calibration time delay to ramp EGR in, sec.  (EGRATE  ramp  time

for TCSTRT <= CTLOW).

-  ER_EGRATE = Engine running requested EGRATE.

-  FN070(N) = N normalizing function for table lookup.

-  FN071(LOAD) = Load normalizing function for table lookup.

-  FN211(ECT) = Multiplier as a function of Engine Coolant Temperature ECT.

-  FN212A(BP) = Multiplier as a function of BP for desired EGR rate.

-  FN217A(BP) = Multiplier as a function of BP for desired EGR rate.

-  FN220(ACT) = Multiplier as a function of Air Charge Temperature.

-  FN248(PCT_LOAD) = Maximum EGR attainable at altitude.

-  FN908A(N,LOAD) = Base desired EGR rate at sea level in percent;

X = FN070(N) - Normalized engine speed, RPM

Y = FN071(LOAD) - Normalized Load.

-  FN908B(N,LOAD) = Base desired EGR rate at altitude in percent;

X = FN070(N) - Normalized engine speed, RPM

Y = FN071(LOAD) - Normalized Load.

12-9

EGR STRATEGY, DESIRED EGR MASS FLOW - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EGR_DESIRED_COM4

BEGIN:  egr_desired

Background process to calculate the desired EGR flow rate.

TIME SINCE EGR ENABLED LOGIC

TCSTRT > CTLOW ------------| TSEGRE = EGRMPT

;coolant temp at start    |  ;make TSEGRE/EGRMPT = 1

;high                     |

| --- ELSE ---

|

EGREN = 1 -----------------| TSEGRE_FLG = 1

;EGR enabled              |  ;count up timer TSEGRE

|  ;ramp TSEGRE/EGRMPT multiplier to 1

| TSEGRE = min(TSEGRE,EGRMPT)

|  ;clip multiplier to 1

|

| --- ELSE ---

|

| TSEGRE_FLG = 0

|  ;freeze TSEGRE timer

FINAL DESIRED EGR RATE CALCULATION

The base desired egr rate is calculated as a function of engine speed,  load,

coolant  temperature,  air  charge temperature, and time since first enabled.

It is then clipped to the maximum EGR attainable due  to  limited  vacuum  at

altitude as a function of PCT_LOAD.

EGREN = 0 -----------------| EGRATE = 0

;egr disabled             |  ;do not flow EGR

|

| --- ELSE ---

|

ER_EGR_FLG = 1 ------------| EGRATE = ER_EGRATE

;in KOER                  |  ;set to constant percent egr

|

| --- ELSE ---

|

| Do: egr_rate_lookup

|  ;get desired EGR rate

12-10

EGR STRATEGY, DESIRED EGR MASS FLOW - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DESIRED EGR MASS FLOW RATE CALCULATION

Convert desired EGR rate to desired EGR mass  flow  rate,  DESEM,  which  has

units of lbm/min.  The actual definition of DESEM is ( EGRATE * AMPEM / 100).

It is not formulated this way in the strategy because  the  addition  of  EGR

changes  AMPEM  which  affects  DESEM.  In other words this implementation is

iterative and causes a delay in achieving the  final  value  of  DESEM.   The

relationship is reformulated to give the correct value directly:

always --------------------| DESEM = EGRATE * AM / ( 100 - EGRATE )

|  ;calc. desired egr mass

END:  egr_desired

BEGIN:  egr_rate_lookup

Local procedure to perform the table lookups and  calculations  necessary  to

determine the desired EGR rate.

unconditionally -----------| egrate_tmp = FN211(ECT) * FN220(ACT) *

|              (FN908A(N,LOAD) * FN212A(BP) +

|               FN908B(N,LOAD) * FN217A(BP)) *

|              (TSEGRE / EGRMPT)

|  ;lookup desired EGR rate

| egrate_tmp = min(egrate_tmp,FN248(PCT_LOAD))

|  ;clip to max attainable given vacuum

|  ;constraints

EGR_IMR_MOD = 1 -----------| egrate_tmp = egrate_tmp * EGR_IMR_MUL

;IMRC requests EGR        |  ;modify desired EGR for IMRC

;modification

unconditionally -----------| EGRATE = (EGR_MUL * egrate_tmp) + EGR_ADD

|  ;apply development multipliers

END:  egr_rate_lookup

12-11

EGR STRATEGY, DELTA PRESSURE FEEDBACK EGR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

12.1.4  DELTA PRESSURE FEEDBACK EGR (CDAN0)

OVERVIEW

DPFE EGR precisely regulates the EGR flow according to calibrated tables  and

functions.   The  DPFE  EGR  strategy  uses  delta pressure across an orifice

measured using an EGR pressure transducer (EPT) as the  feedback  signal  for

control  of EGR.  The EGR flow is proportional to square root of the pressure

drop across the sharp-edged orifice.  The EGR valve itself  operates  as  the

downstream  pressure regulator.  When the valve is closed, the downstream and

upstream pressures are equal.  As the valve opens,  the  downstream  pressure

decreases due to the influence of the intake manifold vacuum.

|------------------------------|

|    Exhaust Pressure          |

| |--------------------------| |

| |                      |---| |---|     EPT

| |                      |         |<-- Sensor

| |      Control         |---------|           EGR

| |      Orifice         |         |          Valve

| |         |            |---| |---|            |

| |         V                | |                V

---| |--------------------------| |----------------------

|                                 \    /

|                                  \  /      Intake

Exhaust                                             \/      Manifold

Manifold                                            /\       Vacuum

|          Control chamber         /  \

|                                 /    \

---------------------------------------------------------

The EEC controls the EGR  valve  with  vacuum  supplied  by  the  EGR  Vacuum

Regulator  (EVR).  The strategy in the EEC accomplishes this by calculating a

desired pressure in the control chamber called CONPR.  The actual pressure is

measured  and  is called EPTBAR.  The EVR controller uses EGR_ERROR, which is

the difference between CONPR and EPTBAR, to modulate the EVR  valve  so  that

EPTBAR is as close as possible to CONPR.

12-12

EGR STRATEGY, DELTA PRESSURE FEEDBACK EGR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  AM = Air mass flow, lb/min.

-  APT = Throttle Mode Flag; -1 -> Closed Throttle, 1 -> Wide Open Throttle,

0 -> Part Throttle.

-  BP = Barometric pressure, " Hg.

-  BP_WORD = Word resolution value of BP, " Hg.

-  CCM_ER_ENA = Engine running on demand test for CCM enabled.

-  CONPR = Desired EPT sensor value, counts.

-  DELPR = Actual pressure drop across the control orifice, " H20.

-  DESDP = Filtered desired downstream pressure, " H2O.

-  DESEM = Desired EM, lb/min.

-  DSDRPM = Desired RPM as defined in ISC section.

-  EGREN_TMR = EGR enable timer.

-  EGRENA_TM = Time since start at which EGR is enabled.

-  EGR_ERROR = Error in control chamber pressure  in  PFE  systems  and  EGR

valve position in sonic systems, counts.

-  EGR_N_FIL = Filtered engine speed for EGR logic.

-  EGR_SHT_TMR = Unconditionally up counting timer to indicate elapsed  time

since EGR valve was commanded shut.

-  EM = Actual EGR mass flow, lb/min.

-  EPTBAR = Rolling average of the synchronously sampled  EPT  sensor  (time

constant = TCEPT), counts.

-  EPTZER = Rolling average of the synchronously sampled EPT sensor at  idle

(time constant = TCEPTZER), stored in Keep Alive Memory, counts.

-  EPTZER_OFF = EPTZER when engine is not running.

-  EPTZER_TMR2 = Timer  to  record  time  since  EPTZER  was  last  updated,

seconds.

-  IEGR = EPT sensor data for PFE EGR.

-  N = Engine speed, rpm.

12-13

EGR STRATEGY, DELTA PRESSURE FEEDBACK EGR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Bit Flags:

-  CRKFLG = Flag indicating engine status; 1 -> Crank mode.

-  DESEM_FLG = When set, DESEM is large enough to turn EGR on.   This  is  a

flip flop flag.

-  EGR_ON_FLG = When set, the EVR is being controlled and when  clear  there

is no duty cycle output to the EVR.

-  EGREN = Flag which indicates EGR enabled if set to 1.

-  EFMFLG = Flag indicating that EVP EGR sensor has failed.

-  EPT_OK_FLG = When set, the EPT sensor is not saturated.

-  EVLOPNFM_FLG = EGR valve stuck open failure mode flag.

-  FIRST_PIP = Indicates that first PIP has been received.

-  KAM_ERROR = Indicates keep alive RAM invalid.

-  PxxxFLT = Flag to indicate fault xxx exists; 1 ->  malfunction  currently

exists for fault xxx.

-  WRM_IDL_FLG = Warm idle flag used for EGR.

Calibration Constants:

-  DP_MAX = Maximum clip for DESDP and dp.

-  DP_MIN = Minimum clip for DESDP and dp.

-  EGR_VLV_SHT = Time to allow EGR valve to shut when commanded.

-  EGR_WRM_TM = Time before EGR is first enabled to allow EPTZER to update.

-  EPTBAR_CH = Above this value, the EPT sensor is saturated, counts.

-  EPTBAR_SL = Below this value, the EPT sensor is not saturated, counts.

-  EPTZER_IV = Initial or default value for EPTZER and EPTBAR.

-  EPTZER_UPTOL = Tolerence on EPTZER required before it's value is said  to

be good.

-  FN246A(DELPR) = EGR mass flow as a function of DELPR, lb/min.

-  FN247A(DESEM) = Desired pressure drop, DESDP, across the PFE orifice.

-  IERPMH = Idle RPM hysteresis term for EPTZER update.

-  IXFRPR = Transfer function = 1/XFREPT, Counts/" H2O.

-  MINDES_CL = The desired EGR mass, DESEM, below which the  EGR  is  turned

off because the flow is too low to deliver accurately.

12-14

EGR STRATEGY, DELTA PRESSURE FEEDBACK EGR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  MINDES_SH = The desired EGR mass, DESEM, above which the EGR is turned on

because the flow is high enough to deliver accurately.

-  TCDELPR = Time constant for filtering DELPR, sec.

-  TCDP = dp filter time constant.

-  TCEPT = Time constant for EPT, sec.

-  TCEPTZER = Time constant for filtering EPTZER, sec.

-  TCEPTZER_OFF = Time constant for engine off eptzer reading.

-  XFREPT = Transfer function of EPT sensor, " H20/counts.

OTHER

-  egr_codes = Set of (P1400, P1401).  The set of OBD-II  fault  codes  that

relate to the EGR monitor.

-  P1400 = Fault code for EVP/EPT circuit below minimum voltage.

-  P1401 = Fault code for EVP/EPT circuit above maximum voltage.

12-15

EGR STRATEGY, DELTA PRESSURE FEEDBACK EGR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EGR_PFE_COM3

CHECK EPTZER UPDATE CONDITIONS AND UPDATE EPTZER

always ---------------------------------| EGR_N_FIL = ROLAV(N,TC_EGR_N)

|  ;filter N for warm idle detection

CRKFLG = 0 -----------------------|

;not in crank mode               |

|

APT = -1 -------------------------|

;closed throttle                 |

|

EGR_N_FIL < DSDRPM + IERPMH ------|AND -| WRM_IDL_FLG = 1

;filtered engine speed near idle |     |  ;warm idle conditions met

|     |

EGREN_TMR >= EGRENA_TM -          |     |

EGR_WRM_TM -------------|     |     |

;EGR valve warm            |OR --|     |

|     |     |

CCM_ER_ENA = 1 -------------|     |     |

;in KOER                         |     |

|     |

EGR_SHT_TMR > EGR_VLV_SHT --------|     |

;EGR valve is closed at idle           |

| --- ELSE ---

|

| WRM_IDL_FLG = 0

|  ;warm idle conditions not met

KAM_ERROR = 1 --------------------|

;keep alive memory failure       |

|

P1400FLT = 1 ---------------------|OR --| EPTZER = EPTZER_IV

;EPT sensor failed low           |     |  ;initialize to default value

|     |

P1401FLT = 1 ---------------------|     |

;EPT sensor failed high                |

| --- ELSE ---

|

EVLOPNFM_FLG = 1 -----------------------| EPTZER = EPTZER_OFF

;EGR valve failed open                 |  ;set to engine off reading

|

| --- ELSE ---

|

WRM_IDL_FLG = 1 ------------------------| EPTZER = ROLAV(IEGR,TCEPTZER)

;engine at warm idle                   |  ;update closed valve reading

12-16

EGR STRATEGY, DELTA PRESSURE FEEDBACK EGR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TIME SINCE EPTZER UPDATE CALCULATION

EPTZER_TMR2 is an actuated  up-counting  timer  actuated  by  the  condition:

WRM_IDL_FLG = 0.  When WRM_IDL_FLG = 1, timer is held at it's current value.

EPTZER > EPTBAR - EPTZER_UPTOL ---|

;EPTZER above min                |

|

EPTZER < EPTBAR + EPTZER_UPTOL ---|AND -| EPTZER_TMR2 = 0

;EPTZER below min                |     |  ;indicate short time since EPTZER

|     |  ;update

WRM_IDL_FLG = 1 ------------------|

;EPTZER update conditions met

ENGINE OFF EPTZER CALCULATION

KAM_ERROR = 1 --------------------|

;keep alive memory failure       |

|

P1400FLT = 1 ---------------------|OR --| EPTZER_OFF = EPTZER_IV

;EPT sensor failed low           |     |  ;initialize to default value

|     |

P1401FLT = 1 ---------------------|     |

;EPT sensor failied high               |

| --- ELSE ---

CRKFLG = 1 -----------------------|     |

;in CRANK mode                   |AND -| EPTZER_OFF =

|     |     ROLAV(IEGR,TCEPTZER_OFF)

FIRST_PIP = 0 --------------------|     |  ;engine off EPTZER update

;engine has not rotated

EGR PRESSURE CALCULATION

FIRST_PIP = 0 --------------------|

;engine has not rotated          |OR --| EPTBAR = ROLAV(IEGR,TCEPT)

|     |  ;get filtered valve position

CRKFLG = 0 -----------------------|

;not in crank mode

12-17

EGR STRATEGY, DELTA PRESSURE FEEDBACK EGR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EPT SENSOR SATURATION TEST

EPTBAR < EPTBAR_SL ---------------|S Q -| EPT_OK_FLG

;EPT sensor is not saturated     |     |  ;sensor not saturated flag

|

EPTBAR > EPTBAR_CH ---------------|C

;EPT sensor is saturated

CHECK FOR GREATER THAN MINIMUM EGR REQUEST

DESEM >= MINDES_SH ---------------|S Q -| DESEM_FLG

;more than minimum requested     |

|

DESEM < MINDES_CL ----------------|C

EGR operation is disabled via EGR_ON_FLG if the desired EGR is too low.

EGR_SHT_TMR is an unconditionally up-counting timer to indicate how long EGR

has been disabled.

DESEM_FLG = 1 --------------------|

;more than min EGR requested     |

|

EGREN = 1 ------------------------|AND -| EGR_ON_FLG = 1

;EGR enabled                     |     |  ;OK to turn on EGR

|     | EGR_SHT_TMR = 0

EPT_OK_FLG = 1 -------------------|     |  ;indicate EGR valve open

;EPT sensor is not saturated           |

| --- ELSE ---

|

| EGR_ON_FLG = 0

|  ;turn off EGR

DELTA PRESSURE DETERMINATION

unconditionally ------------------------| delpr = clip((XFREPT *

|              (EPTBAR - EPTZER)),

|              DP_MIN, DP_MAX)

|  ;calculate delta pressure and

|  ;clip

| DELPR = ROLAV(-delpr, TCDELPR)

|  ;filter delta pressure

12-18

EGR STRATEGY, DELTA PRESSURE FEEDBACK EGR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Now that delta pressure across the orifice is known, actual EGR mass flow can

be  computed.   The  computed  EGR  mass flow rate is set to zero under known

closed valve conditions to prevent exhaust pulsations and sensor  drift  from

causing false readings.

Note that the equation for EGR mass is of the form:

dm/dt = K * sqrt(delpr * density)

where  density  =  K2  *  pressure/temperature.   Data  show  P/T   to   vary

proportional to barometric pressure.  Hence,

dm/dt = sqrt(K3 * delpr) * (sqrt(BP/standard BP)).

FN246A is the sqrt(K3 * x) function developed at standard pressure.

Desired pressure is calculated later in the strategy by solving for delpr  in

the above equation:

delpr = ((dm/dt)**2 * (standard BP/BP)) / K3

FN247A is the (x**2/K3) function developed at standard pressure.

EFMFLG = 1 -----------------------|

;EGR sensor failed               |

|

EGR_SHT_TMR > EGR_VLV_SHT --------|OR --| EM = 0

;EGR valve is closed             |     |  ;set EGR mass flow rate to zero

|     |

DELPR <= 0 -----------------------|     |

;negative delta pressure               | --- ELSE ---

|

| em = FN246A(DELPR) *

|      SQRT(BP_WORD / 29.875)

|  ;calculate EGR mass flow rate

| EM = min(em,1.99)

|  ;clip to max

Knowing the desired EGR mass flow through the  valve,  the  desired  pressure

drop across the orifice can be calculated.

EGR_ON_FLG = 1 -------------------------| desdel = clip((-FN247A(DESEM) *

;ok to turn on EGR                     |                29.875 / BP),

|               DP_MIN, DP_MAX)

|  ;calculate desired delta pressure

|

| --- ELSE ---

|

| desdel = 0

|  ;set desired delta pressure to 0

12-19

EGR STRATEGY, DELTA PRESSURE FEEDBACK EGR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Filter desired delta pressure and convert to counts.

always ---------------------------------| DESDP = ROLAV(desdel, TCDP)

|  ;filter desired pressure

| CONPR = IXFRPR * DESDP + EPTZER

|  ;convert to counts

Calculate EGR_ERROR

EGR_ON_FLG = 1 -------------------------| EGR_ERROR = EPTBAR - CONPR

;ok to open EGR valve                  |

| --- ELSE ---

|

| EGR_ERROR = 0

12-20

EGR STRATEGY, EVR CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

12.1.5  EVR CONTROLLER - (EGRDC CALCULATION) (CDAN0)

OVERVIEW

This module calculates EGR duty cycle using a combination of a  second  order

closed  loop  controller and an open loop feed forward prediction of the duty

cycle required to start to open the EGR valve.

DEFINITIONS

INPUTS

Registers:

-  EGRDC = Desired EVR duty cycle, %/100.

-  EGRDC1 = Last pass value of EGRDC.

-  EGRDC2 = Last pass value of EGRDC1.

-  EGR_ERROR = Error in control chamber pressure  in  PFE  systems  and  EGR

valve position in sonic systems, counts.

-  EGR_ERROR1 = Last pass value of EGR_ERROR.

-  EGR_ERROR2 = Last pass value of EGR_ERROR1.

-  ER_EGR_FLG = Flag to indicate EGR KOER test is in progress

-  SYS_DELPR = EGR System delta pressure.

Bit Flags:

-  EGR_ON_FLG = 1 -> EVR is being controlled, 0 -> no duty cycle  output  to

the EVR.

-  EGRFLG = Flag that indicates whether EGR is in closed loop control.

-  OSM_EO_OFF = Off state  requested  for  OSM  outputs  during  engine  off

on-demand test.

-  OSM_EO_ON =  ON  state  requested  for  OSM  outputs  during  engine  off

on-demand test.

Calibration Constants:

-  COEFDC = Coefficient used in EGR control algorithm, used  to  filter  the

EGRDC calculation.

-  COEFER1 = Coefficient used  in  EGR  control  algorithm.   Its  value  is

determined through a control theory analysis.

-  COEFER2 = Coefficient used  in  EGR  control  algorithm.   Its  value  is

determined through a control theory analysis.

12-21

EGR STRATEGY, EVR CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  EGR_TST_DC = Minimum EGRDC for running of 1409 test.

-  EGRDC_MAX = Maximum clip for EGRDC, %/100.

-  EGRGAIN = Coefficient used  to  set  overall  gain  of  the  EGR  control

algorithm.

-  FN201(SYS_DELPR) = EGR Valve Start to Open Duty Cycle vs SYS_DELPR.

OUTPUTS

Registers:

-  EGRDC = See above.

-  EGRDC1 = See above.

-  EGRDC2 = See above.

-  EGR_ERROR1 = See above.

-  EGR_ERROR2 = See above.

Bit Flags:

-  EGRFLG = See above.

12-22

EGR STRATEGY, EVR CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EGR_CONTROL_COM2

OSM_EO_ON = 1 -----------------| egrdc = 1

;OSM test commanding output   |  ;command maximum output

;on                           |

| --- ELSE ---

EGR_ON_FLG = 0 ----------|     |

;DESEM below minimum    |OR --| egrdc = 0

|     |  ;command minimum output

OSM_EO_OFF = 1 ----------|     | EGRFLG = 0

;OSM test commanding          |  ;not in closed loop control

;output off                   | EGRDC1 = 0

| EGRDC2 = 0

|  ;reset last pass values of duty cycle

|

| --- ELSE ---

|

EGRFLG = 0 --------------------| EGR_ERROR1 = EGR_ERROR

;initial entry into closed    | EGR_ERROR2 = EGR_ERROR

;loop control                 |  ;reset last pass values of error

| egrdc = clip(COEFDC * EGRDC1 +

|         (1 - COEFDC) * EGRDC2 +

|        EGRGAIN * [EGR_ERROR -

|        (COEFER1 * EGR_ERROR1) +

|        (COEFER2 * EGR_ERROR2)],

|        -FN201(SYS_DELPR),

|        EGRDC_MAX - FN201(SYS_DELPR))

|  ;calculate control effort

| EGRDC1 = egrdc

|  ;save last pass value of duty cycle

| egrdc = egrdc + FN201(SYS_DELPR)

|  ;add in feed forward start to open

|  ;duty cycle

| EGRFLG = 1

|  ;in closed loop control

|

| --- ELSE ---

|

| egrdc = clip(COEFDC * EGRDC1 +

|         (1 - COEFDC) * EGRDC2 +

|        EGRGAIN * [EGR_ERROR -

|        (COEFER1 * EGR_ERROR1) +

|        (COEFER2 * EGR_ERROR2)],

|        -FN201(SYS_DELPR),

|        EGRDC_MAX - FN201(SYS_DELPR))

|  ;calculate control effort

| EGR_ERROR2 = EGR_ERROR1

| EGR_ERROR1 = EGR_ERROR

|  ;last pass values of error

| EGRDC2 = EGRDC1

| EGRDC1 = egrdc

|  ;last pass values of duty cycle

| egrdc = egrdc + FN201(SYS_DELPR)

|  ;add in feed forward start to open

|  ;duty cycle

12-23

EGR STRATEGY, EVR CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

ER_EGR_FLG = 1 ----------------| egrdc = max(egrdc,EGR_TST_DC)

;in KOER                      |  ;clip to minimum

always ------------------------| Do: substitute(7,egrdc)

|  ;output state control override

| EGRDC = egrdc

12-24

EGR STRATEGY, ACTUAL PERCENT EGR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

12.1.6  ACTUAL PERCENT EGR (CDAA0)

OVERVIEW

EGRACT is used to modify spark advance (see  SAF  calculation).   To  prevent

large instantaneous changes in calculated spark, EGRACT is filtered using the

rolling average filter routine.

DEFINITIONS

INPUTS

Registers:

-  AM = Air mass flow bin 10.

-  AMPEM = Air mass flow plus EGR mass flow, lb/min.

-  EM = Actual EGR mass flow, lb/min.

Calibration Constants:

-  TCEACT = Time constant for filtering EGRACT, sec.

OUTPUTS

Registers:

-  EGRACT = Actual EGR percent = 100 * EM / AMPEM.

PROCESS

STRATEGY MODULE: EGR_ACTUAL_COM1

Calculate AMPEM:

AMPEM = AM + EM

Calculate the actual EGR rate as a percent of AMPEM:

egract = (EM / AMPEM) * 100

EGRACT = ROLAV(egract,TCEACT)

12-25

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

12.1.7  EGR OBDII TEST LOGIC (CDAN0)

OVERVIEW

This strategy module contains the OBDII tests that  are  carried  out  during

normal  engine  operation  and  during on-demand testing on the delta PFE EGR

system.  It sets up flags to light the MIL light and also disables EGR system

operation if necessary.

The tests in this monitor are run in a predefined  sequence  to  prevent  the

setting  of multiple codes for a single failure.  While some tests are run on

a "continuous" basis, no test is allowed to run until it's predecessors  have

been  run.   If  a  failure  occurs,  previous  tests are allowed to continue

running but subsequent tests are inhibited until the fault is cleared.

The following tests are run during various phases of the EGR monitor:

Code    Description

----    -----------

P0401   Insufficient EGR flow

P0402   EGR valve stuck open

P1400   EPT sensor signal out of range low

P1401   EPT sensor signal out of range high

P1405   EPT upstream hose disconnected or plugged

P1406   EPT downstream hose disconnected or plugged

P1408   On-demand insufficient flow

P1409   Output driver fault

During normal engine operation the following test sequence is performed:

P1409     P1400     P1401

|         |         |

|         -----|-----

|              |

|            P0402

|              |

|         -----------

|         |         |

|       P1405     P1406

|         |         |

---------------------

|

P0401

After successful completion of the P0401 test the executive is  flagged  that

the EGR monitor has run to completion.

During  engine  off  on-demand,  the  P1409,  P1400,  and  P1401  tests   run

simultaneously.

12-26

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

During engine running on-demand, the tests run in the following order:

P1409     P1400     P1401

|         |         |

|         -----|-----

|              |

|            P0402

|              |

----------------

|

P1408

After successful completion of the P1408 test the OBDII executive is informed

that  the  EGR  monitor  on-demand  test  has  run  to  completion,  allowing

subsequent tests to run.

DEFINITIONS

Registers:

-  ACT = Air Charge Temperature, deg.  F.

-  ACT_EGR = Filtered ACT for use in EPT hose test.

-  AIR_CHG = Air charge value after the filling model.

-  AIR_CHG_CUR = Air charge mass inducted per intake  stroke  corrected  for

backflow and leakage.

-  AM = Air Mass flow, bin 10.

-  ARCH_PROP_D = The rate  of  change  of  air  charge  being  inducted  any

instant.

-  BP = Barometric Pressure.  (Note:  Upper byte of BP_WORD.)

-  DELPR = Pressure drop across PFE orifice.

-  DELPR_FLW = Filtered DELPR during test for mode 06 reporting, in H2O.

-  DELPR_HOSE = Filtered DELPR during test for mode 06 reporting, in H2O.

-  DEMAND_TIMER = Utility timer for sequenced on demand tests.

-  EGRACT = Actual EGR rate.

-  EGRDC = EGR duty cycle.

-  EGRDC_FLW = Filtered EGRDC during test for mode 06 reporting.

-  EGRENA_TM = Time since start at which EGR is enabled, sec.

-  EGREN_TMR = EGR Enable timer.

-  EGRMON_TMR = EGR monitor timeout timer, sec.

12-27

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  EGR_OFF_TMR = Timer enabled when EGREN = 0.

-  EGR_RTST_TMR = EGR no-flow test retest timer, sec.

-  EPTBAR = Rolling average of EPT transducer

-  EPTBAR_OPN = Filtered EPTBAR during test for mode 06 reporting.

-  EPTZER = EPT sensor value at idle.

-  EPTZER_OFF = EPTZER when engine is not running.

-  EPTZER_TMR2 = Timer to record time since EPTZER was  last  updated  to  a

correct value.

-  ER_RPM = Register that contains RPM value requested during engine running

self test, rpm.

-  ER_STATUS = State pointer that indicated current state of engine  running

State  pointer  that  indicated current state of engine running on demand

test on demand test, state.

-  HOSE_TST_AMB = Value of INFAMB_KAM at start of hose tests, deg.  F.

-  INF_MVAC = Inferred Manifold Vacuum.

-  MIN_DELPR = Minimum allowed DELPR for a given SYS_DELPR at high EGRDC.

-  MIN_DELPR_FL = Filtered MIN_delpr during test for mode 06  reporting,  in

H2O.

-  N = RPM.

-  PEXHAUST = Exhaust Back Pressure, inches H2O.

-  PxxxFLT_TMR = Timer to  indicate  length  of  time  fault  xxx  has  been

present.

-  PxxxTST_TMR = Timer to  determine  length  of  time  test  xxx  has  been

running.

-  SYS_DELPR = EGR System delta pressure.

Bit Flags:

-  CCM_ER_ENA = Engine running on demand test for CCM enabled.

-  CCM_EO_ENA = Engine off on demand test for CCM enabled.

-  CHK_VPLG_FLG = OK to check for EGR Valve Plugged, for EGR OBDII test.

-  DEMAND_ABORT = Flag indicating that an on demand test is in the  progress

of a abort.

-  DEMAND_LAST = Last pass value of DEMAND_MODE.

-  DEMAND_MODE = Flag set when an On-Demand self test mode is active.

12-28

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  EFMFLG = EGR FMEM flag.

-  EGR_EFM_FLG = Flag to disable retry based failure mode.

-  EGREN = EGR enabled flag.

-  EGRFLG = FN201(SYS_DELPR) already added to EGRDC if set.

-  EGR_HOSE_TST = Flag to Enable EGR OBDII Hose Integrity checks.

-  EGR_MON = Monitor flag for EGR (sent to OBD-II exec); 1 -> all EGR faults

have been monitored at least once since power-up.

-  EGRMON_DN = Down count actuation flag for EGRMON_TMR.

-  EGRMON_UP = Up count actuation flag for EGRMON_TMR.

-  EGR_ON_FLG = Flag to indicate more than minimum EGR requested.

-  EGR_RETRY = Second pass of EGR monitor flag.

-  EGR_TST_ENA = EGR test enable flag; (sent by  OBD-II  exec);  1  ->  test

enabled

-  EGR_TST_LST = Last pass value of EGR_MON.

-  EGR_TST_RDY = EGR test ready flag; (sent to OBD-II exec.); 1 -> all local

condition  have  been  met; ready to run EGR tests.  Note:  EGR tests are

allways ready to run (except in the case of an abort).

-  EGR_TST_RNNG = EGR continuous monitor is in progress flag.

-  ENG_STDY_FLG = Engine Steady for EGR OBDII tests.

-  ER_EGR_FLG = EGR is in Engine Running On Demand mode flag.

-  ER_ISC_REQ = Flag to request ER_RPM from ISC system.

-  EVLOPNFM_FLG = EGR valve open failure mode flag.

-  FFG_EGR = OBDII system FMEM flag for the system;  1  ->  EGR  system  not

currently operating reliably.

-  FFG_EGR_LST = Last pass value of FFG_EGR.

-  HOSE_TST_CNT = Flag to force continuation of hose tests.

-  OBDII_RESET = Flag used to simulate the receipt of an  OBD-II  scan  tool

reset message.

-  PxxxDN_FLG = Flag to enable down counting of fault xxx fault timer.

-  PxxxENA = Flag to enable counting up of fault xxx test timer.

-  PxxxFLT = Indicates fault xxx is present.

-  PxxxOLD = Last pass value of PxxxFLT.

12-29

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PxxxMALF = Malfunction flag for code Pxxx; 1 -> a  malfunction  currently

exists for fault Pxxx.

-  PxxxUP_FLG = Flag to enable up counting of fault xxx fault timer.

-  VAC_RNG_FLG = OK Vac Range for EGR OBDII tests.

-  WRM_IDL_FLG = Warm Idle Flag.  Used for EGR.

Calibration Constants:

-  BP_EGRMON = Allow EGR monitor to complete below this BP, in/Hg.

-  DELARCH_EGR = Air Charge delta used in OBDII EGR test.

-  DELPR_ON_DEM = Minimum DELPR allowed to pass OBDII on-demand test.

-  DELPR_THRES1 = Threshold on DELPR to check  for  downstream  vacuum  hose

off.

-  DELPR_THRES3 = Threshold on DELPR to check for upstream vacuum hose off.

-  EGRACT_MON = Minimum EGR_RATE_ACT for proper operation  of  EGR  monitor,

percentage.

-  EGR_CLR_RT = Fraction of EGR fault test time to consider a fault to  have

cleared.

-  EGR_ER_DONE = Value for state of ER_STATUS to indicate engine running EGR

test is complete, pointer.

-  EGR_ER_INIT = Value for state of ER_STATUS to request engine running  egr

test, pointer.

-  EGR_ER_RPM = Engine running on-demand test requested engine speed.

-  EGR_FLT_MAP = Bit mapped word used to selectively remap EGR  fault  codes

to the generic EGR fault code P0400.

-  EGR_FLT_RT = Fraction of EGR fault test time to consider a  fault  to  be

present.

-  EGR_HT_AM_MX = EGR OBDII Hose Test Air Mass Maximum, lb/min.

-  EGR_MIN_AMB = Minimum ambient temp for reporting of EGR faults, deg.  F.

-  EGR_MON_ACTH = Allow EGR monitor to complete above this ACT, degrees F.

-  EGR_MON_ACTL = Allow EGR monitor to complete below this ACT, degrees F.

-  EGRMON_TM = EGR monitor completion timeout time, seconds.

-  EGR_OBD_NMAX = NMAX for EGR OBDII tests.

-  EGR_REP_SW = Enables EGR fault retesting.   1  ->  retest  always,  2  ->

restest after first pass.

12-30

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  EGR_RTST_TM = EGR no-flow test retest time, sec.

-  EGRTST_TM = Time required to run OBDII EGR tests before EGR is enabled.

-  EGR_TWO_TIME = Enables EGR monitor retest mode.

-  EGR_VLV_OPN = Time taken for EGR Valve to open,  worst  case,  for  OBDII

tests, seconds.

-  EGR_VLV_SHT = Time taken for the EGR  Valve  to  shut  under  worst  case

conditions, for OBDII EGR Hose tests.

-  EGR_WRM_TM = Time before EGR is first enabled to allow EPTZER to  update,

sec.

-  EPTMAX = Maximum EPT reading (open), counts.

-  EPTMIN = Minimum EPT reading (short), counts.

-  EPT_VOPN_IDL = EPT change to indicate EGR valve stuck open at idle.

-  EPTZER_VALID = Max time since EPTZER update for valid EPTZER value.

-  FN004 = BP correction for exhaust backpressure calculation, "Hg.

-  FN074B(AM) = Upstream pressure as a function of AM; Used  in  calculating

exhaust pressure, " H2O.

-  FN202(SYS_DELPR) = Minimum DELPR at high EGRDC.

-  MAX_MVAC = Maximum Manifold Vacuum for EGR OBDII tests.

-  MIN_MVAC = Minimum Manifold Vacuum for EGR OBDII tests.

-  PE_OBD_THRES = Exhaust Backpressure Threshold for OBDII EGR tests, " H20.

-  PxxxTST_TM = Duration of test Pxxx.

-  TC_DELPR_M6 = Time constanst for DELPR for mode 06 reporting, sec.

-  TC_EGRDC_M6 = Time constant for EGRDC for mode 06 reporting, sec.

-  TC_EPTBAR_M6 = Time constanst for EPTBAR for mode 06 reporting, sec.

-  TC_HOSE_M6 = Tome constant for DELPR for mode 06 reporting, sec.

-  TC_M_DEL_M6 = Time constant for MIN_DELPR for mode 06 reporting, sec.

-  TCSYS_DELPR = Filter time constant for SYS_DELPR.

-  V_EGRDC_MAX = Minimum EVR duty cycle for running insufficient flow test.

OTHER

-  egr_codes = Set of (P0400, P0401,  P0402,  P1400,  P1401,  P1405,  P1406,

P1408,  P1409).   The  set  of  OBD-II fault codes that relate to the EGR

monitor.

12-31

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  malfunction(egr,Pxxx)= Public procedure updating the MIL  control  status

provided by the MIL control module.

-  P0400 = Fault code for EGR system malfunction.

-  P0401 = Fault code for insufficient EGR flow.

-  P0402 = Fault code for EGR Valve stuck open.

-  P1400 = Fault code for EVP/EPT circuit below minimum voltage.

-  P1401 = Fault code for EVP/EPT circuit above maximum voltage.

-  P1405 = Fault code for DPFE Upstream hose off.

-  P1406 = Fault code for DPFE Downstream hose off.

-  P1408 = Fault code for insufficient EGR flow.

-  P1409 = Fault for driver failure.

12-32

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EGR_OBDII_COM1

OBD-II EXECUTIVE INTERFACE

PFEHP = 1 -------------------|

;EGR enabled                |

|

EGREN_TMR >= EGRENA_TM -     |AND -| EGR_TST_RDY = 1

EGR_WRM_TM ----|     |     |  ;monitor ready

;EGR system warm      |     |     |

;enough to test       |     |     |

|OR --|     |

CCM_ER_ENA = 1 --------|           |

;KOER in progress     |           |

|           |

CCM_EO_ENA = 1 --------|           |

;KOEO in progress                 |

| --- ELSE ---

|

| EGR_TST_RDY = 0

EGR_TST_RDY = 1 -------------|

;monitor enabled            |

|

EGR_TST_ENA = 1 -------|     |AND -| EGR_TST_RNNG = 1

;enabled by exec      |     |     |  ;enable monitor

|     |     |

CCM_ER_ENA = 1 --------|OR --|     |

;KOER in progress     |           |

|           |

CCM_EO_ENA = 1 --------|           |

;KOEO in progress                 |

| --- ELSE ---

|

| EGR_TST_RNNG = 0

|  ;disable monitor

MONITOR RESET CONDITIONS

OBDII_RESET = 1 -------------|

;reset command issued       |OR --| Do: egr_clear_timers

|     |  ;reset monitor

DEMAND_MODE <> DEMAND_LAST --|

;going into or out of

;demand mode

12-33

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

To prevent the setting of false codes and activation of the MIL light, it  is

necessary for a fault condition to be present for a calibrated period of time

before the code is set.  Two timers are used for each test  to  perform  this

function.   The  first  timer is used to indicate the amount of time spent in

the  test  for  determination  of  test  completion.   This  timer  is  named

PxxxTST_TMR,  where xxx is the fault code being tested.  This timer operation

is controlled through the flag PxxxENA according to the following logic:

PxxxTST_TMR

PxxxENA      Operation

--------     -----------

0           freeze

1          count up

The second timer is used to indicate the amount of time the fault  conditions

are  present  during  the  test.   This  timer  is  named  PxxxFLT_TMR and is

contolled by the flags PxxxUP_FLG and PxxxDN_FLG according to  the  following

logic:

PxxxFLT_TMR

PxxxUP_FLG     PxxxDN_FLG     Operation

-----------     -----------    ------------

0               0            freeze

0               1          count down

1               0           count up

EPT OUT OF RANGE LOW TEST

EGR_TST_RNNG = 0 ------------------| Do: egr_test_disabled(P1400)

;monitor not enabled              |

| --- ELSE ---

|

EPTBAR < EPTMIN -------------------| Do: egr_fault_detected(P1400)

;low sensor reading               |

| --- ELSE ---

|

| Do: egr_no_fault(P1400)

EPT OUT OF RANGE HIGH TEST

EGR_TST_RNNG = 0 ------------------| Do: egr_test_disabled(P1401)

;monitor not enabled              |

| --- ELSE ---

|

EPTBAR > EPTMAX -------------------| Do: egr_fault_detected(P1401)

;EPT voltage high                 |

| --- ELSE ---

|

| Do: egr_no_fault(P1401)

12-34

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

STUCK OPEN VALVE TEST

Check for EPTBAR being high at idle, indicating a stuck open valve.

P1400TST_TMR <= P1400TST_TM -|

;sensor failed low test     |

;not complete               |

|

P1401TST_TMR <= P1401TST_TM -|

;sensor failed high test    |

;not complete               |

|

P1400FLT = 1 ----------------|

;sensor failed low          |

|

P1401FLT = 1 ----------------|

;sensor failed high         |

|OR --| Do: egr_test_disabled(P0402)

P1409FLT = 1 ----------------|     |

;EVR driver failed          |     |

|     |

EGR_TST_RNNG = 0 ------------|     |

;monitor not in progress    |     |

|     |

WRM_IDL_FLG = 0 -------------|     |

;not in warm idle state     |     |

|     |

CCM_EO_ENA = 1 --------------|     |

;in KOEO                          |

| --- ELSE ---

|

EPTBAR > EPTZER_OFF +              |

EPT_VOPN_IDL ---------| Do: egr_fault_detected(P0402)

;valve position above engine off  | EPTBAR_OPN = ROLAV(EPTBAR,TC_EPTBAR_M6)

;value + tolerance                |  ;filter EPTBAR for mode 06 reporting

|

| --- ELSE ---

|

| Do: egr_no_fault(P0402)

| EPTBAR_OPN = ROLAV(EPTBAR,TC_EPTBAR_M6)

|  ;filter EPTBAR for mode 06 reporting

always ----------------------------| EPT_OPN_MAX = EPTZER_OFF + EPT_VOPN_IDL

|  ;store failure condition for mode 06

12-35

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DPFE HOSE DISCONNECTION CHECK ENABLE

This test is run when the EGR valve is closed.  When the EGR valve is closed,

the EPT pressure should approach zero (i.e.  EPTBAR should approach EPTZER).

NOTES:

1 - This test is intrusive if EGR is enabled.  It will attempt to run  before

EGR is enabled, however, in which case it is non-intrusive.

2 - Test is disabled if complete and no fault was detected.  If a  fault  was

detected,  the  test  will continue to run non-intrusively.  This is to allow

clearing of the fault in case of ice melting in the EPT lines, etc.

3 - Test is forced to continue if an insufficient EGR flow (0401) is detected

and the ambient temperature at the start of the tests is low.  This is to aid

in recovery from a condition where both EPT hoses are frozen.   If  one  hose

thaws before the other, it will be detected as a plugged or disconnected hose

(1405 or 1406).  Presence of a 1405 or 1406 when an 0401 is present indicates

to  the software that a frozen hose condition is present.  Testing is started

from the beginning if this condition occurs.

P0401FLT = 1 ----------|

;insufficient EGR flow|

|AND -|

HOSE_TST_AMB <         |     |

EGR_MIN_AMB --|     |

;coolant temp at start of   |

;hose tests low             |

|

P1405TST_TMR <= P1405TST_TM -|

;upstream hose test not     |

;complete                   |OR --| HOSE_TST_CNT = 1

|     |  ;continue running hose tests

P1406TST_TMR <= P1406TST_TM -|     |

;downstream hose test not   |     |

;complete                   |     |

|     |

P1405FLT = 1 ----------------|     |

;upstream hose fault        |     |

|     |

P1406FLT = 1 ----------------|     |

;downstream hose fault            |

| --- ELSE ---

|

| HOSE_TST_CNT = 0

|  ;do not force hose test continuation

always ----------------------------| PEXHAUST = FN074B(AM) *

|             29.875 / FN004(BP)

|  ;calculate inferred exh. pressure

|  ;corrected for barometric pressure

12-36

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

HOSE_TST_CNT = 0 ------------|

;hose tests not forced on   |

|

P0402TST_TMR <= P0402TST_TM -|

;stuck open test not done   |

|

P1400FLT = 1 ----------------|

;EPT sensor failed low      |

|

P1401FLT = 1 ----------------|

;EPT sensor failed high     |

|

P0402FLT = 1 ----------------|

;EGR valve stuck open       |

|

P1409FLT = 1 ----------------|

;EVR driver failed          |

|

EGR_TST_RNNG = 0 ------------|OR --| EGR_HOSE_TST := 0

;monitor not enabled        |     |  ;Do not request Hose Test

|     |

PEXHAUST < PE_OBD_THRES -----|     |

;exhaust pressure too low   |     |

|     |

EPTZER_TMR2 > EPTZER_VALID --|     |

;too long since valid valve |     |

;closed position update     |     |

|     |

EGREN_TMR < EGRENA_TM        |     |

- EGRTST_TM ---------|     |

;long enough time since     |     |

;start for valid test -     |     |

;try to run before EGR      |     |

;enabled if possible        |     |

|     |

CCM_ER_ENA = 1 --------------|     |

;in KOER                    |     |

|     |

CCM_EO_ENA = 1 --------------|     |

;in KOEO                    |     |

|     |

AM > EGR_HT_AM_MX -----------|     |

;Air mass flow rate too           |

;high for test due to             |

;intake pulsations                |

| --- ELSE ---

|

| EGR_HOSE_TST := 1

|  ;Request Hose Test.

| DELPR_HOSE = ROLAV(DELPR,TC_HOSE_M6)

|  ;filter DELPR for mode 06 reporting

12-37

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

P1405TST_TMR = 0 ------------|

;hose tests not started     |AND -| HOSE_TST_AMB = INFAMB_KAM

|     |  ;ambient temp at start of hose tests

EGR_HOSE_TST = 1 ------------|

;hose tests requested

EGREN = 1 -------------------------| EGR_OFF_TMR = 0

;EGR enabled                      |  ;restart time since valve closed

UPSTREAM EPT HOSE DISCONNECTED TEST

EGR_HOSE_TST = 0 ------------|

;hose test not enabled      |OR --| Do: egr_test_disabled(P1405)

|     |

EGR_OFF_TMR < EGR_VLV_SHT ---|     |

;valve not closed long enough     |

| --- ELSE ---

|

DELPR <= DELPR_THRES3 -------------| Do: egr_fault_detected(P1405)

;large negative delta pressure    |

| --- ELSE ---

|

| Do: egr_no_fault(P1405)

DOWNSTREAM EPT HOSE DISCONNECTED TEST

EGR_HOSE_TST = 0 ------------|

;hose test not enabled      |OR --| Do: egr_test_disabled(P1406)

|     |

EGR_OFF_TMR < EGR_VLV_SHT ---|     |

;valve not closed long enough     |

| --- ELSE ---

|

DELPR > DELPR_THRES1 --------------| Do: egr_fault_detected(P1406)

;high delta pressure              |

| --- ELSE ---

|

| Do: egr_no_fault(P1406)

12-38

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

MANIFOLD VACUUM RANGE CHECK

INF_MVAC > MIN_MVAC ---------|

|AND -| VAC_RNG_FLG = 1

INF_MVAC < MAX_MVAC ---------|     |  ;vacuum in range

;vacuum within allowable range    |

| --- ELSE ---

|

| VAC_RNG_FLG = 0

RATE OF CHANGE IN AIR CHARGE

always ----------------------------| ARCH_PROP_D = Abs(AIR_CHG_CUR -

|                    AIR_CHG)/AIR_CHG

STEADY ENGINE CHECK

ARCH_PROP_D < DELARCH_EGR ---|

;steady engine load         |AND -| ENG_STDY_FLG = 1

|     |  ;engine steady

N < EGR_OBD_NMAX ------------|     |

;rpm below max                    |

| --- ELSE ---

|

| ENG_STDY_FLG = 0

12-39

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

INSUFFICIENT EGR FLOW TEST

The following test first infers the pressure  difference  between  the  inlet

manifold  and  the  exhaust  back  pressure  (SYS_DELPR).   This  is  used to

determine a minimum allowed EGR delta pressure during high EGR.

The test is performed when EGRDC is above a minimum value and a comparison is

made between actual DELPR (EPT delta pressure) and the expected DELPR for the

current SYS_DELPR and a calibrated EGR valve duty cycle.  Should  the  actual

DELPR  be  less  than  the  estimated  DELPR  for the limiting duty cycle, as

calibrated, then a fault exists.

always ----------------------------| SYS_DELPR = ROLAV(INF_MVAC +

|              PEXHAUST/13.6,TCSYS_DELPR)

|  ;exhaust - intake pressure

VAC_RNG_FLG = 1 -------------|

;vacuum in range            |AND -| CHK_VPLG_FLG = 1

|     |  ;valve plugged test running

ENG_STDY_FLG = 1 ------------|     | MIN_DELPR = FN202(SYS_DELPR)

;engine is steady                 |  ;minimum delta pressure for SYS_DELPR

|

| --- ELSE ---

|

| CHK_VPLG_FLG = 0

|  ;valve plugged test not running

12-40

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

P1405TST_TMR <= P1405TST_TM -|

;upstream hose test not     |

;complete                   |

|

P1406TST_TMR <= P1406TST_TM -|

;downstream hose test not   |

;complete                   |

|

P1409TST_TMR <= P1409TST_TM -|

;drive test not complete    |

|

P1400FLT = 1 ----------------|

;sensor failed low          |

|

P1401FLT = 1 ----------------|

;sensor failed high         |

|

P1405FLT = 1 ----------------|

;upstream hose off          |

|

P1406FLT = 1 ----------------|

;downstream hose off        |OR --| Do: egr_test_disabled(P0401)

|     |

P0402FLT = 1 ----------------|     |

;EGR valve stuck open       |     |

|     |

P1409FLT = 1 ----------------|     |

;EVR driver failed          |     |

|     |

P0401FLT = 1 ----------------|     |

;test has failed            |     |

|     |

EGR_TST_RNNG = 0 ------------|     |

;test not in progress       |     |

|     |

CHK_VPLG_FLG = 0 ------------|     |

;valve plugged test not in        |

;progress                         |

| --- ELSE ---

DELPR < MIN_DELPR -----------|     |

;delta pressure below min   |AND -| Do: egr_fault_detected(P0401)

|     | EGRDC_FLW = ROLAV(EGRDC,TC_EGRDC_M6)

EGRDC > V_EGRDC_MAX ---------|     |  ;filter EGRDC for mode 06

;high duty cycle                  | DELPR_FLW = ROLAV(DELPR,TC_DELPR_M6)

|  ;filter DELPR for mode 06

| MIN_DELPR_FL = ROLAV(MIN_DELPR,

|                       TC_M_DEL_M6)

|  ;filter MIN_DELPR for mode 06

|

| --- ELSE ---

(continued on next page)

12-41

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|

DELPR > MIN_DELPR -----------------| Do: egr_no_fault(P0401)

| EGRDC_FLW = ROLAV(EGRDC,TC_EGRDC_M6)

|  ;filter EGRDC for mode 06

| DELPR_FLW = ROLAV(DELPR,TC_DELPR_M6)

|  ;filter DELPR for mode 06

| MIN_DELPR_FL = ROLAV(MIN_DELPR,

|                       TC_M_DEL_M6)

|  ;filter MIN_DELPR for mode 06

|

| --- ELSE ---

|

| P0401ENA = 0

| P0401UP_FLG = 0

| P0401DN_FLG = 0

|  ;stop test and fault timers

12-42

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

ENGINE RUNNING ON DEMAND - INSUFFICIENT FLOW TEST

The following logic commands the engine speed and EGR flow to desired

values for operation of the P1409 and P1408 tests.  Note that the P1409 test

must run to completion prior to operation of the P1408 test.

ER_STATUS = EGR_ER_INIT -----|

;EGR on demand test         |

;requested                  |

|

ER_EGR_FLG = 0 --------------|AND -| ER_ISC_REQ = 1

;not running last pass      |     |  ;command engine speed

|     | ER_RPM = EGR_ER_RPM

P0402TST_TMR > P0402TST_TM --|     |  ;set commanded engine speed

;stuck open valve                 | ER_EGR_FLG = 1

;test complete                    |  ;command EGR action

| DEMAND_TIMER = 0

|  ;reset time since test started

|

| --- ELSE ---

ER_STATUS = EGR_ER_INIT -----|     |

;EGR on demand test         |     |

;requested                  |     |

|     |

EGR_TST_RNNG = 0 ------|     |     |

;monitor disabled     |     |     |

|     |     |

P1408TST_TMR >         |     |     |

P1408TST_TM ---------|     |     |

;EGR KOER test done   |     |AND -| ER_ISC_REQ = 0

|     |     |  ;stop commanding engine speed

P0402FLT = 1 ----------|     |     | ER_EGR_FLG = 0

;stuck open test done |     |     |  ;stop commanding EGR

|     |     | ER_STATUS = EGR_ER_DONE

P1400FLT = 1 ----------|     |     |  ;indicate EGR on-demand test complete

;low voltage test     |OR --|

;complete             |

|

P1401FLT = 1 ----------|

;high voltage test    |

;complete             |

|

P1409FLT = 1 ----------|

;driver test complete |

|

DEMAND_ABORT = 1 ------|

;KOER aborted

12-43

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

P1409TST_TMR <= P1409TST_TM -|

;EVR driver test not        |

;complete                   |

|

P0402TST_TMR <= P0402TST_TM -|

;valve stuck open test not  |

;complete                   |

|

P1400FLT = 1 ----------------|

;sensor failed low          |

|

P1401FLT = 1 ----------------|

;sensor failed high         |

|

P0402FLT = 1 ----------------|OR --| Do: egr_test_disabled(P1408)

;EGR valve stuck open       |     |

|     |

P1409FLT = 1 ----------------|     |

;EVR driver failed          |     |

|     |

EPTZER_TMR2 > EPTZER_VALID --|     |

;too long since zero        |     |

;flow pressure update       |     |

|     |

EGR_TST_RNNG = 0 ------------|     |

;monitor disabled           |     |

|     |

ER_EGR_FLG = 0 --------------|     |

;Not in ER EGR demand mode  |     |

|     |

DEMAND_TIMER < EGR_VLV_OPN --|     |

;Not on for sufficient time       |

;for valve to open                |

| --- ELSE ---

|

DELPR < DELPR_ON_DEM --------------| Do: egr_fault_detected(P1408)

;DELPR indicates low/no flow      |

| --- ELSE ---

|

| Do: egr_no_fault(P1408)

12-44

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TEST COMPLETION LOGIC

Monitor has run to completion if all continuous monitor filters have exceeded

their respective levels.  Completion is also flagged if the monitor is unable

to run for an excessive amount of time due to high altitude  or  extreme  air

charge temperatures.

ACT < EGR_MON_ACTL ---------|

;low ACT                   |

|

ACT > EGR_MON_ACTH ---------|OR --|

;high ACT                  |     |

|     |

BP < BP_EGRMON -------------|     |

;low barometric pressure         |

|

EGRACT < EGRACT_MON --------------|AND -| EGRMON_UP = 1

;low EGR flow                    |     | EGRMON_DN = 0

|     |  ;count up EGRMON_TMR

EGREN = 1 ------------------------|     |

;egr system enabled              |     |

|     |

P0401TST_TMR = 0 -----------------|     |

;insufficient flow test not            |

;running                               |

| --- ELSE ---

ACT >= EGR_MON_ACTL --------|           |

;ACT not too low           |           |

|           |

ACT <= EGR_MON_ACTH --------|AND -|     |

;ACT not too high          |     |     |

|     |     |

BP >= BP_EGRMON ------------|     |     |

;not at high altitude            |OR --| EGRMON_UP = 0

|     | EGRMON_DN = 1

EGRACT >= EGRACT_MON -------------|     |  ;count down EGRMON_TMR

;high EGR flow                   |     |

|     |

P0401TST_TMR > 0 -----------------|     |

;insufficient flow test is in          |

;progress                              |

| --- ELSE ---

|

| EGRMON_UP = 0

| EGRMON_DN = 0

|  ;freeze EGRMON_TMR

P1405TST_TMR <= P1405TST_TM ------------| EGRMON_TMR = 0

;hose test is not complete             |  ;reset timer

12-45

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

P0401TST_TMR >

P0401TST_TM ----------|

|

P0402TST_TMR >              |

P0402TST_TM ----------|

|

P1400TST_TMR >              |

P1400TST_TM ----------|

|

P1401TST_TMR >              |

P1401TST_TM ----------|

|AND -|

P1405TST_TMR >              |     |

P1405TST_TM ----------|     |

|     |

P1406TST_TMR >              |     |

P1406TST_TM ----------|     |

|     |OR --| EGR_MON = 1

P1409TST_TMR >              |     |     |  ;EGR OBDII monitor has run to

P1409TST_TM ----------|     |     |  ;completion

|     |

EGRMON_TMR >                      |     |

EGRMON_TM ------------------|     |

;timer counted up                      |

| --- ELSE ---

|

OBDII_RESET = 1 ------------------------| EGR_MON = 0

;reset in progress                     |  ;reset to initial state

12-46

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OBDII EGR FAILURE MODE FLAG

All OBDII testing affected by EGR is disabled if FFG_EGR is set.

P0401FLT = 1 ---------------------|

;EGR valve plugged               |

|

P0402FLT = 1 ---------------------|

;EGR valve stuck open            |

|

P1400FLT = 1 ---------------------|

;EPT sensor failed low           |

|

P1401FLT = 1 ---------------------|OR --| FFG_EGR = 1

;EPT sensor failed high          |     |  ;EGR OBDII failure present

|     |

P1405FLT = 1 ---------------------|     |

;EPT upstream hose failure       |     |

|     |

P1406FLT = 1 ---------------------|     |

;EPT downstream hose failure     |     |

|     |

P1409FLT = 1 ---------------------|     |

;EVR driver failed                     |

| --- ELSE ---

|

| FFG_EGR = 0

12-47

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FROZEN HOSE RECOVERY AND FAULT RETEST LOGIC

If a fault is detected during the first pass through  the  EGR  monitor,  all

tests  are restarted from the beginning to verify the fault.  This is done to

avoid setting the wrong code if the test was run out of sequence.

If the upstream or downstream hose tests (1405 or 1406) go from a failed to a

passing  state  and the ACT at the start of the tests was low, ice is assumed

to have melted in one of the hoses and  the  tests  are  restarted  from  the

beginning.

If  an  insufficient  flow  failure  is  present  during  a  low  temperature

condition,  the  tests are restarted after a time delay.  This has the effect

of turning on the EGR system to test 0401 over after  all  other  tests  have

completed.   This  could  result  in  poor driveability if the fault is still

present, as the failure mode effect of ice in  both  EPT  hoses  is  a  fully

opened  valve.   The  system  is  not  turned  on  again,  however, until the

individual hose tests (1405 and 1406) have run to completion.

If an insufficient flow failure is present and an upstream or downstream hose

failure  becomes  active,  the ice in a single hose is assumed to have melted

and the testing is restarted from the beginning.  Note  that  this  condition

can only become true if the ACT at the start of the hose tests was low.

If the insufficient EGR flow code (0401) changes from a  failed  state  to  a

passing  state,  it  is  possible that a single hose has melted subsequent to

both EPT hoses containing ice.  Because the no-flow test is not reliable when

one  hose is plugged, it is necessary under these conditions to start testing

over from the beginning of the hose tests.  This condition can only occur  if

the  ice melts after the 1405 and 1406 tests are complete but before the 0401

test begins.

P0401FLT = 0 ----------------------| EGR_RTST_TMR = 0

;insufficient flow failure        |  ;reset retest timer

;not present

EGR_MON = 1 -----------------|

;monitor complete           |

|

FFG_EGR = 1 -----------------|OR --| EGR_EFM_FLG = 1

;a fault has been detected  |     |  ;first pass complete

|

EGR_REP_SW <> 2--------------|

;retest only after first

;pass complete

12-48

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

P1405FLT = 0 ----|

;upstream test  |

;not failed     |AND -|

|     |

P1405OLD = 1 ----|     |

;failed last pass     |

|

P1406FLT = 0 ----|     |

;downstream test|     |

;not failed     |AND -|OR --|

|     |     |

P1406OLD = 1 ----|     |     |

;failed last pass     |     |AND -|

|     |     |

EGR_RTST_TMR >         |     |     |

EGR_RTST_TM ------|     |     |

;time to retest             |     |

|     |

HOSE_TST_AMB < EGR_MIN_AMB --|     |

;ambient temp at start of         |

;hose tests low                   |

|

P0401FLT = 1 ----------------|     |

;insufficient flow          |     |

|AND -|OR --| Do: egr_clear_timers

P1405FLT = 1 ----------|     |     |     |  ;reset monitor

;upstream hose        |     |     |     | EGR_RETRY = 1

;failure              |OR --|     |     |  ;keep EGR system off

|           |     | FFG_EGR = 0

P1406FLT = 1 ----------|           |     |  ;reset failure flag

;downstream hose failure          |

|

FFG_EGR = 1 -----------------|     |

;a fault has been detected  |     |

|     |

EGR_RETRY = 0 ---------------|AND -|

;on first pass              |

|

EGR_REP_SW <> 0 -------------|

;fault verification enabled

FFG_EGR = 0 ----------------------|

;no faults detected              |

|

FFG_EGR_LST = 1 ------------|     |

;fault present last pass   |     |

|OR --|

EGR_MON = 1 ----------|     |     |AND -| EGR_RETRY = 0

;monitor is complete |AND -|     |     |  ;reset to first pass

|           |

EGR_TST_LST = 0 ------|           |

;monitor not complete            |

;last pass                       |

|

EGR_RETRY = 1 --------------------|

;on second pass

12-49

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EGR FAILURE MODE FLAG

EGR strategy disabled if EFMFLG is set

P0401FLT = 1 ---------------------|

;Insufficient flow failure       |

|

P1400FLT = 1 ---------------------|

;EPT sensor failed low           |

|

P1401FLT = 1 ---------------------|

;EPT sensor failed high          |

|OR --| EFMFLG = 1

P1405FLT = 1 ---------------------|     |  ;disable EGR strategy

;EPT upstream hose failure       |     |

|     |

P1406FLT = 1 ---------------------|     |

;EPT downstream hose failure     |     |

|     |

EGR_RETRY = 1 --------------|     |     |

;on second pass            |     |     |

|     |     |

P1405TST_TMR <=             |     |     |

P1405TST_TM -------------|AND -|     |

;hose reversal test        |           |

;not complete              |           |

|           |

EGR_EFM_FLG = 1 ------------|           |

;EFM not inhibited on                  |

;first pass                            |

| --- ELSE ---

|

| EFMFLG = 0

EGR VALVE STUCK OPEN FAILURE MODE FLAG

EGR valve closed position learning disabled if EVLOPNFM_FLG set

P0402FLT = 1 ------------------------| EVLOPNFM_FLG = 1

;EGR valve stuck open               |  ;disable EPTZER learning

|

| --- ELSE ---

|

| EVLOPNFM_FLG = 0

|  ;EPTZER learning not disabled

12-50

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OBDII EXECUTIVE CODE AND MIL CONTROL INTERFACE

The mapping of EGR  system  faults  to  fault  codes  is  controlled  by  the

calibration  word  EGR_FLT_MAP.  This is a bit-mapped word with the following

mapping:

BIT     CAL VALUE     STRATEGY NAME     CODE

---     ---------     -------------     ----

0         1          p0401bit          P0401

1         2          p0402bit          P0402

2         4          p1400bit          P1400

3         8          p1401bit          P1401

5        32          p1405bit          P1405

6        64          p1406bit          P1406

7       128          p1408bit          P1408

8       256          p1409bit          P1409

HOSE_TST_AMB >= EGR_MIN_AMB --|

;high enough ambient temp    |

|AND -| Do: malfunction(egr,P0401)

P0401FLT = 1 -----------------|     | EGR_MON = 1

|     |  ;test complete

p0401bit = 0 -----------------|     |

| --- ELSE ---

|

P0401MALF = 1 ----------------------| Do: clear_malf(P0401)

INFAMB_KAM >= EGR_MIN_AMB ----|

;high enough ambient temp    |

|AND -| Do: malfunction(egr,P0402)

P0402FLT = 1 -----------------|     | EGR_MON = 1

|     |  ;test complete

p0402bit = 0 -----------------|     |

| --- ELSE ---

|

P0402MALF = 1 ----------------------| Do: clear_malf(P0402)

INFAMB_KAM >= EGR_MIN_AMB ----|

;high enough ambient temp    |

|AND -| Do: malfunction(egr,P1400)

P1400FLT = 1 -----------------|     | EGR_MON = 1

|     |  ;test complete

p1400bit = 0 -----------------|     |

|

|

| --- ELSE ---

|

P1400MALF = 1 ----------------------| Do: clear_malf(P1400)

12-51

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

INFAMB_KAM >= EGR_MIN_AMB ----|

;high enough ambient temp    |

|AND -| Do: malfunction(egr,P1401)

P1401FLT = 1 -----------------|     | EGR_MON = 1

|     |  ;test complete

p1401bit = 0 -----------------|     |

| --- ELSE ---

|

P1401MALF = 1 ----------------------| Do: clear_malf(P1401)

HOSE_TST_AMB >= EGR_MIN_AMB --|

;high enough temp           |

|AND -| Do: malfunction(egr,P1405)

P1405FLT = 1 -----------------|     | EGR_MON = 1

|     |  ;test complete

p1405bit = 0 -----------------|     |

| --- ELSE ---

|

P1405MALF = 1 ----------------------| Do: clear_malf(P1405)

HOSE_TST_AMB >= EGR_MIN_AMB --|

;high enough temp           |

|AND -| Do: malfunction(egr,P1406)

P1406FLT = 1 -----------------|     | EGR_MON = 1

|     |  ;test complete

p1406bit = 0 -----------------|     |

| --- ELSE ---

|

P1406MALF = 1 ----------------------| Do: clear_malf(P1406)

P1408FLT = 1 -----------------|

|AND -| Do: store_code(egr,P1408)

p1408bit = 0 -----------------|     |

| --- ELSE ---

|

P1408MALF = 1 ----------------------| Do: clear_malf(P1408)

INFAMB_KAM >= EGR_MIN_AMB ----|

;high enough ambient temp    |

|AND -| Do: malfunction(egr,P1409)

P1409FLT = 1 -----------------|     | EGR_MON = 1

|     |  ;test complete

p1409bit = 0 -----------------|     |

| --- ELSE ---

|

P1409MALF = 1 ----------------------| Do: clear_malf(P1409)

12-52

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

HOSE_TST_AMB >=

EGR_MIN_AMB ---------|

|

p0401bit = 1 -----------|AND -|

|     |

P0401FLT = 1 -----------|     |

|

INFAMB_KAM >=                 |

EGR_MIN_AMB ---------|     |

;high enough ACT       |     |

|AND -|

p0402bit = 1 -----------|     |

|     |

P0402FLT = 1 -----------|     |

|

INFAMB_KAM >=                 |

EGR_MIN_AMB ---------|     |

;high enough ACT       |     |

|AND -|

p1400bit = 1 -----------|     |

|     |

P1400FLT = 1 -----------|     |

|

INFAMB_KAM >=                 |

EGR_MIN_AMB ---------|     |

;high enough ACT       |     |

|AND -|

p1401bit = 1 -----------|     |

|     |

P1401FLT = 1 -----------|     |

|

HOSE_TST_AMB >=               |

EGR_MIN_AMB ---------|     |

|     |

p1405bit = 1 -----------|AND -|

|     |OR --| Do: malfunction(egr,P0400)

P1405FLT = 1 -----------|     |     |  ;indicate EGR system failure

|     | EGR_MON = 1

HOSE_TST_AMB >=               |     |  ;indicate monitor completion

EGR_MIN_AMB ---------|     |     |

|     |     |

p1406bit = 1 -----------|AND -|     |

|     |     |

P1406FLT = 1 -----------|     |     |

|     |

p1408bit = 1 -----------|     |     |

|AND -|     |

P1408FLT = 1 -----------|     |     |

|     |

INFAMB_KAM >=                 |     |

EGR_MIN_AMB ---------|     |     |

|     |     |

p1409bit = 1 -----------|AND -|     |

|           |

P1409FLT = 1 -----------|           |

| --- ELSE ---

|

P0400MALF = 1 ----------------------| Do: clear_malf(P0400)

;EGR malfunction flag set             ;clear EGR malfunction flag

12-53

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

LAST PASS VALUE STORAGE

always ----------------------------| P1405OLD = P1405FLT

| P1406OLD = P1406FLT

| P0401OLD = P0401FLT

|  ;last pass values of PxxxFLT

| DEMAND_LAST = DEMAND_MODE

|  ;last pass value of demand mode

| FFG_EGR_LST = FFG_EGR

|  ;last pass value of fault flag

| EGR_TST_LST = EGR_MON

|  ;laast pass value of monitor

|  ;completion flag

BEGIN: egr_test_disabled(cxxxx)

This is a global subroutine used to control the fault timers  and  flags  for

the EGR OBDII strategy when the test is disabled.

always ----------------------------| cxxxxENA = 0

|  ;stop test timer

| cxxxxUP_FLG = 0

| cxxxxDN_FLG = 0

|  ;stop fault timer

cxxxxTST_TMR <= cxxxxTST_TM -------| cxxxxTST_TMR = 0

;test not complete                |  ;reset test timer

cxxxxFLT_TMR <= cxxxxTST_TM *

EGR_CLR_RT ----------------| cxxxxFLT = 0

;fault timer below fault clear    |  ;indicate no fault

;time

cxxxxFLT = 0 ----------------------| cxxxxFLT_TMR = 0

;fault not present                |  ;reset fault timer

END: egr_test_disabled

12-54

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: egr_fault_detected(cxxxx)

This is a global subroutine used to control the fault timers  and  flags  for

the EGR OBDII strategy when fault is present.

always ----------------------------| cxxxxENA = 1

|  ;count up test timer

| cxxxxDN_FLG = 0

|  ;do not count down fault timer

cxxxxFLT_TMR < cxxxxTST_TM --------| cxxxxUP_FLG = 1

;fault timer below test time      |  ;count up fault timer

|

| --- ELSE ---

|

| cxxxxUP_FLG = 0

|  ;stop fault timer

cxxxxFLT_TMR > cxxxxTST_TM

* EGR_FLT_RT ----------------| cxxxxFLT = 1

;fault timer above fault          |  ;indicate fault present

;time

END: egr_fault_detected

12-55

EGR STRATEGY, EGR OBDII TEST LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: egr_no_fault(cxxxx)

This is a global subroutine used to control the fault timers  and  flags  for

the EGR OBDII strategy when a passing condition is detected.

always ----------------------------| cxxxxENA = 1

|  ;count up test timer

| cxxxxUP_FLG = 0

| cxxxxDN_FLG = 1

|  ;count down fault timer

cxxxxFLT_TMR <= cxxxxTST_TM *

EGR_CLR_RT ----------------| cxxxxFLT = 0

;fault timer below fault clear    |  ;indicate no fault

;time

END: egr_no_fault

BEGIN: egr_clear_timers

This is a global subroutine used to clear out all of the  EGR  related  fault

timers and fault bits.

always ----------------------------| P0401FLT_TMR = 0

| P0401TST_TMR = 0

| P0401FLT = 0

| P0401OLD = 0

| P0402FLT_TMR = 0

| P0402TST_TMR = 0

| P0402FLT = 0

| P1400FLT_TMR = 0

| P1400TST_TMR = 0

| P1400FLT = 0

| P1401FLT_TMR = 0

| P1401TST_TMR = 0

| P1401FLT = 0

| P1405FLT_TMR = 0

| P1405TST_TMR = 0

| P1405FLT = 0

| P1405OLD = 0

| P1406FLT_TMR = 0

| P1406TST_TMR = 0

| P1406FLT = 0

| P1406OLD = 0

| P1408FLT_TMR = 0

| P1408TST_TMR = 0

| P1408FLT = 0

| P1409FLT_TMR = 0

| P1409TST_TMR = 0

| P1409FLT = 0

END: egr_clear_timers

12-56

EGR STRATEGY, SCCD OUTPUT COMMAND - CDAN2

PED-PTE FoMoCo, PROPRIETARY & CONFIDENTIAL

12.2  SCCD OUTPUT COMMAND (CDAN0)

OVERVIEW

This routine outputs the Exhaust Gas Vacuum Reservoir (EVR)  current  command

to the SCCD smart driver.  It also provides SCCD diagnostics.

DEFINITIONS

INPUTS

Registers:

-  EGRDC = EGR duty cycle.

-  EVR_STATUS = EVR smart output driver fault status.

Calibration Constants:

-  EGR_TST_DC = Minimum duty cycle to allow passing of SCCD test.

OUTPUTS

Registers:

-  EVR_STATUS = See above.

Bit Flags:

-  PxxxENA = Flag to enable up counting of fault xxx test timer.

-  PxxxUP_FLG = Flag to enable up counting of fault xxx fault timer.

-  PxxxDN_FLG = Flag to enable down counting of fualt xxx fault timer.

PROCESS

STRATEGY MODULE: EGR_SCCD_COM1

always ---------------------------------| EVR_STATUS := output(EGRDC)

|  ;output to SCCD

12-57

EGR STRATEGY, SCCD OUTPUT COMMAND - CDAN2

PED-PTE FoMoCo, PROPRIETARY & CONFIDENTIAL

EVR DRIVER DIAGNOSTICS

The  following  logic  interfaces  to   the   EGR   diagnostics.    See   the

EGR_OBDII_COMx module for documentation on the fault filtering scheme.

EVR_STATUS is placed in a non-zero state by the operating system if  an  over

or under current condition is detected.  It is not possible to detect an over

current condition when EGRDC is zero.  The lack of fault logic  is  therefore

only run when EGRDC is non-zero.

EGR_TST_RNNG = 0 -----------------|

;monitor disabled                |

|OR --| Do: egr_test_disabled(P1409)

EGRDC < EGR_TST_DC ---------|     |     |

;duty cycle below min      |AND -|     |

|           |

EVR_STATUS = 0 -------------|           |

;driver status indeterminate           |

| --- ELSE ---

|

EVR_STATUS > 0 -------------------------| Do: egr_fault_detected(P1409)

;output driver fault detected          |

|

| --- ELSE ---

|

EGRDC >= EGR_TST_DC --------------------| Do: egr_no_fault(P1409)

;duty cycle above min

12-58

CHAPTER 13

IDLE SPEED CONTROL

13-1

IDLE SPEED CONTROL, GENERIC ISC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

13.1  GENERIC IDLE SPEED CONTROL (CDAL0)

OVERVIEW

This chapter describes the adaptive air bypass idle speed control system.  In

general,  the  ISC  system  is  designed to regulate the duty cycle to an air

bypass solenoid as necessary to obtain the desired engine speed for all  idle

operating conditions (base idle; hi-cam; various accessory loads) and provide

for a dashpot action.  Predicted airflows for the different  load  states  at

idle are adaptively corrected to minimize the impact of hardware variability.

Acceptable idling performance is achieved by a careful balance of bypass  air

solenoid control and feedback spark control.

Feedback Spark offers the fastest way to change engine torque.  As such,  its

use is important in the control of load transitions like A/C, power steering,

and  neutral/drive  changes.   Aggressive  use  of  Feedback  Spark  is  very

effective  in  limiting  rpm  changes  during these conditions.  The Feedback

Spark strategy is described in the spark chapter.

The amount of airflow through the air bypass is controlled  by  the  solenoid

position,  which  is  in  turn  determined  by  the solenoid duty cycle.  The

objective of the idle speed control strategy  is  to  determine  ISCDTY.   As

mentioned   above,  calibration  of  the  bypass  actuator  control  must  be

coordinated with that of Feedback Spark.

The overall bypass air ISC logic sets ISCDTY to one of the following:

I.   CRANK:

a) Engine Stopped:

ISCDTY = 0%

b) Engine Moving:

ISCDTY = FN884(TCSTRT)%

II. FMEM (MAP and TP sensor out of range):

ISCDTY = FMMISC

III.  NORMAL RUNNING ISC:

ISCDTY = IDCMUL * FN8000(DEBYMA,PERLOAD_ISC) + IDCOFS

IV. SPECIAL SELF TEST CONDITIONS

Key On Engine off; OSM_EO_ON = 1  - ISCDTY = 100%

OSM_EO_OFF = 1 - ISCDTY = 0

Key On Engine Running; ER_ISC_HLD = 1 - ISCDTY is frozen.

13-2

IDLE SPEED CONTROL, GENERIC ISC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

NORMAL ISC

OVERVIEW

Under most operating conditions, ISCDTY  is  obtained  from  the  Normal  ISC

logic.  During Normal ISC, the strategy can operate in any one of four modes:

DASHPOT PREPOSITION, DASHPOT, RPM CONTROL, or LOCKOUT.   The  modes  will  be

described in more detail shortly.

Regardless of  which  of  the  four  modes  is  active,  the  strategy  first

calculates a total desired idle airflow, DESMAF:

DESMAF = DESMAF_PRE + DASPOT + IPSIBR + ISCKAM

ISC_A_PROP + ISC_A_DER

DESMAF represents the total engine airflow  required  for  idle.   There  are

slight  differences  in  the calculation of DESMAF depending on which mode is

active.  These differences are summarized below and will be described in more

detail in the discussion of each individual mode.

DASHPOT PREPOSITION MODE (ISCFLG = 0)

DESMAF_PRE = Initial prediction, based on rpm, load, & temperature

DASPOT = DASPTK * (DSTPBR - (RATCH + DELHYS)) + FN830

IPSIBR = Fixed at last calculated value (not updated in this mode)

ISCKAMn = KAM cell n, where n is selected by ISFLAG

DASHPOT MODE (ISCFLG = -1)

DESMAF_PRE = Initial prediction, based on rpm, load, & temperature

DASPOT = old DASPOT - FN879(DASPOT)

IPSIBR = Fixed at last calculated value (not updated in this mode)

ISCKAMn = KAM cell n, where n is selected by ISFLAG

RPM CONTROL MODE (ISCFLG = 1)

DESMAF_PRE = Initial prediction, based on rpm, load, & temperature

DASPOT <= DASCTL (DASPOT must be below DASCTL to enter RPM control)

IPSIBR = old IPSIBR + ISCPSI

ISCKAMn = KAM cell n, where n is selected by ISFLAG

RPM CONTROL LOCKOUT MODE (ISCFLG = 2)

DESMAF_PRE = Initial prediction, based on rpm, load, & temperature

DASPOT <= DASCTL (DASPOT must be below DASCTL to enter lockout mode)

IPSIBR = old IPSIBR + ISCPSI

ISCKAMn = KAM cell n, where n is selected by ISFLAG

13-3

IDLE SPEED CONTROL, GENERIC ISC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

NORMAL ISC - continued

During Normal ISC, the strategy executes a number  of  tasks  in  a  specific

order.   These  tasks  perform the actual logic and calculations required for

any of the four modes of operation.  The order of execution of the  tasks  is

listed  below.   The  details of each task are described in separate sections

which follow.

NORMAL ISC TASKS

1) DSDRPM_CALC   - calculation of DSDRPM & DESMAF_PRE & setting of ISFLAG

2) RPMERR_CALC   - calculation of RPMERR_A & RPMERR_S

3) DASPOT_CALC   - calculation of DASPOT

4) MODE_SELECT   - selection of mode & setting of ISCFLG

5) IPSIBR_CALC   - IPSIBR update & calculation of DESMAF

6) ISCDTY_CALC   - calculation of DEBYMA & ISCDTY

7) ISCKAM_UPDATE - adaptive update of ISCKAM

13-4

IDLE SPEED CONTROL, GENERIC ISC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  APT = Throttle Mode flag.

-  DSDRPM = Desired engine speed.

-  DSTPBR = Dashpot filtered throttle position.

-  GR_CM = Commanded gear for shift solenoids.

-  GR_CM_LST = Commanded gear in last background pass.

-  IPSIBR = The closed loop integration  component  of  total  DESMAF,  ppm.

Designed to provide integral feedback, IPSIBR adjusts the value of DESMAF

to correct for sustained changes in idle load.  An increase  or  decrease

in IPSIBR results in a corresponding change to bypass valve duty cycle.

-  ISC_A_DER = Derivative contribution to DESMAF.

-  ISC_A_PROP = Proportional contribution to DESMAF.

-  ISCDTY = Idle speed control duty cycle.

-  ISCFLG = ISC mode indicator flag;  -1  ->  Dashpot  Mode,  0  ->  Dashpot

Preposition Mode, 1 -> closed Loop RPM Control Mode, 2 -> Closed Loop RPM

Control (Lock_out entry to RPM control).

-  N = Engine revolutions, RPM.

-  N_RATCH = RPM value  which  only  ratchets  down.   When  not  at  closed

throttle,  N = N_RATCH.  When at closed throttle, N_RATCH is only allowed

to go down.  N_RATCH is an input to the  minimum  daspot  clip.   N_RATCH

ratchets down to prevent rpm flares after a declutch.

-  OSM_EO_ON = On state requested for  OSM  outputs  during  engine  off  on

demand test.

-  OSM_EO_OFF = Off state requested for OSM outputs  during  engine  off  on

demand test.

-  PERLOAD_ISC = Used by ISC logic as the Y input to FN8000.   0-1.0  scale,

unitless.

-  RATCH = Closed throttle position, counts.

-  SUBST_REQ[0] = Output  state  control  substitution  requested  flag  for

channel 0.

-  TCSTRT = Temperature of ECT at Cold Start-up, deg F.

-  TSLPIP = Time since last PIP (msecs).

-  VSBAR = Filtered vehicle speed for transmission.

13-5

IDLE SPEED CONTROL, GENERIC ISC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Bit Flags:

-  CRKFLG = Crank Mode flag; 1 -> Crank mode.

-  ER_ISC_HLD = Engine run ISC Flag used to  freeze  ISCDTY  to  it  current

value.

-  FLG_DASMNQ = VSBAR flip-flop flag for minimum DASPOT clip.

-  HCAMFG = Flag indicating the completion of Hi-Cam; 0 -> no desired engine

speed  adder  exists, 1 -> RPM adder above base idle is present.  Flag is

used in the ISC adaptive update routine to disable updates when HCAMFG  =

1.

-  MFMFLG = MAF Failure flag; 1 -> MAF out of range.

-  NDSFLG = 0 -> transmission in Neutral, 1 -> transmission in gear.

-  RUNUP_FLG = Flag indicating that initial runup is complete; 1 ->  initial

runup complete.

-  TFMFLG = TP Failure flag; 1 -> TP out of range.

Calibration Constants:

-  DASMHYST = Hysteresis for DASMPH.

-  DASMPH = Minimum VSBAR for declutch DASPOT clip, mph.

-  FMMDSD = Failure Mode Management default desired RPM.

-  FMMISC = Default Duty cycle to ISC, fraction.

-  FN884(TCSTRT) = ISC Duty Cycle in Crank, deg.

-  FN8000 = ISCDTY as a function of DEBMYA and PERLOAD_ISC.

-  TRLOAD = Transmission Load.

0 -> Manual Transmission, no clutch or gear switches,

forced neutral state (NDSFLG = 0).

1 -> Manual Transmission, no clutch or gear switch.

2 -> Manual Transmission, one clutch or gear switch.

3 -> Manual Transmission, both clutch and gear switches.

4 -> Auto Transmission, non-electronic, neutral drive switch.

5 -> Auto Transmission, non-electronic, neutral pressure switch,

(AXOD).

6 -> Auto Transmission, electronic, PRNDL sensor - park,

reverse, neutral, overdrive, manual 1, manual 2.

13-6

IDLE SPEED CONTROL, GENERIC ISC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: ISC_OVERVIEW_COM8

OVERALL BYPASS AIR ISC LOGIC

The purpose  of  the  ISC  logic  is  to  determine  ISCDTY.   Under  certain

conditions,   ISCDTY  is  set  to  specific  values.   Under  most  operating

conditions, ISCDTY is determined from the Normal ISC logic which is described

later.    The  logic  below  describes,  at  highest  level,  how  ISCDTY  is

determined.

OSC response logic:  If Output State Control has been  requested  for  ISCDTY

but  the  local  conditions  will  not  allow for the substitution, a general

reject message will be sent.

SUBST_REQ[0] = 0 -----------------------| do nothing

| (no OSC required)

|

| --- ELSE ---

ER_ISC_HLD = 1 -------------------|     |

(flag set by diagnostic test)    |     |

|     |

CRKFLG = 1 -----------------|     |     |

|AND -|     |

TSLPIP < 2000 ms -----------|     |     |

|     |

RUNUP_FLG = 0 --------------|     |OR --| (OSC substitute requested but local

|AND -|     |  conditions will not allow

CRKFLG = 0 -----------|     |     |     |  substitution)

|OR --|     |     |

TSLPIP < 2000 ms -----|           |     | Do: OSC_RESPONSE(0,22h)

(no PIPs yet or stall)           |     |

|     |

MFMFLG = 1 -----------------|     |     |

|AND -|     |

TFMFLG = 1 -----------------|           |

| --- ELSE ---

|

| do nothing

13-7

IDLE SPEED CONTROL, GENERIC ISC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OSM_EO_ON = 1 --------------------------| ISCDTY = 1.0

;output commanded on by KOEO test      |

| --- ELSE ---

|

OSM_EO_OFF = 1 -------------------------| ISCDTY = 0

;output commanded off by KOEO test     |

| --- ELSE ---

|

| (ISC logic disabled)

ER_ISC_HLD = 1 -------------------------| freeze ISCDTY

(flag set by diagnostic test)          | EXIT ISC LOGIC

|

| --- ELSE ---

|

| (engine stopped)

CRKFLG = 1 -----------------------|     |

(in crank mode)                  |AND -| iscdty = 0

|     | (0% - actuator closed)

TSLPIP >= 2000 ms. ---------------|     | IPSIBR = 0

(no PIPs yet or stall)                 | DSTPBR = RATCH

| Do: substitute(0,iscdty)

| ISCDTY = iscdty

|  (output state control override)

|

| EXIT ISC LOGIC

|

| --- ELSE ---

|

CRKFLG = 1 -----------------------|     |  (engine moving)

(in crank mode)                  |OR --| Do: DSDRPM_CALC

|     | Do: RPMERR_CALC

RUNUP_FLG = 0 --------------------|     |  (update RUNUP_FLG)

(runup not completed)                  | ISCDTY = FN884(TCSTRT)%

| (duty cycle function of

|   temperature at start)

| IPSIBR = 0

| DSTPBR = RATCH

| EXIT ISC LOGIC

|

| --- ELSE ---

|

MFMFLG = 1 -----------------------|     | (FMEM fault present)

|AND -| ISCDTY = FMMISC

TFMFLG = 1 -----------------------|     | DSDRPM = FMMDSD

| ISCFLG = 0

| RUNUP_FLG = 1

| HCAMFG = 1

| EXIT ISC LOGIC

|

| --- ELSE ---

|

| (normal ISC)

| update FLG_DASMNQ

| update N_RATCH

| ISCDTY is calculated from the

| Normal ISC logic described later

13-8

IDLE SPEED CONTROL, GENERIC ISC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLG_DASMNQ and N_RATCH LOGIC

This logic is used by DASPOT.  It is shown here because it is executed as

part of the OVERALL ISC LOGIC.  It is executed anytime the Normal ISC logic

is done so it will be available, if required by DASPOT mode.

FLG_DASMNQ

VSBAR >= DASMPH + DASMHYST ------|S Q -| FLG_DASMNQ

|     | (Prepare to add dashpot

VSBAR < DASMPH ------------------|C    |  to prevent declutch stall)

N_RATCH

APT = 0 OR 1 --------------------|

(PT or WOT)                     |

|

TRLOAD = 3 ----------------|     |

(manual w/both switches)  |AND -|

|     |

NDSFLG = 1 ----------------|     |

(driveline engaged)             |

|OR --| N_RATCH = N

TRLOAD > 3 ----------------|     |     |

(automatic)               |     |     |

|     |     |

GR_CM = 3 -----------------|AND -|     |

(now in 3rd gear)         |     |     |

|     |     |

GR_CM_LST = 4 -------------|     |     |

(was in 4th)                    |     |

|     |

APT = -1 ------------------|     |     |

(closed throttle)         |AND -|     |

|           |

N <= N_RATCH --------------|           |

(RPM decreasing)                      |

| --- ELSE ---

|

| no change to N_RATCH

13-9

IDLE SPEED CONTROL, GENERIC ISC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

13.2  FEATURE:  ISC_DSDRPM - V1.A1_ISC_DSDRPM (CDAN0)

13.2.1  DSDRPM CALCULATION (CDAL0)

OVERVIEW

This module controls the calculation of desired idle rpm  (DSDRPM)  including

HICAM.   DSDRPM  is used as a control input for closed loop and adaptive idle

speed control.  It is also used to calculate DESMAF_PRE.

The strategy calculates a desired value for engine speed, DSDRPM.   While  in

idle speed control mode the strategy attempts to maintain the engine speed at

the desired value.  The previous value of DSDRPM is  saved  in  the  register

DSDRPM_LST.   DSDRPM  is  composed  of  a  Base  portion,  IS_TRAN_RPM,  plus

additional adders for A/C, Power  Steering  and  High  Speed  Fan.   It  also

contains a HICAM term which is explained later.

In general terms, DSDRPM is calculated in ISC_DSDRPM_LOADS_COMx as follows,

DSDRPM = IS_TRAN_RPM + HICAM  + ISADD_ACRPM + ISADD_POWS + ISADD_HEDF

GPAS Requirements:

Automatic transmission vehicles operating in drive are limited by  Ford  GPAS

requirements  to a maximum desired idle rpm.  Under these conditions, DSDRPM,

including all adders, is  clipped  to  the  value  GPAS_CLIP  as  a  maximum.

GPAS_CLIP  is  calculated  in  the  ISC_DSDRPM_trans_COMx  module, and passed

across the Transmission/Engine interface.

HICAM portion of DSDRPM:

The HICAM portion of DSDRPM is composed of a variety of  adders  for  special

operating  conditions  (ECT,  ACT,  CTNTMR,  etc.).   It is calculated in the

module ISC_DSDRPM_HICAM_COMx as follows;

HICAM = FN825A + FN825B + FN821B + FN880 + BZZRPM + FN826A + FN826B

DEFINITIONS

See other ISC_DSDRPM_xx modules.

PROCESS

STRATEGY MODULE: ISC_DSDRPM_COM10

BEGIN: DSDRPM_CALC

Unconditionally --------------| do: dsdrpm_timer

| do: dsdrpm_hicam

| do: dsdrpm_base

| do: gpas_clip

| do: dsdrpm_loads

| do: dsdrpm_output

13-10

IDLE SPEED CONTROL, DESIRED RPM CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

END: DSDRPM_CALC

13-11

IDLE SPEED CONTROL, DESIRED RPM CALCULATION CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

13.2.2  IS_TRAN_RPM And MAXIMUM CLIP CALCULATION - (CDAL0)

GENERAL TRANSMISSION

OVERVIEW

This module has two purposes.  The generation of a base desired engine  speed

and the generation of a maximum value for DSDRPM, (i.e GPAS clip).

The first section describes the calculation of desired idle rpm, IS_TRAN_RPM,

appropriate   for   the  current  transmission  state  of  any  transmission.

IS_TRAN_RPM is used in the  ISC_DSDRPM_COMx  module  to  generate  the  final

DSDRPM  value.   This  value  will  reflect  engine  loads in addition to the

transmission state.

The second section calculates the GPAS clip.

DEFINITIONS

INPUTS

Registers:

-  ALT_CAL_FLG = Flat to indicate use of alternate calibration.

-  ATMR3 = Timer which counts up in RUN mode.  (Reset to zero at powerup)

Bit Flags:

-  DNDSUP = Delayed neutral/drive flag; 0 -> in neutral, no load,  1  ->  in

drive, loaded.

Calibration Constants:

-  CRKTIM = Time after start to delay clip on desired RPM.

-  DRBASE = Base desired engine speed in drive.

-  ISCLPD = A clip on the maximum desired speed that can be  requested  with

vehicle in drive.  Usually the GPAS defined speed allowed at 0.2 miles on

a cold start.

-  ISCLPD_ALT = Alternate calibration value of ISCLPD.

-  NUBASE = Base desired engine speed in neutral.

-  DRBASE_ALT = Alternate calibration DRBASE.

-  NUBASE_ALT = Alternate calibration NUBASE.

-  TCTRAN_RPM = Time constant used  to  filter  changes  in  DSDRPM  due  to

changes in transmission state.

13-12

IDLE SPEED CONTROL, DESIRED RPM CALCULATION CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TRLOAD = Transmission Load switch:

0 -> Manual transmission, no clutch or gear switch, NDSFLG = 0

1 -> Manual trans, no clutch or gear switch

2 -> Manual trans, one clutch or gear switch

3 -> Manual trans, both switches

4 -> Automatic trans, NDS

5 -> Automatic trans, NPS

6 -> Automatic trans, 7 position PRNDL

7 -> Automatic trans, PRNDL switches (4EAT)

OUTPUTS

Registers:

-  GPAS_CLIP = Maximum permissible value of DSDRPM, GPAS clip.

-  IS_TRAN_RPM  =  Desired  engine  speed   based   on   transmission   load

requirements.

PROCESS

STRATEGY MODULE: ISC_DSDRPM_BASE_COM1

BEGIN:  dsdrpm_base

Select base on whether alternate calibration is required:

ALT_CAL_FLG = 1 ------------------------| pre_nubase = NUBASE_ALT

| pre_drbase = DRBASE_ALT

|

| --- ELSE ---

|

|  pre_nubase = NUBASE

|  pre_drbase = DRBASE

IS_TRAN_RPM CALCULATION lOGIC:

TRLOAD <= 3 ----------------------|

; Manual Transmission             |OR --| IS_TRAN_RPM = pre_nubase

|     |

DNDSUP = 0 -----------------------|     |

;In neutral                             | --- ELSE ---

|

| IS_TRAN_RPM = ROLAV(pre_drbase,

|                      TCTRAN_RPM)

| ; Filter N-D shifts in DSDRPM

END: dsdrpm_base

13-13

IDLE SPEED CONTROL, DESIRED RPM CALCULATION CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: gpas clip

For automatic transmissions in drive, pre_isclpd is used to prevent  creeping

and harsh engagements.

ALT_CAL_FLG = 1 ------------------------| pre_isclpd := ISCLPD_ALT

|

| --- ELSE ---

|

| pre_isclpd := ISCLPD

ATMR3 > CRKTIM -------------------|

; Delay has timed out.            |

|

TRLOAD > 3 -----------------------|AND -| GPAS_CLIP := pre_iscpld

; Non Man. Trans.                 |     | ;Vehicle stationary clip

|     |

DNDSUP = 1 -----------------------|     |

;In Drive                               | --- ELSE ---

|

|

| GPAS_CLIP := 4080

| ;set clip to max value, i.e no clip

END: gpas clip

13-14

IDLE SPEED CONTROL, DESIRED RPM (HICAM) CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

13.2.3  DSDRPM (HICAM) CALCULATION (CDAJ0)

OVERVIEW

This module describes the calculation of HICAM which  is  a  speed  adder  to

DSDRPM.   DSDRPM is used as a control input for closed loop and adaptive idle

speed control.

The HICAM portion of DSDRPM is composed of a  variety  of  adders  which  are

enabled under specific operating conditions of ECT, ACT, CTNTMR, etc.

HICAM = FN825A + FN825B + FN821B + FN880 + BZZRPM + FN826A + FN826B

HICAM is allowed to rise  instantaneously,  but  any  decrease  in  value  is

filtered to prevent a sudden drop in DSDRPM.  This filtered value of HICAM is

called DESNLO (time constant TCDESN).  The flag, HCAMFG, is set if  HICAM  is

non zero.  HCAMFG is used to prevent adaptive airflow updates (ISCKAM).

DEFINITIONS

INPUTS

Registers:

-  ACT = Air charge temperature, deg.  F.

-  ALT_CAL_FLG = Flat to indicate use of alternate calibration.

-  ATMR1 = Timer which counts up in run/underspeed mode.

-  ATMR3 = Timer which counts up in run mode.  (Reset to 0 only at powerup).

-  DESNLO = Filtered value of the high cam portion of the  DSDRPM  register.

Applied only when Hicam is decreasing, using time constant TCDESN.

-  ECT = Engine coolant temperature, deg.  F.

-  HCTMR = High Cam timer which counts up in RUN/UNDERSPEED mode.

-  HICAM = Symbol used to represent the hi-cam adders to DSDRPM.

-  IS_TRAN_RPM  =  Base  desired  engine  idle   speed.    Determined   from

transmission state.

-  PDL = PRNDL position.

-  TCSTRT = Temperature of ECT at Cold Startup, deg F.

-  VOLTMR = Timer which records  time  that  the  system  voltage  is  below

battery charging level, sec.

13-15

IDLE SPEED CONTROL, DESIRED RPM (HICAM) CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Bit Flags:

-  CTNFLG = 1 -> closed throttle neutral idle.

-  DNDSUP = Delayed neutral/drive flag; 0 -> in neutral, no load,  1  ->  in

drive, loaded.

-  ER_ISC_REQ = Engine Running ISC request flag.  If  set  DSDRPM  value  is

loaded with ER_RPM.

-  HCDFLG = Flag used to ensure that FN826A is locked out of HICAM, once  it

has been used once.

-  LOWVOL_FLG = Flag used to indicate the flip-flop  state  of  Low  voltage

timer.

-  PTSCR = Part throttle since crank mode flag; 0 -> driver has  not  tipped

in since start, 1 -> driver tipped in, kick down desired RPM.

Calibration Constants:

-  BZZRPM = RPM adder intended to provide a short increase in RPM for engine

cleanout  on  start-up.  The buzz-up function is not affected by the part

throttle kickdown until BZZTM expires.

-  BZZRPM_ALT = Alternate calibration for BZZRPM.

-  BZZTM = Time for which BZZRPM adder is in effect.

-  BZZTM_ALT = Alternate calibration for BZZTM.

-  CHGTM = Time delay after leaving Closed  Throttle  to  permit  VOLTMR  to

decrement, sec.

-  FN803A(ACSTRT) = Throttle kick down timer value as a function of ACSTRT.

-  FN821B(VOLTMR) = RPM adder as a function of  VOLTMR  to  provide  battery

charge control.

-  FN825A(ECT) = RPM adder as a  function  of  ECT.   Provides  base  Hi-Cam

function.

-  FN825A_ALT = Alternate calibration for FN825A.

-  FN825B(ACT) = RPM adder as a function of ACT.  Provides  higher  idle  at

very low ambients.

-  FN826A(TCSTRT) = RPM adder as a function of ECT at start.  This adder  is

not  used  when  either  the first part throttle transition since exiting

crank is observed or the time since  start  exceeds  a  calibrated  value

(FN803A).

-  FN826B(ACT) = Start-up RPM adder as a function of ACT.  This adder is not

used  when  either the first part throttle transition since exiting crank

is observed or the time since start exceeds a calibrated value (TKDTN).

13-16

IDLE SPEED CONTROL, DESIRED RPM (HICAM) CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FN880(CTNTMR) = DSDRPM adder vs.  time at idle and in neutral  (CTNTMR  -

See   timer   chapter   for   exact  definition)  Used  as  part  of  the

inspection/maintenance strategy.  Remember that any RPM above  base  idle

disables ISCKAM adaptive learning via HCAMFG.

-  HCDCLP = Maximum HICAM RPM clip for battery charging in drive.

-  HCNCLP = Maximum HICAM RPM clip for battery charging in Neutral.

-  LOWVOL_CH = Hysteresis term for LOWVOL_SL, volts.

-  LOWVOL_SL = System voltage level, below which the battery is discharging,

volts.

-  TC_VBAT = Rolav time constant used to generate VBATBAR in VOLTMR logic.

-  TCDESN = Filter constant for the desired engine  speed  calculated  value

(DSDRPM).   Used  to  slow  changes  in  desired  speed in the decreasing

direction.

-  TKDTN = Time since start after which FN826B(ACT) is eliminated.

-  VBAT = Rolling average of IKYPWR.

-  VOLTCLP = Maximum clip value to freeze VOLTMR at upper threshold.

OUTPUTS

Registers:

-  DESNLO = See above.

-  VBATBAR = Rolling average of VBAT, (Time constant of TC_VBAT).

Bit Flags:

-  HCAMFG = Flag indicating the completion of Hi-Cam; 0 -> no desired engine

speed  adder  exists, 1 -> an RPM adder above base idle is present.  Flag

is used in the ISC adaptive update routine to disable updates when HCAMFG

= 1.

13-17

IDLE SPEED CONTROL, DESIRED RPM (HICAM) CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: ISC_DSDRPM_HICAM_COM1

BEGIN: dsdrpm_hicam

ALT_CAL_FLG = 1 ------------------------| pre_bzzrpm := BZZRPM_ALT

| pre_fn825a := FN825A_ALT

| pre_bzztm := BZZTM_ALT

| ; use alt. values

|

| --- ELSE ---

|

| pre_bzzrpm := BZZRPM

| pre_fn825a := FN825A

| pre_bzztm := BZZTM

| ; use standard values

unconditionally ------------------------| hicam_base := pre_fn825a + FN825B

FN880(CTNTMR) Inspection maintenance logic:

CTNFLG = 1 -----------------------------| hicam_insp := FN880

|

| --- ELSE ---

|

| hicam_insp := 0.0

FN821B(VOLTMR) Battery charge & clipping logic:

unconditionally ------------------------| hicam_bat := FN821B

DNDSUP = 0 -----------------------------| tmp_clip = HCNCLP

| ; Clip for Neutral

|

| --- ELSE ---

|

| tmp_clip = HCDCLP

| ; Clip in DRIVE

hicam_base + hicam_insp >= hicam_bat ---| hicam := hicam_base + hicam_insp

|

| -- ELSE --

|

| hicam := min(hicam_bat,tmp_clip)

13-18

IDLE SPEED CONTROL, DESIRED RPM (HICAM) CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BZZRPM logic:

ATMR1 < pre_bzztm ----------------------| hicam := hicam + pre_bzzrpm

|

| --- ELSE ---

|

| NO ACTION

FN826A(ECT at Start) Logic: This logic is intended to provide:

1) Additional rpm for engine stability and warmup and to assist with

catalyst lightoff.

2) Extended demist capability at low ECT/ACT at start

Note: On an auto. transmission package, the logic is disabled in DRIVE.

In order to reduce RPM as quickly as possible to avoid transmission

engagement harshness, PDL, and not DNDSUP, is used in the logic.

HCDFLG = 0 -----------------------|

;'Lock-out' is false             |

|

TRLOAD =< 3 ----------------|     |

;Man Trans                 |     |

|OR --|

PDL = 7 --------------|     |     |

;Park                |OR --|     |AND -| hicam = hicam + FN826A

|           |     | ; add FN826A(ECT at START)

PDL = 5 --------------|           |     |

;Neutral                         |     |

|     |

HCTMR < FN803A(ACSTRT) -----------|     |

|     |

PTSCR = 0 ------------------|     |     |

;No tip-in yet             |OR --|     |

|           |

ATMR1 < pre_bzztm ----------|           |

| --- ELSE ---

|

| HCDFLG = 1

| ;Set 'lock-out' flag

HCTMR - High Cam Timer Logic:

CRKFLG = 0 -----------------------------| COUNT UP HCTMR

|

| --- ELSE ---

|

| HCTMR = 0

13-19

IDLE SPEED CONTROL, DESIRED RPM (HICAM) CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FN826B(ACT) Logic: This logic is intended to enhance hot start performance.

ATMR1 < TKDTN --------------------|

|AND -| HICAM = hicam + FN826B

PTSCR = 0 ------------------|     |     | ; add FN826B(ACT)

;No tip-in yet             |OR --|     |

|           |

ATMR1 < pre_bzztm ----------|           |

| --- ELSE ---

|

| HICAM = hicam

| ; Assign hicam to HICAM

ON-DEMAND Diagnostic TEST:

ER_ISC_REQ = 1 -------------------------| DESNLO := ER_RPM - IS_TRAN_RPM

| DESNLO := max(DESNLO,0)

|

| --- ELSE ---

|

| NO ACTION

DESNLO down filter for Hicam:

HICAM < DESNLO -------------------------| DESNLO := ROLAV(HICAM,TCDESN)

| HICAM := DESNLO

| ; HICAM is decreasing - do filter

|

| --- ELSE ---

|

| DESNLO := HICAM

| ; Not decreasing - no filter

END: dsdrpm_hicam

13-20

IDLE SPEED CONTROL, DESIRED RPM (HICAM) CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: dsdrpm_timer

VOLTMR - LOW VOLTAGE TIMER

* Low voltage timer (VOLTMR) represents the amount of time that filtered

battery voltage, as indicated by the VBATBAR calculation, is in a

'low voltage' state. This state is indicated by the LOWVOL_FLG flag.

* VOLTMR is referenced by FN821B to determine the increase in engine speed

necessary to provide battery charge compensation.

* A filter time constant, TC_VBAT, is provided to desensitise the logic to

transient changes in VBAT. The default calibration is zero, i.e VBATBAR =

VBAT.

unconditionally ------------------------| VBATBAR := ROLAV(VBAT,TC_VBAT)

| ;Form filtered VBAT

VBATBAR < LOWVOL_SL --------------|S Q -| LOWVOL_FLG

|     | ;In low voltage state

VBATBAR >= LOWVOL_CH -------------|C

CRKFLG = 0 -----------------------|

;RUN/UNDERSPEED mode)             |

|AND -| Increment VOLTMR

LOWVOL_FLG = 1 -------------------|     | ;Clip VOLTMR to VOLTCLP

|     |

APT = -1 -------------------------|     |

;Closed Throttle                        |

| --- ELSE ---

NACTMR >= CHGTM ------------------|     |

|AND -| Decrement VOLTMR

LOWVOL_FLG = 0 -------------------|     |

| --- ELSE ---

|

| Freeze VOLTMR

END: dsdrpm_timer

13-21

IDLE SPEED CONTROL, DESIRED RPM (LOADS) CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

13.2.4  DSDRPM (LOADS) CALCULATION (CDAL0)

OVERVIEW

This module describes the calculation of desired idle rpm  (DSDRPM).   DSDRPM

is  used  as a control input for closed loop and adaptive idle speed control.

It is also used to calculate DESMAF_PRE.

This module is compatible with Transmission/Engine  Modularity  in  that  the

base  desired engine RPM, IS_TRAN_RPM, and the GPAS_CLIP are both passed into

this engine module across the Transmission/Engine interface.

The strategy calculates a desired value for engine speed - DSDRPM.  While  in

idle  speed  control mode the strategy attempts to maintain the actual engine

speed at the value of DSDRPM.  The previous  value  of  DSDRPM  is  saved  as

DSDRPM_LST.

DSDRPM is composed of a Base portion, IS_TRAN_RPM, plus a HICAM portion, plus

additional adders for A/C, Power Steering and High Speed Fan.

In general;

DSDRPM = IS_TRAN_RPM + HICAM  + ISADD_ACRPM + ISADD_POWS + ISADD_HEDF

This module supports the Fail-Safe cooling strategy in that if the engine has

overheated,   so  setting  COOL_FLG,  then  the  calibratable  COOL_BASE,  is

substituted for IS_TRAN_RPM, giving:

DSDRPM = COOL_BASE + HICAM  + ISADD_ACRPM + ISADD_POWS + ISADD_HEDF

The HICAM portion  of  DSDRPM  is  calculated  in  the  ISC_DSDRPM_HICAM_COMx

module.

TOT Based RPM Requirements:

This module supports a minimum desired engine speed clip,  IS_TOT_RPM,  which

when  fully  supported  is  determined  from  transmission  oil  temperature.

IS_TOT_RPM is calculated in the ISC_DSDRPM_TOT_COMx module.

GPAS Requirements:

Automatic transmission vehicles operating in drive are limited by  Ford  GPAS

requirements  to a maximum desired idle rpm.  Under these conditions, DSDRPM,

including all adders, is  clipped  to  the  value  GPAS_CLIP  as  a  maximum.

GPAS_CLIP  is  calculated  in  the  ISC_DSDRPM_trans_COMx  module, and passed

across the Transmission/Engine interface.

13-22

IDLE SPEED CONTROL, DESIRED RPM (LOADS) CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  ACIOTMR = A/C off delay timer, for ISC.  sec

-  DSDRPM = Desired engine speed.  See Overview section  for  definition  of

the various uses of this register.

-  COOL_FLG = Flag indicating  that  engine  is  overheating;  1  ->  Engine

overheated.

-  DSDRPM_LST = Previous value of DSDRPM; from last background loop.

-  ER_RPM = DSDRPM value for engine run diagnostic test.

-  GPAS_CLIP = Maximum permissible value of DSDRPM.   It  is  passed  across

from the Engine/Transmission boundary.

-  HEDFHP = Two speed for output present.

-  HICAM = Symbol used to represent the hi-cam adders to DSDRPM.

-  IBGPSI = BG loop CTR for PSI calac.  in ISC.

-  IS_TRAN_RPM = Base desired engine RPM determined by transmission load.

Bit Flags:

-  ACRQST = A/C requested; 1 -> A/C request.

-  AC_RPM_FLG = A/C request flag to add RPM to DSDRPM.

-  DNDSUP = Delayed neutral/drive flag; 0 -> in neutral, no load,  1  ->  in

drive, loaded.

-  ER_ISC_REQ = Engine Running ISC request flag.  If  set  DSDRPM  value  is

loaded with ER_RPM.

-  HSF_FLG = Flag indicating the status of high speed Fan

-  IS_RPM_NCLIP = Flag indicating whether  DSDRPM  has  been  clipped  to  a

minimum  value  (Clipped  =  1)  -  Used  to set HCAMFG to disable ISCKAM

learning.

-  POWSFG = Flag used to indicate that power steering load  is  high;  1  ->

power steering on.

-  PSFLAG = Flag to indicate last pass value of power steering to check  for

transitions; 1 -> power steering was on.

Calibration Constants:

-  AC_OFF_DLY = Time to maintain DNAC_DR/DNAC_NEUT RPM adder after  A/C  has

been disengaged.

13-23

IDLE SPEED CONTROL, DESIRED RPM (LOADS) CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  COOL_BASE = Base desired engine speed when engine overheated.

-  DNAC_DR = RPM increment requested with the A/C on in drive.

-  DNAC_NEUT = RPM increment requested with the A/C on in neutral.

-  DNPOWS = If a power steering pressure  switch  is  used,  this  parameter

increments the desired RPM when an increased load is sensed.

-  HEDFRPM = DSDRPM adder when high speed fan is on, rpm.

-  TRLOAD = Transmission Load switch:

0 -> Manual transmission, no clutch or gear switch

1 -> Manual trans, no clutch or gear switch

2 -> Manual trans, one clutch or gear switch

3 -> Manual trans, both switches

4 -> Automatic trans, NDS

5 -> Automatic trans, NPS

6 -> Automatic trans, 7 position PRNDL

7 -> Automatic trans, PRNDL switches (4EAT)

OUTPUTS

Registers:

-  DSDRPM = See above.

-  ISADD_ACRPM = Speed adder for air-conditioning load.

-  ISADD_HEDF = Speed adder for high speed fan load.

-  ISADD_POWS = Speed adder for power steering load.

Bit Flags:

-  HCAMFG = Flag indicating the completion of Hi-Cam; 0 -> no desired engine

speed  adder  exists, 1 -> an RPM adder above base idle is present.  Flag

is used in the ISC adaptive update routine to disable updates when HCAMFG

= 1.

-  PSFLAG = Flag to indicate last pass value of power steering to check  for

transitions; 1 -> power steering was on.

13-24

IDLE SPEED CONTROL, DESIRED RPM (LOADS) CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: ISC_DSDRPM_LOADS_COM1

BEGIN: dsdrpm_loads

AC_RPM logic: It may be desirable to reduce the severity of any change

in DSDRPM due to A/C load application. For this reason the AC rpm adder,

ISADD_ACRPM, is filtered in both increasing and decreasing directions.

Filter time constant is ACFILTC. Separate rpm adders are provided for

neutral and drive.

TRLOAD <= 3 ----------------------|

;Manual transmission              |OR --| isadd_acrpm := DNAC_NEUT

|     | ; Select Neutral DNAC

DNDSUP = 0 -----------------------|     |

;In neutral                             |

| --- ELSE ---

|

| isadd_acrpm := DNAC_DR

| ; Select Drive DNAC

ACRQST = 1 -----------------------|S Q -| AC_RPM_FLG

;A/C requested                    |

|

ACIOTMR > AC_OFF_DLY -------------|C

AC_RPM_FLG = 1 -------------------------| ISADD_ACRPM :=

|       rolav(isadd_acrpm,ACFILTC)

|

| --- ELSE ---

|

| ISADD_ACRPM := rolav(0.0,ACFILTC)

13-25

IDLE SPEED CONTROL, DESIRED RPM (LOADS) CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DNPOWS LOGIC: (Also affects PSFLAG, HCAMFG, and IBGPSI)

Note 1: A check on Hardware present switches is made in the appropriate

input processing sections. POWSFG & PSFLAG are therefore always correct.

Note 2: HCAMFG will be set/cleared, in the HCAMFG logic, when POWSFG is

set/cleared.

POWSFG = 1 -----------------------|

;PS on now                        |AND -|  ISADD_POWS := DNPOWS

|     |  ;PS off -> on transition

PSFLAG = 0 -----------------------|     |  PSFLAG := 1

;Off last time                          |  IBGPSI := 0

|  ;HCAMFG is set in the HCAMFG logic

|

| --- ELSE ---

POWSFG = 1 -----------------------|     |

;PS on now                        |AND -|  ISADD_POWS := DNPOWS

|     |  ;PS on, not a transition

PSFLAG = 1 -----------------------|     |  ;HCAMFG is set in the HCAMFG logic

;On last time                           |

| --- ELSE ---

POWSFG = 0 -----------------------|     |

;PS off now                       |AND -|  ISADD_POWS := 0.0

|     |  ;PS on -> off transition

PSFLAG = 1 -----------------------|     |   PSFLAG := 0

;On last time                           |   IBGPSI := 0

|  ;HCAMFG is cleared in the HCAMFG

|  ;logic

|

| --- ELSE ---

|

|  ISADD_POWS := 0.0

|  ;PS off

HEDFRPM logic:

HEDFHP = 1 -----------------------|

|AND -| ISADD_HEDF := HEDFRPM

HSF_FLG = 1 ----------------------|     |

| --- ELSE ---

|

| ISADD_HEDF := 0.0

END: dsdrpm_loads

13-26

IDLE SPEED CONTROL, DESIRED RPM (LOADS) CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: dsdrpm_output

TOT based minimum RPM:

unconditionally ---------------------| do: dsdrpm_tot

| ; Calculate IS_TOT_RPM, minimum

| ; desired RPM based on TOT

Fail-Safe Cooling interupt:

COOL_FLG = 1 ------------------------| base_temp := COOL_BASE

;Fail Safe Cooling mode              | ; Use fail safe value as base RPM

|

|  --- ELSE ---

|

| base_temp := IS_TRAN_RPM

| ; Use base desired value

Calculation of DSDRPM & DSDRPM_LST:

ER_ISC_REQ = 1 ----------------------| dsdrpm := ER_RPM

;On-Demand Diag. test                |

| --- ELSE ---

|

| dsdrpm_raw := base_temp + HICAM

|        + ISADD_ACRPM + ISADD_POWS

|        + ISADD_HEDF

| dsdrpm := max(dsdrpm_raw,IS_TOT_RPM)

| ; Catch any TOT based constraints.

Minimum Clip detect Logic:

dsdrpm > dsdrpm_raw  ----------|

|AND -| IS_RPM_NCLIP := 1

ER_ISC_REQ = 0 ----------------|     | ; dsdrpm has been clipped above

| ; dsdrpm_raw - minimum clip is active,

| ; ISCKAM learning will be disabled.

|

| --- ELSE ---

|

| IS_RPM_NCLIP := 0

| ; No min. clip is active

unconditionally ---------------------| DSDRPM_LST = DSDRPM

| ; Save last value of DSDRPM

|

| DSDRPM := min(dsdrpm,GPAS_CLIP)

| ;Then the GPAS constraints.

END: dsdrpm_output

13-27

IDLE SPEED CONTROL, DESIRED RPM (TOT) CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

13.2.5  IS_TOT_RPM CALCULATION (CDAL0)

OVERVIEW

The  module  provides  the  null  assignment  to  IS_TOT_RPM  for  all  those

strategies  that  do  not  have  the  functional  requirement for a TOT based

minimum desired engine speed.

DEFINITIONS

OUTPUTS

Registers:

-  IS_TOT_RPM  =  Minimum  value  of  desired  engine  speed  based  on  TOT

constraints.

PROCESS

STRATEGY MODULE: ISC_DSDRPM_TOT_COM2

BEGIN: dsdrpm_tot

TOT Based Minimum DSDRPM Clip.

Note : In this strategy, there is no functional requirement

for a TOT based minimum desired RPM. For reasons of

commonality, this module returns a null value.

Unconditionally ----------------------| IS_TOT_RPM := 0.0

END: dsdrpm_tot

13.3  FEATURE:  ISC_DESMAF_PID - V1.2_ISC_DESMAF_PID (CDAN0)

13.3.1  KAM UPDATE (ISCKAM_UPDATE) (CDAN0)

OVERVIEW

This section describes the adaptive ISC update routine.   In  general,  under

steady  state  conditions  on  a  stabilized engine at idle, the adaptive ISC

logic will evaluate whether the open  loop  prediction  of  airflow  requires

correction.  If a correction factor was applied, IPSIBR has a non-zero value,

the adaptive ISC strategy will roll this correction value into KAM and  drive

13-28

IDLE SPEED CONTROL, KAM UPDATE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

the  IPSIBR term back to zero.  Control of the rate at which the IPSIBR value

is driven to zero is calibration-dependent.

There are six ISCKAM cells designated for idle corrections.  The  appropriate

cell  is  pointed  to by the flag ISFLAG which tracks the load state at idle.

The following logic must be satisfied to update KAM:

*  In RPM control

*  Within the RPM deadband for a calibrated time interval (UPDISC)

*  No hi-cam adder present (HCAMFG = 0)

*  No kam errors

*  IBGPSI >= UPDATM

*  ISCKAM learning overide flag, ISCKAM_HLT, is clear.  For  example,  PCOMP

is off

*  IPSIBR non zero

ISCKAM corrections are clipped to the same maximum and minimum limits as  the

C/L  RPM  integrator  (PSIBRM/PSIBRN).   Each  time  the  update criteria are

satisfied both IPSIBR and ISCKAM  are  adjusted  one  bit  (0.00024  ppm)  in

opposite directions until IPSIBR = 0.

If the KAM cells are invalid the fault flag KAM_ERROR will have been set.  In

this  situation  the  ISCKAM  cells will be zeroed and ISCKAM updates will be

inhibited until the KAM_ERROR flag is clear.

An error flag IAC_ERR3 is used to indicate an error condition associated with

the  KAM logic.  This flag is processed by the FMEM module and is used in the

OBDII executive routine.

13-29

IDLE SPEED CONTROL, KAM UPDATE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  IAC_ERR3 = IAC error flag used for KAM malfunction detection.

-  IBGPSI = Background loop counter, used to pace ISCKAMn update.

-  IPSIBR = The closed loop integration  component  of  total  DESMAF,  ppm.

Designed to provide integral feedback, IPSIBR adjusts the value of DESMAF

to correct for sustained changes in idle load.  An increase  or  decrease

in IPSIBR results in a corresponding change to bypass valve duty cycle.

-  ISCKAMn = Adaptive ISC correction for each load condition n, where  n  is

the  value  of  ISFLAG.   The calculated value of ISCKAMn is added to the

total desired idle air flow (DESMAF).

-  ISCTMR = RPM sample timer for  adaptive  ISC,  (0.125  secs).   Timer  is

cleared if |RPMERR_A| exceeds the rpm deadband, RPMDED.

-  ISKSUM = CHECKSUM  for  adaptive  idle  speed  KAM  cells,  used  in  KAM

initialization strategy.

-  RPMERR_A = Filtered value of RPMERR for airflow  control,  time  constant

TCBPA.

Bit Flags:

-  CTPTFG = Closed throttle to part/wide open throttle transistion flag; 1->

transistion occurred

-  HCAMFG = Flag indicating the completion of Hi-Cam; 0 -> no desired engine

speed  adder  exists, 1 -> an rpm adder above base idle is present.  Flag

is used in the ISC adaptive update routine to disable updates when HCAMFG

= 1.

-  ISCFLG = ISC mode indicator flag;  -1  ->  Dashpot  Mode;  0  ->  Dashpot

Preposition Mode; 1 -> Closed Loop RPM Control Mode; 2 -> Closed Loop RPM

Control (Lock-out entry to RPM control)

-  ISFLAG = Idle load state indicator used to select the ISCKAM cell.

-  ISLAST = Value of ISFLAG from last background pass.

-  ISCKAM_HLT = ISCKAM learning override flag.  When set ISCKAM learning  is

disabled.

-  KAM_ERROR = KAM error flag; 1 -> KAM data invalid.

Calibration Constants:

-  PSIBRM = Maximum allowed value for ISCKAMn.  ISCKAMn is clipped  to  this

value.

-  PSIBRN = Minimum allowed value for ISCKAMn.  ISCKAMn is clipped  to  this

value.

13-30

IDLE SPEED CONTROL, KAM UPDATE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  RPMDED =  Adaptive  ISC  learning  deadband.   Learning  is  disabled  if

RPMERR_A exceeds this deadband.

-  UPDATM = Pacing at which the IPSIBR correction factor is rolled into KAM.

Value is in terms of background loop counts.

-  UPDISC = Time that engine speed must be  within  the  specified  deadband

(RPMDED) prior to KAM update.

CALIBRATION INFORMATION

The following typical values are provided for calibration constants:

-  RPMDED = 50 rpm.

-  UPDATM = 5 background passes.

-  UPDISC = 2 seconds.

PROCESS

STRATEGY MODULE: ISC_ISCKAM_COM1

BEGIN:  ISCKAM_UPDATE

; ISCTMR (Update) timer control:-

; The Full description of the strategy describing ISCTMR

; functionality is given in the Timer chapter. The following

; logic is actually implemented within the ISCKAM update code.

|RPMERR_A| > RPMDED --------|

; speed outside range       |AND -| ISCTMR := 0

|     |

ISCFLG = 1 -----------------|     |

; In Closed loop RPM control      |

| --- ELSE ---

|

| ; Increment free

| ; running ISCTMR

; ISCKAM_HLT flag update:

unconditionally ------------------| Do: ISCKAM_HLT_UPDATE

| ; update ISCKAM_HLT flag

13-31

IDLE SPEED CONTROL, KAM UPDATE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

; KAM Error check, Update IBGPSI and determine whether time to update KAM

KAM_ERROR = 1 --------------------|

|

ISCKAMn < PSIBRN -----------------|

;less than minimum               |OR --| Do: ISCKAM_CLEAR

|     |  ;ISCKAM is invalid

ISCKAMn > PSIBRM -----------------|     |  ;kam check failed

;greater than maximum                  |  ;or ISC cells

|  ;corrupted

| IBGPSI := 0

|  ;zero counter

|

| --- ELSE ---

ISCFLG <> 1 ----------------------|     |

;not in closed loop ISC           |     |

|     |

ISCTMR < UPDISC ------------------|     |

;speed not in range long enough   |     |

|     |

HCAMFG = 1 -----------------------|     |

;HICAM mode- learning disabled    |OR --| IBGPSI := 0

|     | ;zero counter

ISCKAM_HLT = 1 -------------------|     |

;ISCKAM learning disabled         |     |

|     |

IPSIBR = 0 -----------------------|     |

;no integral correction           |     |

|     |

ISFLAG <> ISLAST -----------------|     |

;Load State Transistion           |     |

|     |

CTPTFG = 1 -----------------------|     |

;Transistion from closed throttle       |

| --- ELSE ---

|

IBGPSI >= UPDATM -----------------------| IBGPSI := 0

;time to update KAM                     | IAC_ERR3 := 0

| ;clear IAC KAM error flag

| Do: ISCKAM_FINAL

|

| --- ELSE ---

|

| IBGPSI := IBGPSI + 1

| ; increment KAM update counter

| ; and clip at maximum.

END: ISCKAM_UPDATE

13-32

IDLE SPEED CONTROL, KAM UPDATE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ISCKAM_CLEAR

;This process is executed only when called from, or via, ISCKAM_UPDATE

Unconditionally --------------------------|  ISCKAM(0) := 0.0

|  ISCKAM(1) := 0.0

|  ISCKAM(2) := 0.0

|  ISCKAM(3) := 0.0

| ;Assume ISCKAM cells are invalid

| ; and reinitialize to zero

| ISKSUM := 0

| ; Zero check-sum

END: ISCKAM_CLEAR

BEGIN: ISCKAM_FINAL

;This process is executed only when called from ISCKAM_UPDATE

IPSIBR > 0 -----------------------|

|AND -| Increment ISCKAMn

ISCKAMn < PSIBRM -----------------|     | Increment ISKSUM

| Decrement IPSIBR

|

| --- ELSE ---

IPSIBR <= 0 ----------------------|     |

|AND -| Decrement ISCKAMn

ISCKAMn > PSIBRN -----------------|     | Decrement ISKSUM

| Increment IPSIBR

|

| --- ELSE ---

ISCKAMn  = PSIBRN ----------------|     |

|OR --| IAC_ERR3 := 1

ISCKAMn  = PSIBRM ----------------|     | ;set error code

;KAM at limits                         | ;kam on limit

|

| --- ELSE ---

|

| No Action

END: ISCKAM_FINAL

13-33

IDLE SPEED CONTROL, ISCKAM_HLT FLAG CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

13.3.2  ISCKAM_HLT FLAG CONTROL (CDAN0)

OVERVIEW

This section describes the determination of the  ISCKAM_HLT.   This  flag  is

used to disable KAM learning.  This version of the module returns the enable/

disable learn status based on PCOMP requirements.

DEFINITIONS

Bit Flags:

-  FLOW_RUNNING = Purge flow monitor test is running  flag;  1  ->  test  is

running.

-  ISCKAM_HLT = ISCKAM  learning  override  flag;  used  to  disable  ISCKAM

learning when set.

-  PCOMP_ENA = PCOMP strategy enabled flag; 1 -> PCOMP is enabled;  adaptive

fuel disabled.

Calibration Constants:

-  PRG_FLOW_SW =  Calibration  switch  indicating  if  Purge  Flow  test  is

present.

-  PUR_ENA_SW = Purge enable switch for adaptive ISC control; 0 ->  adaptive

ISC  is enabled during purge, 1 -> IPSIBR is reset after purge and ISCKAM

is disabled during purge, 2 -> ISCKAM is  disabled  during  purge.   Note

that ISCKAM learning is enabled/disabled via the ISCKAM_HLT flag.

PROCESS

STRATEGY MODULE: ISC_ISCKAM_HLT_COM2

BEGIN:  ISCKAM_HLT_UPDATE

PRG_FLOW_SW = 1 ------------|

;Test is valid              |AND -|

|     |

FLOW_RUNNING = 1 -----------|     |

;purge flow test is active        |OR --| ISCKAM_HLT = 1

|     | ; Disable ISCKAM

PUR_ENA_SW >= 1 ------------|     |     | ; learning

|AND -|     |

PCOMP_ENA = 1 --------------|           |

;Pcomp is enabled                       |

| --- ELSE ---

|

| ISCKAM_HLT := 0

| ; Enable ISCKAM

| ; learning

13-34

IDLE SPEED CONTROL, ISCKAM_HLT FLAG CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

END: ISCKAM_HLT_UPDATE

13-35

IDLE SPEED CONTROL, IPSIBR CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

13.3.3  IPSIBR CALCULATION (CDAN0)

OVERVIEW

IPSIBR is the closed loop integration component of total DESMAF.  Designed to

provide  integral feedback, IPSIBR adjusts the value of DESMAF to correct for

sustained changes in idle load.  An increase or decrease in IPSIBR results in

a corresponding change to bypass valve duty cycle.

The IPSIBR calculation has the following characteristics:

-  IPSIBR is only updated in rpm control or lockout mode.

-  Different time constants are used depending on whether RPM is too high or

too low.

-  IPSIBR is not updated if the ISC valve is at its  maximum  position,  i.e

ISCDTY  >  99%.   Nor  is  it updated if the requested idle mass airflow,

DEBYMA, is below the minimum clip, DEBYCP.

-  IPSIBR is not updated if  the  closed  loop  air  mass  correction  term,

IPSIBR,  is  already  at its maximum or minimum value (PSIBRM and PSIBRN,

respectively).

-  On load state transitions (A/C, N/DR and CT/PT), IPSIBR is always clipped

to zero as a minimum.

-  The IPSIBR update has no deadband.  Instead, it is driven by an RPM error

term RPMERR_A (A for "air").  The time constant is TCBPA.

-  Calibration of gain is not required.  The term (DESMAF_PRE /  DSDRPM)  in

the   ISCPSI   calculation   automatically  adjusts  the  gain  for  RPM,

temperature and accessory loads.

-  The IPSIBR pacing calibration is controlled by the  two  scalars  TC_OVER

and  TC_UNDER  for  speed  higher  and  lower than desired, respectively.

These scalars represent time constants  for  the  engine  to  respond  to

changes in duty cycle.

-  This module also determines if an OBDII  error  condition  has  occurred.

IAC_ERR1  and  IAC_ERR2  are  used to indicate over/underspeed conditions

respectively.

-  To eliminate speed hang up on entry to  c/loop  ISC  caused  by  abnormal

loading  when  last  in  c/loop,  an  optional  IPSIBR  clip  is provided

(IPSIMAX).  The clip functions  in  dashpot  and  dashpot  prepositioning

load.   This  concern may be apparent with a high clutch switch operating

point.  It is important that the clip is calibrated high enough  to  cope

with the green engine condition.

13-36

IDLE SPEED CONTROL, IPSIBR CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  BG_TMR = Background loop time, secs.

-  DASPOT = Dashpot contribution to idle air flow, ppm.  Used to  provide  a

preposition air flow in Part Throttle and Wide Open Throttle Modes, which

is "bled off" after a transition to Closed Throttle.   This  gradual  air

decrement allows a smooth transition to RPM control.

-  DEBYMA = Desired airflow through the ISC actuator (ppm).

-  DESMAF = Total desired idle air flow, ppm.   Calculated  as  the  sum  of

predicted  air flow (DESMAF_PRE), dashpot air flow (DASPOT), integral air

flow (IPSIBR) and KAM correction (ISCKAMn).

-  DESMAF_PRE = Predicted desired idle air flow, ppm.  This is the open loop

air  flow  prediction which is required to idle, calculated as a function

of ECT and time since start and including A/C, power steering and  heated

windshield   adders.   It  does  not  include  any  closed  loop  or  KAM

corrections.

-  DSDRPM = Desired RPM.  Used as the control  speed  for  closed  loop  RPM

control.

-  FLOW_RUNNING = Purge flow  monitor  test  is  running  flag;  1  ->  test

running.

-  IPSIBR = The closed loop integration  component  of  total  DESMAF,  ppm.

Designed to provide integral feedback, IPSIBR adjusts the value of DESMAF

to correct for sustained changes in idle load.  An increase  or  decrease

in IPSIBR results in a corresponding change to bypass valve duty cycle.

-  ISC_A_DER = Derivative control term used to modify DESMAF.  Contains  the

output of FN851A.

-  ISC_A_PROP = Proportional control term used to modify  DESMAF.   Contains

the output of FN852A.

-  ISCKAM = Adaptive correction for each load condition.

-  ISCPSI = The quantity of air which is added  to  IPSIBR  each  background

pass.

-  ISLAST = Register which tracks the state of engine load from the previous

background  pass.   Used in determining when it is necessary to increment

the filtered air mass (FAM) and clip the C/L idle speed integrator  to  a

minimum value.

-  RPMERR = RPMERR(DESIRED RPM - N) for spark feedback.

-  RPMERR_A = Filtered value of RPMERR, time constant TCBPA.

-  RPMERR_DAO = Old value of RPMERR_DA, for input to FN851A.

-  RUNUPTMR = Time since runup rpm exceeded (0.125 secs).

13-37

IDLE SPEED CONTROL, IPSIBR CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Bit Flags:

-  CTPTFG = Closed throttle to Part/Wide Open Throttle transition flag; 1 ->

Transition occurred.

-  IAC_ERR1 = IAC error flag used for malfunction detection - overspeed.

-  IAC_ERR2 = IAC error flag used for malfunction detection - underspeed.

-  ISCFLG = ISC MODE Flag; -1 -> Dashpot  mode,  0  ->  Dashpot  Preposition

mode, 1 -> RPM Control mode, 2 -> RPM Control Lockout mode.

-  ISFLAG = Idle load indicator, set according to the current state  of  the

A/C (on or off) and of the transmission (neutral or drive).

-  LPCOMP_ENA = State of PCOMP_ENA last background pass.

-  PCOMP_ENA = PCOMP strategy enabled flag.

-  PDL_ERROR = PRNDL error detected and verified.

Calibration Constants:

-  DEBYCP = Minimum value of ISC valve airflow, ppm.

-  FN851A(RPMERR_DAO - RPMERR) = Derivative control function used to  modify

DESMAF.

-  FN852A(RPMERR) = Proportional control function used to modify DESMAF.

-  IAC_HI_LIM = Level of RPMERR_A to set fault flag IAC_ERR1.

-  IAC_LO_LIM = Level of RPMERR_A to set fault flag IAC_ERR2.

-  IPSIMAX  =  Maximum  IPSIBR  clip  value  when  in  dashpot  or   dashpot

preposition modes.

-  IPSI_SW = Calibration switch to use IPSIMAX; 1 -> IPSIBR  is  clipped  to

IPSIMAX in dashpot and dashpot preposition modes.

-  IPSIDLY = Time delay to disable IPSIBR update, secs.

-  PRG_FLOW_SW = Calibration switch indicating if Purge Flow test is present

-  PUR_ENA_SW = Purge enable switch for adaptive ISC control;  0  ->  ISCKAM

enabled, 1 & 2 -> Freeze ISCKAM; 1 -> Reset IPSIBR.

-  PSIBRM = Maximum allowed value for IPSIBR when in normal strategy.

-  PSIBRN = Minimum allowed value for IPSIBR when in normal strategy.

-  TC_OVER = Time constant used to control the integral gain (or pacing)  of

the  term,  IPSIBR.  TC_OVER is used when RPMERR <= 0, ie when the actual

speed is higher than desired.  A large value of TC_OVER corresponds to  a

small integral gain, a small value corresponds to a high gain.

-  TC_UNDER = As with TC_OVER but for overspeed conditions, i.e RPMERR > 0.

13-38

IDLE SPEED CONTROL, IPSIBR CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: ISC_IPSIBR_COM2

BEGIN: IPSIBR_CALC

; Set integrator error flags & calculate integral update term, ISCPSI

ISCFLG <= 0 ------------------------|

;dashpot or dashpot preposition    |OR --|  ;do not update IPSIBR unless

|     |  ;in RPM Control or RPM Control

RUNUPTMR < IPSIDLY -----------------|     |  ;lockout.

;delay IPSIBR update after start         | ISCPSI = 0

| IAC_ERR1 = 0

| IAC_ERR2 = 0

|

| --- ELSE ---

PDL_ERROR = 0 ----------------------|     |

|     |

RPMERR_A < IAC_HI_LIM --------------|AND -| IAC_ERR1 = 1

;large overspeed error             |     | ;Set malf. overspeed flag.

|     | ;flag used in FMEM test

DEBYMA <= DEBYCP -------------------|     | ISCPSI = 0

;actuator airflow at minimum             | ;actuator minimum airflow,

| ;no change to IPSIBR

|

| --- ELSE ---

RPMERR_A >= IAC_LO_LIM -------------|     |

;large underspeed error            |     |

|AND -| IAC_ERR2 = 1

IPSIBR >= PSIBRM -------------|     |     | ;Set malf. underspeed flag.

;IPSIBR at limit              |OR --|     |  which is used in FMEM test

|           | ISCPSI = 0

ISCDTY > 0.99 ----------------|           | ;no change to IPSIBR as

; Valve fully open                        |  integrator at maximum

|  or valve fully open

|

| --- ELSE ---

RPMERR_A < 0 -----------------|           |

;overspeed error             |AND -|     |

|     |     |

DEBYMA <= DEBYCP -------------|     |     |

;actuator airflow at minimum       |OR --| ISCPSI = 0

|     | ;actuator minimum airflow

RPMERR_A >= 0 ----------------|     |     | ;or

;underspeed error            |     |     | ;integrator at maximum

|AND -|     | ;or

IPSIBR >= PSIBRM -------|     |           | ;Valve fully open

;IPSIBR at limit        |OR --|           | IAC_ERR1 = 0

|                 | IAC_ERR2 = 0

ISCDTY > 0.99 ----------|                 | ;clear error flags

; Valve fully open                        | ;rpm error limits not

| ;exceeded

|

| --- ELSE ---

|

| IAC_ERR1 = 0

| IAC_ERR2 = 0

|  ;OK to adjust IPSIBR

| DO: ISCPSI_CALC

13-39

IDLE SPEED CONTROL, IPSIBR CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;Calculate minimum integrator clip

ISFLAG <> ISLAST -----------------|

(load state transition)          |OR --| min_clip := 0.0

|     |  ;Set IPSIBR min. clip to zero

CTPTFG = 1 -----------------------|     |

(transition out of CT)                 |

| --- ELSE ---

|

| min_clip := PSIBRN

|  ;Set IPSIBR min clip to PSIBRN

;Calculate maximum integrator clip:

IPSI_SW = 1 ----------------------|

;clip required                   |AND -| max_clip := IPSIMAX

|     |

ISCFLG <= 0 ----------------------|     |

;dashpot or dashpot pre-position       |

| --- ELSE ---

|

| max_clip := PSIBRM

;Determine IPSIBR:

PRG_FLOW_SW = 0 --------------|

;Test not valid               |OR --|

|     |

FLOW_RUNNING = 0 -------------|     |

; Don't allow reset if set.         |

|

IPSIBR < 0.0 -----------------------|

;reset won't cause stumble          |

|

LPCOMP_ENA = 1 ---------------------|AND -| IPSIBR := 0.0

;pcomp enabled last loop            |     |  ;If it is non-positive,

|     |  ;and flow test is not

PCOMP_ENA = 0 ----------------------|     |  ;running, or flow test is

;pcomp is off                       |     |  ;not valid,  ..then ...

|     |  ;reset IPSIBR on exiting purge.

PUR_ENA_SW = 1 ---------------------|     |

;ipsibr reset switch                      |

| --- ELSE ---

|

| ipsibr_tmp := IPSIBR + ISCPSI

| IPSIBR := CLIP(ipsibr_tmp,

|           min_clip,max_clip)

| ; Clip IPSIBR between limits

13-40

IDLE SPEED CONTROL, IPSIBR CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUE: IPSIBR_CALC

;Calculate Proportional and Derivative ISC terms

ISCFLG > 0 -----------------------|

(RPM control or lockout)         |AND -| ISC_A_PROP = FN852A(RPMERR)

|     | ISC_A_DER =

RUNUPTMR >= IPSIDLY --------------|     |  FN851A(RPMERR_DAO - RPMERR)

(delay control after runup)            |

| --- ELSE ---

|

| ISC_A_PROP = 0

| ISC_A_DER = 0

;unconditionally -------------------------| desmaf_pid := ISC_A_PROP +

|               ISC_A_DER  +

|               IPSIBR +

|               ISCKAM(ISFLAG)

;Determine DESMAF

unconditionally --------------------------| DESMAF := DESMAF_PRE + desmaf_pid

|          +  DASPOT

Set LPCOMP_ENA flag

unconditionally --------------------------| LPCOMP_ENA = PCOMP_ENA

END: IPSIBR_CALC

BEGIN:  ISCPSI_CALC

;This process is executed only when called from IPSIBR_CALC

;Calculate ISCPSI

RPMERR_A >= 0 ----------------------| ISCPSI = RPMERR_A * DESMAF_PRE/DSDRPM *

|          BG_TMR/TC_UNDER

|

| --- ELSE ---

|

| ISCPSI = RPMERR_A * DESMAF_PRE/DSDRPM *

|          BG_TMR/TC_OVER

END: ISCPSI_CALC

13-41

IDLE SPEED CONTROL, ISLAST AND ISFLAG  LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

13.3.4  ISLAST And ISFLAG LOGIC (CDAL0)

OVERVIEW

This logic control the KAM update of ISC cells and  which  adaptive  cell  is

active.

DEFINITIONS

Bit Flags:

-  AC_PPM_RQST = Flag to request an air adder to compensate for A/C load;  1

-> A/C air requested.

-  DNDSUP = Delayed neutral/drive flag; 0 -> in neutral, no load,  1  ->  in

drive, loaded.

-  ISFLAG = Flag that indicates the degree of loading on the engine at Idle;

0 -> Drive, 1 -> Drive + A/C (WAC Relay De-Energized), 2 -> Neutral, 3 ->

Neutral + A/C (WAC Relay De-Energized).

-  ISLAST = Register which indicates the engine load state from the previous

background pass.

-  TRLOAD = Transmission Load Switch.

PROCESS

STRATEGY MODULE: ISC_ISFLAG_COM1

BEGIN: ISFLAG_UPDATE

ISLAST:

unconditionally ------------------------| ISLAST := ISFLAG

| ; Represents the value of ISFLAG

| ; from the last program pass.

13-42

IDLE SPEED CONTROL, ISLAST AND ISFLAG  LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

ISFLAG:

Indicates load state.  Used to determine which adaptive idle cell is active.

AC_PPM_RQST = 0 ------------------|

|

DNDSUP = 1 -----------------------|AND -| ISFLAG := 0

|     | ; Auto in drive

TRLOAD > 3 -----------------------|     |

| --- ELSE ---

AC_PPM_RQST = 1 ------------------|     |

|     |

DNDSUP = 1 -----------------------|AND -| ISFLAG := 1

|     | ; Auto in drive A/C requested

TRLOAD > 3 -----------------------|     |

| --- ELSE ---

AC_PPM_RQST = 0 ------------------|     |

|AND -| ISFLAG := 2

DNDSUP = 0 -----------------|     |     | ; Manual or Auto in neutral

|OR --|     |

DNDSUP = 1 -----------|     |           |

|AND -|           |

TRLOAD <= 3 ----------|                 |

| --- ELSE ---

AC_PPM_RQST = 1 ------------------|     |

|AND -| ISFLAG := 3

DNDSUP = 0 -----------------|     |     | ; Manual or Auto in neutral

|OR --|     | ; A/C requested

DNDSUP = 1 -----------|     |

|AND -|

TRLOAD <= 3 ----------|

END: ISFLAG_UPDATE

13-43

IDLE SPEED CONTROL, RPM ERROR CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

13.3.5  RPM ERROR CALCULATION (CDAL0)

OVERVIEW

Three separate RPM  error  calculations  are  executed  in  RPMERR_CALC.   An

instantaneous  value  (RPMERR)  is  calculated  as the difference between the

desired and actual RPM -- positive value of RPMERR is an engine  speed  below

desired, a negative value indicates engine speed above desired.

Three filtered values of the instantaneous  RPMERR  are  then  obtained  with

three separate time constants:

-  RPMERR_A:  For the bypass air IPSIBR calculation ...  tc = TCBPA

-  RPMERR_DA:  For the bypass air derivative term ......  tc = TCCBPA_DER

In addition, RPMERR_CALC contains the set logic for RUNUP_FLG, which is  used

to  disable  IPSIBR updates and feedback spark during the initial runup.  The

flag is cleared in CRANK/UNDERSPEED/RUN mode selection.

DEFINITIONS

INPUTS

Registers:

-  DSDRPM = Desired engine speed.  See overview section  for  definition  of

the various uses of this register.

-  DSDRPM_LST = DSDRPM last pass.

-  ECTCNT = Number of times the ECT sensor input was read.

-  ISCFLG = ISC mode indicator flag;  -1  ->  Dashpot  mode,  0  ->  Dashpot

Preposition Mode, 1 -> Closed Loop RPM Control Mode, 2 -> Closed Loop RPM

Control (Lock-out entry to RPM control).

-  N = Engine RPM.

-  RPMERR = RPMERR(DESIRED RPM - N) for spark feedback

-  RPMERR_A = Filtered rpm error for bypass air calculations.

-  RPMERR_DA = Filtered RPM for ISC_A_DER DERTERM.

Bit Flags:

-  ALT_CAL_FLG = Flag to indicate use of alternate calibration.

-  RUNUP_FLG = Flag  indicating  initial  runup  is  complete;  1  ->  runup

complete.

13-44

IDLE SPEED CONTROL, RPM ERROR CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  DSDRPM_OFF = Offset from DSDRPM for derivative air control, rpm.

-  RUNUP_DIFF = RPM difference from DSDRPM to set RUNUP_FLG = 1.

-  RUNUP_DIFF_A = Alternate calibration RPM difference from  DSDRPM  to  set

RUNUP_FLG = 1.

-  TCBPA = Time constant for RPMERR_A.

-  TCBPA_DER = Time constant for RPMERR_DA.

OUTPUTS

Registers:

-  RPMERR = Instantaneous rpm error (DSDRPM - N).

-  RPMERR_A = See above.

-  RPMERR_DA = Filtered RPM for ISC_A_DER DERTERM.

-  RPMERR_DAO = Old RPMERR_DA

Bit Flags:

-  RUNUP_FLG = See above.

13-45

IDLE SPEED CONTROL, RPM ERROR CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: ISC_RPMERR_COM3

BEGIN: RPMERR_CALC

Unconditionally ----------------| RPMERR := DSDRPM - N

ALT_CAL_FLG = 1 ----------------| pre_runup := RUNUP_DIFF_A

|

| --- ELSE ---

|

| pre_runup := RUNUP_DIFF

DSDRPM = DSDRPM_LST ------|

|AND -| RPMERR_DAO := RPMERR_DA

N < DSDRPM + DSDRPM_OFF --|     | RPMERR_DA := ROLAV(RPMERR,TCBPA_DER)

|

| --- ELSE ---

|

| RPMERR_DAO := RPMERR

| RPMERR_DA := RPMERR

ISCFLG > 0 ---------------------| RPMERR_A := ROLAV(RPMERR,TCBPA)

; RPM control or lockout        | ; calculate RPM error for airflow control

|

| --- ELSE ---

|

| RPMERR_A := RPMERR

END: RPMERR_CALC

BEGIN: RUNUP_FLG_UPDATE

ECTCNT >= 8 --------------|

; TCSTRT OK to use        |

|

N > DSDRPM + pre_runup ---|AND -| RUNUP_FLG := 1

; Runup RPM exceeded      |     | ; initial runup complete

|     |

RUNUP_FLG = 0 ------------|     |

; First time this start)        |

| --- ELSE ---

|

| No change to RUNUP_FLG

END:  RUNUP_FLG_UPDATE

13-46

IDLE SPEED CONTROL HCAMFG LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

13.3.6  HCAMFG LOGIC (CDAL0)

OVERVIEW

This logic determines the state of the HCAMFG flag.  This flag was originally

used  to  denote  that an elevated RPM above base idle was being requested by

the ISC system, e.g cold start conditions.  In  these  circumstances,  ISCKAM

airflow  correction  learning is disabled via the HCAMFG flag.  The status of

the flag also has implications within the adaptive fuel strategy - see HCAMSW

and the 'special idle KAM cells'.

DEFINITIONS

Registers:

-  HICAM = symbol used to represent the hi-cam adders to DSDRPM.

Bit Flags:

-  COOL_FLG =  Flag  indicating  over  temperature  conditions;  1  ->  Over

temperature.

-  HCAMFG = Flag indicating the completion of Hi-Cam; 0 -> no desired engine

speed  adder  exists, 1 -> an RPM adder above base idle is present.  Flag

is used in the ISC adaptive update routine to disable updates when HCAMFG

= 1.

-  IS_RPM_NCLIP = Flag indicating whether  DSDRPM  has  been  clipped  to  a

minimum  value  (Clipped  =  1)  - Used to set HCAMFG in order to disable

ISCKAM learning.

-  POWSFG = Flag used to indicate that power steering load  is  high;  1  ->

power steering on.

13-47

IDLE SPEED CONTROL HCAMFG LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: ISC_HCAMFG_COM1

BEGIN: HCAMFG_UPDATE

HCAMFG Logic: Flag is used to disable updates of adaptive idle cells

(ISCKAM).

HICAM <> 0 -----------------------|

; RPM adders present              |

|

IS_RPM_NCLIP = 1 -----------------|

; DSDRPM has been min.clipped     |OR --| HCAMFG := 1

|     |

POWSFG = 1 -----------------------|     |

;power steering on                |     |

|     |

COOL_FLG = 1 ---------------------|     |

;Over temperature set.                  |

| --- ELSE ---

|

| HCAMFG := 0

END: HCAMFG_UPDATE

13-48

IDLE SPEED CONTROL, DESIRED MASS AIR FLOW CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

13.4  DESIRED MASS AIR FLOW CALCULATION (CDAJ0)

OVERVIEW

The predicted desired mass air flow (DESMAF_PRE)  is  the  airflow  which  is

expected to be required to provide a particular engine speed.  The prediction

is a function of DSDRPM, ECT, ATMR3 (time since  entering  run  mode),  power

steering,  A/C,  and  heated  windshield.   This  term  is  later added to an

integration term (IPSIBR), a dashpot term  (DASPOT),  and  an  adaptive  term

(ISCKAM), to produce the total DESMAF.

FN875N and D are functions of DSDRPM.  This means that, if a DSDRPM adder  is

used  for  powersteering  or  A/C,  the  airflow  to give the rpm increase is

already accounted for in DESMAF_PRE.  AC_PPM, PS_PPM, and  EDF_PPM  represent

only the airflow needed for the increased load, not the increased rpm.

13-49

IDLE SPEED CONTROL, DESIRED MASS AIR FLOW CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  AC_PPM = AC delta air mass, calculated.

-  ATMR3 = Timer which counts up in run mode.  (Reset to 0 only at powerup).

-  CYLOFF_PPM = Airflow compensation for cylinder cutout during fail safe

cooling.

-  EAM_ANTICIP = Requests additional idle air flow when air pump is on.

-  EAM_PPM = Airflow increment required for Secondary Air pump load.

-  ECT = Engine coolant temperature, deg. F.

-  EDF_PPM = Airflow increment required when electro-drive speed fan is on.

-  PS_PPM = Airflow increment required for power steering load.

-  VSBAR = Filtered vehicle speed.

Bit Flags:

-  DNDSUP = Delayed neutral/drive flag; 0 -> in neutral, no load, 1  ->  in

drive, loaded.

Calibration Constants:

-  FN812(N) = EAM airflow adder.

-  FN875D(DSDRPM) = Airflow required for closed throttle operation in drive.

Input to this function is absolute DSDRPM.

** Airflow requirements must be measured as accurately as possible

over a representative population of vehicles.  Data should be

collected over a range of anticipated desired speeds on a stabilized

engine for both neutral and drive (a temperature modulator (FN1862N or

FN1862D) will automatically adjust calibrated airflow to account for

increased requirements at low ambients).

A hot wire airmeter can be remotely mounted to measure airflow

directly over the desired speed range. Equipment is available at

APTL to perform this procedure.

-  FN875N(DSDRPM) = Airflow required for closed throttle operation in

neutral. Input to this function is absolute DSDRPM.

-  FN1862D = Drive airflow multiplier.  Used to compensate for additional

friction at startup.

X = FN020C(ECT)

Y = FN018B(ATMR3)

13-50

IDLE SPEED CONTROL, DESIRED MASS AIR FLOW CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FN1862N = Neutral airflow multiplier.  Used to compensate for  additional

friction at startup.

X = FN020C(ECT)

Y = FN018B(ATMR3)

-  TRLOAD = Transmission Load switch:

0 -> Manual transmission, no clutch or gear switch, NDSFLG = 0

1 -> Manual trans, no clutch or gear switch

2 -> Manual trans, one clutch or gear switch

3 -> Manual trans, both switches

4 -> Automatic trans, NDS

5 -> Automatic trans, NPS

6 -> Automatic trans, 7 position PRNDL

7 -> Automatic trans, PRNDL switches (4EAT)

OUTPUTS

Registers:

-  DESMAF_PRE = Predicted desired idle air flow, ppm.  This is the open loop

air flow prediction which is required to idle, calculated as a function

of

ECT and time since start and including A/C, power steering and heated

windshield adders.  It does not include any closed loop or KAM

corrections,

and is NOT DISPLAYABLE.

13-51

IDLE SPEED CONTROL, DESIRED MASS AIR FLOW CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: ISC_DESMAF_COM1

BEGIN:  DESMAF_PRE_CALC

;FN1862 and FN875 select logic:

TRLOAD <= 3 ------------|

|OR --| desmaf_pre_tmp = FN875N(DSDRPM) *

DNDSUP = 0 -------------|     |              FN1862N(ECT,ATMR3)

;auto trans in neutral       |

| --- ELSE ---

|

| desmaf_pre_tmp = FN875D(DSDRPM) *

|              FN1862D(ECT,ATMR3)

;EAM_PPM logic:

EAM_ANTICIP = 1 --------------| EAM_PPM = FN812

(EAM load anticipation)      |

| --- ELSE ---

|

| EAM_PPM = 0

;Final DESMAF_PRE determination:

unconditionally --------| Do: AC_PPM_CALC

| Do: EDF_PPM_CALC

| Do: CYLOFF_PPM_CALC

| Do: PS_PPM_CALC

| DESMAF_PRE = desmaf_pre_tmp + AC_PPM + EAM_PPM +

|               PS_PPM + EDF_PPM + CYLOFF_PPM

END: DESMAF_PRE_CALC

13-52

IDLE SPEED CONTROL, ACPPM CALCULATION - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

13.5  ACPPM CALCULATION (CDAL0)

OVERVIEW

This module calculates the value of AC_PPM to be used by ISC_DESMAF_COMx  and

supports systems with or without an A/C head pressure transducer input to the

EEC.

The AC_PPM value is zero when there is to be no adder used, ie.   the  AC  is

off.   It  includes  an  RPM  based  term,  to  overcome  the  inertia of the

compressor and terms to compensate for AC Head  Pressure  effects  (based  on

either  ACPRES or ACT dependent on hardware).  A time function is included in

the Inertia compensation calculation.

The actual AC Pressure load on the compressor  is  a  function  of  both  the

differential  pressure  across  it and the pressure level.  Separate pressure

compensation terms are therefore provided for the first engagement (where the

suction and head pressures are equal) and the cycling condition.

DEFINITIONS

Registers:

-  AC_PPM = A/C delta Airmass - calculated.

-  AC_PPM_TMR = Timer for shaping ACPPM air adder, sec.

-  ACPRES = A/C head pressure, PSI.

-  ACT = Air charge temperature deg.  faren.

-  N = RPM.

Bit Flags:

-  AC_PPM_RQST = Request ISC Air for A/C loading.

-  AC_CYCLE_FLG = A/C is cycling when set.

Calibration Constants:

-  ACPSEN_HP = A/C Pressure Sensor Hardware Present Switch.

-  FN881 = A/C airflow as function of ACPRES.

-  FN881NC = A/C airflow as function of ACPRES - Non-cycling condition.

-  FN885 = A/C airflow for Compressor Inertia.

-  FN885TE = Time  dependent  multiplier  for  Compressor  Inertia  Fn885  -

EXTENDED RANGE.

-  FN887 = A/C airflow correction as function of ACT.

13-53

IDLE SPEED CONTROL, ACPPM CALCULATION - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FN887NC = A/C  airflow  correction  as  function  of  ACT  -  Non-cycling

condition.

PROCESS

STRATEGY MODULE: ISC_DESMAF_ACPPM_COM3

BEGIN: AC_PPM_CALC

;AC_PPM logic:

AC_PPM_RQST = 0 ------------------------| AC_PPM = 0

;A/C load not requested                |

| --- ELSE ---

AC_CYCLE_FLG = 0 -----------------|     |

;A/C not cycling                 |AND -| AC_PPM = FN885(N) *

|     |          FN885TE(AC_PPM_TMR)

ACPSEN_HP = 1 --------------------|     |          + FN881NC(ACPRES)

;pressure sensor present               |

| --- ELSE ---

|

AC_CYCLE_FLG = 0  ----------------------| AC_PPM = FN885(N) *

;A/C not cycling - no sensor           |          FN885TE(AC_PPM_TMR)

|          + FN887NC

|

| --- ELSE ---

|

ACPSEN_HP = 1 --------------------------| AC_PPM = FN885(N) *

;sensor present - cycling              |          FN885TE(AC_PPM_TMR)

|          + FN881(ACPRES)

|

| --- ELSE ---

|

| AC_PPM = FN885(N) *

|          FN885TE(AC_PPM_TMR) +

|          FN887(ACT)

|           ;no pressure sensor and

|             cycling

;AC_PPM_TMR Control:

AC_PPM_RQST = 0 ------------------------| AC_PPM_TMR = 0

;A/C load not requested                |

| --- ELSE ---

|

| increment AC_PPM_TMR

END: AC_PPM_CALC

13-54

IDLE SPEED CONTROL, EDFPPM CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

13.6  EDFPPM CALCULATION (CDAN0)

OVERVIEW

This module calculates the value of EDF_PPM to be  used  by  ISC_DESMAF_COMx.

The value is zero when there is to be no adder used, ie.  the EDF is off.

DEFINITIONS

INPUTS

Registers

-  FANTMR = Time since EDF requested, sec.

Bit Flags:

-  EDFHP = Electro Drive Fan hardware present switch.

-  LSF_RQST = Flag indicating that the low speed fan is requested.

Calibration Constants:

-  EDF_DELAY = Time delay before adding EDFPPM to DESMAF_PRE.

-  EDFPPM = Airflow increment required when electrodrive fan is on.

OUTPUTS

Registers

-  EDF_PPM = EDF delta airmass, calculated.

PROCESS

STRATEGY MODULE: ISC_DESMAF_EDFPPM_COM1

BEGIN: EDF_PPM_CALC

;EDF_PPM logic:

EDFHP = 0 ------------------------------| do nothing

;No EDF Hardware                       |

| --- ELSE ---

LSF_RQST = 1 ---------------------|     |

|AND -| EDF_PPM = EDFPPM

FANTMR >= EDF_DELAY --------------|     |

| --- ELSE ---

|

| EDF_PPM = 0

13-55

IDLE SPEED CONTROL, EDFPPM CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

END: EDF_PPM_CALC

13-56

IDLE SPEED CONTROL, CYLPPM CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

13.7  CYLPPM CALCULATION (CDAJ0)

OVERVIEW

This module calculates the value of CYLOFF_PPM to be used by ISC_DESMAF_COMx.

This  is  part  of  the  failsafe  cooling  strategy  and adds inlet air as a

function of the number of inoperative engine cylinders.  The  value  is  zero

under normal operating conditions.

DEFINITIONS

INPUTS

Bit Flags:

-  COOL_FLG = Flag indicating engine overheating; 1 -> Overheating.

Calibration Constants:

-  FN813 = Airflow adder as a function of INJ_ON - cylinder cutout.

OUTPUTS

Registers

-  CYLOFF_PPM = Airflow Compensation for cylinder cutout.

PROCESS

STRATEGY MODULE: ISC_DESMAF_CYLPPM_COM1

BEGIN: CYLOFF_PPM_CALC

COOL_FLG = 1 ---------------------------| CYLOFF_PPM = FN813

|

| --- ELSE ---

|

| CYLOFF_PPM = 0

END: CYLOFF_PPM_CALC

13-57

IDLE SPEED CONTROL, PS_PPM CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

13.8  PS_PPM CALCULATION (CDAJ0)

OVERVIEW

This module calculates the value of PS_PPM to be used by ISC_DESMAF_COMx.  It

is  for  use  with  a Power Steering Pressure Switch.  The value is zero when

there is to be no adder used, ie.  the PS is off.  The compensation  consists

of two different calibrateable constants, dependant upon vehicle speed.  When

the vehicle is stopped or moving slowly, a shaped adder is also applied based

upon the time since the PS was engaged.

DEFINITIONS

INPUTS

Registers:

-  PS_PPM = PS delta air mass.

-  VSBAR = Filtered vehicle speed.

Bit Flags:

-  POWSFG = Power steering flag.

Calibration Constants:

-  PSPSHP = Power steering pressure switch "Hardware Present" indicator (0 =

NO, 1 = YES).

-  PS_VS = Speed below which PS PPM addition is enabled units are MPH.

OUTPUTS

Registers:

-  PS_PPM = See above.

-  PSPPM = FAM delta AM for power steering load units are LB/MPH.

-  PSPPM_VS = Power steering air adder with vehicle speed moving, lb/sec.

13-58

IDLE SPEED CONTROL, PS_PPM CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: ISC_DESMAF_PSPPM_COM1

BEGIN:  PS_PPM_CALC

;PS_PPM logic:

PSPSHP = 0 -----------------------------| do nothing

;PSPS not present                      |

| --- ELSE ---

|

POWSFG = 0 -----------------------------| PS_PPM = 0

;power steering off                    |

|

| --- ELSE ---

|

VSBAR < PS_VS --------------------------| PS_PPM = PSPPM + FN822(PSTMR)

;vehicle stationary                    |

| --- ELSE ---

|

| PS_PPM = PSPPM_VS

END: PS_PPM_CALC

13-59

IDLE SPEED CONTROL, DASHPOT CALCULATIONS - CDAN2

PED-PTE, PROPRIETARY & CONFIDENTIAL

13.9  DASHPOT CALCULATIONS (CDAJ0)

OVERVIEW

Dashpot Pre-position

Logic controlling the dashpot pre-position airflow is  intended  to  increase

the  ISC  duty cycle during part/WOT operation.  Strategy determines the rate

at which ISC valve flow increases/decreases in part/WOT operation as well  as

the  maximum  allowed  pre-position  airflow.   Adequate pre-position airflow

(DASPOT) is essential prior to entering the dashpot control mode in order  to

avoid  HC  spiking  and/or  deceleration stalls.  The calculated pre-position

airflow increment is added to an adaptively corrected idle  flow  requirement

(DESMAF)  prior  to  output  of  the  ISC  duty  cycle.  Pre-position airflow

(DASPOT) is a function of the difference between a filtered throttle position

(DSTPBR) and the lowest recorded throttle position (RATCH).

DSTPBR is a time dependent rolling average filter of Throttle  position.   It

is updated once per background loop while in RUN or Underspeed Mode.  The two

time constants, TCDASU and TCDASD are  calibratable.   TCDASU  is  used  when

DSTPBR is filtering UP to TP.  TCDASD is used to filter DSTPBR DOWN to TP.

The DASPOT value is adjusted as TP changes to  provide  the  desired  dashpot

action  to  decelerations  as  initiated  over  the  range of possible engine

operating  conditions,  using  separate  time  constants  (TCDASU/TCDASD)  to

control the response of DSTPBR.

Dashpot Bleed

During Closed Throttle Mode, the DASPOT airflow is "bled off" by decrementing

it.   This  action  smooths  the  transition  into  RPM  control by gradually

eliminating the DASHPOT contribution to the Idle airflow, DESMAF.  The  bleed

rate is determined by FN879.

13-60

IDLE SPEED CONTROL, DASHPOT CALCULATIONS - CDAN2

PED-PTE, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  APT = Throttle Mode Flag; -1 -> Closed Throttle, 0 -> Part Throttle,

1 -> Wide Open Throttle.

-  DAS_CTR = Lack of decel recognition  counter.   Used  to  select  between

FN894H and FN894L.

-  DASPOT = Dashpot contribution to  idle  air  flow.   Used  to  provide  a

preposition air flow in Part Throttle and Wide Open Throttle Modes, which

is "bled off" after a transition to Closed Throttle.   This  gradual  air

decrement allows a smooth transition to RPM control.

-  DNDSUP = Delay neutral/drive flag; 0 -> in neutral,  no  load,  1  ->  in

drive, load.

-  DNDT_DAS = Filtered rate of change of RPM for DASPOT.

-  DSDRPM = Desired engine speed.

-  DSTPBR = Time dependent rolling  average  filter  of  throttle  position.

Filtered  using TCDASU when filtering UP to TP, and TCDASD when filtering

DOWN to TP.

-  GEAR4TMR = Time since 3-4 shift, sec.

-  N = Engine revolutions, RPM.

-  N_RATCH = RPM value  which  only  ratchets  down.   When  not  at  closed

throttle,  N = N_RATCH.  When at closed throttle, N_RATCH is only allowed

to go down.  N_RATCH is an input to the  minimum  daspot  clip.   N_RATCH

ratchets down to prevent rpm flares after a declutch.

-  RATCH = Closed throttle position, counts

-  TP = Throttle position sensor.

-  VSBAR = Vehicle speed, MPH.

Bit Flags:

-  FLG_DASCLP4 = FN894H dashpot minimum clip is not applied in  PT/WOT  when

clear and DASCLP_4INH is set.

-  FLG_DASMNQ = VSBAR flip-flop flag for minimum DASPOT clip.

-  GEAR4TH = 4TH gear flag; 1 -> 4th gear.

-  ISCFLG = ISC mode indicator flag;  -1  ->  Dashpot  Mode,  0  ->  Dashpot

Preposition Mode, 1 -> Closed Loop RPM Control Mode, 2 -> Closed Loop RPM

Control (Lock-out entry to RPM control).

13-61

IDLE SPEED CONTROL, DASHPOT CALCULATIONS - CDAN2

PED-PTE, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  BRAKE_DNDT = Threshold for detection of vehicle braking, RPM/sec.

-  DAS_BKDLY = Time delay in 4th gear to recognize vehicle braking, sec.

-  DAS_CNT = Number of DNDT_DAS samples above DAS_DNDT to recognize lack  of

vehicle deceleration.

-  DAS_CTDLY = Closed throttle time delay  before  activating  the  lack  of

decel detection, sec.

-  DAS_DNDT = Threshold for  detection  of  lack  of  vehicle  deceleration,

rpm/sec.

-  DASCLP_4INH = Switch controls the FN894H min daspot clip inhibit logic.

-  DASPTK = Gain associated with the desired DASPOT airflow.   To  calibrate

this  value  first  determine  the throttle position above RATCH at which

maximum DASPOT airflow is desired.  Subtract FN830 from DASMAX and divide

the  result  by the throttle delta between RATCH and this maximum dashpot

airflow to determine the DASPTK value.

-  DELHYS = Closed to part throttle hysteresis in TP counts.  DELHYS  should

be  set equal to DELTA + HYSTS (closed throttle breakpoint).  This starts

the dashpot calculation relative to the C.T./P.T.  breakpoint to  prevent

changes in dashpot when leaving closed throttle.

-  DPNEU_MUL = Neutral DASPOT reduction multiplier.  Used to reduce  minimum

DASPOT during shift of manual transmission.

-  FN830 = Dashpot preposition offset.

-  FN879 = A background driven decrement to the dashpot preposition  airflow

register  (DASPOT)  as  a function of DASPOT.  FN879 can be calibrated to

achieve an exponentially decaying dashpot which is useful in decaying the

large DASPOT values used to control over-rich tip out conditions.

-  FN882A(N - DSDRPM) = Maximum dashpot clip as a function of the RPM  delta

above desired rpm.  (N - DSDRPM) is clipped to 0 as a minimum.

-  FN891(VSBAR) = Dashpot maximum clip as a function of vehicle speed.  Used

in  automatic  transmission  vehicles at higher vehicle speeds to prevent

harsh backout shifts by limiting large values of dashpot.

-  FN894(N_RATCH - DSDRPM) = Minimum DASPOT airflow clip for 1st,  2nd,  and

3rd  gears,  ppm.  For manual transmissions, FN894 can be used to prevent

declutch stalls.  The input is N_RATCH which only ratchets down after the

declutch.   This  is done to prevent the rpm from hanging up if it flares

after the declutch.

-  FN894H(N_RATCH - DSDRPM) = Minimum DASPOT airflow clip for 4th gear  when

vehicle deceleration rate is acceptable, ppm.

-  FN894L(N_RATCH - DSDRPM) = Minimum DASPOT airflow clip for 4th gear  when

lack of vehicle deceleration is detected, ppm.

13-62

IDLE SPEED CONTROL, DASHPOT CALCULATIONS - CDAN2

PED-PTE, PROPRIETARY & CONFIDENTIAL

-  TCDASD = Time constant used when TP is less than or equal to the filtered

TP  value.   Should  be calibrated such that part throttle backouts where

closed throttle is not entered do not exhibit a run-on feel.  Too fast  a

filter  can  have the effect of greatly reducing dashpot airflow prior to

entry into dashpot control.

-  TCDASU = Filter constant used when TP is greater  than  the  filtered  TP

value   (DSTPBR).    The   larger  the  time  constant  the  more  slowly

pre-position airflow will be available  to  respond  to  tip  in/tip  out

actions.   Fast  response can also be obtained by use of the offset value

FN830 without the potential runaway feel that may come with  too  fast  a

filter constant/airflow gain (DASPTK) combination.

OUTPUTS

Registers:

-  DASPOT = See above.

-  DSTPBR = See above.

CALIBRATION INFORMATION

Typical values are supplied for the following calibration constants:

-  DASPTK = 0.002 ppm/TP count

-  FN879 = (0,0.001) (0.1,0.002) (0.3,0.006) (0.75,0.05) (2.00,0.10)

-  TCDASD = 0.75 sec.

-  TCDASU = 3.3 sec.

13-63

IDLE SPEED CONTROL, DASHPOT CALCULATIONS - CDAN2

PED-PTE, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: ISC_DASPOT_COM5

BEGIN: DASHPOT_EXECUTION_PROCESS

This is the root chart for the ISC_DASPOT_COM5 module.  It controls execution

of  the strategy processes defined in the remainder of the module.  The entry

logic to this process is executed in the ISC overview chapter and is repeated

here for clarity only.

MFMFLG = 0 -----------------------|

;MAF sensor function ok          |OR  -|  ; In normal ISC. See ISC_OVERVIEW

|     |    _COM module

TFMFLG = 0 -----------------------|     |

;TP function ok                        | DO: NO_DECEL_COUNTER

| DO: DSTPBR_CALC

| DO: BASE_DASHPOT_CALCULATION

| DO: DASHPOT_MIN_CLIP

|

| --- ELSE ---

|

| NO ACTION

END: DASHPOT_EXECUTION_PROCESS

BEGIN: NO_DECEL_COUNTER

LACK OF DECEL COUNTER LOGIC

GEAR4TH = 0 -----------------------|

(1st, 2nd, or 3rd gear)           |OR --| DAS_CTR = 0

|     |

CTTMR <= DAS_CTDLY ----------------|     |

(minimum dashpot time)                  |

| --- ELSE ---

GEAR4TH = 1 -----------------------|     |

(4th gear)                        |     |

|     |

CTTMR > DAS_CTDLY -----------------|AND -| Increment DAS_CTR

(minimum dashpot time)            |     |  (clip to 255 maximum)

|     |

DNDT_DAS > DAS_DNDT ---------------|     |

(no decel)                              |

| --- ELSE ---

|

| Freeze DAS_CTR

END: NO_DECEL_COUNTER

13-64

IDLE SPEED CONTROL, DASHPOT CALCULATIONS - CDAN2

PED-PTE, PROPRIETARY & CONFIDENTIAL

BEGIN: DSTPBR_CALC

DSTPBR is the rolling average of throttle position used to calculate  Dashpot

in  pre-position  mode.   Separate  time  constants  are used for opening and

closing throttle conditions.

TP > DSTPBR ----------------| DSTPBR = rolav(TP,TCDASU)

|

| --- ELSE ---

|

| DSTPBR = rolav(TP,TCDASD)

END: DSTPBR_CALC

BEGIN: BASE_DASHPOT_CALCULATION

While at PT or WOT, airflow is added to the DASPOT  term  to  prepare  for  a

deceleration.   At closed throttle, the airflow which was previously added to

the DASPOT term is bled away.

APT >= 0 ---------------------| DASPOT = DASPTK * (DSTPBR - (RATCH + DELHYS))

;PT or WOT,                   |          + FN830(N-DSDRPM)

Dashpot Pre-position         |

mode.                        | DO: DASPOT_PREPOS_CLIP_MAX

|

| --- ELSE ---

|

| DASPOT = DASPOT - FN879(DASPOT)

|  ;Dashpot mode

END: BASE_DASHPOT_CALCULATION

BEGIN: DASPOT_PREPOS_CLIP_MAX

(FN882A(N-DSDRPM) *

FN891(VSBAR)) < 1.99 --------| Clip DASPOT to (FN822A(N-DSDRPM) *

;(N-DSDRPM) is clipped       | FN891(VSBAR)) as a maximum.

to zero min.                |

| --- ELSE ---

|

| Clip DASPOT to 1.99 as a maximum

END: DASPOT_PREPOS_CLIP_MAX

13-65

IDLE SPEED CONTROL, DASHPOT CALCULATIONS - CDAN2

PED-PTE, PROPRIETARY & CONFIDENTIAL

BEGIN: DASHPOT_MIN_CLIP

MINIMUM DASPOT CLIP LOGIC

The minimum value for DASPOT can come from one of three functions, or it  can

be  zero  (no clip).  All three functions use relative rpm above DSDRPM as an

input.  One function, FN894, is used for all gears except 4th.  4th gear  has

two  unique  functions which are designed to allow the calibrator to optimize

tip-in/out clunk feel while still having acceptable  run-on  feel.   The  two

functions,  FN894H  &  FN894L (for high & low), are intended to be calibrated

such that FN894H gives good clunk  control,  (close  to  the  zero  driveline

torque  line), while FN894L is lower, to eliminate vehicle run-on feel.  When

a decel is begun in 4th gear, FN894H is always  used  first.   If  the  logic

detects  a lack of deceleration, the minimum clip is dropped to FN894L.  Once

the clip has been dropped to FN894L, it does not  go  back  to  FN894H  until

reset  by  a part throttle or a downshift.  FN894 is always used in 1st, 2nd,

and 3rd gears.  Certain special conditions will cause the  dashpot  to  bleed

all  the way down to zero (no clip).  The flag FLG_DASCLP4 is used to prevent

the application of a high minimum daspot clip from FN894H when exiting closed

throttle in fourth gear and is provided to overcome interaction concerns with

the cruise control system.

Manual transmission packages do not use FN894H and  FN894L.   In  this  case,

FN894 is intended to be used to prevent de-clutch stalls.

APT >= 0 ----------|

; Part or Wide    |

Open throttle   |AND ------|S  Q-| FLG_DASCLP4

|          |

DASPOT >= FN894H --|          |

|

APT = -1 ---------------------|C

: Closed throttle

13-66

IDLE SPEED CONTROL, DASHPOT CALCULATIONS - CDAN2

PED-PTE, PROPRIETARY & CONFIDENTIAL

GEAR4TMR > DAS_BKDLY --------|

(time since 4th gear)       |AND -|

|     |

DNDT_DAS < BRAKE_DNDT -------|     |

(brakes applied)                  |

|

DNDSUP = 0 ------------------|     |

(in neutral or clutch in)   |AND -|

|     |OR --| no minimum clip

TRLOAD > 3 ------------------|     |     |

(automatic transmission)          |     |

|     |

TFMFLG = 1 ------------------------|     |

(failed TP sensor)                |     |

|     |

FLG_DASMNQ = 0 --------------------|     |

(low vehicle speed)               |     |

|     |

DASCLP_4INH = 1 - -----------|     |     |

;Caln switch set            |     |     |

|     |     |

APT >= 0 --------------------|     |     |

Part or WOT                  |AND -|     |

|           |

GEAR4TH = 1 -----------------|           |

|           |

FLG_DASCLP4 = 0 -------------|           |

|

|

| --- ELSE ---

GEAR4TH = 0 -----------------------|     |

(1st, 2nd, or 3rd gear)           |     | clip DASPOT to

|OR --|  FN894(N_RATCH - DSDRPM)

TRLOAD <= 3 -----------------|     |     |  as a minimum

(manual trans)              |AND -|     |

|           |

DNDSUP = 1 ------------------|           |

(in gear)                               |

| --- ELSE ---

|

TRLOAD <= 3 -----------------------------| clip DASPOT to

(Manual trans, in neutral)              |  FN894(N_RATCH - DSDRPM) *

|  DPNEU_MUL as a minimum

|

| --- ELSE ---

|

DAS_CTR >= DAS_CNT ----------------------| clip DASPOT to

(lack of deceleration)                  | FN894L(N_RATCH - DSDRPM)

|  as a minimum

|

| --- ELSE ---

|

| clip DASPOT to

|  FN894H(N_RATCH - DSDRPM)

|  as a minimum

END: DASHPOT_MIN_CLIP

13-67

IDLE SPEED CONTROL, MODE SELECT - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

13.10  MODE SELECT (MODE_SELECT) (CDAN0)

OVERVIEW

Bypass idle speed control has  four  modes  of  operation;  dashpot,  dashpot

preposition,  RPM  control,  and  RPM  control  lockout.   A  flag is used to

identify these modes for both calibrator convenience and required interaction

with fuel modulation and spark feedback strategies.

The mode select logic selects the mode of operation and sets a flag  (ISCFLG)

which  is  used  to  adjust  the total desired airflow through the air bypass

valve.

*  ISCFLG = -1 DASHPOT CONTROL

*  ISCFLG = 0 DASHPOT PRE-POSITION

*  ISCFLG = 1 CLOSED LOOP RPM CONTROL

*  ISCFLG = 2 CLOSED LOOP RPM CONTROL (Lock-out entry to RPM control)

- DASHPOT PRE-POSITION MODE (ISCFLG = 0)

In engine run/underspeed mode and when operating at part  or  wide  open

throttle the ISC system is placed in dashpot pre-position mode.  In this

mode the  ISC  duty  cycle  is  incremented  a  calibratable  amount  in

anticipation  of a required dashpot action.  Proper dashpot operation is

essential on systems having speed density  fuel  controls  in  order  to

avoid tip in/tip out stalls and HC spiking on decels.

- DASHPOT MODE (ISCFLG = -1)

In engine run/underspeed mode and having just transitioned from part  to

closed  throttle  the system is placed in ISC dashpot control mode.  The

length of time the ISC system will remain in  dashpot  control  is  both

hardware/strategy  dependent  (some  applications  have VSS; some manual

transmission applications have gear and clutch switches).  Regardless of

the  length  of  time  required  to enter RPM control, as long as closed

throttle operation is maintained the amount of airflow specified by  the

dashpot pre-position (see dashpot pre- position logic) is decremented at

a constant rate until exhausted (until DASPOT = 0).

- CLOSED LOOP RPM CONTROL (ISCFLG = 1 OR 2)

For normal entry into C/L RPM control the following conditions  must  be

satisfied:

.  If VSS hardware is used it must indicate a speed less than MINMPH

13-68

IDLE SPEED CONTROL, MODE SELECT - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

.  If a manual trans.  with gear/clutch switches; must indicate neutral

13-69

IDLE SPEED CONTROL, MODE SELECT - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

*  Note:  Although  the  system  can  provide  acceptable  function

without  the  above mentioned hardware either item will increase

reliability  in  production.   The  vehicle  speed  sensor   has

calibration  benefits outside of ISC (lean cruise control, etc.)

and should be considered when specifying system assumptions  for

future applications utilizing ISC.

.  Regardless whether the above hardware is used, normal entry into RPM

control  requires  that actual engine speed be less than or equal to

(DSDRPM + RPMCTL) and that closed throttle is indicated.

The following discussion will attempt to describe entry into C/L RPM  control

through  the  lock-out  logic  (ISCFLG  =  2).   In a normal deceleration the

dashpot bleed time will be short relative to the vehicle coastdown time.   As

soon  as  engine  speed  drops  low  enough  the  ISC system should enter RPM

control.  However, due to hysteresis in the bypass  valve,  overspecification

of  idle airflow requirements prior to adaptive ISC learning, ISC learning in

an unusually high state of engine load (400 psi A/C head pressure), etc.  the

ISC  actuator may flow too much air at the specified idle duty cycle to allow

normal entry in RPM control.  When this  condition  occurs  the  system  will

remain in dashpot control until it can recognize that it should in fact be in

RPM control.

Obviously this task is easy if you happen to have a  VSS  or  have  a  manual

calibration  with gear/clutch switches.  The problem without this hardware is

to differentiate between a deceleration condition (especially a constant rate

of  speed  deceleration  --  as  in  a  coast  down  a  mountain)  and a true

locked-out-of-idle condition.  Most  of  the  logic  in  the  above-mentioned

attachment deals with this lock-out feature.

To differentiate between deceleration and idle the rate of change in  RPM  is

first  evaluated  over a calibrated period of time (ISCTM).  If the speed has

remained within a specified deadband (NDIF) for this  time  period  a  second

check  is  performed  to compare LOAD with a calibrated value (LOWLOD for A/C

off; LOWLOD + ACLOD for A/C on).   The  assumption  is  that  all  idle  LOAD

values,  including  green  engine/altitude effects etc., will be greater than

this  value,  and  all  true  deceleration  conditions,  including  the  same

variabilities, will yield lower LOAD.  It goes without saying that great care

must be taken in selecting the correct calibration for LOWLOD.

If the ISC system were locked in dashpot control and both the rate of  engine

speed  change  and  LOAD criteria were satisfied the strategy would be forced

into C/L RPM control with ISCFLG indicating 2.  This state would  be  present

until  the  speed  fell below the normal entry point.  The adaptive ISC would

learn  the  required  correction,  assuming  sufficient  time  at  idle,  and

subsequent  dashpot  to  RPM control transitions should follow a normal entry

path.

13-70

IDLE SPEED CONTROL, MODE SELECT - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  APT = Throttle Mode flag.

-  DASCTL = Value of DASPOT, below which RPM control  can  begin,  with  any

remaining DASPOT airflow being rolled into IPSIBR.

-  DASPOT = Dashpot contribution to  idle  air  flow.   Used  to  provide  a

preposition air flow in Part Throttle and Wide Open Throttle Modes, which

is "bled off" after a transition to Closed Throttle.   This  gradual  air

decrement allows a smooth transition to RPM control.

-  DSDRPM = Desired engine speed.

-  ISCFLG = ISC mode indicator flag;  -1  ->  Dashpot  Mode,  0  ->  Dashpot

Preposition Mode, 1 -> Closed Loop RPM Control Mode, 2 -> Closed Loop RPM

Control (Lock-out entry to RPM control).

-  ISCTMR = RPM sample timer for  lockout  logic,  (0.125  sec).   Timer  is

cleared on each RPM sample.

-  LOAD = Universal LOAD as ratio of air charge over standard.

-  N = Engine speed, RPM.

-  NLAST = Last sampled RPM for lockout logic.  NLAST is re-calculated  when

ISCTMR exceeds ISCTM.

-  SETTMR = RPM control entry delay timer, secs.  Used to delay  entry  into

RPM control until manifold stabilizes.

-  VSBAR = Filtered vehicle speed, mph.

Bit Flags:

-  ACCFLG = A/C engaged flag; 1 -> A/C engaged, 0 -> A/C disengaged.

-  DNDSUP = Delayed neutral/drive flag; 1 -> in drive, loaded.

Calibration Constants:

-  ACLOD = LOWLOD modulator - A/C on decel from idle.

-  ISCTM = Time interval (over which the rate of change in engine  speed  is

evaluated.   -  resolution  0.125  Sec.   Value should be small enough to

avoid prolonged speed hang-ups if the ISC system were locked out  of  C/L

speed  control but not too short such that the rate of speed change check

becomes meaningless.  ..Typical value - 4 sec.

-  LOWLOD = LOAD value to differentiate decel from idle.

13-71

IDLE SPEED CONTROL, MODE SELECT - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  MINMPH = Minimum speed to enter C/L  RPM  control.   Applies  to  systems

having  VSS.   Should be set below the speed at which an automatic trans.

vehicle rolls along in drive without the  brakes.   This  is  to  prevent

going into RPM control during parking lot maneuvers.  ..Typical value - 3

MPH

-  NDIF = The deviation in engine speed allowed  over  the  ISCTM  specified

time  interval.   Values  too  small could lock the ISC system out of C/L

speed control indefinitely.   Values  too  large  invalidate  the  check.

..Typical value - 32 RPM

-  RPMCTL = Added to DSDRPM.  The total defines the engine  speed  threshold

below  which entry into C/L RPM control is allowed.  This value should be

reasonably small to avoid inadvertent  entry  into  C/L  ISC.   ..Typical

value - 90 RPM

-  SETLNG_TM = Manifold stabilization time.  Used to delay  entry  into  RPM

control.

-  TRLOAD = Transmission Load switch;

0 -> Manual transmission, no clutch or gear switch, NDSFLG = 0

1 -> Manual trans, no clutch or gear switch

2 -> Manual trans, one clutch or gear switch

3 -> Manual trans, both switches

4 -> Automatic trans, NDS

5 -> Automatic trans, NPS

6 -> Automatic trans, 7 position PRNDL

7 -> Automatic trans, PRNDL switches (4EAT)

13-72

IDLE SPEED CONTROL, MODE SELECT - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: ISC_MODE_SELECT_COM2

APT >= 0 -----------------------------| ISCFLG = 0

(not closed throttle)                | NLAST = N

| ISCTMR = 0

|  (in dashpot preposition mode)

|

| --- ELSE ---

VSBAR <= MINMPH ----------------|     |

(vehicle stopped)              |     |

|     |

DASPOT <= DASCTL ---------------|     |

(dashpot complete)             |     |

|     |

TRLOAD <> 3 --------------|     |     |

(not man trans           |     |     |

w/both switches)        |     |     |

|OR --|AND -| ISCFLG = 1

DNDSUP = 0 ---------------|     |     |  (in RPM Control Mode)

(neutral)                      |     |

|     |

SETTMR > SETLNG_TM -------------|     |

(manifold stable)              |     |

|     |

N <= DSDRPM + RPMCTL -----------|     |

| --- ELSE ---

VSBAR <= MINMPH ----------------|     |

(vehicle stopped)              |     |

|     |

DASPOT <= DASCTL ---------------|     |

(dashpot complete)             |     |

|     |

TRLOAD <> 3 --------------|     |     |

(not manual transmission |OR --|AND -| ISCFLG = 2

w/ both switches)       |     |     |  (in RPM Control Lockout mode;

|     |     |   same action as RPM Control mode)

DNDSUP = 0 ---------------|     |     |

(neutral)                      |     |

|     |

SETTMR > SETLNG_TM -------------|     |

(manifold stable)              |     |

|     |

LOCKOUT LOGIC TRUE -------------|     |

(locked out of RPM control)          |

|  --- ELSE ---

|

| ISCFLG = -1

|  (in Dashpot mode)

NOTE:  SETTMR and ISCTMR are cleared in MODE_SELECT logic.

13-73

IDLE SPEED CONTROL, MODE SELECT - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

LOCKOUT LOGIC

ISCFLG >= 1 --------------------------| Lockout Logic True

|  (stay in lockout until RPM falls)

| Exit Lockout Logic

|

| --- ELSE ---

|

ISCTMR < ISCTM -----------------------| Lockout Logic False

|  (not time to sample RPM yet)

| Exit Lockout Logic

|

| --- ELSE ---

|

|N - NLAST| > NDIF -------------------| Lockout Logic False

|  (RPM changing quickly,

|   must be decel)

| NLAST = N

| ISCTMR = 0

| Exit Lockout Logic

|

| --- ELSE ---

TRLOAD >= 3 --------------------|     |

(auto trans)                   |     |

|AND -| Lockout Logic True

|     | (can't be decel when in neutral)

DNDSUP = 0 ---------------------|     | Exit Lockout Logic

(in neutral)                         |

| --- ELSE ---

ACCFLG = 1 ---------------|           |

(A/C on)                 |AND -|     |

|     |     |

LOAD < LOWLOD + ACLOD ----|     |     |

(decel LOAD)                   |OR --| Lockout Logic False

|     |  (LOAD indicates decel)

ACCFLG = 0 ---------------|     |     | Exit Lockout Logic

(A/C off)                |AND -|     |

|           |

LOAD < LOWLOD ------------|           |

(decel LOAD)                         |

| --- ELSE ---

|

| Lockout Logic True

| Exit Lockout Logic

13-74

IDLE SPEED CONTROL, PERLOAD_ISC CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

13.11  PERLOAD_ISC CALCULATION (CDAC0)

OVERVIEW

PERLOAD_ISC is used by the ISC logic as the Y  input  to  the  ISCDTY  table,

FN8000(DEBYMA,PERLOAD_ISC).

DEFINITIONS

INPUTS

Registers:

-  BP = Barometric pressure valve stored in KAM.

-  DEBYMA = Desired mass air flow through the ISC valve, ppm.  This quantity

is  calculated  as  the total desired idle air flow (DESMAF) less the air

flow through the throttle plate, and  is  corrected  for  altitide  using

FN890(BP).

-  LOAD = Normalized air meter air mass  flow,  unitless.   Equal  to:   air

mass/(N*ENGCYL*SARCHG).

-  PERLOAD_ISC = Used by ISC logic as the Y input to FN8000.   0-1.0  scale,

Unitless.

Calibration constants:

-  FN035(N) = Peak load at sea level as a function of RPM.

-  FN8000 = Characterization table for the ISC actuator;

X = FN061(DEBYMA)

Y = FN060(PERLOAD_ISC)

OUTPUTS

Registers:

-  PERLOAD_ISC = See above.

PROCESS

STRATEGY MODULE: ISC_PERLOAD_ISC_COM1

LOAD     29.9

PERLOAD_ISC = -------- * ----

FN035(N)    BP

13-75

IDLE AIR CONTROL, FMEM LIMITS TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

13.12  IDLE AIR FEEDBACK CONTROL LIMIT TEST (CDAN0)

OVERVIEW

The purpose of this test is to monitor the Idle Air Feedback  Control  Limits

of  the  actuator,  by  determining  the  maximum duty cycle and the adaptive

limits.  The base strategy has calibratable  limits  which  describes  normal

operation.   Once  these  limits  are met or exceeded, an error is set in the

IPSIBR Calculation or the KAM UPDATE logic.

Three flags have been allocated to indicate  three  separate  failure  modes;

IAC_ERR1  - overspeed error, IAC_ERR2 - underspeed error and IAC_ERR3 - for a

KAM error.

The control of IPSIBR or ISCKAM reaching the maximum or  minimum  limits  are

determined  by  (PSIBRM/PSIBRN).  Each time the update criteria are satisfied

both IPSIBR and ISCKAM are adjusted one bit (0.00024  ppm)  in  the  opposite

directions  until  IPSIBR  = 0.  If correction is needed and the calibratable

limits are met for an  extended  period  of  time  IAC_ERR*  is  set  to  the

appropriate value.

The value of IAC_ERR* and the amount of time at the limit will determine  the

failure  condition.   Three separate error condition timers, IAC_ER_TMR(1-3),

have been allocated for the associated IAC_ERR* failure mode.   Once  failure

has  been  determined, the malfunction flag undergoes a 0 -> 1 transition and

the MIL control logic routine is called.

Please Note:  the IAC_ER_TM2 must be  calibrated  to  allow  detection  of  a

vacuum leak in the inlet system.

Because the effect of airflow on engine speed is load related  and  only  one

set  of  speed  limits  is  provided  to  detect errors, a calibrateable load

comparator, IAC_ISFL_OBD, is provided to allow the error flags to be  cleared

in  low  load  states.   This may be used to limit fault setting to the Drive

condition on auto trans vehicles.  IAC_ISFL_OBD is compared with ISFLAG.

DEFINITIONS

Registers:

-  ECT = Engine Coolant Temperature

-  ER_STATUS = Engine run status register, used to sequence  the  diagnostic

procedure.

-  IAC_ER_TMR1 = Timer that runs only when an IAC_ERR1 fault is present.

-  IAC_ER_TMR2 = Timer that runs only when an IAC_ERR2 fault is present.

-  IAC_ER_TMR3 = Timer that runs only when an IAC_ERR3 fault is present.

-  IAC_ER_TMR = Timer that runs only when an IAC fault is present.

13-76

IDLE AIR CONTROL, FMEM LIMITS TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  IAC_MON_TMR = Timer used to set OBDII monitor  flags  for  completion  of

Idle test

-  ISFLAG = Flag that indicates the degree of loading on the engine at idle;

0  -> Drive, 1 -> Drive and A/C (WAC Relay De-Energized), 2 -> Neutral, 3

-> Neutral and A/C (WAC Relay De-Energized).

-  RUNUPTMR = Time since runup RPM exceeded.

-  USPD_RUN_TMR = Time in underspeed or run mode.

Bit Flags:

-  APT = Throttle Mode flag.

-  CCM_IAC_MON = Monitor flag  for  IAC;  1  ->  all  IAC  fault  have  been

monitored at least once since power-up.

-  CCM_TST_ENA = Idle air test enable switch; 1 -> enable malfunction logic.

-  FFG_ECT = OBDII system FMEM  flag  for  ECT  -  1  ->  ECT  is  currently

unreliable.

-  FFG_TP = OBDII system FMEM flag for TP - 1 -> TP is currently unreliable.

-  IAC_ERR1 = Base strategy IAC error flag used  for  overspeed  malfunction

detection.

-  IAC_ERR2 = Base strategy IAC error flag used for  underspeed  malfunction

detection.

-  IAC_ERR3 = Base strategy IAC  error  flag  used  for  ISCKAM  malfunction

detection.

-  ISCFLG = Idle air mode indicator flag; -1 -> Dashpot Mode, 0  ->  Dashpot

Preposition Mode, 1 -> Closed Loop RPM Control Mode, 2 -> Closed Loop RPM

Control Lock-out Mode.

-  OBDII_RESET = Flag used to simulate the receipt of an  OBD-II  scan  tool

reset message.

-  PxxxxMALF = OBD-II malfunction flag for xxxx; 1-> a malfunction currently

exist for fault xxxx.

-  VSFMFLG = Vehicle speed sensor FMEM flag.

Calibration Constants:

-  CO_ST_ECT = Cold Start ECT value for ISC.

-  IPSIDLY = Time delay to disable IPSIBR update, sec.

-  IAC_ER_TM1 = Minimum amount of time for an overspeed fault to be  present

before setting an IAC fault is present code.

-  IAC_ER_TM2 = Minimum amount of time for an underspeed fault to be present

before setting an IAC fault is present code.

13-77

IDLE AIR CONTROL, FMEM LIMITS TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  IAC_ER_TM3 = Minimum amount of time for ISCKAM fault to be present before

setting an IAC fault is present code.

-  IAC_ISFL_OBD = ISFLAG comparator to limit operation of  IAC  OBDII  error

flags settings.

-  IAC_MON_TM = Minimum amount of time before setting an IAC  fault  monitor

code.

-  IAC_RUN_TM = Minimum time since leaving crank to do IAC fault  setting  -

compared with USPD_RUN_TMR.

-  ISC_LO_CHECK = Value for ER_STATUS when performing ISC low rpm test.

-  V_IAC_TST  =  IAC  test  type  enable  switch;  1   ->   enable   minimum

airflow/maximum duty cycle test, 2 -> enable ISCKAM clips.

OTHER

-  malfunction(ccm,Pxxxx) = Public procedure updating the MIL control status

provided by the MIL Control module.

-  ccm_codes = Set of (P1505,P1506,P1507).  The set of  OBD-II  fault  codes

that relate to the ccm monitor.

-  P1505 = Inlet Air Control system is at the adaptive clip.

-  P1506 = Inlet Air Control system duty cycle is lower than expected.

-  P1507 = Inlet Air Control system duty cycle is higher than expected.

13-78

IDLE AIR CONTROL, FMEM LIMITS TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: ISC_FMEM_COM1

BEGIN:  IAC_FAULT_SET

;Malfunction code setting logic:

CCM_TST_ENA = 0 ------------|

;CCM test disabled          |AND -|

|     |

ER_STATUS <> ISC_LO_CHECK --|     |

;not doing KOER low RPM test      |

|

ECT < CO_ST_ECT ------------------|

;Too cold for test                |

|OR --| IAC_ERR1 = 0

V_IAC_TST = 0 --------------------|     | IAC_ERR2 = 0

;IAC tests disabled               |     | IAC_ERR3 = 0

|     |

FFG_ECT = 1 ----------------------|     |

|     |

VSFMFLG = 1 ----------------------|     |

|     |

FFG_TP = 1 -----------------------|     |

|     |

OBDII_RESET = 1 ------------------|     |

|     |

USPD_RUN_TMR < IAC_RUN_TM --------|     |

;not running long enough          |     |

|     |

ISFLAG > IAC_ISFL_OBD ------------|     |

;error setting not required in          |

this load state                        |

|

| --- ELSE ---

|

| NO ACTION

IAC_ERR1 =  1 --------------------|

|AND -| malfunction(ccm,P1506)

IAC_ER_TMR1 > IAC_ER_TM1 ---------|     | IAC_ERR1 = 0

;Fault present long enough             |

| --- ELSE ---

|

| clear_malf(P1506)

IAC_ERR2 = 1  --------------------|

|AND -| malfunction(ccm,P1507)

IAC_ER_TMR2 > IAC_ER_TM2 ---------|     | IAC_ERR2 = 0

;Fault present long enough             |

| --- ELSE ---

|

| clear_malf(P1507)

13-79

IDLE AIR CONTROL, FMEM LIMITS TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

V_IAC_TST = 2 --------------------|

|

IAC_ERR3 = 1  --------------------|AND -| malfunction(ccm,P1505)

|     | IAC_ERR3 = 0

IAC_ER_TMR3 > IAC_ER_TM3 ---------|     |

| --- ELSE ---

|

| clear_malf(P1505)

;Error timers setting logic:

IAC_ERR1 = 0 ---------------------------| IAC_ER_TMR1 = 0

|

| --- ELSE ---

|

| Increment IAC_ER_TMR1

IAC_ERR2 = 0 ---------------------------| IAC_ER_TMR2 = 0

|

| --- ELSE ---

|

| Increment IAC_ER_TMR2

IAC_ERR3 = 0 ---------------------------| IAC_ER_TMR3 = 0

|

| --- ELSE ---

|

| Increment IAC_ER_TMR3

;IAC monitor logic:

IAC_MON_TMR > IAC_MON_TM ---------------| CCM_IAC_MON = 1

;IAC_MON_TMR - time in idle:

IAC_MON_TMR measures the time since enter closed loop  Idle.   The  timer  is

used to set OBDII monitor flags for completion of Idle test.

APT = -1 -------------------------|

|

RUNUPTMR > IPSIDLY ---------------|AND -| Increment IAC_MON_TMR

|     |

ISCFLG > 0 -----------------------|     |

| --- ELSE ---

|

| IAC_MON_TMR = 0

END: IAC_FAULT_SET

13-80

IDLE SPEED CONTROL, ISC DUTY CYCLE CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

13.13  ISC DUTY CYCLE CALCULATION (ISCDTY_CALC) (CDAF0)

OVERVIEW

The ISC duty cycle is calculated in ISCDTY_CALC.  The mass air  flow  through

the  ISC actuator (DEBYMA) is calculated as the desired mass air flow at idle

(DESMAF) less  the  flow  through  the  throttle  plate  etc,  corrected  for

altitude.

The desired duty cycle is calculated as follows:

Once the desired mass flow value is finalized, the appropriate duty cycle  is

calculated and output.  The final DESMAF value is calculated in IPSIBR_CALC.

The calibrated  leakage  term  (ITHBMA)  and  the  calculated  purge  airflow

(PG_AIR)  are subtracted from DESMAF, to obtain the actual flow required from

the ISC actuator (DEBYMA).  DEBYMA and PERLOAD_ISC are the inputs to the  ISC

duty  cycle  transfer function (FN8000).  Output from FN8000 is the specified

ISC duty cycle.

The final value of ISCDTY includes an offset and  a  multiplier  (IDCOFS  and

IDCMUL) used primarily as calibration tools.

DEFINITIONS

INPUTS

Registers:

-  BP = Barometric pressure, " Hg.

-  DEBYMA = Desired mass air flow through the ISC valve, ppm.  This quantity

is  calculated  as  the total desired idle air flow (DESMAF) less the air

flow through the throttle plate, and  is  corrected  for  altitide  using

FN890(BP).

-  DESMAF = Total desired idle air flow, ppm.   Calculated  as  the  sum  of

predicted  air flow (DESMAF_PRE), dashpot air flow (DASPOT), integral air

flow (IPSIBR) and KAM correction (ISCKAMn).

-  ISCDTY = Idle speed control valve duty cycle,  fraction  of  fully  open.

Calculated  as  a  transfer  function  (FN8000)  of the desired mass flow

(DEBYMA) through the ISC valve, and the load (PERLOAD_ISC).

-  PERLOAD_ISC = Used by ISC logic as the Y input to FN8000.   0-1.0  scale,

unitless.

-  PG_AIR = Calculated airflow through purge sytem.

13-81

IDLE SPEED CONTROL, ISC DUTY CYCLE CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  IDCMUL = ISCDTY multiplier, no units.

-  IDCOFS = ISCDTY adder, fraction of fully open.

-  ITHBMA = Throttle body idle mass air flow with  throttle  plate  at  idle

screw  stop and 0% ISC duty cycle.  This is any airflow which does not go

through the bypass air solenoid, i.e.  throttle plate, PCV system, intake

leakage, etc.

-  DEBYCP = Minimum allowed airflow through the ISC  actuator.   This  is  a

clip on DEBYMA.

-  FN060(PERLOAD_ISC) = Row normalizing function for FN8000.

-  FN061(DEBYMA) = Column normalizing function for FN8000.

-  FN890(BP) = ISC duty cycle altitude compensation subtractor.  Required to

offset the effect with altitude of a varying pressure drop across the ISC

actuator, which affects its air flow.

-  FN8000 = Characterization table for the ISC actuator;

X = FN061(DEBYMA)

Y = FN060(PERLOAD_ISC)

OUTPUTS

Registers:

-  DEBYMA = See above.

-  ISCDTY = See above.

PROCESS

STRATEGY MODULE: ISC_ISCDTY_COM2

unconditionally ---| iscdty = IDCMUL * FN8000(DEBYMA,PERLOAD_ISC) + IDCOFS

| Do: substitute(0,iscdty)

| ISCDTY = iscdty

|  (output state control override)

where

DEBYMA = [DESMAF - FN890(BP) - (ITHBMA + PG_AIR)] * 29.92 / BP

-  DEBYMA is clipped to DEBYCP as a minimum.

-  ISCDTY is clipped to 1.0 as a maximum.

13-82

IDLE SPEED CONTROL, IAC SCCD OUTPUT  - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

13.14  IAC (IDLE SPEED CONTROL) SCCD OUTPUT (CDAL0)

OVERVIEW

This routine determines the idle speed  output  for  the  Switching  Constant

Current  Driver  (SCCD) hardware.  The Strategy I/O map will dictate the SCCD

channel used.  The ISCDTY is converted from duty cycle to a value of current.

Sccf is the current conversion function.  This module also includes the logic

to translate the SCCD driver fault bit information to OBDII code setting.

DEFINITIONS

Registers:

-  IAC_ER_TMR4 = Timer for P1504 fault code setting.

-  ISCDTY = Idle speed control duty cycle.

-  ISC_CURRENT = SCCD percent current register.

Bit Flags:

-  CCM_EO_ENA = Engine off on demand test for CCM enabled.

-  CCM_ER_ENA = Engine running on demand test for CCM enabled.

-  CCM_TST_ENA = OBD-II Comprehensive Component Test  enable  flag.   (1  =>

test enabled).

-  IAC_NO_CUR = 1 -> IAC (ISC-BPA) output driver fault  -  open  circuit  or

short to ground.

-  IAC_OVER_CUR = 1 -> IAC (ISC-BPA) output driver  fault  -  over  current.

Shorted load or shorted to VPWR when driver is on.

-  IAC_TM4_FLG = Control flag for IAC_ER_TMR4 timer - IAC driver fault.

13-83

IDLE SPEED CONTROL, IAC SCCD OUTPUT  - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  IAC_ER_TM4 = Minimum time for IAC (ISC output) driver fault to be present

before setting the fault code.

-  IAC_OT_REQ = 1-> Do IAC output driver test.

-  ISC_PRD_DT = DUCE Normal ISC period in duce_ticks.

-  IAC_TST_DC = Minimum dutycycle for  reliable  driver  over  current  test

(P1504).

OTHER

P1504 = Idle Air Control Circuit Malfunction

13-84

IDLE SPEED CONTROL, IAC SCCD OUTPUT  - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: ISC_DTY_CONV_COM2

Begin: iac_output

Execute this process each background loop.

always -----------------| ISC_CURRENT := sccf(ISCDTY)

|  ;sccf is the SCCD Current Conversion Function

|  ;as defined by the SCCD specification and is

|  ;defined only in software implementation

SCCD IAC DRIVER DIAGNOSTICS

The SCCD/DARC I/O chips provide idle speed control valve (IAC)  driver  fault

status  to  the  strategy.   The states of over current and under current are

diagnosed.  PIDs are separately defined but both faults  will  set  the  same

OBDII  fault  code  (P1504).   It  is  not possible to detect an over current

condition when ISCDTY is zero.   The  associated  error  timer  is  therefore

frozen  when  the  ISC  dutycycle is below a calibrateable level and the open

load/ground short fault bit (IAC_NO_CUR) is not set.

IAC_OT_REQ = 1 ---------------|

;test required               |

|AND -| DO: iac_osm_test

CCM_TST_ENA = 1 --------|     |     |

|     |     |

CCM_ER_ENA = 1 ---------|OR --|     |

|           |

CCM_EO_ENA = 1 ---------|           |

| --- ELSE ---

|

| clear_malf (P1504)

End: iac_output

13-85

IDLE SPEED CONTROL, IAC SCCD OUTPUT  - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Begin: iac_osm_test

Execute this process only when called (from iac_output).

ISCDTY <= IAC_TST_DC ---------|

;dutycycle below minimum     |AND -| IAC_TM4_FLG = 0

|     |  ;freeze IAC_ER_TMR4

IAC_NO_CUR = 0 ---------------|     |   output too low for reliable

;no open circuit/ground            |   detection of overcurrent

short fault                       |   condition.

|

| --- ELSE ---

|

IAC_NO_CUR = 1 ---------------|     | IAC_TM4_FLG = 1

|OR --|  ;there is a fault

IAC_OVER_CUR = 1--------------|     |   allow IAC_ER_TMR4 timer

|   to upcount.

|

| --- ELSE ---

|

| IAC_TM4_FLG = 0

| IAC_ER_TMR4 = 0

|  ;no fault, clear timer

Fault Code Setting:

IAC_ER_TMR4 > IAC_ER_TM4 -----------| malfunction(ccm,P1504)

|

| --- ELSE ---

|

| clear_malf(P1504)

End: iac_osm_test

13-86

IDLE SPEED CONTROL, TIMERS CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

13.15  TIMER SUMMARY (CDAN0)

OVERVIEW

The timers described here are for use within the idle  speed  control  logic.

For general purpose timers see the Utility area.

.---------------------------------------------------.

| TIMER    | DESCRIPTION                 | INTERVAL |

|==========+=============================+==========|

| ISCTMR   | RPM sample/KAM update timer | 0.125 sec|

|----------+-----------------------------+----------|

| RUNUPTMR | Time since runup            | .125 sec |

`---------------------------------------------------'

DEFINITIONS

Registers:

-  APT = Throttle mode flag: -1 = closed, 0 = part, 1 = wide open throttle

mode.

-  DASPOT = Desired mass air flow for dashpot (PPM).

-  ISCFLG = Current mode of the Idle Speed control system.

-  ISCTMR = Time at closed throttle (APT = -1) (0.125 seconds).

-  N = Engine RPM

-  NLAST = Previous value of Engine RPM

-  RPMERR_A = Filtered RPM error (desired rpm - N).

-  RUNUPTMR = time since initial runup (seconds).

-  SETTMR = settling time for entering closed loop RPM control (seconds).

Bit Flags:

-  RUNUP_FLG = status of runup: 0 = not complete, 1 = complete.

Calibration Constants:

-  DASCTL = lower dashpot limit to allow rpm control (PPM)

-  ISCTM = Pacing to evaluate rate of change of engine speed.

-  NDIF = Time dependent RPM limit to differentiate decel from idle

(seconds).

13-87

IDLE SPEED CONTROL, TIMERS CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  RPMDED = RPM deadband for closed loop idle speed control.

13-88

IDLE SPEED CONTROL, TIMERS CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: ISC_TIMERS_COM1

;RUNUPTMR - TIME SINCE RUNUP (.125 sec)

RUNUPTMR measures the time since the runup rpm was first exceeded.  The timer

is  used  to disable IPSIBR update for a time period following initial RUNUP.

For setting of RUNUP_FLG see idle speed control chapter.

RUNUP_FLG = 0 -------------------------| RUNUPTMR := 0

| ; Clear timer

|

| --- ELSE ---

|

| ; Increment free

| ; running RUNUPTMR

SETTMR - RPM CONTROL ENTRY DELAY TIMER (.125 sec)

SETTMR is used to delay entry into RPM control for a time

which corresponds to the manifold stabilization time. SETTMR is

cleared in Idle Speed Control MODE_SELECT.

In mode select ------------------|

|

APT >= 0 ------------------|     |

;not closed throttle      |     |AND -| SETTMR := 0

|     |     | ; Clear timer

APT = -1 ------------|     |     |     |

|AND -|OR --|     |

DASPOT > DASCTL -----|     |           |

;daspot too large         |           |

|           |

RUNUP_FLG = 0 -------------|           |

; RUNUP not complete                  |

| --- ELSE ---

|

| ;Increment free

| ; running SETTMR

13-89

IDLE SPEED CONTROL, TIMERS CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;ISCTMR - RPM SAMPLE / KAM UPDATE DELAY TIMER (0.125 Sec.)

This timer is used for two separate purposes in the normal idle speed control

strategy.  It is always cleared in dashpot-preposition mode.

- In rpm control mode, it is used to prevent ISCKAM updates while

RPMERR_A is outside the deadband allowed for ISCKAM learning.

- In dashpot mode, it is used to pace the rate of change of RPM checks

in order to determine whether or not to go to lockout mode.

; The following piece of logic is actually implemented in three distinct

; groupings. This is reflected in the comments below each of the groups,

; below;

APT >= 0 ---------------------------|

; Not at closed throttle.           |

; Above condition is actioned       |

; within the Mode Select logic      |

|

ISCFLG = 1 -------------------|     |

; Close loop RPM control     |AND -|

|     |

|RPMERR_A| > RPMDED ----------|     |OR --| ISCTMR := 0

;outside deadband                  |     | ;Clear Timer

; Above AND gate is actioned        |     |

; within the ISCKAM update          |     |

; logic                             |     |

|     |

ISCFLG < 1 -------------------|     |     |

;not rpm control or lockout  |     |     |

|     |     |

ISCTMR >= ISCTM --------------|AND -|     |

;time to check RPM           |           |

|           |

|N - NLAST| > NDIF -----------|           |

;RPM changing too fast                   |

| --- ELSE ---

|

| ; Increment free

| ; running ISCTMR

; Above AND gate is actioned within

; the Lockout logic

13-90

CHAPTER 14

A/C CLUTCH STRATEGY

14-1

A/C CLUTCH STRATEGY

14.1  FEATURE:  AIR CONDITIONING CONTROL STRATEGY (ACC) - V2.3_ACC (CDAN0)

14.1.1  A/C CLUTCH INPUT (CDAN0)

OVERVIEW

This strategy module is for use in applications that have the A/C input going

high  when  A/C  is  selected,  and  do  NOT have the Heated Front Windshield

multiplexed on the same input as the A/C.

OBDII code P1464 is set when  the  A/C  is  requested  during  KOER  or  KOEO

selftest.   Seperate  logic  prevents  the  A/C  from being engaged during on

demand testing.

DEFINITIONS

Registers:

-  ACSW = A/C switch input status; 1 -> A/C on.

-  P1464_TMR = Time A/C requested during selftest mode.

Bit Flags:

-  CCM_EO_ENA = Engine off on demand test for CCM enabled flag.

-  CCM_ER_ENA = Engine running on demand test for CCM enabled flag.

Calibration Constants:

-  P1464_TM_LMT = Time  required  to  set  code  for  A/C  requested  during

selftest.

OTHER

-  P1464 = A/C demand out of Self test range.

14-2

A/C CLUTCH STRATEGY, A/C CLUTCH INPUT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: ACC_INPUT_COM3

accs = 1 -------------------------------| ACSW := 1

|  ;A/C input "on"

|

| --- ELSE ---

|

| ACSW := 0

|  ;A/C input "off"

ACSW = 0 -------------------------|

;A/C input off                   |

|OR --| P1464_TMR := 0

CCM_EO_ENA = 0 -------------|     |     | Do: Clear_malf(P1464)

|AND -|     |

CCM_ER_ENA = 0 -------------|           |  ;Either switch is "off"

;Testing is disabled                   |  ;or Engine off and Engine

|  ;running sleftest disabled

|

| --- ELSE ---

|

P1464_TMR > P1464_TM_LMT ---------------| Do: Store_code(P1464)

|

| --- ELSE ---

|

| NO ACTION

|  ;Allow P1464_TMR to increment

14-3

A/C CLUTCH STRATEGY, A/C CLUTCH CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

14.1.2  A/C CLUTCH CONTROL (CDAN0)

FEATURE MEMBERSHIP :  ACC

EXECUTION CONTEXT :  Background

FUNCTIONAL REQUIREMENTS :

This module controls the A/C request flag.  This flag is passed to the module

ACC_OUTPUT for control of the A/C air and the clutch.

FUNCTIONAL DESCRIPTION :

The A/C clutch control strategy disengages the air conditioning  clutch  when

compressor  operation is undesirable.  Control is achieved through the use of

an EEC controlled, normally closed relay or solid state  switch  in  the  A/C

compressor clutch circuit.

The strategy requests that the A/C clutch  be  disengaged  when  any  of  the

following conditions are met.

1.  Time since start is less than a calibration value (5 second maximum).

2.  Throttle position is greater than a calibration value (at  least  90%  of

the  "low  limit"  TP_REL  at  full  throttle  travel) and the time since

disabling due to throttle position is less than a calibration value.

3.  Engine coolant temperature is  greater  than  an  "overheat"  calibration

value.   (248 deg F minimum, maximum 10 deg F hysterisis) or if CFMFLG is

set.  This means that the ECT reading is not valid.

4.  Engine RPM is less than a near stall value (at least 100 RPM  under  base

idle; re-enabled no less than 50 RPM under base idle).

5.  Engine RPM is above the RPM that could cause damage to the A/C compressor

(value depends on compressor type and pulley ratio).

6.  A DIS Coil failure has occurred.

7.  On demand self test mode is active.

8.  The power steering flag has been triggered at the same time as the A/C is

requested.  This will delay the A/C clutch for a calibratable time.

9.  High A/C head pressure.

14-4

A/C CLUTCH STRATEGY, A/C CLUTCH CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The A/C clutch will be requested when all of  the  following  conditions  are

met:

1.  None of the above disabling conditions are true.

2.  The A/C panel switch is closed indicating driver demand for A/C.

3.  The cycling pressure switch in the A/C system is closed.

4.  The A/C clutch has been disengaged for a minimum time.  Used  to  prevent

rapid cycling (2.0 - 2.5 seconds).

***********************  NOTE  ************************

*                                                     *

*   CLIMATE CONTROL DIVISION MUST BE APPRISED OF ANY  *

*     DEVIATIONS FROM THE CALIBRATION GUIDELINES.     *

*                                                     *

***********************  NOTE  ************************

14-5

A/C CLUTCH STRATEGY, A/C CLUTCH CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  ACIOTMR = A/C OFF delay timer.

-  ACTPOFF_TMR = A/C wide open throttle cutoff timer.

-  ACTPTMR2 = Not at high throttle angle timer for A/C clutch control,  sec.

Used  to delay A/C re-enable after TP decreases below the high TP disable

condition.

-  ATMR1 = Time since Start up, sec.

-  ECT = Engine Coolant Temperature, deg.  F.

-  N = Engine speed, rpm.

-  PSTMR = Power steering transient airflow timer.

-  TP_REL = Relative Throttle Position (TP - RATCH), counts.

Bit Flags:

-  ACCFLG = A/C Engaged flag; 1 -> A/C engaged, 0 -> A/C disengaged.

-  ACC_KAM_FLG = KAM flag to signify that AC has been engaged.

-  ACSW = Flag that indicates the state of the AC input.

-  ACECT_FLG = A/C ECT flip flop.

-  ACNLO_FLG = A/C RPM low flag.

-  ACNHI_FLG = A/C RPM high flag.

-  ACPRES_FLG = High A/C pressure flag.

-  ACRQST = A/C requested flag; 1 -> request.

-  ACTP_FLG = A/C throttle position flag.

-  ACTPOFF_RQST = A/C wide open throttle cutout requested.

-  DEMAND_MODE = Flag set when an On-Demand self test mode is active.

-  DIS_FMEM = Flag indicating that a primary side failure has occurred on  a

DIS system and that an alternate strategy has been requested.

-  POWSFG = Power steering flag.

-  KAM_ERROR = KAM error flag, 1 -> KAM data invalid.

14-6

A/C CLUTCH STRATEGY, A/C CLUTCH CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  AC_NEW_RPM = Max RPM to enable a new AC compressor to be engaged.

-  ACECT_CL = Minimum ECT to disable A/C clutch, deg.   F.   Guideline;  not

lower than ACECT_SH minus 10 deg.  F.

-  ACECT_SH = Maximum ECT to enable A/C clutch,  deg.   F.   Guideline;  not

lower than 248 deg.  F.

-  AC_MN_DT = Minimum disable time for A/C clutch.

-  ACMNDT_TP = Time delay to keep A/C disabled after leaving  high  throttle

angle, sec.

-  ACNHI_CL = RPM below which WAC based RPM cutout is cleared.

-  ACNHI_SH = Maximum RPM to enable A/C clutch, rpm.  Guideline; depends  on

compressor type and pulley ratio.

-  ACNLO_CH = Maximum RPM to disable A/C clutch, rpm.  Guideline; not higher

than "base idle rpm - 50".

-  ACNLO_SL = Minimum RPM to enable A/C clutch, rpm.  Guideline; not  higher

than "base idle rpm - 100".

-  ACPSTM = A/C delay time for PS stabilisation.

-  ACSTRD = Time  delay  after  start  up  before  enabling  the  A/C,  sec.

Guideline:  not more than 5 secs.

-  ACTP_SH = Maximum TP_REL to enable A/C  clutch,  counts.   Guideline;  at

least 90% of minimum TP_REL at full throttle travel.

-  WOTCOT = Maximum time to disable A/C at high  TP,  sec.   Guideline;  not

more than 25 seconds.

14-7

A/C CLUTCH STRATEGY, A/C CLUTCH CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: ACC_CONTROL_COM2

Temperature Limiting:

ECT > ACECT_SH -------------|

;overheated                |OR --|S Q -| ACECT_FLG

|     |

CFMFLG = 1 -----------------|     |

;ECT Sensor failed               |

|

ECT < ACECT_CL -------------|     |

|AND -|C

CFMFLG = 0 -----------------|

RPM limiting:

N < ACNLO_SL ---------------------|S Q -| ACNLO_FLG

;near stall                      |

|

N > ACNLO_CH ---------------------|C

N > ACNHI_SH ---------------------|S Q -| ACNHI_FLG

;high rpm                        |

|

N < ACNHI_CL ---------------------|C

Wide Open Throttle cutout logic:

TP_REL > ACTP_SH -----------------|S Q -| ACTPOFF_RQST

;high throttle angle             |     |  ;Cutout requested

|

ACTPTMR2 > ACMNDT_TP -------------|C

;at low TP for a time

ACTPOFF_TMR < WOTCOT -------------|

;WOT cutout timer less than      |

;requested                       |AND -| ACTP_FLG := 1

|     |  ;disable A/C

ACTPOFF_RQST = 1 -----------------|     |

;WOT cutout requested                  |

| --- ELSE ---

|

| ACTP_FLG := 0

14-8

A/C CLUTCH STRATEGY, A/C CLUTCH CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

KAM flag control:

KAM_ERROR = 1 --------------------------| ACC_KAM_FLG := 0

;KAM data invalid, initialize          |  ; Initialize KAM flag

|

| --- ELSE ---

ACCFLG = 1 -----------------------|     |

;A/C on                          |AND -| ACC_KAM_FLG := 1

|     |  ;Store AC on in KAM

ACRQST = 1 -----------------------|     |

;A/C requested                         |

| --- ELSE ---

|

| NO ACTION

14-9

A/C CLUTCH STRATEGY, A/C CLUTCH CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

ACC_KAM_FLG = 0 ------------|

;AC has not been on yet    |AND -|

|     |

N > AC_NEW_RPM -------------|     |

;RPM not low                     |

|

DIS_FMEM = 1 ---------------------|

;DIS failure                     |

|

DEMAND_MODE = 1 ------------------|

;On-Demand self test             |

|

ACSW = 0 -------------------------|

;A/C switch "off"                |

|

ACIOTMR < AC_MN_DT ---------|     |

;minimum off time          |AND -|

|     |

ACRQST = 0 -----------------|     |

;A/C not requested               |OR --| ACRQST := 0

|     |  ;A/C not requested

ATMR1 < ACSTRD -------------------|     |

;start up delay                  |     |

|     |

ACPRES_FLG = 1 -------------------|     |

|     |

ACECT_FLG = 1 --------------------|     |

|     |

ACNLO_FLG = 1 --------------------|     |

|     |

ACNHI_FLG = 1 --------------------|     |

|     |

ACTP_FLG = 1 ---------------------|     |

|     |

ACRQST = 0 -----------------|     |     |

;A/C not requested         |     |     |

|     |     |

POWSFG = 1 -----------------|AND -|     |

;Power Steering ON         |           |

|           |

PSTMR < ACPSTM -------------|           |

;PS On for short time                  |

| --- ELSE ---

|

| ACRQST := 1

|  ;A/C requested

14-10

A/C CLUTCH STRATEGY, A/C CLUTCH CONTROL TIMERS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

14.1.3  A/C CLUTCH CONTROL TIMERS (CDAN0)

OVERVIEW

This module comprises the timers for the A/C Control strategy.

DEFINITIONS

Registers:

-  ACIOTMR = A/C OFF delay timer, for ISC.

-  ACITMR = A/C clutch idle turn on delay timer.

-  ACTPTMR2 = Not at high throttle angle timer for A/C clutch control,  sec.

Used  to delay A/C re-enable after TP decreased below the high TP disable

condition.

-  ACTPOFF_TMR = A/C wide open throttle cutout timer.

-  TP_REL = Relative Throttle Position (TP - RATCH), counts.

Bit Flags:

-  ACRQST = A/C requested flag; 1 -> request.

-  ACTPOFF_RQST = A/C Wide open throttle cutout requested.

-  AC_EN_TRANS = Flag used to delay A/C activation process; 0 -> delay.

Calibration Constants:

-  ACTP_CL = Maximum TP_REL to  increment  ACTPTMR2,  counts.   Provides  TP

hystersis  for  high  throttle  angle  A/C  disable.  Should be less than

ACTP_SH.

14-11

A/C CLUTCH STRATEGY, A/C CLUTCH CONTROL TIMERS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: ACC_TIMER_COM1

A/C CLUTCH CONTROL TIMERS

ACTPOFF_TMR - WIDE OPEN THROTTLE CUTOUT TIMER.  ( Resolution - 0.125 secs )

This timer is used to disable the A/C clutch at high throttle  angles  for  a

calibratable time.

ACTPOFF_RQST = 1 -----------------------| Increment ACTPOFF_TMR

;WOT cutout requested                  |

| --- ELSE ---

|

| ACTPOFF_TMR := 0

ACTPTMR2 - NOT AT HIGH THROTTLE ANGLE TIMER ( Resolution - 0.125 secs )

This timer is used in conjunction with the ACTPOFF_TMR  timer  to  provide  a

means  of  keeping  the A/C clutch disabled when leaving high throttle angles

momentarily to shift gears.  Note:  If WOTCOT is  calibrated  to  zero,  this

timer has no effect on the A/C clutch.

TP_REL < ACTP_CL -----------------------| Increment ACTPTMR2

;throttle is not close to wide open    |

| --- ELSE ---

|

| ACTPTMR2 := 0

ACITMR - A/C CLUTCH ENABLE DELAY TIMER ( Resolution - 0.001 secs )

This timer is used to sequence events that occur when A/C is impending.

AC_EN_TRANS = 1 ------------------|

|AND -| Increment ACITMR

ACRQST = 1 -----------------------|     |

;A/C requested                         | --- ELSE ---

|

| ACITMR := 0

14-12

A/C CLUTCH STRATEGY, A/C CLUTCH CONTROL TIMERS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

ACIOTMR - A/C CLUTCH DISABLE DELAY TIMER.  ( Resolution - 0.001 secs )

This timer is used to sequence  events  that  occur  when  A/C  turn  off  is

impending.

ACRQST = 0 -----------------------------| Increment ACIOTMR

;A/C not requested                     |

| --- ELSE ---

|

| ACIOTMR := 0

14-13

A/C CLUTCH STRATEGY, A/C CLUTCH OUTPUT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

14.1.4  A/C CLUTCH OUTPUT (CDAN0)

OVERVIEW

This strategy module controls the A/C output for the instances where the  CPU

output  is  low to enable A/C, and provides a delay for ISC compensation when

turning A/C on only.  The strategy incorporates two separate turn-on  delays.

The  first,  IDLCOT,  is  for use at start-up only, and enables a significant

time delay to be calibrated to enable the cooling fans to  remove  heat  from

the  A/C  Condensor.  The second, IDLCOT2, is for use when the A/C is cycling

under normal operating conditions, and does not require a delay the length of

IDLCOT.  If the strategy is used such that the compressor is turned on before

the fan then IDLCOT2 should be calibrated equal to IDLCOT.

Note that there are two corresponding values for ISC compensation,  ACC_DELAY

and ACC_DELAY2.

N.B If using the Transmission convertor clutch slip feature ensure that  both

ACC_DELAY  and  ACC_DELAY2 are greater than zero to prevent premature A/C air

addition prior to clutch engagement.

DEFINITIONS

INPUTS

Registers:

-  ACIOTMR = A/C off delay timer, for ISC.

-  ACITMR = A/C clutch enable delay timer, sec.  Used to  delay  A/C  clutch

engagement  to  allow  the  Idle Speed Control strategy to anticipate the

compressor load application.

Bit Flags:

-  ACCFLG = Flag that controls the state of A/C CPU output; 1 -> high.

-  ACCFLG_LAST = Last value of ACCFLG.

-  ACRQST = A/C requested; 1 -> request.

-  AC_CYCLE_FLG = Flag to indicate that the AC is cycling.

-  ACC_STATUS = ACC Smart Output Driver Fault Status.

-  OSM_EO_OFF = Off state  requested  for  OSM  outputs  during  engine  off

on-demand test.

-  OSM_EO_ON =  ON  state  requested  for  OSM  outputs  during  engine  off

on-demand test.

14-14

A/C CLUTCH STRATEGY, A/C CLUTCH OUTPUT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  ACC_DELAY = Time delay before adding air to DESMAF_PRE.

-  ACC_DELAY2 = Second delay for adding A/C air, sec.

-  AC_MN_FT = Minimum time  to  run  low  speed  EDF  after  A/C  clutch  is

disengaged, sec.

-  IDLCOT = Closed Throttle A/C enable delay time to allow ISC compensation.

-  IDLCOT2 = Second delay time for A/C operation, sec.

OUTPUTS

Bit Flags:

-  ACCFLG = See above.

-  ACCFLG_LAST = See above.

-  ACC_STATUS = See above.

-  AC_CYCLE_FLG = See above.

-  AC_PPM_RQST = Flag to request that the ISC adds air to compensate for  AC

loading.

-  ACR = A/C output.

14-15

A/C CLUTCH STRATEGY, A/C CLUTCH OUTPUT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: ACC_OUTPUT_COM1

ACRQST = 0 ------------------------| AC_PPM_RQST := 0

;A/C not requested                |  ;add no AC air

|

| --- ELSE ---

AC_CYCLE_FLG = 0 ------|           |

;A/C not cycling      |AND -|     |

|     |     |

ACITMR >= ACC_DELAY ---|     |     |

;delay 1 expired            |OR --| AC_PPM_RQST := 1

|     |  ;add AC air

AC_CYCLE_FLG = 1 ------|     |     |

;A/C cycling          |AND -|     |

|           |

ACITMR >= ACC_DELAY2 --|           |

;delay 2 expired                  |

| --- ELSE ---

|

| NO ACTION

|  ;Make no change to AC_PPM_RQST

OSM_EO_ON = 0 ---------|

|AND -|

ACRQST = 0 ------------|     |

;A/C not requested          |OR --| ACCFLG := 0

|     | ACR := 1

OSM_EO_OFF = 1 --------------|     |  ;A/C "off"

|

|

| --- ELSE ---

AC_CYCLE_FLG = 0 ------|           |

;A/C not cycling      |AND -|     |

|     |     |

ACITMR > IDLCOT -------|     |     |

;delay 1 expired            |     |

|     |

AC_CYCLE_FLG = 1 ------|     |OR --| ACCFLG := 1

;A/C cycling          |AND -|     | ACR := 0

|     |     |  ;A/C "on"

ACITMR > IDLCOT2 ------|     |     |

;delay 2 expired            |     |

|     |

OSM_EO_ON = 1 ---------------|     |

| --- ELSE ---

|

|  ;Make no change to ACR and ACCFLG

14-16

A/C CLUTCH STRATEGY, A/C CLUTCH OUTPUT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

ACCFLG_LAST = 1 ------|

|AND --|S Q -| AC_CYCLE_FLG

ACCFLG = 0 -----------|      |

;AC has been ON and now OFF |

|

ACIOTMR > AC_MN_FT ----------|C

;AC been off for more than

;fan run-on time

unconditionally -------------------| ACCFLG_LAST := ACCFLG

|  ;store last state

|  ;of ACCFLG

|

| ACC_STATUS := output(ACR)

14-17

A/C CLUTCH STRATEGY, ACC OUTPUT STATE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

14.1.5  ACC OUTPUT STATE MONITOR (CDAN0)

OVERVIEW

This module documents the ACC Output State Monitor.   For  the  Smart  Output

Drivers the status bit should always read Zero for no fault present.

DEFINITIONS

Registers:

-  P1460_TMR = Time ACC OSM failure detected for.

-  VBAT = Battery voltage.

Bit Flags:

-  ACC_KAM_FLG = KAM flag to signify that A/C  has  been  engaged  at  least

once.

-  ACC_STATUS = ACC Smart Output Driver Fault status.

-  CCM_EO_ENA = Engine off on demand test for CCM enabled.

-  CCM_ER_ENA = Engine running on demand test for CCM enabled.

-  CCM_TST_ENA = OBDII Comprehensive Component Test enable flag; 1 -> test.

Calibration Constants:

-  P1460_TM_LMT = Time required to set code for ACC OSM failure.

-  VBAT_CCM_MIN = Minimum voltage to perform OSM tests.

OTHERS

-  P1460 = Fault code for ACC OSM failures.

14-18

A/C CLUTCH STRATEGY, ACC OUTPUT STATE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: ACC_OSM_COM1

ACC_KAM_FLG = 0 ------------------|

;A/C not run yet                 |

|

ACC_STATUS = 0 -------------------|

;OSM status correct              |     |

|OR --| P1460_TMR := 0

VBAT < VBAT_CCM_MIN --------------|     | Do: Clear_malf (P1460)

|     |  ;Either OSM testing is

CCM_TST_ENA = 0 ------------|     |     |  ;disabled, the A/C system has

|     |     |  ;not been run at least once,

CCM_ER_ENA = 0 -------------|AND -|     |  ;or there is no error.

|           |

CCM_EO_ENA = 0 -------------|           |

| --- ELSE ---

|

P1460_TMR > P1460_TM_LMT ---------------| Do: Store_code (P1460)

;Fault has been present long enough    |

| --- ELSE ---

|

| NO ACTION

| ;Allow free running timer

| ;P1460_TMR to increment

14-19

A/C CLUTCH STRATEGY, A/C CLUTCH DIAGNOSTICS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

14.2  A/C CLUTCH DIAGNOSTICS (CDAN2)

OVERVIEW

This strategy module monitors the A/C Cycling  period  and  sets  a  selftest

code, P1469, if the period is short enough that the transmission is no longer

able to slip to pre-empt A/C Clutch engagement.  Note that for this  strategy

to  work,  ACIOTMR must be actuated, to prevent it from counting up and being

reset once per background loop.

DEFINITIONS

Registers:

-  ACIOTMR = A/C OFF delay timer, for ISC.

-  ACIOTMR_LAST = Last pass valuve of ACIOTMR.

-  ACPERIOD = A/C Clutch cycling period.

-  ACPERIOD_TMR = A/C Cycling period timer.

|      -  P1469_CNT = Number of times a short A/C cycle has been detected.

|     Bit Flags:

|      -  CCM_TST_ENA = OBD-II Comprehensive Component Test  enable  flag.   (1  =>

|         test enabled).

|      -  OBDII_RESET = Flag used to simulate the receipt of an  OBD-II  scan  tool

|         reset message.

Calibration Constants:

-  CCSLP_MN_PRD = Transmission CC minimum AC Period to enable slip feature.

|      -  P1469_LMT = Number of short A/C cycles  which  must  be  detected  before

|         setting the malfunction code.

OTHER

-  P1469 = Fault code for A/C cycling period too short.

14-20

A/C CLUTCH STRATEGY, A/C CLUTCH DIAGNOSTICS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: ACC_OBD_COM1

|     CCM_TST_ENA = 0 --------------|

|     ;continuous testing disabled  |    | Clear_malf (P1469)

|                                   |OR -| P1469_CNT := 0

|     OBDII_RESET = 1 --------------|    | ;reset counter and

|     ;reset required                    | ;clear code, either

|                                        | ;testing disabled or

|                                        | ;reset required.

|                                        |

|                                        | --- ELSE ---

|                                        |

|                                        | DO: Determine_ac_period

|                                        | ;determine the period and

|                                        | ;set code if consistantly

|                                        | ;too short

|                                        |

|                                        | ACIOTMR_LAST := ACIOTMR

|                                        | ;store last value of timer

|     BEGIN : Determine_ac_period

|     ACIOTMR < ACIOTMR_LAST ------------| ACPERIOD := ACPERIOD_TMR

|      ;ACIOTMR has been reset           | ACPERIOD_TMR := 0

|      ;end of period                    | ;store A/C period and

|                                        | ;reset period timer to zero

|                                        | ;to count new period

|                                        |

|                                        | DO: Test_ac_period

|                                        | DO: set_clear_code

|                                        |

|                                        | --- ELSE ---

|                                        |

|                                        | ;increment ACPERIOD_TMR

|     END : Determine_ac_period

|     BEGIN : Test_ac_period

|     ACPERIOD < CCSLP_MN_PRD -----------| P1469_CNT := P1469_CNT + 1

|      ;Clutch cycling period less       | clip(P1469_CNT,0,255)

|      ;than minimum - error             | ;cycling period too short

|                                        | ;increment the error

|                                        | ;counter

|                                        |

| --- ELSE ---

|

|                                        | P1469_CNT := P1469_CNT - 1

|                                        | clip(P1469_CNT,0,255)

|                                        | ;no error present

|                                        | ;decrement the counter

|     END : Test_ac_period

14-21

A/C CLUTCH STRATEGY, A/C CLUTCH DIAGNOSTICS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|     BEGIN : set_clear_code

|     P1469_CNT > P1469_LMT -------------| Store_code (P1469)

|                                        | ;too many short cycles

|                                        | ;set the code

|                                        |

|                                        | --- ELSE ---

|                                        |

|                                        | Clear_malf (P1469)

|                                        | ;cycling ok, clear

|                                        | ;the code

|     END : set_clear_code

14-22

CHAPTER 15

CANISTER PURGE STRATEGY

15-1

CANISTER PURGE STRATEGY

15.1  FEATURE:    EVAPORATIVE   PURGE    COMPENSATION    CONTROL    (PCOMP)    -

V1.1D_CANP_PCOMP (CDAN0)

15.1.1  PCOMP ENABLE (CDAM0)

OVERVIEW

The strategy alternates when PCOMP is active and when adaptive is active.

This module enables the PCOMP strategy when:

-  The EEC has entered closed loop control for  the  first  time  (to  allow

initial start-up vapors to be purged); or

-  The EEC is in closed loop control and the adaptive system has adapted for

a maximum period of time; or

-  The EEC is in closed loop control and the  adaptive  system  has  brought

LAMBSE into a calibrated window of control.

After adaptive fuel has timed-out via ADAPT_TMR, if  the  filtered  value  of

either  LAMBSE1  or LAMBSE2 shows that the commanded equivalence ratio is not

under control, PCOMP will remain disabled.  In this case, PCOMP  will  remain

in the disable state until the filtered LAMBSE values (LAM_BAR1 and LAM_BAR2)

are both within the window specified by ADP_LAM_MIN and ADP_LAM_MAX, or until

the  filtered KAMRF values KAM_BAR1 and KAM_BAR2) leave the window calibrated

by ADP_KAM_MIN and ADP_KAM_MAX.

The following conditions will keep PCOMP disabled, even after  the  ADAPT_TMR

has expired (ADP_TM).  This example illustrates a Rich Engine Condition.

RICH ENGINE CONDITION

|++++++++++++++++

|                 ++

|                    ++

|                       ++ <-- LAM_BARn

| - - - - - - - - - - - - -++ - - - - - - - - - - - - - ADP_LAM_MAX

|                           ++

+++++++++++++++++-------------------------------------- 1.0

|                 ++

|                    ++

|                       ++

| - - - - - - - - - - - - -++ - - - - - - - - - - - - - ADP_KAM_MIN

|                             ++ <-- KAM_BARn

|                        ++++++++++++++++++++++++++++++

|                        +

|                        +

|+++++++++++++++++++++++++_____________________________ PCOMP_ENA

|                       ++

|                    ++  +

|                 ++     +

|              ++        +

|           ++           +

15-2

CANISTER PURGE STRATEGY, PCOMP ENABLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

| - - - -++ - - - - - - -+- - - - - - - - - - - - - - - ADP_TM

|     ++                 +

|  ++                    +

|++______________________++++++++++++++++++++++++++++++ ADAPT_TMR

15-3

CANISTER PURGE STRATEGY, PCOMP ENABLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINTIONS

Registers:

-  ADAPT_TMR = Timer that tracks how long Adaptive fuel  strategy  has  been

enabled.

-  ADPTMR = Adaptive fuel timer, seconds.

-  F6K_LST_PS = Output value of FN6000A last background pass.

-  KAMRF1/KAMRF2 = The adaptive fuel strategy F/A ratio correction factor.

-  LAM_BAR1 = Filtered equivalence ratio for HEGO11 injectors.

-  LAM_BAR2 = Filtered equivalence ratio for HEGO21 injectors.

-  PCOMP_W = Reduced range PCOMP.

-  PG_DC = Purge duty cycle, percent on.

-  TCSTRT = Temperature of Engine Coolant at Cold Startup, deg F.

-  TSLADPTMR = Time Since Last Adapt TiMeR.  Tracks time between when  Adapt

is active.

Bit Flags:

-  FADPT_READY = Flag indicating that all conditions have been met to enable

an adaptive fuel.

-  FFS_FMFLG = Flexible Fuel sensor failure mode flag.

-  FRST_ADP = Flag showing that ADAPT_TMR has incremented at least once.

-  OLFLG = Open Loop Fuel flag; 1 -> open loop, 0 -> closed loop possible.

-  PCOMP_ENA = PCOMP strategy enabled flag; 1 -> PCOMP is enabled,  adaptive

fuel disabled.

-  PCOMP_FST_PS = PCOMP first pass flag; 1 -> first pass of PCOMP  has  been

completed.

-  PGM_CVS_FM = Canister vent solenoid FMEM flag; 1 -> CVS  failed.   Danger

exists to pull excessive vacuum on the fuel tank, disable PCOMP.

-  PGM_PG_ON = Purge monitor command to turn PCOMP/PURGE on.

-  PGM_RUN_LST = State of PGM_RUNNING last pass.

-  PGM_RUNNING = Purge monitor is running flag.

Calibration Constants:

-  ADAPTM = Time in ECT desired range before enabling adaptive fuel learning

in seconds.

15-4

CANISTER PURGE STRATEGY, PCOMP ENABLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  ADP_LAM_MAX = If LAM_BAR1 and LAM_BAR2 are greater than this  value,  the

system  will  remain in adaptive fuel control after ADAPT_TMR has expired

(if KAM_BARn is not nearing the ADP_KAM_MIN limit).

-  ADP_LAM_MIN = If LAM_BAR1 and LAM_BAR2 are less than than this value, the

system  will  remain in adaptive fuel control after ADAPT_TMR has expired

(if KAM_BARn is not nearing the ADP_KAM_MAX limit).

-  ADP_KAM_MAX = Maximum  value  of  KAM_BAR1  and  KAM_BAR2  to  remain  in

adaptive fuel control after ADAPT_TMR has expired.

-  ADP_KAM_MIN = Minimum  value  of  KAM_BAR1  and  KAM_BAR2  to  remain  in

adaptive fuel control after ADAPT_TMR has expired.

-  ADP_TM = Time in adaptive learning after which  PCOMP  is  re-enabled  if

LAMBSE within window.

-  ADP_TM_MAX = Maximum amount of time that adaptive strategy is allowed  to

be active before PCOMP is forced.

-  FFS_SW = Flexible Fuel sensor hardware  present  switch;  1  ->  hardware

present.

-  PG_TSLADP_TM = Value of TSLADPTMR that forces  an  adaptive  sequence  to

begin.

-  PCOMP_ECT = TCSTRT value below  which  adaptive  fuel  will  be  executed

first; otherwise PCOMP is executed first.

-  PCOMP_MN_PPM = PCOMP PPM value below which the system will be allowed  to

adapt.

-  PG_DC_D_MIN = Purge valve duty cycle disable minimum.

-  PGM_ADPT_RST = Restart state of PCOMP; 1 -> After the  purge  monitor  is

complete,  start adaptive fuel, 0 -> After the purge monitor is complete,

start PCOMP.

-  PGM_HW_SW = Evaporative System Monitor  hardware  present  switch;  1  ->

hardware present.

15-5

CANISTER PURGE STRATEGY, PCOMP ENABLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CANP_PCOMP_ENABLE_COM7

BEGIN:  canp_pcomp_enable

PGM_RUNNING = 1 -------------------|

; purge monitor test running      |AND -| PCOMP_ENA := PGM_PG_ON

|     |  ; set PCOMP_ENA according to

PGM_HW_SW = 1 ---------------------|     |  ; register set in purge monitor

; purge monitor present                 | PGM_RUN_LST := 1

| ADAPT_TMR := 0

| TSLADPTMR := 0

|  ; clear timers, not needed

|  ; during purge monitor test.

| Exit: canp_pcomp_enable

|

| --- ELSE ---

|

| Do: pcomp_timers

| Do: pcomp_ena_init

| PGM_RUN_LST := 0

END: canp_pcomp_enable

15-6

CANISTER PURGE STRATEGY, PCOMP ENABLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: pcomp_ena_init

; Determine if the purge monitor test has just completed:

PGM_RUNNING = 0 -------------------|

; test is not running now         |

|

PGM_RUN_LST = 1 -------------------|AND -| pc_restart := 1

; test was running last pass      |     |

|     |

PGM_HW_SW = 1 ---------------------|     |

; purge monitor present                 |

| --- ELSE ---

|

| pc_restart := 0

; Initialize PCOMP_ENA for special conditions:

TCSTRT < PCOMP_ECT ----------|

; low ECT, adapt first      |AND -|

|     |

PCOMP_FST_PS = 0 ------------|     |

; first pass                      |

|

PGM_CVS_FM = 1 --------------|     |

; canister vent failure     |AND -|

|     |

PGM_HW_SW = 1 ---------------|     |

; purge monitor present           |

|OR --| PCOMP_ENA := 0

FFS_FMFLG = 1 ---------------|     |     | PCOMP_FST_PS := 1

; flex fuel sensor failure  |AND -|     |  ; disable PCOMP

|     |     |  ; allow adaptive

FFS_SW = 1 ------------------|     |     |

; flex fuel sensor present        |     |

|     |

pc_restart = 1 --------------|     |     |

|AND -|     |

PGM_ADPT_RST = 1 ------------|           |

; purge monitor test done               |

; restart with adaptive fuel            |

| --- ELSE ---

PCOMP_FST_PS = 0 ------------------|     |

; first pass                      |     |

|OR --| PCOMP_ENA := 1

pc_restart = 1 --------------|     |     | PCOMP_FST_PS := 1

|AND -|     |  ; enable PCOMP

PGM_ADPT_RST = 0 ------------|           |

; purge monitor test done               |

; restart with PCOMP first              |

| --- ELSE ---

|

| ; No change to PCOMP_ENA

| ; due to initialization.

| Do: pcomp_enable

END:  pcomp_ena_init

15-7

CANISTER PURGE STRATEGY, PCOMP ENABLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN:  pcomp_enable

ADAPT_TMR > ADP_TM_MAX ---------------------| PCOMP_ENA := 1

; max adaptive time exceeded               |  ; enable PCOMP

| Exit: pcomp_enable

ADAPT_TMR > ADP_TM -------------------|

|

LAM_BAR1 < ADP_LAM_MIN ---|           |

|AND -|     |

KAM_BAR1 < ADP_KAM_MAX ---|     |     |AND -| PCOMP_ENA := 0

|     |     |  ; disable PCOMP

LAM_BAR1 > ADP_LAM_MAX ---|     |     |     |

|AND -|     |     | Exit: pcomp_enable

KAM_BAR1 > ADP_KAM_MIN ---|     |     |     |

|OR --|     |

LAM_BAR2 < ADP_LAM_MIN ---|     |           |

|AND -|           |

KAM_BAR2 < ADP_KAM_MAX ---|     |           |

|           |

LAM_BAR2 > ADP_LAM_MAX ---|     |           |

|AND -|           |

KAM_BAR2 > ADP_KAM_MIN ---|                 | --- ELSE ---

|

|

ADAPT_TMR > ADP_TM -------------------------| PCOMP_ENA := 1

; min adaptive time exceeded               |  ; PCOMP enabled

; lambse IN acceptable window              |

| --- ELSE ---

PCOMP_W < PCOMP_MN_PPM ---------|           |

; small PCOMP value            |           |

; vapor flow low               |           |

|           |

PG_DC > PG_DC_D_MIN ------------|           |

; above duty cycle disable     |           |

|AND -|     |

F6K_LST_PS >= 1 ----------------|     |     |

; make sure that PCOMP_W will  |     |     |

; not be prematurely trimmed   |     |     |

; due to high N/LOAD conditions|     |     |

|     |     |

FADPT_READY = 1 ----------------|     |     |

; adaptive fuel is ready             |     |

|OR --| PCOMP_ENA := 0

FRST_ADP = 0 -------------------|     |     |  ; disable PCOMP

; have not yet adapted         |AND -|     |  ; allow adaptive

|     |     |

FADPT_READY = 1 ----------------|     |     |

; adaptive fuel is ready             |     |

|     |

TSLADPTMR > PG_TSLADP_TM -------|     |     |

; time since adapt too long    |AND -|     |

|           |

FADPT_READY = 1 ----------------|           |

; adaptive fuel is ready                   |

| --- ELSE ---

(Continued on next page)

15-8

CANISTER PURGE STRATEGY, PCOMP ENABLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(Continued from previous page)

|

| ; No change to PCOMP_ENA

END: pcomp_enable

BEGIN: pcomp_timers

; Executed when called.

; this process updates the timers used by the purge strategy

; ADAPT_TMR logic

PCOMP_ENA = 0 ---------------------|

; PCOMP not active                |

|

OLFLG = 0 -------------------|     |

; in closed loop            |OR --|AND -| Increment ADAPT_TMR

|     |     |

FRST_ADP = 1 ----------------|     |     | FRST_ADP := 1

; adapted at least once           |     |

|     |

ADPTMR > ADAPTM -------------------|     |

; adaptive will be enabled due to       |

; ECT criteria and time expired         |

| --- ELSE ---

|

| ADAPT_TMR := 0

; TSLADPTMR - Time Since Last Adapt Timer logic

PCOMP_ENA = 1 ---------------------|

; PCOMP enabled                   |AND -| Increment TSLADPTMR

|     |

FRST_ADP = 1 ----------------------|     |

; adapted at least once                 |

| --- ELSE ---

|

| TSLADPTMR := 0

END: pcomp_timers

15-9

CANISTER PURGE STRATEGY, CANISTER PURGE COMPENSATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

15.1.2  CANISTER PURGE STRATEGY (PCOMP) (CDAN0)

System Overview:

Canister Purge refers to the vacuum-actuated valve assembly that  is  located

in  the  line  between  the carbon canister and the intake manifold.  The EEC

operates the canister purge valve by outputting a variable duty cycle (PG_DC)

to  an  Electronic  Valve  Regulator  (EVR).   The EVR responds by applying a

vacuum signal, proportional to the duty cycle,  to  the  purge  valve.   When

activated,  the purge valve allows fuel vapors to flow from the fuel tank and

carbon canister to the intake manifold at a rate  proportional  to  the  duty

cycle.   The  purge  valve,  the vapor management valve (VMV), is designed to

compensate for changes in manifold vacuum and will maintain a  constant  flow

at any given duty cycle for manifold vacuums between 5" Hg and 20" Hg.

Strategy Operation:

PCOMP strategy operation is divided into six major phases or steps.   It  can

be understood by the following:

PCOMP steps 1,2,3 -> deal with VAPOR FLOW calculation and usage.

PCOMP steps 4,5,6 -> deal with DUTY CYCLE calculation and output.

THE STRATEGY ENABLES CANISTER PURGE VAPOR COMPENSATION  TO  THE  FUEL  SYSTEM

ONLY DURING CLOSED LOOP ENGINE OPERATION.

IN OPEN LOOP, the output valve duty cycle is  loaded  from  FN666  or  FN667,

based on throttle mode.  For no open loop purge, set FN666 and FN667's values

to zero.

IN CLOSED LOOP, a Purge Duty Cycle Multiplier (PG_RP_ML) is ramped up/down by

the values of PG_RAMP1 or PG_RAMP2 and determined by injector pulsewidth, the

value of  PCOMP,  and  the  HEGO  sensor  state  (rich  or  lean).   PG_RAMP1

determines  the ramp up rate based on HEGO switches during normal closed loop

operation.  Every HEGO switch PG_RP_ML is calculated as follows:

PG_RP_ML(new) = PG_RP_ML(old) + PG_RAMP1

PG_RAMP2 determines a ramp up OR down rate based  on  background  loop  time.

PG_RP_ML is calculated every background loop as follows:

PG_RP_ML(new) = PG_RP_ML(old) - PG_RAMP2

A variable duty cycle output (PG_DC) is then calculated based on the  desired

maximum   duty   cycle  (PG_DC_DS),  the  above-mentioned  purge  duty  cycle

multiplier (PG_RP_ML).  Then,  to  compensate  for  the  VMV  not  physically

opening until a certain offset value is seen, a purge valve offset (PG_DC_OF)

is added.

PG_DC = (PG_DC_DS * PG_RP_ML) + PG_DC_OF

15-10

CANISTER PURGE STRATEGY, CANISTER PURGE COMPENSATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  ADP_STATEn1 = Adaptive HEGO switchpoint algorithm  state  machine  value.

When this value equals 1,2,or 3, the algorithm is seeking to recovery the

HEGO switchpoint.  The value returns to 0 when HEGO switching resumes.

-  AM = Air mass flow.

-  APT = Throttle mode flag.

-  ATMR4 = Time since ECT >= ECTSTABL, sec; Used with PG_OL_START.

-  CYL_AIR_CHG = Current cylinder air charge.

-  ER_STATUS = State pointer that indicated current state of engine  running

on demend test.

-  F_A_RATIOn = Fuel air ratio for HEGOn1.

-  FLOW_RUNNING = Purge flow  monitor  test  is  running  flag;  1  ->  test

running.

-  ISCDTY = Idle speed duty cycle.

-  PGM_RUNNING = Evaporative System test is running flag.

-  FUELPWn = Fuel injector #n pulsewidth, clock ticks.

-  F6K_LST_PS = Stores the output value of FN6000A for  the  PCOMP  enabling

module.

-  INJ_TR = Ratio of the current number of cylinders onto the  total  number

of cylinders.

-  LAMBSEn = LAMBDA equivalence ratio (HEGOn1).

-  LBMF_INJn = Fuel mass per injection for bank #n.

-  LESTMR11 = Lack HEGO11 switch timer, sec.

-  MINPWT = MINPW converted to clock ticks.

-  N = Engine RPM.

-  PCOMP_LBM = Purge compensation fuel adder, lbs mass/cylinder.

-  PCOMP_PPM = The OUTPUT value of PCOMP in pounds per minute, ppm.

-  PCOMP_PPM_C = The CALCULATED PCOMP in pounds per minute, ppm.

-  PCOMP_W = Reduced range PCOMP.  A more stable value.

-  PC_PCOMP_H = Maximum value of percent of pcomp allowed  before  purge  is

decreased.

15-11

CANISTER PURGE STRATEGY, CANISTER PURGE COMPENSATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PC_PCOMP_L = Maximum value of percent of pcomp allowed  before  purge  is

inhibited from increasing.

-  PC_SCALE_FN = Integrator scaling factor  for  PCOMP;  value  loaded  from

either FN_LAMT1 or FN_LAMA1.

-  PCT_LOAD = Percent inferred LOAD (altitude independant).

-  PG_DC = Purge Duty Cycle.

-  PG_FA_RAT = Weighted average  value  between  F_A_RATIO1  and  F_A_RATIO2

determined by PG_BANK_ML.

-  PG_FUELPW = Weighted average value between FUELPW1 and FUELPW2 determined

by PG_BANK_ML.

-  PG_LAMBSE = Weighted average value of LAMBSE1 and LAMBSE2  determined  by

PG_BANK_ML.

-  PG_LAMAV = Filtered value of PG_LAMBSE.  Determined by TC_PGLAM.

-  PG_LBMF  =  Weighted  average  value  between  LBMF_INJ1  and   LBMF_INJ2

determined by PG_BANK_ML.

-  PG_PC_PCOMP = Percent PCOMP of the total fuel flow, unitless.

-  PG_RAMP1 = Purge Duty Cycle ramp rate multiplier based on HEGO switches.

-  PG_RAMP2 = Purge Duty Cycle ramp rate multiplier  based  background  loop

times.

-  PG_RP_ML = Purge Duty Cycle Ramp Multiplier, unitless.

-  TP_REL = Relative TP (TP - RATCH).

Bit Flags:

-  EGOSTATE1 = State of the EGO sensor 11; 1 -> rich, -1 -> lean.

-  ISCFLG = Mode indicator flag, enumerated.

-  LAM_MOD_FLG = Flag used to initiate high frequency  fuel  modulation  for

the upstream EGO monitor.

-  OLFLG = Open Loop Flag; 1 -> open loop conditions met, 0 ->  closed  loop

conditions met.

-  PCOMP_ENA = PCOMP strategy enabled flag; 1 -> PCOMP is enabled,  adaptive

fuel disabled.

-  OSM_EO_OFF = Off state requested for OSM outputs  during  engine  off  on

demand test.

-  OSM_EO_ON = ON state requested for  OSM  outputs  during  engine  off  on

demand test.

-  PC_RESET = Canister overrich condition flag; 1 -> overrich  purge  reset,

ramp PG_DC down to zero.

15-12

CANISTER PURGE STRATEGY, CANISTER PURGE COMPENSATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PG_MULT_INH = Purge multiplier inhibit  flag;  1  ->  inhibit  multiplier

increase.

-  PG_OBD2_HFL = Flag that indicates a OBDII test is signaling purge to HOLD

its flow and duty cycle for the duration of the test.

-  PG_OBD2_NFL = Flag that indicates a OBDII test is signaling purge to STOP

its flow and duty cycle for the duration of the test.

-  PRG_OFF = Flag indicating that no purge flow is occuring and purge is not

enabled.

-  SAIR_TST_ENA =  OBD-II  secondard  air  test  enable  flag,  (1  =>  test

enabled).

-  SWTFL11 = Bank1 upstream HEGO11 switch flag; 0 -> no HEGO11 switch, 1  ->

HEGO11 switch.

Calibration constants:

-  ENGCYL = Number of pip's per engine revolution; 2 -> 4 cylinder engine (2

pip/rev),  3  -> 6 cylinder engine (3 pip/rev), 4 -> 8 cylinder engine (4

pip/rev).

-  FN068(N) = Engine speed normalizing function for FN6000A.

-  FN069B(PCT_LOAD) = Load normalizing function for FN6000A.

-  FN630(AM) = Purge ramp rate; PG_RAMP1.

-  FN631(AM) = Purge ramp rate; PG_RAMP2.

-  FN666 = Purge duty cycle during open loop, part throttle.

-  FN667 = Purge duty cycle during open loop, wide open throttle.

-  FN6000A(FN068(N),FN069B(PCT_LOAD) = pcomp_ppm_new multiplier for  reduced

flows at low manifold vacuums.

-  FN_LAMA1(PG_LAMBSE) = PC_SCALE_FN value when LAMBSE's direction  is  away

from 1.0.

-  FN_LAMT1(PG_LAMBSE) = PC_SCALE_FN value when LAMBSE's direction is toward

1.0.

-  PC_PCOMP_H_N = Maximum value of percent of PCOMP allowed before purge  is

decreased at decels.  % PCOMP.

-  PC_PCOMP_H_0 = Maximum value of percent of PCOMP allowed before purge  is

decreased at accels/cruise.  % PCOMP.

-  PC_PCOMP_H_1 = Maximum value of percent of PCOMP allowed before purge  is

decreased at idle.  % PCOM.

-  PC_PCOMP_L_N = Maximum value of percent of PCOMP allowed before purge  is

increased at decels.  % PCOMP.

15-13

CANISTER PURGE STRATEGY, CANISTER PURGE COMPENSATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PC_PCOMP_L_0 = Maximum value of percent of PCOMP allowed before purge  is

increased at accels/cruise.  % PCOMP.

-  PC_PCOMP_L_1 = Maximum value of percent of PCOMP allowed before purge  is

increased at idle.  % PCOMP.

-  PC_PCOMP_RS = Maximum value of percent PCOMP allowed before  an  overrich

condition is identified.

-  PC_TP_R = Maximum value of relative TP to determine a  canister  overrich

condition.

-  PG_BANK_ML = Calibration value (0-1.0) used to perform weighted  averages

on selected purge parameters.

-  PG_DC_DS = Maximum desired purge duty cycle.  PG_DC_DS + PG_DC_OF  should

equal 1.0.

-  PG_DC_HYS = Purge Duty Cycle hysterisis term.  Used with PG_DC_OF to ramp

down PG_DC to zero faster without affecting the ramp up rate.

-  PG_DC_OF = Purge Duty Cycle Offset.  Purge D.C.  will start at this value

when ramping on.  Should be equal to the start-to-open point of the purge

valve.  PG_DC_DS + PG_DC_OF should equal 1.0.

-  PG_INJ_PW_HI = Minimum value of Injector Pulsewidth required to  increase

purge multiplier, clock ticks.  One clock tick = 0.0024 msec at 15 Mhz.

-  PG_INJ_PW_LO = Minimum value of injector pulsewidth required;  otherwise,

the purge is decreased.

-  PG_INJ_PW_R = Minimum value of injector pulsewidth required to  determine

if a PC_RESET condition exists.

-  PG_ISCDTY_HI = ISCDTY below which multiplier increase is not allowed.

-  PG_ISCDTY_LO = ISCDTY below which PG_RP_ML is decremented.

-  PG_LAMMAX = Maximum purge LAMBSE clip when closed loop.

-  PG_LAMMIN = Minimum purge LAMBSE clip when closed loop.

-  PGM_TCPRG = Time constant for purge duty  cycle,  used  during  VMV  Flow

test.

-  PG_OL_START = Minimum time before allowing open loop purge, sec.

-  PG_TSLE = Time Since Last HEGO switch while at its clip, sec.

-  PRG_FLOW_SW =  Calibration  switch  indicating  if  Purge  Flow  test  is

present.

-  TC_PGLAM = Time constant for PG_LAMAV.

-  TCPRG = Time constant for purge duty cycle.

15-14

CANISTER PURGE STRATEGY, CANISTER PURGE COMPENSATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CALIBRATION INFORMATION

Typical values are supplied for the following calibration constants:

-  PC_TP_R = 30.0.

-  PG_DC_DS = 0.9.

-  PG_DC_OF = 0.1.

-  PG_INJ_PW_HI = 1042 clock ticks.

-  PG_INJ_PW_LO = 833 clock ticks.

15-15

CANISTER PURGE STRATEGY, CANISTER PURGE COMPENSATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CANP_PCOMP_COM10

BEGIN:  pcomp_control

; this is the main process of PCOMP control.

unconditionally ---------------------| PG_LAMBSE := (PG_BANK_ML * LAMBSE1) +

; determine engine bank dependence  |   ((1 - PG_BANK_ML) * LAMBSE2)

; perform weighted averages         |

| PG_LAMAV := ROLAV_TC(PG_LAMBSE,

|             TC_PGLAM)

|  ; filtered value of PG_LAMBSE

|

| PG_FUELPW := (PG_BANK_ML * FUELPW1) +

|   ((1 - PG_BANK_ML) * FUELPW2)

|

| PG_LBMF := (PG_BANK_ML * LBMF_INJ1) +

|   ((1 - PG_BANK_ML) * LBMF_INJ2)

|

| PG_FA_RAT := (PG_BANK_ML * F_A_RATIO1)

|   + ((1 - PG_BANK_ML) * F_A_RATIO2)

1) Determine the values of pcomp_ppm_new:

; PC_SCALE selection logic

; Allows user to select a different value of PC_SCALE_FN depending upon if

; lambse is heading Toward 1.0 or Away from 1.0.

PG_LAMBSE > 1.0 ---------|

; system rich           |AND -|

|     |

EGOSTATE1 = -1 ----------|     |

; ego lean                    |OR --| PC_SCALE_FN := FN_LAMT1(PG_LAMBSE)

|     |

PG_LAMBSE < 1.0 ---------|     |     |

; system lean           |AND -|     |

|           |

EGOSTATE1 = 1 -----------|           |

; ego rich                          |

| --- ELSE ---

|

| PC_SCALE_FN := FN_LAMA1(PG_LAMBSE)

15-16

CANISTER PURGE STRATEGY, CANISTER PURGE COMPENSATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OLFLG = 1 ---------------|

; open loop             |AND -|

|     |

PG_OBD2_HFL = 0 ---------|     |

; not trying to hold          |

|OR __| ; zero flow value

PC_RESET = 1 ------------------|     | pcomp_ppm_new := 0

; canister overrich           |     |

|     |

PCOMP_ENA = 0 -----------------|     |

; PCOMP not enabled                 |

| --- ELSE ---

PG_FUELPW < MINPWT ------|           |

; low injector flow     |AND -|     |

|     |     | ; hold flow value

PG_LAMBSE > 1.0 ---------|     |OR --| pcomp_ppm_new := PCOMP_PPM

; system is rich              |     |

|     |

PG_OBD2_HFL = 1 ---------------|     |

; a OBD2 test needs purge held      |

; at a steady value for duration    |

; of the test.                      |

| --- ELSE ---

|

| ; calculate new flow value

|

| pcomp_ppm_new := (PCOMP_PPM_C +

|   PC_SCALE_FN * [(PG_LAMBSE - 1.0)

|   * (CYL_AIR_CHG * PG_FA_RAT)]

|   * N * ENGCYL)

|

| ; Clip pcomp_ppm_new to zero as min

|

| F6K_LST_PS := FN6000A(N,PCT_LOAD)

2) Determine the value of percent of PCOMP of total fuel:

unconditionally ---------------------| PG_PC_PCOMP :=

|   PCOMP_LBM /(PG_LBMF +

|               PCOMP_LBM)

| ; Clip to zero as a min

3) Determine if canister purge flow is in an overrich condition:

; load % vapor flow limits

ISCFLG = -1 -------------------------| PC_PCOMP_L := PC_PCOMP_L_N

;closed throttle - decels           | PC_PCOMP_H := PC_PCOMP_H_N

|

| --- ELSE ---

|

ISCFLG = 0 --------------------------| PC_PCOMP_L := PC_PCOMP_L_0

;part throttle - accels and cruise  | PC_PCOMP_H := PC_PCOMP_H_0

|

| --- ELSE ---

|

ISCFLG > 0 --------------------------| PC_PCOMP_L := PC_PCOMP_L_1

;closed throttle - idle             | PC_PCOMP_H := PC_PCOMP_H_1

15-17

CANISTER PURGE STRATEGY, CANISTER PURGE COMPENSATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PG_PC_PCOMP > PC_PCOMP_RS --|

; % vapor reset level      |AND -|

|     |

TP_REL < PC_TP_R -----------|     |

; throttle demand low            |

|OR --| ; Overich condition exists

LESTMR11 > PG_TSLE ---------------|     | PC_RESET := 1

; time since last ego while      |

; at its clip too long           |

|

PG_FUELPW < PG_INJ_PW_R ----------|

; fuel pulse width reset level

4) Determine the state of the multiplier inhibit flag:

ADP_STATE11 >= 1 --------|

|AND -| ;In switchpoint recovery mode for HEGO11

ADP_STATE11 <= 3 --------|     | ;(adaptive switchpoint algorithm)

|

|

ADP_STATE21 >= 1 --------|     |

|AND -| ;In switchpoint recovery mode for HEGO21

ADP_STATE21 <= 3 --------|     |

|

PG_FUELPW < PG_INJ_PW_HI ------|

; injector fuel flow low      |

|

PG_PC_PCOMP > PC_PCOMP_L ------|

; % vapor at inhibit level    |

|OR --| PG_MULT_INH := 1

ISCDTY < PG_ISCDTY_HI ---------|     |  ; multiplier increase not allowed

; idle air at inhibit level   |     |

|     |

PG_LAMAV > PG_LAMMAX ----------|     |

|     |

PG_LAMAV < PG_LAMMIN ----------|     |

; lambse in inhibit areas           |

| --- ELSE ---

|

| PG_MULT_INH := 0

|  ; multiplier increase allowed

15-18

CANISTER PURGE STRATEGY, CANISTER PURGE COMPENSATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

5) Determine the ramp multiplier value:

unconditionally ---------------------| ; load appropriate ramp values

| PG_RAMP1 := FN630(AM)

| PG_RAMP2 := FN631(AM)

OLFLG = 1 ---------------|

; open loop             |AND -|

|     |

PG_OBD2_HFL = 0 ---------|     |

; not trying to hold          |

|

PCOMP_ENA = 0 -----------------|

; PCOMP not enabled           |OR --| ; zero multiplier

|     |

PC_RESET = 1 ------------------|     | PG_RP_ML := 0

; canister overrich           |     |

|     |

PG_OBD2_NFL = 1 ---------------|     |

; a OBD2 test needs no purge        |

| --- ELSE ---

PG_FUELPW < PG_INJ_PW_LO ------|     |

; low fuel injector flow      |     |

|OR --| ; decrease multiplier by PG_RAMP2

PG_PC_PCOMP > PC_PCOMP_H ------|     |

; % vapor too high            |     | PG_RP_ML := PG_RP_ML - PG_RAMP2

|     | ; Clip to zero as a minimum

ISCDTY < PG_ISCDTY_LO ---------|     |

; idle air too low                  |

| --- ELSE ---

SWTFL11 = 1 -------------------|     |

; HEGO11 switched this        |     |

background                  |AND -| ; increase multiplier by PG_RAMP1

|     |

PG_MULT_INH = 0 ---------------|     | PG_RP_ML := PG_RP_ML + PG_RAMP1

; multiplier increase allowed       |  ; Clip to 1.0 as a maximum

|

| --- ELSE ---

|

| No change to PG_RP_ML

15-19

CANISTER PURGE STRATEGY, CANISTER PURGE COMPENSATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

6) Calculate and output the final purge duty cycle and flow values:

unconditionally ---------------------| ; PCOMP OBDII overide logic

| Do: pg_obd2_ovrd

PG_OBD2_NFL = 1 ---------------------| pg_dc_calc := 0

; a OBD2 test needs no purge        |

| --- ELSE ---

|

OLFLG = 0 ---------------------------| ; closed loop purge

; in closed loop                    | pg_dc_calc := PG_DC_DS * PG_RP_ML

|

| --- ELSE ---

APT = 0 -----------------------|     |

; at part throttle            |AND -| ; part throttle open loop purge

|     | pg_dc_calc := FN666(AM)

ATMR4 > PG_OL_START -----------|     |

; delay open loop purge             |

| --- ELSE ---

APT = 1 -----------------------|     |

; WOT                         |AND -| ; wide open throttle open loop purge

|     | pg_dc_calc := FN667(AM)

ATMR4 > PG_OL_START -----------|     |

; delay open loop purge             |

| --- ELSE ---

|

| pg_dc_calc := 0

; NOTE: For no open loop purge, set FN666 and FN667's values to zero.

; a OBD2 purge test is running, use a different ramp constant

FLOW_RUNNING = 1 --------------|

|OR --| pg_dc_new := ROLAV_TC((pg_dc_calc +

PGM_RUNNING = 1 ---------------|     |                PG_DC_OF),PGM_TCPRG)

|

| --- ELSE ---

|

| pg_dc_new := ROLAV_TC((pg_dc_calc +

|                PG_DC_OF),TCPRG)

15-20

CANISTER PURGE STRATEGY, CANISTER PURGE COMPENSATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PG_OBD2_HFL = 1 ---------------------| pg_dc_old := PG_DC

; a OBD2 test needs purge held      | pg_dc_new := pg_dc_old

; at a steady value for duration    |

; of the test.                      | ; output the vapor flow values

| PCOMP_PPM_C := pcomp_ppm_new

| PCOMP_PPM := PCOMP_PPM_C  *

|              FN6000A(N,PCT_LOAD)

| PCOMP_LBM := PCOMP_PPM / (N*ENGCYL)

|

| --- ELSE ---

pg_dc_calc = 0 ----------------|     |

|AND -| ; purge valve being ramped closed

PG_DC > (PG_DC_OF + PG_DC_HYS)-|     |

; valve is still flowing            | pg_dc_old := PG_DC

| pg_factor := (pg_dc_new - PG_DC_OF)/

|              (pg_dc_old - PG_DC_OF)

|  ; Clip pg_factor to 1.0 as max

|

| ; output the vapor flow values

| pcomp_ppm_calc_old := PCOMP_PPM_C

| PCOMP_PPM_C :=  pcomp_ppm_calc_old

|                    * pg_factor

| PCOMP_PPM := PCOMP_PPM_C *

|              FN6000A(N,PCT_LOAD)

| PCOMP_LBM := PCOMP_PPM / (N*ENGCYL)

|

| --- ELSE ---

|

pg_dc_calc = 0 ----------------------| ; purge valve closed

|

| PCOMP_PPM_C := 0

| PCOMP_PPM := 0

| PCOMP_LBM := 0

| PC_RESET  := 0

| pg_dc_new := 0

|

| --- ELSE ---

|

| ; normal purge valve operation

|

| pg_dc_old := PG_DC

| pg_dc_new := pg_dc_calc + PG_DC_OF

|  ; Clip pg_dc_new to 99.99% as max

| pg_factor := (pg_dc_new - PG_DC_OF) /

|              (pg_dc_old - PG_DC_OF)

|  ; Clip pg_factor to 1.0 as max

|

| ; output the vapor flow values

| PCOMP_PPM_C := pcomp_ppm_new *

|                   pg_factor

| PCOMP_PPM := PCOMP_PPM_C  *

|              FN6000A(N,PCT_LOAD)

| PCOMP_LBM := PCOMP_PPM / (N*ENGCYL)

15-21

CANISTER PURGE STRATEGY, CANISTER PURGE COMPENSATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

unconditionally ---------------------| PCOMP_W := PCOMP_PPM

|  ; reduced range version of PCOMP.

|  ; does not overflow.

|

| Do: substitute(9,pg_dc_new)

| PG_DC := pg_dc_new

|  ; output state control override.

|

| Do: pg_output_test

|  ; output test mode override

; Signal to other features that no purge flow is occuring.

PG_DC < PG_DC_OF --------------|

; valve is closed - no flow   |AND -| PRG_OFF := 1

|     |

PCOMP_ENA = 0 -----------------|     |

; PCOMP not enabled                 |

| --- ELSE ---

|

| PRG_OFF := 0

END: pcomp_control

BEGIN: pg_obd2_ovrd

; this process is executed only when called.

; this process contains the logic that interfaces with several OBDII tests

; and determines how purge should function during the duration of each test.

PRG_FLOW_SW = 0 -------------------------| FLOW_RUNNING := 0

; flow test not present

INJ_TR <> 1.0 ---------------------|

; injector cutout scheduled       |OR --| ; turn purge off, stop flow

|     |

FLOW_RUNNING = 1 ------------|     |     | PG_OBD2_NFL := 1

; purge flow test active    |AND -|     |

|           |

PRG_FLOW_SW = 1 -------------|           |

; flow test present                     |

| --- ELSE ---

|

| PG_OBD2_NFL := 0

15-22

CANISTER PURGE STRATEGY, CANISTER PURGE COMPENSATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

ER_STATUS = EGO_ER_INIT -----------|

; KOER EGO test active            |

|

LAM_MOD_FLG = 1 -------------------|OR --| ; hold purge, steady flow

; EGO monitor active              |     |

|     | PG_OBD2_HFL := 1

SAIR_TST_ENA = 1 ------------------|     |

; SAIR test active                      |

| --- ELSE ---

|

| PG_OBD2_HFL := 0

END:  pg_obd2_ovrd

BEGIN:  pg_output_test

; This process overides normal purge logic  when  the  output  ;  test  logic

commands.

OSM_EO_OFF = 1 --------------------------| PG_DC := 0.0 ;  outputs  commanded

off

OSM_EO_ON = 1 ---------------------------| PG_DC := 1.0 ;  outputs  commanded

on

END:  pg_output_test

15-23

CANISTER PURGE STRATEGY, CANISTER PURGE COMPENSATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

15.2  FEATURE:   RUNNING  LOSS  OBD-II   EVAPORATIVE   SYSTEM   MONITOR.    -

V1.9A_CANP_MONITOR (CDAN2)

15.2.1  TANK PRESSURE SENSOR INPUT AND SELF TEST (CDAJ0)

OVERVIEW

This module reads and converts the tank pressure sensor input.   The  A/D  is

read  and  the  raw  counts  (TPR_CNTS)  are converted into engineering units

(TPR_ENG).  TPR_ENG is the value used  when  performing  any  input  testing.

And,  it  is  this  value  that will be reported to the scan tool.  Next, the

TPR_ENG value is tested for "out of range" or other failure conditions.   The

parameter  TPR  will  be modified by any FMEM.  If a failure is present for a

sufficient amount of time, the appropriate  malfunction  flag  (PxxxMALF)  is

set.   Finally,  a  timer  is  checked  to  see  if  the  component  has been

sufficiently monitored for this "trip".

DEFINITIONS

INPUTS

Registers:

-  TPR_CNTS = Raw A/D counts from sensor.

-  TPR_ENG = Engineering units value before any FMEM action.  Used for  FMEM

testing and output to scan tool.

-  TPR_ER_TMR = Timer that runs when a fault is present.

-  TPR_FM_TMR = Timer that is reset when the  failure  conditions  for  FMEM

mode are met, seconds.

-  PUTMR = Time since power up, seconds.

Bit Flags:

-  PURG_TST_ENA = OBDII purge test enable flag ( 1 -> test enabled ).

-  CCM_TST_ENA = OBDII Comprehesive Component test ( CCM ) enable flag.

-  CCM_ER_ENA = On demand test for CCM enable flag.

-  PxxxMALF = OBDII malfunction flag for  fault  xxx;  1  ->  a  malfunction

currently exists for fault xxx.

-  PxxxMON = OBDII monitor flag for fault xxx;  1  ->  fault  xxx  has  been

monitored at least once since power-up.

Calibration Constants:

-  FN610(TPR_CNTS) = Function to convert  from  A/D  counts  to  engineering

units.   (Note:   Since  the  failure mode testing is done in engineering

units, the sensor transfer function MUST be calibrated  from  0  to  1023

counts.)

15-24

CANISTER PURGE STRATEGY, TANK PRESSURE SENSOR INPUT AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TC_TPR = Time constant for tank pressure.

-  TPR_FMEM = FMEM value used when TPR_ENG is out of range, inches of water.

-  TPR_ER_TM = Time with failure present required to set code, seconds.

-  TPR_FM_TM = Minimum time in FMEM mode once initiated, seconds.

-  TPR_HI_ER = Maximum acceptable value for TPR, inches of water.

-  TPR_LO_ER = Minimum acceptable value for TPR, inches of water.

-  TPR_MON_TM = Time required to indicate that a "trip" is complete for this

sensor, seconds.

OUTPUTS

Registers:

-  PGM_TANK_PRS = Purge monitor tank pressure, inches of water.

-  TPR_ENG = See above.

-  TPR_ER_TMR = See above.

-  TPR_FM_TMR = See above.

-  TPR = Engineering units value with any FMEM corrections applied.  Used by

rest of strategy.

Bit Flags:

-  PxxxMALF = See above.

-  PxxxMON = See above.

OTHER

-  purg_codes = Set of {P0442,P0446,P0452,P0453,P1442}.  The  set  of  OBDII

fault codes that relate to purge monitor.

-  malfunction(purg,Pxxx) = Logic process, imported  from  the  MIL  control

Module;  Pxxx  indicates  the  fault  code.   Also sets the corresponding

PxxxMALF flag.

-  clear_malf(Pxxx) = Logic process that clears the appropriate Pxxx code.

-  P0452 = Fault code, fuel tank pressure sensor circuit out of range low.

-  P0453 = Fault code, fuel tank pressure sensor circuit out of range high.

15-25

CANISTER PURGE STRATEGY, TANK PRESSURE SENSOR INPUT AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CANP_INPUT_TPR_COM1

always -------------------------| Increment TPR_ER_TMR

| Increment TPR_FM_TMR

always -------------------------| TPR_ENG = FN610(TPR_CNTS)

|  (read A/D, Convert to engineering units)

PURG_TST_ENA = 1 ---------|

|

CCM_ER_ENA = 1 -----------|OR --| Do: TPR_FAILURE_CHECK

|     |  (check for out of range)

CCM_TST_ENA = 1 ----------|     | Do: TPR_MONITORED_CHECK

|  (test for component monitored)

|

| --- ELSE ---

|

| Exit module.

15-26

CANISTER PURGE STRATEGY, TANK PRESSURE SENSOR INPUT AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: TPR_FAILURE_CHECK

Check for  "open/short"  failures  and  other  failures  as  appropriate  for

particular  sensor.   Perform  FMEM  action  if  a  failure is detected.  Set

malfunction flag when a malfunction has been present for a sufficient time.

TPR_ENG > TPR_HI_ER ------|

(value too high)         |

|

TPR_ER_TMR > TPR_ER_TM ---|AND -| Do: malfunction(purg,P0453)

(failure present long    |     |      (update MIL and set MALF)

enough)                 |     | TPR_FM_TMR = 0

|     | PGM_TPR_FM = 1

P0453MALF = 0 ------------|     |  (set tank pressure sensor FMEM flag)

|

| --- ELSE ---

|

TPR_ENG > TPR_HI_ER ------------| PGM_TPR_FM = 1

(value too high; not present   |  (set tank pressure sensor FMEM flag)

long enough to set            | TPR_FM_TMR = 0

malfunction flag)             |

|

| --- ELSE ---

TPR_ENG < TPR_LO_ER ------|     |

(value too low)          |     |

|     |

TPR_ER_TMR > TPR_ER_TM ---|AND -| Do: malfunction(purg,P0452)

(failure present long    |     |      (update MIL and set MALF)

enough)                 |     | TPR_FM_TMR = 0

|     | PGM_TPR_FM = 1

P0452MALF = 0 ------------|     |  (set tank pressure sensor FMEM flag)

|

|

|

| --- ELSE ---

|

TPR_ENG < TPR_LO_ER ------------| PGM_TPR_FM = 1

(value too low; not present    |  (set tank pressure sensor FMEM flag)

long enough to set            | TPR_FM_TMR = 0

malfunction flag)             |

| --- ELSE ---

(continued on next page)

15-27

CANISTER PURGE STRATEGY, TANK PRESSURE SENSOR INPUT AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

P0453MALF = 1 ------|           |

|OR --|     |

P0452MALF = 1 ------|     |AND -| PGM_TPR_FM = 1

|     |  (Remain in FMEM mode until

TPR_FM_TMR < TPR_FM_TM ---|     |   it times out)

|

| --- ELSE ---

|

| PGM_TANK_PRS =  ROLAV(TPR_ENG,TC_TPR)

|  (no failure detected, return

|   value for normal use)

| TPR_ER_TMR = 0

|  (reset failure present timer)

| PGM_TPR_FM = 0

|  (clear failure flags)

| Do: clear_malf(P0452)

| Do: clear_malf(P0453)

END: TPR_FAILURE_CHECK

BEGIN: TPR_MONITORED_CHECK

If enough time has passed since power up (or run mode, or...) assume that

enough A/D readings have been taken to assume that a hard fault, if present,

would have been detected.

PUTMR > TPR_MON_TM -------------| P0453MON = 1

(enough time for sufficient    | P0452MON = 1

A/D readings)                 |  (codes P0453 & P0452 monitored)

END:  TPR_MONITORED_CHECK

15-28

CANISTER PURGE STRATEGY, CVS FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

15.2.2  CVS FMEM (CDAN0)

OVERVIEW

This module contains 2 processes.  Each deals with the Canister Vent Valve.

[cvs_fmem_check] reads the tank pressure every background loop and determines

if  the  canister  vent  solenoid is erroneously stuck closed.  Normally, the

canister vent solenoid should be at zero duty cycle and open,  except  during

the purge monitor test.  If the vent solenoid is stuck closed, when the purge

monitor test is not on, then tank  pressures  will  be  dangerously  low.   A

FMEM/fault code will be set which will, most importantly, disable purge.

[cvs_osm_eo_test] closes the CVV/CVS (which is normally OPEN)  by  sending  a

duty cycle of 1.0 during the key on, engine off output test mode.

INPUTS/OUTPUTS

Registers:

-  PG_DC = Canister purge duty cycle.

-  PGM_TANK_PRS = Purge monitor tank pressure, inches of water.

Bit Flags:

-  PGM_RUNNING = Purge monitor test is running flag; 1 -> running.

-  PxxxMALF = OBDII malfunction flag for  fault  xxx;  1  ->  a  malfunction

currently exists for fault xxx.

-  PxxxMON = OBDII monitor flag for fault xxx;  1  ->  fault  xxx  has  been

monitored at least once since power-up.

-  PGM_CVS_FM = Canister vent solenoid FMEM flag; 1 -> CVS failed.

Calibration Constants:

-  PGM_PRS_HI  =  Tank  pressure  which  is  acceptable  when  purge  is  on

(PGM_PRS_LO + hysteresis), inches of water.

-  PGM_PRS_LO = Lowest tank pressure allowed when purge  is  on,  inches  of

water.

OTHER

-  purg_codes = Set of {P0442,P0446,P0452,P0453,P1442}.  The  set  of  OBDII

fault codes that relate to purge monitor.

-  malfunction(purg,Pxxx) = Logic process, imported  from  the  MIL  control

Module;  Pxxx  indicates  the  fault  code.  Also sets corresponding MALF

flag.

15-29

CANISTER PURGE STRATEGY, CVS FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  clear_malf(Pxxx) = Logic process that clears the appropriate fault code.

-  P0453 = Fault code, fuel tank pressure sensor circuit out of range high.

-  P0452 = Fault code, fuel tank pressure sensor circuit out of range low.

-  P0446 = Fault code, purge monitor canister vent solenoid stuck closed.

15-30

CANISTER PURGE STRATEGY, CVS FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CANP_CVS_FMEM_COM1

BEGIN: cvs_fmem_check

PGM_TANK_PRS < PGM_PRS_LO -------|

(tank pressure is too low)      |

|

PGM_RUNNING = 0 -----------------|

(purge monitor is not running)  |

|

P0453MON = 1 --------------------|

|

P0452MON = 1 --------------------|

(tank pressure sensor has been  |

monitored)                     |AND -| PGM_CVS_FM = 1

|     |  (Canister Vent Solenoid

P0453MALF = 0 -------------------|     |   erroneously stuck closed)

|     | Do: malfunction(purg,P0446)

P0452MALF = 0 -------------------|     |      (update MIL and set MALF)

(tank pressure sensor is O.K.)  |     | P0446MON = 1

|     |  (code P0446 monitored)

PG_DC > 0 -----------------------|     |

(vapor management valve is on)  |     |

|     |

P0446MALF = 0 -------------------|     |

| --- ELSE ---

PGM_TANK_PRS < PGM_PRS_LO -------|     |

(tank pressure is too low)      |     |

|     |

PGM_RUNNING = 0 -----------------|     |

(purge monitor is not running)  |     |

|     |

P0453MALF = 0 -------------------|     |

|     |

P0452MALF = 0 -------------------|AND -| PGM_CVS_FM = 1

(tank pressure sensor is O.K.)  |     |  (Canister Vent Solenoid

|     |   erroneously stuck closed)

P0453MON = 1 --------------------|     |

|     |

P0452MON = 1 --------------------|     |

(tank pressure sensor has been  |     |

monitored)                     |     |

|     |

PG_DC > 0 -----------------------|     |

(VMV commanded on)                    |

| --- ELSE ---

(continued on next page)

15-31

CANISTER PURGE STRATEGY, CVS FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

PGM_TANK_PRS > PGM_PRS_HI -------|     |

(tank pressure is O.K.)         |     |

|     |

PGM_RUNNING = 0 -----------------|     |

(purge monitor is not running)  |     |

|     |

P0453MALF = 0 -------------------|     |

|     |

P0452MALF = 0 -------------------|AND -| PGM_CVS_FM = 0

(tank pressure sensor is O.K.)  |     |  (Canister Vent Solenoid O.K.)

|     |

P0453MON = 1 --------------------|     | P0446MON = 1

|     |  (code P0446 monitored)

P0452MON = 1 --------------------|     |

(tank pressure sensor has been  |     | Do: clear_malf(P0446)

monitored)                     |     |  (clear malfunction flag)

|

PG_DC > 0 -----------------------|

(VMV commanded on)

END: cvs_fmem_check

15-32

CANISTER PURGE STRATEGY, VAPOR MANAGEMENT VALVE OSM TEST LOGIC- CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

15.2.3  VAPOR MANAGEMENT VALVE OSM TEST LOGIC (CDAM0)

OVERVIEW

This module monitors the control circuitry  of  the  vapor  management  valve

(VMV).   The  voltage across the VMV is monitored and brought into the EEC by

way of an analog channel.  The EEC converts this  voltage  to  a  A/D  counts

value  (VMV_OSM_CNTS).   This  value is compared with two calculated expected

threshold values and this determines if the valve is functioning correctly or

not.

The VMV is a duty cycled solenoid, therefore to test it  correctly,  the  OSM

test  must  be  performed  when the VMV is nearly completely ON (> 90% dc) or

completely OFF (0% dc).  These conditions are commanded by logic in the purge

control  module for the engine off on demand test.  As for the other possible

tests, PG_DC must be set to 1.0 or 0 before this test will complete.

DEFINITIONS

Registers:

-  CANP_FF = Vapor Management Valve fault filter.

-  PG_DC = Canister purge duty cycle; 1.0 -> 100%, 0 -> 0%.

-  VMV_OSM_CNTS = VMV OSM raw analog channel input value.

-  VMV_OSM_TMR = Timer that tracks the amount of time that the VMV has  been

actively monitored.

-  VBAT = Battery voltage.

-  VMV_ZDC_MIN = Calculated expected VMV OSM counts value at 0% DC.

-  VMV_MXDC_MAX = Calculated expected VMV OSM counts value at >90% DC.

Bit Flags:

-  CCM_EO_ENA = Engine off on demand test for CCM enabled.

-  CCM_ER_ENA = On demand test for CCM enabled.

-  CCM_TST_ENA = OBDII Comprehensive Component Test Enable Flag.

-  OSM_EO_ON = ON state requested for OSM outputs during engine off demand.

-  OSM_EO_OFF = OFF state requested for OSM outputs  during  engine  off  on

demand test.

-  OSM_1ST_PS = First pass flag of the VMV OSM test; 1 -> first pass seen.

-  PURG_TST_ENA = OBDII purge test enable flag ( 1 -> test enabled ).

-  P0443MON = OBDII monitored flag for code 0443.

15-33

CANISTER PURGE STRATEGY, VAPOR MANAGEMENT VALVE OSM TEST LOGIC- CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  VMV_MXDC_ML = Multiplier used with VMV_MXDC_MAX.

-  VMV_MXDC_SUB = Subtractor used with calculation of VMV_MXDC_MAX .

-  VMV_ZDC_ML = Multiplier used with VMV_ZDC_MIN.

-  VMV_ZDC_SUB = Subtractor used with VMV_ZDC_MIN.

-  TC_CANP = Time constant used in the ROLAV filtering check of CANP_FF.

-  CANP_THRES = Threshold used with the rolav filtering check of CANP_FF.

-  PG_DC_MAX = Duty cycle of the purge valve above which it is considered to

be at its maximum ON value, %.

-  VMV_OSM_TM = Time above which the VMV is considered completely monitored.

-  VBAT_CCM_MIN = Minimum voltage to perform OSM tests.

OTHER

-  purg_codes = Set  of  (P0442,P0443,P0446,P0452,P0453,P0455)  OBDII  fault

codes.

-  P0443 = Fault code for the vapor management valve circuit malfunction.

15-34

CANISTER PURGE STRATEGY, VAPOR MANAGEMENT VALVE OSM TEST LOGIC- CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CANP_VMV_OSM_COM4

BEGIN: vmv_osm_check        ; done once per background loop

OSM_EO_ON = 1 --------------|

|

OSM_EO_OFF = 1  ------------|

|

CCM_ER_ENA = 1  ------------|OR --|AND -| Do: vmv_osm_test.

|     |     |

CCM_EO_ENA = 1  ------------|     |     |

|     |     |

PURG_TST_ENA = 1 -----------|     |     |

|     |     |

CCM_TST_ENA = 1 ------------|     |     |

|     |

VBAT >= VBAT_CCM_MIN -------------|     |

| --- ELSE ---

|

| Exit vmv_osm_check.

END: vmv_osm_check

BEGIN: vmv_osm_test

; This routine checks to see if the first pass has been seen.

OSM_1ST_PS = 0 -------------------------| VMV_OSM_TMR := 0

; first time through                   |  ; clear timer

| OSM_1ST_PS := 1

; This routine compares the actual count readings of the vapor management

; valve output state monitor (from the A/D channel) to calculated expected

; readings. Based on the comparison, the logic determines if the vapor

; management valve is functioning correctly.

; calculate the expected values from equations governing the OSM circuit

unconditionally ------------------------| VMV_ZDC_MIN :=  (VMV_ZDC_ML*VBAT)

|                  - VMV_ZDC_SUB

| VMV_MXDC_MAX := (VMV_MXDC_ML*VBAT)

|                  - VMV_MXDC_SUB

; compare the measured and the calculated expected values.

PG_DC >= PG_DC_MAX ---------|

; VMV commanded on         |AND -|

|     |

VMV_OSM_CNTS > VMV_MXDC_MAX |     |OR --| CANP_FF := ROLAV(1,TC_CANP)

|     |  ; error seen, filter to 1

PG_DC = 0 ------------------|     |     |

; VMV commanded off        |AND -|     | Increment VMV_OSM_TMR.

|           |

VMV_OSM_CNTS < VMV_ZDC_MIN -|           |

| --- ELSE ---

(continued on next page)

15-35

CANISTER PURGE STRATEGY, VAPOR MANAGEMENT VALVE OSM TEST LOGIC- CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

PG_DC >= PG_DC_MAX ---------|           |

; VMV commanded on         |AND -|     |

|     |     |

VMV_OSM_CNTS < VMV_MXDC_MAX |     |OR --| CANP_FF := ROLAV(0,TC_CANP)

|     |  ; NO error seen, filter to 0

PG_DC = 0 ------------------|     |     |

; VMV commanded off        |AND -|     | Increment VMV_OSM_TMR.

|           |

VMV_OSM_CNTS > VMV_ZDC_MIN -|           |

| --- ELSE ---

|

| Hold CANP_FF at current value.

|  ; PG_DC <> 1 or 0 - cannot test

|

| Hold VMV_OSM_TMR at current value.

CANP_FF > CANP_THRES -------------------| Do: malfunction(purg,P0443)

; apply threshold check to decide the  |

; error status                         |

| --- ELSE ---

|

| Do: clear_malf(P0443)

VMV_OSM_TMR > VMV_OSM_TM ---------------| P0443MON := 1

; check if enough time has passed      |

; to say if completely monitored       |

| --- ELSE ---

|

| P0443MON := 0

END: vmv_osm_test

15-36

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

15.2.4  CANISTER PURGE MONITOR (CDAN2)

OVERVIEW

This logic describes the vacuum leak down test of the canister  purge  system

for  compliance  with  CARB  and EPA OBD-II requirements.  The purpose of the

test is to identify any leaks in the fuel/canister purge  system  that  would

cause  vapor  to  escape  to the atmosphere.  After PCOMP has been active for

some amount of time, the test is run by closing the atmospheric vent  on  the

canister and applying a vacuum to the fuel system and observing if the vacuum

is held.  The test passes if the system can successfully hold the vacuum  for

a period of time.

o  Entry Conditions:

The test will begin if all of the following entry conditions are met:

.  The test has not yet been run this trip.

.  Powertrain load is within the calibrated window.

.  The vehicle speed is within a calibrated window.

.  ACT/ECT is below a calibrated maximum value.

.  Tank pressure before testing is within a calibrated window.

.  Time since closed loop is greater than a calibrated minimum value.

.  PG_DC is above a calibrated value.

.  PGM_PCOMP_W is below a calibrated value.

o  Test Description:

There are six test phases in addition to a pre-test phase.  The  pre-test

phase is simply the time immediately preceding the beginning of the purge

monitor test when PCOMP is  actively  purging  down  the  vapors  in  the

system.  The six test phases begin at phase zero and are as follows:

.  Tank vacuum application phase:

In this portion of the test, an attempt is made to achieve a vacuum of  a

calibrated  value  in  the  fuel system.  If the tank vacuum is ABOVE the

calibratable value, the Canister Vent Valve (CVV/CVS) is ramped closed to

atmosphere.   Since  the  VMV  is open, the engine will see vapor that is

very rich with fuel vapor.  For this reason, PCOMP is  still  enabled  to

allow  the  engine  to  consume  the  vapor,  but the "leakage term" that

represents the unmetered air flowing through the  purge  system  must  be

forced  to  zero.   If the tank vacuum is BELOW a calibratable value, the

VMV is closed and the vacuum is bled out of the CVS until the  target  is

reached.   After  reaching  the target, the CVS is closed.  If the target

vacuum is not reached in a calibrated amount of time, it must be  assumed

that this is a result of a fuel system leak - the test fails and an error

code is stored.  If the target vacuum is reached, the VMV,  if  open,  is

closed and phase 1 is entered.

.  Vacuum stabilization phase:

In this portion of the test, the tank vacuum is allowed to stablize for a

calibratable period of time and then enter phase 2.  This compensates for

overshoots and undershoots which would interfere with the accuracy of the

vacuum bleedup reading.

15-37

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

.  Vacuum hold phase:

This phase tests the capability of the fuel/evap system to hold a vacuum.

Both the VMV and the Canister Vent Valve are held closed in order to hold

the vacuum for a calibrated period of time.   At  the  end  of  the  time

period,  the  change  in fuel tank vacuum is calculated and this value is

compared to a calibrated maximum acceptable pressure  change.   The  test

passes if the pressure change is below the maximum allowable value and if

it is above the maximum, the test continues to phase 3.

.  Tank pressure stabilization phase:

In this portion of the test, the tank pressure is stabilized  by  opening

the  Canister  Vent  Valve  and  closing the Vapor Management Valve.  The

logic waits for a calibratable time to pass and the tank pressure  to  be

within limits to continue to phase 4 of the test.

.  Pressure build phase:

In this portion of the test, the system is sealed by closing both the VMV

and  the Canister Vent Valve.  The pressure is monitored and the increase

in tank pressure is calculated over a period of time.  This part  of  the

test will indicate the extent to which pressure is increasing in the tank

due to vapor  generation.   If  the  increase  in  pressure  is  above  a

calibrated maximum value, the test results are rejected, since the "bleed

down" rate will be skewed by vapor generation.  If the pressure  increase

is  below the calibrated maximum value, vapor generation is within reason

and this value is subtracted from  the  vacuum  bleedup  of  phase  2  to

determine  the  true  bleedup rate of a possible leak.  If the bleedup is

still above the limit, a failure is indicated.

.  End of test:

The final phase of the test returns the purge  system  to  normal  purge.

The  Canister Vent Valve is gradually opened at a calibrated ramp rate to

the full open position.  Finally, the system  is  allowed  to  return  to

either  purge  or  adaptive  fuel learning, depending on which the engine

strategy is requesting at the time.

o  Early Exit Conditions (no error code stored)

Over the duration of the test, a few conditions  are  possible  that  may

require the early termination of the test:

-  operation out of a load window.

-  operation out of the vehicle speed window.

-  failure of the tank pressure sensor.

-  failure of the canister vent valve.

-  re-entry into open loop

-  engine stall

-  excessive pressure/vacuum on the tank

15-38

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  ACT = Air Charge Temperature, deg.  F.

-  CL_TMR = Closed loop timer (actuated).

-  ECT = Engine coolant temperature, deg F.

-  KWUCTR = Warm up counter for Adaptive learning.

-  LOAD = Universal load as a ratio of aircharge over standard.

-  PCT_LOAD = Percent inferred load (altitude independent).

-  PCOMP_W = Calculated PCOMP flow in pounds per minute.

-  PGM_PCOMP_W = Filtered PCOMP flow in pounds per minute.

-  PG_DC = Purge duty cycle.

-  PGM_BLED_KAM = Weighted moving average of the delta of pressures used  to

determine  if  a  small  leak  exists  (P0442 -> unable to hold vacuum in

tank).

-  PGM_CVS_DC = Purge monitor canister vent solenoid valve duty cycle.

-  PGM_ERROR = Present error condition of the purge monitor system.

-  PGM_FK = Filter  constant  used  with  the  weighted  moving  average  of

PGM_BLED_KAM.  Value is either PGM_FK_NMAT or PGM_FK_MAT.

-  PGM_LOAD_DLT = Absolute value of  the  difference  between  the  filtered

PGM_LOAD and the current value of LOAD.

-  PGM_PS0_BEGP = Tank pressure at beginning of phase zero.

-  PGM_P0_BEGK = Value of PGM_PS0_BEGP stored to KAM for SCP reporting.

-  PGM_PS2_BEGP = Tank pressure at beginning of phase two.

-  PGM_PS2_DLTP = Delta pressure in tank during phase 2.

-  PGM_P2_DLTPK = Value of PGM_PS2_DLTP stored to KAM for SCP reporting.

-  PGM_PS4_BEGP = Tank pressure at beginning of phase four.

-  PGM_PS4_DLTP = Delta pressure in tank during phase 4, not caused by vapor

generation.

-  PGM_P4_DLTPK = Value of PGM_PS4_DLTP stored to KAM for SCP reporting.

-  PGM_PS4_DELP = Delta pressure in  tank  during  phase  4,  with  phase  2

compensation.

-  PGM_P4_DELPK = Value of PGM_PS4_DELP stored to KAM for SCP reporting.

15-39

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PGM_BLD_DLTK = Value of PGM_BLD_DLT stored to KAM for SCP reporting.

-  PGM_PS_TMR = Purge monitor phase timer (free run).

-  PGM_RUN_CTR = Purge monitor run counter, tracks how  many  times  monitor

has been run during a single Key-ON event.

-  PGM_RST_CTR = Purge monitor restart counter.

-  PGM_RST_TMR = Timer that counts down time between  attempts  to  run  the

purge monitor test after a reset.

-  PGM_ST = Purge monitor state.

-  PGM_TANK_PRS = Purge monitor tank pressure.

-  PGM_TEMP = Temperature (ECT/ACT combination) used in purge monitor.

-  PGM_TEMP_CTR = Counter that tracks the number of times that the  PGM_TEMP

window has been entered.

-  PGM_TPR_DLT = Absolute value of the difference between the filtered  tank

pressure, PGM_TANK_PRS, and the current value of TPR_ENG.

-  PGM_VBAT_DLT = Absolute value of  the  difference  between  the  filtered

PGM_VBAT and the current value of VBAT.

-  TPR_ENG = Tank pressure converted from A/D.

-  VSBAR = Vehicle speed, MPH.

-  PGM_0442_DLP = Parameter that holds the value of the pressure change used

in PGM_BLED_KAM check.

-  PGM_P0_WPEK = Peak PCOMP_W seen during phase 0.

-  FN_PGM_BLEED = Purge monitor's allowed bleedup pressure  as  function  of

PGM_P0_WPEK.

-  PGM_BLD_DLT = Purge monitor bleed pressure allowed before test is failed,

from FN_PGM_BLEED(PGM_P0_WPEK).

|      -  ER_STATUS = State pointer that indicates current state of engine  running

|         on demand test.

Bit Flags:

-  EGR_MON = OBDII monitor flag for EGR systems; 1=>  all  EGR  faults  been

monitored since power-up.

-  FFG_PURG = OBDII system FMEM flag for the  purge  system  -  1  ->  purge

system is not under control reliably.

-  FLG_STALL = Indicates a stall has occurred; transition.

-  KAM_ERROR = Flag that indicates the keep alive memory (KAM) is invalid.

15-40

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  OBDII_RESET = Flag used to simulate the receipt of  an  OBDII  scan  tool

reset message.

-  OLFLG = Open loop flag; 1 -> open loop mode, 0 -> closed loop mode.

-  PGM_CVS_FM = Canister Vent Valve FMEM flag; 1 -> CVS failed.

-  PGM_DONE = Purge monitor test is done flag; 1 -> test is done.

-  PGM_PG_ON = Purge monitor command to turn PCOMP/PURGE on.

-  PGM_PS0_SET = Phase zero set-up flag; 1 -> set-up complete.

-  PGM_PS1_SET = Phase one set-up flag; 1 -> set-up complete.

-  PGM_PS2_SET = Phase two set-up flag; 1 -> set-up complete.

-  PGM_PS3_SET = Phase three set-up flag; 1 -> set-up complete.

-  PGM_PS4_SET = Phase four set-up flag; 1 -> set-up complete.

-  PGM_RUNNING = Purge monitor test is running flag; 1 -> running.

-  PGM_TPR_FM = Tank Pressure Sensor FMEM flag; 1 -> TPR failed.

-  PGM_TRG_SEEN = Target vacuum range has been entered flag.

-  PGM_VAC_SEEN = Specific vacuum value has been reached  flag.   Used  with

the logic that checks for exessive vacuum.

-  PGM_TMP_SEEN = PGM_TEMP window has been seen this key on flag.

-  PGM_TEMP_FLG = Flag that indicates if PGM_TEMP_CTR has  been  incremented

this Key On.

-  PURG_MON = OBDII monitor flag for  evaporative  purge;  1  ->  all  purge

faults been monitored since power-up.

-  PURG_TST_ENA = Purge test enabled flag; 1 -> Purge test has been  enabled

by the OBD-2 executive.

-  PURG_TST_RDY = Purge test ready flag; 1 -> all local conditions have been

met, ready to run purge test.

-  PxxxxMALF = OBDII malfunction flag for fault xxxx;  1  ->  a  malfunction

currently exists for fault xxxx.

-  PxxxxMON = OBDII monitor flag  for  fault  xxxx;  1  ->  fault  has  been

monitored at least once since power-up.

-  TRIP_NOT_PRG = Flag that indicates all other  OBDII  monitors  have  been

monitored, except for purge.

-  OSM_EO_OFF = Off state requested for OSM outputs  during  engine  off  on

demand.

Calibration Constants:

15-41

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PGM_BLD_INIT = Initial value of PGM_BLED_KAM after KAM reset.

-  PGM_CL_TM = Time in closed loop to allow purge monitor test.

-  PGM_CVS_DEC = Purge  monitor  canister  vent  solenoid  valve  duty  cyle

decrement used during phase five.

-  PG_DC_LOW = Calibratable minimum value of PG_DC.

-  PGM_P0_DCMIN = Calibratable minimum value of PG_DC  in  phase  0.   Below

which, amount of vacuum will probably be too low to run correctly.

-  PGM_FK_NMAT = PGM_FK value used when KAM is NOT considered mature.

-  PGM_FK_MAT = PGM_FK value used when KAM is considered mature.

-  PGM_KWUCNT = Maximum mumber of warm_up cycles to use PGM_FK_NMAT.

-  PGM_LOAD_MAX = Maximum PCT_LOAD to allow purge monitor test.

-  PGM_LOAD_MIN = Minimum PCT_LOAD to allow purge monitor test.

-  PGM_PG_START = Purge duty cycle above which to allow the purge monitor to

begin.

-  PGM_PPM_WMX = Maximum value of  PGM_PCOMP_W  allowed  to  run  the  purge

monitor.

-  PGM_PS1_TM = Allowable time in phase one to stabilize tank pressure, sec.

-  PGM_PS1_MAX = Maximum tank pressure allowed during phase one, in water.

-  PGM_PS3_TM = Allowable time in phase three to  stabilize  tank  pressure,

sec.

-  PGM_PS3_MAX = Maximum tank pressure allowed during phase three, in water.

-  PGM_PS3_MIN = Minimum tank pressure allowed during phase three, in water.

-  PGM_PS4_DP = Maximum allowable delta presure in phase four to allow purge

monitor test.

-  PGM_PS4_DPML = Purge monitor delta pressure multiplier; multiplies  phase

four delta pressure for compatability with phase two delta pressure.

-  PGM_PS4_TM = Allowable time in phase four to build pressure.

-  PGM_PS0_LO = Low tank pressure value of phase zero deadband.

-  PGM_PS0_HI = High tank pressure value of phase zero deadband.

-  PGM_PS0_TM = Allowable time in phase zero to allow system to pull vacuum.

-  PGM_P0_WMX = Maximum value of PCOMP_W allowed in phase 0.

-  PGM_PS2_TM = Allowable time in phase two to verify vacuum is held.

15-42

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PGM_RUN_CNT = Number of times purge monitor is  allowed  to  be  run  per

engine key-on before a malfunction is indicated.

-  PGM_RST_CNT = Number of times purge monitor is allowed to  be  tried  per

engine key-on.

-  PGM_RST_TM = Time required  before  Purge  Monitor  Test  is  allowed  to

restart.

-  PGM_TANK_MAX = Maximum tank pressure to allow purge monitor test.

-  PGM_TANK_MIN = Minimum tank pressure to allow purge monitor test.

-  PGM_TEMP_MAX = Maximum temperature to allow purge monitor test.

-  PGM_TEMP_MIN = Minimum temperature to allow purge monitor test.

-  PGM_TEMP_CNT = Minimum number of times that PGM_TEMP  must  have  entered

the proper window before PURG_MON temp bypass is run.

-  PGM_VS_MAX = Maximum vehicle speed to allow purge monitor test.

-  PGM_VS_MIN = Minimum vehicle speed to allow purge monitor test.

-  PGM_LD_DT_MX = Maximum value of PGM_LOAD_DLT above which the  purge  test

is aborted.

-  PGM_TP_DT_MX = Maximum value of PGM_TPR_DLT above which the purge test is

aborted.

-  PGM_TC_W = Time constant used in the rolav filtering of PGM_PCOMP_W.

|      -  PURG_ER_INIT = Value for state of ER_STATUS to request the engine running

|         purg

|      -  PURG_ER_DONE = Value for the state of  ER_STATUS  to  indicate  that  the

|         engine ru test is complete.

OTHER

-  purg_codes = Set of {P0442,P0446,P0452,P0453,P0455,P1450}.   The  set  of

OBDII fault codes that relate to purge monitor.

-  malfunction(purg,Pxxxx) = Logic process, imported from  the  MIL  control

Module; Pxxxx indicates the fault code.

-  P0442 = Fault code, unable to hold vacuum in tank (small leak).

-  P0443 = Fault code for the canister purge control circuit malfunction.

-  P0446 = Fault code, purge monitor canister vent solenoid stuck closed.

-  P0452 = Fault code, fuel tank pressure sensor circuit out of range low.

-  P0453 = Fault code, fuel tank pressure sensor circuit out of range high.

-  P0455 = Fault code, unable to pull  the  target  vacuum  in  tank  (gross

leak).

15-43

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  P1450 = Fault code, unable to bleed up the tank vacuum (gross vacuum).

15-44

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CANP_MONITOR_COM2

BEGIN: canister_purge_monitor

;Executed once every background loop.

unconditionally ---------------------| Do: pgm_init_kam_values

| Do: pgm_ram_init

|                                          | Do: pgm_er_bypass

| Do: pgm_sensor_processing

| Do: pgm_main

| Do: pgm_output_substitution

| Do: pgm_monitor_temp_bypass

| Do: pgm_check_monitored

| Do: pgm_ffg_purg

BEGIN: pgm_init_kam_values

; Executed only when called.

KAM_ERROR = 1 -----------------|

(KAM has been corrupted)      |OR --| PGM_BLED_KAM := PGM_BLD_INIT

|     |  (reset to reasonable initial value)

OBDII_RESET = 1 ---------------|     | PGM_P0_BEGK  := -64

| PGM_P2_DLTPK := -64

| PGM_P4_DELPK := -64

| PGM_P4_DLTPK := -64

|  (reset to unreasonable values)

| PGM_TEMP_CTR := 0

END: pgm_init_kam_values

BEGIN: pgm_ram_init

; Executed only when called.

RAM initialization ------------------| PGM_ST = -1.0

| PGM_ERROR = -1.0

| TRIP_NOT_PRG = 0

END: pgm_ram_init

|     BEGIN: pgm_er_bypass

|     ; bypass the KOER test logic

|     ER_STATUS = PURG_ER_INIT ---------------| ER_STATUS := PURG_ER_DONE

|      ; KOER test being desired              |  ; KOER test is complete

|     END: pgm_er_bypass

15-45

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: pgm_sensor_processing

; Executed only when called.

unconditionally ---------------------| ; call process that calculates

| ; PGM_TEMP, PGM_VBAT_DLT, PGM_LOAD_DLT

| ; logic found in CANP_PGM_FILT_COMx

| Do: pgm_filtering

|

| ; calculate change in tank pressure

| ; to determine if fuel is sloshing

| PGM_TPR_DLT := | TPR_ENG

|                  -  PGM_TANK_PRS |

| ; absolute value of difference

| ; between instantaneous and filtered

|

| ; calculate a filtered PCOMP_W

| PGM_PCOMP_W := ROLAV(PCOMP_W,PGM_TC_W)

|

| Decrement PGM_PS_TMR.

|  ; Clip to 0.0 as a minimum

PGM_TEMP <= PGM_TEMP_MAX ------|

|AND -| PGM_TMP_SEEN := 1

PGM_TEMP >= PGM_TEMP_MIN ------|     |  ; temp has been seen this key on

(temperature within range)

END: pgm_sensor_processing

BEGIN: pgm_main

; Executed only when called.

OLFLG = 0 ---------------------------| Increment CL_TMR.

(closed loop)                       |

| --- ELSE ---

|

| Freeze CL_TMR.

15-46

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PG_DC > PGM_PG_START ----------|

(duty cycle is high enough)   |

|

PGM_PCOMP_W < PGM_PPM_WMX -----|

(purge vapors not excessive)  |

|

PCT_LOAD >= PGM_LOAD_MIN ------|

(PCT_LOAD within range)       |

|

PCT_LOAD <= PGM_LOAD_MAX ------|

(PCT_LOAD within range)       |

|

PGM_TANK_PRS >= PGM_TANK_MIN --|

(tank pressure within range)  |

|

PGM_TANK_PRS <= PGM_TANK_MAX --|

(tank pressure within range)  |AND -| purg_rdy = 1

|     |  (purge test is ready)

PGM_TEMP <= PGM_TEMP_MAX ------|     |

|     |

PGM_TEMP >= PGM_TEMP_MIN ------|     |

(temp currently within range) |     |

|     |

VSBAR >= PGM_VS_MIN -----------|     |

|     |

VSBAR <= PGM_VS_MAX -----------|     |

(vehicle speed within range)  |     |

|     |

CL_TMR >= PGM_CL_TM -----------|     |

(in closed loop long enough)  |     |

|     |

PGM_CVS_FM = 0 ----------------|     |

|     |

PGM_TPR_FM = 0 ----------------|     |

(purge monitor sensors and    |     |

actuators are O.K.)          |     |

|     |

PGM_RST_TMR = 0 ---------------|     |

(delay before restart test)   |     |

|     |

P0443MON = 1 ------------------|     |

(purge valve circuit checked) |     |

|     |

P0443MALF = 0 -----------------|     |

(no circuit malfunction seen)       |

| --- ELSE ---

|

| purg_rdy = 0

purg_rdy = 1 ------------------|

|OR --| PURG_TST_RDY = 1

PGM_ST > -1 -------------------|     |

| --- ELSE ---

|

| PURG_TST_RDY = 0

15-47

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PGM_DONE = 1 ------------------------| pgm_cvs_dc := 0

; purge monitor complete            | EXIT: pgm_main

|

| --- ELSE ---

PGM_RUNNING = 0 ---------------|     |

(purge monitor test not       |     |

running)                    |     |

|     |

PGM_ST < 0 --------------------|     |

(purge monitor state less than|     |

zero; test not started)      |AND -| PGM_RUNNING = 1

|     |  (set purge monitor running flag)

purg_rdy = 1 ------------------|     | PGM_ST = 0

(test is ready)               |     |  (start purge monitor at phase zero)

|     | Do: PGM_ST_DRVR

PURG_TST_ENA = 1 --------------|     |  (call state driver subroutine)

(test enabled by OBD-2 executive)   |

| --- ELSE ---

PGM_RUNNING = 1 ---------------|     |

(purge monitor test is        |     |

running)                     |     |

|     |

PCT_LOAD < PGM_LOAD_MIN -|     |     |

(PCT_LOAD is not within |     |     |

acceptable range)      |     |     |

|     |AND -| PGM_ST = 5

PCT_LOAD > PGM_LOAD_MAX -|     |     |  (abort test; place test

(PCT_LOAD is not within |     |     |   in phase 5)

acceptable range)      |     |     | PGM_ERROR = 5

|     |     |  (indicate test was aborted due to

PGM_CVS_FM = 1 ----------|     |     |   load, vehicle speed, stall, open

|     |     |   loop, sensor/ actuator criteria,

PGM_TPR_FM = 1 ----------|     |     |   fuel sloshing)

(sensor or actuator     |     |     | Do: PGM_ST_DRVR

failed)                |OR --|     |  (call state driver subroutine)

|           |

OLFLG = 1 ---------------|           |

(in open loop)          |           |

|           |

FLG_STALL = 1 -----------|           |

(vehicle stalled)       |           |

|           |

PURG_TST_ENA = 0 --------|           |

(purge test disabled by |           |

OBD-2 executive)       |           |

|           |

VSBAR < PGM_VS_MIN ------|           |

|           |

VSBAR > PGM_VS_MAX ------|           |

(vehicle speed not in   |           |

the acceptable range)  |           |

|           |

PGM_LOAD_DLT >           |           |

PGM_LD_DT_MX ----|           |

; LOAD change too high              |

(Continued on next page)

15-48

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(Continued from previous page)

| --- ELSE ---

|

PGM_RUNNING = 1 ---------------------| Do: PGM_ST_DRVR

(purge monitor test running)        |  (call state driver subroutine)

END: pgm_main

BEGIN: pgm_output_substitution

; This process controls the Monitor Output State Control substitution:

; Executed only when called.

unconditionally ---------------------| Do: substitute(10,pgm_cvs_dc)

| PGM_CVS_DC := pgm_cvs_dc

; Close CVV/CVS during output test mode OFF command (OSM_EO_OFF = 1). This

; mode is performed with the engine off, therefore, the VMV will have no

; vacuum source to open. The evaporative system will be effectively closed.

;

;                     PGM_CVS_DC -> CVV           PG_DC -> VMV

;  OSM_EO_OFF = 1         (1.0)     (closed)      (0.0)    (closed)

;  OSM_EO_ON  = 1         (0.0)     (open)        (1.0)    (open)

;

;      NOTE: CVV is a normally open valve, the VMV, normally closed.

OSM_EO_OFF = 1 ------------------------| PGM_CVS_DC := 1.0

; NOTE: Unless set by the logic above or by logic contained within the

;       process pgm_output_substitution found in the module

;       CANP_MONITOR_COMx, PGM_CVS_DC should be zero at all times.

END: pgm_output_substitution

15-49

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: pgm_monitor_temp_bypass

; This process checks to see if all other monitors have been

; monitored and counts the number of times that the PGM_TEMP

; window has NOT been entered.

; Executed only when called.

TRIP_NOT_PRG = 0 -----------------|

|

CCM_MON = 1 ----------------|     |

|OR --|

CCM_TST_SW = 0 -------------|     |

|

SAIR_MON = 1 ---------------|     |

|OR --|

SAIR_TST_SW = 0 ------------|     |

|AND -| TRIP_NOT_PRG := 1

FUEL_MON = 1 ---------------|     |     |  ; all other monitors have

|OR --|     |  ; been monitored except

FUEL_TST_SW = 0 ------------|     |     |  ; purge monitor

|

EGO_MON = 1 ----------------|     |

|OR --|

EGO_TST_SW = 0 -------------|     |

|

EGR_MON = 1 ----------------|     |

|OR --|

EGR_TST_SW = 0 -------------|     |

|

MIS_MON = 1 ----------------|     |

|OR --|

MIS_TST_SW = 0 -------------|

15-50

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TRIP_NOT_PRG = 1  -------------|

; all monitors run            |

; except purge                |AND -| PGM_TEMP_CTR := PGM_TEMP_CTR + 1

|     |  ; increment temp counter

PGM_TMP_SEEN = 0 --------------|     |

; temp range not seen         |     | PGM_TEMP_FLG := 1

|     |  ; temp count incremented

PGM_TEMP_FLG = 0 --------------|

; count not incremented yet

END: pgm_monitor_temp_bypass

BEGIN: pgm_check_monitored

; This process checks for monitor completeness.

; Executed only when called.

PGM_DONE = 1 ------------------|

|OR --| PURG_MON := 1

PGM_TEMP_CTR > PGM_TEMP_CNT ---|     |  ; purge has been

|  ; completely monitored

| PGM_TEMP_CTR := 0

|

| --- ELSE ---

|

| PURG_MON := 0

END: pgm_check_monitored

15-51

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

END: canister_purge_monitor

BEGIN: PGM_ST_DRVR

; Executed only when called.

PGM_ST = 0 --------------------------| Do: phase_zero_test

PGM_ST = 1 --------------------------| Do: phase_one_test

PGM_ST = 2 --------------------------| Do: phase_two_test

PGM_ST = 3 --------------------------| Do: phase_three_test

PGM_ST = 4 --------------------------| Do: phase_four_test

PGM_ST = 5 --------------------------| Do: phase_five_test

END: PGM_ST_DRVR

15-52

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: phase_zero_test

PGM_PS0_SET = 0 ---------------------| PGM_PS_TMR = PGM_PS0_TM

(zero set-up not complete)          |  (load phase timer with max

|   phase zero time)

| PGM_PS0_BEGP = PGM_TANK_PRS

|  (store beginning pressure)

| PGM_P0_BEGK = PGM_PS0_BEGP

|  (load to KAM for SCP)

| PGM_PS0_SET = 1

|  (phase zero set-up is complete)

| PGM_TRG_SEEN = 0

|  (clear target vacuum seen flag)

| PGM_VAC_SEEN = 0

|  (specific vacuum seen)

PCOMP_W > PGM_P0_WPEK ---------------| PGM_P0_WPEK = PCOMP_W

(flow larger than previous)         |  (track flow and store the peak value)

PGM_TANK_PRS >= PGM_PS0_LO ----------| pgm_cvs_dc = PGM_CVS_DC + PGM_CVS_DEC

(above target range)                |  ( ramp CVS closed )

| Clip to 1.0 as maximum.

PG_DC < PGM_P0_DCMIN ----|

(duty cycle too low)    |AND -|

|     |

PGM_TRG_SEEN = 0 --------|     |OR --| PGM_ERROR = 5

(target vacuum not seen)      |     |  (set error indicating test aborted)

|     | PGM_ST = 5

PCOMP_W > PGM_P0_WMX ----------|     |  (place state monitor in phase 5)

(too much vapor)

PGM_TANK_PRS > PGM_PS0_HI -----|

(ABOVE target vacuum,         |AND -| PGM_PG_ON = 1

apply additional vacuum)     |     |  (command PCOMP to keep VMV ON)

|     | PGM_VAC_SEEN = 1

PGM_TRG_SEEN = 0 --------------|     |  (specific vacuum seen)

(not entered target range yet)      |

| --- ELSE ---

|

| PGM_PG_ON = 0

|  (command PCOMP to keep VMV OFF)

| PGM_TRG_SEEN = 1

|  (entered target range)

15-53

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PGM_PS_TMR = 0 ----------------|

(timer has cleared before     |

tank vacuum has bled up)     |

|

PGM_TANK_PRS < PGM_PS0_LO -----|AND -|   (GROSS VACUUM SEEN)

(tank vacuum has not          |     |  PGM_ERROR = 0

bled up enough)              |     |   (indicate test was aborted in

|     |    this phase)

PGM_VAC_SEEN = 0 --------------|     |  malfunction(purg,P1450)

(specific vacuum not seen)          |   (code P1450 malfunction)

|   (update MIL)

|  P1450MON = 1

|   (code P1450 monitored)

| PGM_ST = 5

|  (place state monitor in phase 5)

|

| --- ELSE ---

PGM_PS_TMR = 0 ----------------|     |

(timer has cleared before     |     |  (GROSS LEAK SEEN)

required tank pressure has   |AND -| PGM_ERROR = 0

occurred - error)            |     |  (indicate test was aborted in

|     |   this phase)

PGM_TANK_PRS > PGM_PS0_HI -----|     | malfunction(purg,P0455)

(tank pressure has not              |  (code P0455 malfunction)

dropped enough)                    |  (update MIL)

| P0455MON = 1

|  (code P0455 monitored)

| PGM_ST = 5

|  (place state monitor in phase 5)

| PGM_PG_ON = 0

|  (clear purge monitor command

|   to turn PCOMP ON)

|

| --- ELSE ---

PGM_TRG_SEEN = 1 --------------|     |

(entered target range)       |AND -| PGM_ST = 1

|     |  (continue to phase one of

PGM_CVS_DC = 1.0 --------------|     |    purge monitor)

(canister vent valve closed)  |     | P0455MON = 1

|     |  (code P0455 monitored)

PG_DC <= PG_DC_LOW ------------|     | P1450MON = 1

(purge valve closed)                |  (code P1450 monitored)

| clear_malf(P0455)

| clear_malf(P1450)

|  (clear malfunction flags)

END: phase_zero_test

15-54

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: phase_one_test

PGM_PS1_SET = 0 ---------------------| PGM_PS_TMR = PGM_PS1_TM

(phase 1 set-up not complete)       |  (load phase timer with max

|   phase one time - to stabilize

|   tank vacuum)

| PGM_PS1_SET = 1

|  (phase one set-up is complete)

PGM_PS_TMR = 0 ----------------|

(stabilization complete)      |OR --| PGM_ST = 2

|     |  (place state monitor in phase 2)

PGM_TANK_PRS >= PGM_PS1_MAX ---|

(tank pressure is high)

END: phase_one_test

BEGIN: phase_two_test

PGM_PS2_SET = 0 ---------------------| PGM_PS_TMR = PGM_PS2_TM

(phase two set-up not complete)     |  (load phase timer with max

|   phase two time)

| PGM_PS2_BEGP = PGM_TANK_PRS

|  (store starting tank pressure

|   for phase two)

| PGM_PS2_SET = 1

|  (phase two set-up is complete)

; use the peak flow value from phase 0 to load the appropriate bleed up

unconditionally ---------------------| PGM_BLD_DLT =

|          FN_PGM_BLEED(PGM_P0_WPEK)

| PGM_BLD_DLTK = PGM_BLD_DLT

|  ; store in KAM for SCP

PGM_TPR_DLT >

PGM_TP_DT_MX ----------------| PGM_ERROR = 5

; slosh event occurring             |  (set error indicating test aborted)

| PGM_ST = 5

|   (place state monitor in phase 5

|   capability to restart test)

PGM_PS_TMR = 0 ----------------------| PGM_PS2_DLTP = PGM_TANK_PRS -

(phase timer expired; calculate     |                PGM_PS2_BEGP

delta pressure in the tank)        | (calculate delta pressure)

| Clip PGM_PS2_DLTP to zero as min

|

| PGM_P2_DLTPK = PGM_PS2_DLTP

|  (load to KAM for SCP)

15-55

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PGM_PS_TMR = 0 ----------------|

(delta pressure has           |

been calculated)             |AND -| PGM_ST = 5

|     |  (jump to phase five

PGM_PS2_DLTP <= PGM_BLD_DLT ---|     |   of purge monitor)

(delta pressure small;              | PGM_ERROR = 6

vacuum held - test passed)         |  (indicate test has passed)

| PGM_0442_DLP = PGM_PS2_DLTP

|  (load for phase five check)

|

| --- ELSE ---

PGM_PS_TMR = 0 ----------------|     |

(delta pressure has           |     |

been calculated)             |AND -| PGM_ST = 3

|     |  (continue to phase three

PGM_PS2_DLTP > PGM_BLD_DLT ----|     |   of purge monitor)

(delta pressure large;

continue test)

END: phase_two_test

15-56

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: phase_three_test

PGM_PS3_SET = 0 ---------------------| PGM_PS_TMR = PGM_PS3_TM

(phase three set-up not completed)  |  (time to stabilize tank pressure

|   by opening vent valve and

|   closing VMV)

| pgm_cvs_dc = 0

|  (open CVS to atmosphere)

| PGM_PS3_SET = 1

|  (phase three set-up is complete)

|

| --- ELSE ---

PGM_TANK_PRS <= PGM_PS3_MIN ---|     |

(tank pressure is too low)    |AND -| PGM_ERROR = 5

|     |  (set error indicating test aborted)

PGM_PS_TMR = 0 ----------------|     | PGM_ST = 5

|  (place state monitor in phase 5 for

|   capability to restart test)

|

| --- ELSE ---

PGM_TANK_PRS >= PGM_PS3_MAX ---|     |

(tank pressure is too high)   |AND -| PGM_ERROR = 5

|     |  (set error indicating test aborted)

PGM_PS_TMR = 0 ----------------|     | PGM_ST = 5

|  (place state monitor in phase 5 for

|   capability to restart test)

|

| --- ELSE ---

|

PGM_PS_TMR = 0 ----------------------| PGM_ST = 4

|  (continue on to phase four of purge

|   monitor)

END: phase_three_test

15-57

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: phase_four_test

PGM_PS4_SET = 0 ---------------------| PGM_PS_TMR = PGM_PS4_TM

(phase four set-up not completed)   |  (load phase timer with

|   phase four max time)

| PGM_PS4_BEGP = PGM_TANK_PRS

|  (store beginning pressure)

| pgm_cvs_dc = 1.0

|  (set the canister vent valve

|   to 100% duty cycle)

| PGM_PS4_SET = 1

|  (phase four set-up is complete)

PGM_PS_TMR = 0 ----------------------| PGM_PS4_DELP = (PGM_TANK_PRS -

|    PGM_PS4_BEGP) * PGM_PS4_DPML

|  (calculate the delta pressure

|   and compensate for compatability

|   with phase 2)

| Clip PGM_PS4_DELP to zero as a min

|

| PGM_P4_DELPK = PGM_PS4_DELP

|  (load to KAM for SCP)

|

| PGM_PS4_DLTP = PGM_PS2_DLTP -

|                PGM_PS4_DELP

|  (calculate delta pressure not

|   caused by vapor generation)

| Clip PGM_PS4_DLTP to zero as a min

|

| PGM_P4_DLTPK = PGM_PS4_DLTP

|  (load to KAM for SCP)

15-58

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PGM_PS_TMR = 0 ----------------|

(phase four time has          |

passed)                      |AND -| PGM_ST = 5

|     |  (continue to phase five)

PGM_PS4_DLTP <= PGM_BLD_DLT ---|     | PGM_ERROR = 6

(net change in pressure is          |  (indicate test has passed)

small enough to pass test)         | PGM_0442_DLP = PGM_PS4_DLTP

|  (load for phase five check)

|

| --- ELSE ---

PGM_PS_TMR = 0 ----------------|     |

|AND -| PGM_ERROR = 5

PGM_PS4_DELP > PGM_PS4_DP -----|     |  (indicate test was aborted

(change in pressure is too          |   this phase)

large to allow test)               | PGM_ST = 5

|  (place state monitor

|    in phase 5)

|

| --- ELSE ---

PGM_PS_TMR = 0 ----------------|     |

(phase four time has          |     |

passed)                      |AND -| PGM_ST = 5

|     |  (continue to phase five)

PGM_PS4_DLTP > PGM_BLD_DLT ----|     | PGM_ERROR = 4

(change in pressure is not          |  (indicate test has failed

small enough to pass test)         |   in this phase of test)

| PGM_0442_DLP = PGM_PS4_DLTP

|  (load for phase five check)

END: phase_four_test

15-59

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: phase_five_test

unconditionally ---------------------| pgm_cvs_dc = PGM_CVS_DC - PGM_CVS_DEC

|  ; ramp CVS open

|  ; clip PGM_CVS_DC to zero as a min

pgm_cvs_dc = 0 ----------------------| PGM_ST := 6

|  ; place state monitor in non-existant

|  ; state 6; test is complete/aborted)

|

| --- ELSE ---

|

| Exit: phase_five_test

PGM_ERROR = 5 -----------------------| PGM_RST_CTR := PGM_RST_CTR + 1.0

; test was aborted                  |   ; allow test to restart

| Do: check_pgm_done

| PGM_RST_TMR := PGM_RST_TM

|   ; load timer with time before

|   ; which retry will NOT be

|   ; be allowed

|

| --- ELSE ---

|

PGM_ERROR = 4 -----------------------| PGM_RUN_CTR := PGM_RUN_CTR + 1.0

; delta pressure greater            | Do: check_for_p0442_malfunction

; than PGM_BLEED                    | Do: check_pgm_done

| PGM_RST_TMR := PGM_RST_TM

|   ; load timer with time before

|   ; which retry will NOT be

|   ; be allowed

|

| --- ELSE ---

|

PGM_ERROR = 6 -----------------------| Do: check_for_p0442_malfunction

; delta pressure smaller            | Do: check_pgm_done

; than PGM_BLD_DLT                  |

| --- ELSE ---

|

PGM_ERROR = 0 -----------------------| Do: check_pgm_done

15-60

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: check_pgm_done

; Executed only when called.

PGM_ERROR = 0 -----------------|

|

PGM_ERROR = 6 -----------------|OR --| PGM_DONE := 1

|     |  ; test is done

PGM_RUN_CTR >= PGM_RUN_CNT ----|     | PGM_RUNNING := 0

; multiple test runs          |     |  ; clear running flag

|     |

PGM_RST_CTR >= PGM_RST_CNT ----|     |

; exceeded resets limit             | --- ELSE ---

|

| Do: reset_pgm_parms

|   ; allow test to rerun

END: check_pgm_done

BEGIN: check_for_p0442_malfunction

; decide which filter constant to use based on KAM warm up counter value

KWUCTR < PGM_KWUCNT -----------------| PGM_FK := PGM_FK_NMAT

; First few warm_up cycles           |

; KAM considered not mature          |

| --- ELSE ---

|

| PGM_FK := PGM_FK_MAT

; perform the weighted average filtering

unconditionally ---------------------| pgm_bleed_lst = PGM_BLED_KAM

| PGM_BLED_KAM = PGM_FK*PGM_0442_DLP +

|            (1-PGM_FK) * pgm_bleed_lst

| P0442MON = 1

|  (code P0442 monitored)

; check filtered value against calibrated limit

PGM_BLED_KAM > PGM_BLD_DLT ----|

; above bleedup threshold     |AND -| Do: malfunction(purg,P0442)

|     |   ; set malfunction flag

PGM_RUN_CTR >= PGM_RUN_CNT ----|     |

; multiple test runs                |

| --- ELSE ---

|

| Do: clear_malf(P0442)

|   ; clear malfunction flag

END: check_for_p0442_malfunction

15-61

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: reset_pgm_parms

; Process resets selected Evaporative monitor parameters

; Executed only when called.

unconditionally ---------------------| PGM_ST = -1

| PGM_RUNNING = 0

| PGM_ERROR = -1.0

| PGM_PS0_SET = 0

| PGM_PS1_SET = 0

| PGM_PS2_SET = 0

| PGM_PS3_SET = 0

| PGM_PS4_SET = 0

| PURG_TST_RDY = 0

|  ; force test ready to be low for

|  ; one background loop; OBD-II

|  ; executive requirement

| PGM_PS_TMR = 0

| PGM_P0_WPEK = 0

END: reset_pgm_parms

END: phase_five_test

15-62

CANISTER PURGE STRATEGY, CANISTER PURGE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: pgm_ffg_purg

;  Tracks if any of a number of errors occurred with Evaporative Monitor.

;  Set if any of the possible faults currently exist. Cleared only if no

;  faults currently exist.

; Executed only when called.

P0443MALF = 1 -----------------|

; VMV circuit error           |

|

P0455MALF = 1 -----------------|

; gross leak                  |

|

P0442MALF = 1 -----------------|

; small leak                  |

|

P0446MALF = 1 -----------------|

; vent valve stuck closed     |

|OR --| FFG_PURG := 1

P0452MALF = 1 -----------------|     |  ; EVAP faults exist

; TPR circuit low error       |     |

|     |

P0453MALF = 1 -----------------|     |

; TPR circuit high error      |     |

|     |

P1450MALF = 1 -----------------|     |

; excessive vacuum in tank          |

| --- ELSE ---

P0443MALF = 0 -----------------|     |

; no VMV circuit error        |     |

|     |

P0455MALF = 0 -----------------|     |

; no gross leak               |     |

|     |

P0442MALF = 0 -----------------|     |

; no small leak               |     |

|     |

P0446MALF = 0 -----------------|     |

; vent valve not stuck closed |     |

|AND -| FFG_PURG := 0

P0452MALF = 0 -----------------|     |  ; no EVAP faults exist

; TPR circuit not low error   |     |

|     |

P0453MALF = 0 -----------------|     |

; TPR circuit not high error  |     |

|     |

P1450MALF = 0 -----------------|     |

; not excessive vac in tank         |

| --- ELSE ---

|

| No change to FFG_PURG.

END: pgm_ffg_purg

15-63

CANISTER PURGE STRATEGY, Purge Monitor Filtering - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

15.2.5  PURGE MONITOR FILTERING LOGIC (CDAL0)

OVERVIEW

This module describes the logic used by the  Purge  Monitor  test(s)  in  the

calculation and filtering of selected engine parameters.

DEFINITIONS

Registers:

-  ACT = Air Charge Temperature, deg.  F.

-  ECT = Engine coolant temperature, deg F.

-  LOAD = Universal load as a ratio of aircharge over standard.

-  PGM_LOAD = Filtered value of LOAD used in the Purge Flow test.

-  PGM_LOAD_DLT = Absolute value of  the  difference  between  the  filtered

PGM_LOAD and the current value of LOAD.

-  PGM_TEMP = Temperature (ECT/ACT combination) used in purge test.

-  PGM_VBAT = Filtered value of VBAT used in the Purge Flow test.

-  PGM_VBAT_DLT = Absolute value of  the  difference  between  the  filtered

PGM_VBAT and the current value of VBAT.

-  VBAT = Battery voltage.

Calibration Constants:

-  PGM_TC_LOAD = Time constant used with ROLAV filtering of PGM_LOAD.

-  PGM_TC_VBAT = Time constant used with ROLAV filtering of PGM_VBAT.

-  PGM_TEMP_MUL = Purge test fractional multiplier; 1.0 -> use all ECT;  0.0

-> use all ACT.

15-64

CANISTER PURGE STRATEGY, Purge Monitor Filtering - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CANP_PGM_FILT_COM1

BEGIN: pgm_filtering

; this process is executed only when called.

unconditionally ------------------------| PGM_TEMP := (PGM_TEMP_MUL * ECT) +

|          ((1 - PGM_TEMP_MUL) * ACT)

|

| ; calculate current filtered values

| PGM_VBAT := ROLAV(VBAT,PGM_TC_VBAT)

| PGM_LOAD := ROLAV(LOAD,PGM_TC_LOAD)

|

| ; calculate steady state changes

| ; absolute value of differences

| PGM_VBAT_DLT := | PGM_VBAT

|                   - VBAT   |

| PGM_LOAD_DLT := | PGM_LOAD

|                   - LOAD   |

END: pgm_filtering

15-65

CANISTER PURGE STRATEGY, CANISTER PURGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

15.3  CANISTER PURGE STRATEGY (CDAM0)

OVERVIEW

Canister Purge refers to the solenoid and valve combination that  is  located

in  the  line  between the intake manifold and the carbon canister.  When the

solenoid is energized the valve opens, allowing the flow of vapors  from  the

canister to the intake manifold.

The strategy enables canister purge during various  engine  operating  modes.

These  modes  are  calibration items.  Typical calibrations will enable purge

when these conditions are met:

1.  Fuel control is in the desired mode.  The calibrator can  choose  between

purging during closed loop only or during both open loop and closed loop.

2.  The engine has warmed up.

3.  The engine has not overheated.

4.  The 'Not at Closed Throttle' delay has been met.

The strategy includes a feature to prevent the rich surge that may  occur  on

purge  turn on.  When the purge is enabled the output is cycled on and off at

a 10Hz frequency with a variable duty cycle.  The  duty  cycle  ramps  up  to

slowly introduce the canister vapors.  The duty cycle ramp is determined from

FN600 * FN602 * FN605A * FN627.

15-66

CANISTER PURGE STRATEGY, CANISTER PURGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  ACT = Air charge temperature, deg.  F.

-  AM = Air Mass flow, lb/min.

-  ATMR1 = Time since initial engine startup, secs.

-  ATMR2 = Time since Engine Coolant Temp (ECT) reached TEMPFB, sec.

-  ECT = Engine Coolant Temperature, deg F.

-  KAMRF1 = HEGO11 adaptive fuel correction.

-  KAMRF2 = HEGO21 adaptive fuel correction.

-  LAMBSE1 = Desired equivalence ratio for HEGO11 injectors.

-  LAMBSE2 = Desired equivalence ratio for HEGO21 injectors.

-  NACTMR = Not at Closed Throttle Timer, sec.

-  PFS_CNTS = Purge flow sensor input counts.

-  PGM_CNTS = Raw purge flow sensor reading from the A/D channel, counts.

-  PGM_PG_OFF = Purge monitor command to turn Purge off.

-  PURG_ADP_SF = Adaptive learning safety factor; delta from minadp at which

time purge is disabled.

-  TCSTRT = Temperature of Engine Coolant at Cold Startup, deg F.

-  VBAT = Battery voltage.

-  VSBAR = Filtered vehicle speed.

-  ER_STATUS = State pointer that indicated current state of engine  running

on demand test.

Bit Flags:

-  ADT1FMFLG = Adaptive table 1 failure mode.

-  ADT2FMFLG = Adaptive table 2 failure mode.

-  APT = Throttle mode flag.

-  LAM_MOD_FLG = Flag used to initiate high frequency  fuel  modulation  for

the upstream EGO monitor.

-  HVQ1 = Heat valve flip flop flag; 1 -> flip flop set.

15-67

CANISTER PURGE STRATEGY, CANISTER PURGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  LEGOFG11 = Lack of HEGO11 switching flag.

-  LEGOFG21 = Lack of HEGO21 switching flag.

-  LIMIT_PURGE = Flag which indicates Purge Duty Cycle is being limited  due

to LAMBSE being clipped; 1 -> limited Purge.

-  OLFLG = Open loop flag; 1 -> open loop control, 2 -> closed loop control.

-  DFSFLG = Indicates decel fuel shutoff; 1 -> in decel fuel shutoff mode.

-  prg_flg = Purge specification variable; 1 -> conditions required to purge

have been met, 0 -> conditions required to purge have not been met.

-  prg_nact = Flag used to signal if not at closed throttle  conditions  has

existed for some amount of time.

-  PC_RESET = Purge lambse reset flag; 1 -> purge requesting lambse reset.

-  OSM_EO_ON = On state requested for OSMs during on demand test.

-  OSM_EO_OFF = Off state requested for OSMs during on demand test.

-  VSPRGFLG = Vehicle speed purge enable flag.

Calibration Constants:

-  CTHIGH = Temperature of Engine Coolant (ECT) at Hot Startup, deg F.

-  CTLOW = Temperature of Engine Coolant at Cold Startup, deg F.

-  CTPRG = Overheat temperature to turn off purge, deg F.

-  CTPRG_SL = CTPRG - hysteresis.

-  EVTDOT = Purge time delay at Part throttle or WOT.

-  FN600(PRGTMR) = Canister Purge Duty Cycle Multiplier

-  FN602(CPRGTMR) = Canister Purge Duty Cycle Multiplier

-  FN603(ACT)  =  Maximum  LAMBSE  allowed  before  purge  duty   cycle   is

decremented.

-  FN605A(AM) = Canister Purge Duty Cycle vs.  AM

-  FN627(PRGIDLE_TMR)  =  Purge  duty  cycle  modifier  as  a  function   of

PRGIDLE_TMR.

-  MINADP = Minimum allowable correction.

-  PRG_DEC = PG_DC decrement value used by fuel system failing logic.

-  PRGMPH_CL = PRGMPH_SH minus hysteresis.

-  PRGMPH_SH = Minimum vehicle speed to allow purge.

15-68

CANISTER PURGE STRATEGY, CANISTER PURGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PRGTD1 = Canister purge cold startup delay time.

-  PRGTD2 = Canister purge medium temp startup delay time.

-  PRGTD3 = Canister purge hot startup delay time.

-  PRGTD4 = Canister purge cold startup delay time, used with ATRM2.

-  PRGTD5 = Canister purge delay timer, used with ATMR2.

-  PURGSW = Switch to select Open Loop Purge; 1 -> Allow Open Loop Purge.

-  PFS_H_ER = Purge flow sensor counts value below which to allow purge.

-  PFS_L_ER = Purge flow sensor counts value above which to allow purge.

-  PGM_DEC = Value decremented from PG_DC when purge is being commanded  off

in phase one of the purge flow test.

-  VBAT_MX = Maximum battery voltage below which to allow purging.

-  EGO_ER_INIT = Value for state of ER_STATUS to indicate engine running EGO

test is to be run.

-  PRG_LAM_SW = Calibration switch to enable purge induced lambse resets.

-  PRG_LAM_SUB = Calibration constant used as a  subtractor  to  LAMBSEx  in

FN603 logic check.

OUTPUTS

Registers:

-  PRGTMR = Total Purge on time.

-  CPRGTMR = Current Purge on time.

-  PG_DC = Canister Purge Duty Cycle.

-  PRGIDLE_TMR = Time at idle for purge control, sec.

Bit Flags:

-  HVQ1 = See above.

-  LIMIT_PURGE = See above.

-  prg_flg = See above.

-  PRGFLG = Timer control flag for PRGTMR and CPRGTMR.

-  VSPRGFLG = See above.

15-69

CANISTER PURGE STRATEGY, CANISTER PURGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CANP_COM12

APT = -1 -------------------------------| INCREMENT PRGIDLE_TMR.

| Clip to 255 seconds as a maximum.

|

| --- ELSE ---

|

| DECREMENT PRGIDLE_TMR.

| Clip to zero as a minimum.

ECT < CTPRG_SL -------------------------|S Q -| HVQ1

|

ECT > CTPRG ----------------------------|C

(Over temp. disable)

VSBAR >= PRGMPH_SH ---------------------|S Q -| VSPRGFLG

|

VSBAR < PRGMPH_CL ----------------------|C

(vehicle speed disable)

NACTMR >= EVTDOT -----------------------| prg_nact = 1

|

| --- ELSE ---

|

| prg_nact = 0

15-70

CANISTER PURGE STRATEGY, CANISTER PURGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PGM_PG_OFF = 1 -------------------------------| prg_flg = 0

(purge flow test commanding purge off)       |  (disable purge)

(prepare to ramp purge duty cycle)           | purgdc = PG_DC

(load previous duty cycle value)             | pg_dc = PG_DC

|

| --- ELSE ---

HVQ1 = 1 -------------------------------|     |

|     |

TCSTRT >= CTHIGH -----------|           |     |

|AND -|     |     |

ATMR1 >= PRGTD3 ------------|     |     |     |

|     |     |

CTLOW < TCSTRT < CTHIGH ----|     |     |     |

|AND -|OR --|     |

ATMR1 >= PRGTD2 ------|     |     |     |     |

|AND -|     |     |     |

ATMR2 >= PRGTD5 ------|           |     |     |

|     |     |

TCSTRT <= CTLOW ------------|     |     |     |

|AND -|     |     |

ATMR1 >= PRGTD1 ------|     |           |AND -| prg_flg = 1

|AND -|           |     |  (enable purge)

ATMR2 >= PRGTD4 ------|                 |     | purgdc =

|     |  FN600(PRGTMR) *

prg_nact = 1 ---------------------------|     |  FN602(CPRGTMR) *

|     |  FN605A(AM) *

OLFLG = 0 ------------------------|     |     |  FN627(PRGIDLE_TMR)

(closed loop)                    |OR --|     | pg_dc = PG_DC

|     |     | PC_RESET = 0

PURGSW = 1 -----------------------|     |     |

(to allow Open Loop Purge)             |     |

(continued on next page)

15-71

CANISTER PURGE STRATEGY, CANISTER PURGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|     |

LAM_MOD_FLG = 0 ------------------------|     |

(EGO monitor not running)              |     |

|     |

PFS_CNTS > PFS_L_ER --------------------|     |

|     |

PFS_CNTS < PFS_H_ER --------------------|     |

(flow sensor is O.K.)                  |     |

|     |

VBAT < VBAT_MX -------------------------|     |

(battery voltage acceptable)           |     |

|     |

DFSFLG = 0 -----------------------------|     |

(NOT in decel fuel shut off)           |     |

|     |

VSPRGFLG = 1 ---------------------------|     |

|     |

ER_STATUS <> EGO_ER_INIT ---------------|     |

(KOER EGO test is NOT active)                |

| --- ELSE ---

prg_nact = 0 ---------------------------|     |

|     |

PRGFLG = 1 -----------------------------|AND -| PC_RESET = 1

|     | prg_flg = 0

PC_RESET = 0 ---------------------------|     | purgdc = 0

(lambse reset has not occurred)        |     |

|     |

PRG_LAM_SW = 1 -------------------------|     |

(calibration switch to allow reset)    |     |

|     |

LAMBSE1 > FN603(ACT)-PRG_LAM_SUB -|     |     |

|OR --|     |

LAMBSE2 > FN603(ACT)-PRG_LAM_SUB -|           |

| --- ELSE ---

|

| PC_RESET = 0

| prg_flg = 0

|  (disable purge)

| purgdc = 0

| pg_dc = 0

PURGE DUTY CYCLE DECREMENT/INCREMENT LOGIC

LIMIT_PURGE = 0 ------------------------------| pg_dc = purgdc

|

| --- ELSE ---

|

| Continue to Increment/

|  Decrement with value

|  of pg_dc.

15-72

CANISTER PURGE STRATEGY, CANISTER PURGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The following logic allows the reduction of the purge duty cycle (PG_DC) when

conditions indicate that the fuel system may be failing.  The purge is backed

out to prevent  the  continuous  HEGO  TESTS  and  adaptive  fuel  test  from

indicating  failure due to purge overload.  Once a failure has been indicated

or control has been restored for one of the four areas indicated by the logic

show   below  purging  is  resumed.   If  subsequently  another  area  begins

indicating failure purging will again be backed  out  until  the  appropriate

failure is verified or until the system comes back into full control.

The purge duty cycle can also be reduced if the Purge test commands  to  turn

off purge, decrementing the duty cycle by PGM_DEC each pass.

PGM_PG_OFF = 1 -------------------------------------| pg_dc = pg_dc -

|         PGM_DEC

| Clip to zero as

| a minimum.

| Clip to purgdc as

| a maximum.

|

| --- ELSE ---

LAMBSE1 => FN603(ACT) ------------------|           |

(HEGO11 failure condition present)     |AND -|     |

|     |     |

LEGOFG11 = 0 ---------------------------|     |     |

(HEGO11 failure recognition not              |     |

latched)                                    |     |

|     |

LAMBSE2 => FN603(ACT) ------------------|     |     |

(HEGO21 failure condition present)     |AND -|     |

|     |     |

LEGOFG21 = 0 ---------------------------|     |     |

(HEGO21 failure recognition not              |     |

latched)                                    |OR --| pg_dc = pg_dc -

|     |   PRG_DEC

KAMRF1 <= MINADP + 0.5 +                      |     | Clip to zero as a

PURG_ADP_SF ---------|     |     |  minimum

(bank #1 adaptive limit imminent)      |AND -|     | Clip to purgdc as a

|     |     |  maximum

ADT1FMFLG = 0 --------------------------|     |     |

(adaptive failure not yet recognized)        |     |

|     |

KAMRF2 <= MINADP + 0.5 +                      |     |

PURG_ADP_SF ---------|     |     |

(bank #2 adaptive limit imminent)      |AND -|     |

|           |

ADT2FMFLG = 0 --------------------------|           |

(adaptive failure not yet recognized)              |

| --- ELSE ---

|

| pg_dc = pg_dc +

|           PRG_DEC

| Clip to purgdc as a

|  maximum

15-73

CANISTER PURGE STRATEGY, CANISTER PURGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

purgdc > pg_dc -------------------------------------| LIMIT_PURGE = 1

|  (purge is being

|   limited due to LAMBSE

|   being at its clip or

|   purge test is ramping

|   the duty cycle down)

|

| --- ELSE ---

|

| LIMIT_PURGE = 0

ON DEMAND TEST PG_DC OVERRIDE

OSM_EO_ON = 1 --------------------------------------| pg_dc = 1.0

OSM_EO_OFF = 1 -------------------------------------| pg_dc = 0

unconditionally ------------------------------------| Do: substitute(9,pg_dc)

|   (output state control

|    override)

| PG_DC := pg_dc

NOTES;

1.  When purge is enabled, the purge output is cycled at 10Hz

+/- 30%

2.  Set EVTDOT to 0 to Purge at Closed Throttle.

TIMER LOGIC

prg_flg = 1 ----------------------------------|

|AND -| PRGFLG = 1

FN605A(AM) <> 0 ------------------------------|     |  (count up PRGTMR)

|  (count up CPRGTMR)

|

| --- ELSE ---

|

| CPRGTMR = 0

| PRGFLG = 0

|  (clear PRGFLG)

|  (freeze PRGTMR)

|  (reset and freeze

|        CPRGTMR)

15-74

CANISTER PURGE STRATEGY, PURGE FLOW SENSOR INPUT AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

15.4  PURGE FLOW SENSOR INPUT AND SELF TEST (CDAJ0)

OVERVIEW

This module reads the purge flow sensor input.  The A/D is read and  the  raw

counts  (PFS_CNTS) are tested for "out of range" or other failure conditions.

If a failure is present for a sufficient  amount  of  time,  the  appropriate

malfunction  flag  (PxxxMALF)  is set.  Finally, a timer is checked to see if

the component has been sufficiently monitored for this "trip".

DEFINITIONS

INPUTS

Registers:

-  PFS_CNTS = Raw A/D counts from sensor.

-  PFS_ER_TMR = Timer that runs when a fault is present.

-  PFS_FM_TMR = Timer that is reset when the  failure  conditions  for  FMEM

mode are met, seconds.

-  PUTMR = Time since power up, seconds.

Bit Flags:

-  PURG_TST_ENA = OBDII purge test enable flag ( 1 -> test enabled ).

-  CCM_TST_ENA = OBDII Comprehesive Component test ( CCM ) enable flag.

-  CCM_ER_ENA = On demand test for CCM enable flag.

-  PxxxMALF = OBDII malfunction flag for  fault  xxx;  1  ->  a  malfunction

currently exists for fault xxx.

-  PxxxMON = OBDII monitor flag for fault xxx;  1  ->  fault  xxx  has  been

monitored at least once since power-up.

Calibration Constants:

-  TC_PFS = Time constant for purge flow sensor.

-  PFS_ER_TM = Time with failure present required to set code, seconds.

-  PFS_FM_TM = Minimum time in FMEM mode once initiated, seconds.

-  PFS_H_ER = Maximum acceptable value for PFS, counts.

-  PFS_L_ER = Minimum acceptable value for PFS, counts.

-  PFS_MON_TM = Time required to indicate that a "trip" is complete for this

sensor, seconds.

15-75

CANISTER PURGE STRATEGY, PURGE FLOW SENSOR INPUT AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OUTPUTS

Registers:

-  PGM_PFS_FM = Purge thermistor flow sensor FMEM flag.

-  PFS_ER_TMR = See above.

-  PFS_FM_TMR = See above.

-  PGM_PFS = Output counts value used by rest of strategy.

Bit Flags:

-  PxxxMALF = See above.

-  PxxxMON = See above.

OTHER

-  purg_codes = Set of {P0442,P0446,P1444,P1445,P0452,P0453,P1442}.  The set

of OBDII fault codes that relate to purge monitor.

-  malfunction(purg,Pxxx) = Logic process, imported  from  the  MIL  control

Module; Pxxx indicates the fault code.

-  P1445 = Fault code, Purge Flow Sensor input failed high.

-  P1444 = Fault code, Purge Flow Sensor input failed low.

15-76

CANISTER PURGE STRATEGY, PURGE FLOW SENSOR INPUT AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CANP_INPUT_PFS_COM1

always -------------------------| Increment PFS_ER_TMR

| Increment PFS_FM_TMR

PURG_TST_ENA = 1 ---------|

|

CCM_ER_ENA = 1 -----------|OR --| Do: PFS_FAILURE_CHECK

|     |  (check for out of range)

CCM_TST_ENA = 1 ----------|     | Do: PFS_MONITORED_CHECK

|  (test for component monitored)

|

| --- ELSE ---

|

| Exit module.

15-77

CANISTER PURGE STRATEGY, PURGE FLOW SENSOR INPUT AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: PFS_FAILURE_CHECK

Check for  "open/short"  failures  and  other  failures  as  appropriate  for

particular  sensor.   Perform  FMEM  action  if  a  failure is detected.  Set

malfunction flag when a malfunction has been present for a sufficient time.

PFS_CNTS > PFS_H_ER ------|

(value too high)         |

|

PFS_ER_TMR > PFS_ER_TM ---|AND -| malfunction(purg,P1445)

(failure present long    |     |  (code P1445 malfunction)

enough)                 |     |  (update MIL)

|     |

P1445MALF = 0 ------------|     | PFS_FM_TMR = 0

| PGM_PFS_FM = 1

|  (set purge flow sensor FMEM flag)

|

| --- ELSE ---

|

PFS_CNTS > PFS_H_ER ------------| PGM_PFS_FM = 1

(value too high; not present   |  (set purge flow sensor FMEM flag)

long enough to set            | PFS_FM_TMR = 0

malfunction flag)             |

| --- ELSE ---

PFS_CNTS < PFS_L_ER ------|     |

(value too low)          |     |

|     |

PFS_ER_TMR > PFS_ER_TM ---|AND -| malfunction(purg,P1444)

(failure present long    |     |  (code P1444 malfunction)

enough)                 |     |  (update MIL)

|     |

P1444MALF = 0 ------------|     | PFS_FM_TMR = 0

| PGM_PFS_FM = 1

|  (set purge flow FMEM flag)

|

| --- ELSE ---

|

PFS_CNTS < PFS_L_ER ------------| PGM_PFS_FM = 1

(value too low; not present    |  (set purge flow sensor FMEM flag)

long enough to set            | PFS_FM_TMR = 0

malfunction flag)             |

| --- ELSE ---

(continued on next page)

15-78

CANISTER PURGE STRATEGY, PURGE FLOW SENSOR INPUT AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

P1445MALF = 1 ------|           |

|OR --|     |

P1444MALF = 1 ------|     |AND -| PGM_PFS_FM = 1

|     |  (Remain in FMEM mode until

PFS_FM_TMR < PFS_FM_TM ---|     |   it times out)

|

| --- ELSE ---

|

| PGM_PFS =  ROLAV(PFS_CNTS,TC_PFS)

|  (no failure detected, return

|   value for normal use)

| PFS_ER_TMR = 0

|  (reset failure present timer)

| PGM_PFS_FM = 0

|  (clear failure flags)

| Do: clear_malf(P1444)

| Do: clear_malf(P1445)

END: PFS_FAILURE_CHECK

BEGIN: PFS_MONITORED_CHECK

If enough time has passed since power up (or run mode, or...) assume that

enough A/D readings have been taken to assume that a hard fault, if present,

would have been detected.

PUTMR > PFS_MON_TM -------------| P1445MON = 1

(enough time for sufficient    | P1444MON = 1

A/D readings)                 |  (codes P1445 & P1444 monitored)

END:  PFS_MONITORED_CHECK

15-79

CANISTER PURGE STRATEGY, CANISTER PURGE SOLENOID OSM TEST LOGIC- CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

15.5  CANISTER PURGE SOLENOID FAULT TEST LOGIC (CDAJ0)

OVERVIEW

By using the Output  Driver  Feedback  circuitry  (CANP_FAULT),  this  module

monitors  the  control  circuitry  of  the  canister  purge  solenoid (CANP).

CANP_FAULT operates as such:

CANP_FAULT = 0 (No fault detected - valve is functioning properly)

CANP_FAULT = 1 (Fault detected - valve is NOT functioning properly)

The CANP is a duty cycled solenoid, therefore to test it correctly, this test

must  be  performed when the CANP is competely ON (100% dc) or completely OFF

(0% dc).  Otherwise, erroneous readings and a possible false fault code could

result.

Therefore, PG_DC must be >= to some Max duty cycle value or = 0 for the fault

logic  to consider checking for a true fault.  If not, the test will wait for

these conditions to exist before fully running the test.

DEFINITIONS

INPUTS

Registers:

-  CANP_FF = canister purge fault filter.

-  CANP_OSM_TMR = Timer that tracks the amount of time that the purge  valve

has been actively monitored.

-  PG_DC = CANP duty cycle; 1.0 -> 100%, 0 -> 0%.

-  VBAT = Battery voltage.

Bit Flags:

-  CANP_FAULT = Fault status of canister purge valve  (CANP)  returned  from

the driver; 0 -> no fault detected, , 1 -> a fault has been detected.

-  CCM_EO_ENA = Engine off on demand test for CCM enabled.

-  CCM_ER_ENA = Engine running on demand test for CCM enabled.

-  CCM_TST_ENA = OBDII Comprehensive Component Test Enable Flag.

-  OSM_EO_OFF = OFF state requested for outputs during engine off on  demand

test.

-  OSM_EO_ON = ON state requested for outputs during engine  off  on  demand

test.

-  OSM_1ST_PS = First pass flag of the OSM test; 1 -> first pass seen.

15-80

CANISTER PURGE STRATEGY, CANISTER PURGE SOLENOID OSM TEST LOGIC- CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PURG_TST_ENA = OBDII purge test enable flag; 1 -> test enabled.

Calibration Constants:

-  CANP_OSM_TM = Time above which the purge valve is  considered  completely

monitored.

-  CANP_THRES = Threshold  used  with  the  ROLAV  filtering  check  of  the

CANP_OSM.

-  PG_DC_MAX = Duty cycle of the purge valve above which it is considered to

be at its max ON value.

-  TC_CANP = Time  constant  used  in  the  ROLAV  filtering  check  of  the

CANP_OSM.

-  VBAT_CCM_MIN = Minimum voltage to perform CCM tests.

OUTPUTS

Registers:

-  CANP_FF = See above.

-  CANP_OSM_TMR = See above.

-  PG_DC = See above.

Bit Flags:

-  CANP_FAULT = See above.

-  OSM_1ST_PS = See above.

-  P0443MON = OBDII monitor flag for fault 0443.

OTHER

-  purg_codes = Set  of  (P0442,P0443,P0446,P0452,P0453,P1442)  OBDII  fault

codes.

-  malfunction(purg,Pxxx) = Logic process, imported  from  the  MIL  control

Module.

-  clear_malf(Pxxx) = Logic process, imported from the MIL control Module.

-  P0443 = Fault code for the canister purge control circuit malfunction.

15-81

CANISTER PURGE STRATEGY, CANISTER PURGE SOLENOID OSM TEST LOGIC- CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CANP_OSM_TEST_COM2

BEGIN: canp_fault_check  ;done once per background loop

OSM_EO_ON = 1 --------------|

|

OSM_EO_OFF = 1  ------------|

|

CCM_EO_ENA = 1  ------------|OR --|

|     |

CCM_ER_ENA = 1  ------------|     |

|     |

PURG_TST_ENA = 1 -----------|     |AND -| Do: canp_fault_test

|     |     |

CCM_TST_ENA = 1 ------------|     |     |

|     |

VBAT >= VBAT_CCM_MIN -------------|     |

| --- ELSE ---

|

| Exit canp_fault_check

|  ;Exit module.

END: canp_fault_check

15-82

CANISTER PURGE STRATEGY, CANISTER PURGE SOLENOID OSM TEST LOGIC- CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: canp_fault_test

OSM_1ST_PS = 0 -------------------------| CANP_OSM_TMR = 0

;first time through                    |  ;clear timer

| OSM_1ST_PS = 1

;This routine checks the output driver fault status and waits until the

;proper PG_DC conditions before deciding whether a fault exists or not.

;If the proper PG_DC conditions are not met, the test will wait until

;they exist to run the test, holding the values of the fault filter and

;the fault present timer.

PG_DC >= PG_DC_MAX  --------|

;CANP on                   |OR --|AND -| CANP_FF = ROLAV(1,TC_CANP)

|     |     |  ;error seen, filter to 1

PG_DC = 0 ------------------|     |     |

;CANP off                        |     | Increment CANP_OSM_TMR

|     |

CANP_FAULT = 1 -------------------|     |

;fault detected                        |

| --- ELSE ---

PG_DC >= PG_DC_MAX  --------|           |

;CANP on                   |OR --|AND -| CANP_FF = ROLAV(0,TC_CANP)

|     |     |  ;no error seen, filter to 0

PG_DC = 0 ------------------|     |     |

;CANP off                        |     | Increment CANP_OSM_TMR

|     |

CANP_FAULT = 0 -------------------|     |

;fault not detected                    |

| --- ELSE ---

|

| No change to CANP_FF.

| Halt CANP_OSM_TMR.

|  ;PG_DC <> 1 or 0 - cannot test

CANP_FF > CANP_THRES -------------------| Do: malfunction(purg,P0443)

;apply threshold check to decide if    |

;error has been present long enough    |

| --- ELSE ---

|

| Do: clear_malf(P0443)

CANP_OSM_TMR >= CANP_OSM_TM ------------| P0443MON = 1

;check if enough time has passed       |

;to say if completely monitored        |

| --- ELSE ---

|

| P0443MON = 0

END: canp_fault_test

15-83

CANISTER PURGE STRATEGY, CANISTER PURGE OBDII TEST SELECT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

15.6  CANISTER PURGE OBDII TEST SELECT (CDAJ0)

OVERVIEW

This module selects one of the 3 OBDII purge test strategies  to  be  active,

based on the value of the calibration parameter PGM_SELECT.

CANP_THERM_FLOW_COMx is the system that uses the purge thermistor flow sensor

(PFS) and the ON/OFF purge valve.

CANP_VMV_FLOW_COMx is the system that uses PCOMP, the VMV,  and  which  tests

the  effect  of the PCOMP strategy on the Idle Speed Control to infer flow in

the Evaporative purge system.

CANP_MONITOR_COMx is the full RUNNING LOSS compliant system that uses  PCOMP,

a  tank  pressure  sensor  (TPR),  the  VMV  purge valve, and a canister vent

solenoid (CVS) on the atmospheric side of the canister.

DEFINITIONS

INPUTS

Calibration Constants:

-  PGM_SELECT = Purge test  strategy  select  switch;  0  ->  execute  Evap.

Monitor, 1 -> execute thermistor (PFS) test, 2 -> execute VMV flow test.

PROCESS

STRATEGY MODULE: CANP_PGM_SELECT_COM3

PGM_SELECT = 1 -------------------------| Do: CANP_INPUT_PFS_COMx

;thermistor flow test selected         | Do: CANP_THERM_FLOW_COMx

|

| --- ELSE ---

|

PGM_SELECT = 2 -------------------------| Do: CANP_VMV_FLOW_COMx

;VMV flow test selected                |

|

| --- ELSE ---

|

;Evap System Monitor by default        | Do: CANP_INPUT_TPR_COMx

| Do: CANP_CVS_FMEM_COMx

| Do: CANP_MONITOR_COMx

| Do: canister_vent_valve_output

15-84

CANISTER PURGE STRATEGY, CANISTER PURGE SELECT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

15.7  CANISTER PURGE SELECT (CDAJ0)

OVERVIEW

This  strategy  selects  between  the  standard  purge   strategy   and   the

PCOMP/RUNNING LOSS strategy.

DEFINITIONS

INPUTS Calibration Constants:

-  PCOMP_SW = PCOMP strategy select switch; 1 -> execute PCOMP strategy.

-  PG_LK_ML = Purge valve leakage multiplier, lbs/min.

OUTPUTS

Bit Flags:

-  PCOMP_ENA = PCOMP strategy enabled flag; 1 -> PCOMP is enabled;  adaptive

fuel disabled.

Registers:

-  PCOMP_LBM = Purge compensation, lbs mass/cylinder.

CALIBRATION HINTS

When PCOMP_SW=0, set PG_LK_ML = 0

PROCESS

STRATEGY MODULE: CANP_SELECT_COM2

PCOMP_SW = 1 -----------------| Do: VMV OSM test logic.

|     (CANP_VMV_OSM_COM*)

| Do: CANISTER PURGE STRATEGY - PCOMP

|     (CANP_PCOMP_ENABLE_COM*, CANP_PCOMP_COM*)

| Do: vapor_management_valve_output

|

| --- ELSE ---

|

| Do: ON/OFF Purge valve test logic.

|     (CANP_OSM_TEST_COM*)

| Do: CANISTER PURGE STRATEGY - STANDARD

|     (CANP_COM*)

| Do: canister_purge_output

|

| ;PCOMP is NOT active, therefore

| PCOMP_ENA = 0

| PCOMP_LBM = 0

15-85

CANISTER PURGE STRATEGY, CANISTER PURGE FLOW TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

15.8  CANISTER PURGE FLOW TEST (CDAN2)

OVERVIEW

This logic describes the thermistor based flow test  of  the  canister  purge

system  for compliance with CARB and EPA OBD-II requirements.  The purpose of

the test is to determine if flow is properly occuring in the Evaporative Fuel

System.

o  Entry Conditions:

The test will begin if all of the following entry conditions are met:

.  The test has not yet been run this trip.

.  Powertrain load is within the calibrated window.

.  The vehicle speed is within a calibrated window.

.  ACT/ECT is below a calibrated maximum value.

.  Time since closed loop is greater than a calibrated minimum value.

.  PG_DC is above a calibrated value.

.  Inferred manifold vacuum is above a calibrated minimum value.

|          .  Relative throttle position is above a calibrated minimum value.

o  Test Description:

A thermistor in-line with the purge line is  used  to  gauge  flow.   The

thermistor's  voltage  value  is  read  by the processor at two different

times - once when the purge valve is open and manifold vacuum is  purging

down  the  lines  and  canister  - and once when the purge valve has been

turned off.  These values are subtracted to  get  a  delta  and  this  is

compared with a calibratable threshold.  If the flow is occuring properly

in the system, the change in the thermistor's  value  should  be  greater

than the threshold.  If not, an error exists.

DEFINITIONS

Registers:

-  ACT = Air Charge Temperature, deg.  F.

-  CL_TMR = Closed loop timer (actuated).

-  PGM_RDY_TMR = Timer that tracks conditions to  determine  if  purge  flow

test is ready.

-  ECT = Engine coolant temperature, deg F.

-  LOAD = Universal load as a ratio of aircharge over standard.

-  PGM_FLOW_1 = First value of PFS when PURGE is ON; counts.

-  PGM_FLOW_2 = Second value of PFS when PURGE is OFF; counts.

15-86

CANISTER PURGE STRATEGY, CANISTER PURGE FLOW TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PGM_RST_CTR = Purge test restart counter.

-  PGM_TEMP = Temperature (ECT/ACT combination) used in purge test.

-  VSBAR = Vehicle speed, MPH.

-  PGM_RST_TMR = Timer that counts down time between  attempts  to  run  the

purge test after a reset.

-  PG_DC = Purge duty cycle.

-  PGM_PFS = Value of Purge flow sensor; counts.

-  INF_MVAC = Inferred Manifold Vacuum; inches Hg.

-  PGM_TMR = Timer that increments between readings of the PFS.

-  PGM_DLT_FLOW = Difference between 2 readings of the  purge  flow  sensor,

counts.

|      -  TP_REL = Relative TP (TP - RATCH).

|      -  ER_STATUS = State pointer that indicates current state of engine  running

|         on demand test.

Bit Flags:

-  CCM_TST_ENA = OBD-II Comprehensive Component Test  enable  flag.   (1  =>

test enabled)

-  FLG_STALL = Indicates a stall has occurred; transition.

-  FLOW_RUNNING = Purge test is running flag; 1 -> running.

-  OLFLG = Open loop flag; 1 -> open loop mode, 0 -> closed loop mode.

-  PGM_1ST_PS = First pass through the PGM_FLOW_TEST flag when a reading  is

taken; 1 -> first pass seen.

-  PGM_2ND_PS = Second pass through the PGM_FLOW_TEST flag when a reading is

taken; 1 -> second pass seen.

-  PGM_DONE = Purge test is done flag; 1 -> test is done.

-  PGM_PFS_FM = Purge thermistor flow sensor FMEM flag.

-  PGM_PG_OFF = Flag to turn PURGE off during a portion of the purge test.

-  PURG_MON = OBDII test flag for evaporative purge; 1 -> all  purge  faults

been monitored since power-up.

-  PURG_TST_ENA = Purge test enabled flag; 1 -> Purge test has been  enabled

by the OBD-2 executive.

-  PURG_TST_RDY = Purge test ready flag; 1 -> all local  conditions  to  run

the purge test have been met.

15-87

CANISTER PURGE STRATEGY, CANISTER PURGE FLOW TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PxxxMALF = OBDII malfunction flag for  fault  xxx;  1  ->  a  malfunction

currently exists for fault xxx.

-  PxxxMON = OBDII test flag for fault xxx; 1 -> fault has been monitored at

least once since power-up.

Calibration Constants:

-  PGM_CL_TM = Time in closed loop to allow purge test.

-  PGM_LOAD_MAX = Maximum load to allow purge test.

-  PGM_LOAD_MIN = Minimum load to allow purge test.

-  PGM_RST_CNT = Number of times purge test  is  allowed  to  be  tried  per

engine key-on.

-  PGM_TEMP_MAX = Maximum temperature to allow purge test.

-  PGM_TEMP_MIN = Minimum temperature to allow purge test.

-  PGM_TEMP_MUL = Purge test fractional multiplier; 1.0 -> use all ECT;  0.0

-> use all ACT.

-  PGM_VS_MAX = Maximum vehicle speed to allow purge test.

-  PGM_VS_MIN = Minimum vehicle speed to allow purge test.

-  PGM_RST_TM = Time required before Purge Test is allowed to restart.

-  PGM_PG_START = Purge duty cycle above which to allow the  purge  test  to

begin.

-  PGM_PG_LO = Purge duty cycle below which to load in a value of the PFS.

-  PGM_TM = Time to delay between readings of the PFS.

-  PGM_DLT_THRS = Threshold value of PGM_DLT_FLOW below which  an  error  is

assumed.

-  PGM_MVAC_MIN = Value of INF_MVAC above which to run the purge flow test.

-  PGM_RDY_TM = Time to say that  conditions  for  test  have  existed  long

enough.

|      -  PGM_TP_MIN = Minimum value of TP_REL to begin the purge flow test.

|      -  PURG_ER_INIT = Value for state of ER_STATUS to request the engine running

|         purge test.

|      -  PURG_ER_DONE = Value for the state of  ER_STATUS  to  indicate  that  the

|         engine running purge test is complete.

15-88

CANISTER PURGE STRATEGY, CANISTER PURGE FLOW TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OTHER

-  purg_codes = Set of {P0442, P0443, P0446,  P0452,  P0453,  P1442,  P1443,

P1444, P1445}.  The set of OBDII fault codes that relate to purge test.

-  malfunction(purg,Pxxx) = Logic process, imported  from  the  MIL  control

Module; Pxxx indicates the fault code.

-  P1443 = Fault  code,  evaporative  emission  control  system  purge  flow

malfunction.

15-89

CANISTER PURGE STRATEGY, CANISTER PURGE FLOW TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CANP_THERM_FLOW_COM1

|     ; bypass the KOER test logic

|     ER_STATUS = PURG_ER_INIT ------------| ER_STATUS := PURG_ER_DONE

|      ; KOER test being desired           |  ; KOER test is complete

always ------------------------------| PGM_TEMP = (PGM_TEMP_MUL * ECT) +

|          ((1 - PGM_TEMP_MUL) * ACT)

OLFLG = 0 ---------------------------| Increment CL_TMR.

(closed loop)                       |

| --- ELSE ---

|

| Freeze CL_TMR.

PG_DC >= PGM_PG_START ---------|

(duty cycle is high enough)   |

|

LOAD <= PGM_LOAD_MAX ----------|AND -| Increment PGM_RDY_TMR

(load within range)           |     |

|     |

INF_MVAC >= PGM_MVAC_MIN ------|     |

(manifold vacuum above              |

calibratable limit)                |

| --- ELSE ---

|

| PGM_RDY_TMR = 0

15-90

CANISTER PURGE STRATEGY, CANISTER PURGE FLOW TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLOW_RUNNING = 1 ------------------|

|

PGM_PFS_FM = 0 --------------|     |

(flow sensor O.K.)          |     |

|     |

PGM_TEMP <= PGM_TEMP_MAX ----|     |

|     |

PGM_TEMP >= PGM_TEMP_MIN ----|     |

(temperature is within      |     |OR --| PURG_TST_RDY = 1

acceptable range)          |     |     |  (purge test is ready)

|     |     |

VSBAR >= PGM_VS_MIN ---------|     |     |

|     |     |

VSBAR <= PGM_VS_MAX ---------|     |     |

(vehicle speed is within    |     |     |

acceptable range)          |     |     |

|     |     |

CL_TMR >= PGM_CL_TM ---------|AND -|     |

(in closed loop long enough)|           |

|           |

P0443MON = 1 ----------------|           |

(Purge valve checked)       |           |

|           |

P0443MALF = 0 ---------------|           |

(Purge valve O.K.)          |           |

|           |

PGM_RST_TMR = 0 -------------|           |

(delay before restart test) |           |

|           |

PGM_RST_CTR < PGM_RST_CNT ---|           |

(below max reset tries)     |           |

|           |

PGM_RDY_TMR >= PGM_RDY_TM ---|           |

|      (conditions for test have   |           |

|       existed long enough)       |           |

|                                  |           |

|     TP_REL > PGM_TP_MIN ---------|           |

| --- ELSE ---

|

| PURG_TST_RDY = 0

15-91

CANISTER PURGE STRATEGY, CANISTER PURGE FLOW TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PGM_DONE = 1 ------------------------| EXIT

(purge test complete)               |

| --- ELSE ---

FLOW_RUNNING = 0 --------------|     |

(purge test not running)      |     |

|AND -| FLOW_RUNNING = 1

PURG_TST_RDY = 1 --------------|     |  (set purge test running flag)

(test is ready)               |     | Do: PGM_FLOW_TEST

|     |

PURG_TST_ENA = 1 --------|     |     |

(enabled by executive)  |OR --|     |

|           |

CCM_TST_ENA = 1 ---------|           |

(comprehensive component            |

test enabled)                      |

| --- ELSE ---

FLOW_RUNNING = 1 --------------|     |

(purge test is running)       |     |

|     |

LOAD > PGM_LOAD_MAX -----|     |     |

|     |     |

LOAD < PGM_LOAD_MIN -----|     |     |

(LOAD is not within     |     |     |

acceptable range)      |     |     |

|     |AND -| FLOW_RUNNING = 0

PGM_PFS_FM = 1 ----------|     |     |  (test was aborted due to load

(sensor or actuator     |     |     |   vehicle speed, engine stall,

failed)                |     |     |   re-entry into open loop,

|     |     |   low manifold vacuum, sensor/

OLFLG = 1 ---------------|     |     |   actuator criteria, OBDII

|     |     |   executive)

FLG_STALL = 1 -----------|     |     |

(vehicle stalled)       |OR --|     | PGM_RST_TMR = PGM_RST_TM

|           |  (load the reset timer with the

PURG_TST_ENA = 0 --|     |           |   time to wait until retest)

|AND -|           | PGM_RST_CTR = PGM_RST_CTR + 1

CCM_TST_ENA = 0 ---|     |           |  (increment the reset counter)

|           | PGM_1ST_PS = 0

|     VSBAR < PGM_VS_MIN ------|           | PGM_2ND_PS = 0

|           | PGM_DONE = 0

|     VSBAR > PGM_VS_MAX ------|           | PGM_PG_OFF = 0

(vehicle speed is not   |           |

in acceptable range)   |           |

|           |

INF_MVAC < PGM_MVAC_MIN -|           |

(manifold vacuum is     |           |

too low for test)      |           |

|           |

P0443MALF = 1 -----------|           |

|      (Purge valve not O.K.)  |           |

|                              |           |

|     TP_REL < PGM_TP_MIN -----|           |

| --- ELSE ---

|

FLOW_RUNNING = 1 --------------------| Do: PGM_FLOW_TEST

(purge test is running)             |  (run the actual test)

15-92

CANISTER PURGE STRATEGY, CANISTER PURGE FLOW TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Check for test completeness:

P0443MON = 1 -------------------|

(Purge valve checked)          |

|

P1443MON = 1 -------------|     |

(Purge flow checked)     |OR --|

|     |AND -| PURG_MON = 1

P0443MALF = 1 ------------|     |     |  (purge has been completely

(Purge circuit error)          |     |   monitored)

|     | PGM_DONE = 1

P1444MON = 1 -------------|     |     | PGM_1ST_PS = 0

|AND -|     | PGM_2ND_PS = 0

P1445MON = 1 -------------|           | PGM_RST_CTR = 0

(PFS sensor checked)                 | FLOW_RUNNING = 0

|

| --- ELSE ---

|

| PURG_MON = 0

| PGM_DONE = 0

15-93

CANISTER PURGE STRATEGY, CANISTER PURGE FLOW TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: PGM_FLOW_TEST

unconditionally --------------------| Increment PGM_TMR

PGM_1ST_PS = 0 ---------------------| PGM_FLOW_1 = PGM_PFS

|  (read in thermistor value)

| PGM_PG_OFF = 1

|  (turn off PURGE)

| PGM_TMR = 0

|  (clear timer to count up)

| PGM_1ST_PS = 1

|  (first pass done)

|

| --- ELSE ---

PGM_1ST_PS = 1 ---------------|     |

|     |

PG_DC <= PGM_PG_LO -----------|AND -| PGM_FLOW_2 = PGM_PFS

|     |  (read in thermistor value)

PGM_TMR >= PGM_TM ------------|     | PGM_DLT_FLOW = PGM_FLOW_1 -

|                PGM_FLOW_2

|  (clip to 0.0 as min)

| PGM_2ND_PS = 1

| PGM_PG_OFF = 0

|

| --- ELSE ---

|

| No action.

PGM_2ND_PS = 1 ---------------|

|AND -| Do: malfunction(purg,P1443)

PGM_DLT_FLOW <= PGM_DLT_THRS -|     |  (update MIL)

|

| --- ELSE ---

|

| Do: clear_malf(P1443)

|  (clear malfunction flag)

PGM_1ST_PS = 1 ---------------|

|AND -| P1443MON = 1

PGM_2ND_PS = 1 ---------------|     |

| --- ELSE ---

|

| P1443MON = 0

END: PGM_FLOW_TEST

15-94

CANISTER PURGE STRATEGY, VMV FLOW TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

15.9  VMV FLOW TEST (CDAN0)

OVERVIEW

This logic describes the vapor management valve (VMV) based flow test of  the

canister  purge  system for compliance with CARB and EPA OBD-II requirements.

The purpose of the test is to determine if flow is properly occurring in  the

Evaporative Fuel System.

By monitoring the effects of the canister purge strategy (PCOMP) on the  Idle

Speed  Control  strategy  and  then  testing  those  effects against expected

results, flow is inferred to be occurring in the Evaporative Fuel System.

o  Entry Conditions:

The test will begin if all of the following entry conditions are met:

.  The purge system shows no electrical faults.

.  The test has not yet been run this trip.

.  Powertrain load is within the calibrated window.

.  The vehicle speed is within a calibrated window.

.  ACT/ECT is within a calibrated window.

.  Time since closed loop is greater than a calibrated minimum.

.  PG_DC is above a calibrated value.

.  Purge vapor level is below a calibrated maximum.

.  Idle speed control strategy is active.

.  Air conditioning is not active.

.  Steady state  conditions  have  existed  for  VBAT  and  LOAD  for  a

calibrated amount of time before test.

o  Test Description:

The test compares the values of Idle Speed Control parameter IPSIBR at  a

high  calibratable PG_DC point when purge flow is expected to be high and

at PG_DC = 0 when purge flow is expected to be low or nonexistant.  Using

these  two points, the test calculates a delta value and compares this to

an expected calibratable value.  If equal  or  above  expected,  flow  is

inferred to be occurring and that NO malfunction exists.  If however, the

delta is below expected, flow is inferred to NOT  be  occurring  properly

and that a malfunction exists in the Evaporative Fuel System.

The test runs as follows.  If the conditions to run the test are met, the

test  begins  by  loading  in the current value of IPSIBR and turning off

PCOMP  by  setting  FLOW_RUNNING   to   1   (which   is   read   by   the

CANP_PCOMP_ENABLE_COMx  module  and  used  to  disable  PCOMP).   After a

calibratable time (PGM_TM), after PG_DC has decayed down to zero, and  if

NO  steady  state  changes in VBAT or LOAD have occurred, the new current

value of IPSIBR is loaded in and used to calculate a delta  value.   This

value  is  compared  to  PGM_IPS_LIM  and  according  to  the  result,  a

malfunction is either set or cleared.

15-95

CANISTER PURGE STRATEGY, VMV FLOW TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  ACT = Air Charge Temperature, deg.  F.

-  CCM_TST_ENA = OBD-II Comprehensive Component Test enable flag (1 =>  test

enabled).

-  CL_TMR = Closed loop timer (actuated).

-  ECT = Engine coolant temperature, deg F.

-  ISCDTY = Idle speed control duty cycle.

-  ISCFLG = ISC mode Flag; 1 -> RPM control mode.

-  IPSIBR = The closed loop integration  component  of  total  DESMAF,  ppm.

Designed  to  provide  integral  feedback,  IPSIBR  adjusts  the value of

DESMAF.  Changes in IPSIBR result in a  corresponding  change  to  bypass

valve duty cycle.

-  KWUCTR = KAM warm_up counter.  Stores number of warm_ups in KAM.   (Reset

to zero if KAM is corrupted, battery disconnect, etc.)

-  LOAD = Universal load as a ratio of aircharge over standard.

-  PCOMP_W = Value of purge vapors  currently  being  purged,  ppm  (reduced

range PCOMP_PPM).

-  PG_DC = Purge duty cycle.

-  PGM_FK = Filter factor to be used with PGM_IPS_KAM calculation.  Value is

either PGM_FK_NMAT or PGM_FK_MAT.

-  PGM_IPS_1 = First value of IPSIBR that is read in when PG_DC is high.

-  PGM_IPS_2 = Second value of IPSIBR that is read in when PG_DC is low.

-  PGM_IPS_DLT = Difference value between PGM_IPS_1 and PGM_IPS_2.

-  PGM_IPS_KAM = Weighed moving average value of PGM_IPS_DLT.

-  PGM_LOAD = Filtered value of LOAD used in the Purge Flow test.

-  PGM_LOAD_DLT = Absolute value of  the  difference  between  the  filtered

PGM_LOAD and the current value of LOAD.

-  PGM_SS_TMR = Timer that tracks steady state conditions of the purge test.

-  PGM_TEMP = Temperature (ECT/ACT combination) used in purge test.

-  PGM_TMR = Timer that increments between readings of the flow test.

-  PGM_VBAT = Filtered value of VBAT used in the Purge Flow test.

-  PGM_VBAT_DLT = Absolute value of  the  difference  between  the  filtered

PGM_VBAT and the current value of VBAT.

15-96

CANISTER PURGE STRATEGY, VMV FLOW TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PURG_TST_ENA = Purge test enabled flag; 1 -> Purge test has been  enabled

by the OBD-2 executive.

-  VSBAR = Vehicle speed, MPH.

-  VBAT = Battery voltage.

Bit Flags:

-  FLOW_RUNNING = Purge test is running flag; 1 -> running.

-  ISFLAG = Flag that indicates the degree of loading on the engine at idle.

-  ISLAST = Register which indicates the engine load state from the previous

background loop.

-  KAM_ERROR = Indicates keep alive ram invalid.

-  OBDII_RESET = Flag used to simulate the receipt of an  OBD-II  scan  tool

reset message.

-  OLFLG = Open loop flag; 1 -> open loop mode, 0 -> closed loop mode.

-  FRST_ADP = Flag indicating that the ADAPT_TMR has  incremented  at  least

once, therefore, have entered adaptive fuel at least once.

-  PGM_1ST_PS = First pass through the flow test  flag  when  a  reading  is

taken; 1 -> first pass seen.

-  PGM_2ND_PS = Second pass through the flow test flag  when  a  reading  is

taken; 1 -> second pass seen.

-  PGM_DONE = Purge test is done flag; 1 -> test is done.

-  PGM_VALS_GOT = Flag that indicates that the two sets of flow values  have

been read in.

-  PURG_TST_RDY = Purge test ready flag; 1 -> conditions have been met.

-  PxxxMALF = OBDII malfunction flag for  fault  xxx;  1  ->  a  malfunction

currently exists for fault xxx.

-  PxxxMON = OBDII monitor flag for fault xxx; 1 -> fault has been monitored

at least once since power-up.

-  PURG_MON = OBDII monitor flag for  evaporative  purge;  1  ->  all  purge

faults been monitored since power-up.

15-97

CANISTER PURGE STRATEGY, VMV FLOW TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  PGM_CL_TM = Time in closed loop to allow purge test.

-  PGM_FK_MAT = PGM_FK value used when KAM is considered mature.

-  PGM_FK_NMAT = PGM_FK value used when KAM is NOT considered mature.

-  PGM_IPS_INIT = Initial value for PGM_IPS_KAM if KAM is corrupted.

-  PGM_LD_DT_MX = Maximum value of PGM_LOAD_DLT  above  which  the  test  is

aborted.

-  PGM_IPS_LIM = Limit value of PGM_IPS_DLT below which a fault is assumed.

-  PGM_ISC_MAX = Maximum value of ISCDTY to allow purge test above  which  a

possible ISC failure is assumed.

-  PGM_ISC_MIN = Minimum value of ISCDTY to allow purge test below  which  a

possible ISC failure is assumed.

-  PGM_KWUCNT = Maximum mumber of warm_up cycles to use PGM_FK_NMAT.

-  PGM_LOAD_MAX = Maximum load to allow purge test.

-  PGM_LOAD_MIN = Minimum LOAD to allow purge test.

-  PGM_PG_START = Purge duty cycle above which to allow the  purge  test  to

begin.

-  PGM_PPM_WMX = Maximum value of PCOMP_W allowed to run the purge test.

-  PGM_PPM_WMN = Minimum value of PCOMP_W allowed to  bypass  the  VMV  Flow

Test  logic  and  set  the  PURG_MON  flag.   Flow  is assumed if above a

certain, sufficient level.

-  PGM_SS_TM = Time to wait while in steady state conditions  before  trying

purge test.

-  PGM_TC_LOAD = Time constant used with ROLAV filtering of PGM_LOAD.

-  PGM_TC_VBAT = Time constant used with ROLAV filtering of PGM_VBAT.

-  PGM_TEMP_MAX = Maximum temperature to allow purge test.

-  PGM_TEMP_MIN = Minimum temperature to allow purge test.

-  PGM_TEMP_MUL = Purge test fractional multiplier; 1.0 -> use all ECT;  0.0

-> use all ACT.

-  PGM_TM = Time to delay between readings of the flow test.

-  PGM_VB_DT_MX = Maximum value of PGM_VBAT_DLT  above  which  the  test  is

aborted.

-  PGM_VS_MAX = Maximum vehicle speed to allow purge test.

15-98

CANISTER PURGE STRATEGY, VMV FLOW TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OTHER

-  purg_codes = Set of {P0443,P1443}.  The set of  OBDII  fault  codes  that

relate to the evaporative purge OBDII test.

-  malfunction(purg,Pxxx) = Logic process, imported  from  the  MIL  control

Module; Pxxx indicates the fault code.

-  P1443 = Fault  code,  evaporative  emission  control  system  purge  flow

malfunction.

-  P0443 = Fault code, evaporative emission  control  system  purge  circuit

malfunction.

-  P0505 = Inlet Air Control (idle speed) system malfunction.

-  P1505 = Inlet Air Control System at adaptive Clip.

-  P1506 = Inlet Air Control Duty cycle is lower than expected.

-  P1507 = Inlet Air Control Duty cycle is higher than expected.

15-99

CANISTER PURGE STRATEGY, VMV FLOW TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CANP_VMV_FLOW_COM1

BEGIN: flow_inference_test

; this is the main process that calls all others.

OBDII_RESET = 1 ------------------|

|OR --| PGM_DONE := 0

KAM_ERROR = 1 --------------------|     | PGM_IPS_KAM := PGM_IPS_INIT

; bypass the KOER test logic

ER_STATUS = PURG_ER_INIT ---------------| ER_STATUS := PURG_ER_DONE

; KOER test being desired              |  ; KOER test is complete

PGM_DONE = 1 ---------------------------| EXIT flow_inference_test.

; check to see if sufficient flow is seen to bypass the test

P0443MON = 1 ---------------------|

; Purge valve checked            |

|

FRST_ADP = 1 ---------------------|AND -| PURG_MON := 1

; Adapted at least once          |     | ; purge has been completely

|     | ; monitored

PCOMP_W >= PGM_PPM_WMN -----------|     | P1443MON := 1

; Sufficient flow                      | ; flow monitored

| PGM_DONE := 1

| Do: reset_parameters

| EXIT flow_inference_test.

unconditionally ------------------------| Do: pgm_filtering

|  ; call process that calculates

|  ; PGM_TEMP, PGM_VBAT_DLT, and

|  ; PGM_LOAD_DLT. logic found in

|  ; CANP_PGM_FILT_COMx

OLFLG = 0 ------------------------------| Increment CL_TMR.

; in closed loop                       |

| --- ELSE ---

|

| Freeze CL_TMR.

15-100

CANISTER PURGE STRATEGY, VMV FLOW TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

ISCDTY > PGM_ISC_MIN -------------|

|

ISCDTY < PGM_ISC_MAX -------------|

; ISCDTY value reasonable        |

|

ISCFLG = 1 -----------------------|

; in Idle speed control          |

|AND -| Increment PGM_SS_TMR

PGM_VBAT_DLT < PGM_VB_DT_MX ------|     |  ; time in steady state

; change in VBAT below max       |     |

|     |

PGM_LOAD_DLT < PGM_LD_DT_MX ------|     |

; change in LOAD below max       |     |

|     |

ISFLAG = ISLAST ------------------|     |

; no load state change                 |

| --- ELSE ---

|

| PGM_SS_TMR := 0

15-101

CANISTER PURGE STRATEGY, VMV FLOW TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

LOAD > PGM_LOAD_MIN --------------|

|

LOAD < PGM_LOAD_MAX --------------|

; LOAD within acceptable range   |

|

PGM_TEMP < PGM_TEMP_MAX ----------|

|

PGM_TEMP > PGM_TEMP_MIN ----------|

; temperature is within          |

; acceptable range               |

|

VSBAR < PGM_VS_MAX ---------------|

; vehicle speed is within        |

; acceptable range               |

|

CL_TMR > PGM_CL_TM ---------------|

; in closed loop long enough     |

|

P0443MON = 1 ---------------------|

; Purge valve checked            |AND -| PURG_TST_RDY := 1

|     |  ; purge test is ready

P0443MALF = 0 --------------------|     |

; Purge valve O.K.               |     |

|     |

ISCFLG = 1 -----------------------|     |

; idle control is active         |     |

|     |

PGM_SS_TMR > PGM_SS_TM -----------|     |

; in steady state long enough    |     |

|     |

P0505MALF = 0 --------------------|     |

; IAC not malfunctioning         |     |

|     |

P1505MALF = 0 --------------------|     |

; IAC not at adaptive clip       |     |

|     |

P1506MALF = 0 --------------------|     |

; IAC duty cycle is not too low  |     |

|     |

P1507MALF = 0 --------------------|     |

; IAC duty cycle is not too high       |

| --- ELSE ---

|

| PURG_TST_RDY := 0

| Do: reset_parameters

| EXIT flow_inference_test.

15-102

CANISTER PURGE STRATEGY, VMV FLOW TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLOW_RUNNING = 0  ----------------|

; test not running               |

|

PURG_TST_RDY = 1 -----------------|

; test is ready                  |

|

PG_DC > PGM_PG_START -------------|

; duty cycle high enough         |

|AND -| ; Initial conditions met.

PCOMP_W < PGM_PPM_WMX ------------|     | ; Begin flow test.

; purge vapors not excessive     |     | ; Turn off PCOMP.

|     | FLOW_RUNNING := 1

PGM_1ST_PS = 0 -------------------|

; phase one not done yet         |

|

PURG_TST_ENA = 1 -----------|     |

; test enabled by OBD-2    |     |

; executive                |     |

|     |

CCM_TST_ENA = 1 ------------|OR --|

; comprehensive component  |

; test enabled             |

FLOW_RUNNING = 0 -----------------------| PGM_TMR := 0

|

| --- ELSE ---

|

| ; allow PGM_TMR

| ; to increment

FLOW_RUNNING = 1 -----------------------| Do: get_flow_values

|

| --- ELSE ---

|

PGM_VALS_GOT = 1 -----------------------| Do: check_flow_values

|

| --- ELSE ---

|

| Do: reset_parameters

| Exit: flow_inference_test.

END: flow_inference_test

15-103

CANISTER PURGE STRATEGY, VMV FLOW TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: get_flow_values

; This process is called by the process: flow_inference_test.

; Its function is to load the flow readings when appropriate.

PGM_1ST_PS = 0 -------------------------| PGM_IPS_1 := IPSIBR

; first time through                   | PGM_1ST_PS := 1

|

| --- ELSE ---

PGM_1ST_PS = 1 -------------------|     |

|     |

PGM_TMR > PGM_TM -----------------|AND -| PGM_IPS_2 := IPSIBR

|     | PGM_2ND_PS := 1

PGM_2ND_PS = 0 -------------------|

PGM_1ST_PS = 1 -------------------|

|AND -| PGM_VALS_GOT := 1

PGM_2ND_PS = 1 -------------------|     | P1443MON := 1

| FLOW_RUNNING := 0

|

| --- ELSE ---

|

| PGM_VALS_GOT := 0

| P1443MON := 0

END: get_flow_values

BEGIN: check_flow_values

; This process is called by the process: flow_inference_test. Its

; functions are to load the appropriate filter value, calculate the

; delta of the flow readings, perform the weighted average filtering on the

; flow delta, check this filtered value against calibrated values, and set

; or clear the appropriate fault code.

; decide which filter constant to use based on KAM warm up counter value

KWUCTR < PGM_KWUCNT --------------------| PGM_FK := PGM_FK_NMAT

; First few warm_up cycles              |

; KAM considered not mature             |

| --- ELSE ---

|

| PGM_FK := PGM_FK_MAT

; calculate the delta of the flow readings

; perform the weighted average filtering on the flow delta

unconditionally ------------------------| pgm_ips_lst := PGM_IPS_KAM

| PGM_IPS_DLT := PGM_IPS_2 -

|                PGM_IPS_1

| PGM_IPS_KAM := PGM_FK * PGM_IPS_DLT

|          + (1-PGM_FK) * pgm_ips_lst

15-104

CANISTER PURGE STRATEGY, VMV FLOW TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

; check filtered value against calibrated limit

PGM_IPS_KAM < PGM_IPS_LIM --------------| ; Error flow indicated.

| ; Call MIL control procedure

| Do: malfunction(purg,P1443)

|

|

| --- ELSE ---

|

| ; Purge functioning properly.

| ; Flow is indicated.

| Do: clear_malf(P1443)

; check for monitor completeness

P0443MON = 1 ---------------------|

; Purge valve checked            |

|AND -| PURG_MON := 1

P1443MON = 1 ---------------|     |     | ; purge has been completely

; Purge flow checked       |OR --|     | ; monitored

|           | PGM_DONE := 1

P0443MALF = 1 --------------|           | Do: reset_parameters

; Purge circuit error                  |

| --- ELSE ---

|

| PURG_MON := 0

| PGM_DONE := 0

END:  check_flow_values

BEGIN: reset_parameters

; This process is called by the process: flow_inference_test.

; Its function is to clear the appropriate values of the

; flow_inference_test if it is aborted or completed.

unconditionally ------------------------| FLOW_RUNNING := 0

| PGM_1ST_PS := 0

| PGM_2ND_PS := 0

| PGM_VALS_GOT := 0

| PGM_TMR := 0

END: reset_parameters

15-105

CANISTER PURGE STRATEGY, VMV FLOW TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

15-106

CHAPTER 16

ELECTRO-DRIVE FAN STRATEGY

16-1

ELECTRO-DRIVE FAN STRATEGY

16.1  FEATURE:  ELECTRO-DRIVE FAN CONTROL STRATEGY (EDF) - V3.3_EDF (CDAN0)

16.1.1  LOW SPEED FAN STRATEGY (CDAH0)

OVERVIEW

Electro-drive fan refers to an electric motor  driven  fan  used  to  provide

forced  air  circulation through the engine coolant radiator when natural air

flow  is  inadequate;  i.e.,  low  vehicle  speed  or  high  engine   coolant

temperature.   The fan is always turned off during CRANK mode to minimize the

accessory load.

Fan control is accomplished through the use of EEC controlled relays  in  the

fan  power  supply  circuit.   This  strategy  may  be  used  with  either  a

"one-speed", a "two-speed" or two single fans.  The "two-speed" fan motor may

either  be  a dual-wound type or a single wound type with a dropping resistor

to achieve low speed operation.

This Strategy Module performs the function of requesting that the  Low  Speed

Fan be switched on.

When the WOT EDF cutout feature is envoked, the LSF is turned off only if  it

is  not  required  to  be on for some other reason, primarily ECT (LSFECT_FLG

being set).  Therefore, if the LSF is on ONLY because AC is requested and the

vehicle  speed  is  low, then when ACRQST is cleared because ACTP_FLG is set,

LSF_RQST is cleared by ACTP_FLG being equal to 1.

Note that the flag LSF_RQST is not cleared automatically when  LSFECT_FLG  is

cleared:   it  relies  upon  one of the specific conditions documented in the

logic overleaf to clear it, typically ACIOTMR or VSBAR checks.

DEFINITIONS

INPUTS

Registers:

-  ACIOTMR = A/C clutch off timer, sec.  Used to prevent  rapid  cycling  of

the A/C compressor and electro-drive fan.

-  ECT = Engine Coolant Temperature, deg F.

-  VSBAR = Filtered Vehicle Speed, MPH.

Bit Flags:

-  ACTP_FLG = Flag to disable EDF and A/C at high TP.

-  ACRQST = Flag that is set to 1 when A/C clutch desired.

-  CFMFLG = Flag indicating the ECT sensor has failed.

-  CRKFLG = Flag that is set when in CRANK mode.

16-2

ELECTRO-DRIVE FAN STRATEGY, LOW SPEED FAN STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  HSF_RQST = Flag indicating the high speed fan is requested.

-  LSFECT_FLG = LSF ECT F/F flag.

-  OSM_EDF_OFF = Request for EDF Off from OSM or OTM test.

-  OSM_EDF_ON = Request for EDF on from OSM or OTM test.

Calibration Constants:

-  AC_MN_FT = Minimum EDF on time after  A/C  clutch  off,  secs.   Prevents

rapid fan cycling.  (Should be > 10 seconds)

-  EDFHP = Electro drive fan hardware present switch.  Calibrate to "1"  for

either one or two speed fan control; "0" for no fan control.

-  LSFECT_CL = Maximum ECT to turn off Low Speed Fan, deg F.  (Should  be  4

degrees less than LSFECT_SH, per Car Engineering Direction)

-  LSFECT_SH = Minimum ECT to turn on Low Speed Fan, deg F.  (Should be  216

deg F, per Car Engineering Direction)

-  LSFVS_CH = Minimum vehicle speed to turn off  the  Low  Speed  Fan,  MPH.

(Should be 5 MPH higher than LSFVS_SL, per Car Engineering Direction)

-  LSFVS_SL = Maximum vehicle speed to turn on Low Speed Fan because the A/C

is on, MPH.  (Should be 48 MPH, per Car Engineering Direction)

OUTPUTS

Bit Flags:

-  LSF_RQST = Flag indicating Low Speed Fan is requested.

-  LSFECT_FLG = See above.

16-3

ELECTRO-DRIVE FAN STRATEGY, LOW SPEED FAN STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EDF_LSF_CONTROL_COM1

EDFHP = 1 ------------------------------------|

|AND -|S Q -| LSFECT_FLG

ECT > LSFECT_SH ------------------------------|     |

(hot engine)                                       |

|

EDFHP = 1 ------------------------------------|     |

|AND -|C

ECT < LSFECT_CL ------------------------------|

EDFHP = 1 ------------------------------------|

(EDF hardware present)                       |

|

OSM_EDF_OFF = 0 ------------------------------|

(OSM NOT forced off)                         |

|AND -|S Q -| LSF_RQST

OSM_EDF_ON = 1 -------------------------|     |     |

|     |     |

CRKFLG = 0 -----------------------|     |OR --|     |

(not in crank mode)              |     |           |

|     |           |

CFMFLG = 1 -----------------|     |AND -|           |

(ECT sensor failed)        |     |                 |

|     |                 |

HSF_RQST = 1 ---------------|     |                 |

|OR --|                 |

LSFECT_FLG = 1 -------------|                       |

|                       |

ACRQST = 1 -----------|     |                       |

(A/C on)             |AND -|                       |

|                             |

VSBAR < LSFVS_SL -----|                             |

(vehicle speed low)                                |

|

EDFHP = 0 ------------------------------------|     |

(EDF hardware not present)                   |     |

|     |

CRKFLG = 1 -----------------------------------|     |

(in crank mode)                              |     |

|     |

VSBAR > LSFVS_CH -----------------------------|OR --|C

(vehicle speed high)                         |

|

ACIOTMR >= AC_MN_FT --------------------------|

(fan turn off delay)                         |

|

ACTP_FLG = 1 ---------------------------------|

(WOT A/C Cutout invoked)                     |

|

OSM_EDF_OFF = 1 ------------------------------|

16-4

ELECTRO-DRIVE FAN STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

16.1.2  HIGH SPEED FAN STRATEGY (CDAN0)

FEATURE MEMBERSHIP : EDF

EXECUTION CONTEXT : Background

FUNCTIONAL REQUIREMENTS :

This module controls the setting and clearing of  the  flag  HSF_RQST.   This

indicates  that  the  High speed fan has been requested.  The request flag is

passed to the module EDF_CONTROL_COM  to allow the correct sequencing of  the

outputs.

If an A/C medium pressure switch is to be used  the  module  INPUT_ACPSW_COM

controls the flag ACPSW, otherwise it remains at it's base value of zero.  If

an A/C pressure transducer is fitted the  module  INPUT_ACPRES_COM   controls

the flag ACPRES_HSF, otherwise the base value of zero is maintained.

FUNCTIONAL DESCRIPTION :

This module performs the function of requesting the high speed fan.

The High speed fan can be requested by any of the following conditions:

-  Very high ECT

-  ECT sensor failure

-  High A/C head pressure (Transducer)

-  Medium A/C head pressure switch tripped

-  High RPM and Load with low Vehicle speed ( Grade load )

-  Output state control forces ON state

The high speed fan request will be cleared when none  of  the  above  request

conditions are true and :

-  Low ECT and Low A/C head pressure for a sufficient time

However the fan will be forced off by any of the following :

-  Output state control forces it off

-  HEDF Output State Monitor failure

-  Engine in CRANK mode

16-5

ELECTRO-DRIVE FAN STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  ECT = Engine Coolant Temperature, deg F.

-  HSF_ACP_OTMR = HSF AC Pressure Off timer.

-  LOAD = Normalized aircharge parameter (CYLARC_BG/SARCHG), unitless.

-  N_BYTE = Byte RPM.

-  VSBAR = Filtered Vehicle Speed, MPH.

Bit Flags:

-  ACPRES_HSF = Flag indicating that the high speed fan is required for high

A/C head pressure.

-  ACPSW = AC Medium Pressure Switch Input; 1 -> AC Pressure high.

-  CFMFLG = Flag indicating the ECT sensor has failed.

-  CRKFLG = Engine Mode Flag; 1 -> Crank Mode, 0 -> not in Crank Mode.

-  OSM_HEDF_OFF = Request for HEDF Off from OSM or OTM test.

-  OSM_HEDF_ON = Request for HEDF On from OSM or OTM test.

-  P1479MALF = HEDF OSM fault malfunction flag.

16-6

ELECTRO-DRIVE FAN STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  HEDFHP = Two speed fan hardware present switch. Calibrate to "1" for

two speed fan control; "0" for one speed or no fan control.

-  HSF_ACP_ROTM = HSF AC Pressure Run on time.

-  HSFECT_CL = Maximum ECT to turn off High Speed Fan, deg F.

-  HSFECT1_SH = Minimum ECT to turn on High Speed Fan for Grade Load, deg F.

-  HSFECT2_SH = Minimum ECT to turn on High Speed Fan normally, deg F.

-  HSFLOD = Minimum LOAD to turn on High Speed Fan for Grade Load.

-  HSFRPM = Minimum RPM to turn on High Speed Fan for Grade Load, rpm.

-  HSFVS = Maximum vehicle speed to turn on High Speed Fan for Grade Load,

mph.

OUTPUTS

Bit Flags:

-  HSF_RQST = Flag indicating requested status of HSF.

16-7

ELECTRO-DRIVE FAN STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EDF_HSF_CONTROL_COM4

HEDFHP = 1 -----------------------------------|

;HSF hardware present                        |

|

OSM_HEDF_OFF = 0 -----------------------------|

;OSM NOT forced off                          |AND -|S Q -| HSF_RQST

|     |

OSM_HEDF_ON = 1 ------------------------|     |     |

|     |     |

P1479MALF = 0 --------------------|     |     |     |

|     |     |     |

CRKFLG = 0 -----------------------|     |     |     |

;not in Crank mode               |     |OR --|     |

|     |           |

ACPRES_HSF = 1 -------------|     |     |           |

;AC Pressure high          |     |     |           |

|     |AND -|           |

ACPSW = 1 ------------------|     |                 |

;AC Med. Pres. Sw. tripped |     |                 |

|     |                 |

CFMFLG = 1 -----------------|     |                 |

;ECT sensor failed         |     |                 |

|OR --|                 |

ECT >= HSFECT2_SH ----------|                       |

;higher temp. turn on      |                       |

|                       |

N_BYTE >= HSFRPM -----|     |                       |

;high RPM            |     |                       |

|     |                       |

LOAD >= HSFLOD -------|     |                       |

;high load           |AND -|                       |

|                             |

VSBAR <= HSFVS -------|                             |

;low veh. speed      |                             |

|                             |

ECT >= HSFECT1_SH ----|                             |

;lower temp. turn on                               |

|

HEDFHP = 0 -----------------------------------|     |

;HSF hardware not present                    |     |

|     |

ECT < HSFECT_CL ------------------------|     |     |

;HSF turn off temperature              |AND -|     |

|     |OR --|C

HSF_ACP_OTMR > HSF_ACP_ROTM ------------|     |

;low AC Pressure for a time                  |

|

OSM_HEDF_OFF = 1 -----------------------------|

|

P1479MALF = 1 --------------------------------|

|

CRKFLG = 1 -----------------------------------|

;in crank mode

16-8

ELECTRO-DRIVE FAN STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

ACPSW = 1 ------------------------|

;Medium pressure switch tripped  |

|OR --| HSF_ACP_OTMR := 0

ACPRES_HSF = 1 -------------------|     |

;High A/C head pressure                | --- ELSE ---

|

| ;increment HSF_ACP_OTMR

16-9

ELECTRO-DRIVE FAN STRATEGY, ELECTRO-DRIVE FAN CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

16.1.3  ELECTRO-DRIVE FAN CONTROL STRATEGY (CDAN0)

OVERVIEW

This Strategy Module takes the requests for fan operation and  sequences  the

fan outputs using calibratable time delays.

In order to provide anticipatory Idle Speed compensation with any combination

of  relays  and  hardware time delays, two software time delays are provided.

The first, EDF_DELAY, delays Idle Speed action  (see  Desired  RPM  logic  in

ISC),  and  the  second,  FAN_DELAY,  delays  EDF  activation.  If a hardware

activation delay causes the fan to energize too long after  ISC  compensation

(i.e.,  RPM  flare),  set  FAN_DELAY  = 0 and increase EDF_DELAY.  If the fan

energizes too soon (i.e., RPM dip), set EDF_DELAY = 0 and increase FAN_DELAY.

To prevent engine overheating if the  EEC  enters  HLOS  (when  the  EDF/HEDF

outputs are "off"), the fan power supply circuit must energize the fan unless

the EEC output is energized to turn the fan off.  That is, the EDF relay must

be  a  "normally closed" relay, or a logic inverter is needed between the EEC

and a "normally open" relay.  The Integrated Relay Controller  Module  (IRCM)

uses a "normally open" relay with an inverter for the EDF circuit.

This, however, creates another condition that may damage the relay  contacts.

Everytime  the processor is powered-up or during a "reset", the EDF output is

off (fan on) for approximately 100 milliseconds before the output  is  turned

on  (fan  off).   This  will  cause relay contact arcing due to the very high

current flowing at that time.  The relay must be rated for  opening  at  peak

current,  or  a hardware delay may be used to delay closing a "normally open"

relay immediately when the EDF output is de-energized.

MLI modules have a hardware delay only  in  HLOS  mode,  and  have  no  delay

present  during  normal  operation.   MLII  modules  have a hardware delay of

between 110 - 590 milliseconds dependant on tolerance, with a  typical  value

being  400  milliseconds.  This delay occurs every time the EDF is energized.

The IRCM uses a hardware delay of approximately 550  milliseconds  to  bridge

the  initialization  time.   This  delay  also  occurs  every time the EDF is

energized.

CONFIGURATION

If an electro-drive fan is not used, set both EDFHP and HEDFHP = 0.

To control a "one-speed" fan, set EDFHP = 1 and HEDFHP = 0.

To control a "two-speed" dual-wound fan, set both EDFHP and HEDFHP  =  1  and

DWFHP = 1.

To control either a "two-speed" fan with a series resistor, or  two  separate

fans set both EDFHP and HEDFHP = 1 and DWFHP = 0.

16-10

ELECTRO-DRIVE FAN STRATEGY, ELECTRO-DRIVE FAN CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  FANTMR = Time since electro-drive fan was requested, sec.

Bit Flags:

-  LSF_FLG = Flag indicating the status of the Low Speed Fan.

-  HSF_FLG = Flag indicating the status of the High Speed Fan.

-  LSF_RQST = Flag indicating that the low speed fan is requested.

-  HSF_RQST = Flag indicating that the high speed fan is requested.

Calibration Constants:

-  DWFHP = Flag indicating that Dual-Wound Fan hardware is present.

-  EDFHP = Electro drive fan hardware present switch; 1 -> either one or two

speed fan control, 0 -> no fan control.

-  FAN_DELAY = Time delay before energizing EDF output, sec.

-  HEDFHP = Two speed fan hardware  present  switch;  1  ->  two  speed  fan

control, 0 -> one speed or no fan control.

-  HSFAN_DELAY = Time delay before energising HEDF.

-  LSF_RLY_PRCT = Time to delay turn-off of LSF after it  is  requested,  to

prevent relay damage.

OUTPUTS

Registers:

-  FANTMR = See above.

Bit Flags:

-  LSF_FLG = See above.

-  HSF_FLG = See above.

16-11

ELECTRO-DRIVE FAN STRATEGY, ELECTRO-DRIVE FAN CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EDF_CONTROL_COM1

EDFHP = 1 ------------------------------|

;EDF hardware present                  |

|

LSF_RQST = 1 ---------------------------|

|AND -|S Q -| LSF_FLG

FANTMR > FAN_DELAY ---------------------|     |

|     |

DWFHP = 0 ------------------------|     |     |

|OR --|     |

HSF_FLG = 0 ----------------------|           |

|

EDFHP = 1 ------------------------------|     |

|     |

FANTMR > LSF_RLY_PRCT ------------------|     |

;Time delay for LSF Relay protection   |     |

|     |

HSF_FLG = 1 ----------------|           |AND -|C

|AND -|     |

DWFHP = 1 ------------------|     |     |

|OR --|

LSF_RQST = 0 ---------------------|

HEDFHP = 1 -----------------------------|

;HEDF hardware present                 |

|AND -|S Q -| HSF_FLG

HSF_RQST = 1 ---------------------------|     |

|     |

FANTMR > HSFAN_DELAY -------------------|     |

|

HSF_RQST = 0 ---------------------------|     |

|AND -|C

HEDFHP = 1 -----------------------------|

16-12

ELECTRO-DRIVE FAN STRATEGY, ELECTRO-DRIVE FAN CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FANTMR - TIME SINCE LSF REQUESTED

Timer used to provide the proper time delay between ISC compensation and  EDF

electrical  load application.  (See EDF OVERVIEW and DESIRED RPM CALCULATION)

FANTMR increments when a fan is requested, and is only reset  once  the  fans

are turned off.  It is used for 3 purposes (In chronological order):

1.  To provide a delay to enable ISC Compensation before turning on the

Low Speed Fan. (FAN_DELAY)

2.  To ensure that once the Low Speed Fan is turned on, it stays on for

a minimum period, (FANTMR > LSF_RLY_PRCT) to ensure that the relay is

not required to open while passing the high surge currents that are

associated with the first couple of seconds of EDF operation.

3.  To delay the turn on of the High Speed Fan (FANTMR > HSFAN_DELAY) in

order that the Engine may have stabilised and the LSF reached its

minimum electrical loading. Note: In previous strategy levels this

function was provided by a separate timer, EDFTMR, and a Cal. Const.

EDFTM. As a starting point, set HSFAN_DELAY = FAN_DELAY + EDFTM.

HSFAN_DELAY should be > LSF_RLY_PRCT.

LSF_RQST = 0 ---------------------------------|

|

HSF_RQST = 0 ---------------------------------|

|AND -| FANTMR := 0

LSF_FLG = 0 ----------------------------------|     |

|     |

HSF_FLG = 0 ----------------------------------|     |

| --- ELSE ---

|

| ;Increment FANTMR

16-13

ELECTRO-DRIVE FAN STRATEGY, EDF OUTPUT STATE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

16.1.4  FAN OUTPUT STATE MONITOR (CDAJ0)

OVERVIEW

This module documents the Output State Monitor for MLII Smart Output Drivers.

The status bits, EDF_STATUS and HEDF_STATUS should always be zero if there is

no fault present.

DEFINITIONS

INPUTS

Registers:

-  P1474_TMR = Time that EDF OSM fault has been present.

-  P1479_TMR = Time which HEDF OSM fault has been present.

-  VBAT = Battery voltage.

Bit Flags:

-  CCM_EO_ENA = Engine off on demand test for CCM enabled.

-  CCM_ER_ENA = Engine running on demand test for CCM enabled.

-  CCM_TST_ENA = OBDII Comprehensive Component test enable flag; 1 -> test.

-  EDF_STATUS = EDF Smart Output Driver fault status flag.

-  HEDF_STATUS = HEDF Smart Output Driver fault status flag.

16-14

ELECTRO-DRIVE FAN STRATEGY, EDF OUTPUT STATE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration constants:

-  EDFHP = Fan hardware present switch.

-  HEDFHP = Two speed fan output present.

-  VBAT_CCM_MIN = Minimum battery voltage to enable OSM testing.

-  P1474_TM_LMT = Time that EDF OSM fault must  be  present  before  setting

code.

-  P1479_TM_LMT = Time that HEDF OSM fault must be present  before  code  is

set.

OUTPUTS

Others:

-  P1474 = EDF OSM Malfunction code.

-  P1479 = HEDF OSM Malfunction flag.

16-15

ELECTRO-DRIVE FAN STRATEGY, EDF OUTPUT STATE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EDF_OSM_COM1

VBAT < VBAT_CCM_MIN --------------|

|

CCM_TST_ENA = 0 ------------|     |OR --|

|     |     | NO ACTION

CCM_EO_ENA = 0 -------------|AND -|     |  ;Do not perform OSM

|           |  ;tests.

CCM_ER_ENA = 0 -------------|           |

| --- ELSE ---

|

|  ;Perform OSM tests where

|  ;hardware present

| DO: EDF_TESTS

| DO: HEDF_TESTS

BEGIN: EDF_TESTS

EDFHP = 0 ------------------------|

|OR --| P1474_TMR := 0

EDF_STATUS = 0 -------------------|     | DO: Clear_malf (P1474)

|  ;Correct state, clear

|  ;error code

|

| --- ELSE ---

|

P1474_TMR > P1474_TM_LMT ---------------| DO: Store_code (P1474)

;Fault present long enough             |

| --- ELSE ---

|

| ;Fault pending, allow

| ;free running timer

| ;to increment

END: EDF_TESTS

16-16

ELECTRO-DRIVE FAN STRATEGY, EDF OUTPUT STATE MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: HEDF_TESTS

HEDFHP = 0 -----------------------|

|OR --| P1479_TMR := 0

HEDF_STATUS = 0 ------------------|     | DO: Clear_malf (P1479)

|  ;No error present

|  ;clear MALF flag

|

| --- ELSE ---

|

P1479_TMR > P1479_TM_LMT ---------------| DO: Store_code (P1479)

;Fault present long enough             |

| --- ELSE ---

|

| ;Fault pending, allow

| ;free running error

| ;timer to increment

END: HEDF_TESTS

16-17

VARIABLE SPEED FAN STRATEGY, A/C FAN CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

16.1.5  A/C FAN CONTROL (CDAL0)

OVERVIEW

When A/C is demanded and vehicle speed is low the fan is turned on to control

A/C  head  pressure.   The  fan  speed  is  proportional to the filtered head

pressure plus a fast response term if the pressure  increases  quickly  above

the filtered value.

At very high vehicle speeds the ram air effect allows the cooling fan  to  be

turned  off  with  no decrease in cooling performance.  As a failsafe, if the

pressure is too high this feature is not allowed.

DEFINITIONS

Registers:

-  ACIOTMR = A/C OFF delay timer, for ISC.

-  ACPRES = A/C Head pressure.

-  ACPRESBAR = Filtered head pressure for fast rise control, level.

-  VSBAR = Filtered Vehicle Speed, MPH.

-  VSF_AC = VSF requested by A/C.

-  VSF_DPDT = Fast rise pressure term for A/C fan control.

Bit Flags:

-  VSF_AC_FLG = Flag indicating A/C fan demand.

-  VSF_VS_FLG = Flag indicating high vehicle speed to disable fan.

Calibration Constants:

-  AC_MN_FT = Minimum time  to  run  low  speed  EDF  after  A/C  clutch  is

disengaged, sec.

-  FN641(ACPRESBAR) = Proportional fan  speed  fraction  vs  Head  pressure,

level.

-  FN641F(VSF_DPDT) = Fast pressure rise adder, level.

-  TCVSFPRES = Time constant for fast pressure rise adder.

-  VSF_PRES_CL = Pressure below which vehicle speed turn off is enabled.

-  VSF_PRES_SH = Pressure constraint on vehicle speed turn off, PSI.

-  VSF_VS_CL = Vehicle Speed to return to normal fan control, MPH.

16-18

VARIABLE SPEED FAN STRATEGY, A/C FAN CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  VSF_VS_SH = Vehicle Speed to turn off fan during A/C operation, MPH.

16-19

VARIABLE SPEED FAN STRATEGY, A/C FAN CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EDF_VSF_AC_COM1

VSBAR > VSF_VS_SH -------------|

|AND -|S Q -| VSF_VS_FLG

ACPRES < VSF_PRES_SH ----------|     |

|

ACPRES > VSF_PRES_CL ----------|     |

|OR --|C

VSBAR < VSF_VS_CL -------------|

ACRQST = 1 --------------------------|S Q -| VSF_AC_FLG

|

ACIOTMR > AC_MN_FT ------------------|C

ACPRESBAR = ROLAV(ACPRES,TCVSFPRES)

VSF_DPDT =  ACPRES - ACPRESBAR

VSF_VS_FLG = 1 ----------------------|

|OR --| VSF_AC := 0

VSF_AC_FLG = 0 ----------------------|     |  ;no A/C fan required

|

| --- ELSE ---

|

| VSF_AC := FN641(ACPRESBAR)

|    + FN641F(VSF_DPDT)

16-20

VARIABLE SPEED FAN STRATEGY, ECT FAN CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

16.1.6  ECT FAN CONTROL (CDAL0)

OVERVIEW

Shown below is the logic for the control of the  engine  coolant  temperature

around  a  set  point,  VSF_DESECT.   FN642(ect_error)  provides proportional

control for fast response during engine overtemp conditions until the  slower

integral  term responds to the error.  vsf_i is initially calculated from the

last pass value of VSF_I; then, if vsf_i is less than the  clip,  it  becomes

the new value.

DEFINITIONS

Registers:

-  BG_TMR = Background Time, sec.

-  ECT = Engine Coolant Temperature, deg F.

-  VSF_I = Last pass value of ECT integrator, level.

Bit flags:

-  CFMFLG = ECT failure mode flag; 1 -> Failure mode set.

Calibration Constants:

-  FN642(ect_error) = Proportional fan speed fraction adder for ECT, level.

-  VSF_DESECT = Desired ECT control point, deg F.

-  VSF_IK = Gain term for ect_error integrator, level / ECT * sec.

-  VSF_IMAX = Maximum value for integrator, Level (0-31).

16-21

VARIABLE SPEED FAN STRATEGY, ECT FAN CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EDF_VSF_ECT_COM1

ect_error := ECT - VSF_DESECT

vsf_i := VSF_I + [ect_error * VSF_IK * BG_TMR]

When the integrator reaches  full  output  its  value  is  frozen  until  the

temperature  drops  below  the  set  point.   This  keeps the integrator from

increasing when it is no longer in control.

vsf_i > VSF_IMAX ------------------| VSF_I := VSF_IMAX

|

| --- ELSE ----

|

vsf_i < 0 -------------------------| VSF_I := 0

|

| --- ELSE ----

|

| VSF_I := vsf_i

The final value for ECT control adds the proportional and integral terms  and

checks for a good ECT Sensor:

CFMFLG = 1 ------------------------| vsf_ect := 31

|

| --- ELSE ----

|

| vsf_ect := FN642(ect_error) + VSF_I

16-22

VARIABLE SPEED FAN STRATEGY, FINAL FAN DELIVERY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

16.1.7  FINAL FAN DELIVERY (CDAL0)

OVERVIEW

Using the VLCM module the fan voltage can be varied to control the  fan  RPM.

This  allows  closer control of ECT and A/C head pressure to their respective

desired values.  EEC determines the desired fan operation level (a value from

0  to  31,  proportional to voltage).  This value is then communicated to the

VLCM.

The final value for VSF_RQST_SPD is determined by chosing the higher value of

A/C  demanded  speed,  engine  coolant  temperature determined fan speed or a

value requested by Selftest, when applicable.

DEFINITIONS

Registers:

-  BG_TMR = Background Time, sec.

-  DEMAND_MODE = Flag set when an on-demand self test mode is active.

-  VSF = Last pass value of VSF, level.

-  VSF_AC = VSF requested by AC, speed no.

-  VSF_RQST_SPD = VSF requested speed.

-  VSF_WORD = VSF, increased resolution.

Bit Flags:

-  CRKFLG = Engine Mode Flag; 1 -> Crank Mode, 0 <> Crank Mode.

-  VSF_MIN_FLG = Flag indicating VSF  level  is  greater  than  the  minimum

required for the fan to turn.

Calibration Constants:

-  FN649(VSF) = Maximum allowable rate of VSF rise, steps/sec.

-  VSF_DECAY = Desired turn off rate, level/sec.

-  VSF_MIN_CL = Fan turn off point because of level below minimum, level.

-  VSF_MIN_SH = Minimum required for the fan to begin turning, level.

-  VSF_RUNNING = VSF level for Key On Engine Running.

16-23

VARIABLE SPEED FAN STRATEGY, FINAL FAN DELIVERY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EDF_VSF_FINAL_COM1

VSF_RQST_SPD requirements can come from engine coolant  temperature,  A/C  or

VSF_RUNNING.  This block selects the highest value for fan control.

CRKFLG = 1 --------------------------| vsf2 = 0

(in Crank)                          |

| --- ELSE ---

vsf_ect > VSF_AC --------------|     |

|AND -| vsf2 = vsf_ect

vsf_ect > VSF_RUNNING ---|     |     |

|OR --|     |

DEMAND_MODE = 0 ---------|           |

| --- ELSE ---

VSF_AC >= vsf_ect -------------|     |

|AND -| vsf2 = VSF_AC

VSF_AC > VSF_RUNNING ----|     |     |

|OR --|     |

DEMAND_MODE = 0 ---------|           |

| --- ELSE ---

|

| vsf2 = VSF_RUNNING

Because the fan will not begin turning below a certain voltage level, the EEC

will  not  turn  on until the fan level reaches VSF_MIN_SH, and will turn the

fan off when the level fall below VSF_MIN_CL.

vsf2 > VSF_MIN_SH -------------|S Q -| VSF_MIN_FLG

|

vsf2 < VSF_MIN_CL -------------|C

VSF_MIN_FLG = 1 ---------------------| no change to vsf2

|

| --- ELSE ---

|

| vsf2 = 0

16-24

VARIABLE SPEED FAN STRATEGY, FINAL FAN DELIVERY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The logic show below provides for soft  turn  on/off.   Separate  rising  and

falling rates are provided for differing on and off requirements.  The rising

rate is a function of the current fan speed (VSF) to give a  non-linear  ramp

rate.   This  section  uses  the  last pass value of VSF to determine the new

rate.

vsf2 - VSF_WORD

---------------- > FN649(VSF) -------| vsf1 = FN649(VSF) * BG_TMR + VSF_WORD

BG_TMR                          |

| --- ELSE ---

VSF_WORD - vsf2                     |

---------------- > VSF_DECAY --------| vsf1 = VSF_WORD - VSF_DECAY * BG_TMR

BG_TMR                          |

| --- ELSE ---

|

| vsf1 = vsf2

Because the proportional and integral terms can total more or less  than  the

allowable  limits  the  final  value  is  must be clipped so a valid value is

delivered.

vsf1 > 31 ---------------------------| VSF_RQST_SPD = 31

|

| --- ELSE ---

|

vsf1 < 0 ----------------------------| VSF_RQST_SPD = 0

|

| --- ELSE ---

|

| VSF_RQST_SPD = vsf1

16-25

VARIABLE SPEED FAN STRATEGY, VLCM OUTPUT SIGNAL PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

16.1.8  VLCM OUTPUT SIGNAL PROCESSING (CDAL0)

OVERVIEW

The purpose of this module is to provide a value of VSF  for  output  to  the

VLCM,  and  other  parts  of  the strategy.  In the case where this signal is

transferred via SCP link it checks on  the  requested  value  of  Fan  Speed,

VSF_RQST_SPD,  and  the  status of the link, to determine what the actual Fan

Speed will be.  Note that the two FMEM values, VSF_FMEM1  and  2,  should  be

calibrated  to  be  equal  to those that the VLCM will generate by default if

there is a fault in the link.

DEFINITIONS

Registers:

-  VSF = Variable speed fan level.

-  VSF_RQST_SPD = VSF requested speed.

-  VSF_WORD = VSF, increased resolution.

Bit flags:

-  ACRQST = A/C Requested; 1 -> request.

-  FANSCP_FMFLG =  Flag  indicating  that  the  engine  coolant  fan  status

messages are overdue (SCP link failure)

Calibration Constants:

-  VSF_FMEM1 = VSF FMEM speed for A/C off.

-  VSF_FMEM2 = VSF FMEM speed for A/C on.

-  VSFOP_HP = VSF output hardware present; 1 -> Hardware present.

16-26

VARIABLE SPEED FAN STRATEGY, VLCM OUTPUT SIGNAL PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EDF_VSF_OUTPUT_COM2

VSFOP_HP = 0 --------------------------|

;no VSF Output hardware               | VSF_WORD := 0

|

| --- ELSE ---

FANSCP_FMFLG = 1 ----------------|     |

;Comms link down                |AND -| VSF_WORD := VSF_FMEM1

|     |

ACRQST = 0 ----------------------|     |

;AC off                               |

| --- ELSE ---

FANSCP_FMFLG = 1 ----------------|     |

;Comms link down                |AND -| VSF_WORD := VSF_FMEM2

|     |

ACRQST = 1 ----------------------|     |

;AC on                                |

| --- ELSE ---

|

| VSF_WORD := VSF_RQST_SPD

always---------------------------------| VSF := VSF_WORD

16-27

ENGINE COOLANT FAN OUTPUT STATUS TEST, VLCM FAULT CODE PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

16.1.9  ENGINE COOLANT FAN OUTPUT STATUS TEST (CDAN0)

OVERVIEW

This module decodes the engine coolant fan output status information sent  to

the  EEEC  from  the VLCM via SCP.  Fault bits SCP_FAN_xxx are set by the SCP

module and  are  used  to  set  the  appropriate  malfunction  codes.   Fault

filtering  is  done  using  timers which increment when the fault is present.

When the timer exceeds a calibrated level the malfunction code is set.

DEFINITIONS

Registers:

-  FAN_ER_TMR1 = Engine coolant fan fault timer for fault P1484

-  FAN_ER_TMR2 = Engine coolant fan fault timer for fault P1625

-  FAN_ER_TMR3 = Engine coolant fan fault timer for fault P1473

-  FAN_ER_TMR4 = Engine coolant fan fault timer for fault P1483

-  VBAT = Battery voltage, volts.

Bit Flags:

-  CCM_EO_ENA = Engine off on demand test for CCM enabled.

-  CCM_ER_ENA = Engine running on demand test for CCM enabled.

-  CCM_TST_ENA = OBD-II Comprehensive  Component  Test  enable  flag;  1  ->

enabled.

-  CCM_TST_SW = CCM test present calibration switch; 1 -> test present.

-  FANSCP_FMFLG = Flag indicating that the Engine fan  status  messages  are

overdue (SCP link failure).

-  OBDII_RESET = Flag used to signal an OBDII reset.

-  SCP_FAN_GND = Engine coolant fan circuit shorted  to  ground;  0  ->  Not

shorted, 1 -> Shorted.

-  SCP_FAN_ON = Engine coolant fan output state; 0 -> Off, 1 -> On.

-  SCP_FAN_OPEN = Engine coolant fan circuit open; 0 -> Not open, 1 -> Open.

-  SCP_FAN_PGND = Engine coolant fan circuit open power  ground;  0  ->  Not

open, 1 -> Open.

-  SCP_FAN_OVB = Engine coolant fan circuit open Vbatt to output; 0  ->  Not

open, 1 -> Open.

16-28

ENGINE COOLANT FAN OUTPUT STATUS TEST, VLCM FAULT CODE PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  FAN_ER_TM1 = Time delay for fan fault P1484

-  FAN_ER_TM2 = Time delay for fan fault P1625

-  FAN_ER_TM3 = Time delay for fan fault P1473

-  FAN_ER_TM4 = Time delay for fan fault P1483

-  VBAT_CCM_MIN = Minimum voltage to perform CCM testing.

-  VLCM_HP = VLCM hardware present switch; 1 -> VLCM present.

Other:

-  P1473 = Fan secondary high with fan(s) off.

-  P1483 = Power to fan circuit overcurrent.

-  P1484 = Open power GND to VLCM.

-  P1625 = B(+) supply to VLCM fan circuit failure.

16-29

ENGINE COOLANT FAN OUTPUT STATUS TEST, VLCM FAULT CODE PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: FAN_VLCM_TST_COM1

CCM_EO_ENA = 1 -------------|

; Engine OFF CCM enabled    |

|

CCM_ER_ENA = 1 -------------|OR --|

; Engine ON CCM enabled     |     |

|     |

CCM_TST_ENA = 1 ------------|     |

; Continuous CCM enabled          |

|AND -| Do: FAN_VLCM_FAULTS

CCM_TST_SW = 1 -------------------|     | Do: SET_FAN_VLCM_CODES

; CCM test required               |     |

|     | ;Carry out VLCM testing

VBAT > VBAT_CCM_MIN --------------|     |

; Battery voltage sufficient      |     |

|     |

OBDII_RESET = 0 ------------------|     |

; Not a reset                     |     |

|     |

FANSCP_FMFLG = 0 -----------------|     |

; SCP link OK                     |     |

|     | --- ELSE ---

VLCM_HP = 1 ----------------------|     |

; hardware present                      | Clear_malf(P1484)

| Clear_malf(P1625)

| Clear_malf(P1473)

| Clear_malf(P1483)

|

| ; Hold timers to Zero

|

| FAN_ER_TMR1 := 0

| FAN_ER_TMR2 := 0

| FAN_ER_TMR3 := 0

| FAN_ER_TMR4 := 0

16-30

ENGINE COOLANT FAN OUTPUT STATUS TEST, VLCM FAULT CODE PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: FAN_VLCM_FAULTS

SCP_FAN_ON = 1 -------------------|

; Coolant fan ON                  |

|

SCP_FAN_OPEN = 0 -----------------|

; Fan circuit not open            |

|AND -| ;VLCM message L, DTC 1484

SCP_FAN_GND = 0 ------------------|     | ;Allow FAN_ER_TMR1 to inc

; Fan circuit not shorted to Gnd  |     |

|     | FAN_ER_TMR2 := 0

SCP_FAN_PGND = 1 -----------------|     | FAN_ER_TMR3 := 0

; Fan circuit open power ground   |     | FAN_ER_TMR4 := 0

|     |

SCP_FAN_OVB = 0 ------------------|     |

; Fan circuit not open VBAT to output   |

|

| --- ELSE ---

SCP_FAN_ON = 1 -------------------|     |

; Coolant fan ON                  |     |

|     |

SCP_FAN_OPEN = 0 -----------------|     |

; Fan circuit not open            |     |

|AND -| ;VLCM message M, DTC 1625

SCP_FAN_GND = 0 ------------------|     | ;Allow FAN_ER_TMR2 to inc

; Fan circuit not shorted to Gnd  |     |

|     | FAN_ER_TMR1 := 0

SCP_FAN_PGND = 0 -----------------|     | FAN_ER_TMR3 := 0

; Fan circuit not open power Gnd  |     | FAN_ER_TMR4 := 0

|     |

SCP_FAN_OVB = 1 ------------------|     |

; Fan circuit open VBAT to output       |

|

| --- ELSE ---

SCP_FAN_ON = 0 -------------------|     |

; Coolant fan Off                 |     |

|     |

SCP_FAN_OPEN = 1 -----------------|     |

; Fan circuit open                |     |

|AND -| ;VLCM message N, DTC 1473

SCP_FAN_GND = 0 ------------------|     | ;Allow FAN_ER_TMR3 to inc

; Fan circuit not shorted to Gnd  |     |

|     | FAN_ER_TMR1 := 0

SCP_FAN_PGND = 0 -----------------|     | FAN_ER_TMR2 := 0

; Fan circuit not open power Gnd  |     | FAN_ER_TMR4 := 0

|     |

SCP_FAN_OVB = 0 ------------------|     |

; circuit not open VBAT to output |     |

|     |

CCM_EO_ENA = 1 -------------------|     |

; Code 1473 in KOEO mode only           |

(Continued on next page)

16-31

ENGINE COOLANT FAN OUTPUT STATUS TEST, VLCM FAULT CODE PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(Continued from previous page)

| --- ELSE ---

SCP_FAN_ON = 1 -------------------|     |

; Coolant fan ON                  |     |

|     |

SCP_FAN_OPEN = 0 -----------------|     |

; Fan circuit not open            |     |

|AND -| ;VLCM message O, DTC 1483

SCP_FAN_GND = 1 ------------------|     | ;Allow FAN_ER_TMR4 to inc

; Fan circuit shorted to Gnd      |     |

|     | FAN_ER_TMR1 := 0

SCP_FAN_PGND = 0 -----------------|     | FAN_ER_TMR2 := 0

; Fan circuit not open power Gnd  |     | FAN_ER_TMR3 := 0

|     |

SCP_FAN_OVB = 0 ------------------|     |

; Fan circuit not open VBAT to output   |

|

| --- ELSE ---

|

| ; No errors hold timers

| ; to zero

| FAN_ER_TMR1 := 0

| FAN_ER_TMR2 := 0

| FAN_ER_TMR3 := 0

| FAN_ER_TMR4 := 0

END: FAN_VLCM_FAULTS

16-32

ENGINE COOLANT FAN OUTPUT STATUS TEST, VLCM FAULT CODE PROCESSING - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: SET_FAN_VLCM_CODES

FAN_ER_TMR1 > FAN_ER_TM1 ---------| Do: Store_code(P1484)

; VLCM message L present          | ; Fault L

; long enough                     | Do: Clear_malf(P1625)

| Do: Clear_malf(P1473)

| Do: Clear_malf(P1483)

|

| --- ELSE ---

FAN_ER_TMR2 > FAN_ER_TM2 ---------| Do: Store_code(P1625)

; VLCM message M present          | ; Fault M

; long enough                     | Do: Clear_malf(P1484)

| Do: Clear_malf(P1473)

| Do: Clear_malf(P1483)

|

| --- ELSE ---

FAN_ER_TMR3 > FAN_ER_TM3 ---------| Do: Store_code(P1473)

; VLCM message N present          | ; Fault N

; long enough                     | Do: Clear_malf(P1484)

| Do: Clear_malf(P1625)

| Do: Clear_malf(P1483)

|

| --- ELSE ---

FAN_ER_TMR4 > FAN_ER_TM4 ---------| Do: Store_code(P1483)

; VLCM message O present          | ; Fault O

; long enough                     | Do: Clear_malf(P1484)

| Do: Clear_malf(P1625)

| Do: Clear_malf(P1473)

|

| --- ELSE ---

|

| ; No errors yet, clear

| ; all codes

|

| Do: Clear_malf(P1484)

| Do: Clear_malf(P1625)

| Do: Clear_malf(P1473)

| Do: Clear_malf(P1483)

END: SET_FAN_VLCM_CODES

16-33

ELECTRO-DRIVE FAN STRATEGY, ELECTRO-DRIVE FAN OUTPUT STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

16.2  ELECTRO-DRIVE FAN OUTPUT STRATEGY (CDAN0)

OVERVIEW

This module is responsible for writing to the output locations  for  EDF  and

HEDF.

DEFINITIONS

INPUTS

Bit Flags:

-  HSF_FLG = Flag indicating the status of the High Speed Fan.

-  LSF_FLG = Flag indicating the status of the Low Speed Fan.

Calibration Constants:

-  EDFHP = Fan hardware present switch.

-  HEDFHP = Two speed fan output present.

-  LSF_INVRT_HP = LSF Inverting Output Hardware Present.

OUTPUTS

Bit Flags:

-  EDF_STATUS = EDF Smart Output Driver Fault Status.

-  HEDF_STATUS = HEDF Smart Output Driver Fault Status.

-  EDF = Flag for low-speed Electro-Drive Fan output pin bit.

-  HEDF = Flag for High-speed Electro-Drive Fan output pin bit.

16-34

ELECTRO-DRIVE FAN STRATEGY, ELECTRO-DRIVE FAN OUTPUT STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EDF_OUTPUT_COM1

EDFHP = 0 ------------------------------| do nothing

|

| --- ELSE ---

LSF_FLG = 1 ----------------|           |

;fan on                    |AND -|     |

|     |     |

LSF_INVRT_HP = 1 -----------|     |     |

;inverting output                |OR --| EDF := 0

|     | EDF_STATUS := output(EDF)

LSF_FLG = 0 ----------------|     |     |

;fan off                   |AND -|     |

|           |

LSF_INVRT_HP = 0 -----------|           |

;non-inverting output                  |

| --- ELSE ---

|

| EDF := 1

| EDF_STATUS := output(EDF)

HEDFHP = 0 -----------------------------| do nothing

|

| --- ELSE ---

|

HSF_FLG = 1 ----------------------------| HEDF := 1

| HEDF_STATUS := output(HEDF)

|

| --- ELSE ---

|

| HEDF := 0

| HEDF_STATUS := output(HEDF)

16-35

ELECTRO-DRIVE FAN STRATEGY, ELECTRO-DRIVE FAN OUTPUT STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

16-36

CHAPTER 17

THERMACTOR AIR STRATEGY

17-1

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

17.1  EAM AND OUTPUT STATE MONITOR CIRCUIT (MID LINE 2) (CDAN2)

OVERVIEW

Normal Operation:

The Electric Air Management (EAM) strategy controls an  On/Off  electric  air

pump.   The purpose of EAM is to control emissions during the first 20 to 120

seconds of  engine  operation.   By  forcing  excess  air  into  the  exhaust

manifold, Hydocarbons and Carbon Monoxide created by running rich after start

are oxidized.  This reaction will only occur when temperatures are hot enough

at  the  exhaust  port to start the reaction, so it may be desirable to delay

pump turn on for a few seconds until the  reaction  temperature  is  reached.

The  pump  is typically kept on until the catalyst is lit-off and can oxidize

and reduce at stoic without excess air.

At extremely hot engine and ACT it may be necessary to force open  loop.   In

addition,  upstream  air  will  force open loop due to its effect on the HEGO

sensor.  The Thermactor logic commands open loop during these conditions  via

the CHKAIR flag.

Self-Test:

The Self-Test for the EAM systems uses the messages sent by the output driver

(SSPOD,  etc.)  to diagnose problems with the primary circuit, an EAM Monitor

(EAMM) to diagnose problems with the secondary circuit, and a check to detect

if some secondary air flow is detectable when the pump is on.

The EAM Monitor (EAMM) (with an internal pull up) should be held low  by  the

low  resistance  path through the air pump motor when the pump is turned off.

If it is high, there is either an open circuit between the EAMM input to  the

EEC  and the pump motor ground, or there is power being supplied to the pump.

This failure will force open loop fuel operation for catalyst protection.  If

the  EAMM  is  low  when  the  pump  is commanded on, there is either an open

circuit between the battery and the EAMM connection to  the  circuit  or  the

relay has failed to close and supply power.

This flow test is enabled by the OBD-II  diagnostic  executive  after  it  is

determined  that  the  HEGOs  are functional and the engine is in closed loop

idle speed control.  This flow test is unable to  detect  a  particular  flow

rate or a pump that is on when it should be off.  It is intended to determine

if some flow can be detected when the pump is on.  This is done  by  assuming

that  if  the  HEGO  sensor is not stuck rich for an extended period of time,

there must be some secondary air available to make it switch lean.  It should

be  noted  that  even  if  adequate  secondary  air  is available to make the

aggregate air fuel ratio  in  the  exhaust  lean,  the  EGO  sensor  may  not

consistently provide a lean indication.

17-2

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Engine Off On-Demand testing:

During the Key On Engine Off Self-Test,  the  sequencer  controls  the  flags

OSM_EO_ON  and  OSM_EO_OFF  to signal the EAM system to command the output on

and off.  The state of the EAMM circuit  is  checked  along  with  diagnostic

messages from the output driver to determine if there is an electrical fault.

Engine Running On-Demand testing:

During the Key On Engine Running Self-Test, the sequencer sets the  parameter

ER_STATUS  is  set  to  SAIR_ER_INIT to indicate to the EAM system that it is

time to initiate the engine running test.  This test first ramps the fuel  to

achieve a slightly rich condition.  Then, the air pump is commanded on.  With

the pump on, the EAMM monitor and output driver are checked,  and  the  HEGOs

are  monitored  to  see  if  flow can be detected (via normal continuous test

logic).  After testing with the pump on is complete, the pump is  turned  off

and  again,  the  driver and EAMM monitor are checked.  Note that as the test

moves through the sequence, DEMAND_STATE is modified to point to the  current

logic  in use, and when the test is complete ER_STATUS is set to SAIR_ER_DONE

to indicate to the sequencer that the EAM test is complete.

************

..*..        *    g

B+ ********x**********************x***            ..*..        *    r

*                      *  *          ..  *  ..    -----  o

...*...              .....*..*....     .   PUMP  .    ---   u

CHECK !  )  !              !    )  *   !      ..  *  ..      -    n

VALVE ! (   !       RELAY  !   (    |  !        ..*..             d

+   !  )  !              !    )  * | !        ..*..

SOLENOID ! (   !              !   (   *   !          *

!..*..!              !....*..*...!          *

*                      *  ********x*******

*                      *          *

*                      *          *

**x*********************          *

*                               *

*                               *

......*...............................*.......................

!     *                               *                      !

!   driver                            *                      !

!  (SSPOD ect.)                     EAMM                     !

!(with diagnostics)              (Digital input,             !

!     *                           Pulled up)                 !

!   -----          ECU                                       !

!    ---          (EEC-IV)                                   !

!.....-......................................................!

17-3

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  ACT = Air Charge Temperature, deg F.

-  ATMR1 = Time since Engine Startup, sec.

-  BIAS_G1 = Closed loop fuel bias for bank 1.

-  BIAS_G2 = Closed loop fuel bias for bank 2.

-  DEMAND_STATE = Utility state pinter that  is  used  by  on  demand  OBDII

tests.

-  DEMAND_TIMER = VIP state timer.

-  EAM_ANTP_TMR = Used to time the delay in air  pump  activation  to  allow

idle speed load anticipation, seconds.

-  EAMM_TST_TMR = Consecutive amount  of  time  that  EAMM  has  no  errors,

seconds.

-  ECT = Engine Coolant Temperature, deg F.

-  ER_EAM_LEAN = LAMBSE at which EGOS switched lean during EAM test.

-  ER_LAM_DSD1 = LAMBSE1 value requested by engine running on-demand test.

-  ER_LAM_DSD2 = LAMBSE2 value requested by engine running on-demand test.

-  ER_RPM = Register that contains RPM value requested during engine running

self test.

-  ER_STATUS = State pointer that indicates current state of engine  running

on demand test.

|      -  INFAMB_KAM = Inferred ambient air temperature,  used  to  initialize  the

|         cat.  temp.  model.

-  ISCFLG = Mode indicator flag.

-  j1979_01_12 = Bitmap of commanded status of secondary air system.  Bit  2

set  when  air is dumped / off, Bit 1 is set for Downstream air, Bit 0 is

set for Upstream air.

-  LAMAVE1 = Average of LAMBSE1:  averages  the  value  of  LAMBSE1  at  the

current EGO-1 switch and last EGO-1 switch.

-  LAMAVE2 = Average of LAMBSE2:  averages  the  value  of  LAMBSE2  at  the

current EGO-2 switch and last EGO-2 switch.

-  LAMBSE1 = Desired A/F ratio for bank 1 injectors in equivalence units.

-  LAMBSE2 = LAMBDA equivalence ratio (EGO-2).

-  PG_DC = Purge duty cycle.

17-4

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  SAIR_END_FLG = Flag used to delay  ending  of  SAIR  test  when  test  is

aborted because engine idle is exited.

-  SAIR_END_TMR = Timer to delay the ending of the SAIR monitor,  allow  EAM

pump time to spin down.

-  SAIR_HLD_TMR = Amount of time LAMBSE at SAIR_REQV limit.

-  SAIR_LAM_DSD = Required lambse value for continuous secondary air test.

-  SAIR_LN_TMR = Time that both egos are lean in secondary air flow test.

-  SAIR_PRM_TMR = Consecutive amount of time that EAM has no errors.

-  SAIR_RCH_TMR = Time that both egos must be rich to start continuous  flow

test.

-  SAIR_TMRK1 = KAM  snapshot  of  SAIR_LN_TMR  at  completion  of  upstream

thermactor flow test, sec.

-  SAIR_TMRK2 = KAM snapshot of  SAIR_LN_TMR  at  completion  of  downstream

thermactor flow test, sec.

-  SUBST_REQ1 = Substitution Requested flag for channel 1.

-  TCSTRT = Engine coolant temperature at startup, deg F.

-  USPD_RUN_TMR = Time in underspeed or run mode, sec.

-  V_AP_TSTTMR = Time limit for Air pump hot retest, sec.

-  V_EAMM_TMR = Time that EAM monitor (EAMM) error.

-  V_THERM_TMR = Time EAM circuit failure error detected.

-  VEGO11 = Bank1 upstream HEGO voltage.

-  VEGO11_FIL1 = First order filtered value of bank1 upstream HEGO voltage.

-  VEGO11_FIL2 = Second order filtered value of bank1 upstream HEGO voltage.

-  VEGO12_FIL2 = Second  order  filtered  value  of  bank1  downstream  HEGO

voltage.

-  VEGO21 = Bank2 upstream HEGO voltage.

-  VEGO21_FIL1 = First order filtered value of bank2 upstream HEGO voltage.

-  VEGO21_FIL2 = Second order filtered value of bank1 upstream HEGO voltage.

-

Bit Flags:

-  CCM_ER_ENA = OBD-II Comprehensive Component Engine Running On Demand Test

enable flag; 1 -> test enabled.

17-5

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  CCM_TST_ENA = OBD-II Comprehensive Component Continuous Test enable flag;

1 -> test enabled.

-  CHKAIR = Thermactor forced open loop flag.

-  DEMAND_ABORT = Flag to indicate that the engine running on demand test is

in the process of an abort.

-  EAM = Electric Air Management control flag, 1 -> Pump On.

-  EAM_ANTICIP = Requests additional idle air flow when air pump is on.

-  EAM_FAULT = Secondary Air Injection (AIR) output fault indicated.

-  EAMM = State of EAM Monitor digital input.  1 -> high (pump on).

-  EAM_OLD = Last pass value of EAM.

-  ER_ACTIVE = Flag set when engine running on demand test is active.

-  FFG_SAIR = OBDII system FMEM flag for the sair system; 1 -> sair sytem is

not currently under control reliably.

-  LN_CTL_FLG = Control actuator for SAIR_LN_TMR, 1 = increment timer.

-  OBD_PARM_RST = OBDII paramter reset signal; 1-> reset parameters.

-  OSM_EO_OFF = Off state requested for outputs during engine off on  demand

test, 1 -> test off state.

-  OSM_EO_ON = On state requested for outputs during engine off  test  1  ->

test on state.

-  OVERTEMP_FLG = Overtemp flag to force open loop and bypass air.

-  PSPN_FLWTST = Postpone continuous flow until next idle period.

-  PxxxMALF = Malfunction flag for code PXXX:  1 -> a malfunction  currently

exists for fault PXXX.

-  SAIR_END_TM = Maximum time to delay ending of SAIR monitor test  to  wait

for EAM pump to spin down.

-  SAIR_EGO_LN = A lean state has been seen on both egos  in  the  secondary

air test.

-  SAIR_EGO_RC = A rich state has been seen on both sego  in  the  secondary

air test.

-  SAIR_ER_NOW = Indicates that engine running on demand secondary air  test

is in progress.

-  SAIR_MON = OBD-II Monitor flag for secondary air:  1 -> all secondary air

faults have been monitored at least once since powerup.

-  SAIR_OFF_MON = Secondary air EAMM off state monitored.

17-6

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  SAIR_ON_MON = Secondary air on state monitored.

-  SAIR_PRM_FLG =  SAIR  KOER  primary  circuit  test  complete  flag,  1  =

complete.

-  SAIR_TST_ENA = EAM flow test enabled by OBD-II diagnostic monitor.

-  SAIR_TST_RDY = Secondary air test ready flag.  1 =>  local  condition  to

test have been met; ready to run test.

Calibration Constants:

-  ATBYS = Minimum ACT to Bypass thermactor Air, deg F.

-  ATBYS_CL = ACT to reset (clear) Overtemp condition, deg F.

-  CTBYS = ECT to set overtemp condition, deg F.

-  CTBYS_CL = ECT to reset (clear) Overtemp condition, deg F.

-  EAM_ANTP_DLY = Time delay to allow idle speed  load  compensation  before

activating the air pump.

-  EAM_LAM_DEL = Delta LAMAVE to allow air pump hot retest.

-  EAMM_TST_TM = Minimum amount of time EAMM must have no  errors  to  pass,

sec.

-  ER_DELTA_RPM = DELTA RPM TO RAMP SAIR_ER_RPM DURING SECOND AIR KOER TEST.

-  ER_LAM_BAS = Base  value  for  LAMBSE  requested  during  engine  running

on-demand test.

-  FN606(TCSTRT) = Time to delay upstream air after startup, sec.

-  FN607(TCSTRT) = Time to provide upstream air after start, sec.

-  LEQV = Lean limit for LAMBDA units.

-  NUMEGO = Number of HEGOs in the system.

-  REQV = Rich limit for LAMBDA units are LAMBDAS.

|      -  SAIR_AMB_MIN = Minimum ambient temperature to disable setting SAIR  fault

|         codes because of component freezing.

-  SAIR_DLAM = Delta LAMBSE value used  to  ramp  fuel  rich  in  continuous

second air test.

-  SAIR_EGO_LVL = Voltage on EGO sensor indicating lean/rich boundry.

-  SAIR_ER_DONE = Value for state of ER_STATUS to indicate that  the  engine

running secondary air test is complete.

-  SAIR_ER_INIT = Value for state of ER_STATUS to request the engine running

secondary air test.

17-7

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  SAIR_ER_RPM = Engine RPM to be used during secondary air KOER test.

-  SAIR_HOLD_TM = Maximum amount of time to hold LAMBSE at SAIR_REQV  before

exiting continuous second air test.

-  SAIR_LAM_DEL = Delta LAMBSE for secondary air pump test.

-  SAIR_LEQV = Leanest allowable LAMBSE value to start continuous flow test.

-  SAIR_LN_TM = Total amount of time that both egos must  be  lean  to  pass

thermactor flow test.

-  SAIR_PG_MIN = Minimum purge duty cycle to start continuous flow test.

-  SAIR_PRM_TM = Minimum amount of time primary circuit must have no  errors

to pass.

-  SAIR_RAMP_TM = Time for hegos to detect switch in fuel initialization.

-  SAIR_RCH_TM =  Minimum  amount  of  time  EGOs  must  be  rich  to  start

continuous flow test.

-  SAIR_REQV = Richest allowable LAMBSE value  to  conduct  continuous  flow

test.

-  SAIR_STB_TM = Minimum time to wait to run secondary air monitor.

-  SAIR_VIPLR1 = Ramp rate for fuel lean.

-  SAIR_VIPRR1 = Ramp rate for fuel rich.

-  TCVEGO_SAIR = Filter constant for upstream hego voltages.

-  TEMPFB = ECT at which closed loop fuel is allowed.

-  THRMHP = Thermactor hardware present switch; 0 -> no thermactor.

-  V_AP_TST_TM = Time duration to allow hot retest of EAM system.

-  V_EAMM_ENA = Switch to enable  EAMM  (Electric  Air  Management  Monitor)

tests (KOEO, KOER and continuous).

-  V_EAMM_TM = Minimum time with EAM monitor (EAMM secs).

-  VIPLR1 = Rate of ramp lean units are LAMBDAS/sec.

-  VIPRR1 = Ramp rate for fuel rich.

-  V_THERM_TM = Time required to set code for EAM circuit failure.

OTHER

-  malfunction(sair,Pxxx) = A public procedure for updating the MIL  control

process provided by the MIL control module.

17-8

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  sair_codes = set of {P0412,P0411,P1413,P1414}

-  P0411 = Secondary air not detected.

-  P0412 = EAM primary circuit malfunction

-  P1413 = EAM Monitor (EAMM) low with pump commanded on.

-  P1414 = EAM Monitor (EAMM) high with pump commanded off.

17-9

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: THERM_EAM_OBDII_COM4

BEGIN:  therm_main

Execute the following process once per background pass.

Set up some parameter definitions to allow use of symbolic names.

unconditionally -------------------| sair_koer_primary_test := 1

| sair_koer_fuel_init := 2

| sair_koer_fuel_done := 3

ECT, ACT overtemp flag to force open loop and bypass air.

ECT > CTBYS -----------|

|AND -|S Q -| OVERTEMP_FLG

ACT > ATBYS -----------|     |

|

ECT < CTBYS_CL --------|     |

|OR --|C

ACT < ATBYS_CL --------|

Calculate second ordered filtered value of ego voltage for use  in  the  flow

test.

SAIR_TST_RDY = 1 -|

|OR --|

ER_STATUS =       |     |AND -| VEGO11_FIL1 := rolav(VEGO11,TCVEGO_SAIR)

SAIR_ER_INIT --|     |     | VEGO11_FIL2 := rolav(VEGO11_FIL1,TCVEGO_SAIR)

|     |

NUMEGO = 1 -------------|     | VEGO21_FIL2 := FFFFh

| VEGO12_FIL2 := FFFFh

|  ;fill to max value for mode 06 reporting

|

| --- ELSE ---

SAIR_TST_RDY = 1 -------|     |

|OR --| VEGO11_FIL1 := rolav(VEGO11,TCVEGO_SAIR)

ER_STATUS =             |     | VEGO11_FIL2 := rolav(VEGO11_FIL1,TCVEGO_SAIR)

SAIR_ER_INIT --------|     | VEGO21_FIL1 := rolav(VEGO21,TCVEGO_SAIR)

| VEGO21_FIL2 := rolav(VEGO21_FIL1,TCVEGO_SAIR)

|

| VEGO12_FIL2 := FFFFh

|  ;fill to max value for mode 06 reporting

17-10

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

When an OBDII parameter reset is requested, clear all necessary parameters.

OBD_PARM_RST = 1 ------------| SAIR_ON_MON := 0

| SAIR_OFF_MON := 0

| PSPN_FLWTST := 0

| SAIR_ER_NOW := 0

| V_THERM_TMR := 0

| V_EAMM_TMR := 0

| SAIR_HLD_TMR := 0

| SAIR_FLWTST := 0

| LN_CTL_FLG := 0

| SAIR_TST_RDY := 0

| V_AP_TSTTMR := 0

| VEGO11_FIL2 := 0

| VEGO21_FIL2 := 0

| VEGO12_FIL2 := FFFFh

| SAIR_TMRK1 := 0

| SAIR_TMRK2 := FFh

PSPN_FLWTST = 1 -------------|

|AND -| PSPN_FLWTST := 0

ISCFLG = 0 ------------------|

17-11

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

THRMHP = 0 ------------------------| Do: therm_no_hardware

| Exit therm_main

|

| --- ELSE ---

|

ER_STATUS = SAIR_ER_INIT ----------| Do: therm_koer

|

| --- ELSE ---

|

EO_STATUS = ECU_EO_DONE -----------| V_EAMM_TMR := 0

;on demand about to be requested  | V_THERM_TMR := 0

|  ;Clear EAM fault timers before KOEO

|

| --- ELSE ---

|

OSM_EO_ON = 1 ---------------------| eam := 1

;on demand on state requested     |  ;EAM engine off test

|

| --- ELSE ---

|

OSM_EO_OFF = 1 --------------------| eam := 0

;on demand off test requested     |  ;EAM KOEO test

|

| --- ELSE ---

|

| Do: therm_normal

Perform Output State Control logic:

unconditionally -------------------| Do: substitute(1,eam)

|  ;output state control override

| EAM := eam

If EAM state changes occurs, zero the fault timers.

EAM <> EAM_OLD --------------------| V_EAMM_TMR := 0

| V_THERM_TMR := 0

|  ;Clear fault timers

unconditionally -------------------| EAM_OLD := EAM

|  ;Save last pass value of EAM

ER_STATUS = SAIR_ER_INIT ----|

|AND -| Do: therm_koer_circuit_test

DEMAND_STATE =               |     |

sair_koer_primary_test ----|     |

| --- ELSE ---

|

EAM = 1 ---------------------------| Do: therm_primary_test

| Do: therm_commanded_on_test

|

| --- ELSE ---

|

| Do: therm_primary_test

| Do: therm_commanded_off_test

17-12

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The flags SAIR_ON_MON, and SAIR_OFF_MON are set when  the  pump  on  and  off

tests  have  been  completed  (pass or fail).  When they are all set the flag

SAIR_MON is set to indicate that Thermactor monitoring has been completed for

this trip.

SAIR_ON_MON = 1 --------------|

;on test complete            |

|

SAIR_END_TMR > SAIR_END_TM ---|AND -|

;delay ending of SAIR test   |     |

|     |

SAIR_OFF_MON = 1 -------------|     |

|OR --| SAIR_MON := 1

P0412MALF = 1 ----------------|     |     | ;secondary air monitoring

|     |     | ;  complete for this trip

P1413MALF = 1 ----------------|OR --|

|

P1414MALF = 1 ----------------|

;off test complete

BEGIN: therm_pid_definitions

EAM = 0 ---------------------------| pid_def( j1979_01_12, 4)

|  ;air off

|

| --- ELSE ---

|

| pid_def( j1979_01_12, 1)

|  ;air on

END: therm_pid_definitions

END: therm_main

17-13

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: therm_no_hardware

Execute the following process only when called.

SUBST_REQ1 = 1 --------------------| Do: OSC_RESPONSE(1,10h)

|  ;OSC requested but hardware not present

|  ; send general reject.

ER_STATUS = SAIR_ER_INIT ----------| ER_STATUS := SAIR_ER_DONE

;engine running test in           | Exit therm_no_hardware

; progress                        |

| --- ELSE ---

|

OVERTEMP_FLG = 0 ------------------| CHKAIR := 1

;not overtemp allow closed loop   | Exit therm_no_hardware

|

| --- ELSE ---

|

| CHKAIR := 0

|  ;Overtemp, force open loop

| Exit therm_no_hardware

END: therm_no_hardware

17-14

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: therm_koer

Execute the following process only when called.

SAIR_ER_NOW = 0 -------------------| DEMAND_STATE := sair_koer_primary_test

;test not started yet             |  ;clear all counters needed in KOER

|

| DEMAND_TIMER := 0

|  ;restart test timer

|

| LN_CTL_FLG := 0

| SAIR_ON_MON := 0

| SAIR_OFF_MON := 0

|  ;Reset flags used to determine when

|  ; test complete.

| SAIR_EGO_LN := 0

| SAIR_EGO_RC := 0

| SAIR_LN_TMR := 0

|  ;flags used to set up fuel during on

|  ; demand test fuel ramp

| SAIR_PRM_FLG := 0

| SAIR_PRM_TMR := 0

| EAMM_TST_TMR := 0

| V_THERM_TMR := 0

| V_EAMM_TMR := 0

|

| SAIR_ER_NOW := 1

|  ;test in progress

|

| eam := 0

DEMAND_ABORT = 1 ------------------| eam := 0

| ER_STATUS := SAIR_ER_DONE

|  ;signal executive that abort is

|  ; complete

|

| --- ELSE ---

DEMAND_STATE =                     |

sair_koer_primary_test ----|     |

|AND -|

DEMAND_TIMER <= SAIR_PRM_TM -|     | SAIR_LN_TMR := 0

| ER_RPM := max(ER_RPM - ER_DELTA_RPM,

|                SAIR_ER_RPM)

|

| --- ELSE ---

DEMAND_STATE =                     |

sair_koer_primary_test ----|     |

|     |

SAIR_PRM_FLG = 0 ------------|AND -| malfunction(sair,P0412)

|     | ER_STATUS := SAIR_ER_DONE

EAMM = 1 --------------------|     |

;Monitor indicates pump on        | FFG_SAIR := 1

|

| --- ELSE ---

(continued on next page)

17-15

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

DEMAND_STATE =                     |

sair_koer_primary_test ----|     |

|AND -| malfunction(sair,P0412)

SAIR_PRM_FLG = 0 ------------|     | ER_STATUS := SAIR_ER_DONE

|

|

| --- ELSE ---

DEMAND_STATE =                     |

sair_koer_primary_test ----------| malfunction(sair,P1414)

| ER_STATUS := SAIR_ER_DONE

|

| --- ELSE ---

DEMAND_STATE =                     |

sair_koer_fuel_init -------------| Do: therm_fuel_init

|  ;ramp each side lean until the HEGO

|  ; switches, then ramp fuel until

|  ; slightly rich.

|  ;DEMAND_STATE will then be

|  ; incremented to sair_koer_fuel_done

|

| --- ELSE ---

DEMAND_STATE =                     |

sair_koer_fuel_done -------|     |

|AND -| EAM_ANTICIP := 1

EAM_ANTICIP = 0 -------------|     | EAM_ANTP_TMR := 0

|

| --- ELSE ---

DEMAND_STATE =                     |

sair_koer_fuel_done -------|     |

|AND -| ;wait for idle speed compensation

EAM_ANTP_TMR < EAM_ANTP_DLY -|     |

| --- ELSE ---

DEMAND_STATE =                     |

sair_koer_fuel_done -------|     |

|     |

SAIR_ON_MON = 0 -------------|AND -|  ;EAM Engine Running test

|     | eam := 1

EAM := 0 --------------------|     | SAIR_PRM_TMR := 0

| V_THERM_TMR := 0

| SAIR_PRM_FLG := 0

| V_EAMM_TMR := 0

| EAMM_TST_TMR := 0

| V_AP_TSTTMR := 0

| Do: therm_flow_test

|  ;Test EAMM and check for flow

|

| --- ELSE ---

DEMAND_STATE =                     |

sair_koer_fuel_done -------|     |

|AND -| ;EAM Engine Running test

SAIR_ON_MON = 0 -------------|     | Do: therm_flow_test

;monitor not complete             |

| --- ELSE ---

(continued on next page)

17-16

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

DEMAND_STATE =                     |

sair_koer_fuel_done -------|     |

|     |

SAIR_ON_MON = 1 -------------|AND -| ER_STATUS := SAIR_ER_DONE

|     |  ;signal executive that

SAIR_OFF_MON = 1 ------------|     |  ; koer thermactor test is complete

ER_STATUS = SAIR_ER_DONE ----------| SAIR_ER_NOW := 0

| ER_LAM_DSD1 := ER_LAM_BAS

| ER_LAM_DSD2 := ER_LAM_DSD1

END: therm_koer

17-17

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: therm_normal

Execute the following process only when called.

OVERTEMP_FLG = 1 -----------------|

|

EAM = 0 --------------------|     |     | ; START FAILURE MODE

;Pump commanmed off        |     |OR --| eam := 0

|     |     | CHKAIR := 0

EAMM = 1 -------------------|     |     | ;over temperature or EAMM failure

;EAMM indicates pump       |     |     | ; force Open Loop and turn off air

; on when it is            |     |     |

; commanded off            |AND -|     | SAIR_TST_RDY := 0

|           | EAM_ANTICIP := 0

SAIR_END_TMR >              |           |

SAIR_END_TM ------------|           |

;Pump should have stopped  |           |

|           |

V_EAMM_ENA = 1 -------------|           |

;EAMM Test enabled                     |

| --- ELSE ---

|

ER_ACTIVE = 1 --------------------------| eam := 0

| EAM_ANTICIP := 0

| SAIR_TST_RDY : 0

| ; test unable to execute the idle

| ; period wait until next idle

|

| --- ELSE ---

ATMR1 >= FN606(TCSTRT) -                |

EAM_ANTP_DLY ---|           |

;pump will turn on in      |           |

;EAM_ANTP_DLY seconds.     |AND -|     |

|     |     |

ATMR1 <= FN606(TCSTRT) -----|     |     |

|OR --| eam := 0

ATMR1 > FN606(TCSTRT) ------|     |     | EAM_ANTICIP := 1

|     |     | CHKAIR = 1

ATMR1 < FN607(TCSTRT) ------|     |     |  ;allow closed loop

|AND -|     | SAIR_TST_RDY = 0

EAM =  0 -------------------|           |

|           |

EAM_FAULT <> 0 -------------|           |

| --- ELSE ---

ATMR1 > FN606(TCSTRT) ------------|     |

|AND -| eam := 1

ATMR1 < FN607(TCSTRT) ------------|     | CHKAIR := 0

| SAIR_TST_RDY = 0

| EAM_ANTICIP := 1

| ;maintain EAM load correction

| ;in ISC

|

| --- ELSE ---

(continued on next page)

17-18

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|

PSPN_FLWTST = 1 ------------------------| eam := 0

| EAM_ANTICIP := 0

| SAIR_TST_RDY = 0

| ; test unable to execute the idle

| ; period wait until next idle

|

| --- ELSE ---

SAIR_ON_MON = 0 ------------------|     |

;Test not yet complete           |     |

|     |

ISCFLG = 1 -----------------------|     |

;at idle                         |     |

|     |

SAIR_TST_ENA = 0 -----------------|     |

;monitor not yet enabled         |     |

|     | ; entry conditions to enable

abs(1. + BIAS_G1 - LAMAVE1)       |     | ; continuous flow test

< EAM_LAM_DEL ---------|     |

;adaptive learned                |AND -| SAIR_FLWTST := 0

; enough to let lambse           |     |

; be close to 1                  |     | SAIR_TST_RDY := 1

|     | ;signal OBD-II executive that test

NUMEGO = 1 -----------------|     |     | ; is ready to run

|     |     |

NUMEGO = 2 -----------|     |OR --|     | CHKAIR := 1

|     |     |     |  ;allow closed loop

abs(1. + BIAS_G2 -    |AND -|     |     |

LAMAVE2) <       |           |     | eam := 0

EAM_LAM_DEL ------|           |     |

|     | EAM_ANTICIP := 0

ECT > TEMPFB ---------------------|     | EAM_ANTP_TMR := 0

;Warm engine                     |     |

|     | SAIR_LAM_DSD :=

USPD_RUN_TMR > SAIR_STB_TM -------|     |         min(LAMBSE1,LAMBSE2)

;delay SAIR OBD-II test                | SAIR_LAM_DSD :=

|         max(SAIR_REQV,SAIR_LAM_DSD)

|

| --- ELSE ---

|

SAIR_TST_RDY = 1 -----------------|     |  ;no longer at idle, start delay

|     |

ISCFLG = 0 -----------------------|AND -| CHKAIR = 1

;not at idle                     |     |  ;allow closed loop

|     |

SAIR_END_FLG = 0 -----------------|     | EAM_ANTICIP := 0

| eam := 0

| SAIR_END_TMR := 0

| SAIR_END_FLG := 1

|

| --- ELSE ---

(continued on next page)

17-19

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

SAIR_END_FLG = 1 -----------------|

|AND -| NO ACTION

SAIR_END_TMR <= SAIR_END_TM ------|     |  ;Wait until timer expires

|

| --- ELSE ---

|

SAIR_END_FLG = 1 -----------------------| SAIR_TST_RDY := 0

| SAIR_END_FLG := 0

| CHKAIR := 1

| EAM_ANTICIP := 0

| eam := 0

|

| --- ELSE ---

SAIR_TST_RDY = 1 -----------------|     |

|     |

SAIR_ON_MON = 1 ------------------|AND -| eam := 0

|     |  ;Test complete, turn off pump

SAIR_END_TMR > SAIR_END_TM -------|     | CHKAIR = 1

|  ;allow closed loop

| SAIR_TST_RDY := 0

| EAM_ANTICIP := 0

|

| --- ELSE ---

SAIR_TST_RDY = 1 -----------------|     |

|AND -| eam := 0

SAIR_ON_MON = 1 ------------------|     |  ;Test complete, turn off pump

| CHKAIR = 1

|  ;allow closed loop

|

| --- ELSE ---

|

SAIR_TST_ENA = 1 -----------------|     |  ;delay until idle speed

;test enabled by exec            |     |  ; can compensate air pump

|     |  ; and purge is off

EAM_ANTICIP = 0 ------------|     |AND -| eam := 0

|     |     | CHKAIR := 1

EAM_ANTP_TMR <              |     |     |  ;allow closed loop

EAM_ANTP_DLY -----|OR --|     | SAIR_HLD_TMR := 0

;not enough time           |           | SAIR_RCH_TMR := 0

; for ISC to react         |           | EAM_ANTICIP := 1

|           | SAIR_LAM_DSD :=

PG_DC > SAIR_PG_MIN --------|           |         min(LAMBSE1,LAMBSE2)

| SAIR_LAM_DSD :=

|         max(SAIR_REQV,SAIR_LAM_DSD)

|

| --- ELSE ---

(continued on next page)

17-20

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

SAIR_TST_ENA = 1 -----------------|     |

|AND -| Do: therm_fuel_init

SAIR_FLWTST = 0 ------------------|     |

| --- ELSE ---

|

SAIR_TST_ENA = 1 -----------------------|  ;conduct flow test

;test enabled by exec                  | eam := 1

| CHKAIR := 0

|  ;force open loop

| Do: therm_flow_test

|

| --- ELSE ---

|

| ;***** DEFAULT - PUMP OFF *****

|

| eam := 0

| CHKAIR := 1

| SAIR_TST_RDY := 0

|  ;signal OBD-II executive that test

|  ; is off

| EAM_ANTICIP := 0

END: therm_normal

17-21

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: therm_primary_test

Execute the following process only when called.

The Output State Monitor is tested  to  insure  that  it  is  indicating  the

correct state.  If it is incorrect for V_THERM_TM seconds, code P0412 is set.

EAM_FAULT = 0 ---------------------| V_THERM_TMR = 0

;primary monitor indicates        |  ;No error, hold timer at zero

; correct state                   |

| --- ELSE ---

|

V_THERM_TMR > V_THERM_TM ----|     |

;a fault has been present   |     |

; long enough               |     |

|     |

EAMM = 1 --------------------|AND -| malfunction(sair,P0412)

;Monitor indicates pump on  |     |  ;ECA failure or short across relay,

|     |  ; set code P0412

OSM_EO_ON = 1 ---------|     |     |

;KOEO test enabled    |     |     | SAIR_ON_MON := 1

|     |     |  ;monitor complete, failure detected

OSM_EO_OFF = 1 --------|     |     |

;KOEO off test ena.   |     |     | FFG_SAIR := 1

|     |     |  ;Signal OBD-II exectutive that

CCM_ER_ENA = 1 --------|OR --|     |  ; thermactor air is flowing

;KOER test enabled    |           |

|           |

CCM_TST_ENA = 1 -------|           |

;Continuous test ena.             |

| --- ELSE ---

|

V_THERM_TMR > V_THERM_TM ----|     |

;a fault has been present   |     |

; long enough               |     |

|AND -| malfunction(sair,P0412)

OSM_EO_ON = 1 ---------|     |     |  ;ECA failure or short across relay,

;KOEO on test enabled |     |     |  ; set code P0412

|     |     |

OSM_EO_OFF = 1 --------|     |     | SAIR_ON_MON := 1

;KOEO off test ena.   |     |     |  ;monitor complete, failure detected

|     |     |

CCM_ER_ENA = 1 --------|OR --|     |

;KOER test enabled    |           |

|           |

CCM_TST_ENA = 1 -------|           |

;Continuous test ena.             |

| --- ELSE ---

|

| No Action.

END: therm_primary_test

17-22

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: therm_koer_circuit_test

Execute the following process only when called.

The Output State Monitor is tested  to  insure  that  it  is  indicating  the

correct state.  If it is incorrect for V_THERM_TM seconds, code P0412 is set.

EAM_FAULT = 0 ---------------|

;primary monitor indicates  |

; correct state             |AND -| V_THERM_TMR := 0

|     |  ;no error, hold timer at zero

SAIR_PRM_TMR < SAIR_PRM_TM --|     |

| --- ELSE ---

|

EAM_FAULT = 0 ---------------------| V_THERM_TMR := 0

| clear_malf(P0412)

| SAIR_PRM_FLG := 1

|

| --- ELSE ---

|

V_THERM_TMR > V_THERM_TM ----|     |

;a fault has been present  |     |

; long enough              |AND -| malfunction(sair,P0412)

|     |  ;ECA failure or short across relay,

EAMM = 0 --------------------|     |  ; set code P0412

;Monitor indicates pump off       |

| SAIR_ON_MON := 1

|  ;monitor complete, failure detected

|

| ER_STATUS := SAIR_ER_DONE

|

| --- ELSE ---

V_THERM_TMR > V_THERM_TM ----|     |

;a fault has been present  |     |

; long enough              |AND -| malfunction(sair,P0412)

|     | FFG_SAIR := 1

EAMM = 1 --------------------|     |  ;Signal OBD-II exectutive that

;Monitor indicates pump on        |  ; thermactor air is flowing

|

| SAIR_ON_MON := 1

|  ;monitor complete, failure detected

|

| ER_STATUS := SAIR_ER_DONE

|

| --- ELSE ---

|

| SAIR_PRM_TMR := 0

17-23

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EAMM = EAM ------------------|

;secondary monitor shows    |

; correct state             |

|AND -| V_EAMM_TMR := 0

EAMM_TST_TMR < EAMM_TST_TM --|     |  ;no error, hold timer at zero

|     |

EAM = 0 ---------------------|     | FFG_SAIR := 0

|

| --- ELSE ---

EAMM = EAM ------------------|     |

;secondary monitor shows    |     |

; correct state             |AND -| V_EAMM_TMR := 0

|     |  ;no error, hold timer at zero

EAMM_TST_TMR < EAMM_TST_TM --|     |

| --- ELSE ---

EAMM = 0 --------------------|     |

|AND -| V_EAMM_TMR := 0

EAM = 0 ---------------------|     | clear_malf(P1414)

| SAIR_OFF_MON := 1

| FFG_SAIR := 0

|

| --- ELSE ---

EAMM = 1 --------------------|     |

|AND -| V_EAMM_TMR := 0

EAM = 1 ---------------------|     | clear_malf(P1413)

|

| --- ELSE ---

EAMM <> EAM -----------------|     |

;primary monitor indicates  |     |

; wrong state               |     |

|     |

V_EAMM_TMR > V_EAMM_TM ------|AND -| malfunction(sair,P1414)

;a fault has been present   |     |  ;EAMM failure commanded off

; long enough               |     |  ; set code P1414

|     |

EAM = 0 ---------------------|     | SAIR_OFF_MON := 1

|  ;monitor complete, failure detected

|

| ER_STATUS := SAIR_ER_DONE

| FFG_SAIR := 1

|

| --- ELSE ---

(continued on next page)

17-24

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

EAMM <> EAM -----------------|     |

;primary monitor indicates  |     |

; wrong state               |AND -| malfunction(sair,P1413)

|     |  ;EAMM failure commanded off

V_EAMM_TMR > V_EAMM_TM ------|     |  ; set code P1413

;a fault has been present         |

; long enough                     | SAIR_ON_MON := 1

|  ;monitor complete, failure detected

|

| ER_STATUS := SAIR_ER_DONE

|

| --- ELSE ---

EAMM <> EAM -----------------|     |

|AND -| EAMM_TST_TMR := 0

EAM = 0 ---------------------|     |

| --- ELSE ---

|

EAMM <> EAM -----------------------| EAMM_TST_TMR := 0

DEMAND_STATE =

sair_koer_primary_test ----|

|

SAIR_PRM_FLG = 1 ------------|AND -| DEMAND_STATE :=

|     |    sair_koer_fuel_init

SAIR_OFF_MON = 1 ------------|     | DEMAND_TIMER := 0

END: therm_koer_circuit_test

17-25

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: therm_commanded_on_test

Execute the following process only when called.

The EAM Monitor (EAMM) is tested to insure that it is indicating the  correct

state.  If it is incorrect for V_EAMM_TM seconds, code P1413 is set.

EAMM = 1 --------------------|

;Secondary monitor          |

; indicates correct state   |

|OR --| V_EAMM_TMR = 0

V_EAMM_ENA = 0 --------------|     |  ;No error, hold timer at zero

;Test not enabled           |     | clear_malf(P1413)

|     |

P0412MALF = 1 ---------------|     |

;Primary circuit failure         |

| --- ELSE ---

|

V_EAMM_TMR > V_EAMM_TM ------|     |

;a fault has been present   |     |

; long enough               |     |

|AND -| malfunction(sair,P1413)

OSM_EO_ON = 1 ---------|     |     |  ;Circuit open from battery or relay

;KOEO test enabled    |     |     |  ; not closed, set code P1413

|     |     |

CCM_ER_ENA = 1 --------|OR --|     | SAIR_ON_MON = 1

;KOER test enabled    |           |  ;monitor complete, failure detected

|           |

CCM_TST_ENA = 1 -------|           |

;Continuous test ena.             |

| --- ELSE ---

|

| No Action.

END: therm_commanded_on_test

17-26

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN:  therm_flow_test

Execute the following process only when called.

Test for flow (Checks for not stuck rich HEGO with upstream air):

In order to test the pump, it must be capable of driving the HEGO lean  while

the A/F ratio is rich in Open loop.  If the A/F ratio is too rich for the air

pump to drive the HEGO lean at the current Thermactor flow rate (proportional

to exhaust back pressure - which is inferred from AM) then the test cannot be

performed.

P0412MALF = 1 ---------------|

|OR --| exit flow test

P1413MALF = 1 ---------------|     |  ; errors detected

|  ; do not need to test for flow

|

| --- ELSE ---

VEGO11_FIL2 <                      |

SAIR_EGO_LVL ---------|           |

|AND -|     |

NUMEGO = 1 ------------|     |     |

|     |

VEGO11_FIL2 <                |OR --| LN_CTL_FLG := 1

SAIR_EGO_LVL --------|     |     | ;no error detected

|     |     | ; allow V_AP_TSTTMR and

VEGO21_FIL2 <          |     |     | ; SAIR_LN_TMR to run

SAIR_EGO_LVL --------|AND -|     | ; HEGOs are lean, but are warmed

|           | ; up - indicates air pump is

NUMEGO = 2 ------------|           | ; functional

|

| --- ELSE ---

|

| LN_CTL_FLG := 0

| ;freeze SAIR_LN_TMR

| ;allow V_AP_TSTTMR to run

| ;HEGOs are rich and system is testable,

| ; possible error condition

| ;A rich indication from HEGOs indicate

| ; that they have warmed up

17-27

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

If rich error exists for a calibratable time, the pump is considered  failed.

(CODE P0411).  If sufficient test time passes without a failure detected, the

test is passed.

V_AP_TSTTMR > V_AP_TST_TM ---|

;test timed out             |AND -| SAIR_ON_MON = 1

|     |  ;done testing

SAIR_LN_TMR >= SAIR_LN_TM ---|     |  ; "pass"

;no failure                       |

| SAIR_END_TMR := 0

|  ;Begin delay to allow pump to spin down

|

| SAIR_TMRK1 := SAIR_LN_TMR

|  ;update for mode 06 reporting

| SAIR_TMRK2 := FFh

|  ;fill to max value for mode 06

|  ; reporting

|

| --- ELSE ---

V_AP_TSTTMR > V_AP_TST_TM ---|     |

|      ;test timed out             |     |

|                                  |     |

|     SAIR_LN_TMR < SAIR_LN_TM ----|AND -| SAIR_ON_MON = 1

|      ;Error persists under test  |     |  ;a failure was detected, but do not set

|      ; enabled conditions        |     |  ; code because valve could be frozen

|                                  |     |

|     INFAMB_KAM < SAIR_AMB_MIN ---|     | SAIR_TMRK1 := SAIR_LN_TMR

|      ;Ambient temp too low             |  ;update for mode 06 reporting

|                                        | SAIR_TMRK2 := FFh

|                                        |  ;fill to max value for mode 06

|                                        |  ; reporting

|                                        |

|                                        | --- ELSE ---

|     V_AP_TSTTMR > V_AP_TST_TM ---|     |

;test timed out             |AND -| malfunction(sair,P0411)

|     |  ;set failure code

SAIR_LN_TMR < SAIR_LN_TM ----|     | SAIR_ON_MON = 1

;Error persists under test        |  ;a failure was detected while

; enabled conditions              |  ; the test was enabled

|

| SAIR_TMRK1 := SAIR_LN_TMR

|  ;update for mode 06 reporting

| SAIR_TMRK2 := FFh

|  ;fill to max value for mode 06

|  ; reporting

|

| --- ELSE ---

|

| No action

|  ;haven't passed or failed yet

END: therm_flow_test

17-28

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: therm_commanded_off_test

Execute the following process only when called.

If an EAMM failure exists that indicates that there may be secondary air

present when the pump is commanded off, set the flag FFG_SAIR so the OBD-II

executive will disable tests that would be adversly affected by secondary

air.

unconditionally -------------------| V_AP_TSTTMR  = 0

|  ;hold timers used in  thermactor on

|  ; test to zero

P0412MALF = 1 ---------------|     |

|AND -| V_EAMM_TMR = 0

EAMM = 1 --------------------|     |  ;No error, hold timer at zero

;Monitor indicates pump on        | clear_malf(P1414)

| SAIR_OFF_MON = 1

|

| --- ELSE ---

|

EAMM = 0 --------------------|     |

;Secondary monitor indicates|     |

; correct state             |     |

|OR --| V_EAMM_TMR = 0

V_EAMM_ENA = 0 --------------|     |  ;No error, hold timer at zero

;Test not enabled           |     | clear_malf(P1414)

|     | SAIR_OFF_MON = 1

P0412MALF = 1 ---------------|     | FFG_SAIR = 0

;Primary circuit failure          |  ;no eamm failure

|

| --- ELSE ---

|

V_EAMM_TMR > V_EAMM_TM ------|     |

;a fault has been present   |     |

; long enough               |     | malfunction(sair,P1414)

|AND -|  ;ECU failure or primary circuit

CCM_TST_ENA = 1 -------|     |     |  ; failure (open or ground)

;Continuous test ena. |     |     |  ; set code P1414

|     |     |

OSM_EO_OFF = 1 --------|OR --|     | SAIR_OFF_MON = 1

;KOEO test enabled    |           | FFG_SAIR = 1

|           |  ;eamm failure

CCM_ER_ENA = 1 --------|           |

;KOER test enabled                |

| --- ELSE ---

|

| NO ACTION

|  ;allow V_EAMM_TMR to run

END:  therm_commanded_off_test

17-29

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN:  therm_fuel_init

Execute the following process only when called.

SAIR_TST_ENA = 1 -----------------| Do: therm_rich_continuous

|

| --- ELSE ---

|

SAIR_EGO_LN = 0 ------------------| Do: therm_lean_check

|

| --- ELSE ---

|

| Do: therm_rich_check

END: therm_fuel_init

BEGIN: therm_rich_continuous

Execute the following process only when called.

VEGO11_FIL2 < SAIR_EGO_LVL -|

|AND -|

NUMEGO = 1 -----------------|     |

|

VEGO11_FIL2 <                     |OR --| SAIR_EGO_LN := 1

SAIR_EGO_LVL -----|           |     |

|OR --|     |     |

VEGO21_FIL2 <         |     |     |     |

SAIR_EGO_LVL -----|     |AND -|     |

|           |

NUMEGO = 2 -----------------|           |

| --- ELSE ---

|

| SAIR_EGO_LN := 0

| ; ramp LAMBSE until rich is seen

SAIR_LAM_DSD > SAIR_REQV ---------|     | ; on both hegos

|AND -| SAIR_LAM_DSD := max(SAIR_LAM_DSD -

SAIR_LAM_DSD >                    |     |   SAIR_DLAM,SAIR_REQV)

SAIR_LEQV -------------|     |     | SAIR_HLD_TMR := 0

|OR --|     | SAIR_RCH_TMR := 0

SAIR_EGO_LN = 1 ------------|           | eam := 0

| CHKAIR := 0

| ;force open loop

|

| --- ELSE ---

(continued on next page)

17-30

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

SAIR_LAM_DSD <= SAIR_REQV --------|     |

|     |

SAIR_HLD_TMR >                    |AND -| EAM_ANTICIP := 0

SAIR_HOLD_TM --------------|     | PSPN_FLWTST := 1

|     | ; we're at clip for SAIR_HOLD_TM

SAIR_EGO_LN = 1 ------------------|     | ;  seconds and both hegos have not

| ;  seen rich

| ; abort test, inform exec

|

| SAIR_TST_RDY := 0

| eam := 0

| CHKAIR := 1

|  ; allow closed loop

|

| --- ELSE ---

|

SAIR_EGO_LN = 1 ------------------------| ; we're at clip

| ;  allow time for hego

| ;  to see rich state

|

| eam := 0

| CHKAIR = 0

|  ;force open loop)

| SAIR_RCH_TMR := 0

|

| --- ELSE ---

|

SAIR_RCH_TMR < SAIR_RCH_TM -------------| CHKAIR := 0

| ; hegos have not been rich for

| ;  SAIR_RCH_TM seconds

| ;  just wait

|

| eam := 0

| SAIR_HLD_TMR := 0

|

| --- ELSE ---

|

| ;  hegos have been rich for

| ;   SAIR_RCH_TM seconds  start

| ;   flow test

|

| eam : 0

| CHKAIR : 0

|  (force open loop)

|

| SAIR_FLWTST := 1

| SAIR_LN_TMR := 0

| V_AP_TSTTMR := 0

END: therm_rich_continuous

17-31

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: therm_lean_check

Execute the following process only when called.

NUMEGO = 1 -----------------|

|AND -|

VEGO11_FIL2 < SAIR_EGO_LVL -|     |

|OR --| SAIR_EGO_LN = 1

NUMEGO = 2 -----------------|     |     | DEMAND_TIMER := 0

|     |     |  ;completed lean ramp

VEGO11_FIL2 < SAIR_EGO_LVL -|AND -|     |  ; restart timer for rich ramp

|           |

VEGO21_FIL2 < SAIR_EGO_LVL -|           | ER_EAM_LEAN = ER_LAM_DSD1

|  ;save lambse value to use as base

|  ; during ramp in rich direction

|

| --- ELSE ---

|

ER_LAM_DSD1 < LEQV ---------------------| ER_LAM_DSD1 := min(ER_LAM_BAS +

;haven't reached clip yet              |                (DEMAND_TIMER *

|                SAIR_VIPLR1),LEQV)

| ER_LAM_DSD2 := ER_LAM_DSD1

|  ;ramp fuel lean until lean state

|  ;seen on both HEGOs.

|

| --- ELSE ---

ER_LAM_DSD1 = LEQV ---------------|     |

|AND -| do nothing

DEMAND_TIMER < SAIR_RAMP_TM ------|     |  ; allow HEGO time to detect switch

|

| --- ELSE ---

|

ER_LAM_DSD1 >= LEQV --------------------| ER_LAM_DSD1 := ER_LAM_BAS

;at lambda clip                        | ER_LAM_DSD2 := ER_LAM_DSD1

|  (reset fuel)

| ER_STATUS := SAIR_ER_DONE

|  (give up, can't do test.)

|

| --- ELSE ---

|

|  (no action)

END: therm_lean_check

17-32

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: therm_rich_check

Execute the following process only when called.

NUMEGO = 1 -----------------|

|AND _|

VEGO11_FIL2 > SAIR_EGO_LVL -|     |

|OR --| SAIR_EGO_RC = 1

NUMEGO = 2 -----------------|     |     |  ; fuel initialization complete

|     |     | ER_LAM_DSD1 := ER_LAM_DSD1 -

VEGO11_FIL2 > SAIR_EGO_LVL -|AND -|     |                 SAIR_LAM_DEL

|           | ER_LAM_DSD2 := ER_LAM_DSD1

VEGO21_FIL2 > SAIR_EGO_LVL -|           |  ;step fuel a little more rich

| V_THERM_TMR := 0

| DEMAND_STATE := sair_koer_fuel_done

|

| --- ELSE ---

|

ER_LAM_DSD1 > REQV ---------------------| ER_LAM_DSD1 := max(ER_EAM_LEAN -

;havn't reached clip yet               |                (DEMAND_TIMER *

|                SAIR_VIPRR1),REQV)

| ER_LAM_DSD2 := ER_LAM_DSD1

|  ;ramp fuel rich untill rich state

|  ; seen on both HEGOs.

|

| --- ELSE ---

ER_LAM_DSD1 = REQV ---------------|     |

|AND -| do nothing

DEMAND_TIMER < SAIR_RAMP_TM ------|     |  ; allow HEGO time to detect switch

|

| --- ELSE ---

|

ER_LAM_DSD1 = REQV ---------------------| ER_LAM_DSD1 := ER_LAM_BAS

(at lambda clip)                       | ER_LAM_DSD2 := ER_LAM_DSD1

|  (reset fuel)

| ER_STATUS := SAIR_ER_DONE

|  (give up, can't do test.)

END: therm_rich_check

17-33

THERMACTOR AIR STRATEGY, ELECTRIC AIR MANAGEMENT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

17-34

CHAPTER 18

INTAKE MANIFOLD RUNNER CONTROL STRATEGY

18-1

INTAKE MANIFOLD RUNNER CONTROL STRATEGY

18.1  FEATURE:  INTAKE MANIFOLD RUNNER CONTROL - V1.3A_IMRC_ONOFF (CDAN2)

18.1.1  IMRC COMMAND LOGIC (CDAN0)

OVERVIEW

This Intake Manifold Runner Control  (IMRC)  module  determines  the  desired

position  of the IMRC valve.  The strategy is enabled by the IMRC H/W present

switch, (IMRCHP = 1).  The IMRC valve is used  to  increase  airflow  to  the

engine  and improve top-end performance.  The desired valve position is based

on the current engine speed, N, and the throttle position, TP_REL, as defined

in  FN503.   Independent hysterisis values for both engine speed and throttle

position are used to prevent overuse of the valve due  to  jittering  at  the

on/off boundary.

The IMRC strategy supports the following:

1) Normally open and normally closed valves.  The calibrations should  be  as

follows:

Normally Closed:  IMRC_OFF_POS = 0, IMRC_ON_POS = 1

Normally Open:  IMRC_OFF_POS = 1, IMRC_ON_POS = 0

2) OBDII diagnostics, which are enabled by the  sensor  H/W  present  switch,

(IMRCSENSHP = 1), and the driver diagnostics.  KOEO OBDII diagnostics are not

valid for vacuum systems and should be disabled by setting IMRC_TYPE = 0.

3) Disabling IMRC state changes when the transmission  strategy  requests  it

via FLG_IMRC_HLD.

DEFINITIONS

Registers:

-  APT(Input) = Throttle mode flag (UNITLESS).

-  ECT(Input) = Engine Coolant Temperature (DEG F).

-  IMRCSPK_W(Output) = Weighting factor for IMRC spark modifiers.   Unitless

multiplier  from  0  (IMRC  valve fully closed) to 1.99 (IMRC valve fully

open).

-  IMRC_N_REL = Engine speed hysteresis to hold the IMRC valve open (RPM).

-  IMRC_RSP_TMR(Input/Output) = Time since last IMRC  state  change  request

(SEC).

-  IMRC_STATUS(Input) = IMRC status; 0 -> OK, 1 -> stuck closed, 2 ->  stuck

open.

18-2

INTAKE MANIFOLD RUNNER CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  IMRC_TP_REL = Throttle position to enable the IMRC to open (COUNTS).

-  N(Input) = Engine speed (RPM).

-  TP_REL(Input) = Throttle Position (COUNTS).

-  VBAT(Input) = Battery voltage.

-  VS(Input) = Vehicle speed (MPH).

Bit Flags:

-  CCM_EO_ENA(Input) = Continuous monitor, engine off, enable flag.

-  CCM_ER_ENA(Input) = Continuous monitor, engine running, enable flag.

-  CCM_TST_ENA(Input) = Continuous monitor test enabled flag.

-  CRKFLG(Input) = Crank flag; 1 -> cranking.

-  FLG_IMRC_HLD(Input) = Indicates that IMRC should not change state  during

a shift event.

-  IMRC_AS_ENA = IMRC anti-sludging enable flag; 1 -> enable IMRC to open.

-  IMRC_CMND = Commanded state of IMRC; 1 -> port throttle open.

-  IMRC_CMNDOLD = Last commanded state of IMRC; 1 -> port throttle open.

-  IMRC_EN_KOEO = Enable switch for IMRC plate diagnostics  on  KOEO.   Used

for vacuum systems.  1 -> enable KOEO plate diagnostics.  0 -> Disable.

-  IMRCKAM_SWAP = IMRC last-trip cycling information has been uploaded  into

RAM; 1 -> yes.

-  IMRCLAST_KAM = Was IMRC cycled on previous trip?  1 -> yes.

-  IMRCLASTTRIP = Did IMRC cycle on the previous trip?  1 -> yes.

-  IMRCTHISTRIP = Was IMRC cycled yet this trip?  1 -> yes.

-  IMRC_POSN(Input/Output) = Measured valve position;1 -> open.

-  IMRC_POSNOLD = Previous value of IMRC_POSN

-  IMRC_TP_ENA = IMRC throttle position enable; 1 -> IMRC open is  desirable

for engine performance.

-  OSM_EO_ON(Input) = Output state monitor, engine off, on demand test.

-  OSM_EO_OFF(Input) = Output state monitor, engine off, off demand test.

-  P1516MALF(Input) = IMRC bank 1 input error.

-  P1517MALF(Input) = IMRC bank 2 input error.

-  SUBST_REQ21 = Substitution Requested flag for channel 21.

Calibration Constants:

18-3

INTAKE MANIFOLD RUNNER CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FN185 = Function to ramp-out spark as valve closes.

-  FN186 = Function to add spark as valve opens.

-  FN503 = IMRC enable for turning on IMRC as a function of  desired  TP_REL

states and engine speeds.

-  IMRC_CRK_POS = Desired IMRC position during cranking; 0 -> closed,  1  ->

open.

-  IMRC_CYC_ECT = ECT that enables forced IMRC cycling (DEG F).

-  IMRC_CYC_VS = VS that enables forced IMRC cycling (MPH).

-  IMRCHP = IMRC hardware present; 0 -> No IMRC hardware, 1 -> IMRC hardware

present.

-  IMRC_N_HYS = Engine speed hystersis value to avoid valve jitter (RPM).

-  IMRC_OFF_POS = De-energized position; 1 -> open, 0 -> closed.

-  IMRC_ON_POS = Energized position; 1 -> open, 0 -> closed.

-  IMRCSENSHP = Position sensor  hardware  present  switch;  0  ->  no  IMRC

position sensor, 1 -> IMRC position sensor hardware present.

-  IMRC_SFT = Prevents IMRC from changing  states  during  a  shift.   1  ->

prevents state changes; 0 -> allows changes during a shift.

-  IMRC_TP_HYS = Throttle position hystersis value  to  avoid  valve  jitter

(COUNTS).

-  IMRC_TYPE = Electric/Vacuum switch; 1 -> Electric, 0 -> Vacuum

-  VBAT_CCM_MIN  =  Minimum  battery  voltage   necessary   to   run   OBDII

diagnostics.

18-4

INTAKE MANIFOLD RUNNER CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: IMRC_COMMAND_COM1

BEGIN: imrc_command_main

;This process is executed once per background loop.

If IMRC hardware is not present, disable spark modification and exit

IMRC logic completely.

OSC response logic:  If Output State Control has been requested for IMRC_CMND

but IMRC hardware is not present, a sequence error message will be sent.

SUBST_REQ21 = 1 ------------------|

|AND -| Do: OSC_RESPONSE(21,22h)

IMRCHP = 0 -----------------------|     | ;OSC substitute requested but local

| ; conditions will not allow

| ; substitution

|

| IMRCSPK_W := 0

|  ;no spark modification

|

| EXIT imrc_command_main

|  ;do not execute the logic

|  ;below

|

| --- ELSE ---

|

IMRCHP = 0 -----------------------------| IMRCSPK_W := 0

;no IMRC hardware                      |  ;no spark modification

|

| EXIT imrc_command_main

|  ;do not execute the logic

|  ;below

There are two modes of IMRC operation, on-demand and normal

OSM_EO_ON = 1 --------------------|

;Engine-off On-demand test       |OR --| Do: imrc_demand_state

|     |

OSM_EO_OFF = 1 -------------------|     |

;Engine-off Off-demand test            |

| --- ELSE ---

|

| Do: imrc_normal_operation

|  ;set command based on normal

|  ;operating conditions

18-5

INTAKE MANIFOLD RUNNER CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

If the command has changed, then reset the diagnostics timer.

IMRC_CMND = IMRC_CMNDOLD ---------------| NO ACTION

;command has not changed               |  ;let timer run freely

|

| --- ELSE ---

|

| IMRC_RSP_TMR := 0

|  ;command has changed,

|  ;reset response timer

Store the previous commanded and measured positions.

unconditionally ------------------------| IMRC_CMNDOLD := IMRC_CMND

|  ;store old commanded value

|

| IMRC_POSNOLD := IMRC_POSN

|  ;store old position value

18-6

INTAKE MANIFOLD RUNNER CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Spark timing is advanced as  the  plates  move  from  their  closed  to  open

position.  The spark modification is done in a purely open-loop fashion based

on the time since the command position has changed.  If IMRC fails open, then

fully open spark is given.  If IMRC fails closed then spark is not advanced.

IMRC_STATUS = 1 ------------------|

;IMRC is stuck closed.           |

|OR --| IMRCSPK_W := 0

P1516MALF = 1 --------------------|     |  ;Use closed plate spark

;IMRC1 input error               |     |

|     |

P1517MALF = 1 --------------------|     |

;IMRC2 input error                     |

| --- ELSE ---

|

IMRC_STATUS = 2 ------------------------| IMRCSPK_W := FN186( FFh )

;IMRC is stuck open                    |  ;Use max open plate spark

|  ; (Maximum value for IMRC_RSP_TMR)

|

| --- ELSE ---

|

IMRC_CMND = 0 --------------------------| IMRCSPK_W := FN185(IMRC_RSP_TMR)

;commanding plates closed              |  ;OK to adjust spark smoothly to

|  ;fully closed value

|

| --- ELSE ---

|

IMRC_CMND = 1 --------------------------| IMRCSPK_W := FN186(IMRC_RSP_TMR)

;commanding plates open                |  ;OK to adjust spark smoothly to

;IMRC is stuck open                    |  ;fully open value

unconditionally ------------------------| Do: imrc_output_main

|  ;generate output signal

If sensor hardware is present then read the position sensor.  If  the  sensor

is not present, then assume that the actual position is the desired position.

IMRCSENSHP = 1 -------------------------| Do: imrc_input_main

|  ;read position sensor

|

| --- ELSE ---

|

| IMRC_POSN := IMRC_CMND

Record if the measured plate position has changed.   If  so  set  appropriate

flags which will disable anti-sludging routine.

IMRC_POSN <> IMRC_POSNOLD --------|

;measured plate position         |

;has changed                     |AND -| IMRCLAST_KAM := 1

|     | IMRCTHISTRIP := 1

IMRCTHISTRIP = 0 -----------------|     |  ;store KAM info for next trip

;IMRC has not cycled this trip

18-7

INTAKE MANIFOLD RUNNER CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Output driver diagnostics

VBAT >= VBAT_CCM_MIN -------------|

;battery voltage stable          |

|

CCM_TST_ENA = 1 ------------|     |AND -| Do: imrc_diagnostics_osm_main

;continuous CCM test       |     |     |  ;check for driver faults

;enabled                   |     |     |

|     |     |

CCM_ER_ENA = 1 -------------|OR --|     |

;Engine-Running On-Demand  |           |

;test enabled              |           |

|           |

CCM_EO_ENA = 1 -------------|           |

;Engine-Off On-Demand                  |

;test enabled                          |

| --- ELSE ---

|

| NO ACTION

KOEO system diagnostics are not valid for vacuum  systems,  due  to  lack  of

vacuum to actuate valves.

IMRC_TYPE = 1 --------------------|

;Electric IMRC                   |AND -| IMRC_EN_KOEO = 1

|     |  ;Enable KOEO system

CCM_EO_ENA = 1 -------------------|     |  ;diagnostics

;Engine-off On-demand                  |

;test enabled                          |

| --- ELSE ---

|

| IMRC_EN_KOEO = 0

|  ;Disable KOEO system

|  ;diagnostics

18-8

INTAKE MANIFOLD RUNNER CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Systems diagnostics

VBAT >= VBAT_CCM_MIN -------------|

;battery voltage stable          |

|

IMRCSENSHP = 1 -------------------|

;sensor HW present               |

|

CCM_TST_ENA = 1 ------------|     |AND -| Do: imrc_diagnostics_main

;continuous CCM test       |     |     |  ;check for system faults

;enabled                   |     |     |

|     |     |

CCM_ER_ENA = 1 -------------|OR --|     |

;Engine-Running On-Demand  |           |

;test enabled              |           |

|           |

IMRC_EN_KOEO = 1 -----------|           |

;Enable KOEO test                      |

| --- ELSE ---

|

| NO ACTION

END: imrc_command_main

18-9

INTAKE MANIFOLD RUNNER CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: imrc_demand_state

;This routine is executed only when explicitly called to the the IMRC command

;during engine-off on-demand test.

OSM_EO_ON = 1 --------------------------| IMRC_CMND := IMRC_ON_POS

;on-demand OSM test                    |  ;command actuator to energize

;requesting "on" state                 |

| --- ELSE ---

|

| IMRC_CMND := IMRC_OFF_POS

|  ;command actuator to de-energize

END: imrc_demand_state

BEGIN: imrc_normal_operation

;This routine is executed only when explicitly called and is the core

;logic to command the IMRC plates during all engine-running conditions.

After the engine has started, move last trip's cycling info in from KAM, then

clear the KAM ONCE per trip.

N > 600 --------------------------|

|AND -| IMRCLASTTRIP := IMRCLAST_KAM

IMRCKAM_SWAP = 0 -----------------|     |  ;move last trip's cycling

;info from last trip                   |  ;record into RAM

;not yet uploaded                      |

| IMRCLAST_KAM := 0

| IMRCKAM_SWAP := 1

|  ;reset trip record ONCE, at

|  ;beginning of current trip

Determine if open plates are needed for engine  performance.   Hysteresis  is

used to prevent jitter caused by small changes or noise in throttle position.

unconditionally ------------------------| IMRC_TP_REL := FN503(N)

|  ;set TP limit

unconditionally ------------------------| IMRC_N_REL := FN503(N + IMRC_N_HYS)

|  ;set N limit

18-10

INTAKE MANIFOLD RUNNER CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TP_REL > IMRC_TP_REL -------------------| IMRC_TP_ENA := 1

|  ;open plates desired

|  ;for engine performance

|

| --- ELSE ---

TP_REL < IMRC_TP_REL -                  |

IMRC_TP_HYS ------|     |

|AND -| IMRC_TP_ENA := 0

TP_REL < IMRC_N_REL --------------|     |  ;open plates not desired

|  ;for engine performance

;Determine if open plates are needed for anti-sludging.  The

;anti-sludging cycling will only be forced if it was not cycled on the

;last trip, and will be done at most once every other trip.

ECT > IMRC_CYC_ECT ---------------|

;ECT high enough                 |

|

APT = -1 -------------------------|

;closed throttle                 |

|

VS > IMRC_CYC_VS -----------------|AND -| IMRC_AS_ENA := 1

;vehicle speed high enough       |     |  ;open plates desired

|     |  ;for anti-sludging

IMRCLASTTRIP = 0 -----------------|     |

;did not cycle                   |     |

;previous trip                   |     |

|     |

IMRCTHISTRIP = 0 -----------------|     |

;has not cycled this trip              |

| --- ELSE ---

|

| IMRC_AS_ENA := 0

|  ;anti-sludging action

|  ;not needed

18-11

INTAKE MANIFOLD RUNNER CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;Determine the desired position of the IMRC plates.

CRKFLG = 1 -----------------------------| imrc_cmnd := IMRC_CRK_POS

;cranking                              |  ;keep IMRC de-energized

|  ;during cranking

|

| --- ELSE ---

IMRC_SFT = 1 ---------------------|     |

;calibration to hold IMRC        |     |

;during shifts                   |AND -| imrc_cmnd := IMRC_CMNDOLD

|     |  ;do not command IMRC change during

FLG_IMRC_HLD = 1 -----------------|     |  ; transmission events

;Hold IMRC, shift event                |

| --- ELSE ---

P1516MALF = 0 --------------------|     |

;IMRC1 input OK                  |     |

|AND -|

P1517MALF = 0 --------------------|     | imrc_cmnd := 1

;IMRC2 input OK                  |     |  ;command plates to open

|     |

IMRC_TP_ENA = 1 ------------|     |     |

;open plates desired       |     |     |

;for performance           |OR --|     |

|           |

IMRC_AS_ENA = 1 ------------|           |

;open plates desired                   |

;for anti-sludging                     |

| --- ELSE ---

|

| imrc_cmnd := 0

|  ;command plates to close

Output State Control

unconditionally ------------------------| DO: substitute(21,imrc_cmnd)

| IMRC_CMND := imrc_cmnd

| ; output state control override

END: imrc_normal_operation

18-12

INTAKE MANIFOLD RUNNER CONTROL STRATEGY, DIAGNOSTICS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

18.1.2  IMRC DIAGNOSTICS (CDAM0)

OVERVIEW

This IMRC  module  determines  the  status  of  the  IMRC  valves  for  OBDII

diagnostics  and for adding spark.  When plates are commanded open or closed,

the  IMRC  response  timer,  IMRC_RSP_TMR,  is  reset  and  compared  to  the

calibrations,  IMRC_OPN_TIM  or  IMRC_CLS_TIM.   If  the plate sensors do not

measure that the valves  have  reached  the  commanded  position  within  the

cailbration,  the  corresponding  stuck open or stuck closed fault filter are

incremented.  The fault filters are incremented until a  calibratabled  stuck

closed  or  stuck  open  counter  threshold  is  reached.   This triggers the

malfunction counters to increment and  the  fault  filters  to  zero.   Stuck

closed  counters  are  allowed to only increment once per open command event.

Once the malfucntion counter has reached the counter  threshold,  a  code  is

set.  The diagnostics are as follows:

DEFINITIONS

Registers:

-  IMRC_LOD_TMR = Free running timer that is  used  to  disable  stuck  open

diagnosticss (SEC).

-  IMRC_RSP_TMR(Input) = Free running timer to delay checking if a valve has

moved (SEC).

-  IMRC_SC1_CNT = Stuck closed fault counter for bank 1.

-  IMRC_SC1_FIL = Stuck closed fault filter for bank 1.

-  IMRC_SC2_CNT = Stuck closed fault counter for bank 2

-  IMRC_SC2_FIL = Stuck closed fault filter for bank 2.

-  IMRC_SO1_CNT = Stuck open fault counter for bank 1.

-  IMRC_SO1_FIL = Stuck open fault filter for bank 1.

-  IMRC_SO2_CNT = Stuck open fault counter for bank 2.

-  IMRC_SO2_FIL = Stuck open fault filter for bank 2.

-  IMRC_STATE1 = Instantaneous bank 1 failure indicator; 2 -> stuck open,  1

-> stuck closed, 0 -> OK.

-  IMRC_STATE2 = Instantaneous bank 2 failure indicator; 2 -> stuck open,  1

-> stuck closed, 0 -> OK.

-  IMRC_STAOLD1 = Previous event bank 1 failure indicator; 2 -> stuck  open,

1 -> stuck closed, 0 -> OK.

-  IMRC_STAOLD2 = Previous event bank 2 failure indicator; 2 -> stuck  open,

1 -> stuck closed, 0 -> OK.

18-13

INTAKE MANIFOLD RUNNER CONTROL STRATEGY, DIAGNOSTICS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  IMRC_STATUS(Output) = Logical function  of  IMRC_STATE1  and  IMRC_STATE2

used in the IMRC command spark adder logic.

-  PCT_LOAD = Percent Load, unitless.

Bit Flags:

-  CCM_EO_ENA = Engine off on demand test for CCM enabled.

-  IMRC_BK1_SCL = Bank 1 valve stuck closed; 1 -> stuck.

-  IMRC_BK1_SOP = Bank 1 valve stuck open; 1 -> stuck.

-  IMRC_BK2_SCL = Bank 2 valve stuck closed; 1 -> stuck.

-  IMRC_BK2_SOP = Bank2 valve stuck open; 1 -> stuck.

-  IMRC_CMND(Input) = Commanded position of valves; 1 -> open, 0 -> closed.

-  IMRC_POSN_1(Input) = Position of Bank 1 valves.  1 -> open, 0 -> closed.

-  IMRC_POSN_2(Input) = Position of Bank 2 valves.  1 -> open, 0 -> closed.

-  KAM_ERROR = INDICATES KEEP ALIVE RAM INVALID.

-  OBDII_RESET = Flag used to simulate the receipt of an  OBD-II  scan  tool

reset message.

-  P1516MALF = Bank 1 input error currently exists.

-  P1517MALF = Bank 2 input error currently exists.

Calibration Constants:

-  IMRC_CLS_TIM = Time to close valves before fault filtering begins (SEC).

-  IMRC_FALT_TC = Fault filter time constant.

-  IMRC_OPN_TIM = Time to open valves before fault filtering begins (SEC).

-  IMRC_TWOPOSN = IMRC hardware present switch for 2 position inputs;  0  ->

read 1 input, 1 -> read 2 inputs.

-  IMRC_VAC_LOD = PCT_LOAD at which "lack of vacuum" could cause stuck  open

failures (UNITLESS).

-  IMRC_VAC_TIM = Time that "lack of vacuum"  must  be  present  to  disable

failure diagnostics (SEC).

-  IMSO_CNT_LVL = Counter level at which to set stuck open code.

-  IMSO_ERRLVL = Error level threshold to increment stuck open counter.

-  IMSC_CNT_LVL = Counter level at which to set stuck closed IMRC fault.

-  IMSC_ERRLVL = Error level threshold to increment stuck closed counter.

18-14

INTAKE MANIFOLD RUNNER CONTROL STRATEGY, DIAGNOSTICS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OTHER

-  P1512 = Non-MIL fault code for bank one stuck closed.

-  P1513 = Non-MIL fault code for bank two stuck closed.

-  P1518 = MIL fault code for both banks stuck open.

-  P1519 = Non-MIL fault code for both banks stuck closed.

-  P1537 = MIL fault code for bank one stuck open.

-  P1538 = MIL fault code for bank two stuck open.

18-15

INTAKE MANIFOLD RUNNER CONTROL STRATEGY, DIAGNOSTICS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: IMRC_DIAG_OBDII_COM1

BEGIN: imrc_diagnostics_main

;This process is executed only when explicity called.

P1516MALF = 0 --------------------------| Do: imrc_diagnostics_bank1

;The bank 1 pos'n sensor is OK         |

IMRC_TWOPOSN = 1 -----------------|

;Two IMRC position sensors       |

;present                         |AND -| Do: imrc_diagnostics_bank2

|     |

P1517MALF = 0 --------------------|     |

;The bank 2 pos'n sensor is OK         |

| --- ELSE ---

|

| IMRC_SC2_FIL := 0

| IMRC_SC2_CNT := 0

| IMRC_SO2_FIL := 0

| IMRC_SO2_CNT := 0

In the event of an OBDII reset or a KAM error, clear all codes, and reset

all counters.

OBDII_RESET = 1 ------------------------|

|OR --| IMRC_SC1_FIL := 0

KAM_ERROR = 1 --------------------------|     | IMRC_SC1_CNT := 0

| IMRC_SO1_FIL := 0

| IMRC_SO1_CNT := 0

| IMRC_SC2_FIL := 0

| IMRC_SC2_CNT := 0

| IMRC_SO2_FIL := 0

| IMRC_SO2_CNT := 0

| clear_malf(P1512)

| clear_malf(P1513)

| clear_malf(P1518)

| clear_malf(P1519)

| clear_malf(P1537)

| clear_malf(P1538)

18-16

INTAKE MANIFOLD RUNNER CONTROL STRATEGY, DIAGNOSTICS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The IMRC_STATUS variable is used as the input to the spark calculation.  If a

bank  fails  closed,  no  spark  should  be  added until that bank has opened

correctly at least once.  If both banks fail open, then the status should  be

set  such  that  full  open-plate  spark  is  added, to protect the catalyst.

Otherwise, the status flag is cleared, for normal ramp in and ramp out  spark

operation.

IMRC_SC1_FIL > 0 -----------------|

|

IMRC_SC1_CNT > 0 -----------------|

|OR --| IMRC_STATUS := 1

IMRC_SC2_FIL > 0 -----------------|     |  ;At least one bank has failed

|     |  ;closed.  Do not allow spark

IMRC_SC2_CNT > 0 -----------------|     |  ;addition

|

| --- ELSE ----

IMRC_SO1_FIL > 0 -----------|           |

|OR --|     |

IMRC_SO1_CNT > 0 -----------|     |     |

|AND -| IMRC_STATUS := 2

IMRC_SO2_FIL > 0 -----------|     |     |  ;Both banks have failed open.

|OR --|     |  ;Allow only full-open spark.

IMRC_SO2_CNT > 0 -----------|           |

| --- ELSE ---

|

| IMRC_STATUS := 0

|  ;IMRC is working properly

|  ;Allow normal spark addition

18-17

INTAKE MANIFOLD RUNNER CONTROL STRATEGY, DIAGNOSTICS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Vacuum actuated valves can fail due to "lack of vacuum environments".  If the

percent load exceeds a calibrated low vacuum load, IMRC_VAC_LOD, then a timer

is activated.  If the environment exists for more than a  calibratable  time,

IMRC_VAC_TIM, then the malfunction codes are disabled.

IMRC_TYPE = 0 --------------------|

;vaccum actuation                |AND -| Let IMRC_LOD_TMR run freely

|     |

PCT_LOAD > IMRC_VAC_LOD ----------|     |

;Vacuum too low to close valve         |

| --- ELSE ---

|

| IMRC_LOD_TMR := 0

|  ;reset the low vacuum timer

IMRC_TYPE = 0 --------------------|

;vacuum actuation                |AND--| IMRC_RSP_TMR := 0

|     |  ;Do not allow the fault

IMRC_LOD_TMR >= IMRC_VAC_TIM -----|     |  ;filters or counters to

;Lack of vacuum of sufficient          |  ;increment

;duration                              |

| --- ELSE ---

|

| NO ACTION

Perform the appropriate diagnostics.

IMRC_TYPE = 1 --------------------------| Do: imrc_obdii_code_elect

;electric actuation                    |  ;set or clear appropriate

|  ;malfunction codes

|

| --- ELSE ---

|

| Do: imrc_obdii_code_vac

|  ;set or clear appropriate

|  ;malfunction codes

|

END: imrc_diagnostics_main

18-18

INTAKE MANIFOLD RUNNER CONTROL STRATEGY, DIAGNOSTICS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: imrc_diagnostics_bank1

When the IMRC valves are commanded to a position, they  have  a  calibratable

opening  time  and  closing  time  before  the corresponding fault filter are

incremented.  These filter are used as inputs to malfunction  counters.   The

stuck closed counter should be incremented only once per event.

IMRC_RSP_TMR >= IMRC_CLS_TIM -----|

;valves should have closed       |

|

IMRC_POSN_1 = 1 ------------------|AND -| IMRC_SO1_FIL := ROLAV(1,

;bank 1 measuring open           |     |                      IMRC_FALT_TC)

|     |  ;increment the bank 1 stuck open

IMRC_CMND = 0 --------------------|     |  ;fault filter

;commanding closed                     |

| IMRC_STATE1 := 2

|  ;indicate instantaneous failure

|

| --- ELSE ---

IMRC_RSP_TMR >= IMRC_OPN_TIM -----|     |

;valves should have opened       |     |

|     |

IMRC_POSN_1 = 0 ------------------|     |

;bank 1 measuring closed         |AND -| IMRC_SC1_FIL := ROLAV(1,

|     |                      IMRC_FALT_TC

IMRC_CMND = 1 --------------------|     |  ;increment the bank 1 stuck closed

;commanding open                 |     |  ;fault filter

|     |

IMRC_STAOLD1 <> 1 ----------|     |     | IMRC_STATE1 := 1

;Stuck closed counter has  |     |     |  ;indicate instantaneous failure

;not incremnted this event |OR --|     |

|           |

CCM_EO_ENA = 1 -------------|           |

;KOEO test in progress                 |

| --- ELSE ---

|

| IMRC_STATE1 := 0

|  ;clear failure indicator

18-19

INTAKE MANIFOLD RUNNER CONTROL STRATEGY, DIAGNOSTICS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

If the valves are commanded to close, and  the  measured  position  indicates

they  have  closed, then the valves are not stuck open.  Reset the stuck-open

filters and counters to zero.  Likewise, if the valves are commanded open and

the  measured  position indicates they have opened, then reset both the stuck

closed filters and stuck closed counters.  If the valves reach the  commanded

position,  reset  the old status function, IMRC_STAOLD(bank) to indicate that

the valves worked correctly on the last event.

IMRC_CMND = 0 --------------------|

;commanding closed plates        |AND -| IMRC_SO1_FIL := 0

|     |  ;plates closed properly

IMRC_POSN_1 = 0 ------------------|     |  ;at least once, reset

;measuring bank 1 plates               |  ;the bank 1 stuck open

;closed                                |  ;filter

|

| IMRC_SO1_CNT := 0

|  ;plates closed properly

|  ;at least once, reset

|  ;reset the bank 1 stuck

|  ;open counter

|

| IMRC_STAOLD1 = 0

|  ;bank 1 functioned properly

|

| --- ELSE ---

IMRC_CMND = 1 --------------------|     |

;commanding open plates          |AND -| IMRC_SC1_FIL := 0

|     |  ;plates opened properly

IMRC_POSN_1 = 1 ------------------|     |  ;at least once, reset

;measuring bank 1 plates               |  ;the bank 1 stuck closed

;open                                  |  ;filter

|

| IMRC_SC1_CNT := 0

|  ;plates opened properly

|  ;at least once, reset

|  ;reset the bank 1 stuck

|  ;closed counter

|

| IMRC_STAOLD1 = 0

|  ;bank 1 functioned properly

18-20

INTAKE MANIFOLD RUNNER CONTROL STRATEGY, DIAGNOSTICS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

If a fault filter exceeds the corresponding fault filter threshold, then  the

fault  counter  is  incremented.   When  the  counter is incremented, the old

status flag is set to indicate that at least one failure increment occured.

IMRC_SO1_FIL > IMSO_ERRLVL -------------| IMRC_SO1_CNT := IMRC_SO1_CNT + 1

| CLIP(IMRC_SO1_CNT,0,255)

|  ;increment stuck open counter

|

| IMRC_SO1_FIL := 0

|  ;clear filter so another

|  ;error count can occur

|

| IMRC_STAOLD1 := 2

|  ;bank 1 is stuck open

IMRC_SC1_FIL > IMSC_ERRLVL -------------| IMRC_SC1_CNT := IMRC_SC1_CNT + 1

| CLIP(IMRC_SC1_CNT,0,255)

|  ;increment stuck closed counter

|

| IMRC_SC1_FIL := 0

|  ;clear filter so another

|  ;error count can occur

|

| IMRC_STAOLD1 := 1

|  ;bank 1 is stuck closed

Compare each banks stuck open fault counters to the stuck  open  fault  count

threshold.  If the levels are exceeded, then set the bank stuck open flag.

IMRC_SO1_CNT > IMSO_CNT_LVL ------------| IMRC_BK1_SOP := 1

|  ;bank 1 is stuck open

|

| --- ELSE ---

|

| IMRC_BK1_SOP := 0

|  ;Bank 1 is not stuck open

Compare each banks stuck closed fault counters  to  the  stuck  closed  fault

count  threshold.  If the levels are exceeded, then set the bank stuck closed

flag.

IMRC_SC1_CNT > IMSC_CNT_LVL ------------| IMRC_BK1_SCL := 1

|  ;bank 1 is stuck closed

|

| --- ELSE ---

|

| IMRC_BK1_SCL := 0

|  ;bank 1 is not stuck closed

END: imrc_diagnostics_bank1

18-21

INTAKE MANIFOLD RUNNER CONTROL STRATEGY, DIAGNOSTICS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: imrc_diagnostics_bank2

Determine the state and fault filter value for bank 2.

When the IMRC valves are commanded to a position, they  have  a  calibratable

opening  time  and  closing  time  before  the corresponding fault filter are

incremented.  These filter are used as inputs to malfunction  counters.   The

stuck closed counter should be incremented only once per event.

IMRC_RSP_TMR >= IMRC_CLS_TIM -----|

;valves should have closed       |

|

IMRC_POSN_2 = 1 ------------------|AND -| IMRC_SO2_FIL := ROLAV(1,

;bank 2 measuring open           |     |                      IMRC_FALT_TC)

|     |  ;increment the bank 2 stuck open

IMRC_CMND = 0 --------------------|     |  ;fault filter

;commanding closed                     |

| IMRC_STATE2 := 2

|  ;indicate instantaneous failure

|

| --- ELSE ---

IMRC_RSP_TMR >= IMRC_OPN_TIM -----|     |

;valves should have opened       |     |

|     |

IMRC_POSN_2 = 0 ------------------|     |

;bank 2 measuring closed         |AND -| IMRC_SC2_FIL := ROLAV(1,

|     |                      IMRC_FALT_TC)

IMRC_CMND = 1 --------------------|     |  ;increment the bank 2 stuck closed

;commanding open                 |     |  ;fault filter

|     |

IMRC_STAOLD2 <> 1 ----------|     |     | IMRC_STATE2 := 1

;Stuck closed counter has  |     |     |  ;indicate instantaneous failure

;not incremnted this event |OR --|     |

|           |

CCM_EO_ENA = 1 -------------|           |

;KOEO test in progress                 |

| --- ELSE ---

|

| IMRC_STATE2 := 0

|  ;clear instantaneous failure

18-22

INTAKE MANIFOLD RUNNER CONTROL STRATEGY, DIAGNOSTICS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

If the valves are commanded to close, and  the  measured  position  indicates

they  have  closed, then the valves are not stuck open.  Reset the stuck-open

filters and counters to zero.  Likewise, if the valves are commanded open and

the  measured  position indicates they have opened, then reset both the stuck

closed filters and stuck closed counters.  If the valves reach the  commanded

position,  reset  the old status function, IMRC_STAOLD(bank) to indicate that

the valves worked correctly on the last event.

IMRC_CMND = 0 --------------------|

;commanding closed plates        |AND -| IMRC_SO2_FIL := 0

|     |  ;plates closed properly

IMRC_POSN_2 = 0 ------------------|     |  ;at least once, reset

;measuring bank 2 plates               |  ;the bank 2 stuck open

;closed                                |  ;filter

|

| IMRC_SO2_CNT := 0

|  ;plates closed properly

|  ;at least once, reset

|  ;reset the bank 2 stuck

|  ;open counter

|

| IMRC_STAOLD2 := 0

|  ;bank 2 functioned properly

|

| --- ELSE ---

IMRC_CMND = 1 --------------------|     |

;commanding open plates          |AND -| IMRC_SC2_FIL := 0

|     |  ;plates opened properly

IMRC_POSN_2 = 1 ------------------|     |  ;at least once, reset

;measuring bank 2 plates               |  ;the bank 2 stuck closed

;open                                  |  ;filter

|

| IMRC_SC2_CNT := 0

|  ;plates opened properly

|  ;at least once, reset

|  ;reset the bank 2 stuck

|  ;closed counter

|

| IMRC_STAOLD2 := 0

;bank 2 functioned properly

18-23

INTAKE MANIFOLD RUNNER CONTROL STRATEGY, DIAGNOSTICS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

If a fault filter exceeds the corresponding fault filter threshold, then  the

fault  counter  is  incremented.   When  the  counter is incremented, the old

status flag is set to indicate that at least on failure increment occured.

IMRC_SO2_FIL > IMSO_ERRLVL -------------| IMRC_SO2_CNT := IMRC_SO2_CNT + 1

| CLIP(IMRC_SO2_CNT,0,255)

|  ;incrementstuck open

|

| IMRC_SO2_FIL := 0

|  ;clear filter so another

|  ;error count can occur

|

| IMRC_STAOLD2 := 2

|  ;bank 2 is stuck open

IMRC_SC2_FIL > IMSC_ERRLVL -------------| IMRC_SC2_CNT := IMRC_SC2_CNT + 1

| CLIP(IMRC_SC2_CNT,0,255)

|  ;increment stuck closed counter

|

| IMRC_SC2_FIL := 0

|  ;clear filter so another

|  ;error count can occur

|

| IMRC_STAOLD2 := 1

|  ;bank 2 is stuck closed

Compare each banks stuck open fault counters to the stuck  open  fault  count

threshold.  If the levels are exceeded, then set the bank stuck open flag.

IMRC_SO2_CNT > IMSO_CNT_LVL ------------| IMRC_BK2_SOP := 1

|  ;bank 2 is stuck open

|

| --- ELSE ---

|

| IMRC_BK2_SOP := 0

|  ;Bank 2 is not stuck open

Compare each banks stuck closed fault counters  to  the  stuck  closed  fault

count  threshold.  If the levels are exceeded, then set the bank stuck closed

flag.

IMRC_SC2_CNT > IMSC_CNT_LVL ------------| IMRC_BK2_SCL := 1

|  ;bank 2 is stuck closed

|

| --- ELSE ---

|

| IMRC_BK2_SCL := 0

|  ;Bank 2 is not stuck closed

END: imrc_diagnostics_bank2

18-24

INTAKE MANIFOLD RUNNER CONTROL STRATEGY, DIAGNOSTICS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: imrc_obdii_code_elect

If the bank stuck open flags are set, then set the appropriate  MIL  function

codes.   If  the  bank  stuck  closed flags are set, then set the appropriate

Non-MIL function codes.   Currently,  all  electric  IMRC  applications  have

position sensors on only one bank.

IMRC_BK1_SOP = 1 -----------------------| Do: malfunction(CCM,P1518)

;Bank 1 stuck open                     |  ;plates stuck open

|  ;set MIL malfunction flag

|

| --- ELSE ---

|

| clear_malf(P1518)

|  ;clear malfunction flag

IMRC_BK1_SCL = 1 -----------------------| store_code(P1519)

;Bank 1 stuck closed                   |  ;plates stuck closed

|  ;set Non-MIL malfunction flag

|

| --- ELSE ---

|

| clear_malf(P1519)

|  ;clear malfunction flag

END: imrc_obdii_code_elect

18-25

INTAKE MANIFOLD RUNNER CONTROL STRATEGY, DIAGNOSTICS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: imrc_obdii_code_vac

If the bank stuck/open stuck closed flags are set, then set  the  appropriate

MIL  function  codes  and  Non-MIL function codes.  For both banks stuck open

failure and one bank stuck open failure, calibratable switches  are  provided

to  choose  between MIL and Non-MIL codes.  Stuck closed failures are Non-MIL

codes.

IMRC_BK1_SCL = 1 -----------------------| store_code(P1512)

;Bank 1 stuck closed                   |  ;set Non-MIL malfunction flag

|

| --- ELSE ---

|

| clear_malf(P1512)

|  ;clear malfunction flag

IMRC_BK2_SCL = 1 -----------------------| store_code(P1513)

;Bank 2 stuck closed                   |  ;set Non-MIL malfunction flag

|

| --- ELSE ---

|

| clear_malf(P1513)

|  ;clear malfunction flag

IMRC_BK1_SOP = 1 -----------------------| Do: malfunction(CCM,P1537)

;Bank 1 stuck open                     |  ;set MIL malfunction flag

|

| --- ELSE ---

|

| clear_malf(P1537)

|  ;clear malfunction flag

IMRC_BK2_SOP = 1 -----------------------| Do: malfunction(CCM,P1538)

;Bank 2 stuck open                     |  ;set MIL malfunction flag

|

| --- ELSE ---

|

| clear_malf(P1538)

|  ;clear malfunction flag

END: imrc_obdii_code_vac

18-26

INTAKE MANIFOLD RUNNER CONTROL STRATEGY, INPUT LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

18.1.3  IMRC INPUT LOGIC (CDAM0)

OVERVIEW

This IMRC module determines the measured position of the IMRC valves  and  is

called  from  the  IMRC  command  module  if the sensor H/W present switch is

enabled (IMRCSENSHP = 1).  It is configured for IMRC systems with either  one

or  two  sensors.  This configuration is detemined by the calibration switch,

IMRC_TWOPOSN, such that,

IMRC_TWOPOSN = 1 -> Two position sensors

IMRC_TWOPOSN = 0 -> One position sensor.  (Default)

In either case, the HWINT module passes two 10-bit (0  ->  1023  counts)  A/D

values  from ACH-13 and ACH-14.  These count values, IMRCP1_CNTS (ACH-13) and

IMRCP2_CNTS (ACH-14), represent the voltage signals from the sensors on  bank

1  and  2 respectively.  If the system is configured for one sensor, then the

actual sensor is read from ACH-13.

Regardless of  the  sensor  number,  this  module  generates  three  measured

position outputs of the following form:

1 -> IMRC valve measured open.

0 -> IMRC valve measured closed.

There are two measured bank positions, IMRC_POSN_1 and IMRC_POSN_2, which are

used for OBDII diagnostics.  If only one sensor is being used (IMRC_TWOPOSN =

0), then the bit flag, IMRC_POSN_1  is  copied  to  IMRC_POSN_2.   The  third

measured  position  output of this module, IMRC_POSN, is used by the variable

cam timing strategy and the IMRC anti-sludging logic.  The IMRC anti-sludging

logic is found in the the IMRC command module.

The measured position is determined by comparing the A/D counts to an "closed

counts window" and an "open counts window".  For bank 1,

IMRC_CNT1 <= IMRCP1_CNTS <= IMRC_CNT2 -> IMRC_POSN_1 = 0

IMRC_CNT3 <= IMRCP2_CNTS <= IMRC_CNT4 -> IMRC_POSN_1 = 1

There is no diagnostic support for partially open/closed valves.

18-27

INTAKE MANIFOLD RUNNER CONTROL STRATEGY, INPUT LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  IMRC1_FL_TMR = IMRC1 sensor counts out-of-range timer.

-  IMRC2_FL_TMR = IMRC2 sensor counts out-of-range timer.

-  IMRCP1_CNTS(Input) = A/D counts of bank 1 position sensor.

-  IMRCP2_CNTS(Input) = A/D counts of bank 2 position sensor.

Bit Flags:

-  IMRC_POSN(Output) = 1 -> All banks measured open, 0 -> Not all banks

are measured open.

-  IMRC_POSN_1(Output) = Measured bank 1 valve position; 1 -> open.

-  IMRC_POSN_2(Output) = Measured bank 2 valve position; 1 -> open.

-  KAM_ERROR = INDICATES KEEP ALIVE RAM INVALID.

-  OBDII_RESET = Flag used to simulate the receipt of an OBD-II

scan tool reset message.

Calibrations Constants:

-  IMRC_CNT1 = Low  count value of measured closed window.

-  IMRC_CNT2 = High count value of measured closed window.

-  IMRC_CNT3 = Low count value of measured open window.

-  IMRC_CNT4 = High count value of measured open window.

-  IMRC_FL_TM = IMRC input out of range error time.

-  IMRC_TWOPOSN = Two IMRC position sensors present switch;

1 -> two sensors are being used. 0 -> 1 sensor being used.

OTHER

-  P1516 = Bank 1 input error.

-  P1517 = Bank 2 input error.

18-28

INTAKE MANIFOLD RUNNER CONTROL STRATEGY, INPUT LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: IMRC_INPUT_COM2

BEGIN: imrc_input_main

;This process is executed only when explicitly called.

In the event of an OBDII_RESET or a KAM_ERROR, reset  all  fault  timers  and

clear the malfuntion bits.

OBDII_RESET = 1 ------------|

|OR --| IMRC1_FL_TMR := 0

KAM_ERROR = 1 --------------|     | IMRC2_FL_TMR := 0

| Do: clear_malf(P1516)

| Do: clear_malf(P1517)

Determine the measured valve position of bank 1,  IMRC_POSN_1,  by  comparing

the  sensor  counts, IMRCP1_CNTS, to the "closed window counts" and the "open

window counts".

IMRCP1_CNTS >= IMRC_CNT1 ---|

|AND -| IMRC_POSN_1 := 0

IMRCP1_CNTS <= IMRC_CNT2 ---|     |  ;IMRC bank 1 position is

|  ;measured closed

| IMRC1_FL_TMR := 0

|

| --- ELSE ---

IMRCP1_CNTS >= IMRC_CNT3 ---|     |

|AND -| IMRC_POSN_1 := 1

IMRCP1_CNTS <= IMRC_CNT4 ---|     |  ;IMRC bank 1 position is

|  ;measured open

| IMRC1_FL_TMR := 0

|

| --- ELSE ---

|

| Allow IMRC1_FL_TMR to increment

| as a free-running timer.

|  ;Bank 1 input error detected.

18-29

INTAKE MANIFOLD RUNNER CONTROL STRATEGY, INPUT LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

If only one sensor is being used, then copy the  bank  1  measured  position,

IMRC_POSN_1,  to  the bank 2 measured position, IMRC_POSN_2.  If two position

sensors are being used, then use the same algorithm as bank 1 for determining

the measured position.

IMRC_TWOPOSN = 0 -----------------| IMRC_POSN_2 := IMRC_POSN_1

;There is only one sensor        |  ;Set IMRC bank 2 valave position

|  ;equal to bank 1 valve position

| IMRC2_FL_TMR := 0

|

| --- ELSE ---

IMRCP2_CNTS >= IMRC_CNT1 ---|     |

|AND -| IMRC_POSN_2 := 0

IMRCP2_CNTS <= IMRC_CNT2 ---|     |  ;IMRC Bank 2 position is

|  ;measured closed

| IMRC2_FL_TMR := 0

|

| --- ELSE ---

IMRCP2_CNTS >= IMRC_CNT3 ---|     |

|AND -| IMRC_POSN_2 := 1

IMRCP2_CNTS <= IMRC_CNT4 ---|     |  ;IMRC Bank 2 position is

|  ;measured open

| IMRC2_FL_TMR := 0

|

| --- ELSE ---

|

| Allow IMRC2_FL_TMR to increment

| as a free-running timer.

|  ;Bank 2 input error detected.

Determine the value of IMRC_POSN for variable cam  timing  and  anti-sludging

routine.  If both valves are measured open, then the IMRC_POSN is "open".

IMRC_POSN_1 = 1 ------------|

;IMRC bank 1 measured open |AND -| IMRC_POSN := 1

|     |  ;IMRC valve is open

IMRC_POSN_2 = 1 ------------|     |

;IMRC bank 2 measured open       |

| --- ELSE ---

|

| IMRC_POSN := 0

|  ;IMRC valve is closed

18-30

INTAKE MANIFOLD RUNNER CONTROL STRATEGY, INPUT LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

IMRC1_FL_TMR >= IMRC_FL_TM -------| Do: malfunction(CCM,P1516)

;IMRC1 input out of range        |

;too long                        |

| --- ELSE ---

|

| Do: clear_malf(P1516)

IMRC2_FL_TMR >= IMRC_FL_TM -------| Do: malfunction(CCM,P1517)

;IMRC2 input out of range        |

;too long                        |

| --- ELSE ---

|

| Do: clear_malf(P1517)

END: imrc_input_main

18-31

INTAKE MANIFOLD RUNNER CONTROL STRATEGY, DRIVER DIAGNOSTICS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

18.1.4  IMRC DRIVER DIAGNOSTICS (CDAM0)

OVERVIEW

This IMRC module provides diagnostics for the IMRC output driver.  The  HWINT

module  passes  this module the IMRC driver fault bit flag, IMRC_FAULT, which

is defined as follows:

A fault is present -> IMRC_FAULT = 1

No fault is present -> IMRC_FAULT = 0

The IMRC driver fault filter, P1520FIL, is the output of a first order low

pass filter routine, ROLAV, which is filtererd towards 1 if, IMRC_FAULT = 1,

or towards 0 if, IMRC_FAULT = 0.  When the IMRC driver fault filter passes

the calibrated threshold, P1520_ERRLVL, the malfunction code, P1520, is set.

An IMRC driver fault hysterisis calibration, P1520HYS, is provided to

prevent jittering of the malfunction flag (P1520MALF).

DEFINITIONS

Registers:

-  P1520FIL = Value of driver fault error filter.

Bit Flags:

-  IMRC_FAULT(Input) = Driver fault status.

-  KAM_ERROR = INDICATES KEEP ALIVE RAM INVALID.

-  OBDII_RESET = Flag used to simulate the receipt of an OBD-II

scan tool reset message.

Calibration Constants:

-  IMRC_FALT_TC = Fault filter time constant.

-  P1520_ERRLVL = Threshold level to store code for output driver fault.

-  P1520HYS = Filter hysterisis for driver fault filter.

OTHER

-  P1520 = IMRC driver fault malfunction code.

18-32

INTAKE MANIFOLD RUNNER CONTROL STRATEGY, DRIVER DIAGNOSTICS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: IMRC_DIAG_OSM_COM1

BEGIN: imrc_diagnostics_osm_main

;This process is executed only when explicity called.

In the event of an OBDII_RESET or KAM_ERROR, reset the fault filter and clear

the IRMC driver fault flag.

OBDII_RESET = 1 ------------------|

|OR --| Do: clear_malf(P1520)

KAM_ERROR = 1 --------------------|     | P1520FIL := 0

Determine the operational status of the IMRC output driver.

IMRC_FAULT = 0 -------------------------| P1520FIL := ROLAV(0,IMRC_FALT_TC)

;no IMRC driver fault present          |  ;filter toward zero

|

|

| --- ELSE ---

|

|

IMRC_FAULT = 1 -------------------------| P1520FIL := ROLAV(1,IMRC_FALT_TC)

;an IMRC  driver fault is present      |  ;filter toward one

If the fault is present long enough to cause the fault  filter  to  exceed  a

threshold value, set the malfunction flag.

P1520FIL > P1520_ERRLVL ----------------| Do: malfunction(CCM,P1520)

|  ;set IMRC driver fault flag

|

|

| --- ELSE ---

|

|

P1520FIL < P1520_ERRLVL - P1520HYS -----| Do: clear_malf(P1520)

|  ;clear IMRC driver fault flag

END: imrc_diagnostics_osm_main

18-33

INTAKE MANIFOLD RUNNER CONTROL STRATEGY, OUTPUT LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

18.1.5  IMRC OUTPUT CONTROL LOGIC (CDAL0)

OVERVIEW

This IMRC module deteremines if the IMRC  solenoid  should  be  energized  by

comparing  the  commanded IMRC plate position, IMRC_CMND, to the de-energized

IMRC position, IMRC_OFF_POS.  If the commanded position is the  same  as  the

de-energized  position, then the solendoid input, IMRC_OUT, is cleared to the

de-energized state, IMRC_OUT = 0.  If the commanded position is not the  same

as the de-energized position, then the solenoid input is set to the energized

state, IMRC_OUT = 1.

DEFINITIONS

Bit Flags:

-  IMRC_CMND(Input) = Commanded IMRC valve postion; 1 -> open, 0 -> closed.

-  IMRC_OUT(Output)  =  Commanded  solenoid  state;  1  ->  energize  0   ->

de-energize.

Calibration Constants:

-  IMRC_OFF_POS = De-energized position; 1 -> open, 0 -> closed.

PROCESS

STRATEGY MODULE: IMRC_OUTPUT_COM1

BEGIN: imrc_output_main

;This process is executed only when explicitly called.

IMRC_CMND = IMRC_OFF_POS ---------------| IMRC_OUT := 0

;IMRC command is the de-energized      |  ;De-energize IMRC

;position                              |

| --- ELSE ---

|

| IMRC_OUT := 1

|  ;Energize IMRC

END: imrc_output_main

18-34

CHAPTER 19

DATA OUTPUT LINK

19-1

DATA OUTPUT LINK, PULSE CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

19.1  PULSE CALCULATION (CDAI1)

OVERVIEW

The Data Output Link (DOL)  provides  fuel  consumption  information  to  the

vehicle  dashboard  fuel  economy  display  products  (Tripminder  or Message

Center).  The output, in the form of pulses, represents the  amount  of  fuel

used  since  the  last  update.  This information is used in calculating fuel

economy and distance-to-empty for display to the driver.

The injector fuel  flow  (in  lbmf/(injector/port))  is  accumulated  in  the

register  FUEL_SUM  every  time  an  injector  port  is  energized.   Once  a

background loop, FUEL_SUM is converted to the appropriate integer  number  of

DOL  pulses,  and DOL_COUNT is updated according to the equation shown below.

The amount of FUEL_SUM which cannot be converted to an integer count  remains

in FUEL_SUM for the next conversion.

Approximately every two milliseconds, the value of DOL_COUNT is checked.   If

DOL_COUNT  is  greater  than  1.0,  the DOL output is energized for about one

msec, and DOL_COUNT is reduced by 1.0.

DEFINITIONS

INPUTS

Registers:

-  DOL_COUNT = Number of pulses to be output to  the  Fuel  Economy  display

device.

-  FUEL_SUM = Accumulated fuel mass for one injector per output  port  since

the  last  DOL_COUNT  update,  including  the remainder from the previous

conversion, units in lbmf/(injector/port).

Calibration Constants:

-  DOLHP = Data Output Link "hardware present" indicator.  (0 = NO, 1 = YES)

-  PUL_PER_GAL = Number of DOL pulses to be issued for each gallon  of  fuel

used, pulse per gal.

NOTE:  THE VALUE FOR PUL_PER_GAL MUST  BE  OBTAINED  FROM  EED/INSTRUMENT

SYSTEMS FOR EACH APPLICATION.

OUTPUTS

Registers:

-  DOL_COUNT = See above.

-  FUEL_SUM = See above.

19-2

DATA OUTPUT LINK, PULSE CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: DOL_PULSE_CALC_COM4

Once per background loop execute the following:

DOLHP = 1 ----------------| [pulses] :=

|              FUEL_SUM

|   --------------------------------

|   (6.15 lbmf/gallon) / PUL_PER_GAL

|

| DOL_COUNT := DOL_COUNT + integer([pulses])

|

| FUEL_SUM := FUEL_SUM -

|   (integer([pulses]) * 6.15 / PUL_PER_GAL)

19-3

DATA OUTPUT LINK, PULSE CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

19-4

CHAPTER 20

RIDE CONTROL

20-1

RIDE CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

20.1  ADJUSTABLE SHOCK ABSORBER OUTPUT (CDAB0)

OVERVIEW

The adjustable shock absorber module is a stand alone module,  separate  from

the  EEC  IV,  which controls shock absorber firmness.  The EEC-IV ACL output

provides an indication of vehicle acceleration to the shock  control  module.

In  some  vehicles,  the state of this output is sent to the adjustable shock

absorber module via  the  Standard  Corporate  Protocol  (SCP)  network.   In

others,  the  ACL state is sent via a hardwired output from the EEC-IV to the

ride control module.

Other inputs to the shock module come directly from  the  appropriate  sensor

(VSS,  brake  input,  etc.).  The ACL output will be set under engine running

conditions by the logic shown below to indicate hard acceleration.   The  ACL

output  may  also  be  set  during key on, engine off conditions by a high TP

value in order to verify wiring harness (or SCP link) integrity during VIP.

DEFINITIONS

INPUTS

Registers:

-  CYLARC_BG = Air Charge Mass inducted per Intake Stroke, lbm/Intake.

-  TP_REL = Relative Throttle Position, counts.

Bit Flags:

-  CRKFLG = Indicates status of engine; 1 -> engine is in CRANK mode.

Calibration Constants:

-  HPACL = Hardware present switch to indicate if module includes output for

adjustable shock absorber; 1 -> Hardware present.

-  SHK_CHG = Minimum CYLARC_BG  value  to  indicate  if  hard  acceleration.

Typical value:  .00015 lbm/pip.

-  SHKTP = Minimum number of  throttle  counts  above  the  closed  throttle

position  (RATCH)  to  indicate  hard  acceleration.  Typical value:  650

counts.

OUTPUTS

Bit Flags:

-  ACL = Flag indicating vehicle acceleration to the shock control module.

20-2

RIDE CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: RIDE_CONTROL_COM4

Determine the state of ride control output (Hardwire or SCP).

HPACL = 0 ------------------------------| EXIT

| (no change)

|

| --- ELSE ---

CRKFLG = 0 -----------------|           |

(Underspeed/Run Modes)     |AND -|     |

|     |     |

CYLARC_BG >= SHK_CHG -------|     |OR --| ACL = 1

(high engine load)               |     |

|     |

|     | --- ELSE ---

|     |

TP_REL >= SHKTP ------------------|     |

(Large throttle opening)               | ACL = 0

20-3

RIDE CONTROL STRATEGY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

20-4

CHAPTER 21

EXPORT FEATURES

21-1

EXPORT FEATURES, SPEED WARNING CHIME - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

21.1  SPEED WARNING CHIME (CDAJ0)

OVERVIEW

Saudi Arabia has a legal requirement for a warning chime to  sound  when  the

vehicle  speed  exceeds  75  mph.   This  strategy  operates  a  chime when a

calibrateable vehicle speed has been exceeded.

In the event of a vehicle speed sensor fault, there will be no speed  warning

chime.

There is no selftest output circuit check on SWC, however for  test  purposes

the chime can be switched on through the throttle operated Output Test Mode.

DEFINITIONS

INPUTS

Registers:

-  VSBAR = Filtered Vehicle Speed.

-  VSFMFLG = Vehicle speed sensor failure mode flag.

Calibration Constants:

-  SWC_HP = Speed Warning Chime hardware present switch; 1 -> Speed  warning

chime hardware present, 0 -> No speed warning chime hardware present.

-  SWCVS_SH = Minimum vehicle speed to turn on Speed Warning Chime, mph.

-  SWCVS_CL = Maximum vehicle speed to turn off Speed Warning Chime, mph.

OUTPUTS

Bit Flags:

-  SWC = Flag which determines the state of the speed warning  chime;  1  ->

Chime is on, 0 -> Chime is off.

-  SWCVS_FLG  =  Flag  indicating  warning  chime  vehicle  speed  has  been

exceeded; 1 -> vehicle speed exceeded, 0 -> vehicle speed not exceeded.

21-2

EXPORT FEATURES, SPEED WARNING CHIME - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EXP_VS_CHIME_COM2

SWC_HP = 0 ----------------------| Exit module

;no chime hardware present

VSBAR > SWCVS_SH ----------------|S Q -| SWCVS_FLG

|

VSBAR < SWCVS_CL ----------------|C

VSFMFLG = 0 ---------------------| SWC := SWCVS_FLG

;no VSS fault present           |

| --- ELSE ---

|

| No operation on SWC

21-3

EXPORT FEATURES, SPEED WARNING CHIME - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

21-4

CHAPTER 22

ALTERNATIVE CALIBRATION

22-1

ALTERNATIVE CALIBRATION, CALIBRATION INITIALIZE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

22.1  CALIBRATION INITIALIZE LOGIC (CDAI0)

OVERVIEW

The purpose of the alternate calibration strategy is to run the vehicle in  a

pre-delivery condition in order to protect against spark plug fouling.

This module sets the flag which will invoke the alternate calibration.  To do

this, it looks at the value of the parameter NO_OF_STARTS which may have been

downloaded at end of line via SCP, and it also checks the check byte which is

the   one's  complement  of  NO_OF_STARTS.   The  value  of  NO_OF_STARTS  is

subsequently  decremented.   These  checks  are  performed  during  the   RAM

initialization process only.

DEFINITIONS

INPUTS

Registers:

-  NO_OF_STARTS = Number of starts using alternative calibration.

-  NO_START_CHK = Number of starts, check byte.

Bit Flags:

-  ALT_CAL_FLG = Flag to indicate use of alternate calibration.

OUTPUTS

Registers:

-  NO_OF_STARTS = See above.

-  NO_START_CHK = See above.

Bit Flags:

-  ALT_CAL_FLG = See above.

22-2

ALTERNATIVE CALIBRATION, CALIBRATION INITIALIZE LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: ALTR_CAL_INIT_COM1

Performed during RAM Initialization only.

NO_OF_STARTS > 0 ------------------|

|AND -| ALT_CAL_FLG = 1

NO_START_CHK = 255 - NO_OF_STARTS -|     |

| --- ELSE ---

|

| ALT_CAL_FLG = 0

ALT_CAL_FLG = 1 -------------------------| Decrement NO_OF_STARTS

| Increment NO_START_CHK

|

| --- ELSE ---

|

| Do nothing

22-3

ALTERNATIVE CALIBRATION, CALIBRATION CLEAR LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

22.2  CALIBRATION CLEAR LOGIC (CDAI0)

OVERVIEW

This module erases any remaining  alternate  calibration  starts  should  the

vehicle  travel  further  than ALT_CAL_DIST in any one journey.  The value of

ALT_CAL_FLG is also set each background loop to  reduce  the  effect  of  any

possible corruption of the register that may occur.

Should the VSS fail while using the alternate calibration, the distance  will

be  ignored  and  the alternate calibration will be exited immediately.  This

will prevent this calibration from being in the customers hands  and  prevent

the calibration from being active during an EOL emissions test if the VSS had

failed.

DEFINITIONS

INPUTS

Registers:

-  BG_TMR = Background loop timer.

-  DISTANCE = Distance traveled since start, miles.

-  NO_OF_STARTS = Number of starts using alternative calibration.

-  NO_START_CHK = Number of starts, check byte.

-  VSBAR = Filtered vehicle speed.

Calibration Constants:

-  ALT_CAL_DIST = Distance traveled before alternate calibration is revoked,

miles.

OUTPUTS

Registers:

-  DISTANCE = See above.

-  NO_OF_STARTS = See above.

-  NO_START_CHK = See above.

Bit Flags:

-  ALT_CAL_FLG = Flag to indicate use of alternate calibration.

22-4

ALTERNATIVE CALIBRATION, CALIBRATION CLEAR LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: ALTR_CAL_CLR_COM1

DISTANCE = DISTANCE + (VSBAR * BG_TMR) / 3600

VSFMFLG = 1 -----------------------|

|OR --| NO_OF_STARTS = 0

DISTANCE > ALT_CAL_DIST -----------|     | NO_START_CHK = 0

| ALT_CAL_FLG = 0

|

| --- ELSE ---

NO_OF_STARTS > 0 ------------------|     |

|AND -| ALT_CAL_FLG = 1

NO_START_CHK = 255 - NO_OF_STARTS -|     |

| --- ELSE ---

|

| ALT_CAL_FLG = 0

22-5

ALTERNATIVE CALIBRATION, CALIBRATION CLEAR LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

22-6

CHAPTER 23

OBDII STRATEGY

23-1

OBDII STRATEGY

23.1  FEATURE:  OBDII EXECUTIVE - V1.4B_OBDII_EXEC (CDAN0)

23.1.1  DIAGNOSTIC SCHEDULER (CDAN0)

OVERVIEW

+---------------------------------+

| OBDII_SCHEDULER_COM1            |

+---------------------------------+

| PUBLIC PROCESSES:               |

| . definitions                   |

| . scan_tool_interface           |

| . diagnostic_scheduler_control  |

| . determine_demand_mode         |

| . parameter_reset_signal_control|

+---------------------------------+

| PRIVATE PROCESSES:              |

| . diagnostic_scheduler          |

| . disable_all_monitors          |

| . enable_cont_monitors          |

| . scheduler_delay               |

+---------------------------------+

The OBD-II  Executive  Diagnostic  Scheduler  is  the  software  module  that

controls  and  coordinates  the  execution  of  the  individual OBD-II system

monitors (EGO, CAT, PURG, SAIR, FUEL, MIS, EGR, CCM) and the  On-Demand  self

test  procedures  (ER,  EO,  OTM).  The diagnostic scheduler enables/disables

diagnostic tests in order to effect the following requirements:

-  Provide mutual exclusion between  tests  that  are  intrusive,  or  would

otherwise interfere with one another.

-  Ensure that all OBD-II required tests have an opportunity to  run  during

the first two bags of a CVS-75 test.

-  Sequence OBD-II tests such that when a test  runs,  each  input  that  it

relies upon has already been tested.

-  Interface with the scan tool to coordinate  the  performance  of  special

requests or tests.

-  Perform elegant transitions between the various states of the  diagnostic

system, so as not to adversely affect the operation of the vehicle.

-  Allow the engine-off, engine-running, and  output  test  mode  self  test

routine  to execute.  All control of other monitors is turned over to the

individual sequence controller for the diagnostic procedure.

The diagnostic executive  is  the  realization  of  a  finite  state  machine

controlling  the  sequencing  and  mutual  exclusion needed for the intrusive

tests.  See the finite state machine diagrams on the following pages for more

detail on how the system functions.

23-2

OBDII STRATEGY, DIAGNOSTIC SCHEDULER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The  diagnostic  scheduler  also  monitors  the  scan   tool   interface   in

anticipation  of  an OBD-II reset command from the scan tool.  When an OBD-II

reset message is received from the scan  tool,  the  strategy  takes  several

actions  in  order  to  place  the  OBD-II  system in a state of reset.  This

includes resetting both the trip monitoring system, and the MIL  controllers.

The  SCP  communications  interface  sets  the  OBDII_RESET flag when a reset

message is received.  The logic  below  clears  the  flag,  and  effects  the

required reset actions.

There is a manual mode of operation available to the developer.  This can  be

invoked  in  one  way:  through the calibration of the sys_TST_SW bits.  When

the corresponding bit of a system monitor is set, the monitor is enabled  and

the  test  will be executed.  By clearing all but one of the sys_TST_SW bits,

only one test will ever be  executed.   The  automatic  scheduling  of  tests

continues in this mode, only scheduling those which have been selected.

The enable flags are mapped into the byte called OBDII_TST_SW as follows:

bit 0 = CAT_TST_SW

bit 1 = CCM_TST_SW

bit 2 = PURG_TST_SW

bit 3 = SAIR_TST_SW

bit 4 = FUEL_TST_SW

bit 5 = EGO_TST_SW

bit 6 = EGR_TST_SW

bit 7 = MIS_TST_SW

For development purposes, the OBD-II system may be placed in a state of reset

by writing a 1 to the OBDII_RESET flag with an RCON.  This will have the same

affect as receipt of the reset message from the scan tool.

23-3

OBDII STRATEGY, DIAGNOSTIC SCHEDULER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Below is a description of how a finite state machine functions.

A finite state machine is a way to graphically illustrate the functioning  of

a  system  or  process  that can be in a set of DISCRETE states.  The diagram

shows how and when transitions can occur between the states.

+----------------------+

|                      |

|       STATE X        |<-------------------

|                      |        condition

+-----+----------A-----+        ---------

|          |               action

|          |

condition    |          |    condition

---------    |          |    ---------

action      |          |     action

|          |

|          |

+-----V----------+-----+

|                      |

|       STATE Y        |

|                      |

+----------------------+

CONVENTIONS:

· BOXES represent STATES of the system.

· ARROWS represent TRANSITIONS between states.

· CONDITIONS will CAUSE a transition to be taken.

· ACTIONS are done WHEN a transition is taken.

RULES:

· The system can be in only one state at a time.

· The system remains in a given state until an event

occurs to cause a transition.

· No other transitions than those shown can be taken.

23-4

OBDII STRATEGY, DIAGNOSTIC SCHEDULER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Finite State Machine Diagram

ENGINE OFF On-Demand Test Control

EO_TST_RDY=0 &

SCPEO_START=1

-------------

send(GENERAL_RESPONSE:

7Fh,31h,81h,00h,00h,22h)

SCPEO_START=0

+------+

|      |

|   +--V--------+

+---| any state |

|  except   |

| eo_running|

+-----+-----n

|

| EO_TST_RDY=1 &

| SCPEO_START=1

| -------------

| send(GENERAL_RESPONSE:

|  7Fh,31h,81h,00h,00h,00h)

| disable_all_monitors

| EO_TST_ENA=1

| SCPEO_START=0

| inc_warmup_counters

| RESULTS=0

|

+-----V-----+

| eo_running|

+------------------------|  state    |----------------+

|                        +-----+---- 8                |

|                              |                      |

| SCPEO_EXIT=1                 |EO_MON=1              | EO_TST_RDY=0

| ------------                 |------------          | ------------

| EO_TST_ENA=0                 |EO_TST_ENA=0          | EO_TST_ENA=0

| send(GENERAL_RESPONSE:       |EO_MON=0              |

|  7Fh,32h,81h,00h,00h,64h)    |RESULTS=1             |

| SCPEO_EXIT=0                 |                      |

|                              |                      |

+--------------------------+   |   +------------------+

|   |   |

+-V---V---V-+

|  initial  |

|   state   |

+-----------0

23-5

OBDII STRATEGY, DIAGNOSTIC SCHEDULER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Finite State Machine Diagram

ENGINE ON On-Demand Test Control

ER_TST_RDY=0 &

SCPER_START=1

-------------

send(GENERAL_RESPONSE:

7Fh,31h,82h,00h,00h,22h)

SCPER_START=0

+------+

|      |

|   +--V--------+

+---| any state |

|  except   |

| er_running|

+-----+-----n

|

|

| ER_TST_RDY=1 &

| SCPER_START=1

| -------------

| send(GENERAL_RESPONSE:

|  7Fh,31h,82h,00h,00h,00h)

| disable_all_monitors

| ER_TST_ENA=1

| SCPER_START=0

| inc_warmup_counters

| RESULTS=0

|

+-----V-----+

| er_running|

+------------------------|  state    |----------------+

|                        +-----+---- 7                |

|                              |                      |

| SCPER_EXIT=1                 |ER_MON=1              | ER_TST_RDY=0

| ------------                 |------------          | ------------

| ER_TST_ENA=0                 |ER_TST_ENA=0          | ER_TST_ENA=0

| send(GENERAL_RESPONSE:       |ER_MON=0              |

|  7Fh,32h,82h,00h,00h,64h)    |RESULTS=1             |

| SCPER_EXIT=0                 |                      |

|                              |                      |

+--------------------------+   |   +------------------+

|   |   |

+-V---V---V-+

|  initial  |

|   state   |

+-----------0

23-6

OBDII STRATEGY, DIAGNOSTIC SCHEDULER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Finite State Machine Diagram

OUTPUT TEST MODE On-Demand Test Control

OTM_TST_RDY=0 &

SCPOTM_START=1

-------------

send(GENERAL_RESPONSE:

7Fh,31h,84h,00h,00h,22h)

SCPOTM_START=0

+------+

|      |

|   +--V--------+

+---| any state |

|  except   |

|otm_running|

+-----+-----n

|

| OTM_TST_RDY=1 &

| SCPOTM_START=1

| -------------

| send(GENERAL_RESPONSE:

|  7Fh,31h,84h,00h,00h,00h)

| disable_all_monitors

| OTM_TST_ENA=1

| SCPOTM_START=0

| inc_warmup_counters

| RESULTS=0

|

+-----V-----+

|otm_running|

+------------------------|  state    |

|                        +-----+---- 9

|                              |

| SCPOTM_EXIT=1                | OTM_TST_RDY=0

| ------------                 | ------------

| OTM_TST_ENA=0                | OTM_TST_ENA=0

| send(GENERAL_RESPONSE:       |

|  7Fh,32h,84h,00h,00h,61h)    |

| SCPOTM_EXIT=0                |

| RESULTS=1                    |

+--------------------------+   |

|   |

+-V---V-----+

|  initial  |

|   state   |

+-----------0

23-7

OBDII STRATEGY, DIAGNOSTIC SCHEDULER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Finite State Machine Diagram

EGO Monitor Control

+-----------+

|           |<--- .PUTMR<4 seconds  OR

|  initial  |     .On-Demand Exit OR

|   state   |     .OBDII_RESET=true OR

+----------------|           |     .KAM_ERROR=true OR

|                +--+--------0     --------------------

|                   |              .disable_all_monitors

·UNDSP=0 &          |   ·UNDSP=0 &      |

·PUTMR>=4 &         |   ·PUTMR>=4 &     |

·EGO_TST_SW=0       |   ·EGO_FMEM=0 &   |

OR EGO_MON=1       |   ·EGO_TST_SW=1 & |

-----------------   |   ·EGO_MON=0 &    |

·EGO_TST_ENA=0      |   --------------- |          ·EGO_DEL_TMR=0 &

|   ·EGO_TST_ENA=1  |          ·EGO_FMEM=0

|                +--v--------+  ------------------

|                |EGO monitor| ·EGO_TST_ENA=1

|                |  enabled  |<--------------------+

|                +----+----- 1                     |

|                     |         +-----------+      |

|     ·EGO_TST_RDY=1  |         |EGO monitor|------+

|     --------------  |         |  aborted  |<-----+

|     ·change state   |         +---------- 3      |

|                +----v------+                     |

|                |EGO monitor|                     |

|                |  running  |---------------------+

|                +----+----- 2   ·EGO_FMEM=1 OR

|   ·EGO_MON=1        |          ·EGO_MON=0 &

|   --------------    |           EGO_TST_RDY=0

|   ·EGO_TST_ENA=0    |           ----------------

|                     |          ·EGO_TST_ENA=0

+-------------------+ |          ·EGO_DEL_TMR=EGO_DEL_TM

| |

A B

(continued on next page)

23-8

OBDII STRATEGY, DIAGNOSTIC SCHEDULER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Finite State Machine Diagram

SAIR/PURG Monitor Control

and Mutual Exclusion

(continued from previous page)

A B

| |

| |

·SAIR_TST_RDY=0 OR        | |      ·PURG_TST_RDY=0 OR

·SAIR_FMEM=1 OR           | |      ·PURG_FMEM=1 OR

·SAIR_MON=1               | |      ·PURG_MON=1

------------------------- | |      ------------------------

·SAIR_DEL_TMR=SAIR_DEL_TM | |      ·PURG_DEL_TM=PURG_DEL_TM

·SAIR_TST_ENA=0           | |      ·PURG_TST_ENA=0

+------------------+  | |  +-----------------------+

|                  |  | |  |                       |

|  +-----------+   |  | |  |      +-----------+    |

+--|SAIRmonitor|   |  | |  |      |PURGmonitor|----+

+->|  running  |   |  | |  |      |  running  |<---+

|  +---------- 6   |  | |  |      +---------- 5    |

|                 +v--v-v--v--+                    |

|                 | SAIR/PURG |                    |

+-----------------|ARBITRRATOR|--------------------+

+---------- 4

|

·SAIR_TST_SW=1 &        |          ·PURG_TST_SW=1 &

·SAIR_TST_RDY=1 &       |          ·PURG_TST_RDY=1 &

·SAIR_FMEM=0 &          |          ·PURG_FMEM=0 &

·SAIR_DEL_TMR=0 &       |          ·PURG_DEL_TMR=0 &

·SAIR_MON=0             |          ·PURG_MON=0

-----------------       |          ------------------

·SAIR_TST_ENA=1         |          ·PURG_TST_ENA=1

|

|

·PURG_MON=1 OR     |

PURG_TST_SW=0 &   |

·SAIR_MON=1 OR     |

SAIR_TST_SW=0 &   |

|

+-----v-----+

|   Final   |

|   state   |

+---------- 10

23-9

OBDII STRATEGY, DIAGNOSTIC SCHEDULER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  ATMR3 = Time in run mode.

-  CCM_TST_ENA = OBD-II Comprehensive Component Test enable flag; 1 ->  test

enabled.

-  CCM_TST_RDY = CCM test ready flag; 1 -> all local conditions have  be  en

met; rea dy to run CCN test.

-  EGO_DEL_TMR = Time remaining before re-attempting EGO Monitor enablement.

-  EGO_TST_RDY = EGO test ready flag; 1 -> all local  conditions  have  been

met; ready to run EGO test.

-  EGR_TST_ENA = OBD-II EGR Test enable flag; 1 -> test enabled.

-  EGR_TST_RDY = EGR test ready flag; 1 -> all local  conditions  have  been

met; ready to run EGR test.

-  EO_STATUS = State pointer that indicates current state of engine  off  on

demand test sequence.

-  ER_STATUS = State pointer that indicated current state of engine  running

on demand test.

-  FUEL_TST_ENA = OBD-II Fuel Test enable flag; 1 -> test enabled.

-  FUEL_TST_RDY = Fuel system test ready flag; 1  ->  all  local  conditions

have been met; ready to run fuel test.

-  KAM_ERROR = Indicates keep alive RAM invalid.

-  MIS_TST_ENA = OBD-II Misfire Test enable flag; 1 -> test enabled.

-  OTM_TST_ENA = Flag to activate the output test mode on demand test.

-  OTM_TST_RDY = Flag to signal the OBD-II executive that  the  output  test

mode is ready.

-  PURG_DEL_TMR = Minimum possible time remaining before re-attempting  PURG

Monitor enablement.

-  PURG_TST_RDY = PURG test ready flag; 1 -> all local conditions have  been

met; ready to run PURG test.

-  PUTMR = Time after CPU power up.

-  SAIR_DEL_TMR = Minimum possible time remaining before re-attempting  SAIR

Monitor enablement.

-  SAIR_TST_RDY = SAIR test ready flag; 1 -> all local conditoins have  been

met; ready to run test.

23-10

OBDII STRATEGY, DIAGNOSTIC SCHEDULER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Bit Flags:

-  CAT_FMEM = CAT system FMEM flag; 1 -> don't run CAT system due to FMEM.

-  CAT_TST_ENA = OBDII catalyst test enable flag; 1 -> test enabled.

-  CCM_FMEM = CCM system FMEM flag; 1 -> don't run CCM due to FMEM.

-  DEMAND_MODE = Flag set when an On-Demand self test mode is active.

-  EGO_FMEM = EGO system FMEM flag; 1 -> don't run EGO monitor due to FMEM.

-  EGO_MON = OBDII monitor flag for EGO system; 1 -> all  fuel  faults  been

monitored since power-up.

-  EGO_TST_ENA = OBDII EGO test enable flag; 1 -> test enabled.

-  EGR_FMEM = EGR system FMEM flag; 1 -> don't run EGR monitor due to FMEM

-  EO_MON = Flag indicating the engine off on-demand test is complete.

-  EO_TST_ENA = Flag to activate engine off on demand test.

-  EO_TST_RDY = Flag that signals OBD-II executive that the engine off  test

is able to run.

-  ER_MON = Flag indicating the engine running on demand test is complete.

-  ER_TST_ENA = Flag to signal engine running on demand test to run.

-  ER_TST_RDY = Flag to signal OBD-II executive that the engine  running  on

demand test is able to run.

-  FLG_CONT_MON = Flag set when continuous monitors are enabled in the OBDII

system.

-  FUEL_FMEM = FUEL system FMEM flag; 1 -> don't run  FUEL  monitor  due  to

system FMEM.

-  MIS_FMEM = MIS system FMEM flag; 1 -> don't run MIS monitor due to FMEM.

-  OBDII_RESET = Flag used to simulate the erceipt of an  OBD-II  scan  tool

reset message.

-  OBDII_STATE = Current state of the OBDII scheduler.

-  OBD_PARM_RST = OBDII parameter reset signal; 1-> reset parameters.

-  PURG_FMEM = PURG system FMEM flag; 1 -> don't run  PURG  monitor  due  to

FMEM.

-  PURG_MON = OBDII monitor flag for  evaporative  purge;  1  ->  all  purge

faults been monitored since power-up.

-  PURG_TST_ENA = OBDII purg test enable flag.  (1 -> test enabled)

-  RESET_DONE = Flag used to notify that an OBD-II scan tool reset  occurred

the previous background loop.

23-11

OBDII STRATEGY, DIAGNOSTIC SCHEDULER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  RESULTS = Flag indicating self test results are available, 1  ->  results

available.

-  SAIR_FMEM = SAIR system FMEM flag; 1 -> don't run  SAIR  monitor  due  to

FMEM.

-  SAIR_MON = Secomdary air has been monitored for OBD-II trip.

-  SAIR_TST_ENA = OBDII secondary air test enable flag.  (1 -> test enabled)

-  SCPEO_EXIT = Siganl to abort the engine off self test.

-  SCPEO_START = Siganl to start the engine off self test.

-  SCPER_EXIT = Signal to abort the engine running self test.

-  SCPER_START = Signal to start the engine running self test.

-  SCPOTM_EXIT = Signal to abort the output test mode diagnostic procedure.

-  SCPOTM_START = Siganl to start output test mode diagnostic procedure.

-  UNDSP = Underspeed flag.

Calibration Constants:

-  CAT_TST_SW = CAT test present calibration switch; 1 -> CAT test present.

-  CCM_TST_SW = CCM test present calibration switch; 1 -> CCM test present.

-  ECU_EO_INIT = Value of EO_STATUS to signal  for  ECU  test  (Instruction,

KAM, RAM, ROM), pointer.

-  EGO_DEL_TM = Time delay between succesive runs of the EGO monitor.

-  EGO_TST_SW = EGO test present calibration switch; 1 -> EGO test present.

-  EGR_TST_SW = EGR test present calibration switch; 1 -> EGR test present.

-  EO_DONE = Value of EO_STATUS when engine off on demand t est is complete.

-  ER_DONE = Value of ER_STATUS when engine running test is complete.

-  ER_INIT = Value for state of ER_STATUS when  engine  running  test  first

inits.

-  FUEL_TST_SW = FUEL test  present  calibration  switch;  1  ->  FUEL  test

present.

-  MIS_TST_SW = MIS test present calibration switch; 1 -> MIS test present.

-  OBDII_DELAY = Time into run mode before enabling  any  continuous  OBD-II

monitor.

-  PURG_DEL_TM = Time delay between succesive runs of the PURG monitor.

-  PURG_TST_SW = PURG test  present  calibration  switch;  1  ->  PURG  test

present.

23-12

OBDII STRATEGY, DIAGNOSTIC SCHEDULER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  SAIR_DEL_TM = Time delay between succesive runs of the SAIR monitor.

-  SAIR_TST_SW = SAIR test  present  calibration  switch;  1  ->  SAIR  test

present.

OTHER

-  clear_ff = Logic process imported form the Freeze Frame module.

-  reset_codes = Logic process imported form the CODES module.

-  reset_mil = Logic process imported fromt the MIL module.

-  reset_trips = Logic process imported from the TRIPS module.

23-13

OBDII STRATEGY, DIAGNOSTIC SCHEDULER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: OBDII_SCHEDULER_COM1

BEGIN: definitions

;This process is not to be specifically executed.  It exists only to make

;the documentation more readable.

unconditionally -----------------------------| initial_state = 0

;set local symbols to improve              | ego_mon_enabled = 1

;readability of documentation              | ego_mon_running = 2

| ego_mon_aborted = 3

| sair_purg_arb = 4

| sair_mon_running = 5

| purg_mon_running = 6

| er_running = 7

| eo_running = 8

| otm_running = 9

| final_state = 10

| true = 1

| false = 0

END: definitions

23-14

OBDII STRATEGY, DIAGNOSTIC SCHEDULER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: parameter_reset_signal_control

(executed each background loop)

OBDII_RESET = true --------------------|

|

KAM_ERROR = true ----------------------|

|     |

ER_STATUS = ER_INIT -------------------|     |

|OR --| OBD_PARM_RST := true

ER_STATUS = ER_DONE--------------------|     |

|     |

EO_STATUS = ECU_EO_INIT----------------|     |

|     |

EO_STATUS = EO_DONE--------------------|     |

| --- ELSE ---

|

| OBD_PARM_RST := false

END: parameter_reset_signal_control

BEGIN: scan_tool_interface

(executed each background loop)

OBDII_RESET = true --------------------|

|AND -| OBDII_RESET = false

RESET_DONE = true ---------------------|     | RESET_DONE = false

|   ;wait 1 background loop

|   ; to clear OBDII_RESET

KAM_ERROR = true ----------------------|

|OR --| OBDII_STATE = initial_state

OBDII_RESET = true --------------------|     | Do: disable_all_monitors

(obdII reset requested)                     |  (reset sequence controller)

| Do: reset_mil

|  (notify mil control module)

| Do: clear_ff

|  (notify freeze frame control

|   module)

| Do: reset_trips

|  (notify trips module)

| Do: reset_codes

|  (notify fault code module)

| Do: reset_drive_cycle

|  (notify drive cycle module)

| Do: reset_fmem

|  (notify system FMEM module)

| RESULTS = false

OBDII_RESET = true --------------------------| RESET_DONE = true

END: scan_tool_interface

23-15

OBDII STRATEGY, DIAGNOSTIC SCHEDULER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: diagnostic_scheduler_control

(executed ONCE per background loop)

PUTMR >= 4 ----------------------------------| Do: check_sys_mon_complete

| Do: system_fmem

| Do: diagnostic_scheduler

|

| --- ELSE ---

|

| Do: disable_all_monitors

| OBDII_STATE = initial_state

END: diagnostic_scheduler_control

BEGIN: disable_all_monitors

(called from other processes)

unconditionally -----------------------------| CCM_TST_ENA = false

| EGR_TST_ENA = false

| FUEL_TST_ENA = false

| MIS_TST_ENA = false

| EGO_TST_ENA = false

| SAIR_TST_ENA = false

| PURG_TST_ENA = false

| CAT_TST_ENA = false

| ER_TST_ENA = false

| EO_TST_ENA = false

| OTM_TST_ENA = false

| FLG_CONT_MON = false

END: disable_all_monitors

23-16

OBDII STRATEGY, DIAGNOSTIC SCHEDULER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: enable_cont_monitors

(called from diagnostic_scheduler process)

CAT_TST_SW = true ---------------------|

|AND -| CAT_TST_ENA = true

CAT_FMEM = false ----------------------|     |

| --- ELSE ---

|

| CAT_TST_ENA = false

CCM_TST_SW = true ---------------------|

|

CCM_TST_RDY = true --------------------|AND -| CCM_TST_ENA = true

|     |

CCM_FMEM = false ----------------------|     |

| --- ELSE ---

|

| CCM_TST_ENA = false

EGR_TST_SW = true ---------------------|

|

EGR_TST_RDY = true --------------------|AND -| EGR_TST_ENA = true

|     |

EGR_FMEM = false ----------------------|     |

| --- ELSE ---

|

| EGR_TST_ENA = false

FUEL_TST_SW = true --------------------|

|AND -| FUEL_TST_ENA = true

FUEL_FMEM = false ---------------------|     |

| --- ELSE ---

|

| FUEL_TST_ENA = false

MIS_TST_SW = true ---------------------|

|AND -| MIS_TST_ENA = true

MIS_FMEM = false ----------------------|     |

| --- ELSE ---

|

| MIS_TST_ENA = false

unconditionally -----------------------------| FLG_CONT_MON = true

END: enable_cont_monitors

23-17

OBDII STRATEGY, DIAGNOSTIC SCHEDULER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: diagnostic_scheduler

;called from diagnostic_scheduler_control process.

EO_TST_RDY = true ---------------------|

|

SCPEO_START = true --------------------|AND -| OBDII_STATE = eo_running

|     | Do: disable_all_monitors

OBDII_STATE <> eo_running -------------|     | EO_TST_ENA = true

| SCPEO_START = false

| Do: inc_warmup_counters

| send(GENERAL_RESPONSE:

|      7Fh,31h,81h,00h,00h,00h)

| RESULTS = false

|

| --- ELSE ---

OBDII_STATE <> eo_running -------------|     |

|AND -| send(GENERAL_RESPONSE:

SCPEO_START = true --------------------|     |      7Fh,31h,81h,00h,00h,22h)

|  ;unable to enter self-test

| SCPEO_START = false

ER_TST_RDY = true ---------------------|

|

SCPER_START = true --------------------|AND -|

|     | OBDII_STATE = er_running

OBDII_STATE <> er_running -------------|     | Do: disable_all_monitors

| ER_TST_ENA = true

| SCPER_START = false

| Do: inc_warmup_counters

| send(GENERAL_RESPONSE:

|      7Fh,31h,82h,00h,00h,00h)

| RESULTS = false

|

| --- ELSE ---

OBDII_STATE <> er_running -------------|     |

|AND -| send(GENERAL_RESPONSE:

SCPER_START = true --------------------|     |       7Fh,31h,82h,00h,00h,22h)

|  ;unable to enter self-test

| SCPER_START = false

23-18

OBDII STRATEGY, DIAGNOSTIC SCHEDULER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OTM_TST_RDY = true --------------------|

|

SCPOTM_START = true -------------------|AND -| OBDII_STATE = otm_running

|     | Do: disable_all_monitors

OBDII_STATE <> otm_running ------------|     | OTM_TST_ENA = true

| SCPOTM_START = false

| Do: inc_warmup_counters

| send(GENERAL_RESPONSE:

|      7Fh,31h,84h,00h,00h,00h)

| RESULTS = false

|

| --- ELSE ---

OBDII_STATE <> otm_running ------------|     |

|AND -| send(GENERAL_RESPONSE:

SCPOTM_START = true -------------------|     |       7Fh,31h,84h,00h,00h,22h)

|  ;unable to enter output

|  ;test mode

| SCPOTM_START = false

23-19

OBDII STRATEGY, DIAGNOSTIC SCHEDULER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OBDII_STATE = er_running --------------|

|AND -| OBDII_STATE = initial_state

ER_MON = true -------------------------|     | ER_TST_ENA = false

;engine running self-test complete.         | ER_MON = false

| RESULTS = true

|

| --- ELSE ---

OBDII_STATE = er_running --------------|     |

|AND -| OBDII_STATE = initial_state

SCPER_EXIT = true ---------------------|     | ER_TST_ENA = false

| SCPER_EXIT = false

| send(GENERAL_RESPONSE:

|      7Fh,32h,82h,00h,00h,64h)

|  ;test aborted, no results.

|

| --- ELSE ---

OBDII_STATE = er_running --------------|     |

|AND -| ER_TST_ENA = false

ER_TST_RDY = false --------------------|     | OBDII_STATE = initial_state

OBDII_STATE = eo_running --------------|

|AND -| OBDII_STATE = initial_state

EO_MON = true -------------------------|     | EO_TST_ENA = false

;engine off self-test complete.             | EO_MON = false

| RESULTS = true

|

| --- ELSE ---

OBDII_STATE = eo_running --------------|     |

|AND -| OBDII_STATE = initial_state

SCPEO_EXIT = true ---------------------|     | EO_TST_ENA = false

| SCPEO_EXIT = false

| send(GENERAL_RESPONSE:

|      7Fh,32h,81h,00h,00h,64h)

|  ;test aborted, no results.

|

| --- ELSE ---

OBDII_STATE = eo_running --------------|     |

|AND -| EO_TST_ENA = false

EO_TST_RDY = false --------------------|     | OBDII_STATE = initial_state

OBDII_STATE = otm_running -------------|

|AND -| OBDII_STATE = initial_state

SCPOTM_EXIT = true --------------------|     | OTM_TST_ENA = false

| SCPOTM_EXIT = false

| send(GENERAL_RESPONSE:

|      7Fh,32h,84h,00h,00h,61h)

| RESULTS = true

|

| --- ELSE ---

OBDII_STATE = otm_running -------------|     |

|AND -| OTM_TST_ENA = false

OTM_TST_RDY = false -------------------|     | OBDII_STATE = initial_state

23-20

OBDII STRATEGY, DIAGNOSTIC SCHEDULER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

ATMR3 >= OBDII_DELAY ------------------|

|

UNDSP = false -------------------------|

|

OBDII_STATE <> eo_running -------------|AND -| Do: enable_cont_monitors

|     |  (RUN mode, not in self-test)

OBDII_STATE <> er_running -------------|     | EO_TST_ENA := false

|     | ER_TST_ENA := false

OBDII_STATE <> otm_running ------------|     | OTM_TST_ENA := false

|

| --- ELSE ---

OBDII_STATE <> eo_running -------------|     |

|     |

OBDII_STATE <> er_running -------------|AND -| Do: disable_all_monitors

|     | OBDII_STATE = initial_state

OBDII_STATE <> otm_running ------------|     | Exit this process.

|  (not RUN mode, not

|   self-test)

|

| --- ELSE ---

|

| Exit this process.

|  (in self-test)

OBDII_STATE = ego_mon_aborted ---|

|

EGO_DEL_TMR = 0 -----------------|AND -|

|     |

EGO_FMEM = false ----------------|     |

|

OBDII_STATE = initial_state -----|     |OR --| OBDII_STATE = ego_mon_enabled

|     |     | EGO_TST_ENA = true

EGO_MON = false -----------------|     |

|     |

EGO_TST_SW = true ---------------|AND -|

|

EGO_FMEM = false ----------------|

|

ATMR3 >= OBDII_DELAY ------------|

OBDII_STATE = ego_mon_enabled ---------|

|AND -| OBDII_STATE = ego_mon_running

EGO_TST_RDY = true --------------------|

OBDII_STATE = ego_mon_running ---|

|

EGO_MON = false -----------------|AND -|

|     |

EGO_TST_RDY = false -------------|     |OR --| OBDII_STATE = ego_mon_aborted

|     | EGO_TST_ENA = false

OBDII_STATE = ego_mon_running ---|     |     | Do: scheduler_delay(ego)

|AND -|

EGO_FMEM = true -----------------|

23-21

OBDII STRATEGY, DIAGNOSTIC SCHEDULER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OBDII_STATE = ego_mon_running ---|

|AND -|

EGO_MON = true ------------------|     |

|

OBDII_STATE = initial_state -----|     |OR --| OBDII_STATE = sair_purg_arb

|     |     | EGO_TST_ENA = false

ATMR3 >= OBDII_DELAY ------------|     |

|     |

EGO_MON = true ------------|     |AND -|

|OR --|

EGO_TST_SW = false --------|

OBDII_STATE = sair_purg_arb -----------|

|

SAIR_TST_SW = true --------------------|

|

SAIR_TST_RDY = true -------------------|

|AND -| OBDII_STATE = sair_mon_running

SAIR_DEL_TMR = 0 ----------------------|     | SAIR_TST_ENA = true

|

SAIR_MON = false ----------------------|

|

SAIR_FMEM = false ---------------------|

OBDII_STATE = sair_mon_running --------|

|

SAIR_TST_RDY = false ------------|     |AND -| OBDII_STATE = sair_purg_arb

|     |     | SAIR_TST_ENA = false

SAIR_MON = true -----------------|OR --|     | Do: scheduler_delay(sair)

|

SAIR_FMEM = true ----------------|

OBDII_STATE = sair_purg_arb -----------|

|

PURG_TST_SW = true --------------------|

|

PURG_TST_RDY = true -------------------|

|AND -| OBDII_STATE = purg_mon_running

PURG_DEL_TMR = 0 ----------------------|     | PURG_TST_ENA = true

|

PURG_MON = false ----------------------|

|

PURG_FMEM = false ---------------------|

OBDII_STATE = purg_mon_running --------|

|

PURG_TST_RDY = false ------------|     |AND -| OBDII_STATE = sair_purg_arb

|     |     | PURG_TST_ENA = false

PURG_MON = true -----------------|OR --|     | Do: scheduler_delay(purg)

|

PURG_FMEM = true ----------------|

23-22

OBDII STRATEGY, DIAGNOSTIC SCHEDULER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OBDII_STATE = sair_purg_arb ------|

|

SAIR_MON = true ------------|     |

|OR --|

SAIR_TST_SW = false --------|     |AND -| OBDII_STATE = final_state

|

PURG_MON = true ------------|     |

|OR --|

PURG_TST_SW = false --------|

END: diagnostic_scheduler

23-23

OBDII STRATEGY, DIAGNOSTIC SCHEDULER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: determine_demand_mode

(public process executed once per background loop to determine the

state of the DEMAND_MODE flag)

OBDII_STATE = er_running --------------|

|

OBDII_STATE = eo_running --------------|OR --| DEMAND_MODE = true

|     |

OBDII_STATE = otm_running -------------|     |

| --- ELSE ---

|

| DEMAND_MODE = false

END: determine_demand_mode

BEGIN: scheduler_delay(sys_monitor)

(executed on demand from within the disagnostic_scheduler process, where

sys_monitor is one of {ego, sair, purg, cat})

sys_monitor = ego ---------------------------| EGO_DEL_TMR = EGO_DEL_TM

|

| --- ELSE ---

|

sys_monitor = sair --------------------------| SAIR_DEL_TMR = SAIR_DEL_TM

|

| --- ELSE ---

|

sys_monitor = purg --------------------------| PURG_DEL_TMR = PURG_DEL_TM

END: scheduler_delay(sys_monitor)

23-24

OBDII STRATEGY, SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

23.1.2  SYSTEM FMEM (CDAN0)

OVERVIEW

+--------------------------------+

| OBDII_SYSTEM_FMEM_COM1         |

+--------------------------------+

| PUBLIC PROCESSES:              |

| . definitions                  |

| . reset_fmem                   |

| . intermittent_initialization  |

| . system_fmem                  |

| PRIVATE PROCESSES:             |

| . fmem_system_configuration    |

| . determine_sys_fmem_flags     |

| . intermittent_check           |

| . clear_intermittent([sensor]) |

| . set_intermittent([sensor])   |

| . update_mon_lst_flags         |

+--------------------------------+

This module is used to define the eight OBD-II System Monitor FMEM flags:

CAT_FMEM

CCM_FMEM

PURG_FMEM

SAIR_FMEM

FUEL_FMEM

EGO_FMEM

EGR_FMEM

MIS_FMEM

The states of these flags are used by  the  OBD-II  Diagnostic  Scheduler  to

determine  if  an  OBD-II  System  Monitor  can be allowed to execute.  If an

individual System Monitor's corresponding sys_FMEM flag  is  set  (=1),  then

something  upon  which  the  Monitor  depends  for  its proper functioning is

faulted, and the Monitor may give false results if  it's  executed.   If  the

corresponding  sys_FMEM flag is cleared (=0), then all information upon which

the System Monitor depends for its  proper  functioning  is  assumed  by  the

OBD-II Diagnostic Scheduler to be valid (or not yet monitored).

The structure of the Diagnostic Scheduler is designed  so  that  all  of  the

information  upon which a System Monitor depends will have had a chance to be

monitored prior to enablement of the Monitor.  This may not be entirely  true

for those System Monitors that run continuously.  In those cases, the Monitor

could be disabled during its execution if an input is found to be invalid.

The following table, sorted by System Monitor, shows the  inputs  upon  which

each  Monitor  depends  and  the  sensors  which  are used to determine these

inputs.

23-25

OBDII STRATEGY, SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

MONITOR         INPUT PARAMETER         SENSORS         FFG's USED

-------         ---------------         -------         ----------

CATALYST        ECT                     ECT             FFG_ECT

CATALYST        AM                      MAF,ACT,ECT,    FFG_MAF,FFG_ACT,

PIP,TP          FFG_ECT,FFG_PIP,

FFG_TP

CATALYST        active EGOs             EGOnm           FFG_EGOnm

CATALYST        APT                     TP              FFG_TP

CATALYST        sair system             EAM             FFG_SAIR

CATALYST        egr  system             EGV,DEPT        FFG_EGR

CATALYST        purg  system            TPR             FFG_PURG

(CVS, VMV)

MONITOR         INPUT PARAMETER         SENSORS         FFG's USED

-------         ---------------         -------         ----------

CCM             NONE                    N/A             NONE

MONITOR         INPUT PARAMETER         SENSORS         FFG's USED

-------         ---------------         -------         ----------

PURGE           ECT                     ECT             FFG_ECT

PURGE           ACT                     ACT             FFG_ACT

PURGE           VSBAR                   VSS             FFG_VS

PURGE           LOAD                    MAF,ACT,ECT,    FFG_MAF,FFG_ACT,

PIP,TP          FFG_ECT,FFG_PIP,

FFG_TP

PURGE           FLG_STALL               PIP,ECT         FFG_PIP,FFG_ECT

23-26

OBDII STRATEGY, SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

MONITOR         INPUT PARAMETER         SENSORS         FFG's USED

-------         ---------------         -------         ----------

SAIR            ECT                     ECT             FFG_ECT

SAIR            ACT                     ACT             FFG_ACT

SAIR            LAMBSE1                 N/A             FFG_LAMBSE1

SAIR            LAMBSE2                 N/A             FFG_LAMBSE2

SAIR            active EGOs             EGOnm           FFG_EGOnm

MONITOR         INPUT PARAMETER         SENSORS         FFG's USED

-------         ---------------         -------         ----------

FUEL            NONE                    N/A             NONE

MONITOR         INPUT PARAMETER         SENSORS         FFG's USED

-------         ---------------         -------         ----------

EGO             ECT                     ECT             FFG_ECT

EGO             VSBAR                   VSS             FFG_VS

EGO             LOAD                    MAF,ACT,ECT,    FFG_MAF,FFG_ACT,

PIP,TP          FFG_ECT,FFG_PIP,

FFG_TP

EGO             N                       PIP             FFG_PIP

EGO             AM                      MAF,ACT,ECT,    FFG_MAF,FFG_ACT,

PIP,TP          FFG_ECT,FFG_PIP,

FFG_TP

EGO             LAMBSE1                 N/A             FFG_LAMBSE1

EGO             LAMBSE2                 N/A             FFG_LAMBSE2

EGO             misfiring engine        PIP,CID         FFG_MISFIRE

EGO             SYNC_CTR                CID             FFG_CID

23-27

OBDII STRATEGY, SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

MONITOR         INPUT PARAMETER         SENSORS         FFG's USED

-------         ---------------         -------         ----------

EGR             ECT                     ECT             FFG_ECT

EGR             N                       PIP             FFG_PIP

EGR             APT                     TP              FFG_TP

EGR             AIR_CHG                 MAF,ACT,ECT,    FFG_MAF,FFG_ACT,

PIP,TP          FFG_ECT,FFG_PIP,

FFG_TP

EGR             INF_MVAC                MAF,ACT,ECT,    FFG_MAF,FFG_ACT,

PIP,TP          FFG_ECT,FFG_PIP,

FFG_TP

EGR             PEXHAUST                MAF,ACT,ECT,    FFG_MAF,FFG_ACT,

PIP,TP          FFG_ECT,FFG_PIP,

FFG_TP

EGR             WRM_IDL_FLG             ECT,PIP,TP      FFG_ECT,FFG_PIP,

FFG_TP

MONITOR         INPUT PARAMETER         SENSORS         FFG's USED

-------         ---------------         -------         ----------

MISFIRE         NONE                    N/A             NONE

NOTE: The Misfire Monitor checks the PIP validity and the CID

validity in the foreground; therefore the OBD-II FMEM-related

background processes will not attempt to control the enabling

and the disabling of the Misfire Monitor based on bad PIP or

bad CID; these are handled internally in the Misfire Monitor.

23-28

OBDII STRATEGY, SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

If the preceding tables are condensed into only the base set  of  systems  or

sensors  required  to  be functional to enable a given OBD-II system monitor,

then the following matrix is obtained.  This matrix is  implemented  in  this

module.

CAT   CCM   PURG   SAIR   FUEL   EGO   EGR   MIS   RESP. MON

+-----------+----+-----+------+------+------+-----+-----+----+-----------+

|FFG_ECT    | X  |     |  X   |  X   |      |  X  |  X  |    |   CCM     |

+-----------+----+-----+------+------+------+-----+-----+----+-----------+

|FFG_ACT    | X  |     |  X   |  X   |      |  X  |  X  |    |   CCM     |

+-----------+----+-----+------+------+------+-----+-----+----+-----------+

|FFG_MAF    | X  |     |  X   |      |      |  X  |  X  |    |   CCM     |

+-----------+----+-----+------+------+------+-----+-----+----+-----------+

|FFG_PIP    | X  |     |  X   |      |      |  X  |  X  |    |   CCM     |

+-----------+----+-----+------+------+------+-----+-----+----+-----------+

|FFG_TP     | X  |     |  X   |      |      |  X  |  X  |    |   CCM     |

+-----------+----+-----+------+------+------+-----+-----+----+-----------+

|FFG_CID    |    |     |      |      |      |  X  |     |    |   CCM     |

+-----------+----+-----+------+------+------+-----+-----+----+-----------+

|FFG_VS     |    |     |  X   |      |      |  X  |     |    |   CCM     |

+-----------+----+-----+------+------+------+-----+-----+----+-----------+

|FFG_EGOnm  | X  |     |      |  X   |      |     |     |    |   EGO     |

+-----------+----+-----+------+------+------+-----+-----+----+-----------+

|FFG_LAMBSEn|    |     |      |  X   |      |  X  |     |    |  FUEL     |

+-----------+----+-----+------+------+------+-----+-----+----+-----------+

|FFG_PURG   | X  |     |      |      |      |     |     |    |  PURGE    |

+-----------+----+-----+------+------+------+-----+-----+----+-----------+

|FFG_SAIR   | X  |     |      |      |      |     |     |    |  SAIR     |

+-----------+----+-----+------+------+------+-----+-----+----+-----------+

|FFG_EGR    | X  |     |      |      |      |     |     |    |   EGR     |

+-----------+----+-----+------+------+------+-----+-----+----+-----------+

|FFG_MISFIRE|    |     |      |      |      |  X  |     |    | MISFIRE   |

+-----------+----+-----+------+------+------+-----+-----+----+-----------+

PLEASE NOTE:  If a particular OBD-II System Monitor is not present, it is the

responsibility  of the OBD-II Diagnostic Executive to ensure that all related

FFGs are continuously cleared.  The  fmem_system_configuration  process  will

take these actions.

This module will also control a set of  non-MIL  fault  codes.   These  fault

codes are used to identify intermittent faults on a set of sensors.

The related strategy identifies when an obdii monitor is aborted  because  an

intermittent  fault  of  the sensor is detected.  After the obdii monitor has

been aborted a calibratable number of times and the monitor has not yet  been

completed  and the IM readiness code is stored, an additional fault code will

be stored identifying the intermittent sensor signal.

There are five such codes, one each for ect,act, tps,  vss,  and  mafs.   The

calibratable number of times the monitor is aborted prior to storing the code

is selected so that the code can be stored after driving for some  reasonable

amount  of  time  at  the  required  steady state conditions.  The diagnostic

procedure will be written to incorporate this time for  verification  of  the

intermittent fault condition as well as the repair effectiveness.

23-29

OBDII STRATEGY, SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  ACT = Air charge temperature, deg F.

-  ACT_FIL_MAX = Highest value the fault filter ACT_FIL sees.

-  AIR_CHG = Air charge value after the filling model.

-  AM = Air mass flow, lb/min.

-  APT = At Part Throttle; -1 -> Closed throttle, 0 -> Part throttle,

1 -> Wide Open throttle.

-  ECT = Engine coolant temperature, deg F.

-  ECT_FIL_MAX = Highest value the fault filter ECT_FIL achieves.

-  INF_MVAC = Inferred Manifold Vacuum.

-  LAMBSE1 = Desired equivalence ratio for Bank1 injectors.

-  LAMBSE2 = Desired equivalence ratio for Bank2 injectors.

-  LOAD = Universal normalized load parameter.

-  MAF = The air flow that is computed at each PIP edge, lbm/tick.

-  MAF_FIL_MAX = Highest value the fault filter MAF_FIL sees.

-  N = Engine RPM.

-  PEXHAUST = Exhaust Back Pressure, inches H2O.

-  SYNC_CTR = Counter which counts cylinders from the PIP signal and  resets

when equal to the number of cylinders in the engine.

-  TP = Instantaneous throttle position, counts.

-  TP_FIL_MAX = Highest value the fault filter TP_FIL sees.

-  VS = Instantaneous vehicle speed (unverified).

-  VSBAR = Filtered vehicle speed for transmission.

-  VS_FIL_MAX = Highest value the fault filter VS_FIL sees.

Bit Flags:

-  ACT_INT = Flag set when intermittent is detected on the CAT sensor.

-  CAT_FMEM = CAT system FMEM flag; 1 -> don't run CAT system due to FMEM.

-  CAT_MON = OBDII monitor flag for catalyst; 1 -> all catalyst faults  have

been monitored since power-up.

23-30

OBDII STRATEGY, SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  CAT_MON_LST = Last background pass value of CAT_MON.

-  CAT_TST_RDY = CAT test ready flag; 1 -> all local  conditions  have  been

met; ready to run CAT test.

-  CCM_FMEM = CCM system FMEM flag; 1 -> don't run CCM due to FMEM.

-  CCM_MON = Obdii monitor flag for comp components.  1=>all CCM faults have

been monitored since power-up.

-  CCM_MON_LST = Last background pass value of CCM_MON.

-  ECT_INT = Flag set when intermittent is detected on ECT sensor.

-  EGO_FMEM = EGO system FMEM flag; 1 -> don't run EGO monitor due to FMEM.

-  EGO_MON = OBDII monitor flag for EGO system; 1=>  all  fuel  faults  been

monitored since power-up.

-  EGO_MON_LST = Last background pass value of EGO_MON.

-  EGO_TST_RDY = EGO test ready flag; 1 -> all local  conditions  have  been

met; ready to run EGO test.

-  EGR_FMEM = EGR system FMEM flag; 1 -> don't run EGR monitor due to FMEM

-  EGR_MON = OBDII monitor flag for EGR system;  1=>  all  EGR  faults  been

monitored since power-up.

-  EGR_MON_LST = Last pass value of EGR_MON.

-  EGR_TST_RDY = EGR test ready flag; 1 -> all local  conditions  have  been

met; ready to run EGR test.

-  FFG_ACT = OBDII  system  FMEM  flag  for  ACT;  1  ->  ACT  is  currently

unreliable.

-  FFG_CID = OBDII system FMEM flag for CID;  1  ->  CID  is  not  currently

reliable.

-  FFG_ECT = OBDII  system  FMEM  flag  for  ECT;  1  ->  ECT  is  currently

unreliable.

-  FFG_EGO11 = OBDII system FMEM flag for EGO11; 1 -> EGO11 is not currently

switching reliably.

-  FFG_EGO12 = OBDII system FMEM flag for EGO12; 1 -> EGO12 is not currently

switching reliably.

-  FFG_EGO21 = OBDII system FMEM flag for EGO21; 1 -> EGO21 is not currently

switching reliably.

-  FFG_EGO22 = OBDII system FMEM flag for EGO22; 1 -> EGO22 is not currently

switching reliably.

-  FFG_EGR = OBDII system FMEM flag for the EGR system; 1 -> EGR system  not

currently operating reliably.

23-31

OBDII STRATEGY, SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FFG_LAMBSE1 = OBDII system FMEM flag for LAMBSE1; 1  ->  LAMBSE1  is  not

currently reliable.

-  FFG_LAMBSE2 = OBDII system FMEM flag for LAMBSE2; 1  ->  LAMBSE2  is  not

currently reliable.

-  FFG_MAF = OBDII system  FMEM  flag  for  MAF;  1  ->  MAF  not  currently

reliable.

-  FFG_MISFIRE = OBDII system FMEM flag for engine misfire; 1  ->  currently

misfiring.

-  FFG_PIP = OBDII system  FMEM  flag  for  PIP;  1  ->  PIP  not  currently

reliable.

-  FFG_PURG = OBDII system FMEM flag for the purge system; 1 -> purge system

is not under control reliably.

-  FFG_SAIR = OBDII system FMEM flag for the sair system; 1 -> sair sytem is

not currently under control reliably.

-  FFG_TP = OBDII system FMEM  flag  for  TP;  1  ->  TP  is  not  currently

reliable.

-  FFG_VS = OBDII system FMEM flag for VS; 1 -> VS is currently unreliable.

-  FLG_CONT_MON = Flag set when  continuous  monitors  are  enabled  in  the

OBD-II system.

-  FUEL_FMEM = FUEL system FMEM flag; 1 -> don't run  FUEL  monitor  due  to

system FMEM.

-  FUEL_MON = OBDII monitor flag for fuel system; 1 -> all fuel faults  been

monitored since power-up.

-  FUEL_MON_LST = Last background pass value of FUEL_MON.

-  KAM_ERROR = Indicates keep alive RAM invalid.

-  MAF_INT = Flag set when intermittent is detected on the MAF sensor.

-  MIS_FMEM = MIS system FMEM flag; 1 -> don't run MIS monitor due to FMEM.

-  MIS_MON = OBDII misfire test monior complete for trip indication.

-  MIS_MON_LST = Last background pass value of MIS_MON.

-  PURG_FMEM = PURG system FMEM flag; 1 -> don't run  PURG  monitor  due  to

FMEM.

-  OBDII_STATE = Current state of the obdii sequence controller.

-  PURG_MON = OBDII monitor flag for evaporative purge; 1=> all purge faults

been monitored since power-up.

-  PURG_MON_LST = Last background pass value of PURGE_MON.

23-32

OBDII STRATEGY, SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PURG_TST_RDY = Purge test ready flag; 1 -> all local conditions have been

met ;ready to run purge test.

-  PxxxFAULT = Fault flag for code Pxxx; 1 -> a fault has  been  stored  for

code Pxxx.

-  SAIR_FMEM = SAIR system FMEM flag; 1 -> don't run  SAIR  monitor  due  to

FMEM.

-  SAIR_MON = OBDII monitor flag for SAIR system; 1=> all SAIR  faults  been

monitored since power-up.

-  SAIR_MON_LST = Last background pass value of SAIR_MON.

-  TP_INT = Fag set when intermittent is detected on the TP sensor.

-  VS_INT = Flag set when intermittent is detected on the VS sensor.

-  WRM_IDL_FLG = Warm Idle Flag.  Used for EGR.

Calibration Constants:

-  ACT_FIL = Intermittent fault filter for the ACT sensor.

-  ACT_LVL = Fault level for intermittent ACT sensor.

-  ECT_FIL = Intermittent fault filter for the ECT sensor.

-  ECT_LVL = Fault level for intermittent ECT sensor.

-  EGR_TST_SW = EGR test present calibration switch; 1 -> EGR test present.

-  MAF_FIL = Intermittent fault filter for the MAF sensor.

-  MAF_LVL = Fault level for intermittent MAF sensor.

-  MIS_TST_SW = MIS test present calibration switch; 1 -> MIS test present.

-  PURG_TST_SW = PURG test  present  calibration  switch;  1  ->  PURG  test

present.

-  SAIR_TST_SW = SAIR test  present  calibration  switch;  1  ->  SAIR  test

present.

-  TP_FIL = Intermittent fault filter for the TP sensor.

-  TP_LVL = Fault level for intermittent TP sensor.

-  VS_FIL = Intermittent fault filter for the VS sensor.

-  VS_LVL = Fault level for intermittent VS sensor.

23-33

OBDII STRATEGY, SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OTHER

-  P1000 = Additional mixed city and highway driving  required  to  complete

OBD-II system check-out.

-  P1100 = Fault code for intermittent MAF sensor.

-  P1112 = Fault code for intermittent ACT sensor.

-  P1117 = Fault code for intermittent ECT sensor.

-  P1125 = Fault code for intermittent TP sensor.

-  P1500 = Fault code for intermittent VS sensor.

23-34

OBDII STRATEGY, SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: OBDII_SYSTEM_FMEM_COM1

BEGIN: definitions

;process not to be explicitly executed, but to define locally

;used symbols.

unconditionally ------------------------| true := 1

| false := 0

| ego_mon_running := 2

| sair_mon_running := 5

| purg_mon_running := 6

END: definitions

BEGIN: system_fmem

;This process is used as the controlling process for the system fmem

strategy

;contained in this module.  It is called from within the diagnostic

;scheduler.

unconditionally ------------------------| Do: intermittent_check

| Do: update_mon_lst_flags

| Do: fmem_system_configuration

| Do: determine_sys_fmem_flags

END: system_fmem

BEGIN: reset_fmem

;PUBLIC process used to reset the FMEM flags upon OBD-II RESET.  This

;process is called from the scan_tool_interface.

unconditionally ------------------------| CAT_FMEM := false

| CCM_FMEM := false

| PURG_FMEM := false

| SAIR_FMEM := false

| FUEL_FMEM := false

| EGO_FMEM := false

| EGR_FMEM := false

| MIS_FMEM := false

| Do: intermittent_initialization

KAM_ERROR = true -----------------------| MAF_FIL_MAX := 0

| ACT_FIL_MAX := 0

| ECT_FIL_MAX := 0

| TP_FIL_MAX := 0

| VS_FIL_MAX := 0

END: reset_fmem

23-35

OBDII STRATEGY, SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: fmem_system_configuration

;PRIVATE process executed on demand from system_fmem.  This process will

;ensure that those FFGs related to an OBD-II system monitor that is not

;present are always false.  This is a bullet-proofing action.  An

;assumption is being made that all packages will have component monitoring

;to some degree on ECT, ACT, MAF, PIP, TP, CID, VSS, EGOs, AND the adaptive

;fuel system.  Therefore, those FFGs will always be under control in the

;external strategy features and are not need in this process.

MIS_TST_SW = false ---------------------| FFG_MISFIRE := false

SAIR_TST_SW = false --------------------| FFG_SAIR := false

PURG_TST_SW = false --------------------| FFG_PURG := false

EGR_TST_SW = false ---------------------| FFG_EGR := false

END: fmem_system_configuration

23-36

OBDII STRATEGY, SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: determine_sys_fmem_flags

;PRIVATE process used to calculate the flag sys_FMEM flags for use

;in the diagnostic scheduler.

FFG_ECT = true -------------------|

|

FFG_ACT = true -------------------|

|

FFG_MAF = true -------------------|

|

FFG_PIP = true -------------------|

|

FFG_PURG = true ------------------|

|

FFG_TP = true --------------------|

|OR --| CAT_FMEM := true

FFG_EGO11 = true -----------------|     |

|     |

FFG_EGO21 = true -----------------|     |

|     |

FFG_EGO12 = true -----------------|     |

|     |

FFG_EGO22 = true -----------------|     |

|     |

FFG_SAIR = true ------------------|     |

| --- ELSE ---

|

| CAT_FMEM := false

23-37

OBDII STRATEGY, SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FFG_ECT = true -------------------|

|

FFG_ACT = true -------------------|

|

FFG_MAF = true -------------------|

|OR --| PURG_FMEM := true

FFG_PIP = true -------------------|     |

|     |

FFG_TP = true --------------------|     |

|     |

FFG_VS = true --------------------|     |

| --- ELSE ---

|

| PURG_FMEM := false

FFG_ECT = true -------------------|

|

FFG_ACT = true -------------------|

|

FFG_EGO11 = true -----------------|

|

FFG_EGO21 = true -----------------|

|OR --| SAIR_FMEM := true

FFG_EGO12 = true -----------------|     |

|     |

FFG_EGO22 = true -----------------|     |

|     |

FFG_LAMBSE1 = true ---------------|     |

|     |

FFG_LAMBSE2 = true ---------------|     |

| --- ELSE ---

|

| SAIR_FMEM := false

23-38

OBDII STRATEGY, SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FFG_ECT = true -------------------|

|

FFG_ACT = true -------------------|

|

FFG_MAF = true -------------------|

|

FFG_PIP = true -------------------|

|

FFG_TP = true --------------------|

|OR --| EGO_FMEM := true

FFG_CID = true -------------------|     |

|     |

FFG_VS = true --------------------|     |

|     |

FFG_LAMBSE1 = true ---------------|     |

|     |

FFG_LAMBSE2 = true ---------------|     |

|     |

FFG_MISFIRE = true ---------------|     |

| --- ELSE ---

|

| EGO_FMEM := false

FFG_ECT = true -------------------|

|

FFG_ACT = true -------------------|

|

FFG_MAF = true -------------------|OR --| EGR_FMEM := true

|     |

FFG_PIP = true -------------------|     |

|     |

FFG_TP = true --------------------|     |

| --- ELSE ---

|

| EGR_FMEM := false

unconditionally ------------------------| CCM_FMEM := false

unconditionally ------------------------| FUEL_FMEM := false

unconditionally ------------------------| MIS_FMEM := false

END: determine_sys_fmem_flags

23-39

OBDII STRATEGY, SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: intermittent_initialization

;execute during RAM init or OBDII reset conditions.

For [sensor] := {ECT, ACT, TP, MAF, VS} perform the following:

unconditionally -------------------------| [sensor]_FIL := 0

| [sensor]_INT := 0

END: intermittent_initialization

23-40

OBDII STRATEGY, SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: intermittent_check

;PRIVATE process called from the system_fmem process to determine if

;intermittent faults are preventing execution of an OBD-II monitor.

For [sensor] := {ECT, ACT, TP, MAF, VS} perform the following:

[sensor]_INT = 0 --------------------------|

;this FFG_ event has not been             |

;counted yet.                             |

|

P1000FAULT = 1 ----------------------------|

;the IM readiness code is stored, i.e.,   |

;this vehicle cannot get plates.          |

|

FFG_[sensor] = 1 --------------------------|

;the fault is detected the b.g. pass.     |

|

EGR_TST_RDY = 1 ---|                       |AND -| [sensor]_INT := 1

|AND -|                 |     | [sensor]_FIL :=

EGR_MON = 0 -------|     |                 |     |    [sensor]_FIL + 1

|OR --|           |     |

CAT_TST_RDY = 1 ---|     |     |           |     |

|AND -|     |           |     |

CAT_MON = 0 -------|           |AND -|     |     |

|     |     |     |

FLG_CONT_MON = 1 --------------|     |     |     |

|     |     |     |

[sensor] <> VS ----------------|     |     |     |

|     |     |

EGO_TST_RDY = 1 ---------------|     |     |     |

|AND -|     |     |

OBDII_STATE = ego_mon_running -|     |     |     |

;the scheduled EGO monitor          |     |     |

;should be running, but the         |OR --|     |

;sensor fault is preventing it      |           |

|           |

PURG_TST_RDY = 1 --------------|     |           |

|AND -|           |

OBDII_STATE =                  |     |           |

purg_mon_running -----|     |           |

|           |

SAIR_TST_RDY = 1 --------------|     |           |

|     |           |

OBDII_STATE =                  |     |           |

sair_mon_running ------|     |           |

|     |           |

[sensor] <> TP ----------------|AND -|           |

|                 |

[sensor] <> MAF ---------------|                 |

|                 |

[sensor] <> VS ----------------|                 |

| --- ELSE ---

|

FFG_[sensor] = 0 --------------------------------| [sensor]_INT := 0

23-41

OBDII STRATEGY, SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

P1000FAULT = 0 --------------------------|

;the IM readiness code is no longer     |

;stored, this vehicle can get plates.   |

|

SAIR_MON_LST = 0 ------------------|     |

|AND -|

SAIR_MON = 1 ----------------------|     |

|

PURG_MON_LST = 0 ------------------|     |

|AND -|

PURG_MON = 1 ----------------------|     |

|

EGO_MON_LST = 0 -------------------|     |

|AND -|

EGO_MON = 1 -----------------------|     |

;after each monitor in the schedule     |OR --| clear_intermittent([sensor])

;completes, reset the fault filter      |     |

;to prevent storing the code when       |     |

;no real problems exist.                |     |

|     |

SAIR_MON = 1 ----------------|           |     |

|           |     |

[sensor] = VS ---------------|           |     |

|OR --|     |     |

[sensor] = MAF --------------|     |     |     |

|     |     |     |

[sensor] = TP ---------------|     |     |     |

|     |     |

CAT_MON = 1 -----------------|     |     |     |

|OR --|AND -|     |

[sensor] = VS ---------------|     |           |

|           |

EGR_MON = 1 -----------------|     |           |

|OR --|           |

[sensor] = VS ---------------|     |           |

|           |

EGO_MON = 1 -----------------------|           |

|           |

PURG_MON = 1 ----------------------|           |

;all systems that depend on this              |

;sensor input have run to completion.         | --- ELSE ---

|

[sensor]_FIL > [sensor]_LVL -------------------| set_intermittent([sensor])

;enough fault events which stop a monitor

;from running have been encountered, store

;a fault code indicating this.

23-42

OBDII STRATEGY, SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

[sensor]_FIL > [sensor]_FIL_MAX ----------| [sensor]_FIL_MAX := [sensor]_FIL

|  ;ratchet up to the highest value

|  ;encountered.  Used by diag.

END: intermittent_check

BEGIN: clear_intermittent([sensor])

;PRIVATE process called from intermittent_check to erase the fault code.

unconditionally ---------------------------------| [sensor]_FIL := 0

| [sensor]_INT := 0

[sensor] = ECT ----------------------------------| clear_malf(P1117)

|

| --- ELSE ---

|

[sensor] = ACT ----------------------------------| clear_malf(P1112)

|

| --- ELSE ---

|

[sensor] = TP -----------------------------------| clear_malf(P1125)

|

| --- ELSE ---

|

[sensor] = MAF ----------------------------------| clear_malf(P1100)

|

| --- ELSE ---

|

[sensor] = VS -----------------------------------| clear_malf(P1500)

END: clear_intermittent([sensor])

23-43

OBDII STRATEGY, SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: set_intermittent([sensor])

;PRIVATE process called from intermittent_check() to store the fault code.

[sensor] = ECT ----------------------------------| store_code(P1117)

|

| --- ELSE ---

|

[sensor] = ACT ----------------------------------| store_code(P1112)

|

| --- ELSE ---

|

[sensor] = TP -----------------------------------| store_code(P1125)

|

| --- ELSE ---

|

[sensor] = MAF ----------------------------------| store_code(P1100)

|

| --- ELSE ---

|

[sensor] = VS -----------------------------------| store_code(P1500)

END: set_intermittent([sensor])

BEGIN: update_mon_lst_flags

;PRIVATE process used to update the sys_MON_LST flags used by the

;intermittent_check process.

unconditionally ---------------------------------| CAT_MON_LST := CAT_MON

| EGO_MON_LST := EGO_MON

| SAIR_MON_LST := SAIR_MON

| PURG_MON_LST := PURG_MON

| CCM_MON_LST := CCM_MON

| EGR_MON_LST := EGR_MON

| MIS_MON_LST := MIS_MON

| FUEL_MON_LST := FUEL_MON

END: update_mon_lst_flags

23-44

OBDII STRATEGY, MIL CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

23.1.3  MIL CONTROLLER (CDAM0)

OVERVIEW

This logic is triggered by any of the eight  OBDII  systems  at  the  time  a

malfunction  is detected, a trip is completed, power-up is encountered, or an

OBD-II scan tool reset occurs.  The particular  system  and  fault  code  are

indicated by sys and Pxxx respectively.

This module acts as a dispatcher to the rest of  the  MIL  control  strategy.

The  rest  of  the  MIL control logic is divided into logical objects.  Those

objects represent MIL control finite state machine  and  associated  data  as

well  as  MIL  output control and data, freeze frame control and data, OBD-II

trip monitoring and data, and fault code control and data.

The MIL control finite state machines are  event  driven,  those  events  are

malfunction,  trip_complete,  reset_mil,  and  initialize_mil.   This  module

processes incoming requests from the outside environment and makes  calls  to

the appropriate objects where the procedures and data are maintained.

In pictorial form the MIL controller  interacts  with  the  sys_STATE_CONTROL

modules as shown below:

23-45

OBDII STRATEGY, MIL CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

MIL Controller

+---------------------------+

| OBDII_MIL_CONTROLLER      |

+---------------------------+

|.malfunction               |

|.init_mil                  |

|.check_sys_mon_complete    |

|.reset_mil                 |

+-------------+-------------+

|

+--------------------------+           |

|OBDII_TYPE1_STATE_CONTROL |           |

+--------------------------+           |

| . type1_malfunction      |<----------|

| . type1_init_mil         |<----------|

| . type1_trip_complete    |<----------|

| . type1_reset            |<----------|

+--------------------------+           |

|

+--------------------------+           |

|OBDII_MIS_STATE_CONTROL   |           |

+--------------------------+           |

| . mis_malfunction        |<----------|

| . mis_init_mil           |<----------|

| . mis_trip_complete      |<----------|

| . mis_reset              |<----------|

| . mis_conds_seen(Pxxx)   |           |

+--------------------------+           |

|

+--------------------------+           |

|OBDII_FUEL_STATE_CONTROL  |           |

+--------------------------+           |

| . fuel_malfunction       |<----------|

| . fuel_init_mil          |<----------|

| . fuel_trip_complete     |<----------|

| . fuel_reset             |<----------|

| . fuel_conds_seen(Pxxx)  |           |

+--------------------------+           |

|

+--------------------------+           |

|OBDII_CL_STATE_CONTROL    |           |

+--------------------------+           |

| . cl_malfunction         |<----------|

| . cl_init_mil            |<----------|

| . cl_trip_complete       |<----------|

| . cl_reset               |<----------+

+--------------------------+

23-46

OBDII STRATEGY, MIL CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  FF_CODE = Code number of fault for  which  freeze  frame  conditions  are

stored.  freeze frame is loaded.

-  MIS_FF_CODE = Integer representation of  the  first  misfire  malfunction

code number after an unknown misfire.

-  UNK_MIS_CODE = Integer representation of the unknown  cylinder  misfiring

fault code number.

Bit Flags:

-  CAT_MIL_ON = MIL status flag for Catalyst system; 1 -> MIL  required  for

catalyst fault.

-  CAT_MON = OBDII monitor flag for catalyst; 1=>all  catalyst  faults  have

been monitored since power-up.

-  CAT_MON_LST = Last background pass value of CAT_MON.

-  CCM_MON = Obdii monitor flag for comp components.  1=>all CCM faults have

been monitored since power-up.

-  CCM_MON_LST = Last background pass value of CCM_MON.

-  COOL_MIL_ON = MIL status flag for fail safe  cooling  system;  1  ->  MIL

required for fail safe cooling

-  CAT_MON = OBDII monitor flag for catalyst; 1 -> all catalyst faults  have

been monitored since power-up.

-  CAT_MON_LST = Last background pass value of CAT_MON.

-  CCM_MIL_ON = MIL status flag  for  Comprehensive  Components;  1  ->  MIL

required for CCM fault.

-  CL_MIL_ON = MIL status flag for Closed Loop Entry; 1 -> MIL required  for

Closed Loop Entry fault).

-  DEMAND_MODE = Flag set when an On-Demand self test mode is active.

-  EGO_MIL_ON = MIL status flag for EGO; 1 -> MIL required for HEGO fault.

-  EGR_MIL_ON = MIL status flag for EGR; 1 -> MIL required for EGR fault.

-  EGR_MON = Obdii monitor flag for EGR system;  1=>  all  EGR  faults  been

monitored since power-up.

-  EGR_MON_LST = Last pass value of EGR_MON.

-  FF_FUEL_MIS = Freeze frame FUEL or MISFIRE data stored flag; 1 ->  freeze

frame was stored due to FUEL or MISFIRE.

-  FUEL_MIL_ON = MIL status flag for Fuel System; 1 -> MIL required for Fuel

System fault.

23-47

OBDII STRATEGY, MIL CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FUEL_MON = OBDII monitor flag for fuel system; 1 -> all fuel faults  been

monitored since power-up.

-  FUEL_MON_LST = Last background pass value of FUEL_MON.

-  MIS_MIL_ON = MIL status flag for Misfire; 1 -> MIL required  for  Misfire

fault.

-  MIS_MON = OBDII misfire test monior complete for trip indication.

-  MIS_MON_LST = Last background pass value of MIS_MON.

-  PURG_MIL_ON = MIL status flag Purge; 1 -> MIL required for Purge fault.

-  PURG_MON = OBDII monitor flag for  evaporative  purge;  1  ->  all  purge

faults been monitored since power-up.

-  SAIR_MIL_ON = MIL status flag for Secondary Air; 1 ->  MIL  required  for

Secondary Air fault.

-  SAIR_MON = Secomdary air has been monitored for OBD-II trip.

-  SAIR_MON_LST = Last background pass value of SAIR_MON.

Calibration Constants:

-  CAT_TST_SW = Cat test present calibration switch; 1 -> CAT present.

-  EGO_TST_SW = Ego test present calibration switch; 1 -> EGO test present.

-  EGR_TST_SW = Egr test present calibration switch; 1 -> EGR test present.

-  FUEL_TST_SW = Fuel test  present  claibration  switch;  1  ->  FUEL  test

present.

-  MIS_TST_SW = Mis test present claibration switch; 1 -> MIS test present.

-  PURG_TST_SW = Purg test  present  calibration  switch;  1  ->  PURG  test

present.

-  SAIR_TST_SW = Sair test  present  calibration  switch;  1  ->  SAIR  test

present.

23-48

OBDII STRATEGY, MIL CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: OBDII_MIL_CONTROLLER_COM1

BEGIN: malfunction(sys,Pxxx)

;handle freeze frame

sys = mis -----------------|

|

MIS_FF_CODE = 0 -----------|AND -| MIS_FF_CODE = xxx

|     |  ;capture the first code number after an

xxx <> UNK_MIS_CODE -------|     |  ;unknown misfire event. This will be used

|  ;later when misfire can detect which

|  ;cylinder is misfiring.

DEMAND_MODE = 0 -----------|

|

FF_FUEL_MIS = 0 -----------|AND -| Do: store_ff(Pxxx)

|     | FF_FUEL_MIS := 1

sys = fuel ----------|     |     |

|OR --|     |

sys = mis -----------|           |

| --- ELSE ---

DEMAND_MODE = 0 -----------|     |

|AND -| Do: store_ff(Pxxx)

FF_CODE = 0 ---------------|

;freeze frame is empty.

23-49

OBDII STRATEGY, MIL CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

sys = fuel ----------------------| Do: fuel_malfunction(FUEL_MIL_ON, Pxxx)

|

| --- ELSE ---

|

sys = mis -----------------------| Do: mis_malfunction(MIS_MIL_ON, Pxxx)

|

| --- ELSE ---

|

sys = cat -----------------------| Do: type1_malfunction(CAT_MIL_ON, Pxxx,

|     cat)

| --- ELSE ---

|

sys = ccm -----------------------| Do: type1_malfunction(CCM_MIL_ON, Pxxx,

|     ccm)

| --- ELSE ---

|

sys = cool ----------------------| Do: type1_malfunction(COOL_MIL_ON, Pxxx,

|     cool)

|

| --- ELSE ---

|

sys = ego -----------------------| Do: type1_malfunction(EGO_MIL_ON, Pxxx,

|     ego)

| --- ELSE ---

|

sys = egr -----------------------| Do: type1_malfunction(EGR_MIL_ON, Pxxx,

|     egr)

| --- ELSE ---

|

sys = purg ----------------------| Do: type1_malfunction(PURG_MIL_ON, Pxxx,

|     purg)

|

| --- ELSE ---

|

sys = sair ----------------------| Do: type1_malfunction(SAIR_MIL_ON, Pxxx,

|     sair)

|

| --- ELSE ---

|

sys = cl ------------------------| Do: cl_malfunction(CL_MIL_ON, Pxxx)

unconditionally -----------------| PxxxMALF := 1

END: malfunction(sys,Pxxx)

23-50

OBDII STRATEGY, MIL CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: initialize_mil

;Public procedure performed on power up initialization.

unconditionally -----------------| Do: fuel_init_mil(FUEL_MIL_ON)

| Do: mis_init_mil(MIS_MIL_ON)

| Do: type1_init_mil(cat)

| Do: type1_init_mil(ccm)

| Do: type1_init_mil(cool)

| Do: type1_init_mil(ego)

| Do: type1_init_mil(egr)

| Do: type1_init_mil(purg)

| Do: type1_init_mil(sair)

| Do: cl_init_mil(CL_MIL_ON)

| Do: cl_seen_init

END: initialize_mil

BEGIN: check_sys_mon_complete

;PRIVATE procedure used to update state machines, used by diagnostic

; scheduler control

FUEL_MON <> FUEL_MON_LST ---------|

|OR --| Do: fuel_trip_complete

CCM_MON <> CCM_MON_LST -----|     |

|AND -|

FUEL_TST_SW = 0 ------------|

MIS_MON <> MIS_MON_LST -----------|

|OR --| Do: mis_trip_complete

CCM_MON <> CCM_MON_LST -----|     |

|AND -|

MIS_TST_SW = 0 -------------|

CAT_MON <> CAT_MON_LST -----------|

|OR --| Do:

CCM_MON <> CCM_MON_LST -----|     |     | type1_trip_complete(cat,CAT_MIL_ON)

|AND -|

CAT_TST_SW = 0 -------------|

EGR_MON <> EGR_MON_LST -----------|

|OR --| Do:

CCM_MON <> CCM_MON_LST -----|     |     | type1_trip_complete(egr,EGR_MIL_ON)

|AND -|

EGR_TST_SW = 0 -------------|

CCM_MON <> CCM_MON_LST -----------------| Do:

| type1_trip_complete(ccm,CCM_MIL_ON)

| Do: type1_trip_complete(cool,

|       COOL_MIL_ON)

23-51

OBDII STRATEGY, MIL CONTROLLER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EGO_MON <> EGO_MON_LST -----------|

|OR --| Do:

CCM_MON <> CCM_MON_LST -----|     |     | type1_trip_complete(ego,EGO_MIL_ON)

|AND -|     | Do: cl_trip_complete(CL_MIL_ON)

EGO_TST_SW = 0 -------------|

SAIR_MON <> SAIR_MON_LST ---------|

|OR --| Do: type1_trip_complete(sair,

CCM_MON <> CCM_MON_LST -----|     |     |     SAIR_MIL_ON)

|AND -|

SAIR_TST_SW = 0 ------------|

PURG_MON <> PURG_MON_LST ---------|

|OR --| Do: type1_trip_complete(purg,

CCM_MON <> CCM_MON_LST -----|     |     |     PURG_MIL_ON)

|AND -|

PURG_TST_SW = 0 ------------|

END: check_sys_mon_complete

BEGIN: reset_mil

;Public process executed when an obdII reset message is received from

;the scan tool.

unconditionally -----------------| MIL_ON := 0

| Do: fuel_reset

| Do: mis_reset

| Do: type1_reset

| Do: cl_reset

END: reset_mil

23-52

OBDII STRATEGY, FAULT CODES - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

23.1.4  FAULT CODES (CDAM0)

OVERVIEW

+--------------------------------+

| OBDII_CODES_COM1               |

+--------------------------------+

| PUBLIC PROCESSES:              |

| . check_warmup_counters        |

| . store_code(Pxxx)             |

| . erase_code(Pxxx)             |

| . erase_mis_code(Pxxx)         |

| . reset_codes                  |

| . init_codes                   |

| . inc_warmup_counters          |

| . clear_malf(Pxxx)             |

+--------------------------------+

| PRIVATE PROCESSES:             |

| . update_warmup_counters       |

+--------------------------------+

This module handles all the actions having to do  with  fault  codes.   These

actions  include  the  storing  and erasing of codes, as well as reporting of

code status.  Each fault code has associated with it, a warmup counter, which

is  incremented  each  time  a warmup cycle occurs.  A code is erased when at

least 80 warmups have occurred since it was stored.

A specific fault code is referred to by the acronym, Pxxx.   Where  'xxx'  is

the  fault  code number, eg) fault code 131 is referred to as P0131.  Special

sets of fault codes are referred to by labels, such as egr_codes, purg_codes,

etc.   The  label,  egr_codes,  refers to the set of fault codes having to do

with EGR, while purg_codes refers to the set of fault codes having to do with

the purge test.

The complete set of all fault codes is referred to as all_codes.  The set  of

fault  codes  which  affect  the  MIL  is  referred to as mil_codes.  The set

mil_codes is a subset of all_codes.

Each fault code which affects the MIL has the following block  of  parameters

associated with it.

All fault codes  controlling the MIL light will be designated as MIL_CODE

and have the  following record structure:

23-53

OBDII STRATEGY, FAULT CODES - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

MIL_CODE = RECORD

PXXXMALF    = [RAM] Malfunction flag for code Pxxx;

1 -> A malfunction currently exists

for fault Pxxx.

PXXXFAULT   = [KAM] Fault flag for code Pxxx;

1 -> A fault has been stored for

code Pxxx.

PXXXFAULT_A = [KAM] Duplicate fault flag for code Pxxx;

1 -> A fault has been stored for

code Pxxx.

PXXXCNT     = [KAM] Warm-up counter for code Pxxx.

PXXXSTATE   = [KAM] MIL state flag for code Pxxx.

PXXXMIL_ON  = [KAM] MIL flag for code Pxxx;

1 -> The MIL is required to be

on for code Pxxx.

PXXXUPDATED = [RAM] MIL state controller update

status flag for code Pxxx;

1 -> controller has been updated

since pwr-up.

END.

All fault codes NOT controlling the MIL light will be designated as

NO_MIL_CODE and have the following record structure:

NO_MIL_CODE = RECORD

PXXXMALF = See above.

PXXXFAULT = See above.

PXXXFAULT_A = See above.

PXXXCNT = See above.

END.

This module provides certain services, for manipulating  fault  codes,  which

are exported for use by other modules.  These include:

-  check_warmup_counters:   This  is  a  background  task  that  checks  and

increments each of the counters if a warm up has occured.

-  erase_code(Pxxx):  Erases the fault code specified by Pxxx.

-  initialize_codes:  This task is executed on power-up  initialization  and

clears the PxxxMALF and PxxxUPDATED flags.

-  reset_codes:   This  service  performs  fault  code  related  actions  in

response  to  a  system  reset message received from the obdII scan tool.

Each code is erased, and each warmup counter is set to 80 warmups.

-  store_code(Pxxx):  Stores the fault code  specified  by  Pxxx.   For  mil

codes,  this  service  is used exclusively by the MIL Control module.  It

may also be used by other diagnostic tests (for codes which do not affect

the mil).

-  inc_warmup_counters:  This service is exercised to increment all  warm-up

counters  by  one.   One  place  where  this is used is when an on demand

diagnostic mode is entered.  During the  on  demand  mode  PxxxFAULT  and

PxxxFAULT_A  bits  will  NOT  be  set.   When  a fault is identified, the

warm-up counter will only be cleared.  At the end of the on  demand  mode

diagnostic  procedure, all fault code records with their warm-up counters

cleared will be reported.

23-54

OBDII STRATEGY, FAULT CODES - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  CODES_COUNT = Count of total number of fault codes and pending codes (MIL

and NO_MIL) stored by the EEC.

-  ECT = Engine coolant temperature, deg F.

-  FF_CODE = Code number of fault for  which  freeze  frame  conditions  are

stored, freeze frame is loaded.

-  TCSTRT = Temperature of ECT at Cold Start, deg F.

-  PxxxCNT = Warm-up counter for code Pxxx.

Bit Flags:

-  CODES_CHECKD = Flag indicating the warm-up counters for OBD-II codes have

been checked.

-  CRKFLG = Engine Mode Flag; 1 -> Crank Mode, 0 -> not in Crank Mode.

-  DEMAND_MODE = Flag set when an On-Demand self test mode is active.

-  PxxxFAULT = Fault flag for code Pxxx; 1 -> a fault has  been  stored  for

code Pxxx.

-  PxxxFAULT_A = Duplicate fault flag for code Pxxx; 1 -> a fault  has  been

stored for code Pxxx.

-  PXXXMIL_ON = MIL flag for code Pxxx; 1 -> The MIL is required  to  be  on

for code Pxxx.

-  UNDSP = Run/Underspeed Flag; 1 -> Underspeed (or CRANK), 0 -> Run.

-  OBDII_WARMUP = Indicates engine warmup occured.

OUTPUTS

Registers:

-  CODES_COUNT = See above.

-  NUM_CODES = Number of OBD-II fault codes currently stored in memory.

-  OD_CODE_CNT = Count of total number of on-demand codes currently stored.

-  PxxxCNT = Warm-up counter for code Pxxx.

Bit Flags:

-  CODES_CHECKD = See above.

23-55

OBDII STRATEGY, FAULT CODES - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PxxxFAULT = See above.

-  PxxxFAULT_A = See above.

-  PxxxMALF = Malfunction flag for code Pxxx; 1 -> a  malfunction  currently

exists for fault Pxxx.

-  PxxxUPDATED = MIL state controller update status flag for code Pxxx; 1 ->

controller has been updated since PWR-UP.

-  OBDII_WARMUP = See above.

OTHER

-  all_codes = The set of all fault codes.

-  cat_codes = The set of fault codes for the obdII catalyst monitor test.

-  ccm_codes = The set of fault codes for the obdII comprehensive  component

monitor test.

-  cool_codes = The set of fault codes for the fail safe cooling system

-  ego_codes = The set of fault codes for the obdII EGO monitor test.

-  egr_codes = The set of fault codes for the obdII EGR monitor test.

-  fuel_codes = The set of fault codes for the obdII fuel system test.

-  mil_codes = set of {cat_codes,ccm_codes,purg_codes,sair_codes,fuel_codes,

ego_codes,egr_codes,mis_codes}.

-  mis_codes = The set of fault codes for the obdII misfire monitor test.

-  no_mil_codes = The subset of all_codes that are not mil_codes.

-  purg_codes = The set of fault codes for the obdII purge monitor test.

-  sair_codes = The set of fault codes for the obdII secondary  air  monitor

test.

-  type1_codes = set of

{ccm_codes,purg_codes,sair_codes,ego_codes,egr_codes}.

23-56

OBDII STRATEGY, FAULT CODES - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: OBDII_CODES_COM1

BEGIN:  check_warmup_counters

;background process

;increments all warmup counters once per power-up if the fault

;is not currently illuminating the MIL.

OBDII_WARMUP = 0 -----------------|

|

ECT >= TCSTRT + 40 ---------------|AND -| OBDII_WARMUP := 1

|     |  ;cleared on power-up

ECT >= 160 -----------------------|

OBDII_WARMUP = 1 -----------------|

|

CODES_CHECKD = 0 -----------------|AND -| CODES_CHECKD := 1

|     |  ;cleared on power-up

DEMAND_MODE = 0 ------------------|     | Do: update_warmup_counters

|

| --- ELSE ---

|

DEMAND_MODE = 1 ------------------------| CODES_CHECKD := 1

END: check_warmup_counters

23-57

OBDII STRATEGY, FAULT CODES - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN:  update_warmup_counters

;private process, called by check_warmup_counters

;increments the warmup counters for all codes in the set "all_codes"

;if the coresponding code is not currently illuminating the MIL.

For each Pxxx in the set mil_codes loop:

PxxxMIL_ON = 1 -----------------------| PxxxCNT := 0

|

| --- ELSE ---

|

PxxxCNT >= 39 ------------------------| Do: erase_code(Pxxx)

|

| --- ELSE ---

|

| PxxxCNT := PxxxCNT + 1

End loop.

For each Pxxx in the set no_mil_codes loop:

PxxxCNT >= 39 ------------------------| Do: erase_code(Pxxx)

|

| --- ELSE ---

|

| PxxxCNT := PxxxCNT + 1

End loop.

END:  update_warmup_counters

23-58

OBDII STRATEGY, FAULT CODES - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The following services are exported by the fault code module.   They  may  be

used by other modules to manipulate fault codes and warmup counters.

BEGIN:  store_code(Pxxx)

;public process

;stores the fault code specified by Pxxx

PxxxFAULT = 0 --------------------|

|

Pxxx is in the set of mil_codes --|AND -| NUM_CODES :=

|     |    min((NUM_CODES+1), 127)

DEMAND_MODE = 0 ------------------|     |

| --- ELSE ---

|

PxxxFAULT = 0 --------------------|     |

|AND -| CODES_COUNT :=

DEMAND_MODE = 0 ------------------|     |    min((CODES_COUNT+1), 255)

PxxxCNT <> 0 ---------------------|

|AND -| OD_CODE_CNT :=

DEMAND_MODE = 1 ------------------|     |    min((OD_CODE_CNT+1), 255)

DEMAND_MODE = 0 ------------------------| PxxxFAULT := 1

| PxxxFAULT_A := 1

| PxxxCNT := 0

| PxxxMALF := 1

|

| --- ELSE ---

|

| PxxxCNT := 0

| PxxxMALF := 1

END: store_code

23-59

OBDII STRATEGY, FAULT CODES - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN:  erase_code(Pxxx)

;public process, called by update_warmup_counters

;erases the fault code specified by Pxxx

PxxxFAULT = 1 --------------------|

|

Pxxx is in the set of mil_codes --|AND -| NUM_CODES :=

|     |    max((NUM_CODES-1), 0)

DEMAND_MODE = 0 ------------------|

Pxxx is in the set of mil_codes --|

|

PxxxSTATE = 0 --------------------|

|

PxxxFAULT = 1 --------------------|AND -| CODES_COUNT :=

|     |    max((CODES_COUNT-1), 0)

DEMAND_MODE = 0 ------------------|     |

| --- ELSE ---

Pxxx in the set of non_mil_codes -|     |

|     |

PxxxFAULT = 1 --------------------|AND -| CODES_COUNT :=

|     |    max((CODES_COUNT-1), 0)

DEMAND_MODE = 0 ------------------|

PxxxCNT = 0 ----------------------|

|AND -| OD_CODE_CNT :=

DEMAND_MODE = 1 ------------------|     |    max((OD_CODE_CNT-1), 0)

23-60

OBDII STRATEGY, FAULT CODES - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEMAND_MODE = 0 ------------------------| PxxxFAULT := 0

| PxxxFAULT_A := 0

| PxxxCNT := 40

|

| --- ELSE ---

|

| PxxxCNT := min((PxxxCNT+1), 40)

xxx = FF_CODE --------------------------| Do: clear_ff

END:  erase_code

23-61

OBDII STRATEGY, FAULT CODES - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN:  erase_mis_code(Pxxx)

;public process, called by the misfire state controller

;erases the fault code specified by Pxxx

PxxxFAULT = 1 --------------------|

|

Pxxx is in the set of mil_codes --|AND -| NUM_CODES :=

|     |    max((NUM_CODES-1), 0)

DEMAND_MODE = 0 ------------------|

Pxxx is in the set of mil_codes --|

|

PxxxSTATE = 0 --------------------|

|

PxxxFAULT = 1 --------------------|AND -| CODES_COUNT :=

|     |    max((CODES_COUNT-1), 0)

DEMAND_MODE = 0 ------------------|     |

| --- ELSE ---

Pxxx in the set of non_mil_codes -|     |

|     |

PxxxFAULT = 1 --------------------|AND -| CODES_COUNT :=

|     |    max((CODES_COUNT-1), 0)

DEMAND_MODE = 0 ------------------|

PxxxCNT = 0 ----------------------|

|AND -| OD_CODE_CNT :=

DEMAND_MODE = 1 ------------------|     |    max((OD_CODE_CNT-1), 0)

DEMAND_MODE = 0 ------------------------| PxxxFAULT := 0

| PxxxFAULT_A := 0

| PxxxCNT := 40

|

| --- ELSE ---

|

| PxxxCNT := min((PxxxCNT+1), 40)

END:  erase_mis_code

23-62

OBDII STRATEGY, FAULT CODES - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN:  reset_codes

;public process

;resets all fault codes and counters to their initialization values

unconditionally ----------------------| NUM_CODES := 0

| CODES_COUNT := 0

| OD_CODE_CNT := 0

| CODES_CHECKD := 0

For each Pxxx in the set all_codes loop:

unconditionally ----------------------| PxxxFAULT := 0

| PxxxFAULT_A := 0

| PxxxCNT := 40

| PxxxMALF := 0

End loop.

END: reset_codes

23-63

OBDII STRATEGY, FAULT CODES - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN:  init_codes

;public process, executed during power-up initialization

;initialize all malfunction flags and updated flags to their initialization

;values, as well as clearing the OBDII_WARMUP and CODES_CHECKD flags.

unconditionally ----------------------| CODES_CHECKD := 0

| OBDII_WARMUP := 0

For each Pxxx in the set mil_codes loop:

unconditionally ----------------------| PxxxMALF := 0

| PxxxUPDATED := 0

End loop.

For each Pxxx in the set no_mil_codes loop:

unconditionally ----------------------| PxxxMALF := 0

End loop.

END: init_codes

BEGIN: inc_warmup_counters

;public process, used to add one to each and every warmup counter.

unconditionally -------------------------| OD_CODE_CNT := 0

For each Pxxx in the set all_codes loop:

PxxxCNT = 0 --------------------------| PxxxCNT := PxxxCNT + 1

End loop.

END: inc_warmup_counters

BEGIN: clear_malf(Pxxx)

;PUBLIC process used to clear the PxxxMALF flag.

PxxxMALF = 1 ---------------------------| PxxxMALF := 0

END: clear_malf

23-64

OBDII STRATEGY, TRIPS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

23.1.5  TRIPS (CDAM0)

OVERVIEW

+--------------------------------+

| OBDII_TRIPS_COM1               |

+--------------------------------+

| PUBLIC PROCESSES:              |

| . update_trip_flag             |

| . update_ready_flags           |

| . reset_trips                  |

| . init_trips                   |

+--------------------------------+

| PRIVATE PROCESSES:             |

| . check_readiness(sys)         |

+--------------------------------+

The term "trip" was defined by CARB in order to specify the requirements  for

obdII  testing.   A  "trip"  means vehicle operation (following an engine-off

period) of duration and driving mode such that all components and systems are

monitored at least once by the (obdII) diagnostic system.  This definition is

subject to the  limitation  that  the  manufacturer-defined  trip  monitoring

conditions  shall  all  be  encountered  at least once during a CVS-75 cycle.

Note that, because of the engine-off condition, this effectively  means  that

all  obdII  systems  must  be  monitored  during the 1st two bags of a CVS-75

cycle.  All driving cycles are not trips.  A driving cycle may be  too  short

to test all obdII systems.

The TRIP flag indicates that a trip has been completed.  This flag is cleared

on  power-up, and set when all systems are monitored.  It can also be cleared

by a scan tool reset.  Trips are used by the MIL  module  to  extinguish  the

MIL.   It  typically  takes  three  trips without a malfunction, (for a given

fault code), to turn the MIL off (for that code).

23-65

OBDII STRATEGY, TRIPS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  CAT_COUNT = Count of the number  of  complete  catalyst  test  since  KAM

reset.

-  EGO_COUNT = Count of the number of complete EGO test since KAM reset.

-  EGR_COUNT = Count of the number of complete EGR test since KAM reset.

-  PURG_COUNT = Count of the number of complete PURGE test since KAM reset.

-  PxxxSTATE = MIL state flag for code Pxxx.

-  READY_FLAGS = Bitmapped register containing OBDII ready flags and  MIL_ON

flag.

-  SAIR_COUNT = Count of the number of complete SAIR test since KAM reset.

-  TRIP_COUNT = Count of number of completed OBD-II Trips.

Bit Flags:

-  CAT_CHKD = Flag indicating CAT_READY has been checked this power-up.

-  CAT_MON = OBDII monitor flag for catalyst; 1 -> all catalyst faults  have

been monitored at least once since power-up.

-  CAT_READY = Catalyst obdII ready  flag.   Bit  0  of  READY_FLAGS;  1  ->

catalyst not monitored since kam reset.

-  CCM_MON = OBDII monitor flag  for  comprehensive  components;  1  ->  all

component faults have been monitored at least once since power-up.

-  EGO_CHKD = Flag indicating EGO_READY has been checked this power-up.

-  EGO_MON = OBDII monitor flag for exhaust gas oxygen sensor; 1 -> all  EGO

faults have been monitored at least once since power-up.

-  EGO_READY = EGO obdII ready flag.  Bit 5 of READY_FLAGS;  1  ->  ego  not

monitored since kam reset.

-  EGR_CHKD = Flag indicating EGR_READY has been checked this power-up.

-  EGR_MON = OBDII monitor flag for exhaust gas recirculation; 1 -> all  EGR

faults have been monitored at least once since power-up.

-  EGR_READY = EGR obdII ready flag.  Bit 6 of READY_FLAGS;  1  ->  egr  not

monitored since kam reset.

-  EO_MON = Flag indicating the engine off on-demand test is complete.

-  ER_MON = Flag indicating the engine running on-demand test is complete.

-  FUEL_MON = OBDII monitor flag for fuel; 1 -> all fuel  faults  have  been

monitored at least once since power-up.

23-66

OBDII STRATEGY, TRIPS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  MIS_MON = OBDII monitor flag for misfire; 1 -> all  misfire  faults  have

been monitored at least once since power-up.

-  PURG_CHKD = Flag indicating PURG_READY has been checked this power-up.

-  PURG_MON = OBDII monitor flag for purge; 1 -> all purge faults have  been

monitored at least once since power-up.

-  PURG_READY = Purge obdII ready flag.  Bit 2 of READY_FLAGS;  1  ->  purge

not monitored since kam reset.

-  SAIR_CHKD = Flag indicating SAIR_READY has been checked this power-up.

-  SAIR_MON = OBDII monitor flag for secondary air; 1 -> all  secondary  air

faults have been monitored at least once since power-up.

-  SAIR_READY = Secondary Air obdII ready flag.  Bit 3 of READY_FLAGS; 1  ->

secondary air not monitored since kam reset.

-  TRIP = Flag indicating that an obdII trip has been completed; 1  ->  trip

completed.

Calibration Constants:

-  CAT_TST_SW = CAT st present calibration switch; 1 -> CAT present.

-  CCM_TST_SW = CCM test present calibration switch; 1 -> CCM test present.

-  EGO_TST_SW = EGO test present calibration switch; 1 ->EGO test present.

-  EGR_TST_SW = EGR test present calibration switch; 1 -> EGR test present.

-  FUEL_TST_SW = FUEL test  present  claibration  switch;  1  ->  FUEL  test

present.

-  MIS_TST_SW = MIS test present claibration switch; 1 -> MIS test present.

-  PURG_TST_SW = PURG test  present  calibration  switch;  1  ->  PURG  test

present.

-  SAIR_TST_SW = SAIR test  present  calibration  switch;  1  ->  SAIR  test

present.

23-67

OBDII STRATEGY, TRIPS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: OBDII_TRIPS_COM1

BEGIN: update_trip_flag

;executed each background loop

TRIP = 0 -------------------------|

|

CAT_MON = 1 ----------------|     |

|OR --|

CAT_TST_SW = 0 -------------|     |

|

CCM_MON = 1 ----------------|     |

|OR --|

CCM_TST_SW = 0 -------------|     |

|

PURG_MON = 1 ---------------|     |

|OR --|

PURG_TST_SW = 0 ------------|     |

|

SAIR_MON = 1 ---------------|     |

|OR --|

SAIR_TST_SW = 0 ------------|     |

|AND -| TRIP := 1

FUEL_MON = 1 ---------------|     |     |  ;global trip flag

|OR --|     |

FUEL_TST_SW = 0 ------------|     |     | TRIP_COUNT := TRIP_COUNT + 1

|

EGO_MON = 1 ----------------|     |

|OR --|

EGO_TST_SW = 0 -------------|     |

|

EGR_MON = 1 ----------------|     |

|OR --|

EGR_TST_SW = 0 -------------|     |

|

MIS_MON = 1 ----------------|     |

|OR --|

MIS_TST_SW = 0 -------------|

END: update_trip_flag

23-68

OBDII STRATEGY, TRIPS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: update_ready_flags

;public process executed once per background pass to update the ready

;flags and store a code per CARB regulations if all system not ready.

CAT_TST_SW = 0 -------------------| CAT_READY := 0

|

| --- ELSE ---

CAT_MON = 1 ----------------|     |

|     |

CAT_CHKD = 0 ---------------|AND -| CAT_COUNT := CAT_COUNT + 1

|     | DO: check_readiness(cat)

CAT_READY = 1 --------------|     | CAT_CHKD := 1

PURG_TST_SW = 0 ------------------| PURG_READY := 0

|

| --- ELSE ---

PURG_MON = 1 ---------------|     |

|     |

PURG_CHKD = 0 --------------|AND -| PURG_COUNT := PURG_COUNT + 1

|     | DO: check_readiness(purg)

PURG_READY = 1 -------------|     | PURG_CHKD := 1

SAIR_TST_SW = 0 ------------------| SAIR_READY := 0

|

| --- ELSE ---

SAIR_MON = 1 ---------------|     |

|     |

SAIR_CHKD = 0 --------------|AND -| SAIR_COUNT := SAIR_COUNT + 1

|     | DO: check_readiness(sair)

SAIR_READY = 1 -------------|     | SAIR_CHKD := 1

EGO_TST_SW = 0 -------------------| EGO_READY := 0

|

| --- ELSE ---

EGO_MON = 1 ----------------|     |

|     |

EGO_CHKD = 0 ---------------|AND -| EGO_COUNT := EGO_COUNT + 1

|     | DO: check_readiness(ego)

EGO_READY = 1 --------------|     | EGO_CHKD := 1

EGR_TST_SW = 0 -------------------| EGR_READY := 0

|

| --- ELSE ---

EGR_MON = 1 ----------------|     |

|     |

EGR_CHKD = 0 ---------------|AND -| EGR_COUNT := EGR_COUNT + 1

|     | DO: check_readiness(egr)

EGR_READY = 1 --------------|     | EGR_CHKD := 1

23-69

OBDII STRATEGY, TRIPS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CAT_READY = 0 --------------------|

|

PURG_READY = 0 -------------------|

|

SAIR_READY = 0 -------------------|AND -| Do: erase_code(P1000)

|     |  ;signal all tests ready.

EGO_READY = 0 --------------------|     |

|     |

EGR_READY = 0 --------------------|     |

| --- ELSE ---

|

| Do: store_code(P1000)

|  ;hold warmup counter at zero.

END: update_ready_flags

23-70

OBDII STRATEGY, TRIPS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TRIP INITIALIZATION/RESET

INITIALIZATION:  The trip module is initialized on power-up.   The  trip  and

monitor  flags  are  cleared,  indicating  that  no  OBD-II  system  has been

monitored since power-up.  The Ready flags are not affected.

RESET:  The OBD-II scan tool can cause a reset of the trip module via the SCP

communications  interface.   An OBD-II reset performs the same actions as the

initialization above, and also sets the ready flags.  A  value  of  1  for  a

ready  flag  indicates  that the particular system corresponding to that flag

has not been monitored since the last OBD-II reset.

BEGIN: reset_trips

;executed when an obdII reset occurs

unconditionally ------------------------| CAT_READY := 1

| PURG_READY := 1

| SAIR_READY := 1

| EGO_READY := 1

| EGR_READY := 1

| CAT_COUNT := 0

| SAIR_COUNT := 0

| PURG_COUNT := 0

| EGO_COUNT := 0

| EGR_COUNT := 0

| TRIP_COUNT := 0

| CAT_CHKD := 0

| SAIR_CHKD := 0

| PURG_CHKD := 0

| EGO_CHKD := 0

| EGR_CHKD := 0

| Do: store_code(P1000)

| ;indicate not ready for I/M test

unconditionally ------------------------| TRIP := 0

| CAT_MON := 0

| CCM_MON := 0

| PURG_MON := 0

| SAIR_MON := 0

| FUEL_MON := 0

| EGO_MON := 0

| EGR_MON := 0

| MIS_MON := 0

| EO_MON := 0

| ER_MON := 0

| CAT_MON_LST := 0

| EGO_MON_LST := 0

| SAIR_MON_LST := 0

| PURG_MON_LST := 0

| CCM_MON_LST := 0

| EGR_MON_LST := 0

| MIS_MON_LST := 0

| FUEL_MON_LST := 0

| ;indicate OBD-II trip not complete

END: reset_trips

23-71

OBDII STRATEGY, TRIPS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: init_trips

;executed during power-up

unconditionally ------------------------| TRIP := 0

| CAT_MON := 0

| CCM_MON := 0

| PURG_MON := 0

| SAIR_MON := 0

| FUEL_MON := 0

| EGO_MON := 0

| EGR_MON := 0

| MIS_MON := 0

| EO_MON := 0

| ER_MON := 0

| CAT_MON_LST := 0

| EGO_MON_LST := 0

| SAIR_MON_LST := 0

| PURG_MON_LST := 0

| CCM_MON_LST := 0

| EGR_MON_LST := 0

| MIS_MON_LST := 0

| FUEL_MON_LST := 0

| CAT_CHKD := 0

| SAIR_CHKD := 0

| PURG_CHKD := 0

| EGO_CHKD := 0

| EGR_CHKD := 0

| ;indicate OBD-II trip not complete

END: init_trips

BEGIN:  check_readiness(sys)

;PRIVATE process used to determine if a system has been monitored enough

;times to illuminate the MIL,if appropriate.  In the case of the Catalyst

;monitor this would be three checks, in the case of the other monitors, it

;would be two.  The number of trips can be overridden if the first (or

second

;in the case of the Catalyst monitor) does not set any fault codes.

unconditionally ---------------------| sys_ready := 0

For each Pxxx in the set of sys_codes loop:

PxxxSTATE <> 0 ----------------------| sys_ready := 1

End loop.

unconditionally ---------------------| sys_READY := sys_ready

sys_COUNT >= 2 ----------------------| sys_READY := 0

|  ;system monitor is ready.

END:  check_readiness(sys)

23-72

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-TYPE 1 - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

23.1.6  STATE CONTROL-TYPE 1 (CDAK0)

OVERVIEW

This module contains the OBD-II MIL control procedures for the type  1  (egr,

purg,   ego,   ccm,   sair   ,fail   safe  cooling)  monitors.   A  pictorial

representation of what this object holds is shown below:

Type 1 State Controller

+------------------------------+

| OBDII_TYPE1_STATE_CONTROLLER |

+------------------------------+

| . type1_malfunction          |

| . type1_init_mil             |

| . type1_trip_complete        |

| . type1_reset                |

+------------------------------+

The state controller is a realization of  a  finite  state  machine.   It  is

purely  event  driven,  those  events  being  malfunctions,  trip completion,

power-up  initialization,  and  OBDII  reset  commands  from   the   external

environment.   There  are  no  background tasks associated with the MIL state

controllers.

The next page shows the operation of the finite state machine to control  the

type1 mil structures.

23-73

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-TYPE 1 - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Below is a description of how a finite state machine functions.

A finite state machine is a way to graphically illustrate the functioning  of

a  system  or  process  that can be in a set of DISCRETE states.  The diagram

shows how and when transitions can occur between the states.

+----------------------+

|                      |

|       STATE X        |<-------------------

|                      |        condition

+-----+----------A-----+        ---------

|          |               action

|          |

condition    |          |    condition

---------    |          |    ---------

action      |          |     action

|          |

|          |

+-----V----------+-----+

|                      |

|       STATE Y        |

|                      |

+----------------------+

CONVENTIONS:

· BOXES represent STATES of the system.

· ARROWS represent TRANSITIONS between states.

· CONDITIONS will CAUSE a transition to be taken.

· ACTIONS are done WHEN a transition is taken.

RULES:

· The system can be in only one state at a time.

· The system remains in a given state until an event

occurs to cause a transition.

· No other transitions than those shown can be taken.

23-74

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-TYPE 1 - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Type1 State Controller

+---------------------+

+------------------------ |       state 0       | <--------------------

|                         |         NO          |            reset_mil

|    +--------------------|    MALFUNCTIONS     |           -----------

|    | malfunction &      |  ( mil flag off )   | <----+    PxxxSTATE=0

|    | DEMAND_MODE=0      +---A-----------------+      |    PxxxMIL_ON=0

|    | ------------       | power_up                   |

|    | PxxxSTATE=1        | -----------                |

|    | PxxxUPDATED=1      | PxxxSTATE=0                | trip_complete

|    | inc(CODES_COUNT)   | dec(CODES_COUNT)           |      &

|    |   +--------------------+                        | PxxxUPDATED=0

|    |   |      state 2       |                        | -------------

|    |   |  ONE TRIP WITHOUT  |                        | PxxxSTATE=0

|    |   |     MALFUNCTION    |--------+               | PxxxMIL_ON=0

|    |   |  ( mil flag off )  |        |               |

|    |   +---A----------------+        |               |

|    |       |                         |       +---------------------+

|    |       |                         |       |       state 5       |

|    |       | trip_complete           |       |  TWO TRIPS WITHOUT  |

|    |       |      &                  |       |     MALFUNCTIONS    |

|    |       | PxxxUPDATED=0           |<------|   ( mil flag on )   |

|    |       | -------------     malfunction & +---A-----------------+

|    |       | PxxxSTATE=2       DEMAND_MODE=0     |

| +--V------------------+        -------------     | trip_complete

| |       state 1       |        PxxxSTATE=3       |      &

| |         ONE         |        PxxxUPDATED=1     | PxxxUPDATED=0

| |     MALFUNCTION     |        PxxxMIL_ON=1      | -------------

| |   ( mil flag off )  |        store_code        | PxxxSTATE=5

| +---------------------+        sys_MIL_ON = 1    | PxxxUPDATED=1

|    | malfunction &                   |       +---------------------+

|    | DEMAND_MODE=0 &                 |       |       state 4       |

|    | PxxxUPDATED=0                   |       |  ONE TRIP WITHOUT   |

|    | -------------                   |       |    MALFUNCTION      |

|    | PxxxSTATE=3                     |<------|   ( mil flag on )   |

|    | PxxxUPDATED=1                   |       +-------A-------------+

|    | PxxxMIL_ON=1                    |               |

|    | store_code                      |               |

|    | sys_MIL_ON = 1    +-------------V-------+       |

|    +-----------------> |       state 3       |-------+ trip_complete

|                        |     TWO OR MORE     |              &

|+---------------------> |     MALFUNCTIONS    |         PxxxUPDATED=0

malfunction &          |   ( mil flag on )   |         -------------

DEMAND_MODE=0 &        +---A-----------------+         PxxxSTATE=4

sys = cool                 |             |             PxxxUPDATED=1

-------------              |             |

PxxxSTATE=3                +-------------+

PxxxUPDATED=1               malfunction &

PxxxMIL_ON=1                DEMAND_MODE=0

store_code                  -------------

sys_MIL_ON=1                PxxxUPDATED=1

inc(CODES_COUNT)

23-75

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-TYPE 1 - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Bit Flags:

-  DEMAND_MODE = Flag set when an On-Demand self test mode is active.

-  PxxxMIL_ON = MIL flag for code Pxxx; 1 -> the MIL is required  to  be  on

for code Pxxx.

-  PxxxSTATE = MIL state flag for code Pxxx.

-  PxxxUPDATED = MIL state controller update status flag for code Pxxx; 1 ->

controller has been updated since power-up.

OUTPUTS

Bit Flags:

-  CCM_MIL_ON = MIL status flag  for  Comprehensive  Components;  1  ->  MIL

required for CCM fault.

-  CODES_COUNT = Count of total number of fault codes and pending codes (MIL

and NO_MIL) stored by the EEC.

-  CAT_MIL_ON = MIL status flag for Catalyst system; 1 -> MIL  required  for

catalyst fault.

-  COOL_MIL_ON = MIL status flag for fail safe cooling; 1  ->  MIL  required

for fail safe cooling

-  EGR_MIL_ON = MIL status flag for EGR; 1 -> MIL required for EGR fault.

-  PxxxMIL_ON = See above.

-  PxxxSTATE = See above.

-  PxxxUPDATED = See above.

-  PURG_MIL_ON = MIL status flag Purge; 1 -> MIL required for Purge fault.

-  SAIR_MIL_ON = MIL status flag for Secondary Air; 1 ->  MIL  required  for

Secondary Air fault.

23-76

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-TYPE 1 - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: OBDII_TYPE1_STATE_CONTROL_COM1

BEGIN: type1_malfunction(sys_mil_on, Pxxx, sys)

;This logic implements malfunction transitions for the type1 state

;machine.

DEMAND_MODE = 1 -------------------| Do: store_code(Pxxx)

|

| --- ELSE ---

PxxxSTATE = 0 ---------------|     |

|     |

sys = cool ------------------|AND -| PxxxSTATE := 3

|     | PxxxUPDATED := 1

PxxxFAULT = 0 ---------------|     | CODES_COUNT :=

|   min((CODES_COUNT + 1), 255)

| PxxxMIL_ON := 1

| Do: store_code(Pxxx)

| sys_mil_on := 1

|

| ___ ELSE ---

PxxxSTATE = 0 ---------------|     |

|AND -| PxxxSTATE := 3

sys = cool ------------------|     | PxxxUPDATED := 1

| PxxxMIL_ON := 1

| Do: store_code(Pxxx)

| sys_mil_on := 1

|

| ___ ELSE ---

PxxxSTATE = 0 ---------------|     |

|AND -| PxxxSTATE := 1

PxxxFAULT = 0 ---------------|     | PxxxUPDATED := 1

| CODES_COUNT :=

|   min((CODES_COUNT + 1), 255)

|

| --- ELSE ---

|

PxxxSTATE = 0 ---------------------| PxxxSTATE := 1

| PxxxUPDATED := 1

|

| --- ELSE ---

PxxxSTATE = 1 ---------------|     |

|AND -| PxxxSTATE := 3

PxxxUPDATED = 0 -------------|     | PxxxUPDATED := 1

| PxxxMIL_ON := 1

| Do: store_code(Pxxx)

| sys_mil_on := 1

|

| --- ELSE ---

(continued on next page)

23-77

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-TYPE 1 - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

PxxxSTATE = 2 ---------------|     |

|     |

PxxxSTATE = 4 ---------------|OR --| PxxxSTATE := 3

|     | PxxxUPDATED := 1

PxxxSTATE = 5 ---------------|     | PxxxMIL_ON := 1

| Do: store_code(Pxxx)

| sys_mil_on := 1

|

| --- ELSE ---

|

PxxxSTATE = 3 ---------------------| PxxxUPDATED := 1

END: type1_malfunction(sys_mil_on, Pxxx, sys)

BEGIN: type1_init_mil(sys)

;Private procedure performed upon request from the initialize_mil

;procedure.

For each Pxxx in the set of sys_codes loop:

PxxxSTATE = 2 ---------------|

|AND -| PxxxSTATE := 0

PxxxFAULT = 0 ---------------|     | CODES_COUNT :=

|   max((CODES_COUNT-1), 0)

|

| --- ELSE ---

|

PxxxSTATE = 2 ---------------------| PxxxSTATE := 0

End loop.

END: type1_init_mil(sys)

23-78

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-TYPE 1 - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: type1_trip_complete(sys, sys_mil_on)

; this logic updates the type1 state machines for each Pxxx in sys.

unconditionally -------------------| sys_mil_on := 0

For each  Pxxx  in the set of sys_codes loop:

PxxxSTATE = 1 ---------|

|

PxxxSTATE = 3 ---------|OR --|

|     |

PxxxSTATE = 4 ---------|     |AND -| PxxxSTATE := PxxxSTATE + 1

|     | PxxxUPDATED := 1

PxxxUPDATED = 0 -------------|     |

| --- ELSE ---

PxxxSTATE = 5 ---------------|     |

|AND -| PxxxSTATE := 0

PxxxUPDATED = 0 -------------|     | PxxxMIL_ON := 0

PxxxMIL_ON = 1 --------------------| sys_mil_on := 1

End loop.

END: type1_trip_complete(sys, sys_mil_on)

BEGIN: type1_reset

;executed when an obdII reset message is received from the scan tool

For all codes in the set of type1_codes loop:

unconditionally --------------------| PxxxUPDATED := 0

| PxxxSTATE := 0

| PxxxMIL_ON := 0

End loop.

unconditionally ----------------------| CCM_MIL_ON := 0

| COOL_MIL_ON := 0

| PURG_MIL_ON := 0

| SAIR_MIL_ON := 0

| EGO_MIL_ON := 0

| EGR_MIL_ON := 0

| CAT_MIL_ON := 0

END: type1_reset

23-79

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

23.1.7  STATE CONTROL-FUEL (CDAK0)

OVERVIEW

This module contains the OBD-II MIL control procedures for  the  fuel  system

monitor.   A  pictorial  representation  of  what  this object holds is shown

below:

Fuel system State Controller

+------------------------------+

| OBDII_FUEL_STATE_CONTROLLER  |

+------------------------------+

| . fuel_malfunction           |

| . fuel_init_mil              |

| . fuel_trip_complete         |

| . fuel_reset                 |

| . fuel_conds_seen            |

+------------------------------+

The state controller is a realization of  a  finite  state  machine.   It  is

purely  event  driven,  those  events  being  malfunctions,  trip completion,

power-up  initialization,  and  OBDII  reset  commands  from   the   external

environment.   There  are  no  background tasks associated with the MIL state

controllers.

The next page shows the operation of the finite state machine to control  the

fuel mil structures.

23-80

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Below is a description of how a finite state machine functions.

A finite state machine is a way to graphically illustrate the functioning  of

a  system  or  process  that can be in a set of DISCRETE states.  The diagram

shows how and when transitions can occur between the states.

+----------------------+

|                      |

|       STATE X        |<-------------------

|                      |        condition

+-----+----------A-----+        ---------

|          |               action

|          |

condition    |          |    condition

---------    |          |    ---------

action      |          |     action

|          |

|          |

+-----V----------+-----+

|                      |

|       STATE Y        |

|                      |

+----------------------+

CONVENTIONS:

· BOXES represent STATES of the system.

· ARROWS represent TRANSITIONS between states.

· CONDITIONS will CAUSE a transition to be taken.

· ACTIONS are done WHEN a transition is taken.

RULES:

· The system can be in only one state at a time.

· The system remains in a given state until an event

occurs to cause a transition.

· No other transitions than those shown can be taken.

23-81

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Fuel System State Controller

+-------------------+

|       state 0     | <-----------------------

|         NO        |           reset_mil

|    MALFUNCTIONS   |          -----------

+----------------------|  ( mil flag off ) | <---+    PxxxSTATE=0

| malfunction &        +------------A------+     |    PxxxMIL_ON=0

| DEMAND_MODE=0                     |            |    clear_conds

| --------------            fuel_conds_seen      +----+

| PxxxSTATE=1                     OR                  | power_up  &

| PxxxUPDATED=1             (power_up &               | PxxxSEEN=1 &

| get_conds                  PxxxACTIVE=1 &           | CL_SEEN=1

| inc(CODES_COUNT)           PxxxTRIP_CT=79)          | -----------

|                            ----------------         | PxxxSTATE=0

|                            PxxxSTATE=0              | PxxxMIL_ON=0

|                            clear_similar_conds      | clear_conds

|                            dec(CODES_COUNT)         |

|                                   |           +---------------+

|       +-+power_up &               |           |    state 5    |

|       | |PxxxACTIVE=1             |           |  3RD DRIVING  |

|       | |& PxxxTRIP_CT<79         |           |     CYCLE     |

|       | |------------             |   +-------| (mil flag on) |-------+

|       | |PxxxUPDATED=0            |   |       +-----A---------+       |

|       | |PxxxTRIP_FG=0            |   |             | power_up  &     |

|       | |update_pxxxtrip_ct       |   |             | PxxxSEEN=1 &    |

|       | |                         |   |             | CL_SEEN=1       |

|       | |       +-+ power_up &    |   |             | ------------    |

|       | |       | | PxxxACTIVE=0  |   |             | PxxxSTATE=5     |

|       | |       | | ---------     |   |             | PxxxSEEN=0      |

+V-------V-+-------V+| PxxxUPDATED=0 |   |       +---------------+       |

|       state 1     || PxxxACTIVE=1  |   |       |    state 4    |       |

|      VERIFYING    || PxxxTRIP_FG=0 |   |       |  2ND DRIVING  |       |

|     MALFUNCTION   |+ PxxxTRIP_CT=0 |   |<------|     CYCLE     |------>|

|   ( mil flag off )|----------------+   |       | (mil flag on) |       |

+---+-----A-----+---+                    |       +-----A---------+       |

|     |     | trip_complete          |             | power_up  &     |

|     |     | -------------     malfunction &      | PxxxSEEN=1      |

|     +-----+ PxxxTRIP_FG=1     DEMAND_MODE=0      | CL_SEEN=1       |

|                              -------------       | -----------     |

| malfunction &                 PxxxSTATE=2        | PxxxSTATE=4     |

| DEMAND_MODE=0 &                    |             | PxxxSEEN=0      |

| PxxxUPDATED=0                      |       +---------------+       |

| -------------                      |       |    state 3    |       |

| PxxxSTATE=2                        |       |  1ST DRIVING  |       |

| PxxxMIL_ON=1                       |       |     CYCLE     |       |

| store_code                         |<------| (mil flag on) |       |

| FUEL_MIL_ON = 1                    |       +-----A-------A-+       |

|                     +--------------V------+      |       |         |

+-------------------> |       state 2       |------+       +---------+

|     MALFUNCTION     | power_up     power_up  &

|       PRESENT       | ----------   PxxxSEEN=0 &

|   ( mil flag on )   | PxxxSTATE=3  CL_SEEN=1

+---------------------+ PxxxSEEN=0   ----------

PxxxSTATE=3

23-82

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  CODES_COUNT = Count of total number of fault codes and pending codes (MIL

and NO_MIL) stored by the EEC.

Bit Flags:

-  CL_SEEN = Flag indicating if closed-loop fuel  operation  has  been  seen

this driving cycle.

-  DEMAND_MODE = Flag set when an On-Demand self test mode is active.

-  PxxxACTIVE = Similar conditions active flag; 1 -> conditons active.

-  PxxxMIL_ON = MIL flag for code Pxxx; 1 -> the MIL is required  to  be  on

for code Pxxx.

-  PxxxSEEN = Similar conditions seen flag; 1 -> conditions seen.

-  PxxxSTATE = MIL state flag for code Pxxx.

-  PxxxTRIP_CT = Similar conditions trip counter (0 - 80).

-  PxxxTRIP_FG = Similar conditions trip flag; 1 -> trip occured.

-  PxxxUPDATED = MIL state controller update status flag for code Pxxx; 1 ->

controller has been updated since power-up.

OUTPUTS

Registers:

-  CODES_COUNT = See above.

Bit Flags:

-  FUEL_MIL_ON = MIL status flag for Fuel System; 1 -> MIL required for Fuel

System fault.

-  PxxxACTIVE = See above.

-  PxxxMIL_ON = See above.

-  PxxxSEEN = See above.

-  PxxxSTATE = See above.

-  PxxxTRIP_CT = See above.

-  PxxxTRIP_FG = See above.

23-83

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PxxxUPDATED = See above.

PROCESS

STRATEGY MODULE: OBDII_FUEL_STATE_CONTROL_COM1

BEGIN:  fuel_malfunction(sys_mil_on, Pxxx)

;This logic implements malfunction transitions for the fuel state machine

DEMAND_MODE = 1 -------------------| Do: store_code(Pxxx)

|

| --- ELSE ---

PxxxSTATE = 0 ---------------|     |

|AND -| PxxxSTATE := 1

PxxxFAULT = 0 ---------------|     | PxxxUPDATED := 1

| Do: get_similar_conds(Pxxx)

| CODES_COUNT :=

|  min((CODES_COUNT+1), 255)

|

| --- ELSE ---

PxxxSTATE = 0 ---------------------| PxxxSTATE := 1

| PxxxUPDATED := 1

| Do: get_similar_conds(Pxxx)

|

| --- ELSE ---

PxxxSTATE = 1 ---------------|     |

|AND -| PxxxSTATE := 2

PxxxUPDATED = 0 -------------|     | PxxxMIL_ON := 1

| Do: store_code(Pxxx)

| sys_mil_on := 1

|

| --- ELSE ---

PxxxSTATE = 3 ---------------|     |

|     |

PxxxSTATE = 4 ---------------|OR --| PxxxSTATE := 2

|

PxxxSTATE = 5 ---------------|

END:  fuel_malfunction(sys_mil_on, Pxxx)

23-84

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: fuel_init_mil(sys_mil_on)

;Private procedure performed upon request from the initialize_mil

;procedure.  sys_mil_on symbolically represents the flag for the

;mil controller for this system.

unconditionally -------------------| sys_mil_on := 0

For each Pxxx in the set of fuel_codes loop:

PxxxSTATE = 1 ---------------|

|

PxxxTRIP_CT = 79 ------------|AND -| PxxxSTATE := 0

|     | Do: clear_similar_conds(Pxxx)

PxxxFAULT = 0 ---------------|     | CODES_COUNT :=

|  max((CODES_COUNT-1), 0)

|

| --- ELSE ---

PxxxSTATE = 1 ---------------|     |

|AND -| PxxxSTATE := 0

PxxxTRIP_CT = 79 ------------|     | Do: clear_similar_conds(Pxxx)

|

| --- ELSE ---

PxxxSTATE = 1 ---------------|     |

|AND -| PxxxUPDATED := 0

PxxxACTIVE = 0 --------------|     | PxxxACTIVE := 1

| PxxxTRIP_FG := 0

| PxxxTRIP_CT := 0

|

| --- ELSE ---

PxxxSTATE = 1 ---------------|     |

|AND -| PxxxUPDATED := 0

PxxxTRIP_FG = 1 -------------|     | PxxxTRIP_FG := 0

| PxxxTRIP_CT := PxxxTRIP_CT + 1

|

| --- ELSE ---

|

PxxxSTATE = 1 ---------------------| PxxxUPDATED := 0

| PxxxTRIP_FG := 0

|

| --- ELSE ---

|

PxxxSTATE = 2 ---------------------| PxxxSTATE := 3

| PxxxSEEN := 0

|

| --- ELSE ---

(continued on next page)

23-85

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

PxxxSTATE = 3 ---------|           |

|OR --|     |

PxxxSTATE = 4 ---------|     |     |

|AND -| PxxxSTATE := PxxxSTATE + 1

PxxxSEEN = 1 ----------------|     | PxxxSEEN := 0

|     |

CL_SEEN = 1 -----------------|     |

| --- ELSE ---

PxxxSTATE = 4 ---------------|     |

|AND -| PxxxSTATE := 3

CL_SEEN = 1 -----------------|     |

| --- ELSE ---

PxxxSTATE = 5 ---------------|     |

|     |

PxxxSEEN = 1 ----------------|AND -| PxxxSTATE := 0

|     | PxxxMIL_ON := 0

CL_SEEN = 1 -----------------|     | Do: clear_similar_conds(Pxxx)

|

| --- ELSE ---

PxxxSTATE = 5 ---------------|     |

|AND -| PxxxSTATE := 3

CL_SEEN = 1 -----------------|

PxxxMIL_ON = 1 --------------------| sys_mil_on := 1

End loop.

END: fuel_init_mil

BEGIN: fuel_trip_complete

;This logic updates the fuel system state machines for each fuel system

;Pxxx.

For each Pxxx in the set of fuel_codes loop:

PxxxSTATE = 1 ---------------------| PxxxTRIP_FG := 1

End loop.

END: fuel_trip_complete

23-86

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-FUEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN:  fuel_reset

;executed when an obdII reset message is received from the scan tool

For all codes in the set of fuel_codes loop:

unconditionally -------------------| PxxxUPDATED := 0

| PxxxSTATE := 0

| PxxxMIL_ON := 0

| Do: clear_similar_conds(Pxxx)

End loop.

unconditionally -------------------| FUEL_MIL_ON := 0

END:  fuel_reset

BEGIN:  fuel_conds_seen(Pxxx)

;PUBLIC process called on demand from the SIMILAR CONDITIONS module when

;similar conditions have been encountered without a malfunction.

PxxxSTATE = 1 ---------------|

|AND -| PxxxSTATE := 0

PxxxFAULT = 0 ---------------|     | Do: clear_similar_conditions(Pxxx)

| CODES_COUNT := max((CODES_COUNT-1),0)

|

| --- ELSE ---

|

PxxxSTATE = 1 ---------------------| PxxxSTATE := 0

| Do: clear_similar_conditions(Pxxx)

END: fuel_conds_seen(Pxxx)

23-87

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-MISFIRE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

23.1.8  STATE CONTROL-MISFIRE (CDAK0)

OVERVIEW

This module contains the  OBD-II  MIL  control  procedures  for  the  misfire

monitor.   A  pictorial  representation  of  what  this object holds is shown

below:

Mis system State Controller

+------------------------------+

| OBDII_MIS_STATE_CONTROLLER   |

+------------------------------+

| . mis_malfunction            |

| . mis_init_mil               |

| . mis_trip_complete          |

| . mis_reset                  |

| . unknown_misfire            |

| . known_misfire              |

| . mis_conds_seen             |

+------------------------------+

The state controller is a realization of  a  finite  state  machine.   It  is

purely  event  driven,  those  events  being  malfunctions,  trip completion,

power-up  initialization,  and  OBDII  reset  commands  from   the   external

environment.   There  are  no  background tasks associated with the MIL state

controllers.

The next page shows the operation of the finite state machine to control  the

fuel mil structures.

23-88

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-MISFIRE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Below is a description of how a finite state machine functions.

A finite state machine is a way to graphically illustrate the functioning  of

a  system  or  process  that can be in a set of DISCRETE states.  The diagram

shows how and when transitions can occur between the states.

+----------------------+

|                      |

|       STATE X        |<-------------------

|                      |        condition

+-----+----------A-----+        ---------

|          |               action

|          |

condition    |          |    condition

---------    |          |    ---------

action      |          |     action

|          |

|          |

+-----V----------+-----+

|                      |

|       STATE Y        |

|                      |

+----------------------+

CONVENTIONS:

· BOXES represent STATES of the system.

· ARROWS represent TRANSITIONS between states.

· CONDITIONS will CAUSE a transition to be taken.

· ACTIONS are done WHEN a transition is taken.

RULES:

· The system can be in only one state at a time.

· The system remains in a given state until an event

occurs to cause a transition.

· No other transitions than those shown can be taken.

23-89

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-MISFIRE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Misfire State Controller

+-----------------+

+---------------------------|     state 0     | <---------------------

|                           |       NO        |  reset_mil OR known_misfire

|  +------------------------|  MALFUNCTIONS   |  --------------------------

|  | malfunction &          | (mil flag off)  | <-----+    PxxxSTATE=0

|  | DEMAND_MODE=0 &        +--------A--------+       |    PxxxMIL_ON=0

|  | TYPE_A_MIS=0                    |                | clear_similar_conds

|  | --------------            mis_conds_seen()       +---+

|  | PxxxSTATE=1                     OR                   | power_up  &

|  | PxxxUPDATED=1              (power_up &               | PxxxSEEN=1 &

|  | get_similar_conds           PxxxACTIVE=1 &           | CL_SEEN=1

|  | inc(CODES_COUNT)            PxxxTRIP_CT=79)          | -----------

|  |                             ----------------         | PxxxSTATE=0

|  |                             PxxxSTATE=0              | PxxxMIL_ON=0

|  |    +---+ power_up &         clear_similar_conds      | clear_similar_

|  |    |   | PxxxACTIVE=1       dec(CODES_COUNT)         |  conds

|  |    |   | & PxxxTRIP_CT<79       |           +--------+------+

|  |    |   | ------------           |           |    state 5    |

|  |    |   | PxxxUPDATED=0          |           |  3RD DRIVING  |

|  |    |   | PxxxTRIP_FG=0          |           |     CYCLE     |

|  |    |   | update_pxxxtrip_ct     |   +-------| (mil flag on) |-------+

|  |    |   |                        |   |       +-------A-------+       |

|  |    |   |   +-----+power_up &    |   |               | power_up  &   |

|  |    |   |   |     |PXXXACTIVE=0  |   |               | PxxxSEEN=1 &  |

|  |    |   |   |     |----------    |   |               | CL_SEEN=1     |

|  |    |   |   |     |PxxxUPDATED=0 |   |               | -----------   |

|  |    |   |   |     |PxxxACTIVE=1  |   |               | PxxxSTATE=5   |

|+-V----V---+---V--+  |PxxxTRIP_FG=0 |   |               | PxxxSEEN=0    |

||     state 1     |--+PxxxTRIP_CT=0 |   |       +-------+-------+       |

||    VERIFYING    |                 |   |       |    state 4    |       |

||   MALFUNCTION   |                 |   |       |  2ND DRIVING  |       |

|| (mil flag off)  |-----------------+   |       |     CYCLE     |       |

|+---+-----A---+---+                     |<------| (mil flag on) |------>|

|    |     |   | trip_complete           |       +-------A-------+       |

|    |     |   | -------------           |               | power_up  &   |

|    |     +---+ PxxxTRIP_FG=1           |               | PxxxSEEN=1 &  |

|    | malfunction &                malfunction &        | CL_SEEN=1     |

|    | DEMAND_MODE=0 &              DEMAND_MODE=0        | -----------   |

|    | ((PxxxUPDATED=0) OR         -------------         | PxxxSTATE=4   |

|    | (TYPE_A_MIS=1))              PxxxSTATE=2          | PxxxSEEN=0    |

|    | -------------                     |       +-------+-------+       |

|    | PxxxSTATE=2                       |       |    state 3    |       |

|    | PxxxMIL_ON=1                      |       |  1ST DRIVING  |       |

|    | store_code                        |       |     CYCLE     |       |

|    | MIS_MIL_ON=1                      |<------| (mil flag on) |       |

|    |                                   |       +-----A-------A-+       |

|    +---------------------->+-----------V---+         |       |         |

|  unknown_misfire OR        |   state 2     |---------+       +---------+

|(malfunction & DEMAND_MODE=0| MALFUNCTION   | power_up        power_up  &

+- & TYPE_A_MIS=1)---------->|   PRESENT     | -----------     PxxxSEEN=0 &

-------------             |(mil flag on)  | PxxxSTATE=3     CL_SEEN = 1

PxxxSTATE=2              +---------------+ PxxxSEEN=0      ------------

PxxxMIL_ON=1                                               PxxxSTATE=3

MIS_MIL_ON=1

*get_similar_conds

*PxxxACTIVE=1                *Sequence of these two actions is critical.

store_code & inc(CODES_COUNT)

23-90

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-MISFIRE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  CODES_COUNT = Count of total number of fault codes and pending codes (MIL

and NO_MIL) stored by the EEC.

-  FF_CODE = Code number of fault for  which  freeze  frame  conditions  are

stored.  freeze frame is loaded.

Bit Flags:

-  CL_SEEN = Flag indicating if closed-loop fuel  operation  has  been  seen

this driving cycle.

-  DEMAND_MODE = Flag set when an On-Demand self test mode is active.

-  PxxxACTIVE = Similar conditions active flag; 1 -> conditons active.

-  PXXXMIL_ON = MIL flag for code PXXX:  1 -> the MIL in required to  be  on

for code PXXX.

-  PxxxSEEN = Similar conditions seen flag; 1 -> conditions seen.

-  PxxxSTATE = MIL state flag for code Pxxx.

-  PxxxTRIP_CT = Similar conditions trip counter (0 - 80).

-  PxxxTRIP_FG = Similar conditions trip flag; 1 -> trip occured.

-  PxxxUPDATED = MIL state controller update status flag for code Pxxx; 1 ->

controller has been updated since power-up.

-  TYPE_A_MIS = Indicates to executive which misfire criteria.

23-91

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-MISFIRE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OUTPUTS

Registers:

-  CODES_COUNT = See above.

-  MIS_FF_CODE = Integer representation of  the  first  misfire  malfunction

code number after an unknown misfire.

-  UNK_MIS_CODE = Integer representation of the unknown  cylinder  misfiring

fault code number.

Bit Flags:

-  MIS_MIL_ON = MIL status flag for Misfire; 1 -> MIL required  for  Misfire

fault.

-  PxxxACTIVE = See above.

-  PXXXMIL_ON = See above.

-  PxxxSEEN = See above.

-  PxxxSTATE = See above.

-  PxxxTRIP_CT = See above.

-  PxxxTRIP_FG = See above.

-  PxxxUPDATED = See above.

23-92

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-MISFIRE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: OBDII_MIS_STATE_CONTROL_COM1

BEGIN  mis_malfunction(sys_mil_on, Pxxx)

;This logic implements malfunction transitions for the misfire state

;machine

DEMAND_MODE = 1 -------------------| Do: store_code(Pxxx)

|

PxxxSTATE = 0 ---------------|     | --- ELSE ---

|     |

TYPE_A_MIS = 1 --------------|AND -| PxxxSTATE := 2

|     | PxxxMIL_ON := 1

PxxxFAULT = 0 ---------------|     | Do: get_similar_conds(Pxxx)

| PxxxACTIVE := 1

|  ;sequence of the above two actions

|  ;is critical.

| Do: store_code(Pxxx)

| sys_mil_on := 1

| CODES_COUNT :=

|  min((CODES_COUNT+1), 255)

|

| --- ELSE ---

PxxxSTATE = 0 ---------------|     |

|AND -| PxxxSTATE := 2

TYPE_A_MIS = 1 --------------|     | PxxxMIL_ON := 1

| Do: get_similar_conds(Pxxx)

| PxxxACTIVE := 1

|  ;sequence of the above two actions

|  ;is critical.

| Do: store_code(Pxxx)

| sys_mil_on := 1

|

| --- ELSE ---

PxxxSTATE = 0 ---------------|     |

|     |

TYPE_A_MIS = 0 --------------|AND -| PxxxSTATE := 1

|     | PxxxUPDATED := 1

PxxxFAULT = 0 ---------------|     | Do: get_similar_conds(Pxxx)

| CODES_COUNT :=

|  min((CODES_COUNT+1), 255)

|

| --- ELSE ---

PxxxSTATE = 0 ---------------|     |

|AND -| PxxxSTATE := 1

TYPE_A_MIS = 0 --------------|     | PxxxUPDATED := 1

| Do: get_similar_conds(Pxxx)

|

(continued on next page)

23-93

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-MISFIRE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

| --- ELSE ---

PxxxSTATE = 1 ---------------|     |

|AND -| PxxxSTATE := 2

TYPE_A_MIS = 1 --------|     |     | PxxxMIL_ON := 1

|OR --|     | Do: store_code(Pxxx)

PxxxUPDATED = 0 -------|           | sys_mil_on := 1

|

| --- ELSE ---

PxxxSTATE = 3 ---------------|     |

|     |

PxxxSTATE = 4 ---------------|OR --| PxxxSTATE := 2

|

PxxxSTATE = 5 ---------------|

END  mis_malfunction(sys_mil_on, Pxxx)

23-94

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-MISFIRE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: mis_init_mil(sys_mil_on)

;Private procedure performed upon request from the initialize_mil

;procedure.  sys_mil_on symbolically represents the flag for the

;mil controller for this system.

unconditionally -------------------| sys_mil_on := 0

For each Pxxx in the set of mis_codes loop:

PxxxSTATE = 1 ---------------|

|

PxxxTRIP_CT = 79 ------------|AND -| PxxxSTATE := 0

|     | Do: clear_similar_conds(Pxxx)

PxxxFAULT = 0 ---------------|     | CODES_COUNT :=

|  max((CODES_COUNT-1), 0)

|

| --- ELSE ---

PxxxSTATE = 1 ---------------|     |

|AND -| PxxxSTATE := 0

PxxxTRIP_CT = 79 ------------|     | Do: clear_similar_conds(Pxxx)

|

| --- ELSE ---

PxxxSTATE = 1 ---------------|     |

|AND -| PxxxUPDATED := 0

PxxxACTIVE = 0 --------------|     | PxxxACTIVE := 1

| PxxxTRIP_FG := 0

| PxxxTRIP_CT := 0

|

| --- ELSE ---

PxxxSTATE = 1 ---------------|     |

|AND -| PxxxUPDATED := 0

PxxxTRIP_FG = 1 -------------|     | PxxxTRIP_FG := 0

| PxxxTRIP_CT := PxxxTRIP_CT + 1

|

| --- ELSE ---

|

PxxxSTATE = 1 ---------------------| PxxxUPDATED := 0

| PxxxTRIP_FG := 0

|

| --- ELSE ---

|

PxxxSTATE = 2 ---------------------| PxxxSTATE := 3

| PxxxSEEN := 0

|

| --- ELSE ---

(continued on next page)

23-95

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-MISFIRE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

PxxxSTATE = 3 ---------|           |

|OR --|     |

PxxxSTATE = 4 ---------|     |     |

|AND -| PxxxSTATE := PxxxSTATE + 1

PxxxSEEN = 1 ----------------|     | PxxxSEEN := 0

|     |

CL_SEEN = 1 -----------------|     |

| --- ELSE ---

|

PxxxSTATE = 4 ---------------------| PxxxSTATE := 3

|

| --- ELSE ---

PxxxSTATE = 5 ---------------|     |

|     |

PxxxSEEN = 1 ----------------|AND -| PxxxSTATE := 0

|     | PxxxMIL_ON := 0

CL_SEEN = 1 -----------------|     | Do: clear_similar_conds(Pxxx)

|

| --- ELSE ---

|

PxxxSTATE = 5 ---------------------| PxxxSTATE := 3

PxxxMIL_ON = 1 --------------------| sys_mil_on := 1

End loop.

END: mis_init_mil

BEGIN: mis_trip_complete

; this logic updates the misfire state machines for each misfire Pxxx

For each Pxxx in the set of mis_codes loop:

PxxxSTATE = 1 ---------------------| PxxxTRIP_FG := 1

End loop.

END: mis_trip_complete

23-96

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-MISFIRE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN:  mis_reset

;executed when an obdII reset message is received from the scan tool

For all codes in the set of mis_codes loop:

unconditionally -----------------| PxxxUPDATED := 0

| PxxxSTATE := 0

| PxxxMIL_ON := 0

| Do: clear_similar_conds(Pxxx)

End loop.

unconditionally -------------------| MIS_MIL_ON := 0

| UNK_MIS_CODE := 0

| MIS_FF_CODE := 0

END:  mis_reset

BEGIN: unknown_misfire(Pxxx)

;This procedure is called when misfire is detected, but the specific

;misfiring cylinder cannot be identified.

unconditionally -------------------| Do: mis_reset

| PxxxSTATE := 2

| PxxxMIL_ON := 1

| MIS_MIL_ON := 1

| Do: get_similar_conds(Pxxx)

| PxxxACTIVE := 1

| Do: store_code(Pxxx)

| UNK_MIS_CODE := xxx

For each Pyyy in the set of mis_codes loop:

yyy <> xxx ------------------------| Do: erase_mis_code(Pyyy)

yyy = FF_CODE ---------------|

|AND -| MIS_FF_CODE := 0

yyy <> xxx ------------------|     |  ;reset the misfire code

|  ;first seen after code

|  ;this process is executed.

| Do: swap_ff_code(xxx)

|  ;overwrite freeze frame code

|  ;number.

End loop.

END: unknown_misfire(Pxxx)

23-97

OBDII DIAGNOSTIC EXECUTIVE, STATE CONTROL-MISFIRE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: known_misfire(Pxxx)

;This procedure is used to reset the state machines for the misfire system

;when misfire is occuring and the misfiring cylinder can be identified and

;previously, misire occured where the misfiring cylinder could not be

;identified.

unconditionally -------------------| PxxxUPDATED := 0

| PxxxSTATE := 0

| PxxxMIL_ON := 0

| Do: clear_similar_conds(Pxxx)

| Do: erase_mis_code(Pxxx)

FF_CODE = xxx ---------------------| Do: swap_ff_code(MIS_FF_CODE)

END: known_misfire(Pxxx)

BEGIN: mis_conds_seen(Pxxx)

;This process is called from within the similar conditions module when

;misfire similar conditions have been observed.

PxxxSTATE = 1 ---------------|

|AND -| PxxxSTATE := 0

PxxxFAULT = 0 ---------------|     | DO: clear_similar_conds(Pxxx)

| CODES_COUNT :=

|  max((CODES_COUNT-1), 0)

|

| --- ELSE ---

|

PxxxSTATE = 1 ---------------------| PxxxSTATE := 0

| DO: clear_similar_conds(Pxxx)

END: mis_conds_seen(Pxxx)

23-98

OBDII DIAGNOSTIC EXECUTIVE, CLOSED LOOP ENTRY MIL CONTORL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

23.1.9  CLOSED LOOP ENTRY MIL CONTROL (CDAK0)

OVERVIEW

+------------------------------+

| OBDII_CL_STATE_CONTROLLER    |

+------------------------------+

| . cl_malfunction             |

| . cl_init_mil                |

| . cl_trip_complete           |

| . cl_reset                   |

+------------------------------+

This module define the state transitions to control  the  single  fault  code

that  gets  set  whne  the  engine  control system fails to enter closed loop

within a manufacturer specified minimum time.

Below is a description of how a finite state machine functions.

A finite state machine is a way to graphically illustrate the functioning  of

a  system  or  process  that can be in a set of DISCRETE states.  The diagram

shows how and when transitions can occur between the states.

+----------------------+

|                      |

|       STATE X        |<-------------------

|                      |        condition

+-----+----------A-----+        ---------

|          |               action

|          |

condition    |          |    condition

---------    |          |    ---------

action      |          |     action

|          |

|          |

+-----V----------+-----+

|                      |

|       STATE Y        |

|                      |

+----------------------+

CONVENTIONS:

· BOXES represent STATES of the system.

· ARROWS represent TRANSITIONS between states.

· CONDITIONS will CAUSE a transition to be taken.

· ACTIONS are done WHEN a transition is taken.

RULES:

· The system can be in only one state at a time.

· The system remains in a given state until an event

occurs to cause a transition.

· No other transitions than those shown can be taken.

23-99

OBDII DIAGNOSTIC EXECUTIVE, CLOSED LOOP ENTRY MIL CONTORL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FAILURE TO ENTER CLOSED LOOP

MIL STATE CONTROL

NOTE: The calibration switch CL_OBDI_SW must be set (=1) in order to use

the MIL control strategy shown below.  If the switch is clear (=0), the

normal TYPE1 (ego) MIL control strategy will be used.

+-------------+        power_up OR

|   State 0   |        reset_mil

|      NO     |<-----  -------------

| MALFUNCTIONS|        PxxxSTATE=0

+------+------+        PxxxMIL_ON=0

|               PxxxUPDATED=0

|               CL_MIL_ON=0

|

| malfunction &

| DEMAND_MODE=0 &

| PxxxUPDATED=0

| ----------------

| PxxxSTATE=1

| PxxxMIL_ON=1

| CL_MIL_ON=1

| store_code(Pxxx)

| CODES_COUNT=CODES_COUNT+1

|

+--------V--------+

|     State 1     |

| FAILED TO ENTER |

|   CLOSED LOOP   |

+-----------------+

DEFINITIONS

INPUTS

Registers:

-  CODES_COUNT = Count of total number of fault codes and pending codes (MIL

and NO_MIL) stored by the EEC.

Bit Flags:

-  DEMAND_MODE = Flag set when an on-demand self test mode is active.

-  PxxxFAULT = Fault flag for code Pxxx.  1 => a fault has been  stored  for

code Pxxx.

-  PxxxSTATE = MIL state flag for code Pxxx.

-  PxxxUPDATED = MIL state controller update status flag for code Pxxx; 1 ->

controller has been updated since power-up.

Calibration Constants:

23-100

OBDII DIAGNOSTIC EXECUTIVE, CLOSED LOOP ENTRY MIL CONTORL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  CL_OBDI_SW = Closed Loop Entry OBD-I MIL control select switch; 1 ->  use

OBD-I MIL control for closed loop entry.

23-101

OBDII DIAGNOSTIC EXECUTIVE, CLOSED LOOP ENTRY MIL CONTORL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OUTPUTS

Registers:

-  CODES_COUNT = See above.

Bit Flags:

-  CL_MIL_ON = MIL status flag for Closed Loop Entry; 1 -> MIL required  for

Closed Loop Entry fault.

-  PxxxMIL_ON = MIL flag for code Pxxx; 1 -> the MIL is required  to  be  on

for code Pxxx.

-  PxxxSTATE = See above.

-  PxxxUPDATED = See above.

23-102

OBDII DIAGNOSTIC EXECUTIVE, CLOSED LOOP ENTRY MIL CONTORL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: OBDII_CL_STATE_CONTROL_COM1

BEGIN: cl_malfunction(CL_MIL_ON, Pxxx)

;public process, executed when a malfunction(cl,Pxxx) call is made.

;This state machine will illuminate the MIL and store a code if the

;vehicle fails to enter closed loop within a manufacturers specified

;minimum time interval.

CL_OBDI_SW = 0 ----------------| Do: type1_malfunction(CL_MIL_ON, Pxxx)

;use OBDII MIL control.       |

|--- ELSE ---

DEMAND_MODE = 0 ---------|     |

|     |

PxxxUPDATED = 0 ---------|AND -| PxxxSTATE := 1

|     | PxxxMIL_ON := 1

PxxxFAULT = 0 -----------|     | PxxxUPDATED := 1

| Do: store_code(Pxxx)

| CODES_COUNT :=

|  min((CODES_COUNT+1), 255)

| CL_MIL_ON := 1

|

|--- ELSE ---

DEMAND_MODE = 0 ---------|     |

|AND -| PxxxSTATE := 1

PxxxUPDATED = 0 ---------|     | PxxxMIL_ON := 1

| PxxxUPDATED := 1

| Do: store_code(Pxxx)

| CL_MIL_ON := 1

END: cl_malfunction

BEGIN: cl_reset

;public process, executed on-demand to reset the MIL state machine

;related to time to enter closed loop fuel control.

For each Pxxx in the set cl_codes loop:

unconditionally ---------------| PxxxUPDATED := 0

| PxxxSTATE := 0

| PxxxMIL_ON := 0

End loop.

unconditionally ---------------| CL_MIL_ON := 0

END: cl_reset

23-103

OBDII DIAGNOSTIC EXECUTIVE, CLOSED LOOP ENTRY MIL CONTORL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: cl_init_mil(CL_MIL_ON)

;public process used to initialize the closed loop entry fault codes.

CL_OBDI_SW = 1 ----------------| Do: cl_reset

;OBD-I MIL control.           | Exit this process.

For each Pxxx in the set cl_codes loop:

PxxxSTATE = 2 -----------|

|AND -| PxxxSTATE := 0

PxxxFAULT = 0 -----------|     | CODES_COUNT :=

|  max((CODES_COUNT-1), 0)

|

| --- ELSE ---

|

PxxxSTATE = 2 -----------------| PxxxSTATE := 0

End loop.

END: cl_init_mil

BEGIN: cl_trip_complete(CL_MIL_ON)

;public process executed when commanded to after completion of a TRIP.

CL_OBDI_SW = 0 ----------------| Do: type1_trip_complete(cl, CL_MIL_ON)

;OBDII MIL control.

END: cl_trip_complete(CL_MIL_ON)

23-104

OBDII STRATEGY, MIL OUTPUT CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

23.1.10  MIL OUTPUT CONTROL (CDAM0)

OVERVIEW

+--------------------------------+

| OBDII_MIL_OUTPUT_COM1          |

+--------------------------------+

| PUBLIC PROCESSES:              |

| . pid_definitions              |

| . mil_output                   |

+--------------------------------+

The purpose of the Malfunction Indicator Light (MIL) is to alert  the  driver

that  the  computer has detected an OBDII malfunction with the EEC-IV system.

If the MIL were not present, the driver may  not  be  aware  that  a  problem

exists because The Failure Mode Effects Management (FMEM) strategy is capable

of maintaining good drive characteristics with a fault present.  However, the

vehicle  may  not  be operating at an optimum point with regard to emissions,

economy and performance.  When the MIL is  on,  the  driver  of  the  vehicle

should  seek  service  at  his  earliest convenience.  It is not necessary to

immediately shut the vehicle down and have it towed in for service.

The malfunction indicator light (MIL) warning  system  has  been  revised  to

comply with OBDII regulations for the 1994 and beyond model years.

The light, which is labeled  "Check  Engine"  or  "Service  Engine  Soon"  is

located  on the dashboard such that the driver can see it.  Power is supplied

to the light whenever the ignition switch is in the run  or  crank  position.

The ground circuit for the light is provided through the EEC module self test

output (STO).  Whenever the EEC-IV strategy determines that the light  should

be on, the STO output driver is turned on (STO voltage will be low).

The light will be turned on by the EEC module  whenever  an  OBDII  fault  is

detected for any of the monitored systems.

If the light is on due to an OBDII malfunction,  an  OBDII  fault  code  will

always  be present.  The light may also be on due to an intermittent short to

ground of the STO wire, or intermittent operation in HLOS.

The check engine light is turned on in the crank mode until a PIP  signal  is

detected  as  a bulb check.  The bulb check can be disabled by setting MILLIM

to zero.  If the light does not turn off while the  engine  is  cranking,  it

indicates that the EEC module is not receiving PIP signals.

The calibration parameter MIL_SW can take values of 0, 1 or 2.  If MIL_SW  is

set  to zero, the MIL light is not activated by the MIL logic.  Note that the

MIL light will still turn on whenever STO  is  grounded  (HLOS,  self  test).

Production  calibrations must have MIL_SW set to 1 to meet OBDII regulations.

When MIL_SW is set to 2, the MIL light will activate whenever any fault  code

is  present.   Note  that, when MIL_SW is set to 1, the light may be off even

though there are OBDII fault codes present.

There is also  a  calibration  parameter,  OBDII_MIL_SW,  which  is  used  to

configure  the  OBDII  systems with respect to control of the MIL light.  The

parameter is a mapped byte of calibration switches, one per OBDII  system  in

the  order from bit 0 to 8 {CAT_MIL_SW, CCM_MIL_SW, PURG_MIL_SW, SAIR_MIL_SW,

FUEL_MIL_SW,  EGO_MIL_SW,  EGR_MIL_SW,  MIS_MIL_SW}.   If  an  OBDII   system

23-105

OBDII STRATEGY, MIL OUTPUT CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

monitors corresponding bit is clear it will not control the mil.

23-106

OBDII STRATEGY, MIL OUTPUT CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  CODES_COUNT = Count of total number of fault codes and pending codes (MIL

and NO_MIL) stored by the EEC.

-  READY_FLAGS = Bitmapped register containing OBDII ready flags and  MIL_ON

flag.

-  TYPE_A_MIS = Indicates to executive which misfire criteria.

Bit Flags:

-  CAT_MIL_ON = MIL status flag for Catalyst system; 1 -> MIL  required  for

catalyst fault.

-  CCM_MIL_ON = MIL status flag  for  Comprehensive  Components;  1  ->  MIL

required for CCM fault.

-  COOL_MIL_ON = MIL status flag for fail safe cooling; 1  ->  MIL  required

for fail safe cooling fault.

-  CL_MIL_ON = MIL status flag for Closed Loop Entry; 1 -> MIL required  for

Closed Loop Entry fault).

-  CRKFLG = Flag indicating status of CRANK MODE; 1 -> in CRANK MODE,  0  ->

not in CRANK MODE.

-  EGO_MIL_ON = MIL status flag for EGO; 1 -> MIL required for HEGO fault.

-  EGR_MIL_ON = MIL status flag for EGR; 1 -> MIL required for EGR fault.

-  FIRST_PIP = Flag set to 1 when the first PIP is detected.  Reset to  zero

on power-up or stall.

-  FUEL_MIL_ON = MIL status flag for Fuel System; 1 -> MIL required for Fuel

System fault.

-  MIL_ON = OBDII MIL status flag (bit 7 of READY_FLAGS); 1 -> MIL is on.

-  MIS_FCO_FLG = Flag indicating FMEM action is being taken  during  type  A

misfire.

-  MISFIRING = Misfire status flag; 1 -> misfire currently occurring.

-  MIS_MIL_ON = MIL status flag for Misfire; 1 -> MIL required  for  Misfire

fault.

-  PURG_MIL_ON = MIL status flag Purge; 1 -> MIL required for Purge fault.

-  PxxxFAULT = Fault flag for code Pxxx; 1 -> a fault has  been  stored  for

code Pxxx.

-  SAIR_MIL_ON = MIL status flag for Secondary Air; 1 ->  MIL  required  for

Secondary Air fault.

23-107

OBDII STRATEGY, MIL OUTPUT CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  CCM_MIL_SW = CCM MIL enable switch; 1 -> connect CCM test to the MIL.

-  CL_MIL_SW = Closed Loop Entry MIL enable switch; 1 -> connect the  Closed

loop entry test to the MIL.

-  COOL_MIL_SW = fail safe cooling MIL enable switch; 1 -> connect fail safe

cooling test to the MIL.

-  EGO_MIL_SW = EGO MIL enable switch; 1 -> connect the EGO test to the MIL.

-  EGR_MIL_SW = EGR MIL enable switch; 1 -> connect the EGR test to the MIL.

-  FUEL_MIL_SW = FUEL MIL enable switch; 1 -> connect the FUEL test  to  the

MIL.

-  MILLIM = Software switch to enable/disable bulb  check,  unitless;  1  ->

enable, 0 -> disable.

-  MIL_SW = MIL enable switch, unitless;

0 -> Do not turn on MIL,

1 -> Do MIL logic to meet OBDII regulation, must be 1 for

production,

2 -> Turn MIL light on for any continuous fault, development

calibration only.

-  MIS_MIL_SW = MIS MIL enable switch; 1 -> connect the MIS test to the MIL.

-  PURG_MIL_SW = PURG MIL enable switch; 1 -> connect the PURG test  to  the

MIL.

-  SAIR_MIL_SW = SAIR MIL enable switch; 1 -> connect the SAIR test  to  the

MIL.

OTHER

-  P1000 = Additional mixed city and highway driving  required  to  complete

OBD-II system check-out.

CALIBRATION INFORMATION

MIL_SW must be set to 1 for production calibration.

23-108

OBDII STRATEGY, MIL OUTPUT CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: OBDII_MIL_OUTPUT_COM1

BEGIN: pid_definitions

;Define J1979 PIDs related to the fault codes.

MIL_SW = 1 ------------------|

;production setting         |AND -|

|     |

MIL_ON = 1 ------------------|     |

;MIL required for OBDII           |

|OR --| mil_state := 1

MIL_SW = 2 ------------------|     |     |

;on with fault present      |     |     |

|AND -|     |

CODES_COUNT > 1 -------|     |           |

|OR --|           |

CODES_COUNT = 1 -|     |                 |

|AND -|                 |

P1000FAULT = 0 --|                       |

| --- ELSE ---

|

| mil_state := 0

pid_def(j1979_01_011, b0-b6: (b0-b6 of NUM_CODES),

b7: mil_state)

END: pid_definitions

23-109

OBDII STRATEGY, MIL OUTPUT CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN:  mil_output

;executed each background loop

;Update MIL_ON flag:

CAT_MIL_ON = 1 --------|

;catalyst             |AND -|

|     |

CAT_MIL_SW = 1 --------|     |

|

CCM_MIL_ON = 1 --------|     |

;comprehensive comp.  |AND -|

|     |

CCM_MIL_SW = 1 --------|     |

|

COOL_MIL_ON = 1 -------|     |

;fail safe cooling    |AND -|

|     |

COOL_MIL_SW = 1 -------|     |

|

PURG_MIL_ON = 1 -------|     |

;purge                |AND -|

|     |

PURG_MIL_SW = 1 -------|     |

|

SAIR_MIL_ON = 1 -------|     |

;secondary air        |AND -|

|     |

SAIR_MIL_SW = 1 -------|     |OR --| new_mil_on = 1

|     |  ;mil required for OBDII

EGO_MIL_ON = 1 --------|     |     |

;hego                 |AND -|     |

|     |     |

EGO_MIL_SW = 1 --------|     |     |

|     |

EGR_MIL_ON = 1 --------|     |     |

;egr                  |AND -|     |

|     |     |

EGR_MIL_SW = 1 --------|     |     |

|     |

MIS_MIL_ON = 1 --------|     |     |

;misfire              |AND -|     |

|     |     |

MIS_MIL_SW = 1 --------|     |     |

|     |

FUEL_MIL_ON = 1 -------|     |     |

;fuel system          |AND -|     |

|     |     |

FUEL_MIL_SW = 1 -------|     |     |

|     |

CL_MIL_ON = 1 ---------|     |     |

;cl system            |AND -|     |

|           |

CL_MIL_SW = 1 ---------|           |

| --- ELSE ---

|

| new_mil_on = 0

|  ;mil not required for OBDII

23-110

OBDII STRATEGY, MIL OUTPUT CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

MIL_ON = 1 ------------|

|AND -| Do: mil_off_verification(new_mil_on)

new_mil_on = 0 --------|     |   ;this will overwrite new_mil_on if

|   ;if the MIL should stay on.

| MIL_ON := new_mil_on

|

| --- ELSE ---

MIL_ON = 0 ------------|     |

|AND -| Do: mil_on_verification(new_mil_on)

new_mil_on = 1 --------|     |   ;this will overwrite new_mil_on if

|   ;if the MIL should stay off. .

| MIL_ON := new_mil_on

|

| --- ELSE ---

|

| MIL_ON := new_mil_on

23-111

OBDII STRATEGY, MIL OUTPUT CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;Update bulb_check flag:

CRKFLG = 1 ------------------|

;crank mode                 |

|

FIRST_PIP = 0 ---------------|AND -| bulb_check := 1

;no PIPS yet                |     |  ;do check

|     |

MILLIM = 1 ------------------|     |

;bulb check enabled               |

| --- ELSE ---

|

| bulb_check := 0

|  ;don't do check

;Control the light:

MIS_MIL_SW = 1 --------------------|

|

MIS_FCO_FLG = 0 -------------|     |

;not doing fuel cutout      |OR --|

|     |

TYPEA_FL_MIL = 1 ------------|     |

|AND -| flash mil

MISFIRING = 1 ---------------------|     |  ;freq = 1 Hz +/-10%

;misfire occurring                |     |  ;duty cycle = 50% +/-10%

|     |

TYPE_A_MIS = 1 --------------------|     |

| --- ELSE ---

bulb_check = 1 --------------------|     |

;bulb check requested             |     |

|     |

MIL_SW = 1 ------------------|     |     |

;production setting         |AND -|     |

|     |OR --| turn on mil

MIL_ON = 1 ------------------|     |     |

;MIL required for OBDII           |     |

|     |

MIL_SW = 2 ------------------|     |     |

;on with fault present      |     |     |

|AND -|     |

CODES_COUNT > 1 -------|     |           |

|OR --|           |

CODES_COUNT = 1 -|     |                 |

|AND -|                 |

P1000FAULT = 0 --|                       |

| --- ELSE ---

|

| turn mil off

END: mil_output

23-112

OBDII STRATEGY, KAM VERIFICATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

23.1.11  KAM VERIFICATION (CDAM0)

OVERVIEW

+--------------------------------+

| OBDII_KAM_VERIFICATION_COM1    |

+--------------------------------+

| PUBLIC PROCESSES:              |

| . mil_on_verification          |

| . mil_off_verification         |

| . power_up_verification        |

+--------------------------------+

This module is used to verify the fault code data in the OBD-II  system.   It

has  been  discovered  that KAM can be corrupted due to flaws in the hardware

design of the RAM/CART/EBC chip.  If a write is in progress and the module is

powered  down,  the  contents  of  the  memory location being written to will

likely be corrupted.

Given this fact, the following comprehensive verification of  the  data  that

illuminates  the  Malfunction  Indicator Lamp (MIL) has been designed.  These

routine will be invoked when a change in the state of the  MIL  is  required.

The  underlying fault code data will be examined for rationality.  If a fault

is discovered, the corrupted location will be reset and the code  P1000  will

be  stored,  requiring  a full check of the OBD-II diagnostic system prior to

passing the Inspection Maintenance test.

A test is also performed once per power-up to verify all fault code data  and

reset  the  corrupted  locations,  store the P1000 and reset the code couting

variables, NUM_CODES and CODES_COUNT.

DEFINITIONS

Bit Flags:

-  CODES_COUNT = Count of total number of fault codes and pending codes (MIL

and NO_MIL) stored by the EEC.

-  NUM_CODES = Number of OBD_II fault codes currently stored in memory.

-  PxxxCNT = Warm-up counter for code Pxxx.

-  PxxxFAULT = Fault flag for code Pxxx.  1 => a fault has been  stored  for

code Pxxx.

-  PxxxFAULT_A = Fault flag for code Pxxx.  1 => a fault has been stored for

code Pxxx.

-  PxxxMIL_ON = MIL flag for code Pxxx; 1 -> the MIL is required  to  be  on

for code Pxxx.

-  PxxxSTATE = MIL state flag for code Pxxx.

-  PxxxUPDATED = MIL state controller update status flag for code Pxx; 1  ->

controller has been updated since power-up.

23-113

OBDII STRATEGY, KAM VERIFICATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  CL_OBDI_SW = Closed loop Entry OBDI MIL control select switch; 1  ->  use

OBDI MIL control for closed loop entry.

PROCESS

STRATEGY MODULE: OBDII_KAM_VERIFICATION_COM1

BEGIN: mil_on_verification(new_mil_on)

;The following process is executed when the strategy is about to command

;the MIL to turn on.  The KAM fault code data will be examined to ensure

;a valid reason exists to turn on the MIL.

unconditionally ----------------| valid_mil := false

For sys := (purg,sair,ego,egr,ccm,cat,cl,mis,fuel,cool) perform the

following:

unconditionally ----------------| Do: mil_integrity_check(sys)

|  ;this will set valid_mil := true if

|  ;data to support the decision to

|  ;illuminate the MIL is found.

End For loop.

valid_mil = false --------------| Do: power_up_verification

|  ;this will reset the corrupted locations.

unconditionally ----------------| new_mil_on := valid_mil

END: mil_on_verification

BEGIN: mil_off_verification(new_mil_on)

;The following process is executed when the strategy is about to command

;the MIL to turn off.  The KAM fault code data will be examined to ensure

;a valid reason exists to turn off the MIL.

unconditionally ----------------| valid_mil := false

For sys := (purg,sair,ego,egr,ccm,cat,cl,mis,fuel,cool) perform the

following:

unconditionally ----------------| Do: mil_integrity_check(sys)

|  ;this will set valid_mil := true if

|  ;data to support the decision to

|  ;illuminate the MIL is found.

End For loop.

23-114

OBDII STRATEGY, KAM VERIFICATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

valid_mil = true ---------------| Do: power_up_verification

|  ;this will reset the corrupted locations.

unconditionally ----------------| new_mil_on := valid_mil

END: mil_off_verification

23-115

OBDII STRATEGY, KAM VERIFICATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: mil_integrity_check(sys)

;This process will examine the PxxxMIL_ON and PxxxFAULT flags and

;the PxxxSTATE register in the set of sys_codes.  At least one

;PxxxMIL_ON should be set and the corresponding PxxxFAULT and PxxxFAULT_A

;should also be set and the PxxxSTATE should be in a state that says the

;MIL is about to be turned on or has been on for some time.

For each Pxxx in the set of sys_codes perform the following:

sys_MIL_SW = 1 --------------------------|

|

Pxxx is in set of mil_codes -------------|

|

PxxxMIL_ON = 1 --------------------------|

|

PxxxFAULT = 1 ---------------------------|

|

PxxxFAULT_A = 1 -------------------------|

|

PxxxCNT < 40 ----------------------------|

|

PxxxSTATE = 3 ---------|                 |

|                 |

PxxxSTATE = 4 ---------|OR --|           |AND -| valid_mil := true

|     |           |

PxxxSTATE = 5 ---------|     |AND -|     |

|     |     |

sys <> cl -------------|     |     |     |

|OR --|     |     |

sys = cl --------|     |           |     |

|AND -|           |     |

CL_OBDI_SW = 0 --|                 |     |

|     |

PxxxSTATE = 2 ---------------|     |OR --|

|AND -|

sys = fuel ------------|     |     |

|OR --|     |

sys= mis --------------|           |

|

PxxxSTATE = 1 ---------------|     |

|     |

sys = cl --------------------|AND -|

|

CL_OBDI_SW = 1 --------------|

END: mil_integrity_check(sys)

23-116

OBDII STRATEGY, KAM VERIFICATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: power_up_verification

;The following process is executed when the strategy calls it and during

;power-up initialization.  Thee KAM fault code data will be examined to

;ensure consistent data.  If corrupted data is found, it will be reset to

;a erased code state and a non-mil code will be set indicating KAM

;corruption has occured.

For each Pxxx in the set of non_mil_codes perform the following:

PxxxFAULT = 1 -------|

|AND -|

PxxxCNT = 40 --------|     |

|OR --| Do: erase_code(Pxxx)

PxxxCNT > 40 --------------|     | Do: reset_trips

|

PxxxFAULT <> PxxxFAULT_A --|

For each Pxxx in the set of mil_codes perform the following:

PxxxSTATE > 5 --------------------------------|

|

PxxxCNT > 40 ---------------------------------|

|

PxxxFAULT <> PxxxFAULT_A ---------------------|

|

PxxxFAULT = 0 --------------------------|     |

|AND -|

PxxxMIL_ON = 1 -------------------------|     |

|

PxxxMIL_ON = 1 -------------------------|     |

|     |

PxxxSTATE = 0 --------------------------|     |

|AND -|

Pxxx is in set of cl_codes -------------|     |

|     |

CL_ODBI_SW = 1 -------------------------|     |

|

PxxxMIL_ON = 1 -------------------------|     |

|     |

PxxxSTATE <= 2 -------------------------|     |

|AND -|

Pxxx is in set of type1_codes ----|     |     |OR --| PxxxMIL_ON := 0

|OR --|     |     | PxxxSTATE := 0

Pxxx is in set of cl_codes -|     |           |     | PxxxMALF := 0

|AND -|           |     | PxxxUPDATED := 0

CL_OBDI_SW = 0 -------------|                 |     | Do: erase_code(Pxxx)

|     | Do: reset_trips

PxxxMIL_ON = 1 -------------------------|     |

|     |

PxxxSTATE <= 1 -------------------------|     |

|AND -|

Pxxx is in set                          |     |

of mis_codes ---------------------|     |     |

|OR --|     |

Pxxx is in set                    |           |

of fuel_codes --------------------|           |

(continued on next page)

23-117

OBDII STRATEGY, KAM VERIFICATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|

PxxxFAULT = 1 --------------------|           |

|OR --|     |

PxxxMIL_ON = 1 -------------------|     |AND -|

|     |

PxxxCNT = 40 ---------------------------|     |

|

PxxxCNT >  1 ---------------------|           |

|OR --|     |

PxxxMIL_ON = 0 -------------------|     |     |

|     |

PxxxSTATE > 2 --------------|           |     |

|           |     |

Pxxx is NOT in set          |AND -|     |     |

of cl_codes ----------|     |     |     |     |

|     |     |     |AND -|

Pxxx is in            |OR --|     |     |

set of cl_codes |     |           |     |

|AND -|           |     |

CL_OBDI_SW = 0 -|                 |     |

|     |

PxxxSTATE = 2 --------------|     |     |

|     |     |

Pxxx is in set              |AND -|OR --|

of fuel_codes --------|     |     |

|OR --|     |

Pxxx is in set        |           |

of mis_codes ---------|           |

|

PxxxSTATE = 1 --------------|     |

|     |

Pxxx is in set of           |AND -|

cl_codes -------------------|

|

CL_OBDI_SW = 1 -------------|

End For loop.

23-118

OBDII STRATEGY, KAM VERIFICATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;Next, verify the sys_MIL_ON flags assuming all fault code data is correct.

For sys := (purg,sair,ego,egr,ccm,cat,cl,mis,fuel,cool) perform the

following:

unconditionally ----------| valid_mil := false

For each Pxxx in the set of sys_codes perform the following:

Pxxx is in the set

of mil_codes --------|

|AND -| valid_mil := true

PxxxMIL_ON = 1 ------|

END For loop.

valid_mil <> sys_MIL_ON --| sys_MIL_ON := valid_mil

| Do: reset_trips

END For loop.

23-119

OBDII STRATEGY, KAM VERIFICATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;Finally, verify NUM_CODES and CODES_COUNT assuming all the fault code

;data is correct.

unconditionally ----------| new_num_codes := 0

| new_codes_count := 0

For each Pxxx in the set of mil_codes perform the following:

PxxxFAULT = 1 ------------| new_num_codes := min((new_num_codes+1),255)

| new_codes_count := min((new_codes_count+1),255)

|

| --- ELSE ---

|

PxxxSTATE > 0 ------------| new_codes_count := min((new_codes_count+1),255)

END For loop.

For each Pxxx in the set of non_mil_codes perform the following:

PxxxFAULT = 1 ------------| new_codes_count := min((new_codes_count+1,255)

END For loop.

unconditionally ------------------| reset_trips_temp := false

new_codes_count <> CODES_COUNT ---| CODES_COUNT := new_codes_count

| reset_trips_temp := true

new_num_codes <> NUM_CODES -------| NUM_CODES := new_num_codes

| reset_trips_temp := true

reset_trips_temp = true ----------| Do: reset_trips

END: power_up_verification

23-120

OBDII STRATEGY, FREEZE FRAME - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

23.1.12  FREEZE FRAME (CDAM0)

OVERVIEW

This module is used to store and clear the freeze frame conditions (N,  LOAD,

ECT,  VSBAR,  OLFLG,  LAMBSE1, LAMBSE2, KAMRF1, KAMRF2) as required by OBD-II

regulations.

There are two seperate entry points contained in the module.  One is used  to

store  the  freeze  frame  conditions,  the other is used to clear the freeze

frame conditions.

There is ONE data structure in memory that holds the freeze frame conditions.

The  first  time  a  malfunction  occurs  for  any MIL code, the freeze frame

conditions are stored.   If  the  malfunction  was  not  a  FUEL  or  MISFIRE

malfunction,  the  freeze frame conditions can be overwritten by a subsequent

FUEL or MISFIRE malfunction.  FUEL and MISFIRE malfunctions can not overwrite

one another.

The freeze frame conditions are cleared on OBD-II reset or when the fault for

which they are stored is erased.

The freeze frame data structure is a record as shown below:

freeze_frame RECORD

FF_CODE      = [KAM] Code number of

fault for which freeze frame

conditions are stored.

FF_N         = [KAM] Engine speed when

fault occured.

FF_LOAD      = [KAM] Load when fault

occured.

FF_ECT       = [KAM] Engine coolant

temperature when fault occured.

FF_VSBAR     = [KAM] Filtered vehicle

speed when fault occured.

FF_OL_HOLD   = [KAM]  - Not yet met conditions to go

closed loop open loop flag when

fault occured.

FF_OL_FDBACK = [KAM] Closed loop feedback fuel

control flag when fault occured.

FF_OL_DM     = [KAM] Open loop due to driving

conditions flag when fault occured.

FF_OL_FMEM   = [KAM] Open loop due to detected

system fault flag when fault occured.

FF_OL_FAULT  = [KAM] Closed loop but fault with at

least one oxygen sensor flag when

fault occured.

FF_LAMBSE1   = [KAM] LAMBSE1 - short

term fuel trim when fault occured.

FF_LAMBSE2   = [KAM] LAMBSE2 - short

term fuel trim when fault occured.

FF_KAMREF1   = [KAM] KAMRF1 - adaptive

correction when fault occured.

FF_KAMREF2   = [KAM] KAMRF2 - adaptive

correction when fault occured.

23-121

OBDII STRATEGY, FREEZE FRAME - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

END.

23-122

OBDII STRATEGY, FREEZE FRAME - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  ECT = Engine coolant temperature, degrees F.

-  ECT_ENG = Engine coolant temperature without any FMEM modifications.

-  KAMRF1 = Adaptive fuel correction for bank 1.

-  KAMRF2 = Adaptive fuel correction for bank 2.

-  LAMBSE1 = Lambda equivalence ratio for bank 1.

-  LAMBSE2 = Lambda equivalence ratio for bank 2.

-  LOAD = Universal load as ratio of air charge over standard.

-  N = Engine RPM.

-  OL_DRIVE = Open  loop  due  to  driving  conditions  (not  FMEM  or  HOLD

related).

-  LOAD_SAE = Engine load as required by SAE standard J1979.

-  VSBAR = Filtered vehicle speed.

Bit Flags:

-  HEGO_FAULT = Closed loop but fault with at least one oxygen  sensor  flag

when fault occured.

-  HEGO_FDBACK = Closed loop feedback fuel control flag when fault occured.

-  OLFLG = Open loop flag.

-  OL_FMEM = Open loop due to detected system fault flag, 1 -> open loop due

to FMEM.

-  OL_HOLD = Percent conditions not satisfied to go close loop  flag,  1  ->

hold in open loop.

23-123

OBDII STRATEGY, FREEZE FRAME - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OUTPUTS

Registers:

-  FF_CODE = Code number of fault for  which  freeze  frame  conditions  are

stored.

-  FF_N = Value of Engine speed at the time the freeze frame was stored.

-  FF_LOAD = Value of Load at the time the freeze frame was stored.

-  FF_ECT = Value of Engine coolant temperature at the time the freeze frame

was stored.

-  FF_VSBAR = Value of Filtered vehicle speed at the time the  freeze  frame

was stored.

-  FF_LAMBSE1 = Value of LAMBSE1 (short term fuel  trim)  at  the  time  the

freeze frame was stored.

-  FF_LAMBSE2 = Value of LAMBSE2 (short term fuel  trim)  at  the  time  the

freeze frame was stored.

-  FF_KAMREF1 = Value of KAMRF1 (adaptive correction) at the time the freeze

frame was stored.

-  FF_KAMREF2 = Value of KAMRF2 (adaptive correction) at the time the freeze

frame was stored.

Bit Flags:

-  FF_FUEL_MIS = Freeze frame FUEL or MISFIRE data stored flag.  1 -> freeze

frame was stored due to Fuel or Misfire malfunction.

-  FF_OL_FAULT = Closed loop but fault with at least one oxygen sensor  flag

when fault occured.

-  FF_OL_FDBACK = Closed loop feedback fuel control flag when fault occured.

-  FF_OL_FMEM = Open loop due to  detected  system  fault  flag  when  fault

occured.

-  FF_OL_HOLD = Not yet met conditions to go closed loop open loop flag when

fault occured.

-  FF_OL_DM = Open loop due to driving conditions flag when fault occured.

23-124

OBDII STRATEGY, FREEZE FRAME - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: OBDII_FREEZE_FRAME_COM1

BEGIN: pid_definitions

;This process is used to define the PIDs related to the FREEZE

;FRAME as required by SAE standard J1979.

pid_def(j1979_02_00, PIDMAP_1,PIDMAP_2)

pid_def(j1979_02_02, FF_CODE)

pid_def(j1979_02_031, b0:FF_OL_HOLD,

b1:FF_OL_FDBACK,

b2:FF_OL_DM,

b3:FF_OL_FMEM,

b4:FF_OL_FAULT

b5:0,

b6:0,

b7:0)

pid_def(j1979_02_04, (FF_LOAD * 255))

pid_def(j1979_02_05, (((FF_ECT - 32) * 5/9) + 40))

pid_def(j1979_02_06, (128 / FF_LAMBSE1))

pid_def(j1979_02_07, (128 * FF_KAMREF1))

pid_def(j1979_02_08, (128 / FF_LAMBSE2))

pid_def(j1979_02_09, (128 * FF_KAMREF2))

pid_def(j1979_02_0C, FF_N)

pid_def(j1979_02_0D, (FF_VSBAR * 1.609))

END: pid_definitions

23-125

OBDII STRATEGY, FREEZE FRAME - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: store_ff(Pxxx)

;This procedure called by the MIL controller to store the

;freeze frame conditions.  Pxxx is the code which causes the

;freeze frame storage.

unconditionally ------------------| FF_CODE := xxx

|  ;BCD representation of code

|  ;number.

| FF_N := N

| FF_LOAD := LOAD_SAE

| FF_ECT := ECT_ENG

| FF_VSBAR := VSBAR

| FF_OL_HOLD := OL_HOLD

| FF_OL_FDBACK := HEGO_FDBACK

| FF_OL_DM := OL_DRIVE

| FF_OL_FMEM := OL_FMEM

| FF_OL_FAULT := HEGO_FAULT

| FF_LAMBSE1 := LAMBSE1

| FF_LAMBSE2 := LAMBSE2

| FF_KAMREF1 := KAMRF1

| FF_KAMREF2 := KAMRF2

END: store_ff(Pxxx)

23-126

OBDII STRATEGY, FREEZE FRAME - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: clear_ff

;this procedure called by the MIL controller to clear the

;freeze frame conditions.

unconditionally ------------------| FF_CODE := 0

|  ;BCD nil code.

| FF_N := 0

| FF_LOAD := 0

| FF_ECT := 0

| FF_VSBAR := 0

| FF_OL_HOLD := 0

| FF_OL_FDBACK := 0

| FF_OL_DM := 0

| FF_OL_FMEM := 0

| FF_OL_FAULT := 0

| FF_LAMBSE1 := 1.0

| FF_LAMBSE2 := 1.0

| FF_KAMREF1 := 1.0

| FF_KAMREF2 := 1.0

| FF_FUEL_MIS := 0

END: clear_ff

BEGIN: swap_ff_code(code_number)

;This process is called from the misfire state controller to change the

;code number of the freeze frame when unabled to detect which cylinder

;is misfiring because of loss of sync.

unconditionally ------------------| FF_CODE := code_number

END: swap_ff_code(code_number)

23-127

OBD-II DIAGNOSTIC EXECUTIVE, SIMILAR CONDITIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

23.1.13  SIMILAR CONDITIONS (CDAK0)

OVERVIEW

+--------------------------------+

| OBDII_SIMILAR_CONDS_COM1       |

+--------------------------------+

| PUBLIC PROCESSES:              |

| . get_similar_conds(Pxxx)      |

| . clear_similar_conds(Pxxx)    |

| . check_fuel_conds(Pxxx)       |

| . check_mis_conds              |

+--------------------------------+

This module is used within the OBD-II diagnostic executive to manipulate  the

similar conditions data structure.  Similar conditions records are maintained

for each fault code associated with the FUEL and MISFIRE monitors.

The similar  conditions  and  related  data  are  stored  in  a  record  data

structure.   There  is  one  record  for  each Pxxx in the sets mis_codes and

fuel_codes.

The record holds the following information:

similar_cond RECORD

PxxxACTIVE  = [KAM] Flag.  1 -> conditions active.

PxxxSEEN    = [KAM] Flag.  1 -> conditions seen.

PxxxTRIP_FG = [KAM] Flag.  1 -> trip completed.

PxxxTRIP_CT = [KAM] Trips completed (0 - 80).

PxxxN       = [KAM] Stored engine RPM.

PxxxLOAD    = [KAM] Stored PCT_LOAD.

PxxxSTATUS  = [KAM] Flag.  1 -> warm engine.

END.

23-128

OBD-II DIAGNOSTIC EXECUTIVE, SIMILAR CONDITIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  ECT = Engine Coolant Temperature, degrees F.

-  N = Engine speed, RPM.

-  PCT_LOAD = Percent inferred load (altitude independent).

Bit Flags:

-  PxxxLOAD = Similar conditions engine load.

-  PxxxMALF = Malfunction flag for code Pxxx; 1 -> a  malfunction  currently

exists for fault Pxxx.

-  PxxxN = Similar conditions engine speed.

-  PxxxSTATUS =  Similar  conditions  warm-up  status  flag;  1  ->  warm-up

completed.

Calibration Constants:

-  ECT_WARM = Temperature above which the engine is considered warm for  the

purposes of OBDII similar conditions.

OUTPUTS

Bit Flags:

-  PxxxACTIVE = Similar conditions active flag; 1 -> conditons active.

-  PxxxLOAD = See above.

-  PxxxN = See above.

-  PxxxSEEN = Similar conditions seen flag; 1 -> conditions seen.

-  PxxxSTATUS = See above.

-  PxxxTRIP_CT = Similar conditions trip counter (0 - 80).

-  PxxxTRIP_FG = Similar conditions trip flag; 1 -> trip occured.

23-129

OBD-II DIAGNOSTIC EXECUTIVE, SIMILAR CONDITIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: OBDII_SIMILAR_CONDS_COM1

BEGIN: get_similar_conds(Pxxx)

;This procedure is used to save the similar conditions information for a

;particular code .  It is called by the MIL controller.

unconditionally ---------------| PxxxACTIVE := 0

| PxxxSEEN := 0

| PxxxTRIP_FG := 0

| PxxxTRIP_CT := 0

| PxxxN := N

| PxxxLOAD := PCT_LOAD

ECT > ECT_WARM ----------------| PxxxSTATUS := 1

|  ;warm engine.

|

| --- ELSE ---

|

| PxxxSTATUS := 0

|  ;cold engine.

END: get_similar_conds(Pxxx)

BEGIN: clear_similar_conds(Pxxx)

;This procedure is used to clear the similar conditions information for a

;particular code.  It is called by the MIL controller.

unconditionally ---------------| PxxxACTIVE := 0

| PxxxSEEN := 0

| PxxxTRIP_FG := 0

| PxxxTRIP_CT := 0

| PxxxN := 0

| PxxxLOAD := 0

| PxxxSTATUS := 0

END: clear_similar_conds(Pxxx)

23-130

OBD-II DIAGNOSTIC EXECUTIVE, SIMILAR CONDITIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: check_fuel_conds(Pxxx)

;This process is called from within the fuel monitor to determine if the

;similar conditons have been seen yet this driving cycle.

ECT > ECT_WARM -------------------------| status := 1

|

| --- ELSE ---

|

| status := 0

abs(N - PxxxN) <= 375 ------------|

|

abs(PCT_LOAD - PxxxLOAD) <= 0.10 -|AND -| PxxxSEEN := 1

|     | Do: fuel_conds_seen(Pxxx)

PxxxSTATUS = status --------------|

END: check_fuel_conds

BEGIN: check_mis_conds

;This process is called from within the misfire monitor when not

;misfiring to determine if similar conditions have been met.

ECT > ECT_WARM -------------------------| status := 1

|

| --- ELSE ---

|

| status := 0

For each Pxxx in the set mis_codes Do:

abs(N - PxxxN) <= 375 ------------|

|

abs(PCT_LOAD - PxxxLOAD) <= 0.10 -|AND -| PxxxSEEN := 1

|     | Do: mis_conds_seen(Pxxx)

PxxxSTATUS = status --------------|

END loop.

END: check_mis_conds

23-131

OBDII DIAGNOSTIC EXECUTIVE, DRIVING CYCLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

23.1.14  DRIVING CYCLE (CDAF0)

OVERVIEW

+--------------------------------+

| OBDII_DRIVE_CYCLE_COM1         |

+--------------------------------+

| PUBLIC PROCESSES:              |

| . bg_check_for_closed_loop     |

| . cl_seen_init                 |

| . reset_drive_cycle            |

+--------------------------------+

This module is used  to  determine  if  an  OBD-II  driving  cycle  has  been

performed.  This is defined by CARB as:

Engine startup,  vehicle  operation  beyond  the  beginning  of  closed  loop

operation, and engine shut off.

This module will maintain a flag in KAM called  CL_SEEN.   When  closed  loop

fuel  control  is entered this flag will get set.  A utility will be provided

to clear the CL_SEEN after the information stored in it has been used  during

RAM initilization.

Additionally a utility is provided to take KAM reset actions on  the  CL_SEEN

flag.

DEFINITIONS

INPUTS

Registers:

-  ATMR3 = Time in RUN mode, sec.

-  DRIVE_COUNT = Count of number of OBD-II drive cycles completed.

Bit Flags:

-  CL_SEEN = Flag indicating if closed-loop fuel  operation  has  been  seen

this driving cycle.

-  OLFLG = Open loop flag.

23-132

OBDII DIAGNOSTIC EXECUTIVE, DRIVING CYCLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OUTPUTS

Registers:

-  DRIVE_COUNT = See above.

Bit Flags:

-  CL_SEEN = See above.

PROCESS

STRATEGY MODULE: OBDII_DRIVE_CYCLE_COM1

BEGIN: bg_check_for_closed_loop

;public process, executed once per background loop to determine if

;closed loop fuel control has been entered.

CL_SEEN = 0 ---------------------|

|

OLFLG = 0 -----------------------|AND -| CL_SEEN := 1

|     | DRIVE_COUNT := DRIVE_COUNT + 1

ATMR3 > 4 -----------------------|

END: bg_check_for_closed_loop

BEGIN: cl_seen_init

;public process executed to initialize the CL_SEEN flag.

unconditionally -----------------------| CL_SEEN := 0

END: cl_seen_init

BEGIN: reset_drive_cycle

;public process used to reset the CL_SEEN flag and the DRIVE_COUNT counter.

unconditionally -----------------------| CL_SEEN := 0

| DRIVE_COUNT := 0

END: reset_drive_cycle

23-133

OBDII STRATEGY, J1979 PIDS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

23.1.15  J1979 PIDS (CDAM0)

OVERVIEW

+--------------------------------+

| OBDII_J1979_PIDS_COM1          |

+--------------------------------+

| PUBLIC PROCESSES:              |

| . pid_definitions              |

+--------------------------------+

This module is used to  define  parameter  identification  codes  numbers  as

required by SAE standard J1979.  These PIDs are to be accessed by the generic

scan tool as set forth in the CARB regulations.  These PIDs are grouped  here

in  this module as there is no other spot in the strategy where it would make

sense to do so.

DEFINITIONS

INPUTS

Bit Flags:

-  CAT_READY = Catalyst OBDII ready  flag  (bit  0  of  READY_FLAGS);  1  ->

catalyst NOT monitored since kam reset.

-  EGO_READY = EGO OBDII ready flag (bit 5 of READY_FLAGS); 1 ->  HEGOs  NOT

monitored since kam reset.

-  EGR_READY = EGR OBDII ready flag (bit 6 of READY_FLAGS);  1  ->  EGR  NOT

monitored since kam reset.

-  PURG_READY = Purge OBDII ready flag (bit 2 of READY_FLAGS);  1  ->  Purge

NOT monitored since kam reset.

-  SAIR_READY = Secondary Air OBDII ready flag (bit 3 of READY_FLAGS); 1  ->

Secondary Air NOT monitored since kam reset.

Calibration Constants:

-  CAT_TST_SW = CAT test present claibration switch; 1 -> CAT test present.

-  CCM_TST_SW = CCM test present claibration switch; 1 -> CCM test present.

-  EGO_TST_SW = EGO test present claibration switch; 1 -> EGO test present.

-  EGR_TST_SW = EGR test present claibration switch; 1 -> EGR test present.

-  FUEL_TST_SW = Fuel test  present  claibration  switch;  1  ->  fuel  test

present.

-  J1979_01_1C = OBD requirements to which vehicle is designed, where:

01 - OBD II (California ARB)

02 - OBD (Federal EPA)

03 - OBD and OBD II

04 - OBD I

23-134

OBDII STRATEGY, J1979 PIDS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

05 - Not intended to meet any OBD requirements

-  MIS_TST_SW = MIS test present claibration switch; 1 -> MIS test present.

-  PURG_TST_SW = PURG test  present  claibration  switch;  1  ->  PURG  test

present.

-  SAIR_TST_SW = SAIR test  present  claibration  switch;  1  ->  SAIR  test

present.

PROCESS

STRATEGY MODULE: OBDII_J1979_PIDS_COM1

BEGIN: pid_definitions

;Define J1979 PIDs related to the OBD-II executive, not specifically

;associated with any portion of the Diagnostic Executive.

pid_def(j1979_01_00, PIDMAP_3,PIDMAP_4)

pid_def(j1979_01_012, b0: MIS_TST_SW,

b1: FUEL_TST_SW,

b2: CCM_TST_SW,

b3: 0,

b4: 0,

B5: 0,

b6: 0,

b7: 0)

pid_def(j1979_01_013, b0: CAT_TST_SW,

b1: 0,

b2: PURG_TST_SW,

b3: SAIR_TST_SW,

b4: 0,

b5: EGO_TST_SW,

b6: EGO_TST_SW,

b7: EGR_TST_SW)

pid_def(j1979_01_014, b0: CAT_READY,

b1: 0,

b2: PURG_READY,

b3: SAIR_READY,

b4: 0,

b5: EGO_READY,

b6: EGO_READY,

b7: EGR_READY)

23-135

OBDII STRATEGY, J1979 PIDS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continue with pid_definitions)

pid_def(j1979_01_1C, J1979_01_1C)

;SAE RECOMMENDED PRACTICE J1979 E/E DIAGNOSTIC TEST MODES" revised 2/7/94

;has added the requirement for a new J1979 PID 1C to MODE 01 (defined on

;page 14):

; j1979_01_1C     OBD requirements to which vehicle is designed, where:

;                     01 - OBD II (California ARB)

;                     02 - OBD (Federal EPA)

;                     03 - OBD and OBD II

;                     04 - OBD I

;                     05 - Not intended to meet any OBD requirements

;     Although only vehicles meeting California ARB OBD II and Federal OBD

;     regulations are required to support this document, manufacturers of

;     other vehicles may choose to support this request for the convenience

;     of service technicians.

END: pid_definitions

23-136

OBD-II COMPREHENSIVE COMPONENT MONITOR OVERVIEW - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

23.2  COMPREHENSIVE COMPONENTS - MONITOR FLAG AND FAULT CODE SET (CDAF0)

OVERVIEW

Most A/D inputs and some outputs such as the  fuel  pump  do  not  contain  a

specific  monitor  flag  for  input into the OBD-II Trip flag.  It is assumed

that when CCM_MON_TM is reached, a failure of these components  (if  present)

would have been detected.

DEFINITIONS

INPUTS

Registers:

-  ATMR1 = Time since engine start.

Bit Flags:

-  CCM_ECT_MON = OBD-II monitor flag for ECT; 1 -> ECT  monitored  for  this

OBD-II trip.

-  CCM_IAC_MON = Monitor Flag for  IAC;  1  ->  all  IAC  faults  have  been

monitored at least once since power-up.

-  CCM_TRNS_MON  =  Transmission  monitor  flag;  1  ->   All   transmission

electrical components have been monitored at least once this power-up.

OUTPUTS

Bit Flags:

-  CCM_MON = OBD-II monitor flag for comprehensive components.

-  CCM_TST_RDY = Comprehensive Components ready to test.

OTHER

-  ccm_codes = SET OF {act_codes, ect_codes, iac_codes, maf_codes, tp_codes,

tot_codes}

23-137

OBD-II COMPREHENSIVE COMPONENT MONITOR OVERVIEW - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: OBDII_CCM_MON_COM1

always -----------------------------| CCM_TST_RDY = 1

| (ccm test is always ready)

ATMR1 > CCM_MON_TM -----------------| ccm_mon_flg = 1

(enough time for sufficient        | (A/D inputs considered

A/D readings)                     |  monitored)

ccm_mon_flg = 1 --------------|

(enough time for A/D)        |

|

CCM_TRNS_MON = 1 -------------|

(Trans components monitored) |

|

CCM_ECT_MON = 1 --------------|AND -| CCM_MON = 1

(ECT monitored)              |     |  (Comprehensive component monitoring

|     |   complete for trip)

CCM_IAC_MON = 1 --------------|

(Idle Air Control monitored)

23-138

OBDII DIAGNOSTIC EXECUTIVE, FAULT FILTER AND FAULT CODE LOGIC- CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

23.3  FAULT FILTER AND FAULT MONITOR LOGIC (CDAI0)

OVERVIEW

This  strategy   module   contains   the   fault_filter   routine   and   the

fault_filter_clear routine.

It calls the OBDII malfunction,  store_code,  and  clear_malf  routines  when

neccessary.

It is availiable for use by any part of the OBDII diagnostic strategy.

Note that each  of  the  four  routines  are  only  called  "on  demand"  and

individually, by the diagnostic strategies.

DEFINITIONS

INPUTS

Registers:

-  CxxxMON = Filter to count how many times the associated  fault  has  been

monitored.

-  ERROR_DETECTED = Error detected by continuous VIP.

Calibration Constants:

-  CxxxUP = The amount by which CxxxFIL will increment every time it detects

an error (ERROR_DETECTED = 1) w/ fault xxx.  [up count for xxx].

-  FILHYS = FMEM filter count hysteresis, filter counts.

-  MONxxxUP = The amount by which the Code:  xxx  monitor  filter  (CxxxMON)

gets  incremented  each time the test is monitored [upcount for CxxxMON].

(equal to CxxxUP).

OUTPUTS

Registers:

-  ERROR_DETECTED = See above.

23-139

OBDII DIAGNOSTIC EXECUTIVE, FAULT FILTER AND FAULT CODE LOGIC- CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Bit Flags:

-  PxxxMALF = Malfunction flag for fault xxx;  1  ->  malfunction  currently

exists for fault xxx.

OTHER

-  malfunction(egr,Pxxxx)= Public procedure updating the MIL control  status

provided by the MIL control module.

23-140

OBDII DIAGNOSTIC EXECUTIVE, FAULT FILTER AND FAULT CODE LOGIC- CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: OBDII_FAULT_FILTER_COM1

BEGIN:  fault_filter(Pxxxx)

ERROR_DETECTED = 1 ---------------------| CxxxxFIL = CxxxxFIL + CxxxxUP

|

| --- ELSE ---

|

| CxxxxFIL = CxxxxFIL - 1

always ---------------------------------| CxxxxMON = CxxxxMON + MONxxxxUP

If filter has exceeded the  level  then  there  is  a  malfunction.   If  not

inhibited,  the  malfuction  or  store_code  routine is called.  If the fault

filter has dropped enough to clear the fault then clear_malf is called.

CxxxxFIL > CxxxxLVL --------------|

|

ERROR_DETECTED = 1 ---------------|AND -| Do: malfunction(sys,Pxxxx)

|     | ERROR_DETECTED = 0

Pxxxx is a MIL code --------------|     |

| --- ELSE ---

CxxxxFIL > CxxxxLVL --------------|     |

|     |

ERROR_DETECTED = 1 ---------------|AND -| Do: store_code(Pxxxx)

|     | ERROR_DETECTED = 0

Pxxxx is not a MIL code ----------|     |

| --- ELSE ---

|

CxxxxFIL < CxxxxLVL - FILHYS -----------| Do: clear_malf(Pxxxx)

| ERROR_DETECTED = 0

|

| --- ELSE ---

|

| ERROR_DETECTED = 0

END: fault_filter

23-141

OBDII DIAGNOSTIC EXECUTIVE, FAULT FILTER AND FAULT CODE LOGIC- CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: fault_filter_clear(Pxxxx)

CxxxxFIL <= CxxxxLVL - FILHYS ----------| CxxxxFIL = 0

|

| --- ELSE ---

|

| make no change to CxxxxFIL

CxxxxMON <= CxxxxLVL -------------------| CxxxxMON = 0

|

| --- ELSE ---

|

| make no change to CxxxxMON

END: fault_filter_clear

23-142

OBDII STRATEGY, OUTPUT SUBSTITUTION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

23.4  OUTPUT SUBSTITUTION CONTROL (CDAN2)

OVERVIEW

+-----------------------------------+

|      OBDII_OSC_COM                |

+-----------------------------------+

| PUBLIC PROCESSES:                 |

| . definitions                     |

| . substitute                      |

| . osc_initialization              |

| . osc_background_maintainence     |

+-----------------------------------+

|PRIVATE PROCESSES:                 |

| . transform                       |

| . init_osc_table                  |

| . get_osc_allowed_flag            |

| . get_vehicle_state               |

| . set_osc_flags                   |

+-----------------------------------+

The following strategy is used to allow an Off-board Diagnostic Unit (ODU) to

override  the  "normal"  output  control  for  a set of parameters under EEEC

control.  These parameters are mapped to a set  of  channel  numbers.   These

channel  numbers  will be the same in every strategy.  If a specific strategy

does not have the parameter associated with a specific channel  number,  that

channel will not function, but will still be present in the documentation.

The output state control is exercised by using a logic process located in the

diagnostic  executive  called  'substitute(channel_number,parameter)'.   This

logic process is called from within the background loop at each point when  a

parameter  is  written to which SBDS has requested access to via output state

control.  The substitution() process determines if output state control value

substitution  should  be performed, and if so overrides the normal value with

one given by SBDS.  Pictorially, the scheme looks like  the  following  small

segment of the background loop.

.

.

.

.

condition --| X := Y

| Do: substitute(n,X) --->   IF (substitution allowed AND

<--+     substitution requested)

.                        |   THEN

.                        |     override X with OSC_VALUE[n]

.                        |   ELSE

|     do not override X -+

|                        |

+------------------------+

23-143

OBDII STRATEGY, OUTPUT SUBSTITUTION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

There is a four  layer  security  scheme  in  place  to  prevent  inadvertent

substitution.   It  is extremely important that the substitution of values in

the control strategy never occurs unless the vehicle is in  the  service  bay

and  being worked on by the service technician.  The security scheme has been

designed to ensure  this.   The  implementation  in  the  strategy  has  been

designed  to  prevent  activation  by  any  means  other than through the SCP

interface with an authorized ODU.

The first layer  requires  an  ODU  to  transmit  a  TESTER  PRESENT  message

periodically.  This tells the strategy an ODU is currently on the network and

actively communicating.

The second layer requires the ODU to  request  a  seed  (randomly  generated)

value   from   the  PCM  and  succesfully  transform  it  via  a  proprietary

transformation algorithm.  The ODU then returns the transformed value.  If it

matches  the internally calculated value, this layer's requirements have been

met.  This ensures only authorized ODUs gain access to the OSC mode.

The third layer requires the ODU to transmit an  enable  mask  and  requested

substitution  value  for  the channel it wishes to override.  The enable mask

must match the predetermined enable mask that is coded into  the  PCM.   This

allows  for  unlocking  of  each  individual  channel number in the OSC table

seperately, thus minimizing the risk that other channels get overridden  when

not requested.

The final layer verifies, each background loop when the parameter is  written

to,  that  the vehicle is in an operational state that allows the override of

the  requested  channel(s)  and  that  the  override  value  is  within  some

guidelines.   If  the  override  value  is  acceptable and the enable mask is

correct the channel's normal value will be overridden and will remain in that

state  until  one  of  the security layers is breached or the ODU changes the

value.  This layer ensures that the ODU will not damage the vehicle or  place

the vehicle in an unsuitable operating state.

Pictorially the security scheme looks something like the following:

|.ODU present

+----V---------------------+

|                          |

|    |.key is correct      |

|+---V--------------------+|

||                        ||

||   |.enable mask O.K.   ||

||+--V-------------------+||

|||                      |||

|||  |.override currently|||

|||  | allowed           |||

||| +V--------------+    |||

||| | OSC enabled   |    |||

||| +---------------+    |||

||+----------------------+||

|+------------------------+|

+--------------------------+

23-144

OBDII STRATEGY, OUTPUT SUBSTITUTION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Below is a description of how a finite state machine functions.

A finite state machine representation of how the strategy is activated  and

used  is shown below, along with a description of how the finite state

machine

functions.

A finite state machine is a way to graphically illustrate the functioning of

a system or process that can be in a set of DISCRETE states.  The diagram

shows how and when transitions can occur between the states.

+----------------------+

|                      |

|       STATE X        |<-------------------

|                      |        condition

+-----+----------A-----+        ---------

|          |               action

|          |

condition    |          |    condition

---------    |          |    ---------

action      |          |     action

|          |

|          |

+-----V----------+-----+

|                      |

|       STATE Y        |

|                      |

+----------------------+

CONVENTIONS:

· BOXES represent STATES of the system.

· ARROWS represent TRANSITIONS between states.

· CONDITIONS will CAUSE a transition to be taken.

· ACTIONS are done WHEN a transition is taken.

RULES:

· The system can be in only one state at a time.

· The system remains in a given state until an event

occurs to cause a transition.

· No other transitions than those shown can be taken.

23-145

OBDII STRATEGY, OUTPUT SUBSTITUTION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OUTPUT SUBSTITUTION CONTROL FINITE STATE MACHINE

·once per b.g.

---------------                   *key<-      OR

·init_osc_table   *heartbeat<-    *override<-               ·power-up

·OSC_SEED=0       ------------    -----------------    +--- ---------------

·ODU_KEY=0        *ODU_UP_TMR=0   *sequence_error->    |    ·init_osc_table

+-----+            +--+            +--+            |    ·OSC_SEED=0

|     |            |  |            |  |            |    ·ODU_KEY=0

|   +-V------------+--V------------+--V---------+  |

+---|                                           |<-+

|            osc_initial_state              |<-----------------------+

+-------|                                           |                        |

|       +----------A-----------------A--------------+                        |

|*ODU_UP_TMR <     |                 |*key<- &                               |

|    ODU_TIMEOUT & |·ODU_UP_TMR >=   |                                       |

|*send_seed<-      |    ODU_TIMEOUT  |*ODU_KEY<>eec_key                      |

|--------------    |-------------    |-----------------                      |

|*OSC_SEED=random  |·change state    |*error->                               |

|*seed->           |                 |                                       |

|       +----------+-----------------+--------------+                        |

+------>|                                           |                        |

|               osc_seed_sent               |------+*heartbeat<-     |

|                                           |      |------------     |

+-------|                                           |<-----+*ODU_UP_TMR=0    |

|       +-------------------------------------------+                        |

|                                                                            |

|                                                +---------------------------+

|                                                |[{*override<- OR

|                                                |  *heartbeat<-} &

|*ODU_UP_TMR<ODU_TIMEOUT &  *key<-  OR           | *ODU_KEY<>eec_key OR

|*key<-      &              *send_seed<-         | *ODU_KEY=0] OR

|*ODU_KEY=eec_key           ------------         | ·ODU_UP_TMR>=ODU_TIMEOUT

|--------------------       *sequence_error->    | ------------------------

|*general_response->            +--+             | ·*change state

|                               |  |             | *general_response->

|       +-----------------------+--V-------------+--+

+------>|                                           |

|                 key_received              |---+*heartbeat<-   &

|          (substitutions allowed)          |   |*ODU_KEY=eec_key

+-------|                                           |<--+--------------------

|       +-A------------------+--A-------------+--A--+    *ODU_UP_TMR=0

|         |                  |  |             |  |

+---------+                  |  |             +--+

·substitution()      &       |  |            *override<- &

·ODU_UP_TMR<ODU_TIMEOUT   &  |  |            *ODU_KEY=eec_key

·OSC_ENA[no.]=OSC_MASK &     |  |            --------------

·osc_allowed_flg=1           |  |            *update OSC_ENA[no.]

--------------------------   |  |            *update OSC_VALUE[no.]

·override channel no. with   |  |            *general_response->

OSC_VALUE[no.]              +--+

·substitution()       &

[·OSC_ENA[no.]<>OSC_MASK OR

·osc_allowed_flg=0]

------------------------

·do not override channel no.

KEY:

message<- : incoming message to the EEEC.   * : action taken in SCP strategy.

message-> : outgoing message from the EEEC. · : action taken in OSC strategy.

23-146

OBDII STRATEGY, OUTPUT SUBSTITUTION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

There exists a table of values related to the Output State  Control  function

in  the PCM that is located in the HWINT_IO_GP module.  This table is used to

hold the actual values that the offboard diagnostic unit  wishes  to  use  in

palce  of  those calculated by the strategy.  This table is also used to hold

the enable mask transmitted from the offboard diagnostic unit to the  PCM  to

unlock  the  individual  channels.   The  enable  mask is to be compared to a

predetermined, hardcoded enable bit pattern.  This acts as a  key  to  unlock

each  individual  channel  in  the  table.   There  are currently 28 channels

substitution is allowed to be performed on.  These are  shown  in  a  diagram

below.   If  one  or  more  of  these  parameters  are not present in a given

strategy, a request from the ODU to override would have no  external  effect.

This  is  because  the  substitute()  process would never be invoked with the

channel number that does not exist in the strategy.

Implementation note:  The OSC_VALUE[] parameters will have binary scaling  to

match  the  parameter  they will be used to override.  This implies that each

OSC_VALUE[] will have its own unique  scaling.   The  OSC_VAULE[]  parameters

have   been   created   in  the  EDTS  parameter  dictionary  as  OSC_VALUE0,

OSC_VALUE1...  with the appropriate scaling.  The binary scaling  information

is  also  to  be  provided  to the ODU designers so that they will load a bit

pattern into OSC_VALUE[] that is scaled properly and no further  action  will

be  required  by the EEEC once the value has been transmitted and loaded into

the table.

23-147

OBDII STRATEGY, OUTPUT SUBSTITUTION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OSC CHANNEL NUMBERS: The actual OSC table is located in the HWINT_OSC_xx

module. The following list is for documentation only and

provides the OSC channel numbers and their associated parameter.

+----------+--------------+

| channel* |  parameter   |

+----------+--------------+

|  0       |  ISCDTY      |

|  1       |  EAM         |

|  2       |  AM1         |

|  3       |  AM2         |

|  4       |  LAMBSE1     |

|  5       |  LAMBSE2     |

|  6       |  PUMP        |

|  7       |  EGRDC       |

|  8       |  GR_CM       |

|  9       |  PG_DC       |

|  10      |  PGM_CVS_DC  |

|  11      |  FLG_SS_1    |

|  12      |  FLG_SS_2    |

|  13      |  FLG_SS_3    |

|  14      |  FLG_SS_4    |

|  15      |  BCSDC_OUT   |

|  16      |  TV_PRES_BAR |

|  17      |  SAF         |

|  18      |  TRAC_DC     |

|  19      |  FPUMP_DC    |

|  20      |  FP_2SPD_ON  |

|  21      |  IMRC_CMND   |

|  22      |  SS3L_OUT    |

|  23      |  FLG_CS_CM   |

|  24      |  FLG_LK_CM   |

|  25      |  HEGOHTR11   |

|  26      |  HEGOHTR12   |

|  27      |  HEGOHTR21   |

|  28      |  HEGOHTR22   |

+----------+--------------+

'*'...Channel numbers shown are offset 128 from SCP hex values.

Output state control will be invoked from  throughout  the  strategy  at  the

point  the  final  calculation  is made for each one of the parameters listed

above.  A typical example, using channel number 0 (ISCDTY) is  shown  in  the

logic fragment shown below:

condition -------------------| ISCDTY := X + Y

| Do: substitute(0,ISCDTY)

|  ;check to see if value is to be overridden

|  ;by an Offboard Diagnostic Unit.

|  ;ISCDTY will be overwritten, if conditions

|  ;are appropriate and a request has been made

|  ;by the ODU.

If the control strategy requires to know if the value  was  substituted,  the

parameter's  value  can  be  compared  before  and  after the substitute() is

executed.  If the value has changed, it is because the ODU has overridden the

normal value.

23-148

OBDII STRATEGY, OUTPUT SUBSTITUTION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  GR_CM = Commanded gear for shift solenoids.

-  GR_CM_LST = Commanded gear for shift solenoids last background pass.

-  N = Engine speed, RPM.

-  ODU_KEY = Key value received from ODU to unlock OSC.

-  ODU_UP_TMR = ODU tester present timer (free run up counter).

-  OSC_ENAx = Output State Control enable mask, where "x" =  channel  number

(1-17).

-  OSC_SEED = Randomly generated seed value transmitted to ODU.

-  OSC_STATE = Output state control state variable.  (1-17).

-  OSC_VALUEx = Output State Control substitution value, where "x" = channel

number.

-  PDL = PRNDL position - from A to D conversions.

|      -  PGM_TANK_PRS = Purge monitor tank pressure, inches of water.

-  SPD_RATIO = Speed ratio across torque converter (output/input).

-  TOT = Transmission oil temperature, (degrees F).

-  TV_PRES = TV pressure.

-  VSBAR = Filtered vehicle speed for transmission.

Bit Flags:

-  CRANKING = Engine cranking flag.

-  DEMAND_MODE = Flag set when an On-Demand selt test mode is active.

-  NDSFLG = Neutral/Drive flag; 1 -> Drive.

-  OSC_OL = Flag to force open loop during LAMBSE substitution.

-  OSC_SAF_FLG = Output state control substitution of spark advance ->  1  =

substitution performed

-  OSC_SS_FLG = Output state control substitution of a shift solenoid -> 1 =

substition performed

-  PDL_ERROR = MLPS fault detected flag; 1 -> Fault detected.

-  SUBST_REQ[x] = Substitution Requested flag, where "x"  =  channel  number

(1-28).

23-149

OBDII STRATEGY, OUTPUT SUBSTITUTION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  UNDSP = Run/Underspeed Flag; 1 -> Underspeed (or CRANK), 0 -> Run.

-  VSFMFLG = Vehicle speed sensor FMEM flag.

Calibration Constants:

-  MIN_IMRC_RPM = Minimum engine RPM to perform IMRC OSC substitution.

-  MLUS_MX_TOT = TOT high temp limit to test MLUS.

-  NUMEGO = Number of EGO sensors.

|      -  OSC_TANK_MIN = Minimum tank pressure to allow OCS substitution  of  PG_DC

|         and PGM_CVS_DC parameters.

-  ODU_TIMEOUT = Time-out value for certain offboard diagnostic routines.

-  OSC_MASK = Key to unlock each OSC channel.

-  TSTRAT = Transmission strategy switch; 0 -> No transmission control (man,

AOD, ATX, etc.), 1 -> shift indicator.

|      -  VSBAR_MIN = Minimum value of VSBAR necessary for vehicle to be considered

|         moving.

23-150

OBDII STRATEGY, OUTPUT SUBSTITUTION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

|                            STRATEGY MODULE: OBDII_OSC_COM2

BEGIN: definitions

;This process is used to define symbols used locally to make

;the strategy more easy to read.  It is not intended to be executed

unconditionally -----------------------| osc_initial_state := 0

| osc_seed_sent := 1

| key_received := 2

| no_of_channels := 28

| engine_off := 0

| engine_on_pn := 1

| engine_on_gear := 2

| vehicle_moving :=3

| unknown := 4

| true := 1

| false := 0

END: definitions

BEGIN: osc_initialization

;This process is executed during RAM initialization to make the

;OSC parameters ready.  It is also called once per background

;when the OSC mode is not active.

unconditionally -----------------------| Do: init_osc_table

| OSC_SEED := 0

| ODU_KEY := 0

| OSC_OL := 0

| OSC_SAF_FLG := 0

| OSC_STATE := osc_initial_state

| OSC_SS_FLG := 0

END: osc_initialization

23-151

OBDII STRATEGY, OUTPUT SUBSTITUTION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: substitute(channel_number,parameter)

;PUBLIC process used to override parameter values during output

;state control from SBDS/EOL off board diagnostic testing.  This function

;is called from multiple spots in the strategy when a parameter is

;written to to decide if it should be overridden.  The argument 'parameter'

;is the name of the parameter that a substitution request is being made for.

;IMPLEMENTATION NOTE:  this process can be either inline code or subroutine

;call.  The strategy does not require one implementation over the other.

ODU_UP_TMR >= ODU_TIMEOUT ----------|

;ODU offline.                      |OR --| Exit.

|     |  ;no change to parameter.

OSC_STATE <> key_received ----------|     |

(State Machine is in State=2)            | --- ELSE ---

|

TSTRAT > 3 -------------------------------| Do: get_vehicle_state_auto

| Do: get_osc_allowed_flg

| Do: set_osc_flags

|

| --- ELSE ---

|

| Do: get_vehicle_state_manual

| Do: get_osc_allowed_flg

| Do: set_osc_flags

OSC_ENA[channel_number] = OSC_MASK -|

;enable mask correct.              |

|

osc_allowed_flg = true  ------------|AND -| parameter :=

;substitution allowed based on     |     | OSC_VALUE[channel_number]

;state of vehicle operation.       |     |

|     |  ;overwrite the parameter with

SUBST_REQ[channel_number] = 1 ------|     |  ;value from the OSC table.

| Do: OSC_RESPONSE(channel number,

|       00h)

|  ; send general affirmative

|

| --- ELSE ---

OSC_ENA[channel_number] = OSC_MASK -|     |

;enable mask correct.              |AND -| parameter :=

|     | OSC_VALUE[channel_number]

osc_allowed_flg = true -------------|     |  ;overwrite the parameter with

;substitution allowed based on           |  ;value from the OSC table.

;state of vehicle operation.             |

|

| --- ELSE ---

|

SUBST_REQ[channel_number] = 1 ------------| Do: OSC_RESPONSE(channel number,

|                  22h)

|  ; send general reject

| Exit

|  ; no change to parameter

| --- ELSE ---

|

| Exit

|  ; no change to parameter

END: substitute

23-152

OBDII STRATEGY, OUTPUT SUBSTITUTION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: set_osc_flags

PRIVATE process used to set and clear OSC flags to notify other parts of  the

strategy whether or not Output State Control is taking place.

channel_number = 4 -----------|

|OR --|

channel_number = 5 -----------|     |

|

OSC_ENA[4] = OSC_MASK --------|     |

|OR --|

OSC_ENA[5] = OSC_MASK --|     |     |AND -| OSC_OL := true

;enable mask correct.  |AND -|     |     |  ; force open loop fuel

|           |     |

NUMEGO = 2 -------------|           |     |

|     |

osc_allowed_flg = true -------------|     |

| --- ELSE ---

channel_number = 4 -----------------|     |

|OR --| OSC_OL := false

channel_number = 5 -----------------|     | ; allow closed loop fuel

|

| --- ELSE ---

channel_number = 17 ----------------|     |

|     |

OSC_ENA[17] = OSC_MASK -------------|AND -| OSC_SAF_FLG := true

;enable mask correct.              |     |

|     |

osc_allowed_flg = true -------------|     |

| --- ELSE ---

|

channel_number = 17 ----------------------| OSC_SAF_FLG := false

|

| --- ELSE ---

channel_number = 11 ----------|           |

|           |

channel_number = 12 ----------|           |

|OR --|     |

channel_number = 13 ----------|     |     |

|     |     |

channel_number = 14 ----------|     |     |

|AND -| OSC_SS_FLG := true

OSC_ENA[channel_number] = OSC_MASK -|

;enable mask correct.              |

|

osc_allowed_flg = true -------------|

END: set_osc_flags

23-153

OBDII STRATEGY, OUTPUT SUBSTITUTION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: osc_background_maintainence

;PUBLIC process executed once per background pass to ensure the OSC table

;and SBDS_KEY do not get corrupted by some external source (RCON, noise).

OSC_STATE = osc_initial_state ---|

(State Machine is in State=0)   |OR --| Do: osc_initialization

|

ODU_UP_TMR >= ODU_TIMEOUT -------|

END: osc_background_maintainence

BEGIN: init_osc_table

;PRIVATE process used to initialize the output state control table.  Each

;enable mask in the table will be put into a disabled state and each

;substitution value will be cleared.

For x := 0 to no_of_channels execute the following:

unconditionally -----------------------| OSC_VALUE[x] := 0

| OSC_ENA[x] := 0

| SUBST_REQ[x] := 0

END: init_osc_table

23-154

OBDII STRATEGY, OUTPUT SUBSTITUTION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: get_osc_allowed_flg

;PRIVATE process used to decide if vehicle operational conditions

;are correct to allow substitution of the specified parameter.

channel_number > no_of_channels -------|

|

DEMAND_MODE = 1 -----------------------|OR --| osc_allowed_flg := false

|     |

OBDII_RESET = 1 -----------------------|     |

| --- ELSE ---

channel_number <> 8 -------------------|     |

|AND -| osc_allowed_flg := true

vehicle_state = engine_off ------|     |     |

|OR --|     |

vehicle_state = engine_on_pn ----|           |

| --- ELSE ---

|     channel_number <> 0 -------------------|     |

|                                            |     |

|     channel_number <> 6 -------------------|     |

|                                            |     |

channel_number <> 8 -------------------|     |

|     |

channel_number <> 11 ------------------|     |

|     |

channel_number <> 12 ------------------|     |

|     |

channel_number <> 13 ------------------|     |

|     |

channel_number <> 14 ------------------|     |

|     |

channel_number <> 15 ------------------|     |

|     |

channel_number <> 16 ------------------|     |

|AND -| osc_allowed_flg := true

|     channel_number <> 17 ------------------|     |

|                                            |     |

|     channel_number <> 19 ------------------|     |

|                                            |     |

|     channel_number <> 20 ------------------|     |

|                                            |     |

channel_number <> 21 ------------------|     |

|     |

channel_number <> 22 ------------------|     |

|     |

channel_number <> 23 ------------------|     |

|     |

channel_number <> 24 ------------------|     |

|     |

vehicle_state = engine_on_gear --------|     |

| --- ELSE ---

|                               (continued on next page)

23-155

OBDII STRATEGY, OUTPUT SUBSTITUTION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|                            (continued from previous page)

channel_number = 4 --------------|           |

|           |

channel_number = 5 --------------|           |

|           |

channel_number = 25 -------------|           |

|OR --|     |

channel_number = 26 -------------|     |AND -| osc_allowed_flg := true

|     |     |

channel_number = 27 -------------|     |     |

|     |     |

channel_number = 28 -------------|     |     |

|     |

vehicle_state <> unknown --------------|     |

| --- ELSE ---

|     channel_number = 9 --------------------|     |

|                                            |     |

|     vehicle_state <> unknown --------------|     |

|                                            |AND -| osc_allowed_flg := true

|     vehicle_state <> vehicle_moving -|     |     |

|                                      |OR --|     |

|     OSC_VALUE[9] = 0 ----------------|           |

|                                                  | --- ELSE ---

|     channel_number = 10 -------------------|     |

|                                            |     |

|     PGM_TANK_PRS > OSC_TANK_MIN -----------|AND -| osc_allowed_flg := true

|                                            |     |

|     vehicle_state <> unknown --------------|     |

|                                                  | --- ELSE ---

channel_number <> 8 -------------------|     |

|     |

channel_number <> 15 ------------------|     | osc_allowed_flg := false

|     |

channel_number <> 16 ------------------|AND -|

|     |

channel_number <> 21 ------------------|     |

|     |

channel_number <> 24 ------------------|     |

| --- ELSE ---

channel_number = 8 --------------------|     |

|     |

OSC_VALUE[channel_number] > 0 ---------|     |

|     |

OSC_VALUE[channel_number] < 5 ---------|AND -| osc_allowed_flg := true

|     |

abs(OSC_VALUE[channel_number]          |     |

- GR_CM_LST ) <= 1 --|     |

|     |

vehicle_state = vehicle_moving --------|     |

| --- ELSE ---

|                               (continued on next page)

23-156

OBDII STRATEGY, OUTPUT SUBSTITUTION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|                            (continued from previous page)

|

channel_number = 8 --------------------------| osc_allowed_flg := false

|

| --- ELSE ---

channel_number = 15 -------------|           |

|OR --|     |

channel_number = 24 -------------|     |     |

|     |

vehicle_state = vehicle_moving --------|     |

|AND -| osc_allowed_flg := true

OSC_VALUE[channel_number] = 0.0 -|     |     |

|     |     |

OSC_VALUE[channel_number]        |     |     |

= 1.0 -|     |     |     |

|     |OR --|     |

TOT > 60 ------------------|     |           |

|     |           |

TOT < MLUS_MX_TOT ---------|AND -|           |

|                 |

GR_CM > 1 -----------------|                 |

|                 |

SPD_RATIO > 0.7 -----------|                 |

| --- ELSE ---

channel_number = 15 -------------------|     |

|OR --| osc_allowed_flg := false

channel_number = 24 -------------------|     |

| --- ELSE ---

channel_number =  16 ------------------|     |

|     |

vehicle_state = vehicle_moving --------|     | osc_allowed_flg := true

|     |

OSC_VALUE[channel_number] > TV_PRES ---|AND -|

|     |

OSC_VALUE[channel_number] > 0 ---------|     |

|     |

OSC_VALUE[channel_number] < 127 -------|     |

| --- ELSE ---

channel_number = 21 -------------------|     |

|     |

vehicle_state = engine_on_pn ----|     |     |

|OR --|AND -| osc_allowed_flg := true

vehicle_state = engine_on_gear --|     |     |

|     |

N > MIN_IMRC_RPM ----------------------|     |

| --- ELSE ---

|

| osc_allowed_flg := false

END: get_osc_allowed_flg

23-157

OBDII STRATEGY, OUTPUT SUBSTITUTION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: get_vehicle_state_auto

;PRIVATE process used to determine what operational state the vehicle is in.

CRANKING = 1 ---------------|

|

PDL_ERROR = 0 --------------|AND -| vehicle_state := engine_off

|     |

PDL = 5 --------------|     |     |

|OR --|     |

|     PDL = 7 --------------|           |

| --- ELSE ---

UNDSP = 0 ------------------|     |

|     |

|     VSBAR <= VSBAR_MIN ---------|     |

|     |

VSFMFLG = 0 ----------------|AND -| vehicle_state := engine_on_pn

|     |

PDL_ERROR = 0 --------------|     |

|     |

PDL = 5 --------------|     |     |

|OR --|     |

|     PDL = 7 --------------|           |

| --- ELSE ---

UNDSP = 0 ------------------|     |

|     |

|     VSBAR <= VSBAR_MIN ---------|     |

|     |

VSFMFLG = 0 ----------------|AND -| vehicle_state := engine_on_gear

|     |

PDL_ERROR = 0 --------------|     |

|     |

PDL = 6 --------------|     |     |

|OR --|     |

PDL <= 4 -------------|           |

| --- ELSE ---

UNDSP = 0 ------------------|     |

|     |

|     VSBAR > VSBAR_MIN ----------|     |

|     |

VSFMFLG = 0 ----------------|AND -| vehicle_state := vehicle_moving

|     |

PDL_ERROR = 0 --------------|     |

|     |

PDL = 4 --------------------|     |

| --- ELSE ---

|

| vehicle_state := unknown

END: vehicle_state_auto

23-158

OBDII STRATEGY, OUTPUT SUBSTITUTION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: get_vehicle_state_manual

;PRIVATE process used to determine what operational state the vehicle is in

for

; an automatic transmission.

CRANKING = 1 ---------------|

|AND -| vehicle_state := engine_off

NDSFLG = 0 -----------------|     |

| --- ELSE ---

UNDSP = 0 ------------------|     |

|     |

|     VSBAR <= VSBAR_MIN ---------|     |

|AND -| vehicle_state := engine_on_pn

NDSFLG = 0 -----------------|     |

|     |

VSFMFLG = 0 ----------------|     |

| --- ELSE ---

UNDSP = 0 ------------------|     |

|     |

|     VSBAR <= VSBAR_MIN ---------|     |

|AND -| vehicle_state := engine_on_gear

NDSFLG = 1 -----------------|     |

|     |

VSFMFLG = 0 ----------------|     |

| --- ELSE ---

UNDSP = 0 ------------------|     |

|     |

NDSFLG = 1 -----------------|     |

|AND -| vehicle_state := vehicle_moving

|     VSBAR > VSBAR_MIN ----------|     |

|     |

VSFMFLG = 0 ----------------|     |

| --- ELSE ---

|

| vehicle_state := unknown

END: vehicle_state_manual

23-159

OBDII STRATEGY, TRANSMISSION COMPRESHENSIVE COMPONENT MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

23.5  TRANSMISSION COMPREHENSIVE COMPONENT MONITOR (CDAK0)

OVERVIEW

This module determines when all of  the  transmission  electrical  components

have  been monitored for this "trip".  This information is then passed to the

CCM monitor module, which then informs  the  system  when  all  Comprehensive

Component Monitoring is complete.

A "trip" is defined as "vehicle operation of duration and driving  mode  such

that  all  components  and  systems  are  monitored  at  least once".  OBD-II

regulations require that a "trip" be completed during the CVS-72 cycle.

Transmission CCM works as follows - each component fault  that  requires  MIL

illumination carries two flags:

1) PxxxMON  - Fault xxx monitor flag; 1 -> fault xxx has been monitored

at least once this power-up.

2) PxxxMALF - Fault xxx malfunction flag; 1 -> fault xxx has been

detected and currently exits.

Once the monitor flag for each Transmission Electrical Component is set,  the

CCM_TRNS_MON  flag  will be set to signify compeltion of Transmission CCM for

this "trip".

NOTE:  For manual transmission applications, the CCM_TRNS_MON  flag  will  be

set once the vehicle speed sensor (VSS) has been monitored.

DEFINITIONS

INPUTS

Registers:

-  TSTRAT  =  Transmission  Strategy  Switch;  The  TSTRAT  software  switch

controls which transmission control strategy is executed:

0 -> No transmission control (Manual transmission, AOD, ATX, C6,

C3, etc.)

1 -> SIL (Shift indicator light)

2 -> A4LD with 3-4 shift control and converter clutch control

3 -> AXOD

4 -> C64E (E4OD)

5 -> A41DE

6 -> FAX-4

7 -> AOD-E (AOD-I)

8 -> 4EAT

9 -> CD4E

Bit Flags:

-  PDL_MON = MLPS monitored flag; 1 -> MLPS has been monitored at least once

this power-up.

23-160

OBDII STRATEGY, TRANSMISSION COMPRESHENSIVE COMPONENT MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PxxxMON = OBDII monitor flag for fault xxx;  1  ->  fault  xxx  has  been

monitored at least once this power-up.

Calibration Constants:

-  CC_TRIP_SW = Calibration  switch  used  to  enable/disable  counting  the

converter clutch functional test as part of a "trip"; 1 -> enabled.

-  SS_TRIP_SW = Calibration switch used to enable/disable counting the shift

solenoid functional test as part of a "trip"; 1 -> enabled.

OUTPUTS

Bit Flags:

-  CCM_TRNS_MON  =  Transmission  monitor  flag;  1  ->   All   transmission

electrical components have been monitored at least once this power-up.

-  CC_TRIP_DONE  =  Converter  clutch  functional  "trip"  flag;  1  ->  the

Converter clutch has been functionally monitored for this "trip".

-  SS_TRIP_DONE = Shift Solenoid functional "trip"  flag;  1  ->  the  Shift

Solenoids have been functionally monitored for this "trip".

OTHER

-  P0500 = Fault code for vss failure.

-  P0720 = OBDII fault code - Output Speed Sensor Malfunction.

-  P0743 = Torque converter clutch system electrical.

-  P0750 = OBD-II fault code for SSA circuit malfunction.

-  P0755 = OBD-II fault code for SSB circuit malfunction.

-  P1747 = Fault code; EPC VFS short to ground.

23-161

OBDII STRATEGY, TRANSMISSION COMPRESHENSIVE COMPONENT MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: OBDII_TRANS_MON_COM1

Transmission portion of the CCM is complete when each transmission electrical

component  has  been  monitored at least once this power-up (the monitor flag

for that component is set).

SS_TRIP_SW = 0 -------------------|

|

SS_MALF_FLG = 1 ------------------|OR --| SS_TRIP_DONE = 1

|     |  (Either counting shift solenoids

FLG_SS_MON = 1 -------------------|     |   as part of a trip is cal'ed out,

|   or a malfunction is present, or

|   the solenoids have been monitored

|   as "good", set the shift solenoid

|   "trip" done flag)

CC_TRIP_SW = 0 -------------------|

|

SS_MALF_FLG = 1 ------------------|OR --| CC_TRIP_DONE = 1

|     |  (Either counting converter clutch

FLG_PWM_MON = 1 ------------|     |     |   as part of a trip is cal'ed out,

|AND -|     |   or a shift solenoid malfunction

P1744MON = 1 ---------------|           |   is present - can't test

|   converter, or the solenoid has

|   been monitored as "good", set

|   the converter clutch "trip"

|   done flag)

23-162

OBDII STRATEGY, TRANSMISSION COMPRESHENSIVE COMPONENT MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

P0500MON = 1 ---------------------|

(VSS has been monitored)         |

|

TSTRAT <= 3 ----------------|     |

(Manual/non-electric       |     |

transmission)             |     |AND -| CCM_TRNS_MON = 1

|     |     | (All transmission

PDL_MON = 1 ----------|     |     |     |  electrical compnents

(MLPS has been       |     |     |     |  have been monitored

monitored)          |     |     |     |  once this power-up)

|     |OR --|

P0720MON = 1 ---------|     |

(OSS has been        |     |

monitored)          |     |

|     |

CC_TRIP_DONE = 1 -----|     |

(CC functionally     |     |

monitored)          |     |

|     |

P0743MON = 1 ---------|     |

(Converter Clutch    |     |

has been monitored) |AND -|

|

SS_TRIP_DONE = 1 -----|

(SS functionally     |

monitored)          |

|

P0750MON = 1 ---------|

(SS1 has been        |

monitored)          |

|

P0755MON = 1 ---------|

(SS2 has been        |

monitored)          |

|

P1747MON = 1 ---------|

(EPC VFS has been monitored

for a short circuit)

23-163

OBDII STRATEGY, TRANSMISSION COMPRESHENSIVE COMPONENT MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

23.6  FEATURE:  SELF TEST CONTROL - V1.8_OBDII_SELF_TEST_CONTROL (CDAN2)

23.6.1  ENGINE OFF SEQUENCE CONTROLER (CDAB0)

OVERVIEW

The Key On engine off (EO) On-Demand Test sequence is  enabled  in  the  same

manner  as  other OBD-II monitors.  When the local conditions (key on, engine

not running) are met, the flag EO_TST_RDY is set.  When requested  (via  Scan

Tool)  the  OBD-II  executive  will  set  the flag EO_TST_ENA to initiate the

engine off Sequence.

The control of the test sequence is documented by  use  of  a  state  pointer

called  "EO_STATUS".  This parameter is compared to and assigned the value of

non-modifiable calibration constants to control the progress through the test

sequence.

DEFINITIONS

INPUTS

Registers:

-  EO_STATUS = State pointer that indicates current state of engine  off  on

demand test sequence.

-  DEMAND_TIMER = Self Test utility timer.

Bit Flags:

-  CRKFLG = Crank flag.

-  DEMAND_ABORT = Flag to indicate that the on-demand test is in the process

of an abort.

-  DEMAND_MODE = Flag set when an On-Demand self test mode is active.

-  EO_ACTIVE = Flag to indicate that the engine off  on-demand  test  is  in

progress.

-  EO_TST_ENA = Flag that signals the engine off on-demand test to run.

-  EO_TST_RDY = Flag to signal OBD-II executive that conditions to  run  the

engine off on-demand test are met.

Calibration Constants:

-  CCM_EO_DONE = Value of EO_STATUS when CCM on-demand engine  off  test  is

complete.

-  CCM_EO_WAIT = Value of EO_STATUS when waiting for engine off CCM test  to

complete.

23-164

ENGINE OFF SELF TEST, ENGINE OFF SEQUENCE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  ECU_EO_DONE = Value of EO_STATUS to when ECU test is done  during  Engine

off on-demand sequence.

-  ECU_EO_INIT = Value of EO_STATUS to signal  for  ECU  test  (Instruction,

KAM, RAM, ROM).

-  EO_DONE = Value of EO_status when engine off on-demand test is complete.

-  EO_INACTIVE = Value of  ER_STATUS  when  engine  off  on-demand  test  is

inactive.

-  OSM_EO_DONE = Value of EO_STATUS when OSM on test is complete.

-  OSM_EO_INIT = Value of EO_STATUS when OSM test is initialized.

-  OSM_EO_WAIT = Value of EO_STATUS when waiting for OSM  test  to  complete

during engine off on-demand test.

OUTPUTS

Registers:

-  EO_STATUS = See above.

-  DEMAND_TIMER = See above.

Bit Flags:

-  CCM_EO_ENA = Comprehensive Componant engine off tests enabled.

-  DEMAND_ABORT = See above.

-  EO_ACTIVE = See above.

-  EO_DONE = Value of EO_STATUS when engine off on-demand test is complete.

-  EO_TST_ENA = See above.

-  EO_TST_RDY = See above.

-  OSM_EDF_OFF = Request for EDF Off from OSM or OTM test.

-  OSM_EDF_ON = Request for EDF on from OSM or OTM test.

-  OSM_EO_ON = On state requested for for engine off OSM test; 1 -> test  on

state.

-  OSM_EO_OFF = Off State requested for for engine off OSM test; 1  ->  test

off state.

-  OSM_HEDF_OFF = Request for HEDF Off from OSM or OTM test.

-  OSM_HEDF_ON = Request for HEDF on from OSM or OTM test.

23-165

ENGINE OFF SELF TEST, ENGINE OFF SEQUENCE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: VO_OBDII_SEQUENCER_COM1

This diagram provides for some "bullet proofing" when none of the demand mode

tests are active (engine running, engine off, or output test mode).

DEMAND_MODE = 0 ------------------------------| OSM_EO_ON := 0

(None of the on demand tests are activated   | OSM_EO_OFF := 0

by the OBD-II exec)                         | OSM_EDF_ON := 0

| OSM_EDF_OFF := 0

| OSM_HEDF_ON := 0

| OSM_HEDF_OFF := 0

|  (end request for OSM tests)

|

| CCM_EO_ENA := 0

|  (End EO CCM test)

This diagram detects if an abort has been requested  (either  by  the  OBD-II

executive  or  by  this  sequencer).   If  an  abort has been initiated, this

diagram will insure that it  is  completed  and  then  send  the  appropriate

response  on  the  SCP interface to the diagnostic tools.  Then, this diagram

determines if the engine off on demand test is able to and should be  running

now.  And, once activated, the test sequence is controlled by this diagram.

DEMAND_ABORT = 1 -----------------------|

(Abort requested)                      |

|

EO_STATUS = EO_INACTIVE ----------------|AND -| DEMAND_ABORT := 0

(abort complete)                       |     |  (Abort complete)

|     |

EO_ACTIVE = 1 --------------------------|     | EO_TST_RDY = 0

(Abort of engine off test in progress)       |  (Abort done, not running

|   now)

|

| EO_STATUS :=

|        EO_INACTIVE

|

| EO_ACTIVE := 0

|  (test no longer active)

|

| --- ELSE ---

(continued on next page)

23-166

ENGINE OFF SELF TEST, ENGINE OFF SEQUENCE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

DEMAND_ABORT = 1 -----------------------|     |

(Abort requested)                      |AND -| EO_STATUS := EO_INACTIVE

|     |  (abort of individual test

EO_ACTIVE = 1 --------------------------|     |   complete)

(Abort of engine off test in progress)       | OSM_EO_ON := 0

| OSM_EO_OFF := 0

|

| OSM_EDF_ON := 0

| OSM_EDF_OFF := 0

| OSM_HEDF_ON := 0

| OSM_HEDF_OFF := 0

|  (end request for OSM tests)

|

| CCM_EO_ENA := 0

|  (End EO CCM test)

|

| --- ELSE ---

EO_ACTIVE = 1 --------------------------|     |

(test in progress)                     |     |

|AND -| DEMAND_ABORT := 1

EO_TST_ENA = 0 -------------------|     |     | (Test no longer able to run,

(test not enabled by exec)       |OR --|     |   time to bail out...)

|           |

CRKFLG = 0 -----------------------|           | EO_TST_RDY := 1

(not in crank)                               |  (abort in progress)

|

| (Either the test was running

|  and the OBD-II exec. cancled

|  the test or the local

|  conditions (engine not

|  running) were violated.)

|

| --- ELSE ---

EO_ACTIVE = 0 --------------------------|     |

(test not in progress                  |     |

|     |

EO_TST_ENA = 1 -------------------------|AND -| EO_TST_RDY := 0

(test is  enabled by exec)             |     |  (For some strange reason,

|     |   the test is enabled by

CRKFLG = 0 -----------------------------|     |   the exec but can not now

(not in crank)                               |   run. Don't start.)

|  (This is not an abort

|   because the test was not

|   in progress the last

|   background loop.)

|

| EO_STATUS := EO_INACTIVE

|

| --- ELSE ---

(continued on next page)

23-167

ENGINE OFF SELF TEST, ENGINE OFF SEQUENCE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

EO_TST_ENA = 0 -------------------------|     |

(test not enabled by OBD-II exec)      |AND -| EO_STATUS := EO_INACTIVE

|     |

CRKFLG = 1 -----------------------------|     | EO_TST_RDY := 1

(crank mode)                                 |  (Signal OBD-II exec that

|   EO test could run if

|   enabled)

|

| EO_ACTIVE := 0

|  (test not active)

|

| --- ELSE ---

|

EO_TST_ENA = 0 -------------------------------| EO_STATUS := EO_INACTIVE

(test not enabled)                           |

(note that any subsequent logic in this      | EO_TST_RDY := 0

chart will execute only if EO is            |  (test is not currently

enabled by OBD-II exec. and is able to      |   enabled by OBD-II and is

run.)                                       |   not ready to run, exit.)

|

| EO_ACTIVE := 0

|  (test not active)

|

| --- ELSE ---

|

EO_STATUS = EO_INACTIVE ----------------------| EO_STATUS :=

(test not yet started)                       |          ECU_EO_INIT

|  (test RAM / KAM / ROM and

|   instruction set.)

|

| EO_ACTIVE := 1

|  (test now active)

|

| --- ELSE ---

(continued on next page)

23-168

ENGINE OFF SELF TEST, ENGINE OFF SEQUENCE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|

EO_STATUS = ECU_EO_DONE ----------------------| EO_STATUS :=

|        CCM_EO_WAIT

|  (setup to start test)

| OSM_EO_ON := 0

| OSM_EO_OFF := 1

| OSM_EDF_ON := 0

| OSM_EDF_OFF := 1

| OSM_HEDF_ON := 0

| OSM_HEDF_OFF := 1

|  (start OSM off tests)

| CCM_EO_ENA := 1

|  (Enable CCM test)

| DEMAND_TIMER := 0

|  (set up time delay for

|   CCM tests.)

|

| --- ELSE ---

EO_STATUS = CCM_EO_WAIT ----------------|     |

|AND -| EO_STATUS :=

DEMAND_TIMER > EO_CCM_DLY --------------|     |     CCM_EO_DONE

(enough time has passed for CCM test.)       |

| --- ELSE ---

|

EO_STATUS = CCM_EO_WAIT ----------------------| EO_STATUS :=

|             CCM_EO_WAIT

| (Just sit tight, wait for

|  time delay, allow

|  DEMAND_TIMER to run)

|

| --- ELSE ---

|

EO_STATUS = CCM_EO_DONE ----------------------| EO_STATUS :=

|      OSM_EO_WAIT

|

| OSM_EO_OFF = 0

| OSM_EO_ON = 1

| OSM_EDF_ON := 1

| OSM_EDF_OFF := 0

| OSM_HEDF_ON := 1

| OSM_HEDF_OFF := 0

|  (start OSM on tests)

| DEMAND_TIMER := 0

|  (set up time delay to allow

|   OSMs to settle.)

|

| --- ELSE ---

EO_STATUS = OSM_EO_WAIT ----------------|     |

|AND -| EO_STATUS :=

DEMAND_TIMER > EO_OSM_DLY --------------|     |     OSM_EO_DONE

(enough time has passed for OSM test.)       |

| --- ELSE ---

(continued on next page)

23-169

ENGINE OFF SELF TEST, ENGINE OFF SEQUENCE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|

EO_STATUS = OSM_EO_WAIT ----------------------| EO_STATUS :=

|             OSM_EO_WAIT

| (Just sit tight, wait for

|  time delay, allow

|  DEMAND_TIMER to run)

|

| --- ELSE ---

|

EO_STATUS = OSM_EO_DONE ----------------------| EO_STATUS :=

|     EO_DONE

|

| OSM_EO_OFF := 1

| OSM_EO_ON := 0

| OSM_EDF_ON := 0

| OSM_EDF_OFF := 1

| OSM_HEDF_ON := 0

| OSM_HEDF_OFF := 1

|  (turn outputs back off)

|

| CCM_EO_ENA := 0

|  (End EO CCM test)

|

| --- ELSE ---

|

EO_STATUS = EO_DONE --------------------------| EO_STATUS =

|         EO_INACTIVE

|  (done with test)

|

| EO_MON := 1

|  (signal exec)

|

| OSM_EO_OFF := 0

| OSM_EO_ON := 0

| OSM_EDF_ON := 0

| OSM_EDF_OFF := 0

| OSM_HEDF_ON := 0

| OSM_HEDF_OFF := 0

|  (end OSM requests)

|

| EO_ACTIVE := 0

|  (No longer runing test)

|

| --- ELSE ----

|

| (no action)

23-170

ENGINE OFF SELF TEST, OUTPUT TEST MODE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

23.6.2  OUTPUT TEST MODE CONTROLLER (CDAF0)

OVERVIEW

The output test mode allows service  technitions  to  control  the  state  of

certian EEC outputs for diagnostic purpouses.  There are four possible output

modes:  all off, OSM outputs on, EDF on, and HEDF on.

The Output Test Mode (OTM) On-Demand Test sequence is  enabled  in  the  same

manner  as  other OBD-II monitors.  When the local conditions (key on, engine

not running) are met, the flag OTM_TST_RDY is set.  When requested (via  Scan

Tool)  the  OBD-II  executive  will  set the flag OTM_TST_ENA to initiate the

engine off Sequence.  The  scan  tool  will  also  set  one  of  four  flags:

OSM_ON_REQ,  OSM_OFF_REQ,  OSM_EDF_REQ,  and  OSM_HEDF_REQ  to  indicate  the

requested state.  A timer is used to return the outputs to the off  /  normal

state  and  exit  the  OTM  if  the  requested  state  is  not  changed  in a

calibratible ammount of time.

DEFINITIONS

INPUTS

Registers:

-  OTM_STATE = Current State of Output Test Mode Controller (Inactive,  Off,

On, EDF On...).

-  OTM_TIMER = Output Test Mode Time out timer.

-  OUTPUT_MODE = Output Test Mode State requested by SCP.   (Inactive,  Off,

On, EDF on ...).

Bit Flags:

-  CRANKING = Engine mode flag; 1 -> crank mode, 0 -> underspeed/run mode.

-  DEMAND_MODE = Flag set when an On-Demand self test mode is active.

-  OTM_TST_ENA = Output Test Mode Enabled by OBD-II Exec.

Calibration Constants:

-  OTM_MAX_TM = Maximum time in Output Test Mode without changing states.

23-171

ENGINE OFF SELF TEST, OUTPUT TEST MODE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OUTPUTS

Registers:

-  OTM_STATE = See above.

Bit Flags:

-  OSM_EDF_OFF = Request for EDF Off from OSM or OTM test.

-  OSM_EDF_ON = Request for EDF on from OSM or OTM test.

-  OSM_EO_OFF = Off state requested for OSM outputs  during  engine  off  on

demand test.

-  OSM_EO_ON = ON state requested for  OSM  outputs  during  engine  off  on

demand test.

-  OSM_HEDF_OFF = Request for HEDF Off from OSM or OTM test.

-  OSM_HEDF_ON = Request for HEDF On from OSM or OTM test.

-  OTM_TST_RDY = Signal OBD-II Exec.  that Output Test Mode is able to run.

23-172

ENGINE OFF SELF TEST, OUTPUT TEST MODE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: VO_OBDII_OTM_COM1

DEMAND_MODE = 0 ------------------------| OSM_EO_ON := 0

| OSM_EO_OFF := 0

| OSM_EDF_ON := 0

| OSM_EDF_OFF := 0

| OSM_HEDF_ON := 0

| OSM_HEDF_OFF := 0

| (insure that demand mode

| flags are not set when

| no test is in progress.)

OTM_TST_ENA = 0 ------------------|

(test not enabled by OBD-II      |

exec)                           |

|AND -| OTM_TST_RDY := 1

CRANKING = 1 ---------------------|     |  (Signal OBD-II exec that

(crank mode)                     |     |   EO test could run if enabled)

|     | run_otm_mode := 0

OTM_STATE = 0 --------------------|     |  (done with OTM)

(test not active)                      |

| --- ELSE ---

OTM_TST_ENA = 0 ------------------|     |

(test not enabled)               |AND -| OTM_TST_RDY := 0

|     |  (test is not currently

OTM_STATE = 0 --------------------|     |   enabled by OBD-II and is

(test not active)                      |   not ready to run, exit.)

| run_otm_mode := 0

|  (done with OTM)

|

| --- ELSE ---

CRANKING <> 1 --------------------|     |

(not in crank mode)              |     |

|     |

OTM_TIMER > OTM_MAX_TM -----|     |     |

(test timed out)           |AND -|OR --| OTM_TST_RDY := 0

|     |     |  (Bail out)

OTM_STATE <> 0 -------------|     |     | OSM_EO_ON := 0

(test active)                    |     | OSM_EO_OFF := 0

|     | OSM_EDF_ON := 0

OTM_TST_ENA = 0 ------------------|     | OSM_EDF_OFF := 0

(test not enabled by OBD-II exec)      | OSM_HEDF_ON := 0

| OSM_HEDF_OFF := 0

|  (Be sure to turn outputs

|   back to normal state)

| run_otm_mode := 0

|  (done with OTM)

|

| --- ELSE ---

|

| run_otm_mode := 1

|  (continue in OTM mode)

|  (Any subsequent logic in this

|   test should execute only if the

|   OTM is enabled by OBD-II exec.

|   and is able to run)

23-173

ENGINE OFF SELF TEST, OUTPUT TEST MODE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OTM_STATE <> OUTPUT_MODE ---------|

(Change in state requested)      |

|

OUTPUT_MODE <= 4 -----------------|AND -| OTM_TIMER := 0

(4 is the largest valid mode for |     |  (reset timer to zero if a change

this particular strategy module)|     |   in state is requested - something

|     |   is being done to the car.

run_otm_mode = 1 -----------------|     |   Or, keep the timer at zero to

(otm mode is active)                   |   be ready when the test starts)

|

| send(GENERAL_RESPONSE:

|    7Fh, B1h, 00h, 25h,

|    OUTPUT_MODE, 00h)

|  (valid request recieved)

|

| --- ELSE ---

|

| no action

23-174

ENGINE OFF SELF TEST, OUTPUT TEST MODE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

run_otm_mode <> 1 ----------------------| OTM_STATE := 0

(otm mode is not active.)              |  (test is inactive)

|

| --- ELSE ---

|

OUTPUT_MODE = 0 ------------------------| OSM_EO_ON := 0

(Request inactive state)               | OSM_EO_OFF := 0

| OSM_EDF_ON := 0

| OSM_EDF_OFF := 0

| OSM_HEDF_ON := 0

| OSM_HEDF_OFF := 0

|  (no state request)

|

| OTM_STATE := OUTPUT_MODE

|  (inactive state)

|

| --- ELSE ---

|

OUTPUT_MODE = 1 ------------------------| OSM_EO_ON := 0

(Request off states)                   | OSM_EO_OFF := 1

| OSM_EDF_ON := 0

| OSM_EDF_OFF := 1

| OSM_HEDF_ON := 0

| OSM_HEDF_OFF := 1

|  (start OSM off tests)

|

| OTM_STATE := OUTPUT_MODE

|  (off state)

|

| --- ELSE ---

|

OUTPUT_MODE = 2 ------------------------| OSM_EO_ON := 1

(Request on state)                     | OSM_EO_OFF := 0

| OSM_EDF_ON := 0

| OSM_EDF_OFF := 1

| OSM_HEDF_ON := 0

| OSM_HEDF_OFF := 1

|  (start OSM on tests)

|

| OTM_STATE := OUTPUT_MODE

|  (on state)

|

| --- ELSE ---

|

OUTPUT_MODE = 3 ------------------------| OSM_EO_ON := 0

(Request EDF on state)                 | OSM_EO_OFF := 1

| OSM_EDF_ON := 1

| OSM_EDF_OFF := 0

| OSM_HEDF_ON := 0

| OSM_HEDF_OFF := 1

|  (start EDF on test)

|

| OTM_STATE := OUTPUT_MODE

|  (EDF on state)

|

| --- ELSE ---

(continued on next page)

23-175

ENGINE OFF SELF TEST, OUTPUT TEST MODE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|

OUTPUT_MODE = 4 ------------------------| OSM_EO_ON := 0

(Request HEDF on state)                | OSM_EO_OFF := 1

| OSM_EDF_ON := 1

| OSM_EDF_OFF := 0

| OSM_HEDF_ON := 1

| OSM_HEDF_OFF := 0

|  (start HEDF on test)

|

| OTM_STATE := OUTPUT_MODE

|  (HEDF on state

|

| --- ELSE ---

|

| OSM_EO_ON := 0

| OSM_EO_OFF := 0

| OSM_EDF_ON := 0

| OSM_EDF_OFF := 0

| OSM_HEDF_ON := 0

| OSM_HEDF_OFF := 0

|  (end requests)

|

| (An invalid value of

|  output_mode was

|  recieved)

|

| OUTPUT_MODE := 0

| OTM_STATE := 0

|  (inactive state)

|

| send(GENERAL_RESPONSE:

|    7Fh, B1h, 00h, 25h,

|    OUTPUT_MODE, 12h)

|  (invalid request)

23-176

ENGINE RUNNING SELF TEST, ENGINE RUNNING SEQUENCE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

23.6.3  ENGINE RUNNING SEQUENCE CONTROLLER (CDAN2)

OVERVIEW

The Key On Engine Running (ER) On-Demand Test sequence is enabled in the same

manner as other OBD-II monitors.  When the local conditions (vehicle stopped,

not in gear) are met, the flag ER_TST_RDY is set.  When requested  (via  Scan

Tool)  the  OBD-II  executive  will  set  the flag ER_TST_ENA to initiate the

Engine Running Sequence.

The control of the test sequence is documented by  use  of  a  state  pointer

variable  called "ER_STATUS".  This parameter is compared to and assigned the

value of non-modifiable calibration constants to control the progress through

the  test  sequence.  This parameter is incremented by both the sequencer and

individual test modules.

A particular test will become  active  when  ER_STATUS  reaches  a  value  of

xxx_koer_init, where "xxx" refers to a particular test such as EGR, EGO, etc.

And, when a test is complete, the test will set ER_STATUS to xxx_ER_DONE.  At

this  point,  the sequencer will set ER_STATUS to yyy_koer_init, where yyy is

the next test in the sequence.

Some tests (A/D, look for BOO or PSPS input) will be enabled  throughout  the

test  sequence.   These  tests  will  run  when  CCM_ER_ENA  = 1 and will not

increment ER_STATUS.

Since  a  test  starts  on  xxx_ER_INIT  and  then  increments  ER_STATUS  to

xxx_ER_DONE,  each  test is independent of the preceding and following tests.

With this strucuture a test can be inserted or deleted through  a  relatively

small change in the sequencer.

DEFINITIONS

Registers:

-  DEMAND_TIMER = Self Test utility timer.

-  ER_LAM_DSD1 = LAMBSE1 value requesetd by engine running on-demand test

-  ER_LAM_DSD2 = LAMBSE2 value requesetd by engine running on-demand test

-  ER_RPM = Register that contains RPM value  requested  by  engine  running

on-demand test.

-  ER_SPARK = Required value of SAF during on-demand self test.

-  ER_STATUS = Defines current position in ER  sequence.   Control  of  this

psuedo parameter passes between the ER sequencer and individual ER tests.

|      -  KOER_TST_TMR = Amount of  time  that  key  on  engine  running  has  been

|         executing, sec.

-  NBAR = Rolling average RPM.

23-177

ENGINE RUNNING SELF TEST, ENGINE RUNNING SEQUENCE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  VSBAR = Vehicle speed, MPH.

Bit Flags:

-  CCM_ER_ENA = Flag that signals the Comprehensive  Componant  Monitors  to

run engine running on-demand test.

-  CRKFLG = Engine mode flag; 1 -> crank mode, 0 -> underspeed/run mode.

-  DEMAND_ABORT = Flag to indicate that the engine running on-demand test is

in the process of an abort.

-  DEMAND_MODE = Flag set when an On-Demand self test mode is active.

-  ER_ACTIVE = Flag to indicate that the engine running on-demand test is in

progress.

-  ER_CONDTS = Flag  indicating  whether  conditions  exits  to  run  engine

running test; 1 -> do not run KOER test.

-  ER_FUL_REQ = Flag to signal Fuel system to use  ER_LAM_DSD  as  the  open

loop lambse.

-  ER_ISC_HLD = Flag to signal the Idle Speed Control system to  freeze  ISC

duty cycle.

-  ER_ISC_REQ = Flag to signal Idle Speed Control to use value of ER_RPM for

desired RPM.

-  ER_MON = Flag indicating the engine running on-demand test is complete

-  ER_SPK_REQ = Flag to signal Spark control to output a value of ER_SPK.

-  ER_TST_ENA = Flag that signals the engine running on-demand test to run.

-  ER_TST_RDY = Flag to signal OBD-II executive that conditions to  run  the

engine running on-demand test are met.

-  IMFMFLG = Instantaneous MAF sensor failure  flag;  1  ->  sensor  out  of

range.

-  ISC_ER_ENA = Flag that signals the Idle Speed Control system  to  perform

engine running on-demand test.

-  ISC_HI_CHECK = Value for state  of  ER_STATUS  when  testing  RPM  during

engine running high speed ISC test.

-  ISC_HI_DONE = Value for state of ER_STATUS when  ISC  high  rpm  test  is

complete.

-  ISC_HI_WAIT = Value for state  of  ER_STATUS  while  waiting  for  engine

running high speed ISC test.

-  ISC_LO_CHECK = Value for ER_STATUS when performing ISC low rpm test.

-  ISC_LO_DONE = Value for ER_STATUS to indicate that ISC low  RPM  test  is

complete.

23-178

ENGINE RUNNING SELF TEST, ENGINE RUNNING SEQUENCE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  ISC_LO_WAIT = Value for ER_STATUS when waiting for ISC low rpm test.

-  MIS_ER_ENA = Flag that signals the Misfire test to perform engine running

on-demand test.

-  NDSFLG = Neutral/Drive flag; 1 -> drive.

-  OPER_RSP_CHK = ER on-demand test for BOO enabled.

-  OSM_EDF_ON = Request low speed fan during on-demand Self Test; 1  ->  fan

on.

-  PxxxMALF = Malfunction flag for code Pxxx; 1 -> a  malfunction  currently

exists for fault Pxxx.

-  UNDSP = Run/Underspeed Flag; 1 -> Underspeed (or CRANK), 0 -> Run.

Calibration Constants:

-  EGO_ER_DONE = Value for state pointer when engine  running  EGO  test  is

complete.

-  EGO_ER_INIT = Value for state of ER_STATUS to indicate engine running EGO

test is to be run.

-  EGR_ER_DONE = Value for state of ER_STATUS to indicate engine running EGR

test is complete.

-  EGR_ER_INIT = Value for state of ER_STATUS to request engine running  egr

test.

-  ER_CLEANUP = Value of ER_STATUS when "cleanup" portion of engine  running

test is in progress.

-  ER_DONE = Value of ER_STATUS when engine running test is complete.

-  ER_HI_RPM = Base value for RPM requeseted during engine running on-demand

test, RPM.

-  ER_INACTIVE = Value for state ER_STATUS when engine running test  is  not

active.

-  ER_INIT = Value for state of ER_STATUS when  engine  running  test  first

inits.

-  ER_LAM_BAS = Base  value  for  LAMBSE  requested  during  engine  running

on-demand test, unitless.

-  ER_SPARK_BAS =  Base  value  for  Spark  advance  during  engine  running

on-demand test, degrees.

-  KOER_MINMPH = Minimum vehicle speed to conduct KOER self test.

-  KOER_TST_TM = Maximum ammount of time that KOER can run, sec.

-  PURG_ER_DONE = Value for state of ER_STATUS to indicate that  the  engine

running purge test is complete.

23-179

ENGINE RUNNING SELF TEST, ENGINE RUNNING SEQUENCE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PURG_ER_INIT = Value for state of ER_STATUS to request the engine running

purge test.

-  SAIR_ER_DONE = Value for state of ER_STATUS to indicate that  the  engine

running secondary air test is complete.

-  SAIR_ER_INIT = Value for state of ER_STATUS to request the engine running

secondary air test.

-  TRLOAD = Transmission Load;

0 -> Manual Transmission, no clutch or gear switches,

forced neutral state (NDSFLG = 0),

1 -> Manual Transmission, no clutch or gear switch,

2 -> Manual Transmission, one clutch or gear switch,

3 -> Manual Transmission, both clutch and gear switches,

4 -> Auto Transmission, non-electronic, neutral drive switch,

5 -> Auto Transmission, non-electronic, neutral pressure switch

(AXOD),

6 -> Auto Transmission, electronic, PRNDL sensor - park,

reverse, neutral, overdrive, manual 1, manual 2.

-  V_ISC_HI_DLY = Time delay to allow RPM to reach minimum value for  engine

running on-demand test, seconds.

-  V_ISC_HI_MIN = Minimum RPM to pass High RPM  portion  of  engine  running

on-demand test.

-  V_ISC_LO_DLY = Time delay to allow RPM to return to  normal  idle  before

testing ISC duty cycle, seconds.

-  V_ISC_LO_TIM = Time delay to allow ISC to test at low rpm.

OTHER

-  P1001 = KOER test did not complete.

-  P1131 = Lack of EGO11 switch, EGO indicates lean fault code.

-  P1132 = Lack of EGO11 switch, EGO indicates rich fault code.

-  P1151 = Lack of EGO21 switch, EGO indicates lean fault filter.

-  P1152 = Lack of EGO21 switch, EGO indicates rich fault code.

-  P0505 = Inlet air control (Idle speed) system malfunction

-  P1501 = Vehicles speed senwor (VSS) out of range during KOER.

23-180

ENGINE RUNNING SELF TEST, ENGINE RUNNING SEQUENCE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: VR_OBDII_SEQUENCER_COM1

This diagram provides for some "bullet proofing" when none of the demand mode

tests are active (engine running, engine off, or output test mode).

DEMAND_MODE = 0 ------------------------------| OPER_RSP_CHK = 0

(None of the on-demand tests are activated   | MIS_ER_ENA = 0

by the OBD-II exec)                         | CCM_ER_ENA = 0

| ISC_ER_ENA = 0

| ER_ISC_REQ := 0

| ER_ISC_HLD = 0

| ER_SPK_REQ := 0

| ER_FUL_REQ := 0

| (discontinue all requests

|  for engine running on-

|  demand parameters.)

Check status of vehicle to be in vehicle running mode.

unconditionally ------------------------| ER_CONDTS := 0

; reset flag before testing

UNDSP = 1 ------------------------------| ER_CONDTS := 1

TRLOAD => 3 ----------------------|

|

NDSFLG = 1 -----------------------|AND -| ER_CONDTS := 1

|     | store_code(ccm,P1705)

ER_ACTIVE = 1 --------------|     |     |

|OR --|

ER_TST_ENA = 1 -------|     |

|AND -|

ER_ACTIVE = 0 --------|

VSBAR > KOER_MINMPH --------------|

|AND -| ER_CONDTS := 1

ER_ACTIVE = 1 --------------|     |     | store_code(ccm,P1501)

|OR --|     |

ER_TST_ENA = 1 -------|     |

|AND -|

ER_ACTIVE = 0 --------|

IMFMFLG = 1 ----------------------|

|

ER_ACTIVE = 0 --------------------|AND -| ER_CONDTS := 1

|     | store_code(ccm,P1101)

ER_TST_ENA = 1 -------------------|

23-181

ENGINE RUNNING SELF TEST, ENGINE RUNNING SEQUENCE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

This diagram detects if an abort has been requested  (either  by  the  OBD-II

executive  or  by  this  sequencer).   If  an  abort has been initiated, this

diagram will insure that it  is  completed  and  then  send  the  appropriate

response on the SCP interface to the diagnostic tools.

DEMAND_ABORT = 1 -----------------------|

(abort requested)                      |

|

ER_STATUS = ER_DONE --------------------|AND -| DEMAND_ABORT := 0

(abort complete)                       |     |  (Abort complete)

|     |

ER_ACTIVE = 1 --------------------------|     | ER_TST_RDY = 0

|      (Abort of engine run test in progress) |     |  (Abort done, not running

|                                             |     |   now)

|     P1001MALF = false ----------------------|     |

| ER_ACTIVE = 0

|  (No longer running test)

|

| ER_STATUS = ER_INACTIVE

|

| --- ELSE ---

DEMAND_ABORT = 1 -----------------------|     |

(Abort requested)                      |     |

|     |

ER_STATUS <> EGR_ER_INIT ---------------|     |

|AND -| ER_STATUS := ER_CLEANUP

ER_STATUS <> EGO_ER_INIT ---------------|     |  (abort of individual test

(Not in the middle of a test that can't|     |   complete, turn off any

just be bailed out of)                |     |   flags that may be set

|     |   via ER_CLEANUP.)

|     ER_STATUS <> ER_DONE -------------------|     |

|                                             |     |

ER_ACTIVE = 1 --------------------------|     |

(Abort of engine run test in progress)       |

| --- ELSE ---

|

| no action

|  (abort not requested or

|   abort requested but not

|   yet complete, business as

|   usual)

BEGIN: koer_test_timer_logic:

;to be executed once per background loop.

ER_ACTIVE = 1 --------------------------|

|

ER_TST_ENA = 1--------------------------|AND -| store_code(P1001)

|     | DEMAND_ABORT := 1

|     KOER_TST_TMR >= KOER_TST_TM ------------|     | ;test running too long

| ; abort test and store code

|

| --- ELSE ---

ER_TST_ENA = 0 -------------------------|     |

|OR --| KOER_TST_TMR := 0

ER_ACTIVE = 0 --------------------------|

END: koer_test_timer_logic:

23-182

ENGINE RUNNING SELF TEST, ENGINE RUNNING SEQUENCE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

This (long) diagram first determines if the engine off on-demand test is able

to  and  should  be  running  now.  And, once activated, the test sequence is

controlled by this diagram.

DEMAND_ABORT = 1 -----------------------|

(an abort is in progress)              |AND -| no action.

|     |  (some other test is in

ER_ACTIVE = 0 --------------------------|     |   the process of aborting,

(engine running is not the test aborting)    |   just sit tight and wait.)

|

| --- ELSE ---

DEMAND_ABORT = 0 -----------------------|     |

(Abort not in progress yet)            |     |

|AND -| DEMAND_ABORT := 1

ER_ACTIVE = 1 --------------------|     |     |  (Test no longer able to run,

(test in progress)               |     |     |   time to bail out...)

|AND -|     |

ER_TST_ENA = 0 -------------|     |           | ER_TST_RDY := 1

(test not enabled by exec) |OR --|           |  (abort in progress)

|                 |

ER_CONDTS = 1 --------------|                 | (The test was running and

; conditions not met                         |  either the OBD-II exec

:  to be in KOER                             |  cancelled the test or the

|  local conditions (engine in

|  run mode, vehicle stopped

|  were violated)

|

| --- ELSE ---

DEMAND_ABORT = 0 -----------------------|     |

|     |

ER_ACTIVE = 0 --------------------------|     |

(test not in progress)                 |AND -| ER_TST_RDY := 0

|     | ER_MON :=1

|     |  (For some strange reason,

ER_TST_ENA = 1 -------------------------|     |   the test is enabled by

(test enabled by exec)                 |     |   the exec but can not now

|     |   run. Don't start.)

ER_CONDTS = 1 --------------------------|     |  (This is not an abort

|   because the test was not

|   in progress the last

|   background loop.)

|

| --- ELSE ---

(continued on next page)

23-183

ENGINE RUNNING SELF TEST, ENGINE RUNNING SEQUENCE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

DEMAND_ABORT = 0 -----------------------|     |

(Abort not requested)                  |     |

|     |

ER_TST_ENA = 0 -------------------------|     |

(test not enabled by OBD-II exec)      |AND -| ER_STATUS := ER_INACTIVE

|     |

CRKFLG = 0 -----------------------------|     | ER_TST_RDY := 1

|     |  (Signal OBD-II exec that

UNDSP = 0 ------------------------------|     |   ER test could run if

(run mode)                                   |   enabled)

|

| ER_ACTIVE = 0

|  (test not active)

|

|  (test is not running, but

|   could be if enabled)

|

| --- ELSE ---

DEMAND_ABORT = 0 -----------------------|     |

(Abort not requested)                  |AND -| ER_STATUS := ER_INACTIVE

|     |

ER_TST_ENA = 0 -------------------------|     | ER_TST_RDY := 0

(test not enabled)                           |  (test is not currently

(note that any subsequent logic in this      |   enabled by OBD-II and is

chart will execute only if the ER is        |   not ready to run, exit.)

enabled by OBD-II exec. and is able to      |

run, or an abort is in progress.)           | ER_ACTIVE = 0

|  (test not active)

|

| --- ELSE ---

(continued on next page)

23-184

ENGINE RUNNING SELF TEST, ENGINE RUNNING SEQUENCE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|

ER_STATUS = ER_INACTIVE ----------------------| ER_STATUS := ER_INIT

(test not yet started)                       |  (setup to start test)

| ER_ACTIVE = 1

|  (test now active)

|

| ER_RPM := ER_HI_RPM

| ER_ISC_REQ := 1

|  (ER control of idle speed,

|   DSDRPM = ER_RPM)

|

| ER_SPARK := ER_SPARK_BAS

| ER_SPK_REQ :=  1

|  (Request ER_SPARK

|  from Spark Strategy)

|

| ER_LAM_DSD1 := ER_LAM_BAS

| ER_LAM_DSD2 := ER_LAM_BAS

| ER_FUL_REQ := 1

|  (Request ER_LAM_DSD

|   from fuel strategy)

|

| OSM_EDF_ON = 1

|  (request low speed fan to

|   avoid overheating but

|   remain at a constant load)

|

| DEMAND_TIMER := 0

|  (reset for next user)

|

| KOER_TST_TMR := 0

|  ; clear test limit timer

|

|                                                   | Do: erase_code(P1001)

|                                                   |  ; clear test time-out code

|                                                   |

| --- ELSE ---

(continued on next page)

23-185

ENGINE RUNNING SELF TEST, ENGINE RUNNING SEQUENCE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|

ER_STATUS = ER_INIT --------------------------| ER_STATUS := ISC_HI_WAIT

|

| CCM_ER_ENA = 1

|  (Enable CCM test)

|

| --- ELSE ---

ER_STATUS = ISC_HI_WAIT ----------------|     |

|AND -| ER_STATUS := ISC_HI_CHECK

DEMAND_TIMER > V_ISC_HI_DLY ------------|     |

(enough time has passed for ISC to ramp up)  |  (Check RPM, if ok, continue

|   with test. Else exit.)

|

| --- ELSE ---

|

ER_STATUS = ISC_HI_WAIT ----------------------| ER_STATUS := ISC_HI_WAIT

| (Just sit tight, wait for

|  time delay, allow

|  DEMAND_TIMER to run)

|

| --- ELSE ---

ER_STATUS = ISC_HI_CHECK ---------------|     |

|AND -| ER_STATUS := ISC_HI_DONE

NBAR > V_ISC_HI_MIN --------------------|     | (Continue with ER test.)

|

| --- ELSE ---

|

ER_STATUS = ISC_HI_CHECK ---------------------| ER_STATUS := ER_CLEANUP

(RPM did not come up OK)                     | (Bail out time.)

|

| store_code(P0505)

|  (set code indicating

|   inadequate ISC)

|

| --- ELSE ---

|

ER_STATUS = ISC_HI_DONE ----------------------| ER_STATUS := EGR_ER_INIT

|

| DEMAND_TIMER := 0

|  (reset for next user)

|

| --- ELSE ---

(continued on next page)

23-186

ENGINE RUNNING SELF TEST, ENGINE RUNNING SEQUENCE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|

ER_STATUS = EGR_ER_DONE ----------------------| ER_STATUS := ISC_HI_WAIT2

|

| DEMAND_TIMER := 0

|  (reset for next user)

|

| ER_RPM := ER_HI_RPM

| ER_ISC_REQ := 1

|  (ER control of idle speed,

|   DSDRPM = ER_RPM)

|

| --- ELSE ---

ER_STATUS = ISC_HI_WAIT2 ---------------|     |

|AND -| ER_STATUS := EGO_ER_INIT

DEMAND_TIMER > V_ISC_HI_DL2 ------------|     |

(enough time has passed for ISC settle)      |  (RPM has had a chance to

|   settle after EGR test.)

|

| ER_ISC_HLD = 1

| (Freeze ISC duty cycle)

|

| ER_LAM_DSD1 := ER_LAM_BAS

| ER_LAM_DSD2 := ER_LAM_BAS

|

| DEMAND_TIMER := 0

|  (reset for next user)

|

| --- ELSE ---

ER_STATUS = EGO_ER_DONE ----------------|     |

|     |

P1131MALF = 0 --------------------------|     |

|     |

P1132MALF = 0 --------------------------|AND -| ER_STATUS := SAIR_ER_INIT

|     | ER_LAM_DSD1 := ER_LAM_BAS

P1151MALF = 0 --------------------------|     | ER_LAM_DSD2 := ER_LAM_BAS

|     | ER_FUL_REQ := 1

P1152MALF = 0 --------------------------|     |  (Request ER_LAM_DSD

(no upstream HEGO faults)                    |   from fuel strategy)

| ER_ISC_HLD := 0

| DEMAND_TIMER := 0

|  (reset for next user)

|

| MIS_ER_ENA = 1

|  (Enable misfire test)

|

| --- ELSE ---

|

(continued on next page)

23-187

ENGINE RUNNING SELF TEST, ENGINE RUNNING SEQUENCE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

ER_STATUS = EGO_ER_DONE ----------------------| ER_STATUS := PURG_ER_DONE

(ego fault detected)                         |  ;skip secondary air and

|  ; purge tests in the

|  ; event of an EGO

|  ; failure)

|

| DEMAND_TIMER := 0

|  (reset for next user)

|

| MIS_ER_ENA = 1

|  (Enable misfire test)

|

| --- ELSE ---

|

ER_STATUS = SAIR_ER_DONE ---------------------| ER_STATUS := PURG_ER_INIT

|

| ER_ISC_REQ := 0

| ER_FUL_REQ := 0

|

| DEMAND_TIMER := 0

|  (set up for time delay)

|

| --- ELSE ---

|

ER_STATUS = PURG_ER_DONE ---------------------| ER_STATUS := ISC_LO_WAIT

|

| DEMAND_TIMER := 0

|  (set up for time delay)

| ER_ISC_HLD := 0

| ER_ISC_REQ := 0

| ER_SPK_REQ := 0

| ER_FUL_REQ := 0

|  (normal ISC control)

|

| --- ELSE ---

(continued on next page)

23-188

ENGINE RUNNING SELF TEST, ENGINE RUNNING SEQUENCE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

ER_STATUS = ISC_LO_WAIT ----------------|     |

|AND -| ER_STATUS := ISC_LO_CHECK

DEMAND_TIMER > V_ISC_LO_DLY ------------|     |

(enough time has passed for ISC to           | DEMAND_TIMER := 0

ramp down)                                  |  (reset for next user)

|

| --- ELSE ---

|

ER_STATUS = ISC_LO_WAIT ----------------------| ER_STATUS := ISC_LO_WAIT

| (Just sit tight, wait for

|  time delay, allow

|  DEMAND_TIMER to run)

|

| --- ELSE ---

ER_STATUS =  ISC_LO_CHECK --------------|     |

|AND -| ER_STATUS := ISC_LO_DONE

DEMAND_TIMER > V_ISC_LO_TIM ------------|     | (Continue with ER test.)

(enough time has passed for ISC to           |

test)                                       | ISC_ER_ENA := 0

|  (done testing ISC duty

|   cycle)

|

| --- ELSE ---

|

ER_STATUS = ISC_LO_CHECK ---------------------| ER_STATUS := ISC_LO_CHECK

|  (wait for test to run)

| ISC_ER_ENA := 1

|  (test ISC duty cycle)

|

| --- ELSE ---

|

ER_STATUS = ISC_LO_DONE ----------------------| ER_STATUS := ER_CLEANUP

|

| ISC_ER_ENA = 0

|  (End ISC test)

|

| OPER_RSP_CHK = 1

|  (Check BOO, PSPS, etc.

|   on/off flags)

|

| --- ELSE ---

(continued on next page)

23-189

ENGINE RUNNING SELF TEST, ENGINE RUNNING SEQUENCE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|

ER_STATUS = ER_CLEANUP -----------------------| ER_STATUS := ER_DONE

|

| OPER_RSP_CHK = 0

|  (End check of BOO, etc.

|   on/off flags)

|

| MIS_ER_ENA = 0

|  (End misfire test)

|

| CCM_ER_ENA = 0

|  (End CCM test)

|

| ISC_ER_ENA = 0

|  (End ISC test)

|

| ER_ISC_REQ := 0

|  (end  control of idle speed)

|

| ER_ISC_HLD = 0

| (Unfreeze ISC duty cycle)

|

| ER_SPK_REQ := 0

|  (End request for ER_SPARK

|  from Spark Strategy)

|

| ER_FUL_REQ := 0

|  (End request for

|   ER_LAM_DSD from fuel

|   strategy)

|

| OSM_EDF_ON = 0

|  (End request for

|   low speed fan)

|

| --- ELSE ---

|

ER_STATUS = ER_DONE --------------------------| ER_STATUS = ER_INACTIVE

|  (done with test)

|

| ER_MON = 1

|  (signal exec, monitor

|   complete)

|

|                                                   | DEMAND_ABORT := 0

|                                                   |  ; make sure DEMAND_ABORT

|                                                   |  ; is reset if KOER timer

|                                                   |  ; causes abort.

|                                                   |

| ER_ACTIVE = 0

|  (No longer running test)

|

| --- ELSE ----

|

| (no action, ER_STATUS

|  under control of a test)

23-190

CHAPTER 24

AIR CHARGE CALCULATION

24-1

AIR CHARGE CALCULATION

24.1  FEATURE:  AIR CHARGE - V3.1_AIR_CHARGE_97_IMRC (CDAM0)

24.1.1  AIRMETER OVERVIEW (CDAM0)

OVERVIEW

Shown  below  is  the  relationship  between  modules  used  in   calculating

aircharge.

The Air Charge Background Calculation module,  executed  in  the  background,

calculates  variables which do not change rapidly such as those based on RPM,

ECT, ACT, etc.  The Air Meter Background FMEM module is also executed in  the

background  to set relevant flags in case of faulty air meter.  (In the OBDII

strategy, self tests are also carried out in this module.)

In the foreground, the SAMPLE AIR METER and the  Air  Meter  Foreground  FMEM

modules  are  executed.  Depending on the setting of the switch AIR_DOWN_CAL,

the  SAMPLE  AIRMETER  module  will  trigger  the  rest  of   the   aircharge

calculations  on  either  the  up  or  down  edge  of PIP.  This includes the

manifold filling, anticipation, and final aircharge modules.  The final value

used  by  the  rest of the strategy is CYL_AIR_CHG.  AIR_CHG_ST is calculated

separately for use by self test.  The MFMFLG flag is also exported for use in

other modules.

24-2

AIR CHARGE CALCULATION, AIRMETER OVERVIEW - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: AIR_OVERVIEW_COM2

Foreground                           Background

|----------------------------|    |---------------------------------|

+---------------+   IMFMCTR      +---------------+

| AIRMETER FMEM |---IMFMFLG----->| AIRMETER FMEM |-->MFMFLG

|  Foreground   |-----+----------|  Background   |

+---------------+     |          +---------------+

IMFMFLG                 |

|---------------|                V_MAF_ER_FLG

|                                    |

V                                    V

+----------------+                       +---------------+

| SAMPLE         |                       | AIRCHARGE     |

AIR_CHG_ST<---| AIRMETER       |<--- AIR_CHG_LEAK -----| BACKGROUND    |

| CALCULATION    |     CYLARC_BG         | CALCULATIONS  |

+----------------+                       +---------------+

|        |                               |         |

+- AIR_CHG_PRV      |                               |         |

|   +--------- AIR_CHG_CUR                          |         |

|   |               V                               |         |

|   |    +----------------+     AIR_CHG_WOT         |         |

|   |    | MANIFOLD       |     AIR_B3_N            |         |

|   |    | FILLING        |<--- AIR_NUM_MULT  ------+         |

|   |    |                |     MAP_AIR_0                     |

|   |    +----------------+     ARCHFLG                       |

|   |            |              AIR_BG_FIL                    |

+----|---|-------- AIR_CHG           AIR_BF_FLG                    |

|    |   |         AIR_K_OLD                                       |

|    |   |            V                                            |

|    |   |    +----------------+                                   |

|    |   +--->| ANTICIPATION   |                                   |

|    +------->|                |                                   |

|    |        +----------------+                                   |

|    |                |                                            |

|    |             air_ant                                         |

|    |                V                                            |

|    |        +----------------+                                   |

+----|------->| FINAL          |                                   |

|        | AIRCHARGE      |<--- AIR_WOT_ANT  -----------------+

+------->|                |     AIR_CHG_WOT

+----------------+

|

CYL_AIR_CHG

|

V

24-3

AIR CHARGE CALCULATION, SAMPLE AIRMETER CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

24.1.2  SAMPLE AIRMETER CALCULATION (CDAG0)

OVERVIEW

This module samples the signal produced  by  the  mass  air  flow  meter  and

converts  the flow rate data to air mass.  The air meter signal is sampled at

each PIP edge, to obtain data when  the  engine  is  running,  and  once  per

background  loop,  to  obtain  data when the engine is not running.  Sampling

once per background loop does not interfere with the PIP interrupt sampling.

DEFINITIONS

INPUTS

Registers:

-  AIR_CHG_COMP =  Un-metered  air  charge  past  the  throttle,  lbm/intake

stroke.

-  AIR_CHG_CUR = Air meter air charge, lbm/intake stroke.

-  AIR_CHG_ST = Air charge measured at  the  air  meter,  including  leaking

compensation, lbm/intake stroke.

-  AIR_PIP_CNT = Counter incremented every PIP edge to determine if the  MAF

integration is valid, PIP edges.

-  AM_INT_CUR_X = Integrated air mass computed at the current PIP edge, lbm.

-  AM_INT_PRV_X = Integrated air mass computed at  the  previous  PIP  edge,

lbm.

-  CYLARC_BG = Background-calculated cylinder air charge, lbm/intake stroke.

-  DATA_TIME = Time of current PIP edge, clock ticks.

-  IMAF = Sampled air mass flow rate, counts.

-  INTM = Time interval between the current air  mass  integration  and  the

previous air mass integration, ticks.

-  MAF = Sampled air mass flow rate, lbm/tick.

-  MAF_TIME = Time of current air meter sample, clock ticks.

-  TSLPIP = Amount of time which has passed since the  last  PIP  interrupt,

msec.

Bit Flags:

-  BGACFLG = Flag indicating cylinder air charge is background-derived; 1 ->

background-derived, unitless.

24-4

AIR CHARGE CALCULATION, SAMPLE AIRMETER CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PIP_HIGH  =  State  of  PIP  transition;  1  ->  low-to-high  transition,

unitless.

24-5

AIR CHARGE CALCULATION, SAMPLE AIRMETER CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  AIR_DOWN_CAL = Switch indicating when to perform the cylinder air  charge

calculation;  0  -> calculate on PIP up edges, 1 -> calculate on PIP down

edges, unitless.

-  FN036I(IMAF) = Air meter transfer  function  (A/D  counts  to  lbm/tick),

lbm/tick.

OUTPUTS

Registers:

-  AIR_CHG_CUR = See above.

-  AIR_CHG_PRV = Air meter air charge last PIP period, lbm/intake stroke.

-  AIR_CHG_ST = See above.

-  AIR_PIP_CNT = See above.

-  AM_INT_CUR_X = See above.

-  AM_INT_PRV_X = See above.

-  IMAF = See above.

-  INTM = See above.

-  MAF = see above.

-  MAF_TIME = See above.

24-6

AIR CHARGE CALCULATION, SAMPLE AIRMETER CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: AIR_SAMPLE_METER_COM5

BEGIN: air_sample_main

;This process is called at each PIP edge interrupt (both low-to-high and

;high-to-low transitions).

Perform an A-to-D conversion on the air meter signal.

unconditionally --------------------| IMAF := A-to-D measurement of air mass

|         flow

|

| MAF := FN036I(IMAF)

Integrate, using the sampled value of air flow rate,  to  determine  the  air

mass which passed through the air meter during the current half PIP period.

unconditionally --------------------| INTM := DATA_TIME - MAF_TIME(old)

| MAF_TIME := DATA_TIME

|

| AM_INT_PRV_X := AM_INT_CUR_X(old)

|  ;previous integrated air mass value

|

| AM_INT_CUR_X := MAF * INTM

|  ;current integrated air mass value

Increment the PIP edge counter.

unconditionally --------------------| AIR_PIP_CNT := AIR_PIP_CNT + 1

|  ;clip at 255

24-7

AIR CHARGE CALCULATION, SAMPLE AIRMETER CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Sum the current and previous integrated air mass values to determine the  air

mass  which  passed through the air meter during the current full PIP period.

This summation may be performed on either PIP up edges or down edges.

AIR_PIP_CNT >= 3 -------------|

;integration ok              |

|

AIR_DOWN_CAL = 0 -------------|AND -| Do: air_charge_calculation

;calculate on up edges       |     |  ;up edge timing

|     |

PIP_HIGH = 1 -----------------|     |

| --- ELSE ---

AIR_PIP_CNT >= 3 -------------|     |

;integration ok              |     |

|     |

AIR_DOWN_CAL = 1 -------------|AND -| Do: air_charge_calculation

;calculate on down edges     |     |  ;down edge timing

|     |

PIP_HIGH = 0 -----------------|     |

| --- ELSE ---

|

| NO ACTION

END: air_sample_main

24-8

AIR CHARGE CALCULATION, SAMPLE AIRMETER CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: air_charge_calculation

;This process is executed only when explicitly called.

Sum the previous half PIP period air mass, the current half  PIP  period  air

mass, and the estimated unmetered air mass.

unconditionally --------------------| AIR_CHG_ST := AM_INT_PRV_X +

|               AM_INT_CUR_X +

|               AIR_CHG_COMP

BGACFLG = 1 ------------------------| AIR_CHG_PRV := CYLARC_BG

;use background air charge         | AIR_CHG_CUR := CYLARC_BG

|

| --- ELSE ---

|

| AIR_CHG_PRV := AIR_CHG_CUR(old)

| AIR_CHG_CUR := AIR_CHG_ST

END: air_charge_calculation

BEGIN: air_sample_bg

;This process is executed once per background loop.

The mass air flow sensor signal is sampled here to ensure that  accurate  air

flow  data  is  available  in  the  absence of PIP interrupts (e.g.  when the

engine is not running).  This is  especially  important  for  diagnostic  and

service related functions.

Perform an A-to-D conversion on the air meter signal.

TSLPIP > 1000 ----------------------| IMAF := A-to-D measurement of air mass

;more than 1 sec since             |         flow

;last PIP interrupt                |

| MAF := FN036I(IMAF)

END: air_sample_bg

24-9

AIR CHARGE CALCULATION, MAF FMEM - FOREGROUND - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

24.1.3  MAF SENSOR FMEM - FOREGROUND (CDAM0)

OVERVIEW

During the analog to digital conversion of the air meter signal (at each  PIP

interrupt),  a  quick  check  is made to determine if the air meter signal is

within acceptable limits.  If the digital value  is  outside  the  acceptable

range,  the foreground FMEM flag, IMFMFLG, is set.  Although this flag is set

in foreground, it is cleared only in the background, when a counter, IMFMCTR,

is decremented from MFMHYS to zero.

DEFINITIONS

INPUTS

Registers:

-  IMAF = Raw A/D value read at the MAF sensor, counts.

-  N = Engine speed, rpm.

Bit Flags:

-  MFMFLG = Flag indicating that MAF sensor has failed, set in background.

Calibration Constants:

-  MAF_HI_ER = Maximum MAF sensor reading, counts.

-  MAF_LO_ER = Minimum MAF sensor reading, counts.

-  MAF_RPM_MAX = Maximum RPM for checking MAF sensor high limit.

-  MFMHYS = Calibration for IMFMFLG background loop hysteresis, counts.

OUTPUTS

Registers:

-  IMFMCTR = Provides  hysteresis  for  IMFMFLG,  background  loop  counter,

counts.

-  IMAF = See above.

Bit Flags:

-  IMFMFLG  =  Instantaneous  mass  air  flow  sensor  FMEM  flag,  set   in

foreground.

24-10

AIR CHARGE CALCULATION, MAF FMEM - FOREGROUND - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: AIR_FMEM_OBDII_FG_COM1

This logic is performed during the MAF sensor  engineering  units  conversion

(PIP interrupt) and checks if the sensor reading is within limits.

MFMFLG = 1 ------------------------|

(Failure set in background)       |

|

IMAF < MAF_LO_ER ------------------|

(Output below minimum voltage)    |OR --| IMFMFLG = 1

|     |  (Set foreground MAF failure flag)

N < MAF_RPM_MAX -------------|     |     | IMFMCTR = MFMHYS

(RPM not to high for max    |     |     |  (Start countdown counter for

voltage check (MAF_HI_ER)) |AND -|     |   FMEM exit delay)

|           |

IMAF > MAF_HI_ER ------------|           |

(Above maximum sensor voltage)          |

| --- ELSE ---

|

| (MAF Sensor output

|  within limits, no action.

|  Note: IMFMFLG is cleared in

|  logic below when IMFMCTR

|  decrements to zero.)

24-11

AIR CHARGE, MANIFOLD FILLING MODEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

24.1.4  MANIFOLD FILLING MODEL (CDAM0)

OVERVIEW

Air flow into the engine is measured at a location  upstream  of  the  intake

manifold.   At  a  given  instant in time, an air charge calculated from this

measurement may differ  considerably  from  the  air  charge  actually  being

inducted  into  the  cylinder.  The differences are caused by intake manifold

dynamics.  This logic filters the measured air charge to  account  for  these

dynamics.   The  air  charge  value produced represents the air mass inducted

into the cylinder.

A filter  coefficient  is  calculated  in  the  foreground  based  on  engine

displacement   volume,   intake   manifold   volume,  and  engine  volumetric

efficiency.  Adjustments for the effects of changes in ACT, ECT,  BP,  and  N

are made once per background loop.

Filter coefficients calculated in the background are used, or can be used, at

idle,  during  cranking,  or  when  the  air  meter signal is not reliable to

improve stability.  Use of a large filter coefficient enhances  stability  by

richening the mixture slightly when the engine speed decreases.

DEFINITIONS

Registers:

-  AIR_BG_FIL = Background-derived filter coefficient, unitless.

-  AIR_B3_N = Slope of MAP vs.  air charge, inches Hg/(lbm/intake stroke).

-  AIR_CHG = Air charge after the manifold filling model (i.e.  filtered air

charge), lbm/intake stroke.

-  AIR_CHG_CUR = Air charge measured at the air meter, lbm/intake stroke.

-  AIR_CHG_WOT = Maximum acceptable value of cylinder air charge, lbm/intake

stroke.

-  AIR_FK = Filter coefficient for the manifold filling model, unitless.

-  AIR_FK_LAST = Previous value of the filter coefficient, unitless.

-  AIR_K_OLD = Previous value of the foreground-derived filter  coefficient,

unitless.

-  AIR_NUM_MULT = ECT, ACT, and manifold volume factor for manifold filling,

inches Hg/(lbm/intake stroke).

-  MAP_AIR_0 = MAP at 0 air charge, including BP correction, inches Hg.

Bit Flags:

-  AIR_BF_FLG = Flag indicating operation in the air meter backflow  region;

1 -> in backflow region, unitless.

24-12

AIR CHARGE, MANIFOLD FILLING MODEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  ARCHFLG =  Flag  indicating  the  background-derived  filter  coefficient

should be used; 0 -> use background-derived coefficient, unitless.

Calibration Constants:

-  AIR_DEC_MUL = Filter coefficient multiplier for use during decelerations,

unitless.

-  FILFRC = Ratio of unfiltered air charge  to  filtered  air  charge  below

which the deceleration multiplier AIR_DEC_MUL is applied, unitless.

-  FILFRC_IN = Ratio of unfiltered air charge to filtered air  charge  above

which a throttle tip-in is assumed to have occurred, unitless.

Other

-  air_chg = Temporary, unclipped value of filtered air  charge,  lbm/intake

stroke.

-  air_k_base = Temporary, foreground-derived filter coefficient, unitless.

24-13

AIR CHARGE, MANIFOLD FILLING MODEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: AIR_MANIFOLD_FILL_COM3

Determine the foreground-calculated filter coefficient:

unconditionally -----------| air_k_base := AIR_NUM_MULT /

|         [AIR_B3_N + (MAP_AIR_0/AIR_CHG(old))]

Select the appropriate filter coefficient.

If a large increase in air charge at  the  flow  meter  is  detected,  switch

immediately    from    the   special   idle   filter   coefficient   to   the

foreground-calculated  coefficient.   Waiting  for  the  background  loop  to

recognize  the  change  may produce large overshoots in air charge due to the

light filtering in idle mode.

A significant decrease in the air  charge  at  the  flow  meter  indicates  a

throttle tip-out.  When this occurs, apply a tip-out multiplier to reduce the

filtering effect.

ARCHFLG = 0 --------------------------|

;background loop request for         |

;background-calculated filter        |AND --| ;idle mode or no filter

;coefficient                         |      |

|      | AIR_FK := AIR_BG_FIL

(AIR_CHG_CUR/AIR_CHG) < FILFRC_IN ----|      | AIR_FK_LAST := AIR_BG_FIL

;no large air charge increase               |

;indicative of throttle tip-in              |

| --- ELSE ---

|

(AIR_CHG_CUR/AIR_CHG) <= FILFRC -------------| ;throttle tip-out mode

;significant air charge decrease            |

;indicative of throttle tip-out             | AIR_FK := air_k_base *

|           AIR_DEC_MUL

| AIR_FK_LAST := AIR_K_OLD *

|                AIR_DEC_MUL

|

| --- ELSE ---

|

| ;normal mode

|

| AIR_FK := air_k_base

| AIR_FK_LAST := AIR_K_OLD

24-14

AIR CHARGE, MANIFOLD FILLING MODEL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calculate the filtered air charge:

AIR_FK <> 0 ----------------------|

|AND -| air_chg = ((AIR_FK/AIR_FK_LAST)

AIR_FK_LAST <> 0 -----------------|     |           * (1 - AIR_FK)

;avoid dividing by zero                |           * AIR_CHG(old))

|           + (AIR_FK * AIR_CHG_CUR)

|

| --- ELSE ---

|

| air_chg = AIR_CHG(old)

Store the value of the foreground-calculated filter coefficient  for  use  in

the  next pass so that the volumetric efficiency differences from one pass to

the next can be compensated for:

unconditionally -------------------------| AIR_K_OLD := air_k_base

Clip over-estimates of air charge caused by backflow past the air meter:

air_chg > AIR_CHG_WOT -------------|

;greater than maximum             |

;possible charge                  |AND -| AIR_CHG := AIR_CHG_WOT

|     |

AIR_BF_FLG = 1 --------------------|     |

;in backflow region                     |

| --- ELSE ---

|

| AIR_CHG := air_chg

24-15

AIR CHARGE CALCULATION, AIR CHARGE ANTICIPATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

24.1.5  AIR CHARGE ANTICIPATION (CDAM0)

OVERVIEW

To make up for fuel scheduling delays,  aircharge  anticipation  is  used  to

predict what the aircharge will be two events in the future.

Assumptions and description of anticipation equation:

-  Assumption #1

Straight line increase in AIR_CHG_CUR for next event, i+1, then stays  at

that level for the i+2 event.

throttle |         |

aircharge|         |     *     *

|         *

|   *     |

+---+-----+-----+-----+-------

time   i-1    i    i+1   i+2

AIR_CHG_CUR(i+1) = AIR_CHG_CUR(i) + AIR_CHG_CUR(i) - AIR_CHG_CUR(i-1)

AIR_CHG_CUR(i+2) = AIR_CHG_CUR(i+1)

-  Assumption #2

Filling coefficient remains the same for the next two events.

AIR_FK(i+1) = AIR_FK(i)

AIR_FK(i+2) = AIR_FK(i)

Using the manifold filling model, the equations for the current intake  event

and next two events can be written as follows:

AIR_CHG(i)   = AIR_FK * AIR_CHG_CUR(i)

+ [(AIR_FK(i)/AIR_FK(i-1) * (1-AIR_FK(i)) AIR_CHG(i-1)]

AIR_CHG(i+1) = AIR_FK(i+1) * AIR_CHG_CUR(i+1)

+ [(AIR_FK(i+1)/AIR_FK(i) * (1-AIR_FK(i+1)) AIR_CHG(i)]

AIR_CHG(i+2) = AIR_FK(i+2) * AIR_CHG_CUR(i+2)

+ [(AIR_FK(i+2)/AIR_FK(i+1) * (1-AIR_FK(i+2)) AIR_CHG(i+1)]

24-16

AIR CHARGE CALCULATION, AIR CHARGE ANTICIPATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The preceding equations are then simplified to reach the final  form  of  the

equation shown in the strategy process.  The solution is shown below.

EEC notation:

air_thr_bod = AIR_CHG_CUR(i+1) = AIR_CHG_CUR(i+2) = AIR_CHG_CUR(i)

+ AIR_CHG_CUR(i)

- AIR_CHG_CUR(i-1)

AIR_FK = AIR_FK(i)= AIR_FK(i+1) = AIR_FK(i+2)

nxt_arch_2x = AIR_CHG(i+2)

Solve the equations to obtain the aircharge two events in the future.

The filling model output is AIR_CHG(i)

|

+--------------------+

|substitute

AIR_CHG(i+1) = AIR_FK * air_thr_bod                 v

|          + [(AIR_FK/AIR_FK * (1-AIR_FK) * AIR_CHG(i)]

|

+-----------------------------------------------+

|substitute

nxt_arch_2x = AIR_FK * air_thr_bod                  V

+ [(AIR_FK/AIR_FK * (1-AIR_FK) * AIR_CHG(i+1)]

Simplify to final form:

nxt_arch_2x = [(2 - AIR_FK) * AIR_FK * air_thr_bod]

+ [(1 - AIR_FK) * (1- AIR_FK) * AIR_CHG(new)]

When AIR_K_OLD is used in the anticipation model it contains the latest value

for air_k_base.  AIR_FK may contain the idle or no filter.

nxt_arch_2x = [(2 - AIR_K_OLD) * AIR_K_OLD * air_thr_bod]

+ [(1 - AIR_K_OLD) * (1- AIR_K_OLD) * AIR_CHG(new)]

24-17

AIR CHARGE CALCULATION, AIR CHARGE ANTICIPATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  AIR_CHG_PRV = previous value of AIR_CHG_CUR

-  AIR_CHG_CUR = Aircharge from air meter calculation, lbm.

-  AIR_CHG = Aircharge value after the filling model, lbm.

-  AIR_FK = Manifold filling filter constant.

-  AIR_K_OLD = Manifold filling base coefficient derived in manifold filling

module.

Calibration Constants:

-  ANTICIPATION = Calibration switch to use predicted ARCHG; 0 -> do not use

predicted value, 1 -> use predicted value.

OUTPUTS

Registers:

-  air_ant = Anticipated aircharge, lbm.

PROCESS

STRATEGY MODULE: AIR_ANTICIPATION_COM2

air_thr_bod = AIR_CHG_CUR + AIR_CHG_CUR - AIR_CHG_PRV

nxt_arch_2x = [(2 - AIR_K_OLD) * AIR_K_OLD * air_thr_bod] +

[(1 - AIR_K_OLD) * (1- AIR_K_OLD) * AIR_CHG(new)]

ANTICIPATION = 0 ------------------------| air_ant = AIR_CHG

|

| --- ELSE ---

|

| air_ant = nxt_arch_2x

24-18

AIR CHARGE CALCULATION, FINAL CYLINDER AIRCHARGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

24.1.6  FINAL CYLINDER AIRCHARGE (CDAM0)

OVERVIEW

Several intermediate values of air charge are calculated by  EEC  air  charge

strategy before a final result is achieved.  This strategy module defines the

final value of air charge to be used by  the  entire  strategy.   This  value

represents the mass of air in the cylinder and is called CYL_AIR_CHG.

An anticipated value of cylinder air charge is often used to  compensate  for

fueling  delays.   Under  certain  conditions however, use of the anticipated

values is inappropriate.   Large  manifold  filling  overshoots  can  produce

excessive  anticipation,  both  positive  and  negative (i.e.  an anticipated

cylinder air charge greater than the maximum  possible  charge  or  a  large,

rapid  drop  in  cylinder  air  charge).  Under these conditions, alternative

values of air charge, instead of  the  anticipated  value,  are  assigned  to

CYL_AIR_CHG.   Also,  as  air  charge  approachs the maximum possible charge,

anticipation of rapidly changing conditions is no longer required.

DEFINITIONS

INPUTS

Registers:

-  AIR_CHG = Air charge  result  from  manifold  filling  model,  lbm/intake

stroke.

-  AIR_CHG_PRV = Previous value of air charge at the air  meter,  lbm/intake

stroke.

-  AIR_CHG_WOT = Maximum acceptable cylinder air charge, lbm/intake stroke.

-  AIR_WOT_ANT =  Maximum  acceptable  value  for  anticipated  air  charge,

lbm/intake stroke.

-  CYL_AIR_CHG = Current cylinder air charge, lbm/intake stroke.

OUTPUTS

Registers:

-  CYL_AIR_CHG = See above.

OTHER

-  air_ant = Anticipated future value of  cylinder  air  charge,  lbm/intake

stroke.

24-19

AIR CHARGE CALCULATION, FINAL CYLINDER AIRCHARGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: AIR_FINAL_AIRCHARGE_COM1

AIR_CHG > AIR_WOT_ANT -----------|

;charge is near the maximum     |

;possible charge                |

|AND -| CYL_AIR_CHG := AIR_CHG

AIR_CHG_PRV > AIR_CHG_WOT -------|     |  ;Do not anticipate excessively.

;manifold filling overshoot     |     |  ;Use result from manifold

|     |  ;filling model.

air_ant < CYL_AIR_CHG[old] ------|     |

;anticipating a decrease in           |

;cylinder air charge                  |

| --- ELSE ---

AIR_CHG_PRV > AIR_CHG_WOT -------|     |

;manifold filling overshoot     |AND -| CYL_AIR_CHG := CYL_AIR_CHG[old]

|     |  ;Do not anticipate excessively.

air_ant < CYL_AIR_CHG[old] ------|     |  ;Hold current value.

;anticipating a decrease in           |

;cylinder air charge                  |

| --- ELSE ---

|

AIR_CHG > AIR_WOT_ANT -----------------| CYL_AIR_CHG := AIR_CHG

;charge is near the maximum           |  ;Don't use anticipation value when

;possible charge                      |  ;close to WOT air charge.  Use

|  ;manifold filling result.

|

| --- ELSE ---

|

air_ant > AIR_WOT_ANT -----------------| CYL_AIR_CHG := AIR_WOT_ANT

;charge is near the maximum           |  ;Limit anticipation.

;possible charge                      |

| --- ELSE ---

|

| CYL_AIR_CHG := air_ant

|  ;Use anticipated value

unconditionally -----------------------| Do: FOREGROUND FUEL CALCULATION

24-20

AIR CHARGE CALCULATION, AIR CHARGE BACKGROUND CALCULATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

24.1.7  AIR CHARGE BACKGROUND CALCULATIONS (CDAM0)

OVERVIEW

The background portion of the air charge  calculations  handles  calculations

for  manifold  filling, background filtering values, anticipation and failure

modes.  Each is described where it is calculated.

DEFINITIONS

Registers:

-  ACT = Air charge temperature, deg F.

-  AIR_BG_FIL = Background-derived filling coefficient, unitless.

-  AIR_B3_N = Slope of MAP vs.  air charge, inches Hg/(lbm/intake stroke).

-  AIR_CHG = Air charge value after the manifold filling  model,  lbm/intake

stroke.

-  AIR_CHG_WOT = Maximum acceptable cylinder air charge, lbm/intake stroke.

-  AIR_NUM_MULT = ECT, ACT, and manifold volume factor for manifold filling,

inches Hg/(lbm/intake stroke).

-  AIR_WOT_ANT =  Maximum  acceptable  anticipated  air  charge,  lbm/intake

stroke.

-  ATMR1 = Time since engine start, sec.

-  BP = Barometric pressure, inches Hg.

-  CYL_AIR_CHG  =  Foreground-calculated  cylinder  air  charge,  lbm/intake

stroke.

-  CYLARC_BG = Background-calculated cylinder air charge, lbm/intake stroke.

-  ECT = Engine coolant temperature, deg F.

-  EGRACT = Actual EGR flow rate, percent.

-  INF_MAP = Inferred MAP, inches Hg.

-  INF_MVAC = Inferred manifold vacuum, inches Hg.

-  ISCFLG = Idle Speed Control mode register;  -1  ->  dashpot  mode,  0  ->

dashpot  preposition, 1 -> closed-loop RPM mode, 2 -> closed-loop lockout

mode, unitless.

-  MAP_AIR_0 = MAP at 0 air charge, including BP correction, inches Hg.

-  N = Engine speed, rpm.

24-21

AIR CHARGE CALCULATION, AIR CHARGE BACKGROUND CALCULATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TOTLDST = Total throttle-based load (normalized air charge), unitless.

-  TP_REL = Relative throttle position, counts.

-  VBAT = Battery voltage, volts.

24-22

AIR CHARGE CALCULATION, AIR CHARGE BACKGROUND CALCULATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Bit Flags:

-  AIR_BF_FLG = Flag indicating operation in the air meter backflow  region;

1 -> in backflow region, unitless.

-  AIR_CRK_INT = Flag indicating when to initialize the air registers with a

load-based  air  charge  calculation; 0 -> initialize air registers, 1 ->

registers have already been initialized, unitless.

-  ARCHFLG = Flag indicating background-derived filling  coefficient;  0  ->

background-derived, unitless.

-  ARCHGQ1_FLG  =  Flag  indicating  unacceptable  battery  voltage;  1   ->

unacceptable voltage, unitless.

-  BGACFLG = Flag indicating cylinder air charge is background-derived; 1 ->

background-derived, unitless.

-  CRKFLG = Crank mode flag; 1 -> in crank mode, unitless.

-  IMFMFLG = Mass  Air  Flow  sensor  failure  flag;  1  ->  failed  sensor,

unitless.

-  IMRC_POSN = Flag indicating actual position  of  Intake  Manifold  Runner

Control valves; 1 -> valves open, 0 -> valves closed, unitless.

-  MAF_INTP_FLG = Flag indicating an unreliable MAF signal; 1 ->  unreliable

MAF signal, unitless.

-  TFMFLG = Throttle position failure mode flag;  1  ->  failed  TP  sensor,

unitless.

-  UNDSP = Run/underspeed mode flag; 0 -> run mode, 1 -> underspeed or crank

mode, unitless.

Calibration Constants:

-  AIR_MAN_VOL = Intake manifold volume for use in manifold  filling  model,

liters.

-  AIR_STRT = Minimum time since engine start to  use  special  idle  filter

coefficient, sec.

-  ARCANTCOR = Correction factor for maximum anticipated air charge (must be

less than ARCWOTCOR), unitless.

-  ARCHIDLE_SW = Switch to enable special idle filter; 1 -> use special idle

filter.

-  ARCHSW = Manifold filling model select switch; 1 -> use filling model.

-  ARCWOTCOR = Correction factor for maximum cylinder air  charge  (must  be

greater than ARCANTCOR), unitless.

-  FKARCIDLE = Special idle mode filter coefficient, unitless.

-  FN022V(ACT or ECT) = Normalized temperature (ACT  and  ECT)  for  use  in

FN1052, unitless.

24-23

AIR CHARGE CALCULATION, AIR CHARGE BACKGROUND CALCULATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FN040A(N) = Cylinder air charge as a function of engine  speed  (for  MAF

and TP failure management), lbm/intake stroke.

-  FN044(TP_REL) = Normalized relative throttle position, unitless.

-  FN052BF(N) = Throttle position above which  air  meter  backflow  occurs,

counts.

-  FN052BP(BP) = Correction for y-intercept of MAP vs.   air  charge  curve,

unitless.

-  FN052C(ECT) = Normalized air  charge  values  for  initial  cylinder  air

charge, unitless.

-  FN052M(N) = MAP at 0 air charge, inches Hg.

-  FN052N(N) = Slope of MAP vs air charge, inches Hg/(lbm/intake stroke).

-  FN059(ACT)     =     Volumetric     efficiency     correction     factor,

SQRT((ACT+460)/560), unitless.

-  FN059T(ACT)  =  Density  correction  factor  for  cylinder  air   charge,

unitless.

-  FN070E(N) = Normalized engine speed, unitless.

-  FN1036A(N,TP_REL) = Normalized engine air charge at sea  level,  with  no

EGR, and IMRC valves closed, unitless.

X = FN070E(N)

Y = FN044(TP_REL)

-  FN1036B(N,TP_REL) = Normalized engine air charge at sea  level,  with  no

EGR, and IMRC valves open, unitless.

X = FN070E(N)

Y = FN044(TP_REL)

-  FN1052(ACT,ECT) =  Volumetric  efficiency  correction  for  the  manifold

filling model, unitless.

X = FN022V(ACT)

Y = FN022V(ECT)

-  MAPPEREGR = Inferred MAP correction factor for ACT sensor location;

MAPPEREGR = 1.00 for ACT measured in the intake manifold,

MAPPEREGR = 1.25 for ACT measured in the air cleaner.

-  SARCHG = Standard Air Charge, lbm/intake stroke.

= 4.4256 * 10e-5 * (engine size in cubic inches) /

(number of cylinders)

-  TPBKFLOWH = Throttle position hysteresis for air meter  backflow  region,

counts.

-  VBMAFMIN = Minimum acceptable battery  voltage  for  reliable  air  meter

readings, volts.

-  VBMAF_CL = Acceptable battery voltage at which to re-enable  use  of  air

meter readings, volts.

24-24

AIR CHARGE CALCULATION, AIR CHARGE BACKGROUND CALCULATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OTHER

-  adj_fact = Adjustment factor to account for variations in ACT,  ECT,  and

BP, unitless.

-  air_wot = Maximum cylinder air charge, lbm/intake stroke.

-  throttle_load = Normalized engine air charge, unitless.

24-25

AIR CHARGE CALCULATION, AIR CHARGE BACKGROUND CALCULATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: AIR_AIRMETER_BG_COM3

BEGIN: air_background_main

;This process is executed once per background loop.

These are the values required in  the  foreground  portion  of  the  manifold

filling,  anticipation,  air  meter air charge, and final cylinder air charge

modules.

Choose the appropriate throttle-based load.

IMRC_POSN = 1 -----| throttle_load = FN1036B(N,top row)

|

| --- ELSE ---

|

| throttle_load = FN1036A(N,top row)

unconditionally ---| adj_fact = FN1052(ACT,ECT) * (BP/29.875) *

|            FN059T(ACT)

|

| air_wot = throttle_load * SARCHG * adj_fact

|

| AIR_WOT_ANT = air_wot * ARCANTCOR

| AIR_CHG_WOT = air_wot * ARCWOTCOR

| AIR_NUM_MULT = (11781.34/AIR_MAN_VOL) *

|                FN1052(ACT,ECT)

| AIR_B3_N = FN052N(N)

| MAP_AIR_0 = FN052M(N) * FN052BP(BP)

| INF_MAP = MAP_AIR_0 + AIR_B3_N * AIR_CHG

|           * (1 + MAPPEREGR * EGRACT/100) /

|                              FN059(ACT)

| INF_MVAC = BP - INF_MAP

24-26

AIR CHARGE CALCULATION, AIR CHARGE BACKGROUND CALCULATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

During low speed operation on some four cylinder applications, the air  meter

will  over-estimate  the  engine  air  flow due to backflow at the air meter.

Under these conditions, the air charge will be estimated based on  the  total

inferred air charge.

TP_REL >= FN052BF(N)-----------------| AIR_BF_FLG = 1

|

| --- ELSE ---

|

TP_REL < (FN052BF(N) - TPBKFLOWH) ---| AIR_BF_FLG = 0

24-27

AIR CHARGE CALCULATION, AIR CHARGE BACKGROUND CALCULATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

At  low  system  voltages,  the  air  meter  looses  accuracy.   Under  these

conditions, it is preferable to use a failure mode value for air charge.  The

meter can however be used to some degree  even  at  low  system  voltages  to

measure low air mass flow rates (e.g.  during cranking).

VBAT < VBMAFMIN ---------------|

(low battery voltage)         |

|AND -|S Q -| ARCHGQ1_FLG = 1

|     |

UNDSP = 0 ---------------------|     |

(not in crank or underspeed mode)   |

|

VBAT > VBMAF_CL ---------------------|C

IMFMFLG = 1 -------------------------|

(air meter failed)                  |

|

ARCHGQ1_FLG = 1 ---------------------|OR --| MAF_INTP_FLG = 1

(low battery voltage)               |     |  (MAF signal unreliable)

|     |

AIR_BF_FLG = 1 ----------------------|     | --- ELSE ---

(in backflow region)                      |

|

| MAF_INTP_FLG = 0

|  (MAF signal OK)

24-28

AIR CHARGE CALCULATION, AIR CHARGE BACKGROUND CALCULATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

If the air meter signal is not reliable, the value of cylinder air charge  is

based  on the total inferred air charge (primarily a function of engine speed

and throttle position).  If signals from both the  TP  and  MAF  sensors  are

unreliable,  air  charge  is  estimated  as  a function of engine speed only.

Calculated manifold filling coefficients are  used  only  if  the  air  meter

signal is good.  When the background-derived value of the air charge is used,

the ARCHFLG flag disables manifold filling by setting the filling coefficient

to 1.

Initialization  of  air  charge  at  start-up  is  of  special  interest.   A

calibratable  load,  FN052C(ECT),  is converted to air charge.  This value is

used until crank mode is exited.  After AIR_STRT seconds the idle  filter  is

used to enhance stability.

CRKFLG = 1 --------------------------|

|AND -| CYLARC_BG = FN052C(ECT) *

AIR_CRK_INT = 0 ---------------------|     |   SARCHG * (BP/29.875)

| AIR_CHG = CYLARC_BG

| CYL_AIR_CHG = CYLARC_BG

| BGACFLG = 1

| AIR_CRK_INT = 1

|

| (crank mode initializer)

|

| --- ELSE ---

|

MAF_INTP_FLG = 0 --------------------------| CYLARC_BG = CYL_AIR_CHG

(airmeter OK)                             | BGACFLG = 0

|

| --- ELSE ---

|

TFMFLG = 0 --------------------------------| CYLARC_BG = TOTLDST * SARCHG

(TP sensor okay)                          |             * adj_fact

| BGACFLG = 1

|

| --- ELSE ---

|

| CYLARC_BG = FN040A(N)

|             * adj_fact

| BGACFLG = 1

24-29

AIR CHARGE CALCULATION, AIR CHARGE BACKGROUND CALCULATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

When the flag BGACFLG is set,  the  manifold  filling  model  is  essentially

disabled.   Filtering  coefficients  are  then assigned so that air charge is

effectively derived directly from the measurement made at the air meter.  The

calibration  parameter  ARCHSW  provides the ability to disable the model for

testing purposes.

CRKFLG = 1 --------------------------------| AIR_BG_FIL = 0

| ARCHFLG = 0

|  (use special crank filter)

|

| --- ELSE ---

ARCHSW = 0 --------------------------|     |

|OR --| AIR_BG_FIL = 1

BGACFLG = 1 -------------------------|     | ARCHFLG = 0

(no filter)                               |  (use BG filter)

|

| --- ELSE ---

ISCFLG >= 1 -------------------------|     |

|     | AIR_BG_FIL = FKARCIDLE

ARCHIDLE_SW = 1 ---------------------|AND -| ARCHFLG = 0

|     |  (use special idle filter)

ATMR1 > AIR_STRT --------------------|     |

| --- ELSE ---

|

| AIR_BG_FIL = NO CHANGE

| ARCHFLG = 1

unconditionally ---------------------------| Do: AIR LEAKAGE CALCULATION

|

| Do: air_mafs_diag_select

|  ;check for sensor failures

|  ;

|  ;this calls a process external

|  ;to this module

END: air_background_main

24-30

AIR CHARGE CALCULATION, AIR LEAKAGE CALCULATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

24.1.8  AIR LEAKAGE CALCULATION (CDAM0)

OVERVIEW

This module calculates an air charge offset, AIR_CHG_COMP, due  to  unmetered

air  flow.   The calibration constant ARCHLK is intended to account for flows

due to PCV, CANP, EVR solenoid, etc.  The term AIR_BRK is intended to account

for  air  flow  which  occurs  when  vacuum-assisted  brakes  are  applied or

released.  FN123A and FN123E compensate for air meter drifts which  correlate

with changes in ambient and engine coolant temperatures, respectively.

DEFINITIONS

Registers:

-  ACT = Air charge temperature, deg F.

-  AIR_BRK = Air flow rate from brake booster to intake manifold, lbm/min.

-  AIR_CHG_COMP = Total unmetered air charge, lbm.

-  AIR_CHG_CUR = Air charge measured at MAF sensor, lbm.

-  ECT = Engine coolant temperature, deg F.

-  N = Engine Speed, rpm.

-  PG_AIR = Air flow from canister purge system, lbm/min.

-  PG_DC = Canister purge duty cycle, unitless.

Bit Flags:

-  PGM_RUNNING = Flag indicating that the purge monitor test is  running;  1

-> test running.

Calibration Constants:

-  ARCHLK = Air flow leakage, lbm/min.

-  ENGCYL = Number of intake strokes per engine revolution.

-  FN123A(ACT) = Air meter correction for air charge temperature, unitless.

-  FN123E(ECT) =  Air  meter  correction  for  engine  coolant  temperature,

unitless.

-  PG_DC_OF = Canister purge duty cycle offset, unitless.

-  PG_LK_ML = Canister purge valve leakage multiplier, lbm/min.

OTHER

24-31

AIR CHARGE CALCULATION, AIR LEAKAGE CALCULATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  pg_dc = Purge duty cycle, unitless.

-  temp_air = Unclipped, temporary value of air flow rate from brake booster

to intake manifold, lbm/min.

-  temp_chg = Air charge measured by the MAF sensor, lbm.

24-32

AIR CHARGE CALCULATION, AIR LEAKAGE CALCULATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: AIR_AIRLEAK_COM4

Determine the air flow rate from the brake booster  to  the  intake  manifold

when the brakes are applied or released.

unconditionally -----------| Do: air_compensation_brake

|  ;this calls a process external

|  ;to this module

Estimate the flow of air from the canister purge system.

PGM_RUNNING = 1 -----------| PG_AIR := 0

;purge monitor is         |

;running                  | --- ELSE ---

|

| pg_dc := max(0, (PG_DC - PG_DC_OF))

| PG_AIR := pg_dc * PG_LK_ML

|  ;flow from canister purge

Determine the air charge compensation necessary to account for all

unmetered air flows.

always --------------------| temp_chg := AIR_CHG_CUR - AIR_CHG_COMP

|  ;estimate of MAF sensor-measured charge

|

| AIR_CHG_COMP := (ARCHLK + AIR_BRK + PG_AIR)/

|                 (N * ENGCYL) +

|                 (FN123A(ACT) + FN123E(ECT))*

|                 temp_chg

|  ;total unmetered air charge

24-33

AIR CHARGE CALCULATION, BRAKE COMPENSATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

24.1.9  BRAKE COMPENSATION (CDAM0)

OVERVIEW

+-----------------------------+

|AIR_COMP_BRAKE               |

+-----------------------------+

|PUBLIC PROCEDURES:           |

|. air_compensation_brake     |

+-----------------------------+

|PRIVATE PROCEDURES:          |

|. NONE                       |

+-----------------------------+

This module estimates the air flow rate from the power brake booster  to  the

intake  manifold.   Vacuum-assisted  power brakes make use of intake manifold

vacuum to increase braking force, but because  the  connections  between  the

brake  booster  and  the  manifold  are  downstream  of the engine's air flow

sensor, the flow into the manifold is never measured.

To compensate  for  this  disturbance  in  engine  air  flow,  an  open-loop,

feedforward  estimate  of  the  flow  from  the brake booster is computed and

included along with the measured air flow.   AIR_BRK  (in  lbm/min)  is  this

estimate.

Each time the brake is applied or released, air flows from the brake  booster

to  the  manifold.   The  logic below provides the flexibility to specify the

initial air flow, the linear rate at which the flow decreases over time,  and

the  maximum  duration  of  the flow.  The flow can be calibrated to decay to

zero before the maximum duration period.

|     initial

|      flow    ____

|             |    |____     decay rate

|             |         |____

flow  |             |              |____

rate  |             |                   |____

|             |                        |____

|             |                             |

|             |                             |

0| ____________|                             |___________

+--------------------------------------------------------- time

max.

|<--------- duration -------->|

24-34

AIR CHARGE CALCULATION, BRAKE COMPENSATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Assumptions:

(no data available)

Flow rate

Minimum:  0 lbm/min

Maximum:  0.1 lbm/min

Resolution:  0.0001 lbm/min

Decay rate

Minimum:  0 lbm/min/sec

Maximum:  1.0 lbm/min/sec

Resolution:  0.001 lbm/min/sec

Duration

Minimum:  0 sec

Maximum:  5 sec

Resolution:  0.001 sec

DEFINITIONS

Registers:

-  AIR_BRK = Air flow rate from brake booster to intake manifold, lbm/min.

-  AIR_BRK_ET = Elapsed time of brake event, sec.

-  AIR_TM_OLD = Time at which previous compensation  calculation  was  made,

sec.

Bit Flags:

-  AIR_BIFLGLST = Flag indicating  state  of  BIFLG  on  the  previous  pass

through this routine, unitless.

-  AIR_BRK_INIT = Flag indicating that  brake  compensation  variables  have

been initialized; 0 -> not initialized, 1 -> initialized, unitless.

-  BIFLG = Flag indicating state of brake; 0  ->  released,  1  ->  applied,

unitless.

-  CRKFLG = Engine operating mode flag; 0 -> not in  Crank  mode,  1  ->  in

Crank mode, unitless.

Calibration Constants:

-  AIR_BRK_0 = Initial air flow rate from brake booster to intake  manifold,

lbm/min.

-  AIR_BRK_DCY = Desired decay rate of brake booster air flow, lbm/min/sec.

-  AIR_BRK_DUR = Desired duration of brake booster compensation event, sec.

-  AIR_BRK_SW = Switch which enables brake booster air charge  compensation;

0 -> no compensation desired, 1 -> compensation desired, unitless.

24-35

AIR CHARGE CALCULATION, BRAKE COMPENSATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  BIHP = Switch indicating presence of brake switch hardware; 1 -> present,

unitless.

Other

-  air_brk_tmp = Unclipped, temporary value of  air  flow  rate  from  brake

booster to intake manifold, lbm/min.

-  air_time = Time at which current compensation calculation is made, sec.

-  ticks_to_sec = Clock ticks to seconds  conversion  factor,  seconds/clock

tick.

24-36

AIR CHARGE CALCULATION, BRAKE COMPENSATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: AIR_COMP_BRAKE_COM1

BEGIN: air_compensation_brake

;This process is executed only when explicitly called.

Estimate the air flow rate  from  the  power  brake  booster  to  the  intake

manifold when the brakes are applied or released.

unconditionally -------------------| air_time := clock * ticks_to_sec

|  ;timestamp, in seconds, for use later

AIR_BRK_SW = 0 --------------|

;compensation not desired   |

|

BIHP <> 1 -------------------|OR --| air_brk_tmp := 0.0

;brake switch not present   |     |  ;either can't determine flow or

|     |  ;don't need to, so estimate of air

CRKFLG = 1 ------------------|     |  ;flow rate is zero

;engine not at stable speed,      | AIR_BRK_ET := 0.0

;don't complicate air             |  ;reset elapsed time

;charge calculation               | AIR_BRK_INIT := 0

|  ;indicate that variables need to

|  ;be initialized

|

| --- ELSE ---

|

BIFLG <> AIR_BIFLGLST -------------| ;brake just applied or released,

;state of brake changed           | ;initialize variables

;since last pass                  |

| air_brk_tmp := AIR_BRK_0

|  ;initial flow rate

| AIR_BRK_ET := 0.0

|  ;reset elapsed time

| AIR_TM_OLD := air_time

|  ;time at which estimate was

|  ;calculated

| AIR_BRK_INIT := 1

|  ;indicate that variables have been

|  ;initialized

|

| --- ELSE ---

|

(continued on next page)

24-37

AIR CHARGE CALCULATION, BRAKE COMPENSATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|

AIR_BRK_INIT = 1 ------------|     |

;flow terms have been       |     |

;initialized                |AND -| ;air already flowing, estimate

|     | ;flow rate

AIR_BRK_ET < AIR_BRK_DUR ----|     |

;elapsed time less than           | AIR_BRK_ET := AIR_BRK_ET +

;desired timeout period           |               (air_time - AIR_TM_OLD)

|  ;update elapsed time of event

| air_brk_tmp := AIR_BRK_0 -

|                AIR_BRK_DCY*AIR_BRK_ET

|  ;flow rate is decaying

| AIR_TM_OLD := air_time

|  ;time at which estimate was

|  ;calculated

|

| --- ELSE ---

|

| air_brk_tmp := 0.0

|  ;no flow from brake booster,

|  ;event has finished

| AIR_BRK_INIT := 0

|  ;indicate that variables need to

|  ;be initialized

unconditionally -------------------| AIR_BRK := max(0,air_brk_tmp)

|  ;clip estimate of flow rate

|  ;to a minimum of zero

|

| AIR_BIFLGLST := BIFLG

|  ;state of the brake switch

|  ;for use next pass

END: air_compensation_brake

24-38

AIR CHARGE CALCULATION, MAF SENSOR DIAGNOSTIC TEST SELECTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

24.1.10  MAF SENSOR DIAGNOSTIC TEST SELECTION (CDAM0)

OVERVIEW

+--------------------------------+

|AIR_MAFS_DIAG_SELECT            |

+--------------------------------+

|PUBLIC PROCEDURES:              |

|. air_mafs_diag_select          |

+--------------------------------+

|PRIVATE PROCEDURES:             |

|. NONE                          |

+--------------------------------+

Mass air flow sensor readings  are  tested  to  ensure  that  the  sensor  is

functioning  properly.   Under  appropriate  operating  conditions,  the  raw

digital value read from the air meter is used to  identify  "open",  "short",

and "out-of-range" failures.

DEFINITIONS

Bit Flags:

-  CCM_EO_ENA = Flag indicating the Engine-Off On-Demand  test  is  enabled,

unitless.

-  CCM_ER_ENA  =  Flag  indicating  the  Engine-Running  On-Demand  test  is

enabled, unitless.

-  CCM_TST_ENA = Flag indicating  that  Continuous  Comprehensive  Component

monitoring is enabled, unitless.

24-39

AIR CHARGE CALCULATION, MAF SENSOR DIAGNOSTIC TEST SELECTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: AIR_MAFS_DIAG_SELECT_COM1

BEGIN:  air_mafs_diag_select

;This process is executed only when explicitly called.

Only indicate a failure to the OBD-II  executive  if  diagnostic  testing  is

requested  by  the executive routine.  Otherwise, do not call OBDII executive

routines.

CCM_ER_ENA = 1 -------------------------| ;perform Engine-Running On-Demand

;Engine-Running On-Demand              | ;tests

;test enabled                          |

|

| Do: air_over_voltage_entry

| Do: air_under_voltage_er_entry

| Do: air_self_test_er_entry

| Do: air_update_fault_codes

|  ;these call processes external to

|  ;this module

|

| --- ELSE ---

|

CCM_EO_ENA = 1 -------------------------| ;perform Engine-Off On-Demand tests

;Engine-Off On-Demand test             |

;test enabled                          | Do: air_over_voltage_entry

| Do: air_self_test_eo_entry

| Do: air_update_fault_codes

|  ;these call processes external to

|  ;this module

|

|  ;note: tests for under-voltage

|  ;failure are not performed unless

|  ;the engine is running since the

|  ;sensor output is expected to be

|  ;very low

|

| --- ELSE ---

|

CCM_TST_ENA = 1 ------------------------| ;perform Continuous tests

;Continuous test enabled               |

| Do: air_over_voltage_entry

| Do: air_under_voltage_cont_entry

| Do: air_update_fault_codes

|  ;these call processes external to

|  ;this module

24-40

AIR CHARGE CALCULATION, MAF SENSOR DIAGNOSTIC TEST SELECTION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

If any failures are detected, malfunction flags are set to  indicate  to  the

rest  of  the  strategy  that  the  mass  air  flow sensor is not functioning

properly.

unconditionally ------------------------| Do: air_update_failure_flags

|  ;update failure flags used by the

|  ;rest of the strategy

|  ;

|  ;this calls a process external to

|  ;this module

END: air_mafs_diag_select

24-41

AIR CHARGE CALCULATION, MAF SENSOR DIAGNOSTIC TESTS - BACKGROUND - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

24.1.11  MAF SENSOR DIAGNOSTIC TESTS - BACKGROUND (CDAM0)

OVERVIEW

+--------------------------------+

|AIR_MAFS_DIAG_BG                |

+--------------------------------+

|PUBLIC PROCEDURES:              |

|. air_over_voltage_entry        |

|. air_under_voltage_cont_entry  |

|. air_under_voltage_er_entry    |

|. air_self_test_er_entry        |

|. air_self_test_eo_entry        |

|. air_over_voltage_test         |

|. air_under_voltage_test        |

|. air_self_test_er              |

|. air_self_test_eo              |

|. air_update_fault_codes        |

|. air_update_failure_flags      |

+--------------------------------+

|PRIVATE PROCEDURES:             |

|. NONE                          |

+--------------------------------+

This module contains the processes which  perform  Continuous  and  On-Demand

testing  of the mass air flow sensor circuit.  Processes are also included to

update fault codes and malfunction flags.

Fault filtering for the air meter uses the rolling average filter routine and

will filter towards 1 with a fault present and 0 without a fault present.  If

the fault threshold (Pxxxx_LVL) is set to 0.632, the fault code will  be  set

when  a  failure has been present for approximately Pxxxx_TC seconds.  If the

entry criteria for a  particular  test  are  not  met,  the  test  cannot  be

performed,  and  the fault filter holds its value.  If any fault code is set,

the overall air meter malfunction flag, MFMFLG, is set  to  indicate  to  the

rest  of  the  strategy  that  the  mass  air  flow sensor is not functioning

properly.

Different test modes may have different failure thresholds, but  the  failure

types are generally as follows:

out                             out

under        of                              of         over

voltage  |   range   |   in range, OK    |   range   |  voltage

------------|-----------|-------------------|-----------|------------>

increasing

voltage

24-42

AIR CHARGE CALCULATION, MAF SENSOR DIAGNOSTIC TESTS - BACKGROUND - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  DSDRPM = Desired idle speed, rpm.

-  IMAF = Raw mass air flow sensor output, A-to-D counts.

-  IMFMCTR = Hysteresis term for foreground air  meter  failure  indication,

background loop counts.

-  N = Engine speed, rpm.

-  P0102_FIL = Fault filter for fault P0102 (mass  air  flow  circuit  low),

unitless.

-  P0103_FIL = Fault filter for fault P0103 (mass air  flow  circuit  high),

unitless.

-  TP_REL = Relative throttle position, A-to-D counts.

-  TSLPIP = Amount of time which has passed since the last, msec.

Bit Flags:

-  CRKFLG = Crank mode flag; 1 -> in crank mode, unitless.

-  FFG_MAF = OBD-II system FMEM flag for MAF; 1 -> MAF sensor not  currently

reliable, unitless.

-  IMFMFLG = Flag indicating an instantaneous mass air  flow  sensor  fault,

unitless.

-  MFMFLG = Flag indicating a mass air flow  sensor  failure;  1  ->  failed

sensor, unitless.

-  P0102_INST = Flag indicating an instantaneous under-voltage condition;  1

-> condition exists, unitless.

-  P0103_INST = Flag indicating an instantaneous over-voltage  condition;  1

-> condition exists, unitless.

-  PxxxxMALF = OBD-II malfunction flag for fault xxxx; 1  ->  a  malfunction

currently exists for fault xxxx.

-  UNDSP = Run/underspeed mode flag; 0 -> in run mode, 1 -> in underspeed or

crank mode, unitless.

Calibration Constants:

-  FIL_HYST = Hysteresis term to prevent spurious clearing of  fault  codes,

unitless.

-  MAF_HI_ER = MAFS test upper limit, A-to-D counts.

-  MAF_HI_KOEO = Engine-Off On-Demand MAFS test upper limit, A-to-D counts.

24-43

AIR CHARGE CALCULATION, MAF SENSOR DIAGNOSTIC TESTS - BACKGROUND - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  MAF_HI_KOER = Engine-Running On-Demand  MAFS  test  upper  limit,  A-to-D

counts.

-  MAF_LO_ER = MAFS test lower limit, A-to-D counts.

-  MAF_LO_KOER = Engine-Running On-Demand  MAFS  test  lower  limit,  A-to-D

counts.

-  MAF_RPM_MAX = Engine  speed  below  which  air  flow  is  low  enough  to

distinguish normal air flow signal from over-voltage condition, rpm.

-  P0102_LVL = Fault threshold for under-voltage fault, unitless.

-  P0103_LVL = Fault threshold for over-voltage fault, unitless.

-  P0102_TC = Fault filter time constant for under-voltage fault, seconds.

-  P0103_TC = Fault filter time constant for over-voltage fault, seconds.

-  TP_VMAF = TP above which the MAF sensor Continuous test is executed.

-  VMAFPIPLMT = Amount of time since last PIP  which  indicates  engine  has

stalled, msec.

OTHER

-  clear_malf(Pxxxx) = Logic process imported from the  MIL  control  module

which clears the MIL fault code specified by Pxxxx.

-  malfunction(ccm,Pxxxx) = Logic process  imported  from  the  MIL  control

module which stores the MIL fault code specified by Pxxxx.

-  maf_codes = SET OF {P0102, P0103}

-  maf_other_codes = SET OF {P1101}

-  P0102 = Fault code (MIL) - mass air flow circuit low.

-  P0103 = Fault code (MIL) - mass air flow circuit high.

-  P1101 = Fault code (non-MIL) - mass air flow  sensor  outside  self  test

range.

-  store_code(Pxxxx) = Logic process which stores  the  non-MIL  fault  code

specified by Pxxxx.

24-44

AIR CHARGE CALCULATION, MAF SENSOR DIAGNOSTIC TESTS - BACKGROUND - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: AIR_MAFS_DIAG_BG_COM1

BEGIN:  air_over_voltage_entry

;This process is executed only when explicitly called.

This logic controls entry to the over-voltage test.  Because so much  of  the

range  of  the sensor is normally used to sense air flow, the sensor's output

at  high  flow  is  indistinguishable  from  an  over-voltage  failure.    An

over-voltage failure can therefore only be detected at low or moderate flows.

N < MAF_RPM_MAX ------------------------| Do: air_over_voltage_test

;air flow and sensor output            |  ;check for over-voltage

;should not be high                    |  ;condition

|

| --- ELSE ---

|

| NO ACTION

|  ;air flow and sensor output

|  ;should be high, can't detect

|  ;an over-voltage condition,

|  ;do not execute test

END: air_over_voltage_entry

24-45

AIR CHARGE CALCULATION, MAF SENSOR DIAGNOSTIC TESTS - BACKGROUND - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: air_under_voltage_cont_entry

;This process is executed only when explicitly called.

This logic controls entry to the Continuous (as  opposed  to  the  On-Demand)

under-voltage  test.   Because so much of the range of the sensor is normally

used to sense air flow, the sensor's output at low flow is  indistinguishable

from  an  under-voltage failure.  An under-voltage failure can therefore only

be detected at moderate or high flows.

TSLPIP <= VMAFPIPLMT -------------|

;engine has not stalled          |

|

UNDSP = 0 ------------------------|

;in run mode                     |AND -| Do: air_under_voltage_test

|     |  ;check for under-voltage

TP_REL > TP_VMAF -----------------|     |  ;condition

;air flow and sensor output      |     |

;should not be low               |     |

|     |

P0103_INST = 0 -------------------|     |

;no over-voltage condition             |

;detected,                             |

;if there had been, no point           |

;in checking for under-voltage         |

| --- ELSE ---

|

| NO ACTION

|  ;air flow and sensor output

|  ;should be low, can't detect

|  ;an under-voltage condition,

|  ;do not execute test

END: air_under_voltage_cont_entry

24-46

AIR CHARGE CALCULATION, MAF SENSOR DIAGNOSTIC TESTS - BACKGROUND - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: air_under_voltage_er_entry

;This process is executed only when explicitly called.

This logic controls entry to the Engine-Running On-Demand under-voltage test.

Because  so  much  of  the  range of the sensor is normally used to sense air

flow,  the  sensor's  output  at  low  flow  is  indistinguishable  from   an

under-voltage  failure.   An  under-voltage  failure  can  therefore  only be

detected at moderate or high flows.

TSLPIP <= VMAFPIPLMT -------------|

;engine has not stalled          |

|

UNDSP = 0 ------------------------|AND -| Do: air_under_voltage_test

;in run mode                     |     |  ;check for under-voltage

|     |  ;condition

P0103_INST = 0 -------------------|     |

;no over-voltage condition             |

;detected,                             |

;if there had been, no point           |

;in checking for under-voltage         |

| --- ELSE ---

|

| NO ACTION

|  ;air flow and sensor output

|  ;should be low, can't detect

|  ;an under-voltage condition,

|  ;do not execute test

END: air_under_voltage_er_entry

24-47

AIR CHARGE CALCULATION, MAF SENSOR DIAGNOSTIC TESTS - BACKGROUND - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: air_self_test_er_entry

;This process is executed only when explicitly called.

This logic controls entry to the Engine-Running On-Demand in-range test.   To

ensure  sufficient air flow for the test, execution of the test is restricted

to the high  engine  speed  portion  of  the  Engine-Running  On-Demand  test

(typically this is about 1600 rpm).

Also, it is undesirable to indicate an "out of self test range" condition  if

the  circuit is clearly over- or under-voltage (indicating specifically over-

or under-voltage provides more detail than just out-of-range).  Consequently,

if  an  over- or under-voltage condition has been detected, the in-range test

is not performed.

N > 1000 -------------------------|

;engine must be generating       |

;sufficient flow                 |

|

DSDRPM > 1000 --------------------|

;engine must be generating       |

;sufficient flow                 |AND -| Do: air_self_test_er

|     |  ;check for out-of-range

P0103_INST = 0 -------------------|     |  ;condition

;no over-voltage condition       |     |

;detected                        |     |

|     |

P0102_INST = 0 -------------------|     |

;no under-voltage condition            |

;detected                              |

| --- ELSE ---

|

| NO ACTION

|  ;can't do test or higher

|  ;priority code imminent

END: air_self_test_er_entry

24-48

AIR CHARGE CALCULATION, MAF SENSOR DIAGNOSTIC TESTS - BACKGROUND - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: air_self_test_eo_entry

;This process is executed only when explicitly called.

This logic controls entry to the Engine-Off On-Demand in-range test.   It  is

undesirable  to indicate an "out of self test range" condition if the circuit

is  clearly  over-  or  under-voltage  (indicating  specifically   over-   or

under-voltage provides more detail than just out-of-range).  Consequently, if

an over- or under-voltage condition has been detected, the in-range  test  is

not performed.

P0103_INST = 0 -------------------|

;no over-voltage condition       |

;detected                        |AND -| Do: air_self_test_eo

|     |  ;check for out-of-range

P0102_INST = 0 -------------------|     |  ;condition

;no under-voltage condition            |

;detected                              |

| --- ELSE ---

|

| NO ACTION

|  ;higher priority code imminent

END: air_self_test_eo_entry

24-49

AIR CHARGE CALCULATION, MAF SENSOR DIAGNOSTIC TESTS - BACKGROUND - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: air_over_voltage_test

;This process is executed only when explicitly called.

This logic defines the over-voltage test.  It is assumed that if this process

is  called,  engine  operating  conditions are appropriate for detecting this

type of failure.

IMAF > MAF_HI_ER -----------------------| ;over-voltage condition

;sensor output higher than expected    |

| P0103_FIL := ROLAV(1,P0103_TC)

|  ;increment over-voltage fault

|  ;filter

|

| P0102_FIL := ROLAV(0,P0102_TC)

|  ;decrement under-voltage fault

|  ;filter,

|  ;can't be under-voltage if

|  ;over-voltage fault exists

|

| P0103_INST := 1

|  ;instantaneous over-voltage

|  ;condition exists

|

| --- ELSE ---

|

| P0103_FIL := ROLAV(0,P0103_TC)

|  ;decrement over-voltage fault

|  ;filter

|

| P0103_INST := 0

|  ;instantaneous over-voltage

|  ;condition does not exist

END: air_over_voltage_test

24-50

AIR CHARGE CALCULATION, MAF SENSOR DIAGNOSTIC TESTS - BACKGROUND - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: air_under_voltage_test

;This process is executed only when explicitly called.

This logic defines the under-voltage  test.   It  is  assumed  that  if  this

process  is called, engine operating conditions are appropriate for detecting

this type of failure.

IMAF < MAF_LO_ER -----------------------| ;under-voltage condition

;sensor output lower than expected     |

| P0102_FIL := ROLAV(1,P0102_TC)

|  ;increment under-voltage fault

|  ;filter

|

| P0103_FIL := ROLAV(0,P0103_TC)

|  ;decrement over-voltage fault

|  ;filter,

|  ;can't be over-voltage if

|  ;under-voltage fault exists

|

| P0102_INST := 1

|  ;instantaneous under-voltage

|  ;condition exists

|

| --- ELSE ---

|

| P0102_FIL := ROLAV(0,P0102_TC)

|  ;decrement under-voltage fault

|  ;filter

|

| P0102_INST := 0

|  ;instantaneous under-voltage

|  ;condition does not exist

END: air_under_voltage_test

24-51

AIR CHARGE CALCULATION, MAF SENSOR DIAGNOSTIC TESTS - BACKGROUND - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: air_self_test_er

;This process is executed only when explicitly called.

This logic defines the Engine-Running On-Demand in-range test.  It is assumed

that  if  this process is called, engine operating conditions are appropriate

for detecting this type of failure.

IMAF < MAF_LO_KOER  --------------|

;sensor output lower             |

;than expected                   |OR --| ;outside Engine-Running

|     | ;On-Demand range

IMAF > MAF_HI_KOER ---------------|     |

;sensor output higher                  | store_code(P1101)

;than expected                         |  ;filter not used so that

|  ;intermittent faults can

|  ;be detected

|

| --- ELSE ---

|

| NO ACTION

END: air_self_test_er

24-52

AIR CHARGE CALCULATION, MAF SENSOR DIAGNOSTIC TESTS - BACKGROUND - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: air_self_test_eo

;This process is executed only when explicitly called.

This logic defines the Engine-Off On-Demand in-range  test.   It  is  assumed

that  if  this process is called, engine operating conditions are appropriate

for detecting this type of failure.

IMAF > MAF_HI_KOEO ---------------------| ;outside Engine-Off

;sensor output higher than expected    | ;On-Demand range

|

| store_code(P1101)

|  ;filter not used so that

|  ;intermittent faults can

|  ;be detected

|

| --- ELSE ---

|

| NO ACTION

END: air_self_test_eo

24-53

AIR CHARGE CALCULATION, MAF SENSOR DIAGNOSTIC TESTS - BACKGROUND - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: air_update_fault_codes

;This process is executed only when explicitly called.

When a fault filter exceeds the fault level, the malfunction flag is set  for

that  code.   FIL_HYST  provides  hysteresis  before clearing the malfunction

flag.

P0102_FIL > P0102_LVL ------------------| malfunction(ccm,P0102)

;persistent under-voltage condition    |  ;alert diagnostic executive that

|  ;an under-voltage fault exists

|

| --- ELSE ---

|

P0102_FIL < P0102_LVL - FIL_HYST -------| clear_malf(P0102)

;no under-voltage condition            |  ;alert diagnostic executive that

|  ;an under-voltage fault does not

|  ;exists

P0103_FIL > P0103_LVL ------------------| malfunction(ccm,P0103)

;persistent over-voltage condition     |  ;alert diagnostic executive that

|  ;an over-voltage fault exists

|

| --- ELSE ---

|

P0103_FIL < P0103_LVL - FIL_HYST -------| clear_malf(P0103)

;no over-voltage condition             |  ;alert diagnostic executive that

|  ;an over-voltage fault does not

|  ;exists

END: air_update_fault_codes

24-54

AIR CHARGE CALCULATION, MAF SENSOR DIAGNOSTIC TESTS - BACKGROUND - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: air_update_failure_flags

;This process is executed only when explicitly called.

The flag MFMFLG is used to indicate the status of the MAF sensor to the  rest

of  the  strategy.   The  flags P0102MALF and P0103MALF are maintained by the

diagnostic executive routine.

P0102MALF = 1 --------------------|

;MAF circuit failed low         |OR --| MFMFLG := 1

|     |  ;circuit malfunction detected,

P0103MALF = 1 --------------------|     |  ;set flag to alert rest of

;MAF circuit failed high              |  ;strategy

|

| --- ELSE ---

|

| MFMFLG := 0

|  ;no circuit malfunction detected,

|  ;clear flag to alert rest of

|  ;strategy

This logic provides a calibratable delay after the sensor returns  in  range,

equal  to MFMHYS - 1 background loops, before clearing the foreground failure

flag.

CRKFLG = 1 -----------------------------| IMFMFLG := 0

;in crank mode                         | IMFMCTR := 0

|  ;sensor test disabled in

|  ;crank mode

|

| --- ELSE ---

|

IMFMCTR = 0 ----------------------------| IMFMFLG := 0

;FMEM exit delay complete              |  ;clear foreground FMEM flag

;or FMEM has not been active           |

| --- ELSE ---

|

| IMFMCTR := max((IMFMCTR - 1),0)

|  ;clip to a minimum of zero

24-55

AIR CHARGE CALCULATION, MAF SENSOR DIAGNOSTIC TESTS - BACKGROUND - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The FFG_MAF flag is used to immediately signal the OBD-II executive that  MAF

is  currently  unreliable and that a malfunction may exist.  The FFG_MAF flag

bypasses the delays caused by fault filters used  for  malfunction  codes  to

ensure  that  other  OBD-II  tests  which rely on the MAF sensor are promptly

advised that a malfunction may exist.

IMFMFLG = 1 ----------------------------| FFG_MAF := 1

;instantaneous failure                 |  ;MAF failure

|

| --- ELSE ---

|

| FFG_MAF := 0

|  ;no MAF failure

END: air_update_failure_flags

24-56

AIR CHARGE CALCULATION, LOAD CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OVERVIEW

24.1.12  LOAD CALCULATION (CDAM0)

LOAD is a unitless value  showing  the  ratio  of  the  Cylinder  Air  Charge

(CYLARC_BG)  to  the  theoretical  air charge at EEC Standard Temperature and

Pressure  (SARCHG).   Using  LOAD  makes  all  engine  sizes  more   directly

comparable, and allows a similar scaling for all table look-ups.

AIR MASS DETERMINATION

Air mass is computed for use in background in units of lbs/min.

PERLOAD CALCULATION

PERLOAD is a variable used to control air/fuel ratio and upstream  air.   The

air/fuel  ratio control includes:  1) stabilized open loop enrichment, 2) MPG

mode cruise enable and desired  air/fuel,  3)  cold  start  enrichment.   Its

original definition was corrupted when the option was included to set PERLOAD

= LOAD.  Thus a true percent load (PCT_LOAD) had to be introduced to  predict

flow across orifices like the air bypass valve.

DEFINITIONS

Registers:

-  AM = Average air mass flow rate, lbm/min.

-  BP = Barometric pressure, inches Hg abs.

-  CYLARC_BG = Air mass inducted per intake stroke, lbm/intake.

-  LOAD = Normalized cylinder air charge, unitless.

-  N = Engine speed, rpm.

-  PCT_LOAD = Percent inferred LOAD, unitless.

-  PERLOAD = Percent load, unitless.

Calibration constants:

-  ENGCYL = Number of intake strokes per engine revolution.

-  FN035(N) = Peak load  at  sea  level  as  a  function  of  engine  speed,

unitless.

24-57

AIR CHARGE CALCULATION, LOAD CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PRLDSW = Switch to control calculation of PERLOAD, unitless.

24-58

AIR CHARGE CALCULATION, LOAD CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  SARCHG = Standard air charge for a single cylinder, lbm/intake.

= 2.701E-09 * pi * [(B/2)**2] * S   lbm/intake

where B = bore in millimeters

S = stroke in millimeters

(2.701E-09 = denisty of air in lbm per cubic millimeter)

PROCESS

STRATEGY MODULE: AIR_LOAD_COM1

unconditionally ------------------------| LOAD := CYLARC_BG / SARCHG

|

| AM := N * ENGCYL * CYLARC_BG

PRLDSW = 0 -----------------------------| PERLOAD = [LOAD * 29.9] /

|            [FN035(N) * BP]

|

| --- ELSE ---

|

PRLDSW = 1 -----------------------------| PERLOAD = LOAD

|

| --- ELSE ---

|

PRLDSW = 2 -----------------------------| PERLOAD = PCT_LOAD

24-59

AIR CHARGE CALCULATION, INFERRED LOAD - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

24.1.13  INFERRED LOAD (CDAM0)

OVERVIEW

There is a need in EEC strategy for air flow calculations made  independently

of the air meter and BP sensor.  Engine speed, throttle position, and percent

EGR provide sufficient information  to  infer  the  volumetric  flow  of  air

through  the  throttle body.  Given an air density, this volumetric flow rate

can be converted to a mass flow rate.  The air bypass valve's duty cycle  and

pressure  ratio  across  the  orifice  approximately define the flow across a

forced balanced air bypass valve.  (The  lack  of  force  balancing  at  high

manifold vacuums is not yet modeled so errors in prediction exist).

From these inputs the following variables are calculated for use elsewhere in

the strategy:

1) TOTLDST is an estimate of what LOAD would  be  under  the  current  engine

operating  condition  (speed,  throttle  position,  and  EGR rate) and at ECC

Standard Temperature  and  Pressure  (ESTP).   It  includes  components  from

throttle body flow and air bypass flow.

2) BYMAST is an estimate of what the mass flow rate of air  through  the  air

bypass  valve  would  be  at  the  current engine operating condition (speed,

throttle position, and EGR rate) and at ESTP.

3) DELTOTLD  is  the  magnitude  of  the  change  in  inferred  LOAD  between

successive calculations.  It can be used to detect rapid transients.

4) PCT_LOAD is calculated for use in the air bypass model and for an optional

PERLOAD  calculation.   It  has  the  characteristic  of  going to 1.0 at any

altitude and any engine speed when the maximum air flow for that altitude and

engine speed is reached, thus indicating that the driver is demanding maximum

torque.  PCT_LOAD does not use BP in its calculation and is therefore  immune

to any inaccuracy in BP.

NOTE:  EEC Standard Temperature and Pressure (ESTP) is defined to be 100  deg

F  inlet  air  (ACT),  29.92  in Hg barometric pressure, and 200 deg F engine

coolant temperature (ECT).

DEFINITIONS

Registers:

-  BYLDST = Estimate of component of LOAD due to bypass air mass, unitless.

-  BYLDST_INST = Instantaneous load due to air flow  through  bypass  valve,

unitless.

-  BYMAST = Estimate of flow rate of air through the  air  bypass  valve  at

ESTP, lbm/min.

-  DELTOTLD = Magnitude of change in inferred LOAD  between  two  successive

calculations, unitless.

24-60

AIR CHARGE CALCULATION, INFERRED LOAD - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  EGRACT = Actual percent EGR, unitless.

-  ISCDTY = Air bypass valve commanded duty cycle, unitless.

-  N = Engine speed, rpm.

-  PCT_LOAD = Percent inferred LOAD based on mapped engine behavior at ESTP,

unitless.

-  TOTLDST = Total inferred LOAD, unitless.

-  TP_REL = Throttle position, A/D counts.

-  TPLDST = Estimate of LOAD due to mass  of  air  that  flows  through  the

throttle at ESTP during a single cylinder induction event, unitless.

Calibration constants:

-  ENGCYL = Number of intake strokes per engine revolution.

-  FN044(TP_REL) = Normalized throttle position, unitless.

-  FN070E(N) = Normalized engine speed, unitless.

-  FN077(PCT_LOAD) = Normalized percent load, unitless.

-  FN078(ISCDTY) = Normalized air bypass valve duty cycle, unitless.

-  FN1036A = Normalized cyinder air charge at sea level with no EGR and port

throttles closed, unitless.

X = FN070E(N)

Y = FN044(TP_REL)

-  FN1036B = Normalized cyinder air charge at sea level with no EGR and port

throttles open, unitless.

X = FN070E(N)

Y = FN044(TP_REL)

-  FN1037 = TPLDST subtractor as a function of N and TP_REL at ESTP for  10%

EGR, unitless.

X = FN070E(N)

Y = FN044(TP_REL)

-  FN1039 = Flow rate of air through air bypass valve at ESTP, lbm/min.

X = FN078(ISCDTY)

Y = FN077(PCT_LOAD)

-  SARCHG = Standard air charge for a single cylinder, lbm/intake stroke.

-  TCBYLD = Time constant of filter for flow through air bypass valve, sec.

24-61

AIR CHARGE CALCULATION, INFERRED LOAD - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: AIR_LOAD_INFERRED_COM2

BEGIN: air_load_inferred

;This process is executed once per background loop.

IMRC_POSN = 1 -----| ; port throttles open

;port throttles   |

;are open         | load_table = FN1036B(N,TP_REL)

|  ;normalized air charge

|

| load_wot = FN1036B(N,top row)

|  ;normalized air charge at

|  ;wide open throttle

|

| --- ELSE ---

|

| ; port throttles closed

|

| load_table = FN1036A(N,TP_REL)

|  ;normalized air charge

|

| load_wot = FN1036A(N,top row)

|  ;normalized air charge at

|  ;wide open throttle

24-62

AIR CHARGE CALCULATION, INFERRED LOAD - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

unconditionally ---| TPLDST := load_table -

|           EGRACT/10. * FN1037(N,TP_REL)

|  ;estimate of LOAD due to mass of air

|  ;that flows through the throttle at

|  ;ESTP during a single cylinder

|  ;induction event

|

| BYMAST := FN1039(ISCDTY,PCT_LOAD)

|  ;estimate of mass flow rate of air

|  ;through the air bypass valve

|  ;at ESTP

|

| BYLDST_INST := BYMAST/(N * ENGCYL * SARCHG)

|  ;air bypass valve flow rate converted

|  ;to air mass and normalized to load units

|

| BYLDST := ROLAV(BYLDST_INST,TCBYLD)

|  ;filtered estimate of LOAD due to bypass

|  ;air mass

|

| totldst_old := TOTLDST

|  ;save old value for use in calculating

|  ;change in inferred load

|

| TOTLDST := clip((BYLDST + TPLDST),

|                 0,load_wot)

|  ;total inferred load,

|  ;includes components from throttle body

|  ;and air bypass valve

|

| DELTOTLD := abs(TOTLDST - totldst_old)

|  ;magnitude of change in inferred load

|  ;since previous calculation

|

| PCT_LOAD := TOTLDST / load_wot

|  ;normalized inferred LOAD

END: air_load_inferred

24-63

AIR CHARGE CALCULATION, j1979 PARAMETER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

24.1.14  J1979_01_10, MASS AIR FLOW, CALCULATION (CDAM0)

OVERVIEW

SAE standard j1979 requires  that  a  parameter  (j1979_01_10  in  grams  per

second)  be available for the scan tool.  This value must reflect the current

value of the sensor input and not a value  substituted  via  a  FMEM  action.

This parameter is derived from MAF (lbm/tick) which is calculated at each PIP

edge.  Note that it is necessary to convert from ticks to seconds as well  as

from pounds mass to grams.

j1979_01_10 gm/sec = MAF lbm/tick * STCF tic/sec * 453.6 gm/lbm

DEFINITIONS

INPUTS

Registers:

-  MAF = The air flow that is computed at each PIP edge, lbm/tick.

Non-Calibratable Constants:

-  STCF = Seconds to clock ticks conversion factor.

OUTPUTS

Registers:

-  j1979_01_10 =  Current  Mass  air  flow  in  grams  per  second  for  SCP

interface.

PROCESS

STRATEGY MODULE: AIR_J1979_COM1

BEGIN: pid_definitions

unconditionally --------| pid_def(j1979_01_10, (MAF * STCF * 453.6))

|  (Calculate mass air flow in gm/sec)

END:  pid_definitions

24-64

AIR CHARGE CALCULATION, SAE J1979 LOAD PARAMETER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

24.1.15  CALCULATION OF SAE J1979 LOAD PARAMETER (CDAM0)

OVERVIEW

SAE standard J1979 requires that a  parameter  for  current  engine  load  be

available  for the scan tool, expressed as a percent where 00h corresponds to

zero percent and FFh corresponds to  100  percent  of  maximum  load  of  the

engine.  This load is used primarly for engine diagnostics and duplication of

engine operating conditions.

DEFINITIONS

INPUTS

Registers:

-  N = Engine speed, rpm.

Calibration Constants:

-  NSTALL = Engine speed below  which  the  engine  is  considered  to  have

stalled, rpm.

OUTPUTS

Registers:

-  j1979_01_04 = Engine load PID available via the scan tool, unitless.

-  LOAD_SAE = Engine load as required by SEA standard J1979, unitless.

-  PCT_LOAD = Inferred percent  load  (based  on  mapped  engine  behavior),

unitless.

24-65

AIR CHARGE CALCULATION, SAE J1979 LOAD PARAMETER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: AIR_J1979_LOAD_COM1

BEGIN: air_pid_definition_load

;This process is executed once per background loop.

When the  engine  is  not  turning  or  turning  very  slowly  (e.g.   during

engine-off  testing,  during  cranking, following an engine stall, etc.), the

load value is set to zero.  A service technician expecting  a  load  of  zero

under  these  conditions  may  misdiagnosis or replace a good MAF sensor if a

non-zero value is transmitted to diagnostic tools via SCP.

N < NSTALL -----------------| LOAD_SAE := 0

;engine speed below        |  ;engine turning too slowly,

;stall speed               |  ;set engine load for SAE J1979

|  ;to zero

|

| --- ELSE ---

|

| LOAD_SAE := PCT_LOAD

|  ;calculate engine load for SAE J1979

|  ;from mapped engine behavior

unconditionally ------------| pid_def( j1979_01_04, (LOAD_SAE * 255) )

|  ;PID definition for value of engine load

|  ;transmitted via SCP

END: air_pid_definition_load

24-66

CHAPTER 25

TRANSMISSION INPUT CONVERSIONS

25-1

TRANSMISSION INPUT CONVERSIONS, PERFORMANCE WEIGHTING DETERMINATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

25.1  PERFORMANCE WEIGHTING DETERMINATION (CDAI0)

OVERVIEW

This module will continuously determine  the  performance  weighting  factor.

This  will be a function of TP_REL and TPRATE_PWF.  It will be used to adjust

the shift points in the shift schedules  toward  a  performance  value  or  a

economy  value.  This allows the shift points to be biased toward the economy

schedule at low values of TPRATE_PWF.   It  also  allows  biasing  the  shift

points toward the performance schedule for high rates of throttle movement.

DEFINITIONS

INPUTS

Registers:

-  BG_TMR = Background loop timer.

-  PNWF = Performance and Normal Weighting Factor.

-  pnwf = Instantaneous PNWF.

-  PWF_AT_DNSFT = The value of PNWF at the moment a downshift is commanded.

-  TM_PWF_DEC = Time to hold PNWF after an upshift, seconds.

-  TPRATE_PWF = Filtered throttle  position  rate  for  PNWF  determination,

counts.

-  TP_RATE = Throttle position rate.

-  TP_REL = Relative throttle position, counts.

Bit Flags:

-  FLG_FRST_CM = First time a shift is commanded flag; 1 -> shift  commanded

this background pass, 0 -> no shift commanded this background pass.

-  FLG_SFT_DN = Downshift flag.  1 -> last shift was  an  downshift.   0  ->

last shift was a upshift.

-  FLG_SFT_UP = Upshift flag; 1 -> last shift was  an  upshift,  0  ->  last

shift was a downshift.

Calibration Constants:

-  FNPWFE = Performance weighting factor lookup table for economy mode.

X = FNPWFX(TP_REL)

Y = FNPWFY(TP_RATE)

-  FNPWFX(TP_REL) = Normalizing function for TP_REL for input to  FNPWFE  or

FNPWFP.

25-2

TRANSMISSION INPUT CONVERSIONS, PERFORMANCE WEIGHTING DETERMINATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FNPWFY(TP_RATE) = Normalizing function for TP_RATE for input to FNPWFE or

FNPWFP.

-  PWF_DEC_E  =  Decrement  PNWF  by  this  amount  every  second  (after  a

downshift, or when TM_PWF_DEC expires), in economy mode.

-  TMHOLDPWF = Time to hold PNWF after an upshift, seconds.

OUTPUTS

Registers:

-  PNWF = See above.

-  pnwf = See above.

-  PWF_AT_DNSFT = See above.

-  TM_PWF_DEC = See above.

25-3

TRANSMISSION INPUT CONVERSIONS, PERFORMANCE WEIGHTING DETERMINATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INTRN_PWF_DET_COM3

always -------------------------| pnwf = FNPWFE(TP_REL,TPRATE_PWF)

pnwf >= PNWF -------------------| PNWF = pnwf

(increasing PNWF)             |  (pass pnwf through to PNWF)

|

| --- ELSE ---

|

| PNWF = MAX[PWF_AT_DNSFT,pnwf,PNWF -

|                PWF_DEC_E * BG_TMR]

|   (let PWF decay to pnwf with minimum

clip of PWF_AT_DNSFT)

FLG_SFT_DN = 1 -----------|

|AND -| PWF_AT_DNSFT = PNWF

FLG_FRST_CM = 1 ----------|     |  (set minimum clip to PNWF at the

(a downshift is commanded      |   time of the downshift)

this B.G. pass)               |

| TM_PWF_DEC = TMHOLDPWF

|  (set PNWF hold timer)

|

| --- ELSE ---

FLG_SFT_UP = 1 -----|           |

|AND -|     |

FLG_FRST_CM = 1 ----|     |     |

(an upshift is           |     |

comanded this           |OR --| PWF_AT_DNSFT = 0

B.G. pass)             |     |  (set minimum clip to allow

|     |   PNWF to decay to pwf)

TM_PWF_DEC = 0  ----------|

(timer expired)

25-4

TRANSMISSION INPUT CONVERSIONS, TRANSMISSION CONTROL INDICATOR LIGHT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

25.2  TRANSMISSION CONTROL INDICATOR LIGHT (CDAM0)

OVERVIEW

The Transmission Control Indicator Light (TCIL), located  in  the  instrument

panel, visually indicates the status of "Transmission Control" to the driver,

or alerts the driver to certain transmission faults.   If  not  indicating  a

fault,  the  light  is  on  when overdrive is canceled; off when overdrive is

enabled.  A flashing light indicates a transmission fault.  The  transmission

control  switch  will  continue to operate normally in the flashing mode, but

there is no visual indication of cancel mode.

To disable flashing for any fault, set TCIL_TM_DLY = 31.875.

To enable flashing for specific faults, set TCIL_TM_DLY < 31.875, and set the

calibration  switch  for  that fault = 1.  For example, to flash the TCIL for

converter clutch errors and shift errors, set CC_ERR_SW = 1 and SFT_ERR_SW  =

1, and all remaining switches = 0.

DEFINITIONS

Registers:

-  TCILTMR = Time since transmission fault occurred, sec.

-  TCIL_FLASH_TMR = Time since TCIL changed states in flashing mode, sec.

Bit Flags:

-  CRKFLG = Flag indicating engine mode status; 0 -> not in CRANK mode, 1 ->

in CRANK mode.

-  FLG_DIS_CSM = Disable Continuous Slip Mode Flag; 1 ->  -converter  clutch

system is stability detected, disable softlock.

-  FLG_TCS =  Transmission  Control  flag;  0  ->  overdrive  enable,  1  ->

overdrive lockout mode.

-  OTEMP_FM_FLG = Transmission overtemperature FMEM flag; 1 ->  Transmission

is overtemperature, 0 -> Transmission temperature okay.

-  PDL_ERROR =  Transmission  Range  Sensor  error  flag;  1  ->  TRS  error

detected.

-  PxxxMALF = OBD-II malfunction flag for fault xxx; 1  ->  Malfunction  xxx

currently exits.

-  SFT_FM_FLG = Flag indicating whether 1-2 or 2-3 or 3-4 shifts are failing

to occur properly; 0 -> shifts OK, 1 -> Shifts not OK.

-  TCIL_STATE = Flag indicating state of TCIL; 0 -> TCIL off, 1 -> TCIL on.

-  TOT_FM_FLG = TOT failure mode flag; 1 -> TOT fmem in progress.

25-5

TRANSMISSION INPUT CONVERSIONS, TRANSMISSION CONTROL INDICATOR LIGHT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  CC_ERR_SW = Calibration selection switch to enable/disable flashing  TCIL

for converter clutch error; 0 -> disable, 1 -> enable.

-  CNET_ERR_SW = Calibration switch to enable/disable flashing the TCIL  for

Catchnet (converter clutch system instability) errors; 1 -> enable.

-  EPC_ERR_SW = Calibration selection switch to enable/disable flashing TCIL

for EPC open circuit error; 0 -> disable, 1 -> enable.

-  EPC_SHORT_SW = Calibration switch to enable/disable flashing the TCIL for

EPC short circuit error; 1 -> enable.

-  OSS_ERR_SW = Calibration switch to enable/disable flashing the  TCIL  for

OSS malfunctions; 1 -> enable.

-  OS_NOISE_SW = Calibration switch to enable/disable flashing the TCIL  for

a noisy OSS malfunction; 1 -> eanble.

-  OTEMP_ERR_SW = Calibration selection switch  to  enable/disable  flashing

TCIL  for  Transmission  overtemperature  condition;  0  -> disable, 1 ->

enable.

-  SFT_ERR_SW = Calibration selection switch to enable/disable flashing TCIL

for Shift Errors; 0 -> disable, 1 -> enable.

-  SOL_CKT_SW = Calibration switch to enable/disable flashing TCIL for shift

solenoid circuit malfunctions; 1 -> enable.

-  SW_0741_TCIL = Calibration selection switch  to  enable/disable  flashing

TCIL for converter system stuck off error; 0 -> disable.

-  SW_1743_TCIL = Calibration selection switch  to  enable/disable  flashing

TCIL for PWM "on" error; 0 -> disable.

-  TCC_CKT_SW = Calibraiton switch to enable/disable flashing the  TCIL  for

TCC solenoid circuit malfunctions; 1 -> enable.

-  TCILTM1 = Flashing TCIL "ON"/"OFF" time period, sec.

-  TCIL_TM_DLY = Time after fault has occurred before  the  TCIL  begins  to

flash, sec.  Set to 31.875 to disable flashing.

-  TOT_ERR_SW = Calibration selection switch to enable/disable flashing  the

TCIL for TOT failures; 0 -> disable, 1 -> enable.

-  TRS_ERR_SW = Calibration switch to enable/disable flashing the  TCIL  for

Transmission Range Sensor malfunctions; 1 -> enable.

-  VSS_ERR_SW = Calibration switch to enable/disable flashing the  TCIL  for

VSS malfunctions; 1 -> enable.

-  VS_NOISE_SW = Calibration switch to enable/disable flashing the TCIL  for

a noisy VSS malfunction; 1 -> eanble.

25-6

TRANSMISSION INPUT CONVERSIONS, TRANSMISSION CONTROL INDICATOR LIGHT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OTHER

-  P0500 = Fault code for vss failure.

-  P0503 = Noisy VSS malfunction.

-  P0720 = OBD-II fault code - Output Speed Sensor Malfucntion.

-  P0721 = OBD-II fault code, noisy OSS.

-  P0741 = Torque converter clutch system performance or stuck off.

-  P0743 = Torque converter clutch system electrical.

-  P0750 = OBD-II fault code for SSA circuit malfunction.

-  P0755 = OBD-II fault code for SSB circuit malfunction.

-  P1743 = Fault code, PWM mechanically "on" failure (NON MIL code).

-  P1746 = Fault code; EPC VFS open circuit.

-  P1747 = Fault code; EPC VFS short to ground.

25-7

TRANSMISSION INPUT CONVERSIONS, TRANSMISSION CONTROL INDICATOR LIGHT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INTRN_TCIL_STATE_COM5

FLG_TCS = 0 ----------------|

(overdrive enable)         |AND -|

|     |

TCILTMR <= TCIL_TM_DLY -----|     |

(flashing mode entry delay)       |OR --|  (turn TCIL off)

|     | TCIL_STATE = 0

TCIL_FLASH_TMR >= TCILTM1 --|     |     | TCIL_FLASH_TMR = 0

(flashing period expired)  |AND -|     |

|           |

TCIL_STATE = 1 -------------|           |

(TCIL currently on)                    |

| --- ELSE ---

FLG_TCS = 1 ----------------|           |

(overdrive lockout mode)   |AND -|     |

|     |     |

TCILTMR <= TCIL_TM_DLY -----|     |OR --|  (turn TCIL on)

|     | TCIL_STATE = 1

TCIL_FLASH_TMR >= TCILTM1 --|     |     | TCIL_FLASH_TMR = 0

|AND -|     |

TCIL_STATE = 0 -------------|           |

(TCIL currently off)                   |

| --- ELSE ---

|

| Do not change TCIL_STATE

| Increment TCIL_FLASH_TMR

P0750MALF = 0 --|

|AND -|

P0755MALF = 0 --|     |

|OR --|

SOL_CKT_SW = 0 -------|     |

|

P0743MALF = 0 --------|     |

|OR --|

TCC_CKT_SW = 0 -------|     |AND -| tr_out_flash = 0

|     | ;No transmission output

P1746MALF = 0 --------|     |     | ;faults (for which flashing

|OR --|     | ;the TCIL is cal'ed in)

EPC_ERR_SW = 0 -------|     |     | ;have been detected

|     |

P1747MALF = 0 --------|     |     |

|OR --|     |

EPC_SHORT_SW = 0 -----|           |

| --- ELSE ---

|

| tr_out_flash = 1

| ;Flash TCIL for detected trans

| ;output malfunction

25-8

TRANSMISSION INPUT CONVERSIONS, TRANSMISSION CONTROL INDICATOR LIGHT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SFT_ERR_SW = 0 -------|

|OR --|

SFT_FM_FLG = 0 -------|     |

|

P1743MALF = 0 --------|     |

|OR --|

SW_1743_TCIL = 0 -----|     |

|

P0741MALF = 0 --------|     |

|OR --|AND -| tr_fn_flash = 0

SW_0741_TCIL = 0 -----|     |     | ;no transmission functional

|     | ;faults (for which flashing

FLG_DIS_CSM = 0 ------|     |     | ;the TCIL is cal'ed in) have

|OR --|     | ;been detected

CNET_ERR_SW = 0 ------|     |     |

|     |

OTEMP_FM_FLG = 0 -----|     |     |

|OR --|     |

OTEMP_ERR_SW = 0 -----|           |

| --- ELSE ---

|

| tr_fn_flash = 1

| ;Flash TCIL for detected trans

| ;functional malfunction

TOT_ERR_SW = 0 -------|

|OR --|

TOT_FM_FLG = 0 -------|     |

|

PDL_ERROR = 0 --------|     |

|OR --|

TRS_ERR_SW = 0 -------|     |

|

P0503MALF = 0 --------|     |

|OR --|

VS_NOISE_SW = 0 ------|     |

|AND -| tr_in_flash = 0

P0500MALF = 0 --------|     |     | ;no transmission input

|OR --|     | ;faults (for which flashing

VSS_ERR_SW = 0 -------|     |     | ;the TCIL is cal'ed in)

|     | ;have been detected

P0720MALF = 0 --------|     |     |

|OR --|     |

OSS_ERR_SW = 0 -------|     |     |

|     |

P0721MALF = 0 --------|     |     |

|OR --|     |

OS_NOISE_SW = 0 ------|           |

| --- ELSE ---

|

| tr_in_flash = 1

| ;Flash TCIL for detected trans

| ;input malfunction

25-9

TRANSMISSION INPUT CONVERSIONS, TRANSMISSION CONTROL INDICATOR LIGHT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TCILTMR LOGIC

CRKFLG = 1 -----------------------|

|

tr_in_flash = 0 ------------|     |OR --| TCILTMR = 0

|     |     | ;Conditions to flash

tr_out_flash = 0 -----------|AND -|     | ;the TCIL not met, zero

|           | ;flashing mode entry delay

tr_fn_flash = 0 ------------|           | ;timer

unconditionally ------------------------| output(TCIL_STATE)

|

| (Turn TCIL on/off based

|  on TCIL_STATE)

25-10

TRANSMISSION INPUT CONVERSIONS, TRANSMISSION CONTROL SWITCH - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

25.3  TRANSMISSION CONTROL SWITCH (CDAM0)

OVERVIEW

The momentary contact Transmission Control Switch allows  the  driver  to  1)

select an alternate shift pattern or 2) lockout overdrive (fourth gear).  The

function of the Transmission Control Switch is dependent on the  application.

On  each  power-up,  the  state  of  the  Transmission  Control  Switch is 1)

alternate shift pattern disabled or 2) overdrive lockout disabled.

In addition to the TCS input processing logic, this module contains TCS  self

test.   During  engine  running self test the operator is required to depress

the Transmission Control Switch.  The TCS  circuit  test  monitors  the  ITCS

register for the states corresponding respectively to the open and the closed

states of the switch.  If, during the course of the KOER test sequence,  both

states  are  not  recognized,  code  1780 will be output diring the KOER code

output.

DEFINITIONS

INPUTS

Registers:

-  ER_STATUS = Defines current position in ER  sequence.   Control  of  this

psuedo  parameter  passes  between the ER sequencer and the individual ER

tests.

-  TM_TCS_RES = Transmission Control switch input residence timer, sec.

-  TSTRAT  =  Transmission  strategy  switch;  The  TSTRAT  software  switch

controls which transmission control strategy is executed:

0 -> No transmission control (Manual trans., AOD, ATX, C6, C3, etc.)

1 -> SIL (Shift indicator light)

2 -> A4LD with 3-4 shift control and converter clutch control

3 -> AXOD

4 -> C64E (E4OD)

5 -> A4LDE

6 -> FAX-4

7 -> AOD-E (AOD-I)

8 -> 4EAT

9 -> CD4E

-  VSBART_RT = Filtered vehicle speed compensated for N/V.

Bit Flags:

-  CCM_ER_ENA = Engine Running CCM test enable flag; 1 -> Engine Running CCM

test enabled.

-  FLG_FRST_TCS = Flag used to prevent multiple toggles of FLG_TCS during  a

single  activation of TCS button; 0 -> FLG_TCS has not been toggled, 1 ->

FLG_TCS has been toggled.

25-11

TRANSMISSION INPUT CONVERSIONS, TRANSMISSION CONTROL SWITCH - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  ITCS = Transmission Control switch input state; 0 -> TCS depressed, 1  ->

TCS not depressed.

-  OPER_RSP_CHK = Operator response check flag; 1 -> Engine running response

check  over,  output  code  if  operator response was not observed during

KOER.

-  TCS_OPEN = 1 -> the LOW voltage state of the TCS input pin on the LSI has

been recognized during the KOER TCS test.

-  TCS_SHORT = 1 -> the HIGH voltage state of the TCS input pin on  the  LSI

has been recognized during the KOER TCS test.

Calibration Constants:

-  ER_INIT = Value for state of ER_STATUS when  engine  running  test  first

inits.

-  TMTCS = Transmission Control switch residence time, sec.

-  TCS_TST_ENA = TCS circuit test enable switch; 1 -> enable.

-  VS_OD_CAN = Vehicle speed above which O/D cancel will not be allowed.

OUTPUTS

Registers:

-  TM_TCS_RES = See above.

Bit Flags:

-  FLG_FRST_TCS = See above.

-  FLG_TCS = Transmission control switch flag.

-  TCS_OPEN = See above.

-  TCS_SHORT = See above.

OTHER

-  clear_malf(Pxxx) = Logic process, clear  malfunction  flag  indicated  by

Pxxx.

-  tcs_codes = Set of Transmission Control Switch fault codes {P1780}.

-  store_code(Pxxx) = Stores the fault code specified by Pxxx.

-  P1780 = Fault code; TCS did  not  change  states  during  engine  running

on-demand self test.

25-12

TRANSMISSION INPUT CONVERSIONS, TRANSMISSION CONTROL SWITCH - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INTRN_TCS_OBDII_COM1

At power-up initialize the following parameter:

initially -------------------------------| FLG_TCS = 0

Once per background the following logic is executed:

TSTRAT <= 3 ------------------------------| Exit module

(Manual/non-electric transmission)       |

| --- ELSE ---

|

ITCS = 1 ---------------------------------| Allow TM_TCS_RES to count up

(TCS button depressed, 12 volts)         |

| --- ELSE ---

|

ITCS = 0 ---------------------------------| TM_TCS_RES = 0

(normal state, 0 volts)                  |  (zero residence timer)

| FLG_FRST_TCS = 0

|  (clear first pass flag)

VSBART_RT > VS_OD_CAN --------------------| FLG_TCS = 0

|

| --- ELSE ---

|

TM_TCS_RES >= TMTCS ----------------|     |

(button depressed long enough)     |AND -| Toggle FLG_TCS

|     |  (change TCS state)

FLG_FRST_TCS = 0 -------------------|     | FLG_FRST_TCS = 1

(1st time to toggle TCS)                 |  (set first pass flag)

25-13

TRANSMISSION INPUT CONVERSIONS, TRANSMISSION CONTROL SWITCH - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The on-demand TCS test requires the self test  operator  to  toggle  the  TCS

during  engine running self test.  The logic works as follows - during engine

running initialization the TCS open/short flags are cleared.  Code P1780 will

be  set  if  both  states are not observed during the operator response check

portion of engine running self test.

ER_STATUS = ER_INIT ----------------------| TCS_SHORT = 0

(During KOER initialization)             | TCS_OPEN = 0

| (Initialize TCS

|  test parameters)

|

| --- ELSE ---

CCM_ER_ENA = 0 ---------------|           |

(Engine Runing CCM disabled) |AND -|     |

|     |     |

OPER_RSP_CHK = 0 -------------|     |     |

(Operator response check code      |OR --| Exit module

output disabled)                  |     | (Engine running CCM

|     |  not in progress, or TCS

TCS_TST_ENA = 0 --------------------|     |  test disabled)

(TCS test cal'ed out)                    |

| --- ELSE ---

|

ITCS = 1 ---------------------------------| TCS_SHORT = 1

(TCS button depressed, 12 volts)         | (TCS closed state has

|  been monitored during

|  engine running self test)

|

| --- ELSE ---

|

| TCS_OPEN = 1

| (TCS open state has

|  been monitored during

|  engine running self test)

OPER_RSP_CHK = 1 -------------------|

(Operator response check code      |

output enable)                    |

|AND -| store_code(P1780)

TCS_SHORT = 0 ----------------|     |     |

(TCS closed state has not    |     |     | (Operator response check

been observed)              |OR --|     |  is over and both TCS states

|           |  have not been observed, set

TCS_OPEN = 0 -----------------|           |  code 1780)

(TCS open state has not                  |

been observed)                          |

| --- ELSE ---

|

| clear_malf(P1780)

25-14

TRANSMISSION INPUT CONVERSIONS, TORQUE NET CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

25.4  TORQUE NET CALCULATION (CDAN0)

OVERVIEW

This module calculates the brake torque available at the engine  output.   It

is  based  on a base torque value, modified for spark deviation from MBT, for

fuel deviation from stoichiometry, for engine friction, for  accessory  loads

and for A/C.  It also calculates the torque at the input to the transmission.

If that torque TQ_INP_NET exceeds the limit TQ_LIMIT, then a torque reduction

ratio is calculated.  This value is used by spark to clip spark.

DEFINITIONS

Registers:

-  ECT = Engine coolant temperature, deg F.

-  GR_CM = Commanded gear for shift solenoids.

-  INJ_TR = Torque ratio obtained from turning off a number of injectors.

-  LOAD = Universal load as ratio of air charge over standard.

-  N = RPM.

-  SAFTOT = The SAF spark value after modification by  foreground  strategy,

deg.

-  SPD_RATIO = Speed Ratio Across Torque Converter (Output / Input).

-  SPK_LAMBSE = Value of LAMBSE to be used in spark calculations, unitless.

-  SBK_M_B_T = Spark advance required to achieve maximum brake torque.

-  TQ_BAR = Filtered TQ_NET.

-  TQ_BAR_LST = Last pass value of TQ_BAR

-  TQ_INP_NET = Transmission input shaft torque.

-  TQ_LIMIT = Gear specific transmission input torque limit.

-  TQ_MBT = Maximum gross torque that could be produced by engine.

-  TQ_NET = Net torque into torque converter.

-  TQ_NET_LST = Last pass value of TQ_NET.

-  TQ_NET_S = Torque net from engine (signed).

-  TR_SPK_DELTA = Current torque ratio as  determined  from  spark  advance,

which is being output by engine.

25-15

TRANSMISSION INPUT CONVERSIONS, TORQUE NET CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TR_TQ_LIMIT = Torque ratio required to achieve TQ_LIMIT.

Bit flags:

-  ACCFLG = A/C engaged flag; 1 -> A/C engaged, 0 -> A/C disengaged.

-  FLG_TQ_TRUN = Apply Torque limit when set.

-  REVFLG = AXOD reverse flag; 1 -> reverse.

Calibration Constants:

-  FN617 = Torque converter torque ratio.

-  FN618A = Engine accessory torque without A/C.

-  FN619A = A/C accessory torque.

-  FN623 = Fuel multiplier used in torque calculation.

-  FN766 = Transfer function which calculates the torque ratio at  which  an

engine is operating from the spark delta from MBT.

-  FN1615A = Indicated engine torque for TQ_NET calculation

-  FN1616 = Engine friction torque for TQ_NET calculation;  X  =  normalized

engine speed in rpm.

-  FNFRIC_TQ(N,ECT) = Cold multiplier to the torque loss calculations.

X = FN070C(N) = Normalizing function for engine speed.

Y = FN014(ECT) = Normalizing function for ECT.

-  FNTQ_MAX = Gear specific transmission input torque limit.

-  TCTQ_BAR = Time constanty for TQ_BAR.

25-16

TRANSMISSION INPUT CONVERSIONS, TORQUE NET CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INTRN_TRANS_TQ_NET_CALC_COM12

ACCFLG = 1 ------------| ac_torque = FN619A(N)

(A/C on)              |

| --- ELSE ---

|

| ac_torque = 0

always ----------------| TR_SPK_DELTA = FN766(SPK_M_B_T - SAFTOT)

always ----------------| tq_mbt = FN1615A(N,LOAD) * FN623(SPK_LAMBSE) *

|                INJ_TR

| tq_loss = FN1616(N,LOAD) + FN618A(N)

|            + ac_torque + FNFRIC_TQ(N,ECT)

| TQ_MBT = tq_mbt - tq_loss

| tq_inp_mbt = max(TQ_MBT * FN617(SPD_RATIO), 0.0)

| TQ_NET_S = tq_mbt * TR_SPK_DELTA - tq_loss

| TQ_NET_LST = TQ_NET

| TQ_NET = max(TQ_NET_S, 0)

| TQ_INP_NET = TQ_NET * FN617(SPD_RATIO)

| TQ_BAR_LST = TQ_BAR

| TQ_BAR = ROLAV(TQ_NET,TCTQ_BAR)

REVFLG = 1 ------------| fn_index = 0

|

| --- ELSE ---

|

| fn_index = GR_CM

always ----------------| TQ_LIMIT = FNTQ_MAX[fn_index]

TQ_LIMIT < tq_inp_mbt -| tq_eng = TQ_LIMIT / FN617(SPD_RATIO) + tq_loss

| TR_TQ_LIMIT = min(tq_eng / tq_mbt, 1.0)

| FLG_TQ_TRUN = 1

|

| --- ELSE ---

|

| TR_TQ_LIMIT = 1.0

| FLG_TQ_TRUN = 0

25-17

TRANSMISSION INPUT CONVERSIONS, VS_RATE CALCULATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

25.5  VS_RATE CALCULATIONS (CDAF0)

OVERVIEW

Vehicle acceleration rate VS_RATE is  calculated  by  differentiating  VSBART

with  respect  to  time.   This  is  to  be  used  in calibrating the prevent

powertrain hunting strategy.

The calculation is performed as follows:  Every time VSBARTL is updated,  the

time at which this occurs is stored in VS_SMPL_TM.  Every background loop the

acceleration is computed using the current and previous values of VSBARTL and

VS_SMPL_TM   (backwards   difference  approximation).   The  result  is  then

converted to MPH/seconds, filtered and the  current  values  of  VSBARTL  and

VS_SMPL_TM   become   the   previous  values  in  preperation  for  the  next

calculation.

DEFINITIONS

INPUTS

Registers:

-  VSBARTL = Word value of vehicle speed for transmission use, MPH.

-  VSBARTL_PREV = Previous value of VSBARTL.

-  VS_SMPL_TM = Value in the MS_CLOCK at the time VSBART is calculated.

Calibration Constants:

-  TCVSR = Time constant for vehicle acceleration.

-  TCVSRPH = Time constant for vehicle acceleration.

-  TCVSRSS = Time constant for vehicle acceleration used for shift  schedule

compensation.

-  VSRATE_MAX = Maximum allowable vehicle acceleration to be used  in  shift

schedule compensation.

-  VSRATE_MIN = Minimum allowable vehicle acceleration to be used  in  shift

schedule compensation.

OUTPUTS

Registers:

-  VSBARTL_PREV = See above.

-  VS_RATE = Filtered vehicle acceleration rate.

25-18

TRANSMISSION INPUT CONVERSIONS, VS_RATE CALCULATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  VS_RATEPH = Filtered vehicle acceleration rate.

-  VS_RATE_SS = Filtered vehicle acceleration rate used for  shift  schedule

compensaton.

-  VSSMPLTM_PREV = See above.

25-19

TRANSMISSION INPUT CONVERSIONS, VS_RATE CALCULATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INTRN_CALC_VS_RATE_COM1

unconditionally -----| vs_smpl_prd = (VS_SMPL_TM - VSSMPLTM_PREV) / 1024

|  (time since last calculation in seconds)

| vsr = [(VSBARTL - VSBARTL_PREV) / vs_smpl_prd]

|  (instantaneous vehicle acceleration in MPH/sec)

| VS_RATE = ROLAV(vsr,TCVSR,vs_smpl_prd)

| VS_RATEPH = ROLAV(vsr,TCVSRPH,vs_smpl_prd)

| vs_rate_ss = ROLAV(vsr,TCVSRSS,vs_smpl_prd)

| VS_RATE_SS = min(VSRATE_MAX,

|   (max(vs_rate_ss,VSRATE_MIN))

| VSSMPLTM_PREV = VS_SMPL_TM

| VSBARTL_PREV = VSBARTL

25-20

TRANSMISSION INPUT CONVERSIONS, SHIFT SCHEDULE COMPENSATION CALCULATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

25.6  PREDICTIVE SHIFT SCHEDULE COMPENSATION CALCULATIONS (CDAN0)

OVERVIEW

This module calculates the predictive vehicle used to  compensate  the  shift

schedules  for  system delays (e.g.  solenoid delay times between command and

start of a shift).

Both the upshift and downshift predicted vehicle speeds are calculated.   The

predicted  values  are calculated using the associated delay times based from

the current gear.

Predictive engine speed is also calculated here for  engine  based  upshifts.

The  predicted  values are calculated using background loop, hardware delays,

verify time, and engine acceleration.  It can be calibrated out with a switch

if desired.

The base hardware delay times should be calibrated when the transmission  has

reached  normal  operating  temperature.  A scalar multplier as a function of

TOT is provided to compensate the hardware delay times.

DEFINITIONS

Registers:

-  BG_TMR = Background loop time.

-  DNDT_PSS = Engine acceleration for predictive shift scheduling.

-  GR_CM = Commanded gear.

-  GR_DS = Desired transmission gear.

-  GR_DS_LST = Desired gear in last background pass.

-  NEBART = Engine speed.

-  NEBART_PU = Predicted engine speed for an upshift from current gear.

-  RT_NOVS = Ratio of actual N/V to base N/V stored.

-  TM_12_AKD = Timer register to track time in  1st  gear  after  a  3-1/4-1

kickdown.

-  TOT = Transmission oil temperature (degrees F).

-  TP_REL = Relative TP (TP - RATCH).

-  VSBART = Filtered vehicle speed for transmission in MPH.

-  VSBART_RT = VSBART / RT_NOVS (MPH).

-  VSBART_RT_PD = Predicted vehicle speed for a downshift from current gear.

25-21

TRANSMISSION INPUT CONVERSIONS, SHIFT SCHEDULE COMPENSATION CALCULATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  VSBART_RT_PU = Predicted vehicle speed for an upshift from current gear.

-  VS_RATE_SS = Filtered Vehicle acceleration rate used for  shfit  schedule

compensation.

Bit FLags:

-  FLG_TIP_OUT = Flag which indicates a tip-out upshift in progress; 0 -> no

tip-out upshift in progress, 1 -> a tip-out upshift in progress.

-  FLG_W12_AKD = Flag  indicating  that  a  WOT  1-2  after  a  kickdown  is

possible; 1 -> WOT 1-2 after a kickdown possible.

Calibration Constants:

-  FNTOTDLY =  Transmission  temperature  compensation  for  hardware  delay

times, scalar multiplier.

-  MAXNEPU = Maximum predictive engine speed.  Clip to this value  -  engine

cannot accelerate more than this.

-  NEPUMIN = Minimum engine speed to allow  predictive  engine  speed  based

upshifts to insure reliable engine acceleration rate calculation.

-  NEPU_SW = Switch to allow predictive engine speed based upshifts.

-  TMDN2DLY = Shift hardware delay time associated  with  a  downshift  from

2nd.

-  TMDN3DLY = Shift hardware delay time associated  with  a  downshift  from

3rd.

-  TMDN4DLY = Shift hardware delay time associated  with  a  downshift  from

4th.

-  TMUP1DLY = Shift hardware delay time associated with an upshift from 1st.

-  TMUP2DLY = Shift hardware delay time associated with an upshift from 2nd.

-  TMUP3DLY = Shift hardware delay time associated with an upshift from 3rd.

-  TMVEDN = Time to delay desired automatic downshift, i.e.   hold  original

desired gear for this time, sec.

-  TMVETOUP = Time to delay desired automatic tip-out  upshift,  i.e.   hold

original desired gear for this time, sec.

-  TMVEUP = Time to delay desired automatic  upshift,  i.e.   hold  original

desired gear for this time.  units:  secon.

-  TMW12DLY = Shift hardware delay time associated  with  a  WOT  1-2  shift

after a kickdown.

-  TM12AKD = Time threshold in 1st gear to disable compensation for WOT  1-2

after a 3-1/4-1 kickdown, secs.

-  TPNEPUMIN = Minimum throttle position to allow  predictive  engine  speed

based upshifts to insure reliable engine acceleration rate calculation.

25-22

TRANSMISSION INPUT CONVERSIONS, SHIFT SCHEDULE COMPENSATION CALCULATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TP12AKD = Threshold to determine high throttle condition, counts.

-  TP12HYS = Hysteresis value to determine a low throttle condition, counts.

-  VS31MIN = Minimum vehicle speed to  trigger  the  WOT  1-2  after  a  3-1

kickdown, mph.

PROCESS

STRATEGY MODULE: INTRN_SHFT_SCHLD_COMP_CALC_COM4

FLG_TIP_OUT = 1 ---------------------| tmversftss = TMVETOUP

|

| --- ELSE ---

|

| tmversftss = TMVEUP

Update the high throttle 1-2 after a 3-1/4-1 kickdown possible flag:

TP_REL > TP12AKD --------------|

(high throttle)               |

|

GR_DS = 1 ---------------------|AND -| FLG_W12_AKD = 1

|     | TM_12_AKD = TM12AKD

GR_DS_LST >= 3 ----------------|     | (update flag/1st gear timer for

|     |  a high TP 1-2 after a 3-1/4-1

VSBART_RT > VS31MIN -----------|     |  kickdown)

|

| --- ELSE ---

TP_REL < TP12AKD - TP12HYS ----|     |

|OR --| FLG_W12_AKD = 0

TM_12_AKD = 0 -----------------|     |

| --- ELSE ---

|

| no action

25-23

TRANSMISSION INPUT CONVERSIONS, SHIFT SCHEDULE COMPENSATION CALCULATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLG_W12_AKD = 1 ---------------|

|AND -| hrdupdly = TMW12DLY * FNTOTDLY(TOT)

GR_CM = 1 ---------------------|     |

| VSBART_RT_PU = VSBART_RT + [RT_NOVS

|   * ((VS_RATE_SS * (tmversftss

|   + hrdupdly + BG_TMR)))]

|

| ne_rate = min(MAXNEPU,DNDT_PSS)

|

| --- ELSE ---

|

GR_CM < 4 ---------------------------| hrdupdly = TMUP[GR_CM]DLY *

|                FNTOTDLY(TOT)

|

| VSBART_RT_PU = VSBART_RT + [RT_NOVS

|   * ((VS_RATE_SS * (tmversftss

|   + hrdupdly + BG_TMR)))]

|

| ne_rate = min(MAXNEPU,DNDT_PSS)

GR_CM > 1 ---------------------------| hrddndly = TMDN[GR_CM]DLY *

|               FNTOTDLY(TOT)

| VSBART_RT_PD = VSBART_RT + [RT_NOVS *

|   ((VS_RATE_SS * (TMVEDN + hrddndly

|     + BG_TMR)))]

GR_CM < 4 ---------------------|

|

NEBART > NEPUMIN --------------|

(engine speed high enough     |

for a reliable rate)         |

|AND -| NEBART_PU = NEBART + (ne_rate *

TP_REL > TPNEPUMIN ------------|     |  (tmversftss + hrdupdly + BG_TMR))

(TP high enough for a         |     |

reliable rate)               |     |

|     |

NEPU_SW = 1 -------------------|     |

(predictive cal'ed in for           |

engine based shifts)               |

| --- ELSE ---

|

| NEBART_PU = NEBART

25-24

TRANSMISSION INPUT CONVERSIONS, TRANSMISSION SELF TEST ENABLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

25.7  TRANSMISSION SELF TEST ENABLE (CDAM0)

OVERVIEW

As transmission operating temperature decreases, shift times  and  engagement

times  increase (due to changes in fluid viscosity).  Certain portions of the

transmission control and self test strategy (learned  N/V,  inferred  vehicle

motion,  gear ratio validity) are disabled when shifts and/or engagements are

in progress based on timers.  Theses timers are not valid at  low  (sub-zero)

TOT.

A transmission self test enable flag (TST_ENA_FLG) will be set when it is  ok

to execute the above mentioned transmission control strategy.  This flag will

be set based on transmission operating temperature (TOT  >=  TST_TOT_MIN)  if

TOT  is not failed (FFG_TOT = 0), or a minimum time since the engine has been

running (RUN_TMR >=  TST_ENA_TM)  and  engine  coolant  temperature  (ECT  >=

TST_ECT_MIN).

NOTE:  the RUN_TMR/ECT  condition  also  protects  for  in  range  (low)  TOT

failures,  which  is  why  the  TOT failure mode flag is not checked for this

condition.

DEFINITIONS

INPUTS

Registers:

-  ECT = Engine Coolant Temperature, deg F.

-  TOT = Transmission Oil Temperature, deg F.

-  RUN_TMR = Time since entering "RUN" mode.

Bit Flags:

-  FFG_TOT = OBD-II system FMEM flag for TOT; 1  ->  TOT  is  currently  not

reliable.

Calibration Constants:

-  TST_DIS_TIM = Maximum time transmission self test will be  disable  based

on TOT.

-  TST_ECT_MIN = Minimum ECT to allow transmission self test  entry  if  TOT

fails.

-  TST_TOT_MIN = Minimum TOT to allow transmission self test entry.

25-25

TRANSMISSION INPUT CONVERSIONS, TRANSMISSION SELF TEST ENABLE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OUTPUTS

Bit Flags:

-  TST_ENA_FLG = Transmission self test enable  flag;  1  ->  TOT  dependent

trans logic enabled.

PROCESS

STRATEGY MODULE: INTRN_TST_ENA_COM1

FFG_TOT = 0 ----------------|

(TOT is reliable)          |AND -|

|     |

TOT >= TST_TOT_MIN ---------|     |

(TOT high enough to enable       |

trans self test)                |OR --| TST_ENA_FLG = 1

|     | (Enable TOT dependent

ECT >= TST_ECT_MIN ---------|     |     |  transmission logic)

(ECT high enough to enable |     |     |

trans self test)          |AND -|     |

|           |

RUN_TMR >= TST_DIS_TIM -----|           |

(Max. time to disable trans self       |

test based on TOT has passed)         |

| --- ELSE ---

|

| TST_ENA_FLG = 0

| (Conditions not met to

|  enable TOT dependent

|  transmission logic)

25-26

TRANSMISSION INPUT CONVERSIONS, RT_NOVS_KAM CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

25.8  RT_NOVS_KAM CALCULATION (CDAJ0)

OVERVIEW

This module calculates RT_NOVS_KAM, based on filtered output shaft speed  and

vehicle  speed.   The  calculation  occurs  once  after each power up and KAM

initialization.  The calculation occurs when vehicle speed is in  a  reliable

area,  and  no  related  sensors  have failed.  The N / V ratio is calculated

NOVCNT times and as long as the  samples  calculated  are  approximately  the

same, the average is taken and stored in KAM.

NOTE:  The calibration constant NVBASE_X_NVC = Base N / V * NVCNT.   Choosing

the  constant  this  way  prevents  the  processor from having to perform the

multiplication during execution.

DEFINITIONS

INPUTS

Registers:

-  N = Engine speed, RPM.

-  NOBART = Filtered Output Shaft Speed in RPM.

-  NOV_ACT = Actual computed N / V.

-  NOV_ACT_LST = Last pass value of NOV_ACT.

-  NOVCTR = Number of N / V calculations counter.

-  NOVSUM = Running total of valid N / V samples.

-  RT_NOVS_KAM = Ratio of actual N / V to base N / V.

-  TM_NOV_CALC = Time since last NOV_ACT calculation.

-  VSBARV = Filtered Vehicle Speed, MPH.

-  VSCTR = Count of MPH sensor errors.

Bit Flags:

-  FLG_4X4L = 4x4 low flag.

-  FLG_FRST_NOV = 0 -> A new RT_NOVS KAM value is needed for KAM.

-  FLG_NEW_NOV = New NOV_ACT calculation flag.

-  OSFMFLG = Output shaft speed sensor FMEM flag.

-  VSFMFLG = Vehicle speed sensor FMEM flag.

25-27

TRANSMISSION INPUT CONVERSIONS, RT_NOVS_KAM CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  NOVCNT = Minimum number of consecutive, good N / V  samples  required  to

update KAM.

-  NOVDIF = Maximum difference between N / V samples, RPM / MPH.

-  NVBASE_X_NVC = Base N / V * NOVCNT.

-  RT4X4L = 4 x 4 low transfer case ratio units.

-  RTNVMN = Minimum valid RT_NOVS_KAM value.

-  RTNVMX = Maximum valid RT_NOVS_KAM value.

-  TMRTVSCAL = Time between consecutive NOV calculations.

-  VSRTVSMN = Minimum vehicle speed to calculate N / V.

OUTPUTS

Registers:

-  NOV_ACT = See above.

-  NOV_ACT_LST = See above.

-  NOVCTR = See above.

-  NOVSUM = See above.

-  RT_NOVS = Ratio of actual N / V to base N / V stored.

-  RT_NOVS_KAM = See above.

-  TM_NOV_CALC = See above.

Bit Flags:

-  FLG_FRST_NOV = See above.

-  FLG_NEW_NOV = See above.

-  FLG_NOV_KAM = Flag indicating at least  one  update  of  RT_NOVS_KAM  has

occurred since KAM initialization.

25-28

TRANSMISSION INPUT CONVERSIONS, RT_NOVS_KAM CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INTRN_RT_NOVS_KAM_CALC_COM2

FLG_FRST_NOV = 0 ----------------------|

(need to calculate N / V)             |

|

VSBARV >= VSRTVSMN --------------------|

(vehicle speed signal is reliable)    |

|

TM_NOV_CALC >= TMRTVSCAL --------------|AND -| NOV_ACT_LST = NOV_ACT

(enough time since last calculation)  |     |  (update last pass NOV

|     |   calculation)

VSFMFLG = 0 ---------------------------|     | NOV_ACT = NOBART / VSBARV

|     |  (compute actual NOV)

OSFMFLG = 0 ---------------------------|     | TM_NOV_CALC = 0

|     |  (reset interval pacer)

VSCTR = 0 -----------------------------|     | NOVCTR = NOVCTR + 1

|  (increment sample counter)

| FLG_NEW_NOV = 1

|  (New NOV_ACT occurred)

|

|  --- ELSE ---

|

| FLG_NEW_NOV = 0

|  (No new NOV_ACT)

FLG_NEW_NOV = 1 -----------------------|

(new N / V actual this pass)          |AND -| NOVCTR = 0

|     | (reset sample counter)

ABS(NOV_ACT_LST - NOV_ACT) > NOVDIF ---|     | NOVSUM = 0

(too much variation in NOV calculations)    | (reset running total)

|

| --- ELSE ---

|

FLG_NEW_NOV = 1 -----------------------------| NOVSUM = NOVSUM + NOV_ACT

NOVCTR = NOVCNT -----------------------|

(enough consecutive matches)          |

|

FLG_FRST_NOV = 0 ----------------------|AND -| RT_NOVS_KAM = NOVSUM /

(new value for KAM)                   |     |               NVBASE_X_NVC

|     |  (store new NOV ratio in KAM)

RTNVMN <= (NOVSUM/(NVBASE_X_NVC))      |     | RT_NOVS = RT_NOVS_KAM

<= RTNVMX -----------|     | FLG_FRST_NOV = 1

(reasonable value for N / V has been        |  (do not store another)

calculated)                                | FLG_NOV_KAM = 1

|  (indicate a new update since

|   KAM initialization)

25-29

TRANSMISSION INPUT CONVERSIONS, RT_NOVS_KAM CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLG_4X4L = 1 --------------------------------| RT_NOVS = RT_NOVS_KAM *

|            RT4X4L

|  (modify shift schedule by

|   transfer case ratio)

|

| --- ELSE ---

|

| RT_NOVS = RT_NOVS_KAM

25-30

INPUT CONVERSIONS AND FILTERS, AODE TRANSMISSION CALCULATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

25.9  AODE TRANSMISSION CALCULATIONS (CDAL0)

OVERVIEW

These System Equations are used in the AODE Transmission Calculation Process.

DEFINITIONS

INPUTS

Registers:

-  AM = Air Mass flow through the throttle body, lb/min.

-  BP = Barometric pressure as defined in the Inferred BP Section.

-  ENGCYL = Number of injections per engine revolution = 2, 3, 4 for  4,  6,

and 8-cylinder engines respectively.

-  N = Engine RPM.

-  NEBART = Filtered engine RPM for transmission.

-  NOBART = Filtered transmission output shaft speed.

-  NTBART = Filtered transmission turbine speed, rpm.

-  RATCH = Closed throttle position, counts.

-  RT_GR_CUR = Current transmission gear ratio.

-  TP = Throttle Position, counts.

-  TPBART = Filtered throttle position for transmission.

-  TP_REL = Relative TP, (TP - RATCH).

Calibration Constants:

-  SARCHG = Standard Aircharge = 4.4256E-05 * CID/# of cylinders.

-  TCNE = Time constant for filtered RPM.

-  TCTPREL = Time constant for filtered TP_REL, sec.

-  TCTPTE = Time constant for filtered TP.

-  TCTPTV = Time constant for filtered TP for TV pressure.

25-31

INPUT CONVERSIONS AND FILTERS, AODE TRANSMISSION CALCULATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OUTPUTS

Registers:

-  ARCHG = See above.

-  BP_INTR = BP interpolation factor = FN615(BP).

-  LOAD = Nondimensional, generic engine load.

-  NEBART = See above.

-  NTBART = See above.

-  SLIP_ABS = Absolute value of SLIP = NEBART - NTBART, rpm.

-  SPD_RATIO = Speed ratio across torque converter.

-  TPBART_PWF = Filtered throttle position for performance and normal.

-  TP_RATE = Throttle rate.

-  TPRATE_PWF = Throttle position rate for performanc and normal.

-  TP_REL = See above.

-  TP_REL_BAR = Filtered TP_REL (TP - RATCH).

-  TPBART = See above.

25-32

INPUT CONVERSIONS AND FILTERS, AODE TRANSMISSION CALCULATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INTRN_AODE_INPUT_EQNS_COM1

***********************************************

* FILTERED ENGINE SPEED FOR TRANSMISSIONS USE *

***********************************************

NEBART = ROLAV(N,TCNE)

***************************************************

* FILTERED THROTTLE POSITION FOR TRANSMISSION USE *

***************************************************

TPBART = ROLAV(TP,TCTPTE)

**************************

* THROTTLE POSITION RATE *

**************************

TP_RATE = TP - TPBART (TP_RATE is clipped to +/- 512 counts)

**************************************

* FILTERED THROTTLE POSITION FOR PNN *

**************************************

TPBART_PWF = ROLAV(TP,TCTPPWF)

**********************************

* THROTTLE POSITION RATE FOR PNN *

**********************************

TPRATE_PWF = TP - TPBART_PWF (TPRATE_PWF is clipped to +/- 512 counts)

25-33

INPUT CONVERSIONS AND FILTERS, AODE TRANSMISSION CALCULATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

***************************************

* FILTERED TRANSMISSION TURBINE SPEED *

***************************************

NTBART = NOBART * RT_GR_CUR

***************************

* BP INTERPOLATION FACTOR *

***************************

BP_INTR = FN615(BP)

***************************************

* SPEED RATIO ACROSS TORQUE CONVERTER *

***************************************

NTBART

SPD_RATIO =  ------

NEBART

**********************************************

* FILTERED THROTTLE POSITION FOR TV PRESSURE *

**********************************************

TPBARTV = ROLAV(TP,TCTPTV)

****************************************************************

* FILTERED THROTTLE POSITION FOR TORQUE INTERPOLATION STRATEGY *

****************************************************************

TP_REL_BAR = ROLAV(TP_REL,TCTPREL)

**************************

* ABSOLUTE VALUE OF SLIP *

**************************

SLIP_ABS = |NEBART - NTBART|

25-34

INPUT CONVERSIONS AND FILTERS, COLD SHIFT MULTIPLIER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

25.10  COLD SHIFT MULTIPLIER (CDAF0)

OVERVIEW

During cold weather starts/drives, the initial  shifts  of  the  transmission

seem  delayed.   This  impression  results from the decrease in engine torque

during the early part of the cold start.  Therefore, strategy which  computes

shift  schedules  as  a function of the initial Transmission Oil Temperature,

TOT, (temperature at the time of the start), to the current TOT is required.

The  cold  weather  strategy  output  CS_SFT_MULT,  is  also  used   in   the

unconditional  converter  clutch  unlock  strategy.   When CS_SFT_MULT is not

equal to one, the converter clutch is unconditionally  unlocked;  i.e.,  when

cold  transmission  conditions  exist, the converter clutch is kept unlocked.

Once CS_SFT_MULT gets set to 1.0 (transmission is warm),  the  check  on  TOT

will be disabled.

A time limit is placed upon the time allowed for the actual TOT to rise above

the initial TOT.

FLG_TQM_ENA is set based on a function of Initial TOT or ECT.  FLG_TQM_ENA is

used  to  limit the spark retard due to toarue modulation when the powertrain

is cold.

DEFINITIONS

INPUTS

Registers:

-  ECT = Engine coolant temperature, degrees F.

-  INIT_TOT = TOT at  start-up;  arithmetic  average  of  the  first  8  TOT

reading.

-  TOT = Transmission Oil Temperature, degrees F.

-  RUN_TMR = Time since entering RUN mode, sec.

Bit Flags:

-  FLG_TOT_WRM =  Flag  to  signal  transmission  is  sufficiently  warm  to

lock-up.

-  FLG_TQM_ENA = Flag to signal transmission is sufficiently warm to  permit

full spark retard due to torque modulation.

Calibration Constants:

-  CS_MAX_TIME = Maximum time allowed in cold start shift schedules.

-  CS_MULT = Cold Start Multiplier for all shifts.

-  ECT_TQM  =  Engine  coolant  temperature  required   to   permit   toruqe

modulation.

25-35

INPUT CONVERSIONS AND FILTERS, COLD SHIFT MULTIPLIER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FN690(INIT_TOT)  =  Temperature  required  to  leave  cold  start   shift

schedules.

-  FN690TQM(INIT_TOT) = Transmission  oil  temperature  required  to  permit

torque modulation.

OUTPUTS

Registers:

-  CS_SFT_MULT = Cold Start Shift Multiplier.

Bit Flags:

-  FLG_TQM_ENA = See above.

25-36

INPUT CONVERSIONS AND FILTERS, COLD SHIFT MULTIPLIER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INTRN_CSM_COM3

TOT < FN690(INIT_TOT) ------------|

|

RUN_TMR < CS_MAX_TIME ------------|AND -| CS_SFT_MULT = CS_MULT

|     |

FLG_TOT_WRM = 0 ------------------|     |

(once TOT reaches temperature,         |

do not check again)                   |

| --- ELSE ---

|

| CS_SFT_MULT = 1.0

| FLG_TOT_WRM = 1

TOT < FN690TQM(INIT_TOT) ---|

|OR --|

ECT < ECT_TQM --------------|     |

(Clip spark retard due to        |

torque modulation until the     |AND -| FLG_TQM_ENA = 0

powertrain warms up)            |     |

|     |

FLG_TQM_ENA = 0 ------------------|     |

| --- ELSE ---

|

| FLG_TQM_ENA = 1

25-37

TRANSMISSION INPUT CONVERSIONS, MLPS CONVERSION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

25.11  MLPS CONVERSION (CDAN0)

OVERVIEW

This strategy is a fully calibratable control strategy to allow the use of  a

six   position   Manual  Lever  Position  Sensor  (MLPS)  combined  with  the

Transmission Control Switch (TCS-if  present).   As  defined  by  TAPME,  all

transmissions  have  three forward positions, neutral, reverse and park.  The

three forward positions are overdrive, second (if TCS present) or  drive  (if

no TCS present) and first.  The second to last PRNDL position is calibratable

to second or drive using the MLPS_2 parameter discussed later.

The analog MLPS input is decoded into IPDL to give one of six positions.   In

this  decoding, deadbands can be calibrated between each position range.  The

calibration constants PARKHI, PARKLO, REVHI, REVLO, NEUHI, NEULO, ODHI, ODLO,

MLPS_2HI,   MLPS_2LO,   MAN1HI,  and  MAN1LO  are  the  parameters  for  each

corresponding PRNDL position band.  Anytime a MLPS  fault  is  detected  IPDL

will be set to zero.

A residence timer exists to delay passing the  MLPS  signal  through  to  the

control.   Anytime  a  change  in IPDL occurs the residence timer is reset to

PDL_TIM.  PDL may only change states after the residence timer has timed out.

If the residence timer reaches zero with a valid region present, IPDL will be

passed  into  PDL.  If the residence timer reaches zero in a failed condition

FMMMLP will be passed into PDL.

MLPS diagnostic testing distinguishes four separate failure modes:  in  range

failure,  (P0705), MLPS short to ground, (P0707), MLPS open circuit, (P0708),

and high vehicle speed observed in park, (P1706).  When any of the MLPS codes

are set, PDL_ERROR will be set, which tells the control to enter FMEM.

NOTE:  During EPC VFS short circuit FMEM (OFMFLG = 1), OD will  be  cancelled

to  prevent  failure  of  the  direct  clutch  .  The direct clutch carries a

greater percentage of input torque in 4th (compared to 3rd), and half fuel is

not enough to prevent the direct clutch from slipping in 4th gear.

25-38

TRANSMISSION INPUT CONVERSIONS, MLPS CONVERSION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Below is a diagram showing how the MLPS test parameters are related.

The logic starts  by  checking  for  the  presence  of  three  of  the  fault

conditions,  high  vehicle  speed  in park, the MLPS open circuit and finally

MLPS short to ground.  If any of these faults are present it is flagged, IPDL

set to zero and this logic exited.

Next the logic compares the current MLPS output (INDS) to the Park range,  if

INDS  is  between  PARKHI  and  PARKLO,  IPDL is set to PARK.  If INDS is not

within the PARK range, the strategy compares INDS to the REVERSE range  (then

NEUTRAL,  OD,...,MAN1).   As  soon  as a valid range is found, IPDL is set to

that position and this logic is exited.  If a valid range is not found,  IPDL

is set to zero.

If calibrating out one of the error  bands  is  desired,  the  parameters  on

either  side of the band should be set equal.  Example - to calibrate out the

band between Park and Reverse, PARKLO = 826 = REVHI would work

PRNDL POSITION    CALIBRATION PARAMETER  APPROXIMATE VALUE

OUT OF RANGE HI

_________ ------------ PARKHI              968

|         |

|  PARK   |

|_________|

------------ PARKLO              879

ERROR BAND

_________ ------------ REVHI               773

|         |

| REVERSE |

|_________|

------------ REVLO               701

ERROR BAND

_________ ------------ NEUHI               621

|         |

| NEUTRAL |

|_________|

------------ NEULO               537

ERROR BAND

_________ ------------ ODHI                468

|         |

|   OD    |

|_________|

------------ ODLO                385

ERROR BAND

_________ ------------ MLPS_2HI            314

|         |

| MLPS_2  |

|_________|

------------ MLPS_2LO            245

ERROR BAND

_________ ------------ MAN1HI              160

|         |

| MANUAL1 |

|_________|

------------ MAN1LO               60

OUT OF RANGE LO

25-39

TRANSMISSION INPUT CONVERSIONS, MLPS CONVERSION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  CCM_TST_ENA = OBDII Comprehensive Component test enable flag; 1  ->  test

enabled.

-  INDS = Input from manual lever position sensor (MLPS) in counts.

-  INDS_LST = Previous input from MLPS.

-  IPDL = Unverified PRNDL position.

-  IPDL_LST = Last unverified PRNDL position.

-  NDSFLG = Neutral/Drive Flag; 1 -> Drive.

-  PDL = Verified PRNDL position.

-  PDL_ERR_TMR = PRNDL malfunction test error timer.

-  PDL_LST = Last verified PRNDL value.

-  PDL_MON_TMR = MLPS monitor timer.

-  P0707FIL = MLPS short to ground fault filter.

-  P0708FIL = MLPS open circuit fault filter.

-  TM_PDL_RES = Input residence timer.

-  VS = Vehicle Speed.

Bit Flags:

-  CCM_EO_ENA = Key on engine off CCM test enabled; 1 -> enabled.

-  FLG_TCS = Transmission control switch flag.

-  OFMFLG = EPC VFS short circuit failure mode flag;  1  ->  EPC  VFS  short

circuti failure detected.

-  PARK_ERR = High vehicle speed in park error flag; 1 -> High vehicle speed

in park band error, 0 -> Vehicle speed in park range.

-  PDL_ERROR = PRNDL FMEM flag; 1 -> Enter FMEM for PRNDL, 0 -> Don't  enter

FMEM for PRNDL.

-  PDL_GND = MLPS short to ground present.

-  PDL_MON = MLPS monitored flag;  1  ->  MLPS  monitored,  0  ->  MLPS  not

monitored.

-  PxxxMALF = Malfunction flag for code Pxxx; 1 -> a  malfunction  currently

exists for fault Pxxx.

-  PDL_HGH = MLPS open circuit present.

25-40

TRANSMISSION INPUT CONVERSIONS, MLPS CONVERSION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  DLT_INDS = Minimum change in MLPS voltage which guarantees  a  shift  has

taken place.

-  FMMMLP = IPDL value when PDL_ERROR is present.

-  MLPS_KOEO_EN = MLPS KOEO test enable calibration switch; 1 -> test called

in.

-  MAN1HI = MLPS manual 1 position band high limit.

-  MAN1LO = MLPS manual 1 position band low limit.

-  MLPS_2 = IPDL value when MLPS is in the second to  last  position;  2  ->

Manual  2  has  been  selected  (TCS  present), 3 -> Drive mode/Overdrive

cancel has been selected (no TCS present).

-  MLPS_2HI = MLPS second to last position band high limit.

-  MLPS_2LO = MLPS second to last position band low limit.

-  NEUHI = MLPS neutral position band high limit.

-  NEUHI_EO = MLPS neutral position band high  limit  for  engine  off  self

test.

-  NEULO = MLPS neutral position band low limit.

-  NEULO_EO = MLPS neutral position band low limit for engine off self test.

-  ODHI = MLPS overdrive position band high limit.

-  ODLO = MLPS overdrive position band low limit.

-  PARK_ERR_TM = Park validity test error time, minimum time error  must  be

present to set code, sec.

-  PARKHI = MLPS park position band high limit.

-  PARKHI_EO = MLPS park position band high limit for engine off self test.

-  PARKLO = MLPS park position band low limit.

-  PARKLO_EO = MLPS park position band low limit for engine off self test.

-  PDL_ERR_TM = PRNDL malfunction test error time, minimum time  error  must

be present to set code.

-  PDL_MON_TM = Minimum amount of time MLPS sensor must be observed  to  set

the monitor flag.

-  PDLTIM = PRNDL load residence time.

-  PDL_LVL = Fault filter failure level.

-  PDL_UP = Fault filter up count value.

25-41

TRANSMISSION INPUT CONVERSIONS, MLPS CONVERSION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  REVHI = MLPS reverse postion band high limit.

-  REVLO = MLPS reverse position band low limit.

-  TSTRAT = Transmission Strategy Switch; The TSTRAT software switch selects

which transmission control strategy to be executed:

0 -> No transmission control (Manual trans., AOD,ATX,C6,C3, etc)

1 -> SIL (Shift Indicator Light)

2 -> A4LD with 3-4 shift control and converter clutch control.

3 -> AXOD

4 -> C64E(E40D)

5 -> A4lDE

6 -> FAX-4

7 -> AOD-E (AOD-I)

8 -> 4EAT

9 -> CD4E

-  VSPMIN = Indicate PRNDL error if PRNDL = park and vehicle  speed  is  GE,

this value (MPH).

OTHER

-  MLP_CODES = Set of (P0705, P0707, P0708, P1705, P1706) MLPS  OBDII  fault

codes.

-  NDS_CODES = Set of (P1709) fault codes.

-  P1709 = Fault code for vehicle not in park or neutral.

-  P0705 = Fault code for out of range.

-  P0707 = Fault code for MLPS short to ground.

-  P0708 = Fault code for MLPS open circuit.

-  P1705 = Fault code for MLPS indicating incorrect position during KOEO.

-  P1706 = Fault code for high vehicle speed observed in park.

25-42

TRANSMISSION INPUT CONVERSIONS, MLPS CONVERSION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INTRN_MLPS_CONV_COM9

As the PRNDL switch moves from position to position, the next contact is made

before  the current contact is broken.  This prevents sensing an open circuit

condition during PRNDL transitions.

At power-up initialize the following parameter:

initially ------------------------------| PDL = 7

| PDL_LST = 4

| IPDL = 5

Once per Background the following logic is executed:

always ---------------------------------| PDL_LST = PDL

| IPDL_LST = IPDL

TSTRAT >= 4 ----------------------------| Do: mlps_range_check

(Electronic Automatic Transmission     | Do: mlps_prndl_verification

present)

CCM_EO_ENA = 1 -------------------------| Do: mlps_eo_test

(Engine Off CCM test enabled           |

| --- ELSE ---

CCM_TST_ENA = 1 ------------------|     |

(Continuous CCM test enabled)    |AND -| Do: mlps_prndl_malfunction

|     |

TSTRAT >= 4 ----------------------|     |

(Electronic Automatic Transmission     |

present)                              |

| --- ELSE ---

|

| Exit Module

25-43

TRANSMISSION INPUT CONVERSIONS, MLPS CONVERSION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: mlps_eo_test

MLPS_KOEO_EN = 1 -----------------|

|

INDS < PARKLO_EO -----------|     |

|OR --|

INDS > PARKHI_EO -----------|     |AND -| store_code(P1705)

(INDS not indicating park)       |     |

|     | (MLPS not indicating required

INDS < NEULO_EO ------------|     |     |  position during KOEO)

|OR --|     |

INDS > NEUHI_EO ------------|           |

(INDS not indicating neutral)          |

| --- ELSE ---

TSTRAT < 4 -----------------------|     |

(manual or non_electronic         |     |

automatic trans,)                |AND -| store_code(P1709)

|     |

NDSFLG = 1 -----------------------|     | (Park/Neutral Position switch

|  not indicating Neutral during

|  KOEO)

|

| --- ELSE ---

|

| clear_malf(P1705)

| clear_malf(P1709)

| (MLPS/PNP reading correctly

|  during KOEO)

END: mlps_koeo_test

BEGIN: mlps_range_check

To verify the INDS signal is in an allowable range and convert the counts

into a PRNDL position.

always ---------------------------------| PARK_ERR = 0

| PDL_HGH = 0

| PDL_GND = 0

25-44

TRANSMISSION INPUT CONVERSIONS, MLPS CONVERSION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PARKLO <= INDS < PARKHI ----------|

|AND -| IPDL = 0

VS > VSPMIN ----------------------|     | PARK_ERR = 1

|

| --- ELSE ---

|

INDS >= PARKHI -------------------------| IPDL  = 0

| PDL_HGH = 1

|

| --- ELSE ---

|

INDS < MAN1LO --------------------------| IPDL  = 0

| PDL_GND = 1

|

| --- ELSE ---

|

PARKLO <= INDS < PARKHI ----------------| (PARK position)

| IPDL = 7

|

| --- ELSE ---

|

REVLO <= INDS < REVHI ------------------| (REVERSE position)

| IPDL = 6

|

| --- ELSE ---

|

NEULO <= INDS < NEUHI ------------------| (NEUTRAL position)

| IPDL = 5

|

| --- ELSE ---

|

ODLO <= INDS < ODHI --------------------| (OVERDRIVE position)

| IPDL = 4

|

| --- ELSE ---

|

MLPS_2LO <= INDS < MLPS_2HI ------------| (MLPS_2 postion)

| IPDL = MLPS_2

|

| --- ELSE ---

|

MAN1LO <= INDS < MAN1HI ----------------| (MANUAL 1 postion)

| IPDL = 1

|

| --- ELSE ---

|

| (in range fault detected)

| IPDL = 0

END: mlps_range_check

25-45

TRANSMISSION INPUT CONVERSIONS, MLPS CONVERSION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN:  mlps_prndl_verification

IPDL <> IPDL_LST -----------------|

|OR --| TM_PDL_RES = PDLTIM

IPDL = 0 -------------------|     |     | (load residence timer)

|AND -|

|INDS - INDS_LST| >         |

DLT_INDS --|

always ---------------------------------| INDS_LST = INDS

Set PDL based on IPDL, TCS (if present), and fault flags:

IPDL = 0 -------------------|

|AND -|

TM_PDL_RES = 0 -------------|     |

|

P0707MALF = 1 --------------------|OR --| Do: PDL FMEM Logic

(MLPS short circuit present)     |     |

|     |

P0708MALF = 1 --------------------|     |

(MLPS open circuit present)            |

| --- ELSE ---

|

TM_PDL_RES > 0 -------------------------| No Action Taken

|

| --- ELSE ---

IPDL = 4 -------------------------|     |

(overdrive position)             |     |

|     |

FLG_TCS = 1 ----------|           |AND -| PDL = 3

(overdrive cancel)   |AND -|     |     | (Overdrive cancel/Drive

|     |     |     |  position selected)

MLPS_2 = 2 -----------|     |OR --|     |

(TCS input present)        |           |

|           |

OFMFLG = 1 -----------------|           |

(EPC VFS short circuit                 |

fault present)                        |

| --- ELSE ---

|

| PDL = IPDL

| (let input pass through)

END: mlps_prndl_verification

25-46

TRANSMISSION INPUT CONVERSIONS, MLPS CONVERSION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Begin: PDL FMEM Logic

FMMMLP = 4 -----------------------|

(overdrive is PDL FMEM value)    |

|

FLG_TCS = 1 ----------|           |AND -| PDL = 3

(overdrive cancel)   |AND -|     |     |

|     |     |     |

MLPS_2 = 2 -----------|     |OR --|     |

(TCS input present)        |           |

|           |

OFMFLG = 1 -----------------|           |

(EPC VFS short circuit                 |

fault present)                        |

| --- ELSE ---

|

| PDL = FMMMLP

END: PDL FMEM logic

25-47

TRANSMISSION INPUT CONVERSIONS, MLPS CONVERSION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: mlps_prndl_malfunction

To perform PRNDL malfunction check:

IPDL = 0 -------------------------------| PDL_MON_TMR = 0

(mlps is suspect)                      |

| --- ELSE ---

|

| PDL_ERR_TMR = 0

| clear_malf(P0705)

| clear_malf(P1706)

|  (mlps is valid,

|   zero error timer

|   and malfunction flags

|   set based on the timer)

PDL_GND = 1 ----------------------------| P0707FIL = P0707FIL + PDL_UP

(MLPS out of range low)                | Do: mlps_short_check

|

| --- ELSE ---

|

PDL_HGH = 1 ----------------------------| P0708FIL = P0708FIL + PDL_UP

(MLPS out of range low)                | Do: mlps_open_check

|

| --- ELSE ---

PARK_ERR = 1 ---------------------|     |

(high vs in park error)          |AND -| store_code(P1706)

|     |

PDL_ERR_TMR > PARK_ERR_TM --------|     |

| --- ELSE ---

PARK_ERR = 0 ---------------------|     |

(in range fault)                 |AND -| malfunction(ccm,P0705)

|     |

PDL_ERR_TMR > PDL_ERR_TM ---------|     |

| --- ELSE ---

|

| P0707FIL = P0707FIL - 1

| P0708FIL = P0708FIL - 1

| Do: mlps_open_check

| Do: mlps_short_check

25-48

TRANSMISSION INPUT CONVERSIONS, MLPS CONVERSION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

P0705MALF = 1 --------------------|

|

P0707MALF = 1 --------------------|

|OR --| PDL_ERROR = 1

P0708MALF = 1 --------------------|     | (mlps fault present,

|     |  enter fmem)

P1706MALF = 1 --------------------|     |

| --- ELSE ---

|

| PDL_ERROR = 0

| (no mlps fault present)

PDL_ERROR = 1 --------------------|

(mlps fault present)             |OR --| PDL_MON = 1

|     | (mlps has been monitored

PDL_MON_TMR > PDL_MON_TM ---------|     |  this power-up)

END: mlps_prndl_malfunction

BEGIN: mlps_short_check

P0707FIL > PDL_LVL ---------------------| malfunction(ccm,P0707)

|

| --- ELSE ---

|

| clear_malf(P0707)

END: mlps_short_check

BEGIN: mlps_open_check

P0708FIL > PDL_LVL ---------------------| malfunction(ccm,P0708)

|

| --- ELSE ---

|

| clear_malf(P0708)

END: mlps_open_check

25-49

TRANSMISSION INPUT CONVERSIONS, TORQUE INTERPOLATION CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

25.12  TORQUE INTERPOLATION CALCULATION (CDAN0)

OVERVIEW

Shift schedules and converter lock schedules need to  be  adjusted  based  on

torque  losses.   This module calculates the percent torque loss by comparing

TQ_NET to the expected torque output at that speed and TP_REL point.

The expected torque output table, FN1619A(NEBART, TP_REL_BAR), is  calibrated

at the time the base shift schedule is developed.

To prevent rapid swings in TQ_NORM, several  conditions  are  placed  on  the

uupdate  of  the  parameter.  TQ_NET values must be steady, otherwise a delay

timer is started and the update occurs once the trasnient  settles.   Because

low  TQ_NET  values  may  be inaccurate or not repeatable, a minimum required

value is placed on TQ_NET for an update of  TQ_NORM  to  occur.   Because  of

rapid  changes  in  engine  speed  during  a  shiftand  the effects of Torque

Modulation during that time, TQ_NORM may not update unless there is no  shift

in  progress.   The  amount that TQ_NORM is allowed to change from background

loop to background loop is also limited.   Finally,  a  low  pass  filter  is

placed on TQ_NORM to smooth out the signal and prevent step changes

DEFINITIONS

Registers:

-  BTR_BASE = Output of FN1619A,  expected  torque  output  at  the  current

engine speed and TP_REL point, ft-lbs.

-  FLG_SFT_IN = 1 -> shift is in progress.

-  NEBART = Filtered engine RPM for transmission.

-  TP_REL_BAR = Filtered value of TP_REL (TP - RATCH).

-  TQ_INTRP = Torque interpolation factor.

-  TQ_NET = Net torque into torque converter, ft-lbs.

-  TQ_NET_LST = Last background pass value of TQ_NET, ft-lbs.

-  TQ_NORM = Ratio of actual to expected torque output.

-  TQ_NORM_CUR = Current  unfiltered  value  of  the  ratio  of  TQ_NET  and

BTR_BASE.

-  TQ_NORM_LST = Last background pass value of TQ_NORM_NEW.

-  TQ_NORM_NEW = Unfiltered value of TQ_NORM.

-  UPDATE_TMR = Timer which delays  the  update  of  TQ_NORM  due  to  large

changes in TQ_NET, seconds.

25-50

TRANSMISSION INPUT CONVERSIONS, TORQUE INTERPOLATION CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  FN1619A = Expected torque output at the current engine speed  and  TP_REL

point, normalized.

x = FN070P(NEBART) = Normalized engine speed.

y = FN067D(TP_REL_BAR) = Normalized TP_REL_BAR.

-  FN615TQ = Normalized torque vs.  interpolation factor.

-  FN067D = Normalizing function for TP_REL_BAR.

-  FN070P = Normalizing function for NEBART

-  TCTQ_NORM = Time constant for TQ_NORM, seconds.

-  TQ_MAX_DELT = Maximum allowed  change  in  TQ_NET  to  allow  TQ_NORM  to

update, ft-lbs.

-  TQ_MAX_TM = Time to wait before TQ_NORM is allowed to  update  due  to  a

large change in TQ_NET, seconds.

-  TQ_NET_MIN = Minimum value of TQ_NET to allow TQ_NORM to update.

-  TQ_NORM_UP = Maximum allowed change in TQ_NORM per background pass.

PROCESS

STRATEGY MODULE: INTRN_TQ_INTR_COM1

always -----------------------------| BTR_BASE = FN1619A(NEBART, TP_REL_BAR)

| ;"base" torque, ft-lbs

BTR_BASE <> 0 ----------------------| TQ_NORM_CUR = TQ_NET / BTR_BASE

| ;ratio of actual torque

| ;and base torque

|

| --- ELSE ---

|

| TQ_NORM_CUR = 1

25-51

TRANSMISSION INPUT CONVERSIONS, TORQUE INTERPOLATION CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

abs(TQ_NET - TQ_NET_LST) >

TQ_MAX_DELT ---------| UPDATE_TMR = TQ_MAX_TM

| ;wait for transient to settle

| TQ_NORM_NEW = TQ_NORM_LST

| ;do not update TQ_NORM_NEW

|

| --- ELSE ---

FLG_SFT_IN = 1 ---------------|     |

;shift in progress            |     |

|     |

TQ_NET < TQ_NET_MIN ----------|OR --| TQ_NORM_NEW = TQ_NORM_LST

;TQ_NET is too low            |     | ;do not update TQ_NORM_NEW

|     |

UPDATE_TMR > 0 ---------------|     |

;waiting for a transient to         |

;settle                             |

| --- ELSE ---

abs(TQ_NORM_CUR - TQ_NORM_LST)      |

<  TQ_NORM_UP ----| TQ_NORM_NEW = TQ_NORM_CUR

;maximum amount TQ_NORM_NEW can     |

;change is set by TQ_NORM_UP        |

| --- ELSE ---

|

TQ_NORM_CUR > TQ_NORM_LST ----------| TQ_NORM_NEW = TQ_NORM_LST

|     + TQ_NORM_UP

|

| --- ELSE ---

|

| TQ_NORM_NEW = TQ_NORM_LST

|     - TQ_NORM_UP

always -----------------------------| TQ_NORM =

|   ROLAV(TQ_NORM_NEW,TCTQ_NORM)

|

| TQ_INTRP = FN615TQ(TQ_NORM)

| ;torque interpolation factor

|

| TQ_NORM_LST = TQ_NORM_NEW

| ;save last unfiltered value of

| ;the ratio of TQ_NORM

25-52

TRANSMISSION INPUT CONVERSIONS, AODE OVERVIEW - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

25.13  AODE SHIFT CONTROL OVERVIEW (CDAN0)

The  AODE  strategy  and  software  are  comprised  of  a  set  of  distinct,

independent modules, each with a specific function.  The modules are designed

to minimize the software impact of different transmission  hardware  and  can

thus  be  re-used  in other future transmission strategies.  The main modules

and subroutines are show on the following page.  They  are  executed  in  the

order  shown except for INTRN_AODE_INPUT_EQNS_COMx which are done immediately

after input conversion.  Also shown are the main  output  parameters  of  the

modules.

All modules listed are executed if TSTRAT = 7  (AODE  transmission  present).

For  TSTRAT  not equal to 7, ONLY those modules marked by an asterisk (*) are

executed.

*******************************************************************

*******************************************************************

***                                                             ***

***                                                             ***

***     All references to "PDL", "PRNDL", or variations thereof ***

*** with respect to electronic transmission controls are        ***

*** synonymous with "Manual Level Indicated Position" as deter- ***

*** mined from the manual lever position sensor.                ***

***                                                             ***

***     All references to "TV", "ETV", or terms containing "TV" ***

*** with respect to electronic transmission controls are synon- ***

*** ymous with "Electronic Pressure Control", and are not       ***

*** associated with any control function of the engine throttle.***

***                                                             ***

***                                                             ***

*******************************************************************

*******************************************************************

25-53

TRANSMISSION INPUT CONVERSIONS, AODE OVERVIEW - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

STRATEGY MODULE: INTRN_AODE_OVERVIEW_COM2

STRUCTURE

If TSTRAT = 7, execute the following modules.  Otherwise, execute only those

modules that are marked with an asterisk.

MAIN ROUTINE

SUBROUTINES                                      OUTPUT PARAMETERS

---------------------------------------------------------------------------

DESRD_GR_DETR_COMx                                        GR_DS

SC_GR_DS_AUTO_COMx                               FLG_FRST_DS

SC_VER_AUTO_SHFT_COMx                            FLG_SFT_UP

SC_GR_SEQ_PNTR_COMx                              GR_DS_TV

FLG_SFT_DN

SC_CM_GR_DETR_COMx                                        GR_CM

SC_CM_GR_MAN1_COMx                               GR_OLD

SC_CM_GR_MAN2_COMx                               RT_GR_OLD

SC_CM_GR_AUTO_UP_COMx                            FLG_FRST_CM

SC_CM_GR_AUTO_DWN_COMx

SC_TIMER_COMx                                             TM_SFT_IN

FLG_SFT_IN

FLG_SFT_MDN

SC_SOL_CTL_COMx                                           FLG_SS_1

SC_SS1_OBDII_COMx                                FLG_SS_2

SC_SS2_OBDII_COMx                                RT_GR_CUR

GEAR_CUR

SC_VALID_COMx                                             SFT_FM_FLG

FLG_SFT_VER

FLG_SFT_VAL

SC_TQMOD_COMx                                             NICT_TRIG_ENT

NICT_TRIG_ST

*SC_RLS_COM1                                               FLG_RLS

EPC_GUIDE_COMx                                            TV_PRES

EPC_ASILT_COMx                                   FLG_ASILT

EPC_STARTUP_COMx                                 TV_STAT

EPC_ENGMT_STALL_COMx                             TV_COUNTS

EPC_NORM_COMx                                    TV_DYN

EPC_TQ_IALPHA_COMx

EPC_DYNAMIC_COMx

EPC_VFS_OBDII_COMx                                        DUCE_VFS_1

EPC_VFS_DUCE_REPEATER_COMx                                VFS_OUT_6

VFS_OUT_FLG

EPC_OTEMP_TEST_OBDII_COMx                                  OTEMP_FM_FLG

25-54

TRANSMISSION INPUT CONVERSIONS, AODE OVERVIEW - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

MAIN ROUTINE

SUBROUTINES                                      OUTPUT PARAMETERS

---------------------------------------------------------------------------

CCC_BYP_CLTCH_CTL_COMx                                    FLG_OT_LK

CCC_HOT_LCK_COMx                                 FLG_HDR_LK

CCC_FMEM_COMx                                    FLG_FMM_LK

CCC_SCHLD_LCK_UNLCK_COMx                         TM_CRV_UNLK

CCC_SCHLD_BYP_SLIP_COMx                          FLG_CRV_LK

CCC_UNCOND_UNLK_COMx                             FLG_STRK_UNLK

CCC_MOD_COMx                                     FLG_UNC_UNLK

CCC_BCSDC_CALC_COMx                              FLG_ENG_LK

CCC_BYP_PID_CTL_COMx                         BCSDC

CCC_BYP_SLIP_CALC_COMx

CCC_MLUS_OBDII_COMx                                       BCSDC_OUT

LUS_PW_DUCE

INTRN_TCIL_CONTROL_COMx

INTRN_TCS_COMx                                   FLG_TCS

INTRN_TCIL_STATE_COMx                            TCIL_STATE

INTRN_TCIL_REPEAT_COMx

*INTRN_TQ_INTR_COMx                                        TQ_INTR

*INTRN_TRANS_TQ_NET_CALC_COMx                              TQ_NET

INTRN_CALC_VS_RATE_COMx                                   VS_RATE

INTRN_RT_NOVS_KAM_CALC_COMx                               RT_NOVS

INTRN_AODE_INPUT_EQNS_COMx                                NEBART

TPBART

TPVARTV

TPBARTC

TP_RATE

NTBART

BP_INTR

SPD_RATIO

SLIP_ABS

INTRN_CSM_COMx                                            CS_SFT_MULT

*INTRN_MLPS_CONV_COMx                                      PDL

*INTRN_AC_EN_TRANS_COMx                                    AC_EN_TRANS

25-55

TRANSMISSION INPUT CONVERSIONS, 4 x 4 LOW SWITCH - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

25.14  THE 4 X 4 LOW SWITCH INPUT PROCESSING AND SELF TEST (CDAJ0)

OVERVIEW

This module contains 4x4L input processing and self test.  The 4 x  4  switch

indicates  that  the  driver  has attempted to shift the transfer case to low

range.  In the "shift on the fly" package, the 4 x 4 light will flash if  the

transfer  case has not been allowed to shift into 4 x 4 low.  If the transfer

case has been allowed to shift into 4  x  4  low  or  in  the  non-electronic

system,  the  light will remain steadily on if in 4 x 4 low mode.  If in this

mode, the shift schedule will be adjusted by the transfer case ratio  to  get

shifts at the correct output shift speed.  This is done by modifying RT_NOVS.

Since 12 volts at the module pin means normal mode and 0 volts means  4  x  4

mode, the input to the CPU is read as an inverted input.

During Engine Off on demand self test, the 4x4L switch must indicate  "normal

mode", or code 1781 will be set.

DEFINITIONS

INPUTS

Registers:

-  I4X4L = 4x4 input.

-  I4X4L_LST = 4x4 input, last pass.

-  TM_4X4L_RES = 4x4 low switch input residence timer.

Bit Flags:

-  CCM_EO_ENA = Engine Off CCM test enable flag; 1 -> Engine Off CCM enabled

by OBDII sequencer.

Calibration Contants:

-  HP_4X4L = 4X4L hardware present switch, 1 = 4X4L input present.

-  TM4X4L = 4x4 low switch residence time.

OUTPUTS

Registers:

-  I4X4L_LST = See above.

-  TM_4X4L_RES = See above.

Bit Flags:

-  FLG_4X4L = 4x4 low flag.

25-56

TRANSMISSION INPUT CONVERSIONS, 4 x 4 LOW SWITCH - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OTHER

-  4x4_codes = set of 4x4L switch error codes {P1781}

-  P1781 = 4x4L switch indicating 4x4L during engine off self test.

-  store_code(Pxxx) = Stores the fault code specified by Pxxx.

PROCESS

STRATEGY MODULE: INTRN_4X4L_OBDII_COM1

HP_4X4L = 0 -----------------------| EXIT

(No 4x4L hardware)                |

I4X4L_LST <> I4X4L ----------------| I4X4L_LST = I4X4L

|  (update state change register)

| TM_4X4L_RES = TM4X4L

|  (load residence timer)

TM_4X4L_RES = 0 -------------------| FLG_4X4L = I4X4L

|  (pass along current state,

|   FLG_4X4L = 1 is 4 x 4 mode)

When On-Demand self test is run, the operator is told to place the vehicle

in "normal mode". If the 4x4L switch indicates 4x4L mode during engine off

self test, code 1781 will be set.

CCM_EO_ENA = 1 --------------|

(Engine Off CCM enabled)    |

|AND -| P1781MALF = 1

|     | store_code(P1781)

I4X4L = 1 -------------------|     | (4x4L switch closed

(Switch indicates 4X4L mode)      |  during Engine Off

|  self test)

|

| --- ELSE ---

|

| P1781MALF = 0

25-57

TRANSMISSION INPUT CONVERSIONS, PWM FUNCTIONALITY TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

25.15  PWM FUNCTIONALITY TEST (CDAK0)

OVERVIEW

This module determines if the PWM solenoid is mechanically "on"  when  it  is

commanded "off" (commanded EEC MLUS duty cycle is zero).

The PWM mechanically "on" when commanded "off" determination is  accomplished

by  comparing  1st  gear  actual converter clutch slip (SLIP_ACT) with a open

converter slip value (SLIP_OPEN1) calculated using a torque converter  clutch

model.   If  actual  slip  is  considerably less the the open converter model

based value, then the converter clutch is being applied.

By monitoring the apply state of the converter clutch in 1st gear,  the  "on"

state  of  the  bypass  clutch  control  valve  can  be determined, since the

application of the converter clutch is hydraulically locked out in  1st.   If

the  bypass  valve  is  "on"  in  first gear, the mechanical state of the PWM

solenoid cannot be determined because the converter will  always  be  applied

regardless of the PWM state.

If the bypass valve is monitored in 1st gear, and is  determined  to  not  be

stuck "on", then if the converter is applied in 2nd, 3rd, or 4th when the EEC

is not trying to apply the clutch, the PWM is said to be mechanically "on".

DEFINITIONS

INPUTS

Registers:

-  BCSDC = Modulated bypass clutch duty cycle.

-  BCV_OFF_CNT = Counter tracking number of samples bypass valve  NOT  stuck

"on" in first gear.

-  BCV_ON_CNT = Counter tracking number of samples bypass valve  stuck  "on"

in first gear.

-  CC_OFF_CNT = Counter tracking number of samples converter is not applied.

-  CC_ON_CNT = Counter to track number of samples converter is applied.

-  GR_CM = Commanded gear for shift solenoids.

-  INIT_TOT = TOT at  start-up;  arithmetic  average  of  the  first  8  TOT

readings.

-  NOBART = Filtered transmission output shaft speed, rpm.

-  NTBART = Filtered transmission turbine speed, rpm.

-  PWM_OFF_CNT  =  Counter  tracking  number  of  samples  PWM  solenoid  is

mechanically "off" when commanded "off".

25-58

TRANSMISSION INPUT CONVERSIONS, PWM FUNCTIONALITY TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PWM_ON_CNT  =  Coutner  tracking  number  of  samples  PWM  solenoid   is

mechanically "on" when commanded "off".

-  SLIP_ACT = Actual torque converter slip signed, rpm.

-  SLIP_DIF = Difference between calculated open converter slip and  actual,

rpm.

-  SLIP_OPEN1 = Calculated open converter slip, rpm.

-  SPD_RATIO = Speed ratio across the torque converter.

-  TOT = Transmission oil temperature, deg.  F.

-  TP = Throttle position sensor.

-  TPBART = Filtered throttle position.

-  TP_REL = Relative throttle position, counts.

-  TQ_BAR = Filtered TQ_NET.

-  TQ_BAR_LST = Last pass value of TQ_BAR.

-  TQ_BAR_PWM = Filtered TQ_NET for PWM functionality test, ft-lbs.

-  TQ_NET = Net torque into the torque converter, ft-lbs.

-  TST_DLY_TMR = PWM functionality test transient delay timer, secs.

Bit Flags:

-  FLG_BCV_MON = Flag indicating bypass clutch control valve state in  first

gear monitiored; 1 -> state monitored.

-  FLG_BCV_ON = Flag indicating "on" state  of  the  bypass  clutch  control

valve; 1 -> bypass valve stuck "on".

-  FLG_PWM_MON = Flag indicating PWM solenoid state monitored;  1  ->  state

monitored.

-  FLG_PWM_ON = PWM solenoid "on" state flag; 1 -> PWM solenoid mechanically

"on" when commanded "off".

-  FLG_SFT_IN = Shift in progress flag; 1 -> shift is in progress.

-  FLG_SS_MON = Flag indicating all shifts have been  monitored;  1  ->  all

shifts monitored.

-  FLG_TST_CMP = Flag indicating if a test has been compeleted this entry; 1

-> test completed.

-  FLG_TST_FP = PWM functionality test first pass flag; 1 -> not first pass.

Calibration Constants:

-  BCVOFFDN = Down count for bypass valve "off" in 1st gear counter.

25-59

TRANSMISSION INPUT CONVERSIONS, PWM FUNCTIONALITY TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  BCVOFFUP = Up count for bypass valve "off" in 1st gear counter.

-  BCVONDN = Down count for bypass valve "on" in 1st gear counter.

-  BCVONUP = Up count for bypass valve "on" in 1st gear counter.

-  BCV_OFF_LVL = Threshold to determine if bypass valve NOT  stuck  "on"  in

first gear.

-  BCV_ON_LVL = Threshold to determine if bypass valve stuck "on" in first.

-  CCOFFDN = Down count for converter "not-applied" counter.

-  CCOFFUP = Up count for converter "not-applied" counter.

-  CCONDN = Down count for converter "applied" counter.

-  CCONUP = Up count for converter "applied" counter.

-  CC_OFF_LVL = Threshold to determine if converter not-applied.

-  CC_ON_LVL = Threshold to determine if converter applied.

-  CC_NO_DL = Output shaft speed threshold below which  PWM  test  disabled,

rpm.

-  CC_SR_DL = Speed Ratio threshold below which PWM test disabled.

-  CC_SR_DH = Speed Ratio threshold above which PWM test disabled.

-  CC_TOT_DL = TOT threshold below which PWM test disabled, deg F.

-  CC_TOT_DH = TOT threshold above which PWM test disabled, deg F.

-  CC_TP_DL = Throttle threshold below which PWM test disabled, counts.

-  CC_TP_DH = Throttle threshold above which PWM test disabled, counts.

-  CC_TP_SS = Maximum allow able change in TP_REQ to allow PWM test, counts.

-  CC_TQ_DH = Input torque threshold above which PWM test disabled, ft-lbs.

-  CC_TQ_DL = Input torque threshold below which PWM test disabled, ft-lbs.

-  CC_TQ_SS = Maximum allowable change in input torque to  allow  PWM  test,

ft-lbs.

-  CC_TQ_SS1 = Maximum allowable change in input torque to allow  PWM  test,

ft-lbs.

-  FN690B(INIT_TOT) = TOT required to allow enable PWM  functionality  test,

deg F.

-  K_CONV1 = Torque converter coefficient that relates input torque to slip,

multiplied by turbine speed; ft-lbs/rpm*rpm.

-  PWMOFFDN = Down count for PWM "mechanically off" counter.

25-60

TRANSMISSION INPUT CONVERSIONS, PWM FUNCTIONALITY TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PWMOFFUP = Up count for PWM "mechanically off" counter.

-  PWMONDN = Down count for PWM "mechanically on" counter.

-  PWMONUP = Up count for PWM "mechanically on" counter.

-  PWM_OFF_LVL = Threshold to determine if PWM solenoid  mechanically  "off"

when commanded "off".

-  PWM_ON_LVL = Threshold to determine if  PWM  solenoid  mechanically  "on"

when commanded "off".

-  SLIP_ACT_LK = Threshold of slip below which converter is locked, rpm.

-  SLP_CC_ON = Slip difference (SLIP_OPEN1 - SLIP_ACT) threshold above which

converter is said to be applied.

-  SLP_OPN_MIN = Threshold of  SLIP_OPEN1  above  which  the  state  of  the

converter can be determined, rpm.

-  SW_PWM_TST = Test switch to disable BCSDC criteria to allow  test;  1  ->

disable BCSDC criteria for test entry.

-  SW_1742_MIL = Switch to allow setting a MIL code for a P1742 fault; 1  ->

set MIL code.

-  SW_1743_NMIL = Switch to allow setting a NON-MIL code for a P1743  fault;

1 -> set the NON-MIL code.

-  TCTQPWM = Time constant for TQ_BAR_PWM.

-  TSTDLYTM = PWM functionality test delay time, secs.

OUTPUTS

Registers:

-  BCV_OFF_CNT = See above.

-  BCV_ON_CNT = See above.

-  CC_OFF_CNT = See above.

-  CC_ON_CNT = See above.

-  PWM_OFF_CNT = See above.

-  PWM_ON_CNT = See above.

-  SLIP_ACT = See above.

-  SLIP_DIF = See above.

-  SLIP_OPEN1 = See above.

25-61

TRANSMISSION INPUT CONVERSIONS, PWM FUNCTIONALITY TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TQ_BAR_LST = See above.

-  TQ_BAR_PWM = See above.

-  TST_DLY_TMR = See above.

Bit Flags:

-  FLG_BCV_MON = See above.

-  FLG_BCV_ON = See above.

-  FLG_PWM_MON = See above.

-  FLG_PWM_ON = See above.

-  FLG_TST_CMP = See above.

-  FLG_TST_FP = See above.

-  P1742 = See above.

-  P1743 = See above.

-  P1742MALF = Malfunction flag for P1742 code; 1 -> a malfunction currently

exists for fault P1742.

-  P1743MALF = Malfunction flag for P1743 code; 1 -> a malfunction currently

exists for fault P1743.

OTHER

-  malfunction(ccm,P1742) = Logic process, imported  from  the  MIL  control

module; P1742 indicates a fault code.

-  P1742 = Fault code, PWM mechanically "on" failure (MIL code).

-  P1743 = Fault code, PWM mechanically "on" failure (NON MIL code).

-  store_code(Pxxxx) = Logic process, used when NON_MIL faults are detected.

25-62

TRANSMISSION INPUT CONVERSIONS, PWM FUNCTIONALITY TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INTRN_PWM_FUNCT_TEST_COM3

TOT < FN690B(INIT_TOT) -----------|

|

TOT < CC_TOT_DL ------------------|

|

TOT > CC_TOT_DH ------------------|

|

TP_REL < CC_TP_DL ----------------|

|

TP_REL > CC_TP_DH ----------------|

|

abs(TP - TPBART) > CC_TP_SS ------|

|

abs(TQ_BAR - TQ_BAR_LST) >        |

CC_TQ_SS -----|

|

abs(TQ_NET - TQ_BAR_PWM) >        |OR --| FLG_TST_FP = 0

CC_TQ_SS1 ----|     | FLG_TST_CMP = 0

|     | TQ_BAR_LST = TQ_BAR

TQ_BAR_PWM < CC_TQ_DL ------------|     | TQ_BAR_PWM =

|     |      ROLAV(TQ_NET,TCTQPWM)

TQ_BAR_PWM > CC_TQ_DH ------------|     | EXIT THIS MODULE

|

NOBART < CC_NO_DL ----------------|

|

SPD_RATIO < CC_SR_DL -------------|

|

SPD_RATIO > CC_SR_DH -------------|

|

FLG_SFT_IN = 1 -------------------|

|

BCSDC > 0 ------------------|     |

(converter commanded "on") |AND -|

|

SW_PWM_TST = 0 -------------|

(enable BCSDC criteria for

test validation)

NTBART = 0 -----------------------------| SLIP_OPEN1 = 0

|

| --- ELSE ---

|

| SLIP_OPEN1 = TQ_BAR / (K_CONV1

* NTBART)

25-63

TRANSMISSION INPUT CONVERSIONS, PWM FUNCTIONALITY TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

always ---------------------------------| SLIP_DIF = SLIP_OPEN1 - SLIP_ACT

| TQ_BAR_LST = TQ_BAR

| TQ_BAR_PWM = ROLAV(TQ_NET,TCTQPWM)

FLG_TST_FP = 0 -------------------------| TST_DLY_TMR = TSTDLYTM

(first pass; initialize appropriate    | CC_ON_CNT = 0

registers)                            | CC_OFF_CNT = 0

| FLG_TST_FP = 1

TST_DLY_TMR > 0 ------------------------| EXIT THIS MODULE

(transient delay timer not expired)

GR_CM = 1 ------------------|

|AND -|

SLIP_DIF >= SLP_CC_ON ------|     |

|

GR_CM > 1 ------------------|     |OR --| CC_ON_CNT = CC_ON_CNT + CCONUP

|     |     | CC_OFF_CNT = CC_OFF_CNT - CCOFFDN

SLIP_OPEN1 >= SLP_OPN_MIN --|     |     |

(normal open slip value    |AND -|     |

large enough insure       |           |

accurate test)            |           |

|           |

SLIP_ACT <= SLIP_ACT_LK ----|           |

(converter physically                  |

locked)                               |

| --- ELSE ---

|

| CC_ON_CNT = CC_ON_CNT - CCONDN

| CC_OFF_CNT = CC_OFF_CNT + CCOFFUP

25-64

TRANSMISSION INPUT CONVERSIONS, PWM FUNCTIONALITY TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLG_TST_CMP = 0 ------------------|

(test has not completed          |

this entry)                     |

|AND -| FLG_TST_CMP = 1

GR_CM = 1 ------------------------|     | BCV_ON_CNT = BCV_ON_CNT + BCVONUP

|     | BCV_OFF_CNT = BCV_OFF_CNT -

CC_ON_CNT >= CC_ON_LVL -----------|     |                BCVOFFDN

(converter physically applied          |

when it is commanded open)            |

| --- ELSE ---

FLG_TST_CMP = 0 ------------------|     |

|     |

GR_CM = 1 ------------------------|AND -| FLG_TST_CMP = 1

|     | BCV_ON_CNT = BCV_ON_CNT - BCVONDN

CC_OFF_CNT >= CC_OFF_LVL ---------|     | BCV_OFF_CNT = BCV_OFF_CNT +

(converter phsically open              |                BCVOFFUP

when it is commanded open)            |

| --- ELSE ---

FLG_TST_CMP = 0 ------------------|     |

|AND -| FLG_TST_CMP = 1

CC_OFF_CNT >= CC_OFF_LVL ---------|     | BCV_ON_CNT = BCV_ON_CNT - BCVONDN

| BCV_OFF_CNT = BCV_OFF_CNT +

|                BCVOFFUP

| PWM_ON_CNT = PWM_ON_CNT - PWMONDN

| PWM_OFF_CNT = PWM_OFF_CNT +

|                PWMOFFUP

|

| --- ELSE ---

FLG_TST_CMP = 0 ------------------|     |

|     |

GR_CM > 1 ------------------------|     |

|     |

FLG_BCV_ON = 0 -------------------|     |

(bypass clutch valve not "on")   |     |

|AND -| FLG_TST_CMP = 1

FLG_BCV_MON = 1 ------------------|     | PWM_ON_CNT = PWM_ON_CNT + PWMONUP

(bypass clutch valve state       |     | PWM_OFF_CNT = PWM_OFF_CNT -

monitored)                      |     |                PWMOFFDN

|     |

CC_ON_CNT >= CC_ON_LVL -----------|     |

| --- ELSE ---

|

| no action

25-65

TRANSMISSION INPUT CONVERSIONS, PWM FUNCTIONALITY TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BCV_ON_CNT >= BCV_ON_LVL ---------------| FLG_BCV_ON = 1

(bypass valve verified "on")          | FLG_BCV_MON = 1

| FLG_PWM_ON = 0

|  (bypass valve "on"; assume

|   PWM "off")

| FLG_PWM_MON = 1

|

| --- ELSE ---

|

BCV_OFF_CNT >= BCV_OFF_LVL -------------| FLG_BCV_ON = 0

(bypass valve verified "off")         | FLG_BCV_MON = 1

|

| --- ELSE ---

|

| no action

PWM_OFF_CNT >= PWM_OFF_LVL -------------| FLG_PWM_ON = 0

(pwm verified "off")                  | FLG_PWM_MON = 1

|

| --- ELSE ---

|

PWM_ON_CNT >= PWM_ON_LVL ---------------| FLG_PWM_ON = 1

(PWM verified "on")                   | FLG_PWM_MON = 1

|

| --- ELSE ---

|

| no action

25-66

TRANSMISSION INPUT CONVERSIONS, PWM FUNCTIONALITY TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SW_1742_MIL = 1 ------------------|

(set MIL code for this fault)   |

|

FLG_PWM_ON = 1 -------------------|AND -| malfunction (CCM,P1742)

(PWM is "ON")                    |     | P1742MALF = 1

|     |

FLG_SS_MON = 1 -------------------|     |

(all shift solenoids have              |

been verified as functioning)         |

| --- ELSE ---

FLG_SS_MON = 0 -------------------|     |

|OR --| P1742MALF = 0

FLG_PWM_ON = 0 -------------|     |     |

|AND -|     |

FLG_PWM_MON = 1 ------------|           |

(PWM state montitored)                 |

| --- ELSE ---

|

| no action

SW_1743_NMIL = 1 -----------------|

(set NON-MIL code for this fault)|

|

FLG_PWM_ON = 1 -------------------|AND -| store_code (P1743)

(PWM is "ON")                    |     | P1743MALF = 1

|     |  (NOTE: This is a NON-MIL code

FLG_SS_MON = 1 -------------------|     |   used to flash the TCIL)

(all shift solenoids have              |

been verified as functioning)         |

| --- ELSE ---

FLG_SS_MON = 0 -------------------|     |

|OR --| P1743MALF = 0

FLG_PWM_ON = 0 -------------|     |     |

|AND -|     |

FLG_PWM_MON = 1 ------------|           |

(PWM state montitored)                 |

| --- ELSE ---

|

| no action

25-67

TRANSMISSION INPUT CONVERSIONS, A/C ENABLE CONTROL  - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

25.16  A/C ENABLE CONTROL (CDAK0)

OVERVIEW

The modulated bypass A/C  clutch  compensation  determines  the  closed  loop

control  region that will use ACSLP for slip target.  This is done to provide

sufficient slip in the converter to absorb torque disturbances which could be

transmitted  through  the  driveline  when  the A/C fan and/or compressor are

activated.

This module also disables the A/C during shifts to  prevent  distrubances  in

the engine speed trace that may cause false torque modulation triggers.

The output of this module is a flag (AC_EN_TRANS) which indicates that either

the  converter is controlling to the A/C compensation slip target, a shift is

in progress, or that a default timer threshold has been exceeded.   When  any

of the conditions are met, normal A/C operation will be allowed.  The feature

which uses the modulated bypass clutch to decouple the A/C hits from the rest

of  the powertrains can be calibrated out by setting AC_SLP_WDOG = 0.  Normal

slip control will be returned when the A/C timer (ACITMR) exceeds AC_CTL_OFF.

Disabling  the  A/C  during  shifts  cannot  be   calibrated   out;   it   is

unconditional.

Additional logic is provided to protect against a low A/C  charge  situation.

When  the  A/C  charge  level  is  low, the A/C compressor will tend to cycle

rapidly.  The A/C cycle period is  monitored.   When  the  cycling  frequency

becomes too high, the Anticipatory A/C compensation logic is de-activated.  A

special option is provided in regards to disabling the Anticipatory logic.  A

fault  filter type structure is used to disable (fault filter update logic is

located in the A/C strategy section) However, if the rapid cycling  situation

ceases,  the  Anticipatory  logic  could  be  re-enabled  during the same EEC

power-up (fault  filter  counts  down).   Therefore,  a  switch  is  provided

(SW_DAC_NPU)  which can be used to keep the Anticapatory logic disabled until

the next EEC power-up.

If the calibration indicates a manual transmission (TSTRAT <= 3), the A/C  is

allowed to function normally.

25-68

TRANSMISSION INPUT CONVERSIONS, A/C ENABLE CONTROL  - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  ACITMR = Closed throttle A/C clutch enable delay  timer,  sec.   Used  to

delay  A/C  clutch  engagement at closed throttle to allow the Idle speed

Control strategy to anticipate the compressor load application.

-  AC_CTL_TMR = Slip in control timer after A/C requested.

-  AC_SLP_TMR = Default timer to activate A/C  engagement  process  if  slip

control not established.

-  ERR_T0 = Current slip error (SLIP_DES_S - SLIP_ACT).

-  GR_DS = Desired Gear

-  GR_CM = Current Commanded Gear For Shift Solenoids.

-  P1469MALF = Malfunction flag for A/C cycling period to short.

-  SLIP_DES_S = Desired value of slip measured in rpm, signed.

-  SLIP_TRG_S = Target slip value measured in rpm, signed.

Bit Flags:

-  ACRQST = A/C requested flag; 1 -> request.

-  AC_EN_TRANS = Flag used to delay  A/C  engagement  process;  0  ->  delay

process.

-  FLG_AC_CMP = Flag indicating A/C compensation cycle complete,  return  to

normal operating conditions; 1 -> process complete.

-  FLG_AC_CTL = Flag indicating when to use  A/C  compensation  slip  target

(ACSLP); 1 -> use ACSLP.

-  FLG_SFT_IN = Shift in progress flag; 1 -> shift in progress.

-  FLG_SLP_CTL = Slip in control after A/C requested; 1 -> slip in control.

-  FLG_CRV_LK = Scheduled curve lockup flag; 1 -> Scheduled lockup.

-  FLG_VE_DSGR = Verify Desired Gear First Pass Flag.

Calibration Constants:

-  AC_CTL_OFF = Timer threshold to de-activate A/C compensation.

-  AC_CTL_TM = Timer threshold indicating converter slip in control.

-  AC_SLP_WDOG = Timer threshold to allow A/C engagement if slip control not

established

-  ERR_T0_AC = Error threshold establishing control region.

25-69

TRANSMISSION INPUT CONVERSIONS, A/C ENABLE CONTROL  - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  SLIP_DIF_AC = Threshold to determine when  slip  should  start  being  in

control.

-  SW_DAC_NPU = Switch to disable Anticipatory A/C logic till next power-up;

1 -> disable till next power up.

-  TSTRAT  =  Transmission  Strategy  Switch;  The  TSTRAT  software  switch

controls which transmission control strategy is executed:

0 -> No transmission control (Manual transmission, AOD, ATX, C6,

C3, etc.)

1 -> SIL (Shift indicator light)

2 -> A4LD with 3-4 shift control and converter clutch control

3 -> AXOD

4 -> C64E (E4OD)

5 -> A41DE

6 -> FAX-4

7 -> AOD-E (AOD-I)

8 -> 4EAT

9 -> CD4E

25-70

TRANSMISSION INPUT CONVERSIONS, A/C ENABLE CONTROL  - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INTRN_AC_EN_TRANS_COM1

TSTRAT <= 3 ----------------------------| AC_EN_TRANS = 1

(manual transmission)                  | (allow A/C to operate

|  normally)

|

| --- ELSE ---

FLG_VE_DSGR = 1 ------------------|     |

(verifying a shift)              |     |

|     |

GR_DS <> GR_CM -------------------|OR --| AC_EN_TRANS = 0

(shift is desired; or sequenced  |     | (delay engagment of

shift in progress)              |     |  A/C clutch)

|     | FLG_AC_CTL = 0

FLG_SFT_IN = 1 -------------------|     | FLG_SLP_CTL = 0

(shift is in progress)                 | AC_CTL_TMR = 0

| AC_SLP_TMR = 0

| FLG_AC_CMP = 0

| EXIT THIS MODULE

|

| --- ELSE ---

|

P1469MALF = 1 --------------------------| AC_EN_TRANS = 1

(disable anticipatory A/C logic,       | FLG_AC_CMP = 0

A/C cycle period too short)           |

|

| --- ELSE ---

|

ACRQST = 0 -----------------------------| AC_EN_TRANS = 0

| FLG_AC_CMP = 0

|

| --- ELSE ---

|

| no action

P1469MALF = 1 --------------------|

|

ACRQST = 0 -----------------------|OR --| FLG_AC_CTL = 0

|     | FLG_SLP_CTL = 0

FLG_AC_CMP = 1 -------------------|     | AC_CTL_TMR = 0

| AC_SLP_TMR = 0

| EXIT THIS MODULE

|

| --- ELSE ---

|

| FLG_AC_CTL = 1

| Increment AC_SLP_TMR

25-71

TRANSMISSION INPUT CONVERSIONS, A/C ENABLE CONTROL  - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|ERR_T0| <= ERR_T0_AC ------------|

|AND -| Increment AC_CTL_TMR

|SLIP_TRG_S - SLIP_DES_S| <=      |     |

SLIP_DIF_AC ------|     |

| --- ELSE ---

|

| AC_CTL_TMR = 0

AC_CTL_TMR >= AC_CTL_TM ----------|

|OR --|  FLG_SLP_CTL = 1

FLG_SLP_CTL = 1 ------------------|     |

| --- ELSE ---

|

| FLG_SLP_CTL = 0

AC_SLP_TMR >= AC_SLP_WDOG --------|

|

FLG_SLP_CTL = 1 ------------------|OR --| AC_EN_TRANS = 1

|     |

FLG_CRV_LK = 0 -------------------|     |

| --- ELSE ---

|

| AC_EN_TRANS = 0

AC_EN_TRANS = 1 ------------------|

|AND -| FLG_AC_CMP = 1

ACITMR >= AC_CTL_OFF -------------|     | FLG_AC_CTL = 0

|

| --- ELSE ---

|

| NO ACTION

25-72

TRANSMISSION INPUT CONVERSIONS, Input Torque Calculation - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

25.17  TRANSMISSION INPUT TORQUE CALCULATION (CDAL0)

OVERVIEW

The engine torque calculation is used to determine the magnitude and sign  of

torque  being  transmitted to the transmission.  If transmission input torque

is positive, a load must be present (ie the vehicle is power on).

The logic works as follows:

TRANS_TQ_IN = TQ_BRK_SBAR - INERTIA_ENG * DNDT_TQI

Where:

-  TRANS_TQ_IN = Torque into the transmission.

-  TQ_BRK_SBAR = Filtered engine combustion torque.

-  INERTIA_ENG = Engine intertia.

-  DNDT_TQI = Engine acceleration.

-  INERTIA_ENG  *  DNDT_TQI  =  Combustion   torque   required   to

accelerate the engine.

DEFINITIONS

INPUTS

Registers:

-  DNDT_TQI = Derivative of engine RPM for use in transmission input  torque

calculation, RPM/second.

-  TQ_BRAKE_S = Net engine torque (signed).

-  TQ_BRK_SBAR = Filtered net engine torque (signed).

Calibration Constants:

-  INERTIA_ENG = Engine intertia.

-  TCTQBRKS = Time constant for filtered net engine torque (signed).

-  TCDNDT_TQI = Time constant for DNDT_TQI.

OUTPUTS

Registers:

-  DNDT_TQI = See above

-  TQ_BRK_SBAR = Filtered net engine torque (signed).

25-73

TRANSMISSION INPUT CONVERSIONS, Input Torque Calculation - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TRANS_TQ_IN = Transmission input torque, used as power on/off indicator.

25-74

TRANSMISSION INPUT CONVERSIONS, Input Torque Calculation - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INTRN_TQ_IN_CALC_COM2

Once per Background the following logic is executed:

unconditionally ------------------------| TQ_BRK_SBAR = ROLAV(TQ_BRAKE_S,

|       TCTQBRKS)

| ;Calculate filtered signed torque

|

| DNDT_TQI = ROLAV(DNDTI, TCDNDT_TQI)

| ;Calculate filtered engine

| ;acceleration

|

| TRANS_TQ_IN = (TQ_BRK_SBAR -

|     INERTIA_ENG * DNDT_TQI)

| ;Calculate input torque

25-75

TRANSMISSION INPUT CONVERSIONS, DIGITAL TRS CONVERSION  - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

25.18  DIGITAL TRANSMISSION RANGE SENSOR INPUT CONVERSION (CDAN0)

OVERVIEW

This module determines whether to execute the analog  Manual  Lever  Position

Sensor (MLPS) input conversion logic or the digital Transmission Range Sensor

(TRS) input conversion logic.  Which strategy is executed is  dependent  upon

the  value  of  TRS_TYPE.   The  following  is  a guide to calibration switch

settings for the MLPS/TRS/NDS conversion strategies:

Application                      TSTRAT      TRS_TYPE

--------------------------       ------      --------

A/T with analog MLPS               >3           0

A/T with digital TRS               xxx          1

M/T with Neutral/Drive Switch      <4          xxx

NOTE: "xxx" indicates the setting is irrelevant.

The digital Transmission Range Sensor (TRS) utilizes four binary inputs which

together can be decoded into a gear selector position.  The inputs, TR1, TR2,

TR3, and TR4, are decoded as follows:

gear selector

TR4  TR3  TR2  TR1    position (PDL)

---  ---  ---  ---    -------------

0    0    0    0     PARK (7)

1    1    0    0     REVERSE (6)

0    1    1    0     NEUTRAL (5)

1    1    1    1     DRIVE (3/4)

1    0    0    1     SECOND (2)

0    0    1    1     FIRST (1)

0 = Switch closed

1 = Switch open

Inputs TR4, TR2, and TR1 are read from EEC digital inputs.  TR3 is  converted

from the MLPS/NDS analog input.  The analog-to-digital conversion follows the

follwing logic:

Input condition                      TR3 setting

------------------------           ----------------------

0 <= INDS <= TR3A_CLOSED            Switch closed (0)

TR3A_CLOSED < INDS <= TR3A_OPEN     Switch open (1)

INDS > TR3A_OPEN                    ERROR - Open SIG RTN

Base values:  TR3A_CLOSED = 266 counts (1.3V)

TR3A_OPEN = 532 counts (2.6V)

During a transition from one gear selector  position  to  another,  all  four

switches  will  be  momentarily open.  Since this is the same state as DRIVE,

the logic employs a state transition  timer  (TM_PDL_RES)  to  prevent  false

recognition  of the DRIVE position.  The transition bit pattern (0100) is the

same between Reverse to Park and Reverse  to  Neutral.   The  transition  bit

pattern  (1110) identifies the TRS between Neutral and Drive.  The transition

bit pattern between Drive to Manual 2 and Manual 2 to Manual 1 is 1011.

25-76

TRANSMISSION INPUT CONVERSIONS, DIGITAL TRS CONVERSION  - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  INDS = Neutral/Drive or MLPS analog input (A/D COUNTS).

-  IPDL = Unverified MLPS/PRNDL position.

-  IPDL_LST = Last pass unverified MLPS/PRNDL position.

-  PDL = PRNDL/TRS/MLPS position.

-  PDL_ERR_TMR = MLPS/PRNDL malfunction test error timer (SECONDS).

-  PDL_MON_TMR = MLPS monitor timer (SECONDS).

-  TM_PDL_RES = MLPS/PRNDL input residence timer (SECONDS).

-  TRNS_ERR_TMR  =  TRS/PRNDL  transition  malfunction  test  error   timer,

(SECONDS).

Bit Flags:

-  CCM_EO_ENA = Engine off on-demand test for CCM enabled.

-  CCM_TST_ENA = OBD-II Comprehensive Component Test enabled.

-  FLG_TCS = Transmission Control Switch Flag.

-  FLG_TRNS_BIT = Indicate if the TRS is between positions, a transition bit

pattern has been decoded.

-  OBDII_RESET = Flag used to simulate the receipt of an  OBD-II  scan  tool

reset message.

-  OFMFLG = ETV solenoid failure flag.

-  PxxxMALF = Malfunction flag for code Pxxx; 1 -> a  malfunction  currently

exists for Pxxx.

-  PDL_ERROR = MLPS/TRS FMEM failure mode flag.

-  PDL_MON = MLPS monitored flag.

-  TR1 = Digital transmission range sensor input, Bit 1.

-  TR2 = Digital transmission range sensor input, Bit 2.

-  TR3 = Digital transmission range sensor input, Bit 3.

-  TR3_ERROR = Analog input TR3A out of range error flag.

-  TR4 = Digital transmission range sensor input, Bit 4.

Calibration Constants:

-  CCM_TST_SW = CCM test presetn calibration switch.

25-77

TRANSMISSION INPUT CONVERSIONS, DIGITAL TRS CONVERSION  - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FMMMLP = PDL value when error present.

-  FNCPTRS = Function to provide the  PDL  value  from  the  output  of  the

digital TRS.

-  MLPS_KOEO_EN = MLPS KOEO test enable switch.

-  PDL_ERR_TM = MLPS/PRNDL malfunction test error time threshold (SECONDS).

-  PDL_MON_TM = Minimum time MLPS/TRS must be observed to set  monitor  flag

(SECONDS).

-  PDLTIM = Minimum time to recognize new MLPS/TRS state (SECONDS).

-  TR3A_CLOSED = A/D input value below which TR3 switch is considered closed

(A/D COUNTS).

-  TR3A_OPEN = A/D input value below which TR3  switch  is  considered  open

(A/D COUNTS).

-  TRS_TYPE = Type of transmission range sensor; 0  ->  Analog  MLPS,  1  ->

Digital TRS.

-  TRNS_ERR_TM = Minimum time for a transition bit pattern  to  be  observed

before indicating a failure, (SECONDS).

OTHER

-  P0705 = Transmission Range Sensor circuit malfunction (MIL).

-  P1705 = Transmission Range Sensor not indicating required position during

KOEO (non-MIL).

-  P1704 = Transmission Range Sensor is misaligned and is  reading  a  value

between the indicated PRNDL position.

25-78

TRANSMISSION INPUT CONVERSIONS, DIGITAL TRS CONVERSION  - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INTRN_TRS_COM1

BEGIN: transmission_range_sensor_selection

; This process is executed once per background loop.

; Determine which strategy to execute:

TRS_TYPE = 0 -------------------------| Do: INTRN_MLPS_CONV_COMx

;Analog MLPS                         |  call analog TRS module

|

| --- ELSE ---

|

| Do: digital_transmission_range_sensor

END: transmission_range_sensor_selection

BEGIN: digital_transmission_range_sensor

; This process is executed only when called.

unconditionally ------------------------| IPDL_LST := IPDL

| ; Save last decoded

| ; state of TRS

|

| Do: tr3a_conversion

| Do: trs_decode

| Do: trs_self_test

END: digital_transmission_range_sensor

25-79

TRANSMISSION INPUT CONVERSIONS, DIGITAL TRS CONVERSION  - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: tr3a_conversion

; This process is executed only when called.

; TRS input, bit 3, is received on an analog input.

; This signal must be converted to a binary value.

INDS <= TR3A_CLOSED --------------------| TR3 := 0

;Analog input level indicates          |

;switch is closed                      | TR3_ERROR := 0

| ; Input is valid.

|

| --- ELSE ---

|

INDS <= TR3A_OPEN ----------------------| TR3 := 1

;Analog input level indicates          |

;switch is open                        | TR3_ERROR := 0

| ; Input is valid.

|

| --- ELSE ---

|

| TR3_ERROR := 1

| ; Analog input level

| ; indicates an open

| ; SIG RTN wire.

END: tr3a_conversion

25-80

TRANSMISSION INPUT CONVERSIONS, DIGITAL TRS CONVERSION  - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

; A function FNCPTRS has been provided to calibrate the bit

; pattern for either levels of ditigal TRS', (SP or CP).  The base

; DECIMAL calibration values for the funciton are set to work for

; the CP level of hardware and is listed below:

;FNCPTRS[0] = 7

;FNCPTRS[1] = 127.5

;FNCPTRS[2] = 127.5

;FNCPTRS[3] = 1

;FNCPTRS[4] = 7.5

;FNCPTRS[5] = 127.5

;FNCPTRS[6] = 5

;FNCPTRS[7] = 127.5

;FNCPTRS[8] = 127.5

;FNCPTRS[9] = 2

;FNCPTRS[10] = 127.5

;FNCPTRS[11] = 120

;FNCPTRS[12] = 6

;FNCPTRS[13] = 127.5

;FNCPTRS[14] = 0

;FNCPTRS[15] = 4

;Below is the recommendation for calibrating FNCPTRS for SP level of hardware

;for digital TRS.

;FNCPTRS[0] = 7

;FNCPTRS[1] = 127.5

;FNCPTRS[2] = 127.5

;FNCPTRS[3] = 1

;FNCPTRS[4] = 127.5

;FNCPTRS[5] = 2

;FNCPTRS[6] = 5

;FNCPTRS[7] = 127.5

;FNCPTRS[8] = 127.5

;FNCPTRS[9] = 127.5

;FNCPTRS[10] = 127.5

;FNCPTRS[11] = 127.5

;FNCPTRS[12] = 6

;FNCPTRS[13] = 127.5

;FNCPTRS[14] = 127.5

;FNCPTRS[15] = 4

25-81

TRANSMISSION INPUT CONVERSIONS, DIGITAL TRS CONVERSION  - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: trs_decode

; This process is executed only when called.

; Decode the four binary TRS inputs and check their validity.

TR3_ERROR = 0 --------------------------| trs_input :=

;Valid analog input for TR3            | (TR4 * 8) + (TR3 * 4) +

| (TR2 * 2) + TR1

| ; Convert the four binary

| ; inputs to an integer

|

| IPDL := FNCPTRS(trs_input)

| ; Get TRS state corresponding

| ; to current switch positions

|

| --- ELSE ---

|

| IPDL := 0xFF

| ; Indicate invalid TRS state

25-82

TRANSMISSION INPUT CONVERSIONS, DIGITAL TRS CONVERSION  - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

;Determine value of PDL.

IPDL = 0xFF ----------------------------| PDL_MON_TMR := 0

;Current TRS state is invalid          | ; Clear timer used to

| ; control monitor flag

|

| --- ELSE ---

|

IPDL <> IPDL_LST -----------------------| TM_PDL_RES := PDLTIM

;TRS position has changed.             | ; Initialize transition

| ; timer.

|

| PDL_ERR_TMR := 0

| ; Clear TRS error timer

|

| --- ELSE ---

|

TM_PDL_RES = 0 -------------------------| Do: save_new_pdl

;Transition timer has expired.         | ; Assume TRS in valid

| ; state.

|

| PDL_ERR_TMR := 0

| ; Clear TRS error timer

|

| --- ELSE ---

|

| PDL_ERR_TMR := 0

| ; Clear TRS error timer

;If a transition bit pattern has been decoded indicate that the

;TRS is in transition and allow the transition error timer to

;continue to count up. Otherwise, clear the timer if the TRS

;is not within a transition region.

IPDL = 0xF0 -----------------------|

|

IPDL = 0x00 -----------------------|OR -| FLG_TRNS_BIT = 1

|    |

IPDL = 0x0F -----------------------|    | --- ELSE ---

|

| FLG_TRNS_BIT = 0

| TRNS_ERR_TMR = 0

END: trs_decode

25-83

TRANSMISSION INPUT CONVERSIONS, DIGITAL TRS CONVERSION  - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: save_new_pdl

; This process is executed only when called.

IPDL = 0x0F ----------------------------| PDL = 6

|

| --- ELSE ---

|

IPDL = 0x00 ----------------------------| PDL = 4

|

| --- ELSE ---

|

IPDL = 0xF0 ----------------------------| PDL = 2

|

| --- ELSE ---

|

IPDL <> 4 ------------------------------| PDL := IPDL

|

| --- ELSE ---

FLG_TCS = 1 ----------------------|     |

;O/D cancel activated            |OR --| PDL := 3

|     |

OFMFLG = 1 -----------------------|     |

;EPC VFS short circuit fault present   |

| --- ELSE ---

|

| PDL := 4

END: save_new_pdl

BEGIN: trs_fmem

; This process is executed only when called.

; TRS position cannot be determined.

; Substitute FMEM value.

FMMMLP = 4 -----------------------|

;PDL FMEM value                  |

;is O/D                          |AND -| PDL := 3

|     |

FLG_TCS = 1 ----------------|     |     |

;O/D cancelled             |OR --|     |

|           |

OFMFLG = 1 -----------------|           |

;VFS EPC short circuit fault           |

| --- ELSE ---

|

| PDL := FMMMLP

END: trs_fmem

25-84

TRANSMISSION INPUT CONVERSIONS, DIGITAL TRS CONVERSION  - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: trs_self_test

; This process is executed only when called.

; Check entry conditions for self test.

CCM_EO_ENA = 1 -------------------|

;Engine Off test enabled         |AND -| Do: trs_engine_off_test

|     |

MLPS_KOEO_EN = 1 -----------------|     |

;MLPS KOEO test enabled                |

| --- ELSE ---

CCM_TST_ENA = 1 ------------------|     |

;Continuous CCM enabled          |     |

|     |

CCM_TST_SW = 1 -------------------|AND -| Do: trs_continuous_test

;Continuous CCM present          |     |

|     |

OBDII_RESET = 0 ------------------|     |

| --- ELSE ---

|

| Do: clear_malf(P1705)

| Do: clear_malf(P0705)

BEGIN: trs_engine_off_test

; This process is executed only when called.

; Insure TRS indicates vechicle is in PARK.

IPDL <> 5 ------------------------|

|AND -| Do: store_code(P1705)

IPDL <> 7 ------------------------|     |

;TRS not indicating PARK               | --- ELSE ---

|

| Do: clear_malf(P1705)

END: trs_engine_off_test

BEGIN: trs_continuous_test

; This process is executed only when called.

; Perform continuous tests on transmission range sensor.

IPDL = 0xFF ----------------------|

;Invalid TRS state               |AND -| Do: trs_fmem

|     | Do: malfunction(ccm,P0705)

PDL_ERR_TMR > PDL_ERR_TM ---------|     | PDL_ERROR := 1

;Error present long enough             |

| --- ELSE ---

|

FLG_TRNS_BIT = 1 -----------------|     | Do: trs_fmem

|AND -| Do: malfunction(ccm,P1704)

TRNS_ERR_TMR > TRNS_ERR_TM -------|     | PDL_ERROR = 1

;Transition error present long         |

;enough                                | --- ELSE ---

|

| Do: clear_malf(P0705)

| Do: clear_malf(P1704)

| PDL_ERROR := 0

25-85

TRANSMISSION INPUT CONVERSIONS, DIGITAL TRS CONVERSION  - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

P1704MALF = 1 --------------------|

|

P0705MALF = 1 --------------------|OR --| PDL_MON := 1

|     | ;TRS has been monitored

PDL_MON_TMR > PDL_MON_TM ---------|     | ;this power-up.

END: trs_continuous_test

25-86

TRANSMISSION INPUT CONVERSIONS, IMRC/TRANSMISSION INTERACTION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

25.19  IMRC/TRANSMISSION INTERACTION CONTROL (CDAN0)

OVERVIEW

Movement of the IMRC during a shift event will create  a  torque  disturbance

that  negatively  affects  shift  feel.   If the IMRC is scheduled to open or

close during the shift, it's position should not change.   Rather,  the  IMRC

will hold its position from the time that a new gear is commnded up until the

end of the shift (end of the inertia phase).

The upshift torque modulation flag, FLG_UP_TQM,  will  be  used  to  tie  the

opening  or  closing  of  the  IMRC  to  the actual end of the interia phase.

Because FLG_UP_TQM will not pinpoint the end exactly, a timer will  be  added

to  compensate.  The IMRC logic will also use a calibration switch, IMRC_SFT,

in conjunction with FLG_IMRC_HLD.  The switch  will  be  used  to  enable  or

disable this "hold during a shift" feature.

Note:  Timer values need to be considered carefully.  If  the  IMRC  is  held

closed too long after the end of the shift, and are then allowed to open, the

vehicle will exhibit a delayed surge of power.

DEFINITIONS

Registers:

-  HLD_IMRC_TMR = Timer to control how long past  the  end  of  the  inertia

phase to hold the IMRC in its current position.

Bit Flags:

-  FLG_UP_TQM = Upshift torque modulation flag.

-  FLG_IMRC_HLD = Indicates that IMRC should not change state during a shift

event.

-  FLG_IMRC_TM = Indicates imrc_hld_timer is counting down.

Calibration Constants:

-  HLD_IMRC_TM = Time to to disallow the IMRC to change position  after  the

end of a shift is detected.

25-87

TRANSMISSION INPUT CONVERSIONS, IMRC/TRANSMISSION INTERACTION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INTRN_IMRC_CTL_COM1

Set timer value after inertia phase of upshift has completed

FLG_UP_TQM = 0 ----------------------------|

(end of shift is detected)                |

|

FLG_IMRC_HLD = 1 --------------------------|

(IMRC state is being held)                |AND -| HLD_IMRC_TMR = HLD_IMRC_TM

|     | FLG_IMRC_TM = 1

FLG_IMRC_TM = 0 ---------------------------|     | (load IMRC hold timer)

|     |

HLD_IMRC_TMR = 0 --------------------------|     |

(IMRC hold timer has not been set)              | --- ELSE ---

|

FLG_IMRC_HLD = 0 --------------------------------| FLG_IMRC_TM = 0

| (Reset flag for next

|  upshift)

Determine state of FLG_IMRC_HLD

FLG_UP_TQM = 1 ----------------------------------|

(An upshift has been commanded)                 |OR --| FLG_IMRC_HLD = 1

|     | (disallow IMRC from

HLD_IMRC_TMR > 0 --------------------------------|     |  changing state

(or, timer is set)                                    |  during an upshift)

|

| --- ELSE ---

|

| FLG_IMRC_HLD = 0

25-88

TRANSMISSION INPUT CONVERSIONS, OSS INPUT PROCESSING AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

25.20  OSS INPUT PROCESSING AND SELF TEST (CDAN0)

OVERVIEW

This module performs the foreground processing  of  the  output  shaft  speed

input,  the  calculation  of  unfiltered  output shaft speed (NO), and output

shaft speed sensor testing.

The foreground processing logic counts the number of rising edges  since  the

last  output shaft speed calculation was performed in background, and records

the time of the most recent rising edge.

Once a background loop, the ouput shaft speed is calculated if at  least  one

new  rising  edge occurred since the last calculation.  If the time since the

last edge becomes large, the shaft speed is set to zero, indicating the shaft

is stopped, or the sensor has failed.

The output shaft speed sensor test verifies sensor output is present when the

vehicle  is  moving.  The test relies on the inferred vehicle motion detector

flag (VMOVING) to determine vehicle motion.  If  a  failure  is  present  (no

output  when  output  is  expected)  for  a  sufficient  amount  of  time,  a

malfunction flag is set (P0720MALF), and failure mode action is initiated.

In the event that a failure is intermittent,  failure  mode  action  will  be

executed  for  a  calibratable  amount  of  time  after  the sensor begins to

function again.

Once a fault has been detected, or the output shaft  speed  sensor  has  been

observed  to  function  for  a  calibratable amount of time, the output shaft

speed sensor monitor flag (P0720MON) will be set.

DEFINITIONS

Registers:

-  NO = Transmission output shaft speed, rpm.

-  NO_PREV = Last background pass value of  unfiltered  transmission  output

shaft speed, rpm.

-  NOBART = Filtered transmission output shaft speed.

-  OSCTR = Count of unrealistic changes of output speed.

-  OSSCNT = Number of OSS rising edges since last sample.

-  OSSTIM1 = Time of the last valid edge event captured by  the  DARC,  DARC

ticks.

-  OSSTIM2 = Time interval between background samples, DARC ticks.

25-89

TRANSMISSION INPUT CONVERSIONS, OSS INPUT PROCESSING AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  OSSTMR = Output (shaft) speed sensor fault timer.

-  OSS_FM_TMR = Output (shaft) speed sensor failure mode timer.

-  OSS_MON_TMR = Output (shaft) speed sensor monitor timer.

-  OS_NOISE_CTR =  Count  of  unrealistic  changes  in  output  shaft  speed

observed during the current power-up.

-  TCR_OSS = Time capture register for the OSS input  signal  to  the  DARC,

DARC ticks.

-  TSLOSS = Time since last OSS interrupt, sec.

-  TSLOS_SPK = Time since last OS spike, seconds.

-  TSTRAT  =  Transmission  strategy  switch;  The  TSTRAT  software  switch

controls which transmission control strategy is executed.

0 -> No transmission control (Manual trans., AOD, ATX, C6, C3, etc.)

1 -> SIL (Shift indicator light)

2 -> A4LD with 3-4 shift control and converter clutch control

3 -> AXOD

4 -> C64E (E4OD)

5 -> A4LDE

6 -> FAX-4

7 -> AOD-E (AOD-I)

8 -> 4EAT

9 -> CD4E

Bit Flags:

-  CCM_TST_ENA = comprehensive components monitor test  enable  flag;  1  ->

diagnostic  executive  has  determined  that  codintions  to  perform ccm

testing have been met.

-  OSFMFLG = Output (shaft) speed sensor failure mode flag.

-  P0720MON = OBDII fault 0720 monitor  flag;  1  ->  fault  0720  has  been

sufficiently  monitored  this "trip".  (the sensor has been tested and is

functioning or a fault was detected.)

-  PXXXMALF = OBDII fault xxx malfunction flag; 1 -> a malfunction for fault

xxx currently exists.

-  VMOVING = Flag indicating inferred vehicle motion.

Calibration Constants:

-  MAX_NODELT = Maximum realistic change in unfiltered  output  shaft  speed

from one background loop to the next.

-  MNOSSPER = Minimum time between OSS rising  edges  for  the  edge  to  be

valid,  ticks.   Value depends on maximum engine rpm, number of edges per

revolution and clock frequency.

-  MX_TSLOS_SPK = Maximum time between OS spikes for which the noise counter

will be updated.

25-90

TRANSMISSION INPUT CONVERSIONS, OSS INPUT PROCESSING AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  OSCNT = Upcount for unrealistic changes in output shaft speed.

-  OSSPUL = Number of OSS pulses per revolution.

-  OSSMIN = Minimum acceptable oss output when vehicle motion is inferred.

-  OSSTIM = Minimum time with inferred vehicle motion and no output  (shaft)

speed sensor output required to recognize a fault.

-  OSS_FM_TIM = Minimum time the output (shaft) speed sensor  must  function

after a detected failure before the sensor failure mode will be exited.

-  OSS_MON_TIM = Minimum time  the  output  (shaft)  speed  sensor  must  be

observed to function before the monitor flag will be set.

-  OSS_SW = Output shaft speed sensor test calibration switch; 1 -> oss test

cal'ed in.

-  OS_NOISE_LVL = Noisy OSS fault threshold.

OTHER

-  oss_codes = SET OF {P0720, P0721} the  set  of  OBDII  fault  codes  that

relate to the oss.

-  ccm_malfunction {P0720}  =  Logic  process,  imported  from  MIL  control

module; Pxxx indicates a fault code.

-  clear_malf(Pxxx)  =  Logic  process,  used  to  clear  malfunction   flag

indicated by Pxxx.

-  P0720 = Fault code, oss failure.

-  P0721 = Fault code, noisy OSS failure.

25-91

TRANSMISSION INPUT CONVERSIONS, OSS INPUT PROCESSING AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INTRN_OSS_OBDII_COM1

Do the following in foreground.

always ---------------------------------| OSSTIM1 = TCR_OSS

FIRST_OSS = 0 --------------------------| OSSTIM2 = OSSTIM1

| OSSCNT = 0

| FIRST_OSS = 1

|

| --- ELSE ---

|

DT_OSS < MNOSSPER ----------------------| OSS period too short,

(OSS noise filter)                     | must be noise, ignore edge

| Exit OSS routine

|

| --- ELSE ---

|

| OSSCNT = OSSCNT + 1

Do the following once per background loop:

unconditionally ------------------------| NO_PREV = NO

| ;Save the last background value

| ;of unfiltered output shaft speed

TSTRAT <= 3 ----------------------------| EXIT MODULE

(manual/non-electric transmission)     |

| --- ELSE ---

|

TSLOSS >= 255 msec ---------------------| NO = 0

(less than 59 rpm if 4 pulses/rev)     | NOBART = 0

|

| --- ELSE ---

|

| NO = ((60 * OSSCNT) / (OSSPUL *

|   (OSSTIM1 - OSSTIM2) * dtscf))

| TSLOSS = 0

Where, dtscf = DARC ticks to seconds conversion factor.

25-92

TRANSMISSION INPUT CONVERSIONS, OSS INPUT PROCESSING AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Check for unrealistic changes in output shaft speed:

abs(NO - NO_PREV) > MAX_NODELT ---------| OSCTR = OSCTR + OSCNT

;Unrealistic change in NO              |  ;Upcount OSCTR for bad event

|

| increment(OS_NOISE_CTR)

| ;Upcount the noisy OSS event

| ;counter

|

| TSLOS_SPK = 0

| ;Zero time since last OS spike

|

| --- ELSE ---

|

NO > 0 ---------------------------------| Decrement OSCTR

;Output from OSS present               |  ;Downcount OSCTR if there is

|  ;output from the OSS

Check to see if enough unrealistic changes in output speed have occurred

to store a fault code.

NOTE: It has been observed  driver  induced spikes due  to operation  of ice

or due  to high engine  rpm engagements (where the driver squawks  the

tires)  or any time the brakes are  applied quickly (tiers locked up),

are always  followed  by long periods  (seconds) of  time where  OS is

stable.

A maximum time since last OS spike will be used to clear the bad event

counter -  which will prevent driver induced  spikes from tricking the

test into setting false OS noise fault codes.

TSLOS_SPK > MX_TSLOS_SPK ---------------| OS_NOISE_CTR = 0

;Time since last OS spike indicates    | ;zero the OS noise counter

;spike was NOT due to noise            |

OS_NOISE_CTR > OS_NOISE_LVL ------------| store_code(P0721)

;enough bad events to store a code     | ;store a noisy

| ;OSS fault code

25-93

TRANSMISSION INPUT CONVERSIONS, OSS INPUT PROCESSING AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The output shaft speed sensor test verifies that  sensor  output  is  present

when output is expected.

OSS_SW = 0 -----------------------|

(test cal'ed out)                |OR --| OSSTMR = 0

|     | OSS_MON_TMR = 0

CCM_TST_ENA = 0 ------------------|     | EXIT

(CCM tests disabled)                   |

| --- ELSE ---

VMOVING = 1 ----------------------|     |

(vehicle motion inferred)        |     |

|     |

NOBART < OSSMIN ------------------|AND -| malfunction(ccm,P0720)

(no output from output)          |     |

shaft speed sensor)             |     | OSFMFLG = 1

|     | OSS_FM_TMR = 0

OSSTMR > OSSTIM ------------------|     |

(error present long enough)            |

|

| --- ELSE ---

OSFMFLG = 1 ----------------------|     |

(currently in FMEM)              |AND -| OSS_FM_TMR = 0

|     | (no sensor signal, clear FMEM

NOBART < OSSMIN ------------------|     |  timer - remain in FMEM for

(no output from sensor)                |  a calibratable amount of time

|  after signal returns)

|

| --- ELSE ---

OSFMFLG = 1 ----------------------|     |

(currently in FMEM)              |     |

|     |

OSS_FM_TMR > OSS_FM_TIM ----------|AND -| OSFMFLG = 0

(minimum time in FMEM            |     | (exit FMEM)

has passed)                     |     |

|     | clear_malf(P0720)

NOBART >= OSSMIN -----------------|     |

(sensor output present)                |

| --- ELSE ---

NOBART >= OSSMIN -----------------|     |

(sensor output present)          |OR --| OSSTMR = 0

|

VMOVING = 0 ----------------------|

(no inferred vehicle motion)

OSS_MON_TMR > OSS_MON_TIM --------|

|OR --| P0720MON = 1

P0720MALF = 1 --------------------|     | (the output shaft speed sensor has

|  been monitored - the sensor is

|  functioning or a fault has been

|  detected)

|

| --- ELSE ---

|

NOBART < OSSMIN ------------------------| OSS_MON_TMR = 0

25-94

TRANSMISSION INPUT CONVERSIONS, VEHICLE SPEED SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

25.21  VEHICLE SPEED SELF TEST (CDAN0)

OVERVIEW

This module contains four vehicle speed sensor routines:

-  unrealistic_delta_vs_check = A  check  for  for  unrealistic  changes  in

unfiltered vehicle speed.

-  vss_reliable_check = An instantaneous check on  VSS  reliability  (FFG_VS

logic).

-  vehicle_speed_sensor_test = The VSS self test logic.

-  vss_monitor = additional fault filtering logic for manual applications.

The vehicle speed sensor test verifies sensor  output  is  present  when  the

vehicle  is  moving.  The test relies on the inferred vehicle motion detector

flag (VMOVING) to determine vehicle motion.  If a failure is present (no  VSS

output  when  output  is  expected)  for  a  sufficient  amount  of  time,  a

malfunction flag is set (P0500MALF), and failure mode action is initiated.

In the event that a failure is intermittent,  failure  mode  action  will  be

executed  for  a  calibratable  amount  of  time  after  the sensor begins to

function again.

Once a fault has been detected, or the vehicle speed sensor has been observed

to  function  for  a  calibratable  amount  of time, the vehicle speed sensor

monitor flag (P0500MON) will be set.

Additional fault filter logic has been added for  manual  transmissons  which

utilizes  and  error  counter  (VSS_ERR_CTR)  and  a calibratable error level

(VSS_ERR_LVL) to control the P0500MALF malfunction flag.

DEFINITIONS

Registers:

-  TSLVS_SPK = Time since last VS spike, seconds.

-  VS_NOISE_CTR = Count of unrealistic changes  in  vehicle  speed  observed

this power-up.

-  VS_HW = Unfiltered vehicle speed from VSS hardware.

-  VS_PREV = Previous calculated value of VS (not updated every background).

-  VSS_ERR_CTR = VSS error counter

-  VSBARV = Filtered vehicle speed for self test.

25-95

TRANSMISSION INPUT CONVERSIONS, VEHICLE SPEED SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  VSCTR = Count of unrealistic vehicle speed changes.

-  VSSTMR = Vehicle speed sensor fault timer.

-  VSS_FM_TMR = Vehicle speed sensor failure mode timer.

-  VSS_MON_TMR = Vehicle speed sensor monitor timer.

Bit Flags:

-  CCM_TST_ENA = OBD-II Comprehensive Component Test enable flag; 1 ->  test

enabled.

-  FFG_VS = OBD-II system FMEM  flag  for  the  VSS;  1  ->  VSS  signal  is

currently unreliable.

-  PxxxMALF = OBDII malfunction flag for fault  Pxxx;  1  ->  a  malfunction

currently exists for fault Pxxx.

-  PxxxMON = OBDII monitor flag for fault Pxxx; 1 ->  fault  Pxxx  has  been

monitored at least once since power-up.

-  VMOVING = Flag indicating inferred vehicle motion.

-  VSFMFLG = Vehicle speed sensor failure mode flag.

-  VSS_MON_FLG = Vehicle speed sensor monitor flag; 1 -> vehicle  speed  has

been monitored functioning this power-up.

Calibration Constants:

-  MAX_VSDELT = Maximum realistic change  in  unfiltered  vehicle  from  one

background loop to the next, ignoring slick surfaces.

-  MX_TSLVS_SPK = Maximum time between VS spikes for which the noise counter

will be updated.

-  VSCNT = Increment to VSCTR indicating failed MPH sensor.

-  VSSMN1 = Minimum acceptable vehicle  speed  sensor  output  when  vehicle

motion is inferred.

-  VSSTIM = Minimum time with inferred vehicle motion and no  vehicle  speed

sensor output required to recognize a fault.

-  VSS_ERR_LVL = VSS error fault threshold.

-  VSS_ERR_UP = VSS error upcount.

-  VSS_FM_TIM = Minimum time the vehicle speed sensor must function after  a

detected  failure  before  the  vehicle speed sensor failure mode will be

exited.

-  VSS_MON_TIM = Minimum time the vehicle speed sensor must be  observed  to

function before the VSS monitor flag will be set.

-  VS_NOISE_LVL = Fault threshold for a noisy VSS malfunction.

25-96

TRANSMISSION INPUT CONVERSIONS, VEHICLE SPEED SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OTHER

-  vs_codes = SET OF {P0500} the set of OBDII fault codes that relate to the

vehicle speed sensor.

-  malfunction(ccm,Pxxx) = Locic process,  imported  from  the  MIL  control

module; Pxxx indicates a fault code.

-  P0500 = Fault code, vehicle speed sensor failure.

-  P0503 = Fault code, vehicle speed sensor noisy.

25-97

TRANSMISSION INPUT CONVERSIONS, VEHICLE SPEED SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INTRN_VSS_TEST_COM3

The following logic is called from the VSS input processing module after

vehicle speed calculations are made:

unconditionally -----------------------| Do: unrealistic_delta_vs_check

| Do: vss_reliable_check

CCM_TST_ENA = 1 -----------------------| Do: vehicle_speed_sensor_test

;Continuous CCM testing enabled       |

| --- ELSE ---

|

| VSSTMR = 0

| ;Hold VSS error timer at zero

|   when the VSS test is not run

BEGIN: unrealistic_delta_vs_check

Check for unrealistic changes in vehicle speed:

abs(VS_HW - VS_PREV) > MAX_VSDELT ------| VSCTR = VSCTR + VSCNT

;Unrealistic change in vs              |  ;Upcount VSCTR for bad event

|

| increment(VS_NOISE_CTR)

| ;Upcount the noisy VS event

| ;counter

|

| TSLVS_SPK = 0

| ;Zero time since last VS spike

|

| --- ELSE ---

|

VS_HW > 0 ------------------------------| Decrement VSCTR

;Output from VSS present               |  ;Downcount VSCTR if there is

|  ;output from the VSS

25-98

TRANSMISSION INPUT CONVERSIONS, VEHICLE SPEED SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Check to see if enough unrealistic changes in vehicle speed have occurred

to store a fault code.

NOTE:

the vehcile speed sensor does not  sense  actual  vehicle  speed  -  it

senses a shaft or wheel speed.  This means calculated vehicle speed can

change by large, unrealistic amounts any  time  the  tires  slip  (even

though true vehicle speed has not changed unrealistically).

It has been observed driver induced spikes due to operation on  ice  or

due  to  a  high  engine  rpm engagements (where the driver squawks the

tires) or any time the brakes are applied quickly  (tires  locked  up),

are  always  followed  by  long  periods  (seconds) of time where VS is

stable.

A maximum time since last VS spike will be used to clear the bad  event

counter  -  which  will prevent driver induced spikes from tricking the

test into setting false VS noise fault codes.

TSLVS_SPK > MX_TSLVS_SPK ---------------| VS_NOISE_CTR = 0

;Time since last VS spike indicates    | ;zero the VS noise counter

;spike was NOT due to noise            |

VS_NOISE_CTR > VS_NOISE_LVL ------------| store_code(P0503)

;enough bad events to store a code     | ;store a noisy

| ;VSS fault code

END: unrealistic_delta_vs_check

BEGIN: vss_reliable_check

Determine if the vehicle speed sensor signal is reliable enough to be used

in OBD-II CAT, EGO, and PURGE monitor entry conditions:

VSBARV < VSSMN1 ------------|

;No output from the VSS    |AND -|

|     |

VMOVING = 1 ----------------|     |

;Output from the VSS is          |

expected                        |OR --| FFG_VS = 1

|     | ;VSS signal suspect

VSCTR > 0 ------------------------|     |

;Unrealistic change in VS        |     |

|     |

VSFMFLG = 1 ----------------------|     |

;In vehicle speed sensor FMEM          |

| --- ELSE ---

|

| FFG_VS = 0

| ;VSS signal ok

END: vss_reliable_check

25-99

TRANSMISSION INPUT CONVERSIONS, VEHICLE SPEED SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: vehicle_speed_sensor_test

VSS_MON_TMR > VSS_MON_TIM --------------| decrement VSS_ERR_CTR

| Do: vss_monitor

| VSS_MON_TMR = 0

| ;VSS output present long enough to

| ;call event good, decrement error

| ;counter, call vss monitor, and

| ;zero timer for next event

|

| --- ELSE ---

|

VSBARV < VSSMN1 ------------------------| VSS_MON_TMR = 0

(No VSS output present)                | ;Zero monitor timer

VMOVING = 1 ----------------------|

;vehicle motion inferred         |

|

VSBARV < VSSMN1 ------------------|AND -| VSS_ERR_CTR = VSS_ERR_CTR +

;no output from VSS              |     |               VSS_ERR_UP

|     | Do: vss monitor

VSSTMR >= VSSTIM -----------------|     | VSSTMR = 0

;error present long enough             | VSFMFLG = 1

| VSS_FM_TMR = 0

| ;Upcount VSS fault filter,

| ;Zero error timer for next

| ;bad event, enter fmem on

| ;automatic transmission

| ;after 1 bad event

|

| --- ELSE ---

VSFMFLG = 1 ----------------------|     |

;currently in FMEM               |AND -| VSS_FM_TMR = 0

|     | ;no VSS signal, clear FMEM

VSBARV < VSSMN1 ------------------|     |  timer - remain in FMEM for

;no output from VSS                    |  a calibratable amount of time

|  after VSS returns

|

| --- ELSE ---

VSFMFLG = 1 ----------------------|     |

;currently in FMEM               |     |

|     |

VSS_FM_TMR > VSS_FM_TIM ----------|AND -| VSFMFLG = 0

;minimum time in FMEM            |     | ;exit fmem

has passed                      |     | ;Note: VSS fmem will be exited

|     | ;if VSS output is present for

VSBARV >= VSSMN1 -----------------|     | ;VSS_FM_TIM (even if VSS_ERR_CTR

;VSS output present                    | ;is non-zero).

|

| --- ELSE ---

VSBARV >= VSSMN1 -----------------|     |

;VSS output present              |OR --| VSSTMR = 0

|     | ;VSS is functioning, or

VMOVING = 0 ----------------------|     |  not enough information is

;no inferred vehicle motion            |  present to test the sensor

END: vehicle_speed_sensor_test

25-100

TRANSMISSION INPUT CONVERSIONS, VEHICLE SPEED SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: vss_monitor

Use the VSS error counter (VSS_ERR_CTR) to determine the status of the VSS.

NOTE: This logic has been added because a robust VMOVING strategy/calibration

for manual transmissions has not been developed. It is assumed the second

level of fault filtering (VSSTMR is the first) will not be used on automatic

transmission applications (for which VMOVING is robust).

VSS_ERR_CTR > VSS_ERR_LVL --------------| malfunction(ccm, p0500)

;enough bad events                     | P0500MON = 1

| ;VSS malfunctioning, store code

| ;and indicate VSS monitored.

|

| --- ELSE ---

|

VSS_ERR_CTR = 0 ------------------------| clear_malf(P0500)

;no bad events                         | P0500MON = 1

| ;VSS functioning, clear malfunction

| ;and indicate VSS monitored.

END: vss_monitor

25-101

TRANSMISSION INPUT CONVERSIONS, VEHICLE SPEED SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

25-102

CHAPTER 26

SHIFT CONTROL

26-1

SHIFT CONTROL, PRNDL BASED DESIRED GEAR DETERMINATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

26.1  PRNDL BASED DESIRED GEAR DETERMINATION (CDAN2)

OVERVIEW

The  desired  transmission  gear  is  calculated  based  on  PRNDL  position.

Possible gears are:

GR_CM   Transmission State      GEAR_CUR   RT_GR_CUR

------  ------------------      --------   ---------

1      1ST                        1        GRRAT1

2      2ND                        2        GRRAT2

3      3RD                        3        GRRAT3

4      4TH                        4        GRRAT4

In the normal Drive/Overdrive position, the desired gear is calculated  based

on a maximum WOT RPM shift point or as a function of throttle position versus

vehicle speed.  All shift points are adjusted for  altitude.   There  are  no

excluded  shifts  in automatic mode, that is 1 - 4 shifts or 3 - 1 shifts are

permitted if the calibration calls for it.  In addition,  the  gear  sequence

for a multi-step shift is completely calibratable.

In manual 2 or 1, desired gear is set to the ultimate  desired  gear:   2  in

manual 2 and 1 in manual 1.  Sequencing through the downshift routine is left

to the commanded gear routine.

The main outputs of the desired gear routine are:

-  GR_DS, the desired gear;

-  FLG_FRST_DS, global flag to indicate a shift is desired  this  background

pass.

DEFINITIONS

INPUTS

Registers:

-  GEAR_CUR = Global gear indicator, reflects only.

-  GEAR_OLD = Global gear indicator for last commanded.

-  GR_DS = Desired transmission gear.

-  GR_DS_LST = Desired gear in the last background pass.

-  GR_DS_TV = Commanded gear for TV.

-  PDL = Current PRNDL position.

-  PDL_LST = Last PRNDL position.

26-2

SHIFT CONTROL, PRNDL BASED DESIRED GEAR DETERMINATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Bit Flags:

-  FLG_SFT_IN = Shift in progress flag.

Calibration Constants:

-  SW_SFTIN = Desired gear shift in progress switch; 0 -> FLG_SFT_IN = 0 for

new desired gear, 1 -> always determine new desired gear.

-  TM_M12 = Time that third gear solenoid states should be  commanded  on  a

manual 1-2 shift.

OUTPUTS

Registers:

-  GR_DS = See above.

-  GR_DS_LST = See above.

-  GR_DS_TV = See above.

-  IGR_DS = Unverified desired transmission gear.

-  TMR_M12 = Timer used to control commanding 3rd gear shift solenoid states

at a manual 1-2 shift.

-  TM_SFT_IN = Shift in progress timer.

Bit Flags:

-  FLG_FRST_DS = First time a shift is desired flag; 0 -> no shift  desired,

1 -> shift desired this background pass.

-  FLG_SFT_DN = Downshift flag.

-  FLG_SFT_IN = See above.

-  FLG_SFT_UP = Upshift flag.

26-3

SHIFT CONTROL, PRNDL BASED DESIRED GEAR DETERMINATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SC_DESRD_GR_DETR_COM10

PDL = 3 --------------------|

|OR --|

PDL = 4 --------------------|     |

|AND -| TM_SFT_IN = 0

PDL_LST <> 3 ---------------------|     | FLG_SFT_IN = 0

|     |

PDL_LST <> 4 ---------------------|     | (clear timer/flag if

|  you are making a manual

|  shift)

|     always ---------------------------------| Do: INTRN_SHFT_SCHLD_COMP_CALC

|                                             |   (calculate predictive vehicle

|                                             |    and engine speed)

|                                             | GR_DS_LST = GR_DS

PDL_LST = 1 ----------------------|

|

PDL = 2 --------------------------|AND -| TMR_M12 = TM_M12

|

GEAR_CUR = 1 ---------------------|

PDL = 1 --------------------------------| IGR_DS = 1

(PRNDL in manual 1)                    | GR_DS = 1

|

| --- ELSE ---

|

PDL = 2 --------------------------------| IGR_DS = 2

(PRNDL in manual 2)                    | GR_DS = 2

|

| --- ELSE ---

SW_SFTIN = 1 ---------------|           |

|OR --|     |

FLG_SFT_IN = 0 -------------|     |     |

(no shift in progress)           |     |

|AND -| Do "GR_DS, PRNDL = 3 OR 4" Logic

PDL = 3 --------------------|     |     | Do "DELAY/VERIFY SHIFT" Logic

(PRNDL in overdrive cancel)|OR --|     | Do "GR_SEQ_PNTR CALCULATION"

|           |

PDL = 4 --------------------|           |

(PRNDL in overdrive)                   |

| --- ELSE ---

PDL = 6 --------------------------|     |

(PRNDL in reverse)               |     |

|     |

PDL = 7 --------------------------|OR --| IGR_DS = 1

(PRNDL in park)                  |     | GR_DS = 1

|     | GR_DS_TV = 1

PDL = 5 --------------------------|

26-4

SHIFT CONTROL, PRNDL BASED DESIRED GEAR DETERMINATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

GR_DS_TV > GEAR_CUR --------------|

(upshift is being verified or    |

will be commanded this pass)    |

|OR --| FLG_SFT_UP = 1

GR_DS_TV = GEAR_CUR --------|     |     | FLG_SFT_DN = 0

(no shift pending)         |     |     |  (indicate upshift)

|AND -|     |

GEAR_CUR >= GEAR_OLD -------|           |

(last shift was an upshift)            |

| --- ELSE ---

|

| FLG_SFT_UP = 0

| FLG_SFT_DN = 1

|  (indicate downshift)

GR_DS <> GR_DS_LST ---------------------| FLG_FRST_DS = 1

(desired gear has changed)             |  (new desired gear for

|   this program pass only)

|

| --- ELSE ---

|

| FLG_FRST_DS = 0

26-5

SHIFT CONTROL, GR_DS, PRNDL = 3 OR 4 LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

26.2  GR_DS, PRNDL = 3 OR 4 LOGIC (CDAM0)

OVERVIEW

This module handles the Desired Gear computation when PRNDL = 3 or 4 based on

shift  curves  of  vehicle  speed  vs  relative  throttle  position.  Vehicle

acceleration rate compensation is accomplished by comparing the output of the

shift curves to a predicted vehicle speed.

DEFINITIONS

Registers:

-  BP_INTR = BP interpolation factor.

-  COMP_FCTR = Compensation factor for shfit schedules.

-  COOL_PI_INT = Integer PI controller output; number  of  cylinders  active

during fail-safe cooling.

-  CS_SFT_MULT = Cold start shift multiplier.

-  FSC_ADD_U = Value added to FN689U to lower shift  points  for  fail  safe

cooling mode.

-  FSC_ADD_D = Value added to FN689U to lower shift  points  for  fail  safe

cooling mode.

-  GR_CM = Commanded gear for shift solenoids.

-  HOT21_ADDR  =  2-1  downshift  adder  (FN21_HOT)  when  transmission   is

overtemperature; 0 -> when not overtemperature.

-  IGR_DS = Unverified desired transmission gear.

-  NEBART = Filtered engine RPM for transmission.

-  NEBART_PU = Predicted engine speed for an upshift from current gear.

-  OSCTR = Count of unrealistic changes in output shaft speed.

-  PDL = Current PRNDL position.

-  PDL_LST = PRNDL position last background pass.

-  PNWF = Performance and Normal Weighting Factor.

-  RT_NOVS = Ratio of actual N/V to base N/V in KAM.

-  TOT = Transmission oil temperature (degrees F).

-  TP_REL = Relative throttle position, counts.

-  TP_REL_H = Relative TP (TP - RATCH); high byte only.

26-6

SHIFT CONTROL, GR_DS, PRNDL = 3 OR 4 LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TQ_INTRP = Torque interpolation factor.

-  VSBART_RT_PD  =  Filtered  predicted  downshift  vehicle  speed  (vehicle

acceleration compensation).

-  VSBART_RT_PU  =  Filtered  predicted  upshift  vehicle   speed   (vehicle

acceleration compensation).

-  VS_RATEPH = Vehicle acceleration rate for Powertrain Hunting.

Bit Flags:

-  COOL_FLG = Overtemperature Flag :  1 -> overtemperature

-  FLG_4X4L = 4x4 low flag.

-  FLG_NE_WAIT = Wot engine RPM upshift has begun - wait for RPM drop before

checking for another.

-  FLG_UP_NE = WOT engine RPM upshift flag; 1 -> upshift due to WOT  RPM,  0

-> upshift due to shift curves.

-  OSFMFLG = Output shaft speed sensor FMEM flag.

-  VSFMFLG = Vehicle Speed Sensor FMEM flag.

Calibration Constants:

-  FN12COMP(TP_REL_H) = Vehicle speed adjustment  for  1  -  2  upshift  for

altitude or engine output torque loss.

-  FN12P(TP_REL_H) = Delta vehicle speed  for  1-2  upshift  in  performance

mode.

-  FN12S(TP_REL_H) = Vehicle speed for 1 - 2 upshift at sea level.

-  FN12S_4X4L(TP_REL_H) = Vehicle speed for 1 - 1 upshift at  sea  level  in

4X4L.

-  FN21COMP(TP_REL_H) = Vehicle speed adjustment for 2  -  1  downshift  for

altitude or engine output torque loss.

-  FN21_HOT(TP_REL_H)  =  2-1   downshift   adder   when   transmission   is

overtemperature.

-  FN21P(TP_REL_H) = Delta vehicle speed for 2-1  downshift  in  performance

mode.

-  FN21S(TP_REL_H) = Vehicle speed for 2 - 1 downshift at sea level.

-  FN21S_4X4L(TP_REL_H) = Vehicle speed for 2 - 1 downshift at sea level  in

4X4L.

-  FN23COMP(TP_REL_H) = Vehicle speed adjustment  for  2  -  3  upshift  for

altitude or engine output torque loss.

-  FN23P(TP_REL_H) = Delta vehicle speed  for  2-3  upshift  in  performance

mode.

26-7

SHIFT CONTROL, GR_DS, PRNDL = 3 OR 4 LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FN23PPH(TP_REL) = Min VS_RATEPH to allow 2 - 3 upshift.

-  FN23S(TP_REL_H) = Vehicle speed for 2 - 3 upshift at sea level.

-  FN23S_4X4L(TP_REL_H) = Vehicle speed for 2 - 3 upshift at  sea  level  in

4x4l.

-  FN32COMP(TP_REL_H) = Vehicle speed adjustment for 3  -  2  downshift  for

altitude or engine output torque loss.

-  FN32P(TP_REL_H) = Delta vehicle speed for 3-2  downshift  in  performance

mode.

-  FN32S(TP_REL_H) = Vehicle speed for 3 - 2 downshift at sea level.

-  FN32S_4X4L(TP_REL_H) = Vehicle speed for 3 - 2 downshift at sea level  in

4x4l.

-  FN34COMP(TP_REL_H) = Vehicle speed adjustment  for  3  -  4  upshift  for

altitude or engine output torque loss.

-  FN34P(TP_REL_H) = Delta vehicle speed  for  3-4  upshift  in  performance

mode.

-  FN34PPH(TP_REL) = Minimum VS_RATEPH to allow 3 - 4 upshift.

-  FN34S(TP_REL_H) = Vehicle speed for 3 - 4 upshift at sea level.

-  FN34S_4X4L(TP_REL_H) = Vehicle speed for 3 - 4 upshift at  sea  level  in

4x4l.

-  FN43COMP(TP_REL_H) = Vehicle speed adjustment for 4  -  3  downshift  for

altitude or engine output torque loss.

-  FN43P(TP_REL_H) = Delta vehicle speed for 4-3  downshift  in  performance

mode.

-  FN43S(TP_REL_H) = Vehicle speed for 4 - 3 downshift at sea level.

-  FN43S_4X4L(TP_REL_H) = Vehicle speed for 4 - 3 downshift at sea level  in

4x4l.

-  FN689D(TP_REL) = Engine speed for downshifts during VSS failure.

-  FN689U(TP_REL) = Engine speed for upshifts during VSS failure.

-  FNFSC_CYL = Multiplication value based on active cylinders  for  reducing

shift points in Fail Safe Cooling mode.

-  FNFSC_TPD = Multiplication value based on throttle position for  reducing

downshift points in Fail Safe Cooling.

-  FNFSC_TPU = Multiplication value based on throttle position for  reducing

upshift points in Fail Safe Cooling.

-  HOT21_C = Temperature at which the hot 2-1 shift schedule  is  no  longer

used.

26-8

SHIFT CONTROL, GR_DS, PRNDL = 3 OR 4 LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  HOT21_S = Temperature at which the hot 2-1 shift schedule is used.

-  NE12COMP = WOT RPM 1 - 2 shift point adjustment for  altitude  or  engine

output torque loss.

-  NE12S = WOT RPM 1 - 2 shift point, sea level.

-  NE12S4L = WOT RPM 1 - 2 shift point in 4X4L.

-  NE23COMP = WOT RPM 2 - 3 shift point adjustment for  altitude  or  engine

output torque loss.

-  NE23S = WOT RPM 2 - 3 shift point, sea level.

-  NE23S4L = WOT RPM 2 - 3 shift point in 4X4L.

-  NE34COMP = WOT RPM 3 - 4 shift point adjustment for  altitude  or  engine

output torque loss.

-  NE34S = WOT RPM 3 - 4 shift point, sea level.

-  NE34S4L = WOT RPM 3 - 4 shift point in 4X4L.

-  NEDROP = Engine RPM to  drop  before  checking  for  another  engine  RPM

upshift.

-  SFT_MPH = Minimum Vehicle speed to use VS in Shift logic,  MPH.   Typical

value is 0 mph.

-  SW_BP_INTR = 1 -> Select BP_INTR for shift schedule  compensation;  0  ->

select TQ_INTRP for shift schedule compensation.

26-9

SHIFT CONTROL, GR_DS, PRNDL = 3 OR 4 LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SC_GR_DS_AUTO_COM22

OSCTR = 0 ---------------------|

(OSS not noisy)               |OR --|

|     |

OSFMFLG = 1 -------------------|     |

(Vehicle speed is being             |

substituted for output speed)      |

|AND -| Do: "OUTPUT SHAFT SPEED SENSOR

OSFMFLG = 0 -------------------|     |     |      OK SHIFT LOGIC"

(output shaft sensor OK)      |OR --|     |

|     |     |

VSFMFLG = 0 -------------------|     |     |

(Vehicle Speed Sensor OK)           |     |

|     |

COOL_FLG = 0 ------------------------|     |

(Fail Safe Cooling inactive)              |

| --- ELSE ---

|

FLG_UP_NE = 0 -----------------------------| Do: "OUTPUT SHAFT SPEED SENSOR

|      FAILURE SHIFT LOGIC"

|

| --- ELSE ---

|

| EXIT MODULE

OUTPUT SHAFT SPEED SENSOR OK SHIFT LOGIC

TOT > HOT21_S ---------------------  ------| HOT21_ADDR = FN21_HOT(TP_REL_H)

(Transmission is hot; alter 2-1           |

shift schedule)                          | --- ELSE ---

|

TOT < HOT21_C -----------------------------| HOT21_ADDR = 0

|

| --- ELSE ---

|

| NO Action

SW_BP_INTR = 1 ----------------------------| COMP_FCTR = BP_INTR

|

| --- ELSE ---

|

| COMP_FCTR = TQ_INTRP

FLG_SFT_IN = 0 ----------------------------| FLG_NE_WAIT = 0

(no engine spd upshift in progress)       |

| --- ELSE ---

|

| No Action

26-10

SHIFT CONTROL, GR_DS, PRNDL = 3 OR 4 LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

GR_CM = 1 -----------------------------|

|

NEBART_PU > NE12S +                    |

[COMP_FCTR * NE12COMP] --|          |AND ------| IGR_DS = 2

|AND -|    |          | FLG_UP_NE = 1

FLG_4X4L = 0 ---------------|     |    |          | FLG_NE_WAIT = 1

|OR -|          |

NEBART_PU > NE12S4L --------|     |               |

|AND -|               |

FLG_4X4L = 1 ---------------|                     |

|

| --- ELSE ---

FLG_NE_WAIT = 0 -----------------------|          |

|          |

GR_CM = 2 -----------------------------|          |

|AND ------| IGR_DS = 3

NEBART_PU > NE23S +                    |          | FLG_UP_NE = 1

[COMP_FCTR * NE23COMP] --|          |          | FLG_NE_WAIT = 1

|AND -|    |          |

FLG_4X4L = 0 ---------------|     |    |          |

|OR -|          |

NEBART_PU > NE23S4L --------|     |               |

|AND -|               |

FLG_4X4L = 1 ---------------|                     |

|

| --- ELSE ---

PDL = 4 -------------------------------|          |

|          |

FLG_NE_WAIT = 0 -----------------------|          |

|          |

GR_CM >= 3 ----------------------------|AND ------| IGR_DS = 4

|          | FLG_UP_NE = 1

NEBART_PU > NE34S +                    |          | FLG_NE_WAIT = 1

[COMP_FCTR * NE34COMP] --|          |          |

|AND -|    |          |

FLG_4X4L = 0 ---------------|     |    |          |

|OR -|          |

NEBART_PU > NE34S4L --------|     |               |

|AND -|               |

FLG_4X4L = 1 ---------------|                     |

|

| --- ELSE ---

(continued on next page)

26-11

SHIFT CONTROL, GR_DS, PRNDL = 3 OR 4 LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

GR_CM < 4 ----------------------------------|     |

|     |

PDL = 4 ------------------------------------|     |

|     |

PDL <> PDL_LST ------------------|          |     |

|          |     |

VS_RATEPH > FN34PPH * RT_NOVS ---|OR -------|     |

|          |     |

IGR_DS = 4 ----------------------|          |AND -| IGR_DS = 4

|     | FLG_UP_NE = 0

VSBART_RT_PU > [FN34S +                     |     | FLG_NE_WAIT = 1

(COMP_FCTR * FN34COMP) + (PNWF            |     |

* FN34P)] * CS_SFT_MULT -------|          |     |

|AND -|    |     |

FLG_4X4L = 0 --------------------|     |    |     |

|OR -|     |

VSBART_RT_PU > [FN34S_4X4L +           |          |

(COMP_FCTR * FN34COMP) + (PNWF       |          |

* FN34P)} * CS_SFT_MULT -------|     |          |

|AND -|          |

FLG_4X4L = 1 --------------------|                |

| --- ELSE ---

GR_CM < 3 ----------------------------------|     |

|     |

PDL <> PDL_LST ----------------|            |     |

|            |     |

VS_RATEPH > FN23PPH *          |            |     |

RT_NOVS ---------|OR ---------|AND -| IGR_DS = 3

(min. accel. threshold met)   |            |     | FLG_UP_NE = 0

|            |     | FLG_NE_WAIT = 1

IGR_DS = 3 --------------------|            |     |

|     |

VSBART_RT_PU > [FN23S +                     |     |

(COMP_FCTR * FN23COMP) +                  |     |

(PNWF * FN23P)] * CS_SFT_MULT -|          |     |

|AND -|    |     |

FLG_4X4L = 0 --------------------|     |    |     |

|OR -|     |

VSBART_RT_PU > [FN23S_4X4L +           |          |

(COMP_FCTR * FN23COMP) +             |          |

(PNWF * FN23P)] * CS_SFT_MULT -|     |          |

|AND -|          |

FLG_4X4L = 1 --------------------|                |

|

| --- ELSE ---

(continued on next page)

26-12

SHIFT CONTROL, GR_DS, PRNDL = 3 OR 4 LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

GR_CM < 2 ----------------------------------|     |

|     |

VSBART_RT_PU > [FN12S +                     |     |

(COMP_FCTR * FN12COMP) +                  |AND -| IGR_DS = 2

(PNWF * FN12P)] * CS_SFT_MULT -|          |     | FLG_UP_NE = 0

|AND -|    |     | FLG_NE_WAIT = 1

FLG_4X4L = 0 --------------------|     |    |     |

|OR -|     |

VSBART_RT_PU > [FN12S_4X4L +           |          |

(COMP_FCTR * FN12COMP) +             |          |

(PNWF * FN12P)] * CS_SFT_MULT -|     |          |

|AND -|          |

FLG_4X4L = 1 --------------------|                |

|

| --- ELSE ---

GR_CM > 1 ----------------------------------|     |

|     |

VSBART_RT_PD < [FN21S +                     |     |

HOT21_ADDR + (COMP_FCTR                   |     |

* FN21COMP) + (PNWF *                     |AND -| IGR_DS = 1

FN21P)] * CS_SFT_MULT ---------|          |     | FLG_UP_NE = 0

|AND -|    |     | FLG_NE_WAIT = 0

FLG_4X4L = 0 --------------------|     |    |     |

|    |     |

VSBART_RT_PD < [FN21S_4X4L +           |OR -|     |

HOT21_ADDR + (COMP_FCTR              |          |

* FN21COMP) + (PNWF *                |          |

FN21P)] * CS_SFT_MULT ---------|     |          |

|AND -|          |

FLG_4X4L = 1 --------------------|                |

| --- ELSE ---

|

GR_CM > 2 ----------------------------------|     |

|     |

VSBART_RT_PD < [FN32S +                     |     |

+ (COMP_FCTR * FN32COMP) +                |AND -| IGR_DS = 2

(PNWF * FN32P)] * CS_SFT_MULT -|          |     | FLG_UP_NE = 0

|AND -|    |     | FLG_NE_WAIT = 0

FLG_4X4L = 0 --------------------|     |    |     |

|    |     |

VSBART_RT_PD < [FN32S_4X4L +           |OR -|     |

+ (COMP_FCTR * FN32COMP) +           |          |

(PNWF * FN32P)] * CS_SFT_MULT -|     |          |

|AND -|          |

FLG_4X4L = 1 --------------------|                |

|

| --- ELSE ---

(continued on next page)

26-13

SHIFT CONTROL, GR_DS, PRNDL = 3 OR 4 LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

GR_CM > 3 ----------------------------------|     |

|     |

VSBART_RT_PD < [FN43S +                     |     |

+ (COMP_FCTR * FN43COMP) +                |AND -| IGR_DS = 3

(PNWF * FN43P)] * CS_SFT_MULT -|          |     | FLG_UP_NE = 0

|AND -|    |     | FLG_NE_WAIT = 0

FLG_4X4L = 0 --------------------|     |    |     |

|    |     |

VSBART_RT_PD < [FN43S_4X4L +           |    |     |

+ (COMP_FCTR * FN43COMP) +           |OR -|     |

(PNWF * FN43P)] * CS_SFT_MULT -|     |          |

|AND -|          |

FLG_4X4L = 1 --------------------|     |          |

|          |

PDL = 3 -------------------------------|          |

|

| --- ELSE ---

|

| IGR_DS = GR_CM

OUTPUT SHAFT SPEED SENSOR FAILURE AND FAIL SAFE COOLING SHIFT LOGIC

COOL_FLG = 1 ------------------------------| FSC_ADD_U = FNFSC_TPU(TP_REL)

|             * FNFSC_CYL

|               (COOL_PI_INT)

| FSC_ADD_D = FNFSC_TPD(TP_REL)

|             * FNFSC_CYL

|               (COOL_PI_INT)

|

| --- ELSE ---

|

| FSC_ADD_U = 0

| FSC_ADD_D = 0

NEBART > FN689U(TP_REL) + FSC_ADD_U -|

|AND -| IGR_DS = GR_CM + 1

GR_CM < 4 ---------------------------|     | FLG_UP_NE = 1

|

| --- ELSE ---

NEBART < FN689D(TP_REL) + FSC_ADD_D -|     |

|AND -| IGR_DS = GR_CM - 1

GR_CM > 1 ---------------------------|     | FLG_UP_NE = 1

|

| --- ELSE ---

|

| IGR_DS = GR_CM

IGR_DS = 4 --------------------------|

|AND -| IGR_DS = 3

PDL = 3 -----------------------------|     | FLG_UP_NE = 0

26-14

SHIFT CONTROL, DELAY/VERIFY SHIFT LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

26.3  DELAY/VERIFY SHIFT LOGIC (CDAJ0)

OVERVIEW

This logic delays the commanding of the desired gears a  calibratable  amount

of time to:

-  Allow for the TV pressure to ramp-up prior to commanding the  shift.   TV

is  increased  tremendously  during  a  shift.   It is necessary to start

commanding the extra TV  prior  to  the  shift  to  overcome  the  delays

associated with the time constant of the TV solenoid.

-  Allow  the  RPM  to  decrease  before  commanding  a   tip-out   upshift.

Commanding  an upshift immediately after a tip-out feels harsh, since the

clutches will be used to slow the engine down.  Waiting a period of  time

before commanding this shift, smoothes the shift significantly.

-  Verify a shift.  Noise perturbations may result in incorrect gears  being

desired  for  one  or two background loops.  Delaying the shift, verifies

that a gear is truly desired, and that it is not just noise.

Entering into this logic is the parameter, GR_DS, the desired gear.

The delay logic then proceeds to delay the shift by setting the GR_DS to  its

previous  value  until the delay timer runs out.  When the timer expires, the

GR_DS is no longer set to its previous value, but is allowed to pass  through

to  the  Commanded  Gear  Determination  logic and Shift Solenoid logic which

actually processes the GR_DS into shift solenoid commands; i.e., the shift is

commanded.

By the time the shift is commanded, the necessary TV required for  the  shift

will  have  been  commanded, the engine RPM will have slowed some to make the

tip-out upshifts feel smooth, and the shift will have been verified.

If the GR_DS changes prior to the timer  expiring,  the  timer  continues  to

count  down.   The timer does not affect the amount of TV which is commanded,

because GR_DS_TV will always be set to the gear in sequence, and the TV  will

be  commanded  based  on  that  value.   Worst  case, the TV pressure will be

unnecessarily increased for a short period  of  time  if  the  GR_DS/GR_DS_TV

fluctuates  due  to  a  noise spike.  When the timer becomes zero, the latest

GR_DS is passed through.

NOTES:

The delay timer is set to one of four values.  These are:

-  delay to allow TV to ramp up for an upshift, and/or the delay to verify a

gear; TMVEUP

-  delay to allow TV to ramp up for an upshift, and/or the delay to verify a

gear in 4X4L; TMVEUP_4X4L

-  delay to allow TV to ramp up for a downshift, and/or the delay to  verify

a gear; TMVEDN

26-15

SHIFT CONTROL, DELAY/VERIFY SHIFT LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  delay for a tip-out upshift to allow the RPM decrease, TMVETOUP

26-16

SHIFT CONTROL, DELAY/VERIFY SHIFT LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  GR_DS_LST = Desired gear in last background pass.

-  IGR_DS = Unverified desired transmission gear.

-  PDL = Current PRNDL position.

-  PDL_LST = Last PRNDL position.

-  TM_VER_SFT = Time to delay/verify automatic desired shift.

-  TP_RATE = Throttle rate = TP - TPBART.

Bit Flags:

-  FLG_4X4L = 4X4L Flag.

-  FLG_VE_DSGR = Delay desired gear first pass flag; 0 -> First pass through

DELAY DESIRED GEAR, 1 -> Delay desired gear in process.

Calibration Constants:

-  TMVEDN = Time to delay/verify a downshift.

-  TMVETOUP = Time to delay a tip-out upshift.

-  TMVEUP = Time to delay/verify an upshift.

-  TMVEUP_4X4L = Time to delay/verify an upshift in 4X4L.

-  TO_TP_RATE = TP_RATE required to recognize a tip-out.

OUTPUTS

Registers:

-  GR_DS = Desired transmission gear.

-  TM_VER_SFT = See above.

Bit Flags:

-  FLG_NV_SHFT = Flag indicating if shift verification failed; 0 -> did  not

fail verification, 1 -> failed verification.

-  FLG_VE_DSGR = See above.

-  FLG_TIP_OUT = Flag which indicates a tip-out upshift in progress; 0 -> no

tip-out upshift in progress, 1 -> a tip-out upshift in progress.

26-17

SHIFT CONTROL, DELAY/VERIFY SHIFT LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SC_VER_AUTO_SHFT_COM2

PDL = 3 OR 4 -----------------|

|AND -| GR_DS = IGR_DS

PDL_LST <> 3 OR 4 ------------|     |  (PDL moved to 3 or 4; allow first

|   desired gear to pass through

|   with no delay)

| FLG_VE_DSGR = 0

|

| --- ELSE ---

IGR_DS <> GR_DS_LST ----------|     |

(shift is desired)           |     |

|     |

FLG_VE_DSGR = 1 --------------|AND -| GR_DS = GR_DS_LST

(delay in process)           |     |  (hold original desired gear until new

|     |   desired gear is verified)

TM_VER_SFT > 0 ---------------|     |

(timer not expired)                |

| --- ELSE ---

IGR_DS <> GR_DS_LST ----------|     |

(shift is desired)           |     |

|     |

FLG_VE_DSGR = 1 --------------|AND -| GR_DS = IGR_DS

(delay in process)           |     |  (new desired gear is delayed, allow it

|     |   to pass through to commanded gear

TM_VER_SFT = 0 ---------------|     |   module)

(timer expired)                    | FLG_VE_DSGR = 0

|

| --- ELSE ---

IGR_DS > GR_DS_LST -----------|     |

(upshift is desired)         |AND -| GR_DS = GR_DS_LST

|     |  (hold original desired gear until new

FLG_VE_DSGR = 0 --------------|     |   desired gear is delayed)

(first pass through)               | FLG_VE_DSGR = 1

|  (set first pass flag)

| FLG_NV_SHFT = 0

|  (clear "failed verify" flag)

| DO "LOAD TM_VER_SFT FOR UPSHIFTS"

|  (load timer)

|

| --- ELSE ---

(continued on next page)

26-18

SHIFT CONTROL, DELAY/VERIFY SHIFT LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

IGR_DS < GR_DS_LST -----------|     |

(downshift is desired)       |AND -| GR_DS = GR_DS_LST

|     |  (hold original desired gear until

FLG_VE_DSGR = 0 --------------|     |   new desired gear is delayed)

(first pass through)               | FLG_VE_DSGR = 1

|  (set first pass flag)

| TM_VER_SFT = TMVEDN

|  (load timer)

| FLG_TIP_OUT = 0

|

| --- ELSE ---

IGR_DS = GR_DS_LST -----------|     |

|AND -| GR_DS = IGR_DS

FLG_VE_DSGR = 1 --------------|     |  (pass through current desired gear)

| FLG_VE_DSGR = 0

| FLG_NV_SHFT = 1

|  (set no shift verify, first pass flag)

|

| --- ELSE ---

|

| GR_DS = IGR_DS

|  (pass through current desired gear)

| FLG_VE_DSGR = 0

|  (clear first pass flag)

| FLG_NV_SHFT = 0

LOAD TM_VER_SFT FOR UPSHIFTS LOGIC

FLG_4X4L = 1 -----------------------| TM_VER_SFT = TMVEUP_4X4L

| FLG_TIP_OUT = 0

|

| --- ELSE ---

|

TP_RATE < TO_TP_RATE ---------------| TM_VER_SFT = TMVETOUP

| FLG_TIP_OUT = 1

|

| --- ELSE ---

|

| TM_VER_SFT = TMVEUP

| FLG_TIP_OUT = 0

26-19

SHIFT CONTROL, GR_SEQ_PNTR CALCULATION - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

26.4  GR_SEQ_PNTR CALCULATION (CDAA0)

OVERVIEW

The GR_SEQ_PNTR calculation allows for the sequencing of  multi-step  shifts.

For example, if GR_DS = 4 and GEAR_CUR = 1, the 1-4 shift can be accomplished

in four different ways:  a direct 1-4 shift, a 1-2-4 shift, a 1-3-4 shift  or

a  1-2-3-4 shift.  The particular shift sequence desired can be calibrated in

a series of calibration parameters.  This sequence is used by the TV strategy

to  allow  TV  pressure  to  build  up  prior  to the shift as well as by the

commanded gear routine.

The GR_SEQ_PNTR register contains an address, not the next gear.  The address

pointed  to by GR_SEQ_PNTR is one of the cal parameters GR_SEQ_131, etc.,etc.

Thus, the contents of the address pointed to by GR_SEQ_PNTR contains the next

gear to be used in the shift sequence.

26-20

SHIFT CONTROL, GR_SEQ_PNTR CALCULATION - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Desired Shift       | Possible choices   GR_SEQ_XXX

--------------------|----------------------------------------

1-3              |  2-3     3-3

|

1-4              |  2-3-4   3-4-4   2-4-4   4-4-4

|

2-4              |  3-4     4-4

|

3-1              |  2-1     1-1

|

4-1              |  3-2-1   3-1-1   2-1-1   1-1-1

|

4-2              |  3-2     2-2

GR_SEQ_PNTR        | Cal Parameter            | Possible

(pointer address)  | Description              | Choices (contents)

-------------------|--------------------------|--------------------

GR_SEQ_131        | first step of 1-3 shift  |  2 or 3

GR_SEQ_132        | second step of 1-3 shift |  3

-------------------|--------------------------|--------------------

GR_SEQ_141        | first step of 1-4 shift  |  2 or 3 or 4

GR_SEQ_142        | second step of 1-4 shift |  3 or 4

GR_SEQ_143        | third step of 1-4 shift  |  4

-------------------|--------------------------|--------------------

GR_SEQ_241        | first step of 2-4 shift  |  3 or 4

GR_SEQ_242        | second step of 2-4 shift |  4

-------------------|--------------------------|--------------------

GR_SEQ_311        | first step of 3-1 shift  |  2 or 1

GR_SEQ_312        | second step of 3-1 shift |  1

-------------------|--------------------------|--------------------

GR_SEQ_411        | first step of 4-1 shift  |  3 or 2 or 1

GR_SEQ_412        | second step of 4-1 shift |  2 or 1

GR_SEQ_413        | third step of 4-1 shift  |  1

-------------------|--------------------------|--------------------

GR_SEQ_421        | first step of 4-2 shift  |  3 or 2

GR_SEQ_422        | second step of 4-2 shift |  2

-------------------|--------------------------|--------------------

26-21

SHIFT CONTROL, GR_SEQ_PNTR CALCULATION - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  GEAR_CUR = Current transmission gear.

-  GR_CM = Commanded gear for shift solenoids.

-  GR_DS = Desired transmission gear.

-  GR_DS_LST = Desired gear in last background pass.

-  IGR_DS = Unverified desired transmission gear.

-  PDL = Current PRNDL position.

-  PDL_LST = Last PRNDL position.

Bit Flags:

-  FLG_SFT_IN = Shift in progress flag.

Calibration Constants:

-  GR_SEQ_XXX = See Overview section for descriptions.

OUTPUTS

Registers:

-  GR_SEQ_PNTR = Commanded gear sequence pointer.

-  GR_DS_TV = Gear to be used for dynamic TV calculation.

26-22

SHIFT CONTROL, GR_SEQ_PNTR CALCULATION - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SC_GR_SEQ_PNTR_COM4

FLG_SFT_IN = 0 --------------------| gr_ds_temp = IGR_DS

(no shift in progress)            |  (use unverified desired gear)

|

| --- ELSE ---

|

| gr_ds_temp = GR_DS

|  (use verified desired gear)

TM_SEQ_SFT > 0 --------------------| Exit GR_SEQ_PNTR module

|

| --- ELSE ---

|

gr_ds_temp = GEAR_CUR -------------| GR_DS_TV = GEAR_CUR

| GR_SEQ_PNTR = address of GR_DS_TV

|

| --- ELSE ---

|

gr_ds_temp - GEAR_CUR > 1 ---------| GR_SEQ_PNTR = address of cal

(upshift, multi step shift)       |  parameter  GR_SEQ_131 if 1-3 shift

|             GR_SEQ-141 if 1-4 shift

|             GR_SEQ_241 if 2-4 shift

|  (first step of a multi step shift)

| GR_DS_TV = contents of GR_SEQ_PNTR

|  (pass next gear on to TV routine)

|

| --- ELSE ---

|

gr_ds_temp - GEAR_CUR = 1 ---------| GR_DS_TV = GR_CM + 1

(upshift, single step shift)      | GR_SEQ_PNTR = address of GR_DS_TV

|

| --- ELSE ---

|

gr_ds_temp - GEAR_CUR < -1 --------| GR_SEQ_PNTR = address of cal

(downshift, multi step shift)     |  parameter  GR_SEQ_311 if 3-1 shift

|             GR_SEQ_411 if 4-1 shift

|             GR_SEQ_421 if 4-2 shift

|  (first step of a multi step shift)

| GR_DS_TV = contents of GR_SEQ_PNTR

|  (pass next gear on to TV routine)

|

| --- ELSE ---

|

gr_ds_temp - GEAR_CUR = -1 --------| GR_DS_TV = GR_CM - 1

(downshift, single step shift)    | GR_SEQ_PNTR = address of GR_DS_TV

26-23

SHIFT CONTROL, PRNDL BASED COMMANDED GEAR DETERMINATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

26.5  PRNDL BASED COMMANDED GEAR DETERMINATION (CDAN0)

OVERVIEW

The commanded transmission gear is calculated based on PRNDL  position.   The

logic  looks  at  the  current  gear  and the desired gear to determine if an

upshift or a downshift is required and then  commands  the  next  appropriate

gear in the sequence.

In the drive or overdrive position, the shift sequence is controlled  by  the

calibration  pointed to by the GR_SEQ_PNTR register.  The only restriction is

that downshifts are done only after the converter clutch has unlocked.

In the manual 2 or 1 position, the downshift sequence is controlled solely by

the  commanded  gear  routine.   The  GR_SEQ_PNTR  is  not  used  for  manual

downshifts.

The main outputs of the commanded gear routine are:

-  GR_CM, commanded gear which reflects the actual shift solenoid states

-  GR_OLD, GEAR_OLD, RT_GR_OLD, last gear the transmission was in

-  FLG_FRST_CM, global flag to indicate a shift is commanded this background

pass

DEFINITIONS

Registers:

-  GEAR_CUR = Current transmission gear (global register).

-  GEAR_OLD = Last commanded transmission gear (global register).

-  GR_CM = Commanded gear for shift solenoids.

-  GR_CM_KAM = KAM value equivalent of GR_CM for reinit protection.

-  GR_CM_LST = Commanded gear in last background pass.

-  GR_DS = Desired transmission gear.

-  GR_OLD = Last commanded gear.

-  PDL = Current PRNDL position.

-  RT_GR_CUR = Current transmission gear ratio.

-  RT_GR_OLD = Last gear transmission gear ratio.

26-24

SHIFT CONTROL, PRNDL BASED COMMANDED GEAR DETERMINATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  SLIP_ABS = Absolute value of slip.

-  TM_UNLK_WDOG = Watch dog time to  force  downshift  if  SLIP_ABS  is  not

greater than SLIP_A_UNLK.

-  TMR_41AUT = Timer to be loaded when leaving 4th gear to determine a  4-1,

sec.

-  TMR_42AUT = Timer to be loaded when leaving 4th gear to determine a  4-2,

sec.

-  TM_IN_GR4 = Time in 4th gear used to improve 4-2 sequence times.

Bit Flags:

-  FLG_AUT41 = Identifies a Automatic 4-1 pull-in.

-  FLG_AUT42 = Identifies a Automatic 4-2 pull-in.

-  FLG_FRST_CM = First time a shift is commanded flag; 1 -> shift  commanded

this background pass, 0 -> no shift commanded this background pass.

-  FLG_FRST_DS = First time a shift is desired flag; 0 -> no  shift  desired

this BG pass, 1 -> shift desired this BG pass.

-  FLG_MAN42 = Identifies a Manual 4-2 pull-in.

-  FLG_SF_AUTO = Automatic upshift/downshift  flag;  1  ->  automatic  shift

(PRNDL = 3 or 4), 0 -> manual shift (PRNDL = 2 or 1).

-  FLG_SFT_IN = Shift in progress flag.

-  LST_SFT_CMP = Last  shift  complete  flag;  1  ->  last  commanded  shift

completed (timed out).

-  FLG_UNLK_DS = Converter unlock in progress flag; 1 -> unlock in progress.

Calibration Constants:

-  GRMSFT = Gear commanded for manual shifting.

-  SLIP_A_UNLK = Slip value above which converter is  sufficiently  unlocked

to allow downshift.

-  SW_MSF = Switch to select GR_CM manually; 1 -> manual gear selection, NOT

1 -> automatic gear selection.

-  TMDNUN = Time for converter  clutch  to  unlock  prior  to  commanding  a

downshift.

-  TM41AUT = Time to determine if a 4-1 was initiated, sec.

-  TM42AUT = Time to determine if a 4-2 was initiated, sec.

26-25

SHIFT CONTROL, PRNDL BASED COMMANDED GEAR DETERMINATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SC_CM_GR_DETR_COM8

FLG_FRST_DS = 1 ------------------|

|

GR_DS < GR_CM --------------------|AND -| TM_UNLK_WDOG = TMDNUN

|     |  (maximum time to delay downshift

PDL = 3 OR 4 ---------------------|     |   due to low converter slip)

TM_UNLK_WDOG <> 0 ----------------|

|

SLIP_ABS < SLIP_A_UNLK -----------|AND -| FLG_UNLK_DS = 1

|     | (delay downshift; low converter

GR_DS < GR_CM --------------------|     |  slip)

|

| --- ELSE ---

|

| FLG_UNLK_DS = 0

| TM_UNLK_WDOG = 0

always ---------------------------------| GR_CM_LST = GR_CM

|  (update last pass current gear)

SW_MSF = 1 -----------------------------| GR_CM = GRMSFT

(manual shift selection)               |

| --- ELSE ---

|

PDL = 1 --------------------------------| Do: "GR_CM, PRNDL = 1" LOGIC

|

| --- ELSE ---

|

PDL = 2 --------------------------------| Do: "GR_CM, PRNDL = 2" LOGIC

|

| --- ELSE ---

PDL = 3 OR 4 ---------------------|     |

|AND -| Do: "GR_CM, PRNDL = 3 OR 4

GR_DS > GR_CM --------------------|     |      UPSHIFT" LOGIC

|

| --- ELSE ---

PDL = 3 OR 4 ---------------------|     |

|AND -| Do: "GR_CM, PRNDL = 3 OR 4

GR_DS < GR_CM --------------------|     |      DOWNSHIFT" LOGIC

|

| --- ELSE ---

PDL = 5 --------------------------|     |

(neutral)                        |     |

|     |

PDL = 6 --------------------------|OR --| GR_CM = 1

(reverse)                        |

|

PDL = 7 --------------------------|

(park)

26-26

SHIFT CONTROL, PRNDL BASED COMMANDED GEAR DETERMINATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

always ---------------------------------| Do: substitute(8,GR_CM)

|  (output state control override)

| GR_CM_KAM = GR_CM

GR_CM_LST <> GR_CM ---------------|

(a shift has been commanded)     |AND -| LST_SFT_CMP = 0

|     | ;Indicate last shift incomplete

FLG_SFT_IN = 1 -------------------|     |

(Last shift has not completed)         |

| --- ELSE ---

|

FLG_SFT_IN = 0 -------------------------| LST_SFT_CMP = 1

| ;current commanded shift has

| ;completed

GR_CM_LST <> GR_CM ---------------------| FLG_FRST_CM = 1

(a shift has been commanded)           |  (new commanded gear for

|   this program pass only)

| GR_OLD = GR_CM_LST

|  (update old gear)

| GEAR_OLD = GEAR_CUR

| RT_GR_OLD = RT_GR_CUR

|

|  --- ELSE ---

|

| FLG_FRST_CM = 0

|  (no shift this program pass)

FLG_FRST_CM = 1 ------------------|

|

PDL <> 3 OR 4 --------------|     |AND -| FLG_SF_AUTO = 0

|     |     |  (manual shift)

PDL = 3 --------------|     |OR --|     |

|     |           |

GR_OLD = 4 -----------|AND -|           |

|                 |

GR_CM = 3 ------------|                 |

| --- ELSE ---

|

FLG_FRST_CM = 1 ------------------------| FLG_SF_AUTO = 1

|  (automatic shift)

FLG_FRST_CM = 1 ------------|

|

PDL = 2 --------------------|AND -|

|     |

GR_CM_LST = 4 --------------|     |OR --| FLG_MAN42 = 1

|     | (Manual 4-2 pull-in flag)

FLG_MAN42 = 1 --------------|     |     |

|AND -|     |

FLG_SFT_IN = 1 -------------|           |

| --- ELSE ---

|

| FLG_MAN42 = 0

26-27

SHIFT CONTROL, PRNDL BASED COMMANDED GEAR DETERMINATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

GR_CM = 4 ------------------------|

|AND -| TMR_41AUT = 0

FLG_FRST_CM = 1 ------------------|     | TMR_42AUT = 0

| TM_IN_GR4 = 0

(reset time-in-4th timer)

PDL = 4 --------------------------|

|

GR_CM_LST = 4 --------------------|AND -| TMR_41AUT = TM41AUT

|     | TMR_42AUT = TM42AUT

FLG_FRST_CM = 1 ------------------|

FLG_AUT41 = 1 --------------------|

|AND -| FLG_AUT41 = 0

GR_DS > 1 ------------------------|     | (Backout of 4-1)

|

GR_DS = 1 ------------------|           | --- ELSE ---

|AND -|     |

TMR_41AUT <> 0 -------------|     |     |

|OR --| FLG_AUT41 = 1

FLG_AUT41 = 1 --------------|     |     | (Automatic 4-1 pull-in flag)

|AND -|     | FLG_AUT42 = 0

FLG_SFT_IN = 1 -------------|           |

| --- ELSE ---

|

| FLG_AUT41 = 0

FLG_AUT42 = 1 --------------------|

|AND -| FLG_AUT42 = 0

GR_DS > 2 ------------------------|     | FLG_AUT41 = 0

| (Backout of 4-2)

|

GR_DS = 2 ------------------|           | --- ELSE ---

|AND -|     |

TMR_42AUT <> 0 -------------|     |     |

|OR --| FLG_AUT42 = 1

FLG_AUT42 = 1 --------------|     |     | (Automatic 4-2 pull-in flag)

|AND -|     | FLG_AUT41 = 0

FLG_SFT_IN = 1 -------------|           |

| --- ELSE ---

|

| FLG_AUT42 = 0

26-28

SHIFT CONTROL, GR_CM, PRNDL = 1 LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

26.6  GR_CM, PRNDL = 1 LOGIC (CDAM0)

OVERVIEW

This logic determines the commanded gear when PRNDL = 1.

DEFINITIONS

Registers:

-  GR_CM = Current transmission gear.

-  GR_DS_TV = Gear to be used for dynamic TV calculation.

-  NEBART = Filtered engine speed.

-  PDL_LST = Manual lever position previous background pass.

-  RT_NOVS = Ratio of actual N/V to base N/V in KAM.

-  TMR_HS_M1 = Timer which determines when second may be forced.

-  TMR_M12 = Control timer for manual 1-2 shift.

-  TM_SEQ_SFT = Shift sequencing timer.

-  VSBART_RT = Filtered vehicle speed adjusted for RT_NOVS for transmission.

Bit Flags:

-  OSFMFLG = Output shaft speed sensor FMEM flag.

-  FLG_HS_M1 = Force second in manual one when high engine speed for a  long

period of time; 1 -> forcing second.

Calibration Constants:

-  GR_CM_LST = Commanded Gear Last Background Pass.

-  NEHSM1 = Engine speed above which second may be forced.

-  TM_HS_M12 = Time to command 3rd gr sol.  st.   on  Manual  1  high  speed

kickout upshift, seconds.

-  TMSQ21P1 = Minimum time in 2nd gear prior to 3 - 1 shift, PRNDL = 1.

-  TMHSM1 = Time that  engine  speed  must  stay  above  calibration  before

forcing second gear.

-  VSHSM1 = Vehicle speed below which forcing second may be exited.

-  VS21PI = Maximum vehicle speed for 2 - 1 pull in.

26-29

SHIFT CONTROL, GR_CM, PRNDL = 1 LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SC_CM_GR_MAN1_COM5

GR_CM = 1 -------------------|

|AND -| NO ACTION

NEBART > NEHSM1 -------------|     |

| --- ELSE ---

|

| TMR_HS_M1 = TMHSM1

TMR_HS_M1 = 0 ---------------------| FLG_HS_M1 = 1

|

| --- ELSE ---

|

VSBART_RT < VSHSM1 ----------------| FLG_HS_M1 = 0

FLG_HS_M1 = 1 ---------------------| GR_CM = 2

| GR_DS_TV = 2

|

| --- ELSE ---

|

GR_CM >= 3 ------------------------| GR_CM = 2

| TM_SEQ_SFT = TMSQ21P1

| GR_DS_TV = 2

|

| --- ELSE ---

GR_CM = 2 -------------|           |

|OR --|     |

PDL_LST = 5 -----------|     |     |

|     |

TM_SEQ_SFT > 0 --------|     |AND -| GR_CM = 2

|     |     | GR_DS_TV = 2

OSFMFLG = 1 -----------|     |     |

(failed output shaft  |OR --|     |

speed sensor)        |           |

|           |

VSBART_RT > VS21PI ----|           |

(above 1st gear pull-in)          |

| --- ELSE ---

|

| GR_CM = 1

| GR_DS_TV = 1

FLG_HS_M1 = 1 ---------------|

(Manual 1 kickout upshift)  |

|AND -| TMR_M12 = TM_HS_M12

GR_CM = 2 -------------------|     | (Load timer to control

|     |  commanding 3rd gear

GR_CM_LST = 1 ---------------|     |  shift solenoid states

|  before 2nd gear shift

|  solenoid states)

26-30

SHIFT CONTROL, GR_CM, PRNDL = 2 LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

26.7  GR_CM, PRNDL = 2 LOGIC (CDAG0)

OVERVIEW

This module determines the commanded gear when PRNDL = 2.

DEFINITIONS

INPUTS

Registers:

-  GR_CM = Current transmission gear.

-  VSBART_RT = Filtered vehicle speed adjusted for RT_NOVS for transmission.

-  PDL_LST = PRNDL Position Last Background Pass.

Calibration Constants:

-  SW_SEQ_MAN2 = Switch used to  allow  or  disallow  a  direct  4-2  manual

pull-in;  1  ->  sequence  through 3rd gear on a manaul 4-2 pull-in, 0 ->

direct 4-2 pull-in.

-  TMSQ32P2 = Minimum time in third gear prior to 3-2, PRNDL = 2, seconds.

-  VS32P2 = 3-2 VS pull-in when PDL = 2.

OUTPUTS

Registers:

-  GR_CM = See above.

-  GR_DS_TV = Gear to be used for dynamic TV calculation.

26-31

SHIFT CONTROL, GR_CM, PRNDL = 2 LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SC_CM_GR_MAN2_COM5

GR_CM = 4 ------------------------|

|AND -| GR_CM = 3

SW_SEQ_MAN2 = 1 ------------------|     | GR_DS_TV = 3

(sequence through 3rd gear)             | TM_SEQ_SFT = TMSQ32P2

| (trans in 2nd, no engine

|  braking, set timer)

|

| --- ELSE ---

GR_CM = 3 ------------------|           |

|           |

PDL_LST = 5 ----------------|OR --|     |

|     |     |

SW_SEQ_MAN2 = 0 ------------|     |     |

(direct 4-2; use vehicle          |AND -| GR_CM = 3

speed threshold only)            |     | GR_DS_TV = 3

|     |

TM_SEQ_SFT > 0 -------------|     |     |

|OR --|     |

VSBART_RT > VS32P2 ---------|           |

| --- ELSE ---

|

| GR_CM = 2

| GR_DS_TV = 2

26-32

SHIFT CONTROL, COMMANDED GEAR, AUTO UPSHIFT LOGIC - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

26.8  COMMANDED GEAR, AUTO UPSHIFT LOGIC (CDAJ0)

OVERVIEW

This module determines the commanded gear on an upshift when PRNDL = 3 or  4.

The   sequence   for  multi-step  shifts  is  calibratable  (see  GR_SEQ_PNTR

calculation).  The time spent in intermediate gears on a multi-step shift  is

calibrated in this module.

DEFINITIONS

INPUTS

Registers:

-  DELTA_RATIO = Delta Shift Ratio change since command of shift.

-  GR_CM = Commanded gear for shift solenoids.

-  GR_DS_TV = Gear to be used for dynamic TV calculation.

-  GR_SEQ_PNTR = Address of next gear in the shift sequence.

-  TM_DEL_SFT = Time to delay a shift to allow TV pressure to build,

-  TM_SEQ_SFT = Shift sequencing timer.

Bit Flags:

-  FLG_4X4L = 4X4L Flag.

-  FLG_DE_CMGR = First pass through shift delay timer.

-  FLG_FRST_DS = First time a shift is desired flag; 0 -> no  shift  desired

this BG pass, 1 -> shift desired this BG pass.

Calibration Constants:

-  DRC23 = DELTA_RATIO when 2-3 is judged complete.

-  DRC34 = DELTA_RATIO when 3-4 is judged complete.

-  TMDELUP = Time to delay upshift for TV buildup, sec.

-  TMSQ23P4 = Minimum time in second gear prior to 2-3 PRNDL = 3 or 4, sec.

-  TMSQ34P4 = Minimum time in third gear prior to 3-4 PRNDL = 3 or 4, sec.

26-33

SHIFT CONTROL, COMMANDED GEAR, AUTO UPSHIFT LOGIC - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OUTPUTS

Registers:

-  GR_CM = See above.

-  GR_DS_TV = See above.

-  TM_DEL_SFT = See above.

-  TM_SEQ_SFT = See above.

Bit Flags:

-  FLG_DE_CMGR = See above.

26-34

SHIFT CONTROL, COMMANDED GEAR, AUTO UPSHIFT LOGIC - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SC_CM_GR_AUTO_UP_COM7

always ------------------------------| If GR_SEQ_PNTR is not pointing at a

|  GR_SEQ_xxx term, then it is set to

|  the address of GR_DS_TV.

TM_SEQ_SFT > 0 ----------|

|AND -|

GR_CM < GR_OLD ----------|     |

(downshift)                    |

|

TM_SEQ_SFT > 0 ----------|     |

|     |

GR_CM = 2 ---------------|AND -|OR --| TM_SEQ_SFT = 0

|     |     |  (Override gear sequence

DELTA_RATIO > DRC23 -----|     |     |   timer. Ratio change has

|     |   been completed.)

TM_SEQ_SFT > 0 ----------|     |     |

|     |     |

GR_CM = 3 ---------------|AND -|     |

|           |

DELTA_RATIO > DRC34 -----|           |

| --- ELSE ---

FLG_DE_CMGR = 1 ---------|           |

|AND -|     |

TM_DEL_SFT > 0 ----------|     |OR --| no action

|     |

TM_SEQ_SFT > 0 ----------------|     |

| --- ELSE ---

FLG_DE_CMGR = 1 ---------|           |

|AND -|     |

TM_DEL_SFT = 0 ----------|     |     |

|     |

FLG_FRST_DS = 1 ---------------|OR --| FLG_DE_CMGR = 0

|     | GR_DS_TV = contents of address

TMDELUP = 0 -------------------|     |  pointed to by GR_SEQ_PNTR

(no delay desired)                  | GR_CM = GR_DS_TV

| Increment GR_SEQ_PNTR

| Do: "UPSHIFT DELAY" logic

|

| --- ELSE ---

|

FLG_DE_CMGR = 0 ---------------------| FLG_DE_CMGR = 1

| TM_DEL_SFT = TMDELUP

| GR_DS_TV = contents of address

|  pointed to by GR_SEQ_PNTR

|  (first pass through shift sequence;

|   don't command new gear yet, allow

|   TV pressure to ramp up)

26-35

SHIFT CONTROL, COMMANDED GEAR, AUTO UPSHIFT LOGIC - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

"UPSHIFT DELAY" LOGIC

FLG_4X4L = 1 ------------------------| TM_SEQ_SFT = 0

| (no sequence time required in

|  4X4L)

|

| --- ELSE ---

|

GR_CM = 2 ---------------------------| TM_SEQ_SFT = TMSQ23P4

(upshift to 2nd gear)               |

| --- ELSE ---

|

GR_CM = 3 ---------------------------| TM_SEQ_SFT = TMSQ34P4

(upshift to 3rd gear)               |

| --- ELSE ---

|

| TM_SEQ_SFT = 0

|  (no sequence time required)

26-36

SHIFT CONTROL, GR_CM, PRNDL = 3 OR 4, DOWNSHIFT LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

26.9  GR_CM, PRNDL = 3 OR 4, DOWNSHIFT LOGIC (CDAN0)

OVERVIEW

This module determines the commanded gear on an downshift when PRNDL =  3  or

4.   The  sequence  for  multi-step  shifts  is calibratable (see GR_SEQ_PNTR

calculation).  The time spent in intermediate gears on a multi-step shift  is

calibrated in this module.

DEFINITIONS

Registers:

-  DELTA_RATIO = Delta Shift Ratio change since command of shift.

-  GR_CM = Commanded gear for shift solenoids.

-  GR_DS_TV = Gear to be used for dynamic TV calculation.

-  GR_SEQ_PNTR = Address of next gear in the shift sequence.

-  SPD_RATIO = Speed ratio across the torque converter (output/input).

-  TM_DEL_SFT = Time to delay a shift to allow TV pressure to build.

-  TM_IN_GR4 = Time in 4th gear used to improve 4-2 sequence times, secs.

-  TM_SEQ_SFT = Shift sequencing timer.

-  VSBART_RT = Filtered vehicle speed, adjusted for N/V (mph).

Bit Flags:

-  FLG_DE_CMGR = First pass through shift delay timer.

-  FLG_FRST_DS = First time a shift is desired flag; 0 -> no  shift  desired

this BG pass, 1 -> shift desired this BG pass.

-  FLG_UNLK_DS = Converter unlock in progress flag.

Calibration Constants:

-  DRC21 = DELTA_RATIO when 2-1 is judged complete.

-  TMDELDN = Time to delay downshift to allow TV pressure to build.

-  TMSQ21P4 = Minimum time in 2nd gear prior to a 2-1 shift, PRNDL = 3 or 4.

-  FNTMSEQ32P4(FNXVSBART_RT,FNXTMINGR4) = Minimum time in 3rd gear prior  to

a  3-2  shift,  PRNDL = 3 or 4, as a function of normalized VSBART_RT and

TM_IN_GR4.  Time in 4th is used to reduce sequence times if time  in  4th

(forward clutch off) is less than a calibratable amount of time.

26-37

SHIFT CONTROL, GR_CM, PRNDL = 3 OR 4, DOWNSHIFT LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FNXTMINGR4 = Normalized time in 4th gear (TM_IN_GR4) used to improve  4-2

sequence times.

-  FNXVSBART_RT = Normalized filtered vehicle speed, VSBART_RT.

26-38

SHIFT CONTROL, GR_CM, PRNDL = 3 OR 4, DOWNSHIFT LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SC_CM_GR_AUTO_DWN_COM7

always ------------------------------| If GR_SEQ_PNTR is not pointing at a

|  GR_SEQ_xxx term, then it is set to

|  the address of GR_DS_TV.

TM_SEQ_SFT > 0 ----------|

|AND -|

GR_CM > GR_OLD ----------|     |

(upshift)                      |

|OR --|TM_SEQ_SFT = 0

TM_SEQ_SFT > 0 ----------|     |     | (Override gear sequence

|     |     |  timer. Ratio change has

GR_CM = 2 ---------------|AND -|     |  been completed).

|           |

DELTA_RATIO > DRC21 -----|           |

| --- ELSE ---

FLG_DE_CMGR = 1 ---------|           |

|AND -|     |

TM_DEL_SFT > 0 ----------|     |     |

|     |

TM_SEQ_SFT > 0 ----------------|OR --| no action

|     |

FLG_UNLK_DS = 1 ---------------|     |

| --- ELSE ---

FLG_DE_CMGR = 1 ---------|           |

|AND -|     |

TM_DEL_SFT = 0 ----------|     |     |

|OR --| FLG_DE_CMGR = 0

FLG_FRST_DS = 1 ---------------|     | GR_DS_TV = contents of address

|     |  pointed to by GR_SEQ_PNTR

TMDELDN = 0 -------------------|     | GR_CM = GR_DS_TV

(no delay desired)                  | Increment GR_SEQ_PNTR

| Do: "DOWNSHIFT DELAY" logic

|

| --- ELSE ---

|

FLG_DE_CMGR = 0 ---------------------| FLG_DE_CMGR = 1

| TM_DEL_SFT = TMDELDN

| GR_DS_TV = contents of address

|  pointed to by GR_SEQ_PNTR

|  (first pass through shift sequence;

|   don't command new gear yet, allow

|   TV pressure to ramp up)

26-39

SHIFT CONTROL, GR_CM, PRNDL = 3 OR 4, DOWNSHIFT LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

"DOWNSHIFT DELAY" LOGIC

GR_CM = 3 ---------------------------| TM_SEQ_SFT = FNTMSEQ32P4(FNXVSBART_RT,

(downshift to 3rd gear)             |                FNXTMINGR4)

|

| --- ELSE ---

|

GR_CM = 2 ---------------------------| TM_SEQ_SFT = TMSQ21P4

(downshift to 2nd gear)             |

| --- ELSE ---

|

| TM_SEQ_SFT = 0

|  (no sequence time required)

26-40

SHIFT CONTROL, LOAD SHIFT IN PROGRESS TIMER - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

26.10  LOAD SHIFT IN PROGRESS TIMER (CDAA0)

OVERVIEW

The shift in progress timer (TM_SFT_IN) is a  down-counting  timer  which  is

used  in  many  places in the strategy to determine that a shift is currently

taking place.  When the timer has a value of 0, no shift is taking place.   A

worst  case  time is loaded into the timer at the start of a shift.  The only

exception is a power-off manual  downshift.   Different  default  values  are

provided for upshifts, downshifts, power on, and power off.

DEFINITIONS

INPUTS

Registers:

-  GR_CM = Commanded gear for shift solenoids.

-  GR_OLD = Last commanded gear.

-  TM_SFT_IN = Time during which shift is in progress.

-  TM_VER_SFT = Time to delay/verify automatic desired shift.

Bit Flags:

-  FLG_FRST_CM = First time a shift is commanded flag:  1 -> shift commanded

this background pass; 0 -> no shift commanded this background pass.

-  FLG_PWR = Power mode flag:  1 -> power on mode; 0 -> power off mode.

-  FLG_SF_AUTO = Automatic upshift/downshift flag:   1  ->  automatic  shift

(PRNDL = 3 or 4); 0 -> manual shift (PRNDL = 2 or 1).

-  FLG_SFT_MDN = Power off manual downshift flag:  1  ->  power  off  manual

downshift in progress; 0 -> power off manual downshift not in progress.

26-41

SHIFT CONTROL, LOAD SHIFT IN PROGRESS TIMER - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  TCD43AOF = Time to complete 4/3 downshift, automatic, power off.

-  TCD43ON = Time to complete 4/3 downshift, power on.

-  TCDNAF = Time to complete auto downshift, power off (all except 4/3).

-  TCDNMF = Time to complete manual downshift, power off.

-  TCDNON = Time to complete downshift, power on (all except 4/3).

-  TCUPOF = Time to complete upshift, power off.

-  TCUPON = Time to complete upshift, power on.

OUTPUTS

Registers:

-  TM_SFT_IN = See above.

Bit Flags:

-  FLG_SF_AUTO = See above.

-  FLG_SFT_IN = Shift in progress flag; 0 -> no  shift  in  progress,  1  ->

shift in progress.

-  FLG_SFT_MDN = See above.

-  FLG_TIP_OUT = 0 -> no tip-out upshift in progress; 1 -> a tip-out upshift

in progress.

-  FLG_UP_NE = RPM based upshift flag.

26-42

SHIFT CONTROL, LOAD SHIFT IN PROGRESS TIMER - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SC_TIMER_COM2

FLG_FRST_CM = 1 ------------------|

(shift has been commanded)       |

|

GR_CM > GR_OLD -------------------|AND -| TM_SFT_IN = TCUPON

(upshift)                        |     |  (time to complete upshift,

|     |   power on)

FLG_PWR = 1 ----------------------|     | FLG_SFT_MDN = 0

(power on)                             |  (not power off manual downshift)

|

| --- ELSE ---

FLG_FRST_CM = 1 ------------------|     |

(shift has been commanded)       |     |

|     |

GR_CM > GR_OLD -------------------|AND -| TM_SFT_IN = TCUPOF

(upshift)                        |     |  (time to complete

|     |   upshift, power off)

FLG_PWR = 0 ----------------------|     | FLG_SFT_MDN = 0

(power off)                            |

| --- ELSE ---

FLG_FRST_CM = 1 ------------------|     |

(shift has been commanded)       |     |

|     |

GR_OLD = 4 -----------------------|AND -| TM_SFT_IN = TCD43ON

|     |  (time to complete 4/3 downshift,

GR_CM = 3 ------------------------|     |   power on)

|     | FLG_SFT_MDN = 0

FLG_PWR = 1 ----------------------|     |

(power on)                             |

| --- ELSE ---

FLG_FRST_CM = 1 ------------------|     |

(shift has been commanded)       |     |

|     |

GR_OLD = 4 -----------------------|     |

|     |

GR_CM = 3 ------------------------|AND -| TM_SFT_IN = TCD43AOF

|     |  (time to complete 4/3 downshift,

FLG_PWR = 0 ----------------------|     |   auto, power off)

(power off)                      |     | FLG_SFT_MDN = 0

|     |

FLG_SF_AUTO = 1 ------------------|     |

(automatic shift)                      |

| --- ELSE ---

FLG_FRST_CM = 1 ------------------|     |

(shift has been commanded)       |     |

|     |

GR_CM < GR_OLD -------------------|AND -| TM_SFT_IN = TCDNON

(downshift)                      |     |  (time to complete downshift,

|     |   power on)

FLG_PWR = 1 ----------------------|     | FLG_SFT_MDN = 0

(power on)                             |

| --- ELSE ---

(continued on next page)

26-43

SHIFT CONTROL, LOAD SHIFT IN PROGRESS TIMER - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

FLG_FRST_CM = 1 ------------------|     |

(shift has been commanded)       |     |

|     |

GR_CM < GR_OLD -------------------|     |

(downshift)                      |     |

|AND -| TM_SFT_IN = TCDNAF

FLG_PWR = 0 ----------------------|     |  (time to complete downshift,

(power off)                      |     |   auto, power off)

|     | FLG_SFT_MDN = 0

FLG_SF_AUTO = 1 ------------------|     |

(automatic shift)                      |

| --- ELSE ---

FLG_FRST_CM = 1 ------------------|     |

(shift has been commanded)       |     |

|     |

GR_CM < GR_OLD -------------------|     |

(downshift)                      |     | TM_SFT_IN = TCDNMF

|AND -|  (time to complete downshift,

FLG_PWR = 0 ----------------------|     |   manual, power off)

(power off)                      |     | FLG_SFT_MDN = 1

|     |  (manual power off downshift

FLG_SF_AUTO = 0 ------------------|     |   in progress)

(manual shift)                         |

|

| --- ELSE ---

FLG_SFT_MDN = 1 ------------------|     |

(manual power off downshift      |     |

in progress)                    |AND -| TM_SFT_IN = TCDNON

|     |  (reset timer to power on value

FLG_PWR = 1 ----------------------|     |   if power mode changes in the

(suddenly becomes power on)            |   middle of a downshift)

| FLG_SFT_MDN = 0

|  (clear manual downshift flag)

TM_SFT_IN = 0 --------------------------| FLG_SFT_MDN = 0

(timer expired)                        |  (manual downshift is complete)

| FLG_SF_AUTO = 0

|  (clear auto shift flag)

| FLG_SFT_IN = 0

|  (no shift in progress)

| FLG_UP_NE = 0

|

| --- ELSE ---

|

| FLG_SFT_IN = 1

|  (shift in progress)

TM_SFT_IN = 0 --------------------|

(no shift in progress)           |AND -| FLG_TIP_OUT = 0

|     |  (reset tip-out flag)

TM_VER_SFT = 0 -------------------|

(no delay shift in progress)

26-44

SHIFT CONTROL, DETERMINE SHIFT SOLENOID STATES - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

26.11  DETERMINE SHIFT SOLENOID STATES (CDAL0)

OVERVIEW

The shift solenoid state logic configures the shift  solenoid  output  states

based  on  the  PRNDL  position  and  commanded  gear  during  normal vehicle

operation, or the  state  of  the  Engine  Off  Output  State  Monitor  flags

(EO_OSM_ON/OFF) during Engine Off On-Demand self test.

The main outputs of the shift solenoid routine, besides  the  shift  solenoid

states,  are  RT_GR_CUR,  current  transmission  gear  ratio,  and  the shift

solenoid monitor and malfunction flags.

Shift solenoid state monitoring is done using feedback from  the  ML-II  EEEC

driver.   When  the  solenoid states are output in background the driver will

return a fault status:

SSn_FAULT = 0 --> no fault present

SSn_FAULT = 1 --> fault present

NOTE:  The fault status lags commanded state by 1 background - ie is for  the

last  command.  When a solenoid state change is commanded, the shift solenoid

monitors will be disabled for one background to allow  the  fault  status  to

update to the new state.

The shift solenoid monitor flags will be set when both the ON and OFF  states

of  that  solenoid  have  been  observed to function OR when a malfunction is

detected.

The shift solenoid malfunction flags are set when a  fault  is  present  long

enough to be defined as a malfunction.

DEFINITIONS

Registers:

-  CYCCTR = Cold shift solenoid cycling counter.

-  GEAR_CUR = Current transmission gear (global register).

-  GR_CM = Commanded gear for shift solenoids.

-  PDL = Current PRNDL position.

-  RT_GR_CUR = Current transmission gear ratio.

-  SS1_ERR_TMR = Shift Solenoid #1 test error timer.

-  SS1_FAULT = Fault status for SS1 returned from the driver; 0 -> no  fault

present, 1 -> a fault has been detected.

26-45

SHIFT CONTROL, DETERMINE SHIFT SOLENOID STATES - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  SS2_ERR_TMR = Shift Solenoid #2 test error timer.

-  SS2_FAULT = Fault status for SS2 returned from the driver; 0 -> no  fault

present, 1 -> a fault has been detected.

-  TMR_M12 = Timer which controls commanding 3rd gear solenoid states  after

a manual 1-2 shift.

Bit Flags:

-  CCM_TST_ENA = Comprehensive Component Enable Test Flag.

-  FLG_SS_1 = Shift solenoid 1 output state; 1 -> SS1 energized,  0  ->  SS1

de-energized.

-  FLG_SS_2 = Shift solenoid 2 output state; 1 -> SS2 energized,  0  ->  SS2

de-energized.

-  OSM_EO_OFF = Engine Off Output State Monitor off test; 1 -> OSM Off  test

in progress - command actuator off and monitor for off state.

-  OSM_EO_ON = Engine Off Output State Monitor on test; 1 -> OSM On test  in

progress - command actuator on and monitor for on state.

-  PxxxMALF = OBDII malfunction flag for code xxx.

-  PxxxMON = OBDII monitor flag for code XXX.

-  FLG_FRST_TV = Start-up TV pressure flag; 0 -> do start-up TV logic, 1  ->

do not do start-up TV logic.

-  FLG_TVENG_CD = Flag which indicates cold temperature for engagement TV; 0

-> don't use TVEMAX in engagement TV; 1 -> use TVEMAX in engagement TV.

-  FLG_TVENG_MD = Flag which indicates moderate temperature  for  engagement

TV; 0 -> don't use TVEMOD in engagement TV; 1 -> use TVEMOD in engagement

TV.

-  SS1_COM = Shift solenoid #1 command flag; 1 -> solenoid #1 commanded  on,

0 -> solenoid #1 commanded off.

-  SS1_COM_LST = Last verified position of solenoid #1.

-  SS1_DES = Desired state of shift solenoid #1.

-  SS1_MON_OFF = Shift Solenoid #1 monitor off flag.

-  SS1_MON_ON = Shift Solenoid #1 monitor on flag.

-  SS2_COM = Shift solenoid #2 command flag; 1 -> solenoid #2 commanded  on,

0 -> solenoid #2 commanded off.

-  SS2_COM_LST = Last verified position of solenoid #2.

-  SS2_DES = Desired state of shift solenoid #2.

-  SS2_MON_OFF = Shift Solenoid #2 monitor off flag.

26-46

SHIFT CONTROL, DETERMINE SHIFT SOLENOID STATES - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  SS2_MON_ON = Shift Solenoid #2 monitor on flag.

-  SUBST_REQ11 = Substitute requested flag for channel 11.

-  SUBST_REQ12 = Substitute requested flag for channel 12.

-  UNDSP = Run/Underspeed flag; 1 -> Underspeed or Crank, 0 -> Run.

Calibration Constants:

-  CLDCTM = Cold shift solenoid cycle time in background loops.

-  FNSSCTL2 = Table of shift solenoid values, based on GR_CM and PDL.

-  GRRAT1 = First gear ratio.

-  GRRAT2 = Second gear ratio.

-  GRRAT3 = Third gear ratio.

-  GRRAT4 = Fourth gear ratio.

-  SS1_ERROR_TM = Shift Solenoid #1 time, minimum time error must be present

to set a code.

-  SS2_ERROR_TM = Shift Solenoid #2 time, minimum time error must be present

to set a code.

-  SSCYCCLD = Shift solenoid states for cold cycling.

-  TSTRAT  =  Transmission  strategy  switch;  The  TSTRAT  software  switch

controls which transmission control strategy is executed:

0 -> No transmission control (Manual trans., AOD, ATX, C6, C3, etc.)

1 -> SIL (Shift indicator light)

2 -> A4LD with 3-4 shift control and converter clutch control

3 -> AXOD

4 -> C64E (E4OD)

5 -> A4LDE

6 -> FAX-4

7 -> AOD-E

8 -> 4EAT

9 -> CD4E

OTHER

-  SS1_CODES = Set of (P0750) SS1 OBDII fault code.

-  P0750 = Fault code for shift solenoid #1

-  SS2_CODES = Set of (P0755) SS1 OBDII fault code.

-  P0755 = Fault code for shift solenoid #2

26-47

SHIFT CONTROL, DETERMINE SHIFT SOLENOID STATES - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SC_SOL_CTL_COM18

TSTRAT <= 3 ----------------------|

(Non-electronic trans)           |AND -| Do: OSC_RESPSONE(11,10h)

|     |  (send general reject)

SUBST_REQ11 = 1 ------------------|

TSTRAT <= 3 ----------------------|

(Non-electronic trans)           |AND -| Do: OSC_RESPSONE(12,10h)

|     |  (send general reject)

SUBST_REQ12 = 1 ------------------|

TSTRAT <= 3 ----------------------------| Exit module

(Non-electronic trans)                 |

| --- ELSE ---

|

| Increment CYCCTR

CYCCTR >= CLDCTM -----------------------| CYCCTR = 0

FLG_FRST_TV = 0 ------------------|

(no engagement yet)              |

|

CYCCTR > CLDCTM/2 ----------------|

(cycle shift solenoids)          |

|

FLG_TVENG_MD = 1 -----------|     |

|OR --|AND -| SS1_DES = bit 0 of SSCYCCLD

FLG_TVENG_CD = 1 -----------|     |     | SS2_DES = bit 1 of SSCYCCLD

(moderately cold)                |     | RT_GR_CUR = GRRAT1

|     | GEAR_CUR = 1

UNDSP = 0 ------------------------|     |  (do cold shift solenoid

(RUN mode)                       |     |   cycling strategy)

|     |

GR_CM = 1 ------------------------|     |

| --- ELSE ---

TMR_M12 > 0 ----------------------|     |

(man. 1-2 just started)          |AND -| SS1_DES = bit 0 of FNSSCTL2(2, 1)

|     | SS2_DES = bit 1 of FNSSCTL2(2, 1)

GR_CM = 2 ------------------------|     |  (command third gear, PDL = 2

|   solenoid states)

|

| --- ELSE ---

|

| SS1_DES = bit 0 of FNSSCTL2

|               (GR_CM - 1, PDL - 1)

| SS2_DES = bit 1 of FNSSCTL2

|               (GR_CM - 1, PDL - 1)

| RT_GR_CUR = GRRAT[GR_CM]

| GEAR_CUR = GR_CM

26-48

SHIFT CONTROL, DETERMINE SHIFT SOLENOID STATES - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

This is the calibration for the AODE transmission.

Cal. Const.          Binary          Decimal

SSCYCCLD             0010               2

FNSSCTL2             PDL - 1

0           1           2            3          4,5,6

GR_CM - 1 = 0    0001 (1)    0001 (1)    0001 (1)     0001 (1)    0001 (1)

1    0000 (0)    0000 (0)    0000 (0)     0000 (0)    0000 (0)

2    0010 (2)    0010 (2)    0010 (2)     0010 (2)    0010 (2)

3    0011 (3)    0011 (3)    0011 (3)     0011 (3)    0011 (3)

NOTE: FNSSCTL2(commanded gear, PRNDL position) is indexed using GR_CM - 1,

PDL - 1 to save bytes because the first row and column of a table

must both be zero.

OSM_EO_OFF = 1 -------------------------| SS1_COM = 0

(Engine Off OSM off testing            | SS2_COM = 0

in progress)                          | (Turn all solenoids off)

|

| --- ELSE ---

|

OSM_EO_ON = 1 --------------------------| SS1_COM = 1

(Engine Off OSM on testing             | SS2_COM = 1

in progress)                          | (Turn all solenoids on)

|

| --- ELSE ---

|

| SS1_COM = SS1_DES

| SS2_COM = SS2_DES

| (Pass through desired state)

unconditionally ------------------------| flg_ss_1 = SS1_COM

| Do: substitute(11,flg_ss_1)

| FLG_SS_1 = flg_ss_1

|  (output state control override)

| SS1_COM_LST = FLG_SS_1

|

| flg_ss_2 = SS2_COM

| Do: substitute(12,flg_ss_2)

| FLG_SS_2 = flg_ss_2

|  (output state control override)

| SS2_COM_LST = FLG_SS_2

26-49

SHIFT CONTROL, DETERMINE SHIFT SOLENOID STATES - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OSM_EO_OFF = 1 -------------------|

(EO OFF self test enabled)       |

|

OSM_EO_ON = 1 --------------------|OR --| Do: ss1_mon

(EO ON self test enabled)        |     | Do: ss2_mon

|     |

CCM_TST_ENA = 1 ------------------|     |

(CCM testing enabled)                  |

| --- ELSE ---

|

| SS1_ERR_TMR = 0

| SS2_ERR_TMR = 0

| EXIT

| (CCM testing disabled,

|  hold error timers at zero

|  and exit)

BEGIN: ss1_mon

Monitor Shift Solenoid 1 using feedback from the driver:

FLG_SS_1 <> SS1_COM_LST ----------------| SS1_ERR_TMR = 0

(Solenoid state change)                | (Wait 1 background

|  for fault status to

|  update to new state)

|

| --- ELSE ---

FLG_SS_1 = 1 ---------------------|     |

(SS1 commanded on)               |AND -| SS1_MON_ON = 1

|     | SS1_ERR_TMR = 0

SS1_FAULT = 0 --------------------|     | clear_malf(P0750)

(no fault present)                     |  (On state commanded and observed,

|  set SS1 monitored on flag, clear

|  malf flag, and zero error timer)

|

| --- ELSE ---

FLG_SS_1 = 0 ---------------------|     |

(SS1 commanded off)              |AND -| SS1_MON_OFF = 1

|     | SS1_ERR_TMR = 0

SS1_FAULT = 0 --------------------|     | clear_malf(P0750)

(no fault present)                     |  (Off state commanded and observed,

|  set SS1 monitored off flag, clear

|  malf flag, and zero error timer)

|

| --- ELSE ---

|

SS1_ERR_TMR >= SS1_ERROR_TM ------------| malfunction(ccm,P0750)

|  (error present long enough,

|   set SS1 error code)

26-50

SHIFT CONTROL, DETERMINE SHIFT SOLENOID STATES - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

P0750MALF = 1 -------------|

(SS1 malfunction          |

currently present)       |OR --| P0750MON = 1

|     | (SS1 has been monitored

SS1_MON_ON = 1 ------|     |     |  for this power-up)

|AND -|

SS1_MON_OFF = 1 -----|

(Both states have

been observed)

END: ss1_mon

BEGIN: ss2_mon

Monitor Shift Solenoid 2 using feedback from the driver:

FLG_SS_2 <> SS2_COM_LST ----------------| SS2_ERR_TMR = 0

(Solenoid state change)                | (Wait 1 background

|  for fault status to

|  update to new state)

|

| --- ELSE ---

FLG_SS_2 = 1 ---------------------|     |

(SS2 commanded on)               |AND -| SS2_MON_ON = 1

|     | SS2_ERR_TMR = 0

SS2_FAULT = 0 --------------------|     | clear_malf(P0755)

(no fault present)                     |  (On state commanded and observed,

|  set SS2 monitored on flag, clear

|  malf flag, and zero error timer)

|

| --- ELSE ---

FLG_SS_2 = 0 ---------------------|     |

(SS2 commanded off)              |AND -| SS2_MON_OFF = 1

|     | SS2_ERR_TMR = 0

SS2_FAULT = 0 --------------------|     | clear_malf(P0755)

(no fault present)                     |  (Off state commanded and observed,

|  set SS2 monitored off flag, clear

|  malf flag, and zero error timer)

|

| --- ELSE ---

|

SS2_ERR_TMR >= SS2_ERROR_TM ------------| malfunction(ccm,P0755)

|  (error present long enough,

|   set SS2 error code)

P0755MALF = 1 --------------------|

(SS2 malfunction currently       |

present)                        |OR --| P0755MON = 1

|     | (SS2 has been monitored

SS2_MON_ON = 1 -------------|     |     |  for this power-up)

|AND -|

SS2_MON_OFF = 1 ------------|

(Both states have

been observed)

END: ss2_mon

26-51

SHIFT CONTROL, SHIFT VALIDATION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

26.12  SHIFT VALIDATION LOGIC (CDAN0)

OVERVIEW

The Shift Validation logic verifies that a shift has taken place after it  is

commanded.   This  logic is only capable of verifying shifts which take place

under steady operating conditions.  The shifts must  be  automatic  upshifts,

with  the  vehicle off idle and the vehicle speed over a calibratable minimum

speed for each gear (designed to ensure that the vehicle is power on and  the

torque converter is operating above the coupling point).

The logic works as follows:  if a power on  upshift  is  commanded,  and  the

vehicle speed entry condition is met, then TP, VS, and NE are recorded at the

start of the shift.  The engine rpm drop that can be expected as a result  of

the  upshift  is  then determined based on the current state of the converter

clutch - a larger drop can be expected if the  converter  is  locked  at  the

start  (and  end)  of the shift.  The shift is considered to be a shift which

can be validated as long as  the  TP  and  VS  are  steady.   Engine  rpm  is

monitored  until  it  peaks.   Engine rpm must then drop from the peak by the

expected amount within the time allowed to complete the shift.  If the engine

rpm  drop lasts for a specified amount of time (verification the drop was due

to a shift and not due to noise), it is considered to represent a shift.   If

engine rpm did not drop within the allowed time, it is noted that there was a

shift error (missed shift).

The results of shift validation are used by  the  shift  solenoid  functional

test,  which  decodes  the missed shifts into the shift solenoid status.  For

any solenoid failure at least two (of three) single  step  upshifts  will  be

"missed".   It  has  been discovered that for certain solenoid failures it is

only possible to detect one of the two missed  shifts  (due  to  TCC  control

causing  an  RPM  drop  during  the shift - which looks the same as the shift

occurring).

Detecting both missed shift events for a  solenoid  failure  is  required  to

comply  with OBD-II.  Logic has been added to revise TCC control in the event

that one shift error code is stored, which will allow the other  shift  error

to  be dectected (if present).  Shift validation will pass a shift error flag

(SFT_ERR_FLG) to the converter clutch control strategy.  This  flag  will  be

set  when a shift error code is stored, and will remain set for the remainder

of the current power-up.   Each  subsequent  power-up  the  failure  must  be

re-detected before this failure mode action will be initiated.

Additional fmem action of clipping EPC to TVFMMN and flashing the  TCIL  will

begin  when  a  shift  error  code  is  stored,  and  will  be executed for a

calibratable number of warm up cycles.

NOTE: If  the  converter  clutch  is  applied at the beginning  of  shift

validation,  and  unlocks for  any reason during  shift validation,

shift validaiton  will be exited.  Unlocking  the converter  during

an upshift  may result in  an increase  in engine  speed which  may

negate the engine speed drop due to the shift occurring -> tricking

the test into calling a  "good" shift "missed".

26-52

SHIFT CONTROL, SHIFT VALIDATION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  GR_CM = Current commanded gear.

-  GEAR_OLD = Last commanded gear.

-  IPDL = Unfiltered MLPS position.

-  NE_DROP_EX = Expected engine rpm drop for the shift being validated.

-  NE_DROP_TM = Current time (in milliseconds) when the  engine  speed  drop

occurred during a verifiable shift.

-  NEBART = Filtered engine speed.

-  NEV_PEAK_SFT = Peak engine speed during a verifiable shift.

-  P0781CNT = 1-2 shift error warm up counter.

-  P0781FIL = 1-2 missed shift fault filter.

-  P0782CNT = 2-3 shift error warm up counter.

-  P0782FIL = 2-3 missed shift fault filter.

-  P0783CNT = 3-4 shift error warm up counter.

-  P0783FIL = 3-4 missed shift fault filter.

-  SFT_12_CNT = Number of 1-2 shifts validated to be good.

-  SFT_23_CNT = Number of 2-3 shifts validated to be good.

-  SFT_34_CNT = Number of 3-4 shifts validated to be good.

-  TOT = Transmission Oil Temperature, deg F.

-  TP_REL = Relative throttle position; TP - RATCH.

-  TPV_STRT_SFT = Throttle position at start of the shift.

-  TSTRAT  =  Transmission  strategy  switch;  The  TSTRAT  software  switch

controls which transmission control strategy is executed:

0 -> No transmission control (Manual trans., AOD, ATX, C6, C3, etc.)

1 -> SIL (Shift indicator light)

2 -> A4LD with 3-4 shift control and converter clutch control

3 -> AXOD

4 -> C64E (E4OD)

5 -> A4LDE

6 -> FAX-4

7 -> AOD-E (AOD-I)

8 -> 4EAT

9 -> CD4E

26-53

SHIFT CONTROL, SHIFT VALIDATION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  VSBART_RT = Filtered vehicle speed adjusted for RT_NOVS for transmission,

mph.

-  VSV_STRT_SFT = Vehicle speed at start of the shift.

Bit Flags:

-  CCM_TST_ENA = Comprehensive component  monitor  enable  flag;  1  ->  the

diagnostic  executive has decided that conditions are such that ccm tests

can be executed.

-  CCV_STRT_SFT = Converter clutch state at the start of shift validation; 1

-> TCC applied at start of shift.

-  FLG_FRST_CM = New commanded gear this pass flag.

-  FLG_NOV_KAM = Flag indicating at least one update of RT_NOVS_KAM.

-  FLG_OPEN = Convertor clutch state flag; 1 -> CC open, 0 -> CC closed.

-  FLG_PWR = Power mode flag ; 1 -> power on mode, 0 -> power off mode.

-  FLG_SF_AUTO = Automatic shift flag.

-  FLG_SFT_IN = Shift in progress flag; 1 -> shift in progress.

-  FLG_SFT_VAL = Shift validity flag; 0 -> Shift cannot be  verified,  1  ->

Shift may be verified.

-  FLG_SFT_VER = Shift verify flag; 0 -> Shift is not being verified,  1  ->

Shift is being verified.

-  FLG_SS_MON = Flag indicating all shifts have been monitored and are good,

used  to  enable/disable  converter clutch functional checks; 1 -> shifts

good, enable converter clutch tests.

-  LST_SFT_CMP = Last  shift  complete  flag;  1  ->  last  commanded  shift

completed (timed out).

-  OSFMFLG = Output Shaft speed sensor FMEM flag; 1 ->  output  shaft  speed

sensor failure detected.

-  P0781FAULT = 1-2 shift error detected.

-  P0782FAULT = 2-3 shift error detected.

-  P0783FAULT = 3-4 shift error detected.

-  P0781MON = 1-2 shift monitor flag; 1 -> at least one 1-2 shift  has  been

validated.

-  P0782MON = 2-3 shift monitor flag; 1 -> at least one 2-3 shift  has  been

validated.

-  P0783MON = 3-4 shift monitor flag; 1 -> at least one 3-4 shift  has  been

validated.

26-54

SHIFT CONTROL, SHIFT VALIDATION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TFMFLG = Throttle position FMEM flag;  1  ->  throttle  position  failure

detected.

Calibration Constants:

-  GOOD_12_LVL = Number of 1-2 shifts which must be validated  good  to  set

the shift solenoid monitor flag.

-  GOOD_23_LVL = Number of 2-3 shifts which must be validated  good  to  set

the shift solenoid monitor flag.

-  GOOD_34_LVL = Number of 3-4 shifts which must be validated  good  to  set

the shift solenoid monitor flag.

-  NE_DROP_OPEN = Epected engine rpm drop for a  power  on  upshift  started

with an open converter.

-  NE_DROP_LCK = Epected engine rpm drop for a power on upshift started with

a locked converter.

-  S_VAL_TPADD = Tolerance on TP to verify steady  TP  (positive  direction)

during the shift validation.

-  S_VAL_TPSUB = Tolerance on TP to verify steady  TP  (negative  direction)

during the shift validation.

-  S_VAL_VSADD = Tolerance on VS to verify steady  vehicle  speed  (positive

direction) during the shift validation.

-  S_VAL_VSSUB = Tolerance on VS to verify steady  vehicle  speed  (negative

direction) during the shift validation.

-  SFT_ERR_FLG = Shift error flag, used to revise TCC control; 1 -> a  shift

error has been detected.

-  SFT_ERR_LVL = Shift error level, the fault threshold for shift errors.

-  SFT_ERR_UP = Shift error upcount, the amount PxxxFIL will be increased by

when a shift error is detected.

-  SFT_FM_FLG = Shift error failure mode flag; 0 -> no failure mode  action,

1 -> shift error failure mode action will be executed.

-  SFT_FM_LVL = Total number of warm up cycles that failure mode action will

be executed after a shift error is detected.

-  SFT_VAL_TM = Length of time to verify a shift (in milliseconds).

-  TOT_SFTV_MX = Maximum TOT to allow shifts to be validated.

-  TOT_SFTV_MN = Minimum TOT to allow shifts to be validated.

-  TP_SH_VAL2 = Minimum TP to validate an upshift to second gear.

-  TP_SH_VAL3 = Minimum TP to validate an upshift to third gear.

-  TP_SH_VAL4 = Minimum TP to validate an upshift to fourth gear.

26-55

SHIFT CONTROL, SHIFT VALIDATION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  VS_SH_VAL2 = Minimum vehicle speed to validate an upshift to 2nd gear.

-  VS_SH_VAL3 = Minimum vehicle speed to validate an upshift to 3rd gear.

-  VS_SH_VAL4 = Minimum vehicle speed to validate an upshift to 4th gear.

OTHER

-  sft_codes = Set of {P0781,P0782,P0783} OBDII fault codes that  relate  to

shift errors.

-  malfunction(ccm,Pxxx) = Logic process,  imported  from  the  MIL  control

module, Pxxx indicates a fault code.

-  P0781 = Fault code for a 1-2 shift malfunction.

-  P0782 = Fault code for a 2-3 shift malfunction.

-  P0783 = Fault code for a 3-4 shift malfunction.

26-56

SHIFT CONTROL, SHIFT VALIDATION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SC_VALID_OBDII_COM1

TSTRAT <= 3 --------------------------------| Exit module

(Manual/non-electric transmission)         |

| --- ELSE ---

CCM_TST_ENA = 1 ----------------------|     |

|     |

FLG_FRST_CM = 1 ----------------------|     |

|     |

FLG_SF_AUTO = 1 ----------------------|     |

|     |

LST_SFT_CMP = 1 ----------------------|     |

|     |

IPDL <> 0 ----------------------------|     |

|     |

TOT <= TOT_SFTV_MX -------------------|     |

|     |

TOT > TOT_SFTV_MN --------------------|     |

|     |

GR_CM = GEAR_OLD + 1 -----------------|     |

|     |

GR_CM = 2 ----------------|           |     |

|           |     |

VSBART_RT > VS_SH_VAL2 ---|AND -|     |     |

|     |     |     |

TP_REL > TP_SH_VAL2 ------|     |     |     |

|     |AND -| Do: expected_ne_drop

GR_CM = 3 ----------------|     |     |     |  (set expected engine rpm

|     |     |     |   drop based on CC state)

VSBART_RT > VS_SH_VAL3 ---|AND -|OR --|     | TPV_STRT_SFT = TP_REL

|     |     |     |  (record TP at start

TP_REL > TP_SH_VAL3 ------|     |     |     |   of shift)

|     |     | NEV_PEAK_SFT = NEBART

GR_CM = 4 ----------------|     |     |     |  (record filtered engine

|     |     |     |   speed at start of shift)

VSBART_RT > VS_SH_VAL4 ---|AND -|     |     | VSV_STRT_SFT = VSBART_RT

|           |     |  (record filtered vehicle

TP_REL > TP_SH_VAL4 ------|           |     |   speed at start of shift)

|     | FLG_SFT_VAL = 1

FLG_NOV_KAM = 1 ----------------------|     |  (shift may be checked

|     |   for validity)

FLG_PWR = 1 --------------------------|     | FLG_SFT_VER = 0

(power on mode)                      |     |  (clear shift verify flag)

|     | Exit module

TFMFLG = 0 ---------------------------|     |

|     |

OSFMFLG = 0 --------------------------|     |

| --- ELSE ---

|

(continued on next page)

26-57

SHIFT CONTROL, SHIFT VALIDATION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|

FLG_SFT_VAL = 0 ----------------|           |

|AND -|     |

FLG_SFT_VER = 0 ----------------|     |     |

|     |

CCV_STRT_SFT = 1 ---------------|     |     |

|AND -|     |

FLG_OPEN = 1 -------------------|     |     |

;TCC locked at start, open now       |     |

|     |

VSBART_RT < VSV_STRT_SFT -            |     |

S_VAL_VSSUB ----------|     |

|     |

VSBART_RT > VSV_STRT_SFT +            |     |

S_VAL_VSADD ----------|OR --| FLG_SFT_VAL = 0

(unsteady vehicle speed)             |     | FLG_SFT_VER = 0

|     | Exit module

TP_REL < TPV_STRT_SFT - S_VAL_TPSUB --|     |

|     |

TP_REL > TPV_STRT_SFT + S_VAL_TPADD --|     |

(unsteady throttle)                  |     |

|     |

FLG_FRST_CM = 1 ----------------------|     |

;shift which does not meet entry     |     |

;conditions commanded                |     |

|     |

CCM_TST_ENA = 0 ----------------------|     |

| --- ELSE ---

|

| Do: shift_validation

BEGIN: expected_ne_drop

Determine the engine rpm drop that can be expected for  the  current  upshift

based  on  the  state  of  the  converter clutch at the start of the shift (a

larger drop can be expected for a locked to locked shift).

FLG_OPEN = 1 -------------------------------| NE_DROP_EX = NE_DROP_OPEN

(Conv. Cl. open at start of shift)         | (Set expected engine rpm drop

|  to the open to open/locked

|  upshift value)

|

| CCV_STRT_SFT = 0

| ;TCC open at start of shift

|

| --- ELSE ---

|

| NE_DROP_EX = NE_DROP_LCK

| (Set expected engine rpm

|  drop to the locked to locked

|  upshift value)

|

| CCV_STRT_SFT = 1

| ;CTCC applied at start of shift

END: expected_ne_drop

26-58

SHIFT CONTROL, SHIFT VALIDATION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: shift_validation

FLG_SFT_IN = 0 -----------------------------| error_detected = 1

(shift completed)                          | FLG_SFT_VAL = 0

| FLG_SFT_VER = 0

| Do: shift_gear_command_logic

| Do: shift_fmem_logic

| FLG_SS_MON = 0

| (Disable converter clutch tests

|  if a shift error is detected)

|

| --- ELSE ---

NEBART > NEV_PEAK_SFT ----------------|     |

(check for peak engine speed)        |AND -| NEV_PEAK_SFT = NEBART

|     | (store new peak rpm)

FLG_SFT_VAL = 1 ----------------------|     |

| --- ELSE ---

FLG_SFT_VAL = 1 ----------------------|     |

(shift can be checked for validity)  |AND -| FLG_SFT_VER = 1

|     |  (set shift verify flag)

NEBART <= NEV_PEAK_SFT - NE_DROP_EX --|     | FLG_SFT_VAL = 0

(engine speed drop from peak               |  (clear shift validate

has occurred)                             |   enable flag)

| NE_DROP_TM = store current

|               time in

|               milliseconds

|

| --- ELSE ---

FLG_SFT_VER = 1 ----------------------|     |

|     |

NEBART <= NEV_PEAK_SFT - NE_DROP_EX --|AND -| (shift verified as having

|     |  occurred, downcount the

current time - NE_DROP_TM >=          |     |  fault filter for that shift)

SFT_VAL_TM ----|     | FLG_SFT_VER = 0

(shift validation timer                    |  (clear shift verify flag)

has run out)                             | error_detected = 0

| Do: shift_gear_command_logic

| Do: ss_mon_logic

| Do: shift_fmem_logic

|

| --- ELSE ---

FLG_SFT_VER = 1 ----------------------|     |

|AND -| FLG_SFT_VER = 0

NEBART > NEV_PEAK_SFT - NE_DROP_EX ---|     | FLG_SFT_VAL = 1

| (drop in rpm did not last

|  long enough, re-enter

|  validation)

END: shift_validation

26-59

SHIFT CONTROL, SHIFT VALIDATION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: shift_gear_command_logic

This logic is used to distinguish between a missed shift and a valid shift to

either upcount or downcount the fault filter.

GR_CM = 2 --------------------|

|AND -| P0781FIL = P0781FIL + SFT_ERR_UP

error_detected = 1 -----------|     |  (missed 1-2 shift)

| Do: shift_fault_logic

|

| P0781MON = 1

|

| --- ELSE ---

|

GR_CM = 2 --------------------------| P0781FIL = P0781FIL - 1

|  (1-2 shift occurred)

|

| P0781MON = 1

| SFT_12_CNT = SFT_12_CNT + 1

|  (Upcount good 1-2 shift counter)

|

| --- ELSE ---

GR_CM = 3 --------------------|     |

|AND -| P0782FIL = P0782FIL + SFT_ERR_UP

error_detected = 1 -----------|     |  (missed 2-3 shift)

| Do: shift_fault_logic

|

| P0782MON = 1

|

| --- ELSE ---

|

GR_CM = 3 --------------------------| P0782FIL = P0782FIL - 1

|  (2-3 shift occurred)

|

| P0782MON = 1

| SFT_23_CNT = SFT_23_CNT + 1

|  (Upcount good 2-3 shift counter)

|

| --- ELSE ---

GR_CM = 4 --------------------|     |

|AND -| P0783FIL = P0783FIL + SFT_ERR_UP

error_detected = 1 -----------|     |  (missed 3-4 shift)

| Do: shift_fault_logic

|

| P0783MON = 1

|

| --- ELSE ---

|

GR_CM = 4 --------------------------| P0783FIL = P0783FIL - 1

|  (3-4 shift occurred)

|

| P0783MON = 1

| SFT_34_CNT = SFT_34_CNT + 1

|  (Upcount good 3-4 shift counter)

END: shift_gear_command_logic

26-60

SHIFT CONTROL, SHIFT VALIDATION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: shift_fault_logic

This logic is designed to set the appropriate fault flag and KAM code in  the

event  that  a  fault is detected.  This logic also sets the shift error flag

(SFT_ERR_FLG), which is used in the TCC control strategy to disable  softlock

and prevent the TCC from changing states during upshifts.

GR_CM = 2 ----------------------------|

|AND -| store_code(P0781)

P0781FIL >= SFT_ERR_LVL --------------|     | SFT_ERR_FLG = 1

|

| --- ELSE ---

GR_CM = 3 ----------------------------|     |

|AND -| store_code(P0782)

P0782FIL >= SFT_ERR_LVL --------------|     | SFT_ERR_FLG = 1

|

| --- ELSE ---

GR_CM = 4 ----------------------------|     |

|AND -| store_code(P0783)

P0783FIL >= SFT_ERR_LVL --------------|     |SFT_ERR_FLG = 1

END: shift_fault_logic

BEGIN: ss_mon_logic

When a "good" shift is completed, this logic is called to determine if enough

"good" shifts (without bad shifts being detected) have been validated to

allow the converter clutch functional tests to run:

SFT_12_CNT >= GOOD_12_LVL -----------|

(enough good 1-2 shifts)            |

|

P0781FIL = 0 ------------------------|

(No bad 1-2 shifts)                 |

|

SFT_23_CNT >= GOOD_23_LVL -----------|

(enough good 2-3 shifts)            |AND -| FLG_SS_MON = 1

|     |  (Enough "good" shifts

P0782FIL = 0 ------------------------|     |   have been seen to allow

(No bad 2-3 shifts)                 |     |   the converter clutch

|     |   functional tests to run)

SFT_34_CNT >= GOOD_34_LVL -----------|     |

(enough good 3-4 shifts)            |     |

|     |

P0783FIL = 0 ------------------------|     |

(No bad 3-4 shifts)                       |

| --- ELSE ---

|

| FLG_SS_MON = 0

|  (Either "missed" shifts have

|   occurred or not enough "good"

|   shifts to reliably run the

|   converter clutch tests)

END ss_mon_logic

26-61

SHIFT CONTROL, SHIFT VALIDATION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: shift_fmem_logic

This logic sets the failure mode flag so the appropriate failure mode  action

can be taken.

P0781FAULT = 1 -----------------|

(1-2 shift error detected)     |AND -|

|     |

P0781CNT < SFT_FM_LVL ----------|     |

(not enough warm up cycles           |

have passed since failure           |

detection)                          |

|

P0782FAULT = 1 -----------------|     |

(2-3 shift error detected)     |AND -|

|     |OR --| SFT_FM_FLG = 1

P0782CNT < SFT_FM_LVL ----------|     |     |  (a shift error has been

(not enough warm up cycles           |     |   detected, perform failure

have passed since failure           |     |   mode action for SFT_FM_LVL

detection)                          |     |   warm up cycles since the

|     |   last failure detection)

P0783FAULT = 1 -----------------|     |     |

(3-4 shift error detected)     |AND -|     |

|           |

P0783CNT < SFT_FM_LVL ----------|           |

(not enough warm up cycles have            |

passed since failure detection)           |

| --- ELSE ---

|

| SFT_FM_FLG = 0

END: shift_fmem_logic

26-62

SHIFT CONTROL, TORQUE MODULATION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

26.13  TORQUE MODULATION CONTROL (WITHOUT TURBINE SPEED) (CDAN0)

OVERVIEW

Torque Modulation is performed by retarding spark  during  the  shift  event.

For  non  turbine  speed sensor applications, torque modulation occurs during

upshifts only.

Minimum TP_REL and and steady throttle conditions must be met in order to  do

torqe  modulation.   In  addition, if trigger conditions are not met within a

certain time (TQ_TIMEOUT), torque modulation is aborted.

The desired  torque  level  during  the  shift  is  calculated  by  taking  a

percentage  of  the current engine output torque.  The spark control strategy

uses  this  information  to  adjust  spark  (to  achieve  this  torque  level

(TQ_LEVEL).

In foreground, the maximum engine speed during the shift, in clock ticks,  is

used  to  trigger the start of torque modulation.  The max engine speed value

is then used to calculate a speed ratio.  value for the end trigger.  This is

tagged as the speed ratio at the start of the inertia phase (SPD_RT_MAX).  To

end torque modulation, a calibratible  delta  speed  ratio  from  SPD_RT_MAX,

covertered to clock ticks, is used.

DEFINITIONS

Registers:

-  DT12S_AVG = Filtered PIP delta for transmission, clock ticks.

-  GR_CM = Commanded gear for shift solenoids.

-  GR_OLD = Last commanded gear.

-  NEBART = Filtered engine RPM for transmission.

-  NEMAX = Maximum engine speed during an upshift.

-  NE_SFT_END = Engine speed value to end torque modulation, base  on  delta

speed ratio, clock ticks.

-  NE_SFT_STRT = Maximum engine speed detected at start of shift, ticks.

-  NE_TRIG_STRT = Delta engine speed to trigger start of torque  modulation,

clock ticks.

-  NOBART = Filtered output shaft speed, RPM.

-  PCT_TQ = Percent of  TQ_NET  to  control  to  during  torque  modulation,

percent.

-  RT_GR_CUR = Current transmission gear ratio, dimensionless.

26-63

SHIFT CONTROL, TORQUE MODULATION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  SPD_RT_MAX = Speed ratio across torque converter at start of shift.

-  SR_TRIG_END = Delta speed ratio to  trigger  end  of  torque  modulation,

dimensionless.

-  TP_REL = Relative TP ( TP - RATCH ).

-  TP_REL_H = Relative TP, high byte.

-  TP_STRT_SFT = TP_REL_H at the start of the shift, counts.

-  TQ_LEVEL = Torque level required by transmission for  torque  modulation,

ft-lbs.

-  TQ_MIN_CLIP = Minimum TQ_NET below which torque modulation does not  take

place.

-  TQMOD_TMR = Torque Modulation default timer (msec)

-  TQ_NET = Net torque produced by the engine, ft-lbs.

Bit Flags:

-  FLG_EPC_RMP = Flag to trigger the start of static EPC ramps at the end of

a s shift.

-  FLG_FRST_CM = First time a shift is commanded flag; 0 -> no.

-  FLG_FRST_SPK = Flag which indicates max engine  speed  has  been  reached

during torque modulation.

-  FLG_SFT_IN = Shift in progress flag.

-  FLG_SPK_TQM = Torque modulation spark retard flag.

-  FLG_TQM_BGS = Torque modulation TV pressure flag.

-  FLG_UP_TQM = Upshift torque modulation flag.

Calibration Constants:

-  FNPCTQ12(TP_REL_H) = Percent of TQ_NET to control to during  1-2  upshift

torque modulation, percent.

-  FNPCTQ23(TP_REL_H) = Percent of TQ_NET to control to during  2-3  upshift

torque modulation, percent.

-  FNPCTQ34(TP_REL_H) = Percent of TQ_NET to control to during  3-4  upshift

torque modulation, percent.

-  FNTQ2SRE(TP_REL_H) = Delta Shift Ratio to end  torque  modulation  on  an

upshift to 2nd.

-  FNTQ3SRE(TP_REL_H) = Delta Shift Ratio to end  torque  modulation  on  an

upshift to 3rd.

-  FNTQ4SRE(TP_REL_H) = Delta Shift Ratio to end  torque  modulation  on  an

upshift to 4th.

26-64

SHIFT CONTROL, TORQUE MODULATION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  PUL_PER_REV = Pulses per revolution number of PIPs per engine  revolution

for E4OD_GAS 1/2 the number of fuel pump.

-  TP_DELTL = Lower limit on the delta counts that TP_REL can move in  order

to continue to do torque modulation, counts.

-  TP_DELTU = Upper limit on the delta counts that TP_REL can move in  order

to continue to do torque modulation, counts.

-  TP_MIN_TQM = Minimum TP_REL to do torque modulation, counts.

-  TQMIN12 = Minimum TQ_NET below which  torque  modulation  does  not  take

place during a 1-2 upshift.

-  TQMIN23 = Minimum TQ_NET below which  torque  modulation  does  not  take

place during a 2-3 upshift.

-  TQMIN34 = Minimum TQ_NET below which  torque  modulation  does  not  take

place during a 3-4 upshift.

-  TQ2DTS = Delta ticks to start torque modulation on an upshift to 2nd.

-  TQ3DTS = Delta ticks to start torque modulation on an upshift to 3rd.

-  TQ4DTS = Delta ticks to start torque modulaiton on an upshift to 4th.

-  TQ_TIMEOUT = Maximum time  to  look  for  starting  triggers  for  torque

modulation (seconds).

26-65

SHIFT CONTROL, TORQUE MODULATION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SC_TQMOD_COM9

BEGIN:  MAIN

FLG_FRST_CM = 1 ----------------|

|

GR_CM > GR_OLD -----------------|AND -| FLG_SPK_TQM = 0

|     | FLG_UP_TQM = 1

TP_REL_H > TP_MIN_TQM ----------|     | FLG_FRST_SPK = 0

| FLG_EPC_RMP = 0

| TQMOD_TMR = 0

| (reset torque modulation

|  timer)

| NE_SFT_STRT = DT12S_AVG

| Do: "UPSHIFT TORQUE MODULATION

|      SET-UP"

|

| --- ELSE ---

FLG_FRST_CM = 1 ----------------|     |

|AND -| FLG_SPK_TQM = 0

GR_CM <= GR_OLD  ---------------|     | FLG_UP_TQM = 0

| FLG_FRST_SPK = 0

| FLG_EPC_RMP = 0

| EXIT MODULE

|

| --- ELSE ---

FLG_UP_TQM = 1 -----------------|     |

|AND -| Do: "UPSHIFT TORQUE MODULATION

FLG_SPK_TQM = 0 ----------------|     |      SET-UP"

FLG_UP_TQM = 1 -----|

|AND -|

FLG_SPK_TQM = 0 ----|     |

(torque modulation has   |

not started)            |

|

TQMOD_TMR >               |AND -|

TQ_TIMEOUT ---|     |     |

|     |     |

TP_REL_H >          |     |     |

TP_STRT_SFT +    |     |     |

TP_DELTU ---|OR --|     |OR --| FLG_SPK_TQM = 0

|           |     | FLG_UP_TQM = 0

TP_REL_H <          |           |     | FLG_FRST_SPK = 0

TP_STRT_SFT -    |           |     | FLG_EPC_RMP = 0

TP_DELTL ---|           |     | (Reset torque modulation

|     |   flags)

FLG_SFT_IN = 0 -----------------|     | EXIT MODULE

|

| --- ELSE ---

|

FLG_UP_TQM = 0 -----------------------| EXIT MODULE

| (Torque modulation not requested;

|  skip the rest of the module)

26-66

SHIFT CONTROL, TORQUE MODULATION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLG_FRST_SPK = 0 ---------------------| NEMAX = 60 / (PUL_PER_REV *

|  NE_SFT_STRT * tscf)

|

| SPD_RT_MAX = NTBART / NEMAX

|

| --- ELSE ---

|

| Do not update NEMAX or SPD_RT_MAX

FLG_SPK_TQM = 1 ----------------------| NE_SFT_END = (SPD_RT_MAX +

|   SR_TRIG_END) * 60 * STCF /

|   (NOBART * RT_GR_CUR * PUL_PER_REV)

|

| --- ELSE ---

|

| NE_SFT_END = 65535

|  (set NE_SFT_END to its maximum

|   value to avoid false triggers)

FLG_SPK_TQM = 1 ----------------|

|AND -| FLG_FRST_SPK = 1

FLG_FRST_SPK = 0 ---------------|     | (Don't update NEMAX or SPD_RT_MAX)

END: MAIN

26-67

SHIFT CONTROL, TORQUE MODULATION CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: UPSHIFT TORQUE MODULATION SET-UP

GR_CM = 2 ----------------------------| PCT_TQ = FNPCTQ12(TP_REL_H)

| NE_TRIG_STRT = TQ2DTS

| SR_TRIG_END = FNTQ2SRE(TP_REL)

| TQ_LEVEL = PCT_TQ * TQ_NET

| TQ_MIN_CLIP = TQMIN12

|

| --- ELSE ---

|

GR_CM = 3 ----------------------------| PCT_TQ = FNPCTQ23(TP_REL_H)

| NE_TRIG_STRT = TQ3DTS

| SR_TRIG_END = FNTQ3SRE(TP_REL)

| TQ_LEVEL = PCT_TQ * TQ_NET

| TQ_MIN_CLIP = TQMIN23

|

| --- ELSE ---

|

| PCT_TQ = FNPCTQ34(TP_REL_H)

| NE_TRIG_STRT = TQ4DTS

| SR_TRIG_END = FNTQ4SRE(TP_REL)

| TQ_LEVEL = PCT_TQ * TQ_NET

| TQ_MIN_CLIP = TQMIN34

TQ_LEVEL < TQ_MIN_CLIP ---------------| TQ_LEVEL = TQ_MIN_CLIP

END:  UPSHIFT TORQUE MODULATION SET-UP

26-68

SHIFT CONTROL, TORQUE MODULATION FOREGROUND CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

26.14  TORQUE MODULATION FOREGROUND CONTROL (WITHOUT TURBINE SPEED) (CDAN0)

OVERVIEW

Torque modulation is triggered in foreground.  The start trigger is based  on

the  maximum  engine  speed during the shift plus a calibratible delta engine

speed.  The max engine speed (NICT_SFT_STRT)  is  found  by  ratcheting  down

DT12S_AVG,  the  PIP period in clock ticks, until the minimum value is found.

The trigger is then calculated  by  adding  the  delta  engine  speed  to  it

(NICT_SFT_STRT  +  NICT_TRIG_STRT).   Torque modulation begins when DT12S_AVG

reaches this value.

In background, the max engine speed is used to calculate a speed ratio.  This

speed  ratio  is  tagged as the speed ratio at the start of the inertia phase

(SPD_RT_MAX).  To end torque modulation, a  calibratible  delta  speed  ratio

from SPD_RT_MAX, covertered to clock ticks, is used.

The default timer (TQMOD_TMR) will time out torque modulation,  in  case  the

end trigger criteria is not met.

When torque modulation ends, and flag is set to initalize a static  EPC  ramp

(FLG_RMP_EPC).

DEFINITIONS

Registers:

-  DT12S_AVG = Filtered PIP delta for transmission, clock ticks.

-  NE_SFT_END = Engine speed value to end torque modulation, base  on  delta

speed ratio, clock ticks.

-  NE_SFT_STRT = Maximum engine speed detected  at  start  of  shift,  clock

ticks.

-  NE_TRIG_STRT = Delta engine speed to trigger start of torque  modulation,

clock ticks.

-  RAMP_TM_IN = Default time for spark ramp at  end  of  torque  modulation,

msec.

-  RAMP_TM_OUT = Default time for spark ramp at start of torque  modulation,

msec.

-  SAF_DES = Spark advance required to  achieve  the  desired  torque  level

during torque modulation, deg.

-  SAF_MOD = Maximum total spark advance during torque modulation, deg.

-  SAF_MOD_LST = Last pass value of SAF_MOD, deg.

-  SAFTOT = Total spark advance, deg.

26-69

SHIFT CONTROL, TORQUE MODULATION FOREGROUND CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  SPK_CLK_LST = Last pass value of CLOCK; used to compute spark  ramps  for

toruqe modulation, msec.

-  SPK_DELT_OUT = Delta spark stepped out during torque modulation, deg.

-  SPK_RMP_TMR = Timer used to control  the  duration  of  spark  ramps  for

torque modulation, msec.

-  SPKSLP = Spark ramp rate for torque modulation, deg/msec.

-  TQMOD_TMR = Torque Modulation default timer (msec)

Bit Flags:

-  FLG_EPC_RMP =  Indicates  end  of  ratio  change,  begin  ramping  static

capacity.

-  FLG_REP_ENA = Indicates 16 msec repeater is enabled for spark  ramps  for

torque modulation.

-  FLG_RMP_INIT = Indicates the first pass spark  retard  is  requested  for

torque modulation; initialize ramp parameters.

-  FLG_SPK_TQM = Torque modulation spark retard flag.

-  FLG_UP_TQM = Upshift torque modulation flag.

Calibration Constants:

-  TQMODTM = Maximum duration for torque modulation (seconds).

26-70

SHIFT CONTROL, TORQUE MODULATION FOREGROUND CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SC_TQMOD_FG_COM1

(Performed every PIP)

FLG_UP_TQM = 1 -----------------|

|

FLG_SPK_TQM = 0 ----------------|AND -| NE_SFT_STRT = DT12S_AVG

|     |  (rachet up input shaft speed

DT12S_AVG < NE_SFT_STRT --------|     |   to capture max speed during

|   the shift)

FLG_UP_TQM = 1 -----------------|

|

FLG_SPK_TQM = 0 ----------------|AND -| FLG_SPK_TQM = 1

|     | TQMOD_TMR = 0

DT12S_AVG > NE_SFT_STRT +       |     | (Reset Default Timer)

NE_TRIG_STRT ----|     | SPK_RMP_TMR = RAMP_TM_OUT

| FLG_RMP_INIT = 1

|   (Initialize spark ramp)

|

| --- ELSE ---

FLG_UP_TQM = 1 -----------------|     |

|     |

FLG_SPK_TQM = 1 ----------------|AND -| FLG_SPK_TQM = 0

|     | FLG_UP_TQM = 0

DT12S_AVG > NE_SFT_END ---|     |     | SPK_RMP_TMR = RAMP_TM_IN

|OR --|     | FLG_EPC_RMP = 1

TQMOD_TMR > TQMODTM ------|           | (ramp static capacity)

(Default timer expires)

FLG_RMP_INIT = 1 ----------------------| SPK_DELT_OUT := SPKSLP * (16 msec )

|  ; assume one 16 msec interval to

|  ; initialize ramp

| SAF_MOD := SAFTOT - SPK_DELT_OUT

| SAF_MOD := max( SAF_MOD, SAF_DES )

| SAF_MOD_LST := SAF_MOD

| SPK_CLK_LST := current CLOCK time i

|                in milliseconds

| FLG_RMP_INIT := 0

| FLG_REP_ENA := 1

|  ; enable 16 msec repeater for

|  ; spark ramps

26-71

SHIFT CONTROL, REVERSE LOCK-OUT SOLENOID CONTROL - CDAN2

PED-PTE FoMoCo, PROPRIETARY & CONFIDENTIAL

26.15  REVERSE LOCK-OUT SOLENOID CONTROL (CDAI0)

OVERVIEW

The reverse lock-out solenoid,  RLS,  is  used  in  manual  transmissions  to

prohibit  shifting  into  reverse  gear  at  vehicle  speeds  greater  than a

calibration.  This feature is intended to prevent catastrophic failure of the

T56 transmission.  A calibratable hardware present switch is available.

DEFINITIONS

Registers:

-  VSBART = Vehicle speed.

Bit Flags

-  FLG_RLS = Reverse lock-out flag; 1  ->  Reverse  allowed,  0  ->  Reverse

locked-out.

Calibration Constants:

-  FLG_RLS_HP = Reverse lock-out solenoid  hardware  present  switch;  0  ->

Reverse  lock-out  solenoid  not  present, 1 -> Reverse lock-out solenoid

present.

-  TSTRAT = Transmission hardware type definition.

-  VS_RLS = Vehicle speed at which reverse lock-out solenoid activates.

-  VS_RLSH = Hysteresis for reverse lock-out vehicle speed check.

OUTPUTS

Bit Flags

-  FLG_RLS = See above.

26-72

SHIFT CONTROL, REVERSE LOCK-OUT SOLENOID CONTROL - CDAN2

PED-PTE FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SC_RLS_COM1

TSTRAT > 1 -----------------------|

(not a manual transmission)      |OR --| EXIT MODULE

|     |

FLG_RLS_HP = 0 -------------------|     |

(reverse lock-out solenoid             |

not present)                          |

| --- ELSE ---

|

VSBART > VS_RLS ------------------------| FLG_RLS = 0

|

| --- ELSE ---

|

VSBART < VS_RLS - VS_RLSH --------------| FLG_RLS = 1

|

| --- ELSE ---

|

| NO UPDATE TO FLG_RLS

26-73

SHIFT CONTROL, SHIFT SOLENOID FUNCTIONAL TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

26.16  SHIFT SOLENOID FUNCITONAL TEST (CDAK0)

OVERVIEW

The shift solenoid functional test logic uses  the  results  from  the  shift

validation  logic  to  determine if shift solenoids are failed or not.  After

1-2, 2-3 and 3-4 upshifts have been  monitored,  the  combination  of  failed

upshifts  is  checked  to  determine if the failed shifts point to a solenoid

error.  One failed shift could not be the result of a solenoid failure.  Each

solenoid  failure  causes  two shift failures.  If both solenoids are failed,

all three shifts will fail.

The following information is used to determine if detected shift  errors  are

most likely due to solenoid failres (or other hydraulic/mechanical failures):

AODE

GEAR   SS2   SS1

1      0     1

2      0     0

3      1     0

4      1     1

With no faults present, actual gear = commanded, all shifts will be good.

SS1 failed OFF                             SS1 failed ON

GR_CM  SS2  SS1  Actual gear               GR_CM  SS2  SS1  Actual gear

1     0    0        2                      1     0    1        1

2     0    0        2                      2     0    1        1

3     1    0        3                      3     1    1        4

4     1    0        3                      4     1    1        4

1-2 and 3-4 shifts bad, 2-3 good           1-2 and 3-4 shifts bad, 2-3 good

SS2 failed OFF                             SS2 failed ON

GR_CM  SS2  SS1  Actual gear               GR_CM  SS2  SS1  Actual gear

1     0    1        1                      1     1    1        4

2     0    0        2                      2     1    0        3

3     0    0        2                      3     1    0        3

4     0    1        1                      4     1    1        4

2-3 and 3-4 shifts bad, 1-2 good           1-2 and 2-3 shifts bad, 3-4 good

Double failure - all shifts bad.

26-74

SHIFT CONTROL, SHIFT SOLENOID FUNCTIONAL TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  P0781FIL = 1-2 missed shift fault filter.

-  P0782FIL = 2-3 missed shift fault filter.

-  P0783FIL = 3-4 missed shift fault filter.

Bit Flags:

-  P0781MON = 1-2 shift monitor flag.

-  P0782MON = 2-3 shift monitor flag.

-  P0783MON = 3-4 shift monitor flag.

Calibration Constants:

-  SFT_12_ERR = Number of 1-2 shift errors for solenoid functionality test.

-  SFT_23_ERR = Number of 2-3 shift errors for solenoid functionality test.

-  SFT_34_ERR = Number of 3-4 shift errors for solenoid functionality test.

OUTPUTS

Bit Flags:

PxxxMALF = Malfunction  flag  for  fault  Pxxx;  1  ->  malfunction  Pxxx

currently exits.

-  SS1MALF  =  Shift  solenoid  1  malfunction  flag;  1  ->  SS1  currently

malfunctioning.

-  SS2MALF  =  Shift  solenoid  2  malfunction  flag;  1  ->  SS2  currently

malfunctioning.

-  SS_MALF_FLG = Shift solenoid malfunction flag;  1  ->  a  shift  solenoid

malfunction is currently present.

OTHER

-  ss_mil_codes = Set of {P0751,P0756} OBDII MIL fault codes that relate  to

shift solenoid functional test faults.

-  ss_non_mil_codes = Set of {P1751,P1756} OBDII NON-MIL  fault  codes  that

relate to shift solenoid functional test faults.

-  malfunction(ccm,Pxxx) = Logic process,  imported  from  the  MIL  control

module, Pxxx indicates a fault code.

26-75

SHIFT CONTROL, SHIFT SOLENOID FUNCTIONAL TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: SC_SOL_FN_TEST_COM1

P0781MON = 0 ---------------------|

(1-2 shift has not been          |

monitored)                      |

|

P0782MON = 0 ---------------------|OR --| Exit module

(2-3 shift has not been          |     |

monitored)                      |     |

|     |

P0783MON = 0 ---------------------|     |

(3-4 shift has not been                |

monitored)                            |

| --- ELSE ---

P0781FIL > SFT_12_ERR ------------|     |

(1-2 shifts bad)                 |     |

|     |

P0782FIL = 0 ---------------------|AND -| SS1MALF = 1

(Upshift occurred when 2-3       |     | Do: ss1_fault_code_logic

commanded)                      |     |

|     |

P0783FIL > SFT_34_ERR ------------|     | SS2MALF = 0

(3-4 shifts bad)                       | P0756MALF = 0

| P1756MALF = 0

|

| (shift solenoid 1 failed)

|

| --- ELSE ---

P0781FIL = 0 ---------------|           |

(Upshift occurred when 1-2 |           |

commanded)                |           |

|           |

P0782FIL > SFT_23_ERR ------|AND -|     |

(2-3 shifts bad)           |     |     |

|     |     |

P0783FIL > SFT_34_ERR ------|     |     |

(3-4 shifts bad)                 |     |

|OR --| SS1MALF = 0

P0781FIL > SFT_12_ERR ------|     |     | P0751MALF = 0

(1-2 shifts bad)           |     |     | P1751MALF = 0

|     |     |

P0782FIL > SFT_23_ERR ------|AND -|     | SS2MALF = 1

(2-3 shifts bad)           |           | Do: ss2_fault_code_logic

|           |

P0783FIL = 0 ---------------|           | (shift solenoid 2 failed)

(Upshift occured when 3-4              |

commanded)                            |

| --- ELSE ---

(Continued on the following page)

26-76

SHIFT CONTROL, SHIFT SOLENOID FUNCTIONAL TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(Continued from the previous page)

P0781FIL > SFT_12_ERR ------------|     |

(1-2 shifts bad)                 |     |

|     |

P0782FIL > SFT_23_ERR ------------|AND -| SS1MALF = 1

(2-3 shifts bad)                 |     | Do: ss1_fault_code_logic

|     |

P0783FIL > SFT_34_ERR ------------|     | SS2MALF = 1

(3-4 shifts bad)                       | Do: ss2_fault_code_logic

|

| --- ELSE ---

|

| SS1MALF = 0

| P0751MALF = 0

| P1751MALF = 0

|

| SS2MALF = 0

| P0756MALF = 0

| P1756MALF = 0

|

| (No fault present, or

|  shift errors do not

|  match solenoid failure

|  patterns, clear shift

|  solenoid funcational

|  test malfunciton flags)

SS1MALF = 1 ----------------------|

(SS1 fault currently present)    |OR --| SS_MALF_FLG = 1

|     |  (Set shift solenoid malfunction

SS2MALF = 1 ----------------------|     |   flag, used to flash the TCIL)

(SS2 fault currently present)          |

| --- ELSE ---

|

| SS_MALF_FLG = 0

26-77

SHIFT CONTROL, SHIFT SOLENOID FUNCTIONAL TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ss1_fault_code_logic

It has been requested by TED management that  illumination  of  the  MIL  for

shift  solenoid functional test codes be calibratable (can set a code without

turning the MIL on).  This can only be done by having two codes (one MIL, one

NON-MIL)  which  mean  the  same  thing,  with a calibration switch to select

between them.

SS_MIL_SW = 1 --------------------------| P0751MALF = 1

(MIL codes cal'ed in)                  | malfunction(ccm,P0751)

| (Set SS1 MIL code)

|

| --- ELSE ---

|

| P1751MALF = 1

| store_code(P1751)

| (Set SS1 NON-MIL code)

END: ss1_fault_code_logic

BEGIN: ss2_fault_code_logic

It has been requested by TED management that  illumination  of  the  MIL  for

shift  solenoid functional test codes be calibratable (can set a code without

turning the MIL on).  This can only be done by having two codes (one MIL, one

NON-MIL)  which  mean  the  same  thing,  with a calibration switch to select

between them.

SS_MIL_SW = 1 --------------------------| P0756MALF = 1

(MIL codes cal'ed in)                  | malfunction(ccm,P0756)

| (Set SS2 MIL code)

|

| --- ELSE ---

|

| P1756MALF = 1

| store_code(P1756)

| (Set SS2 NON-MIL code)

END: ss2_fault_code_logic

26-78

CHAPTER 27

ELECTRONIC PRESSURE CONTROL

27-1

ELECTRONIC PRESSURE CONTROL, ELECTRONIC PRESSURE CONTROL GUIDE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

27.1  ELECTRONIC PRESSURE CONTROL GUIDE (CDAN0)

OVERVIEW

EPC pressure is regulated by a variable force solenoid which is under  EEC-IV

control.   The  purpose of EPC pressure is to modulate the hydraulic pressure

used to apply, release, and hold  the  various  clutches  and  bands  in  the

transmission.   The higher the pressure, the more torque the transmission can

transmit.   This  input  torque  in  a  conventional  transmission  has  been

approximated by either a mechanical linkage connected to the throttle plates,

or a vacuum  diaphragm  which  sees  manifold  vacuum.   The  electronic  EPC

strategy  looks  up engine torque from a table and varies the EPC pressure to

contain the static capacity requirement of the transmission.

In general, EPC pressure is calculated as follows:

-  Static Capacity - This is the EPC required to hold the  weakest  friction

element  due to combustion torque (TQ_NET) and inertia torque (TQ_IALPHA)

during a shift.  Inertia torque is 0 when a shift is  not  taking  place.

The  sum of the combustion torque and inertia torque values is multiplied

by the torque converter torque ratio to determine the  total  torque  the

transmission  must  transmit  (tq_stat_cp).   This in turn determines the

static TV capacity requirement (TV_STAT).

-  Dynamic TV - This is TV required to obtain acceptable shift feel  and  is

the  powertrain  developers  main calibration tool.  A switch is provided

(SW_DYN) to allow the developer to either freeze RPM at a the start of  a

shift,  or  to  allow  dynamic TV pressure to follow RPM during the shift

(TV_DYN).  The combustion torque always updates, even during the shift.

-  AETV - Additional TV provided on quick tip-ins to counteract the  lag  in

EEC-IV updates to torque and to compensate for hydraulic lag times in the

VFS/TV hydraulic system.

-  Total TV - (TV_PRES) is simply  the  sum  of  static,  dynamic  and  AETV

requirements.

Additional features -

-  Cold Starts - additional TV can be requested to  counteract  the  viscous

effect of cold transmission oil on engagements

-  Rock cycling and high speed engagements - additional TV can be  requested

to protect the transmission capacity during severe engagement conditions

-  Tip-out from stall capacity hold - delay the release of stall TV pressure

during a quick tip-out to contain powertrain wind-up

-  Stall TV - at low speed ratio and vehicle speed stall TV is computed as a

function of throttle position

-  Coast boost - coast boost on a manual downshift is computed as a function

of output shaft speed.

-  Start-up - additional TV can be requested once per start-up  to  "charge"

the TV system in extremely cold ambients.

-  Engagement TV - A function of engine  speed  is  provided  to  model  the

engagement accumulator.

27-2

ELECTRONIC PRESSURE CONTROL, ELECTRONIC PRESSURE CONTROL GUIDE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  EIP_TMR = Engagement in progress timer.

-  EPC_BLP_TMR = Engagement EPC blip timer, sec.

-  EPC_ENG_TMR = Engagement EPC ramp timer, sec.

-  GR_CM = Current transmission gear.

-  GR_DS_TV = Desired gear used to compute TV pressure.

-  N = Engine speed, rpm.

-  NEBART = Filtered engine RPM for transmission, rpm.

-  NE_ENG_STRT = Engine speed at start of engagement, rpm.

-  NEU_RES_TMR = Neutral residency timer, sec.

-  N_RUN = Minimum engine speed to exit Crank mode.

-  PDL = Current PRNDL position.

-  PDL_LST = PRNDL position last background pass.

-  RT_NOVS = Ratio of actual N/V to base N/V in KAM.

-  SPD_RATIO = Speed ratio across torque converter.

-  TMR_ASILT = Timer which controls anti-silting logic.

-  TOT = Transmission oil temperature (degrees F).

-  TV_PRES = TV pressure, psi.

-  TV_PRES_BAR = Filtered EPC pressure, psi.

-  VSBART_RT = Filtered vehicle speed adjusted for RT_NOVS for transmission,

mph.

-  VSCTR = Count of MPH sensor errors.

Bit Flags:

-  CC_FM_FLG = Converter clutch failure mode flag; 0 -> normal operation,  1

-> execute converter clutch failure mode.

-  DRV2NEU_FLG = Forward prior to neutral flag.

-  ETV_TEST = ETV open/short test in progress.

-  FLG_ASILT = Anti-silting logic status; 1 -> anti-silting logic active.

-  FLG_DE_CMGR = Indicates  first  pass  through  the  delay  shift  for  TV

routine.

27-3

ELECTRONIC PRESSURE CONTROL, ELECTRONIC PRESSURE CONTROL GUIDE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FLG_DRV_REV = Forward to reverse engagement flag.

-  FLG_ENG_IN = Engagement in progress flag.

-  FLG_ENG_LS = Engagement in progress last pass flag.

-  FLG_ENG_TV = Engagement TV pressure flag; 0 -> do engagement logic, 1  ->

do not do engagement logic.

-  FLG_FRST_DS = First time a shift is desired flag; 0 -> no  shift  desired

this BG pass, 1 -> shift desired this BG pass.

-  FLG_FRST_TV = Start-up TV pressure flag; 0 -> do start-up TV logic, 1  ->

do not do start-up TV logic.

-  FLG_FWD_REV = Rock cycling engagement in progress flag.

-  FLG_NEU_DRV = Neutral to drive engagement flag.

-  FLG_NEU_REV = Neutral to reverse engagement flag.

-  FLG_PWR = Power mode flag; 1 -> power on mode, 0 -> power off mode.

-  FLG_REV_DRV = Reverse to drive engagement flag.

-  FLG_SFT_IN = Shift in progress flag; 1 -> shift in progress.

-  FLG_SFT_UP = Upshift flag; 1 -> current or last shift was an upshift.

-  FLG_TVENG_MD = Moderate temperature for TV engagement flag.

-  FLG_VE_DSGR = Verify desired gear first pass flag.

-  MFMFLG = MAP/MAF FMEM flag.

-  OTEMP_FM_FLG = Transmission overtemperature FMEM flag; 1 ->  transmission

is overtemperature.

-  PDL_ERROR = Flag indicating a MLIP sensor failure; 1 -> failure.

-  P0721MALF = Noisy OSS malfunction flag; 1 -> a noisy OSS failure has been

detected.

-  REV2NEU_FLG = Reverse prior to neutral flag.

-  SFT_FM_FLG = Shift error failure mode flag; 0 -> no failure mode  action,

1 -> failure mode action will be executed.

-  TFMFLG = TP FMEM flag.

-  OSFMFLG = Output shaft speed sensor FMEM flag.

Calibration Constants:

-  EPCFMRV = Failure mode EPC pressure for Reverse operation, psi.

-  FN12T(TOT) = TV_PRES multiplier versus TOT for upshift to 2nd gear.

27-4

ELECTRONIC PRESSURE CONTROL, ELECTRONIC PRESSURE CONTROL GUIDE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FN21T(TOT) = TV_PRES multiplier versus TOT for downshift to 1st gear.

-  FN23T(TOT) = TV_PRES multiplier versus TOT for upshift to 3rd gear.

-  FN32T(TOT) = TV_PRES multiplier versus TOT for downshift to 2nd gear.

-  FN34T(TOT) = TV_PRES multiplier versus TOT for upshift to 4th gear.

-  FN43T(TOT) = TV_PRES multiplier versus TOT for downshift to 3rd gear.

-  NEUTIM = Minimum time in neutral to use neutral to in gear engagement EPC

functions, sec.

-  RTSTAL = Maximum SPD_RATIO to do stall TV.

-  SW_STALTV = Software switch to do stall TV; 0  ->  do  normal  TV,  1  ->

always use stall TV.

-  TCEPC = Time constant for TV_PRES_BAR, sec.

-  TM_ASILT = Time for each step in anti-silting cycle.

-  TM46BLP = Time after a forward to reverse engagement to use FN46B for  TV

engagement pressure, sec.

-  TM54BLP = Time after a neutral to forward engagement to use FN54B for  TV

engagement pressure, sec.

-  TM56BLP = Time after a neutral to reverse engagement to use FN56B for  TV

engagement pressure, sec.

-  TM64BLP = Time after a reverse to drive engagement to use  FN64B  for  TV

engagement pressure, sec.

-  TMDRVREV = Time to complete a forward to reverse engagement, sec.

-  TMENGACT = Time to complete an engagement.

-  TMNEUDRV = Time to complete a neutral to forward engagement, sec.

-  TMNEUREV = Time to complete a neutral to reverse engagement, sec.

-  TMREVDRV = Time to complete a revolution to forward engagement, sec.

-  TVFMMN = Minimum TV clip when VS or PRNDL sensor has failed, psi.

-  TVPMIN_1 = Minimum TV pressure clip when GR_DS_TV = 1, psi.

-  TVPMIN_2 = Minimum TV pressure clip when GR_DS_TV = 2, psi.

-  TVPMIN_3 = Minimum TV pressure clip when GR_DS_TV = 3, psi.

-  TVPMIN_4 = Minimum TV pressure clip when GR_DS_TV = 4, psi.

-  TVPMIN_ENG = Minimum TV pressure clip during engagment, psi.

-  TVPMINPT_1 = Minimum TV pressure clip when PT and GR_DS_TV = 1, psi.

27-5

ELECTRONIC PRESSURE CONTROL, ELECTRONIC PRESSURE CONTROL GUIDE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TVPMINPT_2 = Minimum TV pressure clip when PT and GR_DS_TV = 2, psi.

-  TVPMINPT_3 = Minimum TV pressure clip when PT and GR_DS_TV = 3, psi.

-  TVPMINPT_4 = Minimum TV pressure clip when PT and GR_DS_TV = 4, psi.

-  TVPMINPT_ENG = Minimum TV pressure clip when PT during engagment, psi.

-  TVPMINST_1 = Minimum non-shifting TV_PRES, GR_DS_TV -> 1.

-  TVPMINST_2 = Minimum non-shifting TV_PRES, GR_DS_TV -> 2.

-  TVPMINST_3 = Minimum non-shifting TV_PRES, GR_DS_TV -> 3.

-  TVPMINST_4 = Minimum non-shifting TV_PRES, GR_DS_TV -> 4.

-  TV_ASILT1 = TV for stage one of anti-silting strategy.

-  TV_ASILT2 = TV for stage two of anti-silting strategy.

-  TV_ASILT3 = TV for stage three of anti-silting strategy.

-  VSSTAL = Maximum vehicle speed to do stall TV, mph.

27-6

ELECTRONIC PRESSURE CONTROL, ELECTRONIC PRESSURE CONTROL GUIDE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EPC_GUIDE_COM17

N > N_RUN -------------------|

|AND -|

PDL <= 4 --------------------|     |

(in fwd)                          |OR --| FLG_FRST_TV = 1

|     |  (Stop charging TV circuit

N > N_RUN -------------------|     |     |   after the 1st engagement)

|AND -|

PDL = 6 ---------------------|

(in rev)

always ----------------------------------| FLG_ENG_LS = FLG_ENG_IN

ENGAGEMENT FLAGS/TIMERS

PDL = 5 ---------------------|

(in neutral)                |OR --|

|     |

PDL = 7 ---------------------|     |AND -| NEU_RES_TMR = NEUTIM

(in park)                         |     |  (load neutral timer)

|     | DRV2NEU_FLG = 1

PDL_LST <= 4 ----------------------|     | REV2NEU_FLG = 0

(fwd last pass)                         |  (indicate fwd prior to neu)

|

| --- ELSE ---

PDL = 5 ---------------------|           |

(in neutral)                |OR --|     |

|     |     |

PDL = 7 ---------------------|     |AND -| NEU_RES_TMR = NEUTIM

(in park)                         |     |  (load neutral timer)

|     | DRV2NEU_FLG = 0

PDL_LST = 6 -----------------------|     | REV2NEU_FLG = 1

(rev last pass)                         |  (indicate rev prior to neu)

|

| --- ELSE ---

|

| NO ACTION

27-7

ELECTRONIC PRESSURE CONTROL, ELECTRONIC PRESSURE CONTROL GUIDE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PDL = 6 ---------------------------|

(in rev)                          |

|

PDL_LST = 5 -----------------|     |

(neu last pass)             |OR --|     |  (neutral to reverse)

|     |AND -| NE_ENG_STRT = NEBART

PDL_LST = 7 -----------------|     |     | EPC_ENG_TMR = TMNEUREV +

(park last pass)                  |     |          TM56BLP

|     | EPC_BLP_TMR = TM56BLP

|     | EIP_TMR = TMENGACT

NEU_RES_TMR = 0 -------------|     |     | FLG_DRV_REV = 0

(in neu long enough)        |OR --|     | FLG_NEU_DRV = 0

|           | FLG_NEU_REV = 1

DRV2NEU_FLG = 0 -------------|           | FLG_REV_DRV = 0

(not fwd prior to neu)                  |

| --- ELSE ---

|

PDL = 6 ---------------------------|     |  (drive to reverse)

(in rev)                          |AND -| NE_ENG_STRT = NEBART

|     | EPC_ENG_TMR = TMDRVREV +

PDL_LST <> 6 ----------------------|     |          TM46BLP

(fwd to rev or                          | EPC_BLP_TMR = TM46BLP

fwd to neu to rev with                 | EIP_TMR = TMENGACT

short time in neu)                     | FLG_DRV_REV = 1

| FLG_NEU_DRV = 0

| FLG_NEU_REV = 0

| FLG_REV_DRV = 0

|

| --- ELSE ---

PDL <= 4 --------------------------|     |

(in fwd)                          |     |

|     |

PDL_LST = 5 -----------------|     |     |

(neu last pass)             |OR --|     |  (neutral to drive)

|     |AND -| NE_ENG_STRT = NEBART

PDL_LST = 7 -----------------|     |     | EPC_ENG_TMR = TMNEUDRV +

(park last pass)                  |     |          TM54BLP

|     | EPC_BLP_TMR = TM54BLP

|     | EIP_TMR = TMENGACT

NEU_RES_TMR = 0 -------------|     |     | FLG_DRV_REV = 0

(in neu long enough)        |OR --|     | FLG_NEU_DRV = 1

|           | FLG_NEU_REV = 0

REV2NEU_FLG = 0 -------------|           | FLG_REV_DRV = 0

(not rev prior to neu)                  |

| --- ELSE ---

|

PDL <= 4 --------------------------|     |  (reverse to drive)

(in fwd)                          |AND -| NE_ENG_STRT = NEBART

|     | EPC_ENG_TMR = TMREVDRV +

PDL_LST > 4 -----------------------|     |          TM64BLP

(rev to fwd or                          | EPC_BLP_TMR = TM64BLP

rev to neu to fwd                      | EIP_TMR = TMENGACT

with short time in neu)                | FLG_DRV_REV = 0

| FLG_NEU_DRV = 0

| FLG_NEU_REV = 0

| FLG_REV_DRV = 1

27-8

ELECTRONIC PRESSURE CONTROL, ELECTRONIC PRESSURE CONTROL GUIDE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EPC_ENG_TMR > 0 -------------------------| FLG_ENG_IN = 1

|  (engagement in progress)

|

| --- ELSE ---

|

| FLG_ENG_IN = 0

|  (no engagement in progress)

FLG_ENG_IN = 1 --------------------|

(engagement in progress)          |

|

FLG_DRV_REV = 1 -------------|     |

(fwd to rev)                |     |

|     |AND -| FLG_FWD_REV = 1

FLG_REV_DRV = 1 -------------|     |     | DRV2NEU_FLG = 0

(rev to fwd)                |     |     | REV2NEU_FLG = 0

|     |     |  (set direction change flag

FLG_NEU_REV = 1 -------|     |OR --|     |   and clear "prior to neutral"

(neu to rev)          |AND -|           |   flags)

|     |           |

DRV2NEU_FLG = 1 -------|     |           |

(fwd prior to neu)          |           |

|           |

FLG_NEU_DRV = 1 -------|     |           |

(neu to fwd)          |AND -|           |

|                 |

REV2NEU_FLG = 1 -------|                 |

(rev prior to neu)                      |

| --- ELSE ---

|

FLG_ENG_IN = 0 --------------------------| FLG_FWD_REV = 0

(no engagement in progress)             |  (clear direction change flag)

|

| --- ELSE ---

|

| NO ACTION

always ----------------------------------| Do: EPC_ASILT_COM1

27-9

ELECTRONIC PRESSURE CONTROL, ELECTRONIC PRESSURE CONTROL GUIDE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLG_FWD_REV = 1 -------------------|

(direction change)                |

|

PDL >= 5  -------------------------|OR --| FLG_ENG_TV = 1

(not in forward gear)             |     |  (perform engagement/stall TV)

|     |

GR_DS_TV = GR_CM ------------|     |     |

(no verify in progress)     |AND -|     |

|           |

FLG_ENG_IN = 1 --------------|           |

(engagement in progress)                |

| --- ELSE ---

GR_DS_TV <> GR_CM -----------------|     |

(desired shift being verified)    |     |

|     |

FLG_FRST_DS = 1 -------------------|OR --| FLG_ENG_TV = 0

(shift is desired)                |     |  (Stop doing engagement/stall

|     |   TV; forward engagement is over

PDL <= 4 --------------------|     |     |   and trans is warm or a shift

(forward gear)              |AND -|     |   is pending)

|           |

FLG_TVENG_MD = 0 ------------|           |

(transmission warm)                     |

| --- ELSE ---

|

| NO ACTION

27-10

ELECTRONIC PRESSURE CONTROL, ELECTRONIC PRESSURE CONTROL GUIDE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PDL = 5 ---------------------------|

(neutral)                         |OR --| Do: "START-UP TV" LOGIC

|     |

PDL = 7 ---------------------------|     | flg_eng_stall = 1

(park)                                  |

| --- ELSE ---

SW_STALTV = 1 ---------------------|     |

(forced stall TV)                 |     |

|     |

FLG_ENG_TV = 1 --------------------|     |

(1st engagement)                  |     |

|OR --| Do: "ENGAGEMENT/STALL TV" LOGIC

SPD_RATIO <= RTSTAL ---------|     |     |

(low speed ratio)           |     |     | flg_eng_stall = 1

|     |     |

VSBART_RT <= VSSTAL ---------|AND -|     |

(low vehicle speed)         |           |

|           |

FLG_SFT_IN = 0 --------------|           |

(no shift in progress)                  |

| --- ELSE ---

|

| Do: "NORMAL TV CALCULATION"

|

| flg_eng_stall = 0

27-11

ELECTRONIC PRESSURE CONTROL, ELECTRONIC PRESSURE CONTROL GUIDE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLG_SFT_IN = 0 --------------|

|AND -|

GR_DS_TV = GR_CM ------------|     |

|OR --| NO ACTION

FLG_PWR = 0 -----------------|     |     |  (do not adjust TV_PRES for

|AND -|     |   transmission oil temperature

flg_eng_stall = 0 -----------|           |   unless shifting)

|

| --- ELSE ---

|

GR_DS_TV = 1 ----------------------------| TV_PRES = TV_PRES * FN21T(TOT)

|

| --- ELSE ---

GR_DS_TV = 2 ----------------------|     |

|AND -| TV_PRES = TV_PRES * FN12T(TOT)

FLG_SFT_UP = 1 --------------------|     |

| --- ELSE ---

|

GR_DS_TV = 2 ----------------------------| TV_PRES = TV_PRES * FN32T(TOT)

|

| --- ELSE ---

GR_DS_TV = 3 ----------------------|     |

|AND -| TV_PRES = TV_PRES * FN23T(TOT)

FLG_SFT_UP = 1 --------------------|     |

| --- ELSE ---

|

GR_DS_TV = 3 ----------------------------| TV_PRES = TV_PRES * FN43T(TOT)

|

| --- ELSE ---

|

GR_DS_TV = 4 ----------------------------| TV_PRES = TV_PRES * FN34T(TOT)

27-12

ELECTRONIC PRESSURE CONTROL, ELECTRONIC PRESSURE CONTROL GUIDE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

flg_eng_stall = 1 -----------------------| temp_tvpminpt = TVPMINPT_ENG

| temp_tvpmin = TVPMIN_ENG

|

| --- ELSE ---

|

GR_DS_TV = 1 ----------------------------| temp_tvpminpt = TVPMINPT_1

| temp_tvpmin = TVPMIN_1

| temp_tvpminst = TVPMINST_1

|

| --- ELSE ---

|

GR_DS_TV = 2 ----------------------------| temp_tvpminpt = TVPMINPT_2

| temp_tvpmin = TVPMIN_2

| temp_tvpminst = TVPMINST_2

|

| --- ELSE ---

|

GR_DS_TV = 3 ----------------------------| temp_tvpminpt = TVPMINPT_3

| temp_tvpmin = TVPMIN_3

| temp_tvpminst = TVPMINST_3

|

| --- ELSE ---

|

| temp_tvpminpt = TVPMINPT_4

| temp_tvpmin = TVPMIN_4

| temp_tvpminst = TVPMINST_4

FLG_SFT_IN = 1 --------------|

|

FLG_VE_DSGR = 1 -------------|

|OR --|

FLG_DE_CMGR = 1 -------------|     |

|     |AND -| TV_PRES =

flg_eng_stall = 1 -----------|     |     |   max(TV_PRES,temp_tvpminpt)

|     |

FLG_PWR = 1 -----------------------|     |

| --- ELSE ---

FLG_SFT_IN = 1 --------------|           |

|           |

FLG_VE_DSGR = 1 -------------|OR --|     |

|     |     |

FLG_DE_CMGR = 1 -------------|     |AND -| TV_PRES =

|     |   max(TV_PRES, temp_tvpmin)

FLG_PWR = 0 -----------------------|     |

| --- ELSE ---

flg_eng_stall = 1 -----------------|     |

|AND -| TV_PRES =

FLG_PWR = 0 -----------------------|     |   max(TV_PRES, temp_tvpmin)

|

| --- ELSE ---

|

| TV_PRES =

|   max(TV_PRES, temp_tvpminst)

27-13

ELECTRONIC PRESSURE CONTROL, ELECTRONIC PRESSURE CONTROL GUIDE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TFMFLG = 1 ------------------------|

(TP failed)                       |

|

MFMFLG = 1 ------------------------|AND -| TV_PRES = EPCFMRV

(MAP failed)                      |     |

|     |

PDL = 6 ---------------------------|     |

(PRNDL in Reverse)                      |

| --- ELSE ---

TFMFLG = 1 ------------------|           |

(TP failed)                 |AND -|     |

|     |     |

MFMFLG = 1 ------------------|     |OR --| TV_PRES = 127.5 (Maximum)

(MAP failed)                      |     |

|     |

ETV_TEST = 1 ----------------------|     |

| --- ELSE ---

OSFMFLG = 1 -----------------------|     |

(output shaft speed sensor        |     |

failed)                          |     |

|     |

P0721MALF = 1 ---------------------|     |

;Noisy OSS malfunction detected   |     |

|     |

SFT_FM_FLG = 1 --------------------|     |

(shift error detected)            |OR --| TV_PRES = max(TV_PRES,TVFMMN)

|

PDL_ERROR = 1 ---------------------|

(MLPS failed)                     |

|

CC_FM_FLG = 1 ---------------------|

(Converter Clutch error detected) |

|

OTEMP_FM_FLG = 1 ------------------|

(Transmission overtemperature)

27-14

ELECTRONIC PRESSURE CONTROL, ELECTRONIC PRESSURE CONTROL GUIDE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLG_ASILT = 0 -------------| tv_pres_bar = TV_PRES_BAR

| tv_pres_bar = rolav(TV_PRES,TCEPC)

| TV_PRES_BAR = tv_pres_bar

|

| --- ELSE ---

|

TMR_ASILT < TM_ASILT ------| TV_PRES = TV_ASILT1

| TV_PRES_BAR = TV_ASILT1

|

| --- ELSE ---

|

TMR_ASILT < 2 * TM_ASILT --| TV_PRES = TV_ASILT2

| TV_PRES_BAR = TV_ASILT2

|

| --- ELSE ---

|

TMR_ASILT < 3 * TM_ASILT --| TV_PRES = TV_ASILT3

| TV_PRES_BAR = TV_ASILT3

|

| --- ELSE ---

|

TMR_ASILT < 4 * TM_ASILT --| TV_PRES = TV_ASILT2

| TV_PRES_BAR = TV_ASILT2

|

| --- ELSE ---

|

| TV_PRES = TV_ASILT1

| TV_PRES_BAR = TV_ASILT1

always --------------------| Do: substitute(16,TV_PRES_BAR)

|  (output state control override)

always --------------------| Do: "EPC VFS CONTROL AND SELF

|      TEST" MODULE.

27-15

ELECTRONIC PRESSURE CONTROL, START-UP TV LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

27.2  START-UP TV LOGIC (CDAA0)

OVERVIEW

Start up TV is used to charge the TV system during cold ambient conditions.

DEFINITIONS

INPUTS

Registers:

-  RATCH = Closed throttle position, counts.

-  TP = Throttle position sensor.

-  TP_REL = Relative TP = TP - RATCH, counts.

Bit Flags:

-  FLG_FRST_TV = Start-up TV pressure flag:  0 -> do start-up TV logic; 1 ->

do not do start-up TV logic.

-  FLG_TVSTR_CD = Flag which indicates cold temperature for start-up TV:   0

-> Don't use TVCHRG in start-up TV; 1 -> Use TVCHRG in start-up TV.

Calibration Constants:

-  TVCHRG = TV charge pressure for first start-up.

OUTPUTS

Registers:

-  FN616F(TP_REL) = Stall TV pressure for forward.

-  TV_PRES = TV pressure.

27-16

ELECTRONIC PRESSURE CONTROL, START-UP TV LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: TV_STARTUP_COM5

FLG_TVSTR_CD = 1 -|

(cold engine)    |AND -| TV_PRES = TVCHRG

|     |  (charge TV circuit, first start-up only)

FLG_FRST_TV = 0 --|     |

(first start-up)       | --- ELSE ---

|

| TV_PRES = FN616F(TP_REL)

27-17

ELECTRONIC PRESSURE CONTROL, ENGAGEMENT/STALL TV LOGIC - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

27.3  ENGAGEMENT/STALL TV LOGIC (CDAA0)

OVERVIEW

Engagement/stall TV provides the greater of engagement TV or stall TV  unless

a  drive/reverse  engagement is taking place.  The engagement logic takes the

place of an engagement accumulator.  Note that  engagement  TV  is  generally

lower than stall TV at closed throttle.

DEFINITIONS

INPUTS

Registers:

-  NEBART = Filtered engine RPM for transmission, rpm.

-  PDL = Current PRNDL position.

-  TP = Instantaneous throttle position, counts.

-  TP_REL = Relative throttle position, counts.

-  TPBARTV = Filtered TP for TV strategy.  This filtered  throttle  position

is  calculated  using  the  ROLAV  subroutine,  but with a different time

constant then the values used in the TPBART and TPBARTC calculation.

TPBARTV is used in the  TV  routine  to  determine  a  tip-in  condition.

During a tip-in condition in the TV routine, extra TV is added.

-  TV_PRES = TV pressure.

-  VSBART_RT = Filtered vehicle speed adjusted for RT_NOVS for transmission,

mph.

Bit Flags:

-  FLG_ENG_IN = Engagement in progress flag.

-  FLG_ENG_TV = Engagement TV pressure flag; 0 -> do engagement logic, 1  ->

do not do engagement logic.

-  FLG_FWD_REV = Rock cycling engagment in progress flag.

-  FLG_TVENG_CD = Flag which indicates cold temperature for engagement TV; 0

-> Don't use TVEMAX in engagement TV, 1 -> Use TVEMAX in engagement TV.

-  FLG_TVENG_MD = Flag which indicates moderate temperature  for  engagement

TV; 0 -> Don't use TVEMOD in engagement TV, 1 -> Use TVEMAX in engagement

TV.

Calibration Constants:

27-18

ELECTRONIC PRESSURE CONTROL, ENGAGEMENT/STALL TV LOGIC - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FN616_SW = Switch to select FN616 usage during engagements.

-  FN616F(TP_REL) = Stall EPC for forward.

-  FN616R = Stall EPC for reverse.

-  NETVMN = Minimum RPM to use TVEMOD engagement TV.

-  NETVMX = Minimum RPM to use TVEMAX engagement TV.

-  REV_ENG_ADD = Reverse Engagement Adder.

-  SW_STALTV = Software switch to do stall TV; 0  ->  do  normal  TV,  1  ->

always use stall TV.

-  TOTVTP = Tip-out TP change to hold stall TV.

-  TPTVMN = Minimum TP_REL to use TVEMOD engagement TV.

-  TPTVMX = Minimum TP_REL to use TVEMAX engagement TV.

-  TVEMAX = TV for worst case engagement.

-  TVEMOD = TV for moderate engagement.

-  VSTVMN = Minimum vehicle speed to use TVEMOD engagement TV.

-  VSTVMX = Minimum vehicle speed to use TVEMAX engagement TV.

OUTPUTS

Registers:

-  TV_PRES = TV pressure.

27-19

ELECTRONIC PRESSURE CONTROL, ENGAGEMENT/STALL TV LOGIC - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EPC_ENGMT_STALL_COM6

ENGAGEMENT/STALL TV LOGIC

SW_STALTV = 0 -------------------------|

(not forcing Stall EPC)               |

|

FN616_SW = 1 --------------------------|AND -| tempreg1 = 0

(FN616 selection switch)              |     |  (do not consider FN616

|     |   during engagement)

FLG_ENG_IN = 1 ------------------------|     |

(engagement in progress)                    | --- ELSE ---

|

PDL = 6 -------------------------------------| tempreg1 = FN616R(TP_REL)

|  (determine reverse stall

|   TV)

|

| --- ELSE ---

|

| tempreg1 = FN616F(TP_REL)

|  (determine forward stall

|   TV)

27-20

ELECTRONIC PRESSURE CONTROL, ENGAGEMENT/STALL TV LOGIC - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

ENGAGEMENT/STALL TV LOGIC (continued)

VSBART_RT >= VSTVMX -------------|

(high vehicle speed)            |AND -|

|     |

FLG_FWD_REV = 1 -----------------|     |

(fwd/rev or rev/fwd)                  |

|OR --| tempreg2 = TVEMAX

FLG_ENG_TV = 1 ------------------|     |     |      [ + REV_ENG_ADD]

(engagement situation)          |     |     |  (set maximum TV

|     |     |   engagement pressure)

TP_REL >= TPTVMX ----------|     |AND -|     |

(high TP)                 |     |           |

|     |           |

NEBART >= NETVMX ----------|OR --|           |

(high engine speed)       |                 |

|                 |

FLG_TVENG_CD = 1 ----------|                 |

(cold temperature)                          |

| --- ELSE ---

VSTVMN < VSBART_RT < VSTVMX -----|           |

(moderate vehicle speed)        |AND -|     |

|     |     |

FLG_FWD_REV = 1 -----------------|     |     |

(fwd/rev or rev/fwd)                  |     |

|OR --| tempreg2 = TVEMOD

FLG_ENG_TV = 1 ------------------|     |     |      [ + REV_ENG_ADD]

(engagement situation)          |     |     |  (set moderate TV

|     |     |   engagement pressure)

TPTVMN < TP_REL < TPTVMX --|     |AND -|     |

(moderate TP)             |     |           |

|     |           |

NETVMN < NEBART < NETVMX --|OR --|           |

(moderate engine speed)   |                 |

|                 |

FLG_TVENG_MD = 1 ----------|                 |

(moderate temperature)                      |

| --- ELSE ---

|

| tempreg2 = 0

Note:  "[ ]" indicates optional term in tempreg2 value when TVEMAX or  TVEMIN

are selected.

27-21

ELECTRONIC PRESSURE CONTROL, ENGAGEMENT/STALL TV LOGIC - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

ENGAGEMENT/STALL TV LOGIC (continued)

The following logic controls REV_ENG_ADD usage:

PDL = 6 -------------------------------------| include REV_ENG_ADD in

|  tempreg2 if using

|  TVEMAX or TVEMOD

|

| --- ELSE ---

|

| do not include

|  REV_ENG_ADD

tempreg2 > tempreg1 -------------------------| tempreg1 = tempreg2

| (choose the larger of the

|  stall TV pressure or

|  engagement TV pressure,

|  put in tempreg1)

(TPBARTV - TP) >= TOTVTP --------------|

(high tip-out rate)                   |AND -| No change to TV_PRES

|     |  (use last pass value to

TV_PRES > tempreg1 --------------------|     |   hold TV pressure on tip-out)

(last pass value has enough capacity)       |

| --- ELSE ---

|

| TV_PRES = tempreg1

|  (use the higher of stall

|   TV or engagement TV)

27-22

ELECTRONIC PRESSURE CONTROL, NORMAL TV CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

27.4  NORMAL TV CALCULATION (CDAN0)

OVERVIEW

Normal EPC computes the EPC pressure required  to  maintain  static  capacity

based  on  engine  output  torque  (TQ_NET).   During a shfit, inertia torque

(TQ_IALPHA) is added to the total torque.  A seperate slope and intercept  is

provided for calculating the static capacity during shifting and non-shifting

operation.  Dynamic EPC is called from this module.

During shift verification, cold operation (FLG_DYN_CD  =  1),  and  when  the

vehicle  is  in  4X4L,  the  shifting static and dynamic EPC calculations are

performed as well as the non-sfhiting static calculation.  The strategy  then

chooses the higher of the two pressures (shifting vs.  non-shifting).

An option is added to the shifting static calculation to hold or ramp  static

capacity  at  the  end  of  the  ratio  change during an upshift (when torque

modulation ends).  Calibrating the EPC scalar to 0 (EPC_ST_12, EPC_ST_23, and

EPC_ST_34), the strategy will hold the last pass value of the shifting static

EPC until the EPC_RMP_TMR expires, or the shift ends.  A non-zero  value  for

EPC_RMP_TM and EPC_ST_RMP is required for the ramp to initiate.  The ramp can

be calibrated out by setting  the  EPC_RMP_TM  values  to  zero  (EPC_RMP_12,

EPC_RMP_23, and EPC_RMP_34).

During heavy tip-ins, an additional  pressure  is  added  to  the  static  to

compenstate for system delays (AETV).

DEFINITIONS

INPUTS

Registers:

-  EPC_RMP_TM = Time over which static capacity is ramped at the end of  the

ratio change, seconds.

-  EPC_RMP_TMR = Timer which control static EPC ramp, initiated  at  end  of

the ratio change, seconds.

-  EPC_ST_RMP = EPC value  which  sets  the  slope  of  the  ramp  (slope  =

EPC_ST_RMP/ramp time), psi.

-  GR_CM = Current transmission gear.

-  GR_DS_TV = Desired gear used to compute EPC pressure.

-  NEBART = Filtered engine RPM for transmission.

-  RATCH = Closed throttle position, counts.

-  SPD_RATIO = Speed ratio across the torque converter.

-  TP = Throttle Position, counts.

27-23

ELECTRONIC PRESSURE CONTROL, NORMAL TV CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TPBARTV = Filtered TP for TV strategy.  TPBARTV is used in the TV routine

to  determine  a  tip-in  condition.  During a tip-in condition in the TV

routine, extra TV is added.

-  TP_REL = Relative Throttle Position, TP - RATCH.

-  TP_STRT_SFT = TP_REL at the start of a shift, counts.

-  TQ_IALPHA = I-ALPHA torque due to ratio change.

-  TQ_NET = Net torque into torque converter.

-  TQ_STAT_CP = Static Capacity Torque of transmission.

-  TR_STRT_SFT = Torque ratio at the start of a shift.

-  TV_DYN = TV pressure required for dynamic shift control.

-  TV_PRES = TV pressure.

-  TV_STAT = TV pressure required for static capacity.

-  TV_ST_LST = Last pass value of TV_ST_SFT, psi.

-  TV_ST_SFT = Static TV pressure while shifting.

-  VSBART_RT = Filtered vehicle speed adjusted for RT_NOVS for transmission,

MPH.

27-24

ELECTRONIC PRESSURE CONTROL, NORMAL TV CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Bit Flags:

-  FLG_4X4L = 4X4L flag:  0 -> not in 4X4 L; 1 -> in 4X4 L.

-  FLG_DYN_CD = Flag which indicates that it is necessary to add dynamic  TV

due to cold transmission conditions.

-  FLG_EPC_RMP =  Indicates  end  of  ratio  change,  begin  ramping  static

capacity.

-  FLG_FRST_CM = Flag indicating a shift was commanded this background loop.

-  FLG_SPK_TQM  =  Torque  modulation  spark  retard  flag.   1  ->   toruqe

modulation is in progress.

Calibration Constants:

-  AETV = Anticipatory TV adder for heavy tip-ins.

-  AETVTP = Minimum throttle change to include AETV in TV_STAT.

-  EPC_RMP_12 = Time over which to ramp static EPC at the end of  the  ratio

change for a 1-2 upshift, seconds.

-  EPC_RMP_23 = Time over which to ramp static EPC at the end of  the  ratio

change for a 2-3 upshift, seconds.

-  EPC_RMP_34 = Time over which to ramp static EPC at the end of  the  ratio

change for a 3-4 upshift, seconds.

-  EPC_ST_12 = EPC value which sets the slope of the static EPC ramp  for  a

1-2 upshift (slope = EPC_ST_12/ramp time), seconds.

-  EPC_ST_23 = EPC value which sets the slope of the static EPC ramp  for  a

2-3 upshift (slope = EPC_ST_23/ramp time), seconds.

-  EPC_ST_34 = EPC value which sets the slope of the static EPC ramp  for  a

3-4 upshift (slope = EPC_ST_34/ramp time), seconds.

-  FN617(SPD_RATIO) = Torque converter torque ratio.

-  SCINT1 = Static capacity TV intercept, first gear.

-  SCINT2 = Static capacity TV intercept, second gear.

-  SCINT3 = Static capacity TV intercept, third gear.

-  SCINT4 = Static capacity TV intercept, fourth gear.

-  SCSLP1 = Static capacity TV slope, first gear.

-  SCSLP1SD = Static capacity TV slope, downshift to first gear.

-  SCSLP2 = Static capacity TV slope, second gear.

-  SCSLP2SD = Static capacity TV slope, downshift to second gear.

27-25

ELECTRONIC PRESSURE CONTROL, NORMAL TV CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  SCSLP2SU = Static capacity TV slope, upshift to second gear.

-  SCSLP3 = Static capacity TV slope, third gear.

-  SCSLP3SD = Static capacity TV slope, downshift to third gear.

-  SCSLP3SU = Static capacity TV slope, upshift to third gear.

-  SCSLP4 = Static capacity TV slope, fourth gear.

-  SCSLP4SU = Static capacity TV slope, upshift to fourth gear.

OUTPUTS

Registers:

-  EPC_RMP_TM = See above.

-  EPC_RMP_TMR = See above.

-  EPC_ST_RMP = See above.

-  NE_STRT_SFT = Engine RPM at strat of shift.

-  TQ_STAT_CP = See above.

-  TP_STRT_SFT = See above.

-  TV_DYN = See above.

-  TV_PRES = See above.

-  VS_STRT_SFT = Filtered vehicle speed at start of shift, MPH.

Bit Flags:

-  FLG_EPC_RMP = See above.

27-26

ELECTRONIC PRESSURE CONTROL, NORMAL TV CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EPC_NORM_COM1

BEGIN:  MAIN

FLG_FRST_CM = 1 ---------------------| NE_STRT_SFT = NEBART

(start of shift)                    | VS_STRT_SFT = VSBART_RT

| TR_STRT_SFT = FN617(SPD_RATIO)

| TP_STRT_SFT = TP_REL

|  (capture engine speed, torque

|   ratio and throttle position at

|   start of shift)

| DO "TQ_IALPHA CALCULATION"

Now calculate the minimum static capacity.

FLG_SFT_IN = 1 ----------------------| TQ_STAT_CP = (TQ_NET + TQ_IALPHA) *

(shift in progress,                 |               TR_STRT_SFT

use IALPHA term)                   |  (calculate static torque capacity

|   required)

|

| --- ELSE ---

|

FLG_SFT_IN = 0 ----------------------| TQ_IALPHA = 0

(shift done, use steady torque      | TQ_STAT_CP = TQ_NET * FN617(SPD_RATIO)

requirement)                       |  (calculate steady state input torque

|   through torque converter ratio)

Calculate steady state pressure for current gear.

FLG_SFT_IN = 0 ----------------------| DO "NON-SHIFTING STATIC TV

|     CALCULATION"

27-27

ELECTRONIC PRESSURE CONTROL, NORMAL TV CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calculate pressure for shift using shifting slopes and Dynamic TV.

FLG_SFT_IN = 1 ----------------|

|

GR_DS_TV <> GR_CM -------------|OR --| DO "SHIFTING STATIC TV CALCULATION"

|     | DO "DYNAMIC TV CALCULATION"

FLG_DYN_CD = 1 ----------------|

|

FLG_4X4L = 1 ------------------|

Select proper value for TV_PRES.

FLG_SFT_IN = 0 ----------------|

|

GR_DS_TV = GR_CM --|           |AND -| TV_PRES = TV_STAT

|           |     |

FLG_4X4L = 0 ------|AND -|     |     |

|     |     |     |

FLG_DYN_CD = 0 ----|     |OR --|     |

|           |

TV_STAT > TV_ST_SFT      |           |

+ TV_DYN -----|           |

| --- ELSE ---

|

| TV_PRES = TV_ST_SFT + TV_DYN

|  (Clip TV_PRES to zero minimum)

Now add anticipatory TV value for a tip-in.

FLG_SFT_IN = 0 ----------------|

|AND -| TV_PRES = TV_PRES + AETV

(TP - TPBARTV) >= AETVTP ------|     |  (adjust TV pressure for heavy

|   tip-in to compensate for all

|   system delays)

END: MAIN

27-28

ELECTRONIC PRESSURE CONTROL, NORMAL TV CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: NON-SHIFTING STATIC TV CALCULATION

GR_CM = 1 ---------------------------| TV_STAT = (TQ_STAT_CP * SCSLP1) +

|            SCINT1

|

| --- ELSE ---

|

GR_CM = 2 ---------------------------| TV_STAT = (TQ_STAT_CP * SCSLP2) +

|            SCINT2

|

| --- ELSE ---

|

GR_CM = 3 ---------------------------| TV_STAT = (TQ_STAT_CP * SCSLP3) +

|            SCINT3

|

| --- ELSE ---

|

| TV_STAT = (TQ_STAT_CP * SCSLP4) +

|            SCINT4

END: NON-SHIFTING STATIC TV CALCULATION

27-29

ELECTRONIC PRESSURE CONTROL, NORMAL TV CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: SHIFTING STATIC TV CALCULATION

FLG_SPK_TQM = 1 ---------------------| TV_ST_LST = TV_ST_SFT

| ;store last pass value of

| ;TV_ST_SFT during torque

| ;modulation

GR_DS_TV = 1 ------------------------| TV_ST_SFT = (TQ_STAT_CP * SCSLP1SD) +

|              SCINT1

| EPC_RMP_TM = 0

| EPC_ST_RMP = 0

|

| --- ELSE ---

GR_DS_TV = 2 ------------------|     |

|AND -| TV_ST_SFT = (TQ_STAT_CP * SCSLP2SU) +

FLG_SFT_UP = 1 ----------------|     |              SCINT2

| EPC_RMP_TM = EPC_RMP_12

| EPC_ST_RMP = EPC_ST_12

|

| --- ELSE ---

|

GR_DS_TV = 2 ------------------------| TV_ST_SFT = (TQ_STAT_CP * SCSLP2SD) +

|              SCINT2

| EPC_RMP_TM = 0

| EPC_ST_RMP = 0

|

| --- ELSE ---

GR_DS_TV = 3 ------------------|     |

|AND -| TV_ST_SFT = (TQ_STAT_CP * SCSLP3SU) +

FLG_SFT_UP = 1 ----------------|     |              SCINT3

| EPC_RMP_TM = EPC_RMP_23

| EPC_ST_RMP = EPC_ST_23

|

| --- ELSE ---

|

GR_DS_TV = 3 ------------------------| TV_ST_SFT = (TQ_STAT_CP * SCSLP3SD) +

|              SCINT3

| EPC_RMP_TM = 0

| EPC_ST_RMP = 0

|

| --- ELSE ---

|

| TV_ST_SFT = (TQ_STAT_CP * SCSLP4SU) +

|              SCINT4

| EPC_RMP_TM = EPC_RMP_34

| EPC_ST_RMP = EPC_ST_34

27-30

ELECTRONIC PRESSURE CONTROL, NORMAL TV CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLG_FRST_CM = 1 ---------------------| EPC_RMP_TMR = 0

| ;insure ramp is

| ;zeroed out

|

| --- ELSE ---

|

FLG_EPC_RMP = 1 ---------------------| EPC_RMP_TMR = EPC_RMP_TM

| ;initalize timer for

| ;ramping static capacity

| FLG_EPC_RMP = 0

EPC_RMP_TM = 0 ----------------|

|OR --| NO Action

EPC_RMP_TMR = 0 ---------------|     | ;Allow TV_ST_SFT to pass

| ;through

|

| --- ELSE ---

|

EPC_RMP_TMR > 0 ---------------------| tv_st_tmp = TV_ST_LST +

|  EPC_ST_RMP (1 - (EPC_RMP_TMR/

|  EPC_RMP_TM))

| TV_ST_SFT = min(tv_st_tmp,

|  TV_ST_SFT)

| ;clip value to TV_ST_SFT

END: SHIFTING STATIC TV CALCULATION

27-31

ELECTRONIC PRESSURE CONTROL, TQ_IALPHA CALCULATION - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

27.5  TQ_IALPHA CALCULATION (CDAA0)

OVERVIEW

The I-ALPHA torque calculation determines the  torque  that  results  from  a

transmission upshift or downshift.

DEFINITIONS

INPUTS

Registers:

-  GEAR_CUR = Current transmission gear.

-  GEAR_OLD = Last commanded gear.

-  NE_STRT_SFT = Engine RPM at start of a shift.

-  RT_GR_CUR = Current transmission gear ratio.

-  RT_GR_OLD = Last gear transmission gear ratio.

Calibration Constants:

-  TQIA12 = I-ALPHA torque constant for 1 - 2.

-  TQIA21 = I-ALPHA torque constant for 2 - 1.

-  TQIA23 = I-ALPHA torque constant for 2 - 3.

-  TQIA32 = I-ALPHA torque constant for 3 - 2.

-  TQIA34 = I-ALPHA torque constant for 3 - 4.

-  TQIA43 = I-ALPHA torque constant for 4 - 3.

OUTPUTS

Registers:

-  TQ_IALPHA = I-ALPHA torque due to ratio change.

27-32

ELECTRONIC PRESSURE CONTROL, TQ_IALPHA CALCULATION - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EPC_TQ_IALPHA_COM4

GEAR_CUR = 2 --|

|AND --| TQ_IALPHA = NE_STRT_SFT * TQIA12 *

GEAR_OLD = 1 --|      |   [1 - (RT_GR_CUR/RT_GR_OLD)]

|

| --- ELSE ---

GEAR_CUR = 1 --|      |

|AND --| TQ_IALPHA = NE_STRT_SFT * TQIA21 *

GEAR_OLD > 1 --|      |   [1 - (RT_GR_CUR/RT_GR_OLD)]

|

| --- ELSE ---

GEAR_CUR = 3 --|      |

|AND --| TQ_IALPHA = NE_STRT_SFT * TQIA23 *

GEAR_OLD < 3 --|      |   [1 - (RT_GR_CUR/RT_GR_OLD)]

|

| --- ELSE ---

GEAR_CUR = 2 --|      |

|AND --| TQ_IALPHA = NE_STRT_SFT * TQIA32 *

GEAR_OLD > 2 --|      |   [1 - (RT_GR_CUR/RT_GR_OLD)]

|

| --- ELSE ---

GEAR_CUR = 4 --|      |

|AND --| TQ_IALPHA = NE_STRT_SFT * TQIA34 *

GEAR_OLD < 4 --|      |   [1 - (RT_GR_CUR/RT_GR_OLD)]

|

| --- ELSE ---

GEAR_CUR = 3 --|      |

|AND --| TQ_IALPHA = NE_STRT_SFT * TQIA43 *

GEAR_OLD = 4 --|      |   [1 - (RT_GR_CUR/RT_GR_OLD)]

27-33

ELECTRONIC PRESSURE CONTROL, DYNAMIC EPC CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

27.6  DYNAMIC EPC CALCULATION (CDAM0)

OVERVIEW

Dynamic EPC calculates the EPC pressure required to obtain good shift feel.

DEFINITIONS

Registers:

-  GEAR_CUR = Current commanded gear.

-  GR_DS_TV = Gear to be used for dynamic EPC calculation.

-  PNWF = Performance and Normal Weighting Factor.

-  TP_REL = Relative throttle position, counts.

-  TP_STRT_SFT = Throttle position at start of a shift, counts.

-  TV_DYN = EPC pressure required for dynamic shift control.

-  TV_RAMP = Stores added EPC value during upshifts.

-  TV_RAMP_TMR = EPC ramp timer during upshifts.

-  TVRMPTM = Stores EPC ramp timer for upshifts.

-  VSBART_RT = Filtered vehicle speed adjusted for RT_NOVS for transmission,

mph.

-  VS_STRT_SFT = Filtered vehicle speed at start of shift, MPH.

Bit Flags:

-  FLG_MAN42 = Identifies a Manual 4-2 pull-in.

-  FLG_AUT41 = Identifies a Automatic 4-1 pull-in.

-  FLG_AUT42 = Identifies a Automatic 4-2 pull-in.

-  FLG_FRST_CM = First time a shift is commanded flag; 0 -> no.

-  FLG_PWR = 1 -> power on; 0 -> power off.

-  FLG_SFT_DN = Downshift flag.

-  FLG_SFT_IN = Shift in progress flag; 0 -> no  shift  in  progress,  1  ->

shift in progress.

-  FLG_SFT_UP = Upshift flag; 1 -> last/current shift is upshift.

-  FLG_SPK_TQM = Torque Modulation spark retard flag.

27-34

ELECTRONIC PRESSURE CONTROL, DYNAMIC EPC CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FLG_TRIG_RMP = Triggers start of  dynamic  EPC  ramps  -  set  by  Torque

Modulation strategy.

Calibration Constants:

-  EPC_MAN42 = Dynamic EPC adder for a Manual 4-2 pull-in, psi.

-  EPC_AUT41 = Dynamic EPC adder for a Automatic 4-1 pull-in, psi.

-  EPC_AUT423 = Dynamic EPC scalar adder for the 4-3 part  of  an  Automatic

4-2 shift, psi.

-  FN12DYTX_VS = Dynamic EPC pressure for 1 - 2 shift, table of  TP_REL  and

VSBART_RT.

X = FNX12T(tempreg) = Normalizing function for TP_REL.

Y = FNX12V(tempreg_vs) = Normalizing function for VSBART_RT.

-  FN12DCP = Dynamic EPC pressure performance  mode  adjustment  for  a  1-2

shift.

-  FN21DYTX_VS = Dynamic EPC pressure for 2 - 1 shift.

X = FNX21T(tempreg) = Normalizing function for TP_REL.

Y = FNX21V(tempreg_vs) = Normalizing function for VSBART_RT.

-  FN21DCP = Dynamic EPC pressure performance  mode  adjustment  for  a  2-1

shift.

-  FN23DCP = Dynamic EPC pressure performance  mode  adjustment  for  a  2-3

shift.

-  FN23DYTX_VS = Dynamic EPC pressure for 2 - 3 shift, table of  TP_REL  and

VSBART_RT.

X = FNX23T(tempreg) = Normalizing function for TP_REL.

Y = FNX23V(tempreg_vs) = Normalizing function for VSBART_RT.

-  FN32DYTX_VS = Dynamic EPC pressure for 3 - 2 shift, table of  TP_REL  and

VSBART_RT.

X = FNX32T(tempreg) = Normalizing function for TP_REL.

Y = FNX32V(tempreg_vs) = Normalizing function for VSBART_RT.

-  FN32DCP = Dynamic EPC pressure performance  mode  adjustment  for  a  3-2

shift.

-  FN34DCP = Dynamic EPC pressure performance  mode  adjustment  for  a  3-4

shift.

-  FN34DYTX_VS = Dynamic EPC pressure for 3 - 4 shift, table of  TP_REL  and

VSBART_RT.

X = FNX34T(tempreg) = Normalizing function for TP_REL.

Y = FNX34V(tempreg_vs) = Normalizing function for VSBART_RT.

-  FN422A_EPC = Dynamic EPC adder for the  3-2  part  of  an  Automatic  4-2

shift, psi.  (function of VSBART_RT).

-  FN43DCP = Dynamic EPC pressure performance  mode  adjustment  for  a  4-3

shift.

27-35

ELECTRONIC PRESSURE CONTROL, DYNAMIC EPC CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FN43DYTX_VS = Dynamic EPC pressure for 4 - 3 shift, table of  TP_REL  and

VSBART_RT.

X = FNX43T(tempreg) = Normalizing function for TP_REL.

Y = FNX43V(tempreg_vs) = Normalizing function for VSBART_RT.

-  SW_DYN = Software switch for dynamic EPC; 0 -> allow  shift  dynamics  to

vary with TP, 1 -> use TP at start of shift for dynamic EPC.

-  SW_DYN_VS = Software switch for dynamic EPC for 4-3 and 3-2 shifts; 0  ->

allow  shift  dynamics  for  vary with VS, 1 -> use VSBART_RT at start of

shift for dynamic EPC.

-  TMTVRMP_12 = EPC ramp timer for 1-2 shift, sec.

-  TMTVRMP_23 = EPC ramp timer for 2-3 shift, sec.

-  TMTVRMP_34 = EPC ramp timer for 3-4 shift, sec.

-  TVRMP_12 = Added EPC during 1-2 shift.

-  TVRMP_23 = Added EPC during 2-3 shift.

-  TVRMP_34 = Added EPC during 3-4 shift.

-  TV_4L_12 = 4x4l adder for 1 - 2 shift, psi.

-  TV_4L_21 = 4x4l adder for 2 - 1 shift, psi.

-  TV_4L_23 = 4x4l adder for 2 - 3 shift, psi.

-  TV_4L_32 = 4x4l adder for 3 - 2 shift, psi.

-  TV_4L_34 = 4x4l adder for 3 - 4 shift, psi.

-  TV_4L_43 = 4x4l adder for 4 - 3 shift, psi.

27-36

ELECTRONIC PRESSURE CONTROL, DYNAMIC EPC CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EPC_DYNAMIC_COM14

SW_DYN = 1 -------|

|AND -| tempreg = TP_STRT_SFT

FLG_SFT_IN = 1 ---|     |  (do not allow shift dynamics to vary)

|

| --- ELSE ---

|

| tempreg = TP_REL

|  (allow shift dynamics to vary)

SW_DYN_VS = 1 ----|

|AND -| tempreg_vs = VS_STRT_SFT

FLG_SFT_IN = 1 ---|     |  (do not allow shift dynamics to vary)

|

| --- ELSE ---

|

| tempreg_vs = VSBART_RT

|  (allow shift dynamics to vary)

27-37

ELECTRONIC PRESSURE CONTROL, DYNAMIC EPC CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

GR_DS_TV = 1 -----|

|AND -| TV_DYN = FN21DYTX_VS(tempreg, tempreg_vs)

FLG_AUT41 = 1 ----|     |           + EPC_AUT41

|           + [PNWF * FN21DCP(TP_REL)]

|

| --- ELSE ---

|

GR_DS_TV = 1 -----------| TV_DYN = FN21DYTX_VS(tempreg, tempreg_vs) +

|             [PNWF * FN21DCP(TP_REL)]

|

| --- ELSE ---

GR_DS_TV = 2 -----|     |

|     |

FLG_MAN42 = 1 ----|AND -| TV_DYN = FN32DYTX_VS(tempreg, tempreg_vs)

|     |           + EPC_MAN42

FLG_PWR = 1 ------|     |           + [PNWF * FN32DCP(TP_REL)]

|

| --- ELSE ---

GR_DS_TV = 2 -----|     |

|AND -| TV_DYN = FN12DYTX_VS(tempreg, tempreg_vs) +

FLG_SFT_UP = 1 ---|     |             [PNWF * FN12DCP(TP_REL)]

|

| --- ELSE ---

GR_DS_TV = 2 -----|     |

|AND -| TV_DYN = FN32DYTX_VS(tempreg, tempreg_vs)

FLG_AUT41 = 1 ----|     |           + EPC_AUT41

|           + [PNWF * FN32DCP(TP_REL)]

|

| --- ELSE ---

GR_DS_TV = 2 -----|     |

|AND -| TV_DYN = FN32DYTX_VS(tempreg, tempreg_vs)

FLG_AUT42 = 1 ----|     |           + FN422A_EPC

|           + [PNWF * FN32DCP(TP_REL)]

|

| --- ELSE ---

|

GR_DS_TV = 2 -----------| TV_DYN = FN32DYTX_VS(tempreg, tempreg_vs) +

|             [PNWF * FN32DCP(TP_REL)]

|

| --- ELSE ---

GR_DS_TV = 3 -----|     |

|     |

FLG_MAN42 = 1 ----|AND -| TV_DYN = FN43DYTX_VS(tempreg, tempreg_vs)

|     |           + EPC_MAN42

FLG_PWR = 1 ------|     |           + [PNWF * FN43DCP(TP_REL)]

|

| --- ELSE ---

GR_DS_TV = 3 -----|     |

|AND -| TV_DYN = FN23DYTX_VS(tempreg, tempreg_vs) +

FLG_SFT_UP = 1 ---|     |             [PNWF * FN23DCP(TP_REL)]

|

| --- ELSE ---

(continued on next page)

27-38

ELECTRONIC PRESSURE CONTROL, DYNAMIC EPC CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

GR_DS_TV = 3 -----|     |

|AND -| TV_DYN = FN43DYTX_VS(tempreg, tempreg_vs)

FLG_AUT41 = 1 ----|     |           + EPC_AUT41

|           + [PNWF * FN43DCP(TP_REL)]

|

| --- ELSE ---

GR_DS_TV = 3 -----|     |

|AND -| TV_DYN = FN43DYTX_VS(tempreg, tempreg_vs)

FLG_AUT42 = 1 ----|     |           + EPC_AUT423

|           + [PNWF * FN43DCP(TP_REL)]

|

| --- ELSE ---

|

GR_DS_TV = 3 -----------| TV_DYN = FN43DYTX_VS(tempreg, tempreg_vs) +

|            [PNWF * FN43DCP(TP_REL)]

|

| --- ELSE ---

|

GR_DS_TV = 4 -----------| TV_DYN = FN34DYTX_VS(tempreg, tempreg_vs) +

|            [PNWF * FN34DCP(TP_REL)]

|

| --- ELSE ---

|

| TV_DYN = 0

FLG_FRST_CM = 1 --------| TVRMPTM = 1

| TV_RAMP_TMR = 1

| TV_RAMP = 0

| (reset TV_RAMP value)

| FLG_TRIG_RMP = 1

| (trigger EPC ramps)

FLG_TRIG_RMP = 1 -|

|

FLG_SFT_UP = 1 ---|AND -| TVRMPTM = TMTVRMP_12

|     | TV_RAMP_TMR = TMTVRMP_12

GR_DS_TV = 2 -----|     | TV_RAMP = TVRMP_12

|     | FLG_TRIG_RMP = 0

FLG_SPK_TQM = 1 --|     |

|

|

| --- ELSE ---

FLG_TRIG_RMP = 1 -|     |

|     |

FLG_SFT_UP = 1 ---|AND -| TVRMPTM = TMTVRMP_23

|     | TV_RAMP_TMR = TMTVRMP_23

GR_DS_TV = 3 -----|     | TV_RAMP = TVRMP_23

|     | FLG_TRIG_RMP = 0

FLG_SPK_TQM = 1 --|     |

|

|

| --- ELSE ---

(continued on next page)

27-39

ELECTRONIC PRESSURE CONTROL, DYNAMIC EPC CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

FLG_TRIG_RMP = 1 -|     |

|     |

FLG_SFT_UP = 1 ---|AND -| TVRMPTM = TMTVRMP_34

|     | TV_RAMP_TMR = TMTVRMP_34

GR_DS_TV = 4 -----|     | TV_RAMP = TVRMP_34

|     | FLG_TRIG_RMP = 0

FLG_SPK_TQM = 1 --|     |

FLG_SFT_IN = 1 ---------| TV_DYN = TV_DYN + TV_RAMP *

|          [(TVRMPTM - TV_RAMP_TMR) / TVRMPTM]

|

| --- ELSE ---

|

| No action

Compensate for 4X4L operation:

FLG_4X4L = 0 ------------------------------| No Action

|

| --- ELSE ---

|

GEAR_CUR = 1 ------------------------------| TV_DYN = TV_DYN + TV_4L_21

|

| --- ELSE ---

GEAR_CUR = 2 ------------------|           |

|AND -------| TV_DYN = TV_DYN + TV_4L_12

FLG_SFT_UP = 1 ----------------|           |

| --- ELSE ---

|

GEAR_CUR = 2 ------------------------------| TV_DYN = TV_DYN + TV_4L_32

|

| --- ELSE ---

GEAR_CUR = 3 ------------------|           |

|AND -------| TV_DYN = TV_DYN + TV_4L_23

FLG_SFT_UP = 1 ----------------|           |

| --- ELSE ---

|

GEAR_CUR = 3 ------------------------------| TV_DYN = TV_DYN + TV_4L_43

|

| --- ELSE ---

|

always ------------------------------------| TV_DYN = TV_DYN + TV_4L_34

27-40

ELECTRONIC PRESSURE CONTROL, EPC CURRENT MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

27.7  EPC VFS CONTROL AND SELF TEST (CDAG0)

OVERVIEW

The Electronic Pressure Control (EPC) Variable  Force  Solenoid  (VFS)  is  a

current control device with a 0 to 1 amp control range.  One amp produces the

lowest line pressure (handles idle torque capacity).  Current to the  VFS  is

controlled  by  a  custom  integrated  circuit which converts an 8 bit binary

value to a proportional current using the following relationship:

I ave. = requested counts/255

This 8 bit word is clocked into the IC via two low speed  output  lines  from

the DUCE chip called data and clock.

The EPC VFS control and self test module contains all of the logic  necessary

to convert a desired pressure into a required current, output this current to

the VFS, and test the EPC VFS control circuit during continuous and on-demand

self test (based on the overcurrent circuit monitor).

This module is called from the EPC_GUIDE after TV_PRES_BAR (desired EPC)  has

been  calculated.   Required  current  to the VFS is then calculated based on

TV_PRES_BAR and current vehicle operating conditions:

ON-DEMAND SELF TEST - During on demand self test  TV_COUNTS  is  calculated

based  on  desired  vfs  output  (TV_PRES_BAR)  except  under the following

conditions:

CCM Engine Off test in progress (CCM_EO_ENA = 1)

TV_COUNTS = KOEO_TV_CNTS unless the EPC current monitor is  attempting

to  distinguish between an OPEN and a SHORT circuit (ETV_TEST = 1), in

which case TV_COUNTS = 0.

Output cycling in progress

Upon completion of Engine Off self test, the technician  can  initiate

output  cycling  to aid diagnostics (track faults through the wiring).

During output cycling:

OSM_EO_ON = 1 --> TV_COUNTS = KOEO_TV_CNTS

OSM_EO_OFF = 1 --> TV_COUNTS = 0

NORMAL OPERATION - TV_COUNTS is calculated  based  on  desired  vfs  output

(TV_PRES_BAR),  unless the EPC current monitor is attempting to distinguish

between an OPEN and a SHORT circuit failure (ETV_TEST = 1), in  which  case

TV_COUNTS = 0.

TV_COUNTS will be output to the DUCE chip during the 1  millisecond  repeater

if  a new value has been calculated (this is due to the way the DUCE channels

are updated, and is NOT the result of the EPC VFS needing to be controlled in

foreground).

27-41

ELECTRONIC PRESSURE CONTROL, EPC CURRENT MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

All EPC VFS self test is  based  on  the  EPC  Overcurrent  Circuit  Monitor.

EPCOCM1 is a voltage representative of the current in the EPC solenoid.  This

voltage is read by the EEC-IV through the A/D (see  circuit  diagram  below).

The  expected  monitor  voltage is a function of battery voltage and solenoid

current.  Measured voltage decreases with increasing  current  or  decreasing

battery voltage.

Vbat

|

___|___

|     |                    Rpull-up = 6.81K ohms

\     \                    R236 = 1 ohm

Rpull-up /     /Rload (VFS)         R287 = 61.9K ohms

\     \                    R288 = 26.7K ohms

|_____|                    Rload = hardware dependent,

|      R236                     approx. 3-5 ohms

Vn|_____/\/\/\__________

|                     | Note: all resistance values

|                     |       approximate.

\                   |/

R287 /   __          Q1  |

\  |             ---|

|__|A/D CONV        |

|  |(EPCOCM1)       |\

\  |__                |

R288 /                     |

__\__                 __|__

___                   ___

_                     _

Due to the presence of the pull-up resistor, an open circuit will look like a

short  to  ground  for  all non-zero TV_COUNTS values (non-zero TV_COUNTS ==>

transistor Q1 will be on).

If the measured voltage is less than the allowable minimum (etv_ocm_min),  an

"EPC Test" sequence will be initiated.  This sequence commands zero TV_COUNTS

and then rechecks the voltage to differentiate an open from a short.  Through

the  use  of  a  "pull-up"  circuit  connected  to  the  EPC  output, at zero

TV_COUNTS, a short will result in low voltage on EPCOCM1, while an open  will

show  a  near-normal  voltage (see analysis below).  If an error is detected,

the appropriate error code  (error  code  1746  -  EPC  solenoid/driver  open

circuit -or- code 1747 - EPC solenoid/driver short to ground) is set.

27-42

ELECTRONIC PRESSURE CONTROL, EPC CURRENT MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EPC Circuit Steady-State Analysis

_____________________________________________________________________

|          OPEN CIRCUIT             |    SHORT TO GROUND              |

|___________________________________|_________________________________|

| Q1 ON (TV_COUNTS <> 0):           | Q1 ON (TV_COUNTS <> 0):         |

|                                   |                                 |

|  VEPCOCM1 = Vn*[R288/(R287+R288)] | VEPCOCM1 = Vn*[R288/(R287+R288)]|

|                                   |                                 |

|        Vbat*R236||(R287+R288)     |   Vn is approximately zero      |

| Vn = ---------------------------- |                                 |

|       Rpull-up + R236||(R287+R288)| VEPCOCM1 will be a small value  |

|    R236 = 1 ohm                   |                                 |

|         |                         |                                 |

|         |                         |                                 |

|         v                         |                                 |

|    R236||(R287+R288) will be less |                                 |

|    than 1 ohm                     |                                 |

|         |                         |                                 |

|         |                         |                                 |

|         v                         |                                 |

|        Vbat*(approx. 1)           |                                 |

|  Vn = -----------------           |                                 |

|          6.81K + 1                |                                 |

|         |                         |                                 |

|         |                         |                                 |

|         v                         |                                 |

|  VEPCOCM1 will be a small value   |                                 |

|   (similar to an short circuit)   |                                 |

|___________________________________|_________________________________|

| Q1 OFF (TV_COUNTS = 0):           | Q1 OFF (TV_COUNTS = 0):         |

|                                   |                                 |

| VEPCOCM1 = Vn*[R288/(R287+R288)]  | VEPCOCM1 = Vn*[R288/(R287+R288)]|

|                                   |                                 |

|        Vbat*(R287+R288)           |  Vn is approximately zero       |

| Vn = ------------------------     |                                 |

|       Rpull-up + (R287+R288)      | VEPCOCM1 will be a small value  |

|         |                         |                                 |

|         |                         |                                 |

|         v                         |                                 |

|  VEPCOCM1 will be a near normal   |                                 |

|  voltage for zero TV_COUNTS,      |                                 |

|  making it possible to distinguish|                                 |

|  between an open and a short.     |                                 |

|___________________________________|_________________________________|

27-43

ELECTRONIC PRESSURE CONTROL, EPC CURRENT MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Registers:

-  EPC_DLY_TMR = EPC test delay timer.

-  EPCOCM1 = EPC VFS overcurrent circuit monitor, counts.

-  P1747FIL = EPC short circuit failure fault filter.

-  P1746FIL = EPC open circuit failure fault filter.

-  SPD_RATIO = Speed ratio across torque converter (output/input).

-  TOT = Transmission oil temperature (degrees F).

-  TQ_NET = Net torque into torque converter.

-  TSTRAT  =  Transmission  strategy  switch;  The  TSTRAT  software  switch

controls which transmission control strategy is executed:

0 -> No transmission control (Manual trans., AOD, ATX, C6, C3, etc.)

1 -> SIL (Shift indicator light)

2 -> A4LD with 3-4 shift control and converter clutch control

3 -> AXOD

4 -> C64E (E4OD)

5 -> A4LDE

6 -> FAX-4

7 -> AOD-E (AOD-I)

8 -> 4EAT

9 -> CD4E

-  TV_COUNTS = Required counts value  to  produce  desired  EPC  VFS  output

pressure.

-  TV_COUNT_LST = TV pressure counts last update.

-  TV_PRES_BAR = Filtered desired EPC, calculated in the EPC GUIDE.

-  VBAT = Battery voltage.

Bit Flags:

-  CCM_EO_ENA = Comprehensive component monitor engine off test enable flag;

1 -> KOEO CCM tests are enabled.

-  CCM_TST_ENA = Comprehensive component monitor test enable flag; 1 ->  CCM

tests are enabled.

-  ETV_TEST = ETV open/short test in progress.

-  OSM_EO_OFF = Output State Monitor KOEO test actuator for OFF state  flag;

1 -> turn actuator off and monitor the output.

-  OSM_EO_ON = Output State Monitor KOEO test actuator for ON state flag;  1

-> turn actuator on and monitor the output.

27-44

ELECTRONIC PRESSURE CONTROL, EPC CURRENT MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  EPC_CM_ENA = Calibration switch to enable EPC current monitor test.

-  EPC_DLY_TIM = EPC test transient delay time.

-  ETV_BIAS = EPCOCM1 voltage at 10V VBAT and 0 TV_COUNTS.

-  ETV_GAIN = EPCOCM1 voltage gain/one count TV_COUNTS.

-  ETV_GAIN_BAT = EPCOCM1 count gain/one volt VBAT.

-  FN617 = Torque converter torque ratio.

-  FN620 = EPC VFS transfer function, used  to  converted  desired  pressure

(TV_PRES_BAR) into counts.

-  FN622 = Temperature dependent VFS transfer function modifier.

-  FN622A = Temperature dependent VFS transfer function multiplier.

-  KOEO_TV_CNTS = Commanded value of TV_COUNTS during KOEO self test.

-  OFMTQMAX_CL = Torque below which normal  fire  is  requested  during  ETV

overcurrent.

-  OFMTQMAX_SH = Torque above which alternate fire is requested  during  ETV

overcurrent.

-  P1747LVL = EPC short circuit failure fault threshold.

-  P1747UP = EPC short circuit failure upcount.

-  P1746LVL = EPC open circuit failure fault threshold.

-  P1746UP = EPC open circuit failure upcount.

-  TVCDLT = Maximum change in TV counts to consider ETV as steady state.

-  VBAT_MIN = Minimum battery voltage to allow EPC test entry.

27-45

ELECTRONIC PRESSURE CONTROL, EPC CURRENT MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OUTPUTS

Registers:

-  P1747FIL = See above.

-  P1746FIL = See above.

-  EPC_DLY_TMR = See above.

-  ETV_OCM_MN_D = *DISPLAY ONLY* expected ETV current.

-  TV_COUNTS = See above.

-  TV_COUNT_LST = See above.

Bit Flags:

-  ETV_TEST = See above.

-  OFMFLG = ETV solenoid failure flag.

-  OFM_FMEM = ETV overcurrent  based  alternate  fire  request  flag;  1  ->

Alternate fire requested, 0 -> Normal fire.

-  PxxxMALF = OBDII malfunction flag for fault XXX; 1 -> Fault XXX currently

exits.

-  PxxxMON = OBDII monitor flag for fault XXX;  1  ->  Fault  XXX  has  been

monitored this power-up.

OTHER

-  epc_codes = SET OF {P1746, P1747} the  set  of  OBDII  fault  codes  that

relate to the EPC VFS.

-  clear_malf(Pxxx) = Clears malfunction flag specified by Pxxx.

-  ccm_malfunction{Pxxx} = Logic process, imported from MIL control  module;

Pxxx indicates a fault code.

-  store_code(Pxxx) = Stores fault code specified by Pxxx.

-  P1746 = Fault code; EPC VFS open circuit.

-  P1747 = Fault code; EPC VFS short to ground.

27-46

ELECTRONIC PRESSURE CONTROL, EPC CURRENT MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EPC_VFS_OBDII_COM11

This logic calculates TV_COUNTS based on desired vfs output (TV_PRES_BAR) and

current  vehicle  operating  conditions  (see  module  overview  for  a brief

description):

TSTRAT <= 3 ----------------------------| Exit module

(Manual/non-electric transmission)     |

| --- ELSE ---

|

| TV_COUNT_LST = TV_COUNTS

| (save previous value of TV_COUNTS)

OSM_EO_ON = 1 --------------|

(Cycle output on           |

requested)               |OR --|

|     |

CCM_EO_ENA = 1 -------------|     |

(Engine Off CCM in               |AND -| TV_COUNTS = KOEO_TV_CNTS

progress)                       |     |

|     |

ETV_TEST = 0 ---------------------|     |

(EPC open/short test not               |

in progress)                          |

| --- ELSE ---

OSM_EO_OFF = 1 -------------------|     |

(Cycle output off                |     |

requested)                      |OR --| TV_COUNTS = 0

|     |

ETV_TEST = 1 ---------------------|     |

(EPC open/short test in                |

progress)                             |

| --- ELSE ---

|

| tv_comp = TV_PRES_BAR * FN622A(TOT)

| TV_COUNTS = max [FN620(tv_comp) +

|   FN622(TOT),0]

unconditionally ------------------------| output(TV_COUNTS)

| (Output TV_COUNTS

|  to the EPC VFS)

27-47

ELECTRONIC PRESSURE CONTROL, EPC CURRENT MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

This logic delays epc test entry (when enabled) for a calibratable amount  of

time when transients (large changes in desired current) occur.

CCM_EO_ENA = 0 -------------|

(Engine Off CCM disabled)  |AND -|

|     |

CCM_TST_ENA = 0 ------------|     |

(Continuous CCM disabled)        |

|

EPC_CM_ENA = 0 -------------------|OR --| EPC_DLY_TMR = 0

(Test cal'ed out)                |     |

|     | EXIT MODULE

ABS(TV_COUNT_LST -                |     | (Conditions not met

TV_COUNTS) > TVCDLT -------|     |  to monitor EPC circuit,

(Transient occurred)             |     |  zero transient delay

|     |  timer and exit module)

VBAT < VBAT_MIN ------------------|     |

(Battery voltage too low to monitor    |

EPC VFS circuit)                      |

| --- ELSE ---

|

EPC_DLY_TMR >= EPC_DLY_TIM -------------| Do: calculate_min_epcocm1

(Transient delay is over)              | Do: epc_current_monitor

|

| --- ELSE ---

|

| EXIT

27-48

ELECTRONIC PRESSURE CONTROL, EPC CURRENT MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: calculate_min_epcocm1

Calculate the minimum allowed value of EPCOCM1 based on the  current  vehicle

operating conditions.

always ----------------| etv_ocm_min = [ETV_BIAS - (ETV_GAIN*TV_COUNTS)] +

|   [(VBAT - 10.0) * ETV_GAIN_BAT]

|  (clip to zero)

| ETV_OCM_MN_D = etv_ocm_min

|  (save for display)

END: calculate_min_epcocm1

27-49

ELECTRONIC PRESSURE CONTROL, EPC CURRENT MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: epc_current_monitor

EPCOCM1 < etv_ocm_min ------------|

(EPCOCM1 below allowed minimum)  |AND -|  (initiate open/short test)

|     | OFMFLG = 0

ETV_TEST = 0 ---------------------|     | ETV_TEST = 1

(epc test sequence is not in progress) | tq_ofm = 0

|

| --- ELSE ---

EPCOCM1 < etv_ocm_min ------------|     |

(EPCOCM1 low - short circuit)    |AND -|  (over current/short

|     |   circuit)

ETV_TEST = 1 ---------------------|     | P1747FIL = P1747FIL + P1747UP

(epc test sequence is in progress)     | OFMFLG = 1

| tq_ofm = TQ_NET *

|   FN617(SPD_RATIO)

|  (torque into the gear set)

| Do: epc_monitor_logic

|

| --- ELSE ---

EPCOCM1 >= etv_ocm_min -----------|     |

(EPCOCM1 high - open circuit)    |AND -|  (under current/open

|     |   circuit)

ETV_TEST = 1 ---------------------|     | P1746FIL = P1746FIL + P1746UP

(epc test sequence is in progress)     | OFMFLG = 0

| ETV_TEST = 0

| tq_ofm = 0

| Do: epc_monitor_logic

|

| --- ELSE ---

|

|  (no error)

| P1747FIL = P1747FIL - 1

| P1746FIL = P1746FIL - 1

| OFMFLG = 0

| ETV_TEST = 0

| tq_ofm = 0

|

| Do: epc_monitor_logic

tq_ofm >= OFMTQMAX_SH ------------|S Q -| OFM_FMEM

|     | (torque limit exceeded

tq_ofm < OFMTQMAX_CL -------------|C    |  alternate fire)

END: epc_current_monitor

27-50

ELECTRONIC PRESSURE CONTROL, EPC CURRENT MONITOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: epc_monitor_logic

This logic sets the epc circuit test monitor and malfunction flags  based  on

the current circuit status.

P1746FIL >= P1746LVL -------------------| store_code(P1746)

| (epc open circuit

|  malfunction present)

|

| --- ELSE ---

|

P1746FIL = 0 ---------------------------| clear_malf(P1746)

| (no epc open circuit

|  malfunction presnet)

P1747FIL >= P1747LVL -------------------| P1747MON = 1

| malfunction(ccm,P1747)

| (epc short to ground

|  malfunction present)

|

| --- ELSE ---

|

P1747FIL = 0 ---------------------------| P1747MON = 1

| clear_malf(P1747)

| (epc circuit has been

|  monitored for a short

|  circuit, no malfunction

|  currently present)

END: epc_monitor_logic

27-51

ELECTRONIC PRESSURE CONTROL, TRANSMISSION OVERTEMPERATURE TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

27.8  TRANSMISSION OVERTEMPERATURE TEST (CDAN0)

27.8.1  PROCEDURE Transmission_overtemp_test

BEGIN transmission_overtemp_test

FUNCTIONAL REQUIREMENTS

At very high  temperatures,  the  torque  capacity  of  the  transmission  is

decreased,  which  can  impact  its  durabilty.  When a high transmission oil

temperature is reached, the malfunction flag is set (P1783MALF)  and  failure

mode  action  is initiated.  The failure mode flag (OTEMP_FM_FLG) will remain

set  until  the   transmission   drops   below   the   overtemperature   exit

(OTEMP_EXIT_S) value.

LOGICAL REQUIREMENTS

Logic Diagram

This structure diagram shows calling relations between logic charts  in  this

process.

LEVEL     ENTITY CALLED

1       Chart transmission_overtemp_test_0

2          Function clear-malf

2          Function store_code

EXTERNAL INTERFACE

Inputs:

-  CCM_TST_ENA

-  FFG_TOT

-  OTEMP_EXIT_S

-  OTEMP_TIM

-  P1783

-  TOT

-  TOT_OTEMP_S

-  TSTRAT

Outputs:

No parameters are applicable.

PARAMETER DECLARATIONS

Registers:

-  TOT = TRANSMISSION OIL TEMPERATURE (DEGREES F)

Resolution: 0.12500000  Units: --               Init. Value: 60

Min. Value: -4096.0000  Max. Value: 4095.87500  Reg. Type: RAM

Logic Chart transmission_overtemp_test_0

STRATEGY FILE: EPC_OTEMP_TEST_OBDII_COM1/GEN=13

27-52

ELECTRONIC PRESSURE CONTROL, TRANSMISSION OVERTEMPERATURE TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TSTRAT <= 3 ---------------------------| Exit module

;Manual / non - electric transmission |

|

|  --- ELSE ---

|

CCM_TST_ENA = 0 -----------------|     | OTEMP_TMR := 0

;CCM testing disabled           |     |  ;Conditions to run the test not

|     |  ;met, or TOT is below the overtemp

FFG_TOT = 1 ---------------------|OR --|  ;value - > zero the error timer

;TOT unreliable                 |     |

|     |

TOT <= TOT_OTEMP_S --------|     |     |

;NOT overtemperature      |AND -|     |

|           |

OTEMP_FM_FLG = 0 ----------|           |

;NOT in otemp fmem                    |

|

|  --- ELSE ---

|

TOT <= OTEMP_EXIT_S -------------------| DO: clear_malf(P1783)

;In fmem AND TOT low enough to exit   |  ;transmission temperature is okay,

|  ;clear the malfunction flag

|

| OTEMP_FM_FLG := 0

|  ;exit FMEM

|

|  --- ELSE ---

|

OTEMP_TMR > OTEMP_TIM -----------------| DO: store_code(P1783)

;Error present long enough            |

| OTEMP_FM_FLG := 1

|  ;malfunction detected, set flag

|  ;and enter FMEM)

|

END:  transmission_overtemp_test

STRATEGY FILE: EPC_OTEMP_TEST_OBDII_COM1/GEN=13

27-53

TRANSMISSION INPUT CONVERSIONS, ANTI-SILTING ENABLING LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

27.8.2  ANTI-SILTING ENABLING LOGIC (CDAN0)

DEFINITIONS

Registers:

-  PDL = PRNDL position - from A to D conversion.

-  TMR_ASILT = Anti-silting timer.

-  VSCTR = Counter which tracks unrealistic changes in vehicle speed.

-  VS = Unfiltered vehicle speed.

Bit Flags:

-  BIFLG = Brake input flag.

-  FLG_ASILT = Anti-silting strategy enabled;  1  ->  anti-silting  strategy

enabled.

-  FLG_ASILT_LS = Anti-silting strategy enabled last pass; 1 -> anti-silting

strategy enabled last pass.

-  FLG_ENG_IN = Engagement in progress status; 1 -> engagement in progress.

-  FLG_ENG_LS = Engagement in progress status last pass; 1 -> engagement  in

progress last pass.

-  FLG_FWD_REV = Direction change flag; 1 -> direction changed.

-  FLG_TP_CT = Closed throttle flag; 1 -> at closed throttle.

-  FLG_TR_ENG = Transmission engagement  status;  1  ->  transmission  fully

engaged.

-  TFMFLG = Throttle position sensor failed; 1 -> TP sensor failed.

-  VSFMFLG = Vehicle speed sensor failed; 1 -> Vehicle speed sensor failed.

Calibration Constants:

-  ASILT_BR_SW = Calibration switch to enable anti-silting when  vehicle  is

at rest with the brake on; 1 -> anti-silting logic on.

-  TM_ASILT = Length of time for each stage of anti-silting cycle.

27-54

TRANSMISSION INPUT CONVERSIONS, ANTI-SILTING ENABLING LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EPC_ASILT_COM1

always ---------------------------------| FLG_ASILT_LS = FLG_ASILT

PDL <= 4 -----------------|

|OR --|

PDL = 6 ------------------|     |

|

FLG_FWD_REV = 0 ----------------|AND -| FLG_TR_ENG = 1

|     |

FLG_ENG_IN = 0 -----------------|     |

|     |

FLG_ENG_LS = 1 -----------------|     |

| --- ELSE ---

|

PDL = 5 ------------------------|     |

|     |

FLG_FWD_REV = 1 ----------------|OR --| FLG_TR_ENG = 0

|

PDL = 7 ------------------------|

TFMFLG = 0 -----------------------|

|

VSFMFLG = 0 ----------------------|

|

FLG_TR_ENG = 1 -------------------|

|

VSCTR = 0 ------------------------|

|AND -| FLG_ASILT = 1

FLG_TP_CT = 1 --------------------|     |

|     |

VS = 0 ---------------------------|     |

|     |

BIFLG = 1 ------------------------|     |

|     |

ASILT_BR_SW = 1 ------------------|     |

| --- ELSE ---

|

| FLG_ASILT = 0

FLG_ASILT_LS = 0 -----------|

|AND -|

FLG_ASILT = 1 --------------|     |OR --| TMR_ASILT = 0

|     |  (reset anti-silting timer)

TMR_ASILT > 5 * TM_ASILT ---------|

27-55

TRANSMISSION INPUT CONVERSIONS, ANTI-SILTING ENABLING LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

27-56

CHAPTER 28

CONVERTER CLUTCH CONTROL

28-1

CONVERTER CLUTCH CONTROL, BYPASS CLUTCH CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

28.1  BYPASS CLUTCH CONTROL (CDAM0)

OVERVIEW

The modulated bypass clutch control module is the main  calling  routine  for

modulated  bypass  clutch  strategy.  This module provides the order in which

all other modules are called.

DEFINITIONS

Calibration Constants:

-  SWBCTST = Modulated Bypass Clutch control test switch.

PROCESS

STRATEGY MODULE: CCC_BYP_CLTCH_CTL_COM6

SWBCTST > 0 -----------| Do: CCC_BYP_CLTCH_TESTS_COMx

| EXIT MODULATED BYPASS CONVERTER CLUTCH CONTROL

|

| --- ELSE ---

|

| Do: CCC_FMEM_COM3

| Do: CCC_HOT_LCK_COMx

| Do: CCC_SCHLD_LCK_UNLCK_COMx

| Do: CCC_UNCOND_UNLK_COMx

| Do: CCC_MOD_COMx

| Do: CCC_SCHLD_BYP_SLIP_COMx

| Do: CCC_BYP_SLIP_CALC_COMx

| Do: CCC_TCAP_DETR_COMx

| Do: CCC_TCAP_CONV_COMX

28-2

CONVERTER CLUTCH CONTROL, HOT LOCKUP LOGIC - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

28.2  HOT LOCKUP LOGIC (CDAA0)

OVERVIEW

The hot lockup logic uses ECT or TOT to set an  overtemperature  lockup  mode

flag.   The intent of the overtemperature lockup mode is to reduce the amount

of transmission generated heat by hardlocking the torque converter,  thereby,

increasing  its  efficiency.   Since  driveability decreases substantially in

this mode, it is used only when absolutely required, typically  when  TOT  is

greater than 275 deg.  F.

DEFINITIONS

INPUTS

Registers:

-  ECT = Engine Coolant Temperature, deg.  F.

-  TOT = Transmission Oil Temperature, deg.  F.

Calibration Constants:

-  LUHECT_CL = ECT below which overtemp lockup mode can be disabled.

-  LUHECT_SH = ECT above which overtemp lockup mode always enabled.

-  LUHTOT_CL = TOT below which overtemp lockup mode can be disabled.

-  LUHTOT_SH = TOT above which overtemp lockup mode always enabled.

OUTPUTS

Bit Flags:

-  FLG_OT_LK = Transmission overtemp lockup mode flag; 1 ->  overtemperature

mode.

PROCESS

STRATEGY MODULE: CCC_HOT_LCK_COM1

ECT >= LUHECT_SH ------------|

|OR --| FLG_OT_LK = 1

TOT >= LUHTOT_SH ------------|     |  (overtemp lockup mode)

|

| --- ELSE ---

ECT < LUHECT_CL -------------|     |

|AND -| FLG_OT_LK = 0

TOT < LUHTOT_CL -------------|

28-3

CONVERTER CLUTCH CONTROL, SCHEDULED BYPASS CLUTCH SLIP - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

28.3  SCHEDULED BYPASS CLUTCH SLIP (CDAN0)

OVERVIEW

The Scheduled Bypass  Clutch  Slip  module  determines  the  amount  of  slip

(SLIP_TARGET)  requested  by  the calibrator based on tables of vehicle speed

and throttle position.  Unique tables are  provided  for  2nd,  3rd  and  4th

gears.   There  are  three  unique  altitude  adjustment  tables as well.  In

addition, an overtemp lockup mode table provides the ability to  hardlock  to

increase  converter  efficiency in cases of extreme heat.  No 1st gear tables

are provided since it was felt that no one would schedule slip in 1st gear.

The slip tables  can  have  values  between  0  and  1,020  rpm  with  4  rpm

resolution.  Slip greater than 1,020 rpm can be commanded by setting MAXSLIP1

greater than 1,020.  The final value  of  target  slip  also  determines  the

control mode.

A slip target between 0 and 4 rpm is normally considered hardlock mode,  that

is,  essentially no slip is allowed.  The controller will increase duty cycle

to achieve no slip.  Hardlock  mode  is  not  recommended  for  transmissions

without  a  damper.  Hardlock mode can be calibrated out by setting SLPHRD to

0.

When the PRNDL is in the D position ( overdrive lock-out mode),  hardlock  is

commaned  above  a throttle position break-point (TP3LCK).  This is to insure

transmission durability when towing trailers.

A slip target between 4 and 500 rpm is normally considered engage mode,  that

is,  the  controller  controls  slip  in  a closed loop manner to the desired

value.

DEFINITIONS

Registers:

-  ACT = Air charge temperature, degree F.

-  ECT = Coolant temp, degree F.

-  GR_CM = Current transmission gear.

-  GR_DS_TV = Commanded gear for TV.

-  PDL = PRNDL position - from A to D conversion.

-  SLIP_TARGET = Target slip value from tables, rpm.

-  TM_VER_SFT = Time to verify automatic desired shift.

-  TOT = Transmission Oil Temperature (Degrees Fahrenheit).

-  TP_REL = Relative Throttle Position, counts.

28-4

CONVERTER CLUTCH CONTROL, SCHEDULED BYPASS CLUTCH SLIP - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Bit Flags:

-  FLG_AC_CTL = Flag indicating when to use A/C comp.  slip target; 1 -> use

target.

-  FLG_CLUP = Flag indicating closed loop/bcsdc freeze region for upshifts.

-  FLG_CRV_LK = Scheduled curve lockup flag.

-  FLG_DIS_CSM = Disable continuous slip mode flag;  0  ->  continuous  slip

enabled, 1 -> continuous slip disabled.

-  FLG_DNS_CTL = Downshift slip modulation flag.

-  FLG_ENG_LK = Modulated bypass clutch engage mode flag;  1  ->  in  engage

mode.

-  FLG_FMM_LK = Failure Mode Management lock-up flag; 1  ->  lock  converter

due to FMEM action.

-  FLG_HRD_EN = Base table slip_target value indicates HARDLOCK mode.

-  FLG_HRD_LK = Modulated bypass clutch hardlock mode flag; 1 -> in hardlock

mode.

-  FLG_HRD_LST = Last pass value of FLG_HRD_LK used to detect transitions.

-  FLG_HT_MOD = Flag indicating high temperature slip modulation allowed;  1

-> modulation allowed, 0 -> not allowed.

-  FLG_OPEN = MLUS is in open mode; no capacity when set.

-  FLG_OPEN_LST = Last pass value of FLG_OPEN used to detect transitions.

-  FLG_OT_LK = Transmission overtemperature lockup mode flag.

-  FLG_TP3_LCK = Flag indicating TP_REL is in hardlock zone  for  3rd  gear,

PDL = 3.

-  FLG_UNC_UNLK = Converter clutch unconditional unlock flag.

-  FLG_UNLK_DS = Converter unlock in progress flag.

-  SFT_ERR_FLG = Flag indicating that one shift has failed.

Calibration Constants:

-  ACSLIP = Desired slip target for AC Clutch engagement.

-  FNDN2SSLP_0 = Slip adder prior to commanding a downshift to  1st  or  2nd

gear.

-  FNDN3SSLP_0 = Slip adder prior to commanding a downshift to 3rd gear.

-  MAXSLIP1 = Maximum attainable slip target value, rpm.

-  MODACT_CL = ACT below which slip modulation is not allowed.

28-5

CONVERTER CLUTCH CONTROL, SCHEDULED BYPASS CLUTCH SLIP - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  MODACT_SH = ACT above which slip modulation is allowed.

-  MODECT_CL = ECT below which slip modulation is not allowed.

-  MODECT_SH = ECT above which slip modulation is allowed.

-  MODTOT_CL = TOT below which slip modulation is not allowed.

-  SLIP_DES_MIN = SLIP_TARGET while verifying shift.

-  SLIP_HRD_SL = SLIP_TARGET to set hardlock.

-  SLIP_HRD_CH = SLIP_TARGET to clear hardlock.

-  TP3LCK = TP_REL above which hardlock is commanded in 3rd gear, PDL = 3.

-  TP3LCK_HYS = Hysteresis term for PDL = 3, 3rd gear hardlock condition.

PROCESS

STRATEGY MODULE: CCC_SCHLD_BYP_SLIP_COM5

FLG_DIS_CSM = 1 ---------------|

(continous slip mode disabled)|

|

FLG_UNC_UNLK = 1 --------------|

|OR --| FLG_HT_MOD = 0

ACT < MODACT_CL ---------------|     |  (exit high temp. modulation

|     |   region)

ECT < MODECT_CL ---------------|     |

|     |

TOT < MODTOT_CL ---------------|     |

| --- ELSE ---

ACT >= MODACT_SH --------------|     |

|AND -| FLG_HT_MOD = 1

ECT >= MODECT_SH --------------|     |  (enter high temp. modulation

|   region)

TP_REL >= TP3LCK --------------|S Q -| FLG_TP3_LCK

|     | (Allow hardlock in 3rd gear

TP_REL < TP3LCK - TP3LCK_HYS --|     |  when PDL = 3)

28-6

CONVERTER CLUTCH CONTROL, SCHEDULED BYPASS CLUTCH SLIP - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLG_CRV_LK = 1 ----------------|

|

SFT_ERR_FLG = 1 ---------|     |

(hardlock after shift   |     |

solenoid functional    |     |

test detects one bad   |     |

shift)                 |     |

|     |

FLG_FMM_LK = 1 ----------|     |

(lock requested due to  |     |

FMEM action)           |     |

|     |AND -| slip_target = 0

FLG_OT_LK = 1 -----------|     |     |

|     |     |

FLG_HT_MOD = 0 ----------|OR --|     |

(no high temperature    |           |

modulation)            |           |

|           |

PDL = 3 -----------|     |           |

|     |           |

GR_CM = 3 ---------|AND -|           |

|                 |

FLG_TP3_LCK = 1 ---|                 |

(command hardlock when TP_REL        |

is above TP3LCK and PDL = 3)        |

| --- ELSE ---

FLG_CRV_LK = 1 ----------------|     |

|AND -| slip_target1 = FN[GR_CM]SSLP

GR_CM <> 1 --------------------|     |

| --- ELSE ---

|

| slip_target1 = MAXSLIP1

slip_target1 <= SLIP_HRD_SL ---------| FLG_HRD_EN = 1

|

| --- ELSE ---

|

slip_target1 >= SLIP_HRD_CH ---------| FLG_HRD_EN = 0

always ------------------------------| FLG_OPEN_LST = FLG_OPEN

| FLG_HRD_LST = FLG_HRD_LK

28-7

CONVERTER CLUTCH CONTROL, SCHEDULED BYPASS CLUTCH SLIP - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLG_CRV_LK = 0 ----------------------| FLG_OPEN = 1

| FLG_ENG_LK = 0

| FLG_HRD_LK = 0

| slip_target2 = MAXSLIP1

|

| --- ELSE ---

FLG_HRD_EN = 1 ----------------|     |

|     |

TM_VER_SFT = 0 ----------------|     |

|     |

FLG_UNLK_DS = 0 ---------------|AND -| FLG_OPEN = 0

|     | FLG_ENG_LK = 0

FLG_CLUP = 0 ------------------|     | FLG_HRD_LK = 1

|     | slip_target2 = 0

FLG_DNS_CTL = 0 ---------------|     |

|     |

FLG_AC_CTL = 0 ----------------|     |

| --- ELSE ---

TM_VER_SFT > 0 ----------------|     |

|     |

FLG_UNLK_DS = 1 ---------------|OR --| FLG_OPEN = 0

|     | FLG_ENG_LK = 1

FLG_CLUP = 1 ------------------|     | FLG_HRD_LK = 0

| slip_target2 =

|   max(slip_target1,SLIP_DES_MIN)

|

| --- ELSE ---

|

FLG_AC_CTL = 1 ----------------------| FLG_OPEN = 0

| FLG_ENG_LK = 1

| FLG_HRD_LK = 0

| slip_target2 =

|   max(ACSLIP,slip_target1)

|

| --- ELSE ---

|

| FLG_OPEN = 0

| FLG_ENG_LK = 1

| FLG_HRD_LK = 0

| slip_target2 = slip_target1

28-8

CONVERTER CLUTCH CONTROL, SCHEDULED BYPASS CLUTCH SLIP - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLG_UNLK_DS = 1 ---------------|

|AND -| slip_target3 = slip_target2 +

GR_DS_TV <= 2 -----------------|     |                 FNDN2SSLP_0

|  (4-3 downshift slip control)

|

| --- ELSE ---

|

FLG_UNLK_DS = 1 ---------------------| slip_target3 = slip_target2 +

|                 FNDN3SSLP_0

| (downshift slip control)

|

| --- ELSE ---

|

| slip_target3 = slip_target2

always ------------------------------| SLIP_TARGET = min(slip_target3,

|                   MAXSLIP1)

28-9

CONVERTER CLUTCH CONTROL, UNCONDITIONAL UNLOCK LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

28.4  UNCONDITIONAL UNLOCK LOGIC (CDAN0)

OVERVIEW

The Unconditional Unlock Logic module is designed  to  unlock  the  converter

clutch  without  regard  to  the schedule or any other shift control function

taking place, hence the name, unconditional.  Typical  unconditional  unlocks

are:  closed throttle, high tip in or tip out rates, brake, cold engine, high

altitude or time since start of an unlocked  shift.   If  any  of  the  above

conditions  are  present,  the  timer TM_LK_DLY is loaded with a unique value

that must expire before normal converter  clutch  is  allowed.   If  multiple

conditions  are  present,  the longest delay time is used.  All unconditional

unlocks result in 0% duty cycle to the bypass clutch.

Overtemp lock up mode eliminates some unconditional unlocks in an attempt  to

keep  the  converter  hardlocked  as  much  as possible.  This does, however,

degrade driveability.

DEFINITIONS

Registers:

-  BP = Barometric pressure.

-  CS_SFT_MULT = Cold start shift multiplier.

-  ECT = Engine coolant temperature.

-  GR_CM = Current transmission gear.

-  GR_OLD = Last commanded gear.

-  NEBART = Filtered engine speed, RPM.

-  PDL = Current PRNDL position.

-  TM_LK_DLY = Timer for converter clutch unconditional unlock.

-  TM_UN_CT = Timer for closed throttle converter clutch relock.

-  TP_RATE = Throttle position rate.

-  TP_REL = Relative throttle position; TP - RATCH.

-  UNLKDLY_TMR = Timer used to delay closed  throttle  unconditional  unlock

(msec)

-  VSBART_RT = Filtered vehicle speed adjusted for RT_NOVS, MPH.

Bit Flags:

-  BIFLG = Brake applied flag; 0 -> brake not applied, 1 -> brake applied.

28-10

CONVERTER CLUTCH CONTROL, UNCONDITIONAL UNLOCK LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  BIFLG_LST = Last pass value of BIFLG.

-  CC_FM_FLG = Converter clutch failure mode flag;  0  ->  normal  converter

clutch operation, 1 -> converter clutch fmem action in progress.

-  FLG_CT_LST = Last pass value of FLG_TP_CT.

-  FLG_FRST_CM = First time a  shift  is  commanded  flag;  0  ->  no  shift

commanded  this  background  pass,  1  -> shift commanded this background

pass.

-  FLG_HIGH_ALT = Flag indicating vehicle is at high altitude.

-  FLG_LK_CM = Converter clutch lock-up commanded  flag;  0  ->  de-energize

solenoid, unlock converter clutch, 1 -> energize solenoid, lock converter

clutch.

-  FLG_MLUS_FM = Excessive converter clutch slip error unconditional  unlock

flag;  0  ->  no excessive clutch slip unlock, 1 -> excessive clutch slip

unlock.

-  FLG_OT_LK = Transmission overtemp lockup mode flag; 1 ->  overtemperature

mode.

-  FLG_SFT_IN = Shift in progress flag; 1 -> shift in progress.

-  FLG_TP_CT = Flag indicating closed  throttle  break  point;  0  ->  above

closed throttle break point, 1 -> below closed throttle break point.

-  FLG_UN_ALT = High altitude unconditional unlock flag; 0 ->  not  at  high

altitude, 1 -> high altitude.

-  FLG_UN_BRK = Brake applied unconditional unlock  flag;  0  ->  brake  not

applied, 1 -> brake applied.

-  FLG_UN_CT = Closed throttle unconditional unlock flag; 0  ->  not  closed

throttle, 1 -> closed throttle.

-  FLG_UN_MDN = Manual downshift sequence unconditional unlock flag; 0 -> no

manual downshift unlock, 1 -> manual downshift unlock.

-  FLG_UN_PRN = PRNDL position unconditional unlock  flag;  0  ->  no  PRNDL

position  unconditional  unlock, 1 -> PRNDL in park, reverse, neutral, or

manual one.

-  FLG_UN_SPD = Low engine/vehicle speed or low  speed  ratio  unconditional

unlock flag; 0 -> no low speed related unlock, 1 -> low speed unlock.

-  FLG_UN_TEMP = Cold temperature unconditional unlock flag; 0 -> no cold

-  FLG_UN_TRA = Throttle rate accel unconditional unlock flag; 0 -> not high

positive throttle rate, 1 -> high positive throttle rate.

-  FLG_UN_TRD = Throttle rate decel unconditional unlock flag; 0 -> not high

negative throttle rate, 1 -> high negative throttle rate.

-  FLG_UN_ULSF = Unlocked shift unconditional unlock flag; 0  ->  locked  up

prior to start of shift, 1 -> unlocked prior to start of shift.

28-11

CONVERTER CLUTCH CONTROL, UNCONDITIONAL UNLOCK LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FLG_UNC_UNLK = Converter clutch unconditional unlock flag; 0 ->  normally

scheduled MLUS modulation, 1 -> unconditional unlock.

-  OFMFLG = ETV overcurrent monitor failure flag; 0 -> ETV O.K.,  1  ->  ETV

failure mode.

-  OSFMFLG = Output Shaft Sensor FMEM flag; 1 -> OSS fmem in progress.

-  TRAC_ULFLG = Flag forcing transmission unlock; 1 -> unlock.

-  MLUS_FM_FLG = Flag used to allow FMEM action without storage of a service

code.

-  SFT_ERR_FLG = Flag indicating that one shift has failed.

Calibration Constants:

-  BPUNMH = Hysteresis for BPUNMN.

-  BPUNMN = Minimum BP to unlock converter clutch.

-  BRKRLCKTM = Time to remain unlocked after transition from  break  off  to

break on.

-  CTDLY = Closed throttle relock delay.

-  D21DLY = Unlocked 2 -> 1 shift relock delay.

-  D32DLY = Unlocked 3 -> 2 shift relock delay.

-  D43DLY = Unlocked 4 -> 3 shift relock delay.

-  ETLUMX = Minimum ECT to enable converter clutch lock-up, deg F.

-  LUDLY = High altitude/cold engine relock delay.

-  MLPS_2 = IPDL value the MLPS is in the  second  to  last  position;  2  =

Manual 2 mode has been selected, TCS is present, 3 = Drive mode/Overdrive

cancel mode has been selected, no TCS present.

-  NELUMN = Minimum NEBART to unconditionally unlock converter, RPM.

-  OTCTDLY = Overtemp closed throttle relock delay, sec.

-  PDL_UNLK = PDLs which force Unconditional  Unlock;  bit  (PDL  -  1)  set

forces Unlock.

-  PRNDLY = PRNDL relock delay.

-  SRLUMN = Maximum SPD_RATIO to unconditionally unlock converter, unitless.

-  SRLUMN0T = maximum speed ratio to unconditionally unlock when FLG_OT_LK =

1

-  TMCTDY = Minimum time before relock at closed throttle.

-  TPUNBRK = Maximum relative TP for brake applied unlock.

28-12

CONVERTER CLUTCH CONTROL, UNCONDITIONAL UNLOCK LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TPUNCH = Hysteresis for TPUNCT.

-  TPUNCT = Maximum relative TP for closed throttle unlock.

-  TPUNTR = Relative TP breakpoint for high and low unlock throttle rate.

-  TRADLY = High throttle rate, accel, relock delay.

-  TRDDLY = High throttle rate, decel, relock delay.

-  TRUHAC = Throttle rate unlock, high TP, accel.

-  TRUHDC = Throttle rate unlock, high TP, decel.

-  TRULAC = Throttle rate unlock, low TP, accel.

-  TRULDC = Throttle rate unlock, low TP, decel.

-  UNLKDLYTM = Time to remain locked upon  reaching  closed  throttle  break

point.

-  U12DLY = Unlocked 1 -> 2 shift relock delay.

-  U23DLY = Unlocked 2 -> 3 shift relock delay.

-  U34DLY = Unlocked 3 -> 4 shift relock delay.

-  VSCTDY = Minimum vehicle speed for closed throttle relock.

-  VSLUMN = Maximum VSBRT_RT to unconditionally unlock converter, mph.

28-13

CONVERTER CLUTCH CONTROL, UNCONDITIONAL UNLOCK LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CCC_UNCOND_UNLK_COM10

always ----------------------------------| FLG_CT_LST = FLG_TP_CT

TP_REL < TPUNCT -------------------|S Q -| FLG_TP_CT

|

TP_REL >= TPUNCT + TPUNCH ---------|C

FLG_CT_LST = 0 --------------------|

|AND -| UNLKDLY_TMR = UNLKDLYTM

FLG_TP_CT = 1 ---------------------|     | (load unlock delay timer)

(transition to closed throttle)

FLG_TP_CT = 1 ---------------------|

|AND -| FLG_UN_CT = 1

UNLKDLY_TMR = 0 -------------------|     | (command closed throttle

(at closed throttle for a               |  unlock)

calibratible time)                     |

| --- ELSE ---

FLG_TP_CT = 0 ---------------------|     |

(not at closed throttle)          |AND -| FLG_UN_CT = 0

|     | (reset closed throttle

FLG_UN_CT = 1 ---------------------|     |  unlock flag)

|

| --- ELSE ---

|

| NO ACTION

TP_RATE >= TRUHAC -----|

|AND -|

TP_REL >= TPUNTR ------|     |

|OR --|

TP_RATE >= TRULAC -----|     |     |

|AND -|     |AND -| FLG_UN_TRA = 1

TP_REL < TPUNTR -------|           |     | (high tip-in rate)

|     |

FLG_OT_LK = 0 ---------------------|     |

| --- ELSE ---

|

| FLG_UN_TRA = 0

28-14

CONVERTER CLUTCH CONTROL, UNCONDITIONAL UNLOCK LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TP_RATE <= TRUHDC -----|

|AND -|

TP_REL >= TPUNTR ------|     |

|OR --|

TP_RATE <= TRULDC -----|     |     |

|AND -|     |AND -| FLG_UN_TRD = 1

TP_REL < TPUNTR -------|           |     | (high tip-in rate)

|     |

FLG_OT_LK = 0 ---------------------|     |

| --- ELSE ---

|

| FLG_UN_TRD = 0

BIFLG_LST = 0 ---------------------|

|

BIFLG = 1 -------------------------|AND -| FLG_UN_BRK = 1

|     |  (brake applied)

TP_REL <= TPUNBRK -----------------|     |

(vehicle speed maintained -             |

4-way flasher not on)                  | --- ELSE ---

|

| FLG_UN_BRK = 0

28-15

CONVERTER CLUTCH CONTROL, UNCONDITIONAL UNLOCK LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

bit (PDL - 1) of PDL_UNLK = 1 -----------| FLG_UN_PRN = 1

|

| --- ELSE ---

|

| FLG_UN_PRN = 0

ECT <= ETLUMX ---------------------|

|OR --| FLG_UN_TEMP = 1

CS_SFT_MULT <> 1.0 ----------------|     |  (cold engine)

|

| --- ELSE ---

|

| FLG_UN_TEMP = 0

PDL = 3 ---------------------------|

|

GR_CM = 4 -------------------------|AND -| FLG_UN_MDN = 1

|     |  (hydraulic 4-3 downshift)

MLPS_2 <> 2 -----------------------|     |

(TCS input NOT present)                 |

| --- ELSE ---

|

| FLG_UN_MDN = 0

BP <= BPUNMN ----------------------|S Q -| FLG_HIGH_ALT

|     | (vehicle at high altitude)

BP > BPUNMN + BPUNMH --------------|C

FLG_HIGH_ALT = 1 ------------------|

|AND -| FLG_UN_ALT = 1

FLG_OT_LK = 0 ---------------------|     |  (high altitude)

|

| --- ELSE ---

|

| FLG_UN_ALT = 0

FLG_FRST_CM = 1 -------------------|

(shift commanded)                 |AND -| FLG_UN_ULSF = 1

|     |  (minimum unlock time after

FLG_UNC_UNLK = 1 ------------------|     |   start of unlocked shift)

(unconditional unlock)                  |

| --- ELSE ---

|

| FLG_UN_ULSF = 0

28-16

CONVERTER CLUTCH CONTROL, UNCONDITIONAL UNLOCK LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

NEBART < NELUMN -------------------|

|

VSBART_RT < VSLUMN ----------------|OR --| FLG_UN_SPD = 1

|     |  (low engine/vehicle speed

SPD_RATIO < SRLUMN ----------|     |     |   or low speed ratio

|     |     |

FLG_OT_LK = 0 ---------------|     |     |

|AND -|     |

FLG_SFT_IN = 0 --------------|     |     |

|     |     |

OSFMFLG = 0 -----------------|     |     |

|     |     |

SFT_ERR_FLG = 0 -------------|     |     |

|     |

SPD_RATIO < SRLUMNOT --------|     |     |

|     |     |

FLG_OT_LK = 1 ---------------|     |     |

|AND -|     |

FLG_SFT_IN = 0 --------------|           |

|           |

OSFMFLG = 0 -----------------|           |

|           |

SFT_ERR_FLG = 0 -------------|           |

| --- ELSE ---

|

| FLG_UN_SPD = 0

FLG_UN_CT = 1 ---------------------------| Allow TM_UN_CT to count up

(closed throttle)                       |

| --- ELSE ---

|

| TM_UN_CT = 0

MLUS_FM_FLG = 1 -------------------|

(exessive slip due to             |

overtemp)                        |OR --| FLG_MLUS_FM = 1

|     |  (excessive converter

CC_FM_FLG = 1 ---------------------|     |   clutch slip)

(converter clutch FMEM in progress)     |

|

| --- ELSE ---

|

| FLG_MLUS_FM = 0

|  (no excessive converter

|   clutch slip)

28-17

CONVERTER CLUTCH CONTROL, UNCONDITIONAL UNLOCK LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLG_UN_CT = 1 ---------------------|

(closed throttle)                 |

|AND -| TM_LK_DLY = OTCTDLY

TM_LK_DLY < OTCTDLY ---------------|     |

|     |

FLG_OT_LK = 1 ---------------------|     |

| --- ELSE ---

FLG_UN_CT = 1 ---------------------|     |

(closed throttle)                 |AND -| TM_LK_DLY = CTDLY

|

TM_LK_DLY < CTDLY -----------------|

FLG_UN_TRA = 1 --------------------|

(high tip-in rate)                |AND -| TM_LK_DLY = TRADLY

|

TM_LK_DLY < TRADLY ----------------|

FLG_UN_TRD = 1 --------------------|

(high tip-out rate)               |AND -| TM_LK_DLY = TRDDLY

|

TM_LK_DLY < TRDDLY ----------------|

FLG_UN_BRK = 1 --------------------|

(brake applied)                   |

|AND -| TM_LK_DLY = BRKRLCKTM

TM_LK_DLY < BRKRLCKTM -------------|

(BRKRLCKTM must be calibrated long

enough to prevent engine stall on

a panic stop)

FLG_UN_PRN = 1 --------------------|

(park, rev, neut)                 |AND -| TM_LK_DLY = PRNDLY

|

TM_LK_DLY < PRNDLY ----------------|

28-18

CONVERTER CLUTCH CONTROL, UNCONDITIONAL UNLOCK LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OFMFLG = 1 ------------------|

(ETV sol. shorted)          |

|

FLG_UN_SPD = 1 --------------|

(low engine/vehicle speed   |

or low speed ratio)        |

|

FLG_UN_TEMP = 1 -------------|

(cold temperature)          |

|

FLG_UN_MDN = 1 --------------|

(manual downshift)          |OR --|

|     |

FLG_UN_ALT = 1 --------------|     |

(high altitude)             |     |

|     |AND -| TM_LK_DLY = LUDLY

TRAC_ULFLG = 1 --------------|     |

(traction control request)  |     |

|     |

FLG_MLUS_FM = 1 -------------|     |

(slip)                            |

|

TM_LK_DLY < LUDLY -----------------|

28-19

CONVERTER CLUTCH CONTROL, UNCONDITIONAL UNLOCK LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLG_UN_ULSF = 1 -------------------|

|

GR_CM = 2 -------------------------|AND -| TM_LK_DLY = U12DLY

|     |

GR_OLD = 1 ------------------------|     |

| --- ELSE ---

FLG_UN_ULSF = 1 -------------------|     |

|     |

GR_CM = 1 -------------------------|AND -| TM_LK_DLY = D21DLY

|     |

GR_OLD > 1 ------------------------|     |

| --- ELSE ---

FLG_UN_ULSF = 1 -------------------|     |

|     |

GR_CM = 3 -------------------------|AND -| TM_LK_DLY = U23DLY

|     |

GR_OLD < 3 ------------------------|     |

| --- ELSE ---

FLG_UN_ULSF = 1 -------------------|     |

|     |

GR_CM = 2 -------------------------|AND -| TM_LK_DLY = D32DLY

|     |

GR_OLD > 2 ------------------------|     |

| --- ELSE ---

FLG_UN_ULSF = 1 -------------------|     |

|     |

GR_CM = 4 -------------------------|AND -| TM_LK_DLY = U34DLY

|     |

GR_OLD < 4 ------------------------|     |

| --- ELSE ---

FLG_UN_ULSF = 1 -------------------|     |

|     |

GR_CM = 3 -------------------------|AND -| TM_LK_DLY = D43DLY

|

GR_OLD = 4 ------------------------|

28-20

CONVERTER CLUTCH CONTROL, UNCONDITIONAL UNLOCK LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLG_UN_CT = 1 ---------------|

|AND -|

TM_UN_CT <= TMCTDY ----------|     |

(closed throttle short time)      |

|

TM_UN_CT > TMCTDY -----------|     |

(closed throt. long enough) |     |

|AND -|

VSBART_RT < VSCTDY ----------|     |

(low speed)                       |

|

FLG_UN_TRA = 1 --------------------|

(high tip-in)                     |

|

FLG_UN_TRD = 1 --------------------|

(high tip-out)                    |

|

FLG_UN_ULSF = 1 -------------------|

(unlocked shift)                  |

|OR --| FLG_UNC_UNLK = 1

FLG_UN_BRK = 1 --------------------|     |  (unconditionally unlock

(brake applied)                   |     |   converter clutch)

|     |

FLG_UN_PRN = 1 --------------------|     |

(park, rev, neut)                 |     |

|     |

FLG_UN_TEMP = 1 -------------------|     |

(cold temperature)                |     |

|     |

FLG_UN_MDN = 1 --------------------|     |

(manual downshift)                |     |

|     |

FLG_UN_ALT = 1 --------------------|     |

(high altitude)                   |     |

|     |

TRAC_ULFLG = 1 --------------------|     |

(traction control request)        |     |

|     |

FLG_UN_SPD = 1 --------------------|     |

(low engine/vehicle speed         |     |

or low speed ratio)              |     |

|     |

OFMFLG = 1 ------------------------|     |

(ETV solenoid shorted)            |     |

|     |

FLG_MLUS_FM = 1 -------------------|     |

(slip)                                  |

| --- ELSE ---

|

TM_LK_DLY = 0 ---------------------------| FLG_UNC_UNLK = 0

|  (allow modulation

|   if scheduled)

|

| --- ELSE ---

|

| no change to FLG_UNC_UNLK

28-21

CONVERTER CLUTCH CONTROL, SHIFT MODULATION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

28.5  SHIFT MODULATION LOGIC (CDAN0)

OVERVIEW

The Shift Modulation logic controls the length  of  time  after  a  shift  is

commanded  that  additional  bypass  clutch modulation takes place to control

converter slip on shifts.

DEFINITIONS

Registers:

-  DELTA_RATIO = Delta Shift Ratio change since command of shift.

-  IGR_DS = Unverified desired gear.

-  GR_CM = Commanded gear for shift solenoids.

-  GR_CM_LST = Commanded gear last past.

-  PDL = Filtered MLPS position.

-  PDL_LST = Filtered MLPS position last pass.

-  TM_SFT_CTL = Timer for closed/open loop PID control - for upshifts.

-  TM_SFT_EXIT = Time remaining to complete shift exit.

-  TM_SFT_SLP = Timer to control SLIP_TARGET during shift, sec.

-  TM_VER_SFT = Time to verify automatic desired shift.

Bit Flags:

-  FLG_CLUP = Flag indicating closed loop/BCSDC freeze region for upshifts.

-  FLG_CLUP_LST = Last pass value of FLG_CLUP.

-  FLG_DLY_SOSC = Flag used to delay  millisecond  "start  of  shift"  check

until SR_PP_LIM is calculated once; 1 -> allow millisecond check.

-  FLG_DNS_CTL = Downshift slip modulation flag; 1 -> Modifying  SLIP_TARGET

during downshift.

-  FLG_DNS_EXIT = In downshift exit mode.

-  FLG_FRST_CM = First time a shift is commanded flag; 1 -> shift  commanded

this background pass; 0 -> no shift commanded this background pass.

-  FLG_NV_SHFT  =  First  pass  flag   indicating   desired   shift   failed

verification.

-  FLG_OLUP = Flag indicating open loop control region for upshifts.

28-22

CONVERTER CLUTCH CONTROL, SHIFT MODULATION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FLG_OLUP_LST = Value of FLG_OLUP from previous background loop.

-  FLG_OPN_BS = Flag indicating converter open when upshift  started;  1  ->

open before upshift.

-  FLG_SFT_DN = Downshift flag; 1 -> Last/current shift is downshift.

-  FLG_SFT_STRT = Set when waiting to detect start of shift  once  its  been

commanded.

-  FLG_SFT_UP = Upshift flag; 1 -> Last/current shift is upshift.

-  FLG_UNC_UNLK = Converter clutch unconditional unlock flag.

-  FLG_OPEN = MLUS is in open mode, no capacity when set.

-  FLG_UCTL_LST = Last pass value of FLG_UPS_CTL.

-  FLG_UPS_CTL = Upshift slip modulation flag; 1  ->  Modifying  SLIP_TARGET

during upshift.

-  FLG_UPS_EXIT = In upshift exit mode.

-  SFT_ERR_FLG = Flag indicating that one shift has failed.

Calibration Constants:

-  DSRDE1 = Delta  Shift  Ratio  on  Downshift  to  1st  to  indicate  shift

complete.

-  DSRDE2 = Delta  Shift  Ratio  on  Downshift  to  2nd  to  indicate  shift

complete.

-  DSRDE3 = Delta  Shift  Ratio  on  Downshift  to  3rd  to  indicate  shift

complete.

-  DSRUE2 = Delta Shift Ratio on Upshift to 2nd to indicate shift complete.

-  DSRUE3 = Delta Shift Ratio on Upshift to 3rd to indicate shift complete.

-  DSRUE4 = Delta Shift Ratio on Upshift to 4th to indicate shift complete.

-  PCUPSCMPT = Fraction of upshift at which it is considered complete.

-  TMCLUP2 = Closed loop pid period during upshift to 2nd.

-  TMCLUP3 = Closed loop pid period during upshift to 3rd.

-  TMCLUP4 = Closed loop pid period during upshift to 4th.

-  TMD1SLP = 1st Downshift SLIP_TARGET hold duration.

-  TMD2SLP = 2nd Downshift SLIP_TARGET hold duration.

-  TMD3SLP = 3rd Downshift SLIP_TARGET hold duration.

-  TMD4SLP = 4th Downshift SLIP_TARGET hold duration.

28-23

CONVERTER CLUTCH CONTROL, SHIFT MODULATION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TMDNSEXIT = Exit downshift duration.

-  TMOLUP2 = Open loop pid period during an upshift to 2nd.

-  TMOLUP3 = Open loop pid period during an upshift to 3rd.

-  TMOLUP4 = Open loop pid period during an upshift to 4th.

-  TMSFTSLPMAX = Watchdog time for TM_SFT_SLP.

-  TMUPSEXIT = Exit upshift duration.

28-24

CONVERTER CLUTCH CONTROL, SHIFT MODULATION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CCC_MOD_COM2

The following logic ignores shifting control once the shift solenoid

functional test detects one bad shift.  This allows the shifts to occur

with out the torque converter clutch causing a change in engine speed.

SFT_ERR_FLG = 1 -------------------|

|AND -| FLG_OLUP = 0

FLG_SFT_UP = 1 --------------|     |     | FLG_CLUP = 0

|OR --|     | FLG_UPS_CTL = 0

IGR_DS > GR_CM --------------|           | FLG_DNS_CTL = 0

| FLG_SFT_STRT = 0

| FLG_UPS_EXIT = 0

| FLG_DNS_EXIT = 0

| EXIT MODULE

FLG_FRST_CM = 1 -------------------|

|AND -| FLG_SFT_STRT = 1

FLG_SFT_UP = 1 --------------------|     |

| --- ELSE

|

FLG_FRST_CM = 1 -------------------------| FLG_SFT_STRT = 0

FLG_FRST_CM = 1 -------------------------| TM_SFT_SLP = TMSFTSLPMAX

|  (set slip control timer)

FLG_UNC_UNLK = 1 ------------------|

|OR --| TM_SFT_CTL = 0

FLG_NV_SHFT = 1 -------------------|     | FLG_CLUP = 0

| FLG_OLUP = 0

always ----------------------------------| FLG_OLUP_LST = FLG_OLUP

| FLG_CLUP_LST = FLG_CLUP

| FLG_UCTL_LST = FLG_UPS_CTL

28-25

CONVERTER CLUTCH CONTROL, SHIFT MODULATION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

IGR_DS > GR_CM --------------|

|OR --|

PDL <> PDL_LST --------|     |     |

|AND -|     |

GR_CM > GR_CM_LST -----|           |AND -| TM_SFT_CTL = TMCLUP[GR_CM]

|     | FLG_CLUP = 1

FLG_CLUP = 0 ----------------------|     | FLG_OLUP = 0

|     | FLG_UPS_CTL = 0

FLG_OPEN = 1 ----------------------|     | FLG_UPS_EXIT = 0

| FLG_OPN_BS = 1

|

| --- ELSE ---

IGR_DS > GR_CM --------------|           |

|OR --|     |

PDL <> PDL_LST --------|     |     |     |

|AND -|     |AND -| TM_SFT_CTL = TMCLUP[GR_CM]

GR_CM > GR_CM_LST -----|           |     | FLG_CLUP = 1

|     | FLG_OLUP = 0

FLG_CLUP = 0 ----------------------|     | FLG_UPS_CTL = 0

| FLG_UPS_EXIT = 0

| FLG_OPN_BS = 0

|

| --- ELSE ---

(continued on next page)

28-26

CONVERTER CLUTCH CONTROL, SHIFT MODULATION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

FLG_CLUP = 1 ----------------------|     |

|     |

FLG_SFT_STRT = 0 ------------------|AND -| TM_SFT_CTL = TMOLUP[GR_CM]

|     | FLG_CLUP = 0

TM_VER_SFT = 0 --------------------|     | FLG_OLUP = 1

| FLG_UPS_CTL = 1

| FLG_UPS_EXIT = 0

| FLG_DLY_SOSC = 0

|

| --- ELSE ---

TM_SFT_CTL = 0 --------------------|     |

|     |

FLG_CLUP = 1 ----------------------|AND -| FLG_UPS_CTL = 1

|     | FLG_UPS_EXIT = 0

TM_VER_SFT = 0 --------------------|     |

| --- ELSE ---

DELTA_RATIO > DSRUE[GR_CM] --------|     |

|     |

TM_SFT_CTL = 0 --------------|     |     |

|AND -|     |

FLG_OLUP = 1 ----------------|     |OR --| TM_SFT_CTL = 0

|     | FLG_CLUP = 0

TM_SFT_SLP = 0 --------------|     |     | FLG_OLUP = 0

|     |     | FLG_UPS_CTL = 0

TM_VER_SFT = 0 --------------|AND -|     | FLG_UPS_EXIT = 1

|           | TM_SFT_EXIT = TMUPSEXIT

FLG_UPS_CTL = 1 -------|     |           | FLG_SFT_STRT = 0

|OR --|           | FLG_OPN_BS = 0

FLG_CLUP = 1 ----------|                 | FLG_DLY_SOSC = 0

|

| --- ELSE ---

FLG_SFT_UP = 1 --------------------|     |

|AND -| FLG_UPS_EXIT = 1

TM_SFT_EXIT > 0 -------------------|     |

| --- ELSE ---

|

| FLG_UPS_EXIT = 0

FLG_SFT_DN = 1 --------------------|

|

DELTA_RATIO > DSRDE[GR_CM] --------|AND -| TM_SFT_SLP = TMD[GR_CM]SLP

|

TM_SFT_SLP > TMD[GR_CM]SLP --------|

28-27

CONVERTER CLUTCH CONTROL, SHIFT MODULATION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLG_SFT_DN = 1 --------------------|

|AND -| FLG_DNS_CTL = 1

TM_SFT_SLP > 0 --------------------|     | FLG_DNS_EXIT = 0

|

| --- ELSE ---

FLG_SFT_DN = 1 --------------------|     |

|     |

FLG_DNS_CTL = 1 -------------------|AND -| FLG_DNS_CTL = 0

|     | FLG_DNS_EXIT = 1

FLG_DNS_EXIT = 0 ------------------|     | TM_SFT_EXIT = TMDNSEXIT

|

| --- ELSE ---

FLG_SFT_DN = 1 --------------------|     |

|AND -| FLG_DNS_CTL = 0

TM_SFT_EXIT > 0 -------------------|     | FLG_DNS_EXIT = 1

|

| --- ELSE ---

|

| FLG_DNS_CTL = 0

| FLG_DNS_EXIT = 0

28-28

CONVERTER CLUTCH CONTROL, BYPASS CLUTCH CONTROL TESTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY  CONFIDENTIAL

28.6  BYPASS CLUTCH CONTROL TESTS (CDAK0)

OVERVIEW

This module provides different test modes to develop the calibration for  the

Bypass  Converter  Clutch  control.   Setting SWBCTST to a value greater than

zero, allows selection of a particular test mode.

NOTE:  Any test in which teh open loop torque capacity equation based off the

converter  model  is  used,  will  not be valid during a shift due to lack of

turbine speed information.

DEFINITIONS

INPUTS

Registers:

-  TCAP_INTR1 = Interim requested torque capacity, ft-lbs.

Calibration Constants:

-  SLIP_DES_S = Desired value of slip, signed.

-  SLIP_TARGET = Target converter slip value.

-  SWBCTST = Modulated Bypass Clutch Control test switch.

-  TSTBCSDC = Fixed test value for BCSDC.

-  TSTSLIP = Fixed test value for SLIP_DES_S.

-  TSTTCAP1 = Fixed test value for TCAP_REQ1.

OUTPUTS

Registers:

-  BCSDC = Modulated Byplass Clutch duty cycle.

-  SLIP_TCAP  =  Total  desired  slip  to  be  used  for   torque   capacity

calculation.

-  TCAP_REQ1 = Requested Bypass Clutch Torque Capacity, ft-lbs.

28-29

CONVERTER CLUTCH CONTROL, BYPASS CLUTCH CONTROL TESTS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY  CONFIDENTIAL

PROCESS

STRATEGY MODULE: CCC_BYP_CLTCH_TESTS_COM2

SWBCTST = 1 ----------------------------| BCSDC = TSTBCSDC

(fixed duty cycle test)               |

| --- ELSE ---

|

SWBCTST = 2 ----------------------------| SLIP_TCAP = TSTSLIP

(fixed desired slip test;             | Do: CCC_TCAP_CALC_COMx

open loop control only               | TCAP_REQ1 = TCAP_INTR1

- do not use during shifts)          | Do: CCC_TCAP_CONV_COMx

|

| --- ELSE ---

|

SWBCTST = 3 ----------------------------| SLIP_DES_S = TSTSLIP

(fixed desired slip test;             | SLIP_TARGET = TSTSLIP

open loop and PID control            | Do: CCC_BYP_SLIP_CALC_COMx

- do not use during shifts)          | Do: CCC_PID_SLIP_CTL_COMx

| Do: CCC_TCAP_CALC_COMx

| TCAP_REQ1 = TCAP_INTR1

| Do: CCC_TCAP_CONV_COMx

|

| --- ELSE ---

|

SWBCTST = 4 ----------------------------| TCAP_REQ1 = TSTTCAP1

(fixed torque capacity test)          | Do: CCC_TCAP_CONV_COMx

28-30

CONVERTER CLUTCH CONTROL, SLIP CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

28.7  SLIP CALCULATION (CDAM0)

OVERVIEW

The Slip Calculation module calulates the desired slip (SLIP_DES_S).

This module also calculates slip error which is the  difference  between  the

desired  slip  (SLIP_DES_S)  and  the  value  of actual slip (SLIP_ACT).  The

result is stored in ERR_T0.  Unique filterd value of ERR_T0 are provided  for

PID slip control.

DEFINITIONS

Registers:

-  ERR_T0 = Current slip error = SLIP_DES_S - SLIP_ACT, rpm.

-  ERR_T0_FD = Filtered value of ERR_T0 for derivative PID slip control.

-  ERR_T0_FI = Filtered value of ERR_T0 for integral PID slip control.

-  ERR_T0_FP = Filtered value of ERR_T0 for proportional PID slip control.

-  ERR_T1 = Last pass value of ERR_T0_FD.

-  GR_CM = Commanded gear for shift solenoids.

-  IGR_DS = Unverified desired gear.

-  NEBART = Filtered engine RPM for transmission, rpm.

-  NOBART = Filtered transmission output shaft speed.

-  NTBART = Filtered value of turbine speed.

-  RT_GR_OLD = Last gear transmission gear ratio.

-  SLIP_ACT = Actual value of slip (signed), rpm.

-  SLIP_DES = Desired value of slip, unsigned.

-  SLIP_DES_LST = Last pass value of SLIP_DES.

-  SLIP_DES_S = Desired value of slip, signed.

-  SLIP_TARGET = Target slip value from tables.

-  SLP_DES_LST = Previous value of SLIP_DES_S.

-  TM_VER_SFT = Time to verify automatic desired shift.

28-31

CONVERTER CLUTCH CONTROL, SLIP CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Bit Flags:

-  FLG_CLUP  =  flag  indicating  closed  loop/TCAP_REQ  freeze  region  for

upshifts.

-  FLG_CRV_LK = Scheduled curve lockup flag.

-  FLG_DB_REG = Flag indicating slip is within Deadband Region;  1  ->  Slip

within Deadband Region.

-  FLG_DNS_CTL = Downshift slip modulation flag; 1 ->  performing  downshift

modulation.

-  FLG_DNS_EXIT = In Downshift exit mode.

-  FLG_UNLK_DS = Converter unlock in progress flag.

-  FLG_UPS_EXIT = In Upshift exit mode.

Calibration Constants:

-  ERR_DB_CH = |ERR_T0| threshold indicating slip is not in Deadband Region.

-  ERR_DB_SL = |ERR_T0| threshold indicating slip is in Deadband Region.

-  GRRAT2 = Second gear ratio.

-  SLIP_STEP = Minimum RPM difference between SLIP_DES_S and  SLIP_TRG_S  to

allow desired slip to step to target.

-  MAXSLIP1 = Maximum attainable slip target value, rpm.

-  TCDNS = Time constant for SLIP_DES_S during a downshift.

-  TCERRD = Time constant for ERR_T0_FD.

-  TCERRI = Time constant for ERR_T0_FI.

-  TCERRP = Time constant for ERR_T0_FP.

-  TCLCK = Time constant for SLIP_DES_S during lockups and modulation.

-  TCLCKL = Time constant for SLIP_DES_S durring low TP_REL lockups.

-  TCULCK = Time constant for SLIP_DES_S during an unlock.

-  TCUPS = Time constant for SLIP_DES_S during an upshift.

-  TPBRKTCSLP = TP_REL break point below which to use TCLCKL.

28-32

CONVERTER CLUTCH CONTROL, SLIP CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CCC_BYP_SLIP_CALC_COM5

At power-up initialize the following parameter:

initially ------------------------------| SLIP_DES_S = MAXSLIP1

always ---------------------------------| SLP_DES_LST = SLIP_DES_S

| SLIP_DES_LST = SLIP_DES

| slip_des = SLIP_DES_S

FLG_CLUP = 1 ---------------------|

|OR --| tcslip = TCUPS

FLG_UPS_EXIT = 1 -----------------|     |  (use upshift time constant)

|

| --- ELSE ---

TM_VER_SFT > 0 -------------|           |

|AND -|     |

IGR_DS < GR_CM -------------|     |     |

|     |

FLG_DNS_CTL = 1 ------------------|OR --| tcslip = TCDNS

|     |  (use downshift time constant)

FLG_DNS_EXIT = 1 -----------------|     |

|     |

FLG_UNLK_DS = 1 ------------------|     |

| --- ELSE ---

|

FLG_CRV_LK = 1 -------------------|     |

|AND--| tcslip = TCLCKL

TP_REL < TPBRKTCSLP --------------|     |  (use low tp lockup time constant)

|

| --- ELSE ---

|

FLG_CRV_LK = 1 -------------------------| tcslip = TCLCK

|  (use lockup time constant)

|

| --- ELSE ---

|

| tcslip = TCULCK

|  (use unlock time constant)

always ---------------------------------| slip_des =

ROLAV(SLIP_TARGET,tcslip)

|SLIP_TARGET - slip_des| < SLIP_STEP ---| SLIP_DES_S = SLIP_TARGET

|

| --- ELSE ---

|

| SLIP_DES_S = slip_des

28-33

CONVERTER CLUTCH CONTROL, SLIP CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SLIP_DES_S >= 0 ------------------------| SLIP_DES = SLIP_DES_S

|

| --- ELSE ---

|

| SLIP_DES = 0

FLG_CLUP = 1 ---------------------|

|AND -| SLIP_ACT =

TM_VER_SFT = 0 -------------------|     |   (NEBART - NOBART * RT_GR_OLD)

|

| --- ELSE ---

GR_CM = 3 ------------------------|     |

|AND -| SLIP_ACT =

PDL = 2 --------------------------|     |   (NEBART - NOBART * GRRAT2)

|

| --- ELSE ---

|

| SLIP_ACT = (NEBART - NTBART)

always ---------------------------------| SLIP_ABS = abs(SLIP_ACT)

| ERR_T1 = ERR_T0_FD

| ERR_T0 = SLIP_DES_S - SLIP_ACT

| ERR_T0_FP = ROLAV(ERR_T0,TCERRP)

| ERR_T0_FI = ROLAV(ERR_T0,TCERRI)

| ERR_T0_FD = ROLAV(ERR_T0,TCERRD)

|ERR_T0| < ERR_DB_SL -------------|S Q -| FLG_DB_REG

|     |  (slip error within deadband

|ERR_T0| >= ERR_DB_CH ------------|C    |   region)

28-34

CONVERTER CLUTCH CONTROL, SCHEDULED LOCK/UNLOCK LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

28.8  SCHEDULED LOCK/UNLOCK LOGIC (CDAN0)

OVERVIEW

This logic schedules converter clutch applies and releases on  the  basis  of

sea level and altitude or engine output torque loss lock and unlock curves.

DEFINITIONS

Registers:

-  COMP_FCTR = Compensation factor for  altitude  or  engine  output  torque

loss.

-  GR_CM = Commanded gear for shift solenoids.

-  PNWF = Performance and normal weighting factor.

-  RATCH = Closed throttle position, counts.

-  RT_NOVS = Ratio of actual N/V to base N/V in KAM.

-  SLIP_ACT = Actual value of slip (signed).

-  SPD_RATIO = Speed ratio across the torque converter.

-  TMR_NO_LK = Timer to delay lockup after SLIP_ACT > SLIPNOLK.

-  TP_REL_H = Relative TP (TP - RATCH); high byte only.

-  TQ_NET = Net torque into torque converter.

-  VSBART_RT = Filtered vehicle speed adjusted for RT_NOVS for transmission,

mph.

Bit Flags:

-  FLG_CRV_DS = Scheduled lockup desired from curve flag.

-  FLG_CRV_LK = Scheduled curve lock-up flag; 0 -> no scheduled  lock-up,  1

-> scheduled lock-up.

-  FLG_CRV_LST = Last pass value of FLG_CRV_DS.

-  FLG_FMM_LK = Failure Mode Management lock-up flag; 1  ->  lock  converter

due to FMEM action.

-  FLG_OT_LK = Transmission overtemp lock-up  mode:   1  ->  overtemperature

mode.

-  FLG_SFT_IN = Shift in progress flag.

-  FLG_SFT_UP = Upshift flag.

28-35

CONVERTER CLUTCH CONTROL, SCHEDULED LOCK/UNLOCK LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FLG_UNC_UNLK = Converter  clutch  unconditional  unlock  flag;  0  ->  no

unconditional unlock, 1 -> unconditional unlock.

-  SFT_ERR_FLG = Flag indicating that one shift has failed.

Calibration Constants:

-  FN2LCOMP(TP_REL_H) = Delta vehicle speed for 2nd gear lockup for altitude

or engine output torque loss.

-  FN2LH(TP_REL_H) = Vehicle speed for 2nd gear overtemp lockup.

-  FN2LS(TP_REL_H) = Vehicle speed for 2nd gear lockup at sea level.

-  FN2LPWF(TP_REL_H)  =  Delta  vehicle  speed  for  2nd  gear   lockup   in

performance mode.

-  FN2UCOMP(TP_REL_H) = Delta Vehicle speed for 2nd gear unlock for altitude

or engine output torque loss.

-  FN2UH(TP_REL_H) = Vehicle speed for 2nd gear overtemp unlock.

-  FN2US(TP_REL_H) = Vehicle speed for 2nd gear unlock at sea level.

-  FN2UPWF(TP_REL_H)  =  Delta  vehicle  speed  for  2nd  gear   unlock   in

performance mode.

-  FN3LCOMP(TP_REL_H) = Delta vehicle speed for 3rd gear lockup for altitude

or engine output torque loss.

-  FN3LH(TP_REL_H) = Vehicle speed for 3rd gear overtemp lockup.

-  FN3LPWF(TP_REL_H)  =  Delta  vehicle  speed  for  3rd  gear   lockup   in

performance mode.

-  FN3LS(TP_REL_H) = Vehicle speed for 3rd gear lockup at sea level.

-  FN3UCOMP(TP_REL_H) = Delta Vehicle speed for 3rd gear unlock for altitude

or engine output torque loss.

-  FN3UH(TP_REL_H) = Vehicle speed for 3rd gear overtemp unlock.

-  FN3UPWF(TP_REL_H)  =  Delta  vehicle  speed  for  3rd  gear   unlock   in

performance mode.

-  FN3US(TP_REL_H) = Vehicle speed for 3rd gear unlock at sea level.

-  FN4LCOMP(TP_REL_H) = Delta vehicle speed for 4th gear lockup for altitude

or engine output torque loss.

-  FN4LH(TP_REL_H) = Vehicle speed for 4th gear overtemp lockup.

-  FN4LPWF(TP_REL_H)  =  Delta  vehicle  speed  for  4th  gear   lockup   in

performance mode.

-  FN4LS(TP_REL_H) = Vehicle speed for 4th gear lockup at sea level.

28-36

CONVERTER CLUTCH CONTROL, SCHEDULED LOCK/UNLOCK LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FN4UCOMP(TP_REL_H) = Delta Vehicle speed for 4th gear unlock for altitude

or engine output torque loss.

-  FN4UH(TP_REL_H) = Vehicle speed for 4th gear overtemp unlock.

-  FN4UPWF(TP_REL_H)  =  Delta  vehicle  speed  for  4th  gear   unlock   in

performance mode.

-  FN4US(TP_REL_H) = Vehicle speed for 4th gear unlock at sea level.

-  FN_SRLK(TQ_NET) = Function to  determine  minimum  speed  ratio  to  lock

comverter based on TQ_NET.

-  SLPNOLK = SLIP_ACT below which converter is not allowed to go  from  open

to modulated.

-  SRLKOT  =  minimum  speed  ratio  to  lock  converter  when  trans.    is

overtemperature.

-  TMNOLKDLY = Time after SLIP_ACT > SLPNOLK to delay lock up.

28-37

CONVERTER CLUTCH CONTROL, SCHEDULED LOCK/UNLOCK LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CCC_SCHLD_LCK_UNLCK_COM16

always -----------------------------------| FLG_CRV_LST = FLG_CRV_DS

GR_CM <> 1 -------------------------|

|AND -| vs_lk = FN[GR_CM]LS +

FLG_OT_LK = 0 ----------------------|     |   COMP_FCTR * FN[GR_CM]LCOMP +

|   PNWF * FN[GR_CM]LPWF

|

| vs_unlk = FN[GR_CM]US +

|   COMP_FCTR * FN[GR_CM]UCOMP +

|   PNWF * FN[GR_CM]UPWF

|

| --- ELSE ---

GR_CM <> 1 -------------------------|     |

|AND -| vs_lk = FN[GR_CM]LH

FLG_OT_LK = 1 ----------------------|     |

| vs_unlk = FN[GR_CM]UH

GR_CM <> 1 -------------------|

|

FLG_OT_LK = 0 ----------------|AND -|

|     |

SPD_RATIO >= FN_SRLK(TQ_NET) -|     |

|OR --| ratio_ok = 1

GR_CM <> 1 -------------------|     |     |

|     |     |

FLG_OT_LK = 1 ----------------|AND -|     |

|     |     |

SPD_RATIO >= SRLKOT ----------|     |     |

|     |

FLG_SFT_IN = 1 ---------------------|     |

(shifting disregard speed ratio)         |

| --- ELSE ---

|

| ratio_ok = 0

28-38

CONVERTER CLUTCH CONTROL, SCHEDULED LOCK/UNLOCK LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SFT_ERR_FLG = 1 --------------------|

|

FLG_SFT_UP = 1 ---------------------|AND -| No change to

|     | FLG_CRV_DS

FLG_SFT_IN = 1 ---------------------|     |

(Do not allow converter                  |

to lock or unlock durring               | --- ELSE ---

a shift after shift                     |

solenoid functional                     |

test detects one bad                    |

shift)                                  |

|

GR_CM = 1 --------------------------------| FLG_CRV_DS = 0

|

| --- ELSE ---

ratio_ok = 1 -----------------|           |

|AND -|     |

VSBART_RT > vs_lk ------------|     |OR --| FLG_CRV_DS = 1

|     |

FLG_FMM_LK = 1 ---------------------|     |

| --- ELSE ---

|

VSBART_RT < vs_unlk ----------------------| FLG_CRV_DS = 0

|

| --- ELSE ---

|

| No change to

| FLG_CRV_DS

SLIP_ACT < SLPNOLK -----------------------| TMR_NO_LK = TMNOLKDLY

28-39

CONVERTER CLUTCH CONTROL, SCHEDULED LOCK/UNLOCK LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLG_UNC_UNLK = 1 -----------------|

|OR ----| FLG_CRV_LK = 0

FLG_CRV_LK = 0 -------------|     |       |  (unconditional unlock,

|AND -|       |   ignore desired state)

SLIP_ACT < SLPNOLK ---|     |             |

|OR --|             |

TMR_NO_LK > 0 --------|                   |

(not allow lockup durring                |

negative slip )                         |

| --- ELSE ---

SFT_ERR_FLG = 1 --------------------|     |

|     |

FLG_SFT_UP = 1 ---------------------|AND -| No change to

|     | FLG_CRV_LK

FLG_SFT_IN = 1 ---------------------|     |

(Do not allow converter                  |

to lock or unlock durring               |

a shift after shift                     |

solenoid functional                     |

test detects one bad                    |

shift)                                  |

|

| --- ELSE ---

|

| FLG_CRV_LK = FLG_CRV_DS

|  (allow desired state to

|   pass through)

28-40

CONVERTER CLUTCH CONTROL, MLUS CONTROL AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

28.9  MLUS CONTROL AND SELF TEST (CDAN0)

OVERVIEW

This module computes the Torque  Converter  Clutch  modulated  bypass  clutch

solenoid pulsewidth (based on BCSDC), outputs the pulsewidth to the DUCE chip

(in units of DUCE ticks), monitors the fault status of the solenoid  circuit,

and performs rationality checks on converter clutch operation.

NOTE:  The terms Modulated  Lock-Up  Solenoid  (MLUS)  and  Torque  Converter

Clutch  Solenoid  (TCC)  are used interchangeably in this module.  TCC is the

"new" term for the solenoid and is being phased in where appropriate.

The High Impedance Torque Converter Clutch solenoid (TCC-H) is a  duty  cycle

controlled  device.   The  driver turns the solenoid "ON" for a percentage of

each control period based on the requested duty cycle.  Duty cycle is  output

to a DUCE chip, which then controls the Universal Power Output Driver (UPOD).

The UPOD driver controls the TCC and returns a "fault  status"  (TCC_H_FAULT)

for the TCC circuit.  The fault status can be decoded as follows:

TCC_H_FAULT = 0 --> no fault present.

TCC_H_FAULT = 1 --> a fault has been detected.

Note:  The fault status lags the commanded state by 1 background - ie is  for

the last commanded duty cycle.

Also, to be able to fully diagnose all faults the solenoid must be tested  at

OFF  (0%  duty cycle) and fully ON (100% duty cycle).  Monitoring TCC_H_FAULT

at mid-range duty cycles will not cause false  error  detections,  but  could

result in a failure being missed.

Due to this restriction, the solenoid will only be tested at 0% and 100% duty

cycle.

In addition to the output state monitor, there will be two rationality checks

performed on the mlus/converter clutch system during continuous operation:

1.  mlus_tst - verifies that the converter clutch is applied  when  commanded

on  by  comparing  actual  slip  to  target  slip.   If excessive slip is

detected when hardlock duty cycle is commanded, error code P0741 will  be

set.

2.  catchnet - monitors the converter clutch system for  instability  (surge)

during soft lock operation.  If converter clutch surge is detected, error

code P1741 will be set.

28-41

CONVERTER CLUTCH CONTROL, MLUS CONTROL AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  BCSDC = Modulated bypass clutch duty cycle.

-  BCSDC_OUT = Modulate bypass clutch duty cycle output to the DUCE.

-  BRK_AWAY_CNT = The number of converter clutch break aways detected.

-  ERR_T0 = Current slip error (SLIP_DES_S - SLIP_ACT).

-  LUS_PW_DTKS  =  Modulated  bypass  clutch  lockup  solenoid  duty   cycle

pulsewidth in duce_ticks.

-  LUS_PW_DUCE = DUCE hardware register.

-  MLUS_DLY_TMR = MLUS test delay and  verification  timer,  delays  testing

during  initial  MLUS controller transients, and determines the durtation

of high slip for error verification.

-  MLUS_ERR_CNT =  Modulated  bypass  clutch  lockup  solenoid  error  fault

filter.

-  MLUS_NT_TMR = Modulated bypass clutch lockup solenoid normal  temperature

delay timer.

-  MLUS_OT_CNT = Modulated bypass clutch lockup solenoid hot fault filter.

-  MLUS_OT_TMR = Modulated bypass  clutch  lockup  solenoid  overtemperature

timer.

-  NTBART = Filtered transmission turbine speed.

-  P0741CNT = Fault 0741 warm up cycle counter.

-  SLIP_DES = Desired value of slip.

-  SLIP_DES_LST = Previous value of SLIP_DES.

-  SLP_TDLY_TMR = Converter clutch  continuous  slip  test  transient  delay

timer.

-  TCC_CTL_TMR = TCC control timer, used in TCC functional test to determine

when the system is under control - defines a "good" lock-up event.

-  TCC_H_FAULT = High impedance TCC fault status; 0 -> no fault present.

-  TCC_OSM_TMR =  Modulated  bypass  clutch  lockup  solenoid  output  state

monitor timer.

-  TMSLBRK = Time since last converter clutch break away.

-  TM_BRK_AWAY = Time at which the current break away occurred.

-  TM_BRK_PREV = Time at which the previous break away occurred.

28-42

CONVERTER CLUTCH CONTROL, MLUS CONTROL AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TM_BRK_ZONE = Total time ERR_T0 has been in the break away zone.

-  TOT = Transmission operating temperature.

-  TP_REL = Relative TP (TP - RATCH).

-  TP_REL_ENT = Value of TP_REL at the entry of continuous slip test mode.

28-43

CONVERTER CLUTCH CONTROL, MLUS CONTROL AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TSTRAT  =  Transmission  strategy  switch;  The  TSTRAT  software  switch

controls which transmission control strategy is executed:

0 -> No transmission control (Manual trans., AOD, ATX, C6, C3, etc.)

1 -> SIL (Shift indicator light)

2 -> A4LD with 3-4 shift control and converter clutch control

3 -> AXOD

4 -> C64E (E4OD)

5 -> A4LDE

6 -> FAX-4

7 -> AOD-E (AOD-I)

8 -> 4EAT

9 -> CD4E

-  TQ_NET = Net torque into torque converter.

-  TQ_NET_ENT = Value of TQ_NET at the entry of continuous slip test mode.

-  VBAT = Battery voltage.

-  VSBART = Filtered vehicle speed for Transmission in MPH.

Bit Flags:

-  ACCFLG = A/C clutch status; 0 -> disengaged, 1 -> engaged.

-  AC_STATE_ENT = AC clutch state at the entry of converter clutch soft lock

test mode.

-  CCM_TST_ENA = Comprehensive component monitor test enable flag; 1 ->  ccm

tests are enabled.

-  CC_FM_FLG = Converter clutch failure mode flag; 0 -> normal operation,  1

-> execute converter clutch failure mode.

-  FLG_CRV_DS = Desired state of converter clutch based on schedule curves.

-  FLG_CRV_LST = Last pass value of FLG_CRV_DES.

-  FLG_DIS_CSM = Disable continuous slip mode flag;  0  ->  continuous  slip

enabled, 1 -> continuous slip disabled.

-  FLG_HIGH_SLP = MLUS test high slip flag; 0 -> slip is ok, 1 ->  excessive

slip detected during hardlock.

-  FLG_HT_MOD = Flag indicating high temperature slip modulation allowed;  1

-> modulation allowed, 0 -> not allowed.

-  FLG_OPEN = MLUS is in open mode, no capacity when set.

-  FLG_OPEN_LST = Last pass value of FLG_OPEN

-  FLG_OT_LK = Trans.  overtemperature lockup mode flag.

-  FLG_PID_CTL = Mod.  byp.  clutch pid control flag.

-  FLG_SFT_IN = Shift in progress flag.

28-44

CONVERTER CLUTCH CONTROL, MLUS CONTROL AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FLG_TST_BRK = continuous slip test entry flag; 0 -> do  not  test,  1  ->

test for break away.

-  FLG_VER_BRK  =  Verify  converter  clutch  break  away  flag;  0  ->   no

verification in progress, 1 -> verification in progress.

-  MLUS_FM_FLG = Flag used to allow FMEM action without storage of a service

code.

-  MLUS_HF_FLG = Flag used to signal half fuel once the converter clutch has

failed.

-  PxxxFAULT = OBDII fault flag for code xxx.

-  PxxxMALF = OBDII malfunction flag for fault XXX;  1  ->  malfunction  xxx

currently exits.

-  PxxxMON = OBDII fault monitor flag.

-  OSM_EO_OFF = KOEO OSM Off test requested by OBDII sequencer; 1 -> Command

output off and monitor circuit for off state.

-  OSM_EO_ON = KOEO OSM On test requested by OBDII sequencer; 1  ->  Command

output on and monitor circuit for on state.

-  SUBST_REQ15 = Substitution requested flag for channel 15.

-  TCC_CTL_FLG = Flag indicating  a  "good"  converter  clutch  lock-up  has

occurred; 1 -> the last TCC lock-up was a "good" event.

-  TCC_MON_OFF = TCC monitor off flag; 1 -> Off state has been  observed  to

function this power-up.

-  TCC_MON_ON = TCC monitor on flag ; 1 -> On state  has  been  observed  to

function this power-up.

Calibration Constants:

-  BCSDC_HRD = BCSDC when converter is hardlocked.

-  BRK_AWAY_LVL = Number of break aways at which  continuous  slip  will  be

disabled.

-  CC_FM_LVL = Number of warm up cycles that converter clutch  failure  mode

action will be executed after error detection.

-  CNET_TP_MIN = Minimum throttle position to allow catchnet test entry.

-  ECSM_LVL = Number of warm up cycles continuous slip mode will be disabled

before allowing continuous slip mode again.

-  ENA_CSLP_TST =  continuous  slip  test  calibration  switch;  0  ->  test

disabled, 1 -> test enabled.

-  ERR_BRK_AWAY = Value of  ERR_T0  (with  no  transients)  that  means  the

converter clutch has broken away.

28-45

CONVERTER CLUTCH CONTROL, MLUS CONTROL AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  ERR_TST_BRK = Max value of ERR_T0 to allow continuous slip test  mode  to

be initiated.

-  LUS_PRD_DT = DUCE LUS period in duce_ticks.

-  MAX_SLIP_DIF = maximum desired slip change (SLIP_DES) allowed before test

transient delay takes effect.

-  MX_SLIP_ERR = maximum ERR_T0 (MLUS PID controller input error) indicating

effective control.

-  MLUS_ERR_LVL = mlus fault threshold for excessive slip error (fault  code

0741).

-  MLUS_MX_TOT = TOT high temp limit to test MLUS,  max  value  of  TOT  for

which  a  detected  converter clutch error will be defined as a fault and

not due to transmission overtemperature.

-  MLUS_NT_DLY = Time allowed between lockup attempts at normal temperature.

-  MLUS_OT_DLY = Time allowed between lockup attempts at high temperature.

-  MLUS_OT_VAL = Maximum counts of attempting Converter Clutch  locks  under

hot conditions before fmem action.

-  MLUS_TDLY_TM = MLUS test initial transient delay time.

-  MLUS_TST_ENA = continuous MLUS test enable switch; 1 -> enable.

-  MLUS_UPCNT = mlus excessive slip error upcount.

-  MX_HISLP_TM = Maximum time slip can be high without being defined  as  an

error.

-  MX_OTSLP_TM = Maximum time slip can be high during transmission  overtemp

mode without being defined as an error.

-  MX_TMSLBRK = Max time between break aways for which the break away  count

will be updated.

-  SLIP_DIF_MX = Max change in slip desired under which a break away will be

recognized.

-  SLP_TDLY_TIM = Slip transient delay time.

-  SW_0741_NMIL = Switch to allow setting a NON-MIL code for a P0741 fault.

-  SW_1744_MIL = Switch to allow setting a MIL code for a P1744 fault.

-  TCC_CTL_SLIP = Maximum amount of TCC slip error (abs[desired  -  actual])

to allow the current lock-up event to be called "good".

-  TCC_CTL_TIM = Amount of time TCC slip must be under control to  call  the

current lock-up event "good".

-  TOT_DES_HF = Temperature to desire half fuel  under  fmem  for  converter

clutch.

28-46

CONVERTER CLUTCH CONTROL, MLUS CONTROL AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TOT_DES_FF = Temperature to desire full fuel  under  fmem  for  converter

clutch.

-  TCC_DC_OFF = Duty cycle below which the  TCC  solenoid  electrically  Off

state will be tested.

-  TCC_DC_ON = Duty cycle above which the TCC solenoid electrically On state

will be tested.

-  TCC_ERR_TIM = Maximum amount of time the mlus output  state  monitor  can

read the wrong state without an error code (0743) being set.

-  TCC_ON_TSTSW = Calibration  switch  used  to  enable/disable  turing  TCC

solenoid On at start-up to test the On state.

-  TP_DIF_MX = Max change in  TP_REL  under  which  a  break  away  will  be

recognized.

-  TQ_DIF_MX = Max change in  TQ_NET  under  which  a  break  away  will  be

recognized.

-  TSSMIN = Minimum turbine speed to assume sensor output is present.

-  VBAT_CCM_MIN = Minimum battery voltage to allow TCC OSM test entry.

-  VS_DES_FF = Vehicle speed to desire full fuel under  fmem  for  converter

clutch.

-  VS_DES_HF = Vehicle speed to desire half fuel under  fmem  for  converter

clutch.

-  VER_BRK_TIM = Min time ERR_T0 must  remain  in  break  away  zone  to  be

counted as a break away.

OTHER

-  mlus_codes = SET OF {P0741, P0743, P1741} the set of  OBDII  fault  codes

that relate to the mlus.

-  malfunction{ccm,Pxxx} = Logic process, imported from MIL control  module;

Pxxx indicates a fault code.

-  P0741 = Fault code, exessive slip detected across the converter clutch.

-  P0743 = Fault code, mlus circuit failure detected (KOEO or CCM).

-  P1741 = Fault code, converter clutch instability (surge) detected by  the

catchnet strategy.

-  store_code(Pxxx) = Logic process, used when NON-MIL faults are detected.

PROCESS

STRATEGY MODULE: CCC_MLUS_OBDII_COM5

28-47

CONVERTER CLUTCH CONTROL, MLUS CONTROL AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Once per background the following logic is executed:

TSTRAT <= 3 ----------------------|

(Manual/non-electric             |

transmission)              |AND -| Do: OSC_RESPONSE(15,10h)

|     | ;send general reject

SUBST_REQ15 = 1 ------------------|     | Exit module

|

| --- ELSE ---

|

TSTRAT <= 3 ----------------------------| Exit module

(Manual/non-electric transmission)     |

| --- ELSE ---

|

| Do: bcsdc_out_calc

| Do: bcsdc_duce_pw

| Do: tcc_h_osm

| Do: mlus_tst

| Do: catchnet

| Exit module

28-48

CONVERTER CLUTCH CONTROL, MLUS CONTROL AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: bcsdc_output

This logic determines the duty cycle to be output to the MLUS as follows:

Normal operation - the duty cycle determined  by  the  converter  clutch

control strategy will be output to the duce.

Engine Off  on-demand  self  test  -  the  MLUS  will  be  commanded  ON

(TCC_DC_ON)  or  OFF  (TCC_DC_OFF)  based  on  the  OSM_EO_ON/OFF  flags

(controlled by the Engine Off OBD-II Sequencer).

unconditionally ------------------------| BCSDC_LST = BCSDC_OUT

| (record last value of duty cycle)

OSM_EO_OFF = 1 -------------------------| bcsdc_out = TCC_DC_OFF

| (Engine Off, OSM off test

|  in progress, command MLUS

|  OFF)

|

| --- ELSE ---

OSM_EO_ON = 1 --------------------|     |

|     |

FLG_FRST_TV = 0 ------------|     |OR --| bcsdc_out = TCC_DC_ON

(no engagement yet)        |     |     |  (Engine Off, OSM on test

|     |     |   in progress; or no engagement

TCC_MON_ON = 0 -------------|AND -|     |   yet, command MLUS ON to test

(On state not monitored)   |           |   the ON state)

|           |

TCC_ON_TSTSW = 1 -----------|           |

(TCC On test cal'ed in)                |

| --- ELSE ---

|

| bcsdc_out = BCSDC

| (Normal operation, output

|  duty cycle determined by

|  MLUS control strategy)

unconditionally ------------------------| Do: substitute(15,bcsdc_out)

| BCSDC_OUT = bcsdc_out

| (perform output state  control

|  override)

END: bscdc_output

28-49

CONVERTER CLUTCH CONTROL, MLUS CONTROL AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: bcsdc_duce_pw

This routine computes the modulated bypass clutch solenoid pulsewidth for the

DUCE chip in units of duce_ticks.

always ---------------------------------| LUS_PW_DTKS = BCSDC_OUT *

|                LUS_PRD_DT

|  (clip to 65535 duce_ticks)

|

| LUS_PW_DUCE = LUS_PW_DTKS

|

| TCC_H_FAULT = fault status from

|  TCC-H UPOD channel.

END: bcsdc_duce_pw

28-50

CONVERTER CLUTCH CONTROL, MLUS CONTROL AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: tcc_h_osm

The Torque Converter Clutch High impedance Solenoid (TCC-H) Circuit is tested

using  feedback from the drive, same logic used for engine off and continuous

self test:

OSM_EO_OFF = 0 -------------|

|

OSM_EO_ON = 0 --------------|AND -|

|     |

CCM_TST_ENA = 0 ------------|     |

(All CCM monitoring disabled)    |

|

BCSDC_OUT <> BCSDC_LST -----------|

(Duty cycle changed)             |OR --| TCC_OSM_TMR = 0

|     | (Comprehensive component

VBAT < VBAT_CCM_MIN --------------|     |  monitor disabled or TCC

(Battry voltage too low)         |     |  operating in mid-range or

|     |  duty cycle changed - wait

BCSDC_OUT > TCC_DC_OFF -----|     |     |  one background for fault

(TCC is not Off)           |AND -|     |  status to update)

|           |

BCSDC_OUT < TCC_DC_ON ------|           |

(TCC not fully On)                     |

| --- ELSE ---

BCSDC_OUT = TCC_DC_ON ------------|     |

(TCC fully On)                   |AND -| TCC_MON_ON = 1

|     | clear_malf(P0743)

TCC_H_FAULT = 0 ------------------|     | TCC_OSM_TMR = 0

(No fault present)                     | (no error - zero error timer)

|

| --- ELSE ---

BCSDC_OUT = TCC_DC_OFF -----------|     |

(TCC fully Off)                  |AND -| TCC_MON_OFF = 1

|     | clear_malf(P0743)

TCC_H_FAULT = 0 ------------------|     | TCC_OSM_TMR = 0

(No fault present)                     | (no error - zero error timer))

|

| --- ELSE ---

|

TCC_OSM_TMR > TCC_ERR_TIM --------------| malfunction(ccm,P0743)

(error present long enough)            | (error detected, set

|  malfunction flag)

P0743MALF = 1 ------------------------|

(TCC circuit failure detected)       |

|

TCC_MON_ON = 1 -----------|           |OR --| P0743MON = 1

(TCC On state monitored) |OR --|     |     | (TCC circuit monitored

|     |     |     |  for the current power-up)

TCC_ON_SW = 0 ------------|     |     |

(Do not count on state         |AND -|

as part of a trip)            |

|

TCC_MON_OFF = 1 ----------------|

(TCC Off state monitored)

END: tcc_h_osm

28-51

CONVERTER CLUTCH CONTROL, MLUS CONTROL AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: mlus_tst

The MLUS is a duty cycled solenoid that controls the converter clutch and  is

controlled  by  the  EEC  MLUS  strategy.   The  MLUS  is  either hardlocked,

softlocked (running continuous slip), or in the  unlocked  state.   When  the

system  is  attempting  to  control  slip  to  a  desired  value (hardlock or

softlock), this test delays testing of the system via MLUS_DLY_TMR until  any

initial  transients have settled.  Once the transient delay time is over, the

test monitors  slip  to  determine  if  the  converter  clutch  is  operating

correctly.

High slip when hard lock or soft lock is commanded can result in two types of

errors:

a)  Error due to a converter clutch  problem  -  unconditionally  unlock  the

converter clutch and fault filter code 0741.

b)  Error due to transmission overtemperature -  unconditionally  unlock  the

converter clutch.

Once a converter clutch error has been detected, the action taken depends  on

the transmission temperature status:

1) If the error is due to normal temperature conditions, the  converter  will

try  to relock after a relatively LONG delay time has expired.  The converter

will also try to relock if a transition from unlock desired to  lock  desired

(based  on  the  schedules)  is detected.  Multiple lockup attempts will only

occur for a calibrated number of times before a fault code is  set.   Once  a

fault  code  is  set,  the  transmission  will  not  be allowed to relock the

converter clutch.

2) If the converter clutch error is  caused  due  to  high  temperature,  the

converter  will try to relock after a set period of time.  After a calibrated

number of failures, the converter will no longer try to  relock  in  overtemp

mode.  No code is set for an overtemp failure.

Note: Once the mlus error code (0741) is set, the converter clutch failure

mode action (unconditional unlock of converter clutch, clipping EPC

to TVFMMN as a minimum, flashing the TCIL) will be executed for a

calibratable number of warm up cycles.

Also, the  only  way  that  the  fault  fiter  for  code  0741  can  be

downcounted is if a transition from locked to unlocked is commanded and

no error has been detected.  (downcount once  for  a  "good"  converter

clutch event)

28-52

CONVERTER CLUTCH CONTROL, MLUS CONTROL AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

failure mode exit logic:

Once a converter clutch fault code has been  stored,  converter  clutch  fmem

action  is  controlled  by  CC_FM_FLG.   This  flag  will  remain  set  for a

calibratable number of warm up cycles; during which the converter clutch will

be unconditionallly unlocked, EPC will be clipped to TVFMMN as a minimum, and

the TCIL will flash (if enabled).

Once CC_FM_FLG is set or the max number of TCC  failures  for  overtemp  have

occured,  the  engine  will  go to half fuel IF vehicle speed and temperature

exceed calibrated values.

P0741FAULT = 1 -------------------|

|

P0741CNT < CC_FM_LVL -------------|

(Execute failure mode action     |AND -|

for a calibratible number       |     |

of warm-up cycles)              |     |

|     |

SW_1744_MIL = 0 ------------------|     |OR --| CC_FM_FLG = 1

(do not light MIL for this fault)       |     | (Execute converter clutch)

|     |  failure mode action)

|     |

P1744MALF = 1 --------------------------|     |

| --- ELSE ---

|

| CC_FM_FLG = 0

For a normal temperature detected error, the flag causing the uncond.  unlock

(MLUS_FM_FLG)  is  cleared after MLUS_NT_DLY timer is downcounted or the next

time the converter clutch lock-unlock schedule is crossed.  For an  overtemp.

detected  failure, the flag causing the unconditional unlock (MLUS_FM_FLG) is

cleared after MLUS_OT_DLY timer is downcounted.  The converter will  continue

to  try  to  relock  when  the  timer downcounts until the number of attempts

exceeds a calibrated amount.

MLUS_FM_FLG = 1 ------------------------|

|

FLG_CRV_DS = 1 -------------------------|

|AND -| MLUS_FM_FLG = 0

|     | (Conditions met

MLUS_NT_TMR = 0 ------|                 |     |  to exit mlus fmem)

|OR --|           |

FLG_CRV_LST = 0 ------|     |           |

|AND -|     |

TOT <= MLUS_MX_TOT ---------|     |     |

|OR --|

MLUS_OT_CNT < MLUS_OT_VAL --|     |

|     |

MLUS_OT_TMR = 0 ------------|AND -|

|

TOT > MLUS_MX_TOT ----------|

28-53

CONVERTER CLUTCH CONTROL, MLUS CONTROL AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

MLUS feedback controller test delay logic:  the MLUS test may be disabled  by

calibrating  MLUS_TST_ENA  to  0.   Tesing of the MLUS feedback controller is

delayed  via  the  MLUS_DLY_TMR,  which  is  cleared  under   the   following

conditions:

P0743MALF = 1 --------------------|

(MLUS electrically               |

malfunctioning)                 |

|

CCM_TST_ENA = 0 ------------------|

(CCM disabled)                   |

|

MLUS_TST_ENA = 0 -----------------|OR --| MLUS_DLY_TMR := 0

(test cal'd out)                 |     | FLG_HIGH_SLP = 0

|     | TCC_CTL_TMR = 0

FLG_SFT_IN = 1 -------------------|     |

(shift in progress)              |     |

|     |

NTBART < TSSMIN ------------------|     |

(Turbine speed unavailable)      |     |

|     |

|SLIP_DES_LST - SLIP_DES|         |     |

> MAX_SLIP_DIF --|     |

(large change in desired slip)         |

| --- ELSE ---

|ERR_T0| > TCC_CTL_SLIP ----------|     |

;Slip is not under control       |OR --| TCC_CTL_TMR = 0

|     | ;TCC does not have

FLG_OPEN = 1 ---------------------|     |  slip under control

;TCC commanded open                    |  OR TCC is open, zero

|  TCC under control timer.

|

| --- ELSE ---

|

TCC_CTL_TMR >= TCC_CTL_TIM -------------| TCC_CTL_FLG = 1

;TCC slip under control long enough    | ;TCC under control

|  for this lock event.

28-54

CONVERTER CLUTCH CONTROL, MLUS CONTROL AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

test logic:

|ERR_T0| <= MX_SLIP_ERR ----------|

|AND -| FLG_HIGH_SLP = 0

FLG_OPEN = 0 ---------------------|     | (Slip is within limits,

(Converter clutch is commanded on)     |  clear the high slip flag)

|

| --- ELSE ---

FLG_HIGH_SLP = 1 -----------------|     |

|     |

MLUS_DLY_TMR > MX_HISLP_TM -------|AND -| MLUS_FM_FLG = 1

|     | FLG_HIGH_SLP = 0

TOT <= MLUS_MX_TOT ---------------|     | MLUS_NT_TMR = MLUS_NT_DLY

| MLUS_ERR_CNT = MLUS_ERR_CNT

|                 + MLUS_UPCNT

| Do: mlus_monitor_logic

| (high slip has lasted long

|  enough to be defined as an

|  error, do fault filtering,

|  unconditionally unlock the

|  converter clutch)

|

| --- ELSE ---

FLG_HIGH_SLP = 1 -----------------|     |

|     |

MLUS_DLY_TMR > MX_OTSLP_TM -------|AND -| MLUS_FM_FLG = 1

|     | FLG_HIGH_SLP = 0

TOT > MLUS_MX_TOT ----------------|     | MLUS_OT_TMR = MLUS_OT_DLY

| MLUS_OT_CNT = MLUS_OT_CNT

|                + MLUS_UPCNT

| (high slip due to

|  overtemperature unconditionally

|  unlock the converter clutch)

|

| --- ELSE ---

FLG_HIGH_SLP = 0 -----------------|     |

|     |

BCSDC >= BCSDC_HRD ---------------|     |

|AND -| FLG_HIGH_SLP = 1

MLUS_DLY_TMR > MLUS_TDLY_TM ------|     | MLUS_DLY_TMR = 0

|     | (high slip detected, reset mlus

|ERR_T0| > MX_SLIP_ERR -----------|     |  timer to allow verification of

|  the error)

| TCC_CTL_FLG = 0

| ;Zero the "good" event flag

|  if a slip error is detected.

|

| --- ELSE ---

(continued on next page)

28-55

CONVERTER CLUTCH CONTROL, MLUS CONTROL AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

MLUS_FM_FLG = 0 ------------------|     |

|     |

TCC_CTL_FLG = 1 ------------------|     |

|AND -| TCC_CTL_FLG = 0

FLG_OPEN = 1 ---------------------|     | ;Only count event once.

|     | MLUS_ERR_CNT = MLUS_ERR_CNT - 1

FLG_OPEN_LST = 0 -----------------|     | Do: mlus_monitor_logic

| (Transition from locked to

|  unlocked commanded, no error

|  detected, and TCC was under

|  control, downcount the fault

|  fiter once for a good event)

Under conditions where the converter clutch fails, it is desired to force the

engine to go to half fuel to deter any more damage to the transmission.

VSBART > VS_DES_HF ---------------------|

(vs high enough to desire half_fuel     |

|AND -| MLUS_HF_FLG =1

TOT > TOT_DES_HF -----------------------|     | (Set flag for half_fuel)

(temp high enough to desire half_fuel)  |     |

|     |

CC_FM_FLG = 1 --------------------|     |     |

(fault code set for CC)           |     |     |

|OR --|     |

MLUS_OT_CNT >= MLUS_OT_VAL -------|           |

(trans too hot to lock CC)                    |

|

VSBART < VS_DES_FF ---------------------|     |

|OR --| MLUS_HF_FLG = 0

TOT < TOT_DES_FF -----------------------|

END: mlus_tst

28-56

CONVERTER CLUTCH CONTROL, MLUS CONTROL AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: mlus_monitor_logic

SW_0741_NMIL = 1 -----------------|

(allow a NON-MIL code for a      |

P0741 fault)                    |

|AND -| P0741MALF = 1

MLUS_ERR_CNT >= MLUS_ERR_LVL -----|     | P0741MON = 1

|     | store_code(P0741)

FLG_SS_MON = 1 -------------------|     | (mlus falut detected)

(Shift validation monitoring           |

complete)                             |

| --- ELSE ---

FLG_SS_MON = 0 -------------------|     |

|OR --| P0741MALF = 0

MLUS_ERR_CNT = 0 -----------------|     | P0741MON = 1

| (mlus monitored or cannot

| perform accurate test)

SW_1744_MIL = 1 ------------------|

(allow a MIL code for            |

this fault)                     |

|AND -| malfunction (CCM,P1744)

MLUS_ERR_CNT >= MLUS_ERR_LVL -----|     | P1744MALF = 1

|     | P1744MON = 1

FLG_SS_MON = 1 -------------------|     |

(Shift validation monitoring           |

complete)                             |

| --- ELSE ---

FLG_SS_MON = 0 -------------------|     |

|OR --| P1744MALF = 0

MLUS_ERR_CNT = 0 -----------------|     | (mlus monitored or cannot

| perform accurate test)

| P1744MON = 1

END: mlus_monitor_logic

28-57

CONVERTER CLUTCH CONTROL, MLUS CONTROL AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: catchnet

Catchnet is designed to dertermine if the converter clutch is  going  out  of

control  (breaks away) during soft lock operation.  In the event that a break

away is detected, a counter is incremented.   If  a  calibratable  number  of

break  aways  have  been  detected  soft  lock  mode  will be disabled (for a

calibratable number of warm up cycles),  and  hard  lock  will  be  commanded

whenever soft lock is asked for by the strategy.

The modulated bypass clutch soft lock test has three modes (ENTRY, TEST,  and

VERIFY).

1.  ENTRY MODE (FLG_TST_BRK = 0, FLG_VER_BRK = 0):

during this mode the test looks  for  slip  to  enter  a  control  region

(SLIP_ABS  - SLIP_DES < ERR_TST_BRK) to determine if the test mode can be

entered.  Upon entering soft lock mode, after a  transient,  or  after  a

break  away, slip must enter the control region before the test mode will

be entered.

2.  TEST MODE (FLG_TST_BRK = 1, FLG_VER_BRK = 0):

Once slip is within the control region, this mode checks that slip  stays

within the expected operating region (SLIP_ABS - SLIP_DES < ERR_BRK_AWAY)

as long as there has been no transient to account for the change in  slip

(the  test  will  return  to  entry  mode if a transient occurs).  If the

converter clutch does break away (SLIP_ABS -  SLIP_DES  >  ERR_BRK_AWAY),

the verify mode is entered.

3.  VERIFY MODE (FLG_TST_BRK = 0, FLG_VER_BRK = 1):

In this mode the time slip enters the break away zone is stored, and  the

test  then  looks  for  slip  to  remain  in  the break away region for a

calibratable amount of time to verify that  a  break  away  has  occurred

(prevent noise from being counted as a break away).  If the break away is

verified this mode looks at the time between break aways to determine  if

the  frequency  of break aways is within the expected range.  if the time

between  break  aways  is  within  range,  the  break  away  counter   is

incremented;  otherwise  the  counter is reset to zero.  The test records

when the break away occurred (saves the time the break away  occurred  at

for the frequency check the next time a break away is verified), and then

returns to entry mode, waiting slip to enter the  control  region  before

testing it again (only count a break away once).

Once a calibratable nunber of break aways have occurred, soft lock mode  will

be disabled until a calibratable number of warm up cycles have occurred.

If too much time has pas since the last break away, the  break  away  counter

will be set to zero.

28-58

CONVERTER CLUTCH CONTROL, MLUS CONTROL AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Disabled continuous slip mode exit logic:

P1741CNT >= ECSM_LVL -------------|

|OR --| FLG_DIS_CSM = 0

P1741FAULT = 0 -------------------|     |

|  (enough warm up cycles have

|   passed to re-enable continous

|   slip mode, or the fault code

|   has been cleared)

Test logic:

Update transient determination conditions  if  slip  is  within  the  control

region:

|ERR_T0| < ERR_TST_BRK -----------|

(Slip within the control region) |AND -| TP_REL_ENT = TP_REL

|     | (TP has decreased, save

TP_REL < TP_REL_ENT --------------|     |  the min. value observed

|  in the control region used

|  to detect transients)

|ERR_T0| < ERR_TST_BRK -----------|

(Slip within the control region) |AND -| TQ_NET_ENT = TQ_NET

|     | (TQ has decreased, save

TQ_NET < TQ_NET_ENT --------------|     |  the min. value observed

|  in the control region, used

|  to detect transients)

28-59

CONVERTER CLUTCH CONTROL, MLUS CONTROL AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CCM_TST_ENA = 0 ------------------|

(CCM disabled)                   |

|

FLG_HT_MOD = 0 -------------------|

(not in continuous slip mode)    |

|

FLG_OT_LK = 1 --------------------|

(in hot lockup mode)             |

|

ENA_CSLP_TST = 0 -----------------|

(test disabled)                  |

|

FLG_PID_CTL = 0 ------------------|

(not in PID control)             |

|

|SLIP_DES - SLIP_DES_LST| >       |

SLIP_DIF_MX --|OR --| SLP_TDLY_TMR = SLP_TDLY_TIM

(Change in desired slip)         |     | BRK_AWAY_CNT = 0

|     | TQ_NET_ENT = TQ_NET

FLG_SFT_IN = 1 -------------------|     | TP_REL_ENT = TP_REL

(Shift in progress)              |     | AC_STATE_ENT = ACCFLG

|     | FLG_TST_BRK = 0

TQ_NET - TQ_NET_ENT > TQ_DIF_MX --|     | FLG_VER_BRK = 0

(Torque has increased enough     |     | (Test entry conditions have not

to cause an increase in slip)   |     |  been met, or a transient has

|     |  occurred, set delay timer and

TP_REL - TP_REL_ENT > TP_DIF_MX --|     |  place the test in entry mode)

(TP has increased enough to      |     |

cause an increase in slip)      |     | EXIT catchnet

|

TP_REL < CNET_TP_MIN -------------|

(TP below min value to allow     |

test entry)                     |

|

ACCFLG <> AC_STATE_ENT -----------|

(AC has changed states)

28-60

CONVERTER CLUTCH CONTROL, MLUS CONTROL AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SLP_TDLY_TMR = 0 -----------------|

(transient delay over)           |

|

FLG_TST_BRK = 0 ------------------|AND -| FLG_TST_BRK = 1

(not in test mode)               |     | FLG_VER_BRK = 0

|     |  (slip has entered the

|ERR_T0| < ERR_TST_BRK -----------|     |   control region, store

(slip is within the test entry region) |   transient determination

|   parameters and initiate

|   test mode)

|

| --- ELSE ---

FLG_TST_BRK = 1 ------------------|     |

(in test mode)                   |AND -| FLG_TST_BRK = 0

|     | FLG_VER_BRK = 1

|ERR_T0| > ERR_BRK_AWAY ----------|     | TM_BRK_AWAY = current time in

(slip is in the break away zone)       |  milliseconds

| TM_BRK_ZONE = 0

|  (possible break away detected,

|   initiate verify mode and store

|   the time at which the break

|   away occurred)

|

| --- ELSE ---

FLG_VER_BRK = 1 ------------------|     |

(in verify mode)                 |AND -| TM_BRK_ZONE = current time in

|     |  milliseconds - TM_BRK_AWAY

|ERR_T0| > ERR_BRK_AWAY ----------|     |  (slip is still in the break away

(slip is in the break away zone)       |   zone, calculate total time in

|   the zone)

|

| --- ELSE ---

|

FLG_VER_BRK = 1 ------------------------| FLG_TST_BRK = 1

(in verify mode)                       | FLG_VER_BRK = 0

|  (slip is no longer in the break

|   away zone return to test mode)

28-61

CONVERTER CLUTCH CONTROL, MLUS CONTROL AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BRK_AWAY_CNT > 0 -----------------------| TMSLBRK = current time in

|  milliseconds - TM_BRK_PREV

|  (calculate time since

|   last break away)

|

| --- ELSE ---

|

| TMSLBRK = 0

|  (break away count is zero,

|   set time since last break

|   away to zero)

NOTE TO SOFTWARE: Please protect for clock roll-overs in the calculation

of TMSLBRK.

FLG_VER_BRK = 1 ------------------|

(in verify mode)                 |

|

TM_BRK_ZONE > VER_BRK_TIM --------|AND -| FLG_VER_BRK = 0

(enough time to verify break     |     | BRK_AWAY_CNT =

away has passed)                |     |  BRK_AWAY_CNT + 1

|     | TM_BRK_PREV = TM_BRK_AWAY

TMSLBRK <= MX_TMSLBRK ------------|     | TM_BRK_ZONE = 0

(time since last break away is         |  (break away detected,

within limit)                         |   frequency of break away

|   is within expected range)

|

| --- ELSE ---

|

TMSLBRK > MX_TMSLBRK -------------------| BRK_AWAY_CNT = 0

BRK_AWAY_CNT > BRK_AWAY_LVL ------------| FLG_DIS_CSM = 1

|  (disable continuous slip

|   mode, allow only hardlock

|   mode)

|

| store_code(P1741)

END: catchnet

28-62

CONVERTER CLUTCH CONTROL, FAILURE MODE MANAGEMENT LOCK-UP - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

28.10  FAILURE MODE MANAGEMENT LOCK-UP (CDAA0)

OVERVIEW

This module determines converter clutch lock-ups during  a  TP  Sensor  or  a

Output  Shaft  Speed Sensor failure.  If the Output Shaft speed sensor fails,

locks are based on a function of NEBART and TP_REL.  If the TP Sensor  fails,

locks are based on SPD_RATIO.

Once an FMEM lock-up is requested, locks continue to be based on this module,

until the next power-up.

Unlocks occur as a result of Unconditional unlocks only.

DEFINITIONS

INPUTS

Registers:

-  GR_CM = Commanded gear.

-  NEBART = Filter engine speed, RPM.

-  SPD_RATIO = Speed Ratio across the torque converter.

Bit Flags:

-  FLG_FMM_LK = Failure Mode Management lock-up flag; 1  ->  lock  converter

due to FMEM action.

-  FLG_FMM_CC = Flag used to insure once FMEM lock-up is  activated,  it  is

not de-activated until next power-up; 0 -> Failure mode lock-up is not in

use this power-up, 1 -> Failure mode lock-up is in use this power-up.

-  FLG_UNC_UNLK = Converter  clutch  unconditional  unlock  flag;  0  ->  no

unconditional unlock, 1 -> unconditional unlock.

-  OSFMFLG = Ouput shaft speed sensor FMEM flag; 0 -> no OSS failure,  1  ->

OSS failure, operating in FMEM mode.

-  TFMFLG = TP FMEM flag; 0 -> no TP failure, 1 -> TP failure, operating  in

FMEM mode.

28-63

CONVERTER CLUTCH CONTROL, FAILURE MODE MANAGEMENT LOCK-UP - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  FN689L(TP_REL) = Engine Speed to lock converter when TP sensor  is  still

available, for Failure Mode Management, RPM.

-  NELK_FM = Engine Speed to lock converter when TP sensor has  failed,  for

Failure Mode Management, RPM.

-  SRLK_FM = Speed Ratio to lock converter for Failure Mode Management.

OUTPUTS

Bit Flags:

-  FLG_FMM_LK = Failure Mode Management lock-up flag; 1  ->  lock  converter

due to FMEM action.

-  FLG_FMM_CC = Flag used to insure once FMEM lock-up is  activated,  it  is

not de-activated until next power-up; 0 -> Failure mode lock-up is not in

use this power-up, 1 -> Failure mode lock-up is in use this power-up.

28-64

CONVERTER CLUTCH CONTROL, FAILURE MODE MANAGEMENT LOCK-UP - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CCC_FMEM_COM3

FLG_FMM_LK = 1 -----------------------------------|

(Failure Mode Lock Requested)                     |AND -| FLG_FMM_LK = 0

|     | (Reset FMEM

FLG_UNC_UNLK = 1 ---------------------------------|     |  lock-up flag)

(Unconditional Unlock Requested)

GR_CM <= 2 ---------------------------------------------| EXIT module

|

| --- ELSE ---

|

OSFMFLG = 1 --------------------------------|           |

(OSS failure)                               |           |

|           |

TFMFLG = 0 ---------------------|           |AND -|     |

(TP Sensor OK)                  |AND -|     |     |     |

|     |     |     |     |

NEBART > FN689L(TP_REL) --------|     |     |     |     |

(Engine Speed above failure           |OR --|     |     |

mode minimum)                        |           |OR --| FLG_FMM_LK = 1

|           |     | FLG_FMM_CC = 1

TFMFLG = 1 ---------------------|     |           |     | (Continue with

(TP Sensor failure)             |AND -|           |     |  FMEM Lock-up

|                 |     |  until next

NEBART > NELK_FM ---------------|                 |     |  power-up)

(Engine Speed above failure                       |     |

mode minimum)                                    |     |

|     |

TFMFLG = 1 ---------------------------|           |     |

(TP Sensor Failure)                   |OR --|     |     |

|     |     |     |

FLG_FMM_CC = 1 -----------------------|     |AND -|     |

(FMEM lock-up activated this power-up)      |           |

|           |

SPD_RATIO > SRLK_FM ------------------------|           |

(Speed Ratio above failure mode minimum)                |

| --- ELSE ---

|

| No Action

28-65

CONVERTER CLUTCH CONTROL, TORQUE CAPACITY DETERMINATION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

28.11  TORQUE CAPACITY DETERMINATION LOGIC (CDAN0)

OVERVIEW

This module determines the appropriate  Torque  Capacity  Calculation  to  be

executed:  "shifting" or "non-shifting" (normal).

DEFINITIONS

Registers:

-  TCAP_INTR1 = Interim requested torque capacity, ft-lbs.

-  TCAP_REQ1 = Final requested torque capacity, ft-lbs

Bit Flags:

-  FLG_CLUP =  Flag  indicating  closed  loop/TCAP_REQ1  freeze  region  for

upshifts.

-  FLG_DNS_CTL = Downshift slip modulation flag; 1  ->  Modifying  TCAP_REQ1

during a downshift.

-  FLG_HRD_JMP = Flag indicating a jump to hardlock based on high TP and  SR

is requested.

-  FLG_HRD_MODE = Flag indicating converter clutch is physically hardlocked.

-  FLG_OLUP = Flag indicating open loop control region for upshifts.

-  FLG_RMP_OPEN = Ramp from hard lock to slip control mode.

-  FLG_SFTTOPID = Flag indicating last background pass was shifting tcap_req

calculation.

-  FLG_STROKE = STROKE mode enabled.

-  FLG_STR_LST = Last pass value of flg_stroke.

-  FLG_UPS_CTL = Upshift slip modulation  flag;  1  ->  Modifying  TCAP_REQ1

during an upshift.

-  FLG_UPS_JMP = Flag indicating jump  to  hardlock  mode  request  -  after

upshift.

Calibration Constants:

-  TCAP_PSTR1 = "Pre-stroke" torque capacity, ft-lbs.

28-66

CONVERTER CLUTCH CONTROL, TORQUE CAPACITY DETERMINATION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CCC_TCAP_DETR_COM2

always ---------------------------------| FLG_STR_LST = FLG_STROKE

FLG_CLUP = 1 ---------------------|

|

FLG_OLUP = 1 ---------------------|

|OR --| FLG_SFTTOPID = 0

FLG_UPS_CTL = 1 ------------------|     | FLG_HRD_MODE = 0

|     | FLG_HRD_JMP = 0

FLG_DNS_CTL = 1 ------------------|     | FLG_UPS_JMP = 0

| FLG_RMP_OPEN = 0

|

| Do: CCC_TCAP_SHFT_CALC_COMx

|    (perform shifting Torque

|     capacity calculation)

|

| FLG_SFTTOPID = 1

|

| --- ELSE ---

|

| Do: CCC_TCAP_NORM_CALC_COMx

|    (perform non-shifting Torque

|     capacity calculation)

| FLG_SFTTOPID = 0

SW_EN_PSTR = 1 -------------------|

|AND -| TCAP_REQ1 =

FLG_CRV_DS = 1 -------------------|     |         max(TCAP_INTR1,TCAP_PSTR1)

(in lock region; apply pre-stroke)     |

| --- ELSE ---

|

| TCAP_REQ1 = TCAP_INTR1

28-67

CONVERTER CLUTCH CONTROL, SHIFTING TORQUE CAPACITY CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

28.12  SHIFTING TORQUE CAPACITY CALCULATION (CDAM0)

OVERVIEW

The Shifting Torque Capacity calculation determines the appropirate value  of

the  interim  requested  Torque  Capacity  (TCAP_INTR1)  during  upshifts  or

downshifts.  Special Torque Capacity calculations outside of  the  open  loop

equation  are necessary due to the lack of turbine speed information during a

shift.  Outside of shift control the open loop equation for  Torque  Capacity

is used.

If not shift is in progress then the "Normal Torque Capacity Control"  module

is executed.

DEFINITIONS

Registers:

-  DELTA_RATIO = Delta shift ratio change since command of shift.

-  DT12S_AVG = Filtered pip delta for transmission, clock ticks.

-  GR_CM = Commanded gear for shift solenoids.

-  NEBART = Filtered engine RPM for transmission.

-  NOBART = Filtered output shaft speed in RPM.

-  RT_GR_CUR = Current transmission gear ratio, dimensionless.

-  SPD_RATIO = Speed ratio across torque converter (output / input).

-  SPD_RT_STRT = Speed ratio across torque converter at start of shift.

-  SR_PP_LIM = Power on upshift speed ratio, pip-to-pip limit.

-  STCF = Seconds-to-ticks coinversion factor.

-  STROKE_TMR = STROKE mode timer, seconds.

-  TCAP_INTR1 = Interim requested Torque Capacity, ft-lbs.

-  TCAP_REQ1 = Final requested torque capacity, ft-lbs.

-  TCAP_SFT_ST1 = Torque Capacity at start of the shift.

-  TQ_BAR = Filtered TQ_NET.

Bit Flags:

-  FLG_DLY_SOSC = Flag used to delay  millisecond  "start  of  shift"  check

until SR_PP_LIM is calculated once; 1 -> allow millisecond check.

28-68

CONVERTER CLUTCH CONTROL, SHIFTING TORQUE CAPACITY CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FLG_CLUP = Flag indicating closed loop/BCSDC freeze region for upshifts.

-  FLG_CRV_LK = Scheduled curve lock-up flag; 0 -> no scheduled lock-up.

-  FLG_DNS_CTL = Downshift slip modulation flag.

-  FLG_OPN_BS = Flag indicating converter open at start  of  upshift;  1  ->

open.

-  FLG_PID_CTL = Modulated Bypass Clutch PID control flag;  1  ->  Modulated

Bypass Clutch in PID control.

-  FLG_STROKE = STROKE mode enabled, BCSDC is held at FNEMGDC1.

-  FLG_TCAPNXT = Set when start of shift  detected  -  causes  execution  of

CCC_SHFT_TCAP_CALC_COMx as next background task.

-  FLG_UCTL_LST = Last pass value of FLG_UPS_CTL.

-  FLG_UPS_CTL = Upshift modulation flag.

-  FLG_FRST_CM = First time a  shift  is  commanded  flag;  0  ->  no  shift

commanded.

-  FLG_SFT_STRT = Set when waiting to detect start of shift  once  its  been

commanded.

Calibration Constants:

-  DSRU2 = Delta Shift Ratio on Upshift to 2nd to indicate shift started.

-  DSRU3 = Delta Shift Ratio on Upshift to 3rd to indicate shift started.

-  DSRU4 = Delta Shift Ratio on Upshift to 4th to indicate start of shift.

-  FNDNSSFT32 = TCAP_REQ1 on 3-2 shifts.

-  FNDNSSFT43 = TCAP_REQ1 on 4-3 shifts.

-  FNUPSSFT23 = TCAP_REQ1 on 2-3 shifts.

-  FNUPSSFT34 = TCAP_REQ1 on 3-4 shifts.

-  PUL_PER_REV = Pulses per revolution number of pips per engine  revolution

for E4OD_GAS 1/2 the number of fuel pump.

-  SW_SUPD = Switch to activate 2nd update of information for c.c.   upshift

control; 1 -> update at end of closed loop control.

-  TCAPSTR = TCAP_REQ used for STROKE mode Capacity.

-  TCAPSTRMULT = Scalar multiplier  of  TQ_BAR  to  determin  stroke  torque

capacity.

28-69

CONVERTER CLUTCH CONTROL, SHIFTING TORQUE CAPACITY CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CCC_TCAP_SHFT_CALC_COM1

(Performed every background loop)

FLG_FRST_CM = 1 -------------|

|

FLG_UPS_CTL = 1 -------|     |OR --| SPD_RATIO = NOBART * RT_GR_CUR / NEBART

|     |     | SPD_RT_STRT = SPD_RATIO

FLG_UCTL_LST = 0 ------|AND -|     | TCAP_SFT_ST1 = TCAP_REQ1

|

SW_SUPD = 1 -----------|

FLG_SFT_STRT = 1 ------------------| SR_PP_LIM = (SPD_RT_STRT + DSRU[GR_CM])

|   * 60 * STCF / (NOBART * RT_GR_CUR *

|   PUL_PER_REV)

| FLG_DLY_SOSC = 1

|  (NOTE to software:  Insure update of

|   FLG_DLY_SOSC follows update of

|   SR_PP_LIM)

(Performed every 1 mS)

FLG_SFT_STRT = 1 ------------|

|

DT12S_AVG > SR_PP_LIM -------|AND -| FLG_SFT_STRT = 0

|     | FLG_TCAPNXT = 1

FLG_DLY_SOSC = 1 ------------|     | FLG_UPS_CTL = 1

| FLG_DLY_SOSC = 0

| TCAP_SFT_ST1 = TCAP_REQ1

28-70

CONVERTER CLUTCH CONTROL, SHIFTING TORQUE CAPACITY CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(Performed in Background Manager BEFORE every Background TASK)

FLG_TCAPNXT = 1 -------------------| Do: Calculate TQ_net as the

|     next background task

| Do: Calculate TCAP_INTR1 as

|     the next background task

| FLG_TCAPNXT = 0

(Performed once per background loop OR when called as next TASK from

background manager)

Calculate TCAP_INTR1:

FLG_UPS_CTL = 1 -------------------| DELTA_RATIO = max( SPD_RATIO -

|                    SPD_RT_STRT, 0)

|

| --- ELSE ---

|

FLG_DNS_CTL = 1 -------------------| DELTA_RATIO = max( SPD_RT_STRT -

|                    SPD_RATIO, 0)

|

| --- ELSE ---

|

| DELTA_RATIO = 0

28-71

CONVERTER CLUTCH CONTROL, SHIFTING TORQUE CAPACITY CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLG_TCAPNXT = 1 -------------|

|

FLG_OPN_BS = 1 --------------|AND -| TCAP_SFT_ST1 = TCAPSTR +

|     |   TQ_BAR * TCAPSTRMULT

FLG_CRV_LK = 1 --------------|     | FLG_PID_CTL = 0

|

| --- ELSE ---

FLG_CRV_LK = 0 --------------|     |

( Includes unconditional    |     |

unlock )                  |OR --| TCAP_INTR1 = -1

|     | FLG_PID_CTL = 0

FLG_OPN_BS = 1 --------|     |     | FLG_STROKE = 0

|AND -|     | STROKE_TMR = 0

FLG_CLUP = 1 ----------|           |

| --- ELSE ---

FLG_UPS_CTL = 1 -------------|     |

|AND -| No Change

FLG_SFT_STRT = 1 ------------|     | FLG_PID_CTL = 0

|

| --- ELSE ---

FLG_UPS_CTL = 1 -------------|     |

|AND -| TCAP_INTR1 = TCAP_SFT_ST1 *

GR_CM <= 3 ------------------|     |          FNUPSSFT23(DELTA_RATIO)

| FLG_PID_CTL = 0

|

| --- ELSE ---

FLG_UPS_CTL = 1 -------------|     |

|AND -| TCAP_INTR1 = TCAP_SFT_ST1

GR_CM = 4 -------------------|     |          FNUPSSFT34(DELTA_RATIO)

| FLG_PID_CTL = 0

|

| --- ELSE ---

FLG_DNS_CTL = 1 -------------|     |

|AND -| TCAP_INTR1 = TCAP_SFT_ST1

GR_CM = 3 -------------------|     |          FNDNSSFT43(DELTA_RATIO)

| FLG_PID_CTL = 0

|

| --- ELSE ---

FLG_DNS_CTL = 1 -------------|     |

|AND -| TCAP_INTR1 = TCAP_SFT_ST1

GR_CM <= 2 ------------------|     |          FNDNSSFT32(DELTA_RATIO)

| FLG_PID_CTL = 0

|

| --- ELSE ---

|

FLG_CLUP = 1 ----------------------| Do: CCC_PID_SLIP_CTL_COMx

|     (perform PID slip control)

| Do: CCC_TCAP_CALC_COMx

|     (calculate torque capacity

|      using the converter model)

|

| --- ELSE ---

|

| no action

28-72

CONVERTER CLUTCH CONTROL, NORMAL TORQUE CAPACITY CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

28.13  NORMAL TORQUE CAPACITY CALCULATION (CDAM0)

OVERVIEW

This module determines the Torque Capacity required at the  converter  clutch

when not shifting.

DEFINITIONS

Registers:

-  BG_TMR = Background loop timer.

-  RMP_OPEN_TMR = Timer used to limit time in ramp open mode.

-  SLP_ABS_CNT = Count of background loops where SLIP_ABS < SLP_ABS_HRD.

-  SLIP_ACT = Actual value of slip, signed.

-  SLIP_DES_S = Desired value of slip, signed.

-  SLIP_TARGET = Target value of slip from tables.

-  SLIP_TCAP  =  Total  desired  slip  to  be  used  for   torque   capacity

calculation.

-  SPD_RATIO = Speed ratio across torque converter (Output / Input).

-  STROKE_SLP = SLIP_ABS at start of STROKE mode.

-  STROKE_TMR = STROKE mode timer.

-  TCAP_INTR1 = Interim requested torque capacity, ft-lbs.

-  TCAP_REQ1 = Final requested torque capacity, ft-lbs.

-  TC_HRD_OFF = Torque capacity offset when in hardlock mode, ft-lbs.

-  TM_OFF_DLY = Converter hardlock torque capacity offset delay timer, secs.

-  TP_REL = Relative TP (TP - RATCH).

-  TQ_BAR = Filtered TQ_NET.

-  TQ_BAR_LST = Last pass value of TQ_BAR, ft-lbs.

Bit Flags:

-  FLG_CLUP = Flag indicating closed loop/BCSDC freeze region for upshifts.

-  FLG_CRV_LK = Scheduled curve lockup flag.

-  FLG_FMM_LK = Failure Mode Management Lock-up flag; 1  ->  lock  converter

due to FMEM action

28-73

CONVERTER CLUTCH CONTROL, NORMAL TORQUE CAPACITY CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FLG_HRD_JMP = Flag indicating a jump to hardlock based on high TP and  SR

is requested.

-  FLG_HRD_LK = Modulated bypass clutch hardlock mode flag; 1 -> In hardlock

mode.

-  FLG_HRD_MODE = Flag indicating converter clutch is physically hardlocked.

-  FLG_NEG_CAP = Flag indication that a negative torque  capacity  has  been

commanded; 1 -> negative capacity commanded.

-  FLG_OLUP = Flag indicating open loop control region for upshifts.

-  FLG_OLUP_LST = Last pass value of FLG_OLUP used to detect transitions.

-  FLG_OT_LK = Transmission overtemperature lockup mode flag.

-  FLG_OPEN = MULS is in open mode; no capacity when set.

-  FLG_OPEN_LST = Last pass value fo FLG_OPEN used to detect transitions.

-  FLG_PID_CTL = Modulated bypass clutch control flag; 1 -> Modulated bypass

clutch in PID control.

-  FLG_RMP_OPEN = Ramp from hard lock to slip control mode.

-  FLG_STROKE = STROKE mode enabled, BCSDC is held at FNEMGDC1.

-  FLG_UNC_UNLK = Converter Clutch uncondtional unlock flag; 0  ->  normally

scheduled MLUS modulation, 1 -> unconditional unlock.

-  FLG_UPS_JMP = Flag indicating jump  to  hardlock  mode  request  -  after

upshift.

Calibration Constants:

-  MXRMPOPENTM = Maximum time to be in ramp open mode.

-  OPNRMPOFF = Constant torque capacity to reset tcap_req  when  going  form

hardlock to slip control.

-  OPNRMPOFFL = Constant torque capacity to reset tcap_req when  going  from

hardlock to slip_control for unlocks.

-  OPNRMPMULT = Multiplier of TQ_BAR  to  reset  tcap_req  when  going  from

hardlock to slip control.

-  OPNRMPMULTL = Multiplier of TQ_BAR to  reset  tcap_req  when  going  from

hardlock to slip_control for unlocks.

-  SLP_ABS_HRD = SLIP_ABS at which BCSDC jumps to BCSDC_HRD is allowed.

-  SLP_ABS_LVL = Value of SLP_ABS_CNT at and above which BCSDC is allowed to

jump to BCSDC_HRD.

-  SLP_HRD_OFF = Converter slip threshold during hardlock to  enable  update

of hardlock torque capacity offset, rpm.

28-74

CONVERTER CLUTCH CONTROL, NORMAL TORQUE CAPACITY CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  SLPRMPOPN = Slip above which ramp open mode is exited.

-  SR_HL = Speed ratio across torque converter at which a jump  to  hardlock

mode is initiated.

-  SR_HL_OT = Speed ratio  across  torque  converter  at  which  a  jump  to

hardlock mode is initiated in overtemp mode.

-  STROKE_DRP = STROKE mode SLIP_ABS drop to exit mode.

-  STROKE_TM = STROKE mode default duration.

-  TCAPOFF = Value by which hardlock torque capacity offset  is  incremented

when converter is slipping, ft-lbs.

-  TCAP_HLK = Scalar multiplier of TQ_BAR to determine hardlock mode.

-  TCAPSTR = TCAP_REQ used for STROKE mode Capacity.

-  TCAPSTRMULT = Scalar multiplier  of  TQ_BAR  to  determin  stroke  torque

capacity.

-  TCOFFDLY1 = Torque transient delay time to  disable  update  of  hardlock

torque capacity offset, secs.

-  TCOFFDLY2 = Minimum time delay between updates  of  the  hardlock  torque

capacity offset, secs.

-  TCRMP1 = TCAP_REQ1 hardlock ramp rate; rate at which TCAP_REQ1 is  ramped

to the hardlock torque capacity.

-  TCRMP2 = TCAP_REQ1 hardlock ramp rate; rate at which TCAP_REQ1 is  ramped

to the hardlock torque capacity.

-  TCRMP3 = TCAP_REQ1 hardlock ramp rate; rate at which TCAP_REQ1 is  ramped

to the hardlock torque capacity.

-  TCRMP10 = tcap_req1 ramp open ramp rate; for a/c hits and softlock.

-  TCRMP10L = tcap_req1 ramp open ramp rate; for unlocks.

-  TP_HL = Relative TP at which a jump to hardlock mode is initiated.

-  TQHRDDIF = Maximum torque change  to  allow  update  of  hardlock  torque

capacity offset, ft-lbs.

28-75

CONVERTER CLUTCH CONTROL, NORMAL TORQUE CAPACITY CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CCC_TCAP_NORM_CALC_COM1

|SLIP_ACT| < SLP_ABS_HRD -----|

|AND -| SLP_ABS_CNT =

FLG_PID_CTL = 1 --------------|     |    min(SLP_ABS_CNT+1,255)

|

| --- ELSE ---

|

| SLP_ABS_CNT = 0

SLP_ABS_CNT >= SLP_ABS_LVL ---------| slp_abs_enb = 1

|

| --- ELSE ---

|

| slp_abs_enb = 0

28-76

CONVERTER CLUTCH CONTROL, NORMAL TORQUE CAPACITY CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TCAP_REQ1 <= 0 ---------------------| FLG_NEG_CAP = 1

(torque capacity negtive)

FLG_OPEN = 0 -----------------|

|

FLG_OPEN_LST = 1 -------------|

|

FLG_NEG_CAP = 1 --------------|AND -| FLG_NEG_CAP = 0

|     | STROKE_TMR = STROKE_TM

FLG_CLUP = 0 -----------------|     | STROKE_SLP = SLIP_ACT

|     |   (transition from OPEN mode with

FLG_OLUP = 0 -----------------|     |   TCAP_REQ1 at OPEN value - Load STROKE

|   timer and grab value of SLIP_ACT)

|

| --- ELSE ---

FLG_OPEN = 1 -----------------|     |

|OR --| STROKE_TMR = 0

STROKE_SLP - SLIP_ACT         |     |  (either STROKE canceled because

> STROKE_DRP ----|     |   OPEN requested, or SLIP_ACT drop

|   seen - clear delay timer)

STROKE_TMR > 0 ---------------------| FLG_STROKE = 1

|  (STROKE mode enabled)

|

| --- ELSE ---

|

| FLG_STROKE = 0

|  (STROKE mode disabled)

TP_REL > TP_HL ---------|

|AND -|

SPD_RATIO > SR_HL ------|     |

|OR --| hrd_jmp_enb = 1

TP_REL > TP_HL ---------|     |     |

|     |     |

FLG_OT_LK = 1 ----------|AND -|     |

|           |

SPD_RATIO > SR_HL_OT ---|           |

| --- ELSE ---

|

| hrd_jmp_enb = 0

28-77

CONVERTER CLUTCH CONTROL, NORMAL TORQUE CAPACITY CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The following logic checks if the converter is slipping during hardlock

mode, and increments the Hardlock offset  (TC_HRD_OFF = TCHRDINT at powerup.)

TM_OFF_DLY > 0 -------------------|

(delay timer not expired)        |OR --| no action

|     |

FLG_HRD_LK = 0 -------------------|     |

(not requesting hardlock)              |

| --- ELSE ---

|

TQ_BAR - TQ_BAR_LST > TQHRDDIF ---------| TM_OFF_DLY = TCOFFDLY1

(torque transient occured)             |

| --- ELSE ---

FLG_HRD_MODE = 1 -----------|           |

|OR --|     |

FLG_HRD_JMP = 1 ------------|     |AND -| TM_OFF_DLY = TCOFFDLY2

|     | TC_HRD_OFF = TC_HRD_OFF +

SLIP_ACT >= SLP_HRD_OFF ----------|     |               TCAPOFF

(converter slipping in hardlock)

FLG_CRV_LK = 0 -----------------------| opnrmpmult = OPNRMPMULTL

(unlock)                              | opnrmpoff = OPNRMPOFFL

| tcrmp10 = TCRMP10L

|

| --- ELSE ---

|

(a/c hits and softlock)               | opnrmpmult = OPNRMPMULT

| opnrmpoff = OPNRMPOFF

| tcrmp10 = TCRMP10

28-78

CONVERTER CLUTCH CONTROL, NORMAL TORQUE CAPACITY CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLG_STROKE = 1 ------------------------------| TCAP_INTR1 = TCAPSTR +

|   TQ_BAR * TCAPSTRMULT

| FLG_PID_CTL = 0

| FLG_HRD_MODE = 0

| FLG_HRD_JMP = 0

| FLG_UPS_JMP = 0

| FLG_RMP_OPEN = 0

|

| --- ELSE ---

FLG_UNC_UNLK = 1 ----------------------|     |

|     |

FLG_OPEN = 1 --------------------|     |OR --| TCAP_INTR1 = -1

|     |     | FLG_PID_CTL = 0

SLIP_DES_S = SLIP_TARGET --|     |AND -|     | FLG_HRD_MODE = 0

|OR --|           | FLG_HRD_JMP = 0

SLIP_TCAP >= SLIP_TARGET --|                 | FLG_UPS_JMP = 0

| FLG_RMP_OPEN = 0

|

| --- ELSE ---

FLG_FMM_LK = 1 ------------------------|     |

|OR --| tcap_intr = TCAP_REQ1 +

FLG_HRD_LK = 1 ------------------|     |     |       (TCRMP1 * BG_TMR)

|AND -|     | TCAP_INTR1 = min(tcap_intr,

slp_abs_enb = 1 -----------|     |           |          (TQ_BAR * TCAP_HLK) +

|OR --|           |           TC_HRD_OFF)

FLG_HRD_MODE = 1 ----------|                 | FLG_PID_CTL = 0

| FLG_HRD_MODE = 1

| FLG_HRD_JMP = 0

| FLG_UPS_JMP = 0

| FLG_RMP_OPEN = 0

|

| --- ELSE ---

FLG_HRD_LK = 1 ------------------------|     |

|AND -| tcap_intr = TCAP_REQ1 +

hrd_jmp_enb = 1 -----------------|     |     |       (TCRMP2 * BG_TMR)

|OR --|     | TCAP_INTR1 = min(tcap_intr,

FLG_HRD_JMP = 1 -----------------|           |          (TQ_BAR * TCAP_HLK) +

|           TC_HRD_OFF)

| FLG_PID_CTL = 0

| FLG_HRD_MODE = 0

| FLG_HRD_JMP = 1

| FLG_UPS_JMP = 0

| FLG_RMP_OPEN = 0

|

| --- ELSE ---

|

(continued on next page)

28-79

CONVERTER CLUTCH CONTROL, NORMAL TORQUE CAPACITY CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

FLG_HRD_LK = 1 ------------------|           |

|           |

FLG_OLUP = 0 --------------------|AND -|     |

|     |     |

FLG_OLUP_LST = 1 ----------------|     |OR --| tcap_intr = TCAP_REQ1 +

|     |              (TCRMP3 * BG_TMR)

FLG_HRD_LK = 1 ------------------|     |     | TCAP_INTR1 = min(tcap_intr,

|AND -|     |          (TQ_BAR * TCAP_HLK) +

FLG_UPS_JMP = 1 -----------------|           |           TC_HRD_OFF)

| FLG_PID_CTL = 0

| FLG_HRD_MODE = 0

| FLG_HRD_JMP = 0

| FLG_UPS_JMP = 1

| FLG_RMP_OPEN = 0

|

| --- ELSE ---

FLG_HRD_MODE = 1 ----------------------|     |

|     |

FLG_HRD_JMP = 1 -----------------------|OR --| TCAP_INTR1 = (opnrmpmult *

|     |            TQ_BAR) + opnrmpoff

FLG_UPS_JMP = 1 -----------------------|     | RMP_OPEN_TMR = 0

(FLG_HRD_LK = 0)                            | FLG_PID_CTL = 0

| FLG_HRD_MODE = 0

| FLG_HRD_JMP = 0

| FLG_UPS_JMP = 0

| FLG_RMP_OPEN = 1

|

| --- ELSE ---

FLG_RMP_OPEN = 1 ----------------------|     |

|     |

SLIP_ACT < SLPRMPOPN ------------------|     |

|AND -| TCAP_INTR1 = TCAP_REQ1 -

TCAP_REQ1 > 0.03125 -------------------|     |            (tcrmp10 * BG_TMR)

|     |

RMP_OPEN_TMR < MXRMPOPENTM ------------|     |

| --- ELSE ---

|

| FLG_HRD_MODE = 0

| FLG_HRD_JMP = 0

| FLG_UPS_JMP = 0

| Do: CCC_PID_SLIP_CTL_COMx

|  (perform PID slip control)

| Do: CCC_TCAP_CALC_COMx

|  (calculate torque capacity

|   using the converter model)

| FLG_RMP_OPEN = 0

28-80

CONVERTER CLUTCH CONTROL, TORQUE CAPACITY CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

28.14  TORQUE CAPACITY CALCULATION (CDAK0)

OVERVIEW

This module calculates the open loop approximation  of  the  torque  capacity

required to achieve a desired slip.

DEFINITIONS

INPUTS

Registers:

-  NTBART = Filtered transmission turbine speed.

-  SLIP_TCAP = Total desired slip to be used to calculate torque capacity.

-  TCAP_CONV1 = Torque being carried by the torque converter; ft-lbs.

-  TQ_BAR = Net torque into torque converter, filtered TQ_NET.

Calibration Constants:

-  K_CONV = Torque converter coefficient that relates input torque to  slip,

multiplied by turbine speed; ft-lbs/rpm-rpm.

OUTPUTS

Registers:

-  TCAP_CONV1 = See above.

-  TCAP_INTR1 = Interim requested torque capacity, ft-lbs.

PROCESS

STRATEGY MODULE: CCC_TCAP_CALC_COM2

always ---------------------------| TCAP_CONV1 = K_CONV * SLIP_TCAP *

|               NTBART

| TCAP_INTR1 = TQ_BAR  - TCAP_CONV1

| TCAP_INTR1 = MAX(TCAP_INTR1,0.03125)

|  (0.03125 is smallest value > 0)

28-81

CONVERTER CLUTCH CONTROL, TORQUE CAPACITY CONVERSION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

28.15  TORQUE CAPACITY CONVERSION (CDAK0)

OVERVIEW

This module determines the bypass clutch PWM duty cycle  required  to  obtain

the  requested  Torque  Capacity (TCAP_REQ1).  This calculation is based only

upon hardware characteristics.

DEFINITIONS

INPUTS

Registers:

-  PWM_OUT = Bypass Clutch PWM output pressure, psi.

-  TCAP_REQ1 = Requested Bypass Clutch Torque Capacity, ft-lbs.

Calibration Constants:

-  FNDCHOFF = Duty cycle intercept when PWM_OUT is greater than or equal  to

PWM_BREAKPNT, function of TOT.

-  FNDCLOFF = Duty cycle intercept when PWM_OUT is less  than  PWM_BREAKPNT,

function of TOT.

-  FNDCHSLP = Duty cycle slope when PWM_OUT is  greater  than  or  equal  to

PWM_BREAKPNT, function of TOT.

-  FNDCLSLP = Duty cycle slope  when  PWM_OUT  is  less  than  PWM_BREAKPNT,

function of TOT.

-  K_CLUTCH = Inverse of converter bypass clutch, gain; psi/lb-ft.

-  K_VALVE = Bypass Clutch control valve pressure gain; psi/psi (unitless).

-  PWM_BREAKPNT = PWM output pressure at which PWM gain changes, psi.

-  PWM_OFFSET = Maximium PWM output pressure at zero (0) pressure difference

across converter clutch; psi.

28-82

CONVERTER CLUTCH CONTROL, TORQUE CAPACITY CONVERSION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OUTPUTS

Registers:

-  BCSDC = Modulated Bypass Clutch Duty Cycle.

-  P_DELTA = Pressure difference across bypass clutch, psi.

-  PWM_OUT = See above.

-  TCAP_REQ1 = See above.

28-83

CONVERTER CLUTCH CONTROL, TORQUE CAPACITY CONVERSION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CCC_TCAP_CONV_COM3

TCAP_REQ1 <= 0 -----------------| BCSDC = 0

| PWM_OUT = 0

| P_DELTA = 0

| EXIT THIS MODULE

Calculate pressure difference (P_DELTA) across converter clutch  required  to

provide TCAP_REQ1, based on clutch size:

always -------------------------| P_DELTA = K_CLUTCH * TCAP_REQ1

Calculate PWM output pressure (PWM_OUT) required to  provide  P_DELTA  across

the  converter  clutch,  based  on  the  bypass clutch control valve hardware

characteristics:

always -------------------------| PWM_OUT = PWM_OFFSET + (K_VALVE * P_DELTA)

Calculate PWM duty cycle (BCSDC) required to provide PWM_OUT  presure,  based

upon solenoid characteristics.  The formula used below is based upon analysis

of solenoid data from the vehicle:

PWM_OUT < PWM_BREAKPNT --------| BCSDC = FNDCLOFF(TOT) +

|   (FNDCLSLP(TOT) * PWM_OUT)

|

| --- ELSE ---

|

| BCSDC = FNDCHOFF(TOT) +

|   (FNDCHSLP(TOT) * PWM_OUT)

28-84

CONVERTER CLUTCH CONTROL, PID SLIP CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

28.16  PID SLIP CONTROL (CDAM0)

OVERVIEW

This module performs closed PID control on slip error and produces a  desired

slip  offset.   This  desired slip offset is added to the normal desired slip

(SLIP_DES_S) which is inputted into the torque capacity equation.

DEFINITIONS

Registers:

-  BG_TMR = Background loop timer.

-  ERR_T0_FD = Filtered value of ERR_T0 for derivative PID slip control.

-  ERR_T0_FI = Filtered value of ERR_T0 for integral PID slip control.

-  ERR_T0_FP = Filtered value of ERR_T0 for proportional PID slip control.

-  ERR_T1 = Last pass slip error.

-  ERR_T0 = Current slip error (SLIP_DES_S - SLIP_ACT).

-  NTBART = Filtered transmission turbine speed.

-  SLIP_ACT = Actual value of slip (signed).

-  SLIP_DES_S = Desired value of slip, signed.

-  SLIP_ERR_D = Derivative slip error.

-  SLIP_ERR_I = Integral slip error.

-  SLIP_ERR_P = Proportional slip error.

-  SLIP_TCAP  =  Total  desired  slip  to  be  used  for   torque   capacity

calculation.

-  SLP_ERR_PID = Total PID slip error.

-  STROKE_TMR = STROKE mode timer.

-  TCAP_REQ1 = Final requested torque capacity, ft-lbs.

-  TQ_BAR = Filtered TQ_NET.

Bit Flags:

-  AC_EN_TRANS = Flag to delay A/C activation process; 0 -> delay.

-  FLG_AC_CTL = Flag indicating when to use A/C comp.  slip target; 1 -> use

target.

28-85

CONVERTER CLUTCH CONTROL, PID SLIP CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FLG_DB_REG = Flag indicating slip is within Deadband Region;  1  ->  Slip

within Deadband Region.

-  FLG_PID_CTL = Modulated bypass clutch control flag; 1 -> Modulated bypass

clutch in PID control.

-  FLG_RMP_OPEN = Ramp from hard lock to slip control mode.

-  FLG_SFTTOPID = Flag indicating last background pass was shifting tcap_req

calculation.

-  FLG_STR_LST = Last pass value of flg_stroke.

Calibration Constants:

-  GAIN_C = Overall PID slip control gain.

-  GAIN_D = Derivative PID slip control gain.

-  GAIN_I = Integral PID slip control gain.

-  GAIN_P = Proportional PID slip control gain.

-  IERRINT = Integral slip error  initialization  value  when  entering  PID

control.

-  IERRMAX = Maximum allowable integral slip error value (SLIP_ERR_I).

-  IERRMIN = Minimum allowable integral slip error value (SLIP_ERR_I).

-  K_CONV = Torque converter coefficient that relates input torque to  slip,

multiplied by turbine speed; ft-lbs/rpm-rpm.

-  SW_AC_PID = Switch to freeze P and I terms while holding slip durring  an

AC hit.

-  SW_DB_D = Switch allow contribution of derivative term when  in  Deadband

Region; 1 -> allow contribution.

28-86

CONVERTER CLUTCH CONTROL, PID SLIP CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: CCC_PID_SLIP_CTL_COM1

FLG_PID_CTL = 0 ------------------|

|

SLIP_DES_S >= 0 ------------------|AND -| FLG_PID_CTL = 1

|     | ERR_T0 = 0

FLG_SFTTOPID = 0 -----------------|     | ERR_T0_FP = 0

|     | ERR_T0_FI = 0

FLG_RMP_OPEN = 0 -----------------|     | ERR_T0_FD = 0

|     | ERR_T1 = 0

FLG_STR_LST = 0 ------------------|     | SLIP_DES_S = SLIP_ACT

| slip_err_i = IERRINT

|  (initialize slip error)

|

| --- ELSE ---

FLG_PID_CTL = 0 ------------------|     |

|     |

SLIP_DES_S >= 0 ------------------|AND -| FLG_PID_CTL = 1

|     | ERR_T0 = 0

FLG_SFTTOPID = 1 -----------|     |     | ERR_T0_FP = 0

|     |     | ERR_T0_FI = 0

FLG_RMP_OPEN = 1 -----------|OR --|     | ERR_T0_FD = 0

|           | ERR_T1 = 0

FLG_STR_LST = 1 ------------|           | SLIP_DES_S = SLIP_ACT

| slip_err_i = (((TQ_BAR - TCAP_REQ1)

|  / (NTBART * K_CONV))

|  - SLIP_DES_S) / GAIN_C

|  (initialize slip error to produce

|   to produce the same TCAP_REQ1 as

|   before the shift)

|

| --- ELSE ---

SLIP_DES_S < 0 -------------------|     |

|OR --| FLG_PID_CTL = 0

STROKE_TMR > 0 -------------------|     |

| --- ELSE ---

|

| slip_err_i = SLIP_ERR_I

28-87

CONVERTER CLUTCH CONTROL, PID SLIP CONTROL - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLG_PID_CTL = 0 ------------------------| SLIP_ERR_P = 0

| slip_err_i = 0

|

| --- ELSE ---

FLG_DB_REG = 1 -------------------|     |

|     |

FLG_AC_CTL = 1 -------------|     |OR --| NO ACTION

|     |     | (this freezes the P & I terms

AC_EN_TRANS = 1 ------------|AND -|     |  when in dead band reigon or

|           |  when holding slip durring

SW_AC_PID = 1 --------------|           |  an AC hit)

|

| --- ELSE ---

|

TCAP_REQ1 > 0.03125 --------------------| SLIP_ERR_P = GAIN_P * ERR_T0_FP

(0.03125 is smallest value > 0)       | inc(slip_err_i, GAIN_I *

|     ERR_T0_FI * BG_TMR)

| (this updates both P & I terms

|  of the PID controler

|

| --- ELSE ---

|

| SLIP_ERR_P = GAIN_P * ERR_T0_FP

|  (this freezes the I term when

|   TCAP_REQx is <= 0.03125 )

always ---------------------------------| SLIP_ERR_I = min(IERRMAX,

|          ((max(slip_err_i,IERRMIN)

|

| (restrict value of integral term)

FLG_PID_CTL = 1 ------------------|

|AND -| SLIP_ERR_D = GAIN_D * ((ERR_T0_FD -

FLG_DB_REG = 0 -------------|     |     |              ERR_T1) / BG_TMR)

|OR --|     |

SW_DB_D = 1 ----------------|           |

| --- ELSE ---

|

FLG_PID_CTL = 0 ------------------------| SLIP_ERR_D = 0

|

| --- ELSE ---

|

| no action

always ---------------------------------| SLP_ERR_PID = GAIN_C * (SLIP_ERR_P

|       + SLIP_ERR_I + SLIP_ERR_D)

|

| SLIP_TCAP = SLIP_DES_S +

|              SLP_ERR_PID

28-88

CHAPTER 29

INPUT CONVERSIONS AND FILTERS

29-1

ENGINE INPUT PROCESSING, NEUTRAL/DRIVE TRANSITION LOGIC - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY _& CONFIDENTIAL

29.1  NEUTRAL/DRIVE TRANSITION LOGIC (CDAN2)

OVERVIEW

The software flags NDSFLG and DNDSUP are the flags used by the engine control

strategy  when  engine  loading  information  is  required.   With manual and

non-EEC controlled automatics, NDSFLG is set and cleared based on  the  state

of the gear/clutch or starter safety switches.  For EEC controlled automatics

with a transmission shaft speed sensor, NDSFLG is set and  cleared  based  on

the manual lever position and transmission shaft speed.

DNDSUP  represents  the  best  approximation  of  the  actual  time  of   the

application  or  removal  of  transmission load (Neutral -> Drive or Drive ->

Neutral transitions).  With manual transmissions, DNDSUP always reflects  the

state  of  NDSFLG.  For automatic transmissions, DNDSUP will be inferred from

|     either turbine speed or time delay functions (FN394F(ECT) & FN394R(ECT)),  or

|     will  directly  reflect  NDSFLG.   If  a turbine speed sensor is present, the

strategy will attempt to use the turbine speed  algorithm.   If  the  turbine

speed  algorithm  cannot  be  used, then the strategy will use the time delay

|     functions to determine DNDSUP.  If this cannot  be  done  either,  due  to  a

|     failed  ECT  sensor, then DNDSUP will reflect the state of NDSFLG.  NDSFLG or

DNDSUP are typically used in idle speed control  mode  select  and  air  flow

computations, fuel enrichment on auto transmission neutral/drive transitions,

adaptive fuel, decel fuel shutoff and  vehicle  speed  control  (as  well  as

selftest).

Raw switch input state (NDSFLG):

-  Manual transmissions can be configured with a clutch and gear  switch,  a

clutch  switch  only,  a  gear  switch only or neither switch.  The input

therefore can be used to determine a neutral state (trans in  neutral  or

clutch depressed) versus in gear state.  If neither clutch or gear switch

is used, the module pull up voltage  provides  an  "in  gear"  indication

which  can  be  overridden  by  proper  selection  of the TRLOAD software

switch.  (Set TRLOAD = 0).

-  Non-EEC controlled automatic  transmissions  typically  use  the  starter

safety  switch  to  indicate neutral or drive.  When the switch is in the

Park or Neutral position, it provides a ground for  the  module  pull  up

voltage  through  the starter relay coil, indicating "neutral".  When not

in Park or Neutral, the switch is open and the pull  up  voltage  is  not

bled off, indicating "drive".

|      -  Electronic automatic transmissions typically use either  a  six  position

|         Manual  Lever  Position  sensor  (MLPS)  or multiple discrete switches to

determine the operator selected gear.  The MLPS is a  ratiometric  sensor

with  six discrete resistors in series.  The sensor is decoded by looking

at the differing voltages produced in each of the  lever  positions.   If

discrete  switches  are  used,  the  lever  position is determined by the

combination of open and closed switches.

29-2

ENGINE INPUT PROCESSING, NEUTRAL/DRIVE TRANSITION LOGIC - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY _& CONFIDENTIAL

DEFINITIONS

Registers:

-  DNDSUP = Delayed neutral drive indication.  For automatic transmissions;

1 -> transmission is in gear,  0 -> transmission is in neutral.

For manual transmissions;  DNDSUP = 0.

|      -  ECT = Coolant temperature, DEGF.

-  INDS = Instantaneous value of the Neutral Drive switch input, counts.

-  NDDTIM = Timer since NDSFLG changed states, sec.

-  REVFLG = Reverse engagement flag; 1 -> reverse, 0 -> drive.

Bit Flags:

-  NDFFLG = Neutral/drive  state  flag,  as  indicated  by  the  time  delay

|         functions (FN394F & FN394R); 1 -> drive.

|      -  NDSFLG = For auto trans; 1 -> MLPS indicates in gear, 0 -> MLPS indicates

|         neutral/park.   For  man  trans;  1  -> clutch out and/or transmission in

|         gear, 0 -> clutch in and/or transmission in neutral.

-  PDL = Manual lever position.

Calibration Constants:

|      -  CFMFLG = Flag indicating that ECT sensor is in/out of range.

|      -  FN394F(ECT) = Time delay, as a function of ECT, before a neutral/park  to

|         in-gear change is reflected in DNDSUP, sec.

|      -  FN394R(ECT) = Time delay, as a function of ECT, before a neutral/drive to

|         reverse transition is reflected in DNDSUP, sec.

-  NDDELT = Time delay before an in-gear to neutral/park change is reflected

in DNDSUP, sec.

-  NDS_SWPT = NDS input switch point.

-  TRLOAD = Transmission Load switch;

0 -> Manual Transmission, no clutch or gear switches,

forced neutral state (NDSFLG = 0).

1 -> Manual Transmission, no clutch or gear switch.

2 -> Manual Transmission, one clutch or gear switch.

3 -> Manual Transmission, both clutch and gear switches.

4 -> Auto Transmission, non-electronic, neutral drive switch.

|                  5 -> Auto Transmission, non-electronic, neutral pressure

switch, (AXOD).

6 -> Auto Transmission, electronic, PRNDL sensor - park,

reverse, neutral, overdrive, manual 1, manual 2.

7 -> Auto Transmission, electronic, multiple discrete switches

for manual lever position. (F4E)

-  TSSHP = Turbine speed sensor hardware present switch; 1 -> present.

29-3

ENGINE INPUT PROCESSING, NEUTRAL/DRIVE TRANSITION LOGIC - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY _& CONFIDENTIAL

-  TSTRAT = Transmission Strategy Switch; 0 -> no transmission control (man,

AOD, ATX, etc.), 1 -> shift indicator.

PROCESS

STRATEGY MODULE: INPUT_NDS_COM3

NDSFLG LOGIC - Determine Neutral/Drive state from raw switch input.

INDS < NDS_SWPT ------------------|

;N/D switch                      |AND -|

|     |

TRLOAD <= 4 ----------------------|     |

|

PDL = 5 OR 7 ---------------------|     |

;MSP indicates                   |     |

|       neutral or park                 |AND -|OR --| Do: nddtim_cntrl_0

|                                       |     |     | NDSFLG := 0

|     TRLOAD >= 6 ----------------------|     |     |  ;neutral state

|      ;MLSP sensor present or                |     |

|      ;automatic switches present            |     |

|                                             |     |

|     TRLOAD = 0 -----------------------------|     |

|      ;forced neutral                              | --- ELSE ---

|                                                   |

|                                                   | Do: nddtim_cntrl_1

|                                                   | NDSFLG := 1

|                                                   |  ;drive/loaded state

|     BEGIN: nddtim_cntrl_0

|     NDSFLG = 1 -----------------------------------| NDDTIM := 0

|                                                   |  ;NDSFLG state is in

|                                                   |  ;transition

|                                                   |  ;zero NDDTIM timer

|                                                   |

|                                                   | --- ELSE ---

|                                                   |

|                                                   | Increment NDDTIM

|     END: nddtim_cntrl_0

|     BEGIN: nddtim_cntrl_1

|     NDSFLG = 0 -----------------------------------| NDDTIM := 0

|                                                   |  ;NDSFLG state is in

|                                                   |  ;transition

|                                                   |  ;zero NDDTIM timer

|                                                   |

|                                                   | --- ELSE ---

|                                                   |

|                                                   | Increment NDDTIM

|     END: nddtim_cntrl_1

29-4

ENGINE INPUT PROCESSING, NEUTRAL/DRIVE TRANSITION LOGIC - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY _& CONFIDENTIAL

REVFLG - FORWARD/REVERSE LOGIC - Applies to NDS or NPS

PDL = 6 --------------------------------|

;manual lever indicates reverse        |AND -| REVFLG := 1

|     |  ;trans in reverse

TSTRAT >= 4 ----------------------------|     |

;electronic trans with MLPS                  | --- ELSE ---

|

| REVFLG := 0

|  ;electronic trans in

|  ;forward

|  ;or manual transmission

NDFFLG LOGIC - Determine Neutral/Drive state from time delay functions.

NDSFLG = 0 -----------------------|

;switch indicates neutral        |AND -|

|     |

NDDTIM >= NDDELT -----------------|     |

;loaded to unloaded delay              |

|

NDSFLG = 1 -----------------------|     |

;switch indicates drive          |     |

|     |

REVFLG = 1 -----------------------|AND -|OR --| NDFFLG := NDSFLG

;in reverse                      |     |     |  ;update delayed

|     |     |  ;neutral/Drive flag

|     NDDTIM >= FN394R(ECT) ------------|     |     |

;neutral to reverse delay              |     |

|     |

NDSFLG = 1 -----------------------|     |     |

;switch indicates drive          |     |     |

|     |     |

REVFLG = 0 -----------------------|AND -|     |

;forward gear                    |           |

|           |

|     NDDTIM >= FN394F(ECT) ------------|           |

;neutral to drive delay                      |

|

| --- ELSE ---

|

| No change to NDFFLG

29-5

ENGINE INPUT PROCESSING, NEUTRAL/DRIVE TRANSITION LOGIC - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY _& CONFIDENTIAL

There are two ways to determine  neutral/drive  transitions.   The  following

|     logic selects which method is used to set DNDSUP:

Automatic Transmissions without turbine speed sensor will use the time  delay

functions.   If  the  time delay functions cannot be used because of a failed

|     ECT sensor, then the raw N/D switch input is used.

Manual Transmissions will always  default  to  the  raw  N/D  (clutch)  input

because TSSHP will be calibrated to zero and TRLOAD will be calibrated <= 3.

Select N/D Transition Detection Method

TRLOAD > 3 -----------------------------|

;automatic transmission                |AND -| DNDSUP := NDFFLG

|     |  ;infer neutral/drive

|     CFMFLG = 0 -----------------------------|     |  ;transitions from

|      ;ECT sensor OK                               |  ;time delay functions

|

| --- ELSE ---

|

| DNDSUP := NDSFLG

|  ;use the raw input from

|  ;the neutral/drive switch

29-6

INPUT CONVERSIONS AND FILTERS, INFERRED BAROMETRIC PRESSURE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

29.2  BAROMETRIC PRESSURE - INFERRED (CDAL0)

OVERVIEW

The barometric pressure is inferred from other sensor inputs.  With air meter

equipped  engines  redundant  sensor  and  actuator  information exists which

allows for the inference of barometric pressure.  The basic equation is:

29.92 * air mass from airmeter

Inferred BP = --------------------------------------------------------

sqrt(560/(ACT+460)) * inferred air mass (29.92,100 ACT)

The model implied by the inferred BP equation is that the inferred  air  mass

is  really  an  air  volume  flow calculation, converted to mass units at the

standard temperature and  pressure.   The  air  meter  is  a  true  air  mass

measurement  device.   Thus  deviations in mass from the inferred calculation

are attributed first to inlet temperature, which is measured and  then  to  a

drop in inlet pressure, which is the inferred BP.

In the EEC the air mass flows are converted to LOAD by the formula:

load = air mass / (N * ENGCYL * SARCHG).

The inferred BP formula in EEC terms is thus:

29.92 * LOAD from airmeter

IBAP = --------------------------

FN059(ACT) * TOTLDST

TOTLDST is the inferred air mass RAM value.   It  is  the  sum  of  the  flow

through  the throttle body, TPLDST and the flow through the air bypass valve,

BYLDST.  TPLDST is a table lookup versus N and TP_REL,  BYLDST  is  the  load

equivalent of BYMAST, the air mass flow through the air bypass valve.  BYMAST

is calculated from a table lookup of air bypass  duty  cycle,  ISCDTY  and  a

calculation of percent load, PCT_LOAD.

For  calibration  purposes,  the  calculated  inferred  value  of  barometric

pressure, can be fixed by using the calibrateable value BAPFMM.

DEFINITIONS

Registers:

-  ACT = Air charge temperature, deg F.

-  BG_TMR = Background loop timer, sec.

-  BP = Barometric Pressure.  (Note:  Upper byte of BP_WORD.)

-  BPUPDTTMR = Timer for determing warm engine, sec.

-  BP_OK_ACC = Time accumulator during which BP is being updated.

29-7

INPUT CONVERSIONS AND FILTERS, INFERRED BAROMETRIC PRESSURE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  DELTOTLD = Change in inferred load (TOTLDST), unitless.

-  ECT = Engine coolant temperature, deg F.

-  IBAP = Unfiltered barometric pressure, inches Hg.

-  LOAD = Normalized inducted air charge, unitless.

-  N = Engine speed, rpm.

-  TOTLDST = Total inferred load, unitless.

-  TP_REL = Relative throttle position, counts.

Bit Flags:

-  BFMFLG = Barometric pressure sensor has failed or inferred BP  value  not

yet established.

-  KAM_ERROR = KAM data not valid; 1 -> KAM data invalid.

-  MAF_INTP_FLG = Flag indicating that MAF signal is not reliable, unitless.

-  MFMFLG = Flag indicating that MAF sensor has failed, unitless.

-  TFMFLG = Flag indicating that TP sensor has failed, unitless.

Calibration Constants:

-  BAPFMM = Default value for barometric pressure used during failure  mode,

inches Hg.

-  BP_CAL_DES = Switch to fix  barometric  pressure  with  the  failure-mode

value  for  calibration  purposes;  0  ->  use calculated value, 1 -> fix

calculated value.

-  BP_OK_TM = Time allowed after KAM reset before clearing BFMFLG.

-  BPSSW = Barometric pressure calculation method switch; 0 ->  inferred,  1

-> sensed.

-  BPUPDENTM = Minimum time since a warm engine state is  reached  to  allow

inference of barometric pressure, sec.

-  DELTOTHI = Maximum allowable change in inferred load to  allow  inference

of barometric pressure, unitless.

-  ECTBPLO = Minimum ECT required to define warm engine state for  inference

of barometric pressure, deg F.

-  FN059(ACT) = Temperature correction factor (square root  of  {560/[ACT  +

460]}), unitless.

-  LDBPLO  =  Minimum  allowable  LOAD  to  allow  inference  of  barometric

pressure, unitless.

-  NBPHI = Maximum allowable engine speed to allow inference  of  barometric

pressure, rpm.

29-8

INPUT CONVERSIONS AND FILTERS, INFERRED BAROMETRIC PRESSURE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  NBPLO = Minimum allowable engine speed to allow inference  of  barometric

pressure, rpm.

-  TPBPLO = Minimum  allowable  throttle  position  to  allow  inference  of

barometric pressure, counts.

-  TPBPHI = Maximum  allowable  throttle  position  to  allow  inference  of

barometric pressure, counts.

PROCESS

STRATEGY MODULE: INPUT_BP_INFERRED_COM1

;This process is executed once per background loop.

BP_CAL_DES = 1 ----------------| BP := BAPFMM

;fixed value desired          |  ;assign failure mode value

|

| Exit

|

| --- ELSE ---

|

BPSSW = 1 ---------------------| Exit

;sensed barometric pressure   |  ;exit this process,

;desired                      |  ;do not execute the logic

|  ;below

|

| --- ELSE ---

|

ECT > ECTBPLO -----------------| Do: bp_inferred_main

;high enough ECT for          |  ;let BPUPDTTMR run freely

;warm engine                  |

| --- ELSE ---

|

| BPUPDTTMR := 0

| Do: bp_inferred_main

29-9

INPUT CONVERSIONS AND FILTERS, INFERRED BAROMETRIC PRESSURE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN:  bp_inferred_main

If operating conditions allow, infer the barometric pressure.  If any  sensor

used in the inference has failed, assign the failure mode value of BP and set

flag BFMFLG, to show failure to infer.

The filter routine is called only after IBAP is updated.   If  no  update  is

made, the filtered value remains unchanged.  This prevents filtering toward a

single occurrence of an inappropriate inference.  This might occur, following

a  transient,  when  the entry condition parameters have not yet been updated

and a poor value of LOAD is used.

The cummulative time taken to infer BP is stored in BP_OK_ACC for use in  the

BFMFLG logic.

29-10

INPUT CONVERSIONS AND FILTERS, INFERRED BAROMETRIC PRESSURE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

MAF_INTP_FLG = 0 --------|

;MAF signal is OK       |

|

TFMFLG = 0 --------------|

;TP sensor is OK        |

|

DELTOTLD < DELTOTHI -----|

;inferred load not      |

;too transient          |

|

N > NBPLO ---------------|

;high enough RPM        |

|

N < NBPHI  --------------|AND -| IBAP := 29.92 * LOAD /

;low enough RPM         |     |        (FN059(ACT) * TOTLDST)

|     |  ;infer barometric pressure

TP_REL > TPBPLO ---------|     |

;high enough TP         |     | Do: bp_filter_main

|     |  ;filter the instantaneous

TP_REL < TPBPHI ---------|     |  ;barometric pressure

;calibration devel only |     |

|     | BP_OK_ACC :=  BP_OK_ACC + BG_TMR

BPUPDTTMR > BPUPDENTM ---|     |  ;conditions OK for

;warm engine long       |     |  ;time accumulation

;enough ECT             |     |

|     | Do: bp_ok

LOAD > LDBPLO -----------|     |

;high enough LOAD             |

| --- ELSE ---

MFMFLG = 1 --------------|     |

;MAF sensor failed      |OR --| IBAP := BAPFMM

|     | BFMFLG := 1

TFMFLG = 1 --------------|     |  ;sensor on which inference

;TP sensor failed             |  ;relies has failed,

|  ;assign failure mode value

|  ;and set failure flag

|

| BP_OK_ACC = 0

|  ;clear accumulator

|

| Do: bp_filter_main

|  ;filter the instantaneous

|  ;barometric pressure

|

| --- ELSE ---

|

| No action

|  ;conditions do not allow inference

|  ;but no sensor failures,

|  ;do not call filtering routine

END: bp_inferred_main

29-11

INPUT CONVERSIONS AND FILTERS, INFERRED BAROMETRIC PRESSURE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: bp_ok

If the KAM is reset, the inferred BP value may not be accurate, especially at

high altitudes.  This could cause problems with the EGR test.  To prevent the

inferred value being used elsewhere before it is established, the BP  failure

flag,  BFMFLG,  is  held  set until a resonable amount of time, BP_OK_TM, for

calculation and filtering to have taken place.

KAM_ERROR = 1 -----------------| BFMFLG := 1

;KAM data invalid             |  ;inferred BP unreliable

| BP_OK_ACC := 0

|  ;clear acculmulator

|

| --- ELSE ---

BFMFLG = 1 --------------|     |

|AND -| BFMFLG := 1

BP_OK_ACC < BP_OK_TM ----|     |  ;time to infer not complete

|

| --- ELSE ---

MFMFLG = 0 --------------|     |

;no MAF sensor fault    |AND -| BFMFLG := 0

|     |  ;clear flag

TFMFLG = 0 --------------|     |  ;inferred value reliable

;no TP sensor fault           |

| --- ELSE ---

|

| No action

END: bp_ok

29-12

INPUT CONVERSIONS AND FILTERS, BAROMETRIC PRESSURE FILTER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

29.3  BAROMETRIC PRESSURE FILTER (CDAL0)

OVERVIEW

This module filters the instantaneous barometric pressure, either  sensed  or

inferred.  The filtered value is clipped to within an allowed range.

DEFINITIONS

INPUTS

Registers:

-  IBAP = Unfiltered barometric pressure, inches Hg.

Bit Flags:

-  KAM_ERROR = Flag indicating Keep Alive Memory is invalid; 1  ->  invalid,

unitless.

Calibration Constants:

-  BAPFMM = Alternate value for barometric pressure used during  failure  or

calibration override, inches Hg.

-  BPMAX = Maximum allowed value for barometric pressure, inches Hg.

-  BPMIN = Minimum allowed value for barometric pressure, inches Hg.

-  TCBBAR = Time constant for barometric pressure filter, sec.

OUTPUTS

Registers:

-  BP = Filtered barometric pressure, inches Hg.

OTHER

-  bp_temp = Filtered, unclipped value of barometric pressure, inches Hg.

29-13

INPUT CONVERSIONS AND FILTERS, BAROMETRIC PRESSURE FILTER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INPUT_BP_FILTER_COM1

BEGIN: bp_filter_main

;This process is executed only when explicitly called.

Since this filter routine is executed only when it is explicitly called,  the

time constant specified below is not related to the passage of absolute time.

Instead, it relates only to the time during  which  the  routine  is  active.

During  such  time,  first  order filtering, with the time constant specified

below, is produced.

KAM_ERROR = 1 --------------------------| BP := BAPFMM

;memory error                          |  ;assign failure mode value

|

| --- ELSE ---

|

| bp_temp := ROLAV(IBAP,TCBBAR)

|  ;filter instantaneous

|  ;BP reading

|

| BP := clip(bp_temp,BPMIN,BPMAX)

|  ;clip to allowed range

END: bp_filter_main

29-14

TRANSMISSION INPUT CONVERSIONS, ETV OVERCURRENT MONITOR VOLTAGE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

29.4  ETV OVERCURRENT MONITOR VOLTAGE (CDAG0)

OVERVIEW

(Performed during A to D conversion)

ML-I Enhanced EEC requires compensation of EPCOCM1 (EPC  overcurrent  monitor

test parameter) for changes in VREF (+/- 5 %) to improve the precision of the

counts value.  This is done by multiplying the A to D  result  (EPCOCM1_CNTS)

by  512/IVCAL,  where 512 is A to D counts for 2.5 volts and IVCAL is the A/D

conversion of a precision voltage that equals 2.5 volts.

With the ML-II Enhanced EEC, compensation of the A to D conversion of the EPC

overcurrent monitor voltage (EPCOCM1_CNTS) is no longer required.  The Analog

Input Conditioner chip (part of the ML-II E-EEC)  eliminates  this  need  for

compensation - the A/D now references a precision voltage.

EPCOCM1 is now set equal to EPCOCM1_CNTS.

DEFINITIONS

INPUTS

Registers:

-  EPCOCM1_CNTS = A to D conversion of the ETV overcurrent monitor  voltage,

counts.

OUTPUTS

Registers:

-  EPCOCM1 = Corrected ETV overcurrent monitor voltage, counts.

PROCESS

STRATEGY MODULE: INPUT_EPCOCM1_COM2

always ---------------------------------| EPCOCM1 = EPCOCM1_CNTS

29-15

INPUT CONVERSIONS AND FILTERS, POWER MODE - GAA0

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

29.5  POWER MODE (CDAA0)

OVERVIEW

This module determines Power-On or Power-Off mode  and  sets  the  flag,

FLG_PWR accordingly.  If the throttle position is greater than the sum of the

minimum throttle position, RATCH, and some calibratable delta,  DELTAT,  then

the engine/transmission is considered to be in the Power-On mode.  Otherwise,

the  engine/transmission  is  considered  to  be  in  the   Power-Off   mode.

Hysteresis is provided for mode stability.

During a throttle position sensor failure, the power mode is  determined

by  the  actual  speed ratio (SPD_RATIO).  If the speed ratio is greater than

some calibratable  minimum,  SPD_PM,  then  the  mode  is  considered  to  be

Power-Off.  Again, hysteresis is provided for mode stability.

DEFINITIONS

INPUTS

Registers:

-  SPD_RATIO = Speed Ratio.

-  RATCH = Kicker off lowest filtered throttle position.

-  TP = Throttle position, counts.

-  TP_REL = Relative Throttle Position, TP - RATCH.

Bit Flags:

-  TFMFLG = Flag indicating TP sensor is in/out of range.

Calibration Constants:

-  DELTAT = Part throttle to closed throttle breakpoint for Power mode.

-  HYSTSPD = Hysteresis for SPD_PM for FMEM Power mode.

-  HYSTST = Hysteresis for DELTAT for Power Mode.

-  SPD_PM = Minimum Speed Ratio for Power-Off mode.

OUTPUTS

Bit Flags:

-  FLG_PWR = Power Mode flag:  1 -> Power-On mode; 0 -> Power-Off mode.

29-16

INPUT CONVERSIONS AND FILTERS, POWER MODE - GAA0

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EQPWRMODE_GA

TFMFLG = 0 --------------------------|

(normal power mode determination)   |

|AND -|

TP_REL <OR= DELTAT ------------|S Q--|     |

|           |

TP_REL > DELTAT + HYSTST ------|C          |

|OR --| Set FLG_PWR = 0

TFMFLG = 1 --------------------------|     |     |  (power off mode)

(FMEM power mode determination)     |     |     |

|AND -|     | --- ELSE ---

SPD_RATIO >OR= SPD_PM ---------|S Q--|           |

|                 | Set FLG_PWR = 1

SPD_RATIO < SPD_PM - HYSTSPD --|C                   (power on mode)

29-17

INPUT CONVERSIONS AND FILTERS, COLD TEMPERATURE TV SOLENOID OPERATION - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

29.6  COLD TEMPERATURE DETERMINATION FOR TV SOLENOID OPERATION (CDAA0)

OVERVIEW

The amount of Engagement TV pressure is a function of  the  Transmission  Oil

Temperature,  TOT.  This logic sets the flag, FLG_TVENG_CD, if the TOT sensor

indicates cold and the flag, FLGTVENG_CD if  moderately  cold.   FLG_TVENG_CD

will  be  cleared  when  the  time  since  the  first  engagement  exceeds  a

calibratable value, even if TOT does not increase.

The amount of start-up TV is a function of TOT also.   This  logic  sets  the

flag, FLG_TVSTR_CD if the TOT sensor indicates cold temperature.

DEFINITIONS

INPUTS

Registers:

-  TOT = Transmission Oil Temperature, deg.  F.

Calibration Constants:

-  TOTTV1 = Maximum TOT to use TVCHRG for start-up, deg F.

-  TOTTV2 = Minimum ECT to use TVEMAX engagement TV, deg F.

-  TOTTV3 = Minimum ECT to use TVEMOD engagement TV, deg F.

OUTPUTS

Bit Flags:

-  FLG_TVENG_CD = Flag which indicates cold temperature for  engagement  TV:

0 -> Don't use TVEMAX in engagement TV; 1 -> Use TVEMAX in engagement TV.

-  FLG_TVENG_MD = Flag which indicates moderate temperature  for  engagement

TV:   0  ->  Don't  use  TVEMOD  in  engagement  TV;  1  -> Use TVEMOD in

engagement TV.

-  FLG_TVSTR_CD = Flag which indicates cold temperature for start-up TV:   0

-> Don't use TVCHRG in start-up TV; 1 -> Use TVCHRG in start-up TV.

29-18

INPUT CONVERSIONS AND FILTERS, COLD TEMPERATURE TV SOLENOID OPERATION - CDAN2

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: EQCTEMP_GA

TOT <= TOTTV1 ----------------------| FLG_TVSTR_CD = 1

|

| --- ELSE ---

|

| FLG_TVSTR_CD = 0

TOT <= TOTTV2 ----------------------| FLG_TVENG_CD = 1

|

| --- ELSE ---

|

| FLG_TVENG_CD = 0

TOT <= TOTTV3 ----------------------| FLG_TVENG_MD = 1

|

| --- ELSE ---

|

| FLG_TVENG_MD = 0

29-19

INPUT CONVERSIONS AND FILTERS, DYNAMIC TV DUE TO COLD TRANSMISSION - GAA0

PED-PTPE, FoMoCo, PROPRIETARY & CONFIDENTIAL

29.7  DYNAMIC TV DUE TO COLD TRANSMISSION (CDAA0)

OVERVIEW

The accumulator pressure  is  the  hydraulic  pressure  used  to  apply,

release,  and  hold the various clutches and bands in the transmission during

shifts.  When the transmission is not shifting,  the  hydraulic  pressure  is

determined  by  the line pressure.  The accumulator pressure is less than the

line pressure.  Therefore,  it  is  necessary  to  add  additional  hydraulic

pressure  via  the  TV  solenoid  during shifts.  This additional pressure is

called "dynamic TV."

The accumulator in the E4OD transmission may stick when the transmission

is  cold.  When the accumulator sticks, the transmission always operates with

accumulator pressure; during shifts and during steady-state gear  conditions.

Of  course,  the  pressure  is inadequate during the steady-state conditions,

since the EEC-IV is not adding the additional TV which is  added  during  the

shifts.   With  inadequate  pressure,  the transmission is unable to transmit

large amounts of torque.

Therefore, logic is needed to also add dynamic TV when conditions exist,

such that the accumulator may stick.

DEFINITIONS

INPUTS

Registers:

-  TOT = Transmission Oil Temperature.

Calibration Constants:

-  TOTTV4 = The maximum temperature where it is no longer necessary  to  add

dynamic TV due to the accumulator sticking.

OUTPUTS

Registers:

-  FLG_DYN_CD = Flag which indicates that it is necessary to add dynamic  TV

due to cold transmission conditions.

PROCESS

TOT < TOTTV4 ----------------------------| Set FLG_DYN_CD = 1

|

| --- ELSE ---

|

| Set FLG_DYN_CD = 0

29-20

ENGINE INPUT PROCESSING, ENGINE SPEED CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

29.8  ENGINE SPEED CALCULATION (CDAM0)

OVERVIEW

The background RPM value  (N)  is  calculated  whenever  the  foreground  has

signalled that PIP information is available, NEW_RPM = 1.  Also at this time,

calculations of the rate of change of RPM (DNDTI)  are  made.   If  the  time

since  last PIP up-edge is large, RPM is set to zero and the ENG_STALLED flag

is set.  ENG_STALLED is used in the background PIP routines.

The calculation of RPM is:

60 Seconds/Min

RPM = -------------------------------------------------------------

Number of PIPs/rev * DT12S Ticks/PIP * Number of seconds/tick

.-----------------------------------------------.

|               | N             | N_BYTE        |

|---------------+---------------+---------------|

| Precision     | double (word) | single (byte) |

| Range         | 0 to 7000+    | 0 to 4080     |

| Resolution    | 0.25          | 16            |

| Initial value | 0             | 0             |

| Units         | RPM           | RPM           |

`-----------------------------------------------'

Note:

If the PIP period becomes >= TSTALL, N and N_BYTE are set to zero.

This ensures that if the PIP signal goes away because of a stall,

N and N_BYTE will become zero to trigger CRANK mode.

DEFINITIONS

Registers:

-  APT = Throttle mode; -1 = closed, 0 = part, 1 = wide open.

-  BG_TMR = Background loop timer, sec.

-  DNDTI = Unfiltered derivative of RPM.

-  DNDT_DAS = Filtered rate of change of RPM for DASPOT, RPM/S.

-  DNDT_IMCC  =  Filtered  rate  of  change  of  RPM  for  intake   manifold

communication control, RPM/S.

-  DNDT_PSS  =  Filtered  rate  of  change  of  RPM  for  predictive   shift

scheduling, RPM/S.

-  DNDT_SPK = Filtered rate of change of RPM for OSCMOD, RPM/S.

29-21

ENGINE INPUT PROCESSING, ENGINE SPEED CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  DT12S = Period between two adjacent rising PIP edges, ticks.

-  N = Engine RPM.

-  NBAR = Filtered engine RPM.

-  NDBAR = Filtered engine speed.

-  NFIL = Filtered engine speed for predictive upshifts.

-  N_BYTE = Byte value of N.

-  N_PREV = Previous value of N.

-  TSLPIP = A timer that indicates  the  time  since  last  PIP  low-to-high

transition.

Bit Flags:

-  ENG_STALLED = Engine stalled (stopped) flag.

-  FIRST_SYNC = Indicates whether the first CID edge has been detected; 1 ->

first CID edge has been detected.

-  FUEL_IN_SYNC = Fuel synchronized with PIP.

-  j1979_01_0C = Current RPM for SCP interface.

-  NEW_RPM = Flag indicating  new  PIP  information  is  available  for  RPM

calculation.

-  SYNFLG = SYNCTR in synchronisation with signature PIP.

Calibration Constants:

-  ENGCYL = Number of PIP's per engine revolution.

-  TCN = Time constant for N, sec.

-  TCNDBR = Time constant for NDBAR, sec.

-  TCNDT_DAS = Time constant for DNDT_DAS, sec.

-  TCNDT_IMCC = Time constant for DNDT_IMCC, sec.

-  TCNDT_PSS = Time constant for DNDT_PSS, sec.

-  TCNDT_SPK = Time constant for DNDT_SPK, sec.

-  TCNFIL = Time constant for NFIL, sec.

-  TSTALL = Time since last PIP before a stall is assumed, msec.

29-22

ENGINE INPUT PROCESSING, ENGINE SPEED CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INPUT_N_CALC_COM2

BEGIN: rpm_main

This is a background task from top downwards.

TSLPIP >= TSTALL ---------| ENG_STALLED := 1

;engine is stopped       | SYNFLG := 0

| FUEL_IN_SYNC := 0

| FIRST_SYNC := 0

| N := 0

| N_BYTE := 0

| N_PREV := 0

| NBAR := 0

| NDBAR := 0

| DNDTI := 0

| NFIL := 0

| DNDT_SPK := 0

| DNDT_DAS := 0

| DNDT_IMCC := 0

| DNDT_PSS := 0

|

| --- ELSE ---

|

NEW_RPM = 1 --------------| ENG_STALLED := 0

;new PIP information     | NEW_RPM := 0

;for calculation of RPM  |  ;clear flags

|

| N_PREV := N

|  ;save previous value

| N := 60 / (ENGCYL * DT12S * tscf)

| N_BYTE := N

|  ;byte value of N

|

| DNDTI := (N - N_PREV) / BG_TMR

|

| NFIL := ROLAV_TC(N,TCNFIL)

| dndtpss := (N - NFIL) / TCNFIL

|

| DNDT_SPK := ROLAV_TC(DNDTI,TCNDT_SPK)

| DNDT_IMCC := ROLAV_TC(DNDTI,TCNDT_IMCC)

| DNDT_DAS := ROLAV_TC(DNDTI,TCNDT_DAS)

| DNDT_PSS := ROLAV_TC(dndtpss,TCNDT_PSS)

|  ;calculate rate of change

|

| NBAR := ROLAV_TC(N, TCN)

| NDBAR := ROLAV_TC(N, TCNDBR)

|  ;RPM data updated this loop

|

| --- ELSE ---

|

| ENG_STALLED := 0

Where: tscf is the ticks to seconds conversion factor.

29-23

ENGINE INPUT PROCESSING, ENGINE SPEED CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

APT >= 0 -----------------| DNDT_DAS := 0

END: rpm_main

BEGIN: pid_definitions

SAE standard j1979  requires  that  a  parameter  for  RPM  (j1979_01_0C)  be

available for the scan tool.

pid_def(j1979_01_0C, N)

END: pid_definitions

29-24

ENGINE INPUT PROCESSING, PIP INPUT ROUTINE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

29.9  PIP INPUT ROUTINE (CDAM0)

OVERVIEW

This module describes PIP related background actions.

The foreground logic first checks to ensure that the edge is "valid" and then

sets  values  for  a  flags  such  as  NEW_RPM  and NEW_PIP that are used for

handshaking with the background calculations.

The background logic calculates some values used by the foreground  (such  as

MIN_PIP_DLY) and reinitializes several parameters when the engine stalls.

The Profile Ignition Pickup (digital input, PIP)  Sensor  is  a  Hall  Effect

Sensor.   The  Hall  Effect  Sensor outputs a square wave pulse producing one

profile pulse of the narrow band of the cycle being sampled.   A  discrepancy

exists between the Hall Effect Sensor and PIP since PIP is not precisely a 50

percent duty cycle.  The  software  has  been  designed  to  accomodate  this

difference providing greater accuracy by use of the KAY factor.

PIP NOISE FILTER

************            *************

*          *            *           *

*          *            *           *

*          *            *           *

*          *            *           *

*          **************           ************

| <-------------------> |

PIP up edge to PIP up edge

filter

| <-----> |

PIP up edge to PIP down edge

filter

In the EEC-IV system, a method of noise blanking to  eliminate  some  of  the

noise  occurs on the PIP input.  A PIP up edge to PIP up edge filter is used.

An interval value is usually picked out of a table and is  in  the  units  of

clock  ticks.   The  equivalent millisecond value of this time corresponds to

some high value of engine rpm.  If a PIP up edge follows a  previous  PIP  up

edge  at  an  interval less than this time, then that PIP up edge is ignored,

and treated as noise.

29-25

ENGINE INPUT PROCESSING, PIP INPUT ROUTINE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

The table value (TABVAL) is divided by four and is used to filter the PIP  up

edge  to PIP down edge interval.  If the computed time from the PIP down edge

to the previous PIP up edge is less than (TABVAL/4), then that PIP down  edge

is ignored.

A typical value for the table value would be the equivalent time interval for

the  PIP  input  at  maximum engine rpm.  For an 8 cylinder engine, the value

would be 2.5 milliseconds (833 clock ticks for 12 mhz).

MNPIPn - PIP FILTER

The PIP filter ignores PIP transitions which occur at a higher rate than  the

maximum  possible  engine  RPM.  The maximum possible engine RPM is generally

assumed to be the RPM at which the valves begin to "float".

MNPIPn is used in the engine speed limiting strategy and is described here.

For example, with four cylinder engine (2 pips/rev) at max RPM of 6500 RPM

with

a clock frequency of 18MHz we have:

60      18 * 1000000

MNPIPn = -------- * ------------

2 * 6500        96

= 865

Base values equivalent to 6500 RPM:

.---------------------------------------------------------.

| MNPIPn        | cylinder (n) | frequency (MHz) | module |

|===============+==============+=================+========|

| MNPIP4  = 865 |      4       |       18        |  8065  |

| MNPIP6  = 576 |      5       |       18        |  8065  |

| MNPIP8  = 432 |      8       |       18        |  8065  |

`---------------------------------------------------------'

Note: MNPIPn should correspond to an RPM which is greater than the speed

at which the strategy turns off the fuel for speed limiting.

DEFINITIONS

Registers:

-  FIRST_PIP = Indicates that first PIP has been received.

-  FIRST_PIP_LO = 1 -> first low PIP has been received.

-  INJ_PIP_CNT1 = Injector PIP counter bank1.

-  INJ_PIP_CNT2 = Injector PIP counter bank2.

29-26

ENGINE INPUT PROCESSING, PIP INPUT ROUTINE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  MIN_PIP_DLY = Minimum PIP delay time filter (blanking).

-  NEW_PIP = New high PIP has occurred.

-  NEW_RPM = New RPM calculation required.

-  TSLPIP = Time since last PIP.

Bit Flags:

-  ENG_STALLED = Flag set to one when  the  engine  has  stalled  (TSLPIP  >

TSTALL).

29-27

ENGINE INPUT PROCESSING, PIP INPUT ROUTINE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INPUT_PIP_BG_COM1

BEGIN:  pip_background

PIP COUNTER CONTROL LOGIC

ENG_STALLED = 1 -----------------| FIRST_PIP := 0

;engine is stopped - no PIP's   | FIRST_PIP_LO := 0

| INJ_PIP_CNT1 := 1

| INJ_PIP_CNT2 := 1

NEW_PIP = 1 ---------------------| TSLPIP := 0

;a PIP has arrived since the    |  ;reset timer

;last background loop           | NEW_PIP := 0

unconditionally -----------------| MIN_PIP_DLY := MNPIPn

|  ;minimum time between valid PIP's

|  ;where n is No. of cyls (4,6,8)

END:  pip_background

29-28

ENGINE INPUT PROCESSING, PIP INPUT ROUTINE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

29.10  PIP INPUT ROUTINE (CDAM0)

OVERVIEW

This module describes foreground actions to be performed on a PIP interupt.

The foreground logic first checks to ensure that the edge is "valid" and then

sets  values  for  a  flags  such  as  NEW_RPM  and NEW_PIP that are used for

handshaking with the background calculations.  Also, a number of time related

parameters are calculated such as DT12S and LAST_HI_PIP.

The background logic calculates some values used by the foreground  (such  as

MIN_PIP_DLY) and reinitializes several parameters when the engine stalls.

DEFINITIONS

Registers:

-  ANPIP1 = Actual number of PIP events.

-  ANPIP2 = Actual number of PIP events.

-  DATA_TIME = 3 byte time of transition.

-  DT12S = The period of time between two adjacent rising edges of PIP.

-  DT23S = Last DT12S time.

-  DT34S = Last DT23S (used in HDR EDIS test).

-  FIRST_PIP = Indicates that first PIP has been received.

-  FIRST_PIP_LO = 1 -> first low PIP has been received.

-  HFDLTA = Time from PIP up to this PIP down edge.

-  LAST_HI_PIP = Time of last high PIP transition.

-  MIN_PIP_DLY = Minimum PIP delay time filter (blanking).

-  N = Engine speed, revolutions per minute, RPM.

-  NEW_PIP = New high PIP has occurred.

-  NEW_RPM = New RPM calculation required.

-  PIPCNT = Number of PIP's which have occurred.

-  PIP_HIGH = PIP input level.

-  TSLPIP = Time since last PIP.

29-29

ENGINE INPUT PROCESSING, PIP INPUT ROUTINE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INPUT_PIP_FG_COM1

PIP edge foreground logic (performed on PIP interrupt)

unconditionally -----------------| pip_delta_time := DATA_TIME - LAST_HI_PIP

|  ;time since the last up edge

PIP up edge foreground logic

PIP_HIGH = 1 --------------|

;up edge                  |AND -| TSLPIP := 0

|     |  ;reset on the first pip

FIRST_PIP = 0 -------------|     | NEW_PIP := 1

;this is the first PIP          |

| Do: valid_hi_pip_processing

|

| --- ELSE ---

PIP_HIGH = 1 --------------|     |

|AND -| NEW_RPM := 1

pip_delta_time >=          |     | NEW_PIP := 1

MIN_PIP_DLY ----|     | DO: valid_hi_pip_processing

;enough time between edges      |

;for valid PIP                  |

| --- ELSE ---

|

PIP_HIGH = 1 --------------------| PIP_HIGH := 0

;up edge, but not a             |  ;ignore edge.

;valid PIP

29-30

ENGINE INPUT PROCESSING, PIP INPUT ROUTINE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PIP down edge foreground logic

PIP_HIGH = 0 --------------|

|

FIRST_PIP = 1 -------------|

;a PIP has been seen      |AND -| HFDLTA := DATA_TIME - LAST_HI_PIP

|     |  ;time from pip up to this

pip_delta_time >=          |     |  ;pip down edge

MIN_PIP_DLY ----|     |

;enough time between edges      | Do: valid_low_pip_processing

;for valid PIP                  |

| --- ELSE ---

|

PIP_HIGH = 0 --------------------| PIP_HIGH := 1

;down edge, but not             |  ;ignore edge.

;a valid PIP

BEGIN: valid_hi_pip_processing

Performed during PIP high interrupt.

unconditionally -----------------| DT34S := DT23S

| DT23S := min(65535, DT12S)

| DT12S := DATA_TIME - LAST_HI_PIP

| LAST_HI_PIP := DATA_TIME

|  ;save time of this edge.

| PIPCNT := PIPCNT + 1

|  ;clip at 255

| ANPIP1 := ANPIP1 + 1

|  ;clip at 255

| ANPIP2 := ANPIP2 + 1

|  ;clip at 255

| FIRST_PIP := 1

END: valid_hi_pip_processing

BEGIN: valid_low_pip_processing

FIRST_PIP_LO = 0 ----------------| FIRST_PIP_LO := 1

| TSLPIP := 0

END: valid_low_pip_processing

29-31

ENGINE INPUT PROCESSING, PIP INPUT ROUTINE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

29-32

CHAPTER 30

TIMERS

30-1

TIMERS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

30.1  TIMER SUMMARY (CDAN0)

TIMER             DESCRIPTION

-----             -----------

GEAR4TMR       Time since 3-4 shift (sec)

NACTMR         Time not at closed throttle (sec)

PRGTMR         Canister purge accumulation timer (sec)

PUTMR          Time since CPU power-up (msec)

RUN_TMR        Time since entering run mode (sec)

TSEGRE         Accumulated time EGR is enabled (sec)

TSLPIP         Time since last PIP (msec)

USPD_RUN_TMR   Time since engine start-up (sec)

DEFINITIONS

Registers:

-  APT = Throttle mode flag; -1 -> closed throttle, 0 -> part throttle, 1 ->

wide open throttle.

-  GEAR4TMR = Time since 3-4 shift, sec.

-  GR_CM = Commanded gear for shift solenoids.

-  N = Engine speed, RPM.

-  NACTMR = Time not at closed throttle (sec)

-  PRGTMR = Canister purge accumulation timer (sec)

-  PUTMR = Time since CPU power-up (msec)

-  RUN_TMR = Time since entering run mode (sec)

-  TCSTRT = Temperature of ECT at cold start-up, deg F.

-  TSEGRE = Accumulated time EGR is enabled (sec)

-  TSLPIP = Time since last PIP (msec)

-  USPD_RUN_TMR = Time since engine start-up, sec.

Bit Flags:

-  EGREN = EGR enabled flag, 1 -> EGR enabled.

30-2

TIMERS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  GEAR4TH = 4th gear flag, 1 -> in 4th gear.

-  NEW_PIP = New PIP rising edge occurred this background loop.

-  UNDSP = Underspeed flag; 1 -> Underspeed mode or Crank.

Calibration Constants:

-  CTLOW = Cold start maximum ECT, deg F.

-  EGRMPT = Egrate ramp time for TCSTRT <= CTLOW, sec.

30-3

TIMERS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: TIMER_CD

GEAR4TMR - TIME SINCE TRANSMISSION 3-4 SHIFT FOR DASPOT DELAY

GR_CM = 4 ------------------------------| GEAR4TH = 1

| count up GEAR4TMR

| clip at maximum value

|

| --- ELSE ---

|

| GEAR4TH = 0

| GEAR4TMR = 0

NACTMR - NOT AT CLOSED THROTTLE TIME

APT = 0 (PART THROTTLE mode)-----|

|OR --| COUNT UP NACTMR

APT = 1 (WIDE OPEN THROTTLE mode)|     |

| --- ELSE ---

|

| NACTMR = 0

PRGTMR - CANISTER PURGE TIMER

CANISTER PURGE OUTPUT ON --------------| COUNT UP PRGTMR

|

| --- ELSE ---

|

| FREEZE PRGTMR

PUTMR - TIME SINCE CPU POWER-UP

CPU POWER ON --------------------------| COUNT UP PUTMR

RUN_TMR - TIME SINCE ENTERING RUN MODE

UNDSP = 0 -----------------------------| Increment RUN_TMR

|

| --- ELSE ---

|

| Freeze RUN_TMR

30-4

TIMERS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TSEGRE - ACCUMULATED TIME EGR IS ENABLED

TCSTRT > CTLOW ------------------------| TSEGRE = EGRMPT ('R' = 1)

|

| --- ELSE ---

|

EGR ENABLED ---------------------------| COUNT UP TSEGRE

(EGREN = 1)                           |  (CLIP AT EGRMPT)

|

| --- ELSE ---

|

| FREEZE TSEGRE

TSLPIP - TIME SINCE LAST PIP

NEW_PIP = 1 ---------------------------| TSLPIP = 0

| NEW_PIP = 0

|

| --- ELSE ---

|

| Count up TSLPIP

Where NEW_PIP is set equal to 1 upon a PIP interrupt.

USPD_RUN_TMR - TIME SINCE ENGINE START-UP (Word)

CRKFLG = 1 ---------------------| USPD_RUN_TMR = 0

(Not RUN or UNDERSPEED mode)    |  (Hold timer to zero)

|

| --- ELSE ---

|

| Allow USPD_RUN_TMR to increment

30-5

TIMERS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

30-6

CHAPTER 31

FAILURE MODE MANAGEMENT

31-1

ENGINE INPUT PROCESSING, ANALOG PRESSURE INPUT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

31.1  ANALOG A/C HEAD PRESSURE INPUT (CDAN0)

FEATURE MEMBERSHIP :  INPUT

EXECUTION CONTEXT :  Background

FUNCTIONAL REQUIREMENTS :

This module converts the analogue input from the  A/C  head  pressure  sensor

into  a  value  of  A/C head pressure in PSI.  It performs diagnostics on the

input and controls the relevant OBDII codes.

This module also controls the flag ACPRES_HSF which is a request for the high

speed  fan  due  to  high  A/C head pressure, and ACPRES_FLG which is the A/C

system disable flag due to excessive head pressure.

FUNCTIONAL DESCRIPTION :

This module processes the A/C head pressure  sensor  input.   IACPRES_SEN  is

read  from an analogue input, checked to diagnose any failures of the sensor,

and the output is a good or failsafe value as ACPRES.

The Open and Short Circuit tests  are  run  every  background  loop,  if  CCM

testing is enabled.  Once a MALF Flag is set, the FMEM value is substituted.

The Insufficient Pressure Change test compares  the  pressure  when  the  A/C

system  is  off,  AC_OFF_PRES,  with  the  pressure  of the system AC_PRES_TM

seconds after the clutch has been engaged.  It looks for a pressure  rise  of

AC_PRES_DLTA.   If  a  rise  of  this magnitude is not seen then P1463_TM_LMT

seconds later the code P1463 will be set and ACPRES will be set to  the  FMEM

value.   If  later  during  the  engagement  of the clutch the pressure rises

sufficiently the code will be cleared and normal operation will continue.

The FMEM value, ACPRES_FMEM, should be set high enough to turn the High speed

fan on, but not so high as to turn the compressor off.

31-2

ENGINE INPUT PROCESSING, ANALOG PRESSURE INPUT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  ACITMR = A/C clutch idle turn on delay timer.

-  ACPRES = A/C Head Pressure.

-  ACPRES_TST = Temporary value of A/C head pressure, used during testing.

-  AC_OFF_PRES = A/C head pressure when the A/C is off, PSI.

-  CCM_EO_ENA = Engine off on demand test for CCM enabled.

-  CCM_ER_ENA = Engine running on demand test for CCM enabled.

-  CCM_TST_ENA = OBD-II Comprehensive Component Test enable flag; 1 -> test

enabled.

-  IACPRES_SEN = A/C pressure, counts, from analogue.

-  P1461_TMR = Timer for fault P1461, Pressure above maximum.

-  P1462_TMR = Timer for fault P1462, Pressure below minimum.

-  P1463_TMR = Timer for fault P1463, Insufficient pressure change.

Bit Flags

-  PxxxMALF = OBDII malfunction flag for fault xxxx; 1 -> a malfunction

currently exists for fault xxxx.

-  ACC_KAM_FLG = KAM flag to signify that AC has been engaged.

-  ACRQST = A/C Requested; 1 -> request.

31-3

ENGINE INPUT PROCESSING, ANALOG PRESSURE INPUT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  ACPSEN_HP = AC Pressure Sensor Hardware Present switch.

-  AC_PRES_DLTA = AC Pressure Delta for code 1463, PSI.

-  ACPRES_CL = A/C head pressure below which A/C can function.

-  ACPRES_SH = A/C head pressure above which A/C should be disabled.

-  ACPRES_FMEM = FMEM value for ACPRES.

-  ACPRE_HSF_SH = A/C head pressure to request high speed fan.

-  ACPRE_HSF_CL = A/C head pressure that High speed fan no longer required.

-  AC_PRES_TM = Time delay before performing A/C pressure rise test.

-  FN703Q(IACPRES_SEN) = A/C head pressure transfer function, for use with

analogue sensor input.

-  IACPRES_MAX = Maximum allowable value of AC Pressure, counts.

-  IACPRES_MIN = Minimum allowable value of AC Pressure, counts.

-  P1461_TM_LMT = Time limit before setting code P1461.

-  P1462_TM_LMT = Time limit before setting code P1462.

-  P1463_TM_LMT = Time limit before setting code P1463.

OUTPUTS

-  ACPRES = A/C head pressure (PSI).

-  ACPRES_FLG = High A/C head pressure, 1 -> Disable A/C system.

-  ACPRES_HSF = Flaf indicating requested status of HSF due to A/C head

pressure.

-  P1461 = A/C Pressure Sensor above maximum.

-  P1462 = A/C Pressure Sensor below minimum.

-  P1463 = A/C pressure sensor insufficient change.

31-4

ENGINE INPUT PROCESSING, ANALOG PRESSURE INPUT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INPUT_ACPRES_COM2

BEGIN: A/C PRESSURE TEST

ACPSEN_HP = 0 --------------------|     | ACPRES := 0

|OR --|  ;Set ACPRES to zero, basic

ACC_KAM_FLG = 0 ------------------|     |  ;strategy assumption

;A/C hardware not present or           |

;A/C system not been run before        | ACPRES_FLG := 0

| ACPRES_HSF := 0

|  ;HSF not required

|

| P1461_TMR := 0

| P1462_TMR := 0

| P1463_TMR := 0

| Clear_malf (P1461,P1462,P1463)

|

| --- ELSE ---

|

| ACPRES_TST := FN703Q(IACPRES_SEN)

|

|  ;Assign a temporary pressure

|  ;and perform testing

|

| DO: AC OFF PRESSURE

| DO: FAILURE CHECKS

| DO: ACPRES ASSIGN

| DO: ACPRES HSF REQUEST

| DO: ACPRES FLAG

END: A/C PRESSURE TEST

BEGIN: AC OFF PRESSURE

ACRQST = 0 -----------------------------| AC_OFF_PRES := ACPRES_TST

;A/C not requested                     |  ;Assign reference value

|  ;for A/C off pressure

|

| --- ELSE ---

|

| NO ACTION

END: AC OFF PRESSURE

31-5

ENGINE INPUT PROCESSING, ANALOG PRESSURE INPUT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: FAILURE CHECKS

CCM_ER_ENA = 0 -------------------|

|     | P1461_TMR := 0

CCM_EO_ENA = 0 -------------------|AND -| P1462_TMR := 0

|     | P1463_TMR := 0

CCM_TST_ENA = 0 ------------------|     |

| ;CCM testing disabled, hold

| ;error timers to zero until

| ;testing resumes

|

| --- ELSE ---

|

IACPRES_SEN <= IACPRES_MAX -------|     | P1461_TMR := 0

|AND -| P1462_TMR := 0

IACPRES_SEN >= IACPRES_MIN -------|     | DO: Clear_malf (P1461,P1462)

|

|  ;No open/short failure,

|  ;clear MALF flags and do

|  ;range checks

|

| DO: AC RANGE CHECKS

|

| --- ELSE ---

IACPRES_SEN > IACPRES_MAX --------|     |

|AND -| DO: Store_code (P1461)

P1461_TMR > P1461_TM_LMT ---------|     | P1462_TMR := 0

| P1463_TMR := 0

| DO: Clear_malf (P1462,P1463)

|  ;Open circuit failure

|

| --- ELSE ---

IACPRES_SEN < IACPRES_MIN --------|     |

|AND -| DO: Store_code (P1462)

P1462_TMR > P1462_TM_LMT ---------|     | P1461_TMR := 0

| P1463_TMR := 0

| DO: Clear_malf (P1461,P1463)

|  ;Closed circuit failure

|

| --- ELSE ---

|

| P1463_TMR := 0

|

|  ;Open/Short failure pending,

|  ;hold range failure timer

|  ;to zero, allow others to

|  ;increment

END: FAILURE CHECKS

31-6

ENGINE INPUT PROCESSING, ANALOG PRESSURE INPUT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: AC RANGE CHECKS

ACITMR < AC_PRES_TM -------------------------| P1463_TMR := 0

|  ;Cannot perform tests yet

|  ;A/C not been on long enough

|  ;hold error timer to zero

|

| --- ELSE ---

|

ACPRES_TST > AC_OFF_PRES + AC_PRES_DLTA -----| P1463_TMR := 0

;Pressure rise OK, no error                 | DO: Clear_malf (P1463)

|

| --- ELSE ---

|

P1463_TMR > P1463_TM_LMT --------------------| DO: Store_code (P1463)

|  ;Fault present long enough

|  ;store code

|

| --- ELSE ---

|

| NO ACTION

|  ;Allow free running timer

|  ;to increment

END: AC RANGE CHECKS

BEGIN: ACPRES ASSIGN

P1461MALF = 1 --------------------|

|

P1462MALF = 1 --------------------|OR --| ACPRES := ACPRES_FMEM

|     |  ;Set ACPRES to FMEM pressure

P1463MALF = 1 --------------------|     |  ;to initiate failure mode action

|

| --- ELSE ---

|

| ACPRES := ACPRES_TST

|  ;Assign ACPRES to correct value

|  ;from sensor for normal operation

END: ACPRES ASSIGN

31-7

ENGINE INPUT PROCESSING, ANALOG PRESSURE INPUT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ACPRES HSF REQUEST

ACPRES > ACPRE_HSF_SH -----------| S Q -| ACPRES_HSF

;High A/C head pressure         |         ;High speed fan required

|

ACPRES < ACPRE_HSF_CL -----------| C

;Fan no longer required

END: ACPRES HSF REQUEST

BEGIN: ACPRES FLAG

ACPRES > ACPRES_SH --------------| S Q -| ACPRES_FLG

;Pressure too high, turn A/C off|

|

ACPRES < ACPRES_CL --------------| C

END: ACPRES FLAG

31-8

ENGINE INPUT PROCESSING, AIR CHARGE TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

31.2  AIR CHARGE TEMPERATURE SENSOR INPUT AND SELFTEST (CDAN2)

OVERVIEW

The A/D is read and the raw counts  (ACT_CNTS)  are  converted  into  deg  F,

(ACT_ENG).   ACT_ENG  is the value used when performing any input testing and

also reported to the scan tool.  Next, the ACT_ENG value is tested  for  "out

of  range"  or  other  failure  conditions.   If the ACT sensor reading is in

range, ACT is set equal to ACT_ENG.  Otherwise, the last "good" value will be

used  for  ACT  until the failure has been present for a sufficient amount of

time.  At that time, the appropriate malfunction flags (P0113MALF,  P0112MALF

and  AFMFLG) are set and the FMEM strategy will substitute either the smaller

of TCSTRT or ACT_FMEM during warm-up or ACT_FMEM thereafter.

The ACT test does not contain a specific monitor  flag  for  input  into  the

OBD-II  Trip  flag.   It  is  assumed  that  when CCM_MON_TIM is reached (see

OBDII_CCM_MON module), a failure, if present, would have been detected.

This module does not contain any specific on-demand (KOEO or KOER) tests.

DEFINITIONS

Registers:

-  ACSTRT = ACT at start-up; arithmetic  average  of  the  first  eight  ACT

readings, deg F.

-  ACSTRT_ACCUM = ACSTRT accumulater, deg F.

-  ACT = Engineering units value with any FMEM corrections applied, deg F.

-  ACT_CNTS = Raw A/D counts from sensor.

-  ACT_ENG = Engineering units value for ACT before any FMEM action, deg  F.

Used for output to the scan tool.

-  ACT_ER_TMR = Timer that runs when a fault  is  present.   This  timer  is

shared by both the high and low failures.

-  ACT_FM_TMR = Timer that is cleared when the failure conditions  for  FMEM

mode are met, sec.

-  ECTCNT = Number of times A/D is read.

-  j1979_01_0F = Inlet air temperature, deg C + 40.  (ie a j1979_01_0f of 70

is equal to an ACT of 30 deg C)

-  PUTMR = Time since power-up, sec.

-  TCSTRT = ECT at startup, deg F.

Bit Flags:

|      -  ACT_FRST_FL = First pass ACT reading flag; 0 -> First pass  for  ACT  A/D

|         reading.

31-9

ENGINE INPUT PROCESSING, AIR CHARGE TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  AFMFLG = Flag indicating that ACT sensor has failed.

-  CCM_EO_ENA = Key-on engine off on-demand test in progress.

-  CCM_ER_ENA = Key-on engine on on-demand test in progress.

-  CCM_TST_ENA = Continuous CCM test enabled.

-  FFG_ACT =  OBD-II  system  FMEM  flag  for  ACT;  1->  ACT  is  currently

unreliable.

-  FFG_ECT = OBD-II system  FMEM  flag  for  ECT;  1  ->  ECT  is  currently

unreliable.

-  OBD_PARM_RST = Flag set whenever OBDII is  reset,  KAM  is  in  error  or

on-demand tests are entered or exited.

-  PxxxMALF = OBD-II malfunction flag for fault  xxx;  1  ->  a  malfunction

currently exists for fault xxx, where Pxxx is a valid OBD-II code.

-  WRMEGO = EGO sensor should be warm; 1 -> warm, 0 -> not warm.

Calibration Constants:

-  ACT_ER_TM = Time with failure present required to set code, sec.

-  ACT_FMEM = Default FMEM value for ACT, deg F.

-  ACT_FM_TM = Minimum time in FMEM mode once initiated, sec.

-  ACT_HI_ER = Maximum acceptable value for ACT, deg F.

-  ACT_LO_ER = Minimum acceptable value for ACT, deg F.

-  ACT_TC = ACT time constant for filtering input.

-  FN703F(ACT_CNTS) = Function to convert from A/D counts to deg  F.   Note:

Since  the  failure mode testing is done in engineering units, the sensor

transfer function MUST  be  calibrated  from  below  ACT_LO_ER  to  above

ACT_HI_ER.

-  NUMEGO = Number of EGO sensors present in system.

-  TKYON2 = Time delay before Key-on updates are permitted, sec.

OTHER

-  act_codes:  Set of {P0113,P0112}.

-  malfunction(ccm,pxxx) = Logic  process  imported  from  the  MIL  control

module, where pxxx is a valid OBD-II code.

-  P0112 = Air charge temp.  (ACT) sensor circuit above maximum voltage/  40

deg F indicated.

-  P0113 = Air charge temp.  (ACT) sensor circuit below minimum voltage/ 254

deg F indicated.

31-10

ENGINE INPUT PROCESSING, AIR CHARGE TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INPUT_ACT_OBDII_COM1

BEGIN:  pid_definitions

unconditionally ------------------------| pid_def(j1979_01_0f,

|    (((ACT_ENG - 32)*5/9) + 40))

|  (deg C + 40)

END: pid_definitions

Initialize ACT at power-up so a "reasonable" value  is  available  until  the

input can be read.

BEGIN:  act_init

power-up initialization ----------------| ACT_ENG := 60

| ACT := 60

END: act_init

BEGIN: act_start_up

OBD_PARM_RST is set whenever OBDII is reset or KAM is in error  or  on-demand

tests  are  entered  or  exited.   At  this  time the error timer needs to be

cleared to prevent codes being carried forward.

CCM_TST_ENA = 0 ------------|

|

CCM_E0_ENA = 0 -------------|AND -|

|     |

CCM_ER_ENA = 0 -------------|     |OR --| ACT_ER_TMR := 0

|     |  ;if test not asked for

OBD_PARM_RST = 1 -----------------|     |  ;clear timer to zero

|

| ACT_ENG := FN703F(ACT_CNTS)

|  ;convert to engineering units

| Do: act_input_check

|  ;check for out of range

|

| --- ELSE ---

|

|  Normal engine operation

|

| ACT_ENG := FN703F(ACT_CNTS)

|  ;convert to engineering units

| Do: act_input_check

|  ;check for out of range

END: act_start_up

31-11

ENGINE INPUT PROCESSING, AIR CHARGE TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: act_input_check

Check for "open/short"  failures.   Perform  FMEM  action  if  a  failure  is

detected.   Set  malfunction  flag  when a malfunction has been present for a

sufficient time.

ACT_ENG > ACT_HI_ER --------------|

;temperature too high            |

|

ACT_ER_TMR > ACT_ER_TM -----------|

;failure present long            |

;long enough                     |AND -| FFG_ACT := 1

|     | AFMFLG := 1

CCM_TST_ENA = 1 ------------|     |     |  ;set failure flags

;continuous test           |     |     | Do: malfunction(ccm,P0112)

|     |     | ACT_FM_TMR := 0

CCM_EO_ENA = 1 -------------|OR --|     |  ;clear failure mode timer

;engine off                |           | Do: act_fmem

;on-demand test            |           |  ;perform FMEM action

|           |

CCM_ER_ENA = 1 -------------|           |

;engine on on-demand test              |

| --- ELSE ---

|

ACT_ENG > ACT_HI_ER --------------------| Do: act_fmem

;temperature too high                  |  ;perform FMEM action

;not present long enough               | FFG_ACT := 1

;to set  malfunction flag              |  ;no change to AFMFLG

|

| --- ELSE ---

|

|

ACT_ENG < ACT_LO_ER --------------|     |

;temperature too low             |     |

|     |

ACT_ER_TMR > ACT_ER_TM -----------|     |

;failure present                 |     |

;long enough                     |AND -| FFG_ACT := 1

|     | AFMFLG := 1

CCM_TST_ENA = 1 ------------|     |     |  ;set failure flag

;continuous test           |     |     | Do: malfunction(ccm,P0113)

|     |     | ACT_FM_TMR := 0

CCM_EO_ENA = 1 -------------|OR --|     |  ;clear failure mode timer

;engine off                |           | Do: act_fmem

;on-demand test            |           |  ;perform FMEM action

|           |

CCM_ER_ENA = 1 -------------|           |

;engine on                             |

;on-demand test                        | --- ELSE ---

|

|                               (continued on next page)

31-12

ENGINE INPUT PROCESSING, AIR CHARGE TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|                            (continued from previous page)

|                                             |

ACT_ENG < ACT_LO_ER --------------------| Do: act_fmem

;temperature too low                   |  ;perform FMEM action

;not present long enough               | FFG_ACT := 1

;to set malfunction flag               |  ;no change to AFMFLG

|

| --- ELSE ---

P0112MALF = 1 --------------|           |

|OR --|     |

P0113MALF = 1 --------------|     |AND -| Do: act_fmem

|     |  ;remain in FMEM mode

ACT_FM_TMR < ACT_FM_TM -----------|     |  ;until it times out

| FFG_ACT := 1

| AFMFLG := 1

|

| --- ELSE ---

|

|     ACT_FRST_FL = 0 ------------------------| ACT := ACT_ENG

|      ;first ACT value                       |  ;initial value for ACT

|                                             |  ;rolling average filter

|                                             | ACT_ER_TMR := 0

|                                             |  ;clear failure present timer

|                                             | ACT_FRST_FL := 1

|                                             | FFG_ACT := 0

|                                             |  ;no failure

|                                             |

|                                             | --- ELSE ---

|                                             |

| ACT := ROLAV_TC(ACT_ENG,ACT_TC)

|  ;no failure detected, return

|  ;value for normal use

| ACT_ER_TMR := 0

|  ;clear failure present timer

| Do: clear_malf(P0112)

| Do: clear_malf(P0113)

|  ;clear failure flags

| Do: acstrt_check

|  ;calculate ACT at start

| FFG_ACT := 0

| AFMFLG := 0

|  ;no failure

END: act_input_check

31-13

ENGINE INPUT PROCESSING, AIR CHARGE TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: act_fmem

FMEM strategy will substitute either the smaller of TCSTRT or ACT_FMEM during

warm-up or ACT_FMEM after warm-up and fault codes set.

WRMEGO = 0 -----------------------|

;start up open loop              |

|AND -| ACT := min(TCSTRT,ACT_FMEM)

NUMEGO <> 0 ----------------------|     |  ;use minimum value

|     |

FFG_ECT = 0 ----------------------|     |

;ECT in range                          |

| --- ELSE ---

P0112MALF = 1 --------------------|     |

|OR --| ACT := ACT_FMEM

P0113MALF = 1 --------------------|     |

| --- ELSE ---

|

| Do not update ACT

END: act_fmem

BEGIN: acstrt_check

The calculation of ACSTRT (ACT at start) is based  on  the  first  eight  ACT

readings.

PUTMR > TKYON2 -------------------|

;avoid noise during              |

;engine cranking                 |AND -| ACSTRT_ACCUM := ACSTRT_ACCUM +

|     |                 (ACT/8)

ECTCNT < 8 -----------------------|     |  ;compute arithemetic average

;accumulate first 8                    |  ;of the first 8 ACT readings

;ECTCNT is calculated                  | ACSTRT := ACT

;in ECT module                         |  ;use current value until ACSTRT

|  ;has been calculated

|

| --- ELSE ---

|

ECTCNT < 8 -----------------------------| ACSTRT := ACT

|  ;use current value until ACSTRT

|  ;has been calculated

|

| --- ELSE ---

|

| ACSTRT := ACSTRT_ACCUM

|  ;load ACSTRT with arithmetic

|  ;average first 8 values

|  ;after PUTMR > TKYON2

END: acstrt_check

31-14

INPUT AND FAILURE MODE AND SELF TEST, AODE INPUT EQUATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

31.3  AODE INPUT EQUATIONS (CDAA0)

OVERVIEW

Once per background loop, the output shaft speed is calculated  if  at  least

one  new  rising  edge occured since the last calculation.  If the time since

the last edge becomes large, the shaft speed is set  to  zero.   Zero  output

shaft speed indicates that the shaft has stopped or the sensor has failed.

The following parameters are calculated primarily by the output  shaft  speed

sensor  (OSS):   NOBAR,  VSBARTL, VSBART_RT.  In the event of an OSS failure,

these parameters will be calculated using the vehicle speed sensor (VSS).

DEFINITIONS

INPUTS

Registers:

-  NO = Unfiltered output shaft speed.

-  NOBART = Filtered output shaft speed for transmission use.

-  RT_NOVS = Ratio of actual N/V to base N/V in KAM.

-  VS = Unfiltered vehicle speed.

-  VSBARV = Filtered vehicle speed for Self-Test.

-  VS_LIM_BAR = Filtered vehicle speed used for vehicle speed limiting.

Bit Flags:

-  OSFMFLG = Output shaft speed sensor failure flag.

Calibration Constants:

-  NVBASE = Base engine speed divided by vehicle speed.

-  TCNO = Time constant for output shaft speed filtering.

-  TCVST = Time constant for filtered vehicle speed.

-  TSTRAT = Transmission type.

31-15

INPUT AND FAILURE MODE AND SELF TEST, AODE INPUT EQUATIONS - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OUTPUTS

Registers:

-  NOBAR = Filtered output shaft speed.

-  NOBART = See above.

-  VSBARTL = Filtered vehicle speed for transmission use.

-  VSBART_RT = Filtered vehicle speed adjusted for RT_NOVS for  transmission

use.

-  VSBARV = see above.

-  VS_SMPL_TM = CLOCK time in milliseconds at the time VSBART is calculated.

PROCESS

STRATEGY MODULE: INPUT_AODE_EQNS_COM1

VEHICLE SPEED PROCESSING

Always ----------------------------| VSBARV = ROLAV(VS,TCVST)

| NOBART = ROLAV(NO,TCNO)

| VS_SMPL_TM = current CLOCK time in

milliseconds

OSFMFLG = 0 -----------------|

(OSS OK)                    |AND -| NOBAR = NOBART

|     | VSBART_RT = NOBART/NVBASE

TSTRAT = 7 ------------------|     |  (clip VSBART_RT to 127.5 max.)

(AODE Transmission)               | VSBARTL = NOBART/(RT_NOVS * NVBASE)

|

| --- ELSE ---

|

| NOBAR = VS_LIM_BAR * NVBASE * RT_NOVS

| VSBART_RT = VSBARV * RT_NOVS

| VSBARTL = ROLAV(VS,TCVST)

31-16

INPUT AND FAILURE MODE AND SELF TEST, BRAKE SWITCH INPUT AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

31.4  BRAKE SWITCH INPUT AND SELF TEST (CDAM0)

OVERVIEW

When the brake switch is depressed, the low speed digital input (BOO_LVL)  is

set  to  one.  To prevent noise falsely triggering this input, the input must

be set to high (1) for at least two reads of the data image.  The brake input

flag  (BIFLG)  is  set  and cleared in background to reflect the state of the

BOO_LVL if there is a brake input to the module.

For the KOEO on-demand test, BIFLG must be low to pass.  For KOER,  the  test

is  to  be  called at the end of the KOER test.  If both states have not been

seen, failure code P1703 is set.

DEFINITIONS

INPUTS

Registers:

-  BOO_LVL = Digital input.

-  ER_STATUS = State pointer that indicated current state of engine  running

on-demand test.

Bit Flags:

-  BIFLG = Current state of BOO input; 0 -> brake off.

-  BOO_KOER_ENA = BOO KOER on demand selftest enabled, to be done at end  of

KOER test.

-  CCM_EO_ENA = Key on engine off on-demand test in progress.

-  OPER_RSP_CHK = Flag to signal that it  is  time  to  check  for  operator

response (BOO, PSPS, etc).

-  V_BOOS_OFF = Off state has been seen since last reset.

-  V_BOOS_ON = On state has been seen since last reset.

Calibration Constants:

-  BIHP = Brake input hardware present switch.

-  ER_INIT = Value for state of ER_STATUS when  engine  running  test  first

inits.

31-17

INPUT AND FAILURE MODE AND SELF TEST, BRAKE SWITCH INPUT AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OUTPUTS

Bit Flags:

-  BIFLG = See above.

-  BIFLG_LST = Last pass value of BIFLG.

-  V_BOOS_OFF = See above.

-  V_BOOS_ON = See above.

OTHER

-  BOO_CODES = Set of {P1703}.

-  store_code(Pxxx) = Stores the fault code specified by Pxxx.

-  P1703 = Brake input high in KOEO or no state change in KOER.

31-18

INPUT AND FAILURE MODE AND SELF TEST, BRAKE SWITCH INPUT AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INPUT_BOO_OBDII_COM1

always ----------------------------| BIFLG_LST := BIFLG

BIHP = 0 --------------------------| BIFLG := 0

;brake hardware not present       |  ;no hardware

|

| --- ELSE ---

BOO_LVL = 1 -----------------|     |

|AND -| BIFLG := 1

|     |  ;brake applied

BIHP = 1 --------------------|     | V_BOOS_ON  := 1

;brake input hardware             |  ;ON state "seen"

;present                          |

| --- ELSE ---

|

| BIFLG := 0

|  ;brake not applied

| V_BOOS_OFF := 1

|  ;OFF state "seen"

ER_STATUS = ER_INIT ---------------| V_BOOS_ON := 0

;engine running on-demand         | V_BOOS_OFF := 0

;test starting now                |  ;restart flags to watch for

|  ;response during on demand test

|

| --- ELSE ---

|

| No action

31-19

INPUT AND FAILURE MODE AND SELF TEST, BRAKE SWITCH INPUT AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BIHP = 0 --------------------------| No action

;brake hardware not present       |

| --- ELSE ---

OPER_RSP_CHK = 1 ------------|     |

;end of KOER test           |     |

|AND -| Do: store_code(P1703)

V_BOOS_ON  = 0 --------|     |     |

;ON state has not     |     |     |

;been seen            |OR --|     |

|           |

V_BOOS_OFF = 0 --------|           |

;OFF state has not been seen      |

| --- ELSE ---

CCM_EO_ENA = 1 --------------|     |

;KOEO on demand test        |AND -| Do: store_code(P1703)

|     |

BIFLG = 1 -------------------|     |

brake "on"                        |

| --- ELSE ---

|

| No action

31-20

ENGINE INPUT PROCESSING, DISTRIBUTORLESS IGNITION SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

31.5  DISTRIBUTORLESS IGNITION SYSTEM/MISFIRE FMEM (CDAL0)

OVERVIEW

When a failure occurs, in the primary side of one of  the  coils  or  in  the

wiring  from the ignition module to one of the coils the ignition module will

not send an IDM signal to the EEC module for the affected cylinders.  Through

logic developed for the Continuous Self Test Strategy, this lack of signal is

identified and classified, in foreground, as to which coil has  suffered  the

failure.   Using  this  information,  an  event counter will determine if the

failure is continuous and if so, will allow a failure mode effects management

(FMEM)  to  be  instituted.   Also  when a misfire is detected by the misfire

detection strategy fmem action for the affected cylinder will be  instituted.

In  either  case only the last two cylinders in the firing sequence that have

been determined to fail will have FMEM action applied to them (e.g.  if on an

eight  cylinder  engine  cylinders  5, 4 and 8 are determined to be misfiring

only 4 and 8 will have FMEM action applied).

The FMEM actions include a strategy that (for sequential fueled systems) will

disable  fuel  to  the  two  cylinders  which  are  not  receiving spark.  In

addition, the fuel can be forced into open loop and the LAMBSE value  can  be

modified  by  the  'C'  multiplier  as  a  function of LOAD.  Additional FMEM

actions can be the use of a spark adder, the disabling of  the  EGR  and  the

disabling of the A/C clutch to reduce the load.

DEFINITIONS

Registers:

-  DISFMEMTMR = Timer that indicates time in DISFMEM.

Bit Flags:

-  DISCUTOUT1 = Denotes one of two injectors which should not be fueled  due

to a DIS primary side failure.

-  DISCUTOUT2 = Denotes one of two injectors which should not be fueled  due

to a DIS primary side failure.

-  DIS_FMEM = Flag indicating that a primary side failure has occurred on  a

DIS system and that an alternate strategy has been requested.

-  MIS200FLG_1 = Misfire on cylinder 1, catalyst temperature criteria.

-  MIS200FLG_2 = Misfire on cylinder 2, catalyst temperature criteria.

-  MIS200FLG_3 = Misfire on cylinder 3, catalyst temperature criteria.

-  MIS200FLG_4 = Misfire on cylinder 4, catalyst temperature criteria.

-  MIS200FLG_5 = Misfire on cylinder 5, catalyst temperature criteria.

-  MIS200FLG_6 = Misfire on cylinder 6, catalyst temperature criteria.

31-21

ENGINE INPUT PROCESSING, DISTRIBUTORLESS IGNITION SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  MIS200FLG_7 = Misfire on cylinder 7, catalyst temperature criteria.

-  MIS200FLG_8 = Misfire on cylinder 8, catalyst temperature criteria.

-  PxxxMALF = Malfunction flag for code Pxxx; 1 -> a  malfunction  currently

exists for fault Pxxx.

Calibration Constants:

-  DISFMEMTM = Time limit on DISFMEM  action.   Allows  for  rechecking  the

presence of the malfunction.

-  DISFMEM_DSRD = A calibratable flag which indicates  whether  FMEM  action

should be taken upon detection of a primary side DIS failure.

-  ENGCYL = The number of PIPs per engine revolution.

-  MISFMEM_DSRD = A calibratable flag which indicates  whether  FMEM  action

should be taken upon detection of a misfire.

-  PAIR_A1 = One of the two  cylinders  using  coil  pack  "A"  in  the  DIS

ignition system.

-  PAIR_A2 = One of the two  cylinders  using  coil  pack  "A"  in  the  DIS

ignition system.

-  PAIR_B1 = One of the two  cylinders  using  coil  pack  "B"  in  the  DIS

ignition system.

-  PAIR_B2 = One of the two  cylinders  using  coil  pack  "B"  in  the  DIS

ignition system.

-  PAIR_C1 = One of the two  cylinders  using  coil  pack  "C"  in  the  DIS

ignition system.

-  PAIR_C2 = One of the two  cylinders  using  coil  pack  "C"  in  the  DIS

ignition system.

-  PAIR_D1 = One of the two  cylinders  using  coil  pack  "D"  in  the  DIS

ignition system.

-  PAIR_D2 = One of the two  cylinders  using  coil  pack  "D"  in  the  DIS

ignition system.

OTHER

-  P0351 = Fault code, Coil A primary failure.

-  P0352 = Fault code, Coil B primary failure.

-  P0353 = Fault code, coil C primary failure.

-  P0354 = Fault code, Coil D primary failure.

31-22

ENGINE INPUT PROCESSING, DISTRIBUTORLESS IGNITION SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INPUT_DIS_COM3

P0351MALF = 1  -------------|

(coil A circuit failure)   |AND -|

|     |

DISFMEM_DSRD = 1 -----------|     |

|OR --| DISCUTOUT1 = PAIR_A1

MIS200FLG_1 = 1 ------------|     |     | DIS_FMEM = 1

(cylinder 1 misfire)       |AND -|

|

MISFMEM_DSRD = 1 -----------|

P0351MALF = 1 --------------|

(coil A circuit failure)   |AND -|

|     |

DISFMEM_DSRD = 1 -----------|     |

|

MIS200FLG_4 = 1 ------------|     |

|     |

ENGCYL = 2 -----------------|AND -|

|     |

MISFMEM_DSRD = 1 -----------|     |

|OR --| DISCUTOUT2 = PAIR_A2

MIS200FLG_5 = 1 ------------|     |     | DIS_FMEM = 1

|     |

ENGCYL = 3 -----------------|AND -|

|     |

MISFMEM_DSRD = 1 -----------|     |

|

MIS200FLG_6 = 1 ------------|     |

|     |

ENGCYL = 4 -----------------|AND -|

|

MISFMEM_DSRD = 1 -----------|

31-23

ENGINE INPUT PROCESSING, DISTRIBUTORLESS IGNITION SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

P0352MALF = 1 --------------|

(coil B circuit failure)   |AND -|

|     |

DISFMEM_DSRD = 1 -----------|     |

|

MIS200FLG_3 = 1 ------------|     |

|     |OR --| DISCUTOUT1= PAIR_B1

ENGCYL = 2 -----------|     |AND -|     | DIS_FMEM = 1

|OR --|     |

ENGCYL = 4 -----------|     |     |

|     |

MISFMEM_DSRD = 1 -----------|     |

|

MIS200FLG_4 = 1 ------------|     |

|     |

ENGCYL = 3 -----------------|AND -|

|

MISFMEM_DSRD = 1 -----------|

P0352MALF ------------------|

(coil B circuit failure)   |AND -|

|     |

DISFMEM_DSRD = 1 -----------|     |

|

MIS200FLG_2 = 1 ------------|     |

|     |

ENGCYL = 2 -----------------|AND -|

|     |

MISFMEM_DSRD = 1 -----------|     |

|OR --| DISCUTOUT2 = PAIR_B2

MIS200FLG_3 = 1 ------------|     |     | DIS_FMEM = 1

|     |

ENGCYL = 3 -----------------|AND -|

|     |

MISFMEM_DSRD = 1 -----------|     |

|

MIS200FLG_5 = 1 ------------|     |

|     |

ENGCYL = 4 -----------------|AND -|

|

MISFMEM_DSRD = 1 -----------|

31-24

ENGINE INPUT PROCESSING, DISTRIBUTORLESS IGNITION SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

P0353MALF = 1  -------------|

(coil C circuit failure)   |AND -|

|     |

DISFMEM_DSRD = 1 -----------|     |

|

MIS200FLG_2 = 1 ------------|     |

|     |

ENGCYL = 3 -----------------|AND -|

|     |

MISFMEM_DSRD = 1 -----------|     |OR --| DISCUTOUT1 = PAIR_C1

|     | DIS_FMEM = 1

MIS200FLG_7 = 1 ------------|     |

|     |

ENGCYL = 4 -----------------|AND -|

|

MISFMEM_DSRD = 1 -----------|

P0353MALF = 1 --------------|

(coil C circuit failure)   |AND -|

|     |

DISFMEM_DSRD = 1 -----------|     |

|

MIS200FLG_6 = 1 ------------|     |

|     |

ENGCYL = 3 -----------------|     |OR --| DISCUTOUT2 = PAIR_C2

|     |     | DIS_FMEM = 1

MISFMEM_DSRD = 1 -----------|     |

|

MIS200FLG_4 = 1 ------------|     |

|     |

ENGCYL = 4 -----------------|AND -|

|

MISFMEM_DSRD = 1 -----------|

31-25

ENGINE INPUT PROCESSING, DISTRIBUTORLESS IGNITION SYSTEM FMEM - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

P0354MALF = 1 --------------|

(coil D circuit failure)   |AND -|

|     |

DISFMEM_DSRD = 1 -----------|     |

|OR -| DISCUTOUT1 = PAIR_D1

MIS200FLG_2 = 1 ------------|     |    | DIS_FMEM = 1

|     |

ENGCYL = 4 -----------------|AND -|

|

MISFMEM_DSRD = 1 -----------|

P0354MALF = 1 --------------|

(coil D circuit failure)   |AND -|

|     |

DISFMEM_DSRD = 1 -----------|     |OR --| DISCUTOUT2 = PAIR_D2

|     | DIS_FMEM = 1

MIS200FLG_8 = 1 ------------|     |

(cylinder 8 misfire)       |     |

ENGCYL = 4 -----------------|AND -|

|

MISFMEM_DSRD = 1 -----------|

The following logic allows injector cutout to remain in effect for DISFMEMTM

seconds.  Injectors will be turned on after FMEM has been in effect DISFMEMTM

seconds.  If the malfunction remains, FMEM will again be inabled.  To

maintain the FMEM action until powerdown, calibrate DISFMEMTM = 255 (sec).

Then DISFMEMTMR will never be greater than DISFMEMTM.

DIS_FMEM = 0 ------------------| DISFMEMTMR := 0

(fmem not requested)          | DISCUTOUT1 := 0

| DISCUTOUT2 := 0

|

| --- ELSE ---

|

DISFMEMTMR > DISFMEMTM --------| DIS_FMEM := 0

(in fmem and the time to      | DISFMEMTMR := 0

be in fmem exceeded)         | MIS200FLG_1 := 0

| MIS200FLG_2 := 0

| MIS200FLG_3 := 0

| MIS200FLG_4 := 0

| MIS200FLG_5 := 0

| MIS200FLG_6 := 0

| MIS200FLG_7 := 0

| MIS200FLG_8 := 0

| DISCUTOUT1 := 0

| DISCUTOUT2 := 0

|

| --- ELSE ---

|

| (fmem to continue, disfmemtmr

|  runs)

31-26

ENGINE INPUT PROCESSING, ENGINE COOLANT TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

31.6  ECT SENSOR INPUT AND SELFTEST (CDAN0)

OVERVIEW

Initial checks are made to determine if an on-demand mode is  activated.   If

no  on-demand mode is active the software falls through and runs the standard

sequence.  The A/D is read and the raw counts (ECT_CNTS) are  converted  into

deg  F.   (ECT_ENG).   ECT_ENG  is  the  value used when performing any input

testing and reported to the scan tool.  Next, the ECT_ENG value is tested for

"out  of range" or other failure conditions.  If the ECT sensor reading is in

range, ECT is set equal to ECT_ENG.  Otherwise, the last "good" value will be

used  for  ECT  until the failure is present for a sufficient amount of time.

Then the appropriate  malfunction  flag  (P0117MALF)  is  set  and  the  FMEM

strategy  substitutes  TOT  or ACT during CRANK or a default value during Run

Mode.

In addition to the "range" checks, a California's OBD-II regulation test  for

the time required to reach feedback temperature is made.  The time allowed is

based on a function (FN654W(INFAMB_KAM)) from inferred  ambient  temperature.

Once  this  test  is  complete (pass or fail) ECT is considered monitored for

this OBD-II trip.

If an on-demand mode is active the KOER or KOEO range tests will be run along

with the standard continuous tests and FMEM action.

DEFINITIONS

Registers:

-  ACT_ENG = Air charge temperature before any FMEM action, deg F.

-  ECT = Engine coolant temperature with FMEM corrections as  required,  deg

F.

-  ECTCNT = Number of times that ECT sensor input was read.

-  ECT_CNTS = Raw A/D counts from sensor.

-  ECT_ENG = Engine coolant temperature before any FMEM  action.   Used  for

FMEM testing and output to scan tool, deg F.

-  TOT_ENG = Transmission oil temperature before any FMEM action, deg F.

-  ECT_ER_TMR = Timer that runs when a fault is present, sec.

-  ECT_FM_TMR = Timer that is reset when the  failure  conditions  for  FMEM

mode are met, sec.

-  ECT_FMEM_TMR = Timer to control ECT ramp to ECT_FMEM rate, sec.

-  ECT_INST = Instantaneous ECT (no rolling average) with FMEM substitutions

if required.

31-27

ENGINE INPUT PROCESSING, ENGINE COOLANT TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  INFAMB_KAM = Inferred ambient air temperature, deg F.

-  PUTMR = Time since power-up, sec.

-  TCSTRT = ECT at start-up; arithmetic  average  of  the  first  eight  ECT

readings.

-  TCSTRT_ACCUM = TCSTRT accumulator, deg F.

-  USPD_RUN_TMR = Time in underspeed or run mode, sec.

Bit Flags:

-  CCM_ECT_MON = OBD-II monitor  flag  for  ECT;  1  ->  ECT  is  considered

monitored.

-  CCM_EO_ENA = Indicates that engine off on-demand mode has been requested.

-  CCM_ER_ENA = Indicates  that  engine  running  on-demand  mode  has  been

requested.

-  CCM_TST_ENA = Continuous CCM test enabled.

-  CFMFLG = Flag indicating that ECT sensor is in/out of range.

-  CRKFLG = Crank flag.

-  ECT_FRST_FL = First pass ECT reading flag; 0 -> First pass  for  ECT  A/D

reading.

-  FFG_ACT = OBD-II system FMEM flag for ACT; 1  ->  ACT  is  currently  not

reliable.

-  FFG_ECT = OBD-II system FMEM flag for ECT; 1  ->  ECT  is  currently  not

reliable.

-  FFG_TOT = OBD-II system FMEM flag for TOT; 1  ->  TOT  is  currently  not

reliable.

-  TOT_FRST_FL = First valid A/D reading for TOT.

-  OBD_PARM_RST = Flag set whenever OBDII is  reset,  KAM  is  in  error  or

on-demand tests are entered or exited.

-  PxxxMALF = OBD-II malfunction flag for fault  xxx;  1  ->  a  malfunction

currently exists for fault xxx, where Pxxx is a valid OBD-II code.

Calibration Constants:

-  ECT_ER_TM = Time with failure present required to set code, sec.

-  ECT_FM_TM = Minimum time in FMEM mode once initiated, sec.

-  ECT_FMEM = FMEM value used when ECT_ENG is out of range, deg F.

-  ECTFMM = FMEM default value for ECT, units are degrees F.

31-28

ENGINE INPUT PROCESSING, ENGINE COOLANT TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  ECT_HI_ER = Maximum acceptable value for ECT, deg F.

-  ECT_HI_KOEO = Maximum value to pass KOEO on-demand test, deg F.

-  ECT_HI_KOER = Maximum value to pass KOER on-demand test, deg F.

-  ECT_LO_ER = Minimum acceptable value for ECT, deg F.

-  ECT_LO_KOEO = Minimum value to pass KOEO on-demand test, deg F.

-  ECT_LO_KOER = Minimum value to pass KOER on-demand test, deg F.

-  ECT_TC1 = Time constant for filtering ECT, sec.

-  ECT_RAMP_TM = Time between updates while ramping ECT to ECT_FMEM, sec.

-  ECT_WUT = Minimum engine coolant temperature for code P0125, deg F.

-  FN703F(ECT_CNTS) = Function to convert from A/D counts to deg F.

-  FN654W(INFAMB_KAM) = Warm up time required to reach ECT_WUT, sec.

-  TKYON2 = Time delay before Key-on updates are permitted, sec.

-  TSTRAT = Calibration device to show transmission type.

OTHER

-  malfunction(ccm,Pxxx) = Logic  process  imported  from  the  MIL  control

module where Pxxx is a valid OBD-II code.

-  store_code(Pxxx) = Stores the fault code specified by Pxxx.

-  ect_codes = Set of {P0117,P0118,P0125}

-  ect_other_codes = Set of {P1116}

-  P0125 = Stabilized feedback temperature not reached in time.

-  P1116 = Engine coolant temperature (ECT) higher or  lower  than  expected

(KOER or KOEO).

-  P0117 = Engine coolant temperature (ECT)  sensor  circuit  below  minimum

voltage / 254 deg F.  indicated.

-  P0118 = Engine coolant temperature (ECT)  sensor  circuit  above  maximum

voltage / -40 deg F.  indicated.

31-29

ENGINE INPUT PROCESSING, ENGINE COOLANT TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INPUT_ECT_OBDII_COM4

BEGIN:  pid_definitions

unconditionally --------| pid_def(j1979_01_05, (((ECT_ENG - 32)*5/9) + 40))

|          (degrees C + 40)

END: pid_definitions

BEGIN: ect_init

Initialize ECT_ENG, ECT and ECT_INST at power-up so a "reasonable" value is

available until the input can be read.

power-up initialization ------------| ECT_ENG := 60

| ECT := 60

| ECT_INST := 60

END: ect_init

31-30

ENGINE INPUT PROCESSING, ENGINE COOLANT TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ect_start_up

unconditionally --------------------| ECT_ENG := FN703F(ECT_CNTS)

|  ;convert to deg F

OBD_PARM_RST is set whenever OBDII is reset or KAM is in error  or  on-demand

tests  are  entered  or  exited.   At  this  time the error timer needs to be

cleared to prevent codes being carried forward.

CCM_TST_ENA = 0 --------|

|

CCM_E0_ENA = 0 ---------|AND -|

|     |

CCM_ER_ENA = 0 ---------|     |OR --| ECT_ER_TMR := 0

|     |  ;if test not asked for

OBD_PARM_RST = 1 -------------|     |  ;clear timer to zero

;reset                             | Do: ect_failure_check

|  ;check for out of range

|  ;perform FMEM action

| Do: ect_warm_up_test

|  ;test for warm-up, calculate TCSTRT

|  ;set component monitored flags

|

| --- ELSE ---

|

CCM_ER_ENA = 1 ---------------------| Do: ect_failure_check

;engine Running on-demand          |  ;check for out of range

;selftest requested                |  ;perform FMEM action

| Do: ect_koer_check

|  ;perform special tests

|  ;for engine running

|

| --- ELSE ---

|

CCM_EO_ENA = 1 ---------------------| Do: ect_failure_check

;engine off on-demand              |  ;check for out of range

;selftest requested                |  ;perform FMEM action

| Do: ect_koeo_check

|  ;perform special tests for engine off

|

| --- ELSE ---

|

|  Normal engine operation

|

| Do: ect_failure_check

|  ;check for out of range

|  ;perform FMEM action

| Do: ect_warm_up_test

|  ;test for warm-up, calculate TCSTRT

|  ;set component monitored flags

END: ect_start_up

31-31

ENGINE INPUT PROCESSING, ENGINE COOLANT TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ect_failure_check

Check for "open/short"  failures.   Perform  FMEM  action  if  a  failure  is

detected.   Set  malfunction  flag  when a malfunction has been present for a

sufficient time.  Otherwise, calculate ECT as a rolling average of ECT_ENG.

ECT_ENG > ECT_HI_ER ----------|

;temperature too high        |

|

ECT_ER_TMR > ECT_ER_TM -------|

;failure present             |AND -| CFMFLG := 1

;long enough                 |     | FFG_ECT := 1

|     |  ;set failure flags

CCM_TST_ENA = 1 --------|     |     | Do: malfunction(ccm,P0117)

|     |     | ECT_FM_TMR := 0

CCM_EO_ENA = 1 ---------|OR --|     | Do: ect_fmem

|     |     |  ;perform FMEM action

CCM_ER_ENA = 1 ---------|           |

| --- ELSE ---

|

ECT_ENG > ECT_HI_ER ----------------| Do: ect_fmem

;temperature too high              |  ;perform FMEM action

;not present long enough           | FFG_ECT := 1

;to set malfunction flag           |  ;set failure flag

|

| --- ELSE ---

ECT_ENG < ECT_LO_ER ----------|     |

;temperature too low         |     |

|     |

ECT_ER_TMR > ECT_ER_TM -------|AND -| CFMFLG := 1

;failure present             |     | FFG_ECT := 1

;long enough                 |     |  ;set failure flag

|     | Do: malfunction(ccm,P0118)

CCM_TST_ENA = 1 --------|     |     | ECT_FM_TMR := 0

|     |     | Do: ect_fmem

CCM_EO_ENA = 1 ---------|OR --|     |  ;perform FMEM action

|           |

CCM_ER_ENA = 1 ---------|           |

| --- ELSE ---

|

(continued on next page)

31-32

ENGINE INPUT PROCESSING, ENGINE COOLANT TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

ECT_ENG < ECT_LO_ER ----------------| Do: ect_fmem

;temperature too low               |  ;perform FMEM action

;not present long enough           | FFG_ECT := 1

;to set malfunction flag           |  ;set failure flag

|

| --- ELSE ---

P0118MALF = 1 ----------|           |

;currently in FMEM     |OR --|     |

|     |     |

P0117MALF = 1 ----------|     |AND -| Do: ect_fmem

|     |  ;remain in FMEM mode for at least

ECT_FM_TMR < ECT_FM_TM -------|     |  ;the minimum time even if the

;minimum time in FMEM              |  ;failure is no longer present

;once it is entered                | FFG_ECT := 1

| CFMFLG := 1

|  ;set failure mode flag

|

| --- ELSE ---

|

ECT_FRST_FL = 0 --------------------| ECT := ECT_ENG

;first ECT value                   |  ;initial value for ECT

|  ;rolling average filter

| ECT_INST := ECT_ENG

|  ;unfiltered value of ECT

| ECT_ER_TMR := 0

|  ;clear failure present timer

| ECT_FRST_FL := 1

| FFG_ECT := 0

|  ;no failure

|

| --- ELSE ---

|

| ECT := ROLAV_TC(ECT_ENG,ECT_TC1)

|  ;no failure detected, return

|  ;value for normal use

| ECT_INST := ECT_ENG

|  ;unfiltered value of ECT

| ECT_ER_TMR := 0

|  ;clear failure present timer

| Do: clear_malf(P0118)

| Do: clear_malf(P0117)

| CFMFLG := 0

| FFG_ECT := 0

|  ;no failure

|  ;clear failure flags

END: ect_failure_check

31-33

ENGINE INPUT PROCESSING, ENGINE COOLANT TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ect_fmem

In crank mode, use TOT in place of ECT.  Otherwise, until a failure has  been

present  long enough to set a failure flag, use last good value of ECT.  Once

the failure flag is set, ramp ECT to the value ECT_FMEM.

TSTRAT <> 0 ------------------|

;TOT available               |

|

CRKFLG = 1 -------------------|

;crank mode                  |

|

ECT_FRST_FL = 0 --------------|

|AND -| ECT := TOT_ENG

FFG_TOT = 0 ------------------|     |  ;substitute a good TOT value for

;TOT in range                |     |  ;the bad ECT while in crank mode

|     | ECT_FRST_FL := 1

TOT_FRST_FL = 1 --------------|     |  ;initial value

;TOT has been read                 | ECT_INST := TOT_ENG

|  ;unfiltered value of ECT

|

| --- ELSE ---

TSTRAT <> 0 ------------------|     |

;TOT available              |     |

|     |

CRKFLG = 1 -------------------|     |

;crank mode                  |AND -| ECT := ROLAV_TC(TOT_ENG,ECT_TC1)

|     |  ;calculate ECT as the rolling

FFG_TOT = 0 ------------------|     |  ;average of ECT_old and TOT

;TOT in range                |     |  ;while in crank mode

|     | ECT_INST := TOT_ENG

TOT_FRST_FL = 1 --------------|     |

;TOT has been read                 |

| --- ELSE ---

CRKFLG = 1 -------------------|     |

;crank mode                  |     |

|AND -| ECT := ACT_ENG

ECT_FRST_FL = 0 --------------|     |  ;substitute a good ACT value for

|     |  ;the bad ECT while in crank mode

FFG_ACT = 0 ------------------|     |  ;as TOT not available

;ACT in range                      |

| ECT_FRST_FL := 1

|  ;initial value

| ECT_INST := ACT_ENG

|

| --- ELSE ---

CRKFLG = 1 -------------------|     |

;crank mode                  |AND -| ECT := ROLAV_TC(ACT_ENG,ECT_TC1)

|     |  ;calculate ECT as the rolling

FFG_ACT = 0 ------------------|     |  ;average of ECT_old and ACT

;ACT in range                      |  ;while in crank mode as

|  ;TOT not available

| ECT_INST = ACT_ENG

|

| --- ELSE ---

(continued on next page)

31-34

ENGINE INPUT PROCESSING, ENGINE COOLANT TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|

P0117MALF = 1 ----------------|     |

|OR --| Do: ect_ramp

P0118MALF = 1 ----------------|     |  ;ramp ECT to ECTFMM

|

| --- ELSE ---

|

| Do not update ECT

|  ;use last good value

END: ect_fmem

BEGIN: ect_ramp

Decrement (or increment) ECT from the current value to ECT_FMEM at  the  rate

of 2 deg (1 bit) every ECT_RAMP_TM seconds.  Since ECT_FMEM_TMR is allowed to

run during normal operations, on the first  call  to  this  routine  it  will

probabably exceed ECT_RAMP_TM and ECT will increment or decrement 2 degrees.

ECT > ECT_FMEM ---------------|

;ECT high                    |AND -| ECT := ECT - 2 deg F

|     |  ;ramp down towards ECT_FMEM

ECT_FMEM_TMR > ECT_RAMP_TM ---|     | ECT_FMEM_TMR := 0

;enough time to allow              |  ;clear timer, set up to wait

;a decrement                       |  ;for next interval

|

| --- ELSE ---

ECT < ECT_FMEM ---------------|     |

;ECT low                     |AND -| ECT := ECT + 2 deg F

|     |  ;ramp up towards ECT_FMEM

ECT_FMEM_TMR > ECT_RAMP_TM ---|     | ECT_FMEM_TMR := 0

;enough time to allow              |  ;clear timer, set up to wait

;a decrement                       |  ;for next interval

|

| --- ELSE ---

|

| Do not update ECT

|  ;ECT is already ECT_FMEM or

|  ;insufficent time since last

|  ;update

END: ect_ramp

31-35

ENGINE INPUT PROCESSING, ENGINE COOLANT TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ect_koer_check

Check against Key-on engine running limits.  If an  error  is  detected,  set

P1116MALF flag.

FFG_ECT = 1 ------------------------|  ;continuous test detected

;ECT unreliable                    |  ;failure, no need to test

|  ;tighter KOER limits

|

| --- ELSE ---

ECT_ENG > ECT_HI_KOER --------|     |

|OR --| Do: store_code(P1116)

ECT_ENG < ECT_LO_KOER --------|     |  ;ECT out of selftest range

|

| --- ELSE ---

|

| Do: clear_malf(P1116)

|  ;no failure detected

END: ect_koer_check

BEGIN: ect_koeo_check

Check against Key-on engine  off  limits.   If  an  error  is  detected,  set

P1116MALF flag.

FFG_ECT = 1 ------------------------|  ;continuous test detected

;ECT unreliable                    |  ;failure, no need to test

|  ;tighter KOER limits

|

| --- ELSE ---

ECT_ENG > ECT_HI_KOEO --------|     |

|OR --| Do: store_code(P1116)

ECT_ENG < ECT_LO_KOEO --------|     |  ;ECT out of selftest range

|

| --- ELSE ---

|

| Do; clear_malf(P1116)

|  ;no failure detected

END: ect_koeo_check

31-36

ENGINE INPUT PROCESSING, ENGINE COOLANT TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ect_warm_up_test

OBD-II regulations require that the ECT sensor be monitored for  achieving  a

stabilized  minimum  temperature  level.  This is the level needed to achieve

closed-loop operation within a manufacturer-specified  time  interval,  after

starting  the  engine.  To pass this test, ECT must excede ECT_WUT within the

time  limit  defined  by  FN654W(INFAMB_KAM).   After   the   time   interval

FN654W(INFAMB_KAM),  the  ECT  sensor  is considered to be monitored for this

trip.

TCSTRT (ECT at  start)  based  on  the  first  eight  ECT  readings  is  also

calculated here.

31-37

ENGINE INPUT PROCESSING, ENGINE COOLANT TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PUTMR > TKYON2 ---------------|

;avoid noise during          |

;engine cranking             |AND -| TCSTRT_ACCUM := TCSTRT_ACCUM + (ECT/8)

|     |  ;compute arithmetic average of

ECTCNT < 8 -------------------|     |  ;the first 8 ECT readings

;accumulate first eight            |

| TCSTRT := ECT

|  ;use current value until TCSTRT

|  ;has been calculated

| ECTCNT := ECTCNT + 1

|  ;increment ECTCNT, also used

|  ;for ACT and TOT

|

| --- ELSE ---

|

ECTCNT < 8 -------------------------| TCSTRT := ECT

|  ;use current value until TCSTRT

|  ;has been calculated

|

| --- ELSE ---

ECT > ECT_WUT ----------------|     |

;stabilized ECT reached      |     |

|     |

CCM_ECT_MON = 0 --------------|AND -| Do: clear_malf(P0125)

;not yet monitored           |     |  ;ECT stabilized

|     | CCM_ECT_MON := 1

CCM_TST_ENA = 1 --------------|     |  ;ECT monitored for OBD-II trip

;CCM test enabled                  | TCSTRT := TCSTRT_ACCUM

|  ;load TCSTRT with arithmetic

|  ;average of first 8 values

|  ;after PUTMR > TKYON2

|

| --- ELSE ---

USPD_RUN_TMR >                      |

FN654W(INFAMB_KAM) -----|     |

;enough time for ECT to      |     |

;stabilize                   |     |

|     |

|AND -| Do: malfunction(ccm,P0125)

CCM_ECT_MON = 0 --------------|     |  ;set failure flag

;not yet monitored           |     |  ;did not warm up in time

|     |  ;ECT or thermostat failure

CCM_TST_ENA = 1 --------------|     |

;CCM test enabled                  | CCM_ECT_MON := 1

|  ;ECT monitored for OBD-II trip

| TCSTRT := TCSTRT_ACCUM

|  ;load TCSTRT with arithmetic

|  ;average of first 8 values

|  ;after PUTMR > TKYON2

|

| --- ELSE ---

|

| TCSTRT := TCSTRT_ACCUM

|  ;load TCSTRT with arithmetic

|  ;average of first 8 values

|  ;after PUTMR > TKYON2

END: ect_warm_up_test

31-38

INPUT AND FAILURE MODE AND SELF TEST, OCTANE ADJUST INPUT TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

31.7  OCTANE ADJUST INPUT TEST (CDAL0)

OVERVIEW

The OCTane ADJust pin is used in service to retard the spark for octane.  The

normal  state  for  the pin is to be shorted to ground, in which case service

spark retard is not required.

Providing octane adjust hardware is present, the purpose of the Octane Adjust

Input  Test  is to check the state of the octane adjust pin and output a self

test code if the pin is open circuit, thus indicating that spark  retard  has

been requested or that there is an open circuit fault.

DEFINITIONS

INPUTS

Bit Flags:

-  CCM_EO_ENA = KOEO on-demand self test enabled.

-  OCTADJ = Field octane adjust input; 1 -> Retard for octane adjustment  is

desired, 0 -> No retard is desired.

-  PxxxMALF = OBD-II malfunction flag for fault  xxx;  1  ->  a  malfunction

currently exists for fault xxx, where Pxxx is a valid OBD-II code.

Calibration Constants:

-  OCTADJHP = Octane adjust pin hardware present switch; 0 -> no OCTADJ  pin

present, 1 -> an OCTADJ pin is present.

OUTPUTS

Bit Flags:

-  PxxxMALF = See above

OTHER

-  octadj_codes = Set of {P1390}

-  store_code(Pxxx) = Stores the fault code specified by Pxxx.

-  erase_code(Pxxx) = Erases the fault code specified by Pxxx.

-  P1390 = Octane adjust service pin in use/circuit open.

31-39

INPUT AND FAILURE MODE AND SELF TEST, OCTANE ADJUST INPUT TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INPUT_OCTADJ_OBDII_COM1

OCTADJHP = 0 ---------------------------| (no action)

(Octane Adjust hardware                |

not present)                          |

| --- ELSE ---

|

CCM_EO_ENA = 1 -------------------|     |

(KOEO on-demand test)            |AND -| store_code(P1390)

|     |

OCTADJ = 1 -----------------------|     |

(Octane Adjust pin is                  |

open circuit)                         |

| --- ELSE ---

|

CCM_EO_ENA = 1 -------------------------| erase_code(P1390)

|

31-40

INPUT AND FAILURE MODE AND SELF TEST, PSPS INPUT AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

31.8  POWER STEERING PRESSURE SWITCH (PSPS) INPUT AND SELF TEST (CDAJ0)

OVERVIEW

PSPS is a switch which opens/closes based on the level of the power  steering

pressure.   If  the  pressure  is greater tha or equal to that exerted by the

switchn the circuit is opened and the flag POWSFG is set to 1; if the  switch

is  closed  (PSPS  is  greater  than pressure) then a signal is output of 0.4

Volts direct current or less and POWSFG is set to zero.

In normal continuous mode, it is assumed that the power steering  would  have

been activated at least once by the time V_PSPS_CNT transitions from zero MPH

to a sustained cruise speed (V_PSPSPD_CTR) are made.  If not, a failure  code

is set.

NOTE:  This code could be set if a vehicle is towed with the engine running.

For the KOEO on-demand test, POWSFG must be low to pass.  For KOER, a failure

is  assumed  and  when  both on and off states have been seen, the KAM bit is

cleared.

DEFINITIONS

INPUTS

Registers:

-  ER_STATUS = State pointer that incicated current state of engine  running

on-demand test.

-  PSPS = Digital input.

-  V_PSPSPD_CTR = Number of vehicle speed transitions from a speed  of  zero

to a speed of V_ZTOSPD_MPH that lasted at least V_ZTOSPD_TM seconds since

last PSPS on and off detected.

Bit Flags:

-  CCM_TST_ENA = Continuous self test enabled.

-  CCM_EO_ENA = KOEO on-demand self test enabled.

-  PxxxMALF = OBD-II malfunction flag  for  fault  xxx;  1->  a  malfunction

currently exists for fault xxx, where Pxxx is a valid OBD-II code.

-  POWSFG = Current state of PSPS input; 0 -> no PSPS load.

-  OPER_RSP_CHK = PSPS KOER on-demand self test enabled, enabled at  end  of

KOER test.

-  V_POWS_OFF = Off state has been seen since last reset.

-  V_POWS_ON = On state has been seen since last reset.

31-41

INPUT AND FAILURE MODE AND SELF TEST, PSPS INPUT AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  ER_INIT = Value for state of ER_STATUS when  engine  running  test  first

inits.

-  PSPSHP = Power steering pressure switch hardware present.

-  V_PSPS_CNT = Number of speed transitions  without  both  PSPS  states  to

assume a failure.

OUTPUTS

Registers:

-  V_PSPSPD_CTR = See above.

Bit Flags:

-  PxxxMALF = See above.

-  POWSFG = See above.

-  V_POWS_ON = See above.

-  V_POWS_OFF = See above.

OTHER

-  store_code(Pxxx) = Stores the fault code specified by Pxxx.

-  psps_codes = SET OF {P1650,P1651}

-  P1650 = PSPS indicates power steering applied during  KOEO  or  no  state

change in KOER.

-  P1651 = PSPS did not change states in Continuous test.

31-42

INPUT AND FAILURE MODE AND SELF TEST, PSPS INPUT AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INPUT_PSPS_COM1

PSPSHP = 0 ------------------------| do nothing

(PSPS hardware not present)       |

| --- ELSE ---

PSPS_LVL = 1 ----------------|     |

|AND -| POWSFG = 1

PSPSHP = 1 ------------------|     |  (power steering load on)

(PSPS input hardware              | V_POWS_ON  = 1

present)                         |  (ON state "seen")

|

| --- ELSE ---

|

| POWSFG = 0

|  (Power steering not applied or

|   hardware not present)

| V_POWS_OFF = 1

|  (OFF state "seen")

ER_STATUS = ER_INIT ---------------| V_POWS_ON = 0

(engine running on-demand         | V_POWS_OFF = 0

test starting now)               |  (restart flags to watch for response

|   during on-demand test.)

|

| V_PSPSPD_CTR = 0

|  (clear counter)

|

| --- ELSE ---

|

| no action.

PSTMR - POWER STEERING TIMER (SEC.)

This timer increments while the power steering is applied.

POWSFG = 1 ------------------|     |

|AND -| Count up PSTMR

PSPSHP = 1 ------------------|     |

|

| --- ELSE ---

|

| PSTMR = 0

31-43

INPUT AND FAILURE MODE AND SELF TEST, PSPS INPUT AND SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PSPSHP = 0 ------------------------| NO ACTION

(PSPS hardware not present)       |

| --- ELSE ---

CCM_TST_ENA = 1 -------------|     |

(Continuous Self Test       |     |

enabled)                   |     |

|     |

V_POWS_ON  = 1 --------------|AND -| clear_malf(P1651)

(ON state has been seen)    |     |  (both states seen, PSPS works)

|     | V_POWS_ON  = 0

V_POWS_OFF = 1 --------------|     | V_POWS_OFF = 0

(OFF state has been seen)         |  (reset flags to allow re-test)

| V_PSPSPD_CTR = 0

|  (restart counter for re-test)

|

| --- ELSE ---

CCM_TST_ENA = 1 -------------|     |

|AND -| store_code(P1651)

V_PSPSPD_CTR > V_PSPS_CNT ---|     |  (failure assumed)

(Enough speed transitions         |

without both PSPS states to      |

assume failure)                  |

| --- ELSE ---

OPER_RSP_CHK = 1 ------------|     |

(KOER PSPS Self Test ena.)  |     |

|AND -| store_code(P1650)

V_POWS_ON  = 0 --------|     |     |  (malfunction)

(ON state has not     |     |     |

been seen)           |OR --|     |

|           |

V_POWS_OFF = 0 --------|           |

(OFF state has not been seen)     |

| --- ELSE ---

CCM_EO_ENA = 1 --------------|     |

(KOEO on-demand test)       |AND -| store_code(P1650)

|     |  (malfunction)

POWSFG = 1 ------------------|     |

(power steering load on)          |

|

| --- ELSE ---

|

| (no action)

31-44

INPUT CONVERSIONS AND FILTERS, POWER STEERING PRESSURE SWITCH INPUT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

31.9  POWER STEERING PRESSURE SWITCH INPUT (CDAA0)

OVERVIEW

PSPS is a switch which opens/closes based on the level of the power  steering

pressure.   If  the pressure is greater than, or equal to that exerted by the

switch the circuit is opened; if the switch is closed (PSPS is  greater  than

pressure) then a signal is output of 0.4 Volts direct current or less.  NOTE:

If the PSPSHP is calibrated to indicate no PSPS, the POWSFG is held at zero.

DEFINITIONS

INPUTS

Registers:

-  PSPS = Power steering pressure switch input.

Calibration Constants:

-  PSPSHP = Power steering pressure switch "Hardware Present" indicator (0 =

NO, 1 = YES).

OUTPUTS

Bit Flags:

-  POWSFG = Power steering flag.

31-45

INPUT CONVERSIONS AND FILTERS, POWER STEERING PRESSURE SWITCH INPUT - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INPUT_PSPS_SWITCH_COM1

PSPS LOW --------------------|

|OR --| POWSFG = 0

PSPSHP = 0 ------------------|     |  (no power steering load)

(PSPS hardware absent)            |

| --- ELSE ---

|

| POWSFG = 1

|  (power steering load)

31-46

ENGINE INPUT PROCESSING, START STOP COUNTER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

31.10  START STOP COUNTER FOR PSPS (CDAN0)

OVERVIEW

This module counts V_PSPSPD_CTR transitions from stopped to a sustained speed

V_ZTOSPD_MPH, in V_ZTOSPD_TM seconds.  These counts are used in the PSPS test

to determine if sufficient vehicle movement has occurred to assume  that  the

power steering should have been used.

DEFINITIONS

Registers:

-  V_PSPSPD_CTR = Number of vehicle speed transitions from a speed  of  zero

to a speed of V_ZTOSPD_MPH that lasted at least V_ZTOSPD_TM seconds since

last PSPS on and off detected.

-  VSBAR = Rolling average of vehicle speed, mph.

-  V_ZTOSPD_TMR = Non cumulative time VSBAR greater than V_ZTOSPD_MPH, sec.

Bit Flags:

-  V_SPDTOZ_FLG = Flag to indicate that vehicle has been stopped.

Calibration Constants:

-  V_ZTOSPD_MPH = Speed required to increment V_PSPSPD_CTR, mph.

-  V_ZTOSPD_TM = Time at speed required to increment V_PSPSPD_CTR, sec

31-47

ENGINE INPUT PROCESSING, START STOP COUNTER - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INPUT_START_STOP_OBDII_COM1

BEGIN:  input_start_stop

This is called once per background loop from the CNVRT software module.

VSBAR = 0 --------------------------| V_SPDTOZ_FLG := 1

;vehicle stopped                   |  ;set up to count next

|  ;zero to speed transition

|

| V_ZTOSPD_TMR := 0

|  ;not above required speed

|  ;hold timer to zero

|

| --- ELSE ---

|

VSBAR <= V_ZTOSPD_MPH --------------| V_ZTOSPD_TMR := 0

|  ;not above required speed

|  ;hold timer to zero

|

| --- ELSE ---

V_ZTOSPD_TMR > V_ZTOSPD_TM ---|     |

;sufficient time at speed    |AND -| V_PSPSPD_CTR := V_PSPSPD_CTR + 1

|     |  ;zero to speed transition made

V_SPDTOZ_FLG = 1 -------------|     |  ;so increment counter

;first time conditions met         |  ;counter zeroed in PSPS

;since last zero speed             |

| V_SPDTOZ_FLG := 0

|  ;wait for next zero speed...

|

| --- ELSE ---

|

| No action

END: input_start_stop

31-48

ENGINE INPUT PROCESSING, TRANSMISSION OIL TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

31.11  TOT SENSOR INPUT AND SELFTEST (CDAM0)

OVERVIEW

Initial checks are made to determine if an on-demand mode is  activated.   If

no  on-demand mode is active the software falls through and runs the standard

sequence.  The A/D is read and the raw counts (TOT_CNTS) are  converted  into

deg  F.   (TOT_ENG).   TOT_ENG  is  the  value used when performing any input

testing and is reported to the scan tool.  Next, the TOT_ENG value is  tested

for "out of range" or other failure conditions.  If the TOT sensor reading is

in range, TOT is set equal to TOT_ENG.  Otherwise, the last "good"  value  of

TOT  (TOT_LST)  will  be  used  for  TOT  until  the failure is present for a

sufficient amount of time.  At that time, the  appropriate  malfunction  flag

(PxxxMALF) is set and the FMEM strategy will determine TOT as follows:

- ECT < warm engine      TOT = ECT

- ECT >= warm engine     TOT is ramped to TOT_MAX.

If TOT was < warm_engine before invalid

reading,the ramp starts at warm_engine;

otherwise the ramp starts at TOT value

before invalid reading.

Ramp time is calibratible with maximum

time of 255 seconds; resolution of

1 second.

This  allows  cold  engagement  strategy  to  function   and   protects   the

transmission when operating at higher temperatures.

If an on-demand mode is active.  The KOER or KOEO range  tests  will  be  run

along with the standard Continuous tests and FMEM action.

DEFINITIONS

Registers:

-  ECT = Engine Coolant Temperature, deg F.

-  FFG_TOT = OBD-II system  FMEM  flag  for  TOT;  1  ->  TOT  is  currently

unreliable.

-  INIT_TOT = TOT at start-up; arithmetic average of  the  first  eight  TOT

readings.

-  INIT_TOT_CNT = Number of times that the TOT sensor has been read.

-  PUTMR = Time since power up, sec.

-  TOT = Transmission oil temperature with FMEM corrections as required, deg

F.

-  TOT_CNTS = Raw A/D counts from sensor.

31-49

ENGINE INPUT PROCESSING, TRANSMISSION OIL TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TOT_ENG = Transmission oil temperature before any FMEM action.  Used  for

FMEM testing and output to scan tool, deg F.

-  TOT_ER_TMR = Timer that runs when a fault is present.

-  TOT_FM_TMR = Timer that is reset when the  failure  conditions  for  FMEM

mode are met, sec.

-  TOT_LST = Last pass value of TOT, deg F.

-  TOT_PREV = Last valid TOT before out of range value, deg F.

-  TOT_TMR = Timer that is reset until FMEM logic calls for TOT to  ramp  to

TOT_MAX, sec.

Bit Flags:

-  CCM_EO_ENA = Indicates that engine off on-demand mode has been requested.

-  CCM_ER_ENA = Indicates  that  engine  running  on-demand  mode  has  been

requested.

-  CCM_TST_ENA = Continuous CCM tests enabled.

-  OBD_PARM_RST = Flag set whenever OBDII is  reset,  KAM  is  in  error  or

on-demand tests are entered or exited.

-  PxxxMALF = OBD-II malfunction flag  for  fault  xxx;  1->  a  malfunction

currently exists for fault xxx, where Pxxx is a valid OBD-II code.

-  TOT_FM_FLG = TOT failure mode flag; 1 -> TOT FMEM in progress.

-  TOT_FRST_FL = First pass TOT reading flag; 0 -> First pass  for  TOT  A/D

reading.

Calibration Constants:

-  ECT_WARM_ENG = Value of ECT above which the powertrain will be considered

warm, deg F.

-  FN703F(TOT_CNTS) = Function to convert from A/D counts to deg F.

-  TKYON2 = Time delay before Key on updates are permitted, sec.

-  TOT_ER_TM = Time with failure present required to set code, sec.

-  TOT_FM_TM = Minimum time in FMEM mode once initiated, sec.

-  TOT_HI_ER = Maximum acceptable value for TOT, deg F.

-  TOT_HI_KOEO = Maximum value to pass KOEO on-demand test, deg F.

-  TOT_HI_KOER = Maximum value to pass KOER on-demand test, deg F.

-  TOT_LO_ER = Minimum acceptable value for TOT, deg F.

-  TOT_LO_KOEO = Minimum value to pass KOEO on-demand test, deg F.

31-50

ENGINE INPUT PROCESSING, TRANSMISSION OIL TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TOT_LO_KOER = Minimum value to pass KOER on-demand test, deg F.

-  TOT_MAX = TOT value during sensor FMEM once powertrain has warmed up.

-  TOT_RMP_TIM = Length of time of FMEM TOT ramp, sec.

-  TOT_TC = Time constant for TOT, sec.

-  TSTRAT  =  Transmission  strategy  switch;  The  TSTRAT  software  switch

controls which transmission control strategy is executed:

0 -> No transmission control (Manual trans., AOD, ATX, C6, C3, etc.)

1 -> SIL (Shift indicator light)

2 -> A4LD with 3-4 shift control and converter clutch control

3 -> AXOD

4 -> C64E (E4OD)

5 -> A4LDE

6 -> FAX-4

7 -> AOD-E (AOD-I)

8 -> 4EAT

9 -> CD4E

OTHER

-  malfunction(ccm,Pxxx) = Logic  process  imported  from  the  MIL  control

module where Pxxx is a valid OBD-II code.

-  store_code(Pxxx) = Stores the fault code specified by Pxxx.

-  P1711 = Transmission oil temperature (TOT) higher or Lower than  expected

(KOER or KOEO)

-  P0712 = Transmission oil temperature (TOT) sensor circuit  below  minimum

voltage / 254 F.  indicated.

-  P0713 = Transmission oil temperature (TOT) sensor circuit  above  maximum

voltage / -40 F.  indicated.

-  TOT_CODES = Set of {P0712,P0713}

-  TOT_OTHER_CODES = Set of {P1711}

31-51

ENGINE INPUT PROCESSING, TRANSMISSION OIL TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INPUT_TOT_OBDII_COM2

BEGIN:  tot_init

Initialize TOT_ENG and TOT at power-up so a "reasonable" value  is  available

until the input can be read.

power-up initialization ------------| TOT_ENG := 60

| TOT := 60

END: tot_init

BEGIN: tot_start_up

Once per background the following logic is executed:

OBD_PARM_RST is set whenever OBDII is reset or KAM is in error  or  on-demand

tests  are  entered  or  exited.   At  this  time the error timer needs to be

cleared to prevent codes being carried forward.

unconditionally --------------------| TOT_LST := TOT

|  ;save last value of TOT

31-52

ENGINE INPUT PROCESSING, TRANSMISSION OIL TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TSTRAT <= 3 ------------------------| Exit module

;manual/non-electric transmission  |

| --- ELSE ---

CCM_TST_ENA = 0 --------|           |

|           |

CCM_E0_ENA = 0 ---------|AND -|     |

|     |     |

CCM_ER_ENA = 0 ---------|     |OR --| TOT_ER_TMR := 0

|     |  ;if test not asked for

OBD_PARM_RST = 1 -------------|     |  ;clear timer to zero

|

| TOT_ENG := FN703F(TOT_CNTS)

|  ;convert A/D input to deg F

| DO: tot_failure_check

|  ;check for out of range

|  ;perform FMEM action

| Do: initial_tot_calculation

|  ;compute arithmetic average of the

|  ;first 8 consecutive TOT readings

|

| --- ELSE ---

|

CCM_ER_ENA = 1 ---------------------| TOT_ENG := FN703F(TOT_CNTS)

;engine running on-demand          |  ;convert A/D input to deg F

;selftest requested                | Do: tot_failure_check

|  ;check for out of range

|  ;perform FMEM action

| Do: tot_koer_check

|  ;perform special tests for

|  ;engine running

|

| --- ELSE ---

|

CCM_EO_ENA = 1 ---------------------| TOT_ENG := FN703F(TOT_CNTS)

;engine off on-demand              |  ;convert A/D input to deg F

;self test requested               | Do: tot_failure_check

|  ;check for out of range

|  ;perform  FMEM action

| Do: tot_koeo_check

|  ;perform special tests for

|  ;engine off

|

| --- ELSE ---

|

| Normal engine operation

|

| TOT_ENG := FN703F(TOT_CNTS)

|  ;convert A/D input to deg F

| DO: tot_failure_check

|  ;check for out of range

|  ;perform FMEM action

| Do: initial_tot_calculation

|  ;compute arithmetic average of the

|  ;first 8 consecutive TOT readings

|

END: tot_start_up

31-53

ENGINE INPUT PROCESSING, TRANSMISSION OIL TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: tot_failure_check

Check for "open/short"  failures.   Perform  FMEM  action  if  a  failure  is

detected.   Set  malfunction  flag  when a malfunction has been present for a

sufficient time.  Otherwise, calculate TOT as a rolling average of TOT_ENG.

TOT_ENG > TOT_HI_ER ----------|

;value too high              |

|

TOT_ER_TMR > TOT_ER_TM -------|

;failure present             |AND -| FFG_TOT := 1

;long enough                 |     | TOT_FM_FLG := 1

|     |  ;set failure flags

CCM_TST_ENA = 1 --------|     |     | Do: store_code(P0712)

|     |     | TOT_FM_TMR := 0

CCM_ER_ENA = 1 ---------|OR --|     | Do: fmem_tot_calc

|           |  ;perform FMEM action

CCM_EO_ENA = 1 ---------|           |

| --- ELSE ---

|

TOT_ENG > TOT_HI_ER ----------------| TOT := TOT_LST

;value too high                    | TOT_FM_TMR := 0

;not present long enough to        | FFG_TOT := 1

;set malfunction flag              |  ;set TOT to TOT_LST

|  ;until error present long

|  ;enough to store a code

|

| --- ELSE ---

TOT_ENG < TOT_LO_ER ----------|     |

;value too low               |     |

|     |

TOT_ER_TMR > TOT_ER_TM -------|     |

;failure present             |AND -| FFG_TOT := 1

;long enough                 |     | TOT_FM_FLG := 1

|     |  ;set failure flag

CCM_TST_ENA = 1 --------|     |     | Do: store_code(P0713)

|     |     | TOT_FM_TMR := 0

CCM_ER_ENA = 1 ---------|OR --|     | Do: fmem_tot_calc

|           |  ;perform FMEM action

CCM_EO_ENA = 1 ---------|           |

|

| --- ELSE ---

|

TOT_ENG < TOT_LO_ER ----------------| TOT := TOT_LST

;value too low                     | TOT_FM_TMR := 0

;not present long enough to        | FFG_TOT := 1

;set malfunction flag              |  ;set TOT to TOT_LST

|  ;until error present long

|  ;enough to store a code

|

| --- ELSE ---

(continued on next page)

31-54

ENGINE INPUT PROCESSING, TRANSMISSION OIL TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|

TOT_FM_FLG = 1 ---------------|     |

;TOT FMEM in progress        |AND -| Do: fmem_tot_calc

|     |  ;remain in FMEM mode for at least

TOT_FM_TMR < TOT_FM_TM -------|     |  ;the minimum time even if the

;minimum time in FMEM              |  ;failure is no longer present

;once it is entered                | FFG_TOT := 0

|

| --- ELSE ---

|

TOT_FRST_FL = 0 --------------------| TOT := TOT_ENG

;valid TOT not seen before         |  ;first valid TOT value

| TOT_ER_TMR := 0

|  ;clear failure present timer

| TOT_FRST_FL := 1

| FFG_TOT := 0

|

| --- ELSE ---

|

| TOT := ROLAV_TC(TOT_ENG,TOT_TC)

|  ;no failure detected, return

|  ;value for normal use

| TOT_ER_TMR := 0

|  ;clear failure present timer

| Do: clear_malf(P0713)

| Do: clear_malf(P0712)

| FFG_TOT := 0

| TOT_FM_FLG := 0

|  ;clear failure flags

END: tot_failure_check

BEGIN: initial_tot_calculation

Compute an arithmetic average of the first eight consecutive TOT readings.

PUTMR > TKYON2 ---------------|

|AND -| INIT_TOT_CNT := INIT_TOT_CNT + 1

INIT_TOT_CNT < 8 -------------|     | INIT_TOT := INIT_TOT + (TOT/8)

END: initial_tot_calculation

31-55

ENGINE INPUT PROCESSING, TRANSMISSION OIL TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: tot_koer_check

Check against Key-On engine running limits.  If an  error  is  detected,  set

P1711MALF flag.

FFG_TOT = 1 ------------------------| No action

;immediate TOT failure             |  ;continuous test detected

|  ;failure, no need to test

|  ;tighter KOER limits

|

| --- ELSE ---

TOT_ENG > TOT_HI_KOER --------|     |

|OR --| Do: store_code(P1711)

TOT_ENG < TOT_LO_KOER --------|     |  ;TOT out of selftest range

|

| --- ELSE ---

|

|  Do: clear_malf(P1711)

|   ;no failure detected

END: tot_koer_check

BEGIN: tot_koeo_check

Check against Key-On engine  off  limits.   If  an  error  is  detected,  set

P1711MALF flag.

FFG_TOT = 1 ------------------------| No action

;immediate TOT failure             |  ;continuous test detected

|  ;failure, no need to test

|  ;tighter KOER limits

|

| --- ELSE ---

TOT_ENG > TOT_HI_KOEO --------|     |

|OR --| Do: store_code(P1711)

TOT_ENG < TOT_LO_KOEO --------|     |  ;TOT out of selftest range

|

| --- ELSE ---

|

|  Do: clear_malf(P1711)

|   ;no failure detected

END: tot_koeo_check

31-56

ENGINE INPUT PROCESSING, TRANSMISSION OIL TEMPERATURE SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: fmem_tot_calc

This logic determines TOT during sensor FMEM:

TOT_FM_FLG = 0 ---------------------| TOT := TOT_LST

;error not present long enough    | TOT_PREV := TOT_LST

;to store a code                  | TOT_TMR := 0

|  ;set TOT equal to last valid TOT

|  ;reading, save TOT in TOT_PREV

|

| --- ELSE ---

|

TOT >= TOT_MAX ---------------------| TOT := TOT_MAX

;after TOT ramps to desired       |

;maximum, freeze TOT at TOT_MAX   |

| --- ELSE ---

|

ECT < ECT_WARM_ENG -----------------| TOT := ECT

;powertrain NOT warmed up         | TOT_TMR := 0

|

| --- ELSE ---

|

TOT_PREV > ECT_WARM_ENG ------------| TOT := TOT_PREV + [(TOT_TMR) *

;powertrain ALREADY warm, ramp    |        (TOT_MAX - TOT_PREV)/

;TOT from the value before        |        (TOT_RMP_TIM)]

;error occurred to TOT_MAX        |

| --- ELSE ---

|

;Powertrain was not warm prior    | TOT := ECT_WARM_ENG + [(TOT_TMR) *

;to invalid reading; ramp from    |        (TOT_MAX - ECT_WARM_ENG)/

;warm engine to TOT_MAX           |        (TOT_RMP_TIM)]

END: fmem_tot_calc

31-57

ENGINE INPUT PROCESSING, IN-RANGE TP-LOAD TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

31.12  IN-RANGE TP-LOAD TEST (CDAM0)

OVERVIEW

This module detects inconsistency  between  the  throttle  position  and  air

charge.  Possible causes for such an inconsistency are:

1.  TP sensor faulty.

2.  Air meter faulty.

3.  Air leak between air meter and throttle body.

Note, this test will be aborted if a series throttle is present and  traction

assist is active.

DEFINITIONS

Registers:

-  CCM_ER_ENA = Engine running on-demand for CCM enabled.

-  ECT = Engine coolant temperature, deg F.

-  LOAD = Normalized cylinder air charge as calculated from  mass  air  flow

meter reading, unitless.

-  N = Engine speed (RPM).

-  TP_REL = Throttle position, counts.

-  V_TP_ERR_TMR = Length of time that error has been detected, sec.

Bit Flags

-  CCM_TST_ENA = OBD-II Comprehensive component test enable flag; 1 ->  test

enabled.

-  MFMFLG = Flag indicating a MAF sensor failure; 1 -> failure, unitless.

-  OBD_PARM_RST = Flag set whenever OBDII is  reset,  KAM  is  in  error  or

on-demand tests are entered or exited.

-  TFMFLG = Flag indicating a TP sensor failure; 1 -> failure, unitless.

-  TRAC_ACTIVE = Flag indicating vehicle is in a traction assist event; 1 ->

T/A active.

Calibration Constants:

-  TEMPFB = Engine coolant temperature below which error cannot be detected,

deg F.

-  TRAC_AIRSW = Traction control switch; 1 -> do airflow  (series  throttle)

control.

31-58

ENGINE INPUT PROCESSING, IN-RANGE TP-LOAD TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  V_LOAD_HI  =  LOAD  above  which  TP-sensor-very-high  cannot   detected,

unitless.

-  V_LOAD_LOW = LOAD below  which  TP-sensor-very-low  cannot  be  detected,

unitless.

-  V_TP_ERR_MIN = Minimum time that inconsistency must be present  in  order

to set error code, sec.

-  V_TP_LOW = Throttle position above  which  TP-sensor-very-low  cannot  be

detected, counts.

-  V_TP_HIGH = Throttle position below which TP-sensor-very-high  cannot  be

detected, counts.

-  V_WOTNMIN = Engine speed below which error cannot be detected, rpm.

OTHER

-  malfunction(ccm,Pxxx) = Logic  process  imported  from  the  MIL  control

module where Pxxx is a valid OBD-II code.

-  P1121 =  Fault  code  indicating  throttle  position  sensor  voltage  is

inconsistent with MAF-based LOAD, unitless.

31-59

ENGINE INPUT PROCESSING, IN-RANGE TP-LOAD TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INPUT_TP_MAF_OBDII_COM1

BEGIN: input_tp_maf_main

;This process is executed once per background loop.

;OBD_PARM_RST is set whenever OBDII is reset or

;KAM is in error or on-demand tests are entered or exited.

;At this time the error timer needs to be cleared to

;prevent codes being carried forward.

CCM_TST_ENA = 1 ------------|

;continuous testing        |

;enabled                   |OR --|

|     |

CCM_ER_ENA = 1 -------------|     |

;on-demand engine                |

;running test enabled            |

|

TFMFLG = 0 -----------------------|

;TP sensor OK                    |

|

MFMFLG = 0 -----------------------|AND -| Do: input_tp_maf_test

;MAF sensor OK                   |     |  ;check for TP/MAF

|     |  ;consistency

OBD_PARM_RST = 0 -----------------|     |

;no reset                        |     | --- ELSE ---

|     |

TRAC_AIRSW = 1 -------|           |     | V_TP_ERR_TMR := 0

;series throttle     |           |     |  ;not testing

;present             |AND -|     |     |  ;clear ER timer

|     |     |

TRAC_ACTIVE = 0 ------|     |     |

;traction assist not       |OR --|

;active                    |

|

TRAC_AIRSW = 0 -------------|

; no series throttle       |

END: input_tp_maf_main

31-60

ENGINE INPUT PROCESSING, IN-RANGE TP-LOAD TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: input_tp_maf_test

;This process is executed only when explicitly called.

TP_REL < V_TP_LOW ----------|

;TP below lower            |

;threshold                 |

|

LOAD > V_LOAD_LOW ----------|AND -|

;LOAD above lower          |     |

;threshold                 |     |

|     |

N > V_WOTNMIN --------------|     |

|OR --| No action

TP_REL > V_TP_HIGH ---------|     |     |  ;allow timer to increment,

;TP above upper            |     |     |  ;TP and MAF are inconsistent

;threshold                 |     |     |

|     |     |

LOAD < V_LOAD_HI -----------|AND -|     |

;LOAD below upper          |           | --- ELSE ---

;threshold                 |           |

|           | V_TP_ERR_TMR := 0

N > V_WOTNMIN --------------|           |  ;no inconsistency detected

V_TP_ERR_TMR > V_TP_ERR_MIN ------|

;inconsistency present           |

;long enough                     |AND -| Do: malfunction(ccm,P1121)

|     |  ;TP/MAF inconsistency

ECT > TEMPFB ---------------------|     |  ;error detected

;engine is warm                        |

| --- ELSE ---

|

| Do: clear_malf(P1121)

|  ;no error detected

END: input_tp_maf_test

31-61

ENGINE INPUT PROCESSING, THROTTLE POSITION SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

31.13  THROTTLE POSITION SENSOR INPUT AND SELFTEST (CDAN2)

OVERVIEW

First checks are made to determine if an on-demand mode is activated.  If  no

on-demand  mode  is  active  the software falls through and runs the standard

|     sequence.  TP_ENG is supplied averaged from  four  reads  in  the  A/D  list.

|     TP_ENG is the value used when performing any input testing and is reported to

|     the scan tool.  Next, the TP_ENG value is tested for "out of range" or  other

|     failure conditions.  If the TP sensor reading is in range, TP is set equal to

|     TP_ENG.  If TP_ENG is out of range during crank mode TP is set to  RATCH_REP.

|     Otherwise  the  last  "good"  value  will be used for TP until the failure is

|     present for a sufficient amount of time.  At that  time,  the  FMEM  strategy

|     will  substitute  a  value  based on air flow from FN096(AM) for TP and RATCH

|     will be set to RATCH_REP.  This action permits  recognition  of  the  various

|     throttle modes (Closed, Part or WOT).

If an on-demand mode is active.  The KOER or KOEO range  tests  will  be  run

along with the standard continuous tests and FMEM action.

DEFINITIONS

Registers:

-  AM = Air mass flow, lbm/min.

-  APT = Throttle mode position; -1 -> closed throttle, 0 -> part  throttle,

1 -> wide open throttle.

-  j1979_01_11 = Absolute throttle position, 0-255 range, (TP_ENG / 4).

-  LOAD = Universal load as ratio of air charge over standard.

-  N = Engine RPM.

-  P0121_FIL = TP at WOT or P/T during KOER.

-  P0122_FIL = TP-low fault filter.

-  P0123_FIL = TP-high fault filter.

-  P1120_FIL = TP lower than idle position fault filter.

-  RATCH = Closed throttle position, counts.

|      -  RATCH_REP = RATCH replacement value, either RATKAM or RATIV, counts.

-  RATKAM = Closed throttle position stored in KAM, counts.

-  TP = Throttle position, counts.

-  TP_DIF_CTR = Number of samples that throttle position is within limit.

31-62

ENGINE INPUT PROCESSING, THROTTLE POSITION SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|      -  TP_ENG = Averaged or filtered value of TP_CNTS, counts.

|      -  TP_ENG_LAST = Last background value of throttle position sensor, counts.

|      -  TPBAR_MT = Filtered TP with TCRTP_MT, used for RATCH in M/T, counts.

-  TP_REL = Relative TP, (TP - RATCH).

-  TP_REL_H = Relative TP, (TP - RATCH), high byte only.

|      -  TSLPIP = Time since last PIP, millisecs.

Bit Flags:

-  BIFLG = Current state of BOO input; 0 -> brake off, 1 -> brake on.

-  CRKFLG = Crank mode flag.

-  CCM_ER_ENA = Indicates  that  engine  running  on-demand  mode  has  been

requested.

-  CCM_EO_ENA = Indicates that engine off on-demand mode has been requested.

-  CCM_TST_ENA = OBD-II comprehensive component test enable flag; 1 ->  test

enabled.

-  FFG_MAF = OBD-II system FMEM flag for TP;  1  ->  MAF  is  not  currently

reliable.

-  FFG_PIP = OBD-II system FMEM flag for TP;  1  ->  PIP  is  not  currently

reliable.

-  FFG_TP = OBD-II system FMEM flag  for  TP;  1  ->  TP  is  not  currently

reliable.

|      -  KAM_ERROR = Indicates keep alive RAM invalid.

-  OBD_PARM_RST = Flag set whenever OBDII is  reset,  KAM  is  in  error  or

on-demand tests are entered or exited.

-  PxxxMALF = OBD-II malfunction flag for fault  xxx;  1  ->  a  malfunction

currently exists for fault xxx, where Pxxx is a valid OBD-II code.

-  TFMFLG = Flag indicating that TP sensor has failed.

Calibration Constants:

|      -  BIHP = Brake hardware present switch; 1 = hardware present.

-  DTPMAX = Maximum delta between TP input and  RATCH  to  enable  RATCH  to

change value.

-  FIL_HYST = Hysteresis term to  prevent  spurious  exit  of  failure  mode

strategy.

-  FN096 = TP counts above RATCH as a function of air mass, used for TP FMEM

value.

31-63

ENGINE INPUT PROCESSING, THROTTLE POSITION SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  IDLMAF = Maximum AM at idle, lb/min.

-  P0121_LVL = Threshold level for recognition of TP at WOT  or  P/T  during

KOER.

-  P0121_TC = Fault filter time constant for recognition of TP at WOT or P/T

during KOER.

-  P0122_LVL = Threshold level for recognition of TP-low failure.

-  P0122_TC = Fault filter time constant for recognition of TP-low failure.

-  P0123_LVL = Threshold level for recognition of TP-high failure.

-  P0123_TC = Fault filter time constant for recognition of TP-high failure.

-  P1120_LVL = TP lower than idle position threshold.

-  P1120_TC = TP lower than idle position fault filter time constant.

|      -  RAT_KAM_SW = Calibration switch to allow RATCH to be stored in KAM; 0  ->

|         do not store RATCH in KAM, 1 -> store RATCH in KAM.

-  RATADD = RATCH adder on power-up to allow initial ratcheting down.

|      -  RATCH_MIN = RATCH minimum clip, counts.

-  RATDIFF = Minimum difference between RATKAM and RATCH  to  allow  KAM  to

update, counts.

-  RATIV = Inititializing value for RATCH typically 250 counts.

|      -  RATKAM_MIN = Minimum RATCH  value  in  KAM,  below  which  is  considered

|         currupted, counts.

-  RATUP_LD = Allow RATCH update below this LOAD value.

-  TCRTP = Time constant for RATCH, for negative downward movement, sec.

|      -  TCRTP_MT = Time constant for TPBAR_MT, used in M/T RATCH, sec.

-  TCRTP_UP = Time constant for RATCH, for positive upward movement, sec.

-  TP_CTR = Maximum sample count where throttle position is within limit.

-  TP_CTR_MAX = Maximum sample count beyond which do not update RATCH.

-  TP_HI_ER = Maximum valid TP value, counts.

-  TP_HI_KOEO = Maximum value for KOEO on-demand selftest.

-  TP_HI_KOER = Maximum value for KOER on-demand selftest.

-  TP_IDL_ER = Minimum value of TP at closed throttle, counts.

-  TP_LO_ER = Minimum valid TP value, counts.

31-64

ENGINE INPUT PROCESSING, THROTTLE POSITION SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TP_LO_KOEO = Minimum value for KOEO on-demand selftest.

-  TP_LO_KOER = Minimum value for KOER on-demand selftest.

|      -  TSTALL = Elapsed time necessary to indicate a stall.

OTHER

-  malfunction(ccm,Pxxx) = Logic  process  imported  from  the  MIL  control

module, where Pxxx is a valid OBD-II code.

-  store_code(Pxxx) = Stores the fault code specified by Pxxx.

-  P0121 = Throttle position (TP) sensor circuit range problem.

-  P0122 = Throttle position (TP) sensor circuit below minimum voltage.

-  P0123 = Throttle position (TP) sensor circuit above maximum voltage.

-  P1120 = Throttle position  (TP)  sensor  circuit  below  closed  throttle

voltage.

-  P1124 = Throttle position (TP) sensor circuit out of selftest range.

-  tp_codes = Set of {P1120,P0122,P0123}

-  tp_other_codes = Set of {P1124,P0121}

31-65

ENGINE INPUT PROCESSING, THROTTLE POSITION SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INPUT_TP_OBDII_COM1

BEGIN: pid_definitions

unconditionally --------------------| pid_def(j1979_01_11, (TP_ENG / 4))

|  ;parameter required by scan tool

END: pid_definitions

BEGIN: tp_init

|     This logic is executed once at each power-up within 'ram_init'.

|     On power-up, RATKAM plus RATADD is used to initialise RATCH and  allow  RATCH

|     to ratchet down immediately.  RATADD must be calibrated greater than RATDIFF.

|     RAT_KAM_SW = 1 ---------------------| RATCH := RATKAM + RATADD

|      ;RATCH stored in KAM               |  ;increment RATCH

|                                         | TPBAR_MT := RATCH

|                                         | RATCH_REP := RATKAM

|                                         |

|                                         | --- ELSE ---

|                                         |

|                                         | RATCH := RATIV

|                                         | TPBAR_MT := RATIV

|                                         | RATCH_REP := RATIV

END: tp_init

|     BEGIN: tp_main

|     This logic is executed once per background loop and is read top down.

|     At a KAM error or if RATKAM falls below a  reasonable  value,  the  value  of

|     RATKAM,  RATCH,  TPBAR_MT  and  the replacement value RATCH_REP is set to the

|     calibrateable value RATIV.

|     KAM_ERROR = 1 ----------------|

|      ;KAM fault                   |OR --| RATKAM := RATIV + RATADD

|                                   |     | RATCH := RATIV

|     RATKAM < RATKAM_MIN ----------|     | TPBAR_MT := RATIV

|      ;RATCH corrupted                   | RATCH_REP := RATIV

|                                         |  ;use default value

|                                         |

|                                         | --- ELSE ---

|                                         |

|                                         | No action

|     unconditionally --------------------| TP_REL := TP - RATCH

|                                         | TP_REL_H := high byte of TP_REL

31-66

ENGINE INPUT PROCESSING, THROTTLE POSITION SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|     If a reset is requested or the tests are not asked for  filters  need  to  be

|     cleared to prevent codes being carried forward.

CCM_TST_ENA = 0 --------|

|

|     CCM_EO_ENA = 0 ---------|AND -|

|     |

CCM_ER_ENA = 0 ---------|     |OR --| P0121_FIL := 0

|     | P0122_FIL := 0

OBD_PARM_RST = 1 -------------|     | P0123_FIL := 0

|      ;reset                             | P1120_FIL := 0

|  ;clear fault filters

|  ;fault detection not enabled

|                                         |

| Do: tp_failure_check

|  ;check for out of range

|  ;fault action

|                                         |

|                                         | --- ELSE ---

|                                         |

|     CCM_ER_ENA = 1 ---------------------| Do: tp_failure_check

|      ;engine running on-demand          |  ;check for out of range

|      ;demand selftest requested         |  ;fault action

|                                         |

|                                         | Do: check_fault_filters

|                                         |  ;check for continuous fault codes

|                                         |

|                                         | Do: tp_koer_check

|                                         |  ;perform special tests for

|                                         |  ;engine running

|                                         |

|                                         | --- ELSE ---

|                                         |

|     CCM_EO_ENA = 1 ---------------------| Do: tp_failure_check

|      ;engine off on-demand              |  ;check for out of range

|      ;selftest requested                |  ;fault action

|                                         |

|                                         | Do: check_fault_filters

|                                         |  ;check for continuous fault codes

|                                         |

|                                         | Do: tp_koeo_check

|                                         |  ;perform special tests for engine off

|                                         |

|                                         | --- ELSE ---

|                                         |

|     CCM_TST_ENA = 1 --------------------|  Normal engine operation

|

| Do: tp_failure_check

|  ;check for out of range

|                                         |  ;fault action

|                                         |

| Do: check_fault_filters

|  ;check for continuous fault codes

|                                         |

|                                         | --- ELSE ---

|

|                                         | Do Nothing

|     END: tp_main

31-67

ENGINE INPUT PROCESSING, THROTTLE POSITION SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: tp_failure_check

Check for "open/short"  failures.   Perform  FMEM  action  if  a  failure  is

detected.   Set  malfunction  flag  when a malfunction has been present for a

sufficient time.  Otherwise, calculate TP as a function of air mass.

TP_ENG > TP_HI_ER ------------------| P0123_FIL := ROLAV_TC(1,P0123_TC)

;value too high                    |  ;upcount fault P0123

| P0122_FIL := ROLAV_TC(0,P0122_TC)

| P1120_FIL := ROLAV_TC(0,P1120_TC)

|  ;down count faults P0122,P1120

| Do: tp_fmem

|  ;perform FMEM action

| FFG_TP := 1

|  ;set fault flag

|

| --- ELSE ---

|

TP_ENG < TP_LO_ER ------------------| P0122_FIL := ROLAV_TC(1,P0122_TC)

;value too low                     |  ;upcount fault P0122

| P0123_FIL := ROLAV_TC(0,P0123_TC)

| P1120_FIL := ROLAV_TC(0,P1120_TC)

|  ;down count faults P0123,P1120

| Do: tp_fmem

|  ;perform FMEM action

| FFG_TP := 1

|  ;set fault flag

|

| --- ELSE ---

|

TP_ENG < TP_IDL_ER -----------------| P1120_FIL := ROLAV_TC(1,P1120_TC)

;value below idle, assume RATCH    |  ;upcount fault P1120

;is corrupted, reset               | P0122_FIL := ROLAV_TC(0,P0122_TC)

| P0123_FIL := ROLAV_TC(0,P0123_TC)

|  ;down count faults P0122,P0123

| FFG_TP := 1

|  ;set fault flag

|

|                                         | RATCH := RATCH_REP

|  ;reset corrupted RATCH

|

| Do: tp_fmem

|  ;perform FMEM action

|

| --- ELSE ---

(continued on next page)

31-68

ENGINE INPUT PROCESSING, THROTTLE POSITION SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|                                         |

|     P1120MALF = 1 ----------------|     |

|                                   |     |

P0122MALF = 1 ----------------|OR --| Do: tp_fmem

|     |  ;perform FMEM action

P0123MALF = 1 ----------------|     | P0123_FIL := ROLAV_TC(0,P0123_TC)

| P0122_FIL := ROLAV_TC(0,P0122_TC)

| P1120_FIL := ROLAV_TC(0,P1120_TC)

|  ;down count faults

|

|                                         |  ;currently in FMEM mode, remain

|                                         |  ;until fault filters downcount

|                                         |  ;and MALF flags are clear

|                                         |

| FFG_TP := 1

|  ;a malfunction detected

|  ;set fault flag

|

| --- ELSE ---

|

| TP := TP_ENG

|  ;no failure detected, return

|  ;value for normal use

|                                         |

| FFG_TP := 0

|  ;clear fault flag

|

| P0123_FIL := ROLAV_TC(0,P0123_TC)

| P0122_FIL := ROLAV_TC(0,P0122_TC)

| P1120_FIL := ROLAV_TC(0,P1120_TC)

|  ;down count faults

|

| Do: ratch_go

|  ;ratchet RATCH if required

END: tp_failure_check

31-69

ENGINE INPUT PROCESSING, THROTTLE POSITION SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: check_fault_filters

P1120_FIL > P1120_LVL --------------| Do: malfunction(ccm,P1120)

|

| --- ELSE ---

|

P1120_FIL < P1120_LVL - FIL_HYST ---| Do: clear_malf(P1120)

P0122_FIL > P0122_LVL --------------| Do: malfunction(ccm,P0122)

|

| --- ELSE ---

|

P0122_FIL < P0122_LVL - FIL_HYST ---| Do: clear_malf(P0122)

P0123_FIL > P0123_LVL --------------| Do: malfunction(ccm,P0123)

|

| --- ELSE ---

|

P0123_FIL < P0123_LVL - FIL_HYST ---| Do: clear_malf(P0123)

TFMFLG is a 'filtered' fault flag, the FFG_TP is an immediate fault flag.

P1120MALF = 1 ----------------|

;TP below closed throttle    |

|

P0122MALF = 1 ----------------|OR --| TFMFLG := 1

;TP failed low               |     |  ;a malfunction detected

|     |  ;set flag for the use of

P0123MALF = 1 ----------------|     |  ;the rest of the strategy

;TP failed high                    |

| --- ELSE ---

|

| TFMFLG := 0

|  ;no malfunction active

END: check_fault_filters

31-70

ENGINE INPUT PROCESSING, THROTTLE POSITION SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

THROTTLE RATCH MECHANISM

The throttle position ratchet (RATCH) continuously seeks  the  shut  throttle

position  by  searching for a greater or lesser value than its current closed

throttle position.  Two separate time constants are used to  allow  a  slower

change  in  the  positive  direction  and a more rapid change in the negative

direction.  RATCH update  is  allowed  at  any  time  except  in  a  positive

|     direction  during  CRANK,  or  in  a positive direction when the brake is not

|     applied.  This is always provided that the throttle is steady and LOAD is low

|     or TP is below RATCH.  RATCH can change during closed throttle decelerations.

|     For vehicles without BOO  input  (eg  manual  transmission),  positive  RATCH

|     cannot  be  employed.  In this case TP is filtered (TPBAR_MT) and is used for

|     the RATCH replacement, assuming conditions are suitable.

BEGIN:  ratch_go

|     This routine is read top downwards.

TP < RATCH -------------------|

;throttle below ratch        |

|

LOAD < RATUP_LD --------------|OR --|

;closed throttle decel       |     |

|     |

APT = -1 ---------------------|     |

;closed throttle                   |

|

abs(TP_ENG - TP_ENG_LAST) < DTPMAX -|

;throttle steady limit             |

|AND -| TP_DIF_CTR := TP_DIF_CTR + 1

|     FFG_PIP = 0 ------------------------|     |  ;TP is stationary

|      ;no PIP failure                    |     |  ;increment counter to

|     |  ;allow RATCH to update

FFG_MAF = 0 ------------------------|     |

|      ;no MAF failure                    |     |

|                                         |     |

|     CRKFLG = 0 -------------------|     |     |

|      ;not in crank                |OR --|     |

|                                   |           |

|     TSLPIP >= TSTALL -------------|           |

|      ;engine stopped                          | --- ELSE ---

|                                               |

|                                               | TP_DIF_CTR := 0

|                                               |  ;clear counter

|  ;no change to RATCH

|     TP = RATCH -------------------------|

|                                         |OR --| Exit ratch_go

|     TP_DIF_CTR < TP_CTR ----------------|     |  ;no change to RATCH

|                                               |  ;insufficient steady time

|                                               |

|                                               | --- ELSE ---

|                                               |

|                                               | Do: ratch_update

END: ratch_go

31-71

ENGINE INPUT PROCESSING, THROTTLE POSITION SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ratch_update

|     This routine is read top downwards but is  not  entered  if  TP  =  RATCH  or

|     insufficient steady time has elapsed.

|     BIHP = 0 ---------------------------------| TPBAR_MT = ROLAV_TC(TP,TCRTP_MT)

|      ;brake switch hardware not present       |

|                                               | --- ELSE ---

|                                               |

|                                               | No change to TPBAR_MT

|     BIHP = 0 ---------------------------|

|      ;brake switch hardware not present |AND -| ratch := TPBAR_MT

|                                         |     |  ;ratch down

|     TPBAR_MT < RATCH -------------------|     |

|      ;time to update                          | Do: ratch_to_kam

|                                               |

|                                               | --- ELSE ---

|                                               |

|     TP < RATCH -------------------------------| ratch := TP

|                                               | ratch := ROLAV_TC(ratch,TCRTP)

|                                               |  ;ratch down

|                                               |

| Do: ratch_to_kam

|

| --- ELSE ---

|     TP_DIF_CTR <= TP_CTR_MAX -----------|     |

|      ;limit time in which to update     |     |

|     |

|     CRKFLG <> 1 ------------------------|AND -| ratch := TP

|      ;not in crank mode                 |     | ratch := ROLAV_TC(ratch,TCRTP_UP)

|                                         |     |  ;ratch up

BIFLG = 1 --------------------------|     |

|      ;brake applied                           | Do: ratch_to_kam

|                                               |

|                                               | --- ELSE ---

|                                               |

|                                               | No action

|     END: ratch_update

31-72

ENGINE INPUT PROCESSING, THROTTLE POSITION SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ratch_to_kam

|     RATCH is clipped between RATCH_MIN and RATIV at the top of this routine.

RATCH is stored in KAM to provide a rapid and accurate TP measurement for use

during  key-on  and  prevent  the normal delay in learning RATCH.  It is also

used as a replacement FMEM value.  RATCH is only  stored  in  KAM  if  it  is

|     different  from RATKAM but above a defined tolerance (RATDIFF) and storage of

|     RATCH in KAM is required, (RAT_KAM_SW = 1).   The  RATDIFF  qualification  is

|     used  to prevent excessive KAM updates with a suggested calibration between 2

|     and 7 counts.

|     unconditionally --------------------------| ratch := min(ratch,RATIV)

|                                               | ratch := max(ratch,RATCH_MIN)

|                                               |  ;clip ratch between

|                                               |  ;RATCH_MIN and RATIV

|                                               |

|                                               | RATCH = ratch

|                                               |  ;and store away

|     abs(RATCH - RATKAM) < RATDIFF ------|

|      ;insufficient divergency           |OR --| Do nothing

|                                         |     |

|     RAT_KAM_SW = 0 ---------------------|     | --- ELSE ---

|      ;RATCH not stored in KAM                 |

|                                               | RATKAM := RATCH

|                                               |  ;store RATCH in KAM

|                                               |

|                                               | RATCH_REP := RATCH

|                                               |  ;use KAM value of RATCH

|                                               |  ;for replacement use

END: ratch_to_kam

31-73

ENGINE INPUT PROCESSING, THROTTLE POSITION SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: tp_koer_check

Check against Key-On engine running limits.  If an  error  is  detected,  set

|     P1124MALF  flag.   If,  at any time part throttle or WOT is detected, set the

P0121MALF flag as this condition can cause the EGR test to lock indefinitely.

FFG_TP = 1 -------------------------| Exit module

;TP sensor immediate failure       |  ;continuous test detected

|  ;failure, no need to test

|  ;tighter KOEO limits

|

| --- ELSE ---

TP_ENG > TP_HI_KOER ----------|     |

;TP above maximum for KOER   |OR --| Do: store_code(P1124)

|     |  ;TP out of selftest range

TP_ENG < TP_LO_KOER ----------|     |

;TP below minimum for KOER         |

| --- ELSE ---

|

APT = 0 ----------------------|     |

;part throttle               |OR --| P0121_FIL := ROLAV_TC(1,P0121_TC)

APT = 1 ----------------------|     |  ;upcount fault P0121

;WOT                               |  ;TP at part throttle or WOT

|

| --- ELSE ---

|

| No failures detected

| Do: clear_malf(P1124)

| P0121_FIL := ROLAV_TC(0,P0121_TC)

|  ;downcount fault P0121

P0121_FIL > P0121_LVL --------------| Do: store_code(P0121)

|

| --- ELSE ---

|

P0121_FIL < P0121_LVL - FIL_HYST ---| Do: clear_malf(P0121)

END: tp_koer_check

31-74

ENGINE INPUT PROCESSING, THROTTLE POSITION SENSOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: tp_koeo_check

Check against Key-on engine  off  limits.   If  an  error  is  detected,  set

P1124MALF flag.

FFG_TP = 1 -------------------------|  ;continuous test detected

;TP sensor immediate failure       |  ;failure, no need to test

|  ;tighter KOEO limits

|

| --- ELSE ---

TP_ENG > TP_HI_KOEO ----------|     |

;TP above KOEO limits        |OR --| Do: store_code(P1124)

|     |  ;TP out of selftest range

TP_ENG < TP_LO_KOEO ----------|     |

;TP below KOEO limits              |

| --- ELSE ---

|

| Do: clear_malf(P1124)

|  ;no failure detected

END: tp_koeo_check

|     BEGIN: tp_fmem

|     In crank mode or at idle air flow, use RATCH_REP in place of TP.   Otherwise,

|     until  a  failure has been present long enough to set a failure flag, use the

|     last good value of TP.  Once the failure flag is set, infer TP from air mass,

|     FN096(AM).

|     CRKFLG = 1 -------------------|

|      ;crank mode                  |OR --| TP := RATCH_REP

|                                   |     |

|     AM < IDLMAF ------------------|     |

|                                         | --- ELSE ---

|                                         |

|     TFMFLG = 1 -------------------------| RATCH := RATCH_REP

|                                         | TP := RATCH + FN096(AM)

|                                         |  ;TP sensor out of limits

|                                         |  ;but probably NOT due to

|                                         |  ;low battery voltage

|                                         |

|                                         | --- ELSE ---

|                                         |

|                                         | No change to TP

|                                         |  ;TP sensor data unreliable

|                                         |  ;Do NOT update until

|                                         |  ;confident data valid

|     END: tp_fmem

31-75

CONTINUOUS SELF TEST, VEHICLE MOVING DETECTOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

31.14  VEHICLE MOVING DETECTOR (CDAL0)

OVERVIEW

This module performs the inferred vehicle motion  check  used  in  all  speed

sensor  tests.  The logic is based on the principle that if the vehicle is in

drive and the torque converter is full, it is then possible to infer  whether

or not the vehicle is moving based on the vehicle operating conditions.

Inferred vehicle speed for an AODE transmission is based  on  the  following;

vehicle speed (OK to test for OSS output), output shaft speed (OK to test for

VSS output), and WOT stall speed of the converter (used only  to  allow  test

entry in the event that both VSS and OSS have failed).

In an attempt to improve robustness of inferred  vehicle  motion,  logic  has

been  added  which  attempts to determine when the transmission is in gear (a

load is present on the engine).

The purpose of this logic is to prevent transmission failures which result in

no  torque capacity (transmission is in neutral) when the transmission should

be in gear from causing false inferred vehicle motion.

This logic uses the transmission input torque calculation (engine  combustion

torque  minus  torque  used  to  accelerate  the  engine)  to  determine if a

transmission load is present.

DEFINITIONS

Registers:

-  ATMR1 = Time since engine start.

-  EIP_TMR = Engagement in progress timer.

-  I4X4L = 4X4 input; 1 -> 4X4 selected.

-  IPDL = Unfiltered MLPS position.

-  ITP = Instantaneous throttle angle, A/D counts.

-  LOAD = Universal LOAD as ratio of air charge over standard.

-  N = Engine speed, RPM.

-  NEBART = Filtered engine RPM for transmission.

-  NOBART = Filtered Output Shaft Speed in RPM.

-  TQ_BRK_SBAR = Filtered engine combustion torque.

-  TR_LOAD_FLG  =  Transmission  load  flag;  1  ->  based  on  torque   the

transmission is in gear.

31-76

CONTINUOUS SELF TEST, VEHICLE MOVING DETECTOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TRANS_TQ_IN = Transmission input torque, engine combustion  torque  minus

torque used to accelerate the engine.

-  VMOVING = 1 -> Vehicle is moving.

-  VSBARV = Filtered vehicle speed for self test.

-  VSSTMR = Vehicle speed sensor fault timer.

Bit Flags:

-  MFMFLG = MAP/MAF FMEM flag.

-  NDSFLG = NEUTRAL/DRIVE flag; 1 -> drive.

-  TFMFLG = TP FMEM flag.

-  TST_ENA_FLG = Transmission self test enable  flag;  1  ->  TOT  dependent

transmission self test enabled.

Calibration Constants:

-  NE_VMOV_MAX = Maximum engine speed at which MPH vehicle  motion  will  be

inferred.

-  OS_MOV = Output shaft speed above which test will be executed.

-  TQ_CK_SW = Calibration switch to enable/disable checking torque  as  part

of inferring vehicle motion; 1 -> torque logic enabled.

-  TQ_CK_VMOV = Engine torque is high enough to infer the vehicle is moving.

-  TQ_VMOV = Transmission input torque is high enough to infer  the  vehicle

is moving.

-  TRLOAD = Transmission Load;

0 -> Manual Transmission, no clutch or gear switches,

forced neutral state (NDSFLG = 0),

1 -> Manual Transmission, no clutch or gear switch,

2 -> Manual Transmission, one clutch or gear switch,

3 -> Manual Transmission, both clutch and gear switches,

4 -> Auto Transmission, non-electronic, neutral drive switch,

5 -> Auto Transmission, non-electronic, neutral pressure switch

(AXOD),

6 -> Auto Transmission, electronic, PRNDL sensor - park,

reverse, neutral, overdrive, manual 1, manual 2.

-  V_MOV_DLY = Start up delay before allowing VMOVING flag to set  to  avoid

codes from partially filled converter.

-  VSAMIN  =  Minimum  engine  speed  to  do   VSS   test   with   automatic

transmissions, RPM.

-  VS_MOV = Vehicle speed above which test will be executed.

-  VSSTP = Minimum TP to enter VSS test.

31-77

CONTINUOUS SELF TEST, VEHICLE MOVING DETECTOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  V_VSS_LOAD = Minimum air charge ratio to do CVIP VSS test.

-  V_VSS_MULT = Multiplier for FN689V when in 4X4.

-  V_VSS_NRHI = High eng.  speed for load range to infer vehicle movement.

-  V_VSS_NRLO = Low engine speed for load range to infer vehicle movement.

31-78

CONTINUOUS SELF TEST, VEHICLE MOVING DETECTOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INPUT_VMOVING_COM4

TQ_CK_SW = 0 -----------------------|

;Checking torque for VMOVING       |

;cal'ed out                        |

|OR --| TR_LOAD_FLG = 1

TQ_BRK_SBAR >= TQ_CK_VMOV ----|     |     | ;Transmission input torque

;Engine torque high enough   |     |     | ;indicates a load is present.

|     |     | ;Used for both manual and

TRANS_TQ_IN >= TQ_VMOV -------|AND -|     | ;automatic transmission

;Trans input tq high enough  |           |

|           |

NEBART < NE_VMOV_MAX ---------|           |

;Engine speed low enough                 |

| --- ELSE ---

|

| TR_LOAD_FLG = 0

| ;Transmission is not in gear

31-79

CONTINUOUS SELF TEST, VEHICLE MOVING DETECTOR - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TRLOAD => 4 -------------------------|

;auto trans                         |

|

ATMR1 >= V_MOV_DLY ------------------|

;long enough since startup          |

;to fill torque converter           |

|

EIP_TMR = 0 -------------------------|

;engagement completed               |

|

IPDL <> 0 ---------------------------|

;PRNDL in range                     |

|

TST_ENA_FLG = 1 ---------------------|

;TOT dependent logic                |

;enabled                            |

|

TFMFLG = 0 --------------------------|

;TP sensor OK                       |

|AND -|

NDSFLG = 1 --------------------------|     |

;in drive                           |     |

|     |

I4X4L = 0 ---------------|           |     |

;not in 4X4 mode        |           |     |

|AND -|     |     |

NEBART => VSAMIN --------|     |     |     |

;high RPM                     |OR --|     |

|     |     |

NEBART >= VSAMIN * V_VSS_MULT--|     |     |

;min. rpm in 4X4 mode               |     |

|     |

ITP => VSSTP ------------------------|     |

|     |

TR_LOAD_FLG = 1 ---------------------|     |

|OR --| VMOVING := 1

TRLOAD <= 3 -------------------------|     |     |  ;Conditions satisfied to

;manual trans                       |     |     |  ;infer vehicle motion

|     |     |

MFMFLG = 0 --------------------------|     |     |

;MAP sensor OK                      |     |     |

|AND -|     |

N <= V_VSS_NRHI ---------------------|     |     |

|     |     |

LOAD => V_VSS_LOAD ------------------|     |     |

|     |     |

N => V_VSS_NRLO ---------------------|     |     |

|     |     |

TR_LOAD_FLG = 1 ---------------------|     |     |

|     |

VSBARV >= VS_MOV --------------------------|     |

|     |

NOBART >= OS_MOV --------------------------|     |

| --- ELSE ---

|

| VMOVING := 0

| VSSTMR := 0

|  ;clear timer

31-80

ENGINE INPUT PROCESSING, VEHICLE SPEED (DARC) AND SELFTEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

31.15  VEHICLE SPEED INPUT AND SELF TEST (CDAN0)

OVERVIEW

The variable reluctance type Vehicle Speed Sensor produces an AC signal  with

frequency  proportional  to  vehicle speed.  Through appropriate gearing, the

sensor generates 8000 cycles/mile, for a frequency range of 0 HZ at 0 MPH  to

283.3  Hz at 127.5 MPH.  Interface hardware in the EEC converts the AC signal

to a digital signal for input to the CPU.  The  strategy  updates  unfiltered

vehicle  speed  (vs) once per background loop if at least one new rising edge

was  received  (MPHCNT  >  0)  during  the  previous  loop.   If,  after  255

milleseconds, no new signals are received (< 1.75 MPH), vs is set to 0.  This

ensures a zero vehicle speed if the vehicle  is  stopped  or  if  the  sensor

fails.

The vehicle speed sensor input processing is designed to calculate unfiltered

vehicle  speed  (VS_HW), and the vehicle speed limiting parameter VS_LIM_BAR.

Once vehicle speed has been calculated the  VSS  diagnostic  module  will  be

called to perform self test on the sensor.

DEFINITIONS

Registers:

-  NOBART = Filtered Output Shaft Speed in RPM.

-  RT_NOVS = Ratio of actual N-over-V to base N-over-V stored.

-  TSLMPH = Time since last rising vehicle speed edge was  seen.   Reset  to

zero when edge is seen, counts up in background when no edge is seen.

-  VS = Instantaneous vehicle speed.

-  VS_HW = Instantaneous vehicle speed, mph.

-  VSBAR = Filtered vehicle speed.

-  VSBARV = Filtered vehicle speed for Self Test.

-  VS_LIM_BAR = High range of VSBAR used for vehicle speed limiting.

-  VS_PRD = VSS input DARC period, DARC ticks.

-  VS_PRD_MIN_1 = VSS input DARC period - previous, DARC ticks.

-  VS_PREV = Previous value of unfiltered vehicle speed vs.

Bit Flags:

-  FIRST_MPH = Counter that indicates when the first edge is seen.

-  VSFMFLG = Vehicle speed sensor failure mode flag.

31-81

ENGINE INPUT PROCESSING, VEHICLE SPEED (DARC) AND SELFTEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Calibration Constants:

-  NVBASE = Base "N-over-V" ratio.

-  TCVS_LIM_BAR = Time constant for VS_LIM_BAR.

PROCESS

STRATEGY MODULE: INPUT_VSS_DARC_COM2

BEGIN: pid_definitions

Parameter  Identification  Declaration  for  j1979_01_0d,  vehicle  speed  in

kilometers/hour.

pid_def(j1979_01_0d, VSBAR*1.609)

END: pid_definitions

Once per background; the following logic is executed:

unconditionally --------------| VS_PREV = VS_HW

| Do: vehicle_speed_calculations

|

| Do: intrn_vss_test_comx

|  ;call VSS self test module

31-82

ENGINE INPUT PROCESSING, VEHICLE SPEED (DARC) AND SELFTEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: vehicle_speed_calculations

Calculate unfiltered vehicle speed vs.

new_vs_period ----------------| TSLMPH := 0

;at least one period seen    |

;since the last update       |

TSLMPH >= 0.255 --------------| FIRST_MPH := 0

;timed out - reset           |

| VS_HW := 0

|

| VS := 0

|

| --- ELSE ---

|

FIRST_MPH = 0 ----------------| FIRST_MPH := 1

;one period seen             |

| VS_HW := min( 405,000 / VS_PRD, 255.0)

|

| VS := min( VS_HW, 127.5)

|

| --- ELSE ---

|

| ave_vs_prd := (VS_PRD + VS_PRD_MIN_1) / 2

|

| VS_HW := min( 405,000 / ave_vs_prd, 255.0)

|

| VS := min( VS_HW, 127.5)

NOTE: TSLMPH is a seconds timer with milliseconds resolution.

Calculate unfiltered vehicle speed VS:

VSFMFLG = 0 ------------------| VS_LIM_BAR := ROLAV(VS_HW,TCVS_LIM_BAR)

|

| VSBAR := VSBARV

|

| --- ELSE ---

|

| VS_LIM_BAR := NOBART / (NVBASE * RT_NOVS)

|

| VSBAR := NOBART / (NVBASE * RT_NOVS)

END: vehicle_speed_calculations

31-83

ENGINE INPUT PROCESSING, CLUTCH SWITCH - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

31.16  CLUTCH SWITCH INPUT AND SELFTEST (CDAL0)

OVERVIEW

This test uses the clutch-starter safety interlock, whereby the clutch  pedal

must be depressed for the starter to function.  This test looks for the first

time that the engine starts to turn and assumes  that  this  is  due  to  the

starter  motor.   If  the clutch switch input indicates that the clutch pedal

has not been depressed, then this test stores the non-MIL code P0704.

Unlike most of the continuous tests,  this  test  runs  when  the  engine  is

cranking.

Note that either of the following could result in the setting of a false code

P0704 when the clutch switch is functioning correctly:

1) The vehicle is push started with both the clutch

released (engaged) and the vehicle in gear,

i.e. the clutch is never depressed during the

push-start attempt.

2) A mechanic uses a "bump starter" to crank the engine

with the ignition on.

To avoid other situations which could result in a false code P0704, this test

is run no more than once per power-up.

CALIBRATION NOTES:

It is important to realise that a clutch switch may not be fitted or  it  may

be fitted in series with a gear switch.  The calibration switches must be set

accordingly and any code output be viewed carefully.

PIP periods which are longer than 0.8 seconds are treated as

zero RPM (N = 0), therefore:

4 cylinder applications N < 37.50 RPM is treated as N = 0

6 cylinder applications N < 25.00 RPM is treated as N = 0

8 cylinder applications N < 18.75 RPM is treated as N = 0

Calibrating  CPP_NMIN  or  CPP_NMAX  below  these  values  is  the  same   as

calibrating AT these values.

31-84

ENGINE INPUT PROCESSING, CLUTCH SWITCH - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  INDS = Neutral/drive input.

-  N = RPM.

Bit Flags:

-  CPP_DISABLE = Indicates clutch switch test disabled.

-  FFG_CPP = Clutch pedal switch unreliable.

Calibration Constants:

-  CPP_NMAX = Maximum RPM for continuous clutch switch test.

-  CPP_NMIN = Minimum RPM for continuous clutch switch test.

-  CPP_SEL = Calibration test select switch; 1 -> do test.

-  NDS_SWPT = NDS input switch point.

-  TRLOAD = Transmission load switch;

0 -> Manual Transmission, no clutch or gear switches,

forced neutral state, NDSFLG = 0.

1 -> Manual Transmission, no clutch or gear switch.

2 -> Manual Transmission, one clutch or gear switch.

3 -> Manual Transmission, both clutch and gear switches.

4 -> Auto Transmission, non-electronic, neutral drive switch.

5 -> Auto Transmission, non-electronic, neutral pressure

switch, (AXOD).

6 -> Auto Transmission, electronic, PRNDL sensor - park,

reverse, neutral, overdrive, manual 1, manual 2.

7 -> Auto Transmission, electronic, multiple discrete switches

for manual lever position, (F4E).

OTHER

-  CPP_CODES = Set of {P0704}

-  store_code(Pxxx) = Stores the fault code specified by Pxxx.

-  P0704 = Clutch switch input circuit malfunction.

31-85

ENGINE INPUT PROCESSING, CLUTCH SWITCH - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: INPUT_CPP_OBDII_COM1

BEGIN:  cpp_test

CPP_DISABLE = 1 ------------------|

;clutch test disabled            |

;during this power up            |

|

N =< CPP_NMIN --------------------|

;engine is not cranking          |OR --| Exit continuous

|     | clutch switch test

CPP_SEL <> 1 ---------------------|     |

;clutch test is calibrated out   |     |

|     |

TRLOAD >= 4 ----------------------|     |

;automatic transmission                |

| --- ELSE ---

N >= CPP_NMAX --------------------|     |

;engine is turning too           |     |

;fast to do test                 |OR --| CPP_DISABLE := 1

|     |  ;disable this test for the

INDS >= NDS_SWPT -----------------|     |  ;remainder of this power-up

;transmission is in neutral            |

;and/or clutch is depressed            |

| --- ELSE ---

|

| Do: store_code(P0704)

|  ;clutch switch not activated

| FFG_CPP := 1

|  ;set failure flag

| CPP_DISABLE := 1

|  ;disable this test for the

|  ;remainder of this power-up

END: cpp_test

31-86

INPUT AND FAILURE MODE AND SELF TEST, NOVS CALCULATION - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

31.17  ENGINE SPEED OVER VEHICLE SPEED (NOVS) (CDAL0)

OVERVIEW

NOVS is the ratio of engine speed over vehicle speed.  It is  used  to  infer

the  transmission  gear  ratio  selected  -  both  in  automatic  and  manual

transmissions

DEFINITIONS

INPUTS

Registers:

-  N = RPM - engine speed in revolutions per minute.

-  VSBAR = Filtered vehicle speed.

OUTPUTS

Registers:

-  NOVS = N/VSBAR to infer transmission gear, rpm/mph.

PROCESS

STRATEGY MODULE: INPUT_NOVS_COM1

always ---------------------------------| NOVS := N/VSBAR

|  ;clip NOVS between 0 and 255

31-87

INPUT AND FAILURE MODE AND SELF TEST, BATTERY VOLLTAGE (ML2) - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

31.18  BATTERY VOLTAGE (VBAT) (CDAL0)

OVERVIEW

The  VBAT  calculation  is  a  time-dependent  rolling  average   filter   of

instantaneous  battery  voltage.   The  VBAT  time  constant  TCVBAT is not a

calibration constant and is set to produce a 0.1  second  VBAT  average  time

constant.

DEFINITIONS

Registers:

-  IIVPWR = Ignition key-on power, A/D counts.

-  VBAT = Battery Voltage.

Calibration Constants:

-  KSF = Keypower scaling factor reciprocal, unitless

-  TCVBAT = Time constant for VBAT, (NON calibrateable) sec.

PROCESS

STRATEGY MODULE: INPUT_VBAT_COM2

Instantaneous battery voltage is calculated from:

|                    5 VOLTS

unconditionally --------------| vbat := IIVPWR * ----------- * KSF

|                  1023 COUNTS

unconditionally --------------| VBAT := ROLAV(vbat,TCVBAT)

31-88

INPUT AND FAILURE MODE AND SELF TEST, CLOCK_SEC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

31.19  CLOCK_SEC (CDAL0)

OVERVIEW

CLOCK_SEC is a register which contains a clock  in  engineering  units.   The

clock  rolls  over  every  60  seconds  and displays seconds with millisecond

accuracy.  As with any other background calculation,  the  clock  is  updated

every background loop.

DEFINITIONS

INPUTS

Registers:

-  BG_TMR = Time to complete the previous background loop.

-  CLOCK_SEC = DAC able clock, seconds.

OUTPUTS

Registers:

-  CLOCK_SEC = See above.

PROCESS

STRATEGY MODULE: INPUT_CLOCK_SEC_COM1

always ---------------------------------| CLOCK_SEC := CLOCK_SEC + BG_TMR

|  ;add current background loop

|  ;to the clock

CLOCK_SEC >= 60.000 --------------------| CLOCK_SEC := CLOCK_SEC - 60.000

|  ;allows clock to roll over

31-89

ENGINE INPUT PROCESSING,  AIR CONDITION PRESSURE SWITCH  - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

31.20  AC PRESSURE (ACP) SWITCH INPUT SIGNAL PROCESSING (CDAN0)

OVERVIEW

This module is for processing the AC pressure switch input  via  an  analogue

input  pin.   It  contains  an inversion as the input is normally high and is

pulled low when the pressure switch is tripped.  The signal  is  sampled  and

considered  to indicate an "on-condition" when the input exceeds (IACPSW) 512

counts.  The module also contains a check on  ACPSW_HP  to  ensure  that  the

input should be read as ACPSW and not something else.  Note that in the logic

below, "IACPSW" refers to the CPU pin designated to be the AC pressure switch

input.

DEFINITIONS

Registers:

-  ACPSW = AC pressure switch input; 1 -> AC pressure high.

-  IACPSW = The analogue ACPSW input register.

Calibration Constants:

-  ACPSW_HP = AC pressure switch hardware present.

PROCESS

STRATEGY MODULE: INPUT_ACPSW_COM2

ACPSW_HP = 1 ---------------------|

|AND -| ACPSW := 1

IACPSW <= 512 counts -------------|     |

| --- ELSE ---

|

| ACPSW := 0

31-90

INPUT ENGINE PROCESSING, THROTTLE POSITION MULTI SAMPLING  - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

31.21  THROTTLE POSITION MULTI SAMPLING (CDAN2)

|     OVERVIEW

|     To prevent noise corrupting the TP signal, the TP is sampled  4  times  every

|     background  loop and averaged.  This is acheived by placing the TP at the top

|     and bottom of the A to D read list, with the other  two  splitting  the  list

|     into thirds.

|     DEFINITIONS

|     Registers:

|      -  TP_CNT1 = 1st input TP from A to D, counts.

|      -  TP_CNT2 = 2nd input TP from A to D, counts.

|      -  TP_CNT3 = 3rd input TP from A to D, counts.

|      -  TP_CNT4 = 4th input TP from A to D, counts.

|      -  TP_ENG1 = 1st TP sample, counts.

|      -  TP_ENG2 = 2nd TP sample, counts.

|      -  TP_ENG3 = 3rd TP sample, counts.

|      -  TP_ENG4 = 4th TP sample, counts.

|      -  TP_ENG = Averaged TP for error analysis, counts.

|      -  TP_ENG_LAST = Last background value of throttle position sensor, counts.

|     PROCESS

|                           STRATEGY MODULE: INPUT_TP_BG_COM1

|     BEGIN: tp_sample

|     Sample the TP sensor four times during each  A  to  D  read,  with  the  time

|     intervals  being spaced as equal as possible.  TP_ENG is supplied to the main

|     TP routine.

|     Every background loop:

|     unconditionally -------| TP_ENG1 := TP_CNT1

|                            | TP_ENG2 := TP_CNT2

|                            | TP_ENG3 := TP_CNT3

|                            | TP_ENG4 := TP_CNT4

|     unconditionally -------| TP_ENG_LAST := TP_ENG

|                            |  ;store current value before recalculating

|                            | TP_ENG := (TP_ENG1+TP_ENG2+TP_ENG3+TP_ENG4)/4

31-91

INPUT ENGINE PROCESSING, THROTTLE POSITION MULTI SAMPLING  - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|     END: tp_sample

31-92

CHAPTER 32

KEEP ALIVE MEMORY

32-1

KEEP ALIVE MEMORY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

32.1  KEEP ALIVE MEMORY (KAM) QUALIFICATION TEST (CDAM0)

OVERVIEW

Each time the vehicle is started, the data stored in KAM may  not  be  valid.

Power  interruptions,  noise,  etc.,  may have altered KAM contents.  Or, the

computer may not be reading KAM registers correctly  because  of  a  hardware

fault.   When  the  KAM registers are intialized, a special binary pattern is

written into three bytes of KAM.  The KAM register names  are  KAMQA,  KAMQB,

and  KAMQC.   During each background loop, the KAM registers are tested.  The

KAM qualification test judges the validity of the KAM data by looking for the

proper binary pattern.  The alternate courses of action are either:

1) If the proper pattern is present, the KAM data is  considered  OK  for

use by the strategy.

2) If not present, the KAM data is suspect.  The KAM is over-written to a

set  of  initial values.  The inital values are also used in place of the

KAM data when the strategy references KAM.

The KAM registers KAMQA, KAMQB, and KAMQC are assigned to different areas  of

the  KAM.   This will help protect for partial KAM failures.  The assignments

are:

KAM     KAM

Register        Address

---------       -------

KAMQA   LOWEST ADDRESS OF KAM

KAMQB   MIDDLE ADDRESS OF KAM

KAMQC   HIGHEST ADDRESS OF KAM

The KAM qualification test is normally performed each  background  loop  when

the computer is running.

32-2

KEEP ALIVE MEMORY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  BP = Barometric pressure stored in KAM, " Hg.

-  GR_CM = Commanded gear for shift solenoids.

-  GR_CM_KAM = KAM equivalent of GR_CM for reinit protection.

-  ISCKAM0 = Idle Speed KAM IPSIBR cell 0.

-  ISCKAM1 = Idle Speed KAM IPSIBR cell 1.

-  ISCKAM2 = Idle Speed KAM IPSIBR cell 2.

-  ISCKAM3 = Idle Speed KAM IPSIBR cell 3.

-  ISCKAM4 = Idle Speed KAM IPSIBR cell 4.

-  ISCKAM5 = Idle Speed KAM IPSIBR cell 5.

-  ISKSUM = Idle Adaptive airflow check sum.

-  KAMQA = KAM Qualification test register 1.

-  KAMQB = KAM Qualification test register 2.

-  KAMQC = KAM Qualification test register 3.

-  NOVCTR = NOV calculation sampling counter.

-  RT_NOVS_KAM = NOV ratio in KAM.

-  V_MODE_SETUP = VIP Throttle Adjust Mode enable flag; 1 -> enabled.

Bit Flags:

-  FLG_FRST_NOV = First pass to store NOV in KAM flag:  0 -> RT_NOVS_KAM has

not been loaded; 1 -> RT_NOVS_KAM has been loaded.

-  FLG_NOV_KAM = Flag indicating at least one update  of  RT_NOVS_KAM  since

last KAM update.

-  KAM_ERROR = KAM error flag; 1 -> KAM data invalid.

-  LEGOFG11 = Lack of HEGO11 switch.

-  LEGOFG21 = Lack of HEGO21 switch.

-  VIP_KAM = Indicates KAM invalid for VIP.

Calibration Constants:

-  BAPFMM = Default value for BP failure.

-  BPMAX = Maximum allowable IBAP value, " Hg.

32-3

KEEP ALIVE MEMORY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  BPMIN = Minimum allowable IBAP value, " Hg.

-  GRCMKIV = KAM initialization value for GR_CM.

-  RTNVMN = Minimum valid RT_NOVS_KAM.

-  RTNVMX = Maximum value for RT_NOVS_KAM.

PROCESS

STRATEGY MODULE: KAM_CC

KAM QUALIFICATION TEST LOGIC

(performed each background loop)

KAM_ERROR = 1 ---------------------| KAMQA = 10101010 BINARY

| KAMQB = 11000110 BINARY

| KAMQC = 01110101 BINARY

|  ;Write the special binary patterns

|  ;to KAM:

| KAM_ERROR = 0

|

| --- ELSE ---

KAMQA <> 10101010 BINARY ----|     |

|     |

KAMQB <> 11000110 BINARY ----|     |

|     |

KAMQC <> 01110101 BINARY ----|OR --| KAMQC = 0

|     | KAM_ERROR = 1

RT_NOVS_KAM < RTNVMN --------|     |  (Individual features will reset

|     |  parameters stored in KAM as required.

RT_NOVS_KAM > RTNVMX --------|     |  Note that this flag will be cleared

|  during the next background loop and

|  reset here the following loop (toggle)

|  if a chronic error is present.)

|

| VIP_KAM = 1

|  (Send error flag to Self-Test.)

|

| ISCKAMn = 0.0

| ISKSUM = 0.0

| BP = BAPFMM

| GR_CM_KAM = GRCMKIV

| GR_CM = GRCMKIV

| NOVCTR = 0

| RT_NOVS_KAM = 1.0

| FLG_FRST_NOV = 0

| FLG_NOV_KAM = 0

| LEGOFG11 = 0

| LEGOFG21 = 0

32-4

KEEP ALIVE MEMORY - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

ISCKAM QUALIFICATION

(Done during Power-up sequence)

|ISCKAM0 + ISCKAM1 + ISCKAM2

+ ISCKAM3 - ISKSUM| <= 1 Bit ----| Assume the ISCKAMs are valid.

| No action taken.

| ISKSUM = ISCKAM0 + ISCKAM1

|  + ISCKAM2 + ISCKAM3

|

| --- ELSE ---

|

| Assume ISCKAMs Data are invalid.

| Re-initialize the ISCKAM

|  ISCKAM0 = 0

|  ISCKAM1 = 0

|  ISCKAM2 = 0

|  ISCKAM3 = 0

|  ISKSUM = 0

VIP THROTTLE MODE SET

(Done each background loop while in running VIP)

V_MODE_SETUP = 1 ------------------| Re-initialize the ISCKAM

(VIP Throttle Adjust Mode)        | ISCKAM0 = 0

| ISCKAM1 = 0

| ISCKAM2 = 0

| ISCKAM3 = 0

| ISKSUM = 0

During power-up sequence, check the range of BP.

BP <= BPMAX -----------------|

|AND -| BP is OK

BP >= BPMIN -----------------|     |

| --- ELSE ---

|

| BP = BAPFMM

32-5

KAM, RAM, AND ROM SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

32.2  ON DEMAND SELF TEST FOR EEC MEMORY (KAM, RAM, ROM) (CDAK0)

OVERVIEW

This module is executed only when called by the diagnostic executive  at  the

beginning  of the KOEO on demand test.  This test consists of four parts, the

"Instruction", "RAM", "ROM Check Sum" and "KAM Test Bytes" tests.

First, the "Instruction" test is performed.  The "Instruction test" takes the

CPU  through  a range of instructions (such as Add, Shift, Exclusive Or, And,

Or, Multiply, Divide, and Stack manipulation)  testing  the  result  at  each

step.   If the test fails, the MIL is turned on and the module is locked into

an infinite  loop  (strategy  execution  stops).   A  failure  of  this  test

indicates that the CPU is unable to properly perform instructions.

If the "Instruction" test passes, then the "RAM" test is executed.   Starting

at  the  beginning  of  external  RAM, the contents of each location is first

saved, then the value AAAA Hex is written and read  back.   If  the  returned

value  is  AAAA  Hex,  5555 Hex is written to and read from the register.  If

5555 Hex is read back, the contents of the register is restored and the  test

moves  to  the  next  register.   As in the instruction test, if a failure is

detected, the MIL is turned on and the module is locked into an infinite loop

(strategy  execution  stops).  By using the values AAAA Hex (1010...  Binary)

and 5555 Hex (0101...  Binary) each bit in external RAM is tested in both the

set and cleared states.  A failure indicates that at least one bit can not be

changed and/or read correctly.

Next, a 16 bit addition of the contents of the ROM locations is performed.  A

parameter  (ROM_TO)  is  set  to  a value that should cause the result of the

summation to be zero.  If the value is not zero, then some value in  ROM  has

been  corrupted.  (Note, during development, the use of patched chips, RCONs,

etc.  can cause this test to fail.) If a failure is detected, Code  P0605  is

set and Self Test continues.

Finally, The parameter VIP_KAM is checked, and if it has been set by the  KAM

QUALIFICATION TEST LOGIC (a check of 3 memory locations failed) code P0603 is

set.  This is usually an indication that the keep-alive power has been  lost.

Note  that  on the first powerup following a battery disconnect, this code is

expected and would not necessarily indicate an EEC hardware failure.

32-6

KAM, RAM, AND ROM SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

INPUTS

Bit Flags:

-  DISABLE_EOLT = Prevents re-entry into EOL test.

-  VIP_KAM = Indicates KAM invalid for Self Test.

OUTPUTS

Bit Flags:

-  DISABLE_EOLT = See above.

OTHER

-  store_code(Pxxx) = Logic process imported from the TBD module where  Pxxx

is a valid Self Test code.

-  eca_other_codes = SET OF {P0603,P0605}

-  P0605 = ROM check sum test failed.

-  P0603 = KAM test failure.

32-7

KAM, RAM, AND ROM SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: KAM_RAM_ROM_TEST_COM1

EO_STATUS = ECU_EO_INIT ----------------| Do: Module instruction test

|  (test module for capability to

|   perform instructions)

|

| Do: RAM Test

|  (write / read patterns to

|   external RAM)

|

| Do: ROM Test

|  (ROM checksum test)

|

| Do: KAM Test

|  (check to see if KAM reset)

|

| EO_STATUS = ECU_EO_DONE

|  (Done with test)

BEGIN: Module instruction test

when called ----------------------------| (Perform module instruction test

|  per ELD specifications)

(fail module test) ---------------------| (Turn MIL on, remain in infinite

|  loop here.)

END: Module Instruction Test

32-8

KAM, RAM, AND ROM SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: RAM Test

(after instruction test) ---------------| (Looping through external RAM:

|  Save register contents,

|  Write AAAA Hex to register,

|  Read register,

|  If contents <> AAAA do ram

|   failure procedure below.

|  Otherwise,

|  Write 5555 Hex to register

|  Read register,

|  If contents <> 5555 do ram

|   failure procedure below.

|  Otherwise,

|  Restore register contents,

|  continue loop)

(ram failure) --------------------------| (Turn MIL on, remain in infinite

|  loop here)

END: RAM Test

32-9

KAM, RAM, AND ROM SELF TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ROM Test

(after register test) ------------------| (perform a 16 bit (word) addition

|  of contents of all ROM locations

|  retaining the 16 least significant

|  bits of the result)

| (Note: The parameter ROM_TO

|  contains a checksum such that the

|  sum of the ROM contents will equal

|  zero)

(If sum <> 0) --------------------------| store_code(P0605)

END: ROM Test

BEGIN: KAM Test

VIP_KAM = 1 ----------------------------| store_code(P0603)

(failed KAM qualification)

END: KAM Test

32-10

CHAPTER 33

ENGINE OFF SELF TEST

33-1

ENGINE OFF SELF TEST, EDIS MODULE TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

33.1  EDIS MODULE TEST (CDAB0)

OVERVIEW

This test outputs a KOEO service code 226 if the CPU  OK  signal  (IDM  pulse

width of 100 +/- 14 usec) from the E-DIS module is not present.

DEFINITIONS

Bit Flags:

-  VKOEO_CPUOK = Flag set in IDM up foreground that indicates a CPU ok pulse

from the E-DIS module has been received.

Calibration Constants:

-  V_CPUOK_ENA = Test enable flag; 1 = enable.

PROCESS

STRATEGY MODULE: VO_EDISMT_COM1

V_CPUOK_ENA = 1 ------------------|

|AND -| Set service code 226

VKOEO_CPUOK = 0 ------------------|

33-2

CHAPTER 34

ENGINE RUNNING SELF TEST

34-1

ENGINE RUNNING SELF TEST, TRANSMISSION CONTROL SWITCH TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

34.1  TRANSMISSION CONTROL SWITCH CIRCUIT TEST (CDAB0)

OVERVIEW

The KOER transmission control switch circuit test monitors the ITCS  register

for  both  the open and closed states.  If during the course of the KOER test

sequence both states are not recognized, a code 632  will  be  output  during

KOER  code  output.  This test requires the operator to depress the overdrive

cancel switch anytime during the KOER test sequence.

DEFINITIONS

Bit Flags:

-  ITCS = Input register indicate current state of TCS, 1 = closed.

-  TCS_OPEN = Indicates open state of TCS has  been  recognized  during  the

KOER TCS test when equal to 1.

-  TCS_SHORT = Indicates closed state of TCS has been recognized during  the

KOER TCS test when equal to 1.

Calibration Constants:

-  V_TCS_ENA = TCS circuit test enable switch; 1 -> enable.

PROCESS

STRATEGY MODULE: VR_TCS_COM2

During KOER initialization, set code 632, TCS_SHORT = 0, TCS_OPEN = 0.

The following is performed in SEQUENCE_2, VIP_BRAKE_TEST (once per background

loop):

ITCS = 1 -------------------------------| TCS_SHORT = 1

|

| --- ELSE ---

|

| TCS_OPEN = 1

The following is performed in SEQUENCE_2, VIP_EXECUTIVE:

V_TCS_ENA = 0 --------------------|

|OR --| Clear Code 632

TCS_OPEN = 1 ---------------|     |

|AND -|

TCS_SHORT = 1 --------------|

34-2

CHAPTER 35

CONTINUOUS SELF TEST

35-1

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

35.1  MISFIRE DETECTION TEST (CDAN2)

OVERVIEW

The misfire detection logic  determines  that  a  cylinder  is  misfiring  by

comparing  accelerations.   To  determine  if  cylinder "x" is misfiring, the

acceleration associated with cylinder "x" is compared with the median of  the

accelerations  of  the  MEDIAN_WIDTH cylinder events surrounding cylinder "x"

where MEDIAN_WIDTH is a calibration parameter.  The acceleration of  cylinder

"x" is included in the list of accelerations used to generate the median.  If

the acceleration of cylinder "x" is sufficiently less than the  median  value

of  the MEDIAN_WIDTH accelerations mentioned above and other criteria are met

(see logic), cylinder "x" is determined to have misfired.  The  threshold  of

the  difference  between  the  median  acceleration  and  the acceleration of

cylinder "x" for determining if cylinder "x" has misfired is  a  function  of

the  indicated  torque  of  the  engine  at the time cylinder "x" was to have

fired.

The misfire detection test logic is divided into two main parts.   The  first

is  performed during the PIP interrupt routine (PIP down if accelerations are

calculated from PIP and the PIP up edge  if  the  rear  "misfire  sensor"  is

used).   This  is  where  the  accelerations  are calculated and MEDIAN_WIDTH

accelerations stored.  The indicated torques associated  with  the  cylinders

("x")  to  be  analysed  are  also  calculated  and  stored.  Two firing data

registers are maintained for each  cylinder  during  the  PIP  routine.   One

indicating  the  number  of  total  firings  for that cylinder since the last

misfire test background logic execution and the other indicating  the  number

of  misfires  for  that cylinder since the last misfire test background logic

execution.  The second part is  performed  during  the  background  sequence.

Here,  conditions  are  checked  to  determine if assessment of data gathered

during the PIP interrupts since the last background may be used to  determine

if  misfires  have  taken  place.   Misfire  rates are maintained and the two

firing data registers for each cylinder are cleared  for  tallying  the  next

series   of  cylinder  events.   Also,  OBDII  requirements  such  as  "trip"

indicators for the misfire detection test are controlled.

This version of the Misfire  Detection  test  is  capable  of  using  profile

correction factors.  Profile refers to the crank shaft angle comprising a pip

period.  For any engine using this logic the  nominal  angle  is  360/ENGCYL.

The number of correction factors therefore is ENGCYL, one for each pip period

in  a  rotation.   The  correction  factors  are  applied  to  the   velocity

calculation  correcting  for  deviations  in  the  profile  from  the nominal

360/ENGCYL degrees.  The calibrator has the choice of  using  either  adapted

correction  factors  which are generated by the Profile Correction algorithm,

or using only the initial values of the profile correction  factors  CFx_INIT

(x refers to the 1 thru ENGCYL factors) which are calibration values.

35-2

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  ACCEL_LST[] = Array of (2 * NUMCYL - 1) registers containing pip down  to

down acceleration data.

-  ACCEL_POINTR = Indicates position of newest acceleration in ACCEL_LST [].

-  ACCEL_SAVE = Acceleration used to calculate decel.

-  ACCS_OLD = State of A/C during previous background loop.

-  ACT = Air charge temperature, degrees.

-  ATMR1 = Time since engine start.

-  BANK1_EXPLIC = Misfire rate on bank 1, updates occur each  200  rotations

of the engine.

-  BANK1_EXPON = Misfire rate on bank 1, calculated on a rolling ave basis.

-  BANK2_EXPLIC = Misfire rate on bank 2, updates occur each  200  rotations

of the engine.

-  BANK2_EXPON = Misfire rate on bank 2, calculated on a rolling ave basis.

-  BG_TMR = Background loop timer.

-  CF1 = adapted correction factor for profile # 1.

-  CF2 = adapted correction factor for profile # 2.

-  CF3 = adapted correction factor for profile # 3.

-  CF4 = adapted correction factor for profile # 4.

-  CMISLEVEL = Misfire test individual cylinder failure threshold.

-  CMIS1FIL = Misfire test cylinder 1 rolling average fault filter.

-  CMIS2FIL = Misfire test cylinder 2 rolling average fault filter.

-  CMIS3FIL = Misfire test cylinder 3 rolling average fault filter.

-  CMIS4FIL = Misfire test cylinder 4 rolling average fault filter.

-  CMIS5FIL = Misfire test cylinder 5 rolling average fault filter.

-  CMIS6FIL = Misfire test cylinder 6 rolling average fault filter.

-  CMIS7FIL = Misfire test cylinder 7 rolling average fault filter.

-  CMIS8FIL = Misfire test cylinder 8 rolling average fault filter.

-  CSUMFIL = Sum of individual cylinder misfire fault filters.

35-3

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  DACCEL_36 = Result of MEDIAN - a(0) in misfire test.

-  DACCEL_HGHT = Expected DACCEL_36 level under misfire.

-  ECT = Engine coolant temp.

-  FN1615_DATA = Misfire test current net torque.

Range = 0 to 1022.984375; Resolution = 0.015625;Units = LB-FT

-  FNMISOK_DATA = current value from FNMISOK table.

-  INJ_ON = Number of injectors to  be  left  on  as  determined  by  torque

control.

|      -  ISCFLG = ISC mode indicator flag;  -1  ->  Dashpot  Mode,  0  ->  Dashpot

|         Preposition Mode, 1 -> closed Loop RPM Control Mode, 2 -> Closed Loop RPM

|         Control (Lock_out entry to RPM control).

-  LOAD = Universal LOAD as ratio of air charge over standard.

-  MAX_PCT_MIS = Contains most recent value from the table FNMAXISPCT.

-  MISBYPSCTR = Cumulative cylinder events  bypassed  by  the  misfire  test

foreground logic.

-  MIS_CF = current profile correction factor is assigned to  this  register

which is then applied to the velocity.

-  MISECTDLY = Number of seconds to delay Misfire monitoring based  on  ECT,

after power-up.

-  MISFDELTAERR = Misfire Comm.  error indicator.

-  MISACTDLY = Number of seconds to delay Misfire monitoring based  on  ACT,

after power-up.

-  MIS_PROFILE = profile associated with current pip period in  the  misfire

test.

-  MIS_SYNC_CTR = Calculated value of sync_ctr used by misfire detection.

-  MISFIRES1000 = Accumulated misfires over 1000 revolutions.

-  MISFIRES200 = Accumulated misfires over 200 revolutions.

-  MISNOCALLFIL = 1st Order filter of nocall by the misfire detection logic.

-  MIS_EVENTS = Cumulative foreground executions of  the  misfire  detection

test.

-  MIS_NOCALL = Cumulative instances of a no-call by the misfire test.

-  MISPOTCALL = Cumulative instances of potential failures  in  the  misfire

test.

-  MISTOTDLY = Total amount of time, in seconds,  misfire  monitor  will  be

bypassed after power-up.

35-4

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  N = Engine speed, RPM.

-  NOISE_COUNT = When non-zero no assessment of misfire due to signal noise.

-  NUMMIS = Cumulative instances of misfire detected by the misfire test.

-  NUMMIS1 = Cumulative instances of misfire detected by  the  misfire  test

from cylinder 1.

-  NUMMIS2 = Cumulative instances of misfire detected by  the  misfire  test

from cylinder 2.

-  NUMMIS3 = Cumulative instances of misfire detected by  the  misfire  test

from cylinder 3.

-  NUMMIS4 = Cumulative instances of misfire detected by  the  misfire  test

from cylinder 4.

-  NUMMIS5 = Cumulative instances of misfire detected by  the  misfire  test

from cylinder 5.

-  NUMMIS6 = Cumulative instances of misfire detected by  the  misfire  test

from cylinder 6.

-  NUMMIS7 = Cumulative instances of misfire detected by  the  misfire  test

from cylinder 7.

-  NUMMIS8 = Cumulative instances of misfire detected by  the  misfire  test

from cylinder 8.

-  PIP_DWN_DEL = Period between two most recent PIP down edges.

-  PIPTSTFIL = PIP circuit failure fault filter.

-  SPK_LAMBSE = Value of LAMBSE to be used in spark calculation.

-  SYNC_CTR = Sequence counter incremented every PIP  rising  edge  that  is

used to identify the cycle and position.

-  TRQLST[TQ_POINTER] = NUMCYL  registers  containing  indicated  torque  at

associated pip down-to-down.

-  TQ_BRAKE_S = Actual net torque produced by engine.

-  TR_SPK_DELTA = current torque ration from spark advance

-  TOTBGTOBG = Total cylinder events between misfire test  background  logic

executions.

-  TOTEVENTS = Cumulative cylinder events total carried over  from  previous

background logic executions.

-  TOTMISBGTOBG = Total number of misfires between misfire  test  background

logic executions.

-  TOTMISFIL = Overall misfire fault rolling average fault filter .

35-5

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  TOTMISLEVEL = Misfire test overall failure threshold for catalyst damage.

-  TOTNOCALLOLD =  Cumulative  nocalls  total  carries  over  from  previous

background logic execution.

-  TQ_POINTER = Indicates position of oldest torque in list.

-  TYPE_A_MIS = Indicates to executive which misfire criteria.

-  USPD_RUN_TMR = Time in underspeed or run mode.

-  V_CYL1_MIS = Number of misfires of cylinder 1  since  last  misfire  test

background.

-  V_CYL1_TOT =  Number  of  cylinder  1  events  since  last  misfire  test

background.

-  V_CYL2_MIS = Number of misfires of cylinder 2  since  last  misfire  test

background.

-  V_CYL2_TOT =  Number  of  cylinder  2  events  since  last  misfire  test

background.

-  V_CYL3_MIS = Number of misfires of cylinder 3  since  last  misfire  test

background.

-  V_CYL3_TOT =  Number  of  cylinder  3  events  since  last  misfire  test

background.

-  V_CYL4_MIS = Number of misfires of cylinder 4  since  last  misfire  test

background.

-  V_CYL4_TOT =  Number  of  cylinder  4  events  since  last  misfire  test

background.

-  V_CYL5_MIS = Number of misfires of cylinder 5  since  last  misfire  test

background.

-  V_CYL5_TOT =  Number  of  cylinder  5  events  since  last  misfire  test

background.

-  V_CYL6_MIS = Number of misfires of cylinder 6  since  last  misfire  test

background.

-  V_CYL6_TOT =  Number  of  cylinder  6  events  since  last  misfire  test

background.

-  V_CYL7_MIS = Number of misfires of cylinder 7  since  last  misfire  test

background.

-  V_CYL7_TOT =  Number  of  cylinder  7  events  since  last  misfire  test

background.

-  V_CYL8_MIS = Number of misfires of cylinder 8  since  last  misfire  test

background.

-  V_CYL8_TOT =  Number  of  cylinder  8  events  since  last  misfire  test

background.

35-6

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  V_LST_DPT =  Saved  value  of  angular  velocity  for  next  acceleration

calculation.

-  V_MIS1000_1 = Total misfires of cylinder 1 for 1000 rotations criteria in

misfire test.

-  V_MIS1000_2 = Total misfires of cylinder 2 for 1000 rotations criteria in

misfire test.

-  V_MIS1000_3 = Total misfires of cylinder 3 for 1000 rotations criteria in

misfire test.

-  V_MIS1000_4 = Total misfires of cylinder 4 for 1000 rotations criteria in

misfire test.

-  V_MIS1000_5 = Total misfires of cylinder 5 for 1000 rotations criteria in

misfire test.

-  V_MIS1000_6 = Total misfires of cylinder 6 for 1000 rotations criteria in

misfire test.

-  V_MIS1000_7 = Total misfires of cylinder 7 for 1000 rotations criteria in

misfire test.

-  V_MIS1000_8 = Total misfires of cylinder 8 for 1000 rotations criteria in

misfire test.

-  V_MIS200_1 = Total misfires of cylinder 1 for 200 rotations  criteria  in

misfire test.

-  V_MIS200_2 = Total misfires of cylinder 2 for 200 rotations  criteria  in

misfire test.

-  V_MIS200_3 = Total misfires of cylinder 3 for 200 rotations  criteria  in

misfire test.

-  V_MIS200_4 = Total misfires of cylinder 4 for 200 rotations  criteria  in

misfire test.

-  V_MIS200_5 = Total misfires of cylinder 5 for 200 rotations  criteria  in

misfire test.

-  V_MIS200_6 = Total misfires of cylinder 6 for 200 rotations  criteria  in

misfire test.

-  V_MIS200_7 = Total misfires of cylinder 7 for 200 rotations  criteria  in

misfire test.

-  V_MIS200_8 = Total misfires of cylinder 8 for 200 rotations  criteria  in

misfire test.

-  V_MIS_LOOPS = Number of pip periods remaining before data is valid.

-  V_MIS_THR1 = Calculated threshold above which DACCEL_36 is  considered  a

misfire.

-  V_MIS_THR2 = Calculated threshold below  which  DACCEL_36  is  considered

noise.

35-7

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  V_MIS_TSTCNT =  Register  containing  number  of  misfire  test  testable

events.

-  VMIS_TOT1000 = Total cylinder  events  for  1000  rotations  criteria  in

misfire test.

-  VMIS_TOT200 = Total  cylinder  events  for  200  roatations  criteria  in

misfire test

Bit Flags:

-  ACCFLG = A/C clutch demand status; 1 -> A/C clutch commanded on.

-  CF_KAM_MTR = 1 -> KAM contains mature correction factors.

-  CF_KAM_MTRO = Value of CF_KAM_MTR during the last  bg  loop.   Initial(on

power-up) value is equal to CF_KAM_MTR.

-  DIS_FMEM = 1 -> FMEM action in progress for coil/misfire malfunction.

-  FFG_ECT = when one indicated ect currently not reliable.

-  FFG_MISFIRE = OBDII system FMEM flag form engine misfire; 1 ->  currently

misfiring.

-  MIL_200_1 = Misfire on cylinder 1, catalyst temp criteria, for MIL

-  MIL_200_2 = Misfire on cylinder 2, catalyst temp criteria, for MIL

-  MIL_200_3 = Misfire on cylinder 3, catalyst temp criteria, for MIL

-  MIL_200_4 = Misfire on cylinder 4, catalyst temp criteria, for MIL

-  MIL_200_5 = Misfire on cylinder 5, catalyst temp criteria, for MIL

-  MIL_200_6 = Misfire on cylinder 6, catalyst temp criteria, for MIL

-  MIL_200_7 = Misfire on cylinder 7, catalyst temp criteria, for MIL

-  MIL_200_8 = Misfire on cylinder 8, catalyst temp criteria, for MIL

-  MIL_1000_1 = Misfire on cylinder 1, emissions criteria.

-  MIL_1000_2 = Misfire on cylinder 2, emissions criteria.

-  MIL_1000_3 = Misfire on cylinder 3, emissions criteria.

-  MIL_1000_4 = Misfire on cylinder 4, emissions criteria.

-  MIL_1000_5 = Misfire on cylinder 5, emissions criteria.

-  MIL_1000_6 = Misfire on cylinder 6, emissions criteria.

-  MIL_1000_7 = Misfire on cylinder 7, emissions criteria.

-  MIL_1000_8 = Misfire on cylinder 8, emissions criteria.

35-8

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  MIS200FLG_1 = Misfire on cylinder 1, catalyst temperature criteria.

-  MIS200FLG_2 = Misfire on cylinder 2, catalyst temperature criteria.

-  MIS200FLG_3 = Misfire on cylinder 3, catalyst temperature criteria.

-  MIS200FLG_4 = Misfire on cylinder 4, catalyst temperature criteria.

-  MIS200FLG_5 = Misfire on cylinder 5, catalyst temperature criteria.

-  MIS200FLG_6 = Misfire on cylinder 6, catalyst temperature criteria.

-  MIS200FLG_7 = Misfire on cylinder 7, catalyst temperature criteria.

-  MIS200FLG_8 = Misfire on cylinder 8, catalyst temperature criteria.

-  MISBG_BYPS = Misfire test flag indicating conditions  present  to  bypass

test.

-  MISDLYFLG = When set (=1), indicates that MISTOTDLY has been calculated.

-  MIS_ENA_FRST = indicated first background of  misfire  after  enable  has

taken place.

-  MIS_ENA_OLD = Misfire monitor; previous BG value of MIS_TST_ENA.

-  MIS_ER_ENA = Flag enables misfire test during KOER on demand testing.

-  MIS_FCO_FLG = Flag indicating FMEM action is being taken  during  type  A

misfire.

-  MISFIRING = Flag indicates when a misfire malfunction is being detected.

-  MIS_MON = 1 -> test performed sufficient to recognize a failure.

-  MIS_MON_KAM = Same as MIS_MON but stored in KAM.

-  MIS_TST_ENA = OBD=II misfire test enable flag ( 1=> test enabled).

-  MIS_TST_RDY = When equal to 1 indicated misfire test is available.

-  POTN_FAIL[] = Array of flags holding  preliminary  misfire  status  of  a

cylinder event.

-  PxxxFAULT = Service code xxx KAM bit.

-  VPIPFLG = 1 -> indicates an intermittant PIP with DP.

-  V_SYNCFLG = 1 -> cylinder determination possible.

Calibration Constants:

-  AICE_SETR1_B = AICE setup reg.  "B" value.

-  ALPHAMIS =  Filter  constant  for  misfire  test  rolling  average  fault

filters.

35-9

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  ALPHANOCALL = Filter constant for misfire test MISNOCALLFIL filter.

-  BYPS_BRKPNT = Threshold of value returned from FNMISOK  for  decision  to

test or not for misfire.

-  CF1_INIT = initial profile correction factor for profile #1.

-  CF2_INIT = initial profile correction factor for profile #2.

-  CF3_INIT = initial profile correction factor for profile #3.

-  CF4_INIT = initial profile correction factor for profile #4.

-  CF_KAM_IGNR = 1 -> misfire logic ignores the state of CF_KAM_MTR flag.

-  DACCEL_CALIB = Adjusts DACCEL_HGHT to the  level  of  expected  DACCEL_36

under misfire.

-  DACCEL_THRES = Percent of DACCEL_HGHT above  which  DACCEL_36's  will  be

considered misfires.

-  ENGCYL = Number of PIP's  per  engine  revolution  2  ->  4  cyl  eng  (2

PIP/REV), 3 -> 6 cyl eng (e PIP/REV).

-  FAST_MIS_LVL = Selects method of updating misfire malfunction  threshold:

0 -> always filter, 1 -> filter up, jump down.

-  FN1327(injector_out, null) = A one row (9 x 1) table that maps  'injector

number' to a numerical indication of its associated engine bank.

-  FN1329(INJ_BANK, NULL) = An 8x1 table that translates the sequence number

into the correct cylinder number.

-  FN1615A(FN070C, FN034A) = Misfire table  to  represent  misfire  specific

indicated  engine  torque,  X  =  normalized  engine  speed  in  RPM, Y =

normalized load.

Range = 0 to 510;Resolution = 2; Units = LB-FT

-  FN1615M(FN070M, FN034M) = Misfire table  to  represent  misfire  specific

indicated  engine  torque,  X  =  normalized  engine  speed  in  RPM, Y =

normalized load.

Range = 0 to 510;Resolution = 2; Units = LB-FT

-  FN1615M_MULT(FN070M, FN034M) = Misfire multiplier table, allows the  user

to increase the resolution of FN1615_DATA.

Range = 0 to 3.984375; Resolution = 0.015625;Units = Unitless

-  FN1615M_CAL  :=  When  set   to   1,   misfire-specific   torque   table,

FN1615M(N,LOAD),  and  the  mulitplication  table  FN1615M_MULT are used.

When set to 0, FN1615A(N,LOAD) torque table is used.

-  FN623(SPK_LAMBSE) = Fuel multiplier used in torque calculation.

-  FNMAXMISPCT(FN070C,FN034A)  =  Maximum  misfire   rate   allowed   before

malfunction.

-  FNMISECT(ECT) = A function, it takes ECT temp as input and returns  time,

seconds.

35-10

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FNMISACT(ACT) = A function, it takes ACT temp as input and returns  time,

seconds.

-  FNMISOK(FN070M, FN034M) = Table of N,  LOAD  regions  where  testing  for

misfire allowed, 1 = testing allowed.

-  MEDIAN_WIDTH = In misfire test, number of events used to calculate median

acceleration.

-  MIS_ECTMAX = maximum ECT to activate misfire detection monitor.

-  MIS_ECTMIN = minimum ECT to activate misfire detection monitor.

-  MISLVLEXP = Misfire rate threshold for control of FFG_MISFIRE.

-  MIS_AC_SW = 1 -> bypass misfire test for A/C cycling.

-  MIS_CAT_CONF = Catalyst configuration for misfire detection purposes.   0

->  both  cylinder  banks  into  one  catalyst;  1  ->  one bank into one

catalyst.

-  MIS_FIL_SEL = Select filtering method for misfire test;  0  ->  geometric

rolling average, 1 -> blocks of rotations.

|      -  MIS_ISCF_SW = Switch which  enables  the  Idle  Speed  Control  indicator

|         (ISCFLG)  to be included in the misfire test ready logic.  If this switch

|         is set to 1, then misfire will be disabled during  a  tip  out  condition

|         when  ISCFLG  =  -1 (dashpot mode).  Set this switch to 0 to disable this

|         option.

-  MIS_STRT_DLY = Minimum time  since  exiting  crank  mode  before  misfire

monitor will be ready.

-  MIS_MIN_TQ = Minimum net torque to allow testing for misfire.

-  MIS_USE_CF = in  misfire  test  when  equal  to  1  use  adapted  profile

correction  factors,  when  0  use  initial  values of profile correction

factors.

-  MISLEVEL1000 = Percent misfire threshold for emissions.

-  MISLEVELSMLR = Percent misfire threshold for a similar conditions check.

-  MISTHRESPCT =  Multiplier  (normally  <  1)  for  lower  (noise)  deviant

acceleration threshold.

-  NUMCYL = Number of cylinders in the engine.

-  VMIS_NOISBND = Number of firing events between a noisy event and an event

that may be evaluated.

-  V_MIS_BGMAX = BG  loop  time  above  which  misfire  test  foreground  is

bypassed for chronometrics.

-  V_MIS_MONCNT = Number of firing events consecutively tested to qualify as

a monitoring of misfire.

35-11

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  V_MIS_NMAX = RPM above which misfire test background is bypassed.

-  V_MIS_NMIN = RPM below which misfire test backround is bypassed.

OTHER

-  mis_codes = Set of (P0300, P0301,  P0302,  P0303,  P0304,  P0305,  P0306,

P0307, P0308).

-  P0300 = Misfire failure fault code, more than one  cylinder  or  cylinder

cannot be determined due to lack of syncronization.

-  P0301 = Cylinder 1 misfire failure fault code.

-  P0302 = Cylinder 2 misfire failure fault code.

-  P0303 = Cylinder 3 misfire failure fault code.

-  P0304 = Cylinder 4 misfire failure fault code.

-  P0305 = Cylinder 5 misfire failure fault code.

-  P0306 = Cylinder 6 misfire failure fault code.

-  P0307 = Cylinder 7 misfire failure fault code.

-  P0308 = Cylinder 8 misfire failure fault code.

35-12

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: VC_MISFIRE_DETEC_COM4

BEGIN: MISFIRE_DETECTION_FOREGROUND_MAIN

(performed during PIP interrupt as follows:

1. if the cal parameter AICE_SETR1_B (bit3) = 1, PIP input is used to

calculate accelerations, perform "MISFIRE_DETECTION_FOREGROUND_MAIN" during

the PIP down interrupt after update of EEC registers containing AICE data.

2. if the cal parameter AICE_SETR1_B (bit3) = 0, rear "misfire sensor" input

is used to calculate accelerations, perform

"MISFIRE_DETECTION_FOREGROUND_MAIN" during the PIP up interrupt, prior to

update of SYNC_CTR and prior to the update of the EEC registers that contain

AICE data.)

AICE_SETR1_B (bit3) = 1 --| perform "MISFIRE_DETECTION_FOREGROUND_MAIN"

| during the PIP down interrupt after update

| of EEC registers containing AICE data

|

| --- ELSE ---

|

| perform "MISFIRE_DETECTION_FOREGROUND_MAIN"

| during the PIP up interrupt, prior to

| update of SYNC_CTR and prior to the update of

| the EEC registers that contain AICE data

35-13

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

This is the main misfire detection  logic  procedure  contained  in  the  PIP

routine.  All other misfire detection procedures contained in the PIP routine

are either called  by  MISFIRE_DETECTION_FOREGROUND_MAIN  or  are  called  by

procedures called by MISFIRE_DETECTION_FOREGROUND_MAIN.

MISBG_BYPS = 1 --| MISBYPSCTR := MISBYPSCTR + 1

(background is  |  (for development only)

indicating to  |

bypass test)   |

| --- ELSE ---

|

V_MIS_LOOPS = 0 -| Do: MIS_SYNC_CTR_CALC

(accel_lst[]    | Do: ACCEL_CALC

and            | Do: MAINTAIN_ACCEL_LST

trqlst[]       | Do: MAINTAIN_TORQUE_LIST

filled with    | Do: MAINTAIN_POTN_FAIL_FLG_QUE

good data,     | Do: CALCULATE_MEDIAN_ACCELERATION

however        | Do: POINTER_CONTROL

potn_fail[0]   | MIS_EVENTS := MIS_EVENTS + 1

cannot be      |  (for development only)

considered     | DACCEL_HGHT := DACCEL_CALIB * TRQLST[TQ_POINTER]

to contain     |  (expected daccel level under misfire)

valid data     | V_MIS_THR1 := DACCEL_HGHT * DACCEL_THRES

until          |  (deviant accels above this value

noise_level=0) |   are potential misfires)

| V_MIS_THR2 := V_MIS_THR1 * MISTHRESPCT * -1

|   (deviant accels below this value are noise)

| DACCEL_36 := accel_stat - ACCEL_LST[a0]

|   (deviant acceleration)

| ACCEL_SAVE := ACCEL_LST[a0]

|  (accel used to calculate daccel)

| Do: MIS_DETECTION

|

| --- ELSE ---

|

| Do: MIS_SYNC_CTR_CALC

| Do: ACCEL_CALC

| Do: MAINTAIN_ACCEL_LST

| Do: MAINTAIN_TORQUE_LIST

| Do: MAINTAIN_POTN_FAIL_FLG_QUE

| V_MIS_LOOPS := V_MIS_LOOPS - 1

|  (when decremented to zero,

|   accel_lst[] and trqlst[]

|   have valid data)

| MISBYPSCTR := MISBYSCTR + 1

|  (for development only)

END: MISFIRE_DETECTION_FOREGROUND_MAIN

35-14

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ACCEL_CALC

This procedure calculates the velocity and acceleration  of  the  PIP  period

ending at this PIP down event (or the previous PIP down event if using a rear

"misfire sensor" resulting in performance of this logic on the PIP up).

Do: DETERMINE_WHICH_PROFILE_CF_TO_USE ; what profile is generating

; this pip period

Do: GENERATE_CORRECTION_FACTOR      ; assign MIS_CF a value based on

; current profile and whether to

; use adapted CF values or base

; cal values of CFs

dpt1 := [360/(ENGCYL * PIP_DWN_DEL)] * MIS_CF

(corrected velocity)               ; angular velocity of the most

; recent pip down-to-down

; corrected for profile. (PIP_DWN_DEL

; may be generated

; by the rear "misfire sensor")

calculated acceleration := (dpt1 - V_LST_DPT)/PIP_DWN_DEL

; change in velocity from last

; pip down-to-down

; to this pip down-to-down

V_LST_DPT := dpt1               ; save current angular velocity

; for next acceleration

; calculation.

END: ACCEL_CALC

BEGIN: MAINTAIN_ACCEL_LST

ACCEL_POINTR + 1 = MEDIAN_WIDTH + 1 -| ACCEL_POINTR := 1

| ACCEL_LST[ACCEL_POINTR] :=

|  calculated_acceleration

|

| --- ELSE ---

|

| ACCEL_POINTR := ACCEL_POINTR + 1

| ACCEL_LST[ACCEL_POINTR] :=

|  calculated_acceleration

END: MAINTAIN_ACCEL_LST

35-15

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: DETERMINE_WHICH_PROFILE_CF_TO_USE

The appropriate SYNC_CTR (MIS_SYNC_CTR) comparisons are made by calling

the procedure associated with the number of cylinder.

ENGCYL = 4 -------------------| Do: DETERMINE_8CYL_CF

|

| --- ELSE ---

|

ENGCYL = 3 -------------------| Do: DETERMINE_6CYL_CF

|

| --- ELSE ---

|

ENGCYL = 2 -------------------| Do: DETERMINE_4CYL_CF

END: DETERMINE_WHICH_PROFILE_CF_TO_USE

BEGIN: MIS_SYNC_CTR_CALC

If using the rear misfire sensor (AICE_SET1_B(bit3) = 0) then the current

pip down to down is associated with the current value of SYNC_CTR - 1.

AICE_SET1_B (bit3) = 1 ---| MIS_SYNC_CTR := SYNC_CTR

|  (using front (PIP) sensor)

|

| --- ELSE ---

|

| MIS_SYNC_CTR := SYNC_CTR - 1

| Do: ADJUST_MIS_SYNC_CTR

(using rear misfire sensor)

END: MIS_SYNC_CTR_CALC

BEGIN: ADJUST_MIS_SYNC_CTR

ENGCYL = 4 ---------|

|AND -| MIS_SYNC_CTR := 7

MIS_SYNC_CTR = -1 --|     |

| --- ELSE ---

ENGCYL = 3 ---------|     |

|AND -| MIS_SYNC_CTR := 5

MIS_SYNC_CTR = -1 --|     |

| --- ELSE ---

ENGCYL = 2 ---------|     |

|AND -| MIS_SYNC_CTR := 3

MIS_SYNC_CTR = -1 --|

END: ADJUST_MIS_SYNC_CTR

35-16

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: DETERMINE_8CYL_CF

MIS_SYNC_CTR is used to determine which profile applies during this

interrupt for 8 cylinder engines.

MIS_SYNC_CTR = 3 --|

|OR --| MIS_PROFILE := 4

MIS_SYNC_CTR = 7 --|     |

| --- ELSE ---

MIS_SYNC_CTR = 0 --|     |

|OR --| MIS_PROFILE := 1

MIS_SYNC_CTR = 4 --|     |

| --- ELSE ---

MIS_SYNC_CTR = 1 --|     |

|OR --| MIS_PROFILE := 2

MIS_SYNC_CTR = 5 --|     |

| --- ELSE ---

MIS_SYNC_CTR = 2 --|     |

|OR --| MIS_PROFILE := 3

MIS_SYNC_CTR = 6 --|

END: DETERMINE_8CYL_CF

BEGIN: DETERMINE_6CYL_CF

MIS_SYNC_CTR is used to determine which profile applies during this

interrupt for 6 cylinder engines

MIS_SYNC_CTR = 3 --|

|OR --| MIS_PROFILE := 1

MIS_SYNC_CTR = 0 --|     |

| --- ELSE ---

MIS_SYNC_CTR = 1 --|     |

|OR --| MIS_PROFILE := 2

MIS_SYNC_CTR = 4 --|     |

| --- ELSE ---

MIS_SYNC_CTR = 2 --|     |

|OR --| MIS_PROFILE := 3

MIS_SYNC_CTR = 5 --|

END: DETERMINE_6CYL_CF

BEGIN: DETERMINE_4CYL_CF

MIS_SYNC_CTR is used to determine which profile applies during this

interrupt for 4 cylinder engines.

MIS_SYNC_CTR = 1 --|

|OR --| MIS_PROFILE := 2

MIS_SYNC_CTR = 3 --|     |

| --- ELSE ---

MIS_SYNC_CTR = 2 --|     |

|OR --| MIS_PROFILE := 1

MIS_SYNC_CTR = 0 --|

END: DETERMINE_4CYL_CF

35-17

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: GENERATE_CORRECTION_FACTOR

Assign MIS_CF a value based on current profile and (via MIS_USE_CF)

whether to use adapted CF values or base cal values of CFs.

MIS_PROFILE = 1 -|

|AND -| MIS_CF := CF1

MIS_USE_CF = 1 --|     |

(use adapted CFs)     |

| --- ELSE ---

|

MIS_PROFILE = 1 -------| MIS_CF := CF1_INIT

|  (uncorrected or best guess cal

|   of what profile 1 CF should be)

|

| --- ELSE ---

MIS_PROFILE = 2 -|     |

|AND -| MIS_CF := CF2

MIS_USE_CF = 1 --|     |

(use adapted CFs)     |

| --- ELSE ---

|

MIS_PROFILE = 2 -------| MIS_CF := CF2_INIT

|  (uncorrected or best guess cal

|   of what profile 2 CF should be)

|

| --- ELSE ---

MIS_PROFILE = 3 -|     |

|AND -| MIS_CF := CF3

MIS_USE_CF = 1 --|     |

(use adapted CFs)     |

| --- ELSE ---

|

MIS_PROFILE = 3 -------| MIS_CF := CF3_INIT

|  (uncorrected or best guess cal

|   of what profile 3 CF should be)

|

| --- ELSE ---

MIS_PROFILE = 4 -|     |

|AND -| MIS_CF := CF4

MIS_USE_CF = 1 --|     |

(use adapted CFs)     |

| --- ELSE ---

|

MIS_PROFILE = 4 -------| MIS_CF := CF4_INIT

|  (uncorrected or best guess cal

|   of what profile 4 CF should be)

END: GENERATE_CORRECTION_FACTOR

35-18

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: MAINTAIN_TORQUE_LIST

;assign current value of torque to list.  At this point

;TQ_POINTER is pointing to the oldest member of the list,

;which will be written over and then will be pointing to

;the newest member of the list.

unconditionally -----------| TRQLST[TQ_POINTER] := FN1615_DATA

In the following logic, TQ_POINTER is incremented and now will once again

point to the oldest member of the list.  This is the member of the list

which will be used to calculate DACCEL_HGHT during this interrupt.

TQ_POINTER + 1  = (MEDIAN_WIDTH + 1) / 2 -| TQ_POINTER := 0

(list rollover)                          |

| --- ELSE ---

|

| TQ_POINTER := TQ_POINTER + 1

END: MAINTAIN_TORQUE_LIST

35-19

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: MAINTAIN_POTN_FAIL_FLG_QUE

Shift the POTN_FAIL[] array losing the oldest value in preparation for

adding the most recent value.

b0 := VMIS_NOISBND + 1        ; b0 is index of most current

POTN_FAIL[] element

POTN_FAIL[0] := POTN_FAIL[1]

POTN_FAIL[1] := POTN_FAIL[2]

POTN_FAIL[2] := POTN_FAIL[3]

.

.

.

POTN_FAIL[b0 - 1] := POTN_FAIL[b0]

END: MAINTAIN_POTN_FAIL_FLG_QUE

BEGIN: CALCULATE_MEDIAN_ACCELERATION

Set accel_sort[] array := ACCEL_LST[] array

Sort accel_sort[] array (1 to MEDIAN_WIDTH)

;find the median of accel_sort[] and assign it to accel_stat

accel_stat := accel_sort [(MEDIAN_WIDTH + 1)/2]

END: CALCULATE_MEDIAN_ACCELERATION

BEGIN: POINTER_CONTROL

a0 := ACCEL_POINTR + (MEDIAN_WIDTH + 1)/2

a0 >= MEDIAN_WIDTH + 1 ---| a0 := a0 - MEDIAN_WIDTH

END: POINTER_CONTROL

35-20

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: MIS_DETECTION

Firing events pointed to by  ACCEL_LST[a0]  are  assessed  for  misfire,  the

result  is held in the POTN_FAIL[] array until possible noise surrounding the

event  can  be  determined.   If  noise  exists  in  events  near  the  event

represented  by  POTN_FAIL  elements,  a  misfire  indicated by the POTN_FAIL

element will be disregarded as due to noise.

DACCEL_36 > V_MIS_THR1 ------------------| POTN_FAIL[b0] := 1

(potential misfire recognized, if no    | MISPOTCALL := MISPOTCALL + 1

noise appears in the VMIS_NOISBND      |  (for development only)

firings on either side of the event    | Do: MIS_EVENTS_TALLY

pointed to by accel_lst[a0],           |

this event will become classified      |

as an actual misfire)                  |

| --- ELSE ---

|

DACCEL_36 < V_MIS_THR2 ------------------| POTN_FAIL[b0] := 0

(noisy signal has been recognized,     | NOISE_COUNT := VMIS_NOISBND *

NOISE_COUNT register reset to protect |  2 + 1

against wrongfully calling            | MIS_NOCALL := MIS_NOCALL + 1

neighboring cylinder events misfires) |  (for development only)

|

| --- ELSE ---

|

| POTN_FAIL[b0] := 0

| Do: MIS_EVENTS_TALLY

END: MIS_DETECTION

35-21

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: MIS_EVENTS_TALLY

Final determination of misfire is made here.

NOISE_COUNT = 0 -----------| Do: DETERMINE_CYLINDER_BEING_EVALUATED

(no noise)                | Do:  POTNFAIL_TO_ACTUALFAIL

|  (potn_fail flag for next cylinder to be

|   evaluated is checked)

| Do: INC_EVENTS_CNTR

|  (document that an event for the cylinder

|    determined by "determine_cylinder_being_

|     evaluated" has taken place)

| V_MIS_TSTCNT := V_MIS_TSTCNT + 1

|  (tally consecutive firing events with

|   test active)

|

| --- ELSE ---

|

| MIS_NOCALL := MIS_NOCALL + 1

|  (for development only)

| NOISE_COUNT := NOISE_COUNT - 1

|  (noisy environment is subsiding)

END: MIS_EVENTS_TALLY

35-22

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: POTNFAIL_TO_ACTUALFAIL

POTN_FAIL[0] = 1 ----------| Do: INC_MIS_CNTR

|  (document a misfire for cylinder determined

|   by "determine_cylinder_being_evaluated")

| NUMMIS := NUMMIS + 1

|  (for development only)

END: POTNFAIL_TO_ACTUALFAIL

BEGIN: DETERMINE_CYLINDER_BEING_EVALUATED

med_dist := (MEDIAN_WIDTH - 1) / 2

nois_dist := VMIS_NOISBND + 1

temp := MIS_SYNC_CTR - med_dist - nois_dist

(offset between current value of MIS_SYNC_CTR and

MIS_SYNC_CTR of the cylinder under evaluation)

Do: MODULO_ARITHMETIC

Do: CONVERT_SYNC_CTR_TO_CYLINDER_NUMBER

END: DETERMINE_CYLINDER_BEING_EVALUATED

BEGIN: MODULO_ARITHMETIC

At the end of this process, temp will contain the MIS_SYNC_CTR number

for the cylinder being evaluated (on 8 cylinder engines temp contains

MIS_SYNC_CTR + 1).

temp <= 0 -------------| temp := temp + NUMCYL

| Do: MODULO_ARITHMETIC ; loop

END: MODULO_ARITHMETIC

BEGIN: CONVERT_SYNC_CTR_TO_CYLINDER_NUMBER

temp := temp - 1 ; input to FN1329 requires value 0 thru 7

temp := FN1329(temp)

END: CONVERT_SYNC_CTR_TO_CYLINDER_NUMBER

35-23

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: INC_MIS_CNTR

The counters associated with the misfiring cylinder are incremented.

temp = 1 -----------| V_CYL1_MIS := V_CYL1_MIS + 1

|

| --- ELSE ---

|

temp = 2 -----------| V_CYL2_MIS := V_CYL2_MIS + 1

|

| --- ELSE ---

|

temp = 3 -----------| V_CYL3_MIS := V_CYL3_MIS + 1

|

| --- ELSE ---

|

temp = 4 -----------| V_CYL4_MIS := V_CYL4_MIS + 1

|

| --- ELSE ---

|

temp = 5 -----------| V_CYL5_MIS := V_CYL5_MIS + 1

|

| --- ELSE ---

|

temp = 6 -----------| V_CYL6_MIS := V_CYL6_MIS + 1

|

| --- ELSE ---

|

temp = 7 -----------| V_CYL7_MIS := V_CYL7_MIS + 1

|

| --- ELSE ---

|

temp = 8 -----------| V_CYL8_MIS := V_CYL8_MIS + 1

END: INC_MIS_CNTR

35-24

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: INC_EVENTS_CNTR

The counter associated with the current cylinder, when valid

test conditions are present,is incremented.

temp = 1 -----------| V_CYL1_TOT := V_CYL1_TOT + 1

|

| --- ELSE ---

|

temp = 2 -----------| V_CYL2_TOT := V_CYL1_TOT + 1

|

| --- ELSE ---

|

temp = 3 -----------| V_CYL3_TOT := V_CYL3_TOT + 1

|

| --- ELSE ---

|

temp = 4 -----------| V_CYL4_TOT := V_CYL4_TOT + 1

|

| --- ELSE ---

|

temp = 5 -----------| V_CYL5_TOT := V_CYL5_TOT + 1

|

| --- ELSE ---

|

temp = 6 -----------| V_CYL6_TOT := V_CYL6_TOT + 1

|

| --- ELSE ---

|

temp = 7 -----------| V_CYL7_TOT := V_CYL7_TOT + 1

|

| --- ELSE ---

|

temp = 8 -----------| V_CYL8_TOT := V_CYL8_TOT + 1

END: INC_EVENTS_CNTR

end pip period processing

35-25

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: MISFIRE BACKGROUND MAIN

;execute once per bg loop

unconditionally ----| Do: CLEAR_MISFIRE_KAM_REGS

| Do: SYNC_CHK

| ;clear individual cylinder regs if not in sync.

| Do: ACCESSORY_CHECK

| ;accessory state changes that may appear as

| ;misfires are detected

| Do: MISFIRE_ENA_FLG_TRANSITION_CHK

| ;information for control of misfire ready

| ;flag based on transition of the OBDII

| ;executive misfire enable flag

| Do: BYPASS_TABLE

| Do: MIS_START_DLYTM_CALC

| ;calc time to delay misfire monitor

| Do: MIS_TST_READY_CHK

| ;indicate whether misfire test may be performed

| Do: MISBG_BYPS

| ;perform misfire testing if possible

END: MISFIRE BACKGROUND MAIN

BEGIN: CLEAR_MISFIRE_KAM_REGS

MIS_TST_ENA = 1 -----------|

|AND -|     | NUMMIS1 := 0

MIS_ENA_FRST = 0 ----------|     |     | NUMMIS2 := 0

|OR --| NUMMIS3 := 0

|     | NUMMIS4 := 0

CF_KAM_MTRO = 0 -----------|     |     | NUMMIS5 := 0

|AND -|     | NUMMIS6 := 0

CF_KAM_IGNR = 0 -----------|           | NUMMIS7 := 0

| NUMMIS8 := 0

| NUMMIS := 0

| MIS_MON_KAM := 0

| MIS_NOCALL := 0

| TOTEVENTS := 0

| MIS_ENA_FRST := 1

END: CLEAR_MISFIRE_KAM_REGS

35-26

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: SYNC_CHK

V_SYNCFLG = 0 --------| V_MIS200_1 := 0

| V_MIS200_2 := 0

| V_MIS200_3 := 0

| V_MIS200_4 := 0

| V_MIS200_5 := 0

| V_MIS200_6 := 0

| V_MIS200_7 := 0

| V_MIS200_8 := 0

| V_MIS1000_1 := 0

| V_MIS1000_2 := 0

| V_MIS1000_3 := 0

| V_MIS1000_4 := 0

| V_MIS1000_5 := 0

| V_MIS1000_6 := 0

| V_MIS1000_7 := 0

| V_MIS1000_8 := 0

END: SYNC_CHK

BEGIN: ACCESSORY_CHECK

Any accessory which may cause sudden changes in parasitic torque

and which may affect the misfire detection algorithm are documented.

Also any strategy intended action which may cause or appear as a

misfire (e.g. injector cutout for torque truncation) is documented.

ACCS_OLD <> ACCFLG ---------|

(A/C state change)         |AND -|

|     |

MIS_AC_SW = 1 --------------|     |

(bypass for A/C state change)    |OR --| access_flg := 1

|     | ACCS_OLD := ACCFLG

INJ_ON <> NUMCYL -----------------|     |  (save current ACCFLG)

(one or more injectors commanded       |

off)                                  |

| --- ELSE ---

|

| access_flg := 0

| ACCS_OLD := ACCFLG

END: ACCESSORY_CHECK

35-27

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: MISFIRE_ENA_FLG_TRANSITION_CHK

MIS_TST_ENA = 0 ------------------|

(current ena state)              |AND -| ena_transition_to_off := 1

|     |  (enable transition to off

MIS_ENA_OLD = 1 ------------------|     |  has occurred)

(previous ena state)                   | MIS_ENA_OLD := MIS_TST_ENA

|

| --- ELSE ---

|

| ena_transition_to_off := 0

|  (no on to off transition

|   this time)

| MIS_ENA_OLD := MIS_TST_ENA

END: MISFIRE_ENA_FLG_TRANSITION_CHK

BEGIN: MIS_START_DLYTM_CALC

;execute when called

MISDLYFLG = 0 -----------------| MISECTDLY := FNMISECT(ECT)

| MISACTDLY := FNMISACT(ACT)

| MISTOTDLY := MISECTDLY + MISACTDLY +

MIS_STRT_DLY

| MISDLYFLG := 1

END: MIS_START_DLYTM_CALC

35-28

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: MIS_TST_READY_CHK

When the following logic is true, the misfire test is available for misfire

assessment.  The MIS_TST_RDY flag is controlled.

AICE_SETR1_B(bit3)

= 0 --|

|AND -|

MISFDELTAERR = 0 -----|     |

;rear snsr & no aice err    |OR --|

|     |

AICE_SETR1_B(bit3)          |     |

= 1 --|     |     |

|AND -|     |

PIPTSTFIL = 0 --|     |           |

|OR --|           |

VPIPFLG = 0 ----|                 |

;front snsr & no pip err          |

|

|     ISCFLG <> -1 ---------------|     |

|                                 |OR --|

|     MIS_ISCF_SW = 0 ------------|     |

|                                       |

BG_TMR <= V_MIS_BGMAX ------------|

|

FFG_ECT = 0 ----------------------|

|

ECT > MIS_ECTMIN -----------------|

(ect range to test)              |

|

ECT < MIS_ECTMAX -----------------|

|

N > V_MIS_NMIN -------------------|

(rpm range to test)              |

|

N < V_MIS_NMAX -------------------|

|

access_flg = 0 -------------------|

|AND -| MIS_TST_RDY := 1

TQ_BRAKE_S > MIS_MIN_TQ ----------|     |

|     |

mis_byps_tbl = 1 -----------------|     |

|     |

ena_transition_to_off = 0 --------|     |

|     |

USPD_RUN_TMR >= MISTOTDLY --------|     |

|     |

(continued on next page)

35-29

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|     |

MIS_ER_ENA = 1 -------------|     |     |

(do not bypass while in    |     |     |

fmem if engine-running    |     |     |

test in progress)         |     |     |

|     |     |

DIS_FMEM = 0 ---------------|     |     |

|     |     |

DIS_FMEM = 1 ---------|     |OR --|     |

|     |           |

P0300MALF = 0 --------|     |           |

|     |           |

P0301MALF = 0 --------|     |           |

|     |           |

P0302MALF = 0 --------|     |           |

|     |           |

P0303MALF = 0 --------|AND -|           |

|                 |

P0304MALF = 0 --------|                 |

|                 |

P0305MALF = 0 --------|                 |

|                 |

P0306MALF = 0 --------|                 |

|                 |

P0307MALF = 0 --------|                 |

|                 |

P0308MALF = 0 --------|                 |

(coil failure caused FMEM action.      |

misfire detection continues until     |

it recognizes the misfire due to      |

coil malfunction)                     |

| --- ELSE ---

|

| MIS_TST_RDY := 0

END: MIS_TST_READY_CHK

35-30

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: MISBG_BYPS

MIS_TST_RDY = 1 --------------|

|

MIS_TST_ENA = 1 --------|     |

|OR --|AND -| MISBG_BYPS := 0

MIS_ER_ENA = 1 ---------|     |     |  (misfire test is performed)

|     | Do: TORQUE_CALCULATION

MIS_CAT_CONF = 0 -------------|     | Do: MIS_MONITOR_FLAG

(cat exposed to all cylinders)     | Do: MISFIRE_PARAMETER_CALCULATIONS

| Do: MISFIRE_FAILURE_INDICATION_CONTROL

|     _BY_ENGINE

| Do: CLEAR_MALF_FLAGS

| Do: NOCALL_FILTER_CONTROL

| Do: ZERO_MIS_REGISTERS

|

| --- ELSE ---

MIS_TST_RDY = 1 --------------|     |

|     |

MIS_TST_ENA = 1 --------|     |     |

|OR --|AND -| MISBG_BYPS := 0

MIS_ER_ENA = 1 ---------|     |     |  (misfire test is performed)

|     | Do: TORQUE_CALCULATION

MIS_CAT_CONF = 1 -------------|     | Do: MIS_MONITOR_FLAG

(cat exposed to a bank only)       | Do: MISFIRE_PARAMETER_CALCULATIONS

| Do: MIS_RATE_BY_BANK_INIT

| Do: MISFIRE_FAILURE_INDICATION_CONTROL

|     _BY_BANK

| Do: CLEAR_MALF_FLAGS

| Do: NOCALL_FILTER_CONTROL

| Do: ZERO_MIS_REGISTERS

|

| --- ELSE ---

(continued on next page)

35-31

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

VPIPFLG = 1 ------------------|     |

(pip failure)                |     |

PIPTSTFIL > 0 ----------------|     |

|     |

MISFDELTAERR = 1 -------------|     |

(aice data error)            |     |

|     |

FFG_ECT = 1 ------------------|     |

(ect currently not reliable) |     |

|     |

MIS_TST_ENA = 0 --------|     |     |

(obdII enable flag)    |AND -|     |

|     |OR --| MISBG_BYPS := 1

MIS_ER_ENA = 0 ---------|     |     |  (misfire test is to be bypassed

|     |   filter registers are cleared)

DIS_FMEM = 1 -----------|     |     | V_MIS_LOOPS := MEDIAN_WIDTH - 1

|     |     | NOISE_COUNT := VMIS_NOISBND * 2 + 1

P0300MALF = 1 ----|     |     |     | ACCEL_POINTR := 0

|     |     |     | TQ_POINTER := 0

P0301MALF = 1 ----|     |AND -|     | Do: MIS_FIL_ZERO

|     |           | Do: NOCALL_FILTER_CONTROL

P0302MALF = 1 ----|     |           | Do: ZERO_MIS_REGISTERS

|     |           |

P0303MALF = 1 ----|OR --|           |

|                 |

P0304MALF = 1 ----|                 |

|                 |

P0305MALF = 1 ----|                 |

|                 |

P0306MALF = 1 ----|                 |

|                 |

P0307MALF = 1 ----|                 |

|                 |

P0308MALF = 1 ----|                 |

| --- ELSE ---

|

| MISBG_BYPS := 1

|  (misfire test is to be bypassed

|   filters are maintained at the

|   current level)

| V_MIS_LOOPS := MEDIAN_WIDTH - 1

| NOISE_COUNT := VMIS_NOISBND * 2 + 1

| ACCEL_POINTR := 0

| TQ_POINTER := 0

| Do: NOCALL_FILTER_CONTROL

| Do: ZERO_MIS_REGISTERS

END: MISBG_BYPS

35-32

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: TORQUE_CALCULATION

FN1615M_CAL = 0 --| FN1615_DATA := FN1615A(N,LOAD) * TR_SPK_DELTA

|                   * FN623(SPK_LAMBSE)

|

| --- ELSE ----

|

| FN1615_DATA := FN1615M(N,LOAD) * TR_SPK_DELTA

|                * FN623(SPK_LAMBSE) * FN1615M_MULT(N,LOAD)

END: TORQUE_CALCULATION

BEGIN: BYPASS_TABLE

Each cell of the table FNMISOK will contain "1" if when operating in that

cell, misfires will be allowed to be determined.  If the cell contains "0"

misfire detection will be bypassed.  The calibration parameter BYPS_BRKPNT

allows for 4 point interpolation calculation.

FNMISOK_DATA := FNMISOK(N,LOAD)

FNMISOK_DATA >= BYPS_BRKPNT -----| mis_byps_tbl := 1

|  (misfire detection not bypassed)

|

| --- ELSE ---

|

| mis_byps_tbl := 0

|  (misfire detection is bypassed)

END: BYPASS_TABLE

35-33

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: MISFIRE_PARAMETER_CALCULATIONS

unconditionally --| Do: MIS_KAM_CALCS

| Do: MIS_FILTER_UPDATE_EXPON

| ;misfire rate filters are updated

| Do: MIS_PROFILE_CFMTR_CHK

| Do: MIS_EXPORT_DATA_CONTROL

| ;control od data not used bu misfire

| Do: CALCULATE_EXPLICIT_MISFIRE_RATE

| ;misfire rates based on groups

| ;of engine rotations are calculated

NOISE_COUNT = 0 --| Do: MIS_THRESHOLD_CONTROL

| ;current misfire threshold is calculated only when

| ;misfire malfunctions filters are updated i.e. when

| ;noise_count = 0

END: MISFIRE_PARAMETER_CALCULATIONS

BEGIN: MIS_KAM_CALCS

unconditionally --| NUMMIS1 := NUMMIS1 + V_CYL1_MIS

| NUMMIS2 := NUMMIS2 + V_CYL2_MIS

| NUMMIS3 := NUMMIS3 + V_CYL3_MIS

| NUMMIS4 := NUMMIS4 + V_CYL4_MIS

| NUMMIS5 := NUMMIS5 + V_CYL5_MIS

| NUMMIS6 := NUMMIS6 + V_CYL6_MIS

| NUMMIS7 := NUMMIS7 + V_CYL7_MIS

| NUMMIS8 := NUMMIS8 + V_CYL8_MIS

END: MIS_KAM_CALCS

BEGIN: MIS_PROFILE_CFMTR_CHK

;If profile correction logic indicates that the Correction Factors (CFs)

;in the KAM aren't mature (CF_KAM_MTR = 0)  and  if the cal constant

;CF_KAM_IGNR indicates to check CF's status (CF_KAM_IGNR = 0) then

;exit misfire background logic.

CF_KAM_MTR = 0 ------------|

|AND -| (exit misfire background)

CF_KAM_IGNR = 0 -----------|     | ;cfs aren't mature and cf

;cf mature chk cal'd in          | ;maturity is cal'd in.

|

| --- ELSE ---

|

CF_KAM_MTRO = 0 -----------|     |

|AND -| Do: ZERO_MIS_REGISTERS

CF_KAM_IGNR = 0 -----------|     | Do: MIS_FIL_ZERO

| Do: CLEAR_MISFIRE_KAM_REGS

| CF_KAM_MTRO := 1

END: MIS_PROFILE_CFMTR_CHK

35-34

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: MIS_FILTER_UPDATE_EXPON

TOTMISBGTOBG := V_CYL1_MIS + V_CYL2_MIS + V_CYL3_MIS + V_CYL4_MIS +

V_CYL5_MIS + V_CYL6_MIS + V_CYL7_MIS + V_CYL8_MIS

(total misfires since last misfire test BG

execution)

TOTBGTOBG := V_CYL1_TOT + V_CYL2_TOT + V_CYL3_TOT + V_CYL4_TOT +

V_CYL5_TOT + V_CYL6_TOT + V_CYL7_TOT + V_CYL8_TOT

(total cylinder events since last misfire test BG

execution)

TOTBGTOBG <> 0 --| TOTMISFIL := [(1 - ALPHAMIS * TOTBGTOBG) * TOTMISFIL]

|  + (ALPHAMIS * TOTMISBGTOBG)

|  (update overall misfire filter)

CF_KAM_MTR = 1 --|

|OR -| Do:INDIVIDUAL_CYLINDER_FILTER_UPDATE_EXPON

CF_KAM_IGNR = 1 -|    | ;update each individual cylinder misfire filter

END: MIS_FILTER_UPDATE_EXPON

BEGIN: MIS_EXPORT_DATA_CONTROL

TOTMISFIL > MISLVLEXP ------------------| FFG_MISFIRE := 1

|

| --- ELSE ---

|

| FFG_MISFIRE := 0

END: MIS_EXPORT_DATA_CONTROL

35-35

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: MIS_RATE_BY_BANK_INIT

The following variables are initialized for use by MIS_RATE_BY_BANK.

cyl_num := 1

BANK1_EXPON :=0

BANK2_EXPON :=0

BANK1_EXPLIC :=0

BANK2_EXPLIC :=0

Do: MIS_RATE_BY_BANK

END: MIS_RATE_BY_BANK_INIT

BEGIN: MIS_RATE_BY_BANK

This procedure calculated the misfire rate occurring on each bank of

cylinders of the engine.  If a catalyst configuration is one catalyst on each

bank, then the misfire rate affecting that catalyst temperature must be based

only on the bank associated with that catalyst.

This procedure loops to itself for each cylinder of the engine summing the

misfire rates for the bank associated with the current cylinder number.

bank_num := FN1327(cyl_num)  ; bank associated with cyl_num is assigned to

; bank_num.

cyl_num <= NUMCYL ------|

|AND -| Do: SUM_BANK_1

bank_num = 0 -----------|     |  (current cylinder number is in bank 1

|   the misfire rate for bank 1 will be

|   incremented by the misfire rate of

|   the current cyl_num adjusted by ENGCYL)

| cyl_num := cyl_num + 1

| Do: MIS_RATE_BY_BANK (loop)

|  (continue to call until all cylinders have

|   been accounted for)

|

| --- ELSE ---

cyl_num <= NUMCYL ------|     |

|AND -| Do: SUM_BANK_2

bank_num = 2 -----------|     |  (current cylinder number is in bank 2)

|   the misfire rate for bank 2 will be

|   incremented by the misfire rate of

|   the current cyl_num adjusted by ENGCYL)

| cyl_num := cyl_num + 1

| Do: MIS_RATE_BY_BANK (loop)

|  (continue to call until all cylinders have

|   been accounted for)

END: MIS_RATE_BY_BANK

35-36

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: SUM_BANK_1

cyl_num = 1 --------| BANK1_EXPLIC := BANK1_EXPLIC + mis_rate200_1

| BANK1_EXPON := BANK1_EXPON + CMIS1FIL/ENGCYL

|

| --- ELSE ---

|

cyl_num = 2 --------| BANK1_EXPLIC := BANK1_EXPLIC + mis_rate200_2

| BANK1_EXPON := BANK1_EXPON + CMIS2FIL/ENGCYL

|

| --- ELSE ---

|

cyl_num = 3 --------| BANK1_EXPLIC := BANK1_EXPLIC + mis_rate200_3

| BANK1_EXPON := BANK1_EXPON + CMIS3FIL/ENGCYL

|

| --- ELSE ---

|

cyl_num = 4 --------| BANK1_EXPLIC := BANK1_EXPLIC + mis_rate200_4

| BANK1_EXPON := BANK1_EXPON + CMIS4FIL/ENGCYL

|

| --- ELSE ---

|

cyl_num = 5 --------| BANK1_EXPLIC := BANK1_EXPLIC + mis_rate200_5

| BANK1_EXPON := BANK1_EXPON + CMIS5FIL/ENGCYL

|

| --- ELSE ---

|

cyl_num = 6 --------| BANK1_EXPLIC := BANK1_EXPLIC + mis_rate200_6

| BANK1_EXPON := BANK1_EXPON + CMIS6FIL/ENGCYL

|

| --- ELSE ---

|

cyl_num = 7 --------| BANK1_EXPLIC := BANK1_EXPLIC + mis_rate200_7

| BANK1_EXPON := BANK1_EXPON + CMIS7FIL/ENGCYL

|

| --- ELSE ---

|

cyl_num = 8 --------| BANK1_EXPLIC := BANK1_EXPLIC + mis_rate200_8

| BANK1_EXPON := BANK1_EXPON + CMIS8FIL/ENGCYL

END: SUM_BANK_1

35-37

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: SUM_BANK_2

cyl_num = 1 --------| BANK2_EXPLIC := BANK2_EXPLIC + mis_rate200_1

| BANK2_EXPON := BANK2_EXPON + CMIS1FIL/ENGCYL

|

| --- ELSE ---

|

cyl_num = 2 --------| BANK2_EXPLIC := BANK2_EXPLIC + mis_rate200_2

| BANK2_EXPON := BANK2_EXPON + CMIS2FIL/ENGCYL

|

| --- ELSE ---

|

cyl_num = 3 --------| BANK2_EXPLIC := BANK2_EXPLIC + mis_rate200_3

| BANK2_EXPON := BANK2_EXPON + CMIS3FIL/ENGCYL

|

| --- ELSE ---

|

cyl_num = 4 --------| BANK2_EXPLIC := BANK2_EXPLIC + mis_rate200_4

| BANK2_EXPON := BANK2_EXPON + CMIS4FIL/ENGCYL

|

| --- ELSE ---

|

cyl_num = 5 --------| BANK2_EXPLIC := BANK2_EXPLIC + mis_rate200_5

| BANK2_EXPON := BANK2_EXPON + CMIS5FIL/ENGCYL

|

| --- ELSE ---

|

cyl_num = 6 --------| BANK2_EXPLIC := BANK2_EXPLIC + mis_rate200_6

| BANK2_EXPON := BANK2_EXPON + CMIS6FIL/ENGCYL

|

| --- ELSE ---

|

cyl_num = 7 --------| BANK2_EXPLIC := BANK2_EXPLIC + mis_rate200_7

| BANK2_EXPON := BANK2_EXPON + CMIS7FIL/ENGCYL

|

| --- ELSE ---

|

cyl_num = 8 --------| BANK2_EXPLIC := BANK2_EXPLIC + mis_rate200_8

| BANK2_EXPON := BANK2_EXPON + CMIS8FIL/ENGCYL

END: SUM_BANK_2

35-38

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: MISFIRE_FAILURE_INDICATION_CONTROL_BY_ENGINE

TOTMISFIL > TOTMISLEVEL -------|

(engine is misfiring)         |AND -| Do: MIS_CODE_SET_EXPON

|     |  (determine which cylinders

MIS_FIL_SEL = 0 ---------------|     |   should be considered as

(use filters for rate of misfire)   |   misfiring)

| MISFIRING := 1

|  (indicate misfire currently

|   taking place)

|

| --- ELSE ---

VMIS_TOT200 >= ENGCYL * 200 ---|     |

(mis_rate200 is updated)      |     |

|     | TYPE_A_MIS := 1

mis_rate200 > TOTMISLEVEL -----|AND -| Do: MIS_CODE_SET_EXPLICIT_200

(misfiring over 200 revs)     |     |  (determine which cylinders

|     |   should be considered as

MIS_FIL_SEL = 1 ---------------|     |   misfiring)

(use explicit misfire rate calc)    | MISFIRING := 1

|  (indicate misfire currently

|   taking place)

|

| --- ELSE ---

mis_rate1000 > MISLEVEL1000 ---|     |

(misfiring over 1000 revs)    |     |

|     | TYPE_A_MIS := 0

VMIS_TOT1000 >= ENGCYL * 1000 -|AND -| Do: MIS_CODE_SET_EXPLICIT_1000

(mis_rate1000 is updated)     |     |  (determine which cylinders

|     |   should be considered as

MIS_FIL_SEL = 1 ---------------|     |   misfiring)

(use explicit misfire rate calc)    | MISFIRING := 1

|  (indicate misfire currently

|   taking place)

|

| --- ELSE ---

|

VMIS_TOT1000 >= ENGCYL * 1000 -|     | TYPE_A_MIS := 0

(mis_rate1000 is updated)     |AND -| Do: MIS_SIMILAR_CONDS_HYST_CHK

|     |  ;check similar conditions

MIS_FIL_SEL = 1 ---------------|     |

(use explicit misfire rate calc)    |

| --- ELSE ---

|

MIS_FIL_SEL = 0 ---------------------| MISFIRING := 0

(using filters)                     |  (indicate misfire is not

|   currently taking place)

Do: CYL_DETERMINATION_INTEGRITY_CHK ; determine if cylinder

; identification is possible

END: MISFIRE_FAILURE_INDICATION_CONTROL_BY_ENGINE

35-39

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: MISFIRE_FAILURE_INDICATION_CONTROL_BY_BANK

BANK1_EXPON > TOTMISLEVEL/2 --|

;bank misfire rates           |OR --|

|     |

BANK2_EXPON > TOTMISLEVEL/2 --|     |AND -| Do: MIS_CODE_SET_EXPON

|     | ;determine which cylinders

MIS_FIL_SEL = 0 --------------------|     | ;should be considered as

;use filters for rate of misfire          | ;misfiring

| MISFIRING := 1

| ;indicate misfire currently

| ;taking place

|

| --- ELSE ---

VMIS_TOT200 >= ENGCYL * 200 --------|     |

;mis_rate200 is updated             |     |

|     |

BANK1_EXPLIC > TOTMISLEVEL/2 -|     |     | TYPE_A_MIS := 1

;bank misfire ovr 200 rev     |OR --|AND -| Do: MIS_CODE_SET_EXPLICIT_200

|     |     | ;determine which cylinders

BANK2_EXPLIC > TOTMISLEVEL/2 -|     |     | ;should be considered as

|     | ;misfiring

MIS_FIL_SEL = 1 --------------------|     | MISFIRING := 1

;use explicit misfire rate calc           | ;indicate misfire currently

| ;taking place

|

| --- ELSE ---

mis_rate1000 > MISLEVEL1000 --------|     |

;misfiring over 1000 revs           |     |

|     | TYPE_A_MIS := 0

VMIS_TOT1000 >= ENGCYL * 1000 ------|AND -| Do: MIS_CODE_SET_EXPLICIT_1000

;mis_rate1000 is updated            |     | ;determine which cylinders

|     | ;should be considered as

MIS_FIL_SEL = 1 --------------------|     | ;misfiring

;use explicit misfire rate calc           | MISFIRING := 1

| ;indicate misfire currently

| ;taking place

|

| --- ELSE ---

VMIS_TOT1000 >= ENGCYL * 1000 ------|     |

;mis_rate1000 is updated            |AND -| TYPE_A_MIS := 0

|     | Do: MIS_SIMILAR_CONDS_HYST_CHK

MIS_FIL_SEL = 1 --------------------|     |  ;check similar conditions

;use explicit misfire rate calc           |

|

| --- ELSE ---

|

MIS_FIL_SEL = 0 --------------------------| MISFIRING := 0

;using filters                            | ;indicate misfire is not

| ;currently taking place

Do: CYL_DETERMINATION_INTEGRITY_CHK ; determine if cylinder

; identification is possible

END: MISFIRE_FAILURE_INDICATION_CONTROL_BY_BANK

35-40

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: MIS_SIMILAR_CONDS_HYST_CHK

;execute only when called

mis_rate1000 >= MISLEVELSMLR -----------| MISFIRING := 1

|

| --- ELSE ---

|

| MISFIRING := 0

| ;indicate misfire has not

| ;taken place in 1000 revolutions

| Do: CHECK_MIS_CONDS

| ;public process - call to check

| ;if similar conditions exist

END: MIS_SIMILAR_CONDS_HYST_CHK

BEGIN: MIS_MONITOR_FLAG

The number of consecutive cylinder events while testing for

misfire, as specified by the OBDII regulation, to meet

the misfire test monitor criteria is checked.

V_MIS_TSTCNT >= V_MIS_MONCNT -|

|

CF_KAM_MTR = 1 ---------|     |AND -| MIS_MON := 1

|OR --|     | MIS_MON_KAM := 1

CF_KAM_IGNR = 1 --------|           | ;met OBDII monitor requirement

END: MIS_MONITOR_FLAG

35-41

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: MIS_THRESHOLD_CONTROL

This procedure updates the maximum tolerable misfire rate

malfunction threshold (TOTMISLEVEL).  The values for the

threshold are a function of N and LOAD and are contained in the

table FNMAXMISPCT.  The calibration constant FAST_MIS_LVL selects

the method by which TOTMISLEVEL is updated; 0 = new threshold is

filtered whether increasing or decreasing.  1 = new threshold is

filtered when decreasing but jumps to the new level if the

threshold is increasing.  2 = always jump to new threshold.

FNMAXMISPCT is defined as the maximum misfire rate per catalyst.

For a dual bank system, the strategy calculates the misfire rate

accordingly.  To gather data, misfires induced should be

distributed at a set rate across all cylinders until the catalyst

temperature for damage is exceeeded.  This threshold rate can be

entered into FNMAXMISPCT table for single or dual bank catalyst

without any adjustments.

max_pct_mis_old := MAX_PCT_MIS

MAX_PCT_MIS := FNMAXMISPCT(N,LOAD)

FAST_MIS_LVL = 2 ---------------------|

(always jump to new threshold)       |

|OR --| TOTMISLEVEL := MAX_PCT_MIS

MAX_PCT_MIS >= max_pct_mis_old -|     |     |  (jump to new threshold)

(new threshold greater)        |AND -|     | CMISLEVEL := NUMCYL *

|           |              TOTMISLEVEL

FAST_MIS_LVL = 1 ---------------|           |  (calculate indivudual cylinder

(jump to higher threshold)                 |   misfire threshold for

|   exponential filters)

|

| --- ELSE ---

|

| TOTMISLEVEL :=

|  (1 - ALPHAMIS) * TOTMISLEVEL

|   + ALPHAMIS * MAX_PCT_MIS

|  (filter to new threshold

|   level)

| CMISLEVEL := NUMCYL *

|              TOTMISLEVEL

|  (calculate indivudual cylinder

|   misfire threshold for

|   exponential filters)

END: MIS_THRESHOLD_CONTROL

35-42

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: CALCULATE_EXPLICIT_MISFIRE_RATE

VMIS_TOT200 := VMIS_TOT200 + TOTBGTOBG

(total testable cylinder events for calculation

of catalyst temp criteria misfire rate)

VMIS_TOT1000 := VMIS_TOT1000 + TOTBGTOBG

(total testable cylinder events for calculation

of emission criteria misfire rate)

MISFIRES200 := MISFIRES200 + TOTMISBGTOBG

(accumulating total misfires over 200 revolutions)

MISFIRES1000 := MISFIRES1000 + TOTMISBGTOBG

(accumulating total misfires over 1000 revolutions)

V_MIS200_1 := V_MIS200_1 + V_CYL1_MIS

V_MIS200_2 := V_MIS200_2 + V_CYL2_MIS

V_MIS200_3 := V_MIS200_3 + V_CYL3_MIS

V_MIS200_4 := V_MIS200_4 + V_CYL4_MIS

V_MIS200_5 := V_MIS200_5 + V_CYL5_MIS

V_MIS200_6 := V_MIS200_6 + V_CYL6_MIS

V_MIS200_7 := V_MIS200_7 + V_CYL7_MIS

V_MIS200_8 := V_MIS200_8 + V_CYL8_MIS

V_MIS1000_1 := V_MIS1000_1 + V_CYL1_MIS

V_MIS1000_2 := V_MIS1000_2 + V_CYL2_MIS

V_MIS1000_3 := V_MIS1000_3 + V_CYL3_MIS

V_MIS1000_4 := V_MIS1000_4 + V_CYL4_MIS

V_MIS1000_5 := V_MIS1000_5 + V_CYL5_MIS

V_MIS1000_6 := V_MIS1000_6 + V_CYL6_MIS

V_MIS1000_7 := V_MIS1000_7 + V_CYL7_MIS

V_MIS1000_8 := V_MIS1000_8 + V_CYL8_MIS

35-43

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Individual cylinder misfire rates over 1000 rotations of the enging are

calculated below.  These rates are relative rates based on a comparison of

misfires for that cylinder total events for that of the engine as a whole.

This mean that if cylinder 1 is always misfiring, its filter will contain

0.125 (12.5%) on an eight cylinder engine, 0.167 (16.7%) on a six cylinder

engine and 0.25 (25%) on a four cylinder engine.  To observe absolute misfire

rate by cylinder use the exponential filter for each cylinder (CMISnFIL

n=1..8).

Also the overall misfire rate for the engine over 1000 rotations is

calculated.

VMIS_TOT1000 >= ENGCYL * 1000 --| mis_rate1000 := MISFIRES1000 /

|                    VMIS_TOT1000

| mis_rate1000_1 := V_MIS1000_1 /

|                    VMIS_TOT1000

| mis_rate1000_2 := V_MIS1000_2 /

|                    VMIS_TOT1000

| mis_rate1000_3 := V_MIS1000_3 /

|                    VMIS_TOT1000

| mis_rate1000_4 := V_MIS1000_4 /

|                    VMIS_TOT1000

| mis_rate1000_5 := V_MIS1000_5 /

|                    VMIS_TOT1000

| mis_rate1000_6 := V_MIS1000_6 /

|                    VMIS_TOT1000

| mis_rate1000_7 := V_MIS1000_7 /

|                    VMIS_TOT1000

| mis_rate1000_8 := V_MIS1000_8 /

|                    VMIS_TOT1000

VMIS_TOT1000 >= ENGCYL * 1000 --|

|AND -| MIL_1000_1 := 0

mis_rate1000 <= MISLEVELSMLR ---|     | MIL_1000_2 := 0

| MIL_1000_3 := 0

| MIL_1000_4 := 0

| MIL_1000_5 := 0

| MIL_1000_6 := 0

| MIL_1000_7 := 0

| MIL_1000_8 := 0

35-44

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Individual cylinder misfire rates over 200 rotations of the enging are

calculated below.  These rates are relative rates based on a comparison of

misfires for that cylinder total events for that of the engine as a whole.

This mean that if cylinder 1 is always misfiring, its filter will contain

0.125 (12.5%) on an eight cylinder engine, 0.167 (16.7%) on a six cylinder

engine and 0.25 (25%) on a four cylinder engine.  To observe absolute misfire

rate by cylinder use the exponential filter for each cylinder (CMISnFIL

n=1..8).

Also the overall misfire rate for the engine over 200 rotations is

calculated.

VMIS_TOT200 >= ENGCYL * 200 ---| mis_rate200 := MISFIRES200 / VMIS_TOT200

| mis_rate200_1 := V_MIS200_1 /

|                    VMIS_TOT200

| mis_rate200_2 := V_MIS200_2 /

|                    VMIS_TOT200

| mis_rate200_3 := V_MIS200_3 /

|                    VMIS_TOT200

| mis_rate200_4 := V_MIS200_4 /

|                    VMIS_TOT200

| mis_rate200_5 := V_MIS200_5 /

|                    VMIS_TOT200

| mis_rate200_6 := V_MIS200_6 /

|                    VMIS_TOT200

| mis_rate200_7 := V_MIS200_7 /

|                    VMIS_TOT200

| mis_rate200_8 := V_MIS200_8 /

|                    VMIS_TOT200

| MIL_200_1:= 0

| MIL_200_2:= 0

| MIL_200_3:= 0

| MIL_200_4:= 0

| MIL_200_5:= 0

| MIL_200_6:= 0

| MIL_200_7:= 0

| MIL_200_8:= 0

END: CALCULATE_EXPLICIT_MISFIRE_RATE

35-45

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: INDIVIDUAL_CYLINDER_FILTER_UPDATE_EXPON

Individual cylinder misfire rates are calculated below.  These rates are

absolute rates based on a comparison of misfires and total events for that

cylinder only.  This mean that if cylinder #1 is always misfiring, its filter

will approach 1 (100%) regardless of the number of cylinders in the engine.

(filter constant modified for rate of events for individual cylinders)

unconditionally ---| alphamis_cyl := NUMCYL / [NUMCYL + (1 / ALPHAMIS) - 1]

| cmisfil_tot := 0

For all cylinders calculate CMISxFIL and cmisxfil_tot:

(begin for loop)

For x = 1 to NUMCYL

V_CYLx_TOT <> 0 ---| CMISxFIL := [(1 - alphamis_cyl * V_CYLx_TOT) *

|       CMISxFIL] + (alphamis_cyl * V_CYLx_MIS)

| cmisfil_tot :=  cmisfil_tot +  CMISxFIL

(end for loop)

unconditionally ---| CSUMFIL := cmisxfil_tot/NUMCYL

|  (overall misfire rate calculated)

(the following logic is required for scp mode06 reporting purposes)

ENGCYL = 2 --------| CMIS5FIL := FFFFh

| CMIS6FIL := FFFFh

| CMIS7FIL := FFFFh

| CMIS8FIL := FFFFh

|

| --- ELSE ---

|

ENGCYL = 3 --------| CMIS7FIL := FFFFh

| CMIS8FIL := FFFFh

END: INDIVIDUAL_CYLINDER_FILTER_UPDATE_EXPON

35-46

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: MIS_CODE_SET_EXPON

Set service code when its filter exceeds its threshold.

misfillvlflg := 0

CMIS1FIL > CMISLEVEL --| Do: malfunction(mis,P0301)

|  (call to MIL control procedure)

| misfillvlflg := 1

CMIS2FIL > CMISLEVEL --| Do: malfunction(mis,P0302)

|  (call to MIL control procedure)

| misfillvlflg := 1

CMIS3FIL > CMISLEVEL --| Do: malfunction(mis,P0303)

|  (call to MIL control procedure)

| misfillvlflg := 1

CMIS4FIL > CMISLEVEL --| Do: malfunction(mis,P0304)

|  (call to MIL control procedure)

| misfillvlflg := 1

CMIS5FIL > CMISLEVEL --| Do: malfunction(mis,P0305)

|  (call to MIL control procedure)

| misfillvlflg := 1

CMIS6FIL > CMISLEVEL --| Do: malfunction(mis,P0306)

|  (call to MIL control procedure)

| misfillvlflg := 1

CMIS7FIL > CMISLEVEL --| Do: malfunction(mis,P0307)

|  (call to MIL control procedure)

| misfillvlflg := 1

CMIS8FIL > CMISLEVEL --| Do: malfunction(mis,P0308)

|  (call to MIL control procedure)

| misfillvlflg := 1

misfillvlflg = 0 ------| Do: malfunction(mis,P0300)

|  (call MIL control procedure

END: MIS_CODE_SET_EXPON

35-47

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: MIS_CODE_SET_EXPLICIT_1000

misfillvlflg := 0

mis_rate1000_1 > MISLEVEL1000 ------| Do: malfunction(mis,P0301)

|  (call to MIL control procedure)

| misfillvlflg := 1

| MIL_1000_1 := 1

mis_rate1000_2 > MISLEVEL1000 ------| Do: malfunction(mis,P0302)

|  (call to MIL control procedure)

| misfillvlflg := 1

| MIL_1000_2 := 1

mis_rate1000_3 > MISLEVEL1000 ------| Do: malfunction(mis,P0303)

|  (call to MIL control procedure)

| misfillvlflg := 1

| MIL_1000_3 := 1

mis_rate1000_4 > MISLEVEL1000 ------| Do: malfunction(mis,P0304)

|  (call to MIL control procedure)

| misfillvlflg := 1

| MIL_1000_4 := 1

mis_rate1000_5 > MISLEVEL1000 ------| Do: malfunction(mis,P0305)

|  (call to MIL control procedure)

| misfillvlflg := 1

| MIL_1000_5 := 1

mis_rate1000_6 > MISLEVEL1000 ------| Do: malfunction(mis,P0306)

|  (call to MIL control procedure)

| misfillvlflg := 1

| MIL_1000_6 := 1

mis_rate1000_7 > MISLEVEL1000 ------| Do: malfunction(mis,P0307)

|  (call to MIL control procedure)

| misfillvlflg := 1

| MIL_1000_7 := 1

mis_rate1000_8 > MISLEVEL1000 ------| Do: malfunction(mis,P0308)

|  (call to MIL control procedure)

| misfillvlflg := 1

| MIL_1000_8 := 1

misfillvlflg = 0 -------------------| Do: malfunction(mis,P0300)

|  (call MIL control procedure)

END: MIS_CODE_SET_EXPLICIT_1000

35-48

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: MIS_CODE_SET_EXPLICIT_200

always -----------------------------------| misfillvlflg := 0

mis_rate200_1 > TOTMISLEVEL ---|

|AND -|

MIS_CAT_CONF = 0 --------------|     |

|OR --| Do: malfunction(mis,P0301)

mis_rate200_1 > TOTMISLEVEL/2 -|     |     | ;call to MIL control procedure

|AND -|     | misfillvlflg := 1

MIS_CAT_CONF = 1 --------------|           | MIS200FLG_1 := 1

| MIL_200_1 := 1

|

| --- ELSE ---

|

| MIS200FLG_1 := 0

mis_rate200_2 > TOTMISLEVEL ---|

|AND -|

MIS_CAT_CONF = 0 --------------|     |

|OR --| Do: malfunction(mis,P0302)

mis_rate200_2 > TOTMISLEVEL/2 -|     |     | ;call to MIL control procedure

|AND -|     | misfillvlflg := 1

MIS_CAT_CONF = 1 --------------|           | MIS200FLG_2 := 1

| MIL_200_2 := 1

|

| --- ELSE ---

|

| MIS200FLG_2 := 0

mis_rate200_3 > TOTMISLEVEL ---|

|AND -|

MIS_CAT_CONF = 0 --------------|     |

|OR --| Do: malfunction(mis,P0303)

mis_rate200_3 > TOTMISLEVEL/2 -|     |     | ;call to MIL control procedure

|AND -|     | misfillvlflg := 1

MIS_CAT_CONF = 1 --------------|           | MIS200FLG_3 := 1

| MIL_200_3 := 1

|

| --- ELSE ---

|

| MIS200FLG_3 := 0

mis_rate200_4 > TOTMISLEVEL ---|

|AND -|

MIS_CAT_CONF = 0 --------------|     |

|OR --| Do: malfunction(mis,P0304)

mis_rate200_4 > TOTMISLEVEL/2 -|     |     | ;call to MIL control procedure

|AND -|     | misfillvlflg := 1

MIS_CAT_CONF = 1 --------------|           | MIS200FLG_4 := 1

| MIL_200_4 := 1

|

| --- ELSE ---

|

| MIS200FLG_4 := 0

35-49

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

mis_rate200_5 > TOTMISLEVEL ---|

|AND -|

MIS_CAT_CONF = 0 --------------|     |

|OR --| Do: malfunction(mis,P0305)

mis_rate200_5 > TOTMISLEVEL/2 -|     |     | ;call to MIL control procedure

|AND -|     | misfillvlflg := 1

MIS_CAT_CONF = 1 --------------|           | MIS200FLG_5 := 1

| MIL_200_5 := 1

|

| --- ELSE ---

|

| MIS200FLG_5 := 0

mis_rate200_6 > TOTMISLEVEL ---|

|AND -|

MIS_CAT_CONF = 0 --------------|     |

|OR --| Do: malfunction(mis,P0306)

mis_rate200_6 > TOTMISLEVEL/2 -|     |     | ;call to MIL control procedure

|AND -|     | misfillvlflg := 1

MIS_CAT_CONF = 1 --------------|           | MIS200FLG_6 := 1

| MIL_200_6 := 1

|

| --- ELSE ---

|

| MIS200FLG_6 := 0

mis_rate200_7 > TOTMISLEVEL ---|

|AND -|

MIS_CAT_CONF = 0 --------------|     |

|OR --| Do: malfunction(mis,P0307)

mis_rate200_7 > TOTMISLEVEL/2 -|     |     | ;call to MIL control procedure

|AND -|     | misfillvlflg := 1

MIS_CAT_CONF = 1 --------------|           | MIS200FLG_7 := 1

| MIL_200_7 := 1

|

| --- ELSE ---

|

| MIS200FLG_7 := 0

mis_rate200_8 > TOTMISLEVEL ---|

|AND -|

MIS_CAT_CONF = 0 --------------|     |

|OR --| Do: malfunction(mis,P0308)

mis_rate200_8 > TOTMISLEVEL/2 -|     |     | ;call to MIL control procedure

|AND -|     | misfillvlflg := 1

MIS_CAT_CONF = 1 --------------|           | MIS200FLG_8 := 1

| MIL_200_8 := 1

|

| --- ELSE ---

|

| MIS200FLG_8 := 0

35-50

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

misfillvlflg = 0 -------------------| Do: malfunction(mis,P0300)

| ;call MIL control procedure

| MIS_FCO_FLG := 0

|

| --- ELSE ---

|

| MIS_FCO_FLG := 1

| ;informs MIL output that misfire

| ;FMEM action is active.  Note that

| ;the flags TYPE_A_MIS and MISFIRING

| ;must also be observed to indicate

| ;state of misfire FMEM

END: MIS_CODE_SET_EXPLICIT_200

35-51

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: CYL_DETERMINATION_INTEGRITY_CHK

V_SYNCFLG = 0 indicates that proper cylinder identification is not  possible.

Therefore  any  misfire  service code that has been set will get converted to

code 0300 which indicates that a misfire has taken place but the cylinder  is

unknown.

If V_SYNCFLG = 1 indicating proper cylinder identification is possible, and a

cylinder  specific  misfire  service  code  is  set, then the general misfire

service code is cleared.

V_SYNCFLG = 0 ---------|

|

P0301FAULT = 1 --|     |

|     |

P0302FAULT = 1 --|     |AND -| Do: unknown_misfire(P0300)

|     |     |  (replace specific cylinder

P0303FAULT = 1 --|     |     |   misfire code with

|     |     |   service code P0300)

P0304FAULT = 1 --|     |     |

|OR --|     | Do: CLEAR_FMEM/MIL_RELATED_FLAGS

P0305FAULT = 1 --|           |

|           |

P0306FAULT = 1 --|           |

|           |

P0307FAULT = 1 --|           |

|           |

P0308FAULT = 1 --|           |

| --- ELSE ---

P0301FAULT = 1 --------|     |

|     |

P0302FAULT = 1 --------|     |

|     |

P0303FAULT = 1 --------|     |

|     |

P0304FAULT = 1 --------|     |

|OR --| Do: known_misfire(P0300)

P0305FAULT = 1 --------|        (replace code P0300 with

|         specific cylinder misfire

P0306FAULT = 1 --------|         service code)

|

P0307FAULT = 1 --------|

|

P0308FAULT = 1 --------|

END: CYL_DETERMINATION_INTEGRITY_CHK

35-52

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ZERO_MIS_REGISTERS

Clear foreground event registers for the next misfire test

background pass.

V_CYL1_MIS := 0

V_CYL2_MIS := 0

V_CYL3_MIS := 0

V_CYL4_MIS := 0

V_CYL5_MIS := 0

V_CYL6_MIS := 0

V_CYL7_MIS := 0

V_CYL8_MIS := 0

V_CYL1_TOT := 0

V_CYL2_TOT := 0

V_CYL3_TOT := 0

V_CYL4_TOT := 0

V_CYL5_TOT := 0

V_CYL6_TOT := 0

V_CYL7_TOT := 0

V_CYL8_TOT := 0

VMIS_TOT1000 >= ENGCYL * 1000 --| VMIS_TOT1000 := 0

| MISFIRES1000 := 0

| V_MIS1000_1 := 0

| V_MIS1000_2 := 0   (1000 rotations of the

| V_MIS1000_3 := 0    engine has occurred.

| V_MIS1000_4 := 0    Clear registers for

| V_MIS1000_5 := 0    emissions criteria

| V_MIS1000_6 := 0    misfire assessment of

| V_MIS1000_7 := 0    the next 1000 rotations

| V_MIS1000_8 := 0    of the engine.)

VMIS_TOT200 >= ENGCYL * 200 ----| VMIS_TOT200 := 0

| MISFIRES200 := 0

| V_MIS200_1 := 0

| V_MIS200_2 := 0    (200 rotations of the

| V_MIS200_3 := 0     engine has occurred.

| V_MIS200_4 := 0     Clear registers for

| V_MIS200_5 := 0     catalyst overtemp

| V_MIS200_6 := 0     misfire assessment of

| V_MIS200_7 := 0     the next 200 rotations

| V_MIS200_8 := 0     of the engine.)

END: ZERO_MIS_REGISTERS

35-53

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: MIS_FIL_ZERO

Fault filters will be cleared for the next time the test conditions become

true.  Also clear the register V_MIS_TSTCNT which is used to indicate that

misfire has been monitored.

CMIS1FIL := 0

CMIS2FIL := 0

CMIS3FIL := 0

CMIS4FIL := 0

CMIS5FIL := 0

CMIS6FIL := 0

CMIS7FIL := 0

CMIS8FIL := 0

TOTMISFIL := 0

VMIS_TOT200 := 0

MISFIRES200 := 0

VMIS_TOT1000 := 0

MISFIRES1000 := 0

V_MIS200_1 := 0

V_MIS200_2 := 0

V_MIS200_3 := 0

V_MIS200_4 := 0

V_MIS200_5 := 0

V_MIS200_6 := 0

V_MIS200_7 := 0

V_MIS200_8 := 0

V_MIS1000_1 := 0

V_MIS1000_2 := 0

V_MIS1000_3 := 0

V_MIS1000_4 := 0

V_MIS1000_5 := 0

V_MIS1000_6 := 0

V_MIS1000_7 := 0

V_MIS1000_8 := 0

V_MIS_TSTCNT := 0

END: MIS_FIL_ZERO

BEGIN: NOCALL_FILTER_CONTROL

MIS_ENA_FRST = 1 --| totnocall := MIS_NOCALL + MISBYPSCTR - TOTNOCALLOLD

| TOTNOCALLOLD := TOTNOCALLOLD + totnocall

| totevents_t := MIS_EVENTS + MISBYPSCTR - TOTEVENTS

| TOTEVENTS := TOTEVENTS + totevents_t

| MISNOCALLFIL := [(1 - ALPHANOCALL * totevents_t)

|                 * MISNOCALLFIL]

|                 + (ALPHANOCALL * totnocall)

END: NOCALL_FILTER_CONTROL

35-54

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: CLEAR_FMEM/MIL_RELATED_FLAGS

MIL_200_1 := 0

MIL_200_2 := 0

MIL_200_3 := 0

MIL_200_4 := 0

MIL_200_5 := 0

MIL_200_6 := 0

MIL_200_7 := 0

MIL_200_8 := 0

MIS200FLG_1 := 0

MIS200FLG_2 := 0

MIS200FLG_3 := 0

MIS200FLG_4 := 0

MIS200FLG_5 := 0

MIS200FLG_6 := 0

MIS200FLG_7 := 0

MIS200FLG_8 := 0

MIL_1000_1 := 0

MIL_1000_2 := 0

MIL_1000_3 := 0

MIL_1000_4 := 0

MIL_1000_5 := 0

MIL_1000_6 := 0

MIL_1000_7 := 0

MIL_1000_8 := 0

END: CLEAR_FMEM/MIL_RELATED_FLAGS

35-55

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: CLEAR_MALF_FLAGS

MIL_200_1 = 0 --------|

|AND -| clear_malf(P0301)

MIL_1000_1 = 0 -------|

MIL_200_2 = 0 --------|

|AND -| clear_malf(P0302)

MIL_1000_2 = 0 -------|

MIL_200_3 = 0 --------|

|AND -| clear_malf(P0303)

MIL_1000_3 = 0 -------|

MIL_200_4 = 0 --------|

|AND -| clear_malf(P0304)

MIL_1000_4 = 0 -------|

MIL_200_5 = 0 --------|

|AND -| clear_malf(P0305)

MIL_1000_5 = 0 -------|

MIL_200_6 = 0 --------|

|AND -| clear_malf(P0306)

MIL_1000_6 = 0 -------|

MIL_200_7 = 0 --------|

|AND -| clear_malf(P0307)

MIL_1000_7 = 0 -------|

MIL_200_8 = 0 --------|

|AND -| clear_malf(P0308)

MIL_1000_8 = 0 -------|

35-56

CONTINUOUS SELF TEST, MISFIRE DETECTION TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

MISFIRING = 0 --------|

|

P0301MALF = 1 --------|

|

P0302MALF = 1 --------|

|

P0303MALF = 1 --------|

|

P0304MALF = 1 --------|OR --| clear_malf(P0300)

|

P0305MALF = 1 --------|

|

P0306MALF = 1 --------|

|

P0307MALF = 1 --------|

|

P0308MALF = 1 --------|

END: CLEAR_MALF_FLAGS

35-57

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

35.2  DISTRIBUTORLESS IGNITION SYSTEM TEST (CDAN0)

OVERVIEW

The Continuous DIS Test is actually a number of tests and processes.

Tests performed (by the EEC processor) are as follows:

PIP circuit; code P0320

IDM circuit; code P1351

CID circuit; code P0340  (not a device used by the ignition system

however the nature of its signal and how

it is processed make it natural to test

within this group)

EDIS module test; code 1367

Individual coil driver circuits, primary side of the coils;

coil A primary circuit; code P0351

coil B primary circuit; code P0352

coil C primary circuit; code P0353

coil D primary circuit; code P0354

EEC/EDIS communication interface:

The positive pulsewidth of the IDM signal is used by  the  EDIS  to  transmit

information  to  the  EEC.   The EEC measures the IDM positive pulsewidth and

takes the appropriate action based on the measured  width.   Pulsewidths  for

the following may be output:

-  EDIS module operating properly and the engine is not turning

-  EDIS detects no failures and the engine is turning

Testing is performed by the EEC that detects an IDM positive pulse width that

is  not  defined.   If  this  test is calibrated in, a service code 1357 will

result.

If there is no pulse from the EDIS is present during  Engine  Off  on  demand

self-test, a service code 1358 will be set.

PIP test overview:

The PIP circuit test is divided into  hard  fault  (i.e.   complete  loss  of

signal)  and  intermittents.   Hard  faults  are tested by comparing the time

since the last PIP edge has occurred.  If this  time  exceeds  a  maximum,  a

failure is indicated.  Stalls etc.  are protected against.  Intermittents are

tested by comparing three consecutive PIP periods.  Under the  assumption  of

constant  excelleration  over  the  three  periods,  a  relation of the three

periods can determine if an intermittent failure has occurred.  See  the  PIP

down interrupt logic of these tests for the relationship.

35-58

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

IDM test overview:

The IDM signal when coming from an EDIS processor is governed by two factors;

1)  the  positive pulsewidth is determined by the message the EDIS is sending

to the EEC.  2) an IDM rising edge is output only when a spark event actually

occurs.  During the IDM up interrupt a flag named V_SPK_FLG is set indicating

that a spark event has taken place.  When  the  PIP  down  interrupt  occurs,

V_SPK_FLG  is  tested.   Since,  under  normal  circumstances (no IDM circuit

failures, missed sparks) there will be one and only one spark  event  between

PIP down edges, V_SPK_FLG = 0 indicates that the IDM interrupt did not occur.

This may be the result of loss of  spark  or  an  IDM  circuit  failure.   To

distinguish  between  the  two,  an  IDM failure will not be considered until

VNREVIDM rotations of the engine  have  occured  with  no  IDM  events  being

experienced.   This  distinguishes  an  IDM circuit failure from missed spark

events since loss of a coil will result in at least one spark event during  a

complete  rotation  of  the  engine.  However loss of all coils (or B+ to the

coil pack) will result in the same signal pattern (no IDMs) as a  failed  IDM

circuit.   The  difference is that the engine will run normally with a failed

IDM circuit, loss of the  coils  will  result  in  an  engine  stall.   Rapid

intermittent  IDM is tested knowing that there is a one to one correspondence

between PIP and IDM (when considering  the  PIP  down  edge).   The  register

PIPIDM  is incremented on each PIP down interrupt and decremented on each IDM

up interrupt.  Given the one to one correspondence between PIP  and  IDM  and

also given that PIP is known to be a good signal, than if the PIPIDM register

becomes less  than  -1,  more  IDM  events  than  PIP  events  have  occurred

indicating an intermittent IDM circuit.

CID circuit test overview:

For every NUMCYL PIP down events there is one and only  one  CID  event  (CID

down  for VRS CID sensors and CID up for hall CID sensors).  Therefore if PIP

is known to be valid and exactly one CID event did not occur after NUMCYL PIP

events a CID circuit failure is indicated.

Coil primary circuit test overview:

Simply, during the PIP down interrupt When V_SPK_FLG = 0  indicating  that  a

spark  event  has  been  missed, a register is incremented that is associated

with the coil which missed the  spark.   This  information  is  used  by  the

background test logic in determining whether to set a coil service code.

PIPs while engine not turning overview:

The EDIS module outputs a specific IDM positive pulsewidth when it  determine

via  its 36-1 tooth crank sensor that the engine is not turning.  If a PIP is

encountered while this pulsewidth is received a  service  code  1356  can  be

output if this test is turned on.

35-59

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  CCM_EO_ENA = Engine off on demand test for CCM enabled.

-  CCM_TST_ENA = OBD-II Comprehensive Component Test enable flag; 1 ->  test

enabled.

-  CIDTSTFIL = CID circuit failure fault filter.

-  COILAPFIL = Coil A primary failure fault filter.

-  COILBPFIL = Coil B primary failure fault filter.

-  COILCPFIL = Coil C primary failure fault filter.

-  COILDPFIL = Coil D primary failure fault filter.

-  COILUFIL = Unknown Coil failure fault filter.

-  DATA_TIME = 3 byte time of transition.

-  DT12S = The period of time between two  adjacent  rising  edges  of  PIP,

ticks/pip.

-  DT23S = Last DT12S time.

-  DT34S = Last DT23S time (Used in HDR EDIS Test).

-  ERROR_CNTR = Number of undefined pulsewidths received from EDIS.

-  ERROR_TMR = Invalid IDM positive pw test timer.

-  IDMFIL = IDM circuit failure fault filter.

-  IDM_PW = IDM pulse width for previous PIP period.

-  IDM_PW_STARTM = Last time IDM changed high, clock ticks.

-  N = Engine speed RPM.

-  PIPIDM = Counter incremented on each PIP low and decremented on each  IDM

high.

-  PIPTSTFIL = PIP circuit failure fault filter.

-  PUTMR = Time after CPU power up.

-  SYNC_CTR = Sequence counter incremented every PIP  rising  edge  that  is

used to indentify the cycle and position.

-  TSLIDM = Time since last IDM interrupt.

-  TSLPIP = Time since last PIP.

-  V_CL_BRK_TMR = Driver action procedure timer; timer zeroed when brake  or

clutch depressed.

35-60

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  VCIDCNT = Counter incremented on each CID high (hall sensor) or  CID  low

(VRS sensor).

-  VCIDERRCNT  =  Number  of  CID  failure  indications  between  background

executions.

-  VCIDPASCNT = Number of CID non  failure  indications  between  background

executions.

-  V_COILOK_CTR = Number of consecutive valid spark events.

-  V_CPU_OK_TMR = PIP with engine not turning test timer

-  V_IDM_CTR = Number of undetermined IDM faults.

-  VIDMERRCNT  =  Number  of  IDM  failure  indications  between  background

executions.

-  VIDMPASCNT = Number of IDM non  failure  indications  between  background

executions.

-  V_PAC_A_CTR = Number of coil A primary fault occurrences.

-  V_PAC_B_CTR = Number of coil B primary fault occurrences.

-  V_PAC_C_CTR = Number of coil C primary fault occurrences.

-  V_PAC_D_CTR = Number of coil D primary fault occurrences.

-  V_PAC_U_CTR = Number of unknown coil fault occurrences.

-  VPIPCNT = Counter incremented on each PIP low.

-  VPIPERRCNT  =  Number  of  PIP  failure  indications  between  background

executions.

-  VPIPPASCNT = Number of  PIP  nonfailure  indications  between  background

executions.

Bit Flags:

-  BIFLG = Brake ON/OFF indicator; 0 -> ON, 1 -> OFF.

-  BIHP = Brake input 'hardware present' indicator; 0 -> no, 1 -> yes.

-  FFG_CPP = OBD-II system flag for Clutch Pedal Position (CPP)switch.  1 ->

CPP is currently unreliable.

-  CODE_1356_FLG = PIPs received with engine not turning FG flag.

-  CODE_1357_FLG = Invalid IDM PW received FG flag.

-  CPU_OK_1 = When set first cpu ok IDM PW received from EDIS.

-  CRKFLG = Crank flag.

-  FFG_CID = OBDII system FMEM flag for CID;  1  ->  CID  is  not  currently

reliable.

35-61

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  FFG_PIP = OBDII system FMEM flag for PIP;  1  ->  PIP  is  not  currently

reliable.

-  IDM_BYPASS = When set causes IDM test to be bypassed due to PIP failure.

-  IDM_RLBL_FLG = When 1, indicated that IDM signal is currently reliable.

-  PxxxMALF = Malfunction flag for code Pxxx; 1 -> a  malfunction  currently

exists for fault Pxxx.

-  UNDSP = Underspeed flag.

-  VCID_BYPS = 1 -> the CID test background is bypassed.

-  VCIDFLG = 1 -> CID fault (VIP error flag).

-  V_IDM_FLG = When set to 1 indicates that the first missed IDM after valid

IDMs has passed.

-  VKOEO_CPUOK = Flag when set indicates to KOEO self test that EDIS  module

OK.

-  VPIPFLG = PIP failure flag set in FG.

-  V_PRE_CPP = Previous state of FFG_CPP.

-  V_SPK_FLG = When set indicates IDM (spark) event occurred since last  PIP

falling edge.

-  V_SYNCFLG = When set to zero indicated engine is not in sync.

Calibration Constants:

-  CIDLVLEXP = CID failure fault filter threshold for export use.

-  CIDTSTLVL = CID failure fault filter threshold.

-  CIDUP = CID failure fault filter increment.

-  CPUOK_PW = valid IDM high pulsewidth for KOEO condition, clock tics.

-  CPRIMLVL = Coil primary failure fault filter threshold.

-  CPRIMUP = Coil primary failure fault filter increment.

-  ENGCYL = Number of PIP'S per engine revolution; 2 -> 4  cyl.   engine  (2

PIP/rev),  3  ->  6  cyl.   engine  (3  PIP/rev),  4 -> 8 cyl.  engine (4

PIP/rev).

-  HP_CIDSEL = CID sensor hardware select  switch  set;  0  ->  Hall  Effect

sensor, 1 -> VR sensor if a CID sensor.

-  IDMLVL = IDM failure fault filter threshold.

-  IDMUP = IDM failure fault filter increment.

-  IDM_VBATMIN = Minimum VBAT  level  required  before  IDM  signal  can  be

considered reliable.

35-62

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  IDM_VBATMAX = Maximum VBAT level below which  IDM  signal  is  considered

reliable.

-  IDM_NMAX = Minimum N (RPM) required before IDM signal can  be  considered

reliable.

-  IGNR_CPP = 1 -> ignore clutch pedal position switch test input.

-  MAX_ERROR  =  Maximum  number  of  erroneous  FG  IDM   high   pw   fault

transmissions before setting EDIS transmission fault code.

-  MAX_TIME = Maximum time window in which to  recognize  erroneous  FG  IDM

high pw transmissions and set EDIS transmission fault code.

-  OKIDM_PW = Valid IDM high pulsewidth for KOER with no failures  reported,

clock tics.

-  PIPERR = PIP error level to set PIP fault, unitless ratio of clock tics.

-  PIPLVLEXP = PIP failure fault filter threshold for export use.

-  PIPTSTLVL = PIP failure fault filter threshold.

-  PIPUP = PIP failure fault filter increment.

-  PW_EPSILON2 = IDM FG high pulsewidth tolerance, clock tics.

-  TRLOAD = Transmission load switch.

-  V_CL_BRK_TM = Time to delay after driver uses brake or declutches.

-  VCIDSW = CID test enable switch.

-  VDLY6 = Time to wait after PIP has been  restored  after  failure  before

testing IDM.

-  V_1PAC_ERR = Coil pack FG fault counter threshold  before  uncounting  BG

fault filter.

-  V_CIDN_MAX = CID sensor not tested above this value.

-  V_CIDN_MIN = CID sensor not tested below this value.

-  V_CPU_OK_DUR = Time delay before an erroneous PIP can  be  tested  during

KOEO condition, sec.

-  V_CPUOK_ENA = KOEO EDIS module test enable switch.  1 -> do  test;  0  ->

skip test

-  V_CPU_OK_PTM = Maximum time window in which to detect an  erronewous  PIP

occuring during the KOEO condition.

-  V_IDM_NOISE = Number of PIP periods without an IDM  event  considered  an

IDM failure.

-  VNREVIDM = Number of revolutions  of  the  engine  without  a  valid  IDM

considered an IDM fault.

35-63

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  V_PIP_HARD = TSLPIP greater than this value is considered a loss  of  PIP

signal.

-  VPMAX = Maximum PIP period for PIP test background execution.

OTHER

-  P0320 = Fault code, PIP circuit failure.

-  P0340 = Fault code, CID circuit failure.

-  P0350 = Fault code, undetermined coil primary failure.

-  P0351 = Fault code, Coil A primary failure.

-  P0352 = Fault code, Coil B primary failure.

-  P0353 = Fault code, coil C primary failure.

-  P0354 = Fault code, Coil D primary failure.

-  P0703 = Change in brake state not seen in continuous test.

-  P0704 = Clutch switch failure malfunction code.

-  P1351 = Fault code, IDM circuit failure.

-  P1356 = Fault code, PIP received while engine not turning.

-  P1357 = Fault code, unexpected IDM signal received.

-  P1358 = Fault code, no KOEO CPU OK (IDM) pulse from EDIS.

35-64

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OVERVIEW

The IDM interrupt provides information that the PIP and IDM test  require  to

determine the nature of the fault.

The IDM interrupt also is used to  receive  input  from  the  E-DIS  computer

(IDM_PW).   The  appropriate  action  is  taken based on the value of IDM_PW.

Also V_SPK_FLG is set to indicate an IDM  occurred  during  the  current  PIP

period.

PROCESS

STRATEGY MODULE: VC_DIS_COM6

BEGIN: IDM_PRCS

;execute in the IDM up interrupt routine

The following modifies the PIPIDM register documenting this IDM up event.

It also documents that this interrupt occurred for use by the PIP down

interrupt via V_SPK_FLG.

PIPIDM = -128 ------------| V_SPK_FLG := 1

(PIPIDM lower clip)      |  (indicates to the DIS test PIP down logic

|   that an IDM (spark) has occurred since the

|   previous PIP down)

| IDM_PW_STARTM := DATA_TIME

|  (time of this edge saved, see DIS test IDM

|   down interrupt logic for use of this time)

| Do: IDM_PW_ACTION_SELECT

|  (selects action based on positive pulse

|   width of IDM)

|

| --- ELSE ---

|

| PIPIDM := PIPIDM - 1

| V_SPK_FLG := 1

|  (indicates to the DIS test PIP down logic

|   that an IDM (spark) has occurred since the

|   previous PIP down)

| IDM_PW_STARTM := DATA_TIME

|  (time of this edge saved, see DIS test IDM

|   down interrupt logic for use of this time)

| Do: IDM_PW_ACTION_SELECT

|  (selects action based on positive pulse

|   width of IDM)

END: IDM_PRCS

35-65

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUOUS DIS TEST - IDM Up Interrupt Logic

BEGIN: IDM_PW_ACTION_SELECT

;execute when called from IDM_PRCS

Select the appropriate process to perform based on IDM_PW  generated  by  the

E-DIS module.

(OKIDM_PW - PW_EPSILON2) <= IDM_PW <=  ------| Do: EDIS_NOFAIL

(OKIDM_PW + PW_EPSILON2)                     |

(E-DIS indicates engine is turning and no   |

failure present)                            |

| --- ELSE ---

|

(CPUOK_PW - PW_EPSILON2) <= IDM_PW <= -------| Do: EDIS_CPUOK

(CPUOK_PW + PW_EPSILON2)                     |

(E-DIS indicates engine is not turning)    |

| --- ELSE ---

|

| Do: IDM_PW_ERROR

END: IDM_PW_ACTION_SELECT

35-66

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUOUS DIS TEST - IDM Up Interrupt Logic

BEGIN: IDM_PW_ERROR

;execute when called from IDM_PRCS

This process is executed when a  IDM  pulsewidth  is  received  that  is  not

expected,  e.g.   any  undefined  PW  or a PW previously defined but EDIS has

opted not to utilize it.  In  the  second  example  the  calibratable  enable

register CODEx_ENA will equal -1, where x is 1 thru 6.

When the first error is detected (ERROR_CNTR = 1) the timer ERROR_TMR is  set

to  zero.   This  timer  will  be used to determine the "time density" of the

unexpected IDM_PW's received.  If this density exceeds the calibratable limit

MAX_ERROR in MAX_TIME an error code 1357 will be set.

This test can be disabled by the following calibration:

MAX_ERROR := 65535

MAX_TIME := 0

unconditionally -----------------| ERROR_CNTR := ERROR_CNTR + 1

| (tally number of errors detected)

ERROR_CNTR = 1 ------------------| ERROR_TMR := 0

(this is the first error, zero the timer)

ERROR_CNTR >= MAX_ERROR ---|

|

ERROR_TMR < MAX_TIME ------|

(density exceeded, failure |AND -| CODE_1357_FLG := 1

is indicated)             |     |  (set flag for BG processing)

|     | ERROR_CNTR := 0

CCM_TST_ENA = 1 -----------|     |  (clear counter)

|

| --- ELSE ---

|

ERROR_CNTR >= MAX_ERROR ---------| ERROR_CNTR := 0

(no failure after MAX_ERROR     |  (clear counter for detection of

events has occurred)           |   the next episode)

END: IDM_PW_ERROR

BEGIN: DIS_CLEAR_FILTERS

unconditionally -----------------| COILAPFIL := 0

| COILBPFIL := 0

| COILCPFIL := 0

| COILDPFIL := 0

| COILUFIL  := 0

END: DIS_CLEAR_FILTERS

35-67

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUOUS DIS TEST  - IDM Up Interrupt Logic

BEGIN: EDIS_CPUOK

;execute when called from IDM_PRCS

E-DIS has output an IDM_PW = CPUOK_PW, engine not turning.  This  logic  will

determine if a PIP signal occurs while the EDIS module is indicating that the

engine is not turning.

CPU_OK_1 = 0 ---------------------------| V_CPU_OK_TMR := 0

(this is the first IDM interrupt       | CPU_OK_1 := 1

since E-DIS determined the engine     |  (indicates first IDM received)

is not turning)                       | VKOEO_CPUOK := 1

|  (used by KOEO E-DIS module

|   test)

| Do: DIS_CLEAR_FILTERS

|

| --- ELSE ---

|

V_CPU_OK_TMR <= V_CPU_OK_DUR -----------| Do: DIS_CLEAR_FILTERS

(time limit before determining         |

if PIP's are being received while     |

engine is not turning has not         |

been reached)                         |

| --- ELSE ---

CCM_TST_ENA = 1 ------------------|     |

|AND -| CODE_1356_FLG := 1

TSLPIP <= V_CPU_OK_PTM -----------|     |  (set flag for BG processing)

(PIP's are occurring while             | CPU_OK_1 := 0

the E-DIS is indicating the           |  (clear flag in preparation to

engine is not turning)                |   test again)

| Do: DIS_CLEAR_FILTERS

|

CALIBRATION NOTE:                  |

|

calibration of V_CPU_OK_PTM           |

should be incrementally less          |

than V_CPU_OK_DUR                     | --- ELSE ---

|

| (no pip while engine not

|  turning)

| CPU_OK_1 := 0

|  (clear flag in preparation

|   to test again)

| DO: DIS_CLEAR_FILTERS

END: EDIS_CPUOK

BEGIN: EDIS_NOFAIL

unconditionally ------------------------| CPU_OK_1 := 0

| VKOEO_CPUOK := 0

END: EDIS_NOFAIL

35-68

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUOUS DIS TEST - PIP Down Interrupt Logic

BEGIN: CURRENT_COIL

;execute only when called from 'fgcoilidm' proc

ENGCYL = 2 ----------|

|

SYNC_CTR = 0 --|     |AND -| temp(A) = 1

|     |     | temp(B) = 0

SYNC_CTR = 2 --|OR --|     | temp(C) = 0

|           | temp(D) = 0

SYNC_CTR = 4 --|           |

| --- ELSE ---

ENGCYL = 2 ----------|     |

|AND -| temp(A) = 0

SYNC_CTR = 1 --|     |     | temp(B) = 1

|OR --|     | temp(C) = 0

SYNC_CTR = 3 --|           | temp(D) = 0

|

| --- ELSE ---

ENGCYL = 3 ----------|     |

|     |

SYNC_CTR = 0 --|     |AND -| temp(A) = 1

|     |     | temp(B) = 0

SYNC_CTR = 3 --|OR --|     | temp(C) = 0

|           | temp(D) = 0

SYNC_CTR = 6 --|           |

| --- ELSE ---

ENGCYL = 3 ----------|     |

|AND -| temp(A) = 0

SYNC_CTR = 1 --|     |     | temp(B) = 1

|OR --|     | temp(C) = 0

SYNC_CTR = 4 --|           | temp(D) = 0

|

| --- ELSE ---

ENGCYL = 3 ----------|     |

|AND -| temp(A) = 0

SYNC_CTR = 2 --|     |     | temp(B) = 0

|OR --|     | temp(C) = 1

SYNC_CTR = 5 --|           | temp(D) = 0

|

| --- ELSE ---

(continued on next page)

35-69

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUOUS DIS TEST - PIP Down Interrupt Logic

(continued from previous page)

ENGCYL = 4 ----------|     |

|     |

SYNC_CTR = 0 --|     |AND -| temp(A) = 1

|     |     | temp(B) = 0

SYNC_CTR = 4 --|OR --|     | temp(C) = 0

|           | temp(D) = 0

SYNC_CTR = 8 --|           |

| --- ELSE ---

ENGCYL = 4 ----------|     |

|AND -| temp(A) = 0

SYNC_CTR = 1 --|     |     | temp(B) = 1

|OR --|     | temp(C) = 0

SYNC_CTR = 5 --|           | temp(D) = 0

|

| --- ELSE ---

ENGCYL = 4 ----------|     |

|AND -| temp(A) = 0

SYNC_CTR = 2 --|     |     | temp(B) = 0

|OR --|     | temp(C) = 1

SYNC_CTR = 6 --|           | temp(D) = 0

|

| --- ELSE ---

ENGCYL = 4 ----------|     |

|AND -| temp(A) = 0

SYNC_CTR = 3 --|     |     | temp(B) = 0

|OR --|     | temp(C) = 0

SYNC_CTR = 7 --|           | temp(D) = 1

|

| --- ELSE ---

|

| temp(A) = 0

| temp(B) = 0

| temp(C) = 0

| temp(D) = 0

END: CURRENT_COIL

35-70

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUOUS DIS TEST - PIP Down Interrupt Logic

BEGIN: DIS_PIPDOWN_FG

;execute in the pip down interrupt routine

CRKFLG = 1 ---------------| PIPIDM := 0

;crank mode               | VPIPCNT := 0

| VCIDCNT := 0

| VPIPERRCNT := 0

| VPIPPASCNT := 0

|  ;no testing is performed until crank mode is

|  ;exited

|

| --- ELSE ---

|

| Do: FGPIPTST

|  ;foreground part of PIP test is performed

| Do: IDM_RLBLTY_CHK

|  ;is idm signal reliable

END: DIS_PIPDOWN_FG

BEGIN: IDM_RLBLTY_CHK

;execute when called

;IDM signal is NOT reliable above certain RPM (N) and below a certain

;battery voltage(VBAT) level. When spout test indicates that requested

;spark advance is out of spout test range, bypass IDM/Coil tests.

VBAT < IDM_VBATMIN --|

|

VBAT > IDM_VBATMAX --|

|OR --| IDM_RLBL_FLG := 0

N > IDM_NMAX --------|     |  (idm is currently not reliable)

|     | V_SPK_FLG := 0

IGNTST_RSIR = 0 -----|     |  (clear spark indicator flag)

| PIPIDM := 0

|  (clear pip-idm relation register)

|

| --- ELSE ---

|

| IDM_RLBL_FLG := 1

|  (idm is reliable)

| Do: INCPIPIDM

|  (pip idm relation register is incremented

|   accounting for this pip occurrance)

| Do: FGCOILIDM

|  (foreground logic for the testing of coil

|   failures or idm failures is performed)

END: IDM_RLBLTY_CHK

35-71

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: FGPIPTST

[(DT12S+DT34S)/2]/DT23S <

[1-PIPERR] ---|

(PIP error indicated)   |

|OR --| VPIPFLG := 1

[(DT12S+DT34S)/2]/DT23S > |     |  (failure indicated during this period)

[1+PIPERR] ---|     |  VPIPERRCNT := VPIPERRCNT + 1

|  (tally pip failures to be used by bg

|   pip fault filter)

|

| --- ELSE ---

|

| VPIPFLG := 0

|  (document a valid pip during this

|   period)

| VPIPPASCNT := VPIPPASCNT + 1

|  (tally valid pips, used by bg pip

|   fault filter)

END: FGPIPTST

BEGIN: INCPIPIDM

;execute when called

PIPIDM < 127 -------------------------| PIPIDM := PIPIDM + 1

(PIPIDM not at its upper clip)          (pip idm relation register is

incremented accounting for this

pip occurrance)

END: INCPIPIDM

35-72

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUOUS DIS TEST - PIP Down Interrupt Logic

BEGIN: FGCOILIDM

;execute when called

V_SPK_FLG <> 0 -----------------| V_SPK_FLG := 0

(ignition event since previous |  (reset for next event)

pip down has been documented  | V_IDM_FLG := 0

in the IDM interrupt)         |  (reset for next event)

| V_COILOK_CTR := V_COILOK_CTR + 1

|  (tally consecutive spark events)

| Do: COIL_OK_CTR_CHK

| VIDMPASCNT := VIDMPASCNT + 1

|  (tally valid idm (spark) events,

|   used by bg idm fault filter)

|

| --- ELSE ---

V_IDM_FLG = 0 ------------|     |

(first missed IDM after |     |

valid IDM)             |AND -| V_COILOK_CTR := 0

|     |

V_SYNCFLG = 1 ------------|     | V_IDM_FLG := 1

(coil determination           |  (first missed idm has taken place

possible)                   |   after valid idm)

| V_IDM_CTR := 1

|  (tally first consecutive missed

|   idm event)

| Do: CURRENT_COIL

| Do: DETERMINE_PRIMARY_COIL

|  (document coil associated with

|   missed idm)

|

| --- ELSE ---

|

V_SYNCFLG = 1 ------------------| V_COILOK_CTR := 0

(coil determination possible,  | V_IDM_CTR := V_IDM_CTR + 1

process remainder of          |  (tally missed IDM event - note

consecutively missed IDMs)    |   that a missed IDM event in this

|   case is VNREVIDM rotations of

|   the engine with no IDMs occurring)

| Do: IDM_CTR_CHK

| Do: CURRENT_COIL

| Do: DETERMINE_PRIMARY_COIL

|

| --- ELSE ---

|

V_IDM_FLG = 1 ------------------| (coil determination not possible -

(coil determination not       |  first missed IDM after valid IDM)

possible,  process remainder | V_COILOK_CTR := 0

of consecutively missed      | V_IDM_CTR := V_IDM_CTR + 1

IDMs)                        | Do: IDM_CTR_CHK

| V_PAC_U_CTR := V_PAC_U_CTR + 1

|   (increment Unknown Coil counter)

|

| --- ELSE ---

(continued on next page)

35-73

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|

| V_COILOK_CTR := 0

| V_IDM_FLG := 1

| V_IDM_CTR := 1

| V_PAC_U_CTR := V_PAC_U_CTR + 1

|   (increment Unknown Coil counter)

END: FGCOILIDM

35-74

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUOUS DIS TEST - PIP Down Interrupt Logic

BEGIN: COIL_OK_CTR_CHK

;execute only when called from 'fgcoilidm' proc

V_COILOK_CTR >= ENGCYL * VNREVIDM ---------| V_PAC_A_CTR := 0

(coils are all working)                   | V_PAC_B_CTR := 0

| V_PAC_C_CTR := 0

| V_PAC_D_CTR := 0

| V_PAC_U_CTR := 0

|  (clear any left over

|   coil failure data)

END: COIL_OK_CTR_CHK

BEGIN: IDM_CTR_CHK

;execute only when called from 'fgcoilidm' proc

V_IDM_CTR >= ENGCYL * VNREVIDM ------------| VIDMERRCNT := VIDMERRCNT + 1

(missed IDM determined to be an IDM       |  (tally failed IDM, used by

circuit failure - not due to coil        |   BG IDM fault filter)

failures)

END: IDM_CTR_CHK

BEGIN: DETERMINE_PRIMARY_COIL

;execute only when called from 'fgcoilidm' proc

temp(A) = 1 ------------| V_PAC_A_CTR := V_PAC_A_CTR + 1

(coil A)               |  (tally coil A failures)

|

| --- ELSE ---

|

temp(B) = 1 ------------| V_PAC_B_CTR := V_PAC_B_CTR + 1

(coil B)               |  (tally coil B failures)

|

| --- ELSE ---

|

temp(C) = 1 ------------| V_PAC_C_CTR := V_PAC_C_CTR + 1

(coil C)               |  (tally coil C failures)

|

| --- ELSE ---

|

temp(D) = 1 ------------| V_PAC_D_CTR := V_PAC_D_CTR + 1

(coil D)                  (tally coil D failures)

END: DETERMINE_PRIMARY_COIL

35-75

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUOUS DIS TEST - PIP Up Interrupt Logic

BEGIN: CIDTST_PIPUP_FG

(execute in the PIP up interrupt routine)

CRKFLG = 1 ------|

|OR -----------| VPIPCNT := 0

VPIPFLG = 1 -----|              |  (clear since pip is no longer valid)

(in crank or pip failure)       | VCIDCNT := 0

|  (clear, invalid pip will invalidate

|   the cid test)

|

| --- ELSE ---

|

| VPIPCNT := VPIPCNT + 1

|  (tally consecutive pip periods between

|   cids)

| Do: CIDTST

|  (number of pips/cid test)

END: CIDTST_PIPUP_FG

BEGIN: CIDTST

(execute when called)

VCIDSW = 0 ---------------------|

(CID test cal'd out)           |OR --| VPIPCNT := 0

|     | VCIDCNT := 0

VCID_BYPS = 1 ------------------|     | VCID_BYPS := 0

(bg is bypassing test)               |

| --- ELSE ---

VPIPCNT = ENGCYL * 2 -----------|     |

(number of PIP periods before  |     |

a CID signal is expected)     |AND -| VCIDFLG := 0

|     |  (indicate CID is valid)

VCIDCNT = 1 --------------------|     | VCIDPASCNT := VCIDPASCNT + 1

(exactly one CID signal              |  (tally valid cids, used by bg cid

has occurred; tallied in the        |   fault filter)

CID interrupt logic)                | VPIPCNT := 0

| VCIDCNT := 0

|

| --- ELSE ---

|

VPIPCNT = ENGCYL * 2 -----------------| VCIDFLG := 1

(number of PIP periods before        |  (expected number of cid signals

a CID signal is expected)           |   not seen, indicate a cid

|   failure)

| VCIDERRCNT := VCIDERRCNT + 1

|  (tally cid failures, used by bg cid

|   fault filter)

| VPIPCNT := 0

| VCIDCNT := 0

| V_SYNCFLG := 0

|  (indicates loss of sync)

END: CIDTST

35-76

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUOUS DIS TEST - IDM DOWN Interrupt Logic

Calculate the IDM positive pulsewidth.  (In this logic IDM up

and IDM down refer to the signal as seen at the EEC - vehicle

harness)

unconditionally ---------| IDM_PW := DATA_TIME - IDM_PW_STARTM

CONTINUOUS DIS TEST - CID Down Interrupt Logic

When a VRS type CID sensor is in use, this logic documents this interrupt has

been  executed.   This  logic also indicates whether or not the engine was in

sync when this interrupt occurred.

HP_CIDSEL = 1 -----------|

(vrs type cid sensor)   |

|

UNDSP = 0 ---------------|AND -| VCIDCNT := VCIDCNT + 1

(not underspeed)        |     |  (document number of cid events

|     |   between pip periods)

SYNC_CTR = ENGCYL * 2 ---|     | V_SYNCFLG := 1

(one full rotation of         |  (sync_ctr is indicating the correct

the cam                      |   value when this cid event occurs)

|

| --- ELSE ---

|

HP_CIDSEL = 1 -----------------| VCIDCNT := VCIDCNT + 1

|  (document number of cid events

|   between pip periods)

| V_SYNCFLG := 0

|  (sync_ctr is not indicating the correct

|   value when this cid event occurs or

|   the engine is in underspeed mode)

35-77

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUOUS DIS TEST - CID UP Interrupt Logic

When a Hall type CID sensor is in use, this logic  documents  this  interrupt

has  been  executed.  This logic also indicates whether or not the engine was

in sync when this interrupt occurred.

HP_CIDSEL = 0 -----------|

(hall type cid sensor)  |

|

UNDSP = 0 ---------------|AND -| VCIDCNT := VCIDCNT + 1

(not underspeed)        |     |  (document number of cid events

|     |   between pip periods)

SYNC_CTR = ENGCYL * 2 ---|     | V_SYNCFLG := 1

(one full rotation of         |  (sync_ctr is indicating the correct

the cam)                     |   value when this cid event occurs)

|

| --- ELSE ---

|

HP_CIDSEL = 0 -----------------| VCIDCNT := VCIDCNT + 1

|  (document number of cid events

|   between pip periods)

| V_SYNCFLG := 0

|  (sync_ctr is not indicating the

|   correct value when this cid event

|   occurs or the engine is in

|   underspeed mode)

35-78

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUOUS DIS TEST - Background Logic

CCM_TST_ENA = 1 -----------| Do: CIDBG

| Do: DRIVER_ACTION_PROCEDURE

| Do: PIPBG

| Do: IDMBG

| Do: PRIMARY_COIL_COUNTER_CHK

| Do: PRIMARY_COIL_FILTER_CHK

| Do: CODE_1356_CHK

| Do: CODE_1357_CHK

|

| --- ELSE ---

PUTMR > 4(sec) ------|     |

|AND -| Do: CODE_1356_CHK

CRKFLG = 1 ----------|     | Do: DRIVER_ACTION_PROCEDURE

|

| --- ELSE ---

|

| Do: DRIVER_ACTION_PROCEDURE

35-79

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: CIDBG

VCIDSW = 1 ----------------------|

(cid test cal'd in)             |

|

VPIPFLG = 0 ---------------------|

(no intermittent pip fault)     |

|

PIPTSTFIL = 0 -------------------|AND -| CIDTSTFIL := CIDTSTFIL + (CIDUP *

(no other pip fault indications)|     |  VCIDERRCNT) - VCIDPASCNT

|     | VCIDERRCNT := 0

N > V_CIDN_MIN ------------------|     | VCIDPASCNT := 0

(rpm window)                    |     | Do: CID_FILLVL_CHK

|     |  (determine if cid service code

N < V_CIDN_MAX ------------------|     |   is to be set)

|

| --- ELSE ---

|

VCIDSW = 1 ----------------------------| VCIDPASCNT := 0

| VCIDERRCNT := 0

| VCID_BYPS := 1

END: CIDBG

BEGIN: CID_FILLVL_CHK

When the filter exceeds its level, the service code is set.

CIDTSTFIL > CIDTSTLVL -------------| malfunction(ccm,P0340)

|

| --- ELSE ---

|

| clear_malf(P0340)

CIDTSTFIL > CIDLVLEXP -------------| FFG_CID := 1

(failure indication defined       |

by external user thru cal        |

of CIDLVLEXP)                    |

| --- ELSE ---

|

| FFG_CID := 0

END: CID_FILLVL_CHK

35-80

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUOUS DIS TEST - Background Logic

This logic provides a mechanism for determining an action that  may  cause  a

vehicle  to stall has occurred.  The first action that may cause a vehicle to

stall is the application  of  brakes.   On  a  manual  transmission,  braking

without  depressing the clutch may cause a stall.  Also, releasing the clutch

abruptly may cause a stall.   When  either  one  of  the  above  actions  are

performed  by  the  driver  the  timer V_CL_BRK_TMR is reset (to zero).  This

timer is then used in tests (e.g., hard fault  pip  test)  that  may  require

knowledge  of  these  driver actions that may cause a vehicle to stall.  This

helps prevent false indication  of  failure  (i.e.,  pip  failure)  should  a

vehicle stall occur due to driver actions.

BEGIN: DRIVER_ACTION_PROCEDURE

BIFLG = 1 ------------------------|

(brake on)                       |

|

BIHP = 0 -------------------------|

(no brake input h/w)             |

|

P0703MALF = 1 --------------------|

(boo failure)                    |

|OR --| V_CL_BRK_TMR := 0

TRLOAD = < 2 ---------|           |     | V_PRE_CPP := FFG_CPP

(manual;1/0 swtchs)  |           |     |

|           |     |

P0704MALF = 1 --------|           |     |

(cpp failure)        |OR --|     |     |

|     |     |     |

V_PRE_CPP = 0 --|     |     |     |     |

|AND -|     |AND -|     |

FFG_CPP = 1 ----|           |           |

|           |

IGNR_CPP = 1 ---------------|           |

(clutch input cal'd in)                |

| --- ELSE ---

|

| V_PRE_CPP := FFG_CPP

END: DRIVER_ACTION_PROCEDURE

35-81

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUOUS DIS TEST - Background Logic

BEGIN: PIPBG

DT23S < VPMAX --------------|

(rpm limit via dt23s)      |AND -| Do: INTERMITTENT_PIP_CHK

|     | Do: PIP_FILLVL_CHK

TSLPIP <= V_PIP_HARD -------|     |  (determine if service code is to be set)

(not a hard fault)               | VPIPERRCNT := 0

| VPIPPASCNT := 0

|

| --- ELSE ---

|

DT23S < VPMAX --------------------| IDM_BYPASS := 1

(rpm based on dt23s indicates    | TSLIDM := 0

sufficient rpm to test pip the  |  (tslidm becomes a "time since

last time a pip period was able |   pip has been restored"

to be calculated.  dt23s        |   timer until tslidm > vdly6 when

maintains its last calculated   |   tslidm takes on its normal roll)

value)                          | Do: HARDFAULT_PIP_CHK

| VPIPERRCNT := 0

| VPIPPASCNT := 0

|   (in this decision block, a

|    potential hard fault pip

|    failure has been indicated)

|

| --- ELSE ---

|

| VPIPERRCNT:= 0

| VPIPPASCNT:= 0

END: PIPBG

35-82

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUOUS DIS TEST - Background Logic

BEGIN: INTERMITTENT_PIP_CHK

VPIPERRCNT = 0 ----| PIPTSTFIL := PIPTSTFIL + (PIPUP * VPIPERRCNT) -

(no foreground     |  VPIPPASCNT

pip errors        |  (pip fault filter modified)

indicated)        |

| --- ELSE ---

|

| IDM_BYPASS := 1

|  (pip failure is indicated, bypass idm test)

| TSLIDM := 0

|  (tslidm becomes a "time since pip has been

|   restored" timer until tslidm > vdly6 when

|   tslidm takes on its normal roll)

| PIPTSTFIL := PIPTSTFIL + (PIPUP * VPIPERRCNT) -

|  VPIPPASCNT

|   (pip fault filter modified)

END: INTERMITTENT_PIP_CHK

BEGIN: PIP_FILLVL_CHK

Service code is set when the filter exceeds the level.

PIPTSTFIL > PIPTSTLVL --------| malfunction(ccm,P0320)

|

| --- ELSE ---

|

| clear_malf(P0320)

PIPTSTFIL > PIPLVLEXP --------| FFG_PIP := 1

(failure indication defined  |

by external user thru cal   |

of PIPLVLEXP)               |

| --- ELSE ---

|

| FFG_PIP := 0

END: PIP_FILLVL_CHK

35-83

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUOUS DIS TEST - Background Logic

BEGIN: HARDFAULT_PIP_CHK

V_CL_BRK_TMR >= V_CL_BRK_TM --| malfunction(ccm,P0320)

(no driver actions have      | FFG_PIP := 1

occurred that may appear    |

as a pip failure)           |

| --- ELSE ---

|

| clear_malf(P0320)

| FFG_PIP := 0

END: HARDFAULT_PIP_CHK

BEGIN: IDMBG

IDM_BYPASS = 1 -------------| Do: IDM_BYPASS_TMR_CHK

(idm test is temporarily   | V_PAC_A_CTR := 0

bypassed due to an        | V_PAC_B_CTR := 0

indicated pip failure)    | V_PAC_C_CTR := 0

| V_PAC_D_CTR := 0

| V_PAC_U_CTR := 0

| PIPIDM := 0

| VIDMERRCNT := 0

| VIDMPASCNT := 0

|

| --- ELSE ---

|

VIDMERRCNT > 0 ------------| Do: IDM_NOISE_CHK

(foreground is indicating | Do: IDM_FILLVL_CHK

idm failures)            | V_PAC_A_CTR := 0    (coil failure registers

| V_PAC_B_CTR := 0     are cleared since the

| V_PAC_C_CTR := 0     foreground is

| V_PAC_D_CTR := 0     indicating that an idm

| V_PAC_U_CTR := 0     failure is present)

| PIPIDM := 0

| VIDMERRCNT := 0

| VIDMPASCNT := 0

|

| --- ELSE ---

|

| Do: IDM_NOISE_CHK

| Do: IDM_FILLVL_CHK

| VIDMERRCNT := 0

| VIDMPASCNT := 0

END: IDMBG

35-84

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUOUS DIS TEST - Background Logic

BEGIN: IDM_BYPASS_TMR_CHK

TSLIDM > VDLY6 -----------| IDM_BYPASS := 0

(pip has been restored   | TSLIDM := 0

sufficiently long, the  | V_IDM_CTR := 0

idm test may now be     |

resumed)                | --- ELSE ---

|

| V_IDM_CTR := 0

END: IDM_BYPASS_TMR_CHK

CONTINUOUS DIS TEST - Background Logic

BEGIN: IDM_NOISE_CHK

The PIPIDM register is incremented on  PIP  and  decremented  on  IDM.   This

procedure  is performed only when the PIP is known to be good.  Therefore any

extra IDMs relative to the number of PIPs is an indication that either  noise

has  entered  the IDM circuit or that an intermittent IDM circuit failure has

occurred and the intermittent is sufficiently rapid to cause  more  than  one

IDM interrupt per PIP.

PIPIDM < V_IDM_NOISE -----| IDMFIL := IDMFIL + IDMUP

(more idms than pips     |  (modify the idm fault filter)

have been received)     | PIPIDM := 0

|

| --- ELSE ---

|

| IDMFIL := IDMFIL + IDMUP * VIDMERRCNT -

|  VIDMPASCNT

|  (at this point there may or may not have

|   been any idm failures, the idm filter

|   will be modified according to the values of

|   vidmerrcnt and vidmpascnt during this

|   background pass)

END: IDM_NOISE_CHK

BEGIN: IDM_FILLVL_CHK

Service code is set when the filter exceeds the level.

IDMFIL > IDMLVL ---------| malfunction(ccm,P1351)

|

| --- ELSE ---

|

| clear_malf(P1351)

END: IDM_FILLVL_CHK

35-85

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUOUS DIS TEST - Background Logic

BEGIN: PRIMARY_COIL_COUNTER_CHK

Coil primary fault filters are controlled.

V_PAC_A_CTR > V_1PAC_ERR ---------------| COILAPFIL := COILAPFIL + CPRIMUP

(sufficient coil A failures recognized | V_PAC_A_CTR := 0

in foreground to cause the coil A     |

fault filter to increment)            |

| --- ELSE ---

|

| COILAPFIL := COILAPFIL - 1

V_PAC_B_CTR > V_1PAC_ERR ---------------| COILBPFIL := COILBPFIL + CPRIMUP

(sufficient coil B failures recognized | V_PAC_B_CTR := 0

in foreground to cause the coil B     |

fault filter to increment)            |

| --- ELSE ---

|

| COILBPFIL := COILBPFIL - 1

V_PAC_C_CTR > V_1PAC_ERR ---------------| COILCPFIL := COILCPFIL + CPRIMUP

(sufficient coil C failures recognized | V_PAC_C_CTR := 0

in foreground to cause the coil C     |

fault filter to increment)            |

| --- ELSE ---

|

| COILCPFIL := COILCPFIL - 1

V_PAC_D_CTR > V_1PAC_ERR ---------------| COILDPFIL := COILDPFIL + CPRIMUP

(sufficient coil D failures recognized | V_PAC_D_CTR := 0

in foreground to cause the coil D     |

fault filter to increment)            |

| --- ELSE ---

|

| COILDPFIL := COILDPFIL - 1

V_PAC_U_CTR > V_1PAC_ERR ---------------| COILUFIL := COILUFIL + CPRIMUP

(sufficient coil U failures recognized | V_PAC_U_CTR := 0

in foreground to cause the coil U     |

fault filter to increment)            |

| --- ELSE ---

|

| COILUFIL := COILUFIL - 1

END: PRIMARY_COIL_COUNTER_CHK

35-86

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUOUS DIS TEST - Background Logic

BEGIN: PRIMARY_COIL_FILTER_CHK

Primary coil fault filters are checked.  If the filter exceeds the primary

coil failure threshold CPRIMLVL a service code is stored.

COILAPFIL > CPRIMLVL ---| malfunction(P0351)

|

| --- ELSE ---

|

| clear_malf(P0351)

COILBPFIL > CPRIMLVL ---| malfunction(P0352)

|

| --- ELSE ---

|

| clear_malf(P0352)

COILCPFIL > CPRIMLVL ---| malfunction(P0353)

|

| --- ELSE ---

|

| clear_malf(P0353)

COILDPFIL > CPRIMLVL ---| malfunction(P0354)

|

| --- ELSE ---

|

| clear_malf(P0354)

COILUFIL > CPRIMLVL  ---| malfunction(P0350)

|

| --- ELSE ---

|

| clear_malf(P0350)

END: PRIMARY_COIL_FILTER_CHK

35-87

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CONTINUOUS DIS TEST - Background Logic

BEGIN: CODE_1356_CHK

CODE_1356_FLG = 1 ----------------------| malfunction(ccm,P1356)

(pips have occurred while edis         |

indicates the engine was not turning) | CODE_1356_FLG := 0

|

| --- ELSE ---

|

| clear_malf(P1356)

END: CODE_1356_CHK

BEGIN: CODE_1357_CHK

CODE_1357_FLG = 1 ----------------------| malfunction(ccm,P1357)

(unexpected idm pw's have been         |

received)                             | CODE_1357_FLG := 0

|

| --- ELSE ---

|

| clear_malf(P1357)

END: CODE_1357_CHK

35-88

CONTINUOUS SELF TEST, DISTRIBUTORLESS IGNITION SYSTEM TEST - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

KEY ON ENGINE SELF-TEST OF THE EDIS MODULE

The following test outputs a engine off malfunction code P1358,

if the CPU OK signal (64 us) from the EDIS module is not present.

;begin Key On Engine Off (KOEO) self-test of the edis module.

CCM_EO_ENA = 1 ------------| Do: EO_EDIS_TEST

BEGIN: EO_EDIS_TEST

V_CPUOK_ENA = 1 -----------|

|AND -| malfunction(ccm,P1358)

VKOEO_CPUOK = 0 -----------|     |

| --- ELSE ---

|

| clear_malf(P1358)

END: EO_EDIS_TEST

;end engine off self-test of the edis module

35-89

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

35.3  PROFILE CORRECTION LOGIC (CDAN2)

OVERVIEW

Profile correction refers to mathematically  compensating  for  pip  hardware

deviations  of  a  pip  period  from  360/ENGCYL  degrees  of  rotation.  The

fundamental basis for calculating the correction factor is the fact that  360

degrees  of rotation is accurate on the wheel.  That is the edge representing

0 degrees and the  edge  representing  360  degrees  are  one  in  the  same.

Therefore  a  reference 360 degrees of rotation is available.  The correction

factor is a correction to the velocity calculation.

delta theta / delta time = velocity

where theta is assumed to be 360/ENGCYL.  Corrected velocity is given by:

(delta theta / delta time) * CF = velocity

where CF (one for each pip period in a rotation) is given by:

CF = delta theta actual / delta theta nominal

Under the assumption that the actual  velocity  over  the  pip  period  under

consideration  (the period for which CF is being calculated) and the velocity

over a rotation (360 degrees of crank rotation) containing the pip period  in

question are equal, then:

CF = (delta time over the pip period / delta time over the rotation) * ENGCYL

The adaptive profile correction factors learning is divided into categories:

-  Continuos Learning (To select this feature set CF_MTR_IGNR = 1)

-  Learn until CFs are mature (To select this feature set CF_MTR_IGNR = 0)

Continuous Learning:

During the PIP interrupt, data collection of the previous  pip  down  to  pip

down  period  for the calculation of profile correction factors is performed.

The ENGCYL * 2 - 1 periods are held  in  the  array  CF_PERIOD[].   The  most

recent  period  is  contained  in  the  array  register  CF_PERIOD_1 with the

previously stored periods shifting up one position in the array.  The  oldest

period   is  then  discarded.   Also  the  value  of  SYNC_CTR  is  saved  in

CFSYNCCTRFG.  There are sufficient periods of PIP down to PIP down that every

profile  may  have  a  correction  factor  generated for it.  Having SYNC_CTR

allows  associating  the  periods  with  actual  profiles.   The   background

procedures  then,  when  conditions permit, calculate a correction factor for

each profile from the current data.  These correction  factors  are  averaged

into  the values of the correction factors previously processed and stored in

KAM.

35-90

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Learn until CFs are mature:

During the PIP interrupt the data is collected as described in  the  previous

section.   PIP_EVNT_CTR is incremented by 1 every PIP interrupt.  If DFSFLG =

0 then PIP_EVNT_CTR is reset to  zero  and  CF_DFS0  is  also  set  to  zero.

CF_DFSO  is  set to 1 only if DFSFLG=1 and after a calibratable number of PIP

events (PIP_EVNT_MAX) have elapsed.  This is to make certain that CF learning

is  done  only  after DFSO mode is 'stabilized'.  When conditions permit, the

correction  factors  are  calculated  by  averaging  CFx_NEW_RAW  values  for

CF_DLY_MIN   number  of  events  the  very  first  time  after  power-up  and

CF_SKIP_MIN number of events until CFs are mature.  For a greater control  of

the  step  size  of  correction  factors,  two filter constants are provided.

Namely, CF_FK_FAST and CF_FK_SLOW.  If CF_FAST_THRS  >=  |CFx  -  CFx_AV|  >=

CF_SLOW_THRS,  a slow filter constant (CF_FK_SLOW) is used in the calculation

of CFx_AV and CFx values and if |CFx - CFx_AV| >  CF_FAST_FAST  then  a  fast

filter  constant  (CF_FK_FAST)  is used.  When |CFx - CFx_AV| < CF_SLOW_THRS,

the CFs are considered mature, CF_MTR_FLG,  CF_KAM_MTR  are  set  to  1,  and

profile correction factors will not be updated until next power-up.  If a KAM

reset occurs, all the KAM variables  are  initialized  to  their  appropriate

values (see strategy ladder logic for details).

35-91

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DEFINITIONS

Registers:

-  CF_DLY_CNT = A counter to count the initial (right after power-up) number

of events of CFx_NEW values used in calculating CFx_AV.

-  CF_FC =  This  register  either  contains  the  value  of  CF_FK_FAST  or

CF_FK_SLOW when in 'learn until mature' mode and it contains the value of

CF_FK when in continuous mode.

-  CF_FDGE_FCTR = Profile CF normalizing (i.e.  causes sum of CFs  to  equal

4) factor.

-  CF_LOOPS = Number of positions  in  the  profile  correction  data  array

(cf_tm_stmp[]) remaining to be filled.

-  CF_PERIOD_1 = Pip period  1 in profile correction que of periods.

-  CF_PERIOD_2 = Pip period  2 in profile correction que of periods.

-  CF_PERIOD_3 = Pip period  3 in profile correction que of periods.

-  CF_PERIOD_4 = Pip period  4 in profile correction que of periods.

-  CF_PERIOD_5 = Pip period  5 in profile correction que of periods.

-  CF_PERIOD_6 = Pip period  6 in profile correction que of periods.

-  CF_PERIOD_7 = Pip period  7 in profile correction que of periods.

-  CFSYNCCTRFG = Saved value of sync_ctr in profile correction foreground

-  CF_SYNC_CTR = Value of SYNC_CTR the last time data was entered  into  the

profile correction factor array.

-  CF_SKIP_CNT = A counter to count the number of events of  CFx_NEW  values

used  in  calculating CFx_AV (before CFx_AV is used to compare with CFx).

This reset to zero after CF_SKIP_MIN counts.   If  feature  of  averaging

CFx_AV values before comparing with CF needs to be turned off, a value of

zero should be written to CF_SKIP_MIN.

-  CF1 = Filtered correction factor for profile 1, stored in KAM.

-  CF2 = Filtered correction factor for profile 2, stored in KAM.

-  CF3 = Filtered correction factor for profile 3, stored in KAM.

-  CF4 = Filtered correction factor for profile 4, stored in KAM.

-  CF1_AV = Rolling average value of CF1_NEW.  Initialize to CF1.

-  CF2_AV = Rolling average value of CF2_NEW.  Initialize to CF2.

-  CF3_AV = Rolling average value of CF3_NEW.  Initialize to CF3.

-  CF4_AV = Rolling average value of CF4_NEW.  Initialize to CF4.

35-92

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  CF1_NEW = Corrected new value of profile 1 correction factor.

-  CF2_NEW = Corrected new value of profile 2 correction factor.

-  CF3_NEW = Corrected new value of profile 3 correction factor.

-  CF4_NEW = Corrected new value of profile 4 correction factor.

-  CF1_NEW_RAW = New raw value of profile 1 correction factor.

-  CF2_NEW_RAW = New raw value of profile 2 correction factor.

-  CF3_NEW_RAW = New raw value of profile 3 correction factor.

-  CF4_NEW_RAW = New raw value of profile 4 correction factor.

-  LOAD = Universal LOAD as ratio of air charge over standard.

-  N = RPM.

-  NOISE_COUNT = When non-zero no assessment of misfire is made.

-  PIPTSTFIL = Pip circuit failure fault filter.

-  PIP_DWN_DEL = AICE period as read by the EEC, AICE ticks.

-  PROFILE = Most recent profile of  the  ENGCYL  profiles  that  will  have

correction factors calculated.

-  PIP_EVNT_CTR = Incremented every PIP event.  It is reset to zero if  DFSO

mode is off.

-  SYNC_CTR = Sequence counter incremented every PIP  rising  edge  that  is

used to indentify the cycle and position.

-  TOTMISFIL = Rolling average filter for misfire events.

-  TQ_BRAKE_S = Actual net torque produced by engine.

-  VSBAR = Filtered vehicle speed.

Bit Flags:

-  CF_DFSO = When equal to 1, enables profile correction to learn correction

factors  in  a  decel_fuel_shut_off  mode.   This  flag  is  set  in  the

background logic after counting (pip_evnt_ctr) a calibratable  number  of

pip events (pip_evnt_max) in DFSO.  It is cleared when DFSO mode is off.

-  CF_FC_FLG = Flag indicating continued use of slow filter constant;  1  ->

using slow filter.

-  CF_KAM_MTR = When equal to 1, indicates that the KAM contains matured

-  CF_MTR_FLG = When equal to 1,  indicates  that  CFs  are  mature  and  no

further learning is required.

-  CF_RDY = When equal to 1 it allows update of profile correction factors.

35-93

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  KAM_ERROR = Indicates Keep Alive RAM is invalid.

|      -  MISFDELTAERR = Misfire Comm.  error indicator.

-  NEW_CF_DATA = Indicates to bg profile logic  that  fg  has  updated  data

array since last bg.

-  PIP_HIGH = PIP input level.

-  V_SYNCFLG = VIP indicator, 1 -> engine in sync.

Calibration Constants:

-  AICE_SETR1_B(bit 3) = AICE setup register "B" value.

-  CF_DFSO_IGNR = If set, enables learning of profile CFs regardless of  the

state  of  DFSO  mode.  If clear, enables learning of profile CFs in DFSO

mode only.

-  CF_DLY_MIN = Number of events CFx_AV values be averaged  the  very  first

after power-up before comparing these values with CFx values.

-  CF_ENA = Switch to turn on adaptive profile correction; 1 -> on.

-  CF_FK = Filter constant for "averaging"  of  profile  CFs  in  continuous

mode.

-  CF_FK_FAST = Fast filter constant for "averaging" of  profile  correction

factors, events.

-  CF_FK_SLOW = Slow filter constant for "averaging" of  profile  correction

factors, events.

-  CF_FAST_THRS = The threshold value of |CFx - CFx_AV|, above which the CFs

will be learned faster and below this value Cfs will be learned slower.

-  CF_FK = Filter constant for "averaging" of  profile  correction  factors,

events.

-  CF1_INIT = Initial value of profile correction factor 1.

-  CF2_INIT = Initial value of profile correction factor 2.

-  CF3_INIT = Initial value of profile correction factor 3.

-  CF4_INIT = Initial value of profile correction factor 4.

-  CF_LOAD_MAX = Max load for profile correction factor updates.

-  CF_LOAD_MIN = Min load for profile correction factor update.

|      -  CF_LOCK_SW = Switch which when set to 1  locks  out  profile  coefficient

|         updates  once  the  profile  KAM  mature  (CF_KAM_MTR) flag is set.  This

|         option  should  be  selected  if  learning  coefficients   under   fueled

|         conditions.   This option should not be selected if learning coefficients

|         in DFSO.

35-94

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

-  CFMAX = Maximum allowed profile correction factor value.

-  CFMIN = Minimum allowed profile correction factor value.

-  CF_MTR_IGNR = If set, enables learning of profile CFs all the  time.   If

clear, stops learning of profile CFs after they are mature.

-  CF_N_MAX = Max RPM for profile correction factor update.

-  CF_N_MIN = Min RPM for profile correction factor update.

-  CF_SLOW_THRS = The threshold value of |CFx - CFx_AV| below which the  CFs

are  considered  mature.  If the result of |CFx - CFx_AV| is greater than

CF_SLOW_THRS and less than CF_FAST_THRS, the CFs will  be  learned  at  a

slower rate.

-  CF_SKIP_MIN = Number of events CFx_AV values be averaged before comparing

these  values  with  CFx values.  Note:  This logic is executed every PIP

event as opposed to CF_DLY_MIN, which only runs the very first time after

power-up.

-  CF_VSBARMAX = Max.  speed for profile correction factor updates.

-  CF_VSBARMIN = Min.  speed for profile correction factor update.

-  CF_MAX_TQ = Max net torque for profile correction update, ft/lb.

-  CF_MIN_TQ = Min net torque for profile correction factor update, ft/lb.

-  ENGCYL = Number of PIP'S per engine revolution; 2 -> 4 cylinder engine (2

PIP/rev)  3  ->  6  cylinder engine (3 PIP/rev) 4 -> 8 cylinder engine (4

PIP/rev).

-  NORMALIZE_CF = When equal to 1, causes correction factors to be forced to

sum to ENGCYL, otherwise CF's are taken array.

-  NOISE_IGNR =  If  set,  enables  learning  of  profile  CFs  under  noisy

conditions.   If  clear,  stops  learning  of  profile  CFs  until  noise

disappears.

-  PIP_EVNT_MAX = When in DFSO, number of PIP events to wait before learning

(i.e., CF_DFSO = 1) profile CFs.

|      -  TOTMIS_THRS = Number of TOTMIS events above  which  CF  updates  will  be

|         disabled.

35-95

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROCESS

STRATEGY MODULE: VC_PROFILE_COM7

BEGIN: PROFILE CORRECTION FACTOR PIP UP LOGIC

(performed during the pip up processing)

PIP_EVNT_CTR := PIP_EVNT_CTR + 1

END: PROFILE CORRECTION FACTOR PIP UP LOGIC

BEGIN: PROFILE_RAM_INIT

In RAM init proc initialize the following parameters as shown below:

unconditionally ------------------------| CF_RDY := 0

| CF_DFSO := 0

| CF_FDGE_FCTR := 0

| NEW_CF_DATA := 0

| CF_SYNC_CTR := 0

| CFSYNCCTRFG := 0

| PROFILE := 0

| PIP_EVNT_CTR := 0

| CF_SKIP_CNT := 0

| CF_DLY_CNT := CF_SKIP_MIN

| CF_KAM_MTRO := CF_KAM_MTR

| CF1_AV := CF1

| CF2_AV := CF2

| CF3_AV := CF3

| CF4_AV := CF4

| CF_PERIOD_1 .. CF_PERIOD_7 := 0

| ;set cf_period_1 thru cf_period_7

| ;to zero

| CF1_NEW_RAW .. CF4_NEW_RAW := 0

| ;set cf1_new_raw thru cf4_new_raw

| ;to zero

| CF_FC_FLG := 0

CF_KAM_MTR = 0 -------------------|

|AND -| CF_FC := CF_FK_FAST

CF_MTR_IGNR = 0 ------------------|     |

| --- ELSE ---

CF_KAM_MTR = 1 -------------------|     |

|AND -| CF_FC := CF_FK_SLOW

CF_MTR_IGNR = 0 ------------------|     |

| --- ELSE ---

|

CF_MTR_IGNR = 1 ------------------------| CF_FC := CF_FK

35-96

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

|     ;

|     ; If CF_LOCK_SW is set to 1, lock out profile coefficient updates

|     ; if the coefficients are already mature.  This option should be

|     ; selected only if profile learning is calibrated to occur under

|     ; fueled mode conditions.

|     ;

|     CF_KAM_MTR = 1 -------------------|

|                                       |AND -| CF_MTR_FLG := 1

|     CF_LOCK_SW = 1 -------------------|     |

|                                             | --- ELSE ---

|                                             |

|                                             | CF_MTR_FLG := 0

END: PROFILE_RAM_INIT

PROFILE CORRECTION FACTOR FOREGROUND LOGIC:

(This logic is executed during the PIP interrupt as directed by the

logic, with the following constraints:

- Execute this logic after PIP_HIGH has been updated and before

PIP_DWN_DEL is updated.

- When executing on PIP up edges, execute this logic before SYNC_CTR

is updated.)

CF_MTR_FLG = 1 -------------|

|AND -|

CF_MTR_IGNR = 0 ------------|     |

(CFs are mature; no need to      |OR --| EXIT FOREGROUND LOGIC

collect data in foreground)     |     |

|     |

NEW_CF_DATA = 1 ------------------|     |

(enough data is collected, stop        |

until BG logic can process it)        |

|

| --- ELSE ---

PIP_HIGH = 0 ---------------------|     |

|     |

CF_RDY = 1 -----------------------|AND -| NEW_CF_DATA := 1

|     |  (indicates to BG that new data is

CF_LOOPS = 0 ---------------------|     |   available for calculating CFs)

| Do: SHIFT_PERIODS

| CFSYNCCTRFG := SYNC_CTR

|  (current value of sync_ctr is

|   saved for BG use in associating

|   CFs with appropriate profile)

|

| --- ELSE ---

PIP_HIGH = 0 ---------------------|     |

|AND -| Do: SHIFT_PERIODS

CF_RDY = 1 -----------------------|     | CF_LOOPS := CF_LOOPS - 1

|  (data array is filling)

35-97

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: SHIFT_PERIODS

Collect the current period, shifting the data registers

and discarding the oldest value.

ENGCYL = 4 -----------------------------| CF_PERIOD_7 := CF_PERIOD_6

| CF_PERIOD_6 := CF_PERIOD_5

| CF_PERIOD_5 := CF_PERIOD_4

| CF_PERIOD_4 := CF_PERIOD_3

| CF_PERIOD_3 := CF_PERIOD_2

| CF_PERIOD_2 := CF_PERIOD_1

| CF_PERIOD_1 := PIP_DWN_DEL

|

| --- ELSE ---

|

ENGCYL = 3 -----------------------------| CF_PERIOD_5 := CF_PERIOD_4

| CF_PERIOD_4 := CF_PERIOD_3

| CF_PERIOD_3 := CF_PERIOD_2

| CF_PERIOD_2 := CF_PERIOD_1

| CF_PERIOD_1 := PIP_DWN_DEL

|

| --- ELSE ---

|

ENGCYL = 2 -----------------------------| CF_PERIOD_3 := CF_PERIOD_2

| CF_PERIOD_2 := CF_PERIOD_1

| CF_PERIOD_1 := PIP_DWN_DEL

END: SHIFT_PERIODS

; End PIP UP interrupt (foreground) processing

35-98

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PROFILE CORRECTION FACTOR BACKGROUND LOGIC:

KAM_ERROR = 0 --------------------|

(no kam reset)                   |AND -| Do: CHECK_DFSO_STATUS

|     | Do: CHECK_CONDITIONS

CF_ENA = 1 -----------------------|     |  (determine if conditions are

(adaptive profile correction           |   valid for calculating CFs)

is turned on)                         |

| Do: RUN_PROFILE_CF_BG

|

| --- ELSE ---

|

| (adaptive correction factor

|  generation has been cal'd out or

|  a KAM reset has occurred.

|  Correction factor registers will

|  contain the initial calibration

|  values of the correction factors.

|

| CF1 := CF1_INIT

| CF2 := CF2_INIT

| CF3 := CF3_INIT

| CF4 := CF4_INIT

| CF_KAM_MTR := 0

| Do: PROFILE_RAM_INIT

35-99

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: CHECK_DFSO_STATUS

DFSFLG = 0 ----------------------------| PIP_EVNT_CTR := 0

(not in dfso)                         | CF_DFSO := 0

|

| -- ELSE ---

|

PIP_EVNT_CTR >= PIP_EVNT_MAX ----------| CF_DFSO := 1

(in dfso)

END: CHECK_DFSO_STATUS

BEGIN: RUN_PROFILE_CF_BG

When data has been updated by the foreground and conditions are present

to calculate correction factors, the correction factors will be updated.

NEW_CF_DATA = 1 ------------------|

(new, valid data                 |

available from FG)              |AND -| Do: UPDATE_CFs

|     | CF_LOOPS := NUMCYL - 1

CF_RDY = 1 -----------------------|     | NEW_CF_DATA := 0

(conditions to calculate               |

accurate CFs present)                 |

| --- ELSE ---

|

CF_RDY = 0 -----------------------------| CF_LOOPS := NUMCYL - 1

| NEW_CF_DATA := 0

END: RUN_PROFILE_CF_BG

35-100

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: CHECK_CONDITIONS

The following conditions must be true before accurate CFs can be

calculated.

MIS_TST_RDY = 1 ------------|

(misfire ready)            |OR --|

|     |

CF_DFSO_IGNR = 0 -----------|     |

(dfso mode cal'd in)             |

|

TOTMISFIL < TOTMIS_THRS ----------|

(acceptable misfire rolav)       |

|

MISFDELTAERR = 0 -----------------|

(no AICE errors indicated)       |

|

CF_DFSO = 1 ----------------|     |

|OR --|

CF_DFSO_IGNR = 1 -----------|     |

(cf learning under               |

dfso not required)             |

|

V_SYNCFLG = 1 --------------------|

(in sync)                        |AND -| CF_RDY := 1

|     |  (conditions indicate that

LOAD <= CF_LOAD_MAX --------------|     |   accurate CFs can be calculated)

|     |

LOAD >= CF_LOAD_MIN --------------|     |

|     |

N <= CF_N_MAX --------------------|     |

|     |

N >= CF_N_MIN --------------------|     |

|     |

TQ_BRAKE_S <= CF_MAX_TQ ----------|     |

|     |

TQ_BRAKE_S >= CF_MIN_TQ ----------|     |

|     |

NOISE_COUNT = 0 ------------|     |     |

|OR --|     |

NOISE_IGNR = 1 -------------|     |     |

(noise check cal'd out)          |     |

|     |

CF_MTR_FLG = 0 -------------|     |     |

(cfs are not mature        |     |     |

cont. learning)          |OR --|     |

|     |     |

CF_MTR_IGNR = 1 ------------|     |     |

(always learn cfs)               |     |

|     |

VSBAR <= CF_VSBARMAX -------------|     |

|     |

VSBAR >= CF_VSBARMIN -------------|     |

| --- ELSE ---

|

| CF_RDY := 0

END: CHECK_CONDITIONS

35-101

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: UPDATE_CFs

AICE_SETR1_B(bit 3) = 1 -----------| CF_SYNC_CTR := CFSYNCCTRFG - 1

;using front mfds or pip sensor    |  ; profile for first CF to be

|  ; calculated is one cylinder event

|  ; previous from the cylinder event

|  ; that data was last gathered

|

| --- ELSE ---

|

| CF_SYNC_CTR := CFSYNCCTRFG - 2

|  ; profile for first CF to be calculated

|  ; is one cylinder event previous from

|  ; the cylinder event that data was last

|  ; back gathered and an additional

|  ; period due to the period read for

|  ; the previous sync_ctr value

ENGCYL = 4 -----------------------------| Do: ADJUST_CF_SYNC_CTR

|  (maintain valid range of sync_ctr)

| Do: DETERMINE_8CYL_PROFILE

|  (determine first profile

|   represented in data set)

| Do: CALC_NEW_8CYL_CFs

|  (calculate CFs from current set

|   of data)

| Do: CALC_NEW_8CYL_CF_AV

|

| --- ELSE ---

|

ENGCYL = 3 -----------------------------| Do: ADJUST_CF_SYNC_CTR

|  (maintain valid range of sync_ctr)

| Do: DETERMINE_6CYL_PROFILE

|  (determine first profile

|   represented in data set)

| Do: CALC_NEW_6CYL_CFs

|  (calculate CFs from current set

|   of data)

| Do: CALC_NEW_6CYL_CF_AV

|

| --- ELSE ---

|

ENGCYL = 2 -----------------------------| Do: ADJUST_CF_SYNC_CTR

|  (maintain valid range of sync_ctr)

| Do: DETERMINE_4CYL_PROFILE

|  (determine first profile

|   represented in data set)

| Do: CALC_NEW_4CYL_CFs

|  (calculate CFs from current set

|   of data)

| Do: CALC_NEW_4CYL_CF_AV

END: UPDATE_CFs

35-102

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: ADJUST_CF_SYNC_CTR

CF_SYNC_CTR < 0 ------------------------| CF_SYNC_CTR := CF_SYNC_CTR + NUMCYL

END: ADJUST_CF_SYNC_CTR

BEGIN: DETERMINE_8CYL_PROFILE

CF_SYNC_CTR = 3 ------------------|

|OR --| PROFILE := 4

CF_SYNC_CTR = 7 ------------------|     |

| --- ELSE ---

CF_SYNC_CTR = 0 ------------------|     |

|OR --| PROFILE := 1

CF_SYNC_CTR = 4 ------------------|     |

| --- ELSE ---

CF_SYNC_CTR = 1 ------------------|     |

|OR --| PROFILE := 2

CF_SYNC_CTR = 5 ------------------|     |

| --- ELSE ---

CF_SYNC_CTR = 2 ------------------|     |

|OR --| PROFILE := 3

CF_SYNC_CTR = 6 ------------------|

END: DETERMINE_8CYL_PROFILE

BEGIN: DETERMINE_6CYL_PROFILE

CF_SYNC_CTR = 3 ------------------|

|OR --| PROFILE := 1

CF_SYNC_CTR = 0 ------------------|     |

| --- ELSE ---

CF_SYNC_CTR = 1 ------------------|     |

|OR --| PROFILE := 2

CF_SYNC_CTR = 4 ------------------|     |

| --- ELSE ---

CF_SYNC_CTR = 2 ------------------|     |

|OR --| PROFILE := 3

CF_SYNC_CTR = 5 ------------------|

END: DETERMINE_6CYL_PROFILE

35-103

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: DETERMINE_4CYL_PROFILE

CF_SYNC_CTR = 1 ------------------|

|OR --| PROFILE := 2

CF_SYNC_CTR = 3 ------------------|     |

| --- ELSE ---

CF_SYNC_CTR = 2 ------------------|     |

|OR --| PROFILE := 1

CF_SYNC_CTR = 0 ------------------|

END: DETERMINE_4CYL_PROFILE

35-104

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: CALC_NEW_8CYL_CFs

Raw correction factors are calculated from the current data set (array of

periods).  According to the assumptions and theory, the correction

factors should sum to ENGCYL.  However, the constant velocity assumption may

be violated resulting in the CF's not summing to zero.  This does not

substantially affect the validity of the raw correction factors. However for

consistency of the CF values, the CF's may be corrected to sum to zero if

the calibration parameter NORMALIZE_CF = 1.

CALC_8CYL_CORRECTED_FACTORS performs this adjustment for 8 cylinder engines.

PROFILE = 1 ----------------------------| CF1_NEW_RAW := (CF_PERIOD_2 /

|                (CF_PERIOD_1 +

|                 CF_PERIOD_2 +

|                 CF_PERIOD_3 +

|                 CF_PERIOD_4)) *

|                 ENGCYL

|

| CF4_NEW_RAW := (CF_PERIOD_3 /

|                (CF_PERIOD_2 +

|                 CF_PERIOD_3 +

|                 CF_PERIOD_4 +

|                 CF_PERIOD_5)) *

|                 ENGCYL

|

| CF3_NEW_RAW := (CF_PERIOD_4 /

|                (CF_PERIOD_3 +

|                 CF_PERIOD_4 +

|                 CF_PERIOD_5 +

|                 CF_PERIOD_6)) *

|                 ENGCYL

|

| CF2_NEW_RAW := (CF_PERIOD_5 /

|                (CF_PERIOD_4 +

|                 CF_PERIOD_5 +

|                 CF_PERIOD_6 +

|                 CF_PERIOD_7)) *

|                 ENGCYL

|

| Do: CALC_8CYL_CORRECTED_FACTORS

|

| --- ELSE ---

(continued on next page)

35-105

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|

PROFILE = 2 ----------------------------| CF2_NEW_RAW := (CF_PERIOD_2 /

|                (CF_PERIOD_1 +

|                 CF_PERIOD_2 +

|                 CF_PERIOD_3 +

|                 CF_PERIOD_4)) *

|                 ENGCYL

|

| CF1_NEW_RAW := (CF_PERIOD_3 /

|                (CF_PERIOD_2 +

|                 CF_PERIOD_3 +

|                 CF_PERIOD_4 +

|                 CF_PERIOD_5)) *

|                 ENGCYL

|

| CF4_NEW_RAW := (CF_PERIOD_4 /

|                (CF_PERIOD_3 +

|                 CF_PERIOD_4 +

|                 CF_PERIOD_5 +

|                 CF_PERIOD_6)) *

|                 ENGCYL

|

| CF3_NEW_RAW := (CF_PERIOD_5 /

|                (CF_PERIOD_4 +

|                 CF_PERIOD_5 +

|                 CF_PERIOD_6 +

|                 CF_PERIOD_7)) *

|                 ENGCYL

|

| Do: CALC_8CYL_CORRECTED_FACTORS

|

| --- ELSE ---

(continued on next page)

35-106

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|

PROFILE = 3 ----------------------------| CF3_NEW_RAW := (CF_PERIOD_2 /

|                (CF_PERIOD_1 +

|                 CF_PERIOD_2 +

|                 CF_PERIOD_3 +

|                 CF_PERIOD_4)) *

|                 ENGCYL

|

| CF2_NEW_RAW := (CF_PERIOD_3 /

|                (CF_PERIOD_2 +

|                 CF_PERIOD_3 +

|                 CF_PERIOD_4 +

|                 CF_PERIOD_5)) *

|                 ENGCYL

|

| CF1_NEW_RAW := (CF_PERIOD_4 /

|                (CF_PERIOD_3 +

|                 CF_PERIOD_4 +

|                 CF_PERIOD_5 +

|                 CF_PERIOD_6)) *

|                 ENGCYL

|

| CF4_NEW_RAW := (CF_PERIOD_5 /

|                (CF_PERIOD_4 +

|                 CF_PERIOD_5 +

|                 CF_PERIOD_6 +

|                 CF_PERIOD_7)) *

|                 ENGCYL

|

| Do: CALC_8CYL_CORRECTED_FACTORS

|

| --- ELSE ---

(continued on next page)

35-107

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|

PROFILE = 4 ----------------------------| CF4_NEW_RAW := (CF_PERIOD_2 /

|                (CF_PERIOD_1 +

|                 CF_PERIOD_2 +

|                 CF_PERIOD_3 +

|                 CF_PERIOD_4)) *

|                 ENGCYL

|

| CF3_NEW_RAW := (CF_PERIOD_3 /

|                (CF_PERIOD_2 +

|                 CF_PERIOD_3 +

|                 CF_PERIOD_4 +

|                 CF_PERIOD_5)) *

|                 ENGCYL

|

| CF2_NEW_RAW := (CF_PERIOD_4 /

|                (CF_PERIOD_3 +

|                 CF_PERIOD_4 +

|                 CF_PERIOD_5 +

|                 CF_PERIOD_6)) *

|                 ENGCYL

|

| CF1_NEW_RAW := (CF_PERIOD_5 /

|                (CF_PERIOD_4 +

|                 CF_PERIOD_5 +

|                 CF_PERIOD_6 +

|                 CF_PERIOD_7)) *

|                 ENGCYL

|

| Do: CALC_8CYL_CORRECTED_FACTORS

END: CALC_NEW_8CYL_CFs

35-108

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: CALC_8CYL_CORRECTED_FACTORS

CF_FDGE_FCTR := ENGCYL / (CF1_NEW_RAW + CF2_NEW_RAW +

CF3_NEW_RAW + CF4_NEW_RAW)

NORMALIZE_CF = 1 -----------------------| CF1_NEW := CF1_NEW_RAW *

|             CF_FDGE_FCTR

| CF2_NEW := CF2_NEW_RAW *

|             CF_FDGE_FCTR

| CF3_NEW := CF3_NEW_RAW *

|             CF_FDGE_FCTR

| CF4_NEW := CF4_NEW_RAW *

|             CF_FDGE_FCTR

|

| --- ELSE ---

|

| CF1_NEW := CF1_NEW_RAW

| CF2_NEW := CF2_NEW_RAW

| CF3_NEW := CF3_NEW_RAW

| CF4_NEW := CF4_NEW_RAW

END: CALC_8CYL_CORRECTED_FACTORS

BEGIN: CALC_NEW_8CYL_CF_AV

CF_MTR_IGNR = 1 ------------------------| Do: USE_CF_NEW_TO_CALC_CFs

| Do: MODIFY_8CYL_CF_FILTERS

|

| --- ELSE ----

CF_MTR_IGNR = 0 ------------------|     |

|AND -| Do: CALC_ENGCYL_CF_AV

|     CF_DLY_CNT =< CF_DLY_MIN ---------|     | CF_DLY_CNT := CF_DLY_CNT + 1

|

| --- ELSE ---

CF_MTR_IGNR = 0 ------------------|     |

|AND -| Do: CALC_ENGCYL_CF_AV

|     CF_SKIP_CNT =< CF_SKIP_MIN -------|     | CF_SKIP_CNT := CF_SKIP_CNT + 1

|

| --- ELSE ---

|

| CF_SKIP_CNT := 0

| Do: CHK_KAM_FOR_8CYL_FK

| Do: CALC_ENGCYL_CF_AV

| Do: DETERMINE_8CYL_CF_MATURITY

END: CALC_NEW_8CYL_CF_AV

35-109

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: CALC_ENGCYL_CF_AV

ENGCYL = 4 ------------------------| CF1_AV :=

|   CF1_AV + CF_FC * (CF1_NEW - CF1_AV)

| CF2_AV :=

|   CF2_AV + CF_FC * (CF2_NEW - CF2_AV)

| CF3_AV :=

|   CF3_AV + CF_FC * (CF3_NEW - CF3_AV)

| CF4_AV :=

|   CF4_AV + CF_FC * (CF4_NEW - CF4_AV)

|

| --- ELSE ---

|

ENGCYL = 3 ------------------------| CF1_AV :=

|   CF1_AV + CF_FC * (CF1_NEW - CF1_AV)

| CF2_AV :=

|   CF2_AV + CF_FC * (CF2_NEW - CF2_AV)

| CF3_AV :=

|   CF3_AV + CF_FC * (CF3_NEW - CF3_AV)

|

| --- ELSE ---

|

ENGCYL = 2 ------------------------| CF1_AV :=

|   CF1_AV + CF_FC * (CF1_NEW - CF1_AV)

| CF2_AV :=

|   CF2_AV + CF_FC * (CF2_NEW - CF2_AV)

END: CALC_ENGCYL_CF_AV

35-110

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: CALC_NEW_6CYL_CFs

Raw correction factors are calculated from the current data set (array of

periods).  According to the assumptions and theory, the correction

factors should sum to ENGCYL.  However, the constant velocity assumption may

be violated resulting in the CF's not summing to zero.  This does not

substantially affect the validity of the raw correction factors. However for

consistency of the CF values, the CF's may be corrected to sum to zero if

the calibration parameter NORMALIZE_CF = 1.

CALC_6CYL_CORRECTED_FACTORS performs this adjustment for 6 cylinder engines.

PROFILE = 1 ----------------------------| CF1_NEW_RAW := (CF_PERIOD_2 /

|                (CF_PERIOD_1 +

|                 CF_PERIOD_2 +

|                 CF_PERIOD_3)) *

|                 ENGCYL

|

| CF3_NEW_RAW := (CF_PERIOD_3 /

|                (CF_PERIOD_2 +

|                 CF_PERIOD_3 +

|                 CF_PERIOD_4)) *

|                 ENGCYL

|

| CF2_NEW_RAW := (CF_PERIOD_4 /

|                (CF_PERIOD_3 +

|                 CF_PERIOD_4 +

|                 CF_PERIOD_5)) *

|                 ENGCYL

|

| Do: CALC_6CYL_CORRECTED_FACTORS

|

| --- ELSE ---

|

PROFILE = 2 ----------------------------| CF2_NEW_RAW := (CF_PERIOD_2 /

|                (CF_PERIOD_1 +

|                 CF_PERIOD_2 +

|                 CF_PERIOD_3)) *

|                 ENGCYL

|

| CF1_NEW_RAW := (CF_PERIOD_3 /

|                (CF_PERIOD_2 +

|                 CF_PERIOD_3 +

|                 CF_PERIOD_4)) *

|                 ENGCYL

|

| CF3_NEW_RAW := (CF_PERIOD_4 /

|                (CF_PERIOD_3 +

|                 CF_PERIOD_4 +

|                 CF_PERIOD_5)) *

|                 ENGCYL

|

| Do: CALC_6CYL_CORRECTED_FACTORS

|

| --- ELSE ---

(continued on next page)

35-111

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

(continued from previous page)

|

PROFILE = 3 ----------------------------| CF3_NEW_RAW := (CF_PERIOD_2 /

|                (CF_PERIOD_1 +

|                 CF_PERIOD_2 +

|                 CF_PERIOD_3)) *

|                 ENGCYL

|

| CF2_NEW_RAW := (CF_PERIOD_3 /

|                (CF_PERIOD_2 +

|                 CF_PERIOD_3 +

|                 CF_PERIOD_4)) *

|                 ENGCYL

|

| CF1_NEW_RAW := (CF_PERIOD_4 /

|                (CF_PERIOD_3 +

|                 CF_PERIOD_4 +

|                 CF_PERIOD_5)) *

|                 ENGCYL

|

| Do: CALC_6CYL_CORRECTED_FACTORS

END: CALC_NEW_6CYL_CFs

35-112

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: CALC_6CYL_CORRECTED_FACTORS

CF_FDGE_FCTR := ENGCYL / (CF1_NEW_RAW + CF2_NEW_RAW +

CF3_NEW_RAW)

NORMALIZE_CF = 1 -----------------------| CF1_NEW := CF1_NEW_RAW *

|             CF_FDGE_FCTR

| CF2_NEW := CF2_NEW_RAW *

|             CF_FDGE_FCTR

| CF3_NEW := CF3_NEW_RAW *

|             CF_FDGE_FCTR

|

| --- ELSE ---

|

| CF1_NEW := CF1_NEW_RAW

| CF2_NEW := CF2_NEW_RAW

| CF3_NEW := CF3_NEW_RAW

END: CALC_6CYL_CORRECTED_FACTORS

BEGIN: CALC_NEW_6CYL_CF_AV

CF_MTR_IGNR = 1 -----------------------| Do: USE_CF_NEW_TO_CALC_CFs

| Do: MODIFY_6CYL_CF_FILTERS

|

| --- ELSE ----

CF_MTR_IGNR = 0 -----------------|     |

|AND -| Do: CALC_ENGCYL_CF_AV

|     CF_DLY_CNT =< CF_DLY_MIN --------|     | CF_DLY_CNT := CF_DLY_CNT + 1

|

| --- ELSE ---

CF_MTR_IGNR = 0 -----------------|     |

|AND -| Do: CALC_ENGCYL_CF_AV

|     CF_SKIP_CNT =< CF_SKIP_MIN ------|     | CF_SKIP_CNT := CF_SKIP_CNT + 1

|

| --- ELSE ---

|

| CF_SKIP_CNT := 0

| Do: CHK_KAM_FOR_6CYL_FK

| Do: CALC_ENGCYL_CF_AV

| Do: DETERMINE_6CYL_CF_MATURITY

END: CALC_NEW_6CYL_CF_AV

35-113

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: CALC_NEW_4CYL_CFs

Raw correction factors are calculated from the current data set (array of

periods).  According to the assumptions and theory, the correction

factors should sum to ENGCYL.  However, the constant velocity assumption may

be violated resulting in the CF's not summing to zero.  This does not

substantially affect the validity of the raw correction factors. However for

consistency of the CF values, the CF's may be corrected to sum to zero if

the calibration parameter NORMALIZE_CF = 1.

CALC_4CYL_CORRECTED_FACTORS performs this adjustment for 4 cylinder engines.

PROFILE = 1 ----------------------------| CF1_NEW_RAW := (CF_PERIOD_2 /

|                (CF_PERIOD_1 +

|                 CF_PERIOD_2)) *

|                 ENGCYL

|

| CF2_NEW_RAW := (CF_PERIOD_3 /

|                (CF_PERIOD_2 +

|                 CF_PERIOD_3)) *

|                 ENGCYL

|

| Do: CALC_4CYL_CORRECTED_FACTORS

|

| --- ELSE ---

|

PROFILE = 2 ----------------------------| CF2_NEW_RAW := (CF_PERIOD_2 /

|                (CF_PERIOD_1 +

|                 CF_PERIOD_2)) *

|                 ENGCYL

|

| CF1_NEW_RAW := (CF_PERIOD_3 /

|                (CF_PERIOD_2 +

|                 CF_PERIOD_3)) *

|                 ENGCYL

|

| Do: CALC_4CYL_CORRECTED_FACTORS

END: CALC_NEW_4CYL_CFs

35-114

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: CALC_4CYL_CORRECTED_FACTORS

CF_FDGE_FCTR := ENGCYL / (CF1_NEW_RAW + CF2_NEW_RAW)

NORMALIZE_CF = 1 -----------------------| CF1_NEW := CF1_NEW_RAW *

|             CF_FDGE_FCTR

| CF2_NEW := CF2_NEW_RAW *

|             CF_FDGE_FCTR

|

| --- ELSE ---

|

| CF1_NEW := CF1_NEW_RAW

| CF2_NEW := CF2_NEW_RAW

END: CALC_4CYL_CORRECTED_FACTORS

BEGIN: CALC_NEW_4CYL_CF_AV

CF_MTR_IGNR = 1 ------------------------| Do: USE_CF_NEW_TO_CALC_CFs

| Do: MODIFY_4CYL_CF_FILTERS

|

| --- ELSE ----

CF_MTR_IGNR = 0 ------------------|     |

|AND -| Do: CALC_ENGCYL_CF_AV

|     CF_DLY_CNT =< CF_DLY_MIN ---------|     | CF_DLY_CNT := CF_DLY_CNT + 1

|

| --- ELSE ----

CF_MTR_IGNR = 0 ------------------|     |

|AND -| Do: CALC_ENGCYL_CF_AV

|     CF_SKIP_CNT =< CF_SKIP_MIN -------|     | CF_SKIP_CNT := CF_SKIP_CNT + 1

|

| --- ELSE ---

|

| CF_SKIP_CNT := 0

| Do: CHK_KAM_FOR_4CYL_FK

| Do: CALC_ENGCYL_CF_AV

| Do: DETERMINE_4CYL_CF_MATURITY

END: CALC_NEW_4CYL_CF_AV

35-115

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: DETERMINE_8CYL_CF_MATURITY

|CF1_AV - CF1| < CF_SLOW_THRS -----|

|

|CF2_AV - CF2| < CF_SLOW_THRS -----|

|AND -| CF_MTR_FLG := 1

|CF3_AV - CF3| < CF_SLOW_THRS -----|     |  (cfs are mature; don't need any

|     |    further learning)

|CF4_AV - CF4| < CF_SLOW_THRS -----|     | CF_KAM_MTR := 1

(compare old cfs with new cfs)          |

| --- ELSE ---

|

| CF_MTR_FLG := 0

| Do: USE_CF_AV_TO_CALC_CFs

| Do: MODIFY_8CYL_CF_FILTERS

|  (cfs are not mature

|       continue learning)

END: DETERMINE_8CYL_CF_MATURITY

BEGIN: DETERMINE_6CYL_CF_MATURITY

|CF1_AV - CF1| < CF_SLOW_THRS -----|

|

|CF2_AV - CF2| < CF_SLOW_THRS -----|AND -| CF_MTR_FLG := 1

|     |  (cfs are mature; don't need any

|CF3_AV - CF3| < CF_SLOW_THRS -----|     |    further learning)

(compare old cfs with new cfs)          | CF_KAM_MTR := 1

|

| --- ELSE ---

|

| CF_MTR_FLG := 0

| Do: USE_CF_AV_TO_CALC_CFs

| Do: MODIFY_6CYL_CF_FILTERS

|  (cfs are not mature

|       continue learning)

END: DETERMINE_6CYL_CF_MATURITY

35-116

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: DETERMINE_4CYL_CF_MATURITY

|CF1_AV - CF1| < CF_SLOW_THRS -----|

|AND -| CF_MTR_FLG := 1

|CF2_AV - CF2| < CF_SLOW_THRS -----|     |  (cfs are mature; don't need any

(compare old   cfs with new cfs)        |    further learning)

| CF_KAM_MTR := 1

|

| --- ELSE ---

|

| CF_MTR_FLG := 0

| Do: USE_CF_AV_TO_CALC_CFs

| Do: MODIFY_4CYL_CF_FILTERS

|  (cfs are not mature

|        continue learning)

END: DETERMINE_4CYL_CF_MATURITY

35-117

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: CHK_KAM_FOR_8CYL_FK

CF_KAM_MTR = 1 -----------------------|

|

CF_FC_FLG = 1 ------------------------|

(slow filter used previously)        |

|OR --| CF_FC := CF_FK_SLOW

|CF1 - CF1_AV| =< CF_FAST_THRS -|     |     |

|     |     | CF_FC_FLG := 1

|CF2 - CF2_AV| =< CF_FAST_THRS -|     |     |  (once slow filter is in

|AND -|     |   use, continue to use it)

|CF3 - CF3_AV| =< CF_FAST_THRS -|           |

|           |

|CF4 - CF4_AV| =< CF_FAST_THRS -|           |

|

|

|

|

|

| --- ELSE ---

|

|

|

| CF_FC := CF_FK_FAST

END: CHK_KAM_FOR_8CYL_FK

BEGIN: CHK_KAM_FOR_6CYL_FK

CF_KAM_MTR = 1 -----------------------|

|

CF_FC_FLG = 1 ------------------------|

(slow filter used previously)        |OR --| CF_FC := CF_FK_SLOW

|     |

|CF1 - CF1_AV| =< CF_FAST_THRS -|     |     | CF_FC_FLG := 1

|     |     |  (once slow filter is in

|CF2 - CF2_AV| =< CF_FAST_THRS -|AND -|     |   use, continue to use it)

|           |

|CF3 - CF3_AV| =< CF_FAST_THRS -|           |

|

| --- ELSE ---

|

|

| CF_FC := CF_FK_FAST

END: CHK_KAM_FOR_6CYL_FK

35-118

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: CHK_KAM_FOR_4CYL_FK

CF_KAM_MTR = 1 -----------------------|

|

CF_FC_FLG = 1 ------------------------|

(slow filter used previously)        |OR --| CF_FC := CF_FK_SLOW

|     |

|CF1 - CF1_AV| =< CF_FAST_THRS -|     |     | CF_FC_FLG := 1

|AND -|     |  (once slow filter is in

|CF2 - CF2_AV| =< CF_FAST_THRS -|           |   use, continue to use it)

|

|

| --- ELSE ---

|

| CF_FC := CF_FK_FAST

END: CHK_KAM_FOR_4CYL_FK

BEGIN: USE_CF_NEW_TO_CALC_CFs

cf1_new := CF1_NEW

cf2_new := CF2_NEW

cf3_new := CF3_NEW

cf4_new := CF4_NEW

CF_FC := CF_FK

END: USE_CF_NEW_TO_CALC_CFs

BEGIN: USE_CF_AV_TO_CALC_CFs

cf1_new := CF1_AV

cf2_new := CF2_AV

cf3_new := CF3_AV

cf4_new := CF4_AV

END: USE_CF_AV_TO_CALC_CFs

35-119

CONTINUOUS SELF TEST, PROFILE CORRECTION LOGIC - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BEGIN: MODIFY_8CYL_CF_FILTERS

cf1_new >= CFMIN -----------------|

|

cf1_new <= CFMAX -----------------|

|

cf2_new  >= CFMIN ----------------|

|

cf2_new  <= CFMAX ----------------|

|AND -| CF1 := CF1 + CF_FC*(cf1_new - CF1)

cf3_new  >= CFMIN ----------------|     |

|     | CF2 := CF2 + CF_FC*(cf2_new - CF2)

cf3_new  <= CFMAX ----------------|     |

|     | CF3 := CF3 + CF_FC*(cf3_new - CF3)

cf4_new  >= CFMIN ----------------|     |

|     | CF4 := CF4 + CF_FC*(cf4_new - CF4)

cf4_new  <= CFMAX ----------------|

END: MODIFY_8CYL_CF_FILTERS

BEGIN: MODIFY_6CYL_CF_FILTERS

cf1_new  >= CFMIN ----------------|

|

cf1_new  <= CFMAX ----------------|

|

cf2_new  >= CFMIN ----------------|

|AND -| CF1 := CF1 + CF_FC*(cf1_new - CF1)

cf2_new  <= CFMAX ----------------|     |

|     | CF2 := CF2 + CF_FC*(cf2_new - CF2)

cf3_new  >= CFMIN ----------------|     |

|     | CF3 := CF3 + CF_FC*(cf3_new - CF3)

cf3_new  <= CFMAX ----------------|

END: MODIFY_6CYL_CF_FILTERS

BEGIN: MODIFY_4CYL_CF_FILTERS

cf1_new  >= CFMIN ----------------|

|

cf1_new  <= CFMAX ----------------|

|AND -| CF1 := CF1 + CF_FC*(cf1_new - CF1)

cf2_new  >= CFMIN ----------------|     |

|     | CF2 := CF2 + CF_FC*(cf2_new - CF2)

cf2_new  <= CFMAX ----------------|

END: MODIFY_4CYL_CF_FILTERS

35-120

CHAPTER 36

ROM IDENTIFICATION CODE

36-1

ROM IDENTIFICATION CODE - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

36.1  ROM IDENTIFICATION CODE (CDAA0)

ROM identification codes are used by both IC and module  suppliers.   The  IC

suppliers require a means of identifying ROM chip contents quickly since they

produce different calibration bit patterns on the  same  wafer.   The  module

suppliers  utilize  these  codes to insure that the ROM/module combination is

correct.

In the past, the ROM identification codes (CALID and VERID) were generated by

hand.   They  were  then  distributed  to  Engine  Systems  to  put  in their

calibrations for Cert.  If any change to the Cert calibration was made  or  a

different strategy used, new values had to be generated and calibrated in.

For 1988 and beyond, the procedure has been  changed  to  make  this  process

easier.   The  new  process  removes  CALID  and replaces it with ROM_TO.  In

addition, VERID has been deleted and a new parameter "FIXSUM" has been added.

FIXSUM should always be set to 0.  Specifically:

1.  The non-modifiable Vector  parameter  "ROM_TO"  replaces  the  old  CALID

parameter  as  the ROM chip identifier.  The ROM_TO value is generated by

Vector during a calibration release and is located  at  200A  HEX.   This

value  is  the complement of the ROM pattern CHECKSUM and is also used to

perform the EEC-IV diagnostic "CHECKSUM Memory Test".

2.  The new parameter "FIXSUM" is a Vector calibration parameter  located  at

2004  HEX  and should always be set to 0.  This parameter will be used to

assure the ROM_TO values are unique and will only be changed by the  SWDV

engineer if a duplicate ROM_TO value is found.

ROM IDENTIFICATION CODE PROCEDURES

1.  PEDD SW will set the value of the calibration parameter FIXSUM  to  0  in

the base release.

2.  When Engine Systems releases a CERT calibration, the ROM  chip  ID  code,

ROM_TO, will be automatically generated by VECTOR, and the value is to be

recorded on the calibration release sheet submitted to SWDV.

3.  PEDD SWDV will verify that the ROM_TO value is not the same as any  other

previous  ROM_TO  prior to sending the binary file to EED.  If the ROM_TO

value matches another, PEDD SWDV will change  the  value  of  FIXSUM  and

generate  a  new  ROM_TO  value, which will be checked again for a match.

This process is repeated until a unique ROM_TO value is generated.

4.  For production calibrations only, EED systems will receive and record the

value of ROM_TO for final ROM verification.

5.  EED will then transmit the binary file to the  vendors,  verify  the  ROM

chip  against  the  binary  file,  verify  the  CHECKSUM,  verify the RAM

read/write test, and verify the ROM_TO value and location.

36-2

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

INDEX

01, 23-136                          ACC_STATUS, 4-34, 14-17, 14-19

06, 5-20, 5-43 to 5-44, 5-78,       ACC_STATUS (READ), 5-137

5-117, 8-310, 12-27 to 12-28,   ACCEL_LST[], 35-14 to 35-15

12-31, 12-35, 12-37, 12-41 to   ACCEL_POINTR, 35-15, 35-20, 35-32

12-42, 17-10, 17-28, 35-46      ACCEL_SAVE, 35-14

07, 5-76                            ACCFLG, 10-13, 13-74, 14-9, 14-16

2, 24-17                                to 14-17, 25-17, 28-60, 35-27

ACCFLG (READ), 5-133

A/C CLUTCH CONTROL (CDAN0), 14-4    ACCFLG_LAST, 14-17

A/C CLUTCH CONTROL TIMERS (CDAN0),  ACCLFG_LAST, 14-17

14-11                           ACCS, 4-20

A/C CLUTCH DIAGNOSTICS (CDAN2),     ACCS_OLD, 35-27

14-20                           ACCSTMR, 3-8

A/C CLUTCH INPUT (CDAN0), 14-2      ACECT_CL, 14-8

A/C CLUTCH OUTPUT (CDAN0), 14-14    ACECT_FLG, 14-8, 14-10

A/C ENABLE CONTROL (CDAK0), 25-68   ACECT_SH, 14-8

A/C FAN CONTROL (CDAL0), 16-18      ACFILTC, 13-25

A_RATE_ENA, 5-9 to 5-10, 5-12,      ACIOTMR, 13-25, 14-10, 14-13,

5-63, 5-70                          14-17, 14-21, 16-4, 16-20

ABORT_DONE, 11-13 to 11-14          ACIOTMR_LAST, 14-21

abs (FCALL), 8-307, 8-319           ACITMR, 14-12, 14-16, 25-72, 31-7

AC PRESSURE (ACP) SWITCH INPUT      ACL, 4-35, 20-3

SIGNAL PROCESSING (CDAN0),      ACLOD, 13-74

31-90                           ACMNDT_TP, 14-8

AC_CTL_OFF, 25-72                   ACNHI_CL, 14-8

AC_CTL_TM, 25-72                    ACNHI_FLG, 14-8, 14-10

AC_CTL_TMR, 25-71 to 25-72          ACNHI_SH, 14-8

AC_CYCLE_FLG, 13-54, 14-16 to       ACNLO_CH, 14-8

14-17                           ACNLO_FLG, 14-8, 14-10

AC_EN_TRANS, 14-12, 25-71 to        ACNLO_SL, 14-8

25-72, 28-88                    ACPERIOD, 14-21

AC_MN_DT, 14-10                     ACPERIOD_TMR, 14-21

AC_MN_FT, 14-17, 16-4, 16-20        ACPPM CALCULATION (CDAL0), 13-53

AC_NEW_RPM, 14-10                   ACPRE_HSF_CL, 31-8

AC_OFF_DLY, 13-25                   ACPRE_HSF_SH, 31-8

AC_OFF_PRES, 31-5, 31-7             ACPRES, 13-54, 16-20, 31-5, 31-7

AC_PPM, 13-49, 13-52, 13-54             to 31-8

AC_PPM_RQST, 13-43, 13-54, 14-16    ACPRES_CL, 31-8

AC_PPM_TMR, 13-54                   ACPRES_FLG, 14-10, 31-8

AC_PRES_DLTA, 31-7                  ACPRES_FMEM, 31-7

AC_PRES_TM, 31-7                    ACPRES_HSF, 16-8 to 16-9, 31-5,

AC_RPM_FLG, 13-25                       31-8

AC_SLP_TMR, 25-71 to 25-72          ACPRES_SH, 31-8

AC_SLP_WDOG, 25-72                  ACPRES_TST, 31-5, 31-7

AC_STATE_ENT, 28-60                 ACPRESBAR, 16-20

ACC OUTPUT STATE MONITOR (CDAN0),   ACPSEN_HP, 4-8, 13-54, 31-5

14-18                           ACPSTM, 14-10

ACC_DELAY, 14-16                    ACPSW, 4-8, 16-8 to 16-9, 31-90

ACC_DELAY2, 14-16                   ACPSW (READ), 5-133

ACC_KAM_FLG, 14-9 to 14-10, 14-19,  ACPSW_HP, 4-8, 31-90

31-5                            ACR, 4-34, 14-16 to 14-17

Index-1

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

ACRQST, 13-25, 14-9 to 14-10,       Adaptive Fuel Control and OBD-II

14-12 to 14-13, 14-16, 16-4,        Fuel System Monitor -

16-20, 16-27, 25-71, 31-5           V1.5_FADPT (CDAN0), 8-61

ACRT, 3-7                           ADAPTIVE FUEL CONTROL LOGIC

ACSLIP, 28-8                            (CDAN0), 8-61

ACSTRD, 14-10                       ADAPTIVE FUEL ENABLE LOGIC

ACSTRT, 12-6, 31-14                     (CDAN0), 8-84

ACSTRT_ACCUM, 6-6, 31-14            ADAPTIVE_FUEL_CONTROL

ACSW, 14-3, 14-10                     DEFINITION, 8-70

ACSW (READ), 5-133                  ADAPTM, 8-88

ACT, 3-8, 8-44, 9-18, 9-58, 9-62,   ADAPTM (READ), 15-9

10-10 to 10-11, 10-13, 10-24,   ADEFTR, 8-88

11-33, 12-45, 13-54, 15-65,     ADEGCT, 8-74

15-73, 15-90, 17-10, 23-26 to   ADJUSTABLE SHOCK ABSORBER OUTPUT

23-28, 23-43 to 23-44, 24-26,       (CDAB0), 20-2

24-33, 28-6, 29-7, 29-11,       ADP_KAM_MAX, 15-8

31-11, 31-13 to 31-14, 35-28    ADP_KAM_MIN, 15-8

READ, 8-88                        ADP_LAM_MAX, 15-8

ACT (READ), 5-144, 8-115 to 8-116,  ADP_LAM_MIN, 15-8

8-136                           ADP_STATE11, 11-173 to 11-174

ACT_CATEGOMX, 11-33                 ADP_STATE11 (READ), 15-18

ACT_CNTS, 4-8, 31-11                ADP_STATE21, 11-173 to 11-174

ACT_CNTS (READ), 5-144              ADP_STATE21 (READ), 15-18

ACT_CODES, 31-11                    ADP_TM, 15-8

ACT_EGR, 12-38                      ADP_TM_MAX (READ), 15-8

ACT_ENG, 31-11 to 31-13, 31-34      ADPLOD, 8-88

ACT_ER_TM, 31-12                    ADPTMR

ACT_ER_TMR, 31-11 to 31-13            READ, 8-88

ACT_FIL, 23-40 to 23-43               WRITE, 8-87

ACT_FIL_MAX, 23-35, 23-43           ADPTMR (READ), 15-9

ACT_FIL_MAX (READ), 5-145           ADPTMR_FLG

ACT_FM_TM, 31-13                      WRITE, 8-87

ACT_FM_TMR, 31-12 to 31-13          ADPTP, 8-88

ACT_FMEM, 31-14                     ADT1FMFLG, 15-73

ACT_FRST_FL, 31-13                    WRITE, 8-99

ACT_HI_ER, 31-12                    ADT1FMFLG (READ), 5-132

ACT_INT, 23-40 to 23-41, 23-43      ADT2FMFLG, 15-73

ACT_LO_ER, 31-12 to 31-13             WRITE, 8-99

ACT_LVL, 23-42                      ADT2FMFLG (READ), 5-132

ACT_MIN, 3-8                        AETV, 27-28

ACT_TC, 31-13                       AETVTP, 27-28

ACTOFF (READ), 8-115                AFACT1, 8-88

ACTOFF (WRITE), 8-115 to 8-116      AFACT2, 8-88

ACTP_CL, 14-12                      AFECT1, 8-87

ACTP_FLG, 14-8, 14-10, 16-4         AFECT2, 8-87

ACTP_SH, 14-8                       AFMFLG, 10-9, 11-109, 12-7, 31-12

ACTPOFF_RQST, 14-8, 14-12               to 31-13

ACTPOFF_TMR, 14-8, 14-12              READ, 8-88

ACTPTMR2, 3-8, 14-8, 14-12          AFMFLG (READ), 5-132

ACTUAL PERCENT EGR (CDAA0), 12-25   AFT1_ADRS, 5-131

ADAPT_SEL, 8-72, 8-95               AFT1_ADRS (READ), 5-145

ADAPT_TMR (READ), 15-8              AFT2_ADRS, 5-131

ADAPT_TMR (WRITE), 15-6, 15-9       AFT2_ADRS (READ), 5-145

AHISL, 8-136

AHISL (READ), 8-136

Index-2

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

AHISL_TICK, 8-136, 8-231            AIR_FK, 24-14 to 24-15

AHISL_TICK (READ), 8-180 to 8-181   AIR_FK_LAST, 24-14 to 24-15

AICE ANALOG-TO-DIGITAL INPUTS       AIR_K_OLD, 24-3, 24-14 to 24-15,

(AICE A/D), 4-19                    24-18

AICE EXTERNAL ANALOG-TO-DIGITAL     AIR_MAN_VOL, 24-26

INPUTS (AICE E-A/D), 4-20       AIR_NUM_MULT, 24-3, 24-14, 24-26

AICE_SET1_B, 35-16                  AIR_PIP_CNT, 24-7 to 24-8

AICE_SETR1_B, 4-18, 35-13           AIR_PIP_INT, 6-7

AICE_SETR1_C, 4-18                  AIR_STRT, 24-30

AICE_SETR1_D, 4-18                  AIR_TM_OLD, 24-38

AICE_VERS, 4-18                     AIR_WOT_ANT, 24-3, 24-20, 24-26

AICE_VRS_VAL, 4-18                  AIRMETER OVERVIEW (CDAM0), 24-2

Air Charge -                        AISF, 8-101

V3.1_AIR_CHARGE_97_IMRC         AISFLO, 8-106, 8-110

(CDAM0), 24-2                   AISFM, 8-106

AIR CHARGE ANTICIPATION (CDAM0),    AISFU, 8-106, 8-110

24-16                           ALOSL, 8-136

AIR CHARGE BACKGROUND               ALOSL_TICK, 8-136, 8-231

CALCULATIONS (CDAM0), 24-21     ALOSL_TICK (READ), 8-180

AIR CHARGE TEMPERATURE SENSOR       ALPHAMIS, 35-35, 35-42, 35-46

INPUT AND SELFTEST (CDAN2),     ALPHANOCALL, 35-54

31-9                            ALT_CAL_DIST, 22-5

Air Conditioning Control Strategy   ALT_CAL_FLG, 8-18, 8-45, 8-137,

(ACC) - V2.3_ACC (CDAN0),           9-14, 9-37, 13-13 to 13-14,

14-2                                13-18, 13-46, 22-3, 22-5

AIR LEAKAGE CALCULATION (CDAM0),    ALT_DFSO_FLG (READ), 8-129

24-31                           ALT_DFSO_FLG (WRITE), 8-129

AIR_ACEL, 24-3                      ALT_PIP_MODE

AIR_B3_N, 24-3, 24-14, 24-26          READ, 8-110

AIR_BF_FLG, 24-3, 24-15, 24-27 to     WRITE, 8-172

24-28                           AM, 8-45, 8-146, 9-20, 9-22,

AIR_BG_FIL, 24-3, 24-14, 24-30          10-11 to 10-13, 10-32, 12-11,

AIR_BIFLGLST, 24-38                     12-25, 12-36 to 12-37, 15-20,

AIR_BRK, 24-33, 24-38                   15-71, 15-74, 23-26 to 23-27,

AIR_BRK_0, 24-38                        24-59, 31-75

AIR_BRK_DCY, 24-38                    READ, 8-72, 8-95

AIR_BRK_DUR, 24-38                  AM (READ), 8-116, 15-19

AIR_BRK_ET, 24-38                   AM1_MAX, 10-32

AIR_BRK_INIT, 24-38                 AM1_MIN, 10-32

AIR_BRK_SW, 24-38                   AM2, 3-3

AIR_CHG, 12-39, 23-28, 24-3,        AM2_MAX, 10-32

24-14 to 24-15, 24-18, 24-20,   AM2_MIN, 10-32

24-26, 24-29                    AM3_MAX, 10-32

AIR_CHG_COMP, 24-9, 24-33           AM3_MIN, 10-32

AIR_CHG_CUR, 12-39, 24-3, 24-9,     AM4_MAX, 10-32

24-14 to 24-15, 24-18, 24-33    AM4_MIN, 10-32

AIR_CHG_LEAK, 24-3                  AM5_MAX, 10-32

AIR_CHG_PRV, 24-9, 24-18, 24-20     AM5_MIN, 10-32

AIR_CHG_ST, 24-3, 24-9              AM6_MAX, 10-32

AIR_CHG_W, 24-3                     AM6_MIN, 10-32

AIR_CHG_WOT, 3-8, 24-3, 24-15,      AM7_MAX, 10-32

24-20, 24-26                    AM7_MIN, 10-32

AIR_CRK_INT, 6-7, 24-29             AM8_MAX, 10-32

AIR_DEC_MUL, 24-14                  AM8_MIN, 10-32

AIR_DOWN_CAL, 8-171, 8-223, 24-8    AM9_MAX, 10-32

Index-3

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

AM9_MIN, 10-32                      ASILT_BR_SW, 27-55

AM_CELL, 10-32 to 10-36             ATBYS, 17-10

AM_INT_CUR_X, 24-7, 24-9            ATBYS_CL, 17-10

AM_INT_PRV_X, 24-7, 24-9            ATMR1, 8-14, 8-49, 8-107, 8-121,

AMB_ACC_SUB, 10-13                      8-242, 8-251, 11-109, 13-19

AMB_ACT_TOL, 10-13                      to 13-20, 14-10, 15-71, 17-18,

AMB_DELTA, 10-13                        23-138, 24-30, 31-80

AMB_ECT_MIN, 10-13                  ATMR1 (READ), 5-144, 8-297

AMB_KEON_DLT, 10-10                 ATMR1_HI_RES, 8-14, 8-44, 8-251,

AMB_VS_MIN, 10-13                       10-24

AMP_MULT, 8-257                     ATMR1_LST, 8-242, 8-251

AMPEM, 12-25                        ATMR2, 8-14, 8-121, 8-251, 15-71

AMPMUL, 8-74                        ATMR3, 8-199, 8-252, 9-20, 9-22,

ANALOG A/C HEAD PRESSURE INPUT          13-14, 13-52, 23-21 to 23-22,

(CDAN0), 31-2                       23-133

ANALOG INPUT CHANNELS (ACH), 4-7    ATMR3 (READ), 8-115 to 8-116

ANALOG INPUT CONDITIONER            ATMR4, 8-14, 15-20

CALIBRATION (AICE), 4-18

ANIPIP1, 8-278                      BACKGROUND FUEL (CDAM0), 8-133

ANIPIP2, 8-278                      BACKGROUND INJECTOR CUTOUT

ANPIP, 8-273, 8-275                     (CDAM0), 8-232

ANPIP1, 3-8, 8-275, 8-278, 29-31    BACKGROUND TRANSIENT FUEL LOGIC

ANPIP1 (WRITE), 8-55 to 8-58            (CDAM0), 8-100

ANPIP2, 3-8, 8-275, 8-278, 29-31    bank (READ), 8-300, 8-304, 8-307,

ANPIP2 (WRITE), 8-55 to 8-58            8-309, 8-314, 8-316, 8-318 to

ANTI-SILTING ENABLING LOGIC             8-319

(CDAN0), 27-54                  BANK1_EXPLIC, 35-36 to 35-37,

ANTI_THEFT_STATUS Message (CDAK0),      35-40

5-111                           BANK1_EXPON, 35-36 to 35-37,

ANTICIPATION, 24-18                     35-40

AODE INPUT EQUATIONS (CDAA0),       BANK2_EXPLIC, 35-36, 35-38, 35-40

31-15                           BANK2_EXPON, 35-36, 35-38, 35-40

AODE SHIFT CONTROL OVERVIEW         bank_num, 8-185 to 8-186, 8-199,

(CDAN0), 25-53                      8-202 to 8-203, 8-208 to

AODE TRANSMISSION CALCULATIONS          8-209, 8-219

(CDAL0), 25-31                  bank_num (READ), 8-212 to 8-213

APT, 3-8, 7-2 to 7-4, 8-15 to       BAPFMM, 29-9, 29-11, 29-14, 32-4

8-16, 8-24, 8-49 to 8-50,           to 32-5

8-132, 8-244, 8-246 to 8-247,   BAROMETRIC PRESSURE - INFERRED

8-252, 9-20, 9-22, 9-46,            (CDAL0), 29-7

10-23, 10-33, 10-35, 11-50,     BAROMETRIC PRESSURE FILTER

12-6 to 12-7, 12-16, 13-9,          (CDAL0), 29-13

13-21, 13-73, 13-89 to 13-90,   BASE EMISSIONS AND DRIVE SPARK

15-20, 15-70, 18-11, 23-26,         (CDAL0), 9-11

29-24, 30-4, 31-71, 31-74       BATTERY VOLTAGE (VBAT) (CDAL0),

APT (READ), 5-144, 8-56, 13-80          31-88

ARCANTCOR, 24-26                    BCSDC, 25-63, 28-30, 28-49, 28-56,

ARCH_PROP_D, 12-39                      28-84

ARCHFLG, 24-3, 24-14, 24-30         BCSDC (READ), 5-144

ARCHGQ1_FLG, 24-28                  BCSDC_HRD, 28-56

ARCHIDLE_SW, 24-30                  BCSDC_LST, 28-49, 28-51

ARCHLK, 24-33                       BCSDC_OUT, 4-17, 28-49 to 28-51

ARCHQ1_FLG, 24-28                   BCV_OFF_CNT, 25-65 to 25-66

ARCHSW, 24-30                       BCV_OFF_LVL, 25-66

ARCWOTCOR, 24-26                    BCV_ON_CNT, 25-65 to 25-66

Index-4

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BCV_ON_LVL, 25-66                   BITMAP_FMEM3

BCVOFFDN, 25-65                       PGM_CVS_FM (READ), 5-132

BCVOFFUP, 25-65                       PGM_PFS_FM (READ), 5-132

BCVONDN, 25-65                        PGM_TPR_FM (READ), 5-132

BCVONUP, 25-65                      BITMAP_FMEM3 (READ), 5-146

BFMFLG, 12-7, 29-10                 BITMAP_FMEMx, 5-130

BFMFLG (WRITE), 29-11 to 29-12      BITMAP_KAM_1

BG_TMR, 8-45, 9-27, 9-40, 10-23,      DOL (READ), 5-136

11-143, 11-145, 11-148 to         MIS_MON_KAM (READ), 5-136

11-149, 13-41, 16-22, 16-25,    BITMAP_KAM_1 (READ), 5-145

22-5, 25-4, 25-24, 28-79 to     BITMAP_MON_1

28-80, 28-88, 29-11, 29-23,       EAMM (READ), 5-136

31-89, 35-29                      F_PUMP_S_MON (READ), 5-136

BG_TMR (READ), 8-27, 8-295, 8-309,  BITMAP_MON_1 (READ), 5-144

8-316                           BITMAP_MON_x, 5-130

BGACFLG, 24-9, 24-29 to 24-30       BITMAP_OFD_1

BIAS_G1, 8-274, 8-277, 8-281,         INJ1_FAULT (READ), 5-136

8-284, 11-187, 17-19              INJ2_FAULT (READ), 5-136

READ, 8-75                          INJ3_FAULT (READ), 5-136

BIAS_G1 (READ), 8-55 to 8-58          INJ4_FAULT (READ), 5-136

BIAS_G2, 8-277, 8-281, 8-284,         INJ5_FAULT (READ), 5-136

11-187, 17-19                     INJ6_FAULT (READ), 5-136

READ, 8-75                          INJ7_FAULT (READ), 5-136

BIAS_G2 (READ), 8-55 to 8-58          INJ8_FAULT (READ), 5-136

BIAS_G_RES (DEFINED), 8-318         BITMAP_OFD_1 (READ), 5-146

BIAS_G_RES (READ), 8-319            BITMAP_OFD_2

BIAS_Gx, 8-275                        ACC_STATUS (READ), 5-137

BIAS_MAX, 8-281                       EVR_OPEN (READ), 5-137

BIAS_MIN, 8-281                       EVR_SHORT (READ), 5-137

BIAS_SUM (FCALL), 8-316, 8-319        F_PUMP_ERROR (READ), 5-137

BIFLG, 24-38, 27-55, 28-15, 31-19     FP_2SPD_ERR (READ), 5-137

to 31-20, 31-72, 35-81            IAC_NO_CUR (READ), 5-137

BIFLG (READ), 5-133                   IAC_OVER_CUR (READ), 5-137

BIFLG_LST, 28-15, 31-19               MIL_FAULT (READ), 5-137

BIHP, 24-38, 31-19 to 31-20,          scp_162E_b7 (READ), 5-137

31-72, 35-81                    BITMAP_OFD_2 (READ), 5-146

BITMAP_FMEM1                        BITMAP_OFD_3

AFMFLG (READ), 5-132                CANP_FAULT (READ), 5-138

CFMFLG (READ), 5-132                DOL_FAULT (READ), 5-138

EFMFLG (READ), 5-132                EAM_FAULT (READ), 5-138

MFMFLG (READ), 5-132                EDF_STATUS (READ), 5-138

OFMFLG (READ), 5-132                HEDF_STATUS (READ), 5-138

TFMFLG (READ), 5-132                IMRC_FAULT (READ), 5-138

VSFMFLG (READ), 5-132               scp_162F_b2 (READ), 5-138

BITMAP_FMEM1 (READ), 5-144          BITMAP_OFD_3 (READ), 5-146

BITMAP_FMEM2                        BITMAP_OFD_4

ADT1FMFLG (READ), 5-132             CANVT_FAULT (READ), 5-139

ADT2FMFLG (READ), 5-132             scp_1630_b3 (READ), 5-139

EGO1FMFLG (READ), 5-132           BITMAP_OFD_4 (READ), 5-146

EGO2FMFLG (READ), 5-132           BITMAP_OFD_5

FFG_CID (READ), 5-132               MCC_H_FAULT (READ), 5-141

FLG_MLUS_FM (READ), 5-132           scp_1632_b7 (READ), 5-141

OSFMFLG (READ), 5-132               SS1_FAULT (READ), 5-141

TOT_FM_FLG (READ), 5-132            SS2_FAULT (READ), 5-141

BITMAP_FMEM2 (READ), 5-144            TCIL_FAULT (READ), 5-141

Index-5

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

BITMAP_OFD_5 (READ), 5-146          CADP_RES_TM, 8-97 to 8-98

BKPT_PW, 8-136, 8-231               CALCLP, 8-228

BOO_CODES, 31-18                    CALCULATE DYNAMIC FUEL CORRECTION

BOO_LVL, 4-20, 31-19                    (CDAI0), 8-220

BORDERLINE DETONATION SPARK         CALCULATION OF LAMAVE (CDAA0),

(CDAN0), 9-15                       8-29

BP, 8-49, 9-14, 9-18, 12-19,        CALCULATION OF SAE J1979 LOAD

12-36, 12-45, 13-75, 24-26,         PARAMETER (CDAM0), 24-65

24-29, 24-59, 25-34, 28-16,     CALIBRATION CLEAR LOGIC (CDAI0),

29-14, 32-4 to 32-5                 22-4

BP (READ), 5-144, 8-136             CALIBRATION INITIALIZE LOGIC

BP (WRITE), 29-9                        (CDAI0), 22-2

BP_CAL_DES, 29-9                    CANISTER PURGE FLOW TEST (CDAN2),

BP_EGRMON, 12-45                        15-86

BP_INTR, 3-8, 25-34, 26-10          CANISTER PURGE MONITOR (CDAN2),

BP_OK_ACC, 29-10                        15-37

BP_OK_ACC (WRITE), 29-11 to 29-12   CANISTER PURGE OBDII TEST SELECT

BP_OK_TM, 29-12                         (CDAJ0), 15-84

BP_WORD, 12-19                      CANISTER PURGE SELECT (CDAJ0),

BPMAX, 29-14, 32-5                      15-85

BPMIN, 29-14, 32-5                  CANISTER PURGE SOLENOID FAULT

BPSSW, 29-9                             TEST LOGIC (CDAJ0), 15-80

BPUNMH, 28-16                       CANISTER PURGE STRATEGY (CDAM0),

BPUNMN, 28-16                           15-66

BPUPDENTM, 29-11                    CANISTER PURGE STRATEGY (PCOMP)

BPUPDTTMR, 29-9, 29-11                  (CDAN0), 15-10

BRAKE COMPENSATION (CDAM0), 24-34   CANP, 3-3

BRAKE SWITCH INPUT AND SELF TEST    CANP_FAULT, 4-35, 15-83

(CDAM0), 31-17                  CANP_FAULT (READ), 5-138 to 5-139

BRAKE_DNDT, 13-67                   CANP_FF, 15-35 to 15-36, 15-83

BRK_AWAY_CNT, 28-60, 28-62          CANP_OSM_TM, 15-83

BRK_AWAY_LVL, 28-62                 CANP_OSM_TMR, 15-83

BRKRLCKTM, 28-18                    CANP_PRD_DT, 4-46

BTR_BASE, 25-51                     CANP_THRES, 15-36, 15-83

BYLDST, 24-63                       CANVNT_CMD, 4-35

BYLDST_INST, 24-63                  CANVT_CMD, 4-46 to 4-48

BYMAST, 24-63                       CANVT_FAULT, 4-35

BYPASS CLUTCH CONTROL (CDAM0),      CANVT_FAULT (READ), 5-139

28-2                            CAT1_FAILED, 10-38, 10-41

BYPASS CLUTCH CONTROL TESTS         CAT2_FAILED, 10-40 to 10-41

(CDAK0), 28-29                  CAT_ACT_MAX, 10-24

BYPLES1, 11-167                     CAT_ACT_MIN, 10-24

BYPLES_TM1, 11-118, 11-167,         CAT_ACT_TF1, 10-25

11-179                          CAT_AMPCAL12, 10-28

BYPLES_TM2, 11-167, 11-179          CAT_AMPCAL22, 10-29

BYPS_BRKPNT, 35-33                  CAT_APT_FLG, 10-33, 10-35

BYPS_LIST, 35-21                    CAT_AQU_LS1, 10-33

BZZRPM, 13-18                       CAT_AQU_LS2, 10-35

BZZRPM_ALT, 13-18                   CAT_AQU_TMR1, 10-33

BZZTM_ALT, 13-18                    CAT_AQU_TMR2, 10-35

CAT_CHKD, 23-69, 23-71 to 23-72

1C, 23-136                          CAT_COLD_TME, 10-24

C, 8-44                             CAT_COUNT, 23-69, 23-71

C17xFIL, 11-116                     CAT_CTPT_TME, 10-33

C17xLVL, 11-116

Index-6

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CAT_DN1_FLG, 10-25, 10-33, 10-37    CATALYST_MONITOR (Standard) -

to 10-38, 10-41                     V1.8A_CAT_FTP_MONITOR (CDAN0),

CAT_DN2_FLG, 10-25, 10-35, 10-39        10-15

to 10-41                        CC_FM_FLG, 27-14, 28-17, 28-53,

CAT_ECT_MAX, 10-24                      28-56

CAT_ECT_MIN, 10-24                  CC_FM_LVL, 28-53

CAT_ER_DONE, 23-185                 CC_NO_DL, 25-63

CAT_FMEM, 23-17, 23-35, 23-37       CC_OFF_CNT, 25-64 to 25-65

CAT_HOT_TME, 10-24                  CC_OFF_LVL, 25-65

CAT_KWU_CNT, 10-38, 10-40           CC_ON_CNT, 25-64 to 25-65

CAT_LOW_LOAD, 10-24                 CC_ON_LVL, 25-65

CAT_LOW_TP, 10-24                   CC_SR_DH, 25-63

CAT_MAX, 8-28                       CC_SR_DL, 25-63

CAT_MAX_CL, 8-28                    CC_TOT_DH, 25-63

CAT_MAX_CLR, 8-28                   CC_TOT_DL, 25-63

CAT_MAX_MID, 10-24                  CC_TP_DH, 25-63

CAT_MAX_SAFE, 8-28, 8-45            CC_TP_DL, 25-63

CAT_MED_TME, 10-24                  CC_TP_SS, 25-63

CAT_MIL_ON, 23-50, 23-79, 23-110    CC_TQ_DH, 25-63

CAT_MIL_SW, 23-110                  CC_TQ_DL, 25-63

CAT_MIN_MID, 10-24                  CC_TQ_SS, 25-63

CAT_MON, 10-25, 10-41, 23-41 to     CC_TQ_SS1, 25-63

23-42, 23-44, 23-51, 23-68 to   CC_TRIP_DONE, 23-162 to 23-163

23-69, 23-71 to 23-72           CC_TRIP_SW, 23-162

CAT_MON_LST, 23-44, 23-51           ccm_codes, 23-137

CAT_MON_LST := 0, 23-71             CCM_E0_ENA, 31-11

CAT_MX_LEAN, 11-50                  CCM_ECT_MON, 23-138, 31-38

CAT_READY, 23-69 to 23-71, 23-135   CCM_EO_DONE, 23-169

cat_reset, 23-52                    CCM_EO_ENA, 8-156, 8-160, 11-11

CAT_STBLE_TME, 10-33, 10-35             to 11-12, 12-33, 12-35, 12-37,

CAT_SW_AMP12, 10-28                     13-85, 14-3, 14-19, 15-35,

CAT_SW_AMP22, 10-29                     15-82, 16-16, 16-30 to 16-31,

CAT_SWTFL11, 10-30, 10-33 to            18-8, 18-19, 18-22, 23-166 to

10-34                               23-167, 23-169 to 23-170,

CAT_SWTFL12, 10-30, 10-34, 10-36        24-40, 25-43, 25-57, 25-85,

CAT_SWTFL21, 10-31, 10-35 to            27-47 to 27-48, 31-6, 31-12,

10-36                               31-20, 31-31 to 31-32, 31-40,

CAT_SWTFL22, 10-31                      31-44, 31-53 to 31-54, 31-67,

CAT_TEMP_SW, 8-28                       35-89

CAT_TIER1, 10-38, 10-40             CCM_EO_WAIT, 23-169

CAT_TM_CL, 10-24                    CCM_ER_ENA, 8-152, 8-156, 8-160,

CAT_TM_STRT, 10-24                      9-97, 11-11, 11-88, 12-16,

CAT_TMR_ACT, 10-25, 10-38, 10-40        12-33, 12-37, 13-85, 14-3,

CAT_TP_LAST, 10-23                      14-19, 15-26, 15-35, 15-77,

CAT_TP_MAX, 10-33, 10-35                15-82, 16-16, 16-30, 17-22,

CAT_TP_RATE, 10-23, 10-33, 10-35        17-26, 17-29, 18-8 to 18-9,

CAT_TP_TC, 10-23                        23-181, 23-185 to 23-186,

CAT_TST_ENA, 10-41, 23-16 to            23-190, 24-40, 25-14, 31-6,

23-17                               31-11 to 31-12, 31-31 to

CAT_TST_PRG, 10-25                      31-32, 31-53 to 31-54, 31-60,

CAT_TST_RDY, 10-24 to 10-25,            31-67

23-41                           CCM_FMEM, 23-17, 23-35, 23-39

CAT_TST_SW, 23-17, 23-51, 23-68     CCM_IAC_MON, 23-138

to 23-69, 23-135                CCM_IAC_MON (WRITE), 13-80

CAT_TST_TMR, 10-23 to 10-24         CCM_MIL_ON, 23-50, 23-79, 23-110

Index-7

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CCM_MIL_SW, 23-110                  CF3_INIT, 35-18, 35-99

CCM_MON, 15-50, 23-44, 23-51 to     CF3_NEW, 35-109 to 35-110, 35-113,

23-52, 23-68, 23-71 to 23-72,       35-119

23-138                          CF3_NEW_RAW, 35-96, 35-105 to

CCM_MON_LST, 23-44, 23-51 to            35-109, 35-111 to 35-113

23-52                           CF4, 35-18, 35-96, 35-99, 35-116,

CCM_MON_LST := 0, 23-71                 35-118, 35-120

CCM_MON_TM, 23-138                  CF4_AV, 35-96, 35-110, 35-116,

CCM_TRNS_MON, 23-138, 23-163            35-118 to 35-119

CCM_TST_ENA, 8-152, 8-156, 8-160,   CF4_INIT, 35-18, 35-99

8-327, 10-25, 11-11, 11-88,     CF4_NEW, 35-109 to 35-110, 35-119

11-108, 13-85, 14-19, 14-21,    CF4_NEW_RAW, 35-96, 35-105,

15-26, 15-35, 15-77, 15-82,         35-107 to 35-109

15-92, 15-103, 16-16, 16-30,    CF_DFSO, 35-96, 35-100 to 35-101

17-22, 17-26, 17-29, 18-8 to    CF_DFSO_IGNR, 35-101

18-9, 23-16 to 23-17, 24-40,    CF_DLY_CNT, 35-96, 35-109, 35-113,

25-43, 25-85, 25-94, 25-98,         35-115

26-50, 26-58, 27-48, 28-51,     CF_DLY_MIN, 35-109, 35-113,

28-54, 28-60, 31-6, 31-11 to        35-115

31-12, 31-31 to 31-32, 31-38,   CF_ENA, 35-99

31-44, 31-53 to 31-54, 31-60,   CF_FAST_THRS, 35-118 to 35-119

31-67, 35-67 to 35-68, 35-79    CF_FC, 35-96, 35-110, 35-118 to

CCM_TST_ENA (READ), 13-79, 27-53        35-120

CCM_TST_ENA(r), 9-97                CF_FC_FLG, 35-96, 35-118 to

CCM_TST_RDY, 23-17, 23-138              35-119

CCM_TST_SW, 15-50, 16-30, 23-17,    CF_FDGE_FCTR, 35-96, 35-109,

23-68, 23-135, 25-85                35-113, 35-115

CCOFFDN, 25-64                      CF_FK, 35-96, 35-119

CCOFFUP, 25-64                      CF_FK_FAST, 35-96, 35-118 to

CCONDN, 25-64                           35-119

CCONUP, 25-64                       CF_FK_SLOW, 35-96, 35-118 to

CCSLP_MN_PRD, 14-21                     35-119

CCV_STRT_SFT, 26-58                 CF_KAM_IGNR, 35-26, 35-34 to

CF1, 35-18, 35-96, 35-99, 35-116        35-35, 35-41

to 35-120                       CF_KAM_MTR, 35-34 to 35-35, 35-41,

CF1_AV, 35-96, 35-110, 35-116 to        35-96 to 35-97, 35-99, 35-116,

35-119                              35-118 to 35-119

CF1_INIT, 35-18, 35-99              CF_KAM_MTR (READ), 8-129

CF1_NEW, 35-109 to 35-110, 35-113,  CF_KAM_MTRO, 35-26, 35-34, 35-96

35-115, 35-119                  CF_LOAD_MAX, 35-101

CF1_NEW_RAW, 35-96, 35-105 to       CF_LOAD_MIN, 35-101

35-109, 35-111 to 35-115        CF_LOCK_SW, 35-97

CF2, 35-18, 35-96, 35-99, 35-116    CF_LOOPS, 35-97, 35-100

to 35-120                       CF_MAX_TQ, 35-101

CF2_AV, 35-96, 35-110, 35-116 to    CF_MIN_TQ, 35-101

35-119                          CF_MTR_FLG, 35-97, 35-101, 35-116

CF2_INIT, 35-18, 35-99                  to 35-117

CF2_NEW, 35-109 to 35-110, 35-113,  CF_MTR_IGNR, 35-96 to 35-97,

35-115, 35-119                      35-101, 35-109, 35-113,

CF2_NEW_RAW, 35-96, 35-105 to           35-115

35-109, 35-111 to 35-115        CF_N_MAX, 35-101

CF3, 35-18, 35-96, 35-99, 35-116,   CF_N_MIN, 35-101

35-118, 35-120                  CF_PERIOD_1, 35-96, 35-98, 35-105

CF3_AV, 35-96, 35-110, 35-116,          to 35-108, 35-111 to 35-112,

35-118 to 35-119                    35-114

Index-8

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CF_PERIOD_2, 35-96, 35-98, 35-105   CHKAIR, 8-24, 17-14, 17-18 to

to 35-108, 35-111 to 35-112,        17-19, 17-21, 17-30 to 17-31

35-114                          CHKSUM

CF_PERIOD_3, 35-96, 35-98, 35-105     READ, 8-71, 8-76 to 8-77

to 35-108, 35-111 to 35-112,      WRITE, 8-71, 8-76 to 8-78

35-114                          CHTIL, 4-44

CF_PERIOD_4, 35-96, 35-98, 35-105   CHTIL_CMD, 4-44

to 35-106, 35-108, 35-111 to    CID, 23-27

35-112                          CID_HIGH, 4-11, 8-168

CF_PERIOD_5, 35-96, 35-98, 35-105   CID_HIGH (READ), 5-133

to 35-108, 35-111 to 35-112     CIDLVLEXP, 35-80

CF_PERIOD_6, 35-96, 35-98, 35-105   CIDTSTFIL, 35-80

to 35-108                       CIDTSTLVL, 35-80

CF_PERIOD_7, 35-96, 35-98, 35-105   CIDUP, 35-80

to 35-108                       CINTSW, 8-122

CF_RDY, 35-96 to 35-97, 35-100 to   CINTV, 8-122

35-101                          CIR, 5-26

CF_SKIP_CNT, 35-109, 35-113,        cl_init_mil(CL_MIL_ON), 23-51

35-115                          cl_malfunction(CL_MIL_ON, Pxxx),

CF_SKIP_MIN, 35-96, 35-113,             23-50

35-115                          CL_MIL_ON, 23-50 to 23-51, 23-103

CF_SLOW_THRS, 35-116 to 35-117          to 23-104, 23-110

CF_SYNC_CTR, 35-96, 35-103 to       CL_MIL_SW, 23-110

35-104                          CL_OBDI_SW, 23-103 to 23-104,

CF_VSBARMAX, 35-101                     23-116 to 23-117

CF_VSBARMIN, 35-101                 cl_reset, 23-52

CFMAX, 35-120                       CL_SEEN, 23-86, 23-96, 23-133

CFMFLG, 5-96, 10-9, 11-109, 12-7,   cl_seen_init, 23-51

14-8, 16-4, 16-8, 16-22, 29-6,  CL_THRTL, 7-3 to 7-4

31-32 to 31-33                  CL_TMR, 15-46 to 15-48, 15-90 to

READ, 8-87                            15-91, 15-100, 15-102

CFMFLG (READ), 5-132                CLDCTM, 26-48

CFMIN, 35-120                       clear-malf (FCALL), 27-53

CFSYNCCTRFG, 35-96 to 35-97         clear_malf

CHANGE_FUELPW, 8-194                  CALL, 8-97 to 8-98

CHANGE_FUELPW (WRITE), 8-179,       clear_malf(P0442), 15-61

8-181                           clear_malf(P0443), 15-36, 15-83

CHANGED PAGES, 1, 3, 1-2, 4-2,      clear_malf(P1443), 15-105

8-53, 8-57, 8-279 to 8-281,     clear_similar_conds(Pxxx), 23-86

8-288 to 8-319, 14-20 to            to 23-87, 23-95 to 23-97

14-22, 15-40, 15-43, 15-45,     CLLS_AT_PT, 9-20, 9-22

15-86 to 15-88, 15-90 to        CLOCK, 9-91

15-92, 17-4, 17-7, 17-28,       clock, 8-193, 8-219

23-149 to 23-151, 23-155 to     CLOCK_SEC, 24-37, 31-89

23-159, 23-177, 23-182,         CLOCK_SEC (CDAL0), 31-89

23-185, 23-190, 26-4, 29-2 to   CLOSED LOOP ENTRY MIL CONTROL

29-6, 31-9, 31-12 to 31-13,         (CDAK0), 23-99

31-62 to 31-69, 31-71 to        Closed Loop Fuel Control -

31-75, 31-91 to 31-92, 35-4,        V2.8C_CLFUL_NONFLEX_96

35-11, 35-29, 35-94 to 35-95,       (CDAN2), 8-259

35-97, 35-109, 35-113, 35-115   CLOSED LOOP LAMBSE CALCULATION

check_fuel_conds                        (CDAI0), 8-282

CALL, 8-97 to 8-98                CLOSED LOOP LAMBSE CALCULATION

CHGTM, 13-21                            (CDAN2), 8-279

CHK_VPLG_FLG, 12-40 to 12-41

Index-9

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CLOSED LOOP LAMBSE CONTROLLER       COMMANDED GEAR, AUTO UPSHIFT

(CDAN0), 8-259                      LOGIC (CDAJ0), 26-33

CLOSED THROTTLE NEUTRAL TIMER       COMP_FCTR, 26-10 to 26-14, 28-38

(CDAA0), 8-243                  COMPREHENSIVE COMPONENTS -

closed_loop_timer (PCALL), 8-293        MONITOR FLAG AND FAULT CODE

closed_loop_timer (PROCEDURE),          SET (CDAF0), 23-137

8-301                           compute_fuel_pulsewidth (CALL),

CLUTCH SWITCH INPUT AND SELFTEST        8-179

(CDAL0), 31-84                  compute_fuel_pulsewidth

CLV, 5-124                              (DEFINITION), 8-180

CM_HEAT_TM, 11-74                   CONFIDENTIAL, 28-29

CMIS1FIL, 5-121, 35-37 to 35-38,    CONPR, 12-20

35-46 to 35-47, 35-54           CONT_ENA, 31-44

CMIS2FIL, 5-121, 35-37 to 35-38,    COOL_BASE, 13-22, 13-27

35-46 to 35-47, 35-54           COOL_CAL_SW, 4-44

CMIS3FIL, 5-121, 35-37 to 35-38,    COOL_FLG, 8-24, 8-132, 8-240,

35-46 to 35-47, 35-54               8-320, 13-27, 13-48, 13-57,

CMIS4FIL, 5-121, 35-37 to 35-38,        26-10, 26-14

35-46 to 35-47, 35-54           COOL_PI_INT, 8-240, 26-14

CMIS5FIL, 5-121, 35-37 to 35-38,    COOLING FAIL-SAFE LOGIC (CDAN0),

35-46 to 35-47, 35-54               8-320

CMIS6FIL, 5-121, 35-37 to 35-38,    CPP_CODES, 31-85

35-46 to 35-47, 35-54           CPP_DISABLE, 31-86

CMIS7FIL, 5-121, 35-37 to 35-38,    CPP_NMAX, 31-86

35-46 to 35-47, 35-54           CPP_NMIN, 31-86

CMIS8FIL, 5-121, 35-37 to 35-38,    CPP_SEL, 31-86

35-46 to 35-47, 35-54           CPRGTMR, 15-71, 15-74

CNET_ERR_SW, 25-9                   CPRIMLVL, 35-87

CNET_TP_MIN, 28-60                  CPRIMUP, 35-86

CO_ST_ECT (READ), 13-79             CPU_OK_1, 35-68

CODE_1356_FLG, 35-68, 35-88         CPUOK_PW, 35-66

CODE_1357_FLG, 35-67, 35-88         CRANK, 31-71

CODES_CHECKD, 23-57, 23-63 to       CRANK/UNDERSPEED/RUN MODE

23-64                               SELECTION (CDAI0), 6-2

CODES_COUNT, 5-62, 23-59 to 23-60,  CRANKING, 23-158 to 23-159,

23-62 to 23-63, 23-77 to            23-173

23-78, 23-84 to 23-85, 23-87,   CRK_FUL, 8-193

23-93, 23-95, 23-98, 23-103     CRK_FUL (READ), 8-136, 8-213

to 23-104, 23-109, 23-112,      CRK_I=PIP_INJ, 8-136

23-120                          CRK_LOOP_CTR, 8-254

CODES_COUNT (READ), 5-144           CRK_PIP_CAL, 8-254

COEFDC, 12-23                       CRK_PIP_ECT, 8-254

COILAPFIL, 35-67, 35-86 to 35-87    CRK_PIP_INJ, 8-254

COILBPFIL, 35-67, 35-86 to 35-87    CRK_PIP_INJ SELECTOR LOGIC

COILCPFIL, 35-67, 35-86 to 35-87        (CDAL0), 8-253

COILDPFIL, 35-67, 35-86 to 35-87    CRK_PIP_INT, 8-254

COILUFIL, 35-67, 35-86 to 35-87     CRKCTR_RESET, 8-242

COLD SHIFT MULTIPLIER (CDAF0),

25-35

COLD TEMPERATURE DETERMINATION

FOR TV SOLENOID OPERATION

(CDAA0), 29-18

COLTBU

WRITE, 8-72

Index-10

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

CRKFLG, 3-8, 5-11, 5-57, 6-6, 7-3   CTLOW, 8-14, 8-49, 8-121, 12-10,

to 7-4, 8-14, 8-122, 8-193,         15-71, 30-5

8-199 to 8-200, 8-242, 8-251,   CTNFLG, 8-24, 8-244, 13-18

12-6 to 12-7, 12-16 to 12-17,   CTNTMR, 8-24, 8-244

13-7 to 13-8, 13-19, 13-21,     CTPRG, 15-70

16-4, 16-8, 16-24, 18-12,       CTPRG_SL, 15-70

20-3, 23-112, 23-167 to         CTPTFG, 7-3 to 7-4, 13-32, 13-40

23-168, 23-183 to 23-184,       CTTMR, 8-26, 8-131, 8-252, 13-64

24-29 to 24-30, 24-38, 24-55,   CURR_TST_ENA, 11-13, 11-16, 11-71,

25-10, 30-5, 31-34, 31-71 to        11-74, 11-83

31-72, 31-75, 35-71, 35-79      CURRENT_CKD, 4-41, 11-71, 11-78

READ, 8-82                            to 11-79

CRKFLG (READ), 8-136, 8-179,        current_time, 8-225

8-213                           CVS FMEM (CDAN0), 15-29

CRKPIP_CTR_BG (READ), 8-136         CVS_PERIOD, 4-47

CRKPIP_CTR_W, 8-242                 CxxxxFIL, 23-141 to 23-142

CRKPIPCNT2_W, 8-242                 CxxxxLVL, 23-141 to 23-142

CRKPIPCTR_BW, 8-242                 CxxxxMON, 23-141 to 23-142

CRKTIM, 13-14                       CXXXXUP, 23-141

CRKTM1, 8-242                       CYCCTR, 26-48

CRKTMR_INC, 8-242                   CYL_AIR_CHG, 8-105, 8-107, 8-110,

CROWD_DEC, 8-27                         8-223, 9-18, 9-101 to 9-102,

CS_MAX_TIME, 25-37                      9-104, 24-3, 24-20, 24-29

CS_MULT, 25-37                      CYL_AIR_CHG (READ), 8-180, 15-17

CS_SFT_MULT, 25-37, 26-12 to        CYLARC_BG, 20-3, 24-3, 24-9,

26-14, 28-16                        24-29, 24-59

CSD COMPENSATION ALGORITHM          CYLOFF_PPM, 13-52, 13-57

(CDAN0), 11-172                 CYLPPM CALCULATION (CDAJ0), 13-57

CSD DETECTION ALGORITHM (CDAN0),

11-175                          D, 8-132

CSD_CUMLT11, 11-174                 D21DLY, 28-20

CSD_CUMLT21, 11-174                 D32DLY, 28-20

CSD_LIKELY11, 11-177 to 11-179      D43DLY, 28-20

CSD_LIKELY21, 11-177 to 11-179      DACCEL_36, 35-14, 35-21

CSD_MIN_AMB, 11-127 to 11-130       DACCEL_CALIB, 35-14

CSD_SWITCH11, 11-178 to 11-179      DACCEL_HGHT, 35-14

CSD_SWITCH21, 11-178 to 11-179      DACCEL_THRES, 35-14

CSD_SWITCHES, 11-179                DARC BI-DIRECTIONAL PARALLEL I/O

CSD_TIMER11, 11-177 to 11-179           PORT (DUCE PI/O), 4-22

CSD_TIMER21, 11-177 to 11-179       DARC DELTA-TIME INPUTS (DARC DTI),

CSD_TM, 11-177 to 11-178                4-24

CSD_VOLTAGE, 11-178 to 11-179       DARC MULTIPLEXED PARALLEL PORT

CSDMUL, 11-179                          (DARC MPI/O), 4-27

CSUMFIL, 35-46                      DARC PULSE-WIDTH MODULATED

CTBYS, 17-10                            OUTPUTS (DARC PWM), 4-25

CTBYS_CL, 17-10                     DAS_BKDLY, 13-67

CTDFSO, 8-129, 8-131                DAS_CNT, 13-67

CTDFSO_PC, 8-129                    DAS_CTDLY, 13-64

CTDLY, 28-18                        DAS_CTR, 13-64, 13-67

CTEDSO, 8-129, 8-131                DAS_DNDT, 13-64

CTEDSO_PC, 8-129                    DASCLP_4INH, 13-67

CTFLG, 9-60                         DASCTL, 13-73, 13-89

CTHIGH, 8-14, 8-121, 8-244, 15-71   DASHPOT CALCULATIONS (CDAJ0),

CTHIN, 8-49                             13-60

DASMHYST, 13-9

Index-11

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DASMPH, 13-9                        DEMAND_STATE, 11-138 to 11-142,

DASPOT, 9-14, 9-18, 13-41, 13-65,       11-144, 11-148 to 11-149,

13-67, 13-73, 13-89                 17-12, 17-15 to 17-17, 17-25,

DASPTK, 13-65                           17-33

DATA_TIME, 9-96, 24-7, 29-30 to     DEMAND_TIMER, 11-141 to 11-149,

29-31, 35-65, 35-77                 12-43 to 12-44, 17-15, 17-25,

READ, 8-171                           17-32 to 17-33, 23-169 to

DCECTTM, 8-26                           23-170, 23-185 to 23-189

DCELQ4, 8-130 to 8-131              DESDP, 12-20

DCELQ5, 8-130 to 8-131              DESEM, 12-11, 12-18 to 12-19

DCEVS, 8-26                         DESEM_FLG, 12-18

DCEVSH, 8-26                        DESIRED EGR MASS FLOW (CDAN0),

DEBYCP, 13-39                           12-8

DEBYMA, 13-2, 13-39                 DESIRED MASS AIR FLOW CALCULATION

DECEL FUEL SHUT OFF LOGIC (CDAL0),      (CDAJ0), 13-49

8-125                           DESMAF, 13-3, 13-41

DEGPIP, 8-124                       DESMAF_PRE, 13-3, 13-41, 13-49,

DELAMB, 8-74                            13-52, 13-59

DELARCH_EGR, 12-39                  DESNLO, 13-20

DELAY/VERIFY SHIFT LOGIC (CDAJ0),   DETERMINE SHIFT SOLENOID STATES

26-15                               (CDAL0), 26-45

DELCOL, 8-73                        DFHLOD, 8-129

DELHYS, 13-3, 13-65                 DFHLOD_PC, 8-129

DELPR, 12-18 to 12-19, 12-37 to     DFHLODH, 8-130

12-38, 12-41 to 12-42, 12-44    DFIB_LEAN11, 11-166, 11-173 to

DELPR_FLW, 5-121, 12-41 to 12-42        11-174

DELPR_HOSE, 5-121, 12-37            DFIB_LEAN11 (READ), 8-57

DELPR_ON_DEM, 12-44                 DFIB_LEAN21, 11-166, 11-173 to

DELPR_THRES1, 5-121, 12-38              11-174

DELPR_THRES3, 5-121, 12-38          DFIB_LEAN21 (READ), 8-57

DELROW, 8-73                        DFIB_RICH11, 11-166, 11-173 to

DELTA, 7-3 to 7-4                       11-174

DELTA PRESSURE FEEDBACK EGR         DFIB_RICH11 (READ), 8-57

(CDAN0), 12-12                  DFIB_RICH21, 11-166, 11-173 to

delta_delay, 8-185 to 8-186             11-174

delta_puls_width, 8-224             DFIB_RICH21 (READ), 8-57

DELTA_RATIO, 26-35, 26-39, 28-27,   DFLOD, 8-129 to 8-130

28-71 to 28-72                  DFLOD_PC, 8-129

DELTA_VEGO12, 10-28                 DFLODH, 8-130

DELTA_VEGO22, 10-29                 DFNOVH, 8-130

DELTAT, 29-16 to 29-17              DFNOVS, 8-129

DELTOTHI, 29-11                     DFNOVS_PC, 8-129

DELTOTLD, 24-63, 29-11              DFSECT, 8-129, 8-132

DEMAND_ABORT, 11-138, 12-43,        DFSECT_PC, 8-129

17-15, 23-166 to 23-167,        DFSFLG, 8-106, 8-132, 8-161,

23-182 to 23-185, 23-190            8-240, 10-11, 15-72, 35-100

DEMAND_LAST, 12-33, 12-54           DFSFLG (READ), 8-129, 8-137

DEMAND_MODE, 5-10, 8-324, 11-108,   DFSLAM, 8-47

12-33, 12-54, 14-10, 16-24,     DFSO_PC_SW, 8-129

23-24, 23-49, 23-57, 23-59 to   DFSRPH, 8-130

23-62, 23-77, 23-84, 23-93,     DFSRPM, 8-129 to 8-130

23-103, 23-155, 23-166,         DFSRPM_PC, 8-129

23-173, 23-181                  DFSVS, 8-129

DFSVS_HYS_FG, 8-130, 8-132

DFSVS_PC, 8-129

Index-12

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DFSVSH, 8-130                       DP_MIN, 12-18 to 12-19

DIAGNOSTIC SCHEDULER (CDAN0),       DRBASE, 9-46, 13-13

23-2                            DRBASE_ALT, 13-13

DIGITAL TRANSMISSION RANGE SENSOR   DRC21, 26-39

INPUT CONVERSION (CDAN0),       DRC23, 26-35

25-76                           DRC34, 26-35

DIS INJECTOR SYNCHRONIZATION        DRIVE_COUNT, 23-133

(CDAK0), 8-165                  DRIVE_COUNT (READ), 5-144

DIS_FMEM, 8-24, 8-43, 8-47, 8-49,   DRIVING CYCLE (CDAF0), 23-132

8-240, 12-7, 14-10, 31-23 to    DRV2NEU_FLG, 27-7 to 27-9

31-26, 35-30, 35-32             DS_CM_MAX, 11-74

DISABLE_EOLT, 32-8                  DS_CM_MIN, 11-74

DISCUTOUT1, 8-236 to 8-237, 31-23   DS_CUR_HI, 11-77

to 31-26                        DS_CUR_LOW, 11-77

DISCUTOUT2, 8-236 to 8-237, 31-23   DS_DELAY, 11-51, 11-54

to 31-26                        DS_DLY_TMR, 11-13, 11-51 to 11-56

DISFMEM, 31-26                      DS_EGO_LEAN1, 8-48, 8-51, 11-13,

DISFMEM_DSRD, 31-23 to 31-26            11-54 to 11-56

DISFMEMTM, 31-26                    DS_EGO_LEAN2, 8-48, 8-51, 11-13,

DISFMEMTMR, 31-26                       11-51 to 11-53, 11-55 to

DISTANCE, 22-5                          11-56

DISTRIBUTORLESS IGNITION SYSTEM     DS_EGO_OL, 8-24, 8-48, 11-13,

TEST (CDAN0), 35-58                 11-51 to 11-56

DISTRIBUTORLESS IGNITION            DS_EGO_RICH1, 8-48, 8-51, 11-13,

SYSTEM/MISFIRE FMEM (CDAL0),        11-51 to 11-53, 11-55 to

31-21                               11-56

DLOAD_EXEC_KEY, 5-11, 5-55, 5-57,   DS_EGO_RICH2, 8-48, 8-51, 11-13,

8-324                               11-54 to 11-56

DLOAD_PROG_STRT, 5-11, 5-55, 5-57   DS_FAULTED, 11-89, 11-91

DLT_INDS, 25-46                     DS_HEAT_TM, 11-16

DNAC_DR, 13-25                      DS_HTR_OFF, 11-82 to 11-83

DNAC_NEUT, 13-25                    DS_HTR_ON, 11-80

DNDSUP, 8-18, 8-49 to 8-50, 8-122,  DS_HTR_TMR, 11-12, 11-16, 11-74,

8-246, 8-252, 9-21 to 9-22,         11-92, 11-141

9-46, 13-13 to 13-14, 13-18,    DS_KOER_SW, 11-140

13-25, 13-43, 13-52, 13-67,     DS_LEAN1, 11-47, 11-50, 11-52 to

13-73 to 13-74, 29-6                11-56

DNDT_DAS, 13-64, 13-67, 29-23 to    DS_LEAN2, 11-48 to 11-53, 11-55

29-24                               to 11-56

DNDT_IMCC, 29-23                    DS_OV_ON, 11-12

DNDT_LST, 9-27 to 9-28              DS_RICH1, 11-47, 11-51 to 11-53,

DNDT_PSS, 25-24, 29-23                  11-55 to 11-56

DNDT_SPK, 9-27 to 9-28, 9-48,       DS_RICH2, 11-48 to 11-49, 11-52

29-23                               to 11-56

DNDT_TQI, 25-75                     DS_STATE, 11-16, 11-51 to 11-56

DNDTI, 29-23                        DS_SWAPPED_1, 11-139, 11-144,

DOL, 3-3, 4-15, 4-45                    11-147, 11-150

DOL (READ), 5-136                   DS_SWAPPED_2, 11-139, 11-147,

DOL_COUNT, 3-8, 4-45, 19-3              11-150

DOL_FAULT, 4-34                     DS_SWTPOINT, 5-121, 11-143 to

DOL_FAULT (READ), 5-138                 11-144, 11-146 to 11-149

DOLHP, 4-15, 4-45, 19-3             DS_TST_OK, 11-139 to 11-141,

DOWNSTREAM EGO MONITOR (CDAM0),         11-144, 11-147 to 11-149

11-40                           DSDPRM, 12-16

DP_MAX, 12-18 to 12-19

Index-13

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

DSDRPM, 8-106, 8-130, 9-43, 9-46,   DYNPW, 8-224 to 8-225, 8-228 to

13-10, 13-22, 13-25 to 13-27,       8-229, 8-231

13-41, 13-46, 13-52, 13-65,

13-67, 13-73, 24-48             EAM, 4-35, 17-12 to 17-13, 17-16,

DSDRPM (HICAM) CALCULATION              17-18, 17-24 to 17-25, 17-30

(CDAJ0), 13-15                      to 17-31

DSDRPM (LOADS) CALCULATION          EAM (READ), 5-133

(CDAL0), 13-22                  EAM AND OUTPUT STATE MONITOR

DSDRPM (READ), 5-144                    CIRCUIT (Mid Line 2) (CDAN2),

DSDRPM CALCULATION (CDAL0), 13-10       17-2

DSDRPM_LST, 13-27, 13-46            EAM_ANTICIP, 13-52, 17-16, 17-18

DSDRPM_OFF, 13-46                       to 17-19, 17-21, 17-30

DSRD_FREQ, 8-257, 11-36             EAM_ANTP_DLY, 17-16, 17-18, 17-21

DSRDE1, 28-27                       EAM_ANTP_TMR, 17-16, 17-19, 17-21

DSRDE2, 28-27                       EAM_FAULT, 4-35

DSRDE3, 28-27                       EAM_FAULT (READ), 5-138

DSRU[GR_CM], 28-70                  EAM_LAM_DEL, 17-19

DSRUE2, 28-27                       EAM_OLD, 17-12

DSRUE3, 28-27                       EAM_PPM, 13-52

DSRUE4, 28-27                       EAMM, 4-23, 17-18, 17-23 to 17-26,

DSTM2, 8-131                            17-29

DSTPBR, 13-3, 13-8, 13-65           EAMM (READ), 5-136

DT12S, 8-107, 8-110, 8-219, 8-228,  EAMM_TST_TM, 17-25

9-58, 9-62, 9-65, 9-80 to       EAMM_TST_TMR, 17-15 to 17-16,

9-81, 9-96, 29-23, 29-31,           17-25

35-72                           EBC INITIALIZATION (CDAJ0), 5-25

READ, 8-172                       ECR, 5-17, 5-23, 5-26

DT12S (READ), 8-164, 8-181, 8-213   ECR (WRITE), 5-15, 5-112

DT12S (WRITE), 8-164                ECSM_LVL, 28-59

DT12S(r), 9-97 to 9-98              ECT, 3-8, 5-96, 6-6 to 6-7, 8-8,

DT12S_AVG, 26-66, 26-71, 28-70          8-14 to 8-16, 8-18, 8-44,

DT12SH (READ), 8-164, 8-181             8-101, 8-106 to 8-107, 8-122,

DT12SH (WRITE), 8-164                   8-132, 8-244, 8-251, 8-287,

DT23S, 8-110, 9-96, 29-31, 35-72,       9-14, 9-18, 9-20, 9-22, 9-27,

35-82                               9-31, 9-60 to 9-61, 10-10 to

DT23S (READ), 8-164, 8-213              10-11, 10-13, 10-24, 11-33,

DT23S (WRITE), 8-164                    11-141, 12-11, 13-52, 14-8,

DT23S(r), 9-97                          15-65, 15-70, 15-90, 16-4,

DT34S, 29-31, 35-72                     16-8, 16-22, 17-10, 17-19,

DT_OSS, 25-92                           18-11, 23-26 to 23-28, 23-43

DTPMAX, 31-71                           to 23-44, 23-57, 23-130 to

dtscf, 25-92                            23-131, 24-26, 24-29, 24-33,

DUAL ACCESS RAM CONTROLLER (DARC),      25-17, 25-26, 25-37, 28-3,

4-22                                28-6, 28-16, 29-5, 29-9,

DUTY CYCLE EXPANDER OUTPUTS             31-30, 31-33 to 31-35, 31-38,

(DUCE/DCO), 4-16                    31-55, 31-57, 31-61, 35-28 to

DWFHP, 16-12                            35-29

DYN_MIN_ARC, 8-223                    READ, 8-74, 8-87

dyn_off_tim, 8-231                  ECT (READ), 5-144, 8-115 to 8-116,

dyn_on_edge, 8-225                      8-136, 8-307, 13-79

DYNAMIC EPC CALCULATION (CDAM0),    ECT FAN CONTROL (CDAL0), 16-21

27-34                           ECT SENSOR INPUT AND SELFTEST

DYNAMIC FUEL OFF (CDAI0), 8-230         (CDAN0), 31-27

DYNAMIC TV DUE TO COLD              ECT_CNTS, 4-8, 31-31

TRANSMISSION (CDAA0), 29-20     ECT_CNTS (READ), 5-144

Index-14

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

ECT_CODES, 31-29                    EDFPPM, 13-56

ECT_DEG_C MESSAGE (CDAM0), 5-95     EDFPPM CALCULATION (CDAN0), 13-55

ECT_EGO_MAX, 11-33                  EDIS MODULE TEST (CDAB0), 33-2

ECT_EGO_MIN, 11-33                  EDM_HIGH, 4-11

ECT_ENG, 23-126, 31-30 to 31-33,    EDSEL, 8-200

31-36                           EFFLG1, 8-105 to 8-106

ECT_ER_TM, 31-32                    EFMFLG, 12-7, 12-19, 12-50

ECT_ER_TMR, 31-31 to 31-33          EFMFLG (READ), 5-132

ECT_FIL, 23-40 to 23-43             EFTC, 8-110

ECT_FIL_MAX, 23-35, 23-43           EFTCA, 8-107, 8-110

ECT_FIL_MAX (READ), 5-145           EFTCD, 8-107, 8-110

ECT_FM_TM, 31-33                    EFTR, 8-101, 8-110 to 8-111

ECT_FM_TMR, 31-32 to 31-33            READ, 8-88

ECT_FMEM, 31-35                     EGO FAULT FLAG CONTROL (CDAM0),

ECT_FMEM_TMR, 31-35                     11-152

ECT_FRST_FL, 31-33 to 31-34         EGO HEATER CONTROL (CDAN0), 11-57

ECT_HI_ER, 31-32                    EGO HEATER IO (CDAN0), 11-93

ECT_HI_KOEO, 31-36                  EGO MONITOR CONTROL (CDAN0), 11-2

ECT_HI_KOER, 31-36                  EGO MONITOR CONTROL STRATEGY

ECT_INST, 8-254, 31-30, 31-33 to        (CDAM0), 11-183

31-34                           EGO SWITCHING TEST (CDAM0),

ECT_INT, 23-40 to 23-41, 23-43          11-132

ECT_LO_ER, 31-32 to 31-33           EGO SWITCHING TEST (CDAN0),

ECT_LO_KOEO, 31-36                      11-103

ECT_LO_KOER, 31-36                  EGO11_FREQ, 11-30, 11-36, 11-38

ECT_LVL, 23-42                      EGO11_TESTS, 11-116

ECT_RAMP_TM, 31-35                  EGO1_FL_RAT, 5-121, 10-38

ECT_TC1, 31-33 to 31-34             EGO1_RF_AVE, 5-121, 10-23, 10-38

ECT_TIME, 5-24                      EGO1_RF_DEL, 10-38

ECT_TQM, 25-37                      EGO1_RF_RAT, 10-38

ECT_WARM, 23-130 to 23-131          EGO1_RF_RAT1, 10-37

ECT_WARM_ENG, 31-57                 EGO1FMFLG, 11-33, 11-131, 11-154

ECT_WUT, 31-38                        READ, 8-78

ECTBPLO, 29-9                       EGO1FMFLG (READ), 5-132

ECTCNT, 6-6, 10-9, 13-46, 31-14,    EGO21, 11-29

31-38                           EGO21_FREQ, 11-30, 11-36, 11-38

ECTFMM, 31-35                       EGO21_TESTS, 11-120

ECTIP, 9-60                         EGO2_FL_RAT, 5-121, 10-40

ECTNOK, 9-61                        EGO2_RF_AVE, 5-121, 10-23, 10-40

ECTOFF (READ), 8-115                EGO2_RF_DEL, 10-40

ECTOFF (WRITE), 8-115 to 8-116      EGO2_RF_RAT, 10-36 to 10-37,

ECTSTABL, 8-14                          10-39 to 10-40

ECTSTABL_CL, 8-14                   EGO2_RF_RAT1, 10-39 to 10-40

ECTSTABLTIM, 8-14                   EGO2_RF_RAT2, 10-39

ECU_EO_DONE, 17-12, 23-169, 32-8    EGO2FMFLG, 11-33, 11-131, 11-155

ECU_EO_INIT, 23-15, 32-8              READ, 8-78

EDF, 4-29, 16-35                    EGO2FMFLG (READ), 5-132

EDF_DELAY, 13-56                    EGO_CAT_FKA, 10-38, 10-40

EDF_PPM, 13-49, 13-52, 13-56        EGO_CAT_FKB, 10-38, 10-40

EDF_STAT1, 4-29                     EGO_CHKD, 23-69, 23-71 to 23-72

EDF_STATUS, 4-40, 16-16, 16-35      EGO_CL_TM, 11-33

EDF_STATUS (READ), 5-138            EGO_CL_TMR, 11-33, 11-38

EDFHP, 4-29, 13-56, 16-4, 16-12,    EGO_COUNT, 23-69, 23-71

16-16, 16-35                    EGO_CUR_MON, 11-71, 11-74 to

EDFM, 4-12                              11-75, 11-78 to 11-79, 11-81

Index-15

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EGO_DEL_TM, 23-24                   EGO_TST_SW, 11-11, 15-50, 23-21

EGO_DEL_TMR, 23-21, 23-24               to 23-22, 23-52, 23-68 to

EGO_DS_MON, 11-13, 11-16 to 11-17       23-69, 23-135

EGO_DS_MON (READ), 8-300            EGO_US_MON, 11-13, 11-15 to 11-17

EGO_ER_DONE, 11-138, 11-142,        EGO_VS_ST, 11-35

23-185, 23-188                  EGO_WIND_SET, 11-11, 11-14 to

EGO_ER_INIT, 11-138, 15-72,             11-15, 11-17, 11-35

23-182, 23-187                  EGOCNT11, 11-164, 11-167

EGO_ER_INIT (READ), 15-23           EGOCNT21, 11-164, 11-167

EGO_FMEM, 23-21, 23-35, 23-39       EGOCT11, 8-74

EGO_FREQ_MIN, 11-36                   READ, 8-75 to 8-76

EGO_HTR_MON, 11-13, 11-16 to          WRITE, 8-73

11-17, 11-71, 11-74 to 11-75,   EGOCT21, 8-74

11-88                             READ, 8-75

EGO_HTR_SW, 11-11                     WRITE, 8-73, 8-77

EGO_INIT_RF1, 10-23                 EGOFL11, 8-273, 8-277, 11-118 to

EGO_INIT_RF2, 10-23                     11-119, 11-126, 11-164 to

EGO_KOER_SW, 11-138                     11-166

EGO_KOER_TM1, 11-143 to 11-144        READ, 8-75

EGO_KOER_TM2, 11-145 to 11-148      EGOFL11 (READ), 8-57

EGO_KOER_TM3, 11-148                EGOFL21, 8-273, 8-277, 11-123,

EGO_KOER_TM4, 11-149                    11-126, 11-164 to 11-166

EGO_LOAD_ST, 11-35                    READ, 8-75

EGO_MIL_ON, 23-50, 23-79, 23-110    EGOFL21 (READ), 8-57

EGO_MIL_SW, 23-110                  EGOFLx1, 8-275

EGO_MON, 11-16, 15-50, 23-21 to     egolearn_rat

23-22, 23-42, 23-44, 23-52,       READ, 8-75

23-68 to 23-69, 23-71 to          WRITE, 8-74

23-72                           EGOMAP_J1979, 11-162

EGO_MON11, 11-13 to 11-15, 11-27,   EGOMON_ENTRY, 11-35

11-30, 11-36, 11-39             EGOSS11, 11-118 to 11-119, 11-168

EGO_MON21, 11-13 to 11-15, 11-28,   EGOSS21, 11-122 to 11-123, 11-168

11-30, 11-36, 11-39             EGOSSS1, 11-168

EGO_MON_LST, 23-42, 23-44, 23-52    EGOSTATE1, 8-273 to 8-274, 8-278

EGO_MON_LST := 0, 23-71             EGOSTATE1 (READ), 15-16

EGO_MON_RDY, 11-33 to 11-35         EGOSTATE2, 8-273 to 8-274, 8-278

EGO_MON_TMR, 11-35                  EGOTSTCUMFLG, 11-110 to 11-111

EGO_N_ST, 11-35                     EGOTSTCUMTMR, 11-108, 11-111,

EGO_READY, 23-69 to 23-71, 23-135       11-118 to 11-119, 11-122 to

EGO_RETEST, 11-14                       11-123

EGO_REV_CTR, 11-14 to 11-15,        EGR, 30-5

11-17, 11-184                   EGR - PFE - V1.11_EGR_PFE (CDAN0),

EGO_REV_ENA, 11-14 to 11-15,            12-2

11-17, 11-184                   EGR ENABLE (CDAL0), 12-4

EGO_REV_FLG, 11-14, 11-184          EGR OBDII TEST LOGIC (CDAN0),

EGO_STAT11, 11-17, 11-30, 11-36         12-26

EGO_STAT21, 11-17, 11-30, 11-36     EGR SYSTEM INITIALIZATION (CDAN0),

EGO_SYN_CTR, 11-184                     12-2

EGO_TEST_MON, 11-111                EGR_ADD, 12-11

EGO_TEST_TMR_CLEAR, 11-112          EGR_CHKD, 23-69, 23-71 to 23-72

EGO_TST_ENA, 11-11, 11-13 to        EGR_CLR_RT, 12-54, 12-56

11-16, 11-88, 11-108, 23-16,    EGR_COUNT, 23-69, 23-71

23-21 to 23-22                  EGR_DYNO_DIS, 12-7

EGO_TST_RDY, 11-13, 11-17, 23-21,   EGR_DYNO_ENA, 12-7

23-41                           EGR_EFM_FLG, 12-49 to 12-50

Index-16

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EGR_ER_DONE, 12-43, 23-185,         EGRACT, 9-14, 9-18, 9-31, 10-11,

23-187                              12-25, 12-45, 24-26, 24-63

EGR_ER_INIT, 12-43, 23-182,         EGRACT_MON, 12-45

23-186                          EGRATE, 12-10 to 12-11

EGR_ER_RPM, 12-43                   EGRDC, 4-44, 11-124, 12-24, 12-41

EGR_ERROR, 12-20, 12-23                 to 12-42, 12-57 to 12-58

EGR_ERROR1, 12-23                   EGRDC (READ), 5-144

EGR_ERROR2, 12-23                   EGRDC1, 12-23

EGR_FLT_RT, 12-55                   EGRDC2, 12-23

EGR_FMEM, 23-17, 23-35, 23-39       EGRDC_FLW, 5-121, 12-41 to 12-42

EGR_HOSE_TST, 12-7, 12-37 to        EGRDC_MAX, 12-23

12-38                           EGREN, 11-124, 12-7, 12-10, 12-18,

EGR_HT_AM_MX, 12-37                     12-38, 12-45, 30-5

EGR_IMR_MOD, 12-11                  EGREN_TM_RT, 12-6

EGR_IMR_MUL, 12-11                  EGREN_TMR, 12-6 to 12-7, 12-16,

EGR_INIT_FLG, 12-3                      12-33, 12-37

EGR_MIL_ON, 23-50, 23-79, 23-110    EGRENA_TM, 12-6 to 12-7, 12-16,

EGR_MIN_AMB, 12-36, 12-49, 12-51        12-33, 12-37

to 12-53                        EGRFLG, 12-23

EGR_MON, 12-46, 12-49, 12-51 to     EGRGAIN, 12-23

12-54, 15-50, 23-41 to 23-42,   EGRMON_DN, 12-45

23-44, 23-51, 23-68 to 23-69,   EGRMON_TM, 12-46

23-71 to 23-72                  EGRMON_TMR, 12-45 to 12-46

EGR_MON_ACTH, 12-45                 EGRMON_UP, 12-45

EGR_MON_ACTL, 12-45                 EGRMPT, 12-10 to 12-11, 30-5

EGR_MON_LST, 23-44, 23-51           EGRTST_TM, 12-37

EGR_MON_LST := 0, 23-71             EIP_TMR, 27-8, 31-80

EGR_MUL, 12-11                      EISF, 8-106

EGR_N_FIL, 12-16                    EISF_FGL, 8-110

EGR_OBD_NMAX, 12-39                 EISF_FGU, 8-110

EGR_OFF_TMR, 12-38                  EISFLO, 8-106 to 8-107, 8-110

EGR_ON_FLG, 12-18 to 12-20, 12-23   EISFU, 8-106 to 8-107, 8-110

EGR_READY, 23-69 to 23-71, 23-135   ELECTRO-DRIVE FAN CONTROL

EGR_REP_SW, 12-3, 12-49                 STRATEGY (CDAN0), 16-10

EGR_RETRY, 12-3, 12-49 to 12-50     Electro-drive fan control

EGR_RTST_TM, 12-49                      strategy (EDF) - V3.3_EDF

EGR_RTST_TMR, 12-48 to 12-49            (CDAN0), 16-2

EGR_SHT_TMR, 12-3, 12-16, 12-18     ELECTRO-DRIVE FAN OUTPUT STRATEGY

to 12-19                            (CDAN0), 16-34

EGR_TP_HYST, 12-6                   ELECTRONIC PRESSURE CONTROL GUIDE

EGR_TPCUTOUT, 12-6 to 12-7              (CDAN0), 27-2

EGR_TST_DC, 12-24, 12-58            EM, 12-19, 12-25

EGR_TST_ENA, 12-33, 23-16 to        ENA_CSLP_TST, 28-60

23-17                           ena_transition_to_off

EGR_TST_LST, 12-49, 12-54             WRITE, 8-95

EGR_TST_RDY, 12-33, 23-17, 23-41    ENABLE_DYNAM, 8-223

EGR_TST_RNNG, 12-33 to 12-35,       ENG_OFF_TMR, 10-14

12-37, 12-41, 12-44, 12-58      ENG_STALLED, 29-23, 29-28

EGR_TST_SW, 15-50, 23-17, 23-36,    ENG_STDY_FLG, 12-39 to 12-40

23-51, 23-68 to 23-69, 23-135   ENGAGEMENT/STALL TV LOGIC (CDAA0),

EGR_TWO_TIME, 12-49                     27-18

EGR_VLV_OPN, 12-44

EGR_VLV_SHT, 12-16, 12-19, 12-38

EGR_WOT_TMR, 12-6 to 12-7

EGR_WRM_TM, 12-16, 12-33

Index-17

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

ENGCYL, 8-223, 8-240, 8-287, 9-96,  EPC_RMP_TM, 27-30 to 27-31

24-33, 24-59, 24-63, 29-23,     EPC_RMP_TMR, 27-31

31-23 to 31-26, 35-15 to        EPC_SHORT_SW, 25-8

35-16, 35-37 to 35-40, 35-44    EPC_ST_12, 27-30

to 35-46, 35-53, 35-69 to       EPC_ST_23, 27-30

35-70, 35-75 to 35-78, 35-98,   EPC_ST_34, 27-30

35-102, 35-105 to 35-107,       EPC_ST_LST, 27-31

35-109 to 35-115                EPC_ST_RMP, 27-30

ENGCYL(r), 9-98                     EPCFMRV, 27-14

ENGINE COOLANT FAN OUTPUT STATUS    EPCOCM1, 27-50, 29-15

TEST (CDAN0), 16-28             EPCOCM1 (READ), 5-144

ENGINE OFF SEQUENCE CONTROLER       EPCOCM1_CNTS, 4-20, 29-15

(CDAB0), 23-164                 EPT_OK_FLG, 12-18

ENGINE RUNNING SEQUENCE             EPT_OPN_MAX, 5-121

CONTROLLER (CDAN2), 23-177      EPT_VOPN_IDL, 12-35

ENGINE SPEED CALCULATION (CDAM0),   EPTBAR, 12-3, 12-17 to 12-18,

29-21                               12-20, 12-34 to 12-35

ENGINE SPEED OVER VEHICLE SPEED     EPTBAR_CH, 12-18

(NOVS) (CDAL0), 31-87           EPTBAR_OPN, 5-121, 12-35

ENGINE_PAT_STATUS Message (CDAK0),  EPTBAR_SL, 12-18

5-113                           EPTMAX, 12-34

ENPIP, 8-275                        EPTMIN, 12-34

ENPIP1, 3-8, 8-275, 8-278           EPTZER, 12-3, 12-16 to 12-18,

ENPIP1 (WRITE), 8-55 to 8-58            12-20

ENPIP2, 3-8, 8-275, 8-278           EPTZER_IV, 12-16 to 12-17

ENPIP2 (WRITE), 8-55 to 8-58        EPTZER_OFF, 12-16 to 12-17, 12-35

EO_ACTIVE, 11-80, 23-166 to         EPTZER_TMR2, 12-3, 12-17, 12-37,

23-168, 23-170                      12-44

EO_CCM_DLY, 23-169                  EPTZER_UPTOL, 12-17

EO_DONE, 23-15, 23-170              EPTZER_VALID, 12-37, 12-44

EO_INACTIVE, 23-166 to 23-168,      ER_ACTIVE, 17-18, 23-181 to

23-170                              23-185, 23-190

EO_INIT_FLG, 11-80                  ER_CLEANUP, 23-182, 23-185,

EO_MON, 23-20, 23-71 to 23-72,          23-189 to 23-190

23-170                          ER_CONDTS, 23-181, 23-183

EO_STATUS, 17-12, 23-15, 23-166     ER_DELTA_RPM, 17-15

to 23-170, 32-8                 ER_DONE, 9-97, 11-81, 11-100,

EO_TST_ENA, 23-16, 23-18, 23-20         23-15, 23-182 to 23-183,

to 23-21, 23-167 to 23-168          23-185 to 23-186, 23-190

EO_TST_RDY, 23-18, 23-20, 23-166    ER_EAM_LEAN, 17-32 to 17-33

to 23-168                       ER_EGR_FLG, 12-7, 12-10, 12-24,

EPC VFS CONTROL AND SELF TEST           12-43 to 12-44

(CDAG0), 27-41                  ER_EGRATE, 12-10

EPC_AUT41, 27-38 to 27-39           ER_FUL_REQ, 8-24, 8-47, 11-139,

EPC_AUT423, 27-39                       11-142 to 11-143, 11-145,

EPC_BLP_TMR, 27-8                       11-148 to 11-149, 23-181,

epc_codes, 27-46                        23-185, 23-187 to 23-188,

EPC_DLY_TIM, 27-48                      23-190

EPC_DLY_TMR, 27-48                  ER_HEAT_TIME, 11-141

EPC_ENG_TMR, 27-8 to 27-9           ER_HI_RPM, 23-185

EPC_ERR_SW, 25-8                    ER_INACTIVE, 23-183 to 23-185,

EPC_MAN42, 27-38                        23-190

EPC_RMP34, 27-30                    ER_INIT, 9-97, 11-81, 11-100,

EPC_RMP_12, 27-30                       23-15, 23-185 to 23-186,

EPC_RMP_23, 27-30                       25-14, 31-19, 31-43

Index-18

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

ER_ISC_HLD, 13-7 to 13-8, 23-181,   ETST_SWCUMTM, 11-111, 11-118 to

23-186 to 23-188, 23-190            11-119, 11-122 to 11-123

ER_ISC_REQ, 12-43, 13-20, 13-27,    ETV OVERCURRENT MONITOR VOLTAGE

23-181, 23-185, 23-187 to           (CDAG0), 29-15

23-188, 23-190                  ETV_BIAS, 27-49

ER_LAM_BAS, 17-17, 17-32 to 17-33,  ETV_COM_MN_D, 27-49

23-185, 23-187                  ETV_GAIN, 27-49

ER_LAM_DSD, 9-58, 11-139, 11-143,   ETV_GAIN_BAT, 27-49

11-145                          ETV_TEST, 27-14, 27-47, 27-50

ER_LAM_DSD1, 8-47, 11-148 to        Evaporative Purge Compensation

11-149, 17-17, 17-32 to 17-33,      Control (PCOMP) -

23-185, 23-187                      V1.1D_CANP_PCOMP (CDAN0),

ER_LAM_DSD2, 8-47, 11-148 to            15-2

11-149, 17-17, 17-32 to 17-33,  EVLOPNFM_FLG, 12-16, 12-50

23-185, 23-187                  EVR, 3-3

ER_MON, 23-20, 23-71 to 23-72,      EVR CONTROLLER - (EGRDC

23-183, 23-190                      CALCULATION) (CDAN0), 12-21

ER_RPM, 12-43, 13-20, 17-15,        EVR_CURRENT, 4-36, 4-44

23-185                          EVR_OPEN, 4-36

ER_SPARK, 9-6, 9-58, 9-67, 23-185   EVR_OPEN (READ), 5-137

ER_SPARK_BAS, 23-185                EVR_SHORT, 4-36

ER_SPK_REQ, 9-6 to 9-7, 23-181,     EVR_SHORT (READ), 5-137

23-185, 23-187 to 23-188,       EVR_STATUS, 12-57 to 12-58

23-190                          EVTDOT, 15-70 to 15-71

ER_STATUS, 9-58, 9-67, 9-97,        EXHAUST TEMPERATURE CALCULATION

11-81, 11-100, 11-138, 11-142,      (CDAN0), 10-2

12-43, 15-45, 15-72, 15-90,     Exhaust Temperature Model -

15-100, 17-10, 17-12, 17-14         V1.9_CAT_TEMP_MODEL (CDAN0),

to 17-17, 17-23 to 17-25,           10-2

17-32 to 17-33, 23-15, 23-182   EXT_CATMID, 10-12, 10-24

to 23-190, 25-14, 31-19,        EXT_CIN, 10-11 to 10-12

31-43                           EXT_CMD, 8-28, 8-47, 10-12 to

ER_STATUS (READ), 13-79, 15-23          10-13, 11-50

ER_TST_ENA, 23-16, 23-18, 23-20     EXT_CMD_SF, 8-28

to 23-21, 23-181 to 23-184      EXT_CMS, 10-10, 10-12

ER_TST_RDY, 23-18, 23-20, 23-182    EXT_COLD_CAT, 8-47

to 23-184                       EXT_EXD, 10-12

erase_code(Pxxx), 23-58             EXT_EXO_CL, 10-12

ERR_BRK_AWAY, 28-61                 EXT_EXO_MLT, 10-12

ERR_DB_CH, 28-34                    EXT_EXO_SH, 10-12

ERR_DB_SL, 28-34                    EXT_FEG, 8-14, 10-11

ERR_T0, 25-72, 28-34, 28-56,        EXT_FEH, 10-10 to 10-11

28-59, 28-61, 28-87             EXT_FEH_INT, 10-11

ERR_T0_AC, 25-72                    EXT_FEH_SLP, 10-11

ERR_T0_FD, 28-34, 28-87 to 28-88    EXT_FEU, 10-10 to 10-11, 11-12,

ERR_T0_FI, 28-34, 28-87 to 28-88        11-74, 11-82 to 11-83, 11-109

ERR_T0_FP, 28-34, 28-87 to 28-88    EXT_FL, 10-10 to 10-11

ERR_T1, 28-34, 28-87 to 28-88       EXT_FLANGE, 11-12

ERR_TST_BRK, 28-59, 28-61           EXT_FLG, 8-25, 8-28, 8-45, 8-48

error (READ), 8-304, 8-316          EXT_FMM_FLG, 8-28, 10-9, 11-80

ERROR_CNTR, 35-67                   EXT_INIT, 10-9 to 10-10

ERROR_DETECTED, 23-141              EXT_LS_CIN, 8-45, 10-11

ERROR_TMR, 35-67                    EXT_LS_FEU, 10-11

ETLUMX, 28-16                       EXT_LS_REG, 10-13

Index-19

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

EXT_REG, 10-10, 10-13, 11-12,         DEFINITION, 8-94

11-74, 11-82 to 11-83           FADPT_LEAN11

EXT_REG (READ), 8-304                 READ, 8-99

EXT_REGON_SF, 11-80                   WRITE, 8-97, 8-99

EXT_SET_FLG, 8-28                   FADPT_LEAN21

EXT_SS_CMS, 10-12                     READ, 8-99

EXT_SS_EXD, 10-12                     WRITE, 8-98 to 8-99

EXT_SS_EXO, 8-45, 10-12             FADPT_NEW_UPRATE_CALC

EXT_SS_FEH, 10-11                     CALL, 8-73

EXT_SS_FEU, 10-11                     DEFINITION, 8-79

EXT_SS_FL, 8-45, 10-10 to 10-11     FADPT_READY

EXT_SS_FLS, 8-45, 10-11               READ, 8-89

EXT_SS_REG, 10-13                     WRITE, 8-88

FADPT_READY (READ), 15-8

3F, 5-8                             FADPT_RICH11

f (READ), 8-309                       READ, 8-99

f (WRITE), 8-309                      WRITE, 8-97, 8-99

F6K_LST_PS (READ), 15-8             FADPT_RICH21

F6K_LST_PS (WRITE), 15-17             READ, 8-99

F_A_RATIO1, 3-8, 15-16                WRITE, 8-98 to 8-99

F_A_RATIO1 (READ), 8-180            FADPT_SELF_TEST

F_A_RATIO1 (WRITE), 8-137             CALL, 8-70

F_A_RATIO2, 3-8, 15-16                DEFINITION, 8-71

F_A_RATIO2 (READ), 8-180            FADPT_SW, 8-82, 8-88

F_A_RATIO2 (WRITE), 8-137           FADPT_UPDATE_KAMRF

F_PUMP_ERROR, 4-34, 8-153, 8-157      CALL, 8-70

F_PUMP_ERROR (READ), 5-137            DEFINITION, 8-82

F_PUMP_S_MON, 4-12, 8-158           FADPT_UPDATE_STATUS

F_PUMP_S_MON (READ), 5-136            CALL, 8-70

FADPT_EGO_FAILURE_RESET               DEFINITION, 8-87

CALL, 8-70                        FADPT_UPDATE_TABLE

DEFINITION, 8-78                    CALL, 8-70

FADPT_ENABLE                          DEFINITION, 8-72

READ, 8-73, 8-95 to 8-96          FAEGCT, 8-74

WRITE, 8-89                       FAILED_BANK, 8-271 to 8-275,

FADPT_FM_FAULT_CHK                      8-278, 11-39

CALL, 8-94                        FAILED_BANK (READ), 8-314, 8-318

DEFINITION, 8-96                  FAILURE MODE MANAGEMENT LOCK-UP

FADPT_FM_FAULT_CLEAR                    (CDAA0), 28-63

CALL, 8-94, 8-96                  FALD_BNK_OLD (DEFINED), 8-312

DEFINITION, 8-99                  FALD_BNK_OLD (READ), 8-314

FADPT_FM_READY_CHK                  FALD_BNK_OLD (WRITE), 8-314

CALL, 8-94                        FAN OUTPUT STATE MONITOR (CDAJ0),

DEFINITION, 8-95                      16-14

FADPT_FM_TEST_EGO11_KAM             FAN_DELAY, 16-12

CALL, 8-96                        FAN_ER_TM1, 16-33

DEFINITION, 8-97                  FAN_ER_TM2, 16-33

FADPT_FM_TEST_EGO21_KAM             FAN_ER_TM3, 16-33

CALL, 8-96                        FAN_ER_TM4, 16-33

DEFINITION, 8-98                  FAN_ER_TMR1, 16-30 to 16-33

FADPT_FMEM_CHECK                    FAN_ER_TMR2, 16-30 to 16-33

CALL, 8-94                        FAN_ER_TMR3, 16-30 to 16-33

DEFINITION, 8-99                  FAN_ER_TMR4, 16-30 to 16-33

FADPT_FUEL_MONITOR                  FANSCP_FMFLG, 16-27, 16-30

CALL, 8-70                        FANTMR, 13-56, 16-12 to 16-13

Index-20

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FAO_MON_FLG (FCALL), 8-307, 8-316   FAULT FILTER AND FAULT MONITOR

FAO_MON_TMR1, 11-39                     LOGIC (CDAI0), 23-139

FAO_MON_TMR2, 11-39                 fault_lockout (PCALL), 8-293

FAO_SCP_AVG1, 5-121                 fault_lockout (PROCEDURE), 8-297

FAO_SCP_AVG1 (DEFINED), 8-310       FB_DIFF, 8-136, 8-231

FAO_SCP_AVG1 (WRITE), 8-311         FB_DIFF (READ), 8-180 to 8-181

FAO_SCP_AVG2, 5-121                 FBS_ENTRY_TC, 9-40, 9-43, 9-46

FAO_SCP_AVG2 (DEFINED), 8-310       FEATURES

FAO_SCP_AVG2 (WRITE), 8-311           INTAKE MANIFOLD RUNNER CONTROL

FAOP_EGO_TMP (DEFINED), 8-303             - V1.3A_IMRC_ONOFF (CDAN2),

FAOP_EGO_TMP (READ), 8-304                18-2

FAOS_DT (FCALL), 8-309              Features

FAOS_DT1 (DEFINED), 8-291             Adaptive Fuel Control and

FAOS_DT1 (WRITE), 8-292                   OBD-II Fuel System Monitor

FAOS_DT2 (DEFINED), 8-291                 - V1.5_FADPT (CDAN0), 8-61

FAOS_DT2 (WRITE), 8-292               Air Charge -

FAOS_MON_TM, 11-39                        V3.1_AIR_CHARGE_97_IMRC

faosc2_main (PROCEDURE), 8-292            (CDAM0), 24-2

faosc_avg (FCALL), 8-293              Air Conditioning Control

faosc_avg (FUNCTION), 8-307               Strategy (ACC) - V2.3_ACC

FAOSC_ENA_SW (DEFINED), 8-298             (CDAN0), 14-2

FAOSC_ENA_SW (READ), 8-299            CATALYST_MONITOR (Standard) -

FAOSC_FMEM (DEFINED), 8-298               V1.8A_CAT_FTP_MONITOR

FAOSC_FMEM (READ), 8-300                  (CDAN0), 10-15

FAOSC_FMEM (WRITE), 8-299             Closed Loop Fuel Control -

faosc_i_control (FCALL), 8-314            V2.8C_CLFUL_NONFLEX_96

faosc_i_control (FUNCTION), 8-315         (CDAN2), 8-259

faosc_init (PROCEDURE), 8-288         EGR - PFE - V1.11_EGR_PFE

faosc_p_control (FCALL), 8-293            (CDAN0), 12-2

faosc_p_control (FUNCTION), 8-302     Electro-drive fan control

faosc_scp_report (PCALL), 8-293           strategy (EDF) - V3.3_EDF

faosc_scp_report (PROCEDURE),             (CDAN0), 16-2

8-310                             Evaporative Purge Compensation

FAR_LEAN_LIM, 8-325 to 8-326              Control (PCOMP) -

FAR_MUL, 8-324                            V1.1D_CANP_PCOMP (CDAN0),

FAR_MUL (READ), 8-180                     15-2

FAR_MUL BACKGROUND LOGIC (CDAK0),     Exhaust Temperature Model -

8-321                                 V1.9_CAT_TEMP_MODEL (CDAN0),

far_mul_1 (READ), 5-146                   10-2

far_mul_2 (READ), 5-146               Fuel Pump - V2.1_FPUMP (CDAN0),

FAR_MUL_CAL, 8-324                        8-139

FAR_MUL_CHK, 8-324 to 8-325           HEGO Monitor -

FAR_MUL_FLG, 8-324 to 8-325               V10.6_EGO_MONITOR (CDAN0),

FAR_MUL_FLG (READ), 8-180                 11-2

FAR_MUL_RAMP, 8-325 to 8-326          ISC_DESMAF_PID -

FAR_MUL_RDY, 8-324 to 8-325               V1.2_ISC_DESMAF_PID (CDAN0),

FAR_MUL_RSP, 8-324, 8-326                 13-28

FAR_MUL_RUN, 8-324, 8-326             ISC_DSDRPM - V1.A1_ISC_DSDRPM

FAR_MUL_SCP, 8-324 to 8-326               (CDAN0), 13-10

FAR_MUL_SLP, 8-325 to 8-326           OBDII Executive -

FAR_MUL_SW, 8-324                         V1.4B_OBDII_EXEC (CDAN0),

FAR_MUL_TMR, 8-325 to 8-326               23-2

FAR_RICH_LIM, 8-325 to 8-326

FAST_MIS_LVL, 35-42

FAULT CODES (CDAM0), 23-53

Index-21

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Running Loss OBD-II Evaporative   FEGO2_C_MIN5, 10-35 to 10-36

System Monitor. -             FEGO2_C_MIN6, 10-35 to 10-36

V1.9A_CANP_MONITOR (CDAN2),   FEGO2_C_MIN7, 10-35 to 10-36

15-24                         FEGO2_C_MIN8, 10-35 to 10-36

SCP - V6.2A_SCP_EATC_PATS         FEGO2_C_MIN9, 10-35 to 10-36

(CDAN0), 5-2                  FEGO2_CT1, 10-27, 10-35 to 10-36

Self Test Control -               FEGO2_CT2, 10-27, 10-35 to 10-36

V1.8_OBDII_SELF_TEST_CONTROL  FEGO2_CT21, 10-27

(CDAN2), 23-164               FEGO2_CT22, 10-27

Transient Fuel -                  FEGO2_CT23, 10-27

V1.1_FTRAN_TRANSIENT_FUEL     FEGO2_CT24, 10-27

(CDAM0), 8-100                FEGO2_CT25, 10-27

FEGO1_C_MIN1, 10-34                 FEGO2_CT26, 10-27

FEGO1_C_MIN2, 10-34                 FEGO2_CT27, 10-27

FEGO1_C_MIN3, 10-34                 FEGO2_CT28, 10-27

FEGO1_C_MIN4, 10-34                 FEGO2_CT29, 10-27

FEGO1_C_MIN5, 10-34                 FEGO2_CT3, 10-27, 10-35 to 10-36

FEGO1_C_MIN6, 10-34                 FEGO2_CT4, 10-27, 10-35 to 10-36

FEGO1_C_MIN7, 10-34                 FEGO2_CT5, 10-27, 10-35 to 10-36

FEGO1_C_MIN8, 10-34                 FEGO2_CT6, 10-27, 10-35 to 10-36

FEGO1_C_MIN9, 10-34                 FEGO2_CT7, 10-27, 10-35 to 10-36

FEGO1_CT1, 10-26, 10-33 to 10-34    FEGO2_CT8, 10-27, 10-35 to 10-36

FEGO1_CT2, 10-26, 10-33 to 10-34    FEGO2_CT9, 10-27, 10-35 to 10-36

FEGO1_CT21, 10-26                   FEGO2_CT_TOT, 10-27, 10-35 to

FEGO1_CT22, 10-26                       10-37, 10-39 to 10-40

FEGO1_CT23, 10-26                   FEGO2_MX_CT1, 10-35 to 10-36

FEGO1_CT24, 10-26                   FEGO2_MX_CT2, 10-35 to 10-36

FEGO1_CT25, 10-26                   FEGO2_MX_CT3, 10-35 to 10-36

FEGO1_CT26, 10-26                   FEGO2_MX_CT4, 10-35 to 10-36

FEGO1_CT27, 10-26                   FEGO2_MX_CT5, 10-35 to 10-36

FEGO1_CT28, 10-26                   FEGO2_MX_CT6, 10-35 to 10-36

FEGO1_CT29, 10-26                   FEGO2_MX_CT7, 10-35 to 10-36

FEGO1_CT3, 10-26, 10-33 to 10-34    FEGO2_MX_CT8, 10-35 to 10-36

FEGO1_CT4, 10-26, 10-33 to 10-34    FEGO2_MX_CT9, 10-35 to 10-36

FEGO1_CT5, 10-26, 10-33 to 10-34    FEGO2_MX_TOT, 10-40

FEGO1_CT6, 10-26, 10-33 to 10-34    FF_CODE, 23-49, 23-61, 23-97 to

FEGO1_CT7, 10-26, 10-33 to 10-34        23-98, 23-125 to 23-127

FEGO1_CT8, 10-26, 10-33 to 10-34    FF_ECT, 23-125 to 23-127

FEGO1_CT9, 10-26, 10-33 to 10-34    FF_FUEL_MIS, 23-49, 23-127

FEGO1_CT_TOT, 10-26, 10-33 to       FF_KAMREF1, 23-125 to 23-127

10-34, 10-37 to 10-38           FF_KAMREF2, 23-125 to 23-127

FEGO1_MX_CT1, 10-33                 FF_LAMBSE1, 23-125 to 23-127

FEGO1_MX_CT2, 10-33                 FF_LAMBSE2, 23-125 to 23-127

FEGO1_MX_CT3, 10-33                 FF_LOAD, 23-125 to 23-127

FEGO1_MX_CT4, 10-33                 FF_N, 23-125 to 23-127

FEGO1_MX_CT5, 10-33                 FF_OL_DM, 23-125 to 23-127

FEGO1_MX_CT6, 10-33                 FF_OL_FAULT, 23-125 to 23-127

FEGO1_MX_CT7, 10-33                 FF_OL_FDBACK, 23-125 to 23-127

FEGO1_MX_CT8, 10-33                 FF_OL_FMEM, 23-125 to 23-127

FEGO1_MX_CT9, 10-33                 FF_OL_HOLD, 23-125 to 23-127

FEGO1_MX_TOT, 10-38                 FF_VSBAR, 23-125 to 23-127

FEGO2_C_MIN1, 10-35 to 10-36        FFFFh (READ), 8-311

FEGO2_C_MIN2, 10-35 to 10-36        FFG_ACT, 23-26 to 23-29, 23-37 to

FEGO2_C_MIN3, 10-35 to 10-36            23-39, 23-41, 31-12 to 31-13,

FEGO2_C_MIN4, 10-35 to 10-36            31-34

Index-22

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FFG_CID, 23-27, 23-29, 23-39,       FFS_SW (READ), 15-7

35-80                           fgbeta, 8-185

FFG_CID (READ), 5-132               fgbeta1 (WRITE), 8-181

FFG_CPP, 31-86, 35-81               fgbeta2 (WRITE), 8-181

FFG_CSD11, 8-43, 8-272, 11-113 to   FIECT_3_CL, 8-8

11-114, 11-117, 11-127 to       FIECT_3_SH, 8-8

11-128, 11-154, 11-179          FIL_HYST, 24-54, 31-70, 31-74

READ, 8-82 to 8-83                FILFRC, 24-14

FFG_CSD11 (READ), 8-42              FILFRC_IN, 24-14

FFG_CSD21, 8-43, 8-271, 11-113 to   FILHYS, 11-131, 11-169, 18-33,

11-114, 11-121, 11-129 to           23-141 to 23-142

11-130, 11-155, 11-179          FILOAD_3, 8-8

READ, 8-82 to 8-83                FINAL CYLINDER AIRCHARGE (CDAM0),

FFG_CSD21 (READ), 8-42                  24-19

FFG_ECT, 23-26 to 23-29, 23-37 to   FINAL FAN DELIVERY (CDAL0), 16-23

23-39, 23-41, 31-14, 31-32 to   FINAL SPARK CALCULATION (CDAN0),

31-33, 31-36, 35-29, 35-32          9-4

FFG_ECT (READ), 13-79               final_calc (FCALL), 8-314

FFG_EGO11, 11-154, 23-37 to 23-38   final_calc (FUNCTION), 8-317

FFG_EGO12, 11-155, 23-37 to 23-38   FINCECT_2, 8-8

FFG_EGO21, 11-155, 23-37 to 23-38   FINCTM_1H, 8-8

FFG_EGO22, 11-156, 23-37 to 23-38   FINCTM_1L, 8-9

FFG_EGOnm, 23-26 to 23-27, 23-29    FINCTR_1, 8-8 to 8-9

FFG_EGR, 12-47, 12-49, 23-26,       FINCTR_2, 8-9

23-29, 23-36                    FINVSBP_1, 8-9

FFG_EGR (READ), 8-299               FINVSBP_2, 8-9

FFG_EGR_LST, 12-49, 12-54           FIRPM_3, 8-9

FFG_LAMBSE1, 23-26 to 23-27,        FIRST_MPH, 31-83

23-29, 23-38 to 23-39           FIRST_OSS, 25-92

WRITE, 8-99                       FIRST_PIP, 6-7, 8-200, 12-17,

FFG_LAMBSE2, 23-26 to 23-27,            23-112, 29-28, 29-30 to 29-31

23-29, 23-38 to 23-39           FIRST_PIP_LO, 29-28, 29-31

WRITE, 8-99                       FIRST_SEQNTL, 8-199, 8-202, 8-208

FFG_MAF, 23-26 to 23-29, 23-37 to   FIRST_SYNC, 8-168, 29-23

23-39, 23-41, 24-56, 31-71      FITP_3, 8-9

FFG_MISFIRE, 23-26 to 23-27,        FKARCIDLE, 24-30

23-29, 23-36, 23-39, 35-35      FKEXIT_MIN, 9-40

FFG_MISFIRE (READ), 5-133, 8-299    FL_OLTMR_337, 8-26

FFG_PIP, 23-26 to 23-29, 23-37 to   FL_OLTMR_338, 8-26

23-39, 31-71, 35-83 to 35-84    FLG_11_LN, 11-139, 11-143 to

FFG_PURG, 15-63, 23-26, 23-29,          11-144, 11-150

23-36 to 23-37                  FLG_11_RH, 11-139, 11-145, 11-148,

FFG_SAIR, 17-25, 17-29, 23-26,          11-150

23-29, 23-36 to 23-37           FLG_12_LN, 11-139, 11-144, 11-148

FFG_SAIR (READ), 8-299                  to 11-150

FFG_TOT, 25-26, 31-34, 31-54 to     FLG_12_RH, 11-139, 11-147 to

31-56                               11-148, 11-150

FFG_TOT (READ), 27-53               FLG_21_LN, 11-139, 11-144, 11-146,

FFG_TP, 23-26 to 23-29, 23-37 to        11-148, 11-151

23-39, 23-41, 31-68 to 31-69,   FLG_21_RH, 11-139, 11-143, 11-151

31-71, 31-74 to 31-75           FLG_22_LN, 11-139, 11-144, 11-147

FFG_TP (READ), 13-79                    to 11-148, 11-151

FFG_VS, 23-26 to 23-27, 23-29,      FLG_22_RH, 11-139, 11-144, 11-151

23-38 to 23-39, 23-41, 25-99    FLG_4X4L, 25-30, 25-57, 26-11 to

FFS_FMFLG (READ), 15-7                  26-14, 26-19, 26-36, 27-28

Index-23

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLG_AC_CMP, 25-71 to 25-72          FLG_FRST_DS, 26-4 to 26-5, 26-26,

FLG_AC_CTL, 25-71 to 25-72, 28-8,       26-39, 27-10

28-88                           FLG_FRST_NOV, 25-29, 32-4

FLG_ASILT, 27-15, 27-55             FLG_FRST_SPK, 26-66 to 26-67

FLG_ASILT_LS, 27-55                 FLG_FRST_TCS, 25-13

FLG_AUT41, 26-28, 27-38 to 27-39    FLG_FRST_TV, 26-48, 27-7, 27-16

FLG_AUT42, 26-28, 27-38 to 27-39        to 27-17, 28-49

FLG_BCV_MON, 25-65 to 25-66         FLG_FWD_REV, 27-9 to 27-10, 27-21,

FLG_BCV_ON, 25-65 to 25-66              27-55

FLG_CLUP, 28-8, 28-25 to 28-27,     FLG_HIGH_ALT, 28-16

28-33 to 28-34, 28-67, 28-72,   FLG_HIGH_SLP, 28-54, 28-56

28-77                           FLG_HRD_EN, 28-7 to 28-8

FLG_CLUP_LST, 28-25                 FLG_HRD_JMP, 28-67, 28-78 to

FLG_CONT_MON, 23-16 to 23-17,           28-79

23-41                           FLG_HRD_LK, 28-7 to 28-8, 28-78

FLG_CRV_DS, 28-38 to 28-40, 28-53,      to 28-80

28-67                           FLG_HRD_LST, 28-7

FLG_CRV_LK, 25-72, 28-7 to 28-8,    FLG_HRD_MODE, 28-67, 28-78 to

28-33, 28-40, 28-72, 28-78          28-80

FLG_CRV_LST, 28-38, 28-53           FLG_HS_M1, 26-30

FLG_CT_LST, 28-14                   FLG_HT_MOD, 28-6 to 28-7, 28-60

FLG_DASCLP4, 13-67                  FLG_IMRC_HLD, 18-12, 25-88

FLG_DASMNQ, 13-8 to 13-9, 13-67     FLG_IMRC_TM, 25-88

FLG_DASMNQ LOGIC, 13-9              FLG_MAN42, 26-27, 27-38

FLG_DB_REG, 28-34, 28-88            FLG_MLUS_FM, 28-17, 28-19, 28-21

FLG_DE_CMGR, 26-35, 26-39, 27-13    FLG_MLUS_FM (READ), 5-132

FLG_DFSO_LOAD, 8-130 to 8-131       FLG_NE_WAIT, 26-10 to 26-11,

FLG_DFSO_NOVS, 8-130 to 8-131           26-13 to 26-14

FLG_DIS_CSM, 25-9, 28-6, 28-59,     FLG_NEG_CAP, 28-77

28-62                           FLG_NEU_DRV, 27-8 to 27-9

FLG_DLY_SOSC, 28-27, 28-70          FLG_NEU_REV, 27-8 to 27-9

FLG_DN_TQM, 9-90                    FLG_NEW_NOV, 25-29

FLG_DNS_CTL, 28-8, 28-25, 28-28,    FLG_NOV_KAM, 25-29, 26-58, 32-4

28-33, 28-67, 28-71 to 28-72    FLG_NV_SHFT, 26-18 to 26-19,

FLG_DNS_EXIT, 28-25, 28-28, 28-33       28-25

FLG_DRV_REV, 27-8 to 27-9           FLG_OL_320A, 8-24, 8-26 to 8-27

FLG_DYN_CD, 27-28, 29-20            FLG_OL_337, 8-24, 8-26

FLG_ECTSTABLQ, 8-14, 8-26, 8-28,    FLG_OL_337 (READ), 8-27

8-44, 8-49, 8-51                FLG_OL_338, 8-24, 8-26 to 8-27

FLG_EGO_ER, 11-138, 11-142          FLG_OL_VS, 8-24, 8-27

FLG_EGO_IDLE, 11-46, 11-50          FLG_OLTMR_VSBAR, 8-26

FLG_ENG_IN, 27-7, 27-9 to 27-10,    FLG_OLUP, 28-25 to 28-27, 28-67,

27-20, 27-55                        28-77, 28-80

FLG_ENG_LK, 28-8                    FLG_OLUP_LST, 28-25, 28-80

FLG_ENG_LS, 27-7                    FLG_OPEN, 26-58, 28-7 to 28-8,

FLG_ENG_LST, 27-55                      28-26, 28-54, 28-56, 28-77,

FLG_ENG_TV, 27-10 to 27-11, 27-21       28-79

FLG_EPC_RMP, 26-66, 26-71, 27-31    FLG_OPEN_LST, 28-7, 28-56, 28-77

FLG_FMM_CC, 28-65                   FLG_OPN_BS, 28-26, 28-72

FLG_FMM_LK, 28-7, 28-39, 28-65,     FLG_OT_LK, 28-3, 28-7, 28-14 to

28-79                               28-18, 28-38, 28-60, 28-77

FLG_FRST_CM, 25-4, 26-27 to 26-28,  FLG_OT_LK (READ), 5-134

26-43 to 26-44, 26-58, 26-66,   FLG_PID_CTL, 28-60, 28-72, 28-76,

27-27, 27-39, 28-16, 28-25,         28-79 to 28-80, 28-87 to

28-70                               28-88

Index-24

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FLG_PWM_MON, 23-162, 25-66 to       FLG_TOT_WRM, 25-37

25-67                           FLG_TP3_LCK, 28-6 to 28-7

FLG_PWM_ON, 25-66 to 25-67          FLG_TP_CT, 27-55, 28-14

FLG_PWR, 26-43 to 26-44, 26-58,     FLG_TQ_TRN, 9-26

27-11 to 27-13, 27-38, 29-16    FLG_TQ_TRUN, 25-17

to 29-17                        FLG_TQM_ENA, 9-51, 25-37

FLG_REP_ENA, 9-90 to 9-91, 26-71    FLG_TR_ENG, 27-55

FLG_REV_DRV, 27-8 to 27-9           FLG_TRIG_RMP, 27-39 to 27-40

FLG_RLS, 4-32, 26-73                FLG_TRNS_BIT, 25-83, 25-85

FLG_RLS_HP, 26-73                   FLG_TST_BRK, 28-60 to 28-61

FLG_RMP_INIT, 26-71                 FLG_TST_CMP, 25-63, 25-65

FLG_RMP_OPEN, 28-67, 28-79 to       FLG_TST_FP, 25-63 to 25-64

28-80, 28-87                    FLG_TVENG_CD, 26-48, 27-21, 29-19

FLG_SF_AUTO, 26-27, 26-43 to        FLG_TVENG_MD, 26-48, 27-10, 27-21,

26-44, 26-58                        29-19

FLG_SFT_DN, 25-4, 26-5, 28-27 to    FLG_TVSTR_CD, 27-16 to 27-17,

28-28                               29-19

FLG_SFT_IN, 25-52, 25-63, 25-71,    FLG_UCTL_LST, 28-25, 28-70

26-4, 26-10, 26-23, 26-27 to    FLG_UN_ALT, 28-16, 28-19, 28-21

26-28, 26-44, 26-59, 26-66,     FLG_UN_BRK, 28-15, 28-18, 28-21

27-11 to 27-13, 27-27 to        FLG_UN_CT, 28-14, 28-17 to 28-18,

27-28, 27-37, 27-40, 28-17,         28-21

28-38 to 28-40, 28-54, 28-60    FLG_UN_MDN, 28-16, 28-19, 28-21

FLG_SFT_MDN, 26-43 to 26-44         FLG_UN_PRN, 28-16, 28-18, 28-21

FLG_SFT_STRT, 28-25, 28-27, 28-70,  FLG_UN_SPD, 28-17, 28-19, 28-21

28-72                           FLG_UN_TEMP, 28-16, 28-19, 28-21

FLG_SFT_UP, 25-4, 26-5, 27-12,      FLG_UN_TRA, 28-14, 28-18, 28-21

27-30, 27-39 to 27-40, 28-25,   FLG_UN_TRD, 28-15, 28-18, 28-21

28-27, 28-39 to 28-40           FLG_UN_ULSF, 28-16, 28-20 to

FLG_SFT_VAL, 26-58 to 26-59             28-21

FLG_SFT_VER, 26-58 to 26-59         FLG_UNC_UNLK, 28-6, 28-16, 28-21,

FLG_SFTTOPID, 28-67, 28-87              28-25, 28-40, 28-65, 28-79

FLG_SLP_CTL, 25-71 to 25-72         FLG_UNLK_DS, 26-26, 26-39, 28-8

FLG_SPK_LST, 27-31                      to 28-9, 28-33

FLG_SPK_TQM, 9-52, 9-69, 9-90 to    FLG_UP_NE, 26-10 to 26-11, 26-13

9-91, 26-66 to 26-67, 26-71,        to 26-14, 26-44

27-30, 27-39 to 27-40           FLG_UP_TQM, 9-52, 25-88, 26-66,

FLG_SS_1, 4-32, 26-49 to 26-50          26-71

FLG_SS_1 (READ), 5-134              FLG_UPS_CTL, 28-25, 28-27, 28-67,

FLG_SS_2, 4-32, 26-49, 26-51            28-70 to 28-72

FLG_SS_2 (READ), 5-134              FLG_UPS_EXIT, 28-25 to 28-27,

FLG_SS_MON, 23-162, 25-67, 26-59,       28-33

26-61, 28-57                    FLG_UPS_JMP, 28-67, 28-79 to

FLG_STABLECT, 8-14                      28-80

FLG_STALL, 6-6, 8-242, 15-48,       FLG_VE_DSGR, 25-71, 26-18 to

15-92                               26-19, 27-13

FLG_STR_LST, 28-67, 28-87           FLG_VER_BRK, 28-60 to 28-62

FLG_STROKE, 28-67, 28-72, 28-77,    FLG_W12_AKD, 25-23 to 25-24

28-79                           FLGTCAPNXT, 28-72

FLG_TCAPNXT, 28-70 to 28-71         FLOW_RUNNING, 13-34, 13-40, 15-20,

FLG_TCS, 25-8, 25-13, 25-46 to          15-91 to 15-93, 15-103 to

25-47, 25-84                        15-105

FLG_THFT_FCO, 5-105 to 5-108,       FLOW_RUNNING (READ), 15-22

8-143                           FLOW_RUNNING (WRITE), 15-22

FLG_TIP_OUT, 25-23, 26-19, 26-44    FMEM, 29-16

Index-25

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FMMDSD, 13-8                        FN1323A, 8-107

FMMISC, 13-2, 13-8                  FN1323D, 8-107

FMMMLP, 25-47, 25-84                FN1325A, 8-76 to 8-77, 8-79 to

FN004, 12-36                            8-80

FN012(LOAD), 9-20                   FN1327, 8-202, 8-214 to 8-215,

FN013(LOAD), 9-21                       8-223, 35-36

FN016(N), 9-21                      FN1329, 8-202, 8-212, 8-215,

FN017(N), 9-21                          8-223, 8-237

FN018B(ATMR3), 9-21                 FN1338 (READ), 8-115

FN020C(ECT), 9-21                   FN1347 (READ), 8-116

FN021, 8-281                        FN1348 (READ), 8-116

FN021A, 8-72                        FN1350, 8-136

FN030A, 9-58, 9-62                  FN1351, 8-287, 11-187

FN035, 13-75, 24-59                 FN1351E, 8-287

FN036I, 24-7, 24-9                  FN1352, 8-284

FN039E, 8-281                       FN1353E, 8-281

FN040A, 24-29                       FN1353EB2, 8-281

FN041, 8-72                         FN1360, 8-44

FN052BF, 24-27                      FN1361_H_RES, 8-44

FN052BP, 24-26                      FN1362, 8-45

FN052C, 24-29                       FN1362_ALT, 8-45

FN052M, 24-26                       FN143A, 9-58 to 9-59

FN052N, 24-26                       FN145A, 9-58, 9-65

FN059, 24-26, 29-7, 29-11           FN146B, 9-59

FN059T, 24-26                       FN1615_DATA, 35-19

FN067D, 25-51                       FN1615_DATA(write), 35-33

FN070D, 8-72                        FN1615A, 25-17

FN070P, 25-51                       FN1615A(read), 35-33

FN074B, 12-36                       FN1615M(read), 35-33

FN096, 31-75                        FN1615M_CAL(read), 35-33

FN1036A, 24-26, 24-62               FN1616, 25-17

FN1036B, 24-26, 24-62               FN1619A, 25-51

FN1037, 24-63                       FN179, 9-26

FN1039, 24-63                       FN185, 18-7

FN1052, 24-26                       FN186, 18-7

FN1132, 9-58                        FN1862D, 13-49

FN1134, 9-58                        FN1862N, 13-49, 13-52

FN1135, 9-59                        FN2000, 9-21 to 9-22

FN1144C, 9-58, 9-62, 9-64 to 9-65   FN2001, 9-21 to 9-22

FN1147B, 9-58, 9-62                 FN201, 12-23

FN123A, 24-33                       FN202, 12-40

FN123E, 24-33                       FN2020, 9-48

FN125, 8-247                        FN203, 12-6

FN129A, 8-49                        FN2100, 9-14

FN12COMP, 26-13                     FN211, 12-11

FN12DCP, 27-38                      FN2110, 9-14

FN12DYTX_VS, 27-38                  FN2115, 9-14

FN12P, 26-13                        FN2120, 9-14

FN12S, 26-13                        FN2120_ALT, 9-14

FN12S_4X4L, 26-13                   FN212A, 8-49, 12-11

FN12T, 27-12                        FN2140, 9-14

FN1315, 8-122                       FN2160A, 9-73

FN1321, 8-101, 8-106 to 8-107       FN217A, 12-11

FN1322AL, 8-101, 8-107              FN21_HOT, 26-10

FN1322DL, 8-101, 8-107              FN21COMP, 26-13

Index-26

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FN21DCP, 27-38                      FN338, 8-26

FN21DYTX_VS, 27-38                  FN339, 8-277

FN21P, 26-13                        FN339B, 8-257

FN21S, 26-13                        FN342, 8-275

FN21S_4X4L, 26-13                   FN344, 8-275

FN21T, 27-12                        FN346, 8-275, 11-187

FN220, 12-11                        FN346A, 8-287

FN2200, 9-18                        FN348A, 8-136

FN2220, 9-18                        FN348BP, 8-136

FN2230, 9-18                        FN348RPM, 8-136

FN2240, 9-18                        FN34COMP, 26-12

FN2250, 9-18                        FN34DCP, 27-39

FN2260A, 9-73                       FN34DYTX_VS, 27-39

FN2290, 9-61                        FN34P, 26-12

FN2300, 9-31                        FN34PPH, 26-12

FN230A, 12-6                        FN34S, 26-12

FN2320, 9-31                        FN34T, 27-12

FN2360A, 9-73                       FN367, 8-137

FN23COMP, 26-12                     FN371, 8-18

FN23DCP, 27-38                      FN371_ALT, 8-18

FN23DYTX_VS, 27-38                  FN384, 6-6

FN23P, 26-12                        FN385, 6-7

FN23PPH, 26-12                      FN385(ECT), 6-6

FN23S, 26-12                        FN387_ALT, 8-137

FN23S_4X4L, 26-12                   FN387A, 8-137

FN23T, 27-12                        FN393F, 8-19

FN246A, 12-19                       FN393R, 8-19

FN247A, 12-19                       FN394F, 29-5

FN248, 12-11                        FN394R, 29-5

FN2LCOMP, 28-38                     FN396C, 8-49

FN2LH, 28-38                        FN3LCOMP, 28-38

FN2LS, 28-38                        FN3LH, 28-38

FN2UCOMP, 28-38                     FN3LS, 28-38

FN2UH, 28-38                        FN3UCOMP, 28-38

FN2US, 28-38                        FN3UH, 28-38

FN300, 8-44                         FN3US, 28-38

FN301, 8-49                         FN422A_EPC, 27-38

FN303, 8-50                         FN43COMP, 26-14

FN308, 8-49                         FN43DCP, 27-38 to 27-39

FN309, 8-49                         FN43DYTX_VS, 27-38 to 27-39

FN312S, 8-107                       FN43P, 26-14

FN313, 8-101, 8-106 to 8-107        FN43S, 26-14

FN315, 8-281                        FN43S_4X4L, 26-14

FN320A, 8-26                        FN43T, 27-12

FN321, 8-44                         FN441A, 8-45, 10-11

FN325, 8-44                         FN441B, 10-11

FN32COMP, 26-13                     FN441C, 10-11

FN32DCP, 27-38                      FN441T, 10-11

FN32DYTX_VS, 27-38                  FN442, 10-11

FN32P, 26-13                        FN443, 10-11

FN32S, 26-13                        FN443L, 10-11

FN32S_4X4L, 26-13                   FN4441, 10-11

FN32T, 27-12                        FN445L, 10-11

FN336, 8-122                        FN448, 8-45, 10-12

FN337, 8-26                         FN448A, 8-45, 10-12

Index-27

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FN449, 10-12                        FN711, 9-14

FN449A, 10-12                       FN712, 9-14

FN450, 10-13                        FN721, 9-18

FN450L, 10-13                       FN722, 9-18

FN4LCOMP, 28-38                     FN723, 9-18

FN4LH, 28-38                        FN724A, 9-18

FN4LS, 28-38                        FN725A, 9-18

FN4UCOMP, 28-38                     FN726, 9-18

FN4UH, 28-38                        FN730, 9-31

FN4US, 28-38                        FN731, 9-31

FN503, 18-10                        FN741, 9-22

FN600, 15-71                        FN741(STB_LOLD_TMR), 9-21

FN6000A (READ), 15-17               FN745, 9-48

FN602, 15-71                        FN746, 9-48

FN603, 15-72 to 15-73               FN766, 9-27, 9-33, 9-41, 25-17

FN605A, 15-71, 15-74                FN799, 9-26, 9-28, 9-33, 9-37,

FN606, 17-18                            9-51

FN607, 17-18                        FN8000, 13-2

FN610, 15-26                        FN803A, 13-19

FN615, 25-34                        FN812, 13-52

FN615(BP), 25-34                    FN813, 13-57

FN615TQ, 25-52                      FN815A, 13-41

FN616_SW, 27-20                     FN821B, 13-18

FN616F, 27-17, 27-20                FN822, 13-59

FN616F(TP_REL), 27-16               FN825A, 13-18

FN616R, 27-20                       FN825A_ALT, 13-18

FN617, 25-17, 27-27                 FN825B, 13-18

FN618A, 25-17                       FN826A, 13-19

FN619A, 25-17                       FN826B, 13-19

FN620, 27-47                        FN830, 13-3, 13-65

FN622, 27-47                        FN839, 9-14, 9-18

FN622A, 27-47                       FN851A, 13-36

FN623, 25-17                        FN852A, 13-36, 13-41

FN623(read), 35-33                  FN875D, 13-49, 13-52

FN627, 15-71                        FN875N, 13-49, 13-52

FN630 (READ), 15-19                 FN879, 13-3

FN631 (READ), 15-19                 FN880, 8-244, 13-18

FN641, 16-20                        FN881, 13-54

FN641F, 16-20                       FN881NC, 13-54

FN642, 16-22                        FN884, 13-8

FN649, 16-25                        FN884(TCSTRT), 13-2

FN654W, 31-38                       FN885, 13-54

FN666, 15-20                        FN885TE, 13-54

FN667, 15-20                        FN887, 13-54

FN689D, 26-14                       FN887NC, 13-54

FN689L, 28-65                       FN894, 13-67

FN689U, 26-14                       FN894H, 13-67

FN690, 25-37                        FN894L, 13-67

FN690B(INIT_TOT), 25-63             FN908A, 12-11

FN690TQM, 25-37                     FN908B, 12-11

FN700, 9-14                         FN911, 8-49

FN702, 9-14                         FN[GR_CM]SSLP, 28-7

FN703F, 31-11, 31-31, 31-53         FN_GEN_TMR, 8-257

FN703Q, 31-5                        FN_HTR_XFER, 11-78 to 11-79

FN704, 9-104                        FN_INPUT, 8-257

Index-28

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FN_LAMA1 (READ), 15-16              FOREGROUND FUEL PULSEWIDTH

FN_LAMT1 (READ), 15-16                  CALCULATION (CDAL0), 8-175

FN_PGM_BLEED, 15-55                 FOREGROUND INJECTOR TIMING -

FN_SRLK, 28-38                          IBETA CALCULATION (CDAI0),

FNCPTRS, 25-82                          8-204

FNCYL_OFF, 8-47                     FOREGROUND INJECTOR TIMING -

FNDCHOFF, 28-84                         TOTAL DELAY CALCULATION

FNDCHSLP, 28-84                         (CDAI0), 8-182

FNDCLOFF, 28-84                     FOREGROUND TRANSIENT FUEL LOGIC

FNDCLSLP, 28-84                         (CDAL0), 8-108

FNDN2SSLP_0, 28-9                   fox (FCALL), 8-304, 8-316

FNDN3SSLP_0, 28-9                   FP, 3-4

FNDNSSFT32, 28-72                   FP_2SPD_ERR, 4-33, 8-160

FNDNSSFT43, 28-72                   FP_2SPD_ERR (READ), 5-137

FNEXP, 10-10                        FP_AM_HYS, 8-146

FNFAOSCI (DEFINED), 8-315           FP_DRIVER, 8-148

FNFAOSCI (READ), 8-316              FP_EO_DT_SW, 8-152

FNFAOSCP (DEFINED), 8-303           FP_EO_HL_SW, 8-160

FNFAOSCP (READ), 8-304              FP_EO_ST_SW, 8-156

FNFP_VBAT, 8-146                    FP_ER_DT_SW, 8-152

FNFRIC_TQ, 25-17                    FP_ER_HL_SW, 8-160

FNFSC_ADD_D, 26-14                  FP_ER_ST_SW, 8-156

FNFSC_ADD_U, 26-14                  FP_HI_ENABL1, 8-146

FNFSC_CYL, 26-14                    FP_HI_ENABL2, 8-146

FNFSC_TPD, 26-14                    FP_N_ENA_CL, 8-146

FNFSC_TPU, 26-14                    FP_N_ENA_SH, 8-146

FNMISACT, 35-28                     FP_OT_DT_SW, 8-152

FNMISECT, 35-28                     FP_OT_ST_SW, 8-156

FNMISOK, 35-33                      FP_PU_DT_SW, 8-152

FNMISOK_DATA, 35-33                 FP_PU_ST_SW, 8-156

FNPCTQ12, 26-68                     FP_PU_TM, 8-152, 8-156

FNPCTQ23, 26-68                     FP_TYPE, 4-33, 8-141

FNPCTQ34, 26-68                     FP_TYPE (READ), 5-137

FNPWFE, 25-4                        FPL, 4-33, 8-149

FNSSCTL2, 26-48                     FPMP_ON_TMR, 8-143

FNTMSEQ32P4, 26-40                  FPMP_OSM_ENA, 8-152, 8-160

FNTOTDLY, 25-24                     FPMP_SMON_EN, 8-156

FNTQ2SRE, 26-68                     FPUMP_DC, 4-25, 8-143, 8-148,

FNTQ3SRE, 26-68                         8-156, 8-158

FNTQ4SRE, 26-68                     FPUMP_DC (READ), 5-146

FNTQ_MAX[fn_index], 25-17           FPUMP_FREQ, 4-25, 8-148

FNUPSSFT23, 28-72                   FPUMP_OFF_DC, 8-158

FNUPSSFT34, 28-72                   FPUMP_ON_DC, 8-158

FNVEGOSP (DEFINED), 8-303, 8-316    FPUMP_SPEED, 8-146, 8-149

FNVEGOSP (READ), 8-304, 8-316       FPUMP_SPEED (READ), 5-135

FNWRMENG, 8-14                      FPW_FG_ISF_A, 8-193

FNXTMINGR4, 26-40                   FR_97_COUNT, 5-22, 5-110

FNXVSBART_RT, 26-40                 FR_97_COUNT (WRITE), 5-112

FOFFLG2, 8-7                        FR_PAT_DATA, 5-109 to 5-110

FOFFLG2 (READ), 8-164               FR_PAT_DATA (WRITE), 5-112

FOFFLG3, 8-9                        FRDR1, 5-23

FOFFLG3 (READ), 8-164               FRECR, 5-17

FOREGROUND FUEL CALCULATIONS        FRECR (READ), 5-110

EXECUTION CONTROL (CDAL0),      FRECR (WRITE), 5-110, 5-112

8-169                           FREEZE FRAME (CDAM0), 23-121

Index-29

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FREQERR_BAND, 11-36                 FUEL_INJ_OUT, 8-219

FRST_ADP, 15-100                    FUEL_INJ_OUT (CALL), 8-213

FRST_ADP (READ), 15-8 to 15-9       FUEL_INJ_UPDATE_REQUEST, 8-193

FRST_ADP (WRITE), 15-9                DEFINITION, 8-189

FTP CATALYST MONITOR (CDAN0),       FUEL_INJ_UPDATE_REQUEST (CALL),

10-15                               8-179

FUEL PULSEWIDTH, MAIN FUEL OFF      FUEL_MAIN_OFF, 8-193, 8-219

(CDAI0), 8-190                    CALL, 8-189

Fuel Pump - V2.1_FPUMP (CDAN0),     fuel_malfunction(FUEL_MIL_ON,

8-139                               Pxxx), 23-50

FUEL PUMP CONTROL LOGIC (CDAK0),    FUEL_MIL_ON, 23-50 to 23-51,

8-142                               23-87, 23-110

FUEL PUMP OUTPUT CONTROL STRATEGY   FUEL_MIL_SW, 23-110

(CDAL0), 8-147                  FUEL_MON, 15-50, 23-44, 23-51,

FUEL PUMP PRIMARY CIRCUIT TEST          23-68, 23-71 to 23-72

(CDAM0), 8-150                    WRITE, 8-96

FUEL PUMP SECONDARY CIRCUIT TEST    FUEL_MON_LST, 23-44, 23-51

(CDAM0), 8-154                  FUEL_MON_LST := 0, 23-71

FUEL PUMP SELECTION LOGIC (CDAN0),  FUEL_PIP_INT_SYNC

8-139                             CALL, 8-171

FUEL PUMP SPEED CIRCUIT TEST        fuel_reset, 23-52

(CDAM0), 8-159                  FUEL_SCHEDULE, 8-201, 8-203

FUEL PUMP SPEED OUTPUT (CDAL0),     FUEL_SCHEDULE (DEF), 8-212

8-149                           FUEL_SERVICE, 8-199

FUEL SCHEDULING LOGIC (CDAM0),        CALL, 8-171 to 8-172

8-210                           fuel_service_entry_logic, 8-199,

FUEL SERVICE LOGIC (CDAI0), 8-195       8-203

FUEL STRATEGY OVERVIEW (CDAI0),     fuel_service_exit_check, 8-203

8-2                             FUEL_SOURCE1, 8-44, 8-47 to 8-48,

FUEL SYSTEM MONITOR (CDAM0), 8-90       11-50

fuel_bk1 (READ), 8-180              FUEL_SOURCE2, 8-44, 8-47 to 8-48,

fuel_bk1 (WRITE), 8-180                 11-50

fuel_bk2 (READ), 8-180              FUEL_SUM, 8-194, 8-231, 19-3

fuel_bk2 (WRITE), 8-180             FUEL_TOTAL_DELAY_CALC, 8-185

FUEL_BKPT, 8-136                    FUEL_TOTAL_DELAY_CALC (CALL),

FUEL_BKPT (READ), 8-180                 8-179

FUEL_CUT, 8-327                     FUEL_TST_ENA, 23-16 to 23-17

FUEL_CUT (WRITE), 8-164               READ, 8-95 to 8-96

FUEL_DYN_CALC, 8-223                FUEL_TST_RDY

CALL, 8-172                         READ, 8-96

FUEL_DYN_FIT, 8-225                   WRITE, 8-95

FUEL_ENA_OLD                        FUEL_TST_SW, 15-50, 23-17, 23-51,

WRITE, 8-95                           23-68, 23-135

FUEL_FG_CALC                        FUELPW1, 8-193 to 8-194, 8-202,

CALL, 8-172                           8-214 to 8-215, 8-219, 8-224,

FUEL_FG_CALC (DEFINITION), 8-179        8-277, 15-16

FUEL_FG_PIPEDG                      FUELPW1 (READ), 5-144, 8-181,

DEFINITION, 8-171                     8-213

fuel_fg_transient_calculations      fuelpw1 (READ), 8-181

CALL, 8-172                       FUELPW1 (WRITE), 8-136, 8-181

FUEL_FMEM, 23-17, 23-35, 23-39      fuelpw1 (WRITE), 8-180 to 8-181

FUEL_IBETA_CALC, 8-203, 8-208       FUELPW2, 8-193 to 8-194, 8-202,

FUEL_IN_SYNC, 8-168, 8-174, 8-185,      8-214 to 8-215, 8-219, 8-224,

8-199, 8-208, 29-23                 8-277, 15-16

fuel_init_mil(FUEL_MIL_ON), 23-51

Index-30

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FUELPW2 (READ), 5-144, 8-181,       GR_DS, 3-8, 25-23, 25-71, 26-4 to

8-213                               26-5, 26-18 to 26-19, 26-23,

fuelpw2 (READ), 8-181                   26-26, 26-28

FUELPW2 (WRITE), 8-136, 8-181       GR_DS, PRNDL = 3 OR 4 LOGIC

fuelpw2 (WRITE), 8-180 to 8-181         (CDAM0), 26-6

FULPIPALT_DT, 8-172                 GR_DS_LST, 3-8, 25-23, 26-4 to

26-5, 26-18 to 26-19, 26-23

GR_DS_TV, 26-4 to 26-5, 26-23,

GAIN_C, 28-87 to 28-88                  26-30, 26-32, 26-35, 26-39,

GAIN_D, 28-88                           27-10, 27-12 to 27-13, 27-28,

GAIN_I, 28-88                           27-30, 27-38 to 27-40, 28-9

GAIN_P, 28-88                       GR_OLD, 3-8, 26-27, 26-35, 26-39,

GEAR4TH, 13-64, 13-67, 30-4             26-43 to 26-44, 26-66, 28-20

GEAR4TMR, 13-67, 30-4               GR_SEQ_131, 26-23

GEAR_CUR, 3-8, 26-4 to 26-5,        GR_SEQ_141, 26-23

26-23, 26-48, 27-33, 27-40      GR_SEQ_241, 26-23

GEAR_OLD, 3-8, 26-5, 26-58, 27-33   GR_SEQ_311, 26-23

GENERIC IDLE SPEED CONTROL          GR_SEQ_411, 26-23

(CDAL0), 13-2                   GR_SEQ_421, 26-23

get_similar_conds(Pxxx), 23-93 to   GR_SEQ_PNTR, 26-23, 26-35, 26-39

23-94                           GR_SEQ_PNTR CALCULATION, 26-4

GOOD_12_LVL, 26-61                  GR_SEQ_PNTR CALCULATION (CDAA0),

GOOD_23_LVL, 26-61                      26-20

GOOD_34_LVL, 26-61                  GRAD_CHNG12, 10-28

GOOS_ER_INIT, 9-58                  GRAD_CHNG22, 10-29

GOOS_ER_TEST, 9-58, 9-67            GRCMKIV, 32-4

GOOS_KNK_DET, 9-58, 9-67            GRMSFT, 26-26

GOOS_SPK_INT, 9-58                  GRRAT, 26-48

GOOS_SPK_TST, 9-67                  GRRAT1, 26-48

GOOSE_ER_CODE, 9-58                 GRRAT2, 28-34

GOOSE_KTS_B, 9-58

GOOSE_LAM, 9-58

GPAS_CLIP, 13-14, 13-27             HALF_FUEL, 8-24, 8-47, 8-240

GR_CM, 3-8, 9-52, 13-9, 23-156,     HALF_FUEL (WRITE), 8-164

25-17, 25-24, 25-64 to 25-65,   HARDWARE-SPECIFIC INPUT

25-71, 26-11 to 26-14, 26-23,       PROCESSING (CDAN0), 4-38

26-26 to 26-28, 26-30, 26-32,   HARDWARE-SPECIFIC OUTPUT

26-35 to 26-36, 26-39 to            PROCESSING (CDAN0), 4-42

26-40, 26-43 to 26-44, 26-48,   HC_ACT_MUL1 (READ), 8-115

26-58, 26-60 to 26-61, 26-66,   HC_ACT_MUL2 (READ), 8-115

26-68, 27-10, 27-12, 27-28 to   HC_ACT_MUL3 (READ), 8-115

27-29, 28-7, 28-16, 28-20,      HC_ECT_MUL1 (READ), 8-115

28-25 to 28-26, 28-33, 28-38    HCAMFG, 13-8, 13-26, 13-32, 13-48

to 28-39, 28-65, 28-72, 30-4,     READ, 8-72, 8-82

32-4                            HCAMFG LOGIC (CDAL0), 13-47

GR_CM (READ), 5-144                 HCAMSW, 8-72, 8-82

GR_CM, PRNDL = 1 LOGIC (CDAM0),     HCDFLG, 13-19

26-29                           HCNCLP, 13-18

GR_CM, PRNDL = 2 LOGIC (CDAG0),     HCTMR, 13-19

26-31                           HEATER_HP11, 11-75 to 11-77,

GR_CM, PRNDL = 3 OR 4, DOWNSHIFT        11-100 to 11-101

LOGIC (CDAN0), 26-37            HEATER_HP12, 11-75 to 11-77,

GR_CM_KAM, 26-27, 32-4                  11-100 to 11-101

GR_CM_LST, 3-8, 13-9, 23-156,       HEATER_HP21, 11-75 to 11-78,

26-26 to 26-28, 26-30, 28-26        11-100 to 11-101

Index-31

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

HEATER_HP22, 11-75 to 11-77,        HEGOx1, 8-275

11-79, 11-101                   HFDLTA, 9-81, 29-31

HEATER_STATE, 11-71, 11-74, 11-78   HICAM, 13-20, 13-25, 13-48

to 11-79, 11-81 to 11-85,       HICOMP, 8-136

11-90 to 11-92, 11-97 to        HICOMP (READ), 8-116

11-99                           HICOMP (WRITE), 8-116

HEDF, 4-31, 16-35                   HICOMP_INIT (READ), 8-115

HEDF_STATUS, 4-31, 16-17, 16-35     HICOMP_INIT (WRITE), 8-115

HEDF_STATUS (READ), 5-138           HICOMP_MIN (READ), 8-116

HEDFHP, 4-31, 13-26, 16-8, 16-12,   HIGH DATA RATE ELECTRONIC SPARK

16-17, 16-35                        (CDAM0), 9-74

HEGO BYPLES CALCULATION (CDAN0),    HIGH SPEED DIGITAL INPUTS (HSI),

11-186                              4-10

HEGO HARDWARE TRANSFER FUNCTION     HIGH SPEED DIGITAL OUTPUTS (HSO),

(CDAN0), 11-170                     4-13

HEGO INPUT TRANSFER FUNCTION        HIGH SPEED FAN STRATEGY (CDAN0),

(CDAM0), 11-180                     16-5

HEGO LAMBSE CLIP STUB (CDAN0),      HLD_IMRC_TM, 25-88

11-182                          HLD_IMRC_TMR, 25-88

HEGO Monitor - V10.6_EGO_MONITOR    HLDTM1, 8-24

(CDAN0), 11-2                   HLDTM4, 8-27

HEGO SENSOR INPUT PROCESSING        HLDTM_CR, 8-24

(CDAN0), 11-157                 HLODH, 8-26

HEGO-11, 4-19                       HNO_CL, 8-7

HEGO-12, 4-19                       HNO_SH, 8-7

HEGO-21, 4-19                       HO2S-11, 4-19

HEGO-22, 4-19                       HO2S-12, 4-19

HEGO_CONFIG, 8-274, 10-31, 10-35,   HO2S-21, 4-19

10-39, 10-41, 11-47, 11-49,     HO2S-22, 4-19

11-55, 11-142, 11-144, 11-147   HOSE_TST_AMB, 12-36, 12-38, 12-49,

to 11-148, 11-155 to 11-156,        12-51 to 12-53

11-162, 11-168                  HOSE_TST_CNT, 12-36 to 12-37,

HEGO_CONFIG (DEFINED), 8-300,           12-49

8-311 to 8-312, 8-318           HOT INJECTOR COMPENSATION (CDAN0),

HEGO_CONFIG (READ), 8-300, 8-311,       8-112

8-314, 8-319                    HOT LOCKUP LOGIC (CDAA0), 28-3

hego_fault (READ), 5-134            HOT21_ADDR, 26-10, 26-13

HEGO_FAULT (WRITE), 8-43            HOT21_C, 26-10

HEGO_FC_AVG, 11-31 to 11-32         HOT21_S, 26-10

HEGO_FDBACK (READ), 8-43            HP_4X4L, 25-57

hego_fdback (READ), 5-134           HP_CID, 8-174

HEGO_FDBACK (WRITE), 8-42           HP_CIDSEL, 4-11, 8-168, 35-77 to

HEGO_WARM, 8-14                         35-78

HEGOHTR11, 4-37, 10-11, 11-84,      HPACL, 4-35, 20-3

11-96 to 11-97, 11-101          HSF_ACP_OTMR, 16-8 to 16-9

HEGOHTR11 (READ), 5-134             HSF_ACP_ROTM, 16-8

HEGOHTR12, 4-37, 11-84, 11-96 to    HSF_FLG, 13-26, 16-12 to 16-13,

11-97, 11-102                       16-35

HEGOHTR12 (READ), 5-134             HSF_FLG (READ), 5-133

HEGOHTR21, 4-37, 11-84, 11-96 to    HSF_RQST, 16-4, 16-8, 16-12 to

11-97, 11-101                       16-13

HEGOHTR21 (READ), 5-134             HSFAN_DELAY, 16-12

HEGOHTR22, 4-37, 11-85, 11-96 to    HSFECT1_SH, 16-8

11-97, 11-100, 11-102           HSFECT2_SH, 16-8

HEGOHTR22 (READ), 5-134             HSFECT_CL, 16-8

Index-32

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

HSFLOD, 16-8                        HTR_VOLT_NOM, 11-78 to 11-79

HSFRPM, 16-8                        HTRCM11, 11-70, 11-77 to 11-78

HSFVS, 16-8                         HTRCM11 (READ), 5-146

HSPFLG, 8-7, 8-24, 8-49, 9-26       HTRCM12, 11-70, 11-77, 11-79

HTR_EXE_TM, 11-73                   HTRCM12 (READ), 5-146

HTR_EXE_TMR, 11-71, 11-73           HTRCM21, 11-70, 11-77 to 11-78

HTR_FAULT11, 4-37, 11-76, 11-84,    HTRCM21 (READ), 5-146

11-86 to 11-87, 11-89, 11-96,   HTRCM22, 11-70, 11-77, 11-79

11-101                          HTRCM22 (READ), 5-146

HTR_FAULT11 (READ), 5-134           HTRCM_CNTS, 4-9, 11-78 to 11-79

HTR_FAULT12, 4-37, 11-76, 11-84,    HTRCM_SW, 11-75, 11-78

11-86 to 11-87, 11-89, 11-96,   HVQ1, 15-70 to 15-71

11-101 to 11-102                HWRT, 3-7

HTR_FAULT12 (READ), 5-134           HYST2, 7-3 to 7-4

HTR_FAULT21, 4-37, 11-76, 11-84,    HYSTS, 7-3 to 7-4

11-86 to 11-87, 11-89, 11-96,   HYSTSPD, 29-16 to 29-17

11-101                          HYSTST, 29-16 to 29-17

HTR_FAULT21 (READ), 5-134

HTR_FAULT22, 4-37, 11-76, 11-85     I/O CHANNEL ASSIGNMENTS (CDAN0),

to 11-87, 11-89, 11-96,             4-2

11-101 to 11-102                I4X4L, 4-22, 25-57, 31-80

HTR_FAULT22 (READ), 5-134           I4X4L_LST, 25-57

HTR_MALF11, 11-71, 11-86 to 11-88,  IAC (IDLE SPEED CONTROL) SCCD

11-90 to 11-91                      OUTPUT (CDAL0), 13-83

HTR_MALF12, 11-71, 11-86 to 11-88,  IAC_ER_TM1 (READ), 13-79

11-91                           IAC_ER_TM2 (READ), 13-79

HTR_MALF21, 11-71, 11-86 to 11-88,  IAC_ER_TM3 (READ), 13-80

11-91                           IAC_ER_TM4, 13-86

HTR_MALF22, 11-71, 11-86 to 11-88,  IAC_ER_TMR1 (READ), 13-79

11-91                           IAC_ER_TMR1 (WRITE), 13-80

HTR_MON11, 11-71, 11-75, 11-84,     IAC_ER_TMR2 (READ), 13-79

11-87 to 11-88, 11-92, 11-139   IAC_ER_TMR2 (WRITE), 13-80

HTR_MON12, 11-71, 11-75, 11-84,     IAC_ER_TMR3 (READ), 13-80

11-87 to 11-88, 11-92, 11-140   IAC_ER_TMR3 (WRITE), 13-80

HTR_MON21, 11-71, 11-75, 11-84,     IAC_ER_TMR4, 13-86

11-87 to 11-88, 11-92, 11-139   IAC_ERR1, 13-39

HTR_MON22, 11-71, 11-75, 11-84,     IAC_ERR1 (READ), 13-79 to 13-80

11-87 to 11-88, 11-92, 11-140   IAC_ERR1 (WRITE), 13-79

HTR_RET_MX11, 11-70, 11-72          IAC_ERR2, 13-39

HTR_RET_MX11 (READ), 5-145          IAC_ERR2 (READ), 13-79 to 13-80

HTR_RET_MX12, 11-70, 11-72          IAC_ERR2 (WRITE), 13-79

HTR_RET_MX12 (READ), 5-145          IAC_ERR3, 13-32 to 13-33

HTR_RET_MX21, 11-70, 11-72          IAC_ERR3 (READ), 13-80

HTR_RET_MX21 (READ), 5-145          IAC_ERR3 (WRITE), 13-79

HTR_RET_MX22, 11-70, 11-72          IAC_HI_LIM, 13-39

HTR_RETRY11, 11-71 to 11-72,        IAC_ISFL_OBD (READ), 13-79

11-80, 11-84, 11-87 to 11-92    IAC_LO_LIM, 13-39

HTR_RETRY12, 11-71 to 11-72,        IAC_MON_TM (READ), 13-80

11-80, 11-84, 11-87 to 11-89,   IAC_MON_TMR (READ), 13-80

11-91 to 11-92                  IAC_MON_TMR (WRITE), 13-80

HTR_RETRY21, 11-71 to 11-72,        IAC_NO_CUR, 4-36, 13-86

11-80, 11-84, 11-87 to 11-92    IAC_NO_CUR (READ), 5-137

HTR_RETRY22, 11-71 to 11-72,        IAC_OT_REQ, 13-85

11-80, 11-85, 11-87 to 11-89,   IAC_OVER_CUR, 4-36, 13-86

11-91 to 11-92                  IAC_OVER_CUR (READ), 5-137

Index-33

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

IAC_RUN_TM (READ), 13-79            IEGO22, 4-19, 11-168

IAC_TM4_FLG, 13-86                  IEGR, 4-8, 12-3, 12-16 to 12-17

IAC_TST_DC, 13-86                   IEGR (READ), 5-144

IACPRES_MAX, 31-6                   IERPMH, 12-16

IACPRES_MIN, 31-6                   IERRINT, 28-87

IACPRES_SEN, 4-8, 31-5 to 31-6      IERRMAX, 28-88

IACPRES_SEN (READ), 5-146           IERRMIN, 28-88

IACPSW, 4-8, 31-90                  IGNR_CPP, 35-81

IBAP, 3-8, 29-10, 29-14             IGNTST_RSIR, 9-96 to 9-97

IBAP (WRITE), 29-7, 29-11           IGNTSTADVTOL, 9-96

IBETA1, 8-201, 8-208 to 8-209,      IGNTSTMAXADV, 9-96

8-219                           IGNTSTMINADV, 9-96

IBETA2, 8-199, 8-201, 8-208 to      IGR_DS, 26-4, 26-11 to 26-14,

8-209, 8-219                        26-18 to 26-19, 26-23, 28-25

IBGPSI, 13-26, 13-32                    to 28-26, 28-33

ICHAN_1, 4-7, 4-39                  IIVPWR, 4-20, 31-88

IDBLOCK_ADR, 5-141                  IMAF, 4-19, 24-7, 24-9, 24-11,

IDBLOCK_ADRS, 5-130                     24-50 to 24-53

idblock_adrs, 5-144                 IMAF (READ), 5-146

IDBLOCK_FMT, 5-141                  IMCC_DC, 4-25

IDBLOCKH_ADR, 5-141                 IMCC_FAULT, 4-31

IDCMUL, 13-2                        IMCC_FREQ, 4-25

IDCOFS, 13-2                        IMCCHP, 4-25, 4-31

IDKADD, 8-122                       IMFMCTR, 24-3, 24-11, 24-55

IDKMUL, 8-122                       IMFMFLG, 23-181, 24-3, 24-11,

IDLCOT, 14-16                           24-28, 24-55 to 24-56

IDLCOT2, 14-16                      IMMEDIATE, 9-82

IDLE AIR FEEDBACK CONTROL LIMIT     IMRC, 3-4

TEST (CDAN0), 13-76             IMRC COMMAND LOGIC (CDAN0), 18-2

IDLE_CELL_SW, 8-72, 8-82            IMRC DIAGNOSTICS (CDAM0), 18-13

IDLE_TEMP_SW, 8-122                 IMRC DRIVER DIAGNOSTICS (CDAM0),

IDLFLG, 8-122, 8-246                    18-32

IDLMAF, 31-75                       IMRC INPUT LOGIC (CDAM0), 18-27

IDLRPM, 8-246                       IMRC OUTPUT CONTROL LOGIC (CDAL0),

IDLTMR, 8-24, 8-246                     18-34

IDM_BYPASS, 35-82 to 35-85          IMRC/TRANSMISSION INTERACTION

IDM_NMAX, 35-71                         CONTROL (CDAN0), 25-87

IDM_PW, 35-66, 35-77                IMRC1_FL_TMR, 18-29, 18-31

IDM_PW_STARTM, 35-65, 35-77         IMRC2_FL_TMR, 18-29 to 18-31

IDM_RLBL_FLG, 35-71                 IMRC_AS_ENA, 18-11 to 18-12

IDM_TM (w), 9-96                    IMRC_BK1_SCL, 18-21, 18-25

IDM_TM(r), 9-98                     IMRC_BK1_SOP, 18-25 to 18-26

IDM_VBATMAX, 35-71                  IMRC_BK2_SCL, 18-24

IDM_VBATMIN, 35-71                  IMRC_BK2_SOP, 18-21, 18-24, 18-26

IDMFIL, 35-85                       IMRC_CLS_TIM, 18-19, 18-22

IDMLVL, 35-85                       IMRC_CMND, 18-6 to 18-7, 18-10,

IDMUP, 35-85                            18-12, 18-19 to 18-20, 18-22

IDMUPEDGTM, 9-96                        to 18-23, 18-34

IDMUPEDGTM (r/w), 9-96              IMRC_CMND (READ), 5-133

IDRPMH, 8-246                       IMRC_CMNDOLD, 18-6, 18-12

IEGO11, 4-19, 11-165                IMRC_CNT1, 18-29 to 18-30

IEGO11_NEW, 11-29                   IMRC_CNT2, 18-29 to 18-30

IEGO12, 4-19, 11-168                IMRC_CNT3, 18-29 to 18-30

IEGO21, 4-19, 11-165                IMRC_CNT4, 18-29 to 18-30

IEGO21_NEW, 11-29                   IMRC_CRK_POS, 18-12

Index-34

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

IMRC_CYC_ECT, 18-11                 IMRCHP, 4-31, 9-73, 18-5

IMRC_CYC_VS, 18-11                  IMRCKAM_SWAP, 18-10

IMRC_EN_KOEO, 18-8 to 18-9          IMRCLAST_KAM, 18-7, 18-10

IMRC_FALT_TC, 18-19, 18-22, 18-33   IMRCLASTTRIP, 18-10 to 18-11

IMRC_FAULT, 4-31, 18-33             IMRCP1_CNTS, 4-9, 18-29

IMRC_FAULT (READ), 5-138            IMRCP1_CNTS (READ), 5-146

IMRC_FL_TM, 18-31                   IMRCP2_CNTS, 4-9, 18-30

IMRC_LOD_TMR, 18-18                 IMRCSENSHP, 18-7, 18-9

IMRC_N_HYS, 18-10                   IMRCSPK_W, 9-73, 18-5, 18-7

IMRC_N_REL, 18-10 to 18-11          IMRCTHISTRIP, 18-7, 18-11

IMRC_OFF_POS, 18-10                 IMSC_CNT_LVL, 18-24

IMRC_OFF_POSN, 18-34                IMSC_ERRLVL, 18-21, 18-24

IMRC_ON_POS, 18-10                  IMSO_CNT_LVL, 18-21, 18-24

IMRC_OPN_TIM, 18-19, 18-22          IMSO_ERRLVL, 18-21, 18-24

IMRC_OUT, 4-31, 18-34               IN-RANGE TP-LOAD TEST (CDAM0),

IMRC_POSN, 18-6 to 18-7, 18-30,         31-58

24-26, 24-62                    INDS, 4-7, 25-44 to 25-46, 25-80,

IMRC_POSN_1, 18-19 to 18-20,            31-86

18-29 to 18-30                  INDS (READ), 5-144

IMRC_POSN_2, 18-22 to 18-23,        INDS_LST, 25-46

18-30                           INERTIA_ENG, 25-75

IMRC_POSNOLD, 18-6 to 18-7          INF_MAP, 24-26

IMRC_RSP_TMR, 18-6 to 18-7, 18-18   INF_MVAC, 12-39 to 12-40, 15-90,

to 18-19, 18-22                     15-92, 23-28, 24-26

IMRC_SC1_CNT, 18-16 to 18-17,       INFAMB_KAM, 10-10 to 10-11, 10-13,

18-20 to 18-21                      11-127 to 11-130, 17-28,

IMRC_SC1_FIL, 18-16 to 18-17,           31-38

18-19 to 18-21                  INFERRED LOAD (CDAM0), 24-60

IMRC_SC2_CNT, 18-16 to 18-17,       INIT_TOT, 6-6, 25-37, 31-55

18-23 to 18-24                  INIT_TOT_CNT, 31-55

IMRC_SC2_FIL, 18-16 to 18-17,       init_trip_monitor, 23-72

18-22 to 18-24                  INITIALIZATION ROUTINE, 3-8

IMRC_SFT, 18-12                     INITL_FN1338 (READ), 8-116

IMRC_SO1_CNT, 18-16 to 18-17,       INITL_FN1338 (WRITE), 8-115

18-20 to 18-21                  INJ1_FAULT, 4-30

IMRC_SO1_FIL, 18-16 to 18-17,       INJ1_FAULT (READ), 5-136

18-19 to 18-21                  INJ2_FAULT, 4-29

IMRC_SO2_CNT, 18-16 to 18-17,       INJ2_FAULT (READ), 5-136

18-21, 18-23 to 18-24           INJ3_FAULT, 4-28

IMRC_SO2_FIL, 18-16 to 18-17,       INJ3_FAULT (READ), 5-136

18-22 to 18-24                  INJ4_FAULT, 4-28

IMRC_STAOLD1, 18-19, 18-21          INJ4_FAULT (READ), 5-136

IMRC_STAOLD2, 18-22, 18-24          INJ5_FAULT, 4-32

IMRC_STATE, 18-20                   INJ5_FAULT (READ), 5-136

IMRC_STATE2, 18-23                  INJ6_FAULT, 4-31

IMRC_STATOLD1, 18-21                INJ6_FAULT (READ), 5-136

IMRC_STATOLD2, 18-24                INJ7_FAULT, 4-30

IMRC_STATUS, 18-7, 18-17            INJ7_FAULT (READ), 5-136

IMRC_TP_ENA, 18-11 to 18-12         INJ8_FAULT, 4-30

IMRC_TP_HYS, 18-11                  INJ8_FAULT (READ), 5-136

IMRC_TP_REL, 18-10 to 18-11         INJ_0ON, 8-238

IMRC_TWOPOSN, 18-16, 18-30          INJ_1ON, 8-238

IMRC_TYPE, 18-8, 18-18              INJ_2ON, 8-238

IMRC_VAC_LOD, 18-18                 INJ_3ON, 8-238

IMRC_VAC_TIM, 18-18                 INJ_4ON, 8-238

Index-35

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

INJ_5ON, 8-238                          PROCESS, 29-17

INJ_6ON, 8-238                      int_inj_num, 8-223 to 8-224,

INJ_7ON, 8-238                          8-228

INJ_ACTUAL (WRITE), 8-213           int_seq_num, 8-223

INJ_BANK1, 8-201 to 8-202, 8-208,   INTAKE MANIFOLD RUNNER CONTROL -

8-237                               V1.3A_IMRC_ONOFF (CDAN2),

INJ_BANK1 (READ), 8-212                 18-2

INJ_BANK2, 8-202, 8-208             intcyl_bank_indicator, 8-223 to

INJ_BANK2 (READ), 8-212                 8-224

INJ_CTR, 8-236 to 8-237             INTELLIGENT MOSFET PRE-DRIVER

inj_dyn, 8-231                          (IMP), 4-37

INJ_HOLD, 8-236                     INTM, 24-7

INJ_NEXT_FLG, 8-236 to 8-237        IPDL, 25-43, 25-45 to 25-46,

INJ_OFF, 8-236                          25-48, 25-79, 25-82 to 25-85,

INJ_OFF_LST, 8-236                      26-58, 31-80

INJ_OFF_NEXT, 8-236 to 8-237        IPDL_LST, 25-43, 25-46, 25-79,

INJ_ON, 8-47, 8-236, 8-238, 8-240,      25-83

35-27                           IPSI_SW, 13-40

INJ_ON DETERMINATION LOGIC          IPSIBR, 13-3, 13-8, 13-32 to

(CDAN0), 8-239                      13-33, 13-39 to 13-41, 15-104

INJ_ON_LST, 8-236, 8-240            IPSIBR CALCULATION (CDAN0), 13-36

INJ_PIP_CNT1, 3-8, 8-186, 8-200,    IPSIDLY, 13-36, 13-39, 13-41

8-202, 8-209, 29-28             IPSIDLY (READ), 13-80

INJ_PIP_CNT2, 3-8, 8-186, 8-200,    IPSIMAX, 13-40

8-202, 8-209, 29-28             IPSPS, 4-8, 4-39

INJ_RR_TRIG, 8-237                  IS_RPM_NCLIP, 13-27, 13-48

INJ_TR, 8-240, 10-11 to 10-12,      IS_TOT_RPM, 13-28

25-17                           IS_TOT_RPM CALCULATION (CDAL0),

INJ_TR (READ), 15-22                    13-28

INJCUTOUT_SW, 8-236                 IS_TRAN_RPM, 13-10, 13-13, 13-20,

INJDLY, 8-118, 8-123 to 8-124           13-22, 13-27

INJDLY_BETA, 8-124, 8-185           IS_TRAN_RPM and MAXIMUM CLIP

INJDLY_MIN, 8-124                       CALCULATION - (CDAL0), 13-12

INJECTOR DELAY LOGIC (CDAK0),       ISADD_ACRPM, 13-10, 13-22, 13-25,

8-117                               13-27

INJECTOR OUTPUT CONTROL (CDAI0),    ISADD_HEDF, 13-10, 13-22, 13-26

8-214                               to 13-27

INJECTOR UPDATE REQUEST (CDAM0),    ISADD_POWS, 13-10, 13-22, 13-26

8-187                               to 13-27

INJECTORS, 3-4                      ISC DUTY CYCLE CALCULATION

INJOFF, 8-236 to 8-237                  (ISCDTY_CALC) (CDAF0), 13-81

INJOFF (READ), 8-213                ISC-BPA, 3-4

INJOFF_FMEM, 8-236 to 8-237         ISC_A_DER, 13-3, 13-36, 13-41

INJREF, 8-118, 8-185                ISC_A_PROP, 13-3, 13-36, 13-41

INLRPH, 8-244                       ISC_BIAS_MAX, 8-281

INLRPM, 8-244                       ISC_BIAS_MIN, 8-281

INPUT CONVERSIONS AND FILTERS       ISC_CURRENT, 4-36, 4-44, 13-85

DYNAMIC TV DUE TO COLD            ISC_DESMAF_PID -

TRANSMISSION, 29-20               V1.2_ISC_DESMAF_PID (CDAN0),

DEFINITIONS, 29-20                  13-28

OVERVIEW, 29-20                 ISC_DSDRPM - V1.A1_ISC_DSDRPM

PROCESS, 29-20                      (CDAN0), 13-10

POWER MODE, 29-16                 ISC_ER_ENA, 23-181, 23-185,

DEFINITIONS, 29-16                  23-189 to 23-190

OVERVIEW, 29-16                 ISC_HI_CHECK, 23-186

Index-36

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

ISC_HI_DONE, 23-185 to 23-186       J1979 PIDS (CDAM0), 23-134

ISC_HI_WAIT, 23-186                 j1979_01_00, 5-123, 5-141

ISC_LO_CHECK, 23-189                j1979_01_011, 5-123, 5-141

ISC_LO_CHECK (READ), 13-79          j1979_01_012, 5-123, 5-141

ISC_LO_DONE, 23-185, 23-189         j1979_01_013, 5-123, 5-141

ISC_LO_WAIT, 23-188 to 23-189       j1979_01_014, 5-124, 5-141

ISC_PTP, 8-284                      j1979_01_031, 5-124, 5-128, 5-141

ISC_PTP_SW, 8-281, 8-284            j1979_01_04, 5-124, 5-128, 5-141,

ISCDTY, 4-44, 13-2, 13-8, 13-85         24-66

to 13-86, 15-18 to 15-19,       j1979_01_05, 5-124, 5-141, 31-30

15-101, 24-63                   j1979_01_06, 5-125, 5-129, 5-141

ISCDTY (READ), 5-144                j1979_01_07, 5-125, 5-129, 5-141

ISCFLG, 8-44, 8-46, 8-106, 8-281,   j1979_01_08, 5-125, 5-129, 5-141

8-284, 9-14, 9-18, 9-20, 9-22,  j1979_01_09, 5-125, 5-129, 5-141

9-46, 11-110, 13-3, 13-8,       j1979_01_0C, 5-125, 5-129, 5-141,

13-31 to 13-32, 13-36, 13-39        29-24

to 13-41, 13-46, 13-73 to       j1979_01_0D, 5-125, 5-129, 5-141

13-74, 13-90, 15-17, 15-101     j1979_01_0d, 31-82

to 15-102, 17-11, 17-19,        j1979_01_0E, 5-125, 5-141

24-30, 35-29                    j1979_01_0F, 5-126, 5-141

READ, 8-72, 8-82                  j1979_01_0f, 31-11

ISCFLG (READ), 8-42, 8-55, 13-80    j1979_01_10, 5-126, 5-141, 24-64

ISCFLG_LST, 8-44, 8-46, 8-55        j1979_01_10 (READ), 5-146

ISCFLG_LST (READ), 8-55             j1979_01_10, MASS AIR FLOW,

ISCFLG_LST (WRITE), 8-42                CALCULATION (CDAM0), 24-64

ISCKAM, 13-3, 13-33, 32-4           j1979_01_11, 5-126, 5-141, 31-66

ISCKAM0, 32-5                       j1979_01_12, 5-126, 5-141, 17-13

ISCKAM1, 32-5                       j1979_01_13, 5-126, 5-141

ISCKAM2, 32-5                       j1979_01_14, 5-127, 5-141

ISCKAM3, 32-5                       j1979_01_151, 5-127, 5-141

ISCKAM_HLT, 13-32, 13-34            j1979_01_152, 5-127, 5-141

ISCKAM_HLT FLAG CONTROL (CDAN0),    j1979_01_18, 5-127, 5-141

13-34                           j1979_01_191, 5-127, 5-141

ISCKAMn, 13-3, 13-32, 13-41         j1979_01_192, 5-127, 5-141

ISCLPD, 13-14                       J1979_01_1C, 5-127, 23-136

ISCLPD_ALT, 13-14                   j1979_01_1C, 5-127, 23-136

ISCMOD, 3-8                         j1979_02_00, 5-128, 5-143

ISCPSI, 13-3, 13-39 to 13-41        j1979_02_02, 5-128, 5-143

ISCTM, 13-74, 13-90                 j1979_02_031, 5-128, 5-143

ISCTMR, 13-31 to 13-32, 13-73 to    j1979_02_04, 5-128, 5-143

13-74, 13-90                    j1979_02_05, 5-128, 5-143

ISFLAG, 13-32, 13-40 to 13-43,      j1979_02_06, 5-129, 5-143

15-101                          j1979_02_07, 5-129, 5-143

READ, 8-72, 8-82                  j1979_02_08, 5-129, 5-143

ISFLAG (READ), 8-56, 13-79          j1979_02_09, 5-129, 5-143

ISKSUM, 13-33, 32-4 to 32-5         j1979_02_0C, 5-129, 5-143

ISLAST, 13-32, 13-40, 13-42 to      j1979_02_0D, 5-129, 5-143

13-43, 15-101

ISLAST (READ), 8-56

ISLAST and ISFLAG LOGIC (CDAL0),    K_CLUTCH, 28-84

13-42                           K_CONV, 28-81, 28-87

ITCS, 4-12, 25-13 to 25-14, 34-2    K_CONV1, 25-63

ITCS (READ), 5-133                  K_VALVE, 28-84

ITP, 31-80                          KACRAT, 9-60

IXFRPR, 12-20                       KAM, 32-4

Index-37

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

KAM UPDATE (ISCKAM_UPDATE)            WRITE, 8-73

(CDAN0), 13-28                  KEEP ALIVE MEMORY (KAM)

KAM VERIFICATION (CDAM0), 23-113        QUALIFICATION TEST (CDAM0),

KAM_BAR1                                32-2

READ, 8-97                        KI, 9-64

WRITE, 8-94, 8-96                 KIHP, 4-10, 4-14, 9-61, 9-63

KAM_BAR1 (READ), 15-8               KNK_HIGH, 4-10, 9-63

KAM_BAR2                            KNK_INT, 9-63

READ, 8-98                        KNOCK, 9-64

WRITE, 8-94, 8-96                 KNOCK CONTROL STRATEGY (CDAN0),

KAM_BAR2 (READ), 15-8                   9-53

KAM_BAR_FK, 8-96                    KNOCK RETARD LOGIC, 9-67

KAM_ERROR, 5-8, 5-22, 5-107,        KNOCK STRATEGY ENABLE LOGIC, 9-61

8-271, 9-97, 10-9 to 10-10,     KNOCK_DETECTED, 9-63

10-23, 11-13, 12-16 to 12-17,   KNOCK_ENABLED, 9-59, 9-61, 9-67

13-32, 14-9, 15-100, 18-16,     KNOCK_OCCURRED, 9-63, 9-67

18-29, 18-33, 23-15, 23-35,     KOEO_TV_CNTS, 27-47

29-12, 29-14, 31-66, 32-4,      KOER_MINMPH, 23-181

35-99                           KOER_TST_TM, 23-182

READ, 8-70, 8-94                  KOER_TST_TMR, 23-182, 23-185

KAM_ERROR (READ), 8-115, 8-314      KSF, 31-88

KAM_MON_TMR                         KTS, 9-64 to 9-65

READ, 8-97 to 8-98                KWCTR, 9-63

WRITE, 8-73, 8-95                 KWUCNT, 8-74

KAM_NO_RESET, 8-78                  KWUCTR, 10-38, 10-40, 15-61,

KAMCOL                                  15-104

READ, 8-73, 8-79, 8-81 to 8-82      READ, 8-74

WRITE, 8-72                         WRITE, 8-71, 8-74, 8-78

KAMCOL_CUR

READ, 8-73

WRITE, 8-73

KAMCOL_OLD                          LAM_BAR1

READ, 8-73                          READ, 8-97

WRITE, 8-73                         WRITE, 8-94, 8-96

KAMQA, 5-57, 32-4                   LAM_BAR1 (READ), 15-8

KAMQB, 32-4                         LAM_BAR2

KAMQC, 32-4                           READ, 8-98

KAMRF1, 5-125, 11-118, 15-73,         WRITE, 8-94, 8-96

23-126                          LAM_BAR2 (READ), 15-8

READ, 8-83, 8-96                  LAM_BAR_FK, 8-96

WRITE, 8-82 to 8-83               LAM_COLD_ENG, 8-51

KAMRF1 (READ), 5-144, 8-137         LAM_CYC_MAX, 11-15

KAMRF2, 5-125, 11-122, 15-73,       LAM_CYCLES, 8-257 to 8-258, 11-15

23-126                          LAM_DIFF_FAO (DEFINED), 8-305

READ, 8-83, 8-96                  LAM_DIFF_FAO (READ), 8-307

WRITE, 8-82 to 8-83               LAM_EXT_MAX, 8-45

KAMRF2 (READ), 5-144, 8-137         LAM_EXT_MIN, 8-45

KAMROW                              LAM_EXT_STEP, 8-45

READ, 8-73, 8-79, 8-81 to 8-82    LAM_FINAL1, 8-137

WRITE, 8-72                       LAM_FINAL1 (READ), 8-137

KAMROW_CUR                          LAM_FINAL2, 8-137

READ, 8-73                        LAM_FINAL2 (READ), 8-137

WRITE, 8-73                       LAM_ISC_SW, 8-55

KAMROW_OLD                          LAM_LOAD_SW, 8-56

READ, 8-73                        LAM_MOD_BIAS, 8-257

Index-38

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

LAM_MOD_FLG, 8-257 to 8-258,        LAMMUL RESET LOGIC, 8-18

8-281, 8-287, 10-25, 11-14,     LAMMUL RESET LOGIC (CDAC0), 8-17

11-17, 15-72                    LAMWIN1

READ, 8-88                          READ, 8-75

LAM_MOD_FLG (READ), 8-307, 15-23      WRITE, 8-74

LAM_MOD_MAX, 8-257                  LAMWIN2

LAM_MOD_MIN, 8-257                    READ, 8-75

LAM_OFFSET, 8-257                     WRITE, 8-74

LAM_OLD1, 3-8, 8-30                 LAST_HI_PIP, 8-219, 8-228, 9-82,

LAM_OLD2, 3-8, 8-30                     9-96, 29-30 to 29-31

LAMAVE (FCALL), 8-307                 WRITE, 8-171

LAMAVE1, 3-8, 8-30, 11-33, 17-19    LAST_HI_PIP (r), 9-96

LAMAVE2, 3-8, 8-30, 11-33, 17-19    LAST_LD_MBT, 9-27, 9-42

LAMAVE_MAX, 11-33                   LAST_ON_INJ, 8-219

LAMAVE_MIN, 11-33                   LBMF_INJ, 8-194

LAMBSE, 8-273                       LBMF_INJ1, 15-16

LAMBSE (FCALL), 8-307               LBMF_INJ1 (WRITE), 8-136, 8-180

LAMBSE CLIP LOGIC (CDAH0), 8-59         to 8-181

LAMBSE MODULATION (CDAM0), 8-255    LBMF_INJ2, 15-16

LAMBSE RESET LOGIC (CDAN2), 8-52    LBMF_INJ2 (WRITE), 8-136, 8-180

LAMBSE1, 3-8, 5-125, 8-30, 8-46,        to 8-181

8-51, 8-60, 8-137, 8-271,       LBMF_TAB, 8-194

8-273, 8-276 to 8-278, 11-125,  LC_RATIO, 8-284

11-139, 11-166, 11-179, 15-16,  lcl_ibeta, 8-208 to 8-209

15-72 to 15-73, 17-21, 23-26    lcl_inj_bank, 8-201

to 23-27, 23-126                LD_EGO_DLTA, 11-35

READ, 8-75, 8-96                  LDBPLO, 29-11

LAMBSE1 (READ), 5-144, 8-57,        LDEGO_DLTMX, 11-35

8-137                           LDEH, 8-15 to 8-16

LAMBSE1 (WRITE), 8-55 to 8-56,      LDEL, 8-15 to 8-16

8-58                            LDFLG, 8-15 to 8-16, 8-24, 8-49

LAMBSE2, 3-8, 5-125, 8-30, 8-46,    LDLTM, 8-15 to 8-16

8-51, 8-57, 8-60, 8-137,        LDMH, 8-15 to 8-16

8-271, 8-273, 8-276 to 8-278,   LDMHH, 8-15 to 8-16

11-125, 11-166, 11-179, 15-16,  LEAN_AMP_M, 5-121, 11-37

15-72 to 15-73, 17-21, 23-26    LEAN_THRESH, 11-49, 11-56

to 23-27, 23-126                learn_timer (FCALL), 8-293

READ, 8-75, 8-96                  learn_timer (FUNCTION), 8-304

LAMBSE2 (READ), 5-144, 8-137        learn_timer_running (FCALL),

LAMBSE2 (WRITE), 8-55 to 8-56,          8-307, 8-314

8-58                            LEGAL DISCLAIMER, 25-53

LAMBSE_DRV, 8-44, 8-47 to 8-48      LEGOFG11, 8-25, 8-43, 8-272,

LAMBSE_EXT, 8-45, 8-48                  11-113 to 11-114, 11-117,

LAMBSE_MOD1, 8-137, 8-257               11-164, 11-167, 15-73, 32-4

LAMBSE_MOD2, 8-137, 8-257             READ, 8-82 to 8-83

LAMBSEx, 8-275                      LEGOFG11 (READ), 8-42

LAMMAX, 8-60, 8-273, 11-182         LEGOFG21, 8-25, 8-43, 8-271,

LAMMAX1, 11-125, 11-166, 11-182         11-113 to 11-114, 11-121,

LAMMAX2, 11-125, 11-166, 11-182         11-164, 11-167, 15-73, 32-4

LAMMIN, 8-55, 8-60, 8-273, 11-182     READ, 8-82 to 8-83

LAMMIN1, 11-125, 11-166, 11-179,    LEGOFG21 (READ), 8-42

11-182                          LEGONOTPURG, 11-114, 11-126

LAMMIN2, 11-125, 11-166, 11-179,    LEQV, 11-143, 11-145, 11-149,

11-182                              17-32

LAMMUL, 3-8, 8-18 to 8-19, 8-44     LES_MIN_AMB, 11-127 to 11-130

Index-39

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

LESFLG1, 11-164                       WRITE, 8-73, 8-75 to 8-76

LESTMR11, 11-164, 11-167, 11-179,   LPCT2L

15-18                             READ, 8-75, 8-77

LESTMR11 (WRITE), 8-55 to 8-58        WRITE, 8-73, 8-75, 8-77

LESTMR21, 11-164, 11-167, 11-179    LPCT2R

LESTMR21 (WRITE), 8-55 to 8-58        READ, 8-75, 8-77

LIMIT_PURGE, 15-72, 15-74             WRITE, 8-73, 8-75, 8-77

READ, 8-75                        LSF_FLG, 16-12 to 16-13, 16-35

LLDTM, 8-24                         LSF_FLG (READ), 5-133

LMBJMP (WRITE), 8-55                LSF_INVRT_HP, 16-35

LN_CTL_FLG, 17-11, 17-27            LSF_RLY_PRCT, 16-12

LOAD, 8-8, 8-26, 8-47, 8-49,        LSF_RQST, 13-56, 16-4, 16-12 to

8-101, 8-105 to 8-107, 8-122,       16-13

8-130, 8-247, 8-281, 8-284,     LSFECT_CL, 16-4

9-14, 9-18, 9-20, 9-22, 9-28,   LSFECT_FLG, 16-4

9-31, 9-48, 9-58, 9-60 to       LSFECT_SH, 16-4

9-62, 9-73, 9-85, 10-11,        LSFVS_CH, 16-4

10-24, 11-11, 11-110, 11-187,   LSFVS_SL, 16-4

12-11, 13-74 to 13-75, 15-48,   LST_PW, 8-193 to 8-194, 8-224

15-65, 15-90, 15-92, 15-102,    LST_PW (WRITE), 8-213

16-8, 23-26 to 23-27, 24-59,    LST_SFT_CMP, 26-27, 26-58

25-17, 29-7, 29-10 to 29-11,    LSTCOL

31-61, 31-71, 31-80, 35-33,       WRITE, 8-73

35-101                          LSTROW

READ, 8-72, 8-88, 8-95              WRITE, 8-73

LOAD (READ), 5-144, 8-304, 8-307,   LTMTB1

8-316                             READ, 8-71, 8-76, 8-82

LOAD CALCULATION (CDAM0), 24-57       WRITE, 8-71, 8-76, 8-78

LOAD SHIFT IN PROGRESS TIMER        LTMTB2

(CDAA0), 26-41                    READ, 8-71, 8-77, 8-82

LOAD(read), 35-33                     WRITE, 8-71, 8-77 to 8-78

LOAD_EGO, 11-11, 11-34 to 11-35     LUDLY, 28-19

LOAD_EGO_MAX, 11-34                 LUGGING MODE DETERMINATION

LOAD_EGO_MIN, 11-34                     (CDAA0), 8-15

LOAD_SAE, 23-126, 24-66             LUGSW, 8-247

LODNOK, 9-61                        LUGTIM, 8-247

LOLODH, 8-26                        LUGTMR, 8-16, 8-247, 9-18

LOM LOAD TRANSITION TIMER (CDAA0),  LUHECT_CL, 28-3

8-247                           LUHECT_SH, 28-3

LONG_PULSE, 8-193                   LUHTOT_CL, 28-3

WRITE, 8-189                      LUHTOT_SH, 28-3

LOW SPEED DIGITAL OUTPUTS (LSO),    LUS_PRD_DT, 4-17, 28-50

4-15                            LUS_PW_DTKS, 28-50

LOW SPEED FAN STRATEGY (CDAH0),     LUS_PW_DUCE, 28-50

16-2

LOWLOD, 13-74                       MAF, 4-19, 23-26 to 23-28, 23-41

LOWVOL_CH, 13-21                        to 23-44, 24-7, 24-9, 24-64

LOWVOL_FLG, 13-21                   MAF SENSOR DIAGNOSTIC TEST

LOWVOL_SL, 13-21                        SELECTION (CDAM0), 24-39

LPCOMP_ENA, 13-40 to 13-41          MAF SENSOR DIAGNOSTIC TESTS -

LPCT1L                                  BACKGROUND (CDAM0), 24-42

READ, 8-75 to 8-76                MAF SENSOR FMEM - FOREGROUND

WRITE, 8-73, 8-75 to 8-76             (CDAM0), 24-10

LPCT1R                              MAF_FIL, 23-40 to 23-43

READ, 8-75 to 8-76                MAF_FIL_MAX, 23-35, 23-43

Index-40

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

MAF_FIL_MAX (READ), 5-145           MFMFLG, 10-9, 11-109, 12-7, 13-7

MAF_HI_ER, 24-11, 24-50                 to 13-8, 24-3, 24-11, 24-55,

MAF_HI_KOEO, 24-53                      27-14, 29-11 to 29-12, 31-60,

MAF_HI_KOER, 24-52                      31-80

MAF_INT, 23-40 to 23-41, 23-43        READ, 8-88

MAF_INTP_FLG, 24-28 to 24-29,       MFMFLG (READ), 5-132, 8-307

29-11                           MFMHYS, 24-11

READ, 8-88                        MIDTV, 8-122

MAF_LO_ER, 24-11, 24-51             MIL, 4-29

MAF_LO_KOER, 24-52                  MIL CONTROLLER (CDAM0), 23-45

MAF_LVL, 23-42                      MIL OUTPUT CONTROL (CDAM0),

MAF_RPM_MAX, 24-11, 24-45               23-105

MAF_TIME, 24-7                      MIL_1000_1, 35-48, 35-56

MAFRTN, 4-19                        MIL_1000_2, 35-48, 35-56

mainpw_bank_indicator, 8-202 to     MIL_1000_3, 35-48, 35-56

8-203                           MIL_1000_4, 35-48, 35-56

mainpw_bank_num, 8-193 to 8-194     MIL_1000_5, 35-48, 35-56

mainpw_inj_num, 8-193 to 8-194,     MIL_1000_6, 35-48, 35-56

8-202, 8-219                    MIL_1000_7, 35-48, 35-56

mainpw_inj_num (READ), 8-213        MIL_1000_8, 35-48, 35-56

mainpw_inj_num (WRITE), 8-212       MIL_200_1, 35-49, 35-56

mainpw_seq_num (READ), 8-213        MIL_200_2, 35-49, 35-56

mainpw_seq_num (WRITE), 8-212       MIL_200_3, 35-49, 35-56

MAKUPSPK_FLG, 9-78, 9-80 to 9-82    MIL_200_4, 35-49, 35-56

malfunction                         MIL_200_5, 35-50, 35-56

CALL, 8-97 to 8-98                MIL_200_6, 35-50, 35-56

malfunction(purg,P0442), 15-61      MIL_200_7, 35-50, 35-56

malfunction(purg,P0443), 15-36,     MIL_200_8, 35-50, 35-56

15-83                           MIL_FAULT, 4-29

malfunction(purg,P1443), 15-105     MIL_FAULT (READ), 5-137

MAN1HI, 25-45                       MIL_ON, 23-52, 23-109 to 23-112

MAN1LO, 25-45                       MIL_ON (READ), 5-133

MANIFOLD FILLING MODEL (CDAM0),     MIL_SW, 23-109, 23-112

24-12                           MILLIM, 23-112

MAP_AIR_0, 24-3, 24-14, 24-26       min (FCALL), 8-314, 8-319

MAPPEREGR, 24-26                    MIN_DELPR, 12-40 to 12-42

max (FCALL), 8-314, 8-319           MIN_DELPR_FL, 5-121, 12-41 to

MAX_EGO_VOLT, 11-27, 11-47              12-42

MAX_ERROR, 35-67                    MIN_FLG_BK1, 10-34, 10-38

MAX_MVAC, 12-39                     MIN_FLG_BK2, 10-36, 10-40

MAX_NODELT, 25-93                   MIN_IMRC_RPM, 23-157

MAX_PCT_MIS, 35-42                  MIN_MVAC, 12-39

MAX_RM_SIMUL, 8-199                 MIN_PIP_DLY, 29-28, 29-30 to

MAX_SLIP_DIF, 28-54                     29-31

MAX_TIME, 35-67                     MINACCL, 8-50

MAX_VSDELT, 25-98                   MINADP, 8-76 to 8-77, 15-73

MAXADP, 8-76 to 8-77, 11-118,       MINDES_CL, 12-18

11-122                          MINDES_SH, 12-18

MAXIMUM BRAKE TORQUE SPARK          MINMPH, 9-46, 13-73

(CDAL0), 9-29                   MINPW, 8-136

MAXNEPU, 25-24                      MINPW_LBMF, 8-136

MAXSLIP1, 28-7 to 28-9, 28-33       MINPW_LBMF (READ), 8-180

MCC_H_FAULT (READ), 5-141           MINPWT, 8-136, 8-224, 8-228,

MEDIAN_WIDTH, 35-15, 35-19 to           8-277

35-20, 35-23, 35-32             MINPWT (READ), 8-180, 15-17

Index-41

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

MIS200FLG_1, 31-23, 31-26, 35-49    MIS_TST_RDY, 35-29 to 35-31,

MIS200FLG_2, 31-24 to 31-26,            35-101

35-49                           MIS_TST_SW, 15-50, 23-17, 23-36,

MIS200FLG_3, 31-24, 31-26, 35-49        23-51, 23-68, 23-135

MIS200FLG_4, 31-23 to 31-26,        MIS_USE_CF, 35-18

35-49                           MIS_USE_HALF, 35-102, 35-110

MIS200FLG_5, 31-23 to 31-24,        MISACTDLY, 35-28

31-26, 35-50                    MISBG_BYPS, 35-14, 35-31 to 35-32

MIS200FLG_6, 31-23, 31-25 to        MISBYPSCTR, 35-14, 35-54

31-26, 35-50                    MISDLYFLG, 35-28

MIS200FLG_7, 31-25 to 31-26,        MISECTDLY, 35-28

35-50                           MISFDELTAERR, 35-101

MIS200FLG_8, 31-26, 35-50           MISFIRE DETECTION TEST (CDAN2),

MIS_AC_SW, 35-27                        35-2

MIS_CAT_CONF, 35-31, 35-49 to       MISFIRING, 23-112, 35-39 to 35-41,

35-50                               35-57

MIS_CF, 35-18                       MISFMEM_DSRD, 31-23 to 31-26

MIS_ECTMAX, 35-29                   MISLEVEL1000, 5-121, 35-39 to

MIS_ECTMIN, 35-29                       35-40, 35-48

MIS_ENA_FRST, 35-26, 35-54          MISLEVELSMLR, 35-41

MIS_ENA_OLD, 35-28                  MISLVLEXP, 35-35

MIS_ER_ENA, 23-181, 23-185,         MISNOCALLFIL, 35-54

23-187 to 23-188, 23-190,       MISPOTCALL, 35-21

35-30 to 35-32                  MISTHRESPCT, 35-14

MIS_EVENTS, 35-14, 35-54            MISTOTDLY, 35-28 to 35-29

MIS_FCO_FLG, 23-112, 35-51          MLPS, 4-7

MIS_FF_CODE, 23-49, 23-97 to        MLPS CONVERSION (CDAN0), 25-38

23-98                           MLPS_2, 25-45 to 25-47, 28-16

MIS_FIL_SEL, 35-39 to 35-40         MLPS_2HI, 25-45

MIS_FMEM, 23-17, 23-35, 23-39       MLPS_2LO, 25-45

mis_init_mil(MIS_MIL_ON), 23-51     MLPS_KOEO_EN, 25-44, 25-85

MIS_ISCF_SW, 35-29                  MLUS CONTROL AND SELF TEST

mis_malfunction(MIS_MIL_ON, Pxxx),      (CDAN0), 28-41

23-50                           MLUS_DLY_TMR, 28-54, 28-56

MIS_MIL_ON, 23-50 to 23-51, 23-97,  MLUS_ERR_CNT, 28-56 to 28-57

23-110                          MLUS_ERR_LVL, 28-57

MIS_MIL_SW, 23-110, 23-112          MLUS_FM_FLG, 28-17, 28-53, 28-56

MIS_MIN_TQ, 35-29                   MLUS_HF_FLG, 28-56

MIS_MON, 15-50, 23-44, 23-51,       MLUS_HF_FLG (READ), 8-164

23-68, 23-71 to 23-72, 35-41    MLUS_MX_TOT, 23-156, 28-53, 28-56

MIS_MON_KAM, 35-26, 35-41           MLUS_NT_DLY, 28-56

MIS_MON_KAM (READ), 5-136           MLUS_NT_TMR, 28-53, 28-56

MIS_MON_LST, 23-44, 23-51           MLUS_OT_CNT, 28-53, 28-56

MIS_MON_LST := 0, 23-71             MLUS_OT_DLY, 28-56

MIS_NOCALL, 35-21 to 35-22, 35-26,  MLUS_OT_TMR, 28-53, 28-56

35-54                           MLUS_OT_VAL, 28-53, 28-56

MIS_NOCALL (READ), 5-145            MLUS_TDLY_TM, 28-56

MIS_PROFILE, 35-17 to 35-18         MLUS_TST_ENA, 28-54

mis_reset, 23-52                    MLUS_UPCNT, 28-56

MIS_STRT_DLY, 35-28                 MNOSSPER, 25-92

MIS_SYNC_CTR, 35-16 to 35-17,       MNPIP4, 29-28

35-23                           MNPIP6, 29-28

MIS_TST_ENA, 23-16 to 23-17,        MNPIP8, 29-28

35-26, 35-28, 35-31 to 35-32    MNPIPn, 8-164, 29-28

MODACT_CL, 28-6

Index-42

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

MODACT_SH, 28-6                     NAR, 5-26

MODE SELECT (MODE_SELECT) (CDAN0),  NBAR, 23-186, 29-23

13-68                           NBPHI, 29-11

MODECT_CL, 28-6                     NBPLO, 29-11

MODECT_SH, 28-6                     NCNT, 6-6

MODTOT_CL, 28-6                     NDBAR, 29-23

MONxxxxUP, 23-141                   NDDELT, 29-5

MTEFTC, 8-101, 8-107                NDDTIM, 8-131, 8-252, 29-5

MTEISF, 8-101, 8-106 to 8-107       NDDTIM (READ), 29-4

MULTMR, 8-19, 8-248                 NDDTIM (WRITE), 29-4

MX_HISLP_TM, 28-56                  NDFFLG, 29-5 to 29-6

MX_OTSLP_TM, 28-56                  NDFSO, 8-131

MX_SLIP_ERR, 28-56                  NDIF, 13-74, 13-90

MX_TMSLBRK, 28-62                   NDS_SWPT, 31-86

MX_TSLOS_SPK, 25-93                 NDS_SWPT (READ), 29-4

MX_TSLVS_SPK, 25-99                 NDSFLG, 8-8, 8-131, 8-244, 13-9,

MXRMPOPENTM, 28-80                      23-159, 23-181, 23-183 to

23-184, 25-44, 29-5 to 29-6,

N, 6-6 to 6-7, 8-8, 8-26, 8-44,         31-80

8-47, 8-49 to 8-50, 8-101,      NDSFLG (READ), 5-133, 29-4

8-106 to 8-107, 8-122, 8-130,   NDSFLG (WRITE), 29-4

8-146, 8-244, 8-246, 8-281,     NE12COMP, 26-11

8-284, 8-287, 8-325, 9-14,      NE12S, 26-11

9-18, 9-20, 9-22, 9-31, 9-43,   NE12S4L, 26-11

9-48, 9-58, 9-60 to 9-63,       NE23COMP, 26-11

9-67, 9-73, 9-79, 9-83, 9-85,   NE23S, 26-11

9-104, 10-11, 11-46, 11-187,    NE23S4L, 26-11

12-6, 12-11, 12-16, 12-39,      NE34COMP, 26-11

13-9, 13-46, 13-54, 13-65,      NE34S, 26-11

13-73 to 13-75, 13-90, 14-8,    NE34S4L, 26-11

14-10, 18-10, 23-26 to 23-28,   NE_DROP_EX, 26-58 to 26-59

23-126, 23-130 to 23-131,       NE_DROP_LCK, 26-58

23-157, 24-11, 24-26 to 24-27,  NE_DROP_OPEN, 26-58

24-29, 24-33, 24-45, 24-48,     NE_DROP_TM, 26-59

24-59, 24-62 to 24-63, 24-66,   NE_ENG_STRT, 27-8

25-17, 25-33, 27-7, 29-11,      NE_SFT_END, 26-67, 26-71

29-23 to 29-24, 31-61, 31-80,   NE_SFT_STRT, 26-66 to 26-67,

31-86 to 31-87, 35-29, 35-33,       26-71

35-71, 35-80                    NE_STRT_SFT, 27-27, 27-33

READ, 8-72, 8-95                  NE_TRIG_STRT, 26-68, 26-71

N (READ), 5-144, 8-136, 8-304,      NE_VMOV_MAX, 31-79

8-307, 8-316, 15-17             NEBART, 25-24, 25-33 to 25-34,

N(read), 35-33                          25-51, 26-14, 26-30, 26-58 to

N_BYTE, 16-8, 29-23                     26-59, 27-8, 27-21, 27-27,

N_EGO, 11-11, 11-34 to 11-35            28-17, 28-34, 28-65, 28-70,

N_EGO_DLTA, 11-35                       31-80

N_EGO_MAX, 11-34                    NEBART_PU, 25-24, 26-11

N_EGO_MIN, 11-34                    NEGO_DLTMX, 11-35

N_IDLE_CH, 11-46                    NEHSM1, 26-30

N_IDLE_SL, 11-46                    NELK_FM, 28-65

N_PREV, 29-23                       NELUMN, 28-17

N_RATCH, 13-8 to 13-9, 13-67        NEMAX, 26-67

N_RUN, 6-6, 27-7                    NEPU_SW, 25-24

NACTMR, 13-21, 15-70 to 15-71,      NEPUMIN, 25-24

30-4                            NETVMN, 27-21

Index-43

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

NETVMX, 27-21                       NOISE, 9-64

NEU_RES_TMR, 27-7 to 27-8           NOISE_COUNT, 35-21 to 35-22,

NEUFLG, 8-18                            35-32, 35-34, 35-101

NEUHI, 25-45                        NOISE_IGNR, 35-101

NEUHI_EO, 25-44                     NOMAX, 8-7

NEULO, 25-45                        NORMAL TORQUE CAPACITY

NEULO_EO, 25-44                         CALCULATION (CDAM0), 28-73

NEUTIM, 27-7                        NORMAL TV CALCULATION (CDAN0),

NEUTRAL/DRIVE TRANSITION LOGIC          27-23

(CDAN2), 29-2                   NORMAL_MODE, 11-84 to 11-85,

NEV_PEAK_SFT, 26-58 to 26-59            11-90 to 11-91

new_average (FCALL), 8-309          NORMALIZE_CF, 35-109, 35-113,

NEW_CF_DATA, 35-96 to 35-97,            35-115

35-100                          NOV_ACT, 25-29

NEW_PIP, 9-58 to 9-59, 29-28,       NOV_ACT_LST, 25-29

29-30, 30-5                     NOVCNT, 25-29

NEW_RPM, 29-23, 29-30               NOVCTR, 25-29, 32-4

NEW_TIME, 9-82                      NOVDIF, 25-29

NEXT_SPOUT_BETA, 9-80 to 9-82       NOVS, 8-130, 9-27, 9-48, 31-87

NFIL, 29-23                         NOVSUM, 25-29

NIHYS, 8-244                        NSTALL, 6-6, 24-66

NIOLD, 8-24, 8-244                  NTBART, 25-34, 25-63, 26-67,

NITMR1, 8-121                           28-34, 28-54, 28-81, 28-87

NITMR2, 8-121                       NTBART (READ), 5-145

NITMR3, 8-121                       NTIP, 9-60

NITMR4, 8-121                       NUBASE, 9-46, 13-13

NITMR5, 8-121                       NUBASE_ALT, 13-13

NLAST, 13-73 to 13-74, 13-90        NUM_CODES, 5-67, 23-59 to 23-60,

NLMT_FLG2, 8-7                          23-62 to 23-63, 23-120

NLMT_FLG2 (READ), 8-164             NUMCYL, 4-14, 4-30, 8-47, 8-136,

NLMTHI_0, 8-9                           8-168, 8-174, 8-185 to 8-186,

NLMTHI_1, 8-9                           8-189, 8-200 to 8-202, 8-208

NLMTHI_2, 8-9                           to 8-209, 8-215, 8-236, 8-238,

NLMTLO_0, 8-9                           8-240, 35-23, 35-27, 35-36,

NLMTLO_1, 8-9                           35-46, 35-100, 35-103

NLMTLO_2, 8-9                       NUMCYL (READ), 8-181

NLMTNEUHI_0, 8-9                    NUMEGO, 8-25, 8-43, 8-74 to 8-75,

NLMTNEULO_0, 8-9                        8-77 to 8-78, 8-83, 8-96,

NO, 25-92 to 25-93, 31-16               8-98, 8-179, 8-203, 8-215,

NO (READ), 5-145                        8-271, 8-274, 10-31, 10-35,

NO_FUEL, 8-24, 8-240                    10-39, 10-41, 11-12 to 11-13,

NO_FUEL (WRITE), 8-164                  11-27, 11-29, 11-33, 11-36,

NO_FUEL_SCP (READ), 8-213               11-111, 11-114, 11-125,

NO_FUEL_SCP (WRITE), 5-11               11-142 to 11-143, 11-146,

NO_LIM_STG1, 8-7                        11-155, 11-162, 11-164 to

NO_LIM_STG2, 8-7                        11-165, 11-168, 11-178, 17-10,

255_NO_OF_STARTS, 22-3                  17-19, 17-27, 17-30, 17-32 to

NO_OF_STARTS, 22-3, 22-5                17-33, 23-153, 31-14

NO_PREV, 25-92 to 25-93             NUMEGO (DEFINED), 8-311

NO_START_CHK, 22-3, 22-5            NUMEGO (READ), 8-311

NOBAR, 8-7, 8-49, 31-16             NUMMIS, 35-23, 35-26

NOBART, 25-29, 25-34, 25-63,        NUMMIS (READ), 5-145

25-92, 25-94, 26-67, 28-34,     NUMMIS1, 35-26, 35-34

28-70, 31-16, 31-80, 31-83      NUMMIS1 (READ), 5-145

NOINAL_AFR, 8-137                   NUMMIS2, 35-26, 35-34

Index-44

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

NUMMIS2 (READ), 5-145               ODU_KEY, 5-57, 5-61, 5-72 to 5-74,

NUMMIS3, 35-26, 35-34                   23-151

NUMMIS3 (READ), 5-145               ODU_TIMEOUT, 5-10, 5-57, 5-60,

NUMMIS4, 35-26, 35-34                   23-152, 23-154

NUMMIS4 (READ), 5-145               ODU_UP_STATE, 5-10 to 5-11, 5-74

NUMMIS5, 35-26, 35-34               ODU_UP_TMR, 5-8, 5-10, 5-57, 5-60,

NUMMIS5 (READ), 5-145                   5-73 to 5-74, 23-152, 23-154

NUMMIS6, 35-26, 35-34               off_edge_time, 8-193, 8-219,

NUMMIS6 (READ), 5-145                   8-225

NUMMIS7, 35-26, 35-34               OFF_FAULT11, 11-96 to 11-98,

NUMMIS7 (READ), 5-145                   11-101

NUMMIS8, 35-26, 35-34               OFF_FAULT12, 11-96 to 11-97,

NUMMIS8 (READ), 5-145                   11-102

NUMPR, 8-49                         OFF_FAULT21, 11-96 to 11-98,

NVBASE, 31-16, 31-83                    11-101

NVBASE_X_NVC, 25-29                 OFF_FAULT22, 11-96 to 11-97,

11-102

OBD_MINPER, 10-30 to 10-31          off_temp, 8-193

OBD_PARM_RST, 10-25, 17-11, 23-15,  OFM_FMEM, 27-50

31-11, 31-31, 31-53, 31-60,     OFM_FMEM (READ), 8-164

31-67                           OFMFLG, 25-46 to 25-47, 25-84,

OBD_RCLK_DIF, 10-30 to 10-31            27-50, 28-21

OBD_RCLK_LST, 10-30                 OFMFLG (READ), 5-132

OBD_SW11_DUR, 10-30                 OFMTQMAX_CL, 27-50

OBD_SW12_DUR, 10-30                 OFMTQMAX_SH, 27-50

OBD_SW21_DUR, 10-31                 OKIDM_PW, 35-66

OBD_SW22_DUR, 10-31                 OL_DES, 8-24 to 8-25, 8-48

OBDII Executive -                   OL_DESIRED, 8-25, 8-44, 11-12,

V1.4B_OBDII_EXEC (CDAN0),           11-109

23-2                            OL_DRIVE, 8-42, 8-47, 23-126

OBDII_DELAY, 23-21 to 23-22         OL_DRIVE (READ), 5-134

OBDII_RESET, 5-22, 5-41, 5-64,      OL_FMEM, 8-42 to 8-44, 8-47,

8-153, 8-157, 8-160, 9-97,          23-126

10-25, 11-13, 11-81, 11-100,    OL_FMEM (READ), 5-134

12-33, 14-21, 15-45, 15-100,    OL_HOLD, 8-42 to 8-44, 8-47,

16-30, 18-16, 18-29, 18-33,         23-126

23-15, 23-155, 25-85            OL_HOLD (READ), 5-134

READ, 8-94                        OL_LEAN_TMR, 8-44, 8-51

OBDII_RESET (READ), 13-79           OL_LEAN_TMR (WRITE), 8-42

OBDII_STATE, 23-15 to 23-16,        OLDHEGOHTR11, 11-84, 11-96 to

23-18 to 23-24, 23-41               11-97

OBDII_WARMUP, 23-57, 23-64          OLDHEGOHTR12, 11-84, 11-96 to

OBTAINING STRATEGY EVOLUTION            11-97

INFORMATION (CDAL0), 1-2        OLDHEGOHTR21, 11-84, 11-96 to

OCTADJ, 4-12, 9-18, 31-40               11-97

OCTADJ (READ), 5-133                OLDHEGOHTR22, 11-85, 11-96 to

OCTADJ_CODES, 31-39                     11-97

OCTADJHP, 9-18, 31-40               OLDHTRFALT11, 11-84, 11-96,

OCTANE ADJUST INPUT TEST (CDAL0),       11-101

31-39                           OLDHTRFALT12, 11-84, 11-96,

OD_CODE_CNT, 5-65, 23-59 to 23-60,      11-101

23-62 to 23-64                  OLDHTRFALT21, 11-84, 11-96,

OD_CODE_CNT (READ), 5-144               11-101

ODHI, 25-45                         OLDHTRFALT22, 11-85, 11-96,

ODLO, 25-45                             11-101

Index-45

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OLFLG, 3-9, 8-30, 8-44, 8-47 to     OSC_ENA[17], 23-153

8-48, 8-51, 8-60, 8-271 to      OSC_ENA[4], 23-153

8-272, 10-23, 11-33, 11-38,     OSC_ENA[5], 23-153

11-167, 11-179, 15-19 to        OSC_ENA[channel_number], 23-152

15-20, 15-46, 15-48, 15-71,     OSC_ENA[x], 5-57, 5-61, 23-154

15-90, 15-92, 15-100, 23-133    OSC_MASK, 5-57, 23-152 to 23-153

READ, 8-88                        OSC_OL, 8-24, 23-151, 23-153

OLFLG (READ), 5-133, 8-42 to 8-43,  OSC_SAF_FLG, 9-69 to 9-70, 23-151,

8-302, 15-9, 15-17                  23-153

OLFLG_LST, 8-44                     OSC_SEED, 5-71 to 5-72, 23-151

OLFLG_LST (READ), 8-57              OSC_SS_FLG, 23-151, 23-153

OLFLG_LST (WRITE), 8-42             OSC_STATE, 5-57, 5-60 to 5-61,

OLIDRV, 8-50                            5-71 to 5-74, 23-151 to

OLINEUT, 8-49                           23-152, 23-154

OLITD1, 8-24                        OSC_TANK_MIN, 23-156

OLMCL, 8-44                         OSC_VALUE, 5-91

OLMTD1, 8-49                        OSC_VALUE[channel_number], 23-152,

OLTMR, 8-24, 8-27                       23-156

OLTMR_CROWD, 8-24                   OSC_VALUE[x], 5-61, 5-72, 23-154

OLTMR_CROWD (WRITE), 8-27           OSCILLATION MODULATION (CDAA0),

ON DEMAND SELF TEST FOR EEC             9-47

MEMORY (KAM, RAM, ROM)          OSCNT, 25-93

(CDAK0), 32-6                   OSCTR, 25-93, 26-10

on_edge_time, 8-193, 8-219          OSFMFLG, 25-29, 25-94, 26-10,

ON_FAULT11, 11-96 to 11-98,             26-30, 26-58, 28-17, 28-65,

11-101                              31-16

ON_FAULT12, 11-96 to 11-97,         OSFMFLG (READ), 5-132

11-102                          OSM_1ST_PS, 15-35, 15-83

ON_FAULT21, 11-96 to 11-98,         OSM_EDF_OFF, 16-4, 23-166 to

11-101                              23-167, 23-169 to 23-170,

ON_FAULT22, 11-96 to 11-97,             23-173, 23-175 to 23-176

11-102                          OSM_EDF_ON, 16-4, 23-166 to

OPCLT1, 8-14                            23-167, 23-169 to 23-170,

OPCLT2, 8-14                            23-173, 23-175 to 23-176,

OPCLT3, 8-14                            23-185, 23-190

OPCLT4, 8-14                        OSM_EO_DONE, 23-169 to 23-170

OPCLT5, 8-14                        OSM_EO_ENA, 8-156

OPEN LOOP FLAG DETERMINATION        OSM_EO_OFF, 8-143, 8-152, 11-82,

(CDAM0), 8-20                       12-23, 13-2, 13-8, 14-16,

OPEN LOOP LAMBSE CALCULATION            15-23, 15-35, 15-49, 15-74,

(CDAN0), 8-31                       15-82, 17-12, 17-29, 18-5,

OPEN_EGO11, 11-127 to 11-128,           23-166 to 23-167, 23-169 to

11-164, 11-169                      23-170, 23-173, 23-175 to

OPEN_EGO21, 11-129 to 11-130,           23-176, 26-49 to 26-50, 27-47,

11-164, 11-169                      28-49, 28-51

OPER_RSP_CHK, 23-181, 23-189 to     OSM_EO_ON, 8-143, 8-152, 11-74,

23-190, 25-14, 31-20, 31-44         11-77, 11-81, 12-23, 13-2,

OPNRMPMULT, 28-78                       13-8, 14-16, 15-23, 15-35,

OPNRMPMULTL, 28-78                      15-74, 15-82, 17-12, 17-22,

OPNRMPOFF, 28-78                        17-26, 18-5, 18-10, 23-166 to

OPNRMPOFFL, 28-78                       23-167, 23-169 to 23-170,

OS_MOV, 31-80                           23-173, 23-175 to 23-176,

OS_NOISE_CTR, 25-93                     26-49 to 26-50, 27-47, 28-49,

OS_NOISE_LVL, 25-93                     28-51

OS_NOISE_SW, 25-9                   OSM_EO_WAIT, 23-170

Index-46

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

OSM_HEDF_OFF, 16-8, 23-166 to       OV_DOWN_22, 11-46 to 11-47

23-167, 23-169 to 23-170,       OV_FMEM_11, 11-27

23-173, 23-175 to 23-176        OV_FMEM_12, 11-47

OSM_HEDF_ON, 16-8, 23-166 to        OV_FMEM_21, 11-27 to 11-28

23-167, 23-169 to 23-170,       OV_FMEM_22, 11-47 to 11-48

23-173, 23-175 to 23-176        OV_TIME, 11-27 to 11-28, 11-47 to

OSS INPUT PROCESSING AND SELF           11-48

TEST (CDAN0), 25-89             OV_TMR_11, 8-43, 8-272, 11-27,

oss_codes, 25-91                        11-154

OSS_ERR_SW, 25-9                    OV_TMR_11 (READ), 8-42

OSS_FM_TIM, 25-94                   OV_TMR_12, 11-47, 11-155

OSS_FM_TMR, 25-94                   OV_TMR_21, 8-43, 8-271, 11-27 to

OSS_MON_TIM, 25-94                      11-28, 11-155

OSS_MON_TMR, 25-94                  OV_TMR_21 (READ), 8-42

OSS_SW, 25-94                       OV_TMR_22, 11-47 to 11-48, 11-156

OSSCNT, 25-92                       OV_UP_11, 11-26 to 11-27

OSSMIN, 25-94                       OV_UP_12, 11-46 to 11-47

OSSPUL, 25-92                       OV_UP_21, 11-26 to 11-27

OSSTIM, 25-94                       OV_UP_22, 11-46 to 11-47

OSSTIM1, 25-92                      OVERTEMP_FLG, 17-10, 17-14, 17-18

OSSTIM2, 25-92                      OVERVIEW (CDAI1), 9-2

OSSTMR, 25-94                       OVERVIEW (CDAL0), 3-2

OTCTDLY, 28-18

OTEMP_ERR_SW, 25-9                  P0102, 24-54

OTEMP_EXIT_S (READ), 27-53          P0102_FIL, 24-50 to 24-51, 24-54

OTEMP_FM_FLG, 25-9, 27-14           P0102_INST, 24-48 to 24-49, 24-51

OTEMP_FM_FLG (READ), 5-134, 27-53   P0102_LVL, 24-54

OTEMP_FM_FLG (WRITE), 27-53         P0102_TC, 24-50 to 24-51

OTEMP_TIM (READ), 27-53             P0102MALF, 24-55

OTEMP_TMR (READ), 27-53             P0103, 24-54

OTEMP_TMR (WRITE), 27-53            P0103_FIL, 24-50 to 24-51, 24-54

other_bank (WRITE), 8-318           P0103_INST, 24-46 to 24-50

OTM_MAX_TM, 23-173                  P0103_LVL, 24-54

OTM_STATE, 23-173 to 23-176         P0103_TC, 24-50 to 24-51

OTM_TIMER, 23-173 to 23-174         P0103MALF, 24-55

OTM_TST_ENA, 23-16, 23-19 to        P0112, 31-12

23-21, 23-173                   P0112MALF, 31-13 to 31-14

OTM_TST_RDY, 23-19 to 23-20,        P0113MALF, 31-13 to 31-14

23-173                          P0116MALF, 31-36

OUTPUT SHAFT SPEED LIMITER LOGIC    P0117MALF, 31-32 to 31-33, 31-35

(CDAA0), 8-5                    P0118MALF, 31-32 to 31-33, 31-35

OUTPUT STATE CONTROL TABLE          P0121, 31-74

(CDAL0), 4-49                   P0121_FIL, 31-67, 31-74

OUTPUT SUBSTITUTION CONTROL         P0121_LVL, 31-74

(CDAN2), 23-143                 P0121_TC, 31-74

OUTPUT TEST MODE CONTROLLER         P0122, 31-70

(CDAF0), 23-171                 P0122_FIL, 31-67 to 31-70

OUTPUT_MODE, 5-48 to 5-49, 5-57,    P0122_LVL, 31-70

23-174 to 23-176                P0122_TC, 31-68 to 31-69

output_routing (FCALL), 8-293       P0122MALF, 31-69 to 31-70

output_routing (FUNCTION), 8-311    P0123, 31-70

OV_CLEAR_TM, 11-27, 11-47           P0123_FIL, 31-67 to 31-70

OV_DOWN_11, 11-26 to 11-27          P0123_LVL, 31-70

OV_DOWN_12, 11-46 to 11-47          P0123_TC, 31-68 to 31-69

OV_DOWN_21, 11-26 to 11-27          P0123MALF, 31-69 to 31-70

Index-47

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

P0125MALF, 31-38                    P0301MALF, 35-30, 35-32, 35-48,

P0130FIL, 11-164, 11-169                35-57

P0130LVL, 11-169                    P0302, 35-12

P0130UP, 11-169                     P0302FAULT, 35-52

P0131, 11-25                        P0302MALF, 35-30, 35-32, 35-57

P0132, 11-10, 11-25, 11-27          P0303, 35-12

P0132MALF, 8-25, 11-12              P0303FAULT, 35-52

P0133, 11-25, 11-38, 11-153         P0303MALF, 35-30, 35-32, 35-57

P0133MALF, 11-154                   P0304, 35-12

P0135, 11-25, 11-69, 11-153         P0304FAULT, 35-52

P0135MALF, 8-25, 11-27, 11-139,     P0304MALF, 35-30, 35-32, 35-57

11-154                          P0305, 35-12

P0136, 11-153                       P0305FAULT, 35-52

P0136MALF, 11-56, 11-155            P0305MALF, 35-30, 35-32, 35-57

P0138MALF, 11-47                    P0306, 35-12

P0141, 11-69, 11-137, 11-153        P0306FAULT, 35-52

P0141MALF, 11-47, 11-140, 11-155    P0306MALF, 35-30, 35-32, 35-57

P0150FIL, 11-164, 11-169            P0307, 35-12

P0150LVL, 11-169                    P0307FAULT, 35-52

P0150UP, 11-169                     P0307MALF, 35-30, 35-32, 35-57

P0151, 11-25                        P0308, 35-12

P0152, 11-10, 11-25, 11-28          P0308FAULT, 35-52

P0152MALF, 8-25, 11-12              P0308MALF, 35-30, 35-32, 35-57

P0153, 11-25, 11-38, 11-153         P0320, 35-64, 35-83 to 35-84

P0153MALF, 11-155                   P0325, 9-57

P0155, 11-69, 11-153                P0325MALF, 9-58

P0155MALF, 8-25, 11-27, 11-139,     P0340, 35-64, 35-80

11-155                          P0350, 35-64, 35-87

P0156, 11-153                       P0351, 31-22, 35-64, 35-87

P0156MALF, 11-56, 11-156            P0351MALF, 31-23

P0158MALF, 11-48                    P0352, 31-22, 35-64, 35-87

P0161, 11-69, 11-137, 11-153        P0352MALF, 31-24

P0161MALF, 11-47, 11-140, 11-156    P0353, 31-22, 35-64, 35-87

P01703, 31-20                       P0353MALF, 31-25

P0171MALF                           P0354, 31-22, 35-64, 35-87

CLEAR, 8-97                       P0354MALF, 31-26

SET, 8-97                         P0400, 12-32, 12-53

P0172MALF                           P0401, 12-32, 12-41, 12-51

CLEAR, 8-97                       P0401DN_FLG, 12-41 to 12-42

SET, 8-97                         P0401ENA, 12-41 to 12-42

P0174MALF                           P0401FLT, 12-36, 12-47 to 12-51,

CLEAR, 8-98                           12-53 to 12-54, 12-56

SET, 8-98                         P0401FLT_TMR, 12-56

P0175MALF                           P0401MALF, 12-51

CLEAR, 8-98                       P0401OLD, 12-49, 12-54, 12-56

SET, 8-98                         P0401TST_TM, 12-46

P0230, 8-153                        P0401TST_TMR, 12-45 to 12-46,

P0231, 8-157 to 8-158                   12-56

P0231MALF, 8-158                    P0401UP_FLG, 12-41 to 12-42

P0232, 8-157 to 8-158               P0402, 12-32, 12-35, 12-51

P0300, 35-12                        P0402FLT, 12-37, 12-41, 12-43 to

P0300MALF, 35-30, 35-32, 35-57          12-44, 12-47, 12-50 to 12-51,

P0301, 35-12                            12-53, 12-56

P0301FAULT, 35-52                   P0402FLT_TMR, 12-56

P0402MALF, 12-51

Index-48

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

P0402TST_TM, 12-37, 12-43 to        P0720, 23-161, 25-91

12-44, 12-46                    P0720MALF, 25-9, 25-94

P0402TST_TMR, 12-37, 12-43 to       P0720MON, 23-163, 25-94

12-44, 12-46, 12-56             P0721, 25-91

P0408TST_TM, 12-43                  P0721MALF, 25-9, 27-14

P0408TST_TMR, 12-43                 P0741, 28-47

P0411, 17-28                        P0741CNT, 28-53

P0412, 17-15, 17-22 to 17-23        P0741FAULT, 28-53

P0412MALF, 17-13, 17-26 to 17-27    P0741MALF, 25-9, 28-57

P0420MALF, 10-41                    P0741MON, 28-57

P0430MALF, 10-41                    P0743, 23-161, 28-47

P0442, 15-44                        P0743MALF, 25-8, 28-51, 28-54

P0442MALF, 15-63                    P0743MON, 23-163, 28-51

P0443, 15-34, 15-81, 15-99          P0750, 23-161, 26-47

P0443MALF, 15-47, 15-63, 15-91 to   P0750MALF, 25-8, 26-51

15-93, 15-102                   P0750MON, 23-163, 26-51

P0443MON, 15-36, 15-47, 15-83,      P0751MALF, 26-76 to 26-78

15-91, 15-93, 15-102, 15-105    P0755, 23-161, 26-47

P0446, 15-30, 15-44                 P0755MALF, 25-8, 26-51

P0446MALF, 15-32, 15-63             P0755MON, 23-163, 26-51

P0446MON, 15-32                     P0756MALF, 26-76 to 26-78

P0452, 15-25, 15-30, 15-44          P0781CNT, 26-62

P0452MALF, 15-27 to 15-28, 15-32,   P0781FAULT, 26-62

15-63                           P0781FIL, 26-60 to 26-61, 26-76

P0452MON, 15-28, 15-32                  to 26-77

P0453, 15-25, 15-30, 15-44          P0781MON, 26-60, 26-76

P0453MALF, 15-27 to 15-28, 15-32,   P0782CNT, 26-62

15-63                           P0782FAULT, 26-62

P0453MON, 15-28, 15-32              P0782FIL, 26-60 to 26-61, 26-76

P0455, 15-44, 15-54                     to 26-77

P0455MALF, 15-63                    P0782MON, 26-60, 26-76

P0455MON, 15-54                     P0783CNT, 26-62

P0500, 23-161, 25-97                P0783FAULT, 26-62

P0500MALF, 25-9                     P0783FIL, 26-60 to 26-61, 26-76

P0500MON, 23-163, 25-101                to 26-77

P0503, 25-97                        P0783MON, 26-60, 26-76

P0503MALF, 25-9                     P1000FAULT, 23-41 to 23-42,

P0505, 15-99, 23-180                    23-109, 23-112

P0505MALF, 15-102                   P1001, 23-180, 23-185

P0603, 32-7                         P1001MALF, 23-182

P0605, 32-7                         P1100, 23-43 to 23-44

P0703, 35-64                        P1101, 24-52 to 24-53

P0703MALF, 35-81                    P1112, 23-43 to 23-44

P0704, 31-86, 35-64                 P1117, 23-43 to 23-44

P0704MALF, 35-81                    P1120, 31-70

P0705, 25-85                        P1120_FIL, 31-67 to 31-70

P0705MALF, 25-48 to 25-49, 25-86    P1120_LVL, 31-70

P0707FIL, 25-48 to 25-49            P1120_TC, 31-68 to 31-69

P0707MALF, 25-46, 25-49             P1120MALF, 31-69 to 31-70

P0708FIL, 25-48 to 25-49            P1121, 31-59, 31-61

P0708MALF, 25-46, 25-49             P1121MALF, 31-61

P0712, 31-55                        P1124, 31-75

P0712MALF, 31-55                    P1125, 23-43 to 23-44

P0713, 31-55                        P1127, 11-137, 11-140

P0713MALF, 31-55                    P1128, 11-150

Index-49

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

P1128MALF, 11-150 to 11-151         P1356, 35-64, 35-88

P1129, 11-150                       P1357, 35-64, 35-88

P1129MALF, 11-150 to 11-151         P1358, 35-64, 35-89

P1130FIL, 11-115, 11-117 to         P1359, 9-94

11-119, 11-127, 11-131          P1359(w), 9-98

P1130LVL, 11-115, 11-117, 11-127,   P1359MALF(r), 9-98

11-131                          P1390, 31-40

P1130UP, 11-117 to 11-118           P1400, 12-15, 12-32, 12-34, 12-51

P1131, 11-137, 11-150, 23-180       P1400FLT, 12-16 to 12-17, 12-35,

P1131FIL, 11-115, 11-117 to             12-37, 12-41, 12-43 to 12-44,

11-119, 11-127, 11-131              12-47, 12-50 to 12-51, 12-53,

P1131LVL, 11-115, 11-117, 11-127,       12-56

11-131                          P1400FLT_TMR, 12-56

P1131MALF, 23-187                   P1400MALF, 12-51

P1132, 11-137, 11-150, 23-180       P1400TST_TM, 12-35, 12-46

P1132FIL, 11-115, 11-117 to         P1400TST_TMR, 12-35, 12-46, 12-56

11-119, 11-128, 11-131          P1401, 12-15, 12-32, 12-34, 12-52

P1132LVL, 11-115, 11-117, 11-128,   P1401FLT, 12-16 to 12-17, 12-35,

11-131                              12-37, 12-41, 12-43 to 12-44,

P1132MALF, 23-187                       12-47, 12-52 to 12-53, 12-56

P1132UP, 11-119                     P1401FLT_TMR, 12-56

P1133, 11-39, 11-153                P1401MALF, 12-52

P1133MALF, 11-154                   P1401TST_TM, 12-35, 12-46

P1134, 11-39, 11-153                P1401TST_TMR, 12-35, 12-46, 12-56

P1134MALF, 11-154                   P1405, 12-32, 12-38, 12-52

P1137, 11-137, 11-150               P1405ENA, 12-49

P1138, 11-137, 11-150               P1405FLT, 12-37, 12-41, 12-47,

P1150FIL, 11-115, 11-121 to             12-49 to 12-50, 12-52 to

11-123, 11-129, 11-131              12-54, 12-56

P1150LVL, 11-115, 11-121, 11-129,   P1405FLT_TMR, 12-49, 12-56

11-131                          P1405MALF, 12-52

P1150UP, 11-121 to 11-122           P1405OLD, 12-54, 12-56

P1151, 11-137, 11-151, 23-180       P1405TST_TM, 12-37, 12-41, 12-45

P1151FIL, 11-115, 11-121 to             to 12-46, 12-50

11-123, 11-129, 11-131          P1405TST_TMR, 12-37 to 12-38,

P1151LVL, 11-115, 11-121, 11-129,       12-41, 12-45 to 12-46, 12-49

11-131                              to 12-50, 12-56

P1151MALF, 23-187                   P1406, 12-32, 12-38, 12-52

P1151UP, 11-121                     P1406DN_FLG, 12-38

P1152, 11-137, 11-151, 23-180       P1406ENA, 12-38

P1152FIL, 11-115, 11-121 to         P1406FLT, 12-37, 12-41, 12-47,

11-123, 11-130 to 11-131            12-49 to 12-50, 12-52 to

P1152LVL, 11-115, 11-121, 11-130        12-54, 12-56

to 11-131                       P1406FLT_TMR, 12-49, 12-56

P1152MALF, 23-187                   P1406MALF, 12-52

P1152UP, 11-121, 11-123             P1406OLD, 12-54, 12-56

P1153, 11-39, 11-153                P1406TST_TM, 12-37, 12-41, 12-46

P1153MALF, 11-155                   P1406TST_TMR, 12-37, 12-41, 12-46,

P1154, 11-39, 11-153                    12-49, 12-56

P1154MALF, 11-155                   P1406UP_FLG, 12-38

P1157, 11-137, 11-151               P1408, 12-44, 12-52

P1158, 11-137, 11-151               P1408FLT, 12-52 to 12-53, 12-56

P1232, 8-160                        P1408FLT_TMR, 12-56

P124, 31-74                         P1408MALF, 12-52

P1351, 35-64, 35-85                 P1408TST_TMR, 12-56

Index-50

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

P1409, 12-32, 12-52                 P1479_TMR, 16-17

P1409DN_FLG, 12-58                  P1479MALF, 16-8

P1409ENA, 12-58                     P1483, 16-30, 16-33

P1409FLT, 12-35, 12-37, 12-41,      P1484, 16-30, 16-33

12-43 to 12-44, 12-47, 12-52    P1500, 23-43 to 23-44

to 12-53, 12-56                 P1501, 23-180

P1409FLT_TMR, 12-56                 P1504, 13-85 to 13-86

P1409MALF, 12-52                    P1505, 13-80, 15-99

P1409TST_TM, 12-44, 12-46           P1505MALF, 15-102

P1409TST_TMR, 12-44, 12-46, 12-56   P1506, 13-79, 15-99

P1409UP_FLG, 12-58                  P1506MALF, 15-102

P1413, 17-25 to 17-26               P1507, 13-79, 15-99

P1413MALF, 17-13, 17-27             P1507MALF, 15-102

P1414, 17-16, 17-24, 17-29          P1512, 18-26

P1414MALF, 17-13                    P1513, 18-26

P1443, 15-89, 15-99                 P1516, 18-29, 18-31

P1443MALF, 15-94                    P1516MALF, 18-7, 18-12, 18-16

P1443MON, 15-93 to 15-94, 15-100,   P1517, 18-29, 18-31

15-104 to 15-105                P1517MALF, 18-7, 18-12, 18-16

P1444, 15-76                        P1518, 18-25

P1444MALF, 15-78 to 15-79           P1519, 18-25

P1444MON, 15-79, 15-93              P1519CNT_LVL, 18-21

P1445, 15-76                        P1520, 18-32 to 18-33

P1445MALF, 15-78 to 15-79           P1520_ERRLVL, 18-33

P1445MON, 15-79, 15-93              P1520FIL, 18-33

P1450, 15-44, 15-54                 P1537, 18-26

P1450MALF, 15-63                    P1538, 18-26

P1450MON, 15-54                     P1625, 16-30, 16-33

P1460, 14-19                        P1703MALF, 31-20

P1460_TM_LMT, 14-19                 P1704, 25-85

P1460_TMR, 14-19                    P1704MALF, 25-86

P1461, 31-6                         P1705, 25-85

P1461_TM_LMT, 31-6                  P1705MALF, 25-44

P1461_TMR, 31-6                     P1706MALF, 25-48 to 25-49

P1461MALF, 31-7                     P1709MALF, 25-44

P1462, 31-5 to 31-6                 P1711, 31-56

P1462_TM_LMT, 31-6                  P1711MALF, 31-56

P1462_TMR, 31-6                     P1741, 28-47

P1462MALF, 31-7                     P1741CNT, 28-59

P1463, 31-5 to 31-7                 P1741FAULT, 28-59

P1463_TM_LMT, 31-7                  P1741MALF, 28-62

P1463_TMR, 31-6 to 31-7             P1742, 25-67

P1463MALF, 31-7                     P1742MALF, 25-67

P1464_TM_LMT, 14-3                  P1743, 25-67

P1464_TMR, 14-3                     P1743MALF, 25-9, 25-67

P1469, 14-21 to 14-22               P1744MALF, 28-53, 28-57

P1469_CNT, 14-21 to 14-22           P1744MON, 23-162, 28-57

P1469_LMT, 14-22                    P1746, 27-46

P1469MALF, 25-71                    P1746FIL, 27-50 to 27-51

P1473, 16-30, 16-33                 P1746LVL, 27-51

P1474, 16-16                        P1746MALF, 25-8

P1474_TM_LMT, 16-16                 P1746UP, 27-50

P1474_TMR, 16-16                    P1747, 23-161, 27-46

P1479, 16-17                        P1747FIL, 27-50 to 27-51

P1479_TM_LMT, 16-17                 P1747LVL, 27-51

Index-51

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

P1747MALF, 25-8                     PCOMP_LBM (READ), 8-180

P1747MON, 23-163, 27-51             PCOMP_LBM (WRITE), 15-21

P1747UP, 27-50                      PCOMP_MN_PPM (READ), 15-8

P1751MALF, 26-76 to 26-78           PCOMP_NEA, 13-41

P1756MALF, 26-76 to 26-78           PCOMP_PPM (READ), 15-22

P1780, 25-12                        PCOMP_PPM (WRITE), 15-21

P1781MALF, 25-57                    PCOMP_PPM_C (READ), 15-17

P1783 (READ), 27-53                 PCOMP_PPM_C (WRITE), 15-21

P_DELTA, 28-84                      PCOMP_SE, 11-114

PAIR_A1, 31-23                      PCOMP_SW, 4-16, 4-35, 8-89,

PAIR_A2, 31-23                          11-112, 15-85

PAIR_B1, 31-24                      PCOMP_W, 15-53, 15-100, 15-103

PAIR_B2, 31-24                      PCOMP_W (READ), 15-8

PAIR_C1, 31-25                      PCOMP_W (WRITE), 15-22

PAIR_C2, 31-25                      PCT_LOAD, 12-11, 15-47 to 15-48,

PAIR_D1, 31-26                          18-18, 23-130 to 23-131,

PAIR_D2, 31-26                          24-59, 24-63, 24-66

PARK_ERR, 25-44 to 25-45, 25-48     PCT_LOAD (READ), 15-17

PARK_ERR_TM, 25-48                  PCT_TQ, 26-68

PARKHI, 25-45                       PDL, 13-19, 23-158, 25-43, 25-46

PARKHI_EO, 25-44                        to 25-47, 25-84, 26-4, 26-12

PARKLO, 25-45                           to 26-14, 26-18, 26-26 to

PARKLO_EO, 25-44                        26-28, 26-48, 27-7 to 27-8,

PASSIVE ANTI-THEFT (CDAK0), 5-97        27-10 to 27-11, 27-22, 27-55,

PAT_FAULT, 5-22, 5-107 to 5-108,        28-7, 28-16, 28-26, 29-5

5-110                           PDL (READ), 5-145, 29-4

PAT_FAULT (WRITE), 5-112            PDL_ERR_TM, 25-48, 25-85

PC_PCOMP_H, 15-17, 15-19            PDL_ERR_TMR, 25-48, 25-83, 25-85

PC_PCOMP_H_0, 15-17                 PDL_ERROR, 13-39, 23-158, 25-9,

PC_PCOMP_H_1, 15-17                     25-49, 25-85, 27-14

PC_PCOMP_H_N, 15-17                 PDL_ERROR (READ), 5-133

PC_PCOMP_L, 15-17                   PDL_GND, 25-44 to 25-45, 25-48

PC_PCOMP_L (READ), 15-18            PDL_HGH, 25-44 to 25-45, 25-48

PC_PCOMP_L_0, 15-17                 PDL_LST, 25-43, 26-4, 26-12,

PC_PCOMP_L_1, 15-17                     26-18, 26-30, 26-32, 27-7 to

PC_PCOMP_L_N, 15-17                     27-8, 28-26

PC_PCOMP_RS, 15-18                  PDL_LVL, 25-49

PC_RESET, 15-18, 15-21, 15-71 to    PDL_MON, 23-163, 25-49, 25-86

15-72                           PDL_MON_TM, 25-49, 25-86

PC_RESET (READ), 8-58, 15-17        PDL_MON_TMR, 25-48 to 25-49,

pc_restart (WRITE), 15-7                25-86

PC_RST_RMP, 15-18 to 15-19          PDL_UNLK, 28-16

PC_SCALE_FN (READ), 15-17           PDL_UP, 25-48

PC_SCALE_FN (WRITE), 15-16          PDLTIM, 25-46, 25-83

PC_TP_R, 15-18                      PE_OBD_THRES, 12-37

PCOMP ENABLE (CDAM0), 15-2          PERFORMANCE WEIGHTING

PCOMP_ECT (READ), 15-7                  DETERMINATION (CDAI0), 25-2

PCOMP_ENA, 8-194, 13-34, 13-40,     PERLOAD, 8-15 to 8-16, 8-26, 8-44

15-19, 15-85                        to 8-45, 24-59

READ, 8-89                        PERLOAD_ISC, 13-75

PCOMP_ENA (READ), 15-9, 15-17,      PERLOAD_ISC CALCULATION (CDAC0),

15-22                               13-75

PCOMP_ENA (WRITE), 15-6 to 15-8     PEXHAUST, 12-36 to 12-37, 12-40,

PCOMP_FST_PS (WRITE), 15-7              23-28

PCOMP_LBM, 8-194, 15-17, 15-85      PFEHP, 12-7, 12-33

Index-52

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PFS_CNTS, 4-21, 15-72, 15-78 to     PG_OBD2_NFL (WRITE), 15-22

15-79                           PG_OL_START, 15-20

PFS_CNTS (READ), 5-145              PG_PC_PCOMP, 15-17 to 15-19

PFS_ER_TM, 15-78                    PG_PC_PCOMP (READ), 15-18

PFS_ER_TMR, 15-77 to 15-79          PG_RAMP1, 15-10, 15-19

PFS_FM_TM, 15-79                    PG_RAMP1 (WRITE), 15-19

PFS_FM_TMR, 15-77 to 15-79          PG_RAMP2, 15-10, 15-19

PFS_H_ER, 15-72, 15-78              PG_RAMP2 (WRITE), 15-19

PFS_L_ER, 15-72, 15-78              PG_RP_ML, 15-10, 15-19 to 15-20

PFS_MON_TM, 15-79                   PG_TMR, 15-94

PG_AIR, 24-33                       PG_TSLADP_TM (READ), 15-8

PG_BANK_ML, 15-16                   PG_TSLE, 15-18

PG_DC, 4-16, 4-44, 4-46, 8-75,      PGM_0442_DLP, 15-56, 15-59, 15-61

11-112, 15-10, 15-23, 15-32,    PGM_1ST_PS, 15-92 to 15-94,

15-35 to 15-36, 15-47, 15-53        15-103 to 15-105

to 15-54, 15-71, 15-74, 15-82   PGM_2ND_PS, 15-92 to 15-94,

to 15-83, 15-90, 15-94,             15-104 to 15-105

15-103, 17-21, 24-33            PGM_ADPT_RST (READ), 15-7

READ, 8-89                        PGM_BLD_DLT, 15-55 to 15-56,

pg_dc, 15-72, 15-74                     15-59, 15-61

PG_DC (READ), 5-144, 15-8, 15-21    PGM_BLD_DLTK, 5-121, 15-55

to 15-22                        PGM_BLD_INIT, 15-45

PG_DC (WRITE), 15-22                PGM_BLED_KAM, 15-45, 15-61

PG_DC_D_MIN (READ), 15-8            PGM_CL_TM, 15-47 to 15-48, 15-91

PG_DC_DS, 15-10, 15-20                  to 15-92, 15-102

PG_DC_HYS (READ), 15-21             PGM_CVS_DC, 4-47, 15-49, 15-53 to

PG_DC_LOW, 15-54                        15-54, 15-60

PG_DC_MAX, 15-35 to 15-36, 15-83    PGM_CVS_DC (READ), 5-144

PG_DC_OF, 15-10, 15-20, 24-33       PGM_CVS_DEC, 15-53, 15-60

PG_DC_OF (READ), 15-21 to 15-22     PGM_CVS_FM, 15-32, 15-47 to 15-48

PG_FA_RAT (READ), 15-17             PGM_CVS_FM (READ), 5-132, 15-7

PG_FA_RAT (WRITE), 15-16            PGM_DLT_FLOW, 15-94

PG_FUELPW, 15-18 to 15-19           PGM_DLT_THRS, 15-94

PG_FUELPW (READ), 15-17 to 15-18    PGM_DONE, 15-48, 15-51, 15-61 to

PG_FUELPW (WRITE), 15-16                15-62, 15-92 to 15-93, 15-100,

PG_INJ_PW_HI (READ), 15-18              15-105

PG_INJ_PW_LO, 15-19                 PGM_ERROR, 15-45, 15-48, 15-54,

PG_INJ_PW_R, 15-18                      15-56, 15-59 to 15-62

PG_ISCDTY_HI (READ), 15-18          pgm_filtering, 15-65, 15-100

PG_ISCDTY_LO, 15-19                 PGM_FK, 15-61, 15-104

PG_LAMAV (READ), 15-18              PGM_FK_MAT, 15-61, 15-104

PG_LAMAV (WRITE), 15-16             PGM_FK_NMAT, 15-61, 15-104

PG_LAMBSE (READ), 15-16 to 15-17    PGM_FLOW_1, 15-94

PG_LAMBSE (WRITE), 15-16            PGM_FLOW_2, 15-94

PG_LAMMAX (READ), 15-18             PGM_HW_SW (READ), 15-6 to 15-7

PG_LAMMIN (READ), 15-18             PGM_IPS_1, 15-104

PG_LBMF, 15-17                      PGM_IPS_2, 15-104

PG_LBMF (WRITE), 15-16              PGM_IPS_DLT, 15-104

PG_LK_ML, 15-85, 24-33              PGM_IPS_INIT, 15-100

PG_MULT_INH, 15-19                  PGM_IPS_KAM, 15-100, 15-104 to

PG_MULT_INH (WRITE), 15-18              15-105

PG_OBD2_HFL, 15-21                  PGM_IPS_LIM, 15-105

PG_OBD2_HFL (READ), 15-17, 15-19    PGM_ISC_MAX, 15-101

PG_OBD2_HFL (WRITE), 15-23          PGM_ISC_MIN, 15-101

PG_OBD2_NFL, 15-19 to 15-20         PGM_KWUCNT, 15-61, 15-104

Index-53

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PGM_LD_DT_MX, 15-48, 15-101         PGM_PS_DLTPK, 15-55

PGM_LOAD, 15-65                     PGM_PS_TMR, 15-53 to 15-59, 15-62

PGM_LOAD_DLT, 15-46, 15-48, 15-65,  PGM_RDY_TM, 15-91

15-101                          PGM_RDY_TMR, 15-90 to 15-91

PGM_LOAD_MAX, 15-47 to 15-48,       PGM_RST_CNT, 15-60 to 15-61,

15-90, 15-92, 15-102                15-91

PGM_LOAD_MIN, 15-47 to 15-48,       PGM_RST_CTR, 15-60 to 15-61,

15-92, 15-102                       15-91 to 15-93

PGM_MVAC_MIN, 15-90, 15-92          PGM_RST_TM, 15-60, 15-62, 15-92

PGM_P0_BEGK, 5-121, 15-45, 15-53    PGM_RST_TMR, 15-47, 15-60, 15-91

PGM_P0_DCMIN, 15-53                     to 15-92

PGM_P0_WMX, 15-53                   PGM_RUN_CNT, 15-61

PGM_P0_WPEK, 15-53, 15-55, 15-62    PGM_RUN_CTR, 15-60 to 15-61

PGM_P2_DLTPK, 5-121, 15-45, 15-55   PGM_RUN_LST (READ), 15-7

PGM_P4_DELPK, 5-121, 15-45, 15-58   PGM_RUN_LST (WRITE), 15-6

PGM_P4_DLTPK, 5-121, 15-45, 15-58   PGM_RUNNING, 15-20, 15-32, 15-48

PGM_PCOMP_W, 15-46 to 15-47             to 15-49, 15-62, 24-33

PGM_PFS, 15-79, 15-94               PGM_RUNNING (READ), 15-6 to 15-7

PGM_PFS_FM, 15-78 to 15-79, 15-91   PGM_SELECT, 15-84

to 15-92                        PGM_SELECT (READ), 5-138 to 5-139

PGM_PFS_FM (READ), 5-132            PGM_SS_TM, 15-102

PGM_PG_LO, 15-94                    PGM_SS_TMR, 15-101 to 15-102

PGM_PG_OFF, 15-71, 15-92, 15-94     PGM_ST, 15-45, 15-47 to 15-48,

PGM_PG_ON, 15-53 to 15-54               15-52, 15-54 to 15-57, 15-59

PGM_PG_ON (READ), 15-6                  to 15-60, 15-62

PGM_PG_START, 15-47, 15-90, 15-92,  PGM_TANK_MAX, 15-47 to 15-48

15-103                          PGM_TANK_MIN, 15-47 to 15-48

PGM_PPM_WMN, 15-100                 PGM_TANK_PRS, 15-28, 15-32, 15-46

PGM_PPM_WMX, 15-47, 15-103              to 15-48, 15-53 to 15-55,

PGM_PRS_HI, 15-32                       15-57 to 15-58, 23-156

PGM_PRS_LO, 15-32                   PGM_TANK_PRS (READ), 5-146

PGM_PS0_BEGP, 15-53                 PGM_TC_LOAD, 15-65

PGM_PS0_HI, 5-121, 15-53 to 15-54   PGM_TC_VBAT, 15-65

PGM_PS0_LO, 5-121                   PGM_TC_W, 15-46

PGM_PS0_SET, 15-53, 15-62           PGM_TCPRG, 15-20

PGM_PS0_TM, 15-53                   PGM_TEMP, 15-46 to 15-47, 15-65,

PGM_PS1_MAX, 15-55                      15-90 to 15-92, 15-102

PGM_PS1_SET, 15-55, 15-62           PGM_TEMP_CNT, 15-51

PGM_PS1_TM, 15-55                   PGM_TEMP_CTR, 15-45, 15-51

PGM_PS2_BEGP, 15-55                 PGM_TEMP_FLG, 15-51

PGM_PS2_DLTP, 15-55 to 15-56,       PGM_TEMP_MAX, 15-46 to 15-47,

15-58                               15-91 to 15-92, 15-102

PGM_PS2_SET, 15-55, 15-62           PGM_TEMP_MIN, 15-46 to 15-47,

PGM_PS2_TM, 15-55                       15-91, 15-102

PGM_PS3_MAX, 15-57                  PGM_TEMP_MUL, 15-65, 15-90

PGM_PS3_MIN, 15-57                  PGM_TM, 15-94, 15-104

PGM_PS3_SET, 15-57, 15-62           PGM_TMP_SEEN, 15-46, 15-51

PGM_PS3_TM, 15-57                   PGM_TMR, 15-94, 15-103 to 15-105

PGM_PS4_BEGP, 15-58                 PGM_TP_DT_MX, 15-48, 15-55

PGM_PS4_DELP, 15-58 to 15-59        PGM_TP_MIN, 15-91 to 15-92

PGM_PS4_DLTP, 15-58 to 15-59        PGM_TPR_DLT, 15-46, 15-48, 15-55

PGM_PS4_DP, 5-121, 15-59            PGM_TPR_FM, 15-27 to 15-28, 15-47

PGM_PS4_DPML, 15-58                     to 15-48

PGM_PS4_SET, 15-58, 15-62           PGM_TPR_FM (READ), 5-132

PGM_PS4_TM, 15-58                   PGM_TRG_SEEN, 15-53 to 15-54

Index-54

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PGM_VAC_SEEN, 15-53 to 15-54          SCPBITMAP_5 (WRITE), 5-134

PGM_VALS_GOT, 15-103 to 15-105        SCPBITMAP_6 (WRITE), 5-134

PGM_VB_DT_MX, 15-101                  SCPBITMAP_7 (WRITE), 5-134

PGM_VBAT, 15-65                     PID_MAP_J1979, 23-125

PGM_VBAT_DLT, 15-46, 15-65,         PIDMAP_3, 23-135

15-101                          PIDMAP_4, 23-135

PGM_VS_MAX, 15-47 to 15-48, 15-91   PIP, 9-64

to 15-92, 15-102                PIP COUNTER FOR CRANKING FUEL

PGM_VS_MIN, 15-47 to 15-48, 15-91       (CDAA0), 8-241

to 15-92                        PIP HIGH/LOW TRANSITION (CDAA0),

PID SLIP CONTROL (CDAM0), 28-85         9-71

pid_def                             PIP INPUT ROUTINE (CDAM0), 29-25,

BITMAP_FMEM1 (WRITE), 5-132           29-29

BITMAP_FMEM2 (WRITE), 5-132       PIP INTERRUPT LOGIC FOR FUEL

BITMAP_FMEM3 (WRITE), 5-132           SYNCHRONIZATION (CDAH0),

BITMAP_KAM_1 (WRITE), 5-136           8-173

BITMAP_MON_1 (WRITE), 5-136       PIP_DWN_DEL, 35-15, 35-98

BITMAP_OFD_1 (WRITE), 5-136       PIP_EVNT_CTR, 35-96, 35-100

BITMAP_OFD_2 (WRITE), 5-137       PIP_EVNT_MAX, 35-100

BITMAP_OFD_3 (WRITE), 5-138       PIP_HIGH, 4-10, 8-200, 8-209,

BITMAP_OFD_4 (WRITE), 5-139           9-66 to 9-67, 9-78, 9-101,

BITMAP_OFD_5 (WRITE), 5-141           24-8, 29-30 to 29-31, 35-97

j1979_01_00, 23-135                 READ, 8-171

j1979_01_011, 23-109              PIP_LVL (READ), 5-133

j1979_01_012, 23-135              PIP_ST2_FLG (READ), 8-164

j1979_01_013, 23-135              PIP_ST2_FLG (WRITE), 8-164

j1979_01_014, 23-135              PIPCNT, 6-6 to 6-7, 29-31

j1979_01_031, 8-42                PIPERR, 35-72

j1979_01_06, 8-271                PIPIDM, 35-65, 35-71 to 35-72,

j1979_01_07, 8-83                     35-84 to 35-85

j1979_01_08, 8-271                PIPLIMHYT (READ), 8-164

j1979_01_09, 8-83                 PIPLMT_HIG, 8-8, 8-213

j1979_01_13, 11-162               PIPLMT_HIG (READ), 8-164

j1979_01_14, 11-162               PIPLMT_LOW, 8-8

j1979_01_151, 11-162              PIPLMT_LOW (READ), 8-164

j1979_01_152, 11-162              PIPLVLEXP, 35-83

j1979_01_18, 11-162               PIPNUM, 8-24, 8-47, 8-161

j1979_01_191, 11-162              PIPTSTFIL, 35-29, 35-32, 35-80,

j1979_01_192, 11-162                  35-83

j1979_02_00, 23-125               PIPTSTLVL, 35-83

j1979_02_02, 23-125               PIPUP, 35-83

j1979_02_031, 23-125              PK_LN_FL11, 11-31

j1979_02_04, 23-125               PK_LN_FL21, 11-31

j1979_02_05, 23-125               PK_LN_FLG11, 11-32

j1979_02_06, 23-125               PK_LN_FLG21, 11-32

j1979_02_07, 23-125               PK_RH_FLG11, 11-31 to 11-32

j1979_02_08, 23-125               PK_RH_FLG21, 11-31 to 11-32

j1979_02_09, 23-125               PNWF, 25-4, 26-12 to 26-13, 27-38

j1979_02_0C, 23-125                   to 27-39, 28-38

j1979_02_0D, 23-125               PO442MON, 15-61

SCPBITMAP_1 (WRITE), 5-133        POTN_FAIL[], 35-20 to 35-21,

SCPBITMAP_10 (WRITE), 5-135           35-23

SCPBITMAP_2 (WRITE), 5-133        POWER MODE (CDAA0), 29-16

SCPBITMAP_3 (WRITE), 5-133

SCPBITMAP_4 (WRITE), 5-133

Index-55

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

POWER STEERING PRESSURE SWITCH      PS_VS, 13-59

(PSPS) INPUT AND SELF TEST      PSFLAG, 13-26

(CDAJ0), 31-41                  PSIBRM, 13-32 to 13-33, 13-39 to

POWER STEERING PRESSURE SWITCH          13-40

INPUT (CDAA0), 31-45            PSIBRN, 13-32 to 13-33, 13-40

POWSFG, 13-26, 13-48, 13-59,        PSPN_FLWTST, 17-11, 17-19, 17-30

14-10, 31-43, 31-46             PSPPM, 13-59

PPCTR, 3-9, 8-24, 8-47, 8-132,      PSPPM_VS, 13-59

8-161                           PSPS, 31-46

PPCTR CONTROL (CDAA0), 8-161        PSPS_CODES, 31-42

PREDICTIVE SHIFT SCHEDULE           PSPS_LVL, 4-39, 31-43

COMPENSATION CALCULATIONS       PSPSHP, 13-26, 13-59, 31-43 to

(CDAN0), 25-21                      31-44, 31-46

PRG_DEC, 11-112, 15-73              PSPTHP, 13-59

PRG_FLOW_SW, 13-34, 13-40           PSTMR, 14-10, 31-43

PRG_FLOW_SW (READ), 15-22           PSTPPM, 13-59

PRG_LAM_SUB, 15-72                  PTAMP2, 8-275

PRG_LAM_SW, 15-72                   PTPAMP, 8-275

PRG_LAM_SW (READ), 8-58             PTPAMP1, 8-275 to 8-276, 8-284,

PRG_OFF (WRITE), 15-22                  11-187

PRG_ON_TIME, 4-46 to 4-48             READ, 8-74

PRG_OUT_ENA, 4-46 to 4-48           PTPAMP2, 8-276, 8-284, 11-187

PRG_PERIOD, 4-46 to 4-48              READ, 8-74

PRGFLG, 15-72, 15-74                PTPAMP_MAX, 8-284

PRGIDLE_TMR, 15-70 to 15-71         PTPAMPx, 8-275

PRGMPH_CL, 15-70                    PTSCR, 6-6, 7-3 to 7-4, 13-19 to

PRGMPH_SH, 15-70                        13-20

PRGTD1, 15-71                       PTTMR, 10-23, 10-33

PRGTD2, 15-71                       PUL_PER_GAL, 19-3

PRGTD3, 15-71                       PUL_PER_REV, 26-67, 28-70

PRGTD4, 15-71                       PULSE CALCULATION (CDAI1), 19-2

PRGTD5, 15-71                       PUMP, 3-9

PRGTMR, 15-71, 30-4                 PUMP_OFF_DC, 8-143, 8-152, 8-156

PRLDSW, 24-59                       PUMP_ON_DC, 8-143

PRMERR, 13-41                       PUR_ENA_SW, 13-34

PRNDL, 26-4                         PURG_ADP_SF, 15-73

PRNDL BASED COMMANDED GEAR          PURG_CHKD, 23-69, 23-71 to 23-72

DETERMINATION (CDAN0), 26-24    purg_codes, 15-34, 15-81

PRNDL BASED DESIRED GEAR            PURG_COUNT, 23-69, 23-71

DETERMINATION (CDAN2), 26-2     PURG_DEL_TM, 23-24

PRNDLY, 28-18                       PURG_DEL_TMR, 23-22, 23-24

PROFILE, 35-96, 35-103 to 35-108,   PURG_ER_DONE, 15-45, 15-90,

35-111 to 35-112, 35-114            15-100, 23-188

PROFILE CORRECTION LOGIC (CDAN2),   PURG_ER_INIT, 15-45, 15-90,

35-90                               15-100

PROP_LD_MAX (DEFINED), 8-303        PURG_FMEM, 23-22, 23-35, 23-38

PROP_LD_MAX (READ), 8-304           PURG_MIL_ON, 23-50, 23-52, 23-79,

PROP_LD_MIN (DEFINED), 8-303            23-110

PROP_LD_MIN (READ), 8-304           PURG_MIL_SW, 23-110

PROP_N_MAX (DEFINED), 8-303         PURG_MON, 15-50 to 15-51, 15-93,

PROP_N_MAX (READ), 8-304                15-100, 15-105, 23-22 to

PROP_N_MIN (DEFINED), 8-303             23-23, 23-42, 23-44, 23-52,

PROP_N_MIN (READ), 8-304                23-68 to 23-69, 23-71 to

PS_PPM, 13-49, 13-52                    23-72

PS_PPM CALCULATION (CDAJ0), 13-58   PURG_MON_LST, 23-42, 23-44, 23-52

Index-56

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

PURG_MON_LST := 0, 23-71            PxxxLVL, 11-120

PURG_READY, 23-69 to 23-71,         PxxxMALF, 23-63 to 23-64, 23-117

23-135                          PxxxMIL_ON, 23-58, 23-77 to 23-79,

PURG_TST_ENA, 15-26, 15-35, 15-48,      23-84, 23-86 to 23-87, 23-93

15-77, 15-82, 15-92, 15-103,        to 23-94, 23-96 to 23-98,

23-16, 23-22                        23-103, 23-117 to 23-119

PURG_TST_RDY, 15-47, 15-62, 15-91   PxxxN, 23-130 to 23-131

to 15-92, 15-102 to 15-103,     PxxxSEEN, 23-85 to 23-86, 23-95

23-22, 23-41                        to 23-96, 23-130 to 23-131

PURG_TST_SW, 15-50, 23-22 to        PxxxSTATE, 5-42, 5-62, 23-77 to

23-23, 23-36, 23-52, 23-68 to       23-79, 23-84 to 23-87, 23-93

23-69, 23-135                       to 23-98, 23-103 to 23-104,

PURGE FLOW SENSOR INPUT AND SELF        23-116 to 23-118, 23-120

TEST (CDAJ0), 15-75             PxxxSTATUS, 23-130 to 23-131

PURGE MONITOR FILTERING LOGIC       PxxxTRIP_CT, 23-85, 23-95, 23-130

(CDAL0), 15-64                  PxxxTRIP_FG, 23-85 to 23-86,

PURGSW, 11-114                          23-95 to 23-96, 23-130

PUTMR, 5-10, 5-51, 8-152, 8-156,    PxxxUPDATED, 23-64, 23-77 to

15-28, 15-79, 23-16, 30-4,          23-79, 23-84 to 23-85, 23-87,

31-14, 31-38, 31-55, 35-79          23-93 to 23-95, 23-97 to

PW_EPSILON2, 35-66                      23-98, 23-103, 23-117

PW_MIN_PCT (READ), 8-181            PxxxxMALF, 23-141

PWCAL, 8-224

PWF_AT_DNSFT, 25-4

PWF_DEC_E, 25-4                     QD_CL_TMR2, 10-23 to 10-24

PWM FUNCTIONALITY TEST (CDAK0),

25-58

PWM_BREAKPNT, 28-84                 'R', 30-5

PWM_OFF_CNT, 25-65 to 25-66         R_BIAS (FCALL), 8-314

PWM_OFF_LVL, 25-66                  R_BIAS1, 8-274

PWM_OFFSET, 28-84                   R_BIAS1 (READ), 5-146

PWM_ON_CNT, 25-65 to 25-66          R_BIAS2, 8-274

PWM_ON_LVL, 25-66                   R_BIAS2 (READ), 5-146

PWM_OUT, 28-84                      R_BIAS_AVG (FCALL), 8-309, 8-314

PWMOFFDN, 25-65                     R_BIAS_AVG1, 11-39

PWMOFFUP, 25-65                     R_BIAS_AVG1 (READ), 8-292, 8-311

PWMONDN, 25-65                      R_BIAS_AVG2, 11-39

PWMONUP, 25-65                      R_BIAS_AVG2 (READ), 8-292, 8-311

PWOFF, 8-137, 8-193, 8-219, 8-228   R_BIAS_INT (FCALL), 8-309, 8-314,

to 8-229                            8-319

PWOFF (READ), 8-181                 R_BIAS_INT1 (DEFINED), 8-291

PWOFS_D, 8-137                      R_BIAS_INT1 (READ), 8-319

Pxxx, 23-116 to 23-118              R_BIAS_INT1 (WRITE), 8-292

PxxxACTIVE, 23-85, 23-93, 23-95,    R_BIAS_INT2 (DEFINED), 8-291,

23-97, 23-130                       8-317

PxxxCNT, 5-65, 23-58 to 23-64,      R_BIAS_INT2 (WRITE), 8-292, 8-319

23-116 to 23-118                R_BIAS_PROP (FCALL), 8-304, 8-314

PxxxFAULT, 5-41 to 5-42, 5-62,      RAMP_TM_IN, 9-52, 26-71

23-59 to 23-63, 23-77, 23-103   RAMP_TM_OUT, 9-52, 26-71

to 23-104, 23-116 to 23-118,    RANMUL, 8-72

23-120                          RANNUM, 3-9

PxxxFAULT_A, 23-59, 23-61 to          READ, 8-72

23-63, 23-116 to 23-117           WRITE, 8-72

PxxxFIL, 11-120                     RAT_KAM_SW, 31-66, 31-73

PxxxLOAD, 23-130 to 23-131          RATADD, 31-66

Index-57

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

RATCH, 7-2, 13-3, 13-8, 13-65,      RBIAS_N_MIN (DEFINED), 8-306

27-16, 29-16, 31-66, 31-68,     RBIAS_N_MIN (READ), 8-307

31-71 to 31-73, 31-75           RBIAS_VS_MIN (DEFINED), 8-306

RATCH (READ), 5-144                 RBIAS_VS_MIN (READ), 8-307

RATCH_MIN, 31-73                    RCR, 5-23

RATCH_REP, 31-66, 31-68, 31-73,     REAR EGO TRIM (CDAN2), 8-288

31-75                           REGO1_CT, 10-26, 10-34, 10-37

RATDIFF, 31-73                      REGO1_CT2, 10-34

ratio_puls_width, 8-224             REGO2_CT, 10-27, 10-36 to 10-37,

RATIV, 3-9, 31-66, 31-71 to 31-73       10-39

RATKAM, 31-66, 31-73                REGO2_CT2, 10-36

RATKAM_MIN, 31-66                   rego_ok (FCALL), 8-300, 8-304,

RATUP_LD, 31-71                         8-316

rbias (READ), 8-314                 rego_ok_logic (FCALL), 8-293

rbias (WRITE), 8-314                rego_ok_logic (FUNCTION), 8-299

RBIAS_AVG_MN (DEFINED), 8-312       REGO_TM (DEFINED), 8-296

RBIAS_AVG_MN (READ), 8-314          REGO_TM (READ), 8-297

RBIAS_AVG_MX (DEFINED), 8-313       REP_SPK_DSRD, 9-79

RBIAS_AVG_MX (READ), 8-314          REPET_SPK, 9-78 to 9-79, 9-81

RBIAS_CL_TM (DEFINED), 8-316        REPET_SPK_HI, 9-79

RBIAS_CL_TM (READ), 8-316           REPET_SPK_LO, 9-79

RBIAS_CL_TMR (DEFINED), 8-301       REQUEST_SECURITY_STATUS (CDAK0),

RBIAS_CL_TMR (READ), 8-316              5-115

RBIAS_CL_TMR (WRITE), 8-302         REQV, 11-143, 11-148, 17-33

RBIAS_ECT_MN (DEFINED), 8-305       RESET_DONE, 23-15

RBIAS_ECT_MN (READ), 8-307          RESET_LEAN1, 8-276

RBIAS_ECT_MX (DEFINED), 8-305       RESET_LEAN1 (WRITE), 8-57

RBIAS_ECT_MX (READ), 8-307          RESET_LEAN2, 8-276

RBIAS_EGO_MN, 5-121, 11-39          RESET_LEAN2 (WRITE), 8-57

RBIAS_EGO_MX, 5-121, 11-39          RESET_REQ1, 8-275 to 8-277

RBIAS_EGOSW (FCALL), 8-318 to       RESET_REQ1 (WRITE), 8-57

8-319                           RESET_REQ2, 8-275 to 8-277

rbias_int (READ), 8-319             RESET_REQ2 (WRITE), 8-57

rbias_int (WRITE), 8-319            reset_trip_monitor, 23-71

RBIAS_INT_MN (DEFINED), 8-318       RESULTS, 5-10, 5-65, 23-15

RBIAS_INT_MN (READ), 8-319          RETINC, 9-58, 9-67

RBIAS_INT_MX (DEFINED), 8-318       RETMAX, 9-58, 9-67

RBIAS_INT_MX (READ), 8-319          RETRY_ALLOW, 11-80, 11-84 to

RBIAS_LAM_MN (DEFINED), 8-305           11-85, 11-87, 11-89 to 11-92

RBIAS_LAM_MN (READ), 8-307          REV2NEU_FLG, 27-7 to 27-9

RBIAS_LAM_MX (DEFINED), 8-305       REV_ENG_ADD, 27-21 to 27-22

RBIAS_LAM_MX (READ), 8-307          REVERSE LOCK-OUT SOLENOID CONTROL

RBIAS_LD_MAX (DEFINED), 8-306           (CDAI0), 26-72

RBIAS_LD_MAX (READ), 8-307          REVFLG, 8-19, 25-17, 29-5

RBIAS_LD_MIN (DEFINED), 8-306       REVHI, 25-45

RBIAS_LD_MIN (READ), 8-307          REVLO, 25-45

RBIAS_LN_TM (DEFINED), 8-316        RGM_RST_TM, 15-47

RBIAS_LN_TM (READ), 8-316           RICH_EXCRSN, 8-51

RBIAS_LNTMR (FCALL), 8-307, 8-316   RICH_THRESH, 11-49, 11-56

RBIAS_MAX (DEFINED), 8-313          RMP_OPEN_TMR, 28-80

RBIAS_MAX (READ), 8-314             ROLAV, 9-40, 9-43

RBIAS_MIN (DEFINED), 8-313          rolav (FCALL), 8-295

RBIAS_MIN (READ), 8-314             ROLAV_FK

RBIAS_N_MAX (DEFINED), 8-306          CALL, 8-96

RBIAS_N_MAX (READ), 8-307

Index-58

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

ROM IDENTIFICATION CODE (CDAA0),    S_VAL_TPSUB, 26-58

36-2                            S_VAL_VSADD, 26-58

ROM PORT INPUTS (RPA), 4-12         S_VAL_VSSUB, 26-58

ROM_TO, 32-10                       S_VEGO11, 11-165, 11-178 to

ROWTBU                                  11-179

WRITE, 8-72                       S_VEGO11 (READ), 5-146

RPM ERROR CALCULATION (CDAL0),      S_VEGO12, 11-168

13-44                           S_VEGO12 (READ), 5-146

RPM LIMITER HIGH LOAD (CDAA0),      S_VEGO21, 11-165, 11-169, 11-178

8-8                                 to 11-179

RPM LIMITER LOGIC (CDAN0), 8-162    S_VEGO21 (READ), 5-146

RPMCNL, 9-63                        S_VEGO22, 11-168

RPMCTL, 13-73                       S_VEGO22 (READ), 5-146

RPMDED, 13-31, 13-90                SAF, 3-9, 9-7, 9-27, 9-69, 10-11

RPMERR, 13-36, 13-46                SAF_DES, 9-51 to 9-52, 9-91,

RPMERR_A, 13-31, 13-39, 13-41,          26-71

13-46, 13-90                    SAF_MOD, 9-69, 9-91, 26-71

RPMERR_DA, 13-46                    SAF_MOD_LST, 9-91, 26-71

RPMERR_DAO, 13-36, 13-41, 13-46     SAFTOT, 9-52, 9-69 to 9-70, 9-78,

RPMMAX, 9-59, 9-67                      9-96, 25-17, 26-71

RPMMIN, 9-61                        SAFTOT (READ), 5-144

RR_MULT1, 8-275 to 8-277            SAFTOT_OLD1, 9-96

RR_MULT2, 8-275 to 8-277            SAFTOT_OLD2, 9-96

RR_MULT_GAIN, 8-276                 SAFTOT_OLD2 (r), 9-96

RT4X4L, 25-30                       SAFTOT_OLD2(r), 9-98

RT_GR_CUR, 25-34, 26-27, 26-48,     SAFTOT_PREV, 9-70, 9-78

26-67, 27-33, 28-70             SAIR-PRM_FLG, 17-15

RT_GR_CUR (READ), 5-145             SAIR_AMB_MIN, 17-28

RT_GR_OLD, 26-27, 27-33, 28-34      SAIR_CHKD, 23-69, 23-71 to 23-72

RT_NOVS, 3-9, 25-24, 25-29 to       SAIR_COUNT, 23-69, 23-71

25-30, 26-12, 31-16, 31-83      SAIR_DEL_TM, 23-24

RT_NOVS_KAM, 25-29 to 25-30, 32-4   SAIR_DEL_TMR, 23-22, 23-24

RT_NOVS_KAM CALCULATION (CDAJ0),    SAIR_DLAM, 17-30

25-27                           SAIR_EGO_LN, 17-15, 17-30 to

RTNVMN, 25-29, 32-4                     17-32

RTNVMX, 25-29, 32-4                 SAIR_EGO_LVL, 5-121, 17-27, 17-30,

RTSTAL, 27-11                           17-32 to 17-33

RUN_TMR, 25-26, 25-37, 30-4         SAIR_EGO_RC, 17-15, 17-33

Running Loss OBD-II Evaporative     SAIR_END_FLG, 17-19, 17-21

System Monitor. -               SAIR_END_TM, 17-13, 17-18, 17-21

V1.9A_CANP_MONITOR (CDAN2),     SAIR_END_TMR, 17-13, 17-18 to

15-24                               17-19, 17-21, 17-28

RUNUP_DIFF, 13-46                   SAIR_ER_DONE, 17-14 to 17-17,

RUNUP_DIFF_A, 13-46                     17-23 to 17-25, 17-32 to

RUNUP_FLG, 6-6, 13-7 to 13-8,           17-33, 23-185, 23-187 to

13-46, 13-89                        23-188

RUNUPTMR, 6-6, 13-36, 13-39,        SAIR_ER_INIT, 17-10, 17-12, 17-14,

13-41, 13-89                        23-187

RUNUPTMR (READ), 13-80              SAIR_ER_NOW, 17-11, 17-15, 17-17

RX_PT_STAT_RQST MESSAGE (CDAN0),    SAIR_ER_RPM, 17-15

5-92                            SAIR_FLWTST, 17-11, 17-21, 17-31

SAIR_FMEM, 23-22, 23-35, 23-38

S_USVEGO11, 11-29                   SAIR_HLD_TMR, 17-11, 17-21, 17-30

S_USVEGO21, 11-29                       to 17-31

S_VAL_TPADD, 26-58                  SAIR_HOLD_TM, 17-30

Index-59

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SAIR_LAM_DEL, 17-33                 SAPW_UP_SH, 9-83

SAIR_LAM_DSD, 8-47, 17-21, 17-30    SARCHG, 8-105, 9-18, 24-26, 24-29,

SAIR_LEQV, 17-30                        24-59, 24-63

SAIR_LN_TM, 5-121, 17-28            SAW_ER_TIM(r), 9-98

SAIR_LN_TMR, 17-15, 17-28, 17-31    SAW_ER_TMR(r/w), 9-98

SAIR_MIL_ON, 23-50, 23-52, 23-79,   SAW_ER_TMR(w), 9-97

23-110                          SCCD OUTPUT COMMAND (CDAN0),

SAIR_MIL_SW, 23-110                     12-57

SAIR_MON, 15-50, 17-13, 23-22 to    SCHEDULE DYNAMIC FUEL PULSE

23-23, 23-42, 23-44, 23-52,         ON-EDGE (CDAK0), 8-226

23-68 to 23-69, 23-71 to        SCHEDULED BYPASS CLUTCH SLIP

23-72                               (CDAN0), 28-4

SAIR_MON_LST, 23-42, 23-44, 23-52   SCHEDULED LOCK/UNLOCK LOGIC

SAIR_MON_LST := 0, 23-71                (CDAN0), 28-35

SAIR_OFF_MON, 17-11, 17-13, 17-15   SCINT1, 27-29 to 27-30

to 17-17, 17-24 to 17-25,       SCINT2, 27-29 to 27-30

17-29                           SCINT3, 27-29 to 27-30

SAIR_ON_MON, 17-11, 17-13, 17-15    SCINT4, 27-29 to 27-30

to 17-17, 17-19, 17-22 to       SCP - V6.2A_SCP_EATC_PATS (CDAN0),

17-23, 17-25 to 17-26, 17-28        5-2

SAIR_PG_MIN, 17-21                  SCP BACKGROUND (CDAN0), 5-6

SAIR_PRM_FLG, 17-15 to 17-16,       SCP EXTERNAL INTERRUPT SERVICE

17-23, 17-25                        ROUTINE (CDAK0), 5-13

SAIR_PRM_TM, 17-15, 17-23           SCP MESSAGE CONTROL (CDAN0), 5-18

SAIR_PRM_TMR, 17-15 to 17-16,       SCP MESSAGE OVERVIEW (CDAA0), 5-2

17-23                           SCP MESSAGE TRANSMIT SERVICE

SAIR_RAMP_TM, 17-32 to 17-33            ROUTINE (CDAK0), 5-16

SAIR_RCH_TM, 17-31                  SCP OBDII SAE-J1979 MODE 06

SAIR_RCH_TMR, 17-21, 17-30 to           SUPPORT MAP (CDAN0), 5-117

17-31                           SCP OUTPUT STATE CONTROL MESSAGE

SAIR_READY, 23-69 to 23-71,             RESPONSE HANDLING (CDAM0),

23-135                              5-90

SAIR_REQV, 17-21, 17-30             SCP PID LIST (CDAN0), 5-122

SAIR_STB_TM, 17-19                  scp_162E_b7 (READ), 5-137

SAIR_TMRK1, 5-121, 17-11, 17-28     scp_162E_b7 (WRITE), 5-137

SAIR_TMRK2, 5-121, 17-11, 17-28     scp_162F_b2 (READ), 5-138

SAIR_TST_ENA, 8-47, 10-25, 17-19,   scp_162F_b2 (WRITE), 5-138

17-21, 17-30, 23-16, 23-22      scp_1630_b3 (READ), 5-139

SAIR_TST_ENA (READ), 15-23          scp_1630_b3 (WRITE), 5-139

SAIR_TST_RDY, 17-10 to 17-11,       scp_1632_b7 (READ), 5-141

17-18 to 17-19, 17-21, 17-30,   scp_1632_b7 (WRITE), 5-140

23-22, 23-41                    SCP_DIAG_ST, 5-9 to 5-10, 5-48 to

SAIR_TST_SW, 15-50, 23-22 to            5-49, 5-57, 5-65

23-23, 23-36, 23-52, 23-68 to   SCP_DT1, 5-9

23-69, 23-135                   SCP_DT2, 5-9, 5-109

SAIR_VIPLR1, 17-32                  SCP_DWNLD_ST, 5-9 to 5-11, 5-51

SAIR_VIPRR1, 17-33                      to 5-55, 5-57, 8-324

SAMPLE AIRMETER CALCULATION         SCP_ECT_TMR, 5-24, 5-94

(CDAG0), 24-4                   SCP_ENA_IMS, 5-9, 5-22

SAMPLE_CNT11, 11-30, 11-36          SCP_FAN_GNC, 16-31

SAMPLE_CNT21, 11-30, 11-36          SCP_FAN_GND, 16-32

SAPW, 9-80 to 9-81                  SCP_FAN_ON, 16-31 to 16-32

SAPW_HI_FLG, 9-71, 9-78, 9-80 to    SCP_FAN_OPEN, 16-31 to 16-32

9-83                            SCP_FAN_OVB, 16-31 to 16-32

SAPW_UP_CL, 9-83                    SCP_FAN_PGND, 16-31 to 16-32

Index-60

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SCP_IT1, 5-8                          HTR_FAULT22 (READ), 5-134

scp_odu_ram, 5-130, 8-325           SCPBITMAP_6 (READ), 5-146

scp_odu_ram (READ), 5-146           SCPBITMAP_7

SCP_PACKET[], 5-46 to 5-47            hego_fault (READ), 5-134

SCP_PATS_TMR, 5-107, 5-110            hego_fdback (READ), 5-134

SCP_TMR, 5-8 to 5-9, 5-109            OL_DRIVE (READ), 5-134

SCP_TST_REQ, 5-48 to 5-49, 5-57,      OL_FMEM (READ), 5-134

5-65                              OL_HOLD (READ), 5-134

SCP_UP, 5-9 to 5-10, 5-26           SCPBITMAP_7 (READ), 5-146

SCPBITMAP_1                         SCPBITMAP_x, 5-130

ACSW (READ), 5-133                SCPEO_EXIT, 5-50, 23-20

BIFLG (READ), 5-133               SCPEO_START, 5-48, 23-18

ITCS (READ), 5-133                SCPER_EXIT, 5-50, 23-20

NDSFLG (READ), 5-133              SCPER_START, 5-48, 23-18

PDL_ERROR (READ), 5-133           SCPOTM_EXIT, 5-49, 23-20

SCPBITMAP_1 (READ), 5-144           SCPOTM_START, 5-48, 23-19

SCPBITMAP_10                        SCSLP1, 27-29

FPUMP_SPEED (READ), 5-135         SCSLP1SD, 27-30

SWC (READ), 5-135                 SCSLP2, 27-29

SWC_FAULT (READ), 5-135           SCSLP2SD, 27-30

SCPBITMAP_10 (READ), 5-146          SCSLP2SU, 27-30

SCPBITMAP_2                         SCSLP3, 27-29

ACPSW (READ), 5-133               SCSLP3SD, 27-30

FFG_MISFIRE (READ), 5-133         SCSLP3SU, 27-30

OCTADJ (READ), 5-133              SCSLP4, 27-29

PIP_LVL (READ), 5-133             SCSLP4SU, 27-30

SCPBITMAP_2 (READ), 5-144           SELECT LOWEST SPARK VALUE (CDAL0),

SCPBITMAP_3                             9-8

HSF_FLG (READ), 5-133             SELF TEST CODES

IMRC_CMND (READ), 5-133             1705, 25-44

LSF_FLG (READ), 5-133               1709, 25-44

MIL_ON (READ), 5-133                632, 34-2

OLFLG (READ), 5-133                 P0411, 17-9

TRIP (READ), 5-133                  P0412, 17-9

SCPBITMAP_3 (READ), 5-144             P0503, 25-99

SCPBITMAP_4                           P0705, 25-48

ACCFLG (READ), 5-133                P0707, 25-49

CID_HIGH (READ), 5-133              P0708, 25-49

EAM (READ), 5-133                   P0720, 25-94

TCIL_STATE (READ), 5-133            P0721, 25-93

SCPBITMAP_4 (READ), 5-144             P0741, 28-57

SCPBITMAP_5                           P0743, 28-51

FLG_OT_LK (READ), 5-134             P0750, 26-50

FLG_SS_1 (READ), 5-134              P0751, 26-78

FLG_SS_2 (READ), 5-134              P0755, 26-51

OTEMP_FM_FLG (READ), 5-134          P0756, 26-78

SCPBITMAP_5 (READ), 5-144             P1413, 17-9

SCPBITMAP_6                           P1414, 17-9

HEGOHTR11 (READ), 5-134             P1706, 25-48

HEGOHTR12 (READ), 5-134             P1741, 28-62

HEGOHTR21 (READ), 5-134             P1744, 28-57

HEGOHTR22 (READ), 5-134             P1746, 27-51

HTR_FAULT11 (READ), 5-134           P1747, 27-51

HTR_FAULT12 (READ), 5-134           P1751, 26-78

HTR_FAULT21 (READ), 5-134           P1756, 26-78

Index-61

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

P1780, 25-14                        REPORT_MIL_CODES, 5-67

Self Test Control -                   REPORT_OBDII_CODES, 5-41

V1.8_OBDII_SELF_TEST_CONTROL      REPORT_OBDII_CONT_RESULTS, 5-42

(CDAN2), 23-164                   REPORT_OBDII_FF_PID, 5-41

SELF TEST MIL CODES                   REPORT_OBDII_PID, 5-41

P0102, 24-44                        REPORT_OBDII_RESET, 5-41

P0103, 24-44                        REPORT_OBDII_TRIP_RESULTS, 5-44

P0112, 31-11                        REPORT_PID_DATA, 5-68

P0113, 31-11                        REPORT_RAPID_PACKET, 5-12

P0117, 31-29                        SECURITY_RESPONSE, 5-71

P0118, 31-29                      SEND_ECT, 5-24, 5-94

P0122, 31-65                      SEND_ECT_SW, 5-24

P0123, 31-65                      separate_pulse, 8-228 to 8-229

P0125, 31-29                      sequential_fuel, 8-199, 8-202

P0230, 8-153, 8-159               SETLNG_TM, 13-73

P0231, 8-155, 8-157 to 8-158      setpoint (WRITE), 8-304, 8-316

P0232, 8-155, 8-157 to 8-158      SETTMR, 13-73 to 13-74, 13-89

P0705, 25-78, 25-85               SFT_12_CNT, 26-60 to 26-61

P0712, 31-51                      SFT_12_ERR, 26-76 to 26-77

P0713, 31-51                      SFT_23_CNT, 26-60 to 26-61

P1120, 31-65                      SFT_23_ERR, 26-76 to 26-77

P1121, 31-59                      SFT_34_CNT, 26-60 to 26-61

P1704, 25-78                      SFT_34_ERR, 26-76 to 26-77

SELF TEST NON-MIL CODES             SFT_ERR_FLG, 26-61, 28-7, 28-17,

P1116, 31-29                          28-25, 28-39 to 28-40

SELF TEST non-MIL CODES             SFT_ERR_LVL, 26-61

P0121, 31-65                      SFT_ERR_SW, 25-9

P0704, 31-85                      SFT_ERR_UP, 26-60

P1101, 24-44                      SFT_FM_FLG, 25-9, 26-62, 27-14

P1124, 31-65                      SFT_FM_LVL, 26-62

P1260, 5-107                      SFT_VAL_TM, 26-59

P1390, 31-39                      SHFHYS, 8-130

P1650, 31-42                      SHFRPM, 8-130

P1651, 31-42                      SHIFT MODULATION LOGIC (CDAN0),

P1703, 31-18                          28-22

P1705, 25-78, 25-85               SHIFT SOLENOID FUNCITONAL TEST

P1711, 31-51                          (CDAK0), 26-74

P1781, 25-57                      SHIFT VALIDATION LOGIC (CDAN0),

U1451, 5-107, 5-110                   26-52

SEND                                SHIFTING TORQUE CAPACITY

ENGINE_PAT_STATUS, 5-108              CALCULATION (CDAM0), 28-68

send                                SHK_CHG, 20-3

ANTI_THEFT_STATUS, 5-109 to       SHKTP, 20-3

5-110                         SIAR_LN_TM2, 5-121

CONFIRM_CODES_CLEAR, 5-64         SIGNIFICANT FEATURES OF SCP OBDII

CONFIRM_RAPID_PACKET, 5-46            DIAGNOSTICS (CDAM0), 5-27

DIAGNOSTIC_RESULTS, 5-65          SIM_MIN, 3-9

ECT_DEG_C, 5-22, 5-24, 5-94       SIMILAR CONDITIONS (CDAK0),

ENGINE_PAT_STATUS, 5-116              23-128

GENERAL_RESPONSE, 5-46 to 5-55,   SIMULTANEOUS, 8-199 to 8-200

5-57 to 5-58, 5-60 to 5-68,   simultaneous_fuel, 8-199 to 8-200

5-70 to 5-74, 5-91, 23-18     simultaneous_fuel_scheduling,

to 23-20, 23-174, 23-176          8-200 to 8-201

REPORT_ALL_CODES, 5-62            SIX SERIAL/PARALLEL OUTPUT DRIVER

REPORT_DMR_DATA, 5-66                 (SSPOD), 4-34

Index-62

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SLIP CALCULATION (CDAM0), 28-31     SOAK_TIME (WRITE), 8-115

SLIP_A_UNLK, 26-26                  SOL_CKT_SW, 25-8

SLIP_ABS, 25-34, 26-26, 28-34       SPARK ADVANCE LOGIC, 9-59

SLIP_ABS (READ), 5-145              SPARK FOR FLEXIBLE FUEL (CDAL0),

SLIP_ACT, 25-64, 28-34, 28-39 to        9-86

28-40, 28-76 to 28-78, 28-80,   SPARK FOR LOW LOAD OPERATING

28-87                               CONDITIONS (CDAA0), 9-20

SLIP_ACT_LK, 25-64                  SPARK INTAKE MANIFOLD RUNNER

SLIP_DES, 28-33 to 28-34, 28-54,        CONTROL LOGIC (CDAL0), 9-72

28-60                           SPARK OUTPUT TEST (CDAN0), 9-92

SLIP_DES (READ), 5-146              SPAW, 9-82

SLIP_DES_LST, 28-33, 28-54, 28-60   SPD_PM, 29-16 to 29-17

SLIP_DES_MIN, 28-8                  SPD_RATIO, 23-156, 25-17, 25-34,

SLIP_DES_S, 25-72, 28-30, 28-33         25-63, 26-39, 27-11, 27-27,

to 28-34, 28-79, 28-87 to           27-50, 28-17, 28-38, 28-65,

28-88                               28-70 to 28-71, 28-77, 29-16

SLIP_DIF, 25-64                         to 29-17

SLIP_DIF_AC, 25-72                  SPD_RATIO (READ), 5-145

SLIP_DIF_MX, 28-60                  SPD_RT_MAX, 26-67

SLIP_ERR_D, 28-88                   SPD_RT_STRT, 28-70 to 28-71

SLIP_ERR_I, 28-87 to 28-88          SPEED LIMITER CONTINUOUS SELFTEST

SLIP_ERR_P, 28-88                       CODE SETTING - OBDII (CDAN0),

SLIP_HRD_CH, 28-7                       8-327

SLIP_HRD_SL, 28-7                   SPEED WARNING CHIME (CDAJ0), 21-2

SLIP_OPEN1, 25-63 to 25-64          SPK_ABS_LIM, 9-6, 9-9, 9-33, 9-41

SLIP_STEP, 28-33                    SPK_ACTUAL(r), 9-98

SLIP_TARGET, 28-9, 28-30, 28-33,    SPK_ACTUAL(w), 9-98

28-79                           SPK_ADD, 9-6, 9-102, 9-104

SLIP_TCAP, 28-30, 28-79, 28-81,     SPK_AIR_CHG, 9-101 to 9-102,

28-88                               9-104

SLIP_TRG_S, 25-72, 28-33            SPK_ALT_MIN, 9-37

SLL_DWN_FLG, 9-21 to 9-22           SPK_ARCH_MIN, 9-102

SLL_UP_FLG, 9-21 to 9-22            SPK_ARCH_PRV, 9-101

SLP_ABS_CNT, 28-76                  SPK_BASE, 9-9, 9-14

SLP_ABS_HRD, 28-76                  SPK_BDL, 9-10, 9-18, 9-102, 9-104

SLP_ABS_LVL, 28-76                  SPK_BDL_CAL, 9-101 to 9-102

SLP_CC_ON, 25-64                    SPK_BDL_CLP, 9-70, 9-101 to 9-102

SLP_DES_LST, 28-33                  SPK_BDL_FFS, 9-18, 9-87

SLP_ERR_PID, 28-88                  SPK_BDL_IMRC, 9-18, 9-73

SLP_HRD_OFF, 28-78                  SPK_BDL_UPDT, 9-18 to 9-19, 9-102

SLP_OPN_MIN, 25-64                  SPK_BDL_VCT, 9-18, 9-85

SLP_TDLY_TIM, 28-60                 SPK_CLK_LST, 9-91, 26-71

SLP_TDLY_TMR, 28-60 to 28-61        SPK_CMB_ADJ, 9-14

SLPNOLK, 28-39 to 28-40             SPK_D_AIRCHG, 9-101

SLPRMPOPN, 28-80                    SPK_DARCHMAX, 9-101 to 9-102,

SMPL_EGO_FLG, 8-258, 11-15, 11-17,      9-104

11-29                           SPK_DELT_IN, 9-52, 9-91

SOAK TIME CALCULATION (CDAN0),      SPK_DELT_OUT, 9-91, 26-71

10-14                           SPK_DELTA, 10-11

SOAK_CAT, 10-10, 10-14              SPK_DNDT_C, 9-27

SOAK_HICOMP, 10-14                  SPK_FB_GAIN, 9-43

SOAK_HICOMP (READ), 8-115           SPK_FBS_MIN, 9-37

SOAK_HICOMP (WRITE), 8-115          SPK_FLEX_LIM, 9-6, 9-9, 9-41

SOAK_RST_TM (READ), 8-115           SPK_IDLE_FB, 9-6, 9-37

SOAK_TIME (READ), 8-116

Index-63

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SPK_LAMBSE, 8-51, 9-14, 9-18,       SR_TRIG_END, 26-67 to 26-68

9-31, 10-11 to 10-12, 25-17     SRLK_FM, 28-65

SPK_LAMBSE(read), 35-33             SRLKOT, 28-38

SPK_LDRATE, 9-27                    SRLUMN, 28-17

SPK_LOAD_RES, 9-42                  SRLUNNOT, 28-17

SPK_LOW_LOAD, 9-9, 9-21 to 9-22     SS1_CODES, 26-47

SPK_M_B_T, 9-26, 9-28, 9-31, 9-33,  SS1_COM, 26-49

9-37, 9-41, 9-51, 10-11,        SS1_COM_LST, 26-49 to 26-50

25-17                           SS1_DES, 26-48 to 26-49

SPK_MAX_TRET, 9-102, 9-104          SS1_ERR_TMR, 26-50

SPK_MBT_DELT, 9-58, 9-62            SS1_ERROR_TM, 26-50

SPK_MBT_IMRC, 9-31, 9-73            SS1_FAULT, 4-32, 26-50

SPK_MBT_LAST, 9-27, 9-31            SS1_FAULT (READ), 5-141

SPK_MBT_VCT, 9-31, 9-85             SS1_MON_OFF, 26-50 to 26-51

SPK_MUL, 9-6, 9-102, 9-104          SS1_MON_ON, 26-50 to 26-51

SPK_OSC_OFF, 9-9, 9-48              SS1MALF, 26-76 to 26-77

SPK_OSCMOD, 9-6, 9-48               SS2_CODES, 26-47

SPK_POS_ERR(r), 9-98                SS2_COM, 26-49

SPK_RMP_TMR, 9-69, 9-90 to 9-91,    SS2_COM_LST, 26-49, 26-51

26-71                           SS2_DES, 26-48 to 26-49

SPK_SOURCE, 9-6, 9-69               SS2_ERR_TMR, 26-50 to 26-51

SPK_SRC_IFB, 9-6                    SS2_ERROR_TM, 26-51

SPK_STATE, 9-6, 9-40, 9-43, 9-45    SS2_FAULT, 4-32, 26-51

spk_tar, 9-40                       SS2_FAULT (READ), 5-141

SPK_TIP_BDL, 9-102                  SS2_MON_OFF, 26-51

SPK_TIP_ECT, 9-27                   SS2_MON_ON, 26-51

SPK_TIP_NOVS, 9-27                  SS2MALF, 26-76 to 26-77

SPK_TIPMAX, 9-61, 9-66              SS_MALF_FLG, 23-162, 26-77

SPK_TIPSTATE, 9-101 to 9-102,       SS_MIL_SW, 26-78

9-104                           SS_TRIP_DONE, 23-162 to 23-163

SPK_TRUNC, 9-10, 9-26               SS_TRIP_SW, 23-162

SPKAD, 9-59, 9-61, 9-67, 9-69,      SSCYCCLD, 26-48

9-101 to 9-102                  STAB_LOLD_AM, 9-21 to 9-22

SPKBASE_IMRC, 9-14, 9-73            START STOP COUNTER FOR PSPS

SPKBASE_VCT, 9-14, 9-85                 (CDAN0), 31-47

SPKCTL, 9-46                        START-UP TV LOGIC (CDAA0), 27-16

SPKDELT, 9-52                       START_CVS_PER, 4-48

SPKDELT2, 9-52                      STATE CONTROL-FUEL (CDAK0), 23-80

SPKSLP, 9-52, 9-91, 26-71           STATE CONTROL-MISFIRE (CDAK0),

SPKTM2_IN, 9-52                         23-88

SPKTM2_OUT, 9-52                    STATE CONTROL-TYPE 1 (CDAK0),

SPKTM_IN, 9-52                          23-73

SPKTM_OUT, 9-52                     STB_LOLD_TMR, 9-20 to 9-22

SPKTMR, 9-45 to 9-46                STCF, 24-64, 26-67, 28-70

SPLCLP, 9-7                         stcf, 8-193

SPOUT, 3-4                          STOP_EXCURS, 11-17, 11-50 to

SPOUT CALCULATION (CDAN0), 9-68         11-55

SPOUT_LOW_TIME, 9-80 to 9-82        store_code (FCALL), 27-53

SPTST_CONT, 9-97                    store_code(Pxxx), 23-93 to 23-94

SPTST_ER_ENA, 9-97                  store_ff(Pxxx), 23-49

SPUCLP, 9-7, 9-21 to 9-22, 9-26,    STRAEGY MODULES

9-28                              FUEL_DYN_OFF_COM2, 8-231

SR_HL, 28-77                        STRATEGY MODULE

SR_HL_OT, 28-77                       CAT_FTP_STANDARD_MONITOR, 10-23

SR_PP_LIM, 28-70                    STRATEGY MODULES

Index-64

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

ACC_CONTROL_COM2, 14-8              CCC_UNCOND_UNLK_COM10, 28-14

ACC_INPUT_COM3, 14-3                CLFUL_BIAS_COM4, 8-281

ACC_OBD_COM1, 14-21                 CLFUL_CONTROLLER_COM1, 8-271

ACC_OSM_COM1, 14-19                 CLFUL_PTPAMP_COM4, 8-284

ACC_OUTPUT_COM1, 14-16              CLFUL_TRANS_DELAY_COM2, 8-287

ACC_TIMER_COM1, 14-12               CRK_UNDSP_RUN_COM2, 6-6

AIR_AIRLEAK_COM4, 24-33             DOL_PULSE_CALC_COM4, 19-3

AIR_AIRMETER_BG_COM3, 24-26         EDF_CONTROL_COM1, 16-12

AIR_ANTICIPATION_COM2, 24-18        EDF_HSF_CONTROL_COM4, 16-8

AIR_COMP_BRAKE_COM1, 24-37          EDF_LSF_CONTROL_COM1, 16-4

AIR_FINAL_AIRCHARGE_COM1, 24-20     EDF_OSM_COM1, 16-16

AIR_FMEM_OBDII_FG_COM1, 24-11       EDF_OUTPUT_COM1, 16-35

AIR_J1979_COM1, 24-64               EDF_VSF_AC_COM1, 16-20

AIR_J1979_LOAD_COM1, 24-66          EDF_VSF_ECT_COM1, 16-22

AIR_LOAD_COM1, 24-59                EDF_VSF_FINAL_COM1, 16-24

AIR_LOAD_INFERRED_COM2, 24-62       EDF_VSF_OUTPUT_COM2, 16-27

AIR_MAFS_DIAG_BG_COM1, 24-45        EEC_CD, 3-2

AIR_MAFS_DIAG_SELECT_COM1,          EGO_BYPLES_COM1, 11-187

24-40                           EGO_CONTINUOUS_TEST_COM10,

AIR_MANIFOLD_FILL_COM3, 24-14           11-108

AIR_OVERVIEW_COM2, 24-3             EGO_CSD_COMPENSATION_COM2,

AIR_SAMPLE_METER_COM5, 24-7             11-173

ALTR_CAL_CLR_COM1, 22-5             EGO_CSD_DETECTION_COM1, 11-177

ALTR_CAL_INIT_COM1, 22-3            EGO_DS_MONITOR_COM10, 11-45

ANTI_THEFT_COM4, 5-105              EGO_ER_COM10, 11-138

CANP_COM12, 15-70                   EGO_FFG_CONTROL_COM10, 11-154

CANP_CVS_FMEM_COM1, 15-31           EGO_HEATER_CONTROL_COM10, 11-70

CANP_INPUT_PFS_COM1, 15-77          EGO_HEATER_IO_COM2, 11-95

CANP_INPUT_TPR_COM1, 15-26          EGO_HW_XFER_COM1, 11-171

CANP_MONITOR_COM2, 15-45            EGO_INPUT_COM10, 11-162

CANP_OSM_TEST_COM2, 15-82           EGO_INPUT_XFER_COM1, 11-181

CANP_PCOMP_COM10, 15-16             EGO_LAM_CLIP_COM1, 11-182

CANP_PCOMP_ENABLE_COM7, 15-6        EGO_MONITOR_CONTROL_COM10,

CANP_PGM_FILT_COM1, 15-65               11-11

CANP_PGM_SELECT_COM3, 15-84         EGO_REV_COUNTER_COM1, 11-184

CANP_SELECT_COM2, 15-85             EGO_US_MONITOR_COM10, 11-26

CANP_THERM_FLOW_COM1, 15-90         EGO_US_SWTPOINT_COM1, 11-185

CANP_VMV_FLOW_COM1, 15-100          EGR_ACTUAL_COM1, 12-25

CANP_VMV_OSM_COM4, 15-35            EGR_CONTROL_COM2, 12-23

CAT_TEMP_COM1, 10-9                 EGR_DESIRED_COM4, 12-10

CCC_BYP_CLTCH_CTL_COM6, 28-2        EGR_ENABLE_COM2, 12-6

CCC_BYP_CLTCH_TESTS_COM2, 28-30     EGR_INIT_COM1, 12-3

CCC_BYP_SLIP_CALC_COM5, 28-33       EGR_OBDII_COM1, 12-33

CCC_FMEM_COM3, 28-65                EGR_PFE_COM3, 12-16

CCC_MLUS_OBDII_COM5, 28-47          EGR_SCCD_COM1, 12-57

CCC_MOD_COM2, 28-25                 EPC_ASILT_COM1, 27-55

CCC_PID_SLIP_CTL_COM1, 28-87        EPC_DYNAMIC_COM14, 27-37

CCC_SCHLD_BYP_SLIP_COM5, 28-6       EPC_GUIDE_COM17, 27-7

CCC_SCHLD_LCK_UNLCK_COM16,          EPC_NORM_COM1, 27-27

28-38                           EPC_VFS_OBDII_COM11, 27-47

CCC_TCAP_CALC_COM2, 28-81           EXP_VS_CHIME_COM2, 21-3

CCC_TCAP_CONV_COM3, 28-84           FADPT_FUEL_MONITOR_COM1, 8-94

CCC_TCAP_DETR_COM2, 28-67           FADPT_GENERIC_COM1, 8-70

CCC_TCAP_NORM_CALC_COM1, 28-76      FADPT_UPDATE_STATUS_COM1, 8-87

CCC_TCAP_SHFT_CALC_COM1, 28-70      FAN_VLCM_TST_COM1, 16-30

Index-65

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

FPUMP_2SPD_CTRL_COM1, 8-146         IMRC_DIAG_OSM_COM1, 18-33

FPUMP_CTRL_COM1, 8-143              IMRC_INPUT_COM2, 18-29

FPUMP_DRVR_TST_COM1, 8-152          IMRC_OUTPUT_COM1, 18-34

FPUMP_HI_LO_TST_COM1, 8-160         INPUT_ACPRES_COM2, 31-5

FPUMP_PWM_OUTPUT_COM1, 8-148        INPUT_ACPSW_COM2, 31-90

FPUMP_SELECT_COM2, 8-141            INPUT_ACT_OBDII_COM1, 31-11

FPUMP_SMON_TST_COM1, 8-156          INPUT_AODE_EQNS_COM1, 31-16

FPUMP_SPEED_OUTPUT_COM1, 8-149      INPUT_BOO_OBDII_COM1, 31-19

FTRAN_XTMODEL_BG, 8-105             INPUT_BP_FILTER_COM1, 29-14

FTRAN_XTMODEL_FG, 8-110             INPUT_BP_INFERRED_COM1, 29-9

FUEL_BG_CALC_COM18, 8-136           INPUT_CLOCK_SEC_COM1, 31-89

FUEL_CID_SYNC_COM1, 8-168           INPUT_CPP_OBDII_COM1, 31-86

FUEL_COOL_FAIL_COM2, 8-320          INPUT_DIS_COM3, 31-23

FUEL_CRK_INJ_COM1, 8-254            INPUT_ECT_OBDII_COM4, 31-30

FUEL_CRKPIPCTR_COM1, 8-242          INPUT_EPCOCM1_COM2, 29-15

FUEL_CTNTMR_COM1, 8-244             INPUT_N_CALC_COM2, 29-23

FUEL_DFSO_COM9, 8-129               INPUT_NOVS_COM1, 31-87

FUEL_DYN_CALC_COM3, 8-223           INPUT_OCTADJ_OBDII_COM1, 31-40

FUEL_DYN_FIT_COM2, 8-228            INPUT_PIP_BG_COM1, 29-28

FUEL_FAR_MUL_COM1, 8-324            INPUT_PIP_FG_COM1, 29-30

FUEL_FG_CALC_COM14, 8-179           INPUT_PSPS_COM1, 31-43

FUEL_FG_PIPEDG_COM3, 8-171          INPUT_PSPS_SWITCH_COM1, 31-46

FUEL_HOT_INJ_COMP_COM4, 8-115       INPUT_SOAK_TIMER_COM2, 10-14

FUEL_IBETA_CALC_COM1, 8-208         INPUT_START_STOP_OBDII_COM1,

FUEL_IDLTMR_COM1, 8-246                 31-48

FUEL_INJ_OFF_BG_COM3, 8-236         INPUT_TOT_OBDII_COM2, 31-52

FUEL_INJ_ON_COM1, 8-240             INPUT_TP_BG_COM1, 31-91

FUEL_INJ_OUT_COM12, 8-219           INPUT_TP_MAF_OBDII_COM1, 31-60

FUEL_INJ_UPDATE_REQUEST_COM1,       INPUT_TP_OBDII_COM1, 31-66

8-189                           INPUT_VBAT_COM2, 31-88

FUEL_INJDLY_COM1, 8-121             INPUT_VMOVING_COM4, 31-79

FUEL_LAMBSE_CLIP_COM4, 8-60         INPUT_VSS_DARC_COM2, 31-82

FUEL_LAMBSE_MOD_COM1, 8-257         INTRN_4X4L_OBDII_COM1, 25-57

FUEL_LAMBSE_RESET_COM13, 8-55       INTRN_AC_EN_TRANS_COM1, 25-71

FUEL_MAIN_OFF_COM11, 8-193          INTRN_AODE_INPUT_EQNS_COM1,

FUEL_MULTMR_COM1, 8-248                 25-33

FUEL_NO_LIM_COM1, 8-7               INTRN_AODE_OVERVIEW_COM2, 25-54

FUEL_OL_LAMBSE_COM25, 8-42          INTRN_CALC_VS_RATE_COM1, 25-20

FUEL_OLFLG_COM28, 8-24              INTRN_CSM_COM3, 25-37

FUEL_PIP_INT_SYNC_COM2, 8-174       INTRN_IMRC_CTL_COM1, 25-88

FUEL_RPM_LIM_FG_COM4, 8-164         INTRN_MLPS_CONV_COM9, 25-43

FUEL_RPM_LIM_OBDII_COM1, 8-327      INTRN_OSS_OBDII_COM1, 25-92

FUEL_SCHEDULE_COM13, 8-212          INTRN_PWF_DET_COM3, 25-4

FUEL_SERVICE_COM1, 8-199            INTRN_PWM_FUNCT_TEST_COM3,

FUEL_TIMERS_COM1, 8-251                 25-63

FUEL_TOTAL_DELAY_CALC_COM1,         INTRN_RT_NOVS_KAM_CALC_COM2,

8-185                               25-29

FUEL_WRMEGO_COM1, 8-14

HWINT_INPUT_CD, 4-39

INTRN_SHFT_SCHLD_COMP_CALC_COM4,

HWINT_IO_CD, 4-2                        25-23

APPLICATION SUMMARY, 4-2          INTRN_TCIL_STATE_COM5, 25-8

HWINT_OSC_CD, 4-49                  INTRN_TCS_OBDII_COM1, 25-13

HWINT_OUTPUT_CD, 4-44               INTRN_TQ_IN_CALC_COM2, 25-75

IMRC_COMMAND_COM1, 18-5             INTRN_TQ_INTR_COM1, 25-51

IMRC_DIAG_OBDII_COM1, 18-16

Index-66

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

INTRN_TRANS_TQ_NET_CALC_COM12,      OBDII_TYPE1_STATE_CONTROL_COM1,

25-17                               23-77

INTRN_TRS_COM1, 25-79               RIDE_CONTROL_COM4, 20-3

INTRN_TST_ENA_COM1, 25-26           SC_CM_GR_AUTO_DWN_COM7, 26-39

INTRN_VSS_TEST_COM3, 25-98          SC_CM_GR_DETR_COM8, 26-26

ISC_DASPOT_COM5, 13-64              SC_CM_GR_MAN1_COM5, 26-30

ISC_DESMAF_ACPPM_COM3, 13-54        SC_CM_GR_MAN2_COM5, 26-32

ISC_DESMAF_COM1, 13-52              SC_DESRD_GR_DETR_COM10, 26-4

ISC_DESMAF_CYLPPM_COM1, 13-57       SC_GR_DS_AUTO_COM22, 26-10

ISC_DESMAF_DSPPM_COM1, 13-59        SC_RLS_COM1, 26-73

ISC_DESMAF_EDFPPM_COM1, 13-55       SC_SOL_CTL_COM18, 26-48

ISC_DSDRPM_BASE_COM1, 13-13         SC_SOL_FN_TEST_COM1, 26-76

ISC_DSDRPM_COM10, 13-10             SC_TIMER_COM2, 26-43

ISC_DSDRPM_HICAM_COM1, 13-18        SC_TQMOD_COM9, 26-66

ISC_DSDRPM_LOADS_COM1, 13-25        SC_TQMOD_FG_COM1, 26-71

ISC_DSDRPM_TOT_COM2, 13-28          SC_VALID_OBDII_COM1, 26-57

ISC_DTY_CONV_COM2, 13-85            SC_VER_AUTO_SHFT_COM2, 26-18

ISC_FMEM_COM1, 13-79                SCP_ANTI_THEFT_STATUS_COM1,

ISC_HCAMFG_COM1, 13-48                  5-112

ISC_IPSIBR_COM2, 13-39              SCP_BKGND_COM1, 5-8

ISC_ISCKAM_COM1, 13-31              SCP_ECT_DEG_C_COM1, 5-96

ISC_ISCKAM_HLT_COM2, 13-34          SCP_ENGINE_PAT_STATUS_COM1,

ISC_ISFLAGG_COM1, 13-42                 5-114

ISC_MODE_SELECT_COM2, 13-73         SCP_EXTINT_COM1, 5-15

ISC_OVERVIEW_COM8, 13-7             SCP_INIT_COM2, 5-26

ISC_PERLOAD_ISC_COM1, 13-75         SCP_LOAD_TX_BUFFER_COM1, 5-17

ISC_RPMERR_COM3, 13-46              SCP_MODE06_MAP_NAAO_COM1, 5-120

ISC_TIMERS_COM1, 13-89              SCP_MSG_CONTROL_V6_COM1, 5-22

KAM_CC, 32-4                        SCP_OBDII_DIAG_COM1, 5-40

KAM_RAM_ROM_TEST_COM1, 32-8         SCP_OSC_RESPONSE_COM1, 5-91

OBDII_CCM_MON_COM1, 23-138          SCP_OVERVIEW_COM1, 5-2

OBDII_CL_STATE_CONTROL_COM1,        SCP_PID_MAP_CD, 5-132

23-103

OBDII_CODES_COM1, 23-57

SCP_REQUEST_SECURITY_STATUS_COM1,

OBDII_DRIVE_CYCLE_COM1, 23-133          5-116

OBDII_FAULT_FILTER_COM1, 23-141     SCP_RX_PT_STAT_RQST_COM3, 5-94

OBDII_FREEZE_FRAME_COM1, 23-125     SPARK_BASE_COM10, 9-14

OBDII_FUEL_STATE_CONTROL_COM1,      SPARK_BORDERLINE_COM7, 9-18

23-84                           SPARK_CYL_KNOCK_COM1, 9-58

OBDII_J1979_PIDS_COM1, 23-135       SPARK_FINAL_CALC_COM2, 9-6

OBDII_KAM_VERIFICATION_COM1,        SPARK_IDLE_FB_COM5, 9-32

23-114                          SPARK_IMRC_COM3, 9-73

OBDII_MIL_CONTROLLER_COM1,          SPARK_LOWEST_COM1, 9-9

23-49                           SPARK_MBT_COM1, 9-31

OBDII_MIL_OUTPUT_COM1, 23-109       SPARK_NO_FLEX_FUEL_COM1, 9-87

OBDII_MIS_STATE_CONTROL_COM1,       SPARK_NO_VCT_COM1, 9-85

23-93                           SPARK_OSCMOD_COM1, 9-48

OBDII_OSC_COM2, 23-151              SPARK_SPOUT_COM4, 9-69

OBDII_SCHEDULER_COM1, 23-14         SPARK_SPOUT_TEST_COM1, 9-96

OBDII_SIMILAR_CONDS_COM1,           SPARK_TIPIN_CONTROL_BG_COM1,

23-130                              9-104

OBDII_SYSTEM_FMEM_COM1, 23-35       SPARK_TIPIN_CONTROL_FG_COM1,

OBDII_TRANS_MON_COM1, 23-162            9-101

OBDII_TRIPS_COM1, 23-68             SPARK_TQM_BG_COM5, 9-51

SPARK_TQMOD_RMP_COM2, 9-90

Index-67

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

SPARK_TRQ_TRNC_COM1, 9-26         SWC, 3-4, 4-35

THERM_EAM_OBDII_COM4, 17-10       SWC (READ), 5-135

TIMER_CD, 30-4                    SWC_FAULT, 4-35

VC_DIS_COM6, 35-65                SWC_FAULT (READ), 5-135

VC_MISFIRE_DETEC_COM4, 35-13      SWC_HP, 4-35, 21-3

VC_PROFILE_COM7, 35-96            SWCVS_CH, 21-3

VO_EDISMT_COM1, 33-2              SWCVS_CL, 21-3

VO_OBDII_OTM_COM1, 23-173         SWCVS_FLG, 21-3

VO_OBDII_SEQUENCER_COM1, 23-166   SWITCH_CNT11, 11-30, 11-36

VR_OBDII_SEQUENCER_COM1, 23-181   SWITCH_CNT21, 11-30, 11-36

VR_TCS_COM2, 34-2                 SWITCHING CONSTANT CURRENT DRIVER

STROKE_DRP, 28-77                       (SCCD), 4-36

STROKE_SLP, 28-77                   SWPT_STABL11, 11-173

STROKE_TM, 28-77                    SWPT_STABL21, 11-173

STROKE_TMR, 28-72, 28-77, 28-87     swtfl (READ), 8-318

SUBST_REQ, 5-91                     SWTFL11, 8-74, 8-274, 10-30,

SUBST_REQ1, 17-14                       11-126, 11-164, 11-166 to

SUBST_REQ11, 26-48                      11-168, 11-178, 15-19

SUBST_REQ12, 26-48                    READ, 8-96

SUBST_REQ15, 28-48                  SWTFL21, 8-30, 8-74, 8-274, 10-31,

SUBST_REQ17, 9-7                        11-126, 11-164, 11-166 to

SUBST_REQ21, 18-5                       11-168, 11-178

SUBST_REQ25, 11-100                   READ, 8-96

SUBST_REQ26, 11-100                 SWTFLG11, 8-30

SUBST_REQ27, 11-100                 SWTFLx1, 8-275

SUBST_REQ28, 11-100                 SWTP_BAR11, 11-29, 11-164 to

SUBST_REQ[0], 13-7                      11-165

SUBST_REQ[channel_number], 23-152   SWTP_BAR21, 11-29, 11-164 to

SUBST_REQ[x], 5-61, 23-154              11-165

substitute(0,ISCDTY), 13-82         SWTP_FC_AVG, 11-165

substitute(11,flg_ss_1), 26-49      SWTP_STABL11, 11-33, 11-174

substitute(12,flg_ss_2), 26-49      SWTP_STABL21, 11-33, 11-174

substitute(16,TV_PRES), 27-15       SWTPOINT11, 11-165, 11-173 to

SUBSTITUTE(7,EGRDC), 12-24              11-174

substitute(9,pg_dc_new), 15-22      SWTPOINT21, 11-165, 11-173 to

SW_0741_NMIL, 28-57                     11-174

SW_0741_TCIL, 25-9                  SYMBOLOGY (CDAA0), 2-2

SW_1742_MIL, 25-67                  SYNC_CTR, 8-168, 8-174, 8-202,

SW_1743_NMIL, 25-67                     8-208, 8-223, 11-184, 23-27,

SW_1743_TCIL, 25-9                      35-16, 35-69 to 35-70, 35-77

SW_1744_MIL, 28-53, 28-57               to 35-78, 35-97

SW_AC_PID, 28-88                      READ, 8-172

SW_ANTI_THFT, 5-105 to 5-106        SYNC_CTR_C, 8-174

SW_BP_INTR, 26-10                   SYNC_FAIL, 8-168, 8-174

SW_DB_D, 28-88                      SYNFLG, 8-168, 8-174, 8-199,

SW_DYN, 27-37                           8-208, 8-223, 9-61, 29-23

SW_DYN_VS, 27-37                      READ, 8-171

SW_EN_PSTR, 28-67                   SYS_DELPR, 12-23, 12-40

SW_MSF, 26-26                       sys_MIL_ON, 23-119

SW_PWM_TST, 25-63                   SYSTEM FMEM (CDAN0), 23-25

SW_SEQ_MAN2, 26-32

SW_SFTIN, 26-4

SW_STALTV, 27-11, 27-20             t_current_pw, 8-224

SW_SUPD, 28-70                      T_TO_S, 8-110

SWBCTST, 28-2, 28-30                table (FCALL), 8-304, 8-316

Index-68

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TANK PRESSURE SENSOR INPUT AND      TCC_ERR_TIM, 28-51

SELF TEST (CDAJ0), 15-24        TCC_H_FAULT, 4-33, 28-50 to 28-51

TARMAX, 9-40                        TCC_MON_OFF, 28-51

TBART, 3-9, 9-60, 9-66              TCC_MON_ON, 28-49, 28-51

TC_BYPLES, 11-167                   TCC_ON_SW, 28-51

TC_CANP, 15-36, 15-83               TCC_ON_TSTSW, 28-49

TC_CNTR, 10-9, 10-11 to 10-13       TCC_OSM_TMR, 28-51

TC_DELPR_M6, 12-41 to 12-42         TCD43AOF, 26-43

TC_DFSO_FL, 10-11                   TCD43ON, 26-43

TC_EGR_N, 12-16                     TCDELPR, 12-18

TC_EGRDC_M6, 12-41 to 12-42         TCDESN, 13-20

TC_EPTBAR_M6, 12-35                 TCDNAF, 26-44

TC_HOSE_M6, 12-37                   TCDNMF, 26-44

TC_HRD_OFF, 3-9, 28-78 to 28-80     TCDNON, 26-43 to 26-44

TC_INFAMB, 10-13                    TCDNS, 28-33

TC_M_DEL_M6, 12-41 to 12-42         TCDP, 12-20

TC_OVER, 13-41                      TCEACT, 12-25

TC_PFS, 15-79                       TCEPC, 27-15

TC_PGLAM, 15-16                     TCEPT, 12-17

TC_RUN_FEH, 10-11                   TCEPTZER, 12-16

TC_SOAK_CMS, 10-10                  TCEPTZER_OFF, 12-17

TC_SOAK_FEH, 10-10                  TCERRD, 28-34

TC_SOAK_FEU, 10-10                  TCERRI, 28-34

TC_SOAK_FL, 10-10                   TCERRP, 28-34

TC_SOAK_REG, 10-10                  TCF, 9-60

TC_TPR, 15-28                       TCHRDINT, 3-9

TC_UNDER, 13-41                     TCIL_B, 4-33

TC_VBAT, 13-21                      TCIL_FAULT, 4-33

TC_VEGO_FA (DEFINED), 8-294         TCIL_FAULT (READ), 5-140 to 5-141

TC_VEGO_FA (READ), 8-295            TCIL_FLASH_TMR, 25-8

TC_VEGO_FA_I (DEFINED), 8-295       TCIL_STATE, 25-8, 25-10

TC_VEGO_FA_I (READ), 8-295          TCIL_STATE (READ), 5-133

TCAP_CONV1, 28-81                   TCIL_TM_DLY, 25-8

TCAP_HLK, 28-79 to 28-80            TCILTM1, 25-8

TCAP_INTR1, 28-30, 28-67, 28-72,    TCILTMR, 25-8, 25-10

28-79 to 28-81                  TCINJD, 8-124

TCAP_PSTR1, 28-67                   TCLCK, 28-33

TCAP_REQ1, 28-30, 28-67, 28-70,     TCLCKL, 28-33

28-77, 28-79 to 28-80, 28-84,   TCLOAD_EGO, 11-11

28-87 to 28-88                  TCN, 29-23

TCAP_SFT_ST1, 28-70, 28-72          TCN_EGO, 11-11

TCAPOFF, 28-78                      TCNDBR, 29-23

TCAPSTR, 28-72, 28-79               TCNDT_DAS, 29-23

TCAPSTRMULT, 28-72, 28-79           TCNDT_IMCC, 29-23

TCBBAR, 29-14                       TCNDT_PSS, 29-23

TCBPA, 13-46                        TCNDT_SPK, 29-23

TCBPA_DER, 13-46                    TCNE, 25-33

TCBYLD, 24-63                       TCNFIL, 29-23

TCC_CKT_SW, 25-8                    TCNO, 31-16

TCC_CTL_FLG, 28-54, 28-56           TCOFFDLY1, 28-78

TCC_CTL_SLIP, 28-54                 TCOFFDLY2, 28-78

TCC_CTL_TIM, 28-54                  TCPRG, 15-20

TCC_CTL_TMR, 28-54                  TCPRG (READ), 15-21

TCC_DC_OFF, 28-49, 28-51            TCR_OSS, 25-92

TCC_DC_ON, 28-49, 28-51             TCRBIAS (DEFINED), 8-309

Index-69

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TCRBIAS (READ), 8-309               TFC_HR, 8-105 to 8-106, 8-111

TCRMP1, 28-79                       TFC_HR (READ), 8-180

TCRMP10, 28-78                      TFC_IDLE_OFF, 8-106

TCRMP10L, 28-78                     TFC_MIN, 8-111

TCRMP2, 28-79                       TFC_MULT, 8-110 to 8-111

TCRMP3, 28-80                       TFC_MULTA, 8-107, 8-110

TCRTP, 31-72                        TFC_MULTD, 8-107, 8-110

TCRTP_MT, 31-72                     TFC_SLOPE, 8-107, 8-110

TCRTP_UP, 31-72                     TFC_TMR, 8-105

TCS_OPEN, 25-14, 34-2               TFCISW, 8-106

TCS_SHORT, 25-14, 34-2              TFCTM, 8-105

TCS_TST_ENA, 25-14                  TFMFLG, 11-109, 12-7, 13-7 to

TCSTRT, 8-14, 8-49, 8-121, 12-6,        13-8, 13-67, 24-29, 26-58,

12-10, 13-8, 15-71, 17-18,          27-14, 27-55, 28-65, 29-11 to

23-57, 30-5, 31-14, 31-38           29-12, 29-16 to 29-17, 31-60,

READ, 8-74                            31-70, 31-75, 31-80

TCSTRT (READ), 15-7                   READ, 8-88

TCSTRT_ACCUM, 6-6, 31-38            TFMFLG (READ), 5-132

TCSYS_DELPR, 12-40                  TFSMN, 8-106

TCTPPWF, 25-33                      THBP2, 7-3 to 7-4

TCTPREL, 25-34                      THBP_DECH, 8-136

TCTPT, 9-60                         THEFT_STATE, 5-105 to 5-110

TCTPTE, 25-33                       THEFT_STATUS, 5-105, 5-107, 5-114

TCTPTV, 25-34                       THEFT_STATUS (WRITE), 5-112

TCTQ_NORM, 25-52                    THEFT_TMR, 5-107 to 5-108

TCTQBRKS, 25-75                     THRMHP, 4-23, 4-35, 17-12

TCTQPQM, 25-64                      THROTTLE MODE SELECTION STRATEGY

TCTQPWM, 25-63                          (CDAI0), 7-2

TCTRAN_RPM, 13-13                   THROTTLE POSITION MULTI SAMPLING

TCULCK, 28-33                           (CDAN2), 31-91

TCUPOF, 26-43                       THROTTLE POSITION SENSOR INPUT

TCUPON, 26-43                           AND SELFTEST (CDAN2), 31-62

TCUPS, 28-33                        TIM_INT_FLG, 8-123

TCVBAT, 31-88                       TIME AT IDLE WITH TRANSMISSION IN

TCVEGO_SAIR, 17-10                      DRIVE (CDAA0), 8-245

TCVS_EGO, 11-11                     TIME SINCE INCREMENTING LAMMUL

TCVS_LIM_BAR, 31-83                     (CDAA0), 8-248

TCVSFPRES, 16-20                    TIMER SUMMARY (CDAI0), 8-249

TCVSR, 25-20                        TIMER SUMMARY (CDAN0), 13-87,

TCVSRPH, 25-20                          30-2

TCVSRSS, 25-20                      TIP_ALLOW_LD, 9-27 to 9-28

TCVST, 31-16                        TIP_COUNT, 9-66

TDMULT, 8-287                       TIP_DELTA, 9-27

TDREV_DELAY, 11-187                 TIP_LAST_LD, 9-27 to 9-28

TEMPFB, 8-251, 17-19, 31-61         TIP_MIN_SPK, 9-28

TEST_ENABLED, 11-71, 11-84, 11-88,  TIP_RMP_DLY, 9-66

11-90 to 11-91                  TIP_TR, 9-28

TESTIDMAP_1, 5-121                  TIP_TR_TRIG, 9-28

TESTIDMAP_2, 5-121                  TIPFLG, 9-60, 9-66

TESTIDMAP_3, 5-121                  TIPINC, 9-66

TESTIDMAP_4, 5-121                  TIPLOD_CH, 9-60

TESTIDMAP_5, 5-121                  TIPLOD_SL, 9-60

TESTIDMAP_6, 5-121                  TIPRET, 9-66 to 9-67, 9-69

TFC_CHG, 8-107, 8-110               TKDTN, 13-20

TFC_FG_EN, 8-105 to 8-106, 8-110    TKYON2, 31-14, 31-38, 31-55

Index-70

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TM12AKD, 25-23                      TMNEUREV, 27-8

TM41AUT, 26-28                      TMNOLKDLY, 28-39

TM42AUT, 26-28                      TMOLUP2, 28-27

TM46BLP, 27-8                       TMOLUP3, 28-27

TM4X4L, 25-57                       TMOLUP4, 28-27

TM54BLP, 27-8                       TMR_41AUT, 26-28

TM56BLP, 27-8                       TMR_42AUT, 26-28

TM64BLP, 27-8                       TMR_ASILT, 27-15, 27-55

TM_12_AKD, 25-23                    TMR_HS_M1, 26-30

TM_4X4L_RES, 25-57                  TMR_M12, 26-4, 26-30, 26-48

TM_ASILT, 27-15                     TMR_NO_LK, 28-39 to 28-40

TM_ASILT1, 27-55                    TMREVDRV, 27-8

TM_BRK_AWAY, 28-61 to 28-62         TMRTVSCAL, 25-29

TM_BRK_PREV, 28-62                  TMSFTSLPMAX, 28-25

TM_BRK_ZONE, 28-61 to 28-62         TMSLBRK, 28-62

TM_DEL_SFT, 26-35, 26-39            TMSQ21P1, 26-30

TM_HS_M12, 26-30                    TMSQ21P4, 26-40

TM_IN_GR4, 26-28                    TMSQ23P4, 26-36

TM_LK_DLY, 28-18 to 28-21           TMSQ32P2, 26-32

TM_M12, 26-4                        TMSQ34P4, 26-36

TM_NOV_CALC, 25-29                  TMTCS, 25-13

TM_OFF_DLY, 28-78                   TMTVRMP, 27-40

TM_PDL_RES, 25-46, 25-83            TMTVRMP_12, 27-39

TM_PWF_DEC, 25-4                    TMTVRMP_23, 27-39

TM_SEQ_SFT, 26-30, 26-32, 26-35     TMTVRMP_34, 27-40

to 26-36, 26-39 to 26-40        TMUP1DLY, 25-24

TM_SFT_CTL, 28-25 to 28-27          TMUP2DLY, 25-24

TM_SFT_EXIT, 28-27 to 28-28         TMUP3DLY, 25-24

TM_SFT_IN, 26-4, 26-43 to 26-44     TMUP[GR_CM]DLY, 25-24

TM_SFT_SLP, 28-25, 28-27 to 28-28   TMUPSEXIT, 28-27

TM_TCS_RES, 25-13                   TMVEDN, 25-24, 26-19

TM_UN_CT, 28-17, 28-21              TMVETOUP, 25-23, 26-19

TM_UNLK_WDOG, 26-26                 TMVEUP, 25-23, 26-19

TM_VER_SFT, 26-18 to 26-19, 26-44,  TMVEUP_4X4L, 26-19

28-8, 28-27, 28-33 to 28-34     TMW12DLY, 25-24

TMCLUP2, 28-26                      TO_TP_RATE, 26-19

TMCLUP3, 28-26                      TORQUE CAPACITY CALCULATION

TMCLUP4, 28-26                          (CDAK0), 28-81

TMCTDY, 28-21                       TORQUE CAPACITY CONVERSION

TMD2SLP, 28-27                          (CDAK0), 28-82

TMD3SLP, 28-27                      TORQUE CAPACITY DETERMINATION

TMD4SLP, 28-27                          LOGIC (CDAN0), 28-66

TMDELDN, 26-39                      TORQUE INTERPOLATION CALCULATION

TMDELUP, 26-35                          (CDAN0), 25-50

TMDN2DLY, 25-24                     TORQUE MODULATION BACKGROUND

TMDN3DLY, 25-24                         CALCULATION (CDAK0), 9-49

TMDN4DLY, 25-24                     TORQUE MODULATION CONTROL

TMDN[GR_CM]DLY, 25-24                   (WITHOUT TURBINE SPEED)

TMDNSEXIT, 28-28                        (CDAN0), 26-63

TMDNUN, 26-26                       TORQUE MODULATION FOREGROUND

TMDRVREV, 27-8                          CONTROL (WITHOUT TURBINE

TMENGACT, 27-8                          SPEED) (CDAN0), 26-69

TMHOLDPWF, 25-4                     TORQUE MODULATION RAMP (CDAK0),

TMHSM1, 26-30                           9-88

TMNEUDRV, 27-8

Index-71

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TORQUE NET CALCULATION (CDAN0),     TOTMIS_THRS, 35-101

25-15                           TOTMISBGTOBG, 35-35

TORQUE TRUNCATION VIA SPARK         TOTMISFIL, 5-121, 35-35, 35-39 to

RETARD (CDAE0), 9-23                35-40, 35-54, 35-101

TORQUE-BASED FEEDBACK SPARK AT      TOTMISLEVEL, 35-39 to 35-40,

IDLE (CDAC0), 9-32                  35-42, 35-49 to 35-50

TOT, 23-156, 25-24, 25-26, 25-37,   TOTNOCALLOLD, 35-54

25-63, 26-10, 26-58, 27-12,     TOTTTV1, 29-19

27-47, 28-3, 28-6, 28-53,       TOTTV2, 29-19

28-56, 28-84, 29-19 to 29-20,   TOTTV3, 29-19

31-52, 31-54 to 31-55, 31-57    TOTTV4, 29-20

TOT (READ), 27-53                   TOTVTP, 27-22

TOT SENSOR INPUT AND SELFTEST       TP, 9-40, 9-60, 9-66, 10-23,

(CDAM0), 31-49                      23-26 to 23-28, 23-41 to

TOT_CNTS, 4-7, 31-53                    23-44, 25-33 to 25-34, 25-63,

TOT_CNTS (READ), 5-145                  27-16, 27-22, 27-28, 29-16,

TOT_CODES, 31-51                        31-66, 31-69, 31-71 to 31-72,

TOT_DES_FF, 28-56                       31-75

TOT_DES_HF, 28-56                   TP12AKD, 25-23

TOT_ENG, 31-34, 31-52 to 31-56      TP12HYS, 25-23

TOT_ENG (READ), 5-146               TP3LCK, 28-6

TOT_ER_TM, 31-54                    TP3LCK_HYS, 28-6

TOT_ER_TMR, 31-53 to 31-55          TP_CNT1, 31-92

TOT_ERR_SW, 25-9                    TP_CNT2, 31-92

TOT_FM_FLG, 25-9, 31-54 to 31-55,   TP_CNT3, 31-92

31-57                           TP_CNT4, 31-92

TOT_FM_FLG (READ), 5-132            TP_CNTS, 4-7

TOT_FM_TM, 31-55                    TP_CODES, 31-65

TOT_FM_TMR, 31-54 to 31-55          TP_CTR, 31-71

TOT_FRST_FL, 31-34, 31-55           TP_CTR_MAX, 31-71 to 31-72

TOT_HI_ER, 31-54                    TP_DELTL, 26-66

TOT_HI_KOEO, 31-56                  TP_DELTU, 26-66

TOT_HI_KOER, 31-56                  TP_DIF_CTR, 31-71 to 31-72

TOT_LO_ER, 31-54                    TP_DIF_MX, 28-60

TOT_LO_KOEO, 31-56                  TP_ENG, 31-66, 31-68 to 31-69,

TOT_LO_KOER, 31-56                      31-71, 31-74 to 31-75, 31-92

TOT_LST, 31-52, 31-54, 31-57        TP_ENG (READ), 5-144

TOT_MAX, 31-57                      TP_ENG1, 31-92

TOT_OTEMP_S (READ), 27-53           TP_ENG2, 31-92

TOT_PREV, 31-57                     TP_ENG3, 31-92

TOT_RMP_TIM, 31-57                  TP_ENG4, 31-92

TOT_SFTV_MN, 26-58                  TP_ENG_LAST, 31-71, 31-92

TOT_SFTV_MX, 26-58                  TP_FIL, 23-40 to 23-43

TOT_TC, 31-55                       TP_FIL_MAX, 23-35, 23-43

TOT_TMR, 31-57                      TP_FIL_MAX (READ), 5-145

total_delay, 8-185                  TP_HI_ER, 31-68

TOTAL_DELAY1, 8-185 to 8-186,       TP_HI_KOEO, 31-75

8-208                           TP_HI_KOER, 31-74

TOTAL_DELAY2, 8-185 to 8-186,       TP_HL, 28-77

8-208                           TP_IDL_ER, 31-68

TOTBGTOBG, 35-35, 35-43             TP_INT, 23-40 to 23-41, 23-43

TOTEVENTS, 35-26, 35-54             TP_LAST, 9-40

TOTEVENTS (READ), 5-145             TP_LO_ER, 31-68

TOTLDST, 24-29, 24-63, 29-7,        TP_LO_KOEO, 31-75

29-11                           TP_LO_KOER, 31-74

Index-72

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TP_LVL, 23-42                       TQ3DTS, 26-68

TP_MIN_TQM, 26-66                   TQ4DTS, 26-68

TP_RATE, 25-33, 26-19, 28-14 to     TQ_BAR, 25-17, 25-63 to 25-64,

28-15                               28-72, 28-78 to 28-81, 28-87

TP_REL, 7-3 to 7-4, 8-8, 9-14,      TQ_BAR_LST, 25-17, 25-63 to 25-64,

10-24, 11-110, 12-6, 14-8,          28-78

14-12, 15-18, 15-91 to 15-92,   TQ_BAR_PWM, 25-63 to 25-64

18-11, 20-3, 24-27, 24-46,      TQ_BRAKE_S, 25-75, 35-29, 35-101

24-62 to 24-63, 25-4, 25-23     TQ_BRK_SBAR, 25-75, 31-79

to 25-24, 25-34, 25-63, 26-12   TQ_CK_SW, 31-79

to 26-14, 26-58, 26-68, 27-16   TQ_CK_VMOV, 31-79

to 27-17, 27-20 to 27-21,       TQ_DIF_MX, 28-60

27-27, 27-37 to 27-39, 28-6,    TQ_IALPHA, 27-27, 27-33

28-14 to 28-15, 28-59 to        TQ_IALPHA CALCULATION (CDAA0),

28-60, 28-65, 28-77, 29-11,         27-32

29-16 to 29-17, 31-61, 31-66    TQ_INP_NET, 25-17

READ, 8-88                        TQ_INTRP, 3-9, 25-52, 26-10

TP_REL (READ), 8-136                TQ_LEVEL, 9-51, 26-68

TP_REL_BAR, 25-34, 25-51            TQ_LIMIT, 25-17

TP_REL_ENT, 28-59 to 28-60          TQ_MAX_DELT, 25-52

TP_REL_H, 26-66, 26-68, 31-66       TQ_MAX_TM, 25-52

TP_SH_VAL2, 26-58                   TQ_MBT, 9-51, 25-17

TP_SH_VAL3, 26-58                   TQ_MIN_CLIP, 26-68

TP_SH_VAL4, 26-58                   TQ_NET, 25-17, 25-51 to 25-52,

TP_STRT_SFT, 26-66, 27-37               25-63 to 25-64, 26-68, 27-27,

TP_VMAF, 24-46                          27-50, 28-38, 28-59 to 28-60

TPBAR_MT, 31-66, 31-71 to 31-72     TQ_NET (READ), 5-146

TPBART, 3-9, 25-33, 25-63           TQ_NET_ENT, 28-59 to 28-60

TPBART_PWF, 25-33                   TQ_NET_LST, 25-17, 25-52

TPBARTV, 3-9, 25-34, 27-22, 27-28   TQ_NET_MIN, 25-52

TPBKFLOWH, 24-27                    TQ_NET_S, 25-17

TPBPHI, 29-11                       TQ_NORM, 25-52

TPBPLO, 29-11                       TQ_NORM_CUR, 25-51 to 25-52

TPBRKTCSLP, 28-33                   TQ_NORM_LST, 25-52

TPLDST, 24-63                       TQ_NORM_NEW, 25-52

TPNEPUMIN, 25-24                    TQ_NORM_UP, 25-52

TPR_CNTS, 4-39, 15-26               TQ_POINTER, 35-14, 35-19, 35-32

TPR_CNTS (READ), 5-146              TQ_STAT_CP, 27-27, 27-29 to 27-30

TPR_ENG, 15-26 to 15-28, 15-46      TQ_TIMEOUT, 26-66

TPR_ER_TM, 15-27                    TQ_VMOV, 31-79

TPR_ER_TMR, 15-26 to 15-28          TQHRDDIF, 28-78

TPR_FM_TM, 15-28                    TQIA12, 27-33

TPR_FM_TMR, 15-26 to 15-28          TQIA21, 27-33

TPR_HI_ER, 15-27                    TQIA23, 27-33

TPR_LO_ER, 15-27                    TQIA32, 27-33

TPR_MON_TM, 15-28                   TQIA34, 27-33

TPRATE_PWF, 25-4, 25-33             TQIA43, 27-33

TPTVMN, 27-21                       TQM_COLD_CLP, 9-51

TPTVMX, 27-21                       TQM_MAX_CLP, 9-51

TPUNBRK, 28-15                      TQMIN12, 26-68

TPUNCH, 28-14                       TQMIN23, 26-68

TPUNCT, 28-14                       TQMIN34, 26-68

TPUNTR, 28-14 to 28-15              TQMOD_TMR, 26-66, 26-71

TPV_STRT_SFT, 26-58                 TQMODTM, 26-71

TQ2DTS, 26-68                       TR1, 4-12, 25-82

Index-73

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TR2, 4-22, 25-82                    TRIPS (CDAM0), 23-65

TR3, 25-80, 25-82                   TRLOAD, 8-18, 8-49, 8-131, 13-9,

TR3_ERROR, 25-80, 25-82                 13-13 to 13-14, 13-25, 13-43,

TR3A, 4-7                               13-52, 13-67, 13-73 to 13-74,

TR3A_CLOSED, 25-80                      23-181, 23-183 to 23-184,

TR3A_OPEN, 25-80                        29-6, 31-80, 31-86, 35-81

TR4, 4-23, 25-82                    TRLOAD (READ), 29-4

TR_ABS, 9-33, 9-37, 9-41            TRNS_ERR_TM, 25-85

TR_COR, 9-37, 9-43                  TRNS_ERR_TMR, 25-83, 25-85

TR_DELTA, 9-46                      TRQLST, 35-14, 35-19

TR_LOAD_FLG, 31-79 to 31-80         TRS_ERR_SW, 25-9

TR_RES, 9-37, 9-40, 9-46            TRS_TYPE, 25-79

TR_RES_LAST, 9-40, 9-46             TRUHAC, 28-14

TR_SPK_DELTA, 25-17                 TRUHDC, 28-15

TR_SPK_DELTA(read), 35-33           TRULAC, 28-14

TR_STRT_SFT, 27-27                  TRULDC, 28-15

TR_TQ_LIMIT, 9-26, 25-17            tscf, 29-23

TRAC_ACTIVE, 31-60                  TSEGRE, 12-10 to 12-11, 30-5

TRAC_AIRSW, 31-60                   TSEGRE_FLG, 12-10

TRAC_ULFLG, 28-19, 28-21            TSLADPTMR (READ), 15-8

TRADLY, 28-18                       TSLADPTMR (WRITE), 15-6, 15-9

TRANS_TQ_IN, 25-75, 31-79           TSLADV, 9-59, 9-61

Transient Fuel -                    TSLAMU1, 8-47 to 8-48, 8-273,

V1.1_FTRAN_TRANSIENT_FUEL           8-277 to 8-278

(CDAM0), 8-100                  TSLAMU1 (WRITE), 8-55 to 8-58

TRANSIENT SPARK FOR DETONATION      TSLAMU2, 8-47 to 8-48, 8-273,

PREVENTION (CDAM0), 9-103           8-277 to 8-278

TRANSIENT SPARK FOR DETONATION      TSLAMU2 (WRITE), 8-55 to 8-58

PREVENTION (CDAN0), 9-99        TSLIDM, 35-82 to 35-83, 35-85

TRANSMISSION COMPREHENSIVE          TSLMPH, 3-9, 31-83

COMPONENT MONITOR (CDAK0),      TSLOS_SPK, 25-93

23-160                          TSLOSS, 25-92

TRANSMISSION CONTROL INDICATOR      TSLPIP, 6-7, 8-143, 8-254, 13-7

LIGHT (CDAM0), 25-5                 to 13-8, 24-9, 24-46 to 24-47,

TRANSMISSION CONTROL SWITCH             29-23, 29-28, 29-30 to 29-31,

(CDAM0), 25-11                      30-5, 31-71, 35-68, 35-82

TRANSMISSION CONTROL SWITCH         TSLVS_SPK, 25-98 to 25-99

CIRCUIT TEST (CDAB0), 34-2      TSP_DBAND1, 10-28

Transmission Input Torque           TSP_DBAND2, 10-29

Calculation (CDAL0), 25-73      TSP_MXMN_V12, 10-28

1 TRANSMISSION OVERTEMPERATURE      TSP_MXMN_V22, 10-29

TEST (CDAN0), 27-52             TSS, 4-24

TRANSMISSION SELF TEST ENABLE       TSSMIN, 28-54

(CDAM0), 25-25                  TST_DIS_TIM, 25-26

transmission_overtemp_test          TST_DLY_TMR, 25-64

(PROCEDURE), 27-52              TST_ECT_MIN, 25-26

TRANSPORT DELAY LOOKUP (CDAI0),     TST_ENA_FLG, 25-26, 31-80

8-285                           TST_SW_FLG12, 10-28, 10-30

TRDDLY, 28-18                       TST_SW_FLG22, 10-29, 10-31

TRIP, 23-68, 23-71 to 23-72         TST_TOT_MIN, 25-26

TRIP (READ), 5-133                  TSTALL, 29-23, 31-71

TRIP_COUNT, 15-50, 23-68, 23-71     TSTBCSDC, 28-30

TRIP_COUNT (READ), 5-144            TSTDLYTM, 25-64

TRIP_NOT_PRG, 15-45, 15-50 to

15-51

Index-74

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

TSTRAT, 4-17, 4-32 to 4-33,         type1_init_mil(ego), 23-51

23-152, 23-163, 25-13, 25-43    type1_init_mil(egr), 23-51

to 25-44, 25-71, 25-92, 26-48,  type1_init_mil(purg), 23-51

26-73, 27-47, 28-48, 29-5,      type1_init_mil(sair), 23-51

31-16, 31-34, 31-53             type1_malfunction(CAT_MIL_ON,

TSTRAT (READ), 5-140, 27-53             Pxxx), 23-50

TSTSLIP, 28-30                      type1_malfunction(CCM_MIL_ON,

TSTTCAP1, 28-30                         Pxxx), 23-50

TV_4L_12, 27-40                     type1_malfunction(EGO_MIL_ON,

TV_4L_21, 27-40                         Pxxx), 23-50

TV_4L_23, 27-40                     type1_malfunction(EGR_MIL_ON,

TV_4L_32, 27-40                         Pxxx), 23-50

TV_4L_34, 27-40                     type1_malfunction(PURG_MIL_ON,

TV_4L_43, 27-40                         Pxxx), 23-50

TV_ASILT1, 27-15                    type1_malfunction(SAIR_MIL_ON,

TV_ASILT2, 27-15                        Pxxx), 23-50

TV_ASILT3, 27-15                    type1_reset, 23-52

TV_COUNT_LST, 27-47 to 27-48        TYPE_A_MIS, 23-93 to 23-94,

TV_COUNTS, 4-17, 27-47 to 27-49         23-112, 35-39 to 35-40

TV_DYN, 27-28, 27-38 to 27-40       TYPEA_FL_MIL, 23-112

TV_PRES, 23-156, 27-12 to 27-17,

27-22, 27-28                    U12DLY, 28-20

TV_PRES_BAR, 27-15, 27-47           U23DLY, 28-20

TV_PRES_BAR (READ), 5-145           U34DLY, 28-20

TV_RAMP, 27-39 to 27-40             UIMR, 5-23

TV_RAMP_TMR, 27-39 to 27-40         UNCONDITIONAL UNLOCK LOGIC

TV_ST_LST, 27-30                        (CDAN0), 28-10

TV_ST_SFT, 27-28, 27-30 to 27-31    UNDSP, 3-9, 5-11, 5-57, 5-107,

TV_STAT, 27-28 to 27-29                 6-6 to 6-7, 8-105, 8-252,

TVCDLT, 27-48                           8-325, 9-6 to 9-7, 9-71, 9-78,

TVCHRG, 27-16 to 27-17                  9-83, 10-9, 10-23, 11-49,

TVEMAX, 27-21                           11-82, 11-88, 23-21, 23-158

TVEMOD, 27-21                           to 23-159, 23-181, 23-183 to

TVFMMN, 27-14                           23-184, 24-28, 24-46 to 24-47,

TVPMIN_1, 27-13                         26-48, 30-4, 35-77 to 35-78

TVPMIN_2, 27-13                       READ, 8-74, 8-87

TVPMIN_3, 27-13                     UNDSP (READ), 8-137

TVPMIN_4, 27-13                     UNIVERSAL POWER OUTPUT DRIVER

TVPMIN_ENG, 27-13                       (UPOD), 4-28

TVPMINPT_1, 27-13                   UNK_MIS_CODE, 23-49, 23-97

TVPMINPT_2, 27-13                   UNLKDLY_TMR, 28-14

TVPMINPT_3, 27-13                   UNLKDLYTM, 28-14

TVPMINPT_4, 27-13                   UNRPMH, 6-7

TVPMINPT_ENG, 27-13                 update_injector

TVPMINST_1, 27-13                     CALL, 8-189

TVPMINST_2, 27-13                     DEFINITION, 8-189

TVPMINST_3, 27-13                   UPDATE_TMR, 25-52

TVPMINST_4, 27-13                   update_warmup_counters, 23-57

TVRMP_12, 27-39                     UPDATM, 13-32

TVRMP_23, 27-39                     UPDISC, 13-32

TVRMP_34, 27-40                     UPRATE

TVRMPTM, 27-39 to 27-40               READ, 8-76 to 8-77

TWO-SPEED FUEL PUMP CONTROL LOGIC     WRITE, 8-79, 8-81

(CDAJ0), 8-144                  UPSTREAM EGO MONITOR (CDAM0),

type1_init_mil(ccm), 23-51              11-18

Index-75

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

UPSTREAM SENSOR SWITCHPOINT         V_CYL2_TOT, 35-25, 35-35, 35-46,

CALCULATION (CDAM0), 11-185         35-53

US_CM_MAX, 11-74                    V_CYL3_MIS, 35-24, 35-34 to 35-35,

US_CM_MIN, 11-74                        35-43, 35-46, 35-53

US_CUR_HI, 11-77                    V_CYL3_TOT, 35-25, 35-35, 35-46,

US_CUR_LOW, 11-77                       35-53

US_FAULTED, 11-89 to 11-91          V_CYL4_MIS, 35-24, 35-34 to 35-35,

US_HEAT_TM, 11-74                       35-43, 35-46, 35-53

US_HTR_OFF, 11-82 to 11-83          V_CYL4_TOT, 35-25, 35-35, 35-46,

US_HTR_ON, 11-82                        35-53

US_HTR_TMR, 11-74, 11-92            V_CYL5_MIS, 35-24, 35-34 to 35-35,

US_OFFSET, 11-165, 11-185               35-43, 35-46, 35-53

US_OV_ON, 11-12                     V_CYL5_TOT, 35-25, 35-35, 35-46,

US_REV_DLY, 11-15                       35-53

US_SWAPPED_1, 11-139, 11-143,       V_CYL6_MIS, 35-24, 35-34 to 35-35,

11-145 to 11-146, 11-150            35-43, 35-46, 35-53

US_SWAPPED_2, 11-139, 11-146,       V_CYL6_TOT, 35-25, 35-35, 35-46,

11-150                              35-53

US_SWTPOINT, 5-121, 11-26, 11-30    V_CYL7_MIS, 35-24, 35-34 to 35-35,

to 11-32, 11-143, 11-145 to         35-43, 35-46, 35-53

11-146, 11-164 to 11-165,       V_CYL7_TOT, 35-25, 35-35, 35-46,

11-174, 11-179                      35-53

US_TST_OK, 11-139 to 11-141,        V_CYL8_MIS, 35-24, 35-34 to 35-35,

11-143, 11-145 to 11-146            35-43, 35-46, 35-53

USPD_RUN_TMR, 30-5, 31-38, 35-29    V_CYL8_TOT, 35-25, 35-35, 35-46,

USPD_RUN_TMR (READ), 13-79              35-53

V_DIS_PIPDOT(r), 9-97

V_EAMM_ENA, 17-18, 17-26, 17-29

V_1PAC_ERR, 35-86                   V_EAMM_TM, 17-24 to 17-26, 17-29

V_ADPAM_MAX, 8-95                   V_EAMM_TMR, 17-11 to 17-12, 17-15

V_ADPAM_MIN, 8-95                       to 17-16, 17-24 to 17-26,

V_ADPLD_MAX, 8-95                       17-29

V_ADPLD_MIN, 8-95                   V_EGO1_BYPS, 11-119

V_ADPN_MAX, 8-95                    V_EGO1L_BYPS, 11-118

V_ADPN_MIN, 8-95                    V_EGO2_BYPS, 11-122 to 11-123

V_AP_TST_TM, 17-28                  V_EGO2L_BYPS, 11-122 to 11-123

V_AP_TSTTMR, 17-11, 17-16, 17-28    V_EGO_ENA, 11-108, 11-138

to 17-29, 17-31                 V_EGOIDL_ENA, 11-110

V_BOOS_OFF, 31-19 to 31-20          V_EGOLD_MAX, 11-110

V_BOOS_ON, 31-19 to 31-20           V_EGOLD_MIN, 11-110

V_CIDN_MAX, 35-80                   V_EGORNTM, 11-109

V_CIDN_MIN, 35-80                   V_EGOSWNUM, 11-118 to 11-119,

V_CL_BRK_TM, 35-84                      11-122 to 11-123

V_CL_BRK_TMR, 35-81, 35-84          V_EGOTP_MIN, 11-110

V_COILOK_CTR, 35-73 to 35-75        V_EGOTST_TM, 11-111

V_CPU_OK_DUR, 35-68                 V_EGOTST_TMR, 11-110 to 11-112

V_CPU_OK_PTM, 35-68                 V_EGOWARM, 11-109

V_CPU_OK_TMR, 35-68                 V_EGR_DLY, 11-124

V_CPUOK_ENA, 33-2, 35-89            V_EGR_DLYFG, 11-109, 11-124

V_CYL1_MIS, 35-24, 35-34 to 35-35,  V_EGR_OLD, 11-124

35-43, 35-46, 35-53             V_EGRDC_MAX, 5-121, 12-41

V_CYL1_TOT, 35-25, 35-35, 35-46,    V_FP_ER_TM1, 8-153

35-53                           V_FP_ER_TM2, 8-158

V_CYL2_MIS, 35-24, 35-34 to 35-35,  V_FP_ER_TM3, 8-158

35-43, 35-46, 35-53             V_FP_ER_TM4, 8-160

Index-76

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

V_FP_ER_TMR1, 8-153                 V_MIS200_1, 35-27, 35-43, 35-45,

V_FP_ER_TMR2, 8-157 to 8-158            35-53 to 35-54

V_FP_ER_TMR3, 8-157 to 8-158        V_MIS200_2, 35-27, 35-43, 35-45,

V_FP_ER_TMR4, 8-160                     35-53 to 35-54

V_IAC_TST (READ), 13-79 to 13-80    V_MIS200_3, 35-27, 35-43, 35-45,

V_IDM_CTR, 35-73 to 35-75, 35-85        35-53 to 35-54

V_IDM_FLG, 35-73 to 35-74           V_MIS200_4, 35-27, 35-43, 35-45,

V_IDM_NOISE, 35-85                      35-53 to 35-54

V_INRANG_FLG, 11-109 to 11-110      V_MIS200_5, 35-27, 35-43, 35-45,

V_ISC_HI_DLY, 23-186                    35-53 to 35-54

V_ISC_HI_MIN, 23-186                V_MIS200_6, 35-27, 35-43, 35-45,

V_ISC_LO_DLY, 23-189                    35-53 to 35-54

V_ISC_LO_TIM, 23-189                V_MIS200_7, 35-27, 35-43, 35-45,

V_KAMBAR_MAX, 8-97 to 8-98              35-53 to 35-54

V_KAMBAR_MIN, 8-97 to 8-98          V_MIS200_8, 35-27, 35-43, 35-45,

V_LAMBAR_MAX, 8-97 to 8-98              35-53 to 35-54

V_LAMBAR_MIN, 8-97 to 8-98          V_MIS_BGMAX, 35-29

V_LAMJMP1, 11-126                   V_MIS_LOOPS, 35-14, 35-32

V_LAMJMP1 (WRITE), 8-55 to 8-58     V_MIS_MONCNT, 35-41

V_LAMJMP2, 11-126                   V_MIS_NMAX, 35-29

V_LAMJMP2 (WRITE), 8-55 to 8-58     V_MIS_NMIN, 35-29

V_LEGO_MAX, 11-118 to 11-119,       V_MIS_THR1, 35-14, 35-21

11-122 to 11-123                V_MIS_THR2, 35-14, 35-21

V_LEGOTMR1, 11-112, 11-118 to       V_MIS_TOT1000, 35-54

11-119, 11-126                  V_MIS_TOT200, 35-54

V_LEGOTMR2, 11-112, 11-122 to       V_MIS_TSTCNT, 5-121, 35-22, 35-41,

11-123, 11-126                      35-54

V_LESMUL, 11-118                    V_MODE_SETUP, 32-5

V_LESTM1, 11-118 to 11-119          V_MOV_DLY, 31-80

V_LESTM2, 11-122 to 11-123          V_PAC_A_CTR, 35-75, 35-84, 35-86

V_LESTMR1, 11-112, 11-118 to        V_PAC_B_CTR, 35-75, 35-84, 35-86

11-119, 11-125                  V_PAC_C_CTR, 35-75, 35-84, 35-86

V_LESTMR2, 11-112, 11-122 to        V_PAC_D_CTR, 35-75, 35-84, 35-86

11-123, 11-125                  V_PAC_U_CTR, 35-73 to 35-75,

V_LOAD_HI, 31-61                        35-84, 35-86

V_LOAD_LOW, 31-61                   V_PIP_HARD, 35-82

V_LST_DPT, 35-15                    V_POWS_OFF, 31-43 to 31-44

V_MAF_ER_FLG, 24-3                  V_POWS_ON, 31-43 to 31-44

V_MIS1000_1, 35-27, 35-43 to        V_PRE_CPP, 35-81

35-44, 35-53 to 35-54           V_PSPS_CNT, 31-44

V_MIS1000_2, 35-27, 35-43 to        V_PSPSPD_CTR, 31-43 to 31-44,

35-44, 35-53 to 35-54               31-48

V_MIS1000_3, 35-27, 35-43 to        V_SETLNG_TM, 13-74

35-44, 35-53 to 35-54           V_SPDTOZ_FLG, 31-48

V_MIS1000_4, 35-27, 35-43 to        V_SPK_FLG, 35-65, 35-73

35-44, 35-53 to 35-54           V_SYNCFLG, 35-27, 35-52, 35-73,

V_MIS1000_5, 35-27, 35-43 to            35-76 to 35-78, 35-101

35-44, 35-53 to 35-54           V_TCS_ENA, 34-2

V_MIS1000_6, 35-27, 35-43 to        V_THERM_TM, 17-22 to 17-23

35-44, 35-53 to 35-54           V_THERM_TMR, 17-11 to 17-12,

V_MIS1000_7, 35-27, 35-43 to            17-15 to 17-16, 17-22 to

35-44, 35-53 to 35-54               17-23, 17-33

V_MIS1000_8, 35-27, 35-43 to        V_TP_ERR_MIN, 31-61

35-44, 35-53 to 35-54           V_TP_ERR_TMR, 31-60 to 31-61

V_TP_HIGH, 31-61

Index-77

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

V_TP_LOW, 31-61                     VEGO12 (READ), 5-144, 8-295

V_VSS_LOAD, 31-80                   VEGO12_BAR (DEFINED), 8-294

V_VSS_MULT, 31-80                   VEGO12_BAR (READ), 8-295, 8-297

V_VSS_NRHI, 31-80                   VEGO12_BAR (WRITE), 8-295

V_VSS_NRLO, 31-80                   VEGO12_FIL2, 5-121, 17-10 to

V_WOTNMIN, 31-61                        17-11

V_ZTOSPD_MPH, 31-48                 VEGO21, 11-141 to 11-142, 11-165,

V_ZTOSPD_TM, 31-48                      17-10

V_ZTOSPD_TMR, 31-48                 VEGO21 (READ), 5-144

VABS_OPEN_HI, 11-169                VEGO21_FIL2, 5-121, 17-10 to

VABS_OPEN_LO, 11-169                    17-11, 17-27, 17-30, 17-32 to

VAC_RNG_FLG, 12-39 to 12-40             17-33

VAPOR MANAGEMENT VALVE OSM TEST     VEGO21_NEW, 11-26, 11-29 to 11-32

LOGIC (CDAM0), 15-33            VEGO22, 10-29, 11-47, 11-49,

VARIABLE CAM TIMING SPARK CONTROL       11-141 to 11-142, 11-168

LOGIC (CDAK0), 9-85             VEGO22 (READ), 5-144, 8-295

VBAT, 3-9, 8-137, 8-146, 8-157,     VEGO22_BAR (DEFINED), 8-294

11-78 to 11-79, 13-21, 14-19,   VEGO22_BAR (READ), 8-295, 8-297

15-35, 15-65, 15-72, 15-82,     VEGO22_BAR (WRITE), 8-295

16-16, 16-30, 18-8 to 18-9,     VEGO_12, 10-28

24-28, 27-48 to 27-49, 28-51,   VEGO_AMP11, 5-121, 11-36, 11-38

31-88, 35-71                    VEGO_AMP12, 11-36

VBAT (READ), 5-144                  VEGO_AMP21, 5-121, 11-36, 11-38

VBAT_CCM_MIN, 8-157, 14-19, 15-35,  VEGO_AMP22, 11-36

15-82, 16-16, 16-30, 18-8 to    VEGO_AMPn, 11-37

18-9, 28-51                     VEGO_BAR_I12 (DEFINED), 8-294

VBAT_MIN, 27-48                     VEGO_BAR_I12 (READ), 8-295

VBAT_MX, 15-72                      VEGO_BAR_I12 (WRITE), 8-295

VBATBAR, 13-21                      VEGO_BAR_I22 (DEFINED), 8-294

VBMAF_CL, 24-28                     VEGO_BAR_I22 (READ), 8-295

VBMAFMIN, 24-28                     VEGO_BAR_I22 (WRITE), 8-295

VCAMHP, 8-168, 8-174                VEGO_DLTA, 11-31

VCID_BYPS, 35-76, 35-80             VEGO_FAOS_MN (DEFINED), 8-296

VCIDCNT, 35-71 to 35-72, 35-76 to   VEGO_FAOS_MN (READ), 8-297

35-78                           VEGO_FAOS_MX (DEFINED), 8-296

VCIDERRCNT, 35-76, 35-80            VEGO_FAOS_MX (READ), 8-297

VCIDFLG, 35-76                      vego_filter (PCALL), 8-293

VCIDPASCNT, 35-76, 35-80            vego_filter (PROCEDURE), 8-293

VCIDSW, 35-76, 35-80                VEGO_LNAVG11, 11-31 to 11-32

VDLY6, 35-85                        VEGO_LNAVG21, 11-31 to 11-32

VECT3, 8-74                         VEGO_LST11, 11-26, 11-29 to 11-32

VECT5, 8-74                         VEGO_LST21, 11-26, 11-29 to 11-32

VEG021, 11-27                       VEGO_OK_L (FCALL), 8-300

VEG021_FIL1, 17-10                  VEGO_OK_L1 (DEFINED), 8-296

VEGO11, 11-27, 11-141 to 11-142,    VEGO_OK_L1 (WRITE), 8-297

11-162, 11-165, 17-10           VEGO_OK_L2 (DEFINED), 8-296

VEGO11 (READ), 5-144                VEGO_OK_L2 (WRITE), 8-297

VEGO11_FIL1, 17-10                  VEGO_OK_R (FCALL), 8-300

VEGO11_FIL2, 5-121, 17-10 to        VEGO_OK_R1 (DEFINED), 8-296

17-11, 17-27, 17-30, 17-32 to   VEGO_OK_R1 (WRITE), 8-297

17-33                           VEGO_OK_R2 (DEFINED), 8-296

VEGO11_NEW, 11-26, 11-29 to 11-32   VEGO_OK_R2 (WRITE), 8-297

VEGO12, 10-28, 11-47, 11-49,        VEGO_OLD12, 10-28

11-141 to 11-142, 11-162,       VEGO_OLD22, 10-29

11-168                          VEGO_PK_L11, 11-31

Index-78

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

VEGO_PK_L21, 11-31                  VIDMPASCNT, 35-73, 35-84 to 35-85

VEGO_PK_LN11, 11-32                 VIP_KAM, 32-10

VEGO_PK_LN12, 11-45, 11-49, 11-56   VIP_KNOCK, 9-63

VEGO_PK_LN21, 11-32                 VIPLR1, 11-143, 11-145, 11-149

VEGO_PK_LN22, 11-45, 11-49, 11-56   VIPRR1, 11-148

VEGO_PK_RH11, 11-31 to 11-32        VKOEO_CPUOK, 33-2, 35-68, 35-89

VEGO_PK_RH12, 11-45, 11-49, 11-56   VLCM OUTPUT SIGNAL PROCESSING

VEGO_PK_RH21, 11-31 to 11-32            (CDAL0), 16-26

VEGO_PK_RH22, 11-45, 11-49, 11-56   VLCM_HP, 16-30

vego_ratchet (PCALL), 8-293         VMAFPIPLMT, 24-46 to 24-47

vego_ratchet (PROCEDURE), 8-295     VMIS_NOISBND, 35-20 to 35-21,

VEGO_RH_FF11, 11-154                    35-23, 35-32

VEGO_RH_FF12, 11-155                VMIS_TOT1000, 35-39 to 35-40,

VEGO_RH_FF21, 11-155                    35-43 to 35-44, 35-53

VEGO_RH_FF22, 11-156                VMIS_TOT200, 35-39 to 35-40,

VEGO_RHAV11, 11-31                      35-43, 35-45, 35-53

VEGO_RHAV21, 11-31                  VMOVING, 25-94, 25-99 to 25-100,

VEGO_RHAVG11, 11-31 to 11-32            31-80

VEGO_RHAVG21, 11-31 to 11-32        VMV FLOW TEST (CDAN0), 15-95

VEGO_RHAVG22, 11-49                 VMV_MXDC_MAX, 15-35 to 15-36

VEGO_SLOPE12, 10-28                 VMV_MXDC_ML, 15-35

VEGO_SLOPE22, 10-29                 VMV_MXDC_SUB, 15-35

VEGOBAR11, 11-141, 11-143, 11-145   VMV_OSM_CNTS, 4-9, 15-35 to 15-36

to 11-146                       VMV_OSM_CNTS (READ), 5-146

VEGOBAR12, 11-141 to 11-142,        VMV_OSM_TM, 15-36

11-144, 11-147 to 11-149        VMV_OSM_TMR, 15-35 to 15-36

VEGOBAR21, 11-141 to 11-143,        VMV_PRD_DT, 4-16, 4-44

11-146                          VMV_PW_DTKS, 4-44

VEGOBAR22, 11-141 to 11-142,        VMV_PW_DUCE, 4-44

11-144, 11-147                  VMV_ZDC_MIN, 15-35 to 15-36

VEGOBARR1, 11-142                   VMV_ZDC_ML, 15-35

VEGOLN_BAR11, 11-26, 11-31 to       VMV_ZDC_SUB, 15-35

11-32                           VNREVIDM, 35-75

VEGOLN_BAR21, 11-26, 11-31 to       VOLTCLP, 13-21

11-32                           VOLTMR, 13-21

VEGOPKLEA11, 11-31                  VPIPCNT, 35-71 to 35-72, 35-76

VEGOPKLEA21, 11-31                  VPIPERRCNT, 35-71 to 35-72, 35-82

VEGOPKLEAN11, 11-26, 11-36              to 35-83

VEGOPKLEAN21, 11-26, 11-36          VPIPFLG, 35-29, 35-32, 35-72,

VEGOPKRICH11, 11-26, 11-32, 11-36       35-80

VEGOPKRICH21, 11-26, 11-32, 11-36   VPIPPASCNT, 35-71 to 35-72, 35-82

VEGORH_BAR11, 11-26, 11-31 to           to 35-83

11-32                           VPMAX, 35-82

VEGORH_BAR21, 11-26, 11-31 to       VS, 11-11, 18-11, 23-41 to 23-44,

11-32                               25-45, 27-55, 31-16, 31-83

VEHICLE MOVING DETECTOR (CDAL0),    VS (READ), 5-145

31-76                           VS21PI, 26-30

VEHICLE SPEED INPUT AND SELF TEST   VS31MIN, 25-23

(CDAN0), 31-81                  VS32P2, 26-32

VEHICLE SPEED SELF TEST (CDAN0),    vs_codes, 25-97

25-95                           VS_DES_FF, 28-56

VER_BRK_TIM, 28-62                  VS_DES_HF, 28-56

VHNO_CL, 8-7                        VS_EGO, 11-11, 11-34 to 11-35

VHNO_SH, 8-7                        VS_EGO_DLTA, 11-35

VIDMERRCNT, 35-75, 35-84 to 35-85   VS_EGO_MAX, 11-34

Index-79

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

VS_EGO_MIN, 11-34                   VSCTR, 25-29, 25-98 to 25-99,

VS_FIL, 23-40 to 23-43                  27-55

VS_FIL_MAX, 23-35, 23-43            VSEGO_DLTMX, 11-35

VS_FIL_MAX (READ), 5-145            VSF, 16-27

VS_HW, 25-98, 31-82 to 31-83        VSF_AC, 16-20, 16-24

VS_INT, 23-40 to 23-41, 23-43       VSF_AC_FLG, 16-20

VS_LIM_BAR, 8-8, 31-16, 31-83       VSF_DECAY, 16-25

VS_LVL, 23-42                       VSF_DESECT, 16-22

VS_MOV, 31-80                       VSF_DPDT, 16-20

VS_NOISE_CTR, 25-98 to 25-99        VSF_FMEM1, 16-27

VS_NOISE_LVL, 25-99                 VSF_FMEM2, 16-27

VS_NOISE_SW, 25-9                   VSF_I, 16-22

VS_OD_CAN, 25-13                    VSF_IK, 16-22

VS_OL_CL, 8-27                      VSF_IMAX, 16-22

VS_OL_SH, 8-27                      VSF_MIN_CL, 16-24

VS_PRD, 31-83                       VSF_MIN_FLG, 16-24

VS_PRD_MIN_1, 31-83                 VSF_MIN_SH, 16-24

VS_PREV, 25-98, 31-82               VSF_PRES_CL, 16-20

VS_RATE, 8-50, 25-20                VSF_PRES_SH, 16-20

VS_RATE CALCULATIONS (CDAF0),       VSF_RQST_SPD, 16-25, 16-27

25-18                           VSF_RUNNING, 16-24

VS_RATE_SS, 25-24                   VSF_VS_CL, 16-20

VS_RATEPH, 25-20, 26-12             VSF_VS_FLG, 16-20

VS_RLS, 26-73                       VSF_VS_SH, 16-20

VS_RLSH, 26-73                      VSF_WORD, 16-25, 16-27

VS_SH_VAL2, 26-58                   VSFMFLG, 5-24, 21-3, 23-158 to

VS_SH_VAL3, 26-58                       23-159, 25-29, 25-99 to

VS_SH_VAL4, 26-58                       25-100, 26-10, 27-55, 31-83

VS_SMPL_TM, 25-20, 31-16            VSFMFLG (READ), 5-132, 13-79

VS_STRT_SFT, 27-27, 27-37           VSFOP_HP, 16-27

VSAMIN, 31-80                       VSHSM1, 26-30

VSBAR, 8-26 to 8-27, 8-130, 8-281,  VSLUMN, 28-17

9-26, 9-46, 10-13, 13-9,        VSPMIN, 25-45

13-59, 13-73, 15-47 to 15-48,   VSPRGFLG, 15-70, 15-72

15-70, 15-91 to 15-92, 15-102,  VSRATE_MAX, 25-20

16-4, 16-8, 16-20, 21-3, 22-5,  VSRATE_MIN, 25-20

23-26 to 23-27, 23-126,         VSRATE_SS, 25-20

23-158 to 23-159, 23-181,       VSRTVSMN, 25-29

23-183 to 23-184, 31-48,        VSS_ERR_CTR, 25-100 to 25-101

31-82 to 31-83, 31-87, 35-101   VSS_ERR_LVL, 25-101

VSBAR (READ), 8-307                 VSS_ERR_SW, 25-9

VSBAR_MIN, 23-158 to 23-159         VSS_ERR_UP, 25-100

VSBART, 26-73, 28-56                VSS_FM_TIM, 25-100

VSBART_RT, 25-13, 25-23 to 25-24,   VSS_FM_TMR, 25-100

26-30, 26-32, 26-39, 26-58,     VSS_MON_TIM, 25-100

27-11, 27-21, 27-27, 27-37,     VSS_MON_TMR, 25-100

28-17, 28-21, 28-39, 28-65,     VSSMN1, 25-99 to 25-100

31-16                           VSSMPLTM_PREV, 25-20

VSBART_RT_PD, 25-24, 26-13 to       VSSTAL, 27-11

26-14                           VSSTIM, 25-100

VSBART_RT_PU, 25-24, 26-12          VSSTMR, 25-98, 25-100, 31-80

VSBARTL, 25-20, 31-16               VSSTP, 31-80

VSBARV, 25-29, 25-99 to 25-100,     VSTVMN, 27-21

31-16, 31-80, 31-83             VSTVMX, 27-21

VSCNT, 25-98                        VSV_STRT_SFT, 26-58

Index-80

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

VTCEGO, 11-142                      WOTEGRTM, 12-7

WRM_IDL_FLG, 12-16 to 12-17,

WARM EGO LOGIC (CDAL0), 8-12            12-35, 23-28

WARM_UP                             WRMEGO, 8-15 to 8-16, 8-24, 8-43

WRITE, 8-74                           to 8-44, 8-49, 8-146, 31-14

WINCLD, 9-63                        WRMEGO (WRITE), 8-14

WINDOW_BETA, 9-58

WINDOW_DELTA, 9-58, 9-62

WINLEN, 9-62                        4 x 4 LOW SWITCH INPUT PROCESSING

WINLEN_B, 9-58, 9-64 to 9-65            AND SELF TEST (CDAJ0), 25-56

WOPEN, 9-58, 9-64 to 9-65           4x4_codes, 25-57

WOTCOT, 14-8                        XFREPT, 12-18

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

Please  feel  free  to  use

this  form to record faults

with   the   book,   and/or

general     comments    and

complaints  about  Strategy

Group documentation.  As we

make improvements, we would

also like to know about the

things you  would  like  to

see in the strategy book.

This form, if used,  should

be    directed    to    the

responsible  person  listed

on   the   cover   of  this

document.

INDEX - CDAN2

PED-PTE, FoMoCo, PROPRIETARY & CONFIDENTIAL

***********************************************************************

Do you have any general comments on this book, or its availability?

***********************************************************************

Specific errors found in this book.  Please list the page on which the error

was found, the error itself, and the correction if known.

***********************************************************************

Please indicate your area, and/or primary use made of this book.

If you would like to be notified when a change is made, or if you would like

to discuss something, please write your name and extension below.

Name______________________  Ext_____________   Date___________________